| 3GPP TR 23.700-43 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on CAPIF Phase 4; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Key issues	9

4.1	Key issue #1: Enhancements to API provider function to handle API invoker status change	9

4.1.1	Description	9

4.1.2	Open issues	9

4.2	Key issue #2: Enhancements to API administration in CAPIF domain and API provider domain	10

4.2.1	Description	10

4.2.2	Multiple API provider domains service API information	10

4.2.3	Open issues	10

4.3	Key issue #3: Certificate unavailability	11

4.3.1	Description	11

4.3.2	Open issues	11

4.4	Key Issue #4: Service API and AEF operational status	11

4.4.1	Description	11

4.4.2	Open issues	12

4.5	Key issue #5: Roaming Considerations for Service API Invocation	12

4.5.1	Description	12

4.5.2	Open issues	13

4.6	Key issue #6: Serving Area Considerations for Service API Invocation	13

4.6.1	Description	13

4.6.2	Open issues	13

4.7	Key issue #7: Open Discovery Policies	14

4.7.1	Description	14

4.7.2	Open issues	14

4.x	Key issue #x: <title>	14

4.x.1	Description	14

4.x.2	Open issues	14

5	Architecture enhancements	14

5.x	Option #x: <title>	14

5.x.1	Description	14

6	Solutions	14

6.1	Mapping of solutions to key issues	14

6.2	Solution #1: CAPIF Administrator revocation of API Invoker enrollment information	15

6.2.1	General	15

6.2.2	Architecture Impacts	15

6.2.3	Solution description	15

6.2.4	Solution evaluation	19

6.3	Solution #2: API invoker enrolled Service API notification	19

6.3.1	General	19

6.3.2	Architecture Impacts	19

6.3.3	Solution description	19

8.8.6	List of CAPIF events	21

6.3.4	Solution evaluation	21

6.4	Solution #3: API provider domain remove enrolled APIs for API invoker	21

6.4.1	General	21

6.4.2	Architecture Impacts	22

6.4.3	Solution description	22

6.4.4	Solution evaluation	23

6.5	Solution #4: Enhanced CAPIF events for AEF status	23

6.5.1	General	23

6.5.2	Architecture Impacts	23

6.5.3	Solution description	23

8.8.6	List of CAPIF events	24

6.5.4	Solution evaluation	24

6.6	Solution #5: AEF status and error reporting	24

6.6.1	General	24

6.6.2	Architecture Impacts	25

6.6.3	Solution description	25

6.6.4	Solution evaluation	27

6.7	Solution #6: Support of API provider administrator in CAPIF	28

6.7.0	General description	28

6.7.1	Solution description	28

6.7.2	Architecture Impacts	30

6.7.3	Corresponding APIs	30

6.7.4	Solution evaluation	30

6.8	Solution #7: API Provider domain and CAPIF Administrator policies for Open Discovery	30

6.8.1	General	30

6.8.2	Architecture Impacts	31

6.8.3	Solution description	31

8.3.3	Procedure	32

8.38	Open Discover service APIs	36

8.38.1	General	36

8.38.2	Information flows	36

8.38.3	Procedure	36

6.8.4	Solution evaluation	39

6.9	Solution #8: Providing API analytics for API administration	39

6.9.1	General	39

6.9.2	Architecture Impacts	39

6.9.3	Solution description	39

8.22.3	Procedure	41

6.9.4	Solution evaluation	41

6.10	Solution #9: Permission to usage of service API information	42

6.10.1	General	42

6.10.2	Architecture Impacts	42

6.10.3	Solution description	42

6.10.4	Solution evaluation	45

6.11	Solution #10: Solution for certificate unavailability	45

6.11.1	General	45

6.11.2	Architecture Impacts	45

6.11.3	Solution description	45

6.11.4	Solution evaluation	45

6.12	Solution #11: Heartbeat of the AEF	45

6.12.1	General	45

6.12.2	Architecture Impacts	45

6.12.3	Solution description	45

6.12.4	Solution evaluation	46

6.13	Solution #12: Determination of Service API and AEF unavailability	46

6.13.1	General	46

6.13.2	Architecture Impacts	46

6.13.3	Solution description	46

6.3.5	API publishing function	49

6.13.4	Solution evaluation	49

6.14	Solution #13: CCF support of Roaming Considerations for Service API invocation	49

6.14.1	General	49

6.14.2	Architecture Impacts	50

6.14.3	Solution description	51

6.14.4	Solution evaluation	58

6.15	Solution #14: Handling of Service API invocations based on location of API invoker	58

6.15.1	General	58

6.15.2	Architecture Impacts	58

6.15.3	Solution description	58

8.11.3	Procedure	59

6.x	Solution #x: <Solution Title>	60

6.x.1	General	60

6.x.2	Architecture Impacts	60

6.x.3	Solution description	60

6.x.4	Solution evaluation	60

7	Deployment scenarios	60

7.1	General	60

7.x	Deployment model #x: <title>	60

8	Business Relationships	61

8.1	General	61

9	Overall evaluation	61

9.1	Evaluation of architecture enhancements	61

9.1.x	Evaluation of architecture option #x	61

9.2	Evaluation of key issues	61

9.2.1	Evaluation of key issue #x	61

9.2.2	Evaluation of key issue #x	61

9.2.3	Evaluation of key issue #3	61

10	Conclusions	61

10.1	Architectural conclusions	61

10.2	Key issues conclusions	61

10.2.x	Conclusions of key issue #x	61

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

CAPIF as specified in 3GPP TS 23.222 [2] is a common API framework for enabling exposure of service APIs and access control of the service APIs. CAPIF is now considered by different 3GPP groups (e.g. SA2, SA5) and external bodies like GSMA, ETSI, ORAN Alliance to enable exposure of APIs. In Rel-19, CAPIF was enhanced to support, authentication and authorization interactions between Resource Owner and Authorization Functionality, UE-deployed API invoker accessing resources not owned by that UE, more granular access control (e.g., at the level of service operations or resources) and further enhancements considering AEF status. The security aspects for the CAPIF is specified in 3GPP TS 33.122 [3].

This TR provides a study of potential enhancements to CAPIF under the scope provided in clause 1.

# 1	Scope

The present document studies the potential enhancements to CAPIF (as specified in 3GPP TS 23.222 [2]) to support potential requirements addressing e.g. handling of AEF unavailability, location granularity, use cases related to CAPIF administrator emerging from practical field experiences e.g. in management of API provider domain functions, new potential CAPIF enhancements identified from MSED support, charging, and exposure framework requirements from SDOs/industry forums such as GSMA, ETSI OpenCAPIF, etc.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs".

[3]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[4]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[5]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[6]	3GPP TR 33.700-23: "Study on security aspects of CAPIF phase 4".

[7]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service"

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

API provider administrator: An authorized user with special permissions for API provider domain operations.

Exposure roaming agreement: A formal contract between (typically two) CSPs for allowing API Invokers of the home network to use and receive services when the UE is roaming to a visited network.

Roaming policies for exposure: The roaming policies for exposure consist of the conditions that enable an API Invoker to use and receive services when the UE is roaming to a visited network. The roaming policies for exposure can be based e.g., on CSP defined conditions or can be based on API Invoker provided preferences.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

BSS	Business Support System

CA	Certificate Authority

# 4	Key issues

## 4.1	Key issue #1: Enhancements to API provider function to handle API invoker status change

### 4.1.1	Description

Background: As specified in the clause 6.1 of 3GPP TS 33.122 [3], the API invoker obtains onboarding enrolment information from the API provider domain. Without onboarding enrolment information provided by the API provider domain, the API invoker would not be able to perform onboarding to CCF. The API provider domain (for example, Gaming server) will provide the onboarding enrolment information to the API invoker (for example, Gamin client on the UE) for example if the user has subscribed to (or paid for) the service provided by the API provider domain (for example, Gaming service). So, API provider domain can play a role in enabling API invoker onboarding.

Use cases:

1)	In certain scenario, the user of the UE (on which the API invoker is available) could decide to not continue receiving service from the AEF, and the API invoker would like e.g. to unsubscribe the service (e.g. to not pay for the service). In such scenario, the API provider domain would need to ensure that the API invoker cannot access any service APIs exposed by the AEF of the API provider domain after unsubscription.

2)	Out of band service unsubscription: Service subscription/unsubscription is often a business process that might involve multiple "elementary" services and that is coordinated by a BSS entity. Currently, the API provider domain can support the creation of onboarding credentials which can be used by the subscriber during onboarding the client on the UE (i.e. API invoker). However, the API provider domain currently does not support scenarios related to out of band service unsubscription.

3)	Blocking rogue API invokers: API invokers can prove to be rogue (e.g. invoking APIs even after rejected or expired authorization or even after the maximum invocation rate limit is crossed), or cannot be eligible anymore for API usage (e.g. due to unpaid bills). In such scenario, the API provider domain would need to ensure that the API invoker cannot access any service APIs exposed by the AEF of the API provider domain

Possible gap: Currently, in clause 8.23.3 of 3GPP TS 23.222 [2], AEF revokes the authorization of the API invoker to access specific service API based on a limited set of use cases, e.g. access control. However, there is no provision for API providing function to handle scenarios where e.g. the API invoker unsubsribes or became rogue.

### 4.1.2	Open issues

The following open issues are required to study:

1)	whether and how to handle the scenario where API invoker whose status has changed in the API provider domain, e.g. the API invoker has unsubscribed the services from the API provider domain or being rogue?

## 4.2	Key issue #2: Enhancements to API administration in CAPIF domain and API provider domain

### 4.2.1	Description

3GPP SA1 has specified the service requirements for exposure in 3GPP TS 22.261 [5] including service exposure, information exposure, etc. And 3GPP SA6 has already specified many APIs in various SEAL specifications, EDGEAPP specification etc. And 3GPP SA6 has defined CAPIF as a framework to support the management of such APIs exposure.

API provider administrator: An authorized user with special permissions for the API provider to perform administration of the service APIs.

Below updated definition for API management function in 3GPP TS 23.222 [2] shows the relationship of the API provider administrator and the API management function:

API management function: The entity which enables the API provider administrator at the API provider domain to perform administration (e.g. minimizing downtime, ensuring high availability and performance, identifying potential threats) of the service APIs.

API provider administrator (leveraging API management function) and CAPIF administrator plays a pivotal role in managing and overseeing the CAPIF operations, ensuring smooth deployment, maintenance, and updates, and monitoring performance to meet service level agreements (SLAs). As an API provider administrator and CAPIF administrator, tracking and analysing API usage is crucial for ensuring optimal performance, security, and user satisfaction. However, currently in the CAPIF 3GPP TS 23.222 [2] there is limited support for administrator role via API management function. That is, API management function can Monitor service API invocation and Query service API log for auditing purpose. Hence, going by emerging trends the current support to administrator needs enhancement e.g. via analytics data.

Some key types of API analytics that would be useful for an API provider administrator and CAPIF administrator are Usage Metrics, Error Tracking, User Behaviour Analysis, Security Monitoring, Performance Optimization, Business Insights, Compliance and Auditing.

By leveraging these analytics, API provider administrators and CAPIF administrator can ensure that their APIs are reliable, secure, and aligned with business objectives.

NOTE:	When the CAPIF provider and the service API provider are the same, then the API provider administrators and CAPIF administrator can be the same administrator.

### 4.2.2	Multiple API provider domains service API information

As per 3GPP TS 23.222, using Logging server API invocations procedure, the AEF logs the service API invocations information at the CCF. CCF has service API invocation logs related to AEFs from multiple API provider domains.

The service API invocation logs at CCF belongs to the respective API provider domain. In services such as analytics, the service API information belonging to multiple API provider domains can be used for generating the aggregated analytics results, considering the appropriate permissions are in place. It is up to the API provider to allow the usage of API provider domain’s service API information for aggregated analytics services and to indicate the API provider domain preference to obtain aggregated analytics results, including observations from other API provider domains.

Currently it is not covered the generation of analytics considering the aggregation of multiple API provider domain’s related service API invocation logs.

### 4.2.3	Open issues

To support the enhancements to API administrator role, the following aspects need to be studied:

-	What are the key types of API analytics that would be useful for an API provider administrator?

-	How to provide API analytics for the CAPIF entities in the API provider domain?

-	whether and what are the key types of API analytics that would be useful for an CAPIF administrator?

-	whether and how to provide API analytics to the CAPIF administrator?

-	whether and how the API provider permits the usage of service API information related logs to generate aggregated analytics with information from other API provider domains.

-	whether and how the analytics consumer’s can indicate preferences about obtaining aggregated analytics results with information from other API provider domains.

## 4.3	Key issue #3: Certificate unavailability

### 4.3.1	Description

As described in clause 6.1 of 3GPP TS 33.122 [3], a prerequisite to onboarding an API invoker to CAPIF, an API invoker is required to obtain onboarding enrolment information from the API provider domain (see step 1 of the referenced clause). This information is necessary to enable the API invoker to authenticate with the CAPIF core function (CCF) and establish secure communication over which to perform the onboarding process. The enrolment information must include details of the CCF (i.e., its address in the form of a URI and root certificate authority (CA) certificate) and includes an onboarding credential (i.e., an access token). With this information the API invoker can request to onboard to the CCF (clause 8.1 of 3GPP TS 23.222 [2]) and, if successful, receives its onboarding information in the response, which also includes the CCF assigned API invoker ID. A key attribute of the onboarding information response is the certificate assigned to the API invoker, which is signed e.g., by the CCF CA. API providers are also assigned signed certificates when registering to the CCF.

The problem identified is that an entity (API Invoker or API Provider) can no longer manage their resources at the CCF if they lose access to a valid certificate e.g., certificate expiry). This means for example:

-	The API Provider can not update/delete/create new AEFs, APFs or update/delete AMF.

-	The API Invoker entity cannot perform any operation with CCF (update/delete onboarding information, request security context, …).

### 4.3.2	Open issues

This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.

NOTE:	Coordination will be required with SA3 for the identification of the aspects of certificate lifecycle management. that can affect the CAPIF framework

## 4.4	Key Issue #4: Service API and AEF operational status

### 4.4.1	Description

Currently, the CCF obtains the service API status (e.g., active, inactive) via the API management function as described in clause 6.3.6 and 6.4.8 of 3GPP TS 23.222, or via the APF publish and publish updated procedure.

However, in some scenarios the API Provider Domain is unable to provide the status of Service APIs as the connection between the CAPIF core function and the API provider domain functions is disrupted due to e.g., network issues. In other scenarios, CCF cannot be aware whether an AEF is operational or not where AEFs can be disconnected/unreachable or do not work properly. The consequence is that there are scenarios in which the CCF can be unaware of whether an AEF and the API(s) it offers are active and able to offer service. Of note, registration of AEF does not include time-to-live (TTL) information, so an AEF can be registered indefinitely whether they are up or not.

The implementation and deployment of API provider domain functions may impact the service API status availability to the CCF via APF or API management function when considering the connectivity. The AEF, APF, API management functions in the API provider domain can be implemented in different entities and deployed in different locations. E.g., multiple AEF function instance is distributed deployed and the APF and the API management function instance are individually deployed in a centralized location. In this case, the network connectivity between the CCF and the AEF is different from the network connectivity between the CCF and the APF or API management function. In another example, the AEF and APF may be implemented and deployed together but the API management function is deployed in different locations. In this case, the network connectivity between the CCF and the AEF+APF is different from the network connectivity between the CCF and API management function.

In clause 8.6.2.1 of 3GPP TS 23.222 [2] specifying the contents of the service API update request sent from the APF to CCF there is a note stating that "how to monitor service API status when the APF is unable to update service API status is not specified in this release".

Examples of how this issue has been solved in other SA6 specifications include implementation of a heartbeat mechanism between servers, which is further specified by SA5 in their specification. There are some similar mechanisms:

-	For the AEF is the NEF instance, the NEF Function IOC includes the managedNFProfile, where a heartBeatTimer is included indicating the time between two consecutive heart-beat messages from an NF Instance to the NRF defined in seconds.

-	For AEF is the the EES/EAS/ECS case, similarly, the EAS Function in SA5 specification contains the EASProfile, where an eASAvailabilityReportingPeriod indicates to the EES how often it needs to check the EAS's availability after a successful registration (See EAS Availability Reporting Period defined in clause 8.2.4 in TS 23.558 [2]).

Now the CAPIF supports the dynamic routing of service API invocation and topology hiding. In those cases, the API invocation is firstly received and handled by one AEF (AEF#1) acting as service communication entry point, then AEF#1 will forward it to the destination AEF (AEF#2) for handling service API invocation. During service API invocation stage, if the AEF(AEF#2) does not work well, then the problem becomes how to route the service API invocation to an AEF providing such service API and work well. i.e., the service API status on this AEF is in active status.

### 4.4.2	Open issues

-	This key issue will study approaches to address scenarios in which the API Provider Domain is unable to provide the status of its AEFs with associated Service APIs and whether and how to enhance the architecture and related functions to handle the scenarios in which the connection between the CAPIF core function and the API provider domain functions is disrupted due to network issues.

-	During service API invocation stage, if the one AEF does not work well, how to route the service API invocation to another AEF providing such service API and work well. i.e., the service API status on this AEF is in active status.

## 4.5	Key issue #5: Roaming Considerations for Service API Invocation

### 4.5.1	Description

One existing gap in current deployments, detected by field experiences, is how to avoid in a controlled way that a service API invocation is served when, e.g., the UE is roaming and no exposure related roaming agreements with the visiting network exists in the home network or indicates that the service should not be delivered over a visiting network.

E.g., a QoS boost is not enforced in a roaming network:

PLMN A may define a strict QoS policy for inbound roamers, to ensure the network capacity is mainly enjoyed by home users. PLMN B, based on the service agreement with the Application Provider X about QoS boost, knows beforehand whether the QoS boost can be applied for its outbound roamers in PLMN A. If PLMN B does not apply roaming control for the QoS boost service API invocation by Application Provider X, the QoS request will be rejected by PLMN A, wasting unnecessarily network resources, having useless inter-PLMN signaling, and having KPIs degradation in both PLMN-A&B since the requests are eventually rejected.

Another problem exists when, e.g., the application invoker preferences to access a service API depends on whether the UE is roaming.

To ensure the correct handling of a Service API invocation, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies and application constraints. To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to be aware of the serving PLMN of the UE. To avoid increased complexity in the API Provider domain exposed service APIs and fragmentation derived from different solutions for roaming handling (e.g. per API Provider domain), the service API should not be forced either to become aware of the serving PLMN of the UE.

A solution in this area would enhance CAPIF to enable the control of service API invocations when the UE is roaming. The solution should consider efficient retrieval of roaming information from the Core Network (e.g. avoiding separated interactions with Core Network per service API invocation) and should keep CAPIF agnostic of the access network (i.e., agnostic of whether the access network is 5G, 4G, etc).

### 4.5.2	Open issues

To support the control of service API invocations when the UE is roaming, the following aspects need to be studied:

-	How to determine and ensure correct handling on Service API invocations, based on:

-	whether roaming agreements and policies between CSPs needed for exposure; and

-	whether application preferences related to roaming of the UE.

NOTE 1:	This KI ensures to keep CAPIF agnostic of the access network (i.e., agnostic of whether the access network is 5G, 4G, etc.).

NOTE 2:	Aspects of local access to service APIs in the VPLMN are not included in this present document. This key issue focuses on considering UE roaming information to access service APIs.

Editor's Note:	Core network roaming aspects in this key issue needs to be updated considering service area information of the service API.

## 4.6	Key issue #6: Serving Area Considerations for Service API Invocation

### 4.6.1	Description

One existing challenge is how to ensure that a service API is only invoked when the UE is within a designated service area aligned e.g. with the application preferences. Some services are only valid or cost-effective when the UE is physically inside a predefined service area or meets application-specific location preferences. Today, API invokers must monitor UE presence and prevent service API invocations when the UE is outside the service area, which increases complexity and slows time-to-market for application developers.

The key objective of this Key Issue would be to develop an efficient way to determine the UE’s presence within this service area to prevent unnecesary or unsupported service API invocations.

### 4.6.2	Open issues

To support the control of service API invocations when the UE is within or outside a designated service area, the following aspects need to be studied:

-	How to determine and ensure correct handling on Service API invocations based on location of API invoker.

## 4.7	Key issue #7: Open Discovery Policies

### 4.7.1	Description

One existing gap in current deployments, is how the CAPIF Core Function can become aware of the Service API information that can be disseminated when a requester (not identified in the CAPIF Core Function) invokes the Open Discovery service API.

To ensure a controlled handling of Open Discovery API, knowledge of the service API information that can be disseminated should be known in advance.

### 4.7.2	Open issues

To support the control of the service API information that can be disseminated when a requester invokes the Open Discovery API the following aspects need to be studied:

-	How to enhance the existing handling of Open Discovery API in CCF to support the provisioning of API Provider domain preferences about the dissemination of service API information to a requester, and the existing CCF configuration by the CAPIF Administrator to control the dissemination of service API information to a requester.

## 4.x	Key issue #x: <title>

### 4.x.1	Description

This section provides details about the key issue – by providing possible gaps.

### 4.x.2	Open issues

This section lists open issues to be studied.

# 5	Architecture enhancements

## 5.x	Option #x: <title>

### 5.x.1	Description

This section provides details about the proposed architecture enhancements.

# 6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | … |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |
| Sol #2 | X |  |  |  |  |  |  |  |
| Sol #3 | X |  |  |  |  |  |  |  |
| Sol #4 |  |  |  | X |  |  |  |  |
| Sol #5 |  | X |  | X |  |  |  |  |
| Sol #6 |  | X |  |  |  |  |  |  |
| Sol #7 |  |  |  |  |  |  | X |  |
| Sol #8 |  | X |  |  |  |  |  |  |
| Sol #9 |  | X |  |  |  |  |  |  |
| Sol #10 |  |  | X |  |  |  |  |  |
| Sol #11 |  |  |  | X |  |  |  |  |
| Sol #12 |  |  |  | X |  |  |  |  |
| Sol #13 |  |  |  |  | X |  |  |  |
| Sol #14 |  |  |  |  | X | X |  |  |
| Sol #... |  |  |  |  |  |  |  |  |

## 6.2	Solution #1: CAPIF Administrator revocation of API Invoker enrollment information

### 6.2.1	General

This solution relates to KI #1.

KI#1 identifies several use cases where the API invoker may have changed its status in the API provider domain after onboarding in the CAPIF provider domain. E.g., the API invoker decides to terminate the service agreement with the API provider, or the API Provider decides to terminate the service agreement with the API Invoker because e.g., unpaid bills.

With the current level of specification, the termination of the service agreement between an API Invoker and an API Provider is communicated to the CAPIF Provider (e.g. CSP) via off-line mechanisms out of CAPIF procedures (e.g. BSS specific procedures), and it is not further specified how this termination is provisioned into the CAPIF Provider.

This solution proposes that the CAPIF Administrator can update the API Invoker profile according to the API Provider domain provided information to the CAPIF provider, e.g., to update the set of enrolled APIs and to remove the API Invoker authorization information from the CCF, as necessary.

### 6.2.2	Architecture Impacts

There are no architecture impacts.

### 6.2.3	Solution description

#### 6.2.3.0	General

The following changes are proposed to 3GPP TS 23.222 [2]:

-	Update of Annex E to include the CAPIF administrator provisioning of per API provider domain and API Invoker service API enrollement information.

##### 6.2.3.0.1	Updates to TS 23.222 Annex E

Annex E (normative):
Configuration data for CAPIF

The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator.

The configuration data for CAPIF is specified in table E-1.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information. (see NOTE 2) |
|  | > Network slice identification |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE 1) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE 1) |
| Subclause 4.10 | List of access control policy (for an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.1 | Onboarding information per API provider domain and API invoker. > API Invoker Identification > List of service APIs allowed for enrolment information (see NOTE 3) |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]. NOTE 3: When the CAPIF administrator removes a service API from the list of authorized service APIs for enrollment for an API invoker, the authorization to access service API corresponding to the API invoker and the removed service API is revoked from CAPIF. |  |

#### 6.2.3.1	Information flows

Not applicable.

### 6.2.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.3	Solution #2: API invoker enrolled Service API notification

### 6.3.1	General

This solution relates to KI #1.

KI#1 identifies several use cases where the API invoker may have changed its status in the API provider domain after onboarding in the CAPIF provider domain. E.g., the API invoker decides to terminate the service agreement with the API provider, or the API Provider decides to terminate the service agreement with the API Invoker because e.g., unpaid bills.

When the API invoker is in the onboarded status, the enrollment information for the list of enrolled APIs may change. The CAPIF API events API, using the event for Service API of interest, is able to notify about the service APIs that are enrolled, but currently does not indicate the API invoker that enrolled the service API, which could be useful for the API Provider domain, in case of revocation of API Invoker’s enrolled service APIs by CAPIF Administrator, and in case of API Invoker is modifying the list of enrolled service APIs.

This solution proposes that the subscription to Service API of interest event is updated to indicate whether the subscription is for a specific API invoker or for any API invoker and whether the notification, in case of report of enrolled service APIs, needs to include the identity of the API Invoker that successfully enrolled the service API.

### 6.3.2	Architecture Impacts

This solution proposes an enhancement to the CAPIF_Events API, with the definition of a new subscription filter for the Service API of interest event, and a new subscription indication to include the Identifier of the API Invoker that successfully enrolls the reported service API in the event report of enrolled service API.

### 6.3.3	Solution description

#### 6.3.3.0	General

Figure 6.x.3.0-1 illustrates the procedure for the API provider domain subscribing with the CCF to notification of API Invoker API Invoker enrolled service API event, and the notification from the CCF to API provider domain about changes in the list of service APIs the API invoker successfully enrolled for.

Figure 6.3.3.0-1: Procedure for API invoker enrolled Service API notification

1.	The API Provider domain subscribes with the CCF to notifications about API Invoker enrolled service APIs event (Service API of interest events). The API Provider domain may provide the API Invoker information to identify the API invokers to be notified of and may provide as notification filter the service API(s) the API Invoker may be including for enrolment within the onboarding operation.

2.	The CCF determines the already onboarded API invokers.

3.	The CCF returns the response to the API provider subscription request to API Invoker enrolled service APIs event (Service API of interest events). If the API Provider requested immediate notification, the CCF returns the information available for the currently onboarded API Invokers, including the list of enrolled service APIs per API Invoker.

4.	API Invoker X requests onboarding in the CCF and includes a set of service APIs for enrollment.

5.	The CCF returns a successful API Invoker response, including the successfully enrolled APIs.

6.	The CCF notifies the API Provider domain about a new API invoker enrolled service APIs event (Service API of interest events), including the successfully enrolled Service APIs for the API Invoker.

7.	The API Provider domain acknowledges the notification.

8.	API Invoker X requests to update the set of enrolled service APIs (disenroll and/or enroll).

9.	The CCF acknowledges the request and returns the successfully enrolled service APIs.

10.	The CCF triggers a notification to the API Provider domain indicating the updated list of service APIs the API invoker successfully enrolled for.

11.	The API Provider domain returns a notification request acknowledgement to the CCF.

12.	API invoker X offboards.

13.	CCF acknowledges the offboarding request.

14.	CCF notifies the API provider domain to indicate that there is no service API enrolled for the API Invoker.

#### 6.3.3.1	Information flows

### 8.8.6 List of CAPIF events Table 8.8.6-1 provides a non-exhaustive list of CAPIF events. Table 8.8.6-1: List of CAPIF events Events; Events Description. Availability of service APIs; Availability events of service APIs (e.g. active, inactive). Service API updated; Events related to change in service API information. Monitoring service API invocations; Events corresponding to service API invocations. API invoker status; Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.). API topology hiding status; Events related to API topology hiding status in CAPIF (created, revoked). System related events; Alarm events providing fault information. Performance related events; Events related to system load conditions. Service API of interest events; Events indicating whether a service API is enrolled or a service API is discovered. E.g.1, a consumer (e.g. AMF) may be interested in being notified about whether a specific service API is of interest for the API invoker(s) (e.g., the service API is within the list of service APIs the API Invoker included during onboarding or the service API is within the list of service APIs the API invoker requested during discovery requests and received in responses). In this case, it is not necessary to include the API Invoker Id in the event report. E.g.2, a consumer (e.g. an API Provider Domain) is interested in being notified with the list of service APIs an API invoker successfully enrolled for. In this case, the subscription request includes an indication to include the API invoker Id in the event report..

### 6.3.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.4	Solution #3: API provider domain remove enrolled APIs for API invoker

### 6.4.1	General

The API invoker receives onboarding enrolment information from the API provider domain, which is used by the API invoker to onboard to CCF. If the API invoker has unsubscribed to the services provided by the API provider domain, then the it is required remove enrolled APIs for the APIs provided by API provider domain.

### 6.4.2	Architecture Impacts

This solution does not impact existing CAPIF architecture.

### 6.4.3	Solution description

Figure 6.4.3.1 illustrates the procedure for offboarding API invoker from the CCF initiated by the AEF.

Pre-conditions:

1.	The API invoker had received the onboarding enrolment information from the API provider domain and successfully onboarded to the CCF..

2.	The API provider domain function (i.e. AMF) decides to remove enrolled APIs for the API invoker based on certain condition (e.g. the API invoker has unsubscribed to the services provided by the API provider domain functions).

Figure 6.4.3.1: Procedure for removing enrolled APIs for API invoker

1.	The AMF triggers the remove enrolled APIs for the API invoker.

2.	The AMF sends remove enrolled APIs request for the API invoker to the CAPIF core function with the details of the API invoker. The request message may include the reason for the removing enrolled APIs.

3.	Upon receiving the request, the CAPIF core function invalidates the API invoker authorization corresponding to the service APIs. The CCF may decide to offboard the API invoker if all enrolled APIs are removed from the API invoker profile.

4.	If CCF has decided to offboard the API invoker, the CCF sends an offboard API invoker notify to the API invoker. The notification message may include the reason for the offboarding.

4.	The CAPIF core function sends a remove enrolled APIs response to the AMF.

#### 6.4.3.1	Information Flows

The Offboard API invoker request and Offboard API invoker response messages are same as specified in clause 8.2.2.1 and clause 8.2.2.2 respectively. Table 6.x.3.1-1 describes the information flow offboard API invoker notify from the CAPIF core function to the API invoker.

Table 6.4.3.1-1: Offboard API invoker notify

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker whose has been Offboard. |
| List of APIs | M | List of APIs to be removed for enrolment for the API invoker |
| Cause | M | The cause for revoking the API invoker. |

### 6.4.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.5	Solution #4: Enhanced CAPIF events for AEF status

### 6.5.1	General

This solution is related to key issue#4, specifically on introducing new CAPIF events to handle the scenarios in which the connection between the CAPIF core function and the API provider domain functions is disrupted due to network issues.

### 6.5.2	Architecture Impacts

It is based on existing CAPIF architecture specified in 3GPP TS 23.222 [2].

### 6.5.3	Solution description

In the existing CAPIF framework, introducing a heartbeat mechanism similar to the EAS Availability Reporting Period would enable CCF (CAPIF core function) to detect whether the connection between the CAPIF core function and an AEF is disrupted or not.

Editor's note:	Whether there may be other mechanism than heartbeat and how the heartbeat mechanism can be enabled to detect AEF availability is FFS.

When such connectivity problems arise, the AEF associated APIs may become unavailable. The consequence is that an ongoing service API invocation may terminate abruptly or time out, leading to transactions incomplete. Without timely notifications, API invoker may repeatedly attempt reconnections which may lead to unnecessary resource waste and heighten network congestion.

The following scenario is considered for this solution: when the CAPIF core function and a API provider domain functions are not co-located, CCF detects the AEF is disconnected due to network issue and there are multiple service APIs are accessible through this AEF by API invoker(s). Though AEF availability could be reported through service API availability and service API update events, in order to reduce the number of Service API updated event notification, the CCF may consider following options:

Option 1:  CCF sends a new CAPIF event (i.e., AEF status in CAPIF) without sending Service API updated event notification if the CCF consider Service API information is changed due to AEF status changing.

Option 2: CCF sends AEF status in existing CAPIF event.

Editor's Note:	Whether the AEF identity information should be provided to the API invoker or should be hidden is FFS.

Editor's Note:	Which option is selected is FFS.

AEF status can be defined as the follows:

-	Unreachable: Unable to communicate with the CCF (e.g., due to network issues), though the AEF itself may remain functional.

-	Reachable: The connection between the AEF and CCF is restored to normal after network issues.

Regarding this new CAPIF event, as an example, once the CCF detects the connection between the CAPIF core function and an AEF is disrupted,  it generates event of AEF status and sends the event notification including AEF Identity information and the AEF status (i.e., unreachable) to the subscribing entity (e.g., the API invoker), then the API invoker may stop invoking the AEF associated APIs. After that, once the connection between the AEF and CCF is restored to normal, the CCF generates event of AEF status and sends the event notification including AEF Identity information and the AEF status (i.e., reachable) to the subscribing entity (e.g., the API invoker), then the API invoker may resume invoking the AEF associated APIs.

#### 6.5.3.1	Information flows

The following updates (in bold) to clause 8.8.6 of 3GPP TS 23.222 [2] are proposed for supporting new events for AEF status change if option 1 is adopted.

### 8.8.6 List of CAPIF events Table 8.8.6-1 provides a non-exhaustive list of CAPIF events. Table 8.8.6-1: List of CAPIF events Events; Events Description. Availability of service APIs; Availability events of service APIs (e.g. active, inactive). Service API updated; Events related to change in service API information. Monitoring service API invocations; Events corresponding to service API invocations. API invoker status; Events related to API invoker status in CAPIF (onboarded, offboarded, onboarding criteria not met status along with the criteria information that is not met.). API topology hiding status; Events related to API topology hiding status in CAPIF (created, revoked). System related events; Alarm events providing fault information. Performance related events; Events related to system load conditions. CAPIF-1/1e interaction events; Events related to CAPIF-1/1e interactions for a service API (e.g. onboarded API invokers). AEF status; Events related to AEF status (e.g. unreachable, reachable) in CAPIF.

### 6.5.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.6	Solution #5: AEF status and error reporting

### 6.6.1	General

This solution relates to KI #2 and KI #4. As per KI #2, CCF can collect data required for error tracking. This solution proposes API invoker to share the error reports to the CCF, Further, as per KI #4, CCF needs to know when AEF is unavailable. As stated in the key issue - the API Provider Domain is unable to provide the status of its AEFs.

### 6.6.2	Architecture Impacts

This solution does not impact existing CAPIF architecture.

### 6.6.3	Solution description

In this solution, if CCF requires the status report from the API invoker, the CCF provides the indication about the same in the discovery response (as specified in clause 6.6.3.1.

#### 6.6.3.1	Enhancements to Service API discover response

The service API discovery response is enhanced with new information element as described below:

#### 8.7.2.2 Service API discover response Table 8.7.2.2-1 describes the information flow service API discover response from the CAPIF core function to the API invoker. Table 8.7.2.2-1: Service API discover response Information element; Status; Description. Result; M; Indicates the success or failure of the discovery of the service API information. Service API information (see NOTE 2); O (see NOTE 1, NOTE 3); The service API information as specified in Table 8.3.2.1-1, except for the service API status (e.g. active, inactive).. Interface information; O (NOTE 3); The interface details (e.g. IP address, port number, URI) of the requested service API(s). CAPIF core function identity information; O (see NOTE 1, NOTE 3); Indicates the CAPIF core function serving the service API category provided in the query criteria. AEF status reporting; O; Indicates to report AEF status towards the CCF if the service API invocation fails. Possible values are true or false. That means the API invoker sends report to CCF for reachability failure, e.g. service unavailable,.. > frequency; O; Indicates after number of failures or time after which the AEF status needs to be reported. NOTE 1: The service API information or the CAPIF core function identity information or both shall be present if the Result information element indicates that the service API discover operation is successful. Otherwise, both shall not be present. NOTE 2: If topology hiding is enabled for the service API, the interface details shall be the interface details of AEF acting as service communication entry point for the service API. NOTE 3: "Interface information" IE shall not be present when "Service API information" or "CAPIF core function identity information" is present..

#### 6.6.3.2	AEF Status reporting

##### 6.6.3.2.1	General

The procedure API invoker to report AEF status. The procedure may be invoked when service API invocation towards the AEF fails.

##### 6.6.3.2.2	Information flows

###### 6.6.3.2.2.1	AEF status reporting request

Table 6.6.3.2.2.1-1 describes the information flow AEF status reporting request from the API invoker to the CAPIF core function.

Table 6.6.3.2.2.1-1: AEF status reporting request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | Identity information of the API invoker |
| Service API information | M | The service API information includes the service API name, API provider name, and other information as present in Table 8.3.2.1-1 of 3GPP TS 23.222 [2]. |
| Interface information | O | The interface details (e.g. IP address) of the requested service API(s) |
| Status information | O | Indicates status of the interface (e.g. unauthorized or bad response, etc.) - Mapped to server side HTTP errors that are generated when API invoker requests are received by AEF and may indicate AEF unavailability. |

###### 6.6.3.2.2.2	AEF status reporting response

Table 6.6.3.2.2.2-1 describes the information flow AEF status reporting response from the CAPIF core function to the API invoker.

Table 6.6.3.2.2.2-1: AEF status reporting response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the availability check is in-progress and result will be notified later, success availability check or failure of the request |
| Interface information | O (NOTE 1) | The updated interface details (e.g. IP address of the requested and failed service API(s)) if the service API is available |
| Failure cause | O (NOTE 2) | Indicates reason for failure |
| NOTE 1: These IEs are included for success response. NOTE 2: This IE is included for failure response. |  |  |

Editor's Note:	Notification of AEF status reporting to be notified when the AEF status reporting response is “in-progress” is FFS.

Editor's Note:	Whether more information than Interface Information is needed in the sucessful AEF status reporting response is FFS.

##### 6.6.3.2.3	Procedure

Figure 6.6.3.2.3-1 illustrates the procedure for AEF status reporting.

Pre-conditions:

1.	The API invoker is onboarded, discovered the service APIs and received service authorization to invoke the service APIs.

Figure 6.6.3.2.3-1: AEF Status reporting

1.	The API invoker (on the UE) sends the AEF status reporting request message to the CCF. The request message includes the parameters as specified in Table 6.6.3.2.2.1-1.

2.	Upon receiving the request, the CCF checks the registration status and published API status of the AEF (i.e. interface details). If the registration is active and published APIs status is also active, the CCF sends the in-progress response to the API invoker. The CCF may proceed with step-3 to check whether the information about the AEF in AMF indicates the AEF active or not, because e.g. the internal CCF information about registration and status is obsolete. For application or protocol errors, the response indicates success, and further steps will not be executed.

3.	The CCF sends AEF status check request to AMF including end point details of the AEF(s) whose status is required to check. The AMF checks whether the AEF is active or not. The AMF sends the response message back to CCF. The response includes the AEF status (either active or in-active).

NOTE 1:	For application or protocol errors, the CCF should not check with AMF for AEF inactivity.

NOTE 2:	If connectivity between CCF and AMF is not present, the response from AMF may not be received by the CCF.

4.	If no response received from the AMF (i.e. request timeout occurred) or the AEF is in-active (as informed by AMF), the CCF updates the registration status of the API provider domain function and also updates the status of the published service APIs.

Editor's Note:	AMF action when AEF is inactive is FFS.

5.	The CCF sends the final response to the API invoker with updated service API list.

6.	If the AMF subscribed with the CCF to notifications about AEF/Service API status changes (availability/unavailability or service API information update), the CCF also sends the monitoring service API event notification to the AMF which indicates the change of registration status of the AEF and list of APIs marked as inactive.

### 6.6.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.7	Solution #6: Support of API provider administrator in CAPIF

### 6.7.0	General description

Here are some key types of API analytics that would be useful for an API provider administrator and their applications:

1.	Usage Metrics

-	Requests per Second (RPS): Monitor the number of API requests to identify peak usage times and ensure the system can handle the load.

-	Total Requests: Track the overall number of API calls to understand user engagement and growth trends.

-	Response Times: Measure the time taken to process requests, helping to identify performance bottlenecks.

2.	Error Tracking

-	Error Rates: Monitor the frequency of errors (e.g., 4xx client errors, 5xx server errors) to quickly address issues and improve reliability.

3.	User pattern

-	Most Used Endpoints: Identify which API endpoints are most frequently accessed to optimize resources and focus on high-demand features.

-	Usage patterns:: Usage patterns to improve API offerings and enhance user experience.

4.	Security Monitoring

-	Unauthorized Access Attempts: Detect and log attempts to access unauthorized endpoints or resources.

-	Rate Limiting Violations: Track instances where users exceed rate limits to prevent abuse and ensure fair usage.

5.	Performance Optimization

-	Latency Analysis: Identify slow endpoints and optimize them for faster response times.

6.	Auditing

-	Audit Logs: Maintain detailed logs of API activity for auditing purposes and to investigate potential security incidents.

Examples of how these Analytics are leveraged:

-	Proactive Problem Solving: Early detection of issues allows for quick resolution, minimizing downtime and user frustration.

-	Resource Allocation: Insights into usage patterns help allocate resources efficiently, ensuring high availability and performance.

-	Security Enhancement: Monitoring for suspicious activities helps in identifying potential threats and implementing necessary security measures.

-	Usage pattern: Understanding user pattern and preferences aids in developing new features and services.

-	Efficient Management: Optimizing API performance reduces operational efficiency by minimizing resource wastage and improving efficiency.

### 6.7.1	Solution description

Figure 6.7.1-1 illustrates the procedure for support of API provider administrator in CAPIF.

Pre-conditions:

1.	Authorization details of the API analytics consumer e.g. AMF are available with the CAPIF core function.

Figure 6.7.1-1: Procedure for support of API provider administrator in CAPIF

1.	For querying service API analytics, the API analytics consumer e.g. AMF triggers Query service API analytics request to the CAPIF core function. Table 6.7.1-1 describes the information flow Query service API analytics request from the API analytics consumer to the CAPIF core function.

Table 6.7.1-1: Query service API analytics request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | Identity information of the entity querying service API analytics request |
| Analytics Query information | M | List of query filters related to the required analytics, such as Usage Metrics, Error Tracking, User pattern, Security Monitoring, Performance Optimization, Auditing |
| Preference Information | M | List of preferences related to the usage of set of API provider domain’s service API information (e.g., service API invocation logs). |
| >preference value | M | Preference issued: “Allowed”, “Not Allowed”. |

Editor’s Note: Whether and how to indicate the target identifiers information for which the preference value is applicable is FFS.

2.	Upon receiving the query service API analytics request, the CAPIF core function accesses and/or generates the necessary analytics information. If Preference Information received in step 1, then the CCF generates the requested analytics using the service API invocation logs from multiple API provider domains, as given in the Preference Information.

Editor’s Note: Whether and how to handle the scenario where Analytics consumer preference and API provider domain’s preference are conflicting, is FFS.

3.	The CAPIF core function returns the API analytics information to the API analytics consumer in the query service API analytics response. Table 6.7.1-2 describes the information flow Query service API analytics response from the CAPIF core function to the API management function.

Table 6.7.1-2: Query service API analytics response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of query service API analytics request |
| API invocation analytics information | O (see NOTE) | API analytics information such as Usage Metrics (Requests per Second, Total Requests, Response Times), Error Tracking (Error Rates), User pattern (Most Used Endpoints, Usage patterns), Security Monitoring (Unauthorized Access Attempts, Rate Limiting Violations), Performance Optimization (Latency Analysis), Auditing (Audit Logs) |
| NOTE: Information element shall be present when result indicates success. |  |  |

### 6.7.2	Architecture Impacts

Supporting API analytics functionality is required.

The API analytics function has the following capabilities:

1.	Requests per Second (RPS), Total Requests, Response Times

2.	Error Rates

3.	Most Used Endpoints,  Usage patterns

4.	Unauthorized Access Attempts, Rate Limiting Violations, Anomaly detection

5.	Latency Analysis, Throughput Analysis

6.	Audit Logs

If the analytics functionality is internal to the CAPIF core function, the CAPIF-5 reference point as specified in 3GPP TS 23.222, between the AMF and the CAPIF core function is used for AMF as an API analytics consumer fetching API analytics from the CCF.

When the API provider requests the usage of service API invocation logs belonging to multiple API provider domains, then the AMF includes the preference information in Query Service API Analytics request message. Else, the AMF may generate the API analytics based on service API invocation logs available locally or may fetch the service API information (e.g., service API invocation logs) from CCF using Auditing service API invocation procedure as specified in clause 8.22 of 3GPP TS 23.222.

Editor's Note:	Performing analytics inside or outside of CCF (e.g. ADAES) is FFS and outside is based on data available with CCF (i.e. CCF being data source) is FFS.

### 6.7.3	Corresponding APIs

Editor's Note:	APIs are FFS.

### 6.7.4	Solution evaluation

Editor's Note:	Solution evaluation is FFS.

## 6.8	Solution #7: API Provider domain and CAPIF Administrator policies for Open Discovery

### 6.8.1	General

This solution relates to KI #7.

KI#A describes one existing gap in current deployments, about how the CAPIF Core Function can become aware of the Service API information that can be disseminated when a requester (not identified in the CAPIF Core Function) invokes the Open Discovery service API.

This solution proposes enhancements to the CAPIF framework to ensure correct handling of Open Discovery API invocations by:

-	Getting awareness about API Provider preferences for the dissemination of service API information by the update of the Publish service API.

-	Getting awareness about the CAPIF administrator configuration for the dissemination of service API information.

### 6.8.2	Architecture Impacts

This solution does not have arhictectural impacts.

### 6.8.3	Solution description

#### 6.8.3.0	General

##### 6.8.3.0.1	Introduction

This solution proposes two CAPIF enhancements:

1.	Enhancements on Service API Publish to enable the APF can indicate the API Provider domain preferences about the service API information that can be disseminated using the Open Discovery API (see clause 6.8.3.0.2 and 6.8.3.1)

2.	Enhancements on the Configuration data stored in the CAPIF core function and provided by the CAPIF Administrator (see clause 6.8.3.0.3).

The related enhancements on the Open Discovery procedure are described in clause 6.8.3.0.4.

##### 6.8.3.0.2	Service API Publish with API Provider domain preferences for Open Discovery

### 8.3.3 Procedure Figure 8.3.3-1 illustrates the procedure for publishing the service APIs. The service API publish mechanism is supported by the CAPIF core function. Pre-conditions: 1. Authorization details of the APF are available with the CAPIF core function. 2. API invokers may have subscribed with the CAPIF core function to obtain new service API information. Figure 8.3.3-1: Publish service APIs 1. The API publishing function sends a service API publish request to the CAPIF core function, with the details of the service API. If the service API is to be shared to other CAPIF core functions, the shareable information and the CAPIF provider domain information are included. The API publishing function may also include information about whether the dissemination of the service API information is allowed during Open Discovery. 2. Upon receiving the service API publish request, the CAPIF core function checks whether the API publishing function is authorized to publish service APIs. If the check is successful, the service API information provided by the API publishing function is stored at the CAPIF core function (API registry). 3. The CAPIF core function provides a service API publish response to the API publishing function indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.

##### 6.8.3.0.3	CCF configuration with CAPIF Administrator preferences for Open Discovery

Annex E (normative): Configuration data for CAPIF The configuration data is stored in the CAPIF core function and provided by the CAPIF administrator. The configuration data for CAPIF is specified in table E-1. Table E-1: Configuration data for CAPIF Reference; Parameter description. Subclause 4.2.2; List of published service API discovery restrictions. > Service API identification. > API invoker identity information. > Service API category. Subclause 4.3.2; List of service API authorization restrictions per API invoker. > API Invoker identity information. > Service API identification. > Per resource, operation authorization information. (see NOTE 2). > Network slice identification. Subclause 4.7.2; List of service API log storage durations. > Service API identification. > Service API log storage duration (in hours) (see NOTE 1). Subclause 4.7.4; List of API invoker interactions log storage durations. > Service API identification. API invoker interactions log storage duration (in hours) (see NOTE 1). Subclause 4.10; List of access control policy (for an API provider domain) per API invoker and optionally per network slice. > Volume limit on service API invocations (total number of invocations allowed). > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed). > Rate limit on service API invocations (allowed service API invocations per second). > Service API identification. > API invoker identity information. > Network Slice Info. Subclause 8.34.3; Group context information. > Group identifier. > List of UE IDs (group members). > UE identifier of the Group Resource Owner. > Authorization information related to the list of supported applications. >> Application identifier. >> Purpose. >> Scope. Subclause 8.38; List of service API allowed for dissemination during Open Discovery. > Service API identification. > Service API category. NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]..

##### 6.8.3.0.4	Updated procedure for Open Discovery considering API Provider Domain and CAPIF administrator preferences

## 8.38 Open Discover service APIs ### 8.38.1 General The following procedure in this subclause corresponds to supporting open discover service APIs. Such open discovery operation allows requestor to discover service API information (e.g., API name, API category) about the available set of APIs offered by CCF before onboarding. ### 8.38.2 Information flows #### 8.38.2.1 Open Service API discover request Table 8.38.2.1-1 describes the information flow open service API discover request from the requestor to the CAPIF core function. Table 8.38.2.1-1: Open Service API discover request Information element; Status; Description. Query information; O (see NOTE); Criteria for discovering matching service APIs. It may contain Service API information described in table 8.3.2.1-1, except service API interface details.. NOTE: It should be possible to discover all the service APIs.. #### 8.38.2.2 Open Service API discover response Table 8.38.2.2-1 describes the information flow open service API discover response from the CAPIF core function to the requestor. Table 8.38.2.2-1: Open Service API discover response Information element; Status; Description. Result; M; Indicates the success or failure of the open discovery of the service API information. Service API information; O (see NOTE); Filtered service APIs information that can be shared to the requestor not a recognized user of the CAPIF. It may contain Service API information described in table 8.3.2.1-1, except service API interface details.. NOTE: The service API information shall be present if the Result information element indicates that the open service API discover operation is successful. Otherwise, it shall not be present.. ### 8.38.3 Procedure Figure 8.38.3-1 illustrates the procedure for open discover service APIs. The open service API discovery mechanism is supported by the CAPIF core function. Pre-conditions: 1. The requestor is not a recognized user of the CAPIF. 2. The CAPIF core function is aware of the Service API information that can be disseminated according to the requestor who is not a recognized user of the CAPIF. Figure 8.38.3-1: Open Discover service APIs 1. The requestor sends an open service API discover request to the CAPIF core function. It may include the query information from the requestor. NOTE: The security aspects of this procedure are unspecified in this release of the specification. Editor's note: The security aspects of this procedure depend on the evolution of SA3 on Open Discovery security. 2. Upon receiving the open service API discover request, if query information is included, the CAPIF core function retrieves the stored service API(s) information as per the query information in the open service API discover request, otherwise all stored service API(s) information is retrieved. Further, since the requestor is not a recognized user of CAPIF, the CAPIF core function applies the Open Discovery dissemination information received from the APF (clause 8.3.2.1), and if its not available, the configured Open Discovery dissemination information (specified in Annex E) and performs filtering of service APIs information. 3. The CAPIF core function sends an open service API discover response to the requestor with the filtered list of service API information.

#### 6.8.3.1	Information flows

#### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 2); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods, information about dissemination of the service API information or category during Open Discovery.. Shareable information; O (see NOTE 1); Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained.. NOTE 1: If the shareable information is not present, the service API information is not allowed to be shared. NOTE 2: The active and inactive states of the service APIs of 3GPP AEFs are based on the respective combinations of the management states (administrative, operational) specified in the management domain in TS 28.541 [17], TS 28.532 [18] and TS 28.538 [19].. Editor's note: Whether it is possible to include requester information as criteria for service API information dissemination is FFS. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use..

### 6.8.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.9	Solution #8: Providing API analytics for API administration

### 6.9.1	General

This solution relates to KI #2. In this solution, the CCF is acting as data source to ADAES as already defined in  clause 8.6 of 3GPP TS 23.436 [7], and provides more data (like API Usage Info (Total Requests, Response Times), Error Tracking logs (Error Types)).

### 6.9.2	Architecture Impacts

This solution does not impact CAPIF architecture.

### 6.9.3	Solution description

#### 6.9.3.1	Enhancements to Procedure for supporting service API analytics

The clause 8.6 of 3GPP TS 23.436 [7] is already defined to get Service API analytics. The solution proposes to reuse clause 8.6 of 3GPP TS 23.436 [7] to provide API analytics for API administration, with following enhancements:

1)	The CAPIF administration and/of API provider domain administration are the consumer for the procedure defined in clause 8.6 of 3GPP TS 23.436 [7];

2)	The ADAE Server and AMF (of API provider domain) are not same entity;

2)	the notification message defined in clause 8.6.3.6 of 3GPP TS 23.436 [7] is enhanced as follows:

#### 8.6.3.6 Service API analytics notification Table 8.6.3.6-1 describes information elements for the service API analytics notification to the subscriber/consumer from the ADAE server. Table 8.6.3.6-1: Service API analytics notification Information element; Status; Description. Service API information; M; The service API name or type for which analytics apply.. Analytics ID; O; The identifier of the analytics event. This ID can be for example “service API analytics”.. Analytics Output; M; Stats or predictions based on abstracted or anonymized API logs (for example number of failure API invocations, API availability, frequency and occurrence of API version changes, API location changes for the target API, API throttling events, number of API invocations for a given area and time, API load statistics for a given edge network, Usage Metrics, Error Tracking, Most Used Endpoints, Latency Analysis, Audit Logs, etc).. Confidence level; O; For predictive analytics, the achieved confidence level can be provided.. Area of validity; O; Geographical or topological area for which the analytics apply..

#### 6.9.3.2	Enhancements to Auditing service API invocation

In order to enable ADAES to get service API logs, the clause 8.22.3 of 3GPP TS 23.222 [2] is enhanced as follows.

### 8.22.3 Procedure Figure 8.22.3-1 illustrates the procedure for auditing service API invocation. Pre-conditions: 1. Service API invocation logs are available at the CAPIF core function. 2. Authorization details of the AMF are available with the CAPIF core function. Figure 8.22.3-1: Procedure for auditing service API invocation 1. For auditing service API invocations, the requestor (e.g. API management function, ADAES) triggers query service API log request to the CAPIF core function. 2. Upon receiving the query service API log request, the CAPIF core function checks whether requestor is authorized or not, and if authorized the CCF accesses the necessary service API log information for auditing purposes. 3. The CAPIF core function returns the log information to the requestor (e.g. API management function, ADAES) in the query service API log response. NOTE: The API management function detecting abuse of the service API invocation and actions, subsequent to query service API log response, are out-of-scope of this specification.

#### 6.9.3.3	Information flows

The solution is not defining any new information flows, and the enhancements to existing information flows is captured in clause 6.x.3.1 and clause 6.x.3.2.

### 6.9.4	Solution evaluation

The solution is related to KI#2 and proposes to reuse the existing procedure defined in clause 8.6 from 3GPP TS 23.436 [7]. The solution further enhances the existing procedure defined in clause 8.22 of 3GPP TS 23.222 [2]. The solution enhances the information flow to add API usage information and Error tracking information as data towards ADAES. The CAPIF administration and/of API provider domain administration can use existing ADAES service to get required analytics. The solution proposes minimum changes to the existing procedure and feasible.

## 6.10	Solution #9: Permission to usage of service API information

### 6.10.1	General

This solution relates to KI #2.

KI#2 identifies the use case of controlling the usage of an API provider domain’s service API information such as Service API invocation logs, for analytics services to application entities outside of API provider domain. The open issue is captured on how the API provider permits the usage of service API information.

This solution proposes the enhancement of CAPIF Publish Service API procedure with permission information. The APF includes the permission information in the Service API Publish Request message, as specified in Publish service APIs procedure in clause 8.3 of 3GPP TS 23.222 [2]. The permission information is used by CCF in considering the API provider domain’s service API information for offering analytics (e.g., generating API analytics) or any other service offerings.

### 6.10.2	Architecture Impacts

This solution enhances the existing Publish Service API procedure. There are no new functional elements or reference points introduced in this solution. There are no architecture impacts.

### 6.10.3	Solution description

This solution enhances the existing Publish Service API procedure with permission information. The enhancements proposed to Publish service APIs procedure in clause 8.3 of 3GPP TS 23.222 [2] are shown in below (indicated in bold).

Figure 8.3.3-1 illustrates the procedure for publishing the service APIs. The service API publish mechanism is supported by the CAPIF core function. Pre-conditions: 1. Authorization details of the APF are available with the CAPIF core function. 2. API invokers may have subscribed with the CAPIF core function to obtain new service API information. Figure 8.3.3-1: Publish service APIs 1. The API publishing function sends a service API publish request to the CAPIF core function, with the details of the service API. If the service API is to be shared to other CAPIF core functions, the shareable information and the CAPIF provider domain information are included. The request may include Permission Information, indicating to the CCF to apply the Permission Information for the usage or sharing of the requesting APF’s API provider domain’s service API invocation logs for offering of services (e.g., analytics) to entities outside of API provider domain. - 2. Upon receiving the service API publish request, the CAPIF core function checks whether the API publishing function is authorized to publish service APIs. If the check is successful, the service API information and permission information provided by the API publishing function is stored at the CAPIF core function (API registry). CCF applies the Permission Information for the usage of the requesting APF’s API provider domain service API invocation logs for offering of services (e.g., analytics) 3. The CAPIF core function provides a service API publish response to the API publishing function indicating success or failure result and triggers notifications to subscribed API invokers as described in subclause 8.8.4.

#### 6.10.3.1	Information flows

This solution proposes the update to Service API publish request in clause 8.3.2.1 of 3GPP TS 23.222 [2], with permission information. The updates are shown below (in bold).

#### 8.3.2.1 Service API publish request Table 8.3.2.1-1 describes the information flow service API publish request from the API publishing function to the CAPIF core function. Table 8.3.2.1-1: Service API publish request Information element; Status; Description. API publisher information; M; The information of the API publisher may include identity, authentication and authorization information. Service API information; M (see NOTE 2); The service API information includes the service API name, API provider name (optional), List of public IP ranges of UEs (optional), service API category (e.g. V2X, IoT), service API status (e.g. active, inactive), communication type, description, Serving Area Information (optional), AEF location (optional), interface details (e.g. IP address, port number, URI), protocols, version numbers, data format, Service KPIs (optional), service API operation(s) and resource(s) (optional), and Network Slice Info (optional), security methods.. Shareable information; O (see NOTE 1); Indicates whether the service API information or the service API category can be published to other CCFs. And if sharing, a list of CAPIF provider domain information where the service API information or the service API category can be published is contained.. Permission Information; O; List of permissions related to the usage of API provider domain’s service API invocation logs.. >permission value; M; Permission issued: “Allowed”, “Not Allowed”.. NOTE 1: If the shareable information is not present, the service API information is not allowed to be shared. NOTE 2: The active and inactive states of the service APIs of 3GPP AEFs are based on the respective combinations of the management states (administrative, operational) specified in the management domain in TS 28.541 [17], TS 28.532 [18] and TS 28.538 [19].. The Service KPIs is defined as below: Table 8.3.2.1-2: Service KPIs Information element; Status; Description. Maximum Request rate; O; Maximum request rate from the API Invoker supported by the server.. Maximum Response time; O; The maximum response time advertised for the API Invoker's service requests.. Availability; O; Advertised percentage of time the server is available for the API Invoker's use.. Available Compute; O; The maximum compute resource available for the API Invoker.. Available Graphical Compute; O; The maximum graphical compute resource available for the API Invoker.. Available Memory; O; The maximum memory resource available for the API Invoker.. Available Storage; O; The maximum storage resource available for the API Invoker.. Connection Bandwidth; O; The connection bandwidth in Kbit/s advertised for the API Invoker's use..

Editor’s Note: Whether and how to indicate the target identifiers information for which the permission value is applicable is FFS.

### 6.10.4	Solution evaluation

Editor’s Note: The solution evaluation is FFS.

## 6.11	Solution #10: Solution for certificate unavailability

### 6.11.1	General

This solution relates to KI #3.

### 6.11.2	Architecture Impacts

This solution does not impacts CAPIF architecture.

### 6.11.3	Solution description

The KI#3  has following open issue: “This key issue will study approaches to managing entities (i.e., API Invoker or API Provider) without a valid CCF issued and signed certificate, specifically how such entities can be deleted from the CCF including associated resources to the entities like Security_Context associated to API Invokers and API resources published by APFs served by AEFs.”

As per 3GPP TS 33.122 [3], clause 6.1, (1) the CAPIF Core Function may deliver a certificate to the API invoker in the onboarding response (figure 6.1-1, step 4) or , (2) the CAPIF Core Function may receive an API invoker's client certificate in the Onboard API Invoker request message (NOTE 2 of clause 6.1).

In the second case, for the API Invoker certificate (as well as for the AEF(s), APF(s) and AMF(s) certificates), any certificate management consideration is out of the scope of SA6. In both cases, the certificate life cycle management is within scope of SA3, and any further action in SA6 depends on the progress of the KI#4 of 3GPP TR 33.700-23 [6].

#### 6.11.3.1	Information flows

Editor's note:	The impact on the information flows of procedures in 3GPP TS 23.222 [2] depends on the outcome of KI#4 of 3GPP TR 33.700-23 [6] in SA3.

### 6.11.4	Solution evaluation

The solution is related to KI#3. Certificate management is within the scope of SA3, and any further action in SA6  depends on the progress of the KI#4 of 3GPP TR 33.700-23 [6].

NOTE:	SA6 impacts due to the SA3 outcome of the study on security aspects of credentials unavailability will be determined during normative phase.

## 6.12	Solution #11: Heartbeat of the AEF

### 6.12.1	General

This solution is addressing key issue #4. It introduces the heartbeat mechanism between the AEF and the CAPIF core network.

### 6.12.2	Architecture Impacts

There are no architecture impacts.

### 6.12.3	Solution description

The AEF maintains the heartbeat with the CAPIF core function via a implemention specific heartbeat timer.

Currently, the API invoker is involved in the service API discovery and the service API invocation procedures, which may need the AEF status.

The CAPIF core function may determine the AEF status (i.e., connectivity with CCF) based on the heartbeat from the AEF(s) and use it to filters the service API during service API discovery procedure.

### 6.12.4	Solution evaluation

This solution is proposed to use the heatbeat mechanism as an example and implementation specific option to make the CCF aware of the AEF status (connectivity with the AEF).

## 6.13	Solution #12: Determination of Service API and AEF unavailability

### 6.13.1	General

This solution relates to KI #4.

KI#4 describes one existing gap in current deployments, about how to get awareness of API status (AEF/Service API) when the connection between the CAPIF core function and the AMF and/or APF, entities in charge of indicating AEF/Service API status, is disrupted due to e.g., network issues. Moreover, the AMF/APF may be unaware of API status when the AEF becomes not operational because the AEF are disconnected/unreachable or do not work properly, and consequently, they’re not able to properly interact with the the CCF.

This solution discusses the currently availale procedures in CAPIF and proposes some enhancements in the description of the CAPIF framework to enable the determination AEF/Service API availability based on current functional level of 3GPP TS 23.222 [2] and industry adopted procedures.

### 6.13.2	Architecture Impacts

There are no architecture impacts.

### 6.13.3	Solution description

#### 6.13.3.0	General

##### 6.13.3.0.1	Introduction

This clause describes the optional procedure that may be supported by the CCF to detect unavailability of the AEF, APF and AMF.

In the current level of the specification of 3GPP TS 23.222 [2]:

-	The AMF manages the registration of the AEF, APF and AMF functions in the CCF using the CAPIF_API_Provider_Management_API as described in clauses 8.28, 8.29 and 8.30. The AMF monitors the status of the service APIs as specified in clause 6.3.6 and 6.4.8.

-	The APF publishes API provider the service APIs information in order to enable the discovery of service APIs by the API invoker (see clause 6.3.5)

Based on implementation specific means, and current functional level of 3GPP TS 23.222 [2], when the AMF detects that a service API becomes unavailable (3GPP TS 23.222 [2], clause 6.3.6 and 6.4.8), the AMF, based on implementation specific means, can notify the APF, which could update the CCF with the change on the service API status from Active to Inactive (3GPP TS 23.222 [2], clause 6.3.5).

In case the AMF becomes unavailable, neither the APF and hence the CCF can become aware of changes on the service API status. If during AMF unavailability the service API becomes also unavailable, whether the CCF detects AEF/Service API availability is implementation specific. Equivalently, in case the APF becomes unavailable, whether the CCF detects AEF/Service API availability is implementation specific.

The procedure is described in clause 6.x.3.0.2.

The potential impacts in 3GPP TS 23.222 [2] are collected in clause 6.x.3.0.3.

##### 6.13.3.0.2	Detection of AEF/Service API availability

This clause describes the optional procedure that may be supported by the CCF to detect AEF/Service API unavailability.

Figure 6.13.3.0.2-1 describes the detection of AEF/Service API availability.

Preconditions:

- The AMF supports the detection of AEF/Service API availability by implementation specific means.

- The APF, based on AEF/Service API availability detected by the AMF, supports the update of the active/inactive status for a Service API.

- The CCF determines AMF/APF/AEF availability by implementation specific means.

Figure 6.13.3.0.2-1: Detection of AEF/Service API availability

1.	The API invoker subscribes with the CCF to notifications about Availability of Service API events (i.e., Service API Active/Inactive).

2.	The CCF acknowledges the subscription.

3.	The AEF becomes unavailable.

4.	The AMF detects AEF failure (e.g. based on heartbeat or any other implementation specific mechanism), the APF becomes aware of the AEF unavailability and determines the AEF/Service API change of status to Service API Inactive.

5.	The APF updates the service API(s) status in the CCF using the Publish service API.

6.	The CCF acknowledges the update.

7.	The service API status event triggers the notification to the API invoker about Service API status Inactive.

8.	The API invoker acknowledges the notification.

9.	AEF becomes available again.

10.	The AMF detects AEF recovery, the APF becomes aware of the AEF unavailability and determines the AEF/Service API change of status to Service API Active.

11.	The APF updates the service API(s) status in the CCF using the Publish service API.

12.	The CCF acknowledges the update.

13.	The service API status event triggers the notification to the API invoker about Service API status Active.

14.	The API invoker acknowledges the notification.

The CCF, based on the received Service API status, may determine further CAPIF-1/1e replies to the API invoker.

Based on implementation specific procedures, the CCF could monitor AEF status (e.g. based on heartbeat or any other implementation specific mechanism) as an additional supervision mechanism to detect AEF status changes, in the event, e.g., of AMF or APF unavailability issues. The CCF, based on the detected AEF status, may proceed according to the active/inactive service API status changes, and e.g., may trigger the notification to the API Invoker e.g. about Service API information change. The CCF monitoring of AEF status is out of the scope of TS 23.222.

##### 6.13.3.0.3	Potential impacts in 3GPP TS 23.222 [2]

### 6.3.5 API publishing function The API publishing function enables the API provider to publish the service APIs information in order to enable the discovery of service APIs by the API invoker. The API publishing function consists of the following capability: - Publishing the service API information of the API provider to the CAPIF core function; - Updating the Service API information related to the published API; and - Retrieving service API information from the CAPIF core function.

#### 6.13.3.1	Information flows

This solution does not impact the existing Information flows.

### 6.13.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.14	Solution #13: CCF support of Roaming Considerations for Service API invocation

### 6.14.1	General

This solution relates to KI #5.

KI#5 describes one existing gap in current deployments, experienced in live deployments, about how to avoid in a controlled way that a service API call is progressed towards the VPLMN when e.g. no exposure related roaming agreements with the visiting network exists in the home network. Another problem exists when, e.g., the application invoker preferences to access a service API depends on whether the UE is roaming, or the API invoker preferences indicate that the service API (e.g. a subscription to a event) will be only monitored and detected when the UE is in the HPLMN only.

To ensure the correct handling of a Service API invocation, knowledge of whether the UE is roaming or not is necessary for evaluating roaming policies for exposure, specially for information retrieval (i.e. simple request/response, with not further interactions required). To reduce barriers in the usage of service APIs by applications, the API invoker should not be aware and should not be forced to be aware of the serving PLMN of the UE. To avoid increased complexity in the API Provider domain exposed service APIs and fragmentation derived from different solutions for roaming handling (e.g. per API Provider domain), the service API should not be forced to become aware of the serving PLMN of the UE.

This solution proposes the following enhancements to the CAPIF Common Function of the CAPIF framework to enable the determination and correct handling of service API invocations when the UE is roaming:

- Enhancing the CCF to support the storage and evaluation of roaming policies for exposure based on available UE roaming information.

- Enhancing CAPIF-3 reference point enabling the AEF to obtain and enforce roaming authorization grants.

- Getting awareness about UE roaming information (whether the UE is roaming or not and where (serving PLMN ID)), by the update of the CAPIF Architecture with the N33 reference point, that enables the CCF to interact, when necessary, with the CSP to retrieve and subscribe UE roaming information via CSP exposed service APIs.

Editor's Note:	Whether roaming policies are required or not it is FFS.

Editor's Note:	It is FFS the analysis of a solution based on AEF evaluation of roaming policies.

### 6.14.2	Architecture Impacts

The CAPIF architecture is impacted by the definition of the N33 reference point, that enables the CCF to interact, when necessary, with the CSP to retrieve UE roaming information via CSP exposed service APIs.

Figure 6.14.2-1 shows the updated reference point based functional model for the CAPIF.

Figure 6.14.2-1: Functional model for the CAPIF

The functional model for CAPIF is described in 3GPP TS 23.222 [2] clause 6.2.0.

The reference point based model described in figure 6.x.2-1 represents the N33 reference point, between the CCF and the 3GPP Core Network, which supports the subscription or retrieval of UE roaming information by the CCF from the related exposed Service API. The security aspects of the N33 reference point are specified by SA3.

### 6.14.3	Solution description

#### 6.14.3.0	General

##### 6.14.3.0.1	Roaming Considerations during Service API invocation

Figure 6.14.3.0.1-1 illustrates the procedure to determine and handle the roaming information during service API invocation, showing the interaction between the different CAPIF functions when Roaming policies for exposure need to be applied.

Pre-conditions:

1.	The API Invoker is onboarded and has received an API Invoker identity.

2.	The CAPIF Core Function is configured with roaming policies for exposure (e.g., to restrict service API invocations to when the UE is roaming in a list of VPLMNs).

Figure 6.14.3.0.1-1: Procedure for handling Roaming information during API invocation

- The API invoker requests to obtain authorization information to invoke the service API exposed by the API exposing function (AEF) as specified in 3GPP TS 23.222 [2], clause 8.11.

- The API invoker sends a Service API invocation request to the AEF.

- The API invoker request is authenticated. The AEF may interface with the CCF to obtain API invoker information to obtain security related information as specified in 3GPP TS 23.222 [2], clause 8.16.

- The AEF determines that roaming policies need to be evaluated for the service API invocation and interacts with the CCF by sending an Apply roaming policy request. Based on the accessed service API, the Apply roaming policy request may indicate the retrieval of the roaming authorization information or may also indicate the subscription to changes in the roaming authorization grant.

- The CCF evaluates the roaming authorization policies for the Service API and API invoker. If it is needed to retrieve roaming information for the involved UE, the CCF interacts with the 3GPP Core Network using the N33 reference point (e.g., invoking the Nnef_Location service specified in 3GPP TS 23.502 [x], clause 5.2.6.21).

- The CCF returns to the AEF the result of the roaming policy authorization in the roaming authorization information.

- If roaming authorization is granted, the service API exposed by the AEF applies the regular service logic, e.g., request QoS to 5GC as requested by the service API.

- The API invoker receives the service API invocation response.

##### 6.14.3.0.2	Roaming Authorization changes

Figure 6.14.3.0.2-1 illustrates the procedure where either the UE roaming information changes or the roaming policies change while the service delivered by the service API is running, showing the interaction between the different CAPIF functions when roaming policies for exposure need to be applied.

Pre-conditions:

1.	The API Invoker is getting service from the authorized service API as described in Figure 6.14.3.0.1-1.

Figure 6.14.3.0.2-1: Procedure for handling roaming information changes during service delivery

1a.	The CCF, in collaboration with the 3GPP Core Network, detects that roaming information changes for a UE for whom a previous roaming authorization grant was provided.

1b.	The CCF detects a change in the roaming exposure policies.

2.	The CCF, re-evaluates roaming policies for exposure, and if the roaming authorization information changes, the CCF sends the Apply roaming notification service operation, indicating the roaming authorization information change.

3. The service API exposed by the AEF applies the regular service logic according to the changes in the roaming authorization information. E.g., if the roaming authorization is revoked, the service logic terminates the QoS request in 5GC).

##### 6.14.3.0.3	Apply Roaming Unsubscribe

Figure 6.14.3.0.3-1 illustrates the procedure where the service API invocation is terminated, showing the interaction between the different CAPIF functions when Roaming Control policies need to be applied.

Pre-conditions:

1.	The API Invoker is getting service from the authorized service API as described in Figure 6.14.3.0.1-1.

5

Figure 6.14.3.0.3-1: Procedure for Apply Roaming Unsubscribe

1.	If the AEF subscribed to roaming authorization information changes, the AEF terminates the subscription.

2.	The CCF acknowledges the unsubscription request.

3.	If there are no other service APIs that require to monitor UE roaming information changes, the CCF terminates the subscription to roaming information changes with the 3GPP Core Network.

#### 6.14.3.1	Information flows

Editor's Note:	It is FFS the analysis of a solution based on access control policies.

##### 6.14.3.1.1	Apply Roaming Policy subscription request

Table 6.14.3.1.1-1 describes the information flows for Apply Roaming Policy subscription request from the AEF to the CAPIF core function.

Table 6.14.3.1.1-1: Apply Roaming Policy subscription request

| Information element | Status | Description |
| --- | --- | --- |
| API invoker identity information | M | The information that determines the identity of the API invoker |
| Service API identification | M | The identification information of the service API for which invocation is requested. |
| Target UE | M | Target UE whose roaming information is used during roaming policies evaluation (i.e. GPSI). |
| Notification type | M | Indicates whether the notification is roaming information change event based or one time. |
| Notification reception information | O (NOTE) | The information of the subscribing entity for receiving notification about roaming information changes. |
| NOTE: The inclusion of this information element depends on whether the notification type is event based. |  |  |

##### 6.14.3.1.2	Apply Roaming Policy subscription response

Table 6.14.3.1.2-1 describes the information flows for Apply Roaming Policy subscription response from the CAPIF Core Function to the AEF.

Table 6.14.3.1.2-1: Apply Roaming Policy subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of service API invocation. |
| Subscription identifier | O | Identifier of the subscription to roaming authorization changes. This information element shall be present when the result is success and the notification type included in the subscription request indicated to notify based on roaming information change event. |
| Roaming Authorization information | O | The roaming authorization information indicates whether the service API invocation is authorized or not for the involved UE. This information element shall be present when the result is success. |
| Failure Cause | O | Failure Cause. This information element may be present when the result is failure |

##### 6.14.3.1.3	Apply Roaming Policy Notify request

Table 6.14.3.1.3-1 describes the information flows for Apply Roaming Policy notify request from the CAPIF Core Function to the AEF.

Table 6.14.3.1.3-1: Apply Roaming Policy Notify request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the apply roaming policy subscription |
| Roaming Authorization information | O | The roaming authorization information indicates whether the service API invocation is authorized or not for the involved UE. |

##### 6.14.3.1.4	Apply Roaming Policy unsubscription request

Table 6.14.3.1.4-1 describes the information flows for Apply Roaming Policy unsubscription request from the AEF to the CAPIF core function.

Table 6.14.3.1.4-1: Apply Roaming Policy unsubscription request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the apply roaming policy subscription |

#### 6.14.3.2	Updates in Configuration Data for CAPIF

The configuration data for CAPIF is specified in 3GPP TS 23.222 [2] table E-1. The updates in Configuration Data for CAPIF to support UE roaming information to authorize API invoker request are highlighted in Italic and Bold format.

Table E-1: Configuration data for CAPIF

| Reference | Parameter description |
| --- | --- |
| Subclause 4.2.2 | List of published service API discovery restrictions |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Service API category |
| Subclause 4.3.2 | List of service API authorization restrictions per API invoker |
|  | > API Invoker identity information |
|  | > Service API identification |
|  | > Per resource, operation authorization information. (see NOTE 2) |
|  | > Network slice identification |
| Subclause 4.7.2 | List of service API log storage durations |
|  | > Service API identification |
|  | > Service API log storage duration (in hours) (see NOTE 1) |
| Subclause 4.7.4 | List of API invoker interactions log storage durations |
|  | > Service API identification |
|  | API invoker interactions log storage duration (in hours) (see NOTE 1) |
| Subclause 4.10 | List of access control policy (for an API provider domain) per API invoker and optionally per network slice |
|  | > Volume limit on service API invocations (total number of invocations allowed) |
|  | > Time limit on service API invocations (The time range of the day during which the service API invocations are allowed) |
|  | > Rate limit on service API invocations (allowed service API invocations per second) |
|  | > Service API identification |
|  | > API invoker identity information |
|  | > Network Slice Info |
| Subclause 8.34.3 | Group context information |
|  | > Group identifier |
|  | > List of UE IDs (group members) |
|  | > UE identifier of the Group Resource Owner |
|  | > Authorization information related to the list of supported applications |
|  | >> Application identifier |
|  | >> Purpose |
|  | >> Scope |
| (new)Subclause 8.x.y | List of Roaming policies for exposure per API Invoker and per Service API: |
|  | > List of allowed VPLMN IDs. |
|  | > Service API identification |
|  | > API Invoker identity information |
| NOTE 1: If no value is set for the duration, the duration is assumed to be unlimited. NOTE 2: The API invoker request for access authorization to specific resources or operations of a Service API is specified in 3GPP TS 33.122 [12]. |  |

### 6.14.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 6.15	Solution #14: Handling of Service API invocations based on location of API invoker

### 6.15.1	General

This solution relates to KI #5 and KI #6.

### 6.15.2	Architecture Impacts

This solution has no architecture impacts.

### 6.15.3	Solution description

Clause 8.11 of 3GPP TS 23.222 [2] has been modified as follows:

### 8.11.3 Procedure Figure 8.11.3-1 illustrates the procedure for obtaining authorization to access the service API. Pre-condition: 1. The API invoker is onboarded and has received an API invoker identity. Figure 8.11.3-1: Procedure for the API invoker obtaining authorization for service API access 1. The API invoker sends an obtain service API authorization request to the CAPIF core function for obtaining permission to access the service API by including the API invoker information required for authorization, the information identifying the service API, and any information required for authentication of the API invoker. The request may include desired Network Slice Info of the service API. In addition to the information identifying the service API, the request may include finer granularity level for service API authorization e.g., service API operation and resource. 2. The CAPIF core function validates the authentication of the API invoker (using authentication information) and checks whether the API invoker is permitted to access the requested service API, considering also service API operation(s) and resource(s) restrictions locally configured, if available. The CCF may additionally obtain location of the API invoker by interacting with core network, and validates whether API invoker is within Serving Area of the service API for which authorization is requested. The CAPIF core function may additionally verify the Network Slice Info, e.g., check that the authorized Network Slice Info for the API invoker is included in the supported Network Slice Info for the indicated service API. Editor's Note: How CCF determines that the API invoker is on UE and its identity to interact with the CN to obtains the UE location is FFS. NOTE 1: The authorization of Network Slice Info is performed by the CAPIF core function within the PLMN trust domain. NOTE 2: The authentication process is specified in subclause 6.5.2 of 3GPP TS 33.122 [12]. NOTE 3: The CCF may reject the request if the API invoker is ourside the Serving Area of the service API. 3. Based on the API invoker's subscription information available in the CCF, the authorization information to access the service APIs, which may include access authorization per service operation, resource, optionally including access authorization per service operation and, optionally, resource, is sent to the API invoker in the obtain service API authorization response. NOTE 3: How the finer service API authorization granularity is indicated in the request from API invoker and is returned to the API invoker in the CCF authorization reply is specified in 3GPP TS 33.122 [12]. NOTE 4: The mechanism for distribution of the authorization information for the API invoker to the API exposing function over CAPIF-3 reference point is specified in subclause 6.5.2 of 3GPP TS 33.122 [12].

Further, the AEF upon receiving the service API invocation request can obtain API invoker’s location before serving the API invoker.

Editor’s Note:	The security aspects (including scenario where API invoker gets authorization token within service area but invokes outside the service area) are within scope of SA3.

Editor's Note:	How to handle ongoing service based on changes in UE location, is FFS.

#### 6.15.3.1	Information flows

The solution does not need any new information flows.

## 6.x	Solution #x: <Solution Title>

### 6.x.1	General

This solution relates to KI #N.

This section describes the high level principle of the solution.

### 6.x.2	Architecture Impacts

This section describes any architectural impacts based on the proposed solution.

### 6.x.3	Solution description

This section describes the solution in detail.

#### 6.x.3.1	Information flows

This section describes the information flow tables required for this solution.

### 6.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

# 7	Deployment scenarios

## 7.1	General

This section describes the general information about deployment scenarios.

## 7.x	Deployment model #x: <title>

This section describes the specific deployment scenario based on agreed solutions.

# 8	Business Relationships

## 8.1	General

This section describes the general information about business relationships.

# 9	Overall evaluation

## 9.1	Evaluation of architecture enhancements

### 9.1.x	Evaluation of architecture option #x

This section evaluates architecture option #x.

## 9.2	Evaluation of key issues

### 9.2.1	Evaluation of key issue #x

This section evaluates all solutions related to key issue #x.

### 9.2.2	Evaluation of key issue #x

This section evaluates all solutions related to key issue #x.

### 9.2.3	Evaluation of key issue #3

Changes in SA6 (if required) depend on the progress of the KI#4 of 3GPP TR 33.700-23 [6] and will be considered directly in normative phase.

# 10	Conclusions

## 10.1	Architectural conclusions

This section provides conclusion and suggestion for normative work for the architecture options studied in this TR.

## 10.2	Key issues conclusions

### 10.2.x	Conclusions of key issue #x

This section provides conclusion and suggestion for normative work for the solutions related to key issue #x.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA6#68 | S6-253193 |  |  |  | Skeleton | 0.0.0 |
| 2025-09 | SA6#68 | S6-253504 |  |  |  | Scope for the TR | 0.1.0 |
| 2025-09 | SA6#68 | S6-253505 |  |  |  | Introduction for the TR | 0.1.0 |
| 2025-09 | SA6#68 | S6-253500 |  |  |  | CAPIF_Ph4 Key Issue Certificate unavailability | 0.1.0 |
| 2025-09 | SA6#68 | S6-253501 |  |  |  | CAPIF_Ph4 Key Issue AEF operational status | 0.1.0 |
| 2025-09 | SA6#68 | S6-253721 |  |  |  | Enhancements to API invoker offboarding | 0.1.0 |
| 2025-09 | SA6#68 | S6-253775 |  |  |  | CAPIF_Ph4-KI on enhancements to CAPIF Admin | 0.1.0 |
| 2025-10 | SA6#69 | S6-254500 |  |  |  | Clause 3 | 0.2.0 |
| 2025-10 | SA6#69 | S6-254501 |  |  |  | Pseudo-CR on solution for KI#1, CAPIF Administrator revocation of API Invoker enrollment information. | 0.2.0 |
| 2025-10 | SA6#69 | S6-254502 |  |  |  | Pseudo-CR on solution for KI#1, API invoker enrolled Service API notification | 0.2.0 |
| 2025-10 | SA6#69 | S6-254503 |  |  |  | Pseudo-CR on solution for KI#1 – API invoker status change | 0.2.0 |
| 2025-10 | SA6#69 | S6-254507 |  |  |  | Pseudo-CR on Update key issue#4 | 0.2.0 |
| 2025-10 | SA6#69 | S6-254508 |  |  |  | Solution for KI# 4 on support for AEF operational status | 0.2.0 |
| 2025-10 | SA6#69 | S6-254730 |  |  |  | Pseudo-CR on Update key issue#2 | 0.2.0 |
| 2025-10 | SA6#69 | S6-254759 |  |  |  | Pseudo-CR on solution for AEF status reporting | 0.2.0 |
| 2025-10 | SA6#69 | S6-254764 |  |  |  | Pseudo-CR Solution for enhancing CAPIF Administrator | 0.2.0 |
| 2025-10 | SA6#69 | S6-254778 |  |  |  | Pseudo-CR on new KIs on Roaming Considerations for Service API Invocation | 0.2.0 |
| 2025-11 | SA6#70 | S6-255418 |  |  |  | Pseudo-CR on new KI on Open Discovery Policies | 0.3.0 |
| 2025-11 | SA6#70 | S6-255419 |  |  |  | Pseudo-CR on solution for Open Discovery policies Service API and AEF operational status | 0.3.0 |
| 2025-11 | SA6#70 | S6-255665 |  |  |  | Pseudo-CR on removing Editor’s Notes in Solution#1 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255421 |  |  |  | Solution to KI#2 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255422 |  |  |  | Pseudo-CR on Open issues to Key Issue #2 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255423 |  |  |  | Pseudo-CR on New solution to Key Issue #2 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255424 |  |  |  | Pseudo-CR on Updates to Solution #6 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255425 |  |  |  | pCR_FS_CAPIF_Ph4-Resolving EN about analytics | 0.3.0 |
| 2025-11 | SA6#70 | S6-255426 |  |  |  | Solution to KI#3 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255427 |  |  |  | New solution to KI#4 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255436 |  |  |  | Pseudo-CR on solution for KI#4, on Service API and AEF operational status | 0.3.0 |
| 2025-11 | SA6#70 | S6-255666 |  |  |  | Pseudo-CR on solution for KI#5, on CCF Support of Roaming Considerations for Service API Invocation | 0.3.0 |
| 2025-11 | SA6#70 | S6-255605 |  |  |  | Pseudo-CR on solution for Editor’s Notes in KI#5 | 0.3.0 |
| 2025-11 | SA6#70 | S6-255667 |  |  |  | Solution to KI#6 | 0.3.0 |