| 3GPP TR 23.700-52 V0.4.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on SEAL data delivery Phase 3; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Architecture requirements	8

5	Key issues	8

5.1	Key issue #1: Data delivery using multicast/broadcast from both 4G system and 5G system	8

5.1.1	Description	8

5.1.2	Open issues	9

5.2	Key issue #2: SEALDD-S and SEALDD-UU user plane	9

5.2.1	Description	9

5.2.2	Open issues	9

5.3	Key Issue #3: Application traffic pattern and SEALDD data delivery optimization	10

5.3.1	Description	10

5.3.2	Open Issues	10

5.x	Key issue #x: <Title>	10

6	Solutions	10

6.1	Mapping of solutions to key issues	10

6.1	Solution #1: SEALDD-S and SEALDD-UU user plane functionality description and protocol stack	11

6.1.1	Solution description	11

6.1.1.1	General	11

6.1.1.2	Protocol stack for SEALDD-UU	12

6.1.1.2.1	User plane protocol stack for SEALDD-UU	12

6.1.1.2.2	Control plane protocol stack for SEALDD-UU	12

6.1.1.3	Protocol stack for SEALDD-S	13

6.1.1.3.1	User plane protocol stack for SEALDD-S	13

6.1.1.3.2	Control plane protocol stack for SEALDD-S	13

6.1.1.4	User plane protocol stack examples for SEALDD-UU and SEALDD-S	14

6.1.1.4.1	General	14

6.1.1.4.2	Recommended protocol(s) for Regular Transmission	14

6.1.1.4.3	Recommended protocol(s) for Data Storage	14

6.1.1.4.4	Recommended protocol(s) for XR Transmission	15

6.1.1.4.5	Recommended protocol(s) for Transmission Quality Measurement	15

6.1.2	Architecture Impacts	15

6.1.3	Corresponding APIs	15

6.1.4	Solution evaluation	15

6.2	Solution #2: SEALDD enabled multicast data delivery	16

6.2.1	Solution description	16

6.2.1.1	General	16

6.2.1.2	Multicast data delivery service	16

6.2.1.2.1	Multicast data delivery service create	16

6.2.1.2.2	Multicast data delivery service update	18

6.2.1.2.3	Multicast data delivery service delete	19

6.2.2	Architecture Impacts	20

6.2.3	Corresponding services	20

6.2.4	Solution evaluation	20

6.3	Solution #3: SEALDD enabled broadcast data delivery	20

6.3.1	Solution description	20

6.3.1.1	General	20

6.3.1.2	Broadcast data delivery service	21

6.3.1.2.1	Broadcast data delivery service create	21

6.3.1.2.2	Broadcast data delivery service update	22

6.3.1.2.3	Broadcast data delivery service delete	23

6.3.2	Architecture Impacts	24

6.3.3	Corresponding services	24

6.3.4	Solution evaluation	24

6.4	Solution #4: SEALDD enabled Multicast/Broadcast data delivery service	24

6.4.1	Solution description	24

6.4.1.1	General	25

6.4.1.2	SEALDD enabled Multicast and Broadcast data delivery service	25

6.4.1.2.1	SEALDD enabled Multicast and Broadcast data delivery create procedure	25

6.4.1.2.2	SEALDD enabled Multicast and Broadcast data delivery update procedure	27

6.4.1.2.3	SEALDD enabled Multicast and Broadcast data delivery deletion procedure	28

6.4.2	Solution evaluation	29

6.x	Solution #x: <title>	29

6.x.1	Solution description	29

6.x.2	Architecture Impacts	29

6.x.3	Corresponding APIs	29

6.x.4	Solution evaluation	29

7	Overall evaluation	30

8	Conclusions	30

8.1	General conclusions	30

8.2	Conclusions of key issue #x	30

Annex A: Change history	31

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

SEALDD as defined in 3GPP TS 23.433 [2] provides several data delivery service based on the 3GPP network capabilities for the application service provider. In order to cater the various data delivery requirements (e.g., data delivery optimization based on traffic behaviour) of the application service, the SEALDD should continue the evolution and improvement.

This TR provides a thorough study of potential enhancements to SEALDD under the scope provided in clause 1.

# 1	Scope

The present document is to study on potential enhancements to Service Enabler Architecture Layer (SEAL) data delivery (DD). Specifically, the study is to address data delivery using network multicast/broadcast capability from both 4G and 5G, the SEAL-DD user plane (i.e., SEALDD-UU, SEALDD-S) aspects (e.g., protocol stack(s), user plane functionality description) and the data delivery of new application traffic pattern where the application layer interactions are non-continuous and each of the interactions may be independent with each other.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.433: "Service Enabler Architecture Layer for Verticals (SEAL); Data Delivery enabler for vertical applications".

[3]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services".

[4]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[5]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE)".

[6]	3GPP TS 26.502: "5G multicast-broadcast services; User service architecture".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architecture requirements

Editor's Note:	This clause describes architectural requirements

# 5	Key issues

## 5.1	Key issue #1: Data delivery using multicast/broadcast from both 4G system and 5G system

### 5.1.1	Description

Many services/applications like live broadcast, group communication, and group message delivery have requirements on the one-to-many transmission. Using 3GPP network multicast/broadcast transmission to delivery the one-to-many communication content can help simplify the application DL delivery e.g., only one copy of the content is suffcient.

3GPP system now support both the 4G eMBMS and the 5MBS. However, there are some issues to use the existing mechanisms defined in 3GPP TS 23.247 [3] and 3GPP TS 26.502 [6]:

-	Too many configurations options, interfaces: Before using 3GPP multicast/broadcast transmission, the VAL service provider needs to be aware of the configuration options listed in the annex A of in 3GPP TS 23.247 [3] that the NEF and the MBSF/MBSTF could be optional. Also, the VAL service provider needs to differentiate the transport Only Mode and Service Mode to select the correct interface from a list of interfaces (e.g., the control plane interface including N33, N5, Nmb13, Nmb10, xMB-C, MB2-C) towards the AF. Such complexity makes it very difficult for the VAL service provider to use the 3GPP multicast/broadcast transmission.

-	Application needs to differentiate and handle 4G eMBMS and 5MBS individually. When required both the eMBMS and 5MBS, the VAL server has to create two User services if the network provide the service layer interworking with MBSF and MBSTF as indicated in clause 4.9 of 3GPP TS 26.502 [6], or it has to create the eMBMS bearer towards the BM-SC and the MBS session towards the MB-SMF (optionally via the NEF) individually if the network does not provide the service layer interworking which requires the application server themselves to handle the interworking. From the application layer, such two User Services or individual eMBMS bearer and MBS session increases the difficulty of using 3GPP broadcast and multicast.

-	Application has to select the 5MBS session type. The VAL server needs to select the multicast MBS session and the broadcast MBS session. It also requires the VAL service provider to have enough 3GPP MBS background and knowledge to select a proper type.

The NRM defined in 3GPP TS 23.434 [4] supports the multicast/broadcast resource control via several different operations of the SS_NetworkResourceAdaptation API.

Table 5.1.1-1: NRM API supporting multicast/broadcast

| Operation Name | Applicable for 5G MBS or 4G eMBMS, or both |
| --- | --- |
| Request_Multicast_Resource | 4G eMBMS only |
| Request_Multicast/Broadcast_Resource Update_Multicast/Broadcast_Resource Delete_Multicast/Broadcast_Resource Activate_Multicast_Resource Deactivate_Multicast_Resource | 5G MBS only |

Even with the NRM SS_NetworkResourceAdaptation API, the VAL server has to handle the 5G MBS or 4G eMBMS individually.

In addition, during the MBS session update as described in clause 14.3.4A.3 of 3GPP TS 23.434 [4], new MBS session(s) may be created per each grouped MBS service area, and the NRM returns additional user plane IP address(es) and port for VAL to deliver the DL packet. In this case, the VAL sever needs to maintain several downlink delivery endpoints towards the 3GPP system for the same content. The application layer is burdened with mentioned issues.

Such complexities mentioned above should be hidden from the VAL service provider to promote the usage of 3GPP multicast/broadcast. SEALDD is designed to provide simple data delivery service for the application data. So how to provide a multicast/broadcast data delivery service which hides the complexities e.g., the configuration options, the different 4G eMBMS management (e.g., eMBMS bearer activation) and 5G MBS session management (e.g., 5MB session creation) to the VAL service provider is crucial to the success of 3GPP multicast/broadcast transmission.

### 5.1.2	Open issues

1.	How to design a multicast/broadcast data delivery service which is able to hide the complexities for the VAL server over SEALDD-S (e.g., the configuration options, the different 4G eMBMS management (e.g., eMBMS bearer activation) and 5G MBS session management (e.g., 5MB session creation) session type selection, multiple downlink delivery endpoints towards the 3GPP system for the same content to the VAL service provider)?

## 5.2	Key issue #2: SEALDD-S and SEALDD-UU user plane

### 5.2.1	Description

SEALDD services require both control signalling messages and user plane to support the delivery of application traffic. Currently, in 3GPP TS 23.433 [2] only the control signalling messages over SEALDD-UU and SEALDD-S are specified for SEALDD services. However, SEALDD user plane (i.e., SEALDD-UU, SEALDD-S) aspects are not fully and well specified. For example, the user plane protocol stack(s) is missing.

In addition, the user plane functionalities description is missing even though several user plane functionalities are already described in the specification, including but not limited to:

1)	The SEALDD flow ID is allocated to uniquely identify the application traffic.

2)	The SEALDD enabled data transmission quality measurement in clause 9.7.2 of 3GPP TS 23.433 [2] requires the DL monitoring packet or the dummy DL SEALDD packet.

3)	SEALDD enabled E2E redundant transmission provides the replication and deduplication between the two SEALDD flows carrying application traffic.

In addition, different SEALDD services may use different SEALDD user plane protocols and functionalities to convey the application traffic, e.g., the user plane functionalities used by SEALDD E2E redundant transmission service could be different from the one used by SEALDD regular connection management service.

The overall SEALDD user plane protocol stack information is not specified in 3GPP TS 23.433 [2], which can further assist stage 3 groups to make choice of suitable user plane protocols for supporting SEALDD services.

### 5.2.2	Open issues

1.	The user plane aspects (e.g., user plane protocol stack, user plane functionality description) of SEALDD-S and SEALDD-UU to be used for the SEAL data delivery services should be studied.

## 5.3	Key Issue #3: Application traffic pattern and SEALDD data delivery optimization

### 5.3.1	Description

The application traffic is dynamic and changes over time. For example, the application traffic can be AI-based applications like GenAI apps, live streaming etc. Moreover different application traffic pattern and interaction requires different QoS for a better QoE to the users.

The SEALDD-UU carries the VAL application traffic. The SEALDD server manages the SEALDD-UU (e.g. establishing a redundant connection) and delivers the application traffic over the SEALDD-UU. Currently, the SEALDD server cannot understand the traffic pattern of the VAL app over the SEALDD-UU. It does not know how the VAL app traffic behaves at different times.

Currently, the SEALDD server connection parameters is not based on the different traffic patterns and this may lead to suboptimal configuration of the connection parameters. There are no mechanisms in the SEALDD server to improve the data delivery service with different traffic pattern awareness. Therefore, it becomes necessary for the SEALDD layer to optimize the SEALDD-UU transmission based on different traffic patterns to handle the dynamicity and interaction of the application traffic.

### 5.3.2	Open Issues

1.	How SEALDD server know different SEALDD-UU application traffic patterns (e.g., characterization, differentiation of dynamic application interaction in the SEALDD layer).

2.	How to enable the SEALDD layer (server, client) data transmission to optimize (e.g., QoS provisioning and achieve QoS guarantee) for the different SEALDD-UU traffic patterns.

## 5.x	Key issue #x: <Title>

Editor's Note:	This clause describes the key issue.

Editor's Note:	Please provide a suitable title for the key issue.

# 6	Solutions

## 6.1	Mapping of solutions to key issues

Table 6.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  | X |  |  |
| Sol #2 | X |  |  |  |
| Sol #3 | X |  |  |  |
| Sol #4 | X |  |  |  |

## 6.1	Solution #1:	SEALDD-S and SEALDD-UU user plane functionality description and protocol stack

### 6.1.1	Solution description

#### 6.1.1.1	General

Currently, SEALDD layer provide several data delivery services towards the VAL layer with different user plane characteristics as analysed in table 6.1.1.1-1.

Table 6.1.1.1-1: SEALDD delivery service user plane analysis

| Services of SEALDD service | User plane characteristic of SEALDD-S | User plane characteristic of SEALDD-UU |
| --- | --- | --- |
| Regular Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID |
| URLLC Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL), and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow ID shared by the two paths. |
| Data Storage | Put DL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane | Put UL data to the SEALDD server (storage server) - Push mode in a standalone user plane Fetch data from the SEALDD server - Fetch mode in a standalone user plane |
| BAT data transfer | Refers to the Data storage | Refers to the Data storage |
| XR Transmission | Address: VAL server endpoint address (e.g., IP address and/or port, URL) and SEALDD server endpoint address (e.g., IP address and/or port, URL). | Address: SEALDD server endpoint address (e.g., IP address and/or port, URL) and SEALDD client endpoint address (e.g., UE's addresses/ports). Identifier: SEALDD-UU Flow IDs and multi-modal flow ID. |
| Transmission Quality Measurement |  | SEALDD UU DL monitoring header, or dummy DL SEALDD packet generated by SEALDD server time stamp: local time T1, T2, T3, T4 |

Editor's note:	This table may be not complete and any further updates is FFS.

#### 6.1.1.2	Protocol stack for SEALDD-UU

##### 6.1.1.2.1	User plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-UU.

Figure 6.1.1.2.1-1: SEALDD-UU user plane stack

SEALDD-UU user plane:	This user plane of the SEALDD-UU layer corresponds to the protocol data unit carried between the SEALDD client and the SEALDD server over the SEALDD-UU flow. The protocol(s) used for this SEALDD-UU may varies due to different SEALDD data delivery service.

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the user plane protocol stacks as described in 3GPP TS 23.501 [23.501].

The user plane of SEALDD-UU layer shall support the functions/characteristics listed in table 6.1.1.1.

##### 6.1.1.2.2	Control plane protocol stack for SEALDD-UU

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-UU.

Figure 6.1.1.2.2-1: SEALDD-UU control plane stack

SEALDD-UU control plane: The control plane of the SEALDD-UU layer corresponds to the protocol data unit delivered between the SEALDD client and the SEALDD server. This protocol data unit is used for managing SEALDD-UU user plane (e.g., regular transmission connection establishment, transmission quality measurement, etc.), as per the procedures defined in 3GPP TS 23.433 [2].

N6 protocol stack:	This set of protocols/layers depends on the UPF.

NOTE 1:	For simplicity, only a box named intermediate protocol stacks are shown. For details, it refers the control plane protocol stacks as described in 3GPP TS 23.501 [23.501].

#### 6.1.1.3	Protocol stack for SEALDD-S

##### 6.1.1.3.1	User plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the user plane data delivery related with SEALDD-S.

Figure 6.1.1.3.1-1: SEALDD-S user plane stack

SEALDD-S user plane:	This user plane of SEALDD-S layer corresponds to the protocol data unit carried between the VAL server and the SEALDD server over the SEALDD-S flow. The protocol(s) used for this SEALDD-S user plane may varies due to different SEALDD data delivery service.

The SEALDD-S-U layer shall support the functions/characteristics listed in table 6.1.1.1.

##### 6.1.1.3.2	Control plane protocol stack for SEALDD-S

This clause illustrates the protocol stack for the control plane data delivery related with SEALDD-S.

Figure 6.1.1.3.2-1: SEALDD-S control plane stack

SEALDD-S control plane: The control plane of the SEALDD-S layer corresponds to the protocol data unit delivered between the VAL server and the SEALDD server. This protocol data unit is used for managing SEALDD-S user plane data transmission, etc., as per the procedures defined in 3GPP TS 23.433 [2].

#### 6.1.1.4	User plane protocol stack examples for SEALDD-UU and SEALDD-S

##### 6.1.1.4.1	General

This clause illustrates the recommended protocol(s) for different SEALDD data delivery service related to SEALDD-UU and SEALDD-S user plane.

NOTE:	This clause could serve as the basis for the informative Annex of the TS.

##### 6.1.1.4.2	Recommended protocol(s) for Regular Transmission

Regular transmission supports the data delivery of live streaming. The VAL server (content provider) sends live streaming data to the SEALDD server via RTMP/SRT protocol. The SEALDD server handles all Stream Processing, such as converting the RTMP stream into protocols compatible with various playback devices – including RTMP, HTTP-FLV, or HLS. The SEALDD server may then deliver the data to the VAL UE (player) where the SEALDD client resides, using RTMP, HTTP-FLV, or HLS protocols.

Figure 6.1.1.4.2-1: The recommend protocol(s) for live streaming delivery in SEALDD-UU and SEALDD-S user plane stack

Regular transmission supports the VOD. The VAL server (content provider) could send VOD data to the SEALDD server via HTTP(S) “pull” mode. The SEALDD server handles all Stream Processing, such as converting the HTTP(S) stream into protocols compatible with various playback devices – including HLS, HTTP-FLV, or DASH. The SEALDD server may then deliver the data to the VAL UE (player) where the SEALDD client resides, using HLS, HTTP-FLV, or DASH protocols.

Figure 6.1.1.4.2-2: The recommended protocol(s) for VOD in SEALDD-UU 
and SEALDD-S user plane stack

##### 6.1.1.4.3	Recommended protocol(s) for Data Storage

In the data storage service creation procedure, the VAL server/SEALDD client (content provider) transmits data to be stored to the SEALDD server via HTTP/HTTPS protocol. After authorized verification, the SEALDD server configures the storage service for the request and stores data on the storage server. The SEALDD server then sends a response to the SEALDD client/VAL server via the HTTP/HTTPS protocol.

Figure 6.1.1.4.3-1: The recommended protocol(s) for Data Storage in 
SEALDD-UU and SEALDD-S user plane stack

##### 6.1.1.4.4	Recommended protocol(s) for XR Transmission

XR Transmission supports the data delivery of real-time streaming media. The VAL server (content provider) transmits real-time media data to the SEALDD server via WebRTC protocol. The SEALDD server manages all Stream Processing, including converting the WebRTC data into playback-compatible protocols such as RTMP/HLS/FLV. Subsequently, the SEALDD server delivers processed data to the VAL UE (player) hosting the SEALDD client using the RTMP/HLS/FLV protocol.

Figure 6.1.1.4.4-1: The recommended protocol(s) for XR Transmission 
in SEALDD-UU and SEALDD-S user plane stack

##### 6.1.1.4.5	Recommended protocol(s) for Transmission Quality Measurement

Transmission Quality Measurement supports monitoring of transmission quality in an on-going regular data transmission connection. The monitoring information can be carried in the protocol extension header or in the protocol payload. Accordingly, the recommended Transmission Quality Measurement protocol depends on the protocol employed by the monitored data transmission connection.

### 6.1.2	Architecture Impacts

This solution has no architecture impacts.

### 6.1.3	Corresponding APIs

This solution has no new API.

### 6.1.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 6.2	Solution #2: SEALDD enabled multicast data delivery

### 6.2.1	Solution description

#### 6.2.1.1	General

In this solution, the SEALDD server provides new data delivery services to deliver the same content (e.g., application signalling or application user plane data) from a single source entity to a group of pre-defined recipients. The SEALDD server provides multicast delivery services:

- SEALDD multicast data delivery service. It is used for the application that provides the same content to a group of pre-defined recipient. It requires the support of the multicast 5MBS session feature at the network and UE. Compared with broadcast data delivery service, it provides user level QoS guarantee.

In general, the multicast delivery services require the VAL server to provide its service requirements e.g., service area, QoS requirements. The SEALDD server provides its own address as the ingress address for the VAL server to send the DL data. During the data delivery service update with service area update, more MBS session may be created and additional MB-UPF addresses are returned to the SEALDD server. However, SEALDD server hides the multiple network user plane addresses towards the VAL server and the ingress address keep the same.

In this solution, the SEALDD server interacts with the 3GPP network directly or indirectly via the NRM server to manage the 5G multicast MBS session. Also the NRM-UU is used for the session/service announcement, listening status report, UE session join notification.

Figure 6.2.1.1 -1 shows the high-level functional model for this service.

Figure 6.2.1.1-1:Multicast delivery service functional model

Editor’s note:	Whether to have a single data delivery service to cover both the broadcast data delivery and multicast data delivery is FFS.

#### 6.2.1.2	Multicast data delivery service

##### 6.2.1.2.1	Multicast data delivery service create

Figure 6.2.1.2.1-1 illustrates the high-level procedure of creating a multicast data delivery service for the VAL server DL data delivery.

Figure 6.2.1.2.1-1: Multicast delivery service create

1.	The VAL server invokes the Sdd_MulticastDelivery create service. The service requirements e.g., service area, activation or deactivation, QoS requirements are included. Optionally, the endpoint of a VAL server which ingresses the DL data is provided.

NOTE 1:	The VAL server ingresses the DL data to the SEALDD server can be a different VAL server.

2.	The SEALDD server determines to create a multicast flow. The SEALDD server allocates the multicast flow ID, the source multicast address and port used for DL data delivery to the recipients.

3.	The SEALDD server interacts with the 5GS directly to create the multicast MBS session as described in 3GPP TS 23.247 [3]. Alternatively, the SEALDD server may interact with NRM server to create the multicast MBS session.

NOTE 2:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [4] or can be implementation specific.

4.	The SEALDD server allocates an ingress address for the VAL server to send its DL data. The SEALDD also allocates a data delivery service ID which is used to update and delete the created data delivery service. The SEALDD server returns the Sdd_MulticastDelivery create response with the allocated ingress address to the VAL server.

5.	The VAL server provision the service information including the SEALDD server address, data delivery service ID to the VAL client.

6.	The VAL client sends the service request to the SEALDD client. The SEALDD server address, data delivery service ID are included.

7.	Similar as clause 9.2.2.1 of 3GPP TS 23.433 [2]. The SEALDD client sends the connection request (data delivery service ID) to the SEALDD server.

8.	Similar as clause 9.2.2.1 of 3GPP TS 23.433 [2]. The SEALDD server determines and returns the media related information including multicast flow ID for the DL data delivery based on the data delivery service ID.

9.	The NRM server may send the MBS service announcement with the multicast MBS session ID to the NRM client if the MBS session ID is not included in step 8 via the NRM-UU as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [4].

10.	The UE to join the multicast MBS session identified by the MBS session ID to receive the DL data as described in 3GPP TS 23.247 [3].

11.	After successfully MBS join, the NRM client sends the UE session join notification to the NRM server via the NRM-UU as described in 3GPP TS 23.434 [4].

12.	The NRM client also sends the MBS listening status to the NRM server. This message may be sent multiple times due to the MBS reception quality changes via the NRM-UU as described in 3GPP TS 23.434 [4].

13.	A VAL server starts to send DL data to the allocated ingress address. The SEALDD server encapsulates the received DL data within the multicast flow packet with the source multicast address and further delivers the encapsulated packet(s) via the established multicast MBS session.

NOTE 3:	The VAL server can start to send data to the SEALDD server after step 5.

##### 6.2.1.2.2	Multicast data delivery service update

Figure 6.2.1.2.2-1 illustrates the high-level procedure of updating a multicast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a multicast delivery service as described in clause 6.2.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.2.1.2.2-1: Multicast delivery service update

1.	The VAL server invokes the Sdd_Multicast data delivery update service. The data delivery service ID and updated parameters, e.g., service area, session status (activation or deactivation) or QoS requirements are included. For example, the VAL server temporarily has no DL data to transmit, it may deactivate the multicast MBS session.

2.	The SEALDD server performs the multicast MBS session update accordingly as described in 3GPP TS 23.247 [3]. If the service areas are updated and new multicast MBS session(s) is created, addition MB-UPF ingress address(es) may be also returned. The SEALDD server associates it with the allocated ingress address towards the VAL server. Alternatively, the SEALDD server may interact with NRM server to update multicast MBS session. Updated information may be sent to the NRM client by the NRM server.

NOTE:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [4] or can be implementation specific.

3.	The SEALDD server returns the Sdd_MulticastDelivery update response indicating the success of update.

4.	The VAL server continues to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the updated multicast MBS session(s).

##### 6.2.1.2.3	Multicast data delivery service delete

Figure 6.2.1.2.3-1 illustrates the high-level procedure of deleting a multicast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a multicast delivery service as described in clause 6.2.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.2.1.2.3-1: Multicast delivery service delete

1.	The VAL server no longer requires the multicast delivery service. It invokes the Sdd_Multicast data delivery delete service. The data delivery service ID is included.

2.	The SEALDD server performs the multicast MBS session delete accordingly as described in 3GPP TS 23.247 [3]. Alternatively, the SEALDD server may interact with NRM server to delete the multidcast MBS session. Disannouncement is sent to NRM client from the NRM server.

NOTE:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [4] or can be implementation specific.

3.	The SEALDD server returns the Sdd_MulticastDelivery delete response indicating the success of update.

4.	The UE leaves the multicast MBS session accordingly based on the dis-announcement.

### 6.2.2	Architecture Impacts

There are no architecture impacts.

### 6.2.3	Corresponding services

This solution introduces a new SEALDD data delivery service.

### 6.2.4	Solution evaluation

This solution provides a new data delivery service, i.e., multicast data delivery service. It enable the VAL server to only send single copy data to the SEALDD server, and the SEALDD server will deliver it to multiple recipients. During the service area update, the VAL server is not bothered by creating new MBS sessions with additional ingress addresses.

## 6.3	Solution #3: SEALDD enabled broadcast data delivery

### 6.3.1	Solution description

#### 6.3.1.1	General

In this solution, the SEALDD server provides new data delivery services to deliver the same content (e.g., application signalling or application user plane data) from a single source entity to all the recipients in a certain area. The SEALDD server provides:

SEALDD broadcast data delivery service. It is used for the application that provides the same content to all the recipients in a certain area. It requires the support of the broadcast 5MBS session and/or eMBMS at the network and UE.

In general, the broadcast delivery services require the VAL server to provide its service requirements e.g., service area, start and end time of the delivery, QoS requirements. The SEALDD server provides its own address as the ingress address for the VAL server to send the DL data. During the data delivery service update with service area update, more MBS session may be created and additional MB-UPF addresses are returned to the SEALDD server. However, SEALDD server hides the multiple network user plane addresses towards the VAL server and the ingress address keep the same. For broadcast data delivery service, the SEALDD server determines to create both eMBMS bearer and MBS session based on the service area.

In this solution, the SEALDD server interacts with the 3GPP network directly or indirectly via the NRM server to manage the eMBMS bearer and 5MBS broadcast session. Also the NRM-UU is used for the session/service announcement, listening status report, UE session join notification.

Figure 6.3.1.1 -1 shows the high-level functional model for this service.

Figure 6.3.1.1-1: Broadcast delivery service functional model

#### 6.3.1.2	Broadcast data delivery service

##### 6.3.1.2.1	Broadcast data delivery service create

Figure 6.3.1.2.1-1 illustrates the high-level procedure of creating a broadcast data delivery service for the VAL server DL data delivery.

Pre-conditions:

1.	The VAL client and VAL server already established the application layer connection.

Figure 6.3.1.2.1-1: Broadcast delivery service create

The VAL server invokes the Sdd_BroadcastDelivery create service to distribute its DL content to all its recipients within the service area. The data delivery requirements e.g., service area, start and end time of the delivery, QoS requirements are included. The endpoint of a VAL server which ingresses the DL data may be also provided.

NOTE 1:	This VAL server ingresses the DL data can be a different VAL server than the one in step 1.

2.	The SEALDD server determines to create a broadcast flow. The SEALDD server allocates the broadcast flow ID, the source multicast address and port used for DL data delivery to the recipients.

3.	The SEALDD server may requests the eMBMS bearer as described in 3GPP TS 23.468 [5] and 3GPP TS 23.247 [3], and/or create the broadcast MBS session as described in 3GPP TS 23.247 [3] for this data delivery service and obtains the TMGI(s) from 3GPP network. Alternatively, the SEALDD server may interact with NRM server to request the eMBMS bearer or broadcast MBS session.

NOTE 2:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [4] or can be implementation specific.

4.	The SEALDD server allocates an ingress address for the VAL server to send its DL data. The SEALDD also allocates a data delivery service ID which is used to update and delete the created data delivery service. The SEALDD server returns the Sdd_BroadcastDelivery create response with the allocated ingress address to the VAL server.

5.	The VAL server provision the service information including the SEALDD server address, data delivery service ID to the VAL client.

Editor’s Note: The need for VAL service provisioning for Broadcast service is FFS.

6.	The VAL client sends the service request to the SEALDD client. The SEALDD server address, data delivery service ID are included.

7.	The SEALDD client sends the connection request (data delivery service ID) to the SEALDD server.

8.	The SEALDD server determines and returns broadcast flow ID used for the DL data delivery based on the data delivery service ID.

9.	The NRM server sends the MBS service announcement with TMGI of the broadcast MBS session to the NRM client via the NRM-UU as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [4].

10.	The UE starts to lisening the broadcast channel as inticated by the TMGI to receive the DL data as described in 3GPP TS 23.247 [3].

11.	The NRM client sends the MBS listening status report to the NRM server via the NRM-UU as as described in step 3 clause 14.3.4A.2.2 in 3GPP TS 23.434 [4]. This message may be sent multiple time due the MBS reception quality changes.

12.	The VAL server starts to send DL data to the allocated ingress address. The SEALDD server encapsulates the received DL data within the broadcast flow packet with the source multicast address and further delivers the encapsulated packet(s) via the established eMBMS bearer and/or the broadcast MBS session.

NOTE 3:	The VAL server can start to send DL data to the SEALDD server after step 5. And the SEALDD server can start to send DL data to the network even before step 7. Then the UE can immediately receive the data after step 9.

##### 6.3.1.2.2	Broadcast data delivery service update

Figure 6.3.1.2.2-1 illustrates the high-level procedure of updating a broadcast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast delivery service and obtained the its data delivery service ID as described in clause 6.3.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.3.1.2.2-1: Broadcast delivery service update

1.	The VAL server invokes the Sdd_broadcast data delivery update service. The data delivery service ID and updated parameters, e.g., service area or QoS requirements are included.

2.	The SEALDD server requests the eMBMS bearer update and/or the broadcast MBS session update directly as described in 3GPP TS 23.468 [5] and 3GPP TS 23.247 [3]. If the service areas are updated and new broadcast MBS session(s) is created, addition MB-UPF ingress address(es) may be also returned. The SEALDD server associates it with the allocated ingress address towards the VAL server. Alternatively, the SEALDD server may interact with NRM server to update the eMBMS bearer or broadcast MBS session. Updated information may be sent to the NRM client by the NRM server.

NOTE:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [4] or can be implementation specific.

3.	The SEALDD server returns the Sdd_BroadcastDelivery update response indicating the success of update.

4.	The VAL server continues to send DL data to the allocated ingress address. And the SEALDD further delivers the DL via the updated eMBMS bearer and/or the broadcast MBS session.

##### 6.3.1.2.3	Broadcast data delivery service delete

Figure 6.3.1.2.3-1 illustrates the high-level procedure of deleting a broadcast data delivery service for the VAL server DL data delivery.

Pre-condition:

1.	The VAL server has created a broadcast delivery service and obtained the its data delivery service ID as described in clause 6.3.1.2.1. And there is ongoing DL data is being delivered via this service.

Figure 6.3.1.2.3-1: Broadcast delivery service delete

1.	The VAL server no longer requires the broadcast delivery service. It invokes the Sdd_broadcast data delivery delete service. The data delivery service ID is included.

2.	The SEALDD server performs the eMBMS bearer delete and/or the broadcast MBS session release accordingly and interacts with the SEALDD client as described in 3GPP TS 23.468 [5] and 3GPP TS 23.247 [3]. Alternatively, the SEALDD server may interact with NRM server to delete the eMBMS bearer or broadcast MBS session. Disannouncement is sent to NRM client from the NRM server.

NOTE:	The interactions between the SEALDD server and the NRM can via the NRM-S as described in 3GPP TS 23.434 [4] or can be implementation specific.

3.	The SEALDD server returns the Sdd_BroadcastDelivery delete response indicating the success of deletion.

### 6.3.2	Architecture Impacts

There are no architecture impacts.

### 6.3.3	Corresponding services

This solution introduces a new SEALDD data delivery service.

### 6.3.4	Solution evaluation

This solution provides a new data delivery service, i.e., broadcast data delivery service, where the VAL server only sends one copy of data to the SEALDD server, and the SEALDD server send it to all the recipients of this VAL service within the service area. During the service area update, the VAL server is not bothered by creating new MBS sessions with additional ingress addresses.

## 6.4	Solution #4: SEALDD enabled Multicast/Broadcast data delivery service

### 6.4.1	Solution description

The solution enables the VAL server to use SEALDD server to deliver Multicast and Broadcast data to VAL clients.

#### 6.4.1.1	General

The SEALDD server provides the Multicast and Broadcast service (MBS) data delivery service to the VAL server using a 5G System to send a single copy of the data to several clients. The SEALDD server abstracts the underlying complexity of managing the MBS sessions and MBS data delivery and provides an exposure to the VAL server for MBS data delivery. The VAL server can request the MBS data delivery services by providing the media service requirements like QoS, location etc. to the SEALDD server. The SEALDD server uses the SEAL NRM services to create and manage the MBS sessions and use the SEALDD user plane to perform the data delivery.

The essence of the solution utilizes the NRM server for resource management for the MBS session management (like join notification, MBS session announcement messages etc), whereas SEALDD server is the entity to deliver the multicast and broadcast media over SEALDD-UU.

Editor's NOTE: The support for SEALDD enabled broadcast media delivery over eMBMS is FFS.

Figure 6.4.1.1 -1 shows the high-level functional model for this service.

Figure 6.4.1.1-1: SEALDD enabled Multicast and Broadcast data delivery service

#### 6.4.1.2	SEALDD enabled Multicast and Broadcast data delivery service

##### 6.4.1.2.1	SEALDD enabled Multicast and Broadcast data delivery create procedure

Figure 6.4.1.2.1-1 illustrates the high-level procedure of creating a SEALDD enabled Multicast and Broadcast data delivery service for the VAL server.

Pre-conditions:

1.	The VAL server and VAL client has established the application layer connection and provisioned the VAL client with application layer information for the MBS application or services.

Figure 6.4.1.2.1-1: SEALDD enabled Multicast and Broadcast data delivery create procedure

1.	The VAL server sends the SEALDD MulticastBroadcast service create request to send downlink data to the VAL UEs, VAL group ID. The request includes VAL server ID, VAL service ID, list of VAL UEs, delivery mode (i.e., multicast or broadcast), service area, QoS requirements, and optionally start and end time of the delivery

2.	The SEALDD server allocates the SSM address, port, SEALDD client MBS data delivery ID and invokes the NRM server to send a NRM MulticastBroadcast service create request including the requirements defined in the previous step 1.

3.	The NRM interacts with the 3GPP system based on the procedures in step 2 of clause 14.3.4A.2.2 3GPP TS 23.434[4] to create an eMBMS or an MBS session, respectively. Upon successful creation of an MBS session, the NRM server receives a MBS session ID.

4.	Same as step 3 defined in clause 14.3.4A.2.2 TS 23.434[4].

5.	Same as step 4 defined in clause 14.3.4A.2.2 TS 23.434[4].

6.	Same as step 5 defined in clause 14.3.4A.2.2 TS 23.434[4].

7a-7b.	The VAL client sends a service request to the SEALDD client for the Multicast broadcast session service as per VAL service provisioning information. The SEALDD client communicates with the NRM client about the SSM address, MBS Session ID to receive multicast or broadcast packets. Based on the service request, the SEALDD client establishes the SEALDD-UU connection as per clause 9.2.2.2 SEALDD regular connection establishment. The SEALDD server may include the allocated SEALDD client MBS data delivery ID in the regular connection establishment response for the SEALDD client.

NOTE:	The interaction between the VAL client, SEALDD client and NRM client is implementation-specific and out of scope of 3GPP.

8a.	In case of broadcast delivery type, the NRM client listens to the MBS session ID for the broadcast data, same as step 6b defined in clause 14.3.4A.2.2 TS 23.434[4].

8b.	In case of multicast delivery type, the NRM client joins the multicast session with 5GSystem using the MBS session ID same as step 6a defined in clause 14.3.4A.2.2 TS 23.434[4].

8c.	Same as step 6a defined in clause 14.3.4A.2.2 TS 23.434[4] and clause 14.3.4A.2.2 TS 23.434[4].

9.	Same as step 8 defined in clause 14.3.4A.2.2 TS 23.434[4]

10.	The NRM server sends the MulticastBroadcast service create response to the SEALDD server with the received MBS session ID as per the clause 14.3.4A.2.2 TS 23.434[4]. On receiving the NRM response message, the SEALDD server allocates the endpoint information of the SEALDD server side to receive the data from the VAL server and SEALDD MBS Session ID to manage the MBS sessions.

11.	The SEALDD server sends the SEALDD MulticastBroadcast service create response to the VAL server with the allocated endpoint information to send data and SEALDD MBS session ID for managing the MBS sessions.

12.	The VAL server sends the DL data to the SEALDD server over the allocated ingress connection information. The SEALDD server delivers the DL data to the SEALDD clients over the SEALDD-UU session. The SEALDD clients forward them to the VAL clients. The SEALDD server may determine to switch to the unicast session to deliver the MBS data and back to MBS sessions.

##### 6.4.1.2.2	SEALDD enabled Multicast and Broadcast data delivery update procedure

Figure 6.4.1.2.2-1 illustrates the high-level procedure of updating a SEALDD enabled Multicast and Broadcast data delivery service for the VAL server.

Precondition:

-	A SEALDD server enabled multicast or broadcast session is created and media is delivered.

Figure 6.4.1.2.2-1: SEALDD enabled Multicast and Broadcast data delivery update procedure

1.	The VAL server sends a SEALDD MulticastBroadcast service update request to the SEALDD server. The request includes among others the parameters to be updated related to the MBS session under consideration, e.g., the service requirement or service area, list of VAL UEs, SEALDD MBS session ID.

2.	The SEALDD server sends an NRM Multicast resource update request towards the NRM server as per step 2 defined in clause 14.3.4A.3.2 TS 23.434 [4].

3.	The NRM server sends an NRM MulticastBroadcast service update request towards the 5GS (to the MB-SMF either directly or via the NEF) as per the step 3, step 4 and step 5 of clause 14.3.4A.3.2 TS 23.434 [4].

4.	Same as step step 7 of clause 14.3.4A.3.2 TS 23.434 [4].

5.	The NRM client stores and process the updated received information. If required, the NRM client interacts with the SEALDD client to configure as per the updated information.

NOTE:	It is implementation specific how the NRM client and SEALDD client communicate with each other for in the VAL UE and is out of scope.

6.	Same as step 5 defined in clause 14.3.4A.3.2 TS 23.434 [4].

7.	Same as step 7 defined in clause 14.3.4A.3.2 TS 23.434 [4]. For the updated MBS sessions (like new MBS sessions, update of service areas, etc.), the SEALDD server associates it with the allocated ingress address towards the VAL server.

8.	The SEALDD server sends a SEALDD MulticastBroadcast service update response to the VAL server indicating the success of the update with the updated parameters of the MBS session.

9.	The VAL server continues sending data delivery using the allocated address and SEALDD server delivers the data over the updated MBS session to the users.

##### 6.4.1.2.3	SEALDD enabled Multicast and Broadcast data delivery deletion procedure

Figure 6.4.1.2.3-1 illustrates the high-level procedure of deletion a SEALDD enabled Multicast and Broadcast data delivery service for the VAL server.

Precondition:

-	A SEALDD server enabled multicast or broadcast session upon the request of the VAL server, and media is delivered.

-	There is no ongoing VAL group communication over the MBS session.

Figure 6.4.1.2.2-1: SEALDD enabled Multicast and Broadcast data delivery deletion procedure

1.	The VAL server decides to delete an MBS session if it is no longer needed.

2.	The VAL server sends the SEALDD MulticastBroadcast service delete request to delete the MBS sessions.

3.	The SEALDD server invokes NRM service as per step 2 defined in 14.3.4A.4.2 TS 23.434 [4].

4.	Same as step 3 defined in 14.3.4A.4.2 TS 23.434 [4].

5b.	In case of broadcast MBS session, the NRM client stops monitoring the MBS session under consideration as per step 4a defined in 14.3.4A.4.2 TS 23.434 [4].

5b.	In case of multicast delivery type, the NRM client performs UE session leave procedure as per step 4b 4a defined in 14.3.4A.4.2 TS 23.434 [4].

NOTE:	The communication between NRM client and SEALDD client related to session deletion is implementation specific and out of scope of 3GPP.

6.	Same as step 5 defined in 14.3.4A.4.2 TS 23.434 [4].

7.	Same as step 6 defined in 14.3.4A.4.2 TS 23.434 [4].

7.	Same as step 7 defined in 14.3.4A.4.2 TS 23.434 [4].

8.	The SEALDD server sends a SEALDD MulticastBroadcast service delete response to the VAL server indicating the result of the MBS deletion and MBS session ID.

### 6.4.2	Solution evaluation

The solution enables the VAL server to use the SEALDD layer as the user plane delivery for the broadcast/multicast data to send a single copy to several recipients. The SEALDD server hides the complexity of managing MBS sessions for the VAL server. In MBS session update, there is no change in the user plane data delivery between the SEALDD server and the VAL server. The SEALDD server handles the underlying updates in the MBS session without affecting the VAL server data delivery endpoint, which enables the VAL server to continue sending the data delivery using the allocated information. The SEALDD server utilizes the NRM server for Multicast broadcast resource management based on the existing procedures. This enables the vendor interoperability with flexible deployment scenarios like colocation of the NRM server and SEALDD server. Furthermore, the solution is aligned with the agreed concepts within SEAL specifications, i.e. the NRM server is responsible for resource management and the SEALDD server is responsible for data delivery.

## 6.x	Solution #x: <title>

Editor's Note:	Please provide a suitable title for the solution.

### 6.x.1	Solution description

Editor's Note:	This clause describes the solution. Each solution should clearly describe which of the key issues it covers and how.

### 6.x.2	Architecture Impacts

Editor's note:	This clause describes the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 6.x.3	Corresponding APIs

Editor's note:	This clause describes the corresponding APIs for supporting the solution.

### 6.x.4	Solution evaluation

Editor's note:	This clause describes an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# 7	Overall evaluation

Editor's Note:	This clause describes evaluation of different solutions.

# 8	Conclusions

## 8.1	General conclusions

Editor's note:	This clause describes general conclusions for the study.

## 8.2	Conclusions of key issue #x

Editor's Note:	This clause describes conclusions for the specific key issue.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 | S6-251383 |  |  |  | Skeleton | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implemented the following pCRs, S6-251384, S6-251386, S6-251467, S6-251483 | 0.1.0 |
| 2025-06 | SA6#67 |  |  |  |  | Implemented the following pCR, S6-252407 | 0.2.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implemented the following pCRs, S6-253622, S6-253623 | 0.3.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implemented the following pCRs, S6-254724, S6-254746, S6-254795 | 0.4.0 |