| 3GPP TR 23.700-83 V0.3.0 (2025-08) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on application layer support for AI/ML services Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	10

Introduction	11

1	Scope	12

2	References	12

3	Definitions of terms, symbols and abbreviations	12

3.1	Terms	12

3.2	Symbols	13

3.3	Abbreviations	13

4	Architectural Assumptions and Principles	13

4.1	Analysis of relation between AIMLE and ADAE services	13

4.2	Architectural Assumptions	13

5	Key issues	14

5.1	Key issue #1: Key issue on support for ML model inference	14

5.1.1	Description	14

5.1.2	Open Issues	14

5.2	Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer	14

5.2.1	Description	14

5.2.2	Open Issues	15

5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios	15

5.3.1	Description	15

5.3.2	Open Issues	16

5.4	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers	16

5.4.1	Description	16

5.4.2	Open Issues	17

5.5	Key issue #5: Support for AI Inference Exposure	17

5.5.1	Description	17

5.5.2	Open Issues	17

5.6	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios	17

5.6.1	Description	17

5.6.2	Open Issues	18

5.7	Key Issue #7: VFL sample alignment across VAL servers	18

5.7.1	Description	18

5.7.2	Open Issues	18

5.8	Study on Support for AI Inference service performance management and feedback	19

5.8.1	Description	19

5.8.2	Open Issues	19

6	Enhanced architecture for enabling AI/ML services	20

6.1	General	20

6.2	Enhanced AI/ML enablement functional model	20

6.2.2	AI/ML enablement functional model for roaming scenarios	21

6.2.3	AI/ML enablement functional model for federation	22

6.3	Functional Elements	22

6.4	Reference Points	22

7	Solutions	22

7.0	Mapping of solutions to key issues	22

7.1	Solution #1: Enhancement of AIMLE to support model inference	23

7.1.1	Solution description	23

7.1.2	Architecture Impacts	23

7.1.3	Corresponding APIs	23

7.1.4	Solution evaluation	24

7.2	Solution #2: Enhance AIMLE Data Management Assistance for Data Drift Detection	24

7.2.1	Solution Description	24

7.2.1.1	Impact to existing AIMLE procedures	25

7.2.2	Architecture Impacts	29

7.2.3	Corresponding APIs	29

7.2.4	Solution evaluation	29

7.3	Solution #3: New AIMLE Service on ML Model Maintenance	29

7.3.1	Solution Description	29

7.3.1.1	Procedure	30

7.3.1.2	Information flows	31

7.3.1.2.1	ML model maintenance subscription request	31

7.3.1.2.2	ML model maintenance subscription response	31

7.3.1.2.3	ML model maintenance notification	32

7.3.2	Architecture Impacts	32

7.3.3	Corresponding APIs	32

7.3.4	Solution evaluation	32

7.4	Solution #4: Management of evaluation information for ML model performance and correctness evaluation.	32

7.4.1	Solution description	32

7.4.1.1	Introduction	32

7.4.1.2	ML model evaluation information storage	33

7.4.1.3	ML model evaluation information retrieval	33

7.4.1.4	ML model evaluation information removal	34

7.4.2	Architecture Impacts	35

7.4.3	Corresponding APIs	35

7.4.3.1	ML Evaluation Profile	35

7.4.3.2	ML Evaluation Information Storage	36

7.4.3.3	ML Evaluation Information Retrieval	36

7.4.3.4	ML Evaluation Information Removal	37

7.4.4	Solution evaluation	37

7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness	38

7.5.1	Solution description	38

7.5.2	Architecture Impacts	39

7.5.3	Corresponding APIs	39

7.5.4	Solution evaluation	39

7.6	Solution #6: Enhancements to performance monitoring	40

7.6.1	Solution description	40

7.6.1.1	Enhancements to ML model performance monitoring	40

8.22.2	Procedure	41

7.6.1.2	Enhancements to HFL training	42

8.12.2	Procedure	43

7.6.2	Architecture Impacts	44

7.6.3	Corresponding APIs	44

7.6.4	Solution evaluation	44

7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios	44

7.7.1	Solution description	44

7.7.2	Architecture Impacts	47

7.7.3	Corresponding APIs	47

7.7.4	Solution evaluation	47

7.8	Solution #8: Multi-Client Split AI/ML Operations Pipeline	47

7.8.1	Solution Description	47

7.8.1.1	General	47

7.8.1.2	Multi-client pipeline operation	48

7.8.1.3	Multi-client pipeline service continuity	49

7.8.1.4	Procedures to support Multi-client AI/ML split operation	50

7.8.1.4.1	Split operation pipeline join	50

7.8.1.4.2	Split operation notification	50

7.8.2	Architecture Impacts	51

7.8.3	Corresponding APIs	51

7.8.3.1	Split operation pipeline discovery request	51

7.8.3.2	Split operation pipeline discovery response	52

7.8.3.3	Split operation pipeline create request	52

7.8.3.4	Split operation pipeline join request	53

7.8.3.5	Split operation pipeline join response	53

7.8.4	Solution Evaluation	53

7.9	Solution #9: AIMLE server registration in a hierarchical AIMLE deployment	54

7.9.1	Solution description	54

7.9.1.1	AIMLE server registration	54

7.9.1.2	AIMLE server registration update	55

7.9.1.3	AIMLE server de-registration	56

7.9.2	Architecture impacts	56

7.9.3	Corresponding APIs	56

7.9.3.1	AIMLE server registration request	56

7.9.3.2	AIMLE server registration response	57

7.9.3.3	AIMLE server registration update request	57

7.9.3.4	AIMLE server registration update response	58

7.9.3.5	AIMLE server de-registration request	58

7.9.3.6	AIMLE server de-registration response	58

7.9.4	Solution evaluation	59

7.10	Solution #10: Support of AIMLE services for roaming UEs	59

7.10.1	Solution description	59

7.10.1.1	Introduction	59

7.10.1.2 	Impact to existing AIMLE client registration	59

7.10.2	Architecture Impacts	66

7.10.3	Corresponding APIs	67

7.10.4	Solution evaluation	67

7.11	Solution #11: Solution of FL member registration in multi-operator scenarios	67

7.11.1	Solution description	67

7.11.2	Architecture Impacts	68

7.11.3	Corresponding APIs	68

7.11.4	Solution evaluation	69

7.12	Solution #12: Support for ML model split learning using relays	69

7.12.1	Solution description	69

7.12.2	Architecture Impacts	71

7.12.3	Corresponding APIs	72

7.12.4	Solution evaluation	72

7.13	Solution #13: ML model training in a hierarchical AIMLE deployment	72

7.13.1	Solution description	72

7.13.2	Architecture impacts	73

7.13.3	Corresponding APIs	74

7.13.3.1	Edge ML model training subscription request	74

7.13.3.2	Edge ML model training subscription response	74

7.13.3.3	Edge ML model training notification	75

7.13.4	Solution evaluation	75

7.14	Solution #14: Enhance AIMLE Client Discovery	75

7.14.1	Solution Description	75

7.14.2	Architecture Impacts	77

7.14.3	Corresponding APIs	77

7.14.4	Solution evaluation	77

7.15	Solution #15: Sample alignment for VFL	77

7.15.1	Solution description	77

7.15.2	Architecture Impacts	81

7.15.3	Corresponding APIs	81

7.15.4	Solution evaluation	81

7.16	Solution #16: Solution on AIMLE service migration support in UE roaming scenarios	81

7.16.1	Solution description	81

7.16.2	Architecture Impacts	83

7.16.3	Corresponding APIs	83

7.16.4	Solution evaluation	83

7.17	Solution #17: Enhancement of AIMLE to support model inference	84

7.17.1	Solution description	84

7.17.2	Architecture Impacts	86

7.17.3	Corresponding APIs	86

7.17.4	Solution evaluation	86

7.18	Solution #18: Solution on support for federated AI services	86

7.18.1	Solution description	86

7.18.2	Architecture Impacts	88

7.18.3	Corresponding APIs	88

7.18.4	Solution evaluation	88

7.19 	Solution #19: Cross-PLMN/Domain AIMLE client discovery, selection, monitoring	88

7.19.1 	Solution Description	88

7.19.1.1 	General	88

7.19.2 	Architecture Impacts	90

7.19.3 	Corresponding APIs	91

7.19.4 	Solution Evaluation	91

7.20	Solution #20: ML Model Performance Evaluation with information collection from ML model consumer	91

7.20.1	Solution Description	91

7.20.1.1	Procedure	91

7.20.1.2	Information Flows	92

7.20.1.2.1 	ML model evaluation subscription request	92

7.20.1.2.2	 ML model evaluation subscription response	92

7.20.1.2.3	 ML model performance information collection request	93

7.20.1.2.4 	Information collection request	93

7.20.1.2.5 	Information collection response	93

7.20.1.2.6 	ML model performance information collection response	94

7.20.1.2.7 	ML model evaluation notification	94

7.20.2	Architecture Impacts	94

7.20.3	Corresponding APIs	94

7.20.4	Solution evaluation	95

7.21	Solution #21: Split operation node registration involving multiple AIMLE clients	95

7.21.1	Solution description	95

7.21.2	Architecture Impacts	100

7.21.3	Corresponding APIs	100

7.21.4	Solution evaluation	101

7.22 	Solution #22: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning	101

7.22.1 	Solution Description	101

7.22.1.1 	General	101

7.22.1.2 	Procedure	101

7.22.2 	Architecture Impacts	102

7.22.3	 Corresponding APIs	103

7.22.4	 Solution Evaluation	103

8	Deployment scenarios	103

8.1	General	103

8.x	Deployment model #x: <Title>	103

9	Business Relationships	103

10	Overall evaluation	103

11	Conclusions	104

11.1	General conclusions	104

11.2	Conclusions of key issue #x	104

Annex A (informative): Change history	105

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

As part of Rel-19, SA6 normatively specified the application layer architecture to enable AI/ML services which introduced as a new SEAL service, aka AIMLE, as well as a new ML repository, in 3GPP TS 23.482 [2]. Furthermore, as part of Rel-19 work, in 3GPP TS 23.436 [3] enhancements have been specified for supporting ML-enabled analytics.

In Rel-20, the AIMLE functional architecture requires enhancements to further improve and enhance functionality in 3GPP for improved support for AI/ML services. In this direction, some already defined concepts need further study related to application layer ML model performance monitoring and correctness evaluation, extensions to AIML split operation, enhancements in VFL support considering also VFL participants at the edge/cloud; and enhancements for supporting different deployments and federation/multi-operator scenarios.

This technical report identifies the key issues and corresponding application architecture enhancements and related solutions with recommendations for the normative work.

# 1	Scope

The present document is a technical report which identifies enhancements to the application enablement layer architecture, capabilities, and services to support AI/ML services at the application layer.

The aspects of the study include the continuation of the investigation of application enablement impacts, the application enablement layer architecture enhancements and solutions needed to provide assistance in AI/ML operations at the VAL layer as well as at the application enablement layer (e.g., SEAL ADAES, EDGEAPP).

The study takes into consideration the work done for AI/ML in 3GPP TS 23.436 [3] and 3GPP TS 23.482 [2] and can consider other AI/ML related works within 3GPP.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.482: "Functional architecture and information flows for AIML Enablement Service	".

[3]	3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Vertical Federated Learning (VFL): A federated learning technique in which multiple VFL participants jointly perform training and inference on a common sample space with different feature sets, without exchanging raw data, thereby preserving their local data privacy.

VFL process: A process of performing training or inference using the VFL technique.

VFL participant: An entity that takes part in a VFL process, either as a VFL server or a VFL client.

VFL server: A VFL participant that coordinates the VFL process by aggregating results from VFL clients and propagating updates to them during training.

VFL client: A VFL participant that performs local training and inference, sends results to the VFL server, and updates local parameters based on information received from the VFL server.

VFL sample alignment: A stage in a VFL process during which samples are aligned among VFL participants, ensuring that training and inference are performed consistently across participants on the same sample space.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Architectural Assumptions and Principles

## 4.1	Analysis of relation between AIMLE and ADAE services

In this clause an analysis of the relation of ADAE analytics and AIMLE potential service on ML model inference is discussed. Initially a definition is provided about the possible role of AIMLE vs ADAES in inference and analytics:

1.	Analytics as exposed by ADAE, does not necessarily rely on AI/ML models. The consumer simply requests analytics and receives insights—without needing to know or specify how those insights are derived. Thus, when requesting analytics a consumer does not provide any information in relation to the ML model even if such model is used as it is abstracted away from it.

2.	Inference, on the other hand, is fundamentally tied to the use of a specific ML model. The consumer must either provide a model, model information or define criteria for selecting a model. This makes the ML model an integral part of the inference service. While inference can be used to support analytics, it does not itself expose analytics as a service.

## 4.2	Architectural Assumptions

The following high-level assumptions and principles will be considered in this study for ADAES:

1.	ADAES is the entity providing analytics which can be either ML-enabled or not.

2.	If ADAES analytics is ML-enabled, the ML training capability is provided by AIMLE (Rel-19 AIMLAPP).

3.	If ADAES analytics is ML-enabled, the ML inference capability can be provided as part of the analytics functionality inside ADAES (internal to ADAES logic). ML model inference is one of methods utilized by ADAE for Data analytics.

The following high-level assumptions and principles will be considered in this study for AIMLE:

1.	AIMLE provides ML support capabilities as of Rel-19 for both VAL and other SEAL servers (Rel-19 AIMLAPP).

2.	AIMLE provides ML model training capability for both VAL task and ADAE analytics (Rel-19 AIMLAPP).

3.	AIMLE is not expected to provide ML model inference for tasks related to ADAE analytics.

4.	AIMLE can only be seen as a candidate entity for performing ML model inference for VAL/ASP automation tasks (needs to be discussed further within Rel-20 study).

The following high-level assumptions and principles will be considered in this study for ML repository:

1.	The ML repository is the unique repository / registry per SEAL provider for the application-layer ML models and the parameters required for the AIMLE operations (Rel-19 AIMLAPP).

2.	Different deployments and cardinalities of ML repository within the SEAL provider's domain will be studied as part of Rel-20 work.

# 5	Key issues

## 5.1	Key issue #1: Key issue on support for ML model inference

### 5.1.1	Description

ML model inference, according to 3GPP TS 28.105, is a process of running a set of input data through a trained ML model to produce set of output data, such as predictions.

ML model inference may be provided by an analytics entity (e.g. ADAES) or may be provided by VAL server (in case of VAL-provided analytics) or potentially by AIMLE if the scenario involves the VAL server to request from AIMLE to provide ML model inference functionality (e.g. using network data or edge platform data).

In Rel-19 AIML_App, there was some basic support for operations related to split ML model inference; however, the use of ML model inference for supporting ADAE-layer analytics as well as the AIMLE support for enabling ML model inference (as part of the ML model lifecycle operations) was not discussed.

In this key issue, further enhancements in both AIMLE and ADAES will be explored for supporting ML model inference in ADAES and AIMLE, considering also impacts on the interfaces among AIMLE and ADAE entities in distributed AIMLE / ADAES deployments.

### 5.1.2	Open Issues

The following open issues need to be studied:

1.	Identify deployment models for supporting ML model inference in enablement layer.

2.	Whether and how ADAES needs be enhanced or leverage AIMLE for supporting ML model inference for ADAE analytics.

3.	Whether and how AIMLE needs to be enhanced to enable ML model inference to address VAL request (e.g. for VAL automation task or analytics).

4.	Whether and how to enhance AIMLE to support model inference in edge scenarios.

5.	Whether and how to enhance AIMLE architecture and functions to support collaborative inference between AIMLE client and edge AIMLE server.

NOTE:	Solutions for the above key issue should not introduce any inference services exposed by AIMLE (e.g., producing prediction result to third party).

## 5.2	Key Issue #2: Support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer

### 5.2.1	Description

KI #2 focuses on providing enhanced means to evaluate and determine ML model performance and correctness in the AIML Enablement Layer.

The evaluation of performance and correctness of a ML model may depend on the ML model, and the means for evaluating performance and correctness of a ML model may vary depending on the ML model usage and purpose.

ML model performance and correctness evaluation may require evaluating different aspects of the ML model. For example, the performance and correctness of a ML model may be related to its robustness (e.g., capability for handling unseen or drifting underlying data) or may be associated with its stability (e.g., small changes in input not leading to drastic prediction change).

A procedure to monitor ML model performance degradation by VAL server has been defined in clause 8.22 of 3GPP TS 23.482 [2]. However, this procedure is limited to evaluating performance degradation at the AIMLE client for ML model training and HFL use cases and does not clearly define the information needed for the AIML enablement layer to perform performance and correctness evaluation.

### 5.2.2	Open Issues

This key issue will study the following AIML enablement layer aspects:

1.	How to define ML model performance and ML model correctness in the context of the AIML enablement layer.

2.	Whether and how to evaluate ML model performance and correctness considering application specific ML models.

3.	Whether and how to monitor (e.g., trigger, detect and report) ML model performance and correctness degradation for different ML-enabled operations (e.g., inferencing, training, split AIML operation, HFL, VFL, transfer learning) and considering data managements assistance.

4.	Whether and how to trigger corrective actions on ML model performance and correctness degradation detection (e.g., model maintenance).

## 5.3	Key Issue #3: Key issue on AIMLE Mobility Support for Cross-PLMN and Roaming Scenarios

### 5.3.1	Description

As part of Rel-19, AIMLE was introduced as an enabler for application-layer AI/ML services, supporting model training, distribution primarily within a single operator's network domain. However, as AI/ML services increasingly involve VAL UEs moving across network domains (e.g., cross-PLMN mobility, UE roaming, federation across operators), and AI/ML services becoming increasingly distributed and dynamic, the ability to ensure service continuity and mobility of AIMLE functions becomes a critical requirement.

This key issue aims to study how AIMLE services can support mobility, including roaming scenarios where UEs access AI/ML services in visited networks, and cross-PLMN environments where AI/ML operations involve entities from different operators. This will address both the continuity of AI/ML services and the federation across MNOs and CSPs/ECSPs.

Additionally, the roaming aspects of VAL UEs do not address how the AIMLE entities (AIMLE server and/or ML repository) are aware of the serving PLMN / network information of the UE associated with the AIMLE client. For example, when a UE that is associated with a AIMLE client, moves from PLMN A to PLMN B. Then, in such case, the AIMLE server and/or ML repository may not be aware of this movement of the UE associated with the VAL AIMLE client. Further, the AIMLE server and/or ML repository are not aware of the serving PLMN information of the UE associated with a AIMLE client in order to provide the required support in cross-PLMN roaming scenarios.

In particular, as ML model inference increasingly moves toward the network edge to meet ultra-low latency and bandwidth efficiency demands (e.g., in real-time video analytic, industrial automation), maintaining inference continuity across mobility events becomes critical. When VAL UEs move across network boundaries or service areas, edge AIMLE instances may need to be reselected, dynamically discovered, or seamlessly replaced to ensure uninterrupted AI/ML inference execution and delivery.

The current AIMLE specifications and ongoing studies focus on static deployments and assume the VAL UE remains within its home network. However, real-world deployments often require:

1.	VAL UEs to maintain access to AIMLE services when roaming into visited PLMNs or NPNs,

2.	Cross-border AI/ML service collaboration (e.g. in eV2X or multiplayer gaming scenarios),

3.	Edge inference or AI model access to continue seamlessly even as the UE moves between coverage areas.

Also, the current AIML specification and other aspects of the ongoing studies do not focus on various conditions (for e.g., availability of AIMLE servers, load conditions of serving AIMLE servers, change in ML tasks requirement from AIMLE client, etc.) that can trigger the mobility of AIMLE context. Only VAL UE's mobility is considered.

### 5.3.2	Open Issues

Therefore, the following aspects need to be studied:

1.	Identify the use cases for supporting multiple PLMN/NPN, and across MNOs and CSPs/ECSPs scenarios

2.	Whether and how AIMLE services can support VAL UEs that roam into visited PLMNs or NPNs, e.g. registration, discovery, and access of AIMLE in the visited domain, as well as go back to home domain.

3.	Whether and how to ensure service continuity and switching for inference tasks or AI service sessions during UE mobility across MNOs and CSPs/ECSPs.

4.	Whether and how UEs or VAL functions can discover the most suitable AIMLE instance in edge-specific scenarios while UEs move across network boundaries or service areas, considering latency, capability, and service policy.

5.	Whether and how the AIMLE server and/or ML repository are aware of the serving PLMN/network information of the UE associated with the AIML client.

6.	Whether and how to enhance AIMLE context transfer capability given various conditions related to the AIMLE server capability, load or availability changes.

NOTE: Open issues 2, 3, 4, 5 and 6 will be based on the identified use cases in open issue 1.

## 5.4	Key issue #4: Support for Split AI/ML Operations Involving Multiple AIMLE clients, edge AIMLE servers and central AIMLE servers

### 5.4.1	Description

Split AI/ML operations have been introduced in 3GPP TS 23.482 [2]. The current framework is for single AIMLE client-to-network interaction and does not consider multiple AIMLE clients collaborating in a split AI/ML scenario. If  multiple AIMLE clients were to collaboratively execute split AIML operations within the same pipeline involving an Edge Server, the operation continuity may be disrupted if one client moves out of the service area during execution. Therefore, it is important to study the service continuity and optimization for split AIML operations involving multiple AIML clients.

In addition to the above multi-client scenario, emerging AI/ML applications require collaborative inference across multiple computing nodes (UE(s), edge(s), and cloud(s)) rather than a single operation. For example, in an autonomous driving or V2X scenario, a VAL UE (e.g. a connected vehicle) may only execute part of a model locally and rely on a roadside edge server or even a cloud server to complete the remaining inference. This is due to the vehicle's limited computing resources and the need for low-latency results to ensure safe and efficient driving decisions.

In Release-19, basic support for split ML model inference between an AIMLE client and an AIMLE server was introduced in 3GPP TS 23.482 [2]). However, those solutions assumed a relatively static allocation of inference tasks and did not fully explore more dynamic or collaborative inference involving multiple nodes. Therefore, it is necessary to study whether and how to enhance the AIML application enablement architecture and functions to support cloud-edge-device collaborative inference considering the conditions changes on these computing nodes (e.g. due to variations in edge server load).

### 5.4.2	Open Issues

This key issue will study:

1.	Define the use cases supported in the AIML enablement layer involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers requiring concurrent usage of the same split AI/ML operation pipeline.

2.	Whether and how the AIML enablement layer can support split AI/ML operations involving multiple AIMLE clients, edge AIMLE servers and central AIMLE servers.

3.	Whether and how the AIMLE enablement layer can support service continuity during split AIML operation involving multiple AIMLE clients.

4. Whether and how to support dynamic inference partitioning and coordinating across AIMLE clients, edge AIMLE servers and central AIMLE servers in AIMLE layer.

NOTE:	This open issue may involve the ability to split an AI/ML inference pipeline into segments that can be executed on different nodes based on real-time conditions such as latency requirements, processing capabilities, network bandwidth, and current load (which may be provided by VAL UE or 5GC).

## 5.5	Key issue #5: Support for AI Inference Exposure

### 5.5.1	Description

With the increasing deployment of ML model at the edge and cloud, the need for reusable, secure, and discoverable inference capabilities is becoming critical for enabling intelligent applications in verticals such as industrial automation, automotive, etc.

Currently, AI/ML inference is typically tightly coupled to the operator's own applications or specific vertical services. There is a growing demand to open up these capabilities as a standardized inference service that can be consumed by third-party applications or other service enablers. By exposing inference functions through standardized APIs, operators can enable new use cases, for example, an enterprise application could send sensor data to a mobile edge and request an AI-based analysis (inference) result in real time based on a suitable ML model. This requires exposing the inference capabilities of ADAES with the support from AIMLE services Therefore, enhancements of ADAES is needed to support the exposure of inference services based on the suitable ML model provided by AIMLE service.

### 5.5.2	Open Issues

Therefore, in order to support ML Model inference services exposure, the following aspects need to be studied:

1.	Whether and how to enhance ADAES service capabilities to expose ML model inference result based on the suitable ML model provided by AIMLE service in both central and edge scenarios.

## 5.6	Key issue #6: Support for Architecture and Functions Enhancement on diverse AIMLE deployments scenarios

### 5.6.1	Description

SA6 has specified the application layer architecture to enable AI/ML services. In Rel-20, the AIMLE functional architecture requires enhancements to further improve and enhance functionality in 3GPP for improved support for AI/ML services. It requires for more investigation on the implications when supporting AIMLE capabilities in scenarios where the AIMLE is deployed in distributed or hierarchical fashion as indicated in Annex A.3 and A.4 of 3GPP TS 23.482 [2]. In this study, it is expected to explore potential enhancement for supporting such deployments, considering also different implementations of the ML repository in edge or cloud platforms, and different assumptions on the access of ML repositories by VAL and other enablers. In this regard, considering the various AIML capabilities provided by the VAL UE, the collaboration procedures among AIMLE Clients, edge AIMLE servers, and central AIMLE servers need to be enhanced in terms of data management and AIML operation.

In Rel-19, the AIMLE capabilities assume that the VAL server and AIMLE servers are pre-configured with AIMLE server endpoint information (e.g., URI, IP address). Such pre-configured AIMLE server endpoint information poses the following issues or architectural limitations:

- Limited flexibility – VAL server may need to dynamically select AIMLE server based on location, application domain. For example, a healthcare related VAL server may need to communicate with a dedicated AIMLE server for healthcare for privacy reasons.

- Incompatible with mobile users – If the user location changes, the pre-configured AIMLE server may not be able to serve the user or AIMLE client. This may lead to service disruption. The current specification includes AIMLE context transfer but only after the client is associated with a new server.

- Reduced resilience – If AIMLE server becomes unreachable, there is no fall-back mechanism for the VAL server or AIMLE clients. AIMLE server can enable a graceful shutdown by informing VAL server and AIMLE clients about an alternative server end-point information. However, in case of abrupt shutdown, there is no way to get the information related to AIMLE server.

- Multi-tenant environments – In a multi-tenant setup, different AIMLE servers can enforce different trust zones or security policies. VAL servers must find an AIMLE server that is compliant with the tenant profile.

The need for availing services from another AIMLE server may arise at AIMLE client for various conditions like change in AIMLE client's ML task requirement, AIMLE client's mobility planning/requirements etc. There are no mechanisms specified in Rel-19 for discovery of AIMLE servers.

### 5.6.2	Open Issues

This key issue will study:

1.	Study and identify the use cases and definition of hierarchical and distributed AIMLE architecture.

2.	Whether and how to enhance the hierarchical and distributed AIMLE architecture and related functions to support application layer AI/ML services operating where the AI/ML service is running in AIMLE clients, e.g. AIMLE server/client registration, update, discovery, selection.

3.	Whether and how to enhance the architecture and related functions to support for AIMLE services, AIML operation assuming diverse (e.g., hierarchical and distributed) deployments scenarios.

4.	Whether and how to enhance data management assistance among AIMLE Clients, edge AIMLE servers, and central AIMLE servers in diverse (e.g., hierarchical and distributed) AIMLE deployments scenarios.

5.	Whether and how the AIMLE services can support the discovery of AIMLE servers.

6.	Whether and how the conditions under which a change of AIMLE server is identified.

## 5.7	Key Issue #7: VFL sample alignment across VAL servers

### 5.7.1	Description

In the Vertical Federated Learning (VFL) process, the active party or coordinating server collects partial results from other participants (passive parties) to obtain a global model. VFL participants can be VAL servers, AIMLE servers, and/or AIMLE clients. VFL participants share the same sample space but have different feature sets. Therefore, to achieve VFL process, these different VFL participants require sample alignment to participate in a VFL operation. In some cases, how these samples are identified by different participants can also differ. Sample alignment is crucial for this process as it allows matching corresponding data entries across different datasets in a privacy-preserving manner.

The AIMLE server, which has the ability to know the identity of VAL UEs, can accomplish sample alignment securely and in a privacy-preserving way without revealing the VAL UE identity.

### 5.7.2	Open Issues

This key issue will study:

1.	Whether and how to enhance VFL operation considering also VAL servers and AIMLE servers as VFL participants.

2.	Whether and how to support sample alignment also considering VAL servers / AIMLE servers as VFL participants.

## 5.8	Study on Support for AI Inference service performance management and feedback

### 5.8.1	Description

AIMLE architecture currently provides mechanisms to request ML inference but lacks support for managing and observing the runtime quality of inference services. For example, in a vehicle-road collaboration scenario, a vehicle may request a semantic perception result that must be returned within 50ms. If that is not achieved, the application should be notified. Similarly, in scientific computing, model inference may degrade in accuracy due to hardware limitations, the feedback to the control plane is needed. Therefore, AIMLE architecture should consider providing:

-	A way for VAL UEs or AIMLE clients to express desired service performance parameters when requesting inference (e.g., max latency, expected confidence level),

-	Runtime monitoring of whether these conditions are met, and notification or reporting mechanisms for inference failures, degradation, or exceptions,

Although AI models are central to inference, the service performance of inference delivery depends on more than the model itself. The same model may produce different service experiences depending on runtime conditions (e.g., server load, network delays). The QoS of a model is generally static unless the model is retrained, whereas the performance of inference services may vary depending on resource conditions. Even if the model degrades, the inference service may still meet the required service performance.AI inference service consumers care about whether the results are delivered on time, not just whether the model is accurate. Thus, service performance observability for inference is a distinct layer of functionality, necessary to make AIMLE more reliable and useful in dynamic and distributed deployments. This is particularly important in multi-edge deployments, and industrial scenario.

### 5.8.2	Open Issues

Therefore, the following aspects need to be studied:

-	Whether and how AIMLE should support expressing inference service performance parameters (e.g., expected latency, accuracy, confidence level) in service requests.

-	Whether AIMLE requires mechanisms for real-time feedback from clients (e.g., inference result too late, failed, or inaccurate).

# 6	Enhanced architecture for enabling AI/ML services

## 6.1	General

## 6.2	Enhanced AI/ML enablement functional model

In this clause the architecture enhancements relate to the support for multi-operator services. There are two cases for the multi-operator service support:

- An AIMLE server connected to AIMLE clients over different PLMNs

- Each AIMLE server connected to AIMLE clients of single PLMN, whereas the AIMLE server interact each other via AIML-E.

In the first scenario, the AIML service (VAL service, ASP service) runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN.

In Figure 6.2.1-1, the VAL UE#1 is connected to PLMN#1 and VAL UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. In this scenario, different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

Figure 6.2.1-1 architecture for multi-operator support with common AIMLE server

In the second scenario, as depicted in Figure 6.2.1-2, the VAL UEs are assumed to have AIMLE clients installed and active and connect to different AIMLE servers provided by different MNOs or trusted 3rd parties of the MNOs. In Figure 6.2.1-2, each AIMLE server connects to each PLMN. The VAL UE 1 is connected to PLMN1 and AIMLE server 1 and VAL UE 2 is connected to PLMN 2 and AIMLE server 2.

Figure 6.2.1-2 architecture for multi-operator support with distributed AIMLE servers

Editor's Note 1: It is FFS whether and in which scenarios the VAL server has service agreements with one or both 3GPP network system operators in multi-operator scenarios.

Editor's Note 2: The cardinality aspects of ML repositories and the requirement for new interface among them is FFS based on solutions’ progress.

### 6.2.2	AI/ML enablement functional model for roaming scenarios

This clause describes the AIMLAPP architecture for roaming UEs. This architecture is aligned with network roaming architecture, since it re-uses existing network roaming principles and describes the AIML service continuity while VAL UEs are roaming; hence how AIMLE spans across HPLMN and VPLMN is discussed. Such architecture applies to both roaming models (LBO and HR roaming architectures).

In Figure 6.2.2-1, the AIMLAPP architecture for enabling AI/ML services for roaming scenarios is showcased. In this architecture, AIML-UU is the interface between the AIMLE client and the AIMLE server, which can be over HPLMN or VPLMN which VAL UE is roaming. AIML-E supports the interaction among AIMLE servers deployed by different MNOs, whereas the MLR-E is the new interface supporting the interaction among repositories for aligning the information on the ML models and ML/FL members to allow the continuity of the AIML service while the VAL UE is roaming.

Figure 6.2.2-1 architecture for roaming support

Editor's Note: The requirement for a new interface among ML repositories of different PLMNs in roaming scenario is FFS.

### 6.2.3	AI/ML enablement functional model for federation

Editor's Note: The enhancements for federation support are FFS

## 6.3	Functional Elements

The functional elements corresponding to the architecture will be presented in this clause.

## 6.4	Reference Points

The reference points corresponding to the architecture will be presented in this clause.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 8.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |

## 7.1	Solution #1: Enhancement of AIMLE to support model inference

### 7.1.1	Solution description

This clause describes the procedure for supporting the ML model inference by the AIMLE/ADAE server. The VAL server requests the AIMLE/ADAE server for ML model inference by specifying the details about the ML model or requirements to select a ML model for inference.

Pre-condition:

-	The ML model needed to be used for this ML model inference is available by the AIMLE/ADAE server.

Figure 7.1.1-1 illustrates the procedure for AIMLE / ADAE server to support ML model inference based on the request from the VAL server.

Figure 7.1.1-1: ML model inference

1.	The VAL server sends an ML model inference request to AIMLE / ADAE server, requesting the inference result from the AIMLE / ADAE server. This request consists of ML model information or ML model inference requirement information, etc.

2.	The AIMLE / ADAE server checks whether the VAL server is authorized to perform the ML model inference request. If no model information is provided but only the model inference requirement information is provided in step 1, the AIMLE / ADAE server identifies and selects the appropriate ML model for inference based on the ML model inference requirement information using the procedure as specified in clause 8.11.3. The AIMLE / ADAE server handles the request and execute the inference task.

3.	If the VAL server is authorized, AIME server returns the inference response, otherwise a failure response indication the reason for failure.

### 7.1.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.1.3	Corresponding APIs

Table 7.1.3.1-1 describes the information flow from the VAL server to the AIMLE / ADAE server for requesting the ML model inference.

Table 7.1.3.1-1: ML model inference request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the VAL server performing the request. |
| Inference data (NOTE 1) | O | Original data for inference that may include text, images, audio, numerical data, etc. This IE may be included in the request in the scenario, e.g. the data is small. |
| Inference data index (NOTE 1) | O | An index of the data for inference. It is used to indicate where the ADAE function can get the original data for inference. This IE may be included in the request in the scenario, e.g. the data is big, or stored in different servers. |
| Inference Parameters | O | Additional inference parameters that may be needed, such as maximum output length, temperature, batch size, etc., depending on the specific requirements of the model. |
| ML model information (NOTE 2) | O | Identifies the ML model that VAL server specified to perform inference. This information consists of the model identifier, address (e.g., a URL or an FQDN) of the ML model file or address of the model repository where the ML model resides |
| ML model inference requirement information (NOTE 2) | O | Identity the type or goal of the inference task, such as 'text classification,' 'image recognition,' 'sentiment analysis,' etc |
| NOTE 1: Only one of these IE shall be present. NOTE 2: At least one of these IE shall be present. |  |  |

Table 7.1.3.2-1 describes the information flow of ML model inference response from the AIMLE / ADAE server to the VAL server.

Table 7.1.3.2-2: ML model inference response

| Information element | Status | Description |
| --- | --- | --- |
| Inference response | O | Indicates that the ML model inference request was successful or not. |
| >Inference result (NOTE) | O | If the inference ML model inference request is successful, the Inference result is included in the response. |
| > ML model identifier | O | Identifies the ML model selected by AIMLE / ADAE server for inference. |
| > Cause (NOTE) | M | If the inference ML model inference request is failed, the reason for the failure is included in the response. |
| NOTE: Only one of these information elements shall be present |  |  |

### 7.1.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.2	Solution #2: Enhance AIMLE Data Management Assistance for Data Drift Detection

### 7.2.1	Solution Description

As introduced in 3GPP TS 23.482, the AIMLE data management assistance service is the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation and data analysis. In the context of AI/ML, it is important that the data used by the ML model is not significantly different from the data used to train the model (e.g. the distribution of the features have changed over time). If the difference is higher enough, the model's performance tends to decrease. This discrepancy is known as data drift, that is data drift refers to changes in the statistical properties of data over time leading to a decline in model performance.

Therefore, it is essential that the data management system procedure provides the capability to detect whether data drift has occurred to assist in decision-making if the model needs to be updated due to a decrease in its performance.

Data drift detection procedure allows to assist ML model maintenance procedures. To support ML model management, e.g. assist ML model maintenance, the AIMLE data management assistance service need be enhanced by adding drift detection functionality.

ML model evaluation profile ID is provided as part of data drift requirements to allow the AIMLE server with obtaining ML model evaluation information for performing data drift detection.

The following clauses specify the impact to existing AIMLE procedures for the enhancements of AIMLE data management assistance service.

#### 7.2.1.1	Impact to existing AIMLE procedures

The AIMLE procedures and information flows in 3GPP TS 23.482 can be enhanced (highlighted in bold italics) as follows.

8.15 AIMLE data management assistance 8.15.1 General AIMLE data management assistance is the process of the AIMLE server assisting AIMLE service consumers with managing data operations performed by VAL clients. The data operations include data preparation, and data analysis, and data drift detection. The AIMLE server offloads the AIMLE service consumer from interacting with AIMLE clients to manage the data operations. The VAL clients perform the actual data preparation, and data analysis, and data drift detection operations and send the outputs to the AIMLE server for aggregation. The following clauses specify procedures, information flows, and APIs for AIMLE data management assistance. 8.15.2 Procedure Pre-conditions: 1. AIMLE clients have registered with AIMLE server. 2. UE application data for AI/ML operations have been collected and dataset identifier have been assigned to each dataset. EVEX mechanism can be reused for data collection as described in 3GPP TS 26.531 [10]. Figure 8.15.2-1: AIMLE data management assistance 1. An AIMLE service consumer (e.g., VAL server) makes a request for AIMLE data management assistance. The request includes information as described in Table 8.15.3.1-1. 2. The AIMLE server authenticates and authorizes the request. If authorized, the AIMILE server assigns an identifier for the subscription. 3. The AIMLE server sends an AIMLE data management assistance subscription response to the AIMLE service consumer. The response includes information as described in Table 8.15.3.2-1. 4. The AIMLE server sends Client data processing trigger requests to AIMLE clients. The request can be for data preparation or data analysis. The request includes information as described in Table 8.15.3.4-1. 5. Each AIMLE client sends the requirements to trigger data management for the VAL client to perform the requested data operation. The VAL client performs the operation locally. NOTE: The data preparation and data analysis functions operate in a similar manner as ML models. The AIMLE layer is able to transport the functions to the VAL clients and the VAL clients performs the associated function on the application data as part of data preparation or data analysis in a similar manner as ML training. 6. After the data operation completes, each AIMLE client sends a response to the AIMLE server with information as described in Table 8.15.3.5-1. 7. The AIMLE server aggregates the output of each AIMLE client. Aggregation includes combining or performing a statistical operation on the received data. Data received from the AIMLE clients can be numerical or categorical, which allows the AIMLE server to aggregate the output data from the AIMLE clients. If necessary, steps 4–7 is repeated to complete all the required data operations. 8. The AIMLE server sends an AIMLE data management assistance notification to the AIMLE service consumer with information as described in Table 8.15.3.3-1. 8.15.3 Information flows 8.15.3.1 AIMLE data management assistance subscription request Table 8.15.3.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the AIMLE data management assistance subscription procedure. Table 8.15.3.1-1: AIMLE data management assistance subscription request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management operations; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE 1); Data preparation requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE 1); Data analysis requirements as detailed in Table 8.15.3.1-3.. > Data drift detection requirements; O (NOTE 1); Data drift detection requirements in Table 8.15.3.1-x.. AIMLE clients list; O (NOTE 2); A list of AIMLE clients for which data management should be performed. The list may be specified by AIMLE client set identifier.. AIMLE client selection criteria; O (NOTE 2); Selection criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided.. Table 8.15.3.1-2: Data preparation requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data preparation requirements; M; Requirements for data preparation.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature to process.. > Data preparation function; M; This indicates the function which prepares the data, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. Table 8.15.3.1-3: Data analysis requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Dataset analysis requirements; M; Requirements for data analysis.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data analysis function; M; This indicates the function which performs the data analysis, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. Table 8.15.3.1-x: Data drift detection requirements Information element; Status; Description. Dataset identifier; M; An identifier for the dataset.. Data drift detection requirements; M; Requirements for data drift detection.. > ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. > Dataset ID; M; The identifier for the dataset.. > Dataset feature ID; M; Identifier or name of dataset feature.. > Data drift detection function; O; This indicates the function which performs the drift detection, and it could be an identifier of a function if the function is available locally at the UE or an executable included in the request.. 8.15.3.3 AIMLE data management assistance notify Table 8.15.3.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the AIMLE data management assistance subscription procedure. Table 8.15.3.3-1: AIMLE data management assistance notify Information element; Status; Description. Status; M; The status for the data management operation. Aggregated data preparation outputs; O (NOTE 1, NOTE 2); Provides outputs for data preparation: dataset identifier, dataset features, and prepared data output.. Aggregated data analysis outputs; O (NOTE 1, NOTE 2); Provides outputs for data analysis: dataset identifier, statistical outputs for each feature, list of outlier and anomaly values, and feature correlation information.. Aggregated data drift detection outputs; O (NOTE 1, NOTE 2); Provides outputs for data drift detection: dataset identifier, data drift detection status (e.g. detected data drift of the dataset features).. Timestamp; O; Timestamp of the data management notification. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal.. 8.15.3.4 Client data processing trigger request Table 8.15.3.4-1 shows the request sent by the AIMLE server to AIMLE clients for the AIMLE client data processing trigger procedure. Table 8.15.3.4-1: Client data processing trigger request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Data management type; M; An indicator showing what data management type is being requested: data preparation, data analysis.. Data management requirements; M; Requirements for the data management request:. >Data preparation requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-2.. >Data analysis requirements; O (NOTE); Data collection requirements as detailed in Table 8.15.3.1-3.. > Data drift detection; O (NOTE); Data drift detection requirements as defined in Table 8.15.3.1-x.. Operational schedule; O; A schedule to perform the requested data management operation.. NOTE: At least one of the information elements shall be provided.. 8.15.3.5 Client data processing trigger response Table 8.15.3.5-1 shows the response sent by AIMLE clients to the AIMLE server for the Client data processing trigger procedure. Table 8.15.3.5-1: Client data processing trigger response Information element; Status; Description. Status; M; The status for the data management operation. Data preparation outputs; O (NOTE 1); The output data after performing data preparation. One output data is generated for each requirement. (NOTE 2). Data analysis outputs; O (NOTE 1); The output data generated by data analysis. One output data is generated for each requirement. (NOTE 2). Data drift detection outputs; O; The output of data drift detection status (e.g. detected data drift of the dataset features). (NOTE 2). Timestamp; O; Timestamp of the data management operation. NOTE 1: At least one of the information elements shall be provided in the output. NOTE 2: The output format can be numerical or categorical. If categorical, the format can be nominal or ordinal. The AIMLE server is able to process either type of data..

### 7.2.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.2.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.2.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.3	Solution #3: New AIMLE Service on ML Model Maintenance

### 7.3.1	Solution Description

It is important to continuously monitor the model's performance and check for any changes in the statistical nature of the data that is used for training the ML model or be used by the ML model for its task. In this scenario, ML model maintenance is needed to check e.g., whether the model's performance aligns with expectations or data drift appears. Then, the AIMLE server may initiate model update process.

AIMLE service provides model maintenance service to the consumer (e.g. VAL server) based on request, e.g., to detect data drift, and initiate ML model update process. The information of data drift in specific dataset provided to the AIMLE server may be the data drift analysis output from the AIMLE client, and/or the data drift detection outputs from the VAL client via the AIMLE client.

ML model evaluation profile ID is provided as part of ML model maintenance requirements to allow the AIMLE server with obtaining ML model evaluation information for performing ML model performance evaluation.

The following clauses specify procedures, information flows and APIs for Key Issue #2 to support new AIMLE service on ML model maintenance.

#### 7.3.1.1	Procedure

Pre-conditions:

1.	AIMLE clients have registered with AIMLE server.

Figure 7.3.1.1-1: AIMLE support ML model maintenance procedure

1.	An AIMLE service consumer (e.g., VAL server) sends a ML model maintenance subscription request to the AIMLE server. The request includes information as described in Table 7.3.1.2.1-1.

2.	The AIMLE server authenticates and authorizes the request. If authorized, the AIMLE server assigns an identifier for the subscription.

3.	The AIMLE server sends an ML model maintenance subscription response to the AIMLE service consumer. The response includes information as described in Table 7.3.1.2.2-1.

4.	The AIMLE server performs operations for ML model performance evaluation to check the ML model performance. The AIMLE server retrieves the ML model evaluation information from the provided ML Evaluation profile ID as described in clause 7.4.1.3 and performs the ML model performance evaluation. If the performance of the ML model is not as expected as request in step 1, the AIMLE server determines date drift detection. The ML model performance evaluation can use the ML model performance monitoring service as described in 3GPP TS 23.482 [2] clause 8.22 or use offline ML model evaluation procedure.

Editor's Note:	The detail procedure for offline ML model performance evaluation is FFS.

5.	The AIMLE server request and get client data processing trigger response as described in 3GPP TS 23.482 [2] clause 8.15 with data drift detection outputs from the AIMLE client, by using the enhanced AIMLE data management assistance service as described in Solution #2 steps 4-6 with enhanced information flows.

6.	Based on the data drift information received in step 5 and the ML model evaluation information obtained in step 4, the AIMLE server determines if actions are required (e.g. model replacement (e.g. select other ML model) or transfer learning (e.g. re-train the ML model based on the original ML model)) to keep the ML model performance within the performance requirements received in step 1.

7. The AIMLE server sends an ML model maintenance notification based on the notification requirements received in step 1 to the AIMLE service consumer with information as described in Table 7.3.1.2.3-1.

#### 7.3.1.2	Information flows

##### 7.3.1.2.1	ML model maintenance subscription request

Table 7.3.1.2.1-1 shows the request sent by an AIMLE service consumer to an AIMLE server for the ML model maintenance subscription.

Table 7.3.1.2.1-1: ML model maintenance subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| ML model ID | M | The identifier of the model that will be maintained. |
| ML model retrieval endpoint | O (NOTE 1) | The endpoint (e.g., URL, URI, IP address) where the ML model file can be retrieved. |
| ML model | O (NOTE 1) | The ML model that will be maintained. |
| ML model maintenance requirements | M | Requirements for the ML model maintenance. |
| > ML model performance requirements | M | ML model performance requirements for the maintenance. |
| > ML model evaluation profile ID | M | The identifier of the ML model evaluation profile. |
| > Dataset ID | O (NOTE 2) | An identifier for the dataset, which is used for training the ML model, or be used by the ML model for its task. |
| > AIMLE clients list | O (NOTE 2) | A list of AIMLE clients from which the datasets for the ML model maintenance applied for. |
| > Maintenance mode | M | The maintenance mode, e.g., periodically or event triggered (e.g. ML model performance degradation detected, data drift detected). |
| Notification requirements | M | Requirements for notification, e.g., periodically or event triggered (e.g. ML model updated). Notify with e.g. ML model, or ML model status (e.g. updated ML model ID, ML model version). |
| Notification endpoint | O | The notification endpoint where the notifications should be sent to. |
| Expiration time | O | The proposed expiration time of the subscription. |
| NOTE 1: At least one of the information elements shall be provided. NOTE 2: At least one of the information elements shall be provided. |  |  |

##### 7.3.1.2.2	ML model maintenance subscription response

Table 7.3.1.2.2-1 shows the response sent by the AIMLE server to the AIMLE service consumer for the ML model maintenance subscription.

Table 7.3.1.2.2-1: ML model maintenance subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the ML model maintenance subscription request was successful. |
| > Subscription identifier | M | An identifier for the subscription. |
| > Expiration time | O | Expiration time for the subscription |
| Failure response | O (NOTE) | Indicates that the request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: One of the information elements shall be present. |  |  |

##### 7.3.1.2.3	ML model maintenance notification

Table 7.3.1.2.3-1 shows the notification sent by the AIMLE server to the AIMLE service consumer for the ML model maintenance subscription.

Table 7.3.1.2.3-1: ML model maintenance notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier of the subscription. |
| Model maintenance outputs | M | Provides outputs for model maintenance based on request: e.g. ML model, or ML model status (e.g. updated ML model ID, ML model version). |
| Timestamp | O | Timestamp of the data management notification. |

### 7.3.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.3.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.3.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.4	Solution #4: Management of evaluation information for ML model performance and correctness evaluation.

### 7.4.1	Solution description

#### 7.4.1.1	Introduction

This solution proposes enhancements to address Key Issue #2 – Open Issue #2 and provides procedures required to enable evaluation of ML model performance and correctness considering application specific ML models.

This solution proposes to:

-	define new procedures and information flows to enable management (e.g., storage, retrieval, removal) of ML model evaluation information in the ML model repository;

-	update information flows to link the ML model evaluation information with ML model(s).

The solution allows the owner of ML model(s) to store and manage the evaluation information associated with ML model(s). The evaluation information is information that can be retrieved to perform the evaluation of performance and correctness of ML model(s). This solution can be combined with existing and/or new procedures when evaluation of a ML model performance and correctness is required.

ML models that can be stored and managed in the AIML enablement layer are application specific ML models. Such ML models can be retrieved by the AIMLE client or VAL server to perform inferencing at the VAL layer.

ML model specific evaluation information is required for a consumer of a ML model to perform evaluation of a ML model performance and correctness. Thus, this solution proposes to enable storage and retrieval of ML model evaluation information in the ML model repository for enabling the performance and correctness evaluation of application specific ML models.

The ML model specific evaluation information can be used by monitoring solutions for data drift detection (solution #2) and monitoring a ML model's performance (solutions #3 and #6).

#### 7.4.1.2	ML model evaluation information storage

Figure 7.4.1.2-1 describes procedures for storing ML model evaluation information in the ML model repository.

Figure 7.4.1.2-1: ML evaluation information storage

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information storage request to the AIMLE server. The request includes the requestor identifier and ML evaluation information.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information storage request to store the ML evaluation information in the ML repository. The request includes the requestor identifier and ML evaluation information.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository stores the ML evaluation information, creates an ML evaluation profile including a ML evaluation profile identifier and sends an ML evaluation information storage response to the AIMLE server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information storage response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile identifier if the request was successful.

#### 7.4.1.3	ML model evaluation information retrieval

Figure 7.4.1.3-1 describes procedures for retrieving ML model evaluation information from the ML model repository.

Figure 7.4.1.3-1: ML evaluation information retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information retrieval request to the AIMLE server. The request includes the requestor identifier and ML evaluation information retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information retrieval request to obtain the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information retrieval filters.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository identifies ML evaluation profiles based on the ML evaluation information retrieval filters and sends a ML evaluation information retrieval response to the requestor. The response includes an indication of success for the request and the identified ML evaluation profiles if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information retrieval response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation profile(s) if the request was successful.

#### 7.4.1.4	ML model evaluation information removal

Figure 7.4.1.4-1 describes procedures for removal of ML model evaluation information from the ML model repository.

Figure 7.4.1.4-1. ML evaluation information removal

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML evaluation information removal request to the AIMLE server. The request includes the requestor identifier and ML evaluation information identifier(s) to be removed.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML evaluation information removal request to remove the ML evaluation information from the ML repository. The request includes the requestor identifier and ML evaluation information identifier(s).

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository removes the ML evaluation information based on the ML evaluation information identifier(s) and sends a ML evaluation information removal response to the requestor. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML evaluation information removal response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML evaluation information identifiers that were removed if the request was successful.

### 7.4.2	Architecture Impacts

No architecture impact has been identified.

### 7.4.3	Corresponding APIs

#### 7.4.3.1	ML Evaluation Profile

Table 7.4.3.1-1 describes the ML evaluation profile and Table 7.4.3.1-2 describes the ML evaluation which is used in the ML evaluation information management procedures between the AIMLE client, VAL server, AIMLE server and ML model repository.

Table 7.4.3.1-1: ML Evaluation Profile

| Information element | Status | Description |
| --- | --- | --- |
| ML evaluation profile identifier | M | The identifier of the ML evaluation profile. |
| AIMLE server identifier | M | The identifier of the AIMLE server that stored the ML evaluation information. |
| ML repository identifier | M | The identifier of the ML repository where the ML evaluation information is stored. |
| List of ML evaluations | M | The list of ML evaluation information as defined in Table 7.4.3.1-2. |

Table 7.4.3.1-2: ML Evaluation Information

| Information element | Status | Description |
| --- | --- | --- |
| Evaluation identifier | M | The identifier of the evaluation information used for evaluating ML model(s). |
| List of ML models identifiers | M | The list of ML models identifiers to which the evaluation applies. |
| Source identifier | O | The identifier of the source (e.g., VAL server ID, VAL client ID) that stored the ML evaluation information in the ML repository. |
| Description | M | The description of the ML model evaluation. |
| Metric information | M | The metrics measured by the performing the ML model evaluation. |
| Input information | M | Represents the input information which is aligned with the associated ML model input parameters. The format of input information for application specific ML models is out of scope. |
| Output information | M | Represents the expected output information which is expected as a result of processing the input information by the ML model. The format of output information for application specific ML models is out of scope. |
| Grading information | M | The information for evaluating and grading the performance and correctness of a ML model (e.g., grading the obtained evaluation output based on the expected output). |
| Execution information | O | The execution information which is required to evaluate performance and correctness of a ML model. The information indicates whether the evaluation is executed once, repeatedly in a time window, has its output averaged, etc. If not included, the evaluation is evaluated and graded once. |

#### 7.4.3.2	ML Evaluation Information Storage

Table 7.4.3.2-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for storing ML evaluation information in the ML repository.

Table 7.4.3.2-1: ML Evaluation Information Storage Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation information | M | The list of ML evaluation information to be stored in the ML repository as described in Table 7.4.3.1-2. |

Table 7.4.3.2-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information storage request.

Table 7.4.3.2-2: ML Evaluation Information Storage Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > ML evaluation profile identifier | M | The ML evaluation profile identifier stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.3	ML Evaluation Information Retrieval

Table 7.4.3.3-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for retrieving ML evaluation information from the ML repository.

Table 7.4.3.3-1: ML Evaluation Information Retrieval Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Evaluation information filters | M | The evaluation information filter used to identify the retrieved ML evaluation information. |
| > Evaluation profile identifier | O | Represent the filtering criteria which is a ML evaluation profile identifier. |
| > Evaluation information filter | O | Represents the filtering criteria, which can be any of the ML evaluation information as defined in Table 7.4.3.1-2. |

Table 7.4.3.3-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information retrieval request.

Table 7.4.3.3-2: ML Evaluation Information Retrieval Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profiles | M | The list of retrieved ML evaluation profiles stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.4	ML Evaluation Information Removal

Table 7.4.3.4-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for removing ML evaluation information from the ML repository.

Table 7.4.3.4-1: ML Evaluation Information Removal Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of ML evaluation profile identifiers | M | The list of evaluation profile identifiers to be removed from the ML model repository. |

Table 7.4.3.4-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML evaluation information removal request.

Table 7.4.3.4-2: ML Evaluation Information Removal Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML evaluation profile identifiers | M | The list of ML evaluation profile identifiers removed from the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.4.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

## 7.5	Solution #5: Solution on monitoring ML-enabled analytics correctness

### 7.5.1	Solution description

This solution addresses Key Issue #2 and open issue 3.

In this solution, the ADAES is providing the analytics performance monitoring where the AIMLE server detects an expected or predicted model degradation performance based on the ML-enabled analytics performance feedback. Such ADAE analytics (as captured in 3GPP TS 23.436) can be about the edge load or VAL server or session performance analytics which utilizes ML methods (training by AIMLE) for predicting the end-to-end performance.

Pre-conditions:

1.	One or more ADAE services (at the ADAE server or clients) using the given ML model training service from AIMLE server are ongoing.

2.	The AIMLE server (or VAL server) detects a requirement to monitor the analytics performance. Such trigger may be based on VAL server request for the ML model performance monitoring capability as specified in 3GPP TS 23.482 clause 8.22.2.

Figure 7.5.1-1: Procedure for ML-enabled analytics monitoring

1.	The AIMLE server sends an ADAE analytics performance monitoring subscription request to the ADAE server (ADAES). The subscription request includes the following parameters: an ADAE analytics ID or type, a VAL server ID, an area of interest and time of validity, a monitoring reporting configuration (thresholds for triggering a monitoring event, e.g. minimum accuracy, confidence level, whether the reporting is one time or periodical or event-based), a KPI for the analytics service, VAL policies for triggering an action based on a monitoring event (e.g. if analytics degradation is detected, re-start the analytics process).

2.	The ADAE server authorizes the subscription request and sends an ADAE analytics performance monitoring subscription response to the AIMLE server. The response is a positive or negative acknowledgement of the subscription and includes a subscription ID in case of success and a cause in case of failure (e.g. failure to authorize).

3.	The ADAE server discovers based on the analytics ID the ADAECs or other entities (e.g. edge ADAES) which are undertaking an analytics task for the given analytics ID (e.g. local prediction at UE side or ADAES side).

4.	The ADAES server sends a request to the discovered entities (ADAEC or edge ADAES) indicating a monitoring requirement for the analytics task. This request includes the analytics ID, the requestor ID, the event trigger criteria for providing a monitoring report (e.g. reaching a threshold), the target KPIs to be monitored (e.g. accuracy), the area and time of interest, whether this is a predicted parameter or actual parameter to be monitored. Such request can be in form of an analytics correctness feedback request either real time or when the analytics task finishes.

5. The discovered ADAEC(s) and/or edge ADAES start monitoring the performance of the analytics task and in particular the attributes which are requested (accuracy, delay, evaluation vs ground truth data). This involves the continuous checking of the performance (accuracy, latency) of the analytics service for providing a notification / trigger event to the AIMLE server identifying a deviation in terms of ADAE analytics performance. Such identification can be a detection of a mismatch in the analytics performance with a possible cause (e.g. data producer, ML model, UE/ channel conditions, data drift).

6. The discovered ADAEC(s) and/or edge ADAES based on the trigger criteria, send to the ADAES a monitoring response, indicating an expected or predicted degradation of the performance of the analytics task. Such message may also include the possible cause as in step 6 (e.g. data producer, ML model, UE/ channel conditions, data drift).

7. The ADAES identifies or predicts an ADAE service degradation or performance (against the KPIs for that analytics ID) and sends a notification to the AIMLE server detecting a degradation / mismatch at the target performance for the analytics task.

8. The AIMLE server based on the received monitoring notification from the one or more AIMLE clients, it identifies an expected or predicted ML model degradation using the AIMLE capability for ML model performance monitoring as specified in 3GPP TS 23.482 clause 8.22.2 and can optionally use the ADAE capability for ML model degradation detection in clause 8.19.2.

Alternatively, the AIMLE server after step 2 and/or the ADAE server after step 3 send requests to A-DCCF for collecting analytics performance information. The procedure for ADAE Server to request data collection via A-DCCF are as specified in clause 8.12 of 3GPP TS 23.436 [4].

Editor's Note: Whether the procedure specified in clause 8.12 of 3GPP TS 23.436 [4] can be reused with enhancements for AIMLE Server to request data collection via A-DCCF is FFS.

### 7.5.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.5.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.5.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.6	Solution #6: Enhancements to performance monitoring

### 7.6.1	Solution description

#### 7.6.1.1	Enhancements to ML model performance monitoring

The ML model performance monitoring procedure as specified in clause 8.22 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

### 8.22.2 Procedure Pre-conditions: 1. One or more AIMLE services (at the AIMLE server or clients) using the given ML model are ongoing. Figure 8.22.2-1: ML model performance monitoring 1. The VAL server sends an ML model performance monitoring subscription request to AIMLE server, requesting to assist in monitoring the ML model performance. This request consists of ML model information and optionally the AIML service information, etc. The request also includes the ML model evaluation profile ID which is associated with a ML model evaluation profile containing the baseline validation dataset information which is the baseline reference for evaluation of HFL model after each round of parameter aggregation. 2. The AIMLE server checks whether the VAL server is authorized to perform the ML model performance monitoring request. 3. If the VAL server is authorized, AIMLE server returns the success response, otherwise a failure response indication the reason for failure. 4. The AIMLE server identifies the AIMLE services which are utilizing the requested ML model. Such AIMLE services can be an ML model training service or an HFL service at the AIMLE server or clients. The identification of the AIMLE services may be performed via fetching ML model information from the ML repository using ML model management procedure as in clause 8.11.3. The ML Evaluation profile ID is used to obtain the ML model evaluation profile (e.g., baseline validation dataset) which is used to determine the ML model performance. Then, the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service. In order to get the information, the request is enhanced with following IEs: Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. Aggregated AI/ML model parameters; M; Information about the AI/ML model and model parameters for use in HFL training.. Performance Metrics; M; The performance metrics that are used to evaluate the performance of model (e.g. accuracy, latency). ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. Each AIMLE client use the ML evaluation profile ID to obtain the ML model evaluation profile which is used to determine the ML model performance. Each AIMLE client evaluates the performance of the received aggregated global model on the local data based on the received Aggregated AI/ML model parameters, performance metrics and ML model evaluation profile. Each AIMLE client sends the performance report of aggregated global model on the local data as the response message to AILMLE Server. The performance report response message includes information as described in the following table. The response is enhanced with following IEss: Information element; Status; Description. AIMLE Client ID; M; The identifier of the AIMLE Client.. AIMLE service ID; M; The service identifier for the AIMLE HFL training operation.. Performance records; M; The evaluation performance records (e.g. on accuracy, on latency) of received aggregated global model from AIMLE Server on local data.. ML model evaluation profile ID; M; The identifier of the ML model evaluation profile used to perform the performance evaluation.. 5. The AIMLE server detects an expected ML model degradation (e.g., model drift, data drift) based on the deviation of the performance of the AIMLE service as indicated in step 4 . 6. The AIMLE server based on the expected ML model degradation, it may also indicate and execute a trigger action to ensure meeting the AIMLE service requirement. Further, AIMLE Server sends a configuration request to increase training collaboration to only AIMLE Clients that have degraded performance records. Such trigger action may be either an adaptation of the AIMLE service, such as training of a new ML model for the AIMLE by the same or a different AIMLE client, or re-training of the ML model by the same or different AIMLE client; or termination of the AIMLE service and initiating a new AIMLE service with a new ML model. NOTE: If this action involves re-selecting an AIMLE client for the AIMLE service, this is based on the procedure defined in clause 8.9. 7. The AIMLE server notifies the VAL server on the expected ML model degradation and if requested the triggered adaptation of the AIMLE service. 8.22.3 Information flows 8.22.3.1 ML model performance monitoring subscription request Table 8.22.3.1-1 shows the request sent by a VAL server to an AIMLE server for the ML model performance monitoring. Table 8.22.3.1-1: ML model performance monitoring subscription request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. ML model identifier; M; The identifier of the ML model for which the monitoring applies.. Notification endpoint; M; The notification endpoint (e.g. URL/URI/IP address) where the notifications should be sent to.. AIML operation information; O; The AIMLE operation (ML model training, HFL, VFL, TL) for which the ML model is used.. > VAL service ID; O; The VAL service identifier of the AIMLE service using the ML model (if known by the requestor).. > AIMLE client ID(s); O; The identifier(s) of the AIMLE client(s) training the ML model (if known by the requestor).. > AIMLE service KPI; O; One or more KPIs for the AIMLE service performance (latency, accuracy, etc).. ML model evaluation profile ID; M; The identifier of the ML model evaluation profile.. Monitoring report configuration; M; The reporting configuration for the monitoring service (thresholds for triggering a monitoring event, e.g. minimum accuracy, delay, whether the reporting is one time or periodical or event-triggered).. Area of interest; O; The geographical or service area for which the monitoring applies.. Time validity; O; The time validity for the monitoring subscription.. Trigger Action requirement; O; This requirement identifies policies for triggering an action based on a monitoring event (e.g. if degradation is detected, to train a new model or re-selecting AIMLE clients)..

#### 7.6.1.2	Enhancements to HFL training

As per step 4 of Clause 8.22 of 3GPP TS 23.482 [2], for performance monitoring of AIMLE clients clause 8.12.2 is reused, quote:

“the AIMLE server then starts monitoring the AIMLE service performance (e.g. accuracy, a KPI or QoS metric related to the AIML operation). This step includes receiving information from one or more AIMLE clients performing an operation based on the target ML model (e.g., based on step 7 of procedure in clause 8.12.2 or step 6 of procedure in clause 8.15.2), with an expected or experienced deviation of the required performance of the AIMLE service.”

The current solution proposes to enhance clause 8.12 as follows (new text in bold italics):

### 8.12.2 Procedure Pre-conditions: 1. AIMLE client discovery and selection have been performed. 2. Datasets are available and prepared for AI/ML training at the AIMLE clients. The datasets are assigned identifiers for use in HFL training. Figure 8.12.2-1: HFL training 1. An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. If the AIMLE server determines to use HFL training, the procedure continues to step 2. 2. The AIMLE server retrieves the indicated ML model using the ML model retrieval procedure as described in clause 8.1. 3. For the HFL training, the AIMLE server performs AIMLE client selection using the AIMLE client selection criteria or a list of AIMLE clients provided in step 1. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the HFL training. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the HFL training. 4. Based on the AIMLE client set determined in step 3, the AIMLE server checks with the selected AIMLE clients for their capability and participation in the HFL training as described in the clause 8.10. 5. The AIMLE server configures training schedule for each of the AIMLE client and sends a HFL training subscription request with information as described in Table 8.12.3.1-1. The subscription request is further enhanced to include following: Information element; Status; Description. AI/ML model information; M; Information about the AI/ML model (e.g. Model size, Model architecture, etc.). Current HFL training iteration; O; The number of aggregations that have been done during HFL training. Performance Tracker; O; The KPI records (i.e. performance metrics, such as accuracy, etc.) over different subscriptions rounds.. Editor's Note: Whether performance tracker IE is needed or not is FFS. The Current HFL training iteration informs the client about current training iteration ongoing. 6. Each AIMLE client sends a HFL training subscription response with information as described in Table 8.12.3.2-2. If the AIMLE client is not able to grant the subscription (e.g., is not able to perform the training), the AIMLE client sends a response with a failure status and the procedure skips to step 8. 7. Each AIMLE client performs local training using the configured AI/ML model, model parameters, and the prepared local data associated with the dataset identifier for the specified number of samples according to the operational schedule. 8. Upon completion or due to errors in the training, each AIMLE client sends a HFL training notification to the AIMLE server. The notification includes information as described in Table 8.12.3.1-3. The AIMLE client provides an VAL service ID for the HFL training operation. 9. If errors were encountered, the procedure skips to step 10. If training was successful, the AIMLE server aggregates (e.g. averages) the model parameters received from the AIMLE clients. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 8 are repeated for the next training round. When the training schedule has been exhausted (e.g. there are no remaining training round) and if configured, the AIMLE server makes a ML model information storage request as described in clause 8.11.2 to store the AI/ML model in the ML repository. 10. The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2.

### 7.6.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.6.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.6.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.7	Solution #7: Solution on discovering AIMLE clients in cross-PLMN scenarios

### 7.7.1	Solution description

This solution addresses key issue #3 open issues 1), 3), 4).

According to 3GPP TS 23.434, one of the business models is related to the SEAL provider having PLMN operator service arrangements with multiple home PLMN operators. Additionally, according to [AR-4.2.2-b], the SEAL shall support deployments in which SEAL services are deployed only outside of PLMN network.

This solution corresponds to the following use case:

-	An AI/ML service runs in a certain service area, which can be an edge or cloud service area or geographical area, where UEs connected to different PLMNs are present. Such service can be considered multi-operator AIMLE/VAL service. VAL UEs are assumed to have AIMLE clients installed and active and connect to AIMLE server via different PLMN. The AI/ML service can be an application enablement service (AIMLE client to server) where AIMLE-UU can be over multiple MNO networks.

-	In Figure 7.7.1-1, an example is shown where UE#1 is connected to PLMN#1 and UE#2 to PLMN#2. Both UEs are connected to the same AIMLE server. Different VAL server may connect to UE#1 VAL client and different VAL server to UE#2; however, both VAL servers are connected to the same AIMLE server.

-	One example of such scenario is that AI operation running for a V2X scenario, where VAL UEs of a traffic safety or optimization service (AI-enhanced) are needed and belong to different networks.

Figure 7.7.1-1: Use case for multi-operator AIMLE scenario

The solution provides a mechanism for discovering AI/ML participants (AIMLE clients, VAL clients) in an AIMLE-assisted operation (e.g. ML model training, FL, TL) in a given AIMLE service area. Discovery of AIMLE clients is an important step in the AI/ML process for distributed, federated, split AI/ML, and transfer learning. Due to the nature of such learning, VAL servers need to discover suitable AIMLE clients to fulfil the requirements for the AI/ML application. The VAL server can then use the discovered AIMLE clients to select a set of AIMLE clients to perform AI/ML operations.

In cross-PLMN scenario as in Figure 7.7.1-2, the following enhancements are provided (over AIMLE-S or SEAL-X interface if the AIMLE consumer is another SEAL server, e.g. ADAES).

8.8.2.1	AIMLE client discovery

Pre-conditions:

1.	AIMLE clients that support AI/ML operations have registered with the AIMLE server and included their AIMLE client profiles and optionally a list of supported services.

2.	The AIMLE server can access a ML repository to obtain AIMLE client profiles and supported services associated with AIMLE clients.

Figure 8.8.2.1-1: AIMLE client discovery

1.	A VAL server sends an AIMLE client discovery request to an AIMLE server to discover a list of AIMLE clients that are available to participate in AI/ML operations (e.g., is available and have required data to train an ML model). The request may also include AIML client task capability requirements to discover clients who can perform the AIML tasks like AIML model training/offload/split with compute requirements and task performance preference like green task performance. The AIMLE client discovery request includes information as described in Table 8.8.3.1-1.

This request also includes information on the serving and/or preferred PLMN for the communication of the VAL service, as well as the allowed MNO info from other MNO network from which the AIMLE consumer / VAL server can discover AIMLE clients / VAL UEs.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor is able to discover AIMLE clients for the VAL UEs connected to the serving PLMN. If the requestor is authorized, the AIMLE server performs the following to discover AIMLE clients fulfilling the provided AIMLE client discovery criteria.

The AIMLE server also obtains candidate UEs connected to the serving PLMN from the ML repository.

From the list of candidate UEs, the AIMLE server discovers a list of UEs (i.e., AIMLE clients) that fulfils the discovery criteria based on supported services and AIMLE client profiles.

The AIMLE server determines whether the discovered UEs fulfil the location information in the discovery criteria by using their identifiers to determine their location and includes in the discovery response only those UEs that meet the requirements. The AIMLE server may use SEAL-LMS (as in 3GPP TS 23.434 [5] clause 9.3.9, or 3GPP TS 23.434 [5] clause 9.3.10) or 3GPP 5G Core Network Services (such as GMLC as in 3GPP TS 23.273 [13] and NEF as in 3GPP TS 23.273 [13] or 3GPP TS 23.502 [9]) to determine the UEs which fulfil the location requirement.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML), then the AIMLE server may discover the remaining required AIMLE clients from other AIMLE servers over AIML-E reference point and include them in the response message.

3.	The AIMLE server sends an AIMLE client discovery response that includes information in Table 8.8.3.2-1.

If the required number of AIMLE clients are not included in the response message from the AIMLE server (i.e., the AIMLE clients in the response message are less than the required minimum number of AIMLE clients), then the AIML service consumer may discover the remaining required AIMLE clients from other AIML enablement server indicating the required number of AIMLE clients. The response may also include information related to supported tasks with guaranteed task KPIs, available AIML tasks service duration, AIML task expected KPI like latency.

If the available AIMLE clients (as determined by the AIMLE server) that fulfil discovery criteria are less than the required minimum number of AIMLE clients for AI/ML operation (e.g., split AI/ML) or the AIMLE clients are not available or capable for undertaking the operation, then the AIMLE server determines to discover and discovers the remaining required AIMLE clients connected to other PLMNs based on the allowed and/or preferred MNO information and the permissions as in step 1. The AIMLE server also obtains candidate UEs connected to the target/destination PLMN (other than the serving PLMN) from the ML repository.

Editor's Note: Enhancements on the information flows related to AIMLE client discovery procedure for cross-PLMN scenario is FFS

### 7.7.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.7.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.7.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.8	Solution #8: Multi-Client Split AI/ML Operations Pipeline

### 7.8.1	Solution Description

#### 7.8.1.1	General

This solution addresses key issues related to split AI/ML operations involving multiple AIMLE clients. The proposed solution enhances split AI/ML operations by facilitating multiple AIMLE client collaboration within the same pipeline. It allows the creation of multi-client AI/ML split operation pipeline when such a pipeline is not yet created and allows joining the pipeline when such a pipeline has already been created. The solution also keeps AIMLE clients in sync with the latest status of the pipeline through notifications as more AIMLE clients join and the pipeline is adjusted according to the resources of the newly joining AIMLE clients. The solution ensures efficient resource utilization and continuous service, even with mobility and varying device constraints.

Editor's Note: The feasibility of sharing a pipeline among AIMLE clients is FFS

Use Cases:

-	Enhanced Smart City Surveillance with Mobility: Surveillance cameras, both stationary and mobile, capture video footage and process raw data on the cameras, sending subsequent layers to an edge server for immediate processing. AIML split operations enable real-time improved surveillance and threat detection for public safety through a shared pipeline that allows collaborative data processing. The pipeline needs to be re-adjusted based on computing resource powers for the surveillance cameras. Pipeline re-adjustment is also needed as the mobile surveillance cameras change service areas. This results in efficient resource utilization, energy saving, and larger surveillance.

-	Collaborative Autonomous Driving: A group of autonomous vehicles collaborate by sharing and processing data together. A more concreate example is vehicles utilizing shared Local Dynamic Maps (LDM) which is collaboratively updated and used by each vehicle locally. The raw data of each vehicle is processed locally and it is sent for subsequent layers in the edge server for immediate processing. The shared AIML split operation enable real-time collaboration to optimize driving conditions (e.g., efficient routes) and reduces computational burden in the vehicles. Pipeline re-adjustment is needed based on the computing resource power of the participants. Pipeline-readjustment is also needed as the vehicles change service area during mobility. This results in a larger collaboration coverage.

#### 7.8.1.2	Multi-client pipeline operation

Figure 7.8.1.2-1: Multi-client AI/ML split operation

The solution involves a multi-client split operations pipeline facilitated by AIMLE clients and servers. Initially, an AIMLE client initiates a discovery request to the AIMLE server to participate in a split operation pipeline, as illustrated in Block A of the figure. This is similar to clause 8.14.2.2 in 3GPP TS 23.482 split operation pipeline discovery. A key addition for this solution is an expansion of the split operation discovery filters in clause 8.14.3.2-2 with additional criteria for multi-client split operations within the pipeline. Following the discovery request, the split operation profile in the discovery response, as in clause 8.14.3.3 Table 8.14.3.3-2, is also expanded to include information about a pipeline with multi-client split operation pipeline support, the usage of the multi-client split operation pipeline (in usage information), and whether such a pipeline has already been created or not.

If in the discovery stage, as in Block B, the discovered instance of the split operation pipeline is a pipeline that has not yet been created, the AIMLE client will then request the creation of a new pipeline, explicitly mentioning the option for multi-client operations in the split operation profile, as shown in Block B, and subscribes for the split operation event. This is similar to clause 8.14.2.3 in 3GPP TS 23.482 and clause 8.14.2.5 in 3GPP TS 23.482, with additional expansion of clause 8.14.3.4-1 in 3GPP TS 23.482 to include multi-client support in the pipeline, expansion of the usage information to include further details on the multi-client pipeline, and also the computing resources of the AIMLE client to be able to efficiently adjust the pipeline later on as more AIMLE clients join the pipeline. Furthermore, instead of 3GPP TS 23.482, which has two separate procedures for pipeline creation and subscription, this solution proposes a split operation pipeline create subscription, which will allow creation and subscription together. Although this is a useful procedure for a single AIMLE client case as well, it becomes more important for split AI/ML operations involving multiple AIMLE clients. This is because as other AIMLE clients join or leave and the pipeline is modified, notifications need to be sent to the AIMLE clients so that proper client-side adjustments are made as needed.

Subsequently, once a multi-client AI/ML split operation pipeline has been created, other AIMLE clients who seek to also engage in the multi-client AI/ML split operation pipeline begin by initiating a discovery procedure as in Block C, which is similar to Block A. If a pipeline is already created according to Block B, it identifies the existing created pipeline that supports multi-client AI/ML split operation, fulfilling the respective split operation discovery filters. Therefore, instead of a split operation pipeline create subscription, it will proceed with a split operation pipeline join subscription, which will allow the AIMLE client to join the created pipeline as in Block D. To enable adjustment of the pipeline considering the newly joining AIMLE clients, each client indicates its respective computing resources during the join. This process can be scaled up to accommodate an n-number of AIMLE clients, allowing for extensive multi-client collaboration in split AI/ML operations.

As AIMLE clients with different computing resources join the split operation pipeline, the pipeline may need to be adjusted, and the participating AIMLE clients need to be notified of changes to the pipeline so that they can make client-side adjustments. For this purpose, as shown in Block E, a split AI/ML operation notification is sent to all AIMLE clients as per clause 8.14.2.5.3 of 3GPP TS 23.482.

#### 7.8.1.3	Multi-client pipeline service continuity

A multi-client AI/ML split operation pipeline should be resilient to the mobility of the AIMLE clients. When edges are involved in the split operation pipeline, not only should the pipeline be adjusted, but AIMLE clients should also be enabled to collaborate in the pipeline even if they are in different service areas. This will enable better service continuity support. When an AIMLE client leaves a pipeline, the pipeline also needs to be adjusted to stay optimal.

This solution achieves service continuity in two ways:

1.	First, the pipeline is re-adjusted according to the computing resources of the AIMLE clients to obtain matching layers in the pipeline. For example, when AIMLE clients with low computing resources join, the pipeline is adjusted so that more parts of the pipeline are offloaded to the nodes. Conversely, when an AIMLE client with low resources leaves, the pipeline can be re-adjusted so that more parts of the pipeline can be handled by the remaining AIMLE clients.

2.	Second, when AIMLE clients change service areas, the pipeline can still be adjusted so that the common layers in the pipeline remain matched. When an AIMLE client moves to a different service area, adjustments are made based on the available AIMLE clients in the current service area. When the AIMLE client moves back to the original service area, re-adjustment is performed to keep the common layers in the pipeline matched. For example, when an AIMLE client with low computing resources moves to a different service area, part of the pipeline in the current service area can be relaxed for the other AIMLE clients so that more of the pipeline can be on the client side. The edges in the service area of the low-resource AIMLE clients can be part of the pipeline in line with the computing resources of the AIMLE clients in the service area to which it moved. The source and target AIMLE servers can continue to communicate as in step 1 of 8.24.2 TS 23.482.

#### 7.8.1.4	Procedures to support Multi-client AI/ML split operation

##### 7.8.1.4.1	Split operation pipeline join

Figure 7.8.1.4.1-1 illustrates the procedure for one or more AIMLE clients to join to a split operation pipeline with the AIMLE server.

Pre-conditions:

- The AIMLE client has performed split operation pipeline discovery with the AIMLE Server and a multi-client split operation pipeline has been found.

Figure 7.8.1.4.2-1: Split operation pipeline join

1.	The AIMLE client sends a request to join an existing split operation pipeline. To enable dynamic adjustment of the pipeline based on the capabilities of newly joining clients, each AIMLE client includes its computing resource information in the request.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request.

3.	The AIMLE server responds with either a positive acknowledgment or a negative acknowledgment with a cause of failure (e.g., failure to authorize).

##### 7.8.1.4.2	Split operation notification

Figure 7.8.1.4.2-2 illustrates the procedure for an AIMLE server to notify all AIMLE clients subscribed as per clause 8.14.2.5.3 of 3GPP TS 23.482.

Pre-conditions:

- The AIMLE clients have joined a pipeline.

- The AIMLE clients have subscribed to pipeline event as per step 1.a. of clause 8.14.2.5.2 of 3GPP TS 23.482.

Figure 7.8.1.4.2-1: Split operation notification

- When an AIMLE client joins the split operation pipeline, the AIMLE server evaluates the updated pipeline configuration and determines whether adjustments are needed to maintain optimal performance and coordination. This also triggers, as a result, a pipeline update event as per step 1 of clause 8.14.2.5.3 of 3GPP TS 23.482.

- The AIMLE server sends the split operation notification to all subscribed AIMLE clients as per step 2 of clause 8.14.2.5.3 of 3GPP TS 23.482.

### 7.8.2	Architecture Impacts

No architectural impact has been identified.

### 7.8.3	Corresponding APIs

#### Split operation pipeline discovery request

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

- 3GPP TS 23.482 Clause 8.14.3.2	Split operation pipeline discovery request

3GPP TS 23.482 Table 8.14.3.2-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline discovery request.

3GPP TS 23.482 Table 8.14.3.2-1: Split operation pipeline discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier) |
| Split operation discovery filters | M | Split operation discovery filters. |
| > multi-client split operation pipeline support | O | Indicates if multi-client AIMLE split operation pipeline support is required or not. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.) |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.) |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.) |
| > number of nodes | O | Minimum number of nodes required to support AIML operation splitting |

#### Split operation pipeline discovery response

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

- 3GPP TS 23.482 Clause 8.14.3.3	Split operation pipeline discovery response

3GPP TS 23.482 Table 8.14.3.3-1 shows the response sent by sent by the AIMLE server after processing of split operation pipeline discovery request.

3GPP TS 23.482 Table 8.14.3.3-1: Split operation pipeline discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE 2) | Indicates that the request was successful. |
| > List of nodes (NOTE 1) | O | The list of discovered nodes. |
| > List of split operation profiles (NOTE 1) | O | The list of split operation profiles as specified in Table 8.14.3.3-2 of 3GPP TS 23.482. |
| Failure response | O (NOTE 2) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE 1: Only one of the IE shall be present for successful response. NOTE 2: One of the IEs shall be present. |  |  |

3GPP TS 23.482 Table 8.14.3.3-2: Split operation profile

| Information element | Status | Description |
| --- | --- | --- |
| Split operation pipeline identifier | M | The identifier of the split operation pipeline. |
| Head endpoint | M | The endpoint information of the head node for providing intermediate data. |
| Tail endpoint | M | The endpoint information of the tail node for obtaining results. |
| Multi-client split operation pipeline support | O | Indicates if multi-client split operation pipeline is supported or not. |
| Usage information | O | The usage information of the AI/ML split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| Stage information | M | The list of stage(s) of the AI/ML split operation, the nested information is provided for each stage. |
| > Stage identifier | M | The identifier of the stage. |
| > number of nodes | O | Number of nodes included in the stage |
| > head node | M | Endpoint of the head node - for providing initial the inference data. |
| > tail node | M | Endpoint of the head node - for obtaining inference results. |
| > order of the nodes | M | The order of the nodes in the AI/ML split operation (including head node, tail node). |
| >> list of nodes | M | List of all discovered node in the order in which they process the data. |
| >> notification target | O | Endpoint information where the inference result of the split operation is sent by the nodes |
| > Model information | M | The ML model information used in the stage (e.g., identifiers, versions, etc.). |

#### 7.8.3.3	Split operation pipeline create request

According to clause 7.8.1.2, the following change is needed in 3GPP TS 23.482.

- 3GPP TS 23.482 Clause 8.14.3.4	Split operation pipeline create request

Table 8.14.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline create request.

3GPP TS 23.482 Table 8.14.3.4-1: Split operation pipeline create request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL client ID, AIMLE client ID, UE identifier). |
| Computing resource information | O | The computing resource information of the requestor. |
| Split operation requirements | M | Split operation requirements. |
| > model information | M | Information about the ML models to be used in each stage (e.g., identifiers, versions, etc.). |
| > Multi-client split operation pipeline support | O | Indicates if multi-client split operation pipeline support is required or not. |
| > usage information | O | Information about the planned usage of the split operation (e.g., inputs frequency/size, output frequency/size, etc.). |
| > notification target | O | Endpoint information where the result of the split operation is sent by the tail node. |
| > stage information | M | Information about the split operation stages (e.g., number, order, etc.). |
| >> head node | M | Endpoint of the head node - for providing initial the inference data. |
| >> tail node | M | Endpoint of the head node - for obtaining inference results. |
| >> node information | M | List of Information about the nodes in order of the stage (e.g., identifier, endpoint, etc.). |

#### 7.8.3.4	Split operation pipeline join request

Table 7.8.3.4-1 shows the request sent by a AIMLE client to the AIMLE server for split operation pipeline join request.

Table 7.8.3.4-1: Split operation pipeline join request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identity of the requestor (e.g., VAL server ID). |
| Split operation pipeline identifier | M | Identifier of the split operation pipeline for which the requestor is joining. |
| Computing resource information | O | The computing resource information of the requestor. |

#### 7.8.3.5	Split operation pipeline join response

Table 7.8.3.5-1 shows the response sent by an AIMLE server after processing of split operation pipeline join request.

Table 7.8.3.5-1: Split operation pipeline join response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| Failure response | O (NOTE) | Indicates that the request failed. |
| > Failure cause | O | Indicates the failure cause. |
| NOTE: One of the IEs shall be present. |  |  |

### 7.8.4	Solution Evaluation

Editor's Note: The evaluation of the solution is FFS.

## 7.9	Solution #9: AIMLE server registration in a hierarchical AIMLE deployment

### 7.9.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes AIMLE server registration procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 as shown on Figure 7.9.1-1. This solution makes no assumptions on the cardinality of ML repository(ies) deployed at the EDN or at the centralized DN.

Figure 7.9.1-1 Example hierarchical AIMLE deployment

The registration of an edge AIMLE server with a central AIMLE server enables the central AIMLE server to be aware of the edge AIMLE server capabilities and of the clients registered with the edge AIMLE server.

A central AIMLE server needs information about AIMLE clients available in different EDN service areas when performing operations requiring the selection of AIMLE clients. The AIMLE clients in an EDN service area may register to the edge AIMLE server, therefore, registration of the edge AIMLE server with the central AIMLE server are needed to support hierarchical AIMLE deployment scenarios.

#### 7.9.1.1	AIMLE server registration

Figure 7.9.1.1-1 describes a procedure for an AIMLE server to register with another AIMLE server. In such a scenario, an AIMLE server deployed as EAS, which is located at an EDN registers to a central AIMLE server.

Pre-conditions:

1.	AIMLE clients have registered with an edge deployed AIMLE server (i.e., an AIMLE server deployed as EAS which is located at an EDN) as described in clause 8.7.2.2 of 3GPP TS 23.482. The edge deployed AIMLE server receives AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE server has been pre-configured with endpoint information of a central AIMLE server.

Figure 7.9.1.1-1: AIMLE server registration procedure

1.	The edge deployed AIMLE server sends a AIMLE server registration request to a central AIMLE server. The registration request includes the requestor identifier, endpoint information and a list of AIMLE clients registered with the AIMLE server. The request may include EDN information and AIMLE server capabilities.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server stores the registration information and creates a registration identifier. The central AIMLE server sends a AIMLE server registration response to the requestor. If the central AIMLE server has registered the requestor, the response includes an indication of success, a registration identifier and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure.

#### 7.9.1.2	AIMLE server registration update

Pre-conditions:

1.	The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.2-1: AIMLE server registration update procedure

1.	The edge deployed AIMLE server sends a AIMLE server registration update request to the central AIMLE server. The request includes the registration identifier, and may include updated AIMLE server information as described in Table 7.9.3.3-1. If the request is to renew the registration, only the registration identifier is included in the request.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server updates the information associated with the registration identifier and may update the registration expiration time if the request only includes the registration identifier. The central AIMLE server sends a AIMLE server registration update response to the requestor. If the central AIMLE server has updates the registration, the response includes an indication of success and may include an expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure.

#### 7.9.1.3	AIMLE server de-registration

Pre-conditions:

1.	The edge deployed AIMLE server has registered with the central AIMLE Server.

Figure 7.9.1.3-1: AIMLE server de-registration procedure

1.	The edge deployed AIMLE server sends a AIMLE server de-registration request to the central AIMLE server. The de-registration request includes the registration identifier.

2.	Upon receiving the request, the central AIMLE server validates if the requestor is authorized for the request. If the request is authorized, the central AIMLE server terminates the registration indicated in the request. The central AIMLE server sends a AIMLE server de-registration response to the requestor. If the central AIMLE server has de-registered the requestor, the response includes an indication of success. If the requestor was not de-registered, the response includes an indication of failure and may include a reason for failure.

### 7.9.2	Architecture impacts

No architectural impact has been identified.

### 7.9.3	Corresponding APIs

#### 7.9.3.1	AIMLE server registration request

Table 7.9.3.1-1 shows the request sent by an edge deployed AIMLE server (i.e., an AIMLE server deployed as EAS which is located at an EDN) to a central AIMLE server for registering AIMLE server and clients in the EDN service area.

Table 7.9.3.1-1: AIMLE server registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the registering AIMLE server. |
| AIMLE server endpoint | M | The endpoint information (e.g., URL, URI, IP address, port) of the registering AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registering AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O (NOTE 1) | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O (NOTE 1) | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registering AIMLE server. |
| EAS information | O (NOTE 2) | Indicates information of the AIMLE server deployed as EAS in the EDN, which may include EAS ID, DNAIs. |
| List of registered AIMLE clients | M | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registering AIMLE server. |
| NOTE 1: At least one of the information elements is present. NOTE 2: The information element shall be present for edge deployed AIMLE server. |  |  |

#### 7.9.3.2	AIMLE server registration response

Table 7.9.3.2-1 shows the response sent by the central AIMLE server to the edge deployed AIMLE server in response to the registration request.

Table 7.9.3.2-1: AIMLE server registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration was successful. |
| > Registration ID | M | The identifier of the registration. |
| > Expiration time | O | Indicates the expiration time of the registration. If the Expiration time IE is not included, it indicates that the registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration failed. |
| > Cause | O | Indicates the cause of the failure |
| NOTE: One of the information elements must be present. |  |  |

#### 7.9.3.3	AIMLE server registration update request

Table 7.9.3.3-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server registration update request.

Table 7.9.3.3-1: AIMLE server registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of an existing registration. |
| AIMLE server endpoint | O | The endpoint information (e.g., URL, URI, IP address, port) of the registered AIMLE server. |
| AIMLE server capabilities | O | The capability information of the registered AIMLE server (e.g. ML application type, allowed resource usage level). |
| EDN service area identifier | O | The identifier of the EDN service area associated with the registered AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the registered AIMLE server. |
| EAS information | O | The information of the AIMLE server deployed as EAS in the EDN, which may include EAS ID, DNAIs. |
| List of registered AIMLE clients | O | The list of registered AIMLE client profiles, as described in 3GPP TS 23.482 Table 8.7.3.2-2, that are registered with the registered AIMLE server. |

#### 7.9.3.4	AIMLE server registration update response

Table 7.9.3.4-1 shows the AIMLE server registration update response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.9.3.4-1: AIMLE server registration update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the registration update was successful. |
| > Expiration time | O | Indicates the expiration time of the updated registration. If the Expiration time IE is not included, it indicates that the updated registration never expires. |
| Failure response | O (NOTE) | Indicates that the registration update failed. |
| > Cause | O | Indicates the cause the failure. |
| NOTE: One of the information elements must be present. |  |  |

#### 7.9.3.5	AIMLE server de-registration request

Table 7.9.3.5-1 shows the request sent by an edge deployed AIMLE server to a central AIMLE server for the AIMLE server de-registration request.

Table 7.9.3.5-1: AIMLE server de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration identifier | M | The identifier of the existing registration. |

#### 7.9.3.6	AIMLE server de-registration response

Table 7.9.3.6-1 shows the AIMLE server de-registration response sent by the central AIMLE server to the edge deployed AIMLE server.

Table 7.9.3.6-1: AIMLE server de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the de-registration was successful. |
| Failure response | O (NOTE) | Indicates that the de-registration failed. |
| > Cause | O | Indicates the cause of the failure. |
| NOTE: One of the information elements must be present. |  |  |

### 7.9.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

## 7.10	Solution #10: Support of AIMLE services for roaming UEs

### 7.10.1	Solution description

#### 7.10.1.1	Introduction

This solution proposes enhancements to address Key Issue #3 – Open Issue #2 on support of AIMLE services for roaming UEs.

This solution proposes enhancing AIMLE client registration in roaming scenarios by enabling the H-AIMLE server to indicate to a registering AIMLE client whether it is allowed to use AIMLE services while in a visited network.

The solution assumes the following roaming models:

- Home Routed (HR) roaming architecture

- Local Breakout (LBO) roaming architecture

In the HR roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE server where the UE obtains AIMLE services from the home network.

In the LBO roaming architecture, the traffic of the AIMLE client is routed to the H-AIMLE Server or to the V-AIMLE Server dependent on the AIMLE server endpoint used by the AIMLE client. The AIMLE client is provided with V-AIMLE server information when registering with the H-AIMLE server as described in this solution.

#### 7.10.1.2 	Impact to existing AIMLE client registration

The AIMLE client registration procedures and information flows in 3GPP TS 23.482 are enhanced (highlighted in bold italics) as follows.

# 2 References The following documents contain provisions which, through reference in this text, constitute provisions of the present document. - References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific. - For a specific reference, subsequent revisions do not apply. - For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document. [1] 3GPP TR 21.905: "Vocabulary for 3GPP Specifications". [2] 3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services". [3] 3GPP TS 28.104: "Management and orchestration; Management Data Analytics". [4] 3GPP TS 23.436: "Functional architecture and information flows for Application Data Analytics Enablement Service". [5] 3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows". [6] 3GPP TS 23.501: "System Architecture for the 5G System; Stage 2". [7] 3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access". [8] 3GPP TS 28.105: "Management and orchestration; Artificial Intelligence/ Machine Learning (AI/ML) management". [9] 3GPP TS 23.502: "Procedures for the 5G System (5GS)". [10] 3GPP TS 22.261: "Service requirements for the 5G system". [11] 3GPP TS 26.531: "Data Collection and Reporting; General Description and Architecture". [12] 3GPP TS 23.558: "Architecture for enabling Edge Applications". [13] 3GPP TS 23.273:"5G System (5GS) Location Services (LCS); Stage 2". [14] 3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals". [15] 3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

## 8.7 AIMLE client registration ### 8.7.1 General Prior to participation in AI/ML operations, AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When a UE is roaming and the roaming architecture is home routed (HR) (e.g., based on roaming agreement), the UE registers with an AIMLE server in the home network and AIMLE services provided by the visited network are not used. When a UE is roaming and the roaming architecture is local breakout (LBO) (e.g., based on roaming agreement), the UE first registers with an AIMLE server in the home network, then the UE may perform an AIMLE server registration with an AIMLE server in the visited network if AIMLE services usage is allowed in the visited network. NOTE: Usage of AIMLE service provided by a visited network is dependent on a roaming agreement. The following clauses specify procedures, information flows, and APIs for AIMLE client registration. ### 8.7.2 Procedures #### 8.7.2.1 General The following are supported for AIMLE client registration: - AIMLE client registration procedure; - AIMLE client registration update procedure; and - AIMLE client de-registration procedure #### 8.7.2.2 AIMLE client registration Pre-conditions: 1. The AIMLE client has been pre-configured or has discovered the address (e.g., URI) of the AIMLE server. 2. The AIMLE client has been pre-configured with an AIMLE client profile. Figure 8.7.2.2-1: AIMLE client registration 1. The AIMLE client sends an AIMLE client registration request to the AIMLE server, the registration request includes information as described in Table 8.7.3.2-1. The AIMLE client indicates in the registration request its AI/ML capabilities such as supported ML model types and supported AI/ML operations, supported AIMLE client task capabilities with compute and task performance capabilities to assist with performing AIMLE client discovery and AIMLE client selection. If the UE is roaming, the registration request includes the serving PLMN identifier. 2. The AIMLE server validates the registration request and performs an authentication and authorization check to determine if the AIMLE client is permitted to register to the AIMLE server and participate in AI/ML operations. Upon successful authorization, the AIMLE server saves the context of the AIMLE client registration in the ML repository. When the PLMN identifier is provided, the AIMLE server may verify the serving PLMN information from the AIMLE client, using the 3GPP core network services (for example, NEF monitoring event API as described in 3GPP TS 23.502 [9] and 3GPP TS 23.682 [15]), to obtain the UE roaming status and serving PLMN identifier. Further verify and confirm the serving PLMN information of the AIMLE client. If the UE is roaming, the AIMLE server determines if the registering AIMLE client is allowed to consume AIMLE services for the PLMN identifier and determines allowed AIMLE service operations. 3. The AIMLE server returns an AIMLE client registration response to the AIMLE client with the status of the request. If the AIMLE server has determined that the UE is allowed to consume AIMLE services with the PLMN identifier, the response includes the V-AIMLE server connectivity information and a list of allowed AIML service operations. If the UE is roaming and the response includes V-AIMLE server connectivity information, the AIMLE client registers with the V-AIMLE server. If the AIMLE client has successfully registered with the V-AIMLE server, the AIMLE client sends a registration update to the AIMLE server in the home network to indicate that it is consuming AIMLE services in the visited network. #### 8.7.2.3 AIMLE client registration update Pre-conditions: 1. The AIMLE client has already registered with the AIMLE Server. Figure 8.7.2.3-1: AIMLE client registration update 1. The AIMLE client sends an AIMLE client registration update request to the AIMLE server, the registration update shall include registration identifier, updated list of supported services and updated AIMLE client profile as described in Table 8.7.3.4-1. The AIML client removes services from the list of services identified by Service ID(s) when it ceases to provide a service and adds to the service when it starts to provide a service. If the UE roaming network has changed, the registration request includes the updated serving PLMN identifier. 2. The AIMLE server validates the registration update and performs an authentication and authorization check. Upon successful authorization, the AIMLE server updates the context of the AIMLE client registration in the ML repository. When the PLMN identifier is updated, the AIMLE server may verify the serving PLMN information from the AIMLE client, using the 3GPP core network services (for example, NEF monitoring event API as described in 3GPP TS 23.502 [9] and 3GPP TS 23.682 [15]), to obtain the UE roaming status and serving PLMN identifier. Further verify and confirm the serving PLMN information of the AIMLE client. If the UE roaming network has changed, the AIMLE server determines if the AIMLE client is allowed to consume AIMLE services for the updated PLMN identifier and determines allowed AIMLE service operations. 3. The AIMLE server returns an AIMLE client registration update response to the AIMLE client with the status of the update. If the AIMLE server has determined that the UE is allowed to consume AIMLE services with the updated PLMN identifier, the response includes the V-AIMLE server connectivity information and a list of allowed operations with the V-AIMLE server. If the UE is roaming and the response includes V-AIMLE server connectivity information, the AIMLE client registers with the V-AIMLE server. If the AIMLE client has successfully registered with the V-AIMLE server, the AIMLE client sends a registration update to the AIMLE server in the home network to indicate that it is consuming AIMLE services in the updated visited network. #### 8.7.2.4 AIMLE client de-registration Pre-conditions: 1. The AIMLE client has already registered with the AIMLE Server. Figure 8.7.2.4-1: AIMLE client de-registration 1. The AIMLE client sends an AIMLE client de-registration request to the AIMLE server. The request shall include registration identifier as described in Table 8.7.3.6-1. 2. The AIMLE server validates the deregistration request and performs an authentication and authorization check to determine whether the AIMLE client can deregister from AI/ML operations. Upon successful authorization, the AIMLE server removes the context of the AIMLE client registration from the ML repository. 3. The AIMLE server returns an AIMLE client de-registration response to the AIMLE client with the status of the request. If the UE is roaming and has previously registered with a V-AIMLE server, the AIMLE client de-registers with the V-AIMLE server. ### 8.7.3 Information flows #### 8.7.3.1 General The following information flows are specified for AIMLE client registration: - AIMLE client registration request and response; - AIMLE client registration update request and response; and - AIMLE client de-registration request and response #### 8.7.3.2 AIMLE client registration request Table 8.7.3.2-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration request. Table 8.7.3.2-1: AIMLE client registration request Information element; Status; Description. Requestor identifier; M; The identifier of the requestor.. List of supported profiles; M; Supported AIML client profile(s). For each client profile provided in the list, the supported service information is provided.. > AIMLE client profile; M; Information about the capability of the AIMLE client to support AI/ML operations for the VAL service ID as detailed in Table 8.7.3.2-2.. > List of supported services; M; List of VAL service IDs with corresponding permissions.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > PLMN Identifier; O; The identifier of the PLMN where the UE is currently registered.. Table 8.7.3.2-2: AIMLE client profile Information element; Status; Description. Supported AI/ML model types; O; AI/ML model types supported by the AIMLE client (e.g., decision trees, linear regression, neural networks).. Supported AI/ML operations; M; AI/ML operations supported by the AIMLE client such as training, model transfer, model inference, model offload, model split, continue perform intermediate AI/ML operation/task).. AIMLE client time schedule configurations; O; Indicates the availability schedule of the AIMLE client for the AIML service, e.g., the AIMLE client is (not) available to participate in the AIML operations (e.g. ML model training) in the given time slot(s) and/or day(s) of the week.. AIMLE client location configurations; O; Indicates the location-based configurations of the AIMLE client for the AIML service, e.g., the AI/ML member is (not) available to participate in the AI/ML operations in the given locations represented by coordinates, civic addresses, network areas, or VAL service area ID.. AIMLE client capabilities; M; AIMLE client capability information (e.g. ML application type, allowed resource usage level, supported roaming and roaming service operations).. Dataset availability; O; Dataset availability such as dataset size, age, list of dataset features, and dataset identifiers.. Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g. raw data), supported data processing capabilities (e.g. processed data), and supported exploratory data analysis functions.. AIMLE client task capability; O See NOTE; Indicates the AIML task performing capabilities. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g., Green task, energy-efficient, low costs). NOTE: The Green and Energy-efficient task performance may not be applicable to a UE.. #### 8.7.3.3 AIMLE client registration response Table 8.7.3.3-1 shows the AIMLE client registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.3-1: AIMLE client registration response Information element; Status; Description. Successful response; O (NOTE); Indicates that the registration was successful.. > Registration ID; M; The identifier of the registration.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. > Roaming configuration; O; The configuration for using AIMLE services in the visited network. >> Roaming connectivity information; O; The connectivity information to a V-AIMLE server.. >> Allowed roaming operations; O; The list of allowed operations with the V-AIMLE server. Failure response; O (NOTE); Indicates that the registration request failed.. > Cause; O; Indicates the cause of registration request failure. NOTE: One of the IEs shall be present.. #### 8.7.3.4 AIMLE client registration update request Table 8.7.3.4-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client registration update request. Table 8.7.3.4-1: AIMLE client registration update request Information element; Status; Description. Registration identifier; M; Identifier of the existing registration for which the update request applies.. AIMLE client profile; O; Update information about the capability of the AIMLE client to support AI/ML operations as detailed in Table 8.7.3.2-2.. List of supported services; O; Update to supported service information. Services identified by their VAL Service ID are either removed from the list when the AIMLE client ceases to provide a service and added when the AIMLE client starts to provide a service.. > VAL service ID; M; The identifier of the VAL service.. > Service permission level; O; Service permission level (e.g., premium resource usage, standard resource usage, limited resource usage).. > PLMN Identifier; O; The identifier of the PLMN where the UE is currently registered.. > Roaming AIMLE service indication; O; The indication that the AIMLE client is registered with the AIMLE server in the visited network (e.g., the V-AIMLE server indicated by the H-AIMLE server).. #### 8.7.3.5 AIMLE client registration update response Table 8.7.3.5-1 shows the AIMLE client de-registration update response sent by the AIMLE server to the AIMLE client. Table 8.7.3.5-1: AIMLE client registration update response Information element; Status; Description. Successful response; O (NOTE); Indicates that the registration update request was successful.. > Expiration time; O; Indicates the expiration time of the updated registration. To maintain an active registration status, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the updated registration never expires.. > Roaming configuration; O; The configuration for using AIMLE services in the visited network. >> Roaming connectivity information; O; The connectivity information to a V-AIMLE server.. >> Allowed roaming operations; O; The list of allowed operations with the V-AIMLE server. Failure response; O (NOTE); Indicates that the registration update request failed.. > Cause; O; Indicates the cause of registration update request failure. NOTE: One of the IEs shall be present.. #### 8.7.3.6 AIMLE client de-registration request Table 8.7.3.6-1 shows the request sent by an AIMLE client to an AIMLE server for the AIMLE client de-registration request. Table 8.7.3.6-1: AIMLE client de-registration request Information element; Status; Description. Registration ID; M; Identifier of the existing registration for which the update request applies.. #### 8.7.3.7 AIMLE client de-registration response Table 8.7.3.7-1 shows the AIMLE client de-registration response sent by the AIMLE server to the AIMLE client. Table 8.7.3.7-1: AIMLE client de-registration response Information element; Status; Description. Successful response; O (NOTE); Indicates that the de-registration request was successful.. Failure response; O (NOTE); Indicates that the de-registration request failed.. > Cause; O; Indicates the cause of de-registration request failure. NOTE: One of the IEs shall be present..

### 7.10.2	Architecture Impacts

No architecture impact has been identified.

### 7.10.3	Corresponding APIs

API updates have been described in clause 7.10.1.2.

### 7.10.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

## 7.11	Solution #11: Solution of FL member registration in multi-operator scenarios

### 7.11.1	Solution description

This solution addresses Key Issue #3.

The solution provides a mechanism for supporting registration for candidate FL member (e.g. FL clients) which are application and/or application enablement layer entities (AIMLE server, VAL server, AIMLE clients, VAL clients), where the candidate FL members aim to operate in a service area which is covered by multiple networks operated by different operators, and the candidate FL members are connected to more than one network (either some members are connected to PLMN1 and some to PLMN2 or both).

In cross-PLMN scenario, for registering the FL members using the multi-operator information, the following enhancements are provided in 3GPP TS 23.482 clause 8.4.2 (for registration) and clause 8.4.3 (for registration update).

8.4.2	Procedure on FL member registration

Figure 8.4.2-1 illustrates the procedure where the registration of a candidate FL member happens via the ML repository, serving as AIML service registry.

Figure 8.4.2-1: Procedure for registration on FL member registry

1.	The candidate FL member (e.g., VAL server via AIMLE server or AIMLE server) sends an FL member registration request to the ML repository for registering to the ML repository which acts as the AIML service registry.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

NOTE: Such allowed MNO info is applicable to scenarios when the candidate FL member has limitations being grouped with other FL members being connected to other PLMNs. Such limitations are imposed based on the vendor providing the service in cross-PLMN scenarios, e.g. based on an interoperability indication. If the vendor is the same is the same across FL member connected to different PLMNs, then it may be possible that the FL members of different PLMNs collaborate e.g. in cases like VFL.

2.	The ML repository validates the received request and generates the identity and other security related information for all the FL members listed in the registration request. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving or allowed PLMN.

3.	The ML repository sends the generated information in the FL member registration response message to the candidate FL member.

The procedure for AIMLE client to register to ML repository as FL member is introduced in clause 8.7.2.2.

8.4.3	Procedure on FL member registration update

Figure 8.4.3-1 illustrates the procedure where the registration update of a candidate FL member happens via the ML repository.

Figure 8.4.3-1: Procedure for registration update

1.	The candidate FL member (VAL server via AIMLE server or AIMLE Server) sends an FL member registration update request to the ML repository acting as AIML service registry for updating the registration of the FL member.

In case of multi-operator support, this request also includes information on the serving PLMN for the communication of the FL member (e.g. towards other FL members and/or the network), as well as the allowed MNO info including all MNO networks from which the candidate FL member can cope with if selected in a potential FL operation.

2.	The ML repository validates the received update request and generates the identity and other security related information for the FL member. The ML repository stores the identity of the candidate FL member together with a mapping to one or more PLMNs, and with associated role as serving, or allowed PLMN.  In case of registration update, the mapping to the one or more PLMNs and/or the roles may be updated accordingly.

3.	The ML repository sends the generated information in the FL member registration update response message to the candidate FL member.

The procedure for AIMLE client to update its registration to ML repository as FL member is introduced in clause 8.7.2.3.

### 7.11.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.11.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.11.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.12	Solution #12: Support for ML model split learning using relays

### 7.12.1	Solution description

This solution addresses Key Issue #4. The solution provides a mechanism for identifying and determining an intermediate entity (UE, edge function) to serve as relay for a split learning operation, and particularly a split AI/ML operation.

Such intermediate entity can be an application layer relay UE which either doesn’t provide any processing and is not aware of the data to relay (encrypted), or 2) intermediate node which undertakes some layers of the split operations (e.g. layers 5-15) instead of the source VAL UE. For both cases, the intermediate entity is expected to be associated with the split operation pipeline as processing entity or just relaying entity.

Pre-conditions:

- A split AIML pipeline is created, and the UE and VAL server has an ongoing connection session for undertaking jointly the split learning operation.

- One or more VAL UEs or edge nodes (e.g. EAS) have registered their capabilities in the ML repository / AIMLE server.

Editor's Note: It is FFS how one or more VAL UEs register their capabilities as candidate intermediate nodes for the split learning operation.

Figure 7.12.1-1: Support for split relay operation relaying

1.	A VAL UE detects that a ML model performance is degraded or expected to degrade. The detection can have different flavours either based on UE mobility and channel conditions or due to load/energy status and degradation of the ongoing ML task. More specifically, the detections can be due to:

- VAL client notifies the AIMLE client that the task is delaying or expected to delay.

- AIMLE client utilizes ADAE analytics on VAL session performance (based on TS 23.436) and predicts the downgrade of the link between the VAL UE and the VAL server

- High load or energy consumption at the VAL UE side, due to high app load or lower layer processing

- UE mobility prediction to an area with no coverage or poor coverage or bad channel conditions.

2.	The AIMLE client determines a requirement for updating the pipeline to include a relay/intermediate node and sends a discovery request to the AIMLE server.

The discovery request includes discovery criteria for the selection of the relay, including capability of the VAL UE (apps onboard, workloads supported, processing power and capacity, energy consumption limitations/restrictions), the status of the radio capability of the UE (RATs/spectrum supported, channel conditions, interference limitations), whether the UE is static/low or high mobile UE, whether it supports dual-connectivity (or multi-connectivity), whether it can be connected to the server via non-3GPP means (Wifi, NTN,..), the mobility of the UE or relative mobility to the UE of interest (source UE), etc.

The requirement can include also requested topology/option for the split model operation for which the relay/intermediate nodes need to be discovered. Alternatively, the model type may be provided in the request if there is an association between the type and the topology at the server side (as pre-configuration).

NOTE: Split model topology refers to how a model is divided and distributed across different devices or computational units for training or inference. These topologies can be broadly categorized into data parallelism, model parallelism, and hybrid approaches.

3.	The AIMLE server requests from ML repository and receives the registered nodes (available AIMLE clients, EASs to serve as candidate relays) with their roles as requested at the discovery criteria.

4.	The AIMLE server sends a discovery response to the AIMLE client including the discovered nodes and their capabilities. Such response includes the split learning operation related information such as VAL UE capability and availability information for acting as intermediate / relay node for the Split Operation and the type of relay, the supported split model topologies, VAL policies on the split operation etc.

5.	The AIMLE client after receiving the list of discovered AIMLE clients, it checks whether the other VAL UEs in the list are active and in close proximity. Hence it sends a groupcast/broadcast or unicast message via AIMLE-PC5 to all listed AIMLE clients, to receive reporting on the availability and conditions related to the sidelink interface. The other AIMLE clients can provide response if available and the source AIMLE client will establish a connection/session with them if they fulfil the application layer KPI (e.g. latency for the handshake is less than a pre-defined threshold).

Additionally, the sidelink status check can be in form of requesting from the target UEs additional /supplementary information on the channel status, load, energy and mobility information locally. The status check can be also using ADAE analytics for VAL UE-to-UE session performance stats/predictions as specified in TS 23.436.

6.	The AIMLE client of the VAL UE of interest (source UE) selects one of the candidate intermediate/relay nodes (edge or UE based relays) given criteria which can be equivalent to the discovery criteria and the performance KPI for the split learning operation. This may be based on local or VAL UE or server policies on the selection, e.g. to select UEs to meet KPIs with higher energy /load remaining capacity or prefer static UEs or edge nodes vs UEs.

Then the AIMLE client sends the selected intermediate node (AIMLE client or EAS) to the AIMLE server, including the identifier of the selected node and the time to activate (time of validity) for the addition to the pipeline, as well as to a pipeline ID (of the pipeline to be modified).

7.	AIMLE server associates the selected intermediate node in the AIML pipeline. This can be performed in different ways based on whether the intermediate node is a UE or an edge node and whether the intermediate node is expected to undertake part of the processing:

- If the relay is a UE (AIMLE or VAL client), the AIML pipeline update adds the relay UE as another entity in the existing pipeline with the associated flag as relay/intermediate and the type of relay and the layers that it is going to process (or this can be an indication of the type of processing that the relay will undertake if this is pre-configured, e.g. “relay_split_X corresponds to relay node handling layers 5-15”)

- If the relay is an edge node (EAS/edge AIMLE) then the AIMLE pipeline update add the edge node as another entity in the pipeline or creates two further pipelines (VAL UE-edge node, edge node to VAL server) which are coupled/associated with the first pipeline for the give AIML task/ split learning operation.

8.	After step 7 (or this can be before previous steps given the scenario) the AIMLE server notifies the VAL server and the intermediate AIMLE client on the addition of the intermediate / relay node to the pipeline (or to the creation of dependent/partner pipelines to the existing pipeline).

Editor's Note: Further implications on the procedure based on the type of intermediate node/relay (processing or just relaying) are FFS.

### 7.12.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.12.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.12.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.13	Solution #13: ML model training in a hierarchical AIMLE deployment

### 7.13.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

The solution proposes ML model training procedures considering AIMLE deployments in edge data networks as described in Annex A.4 of 3GPP TS 23.482 [2]. This solution makes no assumptions on the cardinality of ML repository(ies) deployed at the EDN or at the centralized DN.

ML model training procedure is described in Clause 8.3 of 3GPP TS 23.482 [2]. In a hierarchical AIMLE deployment scenario, a VAL server requests the central AIMLE server for ML model training. Since the member clients that will participate in the training process may be distributed in different EDN service areas, the central AIMLE server can distribute the training request to the corresponding edge AIMLE servers.

Figure 7.13.1-1 describes a procedure for central and edge AIMLE servers to support ML model training based on the request from a VAL server in a hierarchical AIMLE deployment.

Pre-conditions:

1.	AIMLE clients have registered with the corresponding edge deployed AIMLE servers as described in clause 8.7.2.2 of 3GPP TS 23.482 [2]. The edge deployed AIMLE servers receive AIMLE client profiles associated with each registered AIMLE client in the EDN service area.

2.	The edge deployed AIMLE servers have registered with the central AIMLE server as described in clause 7.9 (Solution #9).

Figure 7.13.1-1: ML model training in a hierarchical AIMLE deployment

1.	A VAL server sends an ML model training request to a central AIMLE server, as described in step 1 of Figure 8.3.2-1 in TS 23.482. The request includes information described in Table 8.3.3.1-1 in TS 23.482 [2].

2.	The central AIMLE server checks whether the VAL server is authorized to perform the ML model training request, as described in step 2 of Figure 8.3.2-1 in TS 23.482 [2]. The central AIMLE server uses information from the AIMLE server registration procedure described in clause 7.9 to identify edge AIMLE servers that can fulfil the ML model training requirements.

3.	Based on the ML model training request and the identified edge AIMLE servers, the central AIMLE server determines to distribute the training request to the identified edge AIMLE servers. The central AIMLE server identifies the EDN service areas based on information in the ML model training request (e.g. training type, list of member clients, member selection criteria) and the AIMLE client profiles (e.g. supported AI/ML operations, AIMLE client capabilities) received from the AIMLE server registrations. For example, the central AIMLE server determines the distribution of the request based on the required number of clients in the request and the number of available clients in each EDN service area or based on the dataset identifiers in the request and the dataset availability of clients in each EDN service area. The central AIMLE server sends an Edge ML model training request to the edge AIMLE server in each of the identified EDN service areas.

4.	The edge AIMLE server sends an Edge ML model training response to the central AIMLE server.

5.	The central AIMLE server returns the success or failure response, as described in step 3 of Figure 8.3.2-1 in TS 23.482 [2]. The response includes information described in Table 8.3.3.2-1 in TS 23.482 [2].

6. 	The edge AIMLE server notifies the central AIMLE server of the progress of the ML model training, including the list of member clients selected or de-selected for the ML model training, the training output, or errors encountered during the training process.

7.	The central AIMLE server aggregates the notifications received from the edge AIMLE servers and sends the aggregated notification to the VAL server. The notification includes information described in Table 8.3.3.3-1 in TS 23.482 [2].

### 7.13.2	Architecture impacts

No architectural impact has been identified.

### 7.13.3	Corresponding APIs

#### 7.13.3.1	Edge ML model training subscription request

Table 7.13.3.1-1 describes the information flow from the central AIMLE server to the edge AIMLE server for subscribing to ML model training in the EDN service area.

Table 7.13.3.1-1: Edge ML model training subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | Identity of the central AIMLE server of the edge ML training request. |
| Original requestor identity | O | Identity of the VAL server of the original ML training request. |
| Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| Training type | M | Identifies whether the VFL or HFL training to be performed. |
| List of member clients | O (NOTE) | List of member clients in the EDN service area to be utilized for training the ML model. |
| Member selection criteria | O (NOTE) | Identifies the criteria that needs to be continuously monitored for selecting the member clients (e.g., AIMLE clients in a particular location, member availability duration, new member clients registering with the required capabilities). |
| ML model information | M | Identifies the ML model (specified in the original request or selected by the central AIMLE server) that has to be distributed to the selected member clients in the EDN service area for training. The information is described in Table-8.11.4.1 of TS 23.482 [2]. |
| Operational requirements | O | Requirements for the training operations, e.g. latency, QoS requirements, etc. |
| Operational schedule | O | A schedule for when training is to occur. |
| Reporting requirements | O | Indicates what information regarding the training operations needs to be sent to the central AIMLE server by the edge AIMLE server (e.g. update related to new member clients selected or de-selected). The reporting requirements includes a specific schedule for sending notifications. |
| NOTE: At least one of these IEs shall be present. |  |  |

#### 7.13.3.2	Edge ML model training subscription response

Table 7.13.3.2-1 describes the information flow of Edge ML model training subscription response from the edge AIMLE server to the central AIMLE server.

Table 7.13.3.2-1: Edge ML model training subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Success response | O (NOTE 1) | Indicates that the Edge ML model training request was successful. |
| > Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| > EDN service area identifier | M | The identifier of the EDN service area associated with the edge AIMLE server. |
| Failure response | O (NOTE 1) | Indicates that the Edge ML model training request was failure. |
| > Cause | M | Reason for the failure. |
| NOTE: Only one of these information elements shall be present. |  |  |

#### 7.13.3.3	Edge ML model training notification

Table 7.13.3.3-1 describes the information flow of Edge ML model training notification from the edge AIMLE server to the central AIMLE server.

Table 7.13.3.3-1: Edge ML model training notification

| Information element | Status | Description |
| --- | --- | --- |
| Edge training identifier | M | Identifier to associate with the original ML training request (from the VAL server to the central AIMLE server). |
| EDN service area identifier | M | The identifier of the EDN service area associated with the registering AIMLE server. |
| EDN service area information | O | The information of the EDN service area (e.g., location, area, edge load of the area, etc.) associated with the edge AIMLE server. |
| Percentage completion | M | Indicates a completion percentage for the training. |
| List of AIMLE clients | O (NOTE) | Indicates the list of AIMLE clients selected or de-selected for the ML model training in the EDN service area. |
| Training output | O (NOTE) | Output of training, e.g., ML model parameters for the training. |
| Errors list | O (NOTE) | A list of errors, if any, encountered during training process. |
| NOTE: At least one of these IEs shall be present. |  |  |

### 7.13.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

## 7.14	Solution #14: Enhance AIMLE Client Discovery

### 7.14.1	Solution Description

As introduced in 3GPP TS 23.482, the AIMLE AI/ML capable UEs register to an AIMLE server and provide an AIMLE client profile and optionally a list of supported services. The AIMLE server uses the information provided by the AIMLE client to discover and select suitable AIMLE clients for requested AI/ML operations. When the AIMLE server/client are deployed hierarchically, enhancements to the AIMLE client discovery services are needed to support AIMLE service provisioning.

This clause specifies the impact to existing information flows for the enhancements of AIMLE client discovery services.

The AIMLE information flows in 3GPP TS 23.482 for AIMLE client discovery can be enhanced (highlighted in bold italics) as follows.

### 8.8.3 Information flows #### 8.8.3.1 AIMLE client discovery request Table 8.8.3.1-1 shows the request sent by a VAL server to an AIMLE server for the AIMLE client discovery procedure. Table 8.8.3.1-1: AIMLE client discovery request Information element; Status; Description. Requestor identity; M; The identifier of the requestor (e.g., VAL server).. AIMLE client discovery criteria; M; Discovery criteria for finding suitable AIMLE clients for AI/ML operations as detailed in Table 8.8.3.1-2.. Number of required AIMLE clients; O; Indicates the requested number of AIMLE clients to be discovered based on the discovery criteria.. Table 8.8.3.1-2: AIMLE client discovery criteria Information element; Status; Description. Service requirement; M; Information about the required service. > VAL Service ID; M; VAL Service ID that the client is required to support. This identifies the service associated with the requester.. > Service permission level; O; Required corresponding service permission level (e.g., premium resource usage standard resource usage, limited resource usage).. Requested ML model types; O; Requested ML model types (decision trees, linear regression, neural networks).. Requested AIML operations; M; Requested role for AI/ML operations such as model training, model transfer, model inference, model offload, model split.. Application layer AIMLE client capabilities; M; Application layer AIMLE client capability information (e.g., ML application type like FL/TL/SL, client availability to support AIML operations at the UE, AIMLE drop off rate).. Dataset requirements; O; Information about dataset.. > Dataset availability; O; Dataset availability, including dataset identifiers, dataset size, age, list of dataset features.. > Data capabilities; O; A list of data capabilities such as the type of data that can be collected (e.g., raw data), supported data processing capabilities (e.g., processed data) and supported exploratory data analysis functions.. AIML client task capability requirements; O; Indicates the AIML task requirements to discover the AIML clients for performing AIML tasks. It includes compute capabilities (e.g., high, low), task performance preference capabilities (e.g. Green task, Energy-efficient, low costs). AIMLE client velocity; O; Indicates the AIMLE client velocity. It includes mobile (e.g., high, low), static.. Location information; O; Indicates the location information (e.g., Cell Identity, Tracking Area Identity, GPS Coordinates or civic addresses, VAL service area ID, DNN and DNAI(s)) to discover the AIMLE clients.. AIMLE client QoS requirements; O; Indicates the AIMLE client QoS information (like PLR, bandwidth, latency jitter) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above or below) to discover the AIMLE clients.. Edge AIMLE server Information; O; Information about edge deployed AIMLE server(s).. > EAS information; O See NOTE; Indicates information of the AIMLE server deployed as EAS, which may include EAS ID, EAS Endpoint, DNAIs and service area of the EAS.. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server.. NOTE: At least one of the IEs shall be provided when edge deployed AIMLE server.. #### 8.8.3.2 AIMLE client discovery response Table 8.8.3.2-1 shows the response sent by the AIMLE server to the VAL server for the AIMLE client discovery procedure. Table 8.8.3.2-1: AIMLE client discovery response Information element; Status; Description. Status; M; The status for the request: success or fail.. List of AIMLE client IDs; M; A list of AIMLE client IDs that matches the AIML discovery criteria.. > AIML supported tasks; O; Indicates the discovered AIML task providers fulfilling the AIML task and AIML task requirements. It includes expected AIML task KPIs like latency, available AIML tasks service duration, list of AIML task providers like IDs or URI, and supported tasks.. > EAS information; O See NOTE; Indicates information of the AIMLE server deployed as EAS (which serving the AIMLE client), which may include EAS ID, EAS Endpoint, DNAIs and service area of the EAS.. > AIMLE server ID; O See NOTE; The identifier of the AIMLE server which serving the AIMLE client.. NOTE: At least one of the IEs shall be provided when the AIMLE client is served by edge deployed AIMLE server..

### 7.14.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.14.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.14.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.15	Solution #15: Sample alignment for VFL

### 7.15.1	Solution description

The solution enhances the clause 8.18 as follows:

### 8.18.2 Procedure for supporting VFL Pre-conditions: 1. VFL members registered their VFL profile to the AIMLE Server. The VFL members may update their status or information in their VFL profile to the AIMLE Server. 2. The datasets of each of the UEs (where the AIMLE Clients (as VFL members) are deployed on) belong to more than one different data domains. 3. The VAL server has successfully subscribed/registered with the AIMLE server for model training notifications. Figure 8.18.2-1: Procedure for supporting VFL 0. An AIMLE client registers with AIMLE server as specified in clause 8.7 of 3GPP TS 23.482 [2]. The registration message includes the list of feature collection capabilities and also the data domains in which features can be collected and processed. The registration message also includes the list of feature ID(s) status its expiration time. NOTE: The AIMLE server keeps record of all AIMLE clients with similar feature collection capabilities and data domain. 1. An AIMLE server receives an ML model training request from a VAL server as described in clause 8.3.2. The request includes sample pool, and sample binding criteria. The member selection criteria are enhanced to select the members for VFL training if they have features bound to at least one sample withing the sample pool. If ML model training request is received and the AIMLE server determines to use VFL training (VFL training between different data domains), the procedure continues to step 2. The AIMLE server also assigns ML task identity to the determined task. The AIMLE server selects the candidate AIMLE clients for the training based on the corresponding feature collection record of AIMLE clients that are capable of collecting features within the scope of determined VFL task and sample pool. The AIMLE server sends response which includes ML task identity and candidate AIMLE client list. 2. [Optional] The AIMLE server retrieves the indicated ML model in step 1 using the ML model retrieval procedure as described in clause 8.1. If the retrieved model is already trained and meets the machine learning requirements requested by the consumer, the procedure continues to step 7. 3. If AIMLE client selection criteria are provided in step 1, then the AIMLE server continuously monitors and selects AIMLE clients for the VFL training as described step 4 of the clause 8.13.2. If a list of AIMLE clients is provided in step 1, the AIMLE server selects the provided AIMLE clients for the VFL training. 4. [Optional] The AIMLE Server sends a client feature selection request to some clients asking to activate or de-activate some of their feature ID(s)/ feature list. The message includes information as described in following Table. Information element; Status; Description. Requestor identifier; M; The identifier of the AIMLE Server.. Feature ID(s); M; Server requests List of feature ID(s) targeted for activation or deactivation.. Action type; M; Server specifies via indicators whether the request is for activation or deactivation.. Feature Priority Level indicator; O; Indicates the priority of the feature for VFL training. Minimum expiration Time indicator; O; Server informs the client UE(s) on the minimum expiration time to enforce the requested action (activate /deactivate features). 5. [Optional] AIMLE client sends response indicating the activation or deactivation status of their feature ID(s) or feature list. The response message includes information as described in following Table. Based on the response message, the AIMLE server finds common feature ID(s) by comparing features descriptions / feature ID(s) of clients. The AIMLE server constructs the pairwise relation among candidates' datasets considering their shared feature ID(s). Information element; Status; Description. Successful response (NOTE); O; Indicates that the request was successful.. > Action Type status; M; Status of feature ID(s) activate/deactivate operation. > Feature priority level success rate; O; Indicates the percentage of the available features prioritized for VFL training.. > Feature Minimum expiration Time reliability rate; O; Client (UE)s informs the server about the specified feature set/feature ID(s) enforced expiration Time. In case of conflict of interest between UE and server, UE send remarks about further adjustment in expiration time. Failure response (NOTE); O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: any one IE shall be present.. NOTE: The AIMLE server gets the hashed feature values from the candidates for their data samples if the feature ID(s) are subsets of feature space union 6. AIMLE server send a sample binding request to AIMLE clients that are capable of collecting or having features corresponding to the defined VFL task. This request contains a set of samples that are defined to be the sample pool of the VFL task by the VAL server. The message includes information as described in following Table. Table 8.18.2-1: Sample binding request Information element; Status; Description. Requestor identifier; M; The identifier of the AIMLE Server.. VFL Task ID; M; The identifier of the VFL task assigned to AIMLE server by the VAL server. Sample Pool; M; A set of samples and their corresponding IDs that are defined within the scope of VFL task to be trained on them.. Sample binding criteria; M; Identifies the criteria that the available or collecting dataset (features) can be bound to samples within the sample pool.. 7. AIMLE client receives the binding request, and it matches its available dataset (i.e. features) at the UE to a corresponding sample within the sample pool of the VFL task according to the received sample binding criteria. ALMLE client sends the sample binding response to AIMLE server. If the ALMLE client was able to match its dataset (i.e. features) to at least one sample within the sample pool of the VLF task, it sends the successful binding response to ALMLE server to continue the process of VFL sample alignment. Otherwise, it sends a failure response to the sample binding request by AIMLE server. The binding response message includes information as described in following Table. Table 8.18.2-2: Sample binding response Information element; Status; Description. VFL Task ID; M; The identifier of the VFL task assigned to AIMLE server by the VAL server. Binding result; M; The binding result either “success” or “failure”. Editor's note: It is FFS, the details of binding pool and binding criteria information and its relation to VFL sample alignment. 8. The AIMLE Server gets VFL members information (the VFL members information may be included in the request in step 1 (Table 8.3.3.1-1) or be obtained through step 3 or according to the responses received in step 7. The AIMLE Server interacts with the VFL members (AIMLE Clients which are deployed on UEs) for each data domain for ML model training capability evaluation as described in clause 8.19.2. 9. The AIMLE Server determines the VFL members for this VFL training process based on the information received in step 4 and parameters received in step 1 (Table 8.3.3.1-1). The AIMLE Server determines VFL members for each data domain. The criteria used by the AIMLE Server include: - Available data and minimum number of data samples for the same sample among the VFL members. - Feature alignment of the sample/datasets with data labels among the VFL members. - Available time of the VFL members for support the VFL training operations. - Capability and minimum number of the VFL members for the VFL training operations. - AIML model information for the VFL members and for the AIMLE Server. 10. The AIMLE Server coordinates the selected VFL members for VFL training. During VFL training process, the VFL members send intermediate results to the AIMLE Server, and the AIMLE Server responds to the VFL members with the updated information (e.g. gradients). The information from the AIMLE server can be used to update the model parameters maintained at each VFL member for the different data domains. 10a. The AIMLE server may report to the VAL server with the training status, that includes intermediate training results, The VAL server may adjust its request on the ML model training. If the VAL Server is providing data labels to complete the training, the VAL Server sends updated training parameters for the AIMLE Server to distribute to the VFL members. The updated training parameters apply for models of VFL members associated with each data domain. NOTE: How AIMLE Server coordinates the selected VFL members to perform VFL training is out of scope for this release. 11. The AIMLE server sends a ML model training notification to the VAL server as described in clause 8.3.2. If the training schedule is not complete (e.g., there are remaining training rounds), the AIMLE server configures the next set of training schedules and steps 3 to 6 are repeated for the next training round.

### 7.15.2	Architecture Impacts

No architecture impacts are identified for this solution.

### 7.15.3	Corresponding APIs

Editor's Note:	Corresponding APIs for this solution is FFS.

### 7.15.4	Solution evaluation

Editor's Note:	Solution evaluation for this solution is FFS.

## 7.16	Solution #16: Solution on AIMLE service migration support in UE roaming scenarios

### 7.16.1	Solution description

This solution addresses Key Issue #3. The solution provides a mechanism for supporting the continuity of the ML training / inference capability in scenarios where the ML model training / inference happens at the UE (e.g. AIMLE client) while the UE client is expected to move to an area served by another PLMN.

In this solution, the assumption is that the VAL UE undertakes an ML operation (such as training or inference) and is served by a source AIMLE server to which he is connected via H-PLMN, and the VAL UE is roaming while the operation is ongoing. In such scenario, to prevent AIML service disruption, this solution provides a mechanism to enable the migration of AIMLE service to the target AIMLE server who is bound to VPLMN.

Such solution requires that service / roaming agreements between AIMLE service providers in both PLMNs are in place to support service the roaming UEs.

Pre-conditions:

- The AIMLE-provided ML model training/inference support service is ongoing.

Figure 7.16.1-1 procedure for support for VAL UE roaming

1.	The AIMLE client via the VAL client or other SEAL clients (LMC, ADAEC) monitors UE mobility or analytics related to the expected or predicted mobility of the UE. The AIMLE client detects a mobility of UE to an area covered by different PLMN and sends a mobility report to AIMLE server indicating a cross-PLMN mobility event for the VAL UE.

2.	AIMLE server identifies (check with Core Network) the V-PLMN info and the time when the VAL UE is expected to reach V-PLMN.

3.	AIMLE server fetches from ML repository information on available AIMLE servers in target V-PLMN.

NOTE 1: If the ML repository is deployed by PLMN/MNO, then this step requires interaction among ML repositories of the same provider based on the service agreements or synchronization via the AIMLE servers. The mechanism for this coordination is up to implementation.

Editor's Note: It is FFS how the ML repository is aware of the available AIMLE servers for the V-PLMN.

4.	AIMLE server sends a notification or request for selecting a new AIMLE server, where the message includes the list of available AIMLE servers in VPLMN and some context information (e.g. rating/ranking, vendor info, capability information, energy/load status).

5.	AIMLE client based on the received list, selects the T-AIMLE server from a list of AIMLE servers, based on the fetched information. The selection criteria can be:

- Capabilities (computational, processing) and coverage (geographical or topological area covered by T-AIMLE server)

- Load and energy status of the target AIMLE server, based on monitoring or querying possible overload or high energy consumption (can be provided by the target AIMLE service provider, e.g. edge platform provider).

- Expected delay / latency for accessing the T-AIMLE server (e.g., in case of relaying ML inference task). This can be estimating based on probes or based on relative location of platform hosting the T-AIMLE server or based on the interface / API statistics on delay or failures based on high delay.

- Vendor compatibility: this can be the case when the selection of a target AIMLE is based on which vendor provides the server. It can be either the same vendor only, or a list of selected vendors based on the service agreements among MNOs/SEAL providers.

- Cost for task migration: The computational effort and/or processing required, or time required, or energy required for offloading a task to the T-AIMLE server. Such cost may be based on the distance among servers (e.g. edge to regional cloud) or based on the task to be offloaded.

- A rate/ranking/weighting factor of target AIMLE server based on fulfilment of previous tasks and statistics. Such rate can be stored and updated at ML repository (or shared among repositories) after fulfilling each task based on consumer feedback or based on internal evaluation at the AIMLE (based on success or failure of tasks under similar conditions)

NOTE 2: Pre-defined policies from the S-AIMLE server or VAL server may be provided in previous steps to dictate the AIMLE server selection criteria (policies on minimum rating, allowed vendors, preferred AIMLE servers)

6.	The AIMLE client sends to the AIMLE server the selected T-AIMLE server and the expected time and area of roaming.

Editor's Note: Whether the T-AIMLE server can be selected by the S-AIMLE server instead of AIMLE client as in steps 4-6 is FFS.

7.	The AIMLE server requests or negotiates with target AIMLE server to undertake the task of serving the AIMLE client of interest.

8.	The AIMLE server requests AIMLE client to change serving AIMLE server and passes the target AIMLE server info, also possible limitations in target V-PLMN (lower priority, capacity limitations for ML operation). The AIMLE client confirms and connects to target AIMLE server via the source AIMLE server.

-----------UE of interest roams to target PLMN and AIMLE client connects to T-AIMLE server-------

9.	The T-AIMLE server sends a notification to S-AIMLE server about the roaming of the UE(s) of interest. Following the notification may also reach the VAL server (as expected consumer of the ML training/inference output).

10.	The T-AIMLE server if the task is about relaying the ML inference output over the VPLMN, it provides the output via the S-AIMLE server to the VAL server. If the T-AIMLE server is the new AIMLE server, it sends directly the output to VAL server.

### 7.16.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.16.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.16.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.17	Solution #17: Enhancement of AIMLE to support model inference

### 7.17.1	Solution description

This proposal addresses Key Issue #1, Open Issue 5. The proposed solution enables UE-edge collaborative inference, where a VAL UE (with an AIMLE client) and a nearby edge AIMLE server jointly execute an ML model inference. In this approach, the inference pipeline is partitioned: the VAL UE performs the privacy-sensitive and ultra-low-latency portion of the model locally, while the computation-intensive remainder is offloaded to an edge AIMLE server. By splitting the inference execution between the device and edge, the solution meets strict latency requirements and keeps sensitive raw data on the device, while leveraging edge computing for heavy processing.

Although the proposed UE-edge collaborative inference procedure shares the general concept of distributed inference with the pipeline solution defined in Rel-19, there are fundamental differences in terms of architecture, coordination entities, and applicable scenarios. The current proposal specifically targets collaboration between the AIMLE/ADAE client (residing in the UE) and the edge AIMLE/ADAE server, which hosts part of the ML model and provides inference service functions. This solution introduces a new set of interactions, including model selection, authorization, intermediate data upload, and result feedback, none of which are covered by the Rel-19 pipeline solution.

This clause describes the procedure for supporting collaborative inference between AIMLE/ADAE client and edge AIMLE/ADAE server.

Figure 7.17.1-1 illustrates the procedure for AIMLE/ADAE server to support collaborative inference between AIMLE/ADAE client and edge AIMLE/ADAE server.

Pre-condition:

1	The ML model needed to be used for this ML model inference is available by the AIMLE/ADAE server.

Figure 7.17.1-1: ML model collaborative inference

1.	The AIMLE/ADAE client initiates the inference service by sending a request to the edge AIMLE/ADAE server. This request indicates that the UE intends to perform an ML model inference collaboratively (split between UE and edge AIMLE/ADAE). The AIMLE/ADAE client identifies the target ML model or inference task type and that an edge AIMLE/ADAE is needed to complete the inference. This request consists of the ML model identifier or inference task description, and potentially the inference partitioning info, it may also carry an inference session ID or a request for the edge to allocate resources.

2.	The edge AIMLE/ADAE server performs an authorization check. It also ensures the specified model (or sub-model for the later layers) is available. If the client only provided a task type (no specific model ID), the AIMLE/ADAE server selects an appropriate ML model for the task using the AIMLE model discovery/selection procedure (per 3GPP TS 23.482 clause 8.11.3).

3.	If the request is authorized and a model is ready, the edge AIMLE/ADAE server replies with a ML Model collaborative inference response, the Inference Setup Response acknowledging readiness, otherwise a failure response indication the reason for failure.

4.	Upon confirmation, the AIMLE/ADAE client proceeds to execute the first part of the ML model locally. The AIMLE/ADAE client runs the input data (provided by the VAL application) through the initial model layers (up to the agreed split point).

5.	Intermediate data upload: The AIMLE/ADAE client sends the intermediate inference output to the edge AIMLE/ADAE server. This intermediate data, such as a feature vector output from the last locally executed model layer (e.g., FeatureX), serves as input to the next stage of inference. The message includes an identifier to associate it with the earlier setup (e.g. the session ID or pipeline ID from step 2) so that the edge knows which model/task this data corresponds to.

6.	The Edge AIMLE/ADAE server uses the received intermediate data as input to the remaining portion of the ML model. It runs the model's subsequent layers or inference function to produce the final result.

7.	The edge AIMLE/ADAE server sends a collaborative inference result response message back to the AIMLE/ADAE client. This response contains the final inference result (for example, the predicted class label and confidence score, or other output data of the model). It also can include the ML model ID that was used, a status code indicates success or failure, cause value, etc.

### 7.17.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.17.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.17.4	Solution evaluation

## This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.18	Solution #18: Solution on support for federated AI services

### 7.18.1	Solution description

This solution addresses Key Issue #3.

In the context of AI/ML services, a federated AIML service can have different flavour based on the scenario.

- Scenario 1: an FL training operation in different Edge servers (EAS) or cloud servers (CAS): EAS#1 of EDN1 is an FL client, EAS#2 of DN#2 is another FL client and CAS of DN#3 is an FL server/aggregator.

- Scenario 2: an ML model training or inference operation split in different EDNs/DNs. In such scenario, the AIML operation (e.g. inference) is split among UE and network entities. If such network entities belong to different operators, then a federation agreement is in place and the AIML operation is split among different stakeholders /operators.

- Scenario 3: If an ML operation like model training is provided per ADAE analytics event, then the analytics service comprises the ML model training, model inference and the analytics operation which is provided by an analytics engine. Such ML-enabled analytics service may be distributed among different network or server entities provided by different network operators. One example can be the MNO #x is performing ADAES analytics on VAL performance in an analytics engine in the telco platform, and the ML model training is requested and provided by an external entity (e.g. AWS) using data provided by 3rd party. In certain use cases, the ML model training may be performed by a CAS in a less granular fashion, and the analytics is locally performed by one or more ECSPs at the respective edge platforms which may have overlapping or orthogonal coverage.

For the above scenarios, a federated AIML service can be defined as an AI/ML service which requires the joint operation of subsequent AI operations provided by different providers (MNO, ECSP, CSP, Device owner, App developer).

This solution addresses the key issue on how to configure and enforce federation agreements for AI/ML operations requiring joint or split processing by platforms operated by different providers. In particular, the solution provides a mechanism to translate a request for an AI/ML service to a requirement for a federated AI service, given the resource conditions and the federation agreements in place.

The procedure for the configuration of the federated AIML service based on the VAL request for an AI/ML task (ML model training or inference request) is shown below.

Pre-condition:

- AIMLE consumer such as AIMLE client or VAL server has ML model or address to ML model.

Figure 7.18.2-1: Procedure on support for federated AI services

- An AIMLE server of CSP #1 receives an ML model training request from a VAL server as described in clause 8.3.2 of TS 23.482 or an ML model inference request (as described in Solution #1).

- The AIMLE server determines to trigger a federated AIML service, which can be an AIMLE service such as federated training or inference spanning across different CSPs. The determination can be based on the capability or availability limitations of CSP #1 for providing the AI/ML service; hence support from another CSP is needed.

- The AIMLE server performs an ML model information discovery, to discover ML model support for federated AI scenarios, where the discovery request (as specified in 3GPP TS 23.482 clause 8.11.4.3) is enhanced to include federation information, i.e. list of information for different federation agreements related to the ML model, in particular the role of other edge/cloud providers in accessing or operating the ML model (or part of it). This includes information on partner CSPs, permissions, capabilities and availability of the CSP offering part of the AIML service.

- In this step, the AIMLE server obtains the identity and address of the target CSP which is planned to undertake part of the AI/ML service based on step 3.

The AIMLE server determines to select the target AIMLE server of CSP #2 to form the Federated AIML service and generates and Federated Service ID. The AIMLE server of CSP #1 provides a configuration of the federated AIML service including the service area of interest, time validity, operation expected per lead and partner OP, the dataset requirements by the partner OP.

Then, AIMLE server sends a Federated AIML service configuration request to the partner CSP (AIMLE server in target CSP) to provide the configuration parameters.

- The target AIMLE server after authorizing the request, it checks the federation agreement (with its ML repository) and confirms or negotiates the configuration parameters. Then, the target AIMLE server provides a positive or negative response to the source AIMLE server (or CSP #1).

In case of negotiation (which can be due to the load/resource/energy situation in the target edge platform's OP), the target AIMLE server may provide an alternative set of configuration parameters (e.g. limited coverage or time availability or time to accomplish the ML model training, or data availability) and further confirmation from source AIMLE server may be provided.

- The AIMLE server sends an ML model training/inference response including also the trained/inferred model and optionally a notification that the training/inference was outcome of a federation support (this may be needed to require more fees for the charging of the consumer).

- The AIMLE server performs an ML model information storage with the updated mapping of the ML model or AI service with the federation information.

### 7.18.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.18.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.18.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.19 	Solution #19: Cross-PLMN/Domain AIMLE client discovery, selection, monitoring

### 7.19.1 	Solution Description

#### 7.19.1.1 	General

In 3GPP TS 23.482, there is currently a capability to discover and select AIMLE clients. Key Issue #3 has identified that cross-domain and cross-PLMN scenarios are not covered in TS 23.482 Release 19. This solution enhances AIMLE client discovery and selection to include cross-domain and cross-PLMN deployments. Figure 7.19.1.1-1 shows the deployment scenario where the AIMLE servers (AIMLE Server-1 and AIMLE Server-2) are in different domain or PLMN and the AIMLE servers in these different domains or PLMN have federation agreement to achieve discovery, selection and monitoring.

Figure 7.19.1.1-1: Use case for cross-domain/cross-PLMN federation

The following enhancements are proposed to enable cross-domain and cross-PLMN discovery, selection, and monitoring:

- Cross-domain and cross-PLMN deployments have AIMLE servers in different domains. When a discovery request fails to find sufficient AIMLE clients, the AIMLE server can discover AIMLE clients from another AIMLE server in another domain and select the desired clients.

- The AIMLE server, as per 3GPP TS 23.482 clause 8.13, can monitor AIMLE clients in its own domain/PLMN. To monitor AIMLE clients in another domain/PLMN, the AIMLE server can subscribe to the AIMLE server in that other domain/PLMN to receive status updates about those AIMLE clients.

- Based on the status of AIMLE clients in the current domain/PLMN and notifications received from the other domain/PLMN, the AIMLE server can replace AIMLE clients in the other domain/PLMN with AIMLE clients in the current domain/PLMN when they become available, in order to minimize cost.

- During AIMLE client discovery and selection, the AIMLE server can include in its request to the other AIMLE server (in another domain/PLMN) specific AIMLE clients that have migrated, to enhance service continuity.

#### 7.19.1.2 	Procedure

Figure 7.19.1.2-1: Cross-domain/cross-PLMN AIMLE client discovery, selection and monitoring

Editor's Note: Further details of the procedure is FFS.

### 7.19.2 	Architecture Impacts

Editor's Note: Architectural impact of cross-PLMN and cross-domain deployment is FFS.

### 7.19.3 	Corresponding APIs

Editor's Note: The corresponding APIs are FFS.

### 7.19.4 	Solution Evaluation

Editor's Note: The evaluation of the solution is FFS.

## 7.20	Solution #20: ML Model Performance Evaluation with information collection from ML model consumer

### 7.20.1	Solution Description

This clause describes the procedure and information flows for supporting ML model performance evaluation with information collection from ML model consumer.

#### 7.20.1.1	Procedure

Figure 7.20.1.1-1: ML model performance evaluation with information collection from ML model consumer

Figure 7.20.1.1-1 illustrates the procedure on ML model performance with information collection from ML model consumer. The corresponding procedure in detail is as follows:

1. The VAL server sends to the AIMLE server the ML model evaluation subscription request message containing the ML model ID, the VAL Service ID, the ML model evaluation ID, and the purpose on the ML model as specified in Table 7.20.1.2.1-1. The ML evaluation profile ID is used to obtain the ML model evaluation profile which is used to determine the ML model performance in step 10.

2. The AIMLE server sends the ML model performance evaluation subscription response message to the VAL server, containing a success message if the VAL server is authorized to subscribe. Otherwise, an error message is sent to the VAL server. The requested message includes the information specified in Table 7.20.1.2.2-1.

3. The AIMLE sends to the A-DCCF the ML model performance information collection request to get the ML performance of the model. The message contains the information as specified in Table 7.20.1.2.3-1.

4. The A-DCCF determine the list of ML model consumers based on ML model consumer list, the ML model ID, and the info on the purpose on the use the ML model, and the VAL service ID.

5.	The A-DCCF sends to the ML model consumer the information collection request message to obtain the ML model performance of the respective ML model consumer. The information collection request message contains the ML model info (e.g., ML model ID and the purpose on use the ML model) and it is specified in Table 7.20.1.2.4-1.

6.	The ML model consumer generate the ML model performance information based on usage of the ML model.

7. Upon generating the ML model performance, the ML model consumer sends to the A-DCCF the information collection response message containing information as specified in Table 7.20.1.2.5-1.

8.	Upon receiving the Information collection response, the A-DCCF aggregates the information received from each ML model Consumer.

9.	The A-DCCF contacts the AIMLE server with the ML model performance information collected from the ML model consumer. The message contains the ML model info (e.g., ML model ID), the purpose on the usage of ML model, ML model consumer info, the ML model performance metrics and it is specified in Table 7.20.1.2.6-1.

10.	Based on the ML model performance received in the previous step, the AIMLE server computes the ML model performance metrics based on the information provided by the ML consumers and the ML model evaluation profile information.

11.	The AIMLE server sends to the VAL server the ML model performance evaluation subscription notification containing the information about the performance of the requested ML model. The detail information contains in the notification is as specified in Table 7.20.1.2.7-1.

Alternatively, instead of steps 3-9, the AIMLE server collects ML model performance information from the ML model consumer directly.

Editor's Note: Whether the procedure specified in clause 8.12 of 3GPP TS 23.436 [4] can be reused with enhancements for AIMLE Server to request data collection via A-DCCF is FFS.

#### 7.20.1.2	Information Flows

##### 7.20.1.2.1 	ML model evaluation subscription request

Table 7.20.1.2.1-1 describes information elements for the ML model evaluation subscription request from the VAL server to the AIMLE server.

Table 7.20.1.2.1-1: ML model evaluation subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| VAL service ID | M | The identifier of the VAL service |
| ML model evaluation profile ID | M | The identifier of the ML model evaluation profile. |

##### 7.20.1.2.2	 ML model evaluation subscription response

Table 7.20.1.2.2-1 describes information elements for the ML model evaluation subscription response from the AIMLE server to the VAL server.

Table 7.20.1.2.2-1: ML model evaluation subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | The identifier of the subscription. |
| Model ID | M | The identifier of the Model |
| VAL service ID | M | The identifier of the VAL service |
| Request successful | O (NOTE) | ML model evaluation request successful message |
| Error message | O (NOTE) | ML model evaluation request error message |
| NOTE: At least one of these IE shall be present. |  |  |

##### 7.20.1.2.3	 ML model performance information collection request

Table 7.20.1.2.3-1 describes information elements for the ML model performance information request from the AIMLE server to the A-DCCF.

Table 7.20.1.2.3-1: ML model performance information collection request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| ML consumer List | M | The list of consumers that uses the ML model identified by the ML model ID |
| VAL service ID | M | Indicates the VAL service ID |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |

##### 7.20.1.2.4 	Information collection request

Table 7.20.1.2.4-1 describes information elements for the information collection request from the A-DCCF to the ML model consumer.

Table 7.20.1.2.4-1: Information collection request

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | O (NOTE) | The ML model identifier |
| Analytics ID | O (NOTE) | The identifier of the analytics service. |
| Data source IDs | M | The Identifier of the data sources |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |
| Interest time period of the historical data | O | Interest time period of the historical data. |
| NOTE: At least one of the IE shall be provided. |  |  |

##### 7.20.1.2.5 	Information collection response

Table 7.20.1.2.5-1 describes information elements for the Information collection response from the ML model Consumer to the A-DCCF.

Table 7.20.1.2.5-1: Information Collection response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | O (NOTE) | The ML model identifier |
| Analytics ID | O (NOTE) | The identifier of the analytics service. |
| ML model performance information | M | It contains the aggregated information regarding the performance of the ML model identified by the model ID on the data sources (e.g., accuracy, F1 score, AUC). |
| Data source IDs | O | The Identifier of the data sources used to collect the ML model performance |
| Interest time period of the historical data | O | Interest time period of the historical data. |
| NOTE: At least one of the IE shall be provided. |  |  |

##### 7.20.1.2.6 	ML model performance information collection response

Table 7.20.1.2.6-1 describes information elements for the ML model performance information response from the A-DCCF to the AIMLE server.

Table 7.20.1.2.6-1: ML model performance information collection response

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID | M | The ML model identifier |
| ML consumer list | M | The list of consumers that uses the ML model identified by the ML model ID |
| ML model performance indication | M | It contains the information regarding the performance of the ML model identified by the model ID. (e.g., accuracy, F1 score, ROC, AUC) |
| Purpose on the usage of ML model | O | The ML model purpose (e.g., prediction, classification) |

##### 7.20.1.2.7 	ML model evaluation notification

Table 7.20.1.2.7-1 describes information elements for the ML model evaluation notification from the AIMLE server to the VAL server.

Table 7.20.1.2.7-1: ML model evaluation notification

| Information element | Status | Description |
| --- | --- | --- |
| Model ID | M | The identifier of the model |
| VAL service ID | M | The identifier of the VAL service |
| ML model performance information | M | It contains the aggregated information regarding the performance of the ML model identified by the Model ID on the data sources. (e.g., accuracy, F1 score, AUC) |

### 7.20.2	Architecture Impacts

Editor's Note:	The architecture impacts of the solution is FFS.

### 7.20.3	Corresponding APIs

Editor's Note:	The corresponding APIs for supporting the solution is FFS.

### 7.20.4	Solution evaluation

Editor's Note:	The evaluation of the solution is FFS.

## 7.21	Solution #21: Split operation node registration involving multiple AIMLE clients

### 7.21.1	Solution description

This solution is related to Key Issue #6: support for architecture and functions enhancement on diverse AIMLE deployments scenarios.

AIMLE client may provide its computing capability to act as a processing node, especially multiple AIMLE client collaborating scenario. AIMLE client can provide their idle computing resource to perform computing tasks of other UEs. Network may provide benefits for such sharing. For example, network can give a discount to the UE when UE request for network computing tasks. In this solution, both AIMLE client and VAL server can registrater in AIMLE server to participate in split AI/ML operations as a processing node.

The Split operation node registration procedure as specified in clause 8.14 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

Editor's Note:	 The UE registering as a processing node for split operation is subject to the user’s consent, which is FFS.

#### 8.14.2.4 Split operation node registration ##### 8.14.2.4.1 General Clause 8.14.2.4.2 illustrates the split operation node registration procedure. Clause 8.14.2.4.3 illustrates the split operation node registration update procedure. Clause 8.14.2.4.4 illustrates the split operation node de-registration procedure. ##### 8.14.2.4.2 Split operation node registration Figure 8.14.2.4.2-1 illustrates the procedure for VAL server/AIMLE client to register with the AIMLE server to indicate split operation capabilities. Pre-conditions: 1. The VAL server/AIMLE client has received information (e.g. URI, IP address) related to the AIMLE Server; 2. The VAL server/AIMLE client has received security credentials authorizing it to communicate with the AIMLE server; Figure 8.14.2.4.2-1: Split operation node registration 1. The VAL server/AIMLE client sends a split operation node register request to the AIMLE server to indicate the split operation capabilities of the VAL server/AIMLE client. The request includes information defined in Table 8.14.3.6-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request, the AIMLE server stores the registration information. 3. The AIMLE server sends a split operation node register response to the requestor. If the AIMLE server has registered the requestor, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation node registration expires. If the requestor was not registered, the response includes an indication of failure and may include a reason for failure. ##### 8.14.2.4.3 Split operation node registration update Figure 8.14.2.5-1 illustrates the procedure for VAL server/AIMLE client to update a split operation node registration with the AIMLE server. Pre-conditions: 1. The VAL server/AIMLE client has registered with the AIMLE Server. Figure 8.14.2.4.3-1: Split operation node registration update 1. The VAL server/AIMLE client sends a split operation node registration update request to the AIMLE server to update an existing split operation registration. The request includes information defined in Table 8.14.3.8-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server updates the registration information. 3. The AIMLE server sends a split operation node registration update response to the requestor. If the AIMLE server has successfully updated the registration, the response includes an indication of success and may include an updated expiration time. The requestor shall send a registration update request before the expiration time to maintain the registration; otherwise, the split operation registration expires. If the registration was not updated, the response includes an indication of failure and may include a reason for failure. ##### 8.14.2.4.4 Split operation node de-registration Figure 8.14.2.4.4-1 illustrates the procedure for VAL server/AIMLE client to de-register for split operation with the AIMLE server. Pre-conditions: 1. The VAL server/AIMLE client has registered with the AIMLE Server. Figure 8.14.2.4.4-1: Split operation node de-registration 1. The VAL server/AIMLE client sends a split operation node de-register request to the AIMLE server. The request includes information defined in Table 8.14.3.10-1. 2. Upon receiving the request from the VAL server/AIMLE client, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server terminates the registration for split operation. 3. The AIMLE server sends a split operation node de-register response to the requestor. If the AIMLE server has successfully terminated the registration for split operation node, the response includes an indication of success. If the registration for split operation was not terminated, the response includes an indication of failure and may include a reason for failure.

The Information flow of split operation node registration as specified in clause 8.14.3 of 3GPP TS 23.482 [2] is enhanced as follows (new text in bold italics):

#### 8.14.3.6 Split operation node register request Table 8.14.3.6-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node register request. Table 8.14.3.6-1: Split operation node register request Information element; Status; Description. Requestor identifier; M; The identity of the requestor (e.g., VAL server ID, AIMLE client ID).. Security credentials; M; The security credentials of the requestor.. Node information; M; Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.).. Split operation capabilities; M; Split operation capabilities of the VAL server/AIMLE client.. > Model information; M; Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.).. > Usage information; O; Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.).. Expiration time; O; The proposed expiration time of the registration.. #### 8.14.3.7 Split operation node register response Table 8.14.3.7-1 shows the response sent by an AIMLE server after processing of split operation node register request. Table 8.14.3.7-1: Split operation node register response Information element; Status; Description. Successful response; O; Indicates that the request was successful.. > Registration identifier; M; The identifier of the registration.. > Expiration time; O; The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires.. Failure response; O; Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. #### 8.14.3.8 Split operation node registration update request Table 8.14.3.8-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation node registration update request. Table 8.14.3.8-1: Split operation node registration update request Information element; Status; Description. Registration identifier; M; The identifier of the registration.. Security credentials; M; The security credentials of the requestor.. Node information; O; Information about the VAL server/AIMLE client node (e.g., identifier, endpoints, etc.).. Split operation capabilities; O; Split operation capabilities of the VAL server.. > Model information; O; Information about ML model capabilities of the VAL server/AIMLE client for split operation (e.g., identifiers, versions, etc.).. > Usage information; O; Information about usage capabilities of the VAL server/AIMLE client for split operation (e.g., inputs frequency/size, output frequency/size, etc.).. Expiration time; O; The proposed expiration time of the registration.. #### 8.14.3.9 Split operation node registration update response Table 8.14.3.9-1 shows the response sent by an AIMLE server after processing of split operation node registration update request. Table 8.14.3.9-1: Split operation node registration update response Information element; Status; Description. Successful response; O (NOTE); Indicates that the request was successful.. > Expiration time; O; The expiration time of the registration. To maintain an active registration, a registration update is required before the expiration time. If the Expiration time IE is not included, it indicates that the registration never expires.. Failure response; O (NOTE); Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: Either the Successful response or the Failure response is sent, but not both.. #### 8.14.3.10 Split operation node de-register request Table 8.14.3.10-1 shows the request sent by a VAL server/AIMLE client to the AIMLE server for split operation de-register request. Table 8.14.3.10-1: Split operation node de-register request Information element; Status; Description. Registration identifier; M; The identifier of the registration.. #### 8.14.3.11 Split operation node de-register response Table 8.14.3.11-1 shows the response sent by an AIMLE server after processing of split operation de-register request. Table 8.14.3.11-1: Split operation de-register response Information element; Status; Description. Successful response; O (NOTE); Indicates that the request was successful.. Failure response; O (NOTE); Indicates that the request failed.. > Failure cause; O; Indicates the failure cause.. NOTE: Either the Successful response or the Failure respone is sent, but not both..

### 7.21.2	Architecture Impacts

This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.21.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.21.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.22 	Solution #22: Sample Alignment Enablement for VAL Servers in Vertical Federated Learning

### 7.22.1 	Solution Description

#### 7.22.1.1 	General

This solution addresses key issue # 7 which identifies VFL sample alignment among open issues. In a VFL process, a VAL server may act as a VFL server (active party), coordinating training and inference by aggregating results from other VAL servers acting as VFL clients (passive parties). In this context, the sample space consists of user equipment (UEs), each represented by local identifiers specific to the VAL servers.

The VFL process begins by first aligning the samples among VFL participants so that inference and training are performed properly. This is performed without sharing the UE identifiers with other VFL participants. Each UE in the sample space may be identified with different identifications in each of the VFL participants such as different AF specific UE IDs or different public IP address of the UE. Therefore, the process of VFL sample alignment should be able to align the UEs that are involved in a VFL process without sharing the UE identifiers while resolving the potential difference in how the VFL participants identify the UEs.

This solution proposes to enable the AIMLE server to support sample alignment among VFL participants while preserving the privacy of the UEs and while resolving the potential differences in how each VFL participant identifies the UEs.

#### 7.22.1.2 	Procedure

Assumptions:

- VAL Server acting as VFL server coordinates other VAL servers acting as VFL clients to perform VFL training/inference.

- The sample space is associated with UEs which are locally identified by AF Specific UE IDs, IP addresses, or similar identifiers.

Pre-conditions:

- The VAL servers acting as VFL clients and VAL server acting as VFL server have registered with the AIMLE server, providing metadata including supported feature sets, and local UE identifiers.

- The AIMLE server maps each local UE identifier to a corresponding unified UE identifier (e.g., GPSI such as MSISDN) via interaction with the Core Network functions.

Editor's Note: Whether a new procedure is needed for registration or 3GPP TS 23.248 clause 8.4 should be updated for this purpose is FFS.

- The VFL server has discovered the VFL participants with the desired feature set and sample space. The AIMLE server can also assist in discovering VFL clients that match the required feature set and sample space using previously mapped UE identifiers.

Editor's Note: The detailed procedure for VFL participant discovery is FFS.

Figure 7.22.1.2-1: VFL process sample alignment procedure

- The VFL server sends request including the desired VFL clients, feature set, and sample space.

- The AIMLE server selects the required VFL clients fulfilling the required feature set and sample space. The UEs in the sample space of the VFL clients and VFL server is matched by using the Core Network functions to resolve local identifiers (e.g. AF specific UE ID, IP addresses) into a common UE identifiers (e.g, GPSI).

- The AIMLE server assigns a sample correlation ID for each UE in the sample space for consistent referencing during training and inference.

- The AIMLE server sends VFL training/inference initiation request to the selected VAL servers acting as VFL clients, including the assigned sample correlation IDs.

- The VAL servers acting as VFL clients send the initiation response to the AIMLE Server confirming participation to the VFL training/inference.

- The AIME server sends VFL training/inference response to the requesting VFL server, including the sample correlation IDs corresponding to the aligned UEs.

- The VFL server and the VFL clients proceed with the training/inference using the aligned sample space, with sample correlation IDs ensuring consistent sample alignment across particpants.

Editor's Note: Further details of this procedure and other modes of training/inference operations are FFS.

### 7.22.2 	Architecture Impacts

There is no architectural impact from this solution.

### 7.22.3	 Corresponding APIs

Editor's Note: The corresponding APIs are FFS.

### 7.22.4	 Solution Evaluation

Editor's Note: The evaluation of the solution is FFS.

# 8	Deployment scenarios

## 8.1	General

This clause will provide a general description of the deployment scenarios.

## 8.x	Deployment model #x: <Title>

Provide a description of the deployment scenarios.

# 9	Business Relationships

Provide a description of the involved business relationships.

# 10	Overall evaluation

This clause will provide evaluation of different solutions.

# 11	Conclusions

## 11.1	General conclusions

This clause will provide general conclusions for the study.

## 11.2	Conclusions of key issue #x

This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-04 | SA6#66 |  |  |  |  | TS skeleton | 0.0.0 |
| 2025-04 | SA6#66 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-251333, S6-251334, S6-251443, S6-251459, S6-251480, S6-251481, S6-251482, S6-251491 | 0.1.0 |
| 2025-05 | SA6#67 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-252512, S6-252427, S6-252494, S6-252509, S6-252430, S6-252495, S6-252480, S6-252536, S6-252434, S6-252537, S6-252437, S6-252520,S6-252438, S6-252439 | 0.2.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-253646, S6-253647, S6-253648, S6-253652, S6-253653, S6-253672, S6-253674, S6-253676, S6-253677, S6-253678, S6-253679, S6-253681, S6-253711, S6-253745, S6-253746, S6-253747, S6-253748, S6-253768, S6-253769, S6-253771. | 0.3.0 |