| 3GPP TR 23.700-98 V0.3.0 (2021-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Enhanced architecture for enabling Edge Applications; (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Key issues	9

4.1	Key issue #1: Enhanced notification service to the EEC	9

4.2	Key issue #2: Enablement of Service APIs exposed by EAS	10

4.3	Key issue #3: Enhancements to service continuity planning	10

4.4	Key issue #4: EDGE-5	11

4.5	Key issue #5: Alignment of EDGEAPP and ETSI MEC	12

4.6	Key issue #6: Edge services support across ECSPs	12

4.7	Key issue #7: Application traffic filter exposure	13

4.8	Key issue #8: EAS selection synchronization	13

4.9	Key issue #9: Enhancement of dynamic EAS instantiation triggering	14

4.10	Key issue #10: Support for roaming UEs	14

4.11	Key issue #11: ACR between EAS and Cloud Application Server	15

4.12	Key issue #12: EEL service differentiation	15

4.13	Key issue #13: Edge enabler layer support for EAS synchronization	16

4.14	Key issue #14: Application traffic influence for initially selected EAS	17

4.15	Key issue #15: Support of constrained devices for Edge	17

4.x	Key issue #x: <title>	17

5	Architectural requirements	18

5.1	General requirements	18

5.1.1	General	18

5.1.2	Requirements	18

5.2	Enablement of Service APIs exposed by EAS	18

5.2.1	General	18

5.2.2	Requirements	18

5.x	<specific requirements>	18

5.x.1	General	18

5.x.2	Requirements	19

6	Enhanced Application Architecture	19

6.1	Option #1: Roaming architecture	19

6.1.1	Architecture enhancements	19

6.1.1.1	Local breakout roaming architecture	19

6.1.2	Identities	19

6.1.3	Cardinality rules	19

6.x	Option #x: <title>	20

6.x.1	Architecture enhancements	20

6.x.2	Identities	20

6.x.3	Cardinality rules	20

7	Solutions	20

7.0	Mapping of solutions to key issues	20

7.1	Solution #1: Service provisioning via push notification	20

7.1.1	Architecture enhancements	20

7.1.2	Solution description	20

7.1.2.1	General	20

7.1.2.2	Procedure	21

7.1.3	Solution evaluation	22

7.2	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection	22

7.2.1	Architecture enhancements	22

7.2.2	Solution description	22

7.2.3	Solution evaluation	23

7.3	Solution #3: Service provisioning triggering via SMS over NAS	23

7.3.1	Architecture enhancements	23

7.3.2	Solution description	23

7.3.2.1	General	23

7.3.2.2	Procedure	23

7.3.3	Solution evaluation	25

7.4	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs	25

7.4.1	Architecture enhancements	25

7.4.2	Solution description	25

7.4.2.1	General	25

7.4.2.2	Procedure	26

7.4.3	Solution evaluation	27

7.5	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs	27

7.5.1	Architecture enhancements	27

7.5.2	Solution description	27

7.5.2.1	General	27

7.5.2.2	Procedure	27

7.5.3	Solution evaluation	29

7.6	Solution #6: ACR update in service continuity planning	29

7.6.1	Architecture enhancements	29

7.6.2	Solution description	29

7.6.2.1	ACR modification solution	30

7.6.2.1.1	EEC-based ACR modification procedure	30

7.6.2.1.2	EES-based modification procedure	30

7.6.3	Solution evaluation	31

7.7	Solution #7: EES monitors UE mobility for service continuity planning	31

7.7.1	Architecture enhancements	31

7.7.2	Solution description	31

7.7.2.1	General	31

7.7.2.2	Procedure	32

7.8	Solution #8: EAS Service API enablement using CAPIF	34

7.8.1	Architecture enhancements	34

7.8.2	Solution description	34

7.8.2.1	General	34

7.8.2.2	CAPIF operations in Edge Enabler Layer	35

7.8.3	Solution evaluation	36

7.x	Solution #x: <title>	36

7.x.1	Architecture enhancements	36

7.x.2	Solution description	36

7.x.3	Solution evaluation	36

8	Deployment scenarios	36

8.1	<Title>	36

9	Involved entities and relationships	37

9.1	<Title>	37

10	Overall evaluation	37

10.1	Architecture enhancements	37

10.2	Key issue evaluations	37

10.2.1	General	37

10.2.x	Key issue #x	37

11	Conclusions	37

11.1	Architecture enhancements	37

11.2	Solutions	37

Annex A (Informative): ETSI MEC and EDGEAPP system comparison	38

A.1 General	38

A.2 Service consumer and service provider	38

A.3 EAS/MEC application profile provisioning	38

A.4 EAS registration and EAS discovery	40

Annex B (informative): Change history	41

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Rel-17 TS 23.558 [2] defines the application layer architecture to enable edge applications over 3GPP networks. It includes features such as ECS discovery, service provisioning, EAS discovery, EEC/EAS/EES registration, network and Edge Enabler Layer capability exposure, service continuity support with seamless service continuity and EEC context continuity etc., along with cardinality rules, deployment options, involved relationships and mapping with ETSI MEC [3] and GSMA OP [4] architectures.

This TR documents a study on architecture and procedure enhancements to improve the Rel-17 architecture for enabling edge applications, and to support emerging industry requirements.

# 1	Scope

The present document is a technical report capturing the study on enhanced architecture for enabling edge applications over 3GPP networks. The study bases the enhancements on the work done in 3GPP TS 23.558 [2] and takes into consideration other related work done within and outside 3GPP i.e. ETSI MEC [3] and GSMA OP [4].

The aspects of the study include identifying new key issues, architecture requirements, related architecture enhancements and solutions, cardinality rules, deployment options, and involved entities and relationships to enhance the Rel-17 architecture for enabling edge applications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 23.558: "Architecture for enabling Edge Applications".

[3]	ETSI ISG MEC ETSI GS MEC 003, "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[4]	GSMA OPG.02: "Operator Platform Telco Edge Requirements", https://www.gsma.com/futurenetworks/wp-content/uploads/2021/07/GSMA-OPG-Telco-Edge-Requirements-2021.pdf

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TR 23.758: "Study on application architecture for enabling Edge Applications".

[7]	3GPP TR 23.721: "Study on Sponsored Data Connectivity Improvements".

[8]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[9]	3GPP TS 23.203: "Policies and Charging control architecture; Stage 2".

[10]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[11]	3GPP TS 23.503: "Policies and Charging control architecture; Stage 2".

[12]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[13]	ETSI GS MEC 010-2: "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[14]	ETSI GS MEC 011: " Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[15]	ETSI GS MEC 001: " Multi-access Edge Computing (MEC); Terminology ".

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1], 3GPP TS 23.558 [2] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.558 [2].

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1], 3GPP TS 23.558 [2] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1] and 3GPP TS 23.558 [2].

Abbreviation format (EW)

<ABBREVIATION>	<Expansion>

# 4	Key issues

## 4.1	Key issue #1: Enhanced notification service to the EEC

The EEC can be notified with the updated information for edge computing service by EES and ECS over EDGE-1 and EDGE-4, respectively. The notification services available to the EEC (i.e., service provisioning notification, EAS discovery notification, ACR information notification) are specified in release 17, but the details on the notification mechanism is not addressed. In this regard, additional ways (e.g. push notification mechanism and application triggering specified in 3GPP TS 23.501 [5], etc.) of providing the updates to the EEC in real time need to be studied.

Open issues:

1.	Whether and how the EEC acquires the notification target address or a notification channel URI to receive the notifications?

2.	Whether and how EEC, ECS and EES support push notification mechanism. Whether and what additional functional entity is necessary for this?

3.	How are the EEC subscriptions and/or notification targets treated in mobility scenario (e.g. during ACR scenario)?

4.	Whether and how to utilize application triggering method specified in 3GPP TS 23.501 [5] to provide notifications to the EEC?

## 4.2	Key issue #2: Enablement of Service APIs exposed by EAS

As specified in 3GPP TS 23.558 [2] (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES as specified in the clause 8.6 of 3GPP TS 23.558 [2] (Rel-17) and the capabilities provided by the 3GPP core network as specified in the clause 8.7 of 3GPP TS 23.558 [2] (Rel-17).

However, there are several use cases and requirements to exploit Service APIs exposed by EASs (provided by Application service providers) which can be invoked by the other EASs such as Cloud/Edge/Split Rendering in AR/VR use cases; and video content delivery exploiting separate services such as content caching, video encoding/decoding, and video analytics.

With the enablement of Service APIs exposed by EASs in the Edge Enabler Layer, there may expect several benefits to the involved business roles in the edge computing services. For example, Application service providers may simplify the Edge Application logic by composing the service components provided by the other EASs at the network edge; and open a new business to provide one or more service components to be executed and invoked by the other EASs at the network edge.

In order to enable the use of Service APIs exposed by EASs in the Edge Enabler Layer, there have been relevant studies in 3GPP TR 23.758 [6] (Rel-17), which result in Sol#15 as follows:

-	Solution #15: Edge Application Server's service APIs publish and discovery using CAPIF:

-	The Edge Enabler Server can support edge application (owned by 3rd party or by PLMN operator) access to the service APIs offered by other Edge Application Servers within and across the Edge Data Network by providing CAPIF functions in a distributed or centralized manner.

Based on the Rel-17 study result, this key issue focuses on addressing the following open issues.

Open issues:

1.	Identify any gaps in CAPIF to enable EAS Service APIs in the EDGEAPP architecture based on the Sol#15 of 3GPP TR 23.758 [6] (Rel-17).

-	In terms of e.g., service-specific attributes for API publish/discovery, API availability subscription/notification across EES/CCF

2.	If any, whether and how to enhance CAPIF capabilities to address the gap identified as above?

## 4.3	Key issue #3: Enhancements to service continuity planning

In 3GPP TS 23.558 [2], the service continuity planning was specified as part of the Edge Enabler Layer value-add features for supporting seamless service continuity, when information about planned, projected, or anticipated behaviour is available at EESs or provided by EECs.

This key issue studies potential enhancements to the service continuity planning feature, based on additional criteria for detecting a planned ACR, e.g. the network conditions / analytics monitoring, the DN performance monitoring, the expected/predicted UE route etc. This key issue also studies potential enhancements to the service continuity planning feature to allow the EEC to send a timely ACR request. Finally, the key issue studies scenarios when the planned UE behavior changes after the launch of the service and the ACR needs to be modified due to these changes, e.g. due to UE mobility change.

Open Issues:

1.	How to rely on the capability of EES/EEC to detect whether the UE moves to the predicted location or not for service continuity planning?

2.	Whether and how the EEL can support the determination of the ACR request trigger timing in case of service continuity planning?

3.	How to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility?

4.	Whether and what additional capability exposure is required from the 5GS (e.g. NWDAF, OAM) to enhance the service continuity planning?

5.	Potential impact on information exchanged between EAS and EEL.

6.	Potential impact on information to communicate within the EEL.

## 4.4	Key issue #4: EDGE-5

According to the latest version of the EDGEAPP specification in 3GPP TS 23.558 [2]:

"EDGE-5 reference point enables interactions between AC(s) and the EEC."

That specification also notes:

"NOTE:	Detailed specification of this reference point is out of scope of this release of this specification."

According to the latest version of the FS_EDGEAPP specification in 3GPP TR 23.758 [6]:

"The interactions between Application Client(s) and the Edge Enabler Client in the UE are supported by the EDGE-5 reference point. This reference point supports:

-	Obtaining information about Edge Application Servers that Application Client require to connect;

-	Notifications about events related to the connection between Application Clients and their corresponding Edge Application Servers, such as: when an Application Client needs to reconnect to a different Edge Application Server;

-	Providing Application Client information (such as its profile) to be used for various tasks such as, identifying the appropriate Edge Application Server instance to connect to; and

-	Provide the identity of the desired Edge Application Server to the Edge Enabler Client to enable it to use that identity as a filter when requesting information about Edge Application Servers."

The necessary functionality of the EDGE-5 interface needs to be studied for Release 18:

-	What functionality should an EEC provide to an AC (see the interactions copied from 3GPP TR 23.758 [6] and the procedures and information flows specified in 3GPP TS 23.558 [2])?

The following aspects may also need to be studied for Release 18 based upon the provided functionality of the EDGE-5 interface:

1.	Whether the cardinality as currently captured in 3GPP TS 23.558 [2] is to be modified:

"The following cardinality rules apply for EDGE-5:

a)	One AC may communicate with only one EEC; and

b)	One EEC may communicate with one or more AC(s) concurrently."

2.	Whether and how an AC can discover available EEC(s)?

3.	Whether mutual authentication and authorization between an AC and an EEC is necessary, and if so, how is that accomplished?

4.	What APIs should be exposed from an EEC to an AC to support that functionality?

5.	Whether a notification mechanism is necessary from an EEC to an AC?

6.	Whether and how an AC registers to an EEC?

7.	Whether and how an AC de-registers from an EEC?

8.	Whether and how an AC detects an abnormal termination of an EEC?

9.	Whether and how an EEC detects an abnormal termination of an AC?

10.	Whether user's consent is necessary to either AC or EEC operation, and if so, how is it provided?

NOTE 1:	The aspects of defining end-user consent/authorization over APIs and aspects of mutual authentication and authorization between an AC and an EEC are in the scope of SA3.

NOTE 2:	The aspects of the usage of end-user consent/authorization over APIs is in the scope of SA6.

## 4.5	Key issue #5: Alignment of EDGEAPP and ETSI MEC

As described in Annex C of 3GPP TS 23.558 [2] (Rel-17), both EDGEAPP and ETSI MEC can provide support for hosting different edge applications. According to Annex C: "Both EAS and MEC application are application servers and can provide similar application specific functionalities. EAS utilizes the services of EES as specified in this document whereas MEC application utilizes the services provided by MEC platform as specified in ETSI GS MEC 003." As discussed in Annex B.2 of draft GS MEC 003 [3] (v3.0.4), the EES and MEC platform can also be collocated in an implementation.

While the 3GPP TS 23.558 [2] and ETSI GS MEC 003 [3] provide an initial view about the alignment of the two platforms, this KI intends to address the following:

1.	Study and analyse different deployment options of EDGEAPP and ETSI MEC platforms.

2.	Functional architecture and gap analysis between EDGEAPP and ETSI MEC to determine complementary and possibly overlapping APIs and other related functionalities. Annex A captures a comparison of the architectures to facilitate the gap analysis.

3.	Recommendation and enhancements based upon the outcome of (1) and (2).

NOTE:	Backward compatibility is an important aspect of any recommendations & enhancements and will be considered during the study of this KI.

## 4.6	Key issue #6: Edge services support across ECSPs

An edge service or an EAS (e.g. V2X server) can be provided via different EDNs deployed by different ECSPs. Each ECSP may not have the required infrastructure to install the EAS in every EDN due to financial, regulatory and operation constraints. It is assumed that a UE can access the same edge service served by different EASs which are registered to different EESs and deployed by different ECSPs, which have a service level agreement to share edge services. These ECSPs can deploy EESs to serve different PLMNs or different coverages of the same PLMN. A typical scenario is depicted in Figure 4.6-1.

Figure 4.6-1 EAS deployed by different ECSPs

In Figure 4.6-1, the EAS2 resident in EDN1 and EDN2 provides the same service. The UE may be configured with the ECS1 configuration information (e.g., if the UE is a subscriber of ECSP_1). It is not clear how to provision the ECS2 configuration information, deployed by ECSP_2 (a partner of ECSP_1), to the UE when the UE is out of the service area of EAS2 in ECSP1 and cannot find a suitable EES within ECSP1 to discover and connect to EAS2.  The same issue exists when EAS2 becomes unavailable due to other reasons, e.g. overload, or in cases where ECSP_1 does not deploy EAS2 at all and relies on partner ECSP_2 to provide the edge service.

Besides, the UE may have already accessed the EAS2 in the EDN1 and is getting service from that EAS. In that case, it is not clear how to support service continuity due to UE mobility when the UE moves out of the service area of the EAS2 in EDN1 and goes to the service area of the EAS2 in EDN2.

The following study is needed:

1.	Identify potential enhancements to the existing architecture defined in Rel-17 to enable inter-ECSP interactions.

2.	Study potential impact to support ECS discovery and service provisioning based on UE location.

3.	Whether and how EEC registers with an EES deployed by a partner ECSP?

4.	Study potential impact to support service continuity.

5.	How is EEC context continuity maintained across ECSPs with or without ACR?

## 4.7	Key issue #7: Application traffic filter exposure

3GPP TS 23.558 [2] has specified EDGE-3 exposure with different APIs. The session with QoS API provides the capability for the EAS to influence the QoS for the application traffic via EES. The ACR management event API supports "User plane path change", "ACR monitoring" and "ACR facilitation" and all events support to detect user plane path change for the application traffic.

The current definition of session with QoS API and ACR management event API only supports simple IP flow description. E.g. in 3GPP TS 23.558 [2], table 8.6.6.3.2-1 IP flow description identifies the application traffic by 3 or 5 tuples.

Table 8.6.6.3.2-1: Session with QoS create request

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| Security credentials | M | Security credentials of the EAS |
| UE IP address (NOTE 1) | O | The UE IP address. |
| UE ID (NOTE 1) | O | The identifier of the UE (i.e. GPSI) |
| UE Group ID (NOTE 1) | O | Identifies a group of UEs (i.e. internal group ID or external group ID) |
| IP flow description | M | The IP flow description for the application traffic. |
| Requested QoS reference (NOTE 2) | O | Refers to pre-defined QoS information for the data session between AC and EAS (NOTE 3). |
| … | … | … |

Only supporting IP flow description in EDGE-3 exposure APIs is not enough. EPS already supported the application traffic (e.g. encrypted application traffic) detection by more filters than IP flow description for the application (e.g. considering in Rel-14 study 3GPP TR 23.721 [7]) and more traffic filters are supported via PFD management procedure as described in 3GPP TS 23.682 [10] and 3GPP TS 23.203 [9], and such capability is also supported in 5GS in 3GPP TS 23.502 [8] and 3GPP TS 23.503 [11] correspondingly. The EEL should provide the same level of traffic filter for identifying the application traffic.

Open issues:

1.	How to support more application traffic filter for session with QoS API.

2.	How to support more application traffic filter for ACR management event API.

## 4.8	Key issue #8: EAS selection synchronization

Currently, EAS discovery may result in multiple EASs being discovered for a specific AC. The discovery request may trigger at the EES operations such as dynamic instantiation (3GPP TS 23.558 [2] clause 8.12). However, the EEC may select only one or some of the discovered EASs to enable AC communications or may not begin communications right after a discovery. For a discovered but unselected EAS, operations such as EAS instantiation or state change are unnecessary and inefficient,

From a different perspective, the registration procedure results in EEC context establishment at EES, with associated service session management. The EES process for determining EAS selection for service sessions is not specified, and there is no method to synchronize this information with the EEC. Moreover, following a registration the EEC does not have EAS endpoint information which could be leveraged for initiating service sessions, although the registration results in reservation of resources in the EDN.

In another case, many IoT devices are configured for bursty communications at large intervals of time, without service continuity requirements and with many semi-static (e.g. provisioned over-the-top) parameters. For example, a set of EASs may be maintained at the EEC with the expectation of providing signalling optimizations. However, this EEC information cannot be currently leveraged by the system as intended.

While basic edge functionality can be enabled assuming ideal implementations of an algorithm through which the EES determines that a registered EAS is providing services to an AC, inefficiencies in EAS capability use and enabling service session functionality immediately after registration remain. Hence, it is required to study:

1.	How to enable the EES to accurately determine the EAS(s) capabilities needed by EEC for service sessions in order to perform optimal EAS instantiation operations.

2.	Whether and how the service session communications between ACs and EASs can be enabled by the EEC as soon as the EDN capabilities are available after registration.

3.	Whether and how to enable the EES to leverage pre-existent EAS information at the EEC in order to enable service session communications efficiently for IoT devices.

## 4.9	Key issue #9: Enhancement of dynamic EAS instantiation triggering

In order to ensure efficient utilization of EDN resources for EAS deployment, it should be possible to have the proper number of EAS instances in the EDN to accommodate the load for applications. The dynamic EAS instantiation triggered by the EES is supported in release 17, but further details are not addressed. The EES may invoke EAS dynamic instantiation triggering to the EAS management system, e.g,, for considering the service load/capacity of EAS (e.g., number of service session); and for considering the EEC's requesting service characteristics (e.g., location, latency). In this regard, the followings need to be studied further.

Open issues:

1.	What kind of information can be acquired by edge enabling layer and utilized by an EES to decide to trigger dynamic EAS instantiation and which entities can provide such information to an EES

2.	Whether and how to support dynamic EAS termination triggering in order to enable dynamic scaling of EAS (i.e., scale in as needed).

Editor's note:	The aspects of the interaction between EES and EAS management system should be consulted with SA5.

## 4.10	Key issue #10: Support for roaming UEs

When a UE is roaming in VPLMN, EDN configuration information for edge computing service in VPLMNs may not be available at all ECS deployed in HPLMN (termed H-ECS. The EEC in the UE thus needs to obtain information for V-ECSs (ECS available in VPLMN) to obtain service provisioning information in VPLMN based on the business relationship between HPLMN operator, VPLMN operator, and related ECSP(s).

In one scenario, the EEC in the roaming UE needs to discover the availability of edge computing services via ECS(s) available in VPLMN.

In Rel-17, ECS discovery based on VPLMN ID addressed in clause 8.3.2 of 3GPP TS 23.558 [2] do not cover some cases, e.g. when there are multiple available ECSs via the VPLMN for the roaming UE. Therefore, it is required to study ECS discovery in VPLMN and subsequent service provisioning for all relevant deployment models.

Additionally, it is required to clarify how an EEC hosted in the roaming UE can be authenticated and authorized to access the edge computing services available in the VPLMN. The related requirement is described in GSMA OPG as follows: "Access of roaming subscribers to edge applications in the visited network shall be subject to authorisation by the subscriber's Home OP and the Visited OP". Note that EEC authentication/authorization in Rel-17 is not clarified in roaming situation. It is thus required to study the architectural support necessary for SA3-defined procedures for EEC authentication/ authorization in roaming scenarios.

The following aspects shall be studied to support roaming UEs:

1.	Roaming-related deployment scenarios (if any) to be supported, and which may be differentiated by: (i) the relationship between the ECS provider and the PLMN operators, (ii) whether connectivity to an ECS can be established in both involved PLMNs.

2.	How the EEC in the roaming UE knows the availability of ECS(s) and/or EES(s) and discovers them in the VPLMN?

3.	Whether and how edge computing service continuity is supported when transitioning between an HPLMN and VPLMN.

4.	How to support authentication and authorization for an EEC hosted in the roaming UE.

5.	Whether and how to support topology hiding on inter-PLMN/ECSP interfaces on the edge enabler layer.

## 4.11	Key issue #11: ACR between EAS and Cloud Application Server

When a UE moves to a new location, different EASs or Cloud Application Server (CAS) can be more suitable for serving the ACs in the UE. Such transitions can result from a non-mobility event also, requiring support to maintain the continuity of the service.

This key issue is to support service continuity for ACs in the UE to minimize service interruption while switching the application server between Edge and Cloud. To support service continuity, the application context is transferred between EAS and CAS.

Rel-17 Edge Computing work is limited to the service continuity between the EAS(s) and identified several scenarios for service continuity. Detailed study is required to enable service continuity between EAS and CAS, covering the following open issues:

1.	Whether and how to detect that ACR is required between EAS and CAS

2.	Whether and how to decide that ACR is required between EAS and CAS

3.	Whether and how to perform ACR between EAS and CAS

4.	Whether and how to perform post ACR actions

5.	Whether EEL is required on the cloud deployment and what are the potential impacts to the CAS architecture

6.	Whether and what are the impacts of CAS initiated ACR.

## 4.12	Key issue #12: EEL service differentiation

The service differentiation in Rel-17 is very general and not detailed (e.g. ECS use local policy to determine service provisioning).  Details to enable an ECSP to provide different service quality levels should be specified. Such as a principle based on the role of service consumer. For example, a premium user must get the nearest available edge, or there should be certain applications available only to the premium users and so on.

Open issues:

-	What service differentiation should be enabled by the EEL?

-	Which procedure should be enhanced to support service differentiation?

-	Which functional entity is responsible for defining or managing the service differentiation?

## 4.13	Key issue #13: Edge enabler layer support for EAS synchronization

It is possible for the ASP to provide EAS (with same service) in different EDNs. For example, a gaming service provider may have deployed game servers in different EDNs to serve its users. It is required for an EAS to synchronize the particular communication session among other EAS(s). In figure 4.13-1, consider:

1)	a multi-user gaming session-1 is active between 5 gaming users where 2 users are in EDN1 served by EAS-1X (ASP1), while other 3 users are in EDN2 served by EAS-2X (ASP1);

2)	at same time (when gaming session-1 is active), another gaming session-2 is also active between different set of users, where some users are in EDN2 served by EAS-2X (ASP1), while other users are in EDN3 served by EAS-3X (ASP1).

It is required for EAS(s) to synchronize among each other for specific communication session (e.g. multi-user game session) in order to serve the users properly. It is required to study how Edge enabler layer can support EAS from EDN1 to find other EAS(s) in other EDN(s) to synchronize with.

Figure 4.13-1 EAS (with same service) deployed by same ASP in different EDNs

Further, referring to 3GPP TS 23.558 Annex A.4, the application architecture supports SEAL application server functions and Application Enabler Server functions available at the edge. While SEAL application server functions can be made available as an EAS at the edge, it is also possible that certain SEAL application server functions are available either or both at the edge and at the cloud. When the server functions of an application are available both at the edge and at the cloud, there may be a need for interaction between the two corresponding application servers.

This key issue is to support interaction between the two application server functions deployed at the edge or both at the edge and at the cloud.

Open issues:

1)	Whether and how to enable EAS to find other EAS(s) with multi-user communication session to synchronize?

2)	Whether and how to enable EAS to find other interested EAS(s) with specific service to synchronize?

3)	Whether and how to enable EAS to discover and interact with another application server function deployed on the cloud for context synchronization?

4)	Whether and how edge enabler layer could provide support to EAS synchronization?

## 4.14	Key issue #14: Application traffic influence for initially selected EAS

The application traffic is between the AC and EAS and 3GPP CN provides the underlying connectity for the application traffic. The EEC starts with EAS discovery first in order to offer EAS(s) to AC to start communication with the initially selected EAS. Later on, if the EAS relocation criteria is met, the EAS relocation procedure happens, which consists of EAS discovery, application traffic influence, application context transfer and AC communication with the new EAS.

Currently, it is not clear in the specification how to influence the application traffic with best optimal user plane routing when initial EAS discovery is completed in the application layer.

Open issues:

-	Whether and how the EEL can be involved to influence the application traffic routing in AC communication with the initially selected EAS.

## 4.15	Key issue #15: Support of constrained devices for Edge

Energy efficiency requirements are relevant for battery driven low-power IoT devices. Those devices may require edge computing services in a local DN for low-latency and employ the SA6 specified edge enablers.

Clause 6.15 of 3GPP TS 22.261 [12] has provided requirements for energy efficiency for devices, for instance: the 5G system shall support UEs using small rechargeable and single coin cell batteries; and shall support mechanisms to improve battery life for a UE over what is possible in EPS.

In the existing EDGEAPP architecture as described in TS 23.558 [2], many procedures and services are defined for EEC to interaction with EES/ECS. It is needed to study what impact the support for constrained UE may have on EDGEAPP architecture and what procedure can be improved.

Open issues:

-	Whether there are any impacts on the EDGEAPP architecture for constrained UE.

-	Whether and how the existing EDGEAPP architecture and procedures, for constrained UE to network communication (i.e. EDGE-1 and EDGE-4), e.g. can be improved to reduce power consumption.

## 4.16	Key issue #16: support of NAT deployed within the edge data network

In operational deployments of cloud infrastructures including at the edge, Network Address Translation (NAT) are oftentimes deployed.

Such an operational constraint will prevent the EES to have an IP address of the UE that is known by the underlying 3GPP network.

Open issue:

-	How the EES can access 3GPP network services pertaining to a UE when the edge data network employs Network Address Translators (NATs).

Editor's Note: Whether EAS requires to be addressed in the key issue is FFS

## 4.x	Key issue #x: <title>

This clause describes the key issue with a suitable title. Please provide a high-level description of the key issue along with a list of open issues.

# 5	Architectural requirements

## 5.1	General requirements

### 5.1.1	General

This clause provides a basic description of the general architectural requirements.

### 5.1.2	Requirements

This clause lists the general requirements.

## 5.2	Enablement of Service APIs exposed by EAS

### 5.2.1	General

This clause specifies the requirements for EAS Service API enablement in the EDGEAPP architecture to address the open issues of the KI#2. In order to fulfil the specific requirements in this clause, the Solution #X: "EAS Service API enablement using CAPIF" is further specified.

### 5.2.2	Requirements

This solution is based on the following architectural requirements to support EAS Service APIs in the EDGEAPP Rel-18 architecture.

1)	EAS capability exposure

-	The application layer architecture shall support exposure of EAS's capabilities to the other EASs.

2)	EAS Service API publication

-	The application layer architecture shall support EAS to publish its exposing Service API information to EES

-	The application layer architecture shall support EAS to update the published EAS Service API information on the EES.

3)	EAS Service API discovery

-	The application layer architecture shall provide mechanisms for an EAS to discover available EAS Service APIs.

4)	Subscription service

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in dynamic information of EAS Service APIs from an EES.

-	The application layer architecture shall provide subscription and notification mechanisms enabling an EAS to receive changes in availability of EAS Service APIs from an EES.

## 5.x	<specific requirements>

### 5.x.1	General

This clause provides a basic description of specific architectural requirements.

### 5.x.2	Requirements

This clause lists specific requirements.

# 6	Enhanced Application Architecture

## 6.1	Option #1: Roaming architecture

This cluase describes the architecture for roaming UEs, addressing Key Issue #10.

Editor's note:	Whether home-routed roaming architecture is supported is FFS.

### 6.1.1	Architecture enhancements

#### 6.1.1.1	Local breakout roaming architecture

This architecture uses ECSs provided in HPLMN and VPLMN, in which the EEC in the UE obtains services from V-ECS and V-EES. In this architecture, the H-ECS is associated with HPLMN, while the V-ECS and the EDN which the UE accesses is associated with VPLMN. Figure 8.1.2-1 shows the architecture for this model.

NOTE:	H-ECS and V-ECS can be provided by the same ECSP.

Figure 6.1.1.1-1: Local breakout roaming architecture

### 6.1.2	Identities

None.

### 6.1.3	Cardinality rules

None.

## 6.x	Option #x: <title>

This clause provides an enhanced application architecture based on Rel-17 architecture.

### 6.x.1	Architecture enhancements

This clause describes the new functional elements and interfaces proposed by this option.

### 6.x.2	Identities

This clause describes the new identities defined for the functional enhancements proposed by this option.

### 6.x.3	Cardinality rules

This clause describes the new cardinality rules defined for the functional enhancements proposed by this option.

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 | KI #8 | KI #9 | KI #10 | KI #11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 | X |  |  |  |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  | X |  |  |  |  |
| Sol #4 |  |  |  |  |  | X |  |  |  | X |  |
| Sol #5 |  |  |  |  |  | X |  |  |  | X |  |
| Sol #6 |  |  | X |  |  |  |  |  |  |  |  |
| Sol #8 |  | X |  |  |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |  |  |  |

## 7.1	Solution #1: Service provisioning via push notification

### 7.1.1	Architecture enhancements

None.

### 7.1.2	Solution description

#### 7.1.2.1	General

The following solution corresponds to the key issue #1 on enhanced notification service to the EEC in clause 4.1.

In this solution, push notification mechanism is utilized to enhance the service provisioning procedure. It is assumed that at least one push server is available to ECS and the UE has push function supporting the interaction with the push server.

#### 7.1.2.2	Procedure

Pre-conditions:

1.	The UE supports push notification service and the associated push server can be accessed by the ECS.

2.	The address of push server is pre-configured in the Push function.

Figure 7.1.2.2-1: Service provisioning via push notification

1.	The EEC registers with the push function within the UE. The EEC acquires a push token and push server information from the push function.

NOTE:	The push server provides the push function in the UE with a push token, which is delivered to the EEC.

2.	The EEC sends a service provisioning subscribe request to the ECS. The service provisioning request includes push token, push server information (e.g. address) in addition to information elements in clause 8.3.3.3.4 of 3GPP TS 23.558 [2] v17.0.0. The push server address is included as Notification Target Address.

3.	Upon receiving the request, the ECS performs an authorization check as in clause 8.3.3.2.3.2 of 3GPP TS 23.558 [2] and further verify if push notification can be used. If the request is authorized and push server can be used for notification, the ECS creates and stores subscription resource for service provisioning.

4.	If the processing of the request was successful, the ECS responds with a service provisioning subscription response.

5.	If an event occurs at the ECS that satisfies trigger conditions for updating service provisioning of a subscribed EEC and the corresponding subscription is for push notification, the ECS sends push notification request to the push server that is identified by the push server information provided by the EEC in the step 2. The push notification request sent from the ECS to the push server contains the push token, EEC information (e.g. identification or port ID) and service provisioning notification message.

6.	The push server sends the service provisioning notification message to the EEC via the push function, which in turn and delivers the notification message to the corresponding EEC.

### 7.1.3	Solution evaluation

The proposed solution addresses Key Issue #1. The solution is based on Rel-17 service provisioning subscribe procedure and the push notification mechanism supported in a UE. This approach does not require EEC to persistently keep the session with ECS for receiving notification message. Instead, the UE reuses the session kept by the push function and push notification server.

This solution does not introduce impact on Rel-17 architecture.

## 7.2	Solution #2: Traffic filter support for EDGE-3 API addressing application traffic detection

### 7.2.1	Architecture enhancements

None.

### 7.2.2	Solution description

The EAS can provide the domain name to the EES as traffic descriptor and EES can further provision the domain name to the 3GPP CN. The following figure 7.2.2-1 depicts the solution sketch for the Session with QoS create operation with bold text showing the enhancement to the existing procedure in 3GPP TS 23.558 [2], clause 8.6.6.2. Such solution allows the EAS to provide fruitful traffic filters to be applied for the intended session with requested QoS.

Figure 7.2.2-1: Session with QoS API: create operation

In step 1, the EAS requests establishment of a data session between the AC and the EAS with a specific QoS, the EAS sends the domain name as traffic descriptor which also indicates the applicable protocol and matching criteria (e.g. TLS SNI). In step 2a, the EES invokes the PFD management procedure with the 3GPP CN as described in 3GPP TS 23.682 [10] and 3GPP TS 23.502 [8] with an application id that may be derived from the EASID according to local policy. Further the EES provides the same application id for requesting data session with specific QoS in step 2c or step 4.

NOTE:	PFD management is optionally supported in MNO, if EES cannot invoke step 2a, it responds EAS with appropriate error.

If the EAS provides only IP flow description as the traffic filter, the EES can provide the application id (which may be derived from the EASID) to the 3GPP CN when requesting data session with specific QoS and rely on further traffic classification mechanism in MNO to apply specific QoS for the application traffic. In that case, the EES skips the PFD management procedure in step 2a.

Similar approach can be applied for the ACR management event API for EAS to consume EES service.

The EAS can also send other specific filters (e.g. URI) to the EEL allowing a fruitful application traffic detection in the 3GPP CN, such filters will be used by the EES in the PFD management procedure in step 2a.

### 7.2.3	Solution evaluation

This solution address KI#7. It enhances the existing Session with QoS API and ACR management event API in 3GPP TS 23.558 [2] with support for traffic filters more than IP flow description.

## 7.3	Solution #3: Service provisioning triggering via SMS over NAS

### 7.3.1	Architecture enhancements

None.

### 7.3.2	Solution description

#### 7.3.2.1	General

The following solution corresponds to the key issue #1 on enhanced notification service to the EEC.

In this solution, application triggering (device triggering) via SMS over NAS method specified in 3GPP TS 23.501 [5] is utilized to provide the updated EDN configuration information to the EEC.

The EEC indicates to the ECS that SMS over NAS is supported. Then, the ECS checks if the SMS over NAS can be utilized and perform EEC triggering by using SMS over NAS

#### 7.3.2.2	Procedure

Pre-conditions:

1.	The EEC is able to check if the UE supports the SMS over NAS; and

2.	The ECS is allowed to use Nnef_Trigger_Delivery service.

Figure 7.3.2.2-1: EEC triggering via SMS over NAS to perform service provisioning

1.	The EEC checks if the UE supports the SMS over NAS. If the SMS over NAS is supported and allowed in the UE, the EEC sends an indication of SMS over NAS supported to the ECS. This indication may be included in the service provisioning request message and sent to the ECS.

Editor's note:	It is FFS how the EEC checks if the UE supports the SMS over NAS and whether the way is in the scope of SA6.

2.	The ECS acknowledges to the EEC by sending a message including whether the EEC triggering via SMS over NAS is available for service provisioning or not. When the EDN configuration information in the ECS is updated, the ECS triggers EEC to initiate service provisioning request by utilizing the SMS over NAS as described in the below figure 7.3.2.2-2.

Figure 7.3.2.2-2: EEC triggering via SMS over NAS to perform service provisioning

1.	The ECS determines to perform EEC triggering via SMS over NAS for service provisioning (e.g. when the EDN configuration information is updated).

2.	The ECS invokes Nnef_Trigger_Delivery service to send a trigger delivery request message to the NEF as described in clause 4.13.2 of 3GPP TS 23.502 [8]. The message includes indication to trigger service provisioning, EEC ID, UE ID, and ECS information (e.g. address).

3.	The NEF delivers the triggering information via MT SMS Delivery procedure as specified in TS 23.502 [8]. The indication to trigger service provisioning, EEC ID, ECS information are included as the triggering payload in the triggering information

4.	The NEF sends Nnef_Trigger_Delivery service response message to the ECS to inform that the ECS-provided information in step 2 is successfully delivered to the SMS-SC as in clause 4.13.2 of 3GPP TS 23.502 [8].

5.	The NEF sends Nnef_Trigger_Delivery Nofity message to the ECS to inform that the SMS message is successfully delivered to the UE as in clause 4.13.2 of 3GPP TS 23.502 [8].

6.	The UE receives the SMS message containing the ECS-provided triggering information (indication to trigger service provisioning, EEC ID, ECS information) from the 5GC via Mobile Terminated SMS Delivery as in clause 4.13.3 of 3GPP TS 23.502 [8] and the EEC is triggered to perform service provisioning request toward the ECS identified by the ECS information contained in the SMS message.

7.	The EEC may send a service provisioning request to the ECS identified by the triggering information contained in the SMS.

Editor's note:	It is FFS if this solution conforms to the notion of a notification service to the EEC.

### 7.3.3	Solution evaluation

TBD.

## 7.4	Solution #4: ECS discovery through serving ECS to support edge services across ECSPs

### 7.4.1	Architecture enhancements

None.

### 7.4.2	Solution description

#### 7.4.2.1	General

The following solution corresponds to the key issue #6 on edge services support across ECSPs in clause 4.6 and to the key issue #10 on support for roaming UEs.

In this solution, it is assumed that the ECSP1 and ECSP2 have a service level agreement to share edge services. If the ECS1 can not discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES based on UE location and provides it to the requesting EEC or EES.

In a roaming scenario, after a UE selects a PLMN and performs the registration procedure, the EEC gets ECS information to use edge computing service in the VPLMN as follows:

-	The EEC may have been pre-configured or provisioned with V-ECS information.

-	The EEC may try to send a service provisioning request message including service PLMN ID to the H-ECS and retrieves V-ECS information or, if possible, EDN configuration information for VPLMN. The EEC may perform service provisioning procedure with the the V-ECS if the H-ECS provides V-ECS information to the EEC as show in figure 7.x.2.2-1.

-	The EEC may try to derive V-ECS address information from the VPLMN identifier. For example, an EEC can combine the VPLMN identifier and an ECSP code (or ECSP ID) assigned to an edge computing service provider (e.g. "ECS <ECSP-ID/CODE>.ECS.MNC <MNC> .mcc <MCC> .3gppNetwork.org ").

Editor's note:	It is FFS whether and how the precedence among the above method for ECS discovery is defined.

#### 7.4.2.2	Procedure

In this solution, when the ECS1 receives the request of EES from the EEC or source EES, the ECS1 discovers another ECS2 which may have suitable EES and responds with the ECS2 information. After that, the EEC or source EES sends the request of EES to the ECS2.

Pre-conditions:

1.	ECSP1 and ECSP2 have a service level agreement to share edge services.

2.	The EEC has a business relationship/subscription to the ECSP1.

3.	The EEC has ECS1 address information and can access to the ECS1 (in roaming scenario, ECS1 and ECS2indicate the H-ECS and the V-ECS, respectively).

Figure 7.4.2.2-1: Solution X for edge services support across ECSPs

1.	The ECS1 receives request for obtaining EES information, this procedure may be triggered by following events:

1a.	The EEC sends a service provisioning request to the ECS. It is assumed that the EEC has been pre-configured or has provisioned with the address (e.g. URI) of the ECS1. The request may include the UE location. For roaming scenario, the request may also include serving PLMN ID of the UE hosting the EEC.

1b.	The S-EES (EES1) sends the retrieve EES request to the ECS in order to identify the T-EES which has an EAS available to serve the UE.

2.	If the request does not contain the UE location information, the ECS1 interacts with 3GPP core network to retrieve the UE location. If the ECS1 cannot discover a suitable EES to serve the UE at the received or retrieved UE location based on the received information (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers potential ECSs which may have suitable EES based on the information such as the UE location.

The ECS1 may determine a potential ECS information (e.g. address, endpoint or service API information) based on pre-configuration or may discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222.

For roaming scenario, if the request does not contain the serving PLMN ID, the ECS1 may interacts with 3GPP core network to retrieve serving PLMN ID. The H-ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The ECS1 discovers the potential ECSs, e.g. ECS2 which have suitable EES based on the serving PLMN ID. Optionally, the ECS1 may send a request to ECS2 to verify whether the ECS2 is available. If yes, ECS2 returns the success response to the ECS1. Otherwise, a failure response/code is returned.

Editor's note:	In the roaming scenario, it is FFS how an H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the related roaming agreement on edge computing service.

Editor's note:	It is FFS whether and how ECS1 filters the list of the discovered ECSs.

3. ECS1 sends a response message including the list of ECSs information and failure cause indicating redirection to another ECS (for service provisioning request or retrieve EES request) to the requester of step 1 if the ECS1 cannot discover a EES:

3a.	In response to the request in step 1a, the ECS1 sends the response message to the EEC.

3b.	In response to the request in step 1b, the ECS1 sends the response message to the S-EES.

4.	The EEC or source EES can send the request (e.g. resends a service provisioning request or retrieve EES request) to one of the ECSs received from the ECS1 e.g. the ECS2. Correspondingly, the EEC or source EES receives response from the ECS2.

### 7.4.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

## 7.5	Solution #5: ECS enhancement to discover EESs via other ECSs to support edge services across ECSPs

### 7.5.1	Architecture enhancements

A new reference point is required between ECSs.

### 7.5.2	Solution description

#### 7.5.2.1	General

The following solution corresponds to the key issue #6 on edge services support across ECSPs in clause 4.6.

In this solution, it is assumed that the ECSP1 and ECSP2 have a service level agreement to share edge services. If the ECS1 cannot discover a suitable EES to serve the UE at the current location (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the ECS1 discovers another ECS2 which may have suitable EES and discovers the EES via the ECS2.

#### 7.5.2.2	Procedure

In this solution, when the ECS1 receives the request of EES from the EEC / source EES, the ECS1 discovers another ECS2 which may have suitable EES and discovers the EES via the ECS2. The ECS1 then provides the EES information to the EEC / source EES in the response.

Pre-conditions:

1.	ECSP1 and ECSP2 have a service level agreement to share edge services.

2.	The EEC has a business relationship/subscription to the ECSP1.

3.	The EEC has ECS1 address information and can access to the ECS1 (in roaming scenario, ECS1 and ECS2 indicate the H-ECS and the V-ECS respectively).

Figure 7.5.2.2-1: Solution X for edge services support across ECSPs

1.	The ECS1 receives request for obtaining EES information, this procedure may be triggered by following events:

1a.	The EEC sends a service provisioning request to the ECS. It is assumed that the EEC has been pre-configured or has provisioned with the address (e.g. URI) of the ECS1. The request may include the UE location. For roaming scenario, the request may also include serving PLMN ID of the UE hosting the EEC.

1b.	The S-EES (EES1) sends the Retrieve EES request to the ECS in order to identify the T-EES which has an EAS available to serve the UE.

2.	If the request does not contain the UE location information, the ECS1 interacts with 3GPP core network to retrieve the UE location. If the ECS1 cannot discover a suitable T-EES to serve the UE at the received or retrieved UE location based on the received information (e.g., all the EESs registered on the ECS1 do not cover the given UE location), the  ECS1 discovers another ECS2 which may have suitable EES based on the information such as the UE location. The ECS1 may determine the ECS2 information (e.g. address, endpoint or service API information) based on pre-configuration or may discover service API information exposed by that ECS via CAPIF discovery procedure as specified in TS 23.222.

For roaming scenario, if the request does not contain the serving PLMN ID, the ECS1 may interacts with 3GPP core network to retrieve serving PLMN ID. The H-ECS checks if the edge computing service for the EEC can be supported in the VPLMN identified by serving PLMN ID according to the roaming agreement with VPLMN operator for given ECSPs. The ECS1 discovers the ECS2 which have suitable EES based on the serving PLMN ID.

Editor's note:	In the roaming scenario, it is FFS how an H-ECS can check if the edge computing service for the EEC can be supported in the VPLMN based on the related roaming agreement on edge computing service.

3.	The ECS1 further discovers a target EES whose service area can cover the UE location via the ECS2. The ECS1 may interact with the ECS2 directly or indirectly via e.g. a Federation manager.

4.	ECS sends the response to the requester of step 1:

4a.	In response to the request in step 1a, ECS1 sends the EES information to the EEC.

4b. In response to the request in step 1b, ECS1 sends the Retrieve EES response to the requesting EES (EES1).

### 7.5.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

## 7.6	Solution #6: ACR update in service continuity planning

### 7.6.1	Architecture enhancements

Enhancement of the service continuity planning capability is expected to support update of ACR. As can be seen at the Figure 7.6.1-1, this solution proposes the ACR update capabilities as enhancements after the ACR launch to deal with UE behavior changes. This includes a Detection entity, a Decision Update entity and an ACR update execution entity. These entities can be different based on the scenarios identified in TS 23.558, clause 8.8.2.

Figure 7.6.1-1: high level illustration of proposed service continuity planning enhancements

### 7.6.2	Solution description

The solution addresses the key issue #3 and the open issue on how to deal with scenarios when the ACR needs to be modified, e.g. due to UE mobility.

The solution can have two different solution variants based on the type of service continuity planning update. One possible solution is to modify the ACR service after the launch (described in 7.6.2.1) and another possible solution is the pause of the ACR services for a pre-defined time or till the detection entity decides to resume the ACR.

Editor's note:	It is FFS to capture the ACR pause solution variant and whether this can be a standalone procedure or incorporated within the ACR modification procedure.

#### 7.6.2.1	ACR modification solution

##### 7.6.2.1.1	EEC-based ACR modification procedure

Pre-conditions:

1.	The ACR has been launched.

Figure 7.6.2.1.1-1: EEC-based ACR modification procedure

1.	The EEC detects a change of the expected UE behaviour. In particular, it may receive from the AC (over EDGE-5) an indication that the expected/predicted UE location and/or mobility changed. Such message can include the current and new expected/predicted location or current and new expected mobility/speed/direction/velocity, and/or an expected change of the UE route/trajectory.

2.	The EEC identifies that one or more ACR updates are needed based on the change of the UE behaviour and decides the type of the ACR update to be an ACR modification.

3.	The EEC sends an ACR modification request to the S-EES or T-EES (for EEC executed ACR via T-EES scenario) to indicate an ACR modification and to provide the updated parameters, such as the expected completion time. The request also includes the necessary parameters (e.g. IDs) to indicate the ACR that is requested to be updated.

4.	S-EES or T-EES determines the ACR to be modified based on the request in step 3. The ACR modification, is executed by the execution entity.

Editor's note:	The detailed procedure for step 4 is FFS.

5.  The S-EES (or T-EES for EEC executed ACR via T-EES scenario) sends an ACR modification response to the EEC to notify on the result.

6.	The EEC may optionally provide a notification to the AC (over EDGE-5) to inform on the ACR modification result.

##### 7.6.2.1.2	EES-based modification procedure

Pre-conditions:

1.	The ACR has been launched.

Figure 7.6.2.1.2-1: S-EES-based ACR modification procedure

1.	The S-EES detects a change of the expected UE behaviour. In particular, S-EES acting as AF, may receive a UE location report or a monitoring event report from 5GC (assuming that S-EES has subscribed to consume 5GC services like LCS or NEF monitoring events). Such UE location report or monitoring event report may help indicating that the UE is not going to be at the predicted location at the given time and is expected to deviate by the original planning.

2.	The S-EES identifies that one or more ACR updates are needed based on the information on the change of the UE behaviour. The S-EES then decides for each ACR, the type of the ACR update to be an ACR modification and the parameters that need to be updated, such as the expected completion time.

3.	S-EES or T-EES determines the ACR(s) to be modified based on the decision in step 2. For each ACR identified to be updated in step 2, the ACR modification is executed by the execution entity.

Editor's note:	The detailed procedure for step 3 is FFS.

4.  The S-EES sends an ACR modification notification to EEC to notify on the result.

5.	The EEC may optionally provide a notification to the AC (over EDGE-5) to inform on the ACR that is modified.

### 7.6.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.7	Solution #7: EES monitors UE mobility for service continuity planning

### 7.7.1	Architecture enhancements

None.

### 7.7.2	Solution description

#### 7.7.2.1	General

The following solution corresponds to the key issue#3 on enhancements to service continuity planning in clause 4.3

#### 7.7.2.2	Procedure

In this solution, the EES is responsible to identify the ACR type in ACR detection part or receive the ACR type from the EAS and the EEC. When application context transmission is complete, the EAS will send ACR status update message to the EES, once EES detects that UE moves to the predicted/expected location, the EES will notify EEC ACR complete message, then the EEC can be aware of  the compeletion of ACR. With this solution the EAS will not need to monitor the UE mobility and determine when to send the ACR status update message to the EES.

Compared to the procedure specified in 3GPP TS 23.558 clause  8.8.2.5, the following differences are captured below:

2.	The detection entities (EEC, S-EAS, S-EES) detect the ACR may be required and identify the ACID and related ACR type (normal ACR or service continuity planning).

3.	If the EEC or S-EAS detect the ACR event, the EEC or S-EAS should inform S-EES with ACID, ACR type and predicted/expected UE location or EAS service area in the ACR launching procedure.

10. when S-EAS detects application context transmission is complete, the S-EAS sends ACR status update request message to the S-EES with ACID.

11. Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC.

Compared to the procedure specified in 3GPP TS 23.558 clause  8.8.2.3, the following differences are captured below:

4.	The EEC performs ACR lauching procedure indicating S-EES with ACR type (normal ACR or service continuity planning) and predicted/expected UE location or EAS service area.7.	When S-EAS detects application context transmission is complete, the S-EAS sends ACR status update request message to the S-EES with ACID.

9. 	Once S-EES detects the UE has moved to the predicted/expected UE location or EAS service area, then the S-EES sends ACR complete notify message to the EEC indicating that UE has moved to the predicted location.

Enhancements to 3GPP TS 23.558 clause 8.8.3.4	ACR launching procedure

Figure 8.8.3.4-1 illustrates the ACR launching procedure by the EEC or the S-EAS. If this procedure is triggered by the EEC, depending on the ACR action indicated in the ACR request, the procedure is used for either ACR initiation or ACR determination. If this procedure is triggered by the S-EAS, the procedure is used for ACR determination.

Pre-condition:

1.	The EEC has been authorized to communicate with the EES as specified in clause 8.11, if the procedure is triggered by the EEC; and

2.	Information related to the S-EES is available with the S-EAS, if the procedure is triggered by the S-EAS.

Figure 8.8.3.4-1: ACR launching procedure

1.	The EEC or the S-EAS sends an ACR request message to the EES in order to start ACR. The ACR request message includes ACR type to indicate whether the ACR is for normal ACR or service continuity planning. The ACR request message includes Predicted/Expected UE location or EAS service area to indicate that the EES should detect whether the UE has moves to the Predicted/Expected UE location or EAS service area or not in ACR clean-up phase. The ACR request message includes ACR action to indicate either ACR initiation request or ACR determination request. If the procedure is triggered by the S-EAS, the ACR request message is only for ACR determination.

An ACR request for ACR initiation:

-	includes an indication of whether the EEC requests the EES to perform EAS notification; and

-	provides information used by EES to perform AF traffic influence as in 3GPP TS 23 501 [2].

An ACR request for ACR determination informs the EES that the need for ACR has been detected at EEC.

2.	The EES checks if the requestor is authorized for this operation. If authorized, the EES processes the request and performs the required operations.

If the request in step 1 is for ACR initiation:

-	the EES may use information provided in the request to apply the AF traffic influence with the N6 routing information of the T-EAS in the 3GPP Core Network (if applicable), as described in 3GPP TS 23.501 [2], clause 5.6.7.1; and

-	if the EAS notification indication is provided in the step 1 request and the EAS has subscribed to receive such notification, the EES shall notify the EAS about the need to start ACR.

If the request in step 1 is for ACR determination, the EES decides to execute ACR as described in clause 8.8.2.5.

3.	The EES responds to the requestor's request with an ACR response message.

Enhancements to 3GPP TS 23.558 clause 8.8.4.4	ACR request

Table 8.8.4.4-1 describes information elements for the ACR request sent from the EEC either to the S-EES or T-EES.

Table 8.8.4.4-1: ACR request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor Identifier | M | Unique identifier of the requestor (i.e. EECID or EASID). |
| Security credentials | M | Security credentials resulting from a successful authorization for the edge computing service. |
| UE identifier (NOTE 4) | O | The identifier of the UE (i.e. GPSI). |
| ACR type | M | Indicates whether the ACR is for normal ACR or service continuity planning |
| Predicted/Expected UE location or EAS service area (NOTE 5) | O | The predicted/expected location information of the UE. The UE location is described in clause 7.3.2 or the predicted/expected EAS service area as described in clause 7.3.3.3 |
| ACR action (NOTE 3) | M | Indicates the ACR action (ACR initiation or ACR determination) |
| ACR initiation data (NOTE 2) | O | ACR initiation IEs to be included in an ACR request message when ACR action indicates it is ACR initiation request. |
| > T-EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP 3-tuple) of the T-EAS. |
| > DNAI of the T-EAS | O | DNAI information associated with the T-EAS. |
| > N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to the T-EAS DNAI. |
| > EAS notification indication | M | Indicates whether to notify the EAS about the need of ACR. |
| > S-EAS endpoint (NOTE 1) | O | Endpoint information of the S-EAS |
| ACR determination data (NOTE 2) | O | ACR determination IEs to be included in an ACR request message when ACR action indicates it is ACR determination request. |
| > S-EAS endpoint | M | Endpoint information of the S-EAS |
| NOTE 1: This IE shall be present if the EAS notification indication indicates that the EAS needs to be informed. NOTE 2: Either ACR initiation or ACR determination shall be included corresponding to the ACR action. NOTE 3: This IE shall indicate ACR determination if the request originates from the S-EAS. NOTE 4: This IE shall be present if the request originates from the EEC. NOTE 5: This IE shall be present if the ACR type indicates the ACR procedure is for service continuity planning. |  |  |

## 7.8	Solution #8: EAS Service API enablement using CAPIF

### 7.8.1	Architecture enhancements

None.

### 7.8.2	Solution description

#### 7.8.2.1	General

This solution addresses the Key issue #2: Enablement of Service APIs exposed by EAS as specified in the clause 4.2 by supporting for an EAS to expose its Service APIs towards the other EASs.

As specified in TS 23.558 (Rel-17), the Edge Enabler Layer exposes Service APIs towards the EASs. The exposed Service APIs include the capabilities provided by EES (clause 8.6 of TS 23.558); the capabilities provided by the 3GPP core network (clause 8.7 of TS 23.558); and SEAL service APIs (clause A.4 of TS 23.558).

In this solution, the Edge Enabler Layer also supports for an EAS to expose its Service APIs (i.e., EAS Service APIs) towards the other EASs in order to fulfil the architectural requirements specified in the clause 5.xThis solution exploits CAPIF [TS 23.222] to support publication/discovery, and change subscription of EAS Service APIs as studied in Sol#15 of TR 23.758 (Rel-17) with the following architectural assumptions within the CAPIF architecture:

-	An EAS may act as an API provider by implementing API provider domain functions (i.e., API exposing function, API publishing function, and API management function)

-	An EAS may act as an API invoker

-	An EES may act as a CAPIF provider by implementing CAPIF core function (CCF)

Based on the architectural assumption above, the essential operations regarding EAS Service APIs complying with CAPIF are as follows:

-	An EAS (acting as API provider) may publish its EAS Service APIs to EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) may discover EAS Service APIs from EES (acting as CAPIF provider)

-	An EAS (acting as API invoker) may subscribe to be notified of dynamic information or availability of EAS Service APIs from EES (acting as CAPIF provider)

Editor's note: TBD for further considerations in other deployment scenarios such as EAS implementing CCF.

#### 7.8.2.2	CAPIF operations in Edge Enabler Layer

The Figure 7.8.2.2-1 depicts the essential operational steps for EAS Service API enablement using the CAPIF operations as shown in Annex A of TS 23.222.

Pre-conditions:

1.	The EAS #A-1 and EAS #A-2 have completed the EAS registration with the EES #A.

Figure 7.8.2.2-1: CAPIF operations in Edge Enabler Layer for EAS Service API enablement

1.	The EAS #A-2 (AMF) registers its API provider domain functions to the EES #A (CCF) via CAPIF-3.

2.	The EAS #A-2 (APF) publishes its exposing Service API(s) to the EES #A (CCF) via CAPIF-4.

3.	The EAS #A-1 (API invoker) performs onboarding process with the EES #A (CCF) via CAPIF-1.

4.	The EAS #A-1 (API invoker) discovers from the EES #A (CCF) a Service API required to run via CAPIF-1.

5.	The EAS #A-1 (API invoker) invokes the Service API provided by EAS #A-2 (AEF) via CAPIF-2 as discovered from the EES #A (CCF).

6. 	The EAS #A-1 (API invoker) subscribes to notifications of any updates of target Service APIs on the EES #A (CCF) via CAPIF-1.

7.	The EES #A (CCF) and EES #B (CCF) inter-operate with each other via CAPIF-6 for interconnection operations for publication and discovery of Service APIs managed by each EES.

Editor's note: TBD for enhancements to information elements of CAPIF APIs, e.g., Service KPIs, Required Service APIs, Event information across EESs, etc.

### 7.8.3	Solution evaluation

This clause provides an evaluation of the solution.

## 7.x	Solution #x: <title>

This clause describes the solution and its evaluation with a suitable title. Please also update Table 7.0-1 and Table 10.2.1-1.

### 7.x.1	Architecture enhancements

This clause summarizes the architecture enhancements required by this solution – please mention the option from clause 6 which is the basis for this solution. Also list any specific functional enhancements, new identities and cardinality rules that are required for this solution. If no new enhancements are required compared to the Rel-17 architecture, please mention 'None'.

### 7.x.2	Solution description

This clause describes the solution in detail. Please provide description, flow diagram, information flows and APIs for the new solution.

### 7.x.3	Solution evaluation

This clause provides an evaluation of the solution.

# 8	Deployment scenarios

## 8.1	<Title>

Provide an illustrative description of the deployment scenario.

# 9	Involved entities and relationships

## 9.1	<Title>

Provide an illustrative description of the changes in existing or new involved entities and relationships.

# 10	Overall evaluation

This clause provides a summary of architecture enhancements and solution evaluations.

## 10.1	Architecture enhancements

## 10.2	Key issue evaluations

### 10.2.1	General

All the key issues and solutions specified in this technical report are listed in Table 10.2.1-1.

The table provides a mapping of the key issues to the related solutions. It also indicates whether the solution requires enhancement to the Release-17 architecture, identities or cardinalities, and lists the dependencies on other working groups.

Table 10.2.1-1 Key issue and solutions

| Key issues (evaluation clause reference) | Solution | Solution (clause reference) | Enhancements required | Dependency on other working groups |
| --- | --- | --- | --- | --- |
| #1: <title> | #x: <title> | 6.x | Architecture / Identities / Cardinalities / None | <WG> |
|  | #y: <title> | 6.y | Architecture / Identities / Cardinalities / None | <WG> |

### 10.2.x	Key issue #x

This clause provides an overall evaluation of all the solutions defined for Key Issue #x.

# 11	Conclusions

This clause provides conclusions of the study.

## 11.1	Architecture enhancements

## 11.2	Solutions

Annex A (Informative):
ETSI MEC and EDGEAPP system comparison

## A.1 General

This Annex compares EDGEAPP architecture R17 as defined in 3GPP TS 23.558 [2] and ETSI MEC architecture [3] and provides a gap analysis, addressing the objective in Key issue #5 in clause 4.5.

## A.2 Service consumer and service provider

The functionalities enabled via the Mp1 reference point between MEC applications and MEC platform is mainly described in ETSI GS MEC011 [14]. The related functionality includes MEC service registration/deregistration, MEC service discovery and event notifications. Other functionality includes MEC service availability, traffic rules, DNS and time of day.

From ETSI MEC's perspective, there are two types of MEC Applications, i.e., MEC Application that consumes MEC Services and MEC Application that provides MEC service(s). For the MEC Application that provides MEC service(s), the MEC Application sends a service registration request to the MEC platform to register the MEC service during the MEC Application start-up procedure. As for the MEC Application that consumes MEC Services, the MEC Application can send a service query request to the MEC platform to discover a MEC service. It should be noted that the API of registration and discovery is defined for MEC service.

In R17 of SA6, the EES can take the role of the CAPIF core function, and the vertical application enabler server acting the AEF and publish the vertical application enabler server APIs to the EES. Further, the vertical application enabler server APIs is discovered by the EASs acting as the API invoker during the service API discover procedure as specified in 3GPP TS 23.222 [15].

In clause 4.2, the Key issue #2 plans to study Enablement of Service APIs exposed by EAS. The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC:

[Observation A.2-1] The R17 of EDGEAPP only defines the functionality of EAS acting as an invoker, which is similar to MEC Application that consumes MEC Services defined in ETSI MEC. According to the Key issue #2 in clause 4.2, The EAS acting as a service provider is expected to be defined in R18.

## A.3 EAS/MEC application profile provisioning

ETSI MEC and EDGEAPP defined different style of EAS/MEC application profile provisioning. To put it more detail, the MEC application profile is provided by the OAM. The information flows for lifecycle management of MEC applications is described in ETSI MEC010-2 [13]. The MEC application can start producing or consuming MEC Services after the MEC Application is instantiated / start-up. The application Descriptor (AppD), which can be regarded as the MEC application profile, is part of application package and describes application requirements and rules required by the application provider. The attributes of the AppD are quoted from the clause 6.2.1.2 of ETSI MEC010-2 [13] as below:

Table A.3-1: Attributes of the AppD [x1]

| Attribute name | Cardinality | Data type | Description |
| --- | --- | --- | --- |
| appDId | 1 | String | Identifier of this MEC application descriptor. This attribute shall be globally unique. See NOTE 1. |
| appName | 1 | String | Name to identify the MEC application. |
| appProvider | 1 | String | Provider of the application and of the AppD. |
| appSoftVersion | 1 | String | Identifies the version of software of the MEC application. |
| appDVersion | 1 | String | Identifies the version of the application descriptor. |
| mecVersion | 1 | String | Identifies version(s) of MEC system compatible with the MEC application described in this version of the AppD. The value shall be formatted as comma-separated list of strings. Each entry shall have the format <x>.<y>.<z> where <x>, <y> and <z> are decimal numbers representing the version of the present document. Whitespace between list entries shall be trimmed before validation. |
| appInfoName | 0..1 | String | Human readable name for the MEC application. |
| appDescription | 1 | String | Human readable description of the MEC application. |
| virtualComputeDescriptor | 1 | VirtualComputeDescriptor | Describes CPU and memory requirements, as well as optional additional requirements, such as disk and acceleration related capabilities, of the virtualisation container used to realise this MEC application. |
| swImageDescriptor | 1 | SwImageDescriptor | Describes the descriptors of the software image to be used by the virtualisation container used to realise this MEC application. |
| virtualStorageDescriptor | 0..N | VirtualStorageDescriptor | Defines descriptors of virtual storage resources to be used by the MEC application. |
| appExtCpd | 0..N | AppExternalCpd | Describes external interface(s) exposed by this MEC application. |
| appServiceRequired | 0..N | ServiceDependency | Describes services a MEC application requires to run. |
| appServiceOptional | 0..N | ServiceDependency | Describes services a MEC application may use if available. |
| appServiceProduced | 0..N | ServiceDescriptor | Describes services a MEC application is able to produce to the platform or other MEC applications. Only relevant for service-producing apps. |
| appFeatureRequired | 0..N | FeatureDependency | Describes features a MEC application requires to run. |
| appFeatureOptional | 0..N | FeatureDependency | Describes features a MEC application may use if available. |
| transportDependencies | 0..N | TransportDependency | Transports, if any, that this application requires to be provided by the platform. These transports will be used by the application to deliver services provided by this application. Only relevant for service-producing apps. See NOTE 2. |
| appTrafficRule | 0..N | TrafficRuleDescriptor | Describes traffic rules the MEC application requires. |
| appDNSRule | 0..N | DNSRuleDescriptor | Describes DNS rules the MEC application requires. |
| appLatency | 0..1 | LatencyDescriptor | Describes the maximum latency tolerated by the MEC application. |
| terminateAppInstanceOpConfig | 0..1 | TerminateAppInstanceOpConfig | Configuration parameters for the Terminate application instance operation. |
| changeAppInstanceStateOpConfig | 0..1 | ChangeAppInstanceStateOpConfig | Configuration parameters for the change application instance state operation. |
| userContextTransferCapability | 0..1 | UserContextTransferCapability | If the application supports the user context transfer capability, this attribute shall be included. |
| appNetworkPolicy | 0..1 | AppNetworkPolicy | If present, it represents the application network policy of carrying the application traffic. |
| NOTE 1: The appDId shall be used as the unique identifier of the application package that contains this AppD. NOTE 2: This attribute indicates groups of transport bindings which a service-producing MEC application requires to be supported by the platform in order to be able to produce its services. At least one of the indicated groups needs to be supported to fulfil the requirements. NOTE 3: The support of application descriptor containing descriptions of multiple virtualisation containers and/or application software images is out of scope of the present document. |  |  |  |

It can be seen that the AppD is mainly used in the management plane for instantiating an application, and it is typically static and with little differentiation no matter where the MEC hosts is.

In EDGEAPP, the EAS profile is provided in the EAS registration request. According to clause 8.2.4 of 3GPP TS 23.558 [2] the information element of the EAS profile is listed as below:

Table A.3-2: EAS Profile

| Information element | Status | Description |
| --- | --- | --- |
| EASID | M | The identifier of the EAS |
| EAS Endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the EAS. This information maybe discovered by EEC and exposed to ACs so that ACs can establish contact with the EAS. |
| ACID(s) | O | Identifies the AC(s) that can be served by the EAS |
| EAS Provider Identifier | O | The identifier of the ASP that provides the EAS. |
| EAS Type | O | The category or type of EAS (e.g. V2X) |
| EAS description | O | Human-readable description of the EAS |
| EAS Schedule | O | The availability schedule of the EAS (e.g. time windows) |
| EAS Geographical Service Area | O | The geographical service area that the EAS serves. ACs in UEs that are located outside that area shall not be served. |
| EAS Topological Service Area | O | The EAS serves UEs that are connected to the Core Network from one of the cells included in this service area. ACs in UEs that are located outside this area shall not be served. See possible formats in Table 8.2.7-1. |
| EAS Service KPIs | O | Service characteristics provided by EAS, detailed in Table 8.2.5-1 |
| EAS service permission level | O | Level of service permissions e.g. trial, gold-class supported by the EAS |
| EAS Feature(s) | O | Service features e.g. single vs. multi-player gaming service supported by the EAS |
| EAS Service continuity support | O | Indicates if the EAS supports service continuity or not. This IE also indicates which ACR scenarios are supported by the EAS. |
| List of EAS DNAI(s) | O | DNAI(s) associated with the EAS. This IE is used as Potential Locations of Applications in clause 5.6.7 of 3GPP TS 23.501 [14]. It is a subset of the DNAI(s) associated with the EDN where the EAS resides. |
| List of N6 Traffic Routing requirements | O | The N6 traffic routing information and/or routing profile ID corresponding to each EAS DNAI. |
| EAS Availability Reporting Period | O | The availability reporting period (i.e. heartbeat period) that indicates to the EES how often it needs to check the EAS's availability after a successful registration. |
| EAS Required Service APIs | O | A list of the Service APIs that are required by the EAS |
| EAS Status | O | The status of the EAS (e.g. enabled, disabled, etc.) |

Comparison of Table A.3-1 and A.3.2 shows that:

[Observation A.3-1] ETSI MEC and EDGEAPP defined different style of EAS/MEC application profile provisioning. The MEC application profile is provided by the OAM while in EDGEAPP the EAS profile is provided in the EAS registration request. It is FFS whether and how to address such differences in SA6.

[Observation A.3-2] Some IEs of the EAS profile may overlap with the ones defined in AppD (e.g., EAS ID vs. appName, EAS Provider Identifier vs. appProvider, EAS description vs. appDescription). Different from the AppD, most of the IEs of the EAS profile are optional. It is FFS whether and how to address such differences in SA6.

## A.4 EAS registration and EAS discovery

In R17 of EDGEAPP, the EAS Registration procedure is defined to allow an EAS to provide its information to an EES in order to enable its discovery as defined in clause 8.4.3 of 3GPP TS 23.558 [2]. The EAS discovery procedure is used to provide EAS information to the EEC. After the EEC is provisioned with the EAS information, it can establish a connection to the EAS. Besides, in the service continuity scenario, the source EAS may send an EAS discovery request to the EES to discover a target EAS (providing same functionality as the source EAS) to serve the UE as defined in clause 8.8.3.2 of 3GPP TS 23.558 [2].

However, in current ETSI MEC specification, no APIs for MEC Application registration is defined because it is assumed that all MEC Application are on-boarded and managed by MEC Orchestrator, which was specified in ETSI GS MEC 010-2 [13]. API for MEC Application discovery is not defined since the existing MEC service is either defined from the MEC Application's perspective or it is consumed by the MEC Application rather than the UE.

Therefore, the comparison EAS registration and EAS discovery of EDGEAPP [2] and ETSI MEC specification [13] shows that:

[Observation A.4-1] The EAS registration and EAS discovery mechanism is defined in R17 of SA6 while it is not defined in ETSI MEC. It is FFS whether and how to address such differences in SA6, e.g., in support of ETSI MEC.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-07 | SA6#44-e | S6-211758 |  |  |  | TR Skeleton | 0.0.0 |
| 2021-07 | SA6#44-e |  |  |  |  | S6-211754, S6-211759, S6-211760 | 0.1.0 |
| 2021-09 | SA6#45-e |  |  |  |  | S6-212048, S6-212054, S6-212087, S6-212103, S6-212115, S6-212142, S6-212166, S6-212167, S6-212168, S6-212169, S6-212170, S6-212171 | 0.2.0 |
| 2021-10 | SA6#45-BIS-e |  |  |  |  | S6-212478, S6-212402, S6-212479, S6-212480, S6-212481, S6-212482, S6-212207, S6-212411, S6-212206, S6-212403, S6-212483, S6-212401, S6-212484, S6-212486, S6-212456 | 0.3.0 |