| 3GPP TR 23.947 V0.2.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Guidelines for 3GPP Application Enablement usage; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	6

3.1	Terms	6

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview of Frameworks	8

4.1	Introduction	8

4.2	Common API Framework (CAPIF)	8

4.3	Edge Application Enablement (EDGEAPP)	10

4.4	Service Enabler Architecture Layer (SEAL)	12

4.4	Inter-framework Relationships Overview	13

5	Use Cases	13

5.1	Crowd Counting Video Analytics	13

5.1.1	Introduction	13

5.1.2	Use case Description	13

5.1.2	Use case Realisation over Application Enablement Frameworks	14

5.1.3.1	General	14

5.1.3.2	AC Discovering the Serving EES	14

5.1.3.3	CAPIF roles of EDGEAPP entities	15

5.1.4	Use case Flows	15

5.1.4.1	General	15

5.1.4.2	Step 1: AC Registration & Provisioning	15

5.1.4.3	Step 2: EAS Discovery	16

5.1.4.4	Step 3: CAPIF API Discovery	16

5.1.4.5	Step 4: Service Invocation	17

5.1.4.6	Step 5: Mobility with Application Context Relocation	17

5.1.4.7	Step 6: API Update & Resume Streaming	18

5.2	{Use case title}	19

5.2.1	Introduction	19

5.2.2	Use case Description	19

5.2.3	Use case Realisation over Application Enablement Frameworks	19

5.2.4	Use case Flows	19

6	Developer Guidelines	19

6.1	Introduction	19

6.2	Cross-Framework Integration	19

6.3	Example Application Flows	19

6.4	Multi-CAPIF Considerations	19

7	Security Considerations	19

Annex A: Infrastructure Topology Diagrams Examples	20

A.1	CAPIF plus EDGEAPP Topology	20

A.1.1	General	20

A.1.2	Step 1: ECS Registration into CAPIF	21

A.1.3	Step 2: ECS Publishes APIs	21

A.1.4	Step 3: EES Onboarding and Registration into CAPIF	21

A.1.5	Step 4: EES Publishes Its APIs	22

A.1.6	Step 5: EES Discovers ECS APIs	22

A.1.7	Step 6: EES Registers with ECS	22

A.1.8	Step 7: EAS Onboarding and Registration into CAPIF	23

A.1.9	Step 8: EAS Discovers ECS/EES APIs	24

A.1.10	Step 9: EAS Registers with EES	24

A.1.11	Step 10: EAS Publishes its Application APIs	24

A.1.12	Step 11: AC/EEC Access EAS Services	25

A.2	CAPIF+SEAL Topology	25

A.3	CAPIF+EDGEAPP+SEAL Topology	25

Annex B: Change history	26

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

Emerging services from verticals such as connected vehicles (V2X), unmanned aerial vehicles (UAVs) and smart factories require low-latency, high-bandwidth, and context-aware services delivered close to the end-user. SA6 in 3GPP has developed several enablement frameworks to support these requirements:

-	CAPIF (3GPP TS 23.222 [2]) defines a common and secure way to expose and consume network and service APIs across multiple domains and stakeholders.

-	EDGEAPP (3GPP TS 23.558 [4]) specifies the architecture for enabling and orchestrating applications at the network edge, including discovery, service continuity and mobility support.

-	SEAL (3GPP TS 23.434 [6]) provides reusable service enablers that can be consumed by vertical applications via standardised APIs. Examples of these common services are Group management or Location management.

This TR provides use case examples oriented to Application developers for understanding how SA6 enablement frameworks work together and what APIs are essential for developing their applications when dealing with Operators/Service Providers infrastructure that follows SA6 specifications.

# 1	Scope

This Technical Report (TR) explains how application (App) developers can leverage the services from enablement frameworks, namely Common API Framework (CAPIF), Edge Application Enablement (EDGEAPP), and the Service Enabler Architecture Layer (SEAL) to create their software.

To facilitate understanding, this study presents a set of representative deployment options. These are provided as examples to illustrate common operator scenarios, but they do not limit the options that the frameworks offer. The TR will cover the linkage between the available services with different vertical use cases, to guide App developers on how to use the different services to fulfil their use cases.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs (CAPIF); (Stage 2)".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs (CAPIF); (Stage 3)".

[4]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[5]	3GPP TS 29.558: "Enabling Edge Applications; Application Programming Interface (API) specification; (Stage 3)".

[6]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[7]	3GPP TS 33.122: "Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs".

[8]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications".

[9]	3GPP TS 33.434: "Security aspects of Service Enabler Architecture Layer (SEAL) for verticals".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	IETF RFC 7519: "JSON Web Token (JWT)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Application Client (AC): An application component, typically hosted on User Equipment (UE), that consumes services provided by an EAS (as defined in 3GPP TS 23.558 [4]).

Application Context: A set of data about the Application Client that resides in the Edge Application Server (as defined in 3GPP TS 23.558 [4]).

CAPIF API Invoker: An entity that consumes APIs exposed via CAPIF (as defined in 3GPP TS 23.222 [2]).

CAPIF API Provider: An entity that offers APIs via CAPIF (as defined in 3GPP TS 23.222 [2]).

Common API Framework (CAPIF): A 3GPP framework that provides common functions for API exposure, onboarding, discovery, and authorization across multiple domains and stakeholders (as defined in 3GPP TS 23.222 [2]).

Edge Application Server (EAS): An application component hosted on an edge node that provides services to ACs (as defined in 3GPP TS 23.558 [4]).

Edge Enabler Client (EEC): A client-side function, co-located with the AC, that interacts with the Edge Enabler Server (EES) for discovery, registration, and mobility support (as defined in 3GPP TS 23.558 [4]).

Edge Enabler Server (EES): A network function that provides EDGEAPP control plane services to EECs, including EAS discovery, mobility handling, and service continuity (as defined in 3GPP TS 23.558 [4]).

Edge Computing Service Provider (ECSP): An operator or provider responsible for deploying and operating EDGEAPP functions (ECS, EES, EAS) in the network (as defined in 3GPP TS 23.558 [4]).

Service Enabler Architecture Layer (SEAL): A framework providing reusable service enablers such as location, group communications, and messaging, available to vertical applications (as defined in 3GPP TS 23.434 [6]).

SEAL Server: A server implementing SEAL service enabler functions, possibly hosted centrally or at the edge, and accessible via CAPIF (as defined in 3GPP TS 23.434 [6]).

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AC	Application Client

AEF	API Exposing Function

AMF	API Managing Function

APF	API Publishing Function

CAPIF		Common API Framework

CCF	CAPIF Core Function

EAS	Edge Application Server

ECSP	Edge Computing Service Provider

EEC	Edge Enabler Client

EES	Edge Enabler Server

ECS	Edge Configuration Server

EDGEAPP	Edge Application Enablement

JSON	JavaScript Object Notation

OAuth	Open Authorization

REST	Representational State Transfer

SEAL	Service Enabler Architecture Layer

# 4	Overview of Frameworks

## 4.1	Introduction

Emerging 5G vertical services have requirements that can include low-latency, high-bandwidth, and context-aware processing at the network edge. To address these needs, 3GPP has developed several application enablement frameworks, each targeting a specific aspect of the application layer architecture:

-	Common API Framework (CAPIF) – provides a unified, secure way to expose and consume service and network APIs across multiple domains and stakeholders, specified at stage-2 in 3GPP TS 23.222 [2]. CAPIF offers a harmonized approach for API development and exposure, allowing 3GPP network functions and even third parties to publish and discover APIs in a common environment.

-	Edge Application Enablement (EDGEAPP) – specifies an architecture for applications at the network edge, including support for local service discovery, application lifecycle management, and continuity during user mobility, specified at stage-2 in 3GPP TS 23.558 [4]. EDGEAPP enables edge computing with minimal changes to existing applications, abstracting the edge infrastructure complexity behind standardized interfaces.

-	Service Enabler Architecture Layer (SEAL) – defines a layer of reusable service enablers (e.g., location, group communications, messaging) available to vertical applications via standardized APIs, specified at stage-2 in 3GPP TS 23.434 [6]. By providing common functions that many industries require, SEAL allows different vertical applications to leverage the same core capabilities (for example, a group communication service or location service) instead of each vertical implementing its own.

This Technical Report provides an overview of how these frameworks work together and guides application developers on using them to build advanced services.

## 4.2	Common API Framework (CAPIF)

The Common API Framework (CAPIF) is a 3GPP framework that provides common functions for secure API exposure, discovery, onboarding, and authorization across multiple domains, 3GPP TS 23.222 [2]. Its primary role is to harmonize how service APIs are published and consumed, offering a single point of entry for applications (API invokers) towards the common API aspects (CAPF APIs) and a consistent method for API providers to expose their service APIs. By introducing a common API framework, CAPIF prevents fragmentation because application developers and third-party service providers can use one standardized approach, rather than dealing with independent interfaces for each service API. CAPIF is designed to be used in conjunction with other frameworks, where it is agnostic to the content of the APIs and instead focuses on the exposure mechanism for APIs.

CAPIF was first specified in 3GPP Release 15 and has since been adopted for exposing APIs from core network functions (e.g. NEF for network capabilities, MBMS, etc.) as well as from application-enablement layer services, in a secure and controlled manner.

The CAPIF architecture is based on a service-oriented model with three main entity groups as illustrated in Figure 4.2.1, specifically a CAPIF Core Function (CCF), the API Invoker and one or more API Provider domains:

-	CAPIF Core Function (CCF): The CCF handles the onboarding of API invokers and API registering of API provider domain functions (establishing trust and identity for each). It maintains the service API registry for discovery, and provides common services like authentication, authorization token issuance and logging.

-	API Exposing Function (AEF): A function on the API provider domain side that hosts one or more service APIs and exposes them to external consumers (API Invokers). The AEF is the entry point through which an API invoker accesses the provider’s service API; it enforces the service’s access policies and handles the runtime service API calls (e.g. rate limiting). A 3GPP network function or application server can presented themselves as an AEF to offer their service APIs in a standardized way.

-	API Publishing Function (APF): A provider-side function responsible for publishing the service API information and endpoints of the AEF to the CAPIF Core Function. When an API is made available, the APF registers the API’s meta-information (e.g. name, version, resource paths, supported operations) with the CCF, so that prospective invokers can discover it.

-	API Management Function: A provider-side management entity that allows the API provider to administer and monitor their APIs. The API Management Function interfaces with the CCF for reporting and retrieving logs of API invocations (for auditing), subscribing to events (e.g. notifications of certain API usage or threshold breaches).

-	API Invoker: The consuming entity that uses CAPIF to discover and invoke service APIs. An API Invoker could be an external third-party application or another network function/application within the operator’s environment. The invoker first onboards with the CAPIF core function to establish its identity and credentials, then queries the API registry to discover available services, and finally invokes the desired API on the corresponding AEF, obtaining authorization from the CCF as required. In a developer context, the API Invoker role is played by the developer’s application or service that seeks to utilize operator or third-party APIs in a secure manner.

Figure 4.2.1: Functional model for the CAPIF as defined in 3GPP TS 23.222 [2]

CAPIF operates through a set of well-defined procedures and interfaces. At a high level, the lifecycle of API exposure and consumption in CAPIF is as follows:

1.	Onboarding and registration: An API provider (such as an Edge Configuration Server in an edge computing scenario) will register with the CCF to become a recognized provider, typically using credentials or certificates issued by the operator’s CAPIF authority (see clause 8.28 of 3GPP TS 23.222 [2]). Similarly, an application acting as an API invoker (e.g. a vertical application server or a UE client) will onboard with the CCF to obtain a secure invoker identity (see clause 8.1 of 3GPP TS 23.222 [2]). The CAPIF core function can provision digital certificates or JSON Web Tokens (JWT) and record the entity’s public information (like its name, domain, and roles) in its database. This establishes a trust relationship: the CCF recognizes the entity as an authorized API provider or consumer in the system.

2.	API Publication (see clause 8.3 of 3GPP TS 23.222 [2]): Once registered, an API provider exposes its service API through CAPIF. The provider’s API Publishing Function sends the API description to the CAPIF core (via the CAPIF publishing interface, e.g. CAPIF-4) to register the API in the registry. This publication typically includes the API’s endpoint URI (which can point to the AEF), the resource structure and methods, required security scopes or policies, and any other metadata (version, category, etc.). For instance, in an EDGEAPP scenario, after an Edge Configuration Server (ECS) registers, it would publish its available service APIs (like edge node registration API, configuration APIs) to CAPIF so that edge clients or other components can later discover them. The CAPIF core can verify that the publishing entity is authorized to expose those APIs (e.g. checking that the provider is indeed entitled to publish an “Edge Configuration” service). Once published, the API is discoverable by the authorized invokers.

3.	Discovery (see clause 8.7 of 3GPP TS 23.222 [2]): An API invoker queries the CAPIF Core Function to discover information a service API. Discovery can be performed using various criteria (service name, category, provider, etc.). The CCF will return the details of the AEF address, interface details and information on how to invoke it. CAPIF supports a federated discovery if multiple CCFs are interconnected, but within a single domain the CCF acts as the central lookup for all APIs. For example, an Edge Enabler Client (EEC) that needs to call a service on an Edge Application Server (EAS) can query CAPIF (via the CCF) to get the API endpoint and requirements for the EAS’s service API.

4.	API Invocation with Authorization: Before invoking the target API, the API invoker obtains authorization from CAPIF. In CAPIF’s security framework, the CCF can function as an OAuth2 Authorization Server for API access tokens, see 3GPP TS 33.122 [7]. The invoker (acting as an OAuth client) requests an access token from the CCF, proving its identity (using credentials established at onboarding) and specifying the API it wants to call. The CCF issues a OAuth2 token if the invoker is allowed to use that API, with appropriate scope/permissions encoded. Armed with this token, the invoker then sends the API request to the provider’s AEF (over the CAPIF-internal interface to the AEF, e.g. CAPIF-2) and includes the token in the invocation. The AEF validates the token (by verifying its signature and checking with CCF if needed) and, if valid, allows the request to be processed by the underlying service. This ensures that only properly onboarded and authorized applications can invoke protected APIs. CAPIF also supports additional authorization models – for example, a mode where a “Resource Owner” (end-user or owner of the accessed resource) involvement is required for authorization (termed Resource Owner Aware API Access, RNAA).

5.	Logging and Monitoring (see clause 8.19 and 8.21 respectively of 3GPP TS 23.222 [2]): As APIs are invoked, CAPIF provides a logging service. Each service API invocation and response can be logged via the CAPIF core (or the provider’s API Management Function reporting to CCF), recording details such as who invoked which API, when, and with what result. These logs enable auditing and troubleshooting. CAPIF can also emit events or notifications – for instance, if an API provider wants to be notified when a new invoker enrols to its API or when certain thresholds are exceeded, the framework can support that through an events interface.

For application developers, CAPIF greatly simplifies integration with 3GPP network services and other service platforms. Instead of dealing with separate authentication and discovery mechanisms for each API (e.g. one for location services, another for messaging, etc.), a developer can rely on CAPIF to provide a one-stop hub for all API interactions. For example, a developer building a fleet management application could use CAPIF to discover the operator’s “vehicle location API” and invoke it securely, without needing to know the internal details of the network’s location function – CAPIF mediates trust and access. Likewise, if the developer’s application provides its own API (say it’s an analytics service that the operator might call), the developer can publish this API via CAPIF so that it’s uniformly accessible to other authorized parties.

## 4.3	Edge Application Enablement (EDGEAPP)

Edge Application Enablement (EDGEAPP) has been specified by 3GPP to enable applications to be deployed and managed at the edge of the network (closer to end users) to achieve ultra-low latency and high bandwidth efficiency, 3GPP TS 23.558 [4]. By offloading computation and services from centralized cloud to edge nodes (e.g. servers co-located with base stations or local data centers), EDGEAPP enables use cases like real-time video analytics, AR/VR, V2X, and industrial automation that require immediate processing and minimal backhaul delay. The key goal of EDGEAPP is to make this edge deployment transparent to application developers – applications should not need major redesign to benefit from edge computing. The framework accomplishes this by providing a standardized set of functions that handle discovery of edge services, provisioning and relocation of application instances, and continuity of sessions as users move between edge sites. In essence, EDGEAPP extends the 3GPP system with an application-layer control plane for edge computing, while hiding network complexity from the application itself.

The EDGEAPP architecture introduces several new functional entities (as depicted in Figure 4.3-1), which can be grouped into client-side and network-side components:

-	Application Client (AC): The end-user application component that consumes a service. In an edge computing scenario, this could be, for example, a video streaming app, a game client, or an IoT application running on a UE (User Equipment). The AC is typically developed by the Application Service Provider and contains the core application logic for the end user. Importantly, the AC itself does not directly interface with the edge network functions; instead, it interacts with the Edge Enabler Client via local APIs on the device. This design keeps the AC simple and focused on the service logic, while offloading networking tasks to the Edge Enabler Client.

-	Edge Enabler Client (EEC): A client-side middleware function residing on the UE (or whatever device hosts the AC). The EEC is the bridge between the application client and the edge system. It exposes a set of APIs (referred to as EDGE-5 interfaces in 3GPP) to the AC, through which the AC can request edge services (such as registering to the edge system, discovering available edge application servers, etc.). Internally, the EEC communicates with the network’s edge enablement functions: it registers the device/application with an Edge Enabler Server, keeps track of the current “serving” edge zone, and orchestrates any changes needed when the user moves. From the developer’s perspective, the EEC is crucial – by calling the EEC’s APIs, the developer’s application can utilize edge capabilities without needing to implement complex protocols. The EEC also handles obtaining any needed network authorization (for example, via CAPIF tokens when calling edge services) on behalf of the AC.

-	Edge Enabler Server (EES): A network-side function that is typically deployed at an edge data center or site serving a particular geographical zone. The EES is the core control-plane entity in the EDGEAPP framework. It manages edge application services within its zone and interfaces with EECs on UEs. Key responsibilities of the EES include registering new Edge Application Servers (EAS) when they become available in its zone, handling UE registrations (via EECs) so that the system knows which UEs are using edge services in that area and facilitating service discovery for UEs (telling an EEC which local EAS can provide the service the AC needs). The EES also monitors UE mobility (through integration with the cellular network or via the EEC’s updates) – when a UE is about to move or has moved to another zone, the EES can trigger Application Context Relocation (ACR) procedures to ensure continuity of the service session. In summary, the EES is analogous to a local edge broker/manager that coordinates between UEs and application servers at the edge.

-	Edge Application Server (EAS): The actual application server instance that runs on the edge node, providing the service logic to be consumed by the AC. This could be, for example, an instance of a video analytics engine, a game server, or a data caching server, deployed in the edge cloud. The EAS is owned by the application provider (ASP) and hosts the service that the AC uses. Each EAS registers with the local EES so that the system knows the service it offers and can advertise it to interested clients. The EAS relies on the EES for certain lifecycle operations; for instance, the EES can instruct the EAS to transfer state or shut down when a user leaves the zone. The EAS can also consume network and edge services itself – for example, an EAS might call an mobile network operator provided API (like a location service via CAPIF) to augment its application or use EES-provided APIs for retrieving information about UEs or network conditions. In EDGEAPP terminology, there are reference points (like EDGE-3 and EDGE-7) for EAS to interact with the EES and potentially with operator network exposures.

-	Edge Configuration Server (ECS): A centralized function that oversees the configuration and coordination of the edge system across multiple edge sites. The ECS can be thought of as the master controller in a multi-edge deployment. It maintains a global view of all edge sites (each with an EES and perhaps multiple EAS instances). The ECS provides services such as: answering queries from EECs or EESs about where certain application services are available (e.g., if a UE in zone A needs a service that’s currently only in zone B, the ECS can help route or instantiate it), bootstrapping new edge nodes (EESs) into the system, and storing subscription information or policies that apply network wide. In some scenarios, the EEC can contact the ECS at initial registration (EDGE-4 interface) to obtain information like the address of its serving EES if the UE is not preconfigured. The EESs also typically interface with the ECS (EDGE-6) to report new EAS deployments or to get updates on global configurations. The ECS, being a central point, can use CAPIF/NEF to interface with core network functions – for example, to fetch network resource exposure (like cell congestion status) or to publish its own APIs for other systems. The Edge Computing Service Provider (ECSP), for example the mobile network operator, is responsible for running the ECS, EES, and providing the infrastructure on which EAS instances run.

Figure 4.3-1: EDGEAPP Architecture, as specified in 3GPP TS 23.558 [4]

The typical sequence of interactions in EDGEAPP to deliver an edge-augmented service is outlined below (note that some steps can vary depending on deployment options, but this reflects a common scenario in a single operator’s network):

1.	Edge Infrastructure Registration: Before any end-user device participates, the edge nodes must be set up. Each Edge Enabler Server (EES) that comes online (for a new edge site) registers with the Edge Configuration Server (ECS). The ECS thus maintains a registry of all EESs and their capabilities/zones. This registration can involve mutual authentication and authorization – often incorporating CAPIF if the ECS’s APIs are published via CAPIF. In our context, the operator ensures all EES instances are known to the ECS (either through provisioning or dynamic discovery in larger deployments). Similarly, when an application provider wants to deploy an Edge Application Server (EAS) in a zone, they might coordinate with the operator such that the EAS gets registered with the local EES. The EES will record what service the EAS offers (e.g. “VideoAnalyticsService v1”) and possibly pass that information up to the ECS for global awareness. At this stage, CAPIF can be used: for example, the ECS and EES can onboard to CAPIF as API providers and invokers so that their APIs (like “register EAS” or “query edge info”) are accessible securely. By the end of this step, the edge control-plane is prepared: EES know about local EAS services, and the ECS knows about all EESs (and possibly their EAS listings).

2.	UE Discovery of Edge Service (Registration): When a user’s device (UE) powers on or an application starts, the Edge Enabler Client on the UE needs to discover which edge it should connect to. There are multiple ways this can happen. The UE could have a pre-provisioned address for an EES (perhaps provided by the operator’s subscription or via a device management system); or the UE could perform a DNS query or receive an indication from the network (e.g., during attach, the network might provide an “edge pointer”). Another method is the bootstrap via ECS: the EEC contacts the central ECS to ask, “where is my serving EES?” and the ECS responds with the address of the optimal EES for that UE (based on the UE’s location or subscription). Once the EEC knows which Edge Enabler Server to use, it proceeds to register with that EES. EEC-EES registration involves the EEC introducing the AC (application client) and itself to the EES: essentially saying “this UE/application wants to use edge services in your zone.” The EES authenticates the EEC (possibly via network credentials or certificates; security can rely on 3GPP credentials or CAPIF tokens if the EEC is acting as an invoker) and then creates a context for that AC on the edge. After a successful registration, the EES knows this UE (often by a unique Edge UE ID) and can start providing it with services like discovery notifications. The EEC will typically get a confirmation that it is registered. From the AC developer’s viewpoint, this whole discovery and registration process is triggered by a simple API call like Eeec_ACRegistration() from the AC to the EEC – the EEC and the network handle the rest (including contacting ECS or using CAPIF as needed).

3.	Service Discovery and Consumption: After registration, the AC can request an actual application service at the edge. Suppose the AC needs a “Video Analytics” service. The AC (through an API call to EEC, e.g. Eeec_EASDiscovery(serviceType)) asks the EEC to find an appropriate Edge Application Server (EAS) that provides that service. The EES receives this request via the EEC and checks whether an EAS in its zone offers the requested service. If yes, it selects the optimal EAS (there can be multiple instances or versions) and returns the details (e.g. the EAS’s IP address or domain and any required info to connect) to the EEC, which then passes it to the AC. If no local EAS can serve it, the EES might query the ECS to see if the service is available in another zone or if a new instance should be instantiated. Assuming an EAS is found in the local zone (Edge Zone 1 for example), the AC can then start communicating with that EAS to consume the service. Typically, the AC’s requests will still go through the EEC as a proxy (the exact data plane path can vary it could be AC → EEC (on device) → direct IP to EAS, or AC → EEC → EES → EAS, depending on whether the EES needs to anchor the traffic). In any case, the AC’s traffic to the EAS is application-layer communication (e.g. HTTP requests, video stream) and the EES/EEC ensure it is routed to the edge rather than a distant server.

4.	It’s worth noting the security aspect here: if the EAS’s API is protected (which would be the usual scenario expected), the AC/EEC can need to obtain an authorization token to invoke it. This is where CAPIF integration comes in – the EAS can expose its service API via CAPIF (with the EAS acting as an API provider and the EEC or AC as the invoker). In practice, the EEC might use CAPIF to get a token for the EAS’s API and include it in AC’s requests. However, these steps are typically hidden from the developer; they happen under the hood. The developer just sees that after discovery; the AC can call the EAS (likely the EEC provides a local API call that internally forwards to the EAS). In summary, the AC starts consuming the edge service, enjoying much lower latency because the EAS is nearby and potentially tailored to local conditions.

5.	Mobility and Service Continuity: A key feature of EDGEAPP is handling what happens when a user moves out of the coverage of one edge site to another (e.g., driving from one city to the next). Without EDGEAPP, the session to the first edge server would break or see a drastic performance drop. EDGEAPP’s design allows for Application Context Relocation (ACR) to preserve service continuity. In our example, as the UE (with AC and EEC) moves from Edge Zone 1 to Edge Zone 2, the system must transfer the ongoing session from the current EAS (in Zone 1) to a new EAS in Zone 2 (that provides the same service). The EES in Zone 1, detecting that the UE is leaving (possibly via network triggers or the UE’s EEC notifying loss of connection), coordinates with the EES in Zone 2 and the central ECS to set up a target EAS. If a suitable EAS is already running in Zone 2, it can prepare to take over the session; if not, the operator might instantiate one on the fly (depending on how dynamic the system is – this is more advanced). The state of the application (the “application context”) can be transferred from EAS1 to EAS2 – this could involve state sync or handing off relevant data (the exact method can depend on application design; EDGEAPP can signal the EAS to push state). Once Zone 2’s EAS is ready, the EES2 (Edge Enabler Server in Zone 2) will finalize the relocation: the EEC on the UE is informed of the new EAS endpoint (via a notification or updated discovery result). The AC then seamlessly continues communication with EAS2. From the AC’s viewpoint, it might just receive an event or callback that a new server endpoint is now in use – the content of the service continues with minimal disruption. The AC does not need to re-register or rediscover manually; the EEC and EES handle that transition. EDGEAPP thus ensures that mobility across edges is managed gracefully, which is critical for use cases like vehicular connectivity or users walking across a city with an AR application. This continuity is achieved while still enforcing security (if a new token is needed for the new EAS, the EEC would obtain it via CAPIF as well, transparently to the AC).

## 4.4	Service Enabler Architecture Layer (SEAL)

This clause will provide the SEAL role in the provision of reusable service enablers for vertical applications.

## 4.4	Inter-framework Relationships Overview

This clause will describe the role of each framework can provide in an overall framework.

# 5	Use Cases

Each use case is expected to include end-to-end service flow and infrastructure details (e.g., number and placement of edge nodes, optional use of SEAL services for a single PLMN domain)

## 5.1	Crowd Counting Video Analytics

### 5.1.1	Introduction

This section describes a Crowd Counting Video Analytics use case. The use case has been selected to illustrate how application developers can leverage the EDGEAPP framework to discover and utilize Edge infrastructure that supports low-latency video analytics. Specifically, it demonstrates the benefits of deploying computation close to the user equipment (UE) when processing video streams in real time.

For EDGEAPP infrastructure discovery, the CAPIF framework is leveraged to demonstrate how CAPIF exposes standardized API information that enables developers to optimize their applications for efficient utilization of edge services. Through CAPIF, an API Catalogue is made available across all Edge Zones, allowing applications to discover available zones and obtain the corresponding API endpoints required to access and consume the Video Analytics services.

### 5.1.2	Use case Description

“Acme” company is developing a crowd counting video analytics application that leverages edge computing to process video in real time. The application consists of an Application Client (AC) running on a User Equipment (UE), which captures live video and streams it for analytics, and a video analytics application running on an Edge Application Server (EAS), which performs the crowd counting function.

The performance of the video analytics function depends on latency and network efficiency. Deploying the analytics server closer to the UE can significantly reduce latency and minimize the impact of transmitting large video streams across the network.

Developers can choose between two deployment models:

-	Centralized Deployment: The video analytics server is deployed in a central data center, requiring all UEs to stream video to the central location.

-	Edge Deployment: The video analytics server is deployed within Edge Zones, allowing video streams to be processed locally, thereby reducing backhaul traffic and latency.

The UE also hosts an Edge Enabler Client (EEC). The AC communicates with the EEC via the EDGE-5 reference point. The EEC hides all network complexity and interacts with the Edge Enabler Server (EES), the Edge Configuration Server (ECS), and the CAPIF Core Function (CCF) to enable the AC to consume the video analytics service.

As depicted in Figure 5.1.1-1, the topology considered for this use case includes:

- one CCF,

- one ECS,

- two EES (each serving one edge zone),

- two EAS (one per edge zone),

- one UE with EEC and AC.

UE

AC

Central Zone

Edge Zone 2

Edge Zone 1

CCF

ECS

EES-1

EES-2

EAS-1

EAS-2

UE

EEC

AC

EEC

UE Moving from Edge Zone 1 to Edge Zone 2

Figure 5.1.1-1: Crowd Counting Video Analytics use case topology

The UE starts attached in Edge Zone 1, where the AC streams video to EAS 1. When the UE moves into Edge Zone 2, the system triggers an Application Context Relocation (ACR) to transfer the session to EAS 2, preserving service continuity with minimal disruption. The AC itself is only aware that the EEC provides it with an updated service endpoint; all control-plane interactions are performed by the EEC.

### 5.1.3	Use case Realisation over Application Enablement Frameworks

#### 5.1.3.1	General

From the developer’s point of view, it is essential to understand which entity handles which tasks in the EDGEAPP system. The AC is deliberately kept simple: it talks only to the EEC, and all network complexity is abstracted away. When the AC is designed, only the calls to the EEC’s APIs need to be implemented to leverage the EDGEAPP capabilities. Everything else — serving EES discovery, ECS interaction, and service API resolution via CAPIF — is handled transparently by the edge system. Section 5.1.3.2, though, explain the end-to-end flows for developers to understand the full system.

For this use case, we are considering centralized CAPIF core function deployment option as described in Annex A.5.3 of 3GPP TS 23.558 [4].

#### 5.1.3.2	AC Discovering the Serving EES

The EEC running in the UE, needs to discover its EES. The EEC can learn which EES to register with through multiple mechanisms:

-	Pre-provisioned in the UE/application profile.

-	DNS-based discovery, using operator domain naming conventions.

-	Network-provisioned, delivered at onboarding or during mobile attachment.

-	Bootstrap via ECS, where the EEC queries ECS to obtain the correct serving EES.

#### 5.1.3.3	CAPIF roles of EDGEAPP entities

While AC does not interact directly with CCF, it is important to understand the roles that each EDGEAPP entity plays in relation to CAPIF to consume APIs. In this use case, the EEC behaves as a CAPIF API Invoker. The EEC needs to discover EAS APIs and obtain OAuth tokens to consume them. In particular, the EEC queries the CCF to discover the Video Analytics service API exposed by the EAS.

The EEC then exposes the resolved API base URI and parameters to the AC via EDGE-5 (Eeec_EASDiscovery). The AC uses this information to send its video stream (i.e., application data traffic) directly to the EAS and expects to receive analytics results based on that. EES acts as an API Invoker consuming APIs from ECS and is an API provider as it exposes EES APIs to ECS. EAS acts as an API Invoker consuming APIs from EES and is an API provider as it exposes EAS APIs plus Application specific APIs. ECS acts as API Provider as it exposes APIs to EES.

With these considerations, the next section provides an end-to-end view with step-by-step flows describing all EDGEAPP and CAPIF entities and how they interact based in the Developers´ use of EDGE-5 APIs between AC and EEC.

### 5.1.4	Use case Flows

Editor’s note: Appropriate “Focus for Application developer” highlighting, if any, in the figures in this clause is FFS.

#### 5.1.4.1	General

From a developer’s perspective, the use case flows describe how an Application Client (AC) interacts with its local Edge Enabler Client (EEC) to request services, discover the right Edge Application Server (EAS), and sustain service continuity when the UE moves between edge zones. The AC is abstracted from all network-level complexity; it only consumes the EDGE-5 APIs provided by the EEC. Behind the scenes, the EEC engages with the EES, ECS, and CAPIF to resolve service endpoints, discover APIs, and coordinate relocation. Each step below outlines the function, rationale, and developer implications.

#### 5.1.4.2	Step 1: AC Registration & Provisioning

The first step is for the AC to register with the EEC so that it can start consuming services. The developer must ensure the AC calls the Eeec_ACRegistration API. If the EEC already aware of its serving EES (through provisioning, DNS discovery, or cached state), it registers directly as depicted in Figure 5.1.4.2-1. Otherwise, the EEC bootstraps via the ECS to discover the serving EES as depicted in Figure 5.1.4.2-2. This step establishes the control context needed for subsequent service requests.

Figure 5.1.4.2-1: AC registration in EEC (option 1)

Alternatively, EEC can discover a suitable EES from the ECS.

Figure 5.1.4.2-2: AC registration in EEC (option 2)

#### 5.1.4.3	Step 2: EAS Discovery

Once registered, the AC needs to know which EAS can provide the video analytics service. The developer calls Eeec_EASDiscovery, and the EEC queries the EES to find suitable EAS instances in the serving zone. The response includes candidate EAS endpoints, which the EEC returns to the AC via callback. The flow is shown in Figure 5.1.4.3-1.

At this stage, the AC learns where to send its data, but not yet how to use the service API.

Figure 5.1.4.3-1: EAS Discovery

#### 5.1.4.4	Step 3: CAPIF API Discovery

The AC now requires the actual API contract to interact with the selected EAS. The developer does not handle CAPIF directly; instead, the EEC calls CAPIF_Discover_Service_API towards the CCF. CAPIF responds with the API base URI, supported operations, and authorization details. The EEC then informs the AC via a response callback. The flow is shown in Figure 5.1.4.4-1.

The developer can now use this endpoint for direct service invocation.

Figure 5.1.4.4-1: CAPIF API Discovery

EEC notifies the API endpoint information obtained from CAPIF.

#### 5.1.4.5	Step 4: Service Invocation

With the endpoint and API details, the AC begins sending its video stream directly to the EAS. From the developer’s perspective, this involves implementing the client logic that captures frames and invokes the Video Analytics service API as specified by CAPIF. The EAS processes the video in real time and returns crowd counting results, which the AC can render or act upon. The flow is shown in Figure 5.1.4.5-1.

Figure 5.1.4.5-1: Video Analytics API Consumption

#### 5.1.4.6	Step 5: Mobility with Application Context Relocation

The EEC does not detect mobility itself. Mobility/zone-change is determined in the network and conveyed to the EES (e.g., via operator control-plane and exposure functions). The EES then informs the EEC over EDGE-1 using the service continuity/mobility procedures. The EEC translates that into the local EDGE-5 notification the AC can consume. From the developer’s perspective, they only need to:

-	ensure the AC has subscribed (e.g., Eeec_Services_Subscribe) to mobility/ACR notifications, and

-	handle updateNotificationCallback(mobilityEvent: zoneChange → Zone2) to gracefully pause/buffer and wait for the new endpoint before resuming the stream.

Once the EEC has notified the AC of a mobility event (zone change), the next stage is to relocate the application context. The EES is the anchor for this process. When it detects that the UE should be served by another edge zone, it coordinates with the ECS to identify the target EES/EAS. The EEC can either trigger the ACR procedure explicitly (via Eeec_ACRTrigger) or let the EES manage it automatically.

NOTE:	Each edge zone corresponds to an Edge Data Network (EDN) as defined in 3GPP TS 23.558 [4]. An EDN represents the localized network domain that hosts the edge components (EES, EAS, and supporting UPF) providing low-latency application services near the UE. From an application-enablement perspective, the edge zone is the logical area in which an Application Client (AC) is served by a specific EES and its associated EAS instances. When the UE moves to another edge zone, this corresponds to a change of serving EDN at the network level. The EES anchors the control context of each EDN and coordinates the Application Context Relocation (ACR) process to maintain service continuity across edge zones.

From the developer’s perspective, the AC does not implement relocation signalling. It only needs to:

-	handle the notification callback (from Step 4),

-	optionally request relocation via Eeec_ACRTrigger, and

-	be ready to consume the new endpoint information once relocation is complete.

The state transfer between EAS 1 and EAS 2, as well as the context push between EESs, is handled by the edge system.

Figure 5.1.4.6-1: Mobility and Application Context Relocation

#### 5.1.4.7	Step 6: API Update & Resume Streaming

After relocation, the EEC refreshes API discovery with CAPIF to retrieve the new endpoint for EAS 2. The AC receives this updated endpoint via notification callback. The developer must handle this event by re-establishing the video stream to EAS 2 using the new service API. From the AC’s perspective, this looks like a service update rather than a failure, maintaining continuity of the analytics.

Figure 5.1.4.7-1: API Update & Resume Streaming

At the end of the procedure, the Application Client (AC) has successfully transitioned its session from EAS 1 in Zone 1 to EAS 2 in Zone 2. The Application Context Relocation ensured that service state was preserved across edge zones, while the updated API discovery via CAPIF allowed the AC to obtain the new endpoint seamlessly. From the developer’s perspective, the AC continued to stream video to the analytics service with minimal disruption — first to Zone 1’s EAS and, after mobility and relocation, directly to Zone 2’s EAS. This demonstrates how the EDGEAPP framework hides the complexity of mobility and service continuity, enabling the developer to focus on application logic rather than network details.

## 5.2	{Use case title}

### 5.2.1	Introduction

### 5.2.2	Use case Description

### 5.2.3	Use case Realisation over Application Enablement Frameworks

### 5.2.4	Use case Flows

# 6	Developer Guidelines

## 6.1	Introduction

## 6.2	Cross-Framework Integration

CAPIF roles definition for EDGEAPP and SEAL entities

## 6.3	Example Application Flows

The following flows will be considered “CAPIF + EDGEAPP”, “CAPIF + SEAL” and “CAPIF + EDGEAPP + SEAL”

## 6.4	Multi-CAPIF Considerations

Relevant information to consider when several CAPIF are deployed

# 7	Security Considerations

Aspects such as identity management across frameworks, secure onboarding and trust models, framework-specific and integrated authorization will be considered

Annex A:
Infrastructure Topology Diagrams Examples

Will show CAPIF+ EDGEAPP, CAPIF+SEAL and CAPIF+EDGEAPP+SEAL

## A.1	CAPIF plus EDGEAPP Topology

Editor’s Note: Further alignment is required between this clause and Annex A.5 of 3GPP TS 23.558 [4].

### A.1.1	General

This annex describes a deployment in a single PLMN where the Common API Framework (CAPIF) integrates with the Edge Application Enablement (EDGEAPP) architecture. The scenario illustrates how onboarding, registration, API publication, discovery, authorization, and invocation are carried out with CAPIF security in place.

This sample deployment includes the following entities:

-	CAPIF Core Function (CCF): Provides onboarding, registration, API catalogue, authorization, and token issuance.

-	Edge Configuration Server (ECS): Provides supporting functions needed for the EEC to connect with an EES.

-	Two Edge Enablement Servers (EES): One per Edge location.

-	Two Edge Application Servers (EAS): One per Edge location, co-located with each EES.

-	Two User Equipment (UEs): Each has an Edge Enabler Client (EEC) and an Application Client (AC) connected to its nearest Edge location.

The Logical Topology is shown in Figure A.1.1-1, where the letters and numbers are independent, for instance AC-A or AC-B could connected to either EAS.

Central Zone

Edge Zone 2

Edge Zone 1

CCF

ECS

EES-1

EES-2

EAS-1

EAS-2

UE A

UE B

EEC-A

AC-A

EEC-B

AC-B

Figure A.1.1-1: CAPIF plus EDGEAPP sample topology

### A.1.2	Step 1: ECS Registration into CAPIF

The ECS needs first to be pre-provisioned with the CAPIF Root CA, the CAPIF base URL, and a bootstrap JWT/credentials, see clause 6.1 of 3GPP TS 33.122 [7]. It can then register with the CCF as an API Provider (clause 10.12.2 of 3GPP TS 23.222 [2]), establishing its trusted identity. The flow is depicted in Figure A.1.2-1.

Figure A.1.2-1: ECS Registration flow

### A.1.3	Step 2: ECS Publishes APIs

Having obtained a valid certificate, the ECS publishes its service APIs (e.g., EESRegistration, configuration APIs) into CAPIF. The flow is depicted in Figure A.1.3-1.

Figure A.1.3-1: ECS Publish API flow

### A.1.4	Step 3: EES Onboarding and Registration into CAPIF

Before onboarding, each EES is provisioned with:

-	CAPIF Root CA certificate(s), to validate TLS when connecting to the CCF.

-	CAPIF Core Function base URL, the discovery root for onboarding, registration, discovery, and token endpoints.

-	Bootstrap JWT and/or credential material, initial credentials used to authenticate to CCF for onboarding (per operator policy).

The EES establishes a secure TLS session to the CCF using the CA root, authenticates with the bootstrap token/credentials, and performs:

-	API Invoker onboarding (so it can later discover/consume ECS APIs), and

-	API Provider registration (so it can later publish its own EES-facing APIs, e.g., EASRegistration/Discovery).

The EES onboarding as an API Invoker and then registering as an API Provider flows are depicted in Figure A.1.4-1.

Figure A.1.4-1: EES onboarding and registration flow

### A.1.5	Step 4: EES Publishes Its APIs

The EES publishes APIs for EASRegistration, EASDiscovery, and UE-related assistance (EEC), as depicted in Figure A.1.5-1.

Figure A.1.5-1: EES publishing flow

### A.1.6	Step 5: EES Discovers ECS APIs

Before interacting with ECS, the EES discovers ECS’s published APIs through CAPIF, as depicted in Figure A.1.6-1.

Figure A.1.6-1: EES discovery flow

### A.1.7	Step 6: EES Registers with ECS

In order to register in ECS, EES needs to consume the discovered EESRegistration API. For that, the EES establishes a Security Context with the CCF using the client certificate issued/provisioned during API Invoker onboarding (Step 3) selecting the SecurityMethod, in this case, will be OAuth. Then, the EES requests an OAuth2 access token (Client Credentials grant) from the CCF, bound to the established Security Context, with scope for ECS APIs (EESRegistration).

The EES invokes EESRegistration on the ECS (EDGE-6) with the OAuth token in the bearer header. The ECS validates the token and completes EES registration into ECS.

These steps are depicted in Figure A.1.7-1.

Figure A.1.7-1: EES registration flow

### A.1.8	Step 7: EAS Onboarding and Registration into CAPIF

Before onboarding, each EAS is provisioned with:

-	CAPIF Root CA certificate(s): to validate the CCF’s TLS server certificate.

-	CAPIF Core Function base URL(s): the root(s) for CAPIF onboarding, discovery/catalogue, and token endpoints.

-	Bootstrap JWT and/or credential material: initial credentials issued by the operator/CCF that authorize the EAS to onboard; may encode roles for API Invoker and API Provider. JWT is different for API Invoker and API Provider roles.

The EAS establishes a TLS session to the CCF (verifying with the CAPIF Root CA). It authenticates using the bootstrap token/credentials and performs:

-	API Invoker onboarding, so it can later discover and consume ECS and EES APIs

-	API Provider registration, so it can later publish its own application APIs to CAPIF

The EAS onboarding as an API Invoker and then registering as an API Provider flows are depicted in Figure A.1.8-1.

Figure A.1.8-1: EAS Onboarding and Registration flows

### A.1.9	Step 8: EAS Discovers ECS/EES APIs

EAS queries CAPIF to locate both ECS and its serving EES APIs, as depicted in Figure A.1.9-1.

Figure A.1.9-1: EAS discovery flows

### A.1.10	Step 9: EAS Registers with EES

The EAS requests a CAPIF-issued token scoped to EASRegistration and registers into its local EES, as depicted in Figure A.1.10-1.

Figure A.1.10-1: EAS registration flow

### A.1.11	Step 10: EAS Publishes its Application APIs

The EAS publishes its application APIs (telemetry, AR, gaming, etc.) into CAPIF, as depicted in Figure A.1.11-1.

Figure A.1.11-1: EAS Publishing flow

At this point, EDGEAPP infrastructure is setup and ready. All EDGEAPP entitles are registered in CCF and have published their API endpoints. EES and EAS have also registered in ECS for UEs to discover them using EDEAPP APIs.

### A.1.12	Step 11: AC/EEC Access EAS Services

ACs on UEs reach EAS services via EEC and EES. The EEC may leverage CAPIF-discovered info or operator-controlled EES discovery.

The EEC discovers its serving EES either through operator bootstrap configuration (pre-provisioned URL/endpoint), dynamic discovery via DNS or network signalling, or network-assisted redirection at session setup. CAPIF is not usually queried directly by the EEC for EES.

EEC sends an API Discovery request to the CCF to learn what APIs EAS has published (this aligns with Step10 where the EAS published APIs into CAPIF).

The EEC then passes EAS API information to AC. AC invokes the desired EAS API. The EAS processes the request, and responds to the AC.

These steps are depicted in Figure A.1.12-1.

Figure A.1.12-1: AC/EEC API Access flow

## A.2	CAPIF+SEAL Topology

## A.3	CAPIF+EDGEAPP+SEAL Topology

Annex B:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA6#68 | S6-253778 |  |  |  | TR Initial Version | 0.0.0 |
| 2025-08 | SA6#68 | S6-253670 S6-253778 S6-253727 |  |  |  | References Scope Introduction | 0.1.0 |
| 2025-10 | SA6#69 | S6-254653 S6-254655 S6-254733 S6-254656 S6-254740 S6-254658 |  |  |  | Frameworks intro Frameworks EDGEAPP Frameworks CAPIF Use case: Crowd Counting Use case: Crowd Counting realisation Annex CAPIF plus EDGEAPP | 0.2.0 |