| 3GPP TS 24.283 V19.0.0 (2025-09) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Mission Critical Location Management (MCLoc); Protocol specification (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	12

1	Scope	14

2	References	14

3	Definitions of terms, symbols and abbreviations	15

3.1	Terms	15

3.2	Symbols	15

3.3	Abbreviations	15

4	General	15

4.1	Overview	15

4.2	LMS Registration	16

4.3	Identities	16

4.3.1	General	16

4.3.2	MCLOC client ID	17

5	Functional entities and reference points	17

5.1	Introduction	17

5.2	Location management service functional architecture	17

5.3	Functional entities	18

5.3.1	MCLoc client (LMC)	18

5.3.2	MCLoc server (LMS)	18

5.3.3	Key management server (KMS)	19

5.3.4	Configuration management server (CMS)	19

5.4	Reference points	19

5.4.1	Reference point CSC-9 (between the LMS and the key management server)	19

5.4.2	Reference point CSC-14 (between the LMS and the LMC)	19

5.4.3	Reference point CSC-15 (between the LMS and the MC server)	19

5.4.4	Reference point CSC-22 (between the LMS and another LMS)	19

5.4.5	Reference point CSC-24 (between the LMS and the configuration management server)	19

5.4.6	Reference point IWF-4 (between the LMS and the IWF)	19

5.4.7	Reference points HTTP-1, HTTP-2 and HTTP-3	19

6	Services offered by the location management server	20

6.1	Introduction	20

6.2	LMS_Reporting service	20

6.2.1	Service description	20

6.2.2	Service operations	21

6.2.2.1	Introduction	21

6.2.2.2	RegistrationRequest	21

6.2.2.2.1	General	21

6.2.2.2.2	LMC register to LMS using RegistrationRequest operation	21

6.2.2.3	RegistrationUpdate	22

6.2.2.3.1	General	22

6.2.2.3.2	LMC updating registration using RegistrationUpdate operation	22

6.2.2.4	RegistrationDelete	23

6.2.2.4.1	General	23

6.2.2.4.2	LMC deregistering using RegistrationDelete operation	23

6.2.2.5	ConfigurationNotify	24

6.2.2.5.1	General	24

6.2.2.5.2	LMS send configuration using ConfigurationNotify operation	24

6.2.2.6	ConfigurationRequest	24

6.2.2.6.1	General	24

6.2.2.6.2	LMC request the location configuration using ConfigurationRequest operation	25

6.2.2.7	LocationReport	25

6.2.2.7.1	General	25

6.2.2.7.2	LMS send location request using LocationReportRequest	25

6.2.2.8	LocationReportPublish	26

6.2.2.8.1	General	26

6.2.2.8.2	LMC reports to LMS using LocationReportPublish	26

6.3	LMS_Information service	29

6.3.1	Service description	29

6.3.2	Service operations	30

6.3.2.1	Introduction	30

6.3.2.2	LocationRequest	30

6.3.2.2.1	General	30

6.3.2.2.2	Client request location information to LMS using LocationRequest operation	30

6.3.2.3	LocationInfoSubscribe	33

6.3.2.3.1	General	33

6.3.2.3.2	Client request subscription using LocationInfoSubscribe operation	33

6.3.2.4	LocationInfoNotify	34

6.3.2.4.1	General	34

6.3.2.4.2	LMS Notify clients using LocationInfoNotify operation	34

6.3.2.5	LocationInfoSubscribeUpdate	35

6.3.2.5.1	General	35

6.3.2.5.2	Client requests update of subscription using LocationInfoSubscribeUpdate operation	35

6.3.2.6	LocationInfoSubscribeDelete	36

6.3.2.6.1	General	36

6.3.2.6.2	Client requests unsubscribe using LMS_LocationInfoSubscribeDelete operation	36

6.3.2.7	LocationInfoConfigRequest	37

6.3.2.7.1	General	37

6.3.2.7.2	Client request location configuration change for another client from LMS using LocationInfoConfigRequest operation	37

7	LMS API Definitions	38

7.1	LMS_Reporting API	38

7.1.1	Introduction	38

7.1.2	Usage of HTTP	39

7.1.2.1	General	39

7.1.2.2	HTTP standard headers	39

7.1.2.2.1	General	39

7.1.2.2.2	Content type	39

7.1.2.3	HTTP custom headers	39

7.1.3	Resources	39

7.1.3.1	Overview	39

7.1.3.2	Resource: LMC Registrations	41

7.1.3.2.1	Description	41

7.1.3.2.2	Resource Definition	41

7.1.3.2.3	Resource Standard Methods	41

7.1.3.2.4	Resource Custom Operations	42

7.1.3.3	Resource: Individual LMC registration	42

7.1.3.3.1	Description	42

7.1.3.3.2	Resource Definition	42

7.1.3.3.3	Resource Standard Methods	42

7.1.3.3.4	Resource Custom Operations	44

7.1.3.4	Resource: LMC Configurations	45

7.1.3.4.1	Description	45

7.1.3.4.2	Resource Definition	45

7.1.3.4.3	Resource Standard Methods	45

7.1.3.4.4	Resource Custom Operations	45

7.1.3.5	Resource: LMC Location Reports	46

7.1.3.5.1	Description	46

7.1.3.5.2	Resource Definition	46

7.1.3.5.3	Resource Standard Methods	46

7.1.3.5.4	Resource Custom Operations	47

7.1.4	Custom Operations without associated resources	47

7.1.5	Notifications	47

7.1.5.1	General	47

7.1.5.2	ConfigurationNotify notification	47

7.1.5.2.1	Description	47

7.1.5.2.2	Target URI	47

7.1.5.2.3	Standard Methods	47

7.1.5.3	LocationReportRequest notification	48

7.1.5.3.1	Description	48

7.1.5.3.2	Target URI	48

7.1.5.3.3	Standard Methods	48

7.1.6	Data Model	48

7.1.6.1	General	48

7.1.6.2	Structured data types	49

7.1.6.2.1	Introduction	49

7.1.6.2.2	Type: LMCRegistration	50

7.1.6.2.3	Type: LocationConfiguration	50

7.1.6.2.4	Type: TriggeringCriteria	51

7.1.6.2.5	Type: CellChange	51

7.1.6.2.6	Type: AnyCellChange	51

7.1.6.2.7	Type: SpecificCellChange	52

7.1.6.2.8	Type: EutranCell	52

7.1.6.2.9	Type: NrCell	52

7.1.6.2.10	Type: GeographicalAreaChange	52

7.1.6.2.11	Type: MbsfnAreaChange	53

7.1.6.2.12	Type: AnyMbsfnAreaChange	53

7.1.6.2.13	Type: SpecificMbsfnArea	53

7.1.6.2.14	Type: MbsfnAreaId	53

7.1.6.2.15	Type: MbmsSaChange	53

7.1.6.2.16	Type: AnyMbmsSaChange	54

7.1.6.2.17	Type: SpecificMbmsSaIdentity	54

7.1.6.2.18	Type: MbmsSaIdentity	54

7.1.6.2.19	Type: NR5GMbsfsaAreaChange	54

7.1.6.2.20	Type: AnyNR5GMbsfsaAreaChange	54

7.1.6.2.21	Type: SpecificNR5GMbsfsaArea	55

7.1.6.2.22	Type: NR5GMbsfsaAreaIdentity	55

7.1.6.2.23	Type: PeriodicReport	55

7.1.6.2.24	Type: PlmnChange	55

7.1.6.2.25	Type: AnyPlmnChange	55

7.1.6.2.26	Type: SpecificPlmnChange	56

7.1.6.2.27	Type: Plmn	56

7.1.6.2.28	Type: TrackingAreaChange	56

7.1.6.2.29	Type: AnyTrackingAreaChange	56

7.1.6.2.30	Type: SpecificTrackingAreaChange	56

7.1.6.2.31	Type: TrackingArea	57

7.1.6.2.32	Type: TravelledDistance	57

7.1.6.2.33	Type: RequestedLocationInfo	57

7.1.6.2.34	Type: AdaptiveTrigger	58

7.1.6.2.35	Type: AnyAreaChange	60

7.1.6.2.36	Type: SpecificArea	60

7.1.6.2.37	Type: RatTypeChange	60

7.1.6.2.38	Type: AnyRatTypeChange	60

7.1.6.2.39	Type: AnyAreaChange	60

7.1.6.2.40	Type: SignallingEvent	61

7.1.6.2.41	Type: FunctionalAliasActivation	61

7.1.6.2.42	Type: FunctionalAliasDeactivation	61

7.1.6.2.43	Type: InitialLogOn	61

7.1.6.2.44	Type: GroupCallNonEmergency	61

7.1.6.2.45	Type: PrivateCallNonEmergency	62

7.1.6.2.46	Type: LocationConfigurationReceived	62

7.1.6.2.47	Type: LocationRequest	62

7.1.6.2.48	Type: RequestLocationHistory	63

7.1.6.2.49	Type: MCUELocation	63

7.1.6.2.50	Type: LocationInfo	64

7.1.6.2.51	Type: CurrentCoordinate	64

7.1.6.2.52	Type: LocationHistoryData	64

7.1.6.2.53	Type: LocationHistoryEvent	65

7.1.6.2.54	Type: LocationHistoryStatus	65

7.1.6.3	Simple data types and enumerations	65

7.1.6.3.1	Introduction	65

7.1.6.3.2	Simple data types	65

7.1.6.3.3	Enumeration: ConfigScope	65

7.1.6.3.4	Enumeration: ReportType	66

7.1.6.3.5	Enumeration: InterRatType	66

7.1.7	Error Handling	66

7.1.7.1	General	66

7.1.7.2	Protocol Errors	66

7.1.7.3	Application Errors	66

7.1.8	Feature negotiation	67

7.1.9	Security	67

7.1.10	HTTP redirection	67

7.2	LMS_Information API	75

7.2.1	Introduction	75

7.2.2	Usage of HTTP	75

7.2.2.1	General	75

7.2.2.2	HTTP standard headers	75

7.2.2.2.1	General	75

7.2.2.2.2	Content type	75

7.2.2.3	HTTP custom headers	75

7.2.3	Resources	76

7.2.3.1	Overview	76

7.2.3.2	Resource: Client location information request	77

7.2.3.2.1	Description	77

7.2.3.2.2	Resource Definition	77

7.2.3.2.3	Resource Standard Methods	77

7.2.3.3.4	Resource Custom Operations	78

7.2.3.3	Resource: Location information subscriptions	78

7.2.3.3.1	Description	78

7.2.3.3.2	Resource Definition	78

7.2.3.3.3	Resource Standard Methods	78

7.2.3.4	Resource: Individual location information subscription	79

7.2.3.4.1	Description	79

7.2.3.4.2	Resource Definition	79

7.2.3.4.3	Resource Standard Methods	80

7.2.3.4.4	Resource Custom Operations	82

7.2.3.5	Resource: Location information config request	82

7.2.3.5.1	Description	82

7.2.3.5.2	Resource Definition	82

7.2.3.5.3	Resource Standard Methodss	82

7.2.4	Custom Operations without associated resources	83

7.2.5	Notifications	83

7.2.5.1	General	83

7.2.5.2	Location information notification	83

7.2.5.2.1	Description	83

7.2.5.2.2	Target URI	83

7.2.5.2.3	Standard Methods	84

7.2.6	Data Model	84

7.2.6.1	General	84

7.2.6.2	Structured data types	84

7.2.6.2.1	Introduction	84

7.2.6.2.2	Type: ClientOriginatingLocationRequest	85

7.2.6.2.3	Type: MCUserLocation	85

7.2.6.2.4	Type: LocationNotification	85

7.2.6.2.5	Type: LocationSubscriptionRequest	86

7.2.6.3	Simple data types and enumerations	86

7.2.7	Error Handling	86

7.2.7.1	General	86

7.2.7.2	Protocol Errors	86

7.2.7.3	Application Errors	86

7.2.8	Feature negotiation	86

7.2.9	Security	87

7.2.10	HTTP redirection	87

Annex A (normative): OpenAPI specification	87

A.1	General	87

A.2	LMS_Reporting API	87

A.3	LMS_Information API	106

Annex B (informative): Change history	114

For definitive guidance on drafting 3GPP TSs and TRs, see 3GPP TS 21.801 supplemented by the 3GPP web page http://www.3gpp.org/specifications-groups/delegates-corner/writing-a-new-spec.

Ensure all blue guidance text is removed before submitting the TS/TR to the TSG for approval.

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocols needed to support the Mission Critical Location Management service (MCLoc).

Mission critical services are services that require particular features and preferential handling compared to normal telecommunication services, e.g., in support of police or fire brigade.

The MCLoc service can be used for public safety applications as well as for some specialized commercial applications (e.g., utility companies and railways).

In the 3GPP systems, the Mission Critical Location Management service described in this document is instantiated at the application layer and involves interactions between MCLoc application clients on UEs and MCLoc application servers.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.900: "Technical Specification Group working methods".

[2]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[5]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[6]	3GPP TS 23.283: "Mission Critical Communication Interworking with Land Mobile Radio Systems; Stage 2".

[7]	3GPP TS 24.281: "Mission Critical Video (MCVideo) signalling control; Protocol specification".

[8]	3GPP TS 24.282: "Mission Critical Data (MCData) signalling control; Protocol specification".

[9]	3GPP TS 24.379: "Mission Critical Push to Talk (MCPTT) call control Protocol specification".

[10]	3GPP TS 24.482: " Mission Critical Services (MCS) identity management; Protocol specification".

[11]	3GPP TS 24.483: "Mission Critical Services (MCS) Management Object (MO)".

[12]	3GPP TS 24.484: "Mission Critical Services (MCS) configuration management Protocol specification".

[13]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[14]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[15]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[16]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[17]	3GPP TS 33.180: "Security of the mission critical service".

[18]	IETF RFC 9562: "Universally Unique IDentifiers (UUIDs)".

[19]	IETF RFC 6750: "The OAuth 2.0 Authorization Framework: Bearer Token Usage".

[20]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[21]	IETF RFC 9113: "HTTP/2".

[22]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

# 3	Definitions of terms, symbols and abbreviations

This clause and its three subclauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [2], 3GPP TS 23.280 [3] or the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [2] or 3GPP TS 23.280 [3].

Location management client: An entity typically associated with a mobile device (UE), that can request and receive location information from a location management server and can provide location information to the location management server.

Location management server: An entity that can request and receive location information from location management clients, from other location management servers and from IWFs and can provide location information to authorized location management clients, other location management servers and IWFs.

MCLOC client ID: is a globally unique identification of a specific location management client instance. MCLOC client ID is a UUID URN as specified in IETF RFC 9562 [18].

## 3.2	Symbols

No symbols are defined in the present document.

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [2], 3GPP TS 23.280 [3], or the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [2] or 3GPP TS 23.280 [3].

IWF	Interworking Function between a 3GPP Mission Critical system and an LMR system

LMR	Land Mobile Radio

# 4	General

## 4.1	Overview

The Mission Critical Location (MCLoc) is an optional auxiliary mission critical service operating at the application plane layer and supporting the MC services: MCPTT, MCVideo and MCData. It is fully integrated with those services (see 3GPP TS 23.280 [3]) and is consistent with the general design principles of the mission critical system. Salient features include:

-	Trust domains. MCLoc protects the privacy and confidentiality of operation for MC service users, including their identities, by having the ability to restrict the ingestion and dissemination of related information from/to only well trusted entities.

-	Controlled list of users. MCLoc provides services only to associates of mission critical organizations (MCO) who are subscribed through their respective MCO. The MCOs have business agreements with the mission critical system operator, which avail them of control mechanisms such as pre-configuration, policy rules and user and service profiles.

-	Multiple devices. From a user perspective, the identification and addressing is by user, rather than by device. Multiple active devices simultaneously active for the same user are supported, allowing addressing to a user at a particular active device.

-	Support for aliases. MCLoc supports addressing by, reporting from, and notifications to, users identified via mission critical aliases.

-	Support of group operations. Group communications is a key feature of mission critical systems. The mission critical location service is in constant contact with specific mission critical services (i.e., MCPTT, MCVideo, MCData) and, when instructed, can limit the location data collection and distribution to the members of the respective MC service groups, based on their current status (e.g., currently affiliated, currently in an active communication).

-	Rich set of location trigger types and variables. MCLoc supports location event triggering on many various conditions and reporting of values of location variables, beyond just longitude and latitude: for example, z-axis values, speed, etc. In addition, the time and accuracy of the measurement or data acquisition is also recorded and can be reported by the LMC and/or notified by the LMS.

-	Context sensitive and adaptive operation. MCLoc can be informed by various subsystems of changes in the context of operations (e.g., emergency situations, changes of RRC states, etc.). In those cases, MCLoc may enable/disable certain location triggers, use different set of parameters, and report/notify different location variables.

-	Off-network / on-network operation. The location of off-network MC users can also be determined using ProSe communications. If ProSe UE-to-Network relays are employed, the off-network and on-network populations of MC users can be notified of stay updated on each other locations, as appropriate.

-	Interworking with LMR systems. Depending on the location features supported by an interworking LMR system (if any), MCLoc may be able to receive, process, and store location information from the LMR system and disseminate it to MC service users. If the LMR system supports notifications, location information about the MC service users could be sent via the IWF to the LMR system.

## 4.2	LMS Registration

In the context of this specification the LMC needs to register to the LMS prior the MCLoc services can be made available to the MC user. When the LMC has registered to the LMS, the LMC may continuously get location configuration notifications and location requests from the LMS. The LMS registration shall include a LMC contact address (as a callback URI) used by the LMS to send the requests or notifications to the LMC.

The time period during which an LMC is registered may end automatically when all other MC services have deregistered or ended their operation, at the end of the registration expiry time, or, based on local policy and configuration, may continue in order to assist with locating lost or misplaced MC UEs.

## 4.3	Identities

### 4.3.1	General

The LMC is provided with MC service IDs as defined 3GPPP TS 23.280 [3] from the IdMS during user authentication procedure according to 3GPP TS 33.180 [17]. The MC service IDs are provided in the access tokens and ID tokens. The MC UE can get one or several access tokens and ID tokens from the IdMS depending on implementation. The MC service IDs are used in the three MC services and there is no dedicated MC service ID used by the location management server or client.

In the interaction between the LMC and LMS (e.g. location information reports) the LMC shall provide all MC service IDs that are currently available to the LMC. Other entities requesting location information for a specific MC user might only know a specific MC service ID for the MC user of interest.

Additionally, a LMC sending a request to a LMS shall also include the MCLOC client ID as defined in clause 4.3.2. The MCLOC client ID is used to distinguish multiple location management clients used by a single MC user.

### 4.3.2	MCLOC client ID

The LMC assigns the MCLOC client ID when the LMC is used for the first time. The LMC generates the MCLOC client ID as specified in clause 4.2 of IETF RFC 9562 [18].

The LMC preserves the MCLOC client ID:

-	while the LMC is service authorized in the LMS;

-	while the LMC is not service authorized in the LMS and the UE serving the MCLOC client is switched on;

-	while the UE serving the MCLOC client is switched off; and

-	while the UE serving the MCLOC client is power-cycled.

NOTE:	MCLOC client ID is not preserved when the UE is reset to factory settings.

Other network entities (e.g. IWF, MC service servers, other LMS) that consume location information shall also assign a MCLOC client ID to be used in the APIs defined in this specification.

# 5	Functional entities and reference points

## 5.1	Introduction

This clause describes the association of functional entities within a mission critical system with their roles in the mission critical location management service, consistent with the stage 2 architecture document (see 3GPP TS 23.280 [3]).

## 5.2	Location management service functional architecture

Figure 5.2-1 shows the functional model for the location management service in the application plane, based on the architecture in 3GPP TS 23.280 [3] clause 7.3.

Figure 5.2-1: Location management service functional architecture

The protocol used on reference point CSC-9 and reference point CSC-24 is not part of this specification.

## 5.3	Functional entities

### 5.3.1	MCLoc client (LMC)

The MCLoc client can exist in a UE or other device, such as a dispatcher's console. The MCLoc client:

-	interacts with the user playing the role of user agent for all location management application transactions;

-	interacts with the hosting device (e.g., UE) and its subsystems as well as with other mission critical clients co-located in the same device, to obtain necessary information; and

-	interacts with the MCLoc server, executing procedures defined in 3GPP TS 23.280 [3] and in the present document to register, obtain service authorization, request, receive and report location information,

### 5.3.2	MCLoc server (LMS)

The MCLoc server executes location related procedures defined in 3GPP TS 23.280 [3] and in the present document. Specifically, the MCLoc server:

-	interacts with the LMCs to handle requests for location information that it needs to acquire or has stored;

-	interacts with the LMCs to handle reports of location information;

-	interacts with MC service servers acting as authorized LMCs, to handle requests for location information that it needs to acquire or has stored;

-	interacts with MC service servers acting as authorized LMCs, to acquire and store location information that was reported to the MC service servers by MC service clients;

-	interacts with KMS and CMS to obtain proper functional parameters, as necessary;

-	interacts with other LMSs to request or provide location information; and

-	interacts with an IWF to request or provide location information.

### 5.3.3	Key management server (KMS)

See 3GPP TS 23.280 [3] clause 7.4.2.2.8.

### 5.3.4	Configuration management server (CMS)

See 3GPP TS 23.280 [3] clause 7.4.2.2.2.

## 5.4	Reference points

### 5.4.1	Reference point CSC-9 (between the LMS and the key management server)

The CSC-9 reference point, which exists between the LMS and the key management server, is described in 3GPP TS 23.280 [3] clause 7.5.2.9.

### 5.4.2	Reference point CSC-14 (between the LMS and the LMC)

The CSC-14 reference point, which exists between the LMC and the LMS, is described in 3GPP TS 23.280 [3] clause 7.5.2.14.

### 5.4.3	Reference point CSC-15 (between the LMS and the MC server)

The CSC-15 reference point, which exists between the LMS and the MC server, is described in 3GPP TS 23.280 [3] clause 7.5.2.15.

### 5.4.4	Reference point CSC-22 (between the LMS and another LMS)

The CSC-22 reference point, which exists between the LMS and another LMS, is described in 3GPP TS 23.280 [3] clause 7.5.2.23.

### 5.4.5	Reference point CSC-24 (between the LMS and the configuration management server)

The CSC-24 reference point, which exists between the LMS and the configuration management server, is described in 3GPP TS 23.280 [3] clause 7.5.2.25.

### 5.4.6	Reference point IWF-4 (between the LMS and the IWF)

The IWF-4 reference point, which exists between the LMS and the IWF, is described in 3GPP TS 23.283 [6] clause 7.4.4.

### 5.4.7	Reference points HTTP-1, HTTP-2 and HTTP-3

The HTTP-1, HTTP-2 and HTTP-3 reference points, which support the location management system in the signalling control plane (not shown in Figure 5.2-1) are described in 3GPP TS 23.280 [3] clause 7.5.3.

# 6	Services offered by the location management server

## 6.1	Introduction

The table 6.1-1 lists the location management server APIs below the service name. A service description clause for each API gives a general description of the related API.

Table 6.1-1: API Descriptions

| Service Name | Service Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| LMS_Reporting | RegistrationRequest | Request/Response | LMC |
|  | RegistrationUpdate | Request/Response | LMC |
|  | RegistrationDelete | Request/Response | LMC |
|  | ConfigurationNotify | Request/Response | LMS |
|  | ConfigurationRequest | Request/Response | LMC |
|  | LocationReportRequest | Request/Response | LMS |
|  | LocationReportPublish | Request/Response | LMC |
| LMS_Information | LocationInfoRequest | Request/Response | LMC, MC service server, IWF, LMS |
|  | LocationInfoSubscribe | Subscribe/Notify | LMC, MC service server, IWF, LMS |
|  | LocationInfoNotify | Subscribe/Notify | LMS |
|  | LocationInfoSubscribeUpdate | Subscribe/Notify | LMC, MC service server, IWF, LMS |
|  | LocationInfoSubscribeDelete | Subscribe/Notify | LMC, MC service server, IWF, LMS |
|  | LocationInfoConfigRequest | Request/Response | LMC, MC service server, IWF, LMS |

Table 6.1-2 summarizes the corresponding LMS APIs defined in this specification.

Table 6.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| LMS_Reporting | 7.1 | LMS registration and reporting | TS24283_Lms_Reporting.yaml | lms-reporting | A.2 |
| LMS_Information | 7.2 | LMS location information | TS24283_Lms_Information.yaml | lms-information | A.3 |

## 6.2	LMS_Reporting service

### 6.2.1	Service description

The LMS_Reporting service allows the LMC to:

-	Register, update the registration and to deregister in the LMS;

-	Request location configuration and receive location configuration; and

-	Receive location information requests and to send location information reports.

At registration the LMS performs the service authorization according to 3GPP TS 33.180 [17].

### 6.2.2	Service operations

#### 6.2.2.1	Introduction

The service operation defined for LMS_Registration service is shown in the table 6.2.2.1-1.

Table 6.2.2.1-1: Operations of the LMS_LocationRegistration service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| RegistrationRequest | This service operation is used by the LMC to register in the LMS. | LMC |
| RegistrationUpdate | This service operation is used by the LMC to update a registration in the LMS. | LMC |
| RegistrationDelete | This service operation is used by the LMC to deregister in the LMS. | LMC |
| ConfigurationNotify | This service operation is a callback request used by the LMS to send location configuration to the LMC. | LMS |
| ConfigurationRequest | This service operation is used by the LMC to request the location configuration. | LMC |
| LocationReportRequest | This service operation is a callback request used by the LMS to send a location information request to the LMS. | LMS |
| LocationReportPublish | This service operation is used by the LMC to report the location information of the MC UE to the LMS. | LMC |

#### 6.2.2.2	RegistrationRequest

##### 6.2.2.2.1	General

This service operation is used by LMC to register itself in a given LMS.

##### 6.2.2.2.2	LMC register to LMS using RegistrationRequest operation

For a LMC to register in the LMS, the LMC shall send an HTTP POST message to the LMS on the LMS_Reporting collection resource (i.e., LMC registration) to create the resource associated to the registration in the LMC.

The request URI shall be set according to clause 7.1.1.

The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LMS registration is relevant for all valid MC service IDs, hence the request includes all access tokens.

Other HTTP headers shall be set according to clause 7.1.2.2.

The body of the HTTP POST message shall include the LMCRegistration as specified in clause 7.1.6.2.2. The LMCRegistration datatype:

1)	shall include an expTime attribute containing the time and date for the registration expiry;

2)	shall include a mclocClientId attribute identifying the instance of the LMC;

3)	shall include a locationConfigUri attribute specifying the URI of the LMC to which location configuration request shall be sent to;

4)	shall include a locationRequestUri attribute specifying the URI of the LMC to which location request shall be sent to;

5)	may include a pei attribute specifying the imei of the MC UE; and

6)	may include a supi attribute specifying the supi of the subscription.

Upon receiving the HTTP POST message from the LMC, the LMS:

1)	shall validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	shall consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the RegistrationRequest;

3)	if the any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps; and

4)	if the access token validation is successful, then the LMS:

a)	shall create a new resource with the LMC registration information as specified in clause 7.1.3.3.2 together with all LMS authorized MC service IDs;

b)	may initiate a ConfigurationNotify notification as specified in clause 7.1.5.2; and

c)	shall send a HTTP 201 Created response message to the LMC with the following clarifications:

i)	include the LMC registration information in the body of the HTTP 201 (Created) message;

ii)	the response message may include an updated expiration time to indicate when the LMC registration will automatically expire; and

NOTE: The updated expiration time can be shorter than what was initially provided by the LMC.

iii)	the URI of the created resource shall be returned in the "Location" HTTP header.

While the LMC is registered in the LMS, the LMS may repeatedly send:

1)	ConfigurationNotify as specified in clause 6.2.2.5; and

2)	LocationReportRequest as specified in clause 6.2.2.7.

The LMC shall send a Registration Update request (as described in clause 6.2.2.3) prior to the expiration time, to maintain the registration state. If a successful Registration Update request is not received prior to the expiration time, then the LMS shall treat LMC as implicitly deregistered and remove the corresponding LMC registration resource.

The LMS may use the value of pei or supi attribute when interacting over the Le reference point to request location information from the LCS network, see reference 3GPP TS 23.271 [4] and 3GPP TS 23.273 [5].

#### 6.2.2.3	RegistrationUpdate

##### 6.2.2.3.1	General

This service operation is used by the LMC to update its registration information at the LMS.

##### 6.2.2.3.2	LMC updating registration using RegistrationUpdate operation

To update the LMC registration information at the LMS, the LMC shall send an HTTP PUT message to the LMS on resource URI identifying the individual LMC registration resource representation as specified in clause 7.1.3.3.2. The HTTP PUT message shall replace all properties of the existing resource with the LMC registration information in the request.

The request URI shall be set according to clause 7.1.1.

The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LMS registration is relevant for all valid MC service IDs, hence the request includes all access tokens.

Other HTTP headers shall be set according to clause 7.1.2.2.

The body of the HTTP PUT message shall include the LMCRegistration as specified in clause 7.1.6.2.2. The LMCRegistration datatype shall include:

1)	an expTime attribute containing the time and date for the registration expiry;

2)	a mclocClientId attribute identifying the instance of the LMC as defined in clause 4.3.2;

3)	a locationConfigUri attribute specifying the URI of the LMC to which location configuration request shall be sent to; and

4)	a locationRequestUri attribute specifying the URI of the LMC to which location request shall be sent to.

Upon receiving the PUT message from the LMC, if the resource URI does not exist, the LMS shall respond 404 Not Found error to the LMC. Otherwise, the LMS:

1)	shall validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	shall consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the RegistrationRequest;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps; and

4)	if the access token validation is successful, then:

a)	the LMS shall update the resource with the LMC registration information as specified in clause 7.1.3.3.2 together with all LMS authorized MC service IDs; and

b)	shall return the updated LMC registration information in the response. In the response message, the LMS may send "200 OK" response code to provide an updated expiration time to indicate to the LMC when the updated registration will automatically expire. The expTime attribute shall override the current expTime attribute maintained by the LMC. Otherwise, the LMS sends "204 No Content" response code.

The LMC shall send a RegistrationUpdate request prior to the expiration time, to maintain the registration state. If a successful RegistrationUpdate request is not received prior to the expiration time, then the LMS shall treat LMC as implicitly deregistered and remove the corresponding LMC Registration resource.

#### 6.2.2.4	RegistrationDelete

##### 6.2.2.4.1	General

This service operation is used by LMC to deregister itself from a given LMS.

##### 6.2.2.4.2	LMC deregistering using RegistrationDelete operation

To deregister itself from the LMS, the LMC shall send HTTP DELETE message to the LMS, on the resource URI identifying the individual LMC registration resource representation as specified in clause 7.1.3.3.2.

The request URI shall be set according to clause 7.1.1.

The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to the access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LMS registration is relevant for all valid MC service IDs, hence the request includes all access tokens.

Other HTTP headers shall be set according to clause 7.1.2.2.

Upon receiving the HTTP DELETE request, the LMS shall:

1)	verify the identity of the LMC and check if the LMC is authorized to deregister the LMS registration information; and

2)	if the LMC is authorized to deregister the LMS registration information, then the LMS shall

a)	if the resource identified by registrationId is not found, return "404 Not Found" error message to the LMC;

b)	otherwise, deregister the LMC from the LMS and delete the resource representing LMC registration information; and

c)	return the "204 No Content" message to the LMC, indicating the successful deregistration of the LMC information.

#### 6.2.2.5	ConfigurationNotify

##### 6.2.2.5.1	General

This service operation is used by LMS to notify the LMC about a new location configuration.

##### 6.2.2.5.2	LMS send configuration using ConfigurationNotify operation

The LMS may at any time determine to notify a registered LMC about a change in the location configuration. The location configuration is sent to the LMC according to this procedure.

To notify the LMC about a location configuration, the LMS shall send HTTP POST message to the LMC, using the Notification Destination URI received in the registration request, as specified in clause 7.1.5.2.2.

The body of the HTTP POST message shall include the LocationConfiguration data type set to the desired configuration as specified in clause 7.1.6.2.3, and with the following clarifications:

1)	shall include the triggerId attribute, with its already assigned value, if existent and valid, or with a newly generated unique value, if not, where defined for the sub-elements defining the trigger criterion;

2)	may include the mcServiceId attribute set to the identity of the user requesting the location information configuration;

3)	may include the configScope attribute set to "FULL" or "UPDATE". The value "FULL" means that the configuration attributes contains the full location configuration which replaces any previous location configuration. The value "UPDATE" means that the location configuration is in addition to any previous location configuration. To remove configuration elements a "Full" configuration is needed; and

4)	may include the locationHistory attribute set to true to indicate that location information shall be stored in the LMC during the state when the LMC is not able to report location information. If the attribute is not included or set to false location information may not be stored in the LMC if the LMC fails to report location information.

Upon receiving the HTTP POST request, the LMC shall:

When the LMC receives that location configuration, then the LMC:

1)	shall replace or modify the stored contents of the locationConfiguration attribute, consistent with the attribute configScope value, and then store the location configuration;

NOTE:	The LMC only needs to store the latest received location reporting configuration.

2)	shall set the location reporting triggers accordingly;

3)	shall start the minimumReportInterval timer; and

4)	if the locationHistory attribute is set to true and if location information reports cannot be successfully sent to the LMS then the LMC shall locally store the location information reports in the MC UE for later submissions.

#### 6.2.2.6	ConfigurationRequest

##### 6.2.2.6.1	General

This service operation is used by LMC to request the location configuration from the LMS.

##### 6.2.2.6.2	LMC request the location configuration using ConfigurationRequest operation

The LMC may at any time request to download the location configuration from the LMS. The location configuration is sent to the LMC according to this procedure.

To request the location configuration the LMC shall send HTTP GET message to the LMS on the LMS_Reporting  collection resource (i.e., LMC configurations).

The request URI shall be set according to clause 7.1.1.

The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The configuration request is relevant for all valid MC service IDs, hence the request includes all access tokens.

Other HTTP headers shall be set according to clause 7.1.2.2.

Upon receiving the HTTP GET message from the LMC, the LMS:

1)	shall perform service authorization as described in 3GPP TS 33.180 [17];

2)	if the LMC is service authorized, then the LMS shall send a HTTP 200 OK response message to the LMC with the body including the LocationConfiguration datatype set to the configuration that applies to the LMC service ID; and

3)	If the LMC is not service authorized, the LMS shall reject the request with a HTTP 401 (Unauthorized) response.

#### 6.2.2.7	LocationReport

##### 6.2.2.7.1	General

This service operation is used by LMS to send a new location report request to a registered LMC.

##### 6.2.2.7.2	LMS send location request using LocationReportRequest

The LMS may at any time determine to send a location report request to a registered LMC.

To request a new location report from an LMC, the LMS shall send HTTP POST message to the LMC, using the Notification Destination URI received in the registration request, as specified in clause 7.1.5.3.2.

The body of the HTTP POST message shall include the LocationRequest data type as specified in clause 7.1.6.2.47, and with the following clarifications:

1)	shall include the requestedMcServiceId attribute set to the MC service identity of the MC user being requested to send a location report;

2)	shall include the requestingMcServiceId attribute set to the MC service identity of the MC user that request the location information, if the request is initiated from an LMC;

NOTE:	If the request is initiated from the LMS without initial triggered from an LMC then the requesting MCServiceId is not needed.

3)	may include the requestedFunctionalAliasId attribute set to the activated functional alias of the MC user being requested to send a location report;

4)	may include the requestingFunctionalAliasId attribute set to the activated functional alias of the MC user that request the location information;

5)	may include the requestedLocationInfo attribute including details of what location information attributes that shall be included in the location report;

6)	may include the requestedLocationHistory attribute including details of what location information history events that shall be included in the location report; and

7)	may include the requestedLocationStatus attribute indicating that the location history status should be included in the location report.

Upon receiving the HTTP POST request, the LMC shall:

When the LMC receives that location request, then the LMC:

1)	shall send a location report as specified in clause 6.2.2.8; and

2)	shall reset the minimumReportInterval timer.

#### 6.2.2.8	LocationReportPublish

##### 6.2.2.8.1	General

This service operation is used by LMC to publish a location report to a given LMS.

If a location reporting trigger fires the LMC checks if the minimumReportInterval timer is running. If the timer is running the LMC waits until the timer expires. When the minimumReportInterval timer fires, the LMC:

1)	shall, if any of the reporting triggers are still true, send a location report as specified in clause 6.2.2.8.2.

If the LMC receives a LocationReportRequest as specified in clause 7.1.5.3, the LMC shall send a location report as specified in clause 6.2.2.8.2.

##### 6.2.2.8.2	LMC reports to LMS using LocationReportPublish

For a LMC to publish a location report to the LMS, the LMC shall send an HTTP POST message to the LMS on the LMS_Reporting collection resource (i.e., LMC location reports) representing the location information of the MC UE.

The request URI shall be set according to clause 7.1.1.

The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The location report is relevant for all valid MC service IDs, hence the request includes all access tokens.

Other HTTP headers shall be set according to clause 7.1.2.2.

The body of the HTTP POST message shall include the MCUELocation data type as specified in clause 7.1.6.2.49. The MCUELocation datatype shall include:

1)	a reportID attribute if the report was triggered by a location request, set to the value of the requestID attribute in the received request;

2)	a reportType attribute indicating if the report is sent during emergency state or not;

3)	a locTimstamp attribute set to the date and time of the location measurement;

4)	an array with triggerId attributes, where each item is set to the value associated with the trigger that have fired;

5)	a mclocClientId attribute identifying the instance of the LMC; and

6)	a locationData attribute including subitems corresponding to the location configuration, and shall include at least one of the following attributes:

a)	currentCoordinate, containing the geospatial coordinate of the MC UE, with the following clarification:

i)	shall include the geographicalCoordinates attribute coded as defined in 3GPP TS 29.572 [16];

ii) may include altitude attribute coded as defined in 3GPP TS 29.572 [16];

iii)	may include the horizontalaccuracy attribute coded as defined in 3GPP TS 29.572 [16];

iv)	may include the verticalaccuracy attribute coded as defined in 3GPP TS 29.572 [16]; and

v)	may include the bearingAndSpeed attribute coded as defined in3GPP TS 29.572 [16].

b)	ecgi attribute containing the current served EUTRAN cell;

c)	neighbouringEcgi attribute containing an array of ECGI of any neighbouring cell the MC UE can detect;

d)	mbmsSaiId attribute containing the MBMS Service Area Id the LMC is located in;

e)	mbsfnArea attribute containing the MBSFN Area the LMC is located in;

f)	ncgi attribute containing the current served NR cell;

g)	neighbouringNcgi attribute containing an array of NCGI of any neighbouring cell the MC UE can detect; and

h)	nr5GMbsfsaArea attribute containing the 5G MBSFSA the MC UE is located in;

and may include:

1)	a functionalAlias attribute containing the functional alias ID related to the functional alias status change that triggered the location measurement;

2)	an interRatType attribute containing the inter-RAT change type;

3)	an mcServiceUELabel attribute containing the MC Service UE label;

4)	a locationHistoryData attribute containing multiple historical location reports if the location request indicates that historical location reports shall be included; and

5)	a locationHistoryStatus attribute containing a location history status summary.

When the Location report publish is sent the LMC shall set the minimumReportInterval timer to the minimumReportInterval time from the location configuration and start the timer.

Upon receiving the HTTP POST message from the LMC, the LMS:

1)	shall perform service authorization as described in 3GPP TS 33.180 [17];

2)	if the LMC is service authorized, then the LMS:

a)	shall store the location information received in the location report;

b)	shall use the location information as needed; and

c)	shall send a HTTP 201 Created to the LMC; and

3)	If the LMC is not service authorized, the LMS shall reject the request with a HTTP 401 (Unauthorized) response.

## 6.3	LMS_Information service

### 6.3.1	Service description

The LMS_Information service allows an LMC, MC service server, IWF or an LMS to:

-	request location information;

-	subscribe to location notifications;

-	receive location information notifications;

-	update an existing subscription for location information notification;

-	unsubscribe to an existing subscription for location information notifications; and

-	change another LMC location configuration.

### 6.3.2	Service operations

#### 6.3.2.1	Introduction

The service operation defined for LMS_Information service is shown in the table 6.3.2.1-1.

Table 6.3.2.1-1: Operations of the LMS_Information service

| Service operation name | Description | Initiated by |
| --- | --- | --- |
| LocationInfoRequest | This service operation is used to request location information from the LMS. | LMC, MC service server, IWF, LMS |
| LocationInfoSubscribe | This service operation is used by the LMC to subscribe to location information from the LMS. | LMC, MC service server, IWF, LMS |
| LocationInfoNotify | This service operation is used to send notification about location information. | LMS |
| LocationInfoSubscribeUpdate | This service operation is used by the LMC to update a subscription of location information from the LMS. | LMC, MC service server, IWF, LMS |
| LocationInfoSubscribeDelete | This service operation is used by the LMC to terminate the subscription of location information. | LMC, MC service server, IWF, LMS |
| LocationInfoConfigRequest | This services operation is used to change another LMC location configuration. | LMC, MC service server, IWF, LMS |

#### 6.3.2.2	LocationRequest

##### 6.3.2.2.1	General

This service operation is used by the LMC, MC service server, IWF or another LMS to send a location information request to an LMS. In this clause and its sub clauses the sender of the request is referred to as the client.

##### 6.3.2.2.2	Client request location information to LMS using LocationRequest operation

For a client to request location information from the LMS, the client shall send an HTTP POST message to the LMS on the LMS_Information collection resource (i.e., Client location information request) to create the resource associated to the request for location information.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, the HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LocationRequest is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the ClientOriginatingLocationRequest as specified in clause 7.2.6.2.2. The ClientOriginatingLocationRequest datatype shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location information notification shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; and

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested;

4)	may include a refresh attribute to indicate that the location information shall be updated immediately from the requested LMC;

5)	may include the requestedLocationHistory attribute including details of what location information history events that shall be included in the location report; and

6)	may include the requestedLocationStatus attribute indicating that the location history status should be included in the location report.

Editor's note: The usage of an LocationFilter attribute according to TS 24.379 is FFS.

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	shall validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the Locationrequest;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

4)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid location request target client list" in a Warning header field, and shall not continue with the rest of the steps; and

5)	if the access token validation is successful, then:

a)	the LMS shall create a new resource with the client location information request as specified in clause 7.2.3.2.3; and

b)	shall send a HTTP 201 Created response message to the LMC;

6)	determine that the initiating user is authorized to request group based location information for each MC service group ID in the array provided in the attribute mcServiceGroupIds according to the following:

a)	The initiating user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that includes:

i)	a <condition> element containing an <identity> element that matches the MC service group ID; and

ii)	an <action> element containing an <allow-request-location-of-groups> element with the value set to "true"; and

b)	if the item a) above is not fulfilled the MC service group shall be ignored in the remaining steps;

7)	subscribe to group dynamic data to for each MC service group ID that passes the authorization check in step 6) according to the following:

a)	if the MC service group is a MCPTT group send a SIP SUBSCRIBE request as specified in 3GPP TS 24.379 [9] clause 9.2.2.2.16 where the LMS takes the role of an MCPTT server serving the initiating user;

b)	if the MC service group is a MCVideo group send a SIP SUBSCRIBE request as specified in 3GPP TS 24.281 [7] clause 8.2.2.2.16 where the LMS takes the role of an MCVideo server serving the initiating user; and

c)	if the MC service group is a MCData group send a SIP SUBSCRIBE request as specified in 3GPP TS 24.282 [8] clause 8.3.3.8 where the LMS takes the role of an MCData server serving the initiating user;

8)	identify all MC service IDs for which location information is requested, based on the following content of the incoming request:

a)	the MC service IDs from the array provided in the attribute mcServiceIDs;

b)	the MC service IDs of the MC users that have activated the functional alias IDs from the array provided in the attribute functionalAliasIds

NOTE 1:	One way to do that is to use the functional alias status determination procedure defined in 3GPP TS 24.379 [9] clause 9A.2.1.3.

c)	the MC service IDs of each MC client that have affiliated to the MC service groups based on the notifications from the group dynamic data subscriptions;

9)	verify that the initiating user is authorized to request the location of the MC service IDs identified in step 6, according to local policies or to the following:

a)	The initiating user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that include:

i)	a <condition> element containing an <identity> element that match each of identified MC service ID; and

ii)	an <action> element containing an <allow-request-location-of-users> element with the value set to "true"; and

b)	if the item i) above is not fulfilled the MC service ID shall be ignored in the remaining steps;

10)	If any MC Service IDs identified in the steps above are not owned by the LMS, the LMS shall, in accordance with the procedure in this clause, generate a new HTTP request to the LMS that owns those MC Service IDs;

NOTE 3:	How the LMS determines the HTTP request URI of the LMS that owns those MC Service IDs is out of scope of this specification.

11)	verify that each target user identified from step 7 is authorized to share the location information with the initiating MC user, according to local policies or to the following:

a)	Each target user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that include:

i)	a <condition> element containing an <identity> element that match the identity of the initiating user MC service ID; and

ii)	an <action> element containing an <allow-to-share-location-information> element with the value set to "true"; and

b)	if the item i) above is not fulfilled the MC service ID shall be ignored in the remaining steps;

12)	if the refresh attribute is present and set to true, initiate LocationReportRequest operations (see clause 6.2.2.7.2) to each LMC identified in step 8).

The LMS shall create one or multiple location information notification and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message may contain location information from one or several MC UEs.

The location information notification message shall include the MC UE location details.

#### 6.3.2.3	LocationInfoSubscribe

##### 6.3.2.3.1	General

This service operation is used by the LMC to subscribe to location information at the LMS.

##### 6.3.2.3.2	Client request subscription using LocationInfoSubscribe operation

To request a subscription for location information, the client shall send an HTTP POST message to the LMS on LMS_Information collection resource (i.e, Location information subscription) to create the resources associated with the subscription in the LMS.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE 1:	The LocationInfoSubscribe operation is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the LocationSubscriptionRequest as specified in clause 7.2.6.2.5.

The LocationSubscriptionRequest data object shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location notifications shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; or

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested; and

4)	an expTime attribute containing the time and date for the subscription expiry;

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LMS_LocationInformation_request;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

Editor's note:	How to handle the 3GPP-Asserted-Identity header is FFS.

4)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid subscription request target client list" in a Warning header field, and shall not continue with the rest of the steps;

5)	if the MC user's location user configuration data document does not include a <rule> element including a <action> element including a <allow-subscription-on-location-information> element set to true the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Unauthorized for location subscription" in a Warning header field, and shall not continue with the rest of the steps;

6)	generate a unique subscription identifier;

7)	create a new resource with the location subscription request as specified in clause 7.2.3.3.3; and

8)	send a HTTP 201 Created response message to the LMC and include the subscription identifier in the Location header;

The LMS shall identify all MC service IDs for which the subscription request applies according to step 6-11 in the procedure for a one-time location information request as specified in clause 6.3.2.2.2.

The LMS shall create location information notifications and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message shall contain location information for the MC UEs of the identified MC service IDs.

The identified MC service IDs may change during the subscription time if the MC service IDs are determine based on affiliated group members or activated functional aliases.

The notifications shall be sent repeatedly until the subscription expires based on the attribute expTime or until a unsubscribe message is received.

#### 6.3.2.4	LocationInfoNotify

##### 6.3.2.4.1	General

This service operation is used by LMS to notify the clients of a location information.

##### 6.3.2.4.2	LMS Notify clients using LocationInfoNotify operation

The LMS use this procedure to notify an authorized user about other users' location information. The notification is used both because of a one-time location information request as well as repeatedly during a location information subscription.

To notify the LMC about other MC UEs' location information, the LMS shall send HTTP POST message to the LMC, using the notification URI received in the subscription request or in the location information request, as specified in clause 7.2.5.2.2.

The body of the HTTP POST message shall include the LocationNotification data type set to the MC UEs location information as specified in clause 7.2.6.2.4, and with the following clarifications:

1)	shall include the requestingMcServiceId attribute set to the identity of the user requesting the location information;

2)	may include the requestingfunctionalAliasId attribute set to the activated functional alias identity of the user requesting the location information;

3)	shall include the mcUserResponseList attribute set to an array of MCUserLocation objects for each MC user's MC UE location.

When the LMC receives that location notification then the LMC:

1)	shall send a HTTP 201 Created response message to the LMS; and

2)	may use the location information as desired.

#### 6.3.2.5	LocationInfoSubscribeUpdate

##### 6.3.2.5.1	General

This service operation is used by clients to update the subscription of a location information.

##### 6.3.2.5.2	Client requests update of subscription using LocationInfoSubscribeUpdate operation

To update an existing subscription the client shall send an HTTP PUT message to the LMS on the resource URI identifying the individual LMC subscription resource representation as specified in clause 7.2.3.4.2. The HTTP PUT message shall replace all properties of the existing resource with the LMS Location information subscribe request.

The request URI shall be set according to clause 7.2.1.

If the client is an LMC, The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE 1:	The LocationInfoSubscribeUpdate operation is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP PUT message shall include the LocationSubscriptionRequest datatype as specified in clause 7.2.6.2.5.

The LocationSubscriptionRequest data object shall include:

1)	a mclocClientId attribute identifying the instance of the LMC;

2)	a locationNotificationUri attribute specifying the URI of the LMC to which location notifications shall be sent to;

3)	at least one of the following:

a)	an array of MC service IDs of MC users for which location information are requested;

b)	an array of activated functional alias IDs used to determine MC service IDs of MC users for which location information are requested; or

c)	an array of MC service group IDs used to determine the MC service groups' affiliated MC service IDs of MC users for which location information are requested; and

4)	an expTime attribute containing the time and date for the subscription expiry;

Upon receiving the HTTP PUT message from the LMC, the LMS shall:

1)	validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LMS_LocationInformation_request;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

Editor's note:	How to handle the 3GPP-Asserted-Identity header is FFS.

4)	if the array of MC service IDs, the array of functional alias IDs and the array of MC service group IDs are all empty then the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Invalid subscription request target client list" in a Warning header field, and shall not continue with the rest of the steps;

5)	if the MC user's location user configuration data document does not include a <rule> element including a <action> element including a <allow-subscription-on-location-information> element set to true the LMS shall respond with a HTTP 403 (Forbidden) response including warning text set to "Unauthorized for location subscription" in a Warning header field, and shall not continue with the rest of the steps;

6)	update the complete subscription resource with the location subscription information as specified in clause 7.2.3.3.3; and

8)	send a HTTP 204 No Content response message to the LMC.

The LMS shall identify all MC service IDs for which the subscription request applies according to step 6-11 in the procedure for a one-time location information request as specified in clause 6.3.2.2.2.

The LMS shall create location information notifications and send the notifications to the requesting LMC according to the procedure in clause 6.3.2.4. Each location information notification message shall contain location information for the MC UEs of the identified MC service IDs.

The identified MC service IDs may change during the subscription time if the MC service IDs are determine based on affiliated group members or activated functional aliases.

#### The notifications shall be sent repeatedly until the subscription expires based on the attribute expTime or until a unsubscribe message is received. 6.3.2.6	LocationInfoSubscribeDelete

##### 6.3.2.6.1	General

This service operation is used by clients to delete a subscription of a location information.

##### 6.3.2.6.2	Client requests unsubscribe using LMS_LocationInfoSubscribeDelete operation

To delete an subscription in the LMS, the LMC shall send HTTP DELETE message to the LMS, on the resource URI identifying the individual LMC subscription resource representation as specified in clause 7.2.3.4.2.

The request URI shall be set according to clause 7.2.1.

The HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to the access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LMS subscription is relevant for all valid MC service IDs, hence the request includes all access tokens.

Other HTTP headers shall be set according to clause 7.2.2.2.

Upon receiving the HTTP DELETE request, the LMS shall:

1)	verify the identity of the LMC and check if the LMC is authorized to delete the location information subscription; and

2)	if the LMC is authorized to delete the location information subscription, then the LMS shall

a)	if the resource identified by subscription id is not found, return "404 Not Found" error message to the LMC;

b)	otherwise, delete the location information subscription from the LMS and delete the resource representing location information subscription; and

c)	return the "204 No Content" message to the LMC, indicating the successful deletion of the LMC location information subscription.

#### 6.3.2.7	LocationInfoConfigRequest

##### 6.3.2.7.1	General

This service operation is used by the LMC, MC service server, IWF or another LMS to request an LMS to send a location configuration change request to an LMC. In this clause and its sub clauses the sender of the request is referred to as the client.

##### 6.3.2.7.2	Client request location configuration change for another client from LMS using LocationInfoConfigRequest operation

For a client to request location configuration change for another client via the LMS, the client shall send an HTTP POST message to the LMS on the LMS_LocationInformation collection resource (i.e., Location Info Config request) to create the resource associated to the request for configuration change.

The request URI shall be set according to clause 7.2.2.

If the client is an LMC, the HTTP request shall include an Authorization header field with the "Bearer" authentication scheme set to all access tokens of the "bearer" token type as specified in IETF RFC 6750 [19].

NOTE:	The LocationInfoConfigRequest is relevant for all valid MC service IDs, hence the request includes all access tokens.

If the client is a network entity (e.g. an IWF, an MC service server or another LMS) then the client shall insert an X-3GPP-Asserted-Identity header field as specified in 3GPP TS 24.482 [10] clause A.2.1.3. The identity in the 3GPP-Asserted-Identity header field shall be set to the public service identity of the network entity.

Editor's note:	The paragraph above should be added for other service operations relevant for client in network entities.

Other HTTP headers shall be set according to clause 7.2.2.2.

The body of the HTTP POST message shall include the LocationConfiguration as specified in clause 7.1.6.2.3. The LocationConfiguration datatype shall include:

1)	shall include the triggerId attribute, with its already assigned value, if existent and valid, or with a newly generated unique value, if not, where defined for the sub-elements defining the trigger criterion;

2)	shall include the mcServiceId attribute set to the identity of the target user for the location information configuration update; and

3)	may include the locationHistory attribute set to true to indicate that location information shall be stored in the LMC during the state when the LMC is not able to report location information. If the attribute is not included or set to false location information may not be stored in the LMC if the LMC fails to report location information.

Upon receiving the HTTP POST message from the LMC, the LMS shall:

1)	shall validate the access tokens as specified in 3GPP TS 33.180 [17];

2)	consider the MC service IDs derived from the bearer access tokens as the identities of the sender of the LocationInfoConfigRequest;

3)	if any access token is not validated successfully, the LMS shall reject the request with a HTTP 401 (Unauthorized) response and not continue with the remaining steps;

4)	if the access token validation is successful, then:

a)	shall send a HTTP 200 OK response message to the LMC;

5)	identify MC service ID for which location information configuration update is requested, based on content of the attribute mcServiceId;

6)	verify that the initiating user is authorized to update the location information configuration of the MC service ID identified in step 5, according to local policies or to the following:

a)	The initiating user's location user configuration data document according to 3GPP TS 24.484 [12] shall include a <rule> element that include:

i)	a <condition> element containing an <identity> element that matches identified MC service ID; and

ii)	an <action> element containing an <allow-change-location-configuration-of-users> element with the value set to "true"; and

b)	if the item a) above is not fulfilled the MC service ID shall be ignored in the remaining steps;

7)	Location information configuration for the target MC Service Id shall be updated with information LMS received in LocationConfiguration element:

a)	Replace value of mcServiceId attribute in the LocationConfiguration element with the MC Service ID value derived from the bearer access tokens as the identity of the requestor of location information configuration update; and

8)	Upon successful completion of the previous steps, the LMS shall:

a)	Replace value of mcServiceId attribute in the LocationConfiguration element with the MC Service ID value derived from the bearer access tokens as the identity of the requestor of location information configuration update; and

b)	create ConfigurationNotify message using updated configuration information, also taking into account existing LMS policies.

LMS shall send ConfigurationNotify message to the target LMC according to the procedure in clause 6.2.2.5.

# 7	LMS API Definitions

## 7.1	LMS_Reporting API

### 7.1.1	Introduction

The LMS_Reporting shall use the LMS_Reporting Service API.

The API URI of the LMS_Reporting API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the LMC towards the LMS shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 24.483 [11] clause 8.2.44D1.

-	The <apiName> shall be "lms-reporting".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 7.1.3.

### 7.1.2	Usage of HTTP

#### 7.1.2.1	General

HTTP/2, IETF RFC 9113 [21], shall be used as specified in clause 5 of 3GPP TS 29.500 [13].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [13].

The OpenAPI [22] specification of HTTP messages and content bodies for the LMS_Reporting API is contained in clause A.2.

#### 7.1.2.2	HTTP standard headers

##### 7.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [13] for the usage of HTTP standard headers.

##### 7.1.2.2.2	Content type

JSON, IETF RFC 8259 [20], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [13]. The use of the JSON format shall be signalled by the content type "application/json".

#### 7.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [13] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [13] may be supported.

### 7.1.3	Resources

#### 7.1.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.1.3.1-1 depicts the resource URIs structure for the LMS_Reportingn API.

Figure 7.1.3.1-1: Resource URI structure of the LMS_Reporting API

Table 7.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| LMC registrations | /registrations | POST | Create a new LMC registration. |
| Individual LMC registration | /registrations/{registrationId} | PUT | Update an existing LMC registration in the LMS. |
|  |  | DELETE | Delete an existing LMC registration in the LMS. |
| LMC configurations | /configurations | GET | LMC request its location configuration from the LMS. |
| LMC location reports | /locationreports | POST | LMC published its location report in the LMS. |

#### 7.1.3.2	Resource: LMC Registrations

##### 7.1.3.2.1	Description

The resource represents all LMC registrations.

##### 7.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/lms-reporting/<apiVersion>/registrations

This resource shall support the resource URI variables defined in table 7.1.3.2.2-1.

Table 7.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.1.1 |

##### 7.1.3.2.3	Resource Standard Methods

7.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 7.1.3.2.3.1-1.

Table 7.1.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.2.3.1-2 and the response data structures and response codes specified in table 7.1.3.2.3.1-3.

Table 7.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LMCRegistration | M | 1 | LMC registration details information |

Table 7.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LMCRegistration | M | 1 | 201 Created | LMC registration is successfully performed to the LMS. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.1.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 7.1.3.2.4	Resource Custom Operations

None.

#### 7.1.3.3	Resource: Individual LMC registration

##### 7.1.3.3.1	Description

This resource represents an individual registration of an LMC.

##### 7.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/lms-registration//<apiVersion>/registrations/{registrationId}

This resource shall support the resource URI variables defined in table 7.1.3.3.2-1.

Table 7.1.3.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 7.1.1 |
| registrationId | string | The Identifier of a specific LMC registration. |

##### 7.1.3.3.3	Resource Standard Methods

7.1.3.3.3.1	PUT

This method updates the LMC registration data by completely replacing the existing registration data. This method shall support the URI query parameters specified in table 7.1.3.3.3.1-1.

Table 7.1.3.3.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.3.1-2 and the response data structures and response codes specified in table 7.1.3.3.3.1-3.

Table 7.1.3.3.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LMCRegistration | M | 1 | An Individual registration resource to be updated. |

Table 7.1.3.3.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LMCRegistration | M | 1 | 200 OK | An individual LMC registration resource updated successfully and the LMCRegistration data shall be included in the response. |
| n/a |  |  | 204 No Content | An individual LMC registration resource updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.1.3.3.3.1-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.3.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.3.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.3.3.1-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

Table 7.1.3.3.3.1-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

7.1.3.3.3.2	DELETE

This method deregisters (removes) an existing LMC registration. This method shall support the URI query parameters specified in table 7.1.3.3.3.2-1.

Table 7.1.3.3.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.3.3.2-2 and the response data structures and response codes specified in table 7.1.3.3.3.2-3.

Table 7.1.3.3.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.3.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | An individual LMC registration resource deleted successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource termination. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.1.3.3.3.2-4: Headers supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.3.3.2.-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.3.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.3.3.2-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

Table 7.1.3.3.3.2-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

##### 7.1.3.3.4	Resource Custom Operations

None.

#### 7.1.3.4	Resource: LMC Configurations

##### 7.1.3.4.1	Description

The resource represents the LMC location reports collection.

##### 7.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/lms-reporting/<apiVersion>/locationconfiguration

This resource shall support the resource URI variables defined in table 7.1.3.4.2-1.

Table 7.1.3.4.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.1.1 |

##### 7.1.3.4.3	Resource Standard Methods

7.1.3.4.3.1	GET

This method shall support the URI query parameters specified in table 7.1.3.4.3.1-1.

Table 7.1.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.4.3.1-2 and the response data structures and response codes specified in table 7.1.3.4.3.1-3.

Table 7.1.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 7.1.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationConfiguration | M | 1 | 200 Ok | The location configuration applicable for the MC UE. |
| NOTE: The manadatory HTTP error status code for the GET method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.1.3.4.3.1-4: Headers supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 7.1.3.4.4	Resource Custom Operations

None.

#### 7.1.3.5	Resource: LMC Location Reports

##### 7.1.3.5.1	Description

The resource represents the LMC location reports collection.

##### 7.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/lms-reporting/<apiVersion>/locationreports

This resource shall support the resource URI variables defined in table 7.1.3.5.2-1.

Table 7.1.3.5.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.1.1 |

##### 7.1.3.5.3	Resource Standard Methods

7.1.3.5.3.1	POST

This method shall support the URI query parameters specified in table 7.1.3.5.3.1-1.

Table 7.1.3.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.1.3.5.3.1-2 and the response data structures and response codes specified in table 7.1.3.5.3.1-3.

Table 7.1.3.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| MCUELocation | M | 1 | MC UE Location information |

Table 7.1.3.5.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| MCUELocation | M | 1 | 201 Created | MC UE Location report is successfully submitted to the LMS. The LMC location data submitted to the LMS is provided in the response body. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.1.3.5.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.1.3.5.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/lms-location-report/<apiVersion>/locationreports/{locationreportId} |

##### 7.1.3.5.4	Resource Custom Operations

None.

### 7.1.4	Custom Operations without associated resources

None.

### 7.1.5	Notifications

#### 7.1.5.1	General

Table 7.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| ConfigurationNotify | {locationConfigUri} | POST | Notification to an LMC about a new Location configuration. |
| LocationReportRequest | {locationRequestUri} | POST | Request to an LMC about a new location report. |

#### 7.1.5.2	ConfigurationNotify notification

##### 7.1.5.2.1	Description

ConfigurationNotify notification is used by the LMS to notify an LMC on a new location configuration.

##### 7.1.5.2.2	Target URI

The Callback URI "{locationConfigUri}" shall be used with the callback URI variables defined in table 7.1.5.2.2-1.

Table 7.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| locationConfigUri | String formatted as URI with the Callback Uri. |

##### 7.1.5.2.3	Standard Methods

7.1.5.2.3.1	POST

This method shall support the request data structures specified in table 7.1.5.2.3.1-1 and the response data structures and response codes specified in table 7.1.5.2.3.1-2.

Table 7.1.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationConfiguration | M | 1 | Notification of LMC about a new location configuration. |

Table 7.1.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Location configuration is acknowledged. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

#### 7.1.5.3	LocationReportRequest notification

##### 7.1.5.3.1	Description

LocationReportRequest notification is used by the LMS to request an LMC on a new location report.

##### 7.1.5.3.2	Target URI

The Callback URI "{locationRequestUri}" shall be used with the callback URI variables defined in table 7.1.5.3.2-1.

Table 7.1.5.3.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| locationRequestUri | String formatted as URI with the Callback Uri. |

##### 7.1.5.3.3	Standard Methods

7.1.5.3.3.1	POST

This method shall support the request data structures specified in table 7.1.5.3.3.1-1 and the response data structures and response codes specified in table 7.1.5.3.3.1-2.

Table 7.1.5.3.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationRequest | M | 1 | Request an LMC to send a new location report. |

Table 7.1.5.3.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the location request is acknowledged. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

### 7.1.6	Data Model

#### 7.1.6.1	General

This clause specifies the application data model supported by the LMS_Reporting Service API.

Table 7.1.6.1-1 specifies the data types defined for the LMS_Reporting Service interface protocol.

Table 7.1.6.1-1: LMS_Reporting Service specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| LMCRegistration | 7.1.6.2.2 | Includes details of a LMC registration request |  |
| LocationConfiguration | 7.1.6.2.3 | Contains the location configuration details sent as a callback request to the LMC. |  |
| LocationRequest | 7.1.6.2.47 | Contains the location request data type sent as a callback request to the LMC. |  |
| MCUELocation | 7.1.6.2.49 | Includes detailed location information about a MC UE. |  |
| LocationInfo | 7.1.6.2.50 | Includes all location data provided in the location report. |  |

Table 7.1.6.1-2 specifies data types re-used by the LMS_Reporting service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the LMS_Reporting service based interface.

Table 7.1.6.1-2: LMS_Registration Service re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| DateTime | 3GPP TS 29.571 [15] | String with format "date-time" as defined in OpenAPI Specification [22] |  |
| Ecgi | 3GPP TS 29.571 [15] | E-Utran Cell Global Identity. |  |
| GeographicalArea | 3GPP TS 29.572 [16] | Geographical area defined as Polygon or EllipsoidArc |  |
| MbsFsaId | 3GPP TS 29.571 [15] | List of MBS Frequency Selection Area Identifiers, for a broadcast MBS session. |  |
| Ncgi | 3GPP TS 29.571 [15] | NR Cell Global Identity. |  |
| Pei | 3GPP TS 29.571 [15] | Permanent equipment identity. |  |
| PlmnId | 3GPP TS 29.571 [15] | PLMN Identity |  |
| Supi | 3GPP TS 29.571 [15] | Subscription permanent identifier. |  |
| Tac | 3GPP TS 29.571 [15] | Tracking Area |  |
| Uri | 3GPP TS 29.571 [15] | String providing an URI. |  |
| MbsfnArea | 3GPP TS 29.571 [15] | Represents an MBSFN area Identifier |  |
| Altitude | 3GPP TS 29.572 [16] | Double-precision float value of the altitude, expressed in meters. |  |
| HorizontalWithVerticalVelocity | 3GPP TS 29.572 [16] | Velocity estimate including horizontal speed and bearing, and also vertical speed and vertical direction. |  |
| GeographicalCoordinates | 3GPP TS 29.572 [16] | Geographical coordinates. |  |
| Accuracy | 3GPP TS 29.572 [16] | Float value of accuracy, expressed in meters. |  |

#### 7.1.6.2	Structured data types

##### 7.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 7.1.6.2.2	Type: LMCRegistration

Table 7.1.6.2.2-1: Definition of type LMCRegistration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expTime | DateTime | M | 1 | Time and date for the registration expiry. |  |
| mclocClientID | String | M | 1 | Used to identify the LMC that request registration |  |
| locationConfigUri | Uri | M | 1 | URI where the Location Configuration should be delivered to. |  |
| locationRequestUri | Uri | M | 1 | URI where the Location request should be delivered to. |  |
| pei | Pei | O | 0..1 | Includes the permanent equipment identity. |  |
| supi | Supi | O | 0..1 | Includes the imsi number. |  |

##### 7.1.6.2.3	Type: LocationConfiguration

Table 7.1.6.2.3-1: Definition of type LocationConfiguration

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| emergencyTriggeringCriteria | TriggeringCriteria | O | 0..1 | Contains the location reporting trigger criteria used when the MC UE is in emergency state. |  |
| emergencyLocationInformation | RequestedLocationInfo | O | 0..1 | Contains the details of what the location report shall include when the MC UE is in the emergency state. |  |
| triggeringCriteria | TriggeringCriteria | O | 0..1 | Contains the location reporting trigger criteria used when the MC UE is not in emergency state. |  |
| nonEmergencyLocationInformation | RequestedLocationInfo | O | 0..1 | Contains the details of what the location report shall include when the MC UE is not in the emergency state. |  |
| mcServiceId | Uri | O | 0..1 | Optional attribute that is used to indicate the identity of the MC user requesting to set the location configuration or an identity of the MC user for which location configuration needs to be updated. |  |
| configScope | ConfigScope | O | 0..1 | Used to indicate a full new location configuration or a subsequent upate of an existing. |  |
| locationHistory | boolean | O | 0..1 | Optional attribute that if set to true indicates that location information shall be stored in LMC if location reporting can not be done. |  |
| NOTE: At least one of emergencyTriggeringCriteria and triggeringCriteria shall be included. |  |  |  |  |  |

##### 7.1.6.2.4	Type: TriggeringCriteria

Table 7.1.6.2.4-1: Definition of type TriggeringCriteria

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| adaptiveTrigger | AdaptiveTrigger | O | 0..1 | This attribute specifies parameters controlling adaptive combined time and distance triggers for location reporting. |  |
| cellChange | CellChange | O | 0..1 | The attribute specifies which cell changes trigger location reporting. |  |
| geographicalAreaChange | GeographicalAreaChange | O | 0..1 | The attribute specifies which geographical area changes trigger location reporting. |  |
| mbsfnAreaChange | MbsfnAreaChange | O | 0..1 | The attribute specifies which Mbsfn area changes trigger location reporting. |  |
| mbmsSaChange | MbmsSaChange | O | 0..1 | The attribute specifies which Mbms SA changes trigger location reporting. |  |
| nr5GMbsfsaAreaChange | nr5GMbsfsaAreaChange | O | 0..1 | The attribute specifies which 5G Mbsfsa area changes trigger location reporting. |  |
| periodicReport | PeriodicReport | O | 0..1 | The attribute specifies how frequent periodic location reports shall be sent. |  |
| plmnChange | PlmnChange | O | 0..1 | The attribute specifies which PLMN changes trigger location reporting. |  |
| ratTypeChange | RatTypeChange | O | 0..1 | An optional attribute specifying what inter-RAT changes triggers a location report. |  |
| signallingEvent | SignallingEvent | O | 0..1 | An optional attribute specifying what signalling events triggers a location report. |  |
| trackingAreaChange | TrackingAreaChange | O | 0..1 | The attribute specifies which tracking area changes trigger location reporting. |  |
| travelledDistance | TravelledDistance | O | 0..1 | The attribute specifies the required travelled distance for a trigger for location reporting. |  |

##### 7.1.6.2.5	Type: CellChange

Table 7.1.6.2.5-1: Definition of type CellChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyCellChange | AnyCellChange | O | 1 | The presence of this attribute specifies that any cell change is a trigger for location reporting. |  |
| enterSpecificCell | SpecificCellChange | O | 1 | A list of cell Ids which triggers location reporting when entering the cells. |  |
| exitSpecificCell | SpecificCellChange | O | 1 | A list of cell Ids which triggers location reporting when exiting the cells. |  |

##### 7.1.6.2.6	Type: AnyCellChange

Table 7.1.6.2.6-1: Definition of type AnyCellChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.7	Type: SpecificCellChange

Table 7.1.6.2.7-1: Definition of type SpecificCellChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eutranCells | array(EutranCell) | O | 1..N | A list of Eutran or NR cell id that triggers a location reporting. (NOTE) |  |
| nrCells | array(NrCell) | O | 1..N | A list of Nr cell id that triggers a location reporting.(NOTE) |  |
| NOTE: At least on of eutranCells and nrCells shall be present. |  |  |  |  |  |

##### 7.1.6.2.8	Type: EutranCell

Table 7.1.6.2.8-1: Definition of type EutranCell

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eutranCell | Ecgi | M | 1 | A list of Eutran or NR cell id that triggers a location reporting. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.9	Type: NrCell

Table 7.1.6.2.9-1: Definition of type NrCell

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nrCell | Ncgi | M | 1 | A list of Nr cell id that triggers a location reporting. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.10	Type: GeographicalAreaChange

Table 7.1.6.2.10-1: Definition of type GeographicalAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyAreaChange | AnyAreaChange | O | 0..1 | The presence of this attribute specifies that any area change is a trigger for location reporting. |  |
| enterSpecificArea | SpecificArea | O | 0..1 | A geographical area which triggers location reporting when entering or exiting the defined geographical area. |  |
| exitSpecificArea | SpecificArea | O | 0..1 | A geographical area which triggers location reporting when entering or exiting the defined geographical area. |  |

Editor's note:	The purpose of the datatype anyAreaChange is questionable. It is unclear how the LMC shall act on this attribute.

##### 7.1.6.2.11	Type: MbsfnAreaChange

Table 7.1.6.2.11-1: Definition of type MbsfnAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyMbfsnAreaChange | AnyMbfsnAreaChange | O | 1 | The presence of this attribute specifies that any MBSFN area change is a trigger for location reporting. |  |
| enterSpecificMbsfnArea | SpecificMbsfnArea | O | 1 | A list of MBSFN area Ids which triggers location reporting when entering the MBSFN area Ids. |  |
| exitSpecificMbsfnArea | SpecificMbsfnArea | O | 1 | A list of MBSFN area Ids which triggers location reporting when exiting the MBSFN area Ids. |  |

##### 7.1.6.2.12	Type: AnyMbsfnAreaChange

Table 7.1.6.2.12-1: Definition of type AnyMbsfnAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.13	Type: SpecificMbsfnArea

Table 7.1.6.2.13-1: Definition of type SpecificMbsfnArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsfnAreas | array(MbsfnAreaId) | M | 1..N | A list of MBSFN area Ids that triggers location reporting when entering or exiting the MBSFN area Ids. |  |

##### 7.1.6.2.14	Type: MbsfnAreaId

Table 7.1.6.2.14-1: Definition of type MbsfnAreaId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsfnArea | integer | M | 1 | The MBSFN area identity. Integer between 0 and 255. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.15	Type: MbmsSaChange

Table 7.1.6.2.15-1: Definition of type MbmsSaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyMbmsSaChange | AnyMbmsSaChange | O | 1 | The presence of this attribute specifies that any MBMS SA change is a trigger for location reporting. |  |
| enterSpecificMbmsSa | SpecificMbmsSaIdentity | O | 1 | A list of MBMS SAI which triggers location reporting when entering the MBMS SAI. |  |
| exitSpecificMbmsSa | SpecificMbmsSaIdentity | O | 1 | A list of MBMS SAI which triggers location reporting when exiting the MBMS SAI. |  |

##### 7.1.6.2.16	Type: AnyMbmsSaChange

Table 7.1.6.2.16-1: Definition of type AnyMbmsSaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.17	Type: SpecificMbmsSaIdentity

Table 7.1.6.2.17-1: Definition of type SpecificMbmsSaIdentity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbmsSaIdentities | array(MbmsSaIdentity) | M | 1..N | A list of MBMS SAI that triggers location reporting when entering or exiting the MBMS SAI. |  |

##### 7.1.6.2.18	Type: MbmsSaIdentity

Table 7.1.6.2.18-1: Definition of type MbmsSaIdentity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbmsSaIdentity | Integer | M | 1 | The MBMS SA identity. Integer between 0 and 65535. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.19	Type: NR5GMbsfsaAreaChange

Table 7.1.6.2.19-1: Definition of type NR5GMbsfsaAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyNR5GMbsfsaAreaChange | AnyNR5GMbsfsaAreaChange | O | 1 | The presence of this element specifies that any 5G Mbsfsa area change is a trigger for location reporting. |  |
| enterSpecificNR5GMbsfsaArea | SpecificNR5GMbsfsaArea | O | 1 | A list of 5G Mbsfsa area that triggers a location reporting when entering the 5G Mbsfsa area. |  |
| exitSpecificNR5GMbsfsaArea | SpecificNR5GMbsfsaArea | O | 1 | A list of 5G Mbsfsa area that triggers a location reporting when exiting the 5G Mbsfsa area. |  |

##### 7.1.6.2.20	Type: AnyNR5GMbsfsaAreaChange

Table 7.1.6.2.20-1: Definition of type AnyNR5GMbsfsaAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.21	Type: SpecificNR5GMbsfsaArea

Table 7.1.6.2.21-1: Definition of type SpecificNR5GMbsfsaArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nr5GMbsFsaIdentities | array(NR5GMbsfsaAreaIdentity) | M | 1..N | A list of 5G Mbsfsa area that triggers a location reporting when entering or exiting the 5G Mbsfsa area. |  |

##### 7.1.6.2.22	Type: NR5GMbsfsaAreaIdentity

Table 7.1.6.2.22-1: Definition of type NR5GMbsfsaAreaIdentity

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mbsfsa AreaIdentity | MbsFsaId | M | 1 | An 5G Mbsfsa area identity that triggers location reporting. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.23	Type: PeriodicReport

Table 7.1.6.2.23-1: Definition of type PeriodicReport

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| periodicReport | integer | M | 1 | Value in seconds specifies the reporting interval. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.24	Type: PlmnChange

Table 7.1.6.2.24-1: Definition of type PlmnChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyPlmnChange | AnyPlmnChange | O | 1 | The presence of this element specifies that any PLMN change is a trigger for location reporting. |  |
| enterSpecificPlmnArea | SpecificPlmnChange | O | 1 | A list of PLMNs that triggers a location reporting when entering the PLMN. |  |
| exitSpecificPlmnArea | SpecificPlmnChange | O | 1 | A list of PLMNs that triggers a location reporting when exiting the PLMN. |  |

##### 7.1.6.2.25	Type: AnyPlmnChange

Table 7.1.6.2.25-1: Definition of type AnyPlmnChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.26	Type: SpecificPlmnChange

Table 7.1.6.2.26-1: Definition of type SpecificPlmnChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| plmnIds | array(Plmn) | M | 1..N | A list of PLMNs that triggers a location reporting when entering or exiting the PLMN. |  |

##### 7.1.6.2.27	Type: Plmn

Table 7.1.6.2.27-1: Definition of type Plmn

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| plmn | PlmnId | M | 1 | A PLMN identity that triggers location reporting. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.28	Type: TrackingAreaChange

Table 7.1.6.2.28-1: Definition of type TrackingAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyTrackingAreaChange | AnyTrackingAreaChange | O | 1 | The presence of this element specifies that any tracking area change is a trigger for location reporting. |  |
| enterSpecificTrackingArea | SpecificTrackingAreaChange | O | 1 | A list of tracking areas that triggers a location reporting when entering the tracking area. |  |
| exitSpecificTrackingArea | SpecificTrackingAreaChange | O | 1 | A list of tracking areas that triggers a location reporting when exiting the tracking area. |  |

##### 7.1.6.2.29	Type: AnyTrackingAreaChange

Table 7.1.6.2.29-1: Definition of type AnyTrackingAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.30	Type: SpecificTrackingAreaChange

Table 7.1.6.2.30-1: Definition of type SpecificTrackingAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trackingAreas | array(TrackingArea) | M | 1..N | A list of tracking areas that triggers a location reporting when entering or exiting the tracking area. |  |

##### 7.1.6.2.31	Type: TrackingArea

Table 7.1.6.2.31-1: Definition of type TrackingArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| trackingArea | Tac | M | 1 | A tracking area identity that triggers location reporting. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.32	Type: TravelledDistance

Table 7.1.6.2.32-1: Definition of type TravelledDistance

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| travelledDistance | Integer | M | 1 | Value in meter specifies the travelled distance for location reporting. |  |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.33	Type: RequestedLocationInfo

Table 7.1.6.2.33-1: Definition of type RequestedLocationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| bearingAndSpeed | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| Coordinate | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| mbmsSaiId | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| mbsfnArea | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| minimumIntervalLength | integer | O | 1 | Value in seconds, the minimum time between location reports. |  |
| neigbouringEcgi | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| neigbouringNcgi | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| nr5GmbsfsaArea | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| servingEcgi | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |
| servingNcgi | boolean | O | 1 | The presence of this attribute indicates that the item shall be included in the location report. |  |

##### 7.1.6.2.34	Type: AdaptiveTrigger

Table 7.1.6.2.34-1: Definition of type AdaptiveTrigger

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |
| minPeriod | integer | O | 0..1 | An optional attribute specified as a positive or 0 integer number of seconds and defaulting at 0, if not present. This attribute specifies the minimum wait period between consecutive location reports, irrespective of changes in the distance between the current location and the location of the most recent sending of a location report. |  |
| minDistance | integer | O | 0..1 | An optional attribute specified as a strictly positive integer number of meters and defaulting at infinity, if not present. This attribute is used in the decision of sending a location report based on travelled distance, and specifies the minimum required distance, between the current location and the location of the most recent sending of a location report. |  |
| persistencePeriod | integer | O | 0..1 | An optional attribute specified as a positive or 0 integer number of seconds and defaulting at 0, if not present. This attribute specifies the time between the moment when the LMC detected that the distance between the current location and the location of the most recent sending of a location report exceeded the minDistance attribute and the moment when the LMC will check again that the updated current location is still farther away by at least minDistance from the location of the mentioned sending of the location report. If the check is positive, a location report based on travelled distance will be sent. |  |
| additionalTime | integer | O | 0..1 | An optional attribute, specified as a strictly positive integer number of seconds and defaulting at 1. If a location report is not sent based on the travelled distance, the LMC will send a location report after minPeriod + PersistencePeriod + AdditionalTime seconds after the previous location report. |  |
| rrcInactiveMinPeriod | integer | O | 0..1 | An optional attribute with the same semantics and behaviour as described above for the corresponding parameter, but applicable when the UE receives MBS traffic in RRC_INACTIVE state. |  |
| rrcInactiveMinDistance | integer | O | 0..1 | An optional attribute with the same semantics and behaviour as described above for the corresponding parameter, but applicable when the UE receives MBS traffic in RRC_INACTIVE state. |  |
| rrcInactivePersistencePeriod | integer | O | 0..1 | An optional attribute with the same semantics and behaviour as described above for the corresponding parameter, but applicable when the UE receives MBS traffic in RRC_INACTIVE state. |  |
| rrcInactiveadditionalTime | integer | O | 0..1 | An optional attribute with the same semantics and behaviour as described above for the corresponding parameter, but applicable when the UE receives MBS traffic in RRC_INACTIVE state. |  |

##### 7.1.6.2.35	Type: AnyAreaChange

Table 7.1.6.2.35-1: Definition of type AnyAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.36	Type: SpecificArea

Table 7.1.6.2.36-1: Definition of type SpecificArea

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| geoAreas | array(GeographicArea) | M | 1..N | A list of geographical areas that triggers a location reporting when entering or exiting the the geographical areas. |  |

##### 7.1.6.2.37	Type: RatTypeChange

Table 7.1.6.2.37-1: Definition of type RatTypeChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| anyRatTypeChange | AnyRatTypeChange | O | 0..1 | The presence of this optional attribute specifying that any inter-RAT change triggers a location report. |  |

##### 7.1.6.2.38	Type: AnyRatTypeChange

Table 7.1.6.2.38-1: Definition of type AnyRatTypeChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.39	Type: AnyAreaChange

Table 7.1.6.2.39-1: Definition of type AnyAreaChange

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.40	Type: SignallingEvent

Table 7.1.6.2.40-1: Definition of type SignallingEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| functionalAliasActivation | FunctionalAliasActivation | O | 0..1 | This attribute specifies that a functional alias activation triggers a location report. |  |
| functionalAliasDeactivation | FunctionalAliasDeactivation | O | 0..1 | This attribute specifies that a functional alias deactivation triggers a location report. |  |
| initialLogOn | InitialLogOn | O | 0..1 | This attribute specifies that an initial log on triggers a location report. |  |
| groupCallNonEmergency | GroupCallNonEmergency | O | 0..1 | This attribute specifies that a non-emergency group call triggers a location report. |  |
| privateCallNonEmergency | PrivateCallNonEmergency | O | 0..1 | This attribute specifies that a non-emergency private call triggers a location report. |  |
| locationConfigurationReceived | LocationConfigurationReceived | O | 0..1 | This attribute specifies that a received location configuration triggers a location report. |  |

##### 7.1.6.2.41	Type: FunctionalAliasActivation

Table 7.1.6.2.41-1: Definition of type FunctionalAliasActivation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.42	Type: FunctionalAliasDeactivation

Table 7.1.6.2.42-1: Definition of type FunctionalAliasDeactivation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.43	Type: InitialLogOn

Table 7.1.6.2.43-1: Definition of type InitialLogOn

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.44	Type: GroupCallNonEmergency

Table 7.1.6.2.44-1: Definition of type GroupCallNonEmergency

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.45	Type: PrivateCallNonEmergency

Table 7.1.6.2.45-1: Definition of type PrivateCallNonEmergency

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.46	Type: LocationConfigurationReceived

Table 7.1.6.2.46-1: Definition of type LocationConfigurationReceived

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| triggerId | TriggerId | M | 1 | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.2.47	Type: LocationRequest

Table 7.1.6.2.47-1: Definition of type LocationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| requestedMcServiceId | Uri | M | 1 | The MC service ID of the MC user being requested to send a location report. |  |
| requestingMcServiceId | Uri | O | 0..1 | The MC service ID of the MC user from which location information is being requested. |  |
| requestedFunctionalAliasId | Uri | O | 0..1 | The activated functional alias ID of the MC user being requested to send a location report. |  |
| requestingFunctionalAliasId | Uri | O | 0..1 | The activated functional alias ID of the MC user from which location information is being requested. |  |
| requestedLocationInfo | RequestedLocationInfo | O | 0..1 | Contains the details of what the location report shall include. |  |
| requestLocationHistory | RequestLocationHistory | O | 0..1 | Includes details on requested location history information. |  |
| requestHistoryStatus | booelan | O | 0..1 | Indicates that a location history status is requested. |  |

##### 7.1.6.2.48	Type: RequestLocationHistory

Table 7.1.6.2.48-1: Definition of type RequestLocationHistory

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| numberOfReports | integer | O | 0..1 | Indicates the number of requested stored reports. |  |
| startTime | DateTime | O | 0..1 | Indicates that the report shall include location information reports newer than this time. |  |
| endTime | DateTime | O | 0..1 | Indicates that the report shall include location information reports older than this time. |  |
| triggerCriteria | TriggerCriteria | O | 0..1 | Identifies the criteria when the location management client generated location information, while not reporting location information. |  |
| minimumIntervalLength | integer | O | 0..1 | Value in seconds, the minimum time between location reports. |  |

##### 7.1.6.2.49	Type: MCUELocation

Table 7.1.6.2.49-1: Definition of type MCUELocation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| functionalAlias | Uri | O | 0..1 | Includes functional alias change that triggered the location report. |  |
| interRatType | InterRatType | O | 0..1 | Includes the RAT change type. |  |
| locationData | LocationInfo | M | 1 | Includes all location data provided in the location report. |  |
| locTimestamp | DateTime | M | 1 | Includes the date and time the location measurement was made |  |
| mclocClientId | MclocClientId | M | 1 | Used to identify the reporting LMC |  |
| mcServiceUELabel | string | O | 0..1 | Used to identify the MC service UE. |  |
| reportId | string | O | 0..1 | Used to return the request id when the location report is triggered by a location request. |  |
| reportType | ReportType | M | 1 | Used to inform whether the LMC is sending the report in an emergency situation or not. |  |
| triggerId | array(string) | O | 1..N | TriggerId attribute associated with the location trigger that triggered the location report. |  |
| locationHistoryData | LocationHistoryData | O | 0..1 | This attribute contains one or multiple time stamped location information reports. |  |
| locationHistoryStatus | LocationHistroyStatus | O | 0..1 | This attribute contains a location hitstory status information. |  |

##### 7.1.6.2.50	Type: LocationInfo

Table 7.1.6.2.50-1: Definition of type locationInfo

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| currentCoordinate | CurrentCoordinate | O | 0..1 | Object that includes the current coordination data. |  |
| ecgi | Ecgi | O | 0..1 | Attribute containing the ECGI that the LMC is located in. |  |
| mbmsSaiId | string | O | 0..1 | Attribute containing the MBMS Service Area Id the LMC is located in. |  |
| mbsfnArea | MbsfnArea | O | 0..1 | Attribute containing the MBSFN Area the LMC is located in. |  |
| ncgi | Ncgi | O | 0..1 | Attribute containing the NGCI that the LMC is located in. |  |
| neighbouringEcgi | array(Ecgi) | O | 1..N | Attribute containing a list of ECGI of any neighbouring cell the LMC detects. |  |
| neighbouringNcgi | array(Ncgi) | O | 1..N | Attribute containing a list of NCGI of any neighbouring cell the LMC detects. |  |
| nr5GMbsfsaArea | MbsFsaId | O | 0..1 | Attribute containing the 5G MBS Frequency Selection Area Id the LMC is located in. |  |

##### 7.1.6.2.51	Type: CurrentCoordinate

Table 7.1.6.2.51-1: Definition of type CurrentCoordinate

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| altitude | Altitude | O | 0..1 | Attribute containing the altitude. |  |
| bearingAndSpeed | HorizontalWithVerticalVelocity | O | 0..1 | Attribute containing the horizontal and vertical velocity as well as the bearing. |  |
| horizonalaccuracy | Accuracy | O | 0..1 | Attribute containing the accuracy expressed in meter. |  |
| geographicalCoordinates | GeographicalCoordinates | M | 1 | Attribute containing the geographical coordinates e.g. in latitude and longitude. |  |
| verticalaccuracy | Accuracy | O | 0..1 | Attribute containing the accuracy expressed in meter. |  |

##### 7.1.6.2.52	Type: LocationHistoryData

Table 7.1.6.2.52-1: Definition of type LocationHistoryData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locationHistoryEvent | array(LocationHistoryEvent) | M | 1..N | This object contains one or multiple time stamped location information reports. |  |

##### 7.1.6.2.53	Type: LocationHistoryEvent

Table 7.1.6.2.53-1: Definition of type LocationHistoryEvent

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| locationData | LocationInfo | M | 1..N | Includes location data provided in the historical location report. |  |
| locTimestamp | DateTime | M | 1 | Includes the date and time the location measurements were made |  |
| triggerId | array(string) | O | 0..N | TriggerId attribute associated with the location triggers that triggered the location report. |  |

##### 7.1.6.2.54	Type: LocationHistoryStatus

Table 7.1.6.2.54-1: Definition of type LocationHistoryStatus

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| storedReports | integer | M | 1 | Indicates the number of stored location reports that has not been reported. |  |
| startTime | DateTime | M | 1 | Includes the date and time of the oldest stored location report. |  |
| endTime | DateTime | M | 1 | Includes the date and time of the newest stored location report. |  |
| triggerId | array(string) | M | 0..N | TriggerId attribute associated with the location triggers that triggered the location report. |  |

#### 7.1.6.3	Simple data types and enumerations

##### 7.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 7.1.6.3.2	Simple data types

The simple data types defined in table 7.1.6.3.2-1 shall be supported.

Table 7.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| TriggerId | string | The triggerId shall be set to a unique value and is returned in the location report. |  |

##### 7.1.6.3.3	Enumeration: ConfigScope

The enumeration ConfigScope indicates how the location configuration shall be updated. To remove configuration elements a "Full" configuration is needed. It shall comply with the provisions defined in table 7.2.6.3.3-1.

Table 7.2.6.3.3-1: Enumeration ConfigScope

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| FULL | The value "FULL" means that LocationConfiguration attributes contains the full location configuration which replaces any previous location configuration. |  |
| UPDATE | The value "UPDATE" means that the LocationConfiguration is in addition to any previous location configuration. |  |

##### 7.1.6.3.4	Enumeration: ReportType

The enumeration ReportType indicates if the report is done during an emergency situation or not. It shall comply with the provisions defined in table 7.1.6.3.4-1.

Table 7.1.6.3.4-1: Enumeration ReportType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| EMERGENCY | Indicates that the MC UE that sends the Location reports is in emergency state. |  |
| NONEMERGENCY | Indicates that the MC UE that sends the Location reports is not in emergency state. |  |

##### 7.1.6.3.5	Enumeration: InterRatType

The enumeration InterRatType indicates type of inter RAT status change. It shall comply with the provisions defined in table 7.1.6.3.5-1.

Table 7.1.6.3.5-1: Enumeration InterRatType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| 5G_MBS_TO_LTE_MBMS | The inter RAT change is from 5G-MBS to LTE-MBMS |  |
| 5G_MBS_TO_LTE_UNICAST | The inter RAT change is from 5G-MBS to LTE-unicast |  |
| LTE_MBMS_TO_5G_MBS | The inter RAT change is from LTE-MBMS to 5G-MBS |  |
| LTE_MBMS_TO_5G_UNICAST | The inter RAT change is from LTE-MBMS to 5G-unicast |  |

### 7.1.7	Error Handling

#### 7.1.7.1	General

For the LMS_Registration API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [14]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [13] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [13].

In addition, the requirements in the following clauses are applicable for the LMS_Registration API.

#### 7.1.7.2	Protocol Errors

No specific procedures for the LMS_Reporting service are specified.

#### 7.1.7.3	Application Errors

The application errors defined for the LMS_Reporting service are listed in table 7.1.7.3-1.

Table 7.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 7.1.8	Feature negotiation

Table 7.1.8-1 defines the features supported by the LMS_Reporting API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [13].

No feature is defined in this release of the specification.

Table 7.1.8-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |

### 7.1.9	Security

The LMC shall send HTTP requests over TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [10].

Authentication and authorization shall be as specified in 3GPP TS 33.180 [17].

The LMS shall be configured with an authorized MC service server list, containing public service identities of MC service servers.

When handling an HTTP request, the LMS shall determine the identity of the sender of the HTTP request as specified in 3GPP TS 24.482 [10], and shall use the identity of the sender of the HTTP request as an authenticated identity when performing the authorization.

### 7.1.10	HTTP redirection

An HTTP request may be redirected to a different LMS service instance, within the same LMS. When an HTTP server redirects an HTTP request (i.e. service request or notification/callback request) to a different target LMS service instance, the URI of the target LMS service instance towards which the request is redirected shall be given by the Location header field of the 307 Temporary Redirect or 308 Permanent Redirect response. Further details on HTTP redirection are found in 3GPP TS 29.500 [13].

## 7.2	LMS_Information API

### 7.2.1	Introduction

The LMS_Information shall use the LMS_Information Service API.

The API URI of the LMS_Information API shall be:

{apiRoot}/<apiName>/<apiVersion>

The request URIs used in HTTP requests from the LMC towards the LMS shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [14], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 24.483 [11] clause 8.2.44D1.

-	The <apiName> shall be "lms-information".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 7.2.3.

### 7.2.2	Usage of HTTP

#### 7.2.2.1	General

HTTP/2, IETF RFC 9113 [21], shall be used as specified in clause 5 of 3GPP TS 29.500 [13].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [13].

The OpenAPI [22] specification of HTTP messages and content bodies for the LMS_Information API is contained in clause A.4.

#### 7.2.2.2	HTTP standard headers

##### 7.2.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [13] for the usage of HTTP standard headers.

##### 7.2.2.2.2	Content type

JSON, IETF RFC 8259 [20], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [13]. The use of the JSON format shall be signalled by the content type "application/json".

#### 7.2.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [13] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [13] may be supported.

### 7.2.3	Resources

#### 7.2.3.1	Overview

This clause describes the structure for the Resource URIs and the resources and methods used for the service.

Figure 7.2.3.1-1 depicts the resource URIs structure for the LMS_Information API.

Figure 7.2.3.1-1: Resource URI structure of the LMS_Information API

Table 7.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 7.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Client location information request | /requests | POST | Create a new client location information request. |
| Location information subscription | /subscriptions | POST | Create a new client location information subscription. |
| Individual location information subscription | /subscriptions/{subscriptionId} | PUT | Update an existing subscription. |
|  |  | DELETE | Delete an existing subscription. |
| Location information config request | /configrequest | POST | Update location configuration information of a target user |

#### 7.2.3.2	Resource: Client location information request

##### 7.2.3.2.1	Description

The resource represents all client location information request.

##### 7.2.3.2.2	Resource Definition

Resource URI: {apiRoot}/lms-information/<apiVersion>/requests

This resource shall support the resource URI variables defined in table 7.2.3.2.2-1.

Table 7.2.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.2.1 |

##### 7.2.3.2.3	Resource Standard Methods

7.2.3.2.3.1	POST

This method shall support the URI query parameters specified in table 7.2.3.2.3.1-1.

Table 7.2.3.2.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a |  |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.2.3.1-2 and the response data structures and response codes specified in table 7.2.3.2.3.1-3.

Table 7.2.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| ClientOriginatingLocationRequest | M | 1 | Client originating location information details |

Table 7.2.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| ClientOriginatingLocationRequest | M | 1 | 201 Created | Client location information request is successfully created in the LMS. The URI of the created resource shall be returned in the "Location" HTTP header. |
| NOTE: The manadatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.2.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.2.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

##### 7.2.3.3.4	Resource Custom Operations

None.

#### 7.2.3.3	Resource: Location information subscriptions

##### 7.2.3.3.1	Description

The resource represents all location information subscriptions.

##### 7.2.3.3.2	Resource Definition

Resource URI: {apiRoot}/lms-information/<apiVersion>/subscriptions

This resource shall support the resource URI variables defined in table 7.2.2.3.2-1.

Table 7.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.2.1 |

##### 7.2.3.3.3	Resource Standard Methods

7.2.3.3.3.1	POST

This method creates the location information subscription. This method shall support the URI query parameters specified in table 7.2.3.3.3.1-1.

Table 7.2.3.3.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.3.3.1-2 and the response data structures and response codes specified in table 723.3.3.3.1-3.

Table 7.2.3.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscriptionRequest | M | 1 | Location information subscription details |

Table 7.2.3.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationSubscriptionRequest | M | 1 | 201 Created | A location information subscription resource created successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.2.3.3.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.3.3.1-5: Headers supported by the 201 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Locationn | String | M | 1 | Contains the URI of the newly created resource, according to the structure: {apiRoot}/lms-location-information/<apiVersion>/subscriptions/{subscriptionId} |

Table 7.2.3.3.3.1-6: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

Table 7.2.3.3.3.1-7: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

#### 7.2.3.4	Resource: Individual location information subscription

##### 7.2.3.4.1	Description

The resource represents a location information subscription.

##### 7.2.3.4.2	Resource Definition

Resource URI: {apiRoot}/lms-information/<apiVersion>/subscription/{subscriptionID}

This resource shall support the resource URI variables defined in table 7.2.2.3.2-1.

Table 7.2.2.3.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.w.1 |

##### 7.2.3.4.3	Resource Standard Methods

7.2.3.4.3.1	PUT

This method updates a location information subscription. This method shall support the URI query parameters specified in table 7.2.3.4.3.1-1.

Table 7.2.3.4.3.1-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.4.3.1-2 and the response data structures and response codes specified in table 7.2.3.4.3.1-3.

Table 7.2.3.4.3.1-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationSubscriptionRequest | M | 1 | Client location information subscription details to be updated |

Table 7.2.3.4.3.1-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | An individual location information subscription resource updated successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| NOTE: The manadatory HTTP error status code for the PUT method listed in 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.2.3.4.3.1-4: Headers supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.4.3.1-5: Headers supported by the 204 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.4.3.1-6: Links supported by the 204 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.4.3.1-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

Table 7.2.3.4.3.1-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

7.2.3.4.3.2	DELETE

This method deletes a location information subscription. This method shall support the URI query parameters specified in table 7.2.3.4.3.2-1.

Table 7.2.3.4.3.2-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.4.3.2-2 and the response data structures and response codes specified in table 7.2.3.4.3.2-3.

Table 7.2.3.4.3.2-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a | M | 1 | Client location information subscription details to be updated |

Table 7.2.3.4.3.2-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a | M | 1 | 204 No Content | An individual location information subscription resource deleted successfully. |
| n/a |  |  | 307 Temporary Redirect | Temporary redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| n/a |  |  | 308 Permanent Redirect | Permanent redirection, during resource modification. The response shall include a Location header field containing an alternative URI of the resource located in an alternative LMS. |
| NOTE: The manadatory HTTP error status code for the DELETE method listed in 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.2.3.4.3.2-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.4.3.2-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.4.3.2-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

Table 7.2.3.4.3.2-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

Table 7.2.3.4.3.2-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | String | M | 1 | An alternative URI of the resource located in an alternative LMS. |

##### 7.2.3.4.4	Resource Custom Operations

None.

#### 7.2.3.5	Resource: Location information config request

##### 7.2.3.5.1	Description

The resource represents all location information config requests.

##### 7.2.3.5.2	Resource Definition

Resource URI: {apiRoot}/lms-information/<apiVersion>/configrequest

This resource shall support the resource URI variables defined in table 7.2.3.5.2-1.

Table 7.2.3.5.2-1: Resource URI variables for this resource

| Name | Data Type | Definition |
| --- | --- | --- |
| apiRoot | String | See clause 7.2.1 |

##### 7.2.3.5.3	Resource Standard Methodss

7.2.3.5.3.1	POST

This method updates the location configuration information of a target user. This method shall support the URI query parameters specified in table 7.2.3.5.3.1-1.

Table 7.2.3.5.3.1-1: URI query parameters supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

This method shall support the request data structures specified in table 7.2.3.5.3.1-2 and the response data structures and response codes specified in table 7.2.3.5.3.1-3.

Table 7.2.3.5.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationConfiguration | M | 1 | Location configuration details. |

Table 7.2.3.5.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationInfoConfigRequest | M | 1 | 201 Created | A location configuration resource updated successfully. |
| NOTE: The manadatory HTTP error status code for the POST method listed in 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

Table 7.2.3.5.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  |  |  |

### 7.2.4	Custom Operations without associated resources

None.

### 7.2.5	Notifications

#### 7.2.5.1	General

Table 7.2.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| Location Notification | {locationNotificationUri} | POST | Notification to a client containing location information. |

#### 7.2.5.2	Location information notification

##### 7.2.5.2.1	Description

Location configuration notification is used by the LMS to notify a client on a new location information.

##### 7.2.5.2.2	Target URI

The Callback URI "{locationNotificationUri}" shall be used with the callback URI variables defined in table 7.2.5.2.2-1.

Table 7.2.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| locationNotificationUri | String formatted as URI with the Callback Uri. |

##### 7.2.5.2.3	Standard Methods

7.2.5.2.3.1	POST

This method shall support the request data structures specified in table 7.2.5.2.3.1-1 and the response data structures and response codes specified in table 7.2.5.2.3.1-2.

Table 7.2.5.2.3.1-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocationNotification | M | 1 | Notification containing location information. |

Table 7.2.5.2.3.1-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | The receipt of the Location notification is acknowledged. |
| NOTE: The mandatory HTTP error status code for the POST method listed in table 5.2.7.1-1 of 3GPP TS 29.500 [13] also apply. |  |  |  |  |

### 7.2.6	Data Model

#### 7.2.6.1	General

This clause specifies the application data model supported by the LMS_Information Service API.

Table 7.2.6.1-1 specifies the data types defined for the LMS_Information Service interface protocol.

Table 7.2.6.1-1: LMS_Information Service specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| ClientOriginatingLocationRequest | 7.2.6.2.2 | Includes detailed of a client location information request |  |

Table 7.2.6.1-2 specifies data types re-used by the LMS_Information service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the LMS_Information service based interface.

Table 7.2.6.1-2: LMS_Information Service re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| n/a |  |  |  |

#### 7.2.6.2	Structured data types

##### 7.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 7.2.6.2.2	Type: ClientOriginatingLocationRequest

Table 7.2.6.2.2-1: Definition of type ClientOriginatingLocationRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| functionalAliasIds | array(Uri) | O | 1..N | A list of activated Functiona Alias Id of requested MC Users. (NOTE) |  |
| locationNotificationUri | Uri | M | 1 | URI where the Location Notifications should be delivered to. |  |
| mclocClientID | string | M | 1 | Used to identify the LMC that makes a subscription request. |  |
| mcServiceIds | array(Uri) | O | 1..N | A list of MC service IDs of the MC users for which location information is requested MC Users.(NOTE) |  |
| mcServiceGroupIds | array(Uri) | O | 1..N | A list of MC service group IDs of which all affiliated members' location information are being requested. |  |
| refresh | boolean | O | 0..1 | Indicates that the location report shall be refreshed immediately. |  |
| requestLocationHistory | RequestLocationHistory | O | 0..1 | Includes details on requested location history information. |  |
| requestHistoryStatus | booelan | O | 0..1 | Indicates that a location history status is requested. |  |
| NOTE: At least one of functionalAliasIDs, mcServiceIds or mcServiceGroupIds shall be present. |  |  |  |  |  |

##### 7.2.6.2.3	Type: MCUserLocation

Table 7.2.6.2.3-1: Definition of type MCUserLocation

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mcServiceIds | array(Uri) | M | 1..N | A list of MC service IDs of the MC user for which location information is reported. |  |
| functionalAliasIds | array(Uri) | O | 1..N | A list of activated Functional Alias Id of the MC user for which location information is reported. |  |
| mcServiceUELabel | String | O | 0..1 | Used to identify the MC service UE. |  |
| locationInfo | LocationInfo | M | 1 | As defined in clause 7.2.6.2.3 |  |

##### 7.2.6.2.4	Type: LocationNotification

Table 7.2.6.2.4-1: Definition of type LocationNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| mcUEResponseList | array(MCUserLocation) | M | 1..N | Represents one array of MC Users’ device location information. |  |
| requestingMcServiceId | Uri | M | 1 | The MC service ID of the LMC that requested the location report. |  |
| requestingfunctionalAliasId | Uri | O | 0..1 | The activated Functional Alias Id of the LMC that requested the location report. |  |
| subscriptionID | string | O | 0..1 | Optional attribute including the subscriptionID when the location notification is related to an existing subscription. (NOTE) |  |
| NOTE: The subscriptionID is mandatory if the location notification is sent due to a location subscription. |  |  |  |  |  |

##### 7.2.6.2.5	Type: LocationSubscriptionRequest

Table 7.2.6.2.5-1: Definition of type LocationSubscriptionRequest

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| expTime | DateTime | M | 1 | The subscription expiry time and date. |  |
| functionalAliasIds | array(Uri) | O | 1..N | A list of activated Functional Alias Id of subscribed MC Users. (NOTE) |  |
| locationNotificationUri | Uri | M | 1 | URI where the Location Notifications should be delivered to. |  |
| mclocClientID | string | M | 1 | Used to identify the LMC that makes a subscription request. |  |
| mcServiceIds | array(Uri) | O | 1..N | A list of MC service IDs of the MC users for which location information is subscribed. (NOTE) |  |
| mcServiceGroupIds | array(Uri) | O | 1..N | A list of MC service group IDs of which all affiliated members' location information are subscribed. (NOTE) |  |
| NOTE: At least one of functionalAliasIDs, mcServiceIds or mcServiceGroupIds shall be present. |  |  |  |  |  |

#### 7.2.6.3	Simple data types and enumerations

No simple data types or enumerations defined.

### 7.2.7	Error Handling

#### 7.2.7.1	General

For the LMS_Information API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [14]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [13] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [13].

In addition, the requirements in the following clauses are applicable for the LMS_Information API.

#### 7.2.7.2	Protocol Errors

No specific procedures for the LMS_Information service are specified.

#### 7.2.7.3	Application Errors

The application errors defined for the LMS_Information service are listed in table 7.2.7.3-1.

Table 7.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |

### 7.2.8	Feature negotiation

Table 7.2.8-1 defines the features supported by the LMS_Information API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [13].

No feature is defined in this release of the specification.

Table 7.2.8-1: Supported Features

| Feature number | Feature Name | M/O | Description |
| --- | --- | --- | --- |

### 7.2.9	Security

The LMC shall send HTTP requests over TLS connection as specified for the HTTP client in the UE in annex A of 3GPP TS 24.482 [10].

Authentication and authorization shall be as specified in 3GPP TS 33.180 [17].

The LMS shall be configured with an authorized MC service server list, containing public service identities of MC service servers.

When handling an HTTP request, the LMS shall determine the identity of the sender of the HTTP request as specified in 3GPP TS 24.482 [10], and shall use the identity of the sender of the HTTP request as an authenticated identity when performing the authorization.

### 7.2.10	HTTP redirection

An HTTP request may be redirected to a different LMS service instance, within the same LMS. When an HTTP server redirects an HTTP request (i.e. service request or notification/callback request) to a different target LMS service instance, the URI of the target LMS service instance towards which the request is redirected shall be given by the Location header field of the 307 Temporary Redirect or 308 Permanent Redirect response. Further details on HTTP redirection are found in 3GPP TS 29.500 [13].

Annex A (normative):
OpenAPI specification

# A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see clause 5.3.1 of 3GPP TS 29.501 [14] and clause 5B of 3GPP TR 21.900 [1]).

# A.2	LMS_Reporting API

openapi: 3.0.0

info:

title: LMS_Reporting

version: 1.0.0-alpha.8

description: |

API for Registration and location reporting to LMS Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 24.283 V2.0.0; Mission Critical Location Management (MCLoc).

url: http://www.3gpp.org/ftp/Specs/archive/24_series/24.283/

servers:

- url: '{apiRoot}/lms-reporting/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 8.2.44D1 of 3GPP TS 24.483

security:

- BearerAuth: []

paths:

/registrations:

post:

description: Creates a new indiviual LMC registration in the LMS.

security:

- BearerAuth: []

operationId: registerLMC

tags:

- LMS Registration (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

responses:

'201':

description: Successful regitration of LMC.

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

locationConfiguration:

'{request.body#/locationConfigUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationConfiguration'

responses:

'204':

description: No Content (The receipt of the Configuration is acknowledged).

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

locationRequest:

'{request.body#/locationRequestUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationRequest'

responses:

'200':

description: Location report.

content:

application/json:

schema:

$ref: '#/components/schemas/MCUELocation'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/registrations/{registrationId}:

put:

description: >

Updates an existing LMC registration in the LMS

tags:

- Individual LMS registration (Document)

parameters:

- name: registrationId

in: path

description: Identifies an individual LMC registration resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

responses:

'200':

description: >

OK. The individual LMC registration resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/LMCRegistration'

'204':

description: No Content (updated successfully).

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

description: >

Deletes an existing LMC reigstration identified by the registrationId.

operationId: DeleteLMCRegistration

tags:

- Individual LMC registration (Document)

parameters:

- name: registrationId

in: path

description: Identifies an LMC registration resource.

required: true

schema:

type: string

responses:

'204':

description: >

An individual LMC registration resource deleted successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/locationreports:

post:

summary: submit a MC UE location report

security:

- BearerAuth: []

operationId: submitLocationReport

tags:

- MC UE Location Report (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/MCUELocation'

responses:

'201':

description: Successful submission of location report.

content:

application/json:

schema:

$ref: '#/components/schemas/MCUELocation'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/configurations:

get:

summary: request to download the location configuration

security:

- BearerAuth: []

operationId: ConfigurationRequest

tags:

- LMC configurations (Collection)

responses:

'200':

description: Successful submission of location report.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationConfiguration'

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

BearerAuth:

type: http

scheme: bearer

# API specific definitions

schemas:

LMCRegistration:

description: Resprestents an individual LMS registration resource.

type: object

properties:

expTime:

type: string

format: date-time

mclocClientID:

type: string

description: Identify the LMC of the MC UE

locationConfigUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

locationRequestUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

required:

- mclocClientID

- expTime

- locationConfigUri

- locationRequestUri

LocationConfiguration:

description: Represents an location configuraion objects sent from LMS to an LMC

type: object

properties:

emergencyTriggeringCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

emergencyLocationInformation:

$ref: '#/components/schemas/RequestedLocationInfo'

triggeringCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

nonEmergencyLocationInformation:

$ref: '#/components/schemas/RequestedLocationInfo'

mcServiceId:

type: string

format: Uri

description: Optional attribute that is used to indicate the identity of the

MC user requesting to set the location configuration.

configScope:

$ref: '#/components/schemas/ConfigScope'

historyReporting:

type: boolean

description: The attribute indicates that the LMC shall store location

information if location reporting can not be done, for example if the MC UE is out of

coverage. The stored location information can be reported at a later time.

TriggeringCriteria:

description: Contains the location reporting trigger criteria.

type: object

properties:

adaptiveTrigger:

$ref: '#/components/schemas/AdaptiveTrigger'

cellChange:

$ref: '#/components/schemas/CellChange'

geographicalAreaChange:

$ref: '#/components/schemas/GeographicalAreaChange'

mbsfnAreaChange:

$ref: '#/components/schemas/MbsfnAreaChange'

mbmsSaChange:

$ref: '#/components/schemas/MbmsSaChange'

nr5GMbsfsaAreaChange:

$ref: '#/components/schemas/NR5GMbsfsaAreaChange'

periodicReport:

$ref: '#/components/schemas/PeriodicReport'

plmnChange:

$ref: '#/components/schemas/PlmnChange'

ratTypeChange:

$ref: '#/components/schemas/RatTypeChange'

signallingEvent:

$ref: '#/components/schemas/SignallingEvent'

trackingAreaChange:

$ref: '#/components/schemas/TrackingAreaChange'

travelledDistance:

$ref: '#/components/schemas/TravelledDistance'

AdaptiveTrigger:

description: This attribute specifies parameters controlling adaptive combined time

and distance triggers for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

minPeriod:

type: integer

description: An optional attribute specified as a positive or 0 integer number

of seconds and defaulting at 0, if not present. This attribute specifies the minimum

wait period between consecutive location reports, irrespective of changes in the

distance between the current location and the location of the most recent sending of a

location report.

minDistance:

type: integer

description: An optional attribute specified as a strictly positive integer number of

meters and defaulting at infinity, if not present. This attribute is used in the

decision of sending a location report based on travelled distance, and specifies the

minimum required distance, between the current location and the location of the most

recent sending of a location report.

persistencePeriod:

type: integer

description: An optional attribute specified as a positive or 0 integer number of

seconds and defaulting at 0, if not present. This attribute specifies the time between

the moment when the LMC detected that the distance between the current location and

the location of the most recent sending of a location report exceeded the minDistance

attribute and the moment when the LMC will check again that the updated current

location is still farther away by at least minDistance from the location of the

mentioned sending of the location report. If the check is positive, a location report

based on travelled distance will be sent.

additionalTime:

type: integer

description: An optional attribute, specified as a strictly positive integer number of

seconds and defaulting at 1. If a location report is not sent based on the travelled

distance, the LMC will send a location report after minPeriod + PersistencePeriod

+ AdditionalTime seconds after the previous location report.

rrcInactiveMinPeriod:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

rrcInactiveMinDistance:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

rrcInactivePersistencePeriod:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

rrcInactiveadditionalTime:

type: integer

description: An optional attribute with the same semantics and behaviour as described

above for the corresponding parameter, but applicable when the UE receives MBS traffic

in RRC_INACTIVE state.

CellChange:

description: The attribute specifies which cell changes trigger location reporting.

type: object

properties:

anyCellChange:

$ref: '#/components/schemas/AnyCellChange'

enterSpecificCell:

$ref: '#/components/schemas/SpecificCellChange'

exitSpecificCell:

$ref: '#/components/schemas/SpecificCellChange'

AnyCellChange:

description: The presence of this attribute specifies that any cell change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificCellChange:

description: A list of cell Ids which triggers location reporting when

entering or exiting the cells.

type: object

properties:

eutranCells:

$ref: '#/components/schemas/EutranCells'

nrCells:

$ref: '#/components/schemas/NrCells'

anyOf:

- required: [eutranCells]

- required: [nrCells]

EutranCells:

description: A list of Eutran cell Ids which triggers location reporting when

entering or exiting the cells.

type: array

items:

$ref: '#/components/schemas/EutranCell'

NrCells:

description: A list of NR cell Ids which triggers location reporting when

entering or exiting the cells.

type: array

items:

$ref: '#/components/schemas/NrCell'

EutranCell:

description: An Eutran cell id that triggers a location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

eutranCell:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

NrCell:

description: A NR cell id that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

nrCell:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

GeographicalAreaChange:

description: The attribute specifies which geographical area changes that

trigger location reporting.

type: object

properties:

anyAreaChange:

$ref: '#/components/schemas/AnyAreaChange'

enterSpecifcArea:

$ref: '#/components/schemas/SpecificArea'

exitSpecifcArea:

$ref: '#/components/schemas/SpecificArea'

AnyAreaChange:

description: The presence of this attribute specifies that any area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificArea:

description: A geographical area which triggers location reporting when

entering or exiting the defined geographical area.

type: object

properties:

geoAreas:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

MbsfnAreaChange:

description: The attribute specifies which MBSFN area changes trigger

location reporting.

type: object

properties:

anyMbfsnAreaChange:

$ref: '#/components/schemas/AnyMbsfnAreaChange'

enterSpecificMbsfnArea:

$ref: '#/components/schemas/SpecificMbsfnArea'

exitSpecificMbsfnArea:

$ref: '#/components/schemas/SpecificMbsfnArea'

AnyMbsfnAreaChange:

description: The precense of this attribute specifies that any MBSFN area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificMbsfnArea:

description: A list of MBSFN area Ids that triggers location reporting when

entering or exiting the MBSFN area Ids.

type: object

properties:

mbsfnAreas:

type: array

items:

$ref: '#/components/schemas/MbsfnAreaId'

MbsfnAreaId:

description: An MBFSN area id that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

mbsfnArea:

type: integer

minimum: 0

maximum: 255

description: The MBSFN area identity.

MbmsSaChange:

description: The attribute specifies which MBMS SA area changes trigger

location reporting.

type: object

properties:

anyMbmsSaChange:

$ref: '#/components/schemas/AnyMbmsSaChange'

enterSpecificMbmsSa:

$ref: '#/components/schemas/SpecificMbmsSaIdentity'

exitSpecificMbmsSa:

$ref: '#/components/schemas/SpecificMbmsSaIdentity'

AnyMbmsSaChange:

description: The presence of this element specifies that any MBMS SA change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificMbmsSaIdentity:

description: A list of MBMS SAI that triggers location reporting when

entering or exiting the MBMS SAI.

type: object

properties:

mbmsSaIdentities:

type: array

items:

$ref: '#/components/schemas/MbmsSaIdentity'

MbmsSaIdentity:

description: An MBMS SA identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

mbmsSaIdentity:

type: integer

minimum: 0

maximum: 65535

description: The MBMS SA identity.

NR5GMbsfsaAreaChange:

description: The attribute specifies which 5G Mbsfsa area changes trigger

location reporting.

type: object

properties:

anyNR5GMbsfsaAreaChange:

$ref: '#/components/schemas/AnyNR5GMbsfsaAreaChange'

enterSpecificNR5GMbsfsaArea:

$ref: '#/components/schemas/SpecificNR5GMbsfsaArea'

exitSpecificNR5GMbsfsaArea:

$ref: '#/components/schemas/SpecificNR5GMbsfsaArea'

AnyNR5GMbsfsaAreaChange:

description: The presence of this element specifies that any 5G Mbsfsa area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificNR5GMbsfsaArea:

description: A list of 5G Mbsfsa area that triggers a location reporting when

entering or exiting the 5G Mbsfsa area.

type: object

properties:

nr5GMbsFsaIdentities:

type: array

items:

$ref: '#/components/schemas/NR5GMbsfsaAreaIdentity'

NR5GMbsfsaAreaIdentity:

description: An 5G Mbsfsa area identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

mbsfnAreaIdentity:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId'

PeriodicReport:

description: The attribute specifies how frequent periodic location reports

shall be sent.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

periodicReport:

type: integer

description: Value in seconds specifies the reporting interval.

PlmnChange:

description: The attribute specifies which PLMN changes trigger

location reporting.

type: object

properties:

anyPlmnChange:

$ref: '#/components/schemas/AnyPlmnChange'

enterSpecificPlmnArea:

$ref: '#/components/schemas/SpecificPlmnChange'

exitSpecificPlmnArea:

$ref: '#/components/schemas/SpecificPlmnChange'

AnyPlmnChange:

description: The presence of this element specifies that any PLMN change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificPlmnChange:

description: A list of PLMNs that triggers a location reporting when

entering or exiting the PLMN.

type: object

properties:

plmnIds:

type: array

items:

$ref: '#/components/schemas/Plmn'

Plmn:

description: A PLMN identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

plmn:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PlmnId'

RatTypeChange:

description: An optional attribute specifying what inter-RAT changes triggers

a location report.

type: object

properties:

anyRatTypeChange:

$ref: '#/components/schemas/AnyRatTypeChange'

AnyRatTypeChange:

description: This attribute specifies that any inter-RAT change triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SignallingEvent:

description: An optional attribute specifying what signalling events triggers

a location report.

type: object

properties:

functionalAliasActivation:

$ref: '#/components/schemas/FunctionalAliasActivation'

functionalAliasDeactivation:

$ref: '#/components/schemas/FunctionalAliasDeactivation'

initialLogOn:

$ref: '#/components/schemas/InitialLogOn'

groupCallNonEmergency:

$ref: '#/components/schemas/GroupCallNonEmergency'

privateCallNonEmergency:

$ref: '#/components/schemas/PrivateCallNonEmergency'

locationConfigurationReceived:

$ref: '#/components/schemas/LocationConfigurationReceived'

FunctionalAliasActivation:

description: This attribute specifies that a functional alias activation

triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

FunctionalAliasDeactivation:

description: This attribute specifies that a functional Alias deactivation

triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

InitialLogOn:

description: This attribute specifies that an initial log on triggers a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

GroupCallNonEmergency:

description: This attribute specifies that a non-emergency group call triggers

a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

PrivateCallNonEmergency:

description: This attribute specifies that a non-emergency private call triggers

a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

LocationConfigurationReceived:

description: This attribute specifies that a received location configuration triggers

a location report.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

TrackingAreaChange:

description: The attribute specifies which tracking area changes trigger

location reporting.

type: object

properties:

anyTrackingAreaChange:

$ref: '#/components/schemas/AnyTrackingAreaChange'

enterSpecificTrackingArea:

$ref: '#/components/schemas/SpecificTrackingAreaChange'

exitSpecificTrackingArea:

$ref: '#/components/schemas/SpecificTrackingAreaChange'

AnyTrackingAreaChange:

description: The presence of this element specifies that any tracking area change

is a trigger for location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

SpecificTrackingAreaChange:

description: A list of tracking areas that triggers a location reporting when

entering or exiting the tracking area.

type: object

properties:

trackingAreas:

type: array

items:

$ref: '#/components/schemas/TrackingArea'

TrackingArea:

description: A tracking area identity that triggers location reporting.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

trackingArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tac'

## It is FFS if a specific format for Eutran tracking area id is needed

TravelledDistance:

description: The attribute specifies the required travelled distance for a

trigger for location reporting.

shall be sent.

type: object

properties:

triggerId:

$ref: '#/components/schemas/TriggerId'

travelledDistance:

type: integer

description: Value in meter specifies the travelled distance

for location reporting.

RequestedLocationInfo:

description: Contains the details of what the location report shall include.

type: object

properties:

bearingAndSpeed:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

coordinate:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

mbmsSaiId:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

mbsfnArea:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

minimumIntervalLength:

type: integer

description: Value in seconds, the minimum time between location reports.

neigbouringEcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

neigbouringNcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

nr5GmbsfsaArea:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

servingEcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

servingNcgi:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

item shall be included in the location report.

TriggerId:

type: string

description: The triggerId shall be set to a unique value and is

returned in the location report.

LocationRequest:

description: Represents an location request object used by LMS to

request a location report from an LMC.

type: object

properties:

requestedMcServiceId:

type: string

format: uri

description: The MC service ID of the MC user being requested to send a

location report.

requestingMcServiceId:

type: string

format: uri

description: The MC service ID of the MC user from which location information

is being requested.

requestedFunctionalAliasId:

type: string

format: uri

description: The activated functional alias ID of the MC user being requested

to send a location report.

requestingFunctionalAliasId:

type: string

format: uri

description: The activated functional alias ID of the MC user from which location

information is being requested.

requestedLocationInfo:

$ref: '#/components/schemas/RequestedLocationInfo'

requestLocationHistory:

$ref: '#/components/schemas/RequestedLocationHistory'

requestHistoryStatus:

type: boolean

description: This attributes indicates that a location history status report is requested.

ConfigScope:

description: Indicates if the location configuration shall be updated fully and replace

existing configration, or if the location configuration is added to the exsting.

anyOf:

- type: string

enum:

- FULL

- UPDATE

- type: string

description: This string provides forward-compatibility with future extensions to the

enumeration and is not used to encode content defined in the present version

of this API.

RequestedLocationHistory:

description: Includes details on requested location history information

type: object

properties:

numberOfReports:

type: integer

description: Indicates the number of requested stored reports.

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

triggerCriteria:

$ref: '#/components/schemas/TriggeringCriteria'

minimumIntervalLength:

type: integer

description: Value in seconds, the minimum time between location reports.

MCUELocation:

description: Describes the parameters to perform LMS Location reports.

type: object

properties:

functionalAlias:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

interRatType:

$ref: '#/components/schemas/InterRatType'

locationData:

$ref: '#/components/schemas/LocationInfo'

locTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

mclocClientId:

$ref: '#/components/schemas/MclocClientId'

mcServiceUELabel:

type: string

description: Used to identify the reporting MC UE for the end users, this may be used

when multiple MC UEs are used by the same MC service user.

reportId:

type: string

description: Attribute is used to return the value in the RequestId attribute

in the an Location Request.

reportType:

$ref: '#/components/schemas/ReportType'

triggerId:

type: array

items:

type: string

description: Attribute which can occur multiple times that contain the value

of the TriggerId attribute associated with a trigger that has fired.

locationHistoryData:

$ref: '#/components/schemas/LocationHistoryData'

locationHistoryStatus:

$ref: '#/components/schemas/LocationHistoryStatus'

MclocClientId:

type: string

format: uuid

description: Used to identify the reporting LMC, this is required when multiple location

information responses for different requested LMC are sent.

LocationInfo:

description: object that includes coordinates typically obtained from GPS.

type: object

properties:

currentCoordinate:

$ref: '#/components/schemas/CurrentCoordinate'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

mbmsSaiId:

type: integer

minimum: 0

maximum: 65535

description: Attribute containing the MBMS Service Area Id the LMC is located in.

mbsfnArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsfnArea'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

neighbouringEcgi:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

neighbouringNcgi:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

nr5GMbsfsaArea:

$ref: 'TS29571_CommonData.yaml#/components/schemas/MbsFsaId'

CurrentCoordinate:

description: This object contains one specific location point, speed and bearing (e.g.,

received from a GPS reciever).

type: object

properties:

altitude:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Altitude'

bearingAndSpeed:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/HorizontalWithVerticalVelocity'

geographicalCoordinate:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicalCoordinates'

horizontalaccuracy:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Accuracy'

verticalaccuracy:

anyOf:

- $ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/Accuracy'

LocationHistoryData:

description: This object contains one or multiple time stamped location information reports.

type: array

items:

$ref: '#/components/schemas/LocationHistoryEvent'

LocationHistoryEvent:

description: This object respresents a single location information report that has been

stored in the LMC.

type: object

properties:

locationData:

$ref: '#/components/schemas/LocationInfo'

locTimestamp:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

triggerId:

type: array

items:

type: string

description: Attribute which can occur multiple times that contain the value

of the TriggerId attribute associated with a trigger that has fired.

LocationHistoryStatus:

description: This object respresents Location history summary status of the location reports

that has been stored in the LMC and has not yet been reporeted.

type: object

properties:

storedReports:

type: integer

description: set to the number of stored reports.

startTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

endTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

triggerId:

type: array

items:

type: string

description: Attribute which can occur multiple times that contain the value

of the TriggerId attribute associated with a trigger that has fired.

#

# Simple Types:

#

#

# Enumeration data types

#

InterRatType:

description: Attribute containing the inter-RAT change type.

anyOf:

- type: string

enum:

- 5G_MBS_TO_LTE_MBMS

- 5G_MBS_TO_LTE_UNICAST

- LTE_MBMS_TO_5G_MBS

- LTE_MBMS_TO_5G_UNICAST

- type: string

description: This string provides forward-compatibility with future extensions to the

enumeration and is not used to encode content defined in the present version

of this API.

ReportType:

description: Used to inform whether the LMC is sending the report in an emergency

situation or not.

anyOf:

- type: string

enum:

- EMERGENCY

- NONEMERGENCY

- type: string

description: This string provides forward-compatibility with future extensions to the

enumeration and is not used to encode content defined in the present version

of this API.

# A.3	LMS_Information API

openapi: 3.0.0

info:

title: LMS_Information

version: 1.0.0-alpha.7

description: |

API for Requesting and subscribing on Location Information from LMS Service.

© 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 24.283 V2.0.0; Mission Critical Location Management (MCLoc).

url: http://www.3gpp.org/ftp/Specs/archive/24_series/24.283/

servers:

- url: '{apiRoot}/lms-information/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 8.2.44D1 of 3GPP TS 24.483

security:

- BearerAuth: []

paths:

/requests:

post:

description: Creates a new indiviual client location request in the LMS.

security:

- BearerAuth: []

operationId: requestLMC

tags:

- LMS Location Request (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/ClientOriginatingLocationRequest'

responses:

'201':

description: Successful location request submitted from LMC.

content:

application/json:

schema:

$ref: '#/components/schemas/ClientOriginatingLocationRequest'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

locationRequestResponse:

'{request.body#/locationNotificationUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationNotification'

responses:

'204':

description: No Content (The receipt of the Configuration is acknowledged).

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions:

post:

description: Creates a new location subscription in the LMS.

security:

- BearerAuth: []

operationId: LMSsubscriptions

tags:

- Location Subscription Request (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscriptionRequest'

responses:

'201':

description: Successful location subscription request submitted to LMS.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscriptionRequest'

headers:

Location:

description: Contains the URI of the newly created resource.

required: true

schema:

type: string

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

subscriptionNotification:

'{request.body#/subscriptionResponseUri}':

post:

requestBody:

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationNotification'

responses:

'204':

description: No Content (The receipt of the subscription notification

is acknowledged).

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/subscriptions/{subscriptionId}:

put:

description: >

Updates an existing location subscription identified by the subscriptionId.

operationId: UpdateLocationSub

tags:

- Update Location subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual location subscription resource.

required: true

schema:

type: string

requestBody:

description: Parameters to replace the existing subscription.

required: true

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscriptionRequest'

responses:

'200':

description: >

OK. The location subcsription resource was updated successfully.

content:

application/json:

schema:

$ref: '#/components/schemas/LocationSubscriptionRequest'

'204':

description: No Content (updated successfully).

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

description: >

Deletes an location subscription identified by the subscriptionId.

operationId: DeleteLocationSub

tags:

- Delete location Subscription (Document)

parameters:

- name: subscriptionId

in: path

description: Identifies an individual location subscription resource.

required: true

schema:

type: string

responses:

'204':

description: >

A location subscription resource deleted successfully.

'307':

$ref: 'TS29571_CommonData.yaml#/components/responses/307'

'308':

$ref: 'TS29571_CommonData.yaml#/components/responses/308'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/configrequest:

post:

description: Requests server to update location reporting configuraton for a target user

security:

- BearerAuth: []

operationId: configrequestLMC

tags:

- LMS Location Info Config Request (Collection)

requestBody:

required: true

content:

application/json:

schema:

$ref: 'TS24283_Lms_Reporting.yaml#/components/schemas/LocationConfiguration'

responses:

'201':

description: Successful configuration update request submitted to LMS

content:

application/json:

schema:

$ref: 'TS24283_Lms_Reporting.yaml#/components/schemas/LocationConfiguration'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

BearerAuth:

type: http

scheme: bearer

# API specific definitions

schemas:

ClientOriginatingLocationRequest:

description: Represents an location request object used to request

location infromation from the LMS.

type: object

properties:

functionalAliasIds:

type: array

items:

type: string

format: uri

description: A list of activated Functional Alias Id of requested MC Users.

locationNotificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

mclocClientID:

type: string

description: Identifies the LMC of the MC UE.

mcServiceIds:

type: array

items:

type: string

format: uri

description: A list of MC service IDs of the MC users for which

location information is requested MC Users.

mcServiceGroupIds:

type: array

items:

type: string

format: uri

description: A list of MC service Group IDs for which

location information is requested for the affiliated MC Users.

refresh:

type: boolean

nullable: true

description: The presence of this attribute indicates that the

location report shall be refreshed immediately.

requestLocationHistory:

$ref: 'TS24283_Lms_Reporting.yaml#/components/schemas/RequestedLocationHistory'

requestHistoryStatus:

type: boolean

description: This attributes indicates that a location history status report is requested.

required:

- mclocClientID

- locationNotificationUri

anyOf:

- required: [functionalAliasIds]

- required: [mcServiceIds]

- required: [mcServiceGroupIds]

LocationSubscriptionRequest:

description: Represents a location subscription request.

type: object

properties:

expTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

functionalAliasIds:

type: array

items:

type: string

format: uri

description: A list of Functional Alias Id which are actived for the MC Users

for which location is subscribed.

locationNotificationUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

mclocClientID:

type: string

description: Identifies the LMC of the MC UE.

mcServiceIds:

type: array

items:

type: string

format: uri

description: A list of MC service IDs of the MC users for which

location information is subscribed MC Users.

mcServiceGroupIds:

type: array

items:

type: string

format: uri

description: A list of MC service Group IDs for which

location information is subscribed for the affiliated MC Users.

required:

- expTime

- mclocClientID

- locationNotificationUri

anyOf:

- required: [functionalAliasIds]

- required: [mcServiceIds]

- required: [mcServiceGroupIds]

MCUserLocation:

description: Represents one MC User's device location information.

type: object

properties:

mcServiceIds:

type: array

items:

type: string

format: uri

description: A list of MC service IDs of the MC user for which

location information is reported.

functionalAliasIds:

type: array

items:

type: string

format: uri

description: A list of activated Functional Alias Id of the MC user

for which location information is reported.

mcServiceUELabel:

type: string

description: Used to identify the reporting MC UE for the end user, this may be used

when multiple MC UEs are reporting by the same MC service user.

locationInfo:

$ref: 'TS24283_Lms_Reporting.yaml#/components/schemas/LocationInfo'

LocationNotification:

description: Represents an location notification response.

type: object

properties:

mcUEResponseList:

type: array

items:

$ref: '#/components/schemas/MCUserLocation'

requestingMcServiceId:

type: string

format: uri

description: The MC service ID of the LMC that requested the location report.

requestingfunctionalAliasId:

type: string

format: uri

description: The activated Functional Alias Id of the LMC that requested

the location report.

subscriptionID:

type: string

description: Attribute including the subscriptionID when the location notification

is related to an existing subscription.

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2023-08 | CT1#143 | C1-235054 |  |  |  | Based on initial skeleton C1-235054. Incorporates agreed pCRs C1-235056, C1-236139 | 0.1.0 |
| 2024-08 | CT1#150 |  |  |  |  | Incorporates agreed pCR C1-244202 | 0.2.0 |
| 2024-10 | CT1#151 |  |  |  |  | Incorporates agreed pCRs C1-245350, C1-245847. C1-245917 | 0.3.0 |
| 2024-11 | CT1#152 |  |  |  |  | Incorporates agreed pCRs C1-246230, C1-246350, C1-246352, C1-246353, C1-246913, C1-246940, C1-246941, C1-246942 | 0.4.0 |
| 2025-02 | CT1#153 |  |  |  |  | Incorporates agreed pCRs C1-250267, C1-250268, C1-250270, C1-250271, C1-250857 | 0.5.0 |
| 2025-04 | CT1#154 |  |  |  |  | Incorporates agreed pCRs C1-251684, C1-251685, C1-252277 | 0.6.0 |
| 2025-05 | CT1#155 |  |  |  |  | Incorporates agreed pCRs C1-253124, C1-253158, C1-253159, C1-253161, C1-253773, C1-253776, C1-253777 | 0.7.0 |
| 2025-06 | CT#108 | CP-251216 |  |  |  | TS presented for information in CT#108 | 1.0.0 |
| 2025-09 | CT1#156 |  |  |  |  | Incorporates agreed pCRs C1-254710, C1-254711, C1-254712, C1-254713, C1-254714, C1-254718, C1-255367, C1-255369, C1-255392 | 1.1.0 |
| 2025-09 | CT#109 | CP-252118 |  |  |  | Presenting 3GPP TS 24.283 to CT#109 for approval | 2.0.0 |
| 2025-09 | CT#109 | CP-252118 |  |  |  | Approved by TSG CT#109 | 19.0.0 |