| 3GPP TS 24.519 V2.1.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System (5GS); Time-Sensitive Networking (TSN) Application Function (AF) to Device-Side TSN Translator (DS-TT) and Network-Side TSN Translator (NW-TT) protocol aspects; Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	General	8

5	Elementary procedures between TSN AF and DS-TT	8

5.1	General	8

5.2	Procedures	8

5.2.1	Network-requested Ethernet port management procedure	8

5.2.1.1	General	8

5.2.1.2	Network-requested Ethernet port management procedure initiation	9

5.2.1.3	Network-requested Ethernet port management procedure completion	9

5.2.1.4	Abnormal cases on the network side	10

5.2.1.5	Abnormal cases in the DS-TT	10

5.2.2	DS-TT-initiated Ethernet port management procedure	10

5.2.2.1	General	10

5.2.2.2	DS-TT-initiated Ethernet port management procedure initiation	10

5.2.2.3	DS-TT-initiated Ethernet port management procedure accepted by the TSN AF	11

5.2.2.4	DS-TT-initiated Ethernet port management procedure completion	11

5.2.2.5	Abnormal cases on the network side	11

5.2.2.6	Abnormal cases in the DS-TT	11

5.2.3	DS-TT-initiated Ethernet port management capability procedure	12

5.2.3.1	General	12

5.2.3.2	DS-TT-initiated Ethernet port management capability procedure	12

6	Elementary procedures between TSN AF and NW-TT	12

6.1	General	12

6.2	Procedures for Ethernet port management service	12

6.2.1	TSN AF-requested Ethernet port management procedure	12

6.2.1.1	General	12

6.2.1.2	TSN AF-requested Ethernet port management procedure initiation	13

6.2.1.3	TSN AF-requested Ethernet port management procedure completion	13

6.2.1.4	Abnormal cases in the TSN AF	14

6.2.1.5	Abnormal cases in the NW-TT	14

6.2.2	NW-TT-initiated Ethernet port management procedure	14

6.2.2.1	General	14

6.2.2.2	NW-TT-initiated Ethernet port management procedure initiation	14

6.2.2.3	NW-TT-initiated Ethernet port management procedure completion	15

6.2.2.4	Abnormal cases in the TSN AF	15

6.2.2.5	Abnormal cases in the NW-TT	15

7	Handling of unknown, unforeseen, and erroneous Ethernet port management service data	15

7.1	General	15

7.2	Message too short or too long	16

7.2.1	Message too short	16

7.2.2	Message too long	16

7.3	Unknown or unforeseen message type	16

7.4	Non-semantical mandatory information element errors	16

7.5	Unknown and unforeseen IEs in the non-imperative message part	16

7.5.1	IEIs unknown in the message	16

7.5.2	Out of sequence IEs	17

7.5.3	Repeated IEs	17

7.6	Non-imperative message part errors	17

7.6.1	General	17

7.6.2	Syntactically incorrect optional IEs	17

7.6.3	Conditional IE errors	17

7.7	Messages with semantically incorrect contents	17

8	Message functional definition and contents	18

8.1	Manage Ethernet port command	18

8.1.1	Message definition	18

8.2	Manage Ethernet port complete	18

8.2.1	Message definition	18

8.2.2	Ethernet port management capability	19

8.2.3	Ethernet port status	19

8.2.4	Ethernet port update result	19

8.3	Ethernet port management notify	19

8.3.1	Message definition	19

8.4	Ethernet port management notify ack	19

8.4.1	Message definition	19

8.5	Ethernet port management notify complete	20

8.5.1	Message definition	20

8.6	Ethernet port management capability	20

8.6.1	Message definition	20

8.6.2	Ethernet port management capability	21

9	Information elements coding	21

9.1	Ethernet port management service message type	21

9.2	Ethernet port management list	21

9.3	Ethernet port management capability	27

9.4	Ethernet port status	28

9.5	Ethernet port update result	31

9.6	Static filtering entries	34

9.7	Traffic class table	35

10	Timers of Ethernet port management service	39

Annex A (informative):	Change history	40

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the protocols of communication between a DS-TT and a TSN AF or a NW-TT and a TSN AF as specified in 3GPP TS 23.501 [2] for:

a)	Ethernet port management.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[5]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[6]	IEEE 802.1AB-2016: "IEEE Standard for Local and metropolitan area networks -- Station and Media Access Control Connectivity Discovery".

[7]	IEEE 802.1Q-2018: "Standard for Local and metropolitan area networks--Bridges and Bridged Networks".

[8]	IEEE 802.1Qbv-2015: "IEEE Standard for Local and metropolitan area networks -- Bridges and Bridged Networks - Amendment 25: Enhancements for Scheduled Traffic".

[9]	IEEE 802.1Qcc-2018: "Standard for Local and metropolitan area networks - Bridges and Bridged Networks - Amendment: Stream Reservation Protocol (SRP) Enhancements and Performance Improvements".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [2] apply:

5G System

Time Sensitive Communication

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GS	5G System

AF	Application function

DS-TT	Device-Side TSN Translator

EPMS	Ethernet port management service

NW-TT	Network-Side TSN Translator

TSC	Time Sensitive Communication

TSN	Time-Sensitive Networking

# 4	General

For time sensitive communication (TSC), a 5G system (5GS) can be integrated as a bridge in a time-sensitive networking (TSN) network (i.e. a TSN bridge).

The device-side TSN translator (DS-TT) is deployed at the UE-side edge and the network-side TSN translator (NW-TT) is deployed at the network-side edge in order to interface with a TSN network while achieving transparency (see 3GPP TS 23.501 [2]). In addition, the TSN application function (TSN AF) is deployed to exchange TSN bridge information with the central network controller (CNC). The TSN bridge information includes port management information which is related to Ethernet ports located in the DS -TT and NW-TT.

In order to support TSN bridge information exchange between TSN AF and CNC, the DS-TT, NW-TT, and TSN AF support procedures for Ethernet port management. Clause 5 describes details of the elementary procedures between TSN AF and DS-TT. Clause 6 describes details of the elementary procedures between TSN AF and NW-TT.

# 5	Elementary procedures between TSN AF and DS-TT

## 5.1	General

The UE and the network may support transfer of standardized and deployment-specific Ethernet port management information between a time-sensitive networking (TSN) AF and the DS-TT at the UE, to manage the Ethernet port used at the DS-TT for a PDU session of "Ethernet" PDU session type. The Ethernet port management messages are included in a Port management information container IE and transported using the PDU session establishment procedure and PDU session modification procedure as specified in 3GPP TS 23.502 [3].

## 5.2	Procedures

### 5.2.1	Network-requested Ethernet port management procedure

#### 5.2.1.1	General

The purpose of the network-requested Ethernet port management procedure is to enable the TSN AF to:

a)	obtain the list of port management parameters supported by the DS-TT;

b)	obtain the current values of port management parameters at the DS-TT Ethernet port;

c)	set the values of port management parameters at the DS-TT Ethernet port;

d)	subscribe to be notified by the DS-TT if the values of certain port management parameters change at the DS-TT Ethernet port; or

e)	unsubscribe to be notified by the DS-TT for one or more port management parameters.

#### 5.2.1.2	Network-requested Ethernet port management procedure initiation

In order to initiate the network-requested Ethernet port management procedure, the TSN AF shall:

a)	encode the information about the port management parameters values to be read, the port management parameters values to be set, the port management parameters changes to (un)subscribe to and whether the TSN AF requests the list of port management parameters supported by the DS-TT in an Ethernet port management list IE as specified in clause 9.2 and include it in a MANAGE ETHERNET PORT COMMAND message;

c)	send the MANAGE ETHERNET PORT COMMAND message to the UE via the PCF and the SMF as specified in 3GPP TS 23.502 [3]; and

d)	start timer T35xx (see example in figure 5.2.1.2.1).

Figure 5.2.1.2.1: Network-requested Ethernet port management procedure

#### 5.2.1.3	Network-requested Ethernet port management procedure completion

Upon receipt of the MANAGE ETHERNET PORT COMMAND message, for each operation included in the Ethernet port management list IE, the DS-TT shall:

a)	if the operation code is "get capabilities", include the list of Ethernet port management parameters supported by the DS-TT in the Ethernet port management capability IE of the MANAGE ETHERNET PORT COMPLETE message;

b)	if the operation code is "read parameter", attempt to read the value of the parameter at the DS-TT Ethernet port, and:

1)	if the value of the parameter at the DS-TT Ethernet port is read successfully, include the parameter and its current value in the Ethernet port status IE of the MANAGE ETHERNET PORT COMPLETE message; and

2)	if the value of the parameter at the DS-TT Ethernet port was not read successfully, include the parameter and associated Ethernet port management service cause value in the Ethernet port status IE of the MANAGE ETHERNET PORT COMPLETE message;

c)	if the operation code is "set parameter", attempt to set the value of the parameter at the DS-TT Ethernet port to the value specified in the operation, and:

1)	if the value of the parameter at the DS-TT Ethernet port is set successfully, include the parameter and its current value in the Ethernet port update result IE of the MANAGE ETHERNET PORT COMPLETE message; and

2)	if the value of the parameter at the DS-TT Ethernet port was not set successfully, include the parameter and associated Ethernet port management service cause value in the Ethernet port update result IE of the MANAGE ETHERNET PORT COMPLETE message;

d)	if the operation code is "subscribe-notify for parameter", store the request from the TSN AF to be notified of changes in the value of the corresponding parameter;

e)	if the operation code is "unsubscribe for parameter", delete the stored request from the TSN AF to be notified of changes in the value of the corresponding parameter, if any; and

f)	send the MANAGE ETHERNET PORT COMPLETE to the TSN AF via the SMF and the PCF as specified in 3GPP TS 23.502 [3].

#### 5.2.1.4	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	T35xx expired.

The TSN AF shall, on the first expiry of the timer T35xx, retransmit the MANAGE ETHERNET PORT COMMAND message and shall reset and start timer T35xx. This retransmission is repeated four times, i.e. on the fifth expiry of timer T35xx, the TSN AF shall abort the procedure.

#### 5.2.1.5	Abnormal cases in the DS-TT

The following abnormal cases can be identified:

a)	Transmission failure of the MANAGE ETHERNET PORT COMPLETE message indication from lower layers.

The DS-TT shall not diagnose an error and consider the network-initiated Ethernet port management procedure complete.

NOTE:	Considering the network-initiated Ethernet port management procedure complete as a result of this abnormal case does not cause the DS-TT to revert the execution of the operations included in the MANAGE ETHERNET PORT COMMAND message.

### 5.2.2	DS-TT-initiated Ethernet port management procedure

#### 5.2.2.1	General

The purpose of the DS-TT-initiated Ethernet port management procedure is to notify the TSN AF of one or more changes in the value of Ethernet port management parameters for which the TSN AF had requested to be notified of changes via the network-initiated Ethernet port management procedure.

#### 5.2.2.2	DS-TT-initiated Ethernet port management procedure initiation

In order to initiate the DS-TT-initiated Ethernet port management procedure, the DS-TT shall create an ETHERNET PORT MANAGEMENT NOTIFY message and shall:

a)	include the Ethernet port management parameters to be reported to the TSN AF with their current value in the Ethernet port status IE of the ETHERNET PORT MANAGEMENT NOTIFY message;

b)	start timer T35yy; and

c)	send the ETHERNET PORT MANAGEMENT NOTIFY message to the TSN AF via the SMF and the PCF as specified in 3GPP TS 23.502 [3].

Figure 5.2.2.2.1: DS-TT-initiated Ethernet port management procedure

#### 5.2.2.3	DS-TT-initiated Ethernet port management procedure accepted by the TSN AF

Upon receipt of the ETHERNET PORT MANAGEMENT NOTIFY message, the TSN AF shall:

a)	create a MANAGE ETHERNET PORT MANAGEMENT NOTIFY ACK message; and

b)	send the MANAGE ETHERNET PORT MANAGEMENT NOTIFY ACK message to the UE via the PCF and the SMF as specified in 3GPP TS 23.502 [3].

#### 5.2.2.4	DS-TT-initiated Ethernet port management procedure completion

Upon receipt of the ETHERNET PORT MANAGEMENT NOTIFY ACK message, the DS-TT shall:

a)	stop timer T35yy;

b)	create an ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message; and

c)	send the ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message to the TSN AF via the SMF and the PCF as specified in 3GPP TS 23.502 [3].

#### 5.2.2.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Transmission failure of the ETHERNET PORT MANAGEMENT NOTIFY ACK indication from lower layers.

The TSN AF shall not diagnose an error and consider the DS-TT-initiated Ethernet port management procedure complete.

#### 5.2.2.6	Abnormal cases in the DS-TT

The following abnormal cases can be identified:

a)	T35yy expired.

The DS-TT shall, on the first expiry of the timer T35yy, retransmit the ETHERNET PORT MANAGEMENT NOTIFY message and shall reset and start timer T35yy. This retransmission is repeated four times, i.e. on the fifth expiry of timer T35yy, the DS-TT shall abort the procedure.

b)	Transmission failure of the ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message indication from lower layers.

The DS-TT shall not diagnose an error and consider the DS-TT-initiated Ethernet port management procedure complete.

### 5.2.3	DS-TT-initiated Ethernet port management capability procedure

#### 5.2.3.1	General

The purpose of the DS-TT-initiated Ethernet port management capability procedure is to provide the DS-TT supported Ethernet port management capabilities to the TSN AF during PDU session establishment as specified in 3GPP TS 23.502 [3].

#### 5.2.3.2	DS-TT-initiated Ethernet port management capability procedure

In order to initiate the DS-TT-initiated Ethernet port management capability procedure, the DS-TT shall create an ETHERNET PORT MANAGEMENT CAPABILITY message and shall:

a)	include the DS-TT Ethernet port management capabilities in the Ethernet port management capability IE of the ETHERNET PORT MANAGEMENT CAPABILITY message; and

b)	send the ETHERNET PORT MANAGEMENT CAPABILITY message to the TSN AF via the SMF and the PCF as specified in 3GPP TS 23.502 [3].

Figure 5.2.3.2.1: DS-TT-initiated Ethernet port management capability procedure

# 6	Elementary procedures between TSN AF and NW-TT

## 6.1	General

The TSN AF and NW-TT supports transfer of standardized and deployment-specific Ethernet port management information, to manage the Ethernet port used at the NW-TT. The Ethernet port management messages are included in a Port management information container IE and transported using the N4 Session Level Reporting Procedure and the SM policy association modification procedure as specified in 3GPP TS 23.502 [3].

## 6.2	Procedures for Ethernet port management service

### 6.2.1	TSN AF-requested Ethernet port management procedure

#### 6.2.1.1	General

The purpose of the TSN AF-requested Ethernet port management procedure is to enable the TSN AF to:

a)	obtain the list of port management parameters supported by the NW-TT;

b)	obtain the current values of port management parameters at the NW-TT Ethernet port;

c)	set the values of port management parameters at the NW-TT Ethernet port; or

d)	subscribe to be notified by the NW-TT if the values of certain port management parameters change at the NW-TT Ethernet port; or

e)	unsubscribe to be notified by the NW-TT for one or more port management parameters.

#### 6.2.1.2	TSN AF-requested Ethernet port management procedure initiation

In order to initiate the TSN AF-requested Ethernet port management procedure, the TSN AF shall:

a)	encode the information about the port management parameters values to be read, the port management parameters values to be set, the port management parameters changes to (un)subscribe to and whether the TSN AF requests the list of port management parameters supported by the NW-TT in an Ethernet port management list IE as specified in clause 9.2 and include it in a MANAGE ETHERNET PORT COMMAND message;

b)	send the MANAGE ETHERNET PORT COMMAND message to the NW-TT via the PCF and the SMF as specified in 3GPP TS 23.502 [3]; and

c)	start timer T35xx (see example in figure 6.2.1.2.1).

Figure 6.2.1.2.1: TSN AF-requested Ethernet port management procedure

#### 6.2.1.3	TSN AF-requested Ethernet port management procedure completion

Upon receipt of the MANAGE ETHERNET PORT COMMAND message, for each operation included in the Ethernet port management list IE, the NW-TT shall:

a)	if the operation code is "get capabilities", include the list of Ethernet port management parameters supported by the NW-TT in the Ethernet port management capability IE of the MANAGE ETHERNET PORT COMPLETE message;

b)	if the operation code is "read parameter", attempt to read the value of the parameter at the NW-TT Ethernet port, and:

1)	if the value of the parameter at the NW-TT Ethernet port is read successfully, include the parameter and its current value in the Ethernet port status IE of the MANAGE ETHERNET PORT COMPLETE message; and

2)	if the value of the parameter at the NW-TT Ethernet port was not read successfully, include the parameter and associated Ethernet port management service cause value in the Ethernet port status IE of the MANAGE ETHERNET PORT COMPLETE message;

c)	if the operation code is "set parameter", attempt to set the value of the parameter at the NW-TT Ethernet port to the value specified in the operation, and:

1)	if the value of the parameter at the NW-TT Ethernet port is set successfully, include the parameter and its current value in the Ethernet port update result IE of the MANAGE ETHERNET PORT COMPLETE message; and

2)	if the value of the parameter at the NW-TT Ethernet port was not set successfully, include the parameter and associated Ethernet port management service cause value in the Ethernet port update result IE of the MANAGE ETHERNET PORT COMPLETE message;

d)	if the operation code is "subscribe-notify for parameter", store the request from the TSN AF to be notified of changes in the value of the corresponding parameter;

e)	if the operation code is "unsubscribe for parameter", delete the stored request from the TSN AF to be notified of changes in the value of the corresponding parameter, if any; and

f)	send the MANAGE ETHERNET PORT COMPLETE to the TSN AF via the SMF and the PCF as specified in 3GPP TS 23.502 [3].

#### 6.2.1.4	Abnormal cases in the TSN AF

The following abnormal cases can be identified:

a)	T35xx expired.

The TSN AF shall, on the first expiry of the timer T35xx, retransmit the MANAGE ETHERNET PORT COMMAND message and shall reset and start timer T35xx. This retransmission is repeated four times, i.e. on the fifth expiry of timer T35xx, the TSN AF shall abort the procedure.

#### 6.2.1.5	Abnormal cases in the NW-TT

The following abnormal cases can be identified:

a)	Transmission failure of the MANAGE ETHERNET PORT COMPLETE message indication from lower layers.

The NW-TT shall not diagnose an error and consider the TSN AF-initiated Ethernet port management procedure complete.

NOTE:	Considering that the TSN AF-initiated Ethernet port management procedure complete as a result of this abnormal case does not cause the NW-TT to revert the execution of the operations included in the MANAGE ETHERNET PORT COMMAND message.

### 6.2.2	NW-TT-initiated Ethernet port management procedure

#### 6.2.2.1	General

The purpose of the NW-TT-initiated Ethernet port management procedure is to notify the TSN AF of one or more changes in the value of Ethernet port management parameters for which the TSN AF had requested to be notified of changes via the TSN AF-initiated Ethernet port management procedure.

#### 6.2.2.2	NW-TT-initiated Ethernet port management procedure initiation

In order to initiate the NW-TT-initiated Ethernet port management procedure, the NW-TT shall create an ETHERNET PORT MANAGEMENT NOTIFY message and shall:

a)	include the Ethernet port management parameters to be reported to the TSN AF with their current value in the Ethernet port status IE of the ETHERNET PORT MANAGEMENT NOTIFY message;

b)	start timer T35zz; and

c)	send the ETHERNET PORT MANAGEMENT NOTIFY message to the TSN AF via the SMF and the PCF as specified in 3GPP TS 23.502 [3].

Figure 6.2.2.2.1: NW-TT-initiated Ethernet port management procedure

#### 6.2.2.3	NW-TT-initiated Ethernet port management procedure completion

Upon receipt of the ETHERNET PORT MANAGEMENT NOTIFY message, the TSN AF shall:

a)	create a MANAGE ETHERNET PORT MANAGEMENT NOTIFY ACK message; and

b)	send the MANAGE ETHERNET PORT MANAGEMENT NOTIFY ACK message to the NW-TT via the PCF and the SMF as specified in 3GPP TS 23.502 [3].

Upon receipt of the ETHERNET PORT MANAGEMENT NOTIFY ACK message, the NW-TT shall stop timer T35zz.

#### 6.2.2.4	Abnormal cases in the TSN AF

The following abnormal cases can be identified:

a)	Transmission failure of the ETHERNET PORT MANAGEMENT NOTIFY ACK indication from lower layers.

The TSN AF shall not diagnose an error and consider the NW-TT-initiated Ethernet port management procedure complete.

#### 6.2.2.5	Abnormal cases in the NW-TT

The following abnormal cases can be identified:

a)	T35zz expired.

The NW-TT shall, on the first expiry of the timer T35zz, retransmit the ETHERNET PORT MANAGEMENT NOTIFY message and shall reset and start timer T35zz. This retransmission is repeated four times, i.e. on the fifth expiry of timer T35zz, the NW-TT shall abort the procedure.

b)	Transmission failure of the ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message indication from lower layers.

The NW-TT shall not diagnose an error and consider the NW-TT-initiated Ethernet port management procedure complete.

# 7	Handling of unknown, unforeseen, and erroneous Ethernet port management service data

## 7.1	General

The procedures specified in clause 5 and clause 6 apply to those messages which pass the checks described in clause 7.

Clause 7 also specifies procedures for the handling of unknown, unforeseen, and erroneous Ethernet port management service (EPMS) data by the receiving entity. These procedures are called "error handling procedures", but in addition to providing recovery mechanisms for error situations they define a compatibility mechanism for future extensions of the EPMS.

Clauses 7.1 to 7.7 shall be applied in order of precedence.

Detailed error handling procedures in the TNS AF are implementation dependent and may vary from network to network. However, when extensions of EPMS are developed, TNS AFs are assumed to have the error handling which is indicated in this clause as mandatory ("shall") and that is indicated as strongly recommended ("should").

Also, the error handling of the TNS AF is only considered as mandatory or strongly recommended when certain thresholds for errors are not reached during a dedicated connection.

For definition of semantical and syntactical errors see 3GPP TS 24.007 [4], clause 11.4.2.

The procedures specified for TT are applicable for DS-TT and NW-TT.

## 7.2	Message too short or too long

### 7.2.1	Message too short

When a message is received that is too short to contain a complete message type information element, that message shall be ignored, cf. 3GPP TS 24.007 [4].

### 7.2.2	Message too long

The maximum size of an EPMS message is 65535 octets.

## 7.3	Unknown or unforeseen message type

If the TT or the TSN AF receives an EPMS message with message type not defined for the EPMS or not implemented by the receiver, it shall ignore the EPMS message.

NOTE:	A message type not defined for the EPMS in the given direction is regarded by the receiver as a message type not defined for the EPMS, see 3GPP TS 24.007 [4].

If the TT receives a message not compatible with the EPMS state, the TT shall ignore the EPMS message.

If the TSN AF receives a message not compatible with the EPMS state, the TSN AF actions are implementation dependent.

## 7.4	Non-semantical mandatory information element errors

When on receipt of a message,

a)	an "imperative message part" error; or

b)	a "missing mandatory IE" error

is diagnosed or when a message containing:

a)	a syntactically incorrect mandatory IE;

b)	an IE unknown in the message, but encoded as "comprehension required" (see 3GPP TS 24.007 [4]); or

c)	an out of sequence IE encoded as "comprehension required" (see 3GPP TS 24.007 [4]) is received,

the TT shall ignore the EPMS message;

the TSN AF shall proceed as follows:

the TSN AF shall:

1)	try to treat the message (the exact further actions are implementation dependent); or

2)	ignore the message.

## 7.5	Unknown and unforeseen IEs in the non-imperative message part

### 7.5.1	IEIs unknown in the message

The TT shall ignore all IEs unknown in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [4]).

The TSN AF shall take the same approach.

### 7.5.2	Out of sequence IEs

The TT shall ignore all out of sequence IEs in a message which are not encoded as "comprehension required" (see 3GPP TS 24.007 [4]).

The TSN AF should take the same approach.

### 7.5.3	Repeated IEs

If an information element with format T, TV, TLV, or TLV-E is repeated in a message in which repetition of the information element is not specified in clause 8, the TT shall handle only the contents of the information element appearing first and shall ignore all subsequent repetitions of the information element. When repetition of information elements is specified, the TT shall handle only the contents of specified repeated information elements. If the limit on repetition of information elements is exceeded, the TT shall handle the contents of information elements appearing first up to the limit of repetitions and shall ignore all subsequent repetitions of the information element.

The TSN AF should follow the same procedures.

## 7.6	Non-imperative message part errors

### 7.6.1	General

This category includes:

a)	syntactically incorrect optional IEs; and

b)	conditional IE errors.

### 7.6.2	Syntactically incorrect optional IEs

The TT shall treat all optional IEs that are syntactically incorrect in a message as not present in the message.

The TSN AF shall take the same approach.

### 7.6.3	Conditional IE errors

When upon receipt of an EPMS message the TT diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error, or when it receives an EPMS message containing at least one syntactically incorrect conditional IE, the TT shall ignore the message.

When the TSN AF receives a message and diagnoses a "missing conditional IE" error or an "unexpected conditional IE" error or when it receives a message containing at least one syntactically incorrect conditional IE, the TSN AF shall either:

a)	try to treat the message (the exact further actions are implementation dependent); or

b)	ignore the message.

## 7.7	Messages with semantically incorrect contents

When a message with semantically incorrect contents is received, the TT shall perform the foreseen reactions of the procedural part of clause 5 and clause 6. If, however no such reactions are specified, the TT shall ignore the message.

The TSN AF should follow the same procedure.

# 8	Message functional definition and contents

## 8.1	Manage Ethernet port command

### 8.1.1	Message definition

The MANAGE ETHERNET PORT COMMAND message is sent by the TSN AF to the DS-TT or NW-TT to manage the Ethernet port at the DS-TT or NW-TT, see table 8.1.1.1

Message type:	MANAGE ETHERNET PORT COMMAND

Significance:	dual

Direction:		TSN AF to DS-TT, TSN AF to NW-TT

Table 8.1.1.1: MANAGE ETHERNET PORT COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | MANAGE ETHERNET PORT COMMAND message identity | Ethernet port management service message type 9.1 | M | V | 1 |
|  | Ethernet port management list | Ethernet port management list 9.2 | M | LV-E | 3-65534 |
| NOTE: The total length of the MANAGE ETHERNET PORT COMMAND message cannot exceed 65535 octets (see Port management information container contents maximum length as specified in clause 9.11.4.27 of 3GPP TS 24.501 [5]). |  |  |  |  |  |

## 8.2	Manage Ethernet port complete

### 8.2.1	Message definition

The MANAGE ETHERNET PORT COMPLETE message is sent by the DS-TT or NW-TT to the TSN AF to complete the network-initiated Ethernet port management procedure or the TSN AF-initiated Ethernet port management procedure, see table 8.2.1.1

Message type:	MANAGE ETHERNET PORTCOMPLETE

Significance:	dual

Direction:		DS-TT to TSN AF, NW-TT to TSN AF

Table 8.2.1.1: MANAGE ETHERNET PORT COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | MANAGE ETHERNET PORT COMPLETE message identity | Ethernet port management service message type 9.1 | M | V | 1 |
| xx | Ethernet port management capability | Ethernet port management capability 9.3 | O | TLV-E | 5-65534 |
| yy | Ethernet port status | Ethernet port status 9.4 | O | TLV-E | 5-65534 |
| zz | Ethernet port update result | Ethernet port update result 9.5 | O | TLV-E | 7-65534 |
| NOTE: The total length of the MANAGE ETHERNET PORT COMPLETE message cannot exceed 65535 octets (see Port management information container contents maximum length as specified in clause 9.11.4.27 of 3GPP TS 24.501 [5]). |  |  |  |  |  |

### 8.2.2	Ethernet port management capability

This IE shall be included if the TSN AF has included an operation with operation code set to "get capabilities" in the MANAGE ETHERNET PORT COMMAND message.

### 8.2.3	Ethernet port status

This IE shall be included if the TSN AF has included one or more operations with operation code set to "read parameter" in the MANAGE ETHERNET PORT COMMAND message.

### 8.2.4	Ethernet port update result

This IE shall be included if the TSN AF has included one or more operations with operation code set to "set parameter" in the MANAGE ETHERNET PORT COMMAND message.

## 8.3	Ethernet port management notify

### 8.3.1	Message definition

The ETHERNET PORT MANAGEMENT NOTIFY message is sent by the DS-TT or NW-TT to the TSN AF to notify the TSN AF of one or more changes in the value of Ethernet port management parameters, see table 8.3.1.1

Message type:	ETHERNET PORT MANAGEMENT NOTIFY

Significance:	dual

Direction:		DS-TT to TSN AF, NW-TT to TSN AF

Table 8.3.1.1: ETHERNET PORT MANAGEMENT NOTIFY message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ETHERNET PORT MANAGEMENT NOTIFY message identity | Ethernet port management service message type 9.1 | M | V | 1 |
|  | Ethernet port status | Ethernet port status 9.4 | M | LV-E | 4-65533 |
| NOTE: The total length of the ETHERNET PORT MANAGEMENT NOTIFY message cannot exceed 65535 octets (see Port management information container contents maximum length as specified in clause 9.11.4.27 of 3GPP TS 24.501 [5]). |  |  |  |  |  |

## 8.4	Ethernet port management notify ack

### 8.4.1	Message definition

The ETHERNET PORT MANAGEMENT NOTIFY ACK message is sent by the TSN AF to the DS-TT or NW-TT to acknowledge an ETHERNET PORT MANAGEMENT NOTIFY message, see table 8.4.1.1

Message type:	ETHERNET PORT MANAGEMENT NOTIFY ACK

Significance:	dual

Direction:		TSN AF to DS-TT, TSN AF to NW-TT

Table 8.4.1.1: ETHERNET PORT MANAGEMENT NOTIFY ACK message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ETHERNET PORT MANAGEMENT NOTIFY ACK message identity | Ethernet port management service message type 9.1 | M | V | 1 |
| NOTE: The total length of the ETHERNET PORT MANAGEMENT NOTIFY ACK message cannot exceed 65535 octets (see Port management information container contents maximum length as specified in clause 9.11.4.27 of 3GPP TS 24.501 [5]). |  |  |  |  |  |

## 8.5	Ethernet port management notify complete

### 8.5.1	Message definition

The ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message is sent by the DS-TT to the TSN AF to complete the DS-TT-initiated Ethernet port management procedure, see table 8.5.1.1

Message type:	ETHERNET PORT MANAGEMENT NOTIFY COMPLETE

Significance:	dual

Direction:		DS-TT to TSN AF

Table 8.5.1.1: ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message identity | Ethernet port management service message type 9.1 | M | V | 1 |
| NOTE: The total length of the ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message cannot exceed 65535 octets (see Port management information container contents maximum length as specified in clause 9.11.4.27 of 3GPP TS 24.501 [5]). |  |  |  |  |  |

## 8.6	Ethernet port management capability

### 8.6.1	Message definition

The ETHERNET PORT MANAGEMENT CAPABILITY message is sent by the DS-TT to provide the DS-TT supported Ethernet port management capabilities to the TSN AF, see table 8.6.1.1

Message type:	ETHERNET PORT MANAGEMENT CAPABILITY

Significance:	dual

Direction:		DS-TT to network

Table 8.6.1.1: ETHERNET PORT MANAGEMENT CAPABILITY message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | ETHERNET PORT MANAGEMENT CAPABILITY message identity | Ethernet port management service message type 9.1 | M | V | 1 |
|  | Ethernet port management capability | Ethernet port management capability 9.3 | M | LV-E | 4-65533 |
| NOTE: The total length of the ETHERNET PORT MANAGEMENT CAPABILITY message cannot exceed 65535 octets (see Port management information container contents maximum length as specified in clause 9.11.4.27 of 3GPP TS 24.501 [5]). |  |  |  |  |  |

### 8.6.2	Ethernet port management capability

This IE shall be included in the ETHERNET PORT MANAGEMENT CAPABILITY message if the DS-TT needs to provide the supported Ethernet port parameters as part of DS-TT-initiated Ethernet port management capability procedure.

# 9	Information elements coding

## 9.1	Ethernet port management service message type

Table 9.1.1: Ethernet port management service message type

| Bits |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 |  |  | Reserved |
| 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  |  | MANAGE ETHERNET PORT COMMAND message |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 0 |  |  | MANAGE ETHERNET PORT COMPLETE message |
| 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |  |  | ETHERNET PORT MANAGEMENT NOTIFY message |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 0 |  |  | ETHERNET PORT MANAGEMENT NOTIFY ACK message |
| 0 | 0 | 0 | 0 | 0 | 1 | 0 | 1 |  |  | ETHERNET PORT MANAGEMENT NOTIFY COMPLETE message |
| 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 |  |  | ETHERNET PORT MANAGEMENT CAPABILITY message |
| All other values are reserved |  |  |  |  |  |  |  |  |  |  |

## 9.2	Ethernet port management list

The purpose of the Ethernet port management list information element is to transfer from the TSN AF to the DS-TT or NW-TT a list of operations related to Ethernet port management of the DS-TT or NW-TT to be performed at the DS-TT or NW-TT.

The Ethernet port management list information element is coded as shown in figure 9.2.1, figure 9.2.2, figure 9.2.3, figure 9.2.4, figure 9.2.5, and table 9.2.1.

The Ethernet port management list information element has a minimum length of 4 octets and a maximum length of 65535 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port management list IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Ethernet port management list contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Ethernet port management list contents |  |  |  |  |  |  |  | octet 4 octet z |

Figure 9.2.1: Ethernet port management list information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation 1 |  |  |  |  |  |  |  | octet 4 octet a |
| Operation 2 |  |  |  |  |  |  |  | octet a+1* octet b* |
| … |  |  |  |  |  |  |  | octet b+1* … octet c* |
| Operation N |  |  |  |  |  |  |  | octet c+1* octet z* |

Figure 9.2.2: Ethernet port management list contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation code |  |  |  |  |  |  |  | octet d |

Figure 9.2.3: Operation for operation code set to "00000001"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation code |  |  |  |  |  |  |  | octet d |
| Ethernet port parameter name |  |  |  |  |  |  |  | octet d+1 octet d+2 |

Figure 9.2.4: Operation for operation code set to "00000010", "00000100", or "00000101"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operation code |  |  |  |  |  |  |  | octet d |
| Ethernet port parameter name |  |  |  |  |  |  |  | octet d+1 octet d+2 |
| Length of Ethernet port parameter value |  |  |  |  |  |  |  | octet d+3 |
| Ethernet port parameter value |  |  |  |  |  |  |  | octet d+4 octet e |

Figure 9.2.5: Operation for operation code set to "00000011"

Table 9.2.1: Ethernet port management list information element

Value part of the Ethernet port management list information element (octets 4 to z)

The value part of the Ethernet port management list information element consists of one or several operations.

Operation

Operation code (octet d)

Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 Get capabilities 0 0 0 0 0 0 1 0 Read parameter 0 0 0 0 0 0 1 1 Set parameter 0 0 0 0 0 1 0 0 Subscribe-notify for parameter

0 0 0 0 0 1 0 1 Unsubscribe for parameter

All other values are spare.

Ethernet port parameter name (octets d+1 to d+2)

This field contains the name of the Ethernet port parameter to which the operation applies, encoded as follows: - 0000H Reserved; - 0001H txPropagationDelay; - 0002H Traffic class table; - 0003H GateEnabled; - 0004H AdminBaseTime; - 0005H AdminControlListLength; - 0006H AdminControlList; - 0007H AdminCycleTime; - 0008H Tick granularity; - 0020H Static filtering entries; - 0040H lldpV2PortConfigAdminStatusV2; - 0041H lldpV2LocChassisIdSubtype; - 0042H lldpV2LocChassisId; - 0043H lldpV2MessageTxInterval; - 0044H lldpV2MessageTxHoldMultiplier; - 0060H lldpV2LocPortIdSubtypeDS-TT; - 0061H lldpV2LocPortIdDS-TT; - 0080H lldpV2LocPortIdSubtypeNW-TT; - 0081H lldpV2LocPortIdNW-TT; - 00A0H lldpV2RemChassisIdSubtypeDS-TT; - 00A1H lldpV2RemChassisIdDS-TT; - 00A2H lldpV2RemPortIdSubtypeDS-TT; - 00A3H lldpV2RemPortIdDS-TT; - 00A4H lldpTTLDS-TT; - 00C0H lldpV2RemChassisIdSubtypeNW-TT; - 00C1H lldpV2RemChassisIdNW-TT; - 00C2H lldpV2RemPortIdSubtypeNW-TT; - 00C3H lldpV2RemPortIdNW-TT; - 00C4H lldpTTLNW-TT; - 00C5H Stream filter instance table - 00C6H Stream gate instance table - 00C7H to Spare - 7FFFH - 8000H to Reserved for deployment specific parameters - FFFFH

Length of Ethernet port parameter value (octets d+3 to d+4)

This field contains the binary encoding of the length of the Ethernet port parameter value

Ethernet port parameter value (octet d+4 to e)

This field contains the value to be set for the Ethernet port parameter. When the Ethernet port parameter name indicates txPropagationDelay, the Ethernet port parameter value field contains the binary representation of the txPropagationDelay as defined in IEEE 802.1Qcc [9], expressed in unit of nanoseconds and multiplied by 216, with the LSB bit included in bit 1 of the first octet. If the txPropagationDelay is too big to be represented, all bits of the Ethernet port parameter value field shall be coded as "1" except the MSB bit. The length of Ethernet port parameter value indicates a value of 8. When the Ethernet port parameter name indicates Traffic class table, the Ethernet port parameter value field contains the traffic class table as defined in IEEE 802.1Q [7], encoded as the value part of the Traffic class information element as specified in clause 9.7. When the Ethernet port parameter name indicates GateEnabled, the Ethernet port parameter value field contains the value of GateEnabled as defined in IEEE 802.1Qbv [8], with a Boolean value of FALSE encoded as "00000000" and a Boolean value of TRUE encoded as "00000001". The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates AdminBaseTime, the Ethernet port parameter value field contains the value of the administrative base time as specified in IEEE 802.1Qbv [8]. The length of Ethernet port parameter value field indicates a value of 10. When the Ethernet port parameter name indicates AdminControlListLength, the Ethernet port parameter value field contains the value of the AdminControlListLength as specified in IEEE 802.1Qbv [8]. The length of Ethernet port parameter value field indicates a value of 2. When the Ethernet port parameter name indicates AdminControlList, the Ethernet port parameter value field contains the concatenation of AdminControlListLength entries, each encoded as a GateControlEntry as specified in IEEE 802.1Qbv [8]. When the Ethernet port parameter name indicates AdminCycleTime, the Ethernet port parameter value field contains the value of the AdminCycleTime as specified in IEEE 802.1Qbv [8]. The length of Ethernet port parameter value field indicates a value of 8. When the Ethernet port parameter name indicates Tick granularity, the Ethernet port parameter value field contains the value of the Tick granularity as specified in IEEE 802.1Qbv [8]. The length of Ethernet port parameter value field indicates a value of 4. When the Ethernet port parameter name indicates Static filtering entries, the Ethernet port parameter value field contains Static filtering entries as defined in 3GPP TS 23.501 [2] table 5.28.3.1-1, encoded as the value part of the Static filtering entries information element as specified in clause 9.6. When the Ethernet port parameter name indicates lldpV2PortConfigAdminStatusV2, the Ethernet port parameter value field contains values of lldpV2PortConfigAdminStatusV2 as specified in IEEE 802.1AB [6] clause 9.2.5.1 with value of txOnly encoded as 01H, rxOnly encoded as 02H, txAndRx encoded as 03H, and disabled encoded as 04H. The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates lldpV2LocChassisIdSubtype, the Ethernet port parameter value field contains values of lldpV2LocChassisIdSubtype as specified in IEEE 802.1AB [6] clause 8.5.2.2. The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates lldpV2LocChassisId, the Ethernet port parameter value field contains values of lldpV2LocChassisId in the form of an octet string as specified in IEEE 802.1AB [6] clause 8.5.2.3. The length of Ethernet port parameter value field indicates the length of the octet string with a maximum value of 255. When the Ethernet port parameter name indicates lldpV2MessageTxInterval, the Ethernet port parameter value field contains the value of lldpV2MessageTxInterval as specified in IEEE 802.1AB [6] table 11-2. The length of Ethernet port parameter value field indicates a value of 2. When the Ethernet port parameter name indicates lldpV2MessageTxHoldMultiplier, the Ethernet port parameter value field contains the value of lldpV2MessageTxHoldMultiplier as specified in IEEE 802.1AB [6] table 11-2. The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates lldpV2LocPortIdSubtypeDS-TT or ldpV2LocPortIdSubtypeNW-TT, the Ethernet port parameter value field contains values of lldpV2LocPortIdSubtype as specified in IEEE 802.1AB [6] clause 8.5.3.2. The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates lldpV2LocPortIdDS-TT or lldpV2LocPortIdNW-TT, the Ethernet port parameter value field contains values of lldpV2LocPortId in the form of an octet string as specified in IEEE 802.1AB [6] clause 8.5.3.3. The length of Ethernet port parameter value field indicates the length of the octet string with a maximum value of 255. When the Ethernet port parameter name indicates lldpV2RemChassisIdSubtypeDS-TT or lldpV2RemChassisIdSubtypeNW-TT, the Ethernet port parameter value field contains values of lldpV2RemChassisIdSubtype as specified in IEEE 802.1AB [6] clause 8.5.2.2. The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates lldpV2RemChassisIdDS-TT or lldpV2RemChassisIdNW-TT, the Ethernet port parameter value field contains values of lldpV2RemChassisId in the form of an octet string as specified in IEEE 802.1AB [6] clause 8.5.2.3. The length of Ethernet port parameter value field indicates the length of the octet string with a maximum value of 255. When the Ethernet port parameter name indicates lldpV2RemPortIdSubtypeDS-TT or lldpV2RemPortIdSubtypeNW-TT, the Ethernet port parameter value field contains values of lldpV2RemPortIdSubtype as specified in IEEE 802.1AB [6] clause 8.5.3.2. The length of Ethernet port parameter value field indicates a value of 1. When the Ethernet port parameter name indicates lldpV2RemPortIdDS-TT or lldpV2RemPortIdNW-TT, the Ethernet port parameter value field contains values of lldpV2RemPortId in the form of an octet string as specified in IEEE 802.1AB [6] clause 8.5.3.3. The length of Ethernet port parameter value field indicates the length of the octet string with a maximum value of 255. When the Ethernet port parameter name indicates lldpTTLDS-TT or lldpTTLNW-TT, the Ethernet port parameter value field contains the value of TTL as specified in IEEE 802.1AB [6] clause 8.5.4. The length of Ethernet port parameter value field indicates a value of 2. When the Ethernet port parameter name indicates Stream filter instance table, the Ethernet port parameter value field contains a Stream filter instance table as defined in 3GPP TS 23.501 [2] table 5.28.3.1-1, encoded as the value part of the Stream filter instance table information element as specified in clause 9.8. When the Ethernet port parameter name indicates Stream gate instance table, the Ethernet port parameter value field contains a Stream gate instance table as defined in 3GPP TS 23.501 [2] table 5.28.3.1-1, encoded as the value part of the Stream gate instance table information element as specified in clause 9.9. When the hexadecimal encoding of the Ethernet port parameter name is in the "8000H" to "FFFFH" range, the encoding of the Ethernet port parameter value field and the value of the length of Ethernet port parameter value field are deployment-specific.

## 9.3	Ethernet port management capability

The purpose of the Ethernet port management capability information element is to inform the TSN AF of the Ethernet port parameters supported by the DS-TT or NW-TT.

The Ethernet port management capability information element is coded as shown in figure 9.3.1, figure 9.3.2, and table 9.31.

The Ethernet port management capability information element has a minimum length of 5 octets and a maximum length of 65534 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port management capability IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Ethernet port management capability contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Ethernet port management capability contents |  |  |  |  |  |  |  | octet 4 octet z |

Figure 9.3.1: Ethernet port management capability information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Supported Ethernet port parameter name 1 |  |  |  |  |  |  |  | octet 4 octet 5 |
| Supported Ethernet port parameter name 2 |  |  |  |  |  |  |  | octet 6 octet 7 |
| … |  |  |  |  |  |  |  | octet 8 octet z-2 |
| Supported Ethernet port parameter name N |  |  |  |  |  |  |  | octet z-1 octet z |

Figure 9.3.2: Ethernet port management capability contents

Table 9.3.1: Ethernet port management capability information element

Value part of the Ethernet port management capability information element (octets 4 to z)

The value part of the Ethernet port management capability information element consists of one or several supported Ethernet port parameter names, each encoded over 2 octets as specified in table 9.2.1 for the DS-TT or NW-TT to TSN AF direction.

## 9.4	Ethernet port status

The purpose of the Ethernet port status information element is to report the values of Ethernet port parameters of the DS-TT or NW-TT to the TSN AF.

The Ethernet port status information element is coded as shown in figure 9.4.1, figure 9.4.2, figure 9.4.3, figure 9.4.4, figure 9.4.5, and table 9.4.1.

The Ethernet port status information element has a minimum length of 5 octets and a maximum length of 65534 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port status IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Ethernet port status and error contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Ethernet port status contents |  |  |  |  |  |  |  | octet 4 octet a |
| Ethernet port error contents |  |  |  |  |  |  |  | octet a+1 octet z |

Figure 9.4.1: Ethernet port status information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Ethernet port parameters successfully read |  |  |  |  |  |  |  | octet 4 |
| Ethernet port parameter status 1 |  |  |  |  |  |  |  | octet 5* octet b* |
| Ethernet port parameter status 2 |  |  |  |  |  |  |  | octet b+1* octet c* |
| … |  |  |  |  |  |  |  | octet c+1* … octet d* |
| Ethernet port parameter status N |  |  |  |  |  |  |  | octet d+1* octet a* |

Figure 9.4.2: Ethernet port status contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port parameter name |  |  |  |  |  |  |  | octet e octet e+1 |
| Length of Ethernet port parameter value |  |  |  |  |  |  |  | octet e+2 octet e+3 |
| Ethernet port parameter value |  |  |  |  |  |  |  | octet e+4 octet f |

Figure 9.4.3: Ethernet port parameter status

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Ethernet port parameters not successfully read |  |  |  |  |  |  |  | octet a+1 |
| Ethernet port parameter error 1 |  |  |  |  |  |  |  | octet a+2* octet a+3* |
| Ethernet port parameter error 2 |  |  |  |  |  |  |  | octet a+4* octet a+5* |
| … |  |  |  |  |  |  |  | octet a+6* … octet z-2* |
| Ethernet port parameter error N |  |  |  |  |  |  |  | octet z-1* octet z* |

Figure 9.4.4: Ethernet port error contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port parameter name |  |  |  |  |  |  |  | octet i octet i+1 |
| Ethernet port management service cause |  |  |  |  |  |  |  | octet i+2 |

Figure 9.4.5: Ethernet port parameter error

Table 9.4.1: Ethernet port status information element

Value part of the Ethernet port status information element (octets 4 to z)

Ethernet port status contents (octets 4 to a) This field consists of zero or several Ethernet port parameter statuses. Ethernet port parameter status Ethernet port parameter name (octets e to e+1)

This field contains the name of the Ethernet port parameter which could be read successfully, encoded over 2 octets as specified in table 9.2.1 for the DS-TT or NW-TT to TSN AF direction.

Length of Ethernet port parameter value (octets e+2 to e+3)

This field contains the binary encoding of the length of the Ethernet port parameter value

Ethernet port parameter value (octets e+4 to f)

This field contains the value for the Ethernet port parameter, encoded as specified in table 9.2.1.

Ethernet port error contents (octets a+1 to z) This field consists of zero or several Ethernet port parameter errors. Ethernet port parameter error Ethernet port parameter name (octets to i+1)

This field contains the name of the Ethernet port parameter whose value could not be read successfully, encoded over 2 octets as specified in table 9.2.1 for the DS-TT or NW-TT to TSN AF direction.

Ethernet port management service cause (octet i+2) This field contains the Ethernet port management service cause indicating the reason why the value of the Ethernet port parameter could not be read successfully, encoded as follows: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 Ethernet port parameter not supported 0 0 0 0 0 0 1 0 Invalid Ethernet port parameter value 0 1 1 0 1 1 1 1 Protocol error, unspecified The receiving entity shall treat any other value as 0110 1111, "protocol error, unspecified".

## 9.5	Ethernet port update result

The purpose of the Ethernet port update result information element is to report to the TSN AF the outcome of the request from the TSN AF to set one or more Ethernet port parameters to a specific value.

The Ethernet port update result information element is coded as shown in figure 9.5.1, figure 9.5.2, figure 9.5.3, figure 9.5.4, figure 9.5.5, and table 9.5.1.

The Ethernet port update result information element has a minimum length of 5 octets and a maximum length of 65534 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port update result IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Ethernet port update and update error contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Ethernet port update contents |  |  |  |  |  |  |  | octet 4 octet a |
| Ethernet port update error contents |  |  |  |  |  |  |  | octet a+1 octet z |

Figure 9.5.1: Ethernet port update result information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Ethernet port parameters successfully updated |  |  |  |  |  |  |  | octet 4 |
| Ethernet port parameter update 1 |  |  |  |  |  |  |  | octet 5* octet b* |
| Ethernet port parameter update 2 |  |  |  |  |  |  |  | octet b+1* octet c* |
| … |  |  |  |  |  |  |  | octet c+1* … octet d* |
| Ethernet port parameter update N |  |  |  |  |  |  |  | octet d+1* octet a* |

Figure 9.5.2: Ethernet port update contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port parameter name |  |  |  |  |  |  |  | octet e octet e+1 |
| Length of Ethernet port parameter value |  |  |  |  |  |  |  | octet e+2 |
| Ethernet port parameter value |  |  |  |  |  |  |  | octet e+3 octet f |

Figure 9.5.3: Ethernet port parameter update

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Number of Ethernet port parameters not updated successfully |  |  |  |  |  |  |  | octet a+1 |
| Ethernet port parameter error 1 |  |  |  |  |  |  |  | octet a+2* octet a+3* |
| Ethernet port parameter error 2 |  |  |  |  |  |  |  | octet a+4* octet a+5* |
| … |  |  |  |  |  |  |  | octet a+6* … octet z-2* |
| Ethernet port parameter error N |  |  |  |  |  |  |  | octet z-1* octet z* |

Figure 9.5.4: Ethernet port update error contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ethernet port parameter name |  |  |  |  |  |  |  | octet i octet i+1 |
| Ethernet port management service cause |  |  |  |  |  |  |  | octet i+2 |

Figure 9.5.5: Ethernet port parameter error

Table 9.5.1: Ethernet port update result information element

Value part of the Ethernet port update result information element (octets 4 to z)

Ethernet port update contents (octets 4 to a) This field consists of zero or several Ethernet port parameter updates. Ethernet port parameter update Ethernet port parameter name (octets e to e+1)

This field contains the name of the Ethernet port parameter which could be set successfully, encoded over 2 octets as specified in table 9.2.1 for the DS-TT or NW-TT to TSN AF direction.

Length of Ethernet port parameter value (octet e+2)

This field contains the binary encoding of the length of the Ethernet port parameter value

Ethernet port parameter value (octets e+3 to f)

Ethernet port error contents (octets a+1 to z) This field consists of zero or several Ethernet port parameter errors. Ethernet port parameter error Ethernet port parameter name (octets to i+1)

This field contains the name of the Ethernet port parameter whose value could not be set successfully, encoded over 2 octets as specified in table 9.2.1 for the DS-TT or NW-TT to TSN AF direction.

Ethernet port management service cause (octet i+2) This field contains the Ethernet port management service cause indicating the reason why the value of the Ethernet port parameter could not be set successfully, encoded as follows: Bits 8 7 6 5 4 3 2 1 0 0 0 0 0 0 0 0 Reserved 0 0 0 0 0 0 0 1 Ethernet port parameter not supported 0 0 0 0 0 0 1 0 Invalid Ethernet port parameter value 0 1 1 0 1 1 1 1 Protocol error, unspecified The receiving entity shall treat any other value as 0110 1111, "protocol error, unspecified".

## 9.6	Static filtering entries

The purpose of the Static filtering entries information element is to convey Static filtering entries as defined in 3GPP TS 23.501 [2] table 5.28.3.1-1.

The Static filtering entries information element is coded as shown in figure 9.6.1, figure 9.6.2 and table 9.6.1.

The Static filtering entries information element has a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Static filtering entries IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Static filtering entries contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Static filtering entry 1 |  |  |  |  |  |  |  | octet 4 octet 13 |
| … |  |  |  |  |  |  |  |  |
| Static filtering entry n |  |  |  |  |  |  |  | octet 10n-6 octet 10n+3 |

Figure 9.6.1: Static filtering entries information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MacAddress value |  |  |  |  |  |  |  | octet 4 octet 9 |
| VID value |  |  |  |  |  |  |  | octet 10 octet 11 |
| Port value |  |  |  |  |  |  |  | octet 12 octet 13 |

Figure 9.6.2: Static filtering entry

Table 9.6.1: Static filtering entries

Value part of the Static filtering entries information element (octets 4 to 10n+3)

Static filtering entries contents (octets 4 to 10n+3) This field consists of zero or more Static filtering entries.

Static filtering entry (octets 4 to 13)

MacAddress value (octets 4 to 9) MacAddress value contains the value of MAC address as specified in IEEE 802.1Q [7] clause 8.8.1.

VID value (octets 10 to 11) VID value contains the value of VID specification as specified in IEEE 802.1Q [7] clause 8.8.1.

Port value (octets 12 to 13) Port value contains the value of outbound Port as specified in IEEE 802.1Q [7] clause 8.8.1.

## 9.7	Traffic class table

The purpose of the Traffic class table information element is to convey a traffic class table as defined in IEEE 802.1Q [7].

The Traffic class table information element is coded as shown in figure 9.7.1, figure 9.7.2, figure 9.7.3, and table 9.7.1.

The Traffic class table information element has a minimum length of 3 octets and a maximum length of 19 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic class table IEI |  |  |  |  |  |  |  | octet 1 |
| Length of traffic class table contents |  |  |  |  |  |  |  | octet 2 |
| Traffic class table contents |  |  |  |  |  |  |  | octet 3 octet 2n+3 |

Figure 9.7.1: Traffic class table information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Number of traffic classes |  |  |  | octet 3 |
| Information for traffic class 1 |  |  |  |  |  |  |  | octet 4* octet 5* |
| … |  |  |  |  |  |  |  | octet 6* octet n+2* |
| Information for traffic class N |  |  |  |  |  |  |  | octet 2n+2* octet 2n+3* |

Figure 9.7.2: Traffic class table contents

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | Traffic class value |  |  |  | octet m |
| PriorityValue7 | PriorityValue6 | PriorityValue5 | PriorityValue4 | PriorityValue3 | PriorityValue2 | PriorityValue1 | PriorityValue0 | octet m+1 |

Figure 9.7.3: Information for traffic class

Table 9.7.1: Traffic class information

| Number of traffic classes (bit 1 to bit 3 of octet 3) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 0 | No traffic class information is included |
| 0 | 0 | 0 | 1 | Information on one traffic class is included |
| 0 | 0 | 1 | 0 | Information on two traffic classes is included |
| 0 | 0 | 1 | 1 | Information on three traffic classes is included |
| 0 | 1 | 0 | 0 | Information on four traffic classes is included |
| 0 | 1 | 0 | 1 | Information on five traffic classes is included |
| 0 | 1 | 1 | 0 | Information on six traffic classes is included |
| 0 | 1 | 1 | 1 | Information on seven traffic classes is included |
| 1 | 0 | 0 | 0 | Information on eight traffic classes is included |
| 1 | 0 | 0 | 1 |  |
| to |  |  |  | Reserved |
| 1 | 1 | 1 | 1 |  |
| Traffic class value (bit 1 to bit 3 of octet m) |  |  |  |  |  |
| Bits |  |  |  |  |  |
| 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | The value of the traffic class is 0 |  |
| 0 | 0 | 1 | The value of the traffic class is 1 |  |
| 0 | 1 | 0 | The value of the traffic class is 2 |  |
| 0 | 1 | 1 | The value of the traffic class is 3 |  |
| 1 | 0 | 0 | The value of the traffic class is 4 |  |
| 1 | 0 | 1 | The value of the traffic class is 5 |  |
| 1 | 1 | 0 | The value of the traffic class is 6 |  |
| 1 | 1 | 1 | The value of the traffic class is 7 |  |
| PriorityValue0 (bit 1 of octet m+1) Bit |  |  |  |  |  |
| 1 |  |  |  |  |  |
| 0 | Priority value 0 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 0 is assigned to the traffic class |  |  |  |  |
| PriorityValue1 (bit 2 of octet m+1) Bit |  |  |  |  |  |
| 2 |  |  |  |  |  |
| 0 | Priority value 1 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 1 is assigned to the traffic class |  |  |  |  |
| PriorityValue2 (bit 3 of octet m+1) Bit |  |  |  |  |  |
| 3 |  |  |  |  |  |
| 0 | Priority value 2 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 2 is assigned to the traffic class |  |  |  |  |
| PriorityValue3 (bit 4 of octet m+1) Bit |  |  |  |  |  |
| 4 |  |  |  |  |  |
| 0 | Priority value 3 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 3 is assigned to the traffic class |  |  |  |  |
| PriorityValue4 (bit 5 of octet m+1) Bit |  |  |  |  |  |
| 5 |  |  |  |  |  |
| 0 | Priority value 4 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 4 is assigned to the traffic class |  |  |  |  |
| PriorityValue5 (bit 6 of octet m+1) Bit |  |  |  |  |  |
| 6 |  |  |  |  |  |
| 0 | Priority value 5 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 5 is assigned to the traffic class |  |  |  |  |
| PriorityValue6 (bit 7 of octet m+1) Bit |  |  |  |  |  |
| 7 |  |  |  |  |  |
| 0 | Priority value 6 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 6 is assigned to the traffic class |  |  |  |  |
| PriorityValue7 (bit 8 of octet m+1) Bit |  |  |  |  |  |
| 8 |  |  |  |  |  |
| 0 | Priority value 7 is not assigned to the traffic class |  |  |  |  |
| 1 | Priority value 7 is assigned to the traffic class |  |  |  |  |

## 9.8	Stream filter instance table

The purpose of the Stream filter instance table information element is to convey a Stream filter instance table as defined 3GPP TS 23.501 [2] table 5.28.3.1-1.

The Stream filter instance table information element is coded as shown in figure 9.8.1, figure 9.8.2, and table 9.8.1.

The Stream filter instance table is a type 6 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream filter instance table IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Stream filter instance table contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Stream filter instance 1 |  |  |  |  |  |  |  | octet 4* octet 15* |
| … |  |  |  |  |  |  |  |  |
| Stream filter instance n |  |  |  |  |  |  |  | octet 12n-8* octet 12n+3* |

Figure 9.8.1: Stream filter instance table information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Stream filter instance contents |  |  |  |  |  |  |  | octet 4 |
| StreamHandleSpec value |  |  |  |  |  |  |  | octet 5 octet 8 |
| PrioritySpec value |  |  |  |  |  |  |  | octet 9 octet 12 |
| StreamGateInstanceID value |  |  |  |  |  |  |  | octet 13 octet 16 |

Figure 9.8.2: Stream filter instance

Table 9.8.1: Stream filter instance table

Value part of the Stream filter instance table information element (octets 4 to 12n+3)

Stream filter instance table contents (octets 4 to 12n+3) This field consists of zero or more Stream filter instances.

Stream filter instance (octets 4 to 16)

Length of Stream filter instance contents (octet 4) Length of Stream filter instance contents contains the length of the vale part of Stream filter instance in octets.

StreamHandleSpec value (octets 5 to 8) StreamHandleSpec value contains the value of StreamHandleSpec as specified in IEEE 802.1Q [7] table 12-32.

PrioritySpec value (octets 9 to 12) PrioritySpec value contains the value of PrioritySpec as specified in IEEE 802.1Q [7] table 12-32.

StreamGateInstanceID value (octets 13 to 16) StreamGateInstanceID value contains the value of StreamGateInstanceID as specified in IEEE 802.1Q [7] table 12-32.

## 9.9	Stream gate instance table

The purpose of the Stream gate instance table information element is to convey a Stream gate instance table as defined in 3GPP TS 23.501 [2] table 5.28.3.1-1.

The Stream gate instance table information element is coded as shown in figure 9.9.1, figure 9.9.2, and table 9.9.1.

The Stream gate instance table is a type 6 information element with a minimum length of 3 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Stream gate instance table IEI |  |  |  |  |  |  |  | octet 1 |
| Length of Stream gate instance table contents |  |  |  |  |  |  |  | octet 2 octet 3 |
| Stream gate instance 1 |  |  |  |  |  |  |  | octet 4* octet a* |
| … |  |  |  |  |  |  |  |  |
| Stream gate instance N |  |  |  |  |  |  |  | octet b* octet c* |

Figure 9.9.1: Stream gate instance table information element

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Length of Stream gate instance contents |  |  |  |  |  |  |  | octet 4 octet 5 |
| StreamGateInstance |  |  |  |  |  |  |  | octet 6 octet 9 |
| PSFPAdminBaseTime value |  |  |  |  |  |  |  | octet 10 octet 19 |
| PSFPAdminCycleTime value |  |  |  |  |  |  |  | octet 20 octet 27 |
| PSFPTickGranularity value |  |  |  |  |  |  |  | octet 28 octet 31 |
| PSFPAdminControlListLength value |  |  |  |  |  |  |  | octet 32 octet 33 |
| PSFPAdminControlList contents |  |  |  |  |  |  |  | octet 34 octet a |

Figure 9.9.2: Stream gate instance

Table 9.9.1: Stream gate instance table

Value part of the Stream gate instance table information element (octets 4 to c)

Stream gate instance table contents (octets 4 to c) This field consists of zero or more Stream gate instances.

Stream gate instance (octets 4 to a)

Length of Stream gate instance (octets 4 to 5) Length of Stream gate instance contents contains the length of the vale part of Stream gate instance in octets.

StreamGateInstance value (octets 6 to 9) StreamGateInstance value contains the value of StreamGateInstance as specified in IEEE 802.1Q [7] table 12-33.

PSFPAdminBaseTime value (octets 10 to 19) PSFPAdminBaseTime value contains the value of PSFPAdminBaseTime as specified in IEEE 802.1Q [7] table 12-33.

PSFPAdminCycleTime value (octets 20 to 27) PSFPAdminCycleTime value contains the value of PSFPAdminCycleTime as specified in IEEE 802.1Q [7] table 12-33.

PSFPTickGranularity value (octets 28 to 31) PSFPTickGranularity value contains the value of PSFPTickGranularity as specified in IEEE 802.1Q [7] table 12-33.

PSFPAdminControlListLength value (octets 32 to 33) PSFPAdminControlListLength value contains the value of PSFPAdminControlListLength as specified in IEEE 802.1Q [7] table 12-33.

PSFPAdminControlList contents (octets 34 to a) This field contains the concatenation of entries, each encoded as a PSFPGateControlEntry as specified in IEEE 802.1Q [7] table 12-33. PSFPAdminControlListLength value indicates number of entries in this field.

# 10	Timers of Ethernet port management service

Timers of Ethernet port management service are shown in table 10.1, table 10.2, and table 10.3.

Table 10.1: Timers of Ethernet port management service – TSN AF side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |
| --- | --- | --- | --- | --- |
| T35xx | NOTE | Transmission of MANAGE ETHERNET PORT COMMAND message | MANAGE ETHERNET PORT COMPLETE message received | Retransmission of MANAGE ETHERNET PORT COMMAND message |
| NOTE: The value of this timer is network dependent. |  |  |  |  |

Table 10.2: Timers of Ethernet port management service – DS-TT side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |
| --- | --- | --- | --- | --- |
| T35yy | NOTE | Transmission of ETHERNET PORT MANAGEMENT NOTIFY message | ETHERNET PORT MANAGEMENT NOTIFY ACK message received | Retransmission of ETHERNET PORT MANAGEMENT NOTIFY message |
| NOTE: The value of this timer is DS-TT dependent. |  |  |  |  |

Table 10.3: Timers of Ethernet port management service – NW-TT side

| TIMER NUM. | TIMER VALUE | CAUSE OF START | NORMAL STOP | ON THE 1st, 2nd, 3rd, 4th EXPIRY |
| --- | --- | --- | --- | --- |
| T35zz | NOTE | Transmission of ETHERNET PORT MANAGEMENT NOTIFY message | ETHERNET PORT MANAGEMENT NOTIFY ACK message received | Retransmission of ETHERNET PORT MANAGEMENT NOTIFY message |
| NOTE: The value of this timer is NW-TT dependent. |  |  |  |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-11 | CT1#121 | C1-198752 |  |  |  | Draft skeleton provided by the rapporteur | 0.0.0 |
| 2019-11 | CT1#121 |  |  |  |  | Implementation of the following pseudo CRs agreed by CT1: C1-198019, C1-198174, C1-198482, C1-198753, C1-198757, C1-198758, and C1-199024 Corrections made by the rapporteur | 0.1.0 |
| 2019-12 | CT#86 | CP-193159 |  |  |  | Presentation for information at TSG CT | 1.0.0 |
| 2019-12 | CT#86 | CP-193292 |  |  |  | A title corrected | 1.0.1 |
| 2020-03 | CT1#122 |  |  |  |  | Implementation of the following pseudo CRs agreed by CT1: C1-200330, C10200331, C1-200573, C1-200687, C1-200706, C1-200708, C1-200832 Corrections made by the rapporteur | 1.1.0 |
| 2020-03 | CT-87e | CP-200166 |  |  |  | Presentation for approval at TSG CT | 2.0.0 |
| 2020-03 | CT-87e | CP-200288 |  |  |  | Revision after implementation of CP-200095. Presentation for approval at TSG CT | 2.1.0 |