3GPP TR 24.890 V1.0.1 (2017-09)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System – Phase 1;

CT WG1 Aspects

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

5G

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	14

1	Scope	15

2	References	15

3	Definitions, symbols and abbreviations	16

3.1	Definitions	16

3.2	Abbreviations	17

4	Architecture	18

4.1	General	18

4.2	Architecture reference model	18

4.3	Architecture for interworking with E-UTRAN connected to EPC	19

5	Network selection procedures	20

5.1	Concepts	20

5.2	Procedures	21

5.2.1	General	21

5.2.2	Procedures for 3GPP radio access networks	21

5.2.3	Procedures for non-3GPP access networks	22

5.2.4	Procedures for steering UE in VPLMN	22

6	Stage 3 protocol, general aspects	22

6.1	General	22

6.2	Protocols and peer to peer communication	22

6.3	Basic groups of functions	23

6.4	Protocol architecture	23

6.5	Standard L3 messages	24

6.6	General message format and information elements coding	25

6.6.1	Overview	25

6.6.6.2	Extended protocol discriminator	25

6.6.6.3	Security header type	25

6.6.6.4	PDU session identity	26

6.6.6.5	Spare half octet	26

6.6.6.6	Procedure transaction identity	26

6.6.6.7	Message type	26

7	Coordination between the protocols for 5GS mobility management and 5GS session management	26

8	5GS mobility management	26

8.1	Overview	26

8.1.1	General	26

8.1.2	Domain selection	27

8.1.2.1	UE’s usage setting	27

8.1.2.2	Domain selection for UE originating sessions / calls	27

8.1.2.3	Change of UE’s usage setting	28

8.1.2.4	Change or determination of IMS voice availability	28

8.1.3	Types of 5GMM procedures	28

8.1.4	5GMM sublayer states	29

8.1.4.1	General	29

8.1.4.2	5GMM sublayer states over 3GPP access	29

8.1.4.2.1	General	29

8.1.4.2.2	5GMM sublayer states in the UE	30

8.1.4.2.2.1	General	30

8.1.4.2.2.2	Main states	30

8.1.4.2.2.2.1	5GMM-NULL	30

8.1.4.2.2.2.2	5GMM-DEREGISTERED	30

8.1.4.2.2.2.3	5GMM-REGISTERED-INITIATED	30

8.1.4.2.2.2.4	5GMM-REGISTERED	30

8.1.4.2.2.2.5	5GMM-DEREGISTERED-INITIATED	30

8.1.4.2.2.3	Substates of state 5GMM-DEREGISTERED	31

8.1.4.2.2.3.1	General	31

8.1.4.2.2.3.2	5GMM-DEREGISTERED.NORMAL-SERVICE	31

8.1.4.2.2.3.3	5GMM-DEREGISTERED.LIMITED-SERVICE	31

8.1.4.2.2.3.4	5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION	31

8.1.4.2.2.3.5	5GMM-DEREGISTERED.PLMN-SEARCH	31

8.1.4.2.2.3.6	5GMM-DEREGISTERED.NO-SUPI	31

8.1.4.2.2.3.7	5GMM-DEREGISTERED.NO-CELL-AVAILABLE	31

8.1.4.2.2.3.8	5GMM-DEREGISTERED.eCALL-INACTIVE	31

8.1.4.2.2.4	Substates of state 5GMM-REGISTERED	31

8.1.4.2.2.4.1	General	31

8.1.4.2.2.4.2	5GMM-REGISTERED.NORMAL-SERVICE	31

8.1.4.2.2.4.3	5GMM-REGISTERED.NON-ALLOWED-SERVICE	31

8.1.4.2.2.4.4	5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE	32

8.1.4.2.2.4.5	5GMM-REGISTERED.LIMITED-SERVICE	32

8.1.4.2.2.4.6	5GMM-REGISTERED.PLMN-SEARCH	32

8.1.4.2.2.4.7	5GMM-REGISTERED.NO-CELL-AVAILABLE	32

8.1.4.2.3	5GMM sublayer states in the network	32

8.1.4.2.3.1	General	32

8.1.4.2.3.2	5GMM-DEREGISTERED	33

8.1.4.2.3.3	5GMM-COMMON-PROCEDURE-INITIATED	33

8.1.4.2.3.4	5GMM-REGISTERED	33

8.1.4.2.3.5	5GMM-DEREGISTERED-INITIATED	33

8.1.4.3	Behaviour of the UE in state 5GMM-DEREGISTERED and state 5GMM-REGISTERED	33

8.1.4.3.1	General	33

8.1.4.3.2	UE behaviour in state 5GMM-DEREGISTERED	33

8.1.4.3.2.1	General	33

8.1.4.3.2.2	Detailed description of UE behaviour in state 5GMM-DEREGISTERED	33

8.1.4.3.2.2.1	NORMAL-SERVICE	33

8.1.4.3.2.2.2	LIMITED-SERVICE	33

8.1.4.3.2.2.3	ATTEMPTING-REGISTRATION	33

8.1.4.3.2.2.4	PLMN-SEARCH	34

8.1.4.3.2.2.5	NO-SUPI	34

8.1.4.3.2.2.6	NO-CELL-AVAILABLE	34

8.1.4.3.2.2.7	eCALL-INACTIVE	34

8.1.4.3.3	UE behaviour in state 5GMM-REGISTERED	34

8.1.4.3.3.1	General	34

8.1.4.3.3.2	Detailed description of UE behaviour in state 5GMM-REGISTERED	34

8.1.4.3.3.2.1	NORMAL-SERVICE	34

8.1.4.3.3.2.2	NON-ALLOWED-SERVICE	34

8.1.4.3.3.2.3	ATTEMPTING-REGISTRATION-UPDATE	34

8.1.4.3.3.2.4	LIMITED-SERVICE	34

8.1.4.3.3.2.5	PLMN-SEARCH	35

8.1.4.3.3.2.6	NO-CELL-AVAILABLE	35

8.1.5	Permanent identifiers	35

8.1.6	Temporary identities	35

8.1.7	Mobile initiated connection only mode	36

8.1.8	Handling of the periodic registration update timer and implicit deregistration timer	36

8.2	5GS mobility functions in IDLE and CONNECTED mode	36

8.2.1	Registration areas in the 5GS	36

8.2.1.1	General	36

8.2.1.2	Open issues on registration areas in the 5GS	37

8.2.2	UE NAS mobility functions in 5GMM-IDLE mode	37

8.2.3	UE NAS mobility functions in 5GMM-CONNECTED mode	38

8.2.4	5GMM-CONNECTED mode with RRC inactive indication	38

8.3	Establishment of the NAS signalling connection	39

8.4	IP address allocation	39

8.4.1	General	39

8.4.2	IP address allocation via NAS signalling	40

8.5	5GS mobility management procedures	41

8.5.1	5GMM common procedures	41

8.5.1.1	NAS transport procedures	41

8.5.1.1.1	General	41

8.5.1.1.2	Alternative 1 for NAS transport	41

8.5.1.1.2.1	SM transport procedures	41

8.5.1.1.2.1.1	UE-initiated SM message transport procedure	41

8.5.1.1.2.1.1.1	General	41

8.5.1.1.2.1.1.2	UE-initiated SM message transport initiation	41

8.5.1.1.2.1.1.3	UE-initiated SM message transport of messages	42

8.5.1.1.2.1.1.4	Abnormal cases on the network side	44

8.5.1.1.2.1.3	Network-initiated SM message transport procedure	44

8.5.1.1.2.1.3.1	General	44

8.5.1.1.2.1.3.2	Network-initiated SM message transport initiation	44

8.5.1.1.2.1.3.3	Network-initiated SM message transport of messages	45

8.5.1.1.2.2	Non-SM message transport procedures	45

8.5.1.1.2.2.1	General	45

8.5.1.1.2.2.2	UE-initiated non-SM message transport procedure	45

8.5.1.1.2.2.2.1	General	45

8.5.1.1.2.2.2.2	UE-initiated non-SM message transport initiation	45

8.5.1.1.2.2.2.3	UE-initiated non-SM message transport of messages	46

8.5.1.1.2.2.3	Network-initiated non-SM message transport procedure	46

8.5.1.1.2.2.3.1	General	46

8.5.1.1.2.2.3.2	Network-initiated non-SM message transport initiation	46

8.5.1.1.2.2.3.3	Network-initiated non-SM message transport of messages	47

8.5.1.1.3	Alternative 2 for NAS Transport	47

8.5.1.1.3.1	General	47

8.5.1.1.3.2	UE-initiated NAS transport procedure	47

8.5.1.1.3.2.1	General	47

8.5.1.1.3.2.2	UE-initiated NAS transport procedure initiation	47

8.5.1.1.3.2.3	UE-initiated NAS transport of messages	48

8.5.1.1.3.3	Network-initiated NAS transport procedure	49

8.5.1.1.3.3.1	General	49

8.5.1.1.3.3.2	Network-initiated NAS transport procedure initiation	49

8.5.1.1.3.3.3	Network-initiated NAS transport of messages	50

8.5.1.2	Generic UE configuration update procedure	50

8.5.1.2.1	General	50

8.5.1.2.2	UE Configuration update procedure initiated by the network	51

8.5.1.2.3	UE Configuration update accepted by the UE	51

8.5.1.2.4	UE Configuration update completion by the network	51

8.5.1.2.5	Abnormal cases in the UE	51

8.5.1.2.6	Abnormal cases on the network side	52

8.5.1.3	Identification procedure	52

8.5.1.3.1	General	52

8.5.1.3.2	UE Configuration update procedure initiated by the network	52

8.5.1.3.3	Identification response by the UE	52

8.5.1.3.4	Identification completion by the network	52

8.5.1.3.5	Abnormal cases in the UE	52

8.5.1.3.6	Abnormal cases on the network side	52

8.5.2	5GMM specific procedures	52

8.5.2.1	Primary authentication and key agreement procedure	52

8.5.2.1.1	General	52

8.5.2.1.2	EAP based primary authentication and key agreement procedure	53

8.5.2.1.2.1	General	53

8.5.2.1.2.2	EAP-AKA' related procedures	53

8.5.2.1.2.2.1	General	53

8.5.2.1.2.2.2	Initiation	53

8.5.2.1.2.2.3	UE successfully authenticates network	53

8.5.2.1.2.2.4	Errors when handling EAP-request/AKA'-challenge message	53

8.5.2.1.2.2.5	Network successfully authenticates UE	54

8.5.2.1.2.2.6	UE handling EAP-AKA' notification	54

8.5.2.1.2.2.7	Network sending EAP-success	54

8.5.2.1.2.2.8	UE handling EAP-success	54

8.5.2.1.3	5GS based primary authentication and key agreement procedure	54

8.5.2.1.3.1	General	54

8.5.2.1.3.2	Authentication initiation by the network	54

8.5.2.1.3.3	Authentication response by the UE	55

8.5.2.1.3.4	Authentication completion by the network	56

8.5.2.1.3.5	Authentication not accepted by the network	56

8.5.2.1.3.6	Authentication not accepted by the UE	56

8.5.2.1.3.7	Abnormal cases	56

8.5.2.2	Registration procedure	56

8.5.2.2.1	General	56

8.5.2.2.2	Registration procedure for initial registration	57

8.5.2.2.2.1	General	57

8.5.2.2.2.2	Initial registration initiation	57

8.5.2.2.2.3	5GMM common procedure initiation	58

8.5.2.2.2.4	Initial registration accepted by the network	58

8.5.2.2.2.5	Initial registration not accepted by the network	59

8.5.2.2.2.6	Initial registration for emergency services not accepted by the network	59

8.5.2.2.2.7	Abnormal cases in the UE	59

8.5.2.2.2.8	Abnormal cases on the network side	59

8.5.2.2.3	Registration procedure for mobility and periodic registration update	60

8.5.2.2.3.1	General	60

8.5.2.2.3.2	Mobility and periodic registration update initiation	60

8.5.2.2.3.3	5GMM common procedure initiation	61

8.5.2.2.3.4	Mobility and periodic registration update accepted by the network	61

8.5.2.2.3.5	Mobility and periodic registration update not accepted by the network	62

8.5.2.2.3.6	Abnormal cases in the UE	62

8.5.2.2.3.7	Abnormal cases on the network side	63

8.5.2.3	Deregistration procedure	63

8.5.2.3.1	General	63

8.5.2.3.2	UE-initiated deregistration procedure	63

8.5.2.3.2.1	UE-initiated deregistration procedure initiation	63

8.5.2.3.2.3	UE-initiated deregistration procedure completion for 5GS services over both 3GPP access and non-3GPP access	64

8.5.2.3.2.4	Abnormal cases in the UE	64

8.5.2.3.2.5	Abnormal cases in the network side	64

8.5.2.3.3	Network-initiated deregistration procedure	64

8.5.2.3.3.1	Network-initiated deregistration procedure initiation	64

8.5.2.3.3.2	Network-initiated deregistration procedure completion by the UE	65

8.5.2.3.3.3	Network-initiated deregistration procedure completion by the network	65

8.5.2.3.3.4	Abnormal cases in the UE	65

8.5.2.3.3.5	Abnormal cases in the network side	66

8.5.3	5GMM connection management procedures	66

8.5.3.1	Service request procedure	66

8.5.3.1.1	General	66

8.5.3.1.2	Service request procedure initiation	67

8.5.3.1.3	Common procedure initiation	67

8.5.3.1.4	Service request procedure accepted by the network	68

8.5.3.1.5	Service request procedure not accepted by the network	68

8.5.3.1.6	Abnormal cases in the UE	68

8.5.3.1.7	Abnormal cases on the network side	68

8.5.3.2	Paging procedure	68

8.5.3.2.1	General	68

8.5.3.2.2	Paging for 5GS services	68

8.5.3.2.2.1	General	68

8.5.3.2.2.2	Abnormal cases on the network side	69

8.5.3.2.2.3	Abnormal cases in the UE	69

8.5.3.3	Notification procedure	69

8.5.3.3.1	General	69

8.5.3.3.2	Notification procedure initiation	69

8.5.3.3.3	Notification procedure completion	70

8.5.3.3.4	Abnormal cases on the network side	70

8.6	5GS mobility management message coding	70

8.6.1	Authentication failure	70

8.6.1.1	Message definition	70

8.6.2	Authentication reject	71

8.6.2.1	Message definition	71

8.6.3	Authentication request	71

8.6.3.1	Message definition	71

8.6.4	Authentication response	72

8.6.4.1	Message definition	72

8.6.5	Registration request	72

8.6.5.1	Message definition	72

8.6.6	Registration accept	72

8.6.6.1	Message definition	72

8.6.7	Registration complete	72

8.6.7.1	Message definition	72

8.6.8	Registration reject	73

8.6.8.1	Message definition	73

8.6.9	UL SM message transport for Alternative 1	73

8.6.9.1	Message definition	73

8.6.10	DL SM message transport for Alternative 1	74

8.6.10.1	Message definition	74

8.6.11	UL generic transport for Alternative 1	74

8.6.11.1	Message definition	74

8.6.12	UL NAS transport for Alternative 2	75

8.6.12.1	Message definition	75

8.6.13	DL generic transport for Alternative 1	75

8.6.13.1	Message definition	75

8.6.14	DL NAS transport for Alternative 2	75

8.6.14.1	Message definition	75

8.6.15	Deregistration request (UE originating deregistration)	76

8.6.15.1	Message definition	76

8.6.16	Deregistration accept (UE originating deregistration)	76

8.6.16.1	Message definition	76

8.6.17	Deregistration request (UE terminated deregistration)	76

8.6.17.1	Message definition	76

8.6.18	Deregistration accept (UE terminated deregistration)	76

8.6.18.1	Message definition	76

8.6.19	Configuration update command	76

8.6.19.1	Message definition	76

8.6.20	Configuration update complete	77

8.6.20.1	Message definition	77

8.6.21	Identity request	77

8.6.21.1	Message definition	77

8.6.22	Identity response	78

8.6.22.1	Message definition	78

8.7	5GMM information elements	78

8.7.1	5GMM cause	78

8.7.2	NAS key set identifier	78

8.7.3	Authentication parameter RAND	78

8.7.4	Authentication parameter AUTN	78

8.7.5	SM message container (for alternative 1)	78

8.7.6	Request type (for alternative 1)	79

8.7.7	S-NSSAI (for alternative 1)	79

8.7.8	DNN (for alternative 1)	80

8.7.9	Message container type (for alternative 1)	80

8.7.10	Message container (for alternative 1)	81

8.7.11	Acknowledgement requested flag	81

8.7.12	5GS mobile identity	81

8.7.13	Tracking area identity list	82

8.7.14	Mobility restrictions	82

8.7.15	Network name	83

8.7.16	Time zone	83

8.7.17	Time zone and time	83

8.7.18	Daylight saving time	83

8.8	3GPP specific coding information defined within present document	83

8.8.1	Serving network name (SNN)	83

8.9	Timers of 5GS mobility management	83

8.10	Cause values for 5GS mobility management	85

8.10.1	Causes related to UE identification	85

8.10.2	Cause related to subscription options	85

8.10.3	Causes related to PLMN specific network failures and congestion/authentication failures	85

8.10.4	Causes related to invalid messages	85

9	5GS session management	86

9.1	General	86

9.2	Overview	86

9.2.1	PDU session types	86

9.2.2	PDU session management	86

9.2.3	Local area data network	87

9.3	5GSM sublayer states	87

9.3.1	General	87

9.3.2	5GSM sublayer states in the UE	87

9.3.2.1	Overview	87

9.3.2.2	PDU SESSION INACTIVE	88

9.3.2.3	PDU SESSION ACTIVE PENDING	88

9.3.2.4	PDU SESSION ACTIVE	88

9.3.2.5	PDU SESSION INACTIVE PENDING	88

9.3.2.6	PDU SESSION MODIFICATION PENDING	88

9.3.2.7	PDU SESSION ACTIVE PENDING	88

9.3.2.8	PDU SESSION ACTIVE	88

9.3.2.9	PDU SESSION INACTIVE PENDING	88

9.3.2.10	PDU SESSION MODIFICATION PENDING	88

9.3.3	5GSM sublayer states in the network	88

9.3.3.1	Overview	88

9.3.3.2	PDU SESSION INACTIVE	89

9.3.3.3	PDU SESSION ACTIVE	89

9.3.3.4	PDU SESSION INACTIVE PENDING	89

9.3.3.5	PDU SESSION MODIFICATION PENDING	89

9.4	5GS session management procedures	89

9.4.1	General	89

9.4.1.1	Principles of PTI handling for 5GSM procedures	89

9.4.2	UE-requested PDU session establishment procedure	92

9.4.2.1	General	92

9.4.2.2	UE-requested PDU session establishment procedure initiation	92

9.4.2.3	UE-requested PDU session establishment procedure accepted by the network	94

9.4.2.4	UE requested PDU session establishment procedure not accepted by SMF	95

9.4.2.5	Abnormal cases in the UE	95

9.4.2.6	Abnormal cases on the network side	95

9.4.3	PDU session establishment authentication and authorization procedure	96

9.4.3.1	General	96

9.4.3.2	PDU session establishment authentication and authorization procedure initiation	96

9.4.3.3	PDU session establishment authentication and authorization procedure accepting	97

9.4.3.4	PDU session establishment authentication and authorization procedure rejecting	97

9.4.3.5	Abnormal cases on the network side	98

9.4.3.6	Abnormal cases in the UE	98

9.4.4	UE-requested PDU session modification procedure	98

9.4.4.1	General	98

9.4.4.2	UE-requested PDU session modification procedure initiation	98

9.4.4.3	UE-requested PDU session modification procedure accepting	98

9.4.4.4	UE-requested PDU session modification procedure not accepted by the network	99

9.4.4.5	Abnormal cases in the UE	99

9.4.4.6	Abnormal cases on the network side	99

9.4.5	Network-requested PDU session modification procedure	99

9.4.5.1	General	99

9.4.5.2	Network-requested PDU session modification procedure initiation	99

9.4.5.3	Network-requested PDU session modification procedure accepting	100

9.4.5.4	Network-requested PDU session modification procedure not accepted by the UE	101

9.4.5.5	Abnormal cases on the network side	101

9.4.5.6	Abnormal cases in the UE	101

9.4.6	UE-requested PDU session release procedure	101

9.4.6.1	General	101

9.4.6.2	UE-requested PDU session release procedure initiation	101

9.4.6.3	UE-requested PDU session release procedure accepting	102

9.4.6.4	UE-requested PDU session release procedure rejecting	102

9.4.6.5	Abnormal cases in the UE	102

9.4.6.6	Abnormal cases on the network side	102

9.4.7	Network-requested PDU session release procedure	102

9.4.7.1	General	102

9.4.7.2	Network-requested PDU session release procedure initiation	103

9.4.7.3	Network-requested PDU session release procedure accepting	103

9.4.7.4	N1 SM delivery skipped	104

9.4.7.5	Abnormal cases on the network side	104

9.4.7.6	Abnormal cases in the UE	104

9.5	5GS session management coding	104

9.5.1	PDU session establishment request	104

9.5.1.1	Message definition	104

9.5.1.2	PDU session type	105

9.5.1.3	SSC mode	105

9.5.1.4	SM PDU DN request container	105

9.5.1.5	Extended protocol configuration options	105

9.5.2	PDU session establishment accept	105

9.5.2.1	Message definition	105

9.5.2.2	5GSM cause	106

9.5.2.3	PDU address	106

9.5.2.4	EAP message	106

9.5.2.5	RQ timer value	106

9.5.2.6	Extended protocol configuration options	106

9.5.3	 PDU session establishment reject	107

9.5.3.1	Message definition	107

9.5.3.2	Extended protocol configuration options	107

9.5.4	PDU session authentication request	107

9.5.4.1	Message definition	107

9.5.5	PDU session authentication accept	107

9.5.5.1	Message definition	107

9.5.6	PDU session authentication reject	108

9.5.6.1	Message definition	108

9.5.7	PDU session modification request	108

9.5.7.1	Message definition	108

9.5.7.2	Extended protocol configuration options	108

9.5.8	PDU session modification reject	109

9.5.8.1	Message definition	109

9.5.8.2	Extended protocol configuration options	109

9.5.9	PDU session modification command	109

9.5.9.1	Message definition	109

9.5.9.2	5GSM cause	110

9.5.9.3	Authorized QoS rules	110

9.5.9.4	Session AMBR	110

9.5.9.5	PDU session release time	110

9.5.9.6	Extended protocol configuration options	110

9.5.10	PDU session modification accept	110

9.5.10.1	Message definition	110

9.5.10.2	Extended protocol configuration options	111

9.5.11	PDU session modification command reject	111

9.5.11.1	Message definition	111

9.5.11.2	Extended protocol configuration options	111

9.5.12	PDU session release request	112

9.5.12.1	Message definition	112

9.5.12.2	Extended protocol configuration options	112

9.5.13	PDU session release reject	112

9.5.13.1	Message definition	112

9.5.13.2	Extended protocol configuration options	113

9.5.14	PDU session release command	113

9.5.14.1	Message definition	113

9.5.1.2	Extended protocol configuration options	113

9.5.15	PDU session release accept	114

9.5.15.1	Message definition	114

9.5.15.2	Extended protocol configuration options	114

9.6	5GSM information elements	114

9.6.1	PDU session type	114

9.6.2	SSC mode	115

9.6.3	Extended protocol configuration options	115

9.6.4	SM PDU DN request container	115

9.6.5	PDU address	115

9.7.6	5GSM cause	116

9.8.7	GPRS timer	117

9.9	Timers of 5GS session management	117

9.10	Cause values for 5GS session management	119

9.10.1	Causes related to nature of request	119

9.10.2	Protocol errors (e.g., unknown message)	119

10	Access to the 5G core network via non-3GPP access networks	120

10.1	General	120

10.2	Protocol stack	120

10.3	NAS over non-3GPP access	120

10.3.1	General	120

10.3.2	Mobility management over non-3GPP access	120

10.3.3	Session management over non-3GPP access	121

10.4	Registration over non-3GPP access	121

10.4.1	General	121

10.4.2	Registration over non-3GPP access	121

10.4.3	Registration over non-3GPP access and 3GPP access	121

10.4.4	De-registration over non-3GPP access	122

10.5	Selection of N3IWF	122

10.6	Procedures between UE and N3IWF	122

10.7.1	IKE SA creation procedure for UE registration via untrusted non-3GPP access	122

10.7.2	Child SA creation procedure for UE-requested PDU session establishment	122

10.7.2.1	General	123

10.7.2.2	Child SA creation procedure initiation	123

10.7.2.3	Child SA creation procedure accepted by the UE	123

10.7.2.4	Child SA creation procedure not accepted by the UE	123

10.7.2.5	Abnormal cases in the UE	123

10.7.2.6	Abnormal cases in the N3IWF	123

10.7.3	Child SA change procedure for PDU session modification	124

10.7.4	Child SA deletion procedure for PDU session release	124

10.7.4.1	General	124

10.7.4.2	Child SA deletion procedure initiation	124

10.7.4.3	Child SA deletion procedure accepted by the UE	124

10.7.4.5	Abnormal cases in the UE	124

10.7.4.6	Abnormal cases in the N3IWF	124

10.7.5	IKE SA deletion for deregistration via untrusted non-3GPP access	124

10.7.5.1	General	124

10.7.5.2	IKE SA deletion procedure initiation	124

10.7.5.3	IKE SA deletion procedure accepted by the UE	125

10.7.5.5	Abnormal cases in the UE	125

10.7.5.6	Abnormal cases in the N3IWF	125

10.8	IKEv2 payload extensions	125

11	Interworking with E-UTRAN connected to EPC	125

11.1	General	125

11.2	Single-registration mode	125

11.2.1	General	125

11.2.2	Coordination between 5GMM and EMM	125

11.3	Dual-registration mode	125

12	5G System core network impact on services, network functions and capabilities	126

12.1	Specific services	126

12.1.1	Public warning system	126

12.1.1.1	General	126

12.1.1.1.1	Service Requirements	126

12.1.1.1.2	Technology Framework Assumptions	126

12.1.1.2	Architecture alternative 1: Supporting PWS in 5GS via Service based Interface	126

12.1.1.2.1	Overview	126

12.1.1.2.2	Architecture description	126

12.1.1.2.2.1	PWS architecture	126

12.1.1.2.2.2	AMF functionality	127

12.1.1.2.2.3	CBCF functionality	127

12.1.1.2.2.4	NG-RAN functionality	127

12.1.1.2.3	Protocol Stack Overview	128

12.1.1.2.4	CBCF-AMF interface operations	128

12.1.1.3	Architecture alternative 2: Supporting PWS in 5GS via SBc like Interface	129

12.1.1.3.1	Overview	129

12.1.1.3.2	Architecture description	129

12.1.1.3.2.1	PWS architecture	129

12.1.1.3.2.2	AMF functionality	130

12.1.1.3.2.3	CBC functionality	130

12.1.1.3.2.4	NG-RAN functionality	130

12.1.1.3.3	NG-RAN Protocol Overview	131

12.1.1.3.4	CBC-AMF interface	131

12.1.2	SMS over NAS	131

12.1.2.1	Registration requirements	132

12.1.2.1.1	Initial registration initiation	132

12.1.2.1.2	Initial registration accepted by the network	132

12.1.2.1.3	Mobility and periodic registration update initiation	133

12.1.2.1.4	Mobility and periodic registration update accepted by the network	133

12.1.3	Emergency services	133

12.1.3.1	Indication of support of emergency services	133

12.1.3.2	Handling of emergency service for the UE in limited service state	133

12.1.3.3	Emergency PDU session setup	133

12.1.3.4	Handling of local emergency numbers	134

12.1.3.5	Security procedures for handling emergency services	134

12.1.4	Location services	134

12.1.5	Multimedia priority services	134

12.2	Access control	134

12.3	Security	135

12.4	Quality of service (QoS)	135

12.4.1	General	135

12.4.1.1	QoS rules	135

12.4.1.1.1	General	135

12.4.1.1.2	Signalled QoS rules	135

12.4.1.1.3	Derived QoS rules	135

12.4.1.2	Session-AMBR	136

12.4.1.3	UL user data packet matching	136

12.4.1.4	Reflective QoS	136

12.4.1.4.1	General	136

12.4.1.4.2	Derivation of UL packet filter from DL user data packet	136

12.4.1.4.3	Creating a derived QoS rule by reflective QoS in the UE	137

12.4.1.4.4	Prolonging lifetime of a derived QoS rule by reflective QoS in the UE	138

12.4.1.4.4	Deleting a derived QoS rule by reflective QoS in the UE	138

12.4.2	QoS coding	138

12.4.2.1	QoS rules coding	138

12.4.2.2	Session-AMBR coding	144

12.5	IM CN subsystem	144

12.5.1	IMS voice over PS Session supported Indication	144

12.5.2	P-CSCF address delivery	145

12.6	Network sharing	145

12.7	Congestion and overload control	145

12.8	Charging	145

12.9	Mission critical services	145

13	Network slicing	145

13.1	General	145

13.2	Mobility management aspects	145

13.2.1	General	145

13.2.2	NSSAI storage	146

13.3	Session management aspects	146

14	Conclusions and recommendations	146

Annex A: Impacts to specifications	146

A.1	New specifications	146

A.2	Impacted existing specifications	147

Annex B (informative): Proposed changes to 3GPP TS 23.040	148

B.1	First Change	148

1	Scope	148

B.2	Next Change	148

2	References	148

B.3	Next Change	151

Annex L (normative): SMS in 5GS	151

L.1	General	151

Annex C (informative): Proposed changes to 3GPP TS 24.011	152

C.1	First Change	152

1	Scope	152

1.1	References	152

1.2	Abbreviations	153

C.2	Next Change	154

2	Overview of Short Message Service (SMS) support	154

2.1	Protocols and protocol architecture	154

C.3	Next Change	157

2.5B	5GSMS entity in N1 mode	157

C.4	Next Change	157

3.2	Service provided by the CM-sublayer	157

C.5	Next Change	158

5	CM-procedures	158

5.1	General	158

C.6	Next Change	158

5.2.2	SMC-GP, SMC-EP and SMC-5G states at the MS side of the radio interface	158

5.2.2.1	Mobile Originating Case	158

C.7	Next Change	159

5.2.4	SMC-GP, SMC-EP and SMC-5G states at the network side of the radio interface	159

5.2.4.1	Mobile Originating Case	159

C.8	Next Change	159

5.3.2.2	RPDU transfer for GPRS, EPS and 5GS	159

Annex D (informative): Proposed changes to 3GPP TS 24.301	161

D.1	Summary of changes	161

Annex E: Change history	161

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document discusses and describes the CT1 aspects of the 5G System phase 1. In particular, these CT1 aspects include the non-access-stratum (NAS) functions to be performed by the user equipment (UE) in idle mode and the NAS signalling procedures between the UE and the 5G System (5GS) phase 1.

The present document also considers the requirements on the NAS between the UE and the core network which arise from specific services such as public warning system (PWS), SMS over NAS, emergency services, location services (LCS), multimedia priority services (MPS), and also network functions such as access control, security, QoS, IMS, network sharing, charging, congestion and overload control. In addition, the document considers the case of network slicing as well as interworking with E-UTRAN connected to EPC.

The present document is intended as a holding place for CT1 5GS material until it stabilises sufficiently to be moved to appropriate 3GPP technical specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.011: "Service accessibility".

[3]	3GPP TS 22.261: "Service requirements for the 5G system; Stage 1".

[4]	3GPP TS 22.268: "Public Warning System (PWS) Requirements".

[5]	3GPP TS 23.003: "Numbering, addressing and identification".

[6]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[7]	3GPP TS 23.122: "Non-Access-Stratum functions related to Mobile Station (MS) in idle mode".

[8]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[9]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[10]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[11]	3GPP TS 24.008: "Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3".

[12]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3".

[13]	3GPP TS 24.302: "Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3"

[14]	3GPP TS 29.168: "Cell Broadcast Centre interfaces with the Evolved Packet Core".

[15]	3GPP TS 33.501: "Security architecture and procedures for 5G System".

[16]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[17]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2"

[18]	3GPP TS 38.304: "New Generation Radio Access Network; User Equipment (UE) procedures in idle mode".

[19]	3GPP TS 38.413: "NG Radio Access Network (NG-RAN); NG Application Protocol (NGAP)".

[20]	IETF RFC 3736: "Stateless DHCP Service for IPv6".

[21]	IETF RFC 3748: "Extensible Authentication Protocol (EAP)".

[22]	IETF RFC 4191 (November 2005): "Default Router Preferences and More-Specific Routes".

[23]	IETF RFC 4862: "IPv6 Stateless Address Autoconfiguration".

[24]	IETF RFC 4960: "Stream Control Transmission Protocol".

[25]	IETF RFC 5448: "Improved Extensible Authentication Protocol Method for 3rd Generation Authentication and Key Agreement (EAP-AKA')".

[26]	IETF RFC 7296: "Internet Key Exchange Protocol Version 2 (IKEv2)".

[27]	ITU-T Recommendation E.212: "The international identification plan for mobile terminals and mobile users".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

5GMM-IDLE mode: A UE is in 5GMM-IDLE mode when no N1 NAS signalling connection between UE and network over 3GPP access exists. The term 5GMM-IDLE mode used in the present document corresponds to the term CM-IDLE state used in 3GPP TS 23.501 [8].

5GMM-CONNECTED mode: A UE is in 5GMM-CONNECTED mode when a N1 NAS signalling connection between UE and network over 3GPP access exists. The term 5GMM-CONNECTED mode used in the present document corresponds to the term CM-CONNECTED state used in 3GPP TS 23.501 [8].

N1 mode: A mode of a UE allowing access to the 5G core network via the 5G access network.

N1 NAS signalling connection: A peer to peer N1 mode connection between UE and AMF. An N1 NAS signalling connection is either the concatenation of an RRC connection via the Uu reference point and an NG connection via the N2 reference point for 3GPP access, or the concatenation of an IPsec tunnel via the NWu reference point and an NG connection via the N2 reference point for non-3GPP access.

PDU session for LADN: PDU session with a DNN associated with a LADN.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.501 [8] apply:

5G access network

5G core network

5G QoS flow

5G QoS indicator

5G-GUTI

5G System

5G-S-TMSI

Allowed area

NG-RAN

Allowed NSSAI

Configured NSSAI

Local area data network

Network slice

Non-allowed area

PDU session

PEI

Requested NSSAI

SUPI

For the purposes of the present document, the following terms and definitions given in 3GPP TS 38.413 [19] apply:

NG connection

Editor's note:	The term (NG connection) can be modified according to 3GPP TS 38.413 [19].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

4G-GUTI	4G-Globally Unique Temporary Identifier

5GCN	5G Core Network

5G-GUTI	5G-Globally Unique Temporary Identifier

5GS	5G System

5GMM	5GS Mobility Management

5GSM	5GS Session Management

5QI	5G QoS Indicator

AKA	Authentication and Key Agreement

DL	Downlink

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EAP	Extensible Authentication Protocol

EAP-AKA'	Improved Extensible Authentication Protocol method for 3rd generation Authentication and Key Agreement

EPD	Extended Protocol Discriminator

EMM	EPS Mobility Management

EPC	Evolved Packet Core Network

EPS	Evolved Packet System

GFBR	Guaranteed Flow Bit Rate

HRPD	High Rate Packet Data

LADN	Local Area Data Network

LMF	Location Management Function

LPP	LTE Positioning Protocol

MFBR	Maximum Flow Bit Rate

MICO	Mobile Initiated Connection Only

N3IWF	Non-3GPP InterWorking Function

NAI	Network Access Identifier

ngKSI	Key Set Identifier for Next Generation Radio Access Network

NSSAI	Network Slice Selection Assistance Information

NSSP	Network Slice Selection Policy

QFI	QoS Flow Identity

QoS	Quality of Service

QRI	Qos Rule Identifier

RQI	Reflective QoS Indication

S-NSSAI	Single NSSAI

SA	Security Association

SNN	Serving Network Name

UL	Uplink

TA	Tracking Area

TAC	Tracking Area Code

TAI	Tracking Area Identity

# 4	Architecture

## 4.1	General

The 5GS architecture is specified in 3GPP TS 23.501 [8] and the stage 2 procedures and flows for 5GS are specified in 3GPP TS 23.502 [9].

## 4.2	Architecture reference model

The 5GS architecture reference model can be represented in two ways; a service-based representation and a reference point representation (see figures 4.2.1, 4.2.2). The service-based representation also includes point-to-point reference points where necessary. The 5GS architecture reference model covers both roaming and non-roaming scenarios in all aspects, including interworking between 5GS and EPS, mobility within 5GS, policy control and charging, and authentication.

Figure 4.2.1: Non-roaming 5GS architecture in reference point representation

Figure 4.2.2: Non-roaming architecture for 5GCN with untrusted non-3GPP access

A UE, which supports 5GS connectivity, can access the 5GCN via a NG-RAN or a standalone non-3GPP access or both. The NAS signalling with the 5GCN control-plane functions is over the N1 reference point. The N11 reference point is used to connect the core access and mobility management function (AMF) and the session management function (SMF). The SMF handles the session management part of the NAS signalling exchanged with the UE. The NWu reference point is used for establishing secure tunnel(s) between the UE and the N3IWF so that control-plane and user-plane exchanged between the UE and the 5GCN is transferred securely over untrusted non-3GPP access. CT1 will specify the N1 and NWu interfaces.

## 4.3	Architecture for interworking with E-UTRAN connected to EPC

Interworking between the 5GS and the EPC through E-UTRAN is supported and specified in 3GPP TS 23.501 [8] as illustrated by the figure 4.3.1. The system interworking procedures are specified in 3GPP TS 23.502 [9].

Figure 4.3.1: Non-roaming architecture for interworking between the 5GS and the EPC/E-UTRAN

The N26 interface is an inter-CN interface between the MME and the AMF used to enable interworking between the EPC and the 5GCN. Support of the N26 interface is optional. However, single-registration mode (see 3GPP TS 23.501 [8]) requires support for N26 interface between the AMF and the MME to enable seamless session continuity (e.g. for voice services) for N1 mode to S1 mode inter-system change and vice versa.

Editor's note:	The details of the N1 mode to S1 mode inter-system change and S1 mode to N1 mode inter-system change is FFS and will be captured by CT1 based on stage 2 requirements (e.g., UE behaviour, mapping of identities, mapping of contexts).

Editor's note:	Whether to support the single-registration mode without N26 interface is FFS.

# 5	Network selection procedures

## 5.1	Concepts

The 5GS supports both 3GPP and non-3GPP access networks as specified in 3GPP TS 22.261 [3].

As per 3GPP TS 22.261 [3] the 5G system supports the concept of "network slices" where different 5G radio access networks potentially are connected to network slices of different SSTs. Network slicing is not taken into account for PLMN selection. The network slices are selected only after the UE has registered to a PLMN.

Editor's note:	It is for further study how to support the concept of "network slices" of 5G radio access networks.

Requirements on PLMN selection for 3GPP access in 3GPP TS 22.011 [2], clause 3.2 apply together with new requirements specified by 3GPP TS 22.261 [3] subclause 6.19.2.

PLMN selection for 3GPP access does not depend on PLMN selection for non-3GPP access. If a UE is registered over non-3GPP access, the UE performs PLMN selection for 3GPP access independently of the PLMN to which the N3IWF belongs.

Editor's note:	It is FFS whether PLMN selection for non-3GPP access depends on PLMN selection for 3GPP access, e.g. PLMN for 3GPP access can be taken into account for PLMN selection for non-3GPP access.

PLMNs selected for 3GPP access and non-3GPP access may be the same or different.

PLMN selection defines a UE-based procedure, whereby candidate PLMNs are selected, one at a time, for attempted registration. Once the PLMN selection has been performed, the UE follows the signalling procedures defined for the selected RAT.

Editor's note:	It is for further study whether a common PLMN selection procedure can apply to both 3GPP access and non-3GPP access.

## 5.2	Procedures

### 5.2.1	General

The radio access technology (RAT) identifier associated with each entry in the "User controlled PLMN selector with Access Technology", "Operator controlled PLMN selector with Access Technology" and "HPLMN selector with Access Technology" data files can indicate any combination of the values: GSM, GSM COMPACT, UTRAN, E-UTRAN, cdma2000® 1xRTT, cdma2000® HRPD and NG-RAN access technologies.

Editor’s note:	It is FFS whether E-UTRA connected to 5GC access should be distinguished from E-UTRA connected to EPC and whether E-UTRA connected to 5GC should be distinguished from NR.

NOTE:	cdma2000® is a registered trademark of the Telecommunications Industry Association (TIA-USA).

Any signalling procedures after PLMN selection are defined in the specifications related with the radio access technology of the selected PLMN.

### 5.2.2	Procedures for 3GPP radio access networks

The same procedures for PLMN selection applicable for the GSM, UTRAN and E-UTRAN access technologies as described in 3GPP TS 22.011 [2] and 3GPP TS 23.122 [7] also apply for NG-RAN, with the following additions and exceptions:

-	when the UE is informed that an area is forbidden during a registration procedure or a UE configuration update procedure, the UE shall take the following action depending on the received 5GMM cause value:

1)	#12	(tracking area not allowed);

The UE shall enter the state 5GMM-DEREGISTERED.LIMITED-SERVICE and store the current TAI in the list of "5GS forbidden tracking areas for regional provision of service".

2)	#13	(roaming not allowed in this tracking area);

The UE shall enter the state 5GMM-DEREGISTERED.PLMN-SEARCH, store the current TAI in the list of "5GS forbidden tracking areas for roaming" and shall perform PLMN selection according to 3GPP TS 23.122 [7].

The lists of "5GS forbidden tracking areas for roaming" and of "5GS forbidden tracking areas for regional provision of service" shall be erased when:

1)	the UE is switched off;

2)	the UICC containing the USIM is removed; and

3)	periodically (with a period in the range 12 to 24 hours).

When the lists are erased, the UE performs cell selection.

One or more tracking areas are removed from the list of "5GS forbidden tracking areas for roaming" in the UE, as well as the list of "5GS forbidden tracking areas for regional provision of service" if, after a subsequent procedure e.g. registration procedure, one or more tracking areas in the lists are received from the network. If the UE has only one PDU session established which is for emergency services, the tracking areas shall not be removed from these lists if one or more tracking areas in the lists are received from the network.

Each list shall accommodate 40 or more TAIs. When the list is full and a new entry has to be inserted, the oldest entry shall be deleted.

-	one single list of forbidden PLMNs ("forbidden PLMNs" list) is sufficient for 5GS. The existing "forbidden PLMNs for GPRS service" and "forbidden PLMNs for attach in S101 mode" lists are not used for 5GS.

Editor's note:	It is FFS how RAT restriction is achieved.

### 5.2.3	Procedures for non-3GPP access networks

Editor's note:	Currently there are no new requirements available.

### 5.2.4	Procedures for steering UE in VPLMN

The HPLMN can update the list of preferred PLMN/RATs at the UE e.g., depending on the UE current location via NAS signalling.

Editor's note:	It is FFS if this list of preferred PLMN/RATs is a new list to 5GS or an existing list.

Editor's note:	It is FFS how the list preferred PLMN/RATs is managed and used by the UE.

The VPLMN shall not be able to modify the information sent by the HPLMN. The UE shall be able to verify the integrity of the received information.

Editor's note:	SA3 needs to confirm whether it is feasible to ensure the VPLMN cannot alter the contents of the list of preferred PLMN/RATs sent to the UE by the VPLMN.

Editor's note:	End-to-end security between HPLMN and roaming UEs will be defined following SA3 recommendations.

# 6	Stage 3 protocol, general aspects

## 6.1	General

This subclause defines the principal architecture of the 5GS NAS layer 3 protocol and its sublayers, including the message format applied by layer 3.

The layer 3 for 5GS services is composed of three sublayers comprising:

-	the 5GS radio resource management (5GRR) functions;

-	the 5GS mobility management (5GMM) functions; and

-	the 5GS connection management (5GCM) functions.

The 5GS mobility management (5GMM) sublayer is composed of a functional block for:

-	the 5GS mobility management (5GMM) functions.

The 5GS connection management (5GCM) sublayer is composed of a functional block for:

-	the 5GS session management (5GSM) functions.

## 6.2	Protocols and peer to peer communication

In the model used in the present document, there are:

1)	for 5GS services:

-	one 5GRR entity in the MS and one 5GRR entity in the network;

-	one 5GMM entity in the MS and one 5GMM entity in the network;

-	for each functional block of the 5GCM sublayer as defined in subclause 6.1 which is supported in the MS (in the network), there are, depending on the protocol, one or more entities in the MS (in the network). Two different entities of the same functional block in the MS (in the network) are called parallel entities. The entities of the same functional block in the MS correspond in a one-to-one relation to the entities of the functional block in the network. The corresponding entities are called peer entities.

## 6.3	Basic groups of functions

Most functions of layer 3 and its sub-layers are described by the service specifications and protocol specifications of the (sub-)layers.

These functions are in the model realized by protocol control entities, see subclause 6.2.

In addition, routing functions are contained in layer 3 which are related to the transport of messages, e.g. multiplexing and splitting. These routing functions are defined in the 5GRR and 5GMM sub-layers.

1)	They have the task to pass the messages from upper (sub-)layers to lower (sub-)layers.

2)	They also have the task to pass messages provided by lower (sub-)layers to the appropriate (sub-)layer and, if applicable, entity.

The routing functions with task 2 make use of the protocol discriminator (PD) which is part of the message header, or PDU session identity.

A 5GCM sublayer protocol may also define a procedure transaction identity (PTI) as a part of the message header. This is at least the case if there are parallel entities of the same functional block, see subclause 6.2.

-	The 5GMM-sublayer routing function passes the messages of the 5GCM entities as well as of the 5GMM entities of its own sublayer to the service access point of 5GRR. Furthermore, it multiplexes them in case of parallel transactions.

-	The new radioaccess stratum (NRAS) sublayer routing function distributes the messages to be sent according to their message type and protocol discriminator (PD), to the actual channel configuration, and, if applicable, to further information received from upper sub-layers to the appropriate service access point of layer 2. Paging messages received from the PCH are always routed to 5GMM.

-	The messages provided at the different service access points of layer 2 are distributed by the 5GRR sublayer routing function according to their protocol discriminator (PD). Messages with a PD equal to RR are passed to the 5GRR entity of the own sublayer, all other messages are passed to the 5GMM sublayer at the service access point RR-SAP.

-	For 5GS services, the routing function of 5GS NAS passes standard L3 messages according to the protocol discriminator (PD) and, if applicable, the PDU session identity towards the 5GMM entity or towards the 5GSM entities of the various 5GS NAS SAP's.

The message (message header or other parts of the message) are neither changed nor removed by the RR routing function or 5GMM routing function before passing it to the appropriate service access point.

## 6.4	Protocol architecture

The protocol architecture is visualized for each of the 5GS model:

-	Figure 6.4.1 shows the protocol architecture for a MS supporting 5GS services.

Figure 6.4.1: Protocol architecture of Non Access Stratum supporting MSs, MS-side

Editor’s note:	Addition of non-3GPP in the protocol architecture is FFS

Figure 6.4.1 defines three sublayers for 5GS services:

-	the NRAS sublayer provides services to the 5GMM sublayer;

-	the 5GMM sublayer provides services to the entities of the 5GCM sublayer. The 5GMM sublayer further includes one 5GMM entity;

-	the 5GCM sublayer includes 5GSM entities;

-	the 5GSM entity provides services to the QoS flow control (QFC) entity and uses services of the 5GMM sublayer;

-	the QFC entity hides the concepts of radio resources that can be established/released while an 5GS context is active. If uplink data in the terminal is to be sent, and 5GS radio resources have been released, the QFC will trigger a service request procedure in 5GMM.

## 6.5	Standard L3 messages

Editor's note:	5GS NAS message IE applicability, 5GS NAS message standard L3 header, 5GS Protocol discriminator values, Possible new 5GS identities for routing and 5GS message type should be captured in this subclause.

## 6.6	General message format and information elements coding

### 6.6.1	Overview

Within the protocols defined in the present document, every 5GS NAS message is a standard L3 message as defined in 3GPP TS 24.007 [10]. This means that the message consists of the following parts:

Editor's note:	The required updates on a standard L3 message definition in 3GPP TS 24.007 [10] are FFS.

1)	if the message is a plain 5GS NAS message:

a)	extended protocol discriminator;

b)	security header type or PDU session identity;

c)	procedure transaction identity;

d)	message type;

e)	other information elements, as required.

2)	if the message is a security protected 5GS NAS message:

Editor's note:	The message parts of a security protected 5GS NAS message are FFS.

The organization of a plain 5GS NAS message is illustrated in the example shown in figure 6.6.1.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Extended protocol discriminator |  |  |  |  |  |  |  | octet 1 |
| Spare half octet |  |  |  | Security header type or PDU session identity |  |  |  | octet 2 |
| Procedure transaction identity |  |  |  |  |  |  |  | octet 3 |
| Message type |  |  |  |  |  |  |  | octet 4 |
|  |  |  |  |  |  |  |  | octet 5 |
| Other information elements as required |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

# Figure 6.6.1: General message organization example for a plain 5GS NAS message

Editor's note:	The organization of a security protected 5GS NAS message is FFS.

Unless specified otherwise in the message descriptions of clause 8 and clause 9, a particular information element shall not be present more than once in a given message.

When a field extends over more than one octet, the order of bit values progressively decreases as the octet number increases. The least significant bit of the field is represented by the lowest numbered bit of the highest numbered octet of the field.

#### 6.6.6.2	Extended protocol discriminator

Bits 1 to 8 of the first octet of every 5GMM message contain the Extended protocol discriminator (EPD) IE. The EPD and its use are defined in 3GPP TS 24.007 [10]. Bits 1 to 4 of the EPD in the header of a plain 5GMM message is encoded as "1110".

Editor's note:	The definition of the extended protocol discriminator in 3GPP TS 24.007 [10] is FFS.

Editor's note:	Whether the Extended protocol discriminator IE is included in the header of 5GSM message is FFS.

#### 6.6.6.3	Security header type

Bits 1 to 4 of the second octet of every 5GMM message contain the Security header type IE. This IE includes control information related to the security protection of a 5GMM message. The total size of the Security header type IE is 4 bits.

Editor's note:	The values of the Security header type are FFS.

#### 6.6.6.4	PDU session identity

Bits 1 to 4 of the second octet of every 5G session management message contain the PDU session identity IE. The PDU session identity and its use to identify a message flow are defined in 3GPP TS 24.007 [10].

Editor's note:	The definition of the PDU session identity in 3GPP TS 24.007 [10] is FFS.

#### 6.6.6.5	Spare half octet

Bits 5 to 8 of the second octet of every 5GS NAS message contains the spare half octet which is filled with spare bits set to zero.

#### 6.6.6.6	Procedure transaction identity

Bits 1 to 8 of the third octet of every 5GSM message contain the procedure transaction identity. The procedure transaction identity and its use are defined in 3GPP TS 24.007 [10].

Editor's note:	Whether the procedure transaction identity is included in the header of 5GMM message is FFS.

#### 6.6.6.7	Message type

The message type IE and its use are defined in 3GPP TS 24.007 [10].

Editor's note:	The values of message type for 5GS NAS messages are FFS.

# 7	Coordination between the protocols for 5GS mobility management and 5GS session management

A 5GS session management (5GSM) message is piggybacked in a 5GS mobility management (5GMM) message. To this purpose, the 5GSM messages can be transmitted in an information element in the 5GMM messages. In this case, the UE, the AMF and the SMF execute the 5GMM procedure and the 5GSM procedure in parallel. The success of the 5GMM procedure is not dependent on the success of the piggybacked 5GSM procedure.

Editor's note:	It is FFS whether an initial registration request procedure can be used to piggyback 5GSM messages.

The UE can only initiate the 5GSM procedure when there is a 5GMM context established at the UE and the AMF can only forward the 5GSM message for the 5GSM procedure initiated by the SMF when there is a 5GMM context established at the AMF.

Except for the 5GMM procedures piggybacking 5GSM messages, during 5GMM procedures the UE and the AMF shall suspend the transmission of 5GSM messages.

Editor's note:	Which type of procedure will be included as 5GMM procedure is FFS.

# 8	5GS mobility management

Editor’s note:	The names of messages are informational and used for work in progress. Final 5GMM names are TBD.

## 8.1	Overview

### 8.1.1	General

The main function of the 5GS mobility management (5GMM) sublayer is to support the identification, security, mobility of a UE as well as generic message transport.

A further function of the 5GMM sublayer is to provide connection management services to the other sublayer(s).

Editor's note:	Sublayer design can be revisited after the protocol framework is identified.

### 8.1.2	Domain selection

#### 8.1.2.1	UE’s usage setting

The UE’s usage setting indicates whether the UE has preference for voice services over data services or vice-versa, where:

-	voice services include IMS voice; and

-	data services include any kind of user data transfer without a voice media component.

The UE’s usage setting applies to voice capable UEs. The UE’s usage setting can be set to:

-	"voice centric" for 5GS; or

-	"data centric" for 5GS.

If a UE has preference for voice services, then the UE's usage setting is set to "voice centric" for 5GS. If a UE has preference for data services, then the UE's usage setting is set to "data centric" for 5GS. A UE whose setting is "data centric" for 5GS may still require access to voice services. A UE whose setting is "voice centric" for 5GS may still require access to data services.

Editor's note:	The relationship between the UE’s usage setting in 5GS and the UE’s usage setting in EPC is FFS.

#### 8.1.2.2	Domain selection for UE originating sessions / calls

The behaviour of the UE for domain selection is determined by:

-	the UE usage setting; and

-	the availability of IMS voice.

In the present document, "IMS voice not available" refers to one of the following conditions:

a)	the UE does not support IMS voice;

b)	the UE supports IMS voice, but the network indicates in the REGISTRATION ACCEPT message that IMS voice over PS sessions are not supported; or

c)	the UE supports IMS voice, the network indicates in the REGISTRATION ACCEPT message that IMS voice over PS sessions are supported, but the upper layers:

-	provide no indication that the UE is available for voice call in the IMS within a manufacturer determined period of time; or

-	indicate that the UE is not available for voice calls in the IMS.

NOTE:	If conditions a and b evaluate to false, the upper layers need time to attempt IMS registration. In the event an indication from the upper layers that the UE is available for voice calls in the IMS takes longer than the manufacturer determined period of time (e.g. due to delay when attempting IMS registration or due to delay in obtaining a QoS flow for SIP signalling), the NAS layer assumes the UE is not available for voice calls in the IMS.

Other conditions may exist but these are implementation specific.

In the present document, "IMS voice available" applies when "IMS voice not available" does not apply.

When IMS voice is not available, if the UE’s usage setting is "voice centric" for 5GS and the UE is capable of S1 mode, the UE shall disable the N1 mode radio capabilities, and attempt to select an E-UTRA cell connected to EPC. If such a cell is found, the UE shall then perform voice domain selection procedures as defined in 3GPP TS 24.301 [12].

Editor's note:	The N1 mode radio capabilities that the voice centric UE needs to disable in order to not reselect to NR or an E-UTRA cell connected to 5GC (i.e. avoid ping pong) need to be defined by RAN WGs.

The UE shall re-enable the N1 mode radio capabilities when:

-	the UE performs PLMN selection;

-	the UE powers off and powers on again or the USIM is removed; and

-	the UE’s usage setting is changed from "voice centric" for 5GS to "data centric" for 5GS.

Editor's note:	Other conditions when the UE re-enables the N1 mode radio capabilities are FFS.

#### 8.1.2.3	Change of UE’s usage setting

Whenever the UE's usage setting changes, the UE shall execute procedures according to table 8.1.2.3.1:

# Table 8.1.2.3.1: Change of UE's usage setting

| UE's usage setting change | Procedure to execute |
| --- | --- |
| From "data centric" in 5GS to "voice centric" in 5GS and "IMS voice not available" | If the UE is capable of S1 mode, disable the N1 mode radio capabilities and attempt to select an E-UTRA cell connected to EPC. |
| From "voice centric" in 5GS to "data centric" in 5GS and the N1 mode radio capabilities are disabled at the UE | Re-enable the N1 mode radio capabilities. |

#### 8.1.2.4	Change or determination of IMS voice availability

Whenever the IMS voice availability is determined or changes, the UE shall execute procedures according to table 8.1.2.4.1:

# Table 8.1.2.4.1: Change of IMS voice availability

| Change of IMS voice available condition | Procedure to execute |
| --- | --- |
| IMS voice not available and the UE’s usage setting is "voice centric" in 5GS | If the UE is capable of S1 mode, disable the N1 mode radio capabilities and attempt to select an E-UTRA cell connected to EPC. |
| IMS voice available and the N1 mode radio capabilities are disabled at the UE | Re-enable the N1 mode radio capabilities. |

### 8.1.3	Types of 5GMM procedures

Depending on how they can be initiated, three types of 5GMM procedures can be distinguished:

1)	5GMM common procedures

Editor's note:	More procedures (e.g. related to security, identification) will be updated according to the results of the work in SA2 and SA3.

Initiated by the network or the UE and used to carry a 5GMM message and/or any other encapsulated message(s) e.g. SMS or when the AMF wants to change UE configuration. The procedures belonging to this type are:

Initiated by the network:

-	(alternative 1) network-initiated SM message transport.

-	(alternative 1) network-initiated non-SM message transport.

-	(alternative 2) network-initiated NAS transport.

-	generic UE Configuration update

-	identification

Initiated by the UE:

-	(alternative 1) UE-initiated SM message transport.

-	(alternative 1) UE-initiated non-SM message transport.

-	(alternative 2) UE-initiated NAS transport.

Editor's note:	Alternative 1 is to define network/UE-initiated SM/non-SM message transport procedures (see subclause 8.4.2.2.1.1) and alternative 2 is to define network/UE-initiated NAS transport procedures (see subclause 8.4.2.2.3.2). CT1 will choose a single alternative.

2)	5GMM specific procedures:

At any time only one UE initiated 5GMM specific procedure can be running for each of the access network(s) that the UE is camping in. The procedures belonging to this type are:

Initiated by the UE and used e.g. to register to the network for 5GS services and establish a 5GMM context, to update the location/parameter(s) of the UE:

-	registration.

Initiated by the UE or the network and used to deregister from the network for 5GS services and to release a 5GMM context:

-	de-registration.

3)	5GMM connection management procedures:

Initiated by the UE and used to establish a secure connection to the network or to request the resource reservation for sending data, or both:

-	service request.

The service request procedure can only be initiated if no UE initiated 5GMM specific procedure is ongoing for each of the access network(s) that the UE is camping in.

Initiated by the network and used to request the establishment of an N1 NAS signalling connection or to prompt the UE to perform re-registration if necessary as a result of a network failure; not applicable for the non-3GPP access network:

-	paging.

NOTE:	The network can page the UE over 3GPP access for the re-activation of the PDU session(s) associated with non-3GPP access over 3GPP access.

### 8.1.4	5GMM sublayer states

#### 8.1.4.1	General

In the following subclauses, the 5GS mobility management (5GMM) sublayer of the UE and the network is described by means of different state machines. The 5GMM sublayer states is managed per access type independently, i.e. 3GPP access or non-3GPP access. In subclause 8.1.4.2, the states of the 5GMM sublayer over 3GPP access are introduced. The states of the 5GMM sublayer over non-3GPP access are described in subclause 11.2.2.

#### 8.1.4.2	5GMM sublayer states over 3GPP access

##### 8.1.4.2.1	General

In the following subclauses, the possible 5GMM sublayer states over 3GPP access are described. In subclause 8.1.4.2.2, the states of the 5GMM sublayer in the UE are introduced. The states for the network side are described in subclause 8.1.4.2.3.

##### 8.1.4.2.2	5GMM sublayer states in the UE

###### 8.1.4.2.2.1	General

In the following subclauses, the possible 5GMM sublayer states of the UE are described and shown in Figure 8.1.4.2.2.1.1.

NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GMM procedures are FFS.

# Figure 8.1.4.2.2.1.1: 5GMM main states in the UE (3GPP access)

###### 8.1.4.2.2.2	Main states

8.1.4.2.2.2.1	5GMM-NULL

5GS services are disabled in the UE. No 5GS mobility management function shall be performed in this state.

8.1.4.2.2.2.2	5GMM-DEREGISTERED

In the state 5GMM-DEREGISTERED, no 5GMM context has been established and the UE location is unknown to the network and hence it is unreachable by a network. In order to establish a 5GMM context, the UE shall start the initial registration procedure.

8.1.4.2.2.2.3	5GMM-REGISTERED-INITIATED

A UE enters the state 5GMM-REGISTERED-INITIATED after it has started the initial registration procedure, the non-initial registration procedure (including the normal registration update and periodic regstriation update) or the service request procedure and is waiting for a response from the network.

8.1.4.2.2.2.4	5GMM-REGISTERED

In the state 5GMM-REGISTERED, a 5GMM context has been established. Additionally, one or more PDU session context(s) may be activated at the UE. The UE may initiate the non-initial registration procedure (including the normal regstriation update and periodic registration update) and the service request procedure.

8.1.4.2.2.2.5	5GMM-DEREGISTERED-INITIATED

A UE enters the state 5GMM-DEREGISTERED-INITIATED after it has requested release of the 5GMM context by starting the deregistration procedure and is waiting for a response from the network.

###### 8.1.4.2.2.3	Substates of state 5GMM-DEREGISTERED

8.1.4.2.2.3.1	General

The state 5GMM-DEREGISTERED is subdivided into a number of substates as described in this subclause. Valid subscriber data are available for the UE before it enters the substates, except for the substate 5GMM-DEREGISTERED.NO-SUPI.

8.1.4.2.2.3.2	5GMM-DEREGISTERED.NORMAL-SERVICE

The substate 5GMM-DEREGISTERED.NORMAL-SERVICE is chosen in the UE when a suitable cell has been found and the PLMN or tracking area is not in the forbidden list.

8.1.4.2.2.3.3	5GMM-DEREGISTERED.LIMITED-SERVICE

The substate 5GMM-DEREGISTERED.LIMITED-SERVICE is chosen in the UE, when it is known that a selected cell is unable to provide normal service (e.g. the selected cell is in a forbidden PLMN or is in a forbidden tracking area).

8.1.4.2.2.3.4	5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION

The substate 5GMM-DEREGISTERED.ATTEMPTING-REGISTRATION is chosen in the UE if the initial registration procedure failed due to a missing response from the network.

Editor's note:	Other cases in which this substate is chosen are FFS.

8.1.4.2.2.3.5	5GMM-DEREGISTERED.PLMN-SEARCH

The substate 5GMM-DEREGISTERED.PLMN-SEARCH is chosen in the UE, if the UE is searching for PLMNs. This substate is left either when a cell has been selected (the new substate is NORMAL-SERVICE or LIMITED-SERVICE) or when it has been concluded that no cell is available at the moment (the new substate is NO-CELL-AVAILABLE).

8.1.4.2.2.3.6	5GMM-DEREGISTERED.NO-SUPI

The substate 5GMM-DEREGISTERED.NO-SUPI is chosen in the UE, if the UE has no valid subscriber data available (SIM/USIM not available, the SIM/USIM is considered invalid by the UE) and a cell has been selected.

Editor's note:	It is FFS whether other type of SUPI needs to be used, e.g. NAI.

8.1.4.2.2.3.7	5GMM-DEREGISTERED.NO-CELL-AVAILABLE

No 5G cell can be selected. This substate is entered after a first intensive search failed when in substate 5GMM-DEREGISTERED.PLMN-SEARCH. Cells are searched for at a low rhythm. No 5GS services are offered.

8.1.4.2.2.3.8	5GMM-DEREGISTERED.eCALL-INACTIVE

Editor's note:	How the UE enters 5GMM-DEREGISTERED.eCALL-INACTIVE sutstate is FFS.

Editor's note:	Other substates of state 5GMM-DEREGISTERED are FFS, e.g. 5GMM-DEREGISTERED.REGISTRATION-NEEDED.

###### 8.1.4.2.2.4	Substates of state 5GMM-REGISTERED

8.1.4.2.2.4.1	General

The state 5GMM-REGISTERED is subdivided into a number of substates as described in this subclause.

8.1.4.2.2.4.2	5GMM-REGISTERED.NORMAL-SERVICE

The substate 5GMM-REGISTERED.NORMAL-SERVICE is chosen by the UE as the primary substate when the UE enters the state 5GMM-REGISTERED and the cell the UE selected is known to be in an allowed area.

8.1.4.2.2.4.3	5GMM-REGISTERED.NON-ALLOWED-SERVICE

The substate 5GMM-REGISTERED.NON-ALLOWED-SERVICE is chosen in the UE, if the cell the UE selected is known to be in a non-allowed area.

8.1.4.2.2.4.4	5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE

The substate 5GMM-REGISTERED.ATTEMPTING-REGISTRATION-UPDATE is chosen by the UE if the mobility and periodic registration update procedure failed due to a missing response from the network. No 5GMM procedure except the mobility and periodic registration update procedure procedure shall be initiated by the UE in this substate. No data shall be sent or received.

Editor's note:	Other cases in which this substate is chosed are FFS.

8.1.4.2.2.4.5	5GMM-REGISTERED.LIMITED-SERVICE

The substate 5GMM-REGISTERED.LIMITED-SERVICE is chosen in the UE, if the cell the UE selected is known not to be able to provide normal service.

8.1.4.2.2.4.6	5GMM-REGISTERED.PLMN-SEARCH

The substate 5GMM-REGISTERED.PLMN-SEARCH is chosen in the UE, while the UE is searching for PLMNs.

8.1.4.2.2.4.7	5GMM-REGISTERED.NO-CELL-AVAILABLE

5G coverage has been lost or MICO mode is active in the UE. If MICO mode is active, the UE can deactivate MICO mode at any time by activating the AS layer when the UE needs to send mobile originated signalling or user data. Otherwise, The UE shall not initiate any 5GMM procedure except for cell and PLMN reselection.

Editor's note:	Other substates of state 5GMM-REGISTERED are FFS, e.g. 5GMM-REGISTERED.UPDATE-NEEDED.

##### 8.1.4.2.3	5GMM sublayer states in the network

###### 8.1.4.2.3.1	General

In the following subclauses, the possible 5GMM sublayer states of the network are described and shown in Figure 8.1.4.2.3.1.1.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GMM procedures are FFS.

# Figure 8.1.4.2.3.1.1: 5GMM main states in the network (3GPP access)

###### 8.1.4.2.3.2	5GMM-DEREGISTERED

In the state 5GMM-DEREGISTERED, no 5GMM context has been established or the 5GMM context is marked as deregistered. The UE is deregistered. The network may answer to an initial registration procedure initiated by the UE. The network may also answer to a deregistration procedure initiated by the UE.

###### 8.1.4.2.3.3	5GMM-COMMON-PROCEDURE-INITIATED

The network enters the state 5GMM-COMMON-PROCEDURE-INITIATED, after it has started a common 5GMM procedure and is waiting for a response from the UE.

###### 8.1.4.2.3.4	5GMM-REGISTERED

In the state 5GMM-REGISTERED, a 5GMM context has been established. Additionally, one or more PDU session context(s) may be activated at the network.

###### 8.1.4.2.3.5	5GMM-DEREGISTERED-INITIATED

The network enters the state 5GMM-DEREGISTERED-INITIATED after it has started a deregistration procedure and is waiting for a response from the UE.

#### 8.1.4.3	Behaviour of the UE in state 5GMM-DEREGISTERED and state 5GMM-REGISTERED

##### 8.1.4.3.1	General

In this subclause, the detailed behaviour of the UE in the states 5GMM-DEREGISTERED and 5GMM-REGISTERED is described.

##### 8.1.4.3.2	UE behaviour in state 5GMM-DEREGISTERED

###### 8.1.4.3.2.1	General

The state 5GMM-DEREGISTERED is entered in the UE, when:

-	the deregistration is performed either by the UE or by the network (see subclause 8.5.2.3);

-	the registration request is rejected by the AMF (see subclause 8.5.2.2);

-	the service request request is rejected by the MME (see subclause 8.5.3.1); or

-	the UE is switched on.

In state 5GMM-DEREGISTERED, the UE shall behave according to the substate as explained in subclause 8.1.4.3.2.3.

Editor's note:	Other cases in which the UE enters the state 5GMM-DEREGISTERED are FFS.

###### 8.1.4.3.2.2	Detailed description of UE behaviour in state 5GMM-DEREGISTERED

8.1.4.3.2.2.1	NORMAL-SERVICE

The UE shall initiate an initial registration procedure.

8.1.4.3.2.2.2	LIMITED-SERVICE

The UE shall initiate an initial registration procedure when entering a cell which provides normal service.

The UE may initiate initial registration for emergency services.

8.1.4.3.2.2.3	ATTEMPTING-REGISTRATION

Editor's note:	The UE behaviour in this substate is FFS.

8.1.4.3.2.2.4	PLMN-SEARCH

The UE shall perform PLMN selection. If a new PLMN is selected, the UE shall reset the registration attempt counter and initiate the initial registration procedure (see subclause 8.5.2.2.2).

If the selected cell is known not to be able to provide normal service, the UE may initiate registration for emergency services.

8.1.4.3.2.2.5	NO-SUPI

The UE shall perform cell selection.

The UE may initiate registration for emergency services.

8.1.4.3.2.2.6	NO-CELL-AVAILABLE

The UE shall perform cell selection and choose an appropriate substate when a cell is found.

8.1.4.3.2.2.7	eCALL-INACTIVE

Editor's note:	The UE behaviour in this substate is FFS.

Editor's note:	The other required UE behaviour in state 5GMM-DEREGISTERED is FFS, e.g. selection of the substate after power on, substate when back to state 5GMM-DEREGISTERED from another 5GMM state.

##### 8.1.4.3.3	UE behaviour in state 5GMM-REGISTERED

###### 8.1.4.3.3.1	General

The state 5GMM-REGISTERED is entered at the UE, when:

-	the initial registration procedure is performed by the UE (see subclause 8.5.2.2.2).

In state 5GMM-REGISTERED, the UE shall behave according to the substate as explained in subclause 8.1.4.3.3.2.

###### 8.1.4.3.3.2	Detailed description of UE behaviour in state 5GMM-REGISTERED

8.1.4.3.3.2.1	NORMAL-SERVICE

The UE:

-	shall initiate the mobility and periodic registration update procedure (according to conditions given in subclause 8.5.2.2.3);

-	shall initiate the service request procedure (according to conditions given in subclause 8.5.3.1); and

-	shall respond to paging.

8.1.4.3.3.2.2	NON-ALLOWED-SERVICE

The UE shall behave as specified in subclause 8.2.x.

8.1.4.3.3.2.3	ATTEMPTING-REGISTRATION-UPDATE

Editor's note:	The UE behaviour in this substate is FFS.

8.1.4.3.3.2.4	LIMITED-SERVICE

The UE:

-	shall perform cell selection/reselection; and

-	may initiate registration for emergency services.

8.1.4.3.3.2.5	PLMN-SEARCH

The UE shall perform PLMN selection. If a new PLMN is selected, the UE shall reset the registration attempt counter and initiate the initial registration procedure (see subclause 8.5.2.2.2).

If the selected cell is known not to be able to provide normal service, the UE may initiate registration for emergency services.

8.1.4.3.3.2.6	NO-CELL-AVAILABLE

The UE shall perform cell selection and choose an appropriate substate when a cell is found.

### 8.1.5	Permanent identifiers

A globally unique permanent identity, the 5G subscription permanent identifier (SUPI), is allocated to each subscriber for 5GS-based services. In the current release, the IMSI and the network access identifier (NAI) are valid SUPI types. Though non-IMSI based SUPIs are possible by using NAI, the IMSI can be contained within the NAI for the SUPI. The structure of the SUPI and its derivatives will be specified in 3GPP TS 23.003 [5].

A UE supporting NG-RAN includes a SUPI when a valid 5G-GUTI is not available from the PLMN or equivalent PLMN to which the UE is attempting to register or if requested by the network, during the registration and generic UE configuration update procedures.

Each UE supporting NG-RAN contains a permanent equipment identifier (PEI) for accessing 5GS-based services. In the current release, the IMEI and the IMEISV are the only PEI format supported by 5GS.The structure of the PEI and its formats will be specified in 3GPP TS 23.003 [5].

A UE supporting NG-RAN includes a PEI:

-	when no SUPI is available for emergency services in the REGISTRATION REQUEST message; and

Editor’s note:	Details of using the PEI for emergency services during the registration procedure are FFS.

-	when the network requests the PEI by using the identification procedure, in the IDENTIFICATION RESPONSE message.

The AMF can request the PEI at any time by using the identification procedure.

### 8.1.6	Temporary identities

A globally unique temporary user identity for 5GS-based services, the 5G globally unique temporary identity (5G-GUTI), is used for identification within the signalling procedures. The 5G-GUTI is common to both 3GPP and non-3GPP access. In the paging and service request procedures, a shortened form of the 5G-GUTI, the 5G S-temporary mobile subscriber identity (5G-S-TMSI), is used to enable more efficient radio signalling. The purpose of the 5G-GUTI and 5G-S-TMSI is to provide identity confidentiality, i.e., to protect a user from being identified and located by an intruder. The structure of the 5G-GUTI and its derivatives will be specified in 3GPP TS 23.003 [5]. The 5G-GUTI has two main components, the globally unique AMF id (GUAMI) that uniquely identifies the AMF that allocated the 5G-GUTI, and the 5G-S-TMSI that provides for an unambiguous identity of the UE within this AMF.

The AMF is responsible for allocating the 5G-GUTI to the UE. The allocation of the 5G-GUTI can be performed during registration and generic UE configuration update procedures. The AMF uses the S-TMSI for paging purposes.

A UE supporting NG-RAN includes a valid 5G-GUTI, if any is available, in the registration and deregistration request messages. In the service request message, the UE includes a valid 5G-S-TMSI as user identity. The AMF may assign a new 5G-GUTI for a particular UE at successful registration and generic UE configuration update procedures.

If a new 5G-GUTI is assigned by the AMF, the UE and the AMF handle the 5G-GUTI as follows:

-	Upon receipt of a 5GMM message containing a new 5G-GUTI the UE considers the new 5G-GUTI as valid and the old 5G-GUTI as invalid.

-	The AMF considers the old 5G-GUTI as invalid as soon as an acknowledgement for a registration or generic UE configuration update procedure is received.

### 8.1.7	Mobile initiated connection only mode

The UE can request the use of mobile initiated connection only (MICO) mode during the registration procedure (see 3GPP TS 23.501 [8] and 3GPP TS 23.502[9]).

The network accepts the use of MICO mode by providing a MICO indication when accepting the registration procedure. The UE may use MICO mode only if the network has provided the MICO indication IE during the last registration procedure.

If the network accepts the use of MICO mode, the UE may deactivate the AS layer and activate MICO mode by entering the state 5GMM-REGISTERED.NO-CELL-AVAILABLE if:

a)	the UE is in 5GMM-IDLE mode; and

b)	in the 5GMM-REGISTERED.NORMAL-SERVICE state.

When MICO mode is activated all NAS timers are stopped and associated procedures aborted except for periodic registration update timer, any backoff timers, and the timer T controlling the periodic search for HPLMN or EHPLMN or higher prioritized PLMNs (see 3GPP TS 23.122 [7]).

The UE may deactivate MICO mode and activate the AS layer at any time. Upon deactivating MICO mode, the UE may initiate 5GMM procedures (e.g. for the transfer of mobile originated signalling or user data).

If the AMF accepts the use of MICO mode, the AMF starts the implicit deregistration timer when entering 5GMM-IDLE mode.

### 8.1.8	Handling of the periodic registration update timer and implicit deregistration timer

The registration procedure is used to periodically notify the availability of the UE to the network. The procedure is controlled in the UE by the periodic registration update timer, Te.

Upon expiry of the implicit deregistration timer the network shall start the implicit detach timer. The value of the implicit detach timer is network dependent. If MICO is activated, the default value of the implicit detach timer is 4 minutes greater than timer Te. If the implicit detach timer expires before the UE contacts the network, the network shall implicitly detach the UE.

Editor's note:	Further details on timer handling are FFS (e.g. if Te expires while the UE is registered for emergency bearer services).

## 8.2	5GS mobility functions in IDLE and CONNECTED mode

### 8.2.1	Registration areas in the 5GS

#### 8.2.1.1	General

Within the 5GS, the registration area is managed independently per access type, i.e., 3GPP access or non-3GPP access. The AMF assigns a registration area to the UE during the registration procedure. A registration area is defined as a set of tracking areas and each of these tracking areas consists of one or more cells that cover a geographical area. Tracking areas cannot overlap each other. Within the 5GS, the concept of "registration to multiple tracking areas" applies:

-	A tracking area is identified by a TAI which is broadcast in the cells of the tracking area. The TAI is constructed from a TAC and a PLMN identifier. In case of a shared network, a single TAC and multiple PLMN identifiers are broadcast.

-	In order to reduce the tracking area update signalling within the 5GS, the AMF can assign several tracking areas to the UE. These tracking areas construct a list of tracking areas which is identified by a TAI list. When generating the TAI list, the AMF shall include only TAIs that are applicable on the access where the TAI list is sent. The AMF shall be able to allocate a TAI List over different NG-RAN access technologies.

-	The UE considers itself registered to a list of tracking areas and does not need to trigger mobility registration update procedure (i.e. registration procedure with registration type set to "mobility registration update") as long as the UE stays in one of the tracking areas of the list of tracking areas received from the AMF.

Editor's note: 	The maximum number of tracking areas which can be allocated per UE needs to be defined which can be as in EPS or different.

-	The UE will consider the TAI list as valid, until it receives a new TAI list in the next mobility registration update or periodic registration update procedure, or the UE  is commanded by the network to delete the TAI list by a reject message or it is detached from the 5GS. If the registration request is accepted or the TAI list is reallocated by the AMF, the AMF shall provide at least one entry in the TAI list. If the new and the old TAI list are identical, the AMF does not need to provide the new TAI list to the UE during mobility registration update or periodic registration update.

-	The TAI list can be reallocated by the AMF.

-	When the UE is detached from the 5GS, the TAI list in the UE is invalid.

-	The AMF allocates one globally unique temporary identity, which is common between 3GPP and non3GPP, to the UE.

Editor's note:	The details about the handling of the temporary identity are FFS.

#### 8.2.1.2	Open issues on registration areas in the 5GS

-	CT1 need to study whether the periodic registration update timer for the periodic registration updating procedure and the periodic tracking area updating procedure in EPS can have different values, related values, or the same value.

-	CT1 need to study the impacts on the periodic registration update timer when the mobile initiated connection only (MICO) mode (see 3GPP TS 23.501 [8]) is used.

### 8.2.2	UE NAS mobility functions in 5GMM-IDLE mode

In 5GMM-IDLE mode a UE with a valid USIM enabled will:

-	perform cell selection/reselection procedure according to 3GPP TS 38.304 [18] and PLMN selection according to 3GPP TS 23.122 [7];

-	when not registered to the 5GCN:

-	perform registration procedure with registration type set to "initial registration" in order to receive services that require registration in the 5GS when not registered to the EPS;

-	perform registration procedure with registration type set to "mobility registration update" when registered to the EPS;

-	when registered to the 5GCN:

-	perform registration procedure with registration type set to "mobility registration update" when:

-	entering a tracking area not in the list of assigned tracking areas, in order to maintain the registration and enable the AMF to keep track of the UE; or

-	the UE needs to update its capabilities towards the 5GCN;

-	perform registration procedure with registration type set to "periodic registration update" to periodically notify the 5GS that the UE is available;

-	answer to paging from the AMF by performing a service request procedure, unless the UE is in MICO mode or mobility restrictions prevent the UE from responding to paging;

-	perform the service request procedure in order to establish a N1 NAS signalling connection to the network or to request the establishment of radio bearers when uplink user data is to be sent, or both;

-	perform de-registration procedure when the UE is switched off, the USIM is removed or disabled, or the 5GS capability of the UE is disabled.

Editor's note: 	Whether the de-registration procedure is associated with an indication of whether it applies only to the access on which the de-registration procedure is run or to all accesses of the UE (3GPP and non-3GPP access) is FFS.

In 5GMM-IDLE mode a UE without valid USIM enabled will:

-	perform cell selection/reselection according to 3GPP TS 38.304 [18], but not PLMN selection.

Editor's note: 	How the UE without a valid USIM registers to the 5GCN for unauthenticated emergency services is FFS.

### 8.2.3	UE NAS mobility functions in 5GMM-CONNECTED mode

In 5GMM-CONNECTED mode a UE with a valid USIM enabled will:

-	respond to identification requests from the AMF;

-	perform authentication when requested by the AMF;

-	initiate the ciphering and integrity protection for NAS signalling when requested by the AMF;

-	not perform periodic registration update procedure (i.e. registration procedure with registration type set to "periodic registration update");

Editor's note: 	The need of a procedure to reallocate a new temporary identity to the UE is FFS.

-	when attached to the 5GCN:

-	perform mobility registration update procedure (i.e. registration procedure with registration type set to "mobility registration update") when after handover the UE detects that it has entered a tracking area not in the assigned list of tracking areas;

-	perform the service request procedure in order to request the establishment of the radio bearers when uplink user data is to be sent;

-	perform de-registration procedure when the UE is switched off, the USIM is removed or disabled, the 5GS capability of the UE is disabled, or when requested by the AMF.

Editor's note: 	The case of the UE without a valid USIM registered to the 5GCN and using emergency services is FFS.

### 8.2.4	5GMM-CONNECTED mode with RRC inactive indication

The UE is in 5GMM-CONNECTED mode with RRC inactive indication when the UE is in:

-	5GMM-CONNECTED mode at the NAS layer; and

-	RRC_INACTIVE state at the AS layer (see 3GPP TS 38.300 [17]).

Unless stated otherwise, the UE behavior in 5GMM-CONNECTED mode with RRC inactive indication follows the UE behavior in 5GMM-CONNECTED, except that:

-	the UE shall apply the mobility restrictions; and

-	the UE shall perfrom the PLMN selection procedures

as in 5GMM-IDLE mode.

The UE transitions from 5GMM-CONNECTED mode to 5GMM-CONNECTED mode with RRC inactive indicatoin upon indication from the lower layers that the UE has transitioned to RRC_INACTIVE state.

The UE triggers a transition from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-CONNECTED mode upon:

-	Uplink data transfer; or

-	Mobile originaiting signaling.

The UE transitions from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-CONNECTED mode upon receving from the lower layers indication that the UE has transitioned to RRC_CONNECTED state (see 3GPP TS 38.300 [17]).

The UE transitions from 5GMM-CONNECTED mode with RRC inactive indication to 5GMM-IDLE mode upon receving from the lower layers:

-	indication that the transition from RRC_INACTIVE to RRC_CONNECTED has failed; or

-	AMF paging notification.

Editor’s note:	Other triggers for transition to 5GMM_IDLE are FFS.

The AMF does not maintain the 5GMM-CONNECTED mode with RRC inactive indication.

Editor’s note:	Whether the AMF is aware of the UE transitioning between 5GMM-CONNECTED mode with RRC inactive indication and 5GMM-CONNECTED mode or not is FFS.

## 8.3	Establishment of the NAS signalling connection

When the UE is in 5GMM-IDLE mode and needs to transmit an initial NAS message, the UE shall request the lower layer to establish a RRC connection.

Initial NAS messages are:

-	REGISTRATION REQUEST;

-	DEREGISTRATION REQUEST; and

-	SERVICE REQUEST.

For the routing of the initial NAS message to the appropriate AMF, if the UE holds a valid 5G-GUTI, the UE NAS provides the lower layers with either the 5G-S-TMSI or the registered globally unique AMF identifier (GUAMI) that consists of the PLMN ID, the AMF region ID, the AMF set ID and the AMF pointer according to the following rules:

a)	When the tracking area of the current cell is in the list of tracking areas that the UE previously registered in the AMF during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the 5G-S-TMSI, but shall not provide the registered GUAMI to the lower layers;

b)	When the tracking area of the current cell is not in the list of tracking areas that the UE previously registered in the AMF during the NAS signalling connection establishment, the UE NAS shall provide the lower layers with the GUAMI (AMF identifier part of the valid 5G-GUTI);

c)	When the REGISTRATION REQUEST with the registration type set to "initial registration" is sent over 3GPP access in the same PLMN or equivalent PLMN to establish the NAS signalling connection, the UE NAS shall provide the lower layers with the GUAMI (AMF identifier part of the valid 5G-GUTI which is assigned over the non-3GPP access); or

d)	When the REGISTRATION REQUEST with the registration type set to "initial registration" is sent over non-3GPP access in the same PLMN or equivalent PLMN to establish the NAS signalling connection, the UE NAS shall provide the lower layers with the GUAMI (AMF identifier part of the valid 5G-GUTI which is assigned over the 3GPP access).

## 8.4	IP address allocation

### 8.4.1	General

This clause specifies IP address allocation for the PDU session.

In this release of specification, PDU sessions for PDU type IP shall contain only one IP version, i.e. IPv4 PDU type or IPv6 PDU type.

IP address allocation to the UE shall be performed by SMF based on the selected IP version and operator policies. If IPv4 PDU type is selected, an IPv4 address is allocated to the UE. If IPv6 PDU type is selected, an IPv6 prefix is allocated to the UE.

For IPv4 PDU type, the UE can obtain an IPv4 address and IPv4 configuration parameter (e.g. DNS server address)

-	via NAS signalling as specified in subclause 8.4.2; or

-	using DHCPv4.

For IPv6 PDU type, the UE can obtain /64 IPv6 prefix via IPv6 stateless address autoconfiguration as specified in IETF RFC 4862 [23] and IPv6 configuration parameters via stateless DHCPv6 as specified in IETF RFC 3736 [20].

If the UE supports requesting an "IP" PDU type, an "IPv6" PDU type or both, the UE shall support acting as a type C host as specified in IETF RFC 4191 [22].

### 8.4.2	IP address allocation via NAS signalling

The UE shall set the PDU session type IE in the PDU SESSION ESTABLISHMENT REQUEST message, based on its IP stack capabilities and configuration if the UE requests IP connectivity as follows:

a)	A UE, which is configured for the DNN:

-	to support IPv6 and IPv4 and which is IPv6 and IPv4 capable, shall set the PDU session type IE to IP.

-	to support only IPv6 and which is IPv6 and IPv4 capable or only IPv6 capable, shall set the PDU session type IE to IPv6.

-	to support only IPv4 and which is IPv6 and IPv4 capable or only IPv4 capable, shall set the PDN type IE to IPv4.

b)	A UE, which is not configured for the DNN and:

-	which is IPv6 and IPv4 capable, shall set the PDU session type IE to IP.

-	which is only IPv4 capable, shall set the PDU session type IE to IPv4.

-	which is only IPv6 capable, shall set the PDU session type IE to IPv6.

c)	When the IP version capability of the UE is unknown in the UE (as in the case when the MT and TE are separated and the capability of the TE is not known in the MT), the UE shall set the PDU session type IE to IP.

If the UE wants to use DHCPv4 for IPv4 address assignment, it shall indicate that to the network within the extended protocol configuration options IE in the PDU SESSION ESTABLISHMENT REQUEST.

On receipt of the PDU SESSION ESTABLISHMENT REQUEST message sent by the UE, the network when allocating an IP address shall take into account the PDU session type IE, the operator's policies of the network, and the user's subscription data and:

-	if the UE requests the PDU session type IP, but the subscription or SMF configuration is limited to IPv4 only or IPv6 only for the requested DNN, the network shall set the PDU session type IE to either "IPv4" or "IPv6" and the 5GSM cause value to #50 "PDU session type IPv4 only allowed", or #51 "PDU session type IPv6 only allowed" in the PDU SESSION ESTABLISHMENT ACCEPT message, respectively. The UE shall not subsequently initiate another UE requested PDU session establishment procedure to the same DNN to obtain a PDU session type different from the one allowed by the network;

-	if the network sets the PDU session type IE to IPv4, the network shall include an IPv4 address in the PDU address IE; and

-	if the network sets the PDU session type IE to IPv6, the network shall include an IPv6 prefix in the PDU address IE.

## 8.5	5GS mobility management procedures

### 8.5.1	5GMM common procedures

#### 8.5.1.1	NAS transport procedures

##### 8.5.1.1.1	General

The purpose of the NAS transport procedures is to provide a transport of payload between the UE to the AMF. Two alternatives are described: subclause 8.5.1.1.2 describes Alternative 1 where there is a dedicated set of procedures for each of SM transport and non-SM transport; and subclause 8.5.1.1.3 describes Alternative 2 where there is a single set of procedures for both SM and non-SM transport.

Editor's note:	It is FFS whether to extend the UL SM MESSAGE TRANSPORT message and the UL GENERIC TRANSPORT message (for alternative 1 for NAS transport) and the UL NAS TRANSPORT message (for alternative 2 for NAS transport) to enable their usage as initial 5GMM message.

##### 8.5.1.1.2	Alternative 1 for NAS transport

###### 8.5.1.1.2.1	SM transport procedures

8.5.1.1.2.1.1	UE-initiated SM message transport procedure

8.5.1.1.2.1.1.1	General

The purpose of the UE-initiated SM transport message procedure is to provide transport of:

-	an SM message; and

-	associated routing information (the PDU session ID, the S-NSSAI, the DNN, the request type);

from the UE to the AMF in the 5GMM message.

5GSM procedures specified in clause 9 describe conditions for inclusion of the S-NSSAI, and the DNN.

The request type is not provided along SM messages other than the PDU SESSION ESTABLISHMENT REQUEST message.

8.5.1.1.2.1.1.2	UE-initiated SM message transport initiation

The UE shall set the SM message container IE of the UL SM MESSAGE TRANSPORT message to the SM message.

The UE shall set the PDU session ID IE of the UL SM MESSAGE TRANSPORT message to the PDU session ID.

If an S-NSSAI is to be included, the UE shall set the S-NSSAI IE of the UL SM MESSAGE TRANSPORT message to the S-NSSAI.

If a DNN is to be included, the UE shall set the DNN IE of the UL SM MESSAGE TRANSPORT message to the DNN.

If a request type is to be included, the UE shall set the request type IE of the UL SM MESSAGE TRANSPORT message to the request type.

The UE shall send the UL SM MESSAGE TRANSPORT message to the AMF (see example in figure 8.5.1.1.2.1.1.2.1).

# Figure 8.5.1.1.2.1.1.2.1: UE-initiated SM transport procedure

8.5.1.1.2.1.1.3	UE-initiated SM message transport of messages

Upon reception of an UL SM MESSAGE TRANSPORT message, the AMF looks up a PDU session routing context for the PDU session ID of the UL SM MESSAGE TRANSPORT message and the UE and:

a)	if the AMF has a PDU session routing context for the PDU session ID and the UE, and the request type IE is not included in the UL SM MESSAGE TRANSPORT message, the AMF shall forward the SM message, and the PDU session ID of the UL S SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context;

b)	if the AMF has a PDU session routing context for the PDU session ID and the UE, the PDU session routing context indicates that the PDU session is not an emergency PDU session, and the request type IE is included in the UL S SM MESSAGE TRANSPORT message and is set to "existing PDU session", the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context;

c)	if the AMF has a PDU session routing context for the PDU session ID and the UE, the request type IE is included in the UL SM MESSAGE TRANSPORT message and is set to "initial request" and the AMF received a reallocation requested indication from the SMF indicating that the SMF is to be reused, the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context;

d)	if the AMF has a PDU session routing context for the PDU session ID and the UE, the request type IE is included in the UL SM MESSAGE TRANSPORT message and is set to "initial request", the AMF received a reallocation requested indication from the SMF indicating that the SMF is to be reallocated and the PDU session routing context does not contain reallocated SMF ID:

1)	the AMF shall select an SMF. If the DNN is not included in the UL SM MESSAGE TRANSPORT message, the AMF shall use the default DNN as the DNN. If the S-NSSAI is not included in the UL SM MESSAGE TRANSPORT message, the AMF may determine a default S-NSSAI according to the user's subscription context obtained from UDM and, if determined, the AMF may use the default S-NSSAI as the S-NSSAI; and

NOTE:	SMF selection is out of scope of CT1.

2)	if the SMF selection is successful:

A)	the AMF shall set the reallocated SMF ID of the PDU session routing context to the SMF ID of the selected SMF; and

B)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the reallocated SMF ID of the PDU session routing context; and

e)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE,  and the request type IE is included in the UL SM MESSAGE TRANSPORT message and is set to "initial request":

1)	the AMF shall select an SMF. If the DNN is not included in the UL SM MESSAGE TRANSPORT message, the AMF shall use the default DNN as the DNN. If the S-NSSAI is not included in the UL SM MESSAGE TRANSPORT message, the AMF may determine a default S-NSSAI according to the subscription context from UDM and, if determined, the AMF may use the default S-NSSAI as the S-NSSAI; and

NOTE:	SMF selection is out of scope of CT1.

2)	if the SMF selection is successful:

A)	the AMF shall store a PDU session routing context for the PDU session ID and the UE and shall set the SMF ID in the stored PDU session routing context to the SMF ID of the selected SMF; and

B)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context; and

f)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the request type IE of the UL SM MESSAGE TRANSPORT message is included and is set to "existing PDU session", and the user’s subscription context obtained from the UDM contains an SMF ID corresponding to:

1)	the DNN of the UL SM MESSAGE TRANSPORT message, if the DNN is included in the NAS SM MESSAGE TRANSPORT message; or

2)	the default DNN, if the DNN is not included in the UL SM MESSAGE TRANSPORT message;

Editor's note:	FFS how to obtain the correct SMF ID when the UE has several PDU sessions with a given DNN, each PDU session is provided by a different SMF, and the UE attempts to handover one of those PDU sessions.

then:

1)	the AMF shall store a PDU session routing context for the PDU session ID and the UE, shall set the SMF ID in the stored PDU session routing context to the SMF ID corresponding to the DNN in the user’s subscription context obtained from the UDM; and

2)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of with the PDU session routing context;

f)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the request type IE is included in the UL SM MESSAGE TRANSPORT message and is set to "initial emergency request", and the AMF does not have a PDU session routing context for another PDU session ID of the UE indicating that the PDU session is an emergency PDU session:

1)	the AMF shall select an SMF. The AMF shall use the emergency DNN from the AMF emergency configuration data as the DNN, if configured. The AMF shall use the statically configured SMF from the AMF emergency configuration data, if configured; and

NOTE:	SMF selection is out of scope of CT1.

2)	if the SMF selection is successful:

A)	the AMF shall store a PDU session routing context for the PDU session ID and the UE and shall set the SMF ID in the stored PDU session routing context to the SMF ID of the selected SMF and shall store an indication that the PDU session is an emergency PDU session in the stored PDU session routing context; and

B)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context; and

g)	if the AMF does not have a PDU session routing context for the PDU session ID and the UE, the request type IE is included in the UL SM MESSAGE TRANSPORT message and is set to "initial emergency request", and the AMF has a PDU session routing context indicating that the PDU session is an emergency PDU session for another PDU session ID of the UE:

1)	the AMF shall store a PDU session routing context for the PDU session ID and the UE and shall set the SMF ID in the stored PDU session routing context to the SMF ID of the PDU session routing context for the other PDU session ID of the UE; and

2)	the AMF shall forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context.

Editor's note:	Handover between 3GPP access and non-3GPP access of emergency PDU session is FFS.

8.5.1.1.2.1.1.4	Abnormal cases on the network side

The following abnormal cases in AMF are identified:

a)	the AMF does not have a PDU session routing context for the PDU session ID of the UL SM MESSAGE TRANSPORT message and the UE, the request type IE of the UL SM MESSAGE TRANSPORT message is set to "initial request", and the SMF selection fails.

Editor's note:	Handling of this abnormal case is FFS

b)	the AMF does not have a PDU session routing context for the PDU session ID of the UL SM MESSAGE TRANSPORT message and the UE, the request type IE of the UL SM MESSAGE TRANSPORT message is set to "existing PDU session", and the user’s subscription context obtained from the UDM does not contain an SMF ID corresponding to.

1)	the DNN of the UL SM MESSAGE TRANSPORT message, if the DNN is included in the NAS SM MESSAGE TRANSPORT message; or

2)	the default DNN, if the DNN is not included in the UL SM MESSAGE TRANSPORT message.

Editor's note:	Handling of this abnormal case is FFS

c)	if the AMF has a PDU session routing context for the PDU session ID of the UL SM MESSAGE TRANSPORT message and the UE, the request type IE of the UL SM MESSAGE TRANSPORT message is set to "initial request" and the AMF has not received a reallocation requested indication, the AMF should forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context.

d)	if the AMF has a PDU session routing context for the PDU session ID of the UL SM MESSAGE TRANSPORT message and the UE, the PDU session routing context indicates that the PDU session is an emergency PDU session, the request type IE of the UL SM MESSAGE TRANSPORT message is set to "initial emergency request", the AMF should forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the SMF ID of the PDU session routing context.

e)	if the AMF has a PDU session routing context for the PDU session ID of the UL SM MESSAGE TRANSPORT message and the UE, the request type IE of the UL SM MESSAGE TRANSPORT message is set to "initial request", the AMF has received a reallocation requested indication from the SMF indicating that the SMF is to be reallocated, and the PDU session routing context contains reallocated SMF ID, the AMF should forward the SM message, the PDU session ID, the S-NSSAI (if received), the DNN (if received) and the request type of the UL SM MESSAGE TRANSPORT message towards the reallocated SMF ID of the PDU session routing context.

8.5.1.1.2.1.3	Network-initiated SM message transport procedure

8.5.1.1.2.1.3.1	General

The purpose of the network-initiated SM message transport procedure is to provide transport of an SM message and a PDU session ID from the AMF to the UE in a 5GMM message.

8.5.1.1.2.1.3.2	Network-initiated SM message transport initiation

Upon:

a)	reception of an SM message without an N1 SM delivery skip allowed indication for a UE served from an SMF; or

b)	reception of an SM message, and an N1 SM delivery skip allowed indication for a served UE from an SMF and if the UE is in the 5GMM-CONNECTED state;

the AMF shall create a DL SM MESSAGE TRANSPORT message.

The AMF shall set the SM message container IE of the DL SM MESSAGE TRANSPORT message to the SM message from the SMF.

The AMF shall set the PDU session ID IE of the DL SM MESSAGE TRANSPORT message to the PDU session ID.

Editor's note:	FFS how AMF determines the PDU session ID - (a) the AMF can receive the PDU session ID along the SM message in the N11 message from the SMF, or (b) if there is one N11 connection for each PDU session then the AMF can locally derive the PDU session ID from identifier of the N11 connection via which the N11 message is received.

The AMF shall send the DL SM MESSAGE TRANSPORT message to the UE (see example in figure 8.5.1.1.2.1.3.2.1).

# Figure 8.5.1.1.2.1.3.2.1: UE-initiated SM transport procedure

8.5.1.1.2.1.3.3	Network-initiated SM message transport of messages

The SM message and the PDU session ID of a received DL SM MESSAGE TRANSPORT message is handled in the 5GSM procedures specified in clause 9.

###### 8.5.1.1.2.2	Non-SM message transport procedures

8.5.1.1.2.2.1	General

The purpose of the procedures is to provide a transport of non-SM messages between the UE and the AMF. The type of the non-SM message is identified by the message container type IE and corresponds to one of the followings:

-	SMS; or

-	LTE Positioning Protocol (LPP) message (see 3GPP TS 36.355 [16]).

Editor's note:	Other types, such as location-services, are FFS.

8.5.1.1.2.2.2	UE-initiated non-SM message transport procedure

8.5.1.1.2.2.2.1	General

The purpose of the UE-initiated non-SM message transport procedure is to provide a transport of:

-	SMS; or

-	an LPP message

from the UE to the AMF in a 5GMM message.

8.5.1.1.2.2.2.2	UE-initiated non-SM message transport initiation

The UE shall set the message container IE of the UL GENERIC TRANSPORT message to the non-SM message.

The UE shall set the message container type IE of the UL GENERIC TRANSPORT message to the type of the non-SM message.

Editor's note:	How the UE selects the access type to deliver the UL GENERIC TRANSPORT message with the message container type IE set to "SMS" is FFS.

If the message container type IE of the UL GENERIC TRANSPORT message is set to "LTE Positioning Protocol (LPP) message container", the UE shall set the Additional information IE of the UL GENERIC TRANSPORT message to the routing information provided by the upper layer location services application.

Editor's note: The encoding of the Additional information IE in the UL GENERIC TRANSPORT message is FFS.

The UE shall send the UL GENERIC TRANSPORT message to the AMF (see example in figure 8.5.1.1.2.2.2.2.1).

Figure 8.5.1.1.2.2.2.2.1: UE-initiated non-SM message transport procedure

8.5.1.1.2.2.2.3	UE-initiated non-SM message transport of messages

Upon reception of an UL GENERIC TRANSPORT message with the message container type IE set to "SMS", the AMF shall forward the SMS message of the UL GENERIC TRANSPORT message towards the SMSF associated with the UE.

Upon reception of an UL GENERIC TRANSPORT message with the message container type IE set to "LTE Positioning Protocol (LPP) message container":

a)	the AMF shall forward the LPP message of the UL GENERIC TRANSPORT message to the Location Management Function (LMF) associated with the routing information included in the Additional information IE of the UL GENERIC TRANSPORT message:

8.5.1.1.2.2.3	Network-initiated non-SM message transport procedure

8.5.1.1.2.2.3.1	General

The purpose of the network-initiated non-SM message transport procedure is to provide a transport of:

-	SMS; or

-	an LPP message

from the AMF to the UE in a 5GMM message.

8.5.1.1.2.2.3.2	Network-initiated non-SM message transport initiation

Upon reception of a non-SM message for a UE from the other network function (e.g. SMSF), the AMF shall create a DL GENERIC TRANSPORT message.

The AMF shall set the message container IE of the DL GENERIC TRANSPORT message to the non-SM message from the other network function.

The AMF shall set the message container type IE of the DL GENERIC TRANSPORT message to the type of the non-SM message.

If the message container type IE of the DL GENERIC TRANSPORT message is set to "LTE Positioning Protocol (LPP) message container", the AMF shall set the Additional information IE of the UL GENERIC TRANSPORT message to the routing information associated with the LMF from which the LPP message was received.

Editor's note:	The encoding of the Additional information IE in the DL GENERIC TRANSPORT message is FFS.

The AMF shall select the access network (either 3GPP or non-3GPP) through which the DL GENERIC TRANSPORT message is sent. If the AMF sets the message container type IE to "SMS" and the access type selection is required, the AMF shall select the acces type to deliver the DL GENERIC TRANSPORT message as follows:

-	if the UE to receive the DL GENERIC TRANSPORT message is registered to the network via both 3GPP access and non-3GPP access, and the SMS allowed IE in the 5GMM context of the UE is set to "both 3GPP access and non-3GPP access", then the AMF selects either 3GPP access or non-3GPP access; and

-	otherwise, the AMF selects 3GPP access.

NOTE:	The AMF selects an access type between 3GPP access and non-3GPP access based on operator policy.

Editor's note:	The cases when the access type selection is required for SMS deliver are FFS.

The AMF shall send the DL GENERIC TRANSPORT message to the UE (see example in figure 8.5.1.1.2.2.3.2.1).

Figure 8.5.1.1.2.2.3.2.1: Network-initiated non-SM message transport procedure

8.5.1.1.2.2.3.3	Network-initiated non-SM message transport of messages

Upon reception of a DL GENERIC TRANSPORT message, if the message container type IE is set to:

a)	"SMS", the UE shall forward the content of the message container IE to the SMS entity; and

b)	"LTE Positioning Protocol (LPP) message container", the UE shall forward the content of the message container IE and of the Additional information IE to the upper layer location services application.

##### 8.5.1.1.3	Alternative 2 for NAS Transport

###### 8.5.1.1.3.1	General

The purpose of the NAS transport procedures is to provide a transport of payload between the UE to the AMF. The type of the payload is identified by the Payload type information element and includes one or more of the following:

-	a single 5GSM message;

-	SMS; or

-	an LPP message.

Editor’s note:	Other types of payload, such as generic application payload, location-services related payload, are FFS.

Along with the payload, the NAS transport procedure may transport the associated payload information (e.g. PDU session information for 5GSM message payload etc.).

Editor's note:	It is FFS whether the DL NAS TRANSPORT and UL NAS TRANSPORT could be initial 5GMM messages.

###### 8.5.1.1.3.2	UE-initiated NAS transport procedure

8.5.1.1.3.2.1	General

The purpose of the UE-initiated NAS transport procedure is to provide a transport of:

a)	a single 5GSM message;

b)	SMS; or

c)	an LPP message

and optional associated payload routing information from the UE to the AMF in a 5GMM message.

NOTE:	For case a) the procedures specified below may be supplemented by the related aspects of the procedures in subclause 8.5.1.1.2.1.3.2.1 during the normative phase.

8.5.1.1.3.2.2	UE-initiated NAS transport procedure initiation

In the connected mode, the UE initiates the NAS transport procedure by sending the UL NAS TRANSPORT message, as shown in figure 8.5.1.1.3.2.2.1.

In case a) in subclause 8.5.1.1.3.2.1, the UE shall:

-	include the PDU session information (PDU session ID, S-NSSAI, DNN, Request type) in the Payload information IE.

-	set the Payload type IE to "N1 SM information"; and

-	set the Payload container IE to the 5GSM message.

5GSM procedures specified in clause 8 describe conditions for inclusion of the S-NSSAI, and the DNN.

In case b) in subclause 8.5.1.1.3.2.1, the UE shall:

-	set the Payload type IE to "SMS";

-	set the Payload container IE to the SMS payload; and

Editor’s note: Whether Payload information IE is needed for SMS payload or not is FFS.

Editor's note:	How the UE selects the access type to deliver the UL NAS TRANSPORT message with the Payload type IE set to "SMS" is FFS.

In case c) in subclause 8.5.1.1.3.2.1, the UE shall:

-	set the Payload type IE to "LTE Positioning Protocol (LPP) message container";

-	set the Payload container IE to the LPP message payload; and

-	set the Additional information IE to the routing information provided by the upper layer location services application.

Editor’s note:	The encoding of the Additional Information IE in the UL NAS TRANSPORT message is FFS.

# Figure 8.5.1.1.3.2.2.1: UE-initiated NAS transport procedure

8.5.1.1.3.2.3	UE-initiated NAS transport of messages

Upon reception of an UL NAS TRANSPORT message, if the Payload type IE is set to:

a)	"N1 SM information":

i)	if the AMF needs to select an SMF:

Editor's note:	FFS under which condition the AMF needs to select an SMF.

1)	the AMF shall select an SMF. If the DNN is not included in Payload information IE, the AMF shall use the default DNN as the DNN; and

Editor's note:	SMF selection is to be specified by CT4.

2)	if the SMF selection is successful, the AMF shall associate the selected SMF with the PDU session ID; and

Editor's note:	It is FFS what to do if the SMF address selection fails

ii)	the AMF shall forward the PDU session ID, the 5GSM message in the Payload container IE, the S-NSSAI (if received), and the DNN (if received) of the UL NAS TRANSPORT message towards the SMF associated with the PDU session ID:

b)	"SMS", the AMF shall forward the content of the Payload container IE to the SMSF; and

c)	"LTE Positioning Protocol (LPP) message container", the AMF shall forward the content of the Payload container IE to the LMF associated with the routing information included in the Additional information IE of the UL NAS TRANSPORT message.

Editor's note:	What to do if the AMF cannot locate the LMF is FFS.

###### 8.5.1.1.3.3	Network-initiated NAS transport procedure

8.5.1.1.3.3.1	General

The purpose of the network-initiated NAS transport procedure is to provide a transport of:

a)	a single 5GSM message;

b)	SMS; or

c)	an LPP message

from the AMF to the UE in a 5GMM message.

NOTE:	For case a) the procedures specified below may be supplemented by the related aspects of the procedures in subclause 8.5.1.1.3.1 during the normative phase.

###### 8.5.1.1.3.3.2	Network-initiated NAS transport procedure initiation

In connected mode, the AMF initiaties the NAS transport procedure by sending the DL NAS TRANSPORT message, as shown in figure 8.5.1.1.3.3.2.1.

In case a) in subclause 8.5.1.1.3.3.1, the AMF shall:

-	include the PDU session information (PDU session ID) in the Payload information IE;

-	set the Payload type IE to "N1 SM information"; and

-	set the Payload container IE to the 5GSM message.

In case b) in subclause 8.5.1.1.3.3.1, the AMF shall:

-	set the Payload type IE to "SMS";

-	set the Payload container IE to the SMS payload ; and

-	select the access type to deliver the DL NAS TRANSPORT message as follows in case the access type selection is required:

i)	if the UE to receive the DL NAS TRANSPORT message is registered to the network via both 3GPP access and non-3GPP access, and the SMS allowed IE in the 5GMM context of the UE is set to "both 3GPP access and non-3GPP access", then the AMF selects either 3GPP access or non-3GPP access; and

ii)	otherwise, the AMF selects 3GPP access.

NOTE:	The AMF selects an access type between 3GPP access and non-3GPP access based on operator policy.

Editor's note:	The cases when the access type selection is required for SMS deliver are FFS.

In case c) in subclause 8.5.1.1.3.3.1, the AMF shall:

-	set the Payload type IE to "LTE Positioning Protocol (LPP) message container";

-	set the Payload container IE to the LPP message payload received from the LMF; and

-	set the Additional information IE to the routing information associated with the LMF from which the LPP message was received.

Editor’s note: 	The encoding of the Additional Information IE in the DL NAS TRANSPORT message is FFS.

Figure 8.5.1.1.3.3.2.1: Network-initiated NAS transport procedure

###### 8.5.1.1.3.3.3	Network-initiated NAS transport of messages

Upon reception of a DL NAS TRANSPORT message, if the Payload type IE is set to:

a)	 "N1 SM information".the 5GSM message in the Payload container IE is handled in the 5GSM procedures specified in clause 8:

b)	"SMS", the UE shall forward the content of the Payload container IE to the SMS stack entity ; and

c)	"LTE Positioning Protocol (LPP) message container", the UE shall forward the content of the Payload container IE and the routing information included in the Additional information IE to the upper layer location services application.

#### 8.5.1.2	Generic UE configuration update procedure

##### 8.5.1.2.1	General

The purpose of this procedure is to allow the AMF to update UE configuration. The procedure may be initiated by the network and can only be used when the UE has an established 5GMM context, and the UE is in 5GMM-CONNECTED mode. The AMF may require a confirmation response in order to ensure that the parameter has been updated by the UE.

The following parameters are supported by the Generic UE configuration update procedure:

a)	5G-GUTI;

b)	TAI list;

c)	Mobility restrictions; and

d)	Network Identity and Time Zone information (Full name for network, Short name for network, Local time zone, Universal time and local time zone, Network daylight saving time).

Editor's note:	It is FFS which configuration parameters are updated by this procedure (e.g. UE NSSAI information, and policy information).

Editor's note:	It is FFS whether the case when configuration provided by a different NF than the AMF is covered by this or a different NAS procedure (e.g. UE route selection policies (URSP) provided by PCF).

Figure 8.5.1.2.1.1: UE configuration update procedure

##### 8.5.1.2.2	UE Configuration update procedure initiated by the network

The AMF shall initiate the UE configuration procedure by sending the CONFIGURATION UPDATE COMMAND message to the UE. If an acknowledgement from the UE is requested, the AMF shall indicate acknowledgement requested in the Acknowledgement requested flag IE in the UE CONFIGURATION message and shall start timer Tx.

During an established 5GMM context, the network may send none, one, or more CONFIGURATION UPDATE COMMAND messages to the UE. If more than one CONFIGURATION UPDATE COMMAND message is sent, the messages need not have the same content.

##### 8.5.1.2.3	UE Configuration update accepted by the UE

Upon receiving the CONFIGURATION UPDATE COMMAND message, the UE shall use the contents to update appropriate information stored within the UE. If acknowledgement requested is indicated in the Acknowledgement requested flag IE in the UE CONFIGURATION message and all information elements included are successfully accepted by the UE, the UE shall send a CONFIGURATION UPDATE COMPLETE message.

If the UE receives a new 5G-GUTI in the CONFIGURATION UPDATE COMMAND message, the UE considers the new 5G-GUTI as valid and the old 5G-GUTI as invalid; otherwise, the UE shall consider the old 5G-GUTI as valid.

If the UE receives a new TAI list in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new TAI list as valid and the old TAI list as invalid; otherwise, the UE shall consider the old TAI list as valid.

If the UE receives a Mobility restrictions IE in the CONFIGURATION UPDATE COMMAND message, the UE shall consider the new mobility restrictions as valid and the old mobility restrictions as invalid; otherwise, the UE shall consider the old mobility restrictions as valid.

Editor's note:	Further details on handling of specific IEs are FFS.

##### 8.5.1.2.4	UE Configuration update completion by the network

Upon receipt of the CONFIGURATION UPDATE COMPLETE message, the AMF shall stop the timer.

If a new 5G-GUTI was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new 5G-GUTI as valid and the old 5G-GUTI as invalid.

If a new TAI list was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new TAI list as valid and the old TAI list as invalid.

If a Mobility restrictions IE was included in the CONFIGURATION UPDATE COMMAND message, the AMF shall consider the new mobility restrictions as valid and the old mobility restrictions as invalid.

Editor's note:	Further details on handling of specific IEs are FFS.

##### 8.5.1.2.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Transmission failure of the CONFIGURATION UPDATE COMPLETE message.

Editor's note:	Further abnormal cases in the UE are FFS.

##### 8.5.1.2.6	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Expiry of timer Tx.

Editor's note:	Further abnormal cases in the network are FFS.

#### 8.5.1.3	Identification procedure

##### 8.5.1.3.1	General

The purpose of this procedure is to to request a particular UE to provide specific identification parameters, e.g. the SUPI or the PEI. SUPI and PEI definition and structure are specified in 3GPP TS 23.003 [5].

##### 8.5.1.3.2	UE Configuration update procedure initiated by the network

The AMF shall initiate the identification procedure by sending the IDENTITY REQUEST message to the UE and starting timer Tf.

Editor's note:	The IE used to request the identification parameter(s) is FFS.

##### 8.5.1.3.3	Identification response by the UE

A UE shall be ready to respond to an IDENTITY REQUEST message at any time whilst in 5GMM-CONNECTED mode.

Upon receipt of the IDENTITY REQUEST message the UE shall send an IDENTITY RESPONSE message to the network. The IDENTITY RESPONSE message shall contain the identification parameters as requested by the network.

##### 8.5.1.3.4	Identification completion by the network

Upon receipt of the IDENTITY RESPONSE the network shall stop the timer Tf.

##### 8.5.1.3.5	Abnormal cases in the UE

The following abnormal cases can be identified:

- Transmission failure of the IDENTIFY RESPONSE message (if the identification procedure is triggered by a registration procedure).

Editor's note:	Further abnormal cases in the UE are FFS.

##### 8.5.1.3.6	Abnormal cases on the network side

The following abnormal cases can be identified:

a) 	Expiry of timer Tf

The network shall, on the first expiry of the timer Tf, retransmit the IDENTITY REQUEST message and reset and restart the timer Tf. This retransmission is repeated four times, i.e. on the fifth expiry of timer Tf, the network shall abort the identification procedure and any ongoing 5GMM procedure.

Editor's note:	Further abnormal cases in the network are FFS.

### 8.5.2	5GMM specific procedures

#### 8.5.2.1	Primary authentication and key agreement procedure

##### 8.5.2.1.1	General

The purpose of the primary authentication and key agreement procedure is to enable mutual authentication between the UE and the network and to provide keying material that can be used between the UE and network in subsequent security procedures, as specified in 3GPP TS 33.501 [15].

Two methods are defined:

-	EAP based primary authentication and key agreement procedure.

-	EPS AKA* based primary authentication and key agreement procedure.

The UE and the AMF shall support the EAP based primary authentication and key agreement procedure and the EPS AKA* based primary authentication and key agreement procedure.

##### 8.5.2.1.2	EAP based primary authentication and key agreement procedure

###### 8.5.2.1.2.1	General

Extensible authentication protocol (EAP) as specified in IETF RFC 3748 [21] enables authentication using various EAP methods.

Several rounds of exchanges of an EAP-request message and a related EAP-response message can be required to achieve the authentication.

The AUSF and the AMF support exchange of EAP messages using N12.

Editor's note:	Transport of EAP messages between AMF and UE is FFS.

###### 8.5.2.1.2.2	EAP-AKA' related procedures

8.5.2.1.2.2.1	General

The UE shall support acting as EAP-AKA' peer as specified in IETF RFC 5448 [25]. The AUSF may support acting as EAP-AKA' server as specified in IETF RFC 5448 [25].

The EAP-AKA' enables mutual authentication of the UE and the network.

8.5.2.1.2.2.2	Initiation

In order to initiate the EAP based primary authentication and key agreement procedure using EAP-AKA', the AUSF shall send an EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [25]. The AUSF shall set the AT_KDF_INPUT attribute of the EAP-request/AKA'-challenge message to the SNN. The SNN is in format described in subclause 8.8.1. The AUSF may include AT_RESULT_IND attribute in the EAP-request/AKA'-challenge message.

Upon receiving an EAP-request/AKA'-challenge message, the UE checks whether the network name field of the AT_KDF_INPUT attribute is the SNN constructed according to subclause 8.y.1.

Editor's note:	It needs to be described how the UE gets the input for checking the SNN.

8.5.2.1.2.2.3	UE successfully authenticates network

If the SNN check is successful and procedures for handling of the EAP-request/AKA'-challenge message specified in IETF RFC 5448 [25] are successful, the UE shall generate an anchor key and shall send an EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [25].

Editor's note:	Anchor key generation is FFS.

If the EAP-request/AKA'-challenge message contains AT_RESULT_IND attribute, the UE may include AT_RESULT_IND attribute in the EAP-response/AKA'-challenge message.

8.5.2.1.2.2.4	Errors when handling EAP-request/AKA'-challenge message

If the SNN check fails or the UE does not accept AUTN during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [25], the UE shall send an EAP-response/AKA'-authentication-reject message as specified in IETF RFC 5448 [25].

If the SNN check is successful but the UE detects that the sequence number in AUTN is not correct during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [25], the UE shall send an EAP-response/AKA'-synchronization-failure message as specified in IETF RFC 5448 [25].

If the SNN check is successful, the sequence number in AUTN is correct and the UE detects another error during handling of the EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [25], the UE shall send an EAP-response/AKA'-client-error message as specified in IETF RFC 5448 [25].

8.5.2.1.2.2.5	Network successfully authenticates UE

Upon reception of the EAP-response/AKA'-challenge message, if procedures for handling an EAP-response/AKA'-challenge message as specified in IETF RFC 5448 [25] are successful, the AUSF shall generate anchor key and checks whether the AT_RESULT_IND attribute is included in the EAP-response/AKA'-challenge message.

If the AT_RESULT_IND attribute is included in the EAP-response/AKA'-challenge message, the AUSF shall send an EAP-request/AKA'-notification message as specified in IETF RFC 5448 [25].

If the AT_RESULT_IND attribute is not included in the EAP-response/AKA'-challenge message, the AUSF shall send an EAP-success message as specified in IETF RFC 5448 [25] along with the anchor key and shall consider the procedure complete.

8.5.2.1.2.2.6	UE handling EAP-AKA' notification

Upon receiving an EAP-request/AKA'-notification message, the UE shall send an EAP-response/AKA'-notification message as specified in IETF RFC 5448 [25].

8.5.2.1.2.2.7	Network sending EAP-success

Upon reception of the EAP-response/AKA'-notification message, if earlier procedures for handling an EAP-request/AKA'-challenge message as specified in IETF RFC 5448 [25] were successful, the AUSF shall send an EAP-success message as specified in IETF RFC 5448 [258] along with the anchor key and shall consider the procedure complete.

8.5.2.1.2.2.8	UE handling EAP-success

Upon receiving an EAP-success message, the UE shall consider the procedure complete.

##### 8.5.2.1.3	5GS based primary authentication and key agreement procedure

###### 8.5.2.1.3.1	General

The purpose of the 5GS authentication and key agreement (AKA) procedure is to provide mutual authentication between the user and the network and to agree on a key KASME (see 3GPP TS 33.501 [15]). The cases when the 5GS AKA procedure should be used are defined in 3GPP TS 33.501 [15].

The 5G AKA procedure is always initiated and controlled by the network. However, the UE can reject the 5GS authentication challenge sent by the network.

The UE shall proceed with an 5G authentication challenge only if a USIM is present.

A partial native 5G security context is established in the UE and the network when an 5GS authentication is successfully performed. During a successful 5G authentication procedure, the CK and IK are computed by the USIM. CK and IK are then used by the ME as key material to compute a new key, KASME. KASME is stored in the 5GS security contexts (see 3GPP TS 33.501 [15]) of both the network and in the volatile memory of the ME while attached to the network, and is the root for the 5GS integrity protection and ciphering key hierarchy.

###### 8.5.2.1.3.2	Authentication initiation by the network

When an NAS signalling connection exists, the network can initiate an authentication procedure at any time. For restrictions applicable after handover or inter-system handover to N1 mode see subclause 8.5.2.1.2.3.

The network initiates the authentication procedure by sending an AUTHENTICATION REQUEST message to the UE and starting the timer T3560 (see example in figure 8.5.2.1.3.2.1). The AUTHENTICATION REQUEST message contains the parameters necessary to calculate the authentication response (see 3GPP TS 33.501 [15]).

If an ngKSI is contained in an initial NAS message during an 5GMM procedure, the network shall include a different ngKSI value in the AUTHENTICATION REQUEST message when it initiates an authentication procedure.

# Figure 8.5.2.1.3.2.1: Authentication procedure

###### 8.5.2.1.3.3	Authentication response by the UE

The UE shall respond to an AUTHENTICATION REQUEST message. With the exception of the cases described in subclause 8.5.2.1.3.6, the UE shall process the authentication challenge data and respond with an AUTHENTICATION RESPONSE message to the network.

Upon a successful 5GS authentication challenge, the UE shall determine the PLMN identity to be used for the calculation of the new KASME from the authentication challenge data according to the following rules:

a)	When the UE moves from 5GMM-IDLE mode to 5GMM-CONNECTED mode, until the first handover, the UE shall use the PLMN identity of the selected PLMN; and

b)	After handover or inter-system handover to N1-mode,

-	if the target cell is not a shared network cell, the UE shall use the PLMN identity received as part of the broadcast system information;

-	if the target cell is a shared network cell and the UE has a valid 5G-GUTI, the UE shall use the PLMN identity that is part of the 5G-GUTI; and

-	if the target cell is a shared network cell and the UE has a valid GUTI and TAI, but not a valid 5G-GUTI, the UE shall use the PLMN identity that is part of the TAI.

Editor’s note:	Security context coordination between EPS and 5GS is FFS.

Upon a successful 5GS authentication challenge, the new KASME calculated from the authentication challenge data shall be stored in a new 5GS security context in the volatile memory of the ME.

The USIM will compute the authentication response (RES) using the authentication challenge data received from the ME, and pass RES to the ME.

In order to avoid a synchronisation failure, when the UE receives an AUTHENTICATION REQUEST message, the UE shall store the received RAND together with the RES returned from the USIM in the volatile memory of the ME. When the UE receives a subsequent AUTHENTICATION REQUEST message, if the stored RAND value is equal to the new received value in the AUTHENTICATION REQUEST message, then the ME shall not pass the RAND to the USIM, but shall send the AUTHENTICATION RESPONSE message with the stored RES. If there is no valid stored RAND in the ME or the stored RAND is different from the new received value in the AUTHENTICATION REQUEST message, the ME shall pass the RAND to the USIM, shall override any previously stored RAND and RES with the new ones and start, or reset and restart timer T3516.

The RAND and RES values stored in the ME shall be deleted and timer T3516, if running, shall be stopped:

-	upon receipt of a

-	SECURITY MODE COMMAND,

-	SERVICE REJECT,

-	REGISTRATION REJECT,

-	REGISTRATION ACCEPT, or

-	AUTHENTICATION REJECT message;

-	upon expiry of timer T3516;

-	if the UE enters the 5GMM state 5GMM-DEREGISTERED or 5GMM-NULL; or

-	if the UE enters 5GMM-IDLE mode.

###### 8.5.2.1.3.4	Authentication completion by the network

Upon receipt of an AUTHENTICATION RESPONSE message, the network stops the timer T3560 and checks the correctness of RES (see 3GPP TS 33.501 [15]).

If the 5GS authentication procedure has been completed successfully and the related ngKSI is stored in the 5GS security context of the network, the network shall include a different ngKSI value in the AUTHENTICATION REQUEST message when it initiates a new authentication procedure.

Upon receipt of an AUTHENTICATION FAILURE message, the network stops the timer T3560. In the case where the 5GMM cause #21 "synch failure" is received, the core network may renegotiate with the UDM/AUSF and provide the UE with new authentication parameters.

###### 8.5.2.1.3.5	Authentication not accepted by the network

Editor’s note:	Authentication not accepted by the network is FFS.

###### 8.5.2.1.3.6	Authentication not accepted by the UE

Editor’s note:	Authentication not accepted by the UE is FFS.

###### 8.5.2.1.3.7	Abnormal cases

Editor’s note:	Abnormal cases are FFS. However the below message flow is expected to apply, .i.e. an authentication failure message is needed.

# Figure 8.5.2.1.3.7.2: Authentication failure procedure

#### 8.5.2.2	Registration procedure

##### 8.5.2.2.1	General

The registration procedure is always initiated by the UE and can be used for the following purposes:

-	initial registration for 5GS services;

-	mobility registration updating to update the registration of the actual tracking area of a UE in the network;

-	periodic registration updating to periodically notify the availability of the UE to the network;

-	initial registration for emergency services;

-	initial registration for SMS over NAS service, change in the requirements to use SMS over NAS service, and deregistration from SMS over NAS service.

Editor's note:	Further purposes are FFS.

##### 8.5.2.2.2	Registration procedure for initial registration

###### 8.5.2.2.2.1	General

This procedure can be used by a UE for initial registration for 5GS services.

When the UE initiates the registration procedure for initial registration, the UE shall indicate "initial registration" in the 5GS registration type IE.

###### 8.5.2.2.2.2	Initial registration initiation

The UE initiates the registration procedure for initial registration by sending a REGISTRATION REQUEST message to the AMF, starting timer Ta. If timer Tb is currently running, the UE shall stop timer Tb. If timer Tc is currently running, the UE shall stop timer Tc.

If the UE supports network slicing, the UE shall include the requested NSSAI containing the S-NSSAI(s) corresponding to the slice(s) to which the UE wishes to register in the REGISTRATION REQUEST. The requested NSSAI shall be either:

-	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN; or

-	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

-	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and that were not previously rejected with"S-NSSAI not available in this PLMN" or "S-NSSAI not available in this registration area" (see subclause 8.5.2.2.2.4 and subclause 8.5.2.2.3.4) by the network for the present registration area.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to this PLMN, if:

-	the S-NSSAI was not previously rejected with "S-NSSAI not available in this PLMN" or "S-NSSAI not available in this registration area" (see subclause 8.5.2.2.2.4 and subclause 8.5.2.2.3.4) by the network for the present tracking area; or

-	the S-NSSAI was not previously added by the UE in a requested NSSAI.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for this PLMN.

NOTE 1:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 2:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

If the UE suppots S1 mode, the UE shall include the S1 mode supported indication in the REGISTRATION REQUEST message.

Editor's note:	Details on how the S1 mode supported indication is included in the REGISTRATION REQUEST message are FFS.

Figure 8.5.2.2.2.2.1: Registration procedure for initial registration

#### 8.5.2.2.2.3	5GMM common procedure initiation

The network may initiate 5GMM common procedures, e.g. the identification, authentication and security procedures during the registration procedure, depending on the information received in the REGISTRATION REQUEST message.

###### 8.5.2.2.2.4	Initial registration accepted by the network

If the initial registration request is accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE and start timer Td.

The AMF shall include the allowed NSSAI in the REGISTRATION ACCEPT if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the Request NSSAI but rejected by the network associated with rejection cause(s).

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in this PLMN"

The UE shall not attempt to use this S-NSSAI with current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI temporarily not available in this PLMN"

Editor's note:	It is FFS whether the rejection cause "S-NSSAI temporarily not available in this PLMN" is needed.

"S-NSSAI not available in this registration area"

The UE shall not attempt to use this S-NSSAI in the present registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

"S-NSSAI temporarily not available in the registration area"

The UE considers S-NSSAI temporarily not available in the current registration area.

Editor's note:	Further UE action is FFS when the rejection cause is "S-NSSAI temporarily not available in the registration area".

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message and one or more subscribed S-NSSAIs marked as default are available, the AMF shall put the subscribed S-NSSAI(s) marked as default in the allowed NSSAI of the REGISTRAION ACCEPT message.

Editor's note:	It is FFS whether the AMF includes the subscribed S-NSSAI(s) marked as default in the allowed NSSAI also when the UE sent the requested NSSAI.

If the REGISTRATION ACCEPT contains the allowed NSSAI, then the UE shall store the included allowed NSSAI together with the PLMN identity of the registered PLMN and the registration area as specified in subclause 13.2.2.

If the UE included S1 mode supported indication in the REGISTRATION REQUEST message, the AMF supporting intersystem change with EPS shall set the dual-registration supported IE to either:

-	"dual-registration not supported" if the AMF supports the N26 interface; or

-	"dual-registration supported" if the AMF does not support the N26 interface.

The UE shall select the mode for intersystem change as follows:

1)	if the dual-registration supported IE is set to "dual-registration not supported", the UE shall select single-registration mode; or

2)	if the dual-registration support IE is set to "dual-registration supported", the UE shall select dual-registration mode, if the UE supports dual-registration mode.

Editor's note:	Details on the case where the network provides the dual-registration supported IE set to "dual-registration supported" and the UE does not support dual-registration mode, are FFS.

The UE shall take the selected mode for intersystem change as valid for the entire PLMN.

Editor's note:	Further details on initial registration accepted by the network are FFS.

###### 8.5.2.2.2.5	Initial registration not accepted by the network

If the initial registration request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(5GS services not allowed).

Editor's note:	Further details on initial registration not accepted by the network are FFS.

###### 8.5.2.2.2.6	Initial registration for emergency services not accepted by the network

Editor's note:	Possible handling of initial registration for emergency services by the network is FFS.

###### 8.5.2.2.2.7	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

###### 8.5.2.2.2.8	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network are FFS.

##### 8.5.2.2.3	Registration procedure for mobility and periodic registration update

###### 8.5.2.2.3.1	General

This procedure can be used by a UE for mobility and periodic registration update for 5GS services.

When the UE initiates the registration procedure for mobility and periodic registration update, the UE shall indicate "mobility registration update" or "periodic registration update" respectively in the 5GS registration type IE.

The periodic registration updating procedure is controlled in the UE by timer Te. When timer Te expires, the periodic registration area updating procedure is started. Start and reset of timer Te is described in subclause 8.9.

###### 8.5.2.2.3.2	Mobility and periodic registration update initiation

The UE in state 5GMM-REGISTERED shall initiate the mobility and periodic registration updating procedure by sending a REGISTRATION REQUEST message to the AMF,

a)	when the UE detects entering a tracking area that is not in the list of tracking areas that the UE previously registered in the AMF; or

b)	when the periodic registration updating timer Te expires.

Editor's note:	Further cases for initiation of mobility and periodic registration update are FFS.

After sending the REGISTRATION REQUEST message to the AMF the UE shall start timer Ta. If timer Tb is currently running, the UE shall stop timer Tb. If timer Tc is currently running, the UE shall stop timer Tc.

If the UE is initiating the mobility registration update procedure, the UE may include the uplink data status IE to indicate which PDU session(s) have pending user data to be sent.

When the registration procedure is initiated in 5GMM-IDLE mode, the UE may include a PDU session status IE in the REGISTRATION REQUEST message, indicating which PDU session contexts are active in the UE.

If the UE performs mobility from S1 mode to N1 mode, the UE shall indicate "mobility registration update" in the 5GS registration type IE. The UE shall include the 5G-GUTI IE mapped from 4G-GUTI in the REGISTRATION REQUEST message.

Editor’s note:	Mapping between 4G-GUTI and 5G-GUTI is FFS and will be based on the SA2 outcome.

If the UE supports network slicing, the UE shall include the requested NSSAI containing the S-NSSAIs corresponding to the slices to which the UE wishes to register in the REGISTRATION REQUEST. The requested NSSAI shall be either:

-	the configured NSSAI for the current PLMN, or a subset thereof as described below, if the UE has no allowed NSSAI for the current PLMN; or

-	the allowed NSSAI for the current PLMN, or a subset thereof as described below, if the UE has an allowed NSSAI for the current PLMN; or

-	the allowed NSSAI for the current PLMN, or a subset thereof as described below, plus one or more S-NSSAIs from the configured NSSAI for which no corresponding S-NSSAI is present in the allowed NSSAI and that were not previously rejected with"S-NSSAI not available in this PLMN" or "S-NSSAI not available in this registration area"(see subclause 8.5.2.2.2.4 and subclause 8.5.2.2.3.4) by the network for the present registration area.

The subset of configured NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the configured NSSAI applicable to this PLMN, if:

-	the S-NSSAI was not previously rejected with "S-NSSAI not available in this PLMN" (see subclause 8.5.2.2.2.4 and subclause 8.5.2.2.3.4) by the network for the present registration area; or

-	the S-NSSAI was not previously added by the UE in a requested NSSAI.

The subset of allowed NSSAI provided in the requested NSSAI consists of one or more S-NSSAIs in the allowed NSSAI for this PLMN.

NOTE 1:	How the UE selects the subset of configured NSSAI or allowed NSSAI to be provided in the requested NSSAI is implementation.

NOTE 2:	The number of S-NSSAI(s) included in the requested NSSAI cannot exceed eight.

Figure 8.5.2.2.3.2.1: Registration procedure for mobility and periodic registration update

###### 8.5.2.2.3.3	5GMM common procedure initiation

The network may initiate 5GMM common procedures, e.g. the identification, authentication and security procedures during the registration procedure, depending on the information received in the REGISTRATION REQUEST message.

###### 8.5.2.2.3.4	Mobility and periodic registration update accepted by the network

If the registration update request has been accepted by the network, the AMF shall send a REGISTRATION ACCEPT message to the UE. If the AMF assigns a new 5G-GUTI for the UE, a 5G-GUTI shall be included in the REGISTRATION ACCEPT message. In this case, the AMF shall start timer Td.

The AMF shall include the allowed NSSAI in the REGISTRATION ACCEPT if the UE included the requested NSSAI in the REGISTRATION REQUEST message and the AMF allows one or more S-NSSAIs in the requested NSSAI. The AMF may also include rejected NSSAI in the REGISTRATION ACCEPT message. Rejected NSSAI contains S-NSSAI(s) which was included in the Request NSSAI but rejected by the network associated with rejection cause(s).

The UE receiving the rejected NSSAI in the REGISTRATION ACCEPT message takes the following actions based on the rejection cause in the rejected NSSAI:

"S-NSSAI not available in this PLMN"

The UE shall not attempt to use this S-NSSAI with current PLMN until switching off the UE or the UICC containing the USIM is removed.

"S-NSSAI temporarily not available in this PLMN"

Editor's note:	It is FFS whether the rejection cause "S-NSSAI temporarily not available in this PLMN" is needed."S-NSSAI not available in this registration area"

The UE shall not attempt to use this S-NSSAI in the present registration area until switching off the UE, the UE moving out of the current registration area or the UICC containing the USIM is removed.

"S-NSSAI temporarily not available in the registration area"

The UE considers S-NSSAI temporarily not available in the current registration area.

Editor's note:	Further UE action is FFS when the rejection cause is"S-NSSAI temporarily not available in the registration area".

If the UE did not include the requested NSSAI in the REGISTRATION REQUEST message and one or more subscribed S-NSSAIs marked as default are available, the AMF shall put the subscribed S-NSSAI(s) marked as default in the allowed NSSAI of the REGISTRAION ACCEPT message.

Editor's note:	It is FFS whether the AMF includes the subscribed S-NSSAI(s) marked as default in the allowed NSSAI also when the UE sent the requested NSSAI.

If the REGISTRATION ACCEPT contains the allowed NSSAI, then the UE shall store the included allowed NSSAI together with the PLMN identity of the registered PLMN and the registration area as specified in subclause 13.2.2.

If the uplink data status IE is included in the REGISTRATION REQUEST message, the AMF shall:

-	indicate the SMF to re-activate the user plane for the corresponding PDU session contexts; and

-	include PDU session reactivation result IE to indicate the reactivation result of the UE requested reactivation for the PDU session.

If the uplink data status IE is not included in the REGISTRATION REQUEST message, the AMF may indicate the SMF to re-activate the user plane for the PDU session contexts.

If a PDU session context status IE is included in the REGISTRATION REQUEST message, the AMF shall:

-	release all those PDU session contexts locally (without peer-to-peer signalling between the network and the UE) which are in 5GSM state PDU SESSION ACTIVE on the AMF side, but are indicated by the UE as being in 5GSM state PDU SESSION INACTIVE; and

-	include a PDU session context status IE in the REGISTRATION ACCEPT message to indicate which PDU session contexts are active in the AMF.

The AMF may include the LADN availability information in the REGISTRATION ACCEPT message as described in subclause 8.5.2.2.2.4.

The UE shall release all those PDU session contexts locally (without peer-to-peer signalling between the network and the UE) which are in 5GSM state PDU SESSION ACTIVE on the UE side, but are indicated by the AMF as being in 5GSM state PDU SESSION INACTIVE.

Editor's note:	Further details on mobility and periodic registration update accepted by the network are FFS.

###### 8.5.2.2.3.5	Mobility and periodic registration update not accepted by the network

If the mobility and periodic registration update request cannot be accepted by the network, the AMF shall send a REGISTRATION REJECT message to the UE including an appropriate 5GMM cause value.

The UE shall take the following actions depending on the 5GMM cause value received in the REGISTRATION REJECT message.

#3		(Illegal UE);

#6		(Illegal ME); or

#8		(5GS services not allowed).

Editor's note:	Further details on mobility and periodic registration update not accepted by the network are FFS.

###### 8.5.2.2.3.6	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

###### 8.5.2.2.3.7	Abnormal cases on the network side

Editor's note:	Abnormal cases in the network are FFS.

#### 8.5.2.3	Deregistration procedure

##### 8.5.2.3.1	General

The deregistration procedure is used:

-	by the UE to deregister for 5GS services over 3GPP access;

-	by the UE to deregister for 5GS services over both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses;

-	by the network to inform the UE that it is deregistered for 5GS services over 3GPP access;

- 	by the network to inform the UE that it is deregistered for 5GS services over both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses; and

-	by the network to inform the UE to re-register to the network.

The deregistration procedure with appropriate deregistration type shall be invoked by the UE:

- if the UE is switched off,

- the UE wishes to deregister for 5GS services over 3GPP access, or

- the UE wishes to deregistration for 5GS service over both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses.

If the deregistration procedure is triggered due to USIM removal, the UE shall indicate "switch off" in the de-registration type IE.

If the deregistration procedure is requested by the UDM for a UE that has PDU sessions for emergency services, the AMF shall not send a DEREGISTRATION REQUEST message to the UE.

If the deregistration procedure for 5GS services is performed, the PDU sessions, if any, for this particular UE are deactivated locally without peer-to-peer signalling between the UE and the network.

##### 8.5.2.3.2	UE-initiated deregistration procedure

###### 8.5.2.3.2.1	UE-initiated deregistration procedure initiation

The deregistration procedure is initiated by the UE by sending a DEREGISTRATION REQUEST message (see example in figure 8.5.2.3.2.1.1). The Deregistration type IE included in the message indicates whether deregistration procedure is due to a "switch off" or not. The De-registration type IE also indicates whether the de-registration procedure is:

- for 5GS services over 3GPP access, or

- for 5GS services over both 3GPP access and non-3GPP access when the UE is registered in the same PLMN over both accesses.

If the deregistration is not due to switch off and the UE is in the state 5GMM-REGISTERED or 5GMM-REGISTERED-INITIATED, timer Txa shall be started in the UE after the DEREGISTRATION REQUEST message has been sent. The UE shall enter the state EMM-DEREGISTERED-INITIATED.

If the UE is to be switched off, the UE shall try for a period of 5 seconds to send the DEREGISTRATION REQUEST message. During this period, the UE may be switched off as soon as the DEREGISTRATION REQUEST message has been sent.

# Figure 8.5.2.3.2.1.1: UE-initiated deregistration procedure

###### 8.5.2.3.2.3	UE-initiated deregistration procedure completion for 5GS services over both 3GPP access and non-3GPP access

The description for UE-initiated deregistration procedure completion for 5GS services over 3GPP access as specified in subclause 8.5.2.3.2.2 shall be followed. In addition, the following description for deregistration over non-3GPP access applies.

If the DEREGISTRATION REQUEST includes indication that the deregistration procedure is for both 3GPP access and non-3GPP access, the AMF shall trigger SMF to deactivate the PDU session context(s) established over non-3GPP access, if any, locally for this UE without peer-to-peer signalling between the UE and the SMF. The UE shall deactivate the PDU session context(s) established over non-3GPP access, if any, locally without peer-to-peer signalling between the UE and the SMF. The UE is marked as inactive in the AMF for 5GS services for non-3GPP access. The AMF shall enter the state 5GMM-DEREGISTERED over non-3GPP access.

If the UE supports N1 mode only then:

-	If the deregistration procedure was performed due to disabling of 5GS services, then the UE shall enter the 5GMM-NULL state for non-3GPP access.

-	Otherwise, the UE shall enter the 5GMM-DEREGISTERED state for non-3GPP access.

###### 8.5.2.3.2.4	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

###### 8.5.2.3.2.5	Abnormal cases in the network side

Editor's note:	Abnormal cases in the network are FFS.

##### 8.5.2.3.3	Network-initiated deregistration procedure

###### 8.5.2.3.3.1	Network-initiated deregistration procedure initiation

The network initiates the deregistration procedure by sending a DEREGISTRATION REQUEST message to the UE (see example in figure 8.5.2.3.3.1.1).

NOTE:	If the AMF performs a local deregistration, it will inform the UE with a 5GMM messages (e.g. SERVICE REJECT or TRACKING AREA UPDATE reject) with 5GMM cause #x "implicit deregistration" only when the UE initiates a 5GMM procedure.

The network may include a 5GMM cause IE to specify the reason for the DEREGISTRATION REQUEST message. The network shall start timer Tya. The network shall indicate whether re-registration is needed or not in the Deregistration type IE. The network shall also indicate via the Deregistration type IE whether the deregistration procedure is for 3GPP access, or for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN for both accesses. The network shall deactivate the PDU session context(s) over the indicated access(es), if any, for the UE locally and enter state 5GMM-DEREGISTERED-INITIATED.

# Figure 8.5.2.3.3.1.1: Network-initiated deregistration procedure

###### 8.5.2.3.3.2	Network-initiated deregistration procedure completion by the UE

Upon receiving the DEREGISTRATION REQUEST message, and the DEREGISTRATION REQUEST message indicates "re-registration required" and the de-registration request is for 3GPP access , the UE shall deactivate the PDU sessions over 3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall ignore the 5GMM cause IE if received.The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state EMM-DEREGISTERED for 3GPP access. Furthermore, the UE shall, after the completion of the deregistration procedure, and the release of the existing NAS signalling connection, initiate an initial registration. The UE should also re-establish any previously established PDU sessions.

Upon receiving the DEREGISTRATION REQUEST message and the DEREGISTRATION REQUEST message indicates "re-registration required" and the deregistration request is for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN for both accesses, the UE shall deactivate the PDU sessions over both 3GPP access and non-3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall ignore the 5GMM cause IE if received. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for both 3GPP access and non-3GPP access. Furthermore, the UE shall, after the completion of the deregistration procedure, and the release of the existing NAS signalling connection, initiate an initial registration. The UE should also re-establish any previously established PDU sessions.

NOTE 1:	When the deregistration type indicates "re-registration required", user interaction is necessary in some cases when the UE cannot re-activate the PDU session (s), if any, automatically.

Upon receiving the DEREGISTRATION REQUEST message and the DEREGISTRATION REQUEST message indicates "re-registration not required" and the deregistration request is for 3GPP access, the UE shall deactivate the PDU sessions over 3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GEMM-DEREGISTERED for 3GPP access.

Upon receiving the DEREGISTRATION REQUEST message and the DEREGISTRATION REQUEST message indicates "re-registration not required" and the deregistration request is for both 3GPP access and non-3GPP access when the UE is registered in the same PLMN for both accesses, the UE shall deactivate the PDU sessions over both 3GPP access and non-3GPP access, if any, locally without peer-to-peer signalling between the UE and the network. The UE shall send a DEREGISTRATION ACCEPT message to the network and enter the state 5GMM-DEREGISTERED for both 3GPP access and non-3GPP access.

If the deregistration type indicates "re-registration not required", the UE shall take the actions depending on the received 5GMM cause value.

###### 8.5.2.3.3.3	Network-initiated deregistration procedure completion by the network

The network shall stop timer Tya upon receipt of the DEREGISTRATION ACCEPT message. The network shall enter state 5GMM-DEREGISTERED for 3GPP access if the deregistration request is for 3GPP access. The network shall enter state 5GMM-DEREGISTERED for both 3GPP access and non-3GPP access if the de-registration request is for both 3GPP access and non-3GPP access.

###### 8.5.2.3.3.4	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

###### 8.5.2.3.3.5	Abnormal cases in the network side

Editor's note:	Abnormal cases in the network are FFS.

### 8.5.3	5GMM connection management procedures

#### 8.5.3.1	Service request procedure

##### 8.5.3.1.1	General

The purpose of the service request procedure is to change the 5GMM mode from 5GMM-IDLE to 5GMM-CONNECTED mode, and/or to request establishment of user-plane radio resources for PDU sessions which are activated without radio resources. In latter case, the 5GMM mode can be the 5GMM-IDLE mode or the 5GMM-CONNECTED mode if the UE requires radio resources. This procedure is used when:

-	the network has downlink signalling pending over 3GPP access and the UE is in 5GMM-IDLE mode in 3GPP access;

-	the network has downlink signalling pending over non-3GPP access, no N1 NAS signalling connection between the UE and the network over non-3GPP access exists and the UE is in 5GMM-IDLE or 5GMM-CONNECTED mode in 3GPP access;

-	the UE has uplink signalling pending over 3GPP access and the UE is in 5GMM-IDLE mode in 3GPP access;

-	the network has downlink user data pending over 3GPP access and the UE is in 5GMM-IDLE mode in 3GPP access;

-	the network has downlink user data pending over non-3GPP access, no N1 NAS signalling connection between the UE and the network over non-3GPP access exists and the UE is in 5GMM-IDLE or 5GMM-CONNECTED mode in 3GPP access; or

-	the UE has user data pending over 3GPP access and the UE is in 5GMM-IDLE or 5GMM-CONNECTED mode in 3GPP access.

This procedure shall not be used for initiating user data transfer or PDU session modification signalling related to a PDU session for LADN for when the UE is located outside the LADN service area.

The service request procedure is initiated by the UE, however, it can be triggered by the network by means of:

-	the paging procedure (see subclause 8.5.3.2) for the transfer of downlink signalling or user data pending over 3GPP access to a UE in 5GMM-IDLE mode in 3GPP access;

-	the paging procedure (see subclause 8.4.3.2) for the transfer of downlink signalling or user data pending over non-3GPP access to a UE in 5GMM-IDLE mode in 3GPP access and no N1 NAS signalling connection between the UE and the network over non-3GPP access exists; or

-	the notification procedure (see subclause 8.4.3.x) for the transfer of downlink signalling or user data pending over non-3GPP access to a UE in 5GMM-CONNECTED mode in 3GPP access and no N1 NAS signalling connection between the UE and the network over non-3GPP access exists.

The UE shall invoke the service request procedure when:

a)	the UE, in 5GMM-IDLE mode in 3GPP access, receives a paging request from the network;

b)	the UE, in 5GMM-CONNECTED mode in 3GPP access, receives a notification from the network;

c)	the UE, in 5GMM-IDLE mode in 3GPP access, has uplink signalling pending;

d)	the UE, in 5GMM-IDLE mode in 3GPP access, has uplink user data pending;

e)	the UE, in 5GMM-CONNECTED mode in 3GPP access, has user data pending due to no user-plane radio resources established for PDU session(s) used for user data transport;

# Figure 8.5.3.1.1.1: Service Request procedure

##### 8.5.3.1.2	Service request procedure initiation

The UE initiates the service request procedure by sending a SERVICE REQUEST message to the AMF and starts timer Tx.

For case a in subclause 8.5.3.1.1:

-	if the paging request includes an indication for non-3GPP access type, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) that the UE decides to re-activate over 3GPP access;

-	if the UE has uplink user data pending to be sent, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) the UE has pending user data to be sent; or

-	otherwise, the Uplink data status IE shall not be included in the SERVICE REQUEST message.

For case b in subclause 8.5.3.1.1, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) that the UE decides to re-activate over 3GPP access.

For case c in subclause 8.5.3.1.1, the Uplink data status IE shall not be included in the SERVICE REQUEST message.

For cases d and e in subclause 8.5.3.1.1, the Uplink data status IE shall be included in the SERVICE REQUEST message to indicate the PDU session(s) the UE has pending user data to be sent.

The PDU session status information element may be included in the SERVICE REQUEST message to indicate the PDU session(s) available in the UE. If the PDU session status information element is included in the SERVICE REQUEST message, then the AMF shall deactivate all those PDU session contexts locally (without peer-to-peer signalling between the UE and the network) which are in active on the AMF side, but are indicated by the UE as being inactive.

Editor's note:	The content of the SERVICE REQUEST message is FFS.

##### 8.5.3.1.3	Common procedure initiation

Upon receipt of the SERVICE REQUEST message, the AMF may initiate the common procedures e.g. the authentication procedure and security procedure.

##### 8.5.3.1.4	Service request procedure accepted by the network

For cases a, b, c, d and e in subclause 8.5.3.1.1, the UE shall treat the reception of the SERVICE ACCEPT message as successful completion of the procedure and stop timer Tx and enter the state 5GMM-REGISTERED.

Editor's note:	For cases a, b, d and e in subclause 8.5.3.1.1, whether the indication from the lower layers that the user-plane radio resources are set up needs to be considered as a successful completion of the procedure is FFS.

The PDU session status information element may be included in the SERVICE ACCEPT message to indicate the PDU session(s) available in the network. If the PDU session status information element is included in the SERVICE ACCEPT message, then the UE shall deactivate all those PDU session contexts locally (without peer-to-peer signalling between the network and the UE) which are in active on the UE side, but are indicated by the AMF as being inactive.

Editor's note:	The content of the SERVICE ACCEPT message is FFS.

Editor's note:	Timers are FFS.

##### 8.5.3.1.5	Service request procedure not accepted by the network

If the service request cannot be accepted, the network shall return a SERVICE REJECT message to the UE including an appropriate 5GMM cause value and stop timer Tx.

On receipt of the SERVICE REJECT message, if the message is integrity protected, the UE shall stop timer Tx if running. If the SERVICE REJECT message was received without integrity protection, then the UE shall discard the message.

Editor's note:	The content of the SERVICE REJECT message is FFS.

Editor's note:	Timers are FFS.

##### 8.5.3.1.6	Abnormal cases in the UE

Editor's note:	Abnormal cases in the UE are FFS.

##### 8.5.3.1.7	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

#### 8.5.3.2	Paging procedure

##### 8.5.3.2.1	General

The paging procedure is used by the network to request the establishment of an NAS signalling connection to the UE. The paging procedure is also used by the network to request the UE to re-activate the user plane radio resources of PDU sessions for downlink user data transport. Another purpose of the paging procedure is to request the UE to re-activate the PDU session(s) associated with non-3GPP access over 3GPP access.

Additionally, the network can use the paging procedure to initiate the mobile terminating SMS.

##### 8.5.3.2.2	Paging for 5GS services

###### 8.5.3.2.2.1	General

The network shall initiate the paging procedure for 5GS services when NAS signalling messages or user data is pending to be sent to the UE when no NAS signalling connection exists (see example in figure 8.5.3.2.2.1).

Figure 8.5.3.2.2.1.1: Paging procedure

To initiate the procedure the 5GMM entity in the AMF requests the lower layer to start paging and shall start timer Ty. Upon reception of a paging indication, the UE shall initiate a service request procedure to respond to the paging (see 3GPP TS 23.502 [9]).

Editor's note:	It is FFS how the paging procedure is performed depending on different UE Identity of the paging message.

If downlink signalling or user data is pending to be sent over non-3GPP access, the 5GMM entity in the AMF shall indicate the lower layer that the paging is due to signalling or user data associated to non-3GPP access.

The network shall stop timer Ty for the paging procedure when an integrity-protected response is received from the UE and successfully integrity checked by the network. If the response received is not integrity protected, or the integrity check is unsuccessful, timer Ty for the paging procedure shall be kept running.

Upon expiry of timer Ty, the network may reinitiate paging.

Editor's note:	Timers are FFS.

###### 8.5.3.2.2.2	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

###### 8.5.3.2.2.3	Abnormal cases in the UE

The following abnormal cases can be identified:

- Paging message received with access type set to non-3GPP access while the UE is in 5GMM-CONNECTED mode for non-3GPP access.

The UE shall not respond to paging message.

Editor's note:	Abnormal cases on the UE side are FFS.

#### 8.5.3.3	Notification procedure

##### 8.5.3.3.1	General

The notification procedure is used by the network to request the UE to re-activate the PDU session(s) associated with non-3GPP access over 3GPP access when no N1 NAS signalling connection between the UE and the network over non-3GPP access exists and the UE is in 5GMM-CONNECTED mode in 3GPP access.

##### 8.5.3.3.2	Notification procedure initiation

The network shall initiate the notification procedure by sending the NOTIFICATION message to the UE and start timer Tx (see example in figure 8.5.3.3.2.1).

Figure 8.5.3.3.2.1: Notification procedure

Upon reception of NOTIFICATION message, the UE shall initiate a service request procedure as specified in subclauses 8.5.3.1.

Editor's note:	The content of the NOTIFICATION message is FFS.

##### 8.5.3.3.3	Notification procedure completion

Upon reception of SERVICE REQUEST message, the network shall stop Tx and perform service request procedure as specified in subclauses 8.5.3.1..

Editor's note:	Timer is FFS.

##### 8.5.3.3.4	Abnormal cases on the network side

Editor's note:	Abnormal cases on the network side are FFS.

## 8.6	5GS mobility management message coding

### 8.6.1	Authentication failure

#### 8.6.1.1	Message definition

This message is sent by the UE to the network to indicate that authentication of the network has failed. See table 8.6.1.1.

Message type:	AUTHENTICATION FAILURE

Significance:		dual

Direction:			UE to network

# Table 8.6.1.1: AUTHENTICATION FAILURE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | ½ |
|  | Authentication failure message type | Message type 6.6.6.6 | M | V | 1 |
|  | 5GMM cause | 5GMM cause 8.x1.y1 | M | V | 1 |

Editor's note:	The further contents of the Authentication failure message is FFS. At least the following is needed: authentication failure parameter (optional).

### 8.6.2	Authentication reject

#### 8.6.2.1	Message definition

This message is sent by the network to the UE to indicate that the authentication procedure has failed and that the UE shall abort all activities. See table 8.6.2.1.

Message type:	AUTHENTICATION REJECT

Significance:		dual

Direction:			network to UE

# Table 8.6.2.1: AUTHENTICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | ½ |
|  | Authentication reject message type | Message type 6.6.6.6 | M | V | 1 |

Editor's note:	The content of the Authentication reject message is FFS.

### 8.6.3	Authentication request

#### 8.6.3.1	Message definition

This message is sent by the network to the UE to initiate authentication of the UE identity. See table 8.6.3.1.

Message type:	AUTHENTICATION REQUEST

Significance:		dual

Direction:			network to UE

# Table 8.6.3.1: AUTHENTICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | ½ |
|  | Authentication request message type | Message type 6.6.6.6 | M | V | 1 |
|  | ngKSI | NAS key set identifier 8.x1.y2 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | Authentication parameter RAND (5G challenge) | Authentication parameter RAND 8.x1.y3 | M | V | 16 |
|  | Authentication parameter AUTN (5G challenge) | Authentication parameter AUTN 8.x1.y4 | M | LV | 17 |

Editor's note:	The content of the Authentication request message is FFS. At least the following is needed: ngKSI (mandatory), authentication parameter RAND (mandatory), authentication parameter AUTN (mandatory).

### 8.6.4	Authentication response

#### 8.6.4.1	Message definition

This message is sent by the UE to the network to deliver a calculated authentication response to the network. See table 8.6.4.1.

Message type:	AUTHENTICATION RESPONSE

Significance:		dual

Direction:			UE to network

# Table 8.6.4.1: AUTHENTICATION RESPONSE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | ½ |
|  | Authentication response message type | Message type 6.6.6.6 | M | V | 1 |

Editor's note:	The content of the Authentication response message is FFS. At least the following is needed: authentication response parameter (mandatory).

### 8.6.5	Registration request

#### 8.6.5.1	Message definition

The REGISTRATION REQUEST message is sent by the UE to the network.

Message type:	REGISTRATION REQUEST

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the REGISTRATION REQUEST message is FFS.

### 8.6.6	Registration accept

#### 8.6.6.1	Message definition

The REGISTRATION ACCEPT message is sent by the network to the UE.

Message type:	REGISTRATION ACCEPT

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the REGISTRATION ACCEPT message is FFS.

### 8.6.7	Registration complete

#### 8.6.7.1	Message definition

The REGISTRATION COMPLETE message is sent by the UE to the network.

Message type:	REGISTRATION COMPLETE

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the REGISTRATION COMPLETE message is FFS.

### 8.6.8	Registration reject

#### 8.6.8.1	Message definition

The REGISTRATION REJECT message is sent by the network to the UE.

Message type:	REGISTRATION REJECT

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the REGISTRATION REJECT message is FFS.

### 8.6.9	UL SM message transport for Alternative 1

#### 8.6.9.1	Message definition

The UL SM MESSAGE TRANSPORT message conveys an SM message and associated information from the UE to the network.

# Message type:	UL SM MESSAGE TRANSPORT

Significance:		dual

Direction:			UE to network

# Table 8.6.9.1.1: UL SM MESSAGE TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | UL SM MESSAGE TRANSPORT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | SM message container | SM message container 8.x.1 | M | LV-E | 3-65537 |
| a | Request type | Request type 8.x.2 | O | TV | 1 |
| b | S-NSSAI | S-NSSAI 8.x.3 | O | TLV | 3 or 6 |
| c | DNN | DNN 8.x.4 | O | TLV | 3-TBD |

Editor's note:	Whether the procedure transaction identity is included in the header of 5GMM message is FFS.

### 8.6.10	DL SM message transport for Alternative 1

#### 8.6.10.1	Message definition

DL SM MESSAGE TRANSPORT message conveys an SM message and associated information from the network to the UE.

# Message type:	DL SM MESSAGE TRANSPORT

Significance:		dual

Direction:			network to UE

# Table 8.6.10.1.1: DL SM MESSAGE TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | DL SM MESSAGE TRANSPORT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | SM message container | SM message container 8.x.1 | M | LV-E | 3-65537 |

Editor's note:	Whether the procedure transaction identity is included in the header of 5GMM message is FFS.

### 8.6.11	UL generic transport for Alternative 1

#### 8.6.11.1	Message definition

The UL GENERIC TRANSPORT message transports a non-SM message and associated information from the UE to the network.

Message type:	UL GENERIC TRANSPORT

Significance:		dual

Direction:			UE to network

# Table 8.6.11.1.1: UL GENERIC TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | UL GENERIC TRANSPORT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | Message container type | Message container type 8.x.5 | M | V | 1 |
|  | Message container | Message container 8.x.6 | M | LV-E | 2-65537 |

Editor's note:	Whether the procedure transaction identity is included in the header of 5GMM message is FFS.

### 8.6.12	UL NAS transport for Alternative 2

#### 8.6.12.1	Message definition

The UL NAS TRANSPORT message transports message payload and associated information to the network.

# Message type:	UL NAS TRANSPORT

Significance:		dual

Direction:			UE to AMF

Editor's note: 	Content of UL NAS TRANSPORT message is FFS.

### 8.6.13	DL generic transport for Alternative 1

#### 8.6.13.1	Message definition

The DL GENERIC TRANSPORT message transports a non-SM message and associated information from the network to the UE.

Message type:	DL GENERIC TRANSPORT

Significance:		dual

Direction:			Network to UE

# Table 8.6.13.1.1: DL GENERIC TRANSPORT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | DL GENERIC TRANSPORT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | Message container type | Message container type 8.x.5 | M | V | 1 |
|  | Message container | Message container 8.x.6 | M | LV-E | 2-65537 |

Editor's note:	Whether the procedure transaction identity is included in the header of 5GMM message is FFS.

### 8.6.14	DL NAS transport for Alternative 2

#### 8.6.14.1	Message definition

The DL NAS TRANSPORT message transports message payload and associated information to the UE.

# Message type:	DL NAS TRANSPORT

Significance:		dual

Direction:			AMF to UE

Editor's note: 	Content of DL NAS TRANSPORT message is FFS.

### 8.6.15	Deregistration request (UE originating deregistration)

#### 8.6.15.1	Message definition

The DEREGISTRATION REQUEST message is sent by the UE to the network.

Message type:	DEREGISTRATION REQUEST

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the DEREGISTRATION REQUEST message is FFS.

### 8.6.16	Deregistration accept (UE originating deregistration)

#### 8.6.16.1	Message definition

The DEREGISTRATION ACCEPT message is sent by the network to the UE.

Message type:	DEREGISTRATION ACCEPT

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the DEREGISTRATION ACCEPT message is FFS.

### 8.6.17	Deregistration request (UE terminated deregistration)

#### 8.6.17.1	Message definition

The DEREGISTRATION REQUEST message is sent by the UE to the network.

Message type:	DEREGISTRATION REQUEST

Significance:		dual

Direction:			network to UE

Editor's note:	 The content of the DEREGISTRATION REQUEST message is FFS.

### 8.6.18	Deregistration accept (UE terminated deregistration)

#### 8.6.18.1	Message definition

The DEREGISTRATION ACCEPT message is sent by the network to the UE.

Message type:	DEREGISTRATION ACCEPT

Significance:		dual

Direction:			UE to network

Editor's note:	 The content of the DEREGISTRATION ACCEPT message is FFS.

### 8.6.19	Configuration update command

#### 8.6.19.1	Message definition

The CONFIGURATION UPDATE COMMAND message is sent by the network to the UE.

Message type:	CONFIGURATION UPDATE COMMAND

Significance:		dual

Direction:			network to UE

# Table 8.6.19.1.1: CONFIGURATION UPDATE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | ½ |
|  | Configuration update command message type | Message type 6.6.6.6 | M | V | 1 |
|  | Acknowledgement requested flag | Acknowledgement requested flag 8.x1.y1 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | 5G-GUTI | 5GS mobile identity 8.x1.y2 | O | TLV | TBD |
|  | TAI list | Tracking area identity list 8.x1.y3 | O | TLV | 8-98 |
|  | Mobility restrictions | Mobility restrictions 8.x1.y4 | O | TLV | TBD |
|  | Full name for network | Network name 8.x1.y5 | O | TLV | 3-n |
|  | Short name for network | Network name 8.x1.y5 | O | TLV | 3-n |
|  | Local time zone | Time zone 8.x1.y6 | O | TV | 2 |
|  | Universal time and local time zone | Time zone and time 8.x1.y7 | O | TV | 8 |
|  | Network daylight saving time | Daylight saving time 8.x1.y8 | O | TLV | 3 |

Editor's note:	 The further contents of the CONFIGURATION UPDATE COMMAND message is FFS.

### 8.6.20	Configuration update complete

#### 8.6.20.1	Message definition

The CONFIGURATION UPDATE COMPLETE message is sent by the UE to the network.

Message type:	CONFIGURATION UPDATE COMPLETE

Significance:		dual

Direction:			UE to network

# Table 8.6.20.1.1: CONFIGURATION UPDATE COMPLETE message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | Security header type | Security header type 6.6.6.3 | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | ½ |
|  | Configuration update complete message type | Message type 6.6.6.6 | M | V | 1 |

Editor's note:	 The further contents of the CONFIGURATION UPDATE COMPLETE message is FFS.

### 8.6.21	Identity request

#### 8.6.21.1	Message definition

The IDENTITY REQUEST message is sent by the network to the UE to request the UE to provide specified identity.

# Message type:	IDENTITY REQUEST

Significance:		dual

Direction:			AMF to UE

Editor's note: 	The content of IDENTITY REQUEST message is FFS.

### 8.6.22	Identity response

#### 8.6.22.1	Message definition

The IDENTITY RESPONSE message is sent by the UE to the AMF to provide the requested identity.

# Message type:	IDENTITY RESPONSE

Significance:		dual

Direction:			UE to AMF

Editor's note: 	The content of IDENTITY RESPONSE message is FFS.

## 8.7	5GMM information elements

### 8.7.1	5GMM cause

The purpose of the 5GMM cause information element is to indicate the reason why an 5GMM request from the UE is rejected by the network.

The 5GMM cause information element is coded as shown in figure 8.9.3.9.1 and table 8.9.3.9.1.

The 5GMM cause is a type 3 information element with 2 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GMM cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

# Figure 8.7.1.1: 5GMM cause information element

Table 8.7.1.1: 5GMM cause information element

Editor's note:	The listing of 5GMM cause values is FFS..

### 8.7.2	NAS key set identifier

The NAS key set identifier is allocated by the network.

Editor's note:	The definition of NAS key set identifier is FFS.

### 8.7.3	Authentication parameter RAND

See subclause 10.5.3.1 in 3GPP TS 24.008 [11].

### 8.7.4	Authentication parameter AUTN

See subclause 10.5.3.1.1 in 3GPP TS 24.008 [11].

### 8.7.5	SM message container (for alternative 1)

The purpose of the SM message container information element is to transport an 5GSM message within an 5GMM message.

The SM message container information element is coded as shown in figure 8.7.5.1 and table 8.7.5.1.

The SM message container is a type 6 information element with a minimum length of 4 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM message container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of SM message container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| SM message container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

# Figure 8.7.5.1: SM message container information element

Table 8.7.5.1: SM message container information element

SM message container contents (octet 4 to octet n); Max value of 65535 octets

This IE can contain any 5GSM PDU as defined in subclause 9.5.

### 8.7.6	Request type (for alternative 1)

The purpose of the request type information element is to indicate type of the PDU session establishment.

The request type information element is coded as shown in figure 8...1 and table 8.7.6.1.

The request type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Request type IEI |  |  |  | Request type value |  |  |  | Octet 1 |

# Figure 8.7.6.1: Request type information element

Table 8.7.6.1: Request type information element

| Request type value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | initial request |
| 0 | 0 | 1 | 0 | existing PDU session |
| All other values are reserved. |  |  |  |  |

### 8.7.7	S-NSSAI (for alternative 1)

The purpose of the S-NSSAI information element is to identify a network slice.

The S-NSSAI information element is coded as shown in figure 8.7.7.1 and table 8.7.7.1.

The S-NSSAI is a type 4 information element with 3 octets length or with 6 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| S-NSSAI IEI |  |  |  |  |  |  |  | octet 1 |
| Length of S-NSSAI contents |  |  |  |  |  |  |  | octet 2 |
| SST |  |  |  |  |  |  |  | octet 3 |
| SD |  |  |  |  |  |  |  | octet 4 octet 6 |

Figure 8.7.7.1: S-NSSAI information element

Table 8.7.7.1: S-NSSAI information element

Slice/service type (SST) (octet 1)

Slice differentiator (SD) (octet 4 to octet 6)

### 8.7.8	DNN (for alternative 1)

The purpose of the DNN information element is to identify the data network.

The DNN information element is coded as shown in figure 8.7.8.1.

The DNN is a type 4 information element with a minimum length of 3 octets and a maximum length of 102 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DNN IEI |  |  |  |  |  |  |  | octet 1 |
| Length of DNN contents |  |  |  |  |  |  |  | octet 2 |
| DNN value |  |  |  |  |  |  |  | octet 3 octet n |

Figure 8.7.8.1: DNN information element

A DNN value field contains an APN as defined in 3GPP TS 23.003 [5].

### 8.7.9	Message container type (for alternative 1)

The purpose of the message container type information element is to indicate type of payload in the message container.

The message container type information element is coded as shown in figure 8.7.9.1 and table 8.7.9.1.

The message container type is a type 3 information element with 2 octet length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message container type IEI |  |  |  |  |  |  |  | Octet 1 |
| Message container type |  |  |  |  |  |  |  | Octet 2 |

# Figure 8.7.9.1: Message container type information element

Table 8.7.9.1: Message container type information element

| Message container type (octet 2) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |  |
| 8 |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | SMS |
| All other values are reserved. |  |  |  |  |  |  |  |  |  |  |

### 8.7.10	Message container (for alternative 1)

The purpose of the message container information element is to transport a payload indicated by the message container type information element.

The message container information element is coded as shown in figure 8.7.10.1 and table 8.7.10.1.

The message container is a type 6 information element with a minimum length of 3 octets and a maximum length of 65538 octets.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message container IEI |  |  |  |  |  |  |  | octet 1 |
| Length of message container contents |  |  |  |  |  |  |  | octet 2 |
|  |  |  |  |  |  |  |  | octet 3 |
|  |  |  |  |  |  |  |  | octet 4 |
| Message container contents |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | octet n |

# Figure 8.7.10.1: Message container information element

Table 8.7.10.1: Message container information element

Message container contents (octet 4 to octet n); Max value of 65535 octets

### 8.7.11	Acknowledgement requested flag

The purpose of the acknowledgement requested flag information element is to indicate indicate to the receiver whether the sender expects an acknowledgement message or not.

The acknowledgement requested flag information element is coded as shown in figure 8.7.11.1 and table 8.7.11.1.

The acknowledgement requested flag is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Acknowledgement requested flag IEI |  |  |  | 0 Spare | 0 Spare | 0 Spare | ACK | octet 1 |

# Figure 8.7.11.1: Acknowledgement requested flag

Table 8.7.11.1: Acknowledgement requested flag

| Acknowledgement (ACK) value (octet 1, bit 1) |  |
| --- | --- |
| Bits |  |
| 1 |  |
| 0 | acknowledgement not requested |
| 1 | acknowledgement requested |

### 8.7.12	5GS mobile identity

The purpose of the 5GS mobile identity information element is to provide either the IMSI, the 5G-GUTI or the IMEI.

The 5GS mobile identity information element is coded as shown in figures 8.7.12.1 and 8.7.12.2 and table 8.x1.y2.1.

The 5GS mobile identity is a type 4 information element with a minimum length of xx1 octets and a maximum length of xx2 octets.

Editor's note:	 The definition of 5G-GUTI is FFS, and will be added when defined by CT4.

# Figure 8.7.12.1: 5GS mobile identity information element for type of identity "5G-GUTI"

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GS mobile identity IEI |  |  |  |  |  |  |  | octet 1 |
| Length of 5GS mobile identity contents |  |  |  |  |  |  |  | octet 2 |
| Identity digit 1 |  |  |  | odd/ even indic | Type of identity |  |  | octet 3 |
| Identity digit p+1 |  |  |  | Identity digit p |  |  |  | octet 4* |

# Figure 8.7.12.2: 5GS mobile identity information element for type of identity "IMSI" or "IMEI"

Table 8.7.12.1: 5GS mobile identity information element

| Type of identity (octet 3) Bits |  |  |  |
| --- | --- | --- | --- |
| 3 | 2 | 1 |  |
| 0 | 0 | 1 | IMSI |
| 1 | 1 | 0 | 5G-GUTI |
| 0 | 1 | 1 | IMEI |
| All other values are reserved. |  |  |  |
| Odd/even indication (octet 3) Bit |  |  |  |
| 4 |  |  |  |
| 0 |  |  | even number of identity digits |
| 1 |  |  | odd number of identity digits |
| Identity digits (octet 4 etc) For the IMSI, this field is coded using BCD coding. If the number of identity digits is even then bits 5 to 8 of the last octet shall be filled with an end mark coded as "1111". |  |  |  |
| MCC, Mobile country code (octet 4, octet 5 bits 1 to 4) The MCC field is coded as in ITU-T Recommendation E.212 [27], annex A. |  |  |  |
| MNC, Mobile network code (octet 5 bits 5 to 8, octet 6) The coding of this field is the responsibility of each administration but BCD coding shall be used. The MNC shall consist of 2 or 3 digits. If a network operator decides to use only two digits in the MNC, bits 5 to 8 of octet 5 shall be coded as "1111". The contents of the MCC and MNC digits are coded as octets 6 to 8 of the Temporary Mobile Group Identity IE in figure 10.5.154 of 3GPP TS 24.008 [11]. |  |  |  |
| For the IMEI, this field is coded using BCD coding. The format of the IMEI is described in 3GPP TS 23.003 [5]. |  |  |  |

### 8.7.13	Tracking area identity list

See subclause 9.9.3.33 in 3GPP TS 24.301 [12].

### 8.7.14	Mobility restrictions

Editor's note:	 The definition of Mobility restrictions IE is FFS.

### 8.7.15	Network name

See subclause 10.5.3.5a in 3GPP TS 24.008 [11].

### 8.7.16	Time zone

See subclause 10.5.3.8 in 3GPP TS 24.008 [11].

### 8.7.17	Time zone and time

See subclause 10.5.3.9 in 3GPP TS 24.008 [11].

### 8.7.18	Daylight saving time

See subclause 10.5.3.12 in 3GPP TS 24.008 [11].

## 8.8	3GPP specific coding information defined within present document

### 8.8.1	Serving network name (SNN)

The serving network name (SNN) is used in the network name field of the AT_KDF_INPUT attribute defined in IETF RFC 5448 [25].

SNN shall take the generic format of an octet string without terminating null characters.

SNN is of maximum length of 1020 octets.

SNN consists of SNN-prefix and SNN-value, delimited by a colon.

ABNF syntax of SNN is specified in table 8.8.1.1

Table 8.8.1.1: ABNF syntax of SNN

SNN = SNN-prefix ":" SNN-value

SNN-prefix = %x35.47 ; "5G"

SNN-value = TBD

NOTE:	SNN-prefix allows for distinguishing of ANID specified in 3GPP TS 24.302 [13] and SNN as either of SNN or ANID can be carried in the AT_KDF_INPUT attribute.

Editors' note: 	Content of SNN-value depends on SA3 decisions and is FFS.

Editors' note: 	3GPP TS 24.302 [13]  needs to be extended so that ANID-prefix "5G" points to the SNN.

## 8.9	Timers of 5GS mobility management

Timers of 5GS mobility management are shown in table 8.9.1 and table 8.9.2

# Table 8.9.1: Timers of 5GS mobility management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Ta | TBD | TBD | Transmission of REGISTRATION REQUEST message. | REGISTRATION ACCEPT message received or REGISTRATION REJECT message received. | Retransmission of REGISTRATION REQUEST message. |
| Tb | TBD | TBD | At registration failure and the attempt counter is equal to 5. | REGISTRATION REQUEST sent | Initiation of the registration procedure, if still required |
| Tc | TBD | TBD |  | REGISTRATION REQUEST sent | Retransmission of the REGISTRATION REQUEST, if still required |
| Te | TBD | TBD | In 5MM-REGISTERED, when 5MM-CONNECTED mode is left. | When entering state 5MM-DEREGISTERED or when entering 5MM-CONNECTED mode. | Initiation of the periodic registration procedure |
| Txa | TBD | TBD | Transmission of DEREGISTRATION REQUEST message when deregistration procedure is not due to a "switch off". | DEREGISTRATION ACCEPT message received. | Retransmission of DEREGISTRATION REQUEST message. |

# Table 8.9.2: Timers of 5GS mobility management – AMF side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Td | TBD | TBD | REGISTRATION ACCEPT message sent at initial registration. REGISTRATION ACCEPT message sent with 5G-GUTI at mobility or periodic registration. | REGISTRATION COMPLETE received | Retransmission of REGISTRATION ACCEPT message. |
| Tf | TBD | TBD | IDENTITY REQUEST sent | IDENTITY RESPONSE received | Retransmission of IDENTITY REQUEST |
| Tya | TBD | TBD | Transmission of DEREGISTRATION REQUEST message. | DEREGISTRATION ACCEPT received. | Retransmission of DEREGISTRATION REQUEST message. |
| Implicit deregistration timer | NOTE 1 | All except 5GMM-DEREGISTERED | Entering 5GMM-IDLE mode | N1 NAS signalling connection established | Network dependent, but typically paging is halted on 1st expiry if the UE is not registered for emergency bearer services. Implicitly detach the UE which is registered for emergency bearer services. Start implicit detach timer |
| Implicit detach timer | NOTE 2 | All except 5GMM-DEREGISTERED | The implicit deregistration timer expires while the network is in 5GMM-IDLE mode | N1 NAS signalling connection established | Implicitly detach the UE on 1st expiry |
| NOTE 1: The default value of this timer is 4 minutes greater than Te. If the UE is attached for emergency bearer services, the value of this timer is set equal to Te. NOTE 2: The value of this timer is network dependent. . If MICO is activated, the default value of this timer is 4 minutes greater than Te. |  |  |  |  |  |

## 8.10	Cause values for 5GS mobility management

### 8.10.1	Causes related to UE identification

Editor’s note:	5GMM cause values related to UE identification need to be added to this subclause.

### 8.10.2	Cause related to subscription options

Editor’s note:	5GMM cause values related to subscription options need to be added to this subclause.

### 8.10.3	Causes related to PLMN specific network failures and congestion/authentication failures

Editor’s note:	5GMM cause values related to PLMN specific network failures and congestion/authentication failures need to be added to this subclause.

### 8.10.4	Causes related to invalid messages

<Cause value #95 – Semantically incorrect message.

This 5GMM cause is used to report receipt of a message with semantically incorrect contents.

Cause value #96 – Invalid mandatory information.

This cause 5GMM indicates that the equipment sending this 5GMM cause has received a message with a non-semantical mandatory IE error.

Cause value #97 – Message type non-existent or not implemented.

This 5GMM cause indicates that the equipment sending this 5GMM cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this 5GMM cause.

Cause value #98 – Message type not compatible with protocol state.

This 5GMM cause indicates that the equipment sending this 5GMM cause has received a message not compatible with the protocol state.

Cause value #99 – Information element non-existent or not implemented.

This 5GMM cause indicates that the equipment sending this 5GMM cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the 5GMM cause. However, the information element is not required to be present in the message in order for the equipment sending the 5GMM cause to process the message.

Cause value #100 – Conditional IE error.

This 5GMM cause indicates that the equipment sending this cause has received a message with conditional IE errors.

Cause value #101 – Message not compatible with protocol state.

This 5GMM cause indicates that a message has been received which is incompatible with the protocol state.

Cause value #111 – Protocol error, unspecified.

This 5GMM cause is used to report a protocol error event only when no other 5GMM cause in the protocol error class applies.

# 9	5GS session management

Editor’s note:	The names of messages are informational and used for work in progress. Final 5GSM names are TBD.

## 9.1	General

The 5GS architecture reference model defined in 3GPP TS 23.501 [8] shows that the N11 reference point is used to connect the core access and mobility management function (AMF) and the session management function (SMF). The SMF handles the session management part of the NAS signalling exchanged with the UE. 3GPP TS 23.502 [9] specifies that session management procedures (e.g., PDU session establishment) are not performed between the AMF and the SMF but the UE and the SMF.

CT1 will specify the control-plane 5G session management (5GSM) towards the UE and the SMF. This includes the procedures, messages, information elements and the necessary error handling for the session management part of the NAS signalling.

## 9.2	Overview

### 9.2.1	PDU session types

The following PDU Session types are supported:

-	IPv4 type;

-	IPv6 type;

-	Ethernet type (EtherType as defined in IEEE 802.3); and

-	unstructured type.

IP Address Allocation for IPv4 and IPv6 PDU session types is described in subclause 8.3.

Neither a MAC nor an IP address is allocated by the 5GC to the UE for Ethernet PDU session type.

### 9.2.2	PDU session management

The SMF is responsible for the session management functions to provide the PDU connectivity service to the UE via the SM signalling between UE and SMF.  The session management procedures includes:

a)	the UE-requested PDU session establishment procedure;

b)	the PDU session establishment authentication and authorization procedure;

c)	the UE-requested PDU session modification procedure;

d)	the network-requested PDU session modification procedure; and

e)	the UE-requested PDU session release procedure; and

f)	the network-requested PDU session release procedure.

A UE may establish multiple PDU sessions, to the same data network or to different data networks, via 3GPP and via and Non-3GPP access networks at the same time.

The session management messages between UE and SMF are transferred via AMF as specified in subclause 9.5.

### 9.2.3	Local area data network

The UE can receive the Local area data network (LADN) availability information IE consisting of LADN DNN and LADN service area (a set of tracking areas that belong to the current registration area) information based on the UE location during the registration procedure (see 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9]).

The UE may request a PDU session establishment for an available LADN when the UE is located in the LADN service area. If the UE has moved out of the LADN service area, the PDU session associated with the LADN may be deactivated or released by the SMF.

NOTE:	In this release, LADNs apply only to 3GPP accesses.

## 9.3	5GSM sublayer states

## 9.3.1	General

In the following subclauses, the possible states of a 5G PDU session context in the UE and the network side are described.

Editor's note:	The procedure transaction states for 5GSM are FFS.

### 9.3.2	5GSM sublayer states in the UE

#### 9.3.2.1	Overview

In the following subclauses, the possible 5GSM sublayer states of the UE are described and shown in figure 9.3.2.1.1.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GSM procedure are FFS.

Editor's note:	Whether the PDU session establishment authentication and authorization procedure impacts the state transition is FFS.

# Figure 9.3.2.1.1: The 5GSM sublayer states for PDU session handling in the UE (overview)

#### 9.3.2.2	PDU SESSION INACTIVE

No PDU session context exists.

#### 9.3.2.3	PDU SESSION ACTIVE PENDING

The UE has initiated a PDU session establishment procedure towards the network and is waiting for a response from the network.

#### 9.3.2.4	PDU SESSION ACTIVE

The PDU session context is active in the UE.

#### 9.3.2.5	PDU SESSION INACTIVE PENDING

The UE has initiated a PDU session release procedure towards the network and is waiting for a response from the network.

#### 9.3.2.6	PDU SESSION MODIFICATION PENDING

The UE has initiated a PDU session modification procedure towards the network and is waiting for a response from the network.

#### 9.3.2.7	PDU SESSION ACTIVE PENDING

The UE has initiated a PDU session establishment procedure towards the network and is waiting for a response from the network.

#### 9.3.2.8	PDU SESSION ACTIVE

The PDU session context is active in the UE.

#### 9.3.2.9	PDU SESSION INACTIVE PENDING

The UE has initiated a PDU session release procedure towards the network and is waiting for a response from the network.

#### 9.3.2.10	PDU SESSION MODIFICATION PENDING

The UE has initiated a PDU session modification procedure towards the network and is waiting for a response from the network.

### 9.3.3	5GSM sublayer states in the network

#### 9.3.3.1	Overview

In the following subclauses, the possible 5GSM sublayer states of the network are described and shown in Figure 9.3.3.1.1.

# NOTE:	Not all possible transitions are shown in this figure.

Editor's note:	The fatal causes and non-fatal causes used in the 5GSM procedure are FFS.

Editor's note:	Whether the PDU session establishment authentication and authorization procedure impacts the state transition is FFS.

Figure 9.3.3.1.1: The 5GSM sublayer states for PDU session handling in the network (overview)

##### 9.3.3.2	PDU SESSION INACTIVE

No PDU session context exists.

##### 9.3.3.3	PDU SESSION ACTIVE

The PDU session context is active in the network.

##### 9.3.3.4	PDU SESSION INACTIVE PENDING

The network has initiated a PDU session release procedure towards the UE and is waiting for a response from the UE.

##### 9.3.3.5	PDU SESSION MODIFICATION PENDING

The network has initiated a PDU session modification procedure towards the UE and is waiting for a response from the UE.

## 9.4	5GS session management procedures

### 9.4.1	General

#### 9.4.1.1	Principles of PTI handling for 5GSM procedures

When the UE or the network initiates a transaction related procedure (i.e. a procedure consisting of more than one message and the messages are related), it shall include a valid PTI value in the message header of the request message or of the command message.

If a response message is sent as result of a received request message or a received command message, the sending entity shall include in the response message the PTI value received within the request message or within the command message (see examples in figure 9.4.1.1.1, figure 9.4.1.1.2, figure 9.4.1.1.3, and figure 9.4.1.1.4).

If a command message is sent as result of a received request message, the sending entity shall include in the command message the PTI value received with the request message (see examples in figure 9.4.1.1.3).

If a command message is not sent as result of a received request message, the sending entity shall include in the command message the PTI value set to "no procedure transaction identity assigned" (see examples in figure 9.4.1.1.3).

# Figure 9.4.1.1.1: UE-requested transaction related procedure accepted by the network

# Figure 9.4.1.1.2: UE-requested transaction related procedure rejected by the network

Figure 9.4.1.1.3: UE-requested transaction related procedure triggering a network-requested transaction related procedure

# Figure 9.4.1.1.4: network-requested transaction related procedure not triggered by a UE-requested transaction related procedure

### 9.4.2	UE-requested PDU session establishment procedure

#### 9.4.2.1	General

The purpose of the UE-requested PDU session establishment procedure is to establish a new PDU session with a DN or to perform handover of an existing PDU session between 3GPP access and non-3GPP access or to transfer an existing PDN connection in the EPS to the 5GS. If accepted by the network, the PDU session enables exchange of PDUs between the UE and the DN. The UE shall not request a PDU session establishment for an LADN when the UE is located outside the LADN service area.

#### 9.4.2.2	UE-requested PDU session establishment procedure initiation

In order to initiate the UE-requested PDU session establishment procedure, the UE shall create a PDU SESSION ESTABLISHMENT REQUEST message.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION ESTABLISHMENT REQUEST message to the allocated PTI value.

If the UE requests to establish a new PDU session with a DN and requests a PDU session type, the UE shall set the PDU session type IE of the PDU SESSION ESTABLISHMENT REQUEST message to the IP version capability as specified in subclause 8.4.2, "ethernet" or "unstructured".

If the UE requests to establish a new PDU session with a DN and the UE requests an SSC mode, the UE shall set the SSC mode IE of the PDU SESSION ESTABLISHMENT REQUEST message to the SSC mode.

If the UE requests to establish a new PDU session with a DN and needs to provide information for the PDU session authentication and authorization by the external DN, the UE shall include the PDU DN request container with the information for the PDU session authentication and authorization by the external DN.

Editor's note:	FFS whether CT1 need to give more details about coding of the information for the PDU session authentication and authorization by the external DN.

If the UE has an emergency PDU session, the UE shall not perform the UE-requested PDU session establishment procedure is to establish another emergency PDU session.

The UE shall transport:

a)	the PDU SESSION ESTABLISHMENT REQUEST message;

b)	the PDU session ID:

1)	of the existing PDU session, if the UE initiates the UE-requested PDU session establishment procedure upon receiving the PDU SESSION MODIFICATION COMMAND messages with the 5GSM cause IE set to #39 "reactivation requested"; and

2)	of the PDU session being established, otherwise;

c)	the S-NSSAI, if the UE requests a S-NSSAI other than default S-NSSAI;

d)	and the requested DNN, if the UE requests a connectivity to a DNN other than the default DNN; and

e)	the request type set:

1)	to "initial request", if the UE requests to establish a new non-emergency PDU session;

2)	to "existing PDU session", if the UE requests:

i)	handover of an existing non-emergency PDU session between 3GPP access and non-3GPP access; or

ii)	transfer of an existing PDN connection in the EPS to the 5GS. and

3)	to "initial emergency request", if the UE requests to establish a new emergency PDU session;

using the UE-initiated SM message transport procedure as specified in subclause 8.4.2.2.1.1 or the UE-initiated NAS transport procedure as specified in subclause 8.5.1.1.3.2, and the UE shall start timer Tx (see example in figure 9.4.2.2.1).

# Figure 9.4.2.2.1: UE-requested PDU session establishment procedure

Upon receipt of a PDU SESSION ESTABLISHMENT REQUEST message, a PDU session ID, optionally a S-NSSAI, and optionally a DNN, and the request type, the SMF checks whether connectivity with the requested DN can be established. If the requested DNN is not included, the SMF shall use the default DNN. If the S-NSSAI is not included, the SMF may determine a default S-NSSAI according to the subscription context from UDM and, if determined, the SMF may use the default S-NSSAI as the S-NSSAI.

If the PDU session authentication and authorization by the external DN is required due to local policy, the SMF shall check whether the PDU SESSION ESTABLISHMENT REQUEST message includes the PDU DN request container.

If the PDU DN request container is included in the PDU SESSION ESTABLISHMENT REQUEST message and the PDU session authentication and authorization by the external DN is required due to local policy and user's subscription data, the SMF shall:

-	if the information for the PDU session authentication and authorization by the external DN in PDU DN request container is compliant with the local policy and user's subscription data, proceed with the EAP Authentication procedure specified in 3GPP TS 33.501 [15] and refrain from accepting or rejecting the PDU SESSION ESTABLISHMENT REQUEST message until the EAP Authentication procedure finalizes; and

-	if the information for the PDU session authentication and authorization by the external DN in PDU DN request container is not compliant with the local policy, reject the PDU session establishment request including the 5GSM cause #xx "DN authentication failed", in the PDU SESSION ESTABLISHMENT REJECT message.

If the PDU DN request container is not included in the PDU SESSION ESTABLISHMENT REQUEST message and the PDU session authentication and authorization by the external DN is required due to local policy and user's subscription data, the SMF shall proceed with the EAP Authentication procedure specified in 3GPP TS 33.501 [15] and refrain from accepting or rejecting the PDU SESSION ESTABLISHMENT REQUEST message until the EAP Authentication procedure finalizes.

If the PDU session ID in the PDU session ID IE of the PDU SESSION ESTABLISHMENT REQUEST message is different from the PDU session ID received from the AMF and a PDU session exists for the PDU session ID received from the AMF, the SMF shall consider that the request for the PDU session anchor relocation with multiple PDU sessions for SSC mode 3 as specified in 3GPP TS 23.502 [9] is accepted by the UE.

#### 9.4.2.3	UE-requested PDU session establishment procedure accepted by the network

If the connectivity with the requested DN is accepted by the network, the SMF shall create a PDU SESSION ESTABLISHMENT ACCEPT message.

The SMF shall set the authorized QoS rules IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the authorized QoS rules of the PDU session. If the received request type is "initial emergency request", the SMF shall set the authorized QoS rules IE according to the initial QoS parameters used for establishing emergency services configured in the SMF Emergency Configuration Data.

The SMF shall set the selected SSC mode IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected SSC mode of the PDU session.

The SMF shall set the selected S-NSSAI IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected S-NSSAI of the PDU session.

The SMF shall set the selected PDU session type IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected PDU session type of the PDU session.

If the PDU SESSION ESTABLISHMENT REQUEST message includes a PDU session type IE set to "IP", the SMF shall select either "IPv4" or "IPv6" as the selected PDU session type IE of the PDU session. If the subscription, the SMF configuration, or both, are limited to IPv4 only or IPv6 only for the requested DNN, the SMF shall include the 5GSM cause value #50 "PDU session type IPv4 only allowed", or #51 "PDU session type IPv6 only allowed", respectively, in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT ACCEPT message.

If the selected PDU session type is "IPv4", the SMF shall include the PDU address IE in the PDU SESSION ESTABLISHMENT ACCEPT message and shall set the PDU address IE to an IPv4 address is allocated to the UE in the PDU session.

The SMF shall set the selected DNN IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the selected DNN of the PDU session.

The SMF shall set the selected Session-AMBR IE of the PDU SESSION ESTABLISHMENT ACCEPT message to the Session-AMBR of the PDU session.

The SMF may include the RQ timer IE set to an RQ timer value.

The SMF shall send the PDU SESSION ESTABLISHMENT ACCEPT message.

Upon receipt of a PDU SESSION ESTABLISHMENT ACCEPT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.51.1.2.1.3, the UE shall stop timer Tx, shall release the allocated PTI value and shall consider that the PDU session was established.

If the UE requests the PDU session type "IP" and:

a)	the UE receives the selected PDU session type set to "IPv4" and does not receive the 5GSM cause value #50 "PDU session type IPv4 only allowed"; or

b)	 the UE receives the selected PDU session type set to "IPv6" and does not receive the 5GSM cause value #51 "PDU session type IPv6 only allowed";

the UE may subsequently request another PDU session for the other IP version using the UE-requested PDU session establishment procedure to the same DNN with a single address PDN type (IPv4 or IPv6) other than the one already activated.

If the UE requests the PDU session type "IP", receives the selected PDU session type set to "IPv4" and the 5GSM cause value #50 "PDU session type IPv4 only allowed", the UE shall not subsequently request another PDU session for "IPv6" using the UE-requested PDU session establishment procedure to the same DNN and the PDU session type "IPv6" until the PDU session is released.

If the UE requests the PDU session type "IP", receives the selected PDU session type set to "IPv6" and the 5GSM cause value #51 "PDU session type IPv6 only allowed", the UE shall not subsequently request another PDU session for "IPv4" using the UE-requested PDU session establishment procedure to the same DNN and the PDU session type "IPv4" until the PDU session is released.

#### 9.4.2.4	UE requested PDU session establishment procedure not accepted by SMF

If the connectivity with the requested DN is rejected by the network, the SMF shall create a SM PDU SESSION ESTABLISHMENT REJECT message.

If the UE requests a PDU session establishment for an LADN when the UE is located outside the LADN service area, the SMF shall reject the request.

The SMF shall set the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message to indicate the reason for rejecting the PDU session establishment.

The 5GSM cause IE typically indicates one of the following SM cause values:

#31:	request rejected, unspecified;

#50	PDU session type IPv4 only allowed; or

#51	PDU session type IPv6 only allowed.

#xx:	DN authentication failed;

Editor's note:	Further 5GSM causes are FFS.

Editor's note:	Further contents of the SM PDU SESSION ESTABLISHMENT REJECT are FFS.

If the PDU SESSION ESTABLISHMENT REQUEST message includes a PDU session type IE set to "IPv6", and the subscription, the SMF configuration, or both, are limited to IPv4 only for the requested DNN, the SMF shall include the 5GSM cause value #50 "PDU session type IPv4 only allowed" in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message.

If the PDU SESSION ESTABLISHMENT REQUEST message includes a PDU session type IE set to "IPv4", and the subscription, the SMF configuration, or both, are limited to IPv6 only for the requested DNN, the SMF shall include the 5GSM cause value #51 "PDU session type IPv6 only allowed" in the 5GSM cause IE of the PDU SESSION ESTABLISHMENT REJECT message.

The SMF shall send the SM PDU SESSION ESTABLISHMENT REJECT message.

Upon receipt of a PDU SESSION ESTABLISHMENT REJECT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.3, the UE shall stop timer Tx shall release the allocated PTI value and shall consider that the PDU session was not established.

#### 9.4.2.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Tx expired

Editor's note:	Further abnormal cases in the UE are FFS.

#### 9.4.2.6	Abnormal cases on the network side

Editor's note:	Further abnormal cases in the network side are FFS.

The following abnormal cases can be identified:

a)	If the received request type is "initial emergency request" and there is already another emergency PDU session for the UE, the SMF shall reject the PDU SESSION ESTABLISHMENT REQUEST message with ESM cause #31 "request rejected, unspecified".

x)	The information for the PDU session authentication and authorization by the external DN in PDU DN request container is not compliant with local policy and user's subscription data

If the PDU session authentication and authorization by the external DN is required due to local policy and user's subscription data and the information for the PDU session authentication and authorization by the external DN in PDU DN request container is not compliant with local policy and user's subscription data, the SMF shall reject the PDU session establishment request including the 5GSM cause #xx "DN authentication failed", in the PDU SESSION ESTABLISHMENT REJECT message.

### 9.4.3	PDU session establishment authentication and authorization procedure

#### 9.4.3.1	General

The purpose of the PDU session establishment authentication and authorization procedure is to enable the DN:

1)	to authenticate the UE establishing the PDU session;

2)	to authorize the UE establishing the PDU session; or

3)	both of the above.

The PDU session establishment authentication and authorization procedure can be performed only during the UE-requested PDU session establishment procedure.

Editor's note:	FFS whether a dedicated messages for PDU session establishment authentication and authorization procedure are needed or whether some kind of a generic SM message can be defined and used.

#### 9.4.3.2	PDU session establishment authentication and authorization procedure initiation

In order to initiate the PDU session establishment authentication and authorization procedure, the SMF shall create a PDU SESSION AUTHENTICATION REQUEST message.

The SMF shall set the authentication information IE of the PDU SESSION AUTHENTICATION REQUEST message to the authentication information provided by the DN.

Editor's note:	Further contents of the PDU SESSION AUTHENTICATION REQUEST message are FFS.

The SMF shall send the PDU SESSION AUTHENTICATION REQUEST message, and the SMF shall start timer Tm (see example in figure 9.4.3.2.1).

# Figure 9.4.3.2.1: PDU session establishment authentication and authorization procedure

Upon receipt of a PDU SESSION AUTHENTICATION REQUEST message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.3, the UE passes the authentication information IE of the PDU SESSION AUTHENTICATION REQUEST message to the upper layers.

#### 9.4.3.3	PDU session establishment authentication and authorization procedure accepting

If the upper layers provide a responding authentication information, and the UE shall create a PDU SESSION AUTHENTICATION ACCEPT message.

The UE shall set the authentication information IE of the PDU SESSION AUTHENTICATION ACCEPT message to the responding authentication information.

Editor's note:	Further contents of the PDU SESSION AUTHENTICATION ACCEPT message are FFS.

The UE shall transport the PDU SESSION AUTHENTICATION ACCEPT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.

Upon receipt of a PDU SESSION AUTHENTICATION ACCEPT message, the SMF shall stop timer Tm and provides the authentication information to the DN.

#### 9.4.3.4	PDU session establishment authentication and authorization procedure rejecting

If the upper layers does not accept or does not provide a responding authentication information, the UE shall create a PDU SESSION AUTHENTICATION REJECT message.

The UE shall set the SM cause IE of the PDU SESSION AUTHENTICATION REJECT message to indicate the reason for rejecting the PDU SESSION AUTHENTICATION REQUEST message.

The SM cause IE typically indicates one of the following SM cause values:

Editor's note:	SM causes are FFS.

Editor's note:	Further contents of the PDU SESSION AUTHENTICATION REJECT message are FFS.

The UE shall transport the PDU SESSION AUTHENTICATION REJECT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.2.2.1.1.

Upon receipt of a PDU SESSION AUTHENTICATION REJECT message, the SMF shall stop timer Tm.

#### 9.4.3.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Tm expired

Editor's note:	Further abnormal cases are FFS

#### 9.4.3.6	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS

### 9.4.4	UE-requested PDU session modification procedure

#### 9.4.4.1	General

The purpose of the UE-requested PDU session modification procedure is to enable the UE to request modification of a PDU session. The UE shall not request a PDU session modification for an LADN when the UE is located outside the LADN service area.

#### 9.4.4.2	UE-requested PDU session modification procedure initiation

In order to initiate the UE-requested PDU session modification procedure, the UE shall create a PDU SESSION MODIFICATION REQUEST message.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION MODIFICATION REQUEST message to the allocated PTI value.

The UE shall not perform the UE-requested PDU session modification procedure for an emergency PDU session.

The UE shall transport the PDU SESSION MODIFICATION REQUEST message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1, and the UE shall start timer Tk (see example in figure 9.4.4.2.1).

# Figure 9.4.4.2.1: UE-requested PDU session modification procedure

#### 9.4.4.3	UE-requested PDU session modification procedure accepting

Upon receipt of a PDU SESSION MODIFICATION REQUEST message, if the SMF accepts the request to modify the PDU session, the SMF shall perform the network-requested PDU session modification procedure as specified in subclause 9.4.4.

#### 9.4.4.4	UE-requested PDU session modification procedure not accepted by the network

Upon receipt of a PDU SESSION MODIFICATION REQUEST message, if the SMF does not accepts the request to modify the PDU session, the SMF shall create a PDU SESSION MODIFICATION REJECT message.

If the UE requests a PDU session modification for an LADN when the UE is located outside the LADN service area, the SMF shall reject the request.

The SMF shall set the 5GSM cause IE of the PDU SESSION MODIFICATION REJECT message to indicate the reason for rejecting the PDU session modification.

The 5GSM cause IE typically indicates one of the following SM cause values:

#31:	request rejected, unspecified;

Editor's note:	Further 5GSM causes are FFS.

The SMF shall send the PDU SESSION MODIFICATION REJECT message.

Upon receipt of a PDU SESSION MODIFICATION REJECT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.3, the UE shall stop timer Tk, shall release the allocated PTI value and shall consider that the PDU session is not modified.

#### 9.4.4.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Tk expired

Editor's note:	Further abnormal cases are FFS.

#### 9.4.4.6	Abnormal cases on the network side

Editor's note:	Further abnormal cases are FFS.

The following abnormal cases can be identified:

a)	If the PDU session is an emergency PDU session, the SMF shall reject the PDU SESSION MODIFICATION REQUEST message with ESM cause #31 "request rejected, unspecified".

### 9.4.5	Network-requested PDU session modification procedure

#### 9.4.5.1	General

The purpose of the network-requested PDU session modification procedure is to enable the network to modify a PDU session.

Editor's note:	The solution for situation when the UE is in 5GMM-IDLE mode and the AMF decides to invoke asynchronous type communication is FFS.

Editor's note:	It is FFS whether there are cases where the UE can validly reject the PDU SESSION MODIFICATION COMMAND message.

#### 9.4.5.2	Network-requested PDU session modification procedure initiation

In order to initiate the network-requested PDU session modification procedure, the SMF shall create a PDU SESSION MODIFICATION COMMAND message.

If the authorized QoS rules of the PDU session is modified, the SMF shall set the authorized QoS rules IE of the PDU SESSION MODIFICATION COMMAND message to the authorized QoS rules of the PDU session.

If the session-AMBR of the PDU session is modified, the SMF shall set the selected Session-AMBR IE of the PDU SESSION MODIFICATION COMMAND message to the session-AMBR of the PDU session.

If the network-requested PDU session modification procedure is triggered by a UE-requested PDU session modification procedure, the SMF shall set the PTI IE of the PDU SESSION MODIFICATION COMMAND message to the PTI of the PDU SESSION MODIFICATION REQUEST message received as part of the UE-requested PDU session modification procedure.

If the network-requested PDU session modification procedure is not triggered by a UE-requested PDU session modification procedure, the SMF shall set the PTI IE of the PDU SESSION MODIFICATION COMMAND message to "No procedure transaction identity assigned".

If the selected SSC mode of the PDU session is "SSC mode 3" and the SMF requests the PDU session anchor relocation with multiple PDU sessions for SSC mode 3 as specified in 3GPP TS 23.502 [9], the SMF shall include the 5GSM cause IE set to #39 "reactivation requested" , in the PDU SESSION MODIFICATION COMMAND message and the SMF shall start timer Tf.

Editor's note: FFS whether SMF provides the PDU session release time IE indicating the time how long the SMF is willing to maintain the PDU session to the UE, and if so what the UE uses this time for.

The SMF shall send the PDU SESSION MODIFICATION COMMAND message, and the SMF shall start timer Tm (see example in figure 9.4.5.2.1).

NOTE:	If the SMF requests the PDU session anchor relocation with multiple PDU sessions for SSC mode 3 as specified in 3GPP TS 23.502 [9], the reallocation requested indication indicating whether the SMF is to be reallocated or the SMF is to be reused is provided to AMF.

# Figure 9.4.5.2.1: Network-requested PDU session modification procedure

#### 9.4.5.3	Network-requested PDU session modification procedure accepting

Upon receipt of a PDU SESSION MODIFICATION COMMAND message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.1.1.3, if the UE accepts the PDU SESSION MODIFICATION COMMAND message, the UE considers the PDU session as modified and the UE shall create a PDU SESSION MODIFICATION ACCEPT message.

If the PDU SESSION MODIFICATION COMMAND message contains the PTI value allocated in the UE-requested PDU session modification procedure, the UE shall release the PTI value indicated by the PTI IE and shall stop the timer Tk.

If the selected SSC mode of the PDU session is "SSC mode 3" and the PDU SESSION MODIFICATION COMMAND messages includes the 5GSM cause IE set to #39 "reactivation requested", the UE may re-initiate the UE-requested PDU session establishment procedure as specified in subclause 9.4.2 for the PDU session type, the SSC mode, the DNN, and the S-NSSAI as provided in the UE-requested PDU session establishment procedure of the present PDU session, after the complete of the network-requested PDU session modification procedure.

The UE shall transport the PDU SESSION MODIFICATION ACCEPT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.

Upon receipt of a PDU SESSION MODIFICATION ACCEPT message, the SMF shall stop timer Tm and shall consider the PDU session as modified.

#### 9.4.5.4	Network-requested PDU session modification procedure not accepted by the UE

Editor's note: FFS whether to change message name of PDU SESSION MODIFICATION COMMAND because of the possibility of the rejections from the UE

Upon receipt of a PDU SESSION MODIFICATION COMMAND message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.1.1.3 or using the network-initiated NAS transport procedure as specified in subclause 8.5.1.1.3.3, if the UE rejects the PDU SESSION MODIFICATION COMMAND message, the UE considers the PDU session as not modified and the UE shall create a PDU SESSION MODIFICATION COMMAND REJECT message.

If the PDU SESSION MODIFICATION COMMAND message contains the PTI value allocated in the UE-requested PDU session modification procedure, the UE shall release the PTI indicated by the PTI IE and shall stop the timer Tk.

The UE shall set the 5GSM cause IE of the PDU SESSION MODIFICATION COMMAND REJECT message to indicate the reason for rejecting the PDU session modification.

The 5GSM cause IE typically indicates one of the following 5GSM cause values:

Editor's note:	5GSM causes are FFS.

The UE shall transport the PDU SESSION MODIFICATION COMMAND REJECT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1 or using the UE-initiated NAS transport procedure as specified in subclause 8.5.1.1.3.2.

Upon receipt of a PDU SESSION MODIFICATION COMMAND REJECT message, the SMF shall stop timer Tm and shall consider the PDU session as not modified.

#### 9.4.5.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Tm expired

Editor's note:	Further abnormal cases are FFS.

#### 9.4.5.6	Abnormal cases in the UE

Editor's note:	Abnormal cases are FFS.

### 9.4.6	UE-requested PDU session release procedure

#### 9.4.6.1	General

The purpose of the UE-requested PDU session release procedure is to enable by the UE to request a release of a PDU session.

#### 9.4.6.2	UE-requested PDU session release procedure initiation

In order to initiate the UE-requested PDU session release procedure, the UE shall create an PDU SESSION RELEASE REQUEST message.

The UE shall allocate a PTI value currently not used and shall set the PTI IE of the PDU SESSION RELEASE REQUEST message to the allocated PTI value.

The UE shall transport the PDU SESSION RELEASE REQUEST message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1, and the UE shall start timer Tz (see example in figure 9.4.6.2.1).

# Figure 9.4.6.2.1: UE-requested PDU session release procedure

#### 9.4.6.3	UE-requested PDU session release procedure accepting

Upon receipt of an PDU SESSION RELEASE REQUEST message and an PDU session ID, if the SMF accepts the request to release the PDU session, the SMF shall stop timer Tf, if running, and shall perform the network-requested PDU session release procedure as specified in subclause 9.4.7.

#### 9.4.6.4	UE-requested PDU session release procedure rejecting

Upon receipt of an PDU SESSION RELEASE REQUEST message, if the SMF does not accept the request to release the PDU session, the SMF shall create an PDU SESSION RELEASE REJECT message.

The SMF shall set the 5GSM cause IE of the PDU SESSION RELEASE REJECT message to indicate the reason for rejecting the PDU session release.

The 5GSM cause IE typically indicates one of the following SM cause values:

Editor's note:	5GSM causes are FFS.

The SMF shall send the PDU SESSION RELEASE REJECT message.

Upon receipt of an PDU SESSION RELEASE REJECT message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.3, the UE shall stop timer Tz, shall release the allocated PTI value, and shall consider that the PDU session is not released.

#### 9.4.6.5	Abnormal cases in the UE

The following abnormal cases can be identified:

a)	Tz expired

Editors' note:	Further abnormal cases are FFS.

#### 9.4.6.6	Abnormal cases on the network side

Editors' note:	Abnormal cases are FFS.

### 9.4.7	Network-requested PDU session release procedure

#### 9.4.7.1	General

The purpose of the network-requested PDU session release procedure is to enable the network to release a PDU session.

#### 9.4.7.2	Network-requested PDU session release procedure initiation

In order to initiate the network-requested PDU session release procedure, the SMF shall create an PDU SESSION RELEASE COMMAND message.

The SMF shall set the SM cause IE of the PDU SESSION RELEASE COMMAND message to indicate the reason for releasing the PDU session.

The SM cause IE typically indicates one of the following SM cause values:

#36:	regular deactivation; or

#39:	reactivation requested.

Editor's note:	Further 5GSM causes are FFS.

If the selected SSC mode of the PDU session is "SSC mode 2" and the SMF initiates the PDU session anchor relocation as specified in 3GPP TS 23.502 [9], the SMF shall set the 5GSM cause IE to #39 "reactivation requested".

If the network-requested PDU session release procedure is triggered by a UE-requested PDU session release procedure, the SMF shall set the PTI IE of the PDU SESSION RELEASE COMMAND message to the PTI of the PDU SESSION RELEASE REQUEST message received as part of the UE-requested PDU session release procedure.

If the network-requested PDU session release procedure is not triggered by a UE-requested PDU session release procedure, the SMF shall set the PTI IE of the PDU SESSION RELEASE COMMAND message to "No procedure transaction identity assigned".

The SMF shall send:

a)	the PDU SESSION RELEASE COMMAND message; and

b)	if the SMF allows the AMF to skip sending of the N1 SM container to the UE and the 5GSM cause IE is not set to #39 "reactivation requested", the N1 SM delivery skip allowed indication;

towards the AMF, and the SMF shall start timer Ty (see example in figure 9.4.7.2.1).

# Figure 9.4.7.2.1: Network-requested PDU session release procedure

#### 9.4.7.3	Network-requested PDU session release procedure accepting

Upon receipt of an PDU SESSION RELEASE COMMAND message and a PDU session ID, using the network-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.3, the UE considers the PDU session as released and the UE shall create an PDU SESSION RELEASE ACCEPT message.

If the PDU SESSION RELEASE COMMAND message contains the PTI value allocated in the UE-requested PDU session release procedure, the UE shall release the PTI value indicated by the PTI IE and shall stop the timer Tz.

If the PDU SESSION RELEASE COMMAND messages includes the 5GSM cause IE set to 5GSM cause #39 "reactivation requested", the UE should re-initiate the UE-requested PDU session establishment procedure as specified in subclause 9.4.2 for the PDU session type, the SSC mode, the DNN, and the S-NSSAI as provided in the UE-requested PDU session establishment procedure of the released PDU session, after completion of the network-requested PDU session release procedure.

NOTE:	User interaction is necessary in some cases when the UE cannot re-initiate the UE-requested PDU session establishment procedure automatically.

Editor's note:	When the UE re-initiates the UE-requested PDU session establishment procedure, it is FFS how to ensure routing to the SMF serving the released PDU session and whether the PDU session ID is reused.

The UE shall transport the PDU SESSION RELEASE ACCEPT message and the PDU session ID, using the UE-initiated SM message transport procedure as specified in subclause 8.5.1.1.2.1.

Upon receipt of an PDU SESSION RELEASE ACCEPT message, the SMF shall stop timer Ty and shall consider the PDU session as released.

#### 9.4.7.4	N1 SM delivery skipped

If the PDU SESSION RELEASE COMMAND message was sent along the N1 SM delivery skip allowed indication towards AMF, then upon receipt of an indication that N1 SM delivery was skipped, the SMF shall stop timer Ty and shall consider the PDU session as released.

#### 9.4.7.5	Abnormal cases on the network side

The following abnormal cases can be identified:

a)	Ty expired

Editors' note:	Further abnormal cases are FFS.

#### 9.4.7.6	Abnormal cases in the UE

Editors' note:	Abnormal cases are FFS.

## 9.5	5GS session management coding

### 9.5.1	PDU session establishment request

#### 9.5.1.1	Message definition

The PDU SESSION ESTABLISHMENT REQUEST message is sent by the UE to the network to initiate establishment of a PDU session.

Message type:	PDU SESSION ESTABLISHMENT REQUEST

Significance:		dual

Direction:			UE to network

# Table 9.5.1.1.1: PDU SESSION ESTABLISHMENT REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT REQUEST message identity | Message type 6.6.6.6 | M | V | 1 |
| a- | PDU session type | PDU session type 9.x.1 | O | TV | 1 |
| b- | SSC mode | SSC mode 9.x.2 | O | TV | 1 |
| d | SM PDU DN request container | SM PDU DN request container 9.x.4 | O | TBD | TBD |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.1.2	PDU session type

This IE is included in the message when the UE requests to establish a new PDU session with a DN and requests a PDU session type.

#### 9.5.1.3	SSC mode

This IE is included in the message when the UE requests to establish a new PDU session with a DN and requests an SSC mode.

#### 9.5.1.4	SM PDU DN request container

This IE is included in the message when the UE requests to establish a new PDU session with a DN and needs to provide information for the PDU session authentication and authorization by the external DN.

#### 9.5.1.5	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 9.5.2	PDU session establishment accept

#### 9.5.2.1	Message definition

The PDU SESSION ESTABLISHMENT ACCEPT message is sent by the network to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates successful establishment of a PDU session.

Message type:	PDU SESSION ESTABLISHMENT ACCEPT

Significance:		dual

Direction:			network to UE

# Table 9.5.2.1.1: PDU SESSION ESTABLISHMENT ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT ACCEPT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | Selected PDU session type | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | Selected SSC mode | SSC mode 9.x.2 | M | V | 1/2 |
|  | Selected S-NSSAI | S-NSSAI 8.x.4 | M | LV | 2 or 5 |
|  | Selected DNN | DNN 8.x.5 | M | LV | 2-TBD |
|  | Authorized QoS rules | QoS rules 12.4.2.1 | M | LV-E | 2-65537 |
|  | Session AMBR | Session AMBR 12.4.2.2 | M | LV | TBD |
| a | 5GSM cause | 5GSM cause 9.x.6 | O | TV | 2 |
| b | PDU address | PDU address 9.x.5 | O | TLV | 7 |
| c | EAP message | EAP message 8.x.9 | O | TLV-E | 7-1503 |
| d | RQ timer value | GPRS timer 9.x.7 | O | TV | 2 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.2.2	5GSM cause

This IE is included when the selected PDU session type which is different from the PDU session type requested by the UE.

#### 9.5.2.3	PDU address

This IE is included when the selected PDU session type is "IPv4".

#### 9.5.2.4	EAP message

This IE is included when the external DN successfully performed authentication and authorization of the UE.

#### 9.5.2.5	RQ timer value

This IE is included when the network wants to provide the RQ timer value.

#### 9.5.2.6	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 9.5.3	 PDU session establishment reject

#### 9.5.3.1	Message definition

The PDU SESSION ESTABLISHMENT REJECT message is sent by the network to the UE in response to PDU SESSION ESTABLISHMENT REQUEST message and indicates unsuccessful establishment of a PDU session.

Message type:	PDU SESSION ESTABLISHMENT REJECT

Significance:		dual

Direction:			network to UE

# Table 9.5.3.1.1: PDU SESSION ESTABLISHMENT REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION ESTABLISHMENT REJECT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.x.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.3.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 9.5.4	PDU session authentication request

#### 9.5.4.1	Message definition

The PDU SESSION AUTHENTICATION REQUEST message is sent by the network to the UE for authentication of the UE establishing the PDU session.

Message type:	PDU SESSION AUTHENTICATION REQUEST

Significance:		dual

Direction:			network to UE

Editor's note:	The content of the PDU SESSION AUTHENTICATION REQUEST message is FFS. At least the following is needed: the authentication information (mandatory), extended protocol configuration options (optional).

### 9.5.5	PDU session authentication accept

#### 9.5.5.1	Message definition

The PDU SESSION AUTHENTICATION ACCEPT message is sent by the UE to the network in response to the PDU SESSION AUTHENTICATION REQUEST message and indicates acceptance of the PDU SESSION AUTHENTICATION REQUEST message.

Message type:	PDU SESSION AUTHENTICATION ACCEPT

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION AUTHENTICATION ACCEPT message is FFS. At least the following is needed: the authentication information (mandatory), extended protocol configuration options (optional).

### 9.5.6	PDU session authentication reject

#### 9.5.6.1	Message definition

The PDU SESSION AUTHENTICATION REJECT message is sent by the UE to the network in response to the PDU SESSION AUTHENTICATION REQUEST message and indicates rejection of the PDU SESSION AUTHENTICATION REQUEST message.

Message type:	PDU SESSION AUTHENTICATION REJECT

Significance:		dual

Direction:			UE to network

Editor's note:	The content of the PDU SESSION AUTHENTICATION REJECT message is FFS. At least the following is needed: SM cause (mandatory), extended protocol configuration options (optional).

### 9.5.7	PDU session modification request

#### 9.5.7.1	Message definition

The PDU SESSION MODIFICATION REQUEST message is sent by the UE to the network to request a modification of a PDU session.

Message type:	PDU SESSION MODIFICATION REQUEST

Significance:		dual

Direction:			UE to network

# Table 9.5.7.1.1: PDU SESSION MODIFICATION REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION MODIFICATION REQUEST message identity | Message type 6.6.6.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.7.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 9.5.8	PDU session modification reject

#### 9.5.8.1	Message definition

The PDU SESSION MODIFICATION REJECT message is sent by the network to the UE to indicate rejection of the PDU SESSION MODIFICATION REQUEST.

Message type:	PDU SESSION MODIFICATION REJECT

Significance:		dual

Direction:			network to UE

# Table 9.5.8.1.1: PDU SESSION MODIFICATION REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION MODIFICATION REJECT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.x.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.8.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 9.5.9	PDU session modification command

#### 9.5.9.1	Message definition

The PDU SESSION MODIFICATION COMMAND message is sent by the network to the UE to indicate a modification of a PDU session.

Message type:	PDU SESSION MODIFICATION COMMAND

Significance:		dual

Direction:			network to UE

# Table 9.5.9.1.1: PDU SESSION MODIFICATION COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND message identity | Message type 6.6.6.6 | M | V | 1 |
| a | 5GSM cause | 5GSM cause 9.x.6 | O | TV | 2 |
| b | Authorized QoS rules | QoS rules 12.4.2.1 | O | TLV-E | 3-65538 |
| c | Session AMBR | Session AMBR 12.4.2.2 | O | TLV | TBD |
| d | PDU session release time | GPRS timer 9.x.7 | O | TV | 2 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.9.2	5GSM cause

This IE is included when the network performs the PDU session anchor relocation for SSC mode 3.

#### 9.5.9.3	Authorized QoS rules

This IE is included when the authorized QoS rules of the PDU session are modified.

#### 9.5.9.4	Session AMBR

This IE is included when the Session AMBR of the PDU session is modified.

#### 9.5.9.5	PDU session release time

This IE is included when the network performs the PDU session anchor relocation for SSC mode 3.

#### 9.5.9.6	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 9.5.10	PDU session modification accept

#### 9.5.10.1	Message definition

The PDU SESSION MODIFICATION ACCEPT message is sent by the UE to the network in response to the PDU SESSION MODIFICATION COMMAND message and indicates an acceptance of the PDU SESSION MODIFICATION COMMAND message.

Message type:	PDU SESSION MODIFICATION ACCEPT

Significance:		dual

Direction:			UE to network

# Table 9.5.10.1.1: PDU SESSION MODIFICATION ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION MODIFICATION ACCEPT message identity | Message type 6.6.6.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.10.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 9.5.11	PDU session modification command reject

#### 9.5.11.1	Message definition

The PDU SESSION MODIFICATION COMMAND REJECT message is sent by the UE to the network to indicate rejection of the PDU SESSION MODIFICATION COMMAND message.

Message type:	PDU SESSION MODIFICATION COMMAND REJECT

Significance:		dual

Direction:			UE to network

# Table 9.5.11.1.1: PDU SESSION MODIFICATION COMMAND REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION MODIFICATION COMMAND REJECT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.x.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.11.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 9.5.12	PDU session release request

#### 9.5.12.1	Message definition

The PDU SESSION RELEASE REQUEST message is sent by the UE to the network to request a release of a PDU session.

Message type:	PDU SESSION RELEASE REQUEST

Significance:		dual

Direction:			UE to network

# Table 9.5.12.1.1: PDU SESSION RELEASE REQUEST message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION RELEASE REQUEST message identity | Message type 6.6.6.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.12.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

### 9.5.13	PDU session release reject

#### 9.5.13.1	Message definition

The PDU SESSION RELEASE REJECT message is sent by the network to the UE to indicate rejection of request a release of a PDU session.

Message type:	PDU SESSION RELEASE REJECT

Significance:		dual

Direction:			network to UE

# Table 9.5.13.1.1: PDU SESSION RELEASE REJECT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU RELEASE RELEASE REJECT message identity | Message type 6.6.6.6 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.x.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.13.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 9.5.14	PDU session release command

#### 9.5.14.1	Message definition

The PDU SESSION RELEASE COMMAND message is sent by the network to the UE to indicate a release of a PDU session.

Message type:	PDU SESSION RELEASE COMMAND

Significance:		dual

Direction:			network to UE

# Table 9.5.14.1.1: PDU SESSION RELEASE COMMAND message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION RELEASE COMMAND message identity | Message type 6.6.6.6 | M | V | 1 |
|  | 5GSM cause | 5GSM cause 9.x.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.1.2	Extended protocol configuration options

This IE is included in the message when the network wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the UE.

### 9.5.15	PDU session release accept

#### 9.5.15.1	Message definition

The PDU SESSION RELEASE ACCEPT message is sent by the UE to the network in response to the PDU SESSION RELEASE COMMAND message and indicates an acceptance of a release of the PDU session.

Message type:	PDU SESSION RELEASE ACCEPT

Significance:		dual

Direction:			UE to network

# Table 9.5.15.1.1: PDU SESSION RELEASE ACCEPT message content

| IEI | Information Element | Type/Reference | Presence | Format | Length |
| --- | --- | --- | --- | --- | --- |
|  | Extended protocol discriminator | Extended protocol discriminator 6.6.6.2 | M | V | 1 |
|  | PDU session ID | PDU session ID TBD | M | V | 1/2 |
|  | Spare half octet | Spare half octet 6.6.6.4 | M | V | 1/2 |
|  | PTI | Procedure transaction identity 6.6.6.5 | M | V | 1 |
|  | PDU SESSION RELEASE ACCEPT message identity | Message type 6.6.6.6 | M | V | 1 |
| 7B | Extended protocol configuration options | Extended protocol configuration options 9.x.3 | O | TLV-E | 4-65538 |

#### 9.5.15.2	Extended protocol configuration options

This IE is included in the message when the UE wishes to transmit (protocol) data (e.g. configuration parameters, error codes or messages/events) to the network.

## 9.6	5GSM information elements

### 9.6.1	PDU session type

The purpose of the PDU session type information element is to indicate type of the PDU session.

The PDU session type information element is coded as shown in figure 9.6.1.1 and table 9.6.1.1.

The PDU session type is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU session type IEI |  |  |  | PDU session type value |  |  |  | Octet 1 |

# Figure 9.6.1.1: PDU session type information element

Table 9.6.1.1: PDU session type information element

| PDU session type value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | IP |
| 0 | 0 | 1 | 0 | IPv4 |
| 0 | 0 | 1 | 1 | IPv6 |
| 0 | 1 | 0 | 0 | Unstructured |
| 0 | 1 | 0 | 1 | Ethernet |
| All other values are reserved. |  |  |  |  |

### 9.6.2	SSC mode

The purpose of the SSC mode information element is to indicate SSC mode.

The SSC mode information element is coded as shown in figure 9.6.2.1 and table 9.6.2.1.

The SSC mode is a type 1 information element.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SSC mode IEI |  |  |  | SSC mode value |  |  |  | Octet 1 |

# Figure 9.6.2.1: SSC mode information element

Table 9.6.2.1: SSC mode information element

| SSC mode value (octet 1, bit 1 to bit 4) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 0 | 1 | SSC mode 1 |
| 0 | 0 | 1 | 0 | SSC mode 2 |
| 0 | 0 | 1 | 1 | SSC mode 3 |
| All other values are reserved. |  |  |  |  |

### 9.6.3	Extended protocol configuration options

See subclause 10.5.6.3A in 3GPP TS 24.008 [11].

### 9.6.4	SM PDU DN request container

The SM PDU DN request container contains a DN-specific identity of the UE in the network access identifier (NAI) format.

### 9.6.5	PDU address

The PDU address information element can assign an IPv4 address to the UE associated with a PDU session.

The PDU address information element is coded as shown in figure 9.6.5.1 and table 9.6.5.1.

The PDN address is a type 4 information element with 7 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PDU address IEI |  |  |  |  |  |  |  | octet 1 |
| Length of PDU address contents |  |  |  |  |  |  |  | octet 2 |
| 0 Spare | 0 Spare | 0 Spare | 0 Spare | PDU session type value |  |  |  | octet 3 |
| PDU address information |  |  |  |  |  |  |  | octet 4 octet 7 |

# Figure 9.6.5.1: PDU address information element

Table 9.6.5.1: PDU address information element

| PDU session type value (octet 3) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Bits |  |  |  |  |
| 4 | 3 | 2 | 1 |  |
| 0 | 0 | 1 | 0 | IPv4 |
| All other values are reserved. |  |  |  |  |
| PDU address information (octet 4 to 7) |  |  |  |  |
| If the PDU session type value indicates IPv4, the PDU address information in octet 4 to octet 7 contains an IPv4 address. |  |  |  |  |

### 9.7.6	5GSM cause

The purpose of the 5GSM cause information element is to indicate the reason why a 5GSM request is rejected.

The 5GSM cause information element is coded as shown in figure 9.7.6.1 and table 9.7.6.1.

The 5GSM cause is a type 3 information element with 2 octets length.

| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5GSM cause IEI |  |  |  |  |  |  |  | octet 1 |
| Cause value |  |  |  |  |  |  |  | octet 2 |

# Figure 9.7.6.1: 5GSM cause information element

Table 9.7.6.1: 5GSM cause information element

| Cause value (octet 2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Bits |  |  |  |  |  |  |  |  |  |
| 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |
| 0 | 0 | 0 | 1 | 1 | 1 | 0 | 1 |  | User authentication failed |
| 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 |  | Request rejected, unspecified |
| 0 | 0 | 1 | 0 | 0 | 0 | 1 | 0 |  | Service option temporarily out of order |
| 0 | 0 | 1 | 0 | 0 | 1 | 0 | 0 |  | Regular deactivation |
| 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |  | Reactivation requested |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 0 |  | PDU session type IPv4 only allowed |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |  | PDU session type IPv6 only allowed |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 1 |  | Semantically incorrect message |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 |  | Invalid mandatory information |
| 0 | 1 | 1 | 0 | 0 | 0 | 0 | 1 |  | Message type non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 0 |  | Message type not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 0 | 0 | 1 | 1 |  | Information element non-existent or not implemented |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 0 |  | Conditional IE error |
| 0 | 1 | 1 | 0 | 0 | 1 | 0 | 1 |  | Message not compatible with the protocol state |
| 0 | 1 | 1 | 0 | 1 | 1 | 1 | 1 |  | Protocol error, unspecified |
| Any other value received by the UE shall be treated as 0010 0010, "service option temporarily out of order". Any other value received by the network shall be treated as 0110 1111, "protocol error, unspecified". |  |  |  |  |  |  |  |  |  |

### 9.8.7	GPRS timer

See subclause 10.5.7.3 in 3GPP TS 24.008 [11].

## 9.9	Timers of 5GS session management

Timers of 5GS session management are shown in table 9.9.1 and table 9.9.2.

# Table 9.9.1: Timers of 5GS session management – UE side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Tx | TBD | TBD | Transmission of PDU SESSION ESTABLISHMENT REQUEST message. | PDU SESSION ESTABLISHMENT ACCEPT message received or PDU SESSION ESTABLISHMENT REJECT message received. | TBD |
| Tm | TBD | TBD | Transmission of PDU SESSION AUTHENTICATION REQUEST message. | PDU SESSION AUTHENTICATION ACCEPT message received or PDU SESSION AUTHENTICATION REJECT message received. | TBD |
| Tk | TBD | TBD | Transmission of PDU SESSION MODIFICATION REQUEST message. | PDU SESSION MODIFICATION COMMAND message with the same PTI is received or PDU SESSION MODIFICATION REJECT message received. | TBD. |
| Tz | TBD | TBD | Transmission of PDU SESSION RELEASE REQUEST message. | PDU SESSION RELEASE COMMAND message with the same PTI is received or PDU SESSION RELEASE REJECT message received. | TBD |

# Table 9.9.2: Timers of 5GS session management – SMF side

| TIMER NUM. | TIMER VALUE | STATE | CAUSE OF START | NORMAL STOP | ON EXPIRY |
| --- | --- | --- | --- | --- | --- |
| Tm | TBD | TBD | Transmission of PDU SESSION MODIFICATION COMMAND message. | PDU SESSION MODIFICATION ACCEPT message received or PDU SESSION MODIFICATION REJECT message received. | TBD |
| Ty | TBD | TBD | Transmission of PDU SESSION RELEASE COMMAND message. | PDU SESSION RELEASE ACCEPT message received or N1 SM delivery skipped indication received. | TBD |
| Tf | TBD | TBD | Transmission of PDU SESSION MODIFICATION COMMAND message with 5GSM cause IE set to #39 "reactivation requested". | PDU SESSION RELEASE REQUEST message is received. | Network-requested PDU session release procedure is started. |

## 9.10	Cause values for 5GS session management

### 9.10.1	Causes related to nature of request

Editor’s note:	5GSM cause values related to nature of request need to be added to this subclause.

### 9.10.2	Protocol errors (e.g., unknown message)

Cause #95 – Semantically incorrect message

This 5GSM cause is used to report receipt of a message with semantically incorrect contents.

# Cause #96 – Invalid mandatory information

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message with a non-semantical mandatory IE error.

# Cause #97 – Message type non-existent or not implemented

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message with a message type it does not recognize either because this is a message not defined, or defined but not implemented by the equipment sending this 5GSM cause.

# Cause #98 – Message type not compatible with protocol state

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message not compatible with the protocol state.

# Cause #99 – Information element non-existent or not implemented

This 5GSM cause indicates that the equipment sending this 5GSM cause has received a message which includes information elements not recognized because the information element identifier is not defined or it is defined but not implemented by the equipment sending the 5GSM cause. However, the information element is not required to be present in the message in order for the equipment sending the 5GSM cause to process the message.

# Cause #100 – Conditional IE error

This 5GSM cause indicates that the equipment sending this cause has received a message with conditional IE errors.

# Cause #101 – Message not compatible with protocol state

This 5GSM cause indicates that a message has been received which is incompatible with the protocol state.

# Cause #111 – Protocol error, unspecified

This 5GSM cause is used to report a protocol error event only when no other 5GSM cause in the protocol error class applies.

# 10	Access to the 5G core network via non-3GPP access networks

## 10.1	General

In this release, the 5GS supports untrusted non-3GPP accesses and does not support trusted non-3GPP accesses (see 3GPP TS 23.501 [8]).

Access to the 5GCN via non-3GPP access networks shall be supported by the non-3GPP interworking function (N3IWF). Between the UE and the N3IWF, an IPsec SA shall be established. During the IPsec SA establishment, the UE shall be authenticated by and registered to the 5G core network. The N3IWF interfaces to 5G core network via N2 and N3 reference points.

Editor's note:	How to transmit a registration request message is FFS.

If the UE is authenticated by the 5G core network, secure NAS message exchange between the UE and the 5G core network shall be supported using the N1 reference point over the N3IWF.

When a UE is registered via 3GPP and non-3GPP access networks, two N1 NAS signalling connections shall exist for the UE.

A UE simultaneously registered to a single PLMN over 3GPP and non-3GPP access networks, shall be served by a single AMF. In this case, two N1 NAS signalling connections shall exist for the AMF. Otherwise a UE registered to two different PLMNs over 3GPP and non-3GPP access networks, shall be served by two AMFs in two different PLMNs. In this case, single N1 NAS signalling connections shall exist for each of the AMFs.

During the IPsec SA establishment between the UE and the N3IWF, the N3IWF retrieves the NAI of the UE and the NAI is used to identify the UE in the 5G core network.

## 10.2	Protocol stack

Editor's note:	This subclause will specify the control-plane and user-plane protocol stacks of untrusted non-3GPP access to 5GCN.

## 10.3	NAS over non-3GPP access

### 10.3.1	General

From the UE's NAS perspective, the N1 interface is indistiguishable over the 3GPP access or the non-3GPP access in so much as the peer-to-peer signalling procedures and messages defined for UE NAS to 5GCN NAS over 3GPP access will be used for same purposes for UE NAS to 5GCN NAS over non-3GPP access. The 5GS NAS state machines of both 3GPP access or non-3GPP access should likewise have minimum differences. Maximum reuse should be the objective with any deviations or exceptions adopted if without such deviations or exceptions, signalling procedures and intended services will fail or malfunction, or service or archtitectural (or both) requirments cannot be met.

### 10.3.2	Mobility management over non-3GPP access

The mobility management procedures defined for N1 over 3GPP access will be re-used for N1 over non-3GPP access with the following exceptions:

-	the status of the UE's non-3GPP 5GMM instance and the status of the UE's 3GPP 5GMM instance are independent and can be different.

-	single registration mode and dual registration mode do not apply for 5GMM over non-3GPP access.

-	there will be one instance of 5GMM for 3GPP access and one instance of 5GMM for non-3GPP access. The RPLMN over non-3GPP access can be different from the RPLMN over 3GPP access. The MCC of the RPLMN over 3GPP access and the MCC of the RPLMN over the non-3GPP access can also be different.

-	as over non-3GPP access the 5GS operates one common registration area for an entire PLMN, list management of registration areas and resultant registration updating due to registration area change with the registered PLMN is not required. Periodic registration over a non-3GPP access shall not be applied. Registration updating at change of PLMN is still required.

-	the 5GMM over non-3GPP access considers that the N1 NAS signalling connection is established once the IKE_SA association is established succcessfully.

Editor's note:	Further exceptions are possible and are FFS.

-	UE-initiated service request procedure via non-3GPP access is supported. When the UE has uplink data to send for a PDU session over non-3GPP access and the UE is in 5GMM-IDLE mode, UE initiates service request procedure via non-3GPP access to re-activate user plane connections for all PDU sessions associated with non-3GPP access, but not PDU sessions associated with 3GPP access.

-	There is no network initiated service request procedure via non-3GPP Access.

### 10.3.3	Session management over non-3GPP access

The session management procedures defined for N1 over 3GPP access will be re-used for N1 over non-3GPP access with the following exceptions:

-	a PDU session established over non-3GPP access cannot be access transferred for use upon an inter-system change to EPS.

Editor's note:	Further exceptions are possible and are FFS.

## 10.4	Registration over non-3GPP access

### 10.4.1	General

The UE registers separately for 3GPP access and non-3GPP access, respectively.

The UE shall not perform periodic registration update over a non-3GPP access.

Editor's note:	It is FFS whether a registration timer is used for the registration over non-3GPP access

The UE cannot be paged over a non-3GPP access.

### 10.4.2	Registration over non-3GPP access

When the UE is registered with a PLMN over a non-3GPP access, the AMF and the UE maintain:

-	Registration state and state machine over non-3GPP access;

-	Security context;

-	Temporary UE identity; and

-	Registration area for non-3GPP access, which is associated with a fixed well-known N3GPP TAI.

Editor’s note:	What information the N3IWF maintains for a registered UE is FFS.

### 10.4.3	Registration over non-3GPP access and 3GPP access

When the UE is registered with a PLMN over a non-3GPP access and over a 3GPP access, the AMF and the UE maintain:

-	Separate registration states and separate instances of state machines over 3GPP access and non-3GPP access, respectively;

Editor’s note:	The state machine over non-3GPP access is FFS.

-	Separate registration areas; and

-	Same security context and Temporary UE identity, only if the UE is registered with the same PLMN over both accesses. Otherwise, the security contexts and the temporary UE identity shall be separate.

Editor’s note:	To what extent the security context is the same when the UE is registered with the same PLMN over both accesses is FFS.

When the UE initiates registration over a non-3GPP access to a PLMN for which it has a valid temporary UE identity, the UE shall use this temporary UE identity during the registration. When the UE is currently registered with the same PLMN over a 3GPP access, the temporary UE identity is considered as valid.

Editor’s note:	Other conditions for considering a temporary UE identity as valid are FFS.

### 10.4.4	De-registration over non-3GPP access

The UE and the AMF may initiate de-registration for a non-3GPP access by:

-	Initiating a de-registration request over a non-3GPP access; or

-	Initiating a de-registration request over a 3GPP access with an indication that it applies to all accesses.

When the UE has no PDU sessions over non-3GPP access, or the UE moves all the PDU sessions over a non-3GPP access to a 3GPP access, the UE and the AMF need not initiate de-registration over the non-3GPP access.

## 10.5	Selection of N3IWF

The ePDG selection procedure specified in the 3GPP TS 24.302 [13] is reused as mechanism for N3IWF selection except for the following differences:

1)	The UE performs N3IWF selection based on the information configured by the HPLMN in the UE and based on the UE's knowledge of the PLMN it is attached to.

# Editor's note:	The format and contents of N3IWF configuration information are FFS.

2)	The input to the DNS query to retrive IP address(es) of the N3IWF is a N3IWF FQDN (N3IWF fully qualified domain name). A N3IWF FQDN is either provisioned by the home operator or constructed by UE in either the operator identifier FQDN format or the tracking area identity FQDN format. Location area identity FQDN format is not used.

# Editor's note:	The detailed format of the N3IWF FQDN format is FFS and is to be specified in 3GPP TS 23.003 [5].

## 10.6	Procedures between UE and N3IWF

Editor's note:	This subclause specifies the IKEv2 procedures between the UE and N3IWF due to UE registration/de-registration, PDU session setup, PDU session modification and PDU session release.

### 10.7.1	IKE SA creation procedure for UE registration via untrusted non-3GPP access

Editor's note:	This subclause specifies IKE SA creation procedure for UE registration via untrusted non-3GPP access.

### 10.7.2	Child SA creation procedure for UE-requested PDU session establishment

Editor's note:	This subclause specifies child SA creation procedure for PDU session setup via untrusted non-3GPP access.

#### 10.7.2.1	General

The purpose of the Child SA creation procedure during the UE-requested PDU session establishment is to establish a Child SA associating to the QoS profiles of the PDU session. This procedure shall be initiated by the N3IWF.

One Child SA can be accoiated with one or more QoS profiles of the PDU session. The N3IWF will determine the number of Child SAs to establish and the QoS profiles associated with each Child SA based on local policies, configuration and the QoS profiles received from the network.

#### 10.7.2.2	Child SA creation procedure initiation

The N3IWF shall initiate the Child SA creation procedure by sending a CREATE_CHILD_SA request message to the UE as specified in IETF RFC 7296 [26].

The CREATE_CHILD_SA request message shall include:

-	USE_TRANSPORT_MODE notification; and

-	5G_QOS_INFO Notify payload which contains:

-	QoS profile(s); and

-	PDU session ID.

The CREATE_CHILD_SA request message may also include:

-	a DSCP value.

Editor's note:	The definition of 5G_QOS_INFO Notify payload is FFS.

Editor's note:	Whether 5G_QOS_INFO Notify can contain QFI instead of QoS profile need to be FFS.

Editor's note:	How to handle the packet in transport mode is FFS.

#### 10.7.2.3	Child SA creation procedure accepted by the UE

If the UE accepts the CREATE_CHILD_SA request, the UE shall send the CREATE_CHILD_SA response as specified in IETF RFC 7296 [26].

The CREATE_CHILD_SA request message shall include:

-	USE_TRANSPORT_MODE notification.

Any IKEv2 Notify payload indicating an error shall not be included in the CREATE_CHILD_SA response.

#### 10.7.2.4	Child SA creation procedure not accepted by the UE

If the UE does not accept the Child SA creation, the UE shall send a send the CREATE_CHILD_SA response message with the corresponding Notify payload of error type as specified in IETF RFC 7296 [26].

Editor's note:	Whether 5G specific private Notify Message - Error Types needs to be defined is FFS.

Editor's note:	The behaviour of N3IWF is FFS upon receipt of the the CREATE_CHILD_SA response message indicating failure of the procedure.

#### 10.7.2.5	Abnormal cases in the UE

Editor's note:	The abnormal cases in the UE for this specific procedure (not specified in IETF RFC 7296 [26]) are FFS.

#### 10.7.2.6	Abnormal cases in the N3IWF

Editor's note:	The abnormal cases in the N3IWF for this specific procedure (not specified in IETF RFC 7296 [26]) are FFS.

### 10.7.3	Child SA change procedure for PDU session modification

Editor's note:	This subclause specifies child SA change procedure for PDU session modification via untrusted non-3GPP access.

### 10.7.4	Child SA deletion procedure for PDU session release

Editor's note:	This subclause specifies child SA deletion procedure for PDU session release via untrusted non-3GPP access.

#### 10.7.4.1	General

The purpose of the Child SA deletion procedure for PDU session release is to delete all the Child SAs associated with the PDU session. This procedure shall be initiated by the N3IWF.

#### 10.7.4.2	Child SA deletion procedure initiation

The N3IWF shall initiate the Child SA deletion procedure by sending an INFORMATIONAL request message including a Delete payload to the UE as specified in IETF RFC 7296 [26]. The Delete payload shall include:

-	the Protocol ID set to "3" for ESP; and

-	all the N3IWF's ESP Security Parameter Indexe(s) associated to the released PDU session.

#### 10.7.4.3	Child SA deletion procedure accepted by the UE

If the UE accepts the INFORMATIONAL request message for deletion of the Child SAs, the UE shall send the INFORMATIONAL response to the N3IWF including the Delete payload received in the corresponding INFORMATIONAL request message as specified in IETF RFC 7296 [26].

Any IKEv2 Notify payload indicating an error shall not be included in the INFORMATIONAL response.

Editor's note:	The possible Notify messages for status type in the INFORMATIONAL response are FFS.

#### 10.7.4.5	Abnormal cases in the UE

Editor's note:	The abnormal cases in the UE for this specific procedure (not specified in IETF RFC 7296 [26]) are FFS.

#### 10.7.4.6	Abnormal cases in the N3IWF

Editor's note:	The abnormal cases in the N3IWF for this specific procedure (not specified in IETF RFC 7296 [26]) are FFS.

### 10.7.5	IKE SA deletion for deregistration via untrusted non-3GPP access

Editor's note:	This subclause specifies IKE SA deletion procedure for deregistration via untrusted non-3GPP access.

#### 10.7.5.1	General

The purpose of the IKE SA deletion procedure for deregistration via untrusted non-3GPP access is to delete the IKE SA of the UE. This procedure shall be initiated by the N3IWF.

#### 10.7.5.2	IKE SA deletion procedure initiation

The N3IWF shall initiate the IKE SA deletion procedure by sending an INFORMATIONAL request message including a Delete payload to the UE as specified in IETF RFC 7296 [26].

The Protocol ID shall be set to "1" and no subsequent SPI in the Delete payload. This indicates that the IKE security association, and all IPsec ESP security associations that were negotiated within it between N3IWF and UE shall be deleted.

#### 10.7.5.3	IKE SA deletion procedure accepted by the UE

If the UE accepts the INFORMATIONAL request message for deletion of the IKE SA, the UE shall send an empty INFORMATIONAL response to the N3IWF as specified in IETF RFC 7296 [26].

#### 10.7.5.5	Abnormal cases in the UE

Editor's note:	The abnormal cases in the UE for this specific procedure (not specified in IETF RFC 7296 [26]) are FFS.

#### 10.7.5.6	Abnormal cases in the N3IWF

Editor's note:	The abnormal cases in the N3IWF for this specific procedure (not specified in IETF RFC 7296 [26]) are FFS.

## 10.8	IKEv2 payload extensions

Editor's note:	This subclause defines the coding of IKEv2 payload for untrusted non-3GPP access to 5GCN.

# 11	Interworking with E-UTRAN connected to EPC

## 11.1	General

In order to interwork with E-UTRAN connected to EPC, the UE supporting both S1 mode and N1 mode can operate in single-registration mode or dual-registration mode (see 3GPP TS 23.501 [8]). Support of single-registration mode is mandatory for UEs supporting both S1 mode and N1 mode.

During the attach procedure or initial registration procedure, the mode for intersystem change is selected if the UE supports both S1 mode and N1 mode, and the network supports intersystem change. See 3GPP TS 24.301 [12] and subclause 8.4.2.1 for further details.

## 11.2	Single-registration mode

### 11.2.1	General

If both 5GMM and EMM are enabled, a UE, which is capable of N1 mode and S1 mode in the single-registration mode, shall maintain one common registration for 5GMM and EMM.

### 11.2.2	Coordination between 5GMM and EMM

A UE that is not registered shall be in state EMM-DEREGISTERED and in state 5GMM-DEREGISTERED.

In N1 mode, upon successful registration procedure, the UE operating in the single-registration mode shall enter substates 5GMM-REGISTERED.NORMAL-SERVICE and EMM-REGISTERED.NO-CELL-AVAILABLE.

In S1 mode, upon successful attach or tracking area updating procedure, the UE operating in the single-registration mode shall enter substates 5GMM-REGISTERED.NO-CELL-AVAILABLE and EMM-REGISTERED.NOMRAL-SERVICE.

At intersystem change from N1 mode to S1 mode when no PDU session context is active, the UE shall move to state 5GMM-DEREGISTERED and state EMM-DEREGISTERED and then initiate the attach procedure if at least one of the UE and the MME does not support EMM-REGISTERED without PDN connection.

## 11.3	Dual-registration mode

If both 5GMM and EMM are enabled, a UE, which is capable of N1 mode and S1 mode in the dual-registration mode, shall maintain independent registrations for 5GMM and EMM independently and coordination between 5GMM and EMM is not needed.

The UE operating in the dual-registration mode may perfrom the attach procedure towards the MME ahead of PDU session transfer from the 5GS to EPS if both the UE and the MME support EMM-REGISTERED without PDN connection.

If both the UE and the MME support EMM-REGISTERED without PDN connection, the UE that transferred all PDN connections to the 5GS, may be in state EMM-REGISTERED. Otherwise, the UE shall enter state EMM-DEREGISTERED upon transferring all PDN connection to the 5GS.

# 12	5G System core network impact on services, network functions and capabilities

## 12.1	Specific services

### 12.1.1	Public warning system

#### 12.1.1.1	General

##### 12.1.1.1.1	Service Requirements

5G PWS will have the same functionality as E-UTRAN PWS for 4G.

##### 12.1.1.1.2	Technology Framework Assumptions

PWS in LTE is realized using cell broadcast technology (see 3GPP TS 22.268 [4] and 3GPP TS 23.041 [6]). It is expected that same CBS technology framework is used for technical realization of PWS in 5GS.

#### 12.1.1.2	Architecture alternative 1: Supporting PWS in 5GS via Service based Interface

##### 12.1.1.2.1	Overview

In this architecture alternative, the AMF interfaces with CBCF and NG-RAN to enable PWS service with the interface between AMF and CBCF implemented as service based interface. CBCF uses AMF communication services to forward warning messages to NG-RAN and to subscribe to receive warning delivery related notifications.

##### 12.1.1.2.2	Architecture description

###### 12.1.1.2.2.1	PWS architecture

Figure 12.1.1.2.2.1.1 shows the basic network structure of PWS architecture in 5GS.

# Figure 12.1.1.2.2.1.1: PWS architecture

The cell broadcast centre function (CBCF) is part of the core network and connected to the AMF via the Ncbcf service based interface.

Reference point to support PWS architecture:

N2:	Reference point between the NG-RAN and the AMF.

NOTE:	NG-RAN can be NR based or E-UTRA based (See 3GPP TS 23.501 [8] and 3GPP TS 38.413 [19]).

Service based interface to support PWS in 5GS

Namf:	Service-based interface exhibited by AMF.

Ncbcf:	Service-based interface exhibited by CBCF.

The CBE and the interface between CBE and CBCF are out of scope of 3GPP specifications.

###### 12.1.1.2.2.2	AMF functionality

The AMF may interface to one CBCF or multiple CBCFs. An AMF may interface to several NG-RANs (i.e. combinations of gNodeBs and/or eNodeBs).

The AMF is responsible for:

1)	Routing the WRITE-REPLACE-WARNING-REQUEST message to the appropriate NG-RAN entities upon receiving warning message request from CBCF;

2)	Routing the STOP-WARNING-REQUEST message to the appropriate NG-RAN entities upon receiving STOP-WARNING-REQUEST message from CBCF;

3)	Warning message delivery success or failure notification

-	Providing to the CBCF acknowledgement of successful execution of commands received from the CBCF;

-	Reporting to the CBCF failure when a command received from the CBCF is not understood or cannot be executed; and

-	Reporting to the CBCF the Broadcast completed area list, the Broadcast cancelled area list; and

4)	Forward PWS RESTART-INDICATION message and the PWS FAILURE-INDICATION message received from NG-RAN entities to all CBCFs that the AMF interfaces with.

###### 12.1.1.2.2.3	CBCF functionality

CBCF is a network function in the 5G core network. The CBCF may be connected to several AMFs. The CBCF may be connected to several CBEs. The CBCF shall be responsible for the management of CBS messages as specified in subclause 5 of 3GPP TS 23.041 [16].

CBCF supports service based interface. CBCF uses AMF communication services to forward warning messages to NG-RAN and to subscribe to receive warning delivery related notifications.

###### 12.1.1.2.2.4	NG-RAN functionality

The NG-RAN may interface to multiple AMFs.

The NG-RAN is responsible for:

1)	Upon receipt of the WRITE-REPLACE-WARNING-REQUEST message, prioritise its resources and broadcast the warning message to the warning areas indicated in the requested Warning area list;

2)	Upon receipt of the KILL-REQUEST message, stop broadcasting the warning message within for that area indicated in the requested Warning area list and discard the warning message;

3)	Inform the AMF that PWS information for some or all cells of the NG-RAN are available for reloading from the CBCF if needed by sending a PWS RESTART-INDICATION message to the AMF; and

4)	Inform the AMF that ongoing PWS operation for one or more cells of the NG-RAN has failed by sending a PWS FAILURE-INDICATION message to the AMF.

##### 12.1.1.2.3	Protocol Stack Overview

# Legend:

-	NG application protocol information for cell broadcast (NG-AP-CB): Subset of NG-AP information that the AMF relays between the AN and the CBCF. NG-AP-CB corresponds to a subset of NG-AP defined in 3GPP TS 38.413 [19].

-	NG application protocol (NGAP): Application layer protocol between the NG-RAN entity (eNodeB and/or gNodeB) and the AMF. The NGAP protocol is defined in 3GPP TS 38.413 [19].

-	SCTP for the control plane (SCTP): This protocol guarantees delivery of signalling messages between AMF and NG-RAN (N2) and between CBCF and AMF (NBc). SCTP is defined in IETF RFC 4960 [24].

# Figure 12.1.1.2.3.1: CBCF - NG-RAN

NOTE:	Detailed protocol stack and the exact name of the protocol layer on Namf-Ncbcf service based interface is FFS by CT4.

##### 12.1.1.2.4	CBCF-AMF interface operations

The CBCF network function uses Namf_Communication service to communicate with the NG-RAN for sending warning request primitives and receiving warning message delivery indication or restart/failure indication primitives.

The requirements are described by primitives.

The following table shows primitives that are expected to be supported using AMF via Namf interface:

Table 12.1.1.2.4.1: CBC-AMF primitives via Namf interface

| Name | Originator | Type | Namf Service operations |
| --- | --- | --- | --- |
| WRITE-REPLACE-WARNING-REQUEST | CBCF | Request/Indication | Namf_Communication_N2MessageTrigger Namf_Communication_N2InfoSubscribe |
| WRITE-REPLACE-WARNING-CONFIRM | AMF | Response/Confirm | Namf_Communication_N2InfoNotify |
| STOP-WARNING-REQUEST | CBCF | Request/Indication | Namf_Communication_N2MessageTrigger Namf_Communication_N2InfoSubscribe |
| STOP-WARNING-CONFIRM | AMF | Response/Confirm | Namf_Communication_N2InfoNotify |
| WRITE-REPLACE-WARNING-INDICATION | AMF | Request/Indication | Namf_Communication_N2InfoSubscribe Namf_Communication_N2InfoNotify |
| STOP-WARNING-INDICATION | AMF | Request/Indication | Namf_Communication_N2InfoSubscribe Namf_Communication_N2InfoNotify |
| RESTART-INDICATION-NG-RAN | AMF | Request/Indication | Namf_Communication_N2InfoSubscribe Namf_Communication_N2InfoNotify |
| FAILURE-INDICATION-NG-RAN | AMF | Request/Indication | Namf_Communication_N2InfoSubscribe Namf_Communication_N2InfoNotify |

#### 12.1.1.3	Architecture alternative 2: Supporting PWS in 5GS via SBc like Interface

##### 12.1.1.3.1	Overview

In this architecture alternative, the AMF interfaces with CBC and NG-RAN to enable PWS service by using SBc-like protocol interface (see 3GPP TS 29.168 [14]) referred to as "NBc" interface between AMF and CBC. Same protocol stack and procedures for SBc interface will be re-used for NBc interface.

##### 12.1.1.3.2	Architecture description

###### 12.1.1.3.2.1	PWS architecture

Figure 12.1.1.3.2.1.1 shows the basic network structure of PWS architecture in 5GS.

# Figure 12.1.1.3.2.1.1: PWS architecture

The cell broadcast centre function (CBC) is part of the core network and connected to the AMF via the NBc interface.

Reference point to support PWS architecture:

N2:	Reference point between the NG-RAN and the AMF.

NBc:	Reference point between the AMF and the CBC.

NOTE:	NG-RAN can be NR based or E-UTRA based (See 3GPP TS 23.501 [8] and 3GPP TS 38.413 [19]).

The CBE and the interface between CBE and CBC are out of scope of 3GPP specifications.

###### 12.1.1.3.2.2	AMF functionality

The AMF may interface to one CBC or multiple CBCs (i.e. the AMF is allowed to have SCTP transport associations established with one or multiple CBCs). An AMF may interface to several NG-RANs (i.e. combinations of gNodeBs and/or eNodeBs).

The AMF is responsible for:

1)	Routing the WRITE-REPLACE-WARNING-REQUEST message to the appropriate NG-RAN entities upon receiving warning message request from CBC;

2)	Routing the STOP-WARNING-REQUEST message to the appropriate NG-RAN entities upon receiving STOP-WARNING-REQUEST message from CBC;

3)	Warning message delivery success or failure notification

-	Providing to the CBC acknowledgement of successful execution of commands received from the CBC;

-	Reporting to the CBC failure when a command received from the CBC is not understood or cannot be executed; and

-	Reporting to the CBC the Broadcast completed area list, the Broadcast cancelled area list; and

4)	Forward PWS RESTART-INDICATION and the PWS FAILURE-INDICATION received from NG-RAN entities to all CBCs that the AMF interfaces with.

###### 12.1.1.3.2.3	CBC functionality

CBC is integrated as a node in the 5G core network. The CBC may be connected to several AMFs. The CBC may be connected to several CBEs. The CBC shall be responsible for the management of CBS messages as specified in subclause 5 of 3GPP TS 23.041 [16].

CBC reuses existing SBc protocol interface for warning message delivery between AMF and CBC.

###### 12.1.1.3.2.4	NG-RAN functionality

The NG-RAN may interface to multiple AMFs.

The NG-RAN is responsible for:

1)	Upon receipt of the WRITE-REPLACE WARNING REQUEST message, prioritise its resources and broadcast the warning message to the warning areas indicated in the requested Warning area list;

2)	Upon receipt of the KILL REQUEST message, stop broadcasting the warning message within for that area indicated in the requested Warning area list and discard the warning message;

3)	Inform the AMF that PWS information for some or all cells of the NG-RAN are available for reloading from the CBC if needed by sending a PWS RESTART-INDICATION message to the AMF; and

4)	Inform the AMF that ongoing PWS operation for one or more cells of the NG-RAN has failed by sending a PWS FAILURE-INDICATION message to the AMF.

##### 12.1.1.3.3	NG-RAN Protocol Overview

Legend:

-	NBc application protocol (NBc-AP): Application layer protocol between CBC and AMF. This protocol supports transfer of warning messages.

-	NG application protocol (NGAP): Application layer protocol between the 5G-RAN entity (eNodeB and/or gNodeB) and the AMF. The NGAP protocol is defined in 3GPP TS 38.413 [19].

-	SCTP for the control plane (SCTP): This protocol guarantees delivery of signalling messages between AMF and NG-RAN (N2) and between CBC and AMF (NBc). SCTP is defined in IETF RFC 4960 [24].

Figure 12.1.1.3.3.1: CBC - NG-RAN

##### 12.1.1.3.4	CBC-AMF interface

The CBC network function uses Namf_Communication service to communicate with the NG-RAN for sending warning request primitives and receiving warning message delivery indication or restart/failure indication primitives.

The requirements are described by primitives.

The following table shows primitives that are expected to be supported using AMF via Namf interface:

Table 12.1.1.3.4.1: CBC-AMF primitives via Namf interface

| Name | Originator | Type |
| --- | --- | --- |
| WRITE-REPLACE-WARNING-REQUEST | CBC | Request/Indication |
| WRITE-REPLACE-WARNING-CONFIRM | AMF | Response/Confirm |
| STOP-WARNING-REQUEST | CBC | Request/Indication |
| STOP-WARNING-CONFIRM | AMF | Response/Confirm |
| WRITE-REPLACE-WARNING-INDICATION | AMF | Request/Indication |
| STOP-WARNING-INDICATION | AMF | Request/Indication |
| RESTART-INDICATION-NG-RAN | AMF | Request/Indication |
| FAILURE-INDICATION-NG-RAN | AMF | Request/Indication |

### 12.1.2	SMS over NAS

Editor’s note:	Both the names and codings of the SMS parameters sent from the UE to the network and from the network to the UE are FFS.

#### 12.1.2.1	Registration requirements

##### 12.1.2.1.1	Initial registration initiation

If the UE requests to use SMS over NAS, the UE shall include the SMS requested IE in the REGISTRATION REQUEST message and set:

1)	the "supported accesses" bits of the SMS requested IE to:

A)	"SMS over NAS supported via 3GPP access only" if the UE supports SMS delivery over NAS via 3GPP access only; or

B)	"SMS over NAS supported via both 3GPP access and non-3GPP access" if the UE supports SMS delivery over NAS via both 3GPP access and non-3GPP access; and

2)	the "one step" bit of the SMS requested IE to:

A)	"MO SMS using one step approach not supported" if the UE does not support MO SMS using the one step approach in 5GMM-IDLE mode; or

B)	"MO SMS using one step approach supported" if the UE supports MO SMS using the one step approach in 5GMM-IDLE mode.

##### 12.1.2.1.2	Initial registration accepted by the network

If the REGISTRATION REQUEST message contained the SMS requested IE with the supported accesses bits indicating that the UE supports SMS delivery over NAS via 3GPP access only, or via both 3GPP access and non-3GPP access, and SMSF selection is successful, then the AMF shall send the REGISTRATION ACCEPT message either:

a)	prior to requesting activation of the SMS service from the SMSF; or

b)	after the SMSF has confirmed that the activation of the SMS service was successful;

NOTE:	The decision in the AMF to perform a) or b) above is based upon implementation.

Editor's note:	For case a), consideration is needed when SMS activation subsequently fails at the SMSF.

When sending the REGISTRATION ACCEPT message, the AMF shall:

a)	store the SMSF address and the contents of the SMS requested IE in the UE 5GMM context, and consider the UE available for SMS; and

b)	include the SMS allowed IE in the REGISTRATION ACCEPT message with:

1)	the "allowed accesses" bits of the SMS allowed IE to:

A)	"SMS over NAS supported via 3GPP access only" if:

i)	the UE has set the "supported accesses" bits of the SMS requested IE to "SMS over NAS supported via 3GPP access only" and the network allows the SMS delivery over NAS via 3GPP access; or

ii)	the UE has set the "supported accesses" bits of the SMS requested IE to "SMS over NAS supported via both 3GPP access and non-3GPP access" and the network allows the SMS delivery over NAS via 3GPP access only; or

B)	"SMS over NAS supported via both 3GPP access and non-3GPP access" if the UE has set the "supported accesses" bits of the SMS requested IE to "SMS over NAS supported via both 3GPP access and non-3GPP access" and the network allows the SMS delivery over NAS via both 3GPP access and non-3GPP access; and

2)	the "one step" bit of the SMS allowed IE to:

A)	"MO SMS using one step approach not allowed" if the UE indicated the support of the MO SMS using one step approach in 5GMM-IDLE mode or the network does not allow the MO SMS using one step approach; or

B)	"MO SMS using one step approach allowed" if the UE indicated the support of the MO SMS using one step approach in 5GMM-IDLE mode and the network allows the MO SMS using one step approach.

If SMSF selection in the AMF or SMS activation via the SMSF is not successful, then the AMF shall not include the SMS allowed IE in the REGISTRATION ACCEPT message.

If the AMF does not allow the use of SMS over NAS, then the AMF shall not include the SMS allowed IE in the REGISTRATION ACCEPT message.

##### 12.1.2.1.3	Mobility and periodic registration update initiation

The UE in state 5GMM-REGISTERED shall initiate the mobility registration updating procedure by sending a REGISTRATION REQUEST message to the AMF when the UE needs to request the use of SMS over NAS transport or the current requirements to use SMS over NAS transport change in the UE, as specified in subclause 12.1.2.1.1.

When initiating a periodic registration update and the requirements to use SMS over NAS transport have not changed in the UE, the UE shall not include the SMS requested IE in the REGISTRATION REQUEST message.

If the UE no longer requires the use of SMS over NAS transport, then the UE shall include the SMS requested IE in the REGISTRATION REQUEST message with the "supported accesses" bits set to "SMS over NAS not supported".

##### 12.1.2.1.4	Mobility and periodic registration update accepted by the network

If the UE requests the use of SMS over NAS transport or the requirements to use SMS over NAS transport change in the UE, the AMF shall follow the procedures specified in subclause 12.1.2.1.2.If the REGISTRATION REQUEST message contained the SMS requested IE with the "supported accesses" bits set to "SMS over NAS not supported", then the AMF shall:

a)	if an SMSF address is present in the 5GMM context, delete the SMSF address from the 5GMM context;

b)	mark the 5GMM context to indicate that SMS over NAS is not required by the UE; and

NOTE:	The AMF notifies the SMSF that the UE is deregistered from SMS over NAS.

c)	not include the SMS allowed IE in the REGISTRATION ACCEPT message.

### 12.1.3	Emergency services

#### 12.1.3.1	Indication of support of emergency services

During the registration procedure the AMF includes the emergency PDU session service support indicator in the REGISTRATION ACCEPT message (see subclause 8.6.6).

Editor’s note:	The emergency PDU session service support indicator needs to be aligned with the stage 2 as soon as procedural requirements are in place.

Editor’s note:	The emergency service support indicator needs to be added in the subclause 8.6.6.

Editor’s note:	It is FFS in which information element of the REGISTRATION ACCEPT message the emergency service support indicator is included.

The received emergency PDU session service support indicator shall be provided to the upper layers in the UE.

Editor’s note:	The use of the emergency PDU session service support indicator by upper layers is FFS.

#### 12.1.3.2	Handling of emergency service for the UE in limited service state

Editor's note:	This subclause describes how to provide emergency bearer services to the UE in limited services state, especially how the UE in limited service state registers to the 5GC

#### 12.1.3.3	Emergency PDU session setup

Editor's note:	This subclause describes the procedure of PDU session setup.

#### 12.1.3.4	Handling of local emergency numbers

The following mechanism of handling of local emergency numbers applies when the UE accesses the 5GCN via 3GPP access and non-3GPP access.

Editor’s note:	The handling of the case where the UE registers to different PLMNs via 3GPP access and non-3GPP access simultaneously and receive the emergency number list from different PLMNs is FFS.

An operator of a network may provide a list o local emergency numbers to the UE. Optionally to each local emergency number either an emergency service category or an associated sos-URN (i.e. URN with top level service type of "sos" as specified in RFC 5031 [69]) may be provided in the local emergency number list. The emergency number list can be downloaded by the network to the UE at successful completion of the registration procedure. The UE shall store and use the local emergency number list received from the network in addition to the emergency numbers stored on the UICC or user equipment to detect if the user dialled number  is an emergency number and initates an UE detected emergency session.

NOTE:	The sos-URN used by the UE to setup the emergency session is either the associated sos-URN or the sos-URN mapped to the emergency service category (as defined in 3GPP TS 24.229) if these additional information are provided.

Editor’s note:	It is FFS if there is a potential conflict with the emergency numbers list received via an access and those on the UICC.

Editor’s note:	It is FFS if there the usage the associated sos-URN is only allowed in the network where the UE received the emergency list from.

Editor’s note:	If the UE use the CS domain to setup an emergency call, it is FFS if when the user dials a number with an associted sos-URN, the UE setup a normal CS call (with dialed number) or an CS emergency call (without a number).

The network may send a list of local emergency numbers in the REGISTRATION ACCEPT message (see subclause 8.6.6).

Editor’s note:	The emergency number list needs to be added in the subclause 8.6.6.

Editor’s note:	It is FFS in which information element of the REGISTRATION ACCEPT message the emergency number list is included.

The local emergency numbers received from the network are valid only in networks in the same country as where the UE received them.

#### 12.1.3.5	Security procedures for handling emergency services

Editor's note:	This subclause describes the difference on the security aspect between emergency services procedure and normal procedure.

### 12.1.4	Location services

LPP messages shall be transported between the UE and the AMF using the NAS transport procedures specified in subclause 8.5.1.1.

Editor’s note:	It is FFS whether transport of supplementary service messages for location services as specified in 3GPP TS 24.030 is needed in 5G System.

### 12.1.5	Multimedia priority services

This sub-clause will describe the necessary work to support MPS (see 3GPP TS 23.501 [8] subclause 5.16.5 and 3GPP TS 23.502 [9] subclause 4.13.6).

## 12.2	Access control

This sub-clause will describe the CT1 relevant access control aspects to support 5GS.

## 12.3	Security

This sub-clause will describe the CT1 relevant security aspects to support 5GS.

## 12.4	Quality of service (QoS)

### 12.4.1	General

#### 12.4.1.1	QoS rules

##### 12.4.1.1.1	General

In a PDU session of PDU session type "IPv4", "IPv6", and "Ethernet", the NAS protocol enables different forwarding treatments of UL user data packets in one or more QoS flows based on signalled QoS rules, derived QoS rules or any combination of them.

In a PDU session of PDU session type "unstructured", all UL user data packets are associated with the same QoS flow.

##### 12.4.1.1.2	Signalled QoS rules

The NAS protocol enables the network to provide the UE with signalled QoS rules associated with a PDU session.

The network can provide the UE with one or more signalled QoS rules associated with a PDU session at the PDU session establishment or at QoS flow establishment.

Each signalled QoS rule contains:

a)	a QoS rule identifier (QRI);

b)	a QoS flow identity (QFI);

c)	optionally a set of packet filters;

d)	precedence values;

e)	if the flow is a GBR QoS flow:

1)	Guaranteed flow bit rate (GFBR) for UL;

2)	Guaranteed flow bit rate (GFBR) for DL;

3) Maximum flow bit rate (MFBR) for UL; and

4)	Maximum flow bit rate (MFBR) for DL; and

Editor's note:	The length of the 5QI, QFI and QRI is FFS.

Within a PDU session:

a)	each signalled QoS rule has a unique QRI;

b)	there is at least one signalled QoS rule;

c)	there can be zero, one or more signalled QoS rules associated with a given QFI; and

d)	there can be up to one signalled QoS rule without a set of packet filters.

e)	5QI, if the QFI is not the same as the 5QI of the QoS flow identified by the QFI.

##### 12.4.1.1.3	Derived QoS rules

The reflective QoS in the UE creates derived QoS rules associated with a PDU session based on DL user data packets received via the PDU session.

Each derived QoS rule contains:

a)	a QoS flow identity (QFI);

b)	an UL packet filter; and

c)	a precedence value.

Within a PDU session:

a)	there can be zero, one or more derived QoS rules associated with a given QFI; and

b)	there can be up to one derived QoS rule associated with a given UL packet filter.

In the UE, a timer X runs for each derived QoS rule.

#### 12.4.1.2	Session-AMBR

The NAS protocol enables the network to provide the UE with the session-AMBR associated with a PDU session.

#### 12.4.1.3	UL user data packet matching

Upon receiving an UL user data packet from the upper layers for transmission via a PDU session, the UE shall attempt to associate the UL user data packet with:

a)	the QFI of a signalled QoS rule associated with the PDU session which has a set of packet filters containing an UL packet filter matching the UL user data packet;

b)	the QFI of a derived QoS rule associated with the PDU session which has the UL packet filter matching the UL user data packet; or

c)	the QFI of a signalled QoS rule associated with the PDU session which does not have a set of packet filters;

by evaluating the QoS rules in increasing order of their precedence values until the UL user data packet is associated with a QFI or all QoS rules are evaluated.

If the UL user data packet is associated with a QFI, the UE shall pass the QFI along the UL user data packet to the lower layers for transmission.

NOTE:	Marking of the UL user data packet with the QFI is performed by the lower layers.

If all QoS rules are evaluated and the UL user data packet is not associated with a QFI, the UE shall discard the UL user data packet.

#### 12.4.1.4	Reflective QoS

##### 12.4.1.4.1	General

The UE may support reflective QoS.

If the UE supports the reflective QoS, the UE shall support the procedures in the following subclauses.

The reflective QoS is applicable in a PDU session of PDU session type "IPv4", "IPv6" and "Ethernet". The reflective QoS is not applicable in a PDU session of PDU session type "unstructured".

##### 12.4.1.4.2	Derivation of UL packet filter from DL user data packet

The UE shall derive an UL packet filter from a DL user data packet as follows:

a)	if the received DL user data packet belongs to a PDU session of PDU session type "IPv4", the UL packet filter contains the following packet filter components:

1)	an IPv4 remote address component set to the source IP address of the received DL user data packet;

2)	an IPv4 local address component set to the destination IP address of the received DL user data packet;

3)	a Protocol identifier/Next header type component set to the value of the received protocol type field; and

4)	if the received protocol type field indicates a protocol specifying usage of ports:

A)	a Single local port type component set to value of the destination port in the received DL user data packet; and

B)	a Single remote port type component set to the value of the source port in the received DL user data packet;

b)	if the received DL user data packet belongs to a PDU session of PDU session type "IPv6", the UL packet filter contains the following packet filter components:

1)	an IPv6 remote address set to the source IP address of the received DL user data packet;

2)	an IPv6 local address/prefix length component set to the destination IP address of the received DL user data packet;

3)	a Protocol identifier/Next header type component set to the value of the last received next header field; and

4)	if the last received next header field indicates a protocol specifying usage of ports:

A)	a Single local port type component set to value of the destination port in the received DL user data packet; and

B)	a Single remote port type component set to the value of the source port in the received DL user data packet; and

c)	if the received DL user data packet belongs to a PDU session of PDU session type "Ethernet", the UL packet filter contains the following packet filter components:

1)	a destination MAC address component set to the source MAC address of the received DL packet;

2)	a source MAC address component set to the destination MAC address of the received DL packet;

3)	if an 802.1Q C-TAG is included in the received DL packet, an 802.1Q C-TAG VID component set to the 802.1Q C-TAG VID of the received DL packet and an 802.1Q C-TAG PCP/DEI component set to the 802.1Q C-TAG PCP/DEI of the received DL packet;

4)	if an 802.1Q S-TAG is included in the received DL packet, an 802.1Q S-TAG VID component set to the 802.1Q S-TAG VID of the received DL packet and an 802.1Q S-TAG PCP/DEI component set to the 802.1Q S-TAG PCP/DEI of the received DL packet;

5)	If the Ethertype field of the received DL packet is set to a value of 1536 or above, an Ethertype component set to the Ethertype of the received DL packet;

6)	if the Ethertype field of the Ethernet frame header indicates that the data carried in the Ethernet frame is IPv4 data, the UE shall also add to the UL packet filter IP-specific components based on the contents of the DL user data IP header as described in bullet a) above; and

7)	if the Ethertype field of the Ethernet frame header indicates that the data carried in the Ethernet frame is IPv6 data, the UE shall also add to the UL packet filter IP-specific components based on the contents of the DL user data IP header as described in bullet b) above.

Editor's note: use of filters for PDU session type "Ethernet" is to be confirmed by SA2.

##### 12.4.1.4.3	Creating a derived QoS rule by reflective QoS in the UE

If the UE receives a DL user data packet marked with a QFI and an RQI, the DL user data packet belongs to a PDU session of PDU session type "IPv4", "IPv6", or "Ethernet", and the UE does not have a derived QoS rule with the same UL packet filter as the UL packet filter derived from the DL user data packet as specified in subclause 12.4.1.5.0A, then the UE shall create a new derived QoS rule as follows:

a)	the QFI of the derived QoS rule is set to the received QFI;

b)	the precedence value of the derived QoS rule is set to a default standardized precedence value; and

Editor's note:	FFS what is the default standardized precedence value.

c)	the UL packet filter of the derived QoS rule is set to the derived UL packet filter;

and the UE shall start the timer X associated with the derived QoS rule with the RQ timer value received during the UE-requested PDU session establishment procedure as specified in subclause 9.4.2. If the RQ timer value was not received in the UE-requested PDU session establishment procedure, the default standardized RQ timer value is used.

Editor's note: FFS what is the default standardized RQ timer value.

##### 12.4.1.4.4	Prolonging lifetime of a derived QoS rule by reflective QoS in the UE

If the UE receives a DL user data packet associated with a QFI and an RQI, the DL user data packet belongs to a PDU session of PDU session type "IPv4", "IPv6", or "Ethernet", and the UE has a derived QoS rule with the same UL packet filter as the UL packet filter derived from the DL user data packet as specified in subclause 12.4.1.5.0A, then the UE shall re-start the timer X associated with the derived QoS rule with the RQ timer value received during the UE-requested PDU session establishment procedure as specified in subclause 9.4.2. If the RQ timer value was not received in the UE-requested PDU session establishment procedure, the default standardized RQ timer value is used.

Editor's note: FFS what is the default standardized RQ timer value.

##### 12.4.1.4.4	Deleting a derived QoS rule by reflective QoS in the UE

Upon expiration of timer X associated with a derived QoS rule, the UE shall remove the derived QoS rule.

### 12.4.2	QoS coding

#### 12.4.2.1	QoS rules coding

The purpose of the QoS rules information element is to specify the set of parameters that are used by the UE for classification and marking of uplink user traffic. The QoS rules may contain packet filters for the uplink direction. The packet filters determine the traffic mapping to QoS flows.

The QoS rules information element is a type 6 information element with a minimum length of 4 octets. The maximum length for the information element is 65 538 octets.

The Qos rules information element is coded as shown in figure 12.4.2.1.1 and table 12.4.2.1.1.

|  |  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QoS rules IEI |  |  |  |  |  |  |  |  | Octet 1 |  |
|  |  | Length of QoS rules IE |  |  |  |  |  |  |  |  | Octet 2 |  |
|  |  |  |  |  |  |  |  |  |  |  | Octet 3 |  |
|  |  | QoS rule 1 |  |  |  |  |  |  |  |  | Octet 4 Octet u |  |
|  |  | QoS rule 2 |  |  |  |  |  |  |  |  | Octet u+1 Octet v |  |
|  |  | … |  |  |  |  |  |  |  |  | Octet v+1 Octet w |  |
|  |  | QoS rule n |  |  |  |  |  |  |  |  | Octet w+1 Octet x |  |

Figure 12.4.2.1.1: QoS rules information element

|  |  | 8 | 7 | 6 | 5 |  | 4 |  | 3 | 2 | 1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | QoS rule identifier |  |  |  |  |  |  |  |  |  |  | Octet 1 |  |
|  |  | Length of QoS rule |  |  |  |  |  |  |  |  |  |  | Octet 2 |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | Octet 3 |  |
|  |  | QoS rule precedence |  |  |  |  |  |  |  |  |  |  | Octet 5 |  |
|  |  | QoS flow identifier |  |  |  |  |  |  |  |  |  |  | Octet 6 |  |
|  |  | Rule operation code |  |  |  | E bit |  | Number of packet filters |  |  |  |  | Octet 7 |  |
|  |  | Packet filter list |  |  |  |  |  |  |  |  |  |  | Octet 8 Octet z |  |
|  |  | Parameters list |  |  |  |  |  |  |  |  |  |  | Octet z+1 Octet v |  |

Figure 12.4.2.1-2: QoS rule

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 1 |  |  |  | Octet 5 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 2 |  |  |  | Octet 6 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | … |  |  |  |  |  |  |  |  |
|  | 0 | 0 | 0 | 0 | Packet filter identifier N |  |  |  | Octet N+3 |
|  | Spare |  |  |  |  |  |  |  |  |

Figure 12.4.2.1-3: Packet filter list when the rule operation is "delete packet filters from existing QoS rule" (z=N+3)

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 1 |  |  |  | Octet 5 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Packet filter evaluation precedence 1 |  |  |  |  |  |  |  | Octet 6 |
|  | Length of Packet filter contents 1 |  |  |  |  |  |  |  | Octet 7 |
|  | Packet filter contents 1 |  |  |  |  |  |  |  | Octet 8 Octet m |
|  | 0 | 0 | 0 | 0 | Packet filter identifier 2 |  |  |  | Octet m+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Packet filter evaluation precedence 2 |  |  |  |  |  |  |  | Octet m+2 |
|  | Length of Packet filter contents 2 |  |  |  |  |  |  |  | Octet m+3 |
|  | Packet filter contents 2 |  |  |  |  |  |  |  | Octet m+4 Octet n |
|  | … |  |  |  |  |  |  |  | Octet n+1 Octet y |
|  | 0 | 0 | 0 | 0 | Packet filter identifier N |  |  |  | Octet y+1 |
|  | Spare |  |  |  |  |  |  |  |  |
|  | Packet filter evaluation precedence N |  |  |  |  |  |  |  | Octet y+2 |
|  | Length of Packet filter contents N |  |  |  |  |  |  |  | Octet y+3 |
|  | Packet filter contents N |  |  |  |  |  |  |  | Octet y+4 Octet z |

Figure 12.4.2.1-4: Packet filter list when the rule operation is "create new QoS rule", or "add packet filters to existing QoS rule" or "replace packet filters in existing QoS rule"

|  | 8 | 7 | 6 | 5 | 4 | 3 | 2 | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Parameter identifier 1 |  |  |  |  |  |  |  | Octet z+1 |
|  | Length of Parameter contents 1 |  |  |  |  |  |  |  | Octet z+2 |
|  | Parameter contents 1 |  |  |  |  |  |  |  | Octet z+3 Octet k |
|  | Parameter identifier 2 |  |  |  |  |  |  |  | Octet k+1 |
|  | Length of Parameter contents 2 |  |  |  |  |  |  |  | Octet k+2 |
|  | Parameter contents 2 |  |  |  |  |  |  |  | Octet k+3 Octet p |
|  | … |  |  |  |  |  |  |  | Octet p+1 Octet q |
|  | Parameter identifier N |  |  |  |  |  |  |  | Octet q+1 |
|  | Length of Parameter contents N |  |  |  |  |  |  |  | Octet q+2 |
|  | Parameter contents N |  |  |  |  |  |  |  | Octet q+3 Octet v |

Figure 12.4.2.1-5: Parameters list

Table 12.4.2.1.1: QoS rule flow template information element

QoS rule identifier (octet 4) The QoS rule identifier field is used to identify the QoS rule. QoS rule precedence (octet 5) The QoS rule identifier field is used to specify the precedence of the QoS rule among all QoS rules associated with this QoS flow. Higher the value of the Qos rule precedence field, lower the precedence of that QoS rule is. QoS flow identifier (octet 6) The QoS flow identifier field is used to identify the QoS flow. Rule operation code (octet 7) Bits 8 7 6 0 0 0 Ignore this IE 0 0 1 Create new QoS rule 0 1 0 Delete existing QoS rule 0 1 1 Add packet filters to existing QoS rule 1 0 0 Replace packet filters in existing QoS rule 1 0 1 Delete packet filters from existing QoS rule 1 1 0 No packet filter operation 1 1 1 Reserved The Rule operation code "No packet filter operation" shall be used if a parameters list is included but no packet filter list is included in the traffic flow template information element. The Rule operation code "Ignore this IE" shall be used by the UE if the QoS rule information element has presence requirement "M" in a message, but the information element does not serve any useful purpose in the specific procedure for which the message is sent. If the Rule operation code indicates "Ignore this IE", the UE shall also set the E bit and the number of packet filters to zero. If the Rule operation code is set to "Ignore this IE" and the the E bit and the number of packet filters to zero, then the network shall ignore the contents of the QoS rule flow template information element. E bit (bit 5 of octet 7) The E bit indicates if a parameters list is included in the QoS rule IE and it is encoded as follows: 0 parameters list is not included 1 parameters list is included Number of packet filters (octet 7) The number of packet filters contains the binary coding for the number of packet filters in the packet filter list. The number of packet filters field is encoded in bits 4 through 1 of octet 3 where bit 4 is the most significant and bit 1 is the least significant bit. For the "delete existing QoS rule" operation and for the "no packet filter operation", the number of packet filters shall be coded as 0. For all other operations, the number of packet filters shall be greater than 0 and less than or equal to 15. Packet filter list (octets 8 to z) The packet filter list contains a variable number of uplink packet filters. For the "delete existing QoS rule" operation and the "no packet filter operation", the packet filter list shall be empty. For the "delete packet filters from existing QoS rule" operation, the packet filter list shall contain a variable number of packet filter identifiers. This number shall be derived from the coding of the number of packet filters field in octet 7. For the "create new QoS rule" operation, the packet filter list shall contain 0 or a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 7. For the "add packet filters to existing QoS rule" and "replace packet filters in existing QoS rule" operations, the packet filter list shall contain a variable number of packet filters. This number shall be derived from the coding of the number of packet filters field in octet 7. Each packet filter is of variable length and consists of - a packet filter identifier (4 bits); - a packet filter evaluation precedence (1 octet); - the length of the packet filter contents (1 octet); and - the packet filter contents itself (v octets). The packet filter identifier field is used to identify each packet filter in a QoS rule. The least significant 4 bits are used. The packet filter evaluation precedence field is used to specify the precedence for the packet filter among all packet filters in the Qos rule. Higher the value of the packet filter evaluation precedence field, lower the precedence of that packet filter is. The first bit in transmission order is the most significant bit. The length of the packet filter contents field contains the binary coded representation of the length of the packet filter contents field of a packet filter. The first bit in transmission order is the most significant bit. The packet filter contents field is of variable size and contains a variable number (at least one) of packet filter components. Each packet filter component shall be encoded as a sequence of a one octet packet filter component type identifier and a fixed length packet filter component value field. The packet filter component type identifier shall be transmitted first. In each packet filter, there shall not be more than one occurrence of each packet filter component type. Among the "IPv4 remote address type" and "IPv6 remote address type" packet filter components, only one shall be present in one packet filter. Among the "single local port type" and "local port range type" packet filter components, only one shall be present in one packet filter. Among the "single remote port type" and "remote port range type" packet filter components, only one shall be present in one packet filter. The term local refers to the UE and the term remote refers to an external network entity. Packet filter component type identifier Bits 8 7 6 5 4 3 2 1 0 0 0 1 0 0 0 0 IPv4 remote address type 0 0 0 1 0 0 0 1 IPv4 local address type 0 0 1 0 0 0 0 0 IPv6 remote address type 0 0 1 0 0 0 0 1 IPv6 remote address/prefix length type 0 0 1 0 0 0 1 1 IPv6 local address/prefix length type 0 0 1 1 0 0 0 0 Protocol identifier/Next header type 0 1 0 0 0 0 0 0 Single local port type 0 1 0 0 0 0 0 1 Local port range type 0 1 0 1 0 0 0 0 Single remote port type 0 1 0 1 0 0 0 1 Remote port range type 0 1 1 0 0 0 0 0 Security parameter index type 0 1 1 1 0 0 0 0 Type of service/Traffic class type 1 0 0 0 0 0 0 0 Flow label type 1 0 0 0 0 0 0 1 Destination MAC address type 1 0 0 0 0 0 1 0 Source MAC address type 1 0 0 0 0 0 1 1 802.1Q C-TAG VID type 1 0 0 0 0 1 0 0 802.1Q S-TAG VID type 1 0 0 0 0 1 0 1 802.1Q C-TAG PCP/DEI type 1 0 0 0 0 1 1 0 802.1Q S-TAG PCP/DEI type 1 0 0 0 0 1 1 1 Ethertype type All other values are reserved. # Editor's note: Ethernet related packet filters are to be confirmed by SA2. The description and valid combinations of packet filter component type identifiers in a packet filter are defined in 3GPP TS 23.060 [74] subclause 15.3.2. For "IPv4 remote address type", the packet filter component value field shall be encoded as a sequence of a four octet IPv4 address field and a four octet IPv4 address mask field. The IPv4 address field shall be transmitted first. For "IPv4 local address type", the packet filter component value field shall be encoded as defined for "IPv4 remote address type". Both the UE and network indication for support of the Local address in QoS rule are required to use this packet filter component. For "IPv6 remote address type", the packet filter component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and a sixteen octet IPv6 address mask field. The IPv6 address field shall be transmitted first. For "IPv6 remote address/prefix length type", the packet filter component value field shall be encoded as a sequence of a sixteen octet IPv6 address field and one octet prefix length field. The IPv6 address field shall be transmitted first. This parameter shall be used, instead of IPv6 remote address type, when both the UE and network indication for support of the Local address in QoS rule are present. For "IPv6 local address/prefix length type", the packet filter component value field shall be encoded as defined for "IPv6 remote address /prefix length". Both the UE and network indication for support of the Local address in QoS rule are required to use this packet filter component. NOTE: Local IP address and mask can be used when IPv6 prefix delegation is used (see 3GPP TS 23.060 [74] subclause 9.2.1.2). For "Protocol identifier/Next header type", the packet filter component value field shall be encoded as one octet which specifies the IPv4 protocol identifier or IPv6 next header. For "Single local port type" and "Single remote port type", the packet filter component value field shall be encoded as two octet which specifies a port number. For "Local port range type" and "Remote port range type", the packet filter component value field shall be encoded as a sequence of a two octet port range low limit field and a two octet port range high limit field. The port range low limit field shall be transmitted first. For "Security parameter index", the packet filter component value field shall be encoded as four octets which specifies the IPSec security parameter index. For "Type of service/Traffic class type", the packet filter component value field shall be encoded as a sequence of a one octet Type-of-Service/Traffic Class field and a one octet Type-of-Service/Traffic Class mask field. The Type-of-Service/Traffic Class field shall be transmitted first. For "Flow label type", the packet filter component value field shall be encoded as three octets which specifies the IPv6 flow label. The bits 8 through 5 of the first octet shall be spare whereas the remaining 20 bits shall contain the IPv6 flow label. For "Destination MAC address type" and "Source MAC address type", the packet filter component value field shall be encoded as 6 octets which specify a MAC address. For "802.1Q C-TAG VID type", the packet filter component value field shall be encoded as two octets which specify the VID of the Customer-VLAN tag (C-TAG). The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. For "802.1Q S-TAG VID type", the packet filter component value field shall be encoded as two octets which specify the VID of the Service-VLAN tag (S-TAG). The bits 8 through 5 of the first octet shall be spare whereas the remaining 12 bits shall contain the VID. For "802.1Q C-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q C-TAG PCP and DEI. The bits 8 through 5 of the octet shall be spare, the bits 4 through 2 contain the PCP and bit 1 contains the DEI. For "802.1Q S-TAG PCP/DEI type", the packet filter component value field shall be encoded as one octet which specifies the 802.1Q S-TAG PCP. The bits 8 through 5 of the octet shall be spare, the bits 4 through 2 contain the PCP and bit 1 contains the DEI. For "Ethertype type", the packet filter component value field shall be encoded as two octets which specify an Ethertype. Parameters list (octets z+1 to v) The parameters list contains a variable number of parameters that may be transferred. If the parameters list is included, the E bit is set to 1; otherwise, the E bit is set to 0. Each parameter included in the parameters list is of variable length and consists of: - a parameter identifier (1 octet); - the length of the parameter contents (1 octet); and - the parameter contents itself (v octets). The parameter identifier field is used to identify each parameter included in the parameters list and it contains the hexadecimal coding of the parameter identifier. Bit 8 of the parameter identifier field contains the most significant bit and bit 1 contains the least significant bit. In this version of the protocol, the following parameter identifiers are specified: - 01H (5QI); - 02H (GFBR Uplink); - 03H (GFBR Downlink); - 04H (MFBR Uplink); and - 05H (MFBR Downlink). If the parameters list contains a parameter identifier that is not supported by the receiving entity the corresponding parameter shall be discarded. The length of parameter contents field contains the binary coded representation of the length of the parameter contents field. The first bit in transmission order is the most significant bit. When the parameter identifier indicates ARP, the parameter contents field contains the binary representation of ARP value that is one octet in length, When the parameter identifier indicates 5QI, the parameter contents field contains the binary representation of 5G QoS identifier that is one octet in length, When the parameter identifier indicates GFBR Uplink, the parameter contents field contains the binary representation of Guaranteed flow bit rate for uplink that is four octet in length, When the parameter identifier indicates GFBR Downlink, the parameter contents field contains the binary representation of Guaranteed flow bit rate for downlink that is four octet in length, When the parameter identifier indicates MFBR Uplink, the parameter contents field contains the binary representation of Maximum flow bit rate for uplink that is four octet in length, When the parameter identifier indicates MFBR Downlink, the parameter contents field contains the binary representation of Maximum flow bit rate for downlink that is three octet in length, When the parameter identifier indicates Packet Filter Identifier, the parameter contents field contains the binary representation of one or more packet filter identifiers. Each packet filter identifier is encoded in one octet, in the 4 least significant bits. This parameter is used by the MS and the network to identify one or more packet filters in a TFT when modifying the QoS rule without modifying the packet filter itself.

#### 12.4.2.2	Session-AMBR coding

Editor's note:	The exact coding of the session-AMBR is FFS.

## 12.5	IM CN subsystem

### 12.5.1	IMS voice over PS Session supported Indication

The network informs the UE about the support of IMS voice over PS session In a UE, the IMS voice over PS session indicator (IMS VoPS) shall be provided to the upper layers. When initiating a voice call, the upper layers take the IMS voice over PS session support indicator into account.

Editor’s note:	The use of the IMS voice over PS session by upper layers is FFS.

Editor's note:	It is FFS if there are other methods indicating that the UE can expect a successful IMS voice over PS session.

During the registration procedure the AMF includes the IMS voice over PS session supported indicator in the REGISTRATION ACCEPT message (see subclause 8.6.6).

Editor’s note:	The emergency PDU session service support indicator needs to be aligned with the stage 2 as soon as procedural requirements are in place.

Editor’s note:	The emergency service support indicator needs to be added in the subclause 8.6.6.

Editor’s note:	It is FFS in which information element of the REGISTRATION ACCEPT message the emergency service support indicator is included.

When indicating support of IMS voice over PS session to the UE, the AMF should take into account the following factors:

1)	the local policy of the PLMN to which the AMF belongs;

2)	the capabilities of the network;

3)	the capabilities of the UE; and

4)	the roaming agreement between the VPLMN and the HPLMN.

### 12.5.2	P-CSCF address delivery

When the UE requests to establish a PDU session, which can be used for IMS services, the UE includes in the ePCO IE contained in the PDU SESSION ESTABLISHMENT REQUEST message (see subclause 9.5.3) the request for P-CSCF IPv4 address or P-CSCF IPv6 address or both. The AMF forwards the ePCO IE transparently via N11 interface towards the SMF.

When receiving an 5GSM request message containing a P-CSCF address request on the N11 interface, the SMF includes the locally configured P-CSCF address(es) in the ePCO IE of the corresponding 5GSM response message.

## 12.6	Network sharing

This sub-clause will describe the CT1 relevant network sharing aspects to support 5GS (see 3GPP TS 23.501 [8] subclause 5.18).

## 12.7	Congestion and overload control

This sub-clause will describe the CT1 relevant congestion and overload control aspects to support 5GS (see 3GPP TS 23.501 [8] subclause 5.19).

## 12.8	Charging

This sub-clause will describe the CT1 relevant charging aspects to support 5GS.

## 12.9	Mission critical services

This sub-clause will describe the CT1 relevant mission critical services to support 5GS (see 3GPP TS 23.501 [8] subclause 5.16.6 and 3GPP TS 23.502 [9] subclause 4.13.8).

# 13	Network slicing

## 13.1	General

The 5GS supports network slicing as described in 3GPP TS 23.501 [5]. Within a PLMN, a network slice is identified by an S-NSSAI, which is comprised of a slice/service type (SST) and a slice differentiator (SD). Inclusion of an SD in an S-NSSAI is optional. A set of one or more S-NSSAIs is called the NSSAI. There are multiple types of NSSAI (see 3GPP TS 23.501 [8]):

-	default NSSAI;

-	configured NSSAI;

-	requested NSSAI; and

-	allowed NSSAI.

The HPLMN may configure a UE with the configured NSSAI per PLMN.

Editor's note:	Terms on network slicing can be revisited.

## 13.2	Mobility management aspects

### 13.2.1	General

Upon registration to a PLMN, the UE shall send to the AMF the requested NSSAI including one or more S-NSSAIs corresponding to the slice(s) to which the UE wishes to register if the UE has a configured NSSAI and/or an allowed NSSAI for the PLMN according to the rules defined in 3GPP TS 23.501 [8]. The UE NAS shall also provide the lower layers with the requested NSSAI, if available. If the UE has neither a configured NSSAI nor an allowed NSSAI for a PLMN, the UE does not send a requested NSSAI when requesting registration towards the PLMN. In roaming scenarios the requested NSSAI, included in the REGISTRATION REQUEST message shall include only the S-NSSAI values that are valid in the VPLMN. The AMF verifies if the requested NSSAI is permitted based on the UE subscription, and if so then the AMF may provide the UE with the allowed NSSAI for the PLMN. See subclause 8.5.2.2for further details.

The set of network slice(s) for a UE can be changed at any time while the UE is registered to a PLMN, and may be initiated by the network, or the UE. In this case, the allowed NSSAI and associated registration area may be changed during the registration procedure. The network may notify the UE of the change of the supported network slice(s) in order to trigger the registration procedure. Change in the allowed NSSAI may lead to AMF relocation subject to operator policy.

A single UE can be served by at most eight slices at a time. Multiple instances of S-NSSAI may be deployed in a given registration area. The AMF is common across multiple such slice instances.

Editor's note:	The subclause for the notification procedure will be included in the TR and the subclause will be referred.

### 13.2.2	NSSAI storage

Configured NSSAI as well as allowed NSSAI is a set composed of one or more S-NSSAIs associated with a PLMN identity. There shall be no duplicated PLMN identities in each of the list of configured NSSAI(s) and the list of allowed NSSAI(s).

If available, the configured NSSAI(s) and/or allowed NSSAI(s) shall be stored in a non-volatile memory in the ME together with the SUPI from the USIM per PLMN. In addition, the allowed NSSAI(s) shall be marked as valid for current registration area, assigned to the UE during registration procedure. The allowed NSSAI(s) shall be stored until new allowed NSSAI(s) are received for a given PLMN, the UE is switched off, or the USIM is removed. The configured NSSAI(s) and/or allowed NSSAI(s) can only be used if the SUPI from the USIM matches the SUPI stored in the non-volatile memory of the ME; else the UE shall delete them.

## 13.3	Session management aspects

In order for data transmission in a network slice, the UE may request establishment of a PDU session in a network slice towards a data network (DN) which is associated with an S-NSSAI and a data network name (DNN). The S-NSSAI included is part of allowed NSSAI and in roaming scenarios the S-NSSAI value is valid in VPLMN. See subclause 9.4.2 for further details.

The network may provision the UE with NSSP. The UE selects a PDU session taking into account NSSP (if provisioned by the network) and other conditions in the UE (if any).

Editor's note:	Whether the UE configuration update procedure can be used to provision NSSP is FFS.

# 14	Conclusions and recommendations

This clause will contain the conclusions and recommendations of the study including the organization of normative specifications for 5G Phase 1.

Annex A:
Impacts to specifications

Editor's note:	The list of specifications will be updated according to the latest status of the 5GS work in other WGs (e.g., SA1, SA2, SA3, RAN WGs) and discussions in CT1.

## A.1	New specifications

The table A.1.1 identifies new specifications that are required to define the CT1 aspects of the 5GS.

Table A.1.1: New specifications for the CT1 aspects of the 5GS

| Type | Series | Title | Scope |
| --- | --- | --- | --- |
| TS | 24.xyz | Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3 | This TS will be the stage 3 part of 3GPP TS 23.501 [8] and 3GPP TS 23.502 [9] covering protocols for mobility management, session management, and control of NAS security for both 3GPP access and non-3GPP access. Furthermore, the TS would provide support of inter-system mobility between; NG-RAN and E-UTRAN, and between NG-RAN and non-3GPP access network |
| TS | 24.xyz | Access to the 5G System (5GS) via non-3GPP access networks; Stage 3 | This TS will cover the particular aspects of non-3GPP access network discovery and selection procedures which differ from 3GPP access, the authentication and tunnel management procedures with N3IWF, IP mobility mode selection, and in general, any procedure associated with accessing the 5GCN via non-3GPP accesses until the UE has gained IP connectivity through the N3IWF |

## A.2	Impacted existing specifications

The table A.2.1 identifies impacted existing specifications that are required to define the CT1 aspects of the 5GS.

Table A.2.1: Impacted specifications for the CT1 aspects of the 5GS

| Type | Series | Title | Brief description of the impacts |
| --- | --- | --- | --- |
| TS | 23.122 | Non-Access-Stratum functions related to Mobile Station (MS) in idle mode | Enhancements of PLMN selection procedures and access control to support NR |
| TS | 23.040 | Technical realization of Short Message Service (SMS) | Support of SMS for 5G System |
| TS | 23.041 | Technical realization of Cell Broadcast Service (CBS) | Support of PWS for 5G System |
| TS | 24.007 | Mobile radio interface signalling layer 3; General aspects | Protocol principles for the NAS protocol for 5G System |
| TS | 24.301 | Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3 | Inter-system mobility between NR and E-UTRAN. Updates to IEs which can be used by 5GS protocols |
| TS | 24.008 | Mobile Radio Interface Layer 3 specification; Core Network Protocols; Stage 3 | Possible impacts, e.g. updates to Ies which can be used by 5GS protocols |
| TS | 24.302 | Access to the 3GPP Evolved Packet Core (EPC) via non-3GPP access networks; Stage 3 | Possible impacts, e.g. updates to Ies which can be used by 5GS protocols |
| TS | 24.011 | Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface | Updates to the SMS protocols for 5G System |
| TS | 24.312 | Access Network Discovery and Selection Function (ANDSF) Management Object (MO) | Possible impacts, e.g. update to ANDSF Mos to include aspects of the 5G System |
| TS | 24.305 | Selective Disabling of 3GPP User Equipment Capabilities (SdoUE) Management Object (MO) | Updates to selective disabling of UE (SdoUE) for 5G System procedures |
| TS | 24.368 | Non-Access Stratum (NAS) configuration Management Object (MO) | Possible impacts, e.g. update to existing or new leaves to support 5G System |

Annex B (informative):
Proposed changes to 3GPP TS 23.040

# B.1	First Change

# 1	Scope

The present document describes the Short Message Service (SMS) for GSM/UMTS/EPS/5GS networks. It defines:

-	the services and service elements;

-	the network architecture;

-	the Service Centre functionality;

-	the SMS Router functionality;

-	the MSC functionality (with regard to the SMS);

-	the SGSN functionality (with regard to the SMS);

-	the MME functionality (with regard to the SMS);

-	the SMSF functionality;

-	the routing requirements;

-	the protocols and protocol layering;

for the Mobile Originated and Mobile Terminated Short Message Service Teleservices, as specified in  3GPP TS 22.003 [2] and 3GPP TS 22.105 [32].

The use of radio resources for the transfer of short messages between the MS and the MSC or the SGSN or the MME is described in 3GPP TS 24.011 [13].

The present document also describes the Short Message Service (SMS) for EPS networks supported via "SMS in MME" (see 3GPP TS.23.272 [45], annex C).

The present document also describes the SGSN supporting Diameter based protocol for SMS between the SGSN and the central SMS functions (SMS-GMSC, SMS-IWMSC, SMS-Router) in annex J.

The present document describes SMS for 5GS networks (see 3GPP TS 23.501 [xx] and 3GPP TS 23.502 [yy]).

The network aspects of Short Message Service provision are outside the scope of the present document (i.e. the provision of network connectivity between the PLMN subsystems). There is no technical restriction within the present document for the transfer of short messages between different PLMNs. Any such restriction is likely to be subject to commercial arrangements and PLMN operators must make their own provision for interworking or for preventing interworking with other PLMNs as they see fit.

The required and assumed network service offered to the higher layers is defined in the present document.

# B.2	Next Change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void

[2]	3GPP TS 22.003: "Circuit Teleservices supported by a Public Land Mobile Network (PLMN)".

[3]	3GPP TS 22.004: "General on supplementary services".

[4]	3GPP TS 22.041: "Operator Determined Barring (ODB)".

[5]	3GPP TS 23.002: "Network architecture".

[6]	3GPP TS 23.008: "Organization of subscriber data".

[7]	3GPP TS 23.011: "Technical realization of supplementary services".

[8]	3GPP TS 23.015: "Technical realization of Operator Determined Barring (ODB)".

[9]	3GPP TS 23.038: "Alphabets and language-specific information".

[10]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[11]	Void

[12]	3GPP TS 44.008: "Mobile radio interface layer 3 specification".

[13]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) support on mobile radio interface".

[14]	3GPP TS 27.005: "Use of Data Terminal Equipment - Data Circuit terminating Equipment (DTE - DCE) interface for Short Message Service (SMS) and Cell Broadcast Service (CBS)".

[15]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[16]	3GPP TS 51.011 Release 4 (version 4.x.x): "Specification of the Subscriber Identity Module - Mobile Equipment (SIM- ME) interface".

[17]	CCITT Recommendation E.164 (Blue Book): "The international public telecommunication numbering plan".

[18]	CCITT Recommendation E.163 (Blue Book): "Numbering plan for the international telephone service".

[19]	CCITT Recommendation Q.771: "Specifications of Signalling System No.7; Functional description of transaction capabilities".

[20]	CCITT Recommendation T.100 (Blue Book): "International information exchange for interactive videotex".

[21]	CCITT Recommendation T.101 (Blue Book): "International interworking for videotex services".

[22]	CCITT Recommendation X.121 (Blue Book): "International numbering plan for public data networks".

[23]	CCITT Recommendation X.400 (Blue Book): "Message handling services: Message handling system and service overview".

[24]	ISO/IEC10646: "Universal Multiple-Octet Coded Character Set (USC); UCS2, 16 bit coding".

[25]	3GPP TS 22.022: "Personalisation of Mobile Equipment (ME); Mobile functionality specification".

[26]	3GPP TS 23.042: "Compression Algorithm for Text Messaging Services".

[27]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[28]	3GPP TS 31.115: "Secured packet structure for (U)SIM toolkit application".

[29]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[30]	3GPP TS 31.102: "Characteristics of the USIM application".

[31]	3GPP TS 31.101: "UICC – Terminal interface; Physical and logical characteristics".

[32]	3GPP TS 22.105: "Services and Service Capabilites".

[33]	Infrared Data Association. Specifications for Ir Mobile Communications (IrMC).
iMelody.

[34]	IETF RFC 5322 (October 2008): "Internet Message Format".

[35]	Void

[36]	"vCard - The Electronic Business Card", version 2.1,The Internet Mail Consortium (IMC), September 18, 1996, 
URL:http://www.imc.org/pdi/vcard-21.doc".

[37] 	"vCalendar - the Electronic Calendaring and Scheduling Format", version 1.0,
The Internet Mail Consortium (IMC), September 18, 1996,

URL:http://www.imc.org/pdi/vcal-10.doc

[38] 	Scalable Polyphony MIDI Specification, MIDI Manufacturers Association (2002); http://www.midi.org

[39] 	Scalable Polyphony MIDI Device 5-to-24 Note Profile for 3GPP, MIDI Manufacturers Association (2002); http://www.midi.org

[40] 	The Complete MIDI 1.0 Detailed Specification, Incorporating all Recommended Practices, MIDI Manufacturers Association, Document version 96.1, 1996; http://www.midi.org

[41]	3GPP TS 23.097: Multiple Subscriber Profile (MSP) (Phase 2) - Stage 2

[42]	3GPP TS 23.204: "Support of SMS over generic 3GPP IP access; Stage 2".

[43]	IETF RFC 3261 (June 2002): "SIP: Session Initiation Protocol".

[44]	IETF RFC 3428 (December 2002): "Session Initiation Protocol (SIP) Extension for Instant Messaging".

[45]	3GPP TS 23.272: "Circuit Switched (CS) fallback in Evolved Packet System (EPS); Stage 2".

[46]	3GPP TS 29.272: "Evolved Packet System (EPS); Mobility Management Entity (MME) and Serving GPRS Support Node (SGSN) related interfaces based on Diameter protocol".

[47]	3GPP TS 29.118: "Mobility Management Entity (MME) – Visitor Location Register (VLR) SGs interface specification".

[48]	3GPP TS 23.682: "Architecture enhancements to facilitate communications with packet data networks and applications".

[49]	Void.

[50]	3GPP TS 29.338: "Diameter based protocols to support of SMS capable MMEs".

[xx]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

[yy]	3GPP TS 23.502: " Procedures for the 5G System; Stage 2".

# B.3	Next Change

Annex L (normative):
SMS in 5GS

# L.1	General

A 5GS network delivers short message services over 5GS NAS signalling.

For the architecture and the definition of reference points for SMS in 5GS see 3GPP TS 23.501 [xx], subclause 4.2.

Between the architecture and the reference points for SMS in 5GS and the architecture and the reference points shown in subclause 4.1, and figure 5, of the present specification the following mapping is used:

Table L.1: Mapping between architecture and reference points for SMS in 5GS and the present specification

|  | SMS in 5GS | present specification |
| --- | --- | --- |
|  | UDM | HLR |
| Network entities | SMSF | MSC + VLR |
|  | UE | MS |
|  | AMF | MME (NOTE 1) |
|  | N20/Namf/Nsmsf | (not shown in figure 5) |
|  | N21/Nsmsf | (not shown in figure 5) |
|  | N8 | (not shown in figure 5) |
|  | Not shown in 3GPP TS 23.501 | 2 |
| Reference points | Not shown in 3GPP TS 23.501 (NOTE 2) | 3 |
|  | (not applicable) | 4 |
|  | N1 | 5 |
|  | (not applicable) | 6 |
| NOTE 1: In figure 5, the MME is only acting as a relay for SMS transfer via the SGs. NOTE 2: Reference point 3 is used as reference from SMSF to/from SMS-GMSC/IWMSC and SMS Router. |  |  |

For the purpose of supporting SMS in 5GS, the SMSF shall implement the requirements specified in the present specification for the MSC and the VLR with the following modifications:

1)	"GSM/UMTS system" is to be replaced with "5GS".

2)	Throughout the text and in the figures, message names, parameter names and cause values:

"MSC" is to be replaced with "SMSF";

"VLR" is to be replaced with "SMSF internal subscriber register";

"HLR" is to be replaced with "UDM/HLR";

"MS" is to be replaced with "UE"; and

"non GPRS" is to be replaced with "5GS".

3)	Reference point 4 is SMSF internal, and operations performed on this reference point are not standardized. In the message flows of clause 10, the SMSF replaces the combination of MSC and VLR, and messages exchanged between MSC and VLR are replaced with MME internal communication.

Editor's note:	Detailed description of the reference points and their usage for SMS in 5GS needs to be included.

Annex C (informative):
Proposed changes to 3GPP TS 24.011

# C.1	First Change

# 1	Scope

The present document specifies the procedures used across the mobile radio interface by the signalling layer 3 function Short Message Control (SMC) and Short Message Relay function (SM-RL) for circuit switched in A/Gb mode, GPRS, EPS, and 5GS.

## 1.1	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	Void.

[1a]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS) Point-to-Point (PP)".

[3a]	3GPP TS 23.060: "General Packet Radio Service (GPRS); Service description; Stage 2".

[3]	3GPP TS 44.006: "Mobile Station - Base Station System (MS - BSS) interface; Data Link (DL) layer specification".

[4]	3GPP TS 24.007: "Mobile radio interface signalling layer 3; General aspects".

[5]	3GPP TS 24.008: "Mobile radio interface layer 3 specification".

[5a]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[5b]	3GPP TS 33.102: "3G Security; Security Architecture".

[5c]	3GPP TS 42.017: "Subscriber Identity Modules (SIM); Functional characteristics".

[6a]	3GPP TS 44.064: "General Packet Radio Service (GPRS); Logical Link Control (LLC) layer specification ".

[6]	ISO 7498: "Information processing systems - Open Systems Interconnection - Basic Reference Model".

[7]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control Protocol".

[8]	3GPP TS 25.413: "UTRAN Iu interface RANAP signalling".

[9]	3GPP TS 23.401: "General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access"

[10]	3GPP TS 24.301: "Non-Access-Stratum (NAS) protocol for Evolved Packet System (EPS); Stage 3"

[11]	3GPP TS 23.272: "Circuit Switched Fallback in Evolved Packet System; Stage 2"

[12]	3GPP TS 29.118: "Mobility Management Entity (MME) – Visitor Location Register (VLR) SGs interface specification"

[xx]	3GPP TS 23.501: " System Architecture for the 5G System; Stage 2".

[yy]	3GPP TS 23.502: " Procedures for the 5G System; Stage 2".

## 1.2	Abbreviations

For the purpose of the present document, the abbreviations given in 3GPP TR 21.905 [1a] and the following apply:

RR connection: a RR connection is a dedicated physical circuit switched domain connection used by the two RR or RRC peer entities to support the upper layers' exchange of information flows.

PS signalling connection: is a peer to peer UMTS connection between MS and CN packet domain node.

GPRS: Packet Services for GSM and UMTS system.

The label (A/Gb mode only): indicates this section or paragraph applies only to GSM system. For multi system case this is determined by the current serving radio access network.

The label (Iu mode only): indicates this section or paragraph applies only to UMTS system. For multi system case this is determined by the current serving radio access network.

The label (S1 mode only): indicates this section or paragraph applies only to Evolved Packet Core (EPC) and E-UTRAN access. For multi system case this is determined by the current serving radio access network.

In A/Gb mode,...: Indicates this paragraph applies only to GSM System. For multi system case this is determined by the current serving radio access network.

In Iu mode,...: Indicates this paragraph applies only to UMTS System. For multi system case this is determined by the current serving radio access network.

In S1 mode,...: Indicates this paragraph applies only to Evolved Packet Core and E-UTRAN access. For multi system case this is determined by the current serving radio access network.

In N1 mode: Indicates this paragraph applies only to 5G System access.

Editor's note:	Definition of N1 mode requires further study.

SIM: Subscriber Identity Module (see 3GPP TS 42.017 [5c]). This specification makes no distinction between SIM and USIM.

MS: Mobile Station. This specification makes no distinction between MS and UE.

For the purposes of the present document, the following terms and definitions given in 3GPP TS 24.301 [10] apply:

# UE using EPS services with control plane CIoT EPS optimization

# C.2	Next Change

# 2	Overview of Short Message Service (SMS) support

The purpose of the Short Message Service is to provide the means to transfer messages between a GSM PLMN Mobile Station (MS) and a Short Message Entity via a Service Centre, as described in 3GPP TS 23.040 [2]. The terms "MO" - Mobile Originating - and "MT" - Mobile Terminating - are used to indicate the direction in which the short message is sent.

The present document describes the procedures necessary to support the Short Message Service between the MS and the MSC or SGSN or MME or SMSF and vice versa, as described in 3GPP TS 23.040 [2].

The procedures are based on services provided by the Mobility Management sublayer as described in 3GPP TS 24.007 [4]/3GPP TS 24.008 [5] for CS in A/Gb mode and CS/PS services in Iu mode, 3GPP 24.301 [10] for CS/PS services in S1 mode, and the Logical Link Control layer described in 3GPP TS 44.064 [6a] for GPRS services. For CS/PS service in S1 mode, depending on network configuration and UE subscription data, network may choose to use either packet-switched service or circuit-switched service to provide SMS service. If circuit-switched service is used instead of packet-switched service, then the messages are tunneled through the MME. In this case the network also uses procedures described in 3GPP 29.118 [12].

Editor's note:	Mobility management for UE in N1 mode to be included, only possible once the new NAS technical specification is available.

## 2.1	Protocols and protocol architecture

In Iu mode only, integrity protected signalling (see 3GPP TS 24.008 [5], subclause 'Integrity Protection of Signalling Messages' and in general, see 3GPP TS 33.102 [5b]) is mandatory. In Iu mode only, all protocols shall use integrity protected signalling. Integrity protection of all SMS signalling messages is the responsibility of lower layers. It is the network which activates integrity protection. This is done using the security mode control procedure (3GPP TS 25.331 [5a]).

The hierarchical model in figure 2.1a shows the layer structure of the MSC and the MS in A/Gb mode. The hierarchical model in figure 2.1c shows the layer structure of the SGSN and the MS in Iu mode. The hierarchical model in figure 2.1d shows the layer structure of the MSC and the MS in S1 mode. The hierarchical model in figure 2.1e shows the layer structure of the MME and the MS in S1 mode. The hierarchical model in figure 2.1f shows the layer structure of the SMSF and MS in N1 mode.

|  |  |  |  | MSC |  |  |  | MS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-AL |  |  |  |  |  |  |  |  |  |
| SM-TL |  |  |  |  |  |  |  |  |  |
| SM-RL |  |  |  | SMR |  | < SM-RP protocol > |  | SMR |  |
| CM-sublayer |  |  |  | SMC |  | < SM-CP protocol > |  | SMC |  |
| MM-sublayer |  |  |  |  |  |  |  |  |  |
| RR-sublayer |  |  |  |  |  |  |  |  |  |

# Figure 2.1a/3GPP TS 24.011: Protocol hierarchy for circuit-switched service

# The hierarchical model in figure 2.1b shows the layer structure of the SGSN and the MS in A/Gb mode.

|  |  |  |  | SGSN |  |  |  | MS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-AL |  |  |  |  |  |  |  |  |  |
| SM-TL |  |  |  |  |  |  |  |  |  |
| SM-RL |  |  |  | SMR |  | < SM-RP protocol > |  | SMR |  |
| CM-sublayer |  |  |  | SMC |  | < SM-CP protocol > |  | SMC |  |
| LLC-sublayer |  |  |  |  |  |  |  |  |  |
| GRR-sublayer |  |  |  |  |  |  |  |  |  |

# Figure 2.1b/3GPP TS 24.011: Protocol hierarchy for GPRS in A/Gb mode

|  |  |  |  | SGSN |  |  |  | MS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-AL |  |  |  |  |  |  |  |  |  |
| SM-TL |  |  |  |  |  |  |  |  |  |
| SM-RL |  |  |  | SMR |  | < SM-RP protocol > |  | SMR |  |
| CM-sublayer |  |  |  | SMC |  | < SM-CP protocol > |  | SMC |  |
| GMM-sublayer |  |  |  |  |  |  |  |  |  |

# Figure 2.1c/3GPP TS 24.011: Protocol hierarchy for packet-switched service in Iu mode

|  |  |  |  | MSC |  |  |  | MS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-AL |  |  |  |  |  |  |  |  |  |  |  | SM-AL |
| SM-TL |  |  |  |  |  |  |  |  |  |  |  | SM-TL |
| SM-RL |  |  |  | SMR |  | < SM-RP protocol > |  | SMR |  |  |  | SM-RL |
| CM-sublayer |  |  |  | SMC |  | < SM-CP protocol > |  | SMC |  |  |  | CM-sublayer |
| SGs layer |  |  |  |  |  |  |  |  |  |  |  | EMM-sublayer |

NOTE:	SM-CP messages are transferred between the MSC and the MS through the MME. The protocol stack on the MME is not shown.

# Figure 2.1d/3GPP TS 24.011: Protocol hierarchy for circuit-switched service in S1 mode

|  |  |  |  | MME |  |  |  | MS |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-AL |  |  |  |  |  |  |  |  |  |
| SM-TL |  |  |  |  |  |  |  |  |  |
| SM-RL |  |  |  | SMR |  | < SM-RP protocol > |  | SMR |  |
| CM-sublayer |  |  |  | SMC |  | < SM-CP protocol > |  | SMC |  |
| EMM-sublayer |  |  |  |  |  |  |  |  |  |

# Figure 2.1e/3GPP TS 24.011: Protocol hierarchy for packet-switched service in S1 mode

|  |  |  |  | SMSF |  |  |  | MS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SM-AL |  |  |  |  |  |  |  |  |  |  |  | SM-AL |
| SM-TL |  |  |  |  |  |  |  |  |  |  |  | SM-TL |
| SM-RL |  |  |  | SMR |  | < SM-RP protocol > |  | SMR |  |  |  | SM-RL |
| CM-sublayer |  |  |  | SMC |  | < SM-CP protocol > |  | SMC |  |  |  | CM-sublayer |
| N20-layer |  |  |  |  |  |  |  |  |  |  |  | 5GMM-sublayer |

NOTE:	SM-CP messages are transferred between the SMSF and the MS through the AMF. The protocol stack on the AMF is not shown.

# Figure 2.1f/3GPP TS 24.011: Protocol hierarchy in N1 mode

The CM-sublayer, in terms of the Short Message Service Support, provides services to the Short Message Relay Layer.

On the MS-side the Short Message Relay Layer provides services to the Short Message Transfer Layer. The Short Message Relay Layer is the upper layer on the network side (MSC or SGSN or MME), and the SM-user information elements are mapped to TCAP/MAP.

The peer protocol between two SMC entities is denoted SM-CP, and between two SMR entities, SM-RP.

Abbreviations:

SM-AL	Short Message Application Layer

SM-TL	Short Message Transfer Layer

SM-RL	Short Message Relay Layer

SM-RP	Short Message Relay Protocol

SMR	Short Message Relay (entity)

CM-sub	Connection Management sublayer

SM-CP	Short Message Control Protocol

SMC	Short Message Control (entity)

MM-sub	Mobility Management sublayer

GMM-sub	GPRS Mobility Management sublayer

RR-sub	Radio Resource Management sublayer

LLC-sub	Logical Link Control sublayer

GRR-sub	GPRS Radio Resource sublayer in GSM

EMM-sub	EPS Mobility Management sublayer

5GMM-sub	5G Mobility Management sublayer

# C.3	Next Change

## 2.5B	5GSMS entity in N1 mode

It shall be possible for an MS in N1 mode that is attached to the 5GCN to send and receive short messages.

# The 5GSMS entity for MS in N1 mode of operation is shown in figure 2.5B.

# Figure 2.5B/3GPP TS 24.011: 5GSMS entity for MS in N1 mode

# C.4	Next Change

## 3.2	Service provided by the CM-sublayer

In order to support the Short Message Service, the CM-sublayer provides services to the Short Message Relay Layer.

The CM-sublayer services are provided using layer specific functions and lower layer services offered to the CM-sublayer, controlled by short message service control entities called SMCs.

An SMC entity in the MS communicates with an SMC entity in the MSC or the SGSN or the MME or the SMSF by means of a peer protocol, SM-CP (Short Message Service Control Protocol). The arrow diagrams in annex A give an overview of the messaging on the CM-sublayer during a short message transfer.

A mobile station supporting the Short Message Service shall have a minimum of two SMC entities per service type (i.e. two for CS GSM and two for GPRS). This enables the MS to receive MT messages during an MO message transfer.

To ensure that an MS having the minimum of two SMC entities is able to receive MT messages during an MO message transfer, and to send MO messages during MT message transfer, parallel message transfer in the same direction is prohibited. This means that the SMC entities shall not simultaneously perform messaging in the same direction. The rules for concatenation of message transfers are described in subclause 5.4.

The MSC or the SGSN or the MME or the SMSF shall have a minimum of two SMC entities available each during an MT message transfer to a mobile station, one being reserved for MO message transfer. In an MO message transfer, the MSC or the SGSN or the MME or the SMSF shall have one SMC entity reserved for handling of an MT message.

# C.5	Next Change

# 5	CM-procedures

## 5.1	General

This clause describes the procedures used by the SMC entity on the Connection Management sublayer. An SMC entity communicates with a corresponding peer entity using an MM-connection for CS in A/Gb and Iu mode, or the LLC layer for GPRS in A/Gb mode, or the GMM-connection for PS in Iu mode, or the EMM-connection for EPS in S1 mode if packet-switched service is used, or through the MME for S1 mode if circuit-switched service is used, or through the AMF for N1 mode.

Editor's note:	The impact of MO SMS using one step approach in 5GMM-IDLE mode in case of SMS in N1 mode requires further study.

Multiple MM-connections may be established at the same time, allowing parallel transactions. The description of the procedures is related to one single transaction.

For circuit switched service, the CM-procedures described can only be performed if an MM-connection has been established between the mobile station and the network.

For GPRS, no connection has to be established, and thus the CM procedures for GPRS reflect this. Detailed SDL diagrams for SMC entities are contained in annex B.

For EPS when packet-switched service is used, detailed SDL diagrams for SMC entities are contained in annex B.

# C.6	Next Change

### 5.2.2	SMC-GP, SMC-EP and SMC-5G states at the MS side of the radio interface

#### 5.2.2.1	Mobile Originating Case

The states described in this clause are for an SMC-GP entity in a GPRS MS, and for an SMC-EP entity in an EPS MS, and for an SMC-5G entity in a 5GS MS handling mobile originating short message transfer and notification to the network that the mobile has memory available to receive one or more short messages (referred to below as "notification").

# C.7	Next Change

### 5.2.4	SMC-GP, SMC-EP and SMC-5G states at the network side of the radio interface

#### 5.2.4.1	Mobile Originating Case

The states described in this subclause are for an SMC-GP entity in an SGSN and for an SMC-EP entity in an MSC or an MME and an SMC-5G entity in a SMSF handling both mobile originating short message transfer and notification to the network that the mobile has memory available to receive one or more short messages (referred to below as "notification").

# C.8	Next Change

#### 5.3.2.2	RPDU transfer for GPRS, EPS and 5GS

In A/Gb mode, when an SMC-GP or SMC-EP entity or SMC-5G entity is in the Idle state and transfer of an RPDU is requested, the SMC-GP or SMC-EP or SMC-5G entity on the originating side forwards the CP-DATA message containing the RPDU to the lower layer, and sets the timer TC1* and enters the Wait for CP-ACK state. In A/Gb mode, for the SMC-GP entity, the lower layer is the LLC sublayer. For the SMC-EP entity on the MS side, the lower layer is EMM. For the SMC-EP entity on the network side, the lower layer can be either the SGs association as described in 3GPP TS 29.118 [12] or the EMM sublayer. For the SMC-5G entity on the MS side, the lower layer is 5GMM. For the SMC-5G entity on the network side the lower layer is provided by the N20 interface.

Editor’s note:	Appropriate reference for N20 interface to be included once available.

In Iu mode, when an SMC-GP entity in the MS side is in the Idle state and transfer of an RPDU is requested:

-	the SMC-GP entity on the originating side requests the GMM sublayer to establish a PS signalling connection, and enters the GMM-Connection Pending state.

-	after completion of the PS signalling connection establishment, a confirmation is given to the originating side to indicate that the GMM sublayer is ready for RPDU transfer; and.

-	after confirmation of the PS signalling connection establishment, the SMC-GP entity on the originating side forwards the CP-DATA message to the GMM sublayer. This contains the RPDU, and also the SMC-GP entity sets the timer TC1* and enters the Wait for CP-ACK state.

In S1 mode, when an SMC-EP entity in the MS side is in the Idle state and transfer of an RPDU is requested:

-	if the MS is not using Control plane CIoT EPS optimization:

1)	the SMC-EP entity on the originating side requests the EMM sublayer to establish a NAS signalling connection, and enters the EMM-Connection Pending state;

2)	after completion of the NAS signalling connection establishment, a confirmation is given to the originating side to indicate that the EMM sublayer is ready for RPDU transfer; and

3)	after confirmation of the NAS signalling connection establishment, the SMC-EP entity on the originating side forwards the CP-DATA message to the EMM sublayer. This contains the RPDU, and also the SMC-EP entity sets the timer TC1* and enters the Wait for CP-ACK state; or

-	if the MS is using Control plane CIoT EPS optimization, the SMC-EP entity on the originating side forwards the CP-DATA message that contains the RPDU to the EMM sublayer when requesting the EMM sublayer to establish a NAS signalling connection. The SMC-EP entity then sets the timer TC1* and enters the Wait for CP-ACK state immediately.

NOTE:	If the MS in idle mode is using Control plane CIoT optimization, the first CP-DATA message is sent by piggybacking on the CONTROL PLANE SERVICE REQUEST message during the service request procedure as specified in 3GPP 24.301 [10].

In N1 mode, when an SMC-5G entity in the MS side is in the Idle state and transfer of an RPDU is requested:

1)	the SMC-5G entity on the originating side requests the 5GMM sublayer to establish a NAS signalling connection;

2)	after completion of the NAS signalling connection establishment, a confirmation is given to the originating side to indicate that the 5GMM sublayer is ready for RPDU transfer; and

3)	after confirmation of the NAS signalling connection establishment, the SMC-5G entity on the originating side forwards the CP-DATA message to the 5GMM sublayer. This contains the RPDU, and also the SMC-5G entity sets the timer TC1* and enters the Wait for CP-ACK state.

In Iu mode, when an SMC-GP entity in the network side is in Idle state and transfer of an RPDU is requested, the SMC-GP entity on the originating side forwards the CP-DATA message to the GMM sublayer. This contains the RPDU, and also the SMC-GP entity sets the timer TC1* and enters the Wait for CP-ACK state.

In S1 mode and the circuit-switched service is used, when an SMC-EP entity in the network side is in Idle state and transfer of an RPDU is requested, the SMC-EP entity on the MSC forwards the CP-DATA message to the SGs sublayer. This contains the RPDU, and also the SMC-EP entity sets the timer TC1* and enters the Wait for CP-ACK state. The SGs layer transfers the CP-DATA message by using the procedures specified in 3GPP 24.301 [10].

In S1 mode and the packet-switched service is used, when an SMC-EP entity in the network side is in Idle state and transfer of an RPDU is requested, the SMC-EP entity on the originating side forwards the CP-DATA message to the EMM sublayer. This contains the RPDU, and also the SMC-GP entity sets the timer TC1* and enters the Wait for CP-ACK state.

In N1 mode and the packet-switched service is used, when an SMC-5G entity in the network side is in Idle state and transfer of an RPDU is requested, the SMC-EP entity on the SMSF forwards the CP-DATA message to the N20 sublayer. This contains the RPDU, and also the SMC-5G entity sets the timer TC1* and enters the Wait for CP-ACK state.

The value of TC1* may vary with the length of the CP-DATA. However, the value of TC1* shall be sufficiently great to allow the lower layers to transmit the CP-DATA and CP-ACK messages and to allow for some re-transmissions of layer 2 frames.

If an SMC entity in the Wait for CP-ACK state gets an indication that the CP-DATA message has probably been lost then, as an implementation option, that SMC-GP entity may reduce the time until expiry of TC1*.

If the timer TC1* expires in the Wait for CP-ACK state, the CP-DATA message is retransmitted and the state Wait for CP-ACK is re-entered. The maximum number of CP-DATA message re-transmissions is an implementation option but shall be either 1, 2 or 3. If the timer TC1* expires after the maximum number of retransmission attempts, an error indication is passed to SM-RL. The Idle state is then entered.

On receipt of the CP-ACK message in response to the CP-DATA (RP DATA) message in the Wait for CP-ACK state, the SMC-GP resets the timer TC1* and enters the Wait for CP DATA state.

On receipt of the CP-ACK message in response to the CP-DATA (RP ACK) message in the Wait for CP-ACK state, the SMC-GP resets the timer TC1* and enters the Idle State.

On receipt of the CP-ACK message in response to the CP-DATA (RP DATA) message in the Wait for CP-ACK state, the SMC-5G resets the timer TC1* and enters the Wait for CP DATA state.

On receipt of the CP-ACK message in response to the CP-DATA (RP ACK) message in the Wait for CP-ACK state, the SMC-5G resets the timer TC1* and enters the Idle State.

In A/Gb mode or S1 mode or N1 mode, when receiving a CP-DATA message from the lower layer, the SMC-GP or SMC-EP or SMC5G entity checks the parameters relevant to the CP protocol. If these are valid, the RPDU is passed to the SM-RL, the CP-ACK message is sent.

In Iu mode, when receiving a CP-DATA message from the GMM sublayer, the SMC-GP entity checks the parameters relevant to the CP protocol. If these are valid, the RPDU is passed to the SM-RL, the CP-ACK message is sent.

In S1 mode, when receiving a CP-DATA message from the lower layer, the SMC-EP entity checks the parameters relevant to the CP protocol. If these are valid, the RPDU is passed to the SM-RL, the CP-ACK message is sent.

In N1 mode, when receiving a CP-DATA message from the lower layer, the SMC-5G entity checks the parameters relevant to the CP protocol. If these are valid, the RPDU is passed to the SM-RL, the CP-ACK message is sent

If an SMC entity in the Idle state is unable to accept a CP-DATA message, it sends a CP-ERROR message and then enters the Idle state.

Annex D (informative):
Proposed changes to 3GPP TS 24.301

# D.1	Summary of changes

The list of impacts that should be captured in 3GPP TS 24.301 [12] is as follows:

1)	Attach procedure for EPS services

A)	Mode selection for intersystem change

i)	If the UE supports N1 mode, the UE shall set the N1 mode supported bit to "N1 mode supported" in the UE network capability IE of the ATTCH REQUEST message.

ii)	If the MME received the N1 mode supported bit set to "N1 mode supported" in the UE network capability IE of the ATTACH REQUEST message and the MME supports intersystem change with 5GS, in the ATTACH ACCEPT message the MME shall indicate either:

a)	"dual-registration not supported" if the MME supports the N26 interface; or

b)	"dual-registraion supported" if the MME does not support the N26 interface.

iii)	If the MME indicated "dual-registration not supported", the UE shall select single-registration mode. Otherwise if the MME indicated "dual-registration supporte", the UE shall select dual-registration mode if the UE supports dual-registration mode.

Editor's note:	Details on the case where the network provides the dual-registration supported IE set to "dual-registration supported" and the UE does not support dual-registration mode, are FFS.

B)	Temporary identity

Editor's note:	Details on the temporary identity to be included in the ATTACH REQUEST message are FFS.

2)	Normal tracking area updating procedure

Editor's note:	Details on the temporary identity to be included in the TRACKING AREA UPDATE REQUEST message are FFS.

Editor's note:	The use of TIN is FFS.

3)	UE requested PDN connectivity procedure

A)	The PDU CONNECTIVITY REQUEST message(s) triggered by the UE operating in the dual-registration mode intending to transfer PDU sessions in the 5GS to the EPS, shall include the request type IE set to "Handover" (if the semantics of this value can be extended) or "handover from 5GS".

Editor’s note:	The list can be updated through the CT1 meetings.

Annex E:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2017-04 | CT1#103 | C1-171624 |  |  |  | Draft skeleton provided by the rapporteur. | 0.0.0 |
| 2017-04 | CT1#103 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-171512, C1-171516, C1-171623, C1-171632, C1-171634, C1-171638, C1-171959, C1-171960, C1-171662, C1-171971, C1-171973, C1-171974, C1-171975. | 0.1.0 |
| 2017-05 | CT1#104 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-172298, C1-172410, C1-172411, C1-172412, C1-172628, C1-172643, C1-172416, C1-172427, C1-172429, C1-172430, C1-172431, C1-172432, C1-172630, C1-172632, C1-172654, C1-172655, C1-172769, C1-172770, C1-172771, C1-172772, C1-172773, C1-172780, C1-172786, C1-172633, C1-172634, C1-172635, C1-172636, C1-172637, C1-172645, C1-172646, C1-172647, C1-172656, C1-172775, C1-172472, C1-172777, C1-172640, C1-172659, C1-172661, C1-172662, C1-172776, C1-172778, C1-172779 | 0.2.0 |
| 2017-06 | CT1 e-mail review |  |  |  |  | Implementation of C1-172773 under clause 6 instead of clause 10. Also, the TR clauses from clause 11 onwards have been renumbered due to the move of the previous clause 10. | 0.2.1 |
| 2017-08 | CT1#105 |  |  |  |  | Implementing the following p-CRs agreed by CT1: C1-173180, C1-173076, C1-173181, C1-173189, C1-173183, C1-173352, C1-173799, C1-173804, C1-173195, C1-173198, C1-173358, C1-173359, C1-173360, C1-173361, C1-173362, C1-173363, C1-173364, C1-173550, C1-173555, C1-173556, C1-173358, C1-173714, C1-173715, C1-173716, C1-173717, C1-173728, C1-173729, C1-173730, C1-173762, C1-173763, C1-173764, C1-173800, C1-173801, C1-173803, C1-173809, C1-173367, C1-173511, C1-173718, C1-173819, C1-173740, C1-173742, C1-173766, C1-173771, C1-173787, C1-173802, C1-173798, C1-173803, C1-173805, C1-173581, C1-173743, C1-173745, C1-173788, C1-173789, C1-173790, C1-173791, C1-173795, C1-173397, C1-173500, C1-173501, C1-173191, C1-173344, C1-173388, C1-173394, C1-173502, C1-173508, C1-173509, C1-173578, C1-173579, C1-173595, C1-173722, C1-173767, C1-173768, C1-173792, C1-173793, C1-173794, C1-173185, C1-173011, C1-173559, C1-173723, C1-173769, C1-173562 | 0.3.0 |
| 2017-09 | CT1 e-mail review |  |  |  |  | Implementation of part of C1-173364 as some changes were missing. Editorial corrections. | 0.3.1 |
| 2017-09 | CT1 e-mail review |  |  |  |  | Editorial corrections. | 0.3.2 |
| 2017-09 |  | CP-173035 |  |  |  | Version 1.0.0 created for presentation to TSG CT#77 for information | 1.0.0 |
| 2017-09 | CT1 e-mail review |  |  |  |  | Implementation of missing parts of C1-172803, C1-173508, C1-173719, C1-173766 | 1.0.1 |