3GPP TR 26.929 V0.5.0 (2018-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

QoE parameters and metrics relevant to the Virtual Reality user experience (Release 16	)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

virtual reality, QoE, user experience

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	5

1	Scope	6

2	References	6

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	7

3.3	Abbreviations	7

4	VR QoE overview	7

4.1	Introduction	7

4.2	VR QoE metrics evolution	7

4.3	Viewport-related QoE aspects	8

4.3.1	Introduction	8

4.3.2	Single stream region-independent coding	8

4.3.3	Single stream region-dependent encoding	9

4.3.4	Multiple stream region-dependent encoding	9

4.3.5	Region-based encoding, simple head movement	10

4.3.6	Region-based encoding, complex head movements	10

4.3.7	Summary	11

4.4	Viewport-related information flow	11

5	Use case for VR QoE	14

5.1	Introduction	14

5.2	3DoF VR Streaming	14

6	VR QoE reference model under consideration	14

6.1	General description	14

6.2	Observation point 1	15

6.3	Observation point 2	15

6.4	Observation point 3	15

6.5	Observation point 4	16

6.6	Observation point 5	16

7	VR content impact on QoE	16

7.1	Introduction	16

8	Transmission impact on VR QoE	16

8.1	Introduction	16

8.2	QoE metrics relevant with network transmission	16

8.2.1	Average Throughput	16

8.2.2	Buffer Level	17

8.2.3	Play List	17

8.2.4	Presentation Delay	17

8.2.4.1	Introduction	17

8.2.4.2	Ignore viewport and encoding quality	17

8.2.4.3	Consider viewport but not encoding quality	17

8.2.4.4	Consider viewport and encoding quality	18

8.2.4.5	More advanced delay measurements	18

8.2.4.6	Aggregation	18

8.2.4.7	Metric definition	18

9	VR device impact on QoE	19

9.1	Introduction	19

9.2	 QoE metrics relevant with VR device	19

9.2.1	Field of View	19

9.2.2.	Resolution	19

9.2.3.	Refresh Rate	19

9.2.4.	Decoder capability	19

9.2.5	Detailed QoE metrics	20

10	VR QoE configuration and reporting	20

10.1	Introduction	20

10.2	VR metrics	20

11	Conclusions	20

Annex A (informative): Change history	21

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

User experience based network management is important for operators so that they can provide best quality services. In case of a VR service, which is much more complex than traditional video streaming in terms of content creation, network transmission and device requirements, the provision of a satisfying immersive experience is more challenging. The first step of this effort would be to find out what factors have an impact on user experience, and define reference metrics and parameters that would help operators make assessment of user experience, do trouble shooting and design target solutions.

The analysis of impact on user experience involves the whole E2E VR service chain:

- Creation of content. VR content creation would involve multiple steps such as capture, stitching, projection, and encoding, each step would have an impact on the content itself. It is necessary to look into each step and find out what factors are relevant to the VR experience;

- Network transmission. The amount of video data of VR content entails a high streaming bitrate, and may lead to a risk of network and/or access link congestion and re-buffering, and thus an impediment to limiting latency. Latency is one of the key elements to create the feeling of immersiveness, and thus has considerable impact on user experience.

- Device requirement. Compared with a traditional device, be it a mobile phone or a tablet, a VR device exhibits many more attributes, designed to help create immersive experience. The degree of freedom it provides to users, the sensitivity of sensors that enable quick catching of head movement, and many other attributes, all need to be studied and evaluated about their relevance to user experience.

This report investigates the QoE metrics relevant with VR experience from the aforementioned three aspects, and also the way of reporting these QoE metrics to the network for further analysis.

# 1	Scope

This Technical Report provides a study on the QoE metrics relevant to VR service. The study focuses on:

- Defining a device reference model for VR QoE measurement points.

- Studying key performance indicators that may impact the experience of VR service.

- Identifying the existing QoE parameters and metrics defined in SA4 standards such as TS 26.247, TS 26.114 which are relevant to Virtual Reality user experience;

- Identifying and defining new QoE parameters and metrics relevant to Virtual Reality user experience, taking into consideration the use cases listed in TR 26.918, and any sources that show the relevance of new metrics, e.g. scientific literature, specifications/solutions from other standard organizations.

- Analysing potential improvements to the existing QoE reporting so as to better accommodate VR services.

- Providing recommendations to future standards work in SA4 on the QoE parameters and metrics and, as necessary, coordinate with other 3GPP groups and external SDOs, e.g. MPEG, ITU-T.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2] 	3GPP TR 26.918: "Virtual Reality (VR) media services over 3GPP".

[3] 	3GPP TS 26.247: "Transparent end-to-end Packet-switched Streaming Service (PSS); Progressive Download and Dynamic Adaptive Streaming over HTTP (3GP-DASH)".

[4]	ISO/IEC 23009-1: 2014/Amd. 1:2015/Cor.1:2015" Information technology -- Dynamic adaptive streaming over HTTP (DASH) -- Part 1: Media presentation description and segment formats".

[5]	ITU-T P.1203: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

QoE				Quality of Experience

VR					Virtual Reality

UL	Up-link

# 4	VR QoE overview

## 4.1	Introduction

## 4.2	VR QoE metrics evolution

The goal with this study is to suggest improvements to the existing QoE reporting, so that suitable QoE metrics are available to better understand the VR service quality as experienced by the VR users. A complicating factor for the study is the lack of a thorough scientific understanding on exact how different VR conditions and impairments relate to the final user quality.

As VR services becomes more mature, and also more standardized, this understanding will become more clear over time. It is not unlikely that in a long-term perspective there might be standardized objective quality models, similar to the ITU-T P.1203 [5], which translate measurable QoE metrics into the final user experience.

However, there is a significant delay from the time a standard (3GPP, MPEG, ITU-T etc.) is ready, until the corresponding standard features are actually implemented large-scale by the device industry. There is also a further delay until the penetration of such standard-compliant devices gets high enough to become useful for wider analysis.

Thus, waiting with defining VR-related QoE metrics until a full and complete understanding has been gained will cause a significant gap in time, where VR service-quality monitoring cannot be adequately performed, at least not by standardized means.

The question is how to move forward during this transition period, where a detailed understanding of the QoE relations is not always available? And how to do it in a way where stepwise QoE metrics refinement can be done over several releases?

The key for how to succeed with this can actually be found in the existing QoE metrics in TS 26.247. When these metrics were standardized in Rel-10, several of the defined "metrics" are actually not really stand-alone QoE metrics. Rather they are just lists of events initiated by the client or by the user.

A typical example is the PlayList, which contains user and client actions, together with timestamps and a minimal set of related metadata. As we have discussed in earlier SA4 meetings, these "event lists" are not QoE metrics by themselves, and there has also been suggestions to enhance 26.247 with additional "real QoE metrics".

However, a big advantage with the event lists is that they contain the basic information necessary to calculate other derived QoE metrics. For instance, ITU-T P.1203 needs as one input a metric containing the number of rebufferings. As P.1203 was not fully standardized until 2017, there was no knowledge during the Rel-10 QoE work that such a metric would later be needed. However, due to the event-based PlayList it is now actually possible to derive this metric, without changing the 26.247 standard. The same is true for several other metrics needed by P.1203.

A drawback with event lists, especially for the VR case, is that they can potentially contain a lot of events. For normal 2D streaming the interaction from the user is much more limited, basically play, stop, jump, etc. For a VR service all of those are also there, but the main interaction is continuous head (or even body) movements, which then might also cause the player to interact towards the network in different ways (fetching different tiles etc.).

However, there are several ways to handle the report size problem, for instance using a constant (but configurable) sample time for the "movement list", and/or a (configurable) movement threshold before a movement is logged. Although it might be difficult to define exactly what sample time or movement threshold that would be the best compromise between accuracy and report size, we actually don't have to know that right now. It is a decision which can be made (years) later by the operator or the service provider, depending on the QoE use case at that point in time.

When VR services become more mature, it is expected that more optimized versions of the QoE metrics can possibly be standardized, resulting in more tailored and smaller metrics. But having a basic QoE reporting available as early as possible is a big advantage, as it is typically during the early phases of a new service adoption that the need for quality-related feedback and analysis is most important.

Thus a stepwise evolution of VR QoE metric could be to use event lists whenever possible for the initially defined metrics, as this seems to be the most future-proof representation. The lists could allow some basic configurability to enable a flexibility between accuracy vs. report size.This would allow later derivation of more specific QoE metrics, as well as other quality-related aspects, some of which might not even be known today.

## 4.3	Viewport-related QoE aspects

### 4.3.1	Introduction

From the user point of view, one of the main differences between normal 2D video streaming and VR video streaming is the notion of a viewport. Instead of always seeing the complete video, the user only sees a cropped part of the video, the viewport, depending on the direction of the device.

The resulting impairments and quality experience will vary depending on content authoring strategy, the client rendering strategy, and any network impact. The following clauses show some (non-exhaustive and simplified) examples of possible delivery scenarios and resulting impairment types. Note that while the viewport also affects audio, no audio aspects are included in the examples.

The examples should not be seen as any endorsement of specific authoring or delivery technology, they only illustrate some possible impairments as seen from the user point of view. Also, for the sake of simplicity, all examples are drawn with square regions, but in practice regions can have different shapes and also do not need to have clear quality boundaries. Any spatial distortion due to the mapping into spherical rendering is also not considered.

### 4.3.2	Single stream region-independent coding

In this scenario the content is encoded with the same resolution and quality settings over the complete 360 content, and delivered as a single stream. The client decodes the complete stream but shows only the cropped part corresponding to the viewport (the red rectangle).

In the example above the user moves the viewport from left to right, but as the encoding is the same for any viewport, this scenario is very similar to the 2D case. The main additional impairments would likely be related to projection artifacts and any device-internal rendering delay.

### 4.3.3	Single stream region-dependent encoding

The content is encoded with emphasis on one or more regions, where the content producer believes that most users will direct their viewport. Thus the resolution and/or coding quality is higher for selected parts of the spatial area, corresponding to these regions. The video is still delivered as one single 360 stream, and the client decodes and shows a cropped part corresponding to the viewport.

In the example above, two emphasized regions are defined (dark grey), and the user moves the viewport from left to right. For all three viewports the average viewport quality is the same (i.e. 50% high quality and 50% low quality), but it is likely that the quality in the central part of the viewport has largest importance for the user. Thus you would expect viewport #1 to be experienced as best, and viewport #3 as worst.

### 4.3.4	Multiple stream region-dependent encoding

Multiple streams can be used, each emphasizing a given region. The receiver selects to download and render the stream which emphasized region best corresponds to the actual viewport.

In the example above, stream #1 and stream #2 have 25% overlapping region-optimized encoding, and when reaching viewport #3 the receiver decides to switch to stream #2. Thus while turning the head between viewport #2 and #3, the user will see an instant change of quality for the left part (going high to low) and right part (going low to high) of the viewport.

Note that the average viewport quality is the same for viewport #2 and #3 (i.e. ~67% high quality and ~33% low quality) but the dynamic effect of switching between them is probably clearly visible. If the user moves his head back and forth between viewport #2 and #3, and the receiver selects to switch streams, such quality changes can likely be rather annoying.

### 4.3.5	Region-based encoding, simple head movement

With region-based encoding the client typically fetches viewport regions with high quality, while using low-quality regions for the backround around the viewport (or even for the complete 360). The figures below (left and right) illustrate two variants of a simple head movement.

On the left side, the head movement is aligned with the regions, while on the right side it is moved a small bit into the next region column, and also a bit downward. In both cases the main impairment is the visible delay before high-quality versions of the new viewport regoins have been fetched and rendered.

In the example it likely takes a bit longer to update the right scenario (nine regions instead of two), but due to the minimal viewport coverage of the seven outer regions, the user is unlikely to note any quality difference between the left and right scenario after the update of the first two regions. Thus although the final update delay probably is different, the experienced quality might be the same.

Note that the client could in principle instead decide to skip updating the seven outer regions due to their minimal viewport coverage. Alternatively, the client could decide to update them, but use an intermediate quality level instead of the highest quality, resulting in the example below.

### 4.3.6	Region-based encoding, complex head movements

In practice, a user might move his head in more complex patterns, with continous movements of varying speed. During these movements the region update times will likely vary depending on network conditions etc. The example below shows a possible scenario, where the update most of the time lags somewhat compared to the current viewport at any given time.

### 4.3.7	Summary

The shown examples form only a small subset of many possible scenarios, but they still illustrate that the viewport-related interaction between the user, the client, and the server is complex, and the final impact on the end-user quality will depend on many factors.

This interaction and it's principal effect on the perceived quality needs to be understood. The interaction also needs to be mapped into a reference architecture, identifying what information (for instance regarding the viewport) that might be available at different sub-parts of the system.

Note that QoE metrics as such need not map perfectly to the perceived end-user quality, as this is a much wider task usually handled by ITU-T with extensive subjective tests, and advanced quality models. However, QoE metrics should be designed in such a way that they will be able to characterize typical impairments in a consistent, discriminative and meaningful way.

## 4.4	Viewport-related information flow

To better understand what metrics are possible to measure, we need to understand the flow of information inside the VR streaming application. A reference model for VR QoE measurements is described later in clause 6, defining a number of observation points. However, in the current clause we will not specifically focus on observation points, but more on the flow of information, and we will instead use the following picture (from TS 26.118, might be revised) to illustrate a possible information flow for a simple use-case:

Figure 4.4.1: Client Reference Architecture for VR DASH Streaming Applications

The DASH data model in Figure 4.4.2 below is also used as a basis for the use-case:

Figure 4.4.2: DASH Data Model

As seen in Figure 4.4.2, the adaptation sets are the central part for any viewport-related handling of the DASH media. Each adaptation set covers a certain spatial area of the sphere (signalled in CC), and it may also contain additional information on the relative quality ranking (QR) between the sets. Thus adaptation sets are selected depending on the user pose, and within each adaptation set there might be several representations with different encoding bitrates.

Note that depending on the scenario (e.g. single stream region-independent, single stream region-dependent, multiple stream region-dependent) multiple adaptation sets may be used at the same time, for instance a low-quality low-bitrate full-coverage set used for the full 360 view, and one or more high-quality sets which mainly covers the intended viewport. In tiled distributions each adaptation set typically contain only one tile, at a certain resolution.

Figure 4.4.3 below shows a simplified signalling diagram for a use-case with different changes of user pose:

Figure 4.4.3: Simplified signalling use-case

Although very simplified, the use-case illustrates the possible division of responsibility between the VR application and the DASH access engine. The VR application uses the sensor information and the MPD coverage and quality metadata to continuously decide which adaptation sets that shall be used. The task of the DASH access engine is to continuously fetch the media segments for the selected adaptation sets, while possibly adapting between different representations depending on the bitrate conditions in the network

In this use-case the DASH access engine does not have any knowledge about the pose or viewport orientation, or other viewport aspects such as field-of-view. It only tries to fetch suitable segments for the adaptation sets specified by the VR application, and deliver these for decoding and rendering.

Thus any metrics related to pose or viewport handling must either use specific non-viewport-related events known at DASH level (such as change of requested adaptation sets and the later delivery of the related media frames), or use combined trigger events derived from the VR application and the Decoder/Renderer (which might be difficult).

# 5	Use case for VR QoE

## 5.1	Introduction

There are 12 use cases defined in TR26.918 [2], classified as UE consumption of managed/3rd party VR content and VR services including UE-generated content.

## 5.2	3DoF VR Streaming

Although all the 12 use cases are possible scenarios of VR service, the current standard work majorly focuses on 3DoF VR streaming, e.g. in MPEG.

According to [2], the use case of VR streaming is defined as:

“A user watches a VR on-demand video content with a HMD. The content is streamed over the 3GPP network using unicast. The user can navigate within the 360 degree video by moving his head and watch different fields of view within the video”

For this use case,

- Content part: study needs to be conducted on which metadata would help analyze user experience.

- Delivery part: changing network conditions may lead to problems in user experience, especially the impact of transmission latency on user experience, e.g. the initial loading latency, the transmission latency contributing to the latency between head/eye movement and presentation of high-resolution content to the user.

- Device part: device capabilities will also have impact on user experience, e.g. the decoder capability, the sensor detect latency in case of head movement.

QoE metrics relevant with the above aspects need to be studied under this study item, and based on the result of this study, user experience of 3DoF VR streaming could be evaluated, and relevant provisioning and streaming techniques toward improving user experience could be designed.

# 6	VR QoE reference model under consideration

## 6.1	General description

A reference model for VR QoE measurement is illustrated in Figure 1.

Figure 1 Reference model for VR QoE measurement

## 6.2	Observation point 1

The network access element issues network request for VR content depending on the data received from Sensor and projection/orientation metadata carried in network manifest file receives network response containing VR content and decapsulates VR media stream from the network. The interface from the network access element towards metric collection and computation (MCC) is referred to as observation point 1 (OP1).

The OP1 is defined to monitor the following information:

- a sequence of transmitted network requests, each defined by its transmission time, contents, and the TCP connection on which it is sent; and

- for each network response, th reception time and contents of the response header and the reception time of each byte of the response body.

- The projection/orientation metadata carried in network manifest file.

## 6.3	Observation point 2

The media processing element carries out demux and audio, image, video. The interface from the media processing element towards MCC is referred to as observation point 2 (OP2).

The OP2 consists of encoded media samples, and OP2 is defined to monitor:

- media type, e.g. media resolution, media codec, media frame rate, media projection, etc

- media decoding time

## 6.4	Observation point 3

The sensor element acquires user’s head or body position, orientation and motion, the sensor may also acquire environmental data such as light, temperature, magnetic fields, gravity and biometrics etc. The interface from the sensor towards MCC is referred to as observation point 3 (OP3).

The OP3 is defined to monitor the information such as:

- Motion tracking information, e.g. 3DoF (Pitch, Yaw and Roll), 6DoF (X, Y, Z, Pitch, Yaw and Roll)

- Timestamp when user movement is captured

- Depth

## 6.5	Observation point 4

The rendering element carries out colour conversion, projection, media composition and view synthesis for each VR media element.

The media presentation element synchronizes and presents mixed nature and sythentic VR media elements to provide a full immersive VR experience to the user. The interface from the media presentation towards MCC is referred to as observation point 4 (OP4).

The OP4 is defined to monitor the information such as:

- The media type

- The media sample presentation timestamp

- Wall Clock counter

- Actual presentation viewport

- Actual presentation time

- Actual playout frame rate

- Audio-to-video synchronization

- Video-to-motion latency

- Audio-to-motion latency

## 6.6	Observation point 5

The VR client control and management element manages client parameters such as display resolution, frame rate, field of view (FOV), eye to screen distance, lens separation distance, etc. The interface from the VR client control & management towards MCC is referred to as observation point 5 (OP5).

The OP5 is defined to monitor the information such as:

- Display characteristics, e.g. display resolution, display PPI, etc.

- OS support, e.g. OS type, OS version

MCC may acquire data from multiple OPs and derive or compute specific VR metrics such as latency or viewport package loss

# 7	VR content impact on QoE

## 7.1	Introduction

# 8	Transmission impact on VR QoE

## 8.1	Introduction

There are many similarities between VR and traditional streaming in terms of transmission impact on user experience. And some of the QoE metrics defined in TS26.247 could be directly applied for VR service. And of course new QoE metrics are also possible considering specific properties of VR service. All the transmission impact information could be collected by OP1 and OP2 of the reference model described in Section 6.1.

## 8.2	QoE metrics relevant with network transmission

### 8.2.1	Average Throughput

Section 10.2.4 in [3] defines the metric for average throughput information. This information could be observed by OP1 of the reference model.

### 8.2.2	Buffer Level

Annex D.4.5 in ISO/IEC 23009-1 [4] defines the metrics for buffer level status events. This information could be observed by OP1 of the reference model.

### 8.2.3	Play List

Section 10.2.7 in [3] defines the metrics for event that may happen due to user action, the end of the content or a permanent failure. This information could be observed by OP1, OP2, OP3 and OP4 of the reference model.

### 8.2.4	Presentation Delay

#### 8.2.4.1	Introduction

A possible presentation delay metric is measuring the delay between the wanted presentation time for a DASH segment, and the actual (maybe delayed) presentation time. Note that this is not exactly the same as "motion to photon delay" or "motion to high-quality content", as segments might arrive late not only due to user movement and interaction, but also due to other reasons.

On the other hand, from a user perspective, both are equally bad and give rise to the same type of artifacts and quality degradations, so it makes sense to just measure the presentation delay, and disregard the actual reason (e.g. head movement) which caused the delay. However, for most practical VR cases the main reasons for late segment arrival is likely to be fast head movement.

During playout the playhead position continuously moves forward unless the playout stops due to rebuffering. Assuming that any such rebuffering can be handled by the rebuffering metric (i.e. derived from the existing PlayList metric), the presentation delay metric only needs to deal with the possibly late arrival of video data during ongoing playout.

Thus the presentation delay for any DASH segment received is in principle the difference between the intended segment start time and the current playhead position. If the segment start time is later then the playhead position, the data is available on time (at least if we disregard any further device-internal processing delay). If the segment start time is earlier than the playhead position, the segment is late.

The question is how this delay can be practically measured, and how it relates to what the user really experiences. There are several technical possibilities, with varying degree of accuracy and implementation complexity, and the following (non-exhaustive) clauses outline some possible variants.

#### 8.2.4.2	Ignore viewport and encoding quality

A relatively low-complex implementation is to ignore the actual viewport seen by the user, and assume that the VR client does only request segments which are relevant. For instance, for multiple-stream region-dependent encodings, it is assumed that the client changes to a more appropriate track when the user moves his head, and that all data requested and later received is actually used and relevant. In the same way, for region-based encodings, the regions (e.g. the tiles) requested and received are assumed to be relevant.

Thus for every DASH segment received, a delay is calculated based on the current playhead position and the defined start time for the received segment. If the segment request was done after the defined start time, the time of the segment request is used instead of the defined start time (to handle the case when only a part of a segment is needed). If the delay is negative (i.e. the segment arrived on time), the segment delay is set to zero.

Note that all segments are treated equally even if some segments are likely much more important, so this implementation is only a crude estimation of the impact on the user experience.

Input needed: Segment start time, playhead position.

#### 8.2.4.3	Consider viewport but not encoding quality

A more complex implementation is to only consider delays which are probably visible to the user. For instance, when a DASH segment is received, the segment delay is calculated as in the previous clause, but the content coverage of the segment is also derived (e.g. via the CC metadata from the MPD).

The current viewport coverage is also derived (e.g. from the sensor and device data), and the overlap between the segment and the viewport is calculated. The segment delay is only considered if there is any overlap (alternatively, the delay is weighted by the percentage of the viewport covered by the segment).

Note that as the encoding quality of the content is not considered, a low-resolution background segment (which likely has large coverage) might be weighted higher than a high-resolution segment. Also, as the overlap is calculated when the segment is received, the user might have moved the viewport when the data in the segment is later rendered.

Input needed: Segment start time, playhead position, content coverage, viewport coverage, MPD.

#### 8.2.4.4	Consider viewport and encoding quality

An even more complex implementation is to also consider the encoding quality. Encoding quality can be based on the relative QR quality ranking from the MPD, or it can be approximated as BPSPA (bits per second per area, i.e. segment size divided by segment length divided by the angular area coverage).

For instance, the current "steady state" quality level can be estimated by using the maximum quality value (QR or BPSPA) seen within a certain measurement window. In addition to the content and viewport overlap calculated as in the previous clause, the segment delay is only considered if the segment quality is the same (or close to) the steady state quality level. Thus low-quality background segments will not contribute to the delay measurements.

Note that also in this implementation the evaluation is done when a segment is received, so any head movements between evaluation and actual rendering is not accounted for.

Input needed: Segment start time, playhead position, content coverage, viewport coverage, MPD.

#### 8.2.4.5	More advanced delay measurements

If more accurate delay measurements are needed the evaluation must likely be done close to rendering or viewport generation. However, assuming that the delay measurements will mostly be used to approximate the end user experience, it's not obvious that such advanced implementations are worthwhile. More detailed understanding is needed before proposing more advanced implementations.

#### 8.2.4.6	Aggregation

The previous clauses only describe how individual delay measurements could be done, but to be practically useful these need to be aggregated into meaningful metrics (and reporting all individual segment delays is also bandwidth-demanding, especially for region-based (e.g. tiled) encodings).

Aggregation can in principle be done over a complete QoE reporting period, but it is probably better to enable aggregation over shorter intervals to increase the time resolution of the metrics.

Assuming that most VR services will have relatively decent quality (as otherwise they would not be used), many of the individual segment delay measurements should be zero or at least pretty small. Thus using simple linear averaging is probably not very useful, as this might hide any delay peaks.

Histogram binning could be used, as this will clearly show the distribution of the delays. To keep the amount of bins low bin sizes can be varied, e.g. by using logarithmic bins. Another alternative is to use percentiles to catch the best and the worst part of the delay distribution.

Yet another possibility would be to report on detailed segment level, but to only reports segments which:

	- arrive later than a certain delay threshold (e.g  >0 ms, report only late segments)
	- cover more than a certain percentage of the viewport (e.g. >0%, report only visible segments)
	- have a BPSPA value higher than a certain percentage of the steady-state BPSPA value (e.g. 75%)

Assuming that most segments do arrive on time, the resulting report will normally be reasonably small. This approach is exemplified in the next subclause.

#### 8.2.4.7	Metric definition

The table below describes the metric for presentation delay. It is assumed that as part of the QoE configuration the three thresholds for delay, coverage and quality have been set. If not set, all segments will be reported.

Only the timestamps, the playheadPosition and the presentationDelay fields are mandatory, while the viewportCoverage and the relativeQuality fields are only reported if supported by the client.

Table 1 Presentation delay metric

| Key |  |  | Type | Description |
| --- | --- | --- | --- | --- |
| SegmentList |  |  | List | A list of received segments fulfilling the delay, coverage and quality thresholds |
|  | Entry |  | Object | An object containing information for one segment |
|  |  | timestamp | Real-Time | Reception time for the segment |
|  |  | playheadPosition | Media-Time | Playhead position when receiving the segment |
|  |  | presentationDelay | Integer | Delay compared to intended segment presentation time, in milliseconds |
|  |  | viewportCoverage | Integer | Percentage of viewport covered by the segment (optional) |
|  |  | relativeQuality | Integer | Segment BPSPA percentage relative to the steady-state BPSPA value (optional) |

# 9	VR device impact on QoE

## 9.1	Introduction

Compared with traditional streaming video, the key feature of VR service is to create immersive experience and enable smooth interactivity between user and the environment, in which VR device would play an important role. This contribution proposes device information relevant to user experience of VR service. All the device property information could be collected by OP5 of the reference model described in Section 6.1.

## 9.2		QoE metrics relevant with VR device

### 9.2.1	Field of View

One of the factors that contribute to the uniqueness of 360 video experience is the level of immersion induced by the wider FoV of HMD, which represents the extent of observable environment at any given time. A wider FoV could help provide a more authentic feeling of immersion. Thus FoV of the HMD is an important parameter that helps evaluate to what extent a VR device could help create immersive experience.

### 9.2.2.	Resolution

Resolution here is defined as for per eye. An appropriate screen resolution would provide the best and comfortable experience.

### 9.2.3.	Refresh Rate

Refresh rate is the number of times per second the display grabs a new image from the graphic processing unit. Lower refresh rate would contribute to processing latency and lead to VR sickness, i.e. viewing glitches on the screen. While higher refresh rate adds to the sense of presence in virtual worlds.

### 9.2.4.	Decoder capability

The support of codec profile and level is an important property that decides the content types it could decode.

### 9.2.5	Detailed QoE metrics

The QoE metrics relevant with VR device as listed in Table 1 is necessary for assessment of device impact on user experience, and suggested to be collected by VR client. And since these metrics are generally static for VR device, the metric is only logged at the start of each QoE reporting period.

Table 1 QoE metrics relevant with VR device

| Key |  |  |  | Type | Description |
| --- | --- | --- | --- | --- | --- |
| DeviceInformationList |  |  |  | List | A list of device information objects. |
|  | Entry |  |  | Object | A single object containing new device information. |
|  |  | resolution |  | Object | Display resolution for each eye |
|  |  |  | videowidth | Integer | Number of pixels in display width |
|  |  |  | videoheight | Integer | Number of pixels in display height |
|  |  | refreshRate |  | Integer | The number of times in a second that a display hardware updates its buffer |
|  |  | decoderCapability |  | Set | Codec profile and level the device supports |
|  |  | fieldofview |  | Object | Information of end device FoV capability. |
|  |  |  | horizontalFoV | Integer | Horizontal FoV of the device in degrees. |
|  |  |  | verticalFoV | Integer | Vertical FoV of the device in degrees. |

# 10	VR QoE configuration and reporting

## 10.1	Introduction

For the existing (non VR) QoE functionality, configuration is done on DASH level according to TS 26.247. There are three different methods for configuration:

- QoE configuration is placed in the MPD
	- QoE configuration is done via OMA-DM
	- QoE configuration is done via RRC signalling

All three cases re-use similar configuration objects, basically specifying which metrics to report, and how the reporting is done (at regular intervals, at end of streaming etc.). Reporting is done with an XML-defined schema which is identical for all three cases.

## 10.2	VR metrics

As stated earlier in this document, several of the existing metrics in TS 26.247 are also valid for a VR case, while there is a need to add new VR-specific metrics. However, from a configuration and reporting perspective, these can just be added into the existing configuration and reporting structure. Thus only minimal changes are expected due to these additions.

This is true as long as all metrics can be specified in TS 26.247, i.e. they can be calculated based on information available to the DASH client. If a new metric would need input from a functional component which is not covered by TS 26.247 there might be a need of further changes in configuration and or reporting methods.

# 11	Conclusions

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 10-2017 | SA4#95 | S4-170762 |  |  |  | Initial version | 0.0.1 |
| 10-2017 | SA4#95 | S4-170975 |  |  |  | Editorial modifications | 0.0.2 |
| 10-2017 | SA4#95 | S4-170982 |  |  |  | Inclusion of candidate reference model for QoE metric measurement | 0.1.0 |
| 12-2017 | Adhoc | S4-AHV003 |  |  |  | Inclusion of VR QoE Metrics Evolution | 0.1.1 |
| 12-2017 | Adhoc | S4-AHV025 |  |  |  | Inclusion of VR QoE metrics | 0.1.1 |
| 12-2017 | Adhoc | S4-AHV026 |  |  |  | Inclusion of use case for VR QoE Study | 0.1.1 |
| 02-2018 | SA4#97 | S4-180205 |  |  |  | Inclusion of viewport-related QoE aspects | 0.2.0 |
| 04-2018 | SA4#98 | S4-180560 |  |  |  | Inclusion of viewport-related information flow | 0.3.0 |
| 07-2018 | SA4#99 | S4-180757 |  |  |  | Inclusion of presentation delay metrics | 0.4.0 |
| 10-2018 | SA4#100 | S4-181064 |  |  |  | Changes to presentation delay metrics | 0.5.0 |
| 10-2018 | SA4#100 | S4-181065 |  |  |  | Added text to configuration and reporting clause | 0.5.0 |