| 3GPP TS 28.312 V0.6.0 (2021-10) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Intent driven management services for mobile networks (Release 17 ) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	8

4	Concepts and Background	8

4.1	Intent concept	8

4.1.1	Introduction	8

4.1.2	Intent categorizes based on user types	8

4.1.3	Intent categorizes based on management scenario types	9

4.2	Intent driven management	10

4.2.1	Support for intent driven management	10

4.2.2	Intent driven MnS	10

4.3	Intent driven closed-loop	11

4.4	Relation between rule, policy and intent	12

5	Specification Level Requirements	12

5.1	Use cases	12

5.1.1	Network and service deployment	13

5.1.1.1	Intent driven radio network provisioning	13

5.1.1.1.1	Introduction	13

5.1.1.1.2	Requirements	13

5.1.1.2	Intent driven radio service provisioning	13

5.1.1.2.1	Introduction	13

5.1.1.2.2	Requirements	14

5.1.1.3	Intent driven service deployment	14

5.1.1.3.1	Introduction	14

5.1.1.3.2	Requirements	14

5.1.2	Network and service optimization	14

5.1.2.1	Intent driven management for coverage optimization	14

5.1.2.1.1	Introduction	14

5.1.2.1.2	Requirements	14

5.1.2.2	Intent driven management for RAN UE throughput optimization	15

5.1.2.2.1	Introduction	15

5.1.2.2.2	Requirements	15

5.2	Requirements	15

6	Stage 2 definition for Intent Driven Management	15

6.1	Management operation for Intent (MnS component type A)	15

6.2	Information model definition for Intent (MnS component typeB)	16

6.2.1	Information model definition for Intent	16

6.2.1.1	Class diagram	16

6.2.1.1.1	Relationship	16

6.2.1.1.2	Inheritance	16

6.2.1.2	Class definition	17

6.2.1.2.1 	Intent <<IOC>>	17

6.2.1.2.1.1	Definition	17

6.2.1.2.1.2	Attributes	17

6.2.1.2.1.3	Attribute constraints	17

6.2.1.2.2	IntentExpectation	17

6.2.1.2.2.1	Definition	17

6.2.1.2.2.2	Attributes	17

6.2.1.2.2.3	Attribute constraints	17

6.2.1.2.3	IntentReport	17

6.2.1.2.3.1	Definition	17

6.2.1.2.3.2	Attributes	17

6.2.1.2.3.3	Attribute constraints	18

6.2.1.2.4	IntentExpectation	18

6.2.1.2.4.1	Definition	18

6.2.1.4	Attribute definition	19

6.3	Procedures for intent management	19

6.3.1	Introduction	19

6.3.2	Create an intent	19

6.3.3	Modify an intent	21

6.3.4	Delete an intent	21

6.3.5	Query an intent	22

7	Stage 3 definition for Intent Driven Management	22

Annex A (informative): PlantUML source code	23

A.1	Procedures for intent management	23

A.1.1 Create an intent	23

A.1.2	Modify an intent	23

A.1.3	Delete an intent	23

A.1.4 query an intent	23

A.2	Information model definition for Intent	24

A.2.1	Relationship UML diagram for intent (Figure 6.2.1.1.2-1)	24

Annex <X> (informative): Intent Life Cycle Management	24

X.1 Intent Life Cycle Management	24

Annex <X> (informative): Change history	27

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

Introduction

1	Scope

The present document specifies concept, use cases, requirements and solutions for the intent driven management for service or network management.

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 28.531: "Management and orchestration; Provisioning".

[3]	3GPP TS 28.532: " Management and orchestration; Generic management services"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

Intent: the expectations including requirements, goals and constraints given to a 3GPP system, without specifying how to achieve them.

Editor's note: Alignment with other organization is to be considered.

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

4	Concepts and Background

Editor's note: this clause will contain concepts and overview information relevant to the intent driven management.

4.1	Intent concept

4.1.1	Introduction

An intent specifies the expectations including requirements, goals and constraints for a specific service or network management workflow. The intent may provide information on particular objective and possibly some related details. Following are some general concepts for intent:

- An intent is typically understandable by humans, and also needs to be interpreted by the machine without any ambiguity.

- An intent focuses more on describing the “What” needs to be achieved but less on “How” that outcomes should be achieved, The  intent expresses the metrics that need to be achieved and not how to achieve them. This not only relieves the burden of the consumer knowing implementation details but also leaves room to allow the producer to explore alternative options and find optimal solutions. Intent describes the properties that allows a satisfactory outcome.

- The expectations expressed by an intent is agnostic to the underlying system implementation, technology and infrastructure. Area can be used as managed object in the expectations expressed by an intent to achieve system implementation, technology and infrastructure agnostic.

- An intent needs to be quantifiable from network data so that the fulfilment result can be measured and evaluated.

Intent can be categorized based on different user types or different management scenario types.

4.1.2	Intent categorizes based on user types

Based on roles related to 5G networks and network slicing management defined in clause 4.8 in TS 28.530[X], different kinds of intents are applicable for different kinds of standardized reference interfaces.

Figure 4.1.2-1: High-level model of different kind of intents expressed by different roles

-	Intent from Communication Service Customer (Intent-CSC): Intent from Communication Service Customer enables Communication Service Customer (CSC) to express which properties of a communication service the CSC may request from CSP without knowing how to do the detailed management for communication service. For example, Intent-CSC can be 'Enable a V2X communication service for a group of vehicles in certain time'.

-	Intent from Communication Service Provider (Intent-CSP): Intent from Communication Service Provider enables Communication Service Provider (CSP) to express an intent about what CSP would like to do for network without knowing how to do the detailed management for network. For example, Intent-CSP can be 'Provide a network service supporting V2X communications for highway-417 to support 500 vehicles simultaneously'.

-	Intent from Network Operator(Intent-NOP): Intent from Network Operator enables Network Operator (NOP) to provide what NOP would like to do for group of network elements (i.e. subnetwork) management and control without knowing how to do the detailed management for the network elements. For example, Intent-NOP can be 'Provide a radio network service to satisfy the specified coverage requirements and UE throughput requirement in certain area'.

4.1.3	Intent categorizes based on management scenario types

Different types of intents are applicable for different types of management scenarios.

- Intent for network and service design/planning: Editor’s Note: The Intent for network and service design/planning is FFS.

- Intent for network and service deployment: enables a consumer to express the network or service instance to be deployed. Examples of intent for network and service deployment are “Provisioning radio network in the specified area with specified frequency information, transport information, and radio information (e.g. range of PCI, Cell Id), network capacity and performance information” or “provisioning radio service in the specified area with certain service characteristics (e.g. SLS)”.

- Intent for network and service assurance: enables a consumer to express intents for network and service assurance, and the following two sub-groups:

  - Intent for network and service maintenance: enables a consumer to express the network and service status (e.g. performance, alarm, issue) to be monitored or the network and service issues to be addressed. Examples of intent for network and service maintenance are “Monitoring coverage performance in the specified area by geographic grid granularity” or “Addressing the certain weak coverage issues in specified area”.

  - Intent for network and service optimization/assurance: enables a consumer to express the performance objectives of network and service to be improved. Examples of intent for network and service optimization are “Optimize radio network in the specified area with certain RAN UE throughput objectives (e.g. target average RAN UE DL throughput, target percentage of UE with the RAN UE DL throughout less than 5Mbps)” or “Optimize radio network in the specified area with certain coverage objectives (e.g. target coverage ratio, target average RSRP)”.

4.2	Intent driven management

4.2.1	Support for intent driven management

In Intent-driven management, the consumer provides its intent to the producer of a set of management services that would be consumed in a specific domain. For example, for the purpose of requesting a radio network with a new coverage, one possible solution (non-intent driven approach) is to use the set provisioning MnSs to decommission a cell and instantiate the cell to a new Node B for the new coverage. The alternative solution (intent driven approach) is to use management service produced by the domain is what may be referred to as the Intent-driven MnS by stating the intent for the radio network for the new coverage, based on the intent, system can trigger actions (e.g. decommission a cell and instantiate the cell to a new Node B) to satisfy received intent.

The producer of an Intent-driven MnS shall allow the consumer to manage the service and / or network resources through the use of intents.  The producer shall support the capabilities for intent fulfilment, which include the following:

- The consumer states the intent to be fulfilled (which can be implemented by createMOI operation on the Intent IOC) and the producer receives and acknowledges the receipt of the intent.

- The producer validates the intent and then translates the intent to identify the required internal logic needed to fulfil the intent.

- The producer executes the compiled logic to fulfil the intent.

- The producer may report about the fulfilment result of the intent.

4.2.2	Intent driven MnS

Introduction of service-based architecture for 5G, in combination with functional model of business roles, exceeds the level of complexity for managing network in different scenarios (including scenarios for design/planning, deployment, maintenance and optimization) both in a single and multivendor network. New/simpler ways of managing are needed.

Actions of an intent driven MnS related to the fulfilment of intents may be categorized as intent deployment and intent assurance. Intent fulfilment refers to the steps taken to satisfy  a newly received intent or an update to an existing intent. The goal of intent fulfilment is to bring the network or service’s state to satisfy the new or updated intent. The fulfilment of some intents may end at the intent deployment, the case, if the intent’s goal simply describes the availability or presence of a service. In other cases, the intent’s goal describes the assurance requirements for a network or service (e.g., quality of service, end user experience, SLS, etc.) in addition to the need of existence of a service. Those intents have their fulfilment tied to the operation of the referred service or network function and may require frequent recurring actions to keep those assurance requirements achieved. This part of the intent fulfilment is referred to as intent assurance.

An Intent driven MnS allows its consumer to express intents for managing the network and services and obtain the feedback of intent evaluation result. The Intent-driven MnS producer have the following capabilities:

- Validate the intent.

- Translate the received intent to executable actions as follows:

- Performing service or network management tasks

- Identifying, formulating and activating service or network management policies

- Evaluate the result/information about the intent fulfilment (e.g. the intent is initially satisfied or not) and intent assurance (e.g. the intent is continuously satisfied).

The following figure 4.2-1 shows the model of Intent-driven MnS.

Figure 4.2-1: Intent-driven MnS

The intents may be fulfilled by utilizing multiple mechanisms including among others: Rule-based mechanisms, closed loop mechanisms and AI/ML based mechanisms. These mechanisms can be combined in solutions of various complexity, ranging from a simple approach rule-based mechanisms, to more elaborate solutions combining AI/ML, closed loop automation to ensure the fulfilment of intents.

When the intent is created on the MnS producer, the MnS producer may consume other management services (including non-intent driven MnS and intent driven MnS) to fulfil or satisfy the intent, e.g. creating new assurance closed control loop instance(s) or using assurance closed control loop instance ACCL instance(s) to satisfy the intent. The internal implementation of the intent fulfilment will however not be standardized.

An Intent driven MnS includes the following management capabilities to support intent lifecycle management:

- Create an intent, a MnS Consumer request to create a new intent on the MnS producer.

- Activate an intent, MnS Consumer request to activate an intent on the MnS producer when the intent is  suspended.

- De-activate an intent, MnS consumer request to de-activate an intent on the MnS producer for a temporary suspension.

- Delete an intent, MnS Consumer request to remove an intent on the MnS producer.

- Modify an intent, MnS Consumer request to modify the content of the intent (e.g. optimization goal) on the MnS producer.

- Query an intent, MnS Consumer request to return the content and state (e.g. active, inactive) of the intent on the MnS producer.

4.3	Intent driven closed-loop

Intent can be used for management and control of closed-loop automation (e.g. intent can be used to specify the goals for the closed-loop), which means the intent can be translated to policies and management tasks that the MnS producer needs to execute for the closed-loop automation. In the intent driven management approach, the mechanisms that the  MnS producer using closed-loop automation mechanisms to satisfy the intent is the implementation of the MnS producer and shall not be standardized. The relation of the Intent driven MnS and the closed-loop automation with the Intent driven MnS producer is shown in the figure 4.3-1.

Figure 4.3-1 Intent driven closed-loop

4.4	Relation between rule, policy and intent

An intent specifies the expectations including requirements, goals, and constraints for a specific service or network management workflow, while a policy specifies the action(s) to be taken when given condition occurs and rules specifies the explicit or formula logics to be executed. For certain scenarios, policies can be used in conjunction with intents to achieve the autonomous purposes. Figure 4.4-1 describes the relation between rule, policy and intent in the “what-how” view. As it now stands, the telecom systems are mainly focused on "how" and "less what". The current 5G networks brings more operational complexities, and the telecom system need to be able to adapt their operation to the business objectives of the operator as well as expectations of customer, which is driving customer to shift the focus from "how" to "what". The first step towards that shift, has been shift from "Rule based management" to "Policy driven management", with more focus on "how" and less on "what" covering domain specific issues/aspects (an example for policy is when the average throughput is lower than certain threshold, take specified actions). As technologies are evolving and the level of complexity exceeds, the need for an abstraction level description (i.e. Intent) becomes more apparent (an example for intent is the target average throughput for certain area should be assured). An intent driven system will be able to learn the behaviour of networks and services and allows a customer to provide the desired state, without detailed knowledge of how to get to the desired state.

Figure 4.4-1: Relation between rule, policy and intent

5	Specification Level Requirements

Editor's note: this clause will contain specific use case and requirements for intent driven management.

5.1	Use cases

Editor's note: this clause will contain specific use case for intent driven management. The use case can be categorized into some sub-clauses (e.g. one sub-clause for deployment related use case, and one sub-clause for optimization related use cases)

5.1.1	Network and service deployment

5.1.1.1	Intent driven radio network provisioning

5.1.1.1.1	Introduction

This use case describes a scenario where a MnS consumer express intent for radio network provisioning in the specified area to a MnS producer. In this scenario, MnS consumer expresses its intent for radio network provisioning to MnS producer, which includes coverage area information (e.g. geographical areas), radio setting parameter sets (e.g. frequency information, range of gNB Id, range of PCI, range of Cell Id, range of nRTAC), transport setting parameters (including OM transport information (e.g. OMlocalIPaddress, OMremoteIPaddress, OMNextHopInfo) and NG transport information (e.g. list of NGlocalIPaddress, list of NGremoteIPaddress)), and supported network capacity information (e.g. maximum UE number) and network performance information (e.g. UL/DL throughput).

Based on the intent for radio network provisioning received, MnS producer identifies corresponding RAN NEs discovered in the specified coverage area, analyses and generates the configuration parameters (including radio configuration parameters and transport configuration parameters) for each identified RAN NE and corresponding Cells, creates MOI(s) for each RAN NEs and Cells and configure the created MOI(s), and performs verification for configured RAN NEs to enable the radio network in the specified area is successfully provisioned and satisfy the received intent.

MnS producer notifies MnS consumer about the fulfilment information of radio network provisioning intent after the verification is finished.

5.1.1.1.2	Requirements

REQ-Intent_Deploy_Net-CON-1 The intent driven MnS shall have capability enabling MnS consumer to express intent for radio network provisioning for specified area to MnS producer.

REQ-Intent_Deploy_Net-CON-2 The intent driven MnS shall have capability enabling MnS consumer to obtain fulfilment information of intent of radio network provisioning.

5.1.1.2	Intent driven radio service provisioning

5.1.1.2.1	Introduction

This use case describes a scenario where a MnS consumer express intent for radio service (radio network as service) provisioning in the specified area to a MnS producer.

In this scenario, MnS consumer expresses its intent for radio service provisioning to MnS producer, which includes coverage area information (e.g. geographical areas), and supported service capacity information (e.g. maxNumberofUEs, activityFactor) and service performance information (e.g. serviceType, dLThptPerUEPerSubnet, uLThptPerUEPerSubnet).

Note: the slice agnostic parameters in RAN SliceProfile can be used for service capacity information and service performance information.

Based on the intent for radio service provisioning received, MnS producer decides to use radio network with slicing or radio network without slicing to support the intent.

- In case of using radio network with slicing, the use case for network slice subnet creation defined in TS 28.531[2] can be reused.

- In case of using radio network without slicing, MnS producer identifies corresponding RAN NEs and cells in the specified coverage area to support the intent, analyses and configure the service specific configuration parameters for corresponding RAN NE and Cells (e.g. RRM policies, supported services).

MnS producer notifies MnS consumer about the fulfilment information of radio service provisioning intent after the service configuration is finished.

5.1.1.2.2	Requirements

REQ-Intent_Deploy_Sev-CON-1 The intent driven MnS shall have capability enabling MnS consumer to express intent for radio service provisioning for specified area to MnS producer.

REQ-Intent_Deploy_Sev-CON-2 The intent driven MnS shall have capability enabling MnS consumer to obtain fulfilment information of intent of service network provisioning.

5.1.1.3	Intent driven service deployment

5.1.1.3.1	Introduction

This use case describe a scenario where the MnS consumer, express the intent to deploy a service (e.g at the edge of the network). The intent express service characteristics include service type (URLLC, eMBB), service requirements (number of concurrent subscribers and number of concurrent sessions), service availability and the target location.

5.1.1.3.2	Requirements

REQ-Intent_Deploy_Net-CON-1 The 3GPP Management system shall have capability enabling authorized MnS consumer to express intent for deploying a service (e.g at the edge of the network) to MnS producer.

5.1.2	Network and service optimization

5.1.2.1	Intent driven management for coverage optimization

5.1.2.1.1	Introduction

In this scenario, MnS consumer expresses its intent for coverage optimization to NEP, which includes optimization area information (e.g. geographical area), optimization RATs (e.g. NR only, EUTRAN only, or all RATs), optimization time (e.g. start time, end time) or period (e.g. per day, per hour), coverage optimization targets (e.g. target average RSRP, target weak coverage ratio).

Based on the intent for coverage optimization received, MnS producer collects and analyses corresponding coverage related data (e.g. RSRPs of the serving cell and neighbour cells reported by each UE with anonymous id (e.g., C-RNTI) and location information in the MDT reports)) of corresponding RAN NEs in the specified areas, identifies the potential coverage issues which will impact the coverage targets satisfaction, analyses the identified coverage issue and corresponding solutions, evaluates, decides and adjusts the coverage configuration parameters. The Artificial intelligence or machine learning technologies may be used in above workflow to satisfy the intent, for example, online iteration optimization technologies may be used to selecting the best coverage configuration parameters rapidly.

MnS producer continuously monitors the coverage performance (e.g. weak coverage ratio, average RSRP) for the specified area, and decides whether coverage targets described in the intent is satisfied. If not satisfied, NEP iteratively executes above workflows (including collect, identification, analysis, evaluation, decision and adjustment) to fulfil the coverage targets.

MnS producer may notify MnS consumer about the intent fulfilment information, including coverage performance for the specified area (e.g. weak coverage ratio, coverage hole ratio, average RSRP) which enables MnS consumer to monitor the intent for coverage optimization.

5.1.2.1.2	Requirements

REQ-Intent_Opt_Cov-CON-1 The intent driven MnS shall have capability enabling MnS consumer to express intent of coverage optimization for specified area to MnS producer.

REQ-Intent_Opt_Cov-CON-2 The intent driven MnS shall have capability enabling MnS consumer to obtain fulfilment information of coverage optimization.

5.1.2.2	Intent driven management for RAN UE throughput optimization

5.1.2.2.1	Introduction

In this scenario, MnS consumer expresses its intent for RAN UE throughput optimization to MnS producer, which includes optimization area information (e.g. geographical area), optimization RATs (e.g. NR only, EUTRAN only, or all RATs), optimization time (e.g. start time, end time) or period (e.g. per day, per hour), RAN UE throughput optimization targets (e.g. target average UL/DL RAN UE throughput, target percentage of UE with low UL/DL RAN UE throughput (e.g. < 5 Mbps), target percentage of UE with high UL/DL RAN UE throughput (e.g. > 50 Mbps)), optional optimization scope (e.g. specific service type, specific UE groups).

Based on the intent for RAN UE throughput optimization received, MnS producer collects and analyses corresponding RAN UE throughput related data in the specified areas, identifies the potential RAN UE throughput issues (e.g. low RAN UE throughput for certain areas, high load for certain areas, frequent handover), which will impact RAN UE throughput intent satisfaction, analyses, evaluates, decides and adjusts the radio feature configuration parameters for impacted RAN NEs/Cells in the specified areas. The Artificial intelligence or machine learning technologies may be used to select the optimal radio feature configuration parameters set rapidly to satisfy RAN UE throughput target.

MnS producer continuously monitors the RAN UE throughput performance (e.g. average UL/DL RAN UE throughput, percentage of UE with low UL/DL RAN UE throughput (e.g. < 5 Mbps), percentage of UE with high UL/DL RAN UE throughput (e.g. > 50 Mbps)) for the specified area, and decides whether RAN UE throughput target is satisfied.

MnS producer may notify MnS consumer about the intent fulfilment information, including the RAN UE throughput performance (e.g. average UL/DL RAN UE throughput, percentage of UE with low UL/DL RAN UE throughput) for the specified area which enables MnS consumer to monitor the intent for coverage optimization.

5.1.2.2.2	Requirements

REQ-Intent_Opt_Thp-CON-1 The intent driven MnS shall have capability enabling MnS consumer to express intent of RAN UE throughput optimization for specified area to MnS producer.

REQ-Intent_Opt_Thp-CON-2 The intent driven MnS shall have capability enabling MnS consumer to obtain fulfilment information of intent for RAN UE throughput optimization.

5.2	Requirements

Editor's note: this clause will contain specific requirements for intent driven management.

6	Stage 2 definition for Intent Driven Management

Editor's note: this clause will specify the stage2 definition for Intent driven management which may include the management operations, management entities and management information

Editor’s Note: The coordination/alignment work with TM Forum for the intent lifecycle management needs to be considered.

6.1	Management operation for Intent (MnS component type A)

The operations (e.g. createMOI operations) and notifications (e.g. notifyMOIcreation) of generic provisioning MnS defined in TS 28.532[3] can be used for intent lifecycle management. The intent can be treated as object instance.

Editor’s Note: The createMOI defined in TS 28.532[3] needs to add scenario of MnS producer allocate the DN for the intent.

6.2	Information model definition for Intent (MnS component typeB)

6.2.1	Information model definition for Intent

Editor’s Note: The following information model needs to be revisited based on the further discussion, and the alignment/coordination work with other SDO needs to be considered, which may impact the following information model.

6.2.1.1	Class diagram

6.2.1.1.1	Relationship

Figure 6.2.1.1.1-1 Relationship UML diagram for intent

Editor’s Note: The detailed model for Intent, IntentReport and IntentExpectation objects (e.g. is it <<IOC>>, <<DataType>>, or string) is FFS as their relationship needs to be decided later based on the content of these three objects 6.2.1.1.2	Inheritance

Figure 6.2.1.1.2-1 Inheritance UML diagram for intent

6.2.1.2	Class definition

6.2.1.2.1 	Intent <<IOC>>

6.2.1.2.1.1	Definition

This IOC represents the properties of an Intent. The Intent IOC contains one or multiple intentExpectation(s) which includes MnS consumer’s requirements, goals and constraints given to a 3GPP system.

The Intent IOC includes the attribute objectClass and objectInstance from the TOP IOC. The value of attribute objectClass is “Intent” and the value of attribute objectInstance is the DN of the instance of Intent IOC.

6.2.1.2.1.2	Attributes

The Intent includes attributes inherited from TOP IOC (defined in TS 28.622) and the following attributes:

| Attribute Name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| intentExpectation | M | T | T | F | T |
| userLabel | M | T | T | F | T |
| intentFulfilStatus | M | T | F | F | T |

Editor’s Note: whether other the attributes are needed for the Intent IOC needs further discussion.

6.2.1.2.1.3	Attribute constraints

None

6.2.1.2.2	IntentExpectation

6.2.1.2.2.1	Definition

IntentExpectation class represent MnS consumer’s requirements, goals and constraints given to a 3GPP system.

Editor’s Note: more description for IntentExpectation will be added later based on the further discussion.

6.2.1.2.2.2	Attributes

TBD

6.2.1.2.2.3	Attribute constraints

TBD

6.2.1.2.3	IntentReport

6.2.1.2.3.1	Definition

IntentReport class represent intent fulfilment feedback information that MnS consumer can obtained from a 3gpp system.

Editor’s Note: more description for IntentReport will be added later based on the further discussion.

6.2.1.2.3.2	Attributes

TBD

6.2.1.2.3.3	Attribute constraints

TBD

6.2.1.2.4	IntentExpectation

6.2.1.2.4.1	Definition

Following Content for MnS’s expectation on a radio network is used as example to discuss the concrete model for IntentExpectation:

Editor’s Note: following content needs to be revisited based on further discussion for intent model, how to model IntentExpectation is FFS.

Following described the information/attributes for MnS Consumer’ s expectation to deliver/ensure on a radio network,

-	ManagedObject, it describes the managed object (i.e. RadioNetwork) expressed by MnS’s expectation on a radio network.

-	ExpectedAreas, it describes a list of coverage areas for radio network. This can be implemented by multiple choices, e.g. CoverageAreaPolygon, TrackingAreaCode.

-	RANCoverageTarget, it describes the properties of coverage quality for the radio network, which includes:

-	targetWeakRSRPRatioInfo, it describes the list of target Weak RSRP Ratio for the radio network. Each TargetWeakRSRPRatioInfo includes WeakRSRPRatio and WeakRSRPThreshold.

-	targetLowSINRRatioInfos, it describes the list of target low SINR Ratio for the radio network. Each TargetLowSINRRatioInfo includes LowSINRThreshold and LowSINRRatio.

-	RANCapacityTarget, it describes the properties of network capacity for the radio network, which includes：

-	targetMaximumUENumber, it describes the maximum number of should be supported by the radio network

-	targetActivityFactor, it describes the maximum number of should be supported by the radio network

-	RANUEThptTarget, it describes the properties of the RAN UE throughput for the radio network, which includes:

-	targetAveULRANUEThpt, it describes the average UL RAN UE throughput for the radio network

-	targetAveDLRANUEthpt, it describes the average UL RAN UE throughput for the radio network

-	targetLowULRANUEThptRatioInfos, it describes the list of target low uplink RAN UE throughput ratio for the network, which includes LowULRANUEThptRatio and LowULRANUEThptThreshold

-	targetLowDLRANUEThptRatioInfos, it describes the list of target low downlink RAN UE throughput ratio for the radio network, which includes LowDLRANUEThptThreshold and LowDLRANUEThptRatio

-	RANConstraint, it describes the properties of a set of constraints for the radio network, which includes:

-	radioConstraints, it describes the properties of a set of radio constraints for the radio network, which includes cellIdRanges, gNBIdRanges, tACRanges, pLMNIdList and nRPCIRanges.

-	frequencyConstraints, it describes the properties of a set of frequency constraints for radio network, which includes arfcnUL/DL and bSChannelBwUL/DL

-	transportConstraints, it describes the properties of a set of transport constraints to be used in the radio network, including ngcLocalIpAddressList, nguRemoteIpAddressList, logicInterfaceId and nextHopInfo.

6.2.1.4	Attribute definition

| Attribute Name | Documentation and Allowed Values | Properties |
| --- | --- | --- |
| userLabel | A user-friendly (and user assignable) name of the intent. allowedValues: Not Applicable | type: String multiplicity: 1 isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentExpectation | It indicates the expectations including requirements, goals and constraints given to a 3GPP system. The intentExpectation includes the intentAreas which describes a list of coverage areas for the intent to be applied. Editor’s Note: the detailed intentExpectation is for further discussion | type: FFS multiplicity: * isOrdered: F isUnique: F defaultValue: None isNullable: False |
| intentFulfilStatus | It describes the current status of the intent fulfilment result, which is configured by MnS producer and can be read by MnS consumer. allowedValues: "FULFILLED", “NOT_FULFILLED Editor’s Note: whether other allowed values should be supported is FFS, and the name for the attribute intentFulfilStatus is FFS. | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

6.3	Procedures for intent management

6.3.1	Introduction

This clauses describes the procedures for intent management.

Editor’s Note: Capability of version handling of Intent need further discussion.

6.3.2	Create an intent

The Figure 6.3.2-1 illustrates the procedure for create a new intent.

Figure 6.3.2-1 Procedure for create an intent

- MnS Consumer sends a request to create an intent instance to MnS Producer with ‘objectClass’ and list of [Attribute,Vlaue] for the new intent to be created. The detailed [Attribute,Value] see the concrete intent IOC defined in clause 6.2. ‘objectCalss’ is the name for the concrete intent IOC.

- Based on the request, the MnS Producer creates the concrete intent MOI (i.e. instance of intent IOC) with value for attribute ‘objectInstance’ allocated, and configure the new created intent MOI with the received list of [Attribute, Value]. ‘objectInstance’ is the identifier (DN) for the concrete intent MOI.

- MnS Producer sends a response to the MnS consumer with status (OperationSucceeded or OperationFailed) and ‘objectInstance’ of the created intent MOI. The response information may also include the possible reasons for the unsuccessful executions (e.g., conflicting with existing intents, the intent infeasible).

- Based on the intent, MnS Producer identifies the MOI for managed entities (e.g. ManagedElement, ManagedFunction) and derives one or more executable management tasks (including deployment and configuration requirements) for these managed entities, then MnS producer deploys or configures corresponding managed entities to satisfy the intent.

- During the execution of intention, MnS producer continuously monitors intent fulfilment status.

- MnS producer analyses and adjusts the managed entities to ensure the intention is continuously satisfied.

- MnS Producer may notify MnS Consumer about the intent fulfilment information, including DN of intent MOI, and fulfillStatus.

6.3.3	Modify an intent

The Figure 6.3.3-1 illustrates the procedure for modify an existing intent.

Figure 6.3.3-1 Procedure for modify an intent

- MnS Consumer sends a request to modify an intent intsnace to MnS Producer with ‘objectInstance’ of the intent MOI and List of [‘Attrribute’, ‘newValue’] to be modified. The detailed [Attribute,Value] see the concrete intent IOC defined in clause 6.2.

- Based on the request, MnS Producer configure the intent MOI with list of ‘Attribute’ = ’newValue’ which is required to be modified.

- MnS Producer sends a response to the MnS consumer with status (OperationSucceeded or OperationFailed), ‘objectInstance’ of the modified intent MOI and, and list of [‘Attrribute’, ‘newValue’] which is modified.

Editor’s Note: the subsequence after the intent MOI modified is FFS.

6.3.4	Delete an intent

The Figure 6.3.4-1 illustrates the procedure for modify an existing intent.

Figure 6.3.4-1 Procedure for delete an intent

- MnS Consumer sends a request to delete an intent to MnS Producer with ‘objectInstance’ of the intent MOI.

- Based on the request, MnS Producer delete the intent MOI.

- MnS Producer sends response to the MnS consumer with status (OperationSucceeded or OperationFailed), ‘objectInstance’ of the deleted intent MOI.

Editor’s Note: the subsequence after the intent MOI deleted is FFS.

6.3.5	Query an intent

The Figure 6.3.5-1 illustrates the procedure for query an intent.

Figure 6.3.5-1 Procedure for query an intent

- MnS Consumer sends a request to query an intent instance to MnS Producer, with objectInstance of the existing intent MOI, scope, and list of attributes of intent IOC. The list of attributes identifies the attributes to be returned by this operation.

- Based on the request, the MnS Producer queries the concrete intent MOI.

- MnS Producer sends a response to the MnS consumer with objectClass, objectInstance, status (e.g. fulfillStatus and other status), and list of [Attribute,Value] which is defined in clause 6.2.

7	Stage 3 definition for Intent Driven Management

Editor's note: this clause will specify the solution set for Intent Driven Management defined in clause 6.

Annex A (informative): PlantUML source code

A.1	Procedures for intent management

A.1.1 Create an intent

@startuml

title "[Create an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

Collections "ManagedEntity" as ManagedEntity

MnS_Consumer -> MnS_Producer: 1. Request to create an intent instance ('objectClass',list of \n[Attribute,Value] of intent IOC)

MnS_Producer -> MnS_Producer: 2. Create and configure intent MOI

MnS_Producer -> MnS_Consumer: 3. Response for create an intent instance \n (status, DN of intent MOI, reason)

alt status is OperationSucceeded

Ref over MnS_Producer, ManagedEntity: 4. Perform service or network management tasks

loop

Ref over MnS_Producer, ManagedEntity: 5. Evaluate intent fulfilment

opt

Ref over MnS_Producer, ManagedEntity: 6. Adjust to fulfil the intent requirement

end

end

MnS_Producer -> MnS_Consumer:7. Notify of feedback\n (DN of intent MOI, fulfillStatus)

End

hide footbox

@enduml

A.1.2	Modify an intent

@startuml

title "[Modify an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

MnS_Consumer -> MnS_Producer: 1.Request to modify an intent\n('objectInstance' of intent MOI,List of ['Attribute','newValue'])

MnS_Producer -> MnS_Producer: 2.Configure the intent MOI with \n List of ['Attribute' = 'newValue']

MnS_Producer -> MnS_Consumer: 3.Response for modify an intent\n ('objectInstance' of intent MOI)

hide footbox

@enduml

A.1.3	Delete an intent

@startuml

title "[Delete an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

MnS_Consumer -> MnS_Producer: 1.Requst to delete an intent instance\n('objectInstance' of intent MOI)

MnS_Producer -> MnS_Producer: 2.Delete the intent MOI

MnS_Producer -> MnS_Consumer: 3.Response for delete an intent intstance\n ('objectInstance' of intent MOI)

hide footbox

@enduml

A.1.4 query an intent

@startuml

title "[Query an intent]"

actor "MnS Consumer" as MnS_Consumer

participant "MnS Producer" as MnS_Producer

MnS_Consumer -> MnS_Producer: 1.Request to query an intent instance(objectInstance,scope, filter,\n AttributeList of intent IOC )

MnS_Producer -> MnS_Producer: 2.Query the intent MOI

MnS_Producer -> MnS_Consumer: 3.Response for query an intent instance \n (objectClass,objectInstance,status,list of [Attribute,value] of intent IOC )

hide footbox

@enduml

A.2	Information model definition for Intent

A.2.1	Relationship UML diagram for intent (Figure 6.2.1.1.2-1)

@startuml

hide circle

hide methods

hide members

skinparam class {

AttributeIconSize 0

BackgroundColor white

BorderColor black

ArrowColor black

}

skinparam   Shadowing false

skinparam  Monochrome true

skinparam  ClassBackgroundColor White

class “<<proxyClass>> \n  ManagedEntity ” as ManagedEntity{}

class “<<InformationObjectClass>>\n Intent ” as Intent {}

class “<<dataType>>\n IntentExpectation” as IntentExpectation{}

class “<<dataType>>\n IntentReport” as IntentReport{}

ManagedEntity  *-- "*" Intent

ManagedEntity *--  IntentReport: FFS

Intent --> "1..*" IntentExpectation

Intent --  IntentReport: FFS

note top of ManagedEntity

Represents the folllowing IOCs:

Subnetwork or

other IOCs which needs FFS

end note

@enduml

Annex <B> (informative): Intent Life Cycle Management

Editor's note: Some content of this Annex can become a normative content after it is aligned with 3GPP terminology

B.1 Intent Life Cycle Management

As the system state (e.g. number and/or availability of the system resources) can change even after the Intent is accepted by the system, the Intent might not be best aligned with the system state. For example, system resources are overbooked, and an intent is failing to meet expectations of the external author or the resources of the system become underbooked which makes such a solution very expensive and therefore useless. Hence the creation of an Intent and keeping it aligned with the state of the receiving system, should be automated.

This means that the life cycle of the Intent can begin before an Intent is retrieved by the receiving system, e.g., an Intent is being defined in a sourcing system based on requirements towards a target system (e.g., to deliver a service with certain characteristics), then be optimized based on the system state (e.g. availability of system resources in certain area, time, etc.), then be refined if the initially captured requirement needs further detalization, etc.

Figure B.1-1: Intent Life-Cycle involving Automated Intent Creation

The intent lifecycle consists of the following phases:

Figure B.1 -2: Intent Lifecycle Phases

Detection:

In the detection phase, the Intent sourcing system as the system creating an intent, identifies if there is a need to define new or change/remove existing intent to set requirements, goals, constraints. An intent management functionality of this system has its own terminal goals to fulfill. It would break its terminal goals down into a suitable set of detailed instrumental goals. Typically, these instrumental goals need to be fulfilled by other functions and domains and therefore they need to be not only defined but distributed to suitable handlers throughout the system. This is what the system is doing using intent. In the detection phase the asourcing system can react to changes in its own terminal goals or to changes in the fulfillment in its instrumental goals. In this respect the system deriving an intent will need to collect information about the goals' fulfillment. Intent reports coming from an Intent target system, as a system to receive an intent are one source for this information. Through intent reports the sourcing system is able to react on intent handling outcomes in the Target System. In any case it is task of the Intent sourcing system to assure the fulfillment of its terminal goals and the first step is to detect if any changes are needed in its instrumental goals and therefore in the intent objects it owns.

Investigation:

In the investigation phase, the system creatin the Intent finds out what intents are feasible. This has two aspects: first, it needs to find right target system that have the right domain responsibilities and support the intent information the sourcing system wants to define. Intent handler capability management and detection would be used for this process.

The other aspect of investigation would be finding out if the wanted intent is realistic. This means, if the intent handler would be able to successfully reach the wanted goals and meet the requirements. This depends on the current resource situation and state of the system and can vary over time. Typically, the feasibility of intent is done through a guided negotiation process between the intent handler and intent owner. The owner can explore what the handling result of a wanted intent would be, what would be the best result the handler can achieve, or what would be the most challenging requirements, the aspiring intent handler can offer to fulfill.

Definition:

At the end of the investigation phase the system sourcing the Intent knows what is possible and what the target systems to be involved. By combining this information with the needs that were identified in detection, the sourcing system can now decide and plan all needed intents. In the definition phase the intent management functionality in sourcing system formulates the intent it needs to use, and it creates the respective intent objects.

Distribution:

In the distribution phase the sourcing system contacts a target system in order to send a new intent or modify or change an existing one. This way the sourcing system acts on the plan it has made in definition phase. In this phase an intent management functionality in target system start handling the intent by receiving it. The target system decides if it can accept the intent. If not, it would send a report with the rejection reason back to the sourcing system. While this finishes the lifecycle of this particular intent object, the sourcing system can start over with detection to create a new plan. If the target system accepts the intent, it starts operating based on it.

Operation:

Each intent constitutes yet another set of goals and requirements to be considered for decisions and actions by the target systems. The systems operate their domains of responsibility according to the given intents. They also report back to the sourcing system about status and success while continuously reacting to intent fulfillment threats. Intent reports would be evaluated by the sourcing system as part of its detection process, which leads to the next iteration of the intent life cycle.

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-10 | SA5#127 | S5-196174 |  |  |  | TS 28.312_000 | 0.0.0 |
| 2020-08 | SA5#132e | S5-204460 S5-204461 S5-204462 |  |  |  | 1. pCR 28.312 Add scope 2. pCR 28.312 Add skeleton 3. pCR 28.312 Add concept and background | 0.1.0 |
| 2020-11 | SA5#134e | S5-206358 S5-206359 S5-206360 S5-206361 S5-206362 S5-206363 |  |  |  | 1.pCR TS 28312 Update Clause 4.1.3 Intent categorizes based on management scenario type 2. pCR TS 28312 Update Clause 4.1.1 Introduction 3. pCR TS 28312 Add relation of intent driven management and policy driven management 4. pCR TS 28312 Add use case and requirements for coverage optimization 5. pCR TS 28312 Add use case and requirements for RAN UE throughput optimization 6. pCR TS 28312 Add use case and requirements for radio network deployment. | 0.2.0 |
| 2021-01 | SA5#135e | S5-211221 |  |  |  | 1.pCR TS 28.312 Update the intent definition to align with concept | 0.3.0 |
| 2021-03 | SA5#136e | S5-212207 S5-212391 S5-212392 S5-212442 S5-212443 S5-212448 |  |  |  | - pCR TS 28.312 Add concept for intent lifecycle management - pCR TS 28.312 Add conceptual description for intent MnS - pCR TS 28.312 Add stage 2 management operation for intent - pCR TS 28.312 Add stage 2 procedure for intent management - pCR TS 28.312 Add stage 2 information model definition for intent - pCR 28.312 Requirement of Intent for service at edge | 0.4.0 |
| 2021-05 | SA5#137e | S5-213477 |  |  |  | - pCR TS 28.312 Update Clause 4.2 and Clause 4.4 | 0.5.0 |
| 2021-10 | SA5#138e | S5-215367 S5-215369 S5-215540 S5-215541 S5-215542 S5-215654 |  |  |  | 1. pCR TS 28.312 Add subsequence procedure after the intent MOI created 2. pCR TS 28.312 Add procedure of query an intent MOI 3. pCR TS 28.312 Update intent information model 4. pCR TS 28.312 Add concrete RadioNetworkExpectation 5. pCR TS 28.312 Update description in clause 4.2.1 andclause 4.2.2 6. pCR TS 28.312 Add definitions around Intent LCM | 0.6.0 |