| 3GPP TR 28.824 V0.3.0 (2021-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Management and orchestration; Study on network slice management capability exposure (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions of terms, symbols and abbreviations	5

3.1	Terms	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Overview	6

4.1	General	6

4.1.1	Concepts related to network management capability exposure	6

4.1.2	Roles related to network management capability exposure	8

4.1.3	Types of interface for the exposure of network slice	8

4.1.4	Procedures related to network management capability exposure	11

4.2	Key issues	13

4.2.1 	Issue #1: Types of NSCs	13

4.2.2	Issue #2: Types of capabilities available for exposure	13

4.2.3	Issue #3: EGMF/MCEG	14

4.2.4	Issue #4: NSC-NSP service interaction	14

4.2.5	Issue #5: Relation to other SA5 work/study items	14

5	Use cases for network management capability exposure	15

5.1	Network slice management capability exposure	15

5.1.1	Description	15

5.1.2	Issue and gaps	15

5.2	Exposure of MnS for monitoring QoS of video application	15

5.2.1	Description	15

5.2.2	Issue and gaps	16

5.3	eMnS discovery service	16

5.3.1	Description	16

5.3.2	Issue and gaps	16

5.4	eMNS support to discovery systems	17

5.4.1	Description	17

5.4.2	Issue and gaps	17

5.5	Exposure of network slice as a product	17

5.5.1	Description	17

6	Potential requirements for network management capability exposure	21

7	Possible solutions for network management capability exposure	21

7.1	Possible solution for “exposed MnS support to discovery systems”	21

7.2	Possible solutions for eMnS discovery service	22

8	Conclusion and Recommendation	22

8.X	Issue #X	22

8.Y	Issue #Y	22

Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

Introduction

This report is to study on network slice management capability exposure.

1	Scope

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	TM Forum TMF622 Product Order API REST Specification

[3]		TM Forum TMF641 Service Ordering API

[4]	TM Forum TMF652 Resource Order Management API

[5]		3GPP TS 28.531: "Management and orchestration; Concepts, use cases and requirements"

[6]	3GPP TS 28.202: "Charging management; Network slice management charging in the 5G System (5GS); Stage 2"

3	Definitions of terms, symbols and abbreviations

3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BSS	Business Support System

CSP	Communication Service Provider

NOP	Network OPerator

4	Overview

4.1	General

4.1.1	Concepts related to network management capability exposure

4.1.1.1	Exposed Management Services

Exposed MnS (eMnS) represents the MnS that can be exposed by MnS producer to the external MnS consumer. eMnS may rely on a dedicated MnF (e.g. EGMF defined in 3GPP or function defined in other standard like TMF) that manages the exposure aspects.

Editor’s notes: Whether eMnS is exposed transparently to external MnS consumer via BSS or being processed through a dedicated exposure platform is FFS.

4.1.1.2	Exposure of Management Services

Exposure of management services indicates the case that an external MnS consumer which is outside 3GPP management system can indrectly access management capability offered by MnS producer within 3GPP management system. In order to enable the exposure of eMnS, an external customer, which has external MnS consumer, has to sign a contract, which contains the agreement on what eMnS optionally under what condition can be exposed, with an MNO. The signing of the contract may need the interaction with BSS. The condition can be certain constraint of eMnS access based on the contract, e.g. the access quota of certain eMnS, the access frequency of certain eMnS, etc.

4.1.1.3	Exposure via BSS

4.1.1.3.1	General

Exposure of service data to companies that are external to the operator are regulated by contracts. Different customers may have access to different managment capabilities. It may differ on what attributes/policies/intents are allowed to be changed, in which value ranges changes are allowed and which performance metrics are allowed to be exposed.

The slice data with the ServiceProfile is located in the network manager.

Even if the external interface always goes via BSS, there are different scenarios for how a customer, e.g. vertical, can influence the ServiceProfile.

The capabilities required from a Vertical have to be the same on the interface between NOP Network manager and CSP Service manager. The external interface is external to the administrative domain of the operator, while te internal interface is inside the administrative domain of the operator.

4.1.1.3.2	Exposure scenarios

Scenario 1: The NOP may interface to an external CSP that are doing the service management and have a machine to machine interface between the CSP’s "service manager" and the NOP’s "network manager" via BSS.

Figure 4.1.1.3.2-1 Service to network interface beeing an external interface

Scenario 2: The CSP might have a machine to machine interface towards their customers (e.g. a vertical) via their BSS. The CSP needs to have an interface towards NOP from their “service manager”. The NOP may have a machine to machine interface between the "service manager" and the "network manager" via BSS.

Figure 4.1.1.3.2-2 External customer interface to CSP

Scenario 3: The operator may have interface to other companies that are buying services and have machine to machine interface to the operators’s "service manager" via BSS.

Figure 4.1.1.3.2-3 BSS to customer interface being an external interface

Considering scenario 1, 2, and 3 the needs of an external customer always has to be reflected on the internal interface between NOP Network manager and CSP Service manager no matter how many companies are in the chain between the NOP and the External customer, Actions from a Vertical must be possible on the interface between NOP Network manager and CSP Service manager.

The management capabilities that an external customer (e.g. Vertical) has access to via a BSS have to be supported on the internal interface. How the capabilties are exposed to an external customer is not subject to standardization in 3GPP.

4.1.2	Roles related to network management capability exposure

4.1.2.1	Exposed MnS consumer

The logical entity, external to the 3GPP management system, accessing management capability offered by MnS producer is called an eMnS consumer. An eMnS consumer is equivalent to MnS consumer which is outside the 3GPP management system. An eMnS consumer is owned by an external customer (e.g. vertical such as Industry, Internet Company, etc) which may take the role of a CSC or NSC. The external customer usually has specific service requirements on 5G network.

4.1.2.2	Exposed MnS producer

The logical entity offering management capability that can be accessed by an eMnS consumer outside the 3GPP management system is called exposed MnS producer. An eMnS producer is owned by a service provider which may take the role of a CSP or NSP.

4.1.3	Types of interface for the exposure of network slice

4.1.3.1	Introduction

Different types of interfaces may be used during the whole lifecycle of network slice, including ordering, provisioning, operation phases. These interfaces can be categorized as:

- Inter-organization interfaces (represented East-West):

- Between the organizational entity playing the Network Slice Customer (NSC) role, named Company V here, and the organizational entity playing the Network Slice Provider (NSP) role, named Company A here:

- Interface Type-A to:

- create a product (network slice) order,

- retrieve information concerning a product (network slice) order,

- update a product (network slice) order,

- delete a product (network slice) order;

Candidate APIs for Interface Type-A include, but are not limited to, TMF API 622 (Product ordering).

Interface Type-A is not in the scope of 3GPP.

- Between the organizational entity playing the Network Slice Provider (NSP) role, named Company A here, and the two organizational entities playing the Communication Service Provider (CSP) role, acting as ‘partners’ and named Company X and Company Y respectively here

- Interface Type-X to:

- create a product order,

- retrieve information concerning a product order,

- update a product order,

- delete a product order;

Candidate APIs for Interface Type-X include, but are not limited to, TMF API 622 (Product ordering).

Interface Type-X is not in the scope of 3GPP.

- Intra-organization interfaces (represented North-South), internal to the organizational entity playing the Network Slice Provider (NSP) role:

- Between its BSS and its OSS/SML

- Interface Type-1 to:

- create a service order,

- retrieve information concerning a service order,

- update a service order,

- delete a service order;

Candidate APIs for Interface Type-1 include, but are not limited to, TMF API 641 (Service ordering).

Interface Type-1 is not in the scope of 3GPP.

- Between its OSS/SML and its OSS/NML

- Interface Type-2 to:

- create a network slice / network slice subnet,

- retrieve information concerning a network slice / network slice subnet,

- update a network slice / network slice subnet,

- delete a network slice / network slice subnet,

- collect PM and FM date concerning a network slice / network slice subnet;

Candidate APIs for Interface Type-2 include, but are not limited to, 3GPP TS 28.531, TS 28.532, TS 28.545, TS 28.550.

Interface Type-2 is in the scope of 3GPP/SA5.

The following clauses describe use cases may be not exhaustive.

In case of additional use cases, new types of interfaces may be needed.

4.1.3.2	Use case No.1: simple case

In this use case, Company-V (as the NSC) chooses a product from Company-A product offerings and sends a request to Company-A to order the product which is a network slice via Interface Type-A.

As Company-A can satisfy Company-V’s requirements by itself, it doesn’t have to count on partners.

In this use case, Interface Type-A, Type-1 and Type-2 are involved.

4.1.3.3	Use case No.2: partners involved

In this use case, Company-V (as the NSC) chooses a product from Company-A product offerings and sends a request to Company-A to order the product which is a network slice via Interface Type-A.

As Company-A can’t satisfy Company-V’s requirements by itself, it has to count on partners (Company-X and Company-Y), e.g. to provide RAN coverage in their respective countries.

In this use case, Interface Type-A, Type-1, Type-2 and Type-X are involved.

4.1.4	Procedures related to network management capability exposure

4.1.4.1	Introduction

When an NSP receives an order from an NSC for a network slice enabled product, the order is decomposed by the NSP’s BSS. Depending if the NSP employs services from 3rd party CSP’s different procedures may apply for the same order. The different procedures applicable to the same order may be invoked asynchronously and treated as independent procedures, however it may not result in loss of traceability between the original order and the orders that are created as result of decomposition. The following procedures have been identified:

- Procedure invoking internal service order after receiving product order from NSC

- Procedure invoking external product order after receiving product order from NSC

- Procedure invoking external service order after receiving product order from NSC

4.1.4.2	Procedure invoking internal service order after receiving product order from NSC

Figure 4.1.4.2.1 Procedure invoking internal service order after receiving product order from NSC

Editor’s Note: The details and the description of the steps in the sequence diagram as well as the actual names of requests and responses are FFS

4.1.4.3	Procedure invoking external product order after receiving product order from NSC

Figure 4.1.4.3.1 Procedure invoking external product order after receiving product order from NSC

Editor’s Note: The details and the description of the steps in the sequence diagram as well as the actual names of requests and responses are FFS

4.1.4.4	Procedure invoking external service order after receiving product order from NSC

Figure 4.1.4.4.1 Procedure invoking external service order after receiving product order from NSC

Editor’s Note: The details and the description of the steps in the sequence diagram as well as the actual names of requests and responses are FFS

4.2	Key issues

4.2.1 	Issue #1: Types of NSCs

The problem of network slice capability exposure is mostly relevant for B2B/B2B2C market, where Network Slice as a Service (NSaaS) [1] model applies. In this regard, different types of NSCs can be found.

- Baseline vertical customer: it corresponds to a NSC which is only interested in monitoring the network slice, to verify it behaves as expected, according to the SLA. This NSC does typically have no telco experience, and is associated to a network slice that is entirely deployed on a PLMN. The capabilities offered to a baseline vertical customer includes the ability for this NSC to receive information on subscribed items, including network slice status (e.g. active, inactive) and subscribed management data (e.g., KPIs, events/logs, trace data, etc.). The profile of this NSC type is a ‘passive NSC’.

- Advanced vertical customer: it corresponds to an NSC which requests (to the NSP) a dedicated network slice for the provision of PNI-NPN services. In this scenario, a portion of the network slice is deployed within the NSC premises (e.g., RAN, UPF) and the other portion (e.g., 5GC control plane functions) is hosted by one or more PLMN nodes. Unlike the baseline vertical customer, this new NSC does typically have (yet limited) telco knowledge, and wants to retain certain control over the allocated network slice. The capabilities offered to an advanced vertical customer might include (i) monitoring capabilities, i.e. the same capabilities offered to a baseline vertical customer and (ii) device configuration capabilities, i.e. provision of parameters for battery, mobility and communication patterns associated to the device, (iii) edge discovery/selection, e.g. in case the vertical wants to deploy workloads on the telco edge cloud.

- Hyperscaler: it corresponds to a NSC which requests (to the NSP) a dedicated network slice to provide a service-tailored connectivity pipe to a NSC’s customer. With some enterprises (i.e., NSC’s customer) starting to migrate workloads towards hyperscaler nodes, it is necessary for the hyperscaler (i.e., NSC) to provide SLA guarantees to these enterprises, especially for critical processes/services. However, the hyperscaler does not have network resources between its cloud nodes and customer premises, and therefore has to ask the mobile network operator (i.e., NSP) to set up a network slice between these endpoints. The capabilities offered to a hyperscaler might include (i) monitoring capabilities, i.e., the same capabilities offered to a baseline vertical customer; (ii) quality on demand, i.e. dynamic QoS and bandwidth management; (iii) policy control.

- Mobile (Virtual) Network Operator.

NOTE 1: In all the above cases, the NSP role is assumed to be played by an MNO.

NOTE 2: The capabilities mentioned above are neither exhaustive nor accurate, but examples to motivate the need for considering different NSC types.

The NSC types within the scope of FS_NSCE should be use case driven.

4.2.2	Issue #2: Types of capabilities available for exposure

When referring to the capabilities a NSC might be interested to consume, we have three big groups of capabilities that a NSP can make available for consumption:

- Application layer capabilities, within the scope of SA6.

- Management layer capabilities, within the scope of SA5.

- Network layer capabilities, within the scope of SA2.

There is the need to have one single exposure layer to make all the capabilities available for NSCs. This exposure layer should integrate network layer capabilities (@NEF, SA2), management layer capabilities (@MCEG, SA5), application layer capabilities (@SEAL, SA6), together with non-3GPP capabilities (e.g. cloud related capabilities). Otherwise, if every SA WG starts defining their own exposure fabric, (i) the likelyhood of encountering incompatibilities/duplicities across these WG specific solutions is high; (ii) the operators may come up with increased integration efforts, which ultimately may make their systems very hard to build and maintain. Which 3GPP working group is responsible to provide the single exposure layer is FFS.

4.2.3	Issue #3: EGMF/MCEG

The Exposure Governance Management Function (EGMF) was originally defined in TS 28.533 [2] as an MnF providing management capability exposure governance (MCEG). However, the current definition needs more elaboration on the following questions:

- The functional scope of management capability exposure governance, and its relationship with the access control and with existing API GW solutions in carrier networks.

- The impact of management capability exposure governance on the Network Slice NRM fragment. Is within the scope of SA5 or not? If in-scope, then:

  - what NSC related information (e.g., NSC id, NSC granted capabilities) does the NSP send to the NOP?

  - how does the NOP manage this information in relation to the existing NetworkSlice and NetworkSliceSubnet IOCs?

- The need to standardize EGMF. Does SA5 really need to define this MnF? Doesn’t this approach mean moving away from producer centric model of SA5, i.e., focus on service producers rather than MFs?

- If EGMF standardization is within the scope of SA5, then does SA5 need to provide details on EGMF internals? Does SA5 need to decide whether the EGMF is positioned on the Network Management Layer (NML), or the Service Management Layer (SML), or BSS layer?

4.2.4	Issue #4: NSC-NSP service interaction

The NSC-NSP service interactions work (i.e., APIs made available by the NSP, for consumption by the NSCs) is out of the 3GPP scope. Based on the proposal #2, which argues in favour of having one single exposure layer integrating 3GPP SA2/SA5/SA6 capabilities and non-3GPP capabilities, together with the fact that a high number of NSCs are not familiar with 3GPP models, it makes sense to let these interactions be covered in other industry fora. The potential group to provide the single exposure layer is FFS.

4.2.5	Issue #5: Relation to other SA5 work/study items

The work conducted in the FS_NSCE is related to other Rel-17 SI/WIs, including:

- MSAC (Management Service Access Control), on the access control aspects inherent to exposure to 3rd parties. NSP shall expose capabilities to NSCs in a controlled, secure and auditable way.

- OAM_NPN (Management of Non-Public Networks), when the network slice is used for the provisioning of a PNI-NPN. In this case, the modes 1b defined in [3] applies.

- eMEMTANE (Management of enhanced tenant concept), on the need to associate tenants to different NSCs, and manage the corresponding information in the NRM.

- 5GDMS (Discovery of management services in 5G), on the need for NSCs to discover capabilities available for consumption.

The work in FS_NSCE is to leverage outcomes from the Rel-17 study/work items which are listed above. It seems these study/work items provides most (if not all) the ingredients for the network slice capability exposure topic, so the mission of FS_NSCE should be to find out the recipe to combine them and provide overall exposure picture.

5	Use cases for network management capability exposure

5.1	Network slice management capability exposure

5.1.1	Description

A use case of network slice management capability exposure can be described as follows:

1. MNO A selects the MnS that can be exposed to Externals (e.g. verticals) and publishes the related MnS data in certain MnS producer that the BSS can access.

2. In order to enable the exposure of network slice related eMnS, a vertical A firstly makes a contract with the MNO A. The vertical A negotiates its specific requirements for the network slice management capability exposure with the MNO A. The negotiation can be done via the following ways:

a) For vertical A which is small enterprise, it can directly have a view on the network slice related management capability through the BSS (e.g. by using Service Catalog). Based on that, the vertical A can select the network slice related eMnSs which will be covered by the contract.

b) For vertical A which is large enterprise (i.e. Internet giants that have their own service customer), it can select the network slice related eMnSs that are available to be exposed offline (e.g. through a F2F meeting). The MNO A can proceed with the service ordering through BSS based on the contract.

3. The BSS may interact with the OSS in order to complete certain configuration (i.e. permission regarding what eMnS, optionally under what condition, can be exposed) regarding the exposure of eMnS based on the customized requirement from the eMnS consumer (e.g. vertical A).

4. The vertical A can get access to the network slice related management capability offered by eMnS producer within 3GPP management system. The access may need the interaction with BSS (e.g. through Service Catalog).

5.1.2	Issue and gaps

Gap:

An External needs to apply for the access of network slice management capability through BSS. However, there is no discussion and agreement on whether an eMnS is exposed transparently through the BSS or being processed through a dedicated exposure platform before exposing to the external.

The definition and the format of permission for the exposure of network slice related eMnS and its potential impact on internal interface with BSS is not discussed in current SA5 work.

Whether and how to publish eMnS which can be exposed to BSS to a suitable eMnS producer for network management capability exposure is not specified in existing 3GPP management system.

5.2	Exposure of MnS for monitoring QoS of video application

5.2.1	Description

A use case of exposure of MnS for monitoring QoS of video application can be described as follows:

1. A live concert with high-resolution video application service is provided with service provider A (i.e. associate to eMnS consumer). A local hosting network (e.g. SNPN) is available and provides localized services for high-resolution video service of the concert. The local hosting network is managed by the service provider A. The high-resolution video streaming service of the live concert is available to be accessed from both the local hosting network and PLMN. Service provider A and the provider of PLMN (i.e. associate to eMnS provider) are two different parties.

2. The service provider A of the local hosting network can have a contract with the Operator of PLMN before identifying the proper MnS for exposure through BSS (e.g. by using Service Catalog). The BSS may obtain the information of MnS that is allowed to be exposed using a proper MnS service for exposure provided by OSS. The contract may contain the agreement on the exposure of MnSs for accessing certain management MnS related to QoS. According to the contract, the service provider A can have the permission to use the MnS for accessing certain performance MnS related to QoS for the PLMN. The performance MnS can be related to NR and 5GC, e.g. Average DL UE throughput in gNB.

3. Once the contract is made, the corresponding MnF within the 3GPP management system is configured with permission rule through the interface between BSS and OSS. The permission rule defines that the provider of the local hosting network can have the right to access certain management MnS regarding QoS.

4. Through obtaining the measurement MnS using exposure capability, the provider of the local hosting network can determine the situation when PLMN can not support the high-resolution video service with satisfied QoS. In this case, the service provider A can notify its customers to get access to the local hosting network for improving the QoS of the high-resolution video application.

5.2.2	Issue and gaps

Gap:

The definition and the format of the permission rule for the network management capability exposure regarding NR and 5GC related performance MnS is not specified in existing 3GPP management system.

Whehter and how to publish MnS which can be exposed to externals to a suitable MnS service producer for the exposure of performance MnS regarding NR and 5GC is not specified in existing 3GPP management system.

5.3	eMnS discovery service

5.3.1	Description

A use case of eMnS discovery service is described as follows:

1. MNO A provides eMnSs from 3GPP management system for external customers and these eMnSs go through BSS for exposure.

2. In order to provide such discovery service to external customer, MNO A wishes to use an eMnS discovery service producer that takes responsibility for publishing eMnS data for discovery.

3. Once the BSS obtains the eMnS data from 3GPP management system, verticals can discover the eMnS using the API (e.g. Service Catalog) provided by BSS.

5.3.2	Issue and gaps

Gap:

Whether eMnS discovery service is provided by MnS discovery service producer or a dedicated eMnS discovery service producer (e.g. EGMF) is not specified in existing 3GPP management system.

Study is needed on whether the MnS data as defined in 28.533 can also be re-used for eMnS data, or if any extensions are necessary.

5.4	eMNS support to discovery systems

5.4.1	Description

An eMNS should be allowed to register to authorized supported discovery systems such that interested authorized consumers (within or external to the operator) are able to consume it. This implies that the eMnS is only exposed to discovery systems where a pre-existing contract allows for such an exposure.

1. An operator would like to register its eMNS to a trusted discovery system. The operator configures the eMNS with the discovery system and the appropriate level of exposure for the discovery system registration. The eMNS is provided with the appropriate address of the discovery system and register with it at the appropriate level of configured exposure for the discovery system.

2. The operator performs changes in its management system that influence the information exposed by the eMNS to the discovery system. The eMNS should in this case automatically “advertise” the updated information to appropriate set of discovery systems.

3. In case the relationship of the operator to this discovery system ends, which implies the system is no longer trusted. Then the eMNS automatically withdraws its registration in the discovery system.

5.4.2	Issue and gaps

There are several issues that need to be resolved.

Issues:

There is a difference if the discovery system is external or internal to the operator. An internal discovery system may still exist outside the scope of management.

There is an issue with managing which consumers have access to the discovery system and could theoretically consume the management service.

Gap:

To limit issues the exposure to a discovery system may only provide “read” permissions w.r.t the eMNS. To execute the eMNS the consumer still needs to be authorized with the management system. Therefore, there is a gap in the difference in exposure for consumption, and exposure for advertisement alone which needs to be solved.

5.5	Exposure of network slice as a product

5.5.1	Description

This use case involves the following roles:

- NSP: Network Slice Provider

- NSC: Network Slice Customer

- NOP: Network Operator

- CSC: Communication Service Customer

- CSP: Communication Service Provider

, and the following systems:

- BSS: Business Support System

- OSS: Operations Support System, made up of the two following sub-systems:

- SML: Service Management Layer

- NML: Network Management Layer (for sake of simplicity, network management and network element / function management are both in the NML).

5.5.1.1	Sub-use case 1: NSP and NOP played by the same organization

In this scenario, the following organizations play aforementioned roles as follows:

- Company-V plays the role of NSC

- Company-A plays the role of NSP and NOP

- As NSP, it has:

- a BSS, e.g. to manage its customers, products, contracts, and

- a SML, to manage the services that support its products

- As NOP, it has:

- its own 5G network (RAN + core). In this sub-use case, Company-A owns the whole set of network resources used by the service required to support the product ordered by Vertical V

- a NML, to manage the network resources used by services

Figure 5.5.1.1-1: Sub-use case 1 - NSP and NOP played by the same organization

Company-A product catalogue proposes the following product offerings:

- Network Slice eMBB with different flavours: Silver, Gold, Platinum

- Network Slice URLLC with different flavours: Silver, Gold, Platinum

Network Slice MIoT with different flavours: Silver, Gold, Platinum.

In this sub-use case 1:

1. Company-V (as the NSC) chooses a product from Company-A product offerings

2. Company-V sends a request to Company-A (as the NSP) to order the product ‘Network Slice eMBB Platinum’. To achieve this, a candidate API is TMF API 622 (Product Ordering)

2.1 Company-A BSS determines which service supports the product being ordered by Company-V and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering)

2.2 OSS / SML determines which network resources support the service being ordered and issues a request to the OSS / NML to allocate required network resources, e.g. network slice subnet(s), network functions, etc. To achieve this, candidate APIs are from 3GPP TS 28.531 and TS 28.532

2.3 OSS / NML allocates network resources required to support the service and informs OSS / SML back about the characteristics of the network resources being allocated

2.4 OSS / SML associates the allocated network resources to the service and informs its BSS back about the characteristics of the service supporting the product

3. Company-A (as the NSP) sends a reply to Company-V to inform that the product ordered is now available to Company-V.

NOTE: in this use case, aspects related to Transport Network(s) are not addressed as they are out of 3GPP scope.

5.5.1.2	Sub-use case 2: NOP role played simultaneously by different organizations

In this scenario, the following organizations play aforementioned roles as follows:

- Company-V plays the role of NSC

- Company-A plays the role of NSP and NOP

- As NSP, it has:

- a BSS, e.g. to manage its customers, products, contracts, and

- a SML, to manage the services that support its products

- As NOP, it has:

- its own 5G core network. In this sub-use case, Company-A owns the whole set of 5G core network resources used by the service required to support the product ordered by Vertical-V

- a NML, to manage the 5G core network resources used by services

As Company-A has no RAN in all requested areas, it relies on external organizations, namely Company-X and Company-Y, to provide RAN coverage in the US and in Spain respectively. Therefore:

- Company-A plays the role of Communication Service Customer (CSC) wrt. Company-X and Company-Y who both play the role of Communication Service Provider (CSP)

- Both Company-X and Company-Y have their own catalogue of products to offer RAN coverage in their respective countries

- Both Company-X and Company-Y play the role of CSP (for their respective product offerings) and NOP (for their respective RAN).

Figure 5.5.1.2-1: Sub-use case 2 - NOP role played simultaneously by different organizations

Company-A product catalogue proposes the following product offerings:

- Network Slice eMBB with different flavours: Silver, Gold, Platinum

- Network Slice URLLC with different flavours: Silver, Gold, Platinum

Network Slice MIoT with different flavours: Silver, Gold, Platinum.

In this sub-use case 2:

1. Company-V (as the NSC) chooses a product from Company-A product offerings

2. Company-V sends a request to Company-A (as the NSP) to order the product ‘Network Slice eMBB Platinum’. To achieve this, a candidate API is TMF API 622 (Product Ordering)

2.1 Company-A BSS determines which service supports the product being ordered by Company-V and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering)

2.2 Company-A OSS / SML determines which network resources support the service being ordered and:

2.2.1 based on its knowledge that required 5G core network resources are available internally, it issues a request to its own OSS / NML to allocate required 5G core network resources, e.g. network slice subnet(s) for its 5G core network, etc. To achieve this, candidate APIs are from 3GPP TS 28.531 and TS 28.532

2.2.2 Company-A OSS / NML allocates 5G core network resources required to support the service and informs OSS / SML about the characteristics of the network resources being allocated

2.2.3 based on its knowledge that required RAN resources are not available internally, it informs Company-A BSS about missing RAN resources

2.3 Company-A BSS:

2.3.1 acting as a CSC, issues a request to Company-X to order product X-1 (e.g. from the Wholesale offerings) to get RAN coverage in the US. To achieve this, a candidate API is TMF AP 622 (Product ordering). Company-X, as the CSP, receives the product order. Company-X BSS determines which service supports the product being ordered by Company-A and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering). Company-X OSS / SML determines which network resources support the service being ordered, etc. Once completed, Company-X BSS informs Company-A BSS that the product which has been ordered is now available to Company-A

2.3.2 acting as a CSC, issues a request to Company-Y to order product Y-1 (e.g. from the inter-operator network slice offerings) to get RAN coverage in Spain. To achieve this, a candidate API is TMF AP 622 (Product ordering). Company-Y, as the CSP, receives the product order. Company-Y BSS determines which service supports the product being ordered by Company-A and issues a request to its OSS/SML to order this service. This service can be e.g. a network slice. To achieve this, a candidate API is TMF API 641 (Service Ordering). Company-Y OSS / SML determines which network resources support the service being ordered, etc. Once completed, Company-Y BSS informs Company-A BSS that the product which has been ordered is now available to Company-A

2.3.3 informs its own OSS/SML that required RAN resources are available

2.4 Company-A OSS / SML associates the network resources allocated either internally or externally (by Company-X or Company-Y) to the service and informs its BSS about the characteristics of the service supporting the product

3. Company-A (as the NSP) sends a reply to Company-V to inform that the product ordered is now available to Vertical-V.

NOTE: in this use case, aspects related to Transport Network(s) are not addressed as they are out of 3GPP scope.

5.5.1.3	Exposure of the network slice as a product

In both sub-use case 1 and sub-use case 2, the characteristics of the network slice ordered by the NSC to the NSP are exposed by the NSP to the NSC at product-level, i.e. as specified by the product specification in the NSP catalogue. The product specification provides the characteristics of the product being offered by the NSP at business level and is not subject to standardization. These characteristics are generally more abstract than attributes defined in the 3GPP 5G NRM (cf. TS 28.541) and performance measurements defined in TS 28.552.

In these two sub-use cases, there is no exposure of service or network resources directly to the NSC.

6	Potential requirements for network management capability exposure

7	Possible solutions for network management capability exposure

7.1	Possible solution for “exposed MnS support to discovery systems”

The steps of the solution are as follows:

- A MnS consumer configures using the appropriate MnS (for example the generic provisioning service) the details of the external discovery service location and other supporting details (for e.g. authentication). Further, the detail on which parts of which MnS (component A)  IOCs and which instances of the corresponding MOI (component B) and corresponding data (component C) can be registered with the discovery service/system.  As an example, the operator may want to register her the ability to provision (Management object A) a particular slice type (NSSAI-ID) at a certain coverage area (coverageArea item 6.3.3 TS28.541) with some additional details (example: supported latency or maxNumberofUEs or delayTolerance) to an external discovery service or system.

In addition to the information of what is externally registered, information relating to where the appropriate management service to access the exposed MnS also needs to be provided. This could be default information based on the authentication of the operator in the exernal discovery system or in case of a trusted discovery the address of where to access the management service.

- The 3GPP Management System registers the exposed management component A, B and C as configured in step 1 to the appropriate external discovery service/system.

- Eventually, if any of the exposure details change – for example the same slice type can now be supported in a new coverageArea – the registration to the external discovery system may now need to be updated.

7.2	Possible solutions for eMnS discovery service

This clause provides potential solutions for the use case of eMnS discovery service (see clause 5.3). It is assumed that the MnS discovery service has been deployed.

To enable communication between eMnS consumers and eMnS producers, eMnS consumers need a mechanism to discover eMnS producers that are available in the 3GPP management system.

When the operator decides to expose a management service (eMnS), the operator must decide MnS(s) should be exposed, which internal MnS operations should be abstracted/filtered, and which internal MnS data should be abstracted/filtered. As part of this decision, the operator may use the MnS Discovery Service to collect information. The operator designs and implements an EGMF which is capable of exposing the eMnS. The EGMF publishes the eMnS data to the appropriate discovery service/system (e.g. eMnS discovery service producer).

The eMnS discovery service consumer sends a request to appropriate discovery service (e.g. eMnS discovery service producer) to obtain the eMnS data.

8	Conclusion and Recommendation

8.X	Issue #X

8.Y	Issue #Y

Annex A (informative):
Appendix with UML code of the sequence diagrams

A.1			UML code for Figure 4.1.4.2.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

actor NSC

participant BSS_NSP

participant OSS_SML

participant OSS_NML

participant Network

NSC --> BSS_NSP : product order

BSS_NSP --> BSS_NSP : process product order

BSS_NSP --> OSS_SML : service order(s)

OSS_SML --> OSS_SML : process service order(s)

OSS_SML --> OSS_NML : request(s) for management and orchestration\nof resources

OSS_NML --> OSS_NML : process request(s) for management and orchestration\nof resources

OSS_NML --> Network : management and orchestration\nof resources

OSS_NML <-- Network : management and orchestration\nof resources completed

OSS_SML <-- OSS_NML : request(s) for management and orchestration\nof resourcescompleted

BSS_NSP <-- OSS_SML : service order(s) completed

NSC <-- BSS_NSP : product order completed

@enduml

A.2			UML code for Figure 4.1.4.3.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

actor NSC

participant BSS_NSP

participant BSS_CSP #lightgrey

participant OSS_CSP #lightgrey

NSC --> BSS_NSP : product order

BSS_NSP --> BSS_NSP : process product order

BSS_NSP --> BSS_CSP : product order(s)

BSS_CSP --> BSS_CSP : process product order(s)

BSS_CSP --> OSS_CSP : service order(s)

OSS_CSP --> OSS_CSP : process service order(s)

BSS_CSP <-- OSS_CSP : service order(s) completed

BSS_NSP <-- BSS_CSP : product order(s) completed

NSC <-- BSS_NSP : product order completed

note right of OSS_CSP

BSS_NSP belongs to the company

fulfilling the NSP role.

BSS_CSP (grey) and OSS_CSP (grey) belong

to the company fulfilling the CSP role.

end note

@enduml

A.3			UML code for Figure 4.1.4.4.1

@startuml

skinparam sequence {

ArrowColor Black

ActorBorderColor Black

ActorBackgroundColor White

ParticipantBorderColor Black

ParticipantBackgroundColor White

LifeLineBorderColor Black

BackGroundColor <<BSS_Prov>> Black

}

skinparam NoteBackgroundColor White

skinparam NoteBorderColor Black

skinparam shadowing false

hide footbox

autonumber

actor NSC

participant BSS_NSP

participant OSS_NSP

participant BSS_CSP #lightgrey

participant OSS_CSP #lightgrey

NSC --> BSS_NSP : product order

BSS_NSP --> OSS_NSP : service order(s)

OSS_NSP --> OSS_NSP : process service order(s)

OSS_NSP --> OSS_CSP : service order(s)

OSS_CSP --> OSS_CSP : process service order(s)

OSS_CSP --> BSS_CSP : notify service order completed(s)

OSS_NSP <-- OSS_CSP : service order completed(s)

NSC <-- BSS_NSP : product order completed

note right of OSS_CSP

BSS_NSP and OSS_NSP belong

to the company fulfilling the NSP role.

BSS_CSP (grey) and OSS_CSP (grey) belong

To the company fulfilling the CSP role.

end note

@enduml

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-05 | SA5#137e | S5-213046 | - | - | - | 28.824 skeleton-0.0.0 | 0.0.0 |
| 2021-05 | SA5#137e | S5-213565 | - | - | - | pCR 28.824 External exposure via BSS | 0.1.0 |
| 2021-05 | SA5#137e | S5-213566 | - | - | - | pCR 28.824 Add Description of Concept and Roles to TR 28.824 | 0.1.0 |
| 2021-05 | SA5#137e | S5-213658 | - | - | - | pCR 28.824 Use case – network slice management capability exposure | 0.1.0 |
| 2021-05 | SA5#137e | S5-213564 | - | - | - | pCR 28.824 Use case – Exposure of MnS for monitoring QoS of video application | 0.1.0 |
| 2021-08 | SA5#138e | S5-214645 | - | - | - | pCR 28.824 Use case – eMnS discovery service | 0.2.0 |
| 2021-08 | SA5#138e | S5-214646 | - | - | - | pCR 28.824 eMNs discovery by other systems | 0.2.0 |
| 2021-08 | SA5#138e | S5-214128 | - | - | - | pCR 28.824 Improve description of roles | 0.2.0 |
| 2021-10 | SA5#139e | S5-215523 | - | - | - | pCR 28.824 Add use case for the exposure of network slice as a product | 0.3.0 |
| 2021-10 | SA5#139e | S5-215524 | - | - | - | pCR 28.824 Types of interfaces for the exposure of network slice | 0.3.0 |
| 2021-10 | SA5#139e | S5-215525 | - | - | - | pCR 28.824 Service management scenarios between CSC, CSP and NSP | 0.3.0 |
| 2021-10 | SA5#139e | S5-215526 | - | - | - | pCR 28.824 Key issues relative to network slice management capabilities exposure | 0.3.0 |
| 2021-10 | SA5#139e | S5-215578 | - | - | - | Possible solution to support eMnS discovery service | 0.3.0 |
| 2021-10 | SA5#139e | S5-215579 | - | - | - | Solution for external discovery | 0.3.0 |