| 3GPP TR 28.827 V1.0.0 (2022-02) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Charging management; Study on 5G charging for additional roaming scenarios and actors (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Business roles	9

5	Concepts and overview	9

5.1	General	9

5.2 	Home Routed	9

5.3	Local Breakout	10

5.4	Billing and Charging Evolution (BCE)	10

6	Architecture considerations	10

7	Charging scenarios and key issues	10

7.1	Charging in visited MNO for wholesale charging towards home MNO	10

7.1.1	Use cases	10

7.1.1.1	Use case #1a: Visited MNO charging home MNO for 5G data connectivity provided to the home MNO’s subscribers	10

7.1.1.2	Use case #1b: Visited MNO charging home MNO for 5G connection and mobility provided to the home MNO’s CSCs	11

7.1.2	Potential charging requirements	11

7.1.3	Key issues	11

7.1.4	Potential solutions	11

7.1.4.1	Solution #1.1: CDR in VPLMN for wholesale of 5G data connectivity	11

7.1.4.1.1	General	11

7.1.4.1.2	Reference architecture	12

7.1.4.1.3	Message flows	12

7.1.4.2	Solution #1.2: CDR in VPLMN for wholesale of 5G connection and mobility	12

7.1.4.2.1	General	12

7.1.4.2.2	Reference architecture	13

7.1.4.2.3	Message flows	13

7.1.4.3	Solution #1.3: Nchf to HPLMN for wholesale of 5G data connectivity	14

7.1.4.3.1	General	14

7.1.4.3.2	Reference architecture	15

7.1.4.3.3	Message flows	15

7.2	Convey charging information from visited MNO to home MNO	17

7.2.1	Home MNO does retail charging towards subscribers	17

7.2.1.1	Use cases	17

7.2.1.1.1	Use case #2a: Home MNO charging subscriber for 5G data connectivity at roaming using local breakout	17

7.2.1.1.2	Use case #2b: Home MNO charging subscriber for 5G data connectivity at roaming using home routed	17

7.2.2	Potential charging requirements	18

7.2.3	Key issues	18

7.2.4	Potential solutions	18

7.2.4.1 	Solution #2.1: V-CHF communicating with H-CHF for retail charging of 5G data connectivity	18

7.2.4.1.1	General	18

7.2.4.1.2	Reference architecture	18

7.2.4.1.3	Message flows	20

7.2.4.2 	Solution #2.2: Visited NF (CTF) communicating with both H-CHF and V-CHF	23

7.2.4.2.1	General	23

7.2.4.2.2	Reference architecture	23

7.2.4.2.3	Message flows	25

7.3	Charging in an MNO for the purpose of wholesale charging towards an additional actor	28

7.3.1	Use cases	28

7.3.1.1	Use case #3a: MNO does wholesale charging for 5G data connectivity towards additional actors	28

7.3.2	Potential charging requirements	28

7.3.3	Key issues	28

7.3.4	Potential solutions	29

7.3.4.1	Solution #3.1: CDR and Nchf in HPLMN for wholesale of non-roaming 5G data connectivity of additional actor	29

7.3.4.1.1	General	29

7.3.4.1.2	Reference architecture	29

7.3.4.1.3	Message flows	29

7.4	Convey charging from an MNO to an additional actor	30

7.4.1	Use cases	30

7.4.1.1	Use case #4a: Additional actor does retail charging for 5G data connectivity	30

7.4.1.2	Use case #4b: Additional actor does retail billing for 5G data connectivity	30

7.4.2	Potential charging requirements	30

7.4.3	Key issues	30

7.4.4	Potential solutions	31

7.4.4.1	Solution #4.1: Additional actor has CHF and does retail charging	31

7.4.4.1.1	General	31

7.4.4.1.2	Reference architecture	31

7.4.4.1.3	Potential charging flows	31

7.4.4.2	Solution #4.2: Additional actor only does retail billing of subscribers	32

7.4.4.2.1	General	32

7.4.4.2.2	Reference architecture	32

7.4.4.2.3	Potential charging flows	32

7.5	Convey charging from visited MNO to home MNO, and home MNO to an additional actor	33

7.5.1	Use cases	33

7.5.1.1	Use case #5a: Use case for additional actor does retail changing for 5G data connectivity while roaming	33

7.5.2	Potential charging requirements	33

7.5.3	Key issues	33

7.5.4	Potential solutions	34

7.5.4.1	Solution #5.1: CHF communication	34

7.5.4.1.1	General	34

7.5.4.1.2	Reference architecture	34

7.5.4.1.3	Message flows	34

7.6	Reconciliation of wholesale charging from the visited MNO with home MNO information	35

7.6.1	Use cases	35

7.6.1.1	Use case #6a: support for validation of usage information received from visited MNO at the home MNO	35

7.6.2	Potential charging requirements	35

7.6.3	Key issues	35

7.6.4	Potential solutions	35

7.6.4.1	Solution #6.1: Use home routing	35

7.6.4.1.1	General	35

8	Conclusions and recommendations	35

Annex A (informative): Change history	36

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

# 1	Scope

The present document studies the potential use cases, requirements, and solutions for the 5G charging for additional roaming scenarios and actors, in regards to roaming local break out and home routed and taking the requirements brought by adoption of GSMA BCE (Billing Charging Evolution), as a replacement for TAP.

The study will cover and identify gaps for:

-	charging in the visited MNO network for the purpose of wholesale charging towards the home MNO

-	capabilities for reconciliation of wholesale charging reports from the visited MNO network with MNO home network information

-	charging in the visited MNO network and convey it to the home MNO network for the purpose of retail charging

-	charging in the MNO’s network for the purpose of wholesale charging towards an additional charging model actor e.g. MVNO

-	charging in the visited MNO network and convey it to the home MNO’s network for the purpose of home MNO wholesale charging towards an additional charging model actor e.g. MVNO

-	charging in the visited MNO network and convey it to an MVNO hosted by a home MNO for the purpose of retail charging

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501:"System Architecture for the 5G System".

[3]	GSMA TD.201: "Common Billing and Charging Processes ".

[4]	3GPP TS 32.255:" Charging management;5G Data connectivity domain charging; stage 2".

[5]	3GPP TS 32.256:" Charging management; 5G connection and mobility domain charging; Stage 2".

[6]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

<defined term>: <definition>.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

BCE	Billing and Charging Evolution

CHF	Charging Function

HR	Home Routed

LBO	Local Breakout

# 4	Business roles

5G roaming scenarios involves the services or capabilities provided by multiple actors and entities in the form of following business roles:

-	Home Mobile Network Operator (home MNO): This is an operator of PLMN where the MCC and MNC of the PLMN identity match the MCC and MNC of the SUPI, provides related services for its subscribers.

-	Visited Mobile Network Operator (visited MNO): This is an operator of PLMN serving UE which different from the HPLMN,  also referred to as VPLMN

-	Mobile Virtual Network Operator (MVNO): This is an operator which provides mobile services but does not own licensed radio spectrum. The MVNO obtains network services from MNO, and provides to its own subscribers.

In deployments, there could be scenarios where one or more of the business roles are supported by a single enterprise, i.e., one enterprise may play a single business role or multiple business roles. The present document does not impose any restrictions to the possible deployment scenarios.

Editor’s Note: description of home MNO, visited MNO and MVNO (additional actor) is FFS.

There could be various business models in terms of charging for the network services provided and between the entities being served, for example (but not limited to):

-	Home MNO charges the subscribers for usage of network services in visited MNO.

-	Home MNO charges the MVNO for its subscribers’ network service usage in home MNO, or network service usage in visited MNO

-	MVNO charges the subscribers for usage of network services in visited MNO and/or home MNO.

-	Visited MNO charges home MNO for its subscribers’ usage of network services in visited MNO.

The solutions described in the present TR aim to flexibly support the charging scenarios without restriction to any specific business model.

# 5	Concepts and overview

## 5.1	General

The 5G charging for additional roaming scenarios and actors includes the following aspects:

-	HR Roaming (Home Routed), refer to TS 23.501 [2];

-	LBO Roaming (Local Breakout), refer to TS 23.501 [2];

-	BCE (Billing and Charging Evolution), refer to GSMA TD 201 [3].

## 5.2 	Home Routed

The 5G System roaming architecture in the case of home routed scenario with service-based interfaces within the Control Plane, as defined by TS 23.501[2] Figure 4.2.4-3.

The 5G System roaming architecture in the case of home routed scenario using the reference point representation, as defined by TS 23.501[2] Figure 4.2.4-6.

## 5.3	Local Breakout

The 5G System roaming architecture with local breakout with service-based interfaces within the Control Plane as defined by TS 23.501[2] Figure 4.2.4-1.

The 5G System roaming architecture in the case of local break out scenario using the reference point representation, as specified in TS 23.501 [2] Figure 4.2.4-4.

## 5.4	Billing and Charging Evolution (BCE)

The BCE process is a simplified and flexible optional settlement method, tailor made for future wholesale roaming settlement needs of operators. The process is driven by new formats like Usage Data Report, Billing Statement Report and Detailed Data Record which together build a robust framework for seamless revenue assurance.

The BCE Process as specified in GSMA TD.201 [3] clause 2.2.

# 6	Architecture considerations

Editor’s Note: the charging architecture for additional roaming scenarios and actors is FFS along with the related key issues.

# 7	Charging scenarios and key issues

## 7.1	Charging in visited MNO for wholesale charging towards home MNO

### 7.1.1	Use cases

#### 7.1.1.1	Use case #1a: Visited MNO charging home MNO for 5G data connectivity provided to the home MNO’s subscribers

This use case focuses on visited MNO and home MNO business roles.

A CSC (UE) has a subscription with the home MNO which allows usage of 5G data connectivity.

The visited MNO to home MNO charging could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	Rating Group

-	Service ID

-	Location

-	QoS information.

Editor’s Note: the list of the above aspects is ffs and should be check whether in the BCE.

This use case is applicable for the Home Routed Roaming and Local Breakout Roaming.

For the Home Routed Roaming using QBC, the visited MNO to home MNO charging could be based on "Roaming Charging Profile", which may be negotiated between the VPLMN and the HPLMN.

#### 7.1.1.2	Use case #1b: Visited MNO charging home MNO for 5G connection and mobility provided to the home MNO’s CSCs

This use case focuses on visited MNO and home MNO business roles.

A CSC (UE) has a subscription with the home MNO which allows usage of 5G data connectivity connection and mobility.

The visited MNO to home MNO charging is based number of home MNO’s UEs (i.e. unique SUPI) connected to the visited MNO’s network over a specific period of time.

### 7.1.2	Potential charging requirements

REQ-CH_VMNO-01: The charging mechanism in visited MNO should support charging based on the 5G data connectivity usage for each home MNO.

REQ-CH_VMNO-02: The charging mechanism in visited MNO should support collecting charging information related to 5G data connectivity usage for each home MNO.

REQ-CH_VMNO-03: The charging mechanism in visited MNO should support charging based on the 5G connection and mobility usage for each home MNO.

REQ-CH_VMNO-04: The charging mechanism in visited MNO should support collecting charging information related to 5G connection and mobility usage for each home MNO.

REQ-CH_VMNO-05: The charging mechanism in visited MNO may support roaming charging profile negotiation related to 5G data connectivity charging with each home MNO for Home Routed roaming case.

### 7.1.3	Key issues

The following key issues are identified:

-	Key Issue #1a: Aggregation of charging information in visited MNO for 5G data connectivity usage per home MNO.

-	Key Issue #1b: Aggregation of charging information in visited MNO for 5G connection and mobility usage per home MNO.

### 7.1.4	Potential solutions

#### 7.1.4.1	Solution #1.1: CDR in VPLMN for wholesale of 5G data connectivity

##### 7.1.4.1.1	General

A possible solution for key issue 1a, wholesale charging for 5G data connectivity provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

##### 7.1.4.1.2	Reference architecture

Figure 7.1.4.1.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.1.2-2: Roaming 5G data connectivity scenario in reference point representation

The SMF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may send this for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the amount of data per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.1.3	Message flows

For both home routed roaming and LBO, the flows for 5G data connectivity charging would be the same as in clause 5.2.2.12 of TS 32.255 [4] with the difference that for the LBO case would be that there would be no interaction with the home PLMN’s UPF, SMF and CHF.

#### 7.1.4.2	Solution #1.2: CDR in VPLMN for wholesale of 5G connection and mobility

##### 7.1.4.2.1	General

A possible solution for key issues 1b, wholesale charging for 5G connection and mobility (e.g. number of UEs) provided to the home MNO by the visited MNO, covering both home routed and local breakout case.

##### 7.1.4.2.2	Reference architecture

Figure 7.1.4.2.2-1: Roaming 5G connection and mobility scenario in service-based interface representation

Figure 7.1.4.2.2-2: Roaming 5G connection and mobility scenario in reference point representation

The AMF with CTF interact with CHF using Nchf interface. The CDRs produced by CHF may be send for interconnect charging (shown as dashed in the figures), which then can be used to aggregate and calculate the number of connected UEs per home MNO, the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.1.4.2.3	Message flows

The flows for 5G connection and mobility charging would be the same as in clause 5.2.2.5 of TS 32.256 [5], with the exception that the there is no need to interact with the home PLMN’s CHF and that only PEC is required. The CDRs generated in the CHF in VPLMN would in this case be used as input to the wholesale charging of the HPLMN. The following figure 7.1.4.2.2-1 describes a Registration charging message flow in PEC in VPLMN scenario for roaming, based on figure 4.2.2.2.2-1 of TS 23.502 [6] description

Figure 7.1.4.2.3-1: Roaming Registration – PEC in VPLMN

1-20a.	Registration procedure initiated by UE per steps 1 to 20a of figure 4.2.2.2.2-1 of TS 23.502 [6], with PCF, AUSF and UDM in HPLMN.

21.	Upon successful procedure, Registration Accept sent to the UE.

21ch-a. The AMF in VPLMN sends Charging Data Request [Event] to the CHF in VPLMN for the UE successful registration, indicating "roamer in".

21ch-b. The CHF in VPLMN creates the CDR for this registration.

21ch-c. The CHF in VPLMN acknowledges by sending Charging Data Response [Event] to the AMF in VPLMN.

21b to 24. Same steps as in figure 4.2.2.2.2-1 of TS 23.502 [6].

#### 7.1.4.3	Solution #1.3: Nchf to HPLMN for wholesale of 5G data connectivity

##### 7.1.4.3.1	General

A possible solution for key issue #1a, wholesale charging for 5G data connectivity provided to the home MNO by the visited MNO, covering local breakout case. In this case there is also charging information generated in the home MNO.

##### 7.1.4.3.2	Reference architecture

Figure 7.1.4.3.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.1.4.3.2-2: Roaming 5G data connectivity scenario in reference point representation

##### 7.1.4.3.3	Message flows

For local breakout roaming, the flows for 5G data connectivity charging are present as the following figure 7.1.4.3.3-1.

Figure 7.1.4.3.3-1: Local breakout roaming for 5G data connectivity charging

1-3a.	UE initiates a new PDU session. V-SMF selection by the AMF.

4ch-a. The UE is identified as a roamer (PLMN ID of the received SUPI is different from VPLMN PLMN ID), the V-CHF is selected accordingly. A Charging Data Request [Initial, Roaming Charging profile] is sent to V-CHF, indicating "in-bound roamer".

4ch-b. The V-CHF opens a CDR (indicating "in-bound roamer")

4ch-c. The V-CHF acknowledges by sending Charging Data Response [Initial, Roaming Charging profile] to the V-SMF.

7ch-a. A Charging Data Request [Update] is sent to V-CHF with charging information received from V-SMF.

7ch-b. The V-CHF updates the CDR.

7ch-c. The V-CHF acknowledges by sending Charging Data Response [Update] to the V-SMF.

10ch-a. A Charging Data Request [Termination] is sent to V-CHF.

10ch-b. The V-CHF closes the CDR

10ch-c. The V-CHF acknowledges by sending Charging Data Response [Termination] to the V-SMF.

Step 5, Step 8 and Step 11 are detailed in Figure 7.2.4.2.1-2 clause 7.2.

## 7.2	Convey charging information from visited MNO to home MNO

### 7.2.1	Home MNO does retail charging towards subscribers

#### 7.2.1.1	Use cases

##### 7.2.1.1.1	Use case #2a: Home MNO charging subscriber for 5G data connectivity at roaming using local breakout

This use case focuses on home MNO and subscriber business roles.

A subscriber has a UE and a subscription with the home MNO which allows usage of 5G data connectivity while roaming. The subscriber is roaming in a visited network and is allowed to use local breakout.

The home MNO charging of the subscriber could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	Visited network

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

##### 7.2.1.1.2	Use case #2b: Home MNO charging subscriber for 5G data connectivity at roaming using home routed

This use case focuses on home MNO and subscriber business roles.

A subscriber has a UE and a subscription with the home MNO which allows usage of 5G data connectivity while roaming. The subscriber is roaming in a visited network and the connection is home routed.

The home MNO charging of the subscriber could be based on data volume that the home MNO UE has used in the visited MNO’s network considering of the following aspects:

-	Visited network

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.2.2	Potential charging requirements

REQ-CH_CVTOH-01: The charging mechanism in visited MNO should support conveying charging information for the 5G data connectivity usage to the home MNO for each UE.

REQ-CH_ CVTOH-02: The charging mechanism in visited MNO should support collecting charging information related to 5G data connectivity usage for each UE.

### 7.2.3	Key issues

The following key issues are identified:

-	Key Issue #2a: Collection of charging mechanism in visited MNO about the 5G data connectivity usage.

-	Key Issue #2b: Conveying the charging information to the home MNO

- 	Key Issue #2c: Finding the correct CHF

-	Key Issue #2d: Service based interface to use between visited CHF and home CHF

-	Key Issue #2e: Trigger handling between visited CHF and home CHF

### 7.2.4	Potential solutions

#### 7.2.4.1 	Solution #2.1: V-CHF communicating with H-CHF for retail charging of 5G data connectivity

##### 7.2.4.1.1	General

A possible solution for key issues #2a and #2b, retail charging for 5G data connectivity provided to the home MNO’s users by the visited MNO, in the case of local breakout.

##### 7.2.4.1.2	Reference architecture

Figure 7.2.4.1.2-1: Roaming 5G data connectivity scenario in service-based interface representation

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

Figure 7.2.4.1.2-2: Roaming 5G data connectivity scenario in reference point representation

##### 7.2.4.1.3	Message flows

Figure 7.2.4.1.3-1 shows a scenario for session based charging (SCUR) with CHF to CHF communication.

Figure 7.2.4.1.3-1: SCUR - Session based charging with CHF to CHF communication

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by the vCHF before start.

2)	Units determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial, Quota requested]: The NF (CTF) sends the request to the vCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

4)	Determine hCHF: the vCHF determines that it need to interact with hCHF and which hCHF it should contact.

5)	Charging home request [Initial, Quota requested]: The vCHF sends the request to the hCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

6)	Account, Rating, Reservation control: the hCHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the hCHF performs the corresponding reservations.

7)	Open CDR: based on policies, the hCHF opens a CDR related to the service.

8)	Charging home response [Initial, Quota granted]: The hCHF grants authorization to vCHF for the service to start, with the reserved number of units.

9)	Open CDR: based on policies, the vCHF opens a CDR related to the service.

10)	Charging data response [Initial, Quota granted]: The vCHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

11)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

12)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

13)	Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

14)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the vCHF, for more units to be granted for the service to continue and reporting the used units.

15)	Charging home request [Update, Unit used, Quota requested]: the vCHF sends the request to the hCHF, for more units to be granted for the service to continue and reporting the used units.

16)	Account, Rating, Reservation control: The hCHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

17)	 Update CDR: based on policies, the hCHF updates the CDR with charging data related to the service.

18)	Charging home response [Update, Quota granted]: The hCHF grants quota to vCHF for the service to continue, with the reserved number of units.

19)	 Update CDR: based on policies, the vCHF updates the CDR with charging data related to the service.

20)	Charging data response [Update, Quota granted]: The vCHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

21)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

22)	Service delivery ongoing: the NF (CTF) continues the service delivery based on the granted quota.

23)	Service released: the service is terminated or released.

24)	Charging data request [Termination, Unit used]: the NF (CTF) sends the request to the vCHF, for charging data related to the service termination with the final consumed units.

25)	Charging home request [Termination, Unit used]: the vCHF sends the request to the hCHF, for charging data related to the service termination with the final consumed units.

26)	Account, Rating control: The hCHF performs the service termination process involving rating entity and user's account balance.

27)	 Close CDR: based on policies, the hCHF closes the CDR with charging data related to the service termination and the last reported units.

28)	Charging home response [Termination]: The hCHF informs the vCHF on the result of the request.

29)	 Close CDR: based on policies, the vCHF closes the CDR with charging data related to the service termination and the last reported units.

30)	Charging data response [Termination]: The vCHF informs the NF (CTF) on the result of the request.

#### 7.2.4.2 	Solution #2.2: Visited NF (CTF) communicating with both H-CHF and V-CHF

##### 7.2.4.2.1	General

A possible solution for key issues #2a and #2b, retail charging for 5G data connectivity provided to the home MNO’s user by the visited MNO. Roaming 5G data connectivity scenario in service-based interface representation is the same with Figure 7.2.4.1.1-1.

The visited CHF does converged charging for interconnect, while the home CHF does converged charging for the subscriber.

##### 7.2.4.2.2	Reference architecture

Figure 7.2.4.2.2-1: Roaming 5G data connectivity scenario in service-based interface representation

Figure 7.2.4.2.2-2: Roaming 5G data connectivity scenario in reference point representation

##### 7.2.4.2.3	Message flows

Figure 7.2.4.2.3-1 shows a scenario for session based charging (SCUR) with CTF to both CHFs communication.

Figure 7.2.4.2.3-1: SCUR - Session based charging with CTF to two CHFs communication

1)	Request for service delivery: A request for session establishment is received in the NF (CTF). The service is configured to be authorized by both the vCHF and hCHF before start.

2)	Units determination: the NF (CTF) determines the number of units depending on the service requested by the UE in "Decentralized Units determination" scenario.

3)	Charging data request [Initial]: The NF (CTF) sends the request to the vCHF for the service to be granted authorization to start.

4)	Open CDR: based on policies, the vCHF opens a CDR related to the service.

5)	Charging data response [Initial]: The vCHF grants authorization to NF (CTF) for the service to start.

6)	Charging data request [Initial, Quota requested]: The NF (CTF) sends the request to the hCHF for the service to be granted authorization to start, and to reserve the number of units if determined in item 2.

7)	Account, Rating, Reservation control: the hCHF rates the requests either based on the number of units requested or on internal unit determination, checks if corresponding funds can be reserved on the user's account balance. If the account has sufficient funds, the hCHF performs the corresponding reservations.

8)	Open CDR: based on policies, the hCHF opens a CDR related to the service.

9)	Charging data response [Initial, Quota granted]: The hCHF grants authorization to NF (CTF) for the service to start, with the reserved number of units.

10)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

11)	Start of service delivery: the NF (CTF) starts to deliver the content/service based on the reserved number of units.

12)	Quota management trigger: A trigger associated to quota management is met. Unit determination is performed when applicable.

13)	Charging data request [Update, Unit used]: the NF (CTF) sends the request to the vCHF, reporting the used units.

14)	 Update CDR: based on policies, the vCHF updates the CDR with charging data related to the service.

15)	Charging data response [Update]: The vCHF grants authorization to NF (CTF) for the service to continue.

16)	Charging data request [Update, Unit used, Quota requested]: the NF (CTF) sends the request to the hCHF, for more units to be granted for the service to continue and reporting the used units.

17)	Account, Rating, Reservation control: The hCHF performs the process related to the reported usage and the requested reservation, involving rating entity and user's account balance.

18)	 Update CDR: based on policies, the hCHF updates the CDR with charging data related to the service.

19)	Charging data response [Update, Quota granted]: The hCHF grants quota to NF (CTF) for the service to continue, with the reserved number of units.

20)	Granted units supervision: the NF (CTF) monitors the consumption of the granted units.

21)	Service delivery ongoing: the NF (CTF) continues the service delivery based on the granted quota.

22)	Service released: the service is terminated or released.

23)	Charging data request [Termination, Unit used]: the NF (CTF) sends the request to the vCHF, for charging data related to the service termination with the final consumed units.

24)	 Close CDR: based on policies, the vCHF closes the CDR with charging data related to the service termination and the last reported units.

25)	Charging data response [Termination]: The vCHF informs the NF (CTF) on the result of the request.

26)	Charging data request [Termination, Unit used]: the NF (CTF) sends the request to the hCHF, for charging data related to the service termination with the final consumed units.

27)	Account, Rating control: The hCHF performs the service termination process involving rating entity and user's account balance.

28)	 Close CDR: based on policies, the hCHF closes the CDR with charging data related to the service termination and the last reported units.

29)	Charging data response [Termination]: The hCHF informs the NF (CTF) on the result of the request.

## 7.3	Charging in an MNO for the purpose of wholesale charging towards an additional actor

#### 7.3.1	Use cases

##### 7.3.1.1	Use case #3a: MNO does wholesale charging for 5G data connectivity towards additional actors

This use case focuses on MNO and MVNO business roles.

A subscriber has a UE and a subscription with the MVNO which allows usage of 5G data connectivity while in the home MNO.

The MNO to MVNO charging could be based on data volume that the MVNO UE has used in the MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.3.2	Potential charging requirements

REQ-CH_MNOMVNO-01: The charging mechanism in MNO should support collecting charging information related to 5G data connectivity usage for each MVNO.

REQ-CH_MNOMVNO-02: The charging mechanism in MNO should support collecting charging information related to 5G connection and mobility usage for each MVNO.

### 7.3.3	Key issues

The following key issues are identified:

-	Key Issue #3a: Collection of charging mechanism in home MNO about the 5G data connectivity usage per MVNO.

### 7.3.4	Potential solutions

#### 7.3.4.1	Solution #3.1: CDR and Nchf in HPLMN for wholesale of non-roaming 5G data connectivity of additional actor

##### 7.3.4.1.1	General

A possible solution for key issue #3a, charging for 5G data connectivity provided to the additional actor by the home MNO, covering non-roaming case.

##### 7.3.4.1.2	Reference architecture

Figure 7.3.4.2-1: Additional actor 5G data connectivity scenario in service-based interface representation

Figure 7.3.4.1.2-2: Additional actor 5G data connectivity scenario in reference point representation

The SMF with CTF interact with CHF using Nchf interface, without any requirement of quota management. The CCS produces CDRs which may be send for wholesale charging which then can be used to aggregate and calculate the amount of data per additional actor, it could be done based on SUPI, IMSI series, SNSSAI(s), DNN(s) etc., the interconnect part is only included for completeness and is currently outside the scope of 3GPP SA5.

##### 7.3.4.1.3	Message flows

The flows would be the same as the non-roaming cases in clause 5.2.2 of TS 32.255 [4].

## 7.4	Convey charging from an MNO to an additional actor

### 7.4.1	Use cases

#### 7.4.1.1	Use case #4a: Additional actor does retail charging for 5G data connectivity

This use case focuses on MVNO and subscriber business roles.

A subscriber has a UE and a subscription with the MVNO which allows usage of 5G data connectivity. The MVNO has their own billing and charging (CHF), but no other NFs.

The MVNO charging of the subscriber could be based on data volume that the UE has used in the MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

#### 7.4.1.2	Use case #4b: Additional actor does retail billing for 5G data connectivity

This use case focuses on additional actor (e.g., MVNO) and subscriber business roles.

A subscriber has a UE and a subscription with the additional actor which allows usage of 5G data connectivity. The additional actor has their own billing, but no CHFs.

The additional actor charging of the subscriber could be based on data volume that the UE has used in the home MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.4.2	Potential charging requirements

REQ-CH_CHTOA-01: The charging mechanism in MNO should support conveying charging information for the 5G data connectivity usage to the MVNO for each UE.

REQ-CH_ CHTOA-02: The charging mechanism in MNO should support collecting charging information related to 5G data connectivity usage for each UE.

### 7.4.3	Key issues

The following key issues are identified:

-	Key Issue #4a: Collection of charging mechanism in MNO about the 5G data connectivity usage.

-	Key Issue #4b: Conveying the charging information to the MVNO

- 	Key Issue #4c: Finding the additional actor’s CHF

-	Key Issue #4d: Service based interface to use towards the additional actor’s CHF

-	Key Issue #4e: Trigger handling between towards the additional actor’s CHF

### 7.4.4	Potential solutions

#### 7.4.4.1	Solution #4.1: Additional actor has CHF and does retail charging

##### 7.4.4.1.1	General

The possible solutions for key issues #4a and #4b, retail charging for 5G data connectivity provided by the additional actor, to the MVNO by the home MNO.

##### 7.4.4.1.2	Reference architecture

Figure 7.4.4.1.2-1: Additional actor with only CHF  5G data connectivity scenario in service-based interface representation

Figure 7.4.4.1.2-2: Additional actor with only CHF  5G data connectivity scenario in service-based interface representation

The MNO CHF interacts with the additional actor CHF does converged charging for the subscriber.

##### 7.4.4.1.3	Potential charging flows

The flows would be the same as in clause 7.2.4.2 where the vCHF corresponds to the home MNO’s CHF and the hCHF corresponds to the additional actor’s CHF.

#### 7.4.4.2	Solution #4.2: Additional actor only does retail billing of subscribers

##### 7.4.4.2.1	General

The possible solutions for key issues #4a and #4b, retail billing for 5G data connectivity by the additional actor, the 5G data connectivity is provided by the home MNO.

##### 7.4.4.2.2	Reference architecture

Figure 7.4.4.2.2-1: Additional actor with only billing 5G data connectivity scenario in service-based interface representation

Figure 7.4.4.2.2-2: Additional actor with only billing 5G data connectivity scenario in service-based interface representation

The home MNO CCS produces CDRs that the additional actor uses for billing of the subscriber.

##### 7.4.4.2.3	Potential charging flows

The flows would be the same as in clause 7.1.4.1.2 where the vCHF corresponds to the home MNO’s CHF and the interconnect would be replaced by sending information for retail charging.

## 7.5	Convey charging from visited MNO to home MNO, and home MNO to an additional actor

### 7.5.1	Use cases

#### 7.5.1.1	Use case #5a: Use case for additional actor does retail changing for 5G data connectivity while roaming

This use case focuses on visited MNO, home MNO, MVNO and subscriber business roles.

A subscriber has a UE and a subscription with the MVNO which allows usage of 5G data connectivity. The MVNO has their own billing and charging (CHF), but no other NFs. The MVNO has an agreement with the home MNO that allows its subscribers roaming and the home MNO as a roaming agreement with the visited MNO.

The MVNO charging of the subscriber could be based on data volume that the UE has used in the visited MNO’s network considering of the following aspects:

-	RAT type;

-	S-NSSAI;

-	DNN;

-	QoS information.

### 7.5.2	Potential charging requirements

REQ-CH_CVTOHTOA-01: The charging mechanism in visited MNO should support conveying charging information for the 5G data connectivity usage to home MNO, allowing further conveying it to the MVNO for each UE.

REQ-CH_ CVTOHTOA-02: The charging mechanism in home MNO should support conveying charging information for the 5G data connectivity usage to MVNO for each UE.

REQ-CH_ CVTOHTOA-03: The charging mechanism in MVNO should support receiving charging information related to 5G data connectivity usage for each UE.

### 7.5.3	Key issues

The following key issues are identified:

-	Key Issue #5a: Collection of charging mechanism in visited MNO about the 5G data connectivity usage.

-	Key Issue #5b: Conveying the charging information from visited to the home MNO

-	Key Issue #5c: Conveying the charging information to the home MNO and MVNO

- 	Key Issue #5d: Finding the correct CHF in home MNO and MVNO

-	Key Issue #5e: Service based interface to use between visited CHF, home CHF and MVNO

-	Key Issue #5f: Trigger handling between visited CHF, home CHF and MVNO

### 7.5.4	Potential solutions

#### 7.5.4.1	Solution #5.1: CHF communication

##### 7.5.4.1.1	General

A possible solution for key issue #5a, #5b, and #5c, charging for 5G data connectivity provided to the additional actor by the visited MNO through the home MNO.

##### 7.5.4.1.2	Reference architecture

Figure 7.5.4.1.2-1: Additional actor with only CHF 5G data connectivity roaming scenario in service-based interface representation

Figure 7.5.4.1.2-2: Additional actor with only CHF 5G data connectivity roaming scenario in reference point representation

The visited and home MNO CHF does converged charging for interconnect, while the additional actor CHF does converged charging for the subscriber.

##### 7.5.4.1.3	Message flows

This would be a combination of the flows in clauses 7.2.4.1 and 7.4.4.1.

## 7.6	Reconciliation of wholesale charging from the visited MNO with home MNO information

### 7.6.1	Use cases

#### 7.6.1.1	Use case #6a: support for validation of usage information received from visited MNO at the home MNO

This use case focuses on visited MNO and home MNO business roles.

When the home MNO receives the interconnect charges it would like to have the possibility to validate the received information with information controlled by itself.

### 7.6.2	Potential charging requirements

REQ-CH_RECON-01: The charging mechanism should allow the home MNO to monitor the usage without any involvement of the visited MNO.

### 7.6.3	Key issues

The following key issues are identified:

-	Key Issue #6a: Collection of usage of home MNO independent and without control of the visited MNO.

### 7.6.4	Potential solutions

#### 7.6.4.1	Solution #6.1: Use home routing

##### 7.6.4.1.1	General

A possible solution for key issue #6a would be to use home routed, where the connection is sent to the home MNO, see TS 32.255 [4].

# 8	Conclusions and recommendations

Local Breakout has been present in the standards for a long time but a solution for charging has been missing until now. The deployment of Network Slices, Edge Computing and different use cases that need local traffic routing have created the necessity of addressing all the gaps for deployments of local breakout. In this TR some use cases have been covered and some solutions provided.

Considering the different solutions to the use cases, the solutions where the SMF communicates with both the vCHF and hCHF at the same point in time i.e., solutions #2.2 (Visited NF (CTF) communicating with both H-CHF and V-CHF) and #4.1 (Additional actor has CHF and does retail charging), are preferred. This since these are the only solution that can be included in the time frame of Rel-17 and that covers the needed requirements. The focus for the specified solution will be to limit and minimize the impact on and to the CHF and SMF.

Editor’s note: Other solutions may be added depending on further study.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-09 | SA5#138e | S5-214431 |  |  |  | Initial skeleton (v0.0.0) of TR 28.827 | 0.0.0 |
| 2021-09 | SA5#138e | S5-214433 |  |  |  | Study document structure | 0.1.0 |
| 2021-09 | SA5#138e | S5-214434 |  |  |  | Addition of business roles and concepts | 0.1.0 |
| 2021-09 | SA5#138e | S5-214436 |  |  |  | Use case for visited MNO charging home MNO for 5G connection and mobility | 0.1.0 |
| 2021-09 | SA5#138e | S5-214437 |  |  |  | Use case for visited MNO charging home MNO for 5G data connectivity | 0.1.0 |
| 2021-09 | SA5#138e | S5-214703 |  |  |  | Addition of introduction and scope | 0.1.0 |
| 2021-09 | SA5#138e | S5-214716 |  |  |  | Use case for home MNO charging subscriber for 5G data connectivity | 0.1.0 |
| 2021-10 | SA5#139e | S5-215244 |  |  |  | New possible solution for use case #2a | 0.2.0 |
| 2021-10 | SA5#139e | S5-215471 |  |  |  | Use case for additional actor does retail charging for home UE | 0.2.0 |
| 2021-10 | SA5#139e | S5-215472 |  |  |  | Use case for additional actor does retail charging for roaming UE | 0.2.0 |
| 2021-10 | SA5#139e | S5-215447 |  |  |  | Use case for home MNO does wholesale charging towards additional actor | 0.2.0 |
| 2021-10 | SA5#139e | S5-215588 |  |  |  | Clarify the Business Role | 0.2.0 |
| 2021-10 | SA5#139e | S5-215589 |  |  |  | Addition of the LBO,HR and BCE in Concept and Overview | 0.2.0 |
| 2021-10 | SA5#139e | S5-215596 |  |  |  | Clarification on the use cases and key issues for Topic 7.1 | 0.2.0 |
| 2021-10 | SA5#139e | S5-215597 |  |  |  | Addition of the LBO message flows for Topic 7.1 | 0.2.0 |
| 2021-10 | SA5#139e | S5-215598 |  |  |  | Addition the new solution in the Topic 7.2 | 0.2.0 |
| 2021-11 | SA5#140e | S5-216521 |  |  |  | Correction on the potential charging architectures | 0.3.0 |
| 2021-11 | SA5#140e | S5-216526 |  |  |  | Adding reference point architecture to clause 7.1.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216527 |  |  |  | Adding SBI architecture to clause 7.2.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216528 |  |  |  | Adding reference point architecture to clause 7.3.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216529 |  |  |  | Adding reference point architecture to clause 7.4.4 | 0.3.0 |
| 2021-11 | SA5#140e | S5-216530 |  |  |  | New use case MVNO without CHF | 0.3.0 |
| 2021-11 | SA5#140e | S5-216531 |  |  |  | New potential solution MVNO without CHF | 0.3.0 |
| 2021-11 | SA5#140e | S5-216532 |  |  |  | Adding reference point architecture to clause 7.5.4 | 0.3.0 |
| 2021-02 | SA5#141e | S5-221122 |  |  |  | Editorial Correction on the Figures | 1.0.0 |
| 2021-02 | SA5#141e | S5-221322 |  |  |  | Corrections to clause 7.2 | 1.0.0 |
| 2021-02 | SA5#141e | S5-221323 |  |  |  | Corrections to clause 7.3 | 1.0.0 |
| 2021-02 | SA5#141e | S5-221324 |  |  |  | Corrections to clause 7.4 | 1.0.0 |
| 2021-02 | SA5#141e | S5-221656 |  |  |  | Adding Conclusions | 1.0.0 |
| 2021-02 | SA5#141e | S5-221679 |  |  |  | Adding use case and solution for clause 7.6 | 1.0.0 |
| 2021-02 | SA5#141e | S5-221681 |  |  |  | Corrections to clause 7.1 | 1.0.0 |
| 2021-02 | SA5#141e | S5-221732 |  |  |  | Clarification on the Roaming Charging Profile | 1.0.0 |