| 3GPP TS 29.515 V16.0.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Gateway Mobile Location Services; Stage 3 (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

| 3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org |  |
| --- | --- |
| Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association |  |

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	7

3.3	Abbreviations	7

4	Overview	7

5	Services offered by the GMLC	8

5.1	Introduction	8

5.2	Ngmlc_Location Service	8

5.2.1	Service Description	8

5.2.2	Service Operations	8

5.2.2.1	Introduction	8

5.2.2.2	ProvideLocation	9

5.2.2.2.1	General	9

5.2.2.3	LocationUpdate	9

5.2.2.3.1	General	9

5.2.2.4	CancelLocation	10

5.2.2.4.1	General	10

5.2.2.5	EventNotify	10

5.2.2.5.1	General	10

5.2.2.6	LocationUpdateNotify	11

5.2.2.6.1	General	11

6	API Definitions	11

6.1	Mgmlc_Location Service API	11

6.1.1	Introduction	11

6.1.2	Usage of HTTP	12

6.1.2.1	General	12

6.1.2.2	HTTP standard headers	12

6.1.2.2.1	General	12

6.1.2.2.2	Content type	12

6.1.2.3	HTTP custom headers	12

6.1.2.3.1	General	12

6.1.3	Custom Operations without associated resources	12

6.1.3.1	Overview	12

6.1.3.2	Operation: provide-location	13

6.1.3.2.1	Description	13

6.1.3.2.2	Operation Definition	13

6.1.3.3	Operation: cancel-location	14

6.1.3.3.1	Description	14

6.1.3.3.2	Operation Definition	14

6.1.3.4	Operation: location-update	15

6.1.3.4.1	Description	15

6.1.3.4.2	Operation Definition	15

6.1.4	Notifications	15

6.1.4.1	General	15

6.1.4.2	Eventnotify	15

6.1.4.2.1	Description	15

6.1.4.2.2	Notification Definition	15

6.1.4.2.3	Notification Standard Methods	15

6.1.4.2.3.1	POST	15

6.1.4.3	LocationUpdateNotify	16

6.1.4.3.1	Description	16

6.1.4.3.2	Notification Definition	16

6.1.4.3.3	Notification Standard Methods	16

6.1.4.3.3.1	POST	16

6.1.5	Data Model	16

6.1.5.1	General	16

6.1.5.2	Structured data types	18

6.1.5.2.1	Introduction	18

6.1.5.2.2	Type: InputData	19

6.1.5.2.3	Type: LocationData	20

6.1.5.2.4	Type: CancelLocData	20

6.1.5.2.5	Type: LocUpdateData	21

6.1.5.2.6	Type: EventNotifyData	22

6.1.5.2.7	Type: UePrivacyReqirements	22

6.1.5.2.8	Type: UePrivacyCallSessionUnrelatedClass	23

6.1.5.2.9	Type: LocUpdateNotification	23

6.1.5.3	Simple data types and enumerations	23

6.1.5.3.1	Introduction	23

6.1.5.3.2	Simple data types	23

6.1. 5.3.3	Enumeration: PseudonymIndicator	24

6.1.5.3.4	Enumeration: LocationRequestType	24

6.1.5.3.5	Enumeration: LocationTypeRequested	24

6.1.5.3.6	Enumeration: EventNotifyDataType	24

6.1.6	Error Handling	25

6.1.6.1	General	25

6.1.6.2	Protocol Errors	25

6.1.6.3	Application Errors	25

6.1.7	Feature negotiation	25

6.1.8	Security	25

Annex A (normative):	OpenAPI specification	27

A.1	General	27

A.2	Ngmlc_Location API	27

Annex B (informative):	Change history	34

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Ngmlc Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the Nglmc.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [5] and 3GPP TS 29.501 [6].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.273: "5G System Location Services (LCS)".

[5]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[6]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[7]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[8]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[9]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[10]	IETF RFC 7807: "Problem Details for HTTP APIs".

[11]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[12]	3GPP TS 29.572: "5G System; Location Management Services; Stage 3".

[13]	ITU Recommendation E.164: "The international public telecommunication numbering plan".

[14]	3GPP TS 29.503: "5G System; Unified Data Management Services; Stage 3".

[15]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[16]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[17]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[18]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Symbols

Void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

GMLC	Gateway Mobile Location Centre

# 4	Overview

The Gateway Mobile Location Centre (GMLC) is the network entity in the 5G Core Network (5GC) supporting Location Services (LCS). Within the 5GC, the GMLC offers services to the AMF, GMLC and NEF via the Ngmlc service based interface (see 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]).

Figure 4-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the GMLC:

Figure 4-1: Reference model – GMLC

The functionalities supported by the GMLC are listed in clause 4.3.3 of 3GPP TS 23.273 [4].

# 5	Services offered by the GMLC

## 5.1	Introduction

The table 5.1-1 shows the GMLC Services and GMLC Service Operations:

Table 5.1-1: List of GMLC Services

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Ngmlc_Location | ProvideLocation | Request/Response | GMLC, NEF |
|  | LocationUpdate | Request/Response | AMF, GMLC |
|  | LocationUpdateNotify | Notify | NEF |
|  | CancelLocation | Request/Response | GMLC, NEF |
|  | EventNotify | Notify | GMLC, NEF |

Table 5.1-2 summarizes the corresponding APIs defined for this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Ngmlc_Location | 6.1 | Ngmlc Location Service | TS29515_Ngmlc_Location.yaml | ngmlc-loc | A.2 |

## 5.2	Ngmlc_Location Service

### 5.2.1	Service Description

The Ngmlc_Location service enables an NF to request location determination (current geodetic and optionally civic location) for a target UE. The following are the key functionalities of this NF service.

-	Allow the consumer NF to request the current geodetic and optionally civic location of a target UE.

-	Allow the consumer NF to subscribe/unsubscribe the geodetic and optionally civic location of a target UE for some certain events.

-	Allow the consumer NF to cancel an on-going periodic or triggered location request of a target UE.

-	Allow the consumer NF to get notified about the geodetic and optionally civic location of a target UE when some certain events are detected.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Ngmlc_Location services are as follows:

-	ProvideLocation

-	LocationUpdate

-	LocationUpdateNotify

-	CancelLocation

-	EventNotify

#### 5.2.2.2	ProvideLocation

##### 5.2.2.2.1	General

The service operation is used during the procedures:

-	5GC-MT-LR Procedure for the commercial location service (see 3GPP TS 23.273 [4], clause 6.1.2)

-	Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events (see 3GPP TS 23.273 [4], clause 6.3.1)

The ProvideLocation service operation is invoked by a NF Service Consumer, e.g. a NEF or GMLC, towards the GMLC to request to provide the location information (geodetic location and, optionally, civic location) for a target UE or to subscribe to periodic or triggered deferred location for a target UE. See Figure 5.2.2.2.1-1..

Figure 5.2.2.2.1-1: ProvideLocation Request/Response

1.	The NF Service Consumer shall send an HTTP POST request to the URI associated with the "provide-location" custom operation. The input parameters for the request (required QoS, supported GAD shapes, LCS client type, external  Service Identity, Codeword, service coverage, LDR type, serving AMF address,  LDR reference, H-GMLC Callback URI) should be included in the HTTP POST request body.

2a.	On success, "200 OK" shall be returned. The response body shall contain the parameters related to the determined position of the UE if any (geodetic position, civic location, positioning methods…).

2b	On failure, one of the HTTP status code listed in Table 6.1.3.2.2-2 may be returned. For a 4xx/5xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.3.2.2-2.

#### 5.2.2.3	LocationUpdate

##### 5.2.2.3.1	General

The service operation is used during the procedure:

-	5GC-MO-LR Procedure (see 3GPP TS 23.273 [4], clause 6.2)

The LocationUpdate enables the NF consumer (e.g. AMF) to update UE location information towards the GMLC. See Figure 5.2.2.3.1-1.

Figure 5.2.2.3.1-1: LocationUpdate Request/Response

1.	The NF Service Consumer shall send an HTTP POST request to the URI associated with the "location-update" custom operation. The request body shall contain a LocUpdateData object..

2a.	On success, "204 No content" shall be returned by the GMLC.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.4.2-2 may be returned. For a 4xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.4.2-2.

#### 5.2.2.4	CancelLocation

##### 5.2.2.4.1	General

The service operation is used during the procedure:

-	Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events (see 3GPP TS 23.273 [4], clause 6.3.3)

The CancelLocation enables the consumer NF to use the service operation to cancel a deferred 5GC-MT-LR procedure for periodic or triggered location. See Figure 5.2.2.4.1-1.

Figure 5.2.2.4.1-1: CancelLocation Request/Response

1.	The NF Service Consumer shall send an HTTP POST request to the URI associated with the "cancel-location" custom operation. The input parameters for the request ((H-)GMLC contact address, LDR reference number, LMF identification, serving AMF address) should be included in the HTTP POST request body.

2a.	On success, "204 No Content" shall be returned.

2b.	On failure, one of the HTTP status code listed in Table 6.1.3.3.2-2 may be returned. For a 4xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.3.3.2-2.

#### 5.2.2.5	EventNotify

##### 5.2.2.5.1	General

The service operation is used during the procedure:

-	Deferred 5GC-MT-LR Procedure for Periodic, Triggered and UE Available Location Events (see 3GPP TS 23.273 [4], clause 6.3.1 or clause 6.3.2)

The EventNotify enables the consumer NF to get notified about the geodetic and optionally civic location for a target UE when some certain events are detected. See Figure 5.2.2.5.1-1.

Figure 5.2.2.5.1-1: EventNotify Notification

1.	The GMLC shall send an HTTP POST to the callback URI to send a notification. The input parameters for the notification (Notification Correlation ID, UE (SUPI and if available GPSI), Type of location related event (e.g. deferred location for the UE available event, activation of location for periodic or triggered location, mobility of a target UE to a  new AMF or MME for a deferred location, Geodetic Location, Civic Location, Position Methods Used, serving LMF identification) should be included in the HTTP POST request body.

2a.	If the notification is received, the NF Service Consumer shall reply with the status code 204 indicating the notification is received, in the response message.

2b.	On failure, one of the HTTP status code listed in Table 6.1.4.2.3.1-2 may be returned. For a 4xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.4.2.3.1-2.

#### 5.2.2.6	LocationUpdateNotify

##### 5.2.2.6.1	General

The service operation is used during the procedure:

-	5GC-MO-LR Procedure (see 3GPP TS 23.273 [4], clause 6.2)

The LocationUpdateNotify enables the NF consumer (e.g. NEF) to get notified about the UE location information update. See Figure 5.2.2.6.1-1.

Figure 5.2.2.6.1-1: LocationUpdateNotify Notification

1.	The GMLC shall send an HTTP POST request to the callback URI of the NF consumer (e.g. NEF). The response body shall contain a LocUpdateNotification object.

The callback URI (e.g. NEF address for callback) is locally configured on GMLC or discovered via NRF.

2a.	On success, "204 No content" shall be returned by the NF consumer.

2b.	On failure, one of the HTTP status code listed in Table 6.1.4.3.3.1-2 may be returned. For a 4xx response, the message body may contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.4.3.3.1-2.

# 6	API Definitions

## 6.1	Mgmlc_Location Service API

### 6.1.1	Introduction

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [6], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [6].

-	The <apiName> shall be "ngmlc-loc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, as defined in IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

HTTP/2, as defined in IETF RFC 7540 [8], shall be used as specified in clause 5 of 3GPP TS 29.500 [5].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [5].

HTTP messages and bodies for the Ngmlc_Location service shall comply with the OpenAPI [7] specification contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [9], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [5].

-	The Problem Details JSON Object (IETF RFC 7807 [10]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

#### 6.1.2.3	HTTP custom headers

##### 6.1.2.3.1	General

The following HTTP custom headers shall be supported:

-	3gpp-Sbi-Message-Priority: See 3GPP TS 29.500 [5], clause 5.2.3.2.2.

This API does not define any new HTTP custom headers.

### 6.1.3	Custom Operations without associated resources

#### 6.1.3.1	Overview

The structure of the custom operation URIs of the Ngmlc_Location service is shown in Figure 6.1.3.1-1.

Figure 6.1.3.1-1: Custom operation URI structure of the Ngmlc_Location API

Table 6.1.3.1-1 provides an overview of the custom operations and applicable HTTP methods.

Table 6.1.3.1-1: Custom operations

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/ngmlc-loc/<apiVersion>/provide-location | POST | Request or Subscribe the geodetic and optionally civic location of a target UE |
| {apiRoot}/ngmlc-loc/<apiVersion>/cancel-location | POST | Cancel an on-going periodic or triggered location request of a target UE |
| {apiRoot}/ngmlc-loc/<apiVersion>/location-update | POST | Enable the UE to update UE location information towards the consumer NF |

#### 6.1.3.2	Operation: provide-location

##### 6.1.3.2.1	Description

This clause will describe the custom operation and what it is used for, and the custom operations URI.

##### 6.1.3.2.2	Operation Definition

The operation shall support the response data structures and response codes specified in tables 6.1.3.2.2-1 and 6.1.3.2.2-2.

Table 6.1.3.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InputData | M | 1 | Input parameters to the "Provide-Location" operation |

Table 6.1.3.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationData | M | 1 | 200 OK | This case represents the successful retrieval of the location of the UE or successful subscription of periodic or triggered location of the UE. Upon success, a response body is returned containing the different parameters of the location data if obtained, such as: - Geographic Area - Civic Location - Age of Location - Accuracy of Location - Positioning methods |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - POSITIONING_DENIED - UNSPECIFIED - UNSUPPORTED_BY_UE See table 6.1.6.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 500 Internal Server Error | The "cause" attribute may be used to indicate the following application error: - POSITIONING_FAILED See table 6.1.6.3-1 for the description of these errors. |
| ProblemDetails | O | 0..1 | 504 Gateway Timeout | The "cause" attribute may be used to indicate the following application error: - UNREACHABLE_USER See table 6.1.6.3-1 for the description of this error. |

#### 6.1.3.3	Operation: cancel-location

##### 6.1.3.3.1	Description

This clause will describe the custom operation and what it is used for, and the custom operation's URI.

##### 6.1.3.3.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 6.1.3.3.2-1 and table 6.1.3.3.2-2.

Table 6.1.3.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CancelLocData | M | 1 | The information is used to cancel location. |

Table 6.1.3.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful cancellation of location. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - UNSPECIFIED - LOCATION_SESSION_UNKNOWN See table 6.1.6.3-1 for the description of this error. |

#### 6.1.3.4	Operation: location-update

##### 6.1.3.4.1	Description

This clause will describe the custom operation and what it is used for, and the custom operation's URI.

##### 6.1.3.4.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 6.1.3.4.2-1 and table 6.1.3.4.2-2.

Table 6.1.3.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocUpdateData | M | 1 | Input parameters to the "location-update" operation |

Table 6.1.3.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful update of location. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - UNSPECIFIED - UNREQUESTED_BY_UE - UNKOWN_EXTERNAL_CLIENT_OR_AF - UNREACHABLE_EXTERNAL_CLIENT_OR_AF See table 6.1.6.3-1 for the description of this error. |

### 6.1.4	Notifications

#### 6.1.4.1	General

#### 6.1.4.2	Eventnotify

##### 6.1.4.2.1	Description

The EventNotify operation is used to the occurrence of periodic or triggered location event for a target UE to a consumer NF (e.g. GMLC).

##### 6.1.4.2.2	Notification Definition

Call-back URI: {hgmlcCallBackUri}

Call-back URI is provided by NF Service Consumer during requesting the subscription of the periodic or triggered deferred location for a target UE. See clause 5.3.2.2 for the description of how the GMLC obtains the Call-back URI of the NF Service Consumer.

##### 6.1.4.2.3	Notification Standard Methods

###### 6.1.4.2.3.1	POST

This method sends a Location event notify to the NF Service Consumer.

This method shall support the request and response data structures and response codes specified in table 6.1.4.2.3.1-1 and table 6.1.4.2.3.1-2.

Table 6.1.4.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventNotifyData | M | 1 | Input parameters to the "Event Notify" operation |

Table 6.1.4.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful notification of the event. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - UNSPECIFIED - LOCATION_SESSION_UNKNOWN See table 6.1.6.3-1 for the description of this error. |

#### 6.1.4.3	LocationUpdateNotify

##### 6.1.4.3.1	Description

The LocationUpdateNotify operation is used to deliver the location update for a UE to a consumer NF (e.g. NEF).

##### 6.1.4.3.2	Notification Definition

Call-back URI:	{locationUpdateCallbackUri}

##### 6.1.4.3.3	Notification Standard Methods

###### 6.1.4.3.3.1	POST

This method sends a Location update notification to the NF Service Consumer.

This method shall support the request and response data structures and response codes specified in table 6.1.4.3.3.1-1 and table 6.1.4.3.3.1-2.

Table 6.1.4.3.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocUpdateNotification | M | 1 | Input parameters to the "LocationUpdateNotification" operation |

Table 6.1.4.3.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful notification of the event. |
| ProblemDetails | O | 0..1 | 403 Forbidden | The "cause" attribute may be used to indicate one of the following application errors: - UNSPECIFIED - UNKOWN_EXTERNAL_CLIENT_OR_AF - UNREACHABLE_EXTERNAL_CLIENT_OR_AF See table 6.1.6.3-1 for the description of this error. |

### 6.1.5	Data Model

#### 6.1.5.1	General

This clause specifies the application data model supported by the API.

Table 6.1.5.1-1 specifies the data types defined for the Ngmlc_Location service based interface protocol.

Table 6.1.5.1-1: Ngmlc_Location specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| InputData | 6.1.5.2.2 | the input parameters in ProvideLocation service operation |  |
| LocationData | 6.1.5.2.3 | the response parameters in ProvideLocation service operation |  |
| CancelLocData | 6.1.5.2.4 | the input parameters in CancelLocation service operation |  |
| LocUpdateData | 6.1.5.2.5 | the input parameters in LocationUpdate service operation |  |
| EventNotifyData | 6.1.5.2.6 | the input parameters in EventNotify Notification service operation |  |
| UEPrivacyRequirements | 6.1.5.2.7 | UE privacy requirements from LCS client |  |
| UEPrivacyCallSessionUnrelatedClass | 6.1.5.2.8 | UE privacy Call/Session unrelated class |  |
| LocUpdateNotification | 6.1.5.2.9 | Location Update Notification |  |
| PseudonymOfUE | 6.1.5.3.2 | pseudonym of UE |  |
| ServiceIdentity | 6.1.5.3.2 | service identity |  |
| CodeWord | 6.1.5.3.2 | codeword |  |
| ExternalClientIdentification | 6.1.5.3.2 | external client identification |  |
| E164CountryCodeOfGeographicArea | 6.1.5.3.2 | E.164 country codes for geographic areas |  |
| PseudonymIndicator | 6.1.5.3.3 | It defines if a pseudonym is requested |  |
| LocationRequestType | 6.1.5.3.4 | NI-LR, MT-LR or MO-LR |  |
| LocationTypeRequested | 6.1.5.3.5 | the location type requested by the LCS client |  |
| EventNotifyDataType | 6.1.5.3.6 | the type of event that triggers event notification |  |

Table 6.1.5.1-2 specifies data types re-used by the Ngmlc_Location service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Ngmlc_Location service based interface.

Table 6.1.5.1-2: Ngmlc_Location re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| Gpsi | 3GPP TS 29.571 [11] |  |  |
| Supi | 3GPP TS 29.571 [11] |  |  |
| Uri | 3GPP TS 29.571 [11] |  |  |
| AmfId | 3GPP TS 29.571 [11] |  |  |
| NfInstanceId | 3GPP TS 29.571 [11] |  |  |
| ExternalClientType | 3GPP TS 29.572 [12] |  |  |
| LocationQoS | 3GPP TS 29.572 [12] |  |  |
| SupportedGADShapes | 3GPP TS 29.572 [12] |  |  |
| PeriodicEventInfo | 3GPP TS 29.572 [12] |  |  |
| AreaEventInfo | 3GPP TS 29.572 [12] |  |  |
| MotionEventInfo | 3GPP TS 29.572 [12] |  |  |
| LdrType | 3GPP TS 29.572 [12] |  |  |
| LdrReference | 3GPP TS 29.572 [12] |  |  |
| AgeOfLocationEstimate | 3GPP TS 29.572 [12] |  |  |
| PositioningMethod | 3GPP TS 29.572 [12] |  |  |
| AccuracyFulfilmentIndicator | 3GPP TS 29.572 [12] |  |  |
| LmfIdentification | 3GPP TS 29.572 [12] |  |  |
| LcsServiceType | 3GPP TS 29.572 [12] |  |  |
| VelocityRequested | 3GPP TS 29.572 [12] |  |  |
| LcsPriority | 3GPP TS 29.572 [12] |  |  |
| VelocityEstimate | 3GPP TS 29.572 [12] |  |  |
| TerminationCause | 3GPP TS 29.572 [12] |  |  |
| PositioningMethodAndUsage | 3GPP TS 29.572 [12] |  |  |
| GnssPositioningMethodAndUsage | 3GPP TS 29.572 [12] |  |  |
| Lpi | 3GPP TS 29.503 [14] |  |  |
| LocationPrivacyInd | 3GPP TS 29.503 [14] |  |  |
| ValidTimePeriod | 3GPP TS 29.503 [14] |  |  |
| PlmnOperatorClass | 3GPP TS 29.503 [14] |  |  |
| LcsServiceAuth | 3GPP TS 29.571 [11] |  |  |

#### 6.1.5.2	Structured data types

##### 6.1.5.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.5.2.2	Type: InputData

Table 6.1.5.2.2-1: Definition of type InputData

| Attribute name | Data type | P | Cardinality | Description | Applicability |  |
| --- | --- | --- | --- | --- | --- | --- |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identifier |  |  |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |  |
| pseudonymOfUE | PseudonymOfUE | O | 0..1 | pseudonym of the target UE |  |  |
| externalClientType | ExternalClientType | M | 1 | external client type |  |  |
| locationQoS | LocationQoS | O | 0..1 | requested location QoS |  |  |
| supportedGADShapes | array(SupportedGADShapes) | O | 1..N | supported Geographical Area Description shapes |  |  |
| serviceIdentity | ServiceIdentity | O | 0..1 | service identity |  |  |
| serviceCoverage | array(E164CountryCodeOfGeographicArea) | O | 1..N | a list of E.164 country codes for geographic areas [13] where the LCS client is permitted to request and receive UE location information. |  |  |
| ldrType | LdrType | C | 0..1 | location deferred request event type |  |  |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | periodic event information of the location request for a target UE |  |  |
| areaEventInfo | AreaEventInfo | C | 0..1 | area event information of the location request for a target UE |  |  |
| motionEventInfo | MotionEventInfo | C | 0..1 | motion event information of the location request for a target UE |  |  |
| ldrReference | LdrReference | C | 0..1 | notification correlation ID |  |  |
| hgmlcCallBackUri | Uri | O | 0..1 | notification target address |  |
| externalClientIdentification | ExternalClientIdentification | O | 0..1 | external LCS client identification |  |  |
| afId | string | O | 0..1 | the identification of AF that initiated location request |  |
| uePrivacyReqirements | UePrivacyReqirements | O | 0..1 | UE privacy requirement |  |
| lcsServiceType | LcsServiceType | O | 0..1 | the LCS service type |  |  |
| velocityRequested | VelocityRequested | O | 0..1 | velocity of the target UE is requested |  |  |
| priority | LcsPriority | O | 0..1 | priority of the location request |  |  |
| locationTypeRequested | LocationTypeRequested | O | 0..1 | Requested type of location, applicable to location immediate request |  |  |
| maximumAgeOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | requested maximum age of the location estimate |  |  |
| amfid | AmfId | O | 0..1 | the identification of serving AMF |  |  |

##### 6.1.5.2.3	Type: LocationData

Table 6.1.5.2.3-1: Definition of type LocationData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| pseudonymOfUE | PseudonymOfUE | O | 0..1 | pseudonym of the target UE |  |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identitfier |  |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| locationEstimate | GeographicArea | O | 0..1 | geographic area of the target UE |  |
| civicAddress | CivicAddress | O | 0..1 | civic address of the target UE |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | age of location estimate |  |
| positioningDataList | array(PositioningMethodAndUsage) | O | 1..N | If present, this IE shall indicate the usage of each non-GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| gnssPositioningDataList | array(GnssPositioningMethodAndUsage) | O | 1..N | If present, this IE shall indicate the usage of each GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| accuracyFulfilmentIndicator | AccuracyFulfilmentIndicator | O | 0..1 | the indication whether the obtained location estimate satisfies the requested accuracy or not |  |
| ueVelocity | VelocityEstimate | O | 0..1 | responded UE velocity, if requested and available |  |

##### 6.1.5.2.4	Type: CancelLocData

Table 6.1.5.2.4-1: Definition of type CancelLocData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription identifier |  |
| pseudonymOfUE | PseudonymOfUE | O | 0..1 | pseudonym of the target UE |  |
| hgmlcCallBackUri | Uri | M | 1 | notification target address |  |
| ldrReference | LdrReference | M | 1 | notification correlation ID |  |
| lmfIdentification | LmfIdentification | O | 0..1 | the latest LMF identification received |  |
| amfid | AmfId | O | 0..1 | the identification of the serving AMF |  |

##### 6.1.5.2.5	Type: LocUpdateData

Table 6.1.5.2.5-1: Definition of type LocUpdateData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription identitfier |  |
| pseudonymIndicator | PseudonymIndicator | O | 0..1 | pseudonym indicator |  |
| locationRequestType | LocationRequestType | M | 1 | event causing the location estimate (5GC-MO-LR) |  |
| locationEstimate | GeographicArea | M | 1 | geographic area of the target UE |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | M | 1 | age of location estimate |  |
| accuracyFulfilmentIndicator | AccuracyFulfilmentIndicator | M | 1 | the indication whether the obtained location estimate satisfies the requested accuracy or not |  |
| civicAddress | CivicAddress | O | 0..1 | civic address of the target UE |  |
| locationQoS | LocationQoS | M | 1 | the QoS requested by the target UE |  |
| externalClientIdentification | ExternalClientIdentification | O | 0..1 | identity of the LCS client |  |
| afId | string | O | 0..1 | identity of the AF |  |
| hgmlcAddress | Uri | O | 0..1 | the address of H-GMLC |  |
| serviceIdentity | ServiceIdentity | O | 0..1 | service Identity specified by the UE |  |

##### 6.1.5.2.6	Type: EventNotifyData

Table 6.1.5.2.6-1: Definition of type EventNotifyData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| Supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| Gpsi | Gpsi | O | 0..1 | Generic Public Subscription Identitfier |  |
| ldrReference | LdrReference | M | 0..1 | notification correlation ID |  |
| eventNotifyDataType | EventNotifyDataType | M | 0..1 | the type of event that triggers event notification |  |
| locationEstimate | GeographicArea | O | 0..1 | geographic area of the target UE |  |
| civicAddress | civicAddress | O | 0..1 | civic address of the target UE |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | age of location estimate |  |
| positioningDataList | array(PositioningMethodAndUsage) | O | 1..N | If present, this IE shall indicate the usage of each non-GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| gnssPositioningDataList | array(GnssPositioningMethodAndUsage) | O | 1..N | If present, this IE shall indicate the usage of each GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |  |
| lmfIdentification | LmfIdentification | O | 0..1 | LMF identification that stores the location context of the target UE |  |
| amfId | AmfId | O | 0..1 | the identification of AMF that is serving the target UE |  |
| terminationCause | TerminationCause | C | 0..1 | the IE shall be included if event reporting has been terminated |  |

##### 6.1.5.2.7	Type: UePrivacyReqirements

Table 6.1.5.2.7-1: Definition of type UePrivacyReqirements

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lpi | Lpi | O | 0..1 | When present, it shall contain the Location Privacy Indication. If absent, it indicates that the location for the UE is allowed. |  |
| uePrivacyCallSessionUnrelatedClass | UePrivacyCallSessionUnrelatedClass | O | 0..1 | When present, it shall contain Call/Session unrelated Class information for the user. |  |
| plmnOperatorClass | PlmnOperatorClass | O | 0..1 | When present, it shall contain PLMN Operator Class information for the user. |  |

##### 6.1.5.2.8	Type: UePrivacyCallSessionUnrelatedClass

Table 6.1.5.2.8-1: Definition of type UePrivacyCallSessionUnrelatedClass

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| lcsServiceAuthInfo | LcsServiceAuth | O | 0..1 | When present, this IE shall contain an indication of privacy related notification or verification for the target UE. The default value of this parameter if not presents is "LOCATION_ALLOWED_WITHOUT_NOTIFICATION". |  |
| locationValidTimePeriod | ValidTimePeriod | O | 0..1 | a valid time period for the positioning |  |
| codeWord | CodeWord | O | 0..1 | codeword |  |

##### 6.1.5.2.9	Type: LocUpdateNotification

Table 6.1.5.2.9-1: Definition of type LocUpdateNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supi | Supi | O | 0..1 | Subscription Permanent Identifier |  |
| gpsi | Gpsi | O | 0..1 | Generic Public Subscription identitfier |  |
| locationRequestType | LocationRequestType | M | 1 | event causing the location estimate (5GC-MO-LR) |  |
| locationEstimate | GeographicArea | M | 1 | geographic area of the target UE |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | M | 1 | age of location estimate |  |
| accuracyFulfilmentIndicator | AccuracyFulfilmentIndicator | M | 1 | the indication whether the obtained location estimate satisfies the requested accuracy or not |  |
| civicAddress | CivicAddress | O | 0..1 | civic address of the target UE |  |
| locationQoS | LocationQoS | M | 1 | the QoS requested by the target UE |  |
| afId | string | O | 0..1 | identity of the AF |  |

Editor's Note:	Whether "PseudonymOfUE" and "Service Identity" will be used by AF or not is FFS.

#### 6.1.5.3	Simple data types and enumerations

##### 6.1.5.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.5.3.2	Simple data types

The simple data types defined in table 6.1.5.3.2-1 shall be supported.

Table 6.1.5.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |
| ServiceIdentity | string | service identity |  |
| ExternalClientIdentification | string | external LCS client identification |  |
| CodeWord | string | codeword |  |
| E164CountryCodeOfGeographicArea | string | the combination of one, two or three digits identifying a specific country, countries in an integrated numbering plan, or a specific geographic area |  |
| PseudonymOfUE | string | pseudonym of UE |  |

##### 6.1. 5.3.3	Enumeration: PseudonymIndicator

The enumeration PseudonymIndicator represents whether pseudonym should be used as the identity of the target UE. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.5.3.3-1: Enumeration PseudonymIndicator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "PSEUDONYM_REQUESTED" | a pseudonym is requested |  |
| "PSEUDONYM_NOT_REQUESTED" | a pseudonym is not requested |  |

##### 6.1.5.3.4	Enumeration: LocationRequestType

The enumeration LocationRequestType represents how the location request is triggered. It shall comply with the provisions defined in table 6.1.5.3.4-1.

Table 6.1.5.3.4-1: Enumeration LocationRequestType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "NI-LR" | network induced location request |  |
| "MT-LR" | mobile terminated location request |  |
| "MO-LR" | mobile originated location request |  |

##### 6.1.5.3.5	Enumeration: LocationTypeRequested

The enumeration LocationTypeRequested represents the requested type of location which is only applicable to location immediate request. It shall comply with the provisions defined in table 6.1.5.3.5-1.

Table 6.1.5.3.5-1: Enumeration LocationTypeRequested

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "CURRENT_LOCATION" | requesting the current location of the target UE |  |
| "CURRENT_OR_LAST_KNOWN_LOCATION" | requesting the current or last known location of the target UE |  |
| "INITIAL_LOCATION" | requesting the initial location of the target UE |  |
| "NOTIFICATION_VERIFICATION_ONLY" | requesting notification verification only |  |

##### 6.1.5.3.6	Enumeration: EventNotifyDataType

The enumeration EventNotifyDataType represents the type of event notification. It shall comply with the provisions defined in table 6.1.5.3.6-1.

Table 6.1.5.3.6-1: Enumeration EventNotifyDataType

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "UE_AVAILABLE" | UE available event |  |
| "PERIODIC" | periodic event |  |
| "ENTERING_INTO_AREA" | entering area event |  |
| "LEAVING_FROM_AREA" | leaving area event |  |
| "BEING_INSIDE_AREA" | being inside area event |  |
| "MOTION" | motion event |  |
| "MAXIMUM_INTERVAL_EXPIRATION_EVENT" | expiration of maximum reporting interval event |  |
| "LOCATION_CANCELLATION_EVENT" | cancellation of location reporting event |  |
| "ACTIVATION_OF_DEFERRED_LOCATION" | a confirmation that periodic or triggered location was successfully activated in the target UE |  |

### 6.1.6	Error Handling

#### 6.1.6.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [5].

#### 6.1.6.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [5].

#### 6.1.6.3	Application Errors

The application errors defined for the Ngmlc_Location service are listed in Table 6.1.6.3-1.

Table 6.1.6.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| POSITIONING_DENIED | 403 Forbidden | the positioning procedure was denied. |
| UNSPECIFIED | 403 Forbidden | the request is rejected due to unspecified reasons. |
| UNSUPPORTED_BY_UE | 403 Forbidden | the position request for periodic or triggered location is not supported by the target UE |
| LOCATION_SESSION_UNKNOWN | 403 Forbidden | the location context was not found |
| UNREQUESTED_BY_UE | 403 Forbidden | the UE did not request transfer of its location to an LCS Client or AF |
| UNKOWN_EXTERNAL_CLIENT_OR_AF | 403 Forbidden | the external LCS client or AF is unknown |
| UNREACHABLE_EXTERNAL_CLIENT_OR_AF | 403 Forbidden | the external LCS client or AF is unreachable |
| POSITIONING_FAILED | 500 Internal Server Error | the positioning procedure failed |
| UNREACHABLE_USER | 504 Gateway Timeout | the user could not be reached in order to perform positioning procedure |

### 6.1.7	Feature negotiation

The optional features in table 6.1.7-1 are defined for the Ngmlc_Location API.

Table 6.1.7-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

### 6.1.8	Security

As indicated in 3GPP TS 33.501 [15], the access to the Ngmlc_Location API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [16]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [17]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Ngmlc_Location API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [17], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Ngmlc_Location service.

The Ngmlc_Location API defines scopes for OAuth2 authorization as specified in 3GPP TS 33.501 [15]; it defines a single scope consisting on the name of the service (i.e., "ngmlc-loc"), and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

## A.2	Ngmlc_Location API

openapi: 3.0.0

info:

version: '1.0.0.alpha-2'

title: Ngmlc_Location

description: |

Ngmlc_Location Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.515 V16.0.0; 5G System; Gateway Mobile Location Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.515/'

servers:

- url: '{apiRoot}/nglmc-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- ngmlc-loc

paths:

/provide-location:

post:

summary: Request Location of an UE

operationId: RequestLocation

tags:

- Request Location

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/InputData'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LocationData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

EventNotify:

'{$request.body#/hgmlcCallBackUri}':

post:

requestBody:

description: UE Event Notification

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotifyData'

responses:

'204':

description: Expected response to a valid notification

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/cancel-location:

post:

summary: request cancellation of periodic or triggered location

operationId: CancelLocation

tags:

- Cancel Location

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/CancelLocData'

required: true

responses:

'204':

description: Expected response to a successful cancellation

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/location-update:

post:

summary: update UE location information

operationId: UpdateLocation

tags:

- Update Location

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LocUpdateData'

required: true

responses:

'204':

description: Expected response to successful location context transfer

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

ngmlc-loc: Access to the Ngmlc_Location API

callbacks:

LocationUpdateNotify:

'{locationUpdateCallbackUri}': # The {locationUpdateCallbackUri} is configured in GMLC or discovered via NRF

post:

requestBody:

description: Location Update Notification

content:

application/json:

schema:

$ref: '#/components/schemas/LocUpdateNotification'

responses:

'204':

description: Expected response to a valid notification

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

schemas:

#

# COMPLEX TYPES

#

InputData:

type: object

required:

- externalClientType

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pseudonymOfUE:

$ref: '#/components/schemas/PseudonymOfUE'

externalClientType:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/ExternalClientType'

locationQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/SupportedGADShapes'

minItems: 1

serviceIdentity:

$ref: '#/components/schemas/ServiceIdentity'

serviceCoverage:

type: array

items:

$ref: '#/components/schemas/E164CountryCodeOfGeographicArea'

minItems: 1

ldrType:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrType'

periodicEventInfo:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/MotionEventInfo'

ldrReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

hgmlcCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

externalClientIdentification:

$ref: '#/components/schemas/ExternalClientIdentification'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

uePrivacyReqirements:

$ref: '#/components/schemas/UePrivacyReqirements'

lcsServiceType:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LcsServiceType'

velocityRequested:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/VelocityRequested'

priority:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LcsPriority'

locationTypeRequested:

$ref: '#/components/schemas/LocationTypeRequested'

maximumAgeOfLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AgeOfLocationEstimate'

amfid:

$ref: 'TS29571_CommonData.yaml#/components/schemas/AmfId'

LocationData:

type: object

properties:

pseudonymOfUE:

$ref: '#/components/schemas/PseudonymOfUE'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

locationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

civicAddress:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

ageOfLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AgeOfLocationEstimate'

positioningDataList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

accuracyFulfilmentIndicator:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AccuracyFulfilmentIndicator'

ueVelocity:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/VelocityEstimate'

CancelLocData:

type: object

required:

- hgmlcCallBackUri

- ldrReference

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pseudonymOfUE:

$ref: '#/components/schemas/PseudonymOfUE'

hgmlcCallBackUri:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

lmfIdentification:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LMFIdentification'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

LocUpdateData:

type: object

required:

- locationRequestType

- locationEstimate

- ageOfLocationEstimate

- accuracyFulfilmentIndicator

- locationQoS

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pseudonymOfUE:

$ref: '#/components/schemas/PseudonymOfUE'

locationRequestType:

$ref: '#/components/schemas/LocationRequestType'

locationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AgeOfLocationEstimate'

accuracyFulfilmentIndicator:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AccuracyFulfilmentIndicator'

civicAddress:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

locationQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

externalClientIdentification:

$ref: '#/components/schemas/ExternalClientIdentification'

afId:

type: string

hgmlcAddress:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

serviceIdentity:

$ref: '#/components/schemas/ServiceIdentity'

EventNotifyData:

type: object

required:

- eventNotifyDataType

- ldrReference

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

ldrReference:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LdrReference'

eventNotifyDataType:

$ref: '#/components/schemas/EventNotifyDataType'

locationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

civicAddress:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

ageOfLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AgeOfLocationEstimate'

positioningDataList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

lmfIdentification:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LMFIdentification'

afId:

type: string

terminationCause:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/TerminationCause'

UePrivacyReqirements:

type: object

properties:

lpi:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/Lpi'

uePrivacyCallSessionUnrelatedClass:

$ref: '#/components/schemas/UePrivacyCallSessionUnrelatedClass'

plmnOperatorClass:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/PlmnOperatorClass'

UePrivacyCallSessionUnrelatedClass:

type: object

properties:

lcsServiceAuthInfo:

$ref: 'TS29571_CommonData.yaml#/components/schemas/LcsServiceAuth'

locationValidTimePeriod:

$ref: 'TS29503_Nudm_SDM.yaml#/components/schemas/ValidTimePeriod'

codeWord:

$ref: '#/components/schemas/CodeWord'

LocUpdateNotification:

type: object

required:

- locationRequestType

- locationEstimate

- ageOfLocationEstimate

- accuracyFulfilmentIndicator

- locationQoS

properties:

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

locationRequestType:

$ref: '#/components/schemas/LocationRequestType'

locationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AgeOfLocationEstimate'

accuracyFulfilmentIndicator:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/AccuracyFulfilmentIndicator'

civicAddress:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/CivicAddress'

locationQoS:

$ref: 'TS29572_Nlmf_Location.yaml#/components/schemas/LocationQoS'

afId:

type: string

#

# SIMPLE TYPES

#

ServiceIdentity:

type: string

ExternalClientIdentification:

type: string

CodeWord:

type: string

E164CountryCodeOfGeographicArea:

type: string

PseudonymOfUE:

type: string

#

# ENUMS

#

PseudonymIndicator:

anyOf:

- type: string

enum:

- PSEUDONYM_REQUESTED

- PSEUDONYM_NOT_REQUESTED

- type: string

LocationRequestType:

anyOf:

- type: string

enum:

- NI-LR

- MT-LR

- MO-LR

- type: string

LocationTypeRequested:

anyOf:

- type: string

enum:

- CURRENT_LOCATION

- CURRENT_OR_LAST_KNOWN_LOCATION

- INITIAL_LOCATION

- NOTIFICATION_VERIFICATION_ONLY

- type: string

EventNotifyDataType:

anyOf:

- type: string

enum:

- UE_AVAILABLE

- PERIODIC

- ENTERING_INTO_AREA

- LEAVING_FROM_AREA

- BEING_INSIDE_AREA

- MOTION

- MAXIMUM_INTERVAL_EXPIRATION_EVENT

- LOCATION_CANCELLATION_EVENT

- ACTIVATION_OF_DEFERRED_LOCATION

- type: string

Annex B (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-04 | CT4#90 | C4-191340 |  |  |  | Initial Draft of Gateway Mobile Location Services | 0.1.0 |
| 2019-05 | CT4#91 | C4-192485 |  |  |  | V0.2.0 | 0.2.0 |
| 2019-09 | CT4#93 | C4-193845 |  |  |  | Implementation of pCRs agreed at CT4#93 | 0.3.0 |
| 2019-10 | CT4#94 | C4-194555 |  |  |  | Implementation of pCRs agreed at CT4#94 | 0.4.0 |
| 2019-11 | CT4#95 | C4-195413, C4-195409, C4-195296 |  |  |  | Implementation of pCRs agreed at CT4#95 | 0.5.0 |
| 2019-12 | CT#86 | CP-193065 |  |  |  | TS presented for information | 1.0.0 |
| 2020-03 | CT4#96e | C4-200725, C4-200727, C4-200943, C4-200993, C4-200995, C4-201286 |  |  |  | Implementation of pCRs agreed at CT4#96e | 1.1.0 |
| 2020-03 | CT#87e | CP-200060 |  |  |  | TS presented for approval | 2.0.0 |
| 2020/03 | CT#87e |  |  |  |  | Approved at CT87e | 16.0.0 |