| 3GPP TS 29.536 V0.1.0 (2021-04) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Network Slice Admission Control Services Stage 3 (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions, abbreviations	7

3.1	Definitions	7

3.2	Abbreviations	8

4	Overview	8

5	Services offered by the NSACF	8

5.1	Introduction	8

5.2	Nnsacf_NumOfUEsPerSlice Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	AvailabilityCheckAndUpdate	9

5.2.2.2.1	General	9

5.2.2.2.2	AMF initiated network slice admission control	10

5.2.2.3	EACNotify	11

5.2.2.3.1	General	11

5.2.2.3.2	NSACF initiated EAC mode configuration	11

6	API Definitions	11

6.1	Nnsacf_NumOfUEsPerSlice Service API	11

6.1.1	Introduction	11

6.1.2	Usage of HTTP	12

6.1.2.1	General	12

6.1.2.2	HTTP standard headers	12

6.1.2.2.1	General	12

6.1.2.2.2	Content type	12

6.1.2.3	HTTP custom headers	12

6.1.3	Resources	12

6.1.3.1	Overview	12

6.1.3.2	Resource:  Slice Collection Subject to NSAC	13

6.1.3.2.1	Description	13

6.1.3.2.2	Resource Definition	13

6.1.3.2.3	Resource Standard Methods	13

6.1.3.2.3.1	POST	13

6.1.3.2.4	Resource Custom Operations	15

6.1.4	Custom Operations without associated resources	15

6.1.5	Notifications	15

6.1.5.1	General	15

6.1.5.2	EAC Mode Notification	15

6.1.5.2.1	Description	15

6.1.5.2.2	Target URI	15

6.1.5.2.3	Standard Methods	15

6.1.5.2.3.1	POST	15

6.1.6	Data Model	16

6.1.6.1	General	16

6.1.6.2	Structured data types	16

6.1.6.2.1	Introduction	16

6.1.6.2.2	Type: UeACRequestData	16

6.1.6.2.3	Type: UeACResponseData	17

6.1.6.2.4	Type: EACNotification	17

6.1.6.3	Simple data types and enumerations	17

6.1.6.3.1	Introduction	17

6.1.6.3.2	Simple data types	17

6.1.6.3.3	Enumeration: EACMode	17

6.1.6.4	Data types describing alternative data types or combinations of data types	17

6.1.6.5	Binary data	17

6.1.7	Error Handling	18

6.1.7.1	General	18

6.1.7.2	Protocol Errors	18

6.1.7.3	Application Errors	18

6.1.8	Feature negotiation	18

6.1.9	Security	18

Annex A (normative): OpenAPI specification	19

A.1	General	19

A.2	Nnsacf_NumOfUEsPerSlice API	19

Annex <X> (informative): Change history	20

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This clause is optional. If it exists, it is always the second unnumbered clause.

# 1	Scope

This clause will describe the scope of the corresponding service specification.

The present document specifies the stage 3 protocol and data model for the Nnsacf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the NSACF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI Specification Version 3.0.0", https://spec.openapis.org/oas/v3.0.0.

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

# 3	Definitions, abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

5GC	5G Core Network

AMF	Access Management Function

EAC	Early Admission Control

NSAC	Network Slice Admission Control

NSACF	Network Slice Admission Control Function

# 4	Overview

Within the 5GC, the NSACF offers services to the AMF via the Nnsacf service based interface (see 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3]).

Figures 4.1 provides the reference model (in service based interface representation and in reference point representation), with focus on the NSACF and the scope of the present specification.

Figure 4-1: Reference model – NSACF

The functionalities supported by the NSACF are listed in clause 6.2.x of 3GPP TS 23.501 [2].

The services and service operations provided by the Nnsacf interface are listed in clause 5.2.x of 3GPP TS 23.502 [3].

# 5	Services offered by the NSACF

## 5.1	Introduction

The NSACF supports the following services.

Table 5.1-1: NF Services provided by NSACF

| Service Name | Description | Example Consumer |
| --- | --- | --- |
| Nnsacf_NumOfUEsPerSlice | This service allows AMF to request NSACF to perform per slice admission control for the number of UEs. | AMF |

Table 5.1-2 summarizes the corresponding APIs defined for this specification.

Table 5.1-2: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nnsacf_NumOfUEsPerSlice | 6.1 | per slice admission control service for the number of UEs | TS29536_Nnsacf_NumOfUEsPerSlice.yaml | nnsacf-num-of-ues-per-slice | A.2 |

Editor's Note:	The name of "NumOfUEsPerSlice" looks more like a name for a service operation other than a name for a service. It is FFS whether a better name for the service should be considered, e.g. "SAC" (per Slice Admission Control) or "PSAC" (Per Slice Admission Control). If so, and LS to SA2 is needed.

## 5.2	Nnsacf_NumOfUEsPerSlice Service

### 5.2.1	Service Description

The Nnsacf_NumOfUEsPerSlice service provides the service capability for the NF Service Consumer (e.g. AMF) to request admission control for UEs accessing a specific network slice. The following are the key functionalities of this NF service:

-	Request the NSACF to control the number of UEs registered to a specific slice, e.g. perform availability check and update the number of UEs registered to a specific slice;

-	Notify the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC (early admission control) mode for NSAC procedure;

The Nnsacf_NumOfUEsPerSlice service supports the following service operations.

Table 5.2.1-1: Service operations supported by the Nnsacf_NumOfUEsPerSlice service

| Service Operations | Description | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| AvailabilityCheckAndUpdate | Request the NSACF to perform admission control related to the number of UEs registered to a slice. | Request/Response | AMF |
| EACNotify | Notify the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC mode. | Subscribe/Notify | AMF |

Editor's Note:	It is FFS whether a better name for the "AvailabilityCheckAndUpdate" service operation should be considered. If so, an LS to SA2 is needed.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

This clause introduces the related procedures using Nnsacf_NumOfUEsPerSlice service operations to request the NSACF to control the number of UEs registered to a specific slice.

#### 5.2.2.2	AvailabilityCheckAndUpdate

##### 5.2.2.2.1	General

The AvailabilityCheckAndUpdate service operation shall be used by the NF Service Consumer (e.g. AMF) to request the NSACF to control the number of UEs registered to a specific slice, e.g. perform availability check and update the number of UEs registered to a slice. It is used in the following procedures:

-	AMF initiated network slice admission control procedure related to control the number of UEs registered to a slice (see clause 4.2.11.2 of 3GPP TS 23.502 [3]).

##### 5.2.2.2.2	AMF initiated network slice admission control

The NF Service Consumer (e.g. AMF) shall invoke the AvailabilityCheckAndUpdate service operation to request the NSACF to perform network slice admission control procedure related to the number of UEs, by using the HTTP POST method as shown in Figure 5.2.2.2.2-1.

Figure 5.2.2.2.2-1: Availability Check for Number of UEs per Slice

1.	The NF Service Consumer (e.g. AMF) shall send a POST request to the resource representing the network slice admission control related to the number of UEs (i.e. …/slices/ue-ac) in the NSACF. The payload body of the POST request shall contain the input data structure for network slice admission control.

2a.	For each S-NSSAI included in UeACRequestData, the NSACF shall perform the following actions:

-	if the update flag is set to "increase" and EAC mode for this S-NSSAI is enabled, the NSACF shall first check the total number of UEs to this slice will not exceed the maximum number of UEs allowed to be registered to this slice. When counting the total number of the UEs ( including the UEs indicated in the request and the UEs already stored in the NSACF), the NSACF shall check whether a UE indicated in the request is already in the UE ID list stored in the NSACF.

-	if no excess, the NSACF records the indicated UEs to the UE ID list stored in the NSACF, and updates the total number of UEs registered to this slice accordingly;

-	if the total number of UEs exceeds the maximum of UEs allowed to be registered to this slice, the NSACF shall not update the UE ID list stored in the NSACF, and not update the total number of UEs. Instead, the NSACF shall record this S-NSSAI in the failed list of S-NSSAI in the response message, together with a "MAXIMUM_NUM_REACHED" reason;

-	if the update flag is set to "increase" and EAC mode for this S-NSSAI is disabled, the NSACF shall:

-	record the indicated UEs to the UE ID list stored in the NSACF, and updates the total number of UEs registered to this slice accordingly;

-	if the update flag is set to "decrease", the NSACF decrease the total number of UEs registered to this slice, and removes the indicated UEs from the UE ID list stored in the NSACF;

If above NSACF handling is successful, "200 OK" shall be returned, with necessary response data, e.g. indicating the failed S-NSSAI.

Editor's Note:	It is decided to return "200 OK" at this time, as the NSACF handling is regarded successful even the total number of UEs for some S-NSSAI exceed the allowed maximum number.

2b.	On failure or redirection, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned.

A ProblemDetails IE shall be included in the payload body of POST response, with the "cause" attribute of ProblemDetails set to application error codes specified in table 6.1.7.3-1.

#### 5.2.2.3	EACNotify

##### 5.2.2.3.1	General

The EACNotify service operation shall be used by the NSACF to inform the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC mode. It is used in the following procedures:

-	NSACF initiated configuration on EAC mode procedure (see clause 4.2.11.3 of 3GPP TS 23.502 [3]).

##### 5.2.2.3.2	NSACF initiated EAC mode configuration

The EACNotify service operation shall be used by the NSACF to configure the EAC mode to the NF Service Consumer (e.g. AMF).

The NSACF shall notify the NF Service Consumer (e.g. AMF) by using the HTTP POST method as shown in Figure 5.2.2.2.3-1.

Figure 5.2.2.2.3-1: NSACF initiated EAC mode configuration procedure

1.	The NSACF shall send a POST request to the EAC Notification callback URI provided by the NF Service Consumer (e.g. AMF). The payload body of the POST request shall contain the notification payload.

The callback URI may be provided by the AMF during the first interaction with the NSACF, or be retrieved from the AMF profile during NF discovery from the NRF.

2a.	On success, "204 No Content" shall be returned and the payload body of the POST response shall be empty.

2b.	On failure or redirection, one of the HTTP status code listed in Table 6.1.5.2.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.7.3-1.

# 6	API Definitions

## 6.1	Nnsacf_NumOfUEsPerSlice Service API

### 6.1.1	Introduction

The Nnsacf_NumOfUEsPerSlice shall use the Nnsacf_NumOfUEsPerSlice API.

The API URI of the Nnsacf_NumOfUEsPerSlice API shall be:

{apiRoot}/<apiName>/<apiVersion>/

The request URIs used in HTTP requests from the NF service consumer towards the NF service producer shall have the Resource URI structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nnsacf-num-of-ues-per-slice".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 5.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the <API Name> API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be supported, and the optional HTTP custom header fields specified in clause 5.2.3.3 of 3GPP TS 29.500 [4] may be supported.

### 6.1.3	Resources

#### 6.1.3.1	Overview

The figure 6.1.3.1-1 describes the resource URI structure of the Nnsacf-NumOfUEsPerSlice API.

Figure 6.1.3.1-1: Resource URI structure of the nnsacf-num-of-ues-per-slice API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| Slice Collection Subject to NSAC (Collection) | /slices | POST | Request the NSACF to perform network slice admission control related to the number of UEs registered to a slice, for a group of slices. |

#### 6.1.3.2	Resource:  Slice Collection Subject to NSAC

##### 6.1.3.2.1	Description

This resource represents the collection of slice subject to NSAC.

This resource is modelled with the Store resource archetype (see clause C.2 of 3GPP TS 29.501 [5]).

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/<apiName>/<apiVersion>/slices

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Data type | Definition |
| --- | --- | --- |
| apiRoot | string | See clause 6.1.1 |
| apiVersion | string | See clause 6.1.1 |
| <name> | <type> | <definition> |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	POST

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the <method 1> method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| UeACRequestData | M | 1 | Request data for NSAC procedure related to the number of UEs per slice. |

Table 6.1.3.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| UeACResponseData | M | 1 | 200 OK | Response data for NSAC procedure related to the number of UEs per slice. |
|  |  |  | 307 Temporary Redirect | Temporary redirection, during a NSAC procedure. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |
|  |  |  | 308 Permanent Redirect | Permanent redirection, during a NSAC procedure. The response shall include a Location header field containing a different URI. The URI shall be an alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |
| ProblemDetails | O | 0..1 | 404 Not Found | When used to represent the failure of NSAC procedure, the "cause" attribute of the "ProblemDetails" shall be set to one of the following application error codes: - SLICE_NOT_FOUND, if the given S-NSSAI is not found from the NSSAI which are subject to NSAC procedure; |
| NOTE: The manadatory HTTP error status code for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

Table 6.1.3.2.3.1-4: Headers supported by the POST method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

Table 6.1.3.2.3.1-5: Headers supported by the 200 response code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |

Table 6.1.3.2.3.1-6: Links supported by the 200 Response Code on this endpoint

| Name | Resource name | HTTP method or custom operation | Link parameter(s) | Description |
| --- | --- | --- | --- | --- |

Table 6.1.3.2.3.1-7: Headers supported by the 307 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |

Table 6.1.3.2.3.1-8: Headers supported by the 308 Response Code on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| Location | string | M | 1 | An alternative URI of the resource located on an alternative service instance within the NSACF that was selected by the AMF. |

##### 6.1.3.2.4	Resource Custom Operations

In this release of this specification, no custom operations associated to this resource is defined.

### 6.1.4	Custom Operations without associated resources

In this release of this specification, no custom operations without associated resources are defined.

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

Table 6.1.5.1-1: Notifications overview

| Notification | Callback URI | HTTP method or custom operation | Description (service operation) |
| --- | --- | --- | --- |
| EAC Mode Notification | {EACNotificationUri} | POST | Notify the NF Service Customer (e.g. AMF) of the activation/deactivation of EAC mode. |

#### 6.1.5.2	EAC Mode Notification

##### 6.1.5.2.1	Description

The EAC Mode Notification is used by the NSACF to inform the NF Service Consumer (e.g. AMF) of the activation/deactivation of EAC mode.

##### 6.1.5.2.2	Target URI

The Callback URI "{EACNotificationUri}" shall be used with the callback URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Callback URI variables

| Name | Definition |
| --- | --- |
| eacNotificationUri | Notification URI for receiving EAC mode notification. |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EACNotification | M | 1 | EAC mode notification |

Table 6.1.5.2.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nnsacf service based interface protocol.

Table 6.1.6.1-1: Nnsacf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| UeACRequestData | 6.1.6.2.2 | Input data for NSAC procedure related to the number of UEs per slice. |  |
| UeACResponseData | 6.1.6.2.3 | Response data |  |
| EACNotification | 6.1.6.2.4 | EAC mode notification |  |
| EACMode | 6.1.6.3.3 | EAC mode |  |

Table 6.1.6.1-2 specifies data types re-used by the Nnsacf  service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nnsacf service based interface.

Table 6.1.6.1-2: Nnsacf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: UeACRequestData

Table 6.1.6.2.2-1: Definition of type UeACRequestData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

##### 6.1.6.2.3	Type: UeACResponseData

Table 6.1.6.2.3-1: Definition of type UeACResponseData

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |

##### 6.1.6.2.4	Type: EACNotification

Table 6.1.6.2.4-1: Definition of type EACNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| eacModeList | map(EACMode) | O | 1 | a map of EAC Mode where the S-NSSAI serves as the key. |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: EACMode

The enumeration EACMode represents the mode of Early Admission Control. It shall comply with the provisions defined in table 6.1.6.3.3-1.

Table 6.1.6.3.3-1: Enumeration EACMode

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "ACTIVE" | EAC mode is enabled. |  |
| "DEACTIVE" | EAC mode is disabled. |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

In this release, no alternative data types or combinations of data types are defined in this specification.

#### 6.1.6.5	Binary data

In this release, no binary data types are defined in this specification.

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nnsacf_NumOfUEsPerSlice API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nnsacf_NumOfUEsPerSlice API.

#### 6.1.7.2	Protocol Errors

No specific procedures for the Nnsacf_NumOfUEsPerSlice service are specified.

#### 6.1.7.3	Application Errors

The application errors defined for the Nnsacf_NumOfUEsPerSlice service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| SLICE_NOT_FOUND | 404 Not Found | The given S-NSSAI is not found from the NSSAI which are subject to NSAC. |

## 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the <API name> API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |

## 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the <API Name> API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the <API Name> API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the <API Name> service.

The <API Name> API defines a single scope "<API name in lower letters with underscores>" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI  specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

## A.2	Nnsacf_NumOfUEsPerSlice API

openapi: 3.0.0

info:

title: Nnsacf_NumOfUEsPerSlice

version: 1.0.0-alpha.1

description: |

Nnsacf_NumOfUEsPerSlice Service.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.536 V0.1.0; 5G System; Network Slice Admission Control Services; Stage 3.

url: http://www.3gpp.org/ftp/Specs/archive/29_series/29.536/

servers:

- url: '{apiRoot}/nnsacf-num-of-ues-per-slice/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nnsacf-num-of-ues-per-slice

paths:

# API specific definitions

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

<API name in lower letters with underscores>: Access to the <API Name> API

schemas:

# API specific definitions

Annex <X> (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2021-04 | CT4#103E | C4-212108 |  |  |  | TS skeleton | 0.0.1 |
| 2021-04 | CT4#103E | C4-212430 C4-212610 C4-212432 C4-212112 |  |  |  | Implementation of pCRs agreed in CT4#103E | 0.1.0 |