3GPP TS 29.572 V16.1.0 (2019-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

5G System;

Location Management Services;

Stage 3

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

3GPP, 5G system, Location

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	7

2	References	7

3	Definitions and abbreviations	8

3.1	Definitions	8

3.2	Abbreviations	8

4	Overview	8

5	Services Offered by the LMF	9

5.1	Introduction	9

5.2	Nlmf_Location Service	9

5.2.1	Service Description	9

5.2.2	Service Operations	9

5.2.2.1	Introduction	9

5.2.2.2	DetermineLocation	10

5.2.2.2.1	General	10

5.2.2.2.2	Retrieve UE Location	10

5.2.2.3	EventNotify	10

5.2.2.3.1	General	10

5.2.2.3.2	Periodic or Triggered Event Notification	10

5.2.2.4	CancelLocation	11

5.2.2.4.1	General	11

5.2.2.4.2	Cancel Periodic or Triggered Location	11

5.2.2.5	LocationContextTransfer	12

5.2.2.5.1	General	12

5.2.2.5.2	Transfer Location Context	12

5.3	Nlmf_Broadcast Service	13

5.3.1	Service Description	13

5.3.2	Service Operations	13

5.3.2.1	Introduction	13

5.3.2.2	CipheringKeyData	13

5.3.2.2.1	General	13

5.3.2.2.2	Request Ciphering Key Information	13

5.3.2.2.3	Provide Ciphering Key Information	14

6	API Definitions	15

6.1	Nlmf_Location Service API	15

6.1.1	API URI	15

6.1.2	Usage of HTTP	15

6.1.2.1	General	15

6.1.2.2	HTTP Standard Headers	15

6.1.2.2.1	General	15

6.1.2.2.2	Content type	15

6.1.2.3	HTTP custom headers	15

6.1.2.3.1	General	15

6.1.3	Resources	16

6.1.3.1	Overview	16

6.1.4	Custom Operations without associated resources	16

6.1.4.1	Overview	16

6.1.4.2	Operation: determine-location	17

6.1.4.2.1	Description	17

6.1.4.2.2	Operation Definition	17

6.1.4.3	Operation: cancel-location	17

6.1.4.3.1	Description	17

6.1.4.3.2	Operation Definition	17

6.1.4.4	Operation: location-context-transfer	18

6.1.4.4.1	Description	18

6.1.4.4.2	Operation Definition	18

6.1.5	Notifications	18

6.1.5.1	EventNotify	18

6.1.5.1.1	Description	18

6.1.5.1.2	Notification Definition	19

6.1.5.1.3	Notification Standard Methods	19

6.1.5.1.3.1	POST	19

6.1.6	Data Model	19

6.1.6.1	General	19

6.1.6.2	Structured data types	22

6.1.6.2.1	Introduction	22

6.1.6.2.2	Type: InputData	23

6.1.6.2.3	Type: LocationData	24

6.1.6.2.4	Type: GeographicalCoordinates	24

6.1.6.2.5	Type: GeographicArea	25

6.1.6.2.6	Type: Point	25

6.1.6.2.7	Type: PointUncertaintyCircle	25

6.1.6.2.8	Type: PointUncertaintyEllipse	25

6.1.6.2.9	Type: Polygon	26

6.1.6.2.10	Type: PointAltitude	26

6.1.6.2.11	Type: PointAltitudeUncertainty	26

6.1.6.2.12	Type: EllipsoidArc	26

6.1.6.2.13	Type: LocationQoS	27

6.1.6.2.14	Type: CivicAddress	28

6.1.6.2.15	Type: PositioningMethodAndUsage	30

6.1.6.2.16	Type: GnssPositioningMethodAndUsage	30

6.1.6.2.17	Type: VelocityEstimate	30

6.1.6.2.18	Type: HorizontalVelocity	31

6.1.6.2.19	Type: HorizontalWithVerticalVelocity	31

6.1.6.2.20	Type: HorizontalVelocityWithUncertainty	31

6.1.6.2.21	Type: HorizontalWithVerticalVelocityAndUncertainty	31

6.1.6.2.22	Type: UncertaintyEllipse	32

6.1.6.2.23	Type: UeLcsCapability	32

6.1.6.2.24	Type: PeriodicEventInfo	32

6.1.6.2.25	Type: AreaEventInfo	33

6.1.6.2.26	Type: ReportingArea	33

6.1.6.2.27	Type: MotionEventInfo	34

6.1.6.2.28	Type: ReportingAccessTypes	34

6.1.6.2.29	Type: CancelLocData	34

6.1.6.2.30	Type: LocContextData	35

6.1.6.2.31	Type: EventReportMessage	35

6.1.6.2.32	Type: EventReportingStatus	35

6.1.6.2.33	Type: UELocationInfo	35

6.4.6.2.34	Type: EventNotifyData	36

6.1.6.3	Simple data types and enumerations	36

6.1.6.3.1	Introduction	36

6.1.6.3.2	Simple data types	36

6.1.6.3.3	Enumeration: ExternalClientType	38

6.1.6.3.4	Enumeration: SupportedGADShapes	38

6.1.6.3.5	Enumeration: ResponseTime	38

6.1.6.3.6	Enumeration: PositioningMethod	38

6.1.6.3.7	Enumeration: PositioningMode	39

6.1.6.3.8	Enumeration: GnssId	39

6.1.6.3.9	Enumeration: Usage	39

6.1.6.3.10	Enumeration: LcsPriority	39

6.1.6.3.11	Enumeration: VelocityRequested	40

6.1.6.3.12	Enumeration: AccuracyFulfilmentIndicator	40

6.1.6.3.13	Enumeration: VerticalDirection	40

6.1.6.3.14	Enumeration: LdrType	40

6.1.6.3.15	Enumeration: ReportingAreaType	40

6.1.6.3.16	Enumeration: OccurrenceInfo	41

6.1.6.3.17	Enumeration: ReportingAccessType	41

6.1.6.3.18	Enumeration: EventClass	41

6.1.6.3.19	Enumeration: ReportedEventType	41

6.1.6.3.20	Enumeration: TerminationCause	42

6.1.6.3.21	Enumeration: LcsQosClass	42

6.1.7	Error Handling	42

6.1.7.1	General	42

6.1.7.2	Protocol Errors	42

6.1.7.3	Application Errors	42

6.1.8	Security	42

6.2	Nlmf_Broadcast Service API	43

6.2.1	API URI	43

6.2.2	Usage of HTTP	43

6.2.2.1	General	43

6.2.2.2	HTTP Standard Headers	43

6.2.2.2.1	General	43

6.2.2.2.2	Content type	43

6.2.2.3	HTTP custom headers	43

6.2.2.3.1	General	43

6.2.3	Resources	44

6.2.3.1	Overview	44

6.2.4	Custom Operations without associated resources	44

6.2.4.1	Overview	44

6.2.4.4	Operation: cipher-key-data	44

6.2.4.4.1	Description	44

6.2.4.4.2	Operation Definition	44

6.2.5	Notifications	45

6.2.5.1	CipheringKeyData	45

6.2.5.1.1	Description	45

6.2.5.1.2	Notification Definition	45

6.2.5.1.3	Notification Standard Methods	45

6.2.5.1.3.1	POST	45

6.2.6	Data Model	46

6.2.6.1	General	46

6.2.6.2	Structured data types	46

6.2.6.2.1	Introduction	46

6.2.6.2.2	Type: CipheringKeyInfo	47

6.2.6.2.3	Type: CipheringKeyResponse	47

6.2.6.2.4	Type: CipheringDataSet	48

6.2.6.2.5	Type: CipheringSetReport	53

6.2.6.2.6	Type: CipherRequestData	53

6.2.6.2.7	Type: CipherResponseData	54

6.2.6.3	Simple data types and enumerations	54

6.2.6.3.1	Introduction	54

6.2.6.3.2	Simple data types	54

6.2.6.3.3	Enumeration: StorageOutcome	54

6.2.6.3.4	Enumeration: DataAvailability	54

6.2.7	Error Handling	54

6.2.7.1	General	54

6.2.7.2	Protocol Errors	55

6.2.7.3	Application Errors	55

6.2.8	Security	55

Annex A (normative):	OpenAPI specification	55

A.1	General	55

A.2	Nlmf_Location API	55

A.3	Nlmf_Broadcast API	70

Annex B (informative):	Change history	73

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nlmf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the LMF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	IETF RFC 4776: "Dynamic Host Configuration Protocol (DHCPv4 and DHCPv6) Option for Civic Addresses Configuration Information".

[7]	IETF RFC 5139: "Revised Civic Location Format for Presence Information Data Format Location Object (PIDF-LO)".

[8]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[9]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[10]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[11]	3GPP TS 29.510: "Network Function Repository Services; Stage 3".

[12]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[13]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[14]	OpenAPI Initiative, "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md.

[15]	IETF RFC 7807: "Problem Details for HTTP APIs".

[16]	3GPP TR 21.900: "Technical Specification Group working methods".

[17]	3GPP TS 22.071: "Location Services (LCS); Service description; Stage 1".

[18]	3GPP TS 29.002: "Mobile Application Part (MAP) specification".

[19]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[20]	3GPP TS 24.080: "Mobile radio interface layer 3 Supplementary services specification; Formats and coding".

[21]	3GPP TS 36.355: "Evolved Universal Terrestrial Radio Access (E-UTRA); LTE Positioning Protocol (LPP)".

[22]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

LDR	Location Deferred Request

LMF	Location Management Function

# 4	Overview

The Location Management Function (LMF) is the network entity in the 5G Core Network (5GC) supporting the following functionality:

-	Supports location determination for a UE.

-	Obtains downlink location measurements or a location estimate from the UE.

-	Obtains uplink location measurements from the NG RAN.

-	Obtains non-UE associated assistance data from the NG RAN.

-	Provides broadcast assistance data to UEs and forwards associated ciphering keys to an AMF.

Other functions of an LMF are listed in clause 4.3.8 of 3GPP TS 23.273 [19].

Figure 4-1 provides the reference model (in service based interface representation and in reference point representation), with focus on the LMF:

Figure 4-1: Reference model – LMF

# 5	Services Offered by the LMF

## 5.1	Introduction

The LMF offers to other NFs the following services:

-	Nlmf_Location

-	Nlmf_Broadcast

## 5.2	Nlmf_Location Service

### 5.2.1	Service Description

The Nlmf_Location service enables an NF to request location determination (current geodetic and optionally civic location) for a target UE or to request periodic or triggered location for a target UE.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

The service operations defined for the Nlmf_Location service are as follows:

-	DetermineLocation: It provides UE location information to the consumer NF.

-	EventNotify: It notifies the consumer NF of an event for periodic or triggered location for a target UE.

-	CancelLocation: It enables a consumer NF to cancel an ongoing periodic or triggered location for a target UE.

-	LocationContextTransfer: It enables a consumer NF to transfer location context information for periodic or triggered location of a target UE to a new LMF.

#### 5.2.2.2	DetermineLocation

##### 5.2.2.2.1	General

The following procedures are defined, using the "DetermineLocation" service operation:

-	Retrieve UE Location

##### 5.2.2.2.2	Retrieve UE Location

This procedure allows a consumer NF to request the location information (geodetic location and, optionally, civic location) for a target UE or to activate periodic or triggered deferred location for a target UE.

Figure 5.2.2.2.2-1: DetermineLocation Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the "determine-location" custom operation. The input parameters for the request (external client type, LCS correlation identifier, serving cell identifier, location QoS, supported GAD shapes, LDR Type, H-GMLC address, LDR Reference ….) shall be included in the HTTP POST request body.

If UE LCS Capability is received in the request indicating LPP is not supported by the UE, the LMF shall not send LPP messages to the UE in subsequent positioning procedures.

2a.	On success, "200 OK" shall be returned. The response body shall contain the parameters related to the determined position of the UE if any (geodetic position, civic location, positioning methods…).

2b.	On failure, one of the HTTP status code listed in Table 6.1.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.4.2.2-2.

#### 5.2.2.3	EventNotify

##### 5.2.2.3.1	General

The following procedures are defined, using the "EventNotify" service operation:

-	Periodic or Triggered Event Notification

##### 5.2.2.3.2	Periodic or Triggered Event Notification

This procedure notifies the NF Service Consumer (i.e. GMLC) about event information related to periodic or triggered location of a target UE. The notification is delivered to:

-	the callback URI of an H-GMLC received (from an AMF) during an earlier DetermineLocation service operation if still available and if the LMF is configured for direct access to the H-GMLC;

Editor's note [WI: 5G_eLCS, CR#0033]: TS 29.518 needs to be updated to enable the AMF to provide the callback URI of an H-GMLC.

-	the callback URI of an H-GMLC received (from another LMF) during an earlier LocationContextTransfer service operation if still available and if the LMF is configured for direct access to the H-GMLC;

-	the callback URI of an H-GMLC received (from the target UE) in a supplementary services event report if the LMF is configured for direct access to the H-GMLC;

otherwise (if not available),

-	the callback URI of a V-GMLC registered in the NRF, if the V-GMLC registered to the NRF with notification endpoints for periodic or triggered event notifications; or

otherwise (if not available),

-	the URI of a V-GMLC locally provisioned in the LMF.

Figure 5.2.2.3.2-1: EventNotify Request

1.	The LMF shall send a POST request to the GMLC callback URI determined as described above. The request body shall include a notification correlation ID (LDR reference), the UE identification (SUPI and if available GPSI), the type of event and may include a geodetic location, civic location, position methods used, H-GMLC callback URI (if the NF consumer is a V-GMLC) and serving LMF identification.

2a.	On success, "204 No content" shall be returned by the NF Service Consumer.

2b.	On failure, the appropriate HTTP status code (e.g. "403 Forbidden") indicating the error shall be returned and the message body should contain a ProblemDetails structure indicating appropriate additional error information.

#### 5.2.2.4	CancelLocation

##### 5.2.2.4.1	General

The following procedures are defined, using the "CancelLocation" service operation:

-	Cancel Periodic or Triggered Location

##### 5.2.2.4.2	Cancel Periodic or Triggered Location

This procedure allows a consumer NF to cancel periodic or triggered location for a target UE. The cancellation is delivered to a resource URI indicated by a serving LMF identification provided to the consumer NF (i.e. AMF) by a V-GMLC or H-GMLC (see 3GPP TS 23.273 [19]).

Figure 5.2.2.4.2-1: CancelLocation Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the serving LMF. The request body shall include a notification correlation ID (LDR reference) and an H-GMLC callback URI.

2a.	On success, "204 No content" shall be returned by the LMF.

2b.	On failure, one of the HTTP status code listed in Table 6.1.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.1.4.2.2-2.

#### 5.2.2.5	LocationContextTransfer

##### 5.2.2.5.1	General

The following procedures are defined, using the "LocationContextTransfer" service operation:

-	Transfer Location Context

##### 5.2.2.5.2	Transfer Location Context

This procedure allows a consumer NF to transfer location context information for periodic or triggered location for a target UE. The location context information is delivered to:

-	the URI of an LMF received (from an AMF) during an earlier Namf_Communication_N1MessageNotify service operation to the consumer NF;

Editor's note [WI: 5G_eLCS, CR#0033]: TS 29.518 needs to be updated to enable the AMF to provide the callback URI of an LMF.

otherwise (if not available),

-	the URI of an LMF locally provisioned in the consumer NF.

Figure 5.2.2.5.2-1: LocationContextTransfer Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI determined as described above. The request body shall include an AMF identity, Deferred location type, Deferred location parameters, Notification Target Address (H-GMLC callback URI), Notification Correlation ID (LDR reference), an embedded event report message and may include an event reporting status and UE location information.

2a.	On success, "204 No content" shall be returned by the LMF.

2b.	On failure, one of the HTTP status codes listed in Table 6.1.4.2.2-2 shall be returned. For a 4xx/5xx response, the message body should contain a ProblemDetails structure with the "cause" attribute set to one of the application error listed in Table 6.1.4.2.2-2.

## 5.3	Nlmf_Broadcast Service

### 5.3.1	Service Description

The Nlmf_Broadcast service enables an NF to obtain ciphering keys and associated parameters applicable to location assistance data that is broadcast to subscribed UEs in ciphered form.

### 5.3.2	Service Operations

#### 5.3.2.1	Introduction

The service operations defined for the Nlmf_Broadcast service are as follows:

-	CipheringKeyData: It provides the ciphering key information to the consumer NF.

#### 5.3.2.2	CipheringKeyData

##### 5.3.2.2.1	General

The following procedures are defined, using the "CipheringKeyData" service operation:

-	Request Ciphering Key Information

-	Provide Ciphering Key Information

NOTE:	The Request Ciphering Key procedure is included in order to provide a valid context in OpenAPI version 3 for the Provide Ciphering Key Information procedure. The Request Ciphering Key procedure is not used for support of ciphering key transfer in 3GPP TS 23.273 [19] and hence need not be supported by an NF Service Consumer or by an LMF.

##### 5.3.2.2.2	Request Ciphering Key Information

This procedure allows a consumer NF to request ciphering key information.

Figure 5.3.2.2.2-1: CipheringKeyData Request

1.	The NF Service Consumer shall send an HTTP POST request to the resource URI associated with the "cipher-key-data" custom operation. The request body shall include a notification callback URI.

2a.	On success, "200 OK" shall be returned. The response body shall indicate whether the LMF has ciphering key data. If the LMF has ciphering key data, the Provide Ciphering Key Information procedure is used to provide the ciphering key data to the NF Service Consumer.

2b.	On failure, one of the HTTP status codes listed in Table 6.2.4.4.2-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in Table 6.2.7.3-1.

##### 5.3.2.2.3	Provide Ciphering Key Information

This procedure notifies the NF Service Consumer (i.e. AMF) about updated ciphering key information applicable to broadcast of location assistance data in ciphered form to subscribed UEs. The notification is delivered to:

-	the callback URI of an AMF received during an earlier CipheringKeyData request service operation if still available; or

-	a callback URI registered in the NRF, if the AMF registered to the NRF with notification endpoints for ciphering key data notifications;

Otherwise (if not available),

-	an AMF callback URI locally provisioned in the LMF.

The procedure is invoked by issuing a POST request to the callback URI of the NF Service Consumer. See figure 5.3.2.2.3-1.

Figure 5.3.2.2.3-1: CipheringKeyData Notify

1.	The LMF shall send an HTTP POST request to the callback URI for the NF service consumer determined as described above. The request body shall include one or more ciphering keys and for each ciphering key may include a ciphering key value, ciphering key identifier, validity period and set of applicable types of broadcast assistance data.

2a.	On success or partial success, "200 OK" shall be returned. The response body shall indicate which ciphering key information was successfully stored by the NF service consumer.

2b.	On failure to store any ciphering key information, one of the HTTP status codes listed in table 6.2.5.1.3.1-2 shall be returned. For a 4xx/5xx response, the message body shall contain a ProblemDetails structure with the "cause" attribute set to one of the application errors listed in table 6.2.5.1.3.1-2.

# 6	API Definitions

## 6.1	Nlmf_Location Service API

### 6.1.1	API URI

The Nlmf_Location service shall use the Nlmf_Location API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nlmf-loc".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, as defined in IETF RFC 7540 [12], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nlmf_Location service shall comply with the OpenAPI [14] specification contained in Annex A.

#### 6.1.2.2	HTTP Standard Headers

##### 6.1.2.2.1	General

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [13], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 7807 [15]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

#### 6.1.2.3	HTTP custom headers

##### 6.1.2.3.1	General

The following HTTP custom headers shall be supported:

-	3gpp-Sbi-Message-Priority: See 3GPP TS 29.500 [4], clause 5.2.3.2.2.

This API does not define any new HTTP custom headers.

### 6.1.3	Resources

#### 6.1.3.1	Overview

The structure of the Resource URIs of the Nlmf_Location service is shown in figure 6.1.3.1-1.

Figure 6.1.3.1-1: Resource URI structure of the Nlmf_Location API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| DetermineLocation (Custom operation) | {apiRoot}/nlmf-loc/<apiVersion>/determine-location | determine-location (POST) |  |
| CancelLocation (Custom operation) | {apiRoot}/nlmf-loc/<apiVersion>/cancel-location | cancel-location (POST) |  |
| LocationContextTransfer (Custom operation) | {apiRoot}/nlmf-loc/<apiVersion>/location-context-transfer | location-context-transfer (POST) |  |

### 6.1.4	Custom Operations without associated resources

#### 6.1.4.1	Overview

Table 6.1.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/nlmf-loc/<apiVersion>/determine-location | POST |  |
| {apiRoot}/nlmf-loc/<apiVersion>/cancel-location | POST |  |
| {apiRoot}/nlmf-loc/<apiVersion>/location-context-transfer | POST |  |

#### 6.1.4.2	Operation: determine-location

##### 6.1.4.2.1	Description

This sublause will describe the custom operation and what it is used for, and the custom operation's URI.

##### 6.1.4.2.2	Operation Definition

This operation shall support the response data structures and response codes specified in tables 6.1.4.2.2-1 and 6.1.4.2.2-2.

Table 6.1.4.2.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| InputData | M | 1 | Input parameters to the "Deterrmine Location" operation |

Table 6.1.4.2.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| LocationData | M | 1 | 200 OK | This case represents the successful retrieval of the location of the UE or successful activation of periodic or triggered location in the UE. Upon success, a response body is returned containing the different parameters of the location data if obtained, such as: - Geographic Area - Civic Location - Positioning methods |
| ProblemDetails | O | 1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application errors: - POSITIONING_DENIED - UNSPECIFIED - UNSUPPORTED_BY_UE See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 1 | 500 Internal Server Error | The "cause" attribute may be used to indicate the following application error: - POSITIONING_FAILED See table 6.1.7.3-1 for the description of these errors. |
| ProblemDetails | O | 1 | 504 Gateway Timeout | The "cause" attribute may be used to indicate the following application error: - UNREACHABLE_USER See table 6.1.7.3-1 for the description of this error. |

#### 6.1.4.3	Operation: cancel-location

##### 6.1.4.3.1	Description

This clause describes the custom operation and what it is used for.

##### 6.1.4.3.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 6.1.4.3.2-1 and table 6.1.4.3.2-2.

Table 6.1.4.3.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CancelLocData | M | 1 | The information used to cancel location. |

Table 6.1.4.3.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful cancellation of location. |
| ProblemDetails | O | 1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application errors: - UNSPECIFIED - LOCATION_SESSION_UNKNOWN See table 6.1.7.3-1 for the description of this error. |

#### 6.1.4.4	Operation: location-context-transfer

##### 6.1.4.4.1	Description

This clause will describe the custom operation and what it is used for.

##### 6.1.4.4.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 6.1.4.4.2-1 and table 6.1.4.4.2-2.

Table 6.1.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| LocContextData | M | 1 | Input parameters to the "Location Context Transfer" operation |

Table 6.1.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful transfer of the location context. |
| ProblemDetails | O | 1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application errors: - UNSPECIFIED - LOCATION_TRANSFER_NOT SUPPORTED - INSUFFICIENT_RESOURCES - EVENT_REPORT_UNRECOGNIZED See table 6.1.7.3-1 for the description of this error. |

### 6.1.5	Notifications

#### 6.1.5.1	EventNotify

##### 6.1.5.1.1	Description

The EventNotify operation is used to notify the occurrence of periodic or triggered location event for a target UE to a consumer NF (e.g. GMLC).

##### 6.1.5.1.2	Notification Definition

Callback URI: {hgmlcCallBackURI}

See clause 5.2.2.1.2 for the description of how the LMF obtains the Callback URI of the NF Service Consumer (e.g. GMLC).

##### 6.1.5.1.3	Notification Standard Methods

###### 6.1.5.1.3.1	POST

This method sends a Location event notify to the NF Service Consumer.

This method shall support the request and response data structures and response codes specified in table 6.1.5.1.3.1-1 and table 6.1.5.1.3.1-2.

Table 6.1.5.1.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| EventNotifyData | M | 1 | Input parameters to the "Location Event Notify" operation |

Table 6.1.5.1.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | This case represents successful notification of the event. |
| ProblemDetails | O | 1 | 403 Forbidden | The "cause" attribute may be used to indicate the following application errors: - UNSPECIFIED - LOCATION_SESSION_UNKNOWN See table 6.1.7.3-1 for the description of this error. |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nlmf_Location service based interface protocol.

Table 6.1.6.1-1: Nlmf_Location specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| InputData | 6.1.6.2.2 |  |
| LocationData | 6.1.6.2.3 |  |
| GeographicalCoordinates | 6.1.6.2.4 |  |
| GeographicArea | 6.1.6.2.5 |  |
| Point | 6.1.6.2.6 |  |
| PointUncertaintyCircle | 6.1.6.2.7 |  |
| PointUncertaintyEllipse | 6.1.6.2.8 |  |
| Polygon | 6.1.6.2.9 |  |
| PointAltitude | 6.1.6.2.10 |  |
| PointAltitudeUncertainty | 6.1.6.2.11 |  |
| EllipsoidArc | 6.1.6.2.12 |  |
| LocationQoS | 6.1.6.2.13 |  |
| CivicAddress | 6.1.6.2.14 |  |
| PositioningMethodAndUsage | 6.1.6.2.15 |  |
| GnssPositioningMethodAndUsage | 6.1.6.2.16 |  |
| VelocityEstimate | 6.1.6.2.17 |  |
| HorizontalVelocity | 6.1.6.2.18 |  |
| HorizontalWithVerticalVelocity | 6.1.6.2.19 |  |
| HorizontalVelocityWithUncertainty | 6.1.6.2.20 |  |
| HorizontalWithVerticalVelocityAndUncertainty | 6.1.6.2.21 |  |
| UncertaintyEllipse | 6.1.6.2.22 |  |
| UeLcsCapability | 6.1.6.2.23 | Indicates the LCS capability supported by the UE. |
| PeriodicEventInfo | 6.1.6.2.24 |  |
| AreaEventInfo | 6.1.6.2.25 |  |
| ReportingArea | 6.1.6.2.26 |  |
| MotionEventInfo | 6.1.6.2.27 |  |
| ReportingAccessTypes | 6.1.6.2.28 |  |
| CancelLocData | 6.1.6.2.29 |  |
| LocContextData | 6.1.6.2.30 |  |
| EventReportMessage | 6.1.6.2.31 |  |
| EventReportingStatus | 6.1.6.2.32 |  |
| UELocationInfo | 6.1.6.2.33 |  |
| EventNotifyData | 6.1.6.2.34 |  |
| Altitude | 6.1.6.3.2 |  |
| Angle | 6.1.6.3.2 |  |
| Uncertainty | 6.1.6.3.2 |  |
| Orientation | 6.1.6.3.2 |  |
| Confidence | 6.1.6.3.2 |  |
| Accuracy | 6.1.6.3.2 |  |
| InnerRadius | 6.1.6.3.2 |  |
| CorrelationID | 6.1.6.3.2 |  |
| AgeOfLocationEstimate | 6.1.6.3.2 |  |
| HorizontalSpeed | 6.1.6.3.2 |  |
| VerticalSpeed | 6.1.6.3.2 |  |
| SpeedUncertainty | 6.1.6.3.2 |  |
| BarometricPressure | 6.1.6.3.2 |  |
| LcsServiceType | 6.1.6.3.2 |  |
| LdrReference | 6.1.6.3.2 |  |
| ReportingAmount | 6.1.6.3.2 |  |
| ReportingInterval | 6.1.6.3.2 |  |
| MinimumInterval | 6.1.6.3.2 |  |
| MaximumInterval | 6.1.6.3.2 |  |
| SamplingInterval | 6.1.6.3.2 |  |
| ReportingDuration | 6.1.6.3.2 |  |
| LinearDistance | 6.1.6.3.2 |  |
| LMFIdentification | 6.1.6.3.2 |  |
| EventReportCounter | 6.1.6.3.2 |  |
| EventReportDuration | 6.1.6.3.2 |  |
| ExternalClientType | 6.1.6.3.3 |  |
| SupportedGADShapes | 6.1.6.3.4 |  |
| ResponseTime | 6.1.6.3.5 |  |
| PositioningMethod | 6.1.6.3.6 |  |
| GnssMethod | 6.1.6.3.7 |  |
| GnssId | 6.1.6.3.8 |  |
| Usage | 6.1.6.3.9 |  |
| LcsPriority | 6.1.6.3.10 |  |
| VelocityRequested | 6.1.6.3.11 |  |
| AccuracyFulfilmentIndicator | 6.1.6.3.12 |  |
| VerticalDirection | 6.1.6.3.13 |  |
| LdrType | 6.1.6.3.14 |  |
| ReportingAreaType | 6.1.6.3.15 |  |
| OccurrenceInfo | 6.1.6.3.16 |  |
| ReportingAccessType | 6.1.6.3.17 |  |
| EventClass | 6.1.6.3.18 |  |
| ReportedEventType | 6.1.6.3.19 |  |
| TerminationCause | 6.1.6.3.20 |  |
| LcsQosClass | 6.1.6.3.21 |  |

Table 6.1.6.1-2 specifies data types re-used by the Nlmf_Location service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf service based interface.

Table 6.1.6.1-2: Nlmf_Location re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Supi | 3GPP TS 29.571 [8] |  |
| Pei | 3GPP TS 29.571 [8] |  |
| Gpsi | 3GPP TS 29.571 [8] |  |
| Ecgi | 3GPP TS 29.571 [8] |  |
| Ncgi | 3GPP TS 29.571 [8] |  |
| NfInstanceId | 3GPP TS 29.571 [8] |  |
| Uri | 3GPP TS 29.571 [8] |  |
| RefToBinaryData | 3GPP TS 29.571 [8] |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: InputData

Table 6.1.6.2.2-1: Definition of type InputData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| externalClientType | ExternalClientType | O | 0..1 |  |
| correlationID | CorrelationID | O | 0..1 |  |
| amfId | NfInstanceId | O | 0..1 | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |
| locationQoS | LocationQoS | O | 0..1 |  |
| supportedGADShapes | array(SupportedGADShapes) | O | 1..N |  |
| supi | Supi | O | 0..1 |  |
| pei | Pei | O | 0..1 |  |
| gpsi | Gpsi | O | 0..1 |  |
| ecgi | Ecgi | O | 0..1 | When present, this IE shall indicate the identifier of the E-UTRAN cell serving the UE. (NOTE 2) |
| ncgi | Ncgi | O | 0..1 | When present, this IE shall indicate the identifier of the NR cell serving the UE. (NOTE 2) |
| priority | LcsPriority | O | 0..1 |  |
| velocityRequested | VelocityRequested | O | 0..1 |  |
| ueLcsCap | UeLcsCapability | O | 0..1 | When present, this IE shall indicate the LCS capability supported by the UE. |
| lcsServiceType | LcsServiceType | O | 0..1 | The LCS service type |
| ldrType | LdrType | O | 0..1 | The type of LDR |
| hgmlcCallBackURI | Uri | O | 0..1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | O | 0..1 | LDR Reference Number |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting |
| reportingAccessTypes | ReportingAccessTypes | O | 0..1 | Allowed access types for event reporting |
| NOTE 1: At least one of the attributes defined in this table shall be present in the InputData structure. NOTE 2: Attribute "ecgi" and "ncgi" shall not be present at the same time. |  |  |  |  |

##### 6.1.6.2.3	Type: LocationData

Table 6.1.6.2.3-1: Definition of type LocationData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| locationEstimate | GeographicArea | M | 1 | For a request for triggered location where location estimates are not required, the location estimate can be based on current serving cell. |
| accuracyFulfilmentIndicator | AccuracyFulfilmentIndicator | O | 0..1 |  |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 |  |
| velocityEstimate | VelocityEstimate | O | 0..1 |  |
| civicAddress | CivicAddress | O | 0..1 |  |
| positioningDataList | array(PositioningMethodAndUsage) | O | 1..N |  |
| gnssPositioningDataList | array(GnssPositioningMethodAndUsage) | O | 1..N |  |
| ecgi | Ecgi | O | 0..1 |  |
| ncgi | Ncgi | O | 0..1 |  |
| altitude | Altitude | O | 0..1 | Altitude of the positioning estimate. When the shape used in "locationEstimate" supports conveying the altitude parameter, this IE shall be absent. |
| barometricPressure | BarometricPressure | O | 0..1 | If present, this IE contains the barometric pressure measurement as reported by the target UE. |
| servingLMFidentification | LMFIdentification | O | 0..1 |  |

##### 6.1.6.2.4	Type: GeographicalCoordinates

Table 6.1.6.2.4-1: Definition of type GeographicalCoordinates

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| lon | number | M | 1 | Longitude (Double-precision float value): Minimum: -180 Maximum: 180 |
| lat | number | M | 1 | Latitude (Double-precision float value): Minimum: -90 Maximum: 90 |

##### 6.1.6.2.5	Type: GeographicArea

Table 6.1.6.2.5-1: Definition of type GeographicArea as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| Point | 1 | Geographical area consisting of a single point, represented by its longitude and latitude. |
| PointUncertaintyCircle | 1 | Geographical area consisting of a point and an uncertainty value. |
| PointUncertaintyEllipse | 1 | Geographical area consisting of a point, plus an uncertainty ellipse and a confidence value. |
| Polygon | 1 | Geographical area consisting of a list of points (between 3 to 15 points). |
| PointAltitude | 1 | Geographical area consisting of a point and an altitude value. |
| PointAltitudeUncertainty | 1 | Geographical area consisting of a point, an altitude value and an uncertainty value. |
| EllipsoidArc | 1 | Geographical are consisting of an ellipsoid arc. |

##### 6.1.6.2.6	Type: Point

Table 6.1.6.2.6-1: Definition of type Point

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "POINT". |
| point | GeographicalCoordinates | M | 1 |  |

##### 6.1.6.2.7	Type: PointUncertaintyCircle

Table 6.1.6.2.7-1: Definition of type PointUncertaintyCircle

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "POINT_UNCERTAINTY_CIRCLE". |
| point | GeographicalCoordinates | M | 1 |  |
| uncertainty | Uncertainty | M | 1 |  |

##### 6.1.6.2.8	Type: PointUncertaintyEllipse

Table 6.1.6.2.8-1: Definition of type PointUncertaintyEllipse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "POINT_UNCERTAINTY_ELLIPSE". |
| point | GeographicalCoordinates | M | 1 |  |
| uncertaintyEllipse | UncertaintyEllipse | M | 1 |  |
| confidence | Confidence | M | 1 |  |

##### 6.1.6.2.9	Type: Polygon

Table 6.1.6.2.9-1: Definition of type Polygon

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "POLYGON". |
| pointList | array(GeographicalCoordinates) | M | 3..15 | Array with up to15 items, where each item is a "point". |

##### 6.1.6.2.10	Type: PointAltitude

Table 6.1.6.2.10-1: Definition of type PointAltitude

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "POINT_ALTITUDE". |
| point | GeographicalCoordinates | M | 1 |  |
| altitude | Altitude | M | 1 |  |

##### 6.1.6.2.11	Type: PointAltitudeUncertainty

Table 6.1.6.2.11-1: Definition of type PointAltitudeUncertainty

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "POINT_ALTITUDE_UNCERTAINTY". |
| point | GeographicalCoordinates | M | 1 |  |
| altitude | Altitude | M | 1 |  |
| uncertaintyEllipse | UncertaintyEllipse | M | 1 |  |
| uncertaintyAltitude | Uncertainty | M | 1 |  |
| confidence | Confidence | M | 1 |  |

##### 6.1.6.2.12	Type: EllipsoidArc

Table 6.1.6.2.12-1: Definition of type EllipsoidArc

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| shape | SupportedGADShapes | M | 1 | It shall take the value "ELLIPSOID_ARC". |
| point | GeographicalCoordinates | M | 1 |  |
| innerRadius | InnerRadius | M | 1 |  |
| uncertaintyRadius | Uncertainty | M | 1 |  |
| offsetAngle | Angle | M | 1 |  |
| includedAngle | Angle | M | 1 |  |
| confidence | Confidence | M | 1 |  |

##### 6.1.6.2.13	Type: LocationQoS

Table 6.1.6.2.13-1: Definition of type LocationQoS

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hAccuracy | Accuracy | O | 0..1 | Horizontal accuracy |
| vAccuracy | Accuracy | O | 0..1 | Vertical accuracy |
| vertRequested | boolean | O | 0..1 | Vertical accuracy requested (yes/no) |
| responseTime | ResponseTime | O | 0..1 | Low delay vs. Delay tolerant |
| lcsQosClass | LcsQosClass | C | 0..1 | LCS QoS Class, see clause 4.1b of 3GPP TS 23.273 [19]. This IE shall be absent if neither hAccuracy nor vAccuracy is included. |

##### 6.1.6.2.14	Type: CivicAddress

Table 6.1.6.2.14-1: Definition of type CivicAddress

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| country | string | M | 1 | The two-letter ISO 3166 country code in capital ASCII letters, e.g., DE or US IETF RFC 4776 [6] |
| A1 | string | O | 0..1 | National subdivisions (state, canton, region, province, prefecture) IETF RFC 4776 [6] |
| A2 | string | O | 0..1 | County, parish, gun (JP), district (IN) IETF RFC 4776 [6] |
| A3 | string | O | 0..1 | City, township, shi (JP) IETF RFC 4776 [6] |
| A4 | string | O | 0..1 | City division, borough, city district, ward, chou (JP) IETF RFC 4776 [6] |
| A5 | string | O | 0..1 | Neighbourhood, block IETF RFC 4776 [6] |
| A6 | string | O | 0..1 | Group of streets below the neighbourhood level IETF RFC 4776 [6] |
| PRD | string | O | 0..1 | Leading street direction IETF RFC 4776 [6] |
| POD | string | O | 0..1 | Trailing street suffix IETF RFC 4776 [6] |
| STS | string | O | 0..1 | Street suffix or type IETF RFC 4776 [6] |
| HNO | string | O | 0..1 | House number IETF RFC 4776 [6] |
| HNS | string | O | 0..1 | House number suffix IETF RFC 4776 [6] |
| LMK | string | O | 0..1 | Landmark or vanity address IETF RFC 4776 [6] |
| LOC | string | O | 0..1 | Additional location information IETF RFC 4776 [6] |
| NAM | string | O | 0..1 | Name (residence and office occupant) IETF RFC 4776 [6] |
| PC | string | O | 0..1 | Postal/zip code IETF RFC 4776 [6] |
| BLD | string | O | 0..1 | Building (structure) IETF RFC 5139 [7] |
| UNIT | string | O | 0..1 | Unit (apartment, suite) IETF RFC 5139 [7] |
| FLR | string | O | 0..1 | Floor IETF RFC 4776 [6] |
| ROOM | string | O | 0..1 | Room IETF RFC 5139 [7] |
| PLC | string | O | 0..1 | Place-type IETF RFC 5139 [7] |
| PCN | string | O | 0..1 | Postal community name IETF RFC 5139 [7] |
| POBOX | string | O | 0..1 | Post office box (P.O. box) IETF RFC 5139 [7] |
| ADDCODE | string | O | 0..1 | Additional code IETF RFC 5139 [7] |
| SEAT | string | O | 0..1 | Seat (desk, cubicle, workstation) IETF RFC 5139 [7] |
| RD | string | O | 0..1 | Primary road or street IETF RFC 5139 [7] |
| RDSEC | string | O | 0..1 | Road clause IETF RFC 5139 [7] |
| RDBR | string | O | 0..1 | Road branch IETF RFC 5139 [7] |
| RDSUBBR | string | O | 0..1 | Road sub-branch IETF RFC 5139 [7] |
| PRM | string | O | 0..1 | Road pre-modifier IETF RFC 5139 [7] |
| POM | string | O | 0..1 | Road post-modifier IETF RFC 5139 [7] |

EXAMPLE:	The above structure follows the same label naming as in the XML schema shown in IETF RFC 5139 [7]. The same example shown in XML in that RFC, in chapter 5, would be equivalent to the following JSON document:

{

"country": "AU",

"A1": "NSW",

"A3": "Wollongong",

"A4": "North Wollongong",

"RD": "Flinders",

"STS": "Street",

"RDBR": "Campbell Street",

"LMK": "Gilligan's Island",

"LOC": "Corner",

"NAM": "Video Rental Store",

"PC": "2500",

"ROOM": "Westerns and Classics",

"PLC": "store",

"POBOX": "Private Box 15"

}

##### 6.1.6.2.15	Type: PositioningMethodAndUsage

Table 6.1.6.2.15-1: Definition of type PositioningMethodAndUsage

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| method | PositioningMethod | M | 1 |  |
| mode | PositioningMode | M | 1 |  |
| usage | Usage | M | 1 |  |

##### 6.1.6.2.16	Type: GnssPositioningMethodAndUsage

Table 6.1.6.2.16-1: Definition of type GnssPositioningMethodAndUsage

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| mode | PositioningMode | M | 1 |  |
| gnss | GnssId | M | 1 |  |
| usage | Usage | M | 1 |  |

##### 6.1.6.2.17	Type: VelocityEstimate

Table 6.1.6.2.17-1: Definition of type VelocityEstimate as a list of mutually exclusive alternatives

| Data type | Cardinality | Description |
| --- | --- | --- |
| HorizontalVelocity | 1 | Velocity estimate including horizontal speed and bearing. |
| HorizontalWithVerticalVelocity | 1 | Velocity estimate including horizontal speed and bearing, and also vertical speed and vertical direction. |
| HorizontalVelocityWithUncertainty | 1 | Velocity estimate including horizontal speed and bearing; it also includes an uncertainty value. |
| HorizontalWithVerticalVelocityAndUncertainty | 1 | Velocity estimate including horizontal speed and bearing, and also vertical speed and vertical direction; it also includes uncertainty value for horizontal and vertical speeds. |

##### 6.1.6.2.18	Type: HorizontalVelocity

Table 6.1.6.2.18-1: Definition of type HorizontalVelocity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hSpeed | HorizontalSpeed | M | 1 | Horizontal speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angle in degrees, measured clockwise from North. |

##### 6.1.6.2.19	Type: HorizontalWithVerticalVelocity

Table 6.1.6.2.19-1: Definition of type HorizontalWithVerticalVelocity

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hSpeed | HorizontalSpeed | M | 1 | Horizontal speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angel in degrees, measured clockwise from North. |
| vSpeed | VerticalSpeed | M | 1 | Vertical Seed in kilometres per hour. |
| vDirection | VerticalDirection | M | 1 | Vertical Direction: upward or downward. |

##### 6.1.6.2.20	Type: HorizontalVelocityWithUncertainty

Table 6.1.6.2.20-1: Definition of type HorizontalVelocityWithUncertainty

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hSpeed | HorizontalSpeed | M | 1 | Speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angel in degrees, measured clockwise from North. |
| uncertainty | SpeedUncertainty | M | 1 | Uncertainty of horizontal speed in kilometres per hour. |

##### 6.1.6.2.21	Type: HorizontalWithVerticalVelocityAndUncertainty

Table 6.1.6.2.21-1: Definition of type HorizontalWithVerticalVelocityAndUncertainty

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hspeed | HorizontalSpeed | M | 1 | Speed in kilometres per hour. |
| bearing | Angle | M | 1 | Bearing angel in degrees, measured clockwise from North. |
| vSpeed | VerticalSpeed | M | 1 | Vertical Seed in kilometres per hour. |
| vDirection | VerticalDirection | M | 1 | Vertical Direction: upwards or downwards. |
| hUncertainty | SpeedUncertainty | M | 1 | Uncertainty of horizontal speed in kilometres per hour. |
| vUncertainty | SpeedUncertainty | M | 1 | Uncertainty of vertical speed in kilometres per hour. |

##### 6.1.6.2.22	Type: UncertaintyEllipse

Table 6.1.6.2.22-1: Definition of type UncertaintyEllipse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| semiMajor | Uncertainty | M | 1 |  |
| semiMinor | Uncertainty | M | 1 |  |
| orientationMajor | Orientation | M | 1 |  |

##### 6.1.6.2.23	Type: UeLcsCapability

Table 6.1.6.2.x-1: Definition of type UeLcsCapability

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| lppSupport | boolean | O | 0..1 | Indicates whether the UE supports LPP or not. - true (default): LPP supported by the UE - false: LPP not supported by the UE |
| ciotOptimisation | boolean | O | 0..1 | Indicates whether the UE supports and is allowed to use Control Plane CIoT 5GS Optimisation to send an event report for periodic or triggered location or not. Refer to 3GPP TS 23.273 [19] clause 6.7 for more detail. - true: Control Plane CIoT 5GS Optimisation is supported by the UE and allowed - false (default): Control Plane CIoT 5GS Optimisation not supported by the UE or not allowed |

##### 6.1.6.2.24	Type: PeriodicEventInfo

Table 6.1.6.2.24-1: Definition of type PeriodicEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reportingAmount | ReportingAmount | M | 1 | Number of event reports |
| reportingInterval | ReportingInterval | M | 1 | Interval of event reports |
| NOTE: reportingAmount x reportingInterval shall not exceed 8639999 (99 days, 23 hours, 59 minutes and 59 seconds) for compatibility with OMA MLP and RLP. |  |  |  |  |

##### 6.1.6.2.25	Type: AreaEventInfo

Table 6.1.6.2.25-1: Definition of type AreaEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| areaDefinition | array(ReportingArea) | M | 1..250 | One or more reporting areas |
| occurrenceInfo | OccurrenceInfo | O | 0..1 | One time only report indication |
| minimumInterval | MinimumInterval | C | 0..1 | Minimum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| maximumInterval | MaximumInterval | C | 0..1 | Maximum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| samplingInterval | SamplingInterval | O | 0..1 | Maximum time interval between consecutive evaluations by a UE of a trigger event. |
| reportingDuration | ReportingDuration | O | 0..1 | Maximum duration of event reporting. |
| reportingLocationReq | boolean | C | 0..1 | This IE shall be present and set to true if a location estimate is required for each event report. |

##### 6.1.6.2.26	Type: ReportingArea

Table 6.1.6.2.26-1: Definition of type ReportingArea

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| areaType | ReportingAreaType | M | 1 | Type of reporting area. |
| tai | Tai | C | 1 | TAI for EPS or 5GS. This IE shall be present if the reporting area type is EPS TAI or 5GS TAI. |
| ecgi | Ecgi | C | 1 | ECGI. This IE shall be present if the reporting area type is ECGI. |
| ncgi | Ncgi | C | 1 | NCGI. This IE shall be present if the reporting area type is NCGI. |
| NOTE: One of tai, ecgi or ncgi shall be included. |  |  |  |  |

##### 6.1.6.2.27	Type: MotionEventInfo

Table 6.1.6.2.27-1: Definition of type MotionEventInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| linearDistance | LinearDistance | M | 1 | Minimum linear (straight line) distance for motion event reports. |
| occurrenceInfo | OccurrenceInfo | O | 0..1 | One time only report indication |
| minimumInterval | MinimumInterval | C | 0..1 | Minimum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| maximumInterval | MaximumInterval | C | 0..1 | Maximum interval between event reports. This IE shall not be included if occurrenceInfo is present and set to one time event. |
| samplingInterval | SamplingInterval | O | 0..1 | Maximum time interval between consecutive evaluations by a UE of a trigger event. |
| reportingDuration | ReportingDuration | O | 0..1 | Maximum duration of event reporting. |
| reportingLocationReq | boolean | C | 0..1 | This IE shall be present and set to true if a location estimate is required for each event report. |

##### 6.1.6.2.28	Type: ReportingAccessTypes

Table 6.1.6.2.28-1: Definition of type ReportingAccessTypes

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| ReportingAccessTypes | Array(ReportingAccessType) | M | 1..N | This IE shall contain the allowed access types for event reporting. |

##### 6.1.6.2.29	Type: CancelLocData

Table 6.1.6.2.29-1: Definition of type CancelLocData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | M | 1 | LDR Reference |

##### 6.1.6.2.30	Type: LocContextData

Table 6.1.6.2.30-1: Definition of type LocContextData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfId | NfInstanceId | M | 1 | Indicates the AMF Instance serving the UE. LMF shall use the AMF Instance to forward LCS related N1/N2 messages to the UE/RAN. |
| locationQoS | LocationQoS | C | 0..1 | This IE shall contain the location QoS if available. |
| supportedGADShapes | array(SupportedGADShapes) | C | 0..N | This IE shall contain the supported GAD shapes if available. |
| Supi | Supi | C | 0..1 | This IE shall contain the SUPI if available. |
| Gpsi | Gpsi | C | 0..1 | This IE shall contain the GPSI if available. |
| ldrType | LdrType | M | 1 | The type of LDR |
| hgmlcCallBackURI | Uri | M | 1 | Callback URI of the H-GMLC |
| ldrReference | LdrReference | M | 1 | LDR Reference |
| periodicEventInfo | PeriodicEventInfo | C | 0..1 | Information for periodic event reporting |
| areaEventInfo | AreaEventInfo | C | 0..1 | Information for area event reporting |
| motionEventInfo | MotionEventInfo | C | 0..1 | Information for motion event reporting |
| eventReportMessage | EventReportMessage | M | 1 | Contains an embedded event report |
| eventReportingStatus | EventReportingStatus | O | 0..1 | Status of event reporting |
| ueLocationInfo | UELocationInfo | O | 0..1 | Location information for the target UE |
| NOTE: At least one of periodicEventInfo, areaEventInfo or motionEventInfo shall be present in the LocContextData structure. |  |  |  |  |

##### 6.1.6.2.31	Type: EventReportMessage

Table 6.1.6.2.31-1: Definition of EventReportMessage

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventClass | EventClass | M | 1 | This IE shall contain the event class for the message content specified in eventContent. |
| eventContent | RefToBinaryData | M | 1 | This IE shall reference the event report binary data corresponding to the eventClass. |

##### 6.1.6.2.32	Type: EventReportingStatus

Table 6.1.6.2.32-1: Definition of EventReportingStatus

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| eventReportCounter | EventReportCounter | O | 0..1 | This IE shall contain a count of event reports. |
| eventReportDuration | EventReportDuration | O | 0..1 | This IE shall contain the duration of event reporting. |

##### 6.1.6.2.33	Type: UELocationInfo

Table 6.1.6.2.33-1: Definition of UELocationInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| locationEstimate | GeographicArea | O | 0..1 | Previous location estimate for the target UE. |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | Age of previous location estimate. |
| velocityEstimate | VelocityEstimate | O | 0..1 | Previous velocity estimate for the target UE. |
| ageOfVelocityEstimate | AgeOfLocationEstimate | O | 0..1 | Age of previous velocity estimate. |

##### 6.4.6.2.34	Type: EventNotifyData

Table 6.4.6.2.34-1: Definition of type EventNotifyData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| reportedEventType | ReportedEventType | M | 1 | This IE shall contain the type of event being reported. |
| Supi | Supi | C | 0..1 | This IE shall contain the SUPI if available. |
| Gpsi | Gpsi | C | 0..1 | This IE shall contain the GPSI if available. |
| hgmlcCallBackURI | Uri | C | 0..1 | Callback URI of the H-GMLC (NOTE 1) |
| ldrReference | LdrReference | M | 1 | LDR Reference |
| locationEstimate | GeographicArea | O | 0..1 | If present, this IE shall contain an estimate of the location of the UE in universal coordinates and the accuracy of the estimate. |
| ageOfLocationEstimate | AgeOfLocationEstimate | O | 0..1 | If present, this IE shall contain an indication of how long ago the location estimate was obtained. |
| civicAddress | CivicAddress | O | 0..1 |  |
| positioningDataList | array(PositioningMethodAndUsage) | O | 0..N | If present, this IE shall indicate the usage of each non-GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |
| gnssPositioningDataList | array(GnssPositioningMethodAndUsage) | O | 0..N | If present, this IE shall indicate the usage of each GANSS positioning method that was attempted to determine the location estimate, either successfully or unsuccessfully. |
| servingLMFIdentification | LMFIdentification | C | 0..1 | This IE shall be included to identify an LMF which acts as a serving LMF if a serving LMF is used. |
| terminationCause | TerminationCause | C | 0..1 | This IE shall be included if event reporting has been terminated |
| NOTE 1: The hgmlcCallBackURI shall be included when the consumer NF is not the H-GMLC. |  |  |  |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| Altitude | number | Double-precision float value of the altitude, expressed in meters. Minimum = -32767. Maximum = 32767. |
| Angle | integer | Integer value of the angle, expressed in degrees. Minimum = 0. Maximum = 360. |
| Uncertainty | number | Float value of uncertainty, expressed in meters. Minimum = 0 |
| Orientation | integer | Integer value of the orientation angle, expressed in degrees. Minimum = 0. Maximum = 180. |
| Confidence | integer | Integer value of the confidence, expressed in percentage value. Minimum = 0. Maximum = 100. |
| Accuracy | number | Float value of accuracy, expressed in meters. Minimum = 0 |
| InnerRadius | integer | Integer value of the inner radius, expressed in meters. Minimum = 0. Maximum = 327675. |
| CorrelationID | string | LCS Correlation ID. The correlation ID shall be of a minimum length of 1 character and maximum length of 255 characters. |
| AgeOfLocationEstimate | integer | Integer value of the age of the location estimate, expressed in minutes. Minimum: 0. Maximum: 32767. |
| HorizontalSpeed | number | Float value of horizontal speed, expressed in kilometres per hour. Minimum = 0. Maximum = 2047. |
| VerticalSpeed | number | Float value of horizontal speed, expressed in kilometres per hour. Minimum = 0. Maximum = 255. |
| SpeedUncertainty | number | Float value of speed uncertainty, expressed in kilometres per hour. Minimum = 0. Maximum = 255. |
| BarometricPressure | integer | This IE specifies the measured uncompensated atmospheric pressure in units of Pascal (Pa). Minimum = 30000. Maximum = 115000. |
| LcsServiceType | integer | The LCS service type as defined in 3GPP TS 22.071 [17] and clause 17.7.8 of 3GPP TS 29.002 [18]. Minimum = 0. Maximum = 127. |
| LdrReference | String | LDR Reference encoded as a string of hexadecimal characters. The LdrReference shall be of a minimum length of 2 characters and maximum length of 510 characters. |
| ReportingAmount | integer | Number of required periodic event reports. Minimum = 1. Maximum = 8639999. |
| ReportingInterval | integer | Event reporting periodic interval in seconds. Minimum = 1. Maximum = 8639999. ReportingInterval x ReportingAmount shall not exceed 8639999. |
| MinimumInterval | integer | Minimum interval between event reports in seconds. Minimum = 1. Maximum = 32767. |
| MaximumInterval | integer | Maximum interval between event reports in seconds. Minimum = 1. Maximum = 86400. |
| SamplingInterval | integer | Maximum time interval between consecutive evaluations by a UE of a trigger event, in seconds. Minimum = 1. Maximum = 3600 |
| ReportingDuration | integer | Maximum duration of event reporting, in seconds. Minimum = 1. Maximum = 8640000. |
| LinearDistance | integer | The minimum straight line distance moved by a UE to trigger a motion event report, in meters. Minimum = 1. Maximum = 10000. |
| LMFIdentification | String | The serving LMF identification as defined in 3GPP TS 23.273 [19], encoded as a string of hexadecimal characters. |
| EventReportCounter | integer | Number of event reports received from the target UE. Minimum = 1. Maximum = 8640000. Note: the current event report is included in the count. |
| EventReportDuration | integer | Duration of event reporting, in seconds. Minimum = 0. Maximum = 8640000. Note: the duration starts when event reporting is activated in the UE and extends to the current time. |

##### 6.1.6.3.3	Enumeration: ExternalClientType

The enumeration ExternalClientType represents the different types of clients of the location service.

Table 6.1.6.3.3-1: Enumeration ExternalClientType

| Enumeration value | Description |
| --- | --- |
| "EMERGENCY_SERVICES" |  |
| "VALUE_ADDED_SERVICES" |  |
| "PLMN_OPERATOR_SERVICES" |  |
| "LAWFUL_INTERCEPT_SERVICES" |  |
| "PLMN_OPERATOR_BROADCAST_SERVICES" |  |
| "PLMN_OPERATOR_OM" |  |
| "PLMN_OPERATOR_ANONYMOUS_STATISTICS" |  |
| "PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT" |  |

##### 6.1.6.3.4	Enumeration: SupportedGADShapes

The enumeration SupportedGADShapes represents the different types, or shapes, of geographic areas supported by the system.

Table 6.1.6.3.4-1: Enumeration SupportedGADShapes

| Enumeration value | Description |
| --- | --- |
| "POINT" |  |
| "POINT_UNCERTAINTY_CIRCLE" |  |
| "POINT_UNCERTAINTY_ELLIPSE" |  |
| "POLYGON" |  |
| "POINT_ALTITUDE" |  |
| "POINT_ALTITUDE_UNCERTAINTY" |  |
| "ELLIPSOID_ARC" |  |

##### 6.1.6.3.5	Enumeration: ResponseTime

The enumeration ResponseTime represents the acceptable delay in the determination of the location of the UE.

Table 6.1.6.3.5-1: Enumeration ResponseTime

| Enumeration value | Description |
| --- | --- |
| "LOW_DELAY" |  |
| "DELAY_TOLERANT" |  |

##### 6.1.6.3.6	Enumeration: PositioningMethod

The enumeration PositioningMethod represents the method used to determine the location of the UE.

Table 6.1.6.3.6-1: Enumeration PositioningMethod

| Enumeration value | Description |
| --- | --- |
| "CELLID" |  |
| "ECID" |  |
| "OTDOA" |  |
| "BAROMETRIC_PRESSURE" |  |
| "WLAN" |  |
| "BLUETOOTH" |  |
| "MBS" |  |
| "MOTION_SENSOR" |  |

##### 6.1.6.3.7	Enumeration: PositioningMode

The enumeration PositioningMode represents the mode used to determine the location of the UE when a certain positioning method is used.

Table 6.1.6.3.7-1: Enumeration PositioningMode

| Enumeration value | Description |
| --- | --- |
| "UE_BASED" |  |
| "UE_ASSISTED" |  |
| "CONVENTIONAL" |  |

##### 6.1.6.3.8	Enumeration: GnssId

The enumeration GnssId represents the different GNSS systems.

Table 6.1.6.3.8-1: Enumeration GnssId

| Enumeration value | Description |
| --- | --- |
| "GPS" |  |
| "GALILEO" |  |
| "SBAS" |  |
| "MODERNIZED_GPS" |  |
| "QZSS" |  |
| "GLONASS" |  |

##### 6.1.6.3.9	Enumeration: Usage

The enumeration Usage represents the type of usage made of the location measurement from the UE.

Table 6.1.6.3.9-1: Enumeration Usage

| Enumeration value | Description |
| --- | --- |
| "UNSUCCESS" |  |
| "SUCCESS_RESULTS_NOT_USED" |  |
| "SUCCESS_RESULTS_USED_TO_VERIFY_LOCATION" |  |
| "SUCCESS_RESULTS_USED_TO_GENERATE_LOCATION" |  |
| "SUCCESS_METHOD_NOT_DETERMINED" |  |

##### 6.1.6.3.10	Enumeration: LcsPriority

The enumeration LcsPriority represents the priority of the LCS client.

Table 6.1.6.3.10-1: Enumeration LcsPriority

| Enumeration value | Description |
| --- | --- |
| "HIGHEST_PRIORITY" |  |
| "NORMAL_PRIORITY" |  |

##### 6.1.6.3.11	Enumeration: VelocityRequested

The enumeration VelocityRequested represents the indication of velocity requirement.

Table 6.1.6.3.11-1: Enumeration VelocityRequested

| Enumeration value | Description |
| --- | --- |
| "VELOCITY_IS_NOT_REQUESTED" |  |
| "VELOCITY_IS_REQUESTED" |  |

##### 6.1.6.3.12	Enumeration: AccuracyFulfilmentIndicator

The enumeration AccuracyFulfilmentIndicator represents whether the requested accuracy was fulfilled or not.

Table 6.1.6.3.12-1: Enumeration AccuracyFulfilmentIndicator

| Enumeration value | Description |
| --- | --- |
| "REQUESTED_ACCURACY_FULFILLED" |  |
| "REQUESTED_ACCURACY_NOT_FULFILLED" |  |

##### 6.1.6.3.13	Enumeration: VerticalDirection

The enumeration VerticalDirection represents the direction (upward/downward) of the vertical speed.

Table 6.1.6.3.13-1: Enumeration VerticalDirection

| Enumeration value | Description |
| --- | --- |
| "UPWARD" |  |
| "DOWNWARD" |  |

##### 6.1.6.3.14	Enumeration: LdrType

Table 6.1.6.3.14-1: Enumeration LdrType

| Enumeration value | Description |
| --- | --- |
| "UE_AVAILABLE" | UE available event |
| "PERIODIC" | Periodic event |
| "ENTERING_INTO_AREA" | Entering area event |
| "LEAVING_FROM_AREA" | Leaving area event |
| "BEING_INSIDE_AREA" | Being inside area event |
| "MOTION" | Motion event |

##### 6.1.6.3.15	Enumeration: ReportingAreaType

The enumeration ReportingAreaType indicates the type of a reporting area.

Table 6.1.6.3.15-1: Enumeration ReportingAreaType

| Enumeration value | Description |
| --- | --- |
| "EPS_TRACKING_AREA_IDENTITY" |  |
| " E-UTRAN_CELL_GLOBAL_IDENTIFICATION" |  |
| "5GS_TRACKING_AREA_IDENTITY" |  |
| " NR_CELL_GLOBAL_IDENTITY" |  |

##### 6.1.6.3.16	Enumeration: OccurrenceInfo

The enumeration OccurrenceInfo indicates whether event reporting is one time.

Table 6.1.6.3.16-1: Enumeration AreaType

| Enumeration value | Description |
| --- | --- |
| "ONE_TIME_EVENT" |  |
| "MULTIPLE_TIME_EVENT" |  |

##### 6.1.6.3.17	Enumeration: ReportingAccessType

The enumeration ReportingAccessType indicates an allowed access type for event reporting.

Table 6.1.6.3.17-1: Enumeration ReportingAccessType

| Enumeration value | Description |
| --- | --- |
| "NR" |  |
| "EUTRA_CONNECTED_TO_5GC" |  |
| "NON_3GPP_CONNECTED_TO_5GC" |  |

##### 6.1.6.3.18	Enumeration: EventClass

Table 6.1.6.3.18-1: Enumeration EventClass

| Enumeration value | Description |
| --- | --- |
| "SUPPLEMENTARY_SERVICES" | A supplementary services message containing an argument for an lcs-EventReport operation as defined in 3GPP TS 24.080 [20]. |

##### 6.1.6.3.19	Enumeration: ReportedEventType

Table 6.1.6.3.19-1: Enumeration ReportedEventType

| Enumeration value | Description |
| --- | --- |
| "PERIODIC_EVENT" | Periodic reporting event |
| "ENTERING_AREA_EVENT" | Entering area reporting event |
| "LEAVING_AREA_EVENT" | Leaving area reporting event |
| "BEING_INSIDE_AREA_EVENT" | Being inside area reporting event |
| "MOTION_EVENT" | Motion reporting event |
| "MAXIMUM_INTERVAL_EXPIRATION_EVENT" | Expiration of maximum reporting interval event |
| "LOCATION_CANCELLATION_EVENT" | Cancellation of location reporting event |

##### 6.1.6.3.20	Enumeration: TerminationCause

Table 6.1.6.3.20-1: Enumeration TerminationCause

| Enumeration value | Description |
| --- | --- |
| "TERMINATION_BY_UE" |  |
| "TERMINATION_BY_NETWORK" |  |
| "NORMAL_TERMINATION" |  |

##### 6.1.6.3.21	Enumeration: LcsQosClass

Table 6.1.6.3.21-1: Enumeration LcsQosClass

| Enumeration value | Description |
| --- | --- |
| "BEST_EFFORT" | Best Effort Class |
| "ASSURED" | Assured Class |

### 6.1.7	Error Handling

#### 6.1.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.1.7.3	Application Errors

The application errors defined for the Nlmf_Location service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| POSITIONING_DENIED | 403 Forbidden | The positioning procedure was denied. |
| UNSPECIFIED | 403 Forbidden | The request is rejected due to unspecified reasons. |
| UNSUPPORTED_BY_UE | 403 Forbidden | A request for periodic or triggered location is not supported by the UE. |
| LOCATION_SESSION_UNKNOWN | 403 Forbidden | The location context was not found. |
| LOCATION_TRANSFER_NOT_SUPPORTED | 403 Forbidden | Transfer of a location context is not supported |
| INSUFFICIENT_RESOURCES | 403 Forbidden | Insufficient resources for location context transfer |
| EVENT_REPORT_UNRECOGNIZED | 403 Forbidden | The event report is unrecognized or cannot be parsed. |
| POSITIONING_FAILED | 500 Internal Server Error | The positioning procedure failed. |
| UNREACHABLE_USER | 504 Gateway Timeout | The user could not be reached in order to perform positioning procedure. |

### 6.1.8	Security

As indicated in 3GPP TS 33.501 [9], the access to the Nlmf_Location API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [10]), using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [11]) plays the role of the authorization server.

If Oauth2 authorization is used, an NF Service Consumer, prior to consuming services offered by the Nlmf_Location API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [11], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nlmf_Location service.

The Nlmf_Location API defines scopes for OAuth2 authorization as specified in 3GPP TS 33.501 [9]; it defines a single scope consisting on the name of the service (i.e., "nlmf-loc"), and it does not define any additional scopes at resource or operation level.

## 6.2	Nlmf_Broadcast Service API

### 6.2.1	API URI

The Nlmf_Broadcast service shall use the Nlmf_Broadcast API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nlmf-broadcast".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.2.3.

### 6.2.2	Usage of HTTP

#### 6.2.2.1	General

HTTP/2, as defined in IETF RFC 7540 [12], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

HTTP messages and bodies for the Nlmf_Location service shall comply with the OpenAPI [14] specification contained in Annex A.

#### 6.2.2.2	HTTP Standard Headers

##### 6.2.2.2.1	General

##### 6.2.2.2.2	Content type

The following content types shall be supported:

-	JSON, as defined in IETF RFC 8259 [13], shall be used as content type of the HTTP bodies specified in the present specification as indicated in clause 5.4 of 3GPP TS 29.500 [4].

-	The Problem Details JSON Object (IETF RFC 7807 [15]). The use of the Problem Details JSON object in a HTTP response body shall be signalled by the content type "application/problem+json".

#### 6.2.2.3	HTTP custom headers

##### 6.2.2.3.1	General

The following HTTP custom headers shall be supported:

-	3gpp-Sbi-Message-Priority: See 3GPP TS 29.500 [4], clause 5.2.3.2.2.

This API does not define any new HTTP custom headers.

### 6.2.3	Resources

#### 6.2.3.1	Overview

The structure of the Resource URIs of the Nlmf_Broadcast service is shown in figure 6.2.3.1-1.

Figure 6.2.3.1-1: Resource URI structure of the Nlmf_Broadcast APITable 6.2.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.2.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| CipheringKeyData (Custom operation) | {apiRoot}/nlmf-broadcast/<apiVersion>/cipher-key-data | cipher-key-data (POST) |  |

### 6.2.4	Custom Operations without associated resources

#### 6.2.4.1	Overview

Table 6.2.4.1-1: Custom operations without associated resources

| Custom operation URI | Mapped HTTP method | Description |
| --- | --- | --- |
| {apiRoot}/nlmf-broadcast/<apiVersion>/cipher-key-data | POST |  |

#### 6.2.4.4	Operation: cipher-key-data

##### 6.2.4.4.1	Description

This clause describes the custom operation and what it is used for.

##### 6.2.4.4.2	Operation Definition

This operation shall support the request and response data structures and response codes specified in table 6.2.4.4.2-1 and table 6.2.4.4.2-2.

Table 6.2.4.4.2-1: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CipherRequestData | M | 1 | Input parameters to the "Ciphering Key Data" operation |

Table 6.2.4.4.2-2: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CipherResponseData | M | 1 | 200 OK | This case represents a successful request for ciphering key data. Upon success, a response body is returned indicating whether the LMF has ciphering key data. The ciphering key data is returned separately in a CipheringKeyData notification. |
| ProblemDetails | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application errors: - UNSPECIFIED - BROADCAST_CIPHERING_KEYS_NOT_SUPPORTED See table 6.2.7.3-1 for the description of this error. |

### 6.2.5	Notifications

#### 6.2.5.1	CipheringKeyData

##### 6.2.5.1.1	Description

The CipheringKeyData operation is used to notify the occurrence of new ciphering key information to a consumer NF (e.g. AMF).

##### 6.2.5.1.2	Notification Definition

Callback URI: {amfCallBackURI}

See clause 5.3.2.2.2 for the description of how the LMF obtains the Callback URI of the NF Service Consumer (i.e. AMF).

##### 6.2.5.1.3	Notification Standard Methods

###### 6.2.5.1.3.1	POST

This method sends a ciphering key data notify to the NF Service Consumer.

This method shall support the request and response data structures and response codes specified in table 6.2.5.1.3.1-1 and table 6.2.5.1.3.1-2.

Table 6.2.5.1.3.1-1: Data structures supported by the POST Request Body

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| CipheringKeyInfo | M | 1 | Input parameters to the "Ciphering Key Data" operation |

Table 6.2.5.1.3.1-2: Data structures supported by the POST Response Body

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| CipheringKeyResponse | M | 1 | 200 OK | This case represents successful or partially successful storage of ciphering key information by the service consumer NF. A response body is returned containing the following parameters: - List of Ciphering Set IDs successfully stored - List of Ciphering Set IDs not successfully stored |
| ProblemDetails | M | 1 | 403 Forbidden | The "cause" attribute shall be set to one of the following application errors: - UNSPECIFIED - UNABLE_TO_STORE_CIPHERING_KEY_DATA See table 6.2.7.3-1 for the description of this error. |

### 6.2.6	Data Model

#### 6.2.6.1	General

This clause specifies the application data model supported by the API.

Table 6.2.6.1-1 specifies the data types defined for the Nlmf_Broadcast service based interface protocol.

Table 6.2.6.1-1: Nlmf_Broadcast specific Data Types

| Data type | Clause defined | Description |
| --- | --- | --- |
| CipheringKeyInfo | 6.2.6.2.2 |  |
| CipheringKeyResponse | 6.2.6.2.3 |  |
| CipheringDataSet | 6.2.6.2.4 |  |
| CipheringSetReport | 6.2.6.2.5 |  |
| CipherRequestData | 6.2.6.2.6 |  |
| CipherResponseData | 6.2.6.2.7 |  |
| CipheringSetID | 6.2.6.3.2 |  |
| CipheringKey | 6.2.6.3.2 |  |
| C0 | 6.2.6.3.2 |  |
| ValidityDuration | 6.2.6.3.2 |  |
| StorageOutcome | 6.2.6.3.3 |  |
| DataAvailability | 6.2.6.3.4 |  |

Table 6.2.6.1-2 specifies data types re-used by the Nlmf_Broadcast service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nlmf service based interface.

Table 6.2.6.1-2: Nlmf_Broadcast re-used Data Types

| Data type | Reference | Comments |
| --- | --- | --- |
| Binary | 3GPP TS 29.571 [8] |  |
| DateTime | 3GPP TS 29.571 [8] |  |
| Uri | 3GPP TS 29.571 [8] |  |

#### 6.2.6.2	Structured data types

##### 6.2.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.2.6.2.2	Type: CipheringKeyInfo

Table 6.2.6.2.2-1: Definition of type CipheringKeyInfo

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringData | array(CipheringDataSet) | M | 1..N | This IE contains one or more ciphering data sets, where each ciphering data set contains information for one ciphering key. |

##### 6.2.6.2.3	Type: CipheringKeyResponse

Table 6.2.6.2.3-1: Definition of type CipheringKeyResponse

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringDataReport | Array(CipheringSetReport) | O | 1..N | This IE indicates the ciphering data sets which were successfully stored or not stored. The absence of this IE indicates that all ciphering data sets were successfully stored. |

##### 6.2.6.2.4	Type: CipheringDataSet

Table 6.2.6.2.4-1: Definition of type CipheringDataSet

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringSetID | CipheringSetID | M | 1 | Identification of a ciphering data set |
| cipheringKey | CipheringKey | M | 1 | A ciphering key value |
| c0 | C0 | M | 1 | First component of the initial ciphering counter as defined in clause 7.4.2 of 3GPP TS 36.355 [21] |
| ltePosSibTypes | Binary | O | 0..1 | This IE contains a bitmap indicating the LTE positioning SIB types for which the ciphering data set is applicable: - a bit set to 0 indicates that the ciphering data set is not applicable to the corresponding LTE positioning SIB type - a bit set to 1 indicates that the ciphering data set is applicable to the corresponding LTE positioning SIB type The mapping of the bits to the LTE positioning SIB types is as follows: -- bit 8 in the first octet maps to positioning SIB Type 1-1 -- bit 7 in the first octet maps to positioning SIB Type 1-2 -- bit 6 in the first octet maps to positioning SIB Type 1-3 -- bit 5 in the first octet maps to positioning SIB Type 1-4 -- bit 4 in the first octet maps to positioning SIB Type 1-5 -- bit 3 in the first octet maps to positioning SIB Type 1-6 -- bit 2 in the first octet maps to positioning SIB Type 1-7 -- bit 1 in the first octet maps to positioning SIB Type 1-8 -- bit 8 in the second octet maps to positioning SIB Type 2-1 -- bit 7 in the second octet maps to positioning SIB Type 2-2 -- bit 6 in the second octet maps to positioning SIB Type 2-3 -- bit 5 in the second octet maps to positioning SIB Type 2-4 -- bit 4 in the second octet maps to positioning SIB Type 2-5 -- bit 3 in the second octet maps to positioning SIB Type 2-6 -- bit 2 in the second octet maps to positioning SIB Type 2-7 -- bit 1 in the second octet maps to positioning SIB Type 2-8 -- bit 8 in the third octet maps to positioning SIB Type 2-9 -- bit 7 in the third octet maps to positioning SIB Type 2-10 -- bit 6 in the third octet maps to positioning SIB Type 2-11 -- bit 5 in the third octet maps to positioning SIB Type 2-12 -- bit 4 in the third octet maps to positioning SIB Type 2-13 -- bit 3 in the third octet maps to positioning SIB Type 2-14 -- bit 2 in the third octet maps to positioning SIB Type 2-15 -- bit 1 in the third octet maps to positioning SIB Type 2-16 -- bit 8 in the fourth octet maps to positioning SIB Type 2-17 -- bit 7 in the fourth octet maps to positioning SIB Type 2-18 -- bit 6 in the fourth octet maps to positioning SIB Type 2-19 -- bit 5 in the fourth octet maps to positioning SIB Type 2-20 -- bit 4 in the fourth octet maps to positioning SIB Type 2-21 -- bit 3 in the fourth octet maps to positioning SIB Type 2-22 -- bit 2 in the fourth octet maps to positioning SIB Type 2-23 -- bit 1 in the fourth octet maps to positioning SIB Type 3-1 Any unassigned bits are spare and shall be coded as zero. Non-included bits shall be treated as being coded as zero. (NOTE 1) |
| nrPosSibTypes | Binary | O | 0..1 | This IE contains a bitmap indicating the NR positioning SIB types for which the ciphering data set is applicable: - a bit set to 0 indicates that the ciphering data set is not applicable to the corresponding NR positioning SIB type - a bit set to 1 indicates that the ciphering data set is applicable to the corresponding NR positioning SIB type The mapping of the bits to the NR positioning SIB types is as follows: -- bit 8 in the first octet maps to positioning SIB Type 1-1 -- bit 7 in the first octet maps to positioning SIB Type 1-2 -- bit 6 in the first octet maps to positioning SIB Type 1-3 -- bit 5 in the first octet maps to positioning SIB Type 1-4 -- bit 4 in the first octet maps to positioning SIB Type 1-5 -- bit 3 in the first octet maps to positioning SIB Type 1-6 -- bit 2 in the first octet maps to positioning SIB Type 1-7 -- bit 1 in the first octet maps to positioning SIB Type 1-8 -- bit 8 in the second octet maps to positioning SIB Type 2-1 -- bit 7 in the second octet maps to positioning SIB Type 2-2 -- bit 6 in the second octet maps to positioning SIB Type 2-3 -- bit 5 in the second octet maps to positioning SIB Type 2-4 -- bit 4 in the second octet maps to positioning SIB Type 2-5 -- bit 3 in the second octet maps to positioning SIB Type 2-6 -- bit 2 in the second octet maps to positioning SIB Type 2-7 -- bit 1 in the second octet maps to positioning SIB Type 2-8 -- bit 8 in the third octet maps to positioning SIB Type 2-9 -- bit 7 in the third octet maps to positioning SIB Type 2-10 -- bit 6 in the third octet maps to positioning SIB Type 2-11 -- bit 5 in the third octet maps to positioning SIB Type 2-12 -- bit 4 in the third octet maps to positioning SIB Type 2-13 -- bit 3 in the third octet maps to positioning SIB Type 2-14 -- bit 2 in the third octet maps to positioning SIB Type 2-15 -- bit 1 in the third octet maps to positioning SIB Type 2-16 -- bit 8 in the fourth octet maps to positioning SIB Type 2-17 -- bit 7 in the fourth octet maps to positioning SIB Type 2-18 -- bit 6 in the fourth octet maps to positioning SIB Type 2-19 -- bit 5 in the fourth octet maps to positioning SIB Type 2-20 -- bit 4 in the fourth octet maps to positioning SIB Type 2-21 -- bit 3 in the fourth octet maps to positioning SIB Type 2-22 -- bit 2 in the fourth octet maps to positioning SIB Type 2-23 -- bit 1 in the fourth octet maps to positioning SIB Type 3-2 -- bit 8 in the fifth octet maps to positioning SIB Type 3-3 -- bit 7 in the fifth octet maps to positioning SIB Type 3-4 Any unassigned bits are spare and shall be coded as zero. Non-included bits shall be treated as being coded as zero. (NOTE 1) |
| validityStartTime | DateTime | M | 1 | This IE contains the UTC time when the ciphering data set becomes valid. |
| validityDuration | ValidityDuration | M | 1 | The validity duration of the ciphering data set. |
| taiList | Binary | O | 0..1 | This IE contains the TAIs of the tracking areas for which the ciphering data set is applicable. It is encoded as octets 2 to n of the 5GS tracking area identity list IE specified in clause 9.11.3.9 of 3GPP TS 24.501 [22]. If this IE is omitted, the ciphering data set is valid in the entire PLMN. |
| NOTE 1: At least one of ltesibTypes IE and nrsibTypes IE shall be included. |  |  |  |  |

Editor's note [WI: 5G_eLCS, CR#0042]:	The positioning SIBs listed for the IEs lteSibTypes and nrSibTypes include positioning SIBs defined in TS 36.331 and TS 36.355 for E-UTRAN in Rel-15 plus new positioning SIBs for NG-RAN being discussed in RAN2 for Rel-16. These positioning SIBs will need to be aligned later with RAN2 agreements for Rel-16.

##### 6.2.6.2.5	Type: CipheringSetReport

Table 6.2.6.2.5-1: Definition of CipheringSetReport

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| cipheringSetID | CipheringSetID | M | 1 | Identification of a ciphering data set |
| storageOutcome | StorageOutcome | M | 1 | Indication of whether the ciphering data set was successfully stored or was not stored. |

##### 6.2.6.2.6	Type: CipherRequestData

Table 6.2.6.2.6-1: Definition of CipherRequestData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| amfCallBackURI | Uri | M | 1 | Callback URI of the NF Service Consumer |

##### 6.2.6.2.7	Type: CipherResponseData

Table 6.2.6.2.7-1: Definition of CipherResponseData

| Attribute name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| dataAvailability | DataAvailability | M | 1 | An indication of whether the LMF currently has ciphering key data applicable to the NF Service Consumer |

#### 6.2.6.3	Simple data types and enumerations

##### 6.2.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.2.6.3.2	Simple data types

The simple data types defined in table 6.2.6.3.2-1 shall be supported.

Table 6.2.6.3.2-1: Simple data types

| Type Name | Type Definition | Description |
| --- | --- | --- |
| CipheringSetID | integer | The ciphering set ID Minimum = 0. Maximum = 65535 |
| CipheringKey | Binary | A 128 bit ciphering key encoded using 16 octets |
| C0 | Binary | A 128 bit value for C0 encoded using 16 octets |
| ValidityDuration | integer | The validity duration in minutes. Minimum = 1. Maximum = 65535 |

##### 6.2.6.3.3	Enumeration: StorageOutcome

The enumeration StorageOutcome represents the outcome of cipher set data storage at the service consumer NF.

Table 6.2.6.3.3-1: Enumeration StorageOutcome

| Enumeration value | Description |
| --- | --- |
| "STORAGE_SUCCESSFUL" |  |
| "STORAGE_FAILED" |  |

##### 6.2.6.3.4	Enumeration: DataAvailability

The enumeration DataAvailability represents the availability of ciphering key data at an LMF.

Table 6.2.6.3.4-1: Enumeration DataAvailability

| Enumeration value | Description |
| --- | --- |
| "CIPHERING_KEY_DATA_AVAILABLE" |  |
| CIPHERING_KEY_DATA_NOT_AVAILABLE" |  |

### 6.2.7	Error Handling

#### 6.2.7.1	General

HTTP error handling shall be supported as specified in clause 5.2.4 of 3GPP TS 29.500 [4].

#### 6.2.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.2.7.3	Application Errors

The application errors defined for the Nlmf_Broadcast service are listed in table 6.2.7.3-1.

Table 6.2.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| UNSPECIFIED | 403 Forbidden | The request is rejected due to unspecified reasons. |
| UNABLE_TO_STORE_CIPHERING_KEY_DATA | 403 Forbidden | The service consumer NF was unable to store ciphering key data. |
| BROADCAST_CIPHERING_KEYS_NOT_SUPPORTED | 403 Forbidden | Ciphering keys for broadcast are not supported. |

### 6.2.8	Security

The Nlmf_Broadcast  API does not define service operations for which additional security is needed in this version of the specification.

Annex A (normative):	OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the Nlmf Service API. It consists of an OpenAPI 3.0.0 specification, in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on the public 3GPP file server in the following locations (see clause 5B of the 3GPP TR 21.900 [16] for further information):

-	https://www.3gpp.org/ftp/Specs/archive/OpenAPI/<Release>/, and

-	https://www.3gpp.org/ftp/Specs/<Plenary>/<Release>/OpenAPI/.

NOTE 2:	To fetch the OpenAPI specification file after CT#83 plenary meeting for Release 15 in the above links <Plenary> must be replaced with the date the CT Plenary occurs, in the form of year-month (yyyy-mm), e.g. for CT#83 meeting <Plenary> must be replaced with value "2019-03" and <Release> must be replaced with value "Rel-15".

## A.2	Nlmf_Location API

openapi: 3.0.0

info:

version: '1.1.0.alpha-2'

title: 'LMF Location'

description: |

LMF Location Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V16.1.0; 5G System; Location Management Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-loc/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nlmf-loc

paths:

/determine-location:

post:

summary: Determine Location of an UE

operationId: DetermineLocation

tags:

- Determine Location

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/InputData'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/LocationData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

EventNotify:

'{$request.body#/hgmlcCallBackURI}':

post:

requestBody:

description: UE Event Notification

content:

application/json:

schema:

$ref: '#/components/schemas/EventNotifyData'

responses:

'204':

description: Expected response to a valid notification

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/cancel-location:

post:

summary: request cancellation of periodic or triggered location

operationId: CancelLocation

tags:

- Cancel Location

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/CancelLocData'

required: true

responses:

'204':

description: Expected response to a successful cancellation

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

/location-context-transfer:

post:

summary: transfer context information for periodic or triggered location

operationId: LocationContextTransfer

tags:

- Location Context Transfer

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/LocContextData'

required: true

responses:

'204':

description: Expected response to successful location context transfer

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nlmf-loc: Access to the Nlmf_Location API

schemas:

#

# COMPLEX TYPES

#

InputData:

type: object

not:

required: [ ecgi, ncgi ]

properties:

externalClientType:

$ref: '#/components/schemas/ExternalClientType'

correlationID:

$ref: '#/components/schemas/CorrelationID'

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

pei:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Pei'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

priority:

$ref: '#/components/schemas/LcsPriority'

velocityRequested:

$ref: '#/components/schemas/VelocityRequested'

ueLcsCap:

$ref: '#/components/schemas/UeLcsCapability'

lcsServiceType:

$ref: '#/components/schemas/LcsServiceType'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

reportingAccessTypes:

$ref: '#/components/schemas/ReportingAccessTypes'

LocationData:

type: object

required:

- locationEstimate

properties:

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

accuracyFulfilmentIndicator:

$ref: '#/components/schemas/AccuracyFulfilmentIndicator'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

civicAddress:

$ref: '#/components/schemas/CivicAddress'

positioningDataList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: '#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

altitude:

$ref: '#/components/schemas/Altitude'

barometricPressure:

$ref: '#/components/schemas/BarometricPressure'

servingLMFIdentification:

$ref: '#/components/schemas/LMFIdentification'

GeographicArea:

anyOf:

- $ref: '#/components/schemas/Point'

- $ref: '#/components/schemas/PointUncertaintyCircle'

- $ref: '#/components/schemas/PointUncertaintyEllipse'

- $ref: '#/components/schemas/Polygon'

- $ref: '#/components/schemas/PointAltitude'

- $ref: '#/components/schemas/PointAltitudeUncertainty'

- $ref: '#/components/schemas/EllipsoidArc'

GADShape:

type: object

required:

- shape

properties:

shape:

$ref: '#/components/schemas/SupportedGADShapes'

discriminator:

propertyName: shape

mapping:

POINT: '#/components/schemas/Point'

POINT_UNCERTAINTY_CIRCLE: '#/components/schemas/PointUncertaintyCircle'

POINT_UNCERTAINTY_ELLIPSE: '#/components/schemas/PointUncertaintyEllipse'

POLYGON: '#/components/schemas/Polygon'

POINT_ALTITUDE: '#/components/schemas/PointAltitude'

POINT_ALTITUDE_UNCERTAINTY: '#/components/schemas/PointAltitudeUncertainty'

ELLIPSOID_ARC: '#/components/schemas/EllipsoidArc'

Point:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

PointUncertaintyCircle:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- uncertainty

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

uncertainty:

$ref: '#/components/schemas/Uncertainty'

PointUncertaintyEllipse:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- uncertaintyEllipse

- confidence

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

uncertaintyEllipse:

$ref: '#/components/schemas/UncertaintyEllipse'

confidence:

$ref: '#/components/schemas/Confidence'

Polygon:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- pointList

properties:

pointList:

$ref: '#/components/schemas/PointList'

PointAltitude:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- altitude

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

altitude:

$ref: '#/components/schemas/Altitude'

PointAltitudeUncertainty:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- altitude

- uncertaintyEllipse

- uncertaintyAltitude

- confidence

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

altitude:

$ref: '#/components/schemas/Altitude'

uncertaintyEllipse:

$ref: '#/components/schemas/UncertaintyEllipse'

uncertaintyAltitude:

$ref: '#/components/schemas/Uncertainty'

confidence:

$ref: '#/components/schemas/Confidence'

EllipsoidArc:

allOf:

- $ref: '#/components/schemas/GADShape'

- type: object

required:

- point

- innerRadius

- uncertaintyRadius

- offsetAngle

- includedAngle

- confidence

properties:

point:

$ref: '#/components/schemas/GeographicalCoordinates'

innerRadius:

$ref: '#/components/schemas/InnerRadius'

uncertaintyRadius:

$ref: '#/components/schemas/Uncertainty'

offsetAngle:

$ref: '#/components/schemas/Angle'

includedAngle:

$ref: '#/components/schemas/Angle'

confidence:

$ref: '#/components/schemas/Confidence'

GeographicalCoordinates:

type: object

required:

- lon

- lat

properties:

lon:

type: number

format: double

minimum: -180

maximum: 180

lat:

type: number

format: double

minimum: -90

maximum: 90

UncertaintyEllipse:

type: object

required:

- semiMajor

- semiMinor

- orientationMajor

properties:

semiMajor:

$ref: '#/components/schemas/Uncertainty'

semiMinor:

$ref: '#/components/schemas/Uncertainty'

orientationMajor:

$ref: '#/components/schemas/Orientation'

PointList:

type: array

items:

$ref: '#/components/schemas/GeographicalCoordinates'

minItems: 3

maxItems: 15

LocationQoS:

type: object

properties:

hAccuracy:

$ref: '#/components/schemas/Accuracy'

vAccuracy:

$ref: '#/components/schemas/Accuracy'

verticalRequested:

type: boolean

responseTime:

$ref: '#/components/schemas/ResponseTime'

lcsQosClass:

$ref: '#/components/schemas/LcsQosClass'

PositioningMethodAndUsage:

type: object

required:

- method

- mode

- usage

properties:

method:

$ref: '#/components/schemas/PositioningMethod'

mode:

$ref: '#/components/schemas/PositioningMode'

usage:

$ref: '#/components/schemas/Usage'

GnssPositioningMethodAndUsage:

type: object

required:

- mode

- gnss

- usage

properties:

mode:

$ref: '#/components/schemas/PositioningMode'

gnss:

$ref: '#/components/schemas/GnssId'

usage:

$ref: '#/components/schemas/Usage'

CivicAddress:

type: object

properties:

country:

type: string

A1:

type: string

A2:

type: string

A3:

type: string

A4:

type: string

A5:

type: string

A6:

type: string

PRD:

type: string

POD:

type: string

STS:

type: string

HNO:

type: string

HNS:

type: string

LMK:

type: string

LOC:

type: string

NAM:

type: string

PC:

type: string

BLD:

type: string

UNIT:

type: string

FLR:

type: string

ROOM:

type: string

PLC:

type: string

PCN:

type: string

POBOX:

type: string

ADDCODE:

type: string

SEAT:

type: string

RD:

type: string

RDSEC:

type: string

RDBR:

type: string

RDSUBBR:

type: string

PRM:

type: string

POM:

type: string

VelocityEstimate:

oneOf:

- $ref: '#/components/schemas/HorizontalVelocity'

- $ref: '#/components/schemas/HorizontalWithVerticalVelocity'

- $ref: '#/components/schemas/HorizontalVelocityWithUncertainty'

- $ref: '#/components/schemas/HorizontalWithVerticalVelocityAndUncertainty'

HorizontalVelocity:

type: object

required:

- hSpeed

- bearing

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

HorizontalWithVerticalVelocity:

type: object

required:

- hSpeed

- bearing

- vSpeed

- vDirection

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

vSpeed:

$ref: '#/components/schemas/VerticalSpeed'

vDirection:

$ref: '#/components/schemas/VerticalDirection'

HorizontalVelocityWithUncertainty:

type: object

required:

- hSpeed

- bearing

- hUncertainty

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

hUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

HorizontalWithVerticalVelocityAndUncertainty:

type: object

required:

- hSpeed

- bearing

- vSpeed

- vDirection

- hUncertainty

- vUncertainty

properties:

hSpeed:

$ref: '#/components/schemas/HorizontalSpeed'

bearing:

$ref: '#/components/schemas/Angle'

vSpeed:

$ref: '#/components/schemas/VerticalSpeed'

vDirection:

$ref: '#/components/schemas/VerticalDirection'

hUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

vUncertainty:

$ref: '#/components/schemas/SpeedUncertainty'

UeLcsCapability:

type: object

properties:

lppSupport:

type: boolean

default: true

ciotOptimisation:

type: boolean

default: false

PeriodicEventInfo:

type: object

required:

- reportingAmount

- reportingInterval

properties:

reportingAmount:

$ref: '#/components/schemas/ReportingAmount'

reportingInterval:

$ref: '#/components/schemas/ReportingInterval'

AreaEventInfo:

type: object

required:

- areaDefinition

properties:

areaDefinition:

type: array

items:

$ref: '#/components/schemas/ReportingArea'

minItems: 1

maxItems: 250

occurrenceInfo:

$ref: '#/components/schemas/OccurrenceInfo'

minimumInterval:

$ref: '#/components/schemas/MinimumInterval'

maximumInterval:

$ref: '#/components/schemas/MaximumInterval'

samplingInterval:

$ref: '#/components/schemas/SamplingInterval'

reportingDuration:

$ref: '#/components/schemas/ReportingDuration'

reportingLocationReq:

type: boolean

default: true

ReportingArea:

type: object

required:

- areaType

properties:

areaType:

$ref: '#/components/schemas/ReportingAreaType'

tai:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Tai'

ecgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ecgi'

ncgi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Ncgi'

MotionEventInfo:

type: object

required:

- linearDistance

properties:

linearDistance:

$ref: '#/components/schemas/LinearDistance'

occurrenceInfo:

$ref: '#/components/schemas/OccurrenceInfo'

minimumInterval:

$ref: '#/components/schemas/MinimumInterval'

maximumInterval:

$ref: '#/components/schemas/MaximumInterval'

samplingInterval:

$ref: '#/components/schemas/SamplingInterval'

reportingDuration:

$ref: '#/components/schemas/ReportingDuration'

reportingLocationReq:

type: boolean

default: true

ReportingAccessTypes:

type: array

items:

$ref: '#/components/schemas/ReportingAccessType'

minItems: 1

CancelLocData:

type: object

required:

- hgmlcCallBackURI

- ldrReference

properties:

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

LocContextData:

type: object

required:

- amfId

- ldrType

- hgmlcCallBackURI

- ldrReference

- eventReportMessage

properties:

amfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

locationQoS:

$ref: '#/components/schemas/LocationQoS'

supportedGADShapes:

type: array

items:

$ref: '#/components/schemas/SupportedGADShapes'

minItems: 1

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

ldrType:

$ref: '#/components/schemas/LdrType'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

periodicEventInfo:

$ref: '#/components/schemas/PeriodicEventInfo'

areaEventInfo:

$ref: '#/components/schemas/AreaEventInfo'

motionEventInfo:

$ref: '#/components/schemas/MotionEventInfo'

eventReportMessage:

$ref: '#/components/schemas/EventReportMessage'

eventReportingStatus:

$ref: '#/components/schemas/EventReportingStatus'

ueLocationInfo:

$ref: '#/components/schemas/UELocationInfo'

EventReportMessage:

type: object

required:

- eventClass

- eventContent

properties:

eventClass:

$ref: '#/components/schemas/EventClass'

eventContent:

$ref: 'TS29571_CommonData.yaml#/components/schemas/RefToBinaryData'

EventReportingStatus:

type: object

properties:

eventReportCounter:

$ref: '#/components/schemas/EventReportCounter'

eventReportDuration:

$ref: '#/components/schemas/EventReportDuration'

UELocationInfo:

type: object

properties:

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

velocityEstimate:

$ref: '#/components/schemas/VelocityEstimate'

ageOfVelocityEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

EventNotifyData:

type: object

required:

- reportedEventType

- ldrReference

properties:

reportedEventType:

$ref: '#/components/schemas/ReportedEventType'

supi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Supi'

gpsi:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Gpsi'

hgmlcCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

ldrReference:

$ref: '#/components/schemas/LdrReference'

locationEstimate:

$ref: '#/components/schemas/GeographicArea'

ageOfLocationEstimate:

$ref: '#/components/schemas/AgeOfLocationEstimate'

civicAddress:

$ref: '#/components/schemas/CivicAddress'

positioningDataList:

type: array

items:

$ref: '#/components/schemas/PositioningMethodAndUsage'

minItems: 1

gnssPositioningDataList:

type: array

items:

$ref: '#/components/schemas/GnssPositioningMethodAndUsage'

minItems: 1

servingLMFidentification:

$ref: '#/components/schemas/LMFIdentification'

terminationCause:

$ref: '#/components/schemas/TerminationCause'

#

#

# SIMPLE TYPES

#

Altitude:

type: number

format: double

minimum: -32767

maximum: 32767

Angle:

type: integer

minimum: 0

maximum: 360

Uncertainty:

type: number

format: float

minimum: 0

Orientation:

type: integer

minimum: 0

maximum: 180

Confidence:

type: integer

minimum: 0

maximum: 100

Accuracy:

type: number

format: float

minimum: 0

InnerRadius:

type: integer

format: int32

minimum: 0

maximum: 327675

CorrelationID:

type: string

minLength: 1

maxLength: 255

AgeOfLocationEstimate:

type: integer

minimum: 0

maximum: 32767

HorizontalSpeed:

type: number

format: float

minimum: 0

maximum: 2047

VerticalSpeed:

type: number

format: float

minimum: 0

maximum: 255

SpeedUncertainty:

type: number

format: float

minimum: 0

maximum: 255

BarometricPressure:

type: integer

minimum: 30000

maximum: 115000

LcsServiceType:

type: integer

minimum: 0

maximum: 127

LdrReference:

type: string

minLength: 2

maxLength: 510

ReportingAmount:

type: integer

minimum: 1

maximum: 8639999

ReportingInterval:

type: integer

minimum: 1

maximum: 8639999

MinimumInterval:

type: integer

minimum: 1

maximum: 32767

MaximumInterval:

type: integer

minimum: 1

maximum: 86400

SamplingInterval:

type: integer

minimum: 1

maximum: 3600

ReportingDuration:

type: integer

minimum: 1

maximum: 8640000

LinearDistance:

type: integer

minimum: 1

maximum: 10000

LMFIdentification:

type: string

EventReportCounter:

type: integer

minimum: 1

maximum: 8640000

EventReportDuration:

type: integer

minimum: 1

maximum: 8640000

#

# ENUMS

#

ExternalClientType:

anyOf:

- type: string

enum:

- EMERGENCY_SERVICES

- VALUE_ADDED_SERVICES

- PLMN_OPERATOR_SERVICES

- LAWFUL_INTERCEPT_SERVICES

- PLMN_OPERATOR_BROADCAST_SERVICES

- PLMN_OPERATOR_OM

- PLMN_OPERATOR_ANONYMOUS_STATISTICS

- PLMN_OPERATOR_TARGET_MS_SERVICE_SUPPORT

- type: string

SupportedGADShapes:

anyOf:

- type: string

enum:

- POINT

- POINT_UNCERTAINTY_CIRCLE

- POINT_UNCERTAINTY_ELLIPSE

- POLYGON

- POINT_ALTITUDE

- POINT_ALTITUDE_UNCERTAINTY

- ELLIPSOID_ARC

- type: string

ResponseTime:

anyOf:

- type: string

enum:

- LOW_DELAY

- DELAY_TOLERANT

- type: string

PositioningMethod:

anyOf:

- type: string

enum:

- CELLID

- ECID

- OTDOA

- BAROMETRIC_PRESSURE

- WLAN

- BLUETOOTH

- MBS

- MOTION_SENSOR

- type: string

PositioningMode:

anyOf:

- type: string

enum:

- UE_BASED

- UE_ASSISTED

- CONVENTIONAL

- type: string

GnssId:

anyOf:

- type: string

enum:

- GPS

- GALILEO

- SBAS

- MODERNIZED_GPS

- QZSS

- GLONASS

- type: string

Usage:

anyOf:

- type: string

enum:

- UNSUCCESS

- SUCCESS_RESULTS_NOT_USED

- SUCCESS_RESULTS_USED_TO_VERIFY_LOCATION

- SUCCESS_RESULTS_USED_TO_GENERATE_LOCATION

- SUCCESS_METHOD_NOT_DETERMINED

- type: string

LcsPriority:

anyOf:

- type: string

enum:

- HIGHEST_PRIORITY

- NORMAL_PRIORITY

- type: string

VelocityRequested:

anyOf:

- type: string

enum:

- VELOCITY_IS_NOT_REQUESTED

- VELOCITY_IS_REQUESTED

- type: string

AccuracyFulfilmentIndicator:

anyOf:

- type: string

enum:

- REQUESTED_ACCURACY_FULFILLED

- REQUESTED_ACCURACY_NOT_FULFILLED

- type: string

VerticalDirection:

type: string

enum:

- UPWARD

- DOWNWARD

LdrType:

anyOf:

- type: string

enum:

- UE_AVAILABLE

- PERIODIC

- ENTERING_INTO_AREA

- LEAVING_FROM_AREA

- BEING_INSIDE_AREA

- MOTION

- type: string

ReportingAreaType:

anyOf:

- type: string

enum:

- EPS_TRACKING_AREA_IDENTITY

- E-UTRAN_CELL_GLOBAL_IDENTIFICATION

- 5GS_TRACKING_AREA_IDENTITY

- NR_CELL_GLOBAL_IDENTITY

- type: string

OccurrenceInfo:

anyOf:

- type: string

enum:

- ONE_TIME_EVENT

- MULTIPLE_TIME_EVENT

- type: string

ReportingAccessType:

anyOf:

- type: string

enum:

- NR

- EUTRA_CONNECTED_TO_5GC

- NON_3GPP_CONNECTED_TO_5GC

- type: string

EventClass:

anyOf:

- type: string

enum:

- SUPPLEMENTARY_SERVICES

- type: string

ReportedEventType:

anyOf:

- type: string

enum:

- PERIODIC_EVENT

- ENTERING_AREA_EVENT

- LEAVING_AREA_EVENT

- BEING_INSIDE_AREA_EVENT

- MOTION_EVENT

- MAXIMUM_INTERVAL_EXPIRATION_EVENT

- LOCATION_CANCELLATION_EVENT

- type: string

TerminationCause:

anyOf:

- type: string

enum:

- TERMINATION_BY_UE

- TERMINATION_BY_NETWORK

- NORMAL_TERMINATION

- type: string

LcsQosClass:

anyOf:

- type: string

enum:

- BEST_EFFORT

- ASSURED

- type: string

## A.3	Nlmf_Broadcast API

openapi: 3.0.0

info:

version: '1.0.0.alpha-1'

title: 'LMF Broadcast'

description: |

LMF Broadcast Service.

© 2019, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.572 V16.1.0; 5G System; Location Management Services; Stage 3

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.572/'

servers:

- url: '{apiRoot}/nlmf-broadcast/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

paths:

/cipher-key-data:

post:

summary: Request ciphering key data

operationId: CipheringKeyData

tags:

- Request Ciphering Key Data

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/CipherRequestData'

required: true

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CipherResponseData'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

callbacks:

CipheringKeyData:

'{$request.body#/amfCallBackURI}':

post:

requestBody:

description: Ciphering Key Data Notification

content:

application/json:

schema:

$ref: '#/components/schemas/CipheringKeyInfo'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

$ref: '#/components/schemas/CipheringKeyResponse'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'411':

$ref: 'TS29571_CommonData.yaml#/components/responses/411'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'415':

$ref: 'TS29571_CommonData.yaml#/components/responses/415'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

'504':

$ref: 'TS29571_CommonData.yaml#/components/responses/504'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

schemas:

#

# COMPLEX TYPES

#

CipheringKeyInfo:

type: object

required:

- cipheringData

properties:

cipheringData:

type: array

items:

$ref: '#/components/schemas/CipheringDataSet'

minItems: 1

CipheringKeyResponse:

type: object

properties:

cipheringDataReport:

type: array

items:

$ref: '#/components/schemas/CipheringSetReport'

minItems: 1

CipheringDataSet:

type: object

required:

- cipheringSetID

- cipheringKey

- c0

- validityStartTime

- validityDuration

properties:

cipheringSetID:

$ref: '#/components/schemas/CipheringSetID'

cipheringKey:

$ref: '#/components/schemas/CipheringKey'

c0:

$ref: '#/components/schemas/C0'

ltePosSibTypes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

nrPosSibTypes:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

validityStartTime:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

validityDuration:

$ref: '#/components/schemas/ValidityDuration'

taiList:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

CipheringSetReport:

type: object

required:

- cipheringSetID

- storageOutcome

properties:

cipheringSetID:

$ref: '#/components/schemas/CipheringSetID'

storageOutcome:

$ref: '#/components/schemas/StorageOutcome'

CipherRequestData:

type: object

required:

- amfCallBackURI

properties:

amfCallBackURI:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

CipherResponseData:

type: object

required:

- dataAvailability

properties:

dataAvailability:

$ref: '#/components/schemas/DataAvailability'

#

#

# SIMPLE TYPES

#

CipheringSetID:

type: integer

minimum: 0

maximum: 65535

CipheringKey:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

C0:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Binary'

ValidityDuration:

type: integer

minimum: 1

maximum: 65535

#

# ENUMS

#

StorageOutcome:

anyOf:

- type: string

enum:

- STORAGE_SUCCESSFUL

- STORAGE_FAILED

DataAvailability:

anyOf:

- type: string

enum:

- CIPHERING_KEY_DATA_AVAILABLE

- CIPHERING_KEY_DATA_NOT_AVAILABLE

Annex B (informative):	Change history

| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2018-01 | CT4#82 |  |  |  |  | TS Skeleton agreed in CT4#82 | 0.0.0 |
| 2018-01 | CT4#82 | C4-181398 |  |  |  | Initial draft (C4-181119) Incorporation of agreed pCRs from CT4#82: C4-181121, C4-181233, C4-181234 | 0.1.0 |
| 2018-03 | CT4#83 | C4-182444 |  |  |  | Incorporation of agreed pCRs from CT4#83: C4-182181, C4-182427 | 0.2.0 |
| 2018-03 | CT#79 | CP-180034 |  |  |  | Presented for information | 1.0.0 |
| 2018-04 | CT4#84 | C4-183524 |  |  |  | Incorporation of agreed pCRs from CT4#84: C4-183184, C4-183363, C4-183510 | 1.1.0 |
| 2018-05 | CT4#85 | C4-184640 |  |  |  | Incorporation of agreed pCRs from CT4#85: C4-184195, C4-184197, C4-184198, C4-184199, C4-184202, C4-184443, C4-184446, C4-184547 | 1.2.0 |
| 2018-06 | CT#80 | CP-181111 |  |  |  | Presented for approval | 2.0.0 |
| 2018-06 | CT#80 |  |  |  |  | Approved in CT#80 | 15.0.0 |
| 2018-09 | CT#81 | CP-182066 | 0002 | 2 |  | Error Cases | 15.1.0 |
| 2018-09 | CT#81 | CP-182066 | 0003 | - |  | Custom Headers | 15.1.0 |
| 2018-09 | CT#81 | CP-182066 | 0004 | - |  | Overall Clean-up | 15.1.0 |
| 2018-09 | CT#81 | CP-182066 | 0005 | - |  | Description of Structured data types | 15.1.0 |
| 2018-09 | CT#81 | CP-182066 | 0006 | 1 |  | Resource structure presentation | 15.1.0 |
| 2018-09 | CT#81 | CP-182066 | 0007 | 1 |  | LMF servers clause in OpenAPI | 15.1.0 |
| 2018-09 | CT#81 | CP-182066 | 0008 | - |  | API Version Update | 15.1.0 |
| 2018-12 | CT#82 | CP-183025 | 0010 | 1 | F | Cardinality | 15.2.0 |
| 2018-12 | CT#82 | CP-183025 | 0011 | - | F | APIRoot Clarification | 15.2.0 |
| 2018-12 | CT#82 | CP-183025 | 0012 | - | F | AMF Id | 15.2.0 |
| 2018-12 | CT#82 | CP-183025 | 0013 | - | F | Barometric Pressure in Location Data | 15.2.0 |
| 2018-12 | CT#82 | CP-183025 | 0014 | 1 | F | Clarify Serving Cell in Input Data | 15.2.0 |
| 2018-12 | CT#82 | CP-183025 | 0015 | 1 | F | Oauth2 Corrections | 15.2.0 |
| 2018-12 | CT#82 | CP-183025 | 0016 | - | F | API Version | 15.2.0 |
| 2018-12 | CT#82 | CP-183179 | 0017 | - | F | ExternalDocs Update | 15.2.0 |
| 2019-03 | CT#83 | CP-190030 | 0018 | 1 | F | OpenAPI Corrections | 15.3.0 |
| 2019-03 | CT#83 | CP-190030 | 0019 | 1 | F | Application Errors | 15.3.0 |
| 2019-03 | CT#83 | CP-190030 | 0020 | 1 | F | Essential Correction to InnerRadius | 15.3.0 |
| 2019-03 | CT#83 | CP-190030 | 0021 | 1 | F | Mandatory Response Codes | 15.3.0 |
| 2019-03 | CT#83 | CP-190030 | 0022 | 1 | F | Essential correction to OpenAPI definition of GeographicArea | 15.3.0 |
| 2019-03 | CT#83 | CP-190030 | 0023 | - | F | API version update | 15.3.0 |
| 2019-06 | CT#84 | CP-191042 | 0024 | 2 | F | UE Capabilities | 15.4.0 |
| 2019-06 | CT#84 | CP-191042 | 0025 | 2 | F | Storage of OpenAPI specification files | 15.4.0 |
| 2019-06 | CT#84 | CP-191042 | 0027 | 1 | F | Copyright Note in OpenAPI Spec | 15.4.0 |
| 2019-06 | CT#84 | CP-191042 | 0028 | 1 | F | Major API version | 15.4.0 |
| 2019-06 | CT#84 | CP-191042 | 0030 | - | F | Open API Version | 15.4.0 |
| 2019-09 | CT#85 | CP-192113 | 0031 | 1 | F | Missing attribute FLR in Civic Address | 16.0.0 |
| 2019-09 | CT#85 | CP-192192 | 0033 | 2 | B | LMF service operations for a deferred 5GC-MT-LR | 16.0.0 |
| 2019-09 | CT#85 | CP-192192 | 0034 | 1 | B | LMF service operations for a commercial 5GC-MT-LR | 16.0.0 |
| 2019-09 | CT#85 | CP-192192 | 0035 | - | F | High Accuracy Support | 16.0.0 |
| 2019-09 | CT#85 | CP-192113 | 0037 | 1 | D | Correct type Polygon | 16.0.0 |
| 2019-09 | CT#85 | CP-192120 | 0039 | - | F | 3GPP TS 29.572 API version update | 16.0.0 |
| 2019-12 | CT#86 | CP-193033 | 0041 | 1 | A | Motion Sensor Position Method | 16.1.0 |
| 2019-12 | CT#86 | CP-193165 | 0042 | 3 | B | Addition of the LMF Broadcast Service Operations | 16.1.0 |
| 2019-12 | CT#86 | CP-193055 | 0043 | 1 | B | LCS QoS Class | 16.1.0 |
| 2019-12 | CT#86 | CP-193036 | 0045 | 1 | F | ExternalDoc Section | 16.1.0 |
| 2019-12 | CT#86 | CP-193036 | 0046 | 1 | F | ProblemDetails Optional in Error Response | 16.1.0 |
| 2019-12 | CT#86 | CP-193044 | 0048 | - | F | 3GPP TS 29.572 API version update | 16.1.0 |