| 3GPP TS 29.598 V16.1.0 (2020-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; 5G System; Unstructured Data Storage Services Stage 3; (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Overview	9

5	Services offered by the UDSF	10

5.1	Introduction	10

5.2	Nudsf_DataRepository Service	10

5.2.1	Service Description	10

5.2.2	Service Operations	10

5.2.2.1	Introduction	10

5.2.2.2	Query	11

5.2.2.2.1	General	11

5.2.2.2.2	Record Retrieval	11

5.2.2.2.3	Meta Retrieval	11

5.2.2.2.4	Blocks Retrieval	12

5.2.2.2.5	Block Retrieval	12

5.2.2.2.6	Search	13

5.2.2.2.7	Subscriptions Retrieval	13

5.2.2.2.8	Individual Subscription Retrieval	14

5.2.2.3	Create	14

5.2.2.3.1	General	14

5.2.2.3.2	Record Create	15

5.2.2.3.3	Block Create	15

5.2.2.4	Update	16

5.2.2.4.1	General	16

5.2.2.4.2	Record Update	16

5.2.2.4.3	Block Update	17

5.2.2.4.4	Meta Update	17

5.2.2.4.5	Subscription Notification Update	18

5.2.2.4.6	Subscription Notification Update using PUT	18

5.2.2.5	Delete	19

5.2.2.5.1	General	19

5.2.2.5.2	Record Delete	19

5.2.2.5.3	Block Delete	20

5.2.2.6	Notify	20

5.2.2.6.1	General	20

5.2.2.6.2	Record Expiry Notify	20

5.2.2.6.3	Notification due to Data Change	21

5.2.2.7	Subscribe	21

5.2.2.7.1	General	21

5.2.2.7.2	Subscription to notifications of data change	21

6	API Definitions	22

6.1	Nudsf_DataRepository Service API	22

6.1.1	Introduction	22

6.1.2	Usage of HTTP	22

6.1.2.1	General	22

6.1.2.2	HTTP standard headers	23

6.1.2.2.1	General	23

6.1.2.2.2	Content type	23

6.1.2.2.3	Cache-Control	23

6.1.2.2.4	ETag	23

6.1.2.2.5	If-None-Match	23

6.1.2.2.6	If-Match	23

6.1.2.2.7	Last-Modified	23

6.1.2.2.8	If-Modified-Since	24

6.1.2.2.9	When to Use Entity-Tags and Last-Modified Dates	24

6.1.2.2.10	Content-Location	24

6.1.2.3	HTTP custom headers	24

6.1.2.4	HTTP multipart messages	24

6.1.2.4.1	General	24

6.1.2.4.2	Record	24

6.1.2.4.3	BlockCollection	24

6.1.2.4.4	RecordNotification	25

6.1.3	Resources	25

6.1.3.1	Overview	25

6.1.3.2	Resource: RecordCollection (Collection)	26

6.1.3.2.1	Description	26

6.1.3.2.2	Resource Definition	26

6.1.3.2.3	Resource Standard Methods	27

6.1.3.2.3.1	GET	27

6.1.3.3	Resource: Record (Document)	28

6.1.3.3.1	Description	28

6.1.3.3.2	Resource Definition	28

6.1.3.3.3	Resource Standard Methods	28

6.1.3.3.3.1	GET	28

6.1.3.3.3.2	PUT	28

6.1.3.3.3.3	DELETE	29

6.1.3.4	Resource: Meta (Document)	30

6.1.3.4.1	Description	30

6.1.3.4.2	Resource Definition	30

6.1.3.4.3	Resource Standard Methods	31

6.1.3.4.3.1	GET	31

6.1.3.4.3.2	PATCH	31

6.1.3.5	Resource: BlockCollection (Collection)	32

6.1.3.5.1	Description	32

6.1.3.5.2	Resource Definition	32

6.1.3.5.3	Resource Standard Methods	32

6.1.3.5.3.1	GET	32

6.1.3.6	Resource: Block (Document)	33

6.1.3.6.1	Description	33

6.1.3.6.2	Resource Definition	33

6.1.3.6.3	Resource Standard Methods	33

6.1.3.6.3.1	GET	33

6.1.3.6.3.2	PUT	34

6.1.3.6.3.3	DELETE	35

6.1.3.7	Resource: NotificationSubscriptions	36

6.1.3.7.1	Description	36

6.1.3.7.2	Resource Definition	36

6.1.3.7.3	Standard Methods	36

6.1.3.7.3.2	GET	36

6.1.3.8	Resource: IndividualNotificationSubscription	37

6.1.3.8.1	Description	37

6.1.3.8.2	Resource Definition	37

6.1.3.8.3	Resource Standard Methods	37

6.1.3.8.3.1	DELETE	37

6.1.3.8.3.2	PATCH	38

6.1.3.8.3.3	GET	39

6.1.3.8.3.4	PUT	39

6.1.4	Custom Operations without associated resources	40

6.1.5	Notifications	40

6.1.5.1	General	40

6.1.5.2	Timer Expiry Notification	40

6.1.5.2.1	Description	40

6.1.5.2.2	Resource URI	40

6.1.5.2.3	Standard Methods	41

6.1.5.2.3.1	POST	41

6.1.5.3	Notification due to Data Change	41

6.1.5.3.1	Description	41

6.1.5.3.2	Resource URI	41

6.1.5.3.3	Standard Methods	41

6.1.5.3.3.1	POST	41

6.1.6	Data Model	42

6.1.6.1	General	42

6.1.6.2	Structured data types	42

6.1.6.2.1	Introduction	42

6.1.6.2.2	Type: RecordSearchResult	43

6.1.6.2.3	Type: RecordMeta	43

6.1.6.2.4	Type: RecordBody	43

6.1.6.2.5	Type: Record	43

6.1.6.2.6	Type: BlockBody	43

6.1.6.2.7	Type: Block	44

6.1.6.2.8	Type: SearchCondition	44

6.1.6.2.9	SearchComparison	44

6.1.6.2.10	Type: NotificationSubscription	44

6.1.6.2.11	Type: RecordNotification	45

6.1.6.2.12	Type: NotificationDescription	46

6.1.6.2.13	Type: SubscriptionFilter	46

6.1.6.2.14	Type: ClientId	46

6.1.6.3.15	Enumeration: RecordOperation	47

6.1.6.3	Simple data types and enumerations	47

6.1.6.3.1	Introduction	47

6.1.6.3.2	Simple data types	47

6.1.6.3.3	Enumeration: ComparisonOperator	47

6.1.6.3.4	Enumeration: ConditionOperator	47

6.1.6.4	Data types describing alternative data types or combinations of data types	48

6.1.6.4.1	Type: SearchExpression	48

6.1.7	Error Handling	48

6.1.7.1	General	48

6.1.7.2	Protocol Errors	48

6.1.7.3	Application Errors	48

6.1.8	Feature negotiation	48

6.1.9	Security	49

Annex A (normative):	OpenAPI specification	50

A.1	General	50

A.2	Nudsf_DataRepository API	50

Annex B (informative):	Search Examples	74

Annex C (informative):	HTTP Multipart Examples	75

C.1	General	75

C.2	Example HTTP multipart Record	75

C.3	Example HTTP multipart BlockCollection	75

C.4	Example HTTP multipart RecordNotification	76

Annex D (informative):	Change history	77

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document specifies the stage 3 protocol and data model for the Nudsf Service Based Interface. It provides stage 3 protocol definitions and message flows, and specifies the API for each service offered by the UDSF.

The 5G System stage 2 architecture and procedures are specified in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

The Technical Realization of the Service Based Architecture and the Principles and Guidelines for Services Definition are specified in 3GPP TS 29.500 [4] and 3GPP TS 29.501 [5].

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 29.500: "5G System; Technical Realization of Service Based Architecture; Stage 3".

[5]	3GPP TS 29.501: "5G System; Principles and Guidelines for Services Definition; Stage 3".

[6]	OpenAPI: "OpenAPI 3.0.0 Specification", https://github.com/OAI/OpenAPI-Specification/blob/master/versions/3.0.0.md..

[7]	3GPP TR 21.900: "Technical Specification Group working methods".

[8]	3GPP TS 33.501: "Security architecture and procedures for 5G system".

[9]	IETF RFC 6749: "The OAuth 2.0 Authorization Framework".

[10]	3GPP TS 29.510: "5G System; Network Function Repository Services; Stage 3".

[11]	IETF RFC 7540: "Hypertext Transfer Protocol Version 2 (HTTP/2)".

[12]	IETF RFC 8259: "The JavaScript Object Notation (JSON) Data Interchange Format".

[13]	IETF RFC 7807: "Problem Details for HTTP APIs".

[14]	IETF RFC 6902: "JavaScript Object Notation (JSON) Patch".

[15]	IETF RFC 7231: "Hypertext Transfer Protocol (HTTP/1.1): Semantics and Content".

[16]	IETF RFC 7232: "Hypertext Transfer Protocol (HTTP/1.1): Conditional Requests".

[17]	IETF RFC 7234: "Hypertext Transfer Protocol (HTTP/1.1): Caching".

[18]	ISO/IEC 14977: "Information technology – Syntactic metalanguage - Extended BNF".

[19]	3GPP TS 29.571: "5G System; Common Data Types for Service Based Interfaces; Stage 3".

[20]	IETF RFC 2045: "Multipurpose Internet Mail Extensions (MIME) Part One: Format of Internet Message Bodies".

[21]	IETF RFC 2046: "Multipurpose Internet Mail Extensions (MIME) Part Two: Media Types".

[22]	IETF RFC 3986: "Uniform Resource Identifier (URI): Generic Syntax".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

void

## 3.2	Symbols

void

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP  TR 21.905 [1].

5GC	5G Core Network

BNF	Backus–Naur Form

EBNF	Extended BNF

CP	Control Plane

MIME	Multipurpose Internet Mail Extensions

NF	Network Function

UDSF	Unstructured Data Storage Function

# 4	Overview

The UDSF, as depicted in Figure 4.1-1 below, is described in clause 4.2.5 of 3GPP TS 23.501 [2]. Any of the 5GS NFs can make use of the UDSF to store and retrieve unstructured data, i.e., data that is not defined in 3GPP specifications. The UDSF is deployed in the same network where the CP NF is located and the same UDSF may be shared by all the NFs in the PLMN to store/retrieve their respective data or an NF may have its own UDSF depending on operator configuration.

NOTE 1:	Structured data in this specification refers to data for which the structure is defined in 3GPP specifications. Unstructured data refers to data for which the structure is not defined in 3GPP specifications.

Figure 4.1-1: Reference model – UDSF

# 5	Services offered by the UDSF

## 5.1	Introduction

The UDSF offers the following services via the Nudsf service based interface:

-	Nudsf_DataRepository Service

NOTE:	This service corresponds to the Nudsf_UnstructuredDataManagement service in 3GPP TS 23.501 [2] and 3GPP TS 23.502 [3].

Table 5.1-1 summarizes the corresponding APIs defined for this specification.

Table 5.1-1: API Descriptions

| Service Name | Clause | Description | OpenAPI Specification File | apiName | Annex |
| --- | --- | --- | --- | --- | --- |
| Nudsf_DataRepository | 6.1 | UDSF Data Repository Service | TS29598_Nudsf_DataRepository.yaml | nudsf-dr | A.2 |

## 5.2	Nudsf_DataRepository Service

### 5.2.1	Service Description

The UDSF is acting as an NF Service Producer. It provides UDSF data repository service to the NF service consumer. Any NF may use the UDSF to store unstructured data.

NOTE 1:	Structured data in this specification refers to data for which the structure is defined in 3GPP specifications. Unstructured data refers to data for which the structure is not defined in 3GPP specifications.

### 5.2.2	Service Operations

#### 5.2.2.1	Introduction

For the Nudsf_DataRepository service, the following service operations are defined:

-	Query

-	Create

-	Update

-	Delete

#### 5.2.2.2	Query

##### 5.2.2.2.1	General

The following procedures using the Query service operation are supported:

-	Record Retrieval

-	Meta Retrieval

-	Blocks Retrieval

-	Block Retrieval

-	Search

-	Subscriptions Retrieval

-	Individual Subscription Retrieval

##### 5.2.2.2.2	Record Retrieval

Figure 5.2.2.2.2-1 shows a scenario where the NF service consumer sends a request to the UDSF to retrieve a record that matches the provided recordId and optionally includes the query parameter supported-features.

Figure 5.2.2.2.2-1: Requesting a Record

1.	The NF service consumer (any NF) sends a GET request to the resource indicated by recordId.

2a.	On success, the UDSF responds with "200 OK" with the message body containing the record.

2b.	If the record for the given recordId does not exist in the UDSF, the HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the ProblemDetails element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.3	Meta Retrieval

Figure 5.2.2.2.3-1 shows a scenario where the NF service consumer sends a request to the UDSF to retrieve meta data associated with the provided recordId and optionally includes the query parameter supported-features.

Figure 5.2.2.2.3-1: Requesting Meta for a Record

1.	The NF service consumer (any NF) sends a GET request to the meta resource associated with the record indicated by recordId.

2a.	On success, the UDSF responds with "200 OK" with the message body containing the RecordMeta.

2b.	If the record for the given recordId and thus the RecordMeta does not exist in the UDSF, the HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the ProblemDetails element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.4	Blocks Retrieval

Figure 5.2.2.2.4-1 shows a scenario where the NF service consumer sends a request to the UDSF to retrieve (all) the blocks associated with the provided recordId and optionally includes the query parameter supported-features.

Figure 5.2.2.2.4-1: Requesting Blocks

1.	The NF service consumer (any NF) sends a GET request to the resource indicated by recordId.

2a.	On success, the UDSF responds with "200 OK" with the message body containing the Blocks associated with the record.

2b.	If a Block for the given recordId does not exist in the UDSF, the HTTP status code "204 No Content" shall be returned.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.5	Block Retrieval

Figure 5.2.2.2.5-1 shows a scenario where the NF service consumer sends a request to the UDSF to retrieve a single block associated with the provided recordId and blockId and optionally includes the query parameter supported-features.

Figure 5.2.2.2.5-1: Requesting a Block

1.	The NF service consumer (any NF) sends a GET request to the resource indicated by recordId and blockId.

2a.	On success, the UDSF responds with "200 OK" with the message body containing the Block associated with the blockId.

2b.	If the Block for the given recordId and blockId does not exist in the UDSF, the HTTP status code "404 Not Found" shall be returned including additional error information in the response body (in the ProblemDetails element).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.6	Search

Figure 5.2.2.2.6-1 shows a scenario where the NF service consumer sends a request to the UDSF to search a record that matches the provided search criteria.

The request contains the query parameters filter and optionally supported-features, limit-range, page-number and count-indicator.

Figure 5.2.2.2.6-1: Searching for Records

1.	The NF service consumer (any NF) sends a GET request to the Records resource with the filter query parameter indicating the search criteria.

2a.	On success, the UDSF responds with "200 OK" with the message body containing the RecordSearchResult.

2b.	If the UDSF is not able to return any record for the given search criteria, the HTTP status code "204 No Content" shall be returned.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.7	Subscriptions Retrieval

Figure 5.2.2.2.7-1 shows a scenario where the NF service consumer sends a request to the UDSF to retrieve all subscriptions associated with the provided storageId and optionally includes the query parameter supported-features, limit-range and page-number.

Figure 5.2.2.2.7-1: Requesting Subscriptions

1.	The NF service consumer (any NF) sends a GET request to the resource indicated by the storageId.

2.	On success, the UDSF responds with "200 OK" with the message body containing the NotificationSubscriptions associated with the storageId (if any).

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

##### 5.2.2.2.8	Individual Subscription Retrieval

Figure 5.2.2.2.8-1 shows a scenario where the NF service consumer sends a request to the UDSF to retrieve a subscription associated with the provided storageId and subscriptionId.

Figure 5.2.2.2.8-1: Requesting an Individual Subscription

1.	The NF service consumer (any NF) sends a GET request to the resource indicated by the storageId and the subscriptionId.

2a.	On success, the UDSF responds with "200 OK" with the message body containing the NotificationSubscription.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the GET response body.

#### 5.2.2.3	Create

##### 5.2.2.3.1	General

The following procedures using the Create service operation are supported:

-	Record Create

-	Block Create

##### 5.2.2.3.2	Record Create

Figure 5.2.2.3.2-1 shows a scenario where the NF service consumer sends a request to the UDSF to create a record with the provided recordId.

The request contains the recordId and optionally the query parameters supported-features and get-previous.

Figure 5.2.2.3.2-1: Create a Record

1.	The NF service consumer (any NF) sends a PUT request to create the resource indicated by recordId. The request body contains the meta, zero or more blocks. The record meta information is mandatory and shall be the first part and the remaining parts of the request body (if any) shall be child blocks. If the record meta information is received with record expiry details, UDSF shall create an implicit subscription locally and notify the NF service consumer on record expiry.

2a.	On success, "201 Created" shall be returned, the payload body of the PUT response should contain the representation of the created resource, and the "Location" header shall be present and shall contain the URI of the created resource.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

##### 5.2.2.3.3	Block Create

Figure 5.2.2.3.3-1 shows a scenario where the NF service consumer sends a request to the UDSF to create a block with the provided blockId.

The request contains the blockid and optionally the query parameters supported-features and get-previous.

Figure 5.2.2.3.3-1: Create a Block

1.	The NF service consumer (any NF) sends a PUT request to create the resource indicated by blockId.

2a.	On success, "201 Created" shall be returned, the payload body of the PUT response should contain the representation of the created resource, and the "Location" header shall be present and shall contain the URI of the created resource.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

#### 5.2.2.4	Update

##### 5.2.2.4.1	General

The following procedures using the Update service operation are supported:

-	Record Update

-	Block Update

-	Meta Update

-	Subscription Notification Update

##### 5.2.2.4.2	Record Update

Figure 5.2.2.4.2-1 shows a scenario where the NF service consumer sends a request to the UDSF to update a record with the provided recordId.

The request contains the recordId and optionally the query parameters supported-features and get-previous.

The update shall include meta, zero or more blocks. The record meta information shall be the first part and is mandatory and the remaining parts of the body (if any) shall be interpreted as child blocks. Existing record, meta and blocks shall be discarded and the new record, meta and blocks (if any) shall be created.

NOTE:	The order of the returned blocks in the response is not guaranteed and can be different from the order used to create them.

Figure 5.2.2.4.2-1: Update a record

1.	The NF service consumer shall send a PUT request to the resource representing the record that is to be updated, and may include meta, zero or more blocks. The record meta information is mandatory and shall be the first part and the remaining parts of the request body (if any) shall be child blocks. An existing record, i.e., meta and blocks shall be discarded and the new record, meta and blocks (if any) shall be created.

2a.	On success, the UDSF shall respond with "204 No Content" if no record is returned, i.e. the get-previous query parameter was not included in the request.

2b.	On success, the UDSF shall respond with "200 OK" if a record is returned, i.e. the get-previous query parameter was included in the request, or due to operator's policy, the ttl value in the request exceeded the maximum value allowed.

2c.	On failure, the UDSF shall respond with "412 Precondition Failed" if one or more conditions given in the request header fields evaluated to false. The RecordBody shall include the stored Record if the get-previous query parameter was included in the request.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

##### 5.2.2.4.3	Block Update

Figure 5.2.2.4.3-1 shows a scenario where the NF service consumer sends a request to the UDSF to update a block with the provided blockId.

The request contains the recordId, blockId and the optional query parameters supported-features, get-previous and the data that is to be updated.

Figure 5.2.2.4.3-1: Update a block

1.	The NF service consumer shall send a PUT request to the resource representing the block that is to be updated.

2a.	On success, the UDSF shall respond with "204 No Content" if no record is returned, i.e. the get-previous query parameter was not included in the request.

2b.	On success, the UDSF shall respond with "200 OK" if a record is returned, i.e. the get-previous query parameter was included in the request.

2c.	On failure, the UDSF shall respond with "412 Precondition Failed" if one or more conditions given in the request header fields evaluated to false. The BlockBody shall be included with the stored Block if the get-previous query parameter was included in the request.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

##### 5.2.2.4.4	Meta Update

Figure 5.2.2.4.4-1 shows a scenario where the NF service consumer sends a request to the UDSF to update the meta data associated with the provided recordId and optionally the query parameter supported-features and the data that is to be updated.

Figure 5.2.2.4.4-1: Update meta

1.	The NF service consumer shall send a PATCH request to the resource representing the meta of the record.

2a.	On success, if all the modification instructions in the PATCH request have been implemented, the UDSF shall respond with "204 No Content".

2b.	On partial success, i.e. if one or more modification instructions have been discarded, "200 OK" with the execution report, shall be returned.

2c.	On failure, the UDSF shall respond with "404 Not Found" if the record indicated by the recordId and thus the meta does not exist and shall include the ProblemDetails.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

##### 5.2.2.4.5	Subscription Notification Update

Figure 5.2.2.4.5-1 shows a scenario where the NF service consumer sends a request to the UDSF to update the Individual Subscription Notification identified with the storageId and subscriptionId and optionally the query parameter supported-features and the data that is to be updated.

Figure 5.2.2.4.5-1: Update Subscription Notification

1.	The NF service consumer shall send a PATCH request to the resource representing the subscriptionId.

2a.	On success, if all the modification instructions in the PATCH request have been implemented, the UDSF shall respond with "204 No Content".

2b.	On partial success, i.e. if one or more modification instructions have been discarded, "200 OK" with the execution report, shall be returned.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PATCH response body.

##### 5.2.2.4.6	Subscription Notification Update using PUT

Figure 5.2.2.4.6-1 shows a scenario where the NF service consumer sends a request to the UDSF to update a subscription to notifications of data change using PUT. The request contains the subscriptionId and the NotificationSubscription and optionally the query parameter supported-features.

Figure 5.2.2.4/6-1: NF service consumer subscribes to notifications

1.	The NF service consumer sends a PUT request to the resource indicated by the storageId and the subscriptionId.

2a.	On success, the UDSF shall respond with "200 OK" and include the updated NotificationSubscription. The expiry attribute of the received NotificationSubscription may indicate a value or a value different from the request, if due to an operator policy, an expiry time is enforced or if the value in the request exceeded a maximum allowed expiry time.

2b.	On failure, if one or more monitoredResourceUris from the request don't exist in the UDSF, 409 Conflict shall be returned together with the non-existing monitoredResourceUris.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

#### 5.2.2.5	Delete

##### 5.2.2.5.1	General

The following procedures using the Delete service operation are supported:

-	Record Delete

-	Block Delete

##### 5.2.2.5.2	Record Delete

Figure 5.2.2.5.2-1 shows a scenario where the NF service consumer sends a request to the UDSF to Delete a record with the provided recordId.

The request contains the record id and optionally the query parameters supported-features and get-previous.

Figure 5.2.2.5.2-1: Delete a record

1.	The NF service consumer shall send a DELETE request to the resource representing the record. The UDSF shall delete any resource associated with the resource (meta and block(s)).

2a.	On success, the UDSF shall respond with "204 No Content" if no record is returned, i.e. the get-previous query parameter was not included in the request.

2b.	On success, the UDSF shall respond with "200 OK" if a record is returned, i.e. the get-previous query parameter was included in the request.

2c.	On failure, the UDSF shall respond with "404 Not Found" if the record does not exist and shall include the ProblemDetails.

2d.	On failure, the UDSF shall respond with "412 Precondition Failed" if one or more conditions given in the request header fields evaluated to false and the get-previous query parameter was included in the request. The RecordBody shall be included if the get-previous query parameter was included in the request.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

##### 5.2.2.5.3	Block Delete

Figure 5.2.2.5.3-1 shows a scenario where the NF service consumer sends a request to the UDSF to Delete a block with the provided blockId.

The request contains the recordId, blockId and optionally the query parameters supported-features and get-previous.

Figure 5.2.2.5.3-1: Delete a block

1.	The NF service consumer shall send a DELETE request to the resource representing the block.

2a.	On success, the UDSF shall respond with "204 No Content" if no block is returned, i.e. the get-previous query parameter was not included in the request.

2b.	On success, the UDSF shall respond with "200 OK" if a block is returned, i.e. the get-previous query parameter was included in the request.

2c.	On failure, the UDSF shall respond with "404 Not Found" if the block does not exist and shall include the ProblemDetails.

2d.	On failure, the UDSF shall respond with "412 Precondition Failed" if one or more conditions given in the request header fields evaluated to false. The BlockBody shall be included if the get-previous query parameter was included in the request.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the DELETE response body.

#### 5.2.2.6	Notify

##### 5.2.2.6.1	General

The following procedures using the Notify service operation are supported:

-	Record Expiry Notify

-	Notification due to Data Change

##### 5.2.2.6.2	Record Expiry Notify

Figure5.2.2.6.2-1 shows a scenario where the UDSF notifies the NF service consumer of the expired record.

The Notify is sent by the UDSF to the NF Service Consumer when the record expires as indicated by the time to live (ttl) attribute of RecordMeta.

Figure 5.2.2.6.2-1: Record Expiry Notify

1. The UDSF shall send a POST request to the callback URI. The request shall contain the record details.

2a. On success, "204 No content" shall be returned by the NF Service Consumer to UDSF.

2b. If the NF Service Consumer does not consider the "Callback URI" as a valid notification URI, it shall return "404 Not Found" status code and shall include the ProblemDetails providing details of the error.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

##### 5.2.2.6.3	Notification due to Data Change

Figure 5.2.2.6.3-1 shows a scenario where the UDSF notifies the NF service consumer of a change to data associated with a block or a record triggered by one or more a Subscription to Notification.

Figure 5.2.2.6.3-1: Notification due to Data Change

1.	The UDSF shall send a POST request to the callback URI. The request shall contain the RecordNotification details.

2.	On success, "204 No content" shall be returned by the NF Service Consumer to UDSF.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the POST response body.

#### 5.2.2.7	Subscribe

##### 5.2.2.7.1	General

The following procedures using the Subscribe service operation are supported:

-	Subscription to notification of data change

##### 5.2.2.7.2	Subscription to notifications of data change

Figure 5.2.2.7.2-1 shows a scenario where the NF service consumer sends a request to the UDSF to subscribe to notifications of data change. The request contains the subscriptionId, the NotificationSubscription and optionally the query parameter supported-features.

Figure 5.2.2.7.2-1: NF service consumer subscribes to notifications

1.	The NF service consumer sends a PUT request to the resource indicated by the storageId and the subscriptionId.

2a.	On success, the UDSF responds with "201 Created"with the message body containing the NotificationSubscription. The expiry attribute of the received NotificationSubscription may indicate a value or a value different from the request, if due to an operator policy, an expiry time is enforced or if the value in the request exceeded a maximum allowed expiry time.

2b.	On failure, if one or more monitoredResourceUris from the request don't exist in the UDSF, 409 Conflict shall be returned together with the non-existing monitoredResourceUris.

On failure, the appropriate HTTP status code indicating the error shall be returned and appropriate additional error information should be returned in the PUT response body.

# 6	API Definitions

## 6.1	Nudsf_DataRepository Service API

### 6.1.1	Introduction

The Nudsf_DataRepository service shall use the Nudsf_DataRepository API.

The request URI used in HTTP request from the NF service consumer towards the NF service producer shall have the structure defined in clause 4.4.1 of 3GPP TS 29.501 [5], i.e.:

{apiRoot}/<apiName>/<apiVersion>/<apiSpecificResourceUriPart>

with the following components:

-	The {apiRoot} shall be set as described in 3GPP TS 29.501 [5].

-	The <apiName> shall be "nudsf-dr".

-	The <apiVersion> shall be "v1".

-	The <apiSpecificResourceUriPart> shall be set as described in clause 6.1.3.

### 6.1.2	Usage of HTTP

#### 6.1.2.1	General

HTTP/2, IETF RFC 7540 [11], shall be used as specified in clause 5 of 3GPP TS 29.500 [4].

HTTP/2 shall be transported as specified in clause 5.3 of 3GPP TS 29.500 [4].

The OpenAPI [6] specification of HTTP messages and content bodies for the Nudsf_DataRepository API is contained in Annex A.

#### 6.1.2.2	HTTP standard headers

##### 6.1.2.2.1	General

See clause 5.2.2 of 3GPP TS 29.500 [4] for the usage of HTTP standard headers.

##### 6.1.2.2.2	Content type

The following content types shall be supported:

-	JSON, IETF RFC 8259 [12], shall be used as content type of the HTTP bodies specified in the present specification as specified in clause 5.4 of 3GPP TS 29.500 [4]. The use of the JSON format shall be signalled by the content type "application/json".

-	"Problem Details" JSON object shall be used to indicate additional details of the error in a HTTP response body and shall be signalled by the content type "application/problem+json", as defined in IETF RFC 7807 [13].

-	JSON Patch (IETF RFC 6902 [14]). The use of the JSON Patch format in a HTTP request body shall be signalled by the content type "application/json-patch+json".

##### 6.1.2.2.3	Cache-Control

As described in IETF RFC 7234 [17] clause 5.2, a "Cache-Control" header should be included in HTTP responses carrying a representation of cacheable resources. If it is included, it shall contain a "max-age" value, indicating the amount of time in seconds after which the received response is considered stale.

The "max-age" value shall be configurable by operator policy.

##### 6.1.2.2.4	ETag

As described in IETF RFC 7232 [16] clause 2.3, an "ETag" (entity-tag) header should be included in HTTP responses carrying a representation of cacheable or modifiable resources  to allow an NF Service Consumer performing a conditional GET request with an "If-None-Match" header or a conditional PUT/PATCH/DELETE request with an "If-Match" header. If it is included, it shall contain a server-generated strong validator, that allows further matching of this value (included in subsequent client requests) with a given resource representation stored in the server or in a cache.

##### 6.1.2.2.5	If-None-Match

As described in IETF RFC 7232 [16] clause 3.2, an NF Service Consumer may issue conditional GET or PUT requests towards UDSF by including an "If-None-Match" header in HTTP requests containing one or several entity tags received in previous responses for the same resource.

If the If-None-Match header is included with the PUT method, it shall be used with a value of "*" to prevent the inadvertent modification of an existing representation of the target resource when the client believes that the resource does not have a current representation.

##### 6.1.2.2.6	If-Match

As described in IETF RFC 7232 [16] clause 3.1, an NF Service Consumer may issue conditional PUT/PATCH/DELETE request towards UDSF by including an "If-Match" header in HTTP requests containing an entity tag received in previous responses for the same resource.

##### 6.1.2.2.7	Last-Modified

As described in IETF RFC 7232 [16] clause 2.2, a "Last-Modified" header should be included in HTTP responses carrying a representation of cacheable resources to allow an NF Service Consumer performing a conditional request with "If-Modified-Since" header.

##### 6.1.2.2.8	If-Modified-Since

As described in IETF RFC 7232 [16] clause 3.3, an NF Service Consumer may issue conditional GET request towards UDSF, by including an "If-Modified-Since" header in HTTP requests.

##### 6.1.2.2.9	When to Use Entity-Tags and Last-Modified Dates

Both "ETag" and "Last-Modified" headers should be sent in the same HTTP response as stated in IETF RFC 7232 [16] clause 2.4.

NOTE: "ETag" is a stronger validator than the "Last-Modified" and is preferred.

If the UDSF included an "ETag" header with the resource then a conditional GET request for this resource shall be performed with the "If-None-Match" header, and a PUT/PATCH/DELETE request for this resource shall be performed with the "If-Match" header.

##### 6.1.2.2.10	Content-Location

As described in IETF RFC 7231 [15] clause 3.1.4.2, the UDSF shall include the Content-Location header set to the URI of the expired Record when sending a Notification to an NF Consumer.

#### 6.1.2.3	HTTP custom headers

The mandatory HTTP custom header fields specified in clause 5.2.3.2 of 3GPP TS 29.500 [4] shall be applicable.

#### 6.1.2.4	HTTP multipart messages

##### 6.1.2.4.1	General

HTTP multipart messages shall be supported to transfer the opaque Record and Block Information in the following service operations (and HTTP messages):

-	Record (PUT/GET/DELETE)

-	BlockCollection (GET)

##### 6.1.2.4.2	Record

The Record is encoded as an HTTP multipart message with the multipart/mixed content-type as described in IETF RFC 2046 [21].

The boundary parameter is used to delimit each part (Start of parts to RecordMeta, RecordMeta to Block, Block to Block and Block to End of parts) and shall be set to a value as in accordance with IETF  RFC 2046 [21].

The RecordMeta part shall be the first part, and shall always be present. It shall include a Content-ID header set to a "world-unique" value (see IETF RFC 2045 [20]), the Content-Type (see IETF RFC 2045 [20]) header set to "application/json" and include JSON content for the RecordMeta.

Zero or more Block parts may follow the RecordMeta Part. For each Block part in the HTTP multipart/mixed message, the Block part shall include a Content-ID header identifying the Block with a "world-unique" value (see IETF RFC 2045 [20]), a Content-Type (see IETF RFC 2045 [20]) header indicating the MIME type of the Block (any value), e.g. application/octet-stream, application/json or another applicable MIME type and the Content-Transfer-Encoding (see IETF RFC 2045 [20]) header with an appropriate value.

##### 6.1.2.4.3	BlockCollection

The Record is encoded as an HTTP multipart message with the multipart/parallel content-type as described in IETF RFC 2046 [21].

The boundary parameter is used to delimit each part (Start of parts to Block, Block to Block and Block to End of parts) and shall be set to a value as in accordance with IETF  RFC 2046 [21].

For the BlockCollection, zero or more Block parts may be included. For each Block part in the HTTP multipart/parallel message, the Block part shall include a Content-ID header identifying the Block with a "world-unique" value (see IETF RFC 2045 [20]), a Content-Type header indicating the MIME type of the Block (any value) e.g. application/octet-stream, application/json or another applicable MIME type and the Content-Transfer-Encoding (see IETF RFC 2045 [20]) header with an appropriate value.

##### 6.1.2.4.4	RecordNotification

The RecordNotification is encoded as an HTTP multipart message with the multipart/mixed content-type as described in IETF RFC 2046 [21].

The boundary parameter is used to delimit each part (Start of parts to RecordNotification, RecordNotification to RecordMeta, RecordMeta to Block, Block to Block and Block to End of parts) and shall be set to a value in accordance with IETF  RFC 2046 [21].

The RecordNotification shall be the first part, and shall always be present. It shall include a Content-ID header set to a "world-unique" value (see IETF RFC 2045 [20]), the Content-Type (see IETF RFC 2045 [20]) header set to "application/json" and include JSON content for the NotificationDescription.

The RecordMeta part shall be the second part, and shall always be present. It shall include a Content-ID header set to a "world-unique" value (see IETF RFC 2045 [20]), the Content-Type (see IETF RFC 2045 [20]) header set to "application/json" and include JSON content for the RecordMeta.

Zero or more Block parts may follow the RecordMeta Part. For each Block part in the HTTP multipart/mixed message, the Block part shall include a Content-ID header identifying the Block with a "world-unique" value (see IETF RFC 2045 [20]), a Content-Type (see IETF RFC 2045 [20]) header indicating the MIME type of the Block (any value), e.g. application/octet-stream, application/json or another applicable MIME type and the Content-Transfer-Encoding (see IETF RFC 2045 [20]) header with an appropriate value.

### 6.1.3	Resources

#### 6.1.3.1	Overview

Figure 6.1.3.1-1: Resource URI structure of the nudsf-dr API

Table 6.1.3.1-1 provides an overview of the resources and applicable HTTP methods.

Table 6.1.3.1-1: Resources and methods overview

| Resource name | Resource URI | HTTP method or custom operation | Description |
| --- | --- | --- | --- |
| RecordCollection (Collection) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records | GET | Search for records |
| Record (Document) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId} | GET | Retrieve a record |
|  |  | PUT | Create or update a record |
|  |  | DELETE | Delete a record |
| Meta (Document) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}/meta | GET | Retrieve the meta of a record |
|  |  | PATCH | Modify the meta of a record |
| BlockCollection (Collection) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}/blocks | GET | Retrieve all the blocks of a record |
| Block (Document) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}/blocks/{blockId} | GET | Retrieve a block |
|  |  | PUT | Create or update a block |
|  |  | DELETE | Delete a block |
| NotificationSubscriptions (Collection) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/subs-to-notify | GET | Retrieve existing subscriptions |
| Individual NotificationSubscription (Document) | {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/subs-to-notify/{subscriptionId} | DELETE | Delete the subscription identified by {subscriptionId}, i.e. unsubscribe to notification for change of data |
|  |  | PATCH | Update an individual Subscription to notification |
|  |  | PUT | Create or update a subscription to notification, |
|  |  | GET | Retrieve an individual Subscription to notification |

#### 6.1.3.2	Resource: RecordCollection (Collection)

##### 6.1.3.2.1	Description

This resource represents the collection of records within a storage. It can be used to search for specific records matching specific filter criteria.

##### 6.1.3.2.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}

This resource shall support the resource URI variables defined in table 6.1.3.2.2-1.

Table 6.1.3.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id. |
| {storageId} | Represents the storage Id. |

##### 6.1.3.2.3	Resource Standard Methods

###### 6.1.3.2.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.2.3.1-1.

Table 6.1.3.2.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| filter | SearchExpression | M | 1 | The filter criteria for searching the records of the storage. |  |
| limit-range | Uinteger | C | 0..1 | When set, the returned response shall contain at the most the number of record references specified by the parameter value. If the count-indicator parameter is set in the request, this parameter shall be ignored. |  |
| page-number | Uinteger | O | 0..1 | The record references returned in the response shall start from the indicated page number. If not present, the producer shall use a value of 1 as default value. When this parameter is present with a value of greater than 1, the limit-range parameter shall also be present. |  |
| count-indicator | Boolean | O | 0..1 | If this parameter is set, the number of records that matched the criteria shall be returned and no record references shall be returned. |  |

This method shall support the request data structures specified in table 6.1.3.2.3.1-2 and the response data structures and response codes specified in table 6.1.3.2.3.1-3.

Table 6.1.3.2.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.2.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RecordSearchResult | M | 1 | 200 OK | The search result containing the record references matching the filter. |
| n/a |  |  | 204 No Content | The search did not result in any matching record references. |
| NOTE: The manadatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 6.1.3.3	Resource: Record (Document)

##### 6.1.3.3.1	Description

This resource represents a record within a storage.

##### 6.1.3.3.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/records/{recordId}

This resource shall support the resource URI variables defined in table 6.1.3.3.2-1.

Table 6.1.3.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id where the record is stored |
| {storageId} | Represents the storage Id where the record is stored |
| {recordId} | Represents the record Id of the record |

##### 6.1.3.3.3	Resource Standard Methods

###### 6.1.3.3.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.3.3.1-1.

Table 6.1.3.3.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6. |  |

This method shall support the request data structures specified in table 6.1.3.3.3.1-2 and the response data structures and response codes specified in table 6.1.3.3.3.1-3.

Table 6.1.3.3.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RecordBody | M | 1 | 200 OK | A response body containing the record. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.3.3.2	PUT

This method shall support the URI query parameters specified in table 6.1.3.3.3.2-1.

Table 6.1.3.3.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| get-previous | Boolean | O | 0..1 | Request to return the previous record content if a record already exists in the targeted storage for the same record identifier. |  |

When creating or replacing the record, meta and zero or more blocks shall be included. The record meta information shall be the first part and is mandatory. The remaining parts of the body (if any) shall be the blocks to be updated or created. See clause 6.1.2.4 for details on the encoding.

If the operation updates an existing record, then the existing record and its blocks shall be discarded and replaced by the meta and blocks supplied with the request. This also applies to the case when no new blocks are included, i.e. the old blocks (if any) shall be deleted from the record.

This method shall support the request data structures specified in table 6.1.3.3.3.2-2 and the response data structures and response codes specified in table 6.1.3.3.3.2-3.

Table 6.1.3.3.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Record | M | 1 | The record that is to be created including meta and zero or more blocks. |

Table 6.1.3.3.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RecordBody | M | 1 | 200 OK | Upon successful update of a record, a response body containing the previous record value (if get-previous was indicated in the request, and if one exists) will be returned |
| RecordBody | O |  | 201 Created | Upon successful creation of a record, a response body of the created record shall be returned. If due to operator's policy the value of the ttl (if any) in the request exceeded a maximum allowed ttl value with the ttl set to the value applied by the UDSF. |
| n/a |  |  | 204 No Content | Upon successful update of a record, an empty response is returned or if no previous record value was requested. |
| ProblemDetails | M | 1 | 403 FORBIDDEN | If the UDSF (based on operator policy), determines to apply a ttl value of the recordMeta different from the ttl value (if any) of the request, the get-previous query-parameter is present in the request and the request applies to a record that already exists in the UDSF, the "cause" attribute shall be set to one of the following application errors: -TTL_VALUE_NOT_ALLOWED |
| ProblemDetails | MO | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND |
| RecordBody | M | 1 | 412 Precondition Failed | If one or more conditions given in the request header fields evaluated to false and get-previous was indicated in the request, the UDSF shall include the RecordBody in the response. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.3.3.3	DELETE

This method shall support the URI query parameters specified in table 6.1.3.3.3.3-1.

Table 6.1.3.3.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| get-previous | Boolean | O | 0..1 | Request to return the record content if a record exists in the targeted storage for the same record identifier. |  |

This method shall support the request data structures specified in table 6.1.3.3.3.3-2 and the response data structures and response codes specified in table 6.1.3.3.3.3-3.

Table 6.1.3.3.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.3.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RecordBody | O | 1 | 200 OK | Upon success, and a response body containing the record value with meta and associated blocks (if any), if get-previous was indicated in the request. |
| n/a |  |  | 204 No Content | Upon success and no response body containing the record was requested. |
| RecordBody | M | 1 | 412 Precondition Failed | If one or more conditions given in the request header fields evaluated to false and get-previous was indicated in the request, the UDSF shall include the RecordBody in the response. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 6.1.3.4	Resource: Meta (Document)

##### 6.1.3.4.1	Description

This resource represents the meta associated with a record.

##### 6.1.3.4.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}/meta

This resource shall support the resource URI variables defined in table 6.1.3.4.2-1.

Table 6.1.3.4.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id where the record is stored |
| {storageId} | Represents the storage Id where the record is stored |
| {recordId} | Represents the record Id of the record |

##### 6.1.3.4.3	Resource Standard Methods

###### 6.1.3.4.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.4.3.1-1.

Table 6.1.3.4.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.1.3.4.3.1-2 and the response data structures and response codes specified in table 6.1.3.4.3.1-3.

Table 6.1.3.4.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.4.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| RecordMeta | M | 1 | 200 OK | A response body containing the record meta will be returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute shall be set to one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.4.3.2	PATCH

This method shall support the URI query parameters specified in table 6.1.3.4.3.2-1.

Table 6.1.3.4.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.1.3.4.3.2-2 and the response data structures and response codes specified in table 6.1.3.4.3.2-3.

Table 6.1.3.4.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(patchItem) | M | 1..N | A collection of patch items to apply on the record meta. |

Table 6.1.3.4.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | If one or more modification instructions have been discarded, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute maybe used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND |
| NOTE 1: In addition common data structures as listed in table 6.1.7.3-1are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDSF shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, the UDSF shall respond with PatchResult. |  |  |  |  |

#### 6.1.3.5	Resource: BlockCollection (Collection)

##### 6.1.3.5.1	Description

This resource represents the collection of blocks of associated with a record.

##### 6.1.3.5.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}/blocks

This resource shall support the resource URI variables defined in table 6.1.3.5.2-1.

Table 6.1.3.5.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id where the record is stored |
| {storageId} | Represents the storage Id where the record is stored |
| {recordId} | Represents the record Id of the record |

##### 6.1.3.5.3	Resource Standard Methods

###### 6.1.3.5.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.5.3.1-1.

Table 6.1.3.5.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.1.3.5.3.1-2 and the response data structures and response codes specified in table 6.1.3.5.3.1-3.

Table 6.1.3.5.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.5.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(block) | M | 1 | 200 OK | A response body containing one or more blocks. |
| n/a |  |  | 204 No Content | The BlockCollection did not contain any blocks. |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 6.1.3.6	Resource: Block (Document)

##### 6.1.3.6.1	Description

This resource represents a record within a storage.

##### 6.1.3.6.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/records/{recordId}/blocks/{blockId}

This resource shall support the resource URI variables defined in table 6.1.3.6.2-1.

Table 6.1.3.6.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id where the record is stored |
| {storageId} | Represents the storage Id where the record is stored |
| {recordId} | Represents the record Id of the record |
| {blockId} | Represents the block Id of the block |

##### 6.1.3.6.3	Resource Standard Methods

###### 6.1.3.6.3.1	GET

This method shall support the URI query parameters specified in table 6.1.3.6.3.1-1.

Table 6.1.3.6.3.1-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.1.3.6.3.1-2 and the response data structures and response codes specified in table 6.1.3.6.3.1-3.

Table 6.1.3.6.3.1-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.6.3.1-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BlockBody | M | 1 | 200 OK | Upon success, a response body containing the requested block is returned. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute maybe used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND -BLOCK_NOT_FOUND |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.6.3.2	PUT

This method shall support the URI query parameters specified in table 6.1.3.6.3.2-1.

Table 6.1.3.6.3.2-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| get-previous | Boolean | O | 0..1 | Request to return the previous block content if a block already exists in the targeted storage for the same block identifier. |  |

If the operation updates an existing block, then the existing block shall be discarded and replaced by the block supplied with the request.

This method shall support the request data structures specified in table 6.1.3.6.3.2-2 and the response data structures and response codes specified in table 6.1.3.6.3.2-3.

Table 6.1.3.6.3.2-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| Block | M | 1 | The block definition that shall be created. A Content-Type http header can be set to specify the media type of the block's opaque content. If the media-type is not included, the media-type shall be set to application/octet-stream. |

Table 6.1.3.6.3.2-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BlockBody | M | 1 | 200 OK | Upon successful update of a block, a response body containing the previous record value ((if get-previous was indicated in the request and if one exists) will be returned |
| n/a |  |  | 201 Created | Upon successful creation of a record, an empty response shall be returned. |
| n/a |  |  | 204 No Content | Upon successful update of a record, an empty response is returned if no previous record value was requested. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND |
| BlockBody | M | 1 | 412 Precondition Failed | If one or more conditions given in the request header fields evaluated to false and get-previous was indicated in the request, the UDSF shall include the BlockBody in the response. |
| NOTE: The mandatory HTTP error status code for the PUT method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.6.3.3	DELETE

This method shall support the URI query parameters specified in table 6.1.3.6.3.3-1.

Table 6.1.3.6.3.3-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |
| get-previous | Boolean | O | 0..1 | Request to return the previous block content (if any). |  |

This method shall support the request data structures specified in table 6.1.3.6.3.3-2 and the response data structures and response codes specified in table 6.1.3.6.3.3-3.

Table 6.1.3.6.3.3-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

Table 6.1.3.6.3.3-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| BlockBody | O | 1 | 200 OK | Upon success, and a response body containing the block was requested. |
| n/a |  |  | 204 No Content | Upon success, and no response body containing the block was requested. |
| BlockBody | M | 1 | 412 Precondition Failed | If one or more conditions given in the request header fields evaluated to false and get-previous was indicated in the request, the UDSF shall include the BlockBody in the response. |
| ProblemDetails | O | 0..1 | 404 Not Found | The "cause" attribute may be used to indicate one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -RECORD_NOT_FOUND -BLOCK_NOT_FOUND |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 6.1.3.7	Resource: NotificationSubscriptions

##### 6.1.3.7.1	Description

This resource is used to represent notification subscriptions.

##### 6.1.3.7.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/subs-to-notify

This resource shall support the resource URI variables defined in table 6.1.3.7.2-1.

# Table 6.1.3.7.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id. |
| {storageId} | Represents the storage Id. |

##### 6.1.3.7.3	Standard Methods

###### 6.1.3.7.3.2	GET

# This method shall support the URI query parameters specified in table 6.1.3.7.3.2-1.

# Table 6.1.3.7.3.2-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| limit-range | Uinteger | C | 0..1 | When set, the returned response shall contain at the most the number of Notification Subscriptions specified by the parameter value. |
| page-number | Uinteger | O | 0..1 | The record references returned in the response shall start from the indicated page number. If not present, the producer shall use a value of 1 as default value. When this parameter is present with a value of greater than 1, the limit-range parameter shall also be present. |

This method shall support the request data structures specified in table 6.1.3.7.3.2-2 and the response data structures and response codes specified in table 6.1.3.7.3.2-3.

# Table 6.1.3.7.3.2-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  |  |

# Table 6.1.3.7.3.2-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| array(NotificationSubscription) | M | 0..N | 200 OK | Upon success, a response body containing the individual subscriptions shall be returned. |
| ProblemDetails | O | 0..1 | 404 NOT FOUND | The "cause" attribute shall be set to one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND |
| NOTE: In addition common data structures as listed in table 5.5-1 are supported. |  |  |  |  |

#### 6.1.3.8	Resource: IndividualNotificationSubscription

##### 6.1.3.8.1	Description

This resource is used to represent an individual subscriber data subscriptions to notifications.

##### 6.1.3.8.2	Resource Definition

Resource URI: {apiRoot}/nudsf-dr/v1/{realmId}/{storageId}/subs-to-notify/{subscriptionId}

This resource shall support the resource URI variables defined in table 6.1.3.8.2-1.

# Table 6.1.3.8.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| apiRoot | See clause 6.1.1 |
| {realmId} | Represents the realm Id. |
| {storageId} | Represents the storage Id. |
| {subscriptionId} | The subscriptionId identifies an individual NotficiationSubscription. |

##### 6.1.3.8.3	Resource Standard Methods

###### 6.1.3.8.3.1	DELETE

This method shall support the URI query parameters specified in table 6.1.3.8.3.1-1.

# Table 6.1.3.8.3.1-1: URI query parameters supported by the DELETE method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| client-id | ClientId | M | 0..1 | The client-id is used by the UDSF to guard against deletion of notification subscriptions that do not belong to the same NF or NFSet. |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |
| get-previous | Boolean | O | 0..1 | Request to return the associated NotificationSubscription if it exist. |

This method shall support the request data structures specified in table 6.1.3.8.3.1-2 and the response data structures and response codes specified in table 6.1.3.8.3.1-3.

# Table 6.1.3.8.3.1-2: Data structures supported by the DELETE Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 6.1.3.8.1-3: Data structures supported by the DELETE Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NotificationSubscription | O | 1 | 200 OK | Upon successful delete of a NotificationSubscription, a response body containing the NotificationSubscription value before the delete shall be returned if get-previous was indicated in the request. |
| n/a |  |  | 204 NO CONTENTt | Upon success, an empty response body shall be returned. |
| ProblemDetails | O | 0..1 | 403 FORBIDDEN | If the client-id query parameter does not match the clientId of the existing resource, 403 FORBIDDEN shall be returned and the "cause" attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_EXISTS |
| ProblemDetails | O | 1 | 404 NOT FOUND | The "cause" attribute shall be set to one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -SUBSCRIPTION_NOT_FOUND |
| NotificationSubscription | M | 1 | 412 PRECONDITION FAILED | 412 PRECONDITION FAILED is returned if one or more conditions given in the request header fields evaluated to false. If get-previous was indicated in the request, the UDSF shall include the NotificationSubscription in the response. |
| NOTE: The mandatory HTTP error status code for the DELETE method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.8.3.2	PATCH

This method shall support the URI query parameters specified in table 6.1.3.8.3.2-1.

# Table 6.1.3.8.3.2-1: URI query parameters supported by the PATCH method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |

This method shall support the request data structures specified in table 6.1.3.8.3.2-2 and the response data structures and response codes specified in table 6.1.3.8.3.2-3.

# Table 6.1.3.8.3.2-2: Data structures supported by the PATCH Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| array(PatchItem) | M | 1..N | Contains the delta data of the Notification Subscription to be updated. |

# Table 6.1.3.8.3.2-3: Data structures supported by the PATCH Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 NO CONTENT | Upon successful modification, there is no body in the response message. (NOTE 2) |
| PatchResult | M | 1 | 200 OK | Upon success, the execution report is returned. (NOTE 2) |
| ProblemDetails | O | 0..1 | 404 NOT FOUND | The "cause" attribute shall be set to one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -SUBSCRIPTION_NOT_FOUND |
| NOTE 1: In addition common data structures as listed in table 6.1.7.3-1are supported. NOTE 2: If all the modification instructions in the PATCH request have been implemented, the UDSF shall respond with 204 No Content response; if some of the modification instructions in the PATCH request have been discarded, the UDSF shall respond with 200 OK and include the PatchResult. |  |  |  |  |

###### 6.1.3.8.3.3	GET

This method shall support the URI query parameters specified in table 6.1.3.8.3.3-1.

# Table 6.1.3.8.3.3-1: URI query parameters supported by the GET method on this resource

| Name | Data type | P | Cardinality | Description |
| --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6. |

This method shall support the request data structures specified in table 6.1.3.8.3.3-2 and the response data structures and response codes specified in table 6.1.3.8.3.3-3.

# Table 6.1.3.8.3.3-2: Data structures supported by the GET Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| n/a |  |  | The request body shall be empty. |

# Table 6.1.3.8.3.3-3: Data structures supported by the GET Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NotificationSubscription | M | 1 | 200 OK | Upon success, a response body containing the individual subscriptions shall be returned. |
| ProblemDetails | O | 0..1 | 404 NOT FOUND | The "cause" attribute shall be set to one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND -SUBSCRIPTION_NOT_FOUND |
| NOTE: The mandatory HTTP error status code for the GET method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

###### 6.1.3.8.3.4	PUT

This method shall support the URI query parameters specified in table 6.1.3.8.3.4-1.

Table 6.1.3.8.3.4-1: URI query parameters supported by the PUT method on this resource

| Name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| supported-features | SupportedFeatures | O | 0..1 | see 3GPP TS 29.500 [4] clause 6.6 |  |

This method shall support the request data structures specified in table 6.1.3.8.3.4-2 and the response data structures and response codes specified in table 6.1.3.8.3.4-3.

Table 6.1.3.8.3.4-2: Data structures supported by the PUT Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| NotificationSubscription | M | 1 | The Notification Subscription. |

Table 6.1.3.8.3.4-3: Data structures supported by the PUT Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| NotificationSubscription | O | 1 | 201 CREATED | Upon success, the created NotificationSubscription is returned. The HTTP response shall include a "Location" HTTP header that contains the resource URI of the created resource. |
| NotificationSubscription | O | 1 | 200 OK | Upon success, the updated NotificationSubscription is returned. |
| ProblemDetails | O | 0..1 | 404 NOT FOUND | The "cause" attribute shall be set to one of the following application errors: -REALM_NOT_FOUND -STORAGE_NOT_FOUND |
| ProblemDetails | O | 0..1 | 403 FORBIDDEN | If the clientId of the PUT request does not match the clientId of the existing resource, 403 FORBIDDEN shall be returned and the "cause" attribute may be used to indicate one of the following application errors: - SUBSCRIPTION_EXISTS |
| array(Uri) | O | 1..N | 409 CONFLICT | If one or more monitoredResourceUris from the request don't exist in the UDSF, 409 CONFLICT shall be returned together with the non-existing monitoredResourceUris. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.1.4	Custom Operations without associated resources

None.

### 6.1.5	Notifications

#### 6.1.5.1	General

Notifications shall comply to clause 6.2 of 3GPP TS 29.500 [4] and clause 4.6.2.3 of 3GPP TS 29.501 [5].

#### 6.1.5.2	Timer Expiry Notification

##### 6.1.5.2.1	Description

The Timer Expiry Notification is used by the NF service producer to report to an NF Consumer that the Record has expired as indicated by the ttl attribute (if set) of RecordMeta and if a callbackReference was set in the RecordMeta.

##### 6.1.5.2.2	Resource URI

The Notification URI "{callbackReference}" shall be used with the resource URI variables defined in table 6.1.5.2.2-1.

Table 6.1.5.2.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| callbackReference | String formatted as URI with the Notification Uri |

##### 6.1.5.2.3	Standard Methods

###### 6.1.5.2.3.1	POST

This method shall support the request data structures specified in table 6.1.5.2.3.1-1 and the response data structures and response codes specified in table 6.1.5.2.3.1-1.

Table 6.1.5.2.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RecordBody | M | 1 | The RecordBody of the record that was deleted. |

Table 6.1.5.2.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 No Content | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

#### 6.1.5.3	Notification due to Data Change

##### 6.1.5.3.1	Description

The Notification due to Data Change is used by the UDSF to report to an NF Consumer that a Record that is part of a NotificationSubscription has been updated or deleted.

##### 6.1.5.3.2	Resource URI

The Notification URI "{callbackReference}" shall be used with the resource URI variables defined in table 6.1.5.3.2-1.

Table 6.1.5.3.2-1: Resource URI variables for this resource

| Name | Definition |
| --- | --- |
| callbackReference | String formatted as URI with the Notification Uri |

##### 6.1.5.3.3	Standard Methods

###### 6.1.5.3.3.1	POST

This method shall support the request data structures specified in table 6.1.5.3.3.1-1 and the response data structures and response codes specified in table 6.1.5.3.3.1-1.

Table 6.1.5.3.3.1-2: Data structures supported by the POST Request Body on this resource

| Data type | P | Cardinality | Description |
| --- | --- | --- | --- |
| RecordNotification | M | 1 | The notification with the record information. |

Table 6.1.5.3.3.1-3: Data structures supported by the POST Response Body on this resource

| Data type | P | Cardinality | Response codes | Description |
| --- | --- | --- | --- | --- |
| n/a |  |  | 204 NO CONTENT | Upon success, an empty response body shall be returned. |
| NOTE: The mandatory HTTP error status codes for the POST method listed in Table 5.2.7.1-1 of 3GPP TS 29.500 [4] also apply. |  |  |  |  |

### 6.1.6	Data Model

#### 6.1.6.1	General

This clause specifies the application data model supported by the API.

Table 6.1.6.1-1 specifies the data types defined for the Nudsf service based interface protocol. For simple data types defined for the Nudsf_DataRepository service API see table 6.1.6.3.2-1.

Table 6.1.6.1-1: Nudsf specific Data Types

| Data type | Clause defined | Description | Applicability |
| --- | --- | --- | --- |
| RecordSearchResult | 6.1.6.2.2 | Record Search Result |  |
| RecordMeta | 6.1.6.2.3 | Record Meta |  |
| RecordBody | 6.1.6.2.4 | Record Body |  |
| Record | 6.1.6.2.5 | Record |  |
| BlockBody | 6.1.6.2.6 | Block Body |  |
| Block | 6.1.6.2.7 | Block |  |
| SearchCondition | 6.1.6.2.8 | Search Condition |  |
| SearchComparison | 6.1.6.2.9 | Search Comparison |  |
| ComparisonOperator | 6.1.6.3.3 | Comparison Operator |  |
| ConditionOperator | 6.1.6.3.4 | Condition Operator |  |
| SearchExpression | 6.1.6.4.1 | Search Expression |  |
| NotificationSubscription | 6.1.6.2.10 | Notification Subscription |  |
| RecordNotification | 6.1.6.2.11 | Record Notification |  |
| NotificationDescription | 6.1.6.2.12 | Notification Description |  |
| SubscriptionFilter | 6.1.6.2.13 | Subscription Filter |  |
| ClientId | 6.1.6.2.14 | Client Identity |  |
| RecordOperation | 6.1.6.3.15 | Record Operation |  |

Table 6.1.6.1-2 specifies data types re-used by the Nudsf service based interface protocol from other specifications, including a reference to their respective specifications and when needed, a short description of their use within the Nudsf service based interface.

Table 6.1.6.1-2: Nudsf re-used Data Types

| Data type | Reference | Comments | Applicability |
| --- | --- | --- | --- |
| SupportedFeatures | 3GPP TS 29.571 [19] | see 3GPP TS 29.500 [4] clause 6.6. |  |
| PatchItem | 3GPP TS 29.571 [19] | Data structure used for JSON patch. |  |
| PatchResult | 3GPP TS 29.571 [19] |  |  |
| Uri | 3GPP TS 29.571 [19] |  |  |
| DateTime | 3GPP TS 29.571 [19] |  |  |
| NfInstanceId | 3GPP TS 29.571 [19] |  |  |
| NfSetId | 3GPP TS 29.571 [19] |  |  |

#### 6.1.6.2	Structured data types

##### 6.1.6.2.1	Introduction

This clause defines the structures to be used in resource representations.

##### 6.1.6.2.2	Type: RecordSearchResult

Table 6.1.6.2.2-1: Definition of type RecordSearchResult

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| count | Uinteger | M | 1 | The number of records found by the search. |  |
| references | array(Uri) | O | 1..N | The Record references found by the search. |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | See clause 6.1.8 |  |

##### 6.1.6.2.3	Type: RecordMeta

Table 6.1.6.2.3-1: Definition of type RecordMeta

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| tags | map(array(string)) | O | 1..N | A map of tag name/values pairs, where the tag name is a unique string name that is the primary key of the map and is paired with an array of string values. |  |
| ttl | DateTime | O | 0..1 | ttl refers to the lifetime of the record. After the expiry, the record shall be deleted. |  |
| callbackReference | Uri | O | 0..1 | The Uri where the NF Service Consumer shall receive notification on the expiry of the Record as indicated by the ttl attribute if desired. |  |

##### 6.1.6.2.4	Type: RecordBody

Table 6.1.6.2.4-1: Definition of type RecordBody

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| record | Record | M | 1 | The record. |  |

##### 6.1.6.2.5	Type: Record

Table 6.1.6.2.5-1: Definition of type Record

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| meta | RecordMeta | M | 1 | The meta of a record. |  |
| blocks | array(Block) | O | 1..N | The block(s) (if any) making up the record. |  |

##### 6.1.6.2.6	Type: BlockBody

Table 6.1.6.2.6-1: Definition of type BlockBody

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| n/a | Block | M | 1 | The block. |  |

##### 6.1.6.2.7	Type: Block

Table 6.1.6.2.7-1: Definition of type Block

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| value | Any Type | M | 1 | The block value of any data type. |  |

##### 6.1.6.2.8	Type: SearchCondition

Table 6.1.6.2.8-1: Definition of type SearchCondition

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| cond | ConditionOperator | M | 1 | Logical operator ("AND", "OR" or "NOT") |  |
| units | array(SearchExpression) | M | 1..N | For the logical "NOT" operator indicated in the cond attribute, only one member shall be present in the array. For the logical "AND" or "OR" operators indicated in the cond attribute, at least two members shall be present in the array and all the members in the array shall be interpreted as logically concatenated with the logical operator. |  |

#### 6.1.6.2.9	SearchComparison

Table 6.1.6.2.9-1: Definition of type SearchComparison

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| op | ComparisonOperator | M | 1 | Comparison operator |  |
| tag | string | M | 1 | This attribute contains the tag name of an array of strings. |  |
| value | string | M | 1 | The array of strings indicated in the tag attribute compares to the value of this attribute. |  |

##### 6.1.6.2.10	Type: NotificationSubscription

# Table 6.1.6.2.10-1: Definition of type NotificationSubscription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| clientId | ClientId | M | 1 | Identity of the NF or NFSet for which the subscription applies. |  |
| subscriptionId | string | C | 0..1 | The subscriptionId uniquely identifies the subscription to notification within a storage. Shall be included in subscription responses and notifications. If included in a PUT request body with a value different from the one indicated by the request URL, then the value provided by the request URL takes precedence. |  |
| callbackReference | Uri | M | 1 | Identifies the NF or NF pool where the notification shall be sent. |  |
| expiry | DateTime | C | 0..1 | This IE shall be included in a subscription response, if, based on operator policy and taking into account the expiry time included in the request, the UDSF needs to include an expiry time. The expiry time, based on operator policy, may indicate a value that is sooner than the NF consumer requested. The absence of this attribute in the subscription response indicates that the subscription does not have an expiry time. |  |
| subFilter | SubscriptionFilter | O | 0..1 | If not included, the subscription applies to all Create, Delete and Update events of the storage. |  |
| supportedFeatures | SupportedFeatures | O | 0..1 | Used to negotiate the applicability of optional features |  |

##### 6.1.6.2.11	Type: RecordNotification

Table 6.1.6.2.11-1: Definition of type RecordNotification

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| descriptor | NotificationDescription | M | 1 | The block value of any data type. |  |
| meta | RecordMeta | M | 1 | The meta of a record. |  |
| blocks | array(Block) | O | 1..n | The block(s) (if any) making up the record. |  |

##### 6.1.6.2.12	Type: NotificationDescription

Table 6.1.6.2.12-1: Definition of type NotificationDescription

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| recordRef | Uri | M | 1 | The reference of the record triggering the Notification. |  |
| operationType | RecordOperation | M | 1 | The operation type. |  |

##### 6.1.6.2.13	Type: SubscriptionFilter

Table 6.1.6.2.13-1: Definition of type SubscriptionFilter

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| monitoredResourceUris | array(Uri) | O | 1..N | A set of URIs that identify the records for which a modification of the representation triggers a notification. The URI shall take the form of either an absolute URI or an absolute-path reference as defined in IETF RFC 3986 [22]. The monitored resource shall indicate an existing record. If not present, the subscription is applied to all records within the storage and a modification of the representation of any record within the storage triggers a notification. |  |
| operations | array(RecordOperation) | O | 0..3 | The operations that shall generate a notification. If the monitoredResourceUris is present, only "UPDATED" and "DELETED" are allowed values, any other value shall be ignored. If the attribute is not present, all applicable operations shall apply to the subscription. |  |

##### 6.1.6.2.14	Type: ClientId

Table 6.1.6.2.14-1: Definition of type ClientId

| Attribute name | Data type | P | Cardinality | Description | Applicability |
| --- | --- | --- | --- | --- | --- |
| nfId | NfInstanceId | C | 0..1 | The NF Instance Id uniquely identifying the NF Consumer. Shall be present if the nfSetId is absent. |  |
| nfSetId | NfSetId | C | 0..1 | The NF Set Id of the NF Consumer. Shall be present if the nfId is absent. |  |

##### 6.1.6.3.15	Enumeration: RecordOperation

Table 6.1.6.3.15-1: Enumeration RecordOperation

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "CREATED" | Indicates a Create record operation |  |
| "UPDATED" | Indicates an Update record operation |  |
| "DELETED" | Indicates a Delete record operation |  |

#### 6.1.6.3	Simple data types and enumerations

##### 6.1.6.3.1	Introduction

This clause defines simple data types and enumerations that can be referenced from data structures defined in the previous clauses.

##### 6.1.6.3.2	Simple data types

The simple data types defined in table 6.1.6.3.2-1 shall be supported.

Table 6.1.6.3.2-1: Simple data types

| Type Name | Type Definition | Description | Applicability |
| --- | --- | --- | --- |

##### 6.1.6.3.3	Enumeration: ComparisonOperator

The enumeration ComparisonOperator represents the comparison of an array of strings to a string value. The comparison shall be based on lexicographical order. It shall comply with the provisions defined in table 6.1.5.3.3-1.

Table 6.1.6.3.3-1: Enumeration ComparisonOperator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "EQ" | The array contains the string value. |  |
| "NEQ" | The array does not contain the string value. | AdvancedQuery |
| "GT" | The array contains a string that is greater than the string value. | AdvancedQuery |
| "GTE" | The array contains a string that is greater than or equal to the string value. | AdvancedQuery |
| "LT" | The array contains a string that is less than the string value. | AdvancedQuery |
| "LTE" | The array contains a string that is less than or equal to the string value. | AdvancedQuery |

NOTE:	It's recommended to use GT/GTE/LT/LTE on single value tags. If not, the logical operator "NOT" applied over the comparison operator "GT" evaluates to "true" if there are no members in the array that are greater than the value.

##### 6.1.6.3.4	Enumeration: ConditionOperator

Table 6.1.6.3.4-1: Enumeration ConditionOperator

| Enumeration value | Description | Applicability |
| --- | --- | --- |
| "AND" | Logical "AND" |  |
| "OR" | Logical "OR" |  |
| "NOT" | Logical "NOT" |  |

#### 6.1.6.4	Data types describing alternative data types or combinations of data types

##### 6.1.6.4.1	Type: SearchExpression

Table 6.1.6.4.1-1: Definition of type SearchExpression as a list of mutually exclusive alternatives

| Data type | Cardinality | Description | Applicability |
| --- | --- | --- | --- |
| SearchCondition | 1 | A search expression with logic operators | AdvancedQuery |
| SearchComparison | 1 | A minimum unit of the search expression |  |

### 6.1.7	Error Handling

#### 6.1.7.1	General

For the Nudsf_DataRepository API, HTTP error responses shall be supported as specified in clause 4.8 of 3GPP TS 29.501 [5]. Protocol errors and application errors specified in table 5.2.7.2-1 of 3GPP TS 29.500 [4] shall be supported for an HTTP method if the corresponding HTTP status codes are specified as mandatory for that HTTP method in table 5.2.7.1-1 of 3GPP TS 29.500 [4].

In addition, the requirements in the following clauses are applicable for the Nudsf_DataRepository API.

#### 6.1.7.2	Protocol Errors

Protocol errors handling shall be supported as specified in clause 5.2.7 of 3GPP TS 29.500 [4].

#### 6.1.7.3	Application Errors

The application errors defined for the Nudsf_DataRepository service are listed in Table 6.1.7.3-1.

Table 6.1.7.3-1: Application errors

| Application Error | HTTP status code | Description |
| --- | --- | --- |
| TTL_VALUE_NOT_ALLOWED | 403 Forbidden | The ttl value indicated in the request exceeds the maximum value allowed in the UDSF. |
| REALM_NOT_FOUND | 404 Not Found | The realm indicated in the HTTP/2 request is unavailable in the UDSF. |
| STORAGE_NOT_FOUND | 404 Not Found | The storage indicated in the HTTP/2 request is unavailable in the UDSF. |
| RECORD_NOT_FOUND | 404 Not Found | The record indicated in the HTTP/2 request is unavailable in the UDSF. |
| BLOCK_NOT_FOUND | 404 Not Found | The block indicated in the HTTP/2 request is unavailable in the UDSF. |
| SUBSCRIPTION_NOT_FOUND | 404 Not Found | The subscription indicated in the HTTP/2 request is unavailable in the UDSF. |
| SUBSCRIPTION_EXISTS | 409 Conflict | The subscription indicated in the HTTP/2 request already exists in the UDSF. |

## 6.1.8	Feature negotiation

The optional features in table 6.1.8-1 are defined for the Nudsf_DataRepository API. They shall be negotiated using the extensibility mechanism defined in clause 6.6 of 3GPP TS 29.500 [4].

Table 6.1.8-1: Supported Features

| Feature number | Feature Name | Description |
| --- | --- | --- |
| 1 | AdvancedQuery | If an NF consumer detects that the UDSF supports the AdvancedQuery feature, it may use values of the ComparisonOperator besides "EQ" and may also use the cond attribute of the SearchCondition. If an NF consumer detects that the UDSF does not support the AdvancedQuery feature, it shall only use a value of "EQ" of the ComparisonOperator and shall not use the cond attribute of the SearchCondition. |

## 6.1.9	Security

As indicated in 3GPP TS 33.501 [8] and 3GPP TS 29.500 [4], the access to the Nudsf_DataRepository API may be authorized by means of the OAuth2 protocol (see IETF RFC 6749 [9]), based on local configuration, using the "Client Credentials" authorization grant, where the NRF (see 3GPP TS 29.510 [10]) plays the role of the authorization server.

If OAuth2 is used, an NF Service Consumer, prior to consuming services offered by the <API Name> API, shall obtain a "token" from the authorization server, by invoking the Access Token Request service, as described in 3GPP TS 29.510 [10], clause 5.4.2.2.

NOTE:	When multiple NRFs are deployed in a network, the NRF used as authorization server is the same NRF that the NF Service Consumer used for discovering the Nudsf_DataRepository service.

The Nudsf_DataRepository API defines a single scope "nudsf-dr" for the entire service, and it does not define any additional scopes at resource or operation level.

Annex A (normative):
OpenAPI specification

## A.1	General

This Annex specifies the formal definition of the API(s) defined in the present specification. It consists of OpenAPI 3.0.0 specifications in YAML format.

This Annex takes precedence when being discrepant to other parts of the specification with respect to the encoding of information elements and methods within the API(s).

NOTE 1:	The semantics and procedures, as well as conditions, e.g. for the applicability and allowed combinations of attributes or values, not expressed in the OpenAPI definitions but defined in other parts of the specification also apply.

Informative copies of the OpenAPI specification files contained in this 3GPP Technical Specification are available on a Git-based repository hosted in ETSI Forge, that uses the GitLab software version control system (see 3GPP TS 29.501 [5] clause 5.3.1 and 3GPP TR 21.900 [7] clause 5B).

## A.2	Nudsf_DataRepository API

openapi: 3.0.0

info:

title: Nudsf_DataRepository

version: 1.0.0

description: |

Nudsf Data Repository Service.

© 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

externalDocs:

description: 3GPP TS 29.598 UDSF Services, V16.1.0.

url: 'http://www.3gpp.org/ftp/Specs/archive/29_series/29.598/'

servers:

- url: '{apiRoot}/nudsf-dr/v1'

variables:

apiRoot:

default: https://example.com

description: apiRoot as defined in clause 4.4 of 3GPP TS 29.501

security:

- {}

- oAuth2ClientCredentials:

- nudsf-dr

paths:

'/{realmId}/{storageId}/records':

summary: Access to all Records of a Storage

description: >-

root of all Records of a Storage

get:

summary:     Records search with get

description: Retrieve one or multiple Records based on filter

operationId: SearchRecord

tags:

- Record CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: limit-range

in: query

description: The number of record references to fetch per page

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: page-number

in: query

description: The record references page number to fetch

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: filter

in: query

description: Query filter using conditions on tags

content:

application/json:

schema:

$ref: '#/components/schemas/SearchExpression'

- name: count-indicator

in: query

description: Indicates that no more than the number of matching records shall be returned

schema:

type: boolean

default: false

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Successful case. Response contains result of the search.

content:

application/json:

schema:

$ref: '#/components/schemas/RecordSearchResult'

'204':

description: >-

The search condition does not match any Record.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'406':

$ref: 'TS29571_CommonData.yaml#/components/responses/406'

'429':

$ref: 'TS29571_CommonData.yaml#/components/responses/429'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/{realmId}/{storageId}/records/{recordId}':

summary: Access to a specific Record, identified by its RecordId

description: >-

Access to a specific Record

get:

summary: Record access

description: retrieve one specific Record

operationId: GetRecord

tags:

- Record CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200' : #result ok

$ref: '#/components/responses/RecordBody'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create/Modify Record

description: Create or Modify a Record with a user provided RecordId

operationId: CreateOrModifyRecord

tags:

- Record CRUD

parameters:

- name: realmId

in: path

description: Identifier(name) of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: UserRecordValue000000001

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: get-previous

in: query

description: Retrieve the Record before update

required: false

schema:

type: boolean

default: false

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

$ref: '#/components/requestBodies/RecordBody'

callbacks:

recordExpired:

'{$request.body#/callbackReference}':

post:

parameters:

- name: Content-Location

in: header

description: The expired record URI

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

requestBody:

$ref: '#/components/requestBodies/RecordBody'

responses:

'204':

description: Callback executed successfully

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

responses:

'200' : # Update with return

$ref: '#/components/responses/RecordBody'

'201':

description: >-

Create case. The resource has been successfully created, location header indicates

the URI of the created Record.

$ref: '#/components/responses/RecordBody'

headers:

Location:

$ref: '#/components/headers/Location'

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'204': # Update without return

description: >-

Update case. The resource has been successfully updated and no

additional content is included in the response message.

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'412': # Return Record value if get-previous=true

$ref: '#/components/responses/RecordBody'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a Record with an user provided RecordId

operationId: DeleteRecord

tags:

- Record CRUD

parameters:

- name: realmId

in: path

description: Identifier(name) of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: UserRecordValue000000001

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: get-previous

in: query

description: Retrieve the Record before delete

required: false

schema:

type: boolean

default: false

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

$ref: '#/components/responses/RecordBodyDelete'

'204':

description: Successful case.

headers:

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'412': # Return return value if get-previous=true

$ref: '#/components/responses/RecordBody'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/{realmId}/{storageId}/records/{recordId}/meta':

summary: Access to the meta of a specific Record, identified by its RecordId

description: >-

Access to the meta of a specific Record

get:

summary: Record's meta access

description: retrieve meta of a specific Record

operationId: GetMeta

tags:

- Record CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

content:

application/json:

schema:

$ref: '#/components/schemas/RecordMeta'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch: # patch meta data

summary: Record's meta update

description: update meta of a specific Record

operationId: UpdateMeta

tags:

- Record CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

description: Meta data to patch

content:

application/json-patch+json:

example: '[{ "op": "replace", "path": "/tags/ueId", "value": "450005" }, { "op": "remove", "path": "/tags/recordId" }]'

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'200':

description: >-

One or more modification instructions have been discarded, the execution report is returned in response PatchResult.

content:

application/json:

example:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'204':

description: >-

Successful case. The meta has been successfully updated and no return is expected.

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/{realmId}/{storageId}/records/{recordId}/blocks':

summary: Access to the Blocks of a specific Record, identified by its RecordId

description: >-

Access to the Blocks of a specific Record

get:

summary: Record's Blocks access

description: retrieve all Blocks of a specific Record

operationId: GetBlockList

tags:

- Block CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a successful request

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

content:

multipart/parallel:

schema:

type: object

properties:

blocks:

type: array

description: >-

an array of Block parts, can be empty

items:

$ref: '#/components/schemas/Block'

encoding:

blocks:

contentType: '*/*' # Block content type can be of any type.

headers:

Content-ID: # Block identifier is defined by the Content-Id header.

schema:

type: string

required: true

Content-Transfer-Encoding:

schema:

type: string

required: true

'204':

description: Successful response, the record contains no blocks

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/{realmId}/{storageId}/records/{recordId}/blocks/{blockId}':

summary: Access to a Block of a specific Record, identified by its BlockId

description: >-

Access to a specific Block of a specific Record

get:

summary: Retrieve a specific Block

description: retrieve a specific Block

operationId: GetBlock

tags:

- Block CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: blockId

in: path

description: Id of the Block

required: true

schema:

type: string

example: 'userDefjson01'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

$ref: '#/components/responses/BlockBody'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: Create or Update a specific Block in a Record.

description: Create or update a specific Block, related to a Record

operationId: CreateOrModifyBlock

tags:

- Block CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: blockId

in: path

description: Id of the Block

required: true

schema:

type: string

example: 'userDefjson01'

- name: get-previous

in: query

description: Retrieve the Block before update

required: false

schema:

type: boolean

default: false

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

description: information on the Block to create

required: true

content:

'*/*':

schema:

$ref: '#/components/schemas/Block'

responses:

'200':

$ref: '#/components/responses/BlockBody'

'201':

description: >-

Creation case. The Block has been successfully created. Location header indicates the URI of the created Block.

headers:

Location:

$ref: '#/components/headers/Location'

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'204':

description: >-

Successful case. The resource has been successfully updated.

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'412': # Return previous Block value if get-previous=true

$ref: '#/components/responses/BlockBody'

'413':

$ref: 'TS29571_CommonData.yaml#/components/responses/413'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a specific Block. Then update the Record

description: delete a specific Block, related to a Record

operationId: DeleteBlock

tags:

- Block CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: recordId

in: path

description: Identifier of the Record

required: true

schema:

type: string

example: 'UserRecordValue000000001'

- name: blockId

in: path

description: Id of the Block

required: true

schema:

type: string

example: 'userDefjson01'

- name: get-previous

in: query

description: Retrieve the Block before delete

required: false

schema:

type: boolean

default: false

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

$ref: '#/components/responses/BlockBodyDelete'

'204':

description: >-

Successful case. The Block has been successfully deleted.

headers:

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'412': # Return previous Block value if get-previous=true

$ref: '#/components/responses/BlockBody'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/{realmId}/{storageId}/subs-to-notify':

summary: The notification subscription collection resource

description: >-

Access to the subscription resource

get:

summary: Notification subscription retrieval

description: retrieve all notification subscriptions of the storage

operationId: GetNotificationSubscriptions

tags:

- NotificationSubscriptions CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: limit-range

in: query

description: The number of NotificationSubscriptions to fetch per page

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: page-number

in: query

description: The NotificationSubscription page number to fetch

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Expected response to a valid request

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NotificationSubscription'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

'/{realmId}/{storageId}/subs-to-notify/{subscriptionId}':

summary: The notification subscription resource

description: >-

Access to the subscription resource

get:

summary: Notification subscription retrieval

description: retrieve a single notification subscription of the storage

operationId: GetNotificationSubscription

tags:

- NotificationSubscription CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: subscriptionId

in: path

description: Identifier of the NotificationSubscription

required: true

schema:

type: string

example: Subscription01

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Modified-Since

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.3

schema:

type: string

responses:

'200':

description: Expected response to a valid request

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

content:

application/json:

schema:

$ref: '#/components/schemas/NotificationSubscription'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

delete:

summary: Delete a Notification Subscription of the storage

description: delete a single subscriptions of the storage

operationId: DeleteNotificationSubscription

tags:

- NotificationSubscription CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: subscriptionId

in: path

description: Identifier of the NotificationSubscription

required: true

schema:

type: string

example: Subscription01

- name: client-id

in: query

description: Identifies the NF or NFSet

required: true

schema:

$ref: '#/components/schemas/ClientId'

- name: get-previous

in: query

description: Retrieve the NotificationSubscription before delete

required: false

schema:

type: boolean

default: false

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

responses:

'200':

description: Deleted NotificationSubscription if requested with get-previous

content:

application/json:

schema:

type: array

items:

$ref: '#/components/schemas/NotificationSubscription'

'204':

description: >-

Successful case. The SubscriptionNotification has been successfully deleted.

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'412':

description: Return previous NotificationSubscription value if get-previous=true

content:

application/json:

schema:

$ref: '#/components/schemas/NotificationSubscription'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

patch: # patch NotificationSubscription data

summary: NotificationSubscription update

description: update a specific NotificationSubscription

operationId: UpdateNotificationSubscription

tags:

- NotificationSubscription CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: subscriptionId

in: path

description: Identifier of the NotificationSubscription

required: true

schema:

type: string

example: Subscription01

- name: If-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

requestBody:

description: data to patch

content:

application/json-patch+json:

example: 'TBD'

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchItem'

required: true

responses:

'200':

description: >-

One or more modification instructions have been discarded, the execution report is returned in response PatchResult.

content:

application/json:

example:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/PatchResult'

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'204':

description: >-

Successful case. The meta has been successfully updated and no return is expected.

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

put:

summary: NotificationSubscription Create/Update

operationId: CreateAndUpdateNotificationSubscription

tags:

- NotificationSubscription CRUD

parameters:

- name: realmId

in: path

description: Identifier of the Realm

required: true

schema:

type: string

example: Realm01

- name: storageId

in: path

description: Identifier of the Storage

required: true

schema:

type: string

example: Storage01

- name: subscriptionId

in: path

description: Identifier of the NotificationSubscription

required: true

schema:

type: string

example: Subscription01

- name: supported-features

in: query

description: Features required to be supported by the target NF

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

- name: If-None-Match

in: header

description: Validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

- name: If-Match

in: header

description: Record validator for conditional requests, as described in RFC 7232, 3.2

schema:

type: string

requestBody:

content:

application/json:

schema:

$ref: '#/components/schemas/NotificationSubscription'

required: true

responses:

'200' : # Update

description: Expected response to a valid update request

content:

application/json:

schema:

$ref: '#/components/schemas/NotificationSubscription'

'201':

description: Expected response to a valid create request

content:

application/json:

schema:

$ref: '#/components/schemas/NotificationSubscription'

headers:

Location:

description: 'Contains the URI of the newly created resource according to the structure: {apiRoot}/nudsf-dr/<apiVersion>/{realmId}/{storageId}/subs-to-notify/{subscriptionId}'

required: true

schema:

type: string

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'304':

$ref: '#/components/responses/304'

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'404':

$ref: 'TS29571_CommonData.yaml#/components/responses/404'

'408':

$ref: 'TS29571_CommonData.yaml#/components/responses/408'

'409':

description: Conflict

content:

application/json:

schema:

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

'412':

$ref: 'TS29571_CommonData.yaml#/components/responses/412'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

description: Unexpected error

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

callbacks:

onDataChange:

'{request.body#/callbackReference}':

post:

requestBody:

$ref: '#/components/requestBodies/RecordNotificationBody'

responses:

'204':

description: Callback executed successfully

'400':

$ref: 'TS29571_CommonData.yaml#/components/responses/400'

'401':

$ref: 'TS29571_CommonData.yaml#/components/responses/401'

'403':

$ref: 'TS29571_CommonData.yaml#/components/responses/403'

'500':

$ref: 'TS29571_CommonData.yaml#/components/responses/500'

'503':

$ref: 'TS29571_CommonData.yaml#/components/responses/503'

default:

$ref: 'TS29571_CommonData.yaml#/components/responses/default'

components:

securitySchemes:

oAuth2ClientCredentials:

type: oauth2

flows:

clientCredentials:

tokenUrl: '{nrfApiRoot}/oauth2/token'

scopes:

nudsf-dr: Access to the nudsf-dr API

schemas:

RecordSearchResult:

description: Count and collection of Record references matching the providing filter and paging parameters.

type: object

properties:

count:      # The number of elements returned.

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uinteger'

references: # The Record references found. If count-indicator is true, no references are sent back.

type: array

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- count

RecordMeta:

description: Meta data of a Record

type: object

properties:

ttl:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

tags:

type: object # dictionary type

description: >-

A dictionary of {"tagName": [ "tagValue", ...] }. A tag name can be used to retrieve a Record. The tagValue are unique.

additionalProperties:

type: array

items:

type: string

uniqueItems: true

example:  '{"ueId" : [ "455345", "455346" ], "recordId" : [ "1000106" ] }'

example: >-

{ "tags" : {"ueId" : [ "455345", "455346" ], "recordId" : [ "1000106" ] }}

Record:

description: Definition of a Record

type: object

properties:

meta:

# json representation of the Meta Data

$ref: '#/components/schemas/RecordMeta'

blocks:

# List of multipart data

type: array

description: list of opaque Block's in this Record

items:

$ref: '#/components/schemas/Block'

required:

- meta

example: >-

{"meta": { "tags" : {"tag1" : ["value1"], "tag2" :["value2"] } }, "blocks": [{"Content-ID": "userDefBinaryBlob", "Content-Type": "text/plain", "content": "QmxvY2sgY29udGVudA=="}, {"Content-Id": "userDefJsonBlob", "Content-Type": "application/json", "content": "{"key": "ftsimpletype-999550000000002", "value": "A3E71A78377179B5B91A;imsi-999550000000123"}]}

Block:

description: A Block can be of any type

example: >-

"QmxvY2sgY29udGVudA=="

NotificationSubscription:

description: Definition of a notification subscription

type: object

properties:

clientId:

$ref: '#/components/schemas/ClientId'

subscriptionId:

# unique identifier of the NotificationSubscription

type: string

callbackReference:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

expiry:

$ref: 'TS29571_CommonData.yaml#/components/schemas/DateTime'

subFilter:

$ref: '#/components/schemas/SubscriptionFilter'

supportedFeatures:

$ref: 'TS29571_CommonData.yaml#/components/schemas/SupportedFeatures'

required:

- clientId

- callbackReference

RecordNotification:

description: Definition of a notification on a record

type: object

properties:

descriptor:

# json representation of the notification description

$ref: '#/components/schemas/NotificationDescription'

meta:

# json representation of the Meta Data

$ref: '#/components/schemas/RecordMeta'

blocks:

# List of multipart data

type: array

description: list of opaque Block's in this Record

items:

$ref: '#/components/schemas/Block'

required:

- meta

example: >-

{"descriptor": { "recordRef" : "...", "operationType" : "DELETED"}, "meta": { "tags" : {"tag1" : ["value1"], "tag2" :["value2"] } }, "blocks": [{"Content-ID": "userDefBinaryBlob", "Content-Type": "text/plain", "content": "QmxvY2sgY29udGVudA=="}, {"Content-Id": "userDefJsonBlob", "Content-Type": "application/json", "content": "{"key": "ftsimpletype-999550000000002", "value": "A3E71A78377179B5B91A;imsi-999550000000123"}]}

NotificationDescription:

description: Description of a record notification

type: object

properties:

recordRef:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

operationType:

$ref: '#/components/schemas/RecordOperation'

example: >-

{ "record" : "...", "operationType" : "DELETED"}

SubscriptionFilter:

description: A subscription filter

type: object

properties:

monitoredResourceUris:

type: array

description: list of resources applicable to the subscription

items:

$ref: 'TS29571_CommonData.yaml#/components/schemas/Uri'

minItems: 1

operations:

type: array

description: list of resources applicable to the subscription

items:

$ref: '#/components/schemas/RecordOperation'

maxItems: 3

ClientId:

description: Defines the identity of the NF Consumer

type: object

properties:

nfId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfInstanceId'

nfSetId:

$ref: 'TS29571_CommonData.yaml#/components/schemas/NfSetId'

RecordOperation:

description: Indicate operation made on a record

type: string

enum:

- CREATED

- UPDATED

- DELETED

ConditionOperator:

description: TBD

type: string

enum:

- AND

- OR

- NOT

ComparisonOperator:

description: TBD

anyOf:

- type: string

enum:

# Equals

- EQ

# Not Equal

- NEQ

# Greater Than

- GT

# Greater Than or Equal

- GTE

# Less Than

- LT

# Less Than or Equal

- LTE

default: EQ

- type: string

SearchExpression:

description: A logical expression element

type: object

oneOf:

- $ref: '#/components/schemas/SearchCondition'

- $ref: '#/components/schemas/SearchComparison'

example:

{ "cond": "OR", "units": [ { "op": "EQ", "tag" : "ueId", "value" : "455345" }, { "op": "EQ", "tag" : "supi", "value" : "imsi-999559807001001" } ] }

SearchCondition:

description: A logical condition

type: object

properties:

cond:

$ref: '#/components/schemas/ConditionOperator'

units:

type: array

items:

$ref: '#/components/schemas/SearchExpression'

minItems: 1

required:

- cond

- units

example:

{ "cond": "OR", "units": [ { "op": "EQ", "tag" : "ueId", "value" : "455345" }, { "op": "EQ", "tag" : "supi", "value" : "imsi-999559807001001" } ] }

SearchComparison:

description: A comparison to apply on tag/values pairs.

type: object

properties:

op:

$ref: '#/components/schemas/ComparisonOperator'

tag:

type: string

value:

type: string

required:

- tag

- value

example:

{ "op": "EQ", "tag" : "supi", "value" : "imsi-999559807001001" }

headers:

Cache-Control:

description: Cache-Control containing max-age, as described in RFC 7234, 5.2

schema:

type: string

ETag:

description: Entity Tag, containing a strong validator, as described in RFC 7232, 2.3

schema:

type: string

Last-Modified:

description: Timestamp for last modification of the resource, as described in RFC 7232, 2.2

schema:

type: string

Location:

description: Contains the URI of the newly created resource

required: true

schema:

type: string

Retry-After:

description: 'Indicates the time the NF Consumer has to wait before making a new request. It can be a non-negative integer (decimal number) indicating the number of seconds the NF Consumer has to wait before making a new request or an HTTP-date after which the AF can retry a new request.'

schema:

anyOf:

- type: integer

- type: string

requestBodies:

RecordBody:

description: The record multipart request body. The meta part shall be the first part and is mandatory but can be empty and zero or more block parts may follow the meta part.

required: true

content:

multipart/mixed:

schema:

$ref: '#/components/schemas/Record'

encoding:

meta: # The meta part shall be the first part and is mandatory but can be empty

contentType: application/json

headers:

Content-ID:

schema:

type: string

required: true

blocks: # 0 or more block parts may follow the meta part

contentType: '*/*' # Block part can be of any type

headers:

Content-ID: # Block identifier is defined by the Content-ID header.

schema:

type: string

required: true

Content-Transfer-Encoding:

schema:

type: string

required: true

RecordNotificationBody:

description: The record notification multipart request body. The descriptor part shall be the first one, followed by record meta part and by zero or more block parts.

required: true

content:

multipart/mixed:

schema:

$ref: '#/components/schemas/RecordNotification'

encoding:

descriptor: # The descriptor part shall be the first part and is mandatory

contentType: application/json

headers:

Content-ID:

schema:

type: string

required: true

meta: # The meta part shall be the second part and is mandatory but can be empty

contentType: application/json

headers:

Content-ID:

schema:

type: string

required: true

blocks: # 0 or more block parts may follow the meta part

contentType: '*/*' # Block part can be of any type

headers:

Content-ID: # Block identifier is defined by the Content-ID header.

schema:

type: string

required: true

Content-Transfer-Encoding:

schema:

type: string

required: true

responses:

'304': # Etag response if the value might differ from that sent

description: Not Modified

content:

application/problem+json:

schema:

$ref: 'TS29571_CommonData.yaml#/components/schemas/ProblemDetails'

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Retry-After:

$ref: '#/components/headers/Retry-After'

'RecordBody': #  Record value with associated headers

description: >-

- 200 Update. The resource has been successfully updated and previous value must be sent in the response message if requested.

- 200 Get. The resource exists, its value must be sent in the response message

- 412 Precondition Failed, the previous value must be sent in response message if requested.

content:

multipart/mixed:

schema:

$ref: '#/components/schemas/Record'

encoding:

meta: # The meta part shall be the first part and is mandatory but can be empty.

contentType: application/json

headers:

Content-Id: # The meta part is identified by the 'meta' Content-Id header.

schema:

type: string

required: true

blocks: # Zero or more block parts may follow the meta part

contentType: '*/*' # Block parts can be of any type.

headers:

Content-Id: # Block identifier is defined by the Content-Id header.

schema:

type: string

required: true

Content-Transfer-Encoding:

schema:

type: string

required: true

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'RecordBodyDelete': #  Record value with associated headers

description: >-

- 200 Delete. The resource has been successfully delete and previous value must be sent in the response message if requested.

content:

multipart/mixed:

schema:

$ref: '#/components/schemas/Record'

encoding:

meta: # The meta part shall be the first par and is mandatory but can be empty.

contentType: application/json

headers:

Content-ID: # The meta part is identified by the 'meta' Content-Id header.

schema:

type: string

enum:

- meta

required: true

blocks: # Zero or more block parts may follow the meta part.

contentType: '*/*' # Block parts can be of any type.

headers:

Content-ID: # Block identifier is defined by the Content-Id header.

schema:

type: string

required: true

Content-Transfer-Encoding:

schema:

type: string

required: true

headers:

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'BlockBody': #  Block value with associated headers

description: >-

- 200 Update: The resource has been successfully updated and previous value must be sent in the response message if requested.

- 200 Get: The resource exists, its value must be sent in the response message

- 412 Precondition Failed: the previous value must be sent in response message if requested.

content:

'*/*':

schema:

$ref: '#/components/schemas/Block'

headers:

Cache-Control:

$ref: '#/components/headers/Cache-Control'

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

'BlockBodyDelete': #  Block value with associated headers

description: >-

- 200 Delete: The resource has been successfully delete and previous value must be sent in the response message if requested.

content:

'*/*':

schema:

$ref: '#/components/schemas/Block'

headers:

ETag:

$ref: '#/components/headers/ETag'

Last-Modified:

$ref: '#/components/headers/Last-Modified'

Annex B (informative):
Search Examples

The conditional expression is defined by the following Extended Backus-Naur Form (EBNF) [18].

Figure B-1: Search Expression

Search_Expression ::= Search_Comparison | Search_Condition

Figure B-2: Search Comparison

Search_Comparison ::= Tag ( 'EQ' | 'NEQ' | 'GT' | 'GTE' | 'LT' | 'LTE' ) Value

Figure B-3: Search Condition

Search_Condition ::= Search_Expression ( ( 'AND' | 'OR' ) Search_Expression )+

Figure B-4: Search ConditionNot

Search_ConditionNot ::= 'NOT' Search_Expression

Example:

Find all records where the tag "ueId" is equal to "455345" OR the tag "supi" is equal to "imsi-999559807001001":

{

"cond": "OR",

"items": [

{

"op": "EQ",

"tag" : "ueId",

"value" : "455345"

},

{

"op": "EQ",

"tag" : "supi",

"value" : "imsi-999559807001001"

}

]

}

Annex C (informative):
HTTP Multipart Examples

## C.1	General

This clause provides examples of the HTTP multipart messages. The examples do not aim to be a complete representation of the HTTP message, e.g. additional information or headers can be included.

This Annex is informative and the normative descriptions in this specification prevail over the description in this Annex if there is any difference.

## C.2	Example HTTP multipart Record

Content-Type: multipart/mixed; boundary=partboundary

--partboundary

Content-ID: 76e1f8c1-e8ba-47af-8980-e9292bcffd6b

Content-Type: application/json; charset=UTF-8

{"tags":  { "ueId" : ["455345"], "supi" : ["imsi-999559807001001"] } }

--partboundary

Content-ID: 5cda2686-efbb-47e0-a749-a6f92aaa58fb

Content-Type: application/json; charset=UTF-8

Content-Transfer-Encoding: binary

{ "firstName": "John", "lastName": "Doe"}

--partboundary

Content-ID: 25d16458-019d-46a0-af25-92cc1adf2277

Content-Type: image/png

Content-Transfer-Encoding: binary

<binary representation of png>

--partboundary--'

## C.3	Example HTTP multipart BlockCollection

Content-Type: multipart/parallel; boundary=partboundary

--partboundary

Content-ID: 0ecc1f72-70ef-4028-a2eb-1324287e0191

Content-Type: application/json; charset=UTF-8

Content-Transfer-Encoding: binary

{ "firstName": "John", "lastName": "Doe"}

--partboundary

Content-ID: 9e9b8b85-b741-4bd1-b6a7-53cdaea3eaa2

Content-Type: image/png

Content-Transfer-Encoding: base64

iVBORw0KGgoAAAANSUhEUgAAADIAAAAeCAYAAABuUU38AAAAAXNSR0IArs4c6QAAAARnQU1BAACxjwv8YQUAAAAJcEhZcwAADsMAAA7DAcdvqGQAAAAhdEVYdENyZWF0aW9uIFRpbWUAMjAyMDowMjoyMiAxMTowMToxNeLKX7MAAAUTSURBVFhH7Zh5qFVVFIdv2XsN2qRZZoWUPWnSbHrEwwYapMQKbKCRNJ+GmpEN9EdBhCIVRHMREYKWIWqWYTQpKZZmEZTaHPIipcGyySzN9Pvuuys2h3PvPcW70B/vBx/33DPss/faa6+19ulRKqbdIO7dUfn9X2mXym+e9oOx0FqhJ2yE9+BZeB3+hkZpfzgZjoXe8Bu8C8thGxTSQfAlaP1qzIFm6EpprKvgDdgCvmczfAu/VP47mMOhkKZCdPgPeAgeh58q54I7INQCl8PNMAHOhl5QRN53G3wPtrsG7oZTYS/QtTXa8TALvoIBUFcXg26zHSZ6oqKTwPMxkBUwGF6F9HzwKzwIh0ItXQve/z6cAbVcXt0KK6HQGh8EWjnVCEg7/DP8mfyvxu9wL+wLQ+AyuB3uhydgBnjf86Cl6w3EGVoLo8r//oWa4DpwwWU7WRQH7dpbl/z/Gj6Fz8DZ87xesBiOg1rSCE91HtYfeR+4AG6BtOG/QKukshMfwhL4HFykPq+FD4EX4RnQfTbApeBsRuSz/Q9ANz0GDoCzYBXkaQF8A67HqroQjFpZtzFijIYfk3OiVc+EeoZRr4CulKcX4CVwgc8F3SdvHbhW7ZvvrKpdYROkHRUfvBMeSM6JFjbnFNVsMPLkaTgYdp3JPeATuAhSmdOciaehruF0pfWwFdJOZzFB6jbKRn1uHjwHdjiLIdQw7j150vodcH35X6k0GeaDa1TX89kf4C4oWpWUZ6YfXAFvQl54nQQh/TrvnkBL+/L7wKogTyZEr30Hrqd3QE94ufJ/HDhb/1kOygSVdszFnCY8F6eWznNLMck5a1PAHJAnA0h/cDA3gm71FnSptJb5IDpmhMqTITYdQPAk6CK6n9Gplk4EQ7MRy2RZV1q6qE6B3TsPy3KK8553sHlaBHuDs3g0uI6sq0y6DjBkFWDUdGaOANdWIe0D+q3rYCacA2k0MBvfAFG0Ba6HMZDVCRDXrVI91t3CCA7eBe1AroGHwTVg9DP/eM2c48y5Pk6DQrIgzHbQSGEJoRWz2Xw1RPHoArYui4HbScOi16aB1zw2J6RyDRSxtG3Zl0KyAItO1sKK0wJSNxgJkSwd+Ntgxeyses6SY09wFty7WFimsrxY2HlYUwPBoFKo0jUkPgbWOGnHxdlw2q+E7N7jdIi6KcUk1sKoe7OJOJITzR7zm7qrdZsbpCJ6BB7tPKyutHEXnbWNa8b18BG4G9R9qsns64KNUmEpzKLTJtIeZMvz+R3ElK05sFR6jZc5UGV21m2yM5Wng8EodxS4YzR66gFutv5ROpAuk9Znh9TO7wBesAU6YCGJRgMpt7F6gTNdRBpLo1oh94UvQJdrrJiJVlJzCwNphiYSQh+KI0Npw9SQGcF0TYeVSuNpvI2B7IClMBsfabwVu9WtbuUqNifngaEtFqP54Saw9DavnAtuZ61ELegMf8PgEjCxGdvdtxjvDZVmdhOsagNrLjdg5gJzju1eDR/DdLC0sdr1u5glUPTHfnjdPT6pqFxIukfyA13sFO+BVTZgNeqHAHdjIW+0c35jsiQxETloPxG5c/S6+4+h4PV2sGGCVfmFbrJCVrzeo0GsBuyIstx3gO7Ll4ER1GfT/rgbdLAOzpLHax67wXLvb7s+48eQsjWsiXxhKl/sww7AjvsiLRR1j416j4qvgd7vueisims+K1HqxD7f99q2WGln+xPviOeVA1D2jXqs1LwTlVGQ4+/C82oAAAAASUVORK5CYII=

--partboundary--'

## C.4	Example HTTP multipart RecordNotification

Content-Type: multipart/mixed; boundary=partboundary

--partboundary

Content-ID: 860013c6-240e-498a-8110-5a708a96aa54

Content-Type: application/json; charset=UTF-8

{"recordRef" : "...", "operationType" : "DELETED" }

--partboundary

Content-ID: 76e1f8c1-e8ba-47af-8980-e9292bcffd6b

Content-Type: application/json; charset=UTF-8

{"tags":  { "ueId" : ["455345"], "supi" : ["imsi-999559807001001"] } }

--partboundary

Content-ID: 5cda2686-efbb-47e0-a749-a6f92aaa58fb

Content-Type: application/json; charset=UTF-8

Content-Transfer-Encoding: binary

{"firstName": "John", "lastName": "Doe"}

--partboundary

Content-ID: 25d16458-019d-46a0-af25-92cc1adf2277

Content-Type: image/png

Content-Transfer-Encoding: binary

<binary representation of png>

--partboundary--'

Annex D (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2019-11 | CT4#94 | C4-195489 |  |  |  | Initial Draft. | 0.1.0 |
| 2020-03 | CT4#96 | C4-200359 C4-200360 C4-200584 C4-200585 C4-200586 C4-200654 C4-200905 C4-200920 C4-201226 C4-201230 C4-201235 C4-201236 C4-201237 |  |  |  | Implementation of pCRs agreed at CT4#96. | 0.2.0 |
| 2020-03 | CT#87e | CP-200066 |  |  |  | TS presented for information and approval | 1.0.0 |
| 2020-03 | CT#87e |  |  |  |  | Approved at CT#87e | 16.0.0 |
| 2020-06 | CT#88e | CP-201175 | 0001 | 4 | B | Subscribe To Notify | 16.1.0 |
| 2020-06 | CT#88e | CP-201041 | 0002 |  | C | Removal of Editor's Note | 16.1.0 |
| 2020-06 | CT#88e | CP-201041 | 0003 |  | C | Add supportedFeatures to RecordSearchResult | 16.1.0 |
| 2020-06 | CT#88e | CP-201041 | 0004 |  | F | SearchExpression | 16.1.0 |
| 2020-06 | CT#88e | CP-201041 | 0006 | 1 | F | Miscellaneous Corrections | 16.1.0 |
| 2020-06 | CT#88e | CP-201041 | 0007 | 1 | B | Storage of YAML files in ETSI Forge | 16.1.0 |
| 2020-06 | CT#88e | CP-201041 | 0008 |  | F | 204 missing in OpenAPI | 16.1.0 |
| 2020-06 | CT#88e | CP-201073 | 0009 |  | F | Rel-16 API version and External doc update | 16.1.0 |