| 3GPP TR 29.820 V1.2.0 (2022-02) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals Study on Best Practice of PFCP (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Overall Requirements	9

5	Key Issues	10

5.1	General	10

5.2	Key Issue #1: UPF support for multiple network slices	10

5.2.1	Description of the use case	10

5.2.2	Key issue definition	10

5.3	Key Issue #2: Header Enrichment for HTTPS	11

5.3.1	Description of the use case	11

5.3.2	Key issue definition	11

5.4	Key Issue #3: Interoperability issues caused by multiple options co-existence	11

5.4.1	Description of the use case	11

5.4.2	Key issue definition	11

5.5	Key Issue #4: Avoidance of conflict between UE IP address/prefix allocated by SMF(s)	12

5.5.1	Description of the use case	12

5.5.2	Key issue definition	12

5.6	Key Issue #5: L2TP Tunnelling over Sgi/N6	12

5.6.1	Description of the use case	12

5.6.1.1	General	12

5.6.1.2	L2TP Implementation Models	14

5.6.2	Key issue definition	16

6	Solutions	17

6.1	Solution#1: Header Enrichment for HTTPS	17

6.1.1	Description	17

6.1.2	Impacts on services, entities and interfaces	18

6.1.3	Pros	18

6.1.4	Cons	18

6.2	Solution#2: UE IP address/prefix allocation by SMF in SMF Set	18

6.2.1	Description	18

6.2.1.1	General	18

6.2.1.2	SMF recover	19

6.2.1.3	Session release	19

6.2.1.4	SMF added/removed to/from SMF set	19

6.2.1.4.1	General	19

6.2.1.4.2	Solution based on local configuration	19

6.2.1.4.3	Solution based on directly controlled by external entity	19

6.2.2	Signalling flow	20

6.2.2.1	General	20

6.2.2.2	SMF recover	20

6.2.2.3	Session release	20

6.2.2.4	SMF added/removed to/from SMF set	21

6.2.3	Impacts on services, entities and interfaces	22

6.2.4	Pros	22

6.2.5	Cons	22

6.3	Solution #3: Enabling SMF to map network slice information into Network Instance	22

6.3.1	Description	22

6.3.2	Signalling flow	23

6.3.3	Impacts on services, entities and interfaces	24

6.4	Solution #4: Enabling UP function to buffer downlink data as the preferred option	24

6.4.1	Description	24

6.4.2	Impacts on services, entities and interfaces	24

6.5	Solution #5: Enabling UP function to construct End Marker as the preferred option	24

6.5.1	Description	24

6.5.2	Impacts on services, entities and interfaces	25

6.6	Solution #6: Enforce traffic redirection in the UP function as the only option	25

6.6.1	Description	25

6.6.2	Impacts on services, entities and interfaces	25

6.7	Solution#7: Recommending the support of features relating to the core part of the PFCP protocol	26

6.7.1	Description	26

6.7.2	Impacts on services, entities and interfaces	26

6.7.3	Pros	26

6.7.4	Cons	26

6.8	Solution #8: L2TP Tunnel and Session Setup by UP Function	27

6.8.1	Description	27

6.8.2	Protocol impact over N4/Sxb	27

6.8.2.1	L2TP Support Negotiation	27

6.8.2.2	L2TP Tunnel/Session Setup	28

6.8.2.3	L2TP Session/Tunnel Release	30

6.8.3	Impacts on services, entities and interfaces	31

6.8.4	End to End Signalling Flow	31

6.8.5	Security Considerations for L2TP Tunnel and Session Management	34

6.8.5.1	Security handling during an L2TP Tunnel Establishment	34

6.8.5.2	Security handling during an L2TP Session Establishment	35

6.8.5.3	Potential alternative solutions	35

6.8.5.4	Conclusions	35

6.9	Solution#9: Enabling UPF to support multiple network slices	35

6.9.1	Description	35

6.9.2	Procedure	35

6.9.3	Impacts	36

6.10	Solution #10: UP Controlled UE IP Portion	37

6.10.1	Description	37

6.10.2	Impacts on services, entities and interfaces	37

6.10.3	Pros	37

6.10.4	Cons	38

6.11	Solution #11: UE IP address allocation by DHCP or AAA Server in 5GC	38

6.11.1	Description	38

6.11.1.1	General	38

6.11.1.2	Using a DHCP server	38

6.11.1.3	Using a AAA server	38

6.11.2	Impacts on services, entities and interfaces	39

6.11.3	Pros	39

6.11.4	Cons	39

6.12	Solution #12: UE IP address/prefix allocation by SMF self management	40

6.12.1	Description	40

6.12.1.1	General	40

6.12.1.2	SMF recover	40

6.12.1.3	Session release	40

6.12.1.4	SMF added/removed to/from SMF set	40

6.12.2	Impacts on services, entities and interfaces	40

6.12.3	Pros	41

6.12.4	Cons	41

6.13	Solution #13: Per Slice UP Resource Allocation and Usage Report	41

6.12.1	Description	41

6.13.2	Impacts on services, entities and interfaces	42

6.13.3	Pros	42

6.13.4	Cons	42

7	Evaluations and Conclusions	42

7.1	Evaluation and Conclusions of Solutions for Key Issue #1	42

7.1.1	Evaluation	42

7.1.2	Conclusions	43

7.2	Evaluation and Conclusions of Solutions for Key Issue #2	44

7.2.1	Evaluation	44

7.2.2	Conclusions	44

7.3	Evaluation and Conclusions of Solutions for Key Issue #3	44

7.3.1	Evaluation	44

7.3.2	Conclusions	45

7.4	Evaluation and conclusions of solutions for key issue #4	45

7.4.1	Evaluation	45

7.4.2	Conclusions	47

7.5	Evaluation and Conclusions of Solutions for Key Issue #5	47

7.5.1	Evaluation	47

7.5.2	Conclusions	48

Annex A (informative):	Change history	49

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

PFCP was first developed in Rel-14 to support CP and UP separation feature in EPC. From Rel-15 onwards PFCP has been reused for the interface between SMF and UPF (i.e. N4 interface) in 5GC. Open interoperability is one of the essential requirements of PFCP. Example scenarios requiring PFCP open interoperability can be:

-	Vertical market is one important use case of 5G, where UPF can be deployed locally on the customer side, while the SMF is centrally deployed on the operator side, therefore the interoperability between SMF and UPF is mandatory.

-	PDU session continuity when the UE moves between different SMF serving areas is supported by I-SMF and PFCP IEs are signalled over N16a, thus the interoperability regarding PFCP between the SMF and the I-SMF is mandatory.

This Technical Report aims to study the key issues which impact the interoperability of PFCP and potential solutions to resolve the issues.

# 1	Scope

The present document identifies the scenarios where the PFCP interoperability needs to be further enhanced for existing function and specifies the corresponding requirements, identifies the key issues which impact the PFCP interoperability, analyses potential solutions to address the key issues.

PFCP used in 4G (e.g. on Sxa, Sxb, Sxc interfaces) and 5G (e.g. on N4, N16a interfaces) are both within scope of the study.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.214: "Architecture enhancements for control and user plane separation of EPC nodes; Stage 2".

[3]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes; Stage 3".

[4]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[5]	3GPP TS 23.501: "System architecture for the 5G System (5GS); Stage 2".

[6]	IETF RFC 2661: Layer Two Tunneling Protocol "L2TP"

[7]	IETF RFC 3931: Layer Two Tunneling Protocol – Version 3 (L2TPv3)

[8]	IETF RFC 2865: Remote Authentication Dial In User Service (RADIUS)

[9]	IETF RFC 2868: RADIUS Attributes for Tunnel Protocol Support

[10]	3GPP TS 33.210: "Network Domain Security (NDS); IP network layer security"

[11]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[12]	IETF RFC 1334: PPP Authentication Protocols

[13]	IETF RFC 6357: Datagram Transport Layer Security Version 1.2

[14]	3GPP TS 29.061: "Interworking between the Public Land Mobile Network (PLMN) supporting packet based services and Packet Data Networks (PDN)"

[15]	3GPP TS 29.561: "5G System; Interworking between 5G Network and external Data Networks; Stage 3"

[16]	IETF RFC 7155: Diameter Network Access Server Application

[17]	IETF RFC 5426: The Transport Layer Security (TLS) Protocol Version 1.2

[18]	IETF RFC 8446: The Transport Layer Security (TLS) Protocol Version 1.3

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

CP function: A node with a Control Plane function (see 3GPP TS 23.214 [2] and 3GPP TS 23.502 [4]) supporting one or more PFCP entities. A Control Plane function, i.e. a Control Plane Node, is identified by the Node ID, that is set to either an FQDN or an IP address.

UP function: A node with a User Plane function (see 3GPP TS 23.214 [2] and 3GPP TS 23.502 [4]) supporting one or more PFCP entities. A User Plane function, i.e. a User Plane Node, is identified by the Node ID, that is set to either a FQDN or an IP address.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

CP	Control Plane

I-SMF	Intermediate SMF

PFCP	Packet Forwarding Control Protocol

SMF	Session Management Function

UP	User Plane

UPF	User Plane Function

# 4	Overall Requirements

Besides the scenarios addressed by 3GPP TS 23.214 [2] and 3GPP TS 29.244 [3], the study shall especially take following scenarios into account:

-	Scenario#1: multiple UP functions are controlled by one CP function, where the UP functions are from different vendors.

-	Scenario#2: one UP function is controlled by multiple CP functions, where the CP functions are from different vendors.

-	Scenario#3: multiple UP functions are controlled by a set of CP functions, where the UP functions are from different vendors and the CP functions are from same vendor.

-	Scenario#4: multiple UP functions are controlled by a set of CP functions, where the UP functions are shared by several network slices.

-	Scenario#5: the UP function(s) are deployed on the customer side while the CP function(s) are deployed on the operator side.

-	Scenario#6: CP function and UP function are implemented/developed as virtualized/container based NF.

The following requirements shall be considered during the study:

-	Requirement#1: the study shall try to avoid multiple options which may cause interoperability issues.

-	Requirement#2: the study shall identify the potential issues when the UP functions are deployed on the customer side and determine if specific extensions are required to address them.

-	Requirement#3: the study may consider protocol extensions for the widely used features that are not supported by PFCP, provided the corresponding stage 2 requirements are defined, or they do not require stage 2 requirements.

-	Requirement#4: For use cases where multiple CP functions not part of an NF set are considered, proposals shall take into account that each CP function may not support the same functionality.

-	Requirement#5: For use cases where multiple UP functions are considered, proposals shall take into account that each UP function may not support the same functionality.

# 5	Key Issues

## 5.1	General

Key issue definitions should follow common template, which would make it easier to reference them when specifying respective solutions:

-	Only one key issue shall be specified in a clause, identified with the clause title;

-	The first clause shall describe in detail why the use case or scenario is deemed problematic. This is essentially a justification for adding the key issue to the study;

-	The next clause shall formally define the key issue with a short and clear statement. This statement will be used to check how accurately the proposed solution matches the requirement in the key issue.

## 5.2	Key Issue #1: UPF support for multiple network slices

### 5.2.1	Description of the use case

In 5G system, multiple network slices with different functions and performance requirement may have different control plane functions, while they share the same UPF. Therefore, the UPF is expected to serve PDU sessions with different QoS and network separation requirements (in corresponding to different network slice), and the UPF will allocate different user plane resource (e.g. CPU/Memory/bandwidth/delay budget) to those PDU session.

### 5.2.2	Key issue definition

In current specifications, the S-NSSAI transmitted on the N4 interface is only used for performance measurement, when UPF are shared by several network slices (e.g. eMBB slice and IoT slice), UPF needs to know, which slice the PFCP session (PDU session) is pertaining to.

In spite of the fact that the Network Instance ID transmitted on N4 interface can provide the UPF with information about the network slice of the PDU session, the definition of Network Instance ID is not clear in the current specifications, and this approach also has some limitations, e.g. when an operator would like to use the same Network Instance ID for different network slices.

In order to solve the dilemma described above, this key issue aims at addressing following aspects for scenario 4:

1.	Study possible protocol enhancement over PFCP to enable a UPF to support multiple network slices.

2.	Study the potential missing functionalities to enable an UPF to support multiple network slices.

## 5.3	Key Issue #2: Header Enrichment for HTTPS

### 5.3.1	Description of the use case

Lots of websites use HTTPS schema to provide various services, including security sensitive service like personal banking. For those security sensitive services, local regulations may require recording which user has invoked the service. On the other hand, some value-added services (e.g. controlled by operators) may also require providing useful user information (e.g. user location) to the server, e.g. for making accurate service decision based on user information. In order to support such kind of requirements, header enrichment shall be supported for HTTPS, i.e. attaching the UE information (e.g. MSISDN, user location) to the service requests sent from the UE.

### 5.3.2	Key issue definition

This key issue will study the following aspects:

-	How does the SMF instruct the UPF to perform Header Enrichment for HTTPS?

-	How does the UPF detect the HTTPS packets and attach header fields and values to the detected HTTPS packets?

-	If security sensitive information is potentially transmitted to the server, how to ensure the transmission in a safe way?

## 5.4	Key Issue #3: Interoperability issues caused by multiple options co-existence

### 5.4.1	Description of the use case

As described in clause 3 in the WID of BEPoP (C4-203630), PFCP defines multiple technical options to implement certain features, which can result in between different implementations and potential interoperability issues. Solutions for effective interoperability mechanism between CP and UP function(s) need to be identified.

Additionally, new PFCP features have been defined at very new 3GPP release, resulting in a large and ever-growing list of UP function features in particular (e.g. 41 UP function features are defined in 3GPP Release 16). All these features are defined as optional to support in 3GPP TS 29.244 [3], which causes further potential deviations between CP function and UP function implementations from different vendors and potential risks of interoperability issues. It should be studied whether the support of certain features, that relate to the core part of the PFCP protocol (e.g. PDI optimization feature) could be recommended or mandated for NFs complying with 3GPP Release 17, in order to harmonize the set of core features supported by CP function and UP functions and help open interoperability between implementations from different vendors.

### 5.4.2	Key issue definition

This key issue shall study solutions to:

a.	End Marker: Identify whether End Marker should be allowed to be generated only by one side of CP or UP function, or recommend in which scenarios should be generated by CP function and in which scenarios should be generated by UP function.

b.	UE IP address allocation mode: Identify whether UE IP address should be allowed to be allocated only by one side of CP or UP function, or recommend in which scenarios should be allocated by CP function and in which scenarios should be allocated by UP function.

c.	DL data buffering: Identify whether downlink data should be allowed to be buffered only by one side of CP or UP function, or recommend in which scenarios should be buffered by CP function and in which scenarios should be buffered by UP function.

d.	Traffic redirection: Identify whether traffic redirection should be allowed to be enforced only by UP function for EPS to align with 5GS, or propose solutions to solve the interoperability issue caused by traffic redirection between EPS and 5GS;

e.	PDI optimization: Identify whether the support of certain features, that relate to the core of the PFCP protocol (e.g. PDI optimization feature) could be recommended or mandated for NFs complying with 3GPP Release 17.

## 5.5	Key Issue #4: Avoidance of conflict between UE IP address/prefix allocated by SMF(s)

### 5.5.1	Description of the use case

In early stage of 5G deployment, the operator may choose CP function to support the UE IP address/prefix allocation since CP function is required to support this feature as mandatory. But along with the network and business expansion, SSET and MPAS feature may be deployed. Then the operator may want to have the same UE IP address/prefix allocation policy with SSET and MPAS to ensure the solution continuity. In this case, a mechanism to support UE IP address/prefix allocation in SMF should be introduced.

Since a PFCP session may be taken over by another SMF of the same SMF set and different SMFs from different SMF sets could allocate UE IP address/prefix simultaneously, the UE IP address/prefix allocation and usage may be quite complex over N4 interface. So, the mechanism should avoid the UE IP address/prefix allocation conflict.

### 5.5.2	Key issue definition

This key issue shall study:

-	How to avoid conflict between UE IP address/prefix allocated by SMF(s) from one SMF set and/or from different SMF sets.

## 5.6	Key Issue #5: L2TP Tunnelling over Sgi/N6

### 5.6.1	Description of the use case

#### 5.6.1.1	General

L2TP (L2TPv2 described in RFC 2661/L2TPv3 described in RFC 3931) started off as an extension to the PPP model by allowing the L2 and PPP endpoints to reside on different devices interconnected by a packet-switched network. With L2TP, a user has an L2 connection to an access concentrator (e.g. modem bank, ADSL, DSLAM, etc), and the concentrator then tunnels individual PPP frames to the Network Access Server (NAS).

L2TP involves the following concepts:

-	L2TP Access Concentrator (LAC)

A node that acts as one side of an L2TP tunnel endpoint and is a peer to the L2TP Network Server (LNS). The LAC sits between an LNS and a remote system and forwards packets to and from each.

-	L2TP Network Server (LNS)

A node that acts as one side of an L2TP Tunnel endpoint and is a peer to the LAC. It is the logical termination point of a PPP session that is being tunneled from the remote system by the LAC.

-	Network Access Server (NAS)

A device providing local network access to users across a remote access network (e.g. PSTN). A NAS may also serve as a LAC, LNS or both.

Figure 5.6.1.1-1 describes a typical L2TP scenario to tunnel PPP frames between the remote system and an LNS located at a home LAN. The remote system initiates a PPP connection across the PSTN cloud to a LAC. The LAC then tunnels the PPP connection across the internet/frame relay/ATM cloud to an LNS, whereby access to a Home LAN is obtained.

Figure 5.6.1.1-1: Typical L2TP Scenario

The L2TP control connection (L2TP Tunnel) setup messages (SCCRQ: Start-Control-Connection-Request, SCCRP: Start-Control-Connection-Reply, SCCCN: Start-Control-Connection-Connected, ZLB Ack: Zero-Length Body Acknowledge) are described below. The control connection can be teardown with StopCCN and ZLB Ack messages.

Figure 5.6.1.1-2: L2TP Control Connection Setup

After control connection is started, the tunnel authentication takes place and the tunnel is established. The L2TP session establishment phase occurs with message exchanges (ICRQ: Incoming-Call-Request, ICRP: Incoming-Call-Reply, ICCN: Incoming-Call-Connected, ZLB Ack: Zero-Length Body Acknowledge) as described below.

The session connection can be teardown with CDN and ZLB Ack (Zero-Length Body Acknowledge) messages. Finally, the LNS triggers normal AAA or PPP authentication for the AAA server, e.g. RADIUS/Diameter server.

Figure 5.6.1.1-3: L2TP Session Establishment

With 5G, more focus is on the industrial applications of mobile networks, and the use of L2TP and its practice is expected to become even more common. The current ideology is that L2TP (mostly version 2/L2TPv2) is still prevalent in the existing operator networks (by reusing existing infrastructure from old dial-up connectivity), where it is possible for the UE to connect directly to the corporate network. In other words, this protocol (i.e. L2TPv2) is probably still used by many operators (in their LNS) to establish secured connection from the UE all the way into a third-party network, for example a corporate network. In addition, with mode corporate customers (like manufacturing, IoT, etc.), the need for L2TP (and similar) is believed to increase further.

A scenario of using L2TP is described below, used 5GC as an example. PFCP (described in 3GPP TS 29.244) is used on the N4 interface between SMF and UPF to configure the rules of packet detection, forwarding action, QoS enforcement, usage report and buffering for each PDU session.

Figure 5.6.1.1-4: L2TP Tunnels with 5GC

After CP/UP separation, it is difficult for the UP function to get necessary parameters (e.g. LNS Ips, username, password, etc.) to set up the L2TP tunnel to the third-party server. This information might be pre-configured on the CP, or the CP might receive it from an external server as described in RFC 2865.

#### 5.6.1.2	L2TP Implementation Models

In practical deployments, an UP Function with LAC functionality may be deployed in operator network domain, or in private enterprise network. And the L2TP tunnel parameters may be locally configured in the CP Function as per operator demand, or be locally configured in the UP Function located in private enterprise network as per customers demand, or be configured in external entities (e.g. Radius/AAA server).

According to various requirements and deployment situations, the L2TP implementation can be classified into different implementation models.

Model A:

In this model, the UP Function (e.g. UPF) is located in the operator network domain, and the L2TP tunnel parameters are configured in the CP Function (e.g. SMF). The L2TP tunnel parameters include necessary parameters for setting up L2TP tunnel towards the LNS (e.g. LNS address, tunnel password, etc.).

In order to trigger the UP Function to setup L2TP tunnel towards the LNS, the CP Function shall provide the L2TP tunnel parameters to the UP Function over Sxb/N4 interface.

Figure 5.6.1.2-1: L2TP Implementation Model A

Model B:

In this model, the L2TP tunnel parameters are configured in external entity (e.g. RADIUS/AAA server) which may be located in operator network domain or in private enterprise network.

The CP Function needs first retrieve the L2TP tunnel parameters from the external entity, before it can provide such L2TP tunnel parameters to the UP Function over Sxb/N4 interface. The external server address may be stored in the local configuration of the CP Function or be retrieved from the subscription data (e.g. like the dnAAA server address stored in the UE subscription data).

Figure 5.6.1.2-2: L2TP Implementation Model B

Model C:

In this model, the UP Function (e.g. UPF) is deployed in a private enterprise network, and the L2TP tunnel parameters are configured in the UP Function. The L2TP tunnel parameters include necessary parameters for setting up L2TP tunnel towards the LNS (e.g. LNS address, tunnel password, etc.).

As the UP Function knows the detailed L2TP tunnel parameters, it is no need for the CP Function to provide L2TP tunnel parameters to the UP Function over Sxb/N4. The UPF determines the trigger of L2TP tunnel establishment based on its local policy.

Figure 5.6.1.2-3: L2TP Implementation Model C

In the above model A/B, the CP Function needs to determine whether the L2TP tunnel is required for the PDN connection / PDU session. It can be done based on the attribute of APN/DNN/S-NSSAI, which is either stored in the local configuration of the CP Function, or retrieved from subscription data from e.g. HSS/UDM.

### 5.6.2	Key issue definition

This key issue will address:

-	The information used by the CP Function to determine whether an L2TP tunnel is required for a PDN connection / PDU session.

-	The information which is required to be communicated from the CP to the UP to setup a L2TP control connection (an L2TP tunnel) and L2TP sessions (one L2TP session per PDU session/PDN connection) from the UP function to an LNS and how this information should be communicated considering the current security/authentication aspects of L2TP.

-	The information which is required to be communicated from the UP to the CP to indicate the success/failure of a L2TP control connection (an L2TP tunnel)/L2TP session setup.

-	The enhancements (if any) which are required to the PFCP protocol to be able to send this information, considering security aspects.

-	How can the UP inform the CP in the 5GS about the failures related to an existing L2TP tunnel, and how L2TP sessions associated with the tunnel are affected.

-	How to handle the use cases when the LNS assigns the UE IP address.

-	Whether and how to handle the possibility of multiple CP functions (e.g. pertaining to a PGW/SMF SET) creating L2TP sessions (corresponding to PDU sessions/PDN connections) over a L2TP tunnel, i.e. the L2TP tunnel is shared by multiple PDU sessions/PDN connections but controlled by different CP functions.

# 6	Solutions

## 6.1	Solution#1: Header Enrichment for HTTPS

### 6.1.1	Description

The solution is to address the Key Issue #2: Header Enrichment for HTTPS in clause 5.3.

When HTTPS schema is used, HTTP messages are transmitted in encrypted SSL/TLS packets. Before sending HTTP message to the remote server, the client shall first perform SSL/TLS handshake procedure to set up SSL/TLS connection with the remote server. In this solution, the SSL/TLS protocol refers to TLS version 1.2 and onwards (see IETF RFC 5426 [17], IETF RFC 8846 [18]).

The initial SSL/TLS handshake messages are exchanged in clear text, which provides possibility to the UP Function to insert some customized information in the initial SSL/TLS handshake messages. A simple way is, when detecting an initial SSL/TLS handshake message (i.e. ClientHello message) from an UE, the UP Function can append additional SSL/TLS extension to the SSL/TLS handshake message to carry customized field names and values.

In order to support Header Enrichment for HTTPS, both the CP Function (e.g. SMF) and UP Function (e.g. UPF) shall be enhanced to support corresponding functionalities.

To instruct the UP Function to detect SSL/TLS packets and perform Header Enrichment for HTTPS, the CP Function shall:

-	Create UL PDR for detecting uplink SSL/TLS packets towards the remote server;

-	Create FAR and associate it to the UL PDR. In the FAR, provide header field names and values which are required to be inserted into SSL/TLS handshake messages. And set the Header Type of Header Enrichment IE to "SSL/TLS" (or set to "HTTPS").

Upon instruction from the SMF, the UP Function shall:

-	Install the UL PDR and FAR, and keep monitoring uplink IP packets from the UE and check whether it is SSL/TLS packets;

-	Once SSL/TLS packets from the UE is detected, further inspect whether the SSL/TLS packets carries initial SSL/TLS handshake message from the UE (i.e. ClientHello message);

-	Once the initial SSL/TLS handshake message from the UE is detected, insert additional SSL/TLS extension and carry the required field names and the values in the additional SSL/TLS extension, and finally forward the altered SSL/TLS packets onwards.

The encoding of required field names and values in the additional SSL/TLS extension is implementation specific, e.g. based on the service agreement between an operator and its customers. Operators should avoid inclusion of security sensitive information in the SSL/TLS extension as much as possible. If security sensitive information is regarded as necessary, it shall be transmitted in a secure way.

NOTE:	How the application server retrieves the enriched content at TLS layer and sends to upper layers is out of scope for 3GPP.

SA3 has provided the following feedback on this solution, as per the LS S3-214505:

-	Initial TLS packets are not a suitable way to transport the information. Since a man-in-the-middle inserts information to be consumed by the server, without consent from the client, it could be considered a privacy breach. Also, TLS will often be terminated in a frontend, which may not the intended receiver of the information.

-	If the endpoints is outside the 3GPP network domain, IMSI/SUPI, and IMEI are internal identifiers that are not supposed to be sent outside the 3GPP network domain. (NOTE: This is independent on whether appropriate security is setup between the PSA UPF and AS).

-	Depending on regulations, sending other parameters related to subscribers and users could require user consent: UE IP address, User location, and MSISDN etc.

Taking SA3 feedback into account, the usage of this solution shall be limited, i.e. only be used in the following cases: (a) the application server is inside the operator network, or, (b) the application server is inside an operator trusted domain (e.g. in an enterprise network which has secured tunnel with the operator network). In addition, sending the IMSI/SUPI to an AS is limited only when the AS is inside the operator network. When deploying this solution, operators shall guarantee that user information transmission to the application server shall comply with the regional regulations.

### 6.1.2	Impacts on services, entities and interfaces

CP Function:

-	Indicate to UP Function that detection of SSL/TLS handshake message is required and indicated header field names and values are to be inserted to SSL/TLS handshake message;

UP Function:

-	Detect SSL/TLS handshake message from UE;

-	Insert the required header field names and values to the detected SSL/TLS handshake messages from UE.

### 6.1.3	Pros

This solution provides a feasible way for operators to support header enrichment for HTTPS schema.

### 6.1.4	Cons

To ensure this solution works well, service agreements shall be made between an operator and its customers, e.g. to determine the encoding of header fields and the values in TLS extension.

## 6.2	Solution#2: UE IP address/prefix allocation by SMF in SMF Set

### 6.2.1	Description

#### 6.2.1.1	General

The description below is to address the solution for Key Issue# 4.

The SMF deployed in SMF set supports the UE IP address/prefix allocation by SMF. Each SMF in the SMF set has an individual UE IP address/prefix list and make a record for each UE IP address/prefix to indicate whether it is available or not. The individual UE IP address/prefix list for each SMF in the SMF set should not be the same to avoid the UE IP address/prefix allocation conflict. For the UE IP addresses/prefixes partitioning between the SMFs of an SMF set, it will depends on operator's policy.

The SMF allocate the available one from the UE IP address/prefix list by checking the usage indication. When a UE IP address/prefix have been allocated, the SMF marked this UE IP address/prefix as unavailable.

Each SMF also records all the UE IP address/prefix lists controlled by other SMFs in SMF set and dynamically make a record for each IP address/prefix to indicate whether the session using this IP address/prefix is controlled by this SMF or not. This enables SMF keep tracks of whether it is currently serving PFCP sessions with an UE IP address that is assigned to another SMF in the set, such as to be able to signal to that other SMF that the UE IP address is released when the SMF releases the PFCP session later. Then, each SMF in SMF set records the whole UE IP address/prefix table which includes the individual UE IP resource part and the recorded UE IP resource part.

#### 6.2.1.2	SMF recover

The SMF broadcasts the recovery message towards all other SMFs in SMF set after recover from a failure. Upon receiving the recovery message, other SMFs share the maintained IP address/prefix list with usage indication records. The recovered SMF can make the record for its individual UE IP address/prefix list to indicate whether it is available or not according to these shared information. The other SMFs may redirect some of the PFCP sessions back to the recovered SMF which originally created these PFCP sessions. The session redirection can be triggered by other mechanism like SMF set load balance.

#### 6.2.1.3	Session release

The SMF broadcasts the available UE IP address(es)/prefix(s) message towards all the SMFs in SMF set in case these UE IP address(es)/prefix(s) is released and they are not belonging to its individual UE IP address/prefix list. The SMF which originally allocate these UE IP address(es)/prefix(s) will change the usage indicator to available again. If the SMF is failed, other SMFs in the SMF set cannot reuse the UE IP addresses that were still available in the failed SMF.

#### 6.2.1.4	SMF added/removed to/from SMF set

##### 6.2.1.4.1	General

If SMF(s) added/removed to/from the SMF set, the UE IP address/prefix resource will be re-assigned depends on operator's policy. Each SMF will get the new UE IP address/prefix table which is different from the old one.

The UE IP address/prefix resource re-assignment can be done by local configuration or directly controlled by external entity.

##### 6.2.1.4.2	Solution based on local configuration

In local configuration case, some UE IP address(es)/prefix(es) originally controlled by one SMF might be partitioned to another SMF within the same SMF set. The SMFs in SMF set should coordinate with each other in order to synchronize the UE IP address/prefix usage status. Two possible scenarios are list as below.

-	UE IP address(es)/prefix(es) re-assigned to another target SMF but originally controlled by a removed SMF.

-	UE IP address(es)/prefix(es) re-assigned to another target SMF but originally controlled by a SMF in SMF set.

NOTE:	The mechanism of this solution is to synchronize the UE IP address/prefix usage status between SMFs in SMF set after UE IP addresses/prefixes partitioning. There is no additional requirement on all the ongoing PFCP sessions. No impact over N4 interface as well as all the existing PFCP sessions and no requirement for other NFs interacting with the SMF/SMF set.

For the UE IP address(es)/prefix(es) originally controlled by one SMF which is removed from the SMF set but re-assigned to another target SMF, the target SMF shall broadcasts these UE IP address(es)/prefix(es) towards all the other SMFs in SMF set. Other SMFs share the maintained IP address/prefix list with usage indication records. The target SMF can make the record for these individual UE IP address(es)/prefix(es) to indicate whether it is available or not according to these shared information. Optionally, the target SMF requires session redirection according to local policy which can be implementation specific.

For the UE IP address(es)/prefix(es) originally controlled by one SMF but re-assigned to another target SMF, the original SMF shall send all these UE IP address(es)/prefix(es) with the recorded usage indicator value to the target SMF and will not choose any available one from these IP resource when new IP allocation requirement raised. The target SMF will update the usage indicator value(s) accordingly and send a confirmation message back after receiving the information from original SMF. After receiving the confirmation message from target SMF, the original SMF shall change the UE IP address/prefix usage indicator value to available if the corresponding session is not controlled by the original SMF.

##### 6.2.1.4.3	Solution based on directly controlled by external entity

In the external entity controlled case, e.g. an OAM is implemented to re-assignment the UE IP address/prefix table for the SMF in the SMF set:

When a new SMF is to be added to the SMF set, the OAM could require the current SMFs in the SMF set to report their available UE IP address/prefix. The current SMFs in the SMF set should report their available UE IP address/prefix to the OAM (NOTE 1) and isolate or remove these UE IP address/prefix from their IP address pool. Upon receiving these UE IP address/prefix, OAM selects certain UE IP address/prefix based on its configuration or operator's policy, and assigns them to the new SMF. The OAM should subsequently assign the remaining UE IP address/prefix back to the current SMFs in the SMF set (NOTE 2). Upon receiving, the current SMFs in the SMF set should update their IP address pool respectively.

NOTE 1:	In order to prevent SMF having no available IP address to allocate for new session after it reporting its available UE IP address/prefix to the OAM (before OAM assigning the remaining UE IP address/prefix back to this SMF), SMF may decide to report only part of its available UE IP address/prefix to the OAM according to its configuration or the policy issued by the OAM.

NOTE 2:	In order to avoid disrupting the SMF's' IP address pool planning in the SMF set, OAM should try to assign the remaining available UE IP address/prefix reported by certain SMF back to this SMF.

When a current SMF is to be removed from the SMF set, the OAM should require this SMF to report all of its UE IP address/prefix. In particular, if the UE IP addresses /prefixes of this SMF were assigned by the OAM previously and the OAM holds the relevant records, the OAM can also directly read the records and confirm the available UE IP address/prefix. The OAM could keep these UE IP address/prefix on its own for the time being or assign these UE IP address/prefix to certain SMFs in the SMF set, based on its configurations or operator's policy. If the OAM assigns these UE IP address/prefix to certain SMFs in the SMF set, the relevant SMFs in the SMF set should update their IP address pool respectively.

### 6.2.2	Signalling flow

#### 6.2.2.1	General

This clause is only used to illustrate the general solution idea. There is no restriction on the detailed interworking signalling. (e.g. message name, message formats, Information Elements)

#### 6.2.2.2	SMF recover

After the SMF recovered from a failure, The SMF broadcasts the recovery message towards all the other SMFs in SMF set. Upon receiving the recovery message, other SMFs share the maintained IP address/prefix list with usage indication records. Figure 6.2.2.2-1 illustrates the message between SMFs in SMF set.

Figure 6.2.2.2-1: Procedures for SMFs in SMF set - SMF recover

#### 6.2.2.3	Session release

If the released UE IP address(es)/prefix(es) does not belong to the SMF's individual UE IP address/prefix list, then the SMF will broadcasts the available UE IP address(es)/prefix(s) message towards all the other SMFs in SMF set. Figure 6.2.2.3-1 illustrates the signaling flow between SMFs in SMF set.

Figure 6.2.2.3-1: Procedures for SMFs in SMF set - session release

#### 6.2.2.4	SMF added/removed to/from SMF set

As described in clause 6.2.1.4.2, two possible scenarios required the signaling between SMFs in SMF set.

Figure 6.2.2.4-1 illustrates the signaling flow between SMFs in SMF set when UE IP address(es)/prefix(es) re-assigned to another target SMF but originally controlled by a removed SMF.

Figure 6.2.2.4-1: Procedures for SMFs in SMF set –UE IP address(es)/prefix(es) re-assigned to another target SMF but originally controlled by a removed SMF

Figure 6.2.2.4-2 illustrates the signaling flow between SMFs in SMF set when UE IP address(es)/prefix(es) re-assigned to another target SMF but originally controlled by a SMF in SMF set.

Figure 6.2.2.4-2: Procedures for SMFs in SMF set –UE IP address(es)/prefix(es) re-assigned to another target SMF but originally controlled by a SMF in SMF set

### 6.2.3	Impacts on services, entities and interfaces

The solution requires some inter SMF signalling within SMF set. This interface is avoided to be further specified in SA2 and leaved as implementation specific. So, it requires further checking and review by 3GPP CT4.

### 6.2.4	Pros

-	The proposed solution enables the UE IP address/prefix allocation by SMF if SSET and MPAS feature deployed.

-	In early stage of 5G deployment, the operator may choose CP function to support the UE IP address/prefix allocation since CP function is required to support this feature as mandatory. But along with the network and business expansion, SSET and MPAS feature may be deployed. Then the operator may want to have the same UE IP address/prefix allocation policy with SSET and MPAS to ensure the solution continuity. This solution ensures the customer can chooses the same UE IP address/prefix allocation policy when SSET and MPAS feature deployed.

-	From the UE IP planning point of view, the partitioning of UE IP addresses/prefixes for each SMF in SMF set will be easy when the SMF is added/removed from the SMF set. When new SMF added/removed to/from SMF set, the UE IP resource in SMF set can be simply divided to each SMF as individual UE IP addresses/prefixes list no mattter what is the real UE IP resource usage status. And the available UE IP addresses/prefix for each SMF can be contiguous.

### 6.2.5	Cons

-	This solution requires lots of inter SMF signalling within SMF set which may implications on Nxx interface(new reference point between SMFs with SMF set). Whether this solution is fits into the BEPoP SID scope require further checking and review by 3GPP CT4.

-	This solution requires standardization/specification and implementation of a new interface, messages and procedures between SMFs of an SMF set to advertise the list of IP addresses assigned to  SMFs.

-	This solution adds complexity to SMF implementation for the allocation of IP addresses, re-allocation of IP addresses when an SMF is added or removed from the set, and advertisement of release of IP addresses owned by a different SMF of the SMF set.

-	It requires support of an OAM or external entity to decide the IP addresses partitioning between the SMFs of an SMF set.

-	The solution statically allocates IP addresses to SMFs of the SMF set, which might result in the SMF set failing to establish new sessions when IP addresses are still available (e.g. an SMF has failed w/o restart, the IP addresses of that SMF are not re-allocated to other SMFs).

-	Any failure to exchange these new messages between SMFs (e.g. transient communication failure between 2 SMFs, SMF overload) can result in SMFs getting out of synchronization with regard to IP addresses usage and result e.g. in SMF trying to allocate an IP address that is already assigned to an existing PFCP session.

-	This solution requires each SMF in SMF set records the whole UE IP address/prefix table which includes the individual UE IP resource part, the recorded UE IP resource part, and the usage indication of every UE IP address/prefix.

## 6.3	Solution #3: Enabling SMF to map network slice information into Network Instance

### 6.3.1	Description

S-NSSAI and Network Instance already can be transferred between SMF and UPF, e.g. in PDR and FAR. However, UPF can only use S-NSSAI for performance measurement so far. The UPF is not expected currently to use S-NSSAI to identify the network slice when allocating resources to the PDU session. The UPF cannot use the S-NSSAI IE to isolate resources for multiple network slices.

On the other hand, the Network Instance is determined by the SMF and the SMF may determine the Network Instance for N3, N6, N9 and N19 interfaces by taking into account S-NSSAI as specified in clause 5.6.12 of 3GPP TS 23.501 [5]. Network Instance is encoded as an OctetString as specified in clause 8.2.4 of 3GPP TS 29.244 [3]. So a simple way is, the SMF maps the network slice information, DNN, IP domain, etc. into the Network Instance when needed and sends Network Instance to UPF via PFCP Session Establishment/Modification request message. Upon receiving, the UPF use the information in the Network Instance for traffic detection and routing as the same as required by network slice, so as to achieve multiple network slice sharing. This mechanism requires SMF to map between Network Instance and the network slice information but it requires no extra stage 2 modifications and introduces no new call flows or IEs, which means it can be applied under current 3GPP specifications.

Based on the above statement, it is beneficial to enable SMF to map the network slice information into Network Instance and send it to UPF to realize multiple network slice sharing.

### 6.3.2	Signalling flow

The procedure for SMF to map network slice information into Network Instance is shown in Figure 6.106.86.86.8.2-1.

Figure 6.3.2-1: Procedures for SMF to map network slice information into Network Instance

1.	Pre-configure the mapping between the network slice information and the Network Instance on SMF.

2.	SMF sends the Network Instance from which the network slice can be derived to UPF via PFCP Session Establishment/Modification Request message.

3.	Upon receiving above message, UPF use the information in the Network Instance for traffic detection and routing as the same as required by network slice, so as to achieve multiple network slice sharing.

4.	UPF send PFCP Session Establishment/Modification Response message to SMF.

### 6.3.3	Impacts on services, entities and interfaces

SMF:

-	SMF is pre-configured with the mapping between the network slice information and the Network Instance.

UPF:

-	None.

## 6.4	Solution #4: Enabling UP function to buffer downlink data as the preferred option

### 6.4.1	Description

Solution #4 addresses the Key Issue #3 (Interoperability issues caused by multiple options co-existence), sub-key issue #3c (DL data buffering).

It is beneficial to enable UP function to buffer downlink data:

-	As specified in clause 5.8.3.1 of 3GPP TS 23.501 [5] and in clause 5.9.1 of 3GPP TS 23.214 [2], support for buffering in the UP function is mandatory and optional in the CP function.

-	It is the UP function sending the buffered downlink data to the UE. If the downlink data is enabled to be buffered in the CP function, the UP function shall firstly forward these downlink data to the CP function when it needs to be buffered and the CP function shall then forward these buffered data back to the UP function when the UP connection is activated, which leads to extra data transfers and delays.

Based on the above statement, CP function should preferably buffer the downlink data in the UP function when needed. The CP function may buffer itself downlink data when specifically requested e.g. in following scenarios:

-	Control Plane CioT 5GS Optimisation is used where the DL data will travel over NAS through the CP function.

-	For Ues using power saving methods.

### 6.4.2	Impacts on services, entities and interfaces

CP function:

-	CP function should preferably buffer the downlink data in the UP function when needed. The CP function may buffer itself downlink data when specifically requested e.g. in following scenarios:

-	Control Plane CioT 5GS Optimisation is used where the DL data will travel over NAS through the CP function.

-	For Ues using power saving methods.

UP function:

-	None.

## 6.5	Solution #5: Enabling UP function to construct End Marker as the preferred option

### 6.5.1	Description

Solution #5 addresses the Key Issue #3 (Interoperability issues caused by multiple options co-existence), sub-key issue #3e (End Marker).

It is beneficial to enable UP function to construct End Marker:

-	As specified in clause 5.3.2 of 3GPP TS 29.244 [3], if the UP function indicated support of End Marker packets constructed in the UP function, the CP function shall request the UP function to construct and send End Marker packets by sending a Session Modification Request including FAR(s) with the new downstream F-TEID and with the SNDEM (Send End Marker Packets) flag set.

-	As specified in clause 5.8 of 3GPP TS 23.214 [2] and clause 5.8.2.9 of 3GPP TS 23.501 [5]. It is the UP function sending the End Marker to the source RAN and source UP function via S1/N3 and S5/8/N9 interface respectively. If the End Marker is constructed by CP function, the CP function shall construct the End Marker packet(s) and send it to the UP function when the path switch is finished, which leads to extra Sx/N4 signalling interactions and resource consumption in the UP function and the CP function.

Based on the above statement, it is proposed to require Rel-17 onwards UPFs to support constructing End Marker packets and to recommending constructing End Marker in the UPF as the preferred option.

### 6.5.2	Impacts on services, entities and interfaces

CP function:

-	None.

UP function:

-	Support to construct End Marker in the UP function as mandatory.

## 6.6	Solution #6: Enforce traffic redirection in the UP function as the only option

### 6.6.1	Description

Solution #6 addresses the Key Issue #3 (Interoperability issues caused by multiple options co-existence), sub-key issue #3d (Traffic redirection).

It is beneficial to enable UP function to enforce traffic redirection:

-	As specified in clause 5.4.7 of 3GPP TS 29.244 [3], for 5GC, the traffic redirection may be enforced in the UP function only. And if the traffic that the UP function can support may be subject to traffic redirection, traffic redirection enforcement in the UP function shall be supported by the UP function.

-	As specified in clause 5.4.7 of 3GPP TS 29.244 [3] and in clause 5.11.3 of 3GPP TS 23.214 [2], to enforce the traffic redirection in the CP function, the CP function shall instruct the UP function to forward the applicable user traffic to the CP function, which leads to extra data transfers and delays.

-	If UP function is deployed on the customer side, it may be inappropriate to forward user traffic data from the UP function to the CP function due to the customers' privacy issue.

Based on the above statement, it is proposed to remove from the 3GPP TS 29.244 [3], from Rel-17 onwards, the option to enforce traffic redirection in the CP function. In other words, traffic redirection would always be enforced in the UP function.

### 6.6.2	Impacts on services, entities and interfaces

CP function:

-	CP function shall determine to enforce traffic redirection in the UP function when needed.

UP function:

-	Support to enforce traffic redirection in the UP function as mandatory.

## 6.7	Solution#7: Recommending the support of features relating to the core part of the PFCP protocol

### 6.7.1	Description

This solution addresses the Key Issue #3 (Interoperability issues caused by multiple options co-existence) and proposes to recommend the support of features that relate to the core part of the PFCP protocol, by NFs complying with 3GPP Release 17.

NOTE:	Features relating to the "core part of the PFCP protocol" refers to features that relate to the basic implementation of the PFCP packet processing model and the basic interactions between the CP and UP functions to establish PFCP sessions.

It is proposed specifically to recommend the support of the PDI Optimization feature (KI#3e) by 3GPP Rel-17 onwards compliant CP and UP functions:

-	this feature has been specified in 3GPP Rel-15 (see C4-182311, from Huawei, Deutsche Telekom, Cisco, Nokia, Nokia Shanghai Bell, Ericsson);

-	the feature introduced the concept of Traffic EndPoint, corresponding to a GTP-U endpoint or an Sgi/N6 endpoint, to ease the identification of the PFCP session an incoming packet corresponds to (according to the packet forwarding model, see Figure 5.2.1-1 of 3GPP TS 29.244 [3]) and to optimize the Sx and N4 signalling by enabling to create once a traffic endpoint and to refer to it in all the PFCP session's rules associated to a GTP-U or Sgi / N6 endpoint;

-	not supporting this feature requires to send the same Packet Detection Information (PDI) for every rule associated to a GTP-U or Sgi / N6 endpoint and causes a logical discrepancy between the processing flow and the actual UP function implementation, resulting in extra complexity.

### 6.7.2	Impacts on services, entities and interfaces

CP Function:

-	support of the PDI Optimization feature (i.e. concept of Traffic EndPoints), if not already supported by the CP function;

UP Function:

-	support of the PDI Optimization feature (i.e. concept of Traffic EndPoints), if not already supported by the UP function;

### 6.7.3	Pros

-	all vendors' CP function and UP function's implementations complying with 3GPP Rel-17 support the same behaviour and features regarding the core part of the PFCP protocol;

-	Sx and N4 signalling is optimized by provisioning Traffic EndPoints only once in the UP function and referring to them in all rules;

### 6.7.4	Cons

-	none.

## 6.8	Solution #8: L2TP Tunnel and Session Setup by UP Function

### 6.8.1	Description

The solution is to address the Key Issue #5: L2TP Tunnelling over Sgi/N6 as defined in clause 5.6.

An L2TP Session is required between the LAC and LNS when an PDN connection or PDU session is established between the UE and the LNS in the Data Network. An L2TP Tunnel exists between a LAC-LNS pair controlling the connection and consists of one or more L2TP Sessions. One or more L2TP tunnels can be established between a LAC-LNS pair.

The UP function can setup an L2TP session towards the L2TP server for a PDN connection / PDU session, upon the request from the CP Function over Sx/N4 interface during PDN connection / PDU session Establishment. The UP function evaluates which L2TP Tunnel is to be selected, and setups an L2TP Tunnel if the wanted L2TP Tunnel does not exist already, before setting up the L2TP session.

One L2TP tunnel between the UP Function and the L2TP server can be shared by multiple PDN connection / PDU session using the same parameter (e.g. Network Instance, LNS address, Tunnel-Assignment-Id). A single L2TP tunnel can be shared by multiple PDN connections / PDU sessions created by same CP Function or by different CP Functions.

To request the UP Function to setup an L2TP session towards the L2TP server, the CP Function shall:

-	In the PFCP Session Establishment Request, request the UP Function to setup L2TP tunnel and session. Parameters needed to setup the L2TP tunnel (e.g. LNS address, Tunnel-Password, etc.), parameters needed to setup the L2TP session (e.g. Calling Number, Called Number) and User Authentication parameters (e.g. PAP/CHAP, username, password), shall be provided by the CP Function in the PFCP Session Establishment Request.

NOTE:	This solution is based on L2TP version 2, which is widely deployed in the network.

Upon receiving the request for L2TP session setup from the CP Function, the UP Function shall:

-	Select an L2TP Tunnel or create an L2TP tunnel if no suitable tunnel already exists; and

-	Setup an L2TP session.

The UP Function sends a PFCP Setup Session Establishment Response to the CP, carrying the information of Created L2TP Tunnel and L2TP session.

In order to support L2TP tunnel establishment by the UP Function, an L2TP capable UP Function shall be selected. The CP Function may use one of the following methods to select an L2TP capable UP Function: CP Function selects the target UP Function based on the local configuration, CP Function selects the target UP Function based on the UP Function features during PFCP Association Establishment procedure, CP Function discovers the target UP Function from the NRF.

### 6.8.2	Protocol impact over N4/Sxb

#### 6.8.2.1	L2TP Support Negotiation

The UP function shall indicate its support of L2TP in the UP function feature IE during PFCP Association Setup procedure. The UP Function is assumed to have the connectivity towards all LNSs via the given network instance. Figure 6.8.2.1-1 illustrates the L2TP support negotiation during PFCP Association Setup procedure.

Figure 6.8.2.1-1: L2TP Support Negotiation Procedure

The UP Function indicates L2TP support in UE Function Feature IE in PFCP Association Setup Response if the PFCP Association is initiated by the CP function.

1.	The CP Function sends PFCP Association Setup Request to the UP Function.

2.	The UP Function sends PFCP Association Setup Response to the CP Function. Within the message, the UP Function Features IE is included to indicate the L2TP support, and if so, the UP function may include the LAC address(es) configured per Network Instance if the Network Instance, e.g. corresponding to a DNN/APN has been configured with multiple LACs.

NOTE:	The CP function needs to select a right UP function (with a LAC address configured matching the one received from AAA server, e.g. RADIUS/Diameter server if it is received) when it requests UP function to establish a L2TP tunnel and L2TP session. See also clause 3.3 of IETF RFC 2868 [9], clause 4.5.4 of IETF RFC 7155 [16].

The UP Function indicates L2TP support in PFCP Association Setup Request if the PFCP Association is initiated by the UP function.

1.	The UP Function sends PFCP Association Setup Request to the CP Function. Within the message, the UP Function Features IE is included to indicate the L2TP support, and if so, the UP function may include the LAC address(es) configured per Network Instance if the Network Instance, e.g. corresponding to a DNN/APN has been configured with multiple LACs.

2.	The CP Function sends PFCP Association Setup Response to the UP Function.

#### 6.8.2.2	L2TP Tunnel/Session Setup

When the CP Function receives PDN connection / PDU session establishment request, if an L2TP session is required to support the PDN connection/PDU session, the CP Function requests the UP Function to setup an L2TP session towards the L2TP server. Figure 6.8.2.2-1 illustrates the L2TP session setup/tunnel during PFCP Session Establishment procedure.

Figure 6.8.2.2-1: L2TP Session/Tunnel Setup Procedure

1.	The CP Function receives PDN Connection Establishment Request / PDU Session Establishment Request from the UE.

The CP Function may determine that an L2TP tunnel is required for the PDN connection / PDU session based on the attributes of APN/DNN/S-NSSAI, which may be stored in the local configuration of the CP Function, or retrieved from UE subscription data. Or the CP Function may determine the need of L2TP tunnel based on the L2TP tunnel parameters retrieved from RADIUS/Diameter/AAA server during authentication procedure. In the latter case, the CP Function gets the RADIUS/Diameter/AAA server address from its local configuration or from the subscription data (e.g. like the dnAAA address for secondary authentication stored in the UE subscription data).

2.	The CP Function sends PFCP Session Establishment Request message to the UP Function.

If the L2TP tunnel parameters are available at the CP Function, either from the local configuration of the CP Function or retrieved from the RADIUS/AAA server, the CP Function provides the information listed in bullet a/b/c/d as demand to the UP Function.

If the L2TP tunnel parameters are available at the UP Function, the CP Function may only provide the information listed in bullet b/c/d as demand to the UP Function.

a) One or more L2TP Tunnel Information IE is included in the message to provide parameters to control a L2TP tunnel establishment if there is need to establish a new L2TP tunnel. This IE includes LNS Ipv4/Ipv6 address, LNS hostname, tunnel password, LAC Ipv4/Ipv6 address, LAC hostname, Tunnel Assignment ID, as specified in IETF RFC 2868 [9] and IETF RFC 7155 [16].

-	Multiple L2TP Tunnel Information IEs can be sent to provide the UP Function with a list of possible L2TP tunnels that can be used to establish the L2TP session. This can be used to ensure that the session is established even if a specific LNS is temporarily unavailable, or to load balance sessions over many different LNS's.

-	If more than one L2TP Tunnel Information IE is provided, the L2TP Tunnel Preference IE should be also be provided, which instructs the UP Function on how to prioritize among the L2TP Tunnel Information IEs.

b) The L2TP Session Information IE which includes information to control the L2TP session setup. The L2TP Session Information includes the Called Number (optional), Calling Number (optional), Private Group ID (optional) and the MRU (Maximum Receive Unit in LCP/PPP to be set as the value of the MTU received from UE or be configured in the CP function), and the information for Configuration Options in IPCP/PPP, e.g. indications to instruct that the UP Function shall request the LNS to allocate an IP address for the PDN session/PDN connection, to provide DNS server address(es) and/or NBNS server address(es) for the session.

c) The L2TP User Authentication IE (optional) may also be included in L2TP Session Information IE, consisting of parameters such as authentication type, Username and password, Challenge and Challenge Response which is either sent by the UE through PCO in step 1 or as part of configured information on the CP.

d) If the UE IP address should be allocated by the UP Function (or the LNS), the CP Function indicates this as normal in the UE IP Address IEs in the PDR.

If the L2TP tunnel parameters are configured in the UP Function, the UP Function determines the need of L2TP tunnel based on its local configuration. How to make the decision of L2TP tunnel is implementation specific.

3.	The UP Function acting as LAC is responsible to setup the L2TP tunnels and sessions towards the LNS on receiving the L2TP session request from the CP.

On receiving the PFCP Session Establishment Request from the CP Function, the UP function determines whether to use an already established L2TP tunnel or establish a new tunnel. The UP function may have already established L2TP tunnel(s) based on local configured L2TP Tunnel Information.

If an existing L2TP Tunnel is used, the UP Function shall only setup new L2TP session within the existing L2TP tunnel. Otherwise, the UP Function shall perform both L2TP tunnel and L2TP session setup.

4.	The UP Function sends PFCP Session Establishment Response to the CP Function, carrying the Created L2TP Session IE containing information about the L2TP session to be used for statistics or diagnostics. The response message may also carry an LT2P Cause IE (optional), which can be used to provide additional L2TP specific failure information to CP Function in case the session establishment failed.

The PFCP Session Establishment Response message may also include the DNS Server Address, containing the DNS server information. It is also possible to send multiple IEs to provide multiple DNS server addresses, in which case, the order the IEs are put in the message indicates the priority, i.e. the first IE contains the primary DNS server address and the second IE contains the secondary DNS server address. In similar lines, NBNS Server Address may also be included in the response message from UP Function to CP Function.

If the UE IP address was allocated by the UP Function (or the LNS), the UP Function provides the allocated IP address to the CP Function as normal in the UE IP Address IEs in the Created PDR IE and/or Created Traffic Endpoint IE.

5.	The CP Function sends PDN Connection Establishment Response / PDU Session Establishment Response to UE, containing the DNS/NBNS Server Address info via PCO.

#### 6.8.2.3	L2TP Session/Tunnel Release

Since the L2TP session is tightly connected to the PFCP session, when the PFCP session is terminated, the L2TP session is automatically terminated by the UP Function (LAC), without any need for additional indication or signalling. In addition, it is also feasible for the LNS to initiate L2TP tunnel and session release. When the last L2TP session in a L2TP tunnel is terminated, the UP Function, may, terminate the L2TP tunnel as well. However, it is also possible to retain the L2TP tunnel, though all L2TP sessions are terminated.

Figure 6.8.2.3-1: L2TP Session Release Procedure

For a general scenario, where the CP Function receives a PDN/PDU Session Release Request from the UE, the PFCP session deletion procedure, described in clause 6.3.4 in 3GPP TS 29.244 [3] is applicable. In this scenario and in Figure 6.8.2.3-1, the UP Function (LAC) releases the L2TP sessions, based on PFCP session termination, and can be summarized as:

1.	The UE sends a PDN Connection Release Request/PDU Session Release Request to the CP Function.

2.	The CP Function initiates the PFCP session deletion procedure and sends a PFCP Session Deletion Request to the UP Function (LAC) to delete the existing PFCP session.

3.	The UP Function performs the L2TP session release, and tunnel release (when applicable).

4.	The UP Function sends a PFCP Session Deletion Response to the CP Function with a cause success if the corresponding PFCP session context is found.

5.	On receiving the PFCP Session Deletion Response, the CP Functions sends a PDN Session Release Response/PDU Session Release Response to the UE.

When the LNS terminates a L2TP session or a L2TP tunnel, the following applies:

-	The LNS terminates the L2TP session, in which case the UP Function will need to report to the CP Function that the PFCP session is terminated. This can be done by the UP Function sending a PFCP Session Report Request message to the CP, as described in clause 5.18.2 in 3GPP TS 29.244 [3], with an additional LT2P Cause IE indicating the reason for the termination.

-	The LNS terminates the L2TP tunnel, in which case the UP will need to report to the CP that all the PFCP sessions on the tunnel are terminated. The PFCP Session Report Request message, described in Scenario 1 above can be sent by the UP Function to the CP Function, with an L2TP Cause IE indicating the reason for the termination. That is, each session is terminated individually, instead of having a node-level message, this avoids the need for the CP Function to keep track of the Tunnel IDs per session.

-	The LAC should terminate all L2TP sessions towards an LNS when the connectivity to an LNS is lost. This implies that the UP Function terminates each PFCP session as described in Scenario 2 above.

NOTE 1:	The L2TP session cannot be modified as specified in IETF RFC 2661 [6].

NOTE 2:	L2TP Cause IE provides some causes related to L2TP, which can be a new IE or use existing Cause IE.

### 6.8.3	Impacts on services, entities and interfaces

The Sx/N4 interface needs to be enhanced to convey information required to setup the L2TP sessions/tunnels to the UP Function. The CP Function and UP Function should be able to support the following new IEs for the new L2TP functionality:

The PFCP Session Establishment Request message from CP Function to UP Function consists of

-	L2TP Tunnel Information IE (Conditional: if received from AAA server, e.g. Radius/Diameter server or configured in the CP function):

This IE contains information required to setup an LT2P tunnel to an LNS. It contains LNS IP Address, LNS Host Name, Tunnel Password, Tunnel Preference, Tunnel Assignment ID, LAC IP Address, LAC Host Name.

-	L2TP Session Information IE (Mandatory)

This IE contains the information to be used for the L2TP session creation. It contains MRU, Called Number, Calling Number, Private Group ID and Request for IP Indication, Request for DNS Indication, Request for NBNS Indication.

It may also contain the L2TP User Authentication IE which Authentication Type, Authentication Name, Password, Challenge, Challenge Response.

The PFCP Session Establishment Response message from UP Function to CP Function consists of

-	Created L2TP Session IE (Mandatory)

This IE provides the L2TP session information and is for statistics and diagnostics purpose. It contains the following parameters of, LAC IP Address, LAC Port, LAC Tunnel ID, LAC Session ID, LNS IP Address, LNS Port, LNS Tunnel ID, LNS Session ID.

It, may, also contain the L2TP Cause IE. This L2TP Cause IE (Optional) that contains the parameters of, Result Code.

The Created L2TP Session IE may also contain the DNS Server Address and NBNS Server Address.

NOTE:	AVPs introduced in IETF RFC 2661 [6] and IETF RFC 2868 which are used over Sgi/N6 will be checked by CT3 and possibly be documented in 3GPP TS 29.061 [14] and 3GPP TS 29.561 ]15].

### 6.8.4	End to End Signalling Flow

The end to end signalling flow in this clause is for information. For the procedure described below, it is a prerequisite that the UE is already registered into the 5GC, and both SMF and UPF support L2TP feature.

NOTE 1:	The scenario where the UE sends actual PPP frames/signaling towards the LAC, which involves back and forth message exchanges between the UE and LAC, for example, for LCP Negotiation is not in scope. The UE may send any such LCP-based information or authentication information (authentication protocol supported by the UE, that is, PAP/CHAP, and credentials such as username/password) to the SMF via the PCO.

NOTE 2:	The description is based on L2TP version 2, which is widely deployed in the network.

NOTE 3:	Similar end to end signalling flow applies for a PDN connection creation procedure when the UE is registered in EPS and request to establish a PDN connection towards a Packet Data Network requiring L2TP support.

Figure 6.8.4-1: Detailed L2TP Signalling for Tunnel/Session Setup

1.	The SMF receives a PDU session establishment request from the UE.

The UE may include additional (protocol) data (e.g. configuration parameters, error codes or messages/events) associated with an external protocol or an application in the PCO, e.g. the authentication information for PAP and/or CHAP, IPCP/PPP information to request DNS and NBNS Server. The CP Function may optionally be locally configured with the UE authentication information for a given DNN/APN.

The SMF may determine that an L2TP session is required for the PDU session based on local configuration (e.g. DNN/S-NSSAI). The SMF may retrieve the L2TP Tunnel parameters from external servers such as RADIUS/Diameter/AAA server, as described in step 2 below, or be configured locally.

The L2TP Tunnel parameters include the LNS Ipv4/Ipv6 address, LNS hostname, Tunnel Password, Tunnel Assignment ID, Tunnel Private Group ID.

Based on operator configuration, the Calling Number and the Called Number values may be configured to take the form of IMSI/MSISDN and APN Name, respectively.

The Tunnel Assignment ID is used to indicate to LAC (i.e. UP function) the particular L2TP tunnel to which a L2TP session is to be assigned, as described in clause 3.7 of IETF RFC 2868 [9] and clause 4.5.8 of IETF RFC 7155 [16]. 

NBNS server is s a server responsible for maintaining a list of mappings between NetBIOS computer names and network addresses for a network that uses NetBIOS as its naming service.

2.	The SMF may retrieve the L2TP Tunnel Parameters from the RADIUS/Diameter/AAA Server by sending a "Access Request" message or "Diameter EAP Request (DER)" message.

The following tunnel parameters can be retrieved, based on IETF RFC 2868 [9] and IETF RFC 7155 [16]: Tunnel Password, Tunnel Assignment ID, Tunnel Private Group ID. In addition, it can also, provide the L2TP version, LNS Ipv4/Ipv6 address, LNS hostname, which can also be locally configured on the CP Function.

The Tunnel Private Group ID is used to identify the session as belonging to a group that should receive some specific handling in the LNS. The Tunnel Assignment ID (TAID) is used to select an L2TP tunnel and have all the sessions created with the same TAID value in the same L2TP tunnel, and sessions with other Tunnel Assignment ID (TAID) value shall not be created on that tunnel.

The RADIUS/Diameter/AAA server responds with an "Access Response" message or "Diameter EAP Answer (DEA)" message, containing the L2TP parameters, such as, Tunnel Password, Tunnel Assignment ID (TAID), Tunnel Private Group ID, the L2TP version, LNS Ipv4/Ipv6 address and LNS hostname.

3.	If L2TP protocol is determined to support the PDN connection/PDU session, the SMF selects a UPF supporting L2TP and be configured with the LAC name/addresses (which may be included in response from Radius/Diameter/AAA server) and then requests the UPF to setup an L2TP session towards the L2TP server (LNS). For a given DNN/APN requiring L2TP function, multiple LACs may be configured to be connected multiple LNSs in the Data Network.  A UPF may be configured with the multiple LAC names and addresses for a given APN/DNN.

The SMF sends PFCP Session Establishment Request to the UPF, which includes L2TP Tunnel Information for setting up a L2TP tunnel and L2TP Session Information to setup a L2TP session, together with the information for authentication used during L2TP Tunnel setup, as well as for L2TP session. See clause 6.8.5.

The L2TP Tunnel Information includes Tunnel Password, Tunnel Assignment ID (TAID), Tunnel Private Group ID, the L2TP version, LNS Ipv4/Ipv6 address and LNS hostname.

The L2TP Session Information includes specific information related to the PDU session, e.g. a Calling Number which may be set to UE's SUPI, an indication to instruct that the UPF shall request the LNS to allocate an IP address for the PDU session, indications to instruct that the UPF shall request the LNS to provide DNS server addresses or NBNS server addresses, the MRU (Maximum Receive Unit) to use (based on the MTU size).

If the UE IP address should be allocated by the UPF (or the LNS), the SMF indicates this in the UE IP Address IEs in the PDRs and/or Traffic Endpoints.

4.	The UPF checks if any existing L2TP can be used to serve the PDU session according to the information provided in the L2TP Tunnel Information. 

If the UPF decides to setup a new L2TP tunnel, it initiates L2TP Tunnel establishment by sending an SCCRQ message towards the LNS, the UPF will allocate a Tunnel ID, and it may include a CHAP Challenge to authenticate the LNS. The Challenge and Challenge Response (to be included in SCCCN) is produced by the UPF using the Tunnel Password received from the SMF.

The LNS responds with an SCCRP message, containing its allocated Tunnel ID and a CHAP Challenge Response to the Challenge in SCCRQ. Optionally, the SCCRP message from the LNS may also contain a CHAP Challenge message towards the UPF, to authenticate the UPF.

The UPF then responds with a Challenge response for tunnel authentication in the SCCCN message. An L2TP Tunnel is established after the tunnel authentication is successful, with the reception of the SCCCN message sent by the UPF to the LNS.

If the UPF decides to use an already existing L2TP Tunnel for the requested PDU Session from the SMF, it proceeds with step 5 below.

See also IETF RFC 2661 [6].

5.	Once the L2TP Tunnel is established (or already present) between the UPF and the LNS for the PDU Session requested by the UE, the UPF proceeds with L2TP Session setup towards the LNS.

The UPF sends an ICRQ message towards the LNS, which contains the Tunnel ID assigned by the LNS, its assigned Session ID, and optionally, the Calling Number and Called Number. The LNS responds with an ICRP message and provides the Session ID assigned by it to the UPF.

The UPF then sends an ICCN message. The UPF includes the UE authentication information from the SMF received via PCO in step 1. In addition, the UPF (LAC) will act as a PPP endpoint to use LCP to communicate some link control parameters, e.g. MRU; to use PAP/CHAP to perform an authentication procedure; to use IPCP to request PDU Session IP Address, DNS and/or NBNS server address(es).

If proxy authentication is employed, the same ICCN message also provides the Proxy Authen Type and corresponding proxy authentication information to the LNS. This authentication information includes the fields of Proxy Authen Name, Proxy Authen Challenge, Proxy Authen ID and Proxy Authen Response.

If proxy authentication is successful, a success will be returned.

Otherwise, after the reception of the ICCN message, the LNS responds with a ZLB Ack Message. After the reception of ZLB Ack message, the LAC and LNS will use PPP LCP to communicate link specific control parameter, and indicate Authentication type, then either PPP PAP/CHAP takes place. The PPP IPCP transactions takes places to retrieve UE IP Address and/or DNS and/or NBNS server address.

Note:	In case the SMF sends a challenge and response for CHAP authentication, but, the LNS requires PAP, the L2TP session setup will fail, since the information required to perform PAP authentication is not available. If the UE sends PAP information, but, the LNS requires CHAP, the password sent by the UE can be used for CHAP authentication.

See also IETF RFC 2661 [6].

6.	The status of the L2TP session setup is sent by the UPF to the SMF in a PFCP Session Establishment Response.

7.	The SMF sends a PDU Session Establishment Response to the UE and the user data session is initiated, which contains the DNS and NBNS Server information.

### 6.8.5	Security Considerations for L2TP Tunnel and Session Management

#### 6.8.5.1	Security handling during an L2TP Tunnel Establishment

During an L2TP Tunnel establishment, LAC and LNS use CHAP-like tunnel authentication mechanism to authenticate each other as specified in clause 5.1.1 of IETF RFC 2661 [6]. If an LAC or LNS wishes to authenticate the identity of the peer it is contacting or being contacted by, a Challenge AVP is included in the SCCRQ or SCCRP message. If a Challenge AVP is received in an SCCRQ or SCCRP, a Challenge Response AVP MUST be sent in the following SCCRP or SCCCN, respectively. If the expected response and response received from a peer does not match, establishment of the tunnel MUST be disallowed.

To participate in tunnel authentication, a single shared secret MUST exist between the LAC and LNS, i.e. the UP Function needs to have the same shared secret as at the LNS to generate a CHAP Challenge or Challenge Response towards the LNS. The UP Function should be able to receive this shared secret from the CP Function, which receives such information from a radius server.

Such shared secret is retrieved by the CP Function from RADIUS via the encrypted "Tunnel-Password" AVP in the Radius Access-Accept message (see clause 3.5 of IETF RFC 2868 [9]). The CP Function possesses a shared secret, that is also known to the RADIUS server, to be able to decrypt the "Tunnel-Password" AVP. The CP Function, then, sends this shared secret decrypted from the "tunnel-password" to the UP Function, and the UP Function generates a Challenge or Challenge Response using this secret included in SCCRQ or SCCCN to the LNS to establish L2TP Tunnel.

When the CP Function sends the L2TP tunnel password to the UP Function over N4 via PFCP, whether the tunnel password should be sent in a protected manner, e.g. encrypted, and how is the encryption achieved needs SA3 to provide further guideline.

#### 6.8.5.2	Security handling during an L2TP Session Establishment

In order to setup an L2TP session for a PDN connection or a PDU Session, either PAP or CHAP authentication mechanisms may be used. The PPP authentication mechanisms of PAP and CHAP are described in detail in IETF RFC 1334 [12].

Any PPP-based configuration information, e.g. for PAP or CHAP, is received from the UE, which is included in the PCO, e.g. in the PDU Session Establishment Request message or Create Session Request sent towards the CP Function. The clause 10.5.6.3 in 3GPP TS 24.008 [11] specifies that the UE can send the PAP/CHAP authentication information as configuration protocol option lists in PCO to the CP Function.

The PAP authentication information from the UE in PCO consists of username and password. However, when CHAP authentication is being used by the UE, the UE sends the CHAP Challenge and Response in the PCO, which can be used by the UP Function (as Proxy Authen Challenge and Proxy Authen Challenge Response included in the ICCN message) and send towards the LAC to verify.

Whether such authentication information received from UE via PCO which is related to establish a L2TP session, should be sent in a protected manner, e.g. encrypted, and how is the encryption achieved needs SA3 to provide further guideline.

#### 6.8.5.3	Potential alternative solutions

A security mechanism may be required to send aforementioned information (Tunnel-Password (for L2TP tunnel), User name and password for PAP, Challenge and Challenge Response for CHAP (for L2TP session) as described in 6.8.5.1 and 6.8.5.2 from the CP function to the UP function, based on operator policy, e.g. when the CP function and UP function are in different security domains. The following is a list of alternatives that can be considered:

-	Relying on the Network domain security, e.g. using IPSec, as specified in 3GPP TS 33.210 [10]. If so, there is no need to develop any further security mechanism to protect the aforementioned information.

-	Using DTLS over N4/Sxb when supporting L2TP, as described in IETF RFC 6347 [13].

-	Using partial encryption of the sensitive data like RADIUS for tunnel password, as described in IETF RFC 2868, clause 3.5, where CP function and UP function is configured with a shared secret.

#### 6.8.5.4	Conclusions

As per suggested in SA3 LS (S3-212365), the NDS/IP mechanism (as specified in 3GPP TS 33.210 [10]) shall be used, if the information transferred from the CP function to the UP function needs to be protected.

## 6.9	Solution#9: Enabling UPF to support multiple network slices

### 6.9.1	Description

This solution is to address Key Issue #1 "UPF support for multiple network slice sharing". When UPF are shared by several network slices, UPF should know which slice the PFCP session (PDU session) is pertaining to according to information provided by the received S-NSSAI.

Although, in the current specifications, the Network Instance ID transmitted on N4 interface can already provide the UPF with the information, this approach may not work well in some cases, e.g. when a Network Instance ID is selected for multiple different network slices based on operator's configuration, UPF may not tell them apart only by the Network Instance ID. And in the current specification, what Network Instance ID can be encoded as is not clear that it may be encoded as a Domain Name or DNN or S-NSSAI or other defined field (see clause 8.2.4 of 3GPP TS 29.244 [3]). Different vendors or operators may have different definitions of Network Instance ID. Using S-NSSAI to let UPF know which slice the PFCP session is pertaining to would be a more direct approach. Therefore, it's proposed to release the limitation that S-NSSAI is only used for performance measurement.

### 6.9.2	Procedure

The procedure for UPF to support multi-slice sharing is showed in Figure 6.9.2-1.

Figure 6.9.2-1: Procedures for the method that UPF supports multi-slice sharing

1.	Pre-configure supported slice information in UPF.

2.	[Optional] Nnrf_NFManagement_NFRegister_request procedure as in clause 4.17.1 of TS 23.502, carrying supported slice ID.

3.	UE Registration procedure as in clause 4.2.2.2.2 of TS 23.502.

4.	PDU Session Establishment Request procedure as in clause 4.3.2.2.1 of TS 23.502.

5.	AMF searches for and selects a suitable SMF in NRF combining DNN, location, slice identification and other information, as specified in clause 4.3.2.2.1 of TS 23.502.

6.	PDU Session Establishment request procedure as in 4.3.2.2.1 of TS 23.502.

7.	SMF retrieves the user subscription data from the UDM, as specified in clause 4.3.2.2.1 of TS 23.502.

8.	[Optional] SMF searches for the appropriate UPF according to DNN, location information and also supported slice ID from NRF, as the procedure in clause 4.17.4 of TS 23.502.

9.	SMF selects a UPF based on DNN, slice identification, location and other information, as specified in clause 4.3.2.2.1 of TS 23.502.

10.	SMF sends a PFCP Session Establishment request with the slice ID to UPF, as the procedure in clause 4.3.2.2.1 in TS 23.502.

11.	UPF performs corresponding resource allocation (e.g. GTP tunnel resources, CPU resources, etc) and subsequent slice isolation according to the received Network Instance ID, Pool ID (if received) and Network slice ID (i.e. S-NSSAI IE).

12.	UPF returns PFCP Session Establishment response to SMF as in clause 4.3.2.2.1 in TS 23.502.

13.	PDU session establishment procedures, as specified in clause 4.3.2.2.1 of TS 23.502.

### 6.9.3	Impacts

UPF:

-	UPF is pre-configured with the supported S-NSSAI;

-	UPF shall allocate resources to PDU sessions according to the Network Instance ID, Pool ID (if received) and Network slice ID (i.e. S-NSSAI IE);

SMF:

-	None.

## 6.10	Solution #10: UP Controlled UE IP Portion

### 6.10.1	Description

This solution addresses the Key Issue #4 (Avoidance of conflict between UE IP address/prefix allocated by SMF(s)) and proposes to update the UP Function behaviour to support of UE IP Portions:

The UE IP address information may be configured in the UP Function, while operators may request the UE IP address to be allocated by the CP Function, as per the need of a specific deployment scenario. In this case, if one UP Function is controlled by multiple CP Functions, clash of UE IP address allocation may happen easily.

To avoid clash of UE IP allocation among multiple CP Functions, the UP Function can divide the UE IP address information configured in the UP Function into several portions and just provide one portion to each CP Function at the beginning. During PFCP Association Setup procedure, the UP Function provides one UE IP Portion to the CP Function.

A typical example is, if UE IP Address Pool(s) is configured in the UP Function, the UP Function can divide multiple UE IP Address Pool(s) into several portions or can divide one big UE IP Address Pool into multiple subsets of UE IP Address Pool.

To support this functionality, the UP Function shall:

-	Divide the configured UE IP address information (e.g. UE IP Ranges, UE IP Address Pools) into multiple UE IP Portions, when the UP Function is controlled by multiple CP Functions;

-	Provide one UE IP Portion (or small subsets of UE IP Portions) to the CP Function, during PFCP Association Setup procedure;

-	Keep monitoring to the usage of UE IP Portion assigned to a CP Function, and provide subsequent UE IP Portion to the CP Function, if the previous UE IP Portion is to be used out;

-	Recycle the UE IP Portion assigned to a CP Function, if all PFCP sessions and PFCP association to that CP Function is released;

The CP Function shall:

-	Store the UE IP Portion information received from PFCP Association Setup procedure or updated in PFCP Association Update procedure, and allocate UE IP address from the assigned UE IP Portion;

In the case one UP Function serves multiple DNNs / multiple slices and is controlled by multiple CP Functions, this solution can be used to avoid conflict of UE IP address allocation. However, this solution does not work in SMF SET deployment, due to one PDU session can be taken over by any other SMF within the SMF SET if the previous SMF fails. Other mechanisms (e.g. solution described in clause 6.2) may be used in this case.

### 6.10.2	Impacts on services, entities and interfaces

The behaviour of UP Function is enhanced to support UE IP Portion division and assignment.

### 6.10.3	Pros

If UE IP address information is configured in the UP Function and UE IP allocation is controlled by CP Function, clash of UE IP address allocation by CP Function can be eliminated as much as possible, when one UP Function is controlled by multiple CP Functions.

### 6.10.4	Cons

It requires a well-developed mechanism in the UP Function for UE IP portioning and recycling.

This solution does not work in the SMF SET deployment, thus the applicability of this solution is reduced.

Since the UP Function is configured with IP addresses in this solution, it is questionable why not let the UP Function allocate the UE IP addresses upon establishment of PFCP sessions as much as possible.

## 6.11	Solution #11: UE IP address allocation by DHCP or AAA Server in 5GC

### 6.11.1	Description

#### 6.11.1.1	General

This solution addresses the Key Issue #4 (Avoidance of conflict between UE IP address/prefix allocated by SMF(s)).

UE IP address allocation per DNN and per S-NSSAI in (external) DHCP/DN-AAA server is supported in 5GS specifications from 3GPP Rel-15 onwards. See clause 5.8.2.2 of 3GPP TS 23.501 [5] and clauses 10, 11 and 12 of 3GPP TS 29.561 [15].

The solution proposes to use DHCP or AAA severs in 5GC for the allocation of UE IP addresses. SMFs send UE IP address allocation, renewal and release related request messages to DHCP or AAA server, and maintain the corresponding state information. The IP address allocation request sent to DHCP/AAA server may include the IP address pool ID to identify which range of IP address is to be allocated. In this case, SMFs are provisioned with separate IP address pool ID(s), and the mapping between IP address pool ID and UPF Id, DNN, S-NSSAI, IP version. The provision is done by OAM or during the N4 Association Setup procedure (as per existing PFCP procedures).

#### 6.11.1.2	Using a DHCP server

DHCP provides a mechanism for passing a large set of configuration parameters to hosts connected to a TCP/IP network (IP address, sub-net mask, domain name, MTU, etc.) in an automatic manner. Moreover, DHCP may assign IP addresses to clients for a finite lease time, allowing for sequential reassignment of addresses to different users.

The SMF should request, renew and release IP addresses for PDU sessions using similar DHCP procedures as those specified in clause 10 of 3GPP TS 29.561 [15]:

-	the SMF acts as a DHCP client towards the DHCP server;

-	the SMF may determine an IP address pool ID based on the selected UPF ID, S-NSSAI, DNN, and IP version as described in clause 5.8.2.2.1 in 3GPP TS 23.501 [5] and includes the IP address pool ID within 3GPP Vendor-Specific-Option and send it to the DHCP server.

but with the following differences:

-	one (or more) DHCPv4 or DHCPv6 server is deployed within the 5GC;

-	different SMFs of an SMF set may request, renew and release the IP address assigned to given PDU session, e.g. when the PDU session is taken over by a different SMF during the lifetime of the PDU session.

The lease time is chosen by the operator of the DHCP server and may be set to a long time to minimize the need for IP address renewal. When the DHCP lease time is approaching expiry, the SMF should request the DHCP server to renew the IP address (keeping the IP address unchanged).

#### 6.11.1.3	Using a AAA server

Alternatively, the SMF should request, renew and release IP addresses for PDU sessions using similar AAA procedures described in clauses 11 and 12 of 3GPP TS 29.561 [15]:

-	the SMF acts as a RADIUS (or DIAMETER) client towards the AAA server;

-	the SMF may determine an IP address pool ID based on UPF ID, S-NSSAI, DNN, and IP version as described in clause 5.8.2.2.1 in 3GPP TS 23.501 [5] and includes the IP address pool ID within 3GPP-IP-Address-Pool-Info and send it to the AAA; the AAA assigns IPv6 prefix or IPv4 address from the requested IP address pool;

-	the SMF should indicate the type of IP address to be allocated by including the 3GPP-Allocate-IP-Type subattribute in the Access-Request message;

but with the following differences:

-	one (or more) AAA server is deployed within the 5GC;

-	different SMFs of an SMF set may request, renew and release the IP address assigned to given PDU session, e.g. when the PDU session is taken over by a different SMF during the lifetime of the PDU session;

The lease time is chosen by the operator of the AAA server and may be set to a long time to minimize the need for IP address renewal. When the lease time is approaching expiry, the SMF should initiate a AAA re-authorization to renew the IP address (keeping the IP address unchanged.

NOTE:	The lease time is stored in the PDU session context. SMFs of an SMF set share their contexts. So any SMF that may take over the control of the PDU session can determine whether the lease time is approaching expiry.

### 6.11.2	Impacts on services, entities and interfaces

Impacts:

-	DCHP or AAA servers are deployed in the 5GC for the allocation of UE IP addresses;

-	SMFs run existing DHCP or AAA procedures but towards a DHCP or AAA server located in 5GC.

### 6.11.3	Pros

The solution has the following pros:

-	the solution supports UE IP addresses pools shared by SMFs from the same or different SMF sets;

-	UE IP addresses are configured centrally in DHCP or AAA server, and assigned consistently to all SMFs without collisions of UE IP address allocation;

-	SMFs can be scaled-in or out in the SMF set without any impact to the UE IP addresses configuration and allocation and without impacts to existing PFCP sessions;

-	the solution has minimal impacts to SMF as it reuses existing procedures supported towards DN;

-	central configuration and allocation of UE IP addresses allow monitoring (e.g. KPIs) of the usage of IP addresses in the SMF set:

-	the solution does not require standardization work as it leverages existing solutions;

-	the solution allows full sharing of all UE IP addresses across all SMFs (as opposed to solutions based on IP addresses partitioning between different SMFs).

### 6.11.4	Cons

The solution has the following pros:

-	it requires the deployment of DHCP and AAA servers in the 5GC for UE IP address allocations;

-	it requires signalling between SMFs and the DHCP or AAA server during PDU session establishments and release, and possibly for UE IP address renewal, causing slight latency, but this is not different to what happens with external DHCP and AAA servers.

## 6.12	Solution #12: UE IP address/prefix allocation by SMF self management

### 6.12.1	Description

#### 6.12.1.1	General

The description below is to address the solution for Key Issue# 4 and it is based on solution#2 with some changes. The clause 6.2.1.1 in solution#2 can be reused.

The SMFs of an SMF set share the same contexts data where the UE IP address is part of the context information of a PDU session. So, each SMF in SMF set know all the UE IP addresses which is been used in this SMF set by checking the shared context data. These UE IP addresses information can be used for the SMFs to synchronize the IP usage record during SMF recover, session release and SMF added/removed to/from SMF set.

#### 6.12.1.2	SMF recover

If the SMF is recovered from a failure, it will check all the UE IP addresses information from the shared context data and make the records for its individual UE IP address/prefix list. If the UE IP address/prefix is included in the shared context data, the SMF shall mark it as unavailable.

#### 6.12.1.3	Session release

If the SMF release the UE IP address(es)/prefix(s) which are not belonging to its individual UE IP address/prefix list, these UE IP address/prefix might be marked as unavailable by the original SMF without synchronization. In order to avoid this, all the SMFs in SMF set can periodically check all the UE IP addresses information from the shared context data and update the records for its individual UE IP address/prefix list. If the UE IP address/prefix is not included in the shared context data, the SMF will update the record as available.

#### 6.12.1.4	SMF added/removed to/from SMF set

If SMF(s) added/removed to/from the SMF set, the UE IP address/prefix resource will be re-assigned depends on operator's policy. Each SMF will get the new UE IP address/prefix table which is different from the old one.

Some UE IP address(es)/prefix(es) originally controlled by one SMF might be partitioned to another SMF within the same SMF set. Two possible scenarios are list as below.

-	UE IP address(es)/prefix(es) originally controlled by one SMF which is removed from the SMF set but re-assigned to another target SMF.

-	UE IP address(es)/prefix(es) originally controlled by one SMF but re-assigned to another target SMF.

For the UE IP address(es)/prefix(es) originally controlled by one SMF which is removed from the SMF set but re-assigned to another target SMF, the target SMF will check all the UE IP addresses information from the shared context data and make the records for these new individual UE IP address(es)/prefix(es). If the UE IP address/prefix is included in the shared context data, the target SMF shall mark it as unavailable.

For the UE IP address(es)/prefix(es) originally controlled by one SMF but re-assigned to another target SMF, the original SMF shall mark all the UE IP address(es)/prefix(es) as available when the PDU session is not controlled by its own. The target SMF will check all the UE IP addresses information from the shared context data and make the records for these new individual UE IP address(es)/prefix(es). If the UE IP address/prefix is included in the shared context data, the target SMF shall mark it as unavailable.

### 6.12.2	Impacts on services, entities and interfaces

The solution requires no impacts on services, entities and interfaces.

### 6.12.3	Pros

-	The proposed solution enables the UE IP address/prefix allocation by SMF if SSET and MPAS feature deployed.

-	In early stage of 5G deployment, the operator may choose CP function to support the UE IP address/prefix allocation since CP function is required to support this feature as mandatory. But along with the network and business expansion, SSET and MPAS feature may be deployed. Then the operator may want to have the same UE IP address/prefix allocation policy with SSET and MPAS to ensure the solution continuity. This solution ensures the customer can choose the same UE IP address/prefix allocation policy when SSET and MPAS feature deployed.

-	From the UE IP planning point of view, the partitioning of UE IP addresses/prefixes for each SMF in SMF set will be easy when the SMF is added/removed from the SMF set. When new SMF added/removed to/from SMF set, the UE IP resource in SMF set can be simply divided to each SMF as individual UE IP addresses/prefixes list no matter what is the real UE IP resource usage status. And the available UE IP addresses/prefix for each SMF can be contiguous.

-	No external server like AAA/DHCP required to be deployed for SMF set.

-	There is no signalling exchange between the SMF and the external server during the PDU session establishments and release which will be time efficient with no latency.

### 6.12.4	Cons

-	It requires support of an OAM or external entity to decide the IP addresses partitioning between the SMFs of an SMF set.

-	The solution statically allocates IP addresses to SMFs of the SMF set, which might result in the SMF set failing to establish new sessions when IP addresses are still available (e.g. an SMF has failed w/o restart, the IP addresses of that SMF are not re-allocated to other SMFs).

-	This solution requires each SMF in SMF set records the whole UE IP address/prefix table which includes the individual UE IP resource part, the recorded UE IP resource part, and the usage indication of every UE IP address/prefix.

-	This solution is restricted to SMF set with share context, cannot cover the different SMFs from different SMF sets scenario listed in key issue#4.

-	This solution adds complexity to SMF implementation for the allocation of IP addresses, re-allocation of IP addresses when an SMF is added or removed from the set, SMF recover and session release, to check all the UE IP addresses information from the shared context data.

-	If the PDU session is served by original SMF, but the IP address is re-assigned to the target SMF, the SMF shall periodically check all the UE IP addresses information from the shared context data, otherwise the IP address will be unavailable even the PDU session is released.

## 6.13	Solution #13: Per Slice UP Resource Allocation and Usage Report

### 6.12.1	Description

The solution is to address the Key Issue #1: UPF support for multiple network slices in clause 5.2.

An operator may configure their UP Functions to support multiple slices, e.g. a category of network slices with the same slice type but different slice differentiator, for similar businesses belonging to different customers.

To support per slice UP resource management and usage report, the UP Function shall:

-	Allocate the UP resource (e.g. UE IP address) taking into account the Network Instance ID and the S-NSSAI IEs indicated by the CP Function in a PFCP Session Establishment / Modification request;

-	Report the usage of UP resources for a specific network slice, periodically or when the usage reaches the configured threshold;

The N4 interface thus needs the following enhancements:

-	Add S-NSSAI to the UE IP Address Usage Information IE, to support per slice UE IP address usage report;

### 6.13.2	Impacts on services, entities and interfaces

This solution requires UP Function to allocate the UP resources per network slice, and report the usage of UP resources at per slice granularity. N4 interface needs enhancements to support the report of UP resources at per slice granularity.

### 6.13.3	Pros

This solution meets the operator requirements of allocating UP resources and reporting the usage of UP resources at per slice granularity, especially for certain deployment scenario where one UP Function serves different slices targeting different customers.

### 6.13.4	Cons

Enhancements to the UP Function and N4 interface are required.

# 7	Evaluations and Conclusions

## 7.1	Evaluation and Conclusions of Solutions for Key Issue #1

### 7.1.1	Evaluation

For Key Issue #1 – "UPF support for multiple network slice sharing", candidate Solutions #3 and #9 have been proposed.

Candidate solution #3 proposes to map the network slice information, DNN, IP domain, etc. into the Network Instance, and to configure the network instance in SMF and UPF. When establish a PFCP session, the SMF will instruct UPF which network instance should be used for the PFCP session. The network instance is used to achieve multiple network slice sharing.

Candidate solution #9 proposes that UPF can use the S-NSSAI received from SMF to know which slice the PFCP session (PDU Session) is pertaining to and shall allocate resources to PDU sessions according to the Network Instance ID, Pool ID (if received) and Network slice ID (i.e. S-NSSAI IE).

Table 7.1.1-1 summarizes the evaluation of candidate solutions for key issue #1.

Table 7.1.1-1: Evaluation for Key Issue #1

| Analysis | Solutions Overview |  |
| --- | --- | --- |
|  | Solution #3 | Solution #9 |
| Impacts (software) | The Network is pre-configured with a list of Network Instances mapped from a few factors, e.g. S-NSSAI, DNN/APN, which require relevant packets to be treated differently in the user plane. Note: The solution #3 corresponds to the solution defined in existing standards. | UPF is implemented with a separate logic to allocate user plane resource based on the S-NSSAI. |
| Operational | SMF considers the network slice information, DNN, IP domain, etc. and select a Network Instance as configured by the network. | UPF shall allocate resources to PDU sessions according to the Network Instance ID, Pool ID (if received) and Network slice ID (i.e. S-NSSAI IE). |
| Standardization impacts | It requires no extra stage 2 modifications and introduces no new call flows or IEs. | It introduces no new call flows or IEs in stage 2. |
| Advantages | This solution has no impact on SMF and UPF. | This solution has no impact on SMF. UPF can know at once which slice the PDU session is pertaining to according to the S-NSSAI IE without having to derive this information from mappings. |
| Disadvantages | This solution requires provisioning of the mappings in SMF and in UPF. This solution requires the operators to assign a distinct Network Instance ID for every network slice, which increase the provisioning effort in SMF and UPF. | This solution requires UP function to have a separate logic other than existing one which is using a single parameter Network Instance, to allocate or manage the user plane resource, it leads extra complexities. |

### 7.1.2	Conclusions

The following conclusions are agreed:

-	Solution #3 is already supported in the existing standard and has no standard impact on UPF and SMF, but in the scenario where UPF supports multiple slices sharing, it needs the SMF to map the S-NSSAI to a specific NI and require additional configuration.

-	Solution #9 and solution #13 provides the flexibility to fulfil the various requirements for the scenario without changing the mapping of Network Instance in SMF and can manage UPF resources at per slice level in a straightforward way, but UPF implementation shall have a logic/configuration to select resources using S-NSSAI together with Network Instance. In addition, solution #13 supports per slice UP resource usage report with minimal impact on SMF.

Since both solutions can fulfil the scenario where UPF supports for multiple slices, and they can be used to manage UPF resources at different level in the various cases, it is concluded that solution #3 which relates to the solution defined in the current specification shall be supported. In addition, solution #9 and solution #13 may be supported to support the operator's requirement in various business scenarios.

Solution #9 and solution #13 will be standardized in 3GPP Rel-17 as a possible alternative optional solution to the existing solution#3.

## 7.2	Evaluation and Conclusions of Solutions for Key Issue #2

### 7.2.1	Evaluation

The Key Issue #2 "Header Enrichment for HTTPS" as described in clause 5.3 studies how to support the requirement of header enrichment when HTTPS scheme is used.

SA3 commented that solution #1 "Header Enrichment for HTTPS" as described in clause 6.1 has potential issues on the security aspect (see LS from SA3 in S3-214505).

### 7.2.2	Conclusions

Based on the discussion in CT4#108-E meeting, no solution will be standardized for the key issue #2.

## 7.3	Evaluation and Conclusions of Solutions for Key Issue #3

### 7.3.1	Evaluation

A large and ever-growing list of PFCP features have been specified since 3GPP Rel-14, which causes potential deviations between CP function and UP function implementations from different vendors and potential risks of interoperability issues.

Table 7.3.1-1 summarizes the solutions that have been proposed regarding specific technical options.

Table 7.3.1-1: Evaluation for Key Issue #3

| Feature | Solutions |  |
| --- | --- | --- |
|  | Solution Number | Solution Overview |
| End Marker: packets generation by CP or UP function (KI#3a) | #5 (see clause 6.5) | Rel-17 onwards UPF shall support constructing End Marker packets. It is recommended to construct End Marker packets in the UPF as the preferred option. |
| UE IP address allocation mode: by CP or UP function (KI#3b) | No supplementary solution to existing mechanism defined in 3GPP TS 29.244 [3] identified during the study. | Keep existing mechanism defined in 3GPP TS 29.244 [3]. |
| Downlink data buffering in CP or UP function (KI#3c) | #4 (see clause 6.4) | Downlink data should be buffered preferably in the UP function. Downlink data may be buffered in CP function when needed e.g. for Ues using power saving methods or when using Control Plane CioT 5GS Optimisation. |
| Traffic redirection enforcement in CP or UP function (KI#3d) | #6 (see clause 6.6) | Traffic redirection shall be enforced in the UP function. The option to enforce traffic redirection in the CP function in EPC will be removed from 3GPP Rel-17 onwards. |
| PDI optimization: Identify whether the support of certain features could be recommended or mandated for NFs complying with 3GPP Release 17 (KI#3e) | #7 (see clause 6.7) | 3GPP Rel-17 onwards CP and UP functions should support the PDI Optimization feature. |

### 7.3.2	Conclusions

The following conclusions are agreed:

-	End Marker packets generation: solution #5 is agreed.

-	UE IP address allocation mode: keep existing mechanism defined in 3GPP TS 29.244 [3].

-	Downlink data buffering: solution #4 is agreed.

-	Traffic redirection enforcement: solution #6 is agreed

-	PDI Optimization feature: solution #7 is agreed.

## 7.4	Evaluation and conclusions of solutions for key issue #4

### 7.4.1	Evaluation

Candidate solution #2, solution #11 and solution#12 have been proposed for Key Issue #4 – "Avoidance of conflict between UE IP address/prefix allocated by SMF(s)". The candidate solutions provide different mechanisms to avoid the UE IP address/prefix allocation by SMF in SMF set.

Within solution #2, two solutions are proposed in case SMF added/removed to/from SMF set.

Table 7.4.1-1: Evaluation for Key Issue #4

| Analysis | Solution Overview |  |  |  |
| --- | --- | --- | --- | --- |
|  | Solution #2 |  | Solution #11 | Solution #12 |
|  | Local configuration | directly controlled by external entity |  |  |
| Enclosing scope | SMFs in the same SMF set |  | SMFs in the same SMF set or different SMF sets. | SMFs in the same SMF set |
| Impact(Inter SMFs) | It requires a new interface between SMFs in SMF set. It requires lots of inter SMF signaling within SMF set. |  | No requirement. | No requirement. |
| Impact(External Server) | No requirement. | It requires an OAM or external entity to re-assign the UE IP address/prefix in case SMF added/removed to/from SMF set. | It requires the deployment of DHCP and AAA servers in the 5GC for UE IP address allocations and the signalling interactions between SMFs and the DHCP or AAA server during PDU session establishments and release, and possibly during UE IP address renewal, which introduce slight latency. |  |
| Impact (IP partitioning) | It requires an OAM or external entity (e.g. Operator's IP planner) to decide the IP addresses partitioning between the SMFs of an SMF set. |  | No IP partitioning required within SMF set. | Same as Solution#2 |
| UE IP address planning and usage ratio | From the operator point of view, the available UE IP addresses/prefix for each SMF can be contiguous and well planned. During SMF failure or restart, some UE IP addresses might not be used by other SMFs since the UE IP addresses are statically allocated to each SMF within SMF set. This is caused by static allocation (Local configuration solution) or transient period during OAM re-assignment (Directly controlled by external entity solution). |  | The UE IP addresses are configured centrally in DHCP or AAA server, and assigned consistently to all SMFs without collisions of UE IP address allocation; Central configuration and allocation of UE IP addresses allow monitoring (e.g. KPIs) of the usage of IP addresses in the SMF set. | Same as Solution#2 |
| Impact (SMF/SMF Set logical processing) | It requires each SMF in SMF set records the whole UE IP address/prefix table and the IP usage indication for each address/prefix. It requires high reliability for message exchange between SMFs since any failure (e.g. transient communication failure between 2 SMFs, SMF overload) can result in SMFs getting out of synchronization with regard to IP addresses usage. It requires the IP addresses allocation/re-allocation when SMF is added or removed from the set and also, it requires advertisement of releasing IP addresses owned by a different SMF of the SMF set. |  | It requires the SMF acts as a DHCP/RADIUS (or DIAMETER) client towards the DHCP/AAA server which is the same behaviour as already defined for SMF (from Rel-15 onwards). It reuses the same UE IP address allocation as already supported for UE IP address allocation by a DN-AAA. It requires the target SMF to check the IP lease time in shared context data for the PDU session which is switched to this SMF. And it requires the SMF to initiate a AAA re-authorization to renew the IP address(keeping the IP address upchanged) when the lease time is approaching expiry. | It requires each SMF in SMF set records the whole UE IP address/prefix table and the IP usage indication for each address/prefix. It requires SMF to check the shared context data in different cases. E.g. SMF restart, SMF added or removed from the set, IP re-assigned to the target SMF. It requires the IP addresses allocation/re-allocation when SMF is added or removed from the set. |

According to the evaluation table above, a brief evaluation for each solution is list as below:

The solution #2 covered SMFs in the same SMF set and it requires a new interface and lots of inter SMF signaling between SMFs in SMF set.

The solution #11 have covered SMFs in the same SMF set or different SMF sets and it re-use the same IP allocation as already supported by DN-AAA. To avoid the IP expiry, it requires the target SMF to check the IP lease time in shared context data for the PDU session which is switched to this SMF. This solution has re-used the current IP allocation mechanism with a few tweaks.

The solution #12 covered SMFs in the same SMF set and it re-use the information in shared context data. But it requires the IP re-allocation when SMF is added or removed and it may cause the SMF periodically checked the shared context data for synchronization.

### 7.4.2	Conclusions

Solution#11 is adopted for KI#4. The following work is needed during the normative phase:

-	The normative requirements related to this capability in CP function will be specified in 3GPP TS 29.244 [3] based the clauses 5.21.1.

-	The major part of the IP allocation mechanism in solution #11 is already supported. But with the special session handling in SMF set, e.g. PDU session can be taken over by a different SMF during the lifetime of the PDU session, there are some different requirements compared to the existing DHCP/AAA procesures. Therefore, such information should be standardized in 3GPP TS 29.244 [3] clauses 5.21.2 based on the solution#11 as described in clause 6.11.1.

## 7.5	Evaluation and Conclusions of Solutions for Key Issue #5

### 7.5.1	Evaluation

The Key Issue #5 "L2TP Tunnelling over SGi/N6" as described in clause 5.6. studies how to provide a solution to support L2TP, which is a common technology widely deployed in operator's network and used e.g. by POS/ATM machine to establish secured connection with its server, when CUPS (Control Plane and User Plane Separation) is deployed.

One of potential alternative that the CP function is served as a LAC (L2TP Access Concentrator) to establish L2TP tunnel with a LNS (L2TP Access Concentrator) has been discussed but quickly abandoned since the solution will force user plane path goes via control plane, which breaks the fundamentals of CUPS.

Solution #8 "L2TP Access Concentrator" as described in clause 6.8 has covered all aspects as defined in the key issue, therefore it is considered as suitable solution.

### 7.5.2	Conclusions

The following conclusions are agreed:

-	the PFCP protocol extensions to support L2TP tunneling over N6/SGi for 5GC/EPS is to be standardized based on the solution#8 as described in 6.8 in Rel-17.

-	The normative requirements related to transfer L2TP Tunnel Information and L2TP Session Information from the CP function to the UP function together with a short description of L2TP function is to be specified in 3GPP TS 29.244 [3] based the clauses 6.8.1, 6.8.2 and 6.8.3.

-	Existing NDS/IP security mechanism (as specified in 3GPP TS 33.210 [10]) shall be used, if the information transferred from the CP function to the UP function needs to be protected.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | Tdoc | CR | Rev | Cat | Subject/Comment | New version |
| 2020-06 |  | C4-203628 |  |  |  | Version after CT4#98-e including agreed pCRs: C4-203442 C4-203443 C4-203444 C4-203481 C4-203493 | 0.1.0 |
| 2020-09 |  | C4-204585 |  |  |  | Version after CT4#99-e including agreed pCRs: C4-204240 C4-204346 C4-204419 C4-204474 C4-204510 C4-204511 C4-204529 C4-204543 | 0.2.0 |
| 2020-11 |  | C4-205775 |  |  |  | Version after CT4#101-e including agreed pCRs: C4-205509 C4-205527 C4-205777 C4-205778 C4-205779 C4-205780 C4-205781 C4-205791 C4-205805 | 0.3.0 |
| 2021-02 |  | C4-210243 |  |  |  | Version after CT4#101-bis-e including agreed pCRs: C4-210040 C4-210041 C4-210102 C4-210154 C4-210168 C4-210169 C4-210170 C4-210181 C4-210214 C4-210220 C4-210223 | 0.4.0 |
| 2021-03 |  | C4-211755 |  |  |  | Version after CT4#102-e including agreed pCRs: C4-211010 C4-211011 C4-211376 C4-211385 C4-211480 C4-211481 C4-211482 C4-211539 C4-211580 C4-211594 C4-211623 C4-211660 C4-211713 C4-211720 C4-211823 | 0.5.0 |
| 2021-04 |  | C4-212589 |  |  |  | Version after CT4#103-e including agreed pCRs: C4-212297 C4-212433 C4-212463 C4-212464 C4-212614 | 0.6.0 |
| 2021-06 |  | C4-213519 |  |  |  | Version after CT4#104-e including agreed pCRs: C4-213590 | 0.7.0 |
| 2021-10 |  | C4-215524 |  |  |  | Version after CT4#106-e including agreed pCRs: C4-215399 C4-215479 | 0.8.0 |
| 2021-11 |  | C4-216463 |  |  |  | Version after CT4#107-e including agreed pCRs: C4-216417 | 0.9.0 |
| 2021-12 | CT#94 | CP-213150 |  |  |  | V1.0.0 presented for information | 1.0.0 |
| 2022-01 |  | C4-220449 |  |  |  | Version after CT4#107BIS-e including agreed pCRs: C4-220144 C4-220157 C4-220158 C4-220363 C4-220442 | 1.1.0 |
| 2022-02 |  | C4-221586 |  |  |  | Version after CT4#108-e including agreed pCRs: C4-221117 C4-221446 C4-221635 | 1.2.0 |