| 3GPP TR 29.867 V0.3.0 (2025-10) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Core Network and Terminals; Study on IMS Resiliency (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

Introduction	6

1	Scope	7

2	References	7

3	Definitions of terms, symbols and abbreviations	7

3.1	Terms	7

3.2	Symbols	8

3.3	Abbreviations	8

4	Baseline	8

5	Key Issues	8

5.1	Key Issue #1: Preventing UE SIP Registration attempts during IMS Failures	8

5.1.1	Description of the scenario	8

5.1.2	Key issue definition	11

6	Solutions	11

6.1	Solution #1: P-CSCF failure/recovery detection in IMS, employing IMS back off timer	11

6.1.1	Description	11

6.1.1.1	General	11

6.1.1.2	P-CSCF failure/recovery detection	12

6.1.1.2.1	P-CSCF failure detection	12

6.1.1.2.2	P-CSCF recovery detection and new registration	13

6.1.2	Impacts on services, entities, and interfaces	13

6.1.3	Pros	13

6.1.4	Cons	13

6.2	Solution #2: P-CSCF failure notification with PCO/ePCO	13

6.2.1	Description	13

6.2.1.1	General	13

6.2.1.2	P-CSCF failure/recovery detection	14

6.2.1.2.1	P-CSCF failure/recovery detection via PCF	14

6.2.1.2.2	P-CSCF failure/recovery detection via NRF	14

6.2.1.2.3	P-CSCF failure/recovery detection through heart-beat monitoring:	14

6.2.1.2.4	P-CSCF failure/recovery detection by automated operator specific OAM operation	15

6.2.1.3	P-CSCF failure notification to UE	15

6.2.1.3.1	P-CSCF failure notification with PCO/ePCO	15

6.2.1.3.2	Notifying P-CSCF failure notification feature with UE Capability Indication	15

6.2.1.3.3	Procedures for P-CSCF failure notification to UE	16

6.2.1.4	P-CSCF recovery notification to UE	18

6.2.1.5	Deadlock prevention	18

6.2.1.5.1	Re-establish the IMS PDN connection	18

6.2.2	Impacts on services, entities, and interfaces	18

6.2.3	Pros	19

6.2.4	Cons	19

6.3	Solution #3: SMF-initiated PDU session release	19

6.3.1	General	19

6.3.2	Description	19

6.3.2.1	P-CSCF failure detection in SMF	19

6.3.2.1.1	P-CSCF failure detection by using P-CSCF Restoration feature	20

6.3.2.1.2	P-CSCF failure detection via PCF or NRF	20

6.3.2.2	SMF-initiated PDU session release	20

6.3.3	Impacts on services, entities, and interfaces	20

6.3.4	Pros	20

6.3.5	Cons	20

6.4	Solution #4: Access restriction with UAC Access Category 9	21

6.4.1	Description	21

6.4.1.1	Manual recovery	21

6.4.1.2	Automatic recovery	21

6.4.2	Impacts on services, entities, and interfaces	21

6.4.3	Pros	21

6.4.4	Cons	22

7	Evaluations	22

7.1	Evaluation of Solutions for Key Issue#1	22

8	Conclusions	23

Annex A (informative): Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

This Technical Report aims to study potential enhancements on IMS Resiliency, to increase the stability of the IMS system.

# 1	Scope

The present document investigates and identifies enhancements on 5GS and IMS to support the network resiliency. The document will focus on the following aspects:

-	Study and identify IMS congestion scenarios and whether lack of coordination between UE and network exacerbates the congestion

-	Study whether the current standardised solutions can work for the identified scenarios or not

-	If the existing solutions cannot work for the identified scenarios, then study new potential solutions

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.380: "IMS Restoration Procedures".

[3]	3GPP TS 24.229: "IP Multimedia Call Control Protocol based on Session Control Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[4]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[5]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[6]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[7]	3GPP TS 38.300: "NR and NG-RAN Overall Description; Stage 2".

[8]	3GPP TR 29.866: "Study on IMS Disaster Prevention and Restoration Enhancement".

[9]	IETF RFC 5626: "Managing Client Initiated Connections in the Session Initiation Protocol (SIP)".

[10]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[11]	3GPP TS 29.514: "5G System; Policy Authorization Service; Stage 3".

[12]	3GPP TS 29.512: "5G System; Session Management Policy Control Service; Stage 3".

# 3	Definitions of terms, symbols and abbreviations

This clause and its three (sub) clauses are mandatory. The contents shall be shown as "void" if the TS/TR does not define any terms, symbols, or abbreviations.

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Definition format (Normal)

<defined term>: <definition>.

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

CSCF	Call Server Control Function

HSS	Home Subscriber Server

I-CSCF	Interrogating-CSCF

IMS	IP Multimedia Subsystem

PCF	Policy Control Function

P-CSCF	Proxy-CSCF

PGW	PDN Gateway

S-CSCF	Serving CSCF

UDM	Unified Data Management

DNN	Data Network Name

SMF	Session Management Function

PDU	Protocol Data Unit

# 4	Baseline

The IMS Restoration Procedures as specified in 3GPP TS 23.380 [2] shall be taken as the baseline for the study on potential enhancements for IMS Resiliency, to increase the stability of the IMS system.

# 5	Key Issues

## 5.1	Key Issue #1: Preventing UE SIP Registration attempts during IMS Failures

### 5.1.1	Description of the scenario

When a failure occurs in IMS, depending on the type of failure, a number of UEs may experience failed SIP Registration attempts, leading to mass retransmissions in certain configurations. This surge in SIP Registration requests might lead to congestion of IMS nodes, impacting both voice services and data services, making failure recovery increasingly challenging.

Furthermore, if an operator providing voice services to 40 million subscribers sets the SIP Registration retransmission interval to 40 minutes following a severe IMS failure that affects the whole network, nearly 1 million SIP Registration requests per minute would flood the IMS, exacerbating network congestion.

The expected IMS congestion scenarios that can trigger SIP Registration from UEs:

-	P-CSCF related failures: This scenario includes failures of the P-CSCF itself, network failures between PGW/UPF and P-CSCF, and network failures between PCRF/PCF and P-CSCF. In the event of a failure in this scenario, the P-CSCF does not respond to the SIP message or returns a response with error code. For no response to the SIP message, some UEs may repeatedly attempt SIP Registration, while others cannot determine whether there is an abnormality in the IMS or if there is an issue with data connectivity in the IMS PDN.

-	I/S-CSCF related failures: This scenario includes failures of I-CSCF itself, S-CSCF itself, network failures between the I-CSCF and S-CSCF and network failures between the P-CSCF and I-CSCF. In the event of a failure in this scenario, the I-CSCF and S-CSCF may not respond to the SIP message. Consequently, the P-CSCF will send a SIP error, including a Retry-After message to the UEs. Upon receiving the message, the UE will initiate a SIP Registration request after a delay based on the timer value included in the Retry-After message. This results in mitigating the load and avoiding the overload.

-	HSS/UDM related failures: This scenario includes failures of the HSS/UDM itself, network failures between UDM and S-CSCF, and network failures between the HSS/UDM and I-CSCF. In the event of a failure in this scenario, the HSS/UDM may not respond to the Diameter message from the S-CSCF and I-CSCF, may respond with a delay, or may return a response with error code. Consequently, the P-CSCF will send a SIP error, including a Retry-After message to the UEs. Upon receiving the message, the UE will initiate a SIP Registration request after a delay based on the timer value included in the Retry-After message. This results in mitigating the load and avoiding the overload.

Figure 5.1.1-1: IMS failures scenarios for EPS

Figure 5.1.1-2: IMS failures scenarios for 5GS

Several standard solutions exist for different IMS failure scenarios, but not resolving the described issues:

-	P-CSCF Restoration (3GPP TS 23.380 [2]): Mechanism to ensure service continuity by redirecting UEs to an alternative P-CSCF using triggers such as PCO/ePCO, SIP 504 responses, or PDN disconnection. The issues in the solution are as follows:

-	In the scenario where multiple P-CSCF instances fail and the P-CSCF instance selected by the UE is unavailable, the UE continues retrying SIP registration request until it finds an available P-CSCF instance.

-	The GGSN/PGW sends the P-CSCF addresses to the UEs almost simultaneously, and the UEs sends the SIP registration almost simultaneously, which may lead to network congestion.

-	Overload Control (3GPP TS 23.228 [3]): Mechanism to mitigate IMS congestion by limiting SIP Registration attempts. Especially it is commonly used to apply traffic restrictions from the UEs at the P-CSCF, protecting the I-CSCF and S-CSCF, which serve as backend components of the P-CSCF. For example, when Overload Control on the P-CSCF, it is assumed that P-CSCF will send a SIP error, including a Retry-After message as defined in 3GPP TS 24.229 [3] to the regulated UEs. Upon receiving the message, the UE will send a SIP Registration request after a delay based on the timer value included in the Retry-After message. Whether this solution has issues needs to be studied.

-	The solution is designed to protect IMS, so it does not stop but delays SIP Registration requests from UEs to IMS. As a result of UEs repeatedly (potentially with increasing delays) retransmitting SIP Registration.

-	The solution is designed to limit UE access attempts (e.g., SIP Registration request) during the IMS high load to prevent overload and congestion, addressing the scenarios involving IMS failures or precaution measurement to prevent IMS congestion caused by frequent SIP registrations.

-	Congestion caused by SIP Registration can be mitigated by the Overload Control. However, once this mechanism is triggered, it inevitably impacts new registrations and possibly existing voice services to some extent.

-	UAC (3GPP TS 23.501 [5], TS 24.501 [6], TS 38.300 [7]): Mechanism to protect IMS from overload and congestion by setting access categories for IMS registration-related signalling in NG-RAN, triggered by OAM requests or NG-RAN itself. The potential issues in the solution to be studied are as follows:

-	Since there is no clear correlation between the node selection logic of IMS and IMS-related NFs (PGW/PCRF) and the base station's cell or PLMN, it is not possible to precisely apply access control and barring to the UEs affected by the failure. Therefore, to impose the access control and barring that cover 100% of the affected UEs, it is necessary to set UAC excessively, which results in suspending voice services for UEs that are originally unrelated.

-	The solution depends on the 3GPP RAN implementation. Therefore, it is not effective for VoWi-Fi.

-	UAC can only be applied to the RAN operated in house, so IMS failures in roaming-out or IMS failures in MVNO operators cannot be addressed.

-	It is assumed that addressing SIP layer issues between the UE and IMS using protocols between the UE and RAN presents technical challenges. Ideally, the issue should be handled using a protocol closer to the SIP layer.

-	The solution is designed to limit UE access attempts (e.g., SIP Registration request) during the IMS overload and congestion, it does not address the scenarios involving IMS failures or precaution measurement to prevent IMS congestion caused by frequent SIP registrations.

-	From the perspective of actual operation, the following issues are identified:

-	It is necessary to impose access control and barring via UAC on NG-RANs affected by the IMS overload/congestion. This linkage between the area affected by IMS overload and congestion and NG-RAN depends on the operator’s design. However, node bypassing frequently occurs due to various reasons such as maintenance work. As a result, maintaining the uniqueness of the relationship between IMS and RAN is impossible. Therefore, presetting and fixing the linkage configuration would be meaningless.

-	Access control and barring in UAC may take time to become effective depending on the area and scale. When a failure occurs in IMS, within just a few seconds, tens to hundreds of thousands of UEs that failed to SIP Registration requests might emerge, so it does not have an immediate effect.

-	The solution is effective for UEs in specific areas, such as Tracking Areas. However, when considering a nationwide large-scale failure, the effectiveness of the restrictions is expected to be limited. For UEs under restrictions, they will continue retransmitting requests until they succeed. As a result, in the case of a nationwide failure, despite the restrictions imposed, the traffic load on the core network may not be reduced as expected.

-	Dynamic extension of registration expiration timers (3GPP TR 29.866 [8], KI#4/Sol#10): This solution help mitigate HSS overload by adjusting the retransmission timer interval of the UE. The potential issues in the solution to be studied are as follows:

-	This solution cannot stop SIP Registration attempts from the UE itself during the overload.

-	If all HSS instances become unavailable and the HSS cannot send the message, this solution cannot be executed.

-	UE's usage setting (3GPP TS 24.501 [6]): Mechanism defining the preference of voice over data services (or vice versa) for voice-capable UEs. The potential issues in the solution to be studied are as follows:

-	The majority of UEs are configured with a fixed "voice centric" or "data centric", making them dependent on the UE's implementation.

-	During an IMS failure, UEs with a "voice-centric" configuration initiate attach/registration requests, while those with a "data-centric" configuration do not. Nevertheless, periodic SIP registration continues, meaning the SIP registration cannot be stopped during failures.

-	It is technically possible to handle it via firmware updates such as OTA, depending on the UE's implementation, However, the impact of regression testing is significant, making it unusable during failures.

As a result, the UE fails to receive the expected response.

### 5.1.2	Key issue definition

This key issue will be studied in the following aspects:

-	in the case of all available P-CSCFs failure, how to detect the failure and prevent the UE from repeatedly attempting the IMS Registration procedure.

# 6	Solutions

## 6.1	Solution #1: P-CSCF failure/recovery detection in IMS, employing IMS back off timer

### 6.1.1	Description

#### 6.1.1.1	General

This solution is proposed to address Key Issue #1 "Preventing UE SIP Registration attempts during IMS Failures".

In KI#1, the IMS congestion scenarios were identified, and an analysis of the existing standard solutions found that during outage of all P-CSCFs, the UE was unable to receive any response, making it impossible to prevent the UE from repeatedly attempting the SIP Registration procedure.

To address the issue, this solution proposes to use existing timeout mechanism during SIP Registration (Timer F expires for all P-CSCF that are in the list of P-CSCF for a UE) procedure to trigger a registration retry delay timer in the UE. It is to note that this solution builds upon existing procedures in 3GPP TS 24.229 [3] (shown in subclause 6.1.1.2, existing content indicated in italics), which refer to IETF RFC 5626 [9]. Proposed additions on the IMS layer procedures are highlighted in subclause 6.1.1.2 with hard underline.

The procedure is executed in the following steps:

-	UE sends SIP REGISTER message, intended for registration or re-registration.

-	UE does not receive a SIP response to the SIP REGISTER from the first P-CSCF and thus sends a SIP REGISTER message to the next P-CSCF

- 	The UE does not receive a SIP response to the SIP REGISTER from any of its P-CSCFs.

-	After Timer F expires for SIP REGISTER sent to the last P-CSCF in the list of P-CSCFs, UE sets a retry delay timer

-	Only after the retry delay timer expires, the UE will trigger a new registration procedure.

NOTE:	CT4 will coordinate with CT1 to gather their feedback during the evaluation phase.

#### 6.1.1.2	P-CSCF failure/recovery detection

##### 6.1.1.2.1	P-CSCF failure detection

The detection of P-CSCF failure, or failure of all P-CSCFs configured to the UE is based on procedures that are based on the IMS SIP registration procedure.

Initial IMS registration in UE to be handled as follows:

On sending an unprotected REGISTER request….

When the timer F expires at the UE, the UE:

a)	shall mark the currently used P-CSCF address as unavailable for the last duration of the retry delay time computed by the algorithm defined in subclause 4.5 of RFC 5626 [92] plus 5 minutes;

b)	if there is a locally stored P-CSCF address as specified in subclause 5.1.9 which is different from the currently used P-CSCF address and which is not marked as unavailable, may initiate an initial registration as specified in subclause 5.1.1.2 using that P-CSCF; and

c)	if there is no locally stored P-CSCF address as specified in subclause 5.1.9 which is different from the currently used P-CSCF address and which is not marked as unavailable, may get a new set of P-CSCF-addresses as described in subclause 9.2.1 unless otherwise specified in the access specific annexes (as described in annex B, annex L or annex U) and initiate an initial registration as specified in subclause 5.1.1.2 after the amount of time of the last retry delay time computed by the algorithm defined in subclause 4.5 of RFC 5626.

NOTE 17:	It is an implementation option whether these actions are also triggered by other means than expiration of timer F, e.g. based on ICMP messages.

Expiration of timer F as described in italics above is part of 3GPP TS 24.229 [3]; it allows the UE to detect failure of all relevant P-CSCFs during initial IMS registration.

IMS re-registration in UE to be handled as follows:

On sending a REGISTER request that does not contain a challenge response …

When the timer F expires at the UE:

1)	the UE shall stop processing of all ongoing dialogs and transactions associated with that flow, if any (i.e. no further SIP signalling will be sent by the UE on behalf of these transactions or dialogs); and

2)	after releasing all IP-CAN bearers used for the transport of media according to the procedures in subclause 9.2.2:

a)	the UE may select a different P-CSCF address from the list of P-CSCF addresses discovered during the procedures described in subclause 9.2.1 or from its pre-configured list of P-CSCF's IP addresses or domain names;

b)	if no response has been received when attempting to contact all P-CSCFs known by the UE, the UE may get a new set of P-CSCF-addresses as described in subclause 9.2.1 unless otherwise specified in the access specific annexes (as described in annex B, annex L or annex U);

c)	the UE may perform the procedures for initial registration as described in subclause 5.1.1.2, and if the UE performs initial registration, it shall initiate the procedure after the amount of time of the last retry delay time computed by the algorithm defined in subclause 4.5 of RFC 5626; and

d)	the UE shall perform the procedures in RFC 5626 [92] to form a new flow to replace the failed one if it supports multiple registrations. If failed registration attempts occur in the process of creating a new flow, the UE shall implement the flow recovery procedures defined in subclause 4.5 of RFC 5626 [92] for determination of the retry delay time before each new registration attempt. The UE shall use the values of the parameters max-time and base-time, of the algorithm defined in subclause 4.5 of RFC 5626 [92]. If no values of the parameters max-time and base-time (if all failed) have been provided to the UE by the network, the default values defined in subclause 4.5 of RFC 5626 [92] shall be used.

Expiration of timer F as described in italics above is part of 3GPP TS 24.229 [3]; it allows the UE to detect failure of all relevant P-CSCFs during initial IMS re-registration.

##### 6.1.1.2.2	P-CSCF recovery detection and new registration

The detection recovery leverages the usage of the retry delay time which is calculated as described in clause 6.1.1.2.1. The UE detects P-CSCF recovery when it sends a new SIP REGISTER after expiry of the retry delay time and when it gets a (positive) SIP response to the SIP REGISTER.

### 6.1.2	Impacts on services, entities, and interfaces

Enhancements of IMS UE in case of all P-CSCF failure:

-	In case of initial IMS registration, introduce retry delay timer handling.

-	In case of IMS re-registration, introduce retry delay timer handling

### 6.1.3	Pros

The pros of the solution

-	The solution accommodates minimal functional enhancements to the IMS UE without extending the IMS architecture, and without any impact on the packet domain.

-	The solution supports the scenario in which all P-CSCF instances selected by the UE fail.

-	The solution is access independent, it can support 5G, 4G and all other accesses.

### 6.1.4	Cons

The cons of the solution:

-	The SIP Client of the UE needs to be modified, i.e. the solution relies on UE supporting the new functionality.

## 6.2	Solution #2: P-CSCF failure notification with PCO/ePCO

### 6.2.1	Description

#### 6.2.1.1	General

This solution is proposed to address Key Issue #1 "Preventing UE SIP Registration attempts during IMS Failures".

In KI#1, the IMS congestion scenarios were identified, and an analysis of the existing standard solutions found that during P-CSCF related failures, the UE was unable to receive any response, making it impossible to prevent the UE from repeatedly attempting the SIP Registration procedure with the P-CSCF.

To address the issue, this solution proposes notifying P-CSCF failure status of registered UE with PCO/ePCO.

The current PCO/ePCO shows only three IEs as defined in clause 10.5.6.3 of 3GPP TS 24.008 [10]:

-	0001H (P-CSCF IPv6 Address Request), 000CH (P-CSCF IPv4 Address Request): P-CSCF IP address. If there is no valid P-CSCF address, these IEs are not applicable.

-	0012H (P-CSCF Re-selection support): Inform the UE of the valid P-CSCF address. Inform the UE of the P-CSCF Re-selection support.

However, in the scenario where multiple P-CSCF instances fail and the P-CSCF instance selected by the UE is unavailable, the UE continues retrying SIP registration request until it finds an available P-CSCF instance.

It is desirable to introduce new IEs in PCO/ePCO to allow the 5GC to notify the UE about the suppression of IMS registration, based on the P-CSCF failure status.

The P-CSCF failure notification procedures are executed in the following steps:

-	P-CSCF failure/recovery detection

-	P-CSCF failure notification to UE

-	P-CSCF recovery notification to UE

-	Deadlock prevention

NOTE 1:	This solution involves impacts on NAS, which fall under the respective remits of CT1. Accordingly, CT4 will coordinate with CT1 to gather their feedback during the evaluation phase.

#### 6.2.1.2	P-CSCF failure/recovery detection

The following four options are assumed. It is possible to use a combination of these options, or to use just one according to operator policy.

##### 6.2.1.2.1	P-CSCF failure/recovery detection via PCF

The PCF detects the P-CSCF related failure event based on the status of the Rx interface and/or the supervision of multiple P-CSCFs and sends it to the SMF. When the Rx connection is lost and then recovered, the PCF can notify the SMF about the status of the P-CSCF, indicating whether it is down or up.

Figure 6.2.1.2.1-1: P-CSCF failure/recovery detection via PCF

##### 6.2.1.2.2	P-CSCF failure/recovery detection via NRF

The SMF subscribes to notifications at the NRF and when the NRF detects a change in availability/non-availability of the P-CSCF, it notifies the SMF as described in clause 6.2.6.1 of 3GPP TS 23.501 [5].

Figure 6.2.1.2.2-1: P-CSCF failure/recovery detection via NRF

##### 6.2.1.2.3	P-CSCF failure/recovery detection through heart-beat monitoring:

The SMF conducts heart-beat monitoring on the P-CSCF (e.g., ping, SIP message)

NOTE 2: The monitoring configuration of the SMF depends on operator and implementation policy.

Figure 6.2.1.2.3-1: P-CSCF failure/recovery detection through heart-beat monitoring

##### 6.2.1.2.4	P-CSCF failure/recovery detection by automated operator specific OAM operation

Automated operator specific OAM methods can be used to detect P-CSCF failure/recovery and lead to automated re-start of P-CSCF instances. Especially in cases when IMS nodes are deployed in cloud environments automated monitoring of their “health” status by means of the underlying cloud infrastructure (e.g., Kubernetes) allows for automated detecting and self-healing of error cases. In case of link errors in the transport network, OAM is the only central point in the network which can detect such failures and can distinguish between transport network and P-CSCF failures.

#### 6.2.1.3	P-CSCF failure notification to UE

##### 6.2.1.3.1	P-CSCF failure notification with PCO/ePCO

The following new IE is added to PCO/ePCO as described in clause 10.5.6.3 of 3GPP TS 24.008 [10].

-	Request Suppression Timer: The IE is used to notify that the P-CSCF is down and unavailable and set the time until the UE shall not initiate a SIP Registration Request. The IE is used to suppress SIP Registration from the UE for a certain period and prevent potential deadlock in the event that the recovery notification is dropped.

It is also necessary to clearly define the behaviour of the UE upon receiving this information. In the event of P-CSCF failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action should be postponed.

Figure 6.2.1.3.1-1: P-CSCF failure notification with PCO/ePCO

##### 6.2.1.3.2	Notifying P-CSCF failure notification feature with UE Capability Indication

It is necessary to define a new IE to notify whether the UE supports the feature or not. If the network sends the notification to the UE which does not support the option, the UE is expected to ignore the notification upon reception. However, depending on the implementation, if the UE does not ignore it, there is a possibility that the UE may misinterpret the content and may cause unexpected behavior. Thus, it may be safer to apply the P-CSCF failure notification feature based on the indication provided by the UE. A new IE is proposed for inclusion in PCE/ePCO as defined in clause 10.5.6.3 of 3GPP TS 24.008 [10]. This optional extension is based on the possibility for the SMF to identify whether the UE supports the P-CSCF status feature as described in clause 6.2.1.3.1 or not. The UE shall indicate this capability to the SMF at the activation of the IMS PDN connection in the following new IE in the PCO/ePCO. The SMF shall store this UE capability.

-	UE support P-CSCF failure notification indication: Inform the network about the P-CSCF status feature being supported by the UE.

Figure 6.2.1.3.2-1: Notifying P-CSCF failure notification feature with UE Capability Indication

##### 6.2.1.3.3	Procedures for P-CSCF failure notification to UE

The procedures for the P-CSCF failure notification as described in clause 6.2.1.3.1 and clause 6.2.1.3.2 are illustrated in figure 6.2.1.3.3-1.

Figure 6.2.1.3.3-1: Procedure for the P-CSCF failure notification with PCO/ePCO

1.	The UE sends a PDU Session Establishment Request towards the AMF, to establish a PDU session for IMS service. If the UE supports the P-CSCF status notification feature, the UE shall include a P-CSCF status indicator as described in clause 6.2.1.3.2 within the SM container.

2.	The AMF invokes Nsmf_PDUSession_CreateSMContext service operation to the SMF, to request the SMF to create a PDU session for IMS service.

3.	The SMF invokes Npcf_SMFPolicyControl_Get service operation to the PCF, to retrieve the session related policy.

4.	The SMF selects an appropriate UPF for IMS service and requests the UPF to establish an N4 session.

5.	When PDU session is created, the SMF invokes Namf_Communication_N1N2MessageTransfer service operation to the AMF, to notify a PDU session establishment request. The support of P-CSCF status indication as described in clause 6.2.1.3.1 is also included in the PCO/ePCO.

6.	The AMF sends the PDU Establishment Response message to the UE, with the support of P-CSCF status indication included in the PCO/ePCO.

7.	The UE performs an initial SIP registration towards the P-CSCF.

8.	With the methods described in clause 6.2.1.2, a failure in P-CSCF is detected.

9.	The SMF invokes Namf_Communication_N1N2MessageTransfer service operation, to trigger the PDU Session Modification procedure. Within the message, the SMF includes the Request Suppression Timer in the PCO/ePCO as described in clause 6.2.1.3.1.

NOTE 3:	The trigger for the initiation of the Namf_Communication_N1N2MessageTransfer service procedure may be determined by a random timer pre-configured within the SMF. The random timer configuration depends on operator and implementation policy.

NOTE 4:	The SMF is not expected to manage the timer states for each UE, such as tracking which values of the Request Suppression Timer have been provided to which UE.

10.	The AMF triggers the PDU Session Modification procedure to the UE which supports the P-CSCF status indication, to deliver the Request Suppression Timer to the UE.

11. The UE identifies that the P-CSCF is under failure condition. Then, the UE starts the timer according to the Suppression Timer in the PCO IE.

NOTE 5:	In the event of P-CSCF failure, the UE will try to prioritize the restoration of MMTEL service and attempt initial registration, but this action will be suppressed by the Request Suppression timer IE received in step 10.

NOTE 6:	If there is no response from the P-CSCF, the UE cannot determine whether the failure lies with the IMS or the 5GC and may repeatedly send SIP Registration requests. To avoid this, the UE does not send a SIP Registration request until the period specified by the Request Suppression Timer has elapsed. In this case, the UE does not perform IMS PDN Detach or bearer re-establishment such as those triggered by a Back-Off Timer or Retry-After; instead, the IMS PDN session is maintained.

12.	Once the timer is elapsed, the UE sends an initial SIP registration towards the P-CSCF. To prevent all UE from attempting to register at the same time, each UE shall add to or subtract from the given timer a random number of minutes.

#### 6.2.1.4	P-CSCF recovery notification to UE

This option reuses the P-CSCF restoration mechanism as defined in 3GPP TS 23.380[2], When the P-CSCF is recovered and available, the SMF informs the UE of the valid P-CSCF address. With the valid P-CSCF address, the UE initiates registration procedure to the new P-CSCF.

#### 6.2.1.5	Deadlock prevention

If the UE does not send a SIP Registration, several issues could arise such as delayed service recovery and increased unnecessary signalling. Following method can be used to overcome the situation.

##### 6.2.1.5.1	Re-establish the IMS PDN connection

The IMS PDN connection is re-established as defined in 3GPP TS 23.380 [2]. Then, the UE obtains the latest status of the P-CSCF and PCF.

### 6.2.2	Impacts on services, entities, and interfaces

The enhancements of SMF and UE in case of P-CSCF failure handling:

-	The current PCO/ePCO specified in 3GPP TS 24.008 [10] needs enhancements to include IEs that allow:

-	The SMF to notify the UE of Request Suppression Timer.

-	 The UE to notify UE capability for P-CSCF failure notification indication.

The enhancements of NF Service Consumer (e.g., SMF) and NF Service Provider (e.g., AMF) to notify the P-CSCF failure notification feature with UE Capability Indication.

The enhancements of NF Service Consumer (e.g., AMF) and NF Service Provider (e.g., SMF) to notify the P-CSCF failure notification:

-	The trigger for the initiation of the Namf_Communication_N1N2MessageTransfer service procedure needs enhancements by a random timer pre-configured within the SMF.

The solution relies on some existing OAM mechanism for P-CSCF failure detection.

### 6.2.3	Pros

The pros of the solution:

-	The solution accommodates minimal functional enhancements to the 5GC and UE without extending the existing IMS architecture.

-	The solution supports the scenario in which multiple P-CSCF instances fail, and the P-CSCF instance selected by the UE becomes unavailable. In such a case, the SMF can send a Request Suppression Timer to UE for the suppression of IMS registration based on the P-CSCF failure status.

-	The solution can further support deadlock prevention for the scenario in which the UE does not send a SIP Registration.

-	The use of PCO/ePCO allows integration with existing procedures and avoids the need for new signalling paths or protocols.

-	Timer-based control enables operator-defined recovery pacing, helping prevent signalling storms upon P-CSCF recovery.

### 6.2.4	Cons

The cons of the solution:

-	Requires extending IE in PCO/ePCO.

-	The SIP Client of the UE needs to be modified (e.g. SIP registration suppression).

-	This solution describes several methods to detect that the P-CSCF is down. If all methods are used, in combination, the determination of the P-CSCF failure might be reasonably accurate; however, if just one method is used, the conclusion might not be correct; for example, detecting a failure solely by monitoring the Rx interface (as described in clause 6.2.1.2.1) does not imply that IMS Registrations are failing, on one hand P-CSCF may not be down and on the other hand, SIP signalling may follow a different routing path.

-	A bulk failure notification procedure, as described in clause 6.2.1.3, implies sending individual notifications for all users currently registered in this P-CSCF. Depending on the P-CSCF recovery time, the signalling generated for all these notifications can be much higher than the number of retries of IMS Registrations while the P-CSCF is not yet recovered which might not affect all users registered in this P-CSCF.

## 6.3	Solution #3: SMF-initiated PDU session release

### 6.3.1	General

This solution is proposed to address Key Issue #1: "Preventing UE SIP Registration attempts during P-CSCF Failures." In KI#1, it was requested to identify how to prevent the UE from repeatedly attempting the SIP registration procedure with the P-CSCF in the case of P-CSCF failure.

This proposal suggests utilizing the network-requested PDU session release mechanism defined in 3GPP TS 24.501 [6] to release the PDU session used by the UE to send SIP signalling (e.g. PDU session to the IMS DNN) and thus prevent repeated UE SIP registration attempts. In this solution, P-CSCF failure is detected by the SMF according to the features defined in the existing specifications.

### 6.3.2	Description

#### 6.3.2.1	P-CSCF failure detection in SMF

According to the current specification, the SMF is capable of detecting P-CSCF failure, for example, by using the P-CSCF Restoration feature defined in the clause 5.8.3 of 3GPP TS 23.380 [2]. This clause also refers to the detection methods proposed in clause 6.2.1.2.1 and clause 6.2.1.2.2.

##### 6.3.2.1.1	P-CSCF failure detection by using P-CSCF Restoration feature

P-CSCF Failure Detection at SMF/UPF defined in the clause 5.8.3 of 3GPP TS 23.380 [2] can be used for P-CSCF failure detection. This mechanism is executed when an originating request cannot be served due to a P-CSCF failure, as long as there are no other registration flows for this originating UE using an available P-CSCF.

In this detection procedure, the PCF sends a request for P-CSCF restoration to the SMF for the corresponding PDU session as described in the clause 4.2.3.18 of 3GPP TS 29.512 [12].

##### 6.3.2.1.2	P-CSCF failure detection via PCF or NRF

As introduced in clause 6.2.1.2.1 (P-CSCF failure/recovery detection via PCF) and clause 6.2.1.2.2 (P-CSCF failure/recovery detection via NRF), SMF can be notified of a P-CSCF failure by PCF or NRF.

#### 6.3.2.2	SMF-initiated PDU session release

The purpose of the network-requested PDU session release procedure defined in 3GPP TS 24.501 [6] is to enable the network to release a PDU session. The SMF initiates this procedure by creating a PDU SESSION RELEASE COMMAND message and setting the 5GSM cause IE to indicate the reason for releasing the session.

In this proposal, the SMF releases the PDU session used by the UE to send SIP signalling, sets the 5GSM cause IE in the PDU SESSION RELEASE COMMAND to #67: "Insufficient resources for specific slice and DNN" to prevent UE SIP registration attempts during P-CSCF failures and includes a Back-off timer IE in the PDU SESSION RELEASE COMMAND message indicating for how long the UE shall not re-try establishing a PDU session to send SIP signalling .

When the UE receives a PDU SESSION RELEASE COMMAND with cause #67 and the Back-off timer IE, it interprets this as a network-side rejection, indicating that the requested S-NSSAI and DNN combination cannot be supported due to insufficient resources. Consequently, the UE will terminate the affected PDU session as instructed by the network, will start the back-off timer and will not re-try establishing the PDU session to send SIP signalling until the back-off timer expires. Once the back-off timer expires, the UE may send another PDU SESSION ESTABLISHMENT REQUEST message or PDU SESSION MODIFICATION REQUEST message for the same DNN. This ensures that SIP signaling does not resume prematurely during network recovery.

Given that releasing the PDU session to send SIP signalling with a back-off timer will cause voice-centric UEs to disable N1 mode and possibly fall back to EPS to obtain voice services, existing mechanisms such as Access Control and NAS-level mobility management congestion control should be proactively applied to prevent network congestion.

### 6.3.3	Impacts on services, entities, and interfaces

SMF:

-	Explicit description for detecting the P-CSCF failure

### 6.3.4	Pros

The pros of the solution:

-	The solution avoids any impact on the AMF and IMS network.

-	The solution leverages existing features for P-CSCF failure detection.

-	The solution is compatible with legacy UEs, thus offering a practical and backward-compatible alternative.

### 6.3.5	Cons

Editor’s note: To be added.

## 6.4	Solution #4: Access restriction with UAC Access Category 9

### 6.4.1	Description

This solution is proposed to address Key Issue #1: "Preventing UE SIP Registration attempts during P-CSCF Failures". In KI#1, UEs continue to initiate the SIP registration request when the IMS is unable to respond, resulting in sustained traffic congestion. Therefore, it is required that the solution prevents repeated registration attempts by UEs under total P-CSCF failure conditions.

To address the issue, this proposal solution proposes delegating by using Unified Access Control (UAC, defined in 3GPP TS 24.501 [6]. UAC has the access categories and access identifiers to control the restriction of UE accessing more flexible.

For this KI#1, this solution uses the access category 9 which is reserved for MO IMS registration related signaling and defined as "Includes IMS registration related signaling, e.g. IMS initial registration, re-registration, and subscription refresh". Access control may be applied to communications from UEs using access category 9, and the barring rate may be gradually relaxed over time to avoid concentrated load.

Since KI#1 indicates the situation of a severe IMS failure, i.e. all P-CSCF instance fail and it triggers the total IMS service down, affecting for whole PLMN is allowed. In addition, since access category 9 targets only IMS registration-related signalling, barring can be applied without impacting other types of communications.

#### 6.4.1.1	Manual recovery

The manual recovery flow of P-CSCF failure is based on the regular operation flow of UAC barring.

-	Upon detection of a P-CSCF failure, the operator initiates UAC barring for UEs of access category 9.

-	UAC barring information is immediately broadcast to all UEs via SIB1.

-	Based on the configured barring rate, IMS initial registration, re-registration, and subscription refresh requests from affected UEs are suppressed prior to transmission. Affected UEs cannot retry until the barring timer is ended.

-	When the situation changes, e.g. P-CSCF recovery, the operator updates the barring rate to gradually increase the number of UEs permitted to access IMS services.

Since UAC restricts on SIB1, ongoing transactions	are not affected and continue / retry to connect with established NAS session.

#### 6.4.1.2	Automatic recovery

The operator may automate UAC recovery procedures based on operator’s policy. The detection of P-CSCF failure may be performed based on KPI data, for example, alarms, increased failure rates of IMS traffic or status monitor on surrounding node e.g. PCRF/S-CSCF/PGW/UPF.

### 6.4.2	Impacts on services, entities, and interfaces

No impact.

### 6.4.3	Pros

The pros of the solution:

-	This solution only consists of the existing features.

-	IMS registration is immediately restricted once UAC barring is configured to avoid burst load, based on operator configuration.

-	The solution is compatible with legacy UEs.

### 6.4.4	Cons

The cons of the solution:

-	Retried SIP requests from IMS registered UE cannot be prevented, resulting in remaining load.

-	In case of roaming and network sharing including MVNO scenario, SLA or OAM level agreement may be needed.

# 7	Evaluations

## 7.1	Evaluation of Solutions for Key Issue#1

A group of Solutions have been proposed for Key Issue#1 to address to prevent UE SIP Registration attempts during IMS Failures.

Solution #1 proposes leveraging existing timeout mechanism during SIP Registration procedure to trigger a registration retry delay timer in the UE. Specifically, when Timer F expires for all P-CSCFs that are in the list of P-CSCFs for a UE, the retry delay timer is activated. This delay is computed using the algorithm specified in subclause 4.5 of RFC 5626 [9]. This solution builds upon existing procedures in 3GPP TS 24.229 [3]. Recovery detection similarly relies on the retry delay mechanism. The UE detects P-CSCF recovery when it sends a new SIP REGISTER after the expiry of the retry delay time and receives a positive SIP response.

Solution #2 proposes notifying P-CSCF failure status of registered UE with PCO/ePCO. To detect P-CSCF failure and recovery, several options are introduced, leveraging either existing standardized mechanisms or implementation-specific approaches. For P-CSCF failure notification, a new IE, Request Suppression Timer is introduced within PCO/ePCO. This IE indicates that the P-CSCF is unavailable and specifies the duration during which the UE should refrain from sending SIP Registration requests. During this period, the UE does not perform IMS PDN Detach or bearer re-establishment procedures such as those triggered by a Back-Off Timer or Retry-After header. Instead, the IMS PDN session is maintained. Additionally, the solution proposes defining a new IE during PDU Session Establishment to indicate whether the UE supports this feature. For P-CSCF recovery notification, the solution reuses the P-CSCF restoration mechanism as defined in 3GPP TS 23.380 [2].

Solution #3 proposes utilizing the network-requested PDU session release mechanism defined in 3GPP TS 24.501 [6] to release the PDU session used by the UE to send SIP signalling (e.g. PDU session to the IMS DNN) and thus preventing repeated SIP registration attempts by the UE. P-CSCF failure is detected e.g., by the SMF according to the features defined in the existing specifications such as P-CSCF Failure Detection at SMF/UPF defined in the clause 5.8.3 of 3GPP TS23.380 [2]. Upon detection, the SMF initiates the PDU session release used for SIP signalling and sets the 5GSM cause IE in the PDU SESSION RELEASE COMMAND to cause value #67. This prevents the UE from attempting SIP registration during the P-CSCF failure period. Additionally, the SMF includes a Back-off Timer IE in the PDU SESSION RELEASE COMMAND message, indicating the duration for which the UE shall refrain from re-establishing a PDU session for SIP signalling.

Solution #4 proposes leveraging UAC with access category 9 (MO IMS registration) as defined in 3GPP TS 24.501 [6]. By applying access control to UEs using this category, the barring rate can be gradually relaxed to prevent traffic surges. Since the access category 9 is limited to IMS registration signaling, this approach enables targeted barring without affecting other communication services.

The solutions are presented with pro and con in the Table 7.1-1.

Table 7.1-1 Comparison table of Solutions

|  |  | Solution#1 | Solution#2 | Solution#3 | Solution#4 |
| --- | --- | --- | --- | --- | --- |
| Impact | Pro | No impacts to IMS Network. No impacts to the packet domain. | No impacts to IMS Network | No impact to IMS Network. Works with legacy UEs. | No impact to IMS Network and SIP Client of the UE. Works with legacy UEs. |
|  | Con | The SIP Client of the UE needs to be modified, i.e. the solution relies on UE supporting the new functionality. | The SIP Client of the UE needs to be modified i.e. SIP registration suppression. Minimal functional enhancements to the 5GC (SMF, AMF) such as extending IEs in PCO/ePCO. Relying on a single detection method, such as Rx interface monitoring, might lead to inaccurate determination of P-CSCF failure. A bulk failure notification procedure implies sending individual notifications for all users currently registered in the P-CSCF, | T.B.D. | Retried SIP requests from IMS registered UE cannot be prevented, resulting in remaining load. In case of roaming and network sharing including MVNO scenario, SLA or OAM level agreement may be needed. |
| Access dependency |  | Access independent, it can support 5G, 4G and all other accesses. | Covers 5G | Covers 5G | Covers 5G |

Editor's note:	This clause is subject to revision in response to input from SA2, CT1 and CT3.

# 8	Conclusions

This clause describes conclusions.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-06 | CT4#129 | C4-252362 |  |  |  | TS skeleton Implementation of pCRs agreed at CT4#129. | 0.1.0 |
| 2025-08 | CT4#130 | C4-253416 |  |  |  | Pseudo-CR on KI#1 definition | 0.2.0 |
| 2025-08 | CT4#130 | C4-253417 |  |  |  | Pseudo-CR on solution for P-CSCF failure detection based on IMS means | 0.2.0 |
| 2025-08 | CT4#130 | C4-253512 |  |  |  | Pseudo-CR on solution for P-CSCF failure notification with PCO/ePCO | 0.2.0 |
| 2025-08 | CT4#130 | C4-253513 |  |  |  | Pseudo-CR on solution for SMF-initiated PDU session release | 0.2.0 |
| 2025-10 | CT4#131 | C4-254037 |  |  |  | Pseudo-CR on Solution #1 clarification | 0.3.0 |
| 2025-10 | CT4#131 | C4-254395 |  |  |  | Pseudo-CR on P-CSCF Failure recovery with UAC Access Category 9 | 0.3.0 |
| 2025-10 | CT4#131 | C4-254396 |  |  |  | Pseudo-CR on Evaluation of Solutions for Key Issue#1 | 0.3.0 |
| 2025-10 | CT4#131 | C4-254257 |  |  |  | Pseudo-CR on Solution #2 clarification | 0.3.0 |
| 2025-10 | CT4#131 | C4-254397 |  |  |  | Pseudo-CR on update of solution#3 | 0.3.0 |
| 2025-10 | CT4#131 | C4-254258 |  |  |  | Pseudo-CR on Rel-19 Solution #2 drawbacks | 0.3.0 |