| 3GPP TS 31.124 V15.7.0 (2020-03) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Mobile Equipment (ME) conformance test specification; Universal Subscriber Identity Module Application Toolkit (USAT) conformance test specification (Release 15) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	11

1	Scope	12

2	References	13

3	Definitions and abbreviations	15

3.1	Mobile station definition and configurations	15

3.2	Applicability	15

3.2.1	Applicability of the present document	15

3.2.2	Applicability of the individual tests	15

3.2.3	Applicability to terminal equipment	15

3.2.4	Definitions	16

3.2.4.1	Format of the table of optional features	16

3.2.4.2	Format of the applicability table	16

3.2.4.3	Status and notations	16

3.3	Table of optional features	17

3.4	Applicability table	27

27.22.4.27.8 Open Channel (related to NG-RAN)	93

3.5	Conventions for mathematical notations	106

3.5.1	Mathematical signs	106

4	Test equipment	106

5	Testing methodology in general	106

5.1	Testing of optional functions and procedures	106

5.2	Test interfaces and facilities	106

5.3	Information to be provided by the apparatus supplier	106

6	Implicit testing	107

7	Measurement uncertainty	107

8	Format of tests	107

9	Generic call set up procedures	110

10 - 26	Not used	110

27	Testing of the UICC/ME interface	110

27.0	Introduction	110

27.1 - 27.21	Void	110

27.22	USIM Application Toolkit	111

27.22.1A	General Test purpose	111

27.22.2A	Definition of default values for USIM Application Toolkit testing	111

27.22.2B	Definition of default values for LTE related USIM Application Toolkit testing	115

27.22.2B.1	Definition of E-UTRAN/EPC UICC	115

27.22.2B.2	Definition of E-UTRAN parameters	117

27.22.2C	Definition of E-UTRAN/EPC ISIM-UICC	117

27.22.2C.1	Applications on the E-UTRAN/EPC ISIM-UICC	117

27.22.2C.2	Default USIM values of E-UTRAN/EPC ISIM-UICC	117

27.22.2C.3	Default ISIM values of E-UTRAN/EPC ISIM-UICC	117

27.22.2C.3.1	EFAD (Administrative Data)	117

27.22.2C.3.2	EFIST (ISIM Service Table)	117

27.22.2C.3.3	EFIMPI (IMS private user identity)	118

27.22.2C.3.4	EFDOMAIN (Home Network Domain Name)	118

27.22.2C.3.5	EFIMPU (IMS public user identity)	118

27.22.2C.3.6	EFP-CSCF (P-CSCF ADDRESS)	119

27.22.2C.3.7 EFSMS (Short Message Service)	119

27.22.2C.3.8	EFSMSR (Short message status reports)	120

27.22.2C.3.9	EFSMSP (Short message service parameters)	120

27.22.2C.3.10	EFSMSS (SMS Status)	120

27.22.2C.4	Default values at DF_TELECOM	121

27.22.2C.4.1	EFPSISMSC (Public Service Identity of the SM-SC)	121

27.22.2D	Definition of default values for NG-RAN related USIM Application Toolkit testing	121

27.22.2D.1	Definition of NG-RAN UICC	121

27.22.2D.2	Definition of NG-RAN cell parameters	122

27.22.1	Initialization of USIM Application Toolkit Enabled UICC by USIM Application Toolkit Enabled ME (Profile Download)	123

27.22.1.1	Definition and applicability	123

27.22.1.2	Conformance requirement	123

27.22.1.3	Test purpose	123

27.22.1.4	Method of test	123

27.22.1.4.1	Initial conditions	123

27.22.1.4.2	Procedure	123

27.22.1.5	Test requirement	124

27.22.2	Contents of the TERMINAL PROFILE command	124

27.22.2.1	Definition and applicability	124

27.22.2.2	Conformance requirement	124

27.22.2.3	Test purpose	124

27.22.2.4	Method of test	124

27.22.2.4.1	Initial conditions	124

27.22.1.4.2	Procedure	124

27.22.2.5	Test requirement	124

27.22.3	Servicing of proactive UICC commands	125

27.22.3.1	Definition and applicability	125

27.22.3.2	Conformance requirement	125

27.22.3.3	Test purpose	125

27.22.3.4	Method of test	125

27.22.3.4.1	Initial conditions	125

27.22.3.4.2	Procedure	125

27.22.3.5	Test requirement	125

27.22.4	Proactive UICC commands	126

27.22.4.1	DISPLAY TEXT	126

27.22.4.1.1	DISPLAY TEXT (Normal)	126

27.22.4.1.2	DISPLAY TEXT (Support of "No response from user")	127

27.22.4.1.3	DISPLAY TEXT (Display of extension text)	128

27.22.4.1.4	DISPLAY TEXT (Sustained text)	128

27.22.4.1.5	DISPLAY TEXT (Display of icons)	130

27.22.4.1.6	DISPLAY TEXT (UCS2 display in Cyrillic)	131

27.22.4.1.7	DISPLAY TEXT (Variable Time out)	132

27.22.4.1.8	DISPLAY TEXT (Support of Text Attribute)	132

27.22.4.1.9	DISPLAY TEXT (UCS2 display in Chinese)	139

27.22.4.1.10	DISPLAY TEXT (UCS2 display in Katakana)	140

27.22.4.2	GET INKEY	141

27.22.4.2.1	GET INKEY(normal)	141

27.22.4.2.2	GET INKEY (No response from User)	142

27.22.4.2.3	GET INKEY (UCS2 display in Cyrillic)	143

27.22.4.2.4	GET INKEY (UCS2 entry in Cyrillic)	144

27.22.4.2.5	GET INKEY ("Yes/No" Response)	144

27.22.4.2.6	GET INKEY (display of Icon)	145

27.22.4.2.7	GET INKEY (Help Information)	146

27.22.4.2.8	GET INKEY (Variable Time out)	147

27.22.4.2.9	GET INKEY (Support of Text Attribute)	147

27.22.4.2.10	GET INKEY (UCS2 display in Chinese)	154

27.22.4.2.11	GET INKEY (UCS2 entry in Chinese)	155

27.22.4.2.12	GET INKEY (UCS2 display in Katakana)	155

27.22.4.2.13	GET INKEY (UCS2 entry in Katakana)	156

27.22.4.3	GET INPUT	157

27.22.4.3.1	GET INPUT (normal)	157

27.22.4.3.2	GET INPUT (No response from User)	158

27.22.4.3.3	GET INPUT (UCS2 display in Cyrillic)	159

27.22.4.3.4	GET INPUT (UCS2 entry in Cyrillic)	160

27.22.4.3.5	GET INPUT (default text)	161

27.22.4.3.6	GET INPUT (display of Icon)	161

27.22.4.3.7	GET INPUT (Help Information)	162

27.22.4.3.8	GET INPUT (Support of Text Attribute)	163

27.22.4.3.9	GET INPUT (UCS2 display in Chinese)	170

27.22.4.3.10	GET INPUT (UCS2  entry in Chinese)	170

27.22.4.3.11	GET INPUT (UCS2 display in Katakana)	171

27.22.4.3.12	GET INPUT (UCS2  entry in Katakana)	172

27.22.4.4	MORE TIME	173

27.22.4.4.2	Conformance requirement	173

27.22.4.4.3	Test purpose	173

27.22.4.4.4	Method of test	173

27.22.4.4.5	Test requirement	173

27.22.4.5	PLAY TONE	173

27.22.4.5.1	PLAY TONE (Normal)	173

27.22.4.5.2	PLAY TONE (UCS2 display in Cyrillic)	179

27.22.4.5.3	PLAY TONE (display of Icon)	180

27.22.4.5.4	PLAY TONE (Support of Text Attribute)	181

27.22.4.5.5	PLAY TONE (UCS2 display in Chinese)	188

27.22.4.5.6	PLAY TONE (UCS2 display in Katakana)	189

27.22.4.6	POLL INTERVAL	189

27.22.4.6.1	Definition and applicability	189

27.22.4.6.2	Conformance requirement	189

27.22.4.6.3	Test purpose	190

27.22.4.6.4	Method of test	190

27.22.4.7	REFRESH	190

27.22.4.7.1	REFRESH (normal)	190

27.22.4.7.2	REFRESH (IMSI changing procedure)	200

27.22.4.7.3	REFRESH (Steering of roaming)	212

27.22.4.7.4	REFRESH (AID)	230

27.22.4.7.5	REFRESH (IMSI changing procedure, E-UTRAN)	233

27.22.4.8	SET UP MENU and ENVELOPE MENU SELECTION	238

27.22.4.8.1	SET UP MENU (normal) and ENVELOPE MENU SELECTION	238

27.22.4.8.2	SET UP MENU (help request support) and ENVELOPE MENU SELECTION	239

27.22.4.8.3	SET UP MENU (next action support) and ENVELOPE MENU SELECTION	240

27.22.4.8.4	SET UP MENU (display of icons) and ENVELOPE MENU SELECTION	241

27.22.4.8.5	SET UP MENU (soft keys support) and ENVELOPE MENU SELECTION	242

27.22.4.8.6	SET UP MENU (support of Text Attribute) and ENVELOPE MENU SELECTION	243

27.22.4.8.7	SET UP MENU (UCS2 display in Cyrillic) and ENVELOPE MENU SELECTION	250

27.22.4.8.8	SET UP MENU (UCS2 display in Chinese) and ENVELOPE MENU SELECTION	251

27.22.4.8.9	SET UP MENU (UCS2 display in Katakana) and ENVELOPE MENU SELECTION	252

27.22.4.9	SELECT ITEM	253

27.22.4.9.1	SELECT ITEM (mandatory features for ME supporting SELECT ITEM)	253

27.22.4.9.2	SELECT ITEM (next action support)	254

27.22.4.9.3	SELECT ITEM (default item support)	255

27.22.4.9.4	SELECT ITEM (help request support)	255

27.22.4.9.5	SELECT ITEM (icons support)	256

27.22.4.9.6	SELECT ITEM (presentation style)	257

27.22.4.9.7	SELECT ITEM (soft keys support)	258

27.22.4.9.8	SELECT ITEM (Support of "No response from user")	258

27.22.4.9.9	SELECT ITEM (Support of Text Attribute)	259

27.22.4.9.10	SELECT ITEM (UCS2 display in Cyrillic)	265

27.22.4.9.11	SELECT ITEM (UCS2 display in Chinese)	266

27.22.4.9.12	SELECT ITEM (UCS2 display in Katakana)	267

27.22.4.10	SEND SHORT MESSAGE	268

27.22.4.10.1	SEND SHORT MESSAGE (normal)	268

27.22.4.10.2	SEND SHORT MESSAGE (UCS2 display in Cyrillic)	269

27.22.4.10.3	SEND SHORT MESSAGE (icon support)	275

27.22.4.10.4	SEND SHORT MESSAGE (Support of Text Attribute)	281

27.22.4.10.5	SEND SHORT MESSAGE (UCS2 display in Chinese)	319

27.22.4.10.6	SEND SHORT MESSAGE (UCS2 display in Katakana)	324

27.22.4.10.7	SEND SHORT MESSAGE (IMS)	329

27.22.4.10.8	SEND SHORT MESSAGE (over SGs in E-UTRAN)	341

27.22.4.11	SEND SS	342

27.22.4.11.1	SEND SS (normal)	342

27.22.4.11.2	SEND SS (Icon support)	354

27.22.4.11.3	SEND SS (UCS2 display in Cyrillic)	360

27.22.4.11.4	SEND SS (support of Text Attribute)	362

27.22.4.11.5	SEND SS (UCS2 display in Chinese)	396

27.22.4.11.6	SEND SS (UCS2 display in Katakana)	398

27.22.4.12	SEND USSD	400

27.22.4.12.1	SEND USSD (normal)	400

27.22.4.12.2	SEND USSD (Icon support)	410

27.22.4.12.3	SEND USSD (UCS2 display in Cyrillic)	416

27.22.4.12.4	SEND USSD (support of Text Attribute)	418

27.22.4.12.5	SEND USSD (UCS2 display in Chinese)	452

27.22.4.12.6	SEND USSD (UCS2 display in Katakana)	454

27.22.4.13	SET UP CALL	457

27.22.4.13.1	SET UP CALL (normal)	457

27.22.4.13.2	SET UP CALL (second alpha identifier)	469

27.22.4.13.3	SET UP CALL (display of icons)	471

27.22.4.13.4	SET UP CALL (support of Text Attribute)	480

27.22.4.13.5	SET UP CALL (UCS2 Display in Cyrillic)	522

27.22.4.13.6	SET UP CALL (UCS2 Display in Chinese)	525

27.22.4.13.7	SET UP CALL (UCS2 Display in Katakana)	528

27.22.4.14	POLLING OFF	531

27.22.4.14.1	Definition and applicability	531

27.22.4.14.2	Conformance requirement	531

27.22.4.14.3	Test purpose	531

27.22.4.14.4	Method of test	531

27.22.4.14.5	Test requirement	534

27.22.4.15	PROVIDE LOCAL INFORMATION	534

27.22.4.15.1	Definition and applicability	534

27.22.4.15.2	Conformance requirement	534

27.22.4.15.3	Test purpose	534

27.22.4.15.4	Method of tests	535

27.22.4.15.5	Test requirement	555

27.22.4.16	SET UP EVENT LIST	556

27.22.4.16.1	SET UP EVENT LIST (normal)	556

27.22.4.17	PERFORM CARD APDU	564

27.22.4.17.1	PERFORM CARD APDU (normal)	564

27.22.4.17.2	PERFORM CARD APDU (detachable card reader)	565

27.22.4.18	POWER OFF CARD	566

27.22.4.18.1	POWER OFF CARD (normal)	566

27.22.4.18.2	POWER OFF CARD (detachable card reader)	567

27.22.4.19	POWER ON CARD	568

27.22.4.19.1	POWER ON CARD (normal)	568

27.22.4.19.2	POWER ON CARD (detachable card reader)	569

27.22.4.20	GET READER STATUS	569

27.22.4.20.1	GET READER STATUS (normal)	569

27.22.4.20.2	GET CARD READER STATUS (detachable card reader)	570

27.22.4.21	TIMER MANAGEMENT and ENVELOPE TIMER EXPIRATION	571

27.22.4.21.1	TIMER MANAGEMENT (normal)	571

27.22.4.21.2	ENVELOPE TIMER EXPIRATION (normal)	572

27.22.4.22	SET UP IDLE MODE TEXT	573

27.22.4.22.1	SET UP IDLE MODE TEXT (normal)	573

27.22.4.22.2	SET UP IDLE MODE TEXT (Icon support)	578

27.22.4.22.3	SET UP IDLE MODE TEXT (UCS2 support)	579

27.22.4.22.4	SET UP IDLE MODE TEXT (support of Text Attribute)	580

27.22.4.22.5	SET UP IDLE MODE TEXT (UCS2 display in Chinese)	586

27.22.4.22.6	SET UP IDLE MODE TEXT (UCS2 display in Katakana)	587

27.22.4.23	RUN AT COMMAND	588

27.22.4.23.1	RUN AT COMMAND (normal)	588

27.22.4.23.2	RUN AT COMMAND (Icon support)	590

27.22.4.23.3	RUN AT COMMAND (support of Text Attribute)	597

27.22.4.23.4	RUN AT COMMAND (UCS2 display in Cyrillic)	631

27.22.4.23.5	RUN AT COMMAND (UCS2 display in Chinese)	631

27.22.4.23.6	RUN AT COMMAND (UCS2 display in Katakana)	632

27.22.4.24	SEND DTMF	633

27.22.4.24.1	SEND DTMF (Normal)	633

27.22.4.24.2	SEND DTMF (Display of icons)	638

27.22.4.24.3	SEND DTMF (UCS2 display in Cyrillic)	643

27.22.4.24.4	SEND DTMF (support of Text Attribute)	645

27.22.4.24.5	SEND DTMF (UCS2 Display in Chinese)	693

27.22.4.24.6	SEND DTMF (UCS2 Display in Katakana)	695

27.22.4.25	LANGUAGE NOTIFICATION	696

27.22.4.25.1	Definition and applicability	696

27.22.4.25.2	Conformance Requirement	696

27.22.4.25.3	Test purpose	696

27.22.4.25.4	Method of Test	697

27.22.4.25.5	Test requirement	697

27.22.4.26	LAUNCH BROWSER	697

27.22.4.26.1	LAUNCH BROWSER (No session already launched)	697

27.22.4.26.2	LAUNCH BROWSER (Interaction with current session)	704

27.22.4.26.3	LAUNCH BROWSER (UCS2 display in Cyrillic)	708

27.22.4.26.4	LAUNCH BROWSER (icons support)	711

27.22.4.26.5	LAUNCH BROWSER (support of Text Attribute)	716

27.22.4.26.6	LAUNCH BROWSER (UCS2 Display in Chinese)	758

27.22.4.26.7	LAUNCH BROWSER (UCS2 Display in Katakana)	760

27.22.4.27	OPEN CHANNEL	762

27.22.4.27.1	Void	762

27.22.4.27.2	Open Channel (related to GPRS)	762

27.22.4.27.3	Open Channel (default bearer)	779

27.22.4.27.3.1	Open Channel (default bearer, E-UTRAN)	779

27.22.4.27.3.2	Open Channel (Default bearer, GERAN/UTRAN)	779

27.22.4.27.4	Open Channel (Local Bearer)	782

27.22.4.27.5	Open Channel (GPRS, support of Text Attribute)	782

27.22.4.27.6	Open Channel (related to E-UTRAN)	842

27.22.4.27.7	Open Channel (UICC Access to IMS)	864

27.22.4.27.8	Open Channel (related to NG-RAN)	870

27.22.4.28	CLOSE CHANNEL	875

27.22.4.28.1	CLOSE CHANNEL(normal)	875

27.22.4.28.2	CLOSE CHANNEL (support of Text Attribute)	882

27.22.4.28.3	CLOSE CHANNEL(E-UTRAN/EPC)	924

27.22.4.29	RECEIVE DATA	932

27.22.4.29.1	RECEIVE DATA (NORMAL)	932

27.22.4.29.1.5	Test requirement	951

27.22.4.29.2	RECEIVE DATA (support of Text Attribute)	951

27.22.4.30	SEND DATA	990

27.22.4.30.1	SEND DATA (normal)	990

27.22.4.30.2	SEND DATA (support of Text Attribute)	1005

27.22.4.30.3	SEND DATA (E-UTRAN)	1036

27.22.4.31	GET CHANNEL STATUS	1045

27.22.4.31.1	Definition and applicability	1045

27.22.4.31.2	Conformance requirements	1045

27.22.4.31.3	Test purpose	1045

27.22.4.31.4	Method of test	1045

27.22.4.31.5	Test requirement	1060

27.22.5	Data Download to UICC	1060

27.22.5.1	SMS-PP Data Download	1060

27.22.5.1.1	Definition and applicability	1060

27.22.5.1.2	Conformance requirement	1060

27.22.5.1.3	Test purpose	1060

27.22.5.1.4	Method of Test	1061

27.22.5.1.5	Test requirement	1062

27.22.5.2	Cell Broadcast Data Download	1062

27.22.5.2.1	Definition and applicability	1062

27.22.5.2.2	Conformance requirement	1062

27.22.5.2.3	Test purpose	1062

27.22.5.2.4	Method of Test	1062

27.22.5.2.5	Test requirement	1067

27.22.5.3	SMS-PP Data Download over IMS	1067

27.22.5.3.1	Definition and applicability	1067

27.22.5.3.2	Conformance requirement	1067

27.22.5.3.3	Test purpose	1068

27.22.5.3.4	Method of Test	1068

27.22.5.3.5	Test requirement	1077

27.22.5.4	SMS-PP Data Download over SGs in E-UTRAN	1077

27.22.5.4.1	Definition and applicability	1077

27.22.5.4.2	Conformance requirement	1077

27.22.5.4.3	Test purpose	1077

27.22.5.4.4	Method of Test	1078

27.22.5.4.5	Test requirement	1078

27.22.6	CALL CONTROL BY USIM	1078

27.22.6.1	Procedure for Mobile Originated calls	1078

27.22.6.1.1	Definition and applicability	1078

27.22.6.1.2	Conformance requirement	1078

27.22.6.1.3	Test purpose	1078

27.22.6.1.4	Method of tests	1079

27.22.6.1.5	Test requirement	1099

27.22.6.2	Procedure for Supplementary (SS) Services	1099

27.22.6.2.1	Definition and applicability	1099

27.22.6.2.2	Conformance requirement	1100

27.22.6.2.3	Test purpose	1100

27.22.6.2.4	Method of tests	1100

27.22.6.2.5	Test requirement	1107

27.22.6.3	Interaction with Fixed Dialling Number (FDN)	1107

27.22.6.3.1	Definition and applicability	1107

27.22.6.3.2	Conformance requirement	1107

27.22.6.3.3	Test purpose	1107

27.22.6.3.4	Method of tests	1108

27.22.6.3.5	Test requirement	1114

27.22.6.4	Support of Barred Dialling Number (BDN) service	1114

27.22.6.4.1	Definition and applicability	1114

27.22.6.4.2	Conformance requirement	1114

27.22.6.4.3	Test purpose	1115

27.22.6.4.4	Method of tests	1115

27.22.6.4.5	Test requirement	1125

27.22.6.5	Barred Dialling Number (BDN) service handling for terminals not supporting BDN	1126

27.22.6.5.1	Definition and applicability	1126

27.22.6.5.2	Conformance requirement	1126

27.22.6.5.3	Test purpose	1126

27.22.6.5.4	Method of tests	1126

27.22.7	EVENT DOWNLOAD	1127

27.22.7.1	MT Call Event	1127

27.22.7.1.1	MT Call Event (normal)	1127

27.22.7.2	Call Connected Event	1129

27.22.7.2.1	Call Connected Event (MT and MO call)	1129

27.22.7.2.2	Call Connected Event (ME supporting SET UP CALL)	1136

27.22.7.3	Call Disconnected Event	1139

27.22.7.3.1	Call Disconnected Event	1139

27.22.7.4	Location Status Event	1143

27.22.7.4.1	Location Status Event (normal)	1143

27.22.7.4.1.4.2	Procedure	1145

27.22.7.5	User Activity Event	1152

27.22.7.5.1	User Activity Event (normal)	1152

27.22.7.6	Idle screen available event	1153

27.22.7.6.1	Idle Screen Available (normal)	1153

27.22.7.7	Card reader status event	1154

27.22.7.7.1	Card Reader Status (normal)	1154

27.22.7.7.2	Card Reader Status(detachable card reader)	1154

27.22.7.8	Language selection event	1155

27.22.7.8.1	Language selection event (normal)	1155

27.22.7.9	Browser termination event	1156

27.22.7.9.1	Browser termination (normal)	1156

27.22.7.10	Data available event	1158

27.22.7.10.1	Definition and applicability	1158

27.22.7.10.2	Conformance requirements	1158

27.22.7.10.3	Test purpose	1158

27.22.7.10.4	Method of test	1159

27.22.7.11	Channel Status event	1172

27.22.7.11.1	Definition and applicability	1172

27.22.7.11.2	Conformance requirements	1172

27.22.7.11.3	Test purpose	1172

27.22.7.11.4	Method of test	1172

27.22.7.12	Access Technology Change event	1179

27.22.7.13	Display parameter changed event	1183

27.22.7.14	Local Connection event	1183

27.22.7.15	Network search mode change event	1184

27.22.7.15.1	Definition and applicability	1184

27.22.7.15.2	Conformance requirements	1184

27.22.7.15.3	Test purpose	1184

27.22.7.15.4	Method of test	1184

27.22.7.16	Browsing status event	1186

27.22.7.17	Network Rejection Event	1186

27.22.7.18	CSG Cell Selection event	1192

27.22.7.18.1	CSG Cell Selection (normal)	1192

27.22.7.19	IMS registration event	1198

27.22.7.20	Incoming IMS data event	1198

27.22.7.20.1	Incoming IMS data (normal)	1198

27.22.7.21	Data Connection Status Change event	1203

27.22.8	MO SHORT MESSAGE CONTROL BY USIM	1206

27.22.8.1	Definition and applicability	1206

27.22.8.2	Conformance requirement	1206

27.22.8.3	Test purpose	1206

27.22.8.4	Method of tests	1207

27.22.8.4.1	Initial conditions	1207

27.22.8.4.2	Procedure	1208

27.22.8.5	Test requirement	1221

27.22.9	Handling of command number	1221

27.22.9.1	Definition and applicability	1221

27.22.9.2	Conformance requirement	1222

27.22.9.3	Test purpose	1222

27.22.9.4	Method of tests	1222

27.22.9.4.1	Initial conditions	1222

27.22.9.4.2	Procedure	1222

27.22.9.5	Test requirement	1222

27.22.10	CALL CONTROL on EPS PDN Connection	1222

27.22.10.1	Procedure for Mobile Originated calls	1222

27.22.10.1.1	Definition and applicability	1222

27.22.10.1.2	Conformance requirement	1222

27.22.10.1.3	Test purpose	1222

27.22.10.1.4	Method of tests	1223

27.22.11	CALL CONTROL on PDP Context Activation	1232

27.22.11.1	Procedure for Mobile Originated calls	1232

Annex A (normative): Details of Test-SIM (TestSIM)	1252

Annex B (normative): Details of terminal profile support	1254

Annex C (informative): Change history	1268

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document describes the technical characteristics and methods of test for testing the USIM Application Toolkit implemented in 3rd Generation Mobile Equipments (ME) or Mobile Station (MS) for the 5G, LTE-Advanced, LTE, 3G and 2G digital cellular communications systems within the 3GPP digital cellular telecommunications system, in compliance with the relevant requirements, and in accordance with the relevant guidance given in ISO/IEC 9646-7 [19] and ETSI ETS 300 406 [20].

The present document is valid for ME implemented according to 3GPP Release 99, or Release 4, or any later Release.

The present document covers the minimum characteristics considered necessary in order to provide sufficient performance for mobile equipment and to prevent interference to other services or to other users, and to the PLMNs.

It does not necessarily include all the characteristics which may be required by a user or subscriber, nor does it necessarily represent the optimum performance achievable.

The present document is part of the 3GPP-series of technical specifications. The present document neither replaces any of the other 3GPP technical specifications or 3GPP related ETSs or ENs, nor is it created to provide full understanding of (or parts of) the UMTS. The present document lists the requirements, and provides the methods of test for testing the USIM Application Toolkit implemented in a ME for conformance to the 3GPP standard.

For a full description of the system, reference should be made to all the 3GPP technical specifications or 3GPP related ETSIs, ETSs or ENs. Clause 2 provides a complete list of the 3GPP technical specifications, 3GPP related ETSI's EtSs, ENs, and ETRs, on which this conformance test specifications is based.

If there is a difference between this present conformance document, and any other 3GPP technical specification or 3GPP related ETSI, ETS, EN, or 3GPP TS, then the other 3GPP technical specification or 3GPP related ETSI ETS, EN or 3GPP TS shall prevail.

Within the context of this document, the term "terminal" used in ETSI TS 102 384 [26] refers to the Mobile Equipment (ME).

Within the context of this document, the term "UICC" used in ETSI TS 102 384 [26] refers to the USIM card.

Within the context of this document, the term "NAA" used in ETSI TS 102 384 [26] refers to the USIM application.

For the avoidance of doubt, references to clauses of ETSI TS 102 384 [26] or ETSI TS 102 221 [13] include all the subclauses of that clause, unless specifically mentioned.

The target test specification ETSI TS 102 384 [26] contains material that is outside of the scope of 3GPP requirements and the present document indicates which parts are in the scope and which are not.

A 3GPP ME may support functionality that is not required by 3GPP, but the requirements to do so are outside of the scope of 3GPP. Thus the present document does not contain tests or references to ETSI TS 102 384 [26] tests for features which are out of scope of 3GPP.

In the present document, unless explicitly stated otherwise, for Rel-13 onwards the term E-UTRAN implicitly refers to the E-UTRAN in WB-S1 mode. E-UTRAN in NB-S1 mode is always explicitly referred to as NB-IoT.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the relevant Release.

- References to 3GPP Technical Specifications and Technical Reports throughout the present document shall be interpreted according to the Release shown in the formal reference in this clause, based upon the Release of the implementation under test.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.001: "Principles of circuit telecommunication services supported by a Public Land Mobile Network (PLMN)".

[3]	3GPP TS 22.003: "Circuit Teleservices supported by a Public Land Mobile Network (PLMN)".

[4]	3GPP TS 22.004: "General on supplementary services".

[5]	ETSI TS 101 220: "ETSI numbering system for telecommunication application providers"

[6]	3GPP TS 21.904: "UE capability requirements"

[7]	3GPP TS 23.038: "Alphabets and language-specific information".

[8]	3GPP TS 23.040: "Technical realization of the Short Message Service (SMS)".

[9]	3GPP TS 23.041: "Technical realization of Cell Broadcast Service (CBS)".

[10]	3GPP TS 24.008: "Mobile radio interface layer 3 specification; Core network protocols; Stage 3".

[11]	3GPP TS 24.011: "Point-to-Point (PP) Short Message Service (SMS) Support on mobile radio interface".

[12]	3GPP TS 34.108: "Common test environments for User Equipment (UE) conformance testing".

[13]	If the device under test is a

- R99 ME:	ETSI TS 102 221 v3.18.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-4 ME:	ETSI TS 102 221 v4.16.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-5 ME:	ETSI TS 102 221 v5.10.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-6 ME:	ETSI TS 102 221 v6.15.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-7 ME:	ETSI TS 102 221 v7.17.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-8 ME:	ETSI TS 102 221 v8.5.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-9 ME:	ETSI TS 102 221 v9.2.0: "UICC-Terminal interface; Physical and logical characteristics"",

- Rel-10 ME:	ETSI TS 102 221 v10.0.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-11 ME:	ETSI TS 102 221 v11.1.0: "UICC-Terminal interface; Physical and logical characteristics",

- Rel-12 ME:	ETSI TS 102 221 v12.1.0: "UICC-Terminal interface; Physical and logical characteristics".

- Rel-13 ME:	ETSI TS 102 221 v13.1.0: "UICC-Terminal interface; Physical and logical characteristics".

- Rel-14 ME:	ETSI TS 102 221 v14.1.0: "UICC-Terminal interface; Physical and logical characteristics".

- Rel-15 ME:	ETSI TS 102 221 v15.0.0: "UICC-Terminal interface; Physical and logical characteristics".

[14]	3GPP TS 31.102: "Characteristics of the USIM application".

[15]	3GPP TS 31.111: "USIM Application Toolkit (USAT)"

[16]	Void

[17a]	ISO/IEC 10646-1: "Information technology - Universal Multiple Octet Coded Character Set (UCS) - Part 1: Architecture and Basic Multilingual Plane".

[17b]	ISO/IEC 10646-2: "Information technology - Universal Multiple Octet Coded Character Set (UCS) - Part 2: Supplementary Planes".

[18]	3GPP TS 27.007: "AT command set for 3G User Equipment (UE)".

[19]	ISO/IEC 9646-7 (1995): "Information technology - Open Systems Interconnection - Conformance testing methodology and framework - Part 7: Implementation Conformance Statements".

[20]	ETSI ETS 300 406 (1995): "Methods for Testing and Specification (MTS); Protocol and profile conformance testing specifications; Standardization methodology".

[21]	3GPP TS 31.121: "UICC-terminal interface; USIM application test specification"

[22]	3GPP TS 22.101: "Service Aspects; Service principles"

[23]	3GPP TS 51.010-1: "Mobile Station (MS) conformance specification; Part 1: Conformance specification"

[24]	Void.

[25]	TIA/IS-820-A: "Removable User Identity Module (R-UIM) for TIA/EIA Spread Spectrum System".

[26]	ETSI TS 102 384: "Smart cards; UICC-Terminal interface; Card Application Toolkit (CAT) conformance specification".

[27]	3GPP TS 34.123-3: "User Equipment (UE) conformance specification; Part 3: Abstract test suites (ATSs)".

[28]	3GPP TS 31.115: "Secured packet structure for (U)SIM Toolkit applications".

[29]	3GPP TS 23.122: "Non-Access Stratum functions related to Mobile Station (MS) in idle mode".

[30]	3GPP TS 23.107: "Quality of Service (QoS) concept and architecture".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[32]	3GPP TS 24.301: "Technical Specification Group Core Network and Terminals; Non-Access-Stratum (NAS) protocol for Evolved Packet Systems (EPS): Stage 3".

[33]	3GPP TS 36.508: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); Common test environments for User Equipment (UE) conformance testing".

[34]	3GPP TS 36.523-2 " Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Packet Core (EPC); User Equipment (UE) conformance specification;Part 2: Implementation Conformance Statement (ICS) proforma specification"

[35]	3GPP TS 31.103: "Characteristics of the IP Multimedia Services Identity Module (ISIM) application".

[36]	3GPP TS 34.229-1: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); User Equipment (UE) conformance specification; Part 1: Protocol conformance specification".

[37]	3GPP TS 24.341: "Support of SMS over IP networks".

[38]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[39]	3GPP TS 38.508-1: "5GS; User Equipment (UE) conformance specification; Part 1: Common test environment".

[40]	3GPP TS 36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[40]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[41]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[42]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[43]	3GPP TS 24.526: "User Equipment (UE) policies for 5G System (5GS); Stage 3".

[44]	3GPP TS 23.003: "Numbering, addressing and identification".

# 3	Definitions and abbreviations

## 3.1	Mobile station definition and configurations

The mobile station definition and configurations specified in TS 34.108 [12] and TS 36.508 [33] shall apply, unless otherwise specified in the present clause.

## 3.2	Applicability

### 3.2.1	Applicability of the present document

The present specification applies to a terminal equipment that supports the USIM Application Toolkit optional feature.

### 3.2.2	Applicability of the individual tests

Table A.1 lists the optional features for which the supplier of the implementation states the support.

### 3.2.3	Applicability to terminal equipment

The applicability to terminal equipment specified in TS 34.108 [12] and TS 36.508 [33] shall apply, unless otherwise specified in the present clause.

Within the context of this document, the term "USS" refers to the "UMTS System Simulator" when accessing a UTRAN, to the "System Simulator" when accessing a GERAN, the term "E-USS" refers to the "Evolved Universal System Simulator" when accessing an E-UTRAN in WB-S1 mode, the term "NB-SS" refers to the "NB System Simulator" when accessing an E-UTRAN in NB-S1 mode and the term "NG-SS" refers to the "Next Generation System Simulator" when accessing an NG-RAN.

See table B.1.

### 3.2.4	Definitions

For the purposes of the present document, the terms and definitions given in TS 34.108 [12] and TS 31.121 [21] apply.

#### 3.2.4.1	Format of the table of optional features

Option: The optional feature supported or not by the implementation.

Support Answer notation: The support columns shall be filled in by the supplier of the implementation. The following common notations, defined in ISO/IEC 9646-7 [19], are used for the support column in the tables below.

Y or y	supported by the implementation

N or n	not supported by the implementation

N/A, n/a or -	no answer required (allowed only if the status is N/A, directly or after evaluation of a conditional status)

Mnemonic column: The Mnemonic column contains mnemonic identifiers for each item.

#### 3.2.4.2	Format of the applicability table

The applicability of every test in table B.1 is formally expressed by the use of Boolean expression defined in the following clause.

The columns in table B.1 have the following meaning:

-	In the "Item" column a local entry number for the requirement in the table is given.

-	In the "Description" column a short non-exhaustive description of the requirement is found.

-	The "Release" column gives the Release applicable and onwards, for the item in the "Description" column

-	The "Test Sequence(s)" column gives a reference to the test sequence number(s) detailed in the present document and required to validate the implementation of the corresponding item in the "Description" column.

-	For a given Release, the corresponding "Rel X ME" column lists the tests required for a Mobile Station to be declared compliant to this Release.

-	The "Support" column is blank in the proforma, and shall be completed by the manufacturer in respect of each particular requirement to indicate the choices, which have been made in the implementation.

-	The "Network Dependency" column indicates if  a test depends on specific network access technology or requires network connection, but the status may not have an impact on references to ETSI TS 102 384 [26].

-	The "Terminal Profile" column gives a reference to the corresponding Terminal Profile bit(s) that is/are related to the toolkit feature(s) of the respective test(s).

-	The "Additional test case execution parameter" column shall be used in conjunction with the entry in the "Rel-xx ME" column. The column indicates if the test is affected by additional test case execution parameters.

#### 3.2.4.3	Status and notations

"Release X ME" columns show the status of the entries as follows:

The following notations, defined in ISO/IEC 9646-7 [19], are used for the status column:

M	mandatory - the capability is required to be supported.

O	optional - the capability may be supported or not.

N/A	not applicable - in the given context, it is impossible to use the capability.

X	prohibited (excluded) - there is a requirement not to use this capability in the given context.

O.i	qualified optional - for mutually exclusive or selectable options from a set. "i" is an integer which identifies an unique group of related optional items and the logic of their selection which is defined immediately following the table.

Ci	conditional - the requirement on the capability ("M", "O", "X" or "N/A") depends on the support of other optional or conditional items. "i" is an integer identifying an unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." shall be used to avoid ambiguities.

The "Additional test case execution parameter" column shows the status of the entries as follows:

TCEPi	Test Case Execution Parameter –defines additional parameters which have to be taken into account when executing affected test case(s). "i" is an integer identifying an unique parameter which is defined immediately following the table.

A	applicable - the test is applicable according to the corresponding entry in the "Rxx ME" column

R(x)	redundant – the test has to be considered as redundant when the corresponding E-UTRAN/EPC related test "x" of the present document has been validated and successfully executed. In that case the requirement may be verified by means of the E-UTRAN/EPC functionality only.

AERi	Additional test case Execution Recommendation – with respect to the above listed definitions of ("A") and ("R") the test is applicable ("A") or redundant ("R") depending on the support of other optional or conditional items. "i" is an integer identifying a unique conditional status expression which is defined immediately following the table. For nested conditional expressions, the syntax "IF ... THEN (IF ... THEN ... ELSE...) ELSE ..." shall be used to avoid ambiguities.

References to items: For each possible item answer (answer in the support column) there exists a unique reference, used, for example, in the conditional expressions. It is defined as the table identifier, followed by a solidus character "/", followed by the item number in the table. If there is more than one support column in a table, the columns shall be discriminated by letters (a, b, etc.), respectively.

EXAMPLE:	A.1/4 is the reference to the answer of item 4 in table A.1.

## 3.3	Table of optional features

Support of USIM Application Toolkit is optional for Mobile Equipment. However, if an ME states conformance with a specific 3GPP release, it is mandatory for the ME to support all functions of that release, as stated in table B.1, with the exception of the functions:

-	"Alpha identifier in REFRESH command supported by terminal";

-	"Event Language Selection";

-	"Proactive UICC: PROVIDE LOCAL INFORMATION (language)"; and

-	"Proactive UICC: LANGUAGE NOTIFICATION".

In accordance to 3GPP TS 36.300 [40], subclause 4.10, additional exceptions apply for NB-IoT only MEs.
As a number of E-UTRA protocol functions supported by Rel-8 MEs are not required for NB-IoT, the related USIM Application Toolkit functions may not be supported by NB-IoT only MEs.

Note: NB-IoT only MEs are MEs that only support NB-IoT and no other radio access technology.

The support of letter classes, which specify mainly ME hardware dependent features, is optional for the ME and may supplement the USIM Application Toolkit functionality described in the present document. If an ME states conformance to a letter class, it is mandatory to support all functions within the respective letter class.

The supplier of the implementation shall state the support of possible options in table A.1.

Table A.1: Options

| Item | Option | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- |
| 1 | Capability Configuration parameter | M |  | O_Cap_Conf |
| 2 | Sustained text | C002 |  | O_sust_text |
| 3 | UCS2 coding scheme for Entry | O |  | O_Ucs2_Entry |
| 4 | Extended Text String | C002 |  | O_Ext_Str |
| 5 | Help information | O |  | O_Help |
| 6 | Icons | O |  | O_Icons |
| 7 | Class A: Dual Slot | O |  | O_Dual_Slot |
| 8 | Detachable reader | O |  | O_Detach_Rdr |
| 9 | Class B: RUN AT | O |  | O_Run_At |
| 10 | Class C: LAUNCH BROWSER | O |  | O_LB |
| 11 | Class D: Soft keys | O |  | O_Soft_key |
| 12 | Class E: B.I.P related to CSD | O |  | O_BIP_CSD |
| 13 | Screen sizing parameters | O |  | O_Scr_Siz |
| 14 | Screen Resizing | O |  | O_Scr_Resiz |
| 15 | UCS2 coding scheme for Display | O |  | O_Ucs2_Disp |
| 16 | Mobile supporting GPRS | O |  | O_GPRS |
| 17 | Mobile supporting UDP | O |  | O_UDP |
| 18 | Mobile supporting TCP | O |  | O_TCP |
| 19 | Redial in Set Up Call | O |  | O_Redial |
| 20 | Mobile decision to respond with "No response from user" in finite time | O |  | O_D_NoResp |
| 21 | Class E: B.I.P related to GPRS | O |  | O_BIP_GPRS |
| 22 | Mobile supporting Called Party Subaddress | O |  | O_CP_Subaddr |
| 23 | Immediate response | O |  | O_Imm_Resp |
| 24 | Variable Timeout | O |  | O_Duration |
| 25 | void |  |  |  |
| 26 | Class F: B.I.P related to local bearer | O |  | O_BIP_Local |
| 27 | BlueTooth Support | O |  | O_BT |
| 28 | IrDA Support | O |  | O_IrDA |
| 29 | RS232 Support | O |  | O_RS232 |
| 30 | USB Support | O |  | O_USB |
| 31 | WML Browser Support | O |  | O_WML |
| 32 | XHTML Browser Support | O |  | O_XHTML |
| 33 | HTML Browser Support | O |  | O_HTML |
| 34 | CHTML Browser Support | O |  | O_CHTML |
| 35 | Class G: Battery Data | O |  | O_Batt |
| 36 | Class H: Multimedia Call Support | O |  | O_Xmedia_Call |
| 37 | Class I: Frame support | O |  | O_Frames |
| 38 | Class J: Multimedia Messaging Support | O |  | O_MMS |
| 39 | ME requesting for user confirmation before sending the Envelope Call Control command | O |  | O_UC_Before_EnvCC |
| 40 | ME requesting for user confirmation after sending the Envelope Call Control command | O |  | O_UC_After_EnvCC |
| 41 | UCS2 in Cyrillic | O |  | O_UCS2_Cyrillic |
| 42 | UCS2 in Chinese | O |  | O_UCS2_Chinese |
| 43 | UCS2 in Katakana | O |  | O_UCS2_Katakana |
| 44 | Mobile supporting Barred Dialling Numbers | O |  | O_BDN |
| 45 | Mobile supporting Fixed dialling numbers | O |  | O_FDN |
| 46 | Mobile supporting "+CIMI" in combination with Run AT Command | O |  | O_+CIMI |
| 47 | Mobile supporting "+CGMI" in combination with Run AT Command | O |  | O_+CGMI |
| 48 | Mobile supporting Open Channel (GPRS) not containing a Network Access Name TLV when no default Access Point Name is set in the terminal configuration | O |  | O_Open_Channel_GPRS_without_DefaultAPN |
| 49 | Preferred buffer size supported by the terminal for Open Channel command is greater than 0 byte and less than 65535 bytes | O |  | O_BUFFER_SIZE |
| 50 | Text attributes – Alignment left | O |  | O_TAT_AL |
| 51 | Text attributes – Alignment center | O |  | O_TAT_AC |
| 52 | Text attributes – Alignment right | O |  | O_TAT_AR |
| 53 | Text attributes – Font size normal | O |  | O_TAT_FSN |
| 54 | Text attributes – Font size large | O |  | O_TAT_FSL |
| 55 | Text attributes – Font size small | O |  | O_TAT_FSS |
| 56 | Text attributes – Style normal | O |  | O_TAT_SN |
| 57 | Text attributes – Style bold | O |  | O_TAT_SB |
| 58 | Text attributes – Style italic | O |  | O_TAT_SI |
| 59 | Text attributes – Style underlined | O |  | O_TAT_SU |
| 60 | Text attributes – Style strikethrough | O |  | O_TAT_SS |
| 61 | Text attributes – Style text foreground colour | O |  | O_TAT_STFC |
| 62 | Text attributes – Style text background colour | O |  | O_TAT_STFB |
| 63 | Terminal supports Long ForwardToNumber | O |  | O_longFTN |
| 64 | Mobile supporting GERAN | O |  | O_GERAN |
| 65 | Support of global phonebook | C001 |  | O_Global_PB |
| 66 | HSDPA Support | O |  | O_HSDPA |
| 67 | UTRAN PS with extended parameters Support | O |  | O_UTRAN_PS_Ext_Param |
| 68 | Terminal executes User confirmation phase before sending PDP context activation request | O |  | O_User_Confirm_Before_PDP_Context_Request |
| 69 | ME supports Call Hold Supplementary Service | O |  | O_Serv_SS_HOLD |
| 70 | Class E: B.I.P. related to I-WLAN | O |  | O_I-WLAN |
| 71 | Class K: Terminal Applications support | O |  | O_Terminal_Applications |
| 72 | Class E: Terminal supports TCP, UICC in Server Mode | O |  | O_TCP_UICC_ServerMode |
| 73 | Class E: Terminal supports TCP, Terminal in Server Mode | O |  | O_TCP_Terminal_ServerMode |
| 74 | Class E: Terminal supports UDP, Terminal in Server Mode | O |  | O_UDP_Terminal_ServerMode |
| 75 | Void |  |  |  |
| 76 | Void |  |  |  |
| 77 | Void |  |  |  |
| 78 | Terminal supports at least one supplementary service. | O |  | O_AddInfo_SS |
| 79 | Terminal supports "Call Forwarding Unconditional" | O |  | O_ Serv_SS_CFU |
| 80 | Terminal supports "Calling Line Identification Restriction" | O |  | O_Serv_SS_CLIR |
| 81 | Class N:Terminal supports "Geographical location discovery" | O |  | O_Geo_Location_Discovery |
| 82 | Terminal supports melody and theme tones | O |  | O_M_T_Tones |
| 83 | Terminal supports Toolkit-initiated GBA | O |  | O_Toolkit_GBA |
| 84 | Terminal supports display capability | C002 |  | O_ No_Type_ND |
| 85 | Terminal supports keypad | C002 |  | O_No_Type_NK |
| 86 | Terminal supports audio alerting | C002 |  | O_No_Type_NA |
| 87 | Terminal supports speech call | C002 |  | O_No_Type_NS |
| 88 | Terminal supports multiple languages | C002 |  | O_No_Type_NL |
| 89 | Class P:USSD Data Download and application mode | O |  | O_USSD_Data_DL |
| 90 | Terminal displays icons as defined in record 1 of EF(IMG) for Display Text command | O |  | O_Icon Rec1_Disp_Text |
| 91 | Terminal displays icons as defined in record 2 of EF(IMG) for Display Text command | O |  | O_Icon Rec2_Disp_Text |
| 92 | Terminal displays icons as defined in record 5 of EF(IMG) for Display Text command | O |  | O_Icon Rec5_Disp_Text |
| 93 | Terminal displays icons as defined in record 1 of EF(IMG) for Get Inkey command | O |  | O_Icon Rec1_Get_Inkey |
| 94 | Terminal displays icons as defined in record 2 of EF(IMG) for Get Inkey command | O |  | O_Icon Rec2_Get_Inkey |
| 95 | Terminal displays icons as defined in record 5 of EF(IMG) for Get Inkey command | O |  | O_Icon Rec5_Get_Inkey |
| 96 | Terminal displays icons as defined in record 1 of EF(IMG) for Get Input command | O |  | O_Icon Rec1_Get_Input |
| 97 | Terminal displays icons as defined in record 2 of EF(IMG) for Get Input command | O |  | O_Icon Rec2_Get_Input |
| 98 | Terminal displays icons as defined in record 5 of EF(IMG) for Get Input command | O |  | O_Icon Rec5_Get_Input |
| 99 | Terminal displays icons as defined in record 1 of EF(IMG) for Play Tone command | O |  | O_Icon Rec1_Play_Tone |
| 100 | Terminal displays icons as defined in record 2 of EF(IMG) for Play Tone command | O |  | O_Icon Rec2_Play_Tone |
| 101 | Terminal displays icons as defined in record 5 of EF(IMG) for Play Tone command | O |  | O_Icon Rec5_Play_Tone |
| 102 | Terminal displays icons as defined in record 1 of EF(IMG) for Set Up Menu command | O |  | O_Icon_ Rec1_Set_Up_Menu |
| 103 | Terminal displays icons as defined in record 2 of EF(IMG) for Set Up Menu command | O |  | O_Icon_ Rec2_Set_Up_Menu |
| 104 | Terminal displays icons as defined in record 5 of EF(IMG) for Set Up Menu command | O |  | O_Icon_ Rec5_Set_Up_Menu |
| 105 | Terminal displays icons as defined in record 1 of EF(IMG) for Select Item command | O |  | O_Icon_ Rec1_Select_Item |
| 106 | Terminal displays icons as defined in record 2 of EF(IMG) for Select Item command | O |  | O_Icon_ Rec2_Select_Item |
| 107 | Terminal displays icons as defined in record 5 of EF(IMG) for Select Item command | O |  | O_Icon_ Rec5_Select_Item |
| 108 | Terminal displays icons as defined in record 1 of EF(IMG) for Send Short Message command | O |  | O_Icon_ Rec1_Send_SM |
| 109 | Terminal displays icons as defined in record 2 of EF(IMG) for Send Short Message command | O |  | O_Icon_ Rec2_Send_SM |
| 110 | Terminal displays icons as defined in record 5 of EF(IMG) for Send Short Message command | O |  | O_Icon_ Rec5_Send_SM |
| 111 | Terminal displays icons as defined in record 1 of EF(IMG) for Send SS command | O |  | O_Icon_ Rec1_Send_SS |
| 112 | Terminal displays icons as defined in record 2 of EF(IMG) for Send SS command | O |  | O_Icon_ Rec2_Send_SS |
| 113 | Terminal displays icons as defined in record 5 of EF(IMG) for Send SS command | O |  | O_Icon_ Rec5_Send_SS |
| 114 | Terminal displays icons as defined in record 1 of EF(IMG) for Send USSD command | O |  | O_Icon_ Rec1_Send_USSD |
| 115 | Terminal displays icons as defined in record 2 of EF(IMG) for Send USSD command | O |  | O_Icon_ Rec2_Send_USSD |
| 116 | Terminal displays icons as defined in record 5 of EF(IMG) for Send USSD command | O |  | O_Icon_ Rec5_Send_USSD |
| 117 | Terminal displays icons as defined in record 1 of EF(IMG) for Set Up Call command | O |  | O_Icon_ Rec1_Set_Up_Call |
| 118 | Terminal displays icons as defined in record 2 of EF(IMG) for Set Up Call command | O |  | O_Icon_ Rec2_Set_Up_Call |
| 119 | Terminal displays icons as defined in record 5 of EF(IMG) for Set Up Call command | O |  | O_Icon_ Rec5_Set_Up_Call |
| 120 | Terminal displays icons as defined in record 1 of EF(IMG) for Set Up Idle Mode Text command | O |  | O_Icon_ Rec1_Set_Up_Idle_Mode_Text |
| 121 | Terminal displays icons as defined in record 2 of EF(IMG) for Set Up Idle Mode Text command | O |  | O_Icon_ Rec2_Set_Up_Idle_Mode_Text |
| 122 | Terminal displays icons as defined in record 5 of EF(IMG) for Set Up Idle Mode Text command | O |  | O_Icon_ Rec5_Set_Up_Idle_Mode_Text |
| 123 | Terminal displays icons as defined in record 1 of EF(IMG) for Run AT Command command | O |  | O_Icon_ Rec1_Run_AT_Cmd |
| 124 | Terminal displays icons as defined in record 2 of EF(IMG) for Run AT Command command | O |  | O_Icon_ Rec2_Run_AT_Cmd |
| 125 | Terminal displays icons as defined in record 5 of EF(IMG) for Run AT Command command | O |  | O_Icon_ Rec5_Run_AT_Cmd |
| 126 | Terminal displays icons as defined in record 1 of EF(IMG) for Send DTMF command | O |  | O_Icon_ Rec1_Send_DTMF |
| 127 | Terminal displays icons as defined in record 2 of EF(IMG) for Send DTMF command | O |  | O_Icon_ Rec2_Send_DTMF |
| 128 | Terminal displays icons as defined in record 5 of EF(IMG) for Send DTMF command | O |  | O_Icon_ Rec5_Send_DTMF |
| 129 | Terminal displays icons as defined in record 1 of EF(IMG) for Launch Browser command | O |  | O_Icon_ Rec1_Launch_Browser |
| 130 | Terminal displays icons as defined in record 2 of EF(IMG) for Launch Browser command | O |  | O_Icon_ Rec2_Launch_Browser |
| 131 | Terminal displays icons as defined in record 5 of EF(IMG) for Launch Browser command | O |  | O_Icon_ Rec5_Launch_Browser |
| 132 | Class E: Terminal does support eFDD | O |  | pc_BIP_eFDD |
| 133 | Class E: Terminal does support eTDD | O |  | pc_BIP_eTDD |
| 134 | Terminal supports UTRAN | O |  | O_UTRAN |
| 135 | Terminal supports E-UTRAN but neither UTRAN nor GERAN | C003 |  | O_EUTRAN_NO_UTRAN_NO_GERAN |
| 136 | CLASS Q: Terminal supports Event CSG Cell Selection | O |  | O_Event_CSG_Cell_Selection |
| 137 | CLASS Q: Terminal supports CSG Cell Discovery | O |  | O_CSG_Cell_Discovery |
| 138 | Terminal supports selection of default item in Select Item | O |  | O_Select_Item_Default_Item |
| 139 | Terminal supports eFDD | O |  | pc_eFDD |
| 140 | Terminal supports eTDD | O |  | pc_eTDD |
| 141 | Terminal supports SM-over-IP-receiver | O |  | pc_SM-over-IP-receiver |
| 142 | Terminal supports MO SMS over IMS | O |  | pc_MO_SM-over-IMS |
| 143 | Class K: Terminal supports Direct Communication Channel | O |  | O_Direct_Com_Channel |
| 144 | Terminal supports Communication Control for IMS | O |  | O_CC_IMS |
| 145 | Class S: Terminal supports CAT over modem interface | O |  | O_CAT_Modem_Interface |
| 146 | Class E and T: Event Incoming IMS Data | O |  | O_Event_Incoming_IMS_Data |
| 147 | Class E and T: Event IMS Registration | O |  | O_Event_IMS_Registration |
| 148 | Class E and T: UICC Access to IMS support | O |  | O_UICC_ACCESS_IMS |
| 149 | Terminal supports SMS Cell Broadcast Data Download | O |  | O_SMS-CB_Data_Download |
| 150 | Terminal supports IMS | O |  | O_IMS |
| 151 | Terminal operating in PS mode | O |  | O_PS_OPMODE |
| 152 | Terminal supports Short Message Service (SMS) MT over SGs | O |  | O_SMS_SGs_MT |
| 153 | Terminal supports Short Message Service (SMS) MO over SGs | O |  | O_SMS_SGs_MO |
| 154 | Terminal sends RP-ACK for '62XX' and '63XX' for SMS-PP download | C004 |  | O_RP-ACK_for_SMS-PP_ error |
| 155 | Terminal supports browser with multiple sessions/tabs | O |  | O_Browser_tabs |
| 156 | Terminal supports Short Message Service (SMS) MT over CS | O |  | pc_SMS_CS_MT |
| 157 | Terminal supports Short Message Service (SMS) MO over CS | O |  | pc_SMS_CS_MO |
| 158 | Terminal supports Short Message Service (SMS) MT over PS | O |  | pc_SMS_PS_MT |
| 159 | Terminal supports Short Message Service (SMS) MO over PS | O |  | pc_SMS_PS_MO |
| 160 | Terminal rejects Launch Browser with Default URL | C005 |  | O_Rej_Launch_Browser_with DefURL |
| 161 | Terminal supports Event Language Selection | O |  | O_Lang_Select |
| 162 | Terminal supports Provide Local Information (Language) | O |  | O_Provide_Local_LS |
| 163 | Terminal supports Language Notification | O |  | O_Lang_Notif |
| 164 | Terminal supports Alpha Identifier in REFRESH command | O |  | O_Refresh_AlphaIdentifier |
| 165 | ProSe usage information reporting | O |  | O_ProSE |
| 166 | Event: WLAN Access status | O |  | O_WLAN_Access_Status |
| 167 | Class E: WLAN bearer support | O |  | O_WLAN_Bearer |
| 168 | Terminal supports to a I-WLAN or a WLAN | O |  | O_I-WLAN_OR_WLAN |
| 169 | Terminal supports Media Type "Voice" for SET UP CALL and Call Control by USIM | O |  | O_Media_Type_Voice |
| 170 | Terminal supports Media Type "Video" for SET UP CALL and Call Control by USIM | O |  | O_Media_Type_Video |
| 171 | Terminal supports sending location status and access technology that is already available | C006 |  | O_LS_and_ATC_events |
| 172 | Terminal performs USIM deactivation during 3G Session Reset REFRESH | O |  | O_USIM_Deact_during_Refresh |
| 173 | Terminal does support NB-IoT | O |  | pc_NB |
| 174 | Terminal supports EMM-REGISTERED with PDN | O |  | pc_AttachWithPDN |
| 175 | Void | O |  |  |
| 176 | Void | O |  |  |
| 177 | Class E: Terminal does support NB-IoT | O |  | pc_BIP_NB |
| 178 | Support for multiple PDN connections | O |  | pc_Multiple_PDN |
| 179 | Support for SM-over-IP without MSISDN | O |  | O_SM-over-IP_without_MSISDN |
| 180 | Class ae: Originate voice call with URI | O |  | O_Voice_Call_with_URI |
| 181 | Terminal does support deactivation of the UICC in PSM. | O |  | O_PSM_DEAC_UICC |
| 182 | Terminal does support the UICC suspension mechanism in PSM. | O |  | O_PSM_ SUSPEND_UICC |
| 183 | Terminal does support the UICC suspension mechanism during extended DRX | O |  | O_eDRX_ SUSPEND_UICC |
| 184 | Terminal supports 3GPP PS data off | O |  | O_PS_Data_Off |
| 185 | Terminal supports Date-Time and Time zone | O |  | O_DateTime_ Time_zone |
| 186 | Terminal supports NB-IoT only | O |  | O_NB-IoT_only |
| 187 | Terminal supports NG-RAN | O |  | pc_NG_RAN |
| 188 | Class E: Terminal does support NG-RAN | O |  | pc_BIP_NG_RAN |
| C001 If terminal is implemented according to Rel-6 or later AND (A.1/84 AND A.1/85 AND A.1/87) then M, else O |  |  |  |  |
| C002 If feature is implemented according to Rel-8 or later then O, else M. It is possible to implement the related features according to Rel-8 or later even if the generic toolkit implementation is according to a release earlier then Rel-8. |  |  |  |  |
| C003 If terminal is implemented according to Rel-8 or later AND ((A.1/132 OR A.1/133) AND (NOT A.1/64) AND (NOT A.1/134)) THEN M ELSE N/A |  |  |  |  |
| C004 If feature is implemented according to Rel-11 or later then M, else N/A |  |  |  |  |
| C005 If feature is implemented according to Rel-12 or later then O, else N/A |  |  |  |  |
| C006 If feature is implemented according to Rel-13 or later then M, else O |  |  |  |  |
| NOTE: Items 161, 162, 163 and 164 were made optional as a consequence of the approval of CR 0429 against TS 31.111 and CR 0419 against TS 31.124 |  |  |  |  |

## 3.4	Applicability table

NOTE:	It is possible that the applicability of some tests indicated in table B.1 does not match with the value in the Release column, due to late definition of the test sequences. Tests should be performed without considering the Release column, but only based on the conditions indicated for each release.

Table B.1: Applicability of tests

| Item | Description |  |  | Re-lease | Test sequence (s) | Rel 99 ME | Rel-4 ME |  | Rel-5 ME | Rel-6 ME | Rel-7 ME | Rel-8 ME | Rel-9 ME | Rel-10 ME | Rel-11 ME | Rel-12 ME | Rel-13 ME | Rel-14 ME | Rel-15 ME | Terminal Profile | Network Dependency | Sup-port | Additional test case execution parameter |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PROFILE DOWNLOAD 27.22.1 |  |  | R99 | 1 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/1 | No |  |  |
| 2 | Contents of the TERMINAL PROFILE command 27.22.2 |  |  | R99 |  | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/1 | No |  |  |
| 3 | Servicing of Proactive UICC Commands 27.22.3 |  |  | R99 |  | M | M |  | M | M | M | M | M | M | M | M | M | M | M |  | No |  |  |
| 4 | DISPLAY TEXT 27.22.4.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Unpacked |  |  | R99 | 1.1 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Screen busy |  |  | R99 | 1.2 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | high priority |  |  | R99 | 1.3 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Packed |  |  | R99 | 1.4 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | clear after delay |  |  | R99 | 1.5 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | long text up to 160 bytes |  |  | R99 | 1.6 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Backwards move in USIM session |  |  | R99 | 1.7 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 | No |  |  |
|  | Session terminated by user |  |  | R99 | 1.8 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 | No |  |  |
|  | Command not understood by ME |  |  | R99 | 1.9 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
|  | no response from user |  |  | R99 | 2.1 | C120 AND C177 AND C178 | C120 AND C177 AND C178 |  | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | E.1/17 AND E.1/110 AND E.1/111 | No |  |  |
|  | Extension Text |  |  | R99 | 3.1 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/16 AND E.1/110 | No |  |  |
|  | sustained text |  |  | R99 | 4.1, 4.2 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/65 AND E.1/110 | No |  |  |
|  | sustained text |  |  | R99 | 4.3 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/17 AND E.1/65 AND E.1/110 AND E.1/111 | No |  |  |
|  | sustained text |  |  | R99 | 4.4 | C177 AND C180 | C177 AND C180 |  | C177 AND C180 | C177 AND C180 | C177 AND C180 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | C177 AND C180 AND C183 | E.1/17 AND E.1/65 AND E.1/110 | UMTS System Simulator or System Simulator only |  |  |
|  | Icons – basic icon |  |  | R99 | 5.1, 5.3 | C108 AND C177 | C108 AND C177 |  | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | E.1/17 AND E.1/110 | No |  |  |
|  | Icons – colour icon |  |  | R99 | 5.2 | C171 AND C177 | C171 AND C177 |  | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | E.1/17 AND E.1/110 | No |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 6.1 | C118 AND C177 | C118 AND C177 |  | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | E.1/17 AND E.1/15 AND E.1/110 | No |  |  |
|  | Variable Timeout |  |  | Rel-4 | 7.1 |  | C126 AND C177 AND C178 |  | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | E.1/17 AND E.1/137 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 8.1 |  |  |  | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | E.1/17 AND E.1/124 AND E.1/217 AND E.1/110 | No |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 8.2 |  |  |  | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | E.1/17 AND E.1/124 AND E.1/218 AND E.1/110 | No |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 8.3 |  |  |  | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | E.1/17 AND E.1/124 AND E.1/219 AND E.1/110 | No |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 8.4 |  |  |  | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | E.1/17 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | No |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 8.5 |  |  |  | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | E.1/17 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | No |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 8.6 |  |  |  | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | No |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 8.7 |  |  |  | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | No |  |  |
|  | Text attribute – underlined on |  |  | Rel-5 | 8.8 |  |  |  | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | No |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 8.9 |  |  |  | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | E.1/17 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | No |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 8.10 |  |  |  | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | E.1/17 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | No |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 9.1 |  |  |  | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | E.1/17 AND E.1/15 AND E.1/110 | No |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 10.1 |  |  |  | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | E.1/17 AND E.1/15 AND E.1/110 | No |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/17 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
| 5 | GET INKEY 27.22.4.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | prompt unpacked |  |  | R99 | 1.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | prompt packed |  |  | R99 | 1.2 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | Backwards move in UICC session |  |  | R99 | 1.3 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | Session terminated by user |  |  | R99 | 1.4 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | SMS alphabet |  |  | R99 | 1.5 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | Long text up to 160 bytes |  |  | R99 | 1.6 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | no response from user |  |  | R99 | 2.1 | C120 AND C177 AND C178 | C120 AND C177 AND C178 |  | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 3.1 | C118 AND C177 AND C178 | C118 AND C177 AND C178 |  | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display, Long text up to 70 chars in Cyrillic |  |  | R99 | 3.2 | C118 AND C177 AND C178 | C118 AND C177 AND C178 |  | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 entry in Cyrillic |  |  | R99 | 4.1 | C105 AND C177 AND C178 | C105 AND C177 AND C178 |  | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | E.1/18 AND E.1/14 AND E.1/110 AND E.1/111 | No |  |  |
|  | "Yes/No" response |  |  | R99 | 5.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/18 AND E.1/60 AND E.1/110 AND E.1/111 | No |  |  |
|  | Icons – basic icon |  |  | R99 | 6.1, 6.2 | C108 AND C177 AND C178 | C108 AND C177 AND C178 |  | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | Icons – colour icon |  |  | R99 | 6.3, 6.4 | C171 AND C177 AND C178 | C171 AND C177 AND C178 |  | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | Help information |  |  | R99 | 7.1 | C107 AND C177 AND C178 | C107 AND C177 AND C178 |  | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | E.1/18 AND E.1/110 AND E.1/111 | No |  |  |
|  | Variable Timeout |  |  | Rel-4 | 8.1 |  | C126 AND C177 AND C178 |  | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | C126 AND C177 AND C178 | E.1/18 AND E.1/140 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 9.1 |  |  |  | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 9.2 |  |  |  | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 9.3 |  |  |  | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 9.4 |  |  |  | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 9.5 |  |  |  | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 9.6 |  |  |  | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 9.7 |  |  |  | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – underlined on |  |  | Rel-5 | 9.8 |  |  |  | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – strikethough on |  |  | Rel-5 | 9.9 |  |  |  | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | E.1/18 AND E.1/124 E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 9.10 |  |  |  | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | E.1/18 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 10.1 |  |  |  | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Chinese, Long text up to 70 chars |  |  | R99 | 10.2 |  |  |  | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 entry in Chinese |  |  | R99 | 11.1 |  |  |  | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | E.1/18 AND E.1/14 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 12.1 |  |  |  | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Katakana, Long text up to 70 chars |  |  | R99 | 12.2 |  |  |  | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | E.1/18 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 entry in Katakana |  |  | R99 | 13.1 |  |  |  | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | E.1/18 AND E.1/14 AND E.1/110 AND E.1/111 | No |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/18 AND E.1/177 AND E.1/178 AND E.1/110 AND E.1/111 | TBD |  |  |
| 6 | GET INPUT 27.22.4.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | input unpacked |  |  | R99 | 1.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | input packed |  |  | R99 | 1.2 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | digits only |  |  | R99 | 1.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | SMS alphabet |  |  | R99 | 1.3 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | hidden input |  |  | R99 | 1.4 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | min / max acceptable length |  |  | R99 | 1.5, 1.9 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Backwards move in UICC session |  |  | R99 | 1.6 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Session terminated by user |  |  | R99 | 1.7 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Prompt text up to 160 bytes |  |  | R99 | 1.8 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | SMS default alphabet, ME to echo text, packing not required |  |  | R99 | 1.9 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Null length for the text string |  |  | R99 | 1.10 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | no response from user |  |  | R99 | 2.1 | C120 AND C177 AND C178 | C120 AND C177 AND C178 |  | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 3.1, 3.2 | C118 AND C177 AND C178 | C118 AND C177 AND C178 |  | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 entry in Cyrillic |  |  | R99 | 4.1, 4.2 | C105 AND C177 AND C178 | C105 AND C177 AND C178 |  | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | C105 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 | No |  |  |
|  | default text for the input |  |  | R99 | 5.1, 5.2 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Icons – basic icon |  |  | R99 | 6.1, 6.2 | C108 AND C177 AND C178 | C108 AND C177 AND C178 |  | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | C108 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Icons – colour icon |  |  | R99 | 6.3, 6.4 | C171 AND C177 AND C178 | C171 AND C177 AND C178 |  | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | C171 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | help information |  |  | R99 | 7.1 | C107 AND C177 AND C178 | C107 AND C177 AND C178 |  | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | E.1/19 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute– left alignment |  |  | Rel-5 | 8.1 |  |  |  | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 8.2 |  |  |  | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 8.3 |  |  |  | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 8.4 |  |  |  | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 8.5 |  |  |  | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 8.6 |  |  |  | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 8.7 |  |  |  | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – underlined on |  |  | Rel-5 | 8.8 |  |  |  | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 8.9 |  |  |  | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 8.10 |  |  |  | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | E.1/19 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | N o |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 9.1, 9.2 |  |  |  | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 entry in Chinese |  |  | R99 | 10.1, 10.2 |  |  |  | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | C142 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 11.1, 11.2 |  |  |  | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | E.1/19 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 entry in Katakana |  |  | R99 | 12.1, 12.2 |  |  |  | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | C144 AND C177 AND C178 | E.1/19 AND E.1/14 AND E.1/110 AND E.1/111 | No |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/19 AND E.1/177 AND E.1/178 AND E.1/110 AND E.1/111 | TBD |  |  |
| 7 | MORE TIME 27.22.4.4 |  |  | R99 | 1.1 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/20 | No |  |  |
| 8 | PLAY TONE 27.22.4.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | play all tones, display alpha, user termination, superimpose |  |  | R99 | 1.1 | C178 AND C179 AND C180 | C178 AND C179 AND C180 |  | C178 AND C179 AND C180 | C178 AND C179 AND C180 | C178 AND C179 AND C180 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | C178 AND C179 AND C180 AND C183 | E.1/21 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Cyrillic |  |  | R99 | 2.1 | C118 AND C179 | C118 AND C179 |  | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | C118 AND C179 | E.1/21 AND E.1/15 AND E.1/110 | No |  | TCEP001 |
|  | Icons – basic icon |  |  | R99 | 3.1, 3.2 | C108 AND C179 | C108 AND C179 |  | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | C108 AND C179 | E.1/21 AND E.1/110 | No |  | TCEP001 |
|  | Icons – colour icon |  |  | R99 | 3.3, 3.4 | C171 AND C179 | C171 AND C179 |  | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | C171 AND C179 | E.1/21 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | C153 AND C179 | E.1/21 AND E.1/124 AND E.1/217 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | C154 AND C179 | E.1/21 AND E.1/124 AND E.1/218 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | C155 AND C179 | E.1/21 AND E.1/124 AND E.1/219 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | C157 AND C156 AND C179 | E.1/21 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | C158 AND C156 AND C179 | E.1/21 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | C160 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | C161 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – underlined on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | C162 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | C163 AND C159 AND C179 | E.1/21 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | No |  | TCEP001 |
|  | Text attribute– foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | C164 AND C165 AND C179 | E.1/21 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | No |  | TCEP001 |
|  | UCS2 display in Chinese |  |  | R99 | 5.1 |  |  |  | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | C143 AND C179 | E.1/21 AND E.1/15 AND E.1/110 | No |  | TCEP001 |
|  | UCS2 display in Katakana |  |  | R99 | 6.1 |  |  |  | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | C145 AND C179 | E.1/21 AND E.1/15 AND E.1/110 | No |  | TCEP001 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/21 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
|  | Themed and Melody tones |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/21 AND E.1/171 AND E.1/110 | TBD |  |  |
| 9 | POLL INTERVAL 27.22.4.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | duration |  |  | R99 | 1.1 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/22 | No |  |  |
| 10 | REFRESH 27.22.4.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | USIM initialization, enabling FDN mode |  |  | R99 | 1.1 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 |  | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | file change notification of FDN file |  |  | R99 | 1.2 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 |  | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | USIM initialization and file change notification of ADN |  |  | R99 | 1.3 | C168 AND C177 AND C178 | C168 AND C177 AND C178 |  | C168 AND C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/24 AND E.1/110 AND E.1/111 | No |  |  |
|  | USIM initialization and full file change notification, enabling FDN mode |  |  | R99 | 1.4 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 |  | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UICC reset |  |  | R99 | 1.5 |  |  |  |  | M | M | M | M | M | M | M | M | M | M | E.1/24 | No |  |  |
|  | USIM Initialization after SMS-PP data download |  |  | R99 | 1.6 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 |  | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E.1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | USIM Application Reset |  |  | R99 | 1.7 |  |  |  |  | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | C146 AND C177 AND C178 AND C180 AND C183 | E1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UICC Reset for IMSI Changing procedure |  |  | R99 | 2.1 |  |  |  |  |  |  | M | M | M | M | M | M | M | M | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | USIM Application Reset for IMSI Changing procedure |  |  | R99 | 2.2 |  |  |  |  | M | M | M | M | M | M | M | M | M | M | E.1/24 | Yes |  |  |
|  | 3G Session Reset for IMSI Changing procedure |  |  | R99 | 2.3 |  |  |  |  |  |  | M | M | M | M | M | M | M | M | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | reject 3G Session Reset for IMSI Changing procedure during CScall |  |  | R99 | 2.4 |  |  |  |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E 1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | reject UICC Reset for IMSI Changing procedure during CS call |  |  | R99 | 2.5 |  |  |  |  |  |  | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E1/24 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UICC Reset for IMSI Changing procedure during active PDP context |  |  | R99 | 2.6 |  |  |  |  |  |  | C215 | C215 | C215 | C215 | C215 | C215 | C215 | C215 | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | 3G Session Reset for IMSI Change procedure during active PDP context |  |  | R99 | 2.7 |  |  |  |  |  |  | C215 | C215 | C215 | C215 | C215 | C215 | C215 | C215 | E1/24 | UMTS System Simulator or System Simulator only |  |  |
|  | Steering of roaming, UTRAN |  |  | Rel-7 | 3.1 |  |  |  |  |  | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/24 AND E.1/236 | UMTS System Simulator only |  |  |
|  | Steering of roaming, InterRAT |  |  | Rel-7 | 3.2 |  |  |  |  |  | C167 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | C167 AND C184 | E.1/24 AND E.1/236 | UMTS System Simulator and System Simulator |  |  |
|  | Steering of roaming, E-UTRAN |  |  | Rel-8 | 3.3 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/24 AND AND E.1/135 AND E.1/236 | E-USS or NB-SS (See NOTE) |  |  |
|  | Refresh with AID, E-UTRAN or UTRAN |  |  | Rel-8 | 4.1 |  |  |  |  |  | C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | C202 OR C203 | E.1/24 | E-USS only or UMTS System Simulator |  |  |
|  | UICC Reset for IMSI Changing procedure, E-UTRAN |  |  | Rel-8 | 5.1 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/24 | E-USSor NB-SS (See NOTE) |  |  |
|  | 3G Session Reset for IMSI Changing procedure, E-UTRAN |  |  | Rel-8 | 5.2 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/24 | E-USSor NB-SS (See NOTE) |  |  |
| 11 | SET UP MENU 27.22.4.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Set up, menu selection, replace and remove menu |  |  | R99 | 1.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/30 AND E.1/4 AND E.1/110 AND E.1/111 | No |  |  |
|  | Large menu |  |  | R99 | 1.2 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/30 AND E.1/4 AND E.1/110 AND E.1/111 | No |  |  |
|  | help information |  |  | R99 | 2.1 | C107 AND C177 AND C178 | C107 AND C177 AND C178 |  | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | E.1/30 AND E.1/4 AND E.1/110 AND E.1/111 | No |  |  |
|  | next action indicator |  |  | R99 | 3.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/30 AND E.1/110 AND E.1/111 | No |  |  |
|  | Icons |  |  | R99 | 4.1, 4.2 | C172 AND C177 AND C178 | C172 AND C177 AND C178 |  | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | E.1/30 AND E.1/110 AND E.1/111 | No |  |  |
|  | soft key access |  |  | R99 | 5.1 | C112 AND C177 AND C178 | C112 AND C177 AND C178 |  | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | E.1/30 AND E.1/74 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 6.1 |  |  |  | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 6.2 |  |  |  | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 6.3 |  |  |  | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 6.4 |  |  |  | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 6.5 |  |  |  | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 6.6 |  |  |  | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 6.7 |  |  |  | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – underlined on |  |  | Rel-5 | 6.8 |  |  |  | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 6.9 |  |  |  | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 6.10 |  |  |  | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | E.1/30 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 7.1 |  |  |  | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 8.1 |  |  |  | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 9.1 |  |  |  | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
| 12 | SELECT ITEM 27.22.4.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mandatory features |  |  | R99 | 1.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | Large menu |  |  | R99 | 1.2, 1.3, 1.5,1.6 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | Backwards move |  |  | R99 | 1.4 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | user termination |  |  | R99 | 1.5 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | next action indicator |  |  | R99 | 2.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | default selected item |  |  | R99 | 3.1 | C177 AND C178 AND C194 | C177 AND C178 AND C194 |  | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | C177 AND C178 AND C194 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | help information |  |  | R99 | 4.1 | C107 AND C177 AND C178 | C107 AND C177 AND C178 |  | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | C107 AND C177 AND C178 | E 1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | Icons |  |  | R99 | 5.1, 5.2 | C172 AND C177 AND C178 | C172 AND C177 AND C178 |  | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | C172 AND C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | Presentation style |  |  | R99 | 6.1, 6.2 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | Soft keys |  |  | R99 | 7.1 | C112 AND C177 AND C178 | C112 AND C177 AND C178 |  | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | C112 AND C177 AND C178 | E.1/25 AND E.1/73 AND E.1/110 AND E.1/111 | No |  |  |
|  | No Response from user |  |  | R99 | 8.1 | C120 AND C177 AND C178 | C120 AND C177 AND C178 |  | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | C120 AND C177 AND C178 | E.1/25 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 9.1 |  |  |  | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | C153 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 9.2 |  |  |  | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | C154 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 9.3 |  |  |  | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | C155 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 9.4 |  |  |  | C157AND C156 AND C177 AND C178 | C157AND C156 AND C177 AND C178 | C157AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | C157 AND C156 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 9.5 |  |  |  | C158AND C156 AND C177 AND C178 | C158AND C156 AND C177 AND C178 | C158AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | C158 AND C156 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 9.6 |  |  |  | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | C160 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 9.7 |  |  |  | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | C161 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – underline on |  |  | Rel-5 | 9.8 |  |  |  | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | C162 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 9.9 |  |  |  | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | C163 AND C159 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | No |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 9.10 |  |  |  | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | C164 AND C165 AND C177 AND C178 | E.1/25 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 10.1, 10.2, 10.3 |  |  |  | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | C118 AND C177 AND C178 | E.1/39 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 11.1 |  |  |  | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | C143 AND C177 AND C178 | E.1/25 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 12.1, 12.2, 12.3 |  |  |  | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | C145 AND C177 AND C178 | E.1/25 AND E.1/15 AND E.1/110 AND E.1/111 | No |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/25 AND E.1/177 AND E.1/178 AND E.1/110 AND E.1/111 | TBD |  |  |
| 13 | SEND SMS 27.22.4.10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Void |  |  | R99 | 1.1 - 1.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Send Short Message over CS/PS, UTRAN/GERAN |  |  | R99 | 1.9 | C209 | C209 |  | C209 | C209 | C209 | C210 | C210 | C210 | C210 | C210 | C210 | C210 | C210 | E.1/26 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 SMS in Cyrillic |  |  | R99 | 2.1 | C118 | C118 |  | C118 | C118 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Icons – basic icon |  |  | R99 | 3.1, 3.2 | C108 | C108 |  | C108 | C108 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute– left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 | C153 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 | C154 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 | C155 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157AND C156 | C157AND C156 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158AND C156 | C158AND C156 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 | C160 AND C159 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 | C161 AND C159 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 | C162 AND C159 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute– strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 | C163 AND C159 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 | C164 AND C165 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Chinese |  |  | R99 | 5.1 |  |  |  | C143 | C143 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Katakana |  |  | R99 | 6.1 |  |  |  | C145 | C145 | NA | NA | NA | NA | NA | NA | NA | NA | NA | E.1/26 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | SMS-over-IP, E-UTRAN |  |  | Rel-8 | 7.1 |  |  |  |  |  |  | C196 | C196 | C196 | C196 | C196 | C196 | C196 | C196 | E.1/26 AND AND E.1/110 | E-USS only |  | TCEP001 |
|  | SMS-over-IP, UTRAN |  |  | Rel-7 | 7.2 |  |  |  |  |  | C197 | C197 | C197 | C197 | C197 | C197 | C197 | C197 | C197 | E.1/26 AND AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Send Short Message over SGs, E-UTRAN |  |  | Rel-8 | 8.1 |  |  |  |  |  |  | C206 | C206 | C206 | C206 | C206 | C220 | C220 | C220 | E.1/26 AND AND E.1/110 | E-USS or NB-SS (See NOTE) |  | TCEP001 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/26 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  | TCEP001 |
| 14 | SEND SS 27.22.4.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | call forward unconditional, all bearers, successful |  |  | R99 | 1.1 | C166 AND C174 AND C204 | C166 AND C174 AND C204 |  | C166 AND C174 AND C204 | C166 AND C174 AND C204 | C166 AND C174 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, Return Error |  |  | R99 | 1.2 | C174 AND C204 | C174 AND C204 |  | C174 AND C204 | C174 AND C204 | C174 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, Reject |  |  | R99 | 1.3 | C174 AND C204 | C174 AND C204 |  | C174 AND C204 | C174 AND C204 | C174 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, successful, SS request size limit |  |  | R99 | 1.4 | C166 AND C174 AND C204 | C166 AND C174 AND C204 |  | C166 AND C174 AND C204 | C166 AND C174 AND C204 | C166 AND C174 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | interrogate CLIR status, successful, alpha identifier limits |  |  | R99 | 1.5 | C175 AND C204 | C175 AND C204 |  | C175 AND C204 | C175 AND C204 | C175 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | C175 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, successful, null data alpha identifier |  |  | R99 | 1.6 | C166 AND C174 AND C204 | C166 AND C174 AND C204 |  | C166 AND C174 AND C204 | C166 AND C174 AND C204 | C166 AND C174 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, successful, basic icon support |  |  | R99 | 2.1, 2.3 | C108 AND C174 AND C204 | C108 AND C174 AND C204 |  | C108 AND C174 AND C204 | C108 AND C174 AND C204 | C108 AND C174 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | C108 AND C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, successful, colour icon support |  |  | R99 | 2.2 | C171 AND C174 AND C204 | C171 AND C174 AND C204 |  | C171 AND C174 AND C204 | C171 AND C174 AND C204 | C171 AND C174 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | C171 AND C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | call forward unconditional, all bearers, successful, basic icon non self-explanatory, no alpha identifier presented |  |  | R99 | 2.4 | C185 AND C174 AND C204 | C185 AND C174 AND C204 |  | C185 AND C174 AND C204 | C185 AND C174 AND C204 | C185 AND C174 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | C185 AND C174 AND C183 AND C204 | E.1/27 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Cyrillic |  |  | R99 | 3.1 | C118 AND C174 AND C204 | C118 AND C174 AND C204 |  | C118 AND C174 AND C204 | C118 AND C174 AND C204 | C118 AND C174 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | C118 AND C174 AND C183 AND C204 | E.1/27 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 AND C166 AND C174 AND C204 | C153 AND C166 AND C174 AND C204 | C153 AND C166 AND C174 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | C153 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 AND C166 AND C174 AND C204 | C154 AND C166 AND C174 AND C204 | C154 AND C166 AND C174 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | C154 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 AND C166 AND C174 AND C204 | C155 AND C166 AND C174 AND C204 | C155 AND C166 AND C174 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | C155 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157AND C156AND C166 AND C174 AND C204 | C157AND C156ANDC166 AND C174 AND C204 | C157AND C156ANDC166 AND C174 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | C157AND C156ANDC166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158AND C156 AND C166 AND C174 AND C204 | C158AND C156 AND C166 AND C174 AND C204 | C158AND C156 AND C166 AND C174 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | C158AND C156 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 AND C166 AND C174 AND C204 | C160 AND C159 AND C166 AND C174 AND C204 | C160 AND C159 AND C166 AND C174 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | C160 AND C159 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 AND C166 AND C174 AND C204 | C161 AND C159 AND C166 AND C174 AND C204 | C161 AND C159 AND C166 AND C174 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | C161 AND C159 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 AND C166 AND C174 AND C204 | C162 AND C159 AND C166 AND C174 AND C204 | C162 AND C159 AND C166 AND C174 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | C162 AND C159 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 AND C166 AND C174 AND C204 | C163 AND C159 AND C166 AND C174 AND C204 | C163 AND C159 AND C166 AND C174 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | C163 AND C159 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 AND C166 AND C174 AND C204 | C164 AND C165 AND C166 AND C174 AND C204 | C164 AND C165 AND C166 AND C174 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | C164 AND C165 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Chinese |  |  | R99 | 5.1 |  |  |  | C143 AND C166 AND C174 AND C204 | C143 AND C166 AND C174 AND C204 | C143 AND C166 AND C174 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | C143 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Katakana |  |  | R99 | 6.1 |  |  |  | C145 AND C166 AND C174 AND C204 | C145 AND C166 AND C174 AND C204 | C145 AND C166 AND C174 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | C145 AND C166 AND C174 AND C183 AND C204 | E.1/27 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
| 15 | SEND USSD 27.22.4.12 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 7-bit data, successful |  |  | R99 | 1.1 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | 8-bit data, successful |  |  | R99 | 1.2 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 data, successful |  |  | R99 | 1.3 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | 7-bit data, unsuccessful |  |  | R99 | 1.4 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | 7-bit data, unsuccessful |  |  | R99 | 1.5 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | 256 octets, 7-bit data, successful, long alpha identifier |  |  | R99 | 1.6 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | 7-bit data, successful, no alpha identifier |  |  | R99 | 1.7 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  |  |
|  | 7-bit data, successful, null length alpha identifier |  |  | R99 | 1.8 | C204 | C204 |  | C204 | C204 | C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Icons – basic icon |  |  | R99 | 2.1, 2.3 | C108 AND C204 | C108 AND C204 |  | C108 AND C204 | C108 AND C204 | C108 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | C108 AND C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Icons – colour icon |  |  | R99 | 2.2 | C186 AND C204 | C186 AND C204 |  | C186 AND C204 | C186 AND C204 | C186 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | C186 AND C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | 7-bit data, basic icon non self-explanatory, no alpha identifier presented |  |  | R99 | 2.4 | C187 AND C204 | C187 AND C204 |  | C187 AND C204 | C187 AND C204 | C187 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | C187 AND C183 AND C204 | E.1/28 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 in Cyrillic |  |  | R99 | 3.1 | C118 AND C204 | C118 AND C204 |  | C118 AND C204 | C118 AND C204 | C118 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | C118 AND C183 AND C204 | E.1/28 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 AND C204 | C153 AND C204 | C153 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | C153 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 AND C204 | C154 AND C204 | C154 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | C154 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 AND C204 | C155 AND C204 | C155 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | C155 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157AND C156 AND C204 | C157AND C156 AND C204 | C157AND C156 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | C157 AND C156 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158AND C156 AND C204 | C158AND C156 AND C204 | C158AND C156 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | C158 AND C156 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 AND C204 | C160 AND C159 AND C204 | C160 AND C159 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | C160 AND C159 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 AND C204 | C161 AND C159 AND C204 | C161 AND C159 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | C161 AND C159 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 AND C204 | C162 AND C159 AND C204 | C162 AND C159 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | C162 AND C159 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 AND C204 | C163 AND C159 AND C204 | C163 AND C159 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | C163 AND C159 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 AND C204 | C164 AND C165 AND C204 | C164 AND C165 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | C164 AND C165 AND C183 AND C204 | E.1/28 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 in Chinese |  |  | R99 | 5.1 |  |  |  | C143 AND C204 | C143 AND C204 | C143 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | C143 AND C183 AND C204 | E.1/28 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 in Katakana |  |  | R99 | 6.1 |  |  |  | C145 AND C204 | C145 AND C204 | C145 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | C145 AND C183 AND C204 | E.1/28 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
| 16 | SET UP CALL 27.22.4.13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Call confirmed by the user and connected |  |  | R99 | 1.1 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | call rejected by the user |  |  | R99 | 1.2 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/29 |  |  |  |
|  | putting all other calls on hold, ME busy |  |  | R99 | 1.4 | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 |  | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | disconnecting all other calls, ME busy |  |  | R99 | 1.5 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | only if not currently busy on another call, ME busy |  |  | R99 | 1.6 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | putting all other calls on hold, call hold is not allowed |  |  | R99 | 1.7 | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 |  | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | C170 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Capability configuration |  |  | R99 | 1.8 | C101 AND C177 AND C178 AND C180 | C101 AND C177 AND C178 AND C180 |  | C101 AND C177 AND C178 AND C180 | C101 AND C177 AND C178 AND C180 | C101 AND C177 AND C178 AND C180 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | C101 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | long dialling number string |  |  | R99 | 1.9 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | long first alpha identifier |  |  | R99 | 1.10 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Called party subaddress |  |  | R99 | 1.11 | C124 AND C177 AND C178 AND C180 | C124 AND C177 AND C178 AND C180 |  | C124 AND C177 AND C178 AND C180 | C124 AND C177 AND C178 AND C180 | C124 AND C177 AND C178 AND C180 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | C124 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | maximum duration for the redial mechanism |  |  | R99 | 1.12 | C119 AND C177 AND C178 AND C180 | C119 AND C177 AND C178 AND C180 |  | C119 AND C177 AND C178 AND C180 | C119 AND C177 AND C178 AND C180 | C119 AND C177 AND C178 AND C180 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | C119 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | second alpha identifier |  |  | R99 | 2.1 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/63 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Icons – basic icon |  |  | R99 | 3.1,3.2, 3.4 | C108 AND C177 AND C178 AND C180 | C108 AND C177 AND C178 AND C180 |  | C108 AND C177 AND C178 AND C180 | C108 AND C177 AND C178 AND C180 | C108 AND C177 AND C178 AND C180 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | C108 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Icons – colour icon |  |  | R99 | 3.3 | C171 AND C177 AND C178 AND C180 | C171 AND C177 AND C178 AND C180 |  | C171 AND C177 AND C178 AND C180 | C171 AND C177 AND C178 AND C180 | C171 AND C177 AND C178 AND C180 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | C171 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 AND C177 AND C178 AND C180 | C153 AND C177 AND C178 AND C180 | C153 AND C177 AND C178 AND C180 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | C153 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 AND C177 AND C178 AND C180 | C154 AND C177 AND C178 AND C180 | C154 AND C177 AND C178 AND C180 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | C154 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 AND C177 AND C178 AND C180 | C155 AND C177 AND C178 AND C180 | C155 AND C177 AND C178 AND C180 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | C155 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157AND C156 AND C177 AND C178 AND C180 | C157AND C156 AND C177 AND C178 AND C180 | C157AND C156 AND C177 AND C178 AND C180 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | C157AND C156 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158AND C156 AND C177 AND C178 AND C180 | C158AND C156 AND C177 AND C178 AND C180 | C158AND C156 AND C177 AND C178 AND C180 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | C158AND C156 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 AND C177 AND C178 AND C180 | C160 AND C159 AND C177 AND C178 AND C180 | C160 AND C159 AND C177 AND C178 AND C180 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | C160 AND C159 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 AND C177 AND C178 AND C180 | C161 AND C159 AND C177 AND C178 AND C180 | C161 AND C159 AND C177 AND C178 AND C180 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | C161 AND C159 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – underline on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 AND C177 AND C178 AND C180 | C162 AND C159 AND C177 AND C178 AND C180 | C162 AND C159 AND C177 AND C178 AND C180 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | C162 AND C159 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 AND C177 AND C178 AND C180 | C163 AND C159 AND C177 AND C178 AND C180 | C163 AND C159 AND C177 AND C178 AND C180 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | C163 AND C159 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 AND C177 AND C178 AND C180 | C164 AND C165 AND C177 AND C178 AND C180 | C164 AND C165 AND C177 AND C178 AND C180 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | C164 AND C165 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UCS2 Display in Cyrillic |  |  | R99 | 5.1, 5.2. |  |  |  | C118 AND C177 AND C178 AND C180 | C118 AND C177 AND C178 AND C180 | C118 AND C177 AND C178 AND C180 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | C118 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/15 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 6.1, 6.2 |  |  |  | C143 AND C177 AND C178 AND C180 | C143 AND C177 AND C178 AND C180 | C143 AND C177 AND C178 AND C180 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | C143 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/15 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 7.1, 7.2 |  |  |  | C145 AND C177 AND C178 AND C180 | C145 AND C177 AND C178 AND C180 | C145 AND C177 AND C178 AND C180 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | C145 AND C177 AND C178 AND C180 AND C183 | E.1/29 AND E.1/15 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/29 AND E.1/177 AND E.1/178 AND E.1/110 AND E.1/111 | TBD |  |  |
| 17 | POLLING OFF 27.22.4.14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | POLLING OFF |  |  | R99 | 1.1 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C183 | C183 | C183 | C183 | E.1/23 | UMTS System Simulator or System Simulator only |  |  |
|  | POLLING OFF, E-UTRAN |  |  | Rel-8 | 1.2 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/23 | E-USSor NB-SS (See NOTE) |  |  |
| 18 | PROVIDE LOCAL INFORMATION 27.22.4.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | location information |  |  | R99 | 1.1 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/31 | Yes |  | AER003 |
|  | IMEI |  |  | R99 | 1.2 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/31 | No |  |  |
|  | network measurement results and BCCH channel list |  |  | R99 | 1.3 | C167 | C167 |  | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | E.1/32 AND E.1/67 | System Simulator only |  |  |
|  | Date, time and time zone |  |  | R99 | 1.4 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/59 | No |  |  |
|  | language setting |  |  | R99 | 1.5 | C217 | C217 |  | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | C217 | E.1/68 | No |  |  |
|  | Timing advance |  |  | R99 | 1.6 | C167 | C167 |  | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | C167 | E.1/69 | System Simulator only |  |  |
|  | Access Technology |  |  | Rel-4 | 1.7 |  |  |  |  | M | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/72 | UMTS System Simulator only |  | AER004 |
|  | Void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | IMEISV |  |  | Rel-6 | 1.9 |  |  |  |  | M | M | M | M | M | M | M | M | M | M | E.1/143 | No |  |  |
|  | Network Search Mode |  |  | Rel-6 | 1.10 |  |  |  |  |  |  |  | M | M | M | M | M | M | M | E.1/144 | E-USS, UMTS System Simulator or System Simulator |  |  |
|  | Charge State of the Battery |  |  | Rel-6 | 1.11 |  |  |  |  | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | C139 | E.1/170 | No |  |  |
|  | Intra-frequency UTRAN measurements |  |  | Rel-6 | 1.12 |  |  |  |  | M | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/183 | UMTS System Simulator only |  |  |
|  | Inter-frequency UTRAN measurements |  |  | Rel-6 | 1.13 |  |  |  |  | M | M | C184 | C184 | C184 | C184 | C184 | C184 | C184 | C184 | E.1/183 | UMTS System Simulator only |  |  |
|  | Access Technology, E-UTRAN |  |  | Rel-8 | 1.14 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/72 | E-USS or NB-SS (See NOTE) |  |  |
|  | E-UTRAN Intra-Frequency Measurements |  |  | Rel-8 | 1.15 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C190 | C190 | C190 | E.1/183 | E-USS only |  |  |
|  | E-UTRAN Intrer-Frequency Measurements |  |  | Rel-8 | 1.16 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C190 | C190 | C190 | E.1/183 | E-USS only |  |  |
|  | E-UTRAN Local Info (MCC, MNC, TAC & E-UTRAN Cell ID) |  |  | Rel-8 | 1.17 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/31 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | Discovery of surrounding CSG cells |  |  | Rel-9 | 1.18 |  |  |  |  |  |  |  | C195 | C195 | C195 | C195 | C195 | C195 | C195 | E.1/242 | E-USS only |  |  |
|  | Location Information for multiple Access Technologies |  |  | Rel-8 | 1.19 |  |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |  |
|  | NMR for multiple Access Technologies |  |  | Rel-8 | 1.20 |  |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |  |
|  | Current access technologies, multiple Access Technologies |  |  | Rel-8 | 1.21 |  |  |  |  |  |  | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD | TBD |  |  |
|  | NG-RAN Local Info (MCC, MNC, TAC & NG-RAN Cell ID) |  |  | Rel-15 | 1.22 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/31 | NG-SS only |  |  |
|  | Access Technology, NG-RAN |  |  | Rel-15 | 1.23 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/72 | NG-SS only |  |  |
| 19 | SET UP EVENT LIST 27.22.4.16 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Set up call connected event |  |  | R99 | 1.1 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/33 AND E.1/35 | UMTS System Simulator or System Simulator only |  |  |
|  | Replace by new event list |  |  | R99 | 1.2 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/33 AND E.1/35 AND E.1/36 | UMTS System Simulator or System Simulator only |  |  |
|  | Remove event |  |  | R99 | 1.3 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/33 AND E.1/35 | UMTS System Simulator or System Simulator only |  |  |
|  | Remove Event on ME Power Cycle |  |  | R99 | 1.4 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/33 AND E.1/35 | UMTS System Simulator or System Simulator only |  |  |
| 20 | PERFORM CARD APDU 27.22.4.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Additional card inserted, Select MF and Get Response |  |  | R99 | 1.1 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/51 | No |  |  |
|  | Additional card inserted, Select DF GSM, Select EF PLMN , Update Binary, Read Binary on EF PLMN |  |  | R99 | 1.2 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/51 | No |  |  |
|  | Additional card inserted, card powered off |  |  | R99 | 1.3 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/51 | No |  |  |
|  | No card inserted, card powered off |  |  | R99 | 1.4 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/51 | No |  |  |
|  | Invalid card reader identifier |  |  | R99 | 1.5 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/51 | No |  |  |
|  | Detachable reader |  |  | R99 | 2.1 | C116 | C116 |  | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | E.1/51 | No |  |  |
| 21 | POWER OFF CARD 27.22.4.18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Additional card inserted |  |  | R99 | 1.1 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/50 | No |  |  |
|  | No card inserted |  |  | R99 | 1.2 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/50 | No |  |  |
|  | Detachable reader |  |  | R99 | 2.1 | C116 | C116 |  | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | E.1/50 | No |  |  |
| 22 | POWER ON CARD 27.22.4.19 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Additional card inserted |  |  | R99 | 1.1 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/49 | No |  |  |
|  | No ATR |  |  | R99 | 1.2 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/49 | No |  |  |
|  | No card inserted |  |  | R99 | 1.3 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/49 | No |  |  |
|  | Detachable reader |  |  | R99 | 2.1 | C116 | C116 |  | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | E.1/49 | No |  |  |
| 23 | GET READER STATUS 27.22.4.20 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Additional card inserted, card powered |  |  | R99 | 1.1 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/52 | No |  |  |
|  | Additional card inserted, card not powered |  |  | R99 | 1.2 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/52 | No |  |  |
|  | Additional card inserted, card not present |  |  | R99 | 1.3 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/52 | No |  |  |
|  | Detachable reader |  |  | R99 | 2.1 | C116 | C116 |  | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | E.1/52 | No |  |  |
| 24 | TIMER MANAGEMENT 27.22.4.21.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Start timer 1 several times, get the current value of the timer and deactivate the timer successfully |  |  | R99 | 1.1 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/57 AND E.1/58 | No |  |  |
|  | Start timer 2 several times, get the current value of the timer and deactivate the timer successfully |  |  | R99 | 1.2 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/57 AND E.1/58 | No |  |  |
|  | Start timer 8 several times, get the current value of the timer and deactivate the timer successfully |  |  | R99 | 1.3 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/57 AND E.1/58 | No |  |  |
|  | Try to get the current value of a timer which is not started: action in contradiction with the current timer state |  |  | R99 | 1.4 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/57 AND E.1/58 | No |  |  |
|  | Try to deactivate a timer which is not started: action in contradiction with the current timer state |  |  | R99 | 1.5 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/57 AND E.1/58 | No |  |  |
|  | Start 8 timers successfully |  |  | R99 | 1.6 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/57 AND E.1/58 | No |  |  |
| 25 | ENVELOPE TIMER EXPIRATION 27.22.4.21.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Pending proactive UICC command |  |  | R99 | 2.1 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/6 AND E.1/57 | No |  |  |
|  | USIM application toolkit busy |  |  | R99 | 2.2 | M | M |  | M | M | M | M | M | M | M | M | M | M | M | E.1/6 AND E.1/57 AND E.1/20 | No |  |  |
| 26 | SET UP IDLE MODE TEXT 27.22.4.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Display idle mode text |  |  | R99 | 1.1 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Replace idle mode text |  |  | R99 | 1.2 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Remove idle mode test |  |  | R99 | 1.3 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Competing information on ME display |  |  | R99 | 1.4 | C177 AND C179 AND C180 | C177 AND C179 AND C180 |  | C177 AND C179 AND C180 | C177 AND C179 AND C180 | C177 AND C179 AND C180 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | C177 AND C179 AND C180 AND C183 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/110 | UMTS System Simulator or System Simulator only |  |  |
|  | ME powered cycled |  |  | R99 | 1.5 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Refresh with USIM initialization |  |  | R99 | 1.6 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/61 AND E.1/24 AND E.1/33 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Large text string |  |  | R99 | 1.7 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Icons – basic icon |  |  | R99 | 2.1, 2.2 | C108 AND C177 | C108 AND C177 |  | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | C108 AND C177 | E.1/61 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Icons – colour icon |  |  | R99 | 2.3 | C171 AND C177 | C171 AND C177 |  | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | C171 AND C177 | E.1/61 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Icon is not self-explanatory, empty text string |  |  | R99 | 2.4 | C188 AND C177 | C188 AND C177 |  | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | C188 AND C177 | E.1/61 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 3.1 | C118 AND C177 | C118 AND C177 |  | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | C118 AND C177 | E.1/61 AND E.1/15 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | C153 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/217 AND E.1/110 | Yes |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | C154 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/218 AND E.1/110 | Yes |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | C155 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/219 AND E.1/110 | Yes |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157AND C156 AND C177 | C157AND C156 AND C177 | C157AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | C157 AND C156 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | Yes |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158AND C156 AND C177 | C158AND C156 AND C177 | C158AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | C158 AND C156 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | Yes |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | C160 AND C159 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | Yes |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | C161 AND C159 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | Yes |  |  |
|  | Text attribute – underline on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | C162 AND C159 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | Yes |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | C163 AND C159 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | Yes |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | C164 AND C165 AND C177 | E.1/61 AND E.1/33 AND E.1/39 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | Yes |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 5.1 |  |  |  | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | C143 AND C177 | E.1/61 AND E.1/15 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 6.1 |  |  |  | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | C145 AND C177 | E.1/61 AND E.1/15 AND E.1/39 AND E.1/110 | Yes |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/61 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
| 27 | RUN AT COMMAND 27.22.4.23 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No alpha Identifier |  |  | R99 | 1.1 | C110 | C110 |  | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | E.1/62 | No |  |  |
|  | null data alpha identifier presented |  |  | R99 | 1.2 | C110 | C110 |  | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | C110 | E.1/62 | No |  |  |
|  | alpha identifier presented |  |  | R99 | 1.3 | C110 AND C177 | C110 AND C177 |  | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | C110 AND C177 | E.1/62 AND E.1/110 | No |  |  |
|  | Icons – basic icon |  |  | R99 | 2.1, 2.3 | C114 AND C177 | C114 AND C177 |  | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | C114 AND C177 | E.1/62 AND E.1/110 | No |  |  |
|  | Icons – colour icon |  |  | R99 | 2.2, 2.4, | C173 AND C177 | C173 AND C177 |  | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | C173 AND C177 | E.1/62 AND E.1/110 | No |  |  |
|  | basic icon non self-explanatory, no alpha identifier presented |  |  | R99 | 2.5 | C189 AND C177 | C189 AND C177 |  | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | C189 AND C177 | E.1/62 AND E.1/110 | No |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 3.1 |  |  |  | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | C110 AND C153 AND C177 | E.1/62 AND E.1/124 AND E.1/217 AND E.1/110 | No |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 3.2 |  |  |  | C110 ANDC154 AND C177 | C110 ANDC154 AND C177 | C110 ANDC154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | C110 AND C154 AND C177 | E.1/62 AND E.1/124 AND E.1/218 AND E.1/110 | No |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 3.3 |  |  |  | C110 ANDC155 AND C177 | C110 ANDC155 AND C177 | C110 ANDC155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | C110 AND C155 AND C177 | E.1/62 AND E.1/124 AND E.1/219 AND E.1/110 | No |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 3.4 |  |  |  | C110 ANDC157AND C156 AND C177 | C110 ANDC157AND C156 AND C177 | C110 ANDC157AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | C110 AND C157 AND C156 AND C177 | E.1/62 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | No |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 3.5 |  |  |  | C110 ANDC158AND C156 AND C177 | C110 ANDC158AND C156 AND C177 | C110 ANDC158AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | C110 AND C158 AND C156 AND C177 | E.1/62 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | No |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 3.6 |  |  |  | C110 ANDC160 AND C159 AND C177 | C110 ANDC160 AND C159 AND C177 | C110 ANDC160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | C110 AND C160 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | No |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 3.7 |  |  |  | C110 ANDC161 AND C159 AND C177 | C110 ANDC161 AND C159 AND C177 | C110 ANDC161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | C110 AND C161 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | No |  |  |
|  | Text attribute – underline on |  |  | Rel-5 | 3.8 |  |  |  | C110 ANDC162 AND C159 AND C177 | C110 ANDC162 AND C159 AND C177 | C110 ANDC162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | C110 AND C162 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | No |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 3.9 |  |  |  | C110 ANDC163 AND C159 AND C177 | C110 ANDC163 AND C159 AND C177 | C110 ANDC163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | C110 AND C163 AND C159 AND C177 | E.1/62 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | No |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 3.10 |  |  |  | C110 ANDC164 AND C165 AND C177 | C110 ANDC164 AND C165 AND C177 | C110 ANDC164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | C110 AND C164 AND C165 AND C177 | E.1/62 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | No |  |  |
|  | UCS2 Display in Cyrillic |  |  | R99 | 4.1 |  |  |  | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | C149 AND C177 | E.1/62 AND E.1/15 AND E.1/110 | No |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 5.1 |  |  |  | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | C150 AND C177 | E.1/62 AND E.1/15 AND E.1/110 | No |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 6.1 |  |  |  | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | C151 AND C177 | E.1/62 AND E.1/15 AND E.1/110 | No |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/62 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
| 28 | SEND DTMF 27.22.4.24 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Normal |  |  | R99 | 1.1 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/66 | UMTS System Simulator or System Simulator only |  |  |
|  | alpha identifier |  |  | R99 | 1.2, 1.3 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/66 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Mobile is not in a speech call |  |  | R99 | 1.4 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/66 | UMTS System Simulator or System Simulator only |  |  |
|  | Icons – basic icon |  |  | R99 | 2.1, 2.3 | C108 AND C180 | C108 AND C180 |  | C108 AND C180 | C108 AND C180 | C108 AND C180 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | C108 AND C180 AND C183 | E.1/66 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Icons – colour icon |  |  | R99 | 2.2 | C171 AND C180 | C171 AND C180 |  | C171 AND C180 | C171 AND C180 | C171 AND C180 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | C171 AND C180 AND C183 | E.1/66 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Cyrillic |  |  | R99 | 3.1 | C118 AND C180 | C118 AND C180 |  | C118 AND C180 | C118 AND C180 | C118 AND C180 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | C118 AND C180 AND C183 | E.1/66 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – left alignment |  |  | Rel-5 | 4.1 |  |  |  | C153 AND C180 | C153 AND C180 | C153 AND C180 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | C153 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 4.2 |  |  |  | C154 AND C180 | C154 AND C180 | C154 AND C180 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | C154 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 4.3 |  |  |  | C155 AND C180 | C155 AND C180 | C155 AND C180 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | C155 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 4.4 |  |  |  | C157AND C156 AND C180 | C157AND C156 AND C180 | C157AND C156 AND C180 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | C157 AND C156 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 4.5 |  |  |  | C158AND C156 AND C180 | C158AND C156 AND C180 | C158AND C156 AND C180 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | C158 AND C156 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 4.6 |  |  |  | C160 AND C159 AND C180 | C160 AND C159 AND C180 | C160 AND C159 AND C180 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | C160 AND C159 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 4.7 |  |  |  | C161 AND C159 AND C180 | C161 AND C159 AND C180 | C161 AND C159 AND C180 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | C161 AND C159 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 4.8 |  |  |  | C162 AND C159 AND C180 | C162 AND C159 AND C180 | C162 AND C159 AND C180 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | C162 AND C159 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 4.9 |  |  |  | C163 AND C159 AND C180 | C163 AND C159 AND C180 | C163 AND C159 AND C180 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | C163 AND C159 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 4.10 |  |  |  | C164 AND C165 AND C180 | C164 AND C165 AND C180 | C164 AND C165 AND C180 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | C164 AND C165 AND C180 AND C183 | E.1/66 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Chinese |  |  | R99 | 5.1 |  |  |  | C143 AND C180 | C143 AND C180 | C143 AND C180 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | C143 AND C180 AND C183 | E.1/66 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | UCS2 display in Katakana |  |  | R99 | 6.1 |  |  |  | C145 AND C180 | C145 AND C180 | C145 AND C180 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | C145 AND C180 AND C183 | E.1/66 AND E.1/15 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/66 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
| 29 | LANGUAGE NOTIFICATION 27.22.4.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Specific language notification |  |  | R99 | 1.1 | C181 AND C218 | C181 AND C218 |  | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | E.1/70 | No |  |  |
|  | Non specific language notification |  |  | R99 | 1.2 | C181 AND C218 | C181 AND C218 |  | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | C181 AND C218 | E.1/70 | No |  |  |
| 30 | LAUNCH BROWSER 27.22.4.26 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | No session already launched: Connect to the default URL |  |  | R99 | 1.1 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 |  | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | C111 AND C177 AND C178 AND C213 | E.1/71 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | connect to the specified URL, alpha identifier length=0 |  |  | R99 | 1.2 | C111 AND C177 AND C178 | C111 AND C177 AND C178 |  | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | E.1/71 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Browser identity, no alpha identifier |  |  | R99 | 1.3 | C111 AND C177 AND C178 | C111 AND C177 AND C178 |  | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | E.1/71 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | one bearer specified and gateway/proxy identity |  |  | R99 | 1.4 | C122 AND C177 AND C178 | C122 AND C177 AND C178 |  | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | C122 AND C177 AND C178 | E.1/71 AND E.1/98 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | void |  |  | R99 | 1.5 | Void | Void |  | Void | Void | Void | Void | Void | Void | Void | Void | Void | Void | Void | void |  |  |  |
|  | ME does not support Launch Browser with Default URL |  |  | R99 | 1.6 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 |  | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | C111 AND C177 AND C178 AND C214 | E.1/71 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Interaction with current session |  |  | R99 | 2.1, 2.2, 2.3 | C111 AND C177 AND C178 | C111 AND C177 AND C178 |  | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | C111 AND C177 AND C178 | E.1/71 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | UCS2 display in Cyrillic |  |  | R99 | 3.1 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 |  | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | C111 AND C118 AND C177 AND C178 | E.1/71 AND E.1/15 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Icons – basic icon |  |  | R99 | 4.1, 4.2 | C115 AND C177 AND C178 | C115 AND C177 AND C178 |  | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | C115 AND C177 AND C178 | E.1/71 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 5.1 |  |  |  | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | C111 AND C153 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – center alignment |  |  | Rel-5 | 5.2 |  |  |  | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | C111 AND C154 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – right alignment |  |  | Rel-5 | 5.3 |  |  |  | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | C111 AND C155 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – large font size |  |  | Rel-5 | 5.4 |  |  |  | C111 AND C157AND C156 AND C177 AND C178 | C111 AND C157AND C156 AND C177 AND C178 | C111 AND C157AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | C111 AND C157 AND C156 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – small font size |  |  | Rel-5 | 5.5 |  |  |  | C111 AND C158AND C156 AND C177 AND C178 | C111 AND C158AND C156 AND C177 AND C178 | C111 AND C158AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | C111 AND C158 AND C156 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – bold on |  |  | Rel-5 | 5.6 |  |  |  | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | C111 AND C160 AND C159 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – italic on |  |  | Rel-5 | 5.7 |  |  |  | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | C111 AND C161 AND C159 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – underline on |  |  | Rel-5 | 5.8 |  |  |  | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | C111 AND C162 AND C159 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 5.9 |  |  |  | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | C111 AND C163 AND C159 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 5.10 |  |  |  | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | C111 AND C164 AND C165 AND C177 AND C178 | E.1/71 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | UCS2 display in Chinese |  |  | R99 | 6.1 |  |  |  | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | C111 AND C143 AND C177 AND C178 | E.1/71 AND E.1/15 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | UCS2 display in Katakana |  |  | R99 | 7.1 |  |  |  | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | C111 AND C145 AND C177 AND C178 | E.1/71 AND E.1/15 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/71 AND E.1/177 AND E.1/178 AND E.1/110 AND E.1/111 | TBD |  |  |
| 31 | OPEN CHANNEL 27.22.4.27 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | void |  |  | R99 | void | void | void |  | void | void | void | void | void | void | void | void | void | void | void | void |  |  |  |
|  | void |  |  | R99 | 2.1 | void | void |  | void | void | void | void | void | void | void | void | void | void | void | void |  |  |  |
|  | immediate link establishment GPRS, no alpha identifier, with network access name |  |  | R99 | 2.2 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/98 | UMTS System Simulator or System Simulator only |  | AER006 |
|  | immediate link establishment, GPRS, with alpha identifier |  |  | R99 | 2.3 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/98 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002, AER005 |
|  | immediate link establishment, GPRS, with null alpha identifier |  |  | R99 | 2.4 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/98 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | immediate link establishment, GPRS, command performed with modifications (buffer size) |  |  | R99 | 2.5 | C152 | C152 |  | C152 | C152 | C152 | C152 AND C183 | C152 AND C183 | C152 AND C183 | C152 AND C183 | C152 AND C183 | C152 AND C183 | C152 AND C183 | C152 AND C183 | E.1/89 AND E.1/98 | UMTS System Simulator or System Simulator only |  |  |
|  | void |  |  | void | 2.6 | Void | void |  | void | void | void | Void | Void | Void | Void | Void | Void | Void | Void | void |  |  |  |
|  | immediate link establishment, GPRS, open command with alpha identifier, User did not accept the proactive command |  |  | R99 | 2.7 | C169 AND C177 | C169 AND C177 |  | C169 AND C177 | C169 AND C177 | C169 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | C169 AND C183 AND C177 | E.1/89 AND E.1/98 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002, AER007 |
|  | void |  |  | void | 2.8 | Void | void |  | void | void | void | Void | Void | Void | Void | Void | Void | Void | Void | void |  |  |  |
|  | OPEN CHANNEL, immediate link establishment, no alpha identifier, with network access name |  |  | R99 | 2.9 |  |  |  |  |  |  |  | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | E.1/89 AND E.1/98 AND E.1/129 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Multi OPEN CHANNEL, one in TCP Server mode and one in TCP Client mode. |  |  | Rel-7 | 2.10 |  |  |  |  |  |  |  | C192 AND C183 | C192 AND C183 | C192 AND C183 | C192 AND C183 | C192 AND C183 | C192 AND C183 | C192 AND C183 | E.1/89 AND E.1/98 AND E.1/129 AND E.1/131 | UMTS System Simulator or System Simulator only |  |  |
|  | OPEN CHANNEL, Default Bearer, GPRS, with null alpha identifier) |  |  | R99 | 3.1 | C191 | C191 |  | C191 | C191 | C191 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | C191 AND C183 | E.1/89 AND E.1/98 AND C129 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Local Bearer |  |  | Rel-4 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/89 AND E.1/98 AND C132 | TBD |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 5.1 |  |  |  | C121 AND C153 | C121 AND C153 | C121 AND C153 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/217 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – center alignment |  |  | Rel-5 | 5.2 |  |  |  | C121 AND C154 | C121 AND C154 | C121 AND C154 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/218 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – right alignment |  |  | Rel-5 | 5.3 |  |  |  | C121 AND C155 | C121 AND C155 | C121 AND C155 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/219 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – large font size |  |  | Rel-5 | 5.4 |  |  |  | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – small font size |  |  | Rel-5 | 5.5 |  |  |  | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – bold on |  |  | Rel-5 | 5.6 |  |  |  | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – italic on |  |  | Rel-5 | 5.7 |  |  |  | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – underline on |  |  | Rel-5 | 5.8 |  |  |  | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 5.9 |  |  |  | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 5.10 |  |  |  | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | E.1/89 AND E.1/98 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  | TCEP001, TCEP002 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/89 AND E.1/98 AND E.1/177 AND E.1/178 AND E.1/110 AND E.1/111 | TBD |  |  |
|  | Immediate link establishment, E-UTRAN, bearer type '02' |  |  | Rel-8 | 6.1 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | Immediate link establishment, E-UTRAN, bearer type '0B' |  |  | Rel-8 | 6.2 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C182 | C182 | C182 | E.1/89 AND E.1/135 | E-USS only |  |  |
|  | Immediate link establishment, E-UTRAN, bearer type '02', with Network Access Name, with alpha identifier |  |  | Rel-8 | 6.3 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/110 AND E.1/111 AND E.1/135 | E-USS or NB-SS (See NOTE) |  | TCEP001, TCEP002 |
|  | Immediate link establishment, E-UTRAN, bearer type '03', with alpha identifier, user did not accept the proactive command |  |  | Rel-8 | 6.4 |  |  |  |  |  |  | C182 AND C177 | C182 AND C177 | C182 AND C177 | C182 AND C177 | C182 AND C177 | C223 AND C177 | C223 AND C177 | C223 AND C177 | E.1/89 AND E.1/110 AND E.1/111 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | Immediate link establishment, E-UTRAN, bearer type '03', default EPS bearer |  |  | Rel-8 | 6.5 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C223 | C223 | C223 | E.1/89 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | OPEN CHANNEL, BIP is not a 3GPP PS data off exempt service |  |  | Rel-14 | 6.6 |  |  |  |  |  |  |  |  |  |  |  |  | C228 | C228 | E.1/2 AND E.1/89 AND E.1/135 | E-USS |  |  |
|  | OPEN CHANNEL, BIP is a 3GPP PS data off exempt service |  |  | Rel-14 | 6.7 |  |  |  |  |  |  |  |  |  |  |  |  | C228 | C228 | E.1/2 AND E.1/89 AND E.1/135 | E-USS |  |  |
|  | OPEN CHANNEL,Maximum number of open channel requests |  |  | Rel-14 | 6.8 |  |  |  |  |  |  | C230 | C230 | C230 | C230 | C230 | C230 | C230 | C230 | E.1/89 AND E.1/135 | E-USS or NB-SS(See NOTE) |  |  |
|  | OPEN CHANNEL for IMS, IARI list stored on the USIM |  |  | Rel-10 | 7.1 |  |  |  |  |  |  |  |  | C207 | C207 | C207 | C207 | C207 | C207 | E.1/33 AND E.1/89 AND E.1/247 AND E.1/249 | UMTS System Simulator OR E-USS |  |  |
| 32 | CLOSE CHANNEL 27.22.4.28 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | successful |  |  | R99 | 1.1 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/90 | UMTS System Simulator or System Simulator only |  |  |
|  | with an invalid channel identifier |  |  | R99 | 1.2 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/90 | UMTS System Simulator or System Simulator only |  |  |
|  | on an already closed channel |  |  | R99 | 1.3 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/90 | UMTS System Simulator or System Simulator only |  |  |
|  | Text attribute – left alignment |  |  | Rel-5 | 2.1 |  |  |  | C121 AND C153 | C121 AND C153 | C121 AND C153 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 2.2 |  |  |  | C121 AND C154 | C121 AND C154 | C121 AND C154 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 2.3 |  |  |  | C121 AND C155 | C121 AND C155 | C121 AND C155 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 2.4 |  |  |  | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 2.5 |  |  |  | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 2.6 |  |  |  | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 2.7 |  |  |  | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 2.8 |  |  |  | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 2.9 |  |  |  | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – foreground and background colours |  |  | Rel-5 | 2.10 |  |  |  | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | E.1/89 AND E.1/90 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/89 AND E.1/98 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
|  | Default EPS bearer, successful |  |  | Rel-8 | 3.1 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/90 | E-USS or NB-SS (See NOTE) |  |  |
|  | EPS bearer with APN different from default APN, successful |  |  | Rel-8 | 3.2 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/90 | E-USS or NB-SS (See NOTE) |  | TCEP001, TCEP002 |
|  | CLOSE CHANNEL, Command qualifier set to 1 |  |  |  | 3.3 |  |  |  |  |  |  | C230 | C230 | C230 | C230 | C230 | C230 | C230 | C230 | E.1/89 AND E.1/90 | E-USS or NB-SS (See NOTE) |  |  |
| 33 | RECEIVE DATA 27.22.4.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | already opened channel |  |  | R99 | 1.1 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/91 AND E.1/92 | UMTS System Simulator or System Simulator only |  | AER008 |
|  | Text attribute – left alignment |  |  | Rel-5 | 2.1 |  |  |  | C121 AND C153 | C121 AND C153 | C121 AND C153 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | E.1/89 AND E.1/91 AND E.1/92 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 2.2 |  |  |  | C121 AND C154 | C121 AND C154 | C121 AND C154 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 2.3 |  |  |  | C121 AND C155 | C121 AND C155 | C121 AND C155 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 2.4 |  |  |  | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 2.5 |  |  |  | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 2.6 |  |  |  | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 2.7 |  |  |  | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 2.8 |  |  |  | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 2.9 |  |  |  | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute– foreground and background colours |  |  | Rel-5 | 2.10 |  |  |  | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | E.1/89 AND E.1/91 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/89 AND E.1/91 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
|  | Already opened channel – E-UTRAN, APN different from default |  |  | Rel-8 | 1.2 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/91 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
| 34 | SEND DATA 27.22.4.30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | immediate mode |  |  | R99 | 1.1 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/92 | UMTS System Simulator or System Simulator only |  |  |
|  | Store mode |  |  | R99 | 1.2 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/92 | UMTS System Simulator or System Simulator only |  |  |
|  | Store mode, Tx buffer fully used |  |  | R99 | 1.3 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/92 | UMTS System Simulator or System Simulator only |  |  |
|  | 2 consecutive SEND DATA Store mode |  |  | R99 | 1.4 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/92 | UMTS System Simulator or System Simulator only |  |  |
|  | immediate mode with a bad channel identifier |  |  | R99 | 1.5 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/92 | UMTS System Simulator or System Simulator only |  |  |
|  | void |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Text attribute– left alignment |  |  | Rel-5 | 2.1 |  |  |  | C121 AND C153 | C121 AND C153 | C121 AND C153 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | C121 AND C153 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/217 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – center alignment |  |  | Rel-5 | 2.2 |  |  |  | C121 AND C154 | C121 AND C154 | C121 AND C154 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | C121 AND C154 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/218 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – right alignment |  |  | Rel-5 | 2.3 |  |  |  | C121 AND C155 | C121 AND C155 | C121 AND C155 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | C121 AND C155 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/219 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – large font size |  |  | Rel-5 | 2.4 |  |  |  | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157AND C156 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | C121 AND C157 AND C156 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/221 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – small font size |  |  | Rel-5 | 2.5 |  |  |  | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158AND C156 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | C121 AND C158 AND C156 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/222 AND E.1/220 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – bold on |  |  | Rel-5 | 2.6 |  |  |  | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | C121 AND C160 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/226 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – italic on |  |  | Rel-5 | 2.7 |  |  |  | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | C121 AND C161 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/227 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – underline on |  |  | Rel-5 | 2.8 |  |  |  | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | C121 AND C162 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/228 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute – strikethrough on |  |  | Rel-5 | 2.9 |  |  |  | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | C121 AND C163 AND C159 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/225 AND E.1/229 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Text attribute– foreground and background colours |  |  | Rel-5 | 2.10 |  |  |  | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | C121 AND C164 AND C165 AND C183 | E.1/89 AND E.1/92 AND E.1/124 AND E.1/230 AND E.1/231 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Frames |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/89 AND E.1/92 AND E.1/177 AND E.1/178 AND E.1/110 | TBD |  |  |
|  | Immediate mode – E-UTRAN, Default EPS bearer |  |  | Rel-8 | 3.1 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C223 | C223 | C223 | E.1/89 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
|  | Store mode – E-UTRAN, APN different from default APN |  |  | Rel-8 | 3.2 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/92 | E-USS or NB-SS (See NOTE) |  |  |
| 35 | GET CHANNEL STATUS 27.22.4.31 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | without any BIP channel opened |  |  | R99 | 1.1 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/93 | UMTS System Simulator or System Simulator only |  |  |
|  | with a BIP channel currently opened |  |  | R99 | 1.2 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/93 | UMTS System Simulator or System Simulator only |  |  |
|  | after a link dropped |  |  | R99 | 1.3 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/89 AND E.1/93 | UMTS System Simulator or System Simulator only |  |  |
|  | EPS bearer with APN different from default APN |  |  | Rel-8 | 1.4 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/93 | E-USS or NB-SS (See NOTE) |  |  |
|  | EPS bearer with APN different from default APN, after a link dropped |  |  | Rel-8 | 1.5 |  |  |  |  |  |  | C182 | C182 | C182 | C182 | C182 | C224 | C224 | C224 | E.1/89 AND E.1/93 | E-USS or NB-SS (See NOTE) |  |  |
| 36 | DATA DOWNLOAD TO UICC 27.22.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 37 | SMS-PP DATA DOWNLOAD 27.22.5.1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | void |  |  |  | 1.1 - 1.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SMS-PP Data Download over CS, UTRAN/GERAN |  |  | R99 | 1.9 | C211 | C211 |  | C211 | C211 | C211 | C212 | C212 | C212 | C212 | C212 | C212 | C212 | C212 | E.1/2 | UMTS System Simulator or System Simulator |  | TCEP001 |
| 38 | CELL BROADCAST DATA DOWNLOAD 27.22.5.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cell Broadcast(GSM) - ME does not display message |  |  | R99 | 1.1 | C201 | C201 |  | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | E.1/3 | System Simulator only |  |  |
|  | void |  |  |  | 1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Cell Broadcast(GSM) - ME displays message |  |  | R99 | 1.3 | C201 AND C177 | C201AND C177 |  | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | C201AND C177 | E.1/3 AND E.1/110 | System Simulator only |  |  |
|  | Cell Broadcast (UTRAN) - ME does not display message |  |  | Rel-5 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/3 | UMTS System Simulator only |  |  |
|  | Cell Broadcast (UTRAN) -More time |  |  | Rel-5 | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/3 AND E.1/20 | UMTS System Simulator only |  |  |
|  | Cell Broadcast(UTRAN) - ME displays message |  |  | Rel-5 | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/3 | UMTS System Simulator only |  |  |
|  | Cell Broadcast(GSM) - More time (UDH) |  |  | R99 | 1.7 | C201 | C201 |  | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | C201 | E.1/3 AND E.1/20 | System Simulator only |  |  |
| 38A | SMS-PP DATA DOWNLOAD 27.22.5.3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SMS-PP Data Download over IMS, E-UTRAN |  |  | Rel-8 | 3.1 |  |  |  |  |  |  | C198 | C198 | C198 | C198 | C198 | C198 | C198 | C198 | E.1/2 | E-USS only |  | TCEP001 |
|  | SMS-PP Data Download over IMS, UTRAN |  |  | Rel-7 | 3.2 |  |  |  |  |  | C199 | C199 | C199 | C199 | C199 | C199 | C199 | C199 | C199 | E.1/2 | UMTS System Simulator only |  | TCEP001 |
| 38B | SMS-PP DATA DOWNLOAD over SGs in E-UTRAN 27.22.5.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | SMS-PP Data Download over SGs, E-UTRAN |  |  | Rel-8 | 4.1 |  |  |  |  |  |  | C205 | C205 | C205 | C205 | C205 | C221 | C221 | C221 | E.1/2 | E-USS or NB-SS (See NOTE) |  | TCEP001 |
| 39 | CALL CONTROL BY USIM 27.22.6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Procedure for MO calls (Cell identity in envelope call control) |  |  | R99 | 1.1, 1.2, 1.4, 1.6, 1.8 to 1.14 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/29 AND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
|  | Procedure for MO calls (Cell identity in envelope call control) |  |  | R99 | 1.3 A, 1.5 A, 1.7 A | C140 AND C177 AND C178 AND C180 | C140 AND C177 AND C178 AND C180 |  | C140 AND C177 AND C178 AND C180 | C140 AND C177 AND C178 AND C180 | C140 AND C177 AND C178 AND C180 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | C140 AND C177 AND C178 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/29 AND E.1/64 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Procedure for MO calls (Cell identity in envelope call control) |  |  | R99 | 1.3 B, 1.7 B | C141 AND C177 AND C178 AND C180 | C141 AND C177 AND C178 AND C180 |  | C141 AND C177 AND C178 AND C180 | C141 AND C177 AND C178 AND C180 | C141 AND C177 AND C178 AND C180 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | C141 AND C177 AND C178 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/29 AND E.1/64 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | Procedure for MO calls (Cell identity in envelope call control) |  |  | R99 | 1.5 B | C141 AND C180 | C141 AND C180 |  | C141 AND C180 | C141 AND C180 | C141 AND C180 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | C141 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/29 AND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
|  | Procedure for SS (Cell identity in envelope call control) |  |  | R99 | 2.1, 2.2, 2.3, 2.4 | C174 | C174 |  | C174 | C174 | C174 | C174 AND C183 | C174 AND C183 | C174 AND C183 | C174 AND C183 | C174 AND C183 | C174 AND C183 | C174 AND C183 | C174 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
|  | Interaction with FDN (Cell identity in envelope call control) |  |  | R99 | 3.1, 3.2, 3.3, 3.4, 3.5 | C146 AND C180 | C146 AND C180 |  | C146 AND C180 | C146 AND C180 | C146 AND C180 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | C146 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
|  | BDN service enabled |  |  | R99 | 4.1 | C147 AND C177 AND C178 AND C180 | C147 AND C177 AND C178 AND C180 |  | C147 AND C177 AND C178 AND C180 | C147 AND C177 AND C178 AND C180 | C147 AND C177 AND C178 AND C180 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | C147 AND C177 AND C178 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | BDN service enabled, interaction with emergency call codes, R99 only |  |  | R99 | 4.2A | C147 AND C180 |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
|  | BDN service enabled, interaction with emergency call codes, Rel-4+ |  |  | Rel-4 | 4.2B |  | C147 AND C180 |  | C147 AND C180 | C147 AND C180 | C147 AND C180 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | C147 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 ND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
|  | FDN and BDN enabled, set up a call in EFFDN, Allowed with modifications |  |  | R99 | 4.3 | C146 AND C147 AND C177 AND C180 | C146 AND C147 AND C177 AND C180 |  | C146 AND C147 AND C177 AND C180 | C146 AND C147 AND C177 AND C180 | C146 AND C147 AND C177 AND C180 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | C146 AND C147 AND C177 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/110 | UMTS System Simulator or System Simulator only |  |  |
|  | Call control on GPRS |  |  | Rel-5 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/98 AND E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 | TBD |  |  |
|  | BDN service enabled, ME not supporting BDN |  |  | R99 | 5.1 |  |  |  | C176 AND C180 | C176 AND C180 | C176 AND C180 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | C176 AND C180 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | UMTS System Simulator or System Simulator only |  |  |
| 40 | EVENT DOWNLOAD 27.22.7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 27.22.7.1: MT call event |  |  | R99 | 1.1 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/34 AND E.1/33 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.2.1: call connected event |  |  | R99 | 1.1 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/35 AND E.1/33 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.2.1: call connected event (simultaneous call MT-MO) |  |  | R12 | 1.2 |  |  |  |  |  |  |  |  |  |  | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 |  | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.2.1: call connected event (simultaneous call MO-MO) |  |  | R12 | 1.3 |  |  |  |  |  |  |  |  |  |  | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 |  | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.2.1: call connected event (simultaneous call MO-MT) |  |  | R12 | 1.4 |  |  |  |  |  |  |  |  |  |  | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 |  | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.2.2: ME supporting SET UP CALL |  |  | R99 | 2.1 | C177 AND C178 AND C180 | C177 AND C178 AND C180 |  | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | C177 AND C178 AND C180 AND C183 | E.1/35 AND E.1/29 AND E.1/33 AND E.1/110 AND E.1/111 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.3: call disconnected event |  |  | R99 | 1.1 | C180 | C180 |  | C180 | C180 | C180 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | C180 AND C183 | E.1/36 AND E.1/33 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.4: location status event |  |  | R99 | 1.1 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E.1/37 AND E.1/33 | UMTS System Simulator or System Simulator only |  | AER002 |
|  | 27.22.7.4: location status event, E-UTRAN |  |  | Rel-8 | 1.2 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/37 AND E.1/33 AND E.1/135 | E-USS or NB-SS (See NOTE) |  |  |
|  | 27.22.7.4: location status event, NG-RAN |  |  | Rel-15 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/37 AND E.1/33 | NG-SS only |  |  |
|  | ##### 27.22.4.27.8 Open Channel (related to NG-RAN) |  |  | Rel-15 | 8.1, 8.2, 8.3, 8.4 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/89 AND E.1/281 | NG-SS only |  |  |
|  | 27.22.7.5: user activity event |  |  | R99 | 1.1 | C178 | C178 |  | C178 | C178 | C178 | C178 | C178 | C178 | C178 | C178 | C178 | C178 | C178 | E.1/38 AND E.1/33 AND E.1/111 | No |  |  |
|  | 27.22.7.6: idle screen available event |  |  | R99 | 1.1 | C177 AND C178 | C177 AND C178 |  | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | C177 AND C178 | E.1/39 AND E.1/33 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | 27.22.7.7.1: Card reader status normal |  |  | R99 | 1.1 | C109 | C109 |  | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | C109 | E.1/40 AND E.1/33 | No |  |  |
|  | 27.22.7.7.2: Detachable card reader |  |  | R99 | 2.1 | C116 | C116 |  | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | C116 | E.1/40 AND E.1/33 | No |  |  |
|  | 27.22.7.8: language selection event |  |  | R99 | 1.1 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 |  | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | C177 AND C178 AND C181 AND C216 | E.1/41 AND E.1/33 AND E.1/110 AND E.1/111 | No |  |  |
|  | 27.22.7.9: Browser termination event |  |  | R99 | 1.1 | C193 AND C177 AND C178 | C193 AND C177 AND C178 |  | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | C193 AND C177 AND C178 | E.1/42 AND E.1/33 AND E.1/110 AND E.1/111 | Yes |  |  |
|  | 27.22.7.10: Data available event |  |  | R99 | 1.1 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.10: Data available event |  |  | Rel-8 | 1.2 |  |  |  |  |  |  |  |  |  |  |  | C223 | C223 | C223 | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS or NB-SS (See NOTE) |  |  |
|  | 27.22.7.10: Data available event |  |  | Rel-8 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  | C225 | C225 | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS or NB-SS (See NOTE) |  | TCEP003 |
|  | 27.22.7.10: Data available event |  |  | Rel-8 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  | C226 | C226 | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS or NB-SS (See NOTE) |  | TCEP003 |
|  | 27.22.7.10: Data available event |  |  | Rel-8 | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  | C227 | C227 | E.1/43 AND E.1/89 AND E.1/92 AND E.1/33 | E-USS or NB-SS (See NOTE) |  | TCEP004 |
|  | 27.22.7.11: Channel status event |  |  | R99 | 1.1 | C121 | C121 |  | C121 | C121 | C121 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | C121 AND C183 | E.1/44 AND E.1/89 AND E.1/33 | UMTS System Simulator or System Simulator only |  |  |
|  | 27.22.7.11: Channel status event |  |  | Rel-8 | 1.2 |  |  |  |  |  |  |  |  |  |  |  | C223 | C223 | C223 | E.1/44 AND E.1/89 AND E.1/33 | E-USS or NB-SS (See NOTE) |  |  |
|  | 27.22.7.12: Access Technology change event |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Single access technology |  |  | Rel-8 | 1.1 |  |  |  |  |  |  | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | E.1/45 AND E.1/33 | UMTS System Simulator and E-USS |  |  |
|  | Single access technology – WB-S1(Cat M1)/NB-S1 |  |  | Rel-13 | 1.3 |  |  |  |  |  |  |  |  |  |  |  | C167 AND C222 | C167 AND C222 | C167 AND C222 | E.1/45 AND E.1/33 | System Simulator and E-USS/NB-SS |  |  |
|  | Single access technology, NG-RAN |  |  | Rel-15 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/45 AND E.1/33 | NG-SS only |  |  |
|  | Multiple access technologies |  |  | Rel-8 | TBD |  |  |  |  |  |  | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | C184 AND C190 | E.1/45 AND E.1/33 AND E.1/200 | TBD |  |  |
|  | 27.22.7.13: Display parameter changed event |  |  | Rel-4 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/46 AND E.1/33 | TBD |  |  |
|  | 27.22.7.14: Local connection event |  |  | Rel-4 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/47 AND E.1/33 | TBD |  |  |
|  | 27.22.7.15: Network search mode change event |  |  | Rel-6 | 1.1 |  |  |  |  |  |  |  | M | M | M | M | M | M | M | E.1/48 AND E.1/33 | No |  |  |
|  | 27.22.7.16: Browsing status event |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/193 AND E.1/33 | TBD |  |  |
|  | 27.22.7.17: Network Rejection Event, ATTACH REJECT |  |  | Rel-8 | 1.1 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/33 AND E.197 | E-USS only or NB-SS (See NOTE) |  |  |
|  | 27.22.7.17: Network Rejection Event, TRACKING AREA UPDATE REJECT |  |  | Rel-8 | 1.2 |  |  |  |  |  |  | C190 | C190 | C190 | C190 | C190 | C222 | C222 | C222 | E.1/33 AND E.197 | E-USS or NB-SS (See NOTE) |  |  |
|  | 27.22.7.17: Network Rejection Event, REGISTRATION REJECT-Initial Registration |  |  | Rel-15 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/33 AND E.197 | NG-SS only |  |  |
|  | 27.22.7.17: Network Rejection Event, REGISTRATION REJECT- Mobility Registration updating |  |  | Rel-15 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  |  | C231 | E.1/33 AND E.197 | NG-SS only |  |  |
|  | Frame information changed event |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/195 AND E.1/177 AND E.1/178 | TBD |  |  |
|  | 27.22.7.18: CSG cell Selection |  |  | Rel-9 | 1.1 |  |  |  |  |  |  |  | C200 | C200 | C200 | C200 | C200 | C200 | C200 | E.1/201 | E-USS only |  |  |
|  | 27.22.7.19 : IMS registration event (Refer to 27.22.4.27.7 and 27.22.7.20) |  |  | Rel-10 | - |  |  |  |  |  |  |  |  |  |  |  |  |  |  | - | - |  |  |
|  | 27.22.7.20 : Incoming IMS data, IMS Registration and Data available event, IARI list stored on the ISIM |  |  | Rel-10 | 1.1 |  |  |  |  |  |  |  |  | C208 | C208 | C208 | C208 | C208 | C208 | E.1/33 AND E.1/43 AND E.1/89 AND E.1/91 AND E.1/246 AND E.1.247 AND E.1/249 | UMTS System Simulator OR E-USS |  |  |
|  | 27.22.7.21: EVENT DOWNLOAD – Data Connection Status Change event, E-UTRAN, Deactivate PDN |  |  | Rel-14 | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  | C229 | C229 | E.1/275 | E-USS OR NB-SS |  |  |
| 41 | MO SMS Control by USIM 27.22.8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | With proactive command, Allowed , no modification |  |  | R99 | 1.1 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 AND E.1/26 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | With user SMS, Allowed , no modification |  |  | R99 | 1.2 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 | UMTS System Simulator or System Simulator only |  |  |
|  | With proactive command, Not allowed |  |  | R99 | 1.3 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 AND E.1/26 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | With user SMS, Not allowed |  |  | R99 | 1.4 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 | UMTS System Simulator or System Simulator only |  |  |
|  | With proactive command, Allowed, with modifications |  |  | R99 | 1.5 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 AND E.1/26 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | With user SMS, Allowed, with modifications |  |  | R99 | 1.6 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 | UMTS System Simulator or System Simulator only |  |  |
|  | With Proactive command, the USIM responds with '90 00', Allowed, no modification |  |  | R99 | 1.7 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 AND E.1/26 AND E.1/110 | UMTS System Simulator or System Simulator only |  | TCEP001 |
|  | Send Short Message attempt by user, the USIM responds with '90 00', Allowed, no modification |  |  | R99 | 1.8 | M | M |  | M | M | M | C183 | C183 | C183 | C183 | C183 | C183 | C183 | C183 | E1/12 | UMTS System Simulator or System Simulator only |  |  |
|  | Void |  |  |  | 1.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, Allowed, no modification |  |  | Rel-8 | 1.10 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 AND E.1/26 AND E.1/110 | E-USS or NB-SS (See NOTE) |  | TCEP001 |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with user SMS, Allowed, no modification |  |  | Rel-8 | 1.11 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 | E-USS or NB-SS (See NOTE) |  |  |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, Not allowed |  |  | Rel-8 | 1.12 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 AND E.1/26 AND E.1/110 | E-USS or NB-SS (See NOTE) |  | TCEP001 |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with user SMS, Not allowed |  |  | Rel-8 | 1.13 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 | E-USS or NB-SS (See NOTE) |  |  |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, Allowed with modifications' |  |  | Rel-8 | 1.14 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 AND E.1/26 AND E.1/110 | E-USS or NB-SS (See NOTE) |  | TCEP001 |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with user SMS, Allowed with modifications |  |  | Rel-8 | 1.15 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 | E-USS or NB-SS (See NOTE) |  |  |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, the USIM responds with '90 00', Allowed, no modification |  |  | Rel-8 | 1.16 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 AND E.1/26 AND E.1/110 | E-USS or NB-SS (See NOTE) |  | TCEP001 |
|  | MO SM CONTROL BY USIM over SG in E-UTRAN, Send Short Message attempt by user, the USIM responds with '90 00', Allowed, no modification |  |  | Rel-8 | 1.17 |  |  |  |  |  |  |  |  |  |  |  | C220 | C220 | C220 | E1/12 | E-USS or NB-SS (See NOTE) |  |  |
| 42 | SERVICE SEARCH |  |  | Rel-4 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/94 | TBD |  |  |
| 43 | GET SERVICE INFORMATION |  |  | Rel-4 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/95 | TBD |  |  |
| 44 | DECLARE SERVICE |  |  | Rel-4 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/96 | TBD |  |  |
| 45 | RETRIEVE MULTIMEDIA MESSAGE |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/173 | TBD |  |  |
| 46 | SUBMIT MULTIMEDIA MESSAGE |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/173 | TBD |  |  |
| 47 | DISPLAY MULTIMEDIA MESSAGE |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/173 | TBD |  |  |
| 48 | SET FRAMES |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/177 AND E.1/178 | TBD |  |  |
| 49 | GET FRAME STATUS |  |  | Rel-6 | TBD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | E.1/178 AND E.1/177 | TBD |  |  |
| 50 | Handling of command number 27.22.9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | DISPLAY TEXT normal priority |  |  | R99 | 1.1 | C177 | C177 |  | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | C177 | E.1/17 AND E.1/110 | No |  |  |
| 51 | Call Control on EPS PDN connection 27.22.10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CALL CONTROL on EPS PDN for E-UTRAN – default PDN connection activation, allowed without modification |  |  | Rel-8 | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  | C222 | C222 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS or NB-SS (See NOTE) |  |  |
|  | CALL CONTROL on EPS PDN for E-UTRAN – default PDN connection activation, not allowed |  |  | Rel-8 | 1.2 |  |  |  |  |  |  |  |  |  |  |  |  | C222 | C222 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS or NB-SS (See NOTE) |  |  |
|  | CALL CONTROL on EPS PDN for E-UTRAN – default PDN connection activation, allowed with modification |  |  | Rel-8 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  | C222 | C222 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | E-USS or NB-SS (See NOTE) |  |  |
|  | CALL CONTROL on EPS PDN for E-UTRAN – PDN connection triggered by user, UICC sends 90 00 |  |  | Rel-8 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS only |  |  |
|  | CALL CONTROL on EPS PDN for E-UTRAN – PDN connection triggered by user, UICC sends 93 00 |  |  | Rel-8 | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS only |  |  |
|  | CALL CONTROL on EPS PDN for E-UTRAN – PDN connection triggered by user, allowed with modification |  |  | Rel-8 | 1.6 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 | E-USS only |  |  |
|  | CALL CONTROL on EPS PDN - PDN connection activation from OPEN CHANNEL command |  |  | Rel-8 | 1.7 |  |  |  |  |  |  |  |  |  |  |  |  | C182 | C182 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | E-USS only |  |  |
| 52 | Call Control on PDP Context Activation 27.22.11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CALL CONTROL on PDP Context Activation – default PDP connection activation, allowed without modification |  |  | R99 | 1.1 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PDP Context Activation – default PDP connection activation, not allowed |  |  | R99 | 1.2 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PDP Context Activation – default PDP connection activation, allowed with modification |  |  | R99 | 1.3 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PDP Context Activation – PDP connection triggered by user, UICC sends 90 00 |  |  | R99 | 1.4 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PDP Context Activation – PDP connection triggered by user, UICC sends 93 00 |  |  | R99 | 1.5 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PDP Context Activation – PDP connection triggered by user, allowed with modification |  |  | R99 | 1.6 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C183 | C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
|  | CALL CONTROL on PDP Context Activation - PDP connection activation from OPEN CHANNEL command |  |  | R99 | 1.7 | N/A | N/A |  | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | N/A | C191 AND C183 | C191 AND C183 | E.1/7 AND E.1/8 AND E.1/10 AND E.1/11 AND E.1/13 AND E.1/64 AND E.1/142 | UMTS System Simulator or System Simulator only |  |  |
| 53 | Change eCall mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | REFRESH after change eCall mode, disable FDN in EFEST, E-UTRAN |  |  | Rel-8 | 1.1 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/24 AND E.1/2 | E-UTRAN System Simulator only |  |  |
|  | REFRESH after change eCall mode, enable FDN in EFEST, E-UTRAN |  |  | Rel-8 | 1.2 |  |  |  |  |  |  |  |  |  |  |  |  | C190 | C190 | E.1/24 AND E.1/2 | E-UTRAN System Simulator only |  |  |
|  | REFRESH after changing eCall mode, disable FDN in EFEST, IMS Emergency Services in E-UTRAN |  |  | Rel-14 | 1.3 |  |  |  |  |  |  |  |  |  |  |  |  | C202 | C202 | E.1/24 AND E.1/2 | E-UTRAN System Simulator only |  |  |
|  | REFRESH after changing eCall mode, disable FDN in EFEST, UTRAN/GERAN |  |  | Rel-8 | 1.4 |  |  |  |  |  |  |  |  |  |  |  |  | C167 OR C184 | C167 OR C184 | E.1/24 AND E.1/2 | UMTS System Simulator or System Simulator only |  |  |
|  | REFRESH after change eCall mode, enable FDN in EFEST, UTRAN/GERAN |  |  | Rel-8 | 1.5 |  |  |  |  |  |  |  |  |  |  |  |  | C167 OR C184 | C167 OR C184 | E.1/24 AND E.1/2 | UMTS System Simulator or System Simulator only |  |  |
|  |  |  |  |  |  |  |  | NOTE: For Rel -13 if the UE supports NB-IoT, this test case shall be verified by accessing the NB System Simulator (NB-SS). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| C101 | IF A.1/1 THEN M ELSE N/A | -- O_Cap_Conf |
| --- | --- | --- |
| C102 | void |  |
| C103 | void |  |
| C104 | IF A.1/2 THEN M ELSE N/A | -- O_Sust_text |
| C105 | IF A.1/3 AND A.1/41 THEN M ELSE N/A | -- O_Ucs2_Entry AND O_UCS2_Cyrillic |
| C106 | IF A.1/4 THEN M ELSE N/A | -- O_Ext_Str |
| C107 | IF A.1/5 THEN M ELSE N/A | -- O_Help |
| C108 | IF A.1/6 THEN O.1 ELSE N/A | -- O_Icons |
| C109 | IF A.1/7 THEN M ELSE N/A | -- O_Dual_Slot |
| C110 | IF A.1/9 AND A.1/46 THEN M ELSE N/A | -- O_Run_At AND O_+CIMI |
| C111 | IF (A.1/10 OR E.1/71) THEN M ELSE N/A | -- O_LB |
| C112 | IF A.1/11 THEN M ELSE N/A | -- O_Soft_key |
| C113 | void |  |
| C114 | IF C110 AND C108 THEN O.1 ELSE N/A | -- O_Run_At AND O_+CIMI AND O_Icons |
| C115 | IF C111 AND C108 THEN M ELSE N/A | -- O_LB AND O_Icons |
| C116 | IF A.1/7 AND A.1/8 THEN M ELSE N/A | -- O_Dual_Slot AND O_Detach_Rdr |
| C117 | void |  |
| C118 | IF A.1/15 AND A.1/41 THEN M ELSE N/A | -- O_Ucs2_Disp AND O_UCS2_Cyrillic |
| C119 | IF A.1/19 THEN M ELSE N/A | -- O_Redial |
| C120 | IF A.1/20 THEN M ELSE N/A | -- O_D_NoResp |
| C121 | IF A.1/21 AND A.1/17 THEN M ELSE N/A | -- O_BIP_GPRS AND O_UDP |
| C122 | IF C111 AND A.1/16 THEN M ELSE N/A | -- O_LB AND O_GPRS |
| C123 | void |  |
| C124 | IF A.1/22, test x.A M ELSE x.B M (where x is the expected sequence number value) | -- O_CP_Subaddr |
| C125 | IF A.1/23 THEN M ELSE N/A | -- O_Imm_Resp |
| C126 | IF A.1/24 THEN M ELSE N/A | -- O_Duration |
| C127 | void |  |
| C128 | void |  |
| C129 | void |  |
| C130 | void |  |
| C131 | void |  |
| C132 | IF A.1/27 THEN M ELSE N/A | -- O_BIP_Local |
| C133 | void |  |
| C134 | IF A.1/38 THEN M ELSE N/A | -- O_MMS |
| C135 | void |  |
| C136 | void |  |
| C137 | void |  |
| C138 | void |  |
| C139 | IF A.1/35 THEN M ELSE N/A | -- O_Batt |
| C140 | IF A.1/39 THEN M ELSE N/A | -- O_UC_Before_EnvCC |
| C141 | IF A.1/40 THEN M ELSE N/A | -- O_UC_After_EnvCC |
| C142 | IF A.1/3 AND A.1/42 THEN M ELSE N/A | -- O_UCS2_Entry AND O_UCS2_Chinese |
| C143 | IF A.1/15 AND A.1/42 THEN M ELSE N/A | -- O_UCS2_Disp AND O_UCS2_Chinese |
| C144 | IF A.1/3 AND A.1/43 THEN M ELSE N/A | -- O_UCS2_Entry AND O_UCS2_Katakana |
| C145 | IF A.1/15 AND A.1/43 THEN M ELSE N/A | -- O_UCS2_Disp AND O_UCS2_Katakana |
| C146 | IF A. 1/45 THEN M ELSE N/A | -- O_FDN |
| C147 | IF A. 1/44 THEN M ELSE N/A | -- O_BDN |
| C148 | IF (A.1/9 AND A.1/47) THEN M ELSE N/A | -- O_Run_At AND O_+CGMI |
| C149 | IF C148 AND C118 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI AND O_ O_Ucs2_Disp AND O_Ucs2_ Cyrillic |
| C150 | IF C148 AND C143 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI AND O_ O_Ucs2_Disp AND O_Ucs2_ Chinese |
| C151 | IF C148 AND C145 THEN M ELSE N/A | -- O_Run_At AND O_+CGMI AND O_ O_Ucs2_Disp AND O_Ucs2_ Katakana |
| C152 | IF C121 AND A.1/49 THEN M ELSE N/A | -- O_BIP_GPRS AND O_UDP AND O_BUFFER_SIZE |
| C153 | IF A.1/50 THEN M ELSE N/A | -- O_TAT_AL |
| C154 | IF A.1/51 THEN M ELSE N/A | -- O_TAT_AC |
| C155 | IF A.1/52 THEN M ELSE N/A | -- O_TAT_AR |
| C156 | IF A.1/53 THEN M ELSE N/A | -- O_TAT_FSN |
| C157 | IF A.1/54 THEN M ELSE N/A | -- O_TAT_FSL |
| C158 | IF A.1/55 THEN M ELSE N/A | -- O_TAT_FSS |
| C159 | IF A.1/56 THEN M ELSE N/A | -- O_TAT_SN |
| C160 | IF A.1/57 THEN M ELSE N/A | -- O_TAT_SB |
| C161 | IF A.1/58 THEN M ELSE N/A | -- O_TAT_SI |
| C162 | IF A.1/59 THEN M ELSE N/A | -- O_TAT_SU |
| C163 | IF A.1/60 THEN M ELSE N/A | -- O_TAT_SS |
| C164 | IF A.1/61 THEN M ELSE N/A | -- O_TAT_STFC |
| C165 | IF A.1/62 THEN M ELSE N/A | -- O_TAT_STBC |
| C166 | IF A.1/63 THEN test step option n.A M ELSE test step option n.B M | -- O_longFTN |
| C167 | IF A.1/64 THEN M ELSE N/A | -- O_GERAN |
| C168 | IF A.1/65 THEN M ELSE N/A | -- O_Global_PB |
| C169 | IF (C121 AND A.1/68 THEN test x.A M ELSE IF (C121 AND NOT A.1/68) test x.B M ELSE N/A | -- (O_BIP_GPRS AND O_UDP AND O_User_Confirm_Before_PDP_Context_Request) OR (O_BIP_GPRS AND O_UDP AND NOT O_User_Confirm_Before_PDP_Context_Request) |
| C170 | IF A.1/69 THEN M ELSE N/A | -- O_Serv_SS_HOLD |
| C171 | IF A.1/6 THEN O.2 ELSE N/A | -- O_Icons |
| C172 | IF A.1/6 THEN O.4 ELSE N/A | -- O_Icons |
| C173 | IF C110 AND A.1/6 THEN O.2 ELSE N/A | -- O_Run_At AND O_+CIMI AND O_Icons |
| C174 | IF A.1/78 AND A.1/79 THEN M ELSE N/A | -- O_AddInfo_SS AND_O_Serv_SS_CFU |
| C175 | IF A.1/78 AND A.1/80 THEN M ELSE N/A | -- O_AddInfo_SS AND O_Serv_SS_CLIR |
| C176 | IF A. 1/44 THEN N/A ELSE M | -- O_BDN |
| C177 | IF A.1/84 THEN M ELSE N/A | -- O_No_Type_ND |
| C178 | IF A.1/85 THEN M ELSE N/A | -- O_No_Type_NK |
| C179 | IF A.1/86 THEN M ELSE N/A | -- O_No_Type_NA |
| C180 | IF A.1/87 THEN M ELSE N/A | -- O_No_Type_NS |
| C181 | IF A.1/88 THEN M ELSE N/A | -- O_No_Type_NL |
| C182 | IF A.1/18 AND (A.1/132 OR A.1/133) THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD) |
| C183 | IF (NOT A.1/135) AND (A.1/64 OR A.1/134) THEN M ELSE N/A | -- (NOT O_EUTRAN_NO_UTRAN_NO_GERAN) AND (O_GERAN OR O_UTRAN) |
| C184 | IF A.1/134 THEN M ELSE N/A | -- O_UTRAN |
| C185 | IF A.1/6 AND A.1/111 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec1_Send_SS |
| C186 | IF A.1/6 AND A.1/115 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec2_Send_USSD |
| C187 | IF A.1/6 AND A.1/114 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec1_Send_USSD |
| C188 | IF A.1/6 AND A.1/120 THEN M ELSE N/A | -- O_Icons AND O_Icon_Rec1_Set_Up_Idle_Mode_Text |
| C189 | IF C110 AND A.1/6 AND A.1/123 THEN M ELSE N/A | -- O_Run_At AND O_+CIMI AND O_Icons AND O_Icon_Rec1_Run_AT_Cmd |
| C190 | IF (A.1/139 OR A.1/140) THEN M ELSE N/A | -- pc_eTDD OR pc_eFDD |
| C191 | IF A.1/21 AND A.1/18 THEN M ELSE N/A | -- O_BIP_GPRS AND O_TCP |
| C192 | IF (A.1/21 AND A.1/18 AND A.1/72) THEN M ELSE N/A | -- O_BIP_GPRS AND O_TCP AND O_BIP_UICCServer |
| C193 | IF (A.1/10 OR (E.1/71 AND E.1/42)) THEN M ELSE N/A | -- O_LB |
| C194 | IF A.1/138 THEN M ELSE N/A | -- O_Select_Item_Default_Item |
| C195 | IF A.1/137 THEN M ELSE N/A | -- O_CSG_Cell_Discovery |
| C196 | IF (A.1/142 AND (A.1/139 OR A.1/140) THEN M ELSE N/A | -- O_pc_MO_SM-over-IMS AND (pc_eFDD OR pc_eTDD) |
| C197 | IF (A.1/142 AND A.1/134) THEN M ELSE N/A | -- O_pc_MO_SM-over-IMS AND O_UTRAN |
| C198 | IF (A.1/141 AND (A.1/139 OR A.1/140) THEN M ELSE N/A | -- O_pc_SM-over-IP-receiver AND (pc_eFDD OR pc_eTDD) |
| C199 | IF (A.1/141 AND A.1/134) THEN M ELSE N/A | -- O_pc_SM-over-IP-receiver AND O_UTRAN |
| C200 | IF A.1/136 THEN M ELSE N/A | -- O_Event_CSG_Cell_Selection |
| C201 | IF (A.1/64 AND A.1/149) THEN M ELSE N/A | -- O_GERAN AND O_SMS-CB_Data_Download |
| C202 | IF ((A.1/139 OR A.1/140) AND A.1/150) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_IMS |
| C203 | IF (A.1/134 AND A.1/150) THEN M ELSE N/A | -- O_UTRAN AND O_IMS |
| C204 | IF A.1/151 THEN N/A ELSE M | -- O_PS_OPMODE |
| C205 | IF ((A.1/139 OR A.1/140) AND A.1/152) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_SMS_SGs_MT |
| C206 | IF ((A.1/139 OR A.1/140) AND A.1/153) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD) AND O_SMS_SGs_MO |
| C207 | IF (A.1/147 AND A.1/148 AND A.1/150) THEN M ELSE O | -- O_Event_IMS_Registration AND O_UICC_ACCESS_IMS AND O_IMS |
| C208 | IF (A.1/146 AND A.1/147 AND A.1/148 AND A.1/150) THEN M ELSE N/A | -- O_Event_Incoming_IMS_Data AND O_Event_IMS_Registration AND O_UICC_ACCESS_IMS AND O_IMS AND |
| C209 | IF (A.1/157 OR A.1/159) THEN M ELSE N/A | -- (pc_SMS_CS_MO OR pc_SMS_PS_MO) |
| C210 | IF ((NOT A.1/135) AND (A.1/64 OR A.1/134) AND (A.1/157 OR A.1/159)) THEN M ELSE N/A | -- (NOT (O_EUTRAN_NO_UTRAN NO_GERAN) AND (O_GERAN OR O_UTRAN)) AND (pc_SMS_CS_MO OR pc_SMS_PS_MO) |
| C211 | IF (A.1/156 OR A.1/158) THEN M ELSE N/A | -- (pc_SMS_CS_MT OR pc_SMS_PS_MT) |
| C212 | IF ((NOT A.1/135) AND (A.1/64 OR A.1/134) AND (A.1/156 OR A.1/158)) THEN M ELSE N/A | -- (NOT (O_EUTRAN_NO_UTRAN NO_GERAN) AND (O_GERAN OR O_UTRAN)) AND (pc_SMS_CS_MT OR pc_SMS_PS_MT) |
| C213 | IF (NOT A.1/160) THEN M ELSE N/A | -- NOT O_Rej_Launch_Browser_with DefURL |
| C214 | IF (A.1/160) THEN M ELSE N/A | -- O_Rej_Launch_Browser_with DefURL |
| C215 | IF (A.1/16) THEN M ELSE N/A | -- O_GPRS |
| C216 | IF (A.1/161) THEN M ELSE N/A | -- O_Lang_Select |
| C217 | IF (A.1/162) THEN M ELSE N/A | -- O_Provide_Local_LS |
| C218 | IF (A.1/163) THEN M ELSE N/A | -- O_Lang_Notif |
| C219 | IF (A.1/164) THEN M ELSE N/A | -- O_Refresh_AlphaIdentifier |
| C220 | IF ((A.1/139 OR A.1/140 OR A.1/173) AND A.1/153) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MO |
| C221 | IF ((A.1/139 OR A.1/140 OR A.1/173) AND A.1/152) THEN M ELSE N/A | -- (pc_eFDD OR pc_eTDD OR pc_NB) AND O_SMS_SGs_MT |
| C222 | IF (A.1/139 OR A.1/140 OR A.1/173) THEN M ELSE N/A | -- pc_eTDD OR pc_eFDD OR pc_NB |
| C223 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C224 | IF A.1/18 AND A.1/178 AND (A.1/132 OR A.1/133 OR A.1/177) THEN M ELSE N/A | -- O_TCP AND pc_Multiple_PDN AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) |
| C225 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/182 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_ SUSPEND_UICC |
| C226 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/181 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_PSM_DEAC_UICC |
| C227 | IF A.1/18 AND (A.1/132 OR A.1/133 OR A.1/177) AND A.1/183 THEN M ELSE N/A | -- O_TCP AND (pc_BIP_eFDD OR pc_BIP_eTDD OR pc_BIP_NB) AND O_eDRX_ SUSPEND_UICC |
| C228 | IF ((A.1/132 OR A.1/133) AND A.1/152) AND A.1/184 THEN M ELSE N/A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND O_SMS_SGs_MT AND O_PS_Data_Off |
| C229 | IF (A.1/132 OR A.1/133) OR A.1/177 THEN M ELSE N/A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) OR pc_BIP_NB |
| C230 | A.1/17 AND A.1/178 AND ((A.1/132 OR A.1/133) OR A.1/177) THEN M ELSE N/A | --O_UDP AND pc_Multiple_PDN AND ((pc_BIP_eFDD OR pc_BIP_eTDD) OR pc_BIP_NB) |
| C231 | IF A.1/186 THEN M ELSE N/A | -- pc_NG_RAN |
| O.1 | IF A.1/zz tests x.yA M ELSE tests x.yB M (where zz corresponds to the option relating to the command being tested (e.g. A.1/90 if Display Text supports icons as defined in record 1 of EF(IMG)) and x.y is the expected sequence number value) |  |
| O.2 | IF A.1/zz tests x.yA M ELSE tests x.yB M (where zz corresponds to the option relating to the command being tested (e.g. A.1/91 if Display Text supports icons as defined in record 2 of EF(IMG)) and x.y is the expected sequence number value) |  |
| O.3 | void |  |
| O.4 | IF A.1/zz AND A.1/ww tests x.yA M ELSE tests x.yB M (where zz and ww correspond to the option relating to the command being tested (e.g. A.1/90 if Display Text supports icons as defined in record 1 of EF(IMG) and A.1.92 if Display Text supports icons as defined in record 5 of EF(IMG) ) and x.y is the expected sequence number value) |  |
| TCEP001 | IF NOT A.1/84 THEN during the test execution, the display or the non-display of any alpha identifier, text string or icon shall be treated as successfully verified. |  |
| TCEP002 | IF NOT A.1/85 THEN the terminal may open the channel without explicit confirmation by the user. |  |
| TCEP003 | If A.1/181 and/or A.1/182 is supported, in addition to the test case initial conditions, any specific information or particular UE configurations required to ensure that the UE performs UICC deactivation/suspension in PSM shall be provided by the UE manufacturer |  |
| TCEP004 | If A.1/183 is supported, in addition to the test case initial conditions, any specific information or particular UE configurations required to ensure that the UE suspends the UICC in eDRX shall be provided by the UE manufacturer |  |
| AER001 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.27.6, Seq. 6.1) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (O_BIP_eFDD OR O_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER002 | IF ( (A.1/132 OR A.1/133 OR A.1/173) AND (A.1/134 OR A.1/64))) THEN R(27.22.7.4 Seq. 1.2) ELSE A | -- (pc_eTDD OR pc_eFDD OR pc_NB) AND (O_GERAN OR O_UTRAN) |
| AER003 | IF ( (A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.15 Seq. 1.17) ELSE A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER004 | IF ( (A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.15 Seq. 1.14) ELSE A | -- (pc_BIP_eFDD OR pc_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER005 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.27.6, Seq. 6.4) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (O_BIP_eFDD OR O_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER006 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.27.6, Seq. 6.3) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (O_BIP_eFDD OR O_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER007 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.27.6, Seq. 6.5) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (O_BIP_eFDD OR O_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |
| AER008 | IF ((A.1/21 AND A.1/17) AND ((A.1/132 OR A.1/133) AND (A.1/134 OR A.1/64))) THEN R(27.22.4.29, Seq. 1.2) ELSE A | -- (O_BIP_GPRS AND O_UDP) AND (O_BIP_eFDD OR O_BIP_eTDD) AND (O_UTRAN OR O_GERAN) |

## 3.5	Conventions for mathematical notations

The conventions for mathematical notations specified below shall apply.

### 3.5.1	Mathematical signs

The "plus or minus" sign is expressed by "".

The sign "multiplied by" is expressed by "*".

The sign "divided by" is expressed by "/", or the common division bar.

The sign "greater than or equal to" is expressed by "".

The sign "less than or equal to" is expressed by "".

# 4	Test equipment

The test equipment is specified in TS 34.108 [12] clause 4.

# 5	Testing methodology in general

When possible the present document refers to ETSI TS 102 384 [26] to describe generic aspects of application toolkit tests.

## 5.1	Testing of optional functions and procedures

Any function or procedure which is optional, as indicated in the present document, may be subject to a conformance test if it is implemented in the ME.

## 5.2	Test interfaces and facilities

The UICC and NB-SS/E-USS/USS/SS interfaces provide the main test interfaces for the purpose of performing conformance tests.

The tests which require a network simulator shall be carried out with using an NB System Simulator when accessing an E-UTRAN in NB-S1 mode, an Evolved Universal System Simulator when accessing an E-UTRAN in WB-S1 mode, a Universal System Simulator when accessing a UTRAN, and if theses tests have to be peformed additionally when accessing a GERAN a System Simulator shall be used instead.

## 5.3	Information to be provided by the apparatus supplier

The information to be provided by the apparatus supplier specified in TS 36.523-2 [34], TS 36.508 [33], TS 34.108 [12] and TS 51.010-1 [23] shall apply, unless otherwise specified in the present clause.

In addition, the apparatus supplier shall provide the information with respect to the Supported Option table A.1 and to ME's default configuration table A.2.

Table A.2: ME's default configuration

| Item | Description | Value | Status |
| --- | --- | --- | --- |
| 1 | DISPLAY TEXT: No Response from user timeout interval |  | C |
| 2 | GET INKEY: No response from user Timeout interval |  | C |
| 3 | GET INPUT: No response from user Timeout interval |  | C |
| 4 | SELECT ITEM: No response from user Timeout interval |  | C |
| 5 | DISPLAY TEXT Text Attributes Alignment [Left or Center or Right] |  | C |
| 6 | GET INKEY Text Attributes Alignment [Left or Center or Right] |  | C |
| 7 | GET IMPUT Text Attributes Alignment [Left or Center or Right] |  | C |
| 8 | PLAY TONE Text Attributes Alignment [Left or Center or Right] |  | C |
| 9 | SET UP MENU Text Attributes Alignment [Left or Center or Right] |  | C |
| 10 | SELECT ITEM Text Attributes Alignment [Left or Center or Right] |  | C |
| 11 | SEND SHORT MESSAGE Text Attributes Alignment [Left or Center or Right] |  | C |
| 12 | SEND SS Text Attributes Alignment [Left or Center or Right] |  | C |
| 13 | SEND USSD Text Attributes Alignment [Left or Center or Right] |  | C |
| 14 | SET UP CALL Text Attributes Alignment [Left or Center or Right] |  | C |
| 15 | SET UP IDLE MODE TEXT Text Attributes Alignment [Left or Center or Right] |  | C |
| 16 | RUN AT Text Attributes Alignment [Left or Center or Right] |  | C |
| 17 | SEND DTMF Text Attributes Alignment [Left or Center or Right] |  | C |
| 18 | LAUNCH BROWSER Text Attributes Alignment [Left or Center or Right] |  | C |
| 19 | OPEN CHANNEL Text Attributes Alignment [Left or Center or Right] |  | C |
| 20 | CLOSE CHANNEL Text Attributes Alignment [Left or Center or Right] |  | C |
| 21 | RECEIVE DATA Text Attributes Alignment [Left or Center or Right] |  | C |
| 22 | SEND DATA Text Attributes Alignment [Left or Center or Right] |  | C |
| 23 | IMEI |  | M |
| 24 | IMEISV |  | C |
| 25 | [Reserved] |  |  |
| 26 | Additional Card Reader Id |  | C |
| 27 | Channel Id |  | C |
| 28 | Manufacturer identification as implemented according to TS 27.007, cl. 5.1 |  | C |
| 29 | Preferred buffer size supported by the terminal for Open Channel command |  | C |
| Note : Conditional values shall be provided if the corresponding option is supported in the table A.1 |  |  |  |

# 6	Implicit testing

For some 3GPP features conformance is not verified explicitly in the present document. This does not imply that correct functioning of these features is not essential, but that these are implicitly tested to a sufficient degree in other tests.

It should be noted that for these features some aspects have to be and are explicitly tested, e.g. the ability to switch between 1.8v and 3v operation.

Some UICC features will be explicitly tested as result of other tests. These should be identified for the following reason:

-	To identify the areas of overlap and thus provide a more efficient testing.

# 7	Measurement uncertainty

The measured value relating to the corresponding limit shall be used to determine whether or not a terminal equipment meets the requirement. (ETR 028, annex B).

This process is often referred to as "shared risk".

# 8	Format of tests

In general the following basic format for tests is used:

27.22.X.X.	Tested command

27.22.X.X.1	Command tested in «environment #1" (NORMAL, ICONS, UCS2 …)

27.22.X.X.1.1	Definition and applicability

This clause refers back to clause 3.2.2.

27.22.X.X.1.2	Conformance requirement

Only if required, this clause details the necessary core specification references.

27.22.X.X.1.3	Test purpose

This clause details the purpose of the test.

27.22.X.X.1.4	Method of test

27.22.X.X.1.4.1	Initial conditions

If present this clause defines the initial conditions to be established before running each test sequence.

27.22.X.X.1.4.2	Procedure

This clause details the test procedure. Each test sequence shall be carried out independently unless otherwise stated.

-	Sequence 1.1 (further initial conditions, added here)

Command 1.1.1

TERMINAL RESPONSE1.1.1A or 1.1.1B

Command 1.1.2

TERMINAL RESPONSE1.1.2

PROACTIVE COMMAND 1.1.1

TERMINAL RESPONSE 1.1.1A

TERMINAL RESPONSE 1.1.1B

PROACTIVE COMMAND 1.1.2

TERMINAL RESPONSE 1.1.2

-	Sequence 1.2

Command 1.2.1

TERMINAL RESPONSE 1.2.1

Command 1.2.2

TERMINAL RESPONSE1.2.2 (same as TERMINAL RESPONSE 1.2.1)

Command 1.2.3

TERMINAL RESPONSE 1.2.3

PROACTIVE COMMAND 1.2.1

PROACTIVE COMMAND 1.2.2

PROACTIVE COMMAND 1.2.3

TERMINAL RESPONSE 1.2.1

TERMINAL RESPONSE 1.2.2

TERMINAL RESPONSE 1.2.3

-	Sequence 1.3

Command 1.3.1

TERMINAL RESPONSE1.3.1

PROACTIVE COMMAND 1.3.1

TERMINAL RESPONSE 1.3.1

27.22.X.X.1.5	Test requirement

This clause details the conditions to be met for successful completion of the test.

27.22.X.X.2	Command tested in "environment #2" (NORMAL, ICONS, UCS2 …)

27.22.X.X. 2.1	Definition and applicability

27.22.X.X. 2.2	Conformance requirement

27.22.X.X. 2.3	Test purpose

27.22.X.X. 2.4	Method of test

27.22.X.X. 2.4.1.1	Initial conditions

27.22.X.X. 2.4.1.2	Procedure

-	Sequence 2.1

Command 2.1.1

TERMINAL RESPONSE2.1.1A or 2.1.1B

Command 2.1.2

TERMINAL RESPONSE2.1.2

PROACTIVE COMMAND 2.1.1

TERMINAL RESPONSE 2.1.1A

TERMINAL RESPONSE 2.1.1B

PROACTIVE COMMAND 2.1.2

TERMINAL RESPONSE 2.1.2

-	Sequence 2.2

Command 2.2.1

TERMINAL RESPONSE 2.2.1

Command 2.2.2

TERMINAL RESPONSE 2.2.2 (same as TERMINAL RESPONSE 2.2.1)

Command 2.2.3

TERMINAL RESPONSE 2.2.3

PROACTIVE COMMAND 2.2.1

PROACTIVE COMMAND 2.2.2

PROACTIVE COMMAND 2.2.3

Coding TERMINAL RESPONSE 2.2.1

Coding TERMINAL RESPONSE 2.2.2

Coding TERMINAL RESPONSE 2.2.3

27.22.X.X.2.5	Test requirement

# 9	Generic call set up procedures

The generic call set up procedure for PS and CS calls specified for GERAN and UTRAN shall apply.

For a ME accessing E-UTRAN in NB-S1 mode the procedures defined in TS 36.508 [33] shall be the basis for all performed procedures during the test. The procedures in TS 36.508 [33] subclause 8.1.5 describe the default behaviour of a conformant ME regarding the specified protocols to be used for E-UTRAN in NB-S1 mode and the required procedures from the NAS.

For a ME accessing E-UTRAN in WB-S1 mode the procedures defined in TS 36.508 [33] shall be the basis for all performed procedures during the test. The procedures in TS 36.508 [33] subclause 4.5 describe the default behaviour of a conformant ME regarding the specified protocols to be used for E-UTRAN in WB-S1 mode and the required procedures from the NAS.

For a ME accessing GERAN the call set up procedures specified in TS 51.010-1 [23] subclause 26.9 shall apply, for session setup the ones defined in 45.2 and 45.4, unless otherwise specified in the present clause.

For a ME accessing NGRAN the procedures defined in TS 38.508-1 [39] shall be the basis for all performed procedures during the test. The procedures in TS 38.508-1 [39] subclause 4.5 describe the default behaviour of a conformant ME regarding the specified protocols to be used for NGRAN and the required procedures from the NAS.

# 10 - 26	Not used

# 27	Testing of the UICC/ME interface

## 27.0	Introduction

This clause is an addition to TS 31.121 [21] to confirm the correct interpretation of the USIM Application Toolkit commands and the correct operation of the Toolkit facilities.

The definitions, declarations and default values specified in TS 31.121 [21] clause 4.1 shall apply, unless otherwise specified in the present clause.

A USIM Simulator with the appropriate USIM Application Toolkit functionality will be required. Alternatively, USIMs programmed with specific data and USIM Application Toolkit applets may be used.The USIM data defined below shall be used for all test cases unless otherwise specified within the test case.

The comprehension required flags in SIMPLE-TLV objects that are included in a TERMINAL RESPONSE or an ENVELOPE shall be set as described in TS 31.111 [15]. This means that in cases where it is up to the ME to decide if this flag is used or not, the corresponding Tag coding in the TERMINAL RESPONSEs and ENVELOPEs in this document represents only one of the two valid possibilities.

TS 31.111 [15] defines that in case of the general result "Command performed successfully" some proactive commands require additional information in the command result and in which cases this is mandatory or optional.  Thus when additional information bytes are optional in the Result TLV, the additional information bytes of the Result TLVin the Terminal Responses shall be ignored.

## 27.1 - 27.21	Void

## 27.22	USIM Application Toolkit

### 27.22.1A	General Test purpose

Testing of functional conformance to USIM Application Toolkit commands, including proactive UICC commands.

All facilities given by the TERMINAL PROFILE as supported, for which tests exist in the present document, shall be tested.

Many of the proactive UICC commands include an alpha identifier data object. This is intended to be a short one or two word identifier for the ME to optionally display on the screen along with any other indications, at the same time as the ME performs the UICC command.

Note:	The sequence of USIM Application Toolkit commands are specific to the Toolkit Application being executed within the UICC, hence sequential testing of commands is not possible. The testing will therefore have to be performed on a command by command basis.

### 27.22.2A	Definition of default values for USIM Application Toolkit testing

A UICC containing the following default values is used for all tests of this clause unless otherwise stated.

For each item, the logical default values and the coding within the Elementary Files (EF) of the USIM follow, as defined in:

-	TS 31.121 [21], clause 4.1.

-	ETSI TS 102 384 [26], clause 27.22.1B.

Note 1:	Bx represents byte x of the coding.

Note 2:	Unless otherwise defined, the coding values in binary.

EFUST (USIM Service Table)

Logically:

(Service 01)	Local Phone Book available

(Service 02)	Fixed dialling numbers available

(Service 06)	Barred dialling numbers available

(Service 10)	Short Message Storage available

(Service 11)	Short Message Status Reports available

(Service 12)	Short Message Service Parameters available

(Service 15)	Cell Broadcast Message Identifier available

(Services 17, 18)	The Group Identifier level 1 and level 2 not available

(Service 20)	User controlled PLMN selector available

(Service 22)	Image (IMG) available

(Service 27)	The GSM Access available

(Service 28)	Data download via SMS-PP available

(Service 29)	Data download via SMS-CB available

(Service 30)	Call Control by USIM not available

(Service 31)	MO-SMS Control by USIM not available

(Service 32)	RUN AT COMMAND available

(Service 33)	(Packed Switched Domain) shall be set to '1'

(Service 34)	Enabled Services Table available

(Service 85)	EPS Mobility Management Information not available

(Service 86)	Allowed CSG Lists and corresponding indications not available

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 |
| --- | --- | --- | --- | --- | --- | --- |
| binary | xx1x xx11 | x1xx 111x | xx1x 1x00 | 1001 11xx | xxx xx11 | xxxx xxxx |
|  | B7 | B8 | B9 | B10 | B11 |  |
|  | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xx00 xxxx |  |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFEST (Enabled Services Table)

Logically:

(Service 1)	Fixed Dialling number deactivated

(Service 2)	Barred Dialling number deactivated

(Service 3)	APN Control List deactivated

| Coding: | B1 |
| --- | --- |
| binary | 00 |

EFIMSI (International Mobile Subscriber Identity)

Logically:

Length:	8 bytes

IMSI:	001 01 0123456789

| Coding: | '08 09 10 10 10 32 54 76 98' |
| --- | --- |

EFAD (Administrative Data)

Logically:	Type approval operations

OFM to be deactivated by the Terminal

MNC:	2 digit

| Coding: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 80 | 00 | 00 | 02 |

EFLOCI (Location Information)

Logically:

LAI-MCC:	001

LAI-MNC:	01

LAI-LAC:	0001

TMSI:	"FF .. FF"

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | 00 | F1 | 10 | 00 | 01 | FF | 00 |

EFPSLOCI (Packet Switch Location Information)

Logically:

RAI-MCC:	001

RAI-MNC:	01

RAI-LAC:	0001

RAI-RAC:	05

P-TMSI:	"FF….FF"

P-TMSI signature value:	"FF…FF"

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | FF | FF | FF |
| Coding: | B8 | B9 | B10 | B11 | B12 | B13 | B14 |
| Hex | 00 | F1 | 10 | 00 | 01 | 05 | 00 |

EFCBMI (Cell Broadcast Message Identifier)

Logically:

Cell Broadcast Message Identifier 1:	'03 E7'

| Coding: | 03 | E7 | FF | .. | FF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EFCBMID (Cell Broadcast Message Identifier for Data Download)

Logically:

Cell Broadcast Message Identifier 1:	'10 01'

| Coding: | 10 | 01 | FF | .. | FF |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EFFDN (Fixed Dialling Numbers)

Logically:

Record 1:	Length of alp ha identifier:	6 characters;

Alpha identifier:	"FDN111";

Length of BCD number:	"03";

TON and NPI:	Telephony and unknown;

Dialled number:	123;

CCI:	None;

Ext2:	None.

Coding for record 1:

|  | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 31 | 31 | 31 | 03 | 81 | 21 | F3 | FF | FF | FF |
|  | B14 | B15 | B16 | B17 | B18 | B19 | B20 |  |  |  |  |  |  |
|  | FF | FF | FF | FF | FF | FF | FF |  |  |  |  |  |  |

Record 2:	Length of alpha identifier:	6 characters;

Alpha identifier:	"FDN222";

Length of BCD number:	"03";

TON and NPI:	Telephony and Unknown;

Dialled number:	9876;

CCI:	None;

Ext2:	None.

Coding for record 2:

|  | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 32 | 32 | 32 | 03 | 81 | 89 | 67 | FF | FF | FF |
|  | B14 | B15 | B16 | B17 | B18 | B19 | B20 |  |  |  |  |  |  |
|  | FF | FF | FF | FF | FF | FF | FF |  |  |  |  |  |  |

Record 3:	Length of alpha identifier:	6 characters;

Alpha identifier:	"FDN333";

Length of BCD number:	"0B";

TON and NPI:	Telephony and International;

Dialled number:	+12345678901234567890;

CCI:	None;

Ext2:	None.

Coding for record 3:

|  | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 46 | 44 | 4E | 33 | 33 | 33 | 0B | 91 | 21 | 43 | 65 | 87 | 09 |
|  | B14 | B15 | B16 | B17 | B18 | B19 | B20 |  |  |  |  |  |  |
|  | 21 | 43 | 65 | 87 | 09 | FF | FF |  |  |  |  |  |  |

EFBDN (Barred Dialling Numbers)

Logically:

Record 1:	Length of alpha identifier:	6 characters;

Alpha identifier:	"BDN111";

Length of BCD number:	"06";

TON and NPI:	Telephony and International;

Dialled number:	+1357924680;

CCI:	None;

Ext4:				None

Comprehension method pointer: None.

Coding for record 1:

|  | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 44 | 4E | 31 | 31 | 31 | 06 | 91 | 31 | 75 | 29 | 64 | 08 |
|  | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 |  |  |  |  |  |
|  | FF | FF | FF | FF | FF | FF | FF | FF |  |  |  |  |  |

Record 2:	Length of alpha identifier:	6 characters;

Alpha identifier:	"BDN222";

Length of BCD number:	"03";

TON and NPI:	Telephony and Unknown;

Dialled number:	122;

CCI:	None;

Ext4:				None

Comprehension method pointer: None.

Coding for record 2:

|  | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 44 | 4E | 32 | 32 | 32 | 04 | 81 | 21 | F2 | FF | FF | FF |
|  | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 |  |  |  |  |  |
|  | FF | FF | FF | FF | FF | FF | FF | FF |  |  |  |  |  |

Record 3:	Length of alpha identifier:	6 characters;

Alpha identifier:	"BDN333";

Length of BCD number:	"03";

TON and NPI:	Telephony and Unknown;

Dialled number:	112;

CCI:	None;

Ext4:				None.

Comprehension method pointer: None

Coding for record 3:

|  | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | B13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 42 | 44 | 4E | 33 | 33 | 33 | 03 | 81 | 11 | F2 | FF | FF | FF |
|  | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 |  |  |  |  |  |
|  | FF | FF | FF | FF | FF | FF | FF | FF |  |  |  |  |  |

EFECC (Emergency Call Codes)

Logically:	Emergency call code:	"122";

Emergency call code alpha identifier:	"TEST";

Emergency call Service Category:	RFU

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 21 | F2 | FF | 54 | 45 | 53 | 54 | 00 |

EFSMSS (SMS Status)

Logically:	Last used TP-MR set to"00".

Memory capacity available (flag unset b1="1").

| Coding: | B1 | B2 |
| --- | --- | --- |
| Hex | 00 | FF |

EFSMSP (Short message service parameters)

Logically:

Record 1:

Record length:	28 bytes

Parameter Indicators:

TP-Destination Address:	Parameter absent

TS-Service Centre Address:	Parameter present

TP-Protocol Identifier:	Parameter absent

TP-Data Coding Scheme:	Parameter absent

TP-Validity Period:	Parameter absent

TS-Service Centre Address:

TON:	International Number

NPI:	"ISDN / telephone numbering plan"

Dialled number string:	"112233445566778"

| Coding: | B1 | B2 | B3 | ... | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Record 1: | FD | FF | FF | ... | FF | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 |

| B24 | B25 | B26 | B27 | B28 |
| --- | --- | --- | --- | --- |
| FF | FF | FF | FF | FF |

For the display of icon: See ETSI TS 102 384 [26] subclause 27.22.1B.

### 27.22.2B	Definition of default values for LTE related USIM Application Toolkit testing

#### 27.22.2B.1	Definition of E-UTRAN/EPC UICC

For each item, the logical default values and the coding within the Elementary Files (EF) of the USIM follow, as defined in clause 27.22.2A of the present document with the following execptions:

EFUST (USIM Service Table)

Logically:

(Service 01)	Local Phone Book available

(Service 02)	Fixed dialling numbers available

(Service 06)	Barred dialling numbers available

(Service 10)	Short Message Storage available

(Service 11)	Short Message Status Reports available

(Service 12)	Short Message Service Parameters available

(Service 15)	Cell Broadcast Message Identifier available

(Services 17, 18)	The Group Identifier level 1 and level 2 not available

(Service 20)	User controlled PLMN selector available

(Service 22)	Image (IMG) available

(Service 27)	The GSM Access available

(Service 28)	Data download via SMS-PP available

(Service 29)	Data download via SMS-CB available

(Service 30)	Call Control by USIM not available

(Service 31)	MO-SMS Control by USIM not available

(Service 32)	RUN AT COMMAND available

(Service 33)	(Packed Switched Domain) shall be set to '1'

(Service 34)	Enabled Services Table available

(Service 85)	EPS Mobility Management Information available

(Service 86)	Allowed CSG Lists and corresponding indications not available

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 |
| --- | --- | --- | --- | --- | --- | --- |
| binary | xx1x xx11 | x1xx 111x | xx1x 1x00 | 1001 11xx | xxx xx11 | xxxx xxxx |
|  | B7 | B8 | B9 | B10 | B11 |  |
|  | xxxx xxxx | xxxx xxxx | xxxx xxxx | xxxx xxxx | xx01 xxxx |  |

The coding of EFUST shall conform with the capabilities of the USIM used.

EFEPSLOCI (EPS Information)

Logically:	GUTI:			0010100010266341122

Last visited registered TAI:	001/01/0001

EPS update status:		not updated

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex: | 0B | F6 | 00 | F1 | 10 | 00 | 01 | 02 | 66 | 43 | 11 |
|  | B12 | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |
|  | 22 | 00 | F1 | 10 | 00 | 01 | 01 |  |  |  |  |

EFEPSNSC (EPS NAS Security Context)

Logically:	Key Set Identifier KSIASME:		'07'	(no key available)

ASME Key (KSIASME) :		'FF' (not available)

Uplink NAS count:		'00'

Downlink NAS count:		'00'

Identifiers of selected NAS	'FF'
integrity and encryption 
algorithm

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | … | … | … | … | Bxx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | xx | 80 | 01 | 07 | 81 | 00 | … | … | … | … | xx |

#### 27.22.2B.2	Definition of E-UTRAN parameters

The default E-UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	Cell Identity value = 0001;

The default EPS bearer context is defined in "Reference default EPS bearer context #1" in cl. 6.6.1 of TS 36.508 [33].

The default PDP type shall be "IP".

## 27.22.2C	Definition of E-UTRAN/EPC ISIM-UICC

#### 27.22.2C.1	Applications on the E-UTRAN/EPC ISIM-UICC

The E-UTRAN/EPC ISIM-UICC shall contain a USIM as defined in clause 27.22.2B.1 and an ISIM as defined in clause 27.22.2C.3.

#### 27.22.2C.2	Default USIM values of E-UTRAN/EPC ISIM-UICC

The E-UTRAN/EPC ISIM-UICC related test cases require a USIM to access the E-UTRAN/EPC. For this purpose the USIM shall be configured as defined in clause 27.22.2B.1.

#### 27.22.2C.3	Default ISIM values of E-UTRAN/EPC ISIM-UICC

The E-UTRAN/EPC ISIM-UICC shall contain an  ISIM for IMS access with the following values:

#### 27.22.2C.3.1	EFAD (Administrative Data)

Logically:	Type approval operations

| Byte: | B01 | B02 |
| --- | --- | --- |
| Coding: | 80 | 00 |

#### 27.22.2C.3.2	EFIST (ISIM Service Table)

Logically:

(Service 01) P-CSCF Address:						available

(Service 02) Generic Bootstrapping:						not available

(Service 03) HTTP Digest:							not available

(Service 04) GBA Based Local Key Establishment Mechanism:	not available

(Service 05) Support for P-CSCF discovery for IMS local breakout:	not available

(Service 06) Short Message Storage (SMS):					available

(Service 07) Short Message Status Reports (SMSR):			available

(Service 08) Support for SM-over-IP:					available

| Byte: | B01 |
| --- | --- |
| Coding: | 111x xxx1 |

#### 27.22.2C.3.3	EFIMPI (IMS private user identity)

Logically:	001010123456789@test.3gpp.com

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 1D | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 40 | 74 | 65 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 73 | 74 | 2E | 33 | 67 | 70 | 70 | 2E | 63 | 6F |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 6D | FF | FF | FF | FF | FF | FF | FF | FF | FF |

#### 27.22.2C.3.4	EFDOMAIN (Home Network Domain Name)

Logically:	test.3gpp.com

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 0D | 74 | 65 | 73 | 74 | 2E | 33 | 67 | 70 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 70 | 2E | 63 | 6F | 6D | FF | FF | FF | FF | FF |

#### 27.22.2C.3.5	EFIMPU (IMS public user identity)

Record 1 :

Logically:	sip:001010123456789@ims.mnc246.mcc081.3gppnetwork.org

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 35 | 73 | 69 | 70 | 3A | 30 | 30 | 31 | 30 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 31 | 30 | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 39 | 40 | 69 | 6D | 73 | 2E | 6D | 6E | 63 | 32 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 34 | 36 | 2E | 6D | 63 | 63 | 30 | 38 | 31 | 2E |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | 33 | 67 | 70 | 70 | 6E | 65 | 74 | 77 | 6F | 72 |
|  | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | 6B | 2E | 6F | 72 | 67 | FF | FF | FF | FF | FF |

Record 2:

Logically:	sip:+11234567890@test.3gpp.com

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 1E | 73 | 69 | 70 | 3A | 2B | 31 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 30 | 40 | 74 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 65 | 73 | 74 | 2E | 33 | 67 | 70 | 70 | 2E | 63 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 6F | 6D | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Record 3:

Logically:	tel:+11234567890

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 10 | 74 | 65 | 6C | 3A | 2B | 31 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 30 | FF | FF |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B51 | B52 | B53 | B54 | B55 | B56 | B57 | B58 | B59 | B60 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

#### 27.22.2C.3.6	EFP-CSCF (P-CSCF ADDRESS)

Logically:

Address Type:	FQDN

P-CSCF Address:	pcscf1.anyims.test.3gpp.com

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 1C | 00 | 70 | 63 | 73 | 63 | 66 | 31 | 2E |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 61 | 6E | 79 | 69 | 6D | 73 | 2E | 74 | 65 | 73 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 74 | 2E | 33 | 67 | 70 | 70 | 2E | 63 | 6F | 6D |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

Note:	This EF does not apply for 3GPP and shall not be used by a terminal using a 3GPP access network or a 3GPP Interworking WLAN.

#### 27.22.2C.3.7 EFSMS (Short Message Service)

At least 10 records.

All records shall be empty.

Logically:	Status byte set to empty.

Record 1-x (x ≥10):

| Byte: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 | … | B176 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 00 | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF | … | FF |

#### 27.22.2C.3.8	EFSMSR (Short message status reports)

This EF shall contain as many records as EFSMS.

All records shall be empty.

- Logically:	Status byte set to empty.

Record 1-x (x ≥10):

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 00 | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

#### 27.22.2C.3.9	EFSMSP (Short message service parameters)

Logically:

Record 1:

Record length:	28 bytes

Parameter Indicators:

TP-Destination Address:	Parameter absent

TS-Service Centre Address:	Parameter present

TP-Protocol Identifier:	Parameter absent

TP-Data Coding Scheme:	Parameter absent

TP-Validity Period:		Parameter absent

TS-Service Centre Address:

TON:	International Number

NPI:	"ISDN / telephone numbering plan"

- Dialled number string:	"112233445566778"

| Byte: | B1 | B2 | B3 | ... | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 | B21 | B22 | B23 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | FD | FF | FF | ... | FF | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 |
|  | B24 | B25 | B26 | B27 | B28 |  |  |  |  |  |  |  |  |  |  |
|  | FF | FF | FF | FF | FF |  |  |  |  |  |  |  |  |  |  |

- All other records shall be empty.

#### 27.22.2C.3.10	EFSMSS (SMS Status)

Logically:	Last used TP-MR set to"00".

- Memory capacity available (flag unset b1="1").

| Byte: | B1 | B2 |
| --- | --- | --- |
| Coding: | 00 | FF |

#### 27.22.2C.4	Default values at DF_TELECOM

#### 27.22.2C.4.1	EFPSISMSC (Public Service Identity of the SM-SC)

1 record only.

Logically:

Record 1:

Public Service Identity of the SM-SC: tel:+112233445566778

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 14 | 74 | 65 | 6C | 3A | 2B | 31 | 31 | 32 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 32 | 33 | 33 | 34 | 34 | 35 | 35 | 36 | 36 | 37 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | … | Bxx |
|  | 37 | 38 | FF | FF | FF | FF | FF | FF | … | FF |

### 27.22.2D	Definition of default values for NG-RAN related USIM Application Toolkit testing

#### 27.22.2D.1	Definition of NG-RAN UICC

For each item, the logical default values and the coding within the Elementary Files (EF) of the USIM follow, as defined in clause 27.22.2B of the present document with the following exceptions:

EFUST (USIM Service Table)

Logically:

User controlled PLMN selector available

Fixed dialling numbers available

The GSM Access available

The Group Identifier level 1 and level 2 not available

Service n 33 (Packed Switched Domain) shall be set to '1'

Enabled Services Table available

EPS Mobility Management Information available

Allowed CSG Lists and corresponding indications

5GS Mobility Management Information available

5G Security Parameters available

Subscription identifier privacy support available

| Byte: | B1 |  | B2 |  | B3 |  |  | B4 |  | B5 |  |  | B6 |  | B7 |  | B8 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Binary: | xxxx xx1x |  | xxxx xxxx |  | xxxx 1x00 |  |  | xxxx x1xx |  | xxxx xx11 |  |  | xxxx xxxx |  | xxxx xxxx |  | xxxx xxxx |  |  |
|  | B9 |  | B10 |  | B11 |  |  |  |  | B16 |  |  |  |  |  |  |  |  |  |
|  | xxxx xxxx |  | xxxx xxxx |  | xx11 xxxx |  |  | ..... |  | xxx0 111x |  |  |  |  |  |  |  |  |  |

The coding of EFUST shall conform with the capabilities of the USIM used.

EF5GS3GPPLOCI (5GS 3GPP location information)

Logically:

5G-GUTI:	FF FF FF FF FF FF FF FF FF FF

TAI:		246 081 000000

GS update status:	5U2 NOT UPDATED

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | FF | FF | FF | FF | FF | FF | FF | FF |
|  | B9 | B10 | B11 | B12 | B13 | B14 | B15 | B16 |
|  | FF | FF | FF | FF | FF | 42 | 16 | 80 |
|  | B17 | B18 | B19 | B20 |  |  |  |  |
|  | 00 | 00 | 00 | 01 |  |  |  |  |

EFSUCI_Calc_Info (Subscription Concealed Identifier Calculation Information EF)

Logically:

All bytes ‘FF’

EFRouting_Indicator (Routing Indicator EF)

Logically:

Routing Indicator: 17

| Coding: | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Hex | 71 | FF | FF | FF |

EF5GS3GPPNSC (5GS 3GPP Access NAS Security Context EF)

Logically:

5GS NAS Security Context:

ngKSI:				00

KAMF: 			32 bytes, value not checked

Uplink NAS count:			any value

Downlink NAS count:			any value

Identifiers of selected NAS		any value

integrity and encryption algorithms:

Identifiers of selected EPS NAS		any value

integrity and encryption algorithms

for use after mobility to EPS:

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | Bx |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | A0 | XX | 80 | 01 | 00 | 81 | xx | xx | … | xx |

#### 27.22.2D.2	Definition of NG-RAN cell parameters

The default NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	Cell Identity value = 0001 (36 bits);

### 27.22.1	Initialization of USIM Application Toolkit Enabled UICC by USIM Application Toolkit Enabled ME (Profile Download)

#### 27.22.1.1	Definition and applicability

See clause 3.2.2.

#### 27.22.1.2	Conformance requirement

The ME shall support the PROFILE DOWNLOAD command as defined in:

-	TS 31.111 [15] clause 5.2.

#### 27.22.1.3	Test purpose

To verify that the ME sends a TERMINAL PROFILE command in accordance with the above requirements.

#### 27.22.1.4	Method of test

##### 27.22.1.4.1	Initial conditions

The ME is connected to the USIM Simulator. All elementary files are coded as the default Toolkit personalization..

##### 27.22.1.4.2	Procedure

Expected Sequence 1 (PROFILE DOWNLOAD)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Power on ME | [UICC Activation] |
| 2 | ME UICC | Select EF PL |  |
| 3 | UICC ME | Read EF PL |  |
| 4 | ME UICC | TERMINAL PROFILE 1.1 | PROFILE DOWNLOAD |
| 5 | UICC ME | NORMAL ENDING OF COMMAND 1.1 |  |
| 6 | ME UICC | Select USIM Application |  |

TERMINAL PROFILE: 1.1

Logically:

Coding:

| APDU: | CLA=80 | INS=10 | P1=00 | P2=00 | P3=XX |
| --- | --- | --- | --- | --- | --- |

| DATA IN: | YY | ZZ | … |
| --- | --- | --- | --- |

With XX representing the length of the following DATA IN depending on the USIM Toolkit commands supported by the ME, and with YY, ZZ, … representing here the bytes of the TERMINAL PROFILE data, as specified in TS 31.111 [15], clause 5.2.

NORMAL ENDING OF COMMAND: 1.1

Logically:

Coding:

| SW1=90 | SW2=00 |
| --- | --- |

#### 27.22.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.

### 27.22.2	Contents of the TERMINAL PROFILE command

#### 27.22.2.1	Definition and applicability

See table E.1 in annex B.

#### 27.22.2.2	Conformance requirement

The ME shall support the PROFILE DOWNLOAD command as defined in:

-	TS 31.111 [15] clause 5.2.

#### 27.22.2.3	Test purpose

1.	Verify that the TERMINAL PROFILE indicates that Profile Download facility is supported.

2.	Record which USIM Application Toolkit facilities are supported by the ME, to determine which subsequent tests are required.

#### 27.22.2.4	Method of test

##### 27.22.2.4.1	Initial conditions

The ME is connected to the USIM Simulator. All elementary files are coded as the default USIM Application Toolkit personalization.

##### 27.22.1.4.2	Procedure

a)	The ME is powered on.

b)	After the ME sends the TERMINAL PROFILE command to the USIM Simulator, the USIM Simulator shall record the content of the TERMINAL PROFILE.

c)	The USIM Simulator shall return SW1 / SW2 of '90 00'.

d)	The contents of the TERMINAL PROFILE is recorded and compared to the corresponding table E.1 "status" column.

The test is terminated upon the ME sending the TERMINAL PROFILE command to the USIM Simulator.

#### 27.22.2.5	Test requirement

1)	After step a) the ME shall send the TERMINAL PROFILE command to the USIM Simulator with bit 1 of the first byte set to 1 (facility supported by ME).

2)	In table E.1 for the corresponding ME USIM Toolkit Release and Options, The TERMINAL PROFILE information "support" recorded must be in accordance with the "Status" column. Support of features defined only in releases later than currently tested release shall be ignored.

### 27.22.3	Servicing of proactive UICC commands

#### 27.22.3.1	Definition and applicability

See clause 3.2.2.

#### 27.22.3.2	Conformance requirement

On detection of a pending USIM Application Toolkit command from the UICC the ME shall perform the FETCH command to retrieve the proactive UICC command. The result of the executed command shall be transmitted from the ME to the UICC within a TERMINAL RESPONSE command.

The MORE TIME proactive command is used in this test. The ME shall have knowledge of this command, but may not support this USIM Application Toolkit facility.

-	TS 31.111 [15] clause 6.3.

#### 27.22.3.3	Test purpose

To verify that the ME uses the FETCH command to obtain the proactive UICC command, after detection of a pending proactive UICC command. The pending proactive UICC command is indicated by the response parameters '91 xx' from the UICC.

To verify that the ME transmits the result of execution of the proactive UICC command to the UICC in the TERMINAL RESPONSE command.

#### 27.22.3.4	Method of test

##### 27.22.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as the USIM Application Toolkit default.

The USIM Simulator is configured to indicate that a proactive UICC command is pending.

The USIM Simulator is configured to monitor the UICC - ME interface.

##### 27.22.3.4.2	Procedure

a)	The ME is powered on.

b)	After the ME has performed the PROFILE DOWNLOAD procedure, the USIM Simulator indicates that a Proactive UICC Command is pending with SW1 / SW2 of '91 0B'.

c)	After the ME sends the FETCH command to the USIM Simulator, the USIM Simulator returns Proactive UICC Command 2.1: MORE TIME.

#### 27.22.3.5	Test requirement

1)	After step b) the ME shall send the FETCH command to the UICC.

2)	After step c) the ME shall send the TERMINAL REPONSE command with command number "01", type of command "02" and command qualifier "00".

### 27.22.4	Proactive UICC commands

#### 27.22.4.1	DISPLAY TEXT

##### 27.22.4.1.1	DISPLAY TEXT (Normal)

27.22.4.1.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.1.2	Conformance requirements

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

TS 31.111 [15], clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

27.22.4.1.1.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.1.4	Method of test

27.22.4.1.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.1.4.2	Procedure

Expected Sequence 1.1 (DISPLAY TEXT normal priority, Unpacked 8 bit data for Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (DISPLAY TEXT normal priority, Unpacked 8 bit data for Text String, screen busy)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (DISPLAY TEXT, high priority, Unpacked 8 bit data for Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.3.

Expected Sequence 1.4 (DISPLAY TEXT, Packed, SMS default alphabet, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.4.

Expected Sequence 1.5 (DISPLAY TEXT, Clear message after delay, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.5.

Expected Sequence 1.6 (DISPLAY TEXT, Text string with 160 bytes, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.6.

Expected Sequence 1.7 (DISPLAY TEXT, Backward move in UICC session, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.7.

Expected Sequence 1.8 (DISPLAY TEXT, session terminated by user)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.8.

Expected Sequence 1.9 (DISPLAY TEXT, icon and text to be displayed, no text string given, not understood by ME)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.1.4.2, Expected Sequence 1.9.

27.22.4.1.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.9.

##### 27.22.4.1.2	DISPLAY TEXT (Support of "No response from user")

27.22.4.1.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.2.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.1.2.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, and returns a "No response from user" result value in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.2.4	Method of test

27.22.4.1.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

ME Manufacturers shall set the "no response from user" period of time as declared in table A.2/1..

The USIM simulator shall be set to that period of time.

27.22.4.1.2.4.2	Procedure

Expected Sequence 2.1 (DISPLAY TEXT, no response from user)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.2.4.2, Expected Sequence.

2.1.27.22.4.1.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.1.3	DISPLAY TEXT (Display of extension text)

27.22.4.1.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.3.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.15.

27.22.4.1.3.3	Test purpose

To verify that the ME displays the extension text contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.3.4	Method of test

27.22.4.1.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.3.4.2	Procedure

Expected Sequence 3.1 (DISPLAY TEXT, display of the extension text)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.3.4.2, Expected Sequence 3.1.

27.22.4.1.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.1.4	DISPLAY TEXT (Sustained text)

27.22.4.1.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.4.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.15, clause 8.15.

27.22.4.1.4.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, returns a successful result in the TERMINAL RESPONSE command send to the UICC and sustain the display beyond sending the TERMINAL response.

27.22.4.1.4.4	Method of test

27.22.4.1.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.4.4.2	Procedure

Expected Sequence 4.1 (DISPLAY TEXT, sustained text, unpacked data 8 bits, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.4.4.2, Expected Sequence 4.1.

Expected Sequence 4.2 (DISPLAY TEXT, sustained text, clear message after delay, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.4.4.2, Expected Sequence 4.2.

Expected Sequence 4.3 (DISPLAY TEXT, sustained text, wait for user MMI to clear, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.4.4.2, Expected Sequence 4.3.

Expected Sequence 4.4 (DISPLAY TEXT, sustained text, wait for high priority event to clear, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: DISPLAY TEXT 4.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: DISPLAY TEXT 4.4.1 | [wait for user to clear message] |
| 4 | ME USER | Display "Toolkit Test 4" |  |
| 5 | ME UICC | TERMINAL RESPONSE: DISPLAY TEXT 4.4.1 | [Command performed successfully] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | ME USER | Display of "Toolkit Test 4" | Text shall sustain until - a higher priority event occurs. |
| 8 | USS ME | INCOMING MOBILE TERMINATED CALL |  |

PROACTIVE COMMAND: DISPLAY TEXT 4.4.1

Logically:

Command details

Command number:	1

Command type:	DISPLAY TEXT

Command qualifier:	normal priority, wait for user to clear message

Device identities

Source device:	UICC

Destination device:	Display

Text String

Data coding scheme:	unpacked, 8 bit data

Text:	"Toolkit Test 4"

Immediate Response

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 21 | 80 | 82 | 02 | 81 | 02 | 8D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0F | 04 | 54 | 6F | 6F | 6C | 6B | 69 | 74 | 20 | 54 | 65 |
|  | 73 | 74 | 20 | 34 | AB | 00 |  |  |  |  |  |  |

TERMINAL RESPONSE: DISPLAY TEXT 4.4.1

Logically:

Command details

Command number:	1

Command type:	DISPLAY TEXT

Command qualifier:	normal priority, wait for user to clear message

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 21 | 80 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.1.4.5	Test requirement

The ME shall operate in the manner defined in expected sequences 4.1 to 4.4.

##### 27.22.4.1.5	DISPLAY TEXT (Display of icons)

27.22.4.1.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.5.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

27.22.4.1.5.3	Test purpose

To verify that the ME displays the icons which are referred to in the contents of the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.5.4	Method of test

27.22.4.1.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME screen shall be in its normal stand-by display.

27.22.4.1.5.4.2	Procedure

Expected Sequence 5.1A (DISPLAY TEXT, display of basic icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.5.4.2, Expected Sequence 5.1A.

Expected Sequence 5.1B (DISPLAY TEXT, display of basic icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.5.4.2, Expected Sequence 5.1B.

Expected Sequence 5.2A (DISPLAY TEXT, display of colour icon, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.5.4.2, Expected Sequence 5.2A.

Expected Sequence 5.2B (DISPLAY TEXT, display of colour icon, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.5.4.2, Expected Sequence 5.2B.

Expected Sequence 5.3A (DISPLAY TEXT, display of basic icon, not self explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.5.4.2, Expected Sequence 5.3A.

Expected Sequence 5.3B (DISPLAY TEXT, display of basic icon, not self explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.5.4.2, Expected Sequence 5.3B.27.22.4.1.5.5	Test requirement

The ME shall operate in the manner defined in expected sequences 5.1A to 5.3B.

##### 27.22.4.1.6	DISPLAY TEXT (UCS2 display in Cyrillic)

27.22.4.1.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.6.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

The ME shall support the UCS2 alphabet for the coding of the Cyrillic alphabet, as defined in the following technical specification: ISO/IEC 10646 [17].

27.22.4.1.6.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.6.4	Method of test

27.22.4.1.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.6.4.2	Procedure

Expected Sequence 6.1 (DISPLAY TEXT, UCS2 coded in Cyrillic)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.6.4.2, Expected Sequence 6.1.

27.22.4.1.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

##### 27.22.4.1.7	DISPLAY TEXT (Variable Time out)

27.22.4.1.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.7.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31 and clause 8.43.

The ME shall support the variable time out for the display text.

27.22.4.1.7.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.7.4	Method of test

27.22.4.1.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.7.4.2	Procedure

Expected Sequence 7.1 (DISPLAY TEXT, variable timeout of 10 seconds)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.7.4.2, Expected Sequence 7.1.

27.22.4.1.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.1.8	DISPLAY TEXT (Support of Text Attribute)

27.22.4.1.8.1	DISPLAY TEXT (Support of Text Attribute – Left Alignment)

27.22.4.1.8.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.1.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with Left Alignment for the display text.

27.22.4.1.8.1.3	Test purpose

To verify that the ME displays the text formatted according to the left alignment text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.1.4	Method of test

27.22.4.1.8.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.1.4.2	Procedure

Expected Sequence 8.1 (DISPLAY TEXT, Text Attribute with Left Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.1.4.2, Expected Sequence 8.1.

27.22.4.1.8.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1.

27.22.4.1.8.2	DISPLAY TEXT (Support of Text Attribute – Center Alignment)

27.22.4.1.8.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.2.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with Centre Alignment for the display text.

27.22.4.1.8.2.3	Test purpose

To verify that the ME displays the text formatted according to the center alignment text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.2.4	Method of test

27.22.4.1.8.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.2.4.2	Procedure

Expected Sequence 8.2 (DISPLAY TEXT, Text Attribute with Center Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.2.4.2, Expected Sequence 8.2.

27.22.4.1.8.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.2.

27.22.4.1.8.3	DISPLAY TEXT (Support of Text Attribute – Right Alignment)

27.22.4.1.8.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.3.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with Right Alignment for the display text.

27.22.4.1.8.3.3	Test purpose

To verify that the ME displays the text formatted according to the right alignment text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.3.4	Method of test

27.22.4.1.8.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.3.4.2	Procedure

Expected Sequence 8.3 (DISPLAY TEXT, Text Attribute with Right Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.3.4.2, Expected Sequence 8.3.

27.22.4.1.8.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.3.

27.22.4.1.8.4	DISPLAY TEXT (Support of Text Attribute – Large Font Size)

27.22.4.1.8.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.4.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with large font size for the display text.

27.22.4.1.8.4.3	Test purpose

To verify that the ME displays the text formatted according to the large size font text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.4.4	Method of test

27.22.4.1.8.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.4.4.2	Procedure

Expected Sequence 8.4 (DISPLAY TEXT, Text Attribute with Large Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.4.4.2, Expected Sequence 8.4.

27.22.4.1.8.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.4.

27.22.4.1.8.5	DISPLAY TEXT (Support of Text Attribute – Small Font Size)

27.22.4.1.8.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.5.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with small font size for the display text.

27.22.4.1.8.5.3	Test purpose

To verify that the ME displays the text formatted according to the small size font text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.5.4	Method of test

27.22.4.1.8.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.5.4.2	Procedure

Expected Sequence 8.5 (DISPLAY TEXT, Text Attribute with Small Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.5.4.2, Expected Sequence 8.5.

27.22.4.1.8.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.5.

27.22.4.1.8.6	DISPLAY TEXT (Support of Text Attribute – Bold On)

27.22.4.1.8.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.6.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with bold on for the display text.

27.22.4.1.8.6.3	Test purpose

To verify that the ME displays the text formatted according to the bold text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.6.4	Method of test

27.22.4.1.8.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.6.4.2	Procedure

Expected Sequence 8.6 (DISPLAY TEXT, Text Attribute with Bold On)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.6.4.2, Expected Sequence 8.6.

27.22.4.1.8.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.6.

27.22.4.1.8.7	DISPLAY TEXT (Support of Text Attribute – Italic On)

27.22.4.1.8.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.7.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with italic on for the display text.

27.22.4.1.8.7.3	Test purpose

To verify that the ME displays the text formatted according to the italic text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.7.4	Method of test

27.22.4.1.8.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.7.4.2	Procedure

Expected Sequence 8.7 (DISPLAY TEXT, Text Attribute with Italic On)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.7.4.2, Expected Sequence 8.7.

27.22.4.1.8.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.7.

27.22.4.1.8.8	DISPLAY TEXT (Support of Text Attribute – Underline On)

27.22.4.1.8.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.8.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with underline on for the display text.

27.22.4.1.8.8.3	Test purpose

To verify that the ME displays the text formatted according to the underline text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.8.4	Method of test

27.22.4.1.8.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.8.4.2	Procedure

Expected Sequence 8.8 (DISPLAY TEXT, Text Attribute with Underline On)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.8.4.2, Expected Sequence 8.8.

27.22.4.1.8.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.8.

27.22.4.1.8.9	DISPLAY TEXT (Support of Text Attribute – Strikethrough On)

27.22.4.1.8.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.9.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with underline on for the display text.

27.22.4.1.8.9.3	Test purpose

To verify that the ME displays the text formatted according to the strikethrough text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.9.4	Method of test

27.22.4.1.8.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.9.4.2	Procedure

Expected Sequence 8.9 (DISPLAY TEXT, Text Attribute with Strikethrough On)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.9.4.2, Expected Sequence 8.9.

27.22.4.1.8.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.9.

27.22.4.1.8.10	DISPLAY TEXT (Support of Text Attribute – Foreground and Background Colours)

27.22.4.1.8.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.8.10.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.31, clause 8.43 and clause 8.70.

The ME shall support the text attribute with different foreground and background colours for the display text.

27.22.4.1.8.10.3	Test purpose

To verify that the ME displays the text formatted according to the foreground and background colour text attribute configuration contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.8.10.4	Method of test

27.22.4.1.8.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.8.10.4.2	Procedure

Expected Sequence 8.10 (DISPLAY TEXT, Text Attribute with Foreground and Background Colours)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.8.10.4.2, Expected Sequence 8.10.

27.22.4.1.8.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.10.

##### 27.22.4.1.9	DISPLAY TEXT (UCS2 display in Chinese)

27.22.4.1.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.9.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

The ME shall support the UCS2 alphabet for the coding of the Chinese characters, as defined in the following technical specification: ISO/IEC 10646 [17].

27.22.4.1.9.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.9.4	Method of test

27.22.4.1.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.9.4.2	Procedure

Expected Sequence 9.1 (DISPLAY TEXT, UCS2 coded – Chinese characters)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.9.4.2, Expected Sequence 9.1.

27.22.4.1.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.1.

##### 27.22.4.1.10	DISPLAY TEXT (UCS2 display in Katakana)

27.22.4.1.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.1.10.2	Conformance requirement

The ME shall support the DISPLAY TEXT command as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.1, clause 6.5.4, clause 6.6.1, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

The ME shall support the UCS2 alphabet for the coding of the Katakana characters, as defined in the following technical specification: ISO/IEC 10646 [17].

27.22.4.1.10.3	Test purpose

To verify that the ME displays the text contained in the DISPLAY TEXT proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.1.10.4	Method of test

27.22.4.1.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.1.10.4.2	Procedure

Expected Sequence 10.1 (DISPLAY TEXT, UCS2 coded – Katakana characters)

See ETSI TS 102 384 [26] in subclause 27.22.4.1.10.4.2, Expected Sequence 10.1.

27.22.4.1.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 10.1.

#### 27.22.4.2	GET INKEY

##### 27.22.4.2.1	GET INKEY(normal)

27.22.4.2.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.1.2	Conformance Requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.2.1.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the single character entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.1.4	Method of test

27.22.4.2.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be set to a display other than the idle display.

27.22.4.2.1.4.2	Procedure

Expected Sequence 1.1 (GET INKEY, digits only for character, Unpacked 8 bit data for Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (GET INKEY, digits only for character set, SMS default Alphabet for Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (GET INKEY, backward move)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.1.4.2, Expected Sequence 1.3.

Expected Sequence 1.4 (GET INKEY, abort)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.1.4.2, Expected Sequence 1.4.

Expected Sequence 1.5 (GET INKEY, SMS default alphabet for character set, Unpacked 8 bit data for Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.1.4.2, Expected Sequence 1.5.

Expected Sequence 1.6 (GET INKEY, Max length for the Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.1.4.2, Expected Sequence 1.6.

27.22.4.2.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.6.

##### 27.22.4.2.2	GET INKEY (No response from User)

27.22.4.2.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.2.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.2.2.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns a "No response from user" result value in the TERMINAL RESPONSE command send to the UICC.

27.22.4.2.2.4	Method of test

27.22.4.2.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

ME Manufacturers shall set the "no response from user" period of time as declared in table A.2/2.

The USIM Simulator shall be set to that period of time.

27.22.4.2.2.4.2	Procedure

Expected Sequence 2.1 (GET INKEY, no response from the user)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.2.4.2, Expected Sequence 2.1.

27.22.4.2.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.2.3	GET INKEY (UCS2 display in Cyrillic)

27.22.4.2.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.3.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally, the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.2.3.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.3.4	Method of test

27.22.4.2.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.3.4.2	Procedure

Expected Sequence 3.1 (GET INKEY, Text String coding in UCS2 Alphabet in Cyrillic, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.3.4.2, Expected Sequence 3.1.

Expected Sequence 3.2 (GET INKEY, max length for the Text String coding in UCS2 Alphabet in Cyrillic, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.3.4.2, Expected Sequence 3.2.

27.22.4.2.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1 to 3.2.

##### 27.22.4.2.4	GET INKEY (UCS2 entry in Cyrillic)

27.22.4.2.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.4.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally, the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.2.4.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.4.4	Method of test

27.22.4.2.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.4.4.2	Procedure

Expected Sequence 4.1 (GET INKEY, characters from UCS2 alphabet in Cyrillic, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.4.4.2, Expected Sequence 4.1.

27.22.4.2.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

##### 27.22.4.2.5	GET INKEY ("Yes/No" Response)

27.22.4.2.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.5.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.2.5.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.5.4	Method of test

27.22.4.2.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.5.4.2	Procedure

Expected Sequence 5.1(GET INKEY, "Yes/No" Response for the input, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.5.4.2, Expected Sequence 5.1.

27.22.4.2.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.2.6	GET INKEY (display of Icon)

27.22.4.2.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.6.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

27.22.4.2.6.3	Test purpose

To verify that the ME displays the Icon contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.6.4	Method of test

27.22.4.2.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME screen shall be in its normal stand-by display.

27.22.4.2.6.4.2	Procedure

Expected Sequence 6.1A (GET INKEY, Basic icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.1A.

Expected Sequence 6.1B (GET INKEY, Basic icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.1B.

Expected Sequence 6.2A (GET INKEY, Basic icon, non self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.2A.

Expected Sequence 6.2B (GET INKEY, Basic icon, non self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.2B.

Expected Sequence 6.3A (GET INKEY, Colour icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.3A.

Expected Sequence 6.3B (GET INKEY, Colour icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.3B.

Expected Sequence 6.4A (GET INKEY, Colour icon, non self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.4A.

Expected Sequence 6.4B (GET INKEY, Colour icon, non self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.6.4.2, Expected Sequence 6.4B.

27.22.4.2.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1A to 6.4B.

##### 27.22.4.2.7	GET INKEY (Help Information)

27.22.4.2.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.7.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

27.22.4.2.7.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.7.4	Method of test

27.22.4.2.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.7.4.2	Procedure

Expected Sequence 7.1 (GET INKEY, help information available)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.7.4.2, Expected Sequence 7.1.

27.22.4.2.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.2.8	GET INKEY (Variable Time out)

27.22.4.2.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.8.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.31.

27.22.4.2.8.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.8.4	Method of test

27.22.4.2.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.8.4.2	Procedure

Expected Sequence 8.1 (GET INKEY, variable time out of 10 seconds)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.8.4.2, Expected Sequence 8.1.

27.22.4.2.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1.

##### 27.22.4.2.9	GET INKEY (Support of Text Attribute)

27.22.4.2.9.1	GET INKEY (Support of Text Attribute – Left Alignment)

27.22.4.2.9.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.1.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.1.3	Test purpose

To verify that the ME displays the text formatted according to the left alignment text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.1.4	Method of test

27.22.4.2.9.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.1.4.2	Procedure

Expected Sequence 9.1 (GET INKEY, Text attribute with Left Alignment )

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.1.4.2, Expected Sequence 9.1.

27.22.4.2.9.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.1.

27.22.4.2.9.2	GET INKEY (Support of Text Attribute – Center Alignment)

27.22.4.2.9.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.2.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.2.3	Test purpose

To verify that the ME displays the text formatted according to the center alignment text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.2.4	Method of test

27.22.4.2.9.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.2.4.2	Procedure

Expected Sequence 9.2 (GET INKEY, Text attribute with Center Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.2.4.2, Expected Sequence 9.2.

27.22.4.2.9.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.2.

27.22.4.2.9.3	GET INKEY (Support of Text Attribute – Right Alignment)

27.22.4.2.9.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.3.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.3.3	Test purpose

To verify that the ME displays the text formatted according to the right alignment text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.3.4	Method of test

27.22.4.2.9.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.3.4.2	Procedure

Expected Sequence 9.3 (GET INKEY, Text attribute with Right Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.3.4.2, Expected Sequence 9.3.

27.22.4.2.9.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.3.

27.22.4.2.9.4	GET INKEY (Support of Text Attribute – Large Font Size)

27.22.4.2.9.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.4.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.4.3	Test purpose

To verify that the ME displays the text formatted according to the large font size text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.4.4	Method of test

27.22.4.2.9.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.4.4.2	Procedure

Expected Sequence 9.4 (GET INKEY, Text attribute with Large Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.4.4.2, Expected Sequence 9.4.

27.22.4.2.9.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.4.

27.22.4.2.9.5	GET INKEY (Support of Text Attribute – Small Font Size)

27.22.4.2.9.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.5.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.5.3	Test purpose

To verify that the ME displays the text formatted according to the small font size text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.5.4	Method of test

27.22.4.2.9.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.5.4.2	Procedure

Expected Sequence 9.5 (GET INKEY, Text attribute with Small Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.5.4.2, Expected Sequence 9.5.

27.22.4.2.9.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.5.

27.22.4.2.9.6	GET INKEY (Support of Text Attribute – Bold On)

27.22.4.2.9.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.6.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.6.3	Test purpose

To verify that the ME displays the text formatted according to the bold text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.6.4	Method of test

27.22.4.2.9.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.6.4.2	Procedure

Expected Sequence 9.6 (GET INKEY, Text attribute with Bold On)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.6.4.2, Expected Sequence 9.6.

27.22.4.2.9.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.6.

27.22.4.2.9.7	GET INKEY (Support of Text Attribute – Italic On)

27.22.4.2.9.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.7.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.7.3	Test purpose

To verify that the ME displays the text formatted according to the italic text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.7.4	Method of test

27.22.4.2.9.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.7.4.2	Procedure

Expected Sequence 9.7 (GET INKEY, Text attribute with Italic On)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.7.4.2, Expected Sequence 9.7.

27.22.4.2.9.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.7.

27.22.4.2.9.8	GET INKEY (Support of Text Attribute – Underline On)

27.22.4.2.9.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.8.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.8.3	Test purpose

To verify that the ME displays the text formatted according to the underline text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.8.4	Method of test

27.22.4.2.9.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.8.4.2	Procedure

Expected Sequence 9.8 (GET INKEY, Text attribute with Underline On)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.8.4.2, Expected Sequence 9.8.

27.22.4.2.9.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.8.

27.22.4.2.9.9	GET INKEY (Support of Text Attribute – Strikethrough On)

27.22.4.2.9.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.9.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.9.3	Test purpose

To verify that the ME displays the text formatted according to the strikethrough text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.9.4	Method of test

27.22.4.2.9.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.9.4.2	Procedure

Expected Sequence 9.9 (GET INKEY, Text attribute with Strikethrough On)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.9.4.2, Expected Sequence 9.9.

27.22.4.2.9.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.9.

27.22.4.2.9.10	GET INKEY (Support of Text Attribute – Foreground and Background Colour)

27.22.4.2.9.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.9.10.2	Conformance requirement

The ME shall support the GET INKEY command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.5.4, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.8, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3, clause 8.31 and clause 8.70.

27.22.4.2.9.10.3	Test purpose

To verify that the ME displays the text formatted according to the foreground and background colour text attribute configuration contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.9.10.4	Method of test

27.22.4.2.9.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.2.9.10.4.2	Procedure

Expected Sequence 9.10 (GET INKEY, Text attribute with Foreground and Background Colour)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.9.10.4.2, Expected Sequence 9.10.

27.22.4.2.9.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.10.

##### 27.22.4.2.10	GET INKEY (UCS2 display in Chinese)

27.22.4.2.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.10.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally, the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.2.10.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.10.4	Method of test

27.22.4.2.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.10.4.2	Procedure

Expected Sequence 10.1 (GET INKEY, Text String coding in UCS2 Alphabet - Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.10.4.2, Expected Sequence 10.1.

Expected Sequence 10.2 (GET INKEY, max length for the Text String coding in UCS2 Alphabet - Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.10.4.2, Expected Sequence 10.2.

27.22.4.2.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 10.1 to 10.2.

##### 27.22.4.2.11	GET INKEY (UCS2 entry in Chinese)

27.22.4.2.11.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.11.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally, the ME shall support the UCS2 facility for the coding of the Chinese character, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.2.11.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.11.4	Method of test

27.22.4.2.11.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.11.4.2	Procedure

Expected Sequence 11.1 (GET INKEY, characters from UCS2 alphabet - Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.11.4.2, Expected Sequence 11.1.

27.22.4.2.11.5	Test requirement

The ME shall operate in the manner defined in expected sequence 11.1

##### 27.22.4.2.12	GET INKEY (UCS2 display in Katakana)

27.22.4.2.12.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.12.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally, the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.2.12.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.12.4	Method of test

27.22.4.2.12.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.12.4.2	Procedure

Expected Sequence 12.1 (GET INKEY, Text String coding in UCS2 Alphabet - Katakana characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.12.4.2, Expected Sequence 12.1.

Expected Sequence 12.2 (GET INKEY, max length for the Text String coding in UCS2 Alphabet - Katakana characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.12.4.2, Expected Sequence 12.2.

27.22.4.2.12.5	Test requirement

The ME shall operate in the manner defined in expected sequence 12.1 to 12.2.

##### 27.22.4.2.13	GET INKEY (UCS2 entry in Katakana)

27.22.4.2.13.1	Definition and applicability

See clause 3.2.2.

27.22.4.2.13.2	Conformance requirement

The ME shall support the GET INKEY command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.2, clause 6.6.2, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally, the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.2.13.3	Test purpose

To verify that the ME displays the text contained in the GET INKEY proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.2.13.4	Method of test

27.22.4.2.13.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.2.13.4.2	Procedure

Expected Sequence 13.1 (GET INKEY, characters from UCS2 alphabet - Katakana characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.2.13.4.2, Expected Sequence 13.1.

27.22.4.2.13.5	Test requirement

The ME shall operate in the manner defined in expected sequence 13.1

#### 27.22.4.3	GET INPUT

##### 27.22.4.3.1	GET INPUT (normal)

27.22.4.3.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.1.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.3.1.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.1.4	Method of test

27.22.4.3.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.1.4.2	Procedure

Expected Sequence 1.1 (GET INPUT, digits only, SMS default alphabet, ME to echo text, ME supporting 8 bit data Message)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (GET INPUT, digits only, SMS default alphabet, ME to echo text, packing SMS Point-to-point required by ME)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (GET INPUT, character set, SMS Default Alphabet, ME to echo text, ME supporting 8 bit data Message)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.3.

Expected Sequence 1.4 (GET INPUT, digits only, SMS default alphabet, ME to hide text, ME supporting 8 bit data Message)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.4.

Expected Sequence 1.5 (GET INPUT, digits only, SMS default alphabet, ME to echo text, ME supporting 8 bit data Message)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.5.

Expected Sequence 1.6 (GET INPUT, backwards move)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.6.

Expected Sequence 1.7 (GET INPUT, abort)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.7.

Expected Sequence 1.8 (GET INPUT, digits only, SMS default alphabet, ME to echo text, ME supporting 8 bit data Message)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.8.

Expected Sequence 1.9 (GET INPUT, digits only, SMS default alphabet, ME to echo text, ME supporting 8 bit data Message)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.9.

Expected Sequence 1.10 (GET INPUT, null length for the text string, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.1.4.2, Expected Sequence 1.10.

27.22.4.3.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.10.

##### 27.22.4.3.2	GET INPUT (No response from User)

27.22.4.3.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.2.2	Conformance requirement

The ME shall support the GET INPUT command as defined in the following technical specifications :

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.3.2.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns a "No response from user" result value in the TERMINAL RESPONSE command send to the UICC.

27.22.4.3.2.4	Method of test

27.22.4.3.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

ME Manufacturers shall set the "no response from user" period of time as declared in table A.2/3.

The USIM Simulator shall be set to that period of time.

27.22.4.3.2.4.2	Procedure

Expected Sequence 2.1 (GET INPUT, no response from the user)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.2.4.2, Expected Sequence 2.1.

27.22.4.3.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.3.3	GET INPUT (UCS2 display in Cyrillic)

27.22.4.3.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.3.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.3.3.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.3.4	Method of test

27.22.4.3.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.3.4.2	Procedure

Expected Sequence 3.1 (GET INPUT, text string coding in UCS2 in Cyrillic, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.3.4.2, Expected Sequence 3.1.

Expected Sequence 3.2 (GET INPUT, max length for the text string coding in UCS2 in Cyrillic, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.3.4.2, Expected Sequence 3.2.

27.22.4.3.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.2.

##### 27.22.4.3.4	GET INPUT (UCS2 entry in Cyrillic)

27.22.4.3.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.4.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in ISO/IEC 10646 [17].

27.22.4.3.4.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.4.4	Method of test

27.22.4.3.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.4.4.2	Procedure

Expected Sequence 4.1 (GET INPUT, character set from UCS2 alphabet in Cyrillic, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.4.4.2, Expected Sequence 4.1.

Expected Sequence 4.2 (GET INPUT, character set from UCS2 alphabet in Cyrillic, Max length for the input, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.4.4.2, Expected Sequence 4.2.

27.22.4.3.4.5	Test requirement

The ME shall operate in the manner defined in expected sequences 4.1 to 4.2.

##### 27.22.4.3.5	GET INPUT (default text)

27.22.4.3.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.5.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.23.

27.22.4.3.5.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.5.4	Method of test

27.22.4.3.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.5.4.2	Procedure

Expected Sequence 5.1(GET INPUT, default text for the input, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.5.4.2, Expected Sequence 5.1.

Expected Sequence 5.2 (GET INPUT, default text for the input with max length, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.5.4.2, Expected Sequence 5.2.

27.22.4.3.5.5	Test requirement

The ME shall operate in the manner defined in expected sequences 5.1 to 5.2.

##### 27.22.4.3.6	GET INPUT (display of Icon)

27.22.4.3.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.6.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.5.4, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 12.31.

27.22.4.3.6.3	Test purpose

To verify that the ME displays the Icon contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.6.4	Method of test

27.22.4.3.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME screen shall be in its normal stand-by display.

27.22.4.3.6.4.2	Procedure

Expected Sequence 6.1A (GET INPUT, Basic icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.1A.

Expected Sequence 6.1B (GET INPUT, Basic icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.1B.

Expected Sequence 6.2A (GET INPUT, Basic icon, non self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.2A.

Expected Sequence 6.2B (GET INPUT, Basic icon, non self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.2B.

Expected Sequence 6.3A (GET INPUT, Colour icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.3A.

Expected Sequence 6.3B (GET INPUT, Colour icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.3B.

Expected Sequence 6.4A (GET INPUT, Colour icon, non self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.4A.

Expected Sequence 6.4B (GET INPUT, Colour icon, non self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.6.4.2, Expected Sequence 6.4B.

27.22.4.3.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 6.1A to 6.4B.

##### 27.22.4.3.7	GET INPUT (Help Information)

27.22.4.3.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.7.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

27.22.4.3.7.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns a 'help information required by the user' result value in the TERMINAL RESPONSE command sent to the UICC if the user has indicated the need to get help information.

27.22.4.3.7.4	Method of test

27.22.4.3.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.7.4.2	Procedure

Expected Sequence 7.1 (GET INPUT, digits only, ME to echo text, ME supporting 8 bit data Message, help information available)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.7.4.2, Expected Sequence 7.1.

27.22.4.3.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.3.8	GET INPUT (Support of Text Attribute)

27.22.4.3.8.1	GET INPUT (Support of Text Attribute – Left Alignment)

27.22.4.3.8.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.1.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.1.3	Test purpose

To verify that the ME displays the text formatted according to the left alignment text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.1.4	Method of test

27.22.4.3.8.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.1.4.2	Procedure

Expected Sequence 8.1 (GET INPUT, Text attribute – Left Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.1.4.2, Expected Sequence 8.1.

27.22.4.3.8.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1.

27.22.4.3.8.2	GET INPUT (Support of Text Attribute – Center Alignment)

27.22.4.3.8.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.2.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.2.3	Test purpose

To verify that the ME displays the text formatted according to the center alignment text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.2.4	Method of test

27.22.4.3.8.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.2.4.2	Procedure

Expected Sequence 8.2 (GET INPUT, Text attribute – Center Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.2.4.2, Expected Sequence 8.2.

27.22.4.3.8.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.2.

27.22.4.3.8.3	GET INPUT (Support of Text Attribute – Right Alignment)

27.22.4.3.8.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.3.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.3.3	Test purpose

To verify that the ME displays the text formatted according to the right alignment text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.3.4	Method of test

27.22.4.3.8.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.3.4.2	Procedure

Expected Sequence 8.3 (GET INPUT, Text attribute – Right Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.3.4.2, Expected Sequence 8.3.

27.22.4.3.8.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.3.

27.22.4.3.8.4	GET INPUT (Support of Text Attribute – Large Font Size)

27.22.4.3.8.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.4.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.4.3	Test purpose

To verify that the ME displays the text formatted according to the large font size text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.4.4	Method of test

27.22.4.3.8.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.4.4.2	Procedure

Expected Sequence 8.4 (GET INPUT, Text attribute – Large Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.4.4.2, Expected Sequence 8.4.

27.22.4.3.8.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.4.

27.22.4.3.8.5	GET INPUT (Support of Text Attribute – Small Font Size)

27.22.4.3.8.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.5.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.5.3	Test purpose

To verify that the ME displays the text formatted according to the small font size text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.5.4	Method of test

27.22.4.3.8.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.5.4.2	Procedure

Expected Sequence 8.5 (GET INPUT, Text attribute – Small Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.5.4.2, Expected Sequence 8.5.

27.22.4.3.8.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.5.

27.22.4.3.8.6	GET INPUT (Support of Text Attribute – Bold On)

27.22.4.3.8.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.6.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.6.3	Test purpose

To verify that the ME displays the text formatted according to the bold text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.6.4	Method of test

27.22.4.3.8.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.6.4.2	Procedure

Expected Sequence 8.6 (GET INPUT, Text attribute – Bold On)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.6.4.2, Expected Sequence 8.6.

27.22.4.3.8.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.6.

27.22.4.3.8.7	GET INPUT (Support of Text Attribute – Italic On)

27.22.4.3.8.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.7.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.7.3	Test purpose

To verify that the ME displays the text formatted according to the italic text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.7.4	Method of test

27.22.4.3.8.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.7.4.2	Procedure

Expected Sequence 8.7 (GET INPUT, Text attribute – Italic On)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.7.4.2, Expected Sequence 8.7.

27.22.4.3.8.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.7.

27.22.4.3.8.8	GET INPUT (Support of Text Attribute – Underline On)

27.22.4.3.8.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.8.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.8.3	Test purpose

To verify that the ME displays the text formatted according to the underline text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.8.4	Method of test

27.22.4.3.8.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.8.4.2	Procedure

Expected Sequence 8.8 (GET INPUT, Text attribute – Underline On)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.8.4.2, Expected Sequence 8.8.

27.22.4.3.8.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.8.

27.22.4.3.8.9	GET INPUT (Support of Text Attribute – Strikethrough On)

27.22.4.3.8.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.9.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.9.3	Test purpose

To verify that the ME displays the text formatted according to the strikethrough text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.9.4	Method of test

27.22.4.3.8.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.9.4.2	Procedure

Expected Sequence 8.9 (GET INPUT, Text attribute – Strikethrough On)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.9.4.2, Expected Sequence 8.9.

27.22.4.3.8.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.9.

27.22.4.3.8.10	GET INPUT (Support of Text Attribute – Foreground and Background Colour)

27.22.4.3.8.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.8.10.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2, clause 8.15.3 and clause 8.70.

27.22.4.3.8.10.3	Test purpose

To verify that the ME displays the text formatted according to the fore- and background colour text attribute configuration contained in the GET INPUT proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.8.10.4	Method of test

27.22.4.3.8.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.3.8.10.4.2	Procedure

Expected Sequence 8.10 (GET INPUT, Text attribute – Foreground and Background Colour)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.8.10.4.2, Expected Sequence 8.10.

27.22.4.3.8.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.10.

##### 27.22.4.3.9	GET INPUT (UCS2 display in Chinese)

27.22.4.3.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.9.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally the ME shall support the UCS2 facility for the coding of the Chinese character, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.3.9.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.9.4	Method of test

27.22.4.3.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.9.4.2	Procedure

Expected Sequence 9.1 (GET INPUT, text string coding in UCS2 - Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.9.4.2, Expected Sequence 9.1.

Expected Sequence 9.2 (GET INPUT, max length for the text string coding in UCS2 - Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.9.4.2, Expected Sequence 9.2.

27.22.4.3.9.5	Test requirement

The ME shall operate in the manner defined in expected sequences 9.1 to 9.2

##### 27.22.4.3.10	GET INPUT (UCS2  entry in Chinese)

27.22.4.3.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.10.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally the ME shall support the UCS2 facility for the coding of the Chinese character, as defined in ISO/IEC 10646 [17].

27.22.4.3.10.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.10.4	Method of test

27.22.4.3.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.10.4.2	Procedure

Expected Sequence 10.1 (GET INPUT, character set from UCS2 alphabet - Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.10.4.2, Expected Sequence 10.1.

Expected Sequence 10.2 (GET INPUT, character set from UCS2 alphabet - Chinese characters, Max length for the input, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.10.4.2, Expected Sequence 10.2.

27.22.4.3.10.5	Test requirement

The ME shall operate in the manner defined in expected sequences 10.1 to 10.2

##### 27.22.4.3.11	GET INPUT (UCS2 display in Katakana)

27.22.4.3.11.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.11.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.3.11.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.11.4	Method of test

27.22.4.3.11.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.11.4.2	Procedure

Expected Sequence 11.1 (GET INPUT, text string coding in UCS2 in Katakana, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.11.4.2, Expected Sequence 11.1.

Expected Sequence 11.2 (GET INPUT, max length for the text string coding in UCS2 in Katakana, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.11.4.2, Expected Sequence 11.2.

27.22.4.3.11.5	Test requirement

The ME shall operate in the manner defined in expected sequences 11.1 to 11.2

##### 27.22.4.3.12	GET INPUT (UCS2  entry in Katakana)

27.22.4.3.12.1	Definition and applicability

See clause 3.2.2.

27.22.4.3.12.2	Conformance requirement

The ME shall support the GET INPUT command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.11, clause 8.15, clause 8.15.1, clause 8.15.2 and clause 8.15.3.

Additionally the ME shall support the UCS2 facility for the coding of the Chinese character, as defined in ISO/IEC 10646 [17].

27.22.4.3.12.3	Test purpose

To verify that the ME displays the text contained in the GET INPUT proactive UICC command, and returns the text string entered in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.3.12.4	Method of test

27.22.4.3.12.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.3.12.4.2	Procedure

Expected Sequence 12.1 (GET INPUT, character set from UCS2 alphabet in Katakana, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.12.4.2, Expected Sequence 12.1.

Expected Sequence 12.2 (GET INPUT, character set from UCS2 alphabet in Katakana, Max length for the input, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.3.12.4.2, Expected Sequence 12.2.

27.22.4.3.12.5	Test requirement

The ME shall operate in the manner defined in expected sequences 12.1 to 12.2.

#### 27.22.4.4	MORE TIME

27.22.4.4.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.4.2	Conformance requirement

The ME shall support the MORE TIME command as defined in:

-	TS 31.111 [15] clause 6.4.4, clause 6.6.4, clause 5.2, clause 8.6 and clause 8.7.

##### 27.22.4.4.3	Test purpose

To verify that the ME shall send a TERMINAL RESPONSE (OK) to the UICC after the ME receives the MORE TIME proactive UICC command.

##### 27.22.4.4.4	Method of test

27.22.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.4.4.2	Procedure

Expected Sequence 1.1 (MORE TIME)

See ETSI TS 102 384 [26] in subclause 27.22.4.4.4.2, Expected Sequence 1.1.

##### 27.22.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.4.5	PLAY TONE

##### 27.22.4.5.1	PLAY TONE (Normal)

27.22.4.5.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.1.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16 and clause 8.8.

27.22.4.5.1.3	Test purpose

To verify that the ME plays an audio tone of a type and duration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME plays the requested audio tone through the earpiece whilst not in call and shall superimpose the tone on top of the downlink audio whilst in call.

To verify that the ME displays the text contained in the PLAY TONE proactive UICC command.

27.22.4.5.1.4	Method of test

27.22.4.5.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.1.4.2	Procedure

Expected Sequence 1.1 (PLAY TONE)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.1 |  |
| 4 | ME USER | Display "Dial Tone" Play a standard supervisory dial tone through the external ringer for a duration of 5 s |  |
| 5 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.1 | [Command performed successfully] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.2 |  |
| 10 | ME USER | Display "Sub. Busy" Play a standard supervisory called subscriber busy tone for a duration of 5 s |  |
| 11 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.2 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.3 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.3 |  |
| 16 | ME USER | Display "Congestion" Play a standard supervisory congestion tone for a duration of 5 s |  |
| 17 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.3 | [Command performed successfully] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.4 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.4 |  |
| 22 | ME USER | Display "RP Ack" Play a standard supervisory radio path acknowledgement tone |  |
| 23 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.4 | [Command performed successfully] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 25 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.5 |  |
| 26 | ME UICC | FETCH |  |
| 27 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.5 |  |
| 28 | ME USER | Display "No RP" Play a standard supervisory radio path not available / call dropped tone for a duration of 5 s | [Note: The ME will only play three bursts as specified in TS 22.001 [2]] |
| 29 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.5 | [Command performed successfully] |
| 30 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.6 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.6 |  |
| 34 | ME USER | Display "Spec Info" Play a standard supervisory error / special information tone for a duration of 5 s |  |
| 35 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.6 | [Command performed successfully] |
| 36 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.7 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.7 |  |
| 40 | ME USER | Display "Call Wait" Play a standard supervisory call waiting tone for a duration of 5 s |  |
| 41 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.7 | [Command performed successfully] |
| 42 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.8 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.8 |  |
| 46 | ME USER | Display "Ring Tone" Play a standard supervisory ringing tone for duration of 5 s |  |
| 47 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.8 | [Command performed successfully] |
| 48 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 49 | USER ME | Set up a voice call | [ User dials 123456789 to connect to the network manually] |
| 50 | ME USS | Establish voice call | [Voice call is established] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.1 |  |
| 54 | ME USER | Display "Dial Tone" Superimpose the standard supervisory dial tone on the audio downlink for the duration of 5 s |  |
| 55 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.1 | [Command performed successfully] |
| 56 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 57 | USER ME | The user ends the call |  |
| 58 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.9 |  |
| 59 | ME UICC | FETCH |  |
| 60 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.9 |  |
| 61 | ME USER | Display "This command instructs the ME to play an audio tone. Upon receiving this command, the ME shall check if it is currently in, or in the process of setting up (SET-UP message sent to the network, see GSM"04.08"(8)), a speech call. - If the ME I" Play a general beep |  |
| 62 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.9a or TERMINAL RESPONSE: PLAY TONE 1.1.9b | [Command performed successfully] or [Command beyond ME's capabilities] |
| 63 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.10 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.10 |  |
| 67 | ME USER | Display "Beep" Play a ME proprietary general beep |  |
| 68 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.10a Or TERMINAL RESPONSE: PLAY TONE 1.1.10b | [Command performed successfully] or [Command beyond ME's capabilities] |
| 69 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 70 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.11 |  |
| 71 | ME UICC | FETCH |  |
| 72 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.11 |  |
| 73 | ME USER | Display "Positive" Play a ME proprietary positive acknowledgement tone |  |
| 74 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.11a or TERMINAL RESPONSE: PLAY TONE 1.1.11b | [Command performed successfully] or [Command beyond ME's capabilities] |
| 75 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 76 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.12 |  |
| 77 | ME UICC | FETCH |  |
| 78 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.12 |  |
| 79 | ME USER | Display "Negative" Play a ME proprietary negative acknowledgement tone |  |
| 80 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.12a or TERMINAL RESPONSE: PLAY TONE 1.1.12b | [Command performed successfully] or [Command beyond ME's capabilities] |
| 81 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 82 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.13 |  |
| 83 | ME UICC | FETCH |  |
| 84 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.13 |  |
| 85 | ME USER | Display "Quick" Play a ME proprietary general beep |  |
| 86 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.13a or TERMINAL RESPONSE: PLAY TONE 1.1.13b | [Command performed successfully] or [Command beyond ME's capabilities] |
| 87 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 88 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.14 |  |
| 89 | ME UICC | FETCH |  |
| 90 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.14 |  |
| 91 | ME USER | Display "<ABORT>" Play an ME Error / Special information tone until user aborts this command (the command shall be aborted by the user within 1 minute) |  |
| 92 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.14 | [Proactive UICC session terminated by the user] |
| 93 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 94 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.1.15 |  |
| 95 | ME UICC | FETCH |  |
| 96 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.1.15 | [No alpha identifier, no tone tag, no duration tag] |
| 97 | ME User | ME plays general beep, or if not supported any (defined by ME-manufacturer) other supported tone | [ME uses default duration defined by ME-manufacturer] |
| 98 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.1.15 | [Command performed successfully], [ME uses general beep, or if not supported any (defined by ME-manufacturer) other supported tone, uses default duration defined by ME-manufacturer] |
| 99 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

For coding, see ETSI TS 102 384 [26] in subclause 27.22.4.5.1.4.2, Expected Sequence 1.1.

27.22.4.5.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

##### 27.22.4.5.2	PLAY TONE (UCS2 display in Cyrillic)

27.22.4.5.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.2.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.16 and clause 8.8.

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in ISO/IEC 10646 [17].

27.22.4.5.2.3	Test purpose

To verify that the ME displays the text contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME plays the requested audio tone through the earpiece.

27.22.4.5.2.4	Method of test

27.22.4.5.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.2.4.2	Procedure

Expected Sequence 2.1 (PLAY TONE, character set from UCS2 alphabet in Russian, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.2.4.2, Expected Sequence 2.1.

27.22.4.5.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.5.3	PLAY TONE (display of Icon)

27.22.4.5.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.3.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8 and clause 8.31.

27.22.4.5.3.3	Test purpose

To verify that the ME plays an audio tone of a type and duration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME plays the requested audio tone through the earpiece.

To verify that the ME displays the icon contained in the PLAY TONE proactive UICC command.

27.22.4.5.3.4	Method of test

27.22.4.5.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.3.4.2	Procedure

Expected Sequence 3.1A (PLAY TONE, Basic icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.1A.

Expected Sequence 3.1B (PLAY TONE, Basic icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.1B.

Expected Sequence 3.2A (PLAY TONE, Basic icon, non self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.2A.

Expected Sequence 3.2B (PLAY TONE, Basic icon, non self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.2B.

Expected Sequence 3.3A (PLAY TONE, Colour icon, self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.3A.

Expected Sequence 3.3B (PLAY TONE, Colour icon, self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.3B.

Expected Sequence 3.4A (PLAY TONE, Colour icon, non self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.4A.

Expected Sequence 3.4B (PLAY TONE, Colour icon, non self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.3.4.2, Expected Sequence 3.4B.

27.22.4.5.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 3.1A to 3.4B.

##### 27.22.4.5.4	PLAY TONE (Support of Text Attribute)

27.22.4.5.4.1	PLAY TONE (Support of Text Attribute – Left Alignment)

27.22.4.5.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.1.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.1.3	Test purpose

To verify that the ME displays the text formatted according to the left alignment text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.1.4	Method of test

27.22.4.5.4.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.1.4.2	Procedure

Expected Sequence 4.1 (PLAY TONE, Text Attribute – Left Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.1.4.2, Expected Sequence 4.1.

27.22.4.5.4.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.1.

27.22.4.5.4.2	PLAY TONE (Support of Text Attribute – Center Alignment)

27.22.4.5.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.2.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.2.3	Test purpose

To verify that the ME displays the text formatted according to the center alignment text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.2.4	Method of test

27.22.4.5.4.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.2.4.2	Procedure

Expected Sequence 4.2 (PLAY TONE, Text Attribute – Centre Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.2.4.2, Expected Sequence 4.2.

27.22.4.5.4.2.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.2.

27.22.4.5.4.3	PLAY TONE (Support of Text Attribute – Right Alignment)

27.22.4.5.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.3.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.3.3	Test purpose

To verify that the ME displays the text formatted according to the right alignment text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.3.4	Method of test

27.22.4.5.4.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.3.4.2	Procedure

Expected Sequence 4.3 (PLAY TONE, Text Attribute – Right Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.3.4.2, Expected Sequence 4.3.

27.22.4.5.4.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.3.

27.22.4.5.4.4	PLAY TONE (Support of Text Attribute – Large Font Size)

27.22.4.5.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.4.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.4.3	Test purpose

To verify that the ME displays the text formatted according to the large font size text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.4.4	Method of test

27.22.4.5.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.4.4.2	Procedure

Expected Sequence 4.4 (PLAY TONE, Text Attribute – Large Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.4.4.2, Expected Sequence 4.4.

27.22.4.5.4.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.4.

27.22.4.5.4.5	PLAY TONE (Support of Text Attribute – Small Font Size)

27.22.4.5.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.5.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.5.3	Test purpose

To verify that the ME displays the text formatted according to the small font size text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.5.4	Method of test

27.22.4.5.4.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.5.4.2	Procedure

Expected Sequence 4.5 (PLAY TONE, Text Attribute – Small Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.5.4.2, Expected Sequence 4.5.

27.22.4.5.4.5.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.5.

27.22.4.5.4.6	PLAY TONE (Support of Text Attribute – Bold On)

27.22.4.5.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.6.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.6.3	Test purpose

To verify that the ME displays the text formatted according to the bold text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.6.4	Method of test

27.22.4.5.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.6.4.2	Procedure

Expected Sequence 4.6 (PLAY TONE, Text Attribute – Bold On)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.6.4.2, Expected Sequence 4.6.

27.22.4.5.4.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.6.

27.22.4.5.4.7	PLAY TONE (Support of Text Attribute – Italic On)

27.22.4.5.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.7.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.7.3	Test purpose

To verify that the ME displays the text formatted according to the italic text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.7.4	Method of test

27.22.4.5.4.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.7.4.2	Procedure

Expected Sequence 4.7 (PLAY TONE, Text Attribute – Italic On)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.7.4.2, Expected Sequence 4.7.

27.22.4.5.4.7.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.7.

27.22.4.5.4.8	PLAY TONE (Support of Text Attribute – Underline On)

27.22.4.5.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.8.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.8.3	Test purpose

To verify that the ME displays the text formatted according to the underline text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.8.4	Method of test

27.22.4.5.4.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.8.4.2	Procedure

Expected Sequence 4.8 (PLAY TONE, Text Attribute – Underline On)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.8.4.2, Expected Sequence 4.8.

27.22.4.5.4.8.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.8.

27.22.4.5.4.9	PLAY TONE (Support of Text Attribute – Strikethrough On)

27.22.4.5.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.9.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.9.3	Test purpose

To verify that the ME displays the text formatted according to the strikethrough text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.9.4	Method of test

27.22.4.5.4.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.9.4.2	Procedure

Expected Sequence 4.9 (PLAY TONE, Text Attribute – Strikethrough On)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.9.4.2, Expected Sequence 4.9.

27.22.4.5.4.9.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.9.

27.22.4.5.4.10	PLAY TONE (Support of Text Attribute – Foreground and Background Colour)

27.22.4.5.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.4.10.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.5, clause 6.6.5, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.16, clause 8.8, clause 8.31 and clause 8.70.

27.22.4.5.4.10.3	Test purpose

To verify that the ME displays the text formatted according to the foreground and background colour text attribute configuration contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.5.4.10.4	Method of test

27.22.4.5.4.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.4.10.4.2	Procedure

Expected Sequence 4.10 (PLAY TONE, Text Attribute – Foreground and Background Colour)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.4.10.4.2, Expected Sequence 4.10.

27.22.4.5.4.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.10.

##### 27.22.4.5.5	PLAY TONE (UCS2 display in Chinese)

27.22.4.5.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.5.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.16 and clause 8.8.

Additionally the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in ISO/IEC 10646 [17].

27.22.4.5.5.3	Test purpose

To verify that the ME displays the text contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME plays the requested audio tone through the earpiece.

27.22.4.5.5.4	Method of test

27.22.4.5.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.5.4.2	Procedure

Expected Sequence 5.1 (PLAY TONE, character set from UCS2 alphabet in Chinese, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.5.4.2, Expected Sequence 5.1.

27.22.4.5.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.5.6	PLAY TONE (UCS2 display in Katakana)

27.22.4.5.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.5.6.2	Conformance requirement

The ME shall support the PLAY TONE command as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.3, clause 6.6.3, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.16 and clause 8.8.

Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in ISO/IEC 10646 [17].

27.22.4.5.6.3	Test purpose

To verify that the ME displays the text contained in the PLAY TONE proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME plays the requested audio tone through the earpiece.

27.22.4.5.6.4	Method of test

27.22.4.5.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.5.6.4.2	Procedure

Expected Sequence 6.1 (PLAY TONE, with UCS2 in Katakana, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.5.6.4.2, Expected Sequence 6.1.

27.22.4.5.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

#### 27.22.4.6	POLL INTERVAL

##### 27.22.4.6.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.6.2	Conformance requirement

The ME shall support the POLL INTERVAL command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.6, clause 6.6.6, clause 5.2, clause 8.6, clause 8.7 and clause 8.8.

##### 27.22.4.6.3	Test purpose

To verify that the ME shall send a TERMINAL RESPONSE (OK) to the UICC after the ME receives the POLL INTERVAL proactive UICC command.

To verify that the ME gives a valid response to the polling interval requested by the UICC.

To verify that the ME sends STATUS commands to the UICC at an interval no longer than the interval negotiated by the UICC.

##### 27.22.4.6.4	Method of test

27.22.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.6.4.2	Procedure

See ETSI TS 102 384 [26] in subclause 27.22.4.6.4.2, Expected Sequence 1.1.

Note:	If the requested poll interval is not supported by the ME, the ME is allowed to use a different one as stated in TS 31.111 [15], subclause 6.4.6.

27.22.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.4.7	REFRESH

##### 27.22.4.7.1	REFRESH (normal)

27.22.4.7.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.1.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.7, clause 6.6.13, clause 5.2, clause 8.6, clause 8.7 and clause 8.18.

Consequently the ME shall support the USIM Initialization procedure as defined in:

-	TS 31.102 [14] clause 5.1.1.2 and ETSI TS 102 221[13] clause 11.1.2

27.22.4.7.1.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier. This shall require the ME to perform:

-	the UICC and USIM initialization,

-	a re-read of the contents and structure of the EFs on the UICC that have been notified as changed and are either part of initialization or used during the test,

-	a restart of the card session,

-	a successfull return of the result of the execution of the command in the TERMINAL RESPONSE command send to the UICC.

27.22.4.7.1.4	Method of test

27.22.4.7.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table..

The elementary files are coded as Toolkit default except for expected sequence 1.3.

For expected sequence 1.3 the global phonebook shall be present.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

These values might be overwritten with values defined in the expected sequences itself.

Prior to the execution of expected sequence 1.2 the FDN service shall be enabled.

27.22.4.7.1.4.2	Procedure

Expected Sequence 1.1 (REFRESH, USIM Initialization)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.1.1 | [To inform the ME that FDN becomes enabled] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 |  |
| 4 | UICC | EF EST contents states FDN enabled | [New EF EST value: 01] |
| 5 | ME UICC | USIM Initialization including send STATUS[P1='01'] | [ME performs USIM initialization in accordance with TS 31.111 [15] clause 6.4.7] |
| 6 | ME UICC | TERMINAL RESPONSE: REFRESH 1.1.1A Or TERMINAL RESPONSE: REFRESH 1.1.1B | [normal ending] [additional EFs read] |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 8 | USER ME | Call setup to "321" |  |
| 9 | ME USER | Call set up not allowed |  |
| 10 | USER ME | Call setup to "123" |  |
| 11 | ME USS | Setup | Called party BCD number shall be "123" |

PROACTIVE COMMAND: REFRESH 1.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 81 | 82 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.1.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.1.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (REFRESH, File Change Notification)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.2.1 | [To inform the ME that EF FDN will be in an updated state, FDN service already enabled] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 1.2.1 |  |
| 4 | UICC | Update EF FDN RECORD 1 | [EF FDN record 1 updated to contain the dialling string "0123456789"] |
| 5 | ME UICC | TERMINAL RESPONSE: REFRESH 1.2.1A Or TERMINAL RESPONSE: REFRESH 1.2.1B | [normal ending] [additional EFs read] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | USER ME | Call setup to "123" |  |
| 8 | ME USER | Call set up not allowed |  |
| 9 | USER ME | Call setup to "0123456789" |  |
| 10 | ME USS | Setup | Called party BCD number shall be "0123456789" |

PROACTIVE COMMAND: REFRESH 1.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

File List:	EF FDN

Coding:

| BER-TLV: | D0 | 12 | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 01 | 3F | 00 | 7F | FF | 6F | 3B |  |  |  |  |

TERMINAL RESPONSE: REFRESH 1.2.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.2.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.3 (REFRESH, USIM Initialization and File Change Notification)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 1.3.1 |  |
| 4 | UICC | Update EF ADN in the global phonebook | [EF ADN entry 1 of the global phonebook to contain the the new and previously unused alpha identifier "Changed" |
| 5 | ME UICC | USIM Initialization including sending STATUS [P1='01'] | [ME performs USIM initialization in accordance with TS 31.111 [15] clause 6.4.7] |
| 6 | ME UICC | TERMINAL RESPONSE: REFRESH 1.3.1A Or TERMINAL RESPONSE: REFRESH 1.3.1B | [normal ending] [additional EFs read] |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 8 | USER ME | Use an MMI dependent procedure to display the entry with the alpha identifier "Changed" stored in record 1 of EF ADN in the global phonebook | [To ensure that EF ADN in the global phonebook has been read after issuing the Refresh command] |
| 9 | ME USER | The ME shall display the alpha identifier "Changed" for record 1 of EF ADN in the global phonebook |  |

PROACTIVE COMMAND: REFRESH 1.3.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

File List:	ADN in the global phonebook

Coding:

| BER-TLV: | D0 | 12 | 81 | 03 | 01 | 01 | 02 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 1 |  |  |  |  |  |  |  |  |  |  |  |

Note 1:	Length and data of the file list TLV depend on the card configuration used in this test. The global phonebook shall be used. The number of changed files shall be set to '01'.

TERMINAL RESPONSE: REFRESH 1.3.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and File Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.3.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and File Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.4 (REFRESH, USIM Initialization and Full File Change Notification)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 1.4.1 |  |
| 4 | UICC | EF EST contents states FDN enabled | [New EF EST value: 01] |
| 5 | UICC | Update EF FDN | [EF FDN record 1 updated to contain the dialling string "0123456789"] |
| 6 | ME UICC | USIM Initialization including send STATUS[P1='01'] | [ME performs USIM initialization in accordance with TS 31.111 [15] clause 6.4.7] |
| 7 | ME UICC | TERMINAL RESPONSE: REFRESH 1.4.1A Or TERMINAL RESPONSE: REFRESH 1.4.1B | [normal ending] [additional EFs read] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | Call setup to "321" |  |
| 10 | ME USER | Call set up not allowed |  |
| 11 | USER ME | Call setup to "0123456789" |  |
| 12 | ME USS | Setup | Called party BCD number shall be "0123456789" |

PROACTIVE COMMAND: REFRESH 1.4.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and Full File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.4.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and Full file Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.4.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and full File change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.5 (REFRESH, UICC Reset)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 1.5.1 |  |
| 4 | ME UICC | STATUS[P1='02'] | ME indicates to USIM that the termination procedure is starting |
| 5 | ME UICC | ME resets the UICC, performs USIM initialisation, including send STATUS[P1='01'] and no TERMINAL RESPONSE shall be sent |  |

PROACTIVE COMMAND: REFRESH 1.5.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC Reset

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.6 (REFRESH, USIM Initialization after SMS-PP data download)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME shall be in its normal idle mode | [Start a sequence to verify that the ME returns the RP-ACK message back to the USS, if the UICC responds with '90 00'] |
| 2 | USS ME | SMS-PP Data Download Message 1.6.1 |  |
| 3 | ME USER | The ME shall not display the message or alert the user of a short message waiting |  |
| 4 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.6.1 |  |
| 5 | UICC ME | SW1/SW2 of '90 00' |  |
| 6 | ME  USS | RP-ACK |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.1.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 |  |
| 10 | UICC | EF EST contents states FDN enabled | [New EF EST value: 01] |
| 11 | ME UICC | USIM Initialization including send STATUS[P1='01'] | [ME performs USIM initialization in accordance with TS 31.111 [15] clause 6.4.7] |
| 12 | ME UICC | TERMINAL RESPONSE: REFRESH 1.1.1A Or TERMINAL RESPONSE: REFRESH 1.1.1B | [normal ending] [additional EFs read] |
| 13 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 14 | USER ME | Call setup to "321" |  |
| 15 | ME USER | Call set up not allowed |  |
| 16 | USER ME | Call setup to "123" |  |
| 17 | ME USS | Setup | Called party BCD number shall be "123" |

SMS-PP (Data Download) Message 1.6.1

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| Coding | 04 | 04 | 91 | 21 | 43 | 7F | 16 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 53 | 68 | 6F | 72 | 74 | 20 | 4D | 65 | 73 |
|  | 73 | 61 | 67 | 65 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 1.6.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 21 | 43 |
|  | 7F | 16 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 53 | 68 |
|  | 6F | 72 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |  |

Expected Sequence 1.7 (REFRESH, USIM Application Reset)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.7.1 | [To inform the ME that FDN becomes enabled] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 1.7.1 | No UICC reset shall be performed between steps 3 and 9. |
| 4 | ME UICC | STATUS[P1='02'] | ME indicates to USIM that the termination procedure is starting |
| 5 | ME UICC | Select AID=USIM (P2='44') OR (P2='4C') | Application termination |
| 6 | UICC | EF EST contents states FDN enabled | [New EF EST value: 01] |
| 7 | ME UICC | USIM Initialization, including send STATUS[P1='01'] | [ME performs USIM initialization] |
| 8 | ME UICC | TERMINAL RESPONSE: REFRESH 1.7.1 | [normal ending] |
| 9 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 10 | USER ME | Call setup to "321" |  |
| 11 | ME USER | Call set up not allowed |  |
| 12 | USER ME | Call setup to "123" |  |
| 13 | ME USS | Setup | Called party BCD number shall be "123" |
| 14 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 15 | USER ME | The user ends the call after a few seconds. |  |

PROACTIVE COMMAND: REFRESH 1.7.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Application Reset

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 05 | 82 | 02 | 81 | 82 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.7.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Application Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 05 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.7.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.

##### 27.22.4.7.2	REFRESH (IMSI changing procedure)

27.22.4.7.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.2.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.7, clause 6.4.7.1, clause 6, clause 6.6.13, clause 5.2, clause 8.6, clause 8.7 and clause 8.18.

Additionally the ME shall support the USIM Initialization and USIM application closure procedure as defined in:

-	TS 31.102 [14] clause 5.1.2 and Annex I.

27.22.4.7.2.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier and the IMSI changing procedure. This may require the ME to perform:

-	the USIM initialization

-	a re-read of the contents and structure of the IMSI on the USIM

-	a restart of the card session

-	a successful return of the result of the execution of the command in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.7.2.4	Method of test

27.22.4.7.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the USS and registered in idle mode.

The USS uses Network Mode of Operation II according to TS 34.108 [12] clause 7.2.2.

The GERAN or UTRAN parameters of the USS are:

-	Mobile Country Code (MCC) = 246;

-	Mobile Network Code (MNC) = 81;

-	Location Area Code (LAC) = 0001;

-	Routing Area Code (RAC) = 05;

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ATT flag broadcast in the SYSTEM INFORMATION BLOCK TYPE 1 on the BCCH is set to "UEs shall apply IMSI attach and detach procedure" for Expected Sequences 2.1 to 2.7.

27.22.4.7.2.4.2	Procedure

Expected Sequence 2.1 (REFRESH, UICC Reset for IMSI Changing procedure)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 2.1.1 | [To inform the ME that IMSI has changed] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 2.1.1 |  |
| 4 | ME USS | IMSI DETACH INDICATION and/or DETACH REQUEST | Indicates IMSI detach and/or GPRS detach, depending on if the ME is CS and/or PS registered according to its capabilities Note: this step can be performed in parallel or after step 5. |
| 5 | ME UICC | STATUS[P1='02'] | ME indicates to USIM that the termination procedure is starting |
| 6 | ME UICC | ME performs UICC reset | Both cold and warm resets are allowed |
| 7 | UICC | Update EF IMSI, EF LOCI and EF PSLOCI | Update the content of EF IMSI to "246813579", TMSI in EF LOCI and P-TMSI in EF PSLOCI be set to 'FF FF FF FF' |
| 8 | ME UICC | ME performs USIM Initialization, including send STATUS[P1='01'] and no TERMINAL RESPONSE shall be sent |  |
| 9 | ME USS | LOCATION UPDATING REQUEST and/or ATTACH REQUEST | The ME will register using IMSI "246813579" in CS and/or PS depending on its capabilities |
| 10 | USS ME | LOCATION UPDATING ACCEPT and/or ATTACH ACCEPT |  |
| 11 | ME USS | TMSI REALLOCATION COMPLETE and/or ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 2.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.2 (REFRESH, USIM Application Reset for IMSI Changing procedure )

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 2.2.1 | [To inform the ME that IMSI has changed] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 2.2.1 |  |
| 4 | MEUSS | IMSI DETACH INDICATION and/or DETACH REQUEST | Indicates IMSI detach and/or GPRS detach, depending on if the ME is CS and/or PS registered according to its capabilities (performed in parallel or after step 5 and 6) |
| 5 | ME UICC | STATUS[P1='02'] | ME indicates to USIM that the termination procedure is starting |
| 6 | ME UICC |  | Application termination |
| 7 | UICC | Update EF IMSI, EF LOCI and EF PSLOCI | The 3G session termination procedure has been completed by the ME. The content of EF IMSI has been updated to "246813579" and TMSI in EF LOCI and P-TMSI in EF PSLOCI are updated to 'FF FF FF FF' |
| 8 | ME UICC | SELECT AID=USIM (P2='0x') | Application selection |
| 9 | ME UICC | USIM Initialization, including send STATUS[P1='01'] | [ME performs USIM initialization] |
| 10 | ME UICC | TERMINAL RESPONSE: REFRESH 2.2.1 | [normal ending] |
| 11 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 12 | ME  USS | LOCATION UPDATING REQUEST and/or ATTACH REQUEST | The ME will register using IMSI "246813579" in CS and/or PS depending on its capabilities |
| 13 | USS  ME | LOCATION UPDATING ACCEPT and/or ATTACH ACCEPT |  |
| 14 | ME  USS | TMSI REALLOCATION COMPLETE and/or ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 2.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Application Reset

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 05 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 2.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Application Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 05 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.3 (REFRESH, 3G Session Reset for IMSI Changing procedure)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 2.3.1 | [To inform the ME that IMSI has changed] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 2.3.1 |  |
| 4 | MEUSS | IMSI DETACH INDICATION and/or DETACH REQUEST | Indicates IMSI detach and/or GPRS detach, depending on if the ME is CS and/or PS registered according to its capabilities Note: this step can be performed in parallel or after step 5. |
| 5 | ME UICC | STATUS[P1='02'] | If A.1/172 is supported, then the ME indicates to USIM that the termination procedure is starting,. completes the 3G session termination procedure and resets the application via SELECT by DF name command with the AID. The ME performs the USIM initialization. |
| 6 | UICC | Update EF IMSI, EF LOCI and EF PSLOCI | The content of EF IMSI has been updated to "246813579" and TMSI in EF LOCI and P-TMSI in EF PSLOCI are updated to 'FF FF FF FF' |
| 7 | ME UICC | TERMINAL RESPONSE: REFRESH 2.3.1A Or TERMINAL RESPONSE: REFRESH 2.3.1B | [normal ending] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | ME USS | LOCATION UPDATING REQUEST and/or ATTACH REQUEST | The ME will register using IMSI "246813579" in CS and/or PS depending on its capabilities |
| 10 | USS ME | LOCATION UPDATING ACCEPT and/or ATTACH ACCEPT |  |
| 11 | ME USS | TMSI REALLOCATION COMPLETE and/or ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 2.3.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	3

File:	EF IMSI

File:	EF PSLOCI

File:	EF LOCI

Coding:

| BER-TLV: | D0 | 1E | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 03 | 3F | 00 | 7F | FF | 6F | 07 | 3F | 00 | 7F | FF |
|  | 6F | 73 | 3F | 00 | 7F | FF | 6F | 7E |  |  |  |  |

TERMINAL RESPONSE: REFRESH 2.3.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 2.3.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.4 (REFRESH, reject 3G Session Reset for IMSI Changing procedure during CS call)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | MO Call setup |  |
| 2 | ME USS | Call established and maintained |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 2.4.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 2.4.1 |  |
| 6 | ME UICC | TERMINAL RESPONSE: REFRESH 2.4.1A Or TERMINAL RESPONSE: REFRESH 2.4.1B | ME rejects REFRESH proactive command |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED | Note: EF IMSI, EF LOCI and EF PSLOCI are not updated by the UICC, see TS 31.111[15], cl. 6.4.7.1 |
| 8 | USER ME | The MO call is terminated |  |

PROACTIVE COMMAND: REFRESH 2.4.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	3

File:	EF IMSI

File:	EF PSLOCI

File:	EF LOCI

Coding:

| BER-TLV: | D0 | 1E | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 03 | 3F | 00 | 7F | FF | 6F | 07 | 3F | 00 | 7F | FF |
|  | 6F | 73 | 3F | 00 | 7F | FF | 6F | 7E |  |  |  |  |

TERMINAL RESPONSE: REFRESH 2.4.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command
Additional information on result:	ME currently busy on call

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: REFRESH 2.4.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command
Additional information on result:	Screen is busy

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.5 (REFRESH, reject UICC Reset for IMSI Changing procedure during CS call)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | CS MO Call setup |  |
| 2 | ME USS | Call established and maintained |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 2.5.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 2.5.1 |  |
| 6 | ME UICC | TERMINAL RESPONSE: REFRESH 2.5.1A Or TERMINAL RESPONSE: REFRESH 2.5.1B | ME rejects REFRESH proactive command |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED | Note: EF IMSI, EF LOCI and EF PS LOCI are not updated by the UICC, see TS 31.111[15], cl. 6.4.7.1 |
| 8 | USER ME | The CS MO call is terminated |  |

PROACTIVE COMMAND: REFRESH 2.5.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 2.5.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command
Additional information on result:	ME currently busy on call

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: REFRESH 2.5.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command
Additional information on result:	Screen is busy

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.6 (REFRESH, UICC Reset for IMSI Changing procedure during active PDP context)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Data Call setup | PDP context will be established |
| 2 | ME USS | PDP context established and maintained |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH | [To inform the ME that IMSI has changed] |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 2.6.1 or 2.6.2 | IF terminal supports PD_ Refresh_Enforcement_Policy use PROACTIVE COMMAND: REFRESH 2.6.2, ELSE 2.6.1. |
| 6 | MEUSS | Deactivate PDP context | Mobile will deactivate the PDP context Note 1: this step is performed locally and may not reflect on the interface to the USS. Note 2: this step can happen after step 8. |
| 7 | MEUSS | IMSI DETACH INDICATION and/or DETACH REQUEST | Indicates IMSI detach and/or GPRS detach, depending on if the ME is CS and/or PS registered according to its capabilities. Note: this step can happen after step 8 |
| 8 | ME UICC | STATUS[P1='02'] | ME indicates to USIM that the termination procedure is starting |
| 9 | ME UICC | ME performs UICC reset | Both cold and warm resets are allowed |
| 10 | UICC | Update EF IMSI, EF LOCI and EF PSLOCI | The content of EF IMSI has been updated to "246813579" and TMSI in EF LOCI and P-TMSI in EF PSLOCI are updated to 'FF FF FF FF' |
| 11 | ME UICC | ME resets the UICC, perform USIM Initialization, including send STATUS[P1='01'] and no TERMINAL RESPONSE shall be sent | [ME resets and performs USIM initialization] |
| 12 | ME USS | LOCATION UPDATING REQUEST and/or ATTACH REQUEST | The ME will register using IMSI "246813579" in CS and/or PS depending on its capabilities |
| 13 | USS ME | LOCATION UPDATING ACCEPT and/or ATTACH ACCEPT |  |
| 14 | ME USS | TMSI REALLOCATION COMPLETE and/or ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 2.6.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 2.6.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Refresh enforcement policy : Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 |

Expected Sequence 2.7 (REFRESH, 3G Session Reset for IMSI Changing procedure during  active PDP context)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Data Call setup | PDP context will be established |
| 2 | ME USS | PDP context establishedand maintained |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH | [To inform the ME that IMSI has changed] |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 2.7.1 or 2.7.2 | IF terminal supports PD_ Refresh_Enforcement_Policy use PROACTIVE COMMAND: REFRESH 2.7.2, ELSE 2.7.1. |
| 6 | MEUSS | Deactivate PDP context | Mobile will deactivate the PDP context Note: this step can be performed in parallel or after step 8. |
| 7 | MEUSS | IMSI DETACH INDICATION and/or DETACH REQUEST | Indicates IMSI detach and/or GPRS detach, depending on if the ME is CS and/or PS registered according to its capabilities Note 1: this step is performed locally and may not reflect on the interface to the USS. Note 2: this step can be performed in parallel or after step 8. |
| 8 | ME UICC | STATUS[P1='02'] | If A.1/172 is supported, then the ME indicates to USIM that the termination procedure is starting, completes the 3G session termination procedure and resets the application via SELECT by DF name command with the AID. The ME performs the USIM initialization. |
| 9 | UICC | Update EF IMSI, EF LOCI and EF PSLOCI | The content of EF IMSI has been updated to "246813579" and TMSI in EF LOCI and P-TMSI in EF PSLOCI are updated to 'FF FF FF FF' |
| 10 | ME UICC | TERMINAL RESPONSE: REFRESH 2.7.1A Or TERMINAL RESPONSE: REFRESH 2.7.1B | [normal ending] |
| 11 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 12 | ME USS | LOCATION UPDATING REQUEST and/or ATTACH REQUEST | The ME will register using IMSI "246813579" in CS and/or PS depending on its capabilities |
| 13 | USS ME | LOCATION UPDATING ACCEPT and/or ATTACH ACCEPT |  |
| 14 | ME USS | TMSI REALLOCATION COMPLETE and/or ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 2.7.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	3

File:	EF IMSI

File:	EF PSLOCI

File:	EF LOCI

Coding:

| BER-TLV: | D0 | 1E | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 03 | 3F | 00 | 7F | FF | 6F | 07 | 3F | 00 | 7F | FF |
|  | 6F | 73 | 3F | 00 | 7F | FF | 6F | 7E |  |  |  |  |

PROACTIVE COMMAND: REFRESH 2.7.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	3

File:	EF IMSI

File:	EF PSLOCI

File:	EF LOCI

Refresh enforcement policy : Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 21 | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 03 | 3F | 00 | 7F | FF | 6F | 07 | 3F | 00 | 7F | FF |
|  | 6F | 73 | 3F | 00 | 7F | FF | 6F | 7E | 3A | 01 | 02 |  |

TERMINAL RESPONSE: REFRESH 2.7.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 2.7.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.7.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1 to 2.7.

##### 27.22.4.7.3	REFRESH (Steering of roaming)

27.22.4.7.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.3.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.7, clause 6.6.13, clause 5.2, clause 8.2, 8.6, clause 8.7 and clause 8.90.

Consequently the Rel-7 or later ME shall support the steering of roaming procedure as defined in:

-	TS 23.122 [29] clause 4.4.6.

27.22.4.7.3.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier. This shall require the ME to perform:

-	the steering of roaming procedure,

-	a successfull return of the result of the execution of the command in the TERMINAL RESPONSE command send to the UICC.

27.22.4.7.3.4	Method of test

27.22.4.7.3.4.1	Initial conditions

For sequences 3.1 and 3.2 the ME is connected to the USIM Simulator and connected to the USS/SS.

For sequence 3.3 the ME supporting E-UTRAN/NB-IoT is connected to the USIM Simulator and connected to the E-USS/NB-SS.

For sequences 3.1 and 3.2:

The elementary files are coded as Toolkit default with the following exceptions:

EFFPLMN

Logically:	PLMN1:	254 002 (MCC MNC)

PLMN2:	254 003

PLMN3:	254 004

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 00 | 52 | 34 | 00 | 52 | 44 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |  |  |
|  | 32 | 54 | 00 | 32 | 64 | 00 |  |  |  |  |  |  |

EFOPLMNwACT

Logically:	1st PLMN:	254 001 (MCC MNC)

1st ACT:	UTRAN

2nd PLMN:	254 001

2nd ACT:	GSM

3rd PLMN:	274 002

3rd ACT:	UTRAN

4th PLMN:	274 003

4th ACT:	UTRAN

5th PLMN:	274 004

5th ACT:	UTRAN

6th PLMN:	274 005

6th ACT:	UTRAN

7th PLMN:	274 006

7th ACT:	UTRAN

8th PLMN:	274 007

8th ACT:	UTRAN

| Coding: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 14 | 00 | 80 | 00 | 52 | 14 | 00 | 00 | 80 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 72 | 24 | 00 | 80 | 00 | 72 | 34 | 00 | 80 | 00 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 72 | 44 | 00 | 80 | 00 | 72 | 54 | 00 | 80 | 00 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 72 | 64 | 00 | 80 | 00 | 72 | 74 | 00 | 80 | 00 |

For sequence 3.3:

The default E-UTRAN UICC, the default E-UTRAN/NB-SS parameters and the following parameters are used:

EFFPLMN

Logically:	PLMN1:	254 002 (MCC MNC)

PLMN2:	254 003

PLMN3:	254 004

PLMN4:	234 004

PLMN5:	234 005

PLMN6:	234 006

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 | B9 | B10 | B11 | B12 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 24 | 00 | 52 | 34 | 00 | 52 | 44 | 00 | 32 | 44 | 00 |
|  | B13 | B14 | B15 | B16 | B17 | B18 |  |  |  |  |  |  |
|  | 32 | 54 | 00 | 32 | 64 | 00 |  |  |  |  |  |  |

EFOPLMNwACT

Logically:	1st PLMN:	254 001 (MCC MNC)

1st ACT:	E-UTRAN, UTRAN

2nd PLMN:	254 001

2nd ACT:	GSM

3rd PLMN:	274 002

3rd ACT:	E-UTRAN

4th PLMN:	274 003

4th ACT:	E-UTRAN

5th PLMN:	274 004

5th ACT:	E-UTRAN

6th PLMN:	274 005

6th ACT:	E-UTRAN

7th PLMN:	274 006

7th ACT:	E-UTRAN

8th PLMN:	274 007

8th ACT:	UTRAN

| Coding: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 52 | 14 | 00 | C0 | 00 | 52 | 14 | 00 | 00 | 80 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 72 | 24 | 00 | 40 | 00 | 72 | 34 | 00 | 40 | 00 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 72 | 44 | 00 | 40 | 00 | 72 | 54 | 00 | 40 | 00 |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 72 | 64 | 00 | 40 | 00 | 72 | 74 | 00 | 80 | 00 |

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.7.3.4.2	Procedure

Expected Sequence 3.1 (REFRESH, Steering of roaming, UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USS | The first UMTS USS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - LAI (MCC/MNC/LAC): 254/001/0001. - Access control: unrestricted. The second UMTS USS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - LAI (MCC/MNC/LAC): 254/002/0001. - Access control: unrestricted. |  |
| 2 | ME USS | The ME registers to the first USS. |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.1.1 | [Setting up LOCATION STATUS Event] |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1 |  |
| 6a | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1 |  |
| 6b | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.1.2 | This step applies only if A.1/171 |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 3.1.1 | Note: Step 11 can occur at any time during execution of steps 10a to 10d |
| 10a | UICC | Void |  |
| 10b | ME UICC | Update of EF FPLMN | [Deletion of the entries with PLMN 254/003 and PLMN 254/004] |
| 10c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entries with PLMN 254/003 and PLMN 254/004] |
| 10d | ME USS | From steps 9 -13: The ME does not register to another USS than the currently selected and shall not send new LOCATION STATUS event to the UICC. |  |
| 11 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.1 | [normal ending] Note : For a pre-release 11 ME, the UICC simulator does not need to evaluate the response |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 |  | Wait approx. 180 seconds | [The ME does not register to another USS than the currently selected.] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.2 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: REFRESH 3.1.2 | Note: Step 18 can occur at any time during execution of steps 17a to 17c |
| 17a | UICC | Void |  |
| 17b | ME UICC | Update of EF FPLMN | [Deletion of the entry with PLMN 254/002] |
| 17c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entry with PLMN 254/002] |
| 18 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.2 | [normal ending] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | ME USS | The ME registers to the second USS. | Note: The ME might have registered to the second USS also before steps 18/19. |
| 21 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.1.1 | PLMN MCC/MNC: 254/002, Normal service Note: The ME send the Envelope after registration to the second USS, thus might have sent the Envelope also before steps 18/19. |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: REFRESH 3.1.3 | Note: Step 26 can occur at any time during execution of steps 25a to 25c |
| 25a | UICC | Void |  |
| 25b | UICC | EF FPLMN | [PLMN entries 254/003 and PLMN 254/001 not existent in EF FPLMN] |
| 25c | ME | ME's internal memory | [Not explicitly verified: PLMN entries 254/003 and PLMN 254/001 not existent in FPLMN list] |
| 26 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.2 | [normal ending] |
| 27 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 28 | ME USS | The ME registers to the first USS. | Note: The ME might have registered to the first USS also before steps 26/27. |
| 29 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.1.2 | PLMN MCC/MNC: 254/001 Note: The ME send the Envelope after registration to the first USS, thus might have sent the Envelope also before steps 26/27. |
| 30 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.2.1 |  |
| 31 | ME  UICC | FETCH |  |
| 32 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1 | [Event LOCATION STATUS download removed] |
| 33 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1 | The content of the Terminal Response is not part of the evaluation of the test case |
| 34 | USER  ME | SWITCH OFF ME |  |

PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in clause 27.22.7.4.1.4.2.

TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1

Same as TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 in clause 27.22.7.4.1.4.2.

PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.3.2 in clause 27.22.4.16.1.4.2.

TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1

Same as TERMINAL RESPONSE: SET UP EVENT LIST 1.3.2 in clause 27.22.4.16.1.4.2.

PROACTIVE COMMAND: REFRESH 3.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	UTRAN

2ndPLMN:	254/004

2ndACT:	GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | 80 | 00 | 52 | 44 | 00 | 00 | 80 |  |

TERMINAL RESPONSE: REFRESH 3.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 3.1.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/002

1stACT:	UTRAN/GERAN

2ndPLMN:	254/001

2ndACT:	UTRAN/GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 24 | 00 | 80 | 80 | 52 | 14 | 00 | 80 | 80 |  |

TERMINAL RESPONSE: REFRESH 3.1.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 3.1.1

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/002)

LAC	the location Area Code (0001)

Cell ID			Cell Identity Value (0001)

Extended Cell ID		RNC-id value, see also Note 1

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 52 | 24 | 00 | 00 | 01 | 00 | 01 | Note 1 |  |  |

Note 1:	The Extended Cell Identity Value is present. The values of the two bytes shall not be verified.

PROACTIVE COMMAND: REFRESH 3.1.3

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	UTRAN/GERAN

2ndPLMN:	254/001

2ndACT:	UTRAN/GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | 80 | 80 | 52 | 14 | 00 | 80 | 80 |  |

EVENT DOWNLOAD - LOCATION STATUS 3.1.2

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/001)

LAC	the location Area Code (0001)

Cell ID			Cell Identity Value (0001)

Extended Cell ID		RNC-id value, see also Note 1

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 52 | 14 | 00 | 00 | 01 | 00 | 01 | Note 1 |  |  |

Note 1: The Extended Cell Identity Value is present. The values of the two bytes shall not be verified.

Expected Sequence 3.2 (REFRESH, Steering of roaming, InterRAT)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USS | The UMTS USS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - LAI (MCC/MNC/LAC): 254/001/0001. - Access control: unrestricted. The GSM SS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - LAI (MCC/MNC/LAC): 254/002/0001. - Cell ID: 0001 - Access control: unrestricted. |  |
| 2 | ME USS | The ME registers to the UMTS USS and achieves updated idle mode. |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.1.1 | [Setting up LOCATION STATUS Event] |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1 | IF A.1/171 THEN ME sends a ENVELOPE: EVENT DOWNLOAD - Location Status 3.2.2 |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.2.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 3.2.1 | Note: Step 11 can occur at any time during execution of steps 10a to 10c |
| 10a | UICC | Void |  |
| 10b | ME UICC | Update of EF FPLMN | [Deletion of the entry with PLMN 254/002] |
| 10c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entry with PLMN 254/002] |
| 11 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.2 | [normal ending] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | ME USS | The ME registers to the GSM SS and is in updated idle mode. | Note: The ME might have registered to the second USS also before steps 11/12. |
| 14 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.2.1 | PLMN MCC/MNC: 254/002, Normal service Note: The ME send the Envelope after registration to the GSM SS, thus might have sent the Envelope also before steps 11/12. |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.2.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: REFRESH 3.2.2 | Note: Step 19 can occur at any time during execution of steps 18a to 18c |
| 18a | UICC | Void |  |
| 18b | UICC | EF FPLMN | [Entries with PLMN 254/002 and PLMN 254/001 not existent in EF FPLMN] |
| 18c | ME | ME's internal memory | [Not explicitly verified: FPLMN entries with PLMN 254/002 and PLMN 254/001 not existent in FPLMN list] |
| 19 | ME UICC | TERMINAL RESPONSE: REFRESH 3.1.2 | [normal ending] |
| 20 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 21 | ME USS | The ME registers to the UMTS USS and is in updated idle mode. | Note: The ME might have registered to the first USS also before steps 19/20. |
| 22 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.2.2 | PLMN MCC/MNC: 254/001 Note: The ME send the Envelope after registration to the first USS, thus might have sent the Envelope also before steps 19/20. |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.2.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1 | [Event LOCATION STATUS download removed] |
| 26 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1 | The content of the Terminal Response is not part of the evaluation of the test case |
| 27 | USER ME | SWITCH OFF ME |  |

PROACTIVE COMMAND: REFRESH 3.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/002

1stACT:	GERAN

2ndPLMN:	254/001

2ndACT:	UTRAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 24 | 00 | 00 | 80 | 52 | 14 | 00 | 80 | 00 |  |

PROACTIVE COMMAND: REFRESH 3.2.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	GERAN

2ndPLMN:	254/001

2ndACT:	UTRAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | 00 | 80 | 52 | 14 | 00 | 80 | 00 |  |

EVENT DOWNLOAD - LOCATION STATUS 3.2.1

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/002)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D6 | 13 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 52 | 24 | 00 | 00 | 01 | 00 | 01 |  |  |  |

EVENT DOWNLOAD - LOCATION STATUS 3.1.2

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/001)

LAC	the location Area Code (0001)

Cell ID			Cell Identity Value (0001)

Extended Cell ID:	RNC-id value, see also Note 1

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 52 | 14 | 00 | 00 | 01 | 00 | 01 | Note 1 |  |  |

Note 1: The Extended Cell Identity Value is present. The values of the two bytes shall not be verified.

Expected Sequence 3.3 (REFRESH, Steering of roaming, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | E-USS/NB-SS | The first E-USS/NB-SS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - TAI (MCC/MNC/TAC): 254/001/0001. - Access control: unrestricted. The second E-USS/NB-SS transmits on BCCH, with the following network parameters: - Attach/detach: disabled. - TAI (MCC/MNC/TAC): 254/002/0001. - Access control: unrestricted. |  |
| 2 | ME E-USS/NB-SS | The ME registers to the first E-USS/NB-SS. |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.1.1 | [Setting up LOCATION STATUS Event] |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.1.1 | IF A.1/171 THEN ME sends a ENVELOPE: EVENT DOWNLOAD - Location Status 3.3.3 |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.3.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 3.3.1 | Note: Step 11 can occur at any time during execution of steps 10a to 10d |
| 10a | UICC | Void |  |
| 10b | ME UICC | Update of EF FPLMN | [Deletion of the entries with PLMN 254/003 and PLMN 254/004] |
| 10c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entries with PLMN 254/003 and PLMN 254/004] |
| 10d | ME E-USS/NB-SS | From steps 9 -13: The ME does not register to another E-USS/NB-SS than the currently selected and shall not send new LOCATION STATUS event to the UICC. |  |
| 11 | ME UICC | TERMINAL RESPONSE: REFRESH 3.3.1 | [normal ending] Note : For a pre-release 11 ME, the UICC simulator does not need to evaluate the response |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 |  | Wait approx. 180 seconds | [The ME does not register to another E-USS/NB-SS than the currently selected.] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.3.2 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: REFRESH 3.3.2 | Note: Step 18 can occur at any time during execution of steps 17a to 17c |
| 17a | UICC | Void |  |
| 17b | ME UICC | Update of EF FPLMN | [Deletion of the entry with PLMN 254/002] |
| 17c | ME | Update of ME's internal memory | [Not explicitly verified: Deletion of the FPLMN entry with PLMN 254/002] |
| 18 | ME UICC | TERMINAL RESPONSE: REFRESH 3.3.2 | [normal ending] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | ME E-USS/NB-SS | The ME registers to the second E-USS/NB-SS. | Note: The ME might have registered to the second USS also before steps 18/19. |
| 21 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.3.2 | PLMN MCC/MNC: 254/002 Note: The ME send the Envelope after registration to the second E-USS/NB-SS, thus might have sent the Envelope also before steps 18/19. |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 3.1.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: REFRESH 3.3.3 | Note: Step 26 can occur at any time during execution of steps 25a to 25c |
| 25a | UICC | Void |  |
| 25b | UICC | EF FPLMN | [PLMN entries 254/003 and PLMN 254/001 not existent in EF FPLMN] |
| 25c | ME | ME's internal memory | [Not explicitly verified: PLMN entries 254/003 and PLMN 254/001 not existent in FPLMN list] |
| 26 | ME UICC | TERMINAL RESPONSE: REFRESH 3.3.2 | [normal ending] |
| 27 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 28 | ME E-USS/NB-SS | The ME registers to the first E-USS/NB-SS. | Note: The ME might have registered to the first USS also before steps 26/27. |
| 29 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 3.3.3 | PLMN MCC/MNC: 254/001 Note: The ME send the Envelope after registration to the second USS, thus might have sent the Envelope also before steps 26/27. |
| 30 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 3.2.1 |  |
| 31 | ME UICC | FETCH |  |
| 32 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 3.2.1 | [Event LOCATION STATUS download removed] |
| 33 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 3.2.1 | The content of the Terminal Response is not part of the evaluation of the test case |
| 34 | USER ME | SWITCH OFF ME |  |

PROACTIVE COMMAND: REFRESH 3.3.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	E-UTRAN, UTRAN

2ndPLMN:	254/004

2ndACT:	GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | C0 | 00 | 52 | 44 | 00 | 00 | 80 |  |

TERMINAL RESPONSE: REFRESH 3.3.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 3.3.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/002

1stACT:	E-UTRAN/UTRAN/GERAN

2ndPLMN:	254/001

2ndACT:	E-UTRAN/UTRAN/GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 24 | 00 | C0 | 80 | 52 | 14 | 00 | C0 | 80 |  |

TERMINAL RESPONSE: REFRESH 3.3.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 3.3.2

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/002)

TAC	0001

E-UTRAN cell id:	0001 (28bits)

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 52 | 24 | 00 | 00 | 01 | 00 | 00 | 00 | 1F |  |

PROACTIVE COMMAND: REFRESH 3.3.3

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	Steering of roaming

Device identities

Source device:	UICC

Destination device:	ME

PLMNwACT List

1stPLMN:	254/003

1stACT:	E-UTRAN/UTRAN/GERAN

2ndPLMN:	254/001

2ndACT:	E-UTRAN/UTRAN/GERAN

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 01 | 07 | 82 | 02 | 81 | 82 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 52 | 34 | 00 | C0 | 80 | 52 | 14 | 00 | C0 | 80 |  |

EVENT DOWNLOAD - LOCATION STATUS 3.3.3

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (254/001)

TAC	0001

E-UTRAN cell id:	0001 (28bits)

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 52 | 14 | 00 | 00 | 01 | 00 | 00 | 00 | 1F |  |

27.22.4.7.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.3.

##### 27.22.4.7.4	REFRESH (AID)

27.22.4.7.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.4.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.7, clause 6, clause 6.6.13, clause 5.2, clause 8.6, clause 8.7, clause 8.18 and clause 8.60.

The ME shall support the IMS related requirements as defined and tested in:

-	TS 24.229 [38] clause 5.1.1.7 and Annex C.4

-	TS 34.229-1 [36] clause 8.15, Annex C.2, C.17 and C.18

The ME shall support the USIM Initialization procedure as defined in:

-	TS 31.102 [14] clause 5.1.2 and Annex I.

27.22.4.7.4.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier and additionally correctly takes into account the Application Identifier if present in the Refresh command.

- Verification of correct Refresh command execution within the application executed on a any logical channel if the corresponding AID is present in the Refresh command

This may require the ME to perform:

-	a USIM or ISIM initialization

-	a re-read of the contents and structure of the ISIM on the USIM

-	a successful return of the result of the execution of the command in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.7.4.4	Method of test

27.22.4.7.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as defined for the E-UTRAN/EPC ISIM-UICC in clause 27.22.2C.

For sequence 4.1 the ME is connected to the E-USS or the USS.

27.22.4.7.4.4.2	Procedure

Expected Sequence 4.1 (REFRESH with AID)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download, USIM and ISIM initialisation |
| 2 | ME NWS | ME activates the required bearer, discoveres P-CSCF and registers with the values from the ISIM to IMS services | For E-UTRAN: The EPS bearer context activation according to the procedures defined in TS 34.229-1 [36], Annex C.2 and C.18 is performed For UTRAN: A PDP context activation according to the procedures defined in TS 34.229-1 [36], Annex C.2 and C.17 is performed. |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 4.1.1 | To inform the ME that EF_FPLMN shall be re-read. |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: REFRESH 4.1.1 | EF_FPLMN shall be read by the UE, but this might occur even after the Terminal Response. An update of EF_FPLMN by the UICC is not required in this test. |
| 6 | ME UICC | TERMINAL RESPONSE: REFRESH 4.1.1A Or TERMINAL RESPONSE: REFRESH 4.1.1B | [normal ending] [additional EFs read] |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 8 |  | Continue with steps 1 – 4 of the "Expected Sequence" of test 8.15 of TS 34.229-1 with the following parameters: - REFRESH command: PROACTIVE COMMAND: Refresh 4.2.1 - Initial Home Domain name = Updated Home Domain name - New IMPI in EF_IMPI= 00101555666@test.3gpp.com - New IMPU in record 1 of EF_IMPU= 00101555666@ims.mnc246.mcc081.3gppnetwork.org | The following requirements shall be verified: - After step 1 and before step 4 of the "Expected Sequence" of test 8.15 of TS 34.229-1the ME shall have sent TERMINAL RESPONSE: REFRESH 4.2.1A or TERMINAL RESPONSE: REFRESH 4.2.1B - The ME shall have fulfilled the test requieremnts defined in TS 34.229, clause 8.15.5 |

PROACTIVE COMMAND: REFRESH 4.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

File List

File 1:	EF FPLMN

Application Identifier

Content:	The 3GPP USIM AID used in the test system configuration

Coding:

| BER-TLV: | D0 | 24 | 81 | 03 | 01 | 01 | 01 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 92 | 07 | 01 | 3F | 00 | 7F | FF | 6F | 7B | 2F | 10 |
|  | A0 | 00 | 00 | 00 | 87 | 10 | 02 | xx | xx | xx | xx |
|  | xx | xx | xx | xx | xx |  |  |  |  |  |  |

PROACTIVE COMMAND: REFRESH 4.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	ISIM Initialization

Device identities

Source device:	UICC

Destination device:	ME

Application Identifier

Content:	The 3GPP ISIM AID used in the test system configuration

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2F | 10 | A0 | 00 | 00 | 00 | 87 | 10 | 04 | xx | xx |
|  | xx | xx | xx | xx | xx | xx | xx |  |  |  |  |

TERMINAL RESPONSE: REFRESH 4.1.1A/4.2.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM/ISIM Initialization

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 4.1.1B/ 4.2.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM/ISIM Initialization

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.7.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

##### 27.22.4.7.5	REFRESH (IMSI changing procedure, E-UTRAN)

27.22.4.7.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.7.5.2	Conformance requirement

The ME shall support the REFRESH command as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.7, clause 6.4.7.1, clause 6, clause 6.6.13, clause 5.2, clause 8.6, clause 8.7 and clause 8.18.

Additionally the ME shall support the USIM Initialization and USIM application closure procedure as defined in:

-	TS 31.102 [14] clause 5.1.2 and Annex I.

27.22.4.7.5.3	Test purpose

To verify that the ME performs the Proactive Command – REFRESH in accordance with the Command Qualifier and the IMSI changing procedure. This may require the ME to perform:

-	the USIM initialization

-	a re-read of the contents and structure of the IMSI on the USIM

-	a restart of the card session

-	a successful return of the result of the execution of the command in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.7.5.4	Method of test

27.22.4.7.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the E-USS/NB-SS, registered and has the default PDN connection established.

The E-UTRAN/NB-IoT parameters of the E-USS/NB-SS are:

-	Mobile Country Code (MCC) = 246;

-	Mobile Network Code (MNC) = 81;

-	Tracking Area Code (TAC) = 0001;

The elementary files are coded as the default E-UTRAN/EPC UICC,

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.7.5.4.2	Procedure

Expected Sequence 5.1 (REFRESH, UICC Reset for IMSI Changing procedure, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 5.1.1 | [To inform the ME that IMSI has changed] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 5.1.1 or 5.1.2 | IF terminal supports PD_ Refresh_Enforcement_Policy use PROACTIVE COMMAND: REFRESH 5.1.2, ELSE 5.1.1. |
| 4 | MEE-USS/NB-SS | Deactivate PDN Connection | ME will deactivate the PDN Connection Note: this step is performed locally and may not reflect on the interface to the E-USS/NB-SS Note: if the ME supports pc_NB this step is performed only in case pc_AttachWithPDN is supported by the ME. . |
| 5 | MEUSS | DETACH REQUEST | Indicates GPRS detach, Note: this step can happen after step 6 |
| 6 | ME UICC | STATUS[P1='02'] | ME indicates to USIM that the termination procedure is starting |
| 7 | ME UICC | ME performs UICC reset | Both cold and warm resets are allowed |
| 8 | UICC | Update EF IMSI and EF EPSLOCI | The content of EF IMSI has been changed to "246813579" and the GUTI in EF EPSLOCI is updated to 'FF FF FF FF FF FF FF FF FF FF FF FF' |
| 9 | ME UICC | ME performs USIM Initialization, including send STATUS[P1='01'] and no TERMINAL RESPONSE shall be sent | [ME resets and performs USIM initialization] |
| 10 | ME E-USS/NB-SS | ATTACH REQUEST | The ME will register using IMSI "246813579" in PS. |
| 11 | USS ME | ATTACH ACCEPT |  |
| 12 | ME E-USS/NB-SS | ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 5.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: REFRESH 5.1.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	UICC RESET

Device identities

Source device:	UICC

Destination device:	ME

Refresh enforcement policy : Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 01 | 04 | 82 | 02 | 81 | 82 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 |

Expected Sequence 5.2 (REFRESH, 3G Session Reset for IMSI Changing procedure, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 5.2.1 | [To inform the ME that IMSI has changed] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: REFRESH 5.2.1 or 5.2.2 | IF terminal supports PD_ Refresh_Enforcement_Policy use PROACTIVE COMMAND: REFRESH 5.2.2, ELSE 5.2.1. |
| 4 | MEE-USS/NB-SS | Deactivate PDN Connection | ME will deactivate the PDN Connection Note 1: this step is performed locally and may not reflect on the interface to the E-USS/NB-SS Note: if the ME supports pc_NB this step is performed only in case pc_AttachWithPDN is supported by the ME.. Note: this step can be performed in parallel or after step 6. |
| 5 | MEE-USS/NB-SS | DETACH REQUEST | Note: this step can be performed in parallel or after step 6. |
| 6 | ME UICC | STATUS[P1='02'] | If A.1/172 is supported, then the ME indicates to USIM that the termination procedure is starting, completes the 3G session termination procedure and resets the application via SELECT by DF name command with the AID. The ME performs the USIM initialization. |
| 7 | UICC | Update EF IMSI and EF EPSLOCI | The content of EF IMSI has been updated to "246813579" and GUTI in EF EPSLOCI is updated to 'FF FF FF FF FF FF FF FF FF FF FF FF' |
| 8 | ME UICC | TERMINAL RESPONSE: REFRESH 5.2.1A Or TERMINAL RESPONSE: REFRESH 5.2.1B | [normal ending] |
| 9 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 10 | ME E-USS/NB-SS | ATTACH REQUEST | The ME will register using IMSI "246813579" in PS. |
| 11 | USS ME | ATTACH ACCEPT |  |
| 12 | ME E-USS/NB-SS | ATTACH COMPLETE |  |

PROACTIVE COMMAND: REFRESH 5.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	2

File:	EF IMSI

File:	EF EPSLOCI

Coding:

| BER-TLV: | D0 | 18 | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0D | 02 | 3F | 00 | 7F | FF | 6F | 07 | 3F | 00 | 7F | FF |
|  | 6F | E3 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: REFRESH 5.2.2

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	UICC

Destination device:	ME

File list

Number of files:	2

File:	EF IMSI

File:	EF EPSLOCI

Refresh enforcement policy : Force immediate REFRESH even if the terminal is busy on data call

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0D | 02 | 3F | 00 | 7F | FF | 6F | 07 | 3F | 00 | 7F | FF |
|  | 6F | E3 | 3A | 01 | 02 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: REFRESH 5.2.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 5.2.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	3G Session Reset

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.7.5.5	Test requirement

The ME shall operate in the manner defined in expected sequences  5.1 to 5.2.

#### 27.22.4.8	SET UP MENU and ENVELOPE MENU SELECTION

##### 27.22.4.8.1	SET UP MENU (normal) and ENVELOPE MENU SELECTION

27.22.4.8.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.1.2	Conformance requirement

The ME shall support the SET UP MENU command as defined in:

-	TS 31.111 [15] clause 5, clause 6.4.8, clause 6.6.7, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.9 and clause 9.4.

The ME shall support MENU SELECTION as defined in:

-	TS 31.111 [15] clause 4.4, clause 5.2, clause 6.4.8, clause 6.9, clause 7.2, clause 8.7 and clause 8.10.

27.22.4.8.1.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME replaces the current list of menu items with the list of menu items contained in the SET UP MENU command.

To verify that the ME removes the current list of menu items following receipt of a SET UP MENU command with no items.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

To verify that when the help is available for the command and the user gas indicated the need to get help information on one of the items, the ME informs properly the UICC about an HELP REQUEST, using the MENU SELECTION mechanism.

27.22.4.8.1.4	Method of test

27.22.4.8.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.1.4.2	Procedure

Expected Sequence 1.1 (SET UP MENU and MENU SELECTION, without Help Request, Replace and Remove a Toolkit Menu)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (SET UP MENU, Large Menu with many items or with large items or with Large Alpha Identifier)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.1.4.2, Expected Sequence 1.2.

The following table details the test requirements with relation to the tested features:

|  | Proactive UICC Command Facilities |  |  |
| --- | --- | --- | --- |
| Proactive UICC Command Number | Alpha Identifier Length | Number of items | Maximum length of item |
| 1.1.1 | 12 | 4 | 6 |
| 1.1.2 | 12 | 2 | 3 |
| 1.1.3 | 10 | 0 | - |
| 1.2.1 | 10 | 30 | 8 |
| 1.2.2 | 10 | 7 | 37 |
| 1.2.3 | 235 | 1 | 1 |

27.22.4.8.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1 and in expected sequence 1.2.

##### 27.22.4.8.2	SET UP MENU (help request support) and ENVELOPE MENU SELECTION

27.22.4.8.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.2.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	TS 31.111 [15] clause 8.21.

27.22.4.8.2.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that when the help is available for the command and the user has indicated the need to get help information on one of the items, the ME informs properly the UICC about an HELP REQUEST, using the MENU SELECTION mechanism.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.2.4	Method of test

27.22.4.8.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.2.4.2	Procedure

Expected Sequence 2.1 (SET UP MENU and MENU SELECTION, with Help Request, Replace and Remove a Toolkit Menu)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.2.4.2, Expected Sequence 2.1.

27.22.4.8.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.8.3	SET UP MENU (next action support) and ENVELOPE MENU SELECTION

27.22.4.8.3.1	Definition and applicability

See clause 3.2.2.

If the UICC provides an Items Next Action Indicator data object, the comprehension required flag shall be set to '0'.

27.22.4.8.3.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	TS 31.111 [15] clause 8.24.

27.22.4.8.3.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the next action indicator is supported.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.3.4	Method of test

27.22.4.8.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.3.4.2	Procedure

Expected Sequence 3.1 (SET UP MENU, next action indicator "Send SM", "Set Up Call", "Launch Browser", "Provide Local Information", successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.3.4.2, Expected Sequence 3.1.

27.22.4.8.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.8.4	SET UP MENU (display of icons) and ENVELOPE MENU SELECTION

27.22.4.8.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.4.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clause 6.5.4, 8.31 and 8.32.

27.22.4.8.4.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that icons are displayed with the command Set Up Menu in the Alpha Identifier and Items Data Objects. To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.4.4	Method of test

27.22.4.8.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.4.4.2	Procedure

Expected Sequence 4.1A (SET UP MENU, BASIC ICON NOT SELF EXPLANATORY in ALPHA ID and ITEMS DATA OBJECTS, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.4.4.2, Expected Sequence 4.1A.

Expected Sequence 4.1B (SET UP MENU, BASIC ICON NOT SELF EXPLANATORY in ALPHA ID and ITEMS DATA OBJECTS, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.4.4.2, Expected Sequence 4.1B.

Expected Sequence 4.2A (SET UP MENU, BASIC ICON SELF EXPLANATORY in ALPHA ID and ITEMS DATA OBJECTS, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.4.4.2, Expected Sequence 4.2A.

Expected Sequence 4.2B (SET UP MENU, BASIC ICON SELF EXPLANATORY in ALPHA ID and ITEMS DATA OBJECTS, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.4.4.2, Expected Sequence 4.2B.

27.22.4.8.4.5	Test requirement

The ME shall operate in the manner defined in expected sequences 4.1A to 4.2B.

##### 27.22.4.8.5	SET UP MENU (soft keys support) and ENVELOPE MENU SELECTION

27.22.4.8.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.5.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1.

27.22.4.8.5.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that if soft key preferred is indicated in the command details and soft key for SET UP MENU is supported by the ME and the number of icon items does not exceed the number of soft keys available, then the ME displays those icons as soft key.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.5.4	Method of test

27.22.4.8.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.5.4.2	Procedure

Expected Sequence 5.1 (SET UP MENU, SOFT KEY PREFERRED, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.5.4.2, Expected Sequence 5.1.

27.22.4.8.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.8.6	SET UP MENU (support of Text Attribute) and ENVELOPE MENU SELECTION

27.22.4.8.6.1	SET UP MENU (support of Text Attribute – Left Alignment) and ENVELOPE MENU SELECTION

27.22.4.8.6.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.1.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.1.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the left alignment text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.1.4	Method of test

27.22.4.8.6.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.1.4.2	Procedure

Expected Sequence 6.1 (SET UP MENU, Text Attribute – Left Alignment, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.1.4.2, Expected Sequence 6.1.

27.22.4.8.6.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

27.22.4.8.6.2	SET UP MENU (support of Text Attribute – Center Alignment) and ENVELOPE MENU SELECTION

27.22.4.8.6.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.2.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.2.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the center alignment text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.2.4	Method of test

27.22.4.8.6.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.2.4.2	Procedure

Expected Sequence 6.2 (SET UP MENU, Text Attribute – Center Alignment, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.2.4.2, Expected Sequence 6.2.

27.22.4.8.6.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.2.

27.22.4.8.6.3	SET UP MENU (support of Text Attribute – Right Alignment) and ENVELOPE MENU SELECTION

27.22.4.8.6.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.3.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.3.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the right alignment text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.3.4	Method of test

27.22.4.8.6.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.3.4.2	Procedure

Expected Sequence 6.3 (SET UP MENU, Text Attribute – Right Alignment, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.3.4.2, Expected Sequence 6.3.

27.22.4.8.6.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.3.

27.22.4.8.6.4	SET UP MENU (support of Text Attribute – Large Font Size) and ENVELOPE MENU SELECTION

27.22.4.8.6.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.4.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.4.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the large font size text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.4.4	Method of test

27.22.4.8.6.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.4.4.2	Procedure

Expected Sequence 6.4 (SET UP MENU, Text Attribute – Large Font Size, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.4.4.2, Expected Sequence 6.4.

27.22.4.8.6.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.4.

27.22.4.8.6.5	SET UP MENU (support of Text Attribute – Small Font Size) and ENVELOPE MENU SELECTION

27.22.4.8.6.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.5.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.5.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the with small font size text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.5.4	Method of test

27.22.4.8.6.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.5.4.2	Procedure

Expected Sequence 6.5 (SET UP MENU, Text Attribute – Small Font Size, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.5.4.2, Expected Sequence 6.5.

27.22.4.8.6.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.5.

27.22.4.8.6.6	SET UP MENU (support of Text Attribute – Bold On) and ENVELOPE MENU SELECTION

27.22.4.8.6.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.6.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.6.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.6.4	Method of test

27.22.4.8.6.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.6.4.2	Procedure

Expected Sequence 6.6 (SET UP MENU, Text Attribute – Bold On, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.6.4.2, Expected Sequence 6.6.

27.22.4.8.6.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.6.

27.22.4.8.6.7	SET UP MENU (support of Text Attribute – Italic On) and ENVELOPE MENU SELECTION

27.22.4.8.6.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.7.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.7.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.7.4	Method of test

27.22.4.8.6.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.7.4.2	Procedure

Expected Sequence 6.7 (SET UP MENU, Text Attribute – Italic On, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.7.4.2, Expected Sequence 6.7.

27.22.4.8.6.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.7.

27.22.4.8.6.8	SET UP MENU (support of Text Attribute – Underline On) and ENVELOPE MENU SELECTION

27.22.4.8.6.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.8.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.8.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.8.4	Method of test

27.22.4.8.6.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.8.4.2	Procedure

Expected Sequence 6.8 (SET UP MENU, Text Attribute – Underline On, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.8.4.2, Expected Sequence 6.8.

27.22.4.8.6.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.8.

27.22.4.8.6.9	SET UP MENU (support of Text Attribute – Strikethrough On) and ENVELOPE MENU SELECTION

27.22.4.8.6.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.9.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.9.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.9.4	Method of test

27.22.4.8.6.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.9.4.2	Procedure

Expected Sequence 6.9 (SET UP MENU, Text Attribute – Strikethrough On, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.9.4.2, Expected Sequence 6.9.

27.22.4.8.6.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.9.

27.22.4.8.6.10	SET UP MENU (support of Text Attribute – Foreground and Background Colour) and ENVELOPE MENU SELECTION

27.22.4.8.6.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.6.10.2	Conformance requirement

Requirements are the same as in clause 27.22.4.8.1.1, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.8.6.10.3	Test purpose

To verify that the ME correctly integrates the menu items contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that text is displayed according to the text attribute configuration within the command Set Up Menu and the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

27.22.4.8.6.10.4	Method of test

27.22.4.8.6.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.6.10.4.2	Procedure

Expected Sequence 6.10 (SET UP MENU, Text Attribute – Foreground and Background Colour, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.6.10.4.2, Expected Sequence 6.10.

27.22.4.8.6.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.10.

##### 27.22.4.8.7	SET UP MENU (UCS2 display in Cyrillic) and ENVELOPE MENU SELECTION

27.22.4.8.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.7.2	Conformance requirement

The ME shall support the SET UP MENU command as defined in:

-	TS 31.111 [15] clause 5, clause 6.4.8, clause 6.6.7, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.9 and clause 9.4.

The ME shall support MENU SELECTION as defined in:

-	TS 31.111 [15] clause 4.4, clause 5.2, clause 6.4.8, clause 6.9, clause 7.2, clause 8.7 and clause 8.10.

-	Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in ISO/IEC 10646 [17].

27.22.4.8.7.3	Test purpose

To verify that the ME correctly integrates the menu items in UCS2 coding contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME replaces the current list of menu items with the list of menu items contained in the SET UP MENU command.

To verify that the ME removes the current list of menu items following receipt of a SET UP MENU command with no items.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

To verify that when the help is available for the command and the user gas indicated the need to get help information on one of the items,  the ME informs properly the UICC about an HELP REQUEST, using the MENU SELECTION mechanism.

27.22.4.8.7.4	Method of test

27.22.4.8.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.7.4.2	Procedure

Expected Sequence 7.1 (SET UP MENU and MENU SELECTION, without Help Request, Replace and Remove a Toolkit Menu, with UCS2 in Cyrillic Characters)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.7.4.2, Expected Sequence 7.1.

27.22.4.8.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.8.8	SET UP MENU (UCS2 display in Chinese) and ENVELOPE MENU SELECTION

27.22.4.8.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.8.2	Conformance requirement

The ME shall support the SET UP MENU command as defined in:

-	TS 31.111 [15] clause 5, clause 6.4.8, clause 6.6.7, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.9 and clause 9.4

The ME shall support MENU SELECTION as defined in:

-	TS 31.111 [15] clause 4.4, clause 5.2, clause 6.4.8, clause 6.9, clause 7.2, clause 8.7 and clause 8.10.

-	Additionally the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in ISO/IEC 10646 [17].

27.22.4.8.8.3	Test purpose

To verify that the ME correctly integrates the menu items in UCS2 coding contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME replaces the current list of menu items with the list of menu items contained in the SET UP MENU command.

To verify that the ME removes the current list of menu items following receipt of a SET UP MENU command with no items.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

To verify that when the help is available for the command and the user gas indicated the need to get help information on one of the items,  the ME informs properly the UICC about an HELP REQUEST, using the MENU SELECTION mechanism.

27.22.4.8.8.4	Method of test

27.22.4.8.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.8.4.2	Procedure

Expected Sequence 8.1 (SET UP MENU and MENU SELECTION, without Help Request, Replace and Remove a Toolkit Menu, with UCS2 – Chinese characters)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.8.4.2, Expected Sequence 8.1.

27.22.4.8.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1.

##### 27.22.4.8.9	SET UP MENU (UCS2 display in Katakana) and ENVELOPE MENU SELECTION

27.22.4.8.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.8.9.2	Conformance requirement

The ME shall support the SET UP MENU command as defined in:

-	TS 31.111 [15] clause 5, clause 6.4.8, clause 6.6.7, clause 6.8, clause 6.11, clause 8.6, clause 8.7, clause 8.2, clause 8.9 and clause 9.4.

The ME shall support MENU SELECTION as defined in:

-	TS 31.111 [15] clause 4.4, clause 5.2, clause 6.4.8, clause 6.9, clause 7.2, clause 8.7 and clause 8.10.

-	Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in ISO/IEC 10646 [17].

27.22.4.8.9.3	Test purpose

To verify that the ME correctly integrates the menu items in UCS2 coding contained in the SET UP MENU proactive UICC command, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME replaces the current list of menu items with the list of menu items contained in the SET UP MENU command.

To verify that the ME removes the current list of menu items following receipt of a SET UP MENU command with no items.

To verify that the ME correctly passes the identifier of the selected menu item to the UICC using the ENVELOPE (MENU SELECTION) command.

To verify that when the help is available for the command and the user gas indicated the need to get help information on one of the items,  the ME informs properly the UICC about an HELP REQUEST, using the MENU SELECTION mechanism.

27.22.4.8.9.4	Method of test

27.22.4.8.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

The ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.8.9.4.2	Procedure

Expected Sequence 9.1 (SET UP MENU and MENU SELECTION, without Help Request, Replace and Remove a Toolkit Menu, with UCS2 in Katakana Characters)

See ETSI TS 102 384 [26] in subclause 27.22.4.8.9.4.2, Expected Sequence 9.1.

27.22.4.8.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.1.

#### 27.22.4.9	SELECT ITEM

##### 27.22.4.9.1	SELECT ITEM (mandatory features for ME supporting SELECT ITEM)

27.22.4.9.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.1.2	Conformance requirement

The ME shall support the Proactive UICC: Select Item facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 6.4.9, clause 6.6.8, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.9, clause 9.4 and clause 10.

27.22.4.9.1.3	Test purpose

To verify that the ME correctly presents the set of items contained in the SELECT ITEM proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC with the identifier of the item chosen.

To verify that the ME allows a SELECT ITEM proactive UICC command within the maximum 255 byte BER-TLV boundary.

To verify that the ME returns a TERMINAL RESPONSE with "Proactive UICC application session terminated by the user", if the user has indicated the need to end the proactive UICC session.

To verify that the ME returns a TERMINAL RESPONSE with "Backwards move in the proactive UICC application session requested by the user", if the user has indicated the need to go backwards in the proactive UICC application session.

27.22.4.9.1.4	Method of test

27.22.4.9.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.1.4.2	Procedure

Expected Sequence 1.1 (SELECT ITEM, mandatory features, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (SELECT ITEM, large menu, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (SELECT ITEM, call options, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.1.4.2, Expected Sequence 1.3.

Expected Sequence 1.4 (SELECT ITEM, backward move by user, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.1.4.2, Expected Sequence 1.4.

Expected Sequence 1.5 (SELECT ITEM, "Y", successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.1.4.2, Expected Sequence 1.5.

Expected Sequence 1.6 (SELECT ITEM, Large menu, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.1.4.2, Expected Sequence 1.6.

The following table details the test commands with relation to the tested features:

|  | Proactive UICC Command Facilities |  |  |
| --- | --- | --- | --- |
| Proactive UICC Command SELECT ITEM Number | Alpha Identifier Length | Number of items | Maximum length of item |
| 1.1 | 14 | 4 | 6 |
| 1.2 | 10 | 30 | 8 |
| 1.3 | 10 | 7 | 43 |
| 1.4 | 11 | 2 | 3 |
| 1.5 | 236 | 1 | 1 |
| 1.6 | 10 | 7 | 37 |

27.22.4.9.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1, 1.2, 1.3, 1.4, 1.5 and 1.6 (SELECT ITEM, mandatory features).

##### 27.22.4.9.2	SELECT ITEM (next action support)

27.22.4.9.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.2.2	Conformance Requirement

Same as clause 27.22.4.9.1.2.

27.22.4.9.2.3	Test purpose

To verify that the mobile supports next action indicator mode.

27.22.4.9.2.4	Method of test

27.22.4.9.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.2.4.2	Procedure

Expected Sequence 2.1 (SELECT ITEM, next action indicator, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.2.4.2, Expected Sequence 2.1.

27.22.4.9.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1

##### 27.22.4.9.3	SELECT ITEM (default item support)

27.22.4.9.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.3.2	Conformance requirement

Same as clause 27.22.4.9.1.2.

27.22.4.9.3.3	Test purpose

To verify that the mobile supports "default item" mode.

27.22.4.9.3.4	Method of test

27.22.4.9.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.3.4.2	Procedure

Expected Sequence 3.1 (SELECT ITEM, default item, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.3.4.2, Expected Sequence 3.1.

27.22.4.9.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1

##### 27.22.4.9.4	SELECT ITEM (help request support)

27.22.4.9.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.4.2	Conformance requirement

Same as clause 27.22.4.9.1.2.

27.22.4.9.4.3	Test purpose

To verify that the mobile supports "help request" for the command Select Item.

27.22.4.9.4.4	Method of test

27.22.4.9.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.4.4.2	Procedure

Expected Sequence 4.1 (SELECT ITEM, help request, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.4.4.2, Expected Sequence 4.1.

27.22.4.9.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1

##### 27.22.4.9.5	SELECT ITEM (icons support)

27.22.4.9.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.5.2	Conformance requirement

Same as clause 27.22.4.9.1.2 and TS 31.111 [15] clause 8.31 and clause 8.32.

27.22.4.9.5.3	Test purpose

To verify that the mobile displays icons with the command Select Item.

27.22.4.9.5.4	Method of test

27.22.4.9.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.5.4.2	Procedure

Expected Sequence 5.1A (SELECT ITEM, BASIC ICON NOT SELF EXPLANATORY, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.5.4.2, Expected Sequence 5.1A.

Expected Sequence 5.1B (SELECT ITEM, BASIC ICON NOT SELF EXPLANATORY, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.5.4.2, Expected Sequence 5.1B.

Expected Sequence 5.2A (SELECT ITEM, BASIC ICON SELF EXPLANATORY, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.5.4.2, Expected Sequence 5.2A.

Expected Sequence 5.2B (SELECT ITEM, BASIC ICON SELF EXPLANATORY, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.5.4.2, Expected Sequence 5.2B.

27.22.4.9.5.5	Test requirement

The ME shall operate in the manner defined in expected sequences 5.1A to 5.2B.

##### 27.22.4.9.6	SELECT ITEM (presentation style)

27.22.4.9.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.6.2	Conformance requirement

Same as clause 27.22.4.9.1.2.

27.22.4.9.6.3	Test purpose

To verify that the mobile supports the "presentation style" with the command Select Item.

27.22.4.9.6.4	Method of test

27.22.4.9.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.6.4.2	Procedure

Expected Sequence 6.1 (SELECT ITEM, PRESENTATION AS A CHOICE OF NAVIGATION OPTIONS, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.6.4.2, Expected Sequence 6.1.

Expected Sequence 6.2 (SELECT ITEM, PRESENTATION AS A CHOICE OF DATA VALUES, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.6.4.2, Expected Sequence 6.2.

27.22.4.9.6.5	Test requirement

The ME shall operate in the manner defined in expected sequences 6.1 and 6.2.

##### 27.22.4.9.7	SELECT ITEM (soft keys support)

27.22.4.9.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.7.2	Conformance requirement

Same as clause 27.22.4.9.1.2.

27.22.4.9.7.3	Test purpose

To verify that the mobile supports the "soft keys" with the command Select Item.

27.22.4.9.7.4	Method of test

27.22.4.9.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.7.4.2	Procedure

Expected Sequence 7.1 (SELECT ITEM, SELECTING USING SOFT KEYS PREFERRED, successful, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.7.4.2, Expected Sequence 7.1.

27.22.4.9.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.9.8	SELECT ITEM (Support of "No response from user")

27.22.4.9.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.8.2	Conformance requirement

Same as clause 27.22.4.9.1.2.

27.22.4.9.8.3	Test purpose

To verify that after a period of user inactivity the ME returns a "No response from user" result value in the TERMINAL RESPONSE command sent to the UICC.

27.22.4.9.8.4	Method of test

27.22.4.9.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME Manufacturer shall have defined the "no response from user" period of time as declared in table A.2/4.

The USIM Simulator shall be set to that period of time.

27.22.4.9.8.4.2	Procedure

Expected Sequence 8.1 (SELECT ITEM, no response from user)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.8.4.2, Expected Sequence 8.1.

27.22.4.9.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1.

##### 27.22.4.9.9	SELECT ITEM (Support of Text Attribute)

27.22.4.9.9.1	SELECT ITEM (Support of Text Attribute – Left Alignment)

27.22.4.9.9.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.1.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.1.3	Test purpose

To verify that the ME displays text formatted according to the left alignment text attribute configuration within the command Select Item.

27.22.4.9.9.1.4	Method of test

27.22.4.9.9.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.1.4.2	Procedure

Expected Sequence 9.1 (SELECT ITEM, Text Attribute – Left Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.1.4.2, Expected Sequence 9.1.

27.22.4.9.9.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.1.

27.22.4.9.9.2	SELECT ITEM (Support of Text Attribute – Center Alignment)

27.22.4.9.9.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.2.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.2.3	Test purpose

To verify that the ME displays text formatted according to the center alignment text attribute configuration within the command Select Item.

27.22.4.9.9.2.4	Method of test

27.22.4.9.9.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.2.4.2	Procedure

Expected Sequence 9.2 (SELECT ITEM, Text Attribute – Center Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.2.4.2, Expected Sequence 9.2.

27.22.4.9.9.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.2.

27.22.4.9.9.3	SELECT ITEM (Support of Text Attribute – Right Alignment)

27.22.4.9.9.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.3.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.3.3	Test purpose

To verify that the ME displays text formatted according to the right alignment text attribute configuration within the command Select Item.

27.22.4.9.9.3.4	Method of test

27.22.4.9.9.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.3.4.2	Procedure

Expected Sequence 9.3 (SELECT ITEM, Text Attribute – Right Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.3.4.2, Expected Sequence 9.3.

27.22.4.9.9.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.3.

27.22.4.9.9.4	SELECT ITEM (Support of Text Attribute – Large Font Size)

27.22.4.9.9.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.4.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.4.3	Test purpose

To verify that the ME displays text formatted according to the large font size text attribute configuration within the command Select Item.

27.22.4.9.9.4.4	Method of test

27.22.4.9.9.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.4.4.2	Procedure

Expected Sequence 9.4 (SELECT ITEM, Text Attribute – Large Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.4.4.2, Expected Sequence 9.4.

27.22.4.9.9.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.4.

27.22.4.9.9.5	SELECT ITEM (Support of Text Attribute – Small Font Size)

27.22.4.9.9.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.5.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.5.3	Test purpose

To verify that the ME displays text formatted according to the small font size text attribute configuration within the command Select Item.

27.22.4.9.9.5.4	Method of test

27.22.4.9.9.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.5.4.2	Procedure

Expected Sequence 9.5 (SELECT ITEM, Text Attribute – Small Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.5.4.2, Expected Sequence 9.5.

27.22.4.9.9.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.5.

27.22.4.9.9.6	SELECT ITEM (Support of Text Attribute – Bold On)

27.22.4.9.9.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.6.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.6.3	Test purpose

To verify that the ME displays text formatted according to the bold text attribute configuration within the command Select Item.

27.22.4.9.9.6.4	Method of test

27.22.4.9.9.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.6.4.2	Procedure

Expected Sequence 9.6 (SELECT ITEM, Text Attribute – Bold On)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.6.4.2, Expected Sequence 9.6.

27.22.4.9.9.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.6.

27.22.4.9.9.7	SELECT ITEM (Support of Text Attribute – Italic On)

27.22.4.9.9.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.7.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.7.3	Test purpose

To verify that the ME displays text formatted according to the italic text attribute configuration within the command Select Item.

27.22.4.9.9.7.4	Method of test

27.22.4.9.9.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.7.4.2	Procedure

Expected Sequence 9.7 (SELECT ITEM, Text Attribute – Italic On)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.7.4.2, Expected Sequence 9.7.

27.22.4.9.9.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.7.

27.22.4.9.9.8	SELECT ITEM (Support of Text Attribute – Underline On)

27.22.4.9.9.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.8.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.8.3	Test purpose

To verify that the ME displays text formatted according to the underline text attribute configuration within the command Select Item.

27.22.4.9.9.8.4	Method of test

27.22.4.9.9.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.8.4.2	Procedure

Expected Sequence 9.8 (SELECT ITEM, Text Attribute – Underline On)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.8.4.2, Expected Sequence 9.8.

27.22.4.9.9.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.8.

27.22.4.9.9.9	SELECT ITEM (Support of Text Attribute – Strikethrough On)

27.22.4.9.9.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.9.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.9.3	Test purpose

To verify that the ME displays text formatted according to the strikethrough text attribute configuration within the command Select Item.

27.22.4.9.9.9.4	Method of test

27.22.4.9.9.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.9.4.2	Procedure

Expected Sequence 9.9 (SELECT ITEM, Text Attribute – Strikethrough On)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.9.4.2, Expected Sequence 9.9.

27.22.4.9.9.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.9.

27.22.4.9.9.10	SELECT ITEM (Support of Text Attribute – Foreground and Background Colour)

27.22.4.9.9.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.9.10.2	Conformance requirement

Requirements are the same as in clause 27.22.4.9.1.2, with an additional one:

-	3GPP 31.111 [15] clauses 6.5.4, 8.70 and 8.71.

27.22.4.9.9.10.3	Test purpose

To verify that the ME displays text formatted according to the foreground and background colour text attribute configuration within the command Select Item.

27.22.4.9.9.10.4	Method of test

27.22.4.9.9.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.9.10.4.2	Procedure

Expected Sequence 9.10 (SELECT ITEM, Text Attribute – Foreground and Background Colour)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.9.10.4.2, Expected Sequence 9.10.

27.22.4.9.9.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 9.10.

##### 27.22.4.9.10	SELECT ITEM (UCS2 display in Cyrillic)

27.22.4.9.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.10.2	Conformance requirement

The ME shall support the Proactive UICC: Select Item facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 6.4.9, clause 6.6.8, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.9, clause 9.4 and clause 10.

-	Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic characters, as defined in ISO/IEC 10646 [17].

27.22.4.9.10.3	Test purpose

To verify that the ME correctly presents the set of items in UCS2 coding contained in the SELECT ITEM proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC with the identifier of the item chosen.

To verify that the ME allows a SELECT ITEM proactive UICC command within the maximum 255 byte BER-TLV boundary.

To verify that the ME returns a TERMINAL RESPONSE with "Proactive UICC application session terminated by the user", if the user has indicated the need to end the proactive UICC session.

To verify that the ME returns a TERMINAL RESPONSE with "Backwards move in the proactive UICC application session requested by the user", if the user has indicated the need to go backwards in the proactive UICC application session.

27.22.4.9.10.4	Method of test

27.22.4.9.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.10.4.2	Procedure

Expected Sequence 10.1 (SELECT ITEM with UCS2 in Cyrillic characters, 0x80 UCS2 coding, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.10.4.2, Expected Sequence 10.1.

Expected Sequence 10.2 (SELECT ITEM with UCS2 in Cyrillic characters, 0x81 UCS2 coding, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.10.4.2, Expected Sequence 10.2.

Expected Sequence 10.3 (SELECT ITEM with UCS2 in Cyrillic characters, 0x82 UCS2 coding, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.10.4.2, Expected Sequence 10.3.

27.22.4.9.10.5	Test requirement

The ME shall operate in the manner defined in expected sequences 10.1 to 10.3.

##### 27.22.4.9.11	SELECT ITEM (UCS2 display in Chinese)

27.22.4.9.11.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.11.2	Conformance requirement

The ME shall support the Proactive UICC: Select Item facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 6.4.9, clause 6.6.8, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.9, clause 9.4 and clause 10.

-	Additionally the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in ISO/IEC 10646 [17].

27.22.4.9.11.3	Test purpose

To verify that the ME correctly presents the set of items in UCS2 coding contained in the SELECT ITEM proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC with the identifier of the item chosen.

To verify that the ME allows a SELECT ITEM proactive UICC command within the maximum 255 byte BER-TLV boundary.

To verify that the ME returns a TERMINAL RESPONSE with "Proactive UICC application session terminated by the user", if the user has indicated the need to end the proactive UICC session.

To verify that the ME returns a TERMINAL RESPONSE with "Backwards move in the proactive UICC application session requested by the user", if the user has indicated the need to go backwards in the proactive UICC application session.

27.22.4.9.11.4	Method of test

27.22.4.9.11.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.11.4.2	Procedure

Expected Sequence 11.1 (SELECT ITEM with UCS2 in Chinese characters, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.11.4.2, Expected Sequence 11.1.

27.22.4.9.11.5	Test requirement

The ME shall operate in the manner defined in expected sequence 11.1.

##### 27.22.4.9.12	SELECT ITEM (UCS2 display in Katakana)

27.22.4.9.12.1	Definition and applicability

See clause 3.2.2.

27.22.4.9.12.2	Conformance requirement

The ME shall support the Proactive UICC: Select Item facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 6.4.9, clause 6.6.8, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.9, clause 9.4 and clause 10.

-	Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in ISO/IEC 10646 [17].

27.22.4.9.12.3	Test purpose

To verify that the ME correctly presents the set of items in UCS2 coding contained in the SELECT ITEM proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC with the identifier of the item chosen.

To verify that the ME allows a SELECT ITEM proactive UICC command within the maximum 255 byte BER-TLV boundary.

To verify that the ME returns a TERMINAL RESPONSE with "Proactive UICC application session terminated by the user", if the user has indicated the need to end the proactive UICC session.

To verify that the ME returns a TERMINAL RESPONSE with "Backwards move in the proactive UICC application session requested by the user", if the user has indicated the need to go backwards in the proactive UICC application session.

27.22.4.9.12.4	Method of test

27.22.4.9.12.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.9.12.4.2	Procedure

Expected Sequence 12.1 (SELECT ITEM with UCS2 in Katakana characters, 0x80 UCS2 coding, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.12.4.2, Expected Sequence 12.1.

Expected Sequence 12.2 (SELECT ITEM with UCS2 - Katakana characters, 0x81 UCS2 coding, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.12.4.2, Expected Sequence 12.2.

Expected Sequence 12.3 (SELECT ITEM with UCS2 - Katakana characters, 0x82 UCS2 coding, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.9.12.4.2, Expected Sequence 12.3.

27.22.4.9.12.5	Test requirement

The ME shall operate in the manner defined in expected sequences 12.1 to 12.3.

#### 27.22.4.10	SEND SHORT MESSAGE

##### 27.22.4.10.1	SEND SHORT MESSAGE (normal)

27.22.4.10.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.1.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31 and clause 5.2.

27.22.4.10.1.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.1.4	Method of test

27.22.4.10.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the USS.

27.22.4.10.1.4.2	Procedure

Expected Sequence 1.1 (Void)

Expected Sequence 1.2 (Void)

Expected Sequence 1.3 (Void)

Expected Sequence 1.4 (Void)

Expected Sequence 1.5 (Void)

Expected Sequence 1.6 (Void)

Expected Sequence 1.7 (Void)

Expected Sequence 1.8 (Void)

Expected Sequence 1.9 (Send Short Message over CS/PS, UTRAN/GERAN)

In case A.1/157 is supported perform the "CS related procedure" and continue with "Generic Test Procedure 1 (SEND SHORT MESSAGE)" as defined clause 27.22.4.10.7.4.2 as "Expected Sequence 1.9" with the following parameters:

- Used Network Simulator (NWS): USS  (UMTS System Simulator or System Simulator)

- CS domain is used to send and receive short messages

- ME supports UTRAN or GERAN

CS related procedure:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NWS | ME performs CS/PS or CS registration. |  |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SEND SHORT MESSAGE) in clause 27.22.4.10.7.4.2 |  |

In case A.1/157 is not supported but A.1/159 is supported perform the "PS related procedure" and continue with "Generic Test Procedure 1 (SEND SHORT MESSAGE)" as defined clause 27.22.4.10.7.4.2 as "Expected Sequence 1.9" with the following parameters:

- Used Network Simulator (NWS): USS (UMTS System Simulator or System Simulator)

- PS domain is used to send and receive short messages

- ME supports UTRAN or GERAN

PS related procedure:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NWS | ME performs CS/PS or PS registration. |  |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SEND SHORT MESSAGE) in clause 27.22.4.10.7.4.2 |  |

27.22.4.10.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.9.

##### 27.22.4.10.2	SEND SHORT MESSAGE (UCS2 display in Cyrillic)

27.22.4.10.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.2.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31 and clause 5.2.

Additionally, the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.10.2.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.2.4	Method of test

27.22.4.10.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.2.4.2	Procedure

Expected Sequence 2.1 (SEND SHORT MESSAGE, packing not required, UCS2 (16-bit data in Cyrillic))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 2.1.1 | [packing not required, 16-bit data] |
| 4 | ME USER | Display "ЗДРАВСТВУЙТЕ" | [Alpha Identifier] "Hello" in Russian, 0x80 coding of UCS2 format |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 2.1 | Cyrillic |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 2.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 2.1.2 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 2.1.2 |  |
| 12 | ME USER | Display "ЗДРАВСТВУЙТЕ" | [Alpha Identifier] "Hello" in Russian, 0x81 coding of UCS2 format |
| 13 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 2.2 |  |
| 14 | USS ME | SMS RP-ACK |  |
| 15 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 2.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 16 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 17 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 2.1.3 |  |
| 18 | ME UICC | FETCH |  |
| 19 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 2.1.3 | [UCS2 alphabet] |
| 20 | ME USER | Display "ЗДРАВСТВУЙТЕ" | [Alpha Identifier] "Hello" in Russian, 0x82 coding of UCS2 format |
| 21 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 2.3 |  |
| 22 | USS ME | SMS RP-ACK |  |
| 23 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 2.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"ЗДРАВСТВУЙТЕ"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	24

TP-UD	"ЗДРАВСТВУЙТЕ"

Coding:

| BER-TLV: | D0 | 55 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19 | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 |
|  | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 |
|  | 04 | 15 | 86 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 |
|  | F8 | 8B | 24 | 01 | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 |
|  | 40 | 08 | 18 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 |
|  | 12 | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 |
|  | 22 | 04 | 15 |  |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 2.1

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	24

TP-UD	"ЗДРАВСТВУЙТЕ"

Coding:

| Coding | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 | 04 | 21 |
|  | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 | 04 | 15 |

SMS-PP (SEND SHORT MESSAGE) Message 2.2

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"02"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	24

TP-UD	"ЗДРАВСТВУЙТЕ"

Coding:

| Coding | 01 | 02 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 | 04 | 21 |
|  | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 | 04 | 15 |

SMS-PP (SEND SHORT MESSAGE) Message 2.3

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"03"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	24

TP-UD	"ЗДРАВСТВУЙТЕ"

Coding:

| Coding | 01 | 03 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 | 04 | 21 |
|  | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 | 04 | 15 |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 2.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"ЗДРАВСТВУЙТЕ"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	24

TP-UD	"ЗДРАВСТВУЙТЕ"

Coding:

| BER-TLV: | D0 | 4B | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0F | 81 | 0C | 08 | 97 | 94 | A0 | 90 | 92 | A1 | A2 | 92 |
|  | A3 | 99 | A2 | 95 | 86 | 09 | 91 | 11 | 22 | 33 | 44 | 55 |
|  | 66 | 77 | F8 | 8B | 24 | 01 | 00 | 09 | 91 | 10 | 32 | 54 |
|  | 76 | F8 | 40 | 08 | 18 | 04 | 17 | 04 | 14 | 04 | 20 | 04 |
|  | 10 | 04 | 12 | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 |
|  | 19 | 04 | 22 | 04 | 15 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 2.1.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"ЗДРАВСТВУЙТЕ"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	24

TP-UD	"ЗДРАВСТВУЙТЕ"

Coding:

| BER-TLV: | D0 | 4C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 82 | 0C | 04 | 10 | 87 | 84 | 90 | 80 | 82 | 91 | 92 |
|  | 82 | 93 | 89 | 92 | 85 | 86 | 09 | 91 | 11 | 22 | 33 | 44 |
|  | 55 | 66 | 77 | F8 | 8B | 24 | 01 | 00 | 09 | 91 | 10 | 32 |
|  | 54 | 76 | F8 | 40 | 08 | 18 | 04 | 17 | 04 | 14 | 04 | 20 |
|  | 04 | 10 | 04 | 12 | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 |
|  | 04 | 19 | 04 | 22 | 04 | 15 |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.10.3	SEND SHORT MESSAGE (icon support)

27.22.4.10.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.3.2	Conformance requirement

27.22.4.10.3.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.3.4	Method of test

27.22.4.10.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as Toolkit default.

The ME screen shall be in its normal stand-by display.

27.22.4.10.3.4.2	Procedure

Expected Sequence 3.1A (SEND SHORT MESSAGE, basic icon self-explanatory, packing not required, 8-bit data, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 3.1.1 | [packing not required, 8-bit data] |
| 4 | ME USER | Displays the icon and not the alpha identifier | [basic icon self-explanatory] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 3.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 3.1.1A | [Command performed successfully] |

PROACTIVE COMMAND: SEND SHORT MESSAGE 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"NO ICON"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8bit-data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Icon Identifier

Icon Qualifier	self-explanatory

Icon Identifier	1 (number of record in EF IMG)

Coding:

| BER-TLV: | D0 | 3B | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 4E | 4F | 20 | 49 | 43 | 4F | 4E | 86 | 09 | 91 | 11 |
|  | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 18 | 01 | 00 | 09 |
|  | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C | 54 | 65 | 73 |
|  | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 | 9E | 02 | 00 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 3.1

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

TERMINAL RESPONSE: SEND SHORT MESSAGE 3.1.1A

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.1B (SEND SHORT MESSAGE, basic icon self-explanatory, packing not required, 8-bit data, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 3.1.1 | [packing not required, 8-bit data, basic icon self-explanatory]] |
| 4 | ME USER | Displays the alpha identifier without the icon |  |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 3.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 3.1.1B | [Command performed successfully, but requested icon could not be displayed] |

TERMINAL RESPONSE: SEND SHORT MESSAGE 3.1.1B

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.2A (SEND SHORT MESSAGE, basic icon non-self-explanatory, packing not required, 8-bit data, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 3.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 3.2.1 | [packing not required, 8-bit data] |
| 4 | ME USER | display the icon and "Send SM" | [basic icon non-self-explanatory] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 3.2 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 3.2.1A | [Command performed successfully] |

PROACTIVE COMMAND: SEND SHORT MESSAGE 3.2.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier	"Send SM"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8bit-data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Icon Identifier

Icon Qualifier	non-self-explanatory

Icon Identifier	1 (number of record in EF IMG)

Coding:

| BER-TLV: | D0 | 3B | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 53 | 65 | 6E | 64 | 20 | 53 | 4D | 86 | 09 | 91 | 11 |
|  | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 18 | 01 | 00 | 09 |
|  | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C | 54 | 65 | 73 |
|  | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 | 1E | 02 | 01 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 3.2

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

TERMINAL RESPONSE: SEND SHORT MESSAGE 3.2.1A

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.2B (SEND SHORT MESSAGE, basic icon non-self-explanatory, packing not required, 8-bit data, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 3.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 3.2.1 | [packing not required, 8-bit data, basic icon non-self-explanatory ] |
| 4 | ME USER | display "Send SM" without the icon |  |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 3.2 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 3.2.1B | [Command performed successfully, but requested icon could not be displayed] |

TERMINAL RESPONSE: SEND SHORT MESSAGE 3.2.1B

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed;

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1A to 3.2B.

##### 27.22.4.10.4	SEND SHORT MESSAGE (Support of Text Attribute)

27.22.4.10.4.1	SEND SHORT MESSAGE (Support of Text Attribute – Left Alignment)

27.22.4.10.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.1.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.1.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the left alignment text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.1.4	Method of test

27.22.4.10.4.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.1.4.2	Procedure

Expected Sequence 4.1 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Left Alignment, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.1.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with left alignment] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.1.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.1.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/11, no alignment change will take place] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 4.1

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| Coding | 01 | 01 | 02 | 91 | 10 | 40 | F0 | 01 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SMS-PP (SEND SHORT MESSAGE) Message 4.2

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"02"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| Coding | 01 | 02 | 02 | 91 | 10 | 40 | F0 | 01 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SMS-PP (SEND SHORT MESSAGE) Message 4.3

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"03"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| Coding | 01 | 03 | 02 | 91 | 10 | 40 | F0 | 01 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SMS-PP (SEND SHORT MESSAGE) Message 4.4

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"04"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| Coding | 01 | 04 | 02 | 91 | 10 | 40 | F0 | 01 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

27.22.4.10.4.2	SEND SHORT MESSAGE (Support of Text Attribute – Center Alignment)

27.22.4.10.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.2.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.2.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the center alignment text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.2.4	Method of test

27.22.4.10.4.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.2.4.2	Procedure

Expected Sequence 4.2 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Center Alignment, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.2.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with center alignment] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.2.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.2.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.2.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/11, no alignment change will take place] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.2.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 01 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.2.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.2.

27.22.4.10.4.3	SEND SHORT MESSAGE (Support of Text Attribute – Right Alignment)

27.22.4.10.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.3.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.3.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the right alignment text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.3.4	Method of test

27.22.4.10.4.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.3.4.2	Procedure

Expected Sequence 4.3 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Right Alignment, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.3.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with right alignment] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.3.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.3.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.3.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/11, no alignment change will take place] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.3.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 02 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.3.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.3.

27.22.4.10.4.4	SEND SHORT MESSAGE (Support of Text Attribute – Large Font Size)

27.22.4.10.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.4.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.4.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the large font size text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.4.4	Method of test

27.22.4.10.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.4.4.2	Procedure

Expected Sequence 4.4 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Large Font Size, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with large font size] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.4.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.4.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with normal font size] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.4.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.4.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.1 | [packing not required, SMS default alphabet] |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with large font size] |
| 19 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 20 | USS ME | SMS RP-ACK |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.4.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.4.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.3 | [packing not required, SMS default alphabet] |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with normal font size] |
| 26 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 27 | USS ME | SMS RP-ACK |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.4.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 04 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.4.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.4.

27.22.4.10.4.5	SEND SHORT MESSAGE (Support of Text Attribute – Small Font Size)

27.22.4.10.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.5.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.5.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the small font size text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.5.4	Method of test

27.22.4.10.4.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.5.4.2	Procedure

Expected Sequence 4.5 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Small Font Size, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with small font size] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.5.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.5.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with normal font size] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.5.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.5.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.1 | [packing not required, SMS default alphabet] |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with small font size] |
| 19 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 20 | USS ME | SMS RP-ACK |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.5.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.5.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.3 | [packing not required, SMS default alphabet] |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with normal font size] |
| 26 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.4 |  |
| 27 | USS ME | SMS RP-ACK |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.5.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 08 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.5.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.5.

27.22.4.10.4.6	SEND SHORT MESSAGE (Support of Text Attribute – Bold On)

27.22.4.10.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.6.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.6.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the bold text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.6.4	Method of test

27.22.4.10.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.6.4.2	Procedure

Expected Sequence 4.6 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Bold On, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with bold on] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.6.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.6.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with bold off] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.6.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.6.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.1 | [packing not required, SMS default alphabet] |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with bold on] |
| 19 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 20 | USS ME | SMS RP-ACK |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.6.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.6.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.3 | [packing not required, SMS default alphabet] |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with bold off] |
| 26 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.4 |  |
| 27 | USS ME | SMS RP-ACK |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.6.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 10 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.6.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.6.

27.22.4.10.4.7	SEND SHORT MESSAGE (Support of Text Attribute – Italic On)

27.22.4.10.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.7.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.7.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the italic text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.7.4	Method of test

27.22.4.10.4.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.7.4.2	Procedure

Expected Sequence 4.7 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Italic On, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with italic on] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.7.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.7.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with italic off] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.7.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.7.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.1 | [packing not required, SMS default alphabet] |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with italic on] |
| 19 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 20 | USS ME | SMS RP-ACK |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.7.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.7.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.3 | [packing not required, SMS default alphabet] |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with italic off] |
| 26 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.4 |  |
| 27 | USS ME | SMS RP-ACK |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.7.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 20 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.7.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.7.

27.22.4.10.4.8	SEND SHORT MESSAGE (Support of Text Attribute – Underline On)

27.22.4.10.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.8.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.8.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the underline text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.8.4	Method of test

27.22.4.10.4.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.8.4.2	Procedure

Expected Sequence 4.8 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Underline On, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with underline on] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.8.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.8.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with underline off] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.8.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.8.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.1 | [packing not required, SMS default alphabet] |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with underline on] |
| 19 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 20 | USS ME | SMS RP-ACK |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.8.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.8.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.3 | [packing not required, SMS default alphabet] |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with underline off] |
| 26 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.4 |  |
| 27 | USS ME | SMS RP-ACK |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.8.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 40 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.8.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.8.

27.22.4.10.4.9	SEND SHORT MESSAGE (Support of Text Attribute – Strikethrough On)

27.22.4.10.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.9.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.9.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the strikethrough text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.9.4	Method of test

27.22.4.10.4.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.9.4.2	Procedure

Expected Sequence 4.9 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Strikethrough On, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with strikethrough on] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.9.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.9.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with strikethrough off] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.9.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.9.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.1 | [packing not required, SMS default alphabet] |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with strikethrough on] |
| 19 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.3 |  |
| 20 | USS ME | SMS RP-ACK |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.9.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.9.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.3 | [packing not required, SMS default alphabet] |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with strikethrough off] |
| 26 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.4 |  |
| 27 | USS ME | SMS RP-ACK |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.9.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 80 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.9.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.9.

27.22.4.10.4.10	SEND SHORT MESSAGE (Support of Text Attribute – Foreground and Background Colour)

27.22.4.10.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.4.10.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31, 8.67 and clause 5.2.

27.22.4.10.4.10.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) and display the alpha identifier according to the foreground and background colour text attribute configuration as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.4.10.4	Method of test

27.22.4.10.4.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.4.10.4.2	Procedure

Expected Sequence 4.10 (SEND SHORT MESSAGE, alpha identifier with Text attribute – Foreground and Background Colour, packing not required, SMS default alphabet, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.10.1 | [packing not required, SMS default alphabet] |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with foreground and background colour according to text attribute configuration] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.10.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 4.10.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 4.10.2 | [packing not required, SMS default alphabet] |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with ME's default foreground and background colour] |
| 12 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 4.2 |  |
| 13 | USS ME | SMS RP-ACK |  |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 4.10.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 4.10.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"01"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	1

TP-UD	" "

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8B | 09 | 01 | 00 | 02 | 91 | 10 |
|  | 40 | F0 | 01 | 20 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.4.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.10.

##### 27.22.4.10.5	SEND SHORT MESSAGE (UCS2 display in Chinese)

27.22.4.10.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.5.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31 and clause 5.2.

Additionally, the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.10.5.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.5.4	Method of test

27.22.4.10.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.5.4.2	Procedure

Expected Sequence 5.1 (SEND SHORT MESSAGE, packing not required, UCS2 (16-bit data in Chinese))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 5.1.1 | [packing not required, 16-bit data] |
| 4 | ME USER | Display "中一" | [Alpha Identifier] "Middle 1" in Chinese, 0x80 coding of UCS2 format |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 5.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 5.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 5.1.2 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND SEND SHORT MESSAGE 5.1.2 |  |
| 12 | ME USER | Display "中一" | [Alpha Identifier] "Middle 1" in Chinese, 0x81 coding of UCS2 format |
| 13 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 5.2 |  |
| 14 | USS ME | SMS RP-ACK |  |
| 15 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 5.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 16 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 17 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 5.1.3 |  |
| 18 | ME UICC | FETCH |  |
| 19 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 5.1.3 | [UCS2 alphabet] |
| 20 | ME USER | Display "中一" | [Alpha Identifier] "Middle 1" in Chinese, 0x82 coding of UCS2 format |
| 21 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 5.3 |  |
| 22 | USS ME | SMS RP-ACK |  |
| 23 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 5.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"中一"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	24

TP-UD	"中一"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 4E | 2D | 4E | 00 | 86 | 09 | 91 | 11 | 22 | 33 |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 10 | 01 | 00 | 09 | 91 | 10 |
|  | 32 | 54 | 76 | F8 | 40 | 08 | 04 | 4E | 2D | 4E | 00 |  |

SMS-PP (SEND SHORT MESSAGE) Message 5.1

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	24

TP-UD	"中一"

Coding:

| BER-TLV: | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4E | 2D | 4E | 00 |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 5.2

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"02"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	24

TP-UD	"中一"

Coding:

| BER-TLV: | 01 | 02 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4E | 2D | 4E | 00 |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 5.3

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"03"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	24

TP-UD	"中一"

Coding:

| BER-TLV: | 01 | 03 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4E | 2D | 4E | 00 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 5.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"中一"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	24

TP-UD	"中一"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 81 | 02 | 9C | AD | 80 | 86 | 09 | 91 | 11 | 22 | 33 |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 10 | 01 | 00 | 09 | 91 | 10 |
|  | 32 | 54 | 76 | F8 | 40 | 08 | 04 | 4E | 2D | 4E | 00 |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 5.1.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"中一"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	24

TP-UD	"中一"

Coding:

| BER-TLV: | D0 | 2E | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 06 | 82 | 02 | 4E | 00 | AD | 80 | 86 | 09 | 91 | 11 | 22 |
|  | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 10 | 01 | 00 | 09 | 91 |
|  | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 04 | 4E | 2D | 4E | 00 |

TERMINAL RESPONSE: SEND SHORT MESSAGE 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.10.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.10.6	SEND SHORT MESSAGE (UCS2 display in Katakana)

27.22.4.10.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.6.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31 and clause 5.2.

Additionally, the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in the following technical specifications: ISO/IEC 10646 [17].

27.22.4.10.6.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (USS) as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.6.4	Method of test

27.22.4.10.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.10.6.4.2	Procedure

Expected Sequence 6.1 (SEND SHORT MESSAGE, packing not required, UCS2 (16-bit data, in Katakana))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 6.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 6.1.1 | [packing not required, 16-bit data] |
| 4 | ME USER | Display "80ル0" | [Characters in katakana] |
| 5 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 6.1 |  |
| 6 | USS ME | SMS RP-ACK |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 6.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 6.1.2 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 6.1.2 | [packing not required, 16-bit data] |
| 12 | ME USER | Display "81ル1" | [Characters in katakana] |
| 13 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 6.2 |  |
| 14 | USS ME | SMS RP-ACK |  |
| 15 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 6.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 16 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 17 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 6.1.3 |  |
| 18 | ME UICC | FETCH |  |
| 19 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 6.1.3 | [packing not required, 16-bit data] |
| 20 | ME USER | Display "82ル2" | [Characters in katakana] |
| 21 | ME USS | Send SMS-PP (SEND SHORT MESSAGE) Message 6.3 |  |
| 22 | USS ME | SMS RP-ACK |  |
| 23 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 6.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"80ル0"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept a SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	10

TP-UD	"80ル1"

Coding:

| BER-TLV: | D0 | 35 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 80 | 00 | 38 | 00 | 30 | 30 | EB | 00 | 30 | 86 | 09 |
|  | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 14 | 01 |
|  | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 08 | 00 |
|  | 38 | 00 | 30 | 30 | EB | 00 | 31 |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 6.1

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	10

TP-UD	"80ル1"

Coding:

| Coding | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 38 | 00 | 30 | 30 | EB | 00 | 31 |  |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 6.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"81ル1"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept a SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	10

TP-UD	"80ル2"

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 81 | 04 | 61 | 38 | 31 | EB | 31 | 86 | 09 | 91 | 11 |
|  | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 14 | 01 | 00 | 09 |
|  | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 08 | 00 | 38 | 00 |
|  | 30 | 30 | EB | 00 | 32 |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 6.2

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"02"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	10

TP-UD	"80ル2"

Coding:

| Coding | 01 | 02 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 38 | 00 | 30 | 30 | EB | 00 | 32 |  |  |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE: 6.1.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"82ル2"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept a SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	16-bit data

Message class	class 0

TP-UDL	10

TP-UD	"80ル3"

Coding:

| BER-TLV: | D0 | 34 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 82 | 04 | 30 | A0 | 38 | 32 | CB | 32 | 86 | 09 | 91 |
|  | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 14 | 01 | 00 |
|  | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 08 | 00 | 38 |
|  | 00 | 30 | 30 | EB | 00 | 33 |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 6.3

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"03"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	UCS2 (16-bit data)

Message class	class 0

TP-UDL	10

TP-UD	"80ル3"

Coding:

| Coding | 01 | 03 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | 08 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 38 | 00 | 30 | 30 | EB | 00 | 33 |  |  |  |  |

27.22.4.10.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

##### 27.22.4.10.7	SEND SHORT MESSAGE (IMS)

27.22.4.10.7.1	Definition and applicability

See clause 3.2.2.

That the UE correctly implemented the role of an SMS-over-IP sender is tested in clause 18.1 of TS 34.229-1 [36].

27.22.4.10.7.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility for SMS over IP according to:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31 and clause 5.2.

-	TS 31.103 [35].

-	TS 34.229-1 [36], Annexes C.2, C.17 and C.18.

-	TS 24.341 [37], clause 5.3.1.

27.22.4.10.7.3	Test purpose

- To verify that the ME correctly formats and sends a short message via IMS to the E-USS/USS as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

- To verify that the ME uses the default service address as indicated in EF SMSP if no service center address is available in the Send Short Message command.

- To verify that a device of  Class ND does not reject the Send Short Message command if the proactive Send Short Message command contains an alpha identifier.

27.22.4.10.7.4	Method of test

27.22.4.10.7.4.1	Initial conditions

The ME is connected to the USIM Simulator. The elementary files are coded as defined for the E-UTRAN/EPC ISIM-UICC in clause 27.22.2C.

For sequence 7.1 the ME is additionally connected to the E-USS.

For sequence 7.2 the ME is additionally connected to the USS.

27.22.4.10.7.4.2	Procedure

Expected Sequence 7.1 (SEND SHORT MESSAGE, SMS-over-IP, E-UTRAN)

Perform the "IMS related procedure 1" and continue with "Generic Test Procedure 1 (SEND SHORT MESSAGE)" as defined in this clause as "Expected Sequence 7.1" with the following parameters:

- Used Network Simulator (NWS): E-USS

- SMS-over-IP is used to send and receive short messages

- ME supports eFDD or eTDD and SMS-over-IP

Expected Sequence 7.2 (SEND SHORT MESSAGE, SMS-over-IP, UTRAN)

Perform the "IMS related procedure 1" and continue with "Generic Test Procedure 1 (SEND SHORT MESSAGE)" as defined in this clause as "Expected Sequence 7.2" with the following parameters:

- Used Network Simulator (NWS): USS  (UMTS System Simulator only)

- SMS-over-IP is used to send and receive short messages

- ME supports UTRAN and SMS-over-IP

IMS related procedure 1:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profle Download, USIM and ISIM initialisation |
| 2 | ME NWS | ME activates the required bearer, discoveres P-CSCF and registers with the values from the ISIM to IMS services | For E-UTRAN: The EPS bearer context activation according to the procedures defined in TS 34.229-1 [36], Annex C.2 and C.18 is performed For UTRAN: For SMS-over-IP a PDP context activation according to the procedures defined in TS 34.229-1 [36], Annex C.2 and C.17 is performed. |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SEND SHORT MESSAGE) |  |

Generic Test Procedure 1 (SEND SHORT MESSAGE)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 7.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.1 | [packing not required, SMS default alphabet] |
| 7 | ME NWS | Send RP-DATA containing SMS-PP (SEND SHORT MESSAGE) Message 7.1 | See Note 1. In case of SMS-over-IP the RP-Destination Address (SM Service Center Address within the RP-DATA) is taken from the ISIM (EF SMSP) |
| 8 | NWS ME | RP-ACK | See Note 2. |
| 9 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.1 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "01" |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 7.1. 2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.2 | [packing required, 8 bit data] |
| 13 | ME USER | Display "The address data object holds the RP_Destination_Address " | [Alpha Identifier not to be displayed by Terminals of Class_ND] |
| 14 | ME NWS | Send RP-DATA containing SMS-PP (SEND SHORT MESSAGE) Message 7.2 | See Note 1. |
| 15 | NWS ME | RP-ACK | See Note 2. |
| 16 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.2 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "02" |
| 17 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 7.1.3 |  |
| 18 | ME UICC | FETCH |  |
| 19 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.3 | [packing not required, SMS default alphabet] |
| 20 | ME USER | Display "The address data object holds the RP Destination Address " | [Alpha Identifier not to be displayed by Terminals of Class_ND] |
| 21 | ME NWS | Send RP-DATA containing SMS-PP (SEND SHORT MESSAGE) Message 7.3 | See Note 1. |
| 22 | NWS ME | RP-ACK | See Note 2. |
| 23 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.3 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "03" |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 7.1.4 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.4 | [packing not required, 8-bit data] |
| 27 | ME | No information to user | [Alpha identifier length '00'] |
| 28 | ME NWS | Send RP-DATA containing SMS-PP (SEND SHORT MESSAGE) Message 7.4 | See Note 1. |
| 29 | NWS ME | RP-ACK | See Note 2. |
| 30 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.4 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "04" |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 7.1.5 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.5 | [packing not required, 8-bit data] |
| 34 | ME USER | May give information to user concerning what is happening | [No Alpha Identifier] |
| 35 | ME NWS | Send RP-DATA containing SMS-PP (SEND SHORT MESSAGE) Message 7.5 | See Note 1. |
| 36 | NWS ME | RP-ACK | See Note 2. |
| 37 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.5 | [Command performed successfully] The UE shall have updated Last-Used-TP-MR of EF SMSS to "05" |
| 38 | USER ME | The ME is switched off |  |
| Note 1: In case of IMS the RP-DATA is contained in the SIP MESSAGE which is built according to TS 24.341 [37], clause 5.3.1.2 including PSI of the SMSC from EF PSISMSC. Note 2: In case of IMS the RP-ACK message is contained in the message body of the SIP MESSAGE. |  |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 8B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 18 | 01 | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F0 |
|  | 0D | 53 | F4 | 5B | 4E | 07 | 35 | CB | F3 | 79 | F8 | 5C |
|  | 06 |  |  |  |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 7.1

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| Coding | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F0 | 0D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 53 | F4 | 5B | 4E | 07 | 35 | CB | F3 | 79 | F8 | 5C | 06 |

TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.1/7.1.3/7.1.4, 7.1.5

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"The address data object holds the RP_Destination_Address"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8 bit data

Message class	class 0

TP-UDL	160

TP-UD	"Two types are defined: - A short message to be sent to the network in an SMS-SUBMIT message, or an SMS-COMMAND message, where the user data can be passed transp"

Coding:

| BER-TLV: | D0 | 81 | FD | 81 | 03 | 01 | 13 | 01 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 38 | 54 | 68 | 65 | 20 | 61 | 64 | 64 | 72 | 65 | 73 |
|  | 73 | 20 | 64 | 61 | 74 | 61 | 20 | 6F | 62 | 6A | 65 | 63 |
|  | 74 | 20 | 68 | 6F | 6C | 64 | 73 | 20 | 74 | 68 | 65 | 20 |
|  | 52 | 50 | 11 | 44 | 65 | 73 | 74 | 69 | 6E | 61 | 74 | 69 |
|  | 6F | 6E | 11 | 41 | 64 | 64 | 72 | 65 | 73 | 73 | 86 | 09 |
|  | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 81 | AC |
|  | 01 | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | A0 |
|  | 54 | 77 | 6F | 20 | 74 | 79 | 70 | 65 | 73 | 20 | 61 | 72 |
|  | 65 | 20 | 64 | 65 | 66 | 69 | 6E | 65 | 64 | 3A | 20 | 2D |
|  | 20 | 41 | 20 | 73 | 68 | 6F | 72 | 74 | 20 | 6D | 65 | 73 |
|  | 73 | 61 | 67 | 65 | 20 | 74 | 6F | 20 | 62 | 65 | 20 | 73 |
|  | 65 | 6E | 74 | 20 | 74 | 6F | 20 | 74 | 68 | 65 | 20 | 6E |
|  | 65 | 74 | 77 | 6F | 72 | 6B | 20 | 69 | 6E | 20 | 61 | 6E |
|  | 20 | 53 | 4D | 53 | 2D | 53 | 55 | 42 | 4D | 49 | 54 | 20 |
|  | 6D | 65 | 73 | 73 | 61 | 67 | 65 | 2C | 20 | 6F | 72 | 20 |
|  | 61 | 6E | 20 | 53 | 4D | 53 | 2D | 43 | 4F | 4D | 4D | 41 |
|  | 4E | 44 | 20 | 6D | 65 | 73 | 73 | 61 | 67 | 65 | 2C | 20 |
|  | 77 | 68 | 65 | 72 | 65 | 20 | 74 | 68 | 65 | 20 | 75 | 73 |
|  | 65 | 72 | 20 | 64 | 61 | 74 | 61 | 20 | 63 | 61 | 6E | 20 |
|  | 62 | 65 | 20 | 70 | 61 | 73 | 73 | 65 | 64 | 20 | 74 | 72 |
|  | 61 | 6E | 73 | 70 |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 7.2

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"02"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	160

TP-UD	"Two types are defined: - A short message to be sent to the network in an SMS-SUBMIT message, or an SMS-COMMAND message, where the user data can be passed transp"

Coding:

| Coding |  | 01 | 02 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A0 | D4 | FB | 1B | 44 | CF | C3 | CB | 73 | 50 | 58 | 5E |
|  | 06 | 91 | CB | E6 | B4 | BB | 4C | D6 | 81 | 5A | A0 | 20 |
|  | 68 | 8E | 7E | CB | E9 | A0 | 76 | 79 | 3E | 0F | 9F | CB |
|  | 20 | FA | 1B | 24 | 2E | 83 | E6 | 65 | 37 | 1D | 44 | 7F |
|  | 83 | E8 | E8 | 32 | C8 | 5D | A6 | DF | DF | F2 | 35 | 28 |
|  | ED | 06 | 85 | DD | A0 | 69 | 73 | DA | 9A | 56 | 85 | CD |
|  | 24 | 15 | D4 | 2E | CF | E7 | E1 | 73 | 99 | 05 | 7A | CB |
|  | 41 | 61 | 37 | 68 | DA | 9C | B6 | 86 | CF | 66 | 33 | E8 |
|  | 24 | 82 | DA | E5 | F9 | 3C | 7C | 2E | B3 | 40 | 77 | 74 |
|  | 59 | 5E | 06 | D1 | D1 | 65 | 50 | 7D | 5E | 96 | 83 | C8 |
|  | 61 | 7A | 18 | 34 | 0E | BB | 41 | E2 | 32 | 08 | 1E | 9E |
|  | CF | CB | 64 | 10 | 5D | 1E | 76 | CF | E1 |  |  |  |

TERMINAL RESPONSE: SEND SHORT MESSAGE 7.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.3

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"The address data object holds the RP Destination Address"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	160

TP-UD	"Two types are defined: - A short message to be sent to the network in an SMS-SUBMIT message, or an SMS-COMMAND message, where the user data can be passed transp"

Coding:

| BER-TLV: | D0 | 81 | E9 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 38 | 54 | 68 | 65 | 20 | 61 | 64 | 64 | 72 | 65 | 73 |
|  | 73 | 20 | 64 | 61 | 74 | 61 | 20 | 6F | 62 | 6A | 65 | 63 |
|  | 74 | 20 | 68 | 6F | 6C | 64 | 73 | 20 | 74 | 68 | 65 | 20 |
|  | 52 | 50 | 20 | 44 | 65 | 73 | 74 | 69 | 6E | 61 | 74 | 69 |
|  | 6F | 6E | 20 | 41 | 64 | 64 | 72 | 65 | 73 | 73 | 86 | 09 |
|  | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 81 | 98 |
|  | 01 | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F0 | A0 |
|  | D4 | FB | 1B | 44 | CF | C3 | CB | 73 | 50 | 58 | 5E | 06 |
|  | 91 | CB | E6 | B4 | BB | 4C | D6 | 81 | 5A | A0 | 20 | 68 |
|  | 8E | 7E | CB | E9 | A0 | 76 | 79 | 3E | 0F | 9F | CB | 20 |
|  | FA | 1B | 24 | 2E | 83 | E6 | 65 | 37 | 1D | 44 | 7F | 83 |
|  | E8 | E8 | 32 | C8 | 5D | A6 | DF | DF | F2 | 35 | 28 | ED |
|  | 06 | 85 | DD | A0 | 69 | 73 | DA | 9A | 56 | 85 | CD | 24 |
|  | 15 | D4 | 2E | CF | E7 | E1 | 73 | 99 | 05 | 7A | CB | 41 |
|  | 61 | 37 | 68 | DA | 9C | B6 | 86 | CF | 66 | 33 | E8 | 24 |
|  | 82 | DA | E5 | F9 | 3C | 7C | 2E | B3 | 40 | 77 | 74 | 59 |
|  | 5E | 06 | D1 | D1 | 65 | 50 | 7D | 5E | 96 | 83 | C8 | 61 |
|  | 7A | 18 | 34 | 0E | BB | 41 | E2 | 32 | 08 | 1E | 9E | CF |
|  | CB | 64 | 10 | 5D | 1E | 76 | CF | E1 |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 7.3

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"03"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	SMS default alphabet

Message class	class 0

TP-UDL	160

TP-UD	"Two types are defined: - A short message to be sent to the network in an SMS-SUBMIT message, or an SMS-COMMAND message, where the user data can be passed transp"

Coding:

| Coding | 01 | 03 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F0 | A0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D4 | FB | 1B | 44 | CF | C3 | CB | 73 | 50 | 58 | 5E | 06 |
|  | 91 | CB | E6 | B4 | BB | 4C | D6 | 81 | 5A | A0 | 20 | 68 |
|  | 8E | 7E | CB | E9 | A0 | 76 | 79 | 3E | 0F | 9F | CB | 20 |
|  | FA | 1B | 24 | 2E | 83 | E6 | 65 | 37 | 1D | 44 | 7F | 83 |
|  | E8 | E8 | 32 | C8 | 5D | A6 | DF | DF | F2 | 35 | 28 | ED |
|  | 06 | 85 | DD | A0 | 69 | 73 | DA | 9A | 56 | 85 | CD | 24 |
|  | 15 | D4 | 2E | CF | E7 | E1 | 73 | 99 | 05 | 7A | CB | 41 |
|  | 61 | 37 | 68 | DA | 9C | B6 | 86 | CF | 66 | 33 | E8 | 24 |
|  | 82 | DA | E5 | F9 | 3C | 7C | 2E | B3 | 40 | 77 | 74 | 59 |
|  | 5E | 06 | D1 | D1 | 65 | 50 | 7D | 5E | 96 | 83 | C8 | 61 |
|  | 7A | 18 | 34 | 0E | BB | 41 | E2 | 32 | 08 | 1E | 9E | CF |
|  | CB | 64 | 10 | 5D | 1E | 76 | CF | E1 |  |  |  |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.4

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| BER-TLV: | D0 | 30 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 86 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 |
|  | 8B | 18 | 01 | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 |
|  | F4 | 0C | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 |
|  | 67 | 65 |  |  |  |  |  |  |  |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 7.4

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"04"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 01 | 04 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

PROACTIVE COMMAND: SEND SHORT MESSAGE 7.1.5

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| BER-TLV: | D0 | 2E | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 86 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 18 |
|  | 01 | 00 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

SMS-PP (SEND SHORT MESSAGE) Message 7.5

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"05"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 01 | 05 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

27.22.4.10.7.5	Test requirement

The ME supporting eFDD or eTDD shall operate in the manner defined in expected sequence 7.1.

The ME supporting UTRAN shall operate in the manner defined in expected sequence 7.2.

##### 27.22.4.10.8	SEND SHORT MESSAGE (over SGs in E-UTRAN)

27.22.4.10.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.10.8.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.10, clause 6.6.9, clause 8.1, clause 8.2, clause 8.6, clause 8.7, clause 8.13, clause 8.31 and clause 5.2.

-	TS 24.301 [32] clause 5.6.3.1, 5.6.3.3 and 9.9.3.22

27.22.4.10.8.3	Test purpose

To verify that the ME correctly formats and sends a short message to the network (E-USS/NB-SS) using SMS over SGs as indicated in the SEND SHORT MESSAGE proactive UICC command, and returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the Short Message.

27.22.4.10.8.4	Method of test

27.22.4.10.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the E-USS/NB-SS.

27.22.4.10.8.4.2	Procedure

Expected Sequence 8.1 (Send Short Message over SGs, E-UTRAN)

Perform the "SMS over SGs procedure" and continue with "Generic Test Procedure 1 (SEND SHORT MESSAGE)" as defined clause 27.22.4.10.7.4.2 as "Expected Sequence 8.1" with the following parameters:

- Used Network Simulator (NWS): E-USS/NB-SS

- SMS over SGs (DOWNLINK NAS TRANSPORT and UPLINK NAS TRANSPORT messages)
 is used to send and receive short messages

- ME supports eFDD or eTDD or NB-IoT

- ME supports SMS-over-SGs.

SMS over SGs related procedure:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profile Download and USIM initialisation |
| 2 | ME NWS | ME performs regular network registration. | UE is afterwards in state Registered, Idle Mode (state 2) according to TS 36.508 [33]. |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SEND SHORT MESSAGE) in clause 27.22.4.10.7.4.2 |  |

27.22.4.10.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 8.1.

#### 27.22.4.11	SEND SS

##### 27.22.4.11.1	SEND SS (normal)

27.22.4.11.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.1.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.1.3	Test purpose

To verify that the ME correctly translates and sends the supplementary service request indicated in the SEND SS proactive UICC command to the USS.

To verify that the ME returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the SS and any contents of the SS result as additional data.

27.22.4.11.1.4	Method of test

27.22.4.11.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.1.4.2	Procedure

Expected Sequence 1.1A (SEND SS, call forward unconditional, all bearers, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.1.1 |  |
| 4 | ME USER | Display "Call Forward" |  |
| 5 | ME USS | REGISTER 1.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.1.1A |  |

Expected Sequence 1.1B (SEND SS, call forward unconditional, all bearers, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.1.1 |  |
| 4 | ME USER | Display "Call Forward" |  |
| 5 | ME USS | REGISTER 1.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.1.1B |  |

PROACTIVE COMMAND: SEND SS 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Call Forward"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 29 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0C | 43 | 61 | 6C | 6C | 20 | 46 | 6F | 72 | 77 | 61 | 72 |
|  | 64 | 89 | 10 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 |
|  | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |  |  |  |  |

REGISTER 1.1A

Logically (only SS argument):

REGISTER SS ARGUMENT

SS-Code:

- Call Forwarding Unconditional

TeleserviceCode

- All Tele Services

ForwardedToNumber

- nature of address ind.: international

- numbering plan ind.:	ISDN/Telephony (E.164)

- TBCD String:	01234567890123456789

- longFTN-Supported

Coding:

| BER-TLV | 30 | 15 | 04 | 01 | 21 | 83 | 01 | 00 | 84 | 0B | 91 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 | 89 | 00 |  |

REGISTER 1.1B

Logically (only SS argument):

REGISTER SS ARGUMENT

SS-Code:

- Call Forwarding Unconditional

TeleserviceCode

- All Tele Services

ForwardedToNumber

- nature of address ind.:	international

- numbering plan ind.:	ISDN/Telephony (E.164)

- TBCD String:	01234567890123456789

Coding:

| BER-TLV | 30 | 13 | 04 | 01 | 21 | 83 | 01 | 00 | 84 | 0B | 91 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 1.1A

Logically (only from operation code):

REGISTER SS RETURN RESULT

ForwardingInfo

SS-Code

- Call Forwarding Unconditional

ForwardFeatureList

ForwardingFeature

TeleserviceCode

- All Tele Services

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: active

longForwardedToNumber

- nature of address ind.: international

- numbering plan ind.: ISDN/Telephony (E.164)

- TBCD String: 01234567890123456789

Coding:

| Coding | 0A | A0 | 1A | 04 | 01 | 21 | 30 | 15 | 30 | 13 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 84 | 01 | 07 | 89 | 0B | 91 | 10 | 32 | 54 | 76 | 98 |
|  | 10 | 32 | 54 | 76 | 98 |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 1.1B

Logically (only from operation code):

REGISTER SS RETURN RESULT

ForwardingInfo

SS-Code

- Call Forwarding Unconditional

ForwardFeatureList

ForwardingFeature

TeleserviceCode

- All Tele Services

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: active

Coding:

| Coding | 0A | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 84 | 01 | 07 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND SS 1.1.1A

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information:	Operation Code and SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 1E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0A | A0 | 1A | 04 | 01 | 21 | 30 | 15 | 30 | 13 |
|  | 83 | 01 | 00 | 84 | 01 | 07 | 89 | 0B | 91 | 10 | 32 |
|  | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 |  |  |  |

TERMINAL RESPONSE: SEND SS 1.1.1B

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information:	Operation Code and SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0A | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 |
|  | 83 | 01 | 00 | 84 | 01 | 07 |  |  |  |  |  |

Expected Sequence 1.2 (SEND SS, call forward unconditional, all bearers, Return Error)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.1.1 |  |
| 4 | ME USER | Display "Call Forward" |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN ERROR) 1.1 | [Return Error] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.2.1 |  |

RELEASE COMPLETE (SS RETURN ERROR) 1.1

Logically (only from error code):

Error Code:	Facility not supported

Coding:

| Coding | 02 | 01 | 15 |
| --- | --- | --- | --- |

TERMINAL RESPONSE: SEND SS 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	SS Return Error

Additional information:	Error Code

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 34 | 15 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.3 (SEND SS, call forward unconditional, all bearers, Reject)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.1.1 |  |
| 4 | ME USER | Display "Call Forward" |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS REJECT) 1.1. | [Reject] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.3.1 |  |

RELEASE COMPLETE (SS REJECT) 1.1

Logically (only from problem code):

Problem Code:

- General problem

- Unrecognized component

Coding:

| Coding | 80 | 01 | 00 |
| --- | --- | --- | --- |

TERMINAL RESPONSE: SEND SS 1.3.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	SS Return Error

Additional information:	No specific cause can be given

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 34 | 00 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.4A (SEND SS, call forward unconditional, all bearers, successful, SS request size limit)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.4.1 |  |
| 4 | ME USER | Display "Call Forward" |  |
| 5 | ME USS | REGISTER 1.2A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.2A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.4.1A |  |

Expected Sequence 1.4B (SEND SS, call forward unconditional, all bearers, successful, SS request size limit)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.4.1 |  |
| 4 | ME USER | Display "Call Forward" |  |
| 5 | ME USS | REGISTER 1.2B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.2B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.4.1B |  |

PROACTIVE COMMAND: SEND SS 1.4.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Call Forward"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*0123456789012345678901234567*11#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0C | 43 | 61 | 6C | 6C | 20 | 46 | 6F | 72 | 77 | 61 | 72 |
|  | 64 | 89 | 14 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 |
|  | 21 | 43 | 65 | 87 | 09 | 21 | 43 | 65 | A7 | 11 | FB |  |

REGISTER 1.2A

Logically (only SS argument):

REGISTER SS ARGUMENT

RegisterSSArg

SS-Code

Call Forwarding Unconditional

TeleserviceCode

See Note 1

ForwardedToNumber

nature of address ind.:	international

numbering plan ind.:	ISDN/Telephony (E.164)

TBCD String:	0123456789012345678901234567

longFTN-Supported

Coding:

| BER-TLV | 30 | 19 | 04 | 01 | 21 | 83 | 01 | Note 1 | 84 | 0F | 91 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 |
|  | 76 | 89 | 00 |  |  |  |  |  |  |  |  |  |

Note 1: TeleserviceCode is '11' for "Telephony" or is '10' for "allSpeechTransmissionServices"

REGISTER 1.2B

Logically (only SS argument):

REGISTER SS ARGUMENT

RegisterSSArg

SS-Code

Call Forwarding Unconditional

TeleserviceCode

See Note 1

ForwardedToNumber

nature of address ind.:	international

numbering plan ind.:	ISDN/Telephony (E.164)

TBCD String:		0123456789012345678901234567

Coding:

| BER-TLV | 30 | 17 | 04 | 01 | 21 | 83 | 01 | Note 1 | 84 | 0F | 91 | 10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 |
|  | 76 |  |  |  |  |  |  |  |  |  |  |  |

Note 1: TeleserviceCode is '11' for "Telephony" or is '10' for "allSpeechTransmissionServices"

Logically (only from operation code):

REGISTER SS RETURN RESULT

ForwardingInfo

SS-Code

- Call Forwarding Unconditional

ForwardFeatureList

ForwardingFeature

TeleserviceCode

- See Note 1

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: active

longForwardedToNumber

- nature of address ind.: international

- numbering plan ind.: ISDN/Telephony (E.164)

- TBCD String: 0123456789012345678901234567

Coding:

| Coding | 0A | A0 | 1E | 04 | 01 | 21 | 30 | 19 | 30 | 17 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 1 | 84 | 01 | 07 | 89 | 0F | 91 | 10 | 32 | 54 | 76 | 98 |
|  | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 |  |  |  |

Note 1: TeleserviceCode is '11' for "Telephony" or is '10' for "allSpeechTransmissionServices"

RELEASE COMPLETE (SS RETURN RESULT) 1.2B

Logically (only from operation code):

REGISTER SS RETURN RESULT

ForwardingInfo

SS-Code

- Call Forwarding Unconditional

ForwardFeatureList

ForwardingFeature

TeleserviceCode

- See Note 1

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: active

Coding:

| Coding | 0A | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 1 | 84 | 01 | 07 |  |  |  |  |  |  |  |  |

Note 1: TeleserviceCode is '11' for "Telephony" or is '10' for "allSpeechTransmissionServices"

TERMINAL RESPONSE: SEND SS 1.4.1A

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information:	Operation Code and SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0A | A0 | 1E | 04 | 01 | 21 | 30 | 19 | 30 | 17 |
|  | 83 | 01 | Note 1 | 84 | 01 | 07 | 89 | 0F | 91 | 10 | 32 |
|  | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 |
|  | 76 |  |  |  |  |  |  |  |  |  |  |

Note 1: TeleserviceCode is '11' for "Telephony" or is '10' for "allSpeechTransmissionServices"

TERMINAL RESPONSE: SEND SS 1.4.1B

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information:	Operation Code and SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0A | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 |
|  | 83 | 01 | Note 1 | 84 | 01 | 07 |  |  |  |  |  |

Note 1: TeleserviceCode is '11' for "Telephony" or is '10' for "allSpeechTransmissionServices"

Expected Sequence 1.5 (SEND SS, interrogate CLIR status, successful, alpha identifier limits)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.5.1 |  |
| 4 | ME USER | Display "Even if the Fixed Dialling Number service is enabled, the supplementary service control string included in the SEND SS proactive command shall not be checked against those of the FDN list. Upon receiving this command, the ME shall deci" |  |
| 5 | ME USS | REGISTER 1.3 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.3 | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.5.1 |  |

PROACTIVE COMMAND: SEND SS 1.5.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Even if the Fixed Dialling Number service is enabled, the supplementary service control string included in the SEND SS proactive command shall not be checked against those of the FDN list. Upon receiving this command, the ME shall deci"

SS String

TON:	Undefined

NPI:	Undefined

SS string:	"*#31#"

Coding:

| BER-TLV: | D0 | 81 | FD | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 81 | EB | 45 | 76 | 65 | 6 | 20 | 69 | 66 | 20 | 74 |
|  | 68 | 65 | 20 | 46 | 69 | 78 | 65 | 64 | 20 | 44 | 69 | 61 |
|  | 6C | 6C | 69 | 6E | 67 | 20 | 4E | 75 | 6D | 62 | 65 | 72 |
|  | 20 | 73 | 65 | 72 | 76 | 69 | 63 | 65 | 20 | 69 | 73 | 20 |
|  | 65 | 6E | 61 | 62 | 6C | 65 | 64 | 2C | 20 | 74 | 68 | 65 |
|  | 20 | 73 | 75 | 70 | 70 | 6C | 65 | 6D | 65 | 6E | 74 | 61 |
|  | 72 | 79 | 20 | 73 | 65 | 72 | 76 | 69 | 63 | 65 | 20 | 63 |
|  | 6F | 6E | 74 | 72 | 6F | 6C | 20 | 73 | 74 | 72 | 69 | 6E |
|  | 67 | 20 | 69 | 6E | 63 | 6C | 75 | 64 | 65 | 64 | 20 | 69 |
|  | 6E | 20 | 74 | 68 | 65 | 20 | 53 | 45 | 4E | 44 | 20 | 53 |
|  | 53 | 20 | 70 | 72 | 6F | 61 | 63 | 74 | 69 | 76 | 65 | 20 |
|  | 63 | 6F | 6D | 6D | 61 | 6E | 64 | 20 | 73 | 68 | 61 | 6C |
|  | 6C | 20 | 6E | 6F | 74 | 20 | 62 | 65 | 20 | 63 | 68 | 65 |
|  | 63 | 6B | 65 | 64 | 20 | 61 | 67 | 61 | 69 | 6E | 73 | 74 |
|  | 20 | 74 | 68 | 6F | 73 | 65 | 20 | 6F | 66 | 20 | 74 | 68 |
|  | 65 | 20 | 46 | 44 | 4E | 20 | 6C | 69 | 73 | 74 | 2E | 20 |
|  | 55 | 70 | 6F | 6E | 20 | 72 | 65 | 63 | 65 | 69 | 76 | 69 |
|  | 6E | 67 | 20 | 74 | 68 | 69 | 73 | 20 | 63 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 2C | 20 | 74 | 68 | 65 | 20 | 4D | 45 | 20 |
|  | 73 | 68 | 61 | 6C | 6C | 20 | 64 | 65 | 63 | 69 | 89 | 04 |
|  | FF | BA | 13 | FB |  |  |  |  |  |  |  |  |

REGISTER 1.3

Logically (only SS argument):

INTERROGATE SS ARGUMENT

SS-Code

- Calling Line Id Restriction

Coding:

| BER-TLV | 30 | 03 | 04 | 01 | 12 |
| --- | --- | --- | --- | --- | --- |

RELEASE COMPLETE (SS RETURN RESULT) 1.3

Logically (only from operation code):

INTERROGATE SS RESULT

CliRestrictionInfo

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: not active

CliRestrictionOption

- Temporary Def Allowed

Coding:

| Coding | 0E | A4 | 06 | 04 | 01 | 06 | 0A | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: SEND SS 1.5.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information

Operation Code:	SS Code

Parameters:	SS Return Result

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 0A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0E | A4 | 06 | 04 | 01 | 06 | 0A | 01 | 02 |  |

Expected Sequence 1.6A (SEND SS, call forward unconditional, all bearers, successful, null data alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.6.1 |  |
| 4 | ME | Should not give any information to the user on the fact that the ME is sending an SS request |  |
| 5 | ME USS | REGISTER 1.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.1.1A |  |

Expected Sequence 1.6B (SEND SS, call forward unconditional, all bearers, successful, null data alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 1.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 1.6.1 |  |
| 4 | ME | Should not give any information to the user on the fact that the ME is sending an SS request |  |
| 5 | ME USS | REGISTER 1.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.1.1B |  |

PROACTIVE COMMAND: SEND SS 1.6.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	null data object

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 89 | 10 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 |
|  | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |  |  |  |  |

27.22.4.11.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1 to 1.6.

##### 27.22.4.11.2	SEND SS (Icon support)

27.22.4.11.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.2.2	Conformance requirement

27.22.4.11.2.3	Test purpose

To verify that the ME displays the text contained in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

In addition to verify that if an icon is provided by the UICC, the icon indicated in the command may be used by the ME to inform the user, in addition to, or instead of the alpha identifier, as indicated with the icon qualifier.

27.22.4.11.2.4	Method of test

27.22.4.11.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and to the USS. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

The elementary files are coded as Toolkit default.

27.22.4.11.2.4.2	Procedure

Expected Sequence 2.1A (SEND SS, call forward unconditional, all bearers, successful, basic icon self explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.1.1 | [BASIC-ICON, self-explanatory] |
| 4 | ME USER | Display the basic icon without the alpha identifier |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 2.1.1AA or TERMINAL RESPONSE: SEND SS 2.1.1AB | [Command performed successfully] Option AA applies if A.1/63 is supported, Option AB applies if A.1/63 is not supported |

PROACTIVE COMMAND: SEND SS 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Basic Icon"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Icon Identifier:

Icon qualifier:	icon is self-explanatory

Icon Identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | 89 |
|  | 10 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 | 21 | 43 |
|  | 65 | 87 | A9 | 01 | FB | 9E | 02 | 00 | 01 |  |  |  |

TERMINAL RESPONSE: SEND SS 2.1.1AA

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information:	Operation Code and SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 1E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0A | A0 | 1A | 04 | 01 | 21 | 30 | 15 | 30 | 13 |
|  | 83 | 01 | 00 | 84 | 01 | 07 | 89 | 0B | 91 | 10 | 32 |
|  | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 |  |  |  |

TERMINAL RESPONSE: SEND SS 2.1.1AB

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Additional information:	Operation Code and  SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 0A | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 |
|  | 83 | 01 | 00 | 84 | 01 | 07 |  |  |  |  |  |

Expected Sequence 2.1B (SEND SS, call forward unconditional, all bearers, successful, basic icon self explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.1.1 | [BASIC-ICON, self-explanatory] |
| 4 | ME USER | Display "Basic Icon" without the icon |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 2.1.1BA or TERMINAL RESPONSE: SEND SS 2.1.1BB | [Command performed successfully, but requested icon could not be displayed] Option BA applies if A.1/63 is supported, Option BB applies if A.1/63 is not supported |

TERMINAL RESPONSE: SEND SS 2.1.1BA

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Additional information:	Operation Code and SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 1E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 0A | A0 | 1A | 04 | 01 | 21 | 30 | 15 | 30 | 13 |
|  | 83 | 01 | 00 | 84 | 01 | 07 | 89 | 0B | 91 | 10 | 32 |
|  | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 |  |  |  |

TERMINAL RESPONSE: SEND SS 2.1.1BB

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Additional information:	Operation Code and  SS Parameters

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 03 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 0A | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 |
|  | 83 | 01 | 00 | 84 | 01 | 07 |  |  |  |  |  |

Expected Sequence 2.2A (SEND SS, call forward unconditional, all bearers, successful, colour icon self explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.2.1 | [COLOUR-ICON, self-explanatory] |
| 4 | ME USER | Display the colour icon without thealpha identifier |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 2.1.1AA or TERMINAL RESPONSE: SEND SS 2.1.1AB | [Command performed successfully] Option AA applies if A.1/63 is supported, Option AB applies if A.1/63 is not supported |

PROACTIVE COMMAND: SEND SS 2.2.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Colour Icon"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Icon Identifier:

Icon qualifier:	icon is self-explanatory

Icon Identifier:	record 2 in EF(IMG)

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 43 | 6F | 6C | 6F | 75 | 72 | 20 | 49 | 63 | 6F | 6E |
|  | 89 | 10 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 | 21 |
|  | 43 | 65 | 87 | A9 | 01 | FB | 9E | 02 | 00 | 02 |  |  |

Expected Sequence 2.2B (SEND SS, call forward unconditional, all bearers, successful, colour icon self explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.2.1 | [COLOUR-ICON, self-explanatory] |
| 4 | ME USER | Display "Colour Icon" without the icon |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 2.1.1BA or TERMINAL RESPONSE: SEND SS 2.1.1BB | [Command performed but requested icon could not be displayed] Option BA applies if A.1/63 is supported, Option BB applies if A.1/63 is not supported |

Expected Sequence 2.3A (SEND SS, call forward unconditional, all bearers, successful, basic icon non self-explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.3.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME USER | Display "Basic Icon" and the basic icon |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 2.1.1AA or TERMINAL RESPONSE: SEND SS 2.1.1AB | [Command performed successfully] Option AA applies if A.1/63 is supported, Option AB applies if A.1/63 is not supported |

PROACTIVE COMMAND: SEND SS 2.3.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Text:	"Basic Icon"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Icon Identifier

Icon qualifier:	icon is non self-explanatory

Icon Identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | 89 |
|  | 10 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 | 21 | 43 |
|  | 65 | 87 | A9 | 01 | FB | 9E | 02 | 01 | 01 |  |  |  |

Expected Sequence 2.3B (SEND SS, call forward unconditional, all bearers, successful, basic icon non self-explanatory)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.3.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME USER | Display "Basic Icon" without the icon |  |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 2.1.1BA or TERMINAL RESPONSE: SEND SS 2.1.1BB | [Command performed but requested icon could not be displayed] Option BA applies if A.1/63 is supported, Option BB applies if A.1/63 is not supported |

Expected Sequence 2.4 (SEND SS, call forward unconditional, all bearers, successful, basic icon non self-explanatory, no alpha identifier presented)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 2.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 2.4.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME UICC | TERMINAL RESPONSE: SEND SS 2.4.1 | [Command data not understood by ME] |

PROACTIVE COMMAND: SEND SS 2.4.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789#"

Icon Identifier

Icon qualifier:	icon is non self-explanatory

Icon Identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 89 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 | 09 | 21 | 43 |
|  | 65 | 87 | B9 | 9E | 02 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: SEND SS 2.4.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command data not understood by ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.11.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1A to 2.4.

##### 27.22.4.11.3	SEND SS (UCS2 display in Cyrillic)

27.22.4.11.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.3.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5

Additionnally, the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in: ISO/IEC 10646 [17].

27.22.4.11.3.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.3.4	Method of test

27.22.4.11.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.3.4.2	Procedure

Expected Sequence 3.1 (SEND SS, call forward unconditional, all bearers, successful, UCS2 text in Cyrillic)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 3.1.1 |  |
| 4 | ME USER | Display "ЗДРАВСТВУЙТЕ" | ["Hello" in Russian] |
| 5 | ME USS | REGISTER 1.1A Or REGISTER 1.1B | Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1A or RELEASE COMPLETE (SS RETURN RESULT) 1.1B | [Successful] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 1.1.1A or TERMINAL RESPONSE: SEND SS 1.1.1B | [Command performed successfully] Option A applies if A.1/63 is supported, Option B applies if A.1/63 is not supported |

PROACTIVE COMMAND: SEND SS 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Data coding scheme:	UCS2 (16bit)

Text:	"ЗДРАВСТВУЙТЕ"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 36 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19 | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 |
|  | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 |
|  | 04 | 15 | 89 | 10 | 91 | AA | 12 | 0A | 21 | 43 | 65 | 87 |
|  | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |  |  |  |

27.22.4.11.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.11.4	SEND SS (support of Text Attribute)

27.22.4.11.4.1	SEND SS (support of Text Attribute – Left Alignment)

27.22.4.11.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.1.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.1.3	Test purpose

To verify that the ME displays the alpha identifier according to the left alignment text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.1.4	Method of test

27.22.4.11.4.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.1.4.2	Procedure

Expected Sequence 4.1A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.1.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with left alignment] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.1.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.1.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/12, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.1B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.1.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with left alignment] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.1.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.1.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/12, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.1.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

REGISTER 4.1A

Same as cl 27.22.4.11.1.4.2 REGISTER 1.1A

REGISTER 4.1B

Same as cl 27.22.4.11.1.4.2 REGISTER 1.1B

RELEASE COMPLETE (SS RETURN RESULT) 4.1A

Same as cl 27.22.4.11.1.4.2 RELEASE COMPLETE (SS RETURN RESULT) 1.1A

RELEASE COMPLETE (SS RETURN RESULT) 4.1B

Same as cl 27.22.4.11.1.4.2 RELEASE COMPLETE (SS RETURN RESULT) 1.1B

TERMINAL RESPONSE: SEND SS 4.1.1A

Same as cl 27.22.4.11.1.4.2 TERMINAL RESPONSE: SEND SS 1.1.1A

TERMINAL RESPONSE: SEND SS 4.1.1B

Same as cl 27.22.4.11.1.4.2 TERMINAL RESPONSE: SEND SS 1.1.1B

27.22.4.11.4.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

27.22.4.11.4.2	SEND SS (support of Text Attribute – Center Alignment)

27.22.4.11.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.2.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.2.3	Test purpose

To verify that the ME displays the alpha identifier according to the center alignment text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.2.4	Method of test

27.22.4.11.4.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.2.4.2	Procedure

Expected Sequence 4.2A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.2.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with center alignment] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.2.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.2.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/12, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.2B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.2.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with center alignment] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.2.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.2.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/12, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:			"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 01 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.2.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.2.

27.22.4.11.4.3	SEND SS (support of Text Attribute – Right Alignment)

27.22.4.11.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.3.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.3.3	Test purpose

To verify that the ME displays the alpha identifier according to the right alignment text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.3.4	Method of test

27.22.4.11.4.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.3.4.2	Procedure

Expected Sequence 4.3A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.3.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with right alignment] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.3.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.3.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/12, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.3B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.3.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with right alignment] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.3.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.3.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/12, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:			"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 02 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.3.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.3.

27.22.4.11.4.4	SEND SS (support of Text Attribute – Large Font Size)

27.22.4.11.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.4.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.4.3	Test purpose

To verify that the ME displays the alpha identifier according to the large font size text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.4.4	Method of test

27.22.4.11.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.4.4.2	Procedure

Expected Sequence 4.4A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with large font size] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with normal font size] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with large font size] |
| 19 | ME USS | REGISTER 4.1A |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with normal font size] |
| 26 | ME USS | REGISTER 4.1A |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.4B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with large font size] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with normal font size] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with large font size] |
| 19 | ME USS | REGISTER 4.1B |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.4.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.4.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with normal font size] |
| 26 | ME USS | REGISTER 4.1B |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:			"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 04 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.4.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:			"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.4.3

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.4.

27.22.4.11.4.5	SEND SS (support of Text Attribute – Small Font Size)

27.22.4.11.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.5.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.5.3	Test purpose

To verify that the ME displays the alpha identifier according to the small font size text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.5.4	Method of test

27.22.4.11.4.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.5.4.2	Procedure

Expected Sequence 4.5A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with small font size] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with normal font size] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with small font size] |
| 19 | ME USS | REGISTER 4.1A |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with normal font size] |
| 26 | ME USS | REGISTER 4.1A |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.5B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with small font size] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with normal font size] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with small font size] |
| 19 | ME USS | REGISTER 4.1B |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.5.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.5.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with normal font size] |
| 26 | ME USS | REGISTER 4.1B |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 08 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.5.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.5.3

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.5.

27.22.4.11.4.6	SEND SS (support of Text Attribute – Bold On)

27.22.4.11.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.6.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.6.3	Test purpose

To verify that the ME displays the alpha identifier according to the bold text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.6.4	Method of test

27.22.4.11.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.6.4.2	Procedure

Expected Sequence 4.6A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with bold on] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with bold off] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with bold on] |
| 19 | ME USS | REGISTER 4.1A |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with bold off] |
| 26 | ME USS | REGISTER 4.1A |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.6B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with bold on] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with bold off] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with bold on] |
| 19 | ME USS | REGISTER 4.1B |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.6.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.6.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with bold off] |
| 26 | ME USS | REGISTER 4.1B |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 10 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.6.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.6.3

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.6.

27.22.4.11.4.7	SEND SS (support of Text Attribute – Italic On)

27.22.4.11.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.7.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.7.3	Test purpose

To verify that the ME displays the alpha identifier according to the italic text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.7.4	Method of test

27.22.4.11.4.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.7.4.2	Procedure

Expected Sequence 4.7A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with italic on] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with italic off] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with italic on] |
| 19 | ME USS | REGISTER 4.1A |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with italic off] |
| 26 | ME USS | REGISTER 4.1A |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.7B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with italic on] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with italic off] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with italic on] |
| 19 | ME USS | REGISTER 4.1B |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.7.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.7.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with italic off] |
| 26 | ME USS | REGISTER 4.1B |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 20 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.7.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.7.3

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.7.

27.22.4.11.4.8	SEND SS (support of Text Attribute – Underline On)

27.22.4.11.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.8.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.8.3	Test purpose

To verify that the ME displays the alpha identifier according to the underline text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.8.4	Method of test

27.22.4.11.4.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.8.4.2	Procedure

Expected Sequence 4.8A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with underline on] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with underline off] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with underline on] |
| 19 | ME USS | REGISTER 4.1A |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with underline off] |
| 26 | ME USS | REGISTER 4.1A |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.8B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with underline on] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with underline off] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with underline on] |
| 19 | ME USS | REGISTER 4.1B |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.8.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.8.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with underline off] |
| 26 | ME USS | REGISTER 4.1B |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 40 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.8.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.8.3

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.8.

27.22.4.11.4.9	SEND SS (support of Text Attribute – Strikethrough On)

27.22.4.11.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.9.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.9.3	Test purpose

To verify that the ME displays the alpha identifier according to the strikethrough text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.9.4	Method of test

27.22.4.11.4.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.9.4.2	Procedure

Expected Sequence 4.9A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with strikethrough on] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with strikethrough off] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with strikethrough on] |
| 19 | ME USS | REGISTER 4.1A |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with strikethrough off] |
| 26 | ME USS | REGISTER 4.1A |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.9B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with strikethrough on] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with strikethrough off] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with strikethrough on] |
| 19 | ME USS | REGISTER 4.1B |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.9.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND SS 4.9.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Message shall be formatted with strikethrough off] |
| 26 | ME USS | REGISTER 4.1B |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 80 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.9.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.9.3

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.9.

27.22.4.11.4.10	SEND SS (support of Text Attribute – Foreground and Background Colour)

27.22.4.11.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.4.10.2	Conformance requirement

The ME shall support the Proactive UICC: Send SS facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5.

27.22.4.11.4.10.3	Test purpose

To verify that the ME displays the alpha identifier according to the foreground and background colour text attribute configuration in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.4.10.4	Method of test

27.22.4.11.4.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.4.10.4.2	Procedure

Expected Sequence 4.10A (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.10.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with foreground and background colour according to text attribute configuration] |
| 5 | ME USS | REGISTER 4.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.10.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.10.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with ME's default foreground and background colour] |
| 12 | ME USS | REGISTER 4.1A |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1A | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1A |  |

Expected Sequence 4.10B (SEND SS, call forward unconditional, all bearers, successful, alpha identifier with Text attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 4.10.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with foreground and background colour according to text attribute configuration] |
| 5 | ME USS | REGISTER 4.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 4.10.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND SS 4.10.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with ME's default foreground and background colour] |
| 12 | ME USS | REGISTER 4.1B |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1B | [Successful] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND SS 4.1.1B |  |

PROACTIVE COMMAND: SEND SS 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB | D0 |
|  | 04 | 00 | 10 | 00 | B4 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND SS 4.10.2

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 89 | 10 | 91 | AA | 12 | 0A | 21 |
|  | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |

27.22.4.11.4.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.10.

##### 27.22.4.11.5	SEND SS (UCS2 display in Chinese)

27.22.4.11.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.5.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5

Additionnally, the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in: ISO/IEC 10646 [17].

27.22.4.11.5.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.5.4	Method of test

27.22.4.11.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.5.4.2	Procedure

Expected Sequence 5.1A (SEND SS, call forward unconditional, all bearers, successful, UCS2 text in Chinese)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 5.1.1 |  |
| 4 | ME USER | Display "你好" | ["Hello" in Chinese] |
| 5 | ME USS | REGISTER 5.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 5.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 5.1.1A | [Command performed successfully] |

Expected Sequence 5.1B (SEND SS, call forward unconditional, all bearers, successful, UCS2 text in Chinese)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 5.1.1 |  |
| 4 | ME USER | Display "你好" | ["Hello" in Chinese] |
| 5 | ME USS | REGISTER 5.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 5.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 5.1.1B | [Command performed successfully] |

PROACTIVE COMMAND: SEND SS 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Data coding scheme:	UCS2 (16bit)

Text:	"你好"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 4F | 60 | 59 | 7D | 89 | 10 | 91 | AA | 12 | 0A |
|  | 21 | 43 | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |

REGISTER 5.1A

Same as cl 27.22.4.11.1.4.2 REGISTER 1.1A

REGISTER 5.1B

Same as cl 27.22.4.11.1.4.2 REGISTER 1.1B

RELEASE COMPLETE (SS RETURN RESULT) 5.1A

Same as cl 27.22.4.11.1.4.2 RELEASE COMPLETE (SS RETURN RESULT) 1.1A

RELEASE COMPLETE (SS RETURN RESULT) 5.1B

Same as cl 27.22.4.11.1.4.2 RELEASE COMPLETE (SS RETURN RESULT) 1.1B

TERMINAL RESPONSE: SEND SS 5.1.1A

Same as cl 27.22.4.11.1.4.2 TERMINAL RESPONSE: SEND SS 1.1.1A

TERMINAL RESPONSE: SEND SS 5.1.1B

Same as cl 27.22.4.11.1.4.2 TERMINAL RESPONSE: SEND SS 1.1.1B

27.22.4.11.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.11.6	SEND SS (UCS2 display in Katakana)

27.22.4.11.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.11.6.2	Conformance requirement

The ME shall support the Proactive UICC: SEND SHORT MESSAGE facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.11, clause 6.6.10, clause 8.12.1 , clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.14, clause 8.31 and clause 6.5

Additionnally, the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in: ISO/IEC 10646 [17].

27.22.4.11.6.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND SS proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.11.6.4	Method of test

27.22.4.11.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.11.6.4.2	Procedure

Expected Sequence 6.1A (SEND SS, call forward unconditional, all bearers, successful, UCS2 text in Katakana)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 6.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 6.1.1 |  |
| 4 | ME USER | Display "ル" | [Character in Katakana] |
| 5 | ME USS | REGISTER 6.1A |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 6.1A | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 6.1.1A | [Command performed successfully] |

Expected Sequence 6.1B (SEND SS, call forward unconditional, all bearers, successful, UCS2 text in Katakana)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SS 6.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SS 6.1.1 |  |
| 4 | ME USER | Display "ル" | [Character in Katakana] |
| 5 | ME USS | REGISTER 6.1B |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 6.1B | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SS 6.1.1B | [Command performed successfully] |

PROACTIVE COMMAND: SEND SS 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SS

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Data coding scheme:	UCS2 (16bit)

Text:	"ル"

SS String

TON:	International

NPI:	"ISDN / telephone numbering plan"

SS string:	"**21*01234567890123456789*10#"

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 11 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 | 80 | 30 | EB | 89 | 10 | 91 | AA | 12 | 0A | 21 | 43 |
|  | 65 | 87 | 09 | 21 | 43 | 65 | 87 | A9 | 01 | FB |  |  |

REGISTER 6.1A

Same as cl 27.22.4.11.1.4.2 REGISTER 1.1A

REGISTER 6.1B

Same as cl 27.22.4.11.1.4.2 REGISTER 1.1B

RELEASE COMPLETE (SS RETURN RESULT) 6.1A

Same as cl 27.22.4.11.1.4.2 RELEASE COMPLETE (SS RETURN RESULT) 1.1A

RELEASE COMPLETE (SS RETURN RESULT) 6.1B

Same as cl 27.22.4.11.1.4.2 RELEASE COMPLETE (SS RETURN RESULT) 1.1B

TERMINAL RESPONSE: SEND SS 6.1.1A

Same as cl 27.22.4.11.1.4.2 TERMINAL RESPONSE: SEND SS 1.1.1A

TERMINAL RESPONSE: SEND SS 6.1.1B

Same as cl 27.22.4.11.1.4.2 TERMINAL RESPONSE: SEND SS 1.1.1B

27.22.4.11.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

#### 27.22.4.12	SEND USSD

##### 27.22.4.12.1	SEND USSD (normal)

27.22.4.12.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.1.2	Conformance requirement

The ME shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

-	TS 23.038 [7] clause 5

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in: ISO/IEC 10646 [17].

27.22.4.12.1.3	Test purpose

To verify that the ME correctly translates and sends the unstructured supplementary service request indicated in the SEND USSD proactive UICC command to the USS.

To verify that the ME returns a TERMINAL RESPONSE command to the UICC indicating the status of the transmission of the USSD request and including a USSD result as a text string in the TERMINAL RESPONSE.

27.22.4.12.1.4	Method of test

27.22.4.12.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.1.4.2	Procedure

Expected Sequence 1.1 (SEND USSD, 7-bit data, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.1.1 |  |
| 4 | ME USER | Display "7-bit USSD" |  |
| 5 | ME USS | REGISTER 1.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.1.1 |  |

PROACTIVE COMMAND: SEND USSD 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"7-bit USSD"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 50 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 37 | 2D | 62 | 69 | 74 | 20 | 55 | 53 | 53 | 44 | 8A |
|  | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 | E5 | 92 |
|  | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E | B1 | 59 |
|  | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD | 5E | B3 |
|  | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B | 3E | AF |
|  | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 |  |  |

REGISTER 1.1

Logically (only USSD argument)

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD string:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV | 30 | 3D | 04 | 01 | F0 | 04 | 38 | 41 | E1 | 90 | 58 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 1.1

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD string:

- "USSD string received from SS"

Coding:

| BER-TLV | 30 | 1E | 04 | 01 | F0 | 04 | 19 | D5 | E9 | 94 | 08 | 9A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | E5 | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 |
|  | 0C | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

Expected Sequence 1.2 (SEND USSD, 8-bit data, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.2.1 |  |
| 4 | ME USER | Display "8-bit USSD" |  |
| 5 | ME USS | REGISTER 1.2 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.2 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.2.1 |  |

PROACTIVE COMMAND: SEND USSD 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"8-bit USSD"

USSD String

Data coding scheme:	Uncompressed, no message class meaning, 8-bit data

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 58 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 38 | 2D | 62 | 69 | 74 | 20 | 55 | 53 | 53 | 44 | 8A |
|  | 41 | 44 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 4A |
|  | 4B | 4C | 4D | 4E | 4F | 50 | 51 | 52 | 53 | 54 | 55 | 56 |
|  | 57 | 58 | 59 | 5A | 2D | 61 | 62 | 63 | 64 | 65 | 66 | 67 |
|  | 68 | 69 | 6A | 6B | 6C | 6D | 6E | 6F | 70 | 71 | 72 | 73 |
|  | 74 | 75 | 76 | 77 | 78 | 79 | 7A | 2D | 31 | 32 | 33 | 34 |
|  | 35 | 36 | 37 | 38 | 39 | 30 |  |  |  |  |  |  |

REGISTER 1.2

Logically (only USSD argument):

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- Uncompressed, no message class meaning, 8-bit data

USSD string:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV | 30 | 45 | 04 | 01 | 44 | 04 | 40 | 41 | 42 | 43 | 44 | 45 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46 | 47 | 48 | 49 | 4A | 4B | 4C | 4D | 4E | 4F | 50 | 51 |
|  | 52 | 53 | 54 | 55 | 56 | 57 | 58 | 59 | 5A | 2D | 61 | 62 |
|  | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 6A | 6B | 6C | 6D | 6E |
|  | 6F | 70 | 71 | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 7A |
|  | 2D | 31 | 32 | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 30 |  |

RELEASE COMPLETE (SS RETURN RESULT) 1.2

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- Uncompressed, no message class meaning, 8-bit data

USSD string:

- "USSD string received from SS"

Coding:

| BER-TLV | 30 | 21 | 04 | 01 | 44 | 04 | 1C | 55 | 53 | 53 | 44 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 73 | 74 | 72 | 69 | 6E | 67 | 20 | 72 | 65 | 63 | 65 | 69 |
|  | 76 | 65 | 64 | 20 | 66 | 72 | 6F | 6D | 20 | 53 | 53 |  |

TERMINAL RESPONSE: SEND USSD 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	Uncompressed, no message class meaning, 8-bit data

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1D | 04 | 55 | 53 | 53 | 44 | 20 | 73 | 74 |
|  | 72 | 69 | 6E | 67 | 20 | 72 | 65 | 63 | 65 | 69 | 76 |
|  | 65 | 64 | 20 | 66 | 72 | 6F | 6D | 20 | 53 | 53 |  |

Expected Sequence 1.3 (SEND USSD, UCS2 data, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.3.1 |  |
| 4 | ME USER | Display "UCS2 USSD" |  |
| 5 | ME USS | REGISTER 1.3 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.3 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.3.1 |  |

PROACTIVE COMMAND: SEND USSD 1.3.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"UCS2 USSD"

USSD String

Data coding scheme:	Uncompressed, no message class meaning, UCS2 (16 bit)

USSD string:	"ЗДРАВСТВУЙТЕ" ("Hello" in Russian)

Coding:

| BER-TLV: | D0 | 2F | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 55 | 43 | 53 | 32 | 20 | 55 | 53 | 53 | 44 | 8A | 19 |
|  | 48 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 | 04 |
|  | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 | 04 |
|  | 15 |  |  |  |  |  |  |  |  |  |  |  |

REGISTER 1.3

Logically (only USSD argument):

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- Uncompressed, no message class meaning, UCS2 (16 bit)

USSD string:

- "ЗДРАВСТВУЙТЕ" ("Hello" in Russian)

Coding:

| BER-TLV | 30 | 1D | 04 | 01 | 48 | 04 | 18 | 04 | 17 | 04 | 14 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20 | 04 | 10 | 04 | 12 | 04 | 21 | 04 | 22 | 04 | 12 | 04 |
|  | 23 | 04 | 19 | 04 | 22 | 04 | 15 |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 1.3

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- Uncompressed, no message class meaning, UCS2 (16 bit)

USSD string:

- "USSD string received from SS"

Coding:

| BER-TLV | 30 | 3D | 04 | 01 | 48 | 04 | 38 | 00 | 55 | 00 | 53 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 53 | 00 | 44 | 00 | 20 | 00 | 73 | 00 | 74 | 00 | 72 | 00 |
|  | 69 | 00 | 6E | 00 | 67 | 00 | 20 | 00 | 72 | 00 | 65 | 00 |
|  | 63 | 00 | 65 | 00 | 69 | 00 | 76 | 00 | 65 | 00 | 64 | 00 |
|  | 20 | 00 | 66 | 00 | 72 | 00 | 6F | 00 | 6D | 00 | 20 | 00 |
|  | 53 | 00 | 53 |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 1.3.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	Uncompressed, no message class meaning, UCS2 (16 bit)

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 39 | 08 | 00 | 55 | 00 | 53 | 00 | 53 | 00 |
|  | 44 | 00 | 20 | 00 | 73 | 00 | 74 | 00 | 72 | 00 | 69 |
|  | 00 | 6E | 00 | 67 | 00 | 20 | 00 | 72 | 00 | 65 | 00 |
|  | 63 | 00 | 65 | 00 | 69 | 00 | 76 | 00 | 65 | 00 | 64 |
|  | 00 | 20 | 00 | 66 | 00 | 72 | 00 | 6F | 00 | 6D | 00 |
|  | 20 | 00 | 53 | 00 | 53 |  |  |  |  |  |  |

Expected Sequence 1.4 (SEND USSD, 7-bit data, unsuccessful (Return Error))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.1.1 |  |
| 4 | ME USER | Display "7-bit USSD" |  |
| 5 | ME USS | REGISTER 1.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN ERROR) 1.1 | Return Error |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.4.1 |  |

RELEASE COMPLETE (SS RETURN ERROR) 1.1

Logically (only from Return Error code):

ProcessUnstructuredSS-Request RETURN ERROR

Return Error code:

- Unknown alphabet

Coding:

| Coding | 02 | 01 | 47 |
| --- | --- | --- | --- |

TERMINAL RESPONSE: SEND USSD 1.4.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	USSD Return Error

Additional information:	"Unknown alphabet"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 37 | 47 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.5 (SEND USSD, 7-bit data, unsuccessful (Reject))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.1.1 |  |
| 4 | ME USER | Display "7-bit USSD" |  |
| 5 | ME USS | REGISTER 1.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS REJECT) 1.1 | Reject |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.5.1 |  |

RELEASE COMPLETE (SS REJECT) 1.1

Logically (only from Problem code):

ProcessUnstructuredSS-Request REJECT

Invoke Problem code:

- Mistyped parameter

Coding:

| Coding | 81 | 01 | 02 |
| --- | --- | --- | --- |

TERMINAL RESPONSE: SEND USSD 1.5.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	USSD Return Error

Additional information:	"No specific cause can be given"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 37 | 00 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.6 (SEND USSD, 256 octets, 7-bit data, successful, long alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.6.1 |  |
| 4 | ME USER | Display "once a RELEASE COMPLETE message containing the USSD Return Result message not containing an error has been received from the network, the ME shall inform the SIM that the command has" |  |
| 5 | ME USS | REGISTER 1.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.1.1 |  |

PROACTIVE COMMAND: SEND USSD 1.6.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"once a RELEASE COMPLETE message containing the USSD Return Result message not containing an error has been received from the network, the ME shall inform the SIM that the command has"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 81 | FD | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 81 | B6 | 6F | 6E | 63 | 65 | 20 | 61 | 20 | 52 | 45 |
|  | 4C | 45 | 41 | 53 | 45 | 20 | 43 | 4F | 4D | 50 | 4C | 45 |
|  | 54 | 45 | 20 | 6D | 65 | 73 | 73 | 61 | 67 | 65 | 20 | 63 |
|  | 6F | 6E | 74 | 61 | 69 | 6E | 69 | 6E | 67 | 20 | 74 | 68 |
|  | 65 | 20 | 55 | 53 | 53 | 44 | 20 | 52 | 65 | 74 | 75 | 72 |
|  | 6E | 20 | 52 | 65 | 73 | 75 | 6C | 74 | 20 | 6D | 65 | 73 |
|  | 73 | 61 | 67 | 65 | 20 | 6E | 6F | 74 | 20 | 63 | 6F | 6E |
|  | 74 | 61 | 69 | 6E | 69 | 6E | 67 | 20 | 61 | 6E | 20 | 65 |
|  | 72 | 72 | 6F | 72 | 20 | 68 | 61 | 73 | 20 | 62 | 65 | 65 |
|  | 6E | 20 | 72 | 65 | 63 | 65 | 69 | 76 | 65 | 64 | 20 | 66 |
|  | 72 | 6F | 6D | 20 | 74 | 68 | 65 | 20 | 6E | 65 | 74 | 77 |
|  | 6F | 72 | 6B | 2C | 20 | 74 | 68 | 65 | 20 | 4D | 45 | 20 |
|  | 73 | 68 | 61 | 6C | 6C | 20 | 69 | 6E | 66 | 6F | 72 | 6D |
|  | 20 | 74 | 68 | 65 | 20 | 53 | 49 | 4D | 20 | 74 | 68 | 61 |
|  | 74 | 20 | 74 | 68 | 65 | 20 | 63 | 6F | 6D | 6D | 61 | 6E |
|  | 64 | 20 | 68 | 61 | 73 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

Expected Sequence 1.7 (SEND USSD, 7-bit data, successful, no alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.7.1 |  |
| 4 | ME USER | Optionally display an informative message |  |
| 5 | ME USS | REGISTER 1.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.1.1 |  |

PROACTIVE COMMAND: SEND USSD 1.7.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 44 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 8A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 | E5 | 92 |
|  | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E | B1 | 59 |
|  | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD | 5E | B3 |
|  | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B | 3E | AF |
|  | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 |  |  |

Expected Sequence 1.8 (SEND USSD, 7-bit data, successful, null length alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 1.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 1.8.1 |  |
| 4 | ME USER | the ME should not give any information to the user on the fact that the ME is sending a USSD request |  |
| 5 | ME USS | REGISTER 1.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 1.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 1.1.1 |  |

PROACTIVE COMMAND: SEND USSD 1.8.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	""

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 46 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8A | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 |
|  | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E |
|  | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD |
|  | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B |
|  | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 |

27.22.4.12.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 - 1.8.

##### 27.22.4.12.2	SEND USSD (Icon support)

27.22.4.12.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.2.2	Conformance requirement

27.22.4.12.2.3	Test purpose

To verify that the ME displays the text contained in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

In addition to verify that if an icon is provided by the UICC, the icon indicated in the command may be used by the ME to inform the user, in addition to, or instead of the alpha identifier, as indicated with the icon qualifier.

27.22.4.12.2.4	Method of test

27.22.4.12.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and to the USS. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS

The elementary files are coded as Toolkit default.

27.22.4.12.2.4.2	Procedure

Expected Sequence 2.1A (SEND USSD, 7-bit data, successful, basic icon self explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 2.1.1 | [BASIC-ICON, self-explanatory] |
| 4 | ME USER | Display BASIC ICON |  |
| 5 | ME USS | REGISTER 2.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 2.1.1A | [Command performed successfully] |

PROACTIVE COMMAND: SEND USSD 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Basic Icon"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Icon Identifier:

Icon qualifier:	icon is self-explanatory

Icon Identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 54 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | 8A |
|  | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 | E5 | 92 |
|  | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E | B1 | 59 |
|  | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD | 5E | B3 |
|  | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B | 3E | AF |
|  | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 | 9E | 02 |
|  | 00 | 01 |  |  |  |  |  |  |  |  |  |  |

REGISTER 2.1

Logically (only USSD argument)

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD string:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV | 30 | 3D | 04 | 01 | F0 | 04 | 38 | 41 | E1 | 90 | 58 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 2.1

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD string:

- "USSD string received from SS"

Coding:

| BER-TLV | 30 | 1E | 04 | 01 | F0 | 04 | 19 | D5 | E9 | 94 | 08 | 9A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | E5 | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 |
|  | 0C | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 2.1.1A

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

Expected Sequence 2.1B (SEND USSD, 7-bit data, successful, basic icon self explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 2.1.1 | [BASIC-ICON, self-explanatory] |
| 4 | ME USER | Display "Basic Icon" without the icon |  |
| 5 | ME USS | REGISTER 2.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 2.1.1B | [Command performed but requested icon could not be displayed] |

TERMINAL RESPONSE: SEND USSD 2.1.1B

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

Expected Sequence 2.2 (SEND USSD, 7-bit data, successful, colour icon self explanatory)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 2.2.1 | [COLOUR-ICON, self-explanatory] |
| 4 | ME USER | Display COLOUR-ICON or May give information to user concerning what is happening |  |
| 5 | ME USS | REGISTER 2.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 2.1.1A or TERMINAL RESPONSE: SEND USSD 2.1.1B | [Command performed successfully] or [Command performed but requested icon could not be displayed] |

PROACTIVE COMMAND: SEND USSD 2.2.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Color Icon"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Icon Identifier:

Icon qualifier:	icon is self-explanatory

Icon Identifier:	record 2 in EF(IMG)

Coding:

| BER-TLV: | D0 | 54 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 43 | 6F | 6C | 6F | 72 | 20 | 49 | 63 | 6F | 6E | 8A |
|  | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 | E5 | 92 |
|  | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E | B1 | 59 |
|  | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD | 5E | B3 |
|  | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B | 3E | AF |
|  | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 | 9E | 02 |
|  | 00 | 02 |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.3A (SEND USSD, 7-bit data, successful, basic icon non self-explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 2.3.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME USER | Display "Basic Icon" and BASIC-ICON |  |
| 5 | ME USS | REGISTER 2.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 2.1.1A | [Command performed successfully] |

PROACTIVE COMMAND: SEND USSD 2.3.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Basic Icon"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Icon Identifier

Icon qualifier:	icon is non self-explanatory

Icon Identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 54 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | 8A |
|  | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 | E5 | 92 |
|  | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E | B1 | 59 |
|  | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD | 5E | B3 |
|  | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B | 3E | AF |
|  | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 | 9E | 02 |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.3B (SEND USSD, 7-bit data, successful, basic icon non self-explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 2.3.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME USER | Display "Basic Icon" without the icon |  |
| 5 | ME USS | REGISTER 2.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 2.1.1B | [Command performed but requested icon could not be displayed] |

Expected Sequence 2.4 (SEND USSD, 7-bit data, basic icon non self-explanatory, no alpha identifier presented)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 2.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 2.4.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME UICC | TERMINAL RESPONSE: SEND USSD 2.4.1 | [Command data not understood by ME] |

PROACTIVE COMMAND: SEND USSD 2.4.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-

Icon Identifier

Icon qualifier:	icon is non self-explanatory

Icon Identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 48 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 8A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 | 49 | E5 | 92 |
|  | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 | 5E | B1 | 59 |
|  | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A | AD | 5E | B3 |
|  | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 | 3B | 3E | AF |
|  | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 | 60 | 9E | 02 |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 2.4.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command data not understood by ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.12.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1 - 2.4.

##### 27.22.4.12.3	SEND USSD (UCS2 display in Cyrillic)

27.22.4.12.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.3.2	Conformance requirement

The ME shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

-	TS 23.038 [7] clause 5

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in: ISO/IEC 10646 [17].

27.22.4.12.3.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.3.4	Method of test

27.22.4.12.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.3.4.2	Procedure

Expected Sequence 3.1 (SEND USSD, 7-bit data, successful, UCS2 text in Cyrillic)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 3.1.1 |  |
| 4 | ME USER | Display "ЗДРАВСТВУЙТЕ" | ["Hello" in Russian] |
| 5 | ME USS | REGISTER 3.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 3.1 | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 3.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND USSD 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Data coding scheme:	UCS2 (16bit)

Text:	"ЗДРАВСТВУЙТЕ"

USSD String

Data coding scheme:	7-bit default, no message class

USSD String:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-

Coding:

| BER-TLV: | D0 | 5F | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19 | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 |
|  | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 |
|  | 04 | 15 | 8A | 39 | F0 | 41 | E1 | 90 | 58 | 34 | 1E | 91 |
|  | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 | 5A | B5 |
|  | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 | 33 | 3A |
|  | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 | EB | F6 |
|  | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 | C3 | E5 |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |

REGISTER 3.1

Logically (only USSD argument)

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD String:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV | 30 | 3D | 04 | 01 | F0 | 04 | 38 | 41 | E1 | 90 | 58 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 3.1

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD String:

- "USSD string received from SS"

Coding:

| BER-TLV | 30 | 1E | 04 | 01 | F0 | 04 | 19 | D5 | E9 | 94 | 08 | 9A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | E5 | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 |
|  | 0C | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.12.4	SEND USSD (support of Text Attribute)

27.22.4.12.4.1	SEND USSD (support of Text Attribute – Left Alignment)

27.22.4.12.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.1.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.1.3	Test purpose

To verify that the ME displays the alpha identifier according to the left alignment text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.1.4	Method of test

27.22.4.12.4.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.1.4.2	Procedure

Expected Sequence 4.1 (SEND USSD, 7-bit data, successful, with Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.1.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with left alignment] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.1.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.1.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.1.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/13, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.1.1 |  |

PROACTIVE COMMAND: SEND USSD 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.1.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

REGISTER 4.1

Logically (only USSD argument)

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD string:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| Coding | 30 | 3D | 04 | 01 | F0 | 04 | 40 | 41 | E1 | 90 | 58 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 4.1

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD string:

- "USSD string received from SS"

Coding:

| BER-TLV | 30 | 1E | 04 | 01 | F0 | 04 | 19 | D5 | E9 | 94 | 08 | 9A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | E5 | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 |
|  | 0C | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

27.22.4.12.4.2	SEND USSD (support of Text Attribute – Center Alignment)

27.22.4.12.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.2.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.2.3	Test purpose

To verify that the ME displays the alpha identifier according to the center alignment text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.2.4	Method of test

27.22.4.12.4.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.2.4.2	Procedure

Expected Sequence 4.2 (SEND USSD, 7-bit data, successful, with Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.2.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with center alignment] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.2.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.2.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.2.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/13, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.2.1 |  |

PROACTIVE COMMAND: SEND USSD 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 01 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.2.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.2.

27.22.4.12.4.3	SEND USSD (support of Text Attribute – Right Alignment)

27.22.4.12.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.3.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.3.3	Test purpose

To verify that the ME displays the alpha identifier according to the right alignment text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.3.4	Method of test

27.22.4.12.4.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.3.4.2	Procedure

Expected Sequence 4.3 (SEND USSD, 7-bit data, successful, with Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.3.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with right alignment] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.3.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.3.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.3.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/13, no alignment change will take place] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.3.1 |  |

PROACTIVE COMMAND: SEND USSD 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 02 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.3.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.3.

27.22.4.12.4.4	SEND USSD (support of Text Attribute – Large Font Size)

27.22.4.12.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.4.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.4.3	Test purpose

To verify that the ME displays the alpha identifier according to the large font size text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.4.4	Method of test

27.22.4.12.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.4.4.2	Procedure

Expected Sequence 4.4 (SEND USSD, 7-bit data, successful, with Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.4.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with large font size] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.4.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.4.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.4.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed with normal font size] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.4.1 |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.4.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.4.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with large font size] |
| 19 | ME USS | REGISTER 4.1 |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.4.1 |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.4.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.4.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Alpha identifier is displayed with normal font size] |
| 26 | ME USS | REGISTER 4.1 |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.4.1 |  |

PROACTIVE COMMAND: SEND USSD 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 04 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.4.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.4.3

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.4.

27.22.4.12.4.5	SEND USSD (support of Text Attribute – Small Font Size)

27.22.4.12.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.5.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.5.3	Test purpose

To verify that the ME displays the alpha identifier according to the small font size text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.5.4	Method of test

27.22.4.12.4.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.5.4.2	Procedure

Expected Sequence 4.5 (SEND USSD, 7-bit data, successful, with Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.5.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with small font size] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.5.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.5.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.5.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed with normal font size] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.5.1 |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.5.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.5.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with small font size] |
| 19 | ME USS | REGISTER 4.1 |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.5.1 |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.5.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.5.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Alpha identifier is displayed with normal font size] |
| 26 | ME USS | REGISTER 4.1 |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.5.1 |  |

PROACTIVE COMMAND: SEND USSD 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 08 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.5.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.5.3

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.5.

27.22.4.12.4.6	SEND USSD (support of Text Attribute – Bold On)

27.22.4.12.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.6.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.6.3	Test purpose

To verify that the ME displays the alpha identifier according to the bold text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.6.4	Method of test

27.22.4.12.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.6.4.2	Procedure

Expected Sequence 4.6 (SEND USSD, 7-bit data, successful, with Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.6.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with bold on] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.6.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.6.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.6.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed with bold off] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.6.1 |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.6.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.6.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with bold on] |
| 19 | ME USS | REGISTER 4.1 |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.6.1 |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.6.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.6.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Alpha identifier is displayed with bold off] |
| 26 | ME USS | REGISTER 4.1 |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.6.1 |  |

PROACTIVE COMMAND: SEND USSD 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 10 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.6.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.6.3

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.6.

27.22.4.12.4.7	SEND USSD (support of Text Attribute – Italic On)

27.22.4.12.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.7.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.7.3	Test purpose

To verify that the ME displays the alpha identifier according to the italic text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.7.4	Method of test

27.22.4.12.4.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.7.4.2	Procedure

Expected Sequence 4.7 (SEND USSD, 7-bit data, successful, with Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.7.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with italic on] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.7.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.7.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.7.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed with italic off] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.7.1 |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.7.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.7.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with italic on] |
| 19 | ME USS | REGISTER 4.1 |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.7.1 |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.7.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.7.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Alpha identifier is displayed with italic off] |
| 26 | ME USS | REGISTER 4.1 |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.7.1 |  |

PROACTIVE COMMAND: SEND USSD 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 20 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.7.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.7.3

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.7.

27.22.4.12.4.8	SEND USSD (support of Text Attribute – Underline On)

27.22.4.12.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.8.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.8.3	Test purpose

To verify that the ME displays the alpha identifier according to the underline text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.8.4	Method of test

27.22.4.12.4.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.8.4.2	Procedure

Expected Sequence 4.8 (SEND USSD, 7-bit data, successful, with Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.8.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with underline on] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.8.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.8.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.8.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed with underline off] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.8.1 |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.8.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.8.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with underline on] |
| 19 | ME USS | REGISTER 4.1 |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.8.1 |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.8.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.8.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Alpha identifier is displayed with underline off] |
| 26 | ME USS | REGISTER 4.1 |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.8.1 |  |

PROACTIVE COMMAND: SEND USSD 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 40 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.8.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.8.3

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.8.

27.22.4.12.4.9	SEND USSD (support of Text Attribute – Strikethrough On)

27.22.4.12.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.9.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.9.3	Test purpose

To verify that the ME displays the alpha identifier according to the strikethrough text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.9.4	Method of test

27.22.4.12.4.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.9.4.2	Procedure

Expected Sequence 4.9 (SEND USSD, 7-bit data, successful, with Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.9.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with strikethrough on] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.9.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.9.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.9.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Alpha identifier is displayed with strikethrough off] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.9.1 |  |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.9.1 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.9.1 |  |
| 18 | ME USER | Display "Text Attribute 1" | [Alpha identifier is displayed with strikethrough on] |
| 19 | ME USS | REGISTER 4.1 |  |
| 20 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 21 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.9.1 |  |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.9.3 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.9.3 |  |
| 25 | ME USER | Display "Text Attribute 3" | [Alpha identifier is displayed with strikethrough off] |
| 26 | ME USS | REGISTER 4.1 |  |
| 27 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 28 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.9.1 |  |

PROACTIVE COMMAND: SEND USSD 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 80 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.9.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.9.3

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 3"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 33 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.9.

27.22.4.12.4.10	SEND USSD (support of Text Attribute – Foreground and Background Colour)

27.22.4.12.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.4.10.2	Conformance requirement

The terminal shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

27.22.4.12.4.10.3	Test purpose

To verify that the ME displays the alpha identifier according to the foreground and background colour text attribute configuration in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.4.10.4	Method of test

27.22.4.12.4.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.

The elementary files are coded as UICC default. Prior to this test the terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.4.10.4.2	Procedure

Expected Sequence 4.10 (SEND USSD, 7-bit data, successful, with Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.10.1 |  |
| 4 | ME USER | Display "Text Attribute 1" | [Message shall be formatted with foreground and background colour according to text attribute configuration] |
| 5 | ME USS | REGISTER 4.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.10.1 |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 4.10.2 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND USSD 4.10.2 |  |
| 11 | ME USER | Display "Text Attribute 2" | [Message shall be formatted with ME's default foreground and background colour] |
| 12 | ME USS | REGISTER 4.1 |  |
| 13 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 4.1 | ["USSD string received from SS"] |
| 14 | ME UICC | TERMINAL RESPONSE: SEND USSD 4.10.1 |  |

PROACTIVE COMMAND: SEND USSD 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 1"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 5C | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 31 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 | D0 | 04 | 00 | 10 | 00 | B4 |  |  |

PROACTIVE COMMAND: SEND USSD 4.10.2

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Text Attribute 2"

USSD String

Data coding scheme:	7-bit default, no message class

USSD string:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 56 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 54 | 65 | 78 | 74 | 20 | 41 | 74 | 74 | 72 | 69 | 62 |
|  | 75 | 74 | 65 | 20 | 32 | 8A | 39 | F0 | 41 | E1 | 90 | 58 |
|  | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 |
|  | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB |
|  | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F |
|  | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD |
|  | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.4.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.10.

##### 27.22.4.12.5	SEND USSD (UCS2 display in Chinese)

27.22.4.12.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.5.2	Conformance requirement

The ME shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

-	TS 23.038 [7] clause 5

Additionally the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in: ISO/IEC 10646 [17].

27.22.4.12.5.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.5.4	Method of test

27.22.4.12.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.5.4.2	Procedure

Expected Sequence 5.1 (SEND USSD, 7-bit data, successful, UCS2 text in Chinese)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 5.1.1 |  |
| 4 | ME USER | Display "你好" | ["Hello" in Chinese] |
| 5 | ME USS | REGISTER 5.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 5.1 | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND USSD 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Data coding scheme:	UCS2 (16bit)

Text:	"你好"

USSD String

Data coding scheme:	7-bit default, no message class

USSD String:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 4B | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 4F | 60 | 59 | 7D | 8A | 39 | F0 | 41 | E1 | 90 |
|  | 58 | 34 | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 |
|  | E9 | 94 | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 |
|  | CB | E6 | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E |
|  | 9F | D3 | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A |
|  | CD | 76 | C3 | E5 | 60 |  |  |  |  |  |  |  |

REGISTER 5.1

Logically (only USSD argument)

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD String:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| Coding | 30 | 3D | 04 | 01 | F0 | 04 | 38 | 41 | E1 | 90 | 58 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 5.1

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD String:

- "USSD string received from SS"

Coding:

| Coding | 30 | 1E | 04 | 01 | 00 | 04 | 19 | D5 | E9 | 94 | 08 | 9A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | E5 | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 |
|  | 0C | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.12.6	SEND USSD (UCS2 display in Katakana)

27.22.4.12.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.12.6.2	Conformance requirement

The ME shall support the Proactive UICC: Send USSD facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.12, clause 6.6.11, clause 8.12.7, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.17, clause 8.31 and clause 6.5.

-	TS 23.038 [7] clause 5

Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in: ISO/IEC 10646 [17].

27.22.4.12.6.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND USSD proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.12.6.4	Method of test

27.22.4.12.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default. Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.12.6.4.2	Procedure

Expected Sequence 6.1 (SEND USSD, 7-bit data, successful, UCS2 text in Katakana)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND USSD 6.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND USSD 6.1.1 |  |
| 4 | ME USER | Display "ル" | [Character " in Katakana] |
| 5 | ME USS | REGISTER 6.1 |  |
| 6 | USS ME | RELEASE COMPLETE (SS RETURN RESULT) 6.1 | [Successful] |
| 7 | ME UICC | TERMINAL RESPONSE: SEND USSD 6.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND USSD 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Data coding scheme:	UCS2 (16bit)

Text:	"ル"

USSD String

Data coding scheme:	7-bit default, no message class

USSD String:	"ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| BER-TLV: | D0 | 49 | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 | 80 | 30 | EB | 8A | 39 | F0 | 41 | E1 | 90 | 58 | 34 |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

REGISTER 6.1

Logically (only USSD argument)

ProcessUnstructuredSS-Request ARGUMENT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD String:

- "ABCDEFGHIJKLMNOPQRSTUVWXYZ-abcdefghijklmnopqrstuvwxyz-1234567890"

Coding:

| Coding | 30 | 3D | 04 | 01 | F0 | 04 | 38 | 41 | E1 | 90 | 58 | 34 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1E | 91 | 49 | E5 | 92 | D9 | 74 | 3E | A1 | 51 | E9 | 94 |
|  | 5A | B5 | 5E | B1 | 59 | 6D | 2B | 2C | 1E | 93 | CB | E6 |
|  | 33 | 3A | AD | 5E | B3 | DB | EE | 37 | 3C | 2E | 9F | D3 |
|  | EB | F6 | 3B | 3E | AF | 6F | C5 | 64 | 33 | 5A | CD | 76 |
|  | C3 | E5 | 60 |  |  |  |  |  |  |  |  |  |

RELEASE COMPLETE (SS RETURN RESULT) 6.1

Logically (only from USSD result):

ProcessUnstructuredSS-Request RETURN RESULT

USSD-DataCodingScheme:

- 7-bit default, no message class

USSD String:

- "USSD string received from SS"

Coding:

| Coding | 30 | 1E | 04 | 01 | 00 | 04 | 19 | D5 | E9 | 94 | 08 | 9A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D3 | E5 | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 |
|  | 0C | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

TERMINAL RESPONSE: SEND USSD 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND USSD

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Text String

Data coding scheme:	7-bit default, no message class

String:	"USSD string received from SS"

Coding:

| BER-TLV: | 81 | 03 | 01 | 12 | 00 | 82 | 02 | 82 | 81 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 8D | 1A | 00 | D5 | E9 | 94 | 08 | 9A | D3 | E5 |
|  | 69 | F7 | 19 | 24 | 2F | 8F | CB | 69 | 7B | 99 | 0C |
|  | 32 | CB | DF | 6D | D0 | 74 | 0A |  |  |  |  |

27.22.4.12.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

#### 27.22.4.13	SET UP CALL

##### 27.22.4.13.1	SET UP CALL (normal)

27.22.4.13.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.1.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3 and clause 5.2.

27.22.4.13.1.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.1.4	Method of test

27.22.4.13.1.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default, with the following exceptions for sequence 1.1 only:

- The Outgoing Call Information (OCI and OCT) service is available in the USIM Service Table.

- EFOCI (Outgoing Call Information) is present with the following content:

Logically:	Invalid

| Byte: | B01 | … | B41 | B42 | B43 | B44 | B45 | B46 | B47 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | FF | … | FF | 00 | 00 | 00 | 01 | FF | FF |

- EFOCT (Outgoing Call Timer) is present with the following content:

Logically:	Accumulated call timer value: 0

| Byte: | B01 | B02 | B03 |
| --- | --- | --- | --- |
| Coding: | 00 | 00 | 00 |

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.13.1.4.2	Procedure

Expected Sequence 1.1 (SET UP CALL, call confirmed by the user and connected)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.1.1 |  |
| 4 | ME USER | ME displays "Not busy" during user confirmation phase. |  |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  USS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 1.1.1 | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns to idle mode. |  |
| 10 | ME UICC | The ME shall not have updated EF OCI or EF OCT with the call set-up details. |  |

PROACTIVE COMMAND: SET UP CALL 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Not busy"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Coding:

| BER-TLV: | D0 | 1E | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 4E | 6F | 74 | 20 | 62 | 75 | 73 | 79 | 86 | 09 | 91 |
|  | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (SET UP CALL, call rejected by the user)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.1.1 |  |
| 4 | ME USER | ME displays "Not busy" during the user confirmation phase |  |
| 5 | USER ME | The user rejects the set up call | [user rejects the call] |
| 6 | ME UICC | TERMINAL RESPONSE 1.2.1 | [User did not accept call set-up request] |
| 7 | ME  USER | The ME returns in idle mode. |  |

TERMINAL RESPONSE: SET UP CALL 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	User did not accept the proactive command

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.3void

Expected Sequence 1.4 (SET UP CALL, putting all other calls on hold, ME busy)

ME is busy on a call

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.4.1 | [putting all other calls on hold] |
| 4 | ME USER | ME displays "On hold" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirms the call] |
| 6 | ME  USS | The active call is put on hold |  |
| 7 | MEUSS | The ME attempts to set up a call to "+012340123456" |  |
| 8 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 9 | ME UICC | TERMINAL RESPONSE 1.4.1 | [Command performed successfully] |
| 10 | USER ME | The user ends the call after 10 s. The ME retrieves the previous call automatically or on request of the user. |  |

PROACTIVE COMMAND: SET UP CALL 1.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	putting all other calls on hold

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"On hold"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 10 | 02 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 4F | 6E | 20 | 68 | 6F | 6C | 64 | 86 | 09 | 91 | 10 |
|  | 32 | 04 | 21 | 43 | 65 | 1C | 2C |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	putting all other calls on hold

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.5 (SET UP CALL, disconnecting all other calls, ME busy)

ME is busy on a call

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.5.1 | [disconnecting all other calls] |
| 4 | ME USER | ME displays "Disconnect" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirms the call] |
| 6 | ME  USS | The ME disconnects the active call |  |
| 7 | MEUSS | The ME attempts to set up a call to "+012340123456" |  |
| 8 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 9 | ME UICC | TERMINAL RESPONSE 1.5.1 | [Command performed successfully] |
| 10 | USER ME | The user ends the call after 10 s. |  |

PROACTIVE COMMAND: SET UP CALL 1.5.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	disconnecting all other calls

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Disconnect"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 10 | 04 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 44 | 69 | 73 | 63 | 6F | 6E | 6E | 65 | 63 | 74 | 86 |
|  | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C |  |  |

TERMINAL RESPONSE: SET UP CALL 1.5.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	putting all other calls on hold

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 04 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.6 (SET UP CALL, only if not currently busy on another call, ME busy)

ME is busy on a call

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.1.1 | [only if not currently busy on another call] |
| 4 | ME UICC | TERMINAL RESPONSE 1.6.1 | [ME currently unable to process command] |

TERMINAL RESPONSE: SET UP CALL 1.6.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command

Additional Information:	ME currently busy on call

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.7 (SET UP CALL, putting all other calls on hold, call hold is not allowed)

ME is busy on a call. The USS shall be configured to not allow Call Hold.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.4.1 | [putting all other calls on hold] |
| 4 | ME USER | ME displays "On hold" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirms the call] |
| 6 | ME USS | The ME attempts to put the active call on hold. |  |
| 7 | USS->ME | The ME receives the HOLD REJECT message from the USS. | [USS sends "Facility Rejected" as cause value] |
| 8 | ME UICC | TERMINAL RESPONSE 1.7.1A OR TERMINAL RESPONSE 1.7.1B | [Network currently unable to process command] [Option A shall apply only from R99 to Rel-6, whereas option B is applicable in all releases] |

TERMINAL RESPONSE: SET UP CALL 1.7.1A

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	putting all other calls on hold

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Network currently unable to process command

Additional Information:	No specific cause can be given

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 02 | 82 | 02 | 82 | 81 | 83 | 02 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.7.1B

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	putting all other calls on hold

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Network currently unable to process command

Additional Information:	Facility Rejected

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 02 | 82 | 02 | 82 | 81 | 83 | 02 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9D |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.8 (SET UP CALL, Capability configuration)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.8.1 | [Capability configuration parameters: full rate support] |
| 4 | ME USER | ME displays "Capability config" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456" using the capability configuration parameters supplied by UICC |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 1.8.1 | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 1.8.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	if not busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Capability config"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Capability configuration parameters

Information transfer cap:	full rate support only MS

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 43 | 61 | 70 | 61 | 62 | 69 | 6C | 69 | 74 | 79 | 20 |
|  | 63 | 6F | 6E | 66 | 69 | 67 | 86 | 09 | 91 | 10 | 32 | 04 |
|  | 21 | 43 | 65 | 1C | 2C | 87 | 02 | 01 | A0 |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.8.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	if not busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.9 (SET UP CALL, max dialling number string, no alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND SET UP CALL 1.9.1 | [dialling number string, no alpha identifier] |
| 4 | USER ME | The user confirms the set up call | [user confirmation] |
| 5 | MEUSS | The ME attempts to set up a call to "+01234567890123456789012345678901" |  |
| 6 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 7 | ME UICC | TERMINAL RESPONSE 1.9.1 | [Command performed successfully] |
| 8 | USER ME | The user ends the call The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 1.9.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call with redial

Device identities

Source device:	UICC

Destination device:	Network

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string:	"01234567890123456789012345678901"

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 10 | 01 | 82 | 02 | 81 | 83 | 86 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 11 | 91 | 10 | 32 | 54 | 76 | 98 | 10 | 32 | 54 | 76 | 98 |
|  | 10 | 32 | 54 | 76 | 98 | 10 |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.9.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call with redial

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.10 (SET UP CALL,256 octets length, long first alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.10.1 | [ alpha identifier] |
| 4 | ME USER | ME displays "Three types are defined: - set up a call, but only if not currently busy on another call; - set up a call, putting all other calls (if any) on hold; - set up a call, disconnecting all other calls (if any) first. For each of these types, " during the user confirmation phase. |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+01" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 1.10.1 | [Command performed successfully] |
| 9 | USER ME | The user ends the call The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 1.10.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call with redial

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Three types are defined: - set up a call, but only if not currently busy on another call; - set up a call, putting all other calls (if any) on hold; - set up a call, disconnecting all other calls (if any) first. For each of these types, "

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string:	"01"

Coding:

| BER-TLV: | D0 | 81 | FD | 81 | 03 | 01 | 10 | 01 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 81 | ED | 54 | 68 | 72 | 65 | 65 | 20 | 74 | 79 | 70 |
|  | 65 | 73 | 20 | 61 | 72 | 65 | 20 | 64 | 65 | 66 | 69 | 6E |
|  | 65 | 64 | 3A | 20 | 2D | 20 | 73 | 65 | 74 | 20 | 75 | 70 |
|  | 20 | 61 | 20 | 63 | 61 | 6C | 6C | 2C | 20 | 62 | 75 | 74 |
|  | 20 | 6F | 6E | 6C | 79 | 20 | 69 | 66 | 20 | 6E | 6F | 74 |
|  | 20 | 63 | 75 | 72 | 72 | 65 | 6E | 74 | 6C | 79 | 20 | 62 |
|  | 75 | 73 | 79 | 20 | 6F | 6E | 20 | 61 | 6E | 6F | 74 | 68 |
|  | 65 | 72 | 20 | 63 | 61 | 6C | 6C | 3B | 20 | 2D | 20 | 73 |
|  | 65 | 74 | 20 | 75 | 70 | 20 | 61 | 20 | 63 | 61 | 6C | 6C |
|  | 2C | 20 | 70 | 75 | 74 | 74 | 69 | 6E | 67 | 20 | 61 | 6C |
|  | 6C | 20 | 6F | 74 | 68 | 65 | 72 | 20 | 63 | 61 | 6C | 6C |
|  | 73 | 20 | 28 | 69 | 66 | 20 | 61 | 6E | 79 | 29 | 20 | 6F |
|  | 6E | 20 | 68 | 6F | 6C | 64 | 3B | 20 | 2D | 20 | 73 | 65 |
|  | 74 | 20 | 75 | 70 | 20 | 61 | 20 | 63 | 61 | 6C | 6C | 2C |
|  | 20 | 64 | 69 | 73 | 63 | 6F | 6E | 6E | 65 | 63 | 74 | 69 |
|  | 6E | 67 | 20 | 61 | 6C | 6C | 20 | 6F | 74 | 68 | 65 | 72 |
|  | 20 | 63 | 61 | 6C | 6C | 73 | 20 | 28 | 69 | 66 | 20 | 61 |
|  | 6E | 79 | 29 | 20 | 66 | 69 | 72 | 73 | 74 | 2E | 20 | 46 |
|  | 6F | 72 | 20 | 65 | 61 | 63 | 68 | 20 | 6F | 66 | 20 | 74 |
|  | 68 | 65 | 73 | 65 | 20 | 74 | 79 | 70 | 65 | 73 | 2C | 20 |
|  | 86 | 02 | 91 | 10 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.10.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call with redial

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.11A (SET UP CALL, Called party subaddress, command performed successfully)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.11.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.11.1 | [set up a call with called party subaddress] |
| 4 | ME USER | ME displays "Called party" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+012340123456" with the called party subaddress information |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 1.11.1A | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s The ME returns in idle mode. |  |

Expected Sequence 1.11B (SET UP CALL, Called party subaddress, ME not supporting the called party subaddress)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.11.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.11.1 | [set up a call with called party subaddress] |
| 4 | ME UICC | TERMINAL RESPONSE 1.11.1B | [beyond ME's capabilities] |

PROACTIVE COMMAND: SET UP CALL 1.11.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	if not busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Called party"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string:	"012340123456p1p2"

Called party subaddress

Type of subaddress:	NSAP (X.213 / ISO 8348 AD2)

Odd / even indicator:	even number of address signals

Subaddress information:	AFI, 95, 95, 95, 95, 95

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0C | 43 | 61 | 6C | 6C | 65 | 64 | 20 | 70 | 61 | 72 | 74 |
|  | 79 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C |
|  | 88 | 07 | 80 | 50 | 95 | 95 | 95 | 95 | 95 |  |  |  |

TERMINAL RESPONSE: SET UP CALL 1.11.1A

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	if not busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: SET UP CALL 1.11.1B

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	if not busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Beyond ME's capabilities

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.12 (SET UP CALL, maximum duration for the redial mechanism)

The USS shall be configured such that call set up requests will be rejected with cause "User Busy".

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 1.12.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 1.12.1 | [only if not currently busy on another call with redial] |
| 4 | ME USER | ME displays "Duration" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirms the call] |
| 6 | ME  USS | ME attempts to set up a call to "+012340123456" . It stops its attempts after 10 seconds. | [redial mechanism with maximum duration of 10 seconds]] |
| 7 | ME UICC | TERMINAL RESPONSE 1.12.1 | [network currently unable to process command] |
| 8 | ME  USER | The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 1.12.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call with redial

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Duration"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string:	"012340123456p1p2"

Duration

Unit:	Seconds

Interval:	10

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 10 | 01 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 44 | 75 | 72 | 61 | 74 | 69 | 6F | 6E | 86 | 09 | 91 |
|  | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C | 84 | 02 | 01 | 0A |

TERMINAL RESPONSE: SET UP CALL 1.12.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call with redial

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	network currently unable to process command

Additional Information:	User Busy

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 01 | 82 | 02 | 82 | 81 | 83 | 02 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 91 |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.13.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.12.

##### 27.22.4.13.2	SET UP CALL (second alpha identifier)

27.22.4.13.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.2.2	Conformance requirement

Same as clause 27.22.4.13.2.1.

27.22.4.13.2.3	Test purpose

To verify that the ME accepts a Proactive Command - Set Up Call, displays the alpha identifiers to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.2.4	Method of test

27.22.4.13.2.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and is in updated idle mode on the USS.

27.22.4.13.2.4.2	Procedure

Expected Sequence 2.1 (SET UP CALL, two alpha identifiers)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 2.1.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL" | [second alpha identifier] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 2.1.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 2.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL"

Coding:

| BER-TLV: | D0 | 28 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0C | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C |
|  | 85 | 04 | 43 | 41 | 4C | 4C |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 2.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

##### 27.22.4.13.3	SET UP CALL (display of icons)

27.22.4.13.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.3.2	Conformance requirement

27.22.4.13.3.3	Test purpose

To verify that the ME accepts a Proactive Set Up Call , displays the message or icon to the user ,attempts to set up a call to the address, returns the result in the TERMINAL response.

27.22.4.13.3.4	Method of test

27.22.4.13.3.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and is in updated idle mode on the USS.

27.22.4.13.3.4.2	Procedure

Expected Sequence 3.1A (SET UP CALL, display of basic icon during confirmation phase, not self-explanatory, successful )

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.1.1 | Including icon identifier, icon shall be displayed in addition of the first alpha identifier |
| 4 | ME USER | ME displays "Set up call Icon 3.1.1" and the basic icon during a user confirmation phase. |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.1.1A | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 3.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Set up call Icon 3.1.1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Icon identifier

Icon qualifier:	icon is not self-explanatory

Icon identifier:	<record 1 in EF IMG>

Coding:

| BER-TLV: | D0 | 30 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 53 | 65 | 74 | 20 | 75 | 70 | 20 | 63 | 61 | 6C | 6C |
|  | 20 | 49 | 63 | 6F | 6E | 20 | 33 | 2E | 31 | 2E | 31 | 86 |
|  | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C | 9E | 02 |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 3.1.1A

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.1B (SET UP CALL, display of basic icon during confirmation phase, not self-explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.1.1 | Including icon identifier, icon shall be displayed in addition of the first alpha identifier |
| 4 | ME USER | ME displays "Set up call Icon 3.1.1" without the basic icon during a user confirmation phase. |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.1.1B | [Command performed successfully, but requested icon could not be displayed]. |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

TERMINAL RESPONSE: SET UP CALL 3.1.1B

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.2A (SET UP CALL, display of basic icon during confirmation phase, self-explanatory, successful )

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.2.1 | Including icon identifier, icon shall be displayed instead of the first alpha identifier |
| 4 | ME USER | ME displays the basic icon during a user confirmation phase. |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.2.1A | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 3.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Set up call Icon 3.2.1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Icon identifier

Icon qualifier:	icon is self-explanatory

Icon identifier:	<record 1 in EF IMG>

Coding:

| BER-TLV: | D0 | 30 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 53 | 65 | 74 | 20 | 75 | 70 | 20 | 63 | 61 | 6C | 6C |
|  | 20 | 49 | 63 | 6F | 6E | 20 | 33 | 2E | 32 | 2E | 31 | 86 |
|  | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C | 9E | 02 |
|  | 00 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 3.2.1A

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.2B (SET UP CALL, display of basic icon during confirmation phase, self-explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.2.1 | Including icon identifier, icon shall be displayed instead of the first alpha identifier |
| 4 | ME USER | ME display "Set up call Icon 3.2.1" without the icon |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.2.1B | [Command performed successfully, but requested icon could not be displayed]. |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

TERMINAL RESPONSE: SET UP CALL 3.2.1B

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.3A (SET UP CALL, display of colour icon during confirmation phase, not self-explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.3.1 | Including icon identifier, icon shall be displayed in addition of the first alpha identifier |
| 4 | ME USER | ME displays "Set up call Icon 3.3.1" and the colour icon during a user confirmation phase. |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.3.1A | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 3.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Set up call Icon 3.3.1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Icon identifier

Icon qualifier:	icon is not self-explanatory

Icon identifier:	<record 2 in EF IMG>

Coding:

| BER-TLV: | D0 | 30 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 53 | 65 | 74 | 20 | 75 | 70 | 20 | 63 | 61 | 6C | 6C |
|  | 20 | 49 | 63 | 6F | 6E | 20 | 33 | 2E | 33 | 2E | 31 | 86 |
|  | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C | 9E | 02 |
|  | 01 | 02 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 3.3.1A

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.3B (SET UP CALL, display of colour icon during confirmation phase, not self-explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.3.1 | Including icon identifier, icon shall be displayed in addition of the first alpha identifier |
| 4 | ME USER | ME only display alpha string: " Set up call Icon 3.3.1" |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.3.1B | [Command performed successfully, but requested icon could not be displayed]. |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

TERMINAL RESPONSE: SET UP CALL 3.3.1B

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.4A (SET UP CALL, display of self explanatory basic icon during set up call, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.4.1 | Including a second alpha identifier and two icons |
| 4 | ME USER | ME displays the basic icon during a user confirmation phase. |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | MEUSS | The ME attempts to set up a call to "+012340123456". The ME displays the basic icon without the text during the set up call. |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.4.1A | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 3.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Set up call Icon 3.4.1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Icon identifier

Icon qualifier:	icon is self-explanatory

Icon identifier:	<record 1 in EF IMG>

Alpha identifier:	"Set up call Icon 3.4.2"

Icon identifier

Icon qualifier:	icon is self-explanatory

Icon identifier:	<record 1 in EF IMG>

Coding:

| BER-TLV: | D0 | 4C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 53 | 65 | 74 | 20 | 75 | 70 | 20 | 63 | 61 | 6C | 6C |
|  | 20 | 49 | 63 | 6F | 6E | 20 | 33 | 2E | 34 | 2E | 31 | 86 |
|  | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 | 1C | 2C | 9E | 02 |
|  | 00 | 01 | 85 | 16 | 53 | 65 | 74 | 20 | 75 | 70 | 20 | 63 |
|  | 61 | 6C | 6C | 20 | 49 | 63 | 6F | 6E | 20 | 33 | 2E | 34 |
|  | 2E | 32 | 9E | 02 | 00 | 01 |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 3.4.1A

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 3.4B (SET UP CALL, display of self explanatory basic icon during set up call, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 3.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 3.4.1 | Including a second alpha identifier and two icons |
| 4 | ME USER | ME displays "Set up call Icon 3.4.1" without the icon |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "Set up call Icon 3.4.2" without the icon during the set up call. |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 3.4.1B | [Command performed successfully, but requested icon could not be displayed]. |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

TERMINAL RESPONSE: SET UP CALL 3.4.1B

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1A to 3.4B.

##### 27.22.4.13.4	SET UP CALL (support of Text Attribute)

27.22.4.13.4.1	SET UP CALL (support of Text Attribute – Left Alignment)

27.22.4.13.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.1.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.1.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the left alignment text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.1.4	Method of test

27.22.4.13.4.1.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.1.4.2	Procedure

Expected Sequence 4.1 (SET UP CALL, Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.1.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with left alignment] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with left alignment] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.1.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.1.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.1.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [User confirmation shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/14, no alignment change will take place] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [Second alpha identifier shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/14, no alignment change will take place] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.1.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 18 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.1.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

27.22.4.13.4.2	SET UP CALL (support of Text Attribute – Center Alignment)

27.22.4.13.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.2.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.2.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the center alignment text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.2.4	Method of test

27.22.4.13.4.2.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.2.4.2	Procedure

Expected Sequence 4.2 (SET UP CALL, Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.2.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with center alignment] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with center alignment] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.2.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.2.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.2.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [User confirmation shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/14, no alignment change will take place] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [Second alpha identifier shall be formatted without centert alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/14, no alignment change will take place] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.2.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 01 | B4 | D0 | 04 | 00 | 06 | 01 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.2.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.2.

27.22.4.13.4.3	SET UP CALL (support of Text Attribute – Right Alignment)

27.22.4.13.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.3.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.3.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the right alignment text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.3.4	Method of test

27.22.4.13.4.3.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.3.4.2	Procedure

Expected Sequence 4.3 (SET UP CALL, Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.3.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with right alignment] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with right alignment] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.3.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.3.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.3.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [User confirmation shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/14, no alignment change will take place] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [Second alpha identifier shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/14, no alignment change will take place] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.3.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 02 | B4 | D0 | 04 | 00 | 06 | 02 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.3.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.3.

27.22.4.13.4.4	SET UP CALL (support of Text Attribute – Large Font Size)

27.22.4.13.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.4.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.4.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the large font size text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.4.4	Method of test

27.22.4.13.4.4.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.4.4.2	Procedure

Expected Sequence 4.4 (SET UP CALL, Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.4.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with large font size] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with large font size] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.4.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.4.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.4.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with normal font size] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with normal font size] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.4.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.4.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.4.1 |  |
| 22 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 23 | USER ME | The user confirms the set up call | [user confirmation is displayed with large font size] |
| 24 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with large font size] |
| 25 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 26 | ME UICC | TERMINAL RESPONSE 4.4.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 27 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.4.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.4.3 |  |
| 31 | ME USER | ME displays "CONFIRMATION 3" during the user confirmation phase |  |
| 32 | USER ME | The user confirms the set up call | [user confirmation is displayed with normal font size] |
| 33 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 3" | [second alpha identifier is displayed with normal font size] |
| 34 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 35 | ME UICC | TERMINAL RESPONSE 4.4.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 36 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 04 | B4 | D0 | 04 | 00 | 06 | 04 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.4.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.4.3

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 3"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 33 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.4.

27.22.4.13.4.5	SET UP CALL (support of Text Attribute – Small Font Size)

27.22.4.13.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.5.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.5.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the small font size text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.4.5	Method of test

27.22.4.13.4.4.5.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.4.5.2	Procedure

Expected Sequence 4.5 (SET UP CALL, Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.5.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with small font size] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with small font size] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.5.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.5.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.5.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with normal font size] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with normal font size] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.5.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.5.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.5.1 |  |
| 22 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 23 | USER ME | The user confirms the set up call | [user confirmation is displayed with small font size] |
| 24 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with small font size] |
| 25 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 26 | ME UICC | TERMINAL RESPONSE 4.5.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 27 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.5.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.5.3 |  |
| 31 | ME USER | ME displays "CONFIRMATION 3" during the user confirmation phase |  |
| 32 | USER ME | The user confirms the set up call | [user confirmation is displayed with normal font size] |
| 33 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 3" | [second alpha identifier is displayed with normal font size] |
| 34 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 35 | ME UICC | TERMINAL RESPONSE 4.5.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 36 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.5.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 08 | B4 | D0 | 04 | 00 | 06 | 08 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.5.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.5.3

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 3"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 33 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.5.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.5.

27.22.4.13.4.6	SET UP CALL (support of Text Attribute – Bold On)

27.22.4.13.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.6.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.6.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the bold text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.6.4	Method of test

27.22.4.13.4.6.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.6.4.2	Procedure

Expected Sequence 4.6 (SET UP CALL, Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.6.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with bold on] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with bold on] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.6.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.6.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.6.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with bold off] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with bold off] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.6.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.6.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.6.1 |  |
| 22 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 23 | USER ME | The user confirms the set up call | [user confirmation is displayed with bold on] |
| 24 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with bold on] |
| 25 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 26 | ME UICC | TERMINAL RESPONSE 4.6.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 27 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.6.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.6.3 |  |
| 31 | ME USER | ME displays "CONFIRMATION 3" during the user confirmation phase |  |
| 32 | USER ME | The user confirms the set up call | [user confirmation is displayed with bold off] |
| 33 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 3" | [second alpha identifier is displayed with bold off] |
| 34 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 35 | ME UICC | TERMINAL RESPONSE 4.6.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 36 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.6.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 10 | B4 | D0 | 04 | 00 | 06 | 10 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.6.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.6.3

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 3"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 33 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.6.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.6.

27.22.4.13.4.7	SET UP CALL (support of Text Attribute – Italic On)

27.22.4.13.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.7.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.7.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the italic text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.7.4	Method of test

27.22.4.13.4.7.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.7.4.2	Procedure

Expected Sequence 4.7 (SET UP CALL, Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.7.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with italic on] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with italic on] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.7.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.7.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.7.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with italic off] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with italic off] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.7.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.7.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.7.1 |  |
| 22 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 23 | USER ME | The user confirms the set up call | [user confirmation is displayed with italic on] |
| 24 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with italic on] |
| 25 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 26 | ME UICC | TERMINAL RESPONSE 4.7.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 27 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.7.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.7.3 |  |
| 31 | ME USER | ME displays "CONFIRMATION 3" during the user confirmation phase |  |
| 32 | USER ME | The user confirms the set up call | [user confirmation is displayed with italic off] |
| 33 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 3" | [second alpha identifier is displayed with italic off] |
| 34 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 35 | ME UICC | TERMINAL RESPONSE 4.7.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 36 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.7.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 20 | B4 | D0 | 04 | 00 | 06 | 20 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.7.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.7.3

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 3"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 33 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.7.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.7.

27.22.4.13.4.8	SET UP CALL (support of Text Attribute – Underline On)

27.22.4.13.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.8.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.8.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the underline text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.8.4	Method of test

27.22.4.13.4.8.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.8.4.2	Procedure

Expected Sequence 4.8 (SET UP CALL, Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.8.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with underline on] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with underline on] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.8.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.8.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.8.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with underline off] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with underline off] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.8.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.8.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.8.1 |  |
| 22 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 23 | USER ME | The user confirms the set up call | [user confirmation is displayed with underline on] |
| 24 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with underline on] |
| 25 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 26 | ME UICC | TERMINAL RESPONSE 4.8.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 27 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.8.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.8.3 |  |
| 31 | ME USER | ME displays "CONFIRMATION 3" during the user confirmation phase |  |
| 32 | USER ME | The user confirms the set up call | [user confirmation is displayed with underline off] |
| 33 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 3" | [second alpha identifier is displayed with Undeline off] |
| 34 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 35 | ME UICC | TERMINAL RESPONSE 4.8.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 36 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.8.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 40 | B4 | D0 | 04 | 00 | 06 | 40 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.8.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.8.3

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 3"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 33 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.8.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.8.

27.22.4.13.4.9	SET UP CALL (support of Text Attribute – Strikethrough On)

27.22.4.13.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.9.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.9.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the strikethrough text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.9.4	Method of test

27.22.4.13.4.9.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.9.4.2	Procedure

Expected Sequence 4.9 (SET UP CALL, Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.9.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with strikethrough on] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with strikethrough on] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.9.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.9.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.9.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with strikethrough off] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with strikethrough off] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.9.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.9.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.9.1 |  |
| 22 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 23 | USER ME | The user confirms the set up call | [user confirmation is displayed with strikethrough on] |
| 24 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with strikethrough on] |
| 25 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 26 | ME UICC | TERMINAL RESPONSE 4.9.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 27 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.9.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.9.3 |  |
| 31 | ME USER | ME displays "CONFIRMATION 3" during the user confirmation phase |  |
| 32 | USER ME | The user confirms the set up call | [user confirmation is displayed with strikethrough off] |
| 33 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 3" | [second alpha identifier is displayed with strikethrough off] |
| 34 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 35 | ME UICC | TERMINAL RESPONSE 4.9.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 36 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.9.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 80 | B4 | D0 | 04 | 00 | 06 | 80 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.9.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | B4 |  |  |

PROACTIVE COMMAND: SET UP CALL 4.9.3

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 3"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 33 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.9.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.9.

27.22.4.13.4.10	SET UP CALL (support of Text Attribute – Foreground and Background Colour)

27.22.4.13.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.4.10.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3, clause 8.70 and clause 5.2.

27.22.4.13.4.10.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier according to the foreground and background colour text attribute configuration to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.4.10.4	Method of test

27.22.4.13.4.10.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and in the updated idle mode on the USS.

27.22.4.13.4.10.4.2	Procedure

Expected Sequence 4.10 (SET UP CALL, Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.10.1 |  |
| 4 | ME USER | ME displays "CONFIRMATION 1" during the user confirmation phase |  |
| 5 | USER ME | The user confirms the set up call | [user confirmation is displayed with foreground and background colour according to Text Attribute configuration] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 1" | [second alpha identifier is displayed with foreground and background colour according to Text Attribute configuration] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 8 | ME UICC | TERMINAL RESPONSE 4.10.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 4.10.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: SET UP CALL 4.10.2 |  |
| 13 | ME USER | ME displays "CONFIRMATION 2" during the user confirmation phase |  |
| 14 | USER ME | The user confirms the set up call | [user confirmation is displayed with ME's default foreground and background colour] |
| 15 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "CALL 2" | [second alpha identifier is displayed with ME's default foreground and background colour] |
| 16 | USS ME | The ME receives the CONNECT message from the USS. | [The USS also has to handle the START DTMF and STOP DTMF messages sent by the ME in an appropriate way] |
| 17 | ME UICC | TERMINAL RESPONSE 4.10.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 18 | USER ME | The user ends the call after 10 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 4.10.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 1"

Text Attribute (user confirmation phase)

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Text Attribute (call set up phase)

Formatting position:	0

Formatting length:	6

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Bright Yellow Foreground, Dark Green Background

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 31 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0E | 00 | B4 | D0 | 04 | 00 | 06 | 00 | 4B |  |  |

PROACTIVE COMMAND: SET UP CALL 4.10.2

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"CONFIRMATION 2"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456p1p2"

Alpha Identifier (call set up phase):	"CALL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 43 | 4F | 4E | 46 | 49 | 52 | 4D | 41 | 54 | 49 | 4F |
|  | 4E | 20 | 32 | 86 | 09 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |
|  | 1C | 2C | 85 | 06 | 43 | 41 | 4C | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: SET UP CALL 4.10.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.4.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.10.

##### 27.22.4.13.5	SET UP CALL (UCS2 Display in Cyrillic)

27.22.4.13.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.5.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3 and clause 5.2.

The ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.13.5.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier with UCS2 coding to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.5.4	Method of test

27.22.4.13.5.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.13.5.4.2	Procedure

Expected Sequence 5.1 (SET UP CALL with UCS2 – Cyrillic Characters, call confirmed by the user and connected)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 5.1.1 |  |
| 4 | ME USER | ME displays "ЗДРАВСТВУЙТЕ" during user confirmation phase. | ["ЗДРАВСТВУЙТЕ": "Hello" in Russian] |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  USS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 5.1.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 5 s. The ME returns to idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 5.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"ЗДРАВСТВУЙТЕ"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 2D | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19 | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 |
|  | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 |
|  | 04 | 15 | 86 | 07 | 91 | 10 | 32 | 04 | 21 | 43 | 65 |  |

TERMINAL RESPONSE: SET UP CALL 5.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 5.2 (SET UP CALL, two alpha identifiers coded in UCS2 – Cyrillic Characters)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 5.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 5.2.1 |  |
| 4 | ME USER | ME displays "ЗДРАВСТВУЙТЕ1" during the user confirmation phase | ["ЗДРАВСТВУЙТЕ1" : "Hello1" in Russian] |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "ЗДРАВСТВУЙТЕ2" | [second alpha identifier] ["ЗДРАВСТВУЙТЕ2" : "Hello2" in Russian] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 5.2.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 5 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 5.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	" ЗДРАВСТВУЙТЕ1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456"

Alpha Identifier (call set up phase):	" ЗДРАВСТВУЙТЕ2"

Coding:

| BER-TLV: | D0 | 4C | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1B | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 |
|  | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 |
|  | 04 | 15 | 00 | 31 | 86 | 07 | 91 | 10 | 32 | 04 | 21 | 43 |
|  | 65 | 85 | 1B | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 |
|  | 04 | 12 | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 |
|  | 04 | 22 | 04 | 15 | 00 | 32 |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 5.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.5.5	Test requirement

The ME shall operate in the manner defined in expected sequences 5.1 to 5.2.

##### 27.22.4.13.6	SET UP CALL (UCS2 Display in Chinese)

27.22.4.13.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.6.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3 and clause 5.2.

The ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.13.6.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier with UCS2 coding to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.6.4	Method of test

27.22.4.13.6.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.13.6.4.2	Procedure

Expected Sequence 6.1 (SET UP CALL with UCS2 – Chinese characters, call confirmed by the user and connected)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 6.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 6.1.1 |  |
| 4 | ME USER | ME displays "不忙" during user confirmation phase. | ["不忙" : "Not Busy" in Chinese] |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  USS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 6.1.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 5 s. The ME returns to idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 6.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"不忙"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 19 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 4E | 0D | 5F | D9 | 86 | 07 | 91 | 10 | 32 | 04 |
|  | 21 | 43 | 65 |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 6.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 6.2 (SET UP CALL, two alpha identifiers coded in UCS2 – Chinese characters)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 6.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 6.2.1 |  |
| 4 | ME USER | ME displays "确定" during the user confirmation phase | ["确定" : "Confirmation" in Chinese] |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "打电话" | [second alpha identifier] ["打电话" : "CALL" in Chinese] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 6.2.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 5 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 6.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"确定"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456"

Alpha Identifier (call set up phase):	"打电话"

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 78 | 6E | 5B | 9A | 86 | 07 | 91 | 10 | 32 | 04 |
|  | 21 | 43 | 65 | 85 | 07 | 80 | 62 | 53 | 75 | 35 | 8B | DD |

TERMINAL RESPONSE: SET UP CALL 6.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.6.5	Test requirement

The ME shall operate in the manner defined in expected sequences 6.1 to 6.2.

##### 27.22.4.13.7	SET UP CALL (UCS2 Display in Katakana)

27.22.4.13.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.13.7.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Call facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.13, clause 6.6.12, clause 8.6, clause 8.7, clause 8.12, clause 8.12.3 and clause 5.2.

The ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.13.7.3	Test purpose

To verify that the ME accepts the Proactive Command - Set Up Call, displays the alpha identifier with UCS2 coding to the user, attempts to set up a call to the address and returns the result in the TERMINAL RESPONSE.

27.22.4.13.7.4	Method of test

27.22.4.13.7.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.13.7.4.2	Procedure

Expected Sequence 7.1 (SET UP CALL with UCS2 – Katakana characters, call confirmed by the user and connected)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 7.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 7.1.1 |  |
| 4 | ME USER | ME displays "ル" during user confirmation phase. | [Character in Katakana] |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  USS | The ME attempts to set up a call to "+012340123456" |  |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 7.1.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 5 s. The ME returns to idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 7.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"ル"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 17 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 | 80 | 30 | EB | 86 | 07 | 91 | 10 | 32 | 04 | 21 | 43 |
|  | 65 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 7.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 7.2 (SET UP CALL, two alpha identifiers coded in UCS2 – Katakana characters)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP CALL 7.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP CALL 7.2.1 |  |
| 4 | ME USER | ME displays "ル1" during the user confirmation phase | [Character in Katakana] |
| 5 | USER ME | The user confirms the set up call | [user confirmation] |
| 6 | ME USS | The ME attempts to set up a call to "+012340123456". The ME displays "ル2". | [second alpha identifier] [Character in Katakana] |
| 7 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 8 | ME UICC | TERMINAL RESPONSE 7.2.1 The ME shall not update EF LND with the called party address. | [Command performed successfully] |
| 9 | USER ME | The user ends the call after 5 s. The ME returns in idle mode. |  |

PROACTIVE COMMAND: SET UP CALL 7.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"ル1"

Address

TON:	International

NPI:	ISDN / telephone numbering plan

Dialling number string	"012340123456"

Alpha Identifier (call set up phase):	"ル2"

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 30 | EB | 00 | 31 | 86 | 07 | 91 | 10 | 32 | 04 |
|  | 21 | 43 | 65 | 85 | 05 | 80 | 30 | EB | 00 | 32 |  |  |

TERMINAL RESPONSE: SET UP CALL 7.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.13.7.5	Test requirement

The ME shall operate in the manner defined in expected sequences 7.1 to 7.2.

#### 27.22.4.14	POLLING OFF

##### 27.22.4.14.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.14.2	Conformance requirement

The ME shall support the POLLING OFF as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.14, clause 6.8, clause 6.11, clause 8.6 and clause 8.7.

##### 27.22.4.14.3	Test purpose

To verify that the ME cancels the effect of any previous POLL INTERVAL commands and does not effect UICC presence detection.

##### 27.22.4.14.4	Method of test

27.22.4.14.4.1	Initial conditions

For sequence 1.1:

- The elementary files are coded as Toolkit default.

- The ME is connected to the USIM Simulator and to the USS.

For sequence 1.2:

- The default E-UTRAN/EPC UICC, the default E-UTRAN parameters are used.

- The ME is connected to the USIM Simulator and to the E-USS/NB-SS.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.14.4.2	Procedure

Expected Sequence 1.1 (POLLING OFF)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: POLL INTERVAL 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: POLL INTERVAL 1.1.1 | Interval = 1 min |
| 4 | ME UICC | TERMINAL RESPONSE: POLL INTERVAL 1.1.1 A or TERMINAL RESPONSE: POLL INTERVAL 1.1.1B | [command performed successfully, duration depends on the ME's capabilities] |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: POLLING OFF 1.1.2 |  |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: POLLING OFF 1.1.2 |  |
| 8 | ME UICC | TERMINAL RESPONSE: POLLING OFF 1.1.2 | [command performed successfully] |
| 9 | USER  ME | Call to be set up | A call shall be set up using the generic call setup for circuit switched call or to activate a PDP context. |
| 10 | ME  UICC | Periods of inactivity on the UICC-ME interfaceshall not exceed 30 seconds | In case of PDP context for a terminal that supports Rel-12 or later, exchange of data with the network may be required to guarantee the correct result of the test. |
| 11 | USER  ME | Call to be terminated 3 minutes after call setup |  |

PROACTIVE COMMAND: POLL INTERVAL 1.1.1

Logically:

Command details

Command number:	1

Command type:	POLL INTERVAL

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Duration

Time unit:	Minutes

Time interval:	1

Coding:

| BER-TLV: | D0 | 0D | 81 | 03 | 01 | 03 | 00 | 82 | 02 | 81 | 82 | 84 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 00 | 01 |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: POLL INTERVAL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	POLL INTERVAL

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Duration

Time unit:	Minutes

Time interval:	1

Coding:

| BER-TLV: | 81 | 03 | 01 | 03 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 84 | 02 | 00 | 01 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: POLL INTERVAL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	POLL INTERVAL

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Duration

Time unit:	Seconds

Time interval:	60

Coding:

| BER-TLV: | 81 | 03 | 01 | 03 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 84 | 02 | 01 | 3C |  |  |  |  |  |  |  |  |

Note:	If the requested poll interval is not supported by the ME, the ME is allowed to use a different one as stated in TS 31.111 [15], subclause 6.4.6.

PROACTIVE COMMAND: POLLING OFF 1.1.2

Logically:

Command details

Command number:	1

Command type:	POLLING OFF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 04 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: POLLING OFF 1.1.2

Logically:

Command details

Command number:	1

Command type:	POLLING OFF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 04 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (POLLING OFF, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME E-USS/NB-SS | The ME successfully performs EPS bearer context activation |  |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: POLL INTERVAL 1.1.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: POLL INTERVAL 1.1.1 | Interval = 1 min |
| 5 | ME UICC | TERMINAL RESPONSE: POLL INTERVAL 1.1.1 A or TERMINAL RESPONSE: POLL INTERVAL 1.1.1B | [command performed successfully, duration depends on the ME's capabilities] |
| 6 | UICC ME | PROACTIVE COMMAND PENDING: POLLING OFF 1.1.2 |  |
| 7 | ME UICC | FETCH |  |
| 8 | UICC ME | PROACTIVE COMMAND: POLLING OFF 1.1.2 |  |
| 9 | ME UICC | TERMINAL RESPONSE: POLLING OFF 1.1.2 | [command performed successfully] |
| 10 | ME  UICC | Periods of inactivity on the UICC-ME interface shall not exceed 30 seconds | For a terminal that supports Rel-12 or later, exchange of data with the network is required to guarantee the correct result of the test. |

##### 27.22.4.14.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 - 1.2.

#### 27.22.4.15	PROVIDE LOCAL INFORMATION

##### 27.22.4.15.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.15.2	Conformance requirement

The ME shall support the PROVIDE LOCAL INFORMATION facility as defined in:

-	TS 31.111 [15] clause 6.4.15.

##### 27.22.4.15.3	Test purpose

To verify that the ME returns the following requested local information within a TERMINAL RESPONSE:

-	location information:

-	Mobile Country Code (MCC);

-	Mobile Network Code (MNC);

-	Location Area Code (LAC); and

-	cell ID of the current serving cell;

-	the IMEI of the ME;

-	the Network Measurement Results and the BCCH channel list;

-	the current date, time and time zone;

-	the current ME language setting;

-	the Timing Advance;

-	the Access Technology;

-	the IMEISV

-	the Search Mode change

-	the Battery charge State

-	the UTRAN intra- and inter-frequency measurements,

-	the E-UTRAN intra- and inter-frequency measurements.

-	The CSG ID list and corresponding HNB names of surrounding CSG cells (if class "q" is supported).

if the local information is stored in the ME; otherwise, sends the correct error code to the UICC in the TERMINAL RESPONSE.

To verify that the ME returns required error information in the TERMINAL RESPONSE in case requested information cannot be provided due to missing network coverage.

To verify that the E-UTRAN cell identifier is correctly transmitted when requesting the location information while accessing an E-UTRAN.

To verify that the NG-RAN cell identifier is correctly transmitted when requesting the location information while accessing an NG-RAN SA mode Cell.

To verify that the NG-RAN cell identifier is correctly transmitted when requesting the location information while accessing an NG-RAN Cell.

##### 27.22.4.15.4	Method of tests

27.22.4.15.4.1	Initial conditions

The ME is connected to the USIM Simulator.

Except for sequences 1.2, 1.4, 1.5, 1.9, 1.10 and 1.11 the ME is connected to the USS and except for sequence 1.10 has performed the location update procedure or routing area update or combined update procedure.

For sequence 1.22 and 1.23 the ME is connected to NG-SS and it has performed the Registration procedure.

The NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001 (36 bits);

-	NG-RAN Cell Identity value = 0001 (36 bits);

The E- UTRAN/NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The GERAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

-	Timing advance = 0;

-	Neighbour allocations = 561, 565, 568, 569, 573, 575, 577, 581, 582 and 585.

The elementary files are coded as the USIM Application Toolkit default with the exception that for sequences 1.14 to 1.18, the default E-UTRAN/EPC UICC is used and for sequence 1.22 and 1.23 the default NG-RAN UICC is used.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Expected sequence 1.3 and 1.6 shall be used on a USS setting up only a GERAN or PCS 1900 cell and expected sequences 1.7 and 1.12 shall be used on a USS setting up only a UTRAN cell.

Expected sequence 1.12 requires 2 UTRA cells on the same frequency and 1.13 requires 2 UTRA cells on different frequencies.

Expected sequences 1.14 and 1.17 shall be used on a E-USS/NB-SS setting up only a E-UTRAN/NB-IoT cell.

Expected sequence 1.22 and 1.23 shall be used on a NG-SS setting up only a NG-RAN cell.

Expected sequence 1.15 requires 2 E-UTRA cells on the same frequency and 1.16 requires 2 E-UTRA cells on different frequencies, with second cell having EARFCN less than maxEARFCN. For both sequences if the USIM request is triggered in the RRC_CONNECTED state, the system simulator shall configure the corresponding frequency measurement for a sufficient period before sending the USIM request.

Expected sequence 1.18 requires 2 E-UTRAN cells configured in CSG mode.

For sequence 1.18 the default E-UTRAN/EPC UICC is used and the E-USS transmits on 2 cells with the following parameters:

Network parameters for cell 1:

-	TAI (MCC/MNC/TAC):	001/01/0001.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	01 (27 bits)

- Home (e)NB Name	Home ONE

Network parameters for cell 2:

-	TAI (MCC/MNC/TAC):	001/01/0002.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	02 (27 bits)

- Home (e)NB Name	Home TWO

27.22.4.15.4.2	Procedure

Expected Sequence 1.1 (PROVIDE LOCAL INFORMATION, Local Info (MCC, MNC, LAC & Cell ID))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1A or TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1B | [Command performed successfully, MCC MNC LAC and Cell Identity as USS, option A shall apply for 3GPP parameters] [Command performed successfully, MCC MNC LAC and Cell Identity as USS, option B shall apply for PCS1900 parameters] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC LAC and Cell Identity)

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1A

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC LAC and Cell Identity)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Location Information

MCC & MNC:	MCC = 001, MNC = 01

Location Area Code:		0001

Cell Identity Value:		0001

Extended Cell Identity Value:	RNC-id value (for Rel-4 onwards), see also Note 2

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 93 | Note 1 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 2 |  |  |

Note 1: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 2: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.1.1B

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC LAC and Cell Identity)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Location Information

MCC & MNC:	MCC = 001, MNC = 011

Location Area Code:	0001

Cell Identity Value:	0001

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 93 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |  |

Expected Sequence 1.2 (PROVIDE LOCAL INFORMATION, IMEI of the ME)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.2.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.2.1 | [Command performed successfully, IMEI as declared in A.2/23, coded according to TS 24.008 [10], clause 10.5.1, but spare digit shall be zero when transmitted by the ME] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.2.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"01" IMEI of the ME

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 01 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.2.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"01" IMEI of the ME

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

IMEI

IMEI of the ME:	The IMEI of the ME

The result coding depends on the Mobile IMEI value as declared in table A.2/23.

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 94 | 08 | XX | XX | XX | XX | XX | XX | XX | XX |  |  |

As an example, if the IMEI of the mobile is "123456789012345" then XX XX XX XX XX XX XX XX = 1A 32 54 76 98 10 32 04. For further details see also TS 24.008 [10], clause 10.5.1.

Expected Sequence 1.3 (PROVIDE LOCAL INFORMATION, Network Measurement Results (NMR))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.3.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.3.1 | [Command performed successfully, NMR as USS ] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.3.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.3.1

The actual values of the measurements are not tested.

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Measurement Results	RXLEV-FULL-SERVING-CELL=52, BA not used, DTX not used, as an example in the BER-TLV)

BCCH channel list	561, 565, 568, 569, 573, 575, 577, 581, 582 and 585

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 96 | 10 | 34 | 34 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 9D | 0D | 8C | 63 | 58 | E2 |
|  | 39 | 8F | 63 | F9 | 06 | 45 | 91 | A4 | 90 |  |  |  |

Expected Sequence 1.4 (PROVIDE LOCAL INFORMATION, Date, Time, Time Zone)

See ETSI TS 102 384 [26] in subclause 27.22.4.15.4.2, Expected Sequence 1.4.

Expected Sequence 1.5 (PROVIDE LOCAL INFORMATION, Language setting)

See ETSI TS 102 384 [26] in subclause 27.22.4.15.4.2, Expected Sequence 1.5.

Expected Sequence 1.6 (PROVIDE LOCAL INFORMATION, Timing advance)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.6.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.6.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.6.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"05" Timing Advance

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 05 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.6.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"05" Timing Advance

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Timing Advance	2 bytes

ME status:	"00" ME is in idle state

Timing Advance:	0

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 05 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | AE | 02 | 00 | 00 |  |  |  |  |  |  |  |  |

Expected Sequence 1.7 (PROVIDE LOCAL INFORMATION, Access Technology

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.7.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.7.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.7.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.7.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Access Technology

Technology:	UTRAN

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3F | 01 | 03 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.8 (Void)

Expected Sequence 1.9 (PROVIDE LOCAL INFORMATION, IMEISV of the terminal)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.9.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.9.1 | [Command performed successfully, IMEISV as declared in A.2/24, coded as defined in TS 24.008 [10]] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.9.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"08" IMEISV of the ME

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 08 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.9.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"08" IMEISV of the ME

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

IMEISV

IMEISV of the ME:	The IMEISV of the ME

The result coding depends on the ME IMEISV value as declared in table A.2/24.

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 08 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | E2 | 09 | XX | XX | XX | XX | XX | XX | XX | XX | XX |  |

As an example, if the IMEISV of the ME is "1234567890123456" then XX XX XX XX XX XX XX XX XX= 13 32 54 76 98 10 32 54 F6. For further details see also TS 24.008 [10].

Expected Sequence 1.10 (PROVIDE LOCAL INFORMATION, Network Search Mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User | The user sets the ME to manual network selection mode |  |
| 2 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.10.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.10.1 |  |
| 5 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.10.1 | [Command performed successfully] |
| 6 | User | The user selects automatic network selection mode |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.10.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.10.2 |  |
| 10 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.10.2 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.10.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"09" Search Mode

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 09 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.10.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"09" Search Mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Search Mode	Manual mode

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 09 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 65 | 01 | 00 |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.10.2

same as PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.10.1

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.10.2

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"09" Search Mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Search Mode	Automatic mode

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 09 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 65 | 01 | 01 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.11 (PROVIDE LOCAL INFORMATION, charge state of the battery)

See ETSI TS 102 384 [26] in subclause 27.22.4.15.4.2, Expected Sequence 1.11.

Expected Sequence 1.12 (PROVIDE LOCAL INFORMATION, Intra-Frequency UTRAN Measurements)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.12.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.12.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.12.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.12.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	UICC

Destination device:	ME

UTRAN/E-UTRAN Measurement Qualifier

UTRAN/E-UTRAN Measurement Qualifier:  "01" Intra-frequency measurements

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 81 | 82 | 69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.12.1

The actual values of the measurements are not tested.

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Measurement Results	MEASUREMENT REPORT message

intraFreqMeasuredResultsList

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 96 | Note 1 | 80 | Note 2 | Note 3 | Note 4 |  |  |  |  |  |  |

Note 1: This is the length indicator for the following bytes which represent the Measurement report coded in ASN.1 and therefore the length cannot be foreseen.

Note2: This byte shall be checked bitwise against pattern: 0000 xxxx (x – don't care).

Note 3: This byte shall be checked bitwise against pattern: x000 0111 (x – don't care).

Note 4: The remaining bytes shall not be verified.

The network measurement result indicated by the sequence of bytes above is:

MeasurementReport

measurementIdentity

measuredResults: intraFreqMeasuredResultsList ( 0 )

intraFreqMeasuredResultsList

CellMeasuredResults

modeSpecificInfo: fdd ( 0 )

fdd

primaryCPICH-Info

cpich-Ec-N0

cpich-RSCP

pathloss

Expected Sequence 1.13 (PROVIDE LOCAL INFORMATION, Inter-frequency UTRAN Measurements)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.13.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.13.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.13.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.13.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	UICC

Destination device:	ME

UTRAN/E-UTRAN Measurement Qualifier

UTRAN/E-UTRAN Measurement Qualifier:  "02" Inter-frequency measurements

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 81 | 82 | 69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.13.1

The actual values of the measurements are not tested.

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Measurement Results	MEASUREMENT REPORT message

interFreqMeasuredResultsList

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 96 | Note 1 | 80 | Note 2 | Note 3 | Note 4 | Note 4 | Note 5 | Note 6 | Note 7 |  |  |

Note 1: This is the length indicator for the following bytes which represent the Measurement report coded in ASN.1 and therefore the length cannot be foreseen.

Note2: This byte shall be checked bitwise against pattern: 0001 xxx1 (x – don't care).

Note 3: This byte shall be checked bitwise against pattern: 1100 xxxx (x – don't care).

Note 4: This byte shall not be verified.

Note 5: This byte shall be checked bitwise against pattern: xxxx xx00 (x – don't care).

Note 6: This byte shall be checked bitwise against pattern: 0011 1xxx (x – don't care).

Note 7: The remaining bytes shall not be verified.

The network measurement result indicated by the sequence of bytes above is:

MeasurementReport

measurementIdentity

MeasuredResults: interFreqMeasuredResultsList InterFreqMeasuredResultsList ( 1 )

interFreqMeasuredResultsList

InterFreqMeasuredResults

frequencyInfo

utra-CarrierRSSI

interFreqCellMeasuredResultsList

CellMeasuredResults

modeSpecificInfo: fdd ( 0 )

fdd

primaryCPICH-Info

cpich-Ec-N0

cpich-RSCP

pathloss

Expected Sequence 1.14 (PROVIDE LOCAL INFORMATION, Access Technology, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.14.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.14.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.14.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Access Technology

Technology:	E-UTRAN

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3F | 01 | 08 |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.15 (PROVIDE LOCAL INFORMATION, E-UTRAN Intra-Frequency Measurements)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.15.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.15.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.15.1 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.15.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	UICC

Destination device:	ME

UTRAN/E-UTRAN Measurement Qualifier

UTRAN/E-UTRAN Measurement Qualifier:  "05" E-UTRAN Intra-frequency measurements

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 81 | 82 | 69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 05 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.15.1

The actual values of the measurements are not tested.

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Measurement Results	MEASUREMENT REPORT message

measResultNeighCells

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 96 | Note 1 | 02 | Note 2 | Note 3 | Note 4 | Note 5 |  |  |  |  |  |

Note 1:	This is the length indicator for the following bytes which represent the Measurement report coded in ASN.1 and therefore the length cannot be foreseen.

Note 2:	This byte shall be checked bitwise against pattern: 0000 xxxx (x – don't care).

Note 3:	This byte shall not be verified.

Note 4:	This byte shall be checked bitwise against pattern: x000 xxxx (x – don't care).

Note 5:	The remaining bytes shall not be verified.

The network measurement result indicated by the sequence of bytes above is: 

Network Measurement results:
          measurementReport
                criticalExtensions: c1 (0)
                    c1: measurementReport-r8 (0)
                        measurementReport-r8
                            measResults
                               … {Not Verified}
                               measResultNeighCells: 
                               … {Not Verified}

Expected Sequence 1.16 (PROVIDE LOCAL INFORMATION, E-UTRAN Inter-Frequency Measurements)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | Terminal is in RRC idle state |  |
| 2 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.16.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.16.1 |  |
| 5 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.16.1 | [Command performed successfully, limited service] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.16.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	UICC

Destination device:	ME

UTRAN/E-UTRAN Measurement Qualifier

UTRAN/E-UTRAN Measurement Qualifier:  "06" E-UTRAN Inter-frequency measurements

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 81 | 82 | 69 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 06 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.16.1

The actual values of the measurements are not tested.

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"02" Network Measurement Results

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Network Measurement Results	Frequency value of inter-frequency E-UTRAN cell and MEASUREMENT REPORT message

measResultNeighCells

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 96 | Note 1 | Note 2 | Note 2 | 02 | Note 3 | Note 4 | Note 5 | Note 6 |  |  |  |

Note 1: This is the length indicator for the following bytes which contain 2 bytes with the frequency value coded as the ARFCN-ValueEUTRA followed by the Measurement report coded in ASN.1 and therefore the length cannot be foreseen.

Note 2:	This is the frequency of the second E-UTRA cell, coded as ARFCN-ValueEUTRA. This byte shall not be verified.

Note 3:	This byte shall be checked bitwise against pattern: 0000 xxxx (x – don't care).

Note 4:	This byte shall not be verified.

Note 5:	This byte shall be checked bitwise against pattern: x000 xxxx (x – don't care).

Note 6: The remaining bytes shall not be verified.

Expected Sequence 1.17 (PROVIDE LOCAL INFORMATION, E-UTRAN Local Info (MCC, MNC, TAC & E-UTRAN Cell ID))

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.17.1 |  |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1

Sames as PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1 in expected sequence 1.1

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.17.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"00" Location information (MCC MNC TAC and E-UTRAN Cell Identity)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Location Information

MCC & MNC:		MCC = 001, MNC = 01

Tracking Area Code:		0001

E-UTRAN Cell Identifier:	0001 (28 bits)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 93 | 09 | 00 | F1 | 10 | 00 | 01 | 00 | 00 | 00 | 1F |  |

Expected Sequence 1.18 (PROVIDE LOCAL INFORMATION, Discovery of surrounding CSG cells)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | E-USS | Cell 1 is enabled, with csg-indication set to TRUE Cell 2 disabled |  |
| 2 | ME | A manual CSG cell selection is performed. |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.18.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.18.1 | 1 cell in the list |
| 6 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.18.1 | [Command performed successfully] |
| 7 | E-USS | Cell 2 is enabled, with csg-indication set to TRUE |  |
| 8 | ME | A manual CSG cell selection is performed. |  |
| 9 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.18.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.18.1 | 2 cells in the list |
| 12 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.18.2 | [Command performed successfully] |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.18.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"11" CSG ID list and corresponding HNB name

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 26 | 11 | 82 | 02 | 81 | 82 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.18.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"11" CSG ID list and corresponding HNB name

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

CSG ID list Identifier

PLMN					MCC = 001, MNC = 01

CSG ID and Name

CSG ID				01 (27 bits)

HNB name				Home ONE

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 11 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7E | 1C | 80 | 03 | 00 | F1 | 10 | 81 | 15 | 00 | 00 | 00 |
|  | 3F | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 | 20 |
|  | 00 | 4F | 00 | 4E | 00 | 45 |  |  |  |  |  |  |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.18.2

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"11" CSG ID list and corresponding HNB name

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

CSG ID list Identifier

PLMN	MCC = 001, MNC = 01

CSG ID and Name

CSG ID				01 (27 bits)

HNB name				Home ONE

CSG ID and Name

CSG ID				02 (27 bits)

HNB name				Home TWO

| BER-TLV: | 81 | 03 | 01 | 26 | 11 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 7E | 33 | 80 | 03 | 00 | F1 | 10 | 81 | 15 | 00 | 00 | 00 |
|  | 3F | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 | 20 |
|  | 00 | 4F | 00 | 4E | 00 | 45 | 81 | 15 | 00 | 00 | 00 | 5F |
|  | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 | 65 | 00 | 20 | 00 |
|  | 54 | 00 | 57 | 00 | 4F |  |  |  |  |  |  |  |

Expected Sequence 1.19 (PROVIDE LOCAL INFORMATION, Location Information for Multiple Access Technologies)

TBD

Expected Sequence 1.20 (PROVIDE LOCAL INFORMATION, NMR for Multiple Access Technologies)

TBD

Expected Sequence 1.21 (PROVIDE LOCAL INFORMATION, current access technologies, Multiple Access Technologies)

TBD

NOTE:	The above test sequences (1.19, 1.20, 1.21) on Multiple Access Technologies imply the support of one or more non-3GPP access technologies and therefore can not be tested within 3GPP.

Expected Sequence 1.22 (PROVIDE LOCAL INFORMATION, NG-RAN Local Info (MCC, MNC, TAC & NG-RAN Cell ID))

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.22.1 |  |

PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1

Sames as PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.1.1 in expected sequence 1.1

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.22.1

Logically:

Command details

Command number:			1

Command type:			PROVIDE LOCAL INFORMATION

Qualifier:			"00" Location information (MCC MNC TAC and NG-RAN Cell Identity)

Device identities

Source device:			ME

Destination device:			UICC

Result

General Result:			Command performed successfully

Location Information

MCC & MNC:			MCC = 001, MNC = 01

Tracking Area Code:			000001

NG-RAN Cell Identifier:	0001 (36 bits)

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 93 | 0B | 00 | F1 | 10 | 00 | 00 | 01 | 00 | 00 | 00 | 00 |
|  | 1F |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.23 (PROVIDE LOCAL INFORMATION, Access Technology, NG-RAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: PROVIDE LOCAL INFORMATION 1.14.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.23.1 | [Command performed successfully] |

TERMINAL RESPONSE: PROVIDE LOCAL INFORMATION 1.23.1

Logically:

Command details

Command number:	1

Command type:	PROVIDE LOCAL INFORMATION

Qualifier:	"06" Access Technology

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Access Technology

Technology:	3GPP NR

Coding:

| BER-TLV: | 81 | 03 | 01 | 26 | 06 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 3F | 01 | 0A |  |  |  |  |  |  |  |  |  |

##### 27.22.4.15.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.18.

#### 27.22.4.16	SET UP EVENT LIST

##### 27.22.4.16.1	SET UP EVENT LIST (normal)

27.22.4.16.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.16.1.2	Conformance requirement

The ME shall support the Proactive UICC: Set Up Event List facility as defined in:

-	TS 31.111 [15] clause 6.4.16 and clause 6.6.16.

Additionally the ME shall support the Event Download: Call Connect and the Event Download: Call Disconnected mechanism as defined in:

-	TS 31.111 [15] clause 11.2, clause 11.2.1, clause 11.2.2, clause 11.3, clause 11.3.1 and clause 11.3.2.

27.22.4.16.1.3	Test purpose

To verify that the ME accepts a list of events that it shall monitor the current list of events supplied by the UICC, is able to have this current list of events replaced and is able to have the list of events removed.

To verify that when the ME has successfully accepted or removed the list of events, it shall send TERMINAL RESPONSE (OK) to the UICC and when the ME is not able to successfully accept or remove the list of events, it shall send TERMINAL RESPONSE (Command beyond ME's capabilities).

27.22.4.16.1.4	Method of test

27.22.4.16.1.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default with the following exceptions.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.16.1.4.2	Procedure

Expected Sequence 1.1 (SET UP EVENT LIST, Set Up Call Connect Event)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 6 | USS ME | SETUP 1.1.1 | [Incoming call alert] |
| 7 | USER ME | User shall accept the incoming call |  |
| 8 | ME USS | CONNECT 1.1.1 |  |
| 9 | ME UICC | ENVELOPE: EVENT DOWNLOAD CALL CONNECTED 1.1.1 | [Call Connected Event] |
| 10 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SET UP 1.1.1

Logically:

Transaction identifier

TI value:	0 (bit 5-7)

Address

TON:	"Unknown"

NPI:	"ISDN/ telephone numbering plan"

Dialling number string:	"9876"

CONNECT 1.1.1

Logically:

Transaction identifier

TI value:	0 (bit 5-7)

TI flag:	1 (bit 8)

ENVELOPE: EVENT DOWNLOAD CALL CONNECTED 1.1.1

Logically

Event list

Event 1:	Call Connected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 99 | 01 | 01 | 82 | 02 | 82 | 81 | 9C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (SET UP EVENT LIST, Replace Event)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 | [Call Connected and Call Disconnected Events] |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.2.2 |  |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.2 | [Call Disconnected Event] |
| 8 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.2 |  |
| 9 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 10 | USS ME | SETUP 1.2.2 | [Incoming call alert] |
| 11 | USER ME | User shall accept the incoming call |  |
| 12 | ME USS | CONNECT 1.2.2 |  |
| 13 | USS ME | DISCONNECT 1.2.2 |  |
| 14 | ME UICC | ENVELOPE: EVENT DOWNLOAD CALL DISCONNECT 1.2.2A or ENVELOPE: EVENT DOWNLOAD CALL DISCONNECT 1.2.2B | [Call Disconnect Event] |
| 15 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Event 2:	Call Disconnected

Coding:

| BER-TLV: | D0 | 0D | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 01 | 02 |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: SET UP EVENT LIST 1.2.2

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Disconnected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.2.2

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SET UP 1.2.2

Logically:

Transaction identifier

TI value:	0 (bit 5-7)

TI flag:	0 (bit 8)

Address

TON:	"Unknown"

NPI:	"ISDN/ telephone numbering plan"

Dialling number string:	"9876"

CONNECT 1.2.2

Logically:

Transaction identifier

TI value:	0 (bit 5-7)

TI flag:	1 (bit 8)

DISCONNECT 1.2.2

Logically:

Transaction identifier

TI value:	0 (bit 5-7)

TI flag:	0 (bit 8)

Cause

Value:	Normal call clearing

ENVELOPE: EVENT DOWNLOAD CALL DISCONNECTED 1.2.2A

Logically:

Event list

Event 1:	Call Disconnected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Cause

Value:	Normal call clearing

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 02 | 82 | 02 | 83 | 81 | 9C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 02 | 60 | 90 |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD CALL DISCONNECTED 1.2.2B

Logically:

Event list

Event 1:	Call Disconnected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Cause

Value:	Normal call clearing

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 02 | 82 | 02 | 83 | 81 | 9C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 02 | E0 | 90 |  |  |  |  |  |  |  |  |

Expected Sequence 1.3 (SET UP EVENT LIST, Remove Event)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.3.1 | [Call Connected Event] |
|  | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.3.1 |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.3.2 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.3.2 | [Remove Event] |
| 7 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.3.2 |  |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 10 | USS ME | SETUP 1.3.2 | [Incoming call alert] |
| 11 | USER ME | User shall accept the incoming call |  |
| 12 | ME USS | CONNECT 1.3.2 |  |
| 13 | ME UICC | No ENVELOPE: EVENT DOWNLOAD (call connected) sent |  |
| 14 | USS ME | DISCONNECT 1.3.2 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: SET UP EVENT LIST 1.3.2

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list:	Empty

Coding:

| BER-TLV: | D0 | 0B | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.3.2

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SET UP 1.3.2

Logically:

Transaction identifier

Ti value:	0 (bit 5-7)

Ti flag:	0 (bit 8)

Address

TON:	"Unknown"

NPI:	"ISDN/ telephone numbering plan"

Dialling number string:	"9876"

CONNECT 1.3.2

Logically:

Transaction identifier

Ti value:	0 (bit 5-7)

Ti flag:	1 (bit 8)

DISCONNECT 1.3.2

Logically:

Transaction identifier

Ti value:	0 (bit 5-7)

Ti flag:	0 (bit 8)

Cause

Value:	Normal call clearing

Expected Sequence 1.4 (SET UP EVENT LIST, Remove Event on ME Power Cycle)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.4.1 | [Call Connected Event] |
|  | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.4.1 |  |
| 4 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 5 | User ME | Power off ME |  |
| 6 | User ME | Power on ME |  |
| 7 | USS ME | SETUP 1.4.1 | [Incoming call alert] |
| 8 | USER ME | User shall accept the incoming call |  |
| 9 | ME USS | CONNECT 1.4.1 |  |
| 10 | ME UICC | No ENVELOPE: EVENT DOWNLOAD (call connected) sent |  |
| 11 | USS ME | DISCONNECT 1.4.1 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SET UP 1.4.1

Logically:

Transaction identifier

Ti value:	0 (bit 5-7)

Ti flag:	0 (bit 8)

Address

TON:	"Unknown"

NPI:	"ISDN/ telephone numbering plan"

Dialling number string:	"9876"

CONNECT 1.4.1

Logically:

Transaction identifier

Ti value:	0 (bit 5-7)

Ti flag:	1 (bit 8)

DISCONNECT 1.4.1

Logically:

Transaction identifier

Ti value:	0 (bit 5-7)

Ti flag:	0 (bit 8)

Cause

Value:	Normal call clearing

27.22.4.16.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.4.

#### 27.22.4.17	PERFORM CARD APDU

##### 27.22.4.17.1	PERFORM CARD APDU (normal)

27.22.4.17.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.17.1.2	Conformance requirement

The ME shall support the Proactive UICC: Perform Card APDU facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 5.2, clause 6.4.17, clause 6.6.17, clause 6.8, clause 8.6, clause 8.7, clause 8.35, clause 8.36 and clause 8.12.9.

Additionally the ME shall support multiple card operation as defined in:

-	TS 31.111 [15] clause 6.4.19, clause 6.6.19, clause 6.4.18 and clause 6.6.18.

27.22.4.17.1.3	Test purpose

To verify that the ME sends an APDU command to the additional card identified in the PERFORM CARD APDU proactive UICC command, and successfully returns the result of the execution of the command in the TERMINAL RESPONSE command send to the UICC.

The ME-Manufacturer can assign the card reader identifier from 0 to 7.

This test applies for MEs with only one additional card reader.

In this particular case the card reader identifier 1 is chosen.

In this particular case a special Test-SIM (TestSIM) with T=0 protocol is chosen as additional card for the additional ME card reader (for coding of the TestSIM see annex A).

27.22.4.17.1.4	Method of test

27.22.4.17.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The TestSIM is inserted in the additional ME card reader.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

If the ME supports a detachable card reader, the card reader shall be attached to the ME.

The elementary files of the TestSIM are coded as defined in annex A. Another card with different parameters may be used as TestSIM to execute these tests. In this case the USIM Simulator shall take into account the corresponding response data.

27.22.4.17.1.4.2	Procedure

Expected Sequence 1.1 (PERFORM CARD APDU, card reader 1, additional card inserted, Select MF and Get Response)

See ETSI TS 102 384 [26] in subclause 27.22.4.17.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (PERFORM CARD APDU, card reader 1, additional card inserted, Select DF GSM, Select EF PLMN , Update Binary, Read Binary on EF PLMN)

See ETSI TS 102 384 [26] in subclause 27.22.4.17.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (PERFORM CARD APDU, card reader 1, card inserted, card powered off)

See ETSI TS 102 384 [26] in subclause 27.22.4.17.1.4.2, Expected Sequence 1.3.

Expected Sequence 1.4 (PERFORM CARD APDU, card reader 1, no card inserted)

See ETSI TS 102 384 [26] in subclause 27.22.4.17.1.4.2, Expected Sequence 1.4.

Expected Sequence 1.5 (PERFORM CARD APDU, card reader 7 (which is not the valid card reader identifier of the additional ME card reader))

See ETSI TS 102 384 [26] in subclause 27.22.4.17.1.4.2, Expected Sequence 1.5.

27.22.4.17.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.5.

##### 27.22.4.17.2	PERFORM CARD APDU (detachable card reader)

27.22.4.17.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.17.2.2	Conformance requirement

27.22.4.17.2.3	Test purpose

To verify that the ME sends an APDU command to the additional card identified in the PERFORM CARD APDU proactive UICC command, and successfully returns the result of the execution of the command in the TERMINAL RESPONSE command send to the UICC.

27.22.4.17.2.4	Method of test

27.22.4.17.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The card reader shall be detached from the ME.

27.22.4.17.2.4.2	Procedure

Expected Sequence 2.1 (PERFORM CARD APDU, card reader 1, card reader detached)

See ETSI TS 102 384 [26] in subclause 27.22.4.17.2.4.2, Expected Sequence 2.1.

27.22.4.17.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

#### 27.22.4.18	POWER OFF CARD

##### 27.22.4.18.1	POWER OFF CARD (normal)

27.22.4.18.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.18.1.2	Conformance requirement

The ME shall support the Proactive UICC: Power Off Card facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.18, clause 6.6.18, clause 8.6, clause 8.7, clause 8.12, clause 8.12.9, clause 5.2 and annex H.

27.22.4.18.1.3	Test purpose

To verify that the ME closes a session with the additional card identified in the POWER OFF CARD proactive UICC command, and successfully returns result in the TERMINAL RESPONSE command send to the UICC.

The ME-Manufacturer can assign the card reader identifier from 0 to 7.

This test applies for MEs with only one additional card reader.

In this particular case the card reader identifier 1 is chosen.

27.22.4.18.1.4	Method of test

27.22.4.18.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The ME card reader is connected to aSIM Simulator (SIM2). Instead of a SIM Simulator a card with different parameters may be used as SIM2 to execute these tests. In this case the USIM Simulator shall take into account the corresponding response data.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

If the ME supports a detachable card reader, the card reader shall be attached to the ME.

Prior to this test the ME shall have powered on the SIM Simulator (SIM2).

27.22.4.18.1.4.2	Procedure

Expected Sequence 1.1 (POWER OFF CARD, card reader 1)

See ETSI TS 102 384 [26] in subclause 27.22.4.18.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (POWER OFF CARD, card reader 1, no card inserted)

See ETSI TS 102 384 [26] in subclause 27.22.4.18.1.4.2, Expected Sequence 1.2.

27.22.4.18.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.2.

##### 27.22.4.18.2	POWER OFF CARD (detachable card reader)

27.22.4.18.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.18.2.2	Conformance requirement

Void.

27.22.4.18.2.3	Test purpose

To verify that the ME closes a session with the additional card identified in the POWER OFF CARD proactive UICC command, and successfully returns result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.18.2.4	Method of test

27.22.4.18.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The ME card reader is connected to a SIM Simulator (SIM2).

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to this test the ME shall have powered on the SIM Simulator (SIM2).

The card reader shall be detached from the ME.

27.22.4.18.2.4.2	Procedure

Expected Sequence 2.1 (POWER OFF CARD, card reader 1, no card reader attached)

See ETSI TS 102 384 [26] in subclause 27.22.4.18.2.4.2, Expected Sequence 2.1.

27.22.4.18.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

#### 27.22.4.19	POWER ON CARD

##### 27.22.4.19.1	POWER ON CARD (normal)

27.22.4.19.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.19.1.2	Conformance requirement

The ME shall support the Proactive UICC: Power On Card facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.19, clause 6.6.19, clause 8.6, clause 8.7, clause 8.12, clause 8.12.9, clause 8.34, clause 5.2 and annex H.

27.22.4.19.1.3	Test purpose

To verify that the ME starts a session with the additional card identified in the POWER ON CARD proactive UICC command, and successfully returns the Answer To Reset within the TERMINAL RESPONSE command send to the UICC.

The ME-Manufacturer can assign the card reader identifier from 0 to 7.

This test applies for MEs with only one additional card reader.

In this particular case the card reader identifier 1 is chosen.

27.22.4.19.1.4	Method of test

27.22.4.19.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The ME card reader is connected to a SIM Simulator (SIM2). Instead of the SIM Simulator a card with different parameters may be used as SIM2 to execute these tests. In this case the USIM Simulator shall take into account the corresponding response data.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

If the ME supports a detachable card reader, the card reader shall be attached to the ME.

27.22.4.19.1.4.2	Procedure

Expected Sequence 1.1 (POWER ON CARD, card reader 1)

See ETSI TS 102 384 [26] in subclause 27.22.4.19.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (POWER ON CARD, card reader 1, no ATR)

See ETSI TS 102 384 [26] in subclause 27.22.4.19.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (POWER ON CARD, card reader 1, no card inserted)

See ETSI TS 102 384 [26] in subclause 27.22.4.19.1.4.2, Expected Sequence 1.3.

27.22.4.19.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.3.

##### 27.22.4.19.2	POWER ON CARD (detachable card reader)

27.22.4.19.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.19.2.2	Conformance requirement

27.22.4.19.2.3	Test purpose

To verify that the ME starts a session with the additional card identified in the POWER ON CARD proactive UICC command, and successfully returns the Answer To Reset within the TERMINAL RESPONSE command send to the UICC.

27.22.4.19.2.4	Method of test

27.22.4.19.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default with the following exceptions.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The card reader shall be detached from the ME.

27.22.4.19.2.4.2	Procedure

Expected Sequence 2.1 (POWER ON CARD, card reader 1, no card reader attached)

See ETSI TS 102 384 [26] in subclause 27.22.4.19.2.4.2, Expected Sequence 2.1.

27.22.4.19.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

#### 27.22.4.20	GET READER STATUS

##### 27.22.4.20.1	GET READER STATUS (normal)

27.22.4.20.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.20.1.2	Conformance requirement

The ME shall support the Proactive UICC: Get Card Reader Status facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 5.2, clause 6.4.20, clause 6.6.20, clause 6.8, clause 8.6, clause 8.7, clause 8.33, clause 8.57 and annex H.

Additionally the ME shall support multiple card operation as defined in:

-	TS 31.111 [15] clause 6.4.19, clause 6.6.19, clause 6.4.18 and clause 6.6.18.

27.22.4.20.1.3	Test purpose

To verify that the ME sends starts a session with the additional card identified in the GET CARD READER STATUS proactive UICC command, and successfully returns information about all interfaces to additional card reader(s) in the TERMINAL RESPONSE command send to the UICC.

The ME-Manufacturer can assign the card reader identifier from 0 to 7.

This test applies for MEs with only one additional card reader.

In this particular case the card reader identifier 1 is chosen.

In this test case the SIM-Simulator (SIM2) shall response with the ATR "3B 00".

27.22.4.20.1.4	Method of test

27.22.4.20.1.4.1	Initial conditions

The ME shall support the Proactive UICC: Get Card Reader Status (Card Reader Status) facility. The ME is connected to the USIM Simulator.

The ME card reader is connected to a SIM Simulator (SIM2). Instead of the SIM Simulator a card with different parameters may be used as SIM2 to execute these tests. In this case the USIM Simulator shall take into account the corresponding response data.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

If the ME supports a detachable card reader, the card reader shall be attached to the ME.

Prior to this test the ME shall have powered on the SIM Simulator (SIM2).

27.22.4.20.1.4.2	Procedure

Expected Sequence 1.1 (GET CARD READER STATUS, card reader 1, card inserted, card powered)

See ETSI TS 102 384 [26] in subclause 27.22.4.20.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (GET CARD READER STATUS, card reader 1, card inserted, card not powered)

See ETSI TS 102 384 [26] in subclause 27.22.4.20.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (GET CARD READER STATUS, card reader 1, card not present)

See ETSI TS 102 384 [26] in subclause 27.22.4.20.1.4.2, Expected Sequence 1.3.

27.22.4.20.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.3.

##### 27.22.4.20.2	GET CARD READER STATUS (detachable card reader)

27.22.4.20.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.20.2.2	Conformance requirement

Void.

27.22.4.20.2.3	Test purpose

To verify that the ME closes a session with the additional card identified in the GET CARD READER STATUS proactive UICC command, and successfully returns result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.20.2.4	Method of test

27.22.4.20.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to this test the ME shall have powered on the SIM Simulator (SIM2).

The card reader shall be detached from the ME.

27.22.4.20.2.4.2	Procedure

Expected Sequence 2.1 (GET CARD READER STATUS, no card reader attached)

See ETSI TS 102 384 [26] in subclause 27.22.4.20.2.4.2, Expected Sequence 2.1.

27.22.4.20.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 2.1.

#### 27.22.4.21	TIMER MANAGEMENT and ENVELOPE TIMER EXPIRATION

##### 27.22.4.21.1	TIMER MANAGEMENT (normal)

27.22.4.21.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.21.1.2	Conformance Requirement

The ME shall support the TIMER MANAGEMENT as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.21, clause 6.8, clause 8.6, clause 8.7, clause 8.37 and clause 8.38.

27.22.4.21.1.3	Test purpose

To verify that the ME manages correctly its internal timers, start a timer, deactivate a timer or return the current value of a timer according to the Timer Identifier defined in the TIMER MANAGEMENT proactive UICC command.

27.22.4.21.1.4	Method of Test

27.22.4.21.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default with the following exceptions.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.4.21.1.4.2	Procedure

Expected Sequence 1.1 (TIMER MANAGEMENT, start timer 1 several times, get the current value of the timer and deactivate the timer successfully)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (TIMER MANAGEMENT, start timer 2 several times, get the current value of the timer and deactivate the timer successfully)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (TIMER MANAGEMENT, start timer 8 several times, get the current value of the timer and deactivate the timer successfully)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.1.4.2, Expected Sequence 1.3.

Expected Sequence1.4 (TIMER MANAGEMENT, try to get the current value of a timer which is not started: action in contradiction with the current timer state)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.1.4.2, Expected Sequence 1.4.

Expected Sequence1.5 (TIMER MANAGEMENT, try to deactivate a timer which is not started: action in contradiction with the current timer state)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.1.4.2, Expected Sequence 1.5.

Expected Sequence 1.6 (TIMER MANAGEMENT, start 8 timers successfully)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.1.4.2, Expected Sequence 1.6.

27.22.4.21.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.6.

##### 27.22.4.21.2	ENVELOPE TIMER EXPIRATION (normal)

27.22.4.21.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.21.2.2	Conformance requirement

The ME shall support the ENVELOPE (TIMER EXPIRATION) command as defined in the following technical specifications:

-	TS 31.111 [15] clause 4.10, clause 7.4.1 and clause 7.4.2.

The ME shall support the TIMER MANAGEMENT as defined in the following technical specifications:

-	TS 31.111 [15] clause 5.2, clause 6.4.21, clause 6.8, clause 8.6, clause 8.7, clause 8.37 and clause 8.38.

27.22.4.21.2.3	Test purpose

To verify that the ME shall pass the identifier of the timer that has expired and its value using the ENVELOPE (TIMER EXPIRATION) command, when a timer previously started in a TIMER MANAGEMENT proactive command expires.

27.22.4.21.2.4	Method of test

27.22.4.21.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default with the following exceptions.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The timer 1 is not started.

When the UICC is busy when the envelope TIMER EXPIRATION is sent, either the ME retries periodically to send the envelope or it waits for a status not indicating busy.

27.22.4.21.2.4.2	Procedure

Expected Sequence 2.1 (TIMER EXPIRATION, pending proactive UICC command)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.2.4.2, Expected Sequence 2.1.

Expected Sequence 2.2 (TIMER EXPIRATION, UICC application toolkit busy)

See ETSI TS 102 384 [26] in subclause 27.22.4.21.2.4.2, Expected Sequence 2.2.

27.22.4.21.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1 to 2.2.

#### 27.22.4.22	SET UP IDLE MODE TEXT

##### 27.22.4.22.1	SET UP IDLE MODE TEXT (normal)

27.22.4.22.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.1.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 6.4.7 and clause 6.6.13.

Additionally the ME shall support the REFRESH proactive UICC facility as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.1, clause 6.4.7, clause 6.6.13, clause 6.11, clause 8.6, clause 8.7, clause 8.12, clause 9.4 and clause 10.

27.22.4.22.1.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text.

27.22.4.22.1.4	Method of test

27.22.4.22.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.22.1.4.2	Procedure

Expected Sequence 1.1 (SET UP IDLE MODE TEXT, display idle mode text)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.1.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (SET UP IDLE MODE TEXT, replace idle mode text)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.1.4.2, Expected Sequence 1.2.

Expected Sequence 1.3 (SET UP IDLE MODE TEXT, remove idle mode text)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.1.4.2, Expected Sequence 1.3.

Expected Sequence 1.4 (SET UP IDLE MODE TEXT, competing information on ME display)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP IDLE MODE TEXT 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP IDLE MODE TEXT 1.1.1 | ["Idle Mode Text"] |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP IDLE MODE TEXT 1.1.1 | [Command performed successfully] |
| 5 | USER ME | Select idle screen | Only if idle screen not already available |
| 6 | ME USER | Display "Idle Mode Text" |  |
| 7 | USS ME | SMS PP 1.4.1 | [Display immediate SMS] |
| 8 | ME USER | Display "Test Message" |  |
| 9 | USER ME | Clear display and select idle screen |  |
| 10 | ME USER | Display "Idle Mode Text" |  |
| 11 | UICC ME | PROACTIVE COMMAND PENDING: DISPLAY TEXT 1.4.1 |  |
| 12 | ME UICC | FETCH |  |
| 13 | UICC ME | PROACTIVE COMMAND: DISPLAY TEXT 1.4.1 | [Normal priority, wait for user to clear message, unpacked, 8 bit data] |
| 14 | ME USER | Display "Toolkit Test 1" |  |
| 15 | USER ME | Clear Message |  |
| 16 | ME UICC | TERMINAL RESPONSE: DISPLAY TEXT 1.4.1 | [Command performed successfully] |
| 17 | ME USER | Display "Idle Mode Text" |  |
| 18 | UICC ME | PROACTIVE COMMAND PENDING: PLAY TONE 1.4.1 |  |
| 19 | ME UICC | FETCH |  |
| 20 | UICC ME | PROACTIVE COMMAND: PLAY TONE 1.4.1 |  |
| 21 | ME USER | Display "Dial Tone" Play a standard supervisory dial tone through the external ringer for a duration of 5 s |  |
| 22 | ME UICC | TERMINAL RESPONSE: PLAY TONE 1.4.1 | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 24 | ME USER | Display "Idle Mode Text" |  |

SMS-PP 1.4.1

Logically:

SMS TPDU

TP-MTI			SMS-DELIVER

TP-MMS			No more messages waiting for the MS in this SC

TP-RP				TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI			TP-UD field contains only the short message

TP-SRI			A status report will not be returned to the ME

TP-OA

TON			International number

NPI			"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID			"00"

TP-DCS

Coding Group	General Data Coding

Compression		Text is uncompressed

Message Class	Class 0

Alphabet		GSM 7 bit default alphabet

TP-SCTS:			01/01/98 00:00:00 +0

TP-UDL			12

TP-UD			"Test Message"

Coding:

| Coding | 04 | 04 | 91 | 21 | 43 | 00 | 10 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0C | D4 | F2 | 9C | 0E | 6A | 96 | E7 | F3 | F0 |
|  | B9 | 0C |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: DISPLAY TEXT 1.4.1

Logically:

Command details

Command number:	1

Command type:	DISPLAY TEXT

Command qualifier:	normal priority, wait for user to clear message

Device identities

Source device:	UICC

Destination device:	Display

Text String

Data coding scheme:	unpacked, 8 bit data

Text:	"Toolkit Test 1"

Coding:

| BER-TLV: | D0 | 1A | 81 | 03 | 01 | 21 | 80 | 82 | 02 | 81 | 02 | 8D |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0F | 04 | 54 | 6F | 6F | 6C | 6B | 69 | 74 | 20 | 54 | 65 |
|  | 73 | 74 | 20 | 31 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: DISPLAY TEXT 1.4.1

Logically:

Command details

Command number:	1

Command type:	DISPLAY TEXT

Command qualifier:	normal priority, wait for user to clear message

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 21 | 80 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: PLAY TONE 1.4.1

Logically:

Command details

Command number:	1

Command type:	PLAY TONE

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Earpiece

Alpha identifier:	"Dial Tone"

TONe:	Standard supervisory tones: dial tone

Duration

Time unit:	Seconds

Time interval:	5

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 20 | 00 | 82 | 02 | 81 | 03 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 44 | 69 | 61 | 6C | 20 | 54 | 6F | 6E | 65 | 8E | 01 |
|  | 01 | 84 | 02 | 01 | 05 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: PLAY TONE 1.4.1

Logically:

Command details

Command number:	1

Command type:	PLAY TONE

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 20 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.5 (SET UP IDLE MODE TEXT, ME power cycled)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP IDLE MODE TEXT 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP IDLE MODE TEXT 1.1.1 | ["Idle Mode Text"] |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP IDLE MODE TEXT 1.1.1 | [command performed successfully] |
| 5 | USER ME | Select idle screen | Only if idle screen not already available |
| 6 | ME USER | Display "Idle Mode Text" |  |
| 7 | USER ME | Power off ME |  |
| 8 | ME UICC | 3G Session TERMINATION PROCEDURE |  |
| 9 | USER ME | Power on ME |  |
| 10 | ME UICC | 3G Session ACTIVATION PROCEDURE |  |
| 11 | ME UICC | USIM INITIALIZATION |  |
| 12 | USER ME | Select idle screen | Only if idle screen not already available |
| 13 | ME USER | Display idle screen / "Idle Mode Text" not to be displayed |  |

Expected Sequence 1.6 (SET UP IDLE MODE TEXT, REFRESH with USIM Initialization)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP IDLE MODE TEXT 1.1.1 | [Idle Mode Text] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP IDLE MODE TEXT 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP IDLE MODE TEXT 1.1.1 |  |
| 5 | USER ME | Select idle screen | Only if idle screen not already available |
| 6 | ME USER | Display "Idle Mode Text" |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.6.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: REFRESH 1.6.1 | [USIM Initialization] |
| 10 | ME UICC | USIM INITIALIZATION |  |
| 11 | USER ME | Select idle screen | Only if idle screen not already available |
| 12 | ME USER | Display idle screen / "Idle Mode Text" not to be displayed |  |
| 13 | ME UICC | TERMINAL RESPONSE: REFRESH 1.6.1A or TERMINAL RESPONSE: REFRESH 1.6.1B | [Command performed successfully] [Command performed successfully with additional files read] |
| 14 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: REFRESH 1.6.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.6.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.6.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.7 (SET UP IDLE MODE TEXT, large text string)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.1.4.2, Expected Sequence 1.7.

27.22.4.22.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.

##### 27.22.4.22.2	SET UP IDLE MODE TEXT (Icon support)

27.22.4.22.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.2.2	Conformance requirement

27.22.4.22.2.3	Test purpose

To verify that the ME text and / or icon passed to the ME is displayed by the ME as an idle mode text.

To verify that the icon identifier provided with the text string can replace the text string or accompany it.

To verify that if both an alpha identifier or text string, and an icon are provided with a proactive command, and both are requested to be displayed, but the ME is not able to display both together on the screen, then the alpha identifier or text string takes precedence over the icon.

To verify that if the UICC provides an icon identifier with a proactive command, then the ME shall inform the UICC if the icon could not be displayed by sending the general result "Command performed successfully, but requested icon could not be displayed".

To verify that if the ME receives an icon identifier with a proactive command, and either an empty, or no alpha identifier / text string is given by the UICC, than the ME shall reject the command with general result "Command data not understood by ME".

27.22.4.22.2.4	Method of test

27.22.4.22.2.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in update idle mode on the System Simulator.

27.22.4.22.2.4.2	Procedure

Expected Sequence 2.1A (SET UP IDLE MODE TEXT, Icon is self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.1A.

Expected Sequence 2.1B (SET UP IDLE MODE TEXT, Icon is self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.1B.

Expected Sequence 2.2A (SET UP IDLE MODE TEXT, Icon is not self-explanatory, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.2A.

Expected Sequence 2.2B (SET UP IDLE MODE TEXT, Icon is not self-explanatory, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.2B.

Expected Sequence 2.3A (SET UP IDLE MODE TEXT, Icon is self-explanatory, colour icon, successful)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.3A.

Expected Sequence 2.3B (SET UP IDLE MODE TEXT, Icon is self-explanatory, colour icon, requested icon could not be displayed)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.3B.

Expected Sequence 2.4 (SET UP IDLE MODE TEXT, Icon is not self-explanatory, empty text string)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.2.4.2, Expected Sequence 2.4.

27.22.4.22.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1A to 2.4.

##### 27.22.4.22.3	SET UP IDLE MODE TEXT (UCS2 support)

27.22.4.22.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.3.2	Conformance requirement

The ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.22.3.3	Test purpose

To verify that the UCS2 coded text string is displayed by the ME as an idle mode text.

27.22.4.22.3.4	Method of test

27.22.4.22.3.4.1	Initial conditions

The ME is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in update idle mode on the System Simulator..

27.22.4.22.3.4.2	Procedure

Expected Sequence 3.1 (SET UP IDLE MODE TEXT, UCS2 alphabet text)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.3.4.2, Expected Sequence 3.1.

27.22.4.22.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.22.4	SET UP IDLE MODE TEXT (support of Text Attribute)

27.22.4.22.4.1	SET UP IDLE MODE TEXT (support of Text Attribute – Left Alignment)

27.22.4.22.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.1.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.1.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the left alignment text attribute configuration.

27.22.4.22.4.1.4	Method of test

27.22.4.22.4.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.1.4.2	Procedure

Expected Sequence 4.1 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Left Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.1.4.2, Expected Sequence 4.1.

27.22.4.22.4.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

27.22.4.22.4.2	SET UP IDLE MODE TEXT (support of Text Attribute – Center Alignment)

27.22.4.22.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.2.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.2.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the center alignment text attribute configuration.

27.22.4.22.4.2.4	Method of test

27.22.4.22.4.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.2.4.2	Procedure

Expected Sequence 4.2 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Center Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.2.4.2, Expected Sequence 4.2.

27.22.4.22.4.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.2.

27.22.4.22.4.3	SET UP IDLE MODE TEXT (support of Text Attribute – Right Alignment)

27.22.4.22.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.3.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.3.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the right alignment text attribute configuration.

27.22.4.22.4.3.4	Method of test

27.22.4.22.4.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.3.4.2	Procedure

Expected Sequence 4.3 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Right Alignment)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.3.4.2, Expected Sequence 4.3.

27.22.4.22.4.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.3.

27.22.4.22.4.4	SET UP IDLE MODE TEXT (support of Text Attribute – Large Font Size)

27.22.4.22.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.4.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.4.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the large font size text attribute configuration.

27.22.4.22.4.4.4	Method of test

27.22.4.22.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.4.4.2	Procedure

Expected Sequence 4.4 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Large Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.4.4.2, Expected Sequence 4.4.

27.22.4.22.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.4.

27.22.4.22.4.5	SET UP IDLE MODE TEXT (support of Text Attribute – Small Font Size)

27.22.4.22.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.5.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.5.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the small font size text attribute configuration.

27.22.4.22.4.5.4	Method of test

27.22.4.22.4.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.5.4.2	Procedure

Expected Sequence 4.5 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Small Font Size)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.5.4.2, Expected Sequence 4.5.

27.22.4.22.4.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.5.

27.22.4.22.4.6	SET UP IDLE MODE TEXT (support of Text Attribute – Bold On)

27.22.4.22.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.6.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.6.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the bold text attribute configuration.

27.22.4.22.4.6.4	Method of test

27.22.4.22.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.6.4.2	Procedure

Expected Sequence 4.6 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Bold On)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.6.4.2, Expected Sequence 4.6.

27.22.4.22.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.6.

27.22.4.22.4.7	SET UP IDLE MODE TEXT (support of Text Attribute – Italic On)

27.22.4.22.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.7.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.7.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the italic text attribute configuration.

27.22.4.22.4.7.4	Method of test

27.22.4.22.4.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.7.4.2	Procedure

Expected Sequence 4.7 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Italic On)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.7.4.2, Expected Sequence 4.7.

27.22.4.22.4.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.7.

27.22.4.22.4.8	SET UP IDLE MODE TEXT (support of Text Attribute – Underline On)

27.22.4.22.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.8.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.8.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the underline text attribute configuration.

27.22.4.22.4.8.4	Method of test

27.22.4.22.4.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.8.4.2	Procedure

Expected Sequence 4.8 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Underline On)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.8.4.2, Expected Sequence 4.8.

27.22.4.22.4.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.8.

27.22.4.22.4.9	SET UP IDLE MODE TEXT (support of Text Attribute – Strikethrough On)

27.22.4.22.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.9.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.9.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the strikethrough text attribute configuration.

27.22.4.22.4.9.4	Method of test

27.22.4.22.4.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.9.4.2	Procedure

Expected Sequence 4.9 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Strikethrough On)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.9.4.2, Expected Sequence 4.9.

27.22.4.22.4.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.9.

27.22.4.22.4.10	SET UP IDLE MODE TEXT (support of Text Attribute – Foreground and Background Colour)

27.22.4.22.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.4.10.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

27.22.4.22.4.10.3	Test purpose

To verify that the text passed to the ME is displayed as idle mode text according to the foreground and background colour text attribute configuration.

27.22.4.22.4.10.4	Method of test

27.22.4.22.4.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.22.4.10.4.2	Procedure

Expected Sequence 4.10 (SET UP IDLE MODE TEXT, display idle mode text, Text Attribute – Foreground and Background Colour)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.4.10.4.2, Expected Sequence 4.10.

27.22.4.22.4.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.10.

##### 27.22.4.22.5	SET UP IDLE MODE TEXT (UCS2 display in Chinese)

27.22.4.22.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.5.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

The Terminal shall additionally support the UCS2 facility for the coding of the Chinese character, as defined in:

ISO/IEC 10646 [17a/17b].

27.22.4.22.5.3	Test purpose

To verify that the UCS2 coded text string is displayed by the ME as an idle mode text.

27.22.4.22.5.4	Method of test

27.22.4.22.5.4.1	Initial conditions

The Terminal is connected to both the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the Terminal shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.22.5.4.2	Procedure

Expected Sequence 5.1 (SET UP IDLE MODE TEXT, UCS2 alphabet text in Chinese)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.5.4.2, Expected Sequence 5.1.

27.22.4.22.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.22.6	SET UP IDLE MODE TEXT (UCS2 display in Katakana)

27.22.4.22.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.22.6.2	Conformance requirement

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.22, clause 6.6.22, clause 6.4.16, clause 6.6.16, clause 7.5.6, clause 6.8, clause 7.5, clause 7.5.1, clause 8.25, clause 8.70, clause 6.4.7 and clause 6.6.13.

The ME shall additionally support the UCS2 facility for the coding of the Katakana character, as defined in:

ISO/IEC 10646 [17a/17b].

27.22.4.22.6.3	Test purpose

To verify that the UCS2 coded text string is displayed by the ME as an idle mode text.

27.22.4.22.6.4	Method of test

27.22.4.22.6.4.1	Initial conditions

The ME is connected to both the UICC Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.22.6.4.2	Procedure

Expected Sequence 6.1 (SET UP IDLE MODE TEXT, UCS2 alphabet text in Katakana)

See ETSI TS 102 384 [26] in subclause 27.22.4.22.6.4.2, Expected Sequence 6.1.

27.22.4.22.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

#### 27.22.4.23	RUN AT COMMAND

##### 27.22.4.23.1	RUN AT COMMAND (normal)

27.22.4.23.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.1.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31 and clause 8.41.

-	TS 27.007 [18].

27.22.4.23.1.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.1.4	Method of test

27.22.4.23.1.4.1	Initial conditions

The ME is connected to the USIM Simulator. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.1.4.2	Procedure

Expected Sequence 1.1(RUN AT COMMAND, no alpha identifier presented, request IMSI)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 1.1.1 | [no alpha identifier, request IMSI] |
| 4 | ME ( User) | The ME may give information to the user concerning what is happening |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 1.1.1 | [Command performed successfully, AT Response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 1.1.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 1.1.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.2 (RUN AT COMMAND, null data alpha identifier presented, request IMSI)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 1.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 1.2.1 | [null data alpha identifier, request IMSI] |
| 4 | ME | The ME should not give any information to user on the fact that the ME is performing an AT command |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 1.1.1 | [Command performed successfully, AT Response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 1.2.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	null data object

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | A8 | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D |  |

Expected Sequence 1.3 (RUN AT COMMAND, alpha identifier presented, request IMSI)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 1.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 1.3.1 | [alpha identifier, request IMSI] |
| 4 | ME USER | Display "Run AT Command" |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 1.1.1 | [Command performed successfully, AT Response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 1.3.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | A8 | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 |
|  | 0D |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.23.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.3.

##### 27.22.4.23.2	RUN AT COMMAND (Icon support)

27.22.4.23.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.2.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31 and clause 8.41.

-	TS 27.007 [18].

27.22.4.23.2.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

In addition to verify that if an icon is provided by the UICC, the icon indicated in the command may be used by the ME to inform the user, in addition to, or instead of the alpha identifier, as indicated with the icon qualifier.

27.22.4.23.2.4	Method of test

27.22.4.23.2.4.1	Initial conditions

The ME is connected to the USIM Simulator. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

The ME screen shall be in its normal stand-by display.

27.22.4.23.2.4.2	Procedure

Expected Sequence 2.1A (RUN AT COMMAND, basic icon self explanatory, request IMSI, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.1.1 | [BASIC-ICON, self-explanatory, request IMSI] |
| 4 | ME USER | Display BASIC ICON without the alpha identifier |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1A | [Command performed successfully, AT response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 2.1.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha identifier:	"Basic Icon"

AT Command

AT Command string:	"AT+CIMI<CR>"

Icon identifier:

Icon qualifier:	icon is self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | A8 |
|  | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D | 9E | 02 | 00 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 2.1.1A

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.1B (RUN AT COMMAND, basic icon self explanatory, request IMSI, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.1.1 | [BASIC-ICON, self-explanatory, request IMSI] |
| 4 | ME USER | Display "Basic Icon" without the BASIC-ICON |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1B | [Command performed but requested icon could not be displayed, AT response containing IMSI] |

TERMINAL RESPONSE: RUN AT COMMAND 2.1.1B

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.2A (RUN AT COMMAND, colour icon self explanatory, request IMSI, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.2.1 | [COLOUR-ICON, self-explanatory, request IMSI] |
| 4 | ME USER | Display COLOUR-ICON without the alpha identifier |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1A | [Command performed successfully, AT response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 2.2.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha identifier:	"Colour Icon"

AT Command

AT Command string:	"AT+CIMI<CR>"

Icon identifier:

Icon qualifier:	icon is self-explanatory

Icon identifier:	record 2 in EF(IMG)

Coding:

| BER-TLV: | D0 | 24 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 43 | 6F | 6C | 6F | 75 | 72 | 20 | 49 | 63 | 6F | 6E |
|  | A8 | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D | 9E | 02 |
|  | 00 | 02 |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.2B (RUN AT COMMAND, colour icon self explanatory, request IMSI, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.2.1 | [COLOUR-ICON, self-explanatory, request IMSI] |
| 4 | ME USER | Display "Colour Icon" without the COLOUR-ICON |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1B | [Command performed but requested icon could not be displayed, AT response containing IMSI] |

Expected Sequence 2.3A (RUN AT COMMAND, basic icon non self-explanatory, request IMSI, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.3.1 | [BASIC-ICON, non self-explanatory, request IMSI] |
| 4 | ME USER | Display "Basic Icon" and BASIC-ICON |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1A | [Command performed successfully, AT response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 2.3.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha identifier:	"Basic Icon"

AT Command

AT Command string:	"AT+CIMI<CR>"

Icon identifier

Icon qualifier:	icon is non self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | A8 |
|  | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D | 9E | 02 | 01 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.3B (RUN AT COMMAND, basic icon non self-explanatory, request IMSI, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.3.1 | [BASIC-ICON, non self-explanatory, request IMSI] |
| 4 | ME USER | Display "Basic Icon" without BASIC-ICON |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1B | [Command performed but requested icon could not be displayed, AT response containing IMSI] |

Expected Sequence 2.4A (RUN AT COMMAND, colour icon non self-explanatory, request IMSI, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.4.1 | [COLOUR-ICON, non self-explanatory, request IMSI] |
| 4 | ME USER | Display "Colour Icon" and COLOUR-ICON |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1A | [Command performed successfully, AT response containing IMSI] |

PROACTIVE COMMAND: RUN AT COMMAND 2.4.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha identifier:	"Colour Icon"

AT Command

AT Command string:	"AT+CIMI<CR>"

Icon identifier:

Icon qualifier:	icon is self-explanatory

Icon identifier:	record 2 in EF(IMG)

Coding:

| BER-TLV: | D0 | 24 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 43 | 6F | 6C | 6F | 75 | 72 | 20 | 49 | 63 | 6F | 6E |
|  | A8 | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D | 9E | 02 |
|  | 01 | 02 |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.4B (RUN AT COMMAND, colour icon non self-explanatory, request IMSI, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.4.1 | [COLOUR-ICON, non self-explanatory, request IMSI] |
| 4 | ME USER | Display "Colour Icon" without COLOUR-ICON |  |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.1.1B | [Command performed but requested icon could not be displayed, AT response containing IMSI] |

Expected Sequence 2.5 (RUN AT COMMAND, basic icon non self-explanatory, no alpha identifier presented)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 2.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 2.5.1 | [BASIC-ICON, non self-explanatory] |
| 4 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 2.5.1 | [Command data not understood by ME] |

PROACTIVE COMMAND: RUN AT COMMAND 2.5.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

AT Command

AT Command string:	"AT+CIMI<CR>"

Icon identifier

Icon qualifier:	icon is non self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 17 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | A8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 41 | 54 | 2B | 43 | 49 | 4D | 49 | 0D | 9E | 02 | 01 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 2.5.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Result

General Result:	Command data not understood by ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.23.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1 to 2.5.

##### 27.22.4.23.3	RUN AT COMMAND (support of Text Attribute)

27.22.4.23.3.1	RUN AT COMMAND (support of Text Attribute – Left Alignment)

27.22.4.23.3.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.1.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.1.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with left alignment text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.1.4	Method of test

27.22.4.23.3.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.1.4.2	Procedure

Expected Sequence 3.1(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.1.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with left alignment, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.1.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.1.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.1.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [Message shall be formatted without left alignment, request IMSI. Remark: If left alignment is the ME's default alignment as declared in table A.2/16, no alignment change will take place] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.1.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE  COMMAND: RUN AT COMMAND 3.1.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 07 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE  COMMAND: RUN AT COMMAND 3.1.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.1.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

27.22.4.23.3.2	RUN AT COMMAND (support of Text Attribute – Center Alignment)

27.22.4.23.3.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.2.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.2.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with center alignment text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.2.4	Method of test

27.22.4.23.3.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.2.4.2	Procedure

Expected Sequence 3.2(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.2.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with center alignment, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.2.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.2.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.2.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [Message shall be formatted without center alignment, request IMSI. Remark: If center alignment is the ME's default alignment as declared in table A.2/16, no alignment change will take place] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.2.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.2.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2A | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 07 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D04 | 04 | 00 | 10 | 01 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.2.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.2.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.2.

27.22.4.23.3.3	RUN AT COMMAND (support of Text Attribute – Right Alignment)

27.22.4.23.3.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.3.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.3.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with right alignment text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.3.4	Method of test

27.22.4.23.3.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.3.4.2	Procedure

Expected Sequence 3.3(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.3.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with right alignment, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.3.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.3.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.3.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [Message shall be formatted without right alignment, request IMSI. Remark: If right alignment is the ME's default alignment as declared in table A.2/16, no alignment change will take place] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.3.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.3.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 07 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 02 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.3.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.3.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.3.

27.22.4.23.3.4	RUN AT COMMAND (support of Text Attribute – Large Font Size)

27.22.4.23.3.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.4.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.4.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with large font size as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.4.4	Method of test

27.22.4.23.3.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.4.4.2	Procedure

Expected Sequence 3.4(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.4.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with large font size, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.4.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.4.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.4.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with normal font size, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.4.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.4.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.4.1 |  |
| 16 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with large font size, request IMSI] |
| 17 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.4.1 | [Command performed successfully, AT Response containing IMSI] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.4.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.4.3 |  |
| 22 | ME ( USER) | Display "Run AT Command 3" | [alpha identifier is displayed with normal font size, request IMSI] |
| 23 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.4.1 | [Command performed successfully, AT Response containing IMSI] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.4.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 04 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.4.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.4.3

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 3"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 33 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.4.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.4.

27.22.4.23.3.5	RUN AT COMMAND (support of Text Attribute – Small Font Size)

27.22.4.23.3.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.5.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.5.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with small font size as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.5.4	Method of test

27.22.4.23.3.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.5.4.2	Procedure

Expected Sequence 3.5(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.5.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with small font size, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.5.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.5.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.5.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with normal font size, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.5.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.5.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.5.1 |  |
| 16 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with small font size, request IMSI] |
| 17 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.5.1 | [Command performed successfully, AT Response containing IMSI] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.5.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.5.3 |  |
| 22 | ME ( USER) | Display "Run AT Command 3" | [alpha identifier is displayed with normal font size, request IMSI] |
| 23 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.5.1 | [Command performed successfully, AT Response containing IMSI] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.5.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 08 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.5.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.5.3

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 3"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 33 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.5.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.5.

27.22.4.23.3.6	RUN AT COMMAND (support of Text Attribute – Bold On)

27.22.4.23.3.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.6.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.6.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with bold text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.6.4	Method of test

27.22.4.23.3.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.6.4.2	Procedure

Expected Sequence 3.6(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.6.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with bold on, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.6.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.6.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.6.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with bold off, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.6.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.6.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.6.1 |  |
| 16 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with bold on, request IMSI] |
| 17 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.6.1 | [Command performed successfully, AT Response containing IMSI] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.6.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.6.3 |  |
| 22 | ME ( USER) | Display "Run AT Command 3" | [alpha identifier is displayed with bold off, request IMSI] |
| 23 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.6.1 | [Command performed successfully, AT Response containing IMSI] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.6.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 10 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.6.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.6.3

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 3"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 33 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.6.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.6.

27.22.4.23.3.7	RUN AT COMMAND (support of Text Attribute – Italic On)

27.22.4.23.3.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.7.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.7.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with italic text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.7.4	Method of test

27.22.4.23.3.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.7.4.2	Procedure

Expected Sequence 3.7(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.7.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with italic on, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.7.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.7.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.7.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with italic off, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.7.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.7.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.7.1 |  |
| 16 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with italic on, request IMSI] |
| 17 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.7.1 | [Command performed successfully, AT Response containing IMSI] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.7.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.7.3 |  |
| 22 | ME ( USER) | Display "Run AT Command 3" | [alpha identifier is displayed with italic off, request IMSI] |
| 23 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.7.1 | [Command performed successfully, AT Response containing IMSI] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.7.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 20 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.7.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.7.3

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 3"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D |
|  |  | 61 | 6E | 64 | 20 | 33 | A8 | 08 | 41 | 54 | 2B | 43 |
|  |  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.7.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.7.

27.22.4.23.3.8	RUN AT COMMAND (support of Text Attribute – Underline On)

27.22.4.23.3.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.8.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.8.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with underline text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.8.4	Method of test

27.22.4.23.3.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.8.4.2	Procedure

Expected Sequence 3.8(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.8.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with underline on, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.8.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.8.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.8.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with underline off, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.8.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.8.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.8.1 |  |
| 16 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with underline on, request IMSI] |
| 17 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.8.1 | [Command performed successfully, AT Response containing IMSI] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.8.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.8.3 |  |
| 22 | ME ( USER) | Display "Run AT Command 3" | [alpha identifier is displayed with underline off, request IMSI] |
| 23 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.8.1 | [Command performed successfully, AT Response containing IMSI] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.8.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 40 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.8.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.8.3

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 3"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 33 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.8.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.8.

27.22.4.23.3.9	RUN AT COMMAND (support of Text Attribute – Strikethrough On)

27.22.4.23.3.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.9.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.9.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with strikethrough text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.9.4	Method of test

27.22.4.23.3.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.9.4.2	Procedure

Expected Sequence 3.9(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.9.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with strikethrough on, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.9.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.9.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.9.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with strikethrough off, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.9.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.9.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.9.1 |  |
| 16 | ME ( USER) | Display " Run AT Command 1" | [alpha identifier is displayed with strikethrough on, request IMSI] |
| 17 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.9.1 | [Command performed successfully, AT Response containing IMSI] |
| 18 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.9.3 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.9.3 |  |
| 22 | ME ( USER) | Display "Run AT Command 3" | [alpha identifier is displayed with strikethrough off, request IMSI] |
| 23 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.9.1 | [Command performed successfully, AT Response containing IMSI] |
| 24 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.9.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 80 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.9.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.9.3

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 3"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 33 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.9.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.9.

27.22.4.23.3.10	RUN AT COMMAND (support of Text Attribute – Foreground and Background Colour)

27.22.4.23.3.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.3.10.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.3.10.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with foreground and background colour text attribute as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.3.10.4	Method of test

27.22.4.23.3.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.3.10.4.2	Procedure

Expected Sequence 3.10(RUN AT COMMAND, with alpha identifier presented, request IMSI, Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.10.1 |  |
| 4 | ME ( USER) | Display "Run AT Command 1" | [alpha identifier is displayed with foreground and background colour according to the text attribute configuration, request IMSI] |
| 5 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.10.1 | [Command performed successfully, AT Response containing IMSI] |
| 6 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: RUN AT COMMAND 3.10.2 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: RUN AT COMMAND 3.10.2 |  |
| 10 | ME ( USER) | Display "Run AT Command 2" | [alpha identifier is displayed with ME's default foreground and background colour, request IMSI] |
| 11 | ME UICC | TERMINAL RESPONSE: RUN AT COMMAND 3.10.1 | [Command performed successfully, AT Response containing IMSI] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.10.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 1"

AT Command

AT Command string:	"AT+CIMI<CR>"

Text Attribute

Formatting position:	0

Formatting length:	16

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 2B | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 31 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D | D0 | 04 | 00 | 10 | 00 | B4 |  |  |  |

PROACTIVE COMMAND: RUN AT COMMAND 3.10.2

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier

Alpha Identifier	"Run AT Command 2"

AT Command

AT Command string:	"AT+CIMI<CR>"

Coding:

| BER-TLV: | D0 | 25 | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 10 | 52 | 75 | 6E | 20 | 41 | 54 | 20 | 43 | 6F | 6D | 6D |
|  | 61 | 6E | 64 | 20 | 32 | A8 | 08 | 41 | 54 | 2B | 43 | 49 |
|  | 4D | 49 | 0D |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RUN AT COMMAND 3.10.1

Logically:

Command details

Command number:	1

Command type:	RUN AT COMMAND

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

AT Response

AT Response string:	<CR><LF>IMSI<CR><LF><CR><LF>OK<CR><LF>

Coding:

| BER-TLV: | 81 | 03 | 01 | 34 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | A9 | 19 | 0D | 0A | 30 | 30 | 31 | 30 | 31 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 37 | 38 | 39 | 0D | 0A | 0D | 0A | 4F |
|  | 4B | 0D | 0A |  |  |  |  |  |  |  |  |  |

27.22.4.23.3.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.10.

##### 27.22.4.23.4	RUN AT COMMAND (UCS2 display in Cyrillic)

27.22.4.23.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.4.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.4.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with UCS2 alpha identifier as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.4.4	Method of test

27.22.4.23.4.4.1	Initial conditions

The ME is connected to the UICC Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.4.4.2	Procedure

Expected Sequence 4.1(RUN AT COMMAND, alpha identifier presented coded with UCS2 in Cyrillic, request ME Manufacturer ID)

See ETSI TS 102 384 [26] in subclause 27.22.4.23.4.4.2, Expected Sequence 4.1.

27.22.4.23.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

##### 27.22.4.23.5	RUN AT COMMAND (UCS2 display in Chinese)

27.22.4.23.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.5.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.5.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with UCS2 alpha identifier as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.5.4	Method of test

27.22.4.23.5.4.1	Initial conditions

The ME is connected to the UICC Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.5.4.2	Procedure

Expected Sequence 5.1(RUN AT COMMAND, alpha identifier presented coded with UCS2 in Chinese, request ME Manufacturer ID)

See ETSI TS 102 384 [26] in subclause 27.22.4.23.5.4.2, Expected Sequence 5.1.

27.22.4.23.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.23.6	RUN AT COMMAND (UCS2 display in Katakana)

27.22.4.23.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.23.6.2	Conformance requirement

The ME shall support the Proactive UICC: RUN AT COMMAND facility as defined in:

-	TS 31.111 [15] clause 6.4.23, clause 6.6.23, clause 5.2, clause 6.8, clause 8.6, clause 8.7, clause 8.2, clause 8.40, clause 8.31, clause 8.41 and clause 8.70.

-	TS 27.007 [18].

The terminal shall support the text attribute.

27.22.4.23.6.3	Test purpose

To verify that the ME responds to an AT Command contained within a RUN AT COMMAND with UCS2 alpha identifier as though it were initiated by an attached TE, and returns an AT Response within a TERMINAL RESPONSE to the UICC.

27.22.4.23.6.4	Method of test

27.22.4.23.6.4.1	Initial conditions

The ME is connected to the UICC Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

Prior to the test the ME shall be connected to the TE.

The TA-TE interface is set to 8-bit operation.

27.22.4.23.6.4.2	Procedure

Expected Sequence 6.1(RUN AT COMMAND, alpha identifier presented coded with UCS2 in Katakana, request ME Manufacturer ID)

See ETSI TS 102 384 [26] in subclause 27.22.4.23.6.4.2, Expected Sequence 6.1.

27.22.4.23.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

#### 27.22.4.24	SEND DTMF

##### 27.22.4.24.1	SEND DTMF (Normal)

27.22.4.24.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.1.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2 and clause 8.44.

27.22.4.24.1.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that if an alpha identifier is provided by the UICC and is a null data object the ME does not give any information to the user on the fact that the ME is performing a SEND DTMF command.

27.22.4.24.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.1.4. 2	Procedure

Expected Sequence 1.1 (SEND DTMF, normal)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 1.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 1.1.1 |  |
| 7 | ME USER | May give information to the user concerning what is happening. Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 1.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

DTMF String:	"1" pause "2"

Coding:

| BER-TLV: | D0 | 0D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | AC |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | C1 | F2 |  |  |  |  |  |  |  |  |  |

Start DTMF 1.1

Logically:

DTMF String:	"1"

Start DTMF 1.2

Logically:

DTMF String:	"2"

TERMINAL RESPONSE: SEND DTMF 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (SEND DTMF, containing alpha identifier)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 1.2.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 1.2.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | Alpha identifier |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 1.1.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | AC | 05 |
|  | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |  |  |

Start DTMF 1.3

Logically:

DTMF String:	"3"

Start DTMF 1.4

Logically:

DTMF String:	"4"

Start DTMF 1.5

Logically:

DTMF String:	"5"

Start DTMF 1.6

Logically:

DTMF String:	"6"

Start DTMF 1.7

Logically:

DTMF String:	"7"

Start DTMF 1.8

Logically:

DTMF String:	"8"

Start DTMF 1.9

Logically:

DTMF String:	"9"

Start DTMF 1.10

Logically:

DTMF String:	"0"

Expected Sequence 1.3 (SEND DTMF, containing alpha identifier with null data object)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 1.3.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 1.3.1 | Alpha identifier with null data object |
| 7 | ME USER | Do not give any information to the user on the fact that the ME is performing a SEND DTMF command. Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 30 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 1.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 1.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"" (null data object)

DTMF String:	"1" pause pause pause pause pause pause pause pause pause pause "2"

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | AC | 06 | C1 | CC | CC | CC | CC | 2C |  |  |  |

Expected Sequence 1.4 (SEND DTMF, mobile is not in a speech call)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 1.1.1 | [Mobile is not in a speech call] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND DTMF 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SEND DTMF 1.4.1 | [ME currently unable to process command, not in speech call] |
| 5 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

TERMINAL RESPONSE: SEND DTMF 1.4.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command

Additional information:	Not in speech call

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.24.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.4.

##### 27.22.4.24.2	SEND DTMF (Display of icons)

27.22.4.24.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.2.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44, clause 8.31 and clause 6.5.4.

27.22.4.24.2.3	Test purpose

To verify that after a call has been successfully established the ME send the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME do not locally generate audible DTMF tones and play them to the user.

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the icons which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.2.4	Method of test

27.22.4.24.2.4.1	Initial conditions

The ME is connected to the SIM Simulator and only connected to the System Simulator if the System Simulator is mentioned in the sequence table.Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

The elementary files are coded as Toolkit default.

27.22.4.24.2.4.2	Procedure

Expected Sequence 2.1A (SEND DTMF, BASIC ICON self explanatory, successful)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 2.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 2.1.1 | [BASIC-ICON, self-explanatory] |
| 7 | ME USER | Display the BASIC-ICON Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 2.1.1A | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Basic Icon"

DTMF String:	"1" pause "2"

Icon identifier

Icon qualifier:	icon is self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0A | 42 | 61 | 73 | 69 | 63 | 20 | 49 | 63 | 6F | 6E | AC |
|  | 02 | C1 | F2 | 9E | 02 | 00 | 01 |  |  |  |  |  |

DTMF Request 2.1.1

Logically:

DTMF String:	$DTMF_2.1$ = "C1 F2" (given as example)

TERMINAL RESPONSE: SEND DTMF 2.1.1A

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.1B (SEND DTMF, BASIC ICON self explanatory, requested icon could not be displayed)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 2.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 2.1.1 | [BASIC-ICON, self-explanatory] |
| 7 | ME USER | Display "Basic Icon" without the icon Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20 % |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 2.1.1B | [Command performed successfully, but requested icon could not be displayed] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

TERMINAL RESPONSE: SEND DTMF 2.1.1B

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully, but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.2A (SEND DTMF, COLOUR-ICON self explanatory, successful)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 2.2.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 2.2.1 | [COLOUR-ICON] |
| 7 | ME USER | Display the COLOUR-ICON Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 2.1.1A | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 2.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Colour Icon"

DTMF String:	"1" pause "2"

Icon identifier:

Icon qualifier:	icon is self-explanatory

Icon identifier:	record 2 in EF(IMG)

Coding:

| BER-TLV: | D0 | 1E | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 43 | 6F | 6C | 6F | 75 | 72 | 20 | 49 | 63 | 6F | 6E |
|  | AC | 02 | C1 | F2 | 9E | 02 | 00 | 02 |  |  |  |  |

Expected Sequence 2.2B (SEND DTMF, COLOUR-ICON self explanatory, requested icon could not be displayed)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 2.2.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 2.2.1 | [COLOUR-ICON] |
| 7 | ME USER | Display "Colour Icon" without the icon Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 2.1.1B | [Command performed successfully, but requested icon could not be displayed] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

Expected Sequence 2.3A (SEND DTMF, Alpha identifier & BASIC-ICON, not self-explanatory, successful)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 2.3.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 2.3.1 | [Alpha identifier & BASIC-ICON, not self-explanatory] |
| 7 | ME USER | Display "Send DTMF" and the BASIC-ICON Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20 % |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 2.1.1A | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 2.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF"

DTMF String:	"1" pause "2"

Icon identifier:

Icon qualifier:	icon is not self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | AC | 02 |
|  | C1 | F2 | 9E | 02 | 01 | 01 |  |  |  |  |  |  |

Expected Sequence 2.3B (SEND DTMF, Alpha identifier & BASIC-ICON, not self-explanatory, requested icon could not be displayed)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 2.3.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 2.3.1 | [Alpha identifier & BASIC-ICON, not self-explanatory] |
| 7 | ME USER | Display "Send DTMF" without the icon Do not locally generate audible DTMF tones and play them to the user. |  |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 2.1.1B | [Command performed successfully, but requested icon could not be displayed] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

27.22.4.24.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1 to 2.3.

##### 27.22.4.24.3	SEND DTMF (UCS2 display in Cyrillic)

27.22.4.24.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.3.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2 and clause 8.44.

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in:

-	ISO/IEC 10646. [17].

27.22.4.24.3.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND DTMF proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.24.3.4	Method of test

27.22.4.24.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.24.3.4.2	Procedure

Expected Sequence 3.1 (SEND DTMF, successful, UCS2 text in Cyrillic)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 3.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 3.1.1 |  |
| 7 | ME USER | Display "ЗДРАВСТВУЙТЕ" | ["Hello" in Russian] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 3.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Text:	"ЗДРАВСТВУЙТЕ"

DTMF String:	"1" pause "2"

Coding:

| BER-TLV: | D0 | 28 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 19 | 80 | 04 | 17 | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 |
|  | 04 | 21 | 04 | 22 | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 |
|  | 04 | 15 | AC | 02 | C1 | F2 |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successful

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.12.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.24.4	SEND DTMF (support of Text Attribute)

27.22.4.24.4.1	SEND DTMF (support of Text Attribute – Left Alignment)

27.22.4.24.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.1.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.1.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the left alignment text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.1.4	Method of test

27.22.4.24.4.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.1.4.2	Procedure

Expected Sequence 4.1 (SEND DTMF, with text attribute – Left Alignment)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.1.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with left alignment] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.1.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.1.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.1.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Message shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/17, no alignment change will take place] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.1.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.1.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

27.22.4.24.4.2	SEND DTMF (support of Text Attribute – Center Alignment)

27.22.4.24.4.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.2.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.2.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the center alignment text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.2.4	Method of test

27.22.4.24.4.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.2.4.2	Procedure

Expected Sequence 4.2 (SEND DTMF, with text attribute – Center Alignment)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.2.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.2.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with center alignment] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.2.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.2.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.2.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Message shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/17, no alignment change will take place] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.2.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 01 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.2.

27.22.4.24.4.3	SEND DTMF (support of Text Attribute – Right Alignment)

27.22.4.24.4.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.3.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.3.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the right alignment text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.3.4	Method of test

27.22.4.24.4.3.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.3.4.2	Procedure

Expected Sequence 4.3 (SEND DTMF, with text attribute – Right Alignment)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.3.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.3.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with right alignment] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.3.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.3.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.3.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Message shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/17, no alignment change will take place] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.3.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | B0 | 02 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.3.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.3.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.3.

27.22.4.24.4.4	SEND DTMF (support of Text Attribute – Large Font Size)

27.22.4.24.4.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.4.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.4.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the large font size text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.4.4	Method of test

27.22.4.24.4.4.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.4.4.2	Procedure

Expected Sequence 4.4 (SEND DTMF, with text attribute – Large Font Size)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.4.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.4.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with large font size] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.4.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.4.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.4.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with normal font size] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.4.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |
| 41 | User ME | Set up a call to "+0123456789" |  |
| 42 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 43 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 44 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.4.1 |  |
| 45 | ME UICC | FETCH |  |
| 46 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.4.1 |  |
| 47 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with large font size] |
| 48 | ME USS | Start DTMF 1.1 | ["1"] |
| 49 | ME USS | Start DTMF 1.2 | ["2"] |
| 50 | ME USS | Start DTMF 1.3 | ["3"] |
| 51 | ME USS | Start DTMF 1.4 | ["4"] |
| 52 | ME USS | Start DTMF 1.5 | ["5"] |
| 53 | ME USS | Start DTMF 1.6 | ["6"] |
| 54 | ME USS | Start DTMF 1.7 | ["7"] |
| 55 | ME USS | Start DTMF 1.8 | ["8"] |
| 56 | ME USS | Start DTMF 1.9 | ["9"] |
| 57 | ME USS | Start DTMF 1.10 | ["0"] |
| 58 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.4.1 | [Command performed successfully] |
| 59 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 60 | User ME | End the call |  |
| 61 | User ME | Set up a call to "+0123456789" |  |
| 62 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 63 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.4.3 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.4.3 |  |
| 67 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with normal font size] |
| 68 | ME USS | Start DTMF 1.1 | ["1"] |
| 69 | ME USS | Start DTMF 1.2 | ["2"] |
| 70 | ME USS | Start DTMF 1.3 | ["3"] |
| 71 | ME USS | Start DTMF 1.4 | ["4"] |
| 72 | ME USS | Start DTMF 1.5 | ["5"] |
| 73 | ME USS | Start DTMF 1.6 | ["6"] |
| 74 | ME USS | Start DTMF 1.7 | ["7"] |
| 75 | ME USS | Start DTMF 1.8 | ["8"] |
| 76 | ME USS | Start DTMF 1.9 | ["9"] |
| 77 | ME USS | Start DTMF 1.10 | ["0"] |
| 78 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.4.1 | [Command performed successfully] |
| 79 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 80 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 04 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.4.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.4.3

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 3"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 33 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.4.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.4.

27.22.4.24.4.5	SEND DTMF (support of Text Attribute – Small Font Size)

27.22.4.24.4.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.5.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.5.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the small font size text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.5.4	Method of test

27.22.4.24.4.5.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.5.4.2	Procedure

Expected Sequence 4.5 (SEND DTMF, with text attribute – Small Font Size)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.5.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.5.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with small font size] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.5.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.5.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.5.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with normal font size] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.5.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |
| 41 | User ME | Set up a call to "+0123456789" |  |
| 42 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 43 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 44 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.5.1 |  |
| 45 | ME UICC | FETCH |  |
| 46 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.5.1 |  |
| 47 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with small font size] |
| 48 | ME USS | Start DTMF 1.1 | ["1"] |
| 49 | ME USS | Start DTMF 1.2 | ["2"] |
| 50 | ME USS | Start DTMF 1.3 | ["3"] |
| 51 | ME USS | Start DTMF 1.4 | ["4"] |
| 52 | ME USS | Start DTMF 1.5 | ["5"] |
| 53 | ME USS | Start DTMF 1.6 | ["6"] |
| 54 | ME USS | Start DTMF 1.7 | ["7"] |
| 55 | ME USS | Start DTMF 1.8 | ["8"] |
| 56 | ME USS | Start DTMF 1.9 | ["9"] |
| 57 | ME USS | Start DTMF 1.10 | ["0"] |
| 58 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.5.1 | [Command performed successfully] |
| 59 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 60 | User ME | End the call |  |
| 61 | User ME | Set up a call to "+0123456789" |  |
| 62 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 63 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.5.3 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.5.3 |  |
| 67 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with normal font size] |
| 68 | ME USS | Start DTMF 1.1 | ["1"] |
| 69 | ME USS | Start DTMF 1.2 | ["2"] |
| 70 | ME USS | Start DTMF 1.3 | ["3"] |
| 71 | ME USS | Start DTMF 1.4 | ["4"] |
| 72 | ME USS | Start DTMF 1.5 | ["5"] |
| 73 | ME USS | Start DTMF 1.6 | ["6"] |
| 74 | ME USS | Start DTMF 1.7 | ["7"] |
| 75 | ME USS | Start DTMF 1.8 | ["8"] |
| 76 | ME USS | Start DTMF 1.9 | ["9"] |
| 77 | ME USS | Start DTMF 1.10 | ["0"] |
| 78 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.5.1 | [Command performed successfully] |
| 79 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 80 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 08 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.5.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.5.3

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 3"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 33 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.5.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.5.

27.22.4.24.4.6	SEND DTMF (support of Text Attribute – Bold On)

27.22.4.24.4.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.6.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.6.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the bold text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.6.4	Method of test

27.22.4.24.4.6.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.6.4.2	Procedure

Expected Sequence 4.6 (SEND DTMF, with text attribute – Bold On)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.6.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.6.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with bold on] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.6.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.6.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.6.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with bold off] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.6.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |
| 41 | User ME | Set up a call to "+0123456789" |  |
| 42 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 43 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 44 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.6.1 |  |
| 45 | ME UICC | FETCH |  |
| 46 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.6.1 |  |
| 47 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with bold on] |
| 48 | ME USS | Start DTMF 1.1 | ["1"] |
| 49 | ME USS | Start DTMF 1.2 | ["2"] |
| 50 | ME USS | Start DTMF 1.3 | ["3"] |
| 51 | ME USS | Start DTMF 1.4 | ["4"] |
| 52 | ME USS | Start DTMF 1.5 | ["5"] |
| 53 | ME USS | Start DTMF 1.6 | ["6"] |
| 54 | ME USS | Start DTMF 1.7 | ["7"] |
| 55 | ME USS | Start DTMF 1.8 | ["8"] |
| 56 | ME USS | Start DTMF 1.9 | ["9"] |
| 57 | ME USS | Start DTMF 1.10 | ["0"] |
| 58 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.6.1 | [Command performed successfully] |
| 59 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 60 | User ME | End the call |  |
| 61 | User ME | Set up a call to "+0123456789" |  |
| 62 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 63 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.6.3 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.6.3 |  |
| 67 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with bold off] |
| 68 | ME USS | Start DTMF 1.1 | ["1"] |
| 69 | ME USS | Start DTMF 1.2 | ["2"] |
| 70 | ME USS | Start DTMF 1.3 | ["3"] |
| 71 | ME USS | Start DTMF 1.4 | ["4"] |
| 72 | ME USS | Start DTMF 1.5 | ["5"] |
| 73 | ME USS | Start DTMF 1.6 | ["6"] |
| 74 | ME USS | Start DTMF 1.7 | ["7"] |
| 75 | ME USS | Start DTMF 1.8 | ["8"] |
| 76 | ME USS | Start DTMF 1.9 | ["9"] |
| 77 | ME USS | Start DTMF 1.10 | ["0"] |
| 78 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.6.1 | [Command performed successfully] |
| 79 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 80 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 10 |
|  | B4 | 00 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.6.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.6.3

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 3"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 33 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.6.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.6.

27.22.4.24.4.7	SEND DTMF (support of Text Attribute – Italic On)

27.22.4.24.4.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.7.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.7.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the italic text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.7.4	Method of test

27.22.4.24.4.7.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.7.4.2	Procedure

Expected Sequence 4.7 (SEND DTMF, with text attribute – Italic On)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.7.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.7.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with italic on] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.7.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.7.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.7.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with italic off] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.7.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |
| 41 | User ME | Set up a call to "+0123456789" |  |
| 42 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 43 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 44 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.7.1 |  |
| 45 | ME UICC | FETCH |  |
| 46 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.7.1 |  |
| 47 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with italic on] |
| 48 | ME USS | Start DTMF 1.1 | ["1"] |
| 49 | ME USS | Start DTMF 1.2 | ["2"] |
| 50 | ME USS | Start DTMF 1.3 | ["3"] |
| 51 | ME USS | Start DTMF 1.4 | ["4"] |
| 52 | ME USS | Start DTMF 1.5 | ["5"] |
| 53 | ME USS | Start DTMF 1.6 | ["6"] |
| 54 | ME USS | Start DTMF 1.7 | ["7"] |
| 55 | ME USS | Start DTMF 1.8 | ["8"] |
| 56 | ME USS | Start DTMF 1.9 | ["9"] |
| 57 | ME USS | Start DTMF 1.10 | ["0"] |
| 58 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.7.1 | [Command performed successfully] |
| 59 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 60 | User ME | End the call |  |
| 61 | User ME | Set up a call to "+0123456789" |  |
| 62 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 63 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.7.3 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.7.3 |  |
| 67 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with italic off] |
| 68 | ME USS | Start DTMF 1.1 | ["1"] |
| 69 | ME USS | Start DTMF 1.2 | ["2"] |
| 70 | ME USS | Start DTMF 1.3 | ["3"] |
| 71 | ME USS | Start DTMF 1.4 | ["4"] |
| 72 | ME USS | Start DTMF 1.5 | ["5"] |
| 73 | ME USS | Start DTMF 1.6 | ["6"] |
| 74 | ME USS | Start DTMF 1.7 | ["7"] |
| 75 | ME USS | Start DTMF 1.8 | ["8"] |
| 76 | ME USS | Start DTMF 1.9 | ["9"] |
| 77 | ME USS | Start DTMF 1.10 | ["0"] |
| 78 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.7.1 | [Command performed successfully] |
| 79 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 80 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 20 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.7.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.7.3

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 3"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 33 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.7.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.7.

27.22.4.24.4.8	SEND DTMF (support of Text Attribute – Underline On)

27.22.4.24.4.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.8.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.8.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the underline text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.8.4	Method of test

27.22.4.24.4.8.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.8.4.2	Procedure

Expected Sequence 4.8 (SEND DTMF, with text attribute – Underline On)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.8.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.8.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with underline on] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.8.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.8.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.8.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with underline off] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.8.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |
| 41 | User ME | Set up a call to "+0123456789" |  |
| 42 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 43 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 44 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.8.1 |  |
| 45 | ME UICC | FETCH |  |
| 46 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.8.1 |  |
| 47 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with underline on] |
| 48 | ME USS | Start DTMF 1.1 | ["1"] |
| 49 | ME USS | Start DTMF 1.2 | ["2"] |
| 50 | ME USS | Start DTMF 1.3 | ["3"] |
| 51 | ME USS | Start DTMF 1.4 | ["4"] |
| 52 | ME USS | Start DTMF 1.5 | ["5"] |
| 53 | ME USS | Start DTMF 1.6 | ["6"] |
| 54 | ME USS | Start DTMF 1.7 | ["7"] |
| 55 | ME USS | Start DTMF 1.8 | ["8"] |
| 56 | ME USS | Start DTMF 1.9 | ["9"] |
| 57 | ME USS | Start DTMF 1.10 | ["0"] |
| 58 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.8.1 | [Command performed successfully] |
| 59 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 60 | User ME | End the call |  |
| 61 | User ME | Set up a call to "+0123456789" |  |
| 62 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 63 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.8.3 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.8.3 |  |
| 67 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with underline off] |
| 68 | ME USS | Start DTMF 1.1 | ["1"] |
| 69 | ME USS | Start DTMF 1.2 | ["2"] |
| 70 | ME USS | Start DTMF 1.3 | ["3"] |
| 71 | ME USS | Start DTMF 1.4 | ["4"] |
| 72 | ME USS | Start DTMF 1.5 | ["5"] |
| 73 | ME USS | Start DTMF 1.6 | ["6"] |
| 74 | ME USS | Start DTMF 1.7 | ["7"] |
| 75 | ME USS | Start DTMF 1.8 | ["8"] |
| 76 | ME USS | Start DTMF 1.9 | ["9"] |
| 77 | ME USS | Start DTMF 1.10 | ["0"] |
| 78 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.8.1 | [Command performed successfully] |
| 79 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 80 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 40 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.8.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.8.3

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 3"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 33 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.8.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.8.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.8.

27.22.4.24.4.9	SEND DTMF (support of Text Attribute – Strikethrough On)

27.22.4.24.4.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.9.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.9.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the strikethrough text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.9.4	Method of test

27.22.4.24.4.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.9.4.2	Procedure

Expected Sequence 4.9 (SEND DTMF, with text attribute – Strikethrough On)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.9.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.9.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with strikethrough on] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.9.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.9.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.9.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with strikethrough off] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.9.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |
| 41 | User ME | Set up a call to "+0123456789" |  |
| 42 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 43 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 44 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.9.1 |  |
| 45 | ME UICC | FETCH |  |
| 46 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.9.1 |  |
| 47 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with strikethrough on] |
| 48 | ME USS | Start DTMF 1.1 | ["1"] |
| 49 | ME USS | Start DTMF 1.2 | ["2"] |
| 50 | ME USS | Start DTMF 1.3 | ["3"] |
| 51 | ME USS | Start DTMF 1.4 | ["4"] |
| 52 | ME USS | Start DTMF 1.5 | ["5"] |
| 53 | ME USS | Start DTMF 1.6 | ["6"] |
| 54 | ME USS | Start DTMF 1.7 | ["7"] |
| 55 | ME USS | Start DTMF 1.8 | ["8"] |
| 56 | ME USS | Start DTMF 1.9 | ["9"] |
| 57 | ME USS | Start DTMF 1.10 | ["0"] |
| 58 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.9.1 | [Command performed successfully] |
| 59 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 60 | User ME | End the call |  |
| 61 | User ME | Set up a call to "+0123456789" |  |
| 62 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 63 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 64 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.9.3 |  |
| 65 | ME UICC | FETCH |  |
| 66 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.9.3 |  |
| 67 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with strikethrough off] |
| 68 | ME USS | Start DTMF 1.1 | ["1"] |
| 69 | ME USS | Start DTMF 1.2 | ["2"] |
| 70 | ME USS | Start DTMF 1.3 | ["3"] |
| 71 | ME USS | Start DTMF 1.4 | ["4"] |
| 72 | ME USS | Start DTMF 1.5 | ["5"] |
| 73 | ME USS | Start DTMF 1.6 | ["6"] |
| 74 | ME USS | Start DTMF 1.7 | ["7"] |
| 75 | ME USS | Start DTMF 1.8 | ["8"] |
| 76 | ME USS | Start DTMF 1.9 | ["9"] |
| 77 | ME USS | Start DTMF 1.10 | ["0"] |
| 78 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.9.1 | [Command performed successfully] |
| 79 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 80 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 80B |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.9.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.9.3

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 3"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 33 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.9.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.9.

27.22.4.24.4.10	SEND DTMF (support of Text Attribute – Foreground and Background Colour)

27.22.4.24.4.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.4.10.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2, clause 8.44 and clause 8.70.

27.22.4.24.4.10.3	Test purpose

To verify that after a call has been successfully established the ME sends the DTMF string contained in the SEND DTMF proactive UICC command to the network, and returns a successful response in the TERMINAL RESPONSE command sent to the UICC.

To verify that the ME does not locally generate audible DTMF tones and play them to the user.

To verify that if the ME is in idle mode it informs the UICC using TERMINAL RESPONSE '20' with the additional information "Not in speech call".

To verify that the ME displays the text contained in the SEND DTMF proactive UICC command.

To verify that the ME displays the alpha identifier according to the foreground and background colour text attribute configuration which are referred to in the contents of the SEND DTMF proactive UICC command.

27.22.4.24.4.10.4	Method of test

27.22.4.24.4.10.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.4.24.4.10.4.2	Procedure

Expected Sequence 4.10 (SEND DTMF, with text attribute – Foreground and Background Colour)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.10.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.10.1 |  |
| 7 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with foreground and background colour according to the text attribute configuration] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME USS | Start DTMF 1.2 | ["2"] |
| 10 | ME USS | Start DTMF 1.3 | ["3"] |
| 11 | ME USS | Start DTMF 1.4 | ["4"] |
| 12 | ME USS | Start DTMF 1.5 | ["5"] |
| 13 | ME USS | Start DTMF 1.6 | ["6"] |
| 14 | ME USS | Start DTMF 1.7 | ["7"] |
| 15 | ME USS | Start DTMF 1.8 | ["8"] |
| 16 | ME USS | Start DTMF 1.9 | ["9"] |
| 17 | ME USS | Start DTMF 1.10 | ["0"] |
| 18 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.10.1 | [Command performed successfully] |
| 19 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 20 | User ME | End the call |  |
| 21 | User ME | Set up a call to "+0123456789" |  |
| 22 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 23 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 4.10.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DTMF 4.10.2 |  |
| 27 | ME USER | Display "Send DTMF" Do not locally generate audible DTMF tones and play them to the user. | [Alpha identifier is displayed with ME's default foreground and background colour] |
| 28 | ME USS | Start DTMF 1.1 | ["1"] |
| 29 | ME USS | Start DTMF 1.2 | ["2"] |
| 30 | ME USS | Start DTMF 1.3 | ["3"] |
| 31 | ME USS | Start DTMF 1.4 | ["4"] |
| 32 | ME USS | Start DTMF 1.5 | ["5"] |
| 33 | ME USS | Start DTMF 1.6 | ["6"] |
| 34 | ME USS | Start DTMF 1.7 | ["7"] |
| 35 | ME USS | Start DTMF 1.8 | ["8"] |
| 36 | ME USS | Start DTMF 1.9 | ["9"] |
| 37 | ME USS | Start DTMF 1.10 | ["0"] |
| 38 | ME UICC | TERMINAL RESPONSE: SEND DTMF 4.10.1 | [Command performed successfully] |
| 39 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 40 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 1"

DTMF String:	"1234567890"

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 23 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 31 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 | D0 | 04 | 00 | 0B | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DTMF 4.10.2

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send DTMF 2"

DTMF String:	"1234567890"

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 54 | 4D | 46 | 20 | 32 |
|  | AC | 05 | 21 | 43 | 65 | 87 | 09 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 4.10.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.4.10.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.10.

##### 27.22.4.24.5	SEND DTMF (UCS2 Display in Chinese)

27.22.4.24.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.5.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2 and clause 8.44.

Additionally the ME shall support the UCS2 facility for the coding of the Chinese characters , as defined in:

-	ISO/IEC 10646. [17].

27.22.4.24.5.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND DTMF proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.24.5.4	Method of test

27.22.4.24.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table. The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.24.5.4.2	Procedure

Expected Sequence 5.1 (SEND DTMF, successful, UCS2 text in Chinese)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 5.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 5.1.1 |  |
| 7 | ME USER | Display "你好" | ["Hello" in Chinese] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 5.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Text:	"你好"

DTMF String:	"1" pause "2"

Coding:

| BER-TLV: | D0 | 14 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 80 | 4F | 60 | 59 | 7D | AC | 02 | C1 | F2 |  |  |

TERMINAL RESPONSE: SEND DTMF 5.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successful

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.5.5	Test requirement

The ME shall operate in the manner defined in expected sequence 5.1.

##### 27.22.4.24.6	SEND DTMF (UCS2 Display in Katakana)

27.22.4.24.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.24.6.2	Conformance requirement

The ME shall support the Proactive UICC: Send DTMF facility as defined in:

-	TS 31.111 [15] clause 6.1, clause 6.4.24, clause 6.6.24, clause 8.12.2, clause 5.2, clause 8.6, clause 8.7, clause 8.2 and clause 8.44.

Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters , as defined in:

-	ISO/IEC 10646. [17].

27.22.4.24.6.3	Test purpose

To verify that the ME displays the UCS2 text contained in the SEND DTMF proactive UICC command, and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.24.6.4	Method of test

27.22.4.24.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table. The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.24.6.4.2	Procedure

Expected Sequence 6.1 (SEND DTMF, successful, UCS2 text)

Some details of the DTMF protocol have been left out for clarity.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | Set up a call to "+0123456789" |  |
| 2 | ME USS | The ME attempts to set up a call to "+0123456789" |  |
| 3 | USS ME | The ME receives the CONNECT message from the USS. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: SEND DTMF 6.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND: SEND DTMF 6.1.1 |  |
| 7 | ME USER | Display "ル" | [Character in Katakana] |
| 8 | ME USS | Start DTMF 1.1 | ["1"] |
| 9 | ME |  | No DTMF sending for 3 seconds ±20% |
| 10 | ME USS | Start DTMF 1.2 | ["2"] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DTMF 6.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 13 | User ME | End the call |  |

PROACTIVE COMMAND: SEND DTMF 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	Network

Alpha Identifier

Text:	"ル"

DTMF String:	"1" pause "2"

Coding:

| BER-TLV: | D0 | 12 | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 | 80 | 30 | EB | AC | 02 | C1 | F2 |  |  |  |  |

TERMINAL RESPONSE: SEND DTMF 6.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DTMF

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successful

Coding:

| BER-TLV: | 81 | 03 | 01 | 14 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.24.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 6.1.

#### 27.22.4.25	LANGUAGE NOTIFICATION

##### 27.22.4.25.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.25.2	Conformance Requirement

The ME shall conclude the command by sending TERMINAL RESPONSE (OK) to the UICC, as soon as possible after receiving the LANGUAGE NOTIFICATION proactive UICC command.

-	TS 31.111 [15] clause 6.4.25 and clause 6.6.25.

##### 27.22.4.25.3	Test purpose

To verify that the ME shall send a TERMINAL RESPONSE (OK) to the UICC after the ME receives the LANGUAGE NOTIFICATION proactive UICC command.

##### 27.22.4.25.4	Method of Test

27.22.4.25.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.4.25.4.2	Procedure

Expected Sequence 1.1 (LANGUAGE NOTIFICATION)

See ETSI TS 102 384 [26] in subclause 27.22.4.25.4.2, Expected Sequence 1.1.

Expected Sequence 1.2 (LANGUAGE NOTIFICATION)

See ETSI TS 102 384 [26] in subclause 27.22.4.25.4.2, Expected Sequence 1.2.

##### 27.22.4.25.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 and 1.2.

#### 27.22.4.26	LAUNCH BROWSER

##### 27.22.4.26.1	LAUNCH BROWSER (No session already launched)

27.22.4.26.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.1.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15 and clause 8.31.

27.22.4.26.1.3	Test purpose

To verify that when the ME is in idle state, it launches properly the browser session required in LAUNCH BROWSER, and returns a successful result in the TERMINAL RESPONSE command.

27.22.4.26.1.4	Method of test

27.22.4.26.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default browser parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default browser parameters.

The mobile is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

Bearer Parameters

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

GPRS Parameters

Network access name:	TestGp.rs

User login:	UserLog

User password:	UserPwd

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01 (as an example)

Note:	If a data destination address different to 01.01.01.01 is used then the network simulator setup and related UE settings might require a corresponding adaptation.

27.22.4.26.1.4.2	Procedure

Expected Sequence 1.1 (LAUNCH BROWSER, connect to the default URL)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared. The ME supports Launch Browser with Default URL] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 1.1.1 | [connect to the default URL, "launch browser, if not already launched", no null alpha id.] |
| 4 | ME USER | ME displays the alpha identifier |  |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 1.1.1 | [Command performed successfully] |
| 7 | MEUSS | If command was performed successfully, the ME attempts to launch the session with the default browser parameters and the default URL. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the browser session to defined URL is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 1.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	empty

Alpha Identifier	"Default URL"

Coding:

| BER-TLV: | D0 | 18 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 05 | 0B | 44 | 65 | 66 | 61 | 75 | 6C | 74 | 20 | 55 |
|  | 52 | 4C |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 1.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (LAUNCH BROWSER, connect to the specified URL, alpha identifier length=0)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 1.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 1.2.1 | [connect to defined URL, "launch browser, if not already launched, alpha identifier length=0] |
| 4 | ME USER | No information should be displayed. |  |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 1.2.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to connect the URL specified in the LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL is properly connected. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 1.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	empty

Coding:

| BER-TLV: | D0 | 1F | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 00 |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 1.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.3 (LAUNCH BROWSER, Browser identity, no alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 1.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 1.3.1 | [connect to the defined URL, "launch browser, if not already launched, browser identity] |
| 4 | ME USER | ME may display a default message of its own. |  |
| 5 | USER ME | The user may confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 1.3.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default browser session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 1.3.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

Browser Identity	default

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Coding::

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 00 | 31 | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 |
|  | 78 | 78 | 2E | 79 | 79 | 79 | 2E | 7A | 7A | 7A |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 1.3.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.4 (LAUNCH BROWSER, only GPRS bearer specified and gateway/proxy identity, GPRS supported by USS)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode, GPRS supported by USS, GPRS supported by the ME and activated, the terminal might need to be configured with an entry linking the Gateway/Proxy Identity in the proactive command with the corresponding connectivity parameters in the mobile. The browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 1.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 1.4.1 | [connect to the defined URL, "launch browser, if not already launched, 1 bearer specified, gateway/proxy id specified] |
| 4 | ME USER | ME may display a default message |  |
| 5 | USER ME | The user may confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 1.4.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to connect the URL specified in LAUNCH BROWSER command using the requested bearer and proxy identity | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the browser session is properly established with the required bearer. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 1.4.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Bearer	GPRS

Gateway/Proxy id

DCSunpacked, 8 bits data

Text string	abc.def.ghi.jkl (different from the default IP address)

Coding::

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 32 | 01 | 03 | 0D | 10 |
|  | 04 | 61 | 62 | 63 | 2E | 64 | 65 | 66 | 2E | 67 | 68 | 69 |
|  | 2E | 6A | 6B | 6C |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 1.4.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.5 Void

Expected Sequence 1.6 (LAUNCH BROWSER, ME does not support Launch Browser with Default URL)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared. The ME does not support Launch Browser with Default URL] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 1.1.1 | [connect to the default URL, "launch browser, if not already launched", no null alpha id.] |
| 4 | ME USER | The ME may display the alpha identifier |  |
| 5 | USER ME | If the ME displays the alpha identifier then the user confirms the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 1.6.1 ) | [ME unable to process command - Default URL unavailable] |
| 7 | UICC ME | PROACTIVE UICC SESSION ENDED |  |

TERMINAL RESPONSE: LAUNCH BROWSER 1.6.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Launch browser generic error code

Additional data	Default URL unavailable

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.26.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.4

##### 27.22.4.26.2	LAUNCH BROWSER (Interaction with current session)

27.22.4.26.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.2.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 82.2, clause 8.47, optional clause 8.49, optional clause 8.50, clause 8.15 and clause 8.31.

27.22.4.26.2.3	Test purpose

To verify that when the ME is already busy in a browser session, it launches properly the browser session required in LAUNCH BROWSER, and returns a successful result in the TERMINAL RESPONSE.

27.22.4.26.2.4	Method of test

27.22.4.26.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to a Wap gateway is required. The default browser parameters (IP address, gateway/proxy identity, called number…) of the tested mobile shall be properly filled to access that gateway.

The mobile is busy in a browser session, the user navigates in pages different from the URL defined in the test sequence.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.The browser's cache shall have been cleared before execution of each sequence.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

27.22.4.26.2.4.2	Procedure

Expected Sequence 2.1 (LAUNCH BROWSER, use the existing browser, connect to the specified URL)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME | The user is navigating in a browser session (not theURL defined in the test sequence). | [Browser is in use, the current session is not secured] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 2.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 2.1.1 | [connect to the defined URL, "use the existing browser", no null alpha id.] |
| 4 | ME USER | ME displays the alpha identifier |  |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 2.1.1 | [Command performed successfully] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] Usage of a new active tab in the browser is a valid behaviour (see note) |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL specified in LAUNCH BROWSER command is connected; and the previous URL can be retrieved. |  |
| NOTE: Active tab indicates that web page is visible to the user. |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 2.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL"

Coding:

| BER-TLV: | D0 | 2A | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0B | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 2.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.2 (LAUNCH BROWSER, close the existing browser session and launch new browser session, connect to the specified URL)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME | The user is navigating in a browser session (not the URL defined in the test sequence). | [Browser is in use, the current session is not secured] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 2.2.1 | [connect to the defined URL, "close the existing browser session and launch new browser session", no null alpha id.] |
| 4 | ME USER | ME displays the alpha identifier |  |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 2.2.1 | [Command performed successfully] |
| 7 | MEUSS | The ME closes the existing session and attempts to launch the session with the default browser parameters and the URL specified in LAUNCH BROWSER command. IF A.1/155_THEN it is a valid behaviour to keep other sessions/tabs open and start the session in a new active tab (see note). | [The UE has the option of maintaining the currently active PDP Context. The USS shall handle the request of additional URLs as defined in the initial conditions section.] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL specified in LAUNCH BROWSER command is connected. |  |
| NOTE: Active tab indicates that web page is visible to the user. |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 2.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	close the existing browser session and launch new browser session

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL"

Coding:

| BER-TLV: | D0 | 2A | 81 | 03 | 01 | 15 | 03 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0B | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 2.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	close the existing browser session and launch new browser session

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 2.3 (LAUNCH BROWSER, if not already launched)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME | The user is navigating in a browser session (not the URL defined in the test sequence). | [Browser is in use, the current session is not secured] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 2.3.1 | [connect to the defined URL, "launch browser, if not already launched] |
| 4 | ME UICC | IF (NOT A.1/155)_THEN TERMINAL RESPONSE: LAUNCH BROWSER 2.3.1 ELSE IF (A.1/155) THEN TERMINAL RESPONSE:LAUNCH BROWSER 2.3.2 | [ME unable to process command - browser unavailable] If browser supports multiple sessions/tabs, it is valid behaviour to open the session in a new active tab that does not interfere with other sessions (see note). |
| 5 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 6 | USER ME | IF (NOT A.1/155)_THEN the user verifies that the URL specified in LAUNCH BROWSER command has not been connected. |  |
| NOTE: Active tab indicates that web page is visible to the user. |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 2.3.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Coding:

| BER-TLV: | D0 | 1D | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 2.3.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Launch browser generic error code

Additional data	Browser unavailable

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 2.3.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.2.5 Test Requirement

The ME shall operate in the manner defined in expected sequences 2.1 to 2.3.

##### 27.22.4.26.3	LAUNCH BROWSER (UCS2 display in Cyrillic)

27.22.4.26.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.3.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, optional clause 8.49, optional clause 8.50, clause 8.15 and clause 8.31.

Additionally the ME shall support the UCS2 facility for the coding of the Cyrillic alphabet, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.26.3.3	Test purpose

To verify that the ME performs a proper user confirmation with an USC2 alpha identifier, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.3.4	Method of test

27.22.4.26.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default browser parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway").

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default browser parameters.

The mobile is busy in a browser session, the user navigates in pages different from the URL defined by default in Wap parameters.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

The browser's cache shall have been cleared before execution of each sequence.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

27.22.4.26.3.4.2	Procedure

Expected Sequence 3.1 (LAUNCH BROWSER, use the existing browser, connect to the specified URL, UCS2 in Cyrillic)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME | The user is navigating in a browser session (not the URL defined in the test sequence). | [Browser is in use, the current session is not secured] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 3.1.1 | [connect to the defined URL, "use the existing browser", alpha id. In UCS2] |
| 4 | ME USER | ME displays the alpha identifier "ЗДРАВСТВУЙТЕ" | ["Hello" in Russian] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 3.1.1 | [Command performed successfully] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL is connected; and the previous URL can be retrieved. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 3.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier

Data coding scheme:	UCS2 (16 bits)

Text:	"ЗДРАВСТВУЙТЕ"

Coding:

| BER-TLV: | D0 | 38 | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 19 | 80 | 04 | 17 |
|  | 04 | 14 | 04 | 20 | 04 | 10 | 04 | 12 | 04 | 21 | 04 | 22 |
|  | 04 | 12 | 04 | 23 | 04 | 19 | 04 | 22 | 04 | 15 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 3.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.26.4	LAUNCH BROWSER (icons support)

27.22.4.26.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.4.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, optional clause 8.49, optional clause 8.50, clause 8.15 and clause 8.31.

27.22.4.26.4.3	Test purpose

To verify that the ME performs a proper user confirmation with an icon identifier, launches the browser session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.4.4	Method of test

27.22.4.26.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default browser parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway").

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default browser parameters.

The mobile is busy in a browser session, the user navigates in pages different from the URL defined by default in browser parameters.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.The browser's cache shall have been cleared before execution of each sequence.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

27.22.4.26.4.4.2	Procedure

Expected Sequence 4.1A (LAUNCH BROWSER, use the existing browser, icon not self explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 4.1.1 | [Browser is in use, the current session is not secured] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 4.1.1 | [connect to the defined URL, "use the existing browser", no null alpha id.] |
| 4 | ME USER | ME displays the alpha identifier and the icon | ["Not self explan."] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 4.1.1 A | [Command performed successfully] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL specified in LAUNCH BROWSER command is connected; and the previous URL can be retrieved. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 4.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Not self explan."

Icon identifier:

Icon qualifier:	not self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 33 | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 10 | 4E | 6F | 74 |
|  | 20 | 73 | 65 | 6C | 66 | 20 | 65 | 78 | 70 | 6C | 61 | 6E |
|  | 2E | 1E | 02 | 01 | 01 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 4.1.1 A

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 4.1B (LAUNCH BROWSER, use the existing browser, icon not self explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 4.1.1 | [Browser is in use, the current session is not secured] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 4.1.1 | [connect to the defined URL, "use the existing browser", no null alpha id.] |
| 4 | ME USER | ME displays the alpha identifier Without the icon | ["Not self explan."] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 4.1.1 B | [Command performed successfully but requested icon could not be displayed] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL specified in LAUNCH BROWSER command is connected; and the previous URL can be retrieved. |  |

TERMINAL RESPONSE: LAUNCH BROWSER 4.1.1 B

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 4.2A (LAUNCH BROWSER, use the existing browser, icon self explanatory, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 4.2.1 | [Browser is in use, the current session is not secured] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 4.2.1 | [connect to the defined URL, "use the existing browser", alpha id. In UCS2] |
| 4 | ME USER | ME displays only the icon | ["Self explan."] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 4.2.1 A | [Command performed successfully] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL specified in LAUNCH BROWSER command is connected; and the previous URL can be retrieved. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 4.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Self explan."

Icon identifier:

Icon qualifier:	self-explanatory

Icon identifier:	record 1 in EF(IMG)

Coding:

| BER-TLV: | D0 | 2F | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0C | 53 | 65 | 6C |
|  | 66 | 20 | 65 | 78 | 70 | 6C | 61 | 6E | 2E | 1E | 02 | 00 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 4.2.1 A

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 4.2B (LAUNCH BROWSER, use the existing browser, icon self explanatory, requested icon could not be displayed)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 4.2.1 | [Browser is in use, the current session is not secured] |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 4.2.1 | [connect to the defined URL, "use the existing browser", alpha id. In UCS2] |
| 4 | ME USER | ME displays only the alpha identifier | ["Self explan."] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 4.2.1 B | [Command performed successfully] [Command performed successfully but requested icon could not be displayed] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL specified in LAUNCH BROWSER command is connected; and the previous URL can be retrieved. |  |

TERMINAL RESPONSE: LAUNCH BROWSER 4.2.1 B

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully but requested icon could not be displayed

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 04 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 4.1A to 4.2B.

##### 27.22.4.26.5	LAUNCH BROWSER (support of Text Attribute)

27.22.4.26.5.1	LAUNCH BROWSER (support of Text Attribute – Left Alignment)

27.22.4.26.5.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.1.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.1.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the left alignment text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.1.4	Method of test

27.22.4.26.5.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.1.4.2	Procedure

Expected Sequence 5.1 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.1.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with left alignment] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.1.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.1.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.1.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [Message shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/18, no alignment change will take place] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.1.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.1.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.1.

27.22.4.26.5.2	LAUNCH BROWSER (support of Text Attribute – Center Alignment)

27.22.4.26.5.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.2.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.2.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the center alignment text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.2.4	Method of test

27.22.4.26.5.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.2.4.2	Procedure

Expected Sequence 5.2 (LAUNCH BROWSER, connect to the default URL with Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.2.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with center alignment] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.2.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.2.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.2.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [Message shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/18, no alignment change will take place] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.2.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 01 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.2.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.2.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.2.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.2.

27.22.4.26.5.3	LAUNCH BROWSER (support of Text Attribute – Right Alignment)

27.22.4.26.5.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.3.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.3.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the right alignment text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.3.4	Method of test

27.22.4.26.5.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

Before execution of each sequence the browser's cache shall be cleared.

27.22.4.26.5.3.4.2	Procedure

Expected Sequence 5.3 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.3.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with right alignment] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.3.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.3.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.3.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [Message shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/18, no alignment change will take place] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.3.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.3.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 02 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.3.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.3.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.3.

27.22.4.26.5.4	LAUNCH BROWSER (support of Text Attribute – Large Font Size)

27.22.4.26.5.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.4.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.4.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the large font size text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.4.4	Method of test

27.22.4.26.5.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

Before execution of each sequence the browser's cache shall be cleared.

27.22.4.26.5.4.4.2	Procedure

Expected Sequence 5.4 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.4.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with large font size] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.4.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.4.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.4.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with normal font size] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.4.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.4.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.4.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 22 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with large font size] |
| 23 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 24 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.4.1 | [Command performed successfully] |
| 25 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 26 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 27 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.4.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.4.3 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 31 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with normal font size] |
| 32 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 33 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.4.1 | [Command performed successfully] |
| 34 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 35 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 36 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.4.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 04 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.4.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.4.3

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.4.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.4.

27.22.4.26.5.5	LAUNCH BROWSER (support of Text Attribute – Small Font Size)

27.22.4.26.5.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.5.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.5.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the small font size text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.5.4	Method of test

27.22.4.26.5.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.5.4.2	Procedure

Expected Sequence 5.5 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.5.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with small font size] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.5.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.5.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.5.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with normal font size] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.5.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.5.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.5.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 22 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with small font size] |
| 23 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 24 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.5.1 | [Command performed successfully] |
| 25 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 26 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 27 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.5.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.5.3 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 31 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with normal font size] |
| 32 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 33 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.5.1 | [Command performed successfully] |
| 34 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 35 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 36 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.5.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 08 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.5.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.5.3

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.5.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.5.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.5.

27.22.4.26.5.6	LAUNCH BROWSER (support of Text Attribute – Bold on)

27.22.4.26.5.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.6.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.6.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the bold text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.6.4	Method of test

27.22.4.26.5.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.6.4.2	Procedure

Expected Sequence 5.6 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.6.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with bold on] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.6.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.6.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.6.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with bold off] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.6.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.6.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.6.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 22 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with bold on] |
| 23 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 24 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.6.1 | [Command performed successfully] |
| 25 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 26 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 27 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.6.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.6.3 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 31 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with bold off] |
| 32 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 33 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.6.1 | [Command performed successfully] |
| 34 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 35 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 36 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.6.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 10 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.6.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.6.3

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.6.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.6.

27.22.4.26.5.7	LAUNCH BROWSER (support of Text Attribute – Italic On)

27.22.4.26.5.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.7.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.7.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the italic text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.7.4	Method of test

27.22.4.26.5.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.7.4.2	Procedure

Expected Sequence 5.7 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.7.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with italic on] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.7.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.7.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.7.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with italic off] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.7.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.7.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.7.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 22 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with italic on] |
| 23 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 24 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.7.1 | [Command performed successfully] |
| 25 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 26 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 27 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.7.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.7.3 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 31 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with italic off] |
| 32 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 33 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.7.1 | [Command performed successfully] |
| 34 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 35 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 36 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.7.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 20 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.7.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.7.3

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.7.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.7.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.7.

27.22.4.26.5.8	LAUNCH BROWSER (support of Text Attribute – Underline On)

27.22.4.26.5.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.8.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.8.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the underline text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.8.4	Method of test

27.22.4.26.5.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.8.4.2	Procedure

Expected Sequence 5.8 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.8.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with underline on] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.8.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.8.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.8.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with underline off] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.8.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.8.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.8.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 22 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with underline on] |
| 23 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 24 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.8.1 | [Command performed successfully] |
| 25 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 26 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 27 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.8.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.8.3 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 31 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with underline off] |
| 32 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 33 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.8.1 | [Command performed successfully] |
| 34 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 35 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 36 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.8.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 40 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.8.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.8.3

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.8.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.8.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.8.

27.22.4.26.5.9	LAUNCH BROWSER (support of Text Attribute – Strikethrough On)

27.22.4.26.5.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.9.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.9.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the strikethrough text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.9.4	Method of test

27.22.4.26.5.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.9.4.2	Procedure

Expected Sequence 5.9 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.9.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with strikethrough on] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.9.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.9.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.9.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with strikethrough off] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.9.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.9.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.9.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 22 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with strikethrough on] |
| 23 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 24 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.9.1 | [Command performed successfully] |
| 25 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 26 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 27 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.9.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.9.3 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 31 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with strikethrough off] |
| 32 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 33 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.9.1 | [Command performed successfully] |
| 34 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 35 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 36 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.9.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 80 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.9.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.9.3

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 3"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 33 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.9.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.9.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.9.

27.22.4.26.5.10	LAUNCH BROWSER (support of Text Attribute – Foreground and Background Colour)

27.22.4.26.5.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.5.10.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, clause 8.49, clause 8.50, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.26.5.10.3	Test purpose

To verify that the ME performs a proper user confirmation with an alpha identifier according to the foreground and background colour text attribute configuration, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.5.10.4	Method of test

27.22.4.26.5.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway")

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

The ME is in idle mode. To ensure that there are no active PDP contexts established until the proactive command is fetched, the USS shall be configured to ignore any PDP context activation request before the LAUNCH BROWSER command is fetched.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

27.22.4.26.5.10.4.2	Procedure

Expected Sequence 5.10 (LAUNCH BROWSER, connect to the specified URL with Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME |  | [The ME is in idle mode and the browser's cache shall have been cleared.] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.10.1 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 4 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with foreground and background colour according to the text attribute configuration] |
| 5 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.10.1 | [Command performed successfully] |
| 7 | MEUSS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the default Wap session is properly established. The user shall attempt to close the browser or shall at least set the ME to the idle screen. |  |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 5.10.2 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 5.10.2 | [connect to the defined URL, "launch browser, if not already launched", no null alpha id] |
| 13 | ME USER | ME displays the alpha identifier | [alpha identifier is displayed with ME's default foreground and background colour] |
| 14 | USER ME | The user may have to confirm the launch browser. | [option: user confirmation] |
| 15 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 5.10.1 | [Command performed successfully] |
| 16 | ME USS | The ME attempts to launch the session with the default Wap parameters and the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | USER ME | The user verifies that the default Wap session is properly established. |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.10.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 1"

Text Attribute

Formatting position:	0

Formatting length:	13

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 32 | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 31 | D0 | 04 |
|  | 00 | 0D | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: LAUNCH BROWSER 5.10.2

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier	"Defined URL 2"

Coding:

| BER-TLV: | D0 | 2C | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 0D | 44 | 65 | 66 |
|  | 69 | 6E | 65 | 64 | 20 | 55 | 52 | 4C | 20 | 32 |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 5.10.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	launch browser, if not already launched

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.5.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.10.

##### 27.22.4.26.6	LAUNCH BROWSER (UCS2 Display in Chinese)

27.22.4.26.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.6.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, optional clause 8.49, optional clause 8.50, clause 8.15 and clause 8.31.

Additionally the ME shall support the UCS2 facility for the coding of the Chinese characters, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.26.6.3	Test purpose

To verify that the ME performs a proper user confirmation with an USC2 alpha identifier, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.6.4	Method of test

27.22.4.26.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway").

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The mobile is busy in a Wap session, the user navigates in pages different from the URL defined by default in Wap parameters.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

The browser's cache shall have been cleared before execution of the test sequence.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

27.22.4.26.6.4.2	Procedure

Expected Sequence 6.1 (LAUNCH BROWSER, use the existing browser, connect to the specified URL, UCS2 in Chinese)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME | The user is navigating in a Wap session (not the URL specified in the test sequence). | [Browser is in use, the current session is not secured] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 6.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 6.1.1 | [connect to the defined URL, "use the existing browser", alpha id. In UCS2] |
| 4 | ME USER | ME displays the alpha identifier "你好" | ["Hello" in Chinese] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 6.1.1 | [Command performed successfully] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL is connected; and the previous URL can be retrieved. |  |

PROACTIVE COMMAND: LAUNCH BROWSER  6.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier

Data coding scheme:	UCS2 (16 bits)

Text:	"你好"

Coding:

| BER-TLV: | D0 | 24 | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 05 | 80 | 4F | 60 |
|  | 59 | 7D |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: LAUNCH BROWSER 6.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.6.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.26.7	LAUNCH BROWSER (UCS2 Display in Katakana)

27.22.4.26.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.26.7.2	Conformance requirements

The ME shall support the LAUNCH BROWSER Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 5.2, clauses 6.4.26 and 6.6.26, clause 8.6, clause 8.7, clause 8.48, clause 9.2, clause 8.2, clause 8.47, optional clause 8.49, optional clause 8.50, clause 8.15 and clause 8.31.

Additionally the ME shall support the UCS2 facility for the coding of the Katakana characters, as defined in:

-	ISO/IEC 10646 [17].

27.22.4.26.7.3	Test purpose

To verify that the ME performs a proper user confirmation with an USC2 alpha identifier, launches the Wap session required in LAUNCH BROWSER and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.26.7.4	Method of test

27.22.4.26.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

A valid access to 2 different Wap gateways is required:

-	the default Wap parameters (IP address, gateway/proxy identity, called number, URL …) of the tested mobile shall be properly filled to access one of the gateways ("default gateway").

With that default gateway we shall be able to access to an URL different from the default one.

-	another gateway with an IP address different from the one defined in default Wap parameters.

The mobile is busy in a Wap session, the user navigates in pages different from the URL defined by default in Wap parameters.

For URL requests resulting from the LAUNCH BROWSER command execution the USS shall be configured to respond with an HTTP status error code (4xx "Client Error" or 5xx "Server Error") to URL requests which do not match the Default URL or the URL provided in the proactive command. At the same time the USS shall ignore these URL requests regarding the test case verdict generation.

The browser's cache shall have been cleared before execution of the test sequence.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

27.22.4.26.7.4.2	Procedure

Expected Sequence 7.1 (LAUNCH BROWSER, use the existing browser, connect to the specified URL, UCS2 in Katakana)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 0 | ME | The user is navigating in a Wap session (not the URL defined in the test sequence). | [Browser is in use, the current session is not secured]] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: LAUNCH BROWSER 7.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: LAUNCH BROWSER 7.1.1 | [connect to the defined URL, "use the existing browser", alpha id. In UCS2] |
| 4 | ME USER | ME displays the alpha identifier "ル" | [Character in Katakana] |
| 5 | USER ME | The user confirms the launch browser. | [user confirmation] |
| 6 | ME UICC | TERMINAL RESPONSE: LAUNCH BROWSER 7.1.1 | [Command performed successfully] |
| 7 | MEUSS | The ME does not close the existing session and attempts to connect the URL specified in LAUNCH BROWSER command. | [The USS shall handle the request of additional URLs as defined in the initial conditions section] |
| 8 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 9 | USER ME | The user verifies that the URL is connected; and the previous URL can be retrieved. |  |

PROACTIVE COMMAND: LAUNCH BROWSER  7.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	UICC

Destination device:	ME

URL	http://xxx.yyy.zzz (Note: this URL shall be different from the default URL, but it can be reached from the gateway defined by default in the browser parameters of the mobile)

Alpha Identifier

Data coding scheme:	UCS2 (16 bits)

Text:	"ル"

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 81 | 82 | 31 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 05 | 03 | 80 | 30 | EB |  |  |  |  |  |  |
|  | 12 | 68 | 74 | 74 | 70 | 3A | 2F | 2F | 78 | 78 | 78 | 2E |
|  | 79 | 79 | 79 | 2E | 7A | 7A | 7A | 05 | 03 | 80 | 30 | EB |

TERMINAL RESPONSE: LAUNCH BROWSER 7.1.1

Logically:

Command details

Command number:	1

Command type:	LAUNCH BROWSER

Command qualifier:	use the existing browser

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 15 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.26.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

#### 27.22.4.27	OPEN CHANNEL

##### 27.22.4.27.1	Void

##### 27.22.4.27.2	Open Channel (related to GPRS)

27.22.4.27.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.2.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 9.2, clause 8.2, clause 8.15, clause 8.31 and clause 8.70.

27.22.4.27.2.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (OK); or

-	TERMINAL RESPONSE (Command performed with modification); or

-	TERMINAL RESPONSE (User did not accept the proactive command);

-	TERMINAL RESPONSE (ME currently unable to process command);

to the UICC after the ME receives the OPEN CHANNEL proactive command. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

27.22.4.27.2.4	Method of test

27.22.4.27.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The following Bearer Parameters used are those defined in the default Test PDP context for test cases using packet services:

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP ContextDch, as specified in TS 34.123-3 [27], clause 8.10  for test cases using packet services:

Bearer Parameters

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

GPRS Parameters

Network access name:	TestGp.rs

User login:	UserLog

User password:	UserPwd

UICC/ME interface transport level

Transport format:	UDP or TCP mode

Port number:	44444

Data destination address	01.01.01.01 (as an example)

Note:	If a data destination address different to 01.01.01.01 is used then the same value is used in the content of the affected Open Channel commands and the network simulator setup and related UE settings might require a corresponding adaptation.

Prior to test case execution the apparatus supplier shall have provided the "Preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

Pre-condition for successful execution of expected sequence 2.1:

If the terminal does not support the execution of an Open Channel (GPRS) command when no Network Access Name TLV is present in the proactive command and when no default Access Point Name is set in the terminal configuration (s.a. table A.1/48), then "TestGp.rs" shall be set and activated as default Access Point Name in the terminal configuration prior to execution of the proactive command in expected sequence 2.1.

27.22.4.27.2.4.2	Procedure

Expected Sequence 2.1 void

NOTE:	The above sequence has been made void, however the messages defined below are still required for further test sequences.

PROACTIVE COMMAND: OPEN CHANNEL 2.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 36 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

Expected Sequence 2.2 (OPEN CHANNEL, immediate link establishment GPRS, no alpha identifier, with network access name)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.2.1 |  |
| 4 | ME user | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.2.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.2.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 2.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.2.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer Description:

Bearer Type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.2.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer Description:

Bearer Type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

Expected Sequence 2.3 (OPEN CHANNEL, immediate link establishment, GPRS, with alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.3.1 |  |
| 4 | ME user | Confirmation phase with alpha ID | "Open ID" |
| 5 | user ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.1.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 2.3.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	Open ID

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4B | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 35 | 07 | 02 | 03 |
|  | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 | 0A | 06 |
|  | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 | 55 | 73 |
|  | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C | 3E | 05 |
|  | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |  |  |

Expected Sequence 2.4 (OPEN CHANNEL, immediate link establishment, GPRS, with null alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.4.1 |  |
| 4 | ME user | Confirmation phase | [The ME should not give any information] |
| 5 | user ME | The user confirms | [Only if the ME asks for user confirmation] |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.1.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 2.4.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Alpha Identifier	Null

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 44 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 |
|  | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 |
|  | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 |
|  | 01 | AD | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |

Expected Sequence 2.5 (OPEN CHANNEL, immediate link establishment, GPRS, command performed with modifications (buffer size) )

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.5.1 |  |
| 4 | ME user | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.5.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.5.1B | [Command performed with modification] |

PROACTIVE COMMAND: OPEN CHANNEL 2.5.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	65535

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | FF | FF |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.5.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications (07)

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	The buffer size TLV shall be attached and contain the value stated in table A.2/29	"Preferred buffer size supported by the terminal for Open Channel command".

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | Note 1 |  |  |  |  |  |  |  |  |  |  |

Note1:	The buffer size TLV shall be attached and contain the value stated in table A.2/29 "Preferred buffer size supported by the terminal for Open Channel command".

TERMINAL RESPONSE: OPEN CHANNEL 2.5.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications (07)

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	The buffer size TLV shall be attached and contain the value stated in table A.2/29	"Preferred buffer size supported by the terminal for Open Channel command".

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | Note 1 |  |  |  |  |  |  |  |  |  |  |

Note1:	The buffer size TLV shall be attached and contain the value stated in table A.2/29 "Preferred buffer size supported by the terminal for Open Channel command".

Expected Sequence 2.6 Void

Expected Sequence 2.7A (OPEN CHANNEL, immediate link establishment, GPRS, open command with alpha identifier, User did not accept the proactive command)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.7.1 |  |
| 4 | ME user | Confirmation phase with alpha ID | [The ME shall display "Open ID"] |
| 5 | user ME | The user rejects |  |
| 6 | ME USS | No PDP context activation request is sent to the USS |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.7.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.7.1B | [User did not accept the proactive command] |

Expected Sequence 2.7B (OPEN CHANNEL, immediate link establishment, GPRS, open command with alpha identifier, User did not accept the proactive command)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.7.1 |  |
| 4 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 5 | USS ME | PDP context activation accept |  |
| 6 | ME user | Confirmation phase with alpha ID | [The ME shall display "Open ID"] |
| 7 | user ME | The user rejects |  |
| 8 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.7.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.7.1B | [User did not accept the proactive command] |

PROACTIVE COMMAND: OPEN CHANNEL 2.7.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		TestGp.rs

Text String:			UserLog (User login)

Text String:			UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4B | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 35 | 07 | 02 | 03 |
|  | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 | 0A | 06 |
|  | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 | 55 | 73 |
|  | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C | 3E | 05 |
|  | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.7.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	User did not accept the proactive command

Channel status	The presence and content of this TLV shall not be verified

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	Because the value depends in this case on the terminal's implementation, it shall be	ignored.

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 1 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | Note 2 |  |
|  | Note1: The presence and content of the Channel Status TLV shall not be verified. Note2: The buffer size TLV shall be present and because the value depends in this case on the terminal's implementation, the value shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.7.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	User did not accept the proactive command

Channel status	The presence and content of this TLV shall not be verifiedBearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	Because the value depends in this case on the terminal's implementation, it shall be	ignored.

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 1 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F | 02 | Note 2 |  |
|  | Note1: The presence and content of the Channel Status TLV shall not be verified. Note2: The buffer size TLV shall be present and because the value depends in this case on the terminal's implementation, the value shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 2.8 Void

Expected Sequence 2.9 (OPEN CHANNEL, immediate link establishment, no alpha identifier, with network access name)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.9.1 |  |
| 4 | ME user | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.9.1A or TERMINAL RESPONSE: OPEN CHANNEL 2.9.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 2.9.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.9.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status		Channel identifier 1 and link established or PDP context activated

Bearer Description:

Bearer Type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.9.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status		Channel identifier 1 and link established or PDP context activated

Bearer Description:

Bearer Type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

Expected Sequence 2.10 (OPEN CHANNEL, multi Open Channel, one in TCP Server mode and one in TCP Client mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.10.1 | TCP server mode |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.10.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.10.1 | [Command performed successfully] TCP in LISTEN state |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 2.10.2 | TCP Client mode |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 2.10.2 |  |
| 8 | ME user | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 2.10.2A or TERMINAL RESPONSE: OPEN CHANNEL 2.10.2B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 2.10.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	Null

Buffer

Buffer size:	1400

UICC/terminal interface transport level

Transport format:	TCP, UICC in server mode

Port number:	3516

Coding:

| BER-TLV: | D0 | 14 | 81 | 03 | 01 | 40 | 00 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 39 | 02 | 05 | 78 | 3C | 03 | 03 | 0D | BC |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.10.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and TCP in LISTEN state

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 41 | 00 | 39 | 02 | 05 | 78 |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 2.10.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.10.2A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status		Channel identifier 2 and link established or PDP context activated

Bearer Description:

Bearer Type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 82 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 2.10.2B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status		Channel identifier 2 and link established or PDP context activated

Bearer Description:

Bearer Type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 82 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.2 to 2.10.

##### 27.22.4.27.3	Open Channel (default bearer)

##### 27.22.4.27.3.1	Open Channel (default bearer, E-UTRAN)

Open Channel for Default (network) Bearer for E-UTRAN is tested in clause 27.22.4.27.6, expected sequences 6.4 and 6.5.

##### 27.22.4.27.3.2	Open Channel (Default bearer, GERAN/UTRAN)

27.22.4.27.3.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.3.2.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.27 and 6.6.27, clause 8.2, clause 8.6, clause 8.7, clause 8.52, clause 8.55, 8.59 and clause 9.2,

27.22.4.27.3.2.3	Test purpose

To verify that the ME allocates the buffer, activates the PDP context and reports the Channel status using TERMINAL RESPONSE (Command performed successfully) to the UICC after the ME receives the OPEN CHANNEL proactive command.

27.22.4.27.3.2.4	Method of test

27.22.4.27.3.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The following Bearer Parameters used are those defined in the default Test PDP context for test cases using packet services:

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP ContextDch, as specified in TS 34.123-3 [27], clause 8.10  for test cases using packet services:

Bearer Parameters

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

GPRS Parameters

Network access name:	TestGp.rs

User login:	UserLog

User password:	UserPwd

UICC/ME interface transport level

Transport format:	TCP mode

Port number:	44444

Data destination address	01.01.01.01 (as an example)

Note:	If a data destination address different to 01.01.01.01 is used then the same value is used in the content of the affected Open Channel commands and the network simulator setup and related UE settings might require a corresponding adaptation.

Pre-condition for successful execution of expected sequence x.1:

If the terminal does not support the execution of an Open Channel (GPRS) command when no Network Access Name TLV is present in the proactive command and when no default Access Point Name is set in the terminal configuration (s.a. table A.1/48), then "TestGp.rs" shall be set and activated as default Access Point Name in the terminal configuration prior to execution of the proactive command in expected sequence x.1.

27.22.4.27.3.2.4.2	Procedure

Expected Sequence 3.1 (OPEN CHANNEL, Default Bearer, GPRS, with null alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 3.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 3.1.1 |  |
| 4 | ME User | [The ME should not give any information] | [If the ME ask for user confirmation, then the user shall confirm the Open Channel request] |
| 5 | ME USS | PDP context activation request | [The ME may have activated a PDP context at earlier stage. In this case a PDP context activation at this point might not be required if the existing PDP context is reused.] [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 3.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 3.1.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 3.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment, automatic reconnection

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier			Null

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

UICC/ME interface transport level

Transport format:	TCP, UICC in client mode, remote connection

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 1E | 81 | 03 | 01 | 40 | 03 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 35 | 01 | 03 | 39 | 02 | 05 | 78 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment, automatic reconnection

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 01 | 03 | 39 | 02 | 05 | 78 |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment, automatic reconnection

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 03 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.3.2.5	Test requirement

The ME shall operate in the manner defined in expected sequence 3.1.

##### 27.22.4.27.4	Open Channel (Local Bearer)

TBD

##### 27.22.4.27.5	Open Channel (GPRS, support of Text Attribute)

27.22.4.27.5.1	Open Channel (GPRS, support of Text Attribute – Left Alignment)

27.22.4.27.5.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.1.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.1.3	Test purpose

To verify that the ME displays an alpha identifier according to the left alignment text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.1.4	Method of test

27.22.4.27.5.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.1.4.2	Procedure

Expected Sequence 5.1 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.1.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with left alignment] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 5.1.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.1.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.1.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [Message shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/19, no alignment change will take place] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 5.1.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.1.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel

Alpha Identifier	"Close ID"

Coding:

| BER-TLV: | D0 | 14 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 08 | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 5.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.27.5.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.1.

27.22.4.27.5.2	Open Channel (GPRS, support of Text Attribute – Center Alignment)

27.22.4.27.5.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.2.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.2.3	Test purpose

To verify that the ME displays an alpha identifier according to the center alignment text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.2.4	Method of test

27.22.4.27.5.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.2.4.2	Procedure

Expected Sequence 5.2 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.2.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.2.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with center alignment] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.2.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.2.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.2.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.2.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [Message shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/19, no alignment change will take place] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.2.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.2.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 01 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.2.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.2.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.2.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.2.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.2.

27.22.4.27.5.3	Open Channel (GPRS, support of Text Attribute – Right Alignment)

27.22.4.27.5.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.3.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.3.3	Test purpose

To verify that the ME displays an alpha identifier according to the right alignment text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.3.4	Method of test

27.22.4.27.5.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.3.4.2	Procedure

Expected Sequence 5.3 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.3.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.3.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with right alignment] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.3.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.3.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.3.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.3.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [Message shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/19, no alignment change will take place] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.3.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.3.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.3.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 02 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.3.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.3.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.3.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.3.

27.22.4.27.5.4	Open Channel (GPRS, support of Text Attribute – Large Font Size)

27.22.4.27.5.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.4.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.4.3	Test purpose

To verify that the ME displays an alpha identifier according to the large font size text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.4.4	Method of test

27.22.4.27.5.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.4.4.2	Procedure

Expected Sequence 5.4 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.4.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.4.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with large font size] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.4.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.4.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.4.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.4.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with normal font size] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.4.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.4.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.4.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.4.1 |  |
| 32 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with large font size] |
| 33 | USER ME | The user confirms |  |
| 34 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 35 | USS ME | PDP context activation accept |  |
| 36 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.4.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.4.1B | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 40 | ME USS | PDP context deactivation request |  |
| 41 | USS ME | PDP context deactivation accept |  |
| 42 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.4.3 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.4.3 |  |
| 46 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with normal font size] |
| 47 | USER ME | The user confirms |  |
| 48 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 49 | USS ME | PDP context activation accept |  |
| 50 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.4.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.4.1B | [Command performed successfully] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 54 | ME USS | PDP context deactivation request |  |
| 55 | USS ME | PDP context deactivation accept |  |
| 56 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.4.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 04 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.4.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.4.3

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 3"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 33 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.4.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.4.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.4.

27.22.4.27.5.5	Open Channel (GPRS, support of Text Attribute – Small Font Size)

27.22.4.27.5.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.5.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.5.3	Test purpose

To verify that the ME displays an alpha identifier according to the small font size text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.5.4	Method of test

27.22.4.27.5.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.5.4.2	Procedure

Expected Sequence 5.5 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.5.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.5.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with small font size] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.5.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.5.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.5.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.5.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with normal font size] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.5.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.5.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.5.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.5.1 |  |
| 32 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with small font size] |
| 33 | USER ME | The user confirms |  |
| 34 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 35 | USS ME | PDP context activation accept |  |
| 36 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.5.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.5.1B | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 40 | ME USS | PDP context deactivation request |  |
| 41 | USS ME | PDP context deactivation accept |  |
| 42 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.5.3 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.5.3 |  |
| 46 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with normal font size] |
| 47 | USER ME | The user confirms |  |
| 48 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 49 | USS ME | PDP context activation accept |  |
| 50 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.5.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.5.1B | [Command performed successfully] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 54 | ME USS | PDP context deactivation request |  |
| 55 | USS ME | PDP context deactivation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.5.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 08 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.5.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.5.3

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 3"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 33 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.5.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.5.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.5.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.5.

27.22.4.27.5.6	Open Channel (GPRS, support of Text Attribute – Bold On)

27.22.4.27.5.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.6.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.6.3	Test purpose

To verify that the ME displays an alpha identifier according to the bold text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.6.4	Method of test

27.22.4.27.5.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.6.4.2	Procedure

Expected Sequence 5.6 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.6.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.6.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with bold on] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.6.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.6.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.6.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.6.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with bold off] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.6.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.6.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.6.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.6.1 |  |
| 32 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with bold on] |
| 33 | USER ME | The user confirms |  |
| 34 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 35 | USS ME | PDP context activation accept |  |
| 36 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.6.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.6.1B | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 40 | ME USS | PDP context deactivation request |  |
| 41 | USS ME | PDP context deactivation accept |  |
| 42 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.6.3 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.6.3 |  |
| 46 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with bold off] |
| 47 | USER ME | The user confirms |  |
| 48 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 49 | USS ME | PDP context activation accept |  |
| 50 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.6.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.6.1B | [Command performed successfully] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 54 | ME USS | PDP context deactivation request |  |
| 55 | USS ME | PDP context deactivation accept |  |
| 56 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.6.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 10 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.6.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.6.3

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 3"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 33 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.6.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.6.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.6.

27.22.4.27.5.7	Open Channel (GPRS, support of Text Attribute – Italic On)

27.22.4.27.5.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.7.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.7.3	Test purpose

To verify that the ME displays an alpha identifier according to the italic text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.7.4	Method of test

27.22.4.27.5.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.7.4.2	Procedure

Expected Sequence 5.7 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.7.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.7.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with italic on] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.7.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.7.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.7.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.7.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with italic off] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.7.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.7.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.7.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.7.1 |  |
| 32 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with italic on] |
| 33 | USER ME | The user confirms |  |
| 34 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 35 | USS ME | PDP context activation accept |  |
| 36 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.7.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.7.1B | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 40 | ME USS | PDP context deactivation request |  |
| 41 | USS ME | PDP context deactivation accept |  |
| 42 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.7.3 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.7.3 |  |
| 46 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with italic off] |
| 47 | USER ME | The user confirms |  |
| 48 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 49 | USS ME | PDP context activation accept |  |
| 50 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.7.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.7.1B | [Command performed successfully] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 54 | ME USS | PDP context deactivation request |  |
| 55 | USS ME | PDP context deactivation accept |  |
| 56 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.7.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 20 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.7.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.7.3

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 3"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 33 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.7.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.7.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.7.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.7.

27.22.4.27.5.8	Open Channel (GPRS, support of Text Attribute – Underline On)

27.22.4.27.5.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.8.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.8.3	Test purpose

To verify that the ME displays an alpha identifier according to the underline text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.8.4	Method of test

27.22.4.27.5.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.8.4.2	Procedure

Expected Sequence 5.8 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.8.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.8.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with underline on] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.8.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.8.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.8.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.8.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with underline off] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 5.8.1A or TERMINAL RESPONSE: OPEN CHANNEL 5.8.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.8.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.8.1 |  |
| 32 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with underline on] |
| 33 | USER ME | The user confirms |  |
| 34 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 35 | USS ME | PDP context activation accept |  |
| 36 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.8.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.8.1B | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 40 | ME USS | PDP context deactivation request |  |
| 41 | USS ME | PDP context deactivation accept |  |
| 42 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.8.3 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.8.3 |  |
| 46 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with underline off] |
| 47 | USER ME | The user confirms |  |
| 48 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 49 | USS ME | PDP context activation accept |  |
| 50 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.8.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.8.1B | [Command performed successfully] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 54 | ME USS | PDP context deactivation request |  |
| 55 | USS ME | PDP context deactivation accept |  |
| 56 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.8.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 40 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.8.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.8.3

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 3"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 33 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.8.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.8.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.8.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.8.

27.22.4.27.5.9	Open Channel (GPRS, support of Text Attribute – Strikethrough On)

27.22.4.27.5.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.9.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.9.3	Test purpose

To verify that the ME displays an alpha identifier according to the strikethrough text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.9.4	Method of test

27.22.4.27.5.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.9.4.2	Procedure

Expected Sequence 5.9 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.9.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.9.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with strikethrough on] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.9.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.9.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.9.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.9.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with strikethrough off] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.9.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.9.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.9.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.9.1 |  |
| 32 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with strikethrough on] |
| 33 | USER ME | The user confirms |  |
| 34 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 35 | USS ME | PDP context activation accept |  |
| 36 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.9.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.9.1B | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 40 | ME USS | PDP context deactivation request |  |
| 41 | USS ME | PDP context deactivation accept |  |
| 42 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 43 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.9.3 |  |
| 44 | ME UICC | FETCH |  |
| 45 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.9.3 |  |
| 46 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with strikethrough off] |
| 47 | USER ME | The user confirms |  |
| 48 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 49 | USS ME | PDP context activation accept |  |
| 50 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.9.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.9.1B | [Command performed successfully] |
| 51 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 52 | ME UICC | FETCH |  |
| 53 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 54 | ME USS | PDP context deactivation request |  |
| 55 | USS ME | PDP context deactivation accept |  |
| 56 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.9.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 80 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.9.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.9.3

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 3"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 33 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.9.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.9.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.9.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.9.

27.22.4.27.5.10	Open Channel (GPRS, support of Text Attribute – Foreground and Background Colour)

27.22.4.27.5.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.5.10.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.27.5.10.3	Test purpose

To verify that the ME displays an alpha identifier according to the foreground and background colour text attribute configuration in OPEN CHANNEL and returns a successful result in the TERMINAL RESPONSE command send to the UICC.

27.22.4.27.5.10.4	Method of test

27.22.4.27.5.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.27.5.10.4.2	Procedure

Expected Sequence 5.10 (OPEN CHANNEL, immediate link establishment, GPRS, Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING : OPEN CHANNEL 5.10.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.10.1 |  |
| 4 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with foreground and background colour according to the text attribute] |
| 5 | USER ME | The user confirms |  |
| 6 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | USS ME | PDP context activation accept |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.10.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.10.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 12 | ME USS | PDP context deactivation request |  |
| 13 | USS ME | PDP context deactivation accept |  |
| 14 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |
| 15 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 5.10.2 |  |
| 16 | ME UICC | FETCH |  |
| 17 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 5.10.2 |  |
| 18 | ME USER | Confirmation phase with alpha ID | [alpha identifier is displayed with ME's default foreground and background colour] |
| 19 | USER ME | The user confirms |  |
| 20 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 21 | USS ME | PDP context activation accept |  |
| 22 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 5.10.1A or TERMINAL RESPONSE : OPEN CHANNEL 5.10.1B | [Command performed successfully] |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 5.1.1 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 5.1.1 |  |
| 26 | ME USS | PDP context deactivation request |  |
| 27 | USS ME | PDP context deactivation accept |  |
| 28 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 5.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 5.10.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 1"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Text Attribute

Formatting position:	0

Formatting length:	9

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 53 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 31 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 | D0 | 04 | 00 | 09 | 00 |
|  | B4 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL 5.10.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier	"Open ID 2"

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 4D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 05 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 4F | 70 | 65 | 6E | 20 | 49 | 44 | 20 | 32 | 35 | 07 |
|  | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 05 | 78 | 47 |
|  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D |
|  | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 |
|  | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD | 9C |
|  | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.10.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 5.10.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.4.27.5.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 5.10.

##### 27.22.4.27.6	Open Channel (related to E-UTRAN)

27.22.4.27.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.6.2	Conformance requirements

The ME shall support the class "e" commands and E-UTRAN as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 9.2, clause 8.2, clause 8.15, clause 8.52, clause 8.59, clause 8.61 and Annex S

-	TS 23.107 [30], cl 9.1.2.2, clause 9.1.2.3,

-	TS 23.203 [31], cl 6.1.7.2,

-	TS 24.301 [32], cl 9.9.4.3,

-	TS 36.508 [33], cl 6.6.1.

-	TS 24.011 [11], cl 10.

-	TS 31.102 [14], cl 4.2.109

27.22.4.27.6.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (OK); or

-	TERMINAL RESPONSE (Command performed with modification); or

-	TERMINAL RESPONSE (User did not accept the proactive command);

-	TERMINAL RESPONSE (ME currently unable to process command);

to the UICC after the ME receives the OPEN CHANNEL proactive command while accressing E-UTRAN/EPC. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

To verify that the ME sets up a PDN connection with the Access Point Name (APN) indicated in the Open Channel command which differs from the default APN.

To verify that the ME uses the Default EPS bearer when Bearer Type 3 is indicated in the Open Channel command.

To verify that the ME does not disconnect the Deafult EPS bearer when the user rejects the user confirmation of the Open Channel command.

To verify that the ME sends the TERMINAL RESPONSE (ME currently unable to process command) if the 3GPP PS data off status is "active" and the UE is not configured with indication that Bearer Independent Protocol is a 3GPP PS data off exempt service.

To verify that the ME sends the TERMINAL RESPONSE (OK) if the 3GPP PS data off status is "active" and the UE is configured with indication that Bearer Independent Protocol is a 3GPP PS data off exempt service.

To verify that the maximum number of 7 OPEN CHANNEL commands should be handled by the ME.

27.22.4.27.6.4	Method of test

27.22.4.27.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:		Any value other than TestGp.rs or Test12.rs

User login:			UserLog

User password:			UserPwd

UICC/ME interface transport level

Transport format:		TCP

Port number:		44444

Data destination address:	01.01.01.01 (as an example)

Note:	If a data destination address different to 01.01.01.01 is used then the same value is used in the content of the affected Open Channel commands and the network simulator setup and related UE settings might require a corresponding adaptation.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

Prior to test case execution the apparatus supplier shall have provided the "Preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

For sequence 6.1, 6.2 and 6.3 the E-USS shall be able to support 2 active PDN connections at the same time.

In case the ME supports A.1/173 AND A.1/174 AND A.1/178, for sequence 6.1 and 6.3 the NB-SS shall be able to support 2 active PDN connections at the same time.

For sequence 6.6 service n°117 is "available" in EFUST and the Bearer Independent Protocol is not marked in EF3GPPPSDATAOFF as a 3GPP PS data off exempt service. Also SMS over SGs is used to send and receive short messages.

For sequence 6.7 service n°117 is "available" in EFUST and the Bearer Independent Protocol is marked in EF3GPPPSDATAOFF as a 3GPP PS data off exempt service. Also SMS over SGs is used to send and receive short messages.

27.22.4.27.6.4.2	Method of test

Expected Sequence 6.1 (OPEN CHANNEL, immediate link establishment, E-UTRAN, bearer type '02')

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 only one APN will be activated in step 7. |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.1.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.1.1 |  |
| 5 | ME USER | The ME may display channel opening information |  |
| 6 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 8 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 9 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |

PROACTIVE COMMAND: OPEN CHANNEL 6.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		TestGp.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

Expected Sequence 6.2 (OPEN CHANNEL, immediate link establishment, E-UTRAN, bearer type '0B')

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" and "Test12.rs"in the terminal configuration if required | [see initial conditions] |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.2.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.2.1 | The "TestGp.rs" APN is requested |
| 5 | ME USER | The ME may display channel opening information |  |
| 6 | ME E-USS | PDN CONNECTIVITY REQUEST | The PDN CONNECTIVITY REQUEST shall contain APN value "TestGp.rs" [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | E-USS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used with the exception that the "EPS Quality of Service" information element contains QCI = 9 and the maximum and guaranteed bit rates for uplink and downlink shall all be set to 64kbps. The bytes for the extened bit rate values shall not be present in the "EPS Quality of Service" IE] |
| 8 | ME E-USS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 9 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.2.1A OR TERMINAL RESPONSE : OPEN CHANNEL 6.2.1B | [Command performed successfully OR Command performed with modifications] |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.1.1 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND : CLOSE CHANNEL 3.1.1 | The ME can deactivate the EPS bearer |
| 13 | ME UICC | TERMINAL RESPONSE : CLOSE CHANNEL 3.1.1 |  |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.2.2 | The "Test12.rs" APN is requested |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.2.2 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME E-USS | PDN CONNECTIVITY REQUEST | The PDN CONNECTIVITY REQUEST shall contain APN value "Test12.rs" [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | E-USS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used with the exception that the "EPS Quality of Service" information element contains only the QCI which shall be set to "9"] [second PDN context activated] |
| 20 | ME E-USS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 21 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.2.2A OR TERMINAL RESPONSE : OPEN CHANNEL 6.2.2B | [Command performed successfully OR Command performed with modifications] |

PROACTIVE COMMAND: OPEN CHANNEL 6.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:				E-UTRAN / mapped UTRAN packet service

QCI			9

Maximum bit rate for uplink:			0 (Subscribed maximum bit rate for uplink)

Maximum bit rate for downlink:		0 (Subscribed maximum bit rate for downlink)

Guaranteed bit rate for uplink:			0 (Use the value indicated by the maximum bit rate for uplink)

Guaranteed bit rate for downlink:	0 (Use the value indicated by the maximum bit rate for downlink)

Maximum bit rate for uplink (extended):	0

Maximum bit rate for downlink (extended):	0

Guaranteed bit rate for uplink (extended):	0

Guaranteed bit rate for downlink (extended):	0

PDN Type:			IP

Buffer

Buffer size:	1400

Network access name:		TestGp.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 46 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 0B | 09 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 02 |
|  | 39 | 02 | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 |
|  | 70 | 02 | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C |
|  | 6F | 67 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 |
|  | 3C | 03 | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |

TERMINAL RESPONSE: OPEN CHANNEL 6.2.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:				E-UTRAN / mapped UTRAN packet service

QCI			9

PDN Type:			IP

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 03 | 0B | 09 | 02 | 39 | 02 | 05 |
|  | 78 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.2.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:				E-UTRAN / mapped UTRAN packet service

QCI			9

PDN Type:			IP

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 03 | 0B | 09 | 02 | 39 | 02 | 05 |
|  | 78 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1

Same as PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1 in clause 27.22.4.28.3

TERMINAL RESPONSE: CLOSE CHANNEL 3.1.1

Same as TERMINAL RESPONSE: CLOSE CHANNEL 3.1.1 in clause 27.22.4.28.3

PROACTIVE COMMAND: OPEN CHANNEL 6.2.2

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:			E-UTRAN / mapped UTRAN packet service

QCI		9

Maximum bit rate for uplink:		0 (Subscribed maximum bit rate for uplink)

Maximum bit rate for downlink:	0 (Subscribed maximum bit rate for downlink)

Guaranteed bit rate for uplink:	0 (Use the value indicated by the maximum bit rate for uplink)

Guaranteed bit rate for downlink:	0 (Use the value indicated by the maximum bit rate for downlink)

Maximum bit rate for uplink (extended):	0

Maximum bit rate for downlink (extended):	0

Guaranteed bit rate for uplink (extended):	0

Guaranteed bit rate for downlink (extended):	0

PDN Type:		IP

Buffer

Buffer size:	1400

Network access name:		Test12.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 46 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 0B | 09 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 02 |
|  | 39 | 02 | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 31 |
|  | 32 | 02 | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C |
|  | 6F | 67 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 |
|  | 3C | 03 | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |

TERMINAL RESPONSE: OPEN CHANNEL 6.2.2A

same as TERMINAL RESPONSE: OPEN CHANNEL 6.2.1A

TERMINAL RESPONSE: OPEN CHANNEL 6.2.2B

same as TERMINAL RESPONSE: OPEN CHANNEL 6.2.1B

Expected Sequence 6.3 (OPEN CHANNEL, immediate link establishment, E-UTRAN, bearer type '02', with Network Access Name, with alpha identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 only one APN will be activated in step 7. |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.3.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.3.1 |  |
| 5 | ME USER | The terminal shall display the alpha identifier "Open Channel for UICC?" during the confirmation phase | [IF NOT A.1/84 (No display) THEN the terminal shall ignore the alpha identifier] |
| 6 | USER ME | The user confirms | [IF NOT A.1/85 (No keypad) THEN the terminal may open the channel without explicit confirmation by the user] |
| 7 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The PDN CONNECTIVITY REQUEST shall contain the APN "Test12.rs"] [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 8 | USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 9 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 10 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |

PROACTIVE COMMAND: OPEN CHANNEL 6.3.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier:	"Open Channel for UICC?"

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		Test12.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 5A | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 4F | 70 | 65 | 6E | 20 | 43 | 68 | 61 | 6E | 6E | 65 |
|  | 6C | 20 | 66 | 6F | 72 | 20 | 55 | 49 | 43 | 43 | 3F | 35 |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 31 | 32 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

Expected Sequence 6.4 (OPEN CHANNEL, immediate link establishment, E-UTRAN, bearer type '03', with alpha identifier, user did not accept the proactive command)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 no APN will be activated in this step. |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.4.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.4.1 |  |
| 5 | ME USER | The terminal shall display the alpha identifier "Open Channel for UICC?" during the confirmation phase |  |
| 6 | USER ME | The user rejects |  |
| 7 | ME E-USS/NB-SS | The terminal shall not send a PDN CONNECTIVITY REQUEST to the network |  |
| 8 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.4.1 | [User did not accept proactive command] |
| 9 | ME E-USS/NB-SS | The ME shall not send a PDN CONNECTIVITY DISCONNECT REQUEST to the network which would disconnect the default EPS bearer which has been established after the terminal has been powered up. | [Within this period the terminal shall not be switched off] |

PROACTIVE COMMAND: OPEN CHANNEL 6.4.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier:	"Open Channel for UICC?"

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

Network access name:	TestGp.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP, UICC in client mode, remote connection

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 54 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 16 | 4F | 70 | 65 | 6E | 20 | 43 | 68 | 61 | 6E | 6E | 65 |
|  | 6C | 20 | 66 | 6F | 72 | 20 | 55 | 49 | 43 | 43 | 3F | 35 |
|  | 01 | 03 | 39 | 02 | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 |
|  | 74 | 47 | 70 | 02 | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 |
|  | 72 | 4C | 6F | 67 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 |
|  | 77 | 64 | 3C | 03 | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.4.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	User did not accept the proactive command

Channel status	The presence and content of this TLV shall not be verified

Bearer description

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	Because the value depends in this case on the terminal's implementation, it shall be	ignored.

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 1 | 35 | 01 | 03 |  | Note 2 |  |  |  |  |  |  |
|  | Note1: The presence and content of the Channel Status TLV shall not be verified. Note 2: The buffer size TLV shall be present and because the value depends in this case on the terminal's implementation, the value shall be ignored. |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 6.5 (OPEN CHANNEL, immediate link establishment, E-UTRAN, bearer type '03' – Default EPS bearer)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 no APN will be activated in this step. |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.5.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.5.1 |  |
| 5 | ME USER | The ME may display channel opening information |  |
| 6 | ME E-USS/NB-SS | The terminal shall not send a PDN CONNECTIVITY REQUEST to the network Exception: If the ME supports A.1/173 AND NOT A.1/174 PDN CONNECTIVITY REQUEST should be sent by the ME in this step. | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 7 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.5.1A or TERMINAL RESPONSE : OPEN CHANNEL 6.5.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 6.5.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

UICC/ME interface transport level

Transport format:	TCP, UICC in client mode, remote connection

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 03 | 39 | 02 | 05 | 78 | 3C | 03 | 02 | AD | 9C | 3E |
|  | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.5.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 01 | 03 | 39 | 02 | 05 | 78 |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.5.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:				E-UTRAN / mapped UTRAN packet service

QCI			9

Maximum bit rate for uplink:			64 kbps

Maximum bit rate for downlink:		64 kbps

Guaranteed bit rate for uplink:			64 kbps

Guaranteed bit rate for downlink:	64 kbps

Maximum bit rate for uplink (extended):	0

Maximum bit rate for downlink (extended):	0

Guaranteed bit rate for uplink (extended):	0

Guaranteed bit rate for downlink (extended):	0

PDN Type:			IP

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 0B | 0B | 09 | 40 | 40 | 40 | 40 |
|  | 00 | 00 | 00 | 00 | 02 | 39 | 02 | 05 | 78 |  |  |  |

Expected Sequence 6.6 (OPEN CHANNEL, BIP is not a 3GPP PS data off exempt service)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 2 | USER ME | Set 3GPP PS data off status is "active" |  |
| 3 | ME E-USS | Send a Request Bearer Resource Modification message | The ME indicates the change of 3GPP PS Data Off Status to the PDN GW via the PCO (Protocol Configuration Options) |
| 4 | E-USS ME | SMS-PP Data Download 6.6.1 | Send SMS over SGs |
| 5 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 6.6.1 |  |
| 6 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 7 | ME E-USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.1.1 |  |
| 11 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.6.1 | [ME currently unable to process command] |

SMS-PP (Data Download) Message 6.6.1

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| Coding | 04 | 04 | 91 | 21 | 43 | 7F | 16 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 53 | 68 | 6F | 72 | 74 | 20 | 4D | 65 | 73 |
|  | 73 | 61 | 67 | 65 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 6.6.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 21 | 43 |
|  | 7F | 16 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 53 | 68 |
|  | 6F | 72 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.6.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	ME currently unable to process command

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 20 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 6.7 (OPEN CHANNEL, BIP is a 3GPP PS data off exempt service)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 2 | USER ME | Set 3GPP PS data off status is "active" |  |
| 3 | ME E-USS | Send a Request Bearer Resource Modification message | The ME indicates the change of 3GPP PS Data Off Status to the PDN GW via the PCO (Protocol Configuration Options) |
| 4 | E-USS ME | SMS-PP Data Download 6.6.1 | Send SMS over SGs |
| 5 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 6.6.1 |  |
| 6 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 7 | ME E-USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 6.1.1 |  |
| 11 | ME USER | The ME may display channel opening information |  |
| 12 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 13 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 14 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 15 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |

Expected Sequence 6.8 (OPEN CHANNEL, Maximum number of open channel requests)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 7 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 8 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1A | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 12 | ME USER | The ME may display channel opening information |  |
| 13 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1B | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1C | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 22 | ME USER | The ME may display channel opening information |  |
| 23 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1D | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 27 | ME USER | The ME may display channel opening information |  |
| 28 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1E | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 29 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 32 | ME USER | The ME may display channel opening information |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1F | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 37 | ME USER | The ME may display channel opening information |  |
| 38 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1G | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 39 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.8.1 |  |
| 40 | ME UICC | FETCH |  |
| 41 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.8.1 |  |
| 42 | ME USER | The ME may display channel opening information |  |
| 43 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.8.1H | [Command performed successfully]. No channel Available |

PROACTIVE COMMAND: OPEN CHANNEL 6.8.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 2 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 82 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1C

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 3 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 83 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1D

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 4 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 84 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1E

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 5 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 85 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1F

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 6 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 86 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 6.8.1G

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 7 and link established or PDP context activated

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 87 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL6.8.1H

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Bearer Independent Protocol Error – No channel Available

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 02 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 |
|  | 05 | 78 |  |  |  |  |  |  |  |  |  |  |

27.22.4.27.6.5	Test requirement

The ME shall operate in the manner defined in expected sequences 6.1 to 6.8.

##### 27.22.4.27.7	Open Channel (UICC Access to IMS)

27.22.4.27.7.1	Open Channel UICC Access to IMS (UICC IARI on USIM)

27.22.4.27.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.7.2	Conformance requirements

The ME shall support the Open Channel for IMS and Event Download – IMS Registration Event commands as defined in:

-	TS 31.111[15] clauses 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 8.55, clause 8.110

-	TS 31.102 [14] clauses 4.2.8, 4.2.95

The ME shall support the EFUICCIARI reading procedure as defined in:

-	TS 31.102 [14] clause 5.3.42

The ME shall support the EVENT: IMS registration as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 7.5, clause 8.7, clause 8.25, clause 8.111, clause 8.112.-	TS 34.229-1 [36] Annex C.2

Additionally the ME shall be able to carry out the IMS registration procedure according to TS 34.229-1 [36], Annex C.2.

27.22.4.27.7.3	Test purpose

To verify that the ME shall

- open a channel to communicate with the IMS and

- send a TERMINAL RESPONSE (OK) upon successful command execution

to the UICC after the ME receives the OPEN CHANNEL for IMS proactive command.

To verify that when the no ISIM is available the ME reads and uses the IARI stored in the UICC IARI list stored on the USIM if service n°95 is "available" in the USIM service table.

To verify that the ME informs the UICC that an Event: IMS registration has occurred using the ENVELOPE (EVENT DOWNLOAD – IMS registration) command when the ME received a SIP message with Registration information and that it includes the list of active IMPUs.

Note:	Verification of correct Open Channel for IMS support in combination with the UICC IARI list stored on the ISIM is verified in clause 27.22.7.20.

27.22.4.27.7.4	Method of test

27.22.4.27.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the Network Simulator (NWS).

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The channel identifier value used for these tests is set to 1 as an example. This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

Prior to test case execution the apparatus supplier shall have provided the "preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

The USIM contains an IMS subscription, with following IMPU registered in the IM CN subsystem:

sip:uicctest@ims.3gpp.org

The default USIM with the following execptions is used:

EFUST (USIM Service Table)

EFUST shall be configured as defined in 27.22.2A with the exception that Service 95 "support of UICC access to IMS" is available.

EFUICCIARI (UICC IARI list)

Record 1 :

Logically:	urn:ur-7:3gpp-application.ims.iari.uicctest

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 2B | 75 | 72 | 6E | 3A | 75 | 72 | 2D | 37 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 3A | 33 | 67 | 70 | 70 | 2D | 61 | 70 | 70 | 6C |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 69 | 63 | 61 | 74 | 69 | 6F | 6E | 2E | 69 | 6D |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 73 | 2E | 69 | 61 | 72 | 69 | 2E | 75 | 69 | 63 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | 63 | 74 | 65 | 73 | 74 | FF | FF | FF | FF | FF |

27.22.4.27.7.4.2	Procedure

Expected Sequence 7.1 (OPEN CHANNEL for IMS, IARI list stored on the USIM)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 7.1.1 | [As response to the TERMINAL PROFILE command] |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 7.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 7.1.1 | [The ME will read the USIM Service Table and the UICC IARI list on the USIM before it will attempt the initial registration to the IMS network] |
| 5 | ME NWS | ME attempts the intial registration to the IMS network. | [The SIP REGISTER for the intial registration may not contain the UICC IARI from the USIM] |
| 6 | NWS ME | IMS network sends SIP message with error code 504 (Server-Time-Out) | [IMS registration failed] |
| 7 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – IMS registration 7.1.1 | [Contains IMS status code 504] |
| 8 | USER ME | Try to initiate another initial IMS registration, e.g. deactivate and reactivate the radio interface | [To trigger an IMS registration attempt. If no option exists to deactivate and reactivate the radio interface separately, the ME could also be switched off and then on again] |
| 9 | ME NWS NWS  ME | ME attempts to register to IMS services with values derived from the USIM and additionally registers the IARI from EFUICCIARI during the intial registration or subsequent registration to IMS services. | [Initial registration to the IMS network is performed according to TS 34.229-1 [36], Annex C.2. The ME will have read the USIM Service Table and the UICC IARI list on the USIM before it will attempt the initial registration to the IMS network] |
| 10 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – IMS registration 7.1.2 | [After the IARI "urn:ur-7:3gpp-application.ims.iari.uicctest" has been successfully registered during the intial or a subsequent SIP REGISTER message containing this IARI. If the IARI "urn:ur-7:3gpp-application.ims.iari.uicctest" is not registered during the intial registration to the IMS network further Envelopes – Event Download – IMS Registration without the IARI might have been received. These shall be ignored by the USIM Simulator.] |
| 11 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 7.1.1 |  |
| 12 | ME UICC | FETCH |  |
| 13 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL for IMS 7.1.1 |  |
| 14 | ME | Channel id, buffer assigned |  |
| 15 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL for IMS 7.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: SET UP EVENT LIST 7.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	IMS Registration Event

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 17 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 7.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - IMS Registration 7.1.1

Logically:

Event list

Event 1:	IMS Registration

Device identities

Source device:	Network

Destination device:	UICC

IMS status code :	504 (Server-Time-Out)

Coding:

| BER-TLV: | D6 | 0C | 19 | 01 | 17 | 82 | 02 | 83 | 81 | 78 | 03 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 30 | 34 |  |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - IMS Registration 7.1.2

Logically:

Event list

Event 1:	IMS Registration

Device identities

Source device:	Network

Destination device:	UICC

IMPU list :	At least one IMPU containing "urn:ur-7:3gpp-application.ims.iari.uicctest"

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 17 | 82 | 02 | 83 | 81 | 77 | Note 2 | Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Note 1: The TLV length depends on the IMPU list content Note 2: The IMPU TLV length depends on the IMPU list entries. Note 3: The IMPU list shall contain the IMPU "urn:ur-7:3gpp-application.ims.iari.uicctest" and might contain further IMPUs |  |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL for IMS 7.1.1

Logically:

Command details

Command number:	01

Command type:	OPEN CHANNEL

Command qualifier:	00 (RFU)

Device identities

Source device:	UICC

Destination device:	ME

Buffer

Buffer size:	1400

IARI	urn:ur-7:3gpp-application.ims.iari.uicctest

Coding:

| BER-TLV: | D0 | 3A | 81 | 03 | 01 | 40 | 00 | 82 | 02 | 81 | 82 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 05 | 78 | 76 | 2B | 75 | 72 | 6E | 3A | 75 | 72 | 2D |
|  | 37 | 3A | 33 | 67 | 70 | 70 | 2D | 61 | 70 | 70 | 6C | 69 |
|  | 63 | 61 | 74 | 69 | 6F | 6E | 2E | 69 | 6D | 73 | 2E | 69 |
|  | 61 | 72 | 69 | 2E | 75 | 69 | 63 | 63 | 74 | 65 | 73 | 74 |

TERMINAL RESPONSE: OPEN CHANNEL 7.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	00 (RFU)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1, link established.

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 39 | 02 | 05 | 78 |  |  |  |  |

27.22.4.27.7.5	Test requirement

The ME shall operate in the manner defined in expected sequence 7.1.

##### 27.22.4.27.8	Open Channel (related to NG-RAN)

27.22.4.27.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.27.8.2	Conformance requirements

The ME shall support the class "e" commands and NG-RAN as defined in:

-	TS 31.111[15] clause 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 9.2, clause 8.2, clause 8.15, clause 8.52, clause 8.59, clause 8.61,

-	TS 24.501 [40], clauses 6.2.2 and 6.2.3,

-	TS 23.501 [41], clauses 5.15.5.2 and 5.15.5.3,

-	TS 23.503 [42], clause 6.6.2,

-	TS 24.526 [43], clause 4.1, 4.2.2, 5.2 and 5.15.5.3,

-	TS 23.003 [44], clause 9A,

27.22.4.27.8.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (OK); or

-	TERMINAL RESPONSE (Command performed with modification); or

-	TERMINAL RESPONSE (User did not accept the proactive command);

-	TERMINAL RESPONSE (ME currently unable to process command);

to the UICC after the ME receives the OPEN CHANNEL proactive command while accressing NG-RAN/5GC. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

To verify that the ME establishes a PDU session with the parameters indicated in the Open Channel command and additional parameters from matching URSP rule when Bearer Types 0x02 or 0x0C is indicated in the Open Channel command.

To verify the pre-configured policy is applied by the UE only when the UE has not received the same type of policy from the Network (PCF).

To verify that the ME does not setup a new PDU session and uses the Default or Always-on PDU session when Bearer Type 3 is indicated in the Open Channel command.

To verify that the ME does not disconnect the Deafult or Always-on PDU session when the user rejects the user confirmation of the Open Channel command.

27.22.4.27.8.4	Method of test

27.22.4.27.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the NG-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default NG-RAN UICC, the default NG-RAN parameters and the following URSP rules stored in the ME are used:

URSP:

Rule Precedence =1

Traffic Descriptor:

DNN=TestGp.rs

Route Selection Descriptor:

Precedence=1

Network Slice Selection, S-NSSAI : 01 01 01 02 (ST:MBB, SD: 010102)

SSC Mode Selection: SSC Mode 2

Access Type preference: 3GPP access

Rule Precedence = <lowest priority>

Traffic Descriptor: *

Route Selection Descriptor:

Precedence =1

Network Slice Selection, S-NSSAI : 01 01 01 01 (ST: MBB, SD: 010101)

SSC Mode Selection: SSC Mode 3

DNN Selection: internet

The Allowed S-NSSAI list is configured in NG-SS as ‘01 01 01 01’, ‘01 01 01 02’and ‘01 01 01 03’

For sequence 8.2 and 8.3 the NG-SS shall be able to support 2 active PDU sessions at the same time.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME’s default channel identifier as declared in table A.2/27.

Prior to test case execution the apparatus supplier shall have provided the "Preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

27.22.4.27.8.4.2	Procedure

Expected Sequence 8.1 (OPEN CHANNEL, immediate link establishment, NG-RAN, bearer type '03' – Default PDU Session)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Always-on PDU session using DNN “internet” is configured in the terminal | [see initial conditions] |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | Always-on Internel PDU Session is established successfully. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 8.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 8.1.1 |  |
| 7 | ME USER | The ME may display channel opening information |  |
| 8 | ME NG-SS | The terminal shall not send a PDU SESSION ESTABLISHMENT REQUEST to the network |  |
| 9 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 8.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 8.1.1

Same as in PROACTIVE COMMAND: OPEN CHANNEL 6.5.1 in sequence 6.5

TERMINAL RESPONSE: OPEN CHANNEL 8.1.1

Same as in TERMINAL RESPONSE: OPEN CHANNEL 6.5.1A in sequence 6.5

Expected Sequence 8.2 (OPEN CHANNEL, immediate link establishment, NG-RAN, bearer type '0C')

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Always-on PDU session using DNN “internet” is configured in the terminal. | [see initial conditions] |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | Always-on Internel PDU Session is established successfully. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 8.2.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 8.2.1 |  |
| 7 | ME USER | The ME may display channel opening information |  |
| 8 | ME NG-SS | PDU SESSION ESTABLISH REQUEST is sent to the network. | DNN=TestGp.rs, S-NSSAI=’01 01 01 02’, SSC mode=2. |
| 9 | NG-SS ME | PDU SESSION ESTABLISH ACCEPT |  |
| 10 | ME NG-SS | PDU SESSION ESTABLISH COMPLETE |  |
| 11 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 8.2.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 8.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer description

Bearer type:	NG-RAN

Bearer parameter:

PDU Session Type:	02

Buffer

Buffer size:	1400

Network access name: 	TestGp.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number: 	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 3D | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 0C | 02 | 39 | 02 | 05 | 78 | 47 | 0A | 06 | 54 | 65 |
|  | 73 | 74 | 47 | 70 | 02 | 72 | 73 | 0D | 08 | F4 | 55 | 73 |
|  | 65 | 72 | 4C | 6F | 67 | 0D | 08 | F4 | 55 | 73 | 65 | 72 |
|  | 50 | 77 | 64 | 3C | 03 | 02 | AD | 9C | 3E | 05 | 21 | 01 |
|  | 01 | 01 | 01 |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 8.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated or PDU Session established

Bearer description

Bearer type:	NG-RAN

Bearer parameter:

PDU Session Type:	02

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 02 | 0C | 02 | 39 | 02 | 05 | 78 |

Expected Sequence 8.3 (OPEN CHANNEL, NG-RAN, bearer type '0C', after receiving policy update for URSP from network)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Always-on PDU session using DNN “internet” is configured in the terminal. | [see initial conditions] |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | Always-on Internel PDU Session is established successfully. |  |
| 4 | NG-SS -> ME | MANAGE UE POLICY COMMAND to update URSP rule for DNN: "TestGp.rs" | Traffic Descriptor: DNN: ‘TestGp.rs’ Route Selection Descriptor: S-NSSAI: ’01 01 01 03’ |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 8.3.1 |  |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 8.3.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME NG-SS | PDU SESSION ESTABLISH REQUEST is sent to the network. | DNN=TestGp.rs, S-NSSAI=’01 01 01 03’, SSC mode=2. |
| 10 | NG-SS ME | PDU SESSION ESTABLISH ACCEPT |  |
| 11 | ME NG-SS | PDU SESSION ESTABLISH COMPLETE |  |
| 12 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 8.3.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 8.3.1

Same as PROACTIVE COMMAND: OPEN CHANNEL 8.2.1 in sequence 8.2

TERMINAL RESPONSE: OPEN CHANNEL 8.3.1

Same as TERMINAL RESPONSE: OPEN CHANNEL 8.2.1 in sequence 8.2

Expected Sequence 8.4 (OPEN CHANNEL, NG-RAN, bearer type '0C', PDU Session is already available for the same DNN

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure URSP rules with DNN "TestGp.rs" in the terminal configuration if required. Always-on PDU session using DNN “TestGp.rs” is configured in the terminal | [see initial conditions] DNN: “TestGp.rs” for Always on PDU |
| 2 | ME NG-SS | ME successfully REGISTER with NG-RAN cell. |  |
| 3 | ME NG-SS | Always-on Internel PDU Session is established successfully. |  |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 8.4.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 8.4.1 |  |
| 7 | ME USER | The ME may display channel opening information |  |
| 8 | ME NG-SS | The terminal shall not send a PDU SESSION ESTABLISHMENT REQUEST to the network |  |
| 9 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 8.4.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 8.4.1

Same as PROACTIVE COMMAND: OPEN CHANNEL 8.2.1 in sequence 8.2

TERMINAL RESPONSE: OPEN CHANNEL 8.4.1

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.5.1A in sequence 6.5

#### 27.22.4.28	CLOSE CHANNEL

##### 27.22.4.28.1	CLOSE CHANNEL(normal)

27.22.4.28.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.1.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.1.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (Command Performed Successfully); or

-	TERMINAL RESPONSE (Bearer Independent Protocol Error);

to the UICC after the ME receives the CLOSE CHANNEL proactive command. The TERMINAL RESPONSE sent back to the UICC is function of the ME and the network capabilities against asked parameters by the UICC.

27.22.4.28.1.4	Method of Test

27.22.4.28.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27

The following Bearer Parameters used are those defined in the default Test PDP context for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.1.4.2	Procedure

Expected sequence 1.1 (CLOSE CHANNEL, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.1.1 |  |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected sequence 1.2 (CLOSE CHANNEL, with an invalid channel identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.2.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.2.1 |  |
| 11 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.2.1 | [Invalid channel number] |

PROACTIVE COMMAND: CLOSE CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 2

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Bearer Independent Protocol error

Additional Result:	Channel identifier not valid

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 |  |  |  |  |  |  |  |  |  |  |  |

Expected sequence 1.3 (CLOSE CHANNEL, on an already closed channel)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.1.1 |  |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.1.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.3.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.3.1 |  |
| 17 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.3.1A or TERMINAL RESPONSE CLOSE CHANNEL 1.3.1B | [Channel closed] [Channel identifier invalid] |

PROACTIVE COMMAND: CLOSE CHANNEL 1.3.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 1.3.1A

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Bearer Independent Protocol error

Additional Result:	Channel closed

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 |  |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 1.3.1B

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Bearer Independent Protocol error

Additional Result:	Channel identifier invalid

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 |  |  |  |  |  |  |  |  |  |  |  |

27.22.4.28.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.3.

##### 27.22.4.28.2	CLOSE CHANNEL (support of Text Attribute)

27.22.4.28.2.1	CLOSE CHANNEL (support of Text Attribute – Left Alignment)

27.22.4.28.2.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.1.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.1.3	Test purpose

To verify that the ME shall display the alpha identifier according to the left alignment text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.1.4	Method of Test

27.22.4.28.2.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.1.4.2	Procedure

Expected sequence 2.1 (CLOSE CHANNEL, with Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.1.1A |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.1.1 | [alpha identifier is displayed with left alignment] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.1.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.1.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.1.2 | [Message shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/20, no alignment change will take place] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.1.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.1.

27.22.4.28.2.2	CLOSE CHANNEL (support of Text Attribute – Center Alignment)

27.22.4.28.2.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.2.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.2.3	Test purpose

To verify that the ME shall display the alpha identifier according to the center alignment text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.2.4	Method of Test

27.22.4.28.2.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.2.4.2	Procedure

Expected sequence 2.2 (CLOSE CHANNEL, with Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.2.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.2.1 | [alpha identifier is displayed with center alignment] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.2.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.2.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.2.2 | [Message shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/20, no alignment change will take place] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.2.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.2.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 01 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.2.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.2.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.2.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.2.

27.22.4.28.2.3	CLOSE CHANNEL (support of Text Attribute – Right Alignment)

27.22.4.28.2.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.3.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.3.3	Test purpose

To verify that the ME shall display the alpha identifier according to the right alignment text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.3.4	Method of Test

27.22.4.28.2.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.3.4.2	Procedure

Expected sequence 2.3 (CLOSE CHANNEL, with Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.3.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.3.1 | [alpha identifier is displayed with right alignment] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.3.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.3.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.3.2 | [Message shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/20, no alignment change will take place] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.3.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.3.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 02 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.3.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.3.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.3.

27.22.4.28.2.4	CLOSE CHANNEL (support of Text Attribute – Large Font Size)

27.22.4.28.2.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.4.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.4.3	Test purpose

To verify that the ME shall display the alpha identifier according to the large font size text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.4.4	Method of Test

27.22.4.28.2.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.4.4.2	Procedure

Expected sequence 2.4 (CLOSE CHANNEL, with Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.4.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.4.1 | [alpha identifier is displayed with large font size] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.4.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context deactivation request |  |
| 19 | USS ME | PDP context deactivation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.4.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.4.2 | [alpha identifier is displayed with normal font size] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.4.1 | [Command performed successfully] |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 30 | ME USER | The ME may display channel opening information |  |
| 31 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 32 | USS ME | PDP context activation accept |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.4.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.4.1 | [alpha identifier is displayed with large font size] |
| 37 | ME USS | PDP context deactivation request |  |
| 38 | USS ME | PDP context deactivation accept |  |
| 39 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.4.1 | [Command performed successfully] |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 43 | ME USER | The ME may display channel opening information |  |
| 44 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 45 | USS ME | PDP context activation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 47 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.4.3 |  |
| 48 | ME UICC | FETCH |  |
| 49 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.4.3 | [alpha identifier is displayed with normal font size] |
| 50 | ME USS | PDP context deactivation request |  |
| 51 | USS ME | PDP context deactivation accept |  |
| 52 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.4.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.4.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Large Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 04 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.4.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.4.3

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 3"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 33 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.4.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.4.

27.22.4.28.2.5	CLOSE CHANNEL (support of Text Attribute – Small Font Size)

27.22.4.28.2.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.5.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.5.3	Test purpose

To verify that the ME shall display the alpha identifier according to the small font size text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.5.4	Method of Test

27.22.4.28.2.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.5.4.2	Procedure

Expected sequence 2.5 (CLOSE CHANNEL, with Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.5.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.5.1 | [alpha identifier is displayed with small font size] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.5.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.5.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.5.2 | [alpha identifier is displayed with normal font size] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.5.1 | [Command performed successfully] |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 30 | ME USER | The ME may display channel opening information |  |
| 31 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 32 | USS ME | PDP context activation accept |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.5.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.5.1 | [alpha identifier is displayed with small font size] |
| 37 | ME USS | PDP context deactivation request |  |
| 38 | USS ME | PDP context deactivation accept |  |
| 39 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.5.1 | [Command performed successfully] |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 43 | ME USER | The ME may display channel opening information |  |
| 44 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 45 | USS ME | PDP context activation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 47 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.5.3 |  |
| 48 | ME UICC | FETCH |  |
| 49 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.5.3 | [alpha identifier is displayed with normal font size] |
| 50 | ME USS | PDP context deactivation request |  |
| 51 | USS ME | PDP context deactivation accept |  |
| 52 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.5.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.5.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Small Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 08 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.5.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.5.3

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 3"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 33 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.5.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.5.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.5.

27.22.4.28.2.6	CLOSE CHANNEL (support of Text Attribute – Bold On)

27.22.4.28.2.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.6.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.6.3	Test purpose

To verify that the ME shall display the alpha identifier according to the bold text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.6.4	Method of Test

27.22.4.28.2.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.6.4.2	Procedure

Expected sequence 2.6 (CLOSE CHANNEL, with Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.6.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.6.1 | [alpha identifier is displayed with bold on] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.6.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.6.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.6.2 | [alpha identifier is displayed with bold off] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.6.1 | [Command performed successfully] |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 30 | ME USER | The ME may display channel opening information |  |
| 31 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 32 | USS ME | PDP context activation accept |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.6.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.6.1 | [alpha identifier is displayed with bold on] |
| 37 | ME USS | PDP context deactivation request |  |
| 38 | USS ME | PDP context deactivation accept |  |
| 39 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.6.1 | [Command performed successfully] |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 43 | ME USER | The ME may display channel opening information |  |
| 44 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 45 | USS ME | PDP context activation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 47 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.6.3 |  |
| 48 | ME UICC | FETCH |  |
| 49 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.6.3 | [alpha identifier is displayed with bold off] |
| 50 | ME USS | PDP context deactivation request |  |
| 51 | USS ME | PDP context deactivation accept |  |
| 52 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.6.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.6.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 10 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.6.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.6.3

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 3"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 33 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.6.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.6.

27.22.4.28.2.7	CLOSE CHANNEL (support of Text Attribute – Italic On)

27.22.4.28.2.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.7.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.7.3	Test purpose

To verify that the ME shall display the alpha identifier according to the italic text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.7.4	Method of Test

27.22.4.28.2.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.7.4.2	Procedure

Expected sequence 2.7 (CLOSE CHANNEL, with Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.7.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.7.1 | [alpha identifier is displayed with bold on] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.7.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.6.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.7.2 | [alpha identifier is displayed with bold off] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.7.1 | [Command performed successfully] |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 30 | ME USER | The ME may display channel opening information |  |
| 31 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 32 | USS ME | PDP context activation accept |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.7.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.7.1 | [alpha identifier is displayed with bold on] |
| 37 | ME USS | PDP context deactivation request |  |
| 38 | USS ME | PDP context deactivation accept |  |
| 39 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.7.1 | [Command performed successfully] |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 43 | ME USER | The ME may display channel opening information |  |
| 44 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 45 | USS ME | PDP context activation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 47 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.7.3 |  |
| 48 | ME UICC | FETCH |  |
| 49 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.7.3 | [alpha identifier is displayed with bold off] |
| 50 | ME USS | PDP context deactivation request |  |
| 51 | USS ME | PDP context deactivation accept |  |
| 52 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.7.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.7.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 20 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.7.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.7.3

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 3"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 33 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.7.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.7.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.7.

27.22.4.28.2.8	CLOSE CHANNEL (support of Text Attribute – Underline On)

27.22.4.28.2.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.8.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.8.3	Test purpose

To verify that the ME shall display the alpha identifier according to the underline text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.8.4	Method of Test

27.22.4.28.2.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.8.4.2	Procedure

Expected sequence 2.8 (CLOSE CHANNEL, with Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.8.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.8.1 | [alpha identifier is displayed with underline on] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.8.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.8.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.8.2 | [alpha identifier is displayed with underline off] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.8.1 | [Command performed successfully] |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 30 | ME USER | The ME may display channel opening information |  |
| 31 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 32 | USS ME | PDP context activation accept |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.8.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.8.1 | [alpha identifier is displayed with underline on] |
| 37 | ME USS | PDP context deactivation request |  |
| 38 | USS ME | PDP context deactivation accept |  |
| 39 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.8.1 | [Command performed successfully] |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 43 | ME USER | The ME may display channel opening information |  |
| 44 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 45 | USS ME | PDP context activation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 47 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.8.3 |  |
| 48 | ME UICC | FETCH |  |
| 49 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.8.3 | [alpha identifier is displayed with underline off] |
| 50 | ME USS | PDP context deactivation request |  |
| 51 | USS ME | PDP context deactivation accept |  |
| 52 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.8.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.8.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 40 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.8.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.8.3

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 3"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 33 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.8.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.8.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.8.

27.22.4.28.2.9	CLOSE CHANNEL (support of Text Attribute – Strikethrough On)

27.22.4.28.2.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.9.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.9.3	Test purpose

To verify that the ME shall display the alpha identifier according to the strikethrough text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.9.4	Method of Test

27.22.4.28.2.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.9.4.2	Procedure

Expected sequence 2.9 (CLOSE CHANNEL, with Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.9.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.9.1 | [alpha identifier is displayed with strikethrough on] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.9.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.9.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.9.2 | [alpha identifier is displayed with strikethrough off] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.9.1 | [Command performed successfully] |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 30 | ME USER | The ME may display channel opening information |  |
| 31 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 32 | USS ME | PDP context activation accept |  |
| 33 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 34 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.9.1 |  |
| 35 | ME UICC | FETCH |  |
| 36 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.9.1 | [alpha identifier is displayed with strikethrough on] |
| 37 | ME USS | PDP context deactivation request |  |
| 38 | USS ME | PDP context deactivation accept |  |
| 39 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.9.1 | [Command performed successfully] |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 43 | ME USER | The ME may display channel opening information |  |
| 44 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 45 | USS ME | PDP context activation accept |  |
| 46 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 47 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.9.3 |  |
| 48 | ME UICC | FETCH |  |
| 49 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.9.3 | [alpha identifier is displayed with strikethrough off] |
| 50 | ME USS | PDP context deactivation request |  |
| 51 | USS ME | PDP context deactivation accept |  |
| 52 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.9.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.9.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 80 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.9.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.9.3

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 3"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 33 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.9.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.9.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.9.

27.22.4.28.2.10	CLOSE CHANNEL (support of Text Attribute – Foreground and Background Colour)

27.22.4.28.2.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.2.10.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.28.2.10.3	Test purpose

To verify that the ME shall display the alpha identifier according to the foreground and background colour text attribute configuration in the CLOSE CHANNEL proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.28.2.10.4	Method of Test

27.22.4.28.2.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.28.2.10.4.2	Procedure

Expected sequence 2.10 (CLOSE CHANNEL, with Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.10.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.10.1 | [alpha identifier is displayed with foreground and background colour according to the text attribute configuration] |
| 11 | ME USS | PDP context deactivation request |  |
| 12 | USS ME | PDP context deactivation accept |  |
| 13 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.10.1 | [Command performed successfully] |
| 14 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 15 | ME UICC | FETCH |  |
| 16 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 17 | ME USER | The ME may display channel opening information |  |
| 18 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 19 | USS ME | PDP context activation accept |  |
| 20 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 21 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 2.10.2 |  |
| 22 | ME UICC | FETCH |  |
| 23 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 2.10.2 | [alpha identifier is displayed with ME's default foreground and background colour] |
| 24 | ME USS | PDP context deactivation request |  |
| 25 | USS ME | PDP context deactivation accept |  |
| 26 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 2.10.1 | [Command performed successfully] |

PROACTIVE COMMAND: CLOSE CHANNEL 2.10.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 1"

Text Attribute

Formatting position:	0

Formatting length:	10

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 1B | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 31 | D0 | 04 | 00 | 0A | 00 | B4 |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 2.10.2

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Close ID 2"

Coding:

| BER-TLV: | D0 | 15 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 85 | 0A | 43 | 6C | 6F | 73 | 65 | 20 | 49 | 44 | 20 |
|  | 32 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: CLOSE CHANNEL 2.10.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.2.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.10.

##### 27.22.4.28.3	CLOSE CHANNEL(E-UTRAN/EPC)

27.22.4.28.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.28.3.2	Conformance requirements

The ME shall support the class "e" commands and E-UTRAN as defined in:

-	TS 31.111 [15].

27.22.4.28.3.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (Command Performed Successfully); or

-	TERMINAL RESPONSE (Bearer Independent Protocol Error, invalid channel identifier);

to the UICC after the ME receives the CLOSE CHANNEL proactive command. The TERMINAL RESPONSE sent back to the UICC is function of the ME and the network capabilities against asked parameters by the UICC.

To verify that upon reception of CLOSE CHANNEL proactive command with command qualifier set to 1, the same PDN connection can be re-used for next OPEN CHANNEL command by the ME.

27.22.4.28.3.4	Method of Test

27.22.4.28.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:			TestGp.rs

User login:				UserLog

User password:				UserPwd

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.6.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.6.4.1.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

27.22.4.28.3.4.2	Procedure

Expected sequence 3.1 (CLOSE CHANNEL, Default EPS bearer, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 only one APN will be activated in step 6. |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.6.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.6.1 |  |
| 5 | ME USER | The ME may display channel opening information |  |
| 6 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.6.1A or TERMINAL RESPONSE: OPEN CHANNEL 6.6.1B | [Command performed successfully] |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.1.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1 |  |
| 10 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 3.1.1 | [Command performed successfully] |
| 11 | USER ME | Wait 30 seconds, then switch off the terminal |  |

PROACTIVE COMMAND: OPEN CHANNEL 6.6.1

Same as PROACTIVE COMMAND: OPEN CHANNEL 6.5.1 in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.6.1A

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.5.1A in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.6.1B

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.5.1B in clause 27.22.4.27.6.4.

PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 3.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected sequence 3.2 (CLOSE CHANNEL, EPS bearer with APN different from default APN, successful)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required | [see initial conditions] If the ME supports A.1/173 AND NOT A.1/174 only one APN will be activated in step 7. |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.3.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.3.1 |  |
| 5 | ME USER | The terminal shall display the alpha identifier "Open Channel for UICC?" during the confirmation phase | [IF NOT A.1/84 (No display) THEN the terminal shall ignore the alpha identifier] |
| 6 | USER ME | The user confirms | [IF NOT A.1/85 (No keypad) THEN the terminal may open the channel without explicit confirmation by the user] |
| 7 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 8 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |  |
| 9 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 10 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |
| 11 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.2.1 |  |
| 12 | ME UICC | FETCH |  |
| 13 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 3.2.1 |  |
| 14 | ME E-USS/NB-SS | The ME shall send a PDN CONNECTIVITY DISCONNECT REQUEST to the network disconnect only the EPS bearer which has been established with the Open Channel command | If the ME supports A.1/173 this step is optional. |
| 15 | E-USS/NB-SS ME | DEACTIVATE EPS BEARER CONTEXT REQUEST |  |
| 16 | ME E-USS/NB-SS | DEACTIVATE EPS BEARER CONTEXT ACCEPT |  |
| 17 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 3.2.1 | [Command performed successfully] |
| 18 | USER ME | Wait 30 seconds then switch off the terminal |  |

PROACTIVE COMMAND: OPEN CHANNEL 6.3.1

Same as PROACTIVE COMMAND: OPEN CHANNEL 6.3.1 in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B in clause 27.22.4.27.6.4.

PROACTIVE COMMAND: CLOSE CHANNEL 3.2.1

Same as TERMINAL RESPONSE: CLOSE CHANNEL 3.1.1 as used in sequence 3.1

TERMINAL RESPONSE: CLOSE CHANNEL 3.2.1

Same as TERMINAL RESPONSE: CLOSE CHANNEL 3.1.1 as used in sequence 3.1

Expected sequence 3.3 (CLOSE CHANNEL, Command qualifier set to 1)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 3.3.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 3.3.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 7 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 8 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 3.3.1A or TERMINAL RESPONSE: OPEN CHANNEL 3.3.1B | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.3.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 3.3.1.1 |  |
| 12 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 3.3.1.1A | [Command performed successfully] No PDP Deactivation expected |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 3.3.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 3.3.1 |  |
| 16 | ME USER | The ME may display channel opening information |  |
| 17 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 3.3.1A or TERMINAL RESPONSE: OPEN CHANNEL 3.3.1B | [Command performed successfully].It shall re-use same PDN for Current Proactive Command |
| 18 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.2.1 |  |
| 19 | ME UICC | FETCH |  |
| 20 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 3.2.1 |  |
| 21 | ME E-USS/NB-SS | The ME shall send a PDN CONNECTIVITY DISCONNECT REQUEST to the network disconnect only the EPS bearer which has been established with the Open Channel command | If the ME supports A.1/173 this step is optional. |
| 22 | E-USS/NB-SS ME | DEACTIVATE EPS BEARER CONTEXT REQUEST |  |
| 23 | ME E-USS/NB-SS | DEACTIVATE EPS BEARER CONTEXT ACCEPT |  |
| 24 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 3.2.1 | [Command performed successfully] |
| 25 | USER ME | Wait 30 seconds then switch off the terminal |  |

PROACTIVE COMMAND: OPEN CHANNEL 3.3.1.

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.3.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.3.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 3.3.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 01 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 3.3.1.1A

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.4.28.3.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.2.

#### 27.22.4.29	RECEIVE DATA

##### 27.22.4.29.1	RECEIVE DATA (NORMAL)

27.22.4.29.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.1.2	Conformance requirements

The ME shall support the class "e" commands and additionally E-UTRAN for sequence 1.2 as defined in:

-	TS 31.111 [15].

27.22.4.29.1.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (Command Performed Successfully); or

-	TERMINAL RESPONSE (ME currently unable to process command); or

-	TERMINAL RESPONSE (Bearer Independent Protocol Error);

to the UICC after the ME receives the RECEIVE DATA proactive command. The TERMINAL RESPONSE sent back to the UICC is function of the ME and the network capabilities against asked parameters by the UICC.

27.22.4.29.1.4	Method of test

27.22.4.29.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default for sequence 1.1.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 51.010-1 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

For sequence 1.2 the default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:			TestGp.rs

User login:					UserLog

User password:				UserPwd

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.6.4.1

Data destination address	:		Sames Data Destination Address as defined in 27.22.4.27.6.4.1.

27.22.4.29.1.4.2	Procedure

Expected sequence 1.1 (RECEIVE DATA, already opened channel)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 1000 Bytes of data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 1.1.1 | (1000 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.1.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.1.1 | 200 Bytes |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.1.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.1.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.1.2 | 200 Bytes |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.1.2 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.1.3 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.1.3 | 200 Bytes |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.1.3 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.1.4 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.1.4 | 200 Bytes |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.1.4 |  |
| 35 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.1.5 |  |
| 36 | ME UICC | FETCH |  |
| 37 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.1.5 | 200 Bytes |
| 38 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.1.5 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Event list	Data available

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 09 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Data available 1.1.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	FF (more than 255 bytes are available)

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | FF |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.1.2

Logically:

Command details

Command number:	2

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 02 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.1.3

Logically:

Command details

Command number:	3

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 03 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.1.4

Logically:

Command details

Command number:	4

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 04 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.1.5

Logically:

Command details

Command number:	5

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 05 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

TERMINAL RESPONSE: RECEIVE DATA 1.1.2

Logically:

Command details

Command number:	2

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	C8 C9 CA .. FF 00 01 .. 8F (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 02 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | C8 | C9 | CA | .. | FF | 00 | 01 | 02 | .. |
|  | 8F | B7 | 01 | FF |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.1.3

Logically:

Command details

Command number:	3

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	90 91 .. FF 00 01 – 57 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 03 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 90 | 91 | 92 | .. | FF | 00 | 01 | 02 | .. |
|  | 57 | B7 | 01 | FF |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.1.4

Logically:

Command details

Command number:	4

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	58 59 .. FF 00 01 .. 1F (200 Bytes of data)

Channel data length:	C8

Coding:

| BER-TLV: | 81 | 03 | 04 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 58 | 59 | 5A | .. | FF | 00 | 01 | 02 | .. |
|  | 1F | B7 | 01 | C8 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.1.5

Logically:

Command details

Command number:	5

Command type:	RECEIVE DATA

Command qualifier:	RFUDevice identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data:	20 21 .. E7 (200 Bytes of data)

Channel data length:	00

Coding:

| BER-TLV: | 81 | 03 | 05 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 20 | 21 | 22 | .. | E7 | B7 | 01 | 00 |  |

Expected sequence 1.2 (RECEIVE DATA, already opened channel, E-UTRAN, APN different from default)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.2.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.2.1 |  |
| 8 | ME USER | The ME should not display channel opening information |  |
| 9 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The PDN CONNECTIVITY REQUEST shall contain the APN "Test12.rs"] [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 11 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 12 | ME  UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.2.1 |  |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.2.1 |  |
| 16 | ME E-USS/NB-SS | Transfer of 8 Bytes of data to the E-USS/NB-SS through channel 1 | [To retrieve ME's port number at the Access Point defined in the Open Channel command] |
| 17 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.2.1 | [Command performed successfully] |
| 18 | E-USS/NB-SS ME | Transfer of 1000 Bytes of data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 | [Sent from the Access Point different to the one of the default EPS bearer] |
| 19 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 1.2.1 | (1000 Bytes of data in the ME buffer) |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.2.1 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.2.1 | 200 Bytes |
| 23 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.2.1 |  |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.2.2 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.2.2 | 200 Bytes |
| 27 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.2.2 |  |
| 28 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.2.3 |  |
| 29 | ME UICC | FETCH |  |
| 30 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.2.3 | 200 Bytes |
| 31 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.2.3 |  |
| 32 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.2.4 |  |
| 33 | ME UICC | FETCH |  |
| 34 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.2.4 | 200 Bytes |
| 35 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.2.4 |  |
| 36 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.2.5 |  |
| 37 | ME UICC | FETCH |  |
| 38 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.2.5 | 200 Bytes |
| 39 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.2.5 |  |
| 40 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.2.1 |  |
| 41 | ME UICC | FETCH |  |
| 42 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.2.1 |  |
| 43 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.2.1 | [Command performed successfully] |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in expected sequence 1.1

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Same as TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 in expected sequence 1.1

PROACTIVE COMMAND: OPEN CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier:	empty

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		Test12.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 44 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 |
|  | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 31 | 32 | 02 |
|  | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 |
|  | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Data available 1.2.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	FF (more than 255 bytes are available)

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | FF |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.2.2

Logically:

Command details

Command number:	2

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 02 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.2.3

Logically:

Command details

Command number:	3

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 03 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.2.4

Logically:

Command details

Command number:	4

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 04 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.2.5

Logically:

Command details

Command number:	5

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 05 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

TERMINAL RESPONSE: RECEIVE DATA 1.2.2

Logically:

Command details

Command number:	2

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	C8 C9 CA .. FF 00 01 .. 8F (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 02 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | C8 | C9 | CA | .. | FF | 00 | 01 | 02 | .. |
|  | 8F | B7 | 01 | FF |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.2.3

Logically:

Command details

Command number:	3

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	90 91 .. FF 00 01 – 57 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 03 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 90 | 91 | 92 | .. | FF | 00 | 01 | 02 | .. |
|  | 57 | B7 | 01 | FF |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.2.4

Logically:

Command details

Command number:	4

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	58 59 .. FF 00 01 .. 1F (200 Bytes of data)

Channel data length:	C8

Coding:

| BER-TLV: | 81 | 03 | 04 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 58 | 59 | 5A | .. | FF | 00 | 01 | 02 | .. |
|  | 1F | B7 | 01 | C8 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.2.5

Logically:

Command details

Command number:	5

Command type:	RECEIVE DATA

Command qualifier:	RFUDevice identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data:	20 21 .. E7 (200 Bytes of data)

Channel data length:	00

Coding:

| BER-TLV: | 81 | 03 | 05 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 20 | 21 | 22 | .. | E7 | B7 | 01 | 00 |  |

PROACTIVE COMMAND: CLOSE CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 27.22.4.29.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1 to 1.2.

##### 27.22.4.29.2	RECEIVE DATA (support of Text Attribute)

27.22.4.29.2.1	RECEIVE DATA (support of Text Attribute – Left Alignment)

27.22.4.29.2.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.1.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.1.3	Test purpose

To verify that the ME shall display the alpha identifier according to the left alignment text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.1.4	Method of test

27.22.4.29.2.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Sames Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.1.4.2	Procedure

Expected sequence 2.1 (RECEIVE DATA, with Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 400 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1ENVELOPE (Data Available) | (400 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.1.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.1.1 | 200 Bytes with alpha identifier is displayed with left alignment |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.1.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.1.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.1.2 | 200 Bytes with alpha identifier shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/21, no alignment change will take place |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.1.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	FF (more than 255 bytes are available)

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | FF |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 2.1.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.1.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.1.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.1.

27.22.4.29.2.2	RECEIVE DATA (support of Text Attribute – Center Alignment)

27.22.4.29.2.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.2.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.2.3	Test purpose

To verify that the ME shall display the alpha identifier according to the center alignment text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.2.4	Method of test

27.22.4.29.2.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.2.4.2	Procedure

Expected sequence 2.2 (RECEIVE DATA, with Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 400 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (400 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.2.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.2.1 | 200 Bytes with alpha identifier is displayed with center alignment |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.2.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.2.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.2.2 | 200 Bytes with alpha identifier shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/21, no alignment change will take place |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.2.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.2.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Center Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 01 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.2.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.2.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.2.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.2.

27.22.4.29.2.3	RECEIVE DATA (support of Text Attribute – Right Alignment)

27.22.4.29.2.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.3.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.3.3	Test purpose

To verify that the ME shall display the alpha identifier according to the right alignment text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.3.4	Method of test

27.22.4.29.2.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.3.4.2	Procedure

Expected sequence 2.3 (RECEIVE DATA, with Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 400 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (400 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.3.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.3.1 | 200 Bytes with alpha identifier is displayed with right alignment |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.3.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.3.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.3.2 | 200 Bytes with alpha identifier shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/21, no alignment change will take place |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.3.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.3.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Right Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 02 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.3.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.3.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.3.

27.22.4.29.2.4	RECEIVE DATA (support of Text Attribute – Large Font Size)

27.22.4.29.2.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.4.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.4.3	Test purpose

To verify that the ME shall display the alpha identifier according to the large font size text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.4.4	Method of test

27.22.4.29.2.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.4.4.2	Procedure

Expected sequence 2.4 (RECEIVE DATA, with Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 800 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (800 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.4.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.4.1 | 200 Bytes with alpha identifier is displayed with large font size |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.4.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.4.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.4.2 | 200 Bytes with alpha identifier is displayed with normal font size |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.4.1 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.4.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.4.1 | 200 Bytes with alpha identifier is displayed with large font size |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.4.1 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.4.3 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.4.3 | 200 Bytes with alpha identifier is displayed with normal font size |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.4.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.4.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Large Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 04 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.4.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.4.3

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 3"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 33 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.4.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.4.

27.22.4.29.2.5	RECEIVE DATA (support of Text Attribute – Small Font Size)

27.22.4.29.2.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.5.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.5.3	Test purpose

To verify that the ME shall display the alpha identifier according to small font size the text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.5.4	Method of test

27.22.4.29.2.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.5.4.2	Procedure

Expected sequence 2.5 (RECEIVE DATA, with Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 800 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (800 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.5.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.5.1 | 200 Bytes with alpha identifier is displayed with small font size |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.5.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.5.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.5.2 | 200 Bytes with alpha identifier is displayed with normal font size |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.5.1 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.5.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.5.1 | 200 Bytes with alpha identifier is displayed with small font size |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.5.1 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.5.3 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.5.3 | 200 Bytes with alpha identifier is displayed with normal font size |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.5.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.5.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Small Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 08 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.5.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.5.3

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 3"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 33 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.5.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.5.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.5.

27.22.4.29.2.6	RECEIVE DATA (support of Text Attribute – Bold On)

27.22.4.29.2.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.6.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.6.3	Test purpose

To verify that the ME shall display the alpha identifier according to the bold text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.6.4	Method of test

27.22.4.29.2.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.6.4.2	Procedure

Expected sequence 2.6 (RECEIVE DATA, with Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 800 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (800 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.6.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.6.1 | 200 Bytes with alpha identifier is displayed with bold on |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.6.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.6.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.6.2 | 200 Bytes with alpha identifier is displayed with bold off |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.6.1 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.6.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.6.1 | 200 Bytes with alpha identifier is displayed with bold on |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.6.1 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.6.3 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.6.3 | 200 Bytes with alpha identifier is displayed with bold off |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.6.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.6.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 10 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.6.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.6.3

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 3"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 33 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.6.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.6.

27.22.4.29.2.7	RECEIVE DATA (support of Text Attribute – Italic On)

27.22.4.29.2.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.7.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.7.3	Test purpose

To verify that the ME shall display the alpha identifier according to the italic text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.7.4	Method of test

27.22.4.29.2.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.7.4.2	Procedure

Expected sequence 2.7 (RECEIVE DATA, with Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 800 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1ENVELOPE | (800 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.7.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.7.1 | 200 Bytes with alpha identifier is displayed with italic on |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.7.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.7.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.7.2 | 200 Bytes with alpha identifier is displayed with italic off |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.7.1 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.7.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.7.1 | 200 Bytes with alpha identifier is displayed with italic on |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.7.1 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.7.3 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.7.3 | 200 Bytes with alpha identifier is displayed with italic off |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.7.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.7.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 20 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.7.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.7.3

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 3"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 33 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.7.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.7.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.7.

27.22.4.29.2.8	RECEIVE DATA (support of Text Attribute – Underline On)

27.22.4.29.2.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.8.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.8.3	Test purpose

To verify that the ME shall display the alpha identifier according to the underline text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.8.4	Method of test

27.22.4.29.2.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.8.4.2	Procedure

Expected sequence 2.8 (RECEIVE DATA, with Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 800 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (800 kBytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.8.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.8.1 | 200 Bytes with alpha identifier is displayed with underline on |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.8.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.8.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.8.2 | 200 Bytes with alpha identifier is displayed with underline off |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.8.1 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.8.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.8.1 | 200 Bytes with alpha identifier is displayed with underline on |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.8.1 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.8.3 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.8.3 | 200 Bytes with alpha identifier is displayed with underline off |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.8.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.8.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 40 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.8.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.8.3

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 3"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 33 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.8.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.8.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.8.

27.22.4.29.2.9	RECEIVE DATA (support of Text Attribute – Strikethrough On)

27.22.4.29.2.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.9.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.9.3	Test purpose

To verify that the ME shall display the alpha identifier according to the strikethrough text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.9.4	Method of test

27.22.4.29.2.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.9.4.2	Procedure

Expected sequence 2.9 (RECEIVE DATA, with Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 800 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (800 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.9.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.9.1 | 200 Bytes with alpha identifier is displayed with strikethrough on |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.9.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.9.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.9.2 | 200 Bytes with alpha identifier is displayed with strikethrough off |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.9.1 |  |
| 27 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.9.1 |  |
| 28 | ME UICC | FETCH |  |
| 29 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.9.1 | 200 Bytes with alpha identifier is displayed with strikethrough on |
| 30 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.9.1 |  |
| 31 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.9.3 |  |
| 32 | ME UICC | FETCH |  |
| 33 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.9.3 | 200 Bytes with alpha identifier is displayed with strikethrough off |
| 34 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.9.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.9.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 80 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.9.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.9.3

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 3"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 33 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.9.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.9.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.9.

27.22.4.29.2.10	RECEIVE DATA (support of Text Attribute – Foreground and Background Colour)

27.22.4.29.2.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.29.2.10.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.29.2.10.3	Test purpose

To verify that the ME shall display the alpha identifier according to the foreground and background colour text attribute configuration in the RECEIVE DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.29.2.10.4	Method of test

27.22.4.29.2.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.29.2.10.4.2	Procedure

Expected sequence 2.10 (RECEIVE DATA, with Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 15 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 17 | USS ME | Transfer of 400 Bytes data to the ME through channel 1 using the ME's port number, which was retrieved in step 15 |  |
| 18 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1 | (400 Bytes of data in the ME buffer) |
| 19 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.10.1 |  |
| 20 | ME UICC | FETCH |  |
| 21 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.10.1 | 200 Bytes with alpha identifier is displayed with foreground and background colour |
| 22 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.10.1 |  |
| 23 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 2.10.2 |  |
| 24 | ME UICC | FETCH |  |
| 25 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 2.10.2 | 200 Bytes with alpha identifier is displayed with ME's default foreground and background colour |
| 26 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 2.10.1 |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Same as PROACTIVE COMMAND: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

TERMINAL RESPONSE: SEND DATA 1.1.1

Same as TERMINAL RESPONSE: SEND DATA 1.1.1 in clause 27.22.4.29.1.4.2.

ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1

Same as cl. 27.22.4.29.2.1.4.2, ENVELOPE: EVENT DOWNLOAD - Data available 2.1.1.

PROACTIVE COMMAND: RECEIVE DATA 2.10.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 1"

Channel Data Length

Channel Data Length:	200

Text Attribute

Formatting position:	0

Formatting length:	14

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 22 | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 31 | B7 | 01 | C8 | D0 | 04 | 00 | 0E | 00 | B4 |

PROACTIVE COMMAND: RECEIVE DATA 2.10.2

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Receive Data 2"

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 1C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 52 | 65 | 63 | 65 | 69 | 76 | 65 | 20 | 44 | 61 | 74 |
|  | 61 | 20 | 32 | B7 | 01 | C8 |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 2.10.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	FF

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | FF |  |

27.22.4.29.2.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.10.

#### 27.22.4.30	SEND DATA

##### 27.22.4.30.1	SEND DATA (normal)

27.22.4.30.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.1.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.1.3	Test purpose

To verify that the ME shall send a:

-	TERMINAL RESPONSE (Command Performed Successfully); or

-	TERMINAL RESPONSE (ME currently unable to process command); or

-	TERMINAL RESPONSE (Bearer Independent Protocol Error);

-	TERMINAL RESPONSE (Proactive USIM session terminated by the user);

to the UICC after the ME receives the SEND DATA proactive command. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

27.22.4.30.1.4	Method of test

27.22.4.30.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27

The following Bearer Parameters used are those defined in the default Test PDP context for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.1.4.2	Procedure

Expected sequence 1.1 (SEND DATA, immediate mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 11 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 |  |
| 12 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

Expected sequence 1.2 (SEND DATA, Store mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.2.1 | Send 500 Bytes of data (200 + 200 + 100) |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.2.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.2.2 | [200 Bytes] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.2.2 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.3 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA (Immediate mode) 1.2.3 | [100 Bytes] |
| 19 | ME USS | Transfer of 500 Bytes of data to the USS through channel 1 |  |
| 20 | ME UICC | TERMINAL RESPONSE: SEND DATA (Immediate mode) 1.2.3 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	00 01 .. C7 (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | .. | C7 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	C8 C9 .. FF 00 01 .. 8F (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | C8 | C9 | .. | FF | 00 | 01 | .. | 8F |  |

TERMINAL RESPONSE: SEND DATA 1.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.2.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Immediate mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	90 91 .. F3 (100 Bytes of data)

Coding:

| BER-TLV: | D0 | 6F | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 64 | 90 | 91 | .. | F3 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.2.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Immediate mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

Expected sequence 1.3 (SEND DATA, Store mode, Tx buffer fully used)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.1 | Send 1000 Bytes of data by packet of 200 Bytes |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.2 | [200 Bytes] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.2 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.3 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.3 | [200 Bytes] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.3 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.4 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.4 | [200 Bytes] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.4 | [Command performed successfully] |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.5 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.3.5 | [200 Bytes] |
| 27 | ME USS | Transfer of 1000 Bytes of data to the USS through channel 1 |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.3.5 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 1.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | ... | C7 |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.3.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	C8 C9 CA .. FF 00 01 .. 8F (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | C8 | C9 | CA | ... | FF | 00 | 02 | .. | 8F |

TERMINAL RESPONSE: SEND DATA 1.3.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.3.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	90 91 .. FF 00 01 .. 57 (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 90 | 91 | .. | FF | 00 | 01 | .. | 57 |  |

TERMINAL RESPONSE: SEND DATA 1.3.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.3.4

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	58 59 .. FF 00 01 .. 1F (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 58 | 59 | .. | FF | 00 | 01 | .. | 1F |  |

TERMINAL RESPONSE: SEND DATA 1.3.4

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	200 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | C8 |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.3.5

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	20 21 .. E7 (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 20 | 21 | .. | E7 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.3.5

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

Expected sequence 1.4 (SEND DATA, 2 consecutive SEND DATA Store mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.1 | Send 1000 Bytes of data by packet of 200 Bytes |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.2 | [200 Bytes] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.2 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.3 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.3 | [200 Bytes] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.3 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.4 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.4 | [200 Bytes] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.4 | [Command performed successfully] |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.5 | … |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.3.5 | [200 Bytes] |
| 27 | ME USS | Transfer of 1000 Bytes of data to the USS through channel 1 |  |
| 28 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.3.5 | [Command performed successfully] |
| 29 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.1 |  |
| 30 | ME UICC | FETCH |  |
| 31 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.1 | Send 1000 Bytes of data by packet of 200 Bytes |
| 32 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.1 | [Command performed successfully] |
| 33 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.2 |  |
| 34 | ME UICC | FETCH |  |
| 35 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.2 | [200 Bytes] |
| 36 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.2 | [Command performed successfully] |
| 37 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.3 |  |
| 38 | ME UICC | FETCH |  |
| 39 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.3 | [200 Bytes] |
| 40 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.3 | [Command performed successfully] |
| 41 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.4 |  |
| 42 | ME UICC | FETCH |  |
| 43 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 1.3.4 | [200 Bytes] |
| 44 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 1.3.4 | [Command performed successfully] |
| 45 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.3.5 | … |
| 46 | ME UICC | FETCH |  |
| 47 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.3.5 | [200 Bytes] |
| 48 | ME USS | Transfer of 1000 Bytes of data to the USS through channel 1 |  |
| 49 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.3.5 | [Command performed successfully] |

Expected sequence 1.5 (SEND DATA, immediate mode with a bad channel identifier)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.5.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.5.1 |  |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.5.1 | [Invalid channel number] |

PROACTIVE COMMAND: SEND DATA 1.5.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 2

Channel Data

Channel Data :	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 22 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.5.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Bearer Independent Protocol error (3A)

Additional Result:	Channel identifier not valid (03)

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 02 | 3A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 |  |  |  |  |  |  |  |  |  |  |  |

Expected sequence 1.6 Void

27.22.4.30.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.5.

##### 27.22.4.30.2	SEND DATA (support of Text Attribute)

27.22.4.30.2.1	SEND DATA (support of Text Attribute – Left Alignment)

27.22.4.30.2.1.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.1.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.1.3	Test purpose

To verify that the ME shall display the alpha identifier according to the left alignment text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.1.4	Method of test

27.22.4.30.2.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.1.4.2	Procedure

Expected sequence 2.1 (SEND DATA with Text Attribute – Left Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.1.1 | [alpha identifier shall be displayed with left alignment] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.1.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.1.2 | [Message shall be formatted without left alignment. Remark: If left alignment is the ME's default alignment as declared in table A.2/22, no alignment change will take place] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold On, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.1.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.1.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.1.

27.22.4.30.2.2	SEND DATA (support of Text Attribute – Center Alignment)

27.22.4.30.2.2.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.2.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.2.3	Test purpose

To verify that the ME shall display the alpha identifier according to the center alignment text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.2.4	Method of test

27.22.4.30.2.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.2.4.2	Procedure

Expected sequence 2.2 (SEND DATA with Text Attribute – Center Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.2.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.2.1 | [alpha identifier shall be displayed with center alignment] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.2.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.2.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.2.2 | [Message shall be formatted without center alignment. Remark: If center alignment is the ME's default alignment as declared in table A.2/22, no alignment change will take place] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.2.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Center Alignment, Normal Font, Bold On, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 01 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.2.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.2.

27.22.4.30.2.3	SEND DATA (support of Text Attribute – Right Alignment)

27.22.4.30.2.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.3.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.3.3	Test purpose

To verify that the ME shall display the alpha identifier according to the right alignment text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.3.4	Method of test

27.22.4.30.2.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.3.4.2	Procedure

Expected sequence 2.3 (SEND DATA with Text Attribute – Right Alignment)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.3.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.3.1 | [alpha identifier shall be displayed with right alignment] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.3.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.3.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.3.2 | [Message shall be formatted without right alignment. Remark: If right alignment is the ME's default alignment as declared in table A.2/22, no alignment change will take place] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.3.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Right Alignment, Normal Font, Bold On, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 02 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.3.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.3.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.3.

27.22.4.30.2.4	SEND DATA (support of Text Attribute – Large Font Size)

27.22.4.30.2.4.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.4.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.4.3	Test purpose

To verify that the ME shall display the alpha identifier according to the large font size text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.4.4	Method of test

27.22.4.30.2.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.4.4.2	Procedure

Expected sequence 2.4 (SEND DATA with Text Attribute – Large Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.4.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.4.1 | [alpha identifier shall be displayed with large font size] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.4.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.4.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.4.2 | [alpha identifier shall be displayed with normal font size] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.4.1 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.4.1 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.4.1 | [alpha identifier shall be displayed with large font size] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.4.1 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.4.3 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.4.3 | [alpha identifier shall be displayed with normal font size] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.4.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.4.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Large Font, Bold On, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 04 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.4.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.4.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 3"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 33 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.4.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.4.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.4.

27.22.4.30.2.5	SEND DATA (support of Text Attribute – Small Font Size)

27.22.4.30.2.5.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.5.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.5.3	Test purpose

To verify that the ME shall display the alpha identifier according to the small font size text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.5.4	Method of test

27.22.4.30.2.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.5.4.2	Procedure

Expected sequence 2.5 (SEND DATA with Text Attribute – Small Font Size)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.5.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.5.1 | [alpha identifier shall be displayed with small font size] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.5.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.5.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.5.2 | [alpha identifier shall be displayed with normal font size] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.5.1 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.5.1 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.5.1 | [alpha identifier shall be displayed with small font size] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.5.1 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.5.3 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.5.3 | [alpha identifier shall be displayed with normal font size] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.5.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.5.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Small Font, Bold On, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 08 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.5.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.5.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 3"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 33 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.5.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.5.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.5.

27.22.4.30.2.6	SEND DATA (support of Text Attribute – Bold On)

27.22.4.30.2.6.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.6.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.6.3	Test purpose

To verify that the ME shall display the alpha identifier according to the bold text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.6.4	Method of test

27.22.4.30.2.6.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.6.4.2	Procedure

Expected sequence 2.6 (SEND DATA with Text Attribute – Bold On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.6.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.6.1 | [alpha identifier shall be displayed with Bold on] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.6.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.6.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.6.2 | [alpha identifier shall be displayed with bold off] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.6.1 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.6.1 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.6.1 | [alpha identifier shall be displayed with bold on] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.6.1 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.6.3 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.6.3 | [alpha identifier shall be displayed with bold off] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.6.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.6.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold On , Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 10 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.6.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.6.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 3"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 33 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.6.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.6.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.6.

27.22.4.30.2.7	SEND DATA (support of Text Attribute – Italic On)

27.22.4.30.2.7.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.7.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.7.3	Test purpose

To verify that the ME shall display the alpha identifier according to the italic text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.7.4	Method of test

27.22.4.30.2.7.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.7.4.2	Procedure

Expected sequence 2.7 (SEND DATA with Text Attribute – Italic On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.7.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.7.1 | [alpha identifier shall be displayed with Italic on] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.7.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.7.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.7.2 | [alpha identifier shall be displayed with italic off] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.7.1 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.7.1 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.7.1 | [alpha identifier shall be displayed with italic on] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.7.1 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.7.3 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.7.3 | [alpha identifier shall be displayed with italic off] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.7.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.7.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic On, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 20 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.7.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.7.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 3"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 33 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.7.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.7.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.7.

27.22.4.30.2.8	SEND DATA (support of Text Attribute – Underline On)

27.22.4.30.2.8.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.8.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.8.3	Test purpose

To verify that the ME shall display the alpha identifier according to the underline text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.8.4	Method of test

27.22.4.30.2.8.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.8.4.2	Procedure

Expected sequence 2.8 (SEND DATA with Text Attribute – Underline On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.8.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.8.1 | [alpha identifier shall be displayed with underline on] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.8.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.8.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.8.2 | [alpha identifier shall be displayed with underline off] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.8.1 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.8.1 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.8.1 | [alpha identifier shall be displayed with underline on] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.8.1 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.8.3 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.8.3 | [alpha identifier shall be displayed with underline off] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.8.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.8.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline On, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 40 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.8.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.8.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 3"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 33 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.8.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.8.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.8.

27.22.4.30.2.9	SEND DATA (support of Text Attribute – Strikethrough On)

27.22.4.30.2.9.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.9.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.9.3	Test purpose

To verify that the ME shall display the alpha identifier according to the strikethrough text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.9.4	Method of test

27.22.4.30.2.9.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.9.4.2	Procedure

Expected sequence 2.9 (SEND DATA with Text Attribute – Strikethrough On)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.9.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.9.1 | [alpha identifier shall be displayed with strikethrough on] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.9.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.9.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.9.2 | [alpha identifier shall be displayed with strikethrough off] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.9.1 | [Command performed successfully] |
| 16 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.9.1 |  |
| 17 | ME UICC | FETCH |  |
| 18 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.9.1 | [alpha identifier shall be displayed with strikethrough on] |
| 19 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.9.1 | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.9.3 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.9.3 | [alpha identifier shall be displayed with strikethrough off] |
| 23 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.9.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.9.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough On

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 80 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.9.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.9.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 3"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 33 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.9.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.9.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.9.

27.22.4.30.2.10	SEND DATA (support of Text Attribute – Foreground and Background Colour)

27.22.4.30.2.10.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.2.10.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

27.22.4.30.2.10.3	Test purpose

To verify that the ME shall display the alpha identifier according to the foreground and background colour text attribute configuration in the SEND DATA proactive command and send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC.

27.22.4.30.2.10.4	Method of test

27.22.4.30.2.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, as specified in TS 34.108 [12], for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

27.22.4.30.2.10.4.2	Procedure

Expected sequence 2.10 (SEND DATA with Text Attribute – Foreground and Background Colour)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.10.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.10.1 | [alpha identifier shall be displayed with foreground and background colour according to the text attribute configuration] |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.10.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 2.10.2 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: SEND DATA 2.10.2 | [alpha identifier shall be displayed with ME's default foreground and background colour] |
| 15 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 2.10.1 | [Command performed successfully] |

PROACTIVE COMMAND: SEND DATA 2.10.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 1"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Text Attribute

Formatting position:	0

Formatting length:	11

Formatting mode:	Left Alignment, Normal Font, Bold Off, Italic Off, Underline Off, Strikethrough Off

Colour:	Dark Green Foreground, Bright Yellow Background

Coding:

| BER-TLV: | D0 | 26 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 31 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 | D0 | 04 |
|  | 00 | 0B | 00 | B4 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 2.10.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Alpha Identifier	"Send Data 2"

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 20 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0B | 53 | 65 | 6E | 64 | 20 | 44 | 61 | 74 | 61 | 20 | 32 |
|  | B6 | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |

TERMINAL RESPONSE: SEND DATA 2.10.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

27.22.4.30.2.10.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 2.10.

##### 27.22.4.30.3	SEND DATA (E-UTRAN)

27.22.4.30.3.1	Definition and applicability

See clause 3.2.2.

27.22.4.30.3.2	Conformance requirements

The ME shall support the class "e" commands and E-UTRAN as defined in:

-	TS 31.111 [15].

27.22.4.30.3.3	Test purpose

To verify that the ME shall send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC after the ME receives the SEND DATA proactive command. The TERMINAL RESPONSE sent back to the UICC is the result of the ME and the network capabilities against requested parameters by the UICC.

To verify that the ME uses the default EPS bearer as requested in the Open Channel Command.

27.22.4.30.3.4	Method of test

27.22.4.30.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:			TestGp.rs

User login:				UserLog

User password:				UserPwd

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.6.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.6.4.1.

The Channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

Prior to test case execution the apparatus supplier shall have provided the "Preferred buffer size supported by the terminal for Open Channel command" as requested in table A.2/29.

27.22.4.30.3.4.2	Procedure

Expected sequence 3.1 (SEND DATA, E-UTRAN, Defaults EPS bearer, immediate mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 3.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 3.1.1 |  |
| 4 | ME USER | The ME may display channel opening information | [The user shall confirm the channel opening if required] |
| 5 | ME E-USS/NB-SS | No PDN connectivity request PDN CONNECTIVITY REQUEST is sent if the ME supports A.1/173 AND NOT A.1/174. | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 3.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 3.1.1B | [Command performed successfully] If the ME supports A.1/173 only OPEN CHANNEL 3.1.1A shall be sent. |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 3.1.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 3.1.1 |  |
| 10 | ME E-USS/NB-SS | Transfer of 8 Bytes of data to the USS through channel 1 |  |
| 11 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 3.1.1 | [Command performed successfully] |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.1.1 |  |
| 13 | ME UICC | FETCH |  |
| 14 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1 |  |
| 15 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 3.1.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 3.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP, UICC in client mode, remote connection

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 30 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 03 | 39 | 02 | 05 | 78 | 0D | 08 | F4 | 55 | 73 | 65 |
|  | 72 | 4C | 6F | 67 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 |
|  | 77 | 64 | 3C | 03 | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:	Default bearer for requested transport layer

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 01 | 03 | 39 | 02 | 05 | 78 |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer

Bearer type:				E-UTRAN / mapped UTRAN packet service

QCI			9

Maximum bit rate for uplink:			64 kbps

Maximum bit rate for downlink:		64 kbps

Guaranteed bit rate for uplink:			64 kbps

Guaranteed bit rate for downlink:	64 kbps

Maximum bit rate for uplink (extended):	0

Maximum bit rate for downlink (extended):	0

Guaranteed bit rate for uplink (extended):	0

Guaranteed bit rate for downlink (extended):	0

PDN Type:			IP

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 0B | 0B | 09 | 40 | 40 | 40 | 40 |
|  | 00 | 00 | 00 | 00 | 02 | 39 | 02 | 05 | 78 |  |  |  |

PROACTIVE COMMAND: SEND DATA 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 3.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 3.1.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected sequence 3.2 (SEND DATA, E-UTRAN, APN different from default APN, Store mode)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 3.2.1 | If the ME supports A.1/173 AND NOT A.1/174 only one APN will be activated in step 5. |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 3.2.1 |  |
| 4 | ME USER | The ME should not display channel opening information |  |
| 5 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The PDN CONNECTIVITY REQUEST shall contain the APN "Test12.rs"] [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 7 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 8 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 3.2.1 | [Command performed successfully] |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 3.2.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 3.2.1 | Send 500 Bytes of data (200 + 200 + 100) |
| 12 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 3.2.1 | [Command performed successfully] |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 3.2.2 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: SEND DATA (store mode) 3.2.2 | [200 Bytes] |
| 16 | ME UICC | TERMINAL RESPONSE: SEND DATA (store mode) 3.2.2 | [Command performed successfully] |
| 17 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 3.2.3 |  |
| 18 | ME UICC | FETCH |  |
| 19 | UICC ME | PROACTIVE COMMAND: SEND DATA (Immediate mode) 3.2.3 | [100 Bytes] |
| 20 | ME E-USS/NB-SS | Transfer of 500 Bytes of data to the USS through channel 1 |  |
| 21 | ME UICC | TERMINAL RESPONSE: SEND DATA (Immediate mode) 3.2.3 | [Command performed successfully] |
| 22 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 3.2.1 |  |
| 23 | ME UICC | FETCH |  |
| 24 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 3.2.1 |  |
| 25 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 3.2.1 | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 3.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier:	empty

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		Test12.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 44 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 |
|  | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 31 | 32 | 02 |
|  | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 |
|  | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 3.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 3.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	00 01 .. C7 (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | .. | C7 |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 3.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 3.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	C8 C9 .. FF 00 01 .. 8F (200 Bytes of data)

Coding:

| BER-TLV: | D0 | 81 | D4 | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | C8 | C9 | .. | FF | 00 | 01 | .. | 8F |  |

TERMINAL RESPONSE: SEND DATA 3.2.2

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Store mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 3.2.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Immediate mode

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data :	90 91 .. F3 (100 Bytes of data)

Coding:

| BER-TLV: | D0 | 6F | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 64 | 90 | 91 | .. | F3 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SEND DATA 3.2.3

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Immediate mode

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: CLOSE CHANNEL 3.2.1

Same as PROACTIVE COMMAND: CLOSE CHANNEL 3.1.1 from sequence 1.1.

TERMINAL RESPONSE: CLOSE CHANNEL 3.2.1

Same as Terminal Response: CLOSE CHANNEL 3.1.1 from sequence 1.1.

27.22.4.30.3.5	Test Requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.2.

#### 27.22.4.31	GET CHANNEL STATUS

##### 27.22.4.31.1	Definition and applicability

See clause 3.2.2.

##### 27.22.4.31.2	Conformance requirements

The ME shall support the class "e" commands and additionally E-UTRAN for sequences 1.4 to 1.5 as defined in:

-	TS 31.111 [15].

##### 27.22.4.31.3	Test purpose

To verify that the ME shall send a TERMINAL RESPONSE (Command Performed Successfully) to the UICC after the ME receives the GET STATUS proactive command. The TERMINAL RESPONSE sent back to the UICC is function of the ME and the network capabilities against asked parameters by the UICC.

##### 27.22.4.31.4	Method of test

27.22.4.31.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

For sequences 1.1 to 1.3:

The following Bearer Parameters used are those defined in the default Test PDP context3, for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

For sequences 1.4 to 1.5

The ME is connected to the USIM Simulator and the E-USS/NB-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:			TestGp.rs

User login:					UserLog

User password:				UserPwd

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.6.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.6.4.1.

27.22.4.31.4.2	Procedure

Expected sequence 1.1 (GET STATUS, without any BIP channel opened)

For that test, no channel has been opened.

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: GET CHANNEL STATUS 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: GET STATUS 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE GET STATUS 1.1.1 A Or TERMINAL RESPONSE: GET STATUS 1.1.1B Or TERMINAL RESPONSE: GET STATUS 1.1.1C | [Command performed successfully] |

PROACTIVE COMMAND: GET STATUS 1.1.1

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: GET STATUS 1.1.1A

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: GET STATUS 1.1.1B

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel status:	No Channel available, link not established or PDP context not activated

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 00 | 00 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: GET STATUS 1.1.1C

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel 1 status:	Channel identifier 1, Link not established or PDP context not activated

Channel 2 status:	Channel identifier 2, Link not established or PDP context not activated

.

.

Channel n status:	Channel identifier n, Link not established or PDP context not activated

The number of channel status data objects shall be same as the number of channels(n) supported by the ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note1 |  |  |  |  |  |  |  |  |  |  |  |

Note1:	The Terminal Response shall contain as many channel status TLVs as channels are supported by the ME. Each channel status TLV coding shall  indicate the corresponding channel identifier and shall state "Link not established or PDP context not activated". As an example, if the mobile supports two channels then the corresponding channel status data objects coding would be : 'B8 02 01 00 B8 02 02 00'.

Expected sequence 1.2 (GET STATUS, with a BIP channel currently opened)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 4 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 5 | USS ME | PDP context activation accept |  |
| 6 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 7 | UICC ME | PROACTIVE COMMAND PENDING: GET CHANNEL STATUS 1.2.1 |  |
| 8 | ME UICC | FETCH |  |
| 9 | UICC ME | PROACTIVE COMMAND: GET STATUS 1.2.1 |  |
| 10 | ME UICC | TERMINAL RESPONSE GET STATUS 1.2.1 A Or TERMINAL RESPONSE: GET STATUS 1.2.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: GET STATUS 1.2.1

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: GET STATUS 1.2.1A

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel status:	Channel 1 open, link established or PDP context activated

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 81 | 00 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: GET STATUS 1.2.1B

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel 1 status:	Channel identifier 1 open, Link established or PDP context activated

Channel 2 status:	Channel identifier 2, Link not established or PDP context not activated

.

.

Channel n status:	Channel identifier n, Link not established or PDP context not activated

The number of channel status data objects shall be same as the number of channels(n) supported by the ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note1 |  |  |  |  |  |  |  |  |  |  |  |

Note1:	The Terminal Response shall contain as many channel status TLVs as channels are supported by the ME. The channel status TLV coding of the opened channel shall state "Link established or PDP context activated". Each other channel status TLV coding shall indicate the corresponding channel identifier and shall state "Link is not established or PDP context not activated". As an example, if the mobile supports two channels and channel 1 is opened then the corresponding channel status data objects coding would be : 'B8 02 81 00 B8 02 02 00'.

Expected sequence 1.3 (GET STATUS, after a link dropped)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | [Command performed successfully] |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 9 | USS ME | PDP context activation accept |  |
| 10 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 11 | USS ME | DROP LINK |  |
| 12 | ME UICC | ENVELOPE EVENT DOWNLOAD: CHANNEL STATUS 1.3.1 | [Link dropped] |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: GET STATUS 1.3.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: GET STATUS 1.3.1 |  |
| 16 | ME UICC | TERMINAL RESPONSE: GET STATUS 1.3.1A Or TERMINAL RESPONSE: GET STATUS 1.3.1B Or TERMINAL RESPONSE: GET STATUS 1.3.1C Or TERMINAL RESPONSE: GET STATUS 1.3.1D Or TERMINAL RESPONSE: GET STATUS 1.3.1E | [Command performed successfully] |

TERMINAL RESPONSE: GET STATUS 1.3.1A

Same as TERMINAL RESPONSE: GET STATUS 1.1.1A

TERMINAL RESPONSE: GET STATUS 1.3.1B

Same as TERMINAL RESPONSE: GET STATUS 1.1.1B

TERMINAL RESPONSE: GET STATUS 1.3.1C

Same as TERMINAL RESPONSE: GET STATUS 1.1.1C

TERMINAL RESPONSE: GET STATUS 1.3.1D

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel status:	Channel 1, link dropped

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 01 | 05 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: GET STATUS 1.3.1E

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel 1 status:	Channel identifier 1, link dropped

Channel 2 status:	Channel identifier 2, Link not established or PDP context not activated

.

.

Channel n status:	Channel identifier n, Link not established or PDP context not activated

The number of channel status data objects shall be same as the number of channels(n) supported by the ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 01 | 05 | Note1 |  |  |  |  |  |  |  |

Note1:	The Terminal Response shall contain as many channel status TLVs as channels are supported by the ME. Each channel status TLV coding except that one for which the link was dropped by the SS shall  indicate the corresponding channel identifier and shall state "Link not established or PDP context not activated". As an example, if the mobile supports two channels then the corresponding channel status data objects coding would be : 'B8 02 01 05 B8 02 02 00'.

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Channel Status

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 99 | 01 | 0A |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE EVENT DOWNLOAD: CHANNEL STATUS 1.3.1

Logically:

Event list

Event list:	Channel Status

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1, link dropped

Coding:

| BER-TLV: | D6 | 0B | 99 | 01 | 0A | 82 | 02 | 82 | 81 | B8 | 02 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: GET STATUS 1.3.1

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected sequence 1.4 (GET STATUS, EPS bearer with APN different from default APN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.3.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 6.3.1 |  |
| 4 | ME USER | The terminal shall display the alpha identifier "Open Channel for UICC?" during the confirmation phase | [IF NOT A.1/84 (No display) THEN the terminal shall ignore the alpha identifier] |
| 5 | USER ME | The user confirms | [IF NOT A.1/85 (No keypad) THEN the terminal may open the channel without explicit confirmation by the user] |
| 6 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The PDN CONNECTIVITY REQUEST shall contain the APN "Test12.rs"] |
| 7 | E-USS/NB-SS ME | ACTIVATE EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 8 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 9 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: GET CHANNEL STATUS 1.1.1 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: GET STATUS 1.1.1 |  |
| 13 | ME UICC | TERMINAL RESPONSE GET STATUS 1.4.1 A Or TERMINAL RESPONSE: GET STATUS 1.4.1B | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 6.3.1

Same as PROACTIVE COMMAND: OPEN CHANNEL 6.3.1 in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B in clause 27.22.4.27.6.4.

PROACTIVE COMMAND: GET STATUS 1.1.1

Same as PROACTIVE COMMAND:GET STATUS from sequence 1.1

TERMINAL RESPONSE: GET STATUS 1.4.1A

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel status:	Channel 1 open, link established or PDP context activated

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 81 | 00 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: GET STATUS 1.4.1B

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel 1 status:	Channel identifier 1 open, Link established or PDP context activated

Channel 2 status:	Channel identifier 2, Link not established or PDP context not activated

:

:

Channel n status:	Channel identifier n, Link not established or PDP context not activated

The number of channel status data objects shall be same as the number of channels(n) supported by the ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note |  |  |  |  |  |  |  |  |  |  |  |
|  | Note: The Terminal Response shall contain as many channel status TLVs as channels are supported by the ME. The channel status TLV coding of the opened channel shall state "Link established or PDP context activated". Not more than one opened channel shall be indicated. Each other channel status TLV coding shall indicate the corresponding channel identifier and shall state "Link is not established or PDP context not activated". As an example, if the mobile supports two channels and channel 1 is opened then the corresponding channel status data objects coding would be : 'B8 02 81 00 B8 02 02 00'. |  |  |  |  |  |  |  |  |  |  |  |

Expected sequence 1.5 (GET STATUS, EPS bearer with APN different from default APN, after a link dropped)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | [Command performed successfully] |
| 5 | UICC  ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 6.3.1 | See initial conditions |
| 6 | ME  UICC | FETCH |  |
| 7 | UICC  ME | PROACTIVE COMMAND: OPEN CHANNEL 6.3.1 |  |
| 8 | ME USER | The terminal shall display the alpha identifier "Open Channel for UICC?" during the confirmation phase | [IF NOT A.1/84 (No display) THEN the terminal shall ignore the alpha identifier] |
| 9 | USER ME | The user confirms | [IF NOT A.1/85 (No keypad) THEN the terminal may open the channel without explicit confirmation by the user] |
| 10 | ME  E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The PDN CONNECTIVITY REQUEST shall contain the APN "Test12.rs"] |
| 11 | E-USS/NB-SS  ME | ACTIVATE EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 12 | ME  E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 13 | ME  UICC | TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A OR TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B | [Command performed successfully OR Command performed with modifications] |
| 14 | E-USS/NB-SS  ME | DEACTIVATE EPS BEARER CONTEXT REQUEST | [Cause: #38 network failure] |
| 15 | ME  E-USS/NB-SS | DEACTIVATE EPS BEARER CONTEXT ACCEPT |  |
| 16 | ME UICC | ENVELOPE EVENT DOWNLOAD: CHANNEL STATUS 1.3.1 | [Link dropped] |
| 17 | UICC ME | PROACTIVE COMMAND PENDING: GET STATUS 1.3.1 |  |
| 18 | ME UICC | FETCH |  |
| 19 | UICC ME | PROACTIVE COMMAND: GET STATUS 1.3.1 |  |
| 20 | ME UICC | TERMINAL RESPONSE: GET STATUS 1.3.1A Or TERMINAL RESPONSE: GET STATUS 1.3.1B Or TERMINAL RESPONSE: GET STATUS 1.3.1C Or TERMINAL RESPONSE: GET STATUS 1.3.1D Or TERMINAL RESPONSE: GET STATUS 1.3.1E | [Command performed successfully] |

PROACTIVE COMMAND: OPEN CHANNEL 6.3.1

Same as PROACTIVE COMMAND: OPEN CHANNEL 6.3.1 in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.1.1A in clause 27.22.4.27.6.4.

TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B

Same as TERMINAL RESPONSE: OPEN CHANNEL 6.1.1B in clause 27.22.4.27.6.4.

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Channel Status

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 99 | 01 | 0A |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE EVENT DOWNLOAD: CHANNEL STATUS 1.3.1

Logically:

Event list

Event list:	Channel Status

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1, link dropped

Coding:

| BER-TLV: | D6 | 0B | 99 | 01 | 0A | 82 | 02 | 82 | 81 | B8 | 02 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 |  |  |  |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: GET STATUS 1.3.1

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: GET STATUS 1.3.1A

Same as TERMINAL RESPONSE: GET STATUS 1.1.1A

TERMINAL RESPONSE: GET STATUS 1.3.1B

Same as TERMINAL RESPONSE: GET STATUS 1.1.1B

TERMINAL RESPONSE: GET STATUS 1.3.1C

Same as TERMINAL RESPONSE: GET STATUS 1.1.1C

TERMINAL RESPONSE: GET STATUS 1.3.1D

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel status:	Channel 1, link dropped

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 01 | 05 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: GET STATUS 1.3.1E

Logically:

Command details

Command number:	1

Command type:	GET STATUS

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status

Channel 1 status:	Channel identifier 1, link dropped

Channel 2 status:	Channel identifier 2, Link not established or PDP context not activated

:

:

Channel n status:	Channel identifier n, Link not established or PDP context not activated

The number of channel status data objects shall be same as the number of channels(n) supported by the ME

Coding:

| BER-TLV: | 81 | 03 | 01 | 44 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B8 | 02 | 01 | 05 | Note |  |  |  |  |  |  |  |
|  | Note: The Terminal Response shall contain as many channel status TLVs as channels are supported by the ME. Each channel status TLV coding except that one for which the link was dropped by the SS shall indicate the corresponding channel identifier and shall state "Link not established or PDP context not activated". As an example, if the mobile supports two channels then the corresponding channel status data objects coding would be : 'B8 02 01 05 B8 02 02 00'. |  |  |  |  |  |  |  |  |  |  |  |

##### 27.22.4.31.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.5.

### 27.22.5	Data Download to UICC

#### 27.22.5.1	SMS-PP Data Download

##### 27.22.5.1.1	Definition and applicability

See clause 3.2.2.

##### 27.22.5.1.2	Conformance requirement

The ME shall support the Proactive UICC: SMS-PP Data Download facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 7.1, clause 8.1, clause 8.7, clause 8.13 and clause 11.

-	TS 31.115 [28] clause 4.

-	TS 23.038 [7] clause 4..

##### 27.22.5.1.3	Test purpose

To verify that the ME transparently passes the "data download via SMS Point-to-point" messages to the UICC.

To verify that the ME returns the RP-ACK message back to the USS, if the UICC responds with '90 00', '91 XX', '62 XX' or '63 XX'.

To verify that the ME with an SMS-PP download feature implementation prior to Rel-11 returns the RP-ERROR message back to the system Simulator, if the UICC responds with '62 XX' or '63 XX' (while the ME with the Rel-11 or later implemention of this feature returns an RP-ACK in this case).

To verify that the ME returns the response data from the UICC back to the USS in the TP-User-Data element of the RP-ACK message, if the UICC returns response data'.

##### 27.22.5.1.4	Method of Test

27.22.5.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the USS.

The "data download via SMS-PP" service is available in the USIM Service Table.

27.22.5.1.4.2	Procedure

Expected Sequence 1.1 (Void)

Expected Sequence 1.2 (Void)

Expected Sequence 1.3 (Void)

Expected Sequence 1.4 (void)

Expected Sequence 1.5 (void)

Expected Sequence 1.6 (Void)

Expected Sequence 1.7 (Void)

Expected Sequence 1.8 (Void)

Expected Sequence 1.9 (SMS-PP Data Download over CS/PS, UTRAN/GERAN)

In case A.1/156 is supported perform the "CS related procedure 1" and continue with "Generic Test Procedure 1 (SMS-PP Data Download)" as defined in this clause 27.22.5.3.4.2 as "Expected Sequence 1.9" with the following parameters:

- Used Network Simulator (NWS): USS  (UMTS System Simulator or System Simulator)

- CS domain is used to send and receive short messages

- ME supports UTRAN or GERAN

CS related procedure:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profle Download and USIM initialisation |
| 2 | ME NWS | ME performs CS/PS or CS registration. |  |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SMS-PP Data Download) in clause 27.22.5.3.4.2 |  |

In case A.1/156 is not supported but A.1/158 is supported perform the "PS related procedure" and continue with "Generic Test Procedure 1 (SMS-PP Data Download)" as defined in this clause 27.22.5.3.4.2 as "Expected Sequence 1.9" with the following parameters:

- Used Network Simulator (NWS): USS (UMTS System Simulator or System Simulator)

- PS domain is used to send and receive short messages

- ME supports UTRAN or GERAN

PS related procedure:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profle Download and USIM initialisation |
| 2 | ME NWS | ME performs CS/PS or PS registration. |  |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SMS-PP Data Download) in clause 27.22.5.3.4.2 |  |

##### 27.22.5.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.9.

#### 27.22.5.2	Cell Broadcast Data Download

##### 27.22.5.2.1	Definition and applicability

See clause 3.2.2.

##### 27.22.5.2.2	Conformance requirement

The ME shall support the Proactive UICC: Cell Broadcast Data Download facility as defined in:

-	TS 31.111 [15] clause 5, clause 7.1.2, clause 8.5, clause 8.7 and clause 11.

-	TS 31.115 [28] clause 5.

-	TS 23.038 [7] clause 5.

##### 27.22.5.2.3	Test purpose

To verify that the ME transparently passes the "data download via Cell Broadcast" messages to the UICC, which contain a message identifier found in EFCBMID.

##### 27.22.5.2.4	Method of Test

27.22.5.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.The elementary files are coded as Toolkit default with the following exeception:

EF PL shall contain an entry indicating "English".

A USS setting up only a GERAN or PCS 1900 cell  shall be used for Expected sequence 1.1, 1.7 and 1.3.

A USS setting up only a UTRAN cell shall be used on and expected sequence 1.4, 1.5 and 1.6.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

27.22.5.2.4.2	Procedure

Expected Sequence 1.1 (Cell Broadcast Data Download (GSM), ENVELOPE(CELL BROADCAST  DOWNLOAD), ME does not display message)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USS ME | CELL BROADCAST 1.1 | Message identifier '10 01' |
| 2 | ME UICC | ENVELOPE (CELL BROADCAST DOWNLOAD) 1.1 |  |
| 3 | UICC ME | SW1, SW2 '90 00' |  |

Cell Broadcast Message 1.1

Logically:

Message Content

Serial Number

Geographical scope:	Cell wide, normal display mode

Message code:	1

Update number:	1

Message Identifier:	"1001"

Data coding Scheme

Message Coding:	English, language using the GSM 7 bit default alphabet

Page Parameter

Total number of pages:	1

Page number:	1

Content of message:	"Cell Broadcast"

Coding:

| Coding | C0 | 11 | 10 | 01 | 01 | 11 | C3 | 32 | 9B | 0D | 12 | CA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DF | 61 | F2 | 38 | 3C | A7 | 83 | 40 | 20 | 10 | 08 | 04 |
|  | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 |
|  | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 |
|  | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 |
|  | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 |
|  | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 |
|  | 10 | 08 | 04 | 02 |  |  |  |  |  |  |  |  |

ENVELOPE: CELL BROADCAST DOWNLOAD 1.1

Logically:

Cell Broadcast Download

Device identities

Source device:	Network

Destination device:	UICC

Cell Broadcast page

Serial Number

Geographical scope:	Cell wide, normal display mode

Message code:	1

Update number:	1

Message Identifier:	"1001"

Data coding Scheme

Message Coding:	English, language using the GSM 7 bit default alphabet

Page Parameter

Number of pages:	1

Page number:	1

Content of message:	"Cell Broadcast"

Coding:

| BER-TLV: | D2 | 5E | 82 | 02 | 83 | 81 | 8C | 58 | C0 | 11 | 10 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 11 | C3 | 32 | 9B | 0D | 12 | CA | DF | 61 | F2 | 38 |
|  | 3C | A7 | 83 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 |
|  | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 |
|  | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 |
|  | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 |
|  | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 |
|  | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 |

Expected Sequence 1.2 (void)

Expected Sequence 1.3 (Cell Broadcast (GSM), ME may display the message)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USS ME | CELL BROADCAST 1.2 | Message identifier '03 E7' |
| 2a | ME USER | ME may display the message |  |
| 2b | ME UICC | ME shall not download the CB message to the UICC using ENVELOPE (CELL BROADCAST DOWNLOAD) |  |
| 3 | USER ME | The user shall use a MMI dependent procedure to initiate the display of the received CB message | [only if message has not been displayed in step 2a] |
| 4 | ME USER | ME displays the message | [only if message has not been displayed in step 2a] |

Cell Broadcast Message 1.2

Logically:

Message Content

Serial Number

Geographical scope:	Cell wide, normal display mode

Message code:	1

Update number:	1

Message Identifier:	"03E7"

Data coding Scheme

Message Coding:	English, language using the GSM 7 bit default alphabet

Page Parameter

Total number of pages:	1

Page number:	1

Content of message:	"Cell Broadcast".

Coding:

| Coding | C0 | 11 | 03 | E7 | 01 | 11 | C3 | 32 | 9B | 0D | 12 | CA |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DF | 61 | F2 | 38 | 3C | A7 | 83 | 40 | 20 | 10 | 08 | 04 |
|  | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 |
|  | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 |
|  | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 |
|  | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 | 10 | 08 |
|  | 04 | 02 | 81 | 40 | 20 | 10 | 08 | 04 | 02 | 81 | 40 | 20 |
|  | 10 | 08 | 04 | 02 |  |  |  |  |  |  |  |  |

Expected Sequence 1.4 (Cell Broadcast (UMTS), ENVELOPE (CELL BROADCAST DOWNLOAD), ME does not display message)

TBD

Expected Sequence 1.5 (Cell Broadcast (UMTS), ENVELOPE (CELL BROADCAST DOWNLOAD), FETCH, MORE TIME, ME does not display message)

TBD

Expected Sequence 1.6 (Cell Broadcast (UMTS), ME displays message)

TBD

Expected Sequence 1.7 (Cell Broadcast (GSM),, ENVELOPE (CELL BROADCAST DATA DOWNLOAD), FETCH, MORE TIME, ME does not display message, User Data Header Payload)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USS ME | CELL BROADCAST Message 1.7 | Message identifier '10 01' |
| 2 | ME UICC | ENVELOPE (CELL BROADCAST DOWNLOAD) 1.7 |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: MORE TIME 1.2 | SW1/SW2 '91 0B' |
| 4 | ME UICC | FETCH 1.2 |  |
| 5 | UICC ME | PROACTIVE COMMAND:MORE TIME 1.2 |  |
| 6 | ME UICC | TERMINAL RESPONSE: MORE TIME 1.2 |  |
| 7 | UICC ME | SW1/SW2 '90 00' | UICC session ended |

CELL BROADCAST Message 1.7

Logically:

Message Content

Serial Number

Geographical scope:	Cell wide, normal display mode

Message code:	1

Update number:	1

Message Identifier:	"1001"

Data coding Scheme

Message Coding:	8 bit data

Message class:	Class 2 (U)SIM specific message

Page Parameter

Total number of pages:	1

Page number:	1

Secured User Header (Content of message)

TP-UDHL		2

IEI			(U)SIM Toolkit Security Headers

IEIL		0

Command Packet Length:	77

Command Header Identifier:	0

Command Header Length:	13

Security Parameter Indicator:	No RC, CC or DS and No PoR reply to the Sending Entity

Ciphering Key Identifier:	Algorithm known implicitly by both entities

Key Identifier:		Algorithm known implicitly by both entities

Toolkit Application Reference:	Proprietary Toolkit Application

Counter:		1

Padding Counter:		0 (no padding is necessary)

Secure Data:		62 octets set to 'DC' (dummy data)

Coding:

| Coding | C0 | 11 | 10 | 01 | 96 | 11 | 02 | 70 | 00 | 00 | 4D | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0D | 00 | 00 | 00 | 00 | BF | FF | 00 | 00 | 00 | 00 | 00 |
|  | 01 | 00 | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC |  |  |  |  |  |  |  |  |

ENVELOPE: CELL BROADCAST DOWNLOAD 1.7

Logically:

Cell Broadcast Download

Device identities

Source device:	Network

Destination device:	UICC

Cell Broadcast page

Serial Number

Geographical scope:	Cell wide, normal display mode

Message code:	1

Update number:	1

Message Identifier:	"1001"

Data coding Scheme

Message Coding:	8 bit data (Message with User Data Header (UDH) structure)

Message class:	Class 2 (U)SIM specific message

Page Parameter

Number of pages:	1

Page number:	1

Secured User Header (Content of message)

TP-UDHL		2

IEI			(U)SIM Toolkit Security Headers

IEIL		0

Command Packet Length:	77

Command Header Identifier:	0

Command Header Length:	13

Security Parameter Indicator:	No RC, CC or DS and No PoR reply to the Sending Entity

Ciphering Key Identifier:	Algorithm known implicitly by both entities

Key Identifier:		Algorithm known implicitly by both entities

Toolkit Application Reference:	Proprietary Toolkit Application

Counter:		1

Padding Counter:		0 (no padding is necessary)

Secure Data:		62 octets set to 'DC' (dummy data)

Coding:

| BER-TLV: | D2 | 5E | 82 | 02 | 83 | 81 | 8C | 58 | C0 | 11 | 10 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 96 | 11 | 02 | 70 | 00 | 00 | 4D | 00 | 0D | 00 | 00 | 00 |
|  | 00 | BF | FF | 00 | 00 | 00 | 00 | 00 | 01 | 00 | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC | DC |

PROACTIVE COMMAND: MORE TIME 1.2

Logically:

Command details

Command number:	1

Command type:	MORE TIME

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 02 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: MORE TIME 1.2

Logically:

Command details

Command number:	1

Command type:	MORE TIME

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 02 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 27.22.5.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.

#### 27.22.5.3	SMS-PP Data Download over IMS

##### 27.22.5.3.1	Definition and applicability

See clause 3.2.2.

For IMS: That the UE correctly implemented the role of an SMS-over-IP receiver is tested in clause 18.2 of TS 34.229-1 [36].

##### 27.22.5.3.2	Conformance requirement

The ME shall support the Proactive UICC: SMS-PP Data Download facility for SMS over IP as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 7.1, clause 8.1, clause 8.7, clause 8.13 and clause 11.

-	TS 31.115 [28] clause 4.

-	TS 23.038 [7] clause 4.

-	TS 34.229 [36], Annexes C.2, C.17 and C.18.

-	TS 24.341 [37], clause 5.2.3.4.

##### 27.22.5.3.3	Test purpose

To verify that the ME transparently passes the "data download via SMS Point-to-point" messages which have been received over IMS to the UICC.

To verify that the ME returns the RP-ACK message back to the E-USS/USS, if the UICC responds with '90 00', '91 XX', '62 XX' or '63 XX'. In case of IMS the RP-ACK message is contained in the SIP MESSAGE for the SM delivery report.

To verify that the ME with an SMS-PP download feature implementation prior to Rel-11 returns the RP-ERROR message in the SIP MESSAGE for the SM delivery report to the E-USS/USS, if the UICC responds with '62 XX' or '63 XX' (while the ME with the Rel-11 or later implemention of this feature return an RP-ACK in this case). In case of IMS the RP-ERROR message is contained in the SIP MESSAGE for the SM delivery report.

To verify that the ME returns available response data from the UICC in the TP-User-Data element of the RP-ACK message back to the E-USS/USS. In case of IMS the RP-ACK message is contained in the SIP MESSAGE for the SM delivery report.

##### 27.22.5.3.4	Method of Test

27.22.5.3.4.1	Initial conditions

The ME is connected to the USIM Simulator. The elementary files are coded as defined for the E-UTRAN/EPC ISIM-UICC in clause 27.22.2C.

For sequence 3.1 the ME is additionally connected to the E-USS.

For sequence 3.2 the ME is additionally connected to the USS.

27.22.5.3.4.2	Procedure

Expected Sequence 3.1 (SMS-PP Data Download over IMS, E-UTRAN)

Perform the "IMS related procedure 1" and continue with "Generic Test Procedure 1 (SMS-PP Data Download)" as defined in this clause as "Expected Sequence 3.1" with the following parameters:

- Used Network Simulator (NWS): E-USS

- SMS-over-IP is used to send and receive short messages

- ME supports eFDD or eTDD and SMS-over-IP

Expected Sequence 3.2 (SMS-PP Data Download over IMS, UTRAN)

Perform the "IMS related procedure 1" and continue with "Generic Test Procedure 1 (SMS-PP Data Download)" as defined in this clause as "Expected Sequence 3.2" with the following parameters:

- Used Network Simulator (NWS): USS  (UMTS System Simulator only)

- SMS-over-IP is used to send and receive short messages

- ME supports UTRAN

IMS related procedure 1:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profle Download, USIM and ISIM initialisation |
| 2 | ME NWS | ME activates the required bearer, discoveres P-CSCF and registers with the values from the ISIM to IMS services | For E-UTRAN: The EPS bearer context activation according to the procedures defined in TS 34.229-1 [36], Annex C.2 and C.18 is performed For UTRAN: For SMS-over-IP a PDP context activation according to the procedures defined in TS 34.229-1 [36], Annex C.2 and C.17 is performed. |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SMS-PP Data Download) |  |

Generic Test Procedure 1 (SMS-PP Data Download)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 4 | NWS ME | SMS-PP Data Download Message 3.1.1 | See Note 1. |
| 5 | ME USER | The ME shall not display the message or alert the user of a short message waiting. |  |
| 6 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 3.1.1 |  |
| 7 | UICC ME | SMS-PP Data Download UICC Acknowledgement 3.1.1 | [SW1 / SW2 of '90 00'] |
| 8 | ME NWS | SMS-PP Data Download UICC Acknowledgement 3.1.1 in the TP-User-Data element of the RP-ACK message. The values of protocol identifier and data coding scheme in RP-ACK shall be as in the original message. | See Note 2. |
| 9 | NWS ME | SMS-PP Data Download Message 3.1.2 | See Note 1. |
| 10 | ME USER | The ME shall not display the message or alert the user of a short message waiting |  |
| 11 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 3.1.2 |  |
| 12 | UICC ME | PROACTIVE COMMAND PENDING: MORE TIME 3.1.1 | [SW1 / SW2 of '91 0B'] |
| 13 | ME NWS | RP-ACK | See Note 2. |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: MORE TIME 3.1.1 |  |
| 16 | ME UICC | TERMINAL RESPONSE: MORE TIME 3.1.1 |  |
| 17 | UICC ME | PROACTIVE UICC SESSION ENDED |  |
| 18 | NWS ME | SMS-PP Data Download Message 3.1.3 | See Note 1. |
| 19 | ME | The ME shall not display the message or alert the user of a short message waiting |  |
| 20 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 3.1.3 |  |
| 21 | UICC ME | SW1 / SW2 of '90 00' |  |
| 22 | ME NWS | RP-ACK | See Note 2. |
| 23 | NWS ME | SMS-PP Data Download Message 3.1.1 | See Note 1. |
| 24 | ME USER | The ME shall not display the message or alert the user of a short message waiting. |  |
| 25 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 3.1.1 |  |
| 26 | UICC ME | SMS-PP Data Download UICC Acknowledgement 3.1.4 | [SW1 / SW2 of '62 xx' or '63 xx'] |
| 27 | ME NWS | IF A.1/154_THEN SMS-PP Data Download UICC Acknowledgement 3.1.4 in the TP-User-Data element of the RP-ACK message. The values of protocol identifier and data coding scheme in RP-ACK shall be as in the original message. ELSE IF (NOT A.1/154) THENSMS-PP Data Download UICC Acknowledgement 3.1.4 in the TP-User-Data element of the RP-ERROR message. The values of protocol identifier and data coding scheme in RP-ERROR shall be as in the original message. | See Note 2. See Note 3. |
| 28 | NWS ME | SMS-PP Data Download Message 3.1.5 | See Note 1. |
| 29 | ME | The ME shall not display the message or alert the user of a short message waiting |  |
| 30 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 3.1.5 |  |
| 31 | UICC ME | SW1 / SW2 of '90 00' |  |
| 32 | ME NWS | RP-ACK | See Note 2. |
| 33 | USER ME | The ME is switched off |  |
| Note 1: In case of IMS the SMS-PP Data Download Message is contained in the message body of the SIP MESSAGE. Note 2: In case of IMS the RP-ACK message is contained in the message body of the SIP MESSAGE. Note 3: In case of IMS the RP-ERROR message is contained in the message body of the SIP MESSAGE. |  |  |  |

SMS-PP (Data Download) Message 3.1.1

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"TestMessage 1"

Coding:

| Coding | 04 | 04 | 91 | 21 | 43 | 7F | 16 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 54 | 65 | 73 | 74 | 4D | 65 | 73 | 73 | 61 |
|  | 67 | 65 | 20 | 31 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 3.1.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"TestMessage 1"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 21 | 43 |
|  | 7F | 16 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 54 | 65 |
|  | 73 | 74 | 4D | 65 | 73 | 73 | 61 | 67 | 65 | 20 | 31 |  |

SMS-PP Data Download UICC Acknowledgement 3.1.1

| Coding | 44 | 61 | 74 | 61 | 20 | 41 | 63 | 6B |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

SMS-PP (Data Download) Message 3.1.2

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"2143"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"TestMessage 2"

Coding:

| Coding | 04 | 04 | 91 | 12 | 34 | 7F | 16 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 54 | 65 | 73 | 74 | 4D | 65 | 73 | 73 | 61 |
|  | 67 | 65 | 20 | 32 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 3.1.2

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"2143"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"TestMessage 2"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 12 | 34 |
|  | 7F | 16 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 54 | 65 |
|  | 73 | 74 | 4D | 65 | 73 | 73 | 61 | 67 | 65 | 20 | 32 |  |

PROACTIVE COMMAND: MORE TIME 1.1.1

Logically:

Command details

Command number:	1

Command type:	MORE TIME

Command qualifier:	"00"

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 02 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: MORE TIME 1.1.1

Logically:

Command details

Command number:	1

Command type:	MORE TIME

Command qualifier:	"00"

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 02 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SMS-PP (Data Download) Message 3.1.3

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"2233"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	Data Coding / Message Class

Message Coding	8 bit data

Message Class	Class 2 (U)SIM Specific Message

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"TestMessage 3"

Coding:

| Coding | 04 | 04 | 91 | 22 | 33 | 7F | F6 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 54 | 65 | 73 | 74 | 4D | 65 | 73 | 73 | 61 |
|  | 67 | 65 | 20 | 33 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 3.1.3

Logically:

SMS-PP Download

Device identities

Source device:		Network

Destination device:		UICC

Address

TON			International number

NPI			"ISDN / telephone numbering plan"

Dialling number string		"112233445566778"

SMS TPDU

TP-MTI		SMS-DELIVER

TP-MMS		No more messages waiting for the MS in this SC

TP-RP		TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI		TP-UD field contains only the short message

TP-SRI		A status report will not be returned to the SME

TP-OA

TON			International number

NPI			"ISDN / telephone numbering plan"

Address value		"2233"

TP-PID		(U)SIM Data download

TP-DCS

Coding Group		Data Coding / Message Class

Message Coding		8 bit data

Message Class		Class 2 (U)SIM Specific Message

TP-SCTS:		01/01/98 00:00:00 +0

TP-UDL		13

TP-UD		"TestMessage 3"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 22 | 33 |
|  | 7F | F6 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 54 | 65 |
|  | 73 | 74 | 4D | 65 | 73 | 73 | 61 | 67 | 65 | 20 | 32 |  |

SMS-PP Data Download UICC Acknowledgement 3.1.4

| Coding | 44 | 61 | 74 | 61 | 20 | 45 | 72 | 72 | 65 | 72 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

SMS-PP (Data Download) Message 3.1.5

Logically:

SMS TPDU

TP-MTI			SMS-DELIVER

TP-MMS			No more messages waiting for the MS in this SC

TP-RP			TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI			TP-UD field contains user data header and a short message

TP-SRI			A status report will not be returned to the SME

TP-OA

TON			International number

NPI			"ISDN / telephone numbering plan"

Address value			"1234"

TP-PID			(U)SIM Data download

TP-DCS

Coding Group			Data Coding / Message Class

Message Coding			8 bit data

Message Class			Class 2 (U)SIM Specific Message

TP-SCTS:			01/01/98 00:00:00 +0

TP-UDL			30

TP-UD

TP-UDHL			2

IEI			(U)SIM Toolkit Security Headers

IEIL			0

SM (8 bit data)

Command Packet Length:		25

Command Header Identifier:	0

Command Header Length:	13

Security Parameter Indicator:	No RC, CC or DS and No PoR reply to the Sending Entity

Ciphering Key Identifier:		Algorithm known implicitly by both entities

Key Identifier:			Algorithm known implicitly by both entities

Toolkit Application Reference:	Proprietary Toolkit Application

Counter:			1

Padding Counter:			0 (no padding is necessary)

Secure Data:			10 octets set to 'DC' (dummy data)

Coding:

| Coding | 44 | 04 | 91 | 21 | 43 | 7F | F6 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 1E | 02 | 70 | 00 | 00 | 19 | 00 | 0D | 00 | 00 |
|  | 00 | 00 | BF | FF | 00 | 00 | 00 | 00 | 00 | 01 | 00 | DC |
|  | DC | DC | DC | DC | DC | DC | DC | DC | DC |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 3.1.5

Logically:

SMS-PP Download

Device identities

Source device:		Network

Destination device:		UICC

Address

TON			International number

NPI			"ISDN / telephone numbering plan"

Dialling number string		"112233445566778"

SMS TPDU

TP-MTI		SMS-DELIVER

TP-MMS		No more messages waiting for the MS in this SC

TP-RP		TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI		TP-UD field contains user data header and a short message

TP-SRI		A status report will not be returned to the SME

TP-OA

TON			International number

NPI			"ISDN / telephone numbering plan"

Address value		"1234"

TP-PID		(U)SIM Data download

TP-DCS

Coding Group		Data Coding / Message Class

Message Coding		8 bit data

Message Class		Class 2 (U)SIM Specific Message

TP-SCTS:		01/01/98 00:00:00 +0

TP-UDL			30

TP-UD

TP-UDHL			2

IEI			(U)SIM Toolkit Security Headers

IEIL			0

SM (8 bit data)

Command Packet Length:		25

Command Header Identifier:	0

Command Header Length:	13

Security Parameter Indicator:	No RC, CC or DS and No PoR reply to the Sending Entity

Ciphering Key Identifier:		Algorithm known implicitly by both entities

Key Identifier:			Algorithm known implicitly by both entities

Toolkit Application Reference:	Proprietary Toolkit Application

Counter:			1

Padding Counter:			0  (no padding is necessary)

Secure Data:			10 octets set to 'DC' (dummy data)

Coding:

| BER-TLV: | D1 | 3E | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 2D | 44 | 04 | 91 | 21 | 43 |
|  | 7F | F6 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 1E | 02 | 70 |
|  | 00 | 00 | 19 | 00 | 0D | 00 | 00 | 00 | 00 | BF | FF | 00 |
|  | 00 | 00 | 00 | 00 | 01 | 00 | DC | DC | DC | DC | DC | DC |
|  | DC | DC | DC | DC |  |  |  |  |  |  |  |  |

##### 27.22.5.3.5	Test requirement

The ME supporting eFDD or eTDD shall operate in the manner defined in expected sequence 3.1.

The ME supporting UTRAN shall operate in the manner defined in expected sequence 3.2.

#### 27.22.5.4	SMS-PP Data Download over SGs in E-UTRAN

##### 27.22.5.4.1	Definition and applicability

See clause 3.2.2.

##### 27.22.5.4.2	Conformance requirement

The ME shall support the Proactive UICC: SMS-PP Data Download facility for SMS over SGs as defined in the following technical specifications:

-	TS 31.111 [15] clause 5, clause 7.1, clause 8.1, clause 8.7, clause 8.13 and clause 11.

-	TS 31.115 [28] clause 4.

-	TS 23.038 [7] clause 4.

-	TS 24.301 [32] clause 5.6.3.1, 5.6.3.3 and 9.9.3.22

##### 27.22.5.4.3	Test purpose

To verify that the ME transparently passes the "data download via SMS Point-to-point" messages to the UICC.

To verify that the ME returns the RP-ACK message back to the USS, if the UICC responds with '90 00', '91 XX', '62 XX' or '63 XX'.

To verify that the ME with an SMS-PP download feature implementation prior to Rel-11 returns the RP-ERROR message back to the system Simulator, if the UICC responds with '62 XX' or '63 XX' (while the ME with the Rel-11 or later implemention of this feature return an RP-ACK in this case).

To verify that the ME returns the response data from the UICC back to the USS in the TP-User-Data element of the RP-ACK message, if the UICC returns response data'.

##### 27.22.5.4.4	Method of Test

27.22.5.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the E-USS/NB-SS.

The "data download via SMS-PP" service is available in the USIM Service Table.

27.22.5.4.4.2	Procedure

Expected Sequence 4.1 (SMS-PP Data Download over SGs, E-UTRAN)

Perform the "SMS over SGs related procedure" and continue with "Generic Test Procedure 1 (SMS-PP Data Download)" as defined in this clause 27.22.5.3.4.2 as "Expected Sequence 4.1" with the following parameters:

- Used Network Simulator (NWS): E-USS/NB-SS

- SMS over SGs (DOWNLINK NAS TRANSPORT and UPLINK NAS TRANSPORT messages)
 is used to send and receive short messages

- ME supports eFDD or eTDD or NB-IoT

- ME supports  SMS-over-SGs.

SMS over SGs related procedure:

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The ME is switched on | ME will perform Profle Download and USIM initialisation |
| 2 | ME NWS | ME performs regular network registration. | UE is afterwards in state Registered, Idle Mode (state 2) according to TS 36.508 [33]. |
| 3 |  | CONTINUE WITH STEP 4 Generic Test Procedure 1 (SMS-PP Data Download) in clause 27.22.5.3.4.2 |  |

##### 27.22.5.4.5	Test requirement

The ME shall operate in the manner defined in expected sequence 4.1.

### 27.22.6	CALL CONTROL BY USIM

#### 27.22.6.1	Procedure for Mobile Originated calls

##### 27.22.6.1.1	Definition and applicability

See clause 3.2.2.

##### 27.22.6.1.2	Conformance requirement

The ME shall support the CALL CONTROL facility as defined in:

-	TS 31.111 [15] clause 7.3

##### 27.22.6.1.3	Test purpose

To verify that for all call set-up attempts , even those resulting from a SET UP CALL proactive UICC command, the ME shall first pass the call set-up details (dialled digits and associated parameters) to the UICC, using the ENVELOPE (CALL CONTROL).

To verify that if the UICC responds with '90 00', the ME shall set up the call with the dialled digits and other parameters as sent to the UICC.

To verify that if the UICC returns response data, the ME shall use the response data appropriately to set up the call as proposed, not set up the call, or set up a call using the data supplied by the UICC.

To verify that, in the case where the initial call set-up request results from a proactive SET UP CALL, if the call control result is "not allowed" or "allowed with modifications", the ME shall inform the UICC using TERMINAL RESPONSE "interaction with call control by UICC or MO short message control by UICC, action not allowed".

To verify that it is possible for the UICC to request the ME to set up an emergency call by supplying the number "112" as the response data.

##### 27.22.6.1.4	Method of tests

27.22.6.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and USS and has performed the location update procedure.

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

1)	The call control service is available in the USIM Service Table.

2)	Only for sequence 1.9:

EFECC (Emergency Call Codes)

Logically:

Emergency call code:				"1020";

Emergency call code alpha identifier:	empty;

Emergency call Service Category:	RFU

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 01 | 02 | FF | FF | FF | FF | FF | FF |

27.22.6.1.4.2	Procedure

Expected Sequence 1.1 (CALL CONTROL BY USIM , set up call attempt by user, the USIM responds with '90 00')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.1.1A Or ENVELOPE CALL CONTROL 1.1.1B | [Option A shall apply for 3GPP parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | 90 00 |  |
| 4 | ME USS | The ME sets up the call without modification | [Set up call to "+01234567890123456789" |

ENVELOPE CALL CONTROL 1.1.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID			Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | Note 5 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |

ENVELOPE CALL CONTROL 1.1.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | 07 | 00 |
|  | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |

Note 1: Length of BER-TLV is '1A' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

Expected Sequence 1.2 (CALL CONTROL BY USIM , set up call attempt by user, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.2.1 A or ENVELOPE CALL CONTROL 1.2.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.2.1 | [Call control result: "Allowed, no modification"] |
| 4 | ME USS | The ME sets up the call without modification | [Set up call to "+01234567890123456789"] |

ENVELOPE CALL CONTROL 1.2.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | Note 5 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |

ENVELOPE CALL CONTROL 1.2.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | 07 | 00 |
|  | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |

Note 1: Length of BER-TLV is '1A' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

CALL CONTROL RESULT 1.2.1

Logically:

Call control result :	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

Expected Sequence 1.3A (CALL CONTROL BY USIM , set up call attempt resulting from a set up call proactive command, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.3.1 PENDING | [This test applies to MEs asking for user confirmation before sending the ENVELOPE CALL CONTROL command] |
| 2 | MEUICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.3.1 | [Set up call to "+012340123456"] |
| 4 | ME USER | ME displays "+012340123456" during user confirmation phase. |  |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 1.3.1A or ENVELOPE CALL CONTROL 1.3.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 1.3.1 | [Call control result: "Allowed, no modification"] |
| 8 | ME USS | The ME sets up the call without modification | [Set up call to "+012340123456"] |
| 9 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 1.3.1 | [command performed successfully] |

Expected Sequence 1.3 B (CALL CONTROL BY USIM , set up call attempt resulting from a set up call proactive command, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.3.1 PENDING | [This test applies to MEs asking for user confirmation after sending the ENVELOPE CALL CONTROL command] |
| 2 | MEUICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.3.1 | [Set up call to "+012340123456"] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.3.1A or ENVELOPE CALL CONTROL 1.3.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 5 | UICC  ME | CALL CONTROL RESULT 1.3.1 | [Call control result: "Allowed, no modification"] |
| 6 | ME USER | ME displays "+012340123456" during user confirmation phase. |  |
| 7 | USER ME | The user confirms the call set up | [user confirmation] |
| 8 | ME USS | The ME sets up the call without modification | [Set up call to "+012340123456"] |
| 9 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 1.3.1 | [command performed successfully] |

PROACTIVE COMMAND: SET UP CALL 1.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"+012340123456"

Address

TON:	International

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 21 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 0D | 2B | 30 | 31 | 32 | 33 | 34 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 86 | 07 | 91 | 10 | 32 | 04 | 21 |
|  | 43 | 65 |  |  |  |  |  |  |  |  |  |

ENVELOPE CALL CONTROL 1.3.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 06 | 07 | 91 | 10 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 21 | 43 | 65 | Note 2 | Note 3 | 13 | Note 5 | 00 | F1 | 10 |
|  | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |  |

ENVELOPE CALL CONTROL 1.3.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 06 | 07 | 91 | 10 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 21 | 43 | 65 | Note 2 | Note 3 | 13 | 07 | 00 | 11 | 10 |
|  | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |  |

Note 1: Length of BER-TLV is '16' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

CALL CONTROL RESULT 1.3.1

Logically:

Call control result :	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

TERMINAL RESPONSE: SET UP CALL 1.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.4 (CALL CONTROL BY USIM , set up call attempt by user, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.4.1 A or ENVELOPE CALL CONTROL 1.4.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.4.1 | [Call control result: "not Allowed"] |
| 4 | ME USS | The ME does not set up the call |  |

ENVELOPE CALL CONTROL 1.4.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"+01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | Note 5 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |

ENVELOPE CALL CONTROL 1.4.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"+01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | 07 | 00 |
|  | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |

Note 1: Length of BER-TLV is '1A' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

CALL CONTROL RESULT 1.4.1

Logically:

Call control result:	'01' = not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

Expected Sequence 1.5A (CALL CONTROL BY USIM , set up call attempt resulting from a set up call proactive command, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.5.1 PENDING | [This test applies to MEs asking for user confirmation before sending the ENVELOPE CALL CONTROL command] |
| 2 | MEUICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.5.1 | [Set up call to "+012340123456" |
| 4 | ME USER | ME displays "+012340123456" during user confirmation phase. |  |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 1.5.1A or ENVELOPE CALL CONTROL 1.5.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 1.5.1 | [Call control result: "Not Allowed"] |
| 8 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 1.5.1 | [Permanent Problem - Interaction with Call Control by USIM] |
| 9 | ME USS | The ME does not set up the call |  |

Expected Sequence 1.5 B (CALL CONTROL BY USIM , set up call attempt resulting from a set up call proactive command, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.5.1 PENDING | [This test applies to MEs asking for user confirmation after sending the ENVELOPE CALL CONTROL command] |
| 2 | MEUICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.5.1 | [Set up call to "+012340123456" |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.5.1A or ENVELOPE CALL CONTROL 1.5.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 5 | UICC  ME | CALL CONTROL RESULT 1.5.1 | [Call control result: "Not Allowed"] [No user confirmation phase because Call Control has disallowed the request] |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 1.5.1 | [Permanent Problem - Interaction with Call Control by USIM] |
| 7 | ME USS | The ME does not set up the call |  |

PROACTIVE COMMAND: SET UP CALL 1.5.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"+012340123456"

Address

TON:	International

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 21 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 0D | 2B | 30 | 31 | 32 | 33 | 34 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 86 | 07 | 91 | 10 | 32 | 04 | 21 |
|  | 43 | 65 |  |  |  |  |  |  |  |  |  |

ENVELOPE CALL CONTROL 1.5.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 06 | 07 | 91 | 10 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 21 | 43 | 65 | Note 2 | Note 3 | 13 | Note 5 | 00 | F1 | 10 |
|  | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |  |

ENVELOPE CALL CONTROL 1.5.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 06 | 07 | 91 | 10 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 21 | 43 | 65 | Note 2 | Note 3 | 13 | 07 | 00 | 11 | 10 |
|  | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |  |

Note 1: Length of BER-TLV is '16' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

CALL CONTROL RESULT 1.5.1

Logically:

Call control result:	'01' = not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

TERMINAL RESPONSE: SET UP CALL 1.5.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Interaction with call control by USIM or MO short message control by USIM, permanent problem

Additional information:	Action not allowed

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.6 (CALL CONTROL BY USIM , set up call attempt by user, allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.6.1 A or ENVELOPE CALL CONTROL 1.6.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.6.1 | [Call control result: "Allowed with modifications", ] |
| 4 | ME USS | The ME sets up the call to "+010203" |  |

ENVELOPE CALL CONTROL 1.6.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | Note 5 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |

ENVELOPE CALL CONTROL 1.6.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | 07 | 00 |
|  | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |

Note 1: Length of BER-TLV is '1A' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 1.6.1

Logically:

Call control result:	'02' = Allowed with modifications

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"010203"

Coding:

| BER-TLV: | 02 | 06 | 86 | 04 | 91 | 10 | 20 | 30 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.7A (CALL CONTROL BY USIM, set up call attempt resulting from a set up call proactive command, allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.7.1 PENDING | [This test applies to MEs asking for user confirmation before sending the ENVELOPE CALL CONTROL command] |
| 2 | MEUICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.7.1 | [Set up call to "+012340123456"] |
| 4 | ME USER | ME displays "+012340123456" during user confirmation phase. |  |
| 5 | USER ME | The user confirms the call set up | [user confirmation] |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 1.7.1A or ENVELOPE CALL CONTROL 1.7.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 1.7.1 | [Call control result: "Allowed with modifications"] |
| 8 | ME USS | The ME sets up the call to "+011111111111" |  |
| 9 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 1.7.1 | [command performed successfully] |

Expected Sequence 1.7 B (CALL CONTROL BY USIM, set up call attempt resulting from a set up call proactive command, allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.7.1 PENDING | [This test applies to MEs asking for user confirmation after sending the ENVELOPE CALL CONTROL command] |
| 2 | MEUICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 1.7.1 | [Set up call to "+012340123456"] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.7.1A or ENVELOPE CALL CONTROL 1.7.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 5 | UICC  ME | CALL CONTROL RESULT 1.7.1 | [Call control result: "Allowed with modifications"] |
| 6 | ME USER | ME displays "+012340123456" during user confirmation phase. |  |
| 7 | USER ME | The user confirms the call set up | [user confirmation] |
| 8 | ME USS | The ME sets up the call to "+011111111111" | [call is set up to modified address] |
| 9 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 1.7.1 | [command performed successfully] |

PROACTIVE COMMAND: SET UP CALL 1.7.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"+012340123456"

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 21 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 0D | 2B | 30 | 31 | 32 | 33 | 34 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 86 | 07 | 91 | 10 | 32 | 04 | 21 |
|  | 43 | 65 |  |  |  |  |  |  |  |  |  |

ENVELOPE CALL CONTROL 1.7.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 06 | 07 | 91 | 10 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 21 | 43 | 65 | Note 2 | Note 3 | 13 | Note 5 | 00 | F1 | 10 |
|  | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |  |

ENVELOPE CALL CONTROL 1.7.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012340123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 06 | 07 | 91 | 10 | 32 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 04 | 21 | 43 | 65 | Note 2 | Note 3 | 13 | 07 | 00 | 11 | 10 |
|  | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |  |

Note 1: Length of BER-TLV is '16' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 1.7.1

Logically:

Call control result:	'02' = Allowed with modifications

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"011111111111"

Coding:

| BER-TLV: | 02 | 09 | 86 | 07 | 91 | 10 | 11 | 11 | 11 | 11 | 11 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: SET UP CALL 1.7.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.8 (CALL CONTROL BY USIM , set up call attempt by user, allowed with modifications: emergency call)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.8.1A or ENVELOPE CALL CONTROL 1.8.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters |
| 3 | UICC  ME | CALL CONTROL RESULT 1.8.1 | [Call control result: "Allowed with modifications"] |
| 4 | ME USS | The ME sets up an emergency call; |  |

ENVELOPE CALL CONTROL 1.8.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | Note 5 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |

ENVELOPE CALL CONTROL 1.8.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | 07 | 00 |
|  | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |

Note 1: Length of BER-TLV is '1A' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 1.8.1

Logically:

Call control result	Allowed, with modification

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Address value	"112"

Coding:

| BER-TLV: | 02 | 05 | 86 | 03 | 81 | 11 | F2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.9 (CALL CONTROL BY USIM , set up call attempt by user, allowed with modifications: number in EFECC)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.9.1A or ENVELOPE CALL CONTROL 1.9.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.9.1 | [Call control result: "Allowed with modifications"] |
| 4 | ME USS | The ME sets up call with the dialled digits "1020". The ME does not set up an emergency call, but sets up a normal call |  |

ENVELOPE CALL CONTROL 1.9.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | Note 5 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |  |  |  |  |

ENVELOPE CALL CONTROL 1.9.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"01234567890123456789"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 0B | 91 | 10 | 32 | 54 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 76 | 98 | 10 | 32 | 54 | 76 | 98 | Note 2 | Note 3 | 13 | 07 | 00 |
|  | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |  |  |  |  |

Note 1: Length of BER-TLV is '1A' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 1.9.1

Logically:

Call control result	Allowed, with modification

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Address value	"1020"

Coding:

| BER-TLV: | 02 | 05 | 86 | 03 | 81 | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.10 (CALL CONTROL BY USIM , set up call attempt by user to an emergency call)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "112" |  |
| 2 | ME UICC | The ME does not send any ENVELOPE CALL CONTROL |  |
| 3 | ME USS | The ME sets up an emergency call |  |

Expected Sequence 1.11 (CALL CONTROL BY USIM , set up call through call register, the USIM responds with '90 00')

Pre-condition: the ME has a mean to register the last dialled number(s), and the ME will store dialled numbers allowed by call control in its register.

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.1.1A or ENVELOPE CALL CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | 90 00 |  |
| 4 | ME USS | The ME sets up the call without modification | [Set up call to "+01234567890123456789"] |
| 5 | USER  ME | End Call. |  |
| 6 | USER  ME | Recall the last dialled number |  |
| 7 | ME  UICC | ENVELOPE CALL CONTROL 1.1.1A or ENVELOPE CALL CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 8 | UICC  ME | 90 00 |  |
| 9 | ME USS | The ME sets up the call without modification | [Set up call to "+01234567890123456789"] |
| 10 | USER  ME | End Call. |  |

Expected Sequence 1.12 (CALL CONTROL BY USIM , set up call through call register, allowed without modification)

Pre-condition: the ME has a mean to register the last dialled number(s), and the ME will store dialled numbers allowed by call control in its register.

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.2.1A or ENVELOPE CALL CONTROL 1.2.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.2.1 | [Call control result: "Allowed, no modification"] |
| 4 | ME USS | The ME sets up the call without modification | [Set up call to "+01234567890123456789"] |
| 5 | User  ME | End the call then call the last dialled number |  |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 1.2.1A or ENVELOPE CALL CONTROL 1.2.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 1.2.1 |  |
| 8 | ME USS | The ME sets up the call without modification | [Set up call to "+01234567890123456789"] |

Expected Sequence 1.13 (CALL CONTROL BY USIM , set up call through call register, not allowed)

Pre-condition: the ME has a mean to register the last dialled number(s), and the ME will store dialled numbers not allowed by call control in its register.

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.4.1A or ENVELOPE CALL CONTROL 1.4.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.4.1 | [Call control result: "not Allowed"] |
| 4 | ME USS | The ME does not set up the call |  |
| 5 | User  ME | The user calls the last dialled number |  |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 1.4.1A or ENVELOPE CALL CONTROL 1.4.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 1.4.1 | [Call control result: "not Allowed"] |
| 8 | ME USS | The ME does not set up the call |  |

Expected Sequence 1.14 (CALL CONTROL BY USIM , set up call through call register, allowed with modifications)

Pre-condition: the ME has a mean to register the last dialled number(s), and the ME will store dialled numbers allowed with modification by call control in its register.

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | Set up a call to "+01234567890123456789" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 1.6.1A or ENVELOPE CALL CONTROL 1.6.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 1.6.1 | [Call control result: "Allowed with modifications"] |
| 4 | ME USS | The ME sets up the call to "+010203" |  |
| 5 | User  ME | End call and then set up a call to "+01234567890123456789" |  |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 1.6.1A or ENVELOPE CALL CONTROL 1.6.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 1.6.1 | [Call control result: "Allowed with modifications"] |
| 8 | ME USS | The ME sets up the call to "+010203" |  |

##### 27.22.6.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.14.

#### 27.22.6.2	Procedure for Supplementary (SS) Services

##### 27.22.6.2.1	Definition and applicability

See clause 3.2.2.

##### 27.22.6.2.2	Conformance requirement

The ME shall support the CALL CONTROL facility as defined in the following technical specifications:

-	TS 31.111 [15] clause 7.3.1.2.

##### 27.22.6.2.3	Test purpose

To verify that the ME first pass the supplementary service control string corresponding to the supplementary service operation to the USIM, using the ENVELOPE (CALL CONTROL) command.

To verify that, if the UICC responds with '90 00', the ME shall send the supplementary service operation with the information as sent to the UICC.

To verify that, if the UICC returns response data, the ME shall use the response data appropriately to send the supplementary service operation as proposed, not send the SS operation, or instead send the USS operation using the data supplied by the UICC.

##### 27.22.6.2.4	Method of tests

27.22.6.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The elementary files are coded as USIM Application Toolkit default with the following exception:

The call control service is available in the USIM Service Table.

The GERAN/UTRAN parameters of the system simulator are:

- Mobile Country Code (MCC) = 001;

- Mobile Network Code (MNC) = 01 ;

- Location Area Code (LAC) = 0001;

- Cell Identity value = 0001.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

27.22.6.2.4.2	Procedure

Expected Sequence 2.1 (CALL CONTROL BY USIM , send SS, the USIM responds with '90 00')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user selects the facility of the ME which requires an unconditional call forward supplementary service operation to be sent to the network (System Simulator). |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 2.1.1A or ENVELOPE CALL CONTROL 2.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | 90 00 |  |
| 4 | ME USS | REGISTER 2.1A or REGISTER 2.1B | [The ME sends the supplementary service operation with the information as sent to the UICC] |
| 5 | USS  ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 |  |

ENVELOPE CALL CONTROL 2.1.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21**10#"

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D4 | Note1 | 82 | 02 | 82 | 81 | 89 | 05 | FF | 2A | A1 | 1A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B0 | 13 | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |

Note 1: Length of BER-TLV is '14' plus the actual length of all the present optional SIMPLE-TLV data objects

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 2.1.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21**10#"

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D4 | 14 | 82 | 02 | 82 | 81 | 89 | 05 | FF | 2A | A1 | 1A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B0 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |

REGISTER 2.1A

Logically (only SS argument):

ACTIVATE SS ARGUMENT

SS-Code:

- Call Forwarding Unconditional

TeleserviceCode

- All Tele Services

Coding:

| Coding | 30 | 06 | 04 | 01 | 21 | 83 | 01 | 00 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

REGISTER 2.1B

Logically (only SS argument):

ACTIVATE SS ARGUMENT

SS-Code:

- Call Forwarding Unconditional

TeleserviceCode

- All Tele Services

LongFTN Supported

Coding:

| Coding | 30 | 08 | 04 | 01 | 21 | 83 | 01 | 00 | 84 | 00 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

RELEASE COMPLETE (SS RETURN RESULT) 2.1

Logically (only from operation code):

ACTIVATE SS RETURN RESULT

ForwardingInfo

SS-Code

- Call Forwarding Unconditional

ForwardFeatureList

ForwardingFeature

TeleserviceCode

- All Tele Services

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: active

Coding:

| Coding | 0C | A0 | 0D | 04 | 01 | 21 | 30 | 08 | 30 | 06 | 83 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 84 | 01 | 07 |  |  |  |  |  |  |  |  |

Expected Sequence 2.2 (CALL CONTROL BY USIM , send SS, allowed without modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user selects the facility of the ME which requires an unconditional call forward supplementary service operation to be sent to the network (System Simulator). |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 2.2.1A or ENVELOPE CALL CONTROL 2.2.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 2.2.1 | [Call control result: "Allowed without modifications"] |
| 4 | ME  USS | REGISTER 2.1A or REGISTER 2.1B | The ME sends the supplementary service operation with the information as sent to the UICC |
| 5 | USS  ME | RELEASE COMPLETE (SS RETURN RESULT) 2.1 |  |

ENVELOPE CALL CONTROL 2.2.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21**10#"

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID			Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 89 | 05 | FF | 2A | A1 | 1A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B0 | 13 | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |

Note 1: Length of BER-TLV is '14' plus the actual length of all the present optional SIMPLE-TLV data objects

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 2.2.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21**10#"

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D4 | 14 | 82 | 02 | 82 | 81 | 89 | 05 | FF | 2A | A1 | 1A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B0 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |

CALL CONTROL RESULT 2.2.1

Logically:

Call control result	Allowed, no modifications

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

Expected Sequence 2.3 (CALL CONTROL BY USIM , send SS, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user selects the facility of the ME which requires an unconditional call forward supplementary service operation to be sent to the network (System Simulator). |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 2.3.1A or ENVELOPE CALL CONTROL 2.3.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 2.3.1 | [Call control result: "Not Allowed"] |
| 4 | ME USS | The ME does not send the supplementary service operation |  |

ENVELOPE CALL CONTROL 2.3.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21#"

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 89 | 03 | FF | 2A | B1 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 2.3.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21#"

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D4 | 12 | 82 | 02 | 82 | 81 | 89 | 03 | FF | 2A | B1 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |  |  |

CALL CONTROL RESULT 2.3.1

Logically:

Call control result	Not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

Expected Sequence 2.4 (CALL CONTROL BY USIM , send SS, allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user selects the facility of the ME which requires an unconditional call forward supplementary service operation to be sent to the network (System Simulator). |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 2.4.1A or ENVELOPE CALL CONTROL 2.4.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 2.4.1 | [Call control result: "Allowed with modifications"] |
| 4 | ME USS | REGISTER 2.4A or REGISTER 2.4B | [The ME sends the supplementary service operation with the information as sent by the UICC] |
| 5 | USS  ME | RELEASE COMPLETE (SS RETURN RESULT) 2.4 |  |

ENVELOPE CALL CONTROL 2.4.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21#"

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 89 | 03 | FF | 2A | B1 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 2.4.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

SS String

TON/NPI:	"FF"

Dialling number string	"*21#"

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D4 | 12 | 82 | 02 | 82 | 81 | 89 | 03 | FF | 2A | B1 | 13 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |  |  |

CALL CONTROL RESULT 2.4.1

Logically:

Call control result	Allowed, with modifications

SS String

TON/NPI	"FF"

SS String	"*#21#"

Coding:

| BER-TLV: | 02 | 06 | 89 | 04 | FF | BA | 12 | FB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

REGISTER 2.4A

Logically (only SS argument):

INTERROGATE SS ARGUMENT

SS-Code

- Call Forwarding Unconditional

Coding:

| BER-TLV | 30 | 03 | 04 | 01 | 21 |
| --- | --- | --- | --- | --- | --- |

REGISTER 2.4B

Logically (only SS argument):

INTERROGATE SS ARGUMENT

SS-Code

- Call Forwarding Unconditional

LongFTN Supported

Coding:

| BER-TLV | 30 | 05 | 04 | 01 | 21 | 84 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- |

RELEASE COMPLETE (SS RETURN RESULT) 2.4

Logically (only from operation code):

INTERROGATE SS RESULT

Call Forwarding Unconditional

SS-Status

- state ind.: operative

- provision ind.: provisioned

- registration ind.: registered

- activation ind.: not active

Coding:

| BER-TLV | 80 | 01 | 06 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 27.22.6.2.5	Test requirement

The ME shall operate in the manner defined in expected sequences 2.1 to 2.4.

#### 27.22.6.3	Interaction with Fixed Dialling Number (FDN)

##### 27.22.6.3.1	Definition and applicability

See clause 3.2.2.

##### 27.22.6.3.2	Conformance requirement

The ME shall support the CALL CONTROL facility as defined in:

-	TS 31.111 [15] clause 7.3.1.4.

##### 27.22.6.3.3	Test purpose

To verify that the ME checks that the number entered through the MMI is on the FDN list.

To verify that, if the MMI input does not pass the FDN check, the call shall not be set up.

To verify that, if the MMI input does pass the FDN check, the ME shall pass the dialled digits and other parameters to the UICC, using the ENVELOPE (CALL CONTROL) command.

To verify that, if the UICC responds with "allowed, no modification", the ME shall set up the call as proposed.

To verify that, if the UICC responds with "not allowed", the ME shall not set up the call.

To verify that, if the UICC responds with "allowed with modifications", the ME shall set up the call in accordance with the response from the UICC. If the modifications involve changing the dialled digits, the ME shall not re-check this modified number against the FDN list.

##### 27.22.6.3.4	Method of tests

27.22.6.3.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The elementary files are coded as SIM Application Toolkit default with the following exceptions:

The call control service is available in the USIM Service Table.

Fixed Dialling Number service is enabled.

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01 ;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

27.22.6.3.4.2	Procedure

Expected Sequence 3.1 (CALL CONTROL BY USIM , set up a call not in EFFDN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "4321" |  |
| 2 | ME UICC | The ME does not send the ENVELOPE (CALL CONTROL) command to the USIM. |  |
| 3 | ME USS | The ME does not set up the call. |  |

Expected Sequence 3.2 (CALL CONTROL BY USIM , set up a call in EFFDN , the USIM responds with '90 00')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "123" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 3.2.1A or ENVELOPE CALL CONTROL 3.2.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | 90 00 |  |
| 4 | ME USS | The ME sets up the call without modification | [Set up call to "123"] |

ENVELOPE CALL CONTROL 3.2.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 21 | F3 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 3 | 13 | Note 5 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |

ENVELOPE CALL CONTROL 3.2.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 21 | F3 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 3 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

Expected Sequence 3.3 (CALL CONTROL BY USIM , set up a call in EFFDN, Allowed without modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "9876" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 3.3.1A or ENVELOPE CALL CONTROL 3.3.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 3.3.1 | [Call control result: "Allowed without modifications"] |
| 4 | ME USS | The ME sets up the call without modification | [Set up call to "9876"] |

ENVELOPE CALL CONTROL 3.3.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 89 | 67 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 3 | 13 | Note 5 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |

ENVELOPE CALL CONTROL 3.3.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 89 | 67 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 3 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 3.3.1

Logically:

Call control result	Allowed, no modifications

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

Expected Sequence 3.4 (CALL CONTROL BY USIM , set up a call in EFFDN , Not Allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "9876" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 3.4.1A or ENVELOPE CALL CONTROL 3.4.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 3.4.1 | [Call control result: "Not Allowed"] |
| 4 | ME USS | The ME does not set up the call |  |

ENVELOPE CALL CONTROL 3.4.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 89 | 67 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 3 | 13 | Note 5 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |

ENVELOPE CALL CONTROL 3.4.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 89 | 67 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 3 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 3.4.1

Logically:

Call control result	Not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

Expected Sequence 3.5 (CALL CONTROL BY USIM , set up a call in EFFDN , Allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "9876" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 3.5.1A or ENVELOPE CALL CONTROL 3.5.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 3.5.1 | [Call control result: "Allowed with modifications"] |
| 4 | ME USS | The ME sets up the call with data sent by the UICC | [Set up call to "3333"] |

ENVELOPE CALL CONTROL 3.5.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 6

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 89 | 67 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note3 | 13 | Note 5 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 6 | Note 4 |

ENVELOPE CALL CONTROL 3.5.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 89 | 67 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note3 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 4 |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Subaddress may be present at this place. If present, it may take up several octets.

Note 4: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 3.5.1

Logically:

Call control result	Allowed with modifications

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Address value	"3333"

Coding:

| BER-TLV: | 02 | 05 | 86 | 03 | 81 | 33 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- |

##### 27.22.6.3.5	Test requirement

The ME shall operate in the manner defined in expected sequences 3.1 to 3.5.

#### 27.22.6.4	Support of Barred Dialling Number (BDN) service

##### 27.22.6.4.1	Definition and applicability

Barred Dialling Numbers (BDN) is a service defined for the USIM. An enabled BDN service results in call restrictions for the ME. The call restrictions are controlled by the Terminal. To ascertain the type of USIM and state of BDN the ME runs the BDN capability request procedure during UICC-Terminal initialisation. At the time an emergency call is setup using the emergency call code read from the EFECC, the Rel-4+ ME shall use the category of the emergency service indicated.

##### 27.22.6.4.2	Conformance requirement

1)	Recognising the state of the USIM (BDN enabled) the ME shall perform the UICC initialisation procedure as specified.

2)	The ME shall prevent call set-up to any number stored in EFBDN if BDN service is enabled.

3)	The ME shall allow call set-up to any number stored in EFBDN if BDN service is disabled.

4)	Any change to the EFBDN or EFEST does request PIN2.

5)	The ME allows call set-up of an emergency call, even if this number is stored in the USIM.

References:

-	R99: TS 22.101[22], clause 8 and A.19;

-	Rel-4: TS 22.101[22], clause 9 and A.20;

-	Rel-5+: TS 22.101[22], clause 10 and A.21;

-	TS 31.102[14], subclauses 4.2.44, 4.4.2.3, 5.1.1 and 5.3.2;

-	TS 24.008[10], subclause 10.5.4.33;

-	TS 31.111[15], subclause 7.3.1.5

##### 27.22.6.4.3	Test purpose

1)	To verify that the Terminal rejects call set-up to any number that has an entry in EFBDN if BDN service is enabled.

2)	To verify that the Terminal allows call set-up to any number not stored in EFBDN.

3)	To verify that the Terminal allows emergency call set-up even if the number is stored in EFBDN.

4)	To verify that the Rel-4+ Terminal reads correctly the emergency service category stored in EFECC.

5)	To verify that, if the UICC responds with "not allowed", the ME does not set up the call.

6)	To verify that, if the UICC responds with "allowed, no modification", the ME shall set up the call (or the supplementary service operation) as proposed.

7)	To verify that, if the UICC responds with "allowed with modifications", the ME sets up the call in accordance with the response from the UICC. If the modifications involve changing the dialled number the ME does not re-check this modified number against the FDN list when FDN is enabled.

8)	To verify that updating EF BDN or changing the status of BDN service shall be performed by the use of second application PIN only.

9)	To verify that the ME allows call set up to a BDN number if BDN service is disabled.

##### 27.22.6.4.4	Method of tests

27.22.6.4.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The call control service is available in the USIM Service Table.

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

Barred Dialling Number service is enabled.

Fixed Dialling Number service is disabled.

Only prior to the execution of expected sequence 4.3 the FDN service shall be enabled.

The Second Application PIN (key reference 81) shall be enabled, but not verified.

Only in expected sequence 4.2B EFECC shall be used with the following values:

EFECC (Emergency Call Codes)

Logically:	Emergency call code:	"122";

Emergency call code alpha identifier:	"TEST";

Emergency call Service Category:	"Mountain Rescue".

| Coding: | B1 | B2 | B3 | B4 | B5 | B6 | B7 | B8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Hex | 21 | F2 | FF | 54 | 45 | 53 | 54 | 10 |

The GERAN/UTRAN parameters of the system simulator are:

- Mobile Country Code (MCC) = 001;

- Mobile Network Code (MNC) = 01 ;

- Location Area Code (LAC) = 0001;

- Cell Identity value = 0001.

The PCS 1900 parameters of the system simulator are:

Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

27.22.6.4.4.2	Procedure

Expected Sequence 4.1 (CALL CONTROL BY USIM, BDN service enabled)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "+1357924680" | [Number as stored in record 1 of EF BDN] |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 4.1.1A or ENVELOPE CALL CONTROL 4.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 4.1.1 | [Call control result: "Not Allowed"] |
| 4 | ME USS | The ME does not set up the call |  |
| 5 | User  ME | The user sets up a call to the number stored in record 1 of EF ADN |  |
| 6 | ME  UICC | ENVELOPE CALL CONTROL 4.1.2A or ENVELOPE CALL CONTROL 4.1.2B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 7 | UICC  ME | CALL CONTROL RESULT 4.1.2 | [Call control result: "Allowed without modifications"] |
| 8 | ME USS | The ME sets up the call without modification |  |
| 9 | User  ME | The user sets up a call to "123456" |  |
| 10 | ME  UICC | ENVELOPE CALL CONTROL 4.1.3A or ENVELOPE CALL CONTROL 4.1.3B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 11 | UICC  ME | CALL CONTROL RESULT 4.1.2 | [Call control result: "Allowed without modifications"] |
| 12 | ME USS | The ME sets up the call without modification |  |
| 13 | User  ME | The user sets up a call to "1111" |  |
| 14 | ME  UICC | ENVELOPE CALL CONTROL 4.1.4A or ENVELOPE CALL CONTROL 4.1.4B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 15 | UICC  ME | CALL CONTROL RESULT 4.1.3 | [Call control result: "Allowed with modifications"] |
| 16 | ME USS | The ME sets up the call with data sent by the UICC | [Set up call to "2222"] |
| 17 | User  ME | The user shall use a MMI dependent procedure to initiate the disabling of the BDN service |  |
| 18 | ME  User | Ask for second application PIN verification |  |
| 19 | User  ME | The user shall enter the second application PIN |  |
| 20 | ME  UICC | Update EF EST to disable BDN service |  |
| 21 | UICC  ME | UICC responds with SW = "90 00" |  |
| 22 | ME  User | Indicate that the BDN service was disabled successfully |  |
| 23 | User  ME | The user uses the MMI to store the directory number "+876543210" in EFBDN as barred dialling number 1 (record 1). | [The alpha identifier is not changed.] |
| 24 | ME  UICC | Update EF BDN |  |
| 25 | UICC  ME | UICC responds with SW = "90 00" |  |
| 26 | ME  User | The user attempts to set up a call to "+876543210". |  |
| 27a | ME  UICC | No Envelope call control is sent |  |
| 27b | ME USS | The ME sets up the call without modification |  |

ENVELOPE CALL CONTROL 4.1.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	International

NPI	"ISDN / telephone numbering plan"

Dialling number string	"1357924680"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 06 | 91 | 31 | 75 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 64 | 08 | Note 2 | 13 | Note 4 | 00 | F1 | 10 | 00 | 01 | 00 | 01 |
|  | Note5 | Note 3 |  |  |  |  |  |  |  |  |  |  |

ENVELOPE CALL CONTROL 4.1.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	International

NPI	"ISDN / telephone numbering plan"

Dialling number string	"1357924680"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 06 | 91 | 31 | 75 | 29 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 64 | 08 | Note 2 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |
|  | Note 3 |  |  |  |  |  |  |  |  |  |  |  |

Note 1: Length of BER-TLV is '15' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 4: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 5: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 4.1.2A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 5

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 21 | F3 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 4 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 5 | Note 3 |  |

ENVELOPE CALL CONTROL 4.1.2B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 21 | F3 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 4: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 5: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 4.1.3A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 5

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 04 | 81 | 21 | 43 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 13 | Note 4 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 5 | Note 3 |

ENVELOPE CALL CONTROL 4.1.3B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123456"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 04 | 81 | 21 | 43 | 65 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |

Note 1: Length of BER-TLV is '13' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 4: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'.

Note 5: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE CALL CONTROL 4.1.4A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"1111"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 5

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 11 | 11 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 4 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 5 | Note 3 |  |

ENVELOPE CALL CONTROL 4.1.4B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"1111"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 11 | 11 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 4: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 5: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 4.1.1

Logically:

Call control result	Not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

CALL CONTROL RESULT 4.1.2

Logically:

Call control result	Allowed, no modifications

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

CALL CONTROL RESULT 4.1.3

Logically:

Call control result	Allowed with modifications

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Address value	"2222"

Coding:

| BER-TLV: | 02 | 05 | 86 | 03 | 81 | 22 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 4.2A (CALL CONTROL BY USIM, BDN service enabled, interaction with emergency call codes, R99 only)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up an emergency call to an emergency number stored in the terminal. | The used emergency number shall be one of the emergency call codes, which are available when a SIM/USIM is present, according to TS 22.101[22], subclause 8 is used (i.e. "112", or "911"). |
| 2a | ME  UICC | No Envelope call control is sent |  |
| 2b | ME  USS | The ME shall allow an emergency call by indicating the call setup as "Emergency Call". |  |
| 3 | User  ME | End the emergency call. |  |

Expected Sequence 4.2B (CALL CONTROL BY USIM, BDN service enabled, interaction with emergency call codes, Rel-4+)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up an emergency call to an emergency number stored in the terminal. | The used emergency number shall be one of the emergency call codes, which are available when a SIM/USIM is present, according to TS 22.101[22], subclause 9 (Rel-4) or 10 (Rel-5+) is used (i.e. "112", or "911"). |
| 2a | ME  UICC | No Envelope call control is sent |  |
| 2b | ME  USS | The ME shall allow an emergency call by indicating the call setup as "Emergency Call". |  |
| 3 | User  ME | End the emergency call. |  |
| 4 | User  ME | The user sets up an emergency call to an emergency number stored in the USIM. |  |
| 5a | ME  UICC | No Envelope call control is sent |  |
| 5b | ME  USS | The ME shall allow an emergency call by sending the emergency service category correctly as "Mountain Rescue". |  |
| 6 | User  ME | End the emergency call. |  |

Expected Sequence 4.3 (CALL CONTROL BY USIM , FDN and BDN enabled, set up a call in EFFDN, Allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "123" |  |
| 2 | ME  UICC | ENVELOPE CALL CONTROL 4.3.1A or ENVELOPE CALL CONTROL 4.3.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | CALL CONTROL RESULT 4.3.1 | [Call control result: "Allowed with modifications"] |
| 4 | ME USS | The ME sets up the call with data sent by the UICC | [Set up call to "24680"the ME does not re-check this modified number against the FDN list] |

ENVELOPE CALL CONTROL 4.3.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID	RNC-id value (for Rel-4 onwards), see also Note 5

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 21 | F3 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 4 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 5 | Note 3 |  |

ENVELOPE CALL CONTROL 4.3.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"123"

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Subaddress

This parameter is optional. If present, the contents shall not be checked.

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

| BER-TLV: | D4 | Note 1 | 82 | 02 | 82 | 81 | 86 | 03 | 81 | 21 | F3 | Note 2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |

Note 1: Length of BER-TLV is '12' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Capability configuration parameters 1 may be present at this place. If present, it may take up several octets.

Note 3: Capability configuration parameters 2 may be present at this place. If present, it may take up several octets.

Note 4: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 5: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 4.3.1

Logically:

Call control result	Allowed with modifications

Address

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Address value	"24680"

Coding:

| BER-TLV: | 02 | 06 | 86 | 04 | 81 | 42 | 86 | F0 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

##### 27.22.6.4.5	Test requirement

The ME shall operate in the manner defined in expected sequences 4.1 to 4.3.

#### 27.22.6.5	Barred Dialling Number (BDN) service handling for terminals not supporting BDN

##### 27.22.6.5.1	Definition and applicability

Barred Dialling Numbers (BDN) is a service defined for the USIM. An enabled BDN service results in call restrictions for the ME. The call restrictions are controlled by the Terminal. If BDN is enabled, an ME which does not support Call Control shall allow emergency calls but shall not allow MO-CS calls.

##### 27.22.6.5.2	Conformance requirement

1)	Recognising the state of the USIM (BDN enabled) the ME shall perform the UICC initialisation procedure as specified.

2)	The ME shall prevent MO-CS call set-up to any number except to emergency call numbers if the BDN service is enabled.

References:

-	Rel-5+: TS 22.101[22], clause 10 and A.21;

TS 31.102[14], subclauses 4.2.44, 4.4.2.3, 5.1.1.2 and 5.3.2;

TS 31.111[15], subclause 7.3.1.5

##### 27.22.6.5.3	Test purpose

1)	To verify that the Terminal rejects MO-CS call set-up to any number except to emergency call numbers if BDN service is enabled.

2)	To verify that the Terminal allows emergency call set-up even if the BDN service is enabled.

##### 27.22.6.5.4	Method of tests

27.22.6.5.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The call control service is available in the USIM Service Table.

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

Barred Dialling Number service is enabled.

27.22.6.5.4.2	Procedure

Expected Sequence 5.1 (CALL CONTROL BY USIM, BDN service enabled, ME not supporting BDN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user sets up a call to "+1357924680" | [Number as stored in record 1 of EF BDN] |
| 2a | ME  UICC | No ENVELOPE CALL CONTROL is sent |  |
| 2b | ME USS | The ME does not set up the call |  |
| 3 | User  ME | The user sets up a call to the number stored in record 1 of EF ADN |  |
| 4a | ME  UICC | No ENVELOPE CALL CONTROL is sent |  |
| 4b | ME USS | The ME does not set up the call |  |
| 5 | User  ME | The user sets up an emergency call to "112" |  |
| 6a | ME  UICC | No ENVELOPE CALL CONTROL is sent |  |
| 6b | ME USS | The ME sets up the emergency call to "112" |  |
| 7 | User  ME | The user shall terminate the emergency call after 5 seconds. The ME returns to idle mode. |  |

### 27.22.7	EVENT DOWNLOAD

#### 27.22.7.1	MT Call Event

##### 27.22.7.1.1	MT Call Event (normal)

27.22.7.1.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.1.1.2	Conformance requirement

The ME shall support the EVENT: MT Call event as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, and clause 8.25.

27.22.7.1.1.3	Test purpose

To verify that the ME informs the UICC that an Event: MT Call has occurred using the ENVELOPE (EVENT DOWNLOAD - MT Call) command.

27.22.7.1.1.4	Method of test

27.22.7.1.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

27.22.7.1.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD -MT Call event)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | USS  ME | CALL SET UP without CLI | [MT Call Set Up Without CLI] |
| 6 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - MT Call 1.1.1 |  |
| 7 | USS  ME | CALL DISCONNECT |  |
| 8 | USS  ME | CALL SET UP with CLI | [MT Call Set Up With CLI] |
| 9 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - MT Call 1.1.2 |  |
| 10 | USS  ME | CALL DISCONNECT |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	MT call

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 00 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - MT CALL 1.1.1

Logically:

Event list:	MT call event

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 00 | 82 | 02 | 83 | 81 | 1C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - MT CALL 1.1.2

Logically:

Event list:	MT call event

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Address:

TON	Unknown

NPI	"ISDN / telephone numbering plan"

Dialling number string	"9876"

Coding:

| BER-TLV: | D6 | 0F | 19 | 01 | 00 | 82 | 02 | 83 | 81 | 1C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 86 | 03 | 81 | 89 | 67 |  |  |  |  |  |  |  |

27.22.7.1.1.5	Test requirement

The behaviour of the test is as defined in 'Expected Sequence 1.1'.

#### 27.22.7.2	Call Connected Event

##### 27.22.7.2.1	Call Connected Event (MT and MO call)

27.22.7.2.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.2.1.2	Conformance requirement

The ME shall support the EVENT: Call Connected event as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, clause 8.25 and clause 8.28.

27.22.7.2.1.3	Test purpose

To verify that the ME informs the UICC that an Event: Call Connected has occurred using the ENVELOPE (EVENT DOWNLOAD -Call Connected) command.

To verify that the ME provides the correct value of the Transaction identifier to the UICC in the Call Connected Event.

27.22.7.2.1.4	Method of test

27.22.7.2.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

27.22.7.2.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD -CALL CONNECTED)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 | [EVENT: Call Connected active] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | USS  ME | SETUP | [MT Call] TI = 0 |
| 6 | USER  ME | Accept Call Set Up |  |
| 7 | MEUSS | CONNECT |  |
| 8 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.1.1 |  |
| 9 | USS  ME | DISCONNECT |  |
| 10 | USER  ME | Initiate Call to "123" |  |
| 11 | ME  USS | SETUP | [MO Call] TI = 0 |
| 12 | USS  ME | CONNECT |  |
| 13 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.1.2 |  |
| 14 | USER  ME | End Call |  |
| 15 | ME  USS | DISCONNECT |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.1.1

Logically:

Event list:	Call connected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.1.2

Logically:

Event list:	Call connected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 83 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (EVENT DOWNLOAD -CALL CONNECTED, simultaneous calls, MT call followed by MO call)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.2.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 | [EVENT: Call Connected active] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 |  |
| 5 | USS  ME | SETUP | [MT Call] TI = 0 |
| 6 | USER  ME | Accept Call Set Up |  |
| 7 | MEUSS | CONNECT |  |
| 8 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.2.1 |  |
| 9 | USER  ME | Initiate Call to "123" |  |
| 10 | ME  USS | SETUP | [MO Call] TI = 1 |
| 11 | USS  ME | CONNECT |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.2.2 |  |
| 13 | USER  ME | End Call "123" |  |
| 14 | ME  USS | DISCONNECT Call "123" | [MO Call] TI = 1 |
| 15 | USS  ME | DISCONNECT MT Call | [MT Call] TI = 0 |

PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.2.1

Logically:

Event list:	Call connected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.2.2

Logically:

Event list:	Call connected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	1 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 83 | 81 | 1C | 01 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.3 (EVENT DOWNLOAD -CALL CONNECTED, simultaneous calls, MO call followed by MO call)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.3.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.3.1 | [EVENT: Call Connected active] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.3.1 |  |
| 5 | USER  ME | Initiate Call to "123" |  |
| 6 | ME  USS | SETUP | [MO Call] TI = 0 |
| 7 | USS ME | CONNECT |  |
| 8 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.3.1 |  |
| 9 | USER  ME | Initiate Call to "456" |  |
| 10 | ME  USS | SETUP | [MO Call] TI = 1 |
| 11 | USS  ME | CONNECT |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.3.2 |  |
| 13 | USER  ME | End Call "456" |  |
| 14 | ME  USS | DISCONNECT Call "456" | [MO Call] TI = 1 |
| 15 | USS  ME | DISCONNECT Call "123" | [MO Call] TI = 0 |

PROACTIVE COMMAND: SET UP EVENT LIST 1.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.3.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.3.1

Logically:

Event list:	Call connected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 83 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.3.2

Logically:

Event list:	Call connected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	1 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 83 | 81 | 1C | 01 | 90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.4 (EVENT DOWNLOAD -CALL CONNECTED, simultaneous calls, MO call followed by MT call)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.4.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.4.1 | [EVENT: Call Connected active] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.3.1 |  |
| 5 | USER  ME | Initiate Call to "123" |  |
| 6 | ME  USS | SETUP | [MO Call] TI = 0 |
| 7 | USS ME | CONNECT |  |
| 8 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.4.1 |  |
| 9 | USS  ME | SETUP | [MT Call] TI = 0 |
| 10 | USER  ME | Accept Call Set Up |  |
| 11 | MEUSS | CONNECT |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Call Connected 1.4.2 |  |
| 13 | ME  USS | DISCONNECT MT Call | [MO Call] TI = 0 |
| 14 | USS  ME | DISCONNECT MO Call | [MO Call] TI = 0 |

PROACTIVE COMMAND: SET UP EVENT LIST 1.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.4.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.4.1

Logically:

Event list:	Call connected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 83 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 1.4.2

Logically:

Event list:	Call connected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.7.2.1.5	Test requirement

The behaviour of the test is as defined in Expected Sequences 1.1 to 1.4.

##### 27.22.7.2.2	Call Connected Event (ME supporting SET UP CALL)

27.22.7.2.2.1	Definition and applicability

See clause 3.2.2.

27.22.7.2.2.2	Conformance requirement

Additionally the ME shall support the SET UP CALL Proactive UICC Command as defined in:

-	TS 31.111 [15] clause 7.5, clause 6.4.13 and clause 6.6.12.

27.22.7.2.2.3	Test purpose

To verify that the ME informs the UICC that an Event: Call Connected has occurred using the ENVELOPE (EVENT DOWNLOAD -Call Connected) command.

27.22.7.2.2.4	Method of test

27.22.7.2.2.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

27.22.7.2.2.4.2	Procedure

Expected Sequence 2.1 (EVENT DOWNLOAD -CALL CONNECTED, ME supporting SET UP CALL)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 2.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 2.1.1 | [EVENT: Call Connected active] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 2.1.1 |  |
| 5 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP CALL 2.1.1 |  |
| 6 | ME  UICC | FETCH |  |
| 7 | UICC  ME | PROACTIVE COMMAND: SET UP CALL 2.1.1 | [SAT Call] |
| 8 | ME USER | ME displays "+012340123456" during the user confirmation phase. | ME BEHAVIOUR: SET UP CALL |
| 9 | USER  ME | Confirm call set up |  |
| 10 | ME  USS | SETUP | TI=0 |
| 11 | USS  ME | CONNECT |  |
| 12 | ME  UICC | TERMINAL RESPONSE: SET UP CALL 2.1.1 |  |
| 13 | ME  UICC | ENVELOPE: CALL CONNECTED 2.1.1 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 2.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Connected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 2.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: SET UP CALL 2.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"+012340123456"

Address

TON:	International

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"012340123456"

Coding:

| BER-TLV: | D0 | 21 | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 81 | 83 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 | 0D | 2B | 30 | 31 | 32 | 33 | 34 | 30 | 31 | 32 |
|  | 33 | 34 | 35 | 36 | 86 | 07 | 91 | 10 | 32 | 04 | 21 |
|  | 43 | 65 |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP CALL 2.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP CALL

Command qualifier:	Only if not currently busy on another call

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 10 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL CONNECTED 2.1.1

Logically:

Event list:	Call connected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 01 | 82 | 02 | 83 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.7.2.2.5	Test requirement

The behaviour of the test is as defined in 'Expected Sequence 2.1'.

#### 27.22.7.3	Call Disconnected Event

##### 27.22.7.3.1	Call Disconnected Event

27.22.7.3.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.3.1.2	Conformance requirement

The ME shall support the EVENT: Call Disconnected event as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, and clause 8.25.

27.22.7.3.1.3	Test purpose

To verify that the ME informs the UICC that an Event: Call Disconnected has occurred using the ENVELOPE (EVENT DOWNLOAD -Call Disconnected) command.

27.22.7.3.1.4	Method of test

27.22.7.3.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

27.22.7.3.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD -CALL DISCONNECTED)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 | [EVENT: Call Disconnected active] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | USS  ME | SETUP | [ incoming call ] TI=0 |
| 6 | USER  ME | Accept Call Set Up |  |
| 7 | USS  ME | RELEASE | [MT RELEASE] |
| 8 | ME UICC | ENVELOPE: CALL DISCONNECTED 1.1.1 |  |
| 9 | USS  ME | SETUP | [ incoming call ] TI=0 |
| 10 | USER  ME | Accept Call Set Up |  |
| 11 | USS  ME | RELEASE COMPLETE | [MT RELEASE COMPLETE] |
| 12 | ME UICC | ENVELOPE: CALL DISCONNECTED 1.1.1 |  |
| 13 | USS  ME | SETUP | [ incoming call ] TI=0 |
| 14 | USER  ME | Accept Call Set Up |  |
| 15 | USER  ME | End Call |  |
| 16 | ME  USS | DISCONNECT | [MO DISCONNECT] |
| 17 | ME  UICC | ENVELOPE: CALL DISCONNECTED 1.1.2A or ENVELOPE: CALL DISCONNECTED 1.1.2B or ENVELOPE: CALL DISCONNECTED 1.1.2C |  |
| 18 | USS  ME | SETUP | [ incoming call ] TI=0 |
| 19 | USER  ME | Accept Call Set Up |  |
| 20 | USS  ME | DISCONNECT | [MT DISCONNECT + CAUSE: normal call clearing ] |
| 21 | ME UICC | ENVELOPE: CALL DISCONNECTED 1.1.3A or ENVELOPE: CALL DISCONNECTED 1.1.3B |  |
| 22 | USS  ME | SETUP | TI=0 |
| 23 | USER  ME | Accept Call Set Up |  |
| 24 | USS | TX POWER to XX | [RADIO LINK FAILURE] |
| 25 | ME UICC | ENVELOPE: CALL DISCONNECTED 1.1.4A or 1.1.4B |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Call Disconnected

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 02 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.1

Logically:

Event list:	Call Disconnected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Cause:

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 02 | 82 | 02 | 83 | 81 | 1C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.2A

Logically:

Event list:	Call Disconnected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 02 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.2B

Logically:

Event list:	Call Disconnected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Cause:	normal call clearing

Coding:

| BER-TLV: | D6 | 0E | 19 | 01 | 02 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 02 | 60 | 90 |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.2C

Logically:

Event list:	Call Disconnected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Cause:	normal call clearing

Coding:

| BER-TLV: | D6 | 0E | 19 | 01 | 02 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 02 | E0 | 90 |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.3A

Logically:

Event list:	Call Disconnected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Cause:	normal call clearing

Coding:

| BER-TLV: | D6 | 0E | 19 | 01 | 02 | 82 | 02 | 83 | 81 | 1C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 02 | 60 | 90 |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.3B

Logically:

Event list:	Call Disconnected

Device identities

Source device:	Network

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Cause:	normal call clearing

Coding:

| BER-TLV: | D6 | 0E | 19 | 01 | 02 | 82 | 02 | 83 | 81 | 1C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 02 | E0 | 90 |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.4A

Logically:

Event list:	Call Disconnected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	1 (bit 8)

Cause:	radio link failure

Coding:

| BER-TLV: | D6 | 0C | 19 | 01 | 02 | 82 | 02 | 82 | 81 | 1C | 01 | 80 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 00 |  |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - CALL DISCONNECTED 1.1.4B

Logically:

Event list:	Call Disconnected

Device identities

Source device:	ME

Destination device:	UICC

Transaction identifier:

TI value:	0 (bit 5-7) - If A.1/150 is supported, this shall not be verified

TI flag:	0 (bit 8)

Cause:	radio link failure

Coding:

| BER-TLV: | D6 | 0C | 19 | 01 | 02 | 82 | 02 | 82 | 81 | 1C | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 9A | 00 |  |  |  |  |  |  |  |  |  |  |

27.22.7.3.1.5	Test requirement

The behaviour of the test is as defined in 'Expected Sequence 1.1'.

#### 27.22.7.4	Location Status Event

##### 27.22.7.4.1	Location Status Event (normal)

27.22.7.4.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.4.1.2	Conformance requirement

The ME shall support the EVENT: Location Status event as defined in:

-	TS 31.111 [15] clause 5.2, 7.5 and clause 6.4.16

and

-	UTRAN/GERAN for sequence 1.1;

-	E-UTRAN (WB-S1 mode or NB-S1 mode) for sequence 1.2;

-	NG-RAN for sequence 1.3.

27.22.7.4.1.3	Test purpose

To verify that the ME informs the UICC that an Event: MM_IDLE state has occurred using the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

To verify that the ME supporting E-UTRAN/EPC informs the UICC that an Event: EMM_IDLE state has occurred using the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

To verify that the ME supporting E-UTRAN/EPC correctly encodes the E-UTRAN Cell Id in the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

To verify that the ME supporting NG-RAN informs the UICC that an Event: 5GMM_IDLE state has occurred using the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

To verify that the ME supporting NG-RAN correctly encodes the 5G Cell Id in the ENVELOPE (EVENT DOWNLOAD - Location Status) command.

27.22.7.4.1.4	Method of test

27.22.7.4.1.4.1	Initial conditions

For sequence 1.1 the ME is connected to the USIM Simulator and the USS.

The elementary files are coded as the USIM Application Toolkit default.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01 ;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

Two cells are defined. Cell 1 has location area code 1 and cell 2 has location area code 2.

MS is in service on Cell 1.

For sequence 1.2 the ME is connected to the USIM Simulator and the E-USS/NB-SS.

The default E-UTRAN/EPC UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E-UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

For cell 1:

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Id  = 0001 (28 bits);

For cell 2:

-	Tracking Area Code (TAC) = 0002;

-	E-UTRAN Cell Id  = 0002 (28 bits).

The NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

For cell 1:

-	Tracking Area Code (TAC) = 0001;

-	NB-IoT Cell Id = 0001 (28 bits);

For cell 2:

-	Tracking Area Code (TAC) = 0002;

-	NB-IoT Cell Id = 0002 (28 bits).

For sequence 1.3 the ME is connected to the USIM Simulator and the NG-SS.

The default NG-RAN UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The NG-SS transmits on the BCCH, with the following network parameters:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

For cell 1:

-	Tracking Area Code (TAC) = 000001;

-	NG-RAN Cell Id = 0001 (36 bits);

For cell 2:

-	Tracking Area Code (TAC) = 000002;

-	NG-RAN Cell Id = 0002 (36 bits);

27.22.7.4.1.4.2	Procedure

Expected Sequence 1.3 (EVENT DOWNLOAD -LOCATION STATUS, NG-RAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME is registered to cell 1 and in 5GMM_IDLE |  |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 5a | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5b | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.3.1A | This step applies only if A.1/171 |
| 6 | NG-SS | Cell 1 is switched off |  |
| 7 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.3.1 |  |
| 8 | NG-SS | Cell 2 is switched on after Location Status "No service" has been received in step 7 |  |
| 9 | ME | ME performs cell reselection to cell 2 |  |
| 10 | ME NG-SS | ME performs 5GS registration with 5GS registration type IE set to “initial registration” or "mobility registration updating" | [NG-SS cell 2 accepts] |
| 11 | ME | ME reaches 5GMM_IDLE state |  |
| 12 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.3.2 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in sequence 1.1

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Same as Terminal Response: SET UP EVENT LIST 1.1.1 in sequence 1.1

EVENT DOWNLOAD - LOCATION STATUS 1.3.1

Logically:

Event list: 			Location status

Device identities

Source device: 		ME

Destination device: 	UICC

Location status: 			No service

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.3.1A

Logically:

Event list: 			Location status

Device identities

Source device: 		ME

Destination device: 	UICC

Location status: 			normal service

Location Information

MCC & MNC 		the mobile country and network code (00F110)

TAC 			000001

NG-SS cell id: 		0001 (36bits)

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 0B | 00 | F1 | 10 | 00 | 00 | 01 | 00 | 00 | 00 | 00 |
|  | 1F |  |  |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - LOCATION STATUS 1.3.2

Logically:

Event list: 			Location status

Device identities

Source device:	 	ME

Destination device: 	UICC

Location status: 			normal service

Location Information

MCC & MNC 		the mobile country and network code (00F110)

TAC 			000002

NG-SS cell id:	 	0002 (36bits)

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 03 | 02 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 0B | 00 | F1 | 10 | 00 | 00 | 02 | 00 | 00 | 00 | 00 |
|  | 2F |  |  |  |  |  |  |  |  |  |  |  |

27.22.7.4.1.4.2	Procedure

Expected Sequence 1.1(EVENT DOWNLOAD -LOCATION STATUS)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4a | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 4b | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.1A [applies for GERAN/UTRAN parameters] or ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.1B [applies for PCS1900 parameters] | This step applies only if A.1/171 |
| 5 | USS | Cell 1 is switched off |  |
| 6 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.1 |  |
| 7 | USS | Cell 2 is switched on after Location Status "No service" has been received in step 6 |  |
| 8 | ME | ME performs cell reselection to cell 2 |  |
| 9 | ME  USS | LOCATION UPDATING REQUEST or ROUTING AREA UPDATE REQUEST | The ME is CS and/or PS registered depending on its capabilities |
| 10 | USS  ME | LOCATION UPDATING ACCEPT or ROUTING AREA UPDATE ACCEPT |  |
| 11 | ME  USS | TMSI REALLOCATION COMPLETE or ROUTING AREA UPDATE COMPLETE |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.2A or ENVELOPE: EVENT DOWNLOAD - Location Status 1.1.2B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] [Note: The inclusion of the location information is optional: (If location status indicates normal status) |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Location status

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 03 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.1.1

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	No service

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.1.1A

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note 3 |  |  |

Note 1: Depending on the presence of the Extended Cell Identity Value the length is '13' or '15'

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

EVENT DOWNLOAD - LOCATION STATUS 1.1.1B

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D6 | 13 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 | 01 |  |  |  |

EVENT DOWNLOAD - LOCATION STATUS 1.1.2A

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0002)

Cell ID	Cell Identity Value (0002)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | Note 2 | 00 | F1 | 10 | 00 | 02 | 00 | 02 | Note 3 |  |  |

Note 1: Depending on the presence of the Extended Cell Identity Value the length is '13' or '15'

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

EVENT DOWNLOAD - LOCATION STATUS 1.1.2B

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0002)

Cell ID	Cell Identity Value (0002)

Coding:

| BER-TLV: | D6 | 13 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 07 | 00 | 11 | 10 | 00 | 02 | 00 | 02 |  |  |  |

Expected Sequence 1.2 (EVENT DOWNLOAD -LOCATION STATUS, E-UTRAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME is registered to cell one and in EMM_IDLE |  |
| 2 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 3 | ME  UICC | FETCH |  |
| 4 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 5a | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5b | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.2.1A | This step applies only if A.1/171 |
| 6 | E-USS/NB-SS | Cell 1 is switched off |  |
| 7 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.2.1 |  |
| 8 | E-USS/NB-SS | Cell 2 is switched on after Location Status "No service" has been received in step 7 |  |
| 9 | ME | ME performs cell reselection to cell 2 |  |
| 10 | ME  E-USS/NB-SS | ME performs EPS ATTACH or TRACKING AREA UPDATE procedure | [E-UTRAN/NB-IoT cell 2 accepts] |
| 11 | ME | ME reaches EMM_IDLE state |  |
| 12 | ME  UICC | ENVELOPE: EVENT DOWNLOAD - Location Status 1.2.2 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Same as PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 in sequence 1.1

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Same as Terminal Response: SET UP EVENT LIST 1.1.1 in sequence 1.1

EVENT DOWNLOAD - LOCATION STATUS 1.2.1

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	No service

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - LOCATION STATUS 1.2.1A

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

TAC	0001

E-UTRAN cell id:	0001 (28bits)

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 00 | F1 | 10 | 00 | 01 | 00 | 00 | 00 | 1F |  |

EVENT DOWNLOAD - LOCATION STATUS 1.2.2

Logically:

Event list:	Location status

Device identities

Source device:	ME

Destination device:	UICC

Location status:	normal service

Location Information

MCC & MNC	the mobile country and network code (00F110)

TAC	0002

E-UTRAN cell id:	0002 (28bits)

Coding:

| BER-TLV: | D6 | 15 | 19 | 01 | 03 | 82 | 02 | 82 | 81 | 1B | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 13 | 09 | 00 | F1 | 10 | 00 | 02 | 00 | 00 | 00 | 2F |  |

27.22.7.4.1.5	Test requirement

The behaviour of the test shall be as defined in expected sequences 1.1 and 1.2.

#### 27.22.7.5	User Activity Event

##### 27.22.7.5.1	User Activity Event (normal)

27.22.7.5.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.5.1.2	Conformance Requirement

The ME shall support the EVENT DOWNLOAD -USER ACTIVITY as defined in:

-	TS 31.111 [15] clause 5.2, clause 6.4.16, clause 6.8, clause 6.6.16, clause 6.11, clause 7.5, clause 8.6 and clause 8.25.

27.22.7.5.1.3	Test purpose

To verify that the ME performed correctly the procedure of USER ACTIVITY EVENT.

27.22.7.5.1.4	Method of Test

27.22.7.5.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

27.22.7.5.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD -USER ACTIVITY)

See ETSI TS 102 384 [26] in subclause 27.22.7.5.1.4.2, Expected Sequence 1.1.

27.22.7.5.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.6	Idle screen available event

##### 27.22.7.6.1	Idle Screen Available (normal)

27.22.7.6.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.6.1.2	Conformance requirement

The ME shall support the EVENT: IDLE SCREEN AVAILABLE event as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, and clause 8.25.

27.22.7.6.1.3	Test purpose

To verify that the ME informs the UICC that an Event: Idle Screen Available has occurred using the ENVELOPE (EVENT DOWNLOAD - IDLE SCREEN AVAILABLE) command.

27.22.7.6.1.4	Method of test

27.22.7.6.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure and be in updated idle mode on the USS.

27.22.7.6.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD - IDLE SCREEN AVAILABLE)

See ETSI TS 102 384 [26] in subclause 27.22.7.6.1.4.2, Expected Sequence 1.1.

27.22.7.6.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.7	Card reader status event

##### 27.22.7.7.1	Card Reader Status (normal)

27.22.7.7.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.7.1.2	Conformance requirement

The ME shall support the EVENT: Call Card Reader Status event as defined in:

-	TS 31.111 [15] clause 4.7, clause 4.9, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, clause 8.25, clause 8.33, annex F, annex G, clause 8.25 and clause 8.7.

27.22.7.7.1.3	Test purpose

To verify that the ME informs the UICC that an Event: Card Reader Status has changed using the ENVELOPE (EVENT DOWNLOAD - Card Reader Status) command.

The ME-Manufacturer can assign the card reader identifier from 0 to 7.

This test applies for MEs with only one additional card reader.

In this particular case the card reader identifier 1 is chosen.

27.22.7.7.1.4	Method of test

27.22.7.7.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

27.22.7.7.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD, Card reader status, Card reader 1, card reader attached, no card inserted)

See ETSI TS 102 384 [26] in subclause 27.22.7.7.1.4.2, Expected Sequence 1.1.

27.22.7.7.1.5	Test requirement

The behaviour of the test is as defined in expected Sequence 1.1.

##### 27.22.7.7.2	Card Reader Status(detachable card reader)

27.22.7.7.2.1	Definition and applicability

See clause 3.2.2.

27.22.7.7.2.2	Conformance requirement

The ME shall support the EVENT: Call Card Reader Status event as defined in:

-	TS 31.111 [15] clause 4.7, clause 4.9, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, clause 8.25, clause 8.33, annex F, annex G, clause 8.25 and clause 8.7.

27.22.7.7.2.3	Test purpose

To verify that the ME informs the UICC that an Event: Card Reader Status has changed using the ENVELOPE (EVENT DOWNLOAD - Card Reader Status) command.

The ME-Manufacturer can assign the card reader identifier from 0 to 7.

This test applies for MEs with only one additional card reader.

In this particular case the card reader identifier 1 is chosen as an example.

27.22.7.7.2.4	Method of test

27.22.7.7.2.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

27.22.7.7.2.4.2	Procedure

Expected Sequence 2.1 (EVENT DOWNLOAD, Detachable reader, Card reader 1, detachable card reader not attached, no card inserted)

See ETSI TS 102 384 [26] in subclause 27.22.7.7.2.4.2, Expected Sequence 2.1.

27.22.7.7.2.5	Test requirement

The behaviour of the test is as defined in expected Sequence 2.1.

#### 27.22.7.8	Language selection event

##### 27.22.7.8.1	Language selection event (normal)

27.22.7.8.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.8.1.2	Conformance requirement

The ME shall support the EVENT: LANGUAGE SELECTION event as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, and clause 8.25.

27.22.7.8.1.3	Test purpose

To verify that the ME informs the UICC that an Event: Language selection has occurred using the ENVELOPE (EVENT DOWNLOAD - LANGUAGE SELECTION ) command.

27.22.7.8.1.4	Method of test

27.22.7.8.1.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as USIM Application Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The current language shall have been set to English. Another language has to be supported, German is an example.

27.22.7.8.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD - LANGUAGE SELECTION)

See ETSI TS 102 384 [26] in subclause 27.22.7.8.1.4.2, Expected Sequence 1.1.

27.22.7.8.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.9	Browser termination event

##### 27.22.7.9.1	Browser termination (normal)

27.22.7.9.1.1	Definition and applicability

This test is only applicable to ME's that support the EVENT: browser termination event driven information.

27.22.7.9.1.2	Conformance requirement

The ME shall support the EVENT: Browser termination event as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, , clause 8.25, clause 8.51, annex F and clause 8.7.

27.22.7.9.1.3	Test purpose

To verify that the ME informs the UICC of an Event: Browser termination using the ENVELOPE (EVENT DOWNLOAD - Browser Termination) command.

This test applies for MEs which have a browser.

27.22.7.9.1.4	Method of test

27.22.7.9.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

A valid access to a Wap gateway is required. The default browser parameters (IP address, gateway/proxy identity, called number…) of the tested mobile shall be properly filled to access that gateway.

The Bearer Parameters defined in 27.22.4.26.1.4.1 shall be used.

27.22.7.9.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD - Browser termination)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 PENDING |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 | [EVENT: Browser termination Status] |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | [Successfully] |
| 5 | UserME | Launch the browser with the URL selected by the user |  |
| 6 | MEUSS | The ME attempts to launch the session with the default browser parameters and the URL selected by the user. |  |
| 7 | UserME | Stop the session and the browser. |  |
| 8 | ME UICC | ENVELOPE: BROWSER TERMINATION 1.1.1 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Browser termination

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 99 | 01 | 08 |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD BROWSER TERMINATION 1.1.1

Logically:

Event list

Event 1:	Browser termination

Device identities

Source device:	ME

Destination device:	UICC

Browser termination cause:	User termination

Coding:

| BER-TLV: | D6 | 0A | 99 | 01 | 08 | 82 | 02 | 82 | 81 | B4 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.7.9.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.10	Data available event

##### 27.22.7.10.1	Definition and applicability

See clause 3.2.2.

##### 27.22.7.10.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

Additionally the ME shall support ENVELOPE (EVENT DOWNLOAD - Data available).

If the ME supports option A.1/182, and if the UICC supports the UICC suspension mechanism (SUSPEND UICC command), the ME may suspend the UICC after entering the PSM. In this case, the ME shall successfully resume the UICC before it can leave the PSM. Furthermore, the terminal shall maintain the logical status as before the suspension and it shall resume the UICC for any event for which it had previously registered: this includes events registered with SET UP EVENT LIST  proactive command, as specified in ETSI TS 102 221 clause 14.5.6

If the ME supports option A.1/182 and/or A.1/181, if the UE is in PSM and in case the ME wants to deactivate the UICC, it shall wait until the current proactive UICC session, if any, is terminated.

If the ME supports option A.1/183, and in case the UICC supports the UICC suspension mechanism (SUSPEND UICC command), the ME may suspend the UICC during the extended idle mode DRX cycle. In this case, the ME shall resume the UICC successfully before the end of the extended idle mode DRX cycle or before any other transmission to the network. Furthermore, the terminal shall maintain the logical status as before the suspension and it shall resume the UICC for any event for which it had previously registered: this includes events registered with SET UP EVENT LIST  proactive command, as specified in ETSI TS 102 221 clause 14.5.6.

- TS 102 221 [13]

- TS 31.102 [14]

##### 27.22.7.10.3	Test purpose

To verify that the ME shall send an ENVELOPE (EVENT DOWNLOAD - Data available) to the UICC after the ME receives a packet of data from the server by the BIP channel previously opened.

To verify that the ME shall send an ENVELOPE (EVENT DOWNLOAD - Data available) to the UICC when the ME resumes the UICC and receives a packet of data from the server by the BIP channel previously opened.

To verify that the ME is performing BIP session correctly when the ME is configured and using PSM or eDRX.

##### 27.22.7.10.4	Method of test

27.22.7.10.4.1	Initial conditions

The ME is connected to the USIM Simulator and only connected to the USS if the USS is mentioned in the sequence table.. The elementary files are coded as Toolkit default for sequence 1.1.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure. The UICC must have sent the SET UP EVENT LIST to the ME to supply a set of events (event Data available).

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The PROACTIVE COMMAND: SEND DATA 1.1.1 shall be performed successfully to detect the ME's port number, which has to be addressed by the network simulator when data has to be transmitted to the card. The corresponding Terminal Response shall be TERMINAL RESPONSE: SEND DATA 1.1.1.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

For sequence 1.2, 1.3, 1.4 and 1.5 the default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:			TestGp.rs

User login:					UserLog

User password:				UserPwd

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.6.4.1

Data destination address	:		Sames Data Destination Address as defined in 27.22.4.27.6.4.1.

For sequence 1.3 UICC suspension mechanism is indicated as supported by the UICC in the UICC Maximum Power Consumption file (EFUMPC) and the PIN of the USIM is enabled.

For sequence 1.4, UICC suspension mechanism is not indicated as supported by the UICC in the UICC Maximum Power Consumption file (EFUMPC) and the PIN of the USIM is disabled.

Prior to sequence 1.3 and 1.4, the ME was prepared to use PSM, has been powered on, attached to the E-USS/NB-SS where the PSM use was accepted by the network. Immediately after the Active Time (T3324) is started and the UE has performed the PROFILE DOWNLOAD procedure, the test sequence shall be executed.

For sequence 1.5 the UICC suspension mechanism is indicated as supported by the UICC in the UICC Maximum Power Consumption file (EFUMPC), the ME is authorized to modify the polling interval and/or disable the UICC interface during extended DRX cycle in EFAD (Administrative Data) and the PIN of the USIM is enabled.

Prior to sequence 1.5, the ME was prepared to use eDRX, has been powered on, attached to the E-USS/NB-SS where the eDRX use was accepted by the network and performed the PROFILE DOWNLOAD procedure.

27.22.7.10.4.2	Procedure

Expected sequence 1.1 (EVENT DOWNLOAD - Data available)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 | [Command performed successfully] |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 6 | USS ME | PDP context activation accept |  |
| 7 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B |  |
| 8 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.1.1 |  |
| 9 | ME UICC | FETCH |  |
| 10 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.1.1 |  |
| 11 | ME USS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 12 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.1.1 | [Command performed successfully] |
| 13 | USS ME | Data sent through the BIP channel using the ME's port number, which was retrieved in step 11 |  |
| 14 | ME UICC | ENVELOPE 1.1.1 (Event-Data Available) |  |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog  (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Data available 1.1.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	8 Bytes available in Rx buffer

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | 08 |  |  |  |  |  |  |  |  |

Expected sequence 1.2 (EVENT DOWNLOAD - Data available, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 PENDING |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.2.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.2.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 as PDN type.] |
| 10 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 11 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 12 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.2.1 | [Command performed successfully] |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.2.1 |  |
| 16 | ME E-USS/NB-SS | Transfer of 8 Bytes of data to the USS through channel 1 | [To retrieve ME's port number] |
| 17 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.2.1 | [Command performed successfully] |
| 18 | E-USS/NB-SS ME | Data sent through the BIP channel using the ME's port number, which was retrieved in step 16 |  |
| 19 | ME UICC | ENVELOPE 1.2.1 (Event-Data Available) | [Command performed successfully] |

PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Event list	Data available

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 09 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: OPEN CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Alpha Identifier:	empty

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		Test12.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 44 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 |
|  | 05 | 78 | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 31 | 32 | 02 |
|  | 72 | 73 | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 |
|  | 02 | AD | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

PROACTIVE COMMAND: SEND DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data

Channel Data:	00 01 .. 07 (8 Bytes of data)

Coding:

| BER-TLV: | D0 | 13 | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 81 | 21 | B6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 08 | 00 | 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  |  |

TERMINAL RESPONSE: SEND DATA 1.2.1

Logically:

Command details

Command number:	1

Command type:	SEND DATA

Command qualifier:	Send Immediately

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel data length:	More than 255 bytes of space available in the Tx buffer

Coding:

| BER-TLV: | 81 | 03 | 01 | 43 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B7 | 01 | FF |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Data available 1.2.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	8 Bytes available in Rx buffer

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | 08 |  |  |  |  |  |  |  |  |

Expected sequence 1.3 (EVENT DOWNLOAD - Data available, PSM by SUSPEND UICC for E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 PENDING |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.2.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.2.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 as PDN type.] |
| 10 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 11 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 12 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.2.1 | [Command performed successfully] |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.2.1 |  |
| 16 | ME E-USS/NB-SS | Transfer of 8 Bytes of data to the E-USS/NB-SS through channel 1 | [To retrieve ME's port number] |
| 17 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.2.1 | [Command performed successfully] |
| 18 | E-USS/NB-SS ME | Data sent through the BIP channel using the ME's port number, which was retrieved in step 16 |  |
| 19 | ME UICC | ENVELOPE 1.3.1 (Event-Data Available) | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.3.1 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.3.1 |  |
| 23 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.3.1 | [Command performed successfully] |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.3.1 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.3.1 |  |
| 27 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.3.1 | [Command performed successfully] Before performing this step, and during the above session, the ME should not try to suspend the UICC. |
| 28 | ME UICC | Suspend the UICC | ME is in the PSM. |
| 29 | User ME | Wait until the ME resumes the UICC before it leaves the PSM |  |
| 30 |  | Perform steps 5 -17 |  |
| 31 | E-USS/NB-SS ME | Data sent through the BIP channel using the ME's port number, which was retrieved to transfer the data to the E-USS/NB-SS |  |
| 32 | ME UICC | ENVELOPE 1.2.1 (Event-Data Available) | [Command performed successfully] |

ENVELOPE: EVENT DOWNLOAD - Data available 1.3.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	200 Bytes available in Rx buffer

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | C8 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.3.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.3.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	00 01 02 .. C7 (200 Bytes of data)

Channel data length:	C8

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | 00 | 01 | 02 | .. | C7 | B7 | 01 | C8 |  |

PROACTIVE COMMAND: CLOSE CHANNEL 1.3.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 81 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: CLOSE CHANNEL 1.3.1

Logically:

Command details

Command number:	1

Command type:	CLOSE CHANNEL

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 41 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected sequence 1.4 (EVENT DOWNLOAD - Data available, PSM for E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 PENDING |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.2.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.2.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 as PDN type.] |
| 10 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 11 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 12 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.2.1 | [Command performed successfully] |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.2.1 |  |
| 16 | ME E-USS/NB-SS | Transfer of 8 Bytes of data to the E-USS/NB-SS through channel 1 | [To retrieve ME's port number] |
| 17 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.2.1 | [Command performed successfully] |
| 18 | E-USS/NB-SS ME | Data sent through the BIP channel using the ME's port number, which was retrieved in step 16 |  |
| 19 | ME UICC | ENVELOPE 1.3.1 (Event-Data Available) | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.3.1 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.3.1 |  |
| 23 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.3.1 | [Command performed successfully] |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.3.1 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.3.1 |  |
| 27 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.3.1 | [Command performed successfully] |
| 28 | ME UICC | Deactivate the UICC | ME is in the PSM. |
| 29 | User ME | Wait until the ME activates the UICC before it leaves the PSM |  |
| 30 |  | Perform steps 1 -27 |  |

Expected sequence 1.5 (EVENT DOWNLOAD - Data available, eDRX by SUSPEND UICC for E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 PENDING |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 |  |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.2.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.2.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 as PDN type.] |
| 10 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 11 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 12 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.2.1 | [Command performed successfully] |
| 13 | UICC ME | PROACTIVE COMMAND PENDING: SEND DATA 1.2.1 |  |
| 14 | ME UICC | FETCH |  |
| 15 | UICC ME | PROACTIVE COMMAND: SEND DATA (immediate) 1.2.1 |  |
| 16 | ME E-USS/NB-SS | Transfer of 8 Bytes of data to the E-USS/NB-SS through channel 1 | [To retrieve ME's port number] |
| 17 | ME UICC | TERMINAL RESPONSE: SEND DATA (immediate) 1.2.1 | [Command performed successfully] |
| 18 | E-USS/NB-SS ME | Data sent through the BIP channel using the ME's port number, which was retrieved in step 16 |  |
| 19 | ME UICC | ENVELOPE 1.3.1 (Event-Data Available) | [Command performed successfully] |
| 20 | UICC ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.3.1 |  |
| 21 | ME UICC | FETCH |  |
| 22 | UICC ME | PROACTIVE COMMAND: RECEIVE DATA 1.3.1 |  |
| 23 | ME UICC | TERMINAL RESPONSE: RECEIVE DATA 1.3.1 | [Command performed successfully] |
| 24 | UICC ME | PROACTIVE COMMAND PENDING: CLOSE CHANNEL 1.3.1 |  |
| 25 | ME UICC | FETCH |  |
| 26 | UICC ME | PROACTIVE COMMAND: CLOSE CHANNEL 1.3.1 |  |
| 27 | ME UICC | TERMINAL RESPONSE CLOSE CHANNEL 1.3.1 | [Command performed successfully] Before performing this step, and during the above session, the ME should not try to suspend the UICC. |
| 28 | ME UICC | Suspend the UICC | The ME is in extended idle mode DRX cycle. |
| 29 | User ME | Wait until the ME resumes the UICC |  |
| 30 |  | Perform steps 5 -17 |  |
| 31 | E-USS/NB-SS ME | Data sent through the BIP channel using the ME's port number, which was retrieved to transfer the data to the E-USS/NB-SS |  |
| 32 | ME UICC | ENVELOPE 1.2.1 (Event-Data Available) | [Command performed successfully] |

27.22.7.10.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1 and 1.2.

#### 27.22.7.11	Channel Status event

##### 27.22.7.11.1	Definition and applicability

See clause 3.2.2.

##### 27.22.7.11.2	Conformance requirements

The ME shall support the class "e" commands as defined in:

-	TS 31.111 [15].

Additionally the ME shall support ENVELOPE (EVENT DOWNLOAD - Channel Status).

##### 27.22.7.11.3	Test purpose

To verify that the ME shall send an ENVELOPE (EVENT DOWNLOAD - Channel Status) to the UICC after the link dropped between the NETWORK and the ME.

##### 27.22.7.11.4	Method of test

27.22.7.11.4.1	Initial conditions

The ME is connected to the USIM Simulator and the System Simulator. The elementary files are coded as Toolkit default for sequence 1.1.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

For MEs supporting BIP related to GPRS in UDP (i.e condition C121 in table B.1), The PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 shall be executed to open a channel successfully at the beginning of the test. The corresponding Terminal Response shall be TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B.

The channel identifier value used for these tests is set to 1 as an example.

This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The following Bearer Parameters used are those defined in the default Test PDP context3, for test cases using packet services:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.2.4.1

GPRS Parameters:				Same GPRS Parameters as defined in 27.22.4.27.2.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.2.4.1

Data destination address	:		Same Data Destination Address as defined in 27.22.4.27.2.4.1.

For sequence 1.2 the default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Bearer Parameters:				Same Bearer Parameters as defined in 27.22.4.27.6.4.1

UICC/ME interface transport level:	Same UICC/ME transport interface level as defined in 27.22.4.27.6.4.1

Data destination address	:		Sames Data Destination Address as defined in 27.22.4.27.6.4.1.

27.22.7.11.4.2	Procedure

Expected sequence 1.1 (EVENT DOWNLOAD - Channel Status on a link dropped)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 | [EVENT: channel status] |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | [command performed successfully] |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.1.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME USS | PDP context activation request | [The UE may request IPv4 or IPv4v6 address as PDP type.] |
| 10 | USS ME | PDP context activation accept |  |
| 11 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B | [Command performed successfully] |
| 12 | USS ME | Link dropped |  |
| 13 | ME UICC | ENVELOPE 1.1.1 (Event-Channel Status) |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Channel Status

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 99 | 01 | 0A |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Network access name:	TestGp.rs

Text String:	UserLog (User login)

Text String:	UserPwd (User password)

UICC/ME interface transport level

Transport format:	UDP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 03 | 04 | 1F | 02 | 39 | 02 | 03 | E8 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 01 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS

Bearer parameter:

Precedence Class:	00

Delay Class:	04

Reliability Class:	03

Peak throughput class:	04

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1000

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 00 | 04 | 03 | 04 | 1F |
|  | 02 | 39 | 02 | 03 | E8 |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Channel Status 1.1.1

Logically:

Event list

Event:	Channel Status

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1, link dropped

Coding:

| BER-TLV: | D6 | 0B | 99 | 01 | 0A | 82 | 02 | 82 | 81 | B8 | 02 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 |  |  |  |  |  |  |  |  |  |  |  |

Expected sequence 1.2 (EVENT DOWNLOAD - Channel Status on a link dropped, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.2.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1 | [EVENT: channel status] |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1 | [command performed successfully] |
| 5 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.2.1 | See initial conditions |
| 6 | ME UICC | FETCH |  |
| 7 | UICC ME | PROACTIVE COMMAND: OPEN CHANNEL 1.2.1 |  |
| 8 | ME USER | The ME may display channel opening information |  |
| 9 | ME E-USS/NB-SS | PDN CONNECTIVITY REQUEST | [The UE may request IPv4 or IPv4v6 as PDN type.] |
| 10 | E-USS/NB-SS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST |  |
| 11 | ME E-USS/NB-SS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 12 | ME UICC | TERMINAL RESPONSE: OPEN CHANNEL 1.2.1A or TERMINAL RESPONSE: OPEN CHANNEL 1.2.1B | [Command performed successfully] |
| 13 | E-USS/NB-SS ME | Link dropped |  |
| 14 | ME UICC | ENVELOPE 1.2.1 (Event-Channel Status) | [Command performed successfully] |

PROACTIVE COMMAND: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	ME

Event list	Data available

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 0A |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.2.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

PROACTIVE COMMAND: OPEN CHANNEL 1.2.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		TestGp.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.2.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.2.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Channel Status 1.2.1

Logically:

Event list

Event:	Channel Status

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1, link dropped

Coding:

| BER-TLV: | D6 | 0B | 99 | 01 | 0A | 82 | 02 | 82 | 81 | B8 | 02 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 05 |  |  |  |  |  |  |  |  |  |  |  |

27.22.7.11.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1 and 1.2.

#### 27.22.7.12	Access Technology Change event

27.22.7.12.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.12.1.2	Conformance requirement

The ME shall support the EVENT: Access Technology Change event E-UTRAN as defined in:

-	TS 31.111 [15] clause 4.7, 4.12, 7.5.12 and clause 8.61.

27.22.7.12.1.3	Test purpose

If the Access Technology Change event is part of the current event list (as set up by the last SET UP EVENT LIST command), then, when the terminal detects a change in its current access technology, verify that the terminal shall inform the UICC that this has occurred, by using the ENVELOPE (EVENT DOWNLOAD - Access Technology Change).

If the event is set up with support for multiple access technologies, the UICC shall be informed if any of the access technologies changes.

27.22.7.12.1.4	Method of test

27.22.7.12.1.4.1	Initial conditions

For test sequence 1.1,

The ME is connected to the USIM Simulator and the UMTS System Simulator.

The default E-UTRAN/EPC UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E- UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

For test sequence 1.2:

The ME is connected to the USIM Simulator and the GSM System Simulator.

The GSM parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001

The E- UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The NB-SS parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

For test sequence 1.4,

The ME is connected to the USIM Simulator and the UMTS System Simulator.

The default E-UTRAN/EPC and NG-RAN UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

-	NG-RAN Cell Identifier  = 0001 (36 bits);

The E- UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

27.22.7.12.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD – Access Technology Change, single access technology)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4a | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 4b | ME UICC | ENVELOPE: EVENT DOWNLOAD - Access technology change Event | This step applies only if A.1/171 |
| 5 | E-USS | ME detects a change in its current access technology | E-UTRA cell is enabled and UTRA cell is disabled |
| 6 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.1 | Access Technology = E-UTRAN |
| 7 | E-USS | ME detects a change in its current access technology | E-UTRA cell is disabled and UTRA cell is enabled |
| 8 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.2 | Access Technology = UTRAN |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Access Technology Change (single access technology)

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 0B |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.1.1

Logically:

Event list:	Access Technology Change (single access technology)

Device identities

Source device:	ME

Destination device:	UICC

Access Technology:	E-UTRAN

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 0B | 82 | 02 | 82 | 81 | 3F | 01 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.1.2

Logically:

Event list:	Access Technology Change (single access technology)

Device identities

Source device:	ME

Destination device:	UICC

Access Technology:	UTRAN

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 0B | 82 | 02 | 82 | 81 | 3F | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (EVENT DOWNLOAD – Access Technology Change, multiple access technologies)

TBD

Expected Sequence 1.3 (EVENT DOWNLOAD – Access Technology Change, single access technology – WB-S1 (Cat M1)/NB-S1)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Access technology change Event 1.1.3 |  |
| 6 | E-USS/NB-SS | ME detects a change in its current access technology | E-UTRA/NB-IoT cell is enabled and GSM cell is disabled |
| 7 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.1 | Access Technology = E-UTRAN |
| 8 | E-USS/NB-SS | ME detects a change in its current access technology | E-UTRA/NB-IoT cell is disabled and GSM cell is enabled |
| 9 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.3 | Access Technology = GSM |

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.1.3

Logically:

Event list:	Access Technology Change (single access technology)

Device identities

Source device:	ME

Destination device:	UICC

Access Technology:	GSM

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 0B | 82 | 02 | 82 | 81 | 3F | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.4 (EVENT DOWNLOAD – Access Technology Change, single access technology, NG-RAN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4a | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 4b | ME UICC | ENVELOPE: EVENT DOWNLOAD - Access technology change Event | This step applies only if A.1/171 |
| 5 | 5GNR-SS | ME detects a change in its current access technology | NG-RAN cell is enabled and E-UTRA cell is disabled |
| 6 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.4.1 | Access Technology = 3GPP NR |
| 5 | E-USS | ME detects a change in its current access technology | E-UTRA cell is enabled and NG-RAN cell is disabled |
| 6 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Access technology change Event 1.1.1 | Access Technology = E-UTRAN |

ENVELOPE: EVENT DOWNLOAD – Access Technology Change 1.4.1

Logically:

Event list:	Access Technology Change (single access technology)

Device identities

Source device:	ME

Destination device:	UICC

Access Technology:	3GPP NR

Coding:

| BER-TLV: | D6 | 0A | 19 | 01 | 0B | 82 | 02 | 82 | 81 | 3F | 01 | 0A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

#### 27.22.7.13	Display parameter changed event

TBD

#### 27.22.7.14	Local Connection event

TBD

#### 27.22.7.15	Network search mode change event

##### 27.22.7.15.1	Definition and applicability

See clause 3.2.2.

##### 27.22.7.15.2	Conformance requirements

The ME shall support the network search mode mechanism, as described in TS 31.111 [15] clause 4.13.

##### 27.22.7.15.3	Test purpose

To verify that the ME sends an ENVELOPE (EVENT DOWNLOAD – Network search mode change) to the UICC when network search mode is changed in ME.

##### 27.22.7.15.4	Method of test

27.22.7.15.4.1	Initial conditions

The ME is connected to the USIM Simulator. The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME is configured in automatic network search mode.

27.22.7.15.4.2	Procedure

Expected sequence 1.1 (EVENT DOWNLOAD – Network search mode change)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 | [EVENT: network search mode] |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | [command performed successfully] |
| 5 | User | The user sets the ME to manual network selection mode |  |
| 6 | ME UICC | ENVELOPE 1.1.1 (Event - Network search mode change) | [changed to manual] |
| 7 | User | The user sets the ME to automatic network selection mode |  |
| 8 | ME UICC | ENVELOPE 1.1.2 (Event - Network search mode change) | [changed to automatic] |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Network search mode change

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 99 | 01 | 0E |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD – Network search mode change 1.1.1

Logically:

Event list

Event:	Network search mode change

Device identities

Source device:	ME

Destination device:	UICC

Network search mode

Network search mode:	manual

Coding:

| BER-TLV: | D6 | 0A | 99 | 01 | 0E | 82 | 02 | 82 | 81 | E5 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD – Network search mode change 1.1.2

Logically:

Event list

Event:	Network search mode change

Device identities

Source device:	ME

Destination device:	UICC

Network search mode

Network search mode:	automatic

Coding:

| BER-TLV: | D6 | 0A | 99 | 01 | 0E | 82 | 02 | 82 | 81 | E5 | 01 | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

27.22.7.15.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.16	Browsing status event

TBD

#### 27.22.7.17	Network Rejection Event

27.22.7.17.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.17.1.2	Conformance requirement

The ME shall support the EVENT: Network Rejection event E-UTRAN as defined in:

-	TS 31.111 [15] clause 4.7, 5.2, 7.5.2, 8.62 and clause 8.99.

The ME shall support the EVENT: Registration Rejection event for NG-RAN as defined in:

-	TS 31.111 [15] clause 4.7, 5.2, 7.5.2 and 8.62.

27.22.7.17.1.3	Test purpose

For sequences 1.1 and 1.2:

-	To verify that the ME informs the UICC with the Event Network Rejection about the Network Rejection.

-	To verify that the Rejection Cause Code sent to the UICC is the value from the EMM cause information element received from the E-UTRAN.

-	To verify that the correct Access Technology is indicated ENVELOPE: EVENT DOWNLOAD – Network Rejection after the unsuccessful attempt to access the E-UTRAN.

-	To verify that the correct Update/Attach Type is indicated ENVELOPE: EVENT DOWNLOAD – Network Rejection.

For sequence 1.3 and 1.4:

-	To verify that the Rejection Cause Code sent to the UICC is the value from the 5GMM cause information element received from the NG_RAN.

-	To verify that the correct Access Technology is indicated ENVELOPE: EVENT DOWNLOAD – Registration Rejection after the unsuccessful attempt to access the NG-RAN.

-	To verify that the correct Update/Attach/Registration Type is indicated ENVELOPE: EVENT DOWNLOAD – Network Rejection.

27.22.7.17.1.4	Method of test

27.22.7.17.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NG-SS.

The default E-UTRAN/EPC or NG-RAN UICC is used.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E-UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

The NG-RAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 000001;

27.22.7.17.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD – Network Rejection, ATTACH REJECT)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | E-USS/NB-SS | No E-UTRAN/NB-IoT available |  |
| 2 | USER  ME | Switch on the terminal |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 7 | E-USS/NB-SS | The E-UTRAN/NB-IoT cell is switched on |  |
| 8 | USERME | The terminal is made to start a registration attempt to the E-USS/NB-SS |  |
| 9 | MEE-USS/NB-SS | The terminal requests RRC CONNECTION and therefore starts the EPS Attach procedure |  |
| 10 | E-USS/NB-SSME | The E-USS/NB-SS sends EMM ATTACH REJECT with cause "PLMN not allowed" |  |
| 11 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.1.1 or 1.1.2 |  |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Network Rejection

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 12 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.1.1

Logically:

Event list:	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	EPS Attach

Rejection Cause Code:	PLMN not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 09 | 75 | 01 |
|  | 0B |  |  |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.1.2

Logically:

Event list:	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	Combined EPS/IMSI Attach

Rejection Cause Code:	PLMN not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 0A | 75 | 01 |
|  | 0B |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.2 (EVENT DOWNLOAD – Network Rejection, TRACKING AREA UPDATE REJECT)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME is registered to the E-USS/NB-SS and in EMM_IDLE | The E-USS/NB-SS transmits on cell 1 : MCC: 001 MNC: 01 TAC: 0003 |
| 2 | E-USS/NB-SS | Cell 1 is switched off |  |
| 3 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | FETCH |  |
| 5 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 7 | E-USS/NB-SS | The E-UTRAN/NB-IoT cell 2 is switched on | The E-USS/NB-SS transmits on cell 2 : MCC: 001 MNC: 01 TAC: 0001 |
| 8 | ME | The terminal is made to start a re-registration attempt to the E-USS/NB-SS |  |
| 9 | MEE-USS/NB-SS | The terminal send TRACKING AREA UPDATE REQUEST |  |
| 10 | E-USS/NB-SS ME | The E-USS/NB-SS sends TRACKING AREA UPDATE REJECT with cause "TRACKING AREA not allowed" |  |
| 11 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.2.1 or 1.2.2 |  |

EVENT DOWNLOAD –Network Rejection 1.2.1

Logically:

Event list:	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	TA Updating

Rejection Cause Code:	Tracking Area not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 0B | 75 | 01 |
|  | 0C |  |  |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD –Network Rejection 1.2.2

Logically:

Event list:	Network Rejection

Device identities

Source device:	Network

Destination device:	UICC

Tracking Area Identification

MCC:	001

MNC:	01

TAC:	0001

Access Technology:	E-UTRAN

Update/Attach Type:	Combined TA/LA updating

Rejection Cause Code:	Tracking Area not allowed

Coding:

| BER-TLV: | D6 | 17 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 05 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 01 | 3F | 01 | 08 | 74 | 01 | 0C | 75 | 01 |
|  | 0C |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.3 (EVENT DOWNLOAD – Network Rejection, REGISTRATION REJECT – Initial Registration)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | NG-SS | No NG-RAN cell available |  |
| 2 | USER ME | Switch on the terminal |  |
| 3 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | FETCH |  |
| 5 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 7 | NG-SS | The NG-RAN cell is switched on |  |
| 8 | USER ME | The terminal is made to start a Registration attempt to the NG-SS |  |
| 9 | ME NG-SS | The terminal requests RRC CONNECTION and starts the 5GMM REGISTRATION procedure for “Initial Registration” |  |
| 10 | NG-SSME | The NG-SS sends REGISTRATION REJECT with cause "PLMN not allowed" |  |
| 11 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.3.1 |  |

ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.3.1

Logically:

Event list:		Network Rejection

Device identities

Source device:		Network

Destination device:		UICC

Tracking Area Identification

MCC:		001

MNC:		01

TAC:		000001

Access Technology:		3GPP NR

Update/Attach/Registration Type:	Initial Registration

Rejection Cause Code:		PLMN not allowed

Coding:

| BER-TLV: | D6 | 18 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 06 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 00 | 01 | 3F | 01 | 0A | 74 | 01 | 0F | 75 |
|  | 01 | 0B |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.4 (EVENT DOWNLOAD – Network Rejection, REGISTRATION REJECT – Mobility Registration updating)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | Switch on the terminal |  |
| 2 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 3 | ME UICC | FETCH |  |
| 4 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 5 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 6 | ME | The ME is registered to the NG-SS and in 5GMM_IDLE | The NG-SS transmits on cell 1: MCC: 001 MNC: 01 TAC: 000003 |
| 7 | NG-SS | NG-RAN cell 1 is switched off |  |
| 8 | NG-SS | NG-RAN cell 2 is switched on | The NG-SS transmits on cell 2: MCC: 001 MNC: 01 TAC: 000001 |
| 9 | ME | The terminal is made to start a re-registration attempt to the NG-SS |  |
| 10 | ME NG-SS | The terminal sends 5GMM REGISTRATION REQUEST for “Mobility Registration updating” |  |
| 11 | NG-SS ME | The NG-SS sends REGISTRATION REJECT with cause "TRACKING AREA not allowed" |  |
| 12 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Network Rejection 1.4.1 |  |

EVENT DOWNLOAD –Network Rejection 1.4.1

Logically:

Event list: 		Network Rejection

Device identities

Source device:		Network

Destination device:		UICC

Tracking Area Identification

MCC:		001

MNC:		01

TAC:		000001

Access Technology:		3GPP NR

Update/Attach/Registration Type:	Mobility Registration updating

Rejection Cause Code:		Tracking Area not allowed

Coding:

| BER-TLV: | D6 | 18 | 19 | 01 | 12 | 82 | 02 | 83 | 81 | 7D | 06 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F1 | 10 | 00 | 00 | 01 | 3F | 01 | 0A | 74 | 01 | 10 | 75 |
|  | 01 | 0C |  |  |  |  |  |  |  |  |  |  |

27.22.7.17.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 and 1.2.

#### 27.22.7.18	CSG Cell Selection event

##### 27.22.7.18.1	CSG Cell Selection (normal)

27.22.7.18.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.18.1.2	Conformance requirement

The ME shall support the EVENT: CSG Cell selection as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 6.8, clause 7.5, clause 8.25, 8.101, 8.102, 8.103.

27.22.7.18.1.3	Test purpose

To verify that the ME informs the UICC that an Event: CSG Cell selection has occurred using the ENVELOPE (EVENT DOWNLOAD - CSG Cell selection) command when the ME detects a change in its current CSG cell selection status.

27.22.7.18.1.4	Method of test

27.22.7.18.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS.

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E-USS transmits on three cells:

Network parameters of cell 1 :

-	TAI (MCC/MNC/TAC):	246/081/0001.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	01

-	Broadcast information:	Cell 3 is included in the neighbour list information.

Network parameters of cell 2:

-	TAI (MCC/MNC/TAC):	246/081/0002.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	02

-	Home (e)NB Name	HOME 02

Network parameters of cell 3:

-	TAI (MCC/MNC/TAC):	246/081/0003.

-	Access control:	unrestricted.

-	csg-Indication:	FALSE

Network parameters of cell 4 :

-	TAI (MCC/MNC/TAC):	246/081/0004.

-	Access control:	unrestricted.

-	csg-Indication:	TRUE

-	csg-Identity:	04

-	Broadcast information:	Cell 3 is included in the neighbour list information.

-	Home (e)NB Name	HOME 04

Cell 1, Cell 2 and Cell 4 are initially disabled. Cell 3 is enabled.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

EFUST (USIM Service Table)

EFUST shall be configured as defined in 27.22.2B.1 with the exception that Service 86 "Allowed CSG Lists and corresponding indications" is available.

EFACSGL (Allowed CSG Lists)

Logically:

1st CSG list

PLMN:	246 081 (MCC MNC)

1st CSG list	1st CSG Type indication		01

1st CSG list	1st CSG HNB Name indication	01

1st CSG list	1st CSG CSG ID:			01 (27bit)

2nd CSG list	2nd CSG Type indication		01

2nd CSG list	2nd CSG HNB Name indication	01

2nd CSG list	2nd CSG CSG ID:			04 (27bit)

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | A0 | 15 | 80 | 03 | 42 | 16 | 80 | 81 | 06 | 01 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 01 | 00 | 00 | 00 | 3F | 81 | 06 | 01 | 01 | 00 |
|  | B21 | B22 | B23 |  |  |  |  |  |  |  |
|  | 00 | 00 | 9F |  |  |  |  |  |  |  |

All other records are empty.

EFCSGT (CSG Type)

Record 1:

Logically:	  Group ONE

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 89 | 13 | 80 | 00 | 47 | 00 | 72 | 00 | 6F | 00 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 75 | 00 | 70 | 00 | 20 | 00 | 4F | 00 | 4E | 00 |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 45 | FF | FF | FF | FF | FF | FF | FF | FF | FF |

EFHNBN (Home (e)NodeB Name)

Record 1:

Logically:	Home ONE

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 11 | 80 | 00 | 48 | 00 | 6F | 00 | 6D | 00 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 65 | 00 | 20 | 00 | 4F | 00 | 4E | 00 | 45 | FF |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | FF | FF | FF | FF | FF | FF | FF | FF | FF | FF |

27.22.7.18.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD - CSG Cell Selection event)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME | The ME is registered to cell 3 and in EMM_IDLE | Cell 3 = macro cell |
| 2 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 3 | ME  UICC | FETCH |  |
| 4 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 5 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 6 | E-USS | Cell 2 is enabled |  |
| 7 | User ME | A manual CSG cell selection is performed. CSG ID=02 is selected. |  |
| 8 | E-USSME | AttachReject with rejection cause #25 (not authorized for this CSG) | No ENVELOPE command is sent. |
| 9 | E-USS | Cell 2 is disabled Cell 1 is enabled |  |
| 10 | UserME | A manual CSG cell selection is performed. CSG ID=01 is selected. |  |
| 11 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – CSG Cell selection 1.1.1A OR ENVELOPE: EVENT DOWNLOAD – CSG Cell selection 1.1.1B | Camping on CSG cell, CSG ID=01 |
| 12 | E-USS | Cell 1 is disabled |  |
| 13 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – CSG Cell selection 1.1.2 | Leaving CSG cell with CSG ID=01. Not camped on a CSG cell. |
| 14 | E-USS | Cell 4 is enabled |  |
| 15 | User ME | A manual CSG cell selection is performed. CSG ID=04 is selected. |  |
| 16 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – CSG Cell selection 1.1.3A OR ENVELOPE: EVENT DOWNLOAD – CSG Cell selection 1.1.3B | Camping on CSG cell, CSG ID=04 |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	'15' CSG Cell selection Event

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 15 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD – CSG CELL SELECTION 1.1.1A

Logically:

Event list

Event 1:	CSG Cell selection

Device identities

Source device:	Network

Destination device:	UICC

Access Technology

Technology :	E-UTRAN

CSG Cell selection status:	Byte 1 = '01' (camped on a CSG or Hybrid cell of the Operator CSG list or Allowed CSG list), additional information not available

CSG id	01 (27 bit)

Coding:

| BER-TLV: | D6 | 14 | 19 | 01 | 15 | 82 | 02 | 83 | 81 | 3F | 01 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 55 | 02 | 01 | 00 | 56 | 04 | 00 | 00 | 00 | 3F |  |  |

EVENT DOWNLOAD – CSG CELL SELECTION 1.1.1B

Logically:

Event list

Event 1:	CSG Cell selection

Device identities

Source device:	Network

Destination device:	UICC

Access Technology

Technology :	E-UTRAN

CSG Cell selection status:	Byte 1 = '01' (camped on a CSG or Hybrid cell of the Operator CSG list or Allowed CSG list), additional information: result of a manual CSG cell selection.

CSG id	01 (27 bit)

Coding:

| BER-TLV: | D6 | 14 | 19 | 01 | 15 | 82 | 02 | 83 | 81 | 3F | 01 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 55 | 02 | 01 | 41 | 56 | 04 | 00 | 00 | 00 | 3F |  |  |

EVENT DOWNLOAD – CSG CELL SELECTION 1.1.2

Logically:

Event list

Event 1:	CSG Cell selection

Device identities

Source device:	Network

Destination device:	UICC

Access Technology

Technology :	E-UTRAN

CSG Cell selection status :	Byte 1 = '00' (Not camped on  a CSG or Hybrid cell), additional information not available

Coding:

| BER-TLV: | D6 | 0E | 19 | 01 | 15 | 82 | 02 | 83 | 81 | 3F | 01 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 55 | 02 | 00 | 00 |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD – CSG CELL SELECTION 1.1.3A

Logically:

Event list

Event 1:	CSG Cell selection

Device identities

Source device:	Network

Destination device:	UICC

Access Technology

Technology :	E-UTRAN

CSG Cell selection status:	Byte 1 = '01' (camped on a CSG or Hybrid cell of the Operator CSG list or Allowed CSG list), additional information not available

CSG id	04 (27 bit)

HNB name	"HOME 04"

Coding:

| BER-TLV: | D6 | 25 | 19 | 01 | 15 | 82 | 02 | 83 | 81 | 3F | 01 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 55 | 02 | 01 | 00 | 56 | 04 | 00 | 00 | 00 | 9F | 57 | 0F |
|  | 80 | 00 | 48 | 00 | 4F | 00 | 4D | 00 | 45 | 00 | 20 | 00 |
|  | 30 | 00 | 34 |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD – CSG CELL SELECTION 1.1.3B

Logically:

Event list

Event 1:	CSG Cell selection

Device identities

Source device:	Network

Destination device:	UICC

Access Technology

Technology :	E-UTRAN

CSG Cell selection status:	Byte 1 = '01' (camped on a CSG or Hybrid cell of the Operator CSG list or Allowed CSG list), additional information: result of a manual CSG cell selection.

CSG id	04 (27 bit)

HNB name	"HOME 04"

Coding:

| BER-TLV: | D6 | 25 | 19 | 01 | 15 | 82 | 02 | 83 | 81 | 3F | 01 | 08 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 55 | 02 | 01 | 41 | 56 | 04 | 00 | 00 | 00 | 9F | 57 | 0F |
|  | 80 | 00 | 48 | 00 | 4F | 00 | 4D | 00 | 45 | 00 | 20 | 00 |
|  | 30 | 00 | 34 |  |  |  |  |  |  |  |  |  |

27.22.7.18.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.19	IMS registration event

It is expected that the IMS registration event will not be used seperately, but always in combination with the Incoming IMS Data Event and further features which are required for UICC access to IMS.

The IMS registration event is therefore tested in 27.22.4.27.7.1 and 27.22.7.20

#### 27.22.7.20	Incoming IMS data event

##### 27.22.7.20.1	Incoming IMS data (normal)

27.22.7.20.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.20.1.2	Conformance requirement

The ME shall support:

- the EVENT Incoming IMS DATA as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 7.5, clause 8.7, clause 8.25, clause 8.110.

- the EVENT: IMS Registration  as defined in:

- -	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 7.5, clause 8.7, clause 8.25, clause 8.111, clause 8.112.the EVENT: Data available  as defined in:

-	TS 31.111 [15] clause 4.7, clause 5.2, clause 6.4.16, clause 7.5, clause 8.7, clause 8.25, clause 8.56, clause 8.57.

- the Open Channel for IMS and Event Download – IMS Registration Event commands as defined in:

-	TS 31.111[15] clauses 5.2, clauses 6.4.27 and 6.6.27, clause 8.6, clause 8.7, clause 8.55, clause 8.110

-	TS 31.102 [14] clauses 4.2.8, 4.2.95

The ME shall support the EFUICCIARI reading procedure as defined in:

-	TS 31.103 [35] clause 4.2.16

Additionally the ME shall be able to carry out the IMS registration procedure according to TS 34.229-1 [36], Annex C.2.

27.22.7.20.1.3	Test purpose

To verify that the ME informs the UICC that an Event: Incoming IMS data has occurred using the ENVELOPE (EVENT DOWNLOAD – Incoming IMS data) command when the ME received a SIP message for the card, including an UICC IARI.

27.22.7.20.1.4	Method of test

27.22.7.20.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the Network Simulator (NWS).

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The ME activates the required bearer, discovers P-CSCF and registers with the value from the ISIM to IMS services. The ME has registered the IARI associated with active applications intalled on the UICC, stored in EF_UICCIARI on the ISIM.

The channel identifier value used for these tests is set to 1 as an example. This channel identifier is dependent on the ME's default channel identifier as declared in table A.2/27.

The E-UTRAN/EPC ISIM-UICC with the following execptions is used:

EFIST (ISIM Service Table)

EFIST shall be configured as defined in 27.22.2C.3.2 with the exception that Service 10 "Support of UICC access to IMS" is available.

EFUICCIARI (UICC IARI list)

Record 1 :

Logically:	urn:ur-7:3gpp-application.ims.iari.uicctest

| Byte: | B01 | B02 | B03 | B04 | B05 | B06 | B07 | B08 | B09 | B10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Coding: | 80 | 2B | 75 | 72 | 6E | 3A | 75 | 72 | 2D | 37 |
|  | B11 | B12 | B13 | B14 | B15 | B16 | B17 | B18 | B19 | B20 |
|  | 3A | 33 | 67 | 70 | 70 | 2D | 61 | 70 | 70 | 6C |
|  | B21 | B22 | B23 | B24 | B25 | B26 | B27 | B28 | B29 | B30 |
|  | 69 | 63 | 61 | 74 | 69 | 6F | 6E | 2E | 69 | 6D |
|  | B31 | B32 | B33 | B34 | B35 | B36 | B37 | B38 | B39 | B40 |
|  | 73 | 2E | 69 | 61 | 72 | 69 | 2E | 75 | 69 | 63 |
|  | B41 | B42 | B43 | B44 | B45 | B46 | B47 | B48 | B49 | B50 |
|  | 63 | 74 | 65 | 73 | 74 | FF | FF | FF | FF | FF |

27.22.7.20.1.4.2	Procedure

Expected Sequence 1.1	(EVENT DOWNLOAD – Incoming IMS data, IMS Registration and Data available event, IARI list stored on the ISIM)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC  ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 | [As response to the TERMINAL PROFILE command] |
| 2 | ME  UICC | FETCH |  |
| 3 | UICC  ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME  UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 | [The ME will read the ISIM Service Table and the UICC IARI list on the ISIM before it will attempt the initial registration to the IMS network] |
| 5 | ME NWS NWS  ME | ME attempts to register to IMS services with values derived from the ISIM and additionally registers the IARI from EFUICCIARI during the intial registration or subsequent registration to IMS services. | [Initial registration to the IMS network is performed according to TS 34.229-1 [36], Annex C.2] |
| 6 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – IMS registration 1.1.1 | [After the IARI "urn:ur-7:3gpp-application.ims.iari.uicctest" has been successfully registered during the intial or a subsequent SIP REGISTER message containing this IARI. If the IARI "urn:ur-7:3gpp-application.ims.iari.uicctest" is not registered during the intial registration to the IMS network further Envelopes – Event Download – IMS Registration without the IARI might have been received. These shall be ignored by the USIM Simulator.] |
| 7 | NWS ME | IMS network sends SIP INVITE message with UICC IARI |  |
| 8 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – Incoming IMS data 1.1.1 |  |
| 9 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 10 | ME UICC | FETCH |  |
| 11 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL for IMS 1.1.1 |  |
| 12 | ME | Channel id, buffer assigned |  |
| 13 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL for IMS 1.1.1 | [Command performed successfully] |
| 14 | ME  UICC | ENVELOPE: EVENT DOWNLOAD – Data Available 1.1.1 |  |
| 15 | UICC  ME | PROACTIVE COMMAND PENDING: RECEIVE DATA 1.1.1 |  |
| 16 | ME  UICC | FETCH |  |
| 17 | UICC  ME | PROACTIVE COMMAND: RECEIVE DATA 1.1.1 |  |
| 18 | ME  UICC | TERMINAL RESPONSE: RECEIVE DATA 1.1.1 | Contains SIP message received in step 7 |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	IMS Registration

Event 2:	Incoming IMS data Event

Event 3:	Data available

Coding:

| BER-TLV: | D0 | 0E | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 03 | 17 | 18 | 09 |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - IMS Registration 1.1.1

Logically:

Event list

Event 1:	IMS Registration

Device identities

Source device:	Network

Destination device:	UICC

IMPU list :	At least one IMPU containing "urn:ur-7:3gpp-application.ims.iari.uicctest"

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 17 | 82 | 02 | 83 | 81 | 77 | Note 2 | Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Note 1: The TLV length depends on the IMPU list content Note 2: The IMPU TLV length depends on the IMPU list entries. Note 3: The IMPU list shall contain the IMPU "urn:ur-7:3gpp-application.ims.iari.uicctest" and might contain further IMPUs |  |  |  |  |  |  |  |  |  |  |  |  |

ENVELOPE: EVENT DOWNLOAD - Data available 1.1.1

Logically:

Event list

Event:	Data available

Device identities

Source device:	ME

Destination device:	UICC

Channel status

Channel status:	Channel 1 open, link established

Channel Data Length

Channel data length:	200 Bytes available in Rx buffer

Coding:

| BER-TLV: | D6 | 0E | 99 | 01 | 09 | 82 | 02 | 82 | 81 | B8 | 02 | 81 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | B7 | 01 | C8 |  |  |  |  |  |  |  |  |

PROACTIVE COMMAND: OPEN CHANNEL for IMS 1.1.1

Logically:

Command details

Command number:	01

Command type:	OPEN CHANNEL

Command qualifier:	00 (RFU)

Device identities

Source device:	UICC

Destination device:	ME

Buffer

Buffer size:	1400

IARI	urn:ur-7:3gpp-application.ims.iari.uicctest

Coding:

| BER-TLV: | D0 | 3A | 81 | 03 | 01 | 40 | 00 | 82 | 02 | 81 | 82 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 02 | 05 | 78 | 76 | 2B | 75 | 72 | 6E | 3A | 75 | 72 | 2D |
|  | 37 | 3A | 33 | 67 | 70 | 70 | 2D | 61 | 70 | 70 | 6C | 69 |
|  | 63 | 61 | 74 | 69 | 6F | 6E | 2E | 69 | 6D | 73 | 2E | 69 |
|  | 61 | 72 | 69 | 2E | 75 | 69 | 63 | 63 | 74 | 65 | 73 | 74 |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	00 (RFU)

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1, link established.

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 39 | 02 | 05 | 78 |  |  |  |  |

PROACTIVE COMMAND: RECEIVE DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	UICC

Destination device:	Channel 1

Channel Data Length

Channel Data Length:	200

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 81 | 21 | B7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | C8 |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: RECEIVE DATA 1.1.1

Logically:

Command details

Command number:	1

Command type:	RECEIVE DATA

Command qualifier:	RFU

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel Data :	200 Bytes of data, includes SIP message

Channel data length:	00

Coding:

| BER-TLV: | 81 | 03 | 01 | 42 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | B6 | 81 | C8 | ab | cd | ef | .. | xy | B7 | 01 | 00 |  |

Note: The content of the channel data is not tested.

27.22.7.20.1.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1.

#### 27.22.7.21	Data Connection Status Change event

27.22.7.21.1.1	Definition and applicability

See clause 3.2.2.

27.22.7.21.1.2	Conformance requirement

The ME shall support the EVENT: Data Connection Status Change event as defined in:

-	TS 31.111 [15] clause 4.7, 4.12, 7.5.25, 8.25, 8.28, 8.137, 8.138, 8.139 and 8.142

27.22.7.21.1.3	Test purpose

If the Data Connection Status Change event is part of the current event list (as set up by the last SET UP EVENT LIST command), then, upon detection by the ME of a change in the data connection status, the terminal shall inform the UICC that this event has occurred, by using the ENVELOPE (EVENT DOWNLOAD – Data Connection Status Change) command.

27.22.7.21.1.4	Method of test

27.22.7.21.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS System Simulator.

The default E-UTRAN/EPC UICC and the following parameters are used:

Network access name: TestGp.rs

The ME shall be powered on and perform the PROFILE DOWNLOAD procedure.

The E- UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

The NB-IoT parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Identity value = 0001 (28 bits);

27.22.7.21.1.4.2	Procedure

Expected Sequence 1.1 (EVENT DOWNLOAD – Data Connection Status Change event, E-UTRAN, Deactivate PDN)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SET UP EVENT LIST 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1 |  |
| 4 | ME UICC | TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1 |  |
| 5 | ME UICC | ENVELOPE: EVENT DOWNLOAD - Data Connection Status Change event 1.1.1 | [Data connection successful; i.e. accepted by the network and completed by the device] |
| 6 | E-USS/NB-SS ME | DEACTIVATE EPS BEARER CONTEXT REQUEST | The DEACTIVATE EPS BEARER CONTEXT REQUEST message contains an ESM cause #26: insufficient resources |
| 7 | ME E-USS/NB-SS | DEACTIVATE EPS BEARER CONTEXT ACCEPT |  |
| 8 | ME UICC | ENVELOPE: EVENT DOWNLOAD – Data Connection Status Change event 1.1.2 | [Data connection dropped or deactivated] |

PROACTIVE COMMAND: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	UICC

Destination device:	ME

Event list

Event 1:	Data Connection Status Change

Coding:

| BER-TLV: | D0 | 0C | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 81 | 82 | 99 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 1D |  |  |  |  |  |  |  |  |  |  |

TERMINAL RESPONSE: SET UP EVENT LIST 1.1.1

Logically:

Command details

Command number:	1

Command type:	SET UP EVENT LIST

Command qualifier:	'00'

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 05 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

EVENT DOWNLOAD - Data Connection Status Change 1.1.1

Logically:

Event list

Event 1:	Data Connection Status Change

Device identities

Source device:	Network

Destination device:	UICC

Data connection status:	Data connection successful; i.e. accepted by the network and completed by the device.

Data connection type:	PDN connection

Transaction identifier:	the Transaction identifier data object shall contain:

- TI value generated by the terminal to uniquely identify the PDP or PDN  data connection

- TI flag is 0.

Date-Time and Time zone:	Date and time set by the user if A.1/xxx is supported by the ME

Location Information:	Mobile Country Codes (MCC) 001

MNC: 01

TAC: 0001

ECI: 000000001

Access Technology:	E-UTRAN

Location status:	Normal service

Network Access Name:	TestGp.rs

PDP/PDN type	ME dependent

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 1D | 82 | 02 | 83 | 81 | 1D | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2A | 01 | 01 | 1C | Note 2 | Note 3 | 13 | 09 | 00 | F1 | 10 | 00 |
|  | 01 | 00 | 00 | 00 | 1F | 3F | 01 | 08 | 1B | 01 | 00 | C7 |
|  | 0A | 09 | 54 | 65 | 73 | 74 | 47 | 70 | 2E | 72 | 73 | 0B |
|  | Note 4 |  |  |  |  |  |  |  |  |  |  |  |
| Note 1: The length of the BER-TLV is present here. Note 2: Transaction identifier length and data. Note 3: If A.1/229 is supported by the ME, Date-Time and Time zone shall be provided. Note 4: The PDN Type length and data. |  |  |  |  |  |  |  |  |  |  |  |  |

EVENT DOWNLOAD - Data Connection Status Change 1.1.2

Logically:

Event list

Event 1:	Data Connection Status Change

Device identities

Source device:	Network

Destination device:	UICC

Data connection status:	Data connection dropped or deactivated.

Data connection type:	PDN connection

(E)SM cause:	Insufficient resources

Transaction identifier:	the Transaction identifier data object shall contain:

- TI value generated by the terminal to uniquely identify the PDP or PDN  data connection

- TI flag is 0.

Date-Time and Time zone:	Date and time set by the user if A.1/xxx is supported by the ME

Location Information:	MCC: 001

MNC: 01

TAC: 0001

ECI: 000000001

Location status:	No service

Coding:

| BER-TLV: | D6 | Note 1 | 19 | 01 | 1D | 82 | 02 | 83 | 81 | 1D | 01 | 02 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2A | 01 | 01 | 1A | 01 | 1A | 1C | Note 2 | Note 3 | 13 | 09 | 00 |
|  | F1 | 10 | 00 | 01 | 00 | 00 | 00 | 1F | 1B | 01 | 02 |  |
| Note 1: The length of the BER-TLV is present here. Note 2: Transaction identifier length and data. Note 3: If A.1/229 is supported by the ME, Date-Time and Time zone shall be provided. |  |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.2 (EVENT DOWNLOAD – Data Connection Status Change Event, E-UTRAN, Network Rejection)

### 27.22.8	MO SHORT MESSAGE CONTROL BY USIM

#### 27.22.8.1	Definition and applicability

See clause 3.2.2.

#### 27.22.8.2	Conformance requirement

The ME shall support the MO SEND SHORT MESSAGE CONTROL facility as defined in:

-	TS 31.111 [15] clause 7.3.2.

The ME shall also support the SEND SMS facitily as specified in

-	TS 31.111 [15] clause 6.4.10

#### 27.22.8.3	Test purpose

To verify that for all SMS sending attempts, even those resulting from a SEND SHORT MESSAGE proactive UICC command, the ME shall first pass the RP_destination_address of the service center and the TP_Destination_Address to the UICC, using the ENVELOPE (MO Short Message CONTROL).

To verify that if the UICC responds with '90 00', the ME shall send the SMS with the address unchanged.

To verify that if the UICC responds with '93 00', the ME shall not send the SMS and may retry the command.

To verify that if the UICC returns response data, the ME shall use the response data appropriately to send the SM as proposed, not send the SM, or send the SM using the data supplied by the UICC.

To verify that, in the case where the initial SM request results from a proactive SEND SHORT MESSAGE, if the MO SMS CONTROL result is "not allowed" or "allowed with modifications", the ME shall inform the UICC using TERMINAL RESPONSE "interaction with call control by UICC or MO short message control by USIM, action not allowed".

#### 27.22.8.4	Method of tests

##### 27.22.8.4.1	Initial conditions

The ME is connected to the System Simulator and the USIM Simulator.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The MO SMS control service is available in the USIM Service Table.

The SMS service center address in the ME shall be set to "+112233445566778" prior to the execution of the tests.

For test sequences 1.1 to 1.8 the ME is connected to USS or SS.

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001;

The PCS 1900 parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 011;

-	Location Area Code (LAC) = 0001;

-	Cell Identity value = 0001.

For test sequences 1.10 to 1.17 the ME is connected to the E-USS/NB-SS, where:

-	SMS over SGs (DOWNLINK NAS TRANSPORT and UPLINK NAS TRANSPORT messages) is used to send and receive short messages

The E-USS parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Id  = 0001.

The NB-SS parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	NB-IoT Cell Id  = 0001.

##### 27.22.8.4.2	Procedure

Expected Sequence 1.1 (MO SM CONTROL BY USIM , with Proactive command, Allowed, no modification')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC -> ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME -> UICC | FETCH |  |
| 3 | UICC -> ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 |  |
| 4 | ME -> USER | Display "Send SM" | [Alpha Identifier] |
| 5 | ME -> UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1A Or ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 6 | UICC -> ME | MO SMS CONTROL RESULT 1.1.1 | [ "Allowed, no modification"] |
| 7 | ME -> USS | Send SMS-PP Message 1.1 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.1 without modification] |
| 8 | USS -> ME | SMS RP-ACK |  |
| 9 | ME -> UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.1.1 |  |

PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	UICC

Destination device:	Network

Alpha identifier:	"Send SM"

Address

TON:	International number

NPI:	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"00"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| BER-TLV: | D0 | 37 | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 81 | 83 | 85 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 53 | 65 | 6E | 64 | 20 | 53 | 4D | 86 | 09 | 91 | 11 |
|  | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 8B | 18 | 01 | 00 | 09 |
|  | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C | 54 | 65 | 73 |
|  | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |  |  |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.1

Logically:

SMS RPDU

RP-Originator Address	not used

RP-Destination SMSC Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

ENVELOPE MO SHORT MESSAGE CONTROL 1.1.1A

Logically:

Device identities

Source device:	ME

Destination device:	UICC

RP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"112233445566778"

TP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012345678"

Location Information

MCC & MNC	the mobile country and network code (00F110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Extended Cell ID		RNC-id value (for Rel-4 onwards), see also Note 3

Coding:

| Coding | D5 | Note 1 | 02 | 02 | 82 | 81 | 06 | 09 | 91 | 11 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 33 | 44 | 55 | 66 | 77 | F8 | 06 | 06 | 91 | 10 | 32 |
|  | 54 | 76 | F8 | 13 | Note 2 | 00 | F1 | 10 | 00 | 01 | 00 |
|  | 01 | Note 3 |  |  |  |  |  |  |  |  |  |

Note 1: Length of BER-TLV is '20' plus the actual length of all the present optional SIMPLE-TLV data objects.

Note 2: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note 3: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

ENVELOPE MO SHORT MESSAGE CONTROL 1.1.1B

Logically:

Device identities

Source device:	ME

Destination device:	UICC

RP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"112233445566778"

TP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012345678"

Location Information

MCC & MNC	the mobile country and network code (001110)

LAC	the location Area Code (0001)

Cell ID	Cell Identity Value (0001)

Coding:

| BER-TLV: | D5 | 20 | 02 | 02 | 82 | 81 | 06 | 09 | 91 | 11 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 33 | 44 | 55 | 66 | 77 | F8 | 06 | 06 | 91 | 10 | 32 |
|  | 54 | 76 | F8 | 13 | 07 | 00 | 11 | 10 | 00 | 01 | 00 |
|  | 01 |  |  |  |  |  |  |  |  |  |  |

MO SHORT MESSAGE CONTROL RESULT 1.1.1

Logically:

MO Short Message control result :	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

TERMINAL RESPONSE: SEND SHORT MESSAGE 1.1.1

Logically:

Command details

Command number:	1

Command type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (MO SM CONTROL BY USIM , with user SMS, Allowed, no modification')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER -> ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.2. |
| 2 | ME -> UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1A or ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC -> ME | MO SHORT MESSAGE CONTROL RESULT 1.1.1 | [ "Allowed, no modification"] |
| 4 | ME -> USS | Send SMS-PP Message 1.2 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.2 without modification] |
| 5 | USS -> ME | SMS RP-ACK |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.2

Logically:

SMS RPDU

RP-Originator Address	not used

RP-Destination SMSC Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"112233445566778"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	value shall not be verified

TP-VPF	value shall not be verified

TP-RP	value shall not be verified

TP-UDHI	value shall not be verified

TP-SRR	value shall not be verified

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | Note 3 |  |  |

Note 1:	Octet shall not be verified

Note 2:	Only the TP-MTI bits shall be verified

Note 3:	The remaining octets shall  not be verified

Expected Sequence 1.3 (MO SM CONTROL BY USIM , with Proactive command, Not allowed')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC -> ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME -> UICC | FETCH |  |
| 3 | UICC -> ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 |  |
| 4 | ME -> USER | Display "Send SM" | [The display of the Alpha Identifier shall not be verified] |
| 5 | ME -> UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1A or ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 6 | UICC -> ME | MO SHORT MESSAGE CONTROL RESULT 1.3.1 | [ "not Allowed"] |
| 7 | ME -> UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.3.1 | [ Permanent Problem - Interaction with Call Control or MO short message control by USIM ] |
| 8 | ME USS | The ME does not send the Short Message |  |

MO SHORT MESSAGE CONTROL RESULT 1.3.1

Logically:

MO Short Message control result :	'01' = Not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

TERMINAL RESPONSE: SEND SHORT MESSAGE 1.3.1

Logically:

Command details

Command number:	01

Command Type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Interaction with call control or MO-SM by USIM permanent problem

Additional information:	Action not allowed

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 02 | 39 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 |  |  |  |  |  |  |  |  |  |  |  |

Expected Sequence 1.4 (MO SM CONTROL BY USIM , with user SMS, Not allowed ')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER -> ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.2. |
| 2 | ME -> UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1A or ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC -> ME | MO SM CONTROL RESULT 1.3.1 | [ "Not allowed"] |
| 4 | ME USS | The ME does not send the Short Message |  |

Expected Sequence 1.5 (MO SM CONTROL BY USIM , with Proactive command, Allowed with modifications')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC -> ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME -> UICC | FETCH |  |
| 3 | UICC -> ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 | Send SMS to "+012345678" |
| 4 | ME -> USER | Display "Send SM" | [Alpha Identifier] |
| 5 | ME -> UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1A or ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 6 | UICC -> ME | MO SM CONTROL RESULT 1.5.1 | ["Allowed with modifications"] |
| 7 | ME -> USS | Send SMS-PP Message 1.5 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.5 with the data provided by the UICC to the changed Service Center Address "+112233445566779" ] |
| 8 | USS -> ME | SMS RP-ACK |  |
| 9 | ME -> UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.5.1 |  |

MO SHORT MESSAGE CONTROL RESULT 1.5.1

Logically:

MO Short Message control result :	'02' = Allowed with modifications

RP Destination_Address of the Service Center

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string:	"112233445566779"

TP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string:	"012345679"

Coding:

| 02 | 13 | 86 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 77 | F9 | 86 | 06 | 91 | 10 | 32 | 54 | 76 | F9 |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.5

Logically:

SMS RPDU

RP-Originator Address	not used

RP-Destination SMSC Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"112233445566779"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345679"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F9 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F9 | 40 | F4 | 0C |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

TERMINAL RESPONSE: SEND SHORT MESSAGE 1.5.1

Logically:

Command details

Command number:	01

Command Type:	SEND SHORT MESSAGE

Command qualifier:	packing not required

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 13 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.6 (MO SM CONTROL BY USIM , with user SMS, Allowed with modifications')

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER -> ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.2. |
| 2 | ME -> UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1A or ENVELOPE: MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC -> ME | MO SM CONTROL RESULT 1.5.1 | [ "Allowed with modifications"] |
| 4 | ME-> USS | Send SMS-PP Message 1.6 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1. 6 with the data provided by the UICC to the changed Service Center Address "+112233445566779"] |
| 5 | USS -> ME | SMS RP-ACK |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.6

Logically:

SMS RPDU

RP-Originator Address	not used

RP-Destination SMSC Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"112233445566779"

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	value shall not be verified

TP-VPF	value shall not be verified

TP-RP	value shall not be verified

TP-UDHI	value shall not be verified

TP-SRR	value shall not be verified

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345679"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F9 | Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F9 | Note 3 |  |  |

Note 1:	Octet shall not be verified.

Note 2:	Only the TP-MTI bits shall be verified.

Note 3:	The remaining octets shall  not be verified.

Expected Sequence 1.7 (MO SM CONTROL BY USIM , with Proactive command, the USIM responds with '90 00', Allowed, no modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC -> ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME -> UICC | FETCH |  |
| 3 | UICC -> ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 | Send SMS to "+012345678" |
| 4 | ME -> USER | Display "Send SM" | [Alpha Identifier] |
| 5 | ME -> UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1A or ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 6 | UICC -> ME | 90 00 |  |
| 7 | ME ->USS | Send SMS-PP | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.1 without modification] |
| 8 | USS -> ME | SMS RP-ACK |  |
| 9 | ME -> UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.1.1 |  |

Expected Sequence 1.8 (MO SM CONTROL BY USIM , Send Short Message attempt by user, the USIM responds with '90 00', Allowed, no modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User  ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.2. |
| 2 | ME  UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1 A or ENVELOPE : MO SHORT MESSAGE CONTROL 1.1.1B | [Option A shall apply for GERAN/UTRAN parameters] [Option B shall apply for PCS1900 parameters] |
| 3 | UICC  ME | 90 00 |  |
| 4 | ME USS | Send SMS-PP | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.2 without modification] |
| 5 | USS -> ME | SMS RP-ACK |  |

Expected Sequence 1.9void

Expected Sequence 1.10 (MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, Allowed, no modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC -> ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 |  |
| 4 | ME USER | Display "Send SM" | [Alpha Identifier] |
| 5 | ME UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 6 | UICC ME | MO SMS CONTROL RESULT 1.1.1 | [ "Allowed, no modification"] |
| 7 | ME E-USS/NB-SS | Send SMS-PP Message 1.10 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.10 without modification] |
| 8 | E-USS/NB-SS ME | RP-ACK |  |
| 9 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.1.1 |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.10

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | 40 | F4 | 0C |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

ENVELOPE MO SHORT MESSAGE CONTROL 1.10.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

RP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"112233445566778"

TP Destination Address

TON:	International

NPI:	"ISDN / telephone numbering plan" or "unknown"

Dialling number string	"012345678"

Location Information

Mobile Country Codes (MCC)                    	001

Mobile Network Codes (MNC)                    	01

Tracking Area Code (TAC):                  	0001

E-UTRAN Cell Identifier (ECI):             	0001

Coding:

| Coding | D5 | 22 | 02 | 02 | 82 | 81 | 06 | 09 | 91 | 11 | 22 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 33 | 44 | 55 | 66 | 77 | F8 | 06 | 06 | 91 | 10 | 32 |
|  | 54 | 76 | F8 | 13 | 09 | 00 | F1 | 10 | 00 | 01 | 00 |
|  | 00 | 00 | 1F |  |  |  |  |  |  |  |  |

Expected Sequence 1.11 (MO SM CONTROL BY USIM over SG in E-UTRAN, with user SMS, Allowed, no modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The user makes a SMS with the user data "Test Message"and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.11. |
| 2 | ME UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 3 | UICC ME | MO SHORT MESSAGE CONTROL RESULT 1.1.1 | [ "Allowed, no modification"] |
| 4 | ME E-USS/NB-SS | Send SMS-PP Message 1.11 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.11 without modification] |
| 5 | E-USS/NB-SS ME | RP-ACK |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.11

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	value shall not be verified

TP-VPF	value shall not be verified

TP-RP	value shall not be verified

TP-UDHI	value shall not be verified

TP-SRR	value shall not be verified

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345678"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F8 | Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F8 | Note 3 |  |  |

Note 1:	Octet shall not be verified

Note 2:	Only the TP-MTI bits shall be verified

Note 3:	The remaining octets shall not be verified

Expected Sequence 1.12 (MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, Not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 |  |
| 4 | ME USER | Display "Send SM" | [The display of the Alpha Identifier shall not be verified] |
| 5 | ME UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 6 | UICC ME | MO SHORT MESSAGE CONTROL RESULT 1.3.1 |  |
| 7 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.3.1 | [ Permanent Problem - Interaction with Call Control or MO short message control by USIM ] |
| 8 | ME E-USS/NB-SS | The ME does not send the Short Message |  |

Expected Sequence 1.13 (MO SM CONTROL BY USIM over SG in E-UTRAN, with user SMS, Not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.10. |
| 2 | ME UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 3 | UICC ME | MO SM CONTROL RESULT 1.3.1 | [ "Not allowed"] |
| 4 | ME E-USS/NB-SS | The ME does not send the Short Message |  |

Expected Sequence 1.14 (MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, Allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 | Send SMS to "+012345678" |
| 4 | ME USER | Display "Send SM" | [Alpha Identifier] |
| 5 | ME UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 6 | UICC ME | MO SM CONTROL RESULT 1.5.1 | ["Allowed with modifications"] |
| 7 | ME E-USS/NB-SS | Send SMS-PP Message 1.14 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.14 with the data provided by the UICC to the changed Service Center Address "+112233445566779" ] |
| 8 | E-USS/NB-SS ME | RP-ACK |  |
| 9 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.5.1 |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.14

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	Instruct the SC to accept an SMS-SUBMIT for a SM

TP-VPF	TP-VP field not present

TP-RP	TP-Reply-Path is not set in this SMS-SUBMIT

TP-UDHI	The TP-UD field contains only the short message

TP-SRR	A status report is not requested

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345679"

TP-PID	Short message type 0

TP-DCS

Message coding	8-bit data

Message class	class 0

TP-UDL	12

TP-UD	"Test Message"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F9 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 01 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F9 | 40 | F4 | 0C |
|  | 54 | 65 | 73 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |

Expected Sequence 1.15 (MO SM CONTROL BY USIM over SG in E-UTRAN, with user SMS, Allowed with modifications)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | USER ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.15. |
| 2 | ME UICC | ENVELOPE: MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 3 | UICC ME | MO SM CONTROL RESULT 1.5.1 | [ "Allowed with modifications"] |
| 4 | ME E-USS/NB-SS | Send SMS-PP Message 1.15 | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.15 with the data provided by the UICC to the changed Service Center Address "+112233445566779"] |
| 5 | E-USS/NB-SS ME | RP-ACK |  |

SMS-PP (SEND SHORT MESSAGE) Message 1.15

Logically:

SMS TPDU

TP-MTI	SMS-SUBMIT

TP-RD	value shall not be verified

TP-VPF	value shall not be verified

TP-RP	value shall not be verified

TP-UDHI	value shall not be verified

TP-SRR	value shall not be verified

TP-MR	"01"

TP-DA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"012345679"

Coding:

| Coding | 00 | 09 | 91 | 11 | 22 | 33 | 44 | 55 | 66 | 77 | F9 | Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Note 2 | 01 | 09 | 91 | 10 | 32 | 54 | 76 | F9 | Note 3 |  |  |

Note 1:	Octet shall not be verified.

Note 2:	Only the TP-MTI bits shall be verified.

Note 3:	The remaining octets shall not be verified.

Expected Sequence 1.16 (MO SM CONTROL BY USIM over SG in E-UTRAN, with Proactive command, the USIM responds with '90 00', Allowed, no modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: SEND SHORT MESSAGE 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND: SEND SHORT MESSAGE 1.1.1 | Send SMS to "+012345678" |
| 4 | ME USER | Display "Send SM" | [Alpha Identifier] |
| 5 | ME UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 6 | UICC ME | 90 00 |  |
| 7 | ME E-USS/NB-SS | Send SMS-PP | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.10 without modification] |
| 8 | E-USS/NB-SS ME | RP-ACK |  |
| 9 | ME UICC | TERMINAL RESPONSE: SEND SHORT MESSAGE 1.1.1 |  |

Expected Sequence 1.17 (MO SM CONTROL BY USIM over SG in E-UTRAN, Send Short Message attempt by user, the USIM responds with '90 00', Allowed, no modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME | The user makes a SMS with the user data "Test Message" and sends it to +012345678. | [The data entered and the ME settings shall lead to the same SMS-TPDU as defined in SMS-PP (SEND SHORT MESSAGE) Message 1.11. |
| 2 | ME UICC | ENVELOPE : MO SHORT MESSAGE CONTROL 1.10.1 |  |
| 3 | UICC ME | 90 00 |  |
| 4 | ME E-USS/NB-SS | Send SMS-PP | [The ME sends the SM containing SMS-PP (SEND SHORT MESSAGE) Message 1.11 without modification] |
| 5 | E-USS/NB-SS ME | RP-ACK |  |

#### 27.22.8.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.

### 27.22.9	Handling of command number

#### 27.22.9.1	Definition and applicability

See clause 3.2.2.

#### 27.22.9.2	Conformance requirement

The ME shall support the facility as defined in TS 31.111 [15] clause 6.5.1, clause 6.8 and clause 8.6

#### 27.22.9.3	Test purpose

To verify that the ME sends a Terminal Response with the Command number equivalent to the value in the corresponding proactive command.

#### 27.22.9.4	Method of tests

##### 27.22.9.4.1	Initial conditions

The ME is connected to the USIM Simulator.

The elementary files are coded as Toolkit default.

Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The ME screen shall be in its normal stand-by display.

The ME shall support the DISPLAY TEXT command.

##### 27.22.9.4.2	Procedure

Expected Sequence 1.1 (DISPLAY TEXT normal priority, Unpacked 8 bit data for Text String, successful)

See ETSI TS 102 384 [26] in subclause 27.22.9.4.2, Expected Sequence 1.1.

#### 27.22.9.5	Test requirement

The ME shall operate in the manner defined in expected sequence 1.1

### 27.22.10	CALL CONTROL on EPS PDN Connection

#### 27.22.10.1	Procedure for Mobile Originated calls

##### 27.22.10.1.1	Definition and applicability

See clause 3.2.2.

##### 27.22.10.1.2	Conformance requirement

The ME shall support the CALL CONTROL facility as defined in:

-	TS 31.111 [15] clause 7.3, clause 7.3.1.8, clause 8.98

-	TS 24.301 [32], clause 6.4.3.2 and 6.5.4

-	TS 36.508 [33], clause 6.6.1.

##### 27.22.10.1.3	Test purpose

To verify that when the service "call control on EPS PDN connection by USIM" is available in the USIM Service Table, then for all EPS PDN connection activation (including those resulting from a OPEN CHANNEL proactive UICC command where E-UTRAN is selected), the ME shall first pass the corresponding PDN Connectivity Request message to the UICC, using the ENVELOPE (CALL CONTROL) command. The ME shall also pass to the UICC in the ENVELOPE (CALL CONTROL) command the current serving cell.

To verify that the ME interpret the UICC returns response correctly.

##### 27.22.10.1.4	Method of tests

27.22.10.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the E-USS/NB-SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default E-UTRAN/EPC UICC, the default E-UTRAN parameters and the following parameters are used:

Network access name:		TestGp.rs

User login:			UserLog

User password:			UserPwd

UICC/ME interface transport level

Transport format:		TCP

Port number:		44444

Data destination address:	01.01.01.01 (as an example)

The E-USS parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	E-UTRAN Cell Id = 0001.

The NB-SS parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Tracking Area Code (TAC) = 0001;

-	NB-IoT Cell Id = 0001.

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

- The call control on EPS PDN connection by USIM service is available in the USIM Service Table.

27.22.10.1.4.2	Procedure

Expected Sequence 1.1 (CALL CONTROL on EPS PDN for E-UTRAN – default PDN connection activation, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure |
| 2 | UICC ME | CALL CONTROL RESULT 1.1.1 | [Call control result: "Allowed", no modification] |
| 3 | ME E-USS/NB-SS | The PDN connection is established successfully without modification | Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL are used to establish the PDN connection |

ENVELOPE CALL CONTROL 1.1.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

EPS PDN connection activation parameters

Protocol Discriminator:                       			EPS session management messages

EPS bearer identity:                    		No EPS bearer identity assigned

Procedure Transaction Identity:  		1

PDN connectivity request message identity:    	PDN connectivity request

Request type:                                 		Initial request

PDN Type:                                     		IPv4 and/or IPv6

Access Point Name:	TestGp.rs

Protocol configuration options:

Protocol config. optional contents:		content not checked

Capability configuration parameters 1

This parameter is optional. If present, the contents shall not be checked.

Location Information

Mobile Country Codes (MCC)                    	001

Mobile Network Codes (MNC)                    	01

Tracking Area Code (TAC):                  	0001

E-UTRAN Cell Identifier (ECI):             	000000001

Capability configuration parameters 2

This parameter is optional. If present, the contents shall not be checked.

Coding:

Coding:

| BER-TLV: | D4 | Note1 | 02 | 02 | 82 | 81 | 7C | Note 2 | 02 | 01 | D0 | X1 Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | 28 | 0A | 09 | 54 | 65 | 73 | 74 | 47 | 70 | 2E | 72 |
|  | 73 | Note 4 | 13 | 09 | 00 | F1 | 10 | 00 | 01 | 00 | 00 | 00 |
|  | 1F |  |  |  |  |  |  |  |  |  |  |  |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of EPS PDN connection activation parameters, dependent of optional fields.

Note 3: X is the PDN Type.

Note 4:  Optional fields.

CALL CONTROL RESULT 1.1.1

Logically:

Call control result:	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

Expected Sequence 1.2 (CALL CONTROL on EPS PDN for E-UTRAN – default PDN connection activation, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME  UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure |
| 2 | UICC  ME | CALL CONTROL RESULT 1.2.1 | [Call control result: " not Allowed", ] The ME may retry to send the command. |
| 3 | ME E-USS/NB-SS | The ME shall not send the PDN Connectivity Request message. |  |

CALL CONTROL RESULT 1.2.1

Logically:

Call control result:	'01' = not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

Expected Sequence 1.3 (CALL CONTROL on EPS PDN for E-UTRAN – default PDN connection activation, allowed with modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure |
| 2 | UICC ME | CALL CONTROL RESULT 1.3.1 | [Call control result: "Allowed with modifications"] |
| 3 | ME E-USS/NB-SS | The PDN connection is established successfully with modification | Same EPS PDN activation parameters returned by the UICC in the CALL CONTROL RESULT 1.3.1 are used to establish the PDN connection. |

CALL CONTROL RESULT 1.3.1

Logically:

Call control result:	'02' = Allowed with modifications

Address:

EPS PDN connection activation parameters

Protocol Discriminator:                       		EPS session management messages

EPS bearer identity:                         		No EPS bearer identity assigned

Procedure Transaction Identity:               		1

PDN connectivity request message identity: 	PDN connectivity request

Request type:                                 		Initial request

PDN Type:                                     		same PDN Type in step 1)

Access Point Name:					Test12.rs

Coding:

| BER-TLV: | 02 | Note 1 | 7C | Note 2 | 02 | 01 | D0 | X1 Note 3 | 28 | 0A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 54 | 65 | 73 | 74 | 31 | 32 | 2E | 72 | 73 |
|  | Note 4 |  |  |  |  |  |  |  |  |  |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of EPS PDN context activation parameters, dependant of optional fields.

Note 3: X is the PDN Type.

Note 4: Optional fields, same as in ENVELOPE CALL CONTROL 1.1.1.

Expected Sequence 1.4 (CALL CONTROL on EPS PDN for E-UTRAN – PDN connection triggered by user, UICC sends 90 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL in are used to establish the PDN connection |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to establish a PDN connection | [see initial conditions] |
| 4 | ME  UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 5 | UICC  ME | 90 00 |  |
| 6 | ME E-USS | PDN CONNECTIVITY REQUEST |  |
| 7 | E-USS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 8 | ME E-USS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 9 | ME E-USS | The PDN connection is established successfully without modification | Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL in step 5 are used to establish the PDN connection |

ENVELOPE CALL CONTROL 1.4.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

EPS PDN connection activation parameters

Protocol Discriminator:                       		EPS session management messages

EPS bearer identity:                          		No EPS bearer identity assigned

Procedure Transaction Identity:               		2

PDN connectivity request message identity: 	PDN connectivity request

Request type:                                 		Initial request

PDN Type:                                     		IPv4 and/or IPv6

Access Point Name:	Test12.rs

Other Protocol configuration options:

Protocol config. options contents:	not checked

Location Information

Mobile Country Codes (MCC)                    	001

Mobile Network Codes (MNC)                    	01

Tracking Area Code (TAC):                  		0001

E-UTRAN Cell Identifier (ECI):             		000000001

Coding:

| BER-TLV: | D4 | Note 1 | 02 | 02 | 82 | 81 | 7C | Note 2 | 02 | 01 | D0 | Note 3 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | D1 | 28 | 0A | 09 | 54 | 65 | 73 | 74 | 31 | 32 | 2E | 72 |
|  | 73 |  | Note 4 | 13 | 09 | 00 | F1 | 10 | 00 | 01 | 00 | 01 |
|  | 00 | 01 |  |  |  |  |  |  |  |  |  |  |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of EPS PDN connection activation parameters, dependant of optional fields.

Note 3: X is the PDN Type.

Note 4:  Optional fields.

Expected Sequence 1.5 (CALL CONTROL on EPS PDN for E-UTRAN – PDN connection triggered by user, UICC sends 93 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL are used to establish the PDN connection. |
| 2 | UICC  ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to establish a PDN connection | [see initial conditions] |
| 4 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 5 | UICC  ME | 93 00 | The ME may retry to send the command. |
| 6 | ME E-USS | The ME shall not send the PDN Connectivity Request message. |  |

Expected Sequence 1.6 (CALL CONTROL on EPS PDN for E-UTRAN – PDN connection triggered by user, allowed with modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure Same EPS PDN activation parameters used by the ME in the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to establish a PDN connection | [see initial conditions] |
| 4 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 | . |
| 5 | UICC ME | CALL CONTROL RESULT 1.6.1 | [Call control result: "Allowed with modifications", ] |
| 6 | ME E-USS | PDN CONNECTIVITY REQUEST |  |
| 7 | E-USS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 8 | ME E-USS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 9 | ME E-USS | The PDN connection is established successfully with modification | Same EPS PDN activation parameters returned by the UICC in the CALL CONTROL RESULT 1.6.1 are used to establish the PDN connection. |

CALL CONTROL RESULT 1.6.1

Logically:

Call control result:	'02' = Allowed with modifications

Address:

EPS PDN connection activation parameters

Protocol Discriminator:                       		EPS session management messages

EPS bearer identity:                         		No EPS bearer identity assigned

Procedure Transaction Identity:               		2

PDN connectivity request message identity: 	PDN connectivity request

Request type:                                 		Initial request

PDN Type:                                     		same PDN Type in step 5)

Access Point Name:					Test13.rs

Coding:

| BER-TLV: | 02 | Note 1 | 7C | Note 2 | 02 | 02 | D0 | X1 Note 3 | 28 | 0A |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 09 | 54 | 65 | 73 | 74 | 31 | 33 | 2E | 72 | 73 |
|  | Note 4 |  |  |  |  |  |  |  |  |  |

Note 1: The length of the BER-TLV is present here.

Note 2: Length of EPS PDN context activation parameters, dependant of optional fields.

Note 3: X is the PDN Type.

Note 4: Optional fields, same as in ENVELOPE CALL CONTROL 1.4.1.

Expected Sequence 1.7 (CALL CONTROL on EPS PDN -  PDN connection activation from OPEN CHANNEL command)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDN establishment during ATTACH procedure Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL are used to establish the PDN connection |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required | [see initial conditions] |
| 4 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 5 | ME UICC | FETCH |  |
| 6 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 1.1.1 |  |
| 7 | ME USER | The ME may display channel opening information |  |
| 8 | ME  UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 9 | UICC  ME | 90 00 |  |
| 10 | ME E-USS | PDN CONNECTIVITY REQUEST | The UE may request IPv4 or IPv4v6 as PDN type |
| 11 | E-USS ME | ACTIVATE DEFAULT EPS BEARER CONTEXT REQUEST | [The E-UTRAN parameters are used] |
| 12 | ME E-USS | ACTIVATE DEFAULT EPS BEARER CONTEXT ACCEPT |  |
| 13 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 1.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 1.1.1B | [Command performed successfully OR Command performed with modifications] |
| 14 | ME E-USS | The PDN connection is established successfully without modification | Same EPS PDN activation parameters returned by the ME within the ENVELOPE CALL CONTROL in step 8 are used to establish the PDN connection. |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	UICC

Destination device:	ME

Bearer

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Network access name:		Test12.rs

Text String:			"UserLog" (User login)

Text String:			"UserPwd" (User password)

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address	01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 30 | 32 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:	1

Command type:	OPEN CHANNEL

Command qualifier:	immediate link establishment

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed with modifications

Channel status	Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:	GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:

Precedence Class:	03

Delay Class:	04

Reliability Class:	02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:	02 (IP)

Buffer

Buffer size:	1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

27.22.10.1.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.

#### 27.22.11	CALL CONTROL on PDP Context Activation

##### 27.22.11.1	Procedure for Mobile Originated calls

27.22.11.1.1	Definition and applicability

See clause 3.2.2.

27.22.11.1.2	Conformance requirement

The ME shall support the CALL CONTROL for PDP Context Activation facility as defined in:

-	TS 31.111 [15] clause 7.3, clause 7.3.1.7, clause 8.72

-	TS 24.008 [10], clause 6.1.3.3, 9.5.7 and 9.5.8

-	TS 36.508 [33], clause 4.8.4.

27.22.11.1.3	Test purpose

To verify that when the service "call control on GPRS by USIM" is available in the USIM Service Table, then for all PS PDP Context Activation (including those resulting from an OPEN CHANNEL proactive UICC command where UTRAN is selected), the ME shall first pass the corresponding Activate PDP Context message to the UICC, using the ENVELOPE (CALL CONTROL) command. The ME shall also pass to the UICC in the ENVELOPE (CALL CONTROL) command the current serving cell.

To verify that the ME interpret the UICC returns response correctly.

27.22.11.1.4	Method of tests

27.22.11.1.4.1	Initial conditions

The ME is connected to the USIM Simulator and the USS/SS. Prior to this test the ME shall have been powered on and performed the PROFILE DOWNLOAD procedure.

The default GERAN/UTRAN/EPC UICC, the default GERAN/UTRAN parameters and the following parameters are used:

Network access name:	TestGp.rs

User login:		UserLog

User password:		UserPwd

UICC/ME interface transport level

Transport format:	TCP

Port number:	44444

Data destination address:	01.01.01.01 (as an example)

The GERAN/UTRAN parameters of the system simulator are:

-	Mobile Country Code (MCC) = 001;

-	Mobile Network Code (MNC) = 01;

-	Location Area Code (LAC) = 0001;-	Cell Identity Value = 0001;

-	The simulator must accept connections requests for APNs: TestGp.rs, Test12.rs and Test13.rs

The elementary files are coded as USIM Application Toolkit default with the following exceptions:

-	The call control on GPRS by USIM service is available in the USIM Service Table.

27.22.11.4.2	Procedure

Expected Sequence 1.1 (CALL CONTROL on PDP Context Activation – default PDP connection activation, allowed without modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required. | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure |
| 2 | UICC ME | CALL CONTROL RESULT 1.1.1 | [Call control result: "Allowed", no modification] |
| 3 | ME USS/SS | The PDP connection is established successfully without modification | Same PDP activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection |

ENVELOPE CALL CONTROL 1.1.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

PDP Context Activation parameters

Protocol Discriminator:                       		GPRS session management messages

Transaction Identifier:  		0

Request PDP context activation message identity:	Activate PDP context request

Requested NSAPI: 			NSAPI 5

Requested LLC SAPI:		SAPI 3

Requested QoS:			Subscribed QoS parameters

Requested PDP address:

PDP type organisation:			as declared by the ME

PDP type:			as declared by the ME

Address:			as declared by the ME

Access point name:			06 54 65 73 74 47 70 02 72 73 ("TestGp.rs")

Protocol configuration options:

Protocol config. optional contents:		content not checked

Location Information

MCC:	                   		001

MNC:	            	    	01

Location Area Code:         	    		0001

Cell Identity Value:             		0001

Extended Cell Identity Value:		RNC-id value (for Rel-4 onwards), see also Note 6

Coding:

| BER-TLV | D4 | Note1 | 02 |  | 02 | 82 | 81 | 52 | Note2 | 0A | 41 | 05 | 03 | 0E |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 |  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | Note3 | 28 |  | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 |
|  | 73 | Note4 | 13 |  | Note5 | 00 | F1 | 10 | 00 | 01 | 00 | 01 | Note6 |  |

Note1: Length of BER-TLV, dependant of optional fields.

Note2: Length of PDP context activation parameters, dependant of optional fields.

Note3: Requested PDP Address.

Note4: Optional fields.

Note5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified.

CALL CONTROL RESULT 1.1.1

Logically:

Call control result:	'00' = Allowed, no modification

Coding:

| BER-TLV: | 00 | 00 |
| --- | --- | --- |

Expected Sequence 1.2 (CALL CONTROL on PDP Context Activation – default PDP connection activation, not allowed)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER  ME | Set and configure APN "Test.Gp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME  UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure |
| 2 | UICC  ME | CALL CONTROL RESULT 1.2.1 | [Call control result: " not Allowed", ] The ME may retry to send the command. |
| 3 | ME  USS/SS | The ME shall not send the Activate PDP Context Request message. |  |

CALL CONTROL RESULT 1.2.1

Logically:

Call control result:	'01' = not Allowed

Coding:

| BER-TLV: | 01 | 00 |
| --- | --- | --- |

Expected Sequence 1.3 (CALL CONTROL on PDP Context Activation – default PDP connection activation, allowed with modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure |
| 2 | UICC ME | CALL CONTROL RESULT 1.3.1 | [Call control result: "Allowed with modifications"] |
| 3 | ME USS/SS | The PDP connection is established successfully with modification | Same PDP activation parameters returned by the UICC within the CALL CONTROL RESULT 1.3.1 are used to establish the PDP connection |

CALL CONTROL RESULT 1.3.1

Logically:

Call control result:	'02' = Allowed with modifications

Address:

PDP Context Activation parameters

Protocol Discriminator:                       		GPRS session management messages

Transaction Identifier:  		0

Request PDP context activation message identity:	Activate PDP context request

Requested NSAPI: 			NSAPI 5

Requested LLC SAPI:		SAPI 3

Requested QoS:			Subscribed QoS parameters

Requested PDP address:

PDP type organisation:		as declared by the ME

PDP type:			as declared by the ME

Address:			as declared by the ME

Access point name:			06 54 65 73 74 31 32 02 72 73 ("Test12.rs")

Coding:

| BER-TLV: | 02 | Note1 | 52 | Note2 | 0A | 41 | 05 | 03 | 0E | 00 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | Note3 |
|  | 28 | 0A | 06 | 54 | 65 | 73 | 74 | 31 | 32 | 02 | 72 | 73 |
|  | Note4 |  |  |

Note1: Length of BER-TLV, dependant on optional fields.

Note2: Length dependant on optional fields.

Note3: Requested PDP Address.

Note4: Optional fields.

Expected Sequence 1.4 (CALL CONTROL on PDP Context Activation – PDP connection triggered by user, UICC sends 90 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "Test.Gp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure Same PDP activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection. |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to perform a PS call to Activate PDP Context Request connection | [see initial conditions] |
| 4 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 5 | UICC ME | 90 00 |  |
| 6 | ME USS/SS | ACTIVATE DEFAULT PDP CONTEXT REQUEST | [The UTRAN parameters are used] |
| 7 | USS/SS ME | ACTIVATE DEFAULT PDP CONTEXT ACCEPT |  |
| 8 | ME USS/SS | The PDP connection is established successfully without modification | Same PDP activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.4.1 are used to establish the PDN connection |

ENVELOPE CALL CONTROL 1.4.1

Logically:

Device identities

Source device:	ME

Destination device:	UICC

PS PDP connection activation parameters

Protocol Discriminator:                       		GPRS session management messages

Transaction Identifier:               		0

Request PDP context activation message identity:	Activate PDP context request

Requested NSAPI: 			NSAPI 5

Requested LLC SAPI:			SAPI 3

Requested QoS:			Subscribed QoS parameter

Requested PDP address:

PDP type organisation:		as declared by the ME

PDP type:			as declared by the ME

Address:			as declared by the ME

Access Point Name:			06 54 65 73 74 31 32 02 72 73 ( "Test12.rs")

Other Protocol configuration options:

Protocol config. options contents:		not checked

Location Information

MCC:	                   		001

MNC:	            	    	01

Location Area Code:         	    		0001

Cell Identity Value:             		0001

Extended Cell Identity Value:		RNC-id value (for Rel-4 onwards), see also Note 6

Coding:

| BER-TLV | D4 | Note1 | 02 | 02 | 82 | 81 | 52 | Note2 | 0A | 41 | 05 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | 00 | 00 | Note3 | 28 | 0A | 06 | 54 | 65 | 73 | 74 | 31 |
|  | 32 | 02 | 72 | 73 | Note4 | 13 | Note5 | 00 | F1 | 10 | 00 | 01 |
|  | 00 | 01 | Note6 |

Note1: Length of BER-TLV, dependant on optional fields.

Note2: Length of PDP context activation parameters, dependant on optional fields.

Note3: Requested PDP address.

Note4: Optional fields.

Note5: Depending on the presence of the Extended Cell Identity Value the length is '07' or '09'

Note6: The Extended Cell Identity Value is present in Rel-4 and onwards implementations, the values of the two bytes shall not be verified

Expected Sequence 1.5 (CALL CONTROL on PDP Context Activation – PDP connection triggered by user, UICC sends 93 00)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "Test.Gp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure. Same EPS PDN activation parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDN connection. |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to perform a PS call to Activate PDP Context Request connection | [see initial conditions] |
| 4 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 5 | UICC ME | 93 00 | The ME may retry to send the command. |
| 6 | ME USS/SS | The ME shall not send the Activate PDP Context Request message. |  |

Expected Sequence 1.6 (CALL CONTROL on PDP Context Activation – PDP connection triggered by user, allowed with modification)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | ME UICC | ENVELOPE CALL CONTROL 1.1.1 | For default PDP establishment during ATTACH procedure Same PDP parameters used by the ME within the ENVELOPE CALL CONTROL 1.1.1 are used to establish the PDP connection. |
| 2 | UICC ME | 90 00 |  |
| 3 | USER ME | Set and configure APN "Test12.rs" in the terminal configuration if required, and trigger the ME to perform a PS call to Activate PDP Context Request connection. | [see initial conditions] |
| 4 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 5 | UICC ME | CALL CONTROL RESULT 1.6.1 | [Call control result: "Allowed with modifications", ] |
| 6 | ME USS/SS | ACTIVATE PDP CONTEXT REQUEST | [The UTRAN parameters are used] |
| 7 | USS/SS ME | ACTIVATE PDP CONTEXT ACCEPT |  |
| 8 | ME USS/SS | The PDP connection is established successfully with modification | Same PDP parameters returned by the UICC within the CALL CONTROL RESULT 1.6.1 are used to establish the PDP connection. |

CALL CONTROL RESULT 1.6.1

Logically:

Call control result:	'02' = Allowed with modifications

Address:

PDP Context Activation parameters

Protocol Discriminator:                       	GPRS session management messages

Transaction Identifier:  		0

Request PDP context activation message identity:	Activate PDP context request

Requested NSAPI: 			NSAPI 5

Requested LLC SAPI:		SAPI 3

Requested QoS:			Subscribed QoS parameters

Requested PDP address:

PDP type organisation:		as declared by the ME

PDP type:			as declared by the ME

Address:			as declared by the ME

Access point name:	06 54 65 73 74 31 33 02 72 73 ( "Test13.rs")

Coding:

| BER-TLV: | 02 | Note1 | 52 | Note2 | 0A | 41 | 05 | 03 | 0E | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 | 00 |
|  | 00 | Note3 | 28 | 0A | 06 | 54 | 65 | 73 | 74 | 31 | 33 |
|  | 02 | 72 | 73 | Note4 |

Note1: Length of BER-TLV, dependant on optional fields.

Note2: Length dependant on optional fields.

Note3: Requested PDP address.

Note4: Optional fields.

Expected Sequence 1.7 (CALL CONTROL on PDP Context Activation - PDP connection activation from OPEN CHANNEL command)

| Step | Direction | Message / Action | Comments |
| --- | --- | --- | --- |
| 0 | USER ME | Set and configure APN "TestGp.rs" in the terminal configuration if required | [see initial conditions] |
| 1 | UICC ME | PROACTIVE COMMAND PENDING: OPEN CHANNEL 1.1.1 |  |
| 2 | ME UICC | FETCH |  |
| 3 | UICC ME | PROACTIVE COMMAND : OPEN CHANNEL 1.1.1 |  |
| 4 | ME USER | The ME may display channel opening information |  |
| 5 | ME UICC | ENVELOPE CALL CONTROL 1.4.1 |  |
| 6 | UICC ME | 90 00 |  |
| 7 | ME USS/SS | ACTIVATE DEFAULT PDP CONTEXT REQUEST | [The UTRAN parameters are used] |
| 8 | USS/SS ME | ACTIVATE DEFAULT PDP CONTEXT ACCEPT |  |
| 9 | ME UICC | TERMINAL RESPONSE : OPEN CHANNEL 1.1.1A OR TERMINAL RESPONSE : OPEN CHANNEL 1.1.1B | [Command performed successfully OR Command performed with modifications] |
| 10 | ME USS/SS | The PDP connection is established successfully without modification | Same PDP parameters used by the ME within the ENVELOPE CALL CONTROL 1.4.1 are used to establish the PDP connection. |

PROACTIVE COMMAND: OPEN CHANNEL 1.1.1

Logically:

Command details

Command number:		1

Command type:		OPEN CHANNEL

Command qualifier:		immediate link establishment

Device identities

Source device:		UICC

Destination device:		ME

Bearer

Bearer type:		GPRS / UTRAN packet service / E-UTRAN

Precedence Class:		03

Delay Class:		04

Reliability Class:		02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:		02 (IP)

Buffer

Buffer size:		1400

Network access name:		TestGp.rs

Text String:		"UserLog" (User login)

Text String:		"UserPwd" (User password)

UICC/ME interface transport level

Transport format:		TCP

Port number:		44444

Data destination address		01.01.01.01

Coding:

| BER-TLV: | D0 | 42 | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 81 | 82 | 35 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 02 | 03 | 04 | 02 | 09 | 1F | 02 | 39 | 02 | 05 | 78 |
|  | 47 | 0A | 06 | 54 | 65 | 73 | 74 | 47 | 70 | 02 | 72 | 73 |
|  | 0D | 08 | F4 | 55 | 73 | 65 | 72 | 4C | 6F | 67 | 0D | 08 |
|  | F4 | 55 | 73 | 65 | 72 | 50 | 77 | 64 | 3C | 03 | 02 | AD |
|  | 9C | 3E | 05 | 21 | 01 | 01 | 01 | 01 |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1A

Logically:

Command details

Command number:		1

Command type:		OPEN CHANNEL

Command qualifier:		immediate link establishment

Device identities

Source device:		ME

Destination device:		UICC

Result

General Result:		Command performed successfully

Channel status		Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:		GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:		IP (Internet Protocol, IETF STD 5)

Precedence Class:		03

Delay Class:		04

Reliability Class:		02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:		02 (IP)

Buffer

Buffer size:		1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

TERMINAL RESPONSE: OPEN CHANNEL 1.1.1B

Logically:

Command details

Command number:		1

Command type:		OPEN CHANNEL

Command qualifier:		immediate link establishment

Device identities

Source device:		ME

Destination device:		UICC

Result

General Result:		Command performed with modifications

Channel status		Channel identifier 1 and link established or PDP context activated

Bearer description

Bearer type:		GPRS / UTRAN packet service / E-UTRAN

Bearer parameter:		IP (Internet Protocol, IETF STD 5)

Precedence Class:		03

Delay Class:		04

Reliability Class:		02

Peak throughput class:	09

Mean throughput class:	31

Packet data protocol:		02 (IP)

Buffer

Buffer size:		1400

Coding:

| BER-TLV: | 81 | 03 | 01 | 40 | 01 | 82 | 02 | 82 | 81 | 83 | 01 | 07 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 38 | 02 | 81 | 00 | 35 | 07 | 02 | 03 | 04 | 02 | 09 | 1F |
|  | 02 | 39 | 02 | 05 | 78 |  |  |  |  |  |  |  |

#### 27.22.11.5	Test requirement

The ME shall operate in the manner defined in expected sequences 1.1 to 1.7.

##### 27.22.12	Change eCall mode

27.22.12.1	Definition and applicability

See clause 3.2.2.

27.22.12.2	Conformance requirement

The UE shall support eCall feature on the USIM and is able to change the eCall mode as specified in:

-	TS 31.102 [14] clause 5.3.40.

27.22.12.3	Test purpose

To verify that the ME is able to change the eCall mode after the re-configuration of the USIM from eCall only support to eCall and Normal call support, or vice versa, by changing the content of the USIM Service Table, and/or by changing the content of the EFEST, where the ME shall be notified of the changes by using the REFRESH proactive command.

27.22.12.4	Method of test

27.22.12.4.1	Initial conditions

The ME is connected to the USIM Simulator and connected to the E-USS or USS.

For test sequences 1.1 – 1.5 the E-UTRAN parameters of the E-USS are:

-	Mobile Country Code (MCC) = 246;

-	Mobile Network Code (MNC) = 81;

-	Tracking Area Code (TAC) = 0001.

For test sequences 1.6 and 1.7 the GERAN or UTRAN parameters of the USS are:

Mobile Country Code (MCC) = 246;

-	Mobile Network Code (MNC) = 81;

-	Location Area Code (LAC) = 0001.

The elementary files are coded as the default E-UTRAN/EPC UICC, with the following exceptions:

For test sequences 1.1, 1.4:

- EFUST the services  (Service n°2) Fixed Dialling Numbers (FDN),  (Service n°4)  Service Dialling Numbers (SDN) and (Service n°89) eCall Data are available

- EFEST has the Service n°2 Fixed Dialling Numbers (FDN) is enabled

- EFFDN contains only two entries: eCall test number (123) and eCall reconfiguration number (345)

- EFSDN the last two entries contains two entries: eCall test number (456) and eCall reconfiguration number (678)

For test sequences 1.2, and 1.5:

- EFUST the services  (Service n°2) Fixed Dialling Numbers (FDN),  (Service n°4)  Service Dialling Numbers (SDN) and (Service n°89) eCall Data are available

- EFEST has the Service n°2 Fixed Dialling Numbers (FDN) disabled

- EFFDN contains only two entries: eCall test number (123) and eCall reconfiguration number (345)

- EFSDN the last two entries contains two entries: eCall test number (456) and eCall reconfiguration number (678)

For test sequence 1.3:

- EFUST the services (Service n°2) Fixed Dialling Numbers (FDN), (Service n°4) Service Dialling Numbers (SDN), (Service n°99) URI support by UICC and (Service n°112) eCall Data over IMS are available.

- EFEST has the Service n°2 Fixed Dialling Numbers (FDN) enabled

- EFFDNURI contains only two entries: eCall test number (123) and eCall reconfiguration number (345)

- EFSDNURI the last two entries contains two entries: eCall test number (456) and eCall reconfiguration number (678)

PIN of the USIM is disabled.

Prior to this test the ME shall have been powered on, performed the PROFILE DOWNLOAD procedure and registered to the network.

27.22.12.4.2	Procedure

Expected Sequence 1.1 (REFRESH after change eCall mode, disable FDN in EFEST, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME ME E-USS | Set up an eCall | [Call is established using the number in EFFDN] |
| 2 | User ME | Call is terminated after a few seconds. |  |
| 3 | User ME | Set up a normal call to "54321" | Steps 3 – 4 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 4 | ME User | Call set up not allowed |  |
| 5 | E-USS ME | SMS-PP Data Download |  |
| 6 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 |  |
| 7 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 8 | ME E-USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 9 | UICC | EFEST contents state FDN is disabled | [New EFEST value: 00] |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.1.1 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 |  |
| 13 | ME UICC | TERMINAL RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B | [additional EFs read] |
| 14 | User ME ME E-USS | Set up an eCall | [Call is established using the number located in EFSDN] |
| 15 | User ME | Call is terminated after a few seconds. |  |
| 16 | User ME | Set up a normal call to "54321" | Steps 16 – 18 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 17 | ME E-USS | Setup | Call is established |
| 18 | User ME | Call is terminated after a few seconds |  |

SMS-PP (Data Download) Message 1.1.1

Logically:

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| Coding | 04 | 04 | 91 | 21 | 43 | 7F | 16 | 89 | 10 | 10 | 00 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 00 | 00 | 0D | 53 | 68 | 6F | 72 | 74 | 20 | 4D | 65 | 73 |
|  | 73 | 61 | 67 | 65 |  |  |  |  |  |  |  |  |

ENVELOPE: SMS-PP DOWNLOAD 1.1.1

Logically:

SMS-PP Download

Device identities

Source device:	Network

Destination device:	UICC

Address

TON	International number

NPI	"ISDN / telephone numbering plan"

Dialling number string	"112233445566778"

SMS TPDU

TP-MTI	SMS-DELIVER

TP-MMS	No more messages waiting for the MS in this SC

TP-RP	TP-Reply-Path is not set in this SMS-DELIVER

TP-UDHI	TP-UD field contains only the short message

TP-SRI	A status report will not be returned to the SME

TP-OA

TON	International number

NPI	"ISDN / telephone numbering plan"

Address value	"1234"

TP-PID	(U)SIM Data download

TP-DCS

Coding Group	General Data Coding

Compression	Text is uncompressed

Message Class	Class 2 (U)SIM Specific Message

Alphabet	8 bit data

TP-SCTS:	01/01/98 00:00:00 +0

TP-UDL	13

TP-UD	"Short Message"

Coding:

| BER-TLV: | D1 | 2D | 82 | 02 | 83 | 81 | 06 | 09 | 91 | 11 | 22 | 33 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 44 | 55 | 66 | 77 | F8 | 8B | 1C | 04 | 04 | 91 | 21 | 43 |
|  | 7F | 16 | 89 | 10 | 10 | 00 | 00 | 00 | 00 | 0D | 53 | 68 |
|  | 6F | 72 | 74 | 20 | 4D | 65 | 73 | 73 | 61 | 67 | 65 |  |

PROACTIVE COMMAND: REFRESH 1.1.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

File List:	EF EST

Coding:

| BER-TLV: | D0 | 12 | 81 | 03 | 01 | 01 | 02 | 82 | 02 | 81 | 82 | 92 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 07 | 01 | 3F | 00 | 7F | FF | 6F | 56 |  |  |  |  |

TERMINAL RESPONSE: REFRESH 1.1.1A

Logically:

Command details

Command number:        1

Command type:             REFRESH

Command qualifier:      USIM Initialization and File Change Notification

Device identities

Source device:               ME

Destination device:       UICC

Result

General Result:              Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.1.1B

Logically:

Command details

Command number:        1

Command type:             REFRESH

Command qualifier:      USIM Initialization and File Change Notification

Device identities

Source device:               ME

Destination device:       UICC

Result

General Result:              REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 02 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.2 (REFRESH after change eCall mode, enable FDN in EFEST, E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME ME E-USS | Set up an eCall | [Call is established and maintained using the number in EFSDN] |
| 2 | User ME | Call is terminated after a few seconds. |  |
| 3 | User ME | Set up a normal call to "54321" | Steps 3 – 5 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 4 | ME E-USS | Setup | Call is established |
| 5 | User ME | Call is terminated after a few seconds |  |
| 6 | E-USS ME | SMS-PP Data Download |  |
| 7 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 |  |
| 8 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 9 | ME E-USS | SMS-PP Data Download UICC Acknowledgement (RP-ACK) message. |  |
| 10 | UICC | EFEST contents state FDN is enabled | [New EFEST value: 01] |
| 11 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.2.1 |  |
| 12 | ME UICC | FETCH |  |
| 13 | ME UICC | USIM Initialization including send STATUS[P1='01'] | [ME performs USIM initialization in accordance with TS 31.111 [15] clause 6.4.7] |
| 14 | UICC ME | PROACTIVE COMMAND: REFRESH 1.2.1 |  |
| 15 | ME UICC | TERMINAL RESPONSE: REFRESH 1.2.1A or TERMINAL RESPONSE: REFRESH 1.2.1B | [additional EFs read] |
| 16 | ME E-USS | Set up an eCall | [Call is established using the number located in EFFDN] |
| 17 | User ME | Call is terminated after a few seconds. |  |
| 18 | User ME | Set up a normal call to "54321" | Steps 18 – 19 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 19 | ME User | Call set up not allowed |  |

PROACTIVE COMMAND: REFRESH 1.2.1

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and Full File Change Notification

Device identities

Source device:	UICC

Destination device:	ME

Coding:

| BER-TLV: | D0 | 09 | 81 | 03 | 01 | 01 | 00 | 82 | 02 | 81 | 82 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.2.1A

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and Full file Change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	Command performed successfully

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

TERMINAL RESPONSE: REFRESH 1.2.1B

Logically:

Command details

Command number:	1

Command type:	REFRESH

Command qualifier:	USIM Initialization and full File change Notification

Device identities

Source device:	ME

Destination device:	UICC

Result

General Result:	REFRESH performed with additional EFs read

Coding:

| BER-TLV: | 81 | 03 | 01 | 01 | 00 | 82 | 02 | 82 | 81 | 83 | 01 | 03 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Expected Sequence 1.3 (REFRESH after changing eCall mode, disable FDN in EFEST, IMS Emergency Services in E-UTRAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME ME E-USS | Set up an eCall | [Call is established using the number in EFFDNURI] |
| 2 | User ME | Call is terminated after a few seconds. |  |
| 3 | User ME | Set up a normal call to "54321" | Steps 3 – 4 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 4 | ME User | Call set up not allowed |  |
| 5 | E-USS ME | SMS-PP Data Download |  |
| 6 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 |  |
| 7 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 8 | ME E-USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 9 | UICC | EFEST contents state FDN is disabled | New EFEST value: 00 |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.1.1 |  |
| 11 | ME UICC | FETCH |  |
| 12 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 |  |
| 13 | ME UICC | TERMINAL RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B | [additional EFs read] |
| 14 | User ME ME E-USS | Set up an eCall | [Call is established using the number located in EFSDNURI] |
| 15 |  |  |  |
| 16 | User ME | Call is terminated after a few seconds. |  |
| 17 | User ME | Set up a normal call to "54321" | Steps 17 – 19 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 18 | ME E-USS | Setup | Call is established |
| 19 | User ME | Call is terminated after a few seconds |  |

Expected Sequence 1.4 (REFRESH after changing eCall mode, disable FDN in EFEST, UTRAN/GERAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME ME USS | Set up an eCall | [Call is established using the number in EFFDN] |
| 2 | User ME | Call is terminated after a few seconds |  |
| 3 | User ME | Set up a normal call to "54321" | Steps 3 – 4 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 4 | ME User | Call set up not allowed |  |
| 5 | USS ME | SMS-PP Data Download |  |
| 6 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 |  |
| 7 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 8 | ME USS | SMS-PP Data Download UICC acknowledgement (RP-ACK) message. |  |
| 9 | UICC | EFEST contents state FDN is disabled | [New EFEST value: 00] |
| 10 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.1.1 |  |
|  | ME UICC | FETCH |  |
| 11 |  |  |  |
| 12 | UICC ME | PROACTIVE COMMAND: REFRESH 1.1.1 |  |
| 13 | ME UICC | TERMINAL RESPONSE: REFRESH 1.1.1A or TERMINAL RESPONSE: REFRESH 1.1.1B | [additional EFs read] |
| 14 | User ME ME USS | Set up an eCall | [Call is established using the number located in EFSDN] |
| 15 | User ME | Call is terminated after a few seconds. |  |
| 16 | User ME | Set up a normal call to "54321" | Steps 16 – 18 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 17 | ME E-USS | Setup | Call is established |
| 18 | User ME | Call is terminated after a few seconds |  |

Expected Sequence 1.5 (REFRESH after change eCall mode, enable FDN in EFEST, UTRAN/GERAN)

| Step | Direction | MESSAGE / Action | Comments |
| --- | --- | --- | --- |
| 1 | User ME ME USS | Set up an eCall | [Call is established and maintained using the number in EFSDN.] |
| 2 | User ME | Call is terminated after a few seconds. |  |
| 3 | User ME | Set up a normal call to "54321" | Steps 3 – 5 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 4 | ME E-USS | Setup | Call is established |
| 5 | User ME | Call is terminated after a few seconds |  |
| 6 | USS ME | SMS-PP Data Download |  |
| 7 | ME UICC | ENVELOPE: SMS-PP DOWNLOAD 1.1.1 |  |
| 8 | UICC ME | SMS-PP Data Download UICC Acknowledgement | [SW '90 00'] |
| 9 | ME USS | SMS-PP Data Download UICC Acknowledgement (RP-ACK) message. |  |
| 10 | UICC | EFEST contents state FDN is enabled | [New EFEST value: 01] |
| 11 | UICC ME | PROACTIVE COMMAND PENDING: REFRESH 1.2.1 |  |
| 12 | ME UICC | FETCH |  |
| 13 | ME UICC | USIM Initialization including send STATUS[P1='01'] | [ME performs USIM initialization in accordance with TS 31.111 [15] clause 6.4.7] |
| 14 | UICC ME | PROACTIVE COMMAND: REFRESH 1.2.1 |  |
| 15 | ME UICC | TERMINAL RESPONSE: REFRESH 1.2.1A or TERMINAL RESPONSE: REFRESH 1.2.1B | [additional EFs read] |
| 16 | ME USS | Set up an eCall | [Call is established using the number located in EFFDN.] |
| 17 | User ME | Call is terminated after a few seconds. |  |
| 18 | User ME | Set up a normal call to "54321" | Steps 18 – 19 apply only if the ME supports A.1/87 AND A.1/85, else these steps should be skipped. |
| 19 | ME User | Call set up not allowed |  |

Annex A (normative):
Details of Test-SIM (TestSIM)

The TestSIM shall be able to present the following data:

ANSWER TO RESET

Logically:

TS (Initial character):	'3B'

T0 (Format character):	'86' (Following interface characters: TD(1), number of historical characters: 6)

TD1:	'00' (Following interface characters: none, Transfer protocol: T=0)

T1:	91

T2:	99

T3:	00

T4:	12

T5:	C1

T6:	00

Coding:

| Coding: | 3B | 86 | 00 | 91 | 99 | 00 | 12 | C1 | 00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

1.	For a successful outcome of the command "Select MasterFile" the TestSIM shall send SW1/SW2 "9F 1B".

2.	For a successful outcome of the command "Get Response with Length 1B" on the MasterFile the TestSIM shall respond:

RFU:	'00 00'

Not allocated memory:	'653 bytes'

File ID:	Master File

Type of file:	MF

RFU:	00 00 22 FF 01'

Length of following data:	14 bytes'

File characteristics:

Clock Stop:	Not allowed

Min. frequency for GSM algorithm:	13/8 MHz

Technology identification:	3V Technology SIM

CHV1:	disabled

DFs in current directory:	2

EFs in current directory:	8

Number of CHV and admin. Codes:	3

RFU byte 18:	00

CHV1 status:

False representations remaining:	3

RFU-bits 7-5:	000

Secret code:	Initialized

Unlock CHV1 status:

False representations remaining:	10

RFU-bits 7-5:	000

Secret code:	Initialized

CHV2 status:

False representations remaining:	3

RFU-bits 7-5:	000

Secret code:	Initialized

Unlock CHV2 status:

False representations remaining:	10

RFU-bits 7-5:	000

Secret code:	Initialized

RFU bytes 23:	00

Reserved for admin. management:	00 83 00 FF

Status Words

SW1 / SW2:	Normal ending of command

Coding:

| Coding | 00 | 00 | 02 | 8D | 3F | 00 | 01 | 00 | 00 | 22 | FF | 01 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0E | 9B | 02 | 08 | 03 | 00 | 83 | 8A | 83 | 8A | 00 | 00 |
|  | 83 | 00 | FF | 90 | 00 |  |  |  |  |  |  |  |

1.	For a successful outcome of the command "Select GSM" the TestSIM shall send SW1/SW2 "9F 1B".

2.	For a successful outcome of the command "Select PLMN" the TestSIM shall send SW1/SW2 "9F 0F".

3.	EFPLMN Information:

RFU-Bytes 1-2:	00 00

File size:	102 bytes

File ID:	6F30

Type of File:	Elementary file

Byte 8

RFU:	00

Access Condition:

UPDATE:	CHV1

READ/SEEK:	CHV1

RFU-bits 4-1:	1111

INCREASE:	NEVER

INVALIDATE:	NEVER

REHABILITATE:	NEVER

File Status:

Invalidation status:	File not invalidated

Readable/updateable:	Not readable/updatable when invalidated

RFU-bits 8-4, 2:	0000 0

Length of following data:	2 bytes

Structure:	Transparent

Length of record:	00

The initial coding of the EFPLMN shall be FF FF ... FF (logically: Empty).

Annex B (normative):
Details of terminal profile support

Table E.1: TERMINAL PROFILE support

| Item | Byte.bit | Terminal Profile | Ref. | Release | Status | Support | Mnemonic |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1.1 | Profile Download | TS 31.111 §5.2 | R99 | M |  | PD_Pro_Dvnl |
| 2 | 1.2 | SMS-PP data download | TS 31.111 §5.2 | R99 | C279 |  | PD_SMS_PP |
| 3 | 1.3 | Cell Broadcast data download | TS 31.111 §5.2 | R99 | C279 |  | PD_CB |
| 4 | 1.4 | Menu selection | TS 31.111 §5.2 | R99 | C267 AND C268 |  | PD_Menu_sel |
| 5 | 1.5 | Bit =1 if SMS-PP data Download supported | TS 31.111 §5.2 | R99 | C279 |  | PD_SMS_PP |
| 6 | 1.6 | Timer expiration | TS 31.111 §5. | R99 | M |  | PD_TExpir |
| 7 | 1.7 | Bit=1 if Call control supported | TS 31.111 §5.2. | R99 | C304 AND C279 |  | PD_CC |
| 8 | 1.8 | Bit=1 if Call control supported | TS 31.111 §5.2 | R99 | C304 AND C279 |  | PD_CC |
| 9 | 2.1 | Command result | TS 31.111 §5.2 | R99 | M |  | PD_Cmd_Res |
| 10 | 2.2 | Call Control by USIM | TS 31.111 §5.2 | R99 | C304 AND C279 |  | PD_CC |
| 11 | 2.3 | Bit=1 if Call control supported | TS 31.111 §5.2 | R99 | C304 AND C279 |  | PD_CC |
| 12 | 2.4 | MO short message control by USIM | TS 31.111 §5.2 | R99 | C279 |  | PD_MO_SMS_CC |
| 13 | 2.5 | Bit=1 if Call control supported | TS 31.111 §5.2 | R99 | C304 AND C279 |  | PD_CC |
| 14 | 2.6 | UCS2 Entry supported | TS 31.111 §5.2 | R99 | C203 AND C268 |  | PD_UCS2_entry |
| 15 | 2.7 | UCS2 Display supported | TS 31.111 §5.2 | R99 | C204 AND C267 |  | PD_UCS2_Display |
| 16 | 2.8 | Bit=1 if Display Text supported | TS 31.111 §5.2 | R99 | C267 |  | PD_Display_Text |
| 17 | 3.1 | DISPLAY TEXT | TS 31.111 §5.2 | R99 | C267 |  | PD_Display_Text |
| 18 | 3.2 | GET INKEY | TS 31.111 §5.2 | R99 | C267 AND C268 |  | PD_Get_Inkey |
| 19 | 3.3 | GET INPUT | TS 31.111 §5.2 | R99 | C267 AND C268 |  | PD_Get_Input |
| 20 | 3.4 | MORE TIME | TS 31.111 §5.2 | R99 | M |  | PD_More_Time |
| 21 | 3.5 | PLAY TONE | TS 31.111 §5.2 TS 11.14, 5 | R99 | C269 |  | PD_Play_Tone |
| 22 | 3.6 | POLL INTERVAL | TS 31.111 §5.2 TS 11.14, 5 | R99 | M |  | PD_Poll_interval |
| 23 | 3.7 | POLLING OFF | TS 31.111 §5.2 | R99 | M |  | PD_Polling_Off |
| 24 | 3.8 | REFRESH | TS 31.111 §5.2 | R99 | M |  | PD_Refresh |
| 25 | 4.1 | SELECT ITEM | TS 31.111 §5.2 | R99 | C267 AND C268 |  | PD_Select_Item |
| 26 | 4.2 | SEND SHORT MESSAGE | TS 31.111 §5.2 | R99 | C279 |  | PD_Send_SMS |
| 27 | 4.3 | SEND SS | TS 31.111 §5.2 | R99 | C279 |  | PD_Send_SS |
| 28 | 4.4 | SEND USSD | TS 31.111 §5.2 | R99 | C279 |  | PD_Send_USSD |
| 29 | 4.5 | SET UP CALL | TS 31.111 §5.2 | R99 | C291 |  | PD_SetUp_Call |
| 30 | 4.6 | SET UP MENU | TS 31.111 §5.2 | R99 | C267 AND C268 |  | PD_SetUp_Menu |
| 31 | 4.7 | PROVIDE LOCAL INFORMATION (LOCI & IMEI) | TS 31.111 §5.2 | R99 | M |  | PD_Provide_Local |
| 32 | 4.8 | PROVIDE LOCAL INFORMATION (NMR) | TS 31.111 §5.2 | R99 | C279 |  | PD_Provide_Local_NMR |
| 33 | 5.1 | SET UP EVENT LIST | TS 31.111 §5.2 | R99 | M |  | PD_Setup_Evt_List |
| 34 | 5.2 | Event: MT call | TS 31.111 §5.2 | R99 | C270 AND C279 |  | PD_MT_Call |
| 35 | 5.3 | Event: Call connected | TS 31.111 §5.2 | R99 | C270 AND C279 |  | PD_Call_Conn |
| 36 | 5.4 | Event: Call disconnected | TS 31.111 §5.2 | R99 | C270 AND C279 |  | PD_Call_Disc |
| 37 | 5.5 | Event: Location status | TS 31.111 §5.2 | R99 | M |  | PD_Loc_Status |
| 38 | 5.6 | Event: User activity | TS 31.111 §5.2 | R99 | C268 |  | PD_User_Act |
| 39 | 5.7 | Event: Idle screen available | TS 31.111 §5.2 | R99 | C267 |  | PD_Idle_Scr_Avail |
| 40 | 5.8 | Event: Card reader status | TS 31.111 §5.2 | R99 | C206 |  | PD_Evt_Rdr_Status |
| 41 | 6.1 | Event: Language selection | TS 31.111 §5.2 | R99 | C271 |  | PD_Lang_Select |
| 42 | 6.2 | Event: Browser Termination | TS 31.111 §5.2 | R99 | C212 AND C267 AND C268 |  | PD_Browser_Term |
| 43 | 6.3 | Event: Data available | TS 31.111 §5.2 | R99 | C223 |  | PD_Data_Avail |
| 44 | 6.4 | Event: Channel status | TS 31.111 §5.2 | R99 | C223 |  | PD_Evt_Ch_Status |
| 45 | 6.5 | Event:Access Technology Change | TS 31.111 §5.2 | Rel-4 | C306 |  | PD_Evt_ATC |
| 46 | 6.6 | Event: Display Parameters Changed | TS 31.111 §5.2 | Rel-4 | C218 AND C267 |  | PD_Disp_Resiz |
| 47 | 6.7 | Event: Local Connection | TS 31.111 §5.2 | Rel-4 | C224 |  | PD_Evt_LC |
| 48 | 6.8 | Event: Network Search Mode Change | TS 31.111 §5.2 | Rel-6 | M |  | PD_Evt_NSMC |
| 49 | 7.1 | POWER ON CARD | TS 31.111 §5.2 | R99 | C206 |  | PD_C_On |
| 50 | 7.2 | POWER OFF CARD | TS 31.111 §5.2 | R99 | C206 |  | PD_C_Off |
| 51 | 7.3 | PERFORM CARD APDU | TS 31.111 §5.2 | R99 | C206 |  | PD_C_APDU |
| 52 | 7.4 | GET READER STATUS (Card reader status) | TS 31.111 §5.2 | R99 | C206 |  | PD_Get_Rdr_Status |
| 53 | 7.5 | GET READER STATUS (Card reader identifier) | TS 31.111 §5.2 | R99 | C208 |  | PD_Get_Rdr_Id |
| 54 | 7.6 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_54 |
| 55 | 7.7 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_55 |
| 56 | 7.8 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_56 |
| 57 | 8.1 | TIMER MANAGEMENT (start, stop) | TS 31.111 §5.2 | R99 | M |  | PD_Timer_Mgt_Start_Stop |
| 58 | 8.2 | TIMER MANAGEMENT (get current value) | TS 31.111 §5.2 | R99 | M |  | PD_Timer_Val |
| 59 | 8.3 | PROVIDE LOCAL INFORMATION (date, time and time zone) | TS 31.111 §5.2 | R99 | M |  | PD_Provide_Local_D_Time |
| 60 | 8.4 | Bit=1 if Get Inkey | TS 31.111 §5.2 | R99 | C268 |  | PD_Get_Inkey |
| 61 | 8.5 | SET UP IDLE MODE TEXT | TS 31.111 §5.2 | R99 | C267 |  | PD_Stup_Id_Mod_Txt |
| 62 | 8.6 | RUN AT COMMAND (i.e. class "b" is supported) | TS 31.111 §5.2 | R99 | C209 |  | PD_Run_AT |
| 63 | 8.7 | Bit=1 if Set UpCall | TS 31.111 §5.2 | R99 | C267 AND C268 AND C270 |  | PD_SetUp_Call |
| 64 | 8.8 | Bit=1 if Call Control | TS 31.111 §5.2 | R99 | C304 AND C279 |  | PD_CC |
| 65 | 9.1 | Bit=1 if Display Text | TS 31.111 §5.2 | R99 | C267 |  | PD_Display_Text |
| 66 | 9.2 | SEND DTMF command | TS 31.111 §5.2 | R99 | C270 AND C279 |  | PD_Send_DTMF |
| 67 | 9.3 | Bit = 1 if Provide Local Information (NMR) supported | TS 31.111 §5.2 | R99 | C279 |  | PD_Provide_Local |
| 68 | 9.4 | PROVIDE LOCAL INFORMATION (language) | TS 31.111 §5.2 | R99 | C292 |  | PD_Provide_Local_LS |
| 69 | 9.5 | PROVIDE LOCAL INFORMATION (Timing Advance) | TS 31.111 §5.2 | R99 | C280 |  | PD_Provide_Local_TA |
| 70 | 9.6 | LANGUAGE NOTIFICATION | TS 31.111 §5.2 | R99 | C293 |  | PD_Lang_Notif |
| 71 | 9.7 | LAUNCH BROWSER | TS 31.111 §5.2 | R99 | C212 AND C267 AND C268 |  | PD_Launch_Brws |
| 72 | 9.8 | PROVIDE LOCAL INFORMATION (Access Technology) | TS 31.111 §5.2 | Rel-4 | M |  | PD_Provide_Local_AT |
| 73 | 10.1 | Soft keys support for SELECT ITEM | TS 31.111 §5.2 | R99 | C213 |  | PD_Softkey_Select_Item |
| 74 | 10.2 | Soft Keys support for SET UP MENU | TS 31.111 §5.2 | R99 | C213 |  | PD_Softkey_SetUp _Menu |
| 75 | 10.3 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_75 |
| 76 | 10.4 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_76 |
| 77 | 10.5 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_77 |
| 78 | 10.6 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_78 |
| 79 | 10.7 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_79 |
| 80 | 10.8 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_80 |
| 81 | 11.1 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 82 | 11.2 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 83 | 11.3 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 84 | 11.4 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 85 | 11.5 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 86 | 11.6 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 87 | 11.7 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 88 | 11.8 | Maximum number of soft keys available ('FF' = RFU) | TS 31.111 §5.2 | R99 | C214 |  | PD_Max_SoftKey |
| 89 | 12.1 | OPEN CHANNEL | TS 31.111 §5.2 | R99 | C223 |  | PD_Open_Ch |
| 90 | 12.2 | CLOSE CHANNEL | TS 31.111 §5.2 | R99 | C223 |  | PD_Close_Ch |
| 91 | 12.3 | RECEIVE DATA | TS 31.111 §5.2 | R99 | C223 |  | PD_Rx_Data |
| 92 | 12.4 | SEND DATA | TS 31.111 §5.2 | R99 | C223 |  | PD_Send_Data |
| 93 | 12.5 | GET CHANNEL STATUS | TS 31.111 §5.2 | R99 | C223 |  | PD_Get_Ch_Status |
| 94 | 12.6 | SERVICE SEARCH | TS 31.111 §5.2 | Rel-4 | C224 |  | PD_Serv_Search |
| 95 | 12.7 | GET SERVICE INFORMATION | TS 31.111 §5.2 | Rel-4 | C224 |  | PD_Get_Serv_Info |
| 96 | 12.8 | DECLARE SERVICE | TS 31.111 §5.2 | Rel-4 | C224 |  | PD_Declare_Serv |
| 97 | 13.1 | CSD supported by ME | TS 31.111 §5.2 | R99 | C207 |  | PD_CSD |
| 98 | 13.2 | GPRS supported by ME | TS 31.111 §5.2 | R99 | C222 |  | PD_GPRS |
| 99 | 13.3 | Bluetooth supported by terminal | TS 31.111 §5.2 | Rel-4 | C225 |  | PD_BT |
| 100 | 13.4 | IrDA Supported by terminal | TS 31.111 §5.2 | Rel-4 | C226 |  | PD_IrDA |
| 101 | 13.5 | RS232 Supported by terminal | TS 31.111 §5.2 | Rel-4 | C227 |  | PD_RS232 |
| 102 | 13.6 | Number of channels supported by ME | TS 31.111 §5.2 | R99 | C257 |  | PD_Nb_Channel |
| 103 | 13.7 | Number of channels supported by ME | TS 31.111 §5.2 | R99 | C257 |  | PD_Nb_Channel |
| 104 | 13.8 | Number of channels supported by ME | TS 31.111 §5.2 | R99 | C257 |  | PD_Nb_Channel |
| 105 | 14.1 | Number of characters supported down the ME | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char |
| 106 | 14.2 | Number of characters supported down the ME | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char |
| 107 | 14.3 | Number of characters supported down the ME | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char |
| 108 | 14.4 | Number of characters supported down the ME | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char |
| 109 | 14.5 | Number of characters supported down the ME | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char |
| 110 | 14.6 | No display capability (i.e class "ND" is indicated) | TS 31.111 §5.2 | Rel-8 | C276 |  | PD_Type_ND |
| 111 | 14.7 | No keypad available (i.e. class "NK" is indicated) | TS 31.111 §5.2 | Rel-8 | C277 |  | PD_Type_NK |
| 112 | 14.8 | Screen Sizing Parameters | TS 31.111 §5.2 | R99 | C216 |  | PD_Screen_Siz |
| 113 | 15.1 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 114 | 15.2 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 115 | 15.3 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 116 | 15.4 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 117 | 15.5 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 118 | 15.6 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 119 | 15.7 | Number of characters supported across the ME display | TS 31.111 §5.2 | R99 | C274 |  | PD_Nb_Char_Disp |
| 120 | 15.8 | Variable size fonts Supported | TS 31.111 §5.2 | R99 | C274 |  | PD_Var_Font |
| 121 | 16.1 | Display can be resized | TS 31.111 §5.2 | R99 | C218 |  | PD_Disp_Resiz |
| 122 | 16.2 | Text Wrapping supported | TS 31.111 §5.2 | R99 | C273 |  | PD_Txt_Wrap |
| 123 | 16.3 | Text Scrolling supported | TS 31.111 §5.2 | R99 | C273 |  | PD_Txt_Scroll |
| 124 | 16.4 | Text attributes supported | TS 31.111 §5.2 | Rel-5 | C228 |  | PD_Text_Attrib |
| 125 | 16.5 | RFU | TS 11.14, 5 | R96 | X |  | PD_RFU_125 |
| 126 | 16.6 | Width reduction when in a menu | TS 31.111 §5.2 | R99 | C274 |  | PD_Width_Reduc |
| 127 | 16.7 | Width reduction when in a menu | TS 31.111 §5.2 | R99 | C274 |  | PD_Width_Reduc |
| 128 | 16.8 | Width reduction when in a menu | TS 31.111 §5.2 | R99 | C274 |  | PD_Width_Reduc |
| 129 | 17.1 | TCP, UICC in client mode | TS 31.111 §5.2 | R99 | C220 |  | PD_TCP |
| 130 | 17.2 | UDP, UICC in client mode | TS 31.111 §5.2 | R99 | C221 |  | PD_UDP |
| 131 | 17.3 | TCP, UICC in server mode (i.e. class "k" is supported) | TS 31.111 §5.2 | Rel-7 | C262 |  | PD_TCP_UICC_ServerMode |
| 132 | 17.4 | TCP, Terminal in server mode (i.e. class "k" is supported) | TS 31.111 §5.2 | Rel-7 | C263 |  | PD_TCP_Terminal_ServerMode |
| 133 | 17.5 | UDP, Terminal in server mode (i.e. class "k" is supported) | TS 31.111 §5.2 | Rel-7 | C264 |  | PD_UDP_Terminal_ServerMode |
| 134 | 17.6 | Direct communication channel (i.e. class "k" is supported) | TS 31.111 §5.2 | Rel-10 | C284 |  | Direct_Com_Channel |
| 135 | 17.7 | E- UTRAN (i.e. if class "e" is supported) | TS 31.111 §5.2 | Rel-8 | C275 |  | PD_E_UTRAN |
| 136 | 17.8 | HSDPA supported by ME | TS 31.111 §5.2 | Rel-6 | C258 |  | PD_ HSDPA |
| 137 | 18.1 | DISPLAY TEXT (Variable time out) | TS 31.111 §5.2 | Rel-4 | C229 |  | PD_Disp_Var_Timeout |
| 138 | 18.2 | GET INKEY (help is supported while waiting for immediate response or variable time out) | TS 31.111 §5.2 | Rel-4 | C231 |  | PD_Get_Inkey_Help |
| 139 | 18.3 | USB (Bearer Independent protocol supported bearers, class "e") | TS 31.111 §5.2 | Rel-4 | C232 |  | PD_USB |
| 140 | 18.4 | GET INKEY (Variable time out) | TS 31.111 §5.2 | Rel-4 | C229 AND C267 AND C268 |  | PD_Get_Inkey_Var_Timeout |
| 141 | 18.5 | Reserved for 3GPP2: PROVIDE LOCAL INFORMATION (ESN) | TS 31.111 §5.2 | R99 | O |  | Reserved |
| 142 | 18.6 | CALL CONTROL on GPRS | TS 31.111 §5.2 | Rel-5 | C242 |  | PD_CC_GPRS |
| 143 | 18.7 | PROVIDE LOCAL INFORMATION (IMEISV) | TS 31.111 §5.2 | Rel-6 | M |  | PD_Provide_Local_SV |
| 144 | 18.8 | PROVIDE LOCAL INFORMATION (search mode change) | TS 31.111 §5.2 | Rel-6 | M |  | PD_Provide_Local_SMC |
| 145 | 19.1 | Protocol Version | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 146 | 19.2 | Protocol Version | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 147 | 19.3 | Protocol Version | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 148 | 19.4 | Protocol Version | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 149 | 19.5 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_149 |
| 150 | 19.6 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_150 |
| 151 | 19.7 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_151 |
| 152 | 19.8 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_152 |
| 153 | 20.1 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 154 | 20.2 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 155 | 20.3 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 156 | 20.4 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 157 | 20.5 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 158 | 20.6 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 159 | 20.7 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 160 | 20.8 | Reserved by TIA/EIA/IS-820 [25] | TS 31.111 §5.2 | R99 | X |  | Reserved |
| 161 | 21.1 | WML browser supported | TS 31.111 §5.2 | Rel-6 | C233 AND C267 |  | PD_WML |
| 162 | 21.2 | XHTML browser supported | TS 31.111 §5.2 | Rel-6 | C234 AND C267 |  | PD_XHTML |
| 163 | 21.3 | HTML browser supported | TS 31.111 §5.2 | Rel-6 | C235 AND C267 |  | PD_HTML |
| 164 | 21.4 | CHTML browser supported | TS 31.111 §5.2 | Rel-6 | C236 AND C267 |  | PD_CHTML |
| 165 | 21.5 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_165 |
| 166 | 21.6 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_166 |
| 167 | 21.7 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_167 |
| 168 | 21.8 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_168 |
| 169 | 22.1 | Support of UTRAN PS with extended parameters | TS 31.111 §5.2 | Rel-6 | C259 |  | PD_UTRAN_PS_Ext_Param |
| 170 | 22.2 | PROVIDE LOCAL INFORMATION (Battery state) if class "g" supported | TS 31.111 §5.2 | Rel-6 | C239 |  | PD_Provide_Local_Batt |
| 171 | 22.3 | PLAY TONE (Melody tones & themed tones supported) | TS 31.111 §5.2 | Rel-6 | C241 |  | PD_M_T_Tones |
| 172 | 22.4 | Multi-media in SET UP CALL supported (if class "h" supported) | TS 31.111 §5.2 | Rel-6 | C240 |  | PD_Xmedia_Call |
| 173 | 22.5 | Toolkit-initiated GBA | TS 31.111 §5.2 | Rel-6 | C266 |  | PD_Toolkit_GBA |
| 174 | 22.6 | RETRIEVE MULTIMEDIA MESSAGE, (if class "j" is supported) | TS 31.111 §5.2 | Rel-6 | C238 |  | PD_Retrieve_MMS |
| 175 | 22.7 | SUBMIT MULTIMEDIA MESSAGE, (if class "j" is supported) | TS 31.111 §5.2 | Rel-6 | C238 |  | PD_Submit_MMS |
| 176 | 22.8 | DISPLAY MULTIMEDIA MESSAGE, (if class "j" is supported) | TS 31.111 §5.2 | Rel-6 | C238 AND C267 |  | PD_Display_MMS |
| 177 | 23.1 | SET FRAMES supported (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C237 |  | PD_Set_Frames |
| 178 | 23.2 | GET FRAMES STATUS supported (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C237 |  | PD_Get_Frames_Stat |
| 179 | 23.3 | MMS notification download (if class "j" is supported) | TS 31.111 §5.2 | Rel-6 | C238 |  | PD_MMS_Notification |
| 180 | 23.4 | Alpha Identifier in REFRESH command supported by terminal | TS 31.111 §5.2 | Rel-7 | C294 |  | PD_Refresh_AlphaIdentifier |
| 181 | 23.5 | Geographical Location Reporting (if class "n" is supported) | TS 31.111 §5.2 | Rel-8 | C265 |  | PD_Geo_Loaction_Reporting |
| 182 | 23.6 | Reserved for 3GPP2: PROVIDE LOCAL INFORMATION (MEID) | TS 31.111 §5.2 | Rel-6 | O |  | Reserved |
| 183 | 23.7 | PROVIDE LOCAL INFORMATION (NMR (UTRAN/E-UTRAN)) | TS 31.111 §5.2 | Rel-6 | C278 |  | PD_Provide_Local_NMR |
| 184 | 23.8 | USSD Data Download and application mode | TS 31.111 §5.2 | Rel-6 | C272 |  | PD_USSD_DD |
| 185 | 24.1 | Maximum number of frames supported (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C256 |  | PD_Max_Frames |
| 186 | 24.2 | Maximum number of frames supported (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C256 |  | PD_Max_Frames |
| 187 | 24.3 | Maximum number of frames supported (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C256 |  | PD_Max_Frames |
| 188 | 24.4 | Maximum number of frames supported (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C256 |  | PD_Max_Frames |
| 189 | 24.5 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_189 |
| 190 | 24.6 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_190 |
| 191 | 24.7 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_191 |
| 192 | 24.8 | RFU | TS 31.111 §5.2 | R99 | X |  | PD_RFU_192 |
| 193 | 25.1 | Event: browsing status | TS 31.111 §5.2 | Rel-6 | C212 AND C267 AND C268 |  | PD_Browser_Stat |
| 194 | 25.2 | Event: MMS Transfer status (if class "j" is supported) | TS 31.111 §5.2 | Rel-6 | C238 |  | PD_MMS |
| 195 | 25.3 | Event Frame parameters changed (if class "i" supported) | TS 31.111 §5.2 | Rel-6 | C237 |  | PD_Event_Frames |
| 196 | 25.4 | Event: I-WLAN Access status (if class "e" is supported) | TS 31.111 §5.2 | Rel-7 | C260 |  | PD_RFU_Event_I-WLAN |
| 197 | 25.5 | Event: Network Rejection | TS 31.111 §5.2 | Rel-8 | C279 |  | PD_Event_NW_Rejection |
| 198 | 25.6 | Reserved by ETSI | TS 31.111 §5.2 | Rel-7 | O |  | PD_Reserved |
| 199 | 25.7 | Event: Network Rejection for E-UTRAN | TS 31.111 §5.2 | Rel-8 | C283 |  | PD_ Event_NW_Rejection_E_UTRAN |
| 200 | 25.8 | Multiple access technologies supported in Event Access Technology Change and Provide Local Information | TS 31.111 §5.2 | Rel-8 | O |  | PD_Multiple_ACT |
| 201 | 26.1 | Event : CSG Cell Selection (if class "q" is supported) | TS 31.111 §5.2 | Rel-9 | C281 |  | PD_Event_CSG_Cell_Selection |
| 202 | 26.2 | Reserved by ETSI | TS 31.111 §5.2 | Rel-9 | O |  | PD_Reserved |
| 203 | 26.3 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_203 |
| 204 | 26.4 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_204 |
| 205 | 26.5 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_205 |
| 206 | 26.6 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_206 |
| 207 | 26.7 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_207 |
| 208 | 26.8 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_208 |
| 209 | 27.1 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_209 |
| 210 | 27.2 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_210 |
| 211 | 27.3 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_211 |
| 212 | 27.4 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_212 |
| 213 | 27.5 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_213 |
| 214 | 27.6 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_214 |
| 215 | 27.7 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_215 |
| 216 | 27.8 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_216 |
| 217 | 28.1 | Alignment left supported | TS 31.111 §5.2 | Rel-5 | C243 |  | PD_Text_Attrib_Left |
| 218 | 28.2 | Alignment center supported | TS 31.111 §5.2 | Rel-5 | C244 |  | PD_Text_Attrib_Center |
| 219 | 28.3 | Alignment right supported | TS 31.111 §5.2 | Rel-5 | C245 |  | PD_Text_Attrib_Right |
| 220 | 28.4 | Font size normal supported | TS 31.111 §5.2 | Rel-5 | C246 |  | PD_Text_Attrib_Normal |
| 221 | 28.5 | Font size large supported | TS 31.111 §5.2 | Rel-5 | C247 |  | PD_Text_Attrib_Large |
| 222 | 28.6 | Font size small supported | TS 31.111 §5.2 | Rel-5 | C248 |  | PD_Text_Attrib_Small |
| 223 | 28.7 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_223 |
| 224 | 28.8 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_224 |
| 225 | 29.1 | Style normal supported | TS 31.111 §5.2 | Rel-5 | C249 |  | PD_Text_Attrib_Styl_Norm |
| 226 | 29.2 | Style bold supported | TS 31.111 §5.2 | Rel-5 | C250 |  | PD_Text_Attrib_Styl_Bold |
| 227 | 29.3 | Style italic supported | TS 31.111 §5.2 | Rel-5 | C251 |  | PD_Text_Attrib_Styl_Italic |
| 228 | 29.4 | Style underlined supported | TS 31.111 §5.2 | Rel-5 | C252 |  | PD_Text_Attrib_Styl_Underl |
| 229 | 29.5 | Style strikethrough supported | TS 31.111 §5.2 | Rel-5 | C253 |  | PD_Text_Attrib_Styl_Strik |
| 230 | 29.6 | Style text foreground colour supported | TS 31.111 §5.2 | Rel-5 | C254 |  | PD_Text_Attrib_Styl_Text_Fore |
| 231 | 29.7 | Style text background colour supported | TS 31.111 §5.2 | Rel-5 | C255 |  | PD_Text_Attrib_Styl_Text_Back |
| 232 | 29.8 | RFU | TS 31.111 §5.2 | Rel-6 | X |  | PD_RFU_224 |
| 233 | 30.1 | I-WLAN bearer support (if class "e" is supported) | TS 31.111 §5.2 | Rel-7 | C260 |  | PD_I-WLAN |
| 234 | 30.2 | Proactive UICC: PROVIDE LOCAL INFORMATION (WSID of the current I-WLAN connection) | TS 31.111 §5.2 | Rel-7 | C260 |  | PD_Provide_Local_WSID_WLAN |
| 235 | 30.3 | TERMINAL APPLICATIONS (i.e. class "k" is supported) | TS 31.111 §5.2 | Rel-7 | C261 |  | PD_Terminal_Applications |
| 236 | 30.4 | "Steering of Roaming" REFRESH support | TS 31.111 §5.2 | Rel-7 | M |  | PD_Steering_Of_Roaming |
| 237 | 30.5 | Reserved by ETSI | TS 31.111 §5.2 | Rel-7 | O |  | PD_Reserved |
| 238 | 30.6 | Proactive UICC: Geographical Location Request (if class "n" is supported) | TS 31.111 §5.2 | Rel-8 | C265 |  | PD_Geo_Loaction_Request |
| 239 | 30.7 | Reserved by ETSI | TS 31.111 §5.2 | Rel-8 | O |  | PD_Reserved |
| 240 | 30.8 | "Steering of Roaming for I-WLAN" REFRESH support | TS 31.111 §5.2 | Rel-8 | C260 |  | PD_Steering_Of_Roaming _I-WLAN |
| - 241 | 31.1 | Reserved by ETSI | TS 31.111 §5.2 | Rel-9 | O |  | PD_Reserved |
| 242 | 31.2 | Support of CSG cell discovery (if class "q" is supported) | TS 31.111 §5.2 | Rel-9 | C282 |  | PS_CSG_Cell_Discovery |
| 243 | 31.3 | Confirmation parameters supported for OPEN CHANNEL in Terminal Server Mode | TS 31.111 §5.2 | Rel-9 | C285 |  | PD_Open_Channel_Conf_Parameters |
| 244 | 31.4 | Communication Control for IMS | TS 31.111 §5.2 | Rel-10 | C286 |  | PD_IMS_COMMUNICATION_CONTROL |
| 245 | 31.5 | Support of CAT over the modem interface (if class "s" is supported) | TS 31.111 §5.2 | Rel-10 | C287 |  | PD_CAT_Modem_Interface |
| 246 | 31.6 | Support for Incoming IMS Data event (if classes "e" and "t" are supported) | TS 31.111 §5.2 | Rel-10 | C288 |  | PD_Incoming_IMS_Data_Event |
| 247 | 31.7 | Support for IMS Registration event (if classes "e" and "t" are supported) | TS 31.111 §5.2 | Rel-10 | C289 |  | PD_IMS_Reg_Event |
| 248 | 31.8 | Reserved by ETSI | TS 31.111 §5.2 | Rel-10 | O |  | PD_Reserved |
| 249 | 32.1 | IMS support (if class "e" and "t" are supported) | TS 31.111 §5.2 | Rel-10 | C290 |  | PD_UICC_ACCESS_IMS |
| 250 | 32.2 | PROVIDE LOCATION INFORMATION, H(e)NB IP address support (if class "v" is supported)) | TS 31.111 §5.2 | Rel-11 | X |  | PD_PLI_HENB_IP_Adress_support |
| 251 | 32.3 | PROVIDE LOCATION INFORMATION, H(e)NB surrounding macrocells support (if class "w" is supported) | TS 31.111 §5.2 | Rel-11 | X |  | PD_PLI_HENB_surrounding_Macrocell |
| 252 | 32.4 | Reserved by ETSI | TS 31.111 §5.2 | Rel-11 | O |  | PD_Reserved |
| 253 | 32.5 | Reserved by ETSI | TS 31.111 §5.2 | Rel-11 | O |  | PD_Reserved |
| 254 | 32.6 | Reserved by ETSI | TS 31.111 §5.2 | Rel-11 | O |  | PD_Reserved |
| 255 | 32.7 | Reserved by ETSI | TS 31.111 §5.2 | Rel-11 | O |  | PD_Reserved |
| 256 | 32.8 | Reserved by ETSI (Support of refresh enforcement policy) | TS 31.111 §5.2 | Rel-12 | M |  | PD_ Refresh_Enforcement_Policy |
| 257 | 33.1 | Reserved by ETSI | TS 31.111 §5.2 | Rel-12 | O |  | PD_Reserved |
| 258 | 33.2 | Reserved by ETSI | TS 31.111 §5.2 | Rel-12 | O |  | PD_Reserved |
| 259 | 33.3 | Reserved by ETSI | TS 31.111 §5.2 | Rel-12 | O |  | PD_Reserved |
| 260 | 33.4 | ProSe usage information reporting (used only if class "e" is supported) | TS 31.111 §5.2 | Rel-12 | C295 |  | PD_ProSE |
| 261 | 33.5 | Reserved by ETSI | TS 31.111 §5.2 | Rel-12 | O |  | PD_Reserved |
| 262 | 33.6 | Event: WLAN Access status (if class "e" is supported) | TS 31.111 §5.2 | Rel-13 | C296 |  | PD_WLAN_Access_St |
| 263 | 33.7 | WLAN bearer support (if class "e" is supported) | TS 31.111 §5.2 | Rel-13 | C297 |  | PD_WLAN_Bearer |
| 264 | 33.8 | Proactive UICC: PROVIDE LOCAL INFORMATION (WLAN identifier of the current WLAN connection) | TS 31.111 §5.2 | Rel-13 | C298 |  | PD_ Provide_Local_WLAN_ID |
| 265 | 34.1 | URI support for SEND SHORT MESSAGE | TS 31.111 §5.2 | Rel-13 | C299 |  | PD_URI_Send_Short_IMS |
| 266 | 34.2 | IMS URI supported for SET UP CALL | TS 31.111 §5.2 | Rel-13 | C300 |  | PD_IMS_URI_Setup_call |
| 267 | 34.3 | Media Type "Voice" supported for SET UP CALL and Call Control by USIM | TS 31.111 §5.2 | Rel-13 | C301 |  | PD_Voice_Media_ USIM |
| 268 | 34.4 | Media Type "Video" supported for SET UP CALL and Call Control by USIM | TS 31.111 §5.2 | Rel-13 | C302 |  | PD_Video_Media_USIM |
| 269 | 34.5 | Proactive UICC: PROVIDE LOCAL INFORMATION (E-UTRAN Timing Advance Information) | TS 31.111 §5.2 | Rel-13 | C283 |  | PD_ Provide_Local_EUTRAN_TA |
| 270 | 34.6 | Reserved by ETSI | TS 31.111 §5.2 | Rel-13 | O |  | PD_Reserved |
| 271 | 34.7 | Extended Rejection Cause Code in Event: Network Rejection for E-UTRAN | TS 31.111 §5.2 | Rel-14 | O |  | PD_EUTRAN_Extended_Reject_Cause_Code |
| 272 | 34.8 | Reserved by ETSI | TS 31.111 §5.2 | Rel-13 | O |  | PD_Reserved |
| 273 | 35.1 | Reserved by ETSI | TS 31.111 §5.2 | Rel-14 | O |  | PD_Reserved |
| 274 | 35.2 | Data Connection Status Change Event support – PDP Connection (if class "e" is supported) | TS 31.111 §5.2 | Rel-14 | C305 |  | PD_Data_Connection_Status_Change_PDP |
| 275 | 35.3 | Data Connection Status Change Event support – PDN Connection (if class "e" is supported) | TS 31.111 §5.2 | Rel-14 | C283 |  | PD_Data_Connection_Status_Change_PDN |
| 276 | 35.4 | Reserved by ETSI | TS 31.111 §5.2 | Rel-14 | O |  | PD_Reserved |
| 277 | 35.5 | Reserved by ETSI | TS 31.111 §5.2 | Rel-14 | O |  | PD_Reserved |
| 278 | 35.6 | Reserved by ETSI | TS 31.111 §5.2 | Rel-14 | O |  | PD_Reserved |
| 279 | 35.7 | Reserved by ETSI | TS 31.111 §5.2 | Rel-14 | O |  | PD_Reserved |
| 280 | 35.8 | Reserved by ETSI | TS 31.111 §5.2 | Rel-14 | O |  | PD_Reserved |

| C201 | [void] | -- [void] |
| --- | --- | --- |
| C202 | [void] | -- [void] |
| C203 | IF A.1/3 THEN M ELSE O.1 | -- O_Ucs2_Entry |
| C204 | IF A.1/15 THEN M ELSE O.1 | -- O_Ucs2_Disp |
| C205 | [void] | -- [void] |
| C206 | IF A.1/7 THEN M ELSE O | -- O_Dual_Slot |
| C207 | IF A.1/12 THEN M ELSE O.1 | -- O_BIP_CSD |
| C208 | IF (A.1/7 AND A.1/8) THEN M ELSE O.1 | -- O_Dual_Slot AND O_Detach_Rdr |
| C209 | IF A.1/9 THEN M ELSE O.1 | -- O_Run_At |
| C210 | [void] | -- [void] |
| C211 | [void] | -- [void] |
| C212 | IF A.1/10 THEN M ELSE O | -- O_LB |
| C213 | IF (A.1/11 AND A.1/85) THEN M for at least one of the bits 1 - 2 of byte 10 | -- O_Softkey AND O_No_Type_NK |
| C214 | IF C213 THEN M for at least one, but not for all of the bits 1 - 8 of byte 11 | -- O_Softkey AND O_No_Type_NK (parameters) |
| C215 | Void | -- Void |
| C216 | IF (A.1/13 AND A.1/84) THEN M ELSE O.1 | -- O_Scr_Siz AND O_No_Type_ND |
| C217 | Void | -- Void |
| C218 | IF (A.1/14 AND A.1/84) THEN M ELSE O.1 | -- O_Scr_Resiz AND O_No_Type_ND |
| C219 | Void | -- Void |
| C220 | IF A.1/18 THEN M ELSE O.1 | -- O_TCP |
| C221 | IF A.1/17 THEN M ELSE O.1 | -- O_UDP |
| C222 | IF A.1/21 THEN M ELSE O.1 | -- O_BIP_GPRS |
| C223 | IF (A.1/12 OR A.1/21 OR A.1/148 OR (A1.26 AND (A.1/27 OR A.1/28 OR A.1/29 OR A.1/30))) THEN M ELSE O | -- O_BIP_CSD OR O_BIP_GPRS OR O_UICC_ACCESS_IMS OR (O_BIP_Local AND (BIP_BT OR BIP_IrDA OR BIP_RS232 OR BIP_USB)) |
| C224 | IF (A1.26 AND (A.1/27 OR A.1/28 OR A.1/29 OR A.1/30)) THEN M ELSE O | -- O_BIP_Local AND (BIP_BT OR BIP_IrDA OR BIP_RS232 OR BIP_USB) |
| C225 | IF (A.1/26 AND A1.27) THEN M ELSE O.1 | -- O_BIP_Local AND O_BIP_BT |
| C226 | IF (A.1/26 AND A1.28) THEN M ELSE O.1 | -- O_BIP_Local AND O_BIP_IrDA |
| C227 | IF (A.1/26 AND A1.29) THEN M ELSE O.1 | -- O_BIP_Local AND O_BIP_RS232 |
| C228 | IF ((A1./50 OR A.1/51 OR A.1/52 OR A.1/53 OR A.1/54 OR A.1/55 OR A.1/56 OR A.1/57 OR A.1/58 OR A.1/59 OR A.1/60 OR A.1/61 OR A.1/62) AND A.1/84) THEN M ELSE O.1 | -- (O_TAT_AL OR O_TAT_AC OR O_TAT_AR OR O_TAT_FSN OR O_TAT_FSL OR O_TAT_FSS OR O_TAT_SN OR O_TAT_SB OR O_TAT_SI OR O_TAT_SU OR O_TAT_SS OR O_TAT_STFC OR O_TAT_STFB) AND O_No_Type_ND |
| C229 | IF (A.1/24 AND A.1/84) THEN M ELSE O.1 | -- O_Duration AND O_No_Type_ND |
| C230 | Void | -- Void |
| C231 | IF (C229 OR (A.1/23 AND A.1/85)) AND A1.5 THEN M ELSE O.1 | -- O_Help AND ((O_Duration AND O_No_Type_ND) OR (O_Imm_Resp AND O_No_Type_NK)) |
| C232 | IF (A.1/26 AND A.1/30) THEN M ELSE O.1 | -- O_BIP_Local AND O_USB |
| C233 | IF A.1/31 THEN M ELSE O.1 | -- O_WML |
| C234 | IF A.1/32 THEN M ELSE O.1 | -- O_XHTML |
| C235 | IF A.1/33 THEN M ELSE O.1 | -- O_HTML |
| C236 | IF A.1/34 THEN M ELSE O.1 | -- O_CHTML |
| C237 | IF (A.1/37 AND A.1/84) THEN M ELSE O.1 | -- O_Frames AND O_No_Type_ND |
| C238 | IF A.1/38 THEN M ELSE O | -- O_MMS |
| C239 | IF A.1/35 THEN M ELSE O.1 | -- O_Batt |
| C240 | IF (A.1/36 AND A.1/84 AND A.1/85 AND A.1/87) THEN M ELSE O.1 | -- O_Xmedia Call AND O_No_Type_ND AND O_No_Type_NK AND O_No_Type_NS |
| C241 | IF (A.1/82 AND A.1/86) THEN M ELSE O.1 | -- O_M_T_Tones AND O_No_Type_NA |
| C242 | IF (A.1/16 AND A.1/84) THEN M ELSE O.1 | -- O_CC_GPRS AND O_No_Type_ND |
| C243 | IF (A.1/50 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_AL AND O_No_Type_ND |
| C244 | IF (A.1/51 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_AC AND O_No_Type_ND |
| C245 | IF (A.1/52 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_AR AND O_No_Type_ND |
| C246 | IF (A.1/53 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_FSN AND O_No_Type_ND |
| C247 | IF (A.1/54 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_FSL AND O_No_Type_ND |
| C248 | IF (A.1/55 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_FSS AND O_No_Type_ND |
| C249 | IF (A.1/56 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_SN AND O_No_Type_ND |
| C250 | IF (A.1/57 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_SB AND O_No_Type_ND |
| C251 | IF (A.1/58 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_SI AND O_No_Type_ND |
| C252 | IF (A.1/59 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_SU AND O_No_Type_ND |
| C253 | IF (A.1/60 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_SS AND O_No_Type_ND |
| C254 | IF (A.1/61 AND A.1/84) THEN M ELSE O.1 | -- O_TAT_STFC AND O_No_Type_ND |
| C255 | IF (A.1/62 AND A.1/84) THEN M ELSE O.1 | -- OR O_TAT_STFB AND O_No_Type_ND |
| C256 | IF C237 THEN M for at least one of the bits 1 - 4 of byte 24 | -- O_Frames AND O_No_Type_ND |
| C257 | IF (A.1/12 OR A.1/21 OR A.1/132 OR A.1/133 OR A.1/148 OR (A1.26 AND (A.1/27 OR A.1/28 OR A.1/29 OR A.1/30))) THEN M for at least one of the bits 6 - 8 of byte 13 | -- O_BIP_CSD OR O_BIP_GPRS OR pc_BIP_eFDD OR pc_BIP_eTDD OR O_UICC_ACCESS_IMS OR (O_BIP_Local AND (BIP_BT OR BIP_IrDA OR BIP_RS232 OR BIP_USB)) |
| C258 | IF A.1/66 THEN M ELSE O.1 | -- O_HSDPA |
| C259 | IF A.1/67 THEN M ELSE O.1 | -- O_UTRAN_PS_Ext_Param |
| C260 | IF A.1/70 THEN M ELSE O | -- O_I-WLAN |
| C261 | IF A.1/71 THEN M ELSE O.1 | -- O_Terminal_Applications |
| C262 | IF A.1/72 THEN M ELSE O.1 | -- O_TCP_UICC_ServerMode |
| C263 | IF A.1/73 THEN M ELSE O.1 | -- O_TCP_Terminal_ServerMode |
| C264 | IF A.1/74 THEN M ELSE O.1 | -- O_UDP_Terminal_ServerMode |
| C265 | IF A.1/81 THEN M ELSE O.1 | -- O_Geo_Location_Discovery |
| C266 | IF A.1/83 THEN M ELSE O.1 | -- O_Toolkit_GBA |
| C267 | IF A.1/84 THEN M ELSE O.1 | -- O_No_Type_ND |
| C268 | IF A.1/85 THEN M ELSE O.1 | -- O_No_Type_NK |
| C269 | IF A.1/86 THEN M ELSE O.1 | -- O_No_Type_NA |
| C270 | IF A.1/87 THEN M ELSE O.1 | -- O_No_Type_NS |
| C271 | IF (A.1/88 AND A.1/161) THEN M ELSE O.1 | -- O_No_Type_NL AND O_Lang_Select |
| C272 | IF A.1/89 THEN M ELSE O.1 | -- O_USSD_Data_DL |
| C273 | IF A.1/84 THEN O ELSE O.1 | -- O_No_Type_ND |
| C274 | IF A.1/84 THEN bit values "0" / "1" allowed ELSE O.1 | -- O_No_Type_ND |
| C275 | IF A.1/132 OR A.1/133 THEN M ELSE O.1 | -- pc_BIP_eFDD OR pc_BIP_eTDD |
| C276 | IF A.1/84 THEN O.1 ELSE M | -- O_No_Type_ND |
| C277 | IF A.1/85 THEN O.1 ELSE M | -- O_No_Type_NK |
| C278 | IF (A.1/134 OR A.1/139 OR A.1/140) THEN M ELSE O.1 | -- O_UTRAN OR pc_eFDD OR pc_eTDD |
| C279 | IF NOT A.1/135 THEN M ELSE O | -- O_EUTRAN_NO_UTRAN_ NO_GERAN |
| C280 | IF A.1/64 THEN M ELSE O | -- O_GERAN |
| C281 | IF A.1/136 THEN M ELSE O.1 | -- O_Event_CSG_Cell_Selection |
| C282 | IF A.1/137 THEN M ELSE O.1 | -- O_CSG_Cell_Discovery |
| C283 | IF (A.1/139 OR A.1/140) THEN M ELSE O.1 | -- pc_eFDD OR pc_eTDD |
| C284 | IF A.1/143 THEN M ELSE O.1 | -- O_Direct_Com_Channel |
| C285 | IF (A.1/73 AND A.1/84 AND A.1/85) THEN M ELSE O.1 | -- O_TCP_Terminal_ServerMode AND O_No_Type_ND AND O_No_Type_NK |
| C286 | IF A.1/144 THEN M ELSE O.1 | -- O_CC_IMS |
| C287 | IF A.1/145 THEN M ELSE O.1 | -- O_CAT_Modem_Interface |
| C288 | IF A.1/146 THEN M ELSE O.1 | -- O_Event_Incoming_IMS_Data |
| C289 | IF A.1/147 THEN M ELSE O.1 | -- O_Event_IMS_Registration |
| C290 | IF A.1/148 THEN M ELSE O.1 | -- O_UICC_ACCESS_IMS |
| C291 | IF A.1/84 AND A.1/85 AND A.1/87 AND NOT A.1/135 THEN M ELSE O | -- O_No_Type_ND AND O_No_Type_NK AND O_No_Type_NS AND (NOT O_EUTRAN_NO_UTRAN_ NO_GERAN) |
| C292 | IF A.1/162 THEN M ELSE O.1 | -- O_Provide_Local_LS |
| C293 | IF (A.1/88 AND A.1/163) THEN M ELSE O.1 | -- O_No_Type_NL AND O_Lang_Notif |
| C294 | IF (A.1/84 AND A.1/164) THEN M ELSE O.1 | -- O_No_Type_ND AND O_Refresh_AlphaIdentifier |
| C295 | IF A.1/165 THEN M ELSE O.1 | -- O_ProSE |
| C296 | IF A.1/166 THEN M ELSE O.1 | -- O_WLAN_Access_Status |
| C297 | IF A.1/167 THEN M ELSE O.1 | -- O_WLAN_Bearer |
| C298 | IF A.1/168 THEN M ELSE O.1 | -- O_I-WLAN_OR_WLAN |
| C299 | IF (A.1/150 AND A. 1/179) THEN M ELSE O.1 | -- O_IMS AND O_SM-over-IP_without_MSISDN |
| C300 | IF (A.1/150 AND A.1/84 AND A.1/85 AND A.1/87 AND A.1/180) THEN M ELSE O.1 | -- O_IMS AND O_No_Type_ND AND O_No_Type_NK AND O_No_Type_NS AND O_Voice_Call_with_URI |
| C301 | IF A.1/169 THEN M ELSE O.1 | -- O_Media_Type_Voice |
| C302 | IF A.1/170 THEN M ELSE O.1 | -- O_Media_Type_Video |
| C303 | Void |  |
| C304 | IF A.1/87 THEN M ELSE O | -- O_No_Type_NS |
| C305 | IF A.1/64 OR A.1/134 THEN M ELSE O | -- O_GERAN OR O_UTRAN |
| C306 | IF A.1/186 THEN O ELSE M | -- O_NB-IoT_only |
| O.1 | Allowed: Bit value ="0" or bit not present |  |

Annex C (informative):
Change history

| Meeting | Meeting | CP-doc | CR | REV | CAT | SUBJECT | NEW_VERS |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2.0.0 | 2.0.0 | TP-050016 | - | - |  | Approved TP-27, March 2005 | 6.0.0 |
| CT-28 | CT-28 | CP-050144 | 0001 | - | F | Correction of coding in MT Call Event | 6.1.0 |
| CT-28 | CT-28 | CP-050144 | 0002 | - | F | Correction of applicability table | 6.1.0 |
| CT-28 | CT-28 | CP-050144 | 0003 | - | F | Essential Corrections | 6.1.0 |
| CT-28 | CT-28 | CP-050144 | 0004 | - | F | Correction of coding in MT Call Event | 6.1.0 |
| CT-28 | CT-28 | CP-050144 | 0005 | - | F | Removal of GET RESPONSE references | 6.1.0 |
| CT-29 | CT-29 | CP-050447 | 0006 | - | F | Rel-6: Correction of release dependent EF values | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0007 | - | F | Correction of applicability and terminal profile support tables | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0008 | - | F | Correction of EF_BDN coding | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0009 | - | F | Incorrect Dialling Number string in clause 27.22.4.13.1 SEQ 1.9 for PCS 1900 | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0010 | - | f | Essential corrections in display icons Setup Menu and Select Item | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0011 | - | F | Incorrect Ti Flag value for SET UP 1.4.1 in clause 27.22.4.16.1 | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0012 | - | F | Correction of TP-MR (TP Message Reference) of the SMS SUBMIT TPDU submitted to the USS (Network) | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0013 | - | F | Corrections in the Logical description and BER encoding in clause 27.22.6.2 and 27.22.4.11 | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0014 | - | F | Incorrect DCS in SMS-CB data download tests | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0015 | - | F | Essential Corrections in clause 27.22.8 MO SHORT MESSAGE CONTROL BY USIM | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0016 | - | B | Introduction of BDN tests for terminals not supporting BDN | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0017 | - | F | Essential Corrections | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0018 | - | F | Incorrect SMS-PP 1.4.1 TPDU in clause 27.22.4.22.1 | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0019 | - | F | Missing interactions in Bearer Independent Protocol test cases | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0020 | - | F | Correction of Refresh tests | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0022 | - | F | Applicability of TC 27.22.4.7.1 and TCs related to FDN and BDN | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0023 | - | F | Essential correction to Terminal Profile table E.1 | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0024 | - | F | Correction of CB message identifier | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0025 | - | B | Rel-6: Addition of new UCS2 Tests | 6.2.0 |
| CT-29 | CT-29 | CP-050447 | 0027 | - | F | Incorrect Coding of SMS-PP (Data download) Message in clause 27.22.4.7.1 and 27.22.5.1 | 6.2.0 |
| - | - | - | - | - | - | 2005-10: Editorial corrections due to the CRs approved at CP-29 | 6.2.1 |
| CT-30 | CT-30 | CP-050495 | 0028 | - | F | Correction of Send SS (UCS2) tests | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0029 | - | F | Essential Corrections in clause 27.22.4.11 | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0030 | - | F | Corrections to Select Item (icons support) | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0031 | - | F | 27.22.7.4.1 Location Status Event (normal) | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0032 | - | F | Essential Corrections of Set Up Menu test | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0033 | - | F | Correction of applicability table and related addition of missing test sequences | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0034 | - | F | Correction in SMS-PP 1.4.1 TPDU of clause 27.22.4.22.1 | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0035 | - | F | Essential Corrections of SMS-PP download message in Refresh test case | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0036 | - | F | Essential Correction in MO SHORT MESSAGE CONTROL BY USIM Deletion of sequence 1.9 | 6.3.0 |
| CT-30 | CT-30 | CP-050495 | 0037 | - | F | Deletion of SEQ 1.3 in clause 27.22.4.13.1 | 6.3.0 |
| CT-31 | CT-31 | CP-060013 | 0041 | - | F | Deletion of Send Data test sequence | 6.4.0 |
| CT-31 | CT-31 | CP-060013 | 0042 | - | F | Essential correction of Provide Local Information (IMEI) test | 6.4.0 |
| CT-31 | CT-31 | CP-060013 | 0044 | - | F | Essential Correction in SEQ 1.8 of clause 27.22.8 | 6.4.0 |
| CT-31 | CT-31 | CP-060013 | 0045 | - | F | Essential correction on 27.22.7.3.1 Call Disconnected Event | 6.4.0 |
| CT-31 | CT-31 | CP-060013 | 0050 | - | F | Essential correction of Channel Data length in clause 27.22.4.30 | 6.4.0 |
| CT-31 | CT-31 | CP-060014 | 0048 | - | F | Essential Corrections in clause 27.22.4.11 | 6.4.0 |
| CT-31 | CT-31 | CP-060014 | 0052 | - | F | Essential Corrections in clause 27.22.8 MO SHORT MESSAGE CONTROL BY SIM | 6.4.0 |
| CT-31 | CT-31 | CP-060014 | 0049 | - | F | Essential correction in SEQ 1.4 of clause 27.22.4.11.1 SEND SS (normal) | 6.4.0 |
| CT-31 | CT-31 | CP-060014 | 0047 | - | F | Essential corrections of Run AT Command tests | 6.4.0 |
| CT-31 | CT-31 | CP-060014 | 0053 | - | F | Essential corrections to SET UP CALL test sequences | 6.4.0 |
| CT-31 | CT-31 | CP-060015 | 0055 | - | F | Essential Correction in TERMINAL RESPONSE coding of clause 27.22.4.31 | 6.4.0 |
| CT-31 | CT-31 | CP-060015 | 0056 | - | F | Essential corrections to Timer Expiration tests | 6.4.0 |
| CT-31 | CT-31 | CP-060015 | 0054 | - | F | BER-TLV suppressions | 6.4.0 |
| CT-31 | CT-31 | CP-060157 | 0059 | - | B | Add SMS PP Data Download RP-ERROR Test Case | 6.4.0 |
| CT-31 | CT-31 | CP-060022 | 0043 | - | F | Essential Correction in SEQ 1.7 of clause 27.22.4.13.1 | 6.4.0 |
| CT-31 | CT-31 | CP-060022 | 0046 | - | F | Essential correction of Refresh test | 6.4.0 |
| CT-31 | CT-31 | CP-060022 | 0051 | - | F | Essential correction of Channel Data length in Result TLV of clause 27.22.4.30 | 6.4.0 |
| CT-31 | CT-31 | CP-060022 | 0060 | - | F | CR 31.124 Rel-6: Insertion of missing REFRESH (IMSI changing procedure) test cases | 6.4.0 |
| CT-31 | CT-31 | CP-060022 | 0057 | - | F | Essential corrections of references | 6.4.0 |
| CT-32 | CT-32 | CP-060241 | 0061 | - |  | Proposal to the TS 31.124 Split by referencing the relevant USAT Test procedures to TS 102 384 | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0062 | - |  | Essential corrections on test cases 27.22.6.3 and 27.22.6.4 using record 2 in EF FDN | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0063 | - |  | Essential corrections on TC 27.22.6.4 sequence 4.1 | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0064 | - |  | Essential corrections on SEND SHORT MESSAGE test cases | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0065 | - |  | Essential correction of text attributes tests | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0066 | - |  | Definition of appropriate QoS in BIP test cases related to GPRS for 3G | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0071 | - |  | Essential correction of Refresh test in 27.22.7.4.2, seq. 2.4 | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0074 | - |  | Essential corrections of RUN AT Command tests | 6.5.0 |
| CT-32 | CT-32 | CP-060241 | 0067 | - |  | Essential correction of tables B.1 and E.1 | 6.5.0 |
| CT-32 | CT-32 | CP-060242 | 0068 | - | F | Essential Correction in REGISTER 1.2B message coding of clause 27.22.4.11.1 SEND SS (normal) | 6.5.0 |
| CT-32 | CT-32 | CP-060242 | 0069 | - | F | Essential correction of 27.22.4.13.1 SET UP CALL, seq 1.4 | 6.5.0 |
| CT-32 | CT-32 | CP-060242 | 0070 | - | F | Essential correction of second card reader test applicability | 6.5.0 |
| CT-32 | CT-32 | CP-060242 | 0072 | - | F | Correction of TON/NPI coding for Call Control Test case | 6.5.0 |
| CT-32 | CT-32 | CP-060242 | 0073 | - | F | Essential corrections on 27.22.4.11.1 sequence. 1.2 | 6.5.0 |
| CT-32 | CT-32 | CP-060242 | 0075 | - | F | Essential correction of BIP tests | 6.5.0 |
| CT-33 | CT-33 | CP-060389 | 0082 | 1 | F | Wrong reference inside test requirement of TC 27.22.7.2.2 | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0087 | 1 | F | Essential corrections of applicability table | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0088 | 1 | F | Essential correction of IMEISV coding for Provide Local Information | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0089 | 1 | F | Essential corrections of text attribute tests for Send USSD and Close channel | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0090 | 1 | F | Proposal to the TS 31.124 Split by referencing the relevant USAT Test procedures to TS 102 384 | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0091 | 1 | F | Correction to the UCS2 coding in Setup Call test | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0092 | 1 | F | Essential correction of RUN AT Command for text attribute tests | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0095 | 1 | F | Correction of RECEIVE DATA tests | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0096 | 1 | F | Correction of terminology for USIM Service Table | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0097 | 1 | F | Correction of 2nd alpha identifier usages in SET UP CALL tests | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0098 | 1 | F | Correction of various typographical errors | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0101 | 1 | F | Essential corrections to OPEN CHANNEL text attribute test sequences | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0078 | 1 | F | Correction of 'Precedence class' values in Bearer Independent Protocol test cases | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0076 | 1 | F | Essential corrections on PROVIDE LOCAL INFORMATION test sequences | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0080 | 2 | F | Essential corrections on test sequences using the TLV data object Location Information | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0100 | 2 | F | Essential corrections to SET UP CALL (UCS2 Display) test sequences | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0081 | 3 | F | Essential corrections to REFRESH(normal) test sequence | 6.6.0 |
| CT-33 | CT-33 | CP-060389 | 0102 | 1 | F | Essential corrections to SEND SS display tests concerning longForwardedToNumber | 6.6.0 |
| CT-33 | CT-33 | CP-060475 | 0086 | 1 | F | Essential corrections of MMI entries in table E.1 | 6.6.0 |
| CT-33 | CT-33 | CP-060475 | 0077 | 2 | F | Corrections to SET UP CALL test case 27.22.4.13.1 | 6.6.0 |
| CT-33 | CT-33 | CP-060475 | 0099 | 1 | F | Essential corrections to SEND SS concerning longForwardedToNumber | 6.6.0 |
| CT-33 | CT-33 | CP-060475 | 0094 | 2 | F | Corrections to MO SHORT MESSAGE CONTROL BY USIM tests | 6.6.0 |
| CT-33 | CT-33 | CP-060517 | 0084 | 1 | F | Essential corrections Set Up Call, seq. 1.9 | 6.6.0 |
| CT-34 | CT-34 | CP-060540 | 0103 | - | F | Correction of APN Coding in Open Channel test case | 6.7.0 |
| CT-34 | CT-34 | CP-060540 | 0085 | 2 | F | Essential corrections of BIP entries in table E.1 | 6.7.0 |
| CT-34 | CT-34 | CP-060540 | 0110 | 2 | F | Essential correction of Result TLV handling | 6.7.0 |
| CT-34 | CT-34 | CP-060540 | 0111 | - | F | Essential correction of expected sequence in OPEN CHANNEL test case | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0105 | - | F | Some of the Applicability table content is missing when printed or in Print Layout mode | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0106 | 1 | F | Correction to SET UP CALL | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0107 | - | F | Correction to SEND SS | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0058 | 1 | B | Addition of REFRESH USIM Application Reset | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0108 | - | F | Essential corrections on SEND SS (UCS2 display) test cases | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0109 | - | F | Essential corrections on REFRESH TC 27.22.4.7.1 | 6.7.0 |
| CT-34 | CT-34 | CP-060727 | 0104 | 1 | F | Corrections in the interpretation of Katakana Character | 6.7.0 |
| CT-35 | CT-35 | CP-070063 | 0115 | - | F | Essential correction of 27.22.5.2 | 6.8.0 |
| CT-35 | CT-35 | CP-070063 | 0113 | 1 | F | Essential correction of Terminal Profile Support table | 6.8.0 |
| CT-35 | CT-35 | CP-070063 | 0112 | 1 | F | Essential correction of 27.22.4.13.1 Expected Sequence 1.7 | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0116 | - | F | Essential correction of 27.22.4.7, seq. 1.7 | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0119 | - | F | Essential correction of TC 27.22.7.4.1 | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0120 | - | F | CR implementation error correction for 27.22.6.2 SEQ 2.2 | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0121 | - | F | CR implementation error correction for 27.22.4.11.1 SEQ 1.4A | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0117 | 1 | F | Essential clarification of Network Simulator selection | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0122 | 1 | F | Essential correction of 27.22.4.7.2 SEQ 2.2 | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0124 | 2 | C | Addition of new expected sequence to the SMS-PP Data Download test case | 6.8.0 |
| CT-35 | CT-35 | CP-070065 | 0125 | 2 | F | Addition of a new expected sequence to the SMS-CB Data Download test case | 6.8.0 |
| CT-36 | CT-36 | CP-070297 | 0127 | 2 | F | Essential correction of test case applicability | 6.9.0 |
| CT-36 | CT-36 | CP-070297 | 0128 | - | F | Correction of 27.22.4.2 applicability | 6.9.0 |
| CT-36 | CT-36 | CP-070297 | 0129 | 1 | A | Essential correction of test case applicability for 27.22.6.1 | 6.9.0 |
| CT-36 | CT-36 | CP-070297 | 0130 | 1 | A | Essential correction on 27.22.8 | 6.9.0 |
| CT-36 | CT-36 | CP-070297 | 0131 | - | F | Essential correction on 27.22.5.1 | 6.9.0 |
| CT-36 | CT-36 | CP-070297 | 0132 | - | F | Essential correction on 27.22.4.11.1 sequence. 1.4 B | 6.9.0 |
| CT-36 | CT-36 | CP-070297 | 0133 | - | A | Correction of reference to ISO/IEC 7816-3 | 6.9.0 |
| 2007-06 | 2007-06 | - | - | - | - | Update to Rel-7 version (MCC) | 7.0.0 |
| CT-37 | CT-37 | CP-070610 | 0136 | 1 | F | Essential Correction to 27.22.6.2 | 7.1.0 |
| CT-37 | CT-37 | CP-070619 | 0137 | - | F | Essential correction of variable timeout test case applicability | 7.1.0 |
| CT-37 | CT-37 | CP-070610 | 0138 | - | F | Essential correction to 27.22.4.13.1, seq. 1.9 | 7.1.0 |
| CT-37 | CT-37 | CP-070619 | 0139 | - | F | Essential Correction to 27.22.6.1, Seq. 1.1 | 7.1.0 |
| CT-37 | CT-37 | CP-070619 | 0140 | - | F | Essential correction of references | 7.1.0 |
| CT-37 | CT-37 | CP-070619 | 0141 | 1 | F | Essential correction of 27.22.4.13.1, sequence 1.7 | 7.1.0 |
| CT-37 | CT-37 | CP-070619 | 0142 | 1 | F | Test Cases dependant on Radio Access Clarification | 7.1.0 |
| CT-37 | CT-37 | CP-070619 | 0143 | - | F | Essential correction of 27.22.4.7.1, sequence 1.6 | 7.1.0 |
| CT-38 | CT-38 | CP-070843 | 0145 | 1 | A | Essential correction of 27.22.8, sequence 1.3 in order to remove verification of the Alpha Identifier | 7.2.0 |
| CT-38 | CT-38 | CP-070843 | 0154 | 1 | A | Essential correction of 27.22.4.7.1, sequence 1.6 caring of the missing requirements in TS 31.111 | 7.2.0 |
| CT-38 | CT-38 | CP-070843 | 0146 | 1 | A | Essential correction of 27.22.4.26.2.4.2, seq. 2.2 in order to remove the possibility of retrieving a deleted previously visited URL | 7.2.0 |
| CT-38 | CT-38 | CP-070843 | 0155 | - | A | Correction to add optional support of Call Hold Supplementary Service | 7.2.0 |
| CT-38 | CT-38 | CP-070847 | 0147 | - | F | Essential correction terminal profile indication for Local Connection Event | 7.2.0 |
| CT-38 | CT-38 | CP-070847 | 0149 | - | F | Essential correction on test case 27.22.4.5.1 | 7.2.0 |
| CT-38 | CT-38 | CP-070847 | 0150 | - | F | Definition of test sequence 1.7 in test case 27.22.4.15 | 7.2.0 |
| CT-38 | CT-38 | CP-070847 | 0151 | - | F | Definition of test sequence 1.12 and 1.13 in test case 27.22.4.15 | 7.2.0 |
| CT-38 | CT-38 | CP-070847 | 0152 | - | F | Essential correction on test case 27.22.4.28.2.1 correcting wrong implementation of CR 0078 rev1 in C6-060547 | 7.2.0 |
| CT-38 | CT-38 | CP-070847 | 0148 | 1 | F | Introduction of Rel-7 test case applicability | 7.2.0 |
| CT-39 | CT-39 | CP-080172 | 0156 | - | F | Essential correction to 27.22.4.15 | 7.3.0 |
| CT-39 | CT-39 | CP-080172 | 0157 | - | F | Essential correction of 27.22.8, seq. 1.3 | 7.3.0 |
| CT-39 | CT-39 | CP-080172 | 0158 | 1 | F | Essential correction regarding terminal capabilities | 7.3.0 |
| CT-39 | CT-39 | CP-080172 | 0159 | - | F | Essential correction to network dependency of several tests | 7.3.0 |
| CT-40 | CT-40 | CP-080388 | 0160 | 1 | F | Essential correction of icon test case applicability | 7.4.0 |
| CT-40 | CT-40 | CP-080388 | 0161 | 2 | F | Essential correction to 27.22.6.4 | 7.4.0 |
| CT-40 | CT-40 | CP-080388 | 0163 | 3 | F | Essential correction of test case applicability of 27.22.6.2 and 27.22.4.11 | 7.4.0 |
| CT-41 | CT-41 | CP-080588 | 0164 | - | F | Essential correction of TC 27.22.4.12.1 Seq. 1.6 | 7.5.0 |
| CT-41 | CT-41 | CP-080588 | 0165 | - | F | Essential correction of test case applicability | 7.5.0 |
| CT-41 | CT-41 | CP-080588 | 0166 | - | F | Essential correction of TC 27.22.7.8.1 | 7.5.0 |
| CT-42 | CT-42 | CP-080906 | 0168 | - | F | Essential correction of TC 27.22.6.5 seq. 5.1 applicability | 7.6.0 |
| CT-42 | CT-42 | CP-080906 | 0169 | - | F | Essential correction of bearer parameters in browser tests | 7.6.0 |
| CT-42 | CT-42 | CP-080948 | 0170 | 3 | A | Pre-conditions for Launch browser | 7.6.0 |
| CT-42 | CT-42 | CP-080948 | 0171 | - | A | Essential correction of 27.22.4.26.2 Seq. 2.2 | 7.6.0 |
| SP-42 | SP-42 | --------------- | - | - | - | Upgrade to Rel-8 | 8.0.0 |
| CT-43 | CT-43 | CP-090194 | 0173 | 1 | F | Inclusion of Rel-8 test case applicability and Rel-8 feature indication in the terminal profile content | 8.1.0 |
| CT-43 | CT-43 | CP-090194 | 0174 | - | F | Essential correction of tables B.1 and E.1 | 8.1.0 |
| CT-43 | CT-43 | CP-090194 | 0176 | 1 | A | Essential correction to BIP tests - usage of ME's default channel identifier | 8.1.0 |
| CT-44 | CT-44 | CP-090459 | 0175 | 3 | B | Introduction of steering of roaming test cases | 8.2.0 |
| CT-44 | CT-44 | CP-090460 | 0177 | 1 | F | Test case and test case applicability changes for terminals with reduced USAT capabilities | 8.2.0 |
| CT-45 | CT-45 | CP-090718 | 0178 | 3 | F | Essential correction to icon test applicability | 8.3.0 |
| CT-45 | CT-45 | CP-090718 | 0179 | 1 | F | Update of table E.1 regarding E-UTRAN support indication | 8.3.0 |
| CT-45 | CT-45 | CP-090718 | 0180 | 1 | F | Essential correction of 27.22.6.1 sequence 1.9 | 8.3.0 |
| CT-45 | CT-45 | CP-090718 | 0181 | - | F | Essential correction of 27.22.4.7.3, Seq. 3.2 | 8.3.0 |
| CT-45 | CT-45 | CP-090718 | 0182 | - | F | Essential correction of applicability and terminal profile table | 8.3.0 |
| - | - | --------------- | - | - | - | Correction of inconsistency spotted at implementation | 8.3.1 |
| CT-46 | CT-46 | CP-090999 | 0186 | 1 | F | Essential correction of 27.22.4.7.3 | 8.4.0 |
| CT-46 | CT-46 | CP-091000 | 0187 | 1 | F | Update of TS 31.124 for terminals supporting E-UTRAN | 8.4.0 |
| CT-46 | CT-46 | CP-091000 | 0188 | 2 | F | Introduction of OPEN CHANNEL tests for E-UTRAN | 8.4.0 |
| SA-46 | SA-46 | --------------- | - | - | - | Upgrade to Rel-9 | 9.0.0 |
| CT-47 | CT-47 | CP-100192 | 0189 | 1 | B | Introduction of BIP tests for E-UTRAN | 9.1.0 |
| CT-47 | CT-47 | CP-100192 | 0190 | 1 | B | Introduction of Network Rejection Event test | 9.1.0 |
| CT-47 | CT-47 | CP-100192 | 0191 | 1 | B | Introduction of Provide Local Information tests for E-UTRAN | 9.1.0 |
| CT-47 | CT-47 | CP-100192 | 0192 | 1 | B | Introduction of Event Download – Location Status tests for E-UTRAN | 9.1.0 |
| CT-47 | CT-47 | CP-100191 | 0194 | - | F | Introduction of Rel-9 test case applicability | 9.1.0 |
| CT-47 | CT-47 | CP-100179 | 0195 | 1 | A | Correction of typo error | 9.1.0 |
| CT-47 | CT-47 | CP-100191 | 0196 | 2 | B | Dual Open Channel tests in TCP mode | 9.1.0 |
| CT-47 | CT-47 | CP-100191 | 0197 | 1 | B | Open Channel tests for TCP mode and Default Bearer | 9.1.0 |
| CT-47 | CT-47 | CP-100191 | 0198 | 1 | F | Correction of optional features table | 9.1.0 |
| CT-47 | CT-47 | CP-100179 | 0199 | 3 | A | Correction of applicability for 'no alpha identifier presented' sequences | 9.1.0 |
| CT-47 | CT-47 | CP-100179 | 0200 | - | A | Essential correction to the condition table | 9.1.0 |
| CT-48 | CT-48 | CP-100395 | 0202 | - | F | Essential correction of 27.22.4.31.1 Seq. 1.5 | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0205 | - | F | Essential correction of Table E.1 regarding Width reduction when in a menu | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0207 | - | F | Correction to TAC coding in Provide Local Information test | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0201 | 1 | B | Essential correction of table E.1 | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0204 | 1 | F | Essential correction of 27.22.4.27.2 Seq 2.10 test case applicability | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0206 | 1 | F | Correction to applicability table | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0208 | 1 | B | Network Search mode test | 9.2.0 |
| CT-48 | CT-48 | CP-100395 | 0209 | 1 | B | Event download, Network Search mode test | 9.2.0 |
| CT-48 | CT-48 | CP-100396 | 0203 | 1 | B | Introduction of Steering of Roaming test for E-UTRAN | 9.2.0 |
| CT-49 | CT-49 | CP-100591 | 0218 | 3 | A | Essential correction to Open Channel 27.22.4.27.2 sequence 2.4 test | 9.3.0 |
| CT-49 | CT-49 | CP-100592 | 0212 | 1 | F | Update of references | 9.3.0 |
| CT-49 | CT-49 | CP-100593 | 0220 | 1 | F | Essential correction to test case applicability of letter class C features | 9.3.0 |
| CT-49 | CT-49 | CP-100593 | 0214 | 1 | F | Correction of 27.22.4.28.3. Seq 3.2 | 9.3.0 |
| CT-49 | CT-49 | CP-100593 | 0219 | 1 | F | Essential correction to SET UP CALL 27.22.4.13 sequence 1.1 | 9.3.0 |
| CT-49 | CT-49 | CP-100613 | 0215 | 3 | B | Addition of Access Technology change event download tests for E-UTRAN | 9.3.0 |
| CT-49 | CT-49 | CP-100613 | 0216 | 3 | C | Addition of Open Channel test related to E-UTRAN network | 9.3.0 |
| CT-49 | CT-49 | CP-100613 | 0222 | 1 | B | Addition of Call Control tests for E-UTRAN | 9.3.0 |
| CT-49 | CT-49 | CP-100620 | 0221 | 2 | F | Essential correction of test 27.22.4.9.3 | 9.3.0 |
| CT-50 | CT-50 | CP-100835 | 0242 | 1 | B | Addition of Provide local information test , discovery of surrounding CSG cell | 9.4.0 |
| CT-50 | CT-50 | CP-100833 | 0234 | 1 | F | Clarification of 'ELSE' parts in Table E.1 | 9.4.0 |
| CT-50 | CT-50 | CP-100834 | 0235 | 1 | F | Correction of TCP/UDP referencing errors in Table E.1 | 9.4.0 |
| CT-50 | CT-50 | CP-100834 | 0236 | 1 | F | LTE test cases - specifying that default E-UTRAN UICC should be used | 9.4.0 |
| CT-50 | CT-50 | CP-100834 | 0238 | 1 | F | Correction of SET UP CALL sequence 1.1 | 9.4.0 |
| CT-50 | CT-50 | CP-100830 | 0233 | 1 | B | Definition of E-UTRAN/EPC ISIM-UICC for ISIM related testing | 9.4.0 |
| CT-50 | CT-50 | CP-100834 | 0239 | 1 | F | Correction of references to non-existent data items in CLOSE CHANNEL(E-UTRAN/EPC) | 9.4.0 |
|  |  |  |  |  | - | Correction of errors in implementation of CR 234 (MCC). | 9.4.1 |
| CT-51 | CT-51 | CP-110231 | 0217 | 4 | B | Addition of Provide Local Information tests for multiple access technologies | 9.5.0 |
| CT-51 | CT-51 | CP-110230 | 0243 | 4 | B | Introduction ISIM related SMS-PP Data Download tests | 9.5.0 |
| CT-51 | CT-51 | CP-110230 | 0244 | 6 | B | Introduction ISIM related Send Short Message tests | 9.5.0 |
| CT-51 | CT-51 | CP-110231 | 0245 | 2 | C | Optimization of SEND SMS test cases | 9.5.0 |
| CT-51 | CT-51 | CP-110231 | 0246 | 1 | C | Optimization of SMS PP Download test case | 9.5.0 |
| CT-51 | CT-51 | CP-110231 | 0248 |  | B | Introduction of Polling Off test for E-UTRAN | 9.5.0 |
| CT-51 | CT-51 | CP-110231 | 0250 | 1 | F | Essential correction on BIP TCs for E-UTRAN/EPC | 9.5.0 |
| SP-51 | SP-51 |  |  |  |  | Automatic upgrade from previous version 9.5.0 | 10.0.0 |
| CT-52 | CT-52 | CP-110503 | 0241 | 3 | F | Addition of Event download test, CSG cell Selection | 10.1.0 |
| CT-52 | CT-52 | CP-110504 | 0252 |  | F | Introduction ISIM related SMS-PP Data Download tests | 10.1.0 |
| CT-52 | CT-52 | CP-110504 | 0253 | 1 | F | Introduction ISIM related Send Short Message tests | 10.1.0 |
| CT-53 | CT-53 | CP-110719 | 0255 | 3 | F | Essential correction of the Terminal Profile entries in table E.1 | 10.2.0 |
| CT-53 | CT-53 | CP-110719 | 0258 | 1 | F | Essential correction of Send Short message tests | 10.2.0 |
| CT-53 | CT-53 | CP-110592 | 0259 | 1 | A | Essential correction of Data Destination Address settings in BIP and Launch Browser tests | 10.2.0 |
| CT-53 | CT-53 | CP-110719 | 0261 | 1 | F | Essential Correction to Tag length in Provide Local Information test | 10.2.0 |
| CT-53 | CT-53 | CP-110719 | 0262 | 1 | F | Essential Correction to Network Rejection Event test | 10.2.0 |
|  |  |  |  |  |  | Correction of implementation error in CR 255r3 (MCC) | 10.2.1 |
| CT-54 | CT-54 | CP-110904 | 0263 |  | F | Essential correction of SMS-PP Data Download test cases | 10.3.0 |
| CT-54 | CT-54 | CP-110904 | 0265 | 1 | F | Essential correction to Channel Status After Link Dropped in E-UTRA | 10.3.0 |
| CT-54 | CT-54 | CP-110904 | 0266 | 1 | F | Correction to test sequence content 4.3 and 4.4 for test case 27.22.4.1 of Table B.1 | 10.3.0 |
| CT-54 | CT-54 | CP-110904 | 0256 | 2 | F | Essential correction to Steering of Roaming test case | 10.3.0 |
| CT-54 | CT-54 | CP-110906 | 0264 | 1 | A | Essential correction to SMS-CB Applicability | 10.3.0 |
| CT-54 | CT-54 | CP-110906 | 0257 | 2 | A | Essential correction to Play Tone test | 10.3.0 |
| CT-54 | CT-54 | CP-110907 | 0267 |  | F | Correction of incorrect implementation of CR 255r3 | 10.3.0 |
| CT-55 | CT-55 | CP-120151 | 0269 | 1 | A | Test applicability correction of Open Channel with user rejection tests | 10.4.0 |
| CT-55 | CT-55 | CP-120152 | 0271 | 2 | F | Essential correction to test 27.22.4.15 Seq. 1.15 | 10.4.0 |
| CT-55 | CT-55 | CP-120153 | 0268 | 3 | B | Introduction of REFRESH with AID test | 10.4.0 |
| CT-56 | CT-56 | CP-120394 | 0270 | 2 | A | Test applicability correction for terminals operating in PS mode | 10.5.0 |
| CT-56 | CT-56 | CP-120394 | 0275 | 1 | A | Correction of expected Terminal Reponse for unsuccessful Open Channel commands | 10.5.0 |
| CT-56 | CT-56 | CP-120395 | 0277 |  | F | Essential corrections to the Network Rejection Event test cases | 10.5.0 |
| CT-56 | CT-56 | CP-120395 | 0279 |  | B | Introduction of test cases for Send Short Message and SMS PP data download over SGs (E-UTRAN) | 10.5.0 |
| CT-56 | CT-56 | CP-120395 | 0276 | 1 | F | Essential correction of Open Channel with Bearer type 0B tests | 10.5.0 |
| CT-56 | CT-56 | CP-120395 | 0278 | 1 | C | Test modification for Provide Local Information IMEI and IMEISV testing | 10.5.0 |
| CT-57 | CT-57 | CP-120629 | 0282 | 2 | A | Essential correction of Launch Browser tests | 10.6.0 |
| CT-57 | CT-57 | CP-120629 | 0286 | 2 | A | Essential correction of Launch Browser tests | 10.6.0 |
| CT-57 | CT-57 | CP-120630 | 0283 |  | F | Correction of Terminal Profile entries in table E.1 | 10.6.0 |
| CT-57 | CT-57 | CP-120630 | 0281 | 1 | F | Correction of test sequence for PROVIDE LOCAL INFORMATION, Discovery of surrounding CSG cells | 10.6.0 |
| CT-57 | CT-57 | CP-120631 | 0280 | 1 | F | Corrections to test sequence 27.22.7.18.1 for CSG Cell Selection | 10.6.0 |
| CT-57 | CT-57 | CP-120632 | 0284 |  | F | Correction of network simulator dependencies of the tests in 27.22.7.4 | 10.6.0 |
| CT-57 | CT-57 | CP-120633 | 0272 | 5 | B | Addition of UICC Access to IMS tests | 10.6.0 |
| SP-57 | SP-57 |  |  |  |  | Automatic upgrade to Rel-11 | 11.0.0 |
| CT-58 | CT-58 | CP-120875 | 0287 | 1 | F | TERMINAL RESPONSE in steering of roaming test steps | 11.1.0 |
| CT-59 | CT-59 | CP-130149 | 0290 | 1 | A | Applicability of tests for MEs with reduced capabilities | 11.2.0 |
| CT-60 | CT-60 | CP-130370 | 0285 | 6 | B | Superseding of OPEN CHANNEL test sequence 2.1 by Default Bearer test sequence | 11.3.0 |
| CT-60 | CT-60 | CP-130370 | 0291 | 1 | F | Removal of applicability condition C102 | 11.3.0 |
| CT-60 | CT-60 | CP-130370 | 0292 |  | F | Correction to the applicability of test case 27.22.4.7 seq. 4.1 | 11.3.0 |
| CT-60 | CT-60 | CP-130370 | 0293 |  | F | Correction to the applicability of test case 27.22.8 seq. 1.4 | 11.3.0 |
| CT-60 | CT-60 | CP-130371 | 0296 | 2 | F | Correction of test sequence for PROVIDE LOCAL INFORMATION, E-UTRAN Intra-Frequency and Inter-Frequency Measurements | 11.3.0 |
| CT-60 | CT-60 | CP-130370 | 0297 | 1 | F | Change of test sequence for SMS-PP data download | 11.3.0 |
| CT-60 | CT-60 | CP-130373 | 0298 | 2 | A | Changes in LAUNCH BROWSER test cases | 11.3.0 |
| CT-60 | CT-60 | CP-130370 | 0299 | 2 | F | Correction of test sequence for PROVIDE LOCAL INFORMATION, NMR, UTRAN | 11.3.0 |
| CT-60 | CT-60 | CP-130370 | 0300 | 2 | F | Essential correction to the applicability and test procedure of test case 27.22.4.10 Seq 1.9 & 27.22.5.1 Seq 1.9 | 11.3.0 |
| CT-61 | CT-61 | CP-130532 | 0301 | 1 | F | Correction of Terminal Profile evaluation | 11.4.0 |
| CT-61 | CT-61 | CP-130532 | 0304 | 1 | F | Correction of chapter numbering in 27.22.7.15 | 11.4.0 |
| CT-61 | CT-61 | CP-130532 | 0305 | 1 | F | Correction to applicability information of test case 27.22.4.15 seq 1.10 | 11.4.0 |
| CT-62 | CT-62 | CP-130791 | 0302 | 1 | F | Correction of Terminal Profile evaluation for SET UP CALL bit | 11.5.0 |
| CT-62 | CT-62 | CP-130791 | 0307 |  | F | Correction to test case 27.22.5.2 seq. 1.7 | 11.5.0 |
| CT-62 | CT-62 | CP-130791 | 0308 |  | F | Update the status of A.1/154 | 11.5.0 |
| CT-62 | CT-62 | CP-130791 | 0309 |  | F | Update of the Generic Test Procedure 1 (SMS-PP Data Download) | 11.5.0 |
| CT-63 | CT-63 | CP-140173 | 0310 | 1 | F | Usage of URL in test cases for LAUNCH BROWSER command | 11.6.0 |
| CT-64 | CT-64 | CP-140428 | 0313 | 1 | F | Clarification on test case for PROVIDE LOCAL INFORMATION, E-UTRAN Inter-Frequency Measurements | 11.7.0 |
| CT-64 | CT-64 | CP-140426 | 0311 | 2 | B | Changes for validation of TI value | 12.0.0 |
| CT-64 | CT-64 | CP-140433 | 0314 | 1 | F | Modification to test case 27.22.4.28.3 SEQ 3.2 (step 5) | 12.0.0 |
| CT-65 | CT-65 | CP-140705 | 0317 | 1 | F | Open channel terminal response in case of modified parameters | 12.1.0 |
| CT-65 | CT-65 | CP-140705 | 0321 | 3 | F | Change of test sequence for LAUNCH BROWSER with default URL | 12.1.0 |
| CT-65 | CT-65 | CP-140709 | 0316 |  | F | Removal of applicability condition C133, C135, C136, C137 and C138 | 12.1.0 |
| CT-65 | CT-65 | CP-140710 | 0315 |  | F | Correction of Network Dependency of the TBD test sequence | 12.1.0 |
| CT-66 | CT-66 | CP-140965 | 0324 | 1 | F | Change of test sequence for LAUNCH BROWSER with default URL | 12.2.0 |
| CT-66 | CT-66 | CP-140966 | 0323 | 3 | F | Correction of usage of TP-Message-Reference (TP-MR) in Send Short Message 1.9 | 12.2.0 |
| CT-67 | CT-67 | CP-150164 | 0411 |  | B | Added column for Rel.12 in applicability table | 12.3.0 |
| CT-67 | CT-67 | CP-150164 | 0412 | 1 | F | Update of reference to ETSI TS 102 221 and release scope | 12.3.0 |
| CT-67 | CT-67 | CP-150164 | 0416 |  | F | Correction of OPEN CHANNEL Alpha Identifier handling and introduction of new alternative Terminal Response for GET CHANNEL STATUS Sequences 1.4 and 1.5 and CLOSE CHANNEL Sequence 3.2. | 12.3.0 |
| CT-67 | CT-67 | CP-150164 | 0417 |  | F | Correction of usage of TP-Message-Reference (TP-MR) in remaining Send Short Message test cases | 12.3.0 |
| CT-68 | CT-68 | CP-150387 | 0419 | 3 | C | Removal of mandatory clause | 13.0.0 |
| CT-68 | CT-68 | CP-150386 | 0420 | 3 | C | Making features optional | 13.0.0 |
| CT-69 | CT-69 | CP-150562 | 0423 |  | F | Typo in the Option A.1/74 for Class E: Terminal supports UDP, Terminal in Server Mode | 13.1.0 |
| CT-69 | CT-69 | CP-150562 | 0422 | 1 | B | Addition of Rel.13 column to applicability table | 13.1.0 |
| CT-69 | CT-69 | CP-150562 | 0427 | 1 | F | Correction of technical handling of features made optional by TR 31.901 within applicability table and terminal profile. | 13.1.0 |
| CT-69 | CT-69 | CP-150562 | 0428 | 1 | F | Correction to PLI, Inter-frequency UTRAN Measurements test case | 13.1.0 |
| CT-69 | CT-69 | CP-150562 | 0424 | 3 | B | USAT Testing Enhancement by addition of REFRESH with IMSI changing procedure test sequences | 13.1.0 |
| CT-69 | CT-69 | CP-150562 | 0425 | 3 | B | USAT Testing Enhancement by addition of REFRESH with IMSI changing procedure test sequences for E-UTRAN | 13.1.0 |
| CT-70 | CT-70 | CP-150828 | 0430 | 1 | F | Correction of applicability table for Short Message Service (SMS) over SGs | 13.2.0 |
| CT-71 | CT-71 | CP-160144 | 0434 |  | F | Correction of test case for Location status and access technology change events | 13.3.0 |
| CT-71 | CT-71 | CP-160144 | 0431 | 1 | F | Correction of TERMINAL RESPONSE coding in 27.22.4.7.2 sequence 2.3 | 13.3.0 |
| CT-71 | CT-71 | CP-160144 | 0432 | 1 | D | Editorial corrections of 27.22.4.11.1 – Expected Sequence 1.5 | 13.3.0 |
| CT-71 | CT-71 | CP-160144 | 0433 | 2 | F | Inclusion of Rel-12 and Rel-13 feature indication in the terminal profile support in Annex B | 13.3.0 |
| CT-72 | CT-72 | C6-160214 | 0435 |  | F | Addition of execution parameter to the applicability of TC 27.22.4.28.3 sequence 3.2 | 13.4.0 |
| CT-72 | CT-72 | C6-160333 | 0441 |  | F | Clarification of ME behavior after 3G session reset for E-UTRAN | 13.4.0 |
| CT-72 | CT-72 | C6-160237 | 0440 |  | F | Correction to Test Case 27.22.4.15 | 13.4.0 |
| CT-72 | CT-72 | C6-160262 | 0437 | 2 | F | Addition of note to TC 27.22.4.7.2 Seq. 2.6/7 and TC 27.22.4.7.5 Seq. 5.1/2 | 13.4.0 |
| CT-72 | CT-72 | C6-160266 | 0439 | 1 | F | Correction of test case for Location status and access technology change events | 13.4.0 |
| CT-72 | CT-72 | C6-160278 | 0438 | 2 | F | Essential correction of test case 27.22.4.14 for E-UTRAN | 13.4.0 |
| CT-72 | CT-72 | C6-160280 | 0436 | 1 | F | Clarification of ME behavior after 3G session reset | 13.4.0 |
| CT-73 | CT-73 | C6-160402 | 0442 | 1 | F | Essential correction of test case 27.22.4.14 Sequence 1.1 | 13.5.0 |
| CT-73 | CT-73 | C6-160386 | 0443 | 1 | F | Essential corrections on test case 27.22.4.7.3 | 13.5.0 |
| CT-73 | CT-73 | C6-160373 | 0444 | 1 | F | Clarification of ME behaviour after 3G session reset | 13.5.0 |
| CT-73 | CT-73 | C6-160393 | 0445 |  | F | Essential correction to number of BIP channels | 13.5.0 |
| CT-73 | CT-73 | C6-160402 | 0446 | 1 | F | Definition of expected EVENT DOWNLOAD - Location Status content in test case 27.22.7.4 | 13.5.0 |
| CT-74 | CT-74 | C6-160515 | 0447 |  | F | Bit in Terminal Profile for call control functionality | 13.6.0 |
| CT-74 | CT-74 | C6-160562 | 0449 |  | F | Essential correction to test case on PROVIDE LOCAL INFORMATION | 13.6.0 |
| CT-74 | CT-74 | C6-160595 | 0448 | 1 | F | Correction in initial conditions for test case for Open Channel (related to E-UTRAN) | 13.6.0 |
| CT-75 | CT-75 | C6-170090 | 0451 | 1 | B | Modification of test cases 27.22.4.10.8 and 27.22.5.4 to test NB-IoT | 13.7.0 |
| CT-75 | CT-75 | C6-170097 | 0450 | 3 | B | Updating some E-UTRAN test cases applicability to cover NB-IoT implementations | 13.7.0 |
| CT-75 | CT-75 | C6-170044 | 0452 | - | B | Modification of E-UTRAN test sequences under cl. 27.22.4.15 and 27.22.4.14 to cover NB-IoT | 13.7.0 |
| 0453 | 0453 | C6-17045 | 0453 | - | B | Modification of E-UTRAN test sequences under cl. 27.22.4.7.3 and 27.22.4.7.5 to test NB-IoT | 13.7.0 |
| SA-75 | SA-75 |  | - | - | 13.7.0 | Update to Rel-14 version (MCC) | 14.0.0 |
|  |  |  |  |  | 14.0.0 | Correction of implementation error | 14.0.1 |
| CT-76 | CT-76 | C6-170246 | 0460 | - | B | Modification of E-UTRAN BIP test sequences to verify NB-IoT | 14.1.0 |
| CT-76 | CT-76 | C6-170270 | 0459 | 1 | B | Modification of E-UTRAN test sequences under cl. 27.22.7.4 and 27.22.7.17 to test NB-IoT | 14.1.0 |
| CT-76 | CT-76 | C6-170290 | 0461 | 2 | B | Introduction of new test case for Call Control on EPS PDN connection | 14.1.0 |
| CT-77 | CT-77 | C6-170421 | 0466 | - | F | Essential correction to test sequences related to Steering of roaming | 14.2.0 |
| CT-77 | CT-77 | C6-170422 | 0467 | - | F | Conditions for URI support in SEND SHORT MESSAGE command | 14.2.0 |
| CT-77 | CT-77 | C6-170423 | 0468 | - | F | Correction of AT Response in test cases for RUN AT COMMAND | 14.2.0 |
| CT-77 | CT-77 | C6-170520 | 0469 | - | F | Essential correction to the applicability of URI support in SET UP CALL | 14.2.0 |
| CT-77 | CT-77 | C6-170480 | 0470 | 1 | F | Clarification on the requested address during execution of test cases for OPEN CHANNEL | 14.2.0 |
| CT-77 | CT-77 | C6-170488 | 0471 | 3 | F | Correction of wrong implementation of CRs in TS 31.124 | 14.2.0 |
| CT-77 | CT-77 | C6-170504 | 0473 | 2 | B | Introduction of new test sequences for EVENT DOWNLOAD in E-UTRAN | 14.2.0 |
| CT-77 | CT-77 | C6-170505 | 0474 | 3 | F | Corrections of test case 27.22.10 | 14.2.0 |
| CT-77 | CT-77 | C6-170506 | 0475 | 3 | B | Adding content to FFS test sequences under 27.22.10 | 14.2.0 |
| CT-78 | CT-78 | C6-170743 | 0476 | 5 | B | Introduction of new test case for Call Control on PDP Context Activation | 14.3.0 |
| CT-78 | CT-78 | C6-170698 | 0477 | 1 | F | Correction of AT Command in test cases for RUN AT COMMAND | 14.3.0 |
| CT-78 | CT-78 | C6-170634 | 0478 | - | B | Usage of programmed USIM for execution of test cases | 14.3.0 |
| CT-78 | CT-78 | C6-170693 | 0479 | 1 | F | Fixed applicability table for Call Control on EPS PDN connection | 14.3.0 |
| CT-78 | CT-78 | C6-170637 | 0480 | - | F | Correction of call flow for CALL CONTROL on EPS PDN Connection | 14.3.0 |
| CT-78 | CT-78 | C6-170647 | 0481 | - | F | Correction of wrong implemention of CR 0471 | 14.3.0 |
| CT-78 | CT-78 | C6-170747 | 0482 | 3 | D | Introduction of note about applicability of some test cases | 14.3.0 |
| CT-78 | CT-78 | C6-170724 | 0483 | 1 | D | Section number correction of TC 27.22.10 | 14.3.0 |
| CT-78 | CT-78 | C6-170725 | 0484 | 1 | F | Introduction of general definition and environment for E-UTRAN in NB-S1 mode | 14.3.0 |
| CT-78 | CT-78 | C6-170721 | 0485 | 1 | F | Clarification on the requested address during execution of TC 27.22.4.31 and 27.22.7.10 | 14.3.0 |
| CT-79 | CT-79 | C6-180079 | 0487 | 4 | B | Introduction of new test cases on UICC interface in PSM & eDRX | 14.4.0 |
| CT-79 | CT-79 | C6-180061 | 0488 | 1 | F | Correction of wrong implemenation of CR 0471 | 14.4.0 |
| CT-80 | CT-80 | C6-180160 | 0489 | 2 | B | Introduction of new test cases on change eCall mode | 14.5.0 |
| CT-80 | CT-80 | C6-180118 | 0490 | - | B | Inclusion of Rel-14 feature indication in the terminal profile support in Annex B | 14.5.0 |
| CT-80 | CT-80 | C6-180164 | 0491 | 1 | F | Correct incorrectly implemented conditions for testcases. | 14.5.0 |
| CT-80 | CT-80 |  |  |  |  | Update to release 15 |  |
| 2018-09 | CT#81 | C6-180554 | 0497 | 1 | F | Correction to expression of C291 in Table E.1 | 15.1.0 |
| 2018-09 | CT#81 | C6-180555 | 0498 | 1 | F | Correction to TC 27.22.4.28 Seq. 3.2 | 15.1.0 |
| 2018-09 | CT#81 | C6-180556 | 0499 | 1 | F | Correction to test case 27.22.5.4 Seq 4.1 | 15.1.0 |
| 2018-09 | CT#81 | C6-180557 | 0500 | 1 | F | Correction to applicability of TC 27.22.8.1 Seq 1.10 to Seq. 1.17 | 15.1.0 |
| 2018-09 | CT#81 | C6-180558 | 0501 | 1 | F | Correction to applicability of TC 27.22.4.15 Seq. 1.17 | 15.1.0 |
| 2018-09 | CT#81 | C6-180562 | 0502 | 1 | F | TS 31.124: Adding applicability for Rel.15 terminals | 15.1.0 |
| 2018-09 | CT#81 | C6-180331 | 0493 | 2 | B | Addition of new test case to cl. 27.22.7.12 | 15.2.0 |
| 2018-09 | CT#81 | C6-180330 | 0494 | - | F | Addition of Terminal Response verification to Test Sequences 1.3, 1.4 and 1.5 under 27.22.7.10 | 15.2.0 |
| 2018-09 | CT#81 | C6-180380 | 0495 | 1 | B | Addition of new test cases for 3GPP PS Data Off | 15.2.0 |
| 2018-09 | CT#81 | C6-180565 | 0496 | 3 | B | Introduction of new test cases for Data Connection Status Change event | 15.2.0 |
| 2018-09 | CT#81 | C6-180218 | 0492 | - | D | Remove ambiquity in Location Status Event and Access Technology Change Event testcases. | 15.2.0 |
| 2018-12 | CT#82 | C6-180678 | 0504 | 1 | F | Correction to Network Rejection event code for E-UTRAN | 15.3.0 |
| 2018-12 | CT#82 | C6-180634 | 0505 | - | F | Correction of wrong implementation of CR 0488 | 15.3.0 |
| 2019-03 | CT#83 | C6-190027 | 0506 | - | F | Correction of applicability of test case 27.22.7.4 Seq. 1.1 | 15.4.0 |
| 2019-03 | CT#83 | C6-190062 | 0507 | 2 | F | Close channel with Command qualifier Set to 1 | 15.4.0 |
| 2019-03 | CT#83 | C6-190078 | 0508 | 3 | F | Verify the maximum number of Open Channel requests handled by UE | 15.4.0 |
| 2019-06 | CT#84 | C6-191017 | 0511 | 1 | B | Extend the scope of 31.124 to cover 5G aspects | 15.5.0 |
| 2019-12 | CT#86 | C6-190355 | 0513 | 1 | F | Correction of wrong reference in TC 27.22.7.10.1 Seq 1.4 | 15.6.0 |
| 2019-12 | CT#86 | C6-190356 | 0514 | 1 | F | Update test spec to correctly reflect global phonebook support for certain devices | 15.6.0 |
| 2019-12 | CT#86 | C6-190444 | 0516 | 2 | F | Update on Exceptions for NB-IoT | 15.6.0 |
| 2019-12 | CT#86 | C6-190448 | 0518 | 2 | F | Correction of conditional expected values in the TERMINAL PROFILE | 15.6.0 |
| 2019-12 | CT#86 | CP-193263 | 0520 | - | F | Correction of terminal profile support in Annex B | 15.6.0 |
| 2020-03 | CT#87e | CP-200085 | 0522 | 2 | B | 27.22.7.4 Seq 1.3_EVENT DOWNLOAD-LOCATION STATUS, NG-RAN | 15.7.0 |
| 2020-03 | CT#87e | CP-200085 | 0523 | 3 | B | 27.22.4.15 Seq 1.22_PLI_LOCATION_INFORMATION_NG-RAN | 15.7.0 |
| 2020-03 | CT#87e | CP-200085 | 0524 | - | B | 27.22.7.12 Seq 1.4_EVENT DOWNLOAD-Access Tech Event, NG-RAN | 15.7.0 |
| 2020-03 | CT#87e | CP-200085 | 0525 | - | B | 27.22.4.15 Seq 1.23_PLI_Acces_Technology_NG-RAN | 15.7.0 |
| 2020-03 | CT#87e | CP-200085 | 0526 | - | B | 27.22.7.17 Seq 1.3&1.4_EVENT DOWNLOAD-Registration_Reject Event, NG-RAN | 15.7.0 |
| 2020-03 | CT#87e | CP-200085 | 0527 | - | B | T27.22.4.27.8_Open_Channel_related to NG-RAN | 15.7.0 |