3GPP TS 36.133 V14.21.0 (2022-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	78

1	Scope	79

2	References	79

3	Definitions, symbols and abbreviations	80

3.1	Definitions	80

3.2	Symbols	82

3.3	Abbreviations	83

3.4	Test tolerances	85

3.5	Additional notation	85

3.5.1	Groups of bands	85

3.6	General	87

3.6.1	Applicability of requirements in this specification version	87

4	E-UTRAN RRC_IDLE state mobility	93

4.1	Cell Selection	93

4.2	Cell Re-selection	93

4.2.1	Introduction	93

4.2.2	Requirements	93

4.2.2.1	Measurement and evaluation of serving cell	94

4.2.2.2	Void	95

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	95

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	97

4.2.2.5	Measurements of inter-RAT cells	99

4.2.2.5.1	Measurements of UTRAN FDD cells	100

4.2.2.5.2	Measurements of UTRAN TDD cells	101

4.2.2.5.3	Measurements of GSM cells	103

4.2.2.5.4	Measurements of HRPD cells	104

4.2.2.5.5	Measurements of cdma2000 1X	105

4.2.2.6	Evaluation of cell re-selection criteria	106

4.2.2.7	Maximum interruption in paging reception	106

4.2.2.8	void	107

4.2.2.9	UE measurement capability	107

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	108

4.2.2.10	Reselection to CSG cells	108

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	108

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	109

4.2.2.11	Void	110

4.2.2.12	Void	110

4.2.2.13	Void	110

4.3	Minimization of Drive Tests (MDT)	110

4.3.1	Introduction	111

4.3.2	Measurements	111

4.3.2.1	Requirements	111

4.3.3	Relative Time Stamp Accuracy	111

4.3.3.1	Requirements	111

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	111

4.3.4.1	 Requirements	111

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	112

4.3.5.1	Requirements for timeSinceFailure	112

4.4	MBSFN Measurements	112

4.4.1	Introduction	112

4.4.2	MBSFN RSRP measurements	112

4.4.3	MBSFN RSRQ measurements	112

4.4.4	MCH BLER measurements	113

4.5	Proximity-based Services	113

4.5.1	Introduction	113

4.5.2	Requirements	113

4.5.2.1	Interruptions with ProSe Direct Discovery	113

4.5.2.2	Interruptions with ProSe Direct Communication	113

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	113

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	114

4.6	Cell Selection and Re-selection Requirements for UE category NB1	114

4.6.1	Cell Selection	114

4.6.2	Cell Re-selection	114

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	115

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	115

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	117

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	117

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	119

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	120

4.6.2.7	Maximum interruption in paging reception in normal coverage	121

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	121

4.6.2.8	UE measurement capability	122

4.7	Cell Selection and Re-selection Requirements for UE category M1	122

4.7.1	Cell Selection	122

4.7.2	Cell Re-selection	122

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	122

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	122

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	122

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	124

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	125

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	125

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	126

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	126

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	127

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	128

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	130

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	131

4.8	Idle State Positioning Measurement Requirements for UE category NB1	131

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	131

4.8.1.1	RSTD Measurement Reporting Delay	133

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	133

4.8.2.1	RSTD Measurement Reporting Delay	134

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	135

4.8.3.1	RSTD Measurement Reporting Delay	136

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	137

4.8.4.1	RSTD Measurement Reporting Delay	138

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	139

4.8.5.1	Measurement Reporting Delay	140

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	140

4.8.6.1	Measurement Reporting Delay	141

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	141

4.8.7.1	Measurement Reporting Delay	142

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	143

4.8.8.1	Measurement Reporting Delay	144

5	E-UTRAN RRC_CONNECTED state mobility	144

5.1	E-UTRAN Handover	145

5.1.1	Introduction	145

5.1.2	Requirements	145

5.1.2.1	E-UTRAN FDD – FDD	145

5.1.2.1.1	Handover delay	145

5.1.2.1.2	Interruption time	146

5.1.2.2	E-UTRAN FDD – TDD	147

5.1.2.2.1	(Void)	147

5.1.2.2.2	(Void)	147

5.1.2.3	E-UTRAN TDD – FDD	147

5.1.2.3.1	(Void)	147

5.1.2.3.2	(Void)	147

5.1.2.4	E-UTRAN TDD – TDD	147

5.1.2.4.1	Handover delay	147

5.1.2.4.2	Interruption time	148

5.1.2.5	E-UTRAN HD–FDD	149

5.1.2.5.1	Handover delay	149

5.1.2.5.2	Interruption time	149

5.2	Void	151

5.3	Handover to other RATs	151

5.3.1	E-UTRAN - UTRAN FDD Handover	151

5.3.1.1	Introduction	151

5.3.1.1.1	Handover delay	151

5.3.1.1.2	Interruption time	151

5.3.2	E-UTRAN - UTRAN TDD Handover	152

5.3.2.1	Introduction	152

5.3.2.2	Requirements	152

5.3.2.2.1	Handover delay	152

5.3.2.2.2	Interruption time	152

5.3.3	E-UTRAN - GSM Handover	153

5.3.3.1	Introduction	153

5.3.3.2	Requirements	153

5.3.3.2.1	Handover delay	153

5.3.3.2.2	Interruption time	153

5.4	Handover to Non-3GPP RATs	153

5.4.1	E-UTRAN – HRPD Handover	153

5.4.1.1	Introduction	153

5.4.1.1.1	Handover delay	154

5.4.1.1.2	Interruption time	154

5.4.2	E-UTRAN – cdma2000 1X Handover	154

5.4.2.1	Introduction	154

5.4.2.1.1	Handover delay	154

5.4.2.1.2	Interruption time	154

5.5	E-UTRAN Handover for Cat-M1 UEs	155

5.5.1	Introduction	155

5.5.2	Requirements in CEModeA	155

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	155

5.5.2.1.1	Handover delay	155

5.5.2.1.2	Interruption time	155

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	156

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	156

5.5.2.3.1	Void	156

5.5.2.3.2	Void	156

5.5.3	Requirements in CEModeB	156

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	156

5.5.3.1.1	Handover delay	156

5.5.3.1.2	Interruption time	157

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	157

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	157

5.6	Void	157

6	RRC Connection Mobility Control	157

6.1	RRC Re-establishment	157

6.1.1	Introduction	158

6.1.2	Requirements	158

6.1.2.1	UE Re-establishment delay requirement	158

6.2	Random Access	158

6.2.1	Introduction	158

6.2.2	Requirements	158

6.2.2.1	Contention based random access	159

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	159

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	159

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	159

6.2.2.1.4	Void	159

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	159

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	159

6.2.2.2	Non-Contention based random access	159

6.2.2.2.1	Correct behaviour when receiving Random Access Response	159

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	159

6.2.3	Requirements for Cat-M1 UEs	160

6.3	RRC Connection Release with Redirection	160

6.3.1	Introduction	160

6.3.2	Requirements	160

6.3.2.1	RRC connection release with redirection to UTRAN FDD	160

6.3.2.2	RRC connection release with redirection to GERAN	160

6.3.2.3	RRC connection release with redirection to UTRAN TDD	161

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	161

6.4.1	Introduction	161

6.4.2	Requirements	161

6.5	RRC Re-establishment for NB-IoT UEs	162

6.5.1	Introduction	162

6.5.2	Requirements	162

6.5.2.1	UE Re-establishment delay requirement in normal coverage	162

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	163

6.6	Random Access for UE category NB1	163

6.6.1	Introduction	163

6.6.2	Requirements	163

6.6.2.1	Correct behaviour when receiving Random Access Response reception	163

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	164

6.6.2.3	Correct behaviour when receiving a NACK on msg3	164

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	164

6.6.2.5	Correct behaviour when contention Resolution timer expires	164

6.6.2.6	MSG3-based channel quality report for UE Category NB1	164

6.6.3	Requirements for NPRACH configuration	164

6.7	RRC Re-establishment for Cat-M1 UEs	165

6.7.1	Introduction	165

6.7.2	Requirements	165

6.7.2.1	UE Re-establishment delay requirement for CEModeA	165

6.7.2.2	UE Re-establishment delay requirement for CEModeB	166

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	166

6.8.1	Introduction	166

6.8.2	Requirements	166

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	166

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	167

6.9.1	Introduction	167

6.9.2	Requirements	167

7	Timing and signalling characteristics	168

7.1	UE transmit timing	168

7.1.1	Introduction	168

7.1.2	Requirements	168

7.2	UE timer accuracy	169

7.2.1	Introduction	169

7.2.2	Requirements	169

7.3	Timing Advance	170

7.3.1	Introduction	170

7.3.2	Requirements	170

7.3.2.1	Timing Advance adjustment delay	170

7.3.2.2	Timing Advance adjustment accuracy	170

7.4	Cell phase synchronization accuracy (TDD)	170

7.4.1	Definition	170

7.4.2	Minimum requirements	170

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	171

7.5.1	Introduction	171

7.5.2	eNodeB Synchronization Requirements	171

7.5.2.1	Synchronized E-UTRAN	171

7.5.2.2	Non-Synchronized E-UTRAN	171

7.6	Radio Link Monitoring	171

7.6.1	Introduction	171

7.6.2	Requirements	173

7.6.2.1	Minimum requirement when no DRX is used	173

7.6.2.2	Minimum requirement when DRX is used	173

7.6.2.3	Minimum requirement at transitions	175

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	175

7.6.2.5	Minimum requirement under IDC Interference	175

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	175

7.7.1	Introduction	175

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	175

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	176

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Muliple Downlnk SCells	176

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	177

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	178

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	178

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	179

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	179

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	179

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	180

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	181

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	181

7.8	Interruptions with Carrier Aggregation	181

7.8.1	Introduction	181

7.8.2	Requirements	182

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	182

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	182

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	182

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	182

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	182

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	183

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	183

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	183

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	183

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	185

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	185

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	185

7.8.2.13	Interruptions at SRS carrier based switching	186

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	186

7.9.1	Introduction	186

7.9.2	Minimum Requirements for Interband Carrier Aggregation	186

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	187

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	187

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	187

7.10.1	Introduction	187

7.10.2	Requirements	187

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	188

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	188

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	188

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	189

7.11	Radio Link Monitoring for UE Category 0	189

7.11.1	Introduction	189

7.11.2	Requirements for FD-FDD and TDD	190

7.11.2.1	Minimum requirement when no DRX is used	190

7.11.2.2	Minimum requirement when DRX is used	191

7.11.2.3	Minimum requirement at transitions	191

7.11.3	Requirements for HD-FDD	192

7.11.3.1	Minimum requirement when no DRX is used	192

7.11.3.2	Minimum requirement when DRX is used	192

7.11.3.3	Minimum requirement at transitions	193

7.12	Interruptions with Dual Connectivity	193

7.12.1	Introduction	193

7.12.2	Requirements	193

7.12.2.1	Interruptions at PSCell addition/release	193

7.12.2.2	Interruptions at transitions between active and non-active during DRX	193

7.12.2.3	Interruptions at transitions from non-DRX to DRX	194

7.12.2.4	Interruptions at SCell addition/release	194

7.12.2.5	Interruptions at SCell activation/deactivation	194

7.12.2.6	Interruptions during measurements on SCC	195

7.12.2.7	Interruptions at SRS carrier based switching	195

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	196

7.13.1	Definition	196

7.13.2	Minimum requirements	196

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	196

7.14.1	Introduction	196

7.14.2	PSCell Addition Delay Requirement	196

7.14.3	PSCell Release Delay Requirement	197

7.15	Maximum Receive Timing Difference in Dual Connectivity	197

7.15.1	Introduction	197

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	197

7.16	Proximity-based Services	197

7.16.1	Introduction	197

7.16.2	Requirements	197

7.16.2.1	ProSe UE transmission timing	197

7.16.2.1.1	Serving cell or PCell as timing reference	198

7.16.2.1.2	SCell or non-serving cell as timing reference	198

7.16.3	Interruptions with ProSe	198

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	198

7.16.3.2	Interruptions at ProSe Direct Communication configuration	198

7.16.3.3	Interruptions during ProSe Direct Discovery	199

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	199

7.16.3.5	Interruptions during ProSe Direct Communication	199

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	200

7.16.4.1	Measurement and evaluation of selected cell	200

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	200

7.16.5	Selection / Reselection of ProSe relay UE	201

7.16.6	ProSe operation under deactivated SCell	201

7.17	Maximum Transmission Timing Difference in Dual Connectivity	201

7.17.1	Introduction	201

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	202

7.18.1	Introduction	202

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	202

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	202

7.19	Radio Link Monitoring for UE Category M1	202

7.19.1	Introduction	202

7.19.2	Requirements for FD-FDD and TDD CE mode A	202

7.19.2.1	Minimum requirement when no DRX is used	203

7.19.2.2	Minimum requirement when DRX is used	204

7.19.2.3	Minimum requirement at transitions	205

7.19.3	Requirements for HD-FDD with CE mode A	205

7.19.3.1	Minimum requirement when no DRX is used	206

7.19.3.2	Minimum requirement when DRX is used	206

7.19.3.3	Minimum requirement at transitions	207

7.19.4	Requirements for FD-FDD and TDD with CE mode B	207

7.19.4.1	Minimum requirement when no DRX is used	208

7.19.4.2	Minimum requirement when DRX is used	209

7.19.4.3	Minimum requirement at transitions	210

7.19.5	Requirements for HD-FDD with CE mode B	210

7.19.5.1	Minimum requirement when no DRX is used	211

7.19.5.2	Minimum requirement when DRX is used	211

7.19.5.3	Minimum requirement at transitions	212

7.20	UE transmit timing for NB-IoT	212

7.20.1	Introduction	212

7.20.2	Requirements	212

7.21	 UE timer accuracy for NB-IoT	213

7.21.1	Introduction	213

7.21.2	Requirements	213

7.22	Timing Advance for NB-IoT	213

7.22.1	Introduction	213

7.22.2	Requirements	213

7.22.2.1	Timing Advance adjustment delay	213

7.22.2.2	Timing Advance adjustment accuracy	214

7.23	Radio Link Monitoring for Category NB1 UE	214

7.23.1	Introduction	214

7.23.2	Requirements for HD-FDD Category NB1 UE	214

7.23.2.1	Minimum requirement when no DRX is used	214

7.23.2.2	Minimum requirement when DRX is used	215

7.23.2.3	Minimum requirement at transitions	215

7.24	UE transmit timing for Category M1	216

7.24.1	Introduction	216

7.24.2	Requirements	216

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	217

7.25.1	Definition	217

7.25.2	Minimum requirements	217

7.26	UE transmit timing for Category M2	217

7.26.1	Introduction	217

7.26.2	Requirements	217

7.27	UE timer accuracy for category M1	218

7.27.1	Introduction	218

7.27.2	Requirements	218

7.28	Timing Advance for Category M1	218

7.28.1	Introduction	218

7.28.2	Requirements	218

7.29	Interruptions requirements with FeMBMS	218

7.29.1	Introduction	218

7.29.2	Requirements	218

7.30	Numerology switching delay requirements with FeMBMS	218

7.30.1	Introduction	218

7.30.2	Requirements	218

8	UE Measurements Procedures in RRC_CONNECTED State	219

8.1	General Measurement Requirements	219

8.1.1	Introduction	219

8.1.2	Requirements	219

8.1.2.1	UE measurement capability	219

8.1.2.1.1	Monitoring of multiple layers using gaps	223

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	224

8.1.2.1.2	Network controlled small gap	225

8.1.2.2	E-UTRAN intra frequency measurements	227

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	227

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	231

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	235

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	236

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	237

8.1.2.3	E-UTRAN inter frequency measurements	237

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	238

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	243

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	250

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	250

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	250

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	251

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	253

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	254

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	255

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	261

8.1.2.4	Inter RAT measurements	261

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	261

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	266

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	266

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	270

8.1.2.4.5	E-UTRAN FDD – GSM measurements	270

8.1.2.4.6	E-UTRAN TDD – GSM measurements	275

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	275

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	277

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	277

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	277

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	278

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	278

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	278

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	278

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	280

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	280

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	280

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	280

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	281

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	281

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	283

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	283

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	283

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	285

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	287

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	288

8.1.2.6.5	(Void)	290

8.1.2.6.6	(Void)	290

8.1.2.6.7	(Void)	290

8.1.2.6.8	(Void)	290

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	290

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	291

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	292

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	294

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	296

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	297

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	299

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	301

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	302

8.1.2.7	E-UTRAN E-CID Measurements	304

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	304

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	305

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	307

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	307

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	308

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	308

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	311

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	314

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	317

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	321

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	321

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	321

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	321

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	322

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	322

8.2.1	Introduction	322

8.2.2	Requirements	323

8.3	Measurements for E-UTRA carrier aggregation	326

8.3.1	Introduction	326

8.3.2	Measurements of the primary component carrier	326

8.3.3	Measurements of a secondary component carrier	326

8.3.3.1	Measurements of a secondary component carrier with active SCell	326

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	326

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	326

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	327

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	329

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	329

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	329

8.4.1	Introduction	329

8.4.2	Measurements on the primary component carrier	330

8.4.3	Measurements on a secondary component carrier	330

8.4.4	Measurements on both primary component carrier and a secondary component carrier	331

8.4.5	Measurements on different secondary component carriers	332

8.5	Measurements for UE category 0	333

8.5.1	Introduction	333

8.5.2 	Requirements	333

8.5.2.1 	E-UTRAN intra frequency measurements	333

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	333

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	337

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	339

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	342

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	343

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	343

8.6	Discovery signal measurements	344

8.6.1	Introduction	344

8.6.2	Requirements for CRS based discovery signal measurements	345

8.6.2.1	E-UTRAN intra frequency measurements	345

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	345

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	347

8.6.2.2	E-UTRAN inter frequency measurements	350

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	350

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	353

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	355

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	355

8.6.3	Requirements for CSI-RS based discovery signal measurements	356

8.6.3.1	E-UTRAN intra frequency measurements	356

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	356

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	358

8.6.3.2	E-UTRAN inter frequency measurements	361

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	361

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	363

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	366

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	366

8.7	Discovery signal measurements for E-UTRA carrier aggregation	366

8.7.1	Introduction	366

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	367

8.7.2.1	Measurements of the primary component carrier	367

8.7.2.2	Measurements of a secondary component carrier	367

8.7.2.3	Measurements of a secondary component carrier with active SCell	367

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	367

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	367

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	368

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	370

8.7.3.1	Measurements of the primary component carrier	370

8.7.3.2	Measurements of a secondary component carrier	370

8.7.3.3	Measurements of a secondary component carrier with active SCell	370

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	370

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	370

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	372

8.8	Measurements for E-UTRA dual connectivity	373

8.8.1	Introduction	373

8.8.2	Intra-frequency measurements requirements on PCell	373

8.8.3	Intra-frequency measurements requirements on PSCell	374

8.8.4	Inter-frequency and inter-RAT measurement requirements	374

8.8.5	Intra-frequency measurements with autonomous gaps	374

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	374

8.8.5.2	ECGI reporting delay	375

8.8.6	Inter-frequency measurements with autonomous gaps	375

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	375

8.8.6.2	ECGI reporting delay	376

8.8.7	SSTD Measurements	376

8.8.7.1	Introduction	376

8.8.7.2	SSTD Measurement requirements	376

8.8.7.3	SSTD Measurement Reporting Delay	377

8.8.8	Intra-frequency measurements requirements on SCell	377

8.9	MBSFN Measurements	377

8.9.1	Introduction	377

8.9.2	MBSFN RSRP Measurements	377

8.9.3	MBSFN RSRQ Measurements	378

8.9.4	MCH BLER Measurements	378

8.10	Proximity-based Services	378

8.10.1	Introduction	378

8.10.2	Requirements	378

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	378

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	379

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	379

8.11.1	Introduction	379

8.11.2	CRS based discovery signal measurements	380

8.11.2.1	E-UTRAN intra-frequency measurements	380

8.11.2.1.1	Requirements	380

8.11.2.1.1.1	Requirements when no DRX is used	380

8.11.2.1.1.1.1	Measurement Reporting Requirements	381

8.11.2.1.1.2	Requirements when DRX is used	382

8.11.2.1.1.2.1	Measurement Reporting Requirements	384

8.11.2.2	E-UTRAN inter-frequency measurements	385

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	385

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	389

8.11.3	CSI-RS based discovery signal measurements	389

8.11.3.1	E-UTRAN intra-frequency measurements	389

8.11.3.1.1	Requirements	389

8.11.3.1.1.1	Requirements when no DRX is used	389

8.11.3.1.1.1.1	Measurement Reporting Requirements	391

8.11.3.1.1.2	Requirements when DRX is used	391

8.11.3.1.1.2.1	Measurement Reporting Requirements	392

8.11.3.2	E-UTRAN inter-frequency measurements	393

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	393

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	397

8.11.4	RSSI measurements	397

8.11.4.1	E-UTRAN intra-frequency measurements	397

8.11.4.2	E-UTRAN inter-frequency measurements	397

8.11.5	Channel occupancy measurements	398

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	398

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	398

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	398

8.12.1	Introduction	398

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	398

8.12.2.1	Introduction	398

8.12.2.2	Measurements of a secondary component carrier	398

8.12.2.3	Measurements of a secondary component carrier with active SCell	398

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	399

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	399

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	401

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	404

8.12.3.1	Introduction	404

8.12.3.2	Measurements of a secondary component carrier	404

8.12.3.3	Measurements of a secondary component carrier with active SCell	404

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	404

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	404

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	406

8.13	Measurements for UE Category M1	408

8.13.1	Introduction	408

8.13.2	Requirements for UE category M1 with CE mode A	408

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	409

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	409

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	413

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	415

8.13.2.2	Void	418

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	418

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	418

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	421

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	423

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	424

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	424

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	426

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	428

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	429

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	429

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	429

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	430

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	430

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	430

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	431

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	431

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	432

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	433

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	433

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	433

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	437

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	439

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	443

8.13.3	Requirements for UE category M1 with CE mode B	443

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	444

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	444

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	448

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	451

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	456

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	457

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	457

8.13.3.2	Void	458

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	458

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	458

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	460

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	463

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	463

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	463

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	464

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	464

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	465

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	465

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	465

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	466

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	466

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	470

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	473

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	477

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	478

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	478

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	480

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	482

8.14	Measurements for UE category NB1	483

8.14.1	Introduction	483

8.14.2	NB-IoT intra frequency measurements under normal coverage	483

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	483

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	483

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	483

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	483

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	483

8.15	Void	484

8.16	Measurements for UE Category M2	484

8.16.1	Introduction	484

8.16.2	Requirements for UE category M2 with CE mode A	484

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	484

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	484

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	485

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	485

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	486

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	486

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	486

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	486

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	489

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	491

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	492

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	492

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	494

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	496

8.16.3	Requirements for UE category M2 with CE mode B	497

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	497

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	497

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	499

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	502

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	502

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	502

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	504

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	507

8.17	Void	507

8.18	Measurements for non-BL/CE UE	507

8.18.1	Introduction	507

8.18.2	Requirements for non-BL/CE UE with CE Mode B	507

8.18.2.1	E-UTRAN intra frequency measurements	507

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	507

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	508

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	509

9	Measurements performance requirements for UE	509

9.1	E-UTRAN measurements	510

9.1.1	Introduction	510

9.1.2	Intra-frequency RSRP Accuracy Requirements	510

9.1.2.1	Absolute RSRP Accuracy	510

9.1.2.2	Relative Accuracy of RSRP	511

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	512

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	512

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	513

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	514

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	515

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	516

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	517

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	517

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	517

9.1.3	Inter-frequency RSRP Accuracy Requirements	518

9.1.3.1	Absolute RSRP Accuracy	518

9.1.3.2	Relative Accuracy of RSRP	519

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	519

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	520

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	521

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	521

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	521

9.1.4	RSRP Measurement Report Mapping	522

9.1.5	Intra-frequency RSRQ Accuracy Requirements	522

9.1.5.1	Absolute RSRQ Accuracy	522

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	523

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	524

9.1.5.4	Absolute WB-RSRQ Accuracy	525

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	526

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	527

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	527

9.1.6	Inter-frequency RSRQ Accuracy Requirements	527

9.1.6.1	Absolute RSRQ Accuracy	527

9.1.6.2	Relative Accuracy of RSRQ	528

9.1.6.3	Absolute WB-RSRQ Accuracy	529

9.1.6.4	Relative WB-RSRQ Accuracy	529

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	530

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	531

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	531

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	531

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	532

9.1.7	RSRQ Measurement Report Mapping	533

9.1.8	Power Headroom	533

9.1.8.1	Period	533

9.1.8.2	Reporting Delay	533

9.1.8.3	Void	534

9.1.8.4	Report Mapping	534

9.1.9	UE Rx – Tx time difference	534

9.1.9.1	Measurement Requirement	534

9.1.9.2	Measurement Report mapping	535

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	536

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	537

9.1.10	Reference Signal Time Difference (RSTD)	538

9.1.10.1	Intra-Frequency Accuracy Requirement	538

9.1.10.2	Inter-Frequency Accuracy Requirement	539

9.1.10.3	RSTD Measurement Report Mapping	541

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	541

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	542

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	543

9.1.11	Carrier aggregation measurement accuracy	544

9.1.11.1	Primary component carrier accuracy requirement	545

9.1.11.2	Secondary component carrier accuracy requirement	545

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	545

9.1.11.4	Secondary component carrier relative accuracy requirement	545

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	545

9.1.13	Measurement accuracy for UE category 0	546

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	546

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	546

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	547

9.1.14	Accuracy requirements for Discovery Signal Measurements	548

9.1.14.1	Introduction	548

9.1.14.2	RSRP measurements in discovery signal occasions	548

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	548

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	548

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	548

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	549

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	550

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	550

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	550

9.1.14.3.3	CSI-RSRP measurement report mapping	551

9.1.14.4	RSRQ measurements in discovery signal occasions	551

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	551

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	552

9.1.15.1.1	Primary component carrier accuracy requirement	552

9.1.15.1.2	Secondary component carrier accuracy requirement	552

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	552

9.1.15.1.4	Secondary component carrier relative accuracy requirement	552

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	552

9.1.15.2.1	Primary component carrier accuracy requirement	552

9.1.15.2.2	Secondary component carrier accuracy requirement	552

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	552

9.1.15.2.4	Secondary component carrier relative accuracy requirement	552

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	553

9.1.17	RS-SINR Measurements	553

9.1.17.1	Measurement Report Mapping	553

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	554

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	554

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	554

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	554

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	555

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	556

9.1.18.1	Introduction	556

9.1.18.2	RSRP measurements	556

9.1.18.2.1	RSRP measurement report mapping	556

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	556

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	557

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	557

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	558

9.1.18.3	RSRQ measurements	558

9.1.18.3.1	RSRQ measurement report mapping	558

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	558

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	559

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	559

9.1.18.4	CSI-RSRP measurements	560

9.1.18.4.1	CSI-RSRP measurement report mapping	560

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	560

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	560

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	561

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	561

9.1.18.5	RSSI measurements	562

9.1.18.5.1	RSSI measurement report mapping	562

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	562

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	563

9.1.18.6	Channel occupancy measurements	563

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	563

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	563

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	564

9.1.19.1	Introduction	564

9.1.19.2	Accuracy requirements for measurements on SCC	564

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	564

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	564

9.1.20	SFN and Subframe Time Difference (SSTD)	565

9.1.20.1	SSTD Accuracy Requirement	565

9.1.20.2	SSTD Measurement Report Mapping	565

9.1.21	Measurement accuracy for UE category M1	566

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	566

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	567

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	568

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	569

9.1.21.5	RSRP Measurement Report Mapping	570

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	570

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	571

9.1.21.8	RSRQ Measurement Report Mapping	572

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	572

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	573

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	574

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	575

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	576

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	577

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	578

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	579

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	580

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	581

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	582

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	583

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	586

9.1.22	Measurement accuracy for UE Category NB1	589

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	589

9.1.22.2	Void	589

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1 in Normal Mode	589

9.1.22.4	Void	590

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	590

9.1.22.6	Void	590

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	590

9.1.22.8	Void	591

9.1.22.9	NRSRP Measurement Report Mapping	591

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	591

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	592

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	593

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	594

9.1.22.14	NRSRQ Measurement Report Mapping	595

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	596

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	596

9.1.23	Power Headroom for UE Category NB1	597

9.1.23.1	Period	597

9.1.23.2	Reporting Delay	597

9.1.23.3	Report Mapping for UE Category NB1	597

9.1.23.3.1	Void	598

9.1.23.3.2	Void	598

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	598

9.1.24	Void	598

9.1.25	Measurement accuracy for UE category M2	598

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	598

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	599

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	600

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	601

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	603

9.1.26	Measurement Accuracy for non-BL CE UE	604

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	605

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	605

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	606

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	607

9.1.26.5	RSRP Measurement Report Mapping	608

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	608

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	608

9.1.26.8	RSRQ Measurement Report Mapping	609

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	609

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	610

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	611

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	612

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	613

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	613

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	613

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	614

9.2	UTRAN FDD Measurements	615

9.2.1	UTRAN FDD CPICH RSCP	616

9.2.2	Void	616

9.2.3	UTRAN FDD CPICH Ec/No	616

9.3	UTRAN TDD Measurements	617

9.3.1	UTRAN TDD P-CCPCH RSCP	617

9.3.2	Void	617

9.3.3	Void	617

9.4	GSM Measurements	617

9.4.1	GSM carrier RSSI	617

9.5	CDMA2000 1x RTT Measurements	618

9.5.1	CDMA2000 1x RTT Pilot Strength	618

9.6	PCMAX,c	618

9.6.1	Report Mapping	618

9.6.2 	Estimation Period	618

9.6.3	Reporting Delay	618

9.7	IEEE802.11 Measurements	618

9.7.1	WLAN RSSI	619

9.7.2	WLAN RSSI Measurement Report Mapping	619

9.8	MBSFN Measurements	619

9.8.1	Introduction	619

9.8.2	MBSFN RSRP	619

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	619

9.8.2.2	MBSFN RSRP measurement report mapping	620

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	620

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	621

9.8.3	MBSFN RSRQ	621

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	621

9.8.3.2	MBSFN RSRQ measurement report mapping	621

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	622

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	622

9.8.4	MCH BLER	622

9.8.4.1	Measurement report mapping for MCH BLER	622

9.8.4.2	Measurement report mapping for MCH Block Number	623

9.9	ProSe Measurements	624

9.9.1	Introduction	624

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	624

9.9.2.1	Absolute S-RSRP Accuracy	624

9.9.2.2	Relative Accuracy of S-RSRP	625

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	626

9.9.3.1	Absolute SD-RSRP Accuracy	626

9.9.3.2	Relative Accuracy of SD-RSRP	626

9.10	V2X Measurements	627

9.10.1	Introduction	627

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	627

9.10.2.1	Absolute S-RSRP Accuracy	627

9.10.2.2	Relative Accuracy of S-RSRP	628

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	628

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	628

9.10.4	S-RSSI Measurement Accuracy Requirements	629

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	629

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	629

10	Measurements Performance Requirements for E-UTRAN	630

10.1	Received Interference Power	630

10.1.1	Absolute accuracy requirement	630

10.1.2	Relative accuracy requirement	630

10.1.3	Received Interference Power measurement report mapping	630

10.2	Angle of Arrival (AOA)	631

10.2.1	Range/mapping	631

10.3	Timing Advance (TADV)	631

10.3.1	Report mapping	631

11	ProSe Requirements in Any Cell Selection state	632

11.1	Introduction	632

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	632

11.2.1	Introduction	632

11.2.2	ProSe UE transmission timing	632

11.3	Initiation/Cease of SLSS Transmissions	632

11.3.1	Introduction	632

11.3.2	Requirements	632

11.4	Measurements for ProSe in Any Cell Selection State	633

11.4.1	Introduction	633

11.4.2	Requirements	633

11.4.2.1	E-UTRA FDD	633

11.4.2.2	E-UTRA TDD	633

11.5	Selection / Reselection of ProSe Synchronization Reference	634

11.5.1	Introduction	634

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	634

11.5.2.1	Introduction	634

11.5.2.2	Requirements	634

11.6	Void	634

11.7	Selection / Reselection of ProSe relay UE	634

11.7.1	Introduction	634

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	635

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	635

12.1	Introduction	635

12.2	Transmit Timing	635

12.2.1	GNSS as timing reference	635

12.3	Interruption	636

12.4	Reliability of GNSS signal	636

13	V2X Requirements	636

13.1	Introduction	636

13.2	UE Transmit Timing	636

13.2.1	Introduction	636

13.2.2	GNSS as synchronization reference source	636

13.2.3	Serving cell/PCell as synchronization reference source	637

13.2.4	SyncRef UE as synchronization reference source	637

13.3	Initiation/Cease of SLSS Transmissions	637

13.3.1	Introduction	637

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	637

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	638

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	638

13.4	Selection / Reselection of V2X Synchronization Reference Source	638

13.5	Autonomous Resource Selection/Reselection measurements	639

13.5.1	Introduction	639

13.5.2	PSSCH-RSRP measurements	639

13.5.3	S-RSSI measurements	640

13.6	Congestion Control measurements	640

13.7	Interruption	640

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	640

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	640

13.8	Reliability of GNSS signal	641

Annex A (normative): Test Cases	638

A.1	Purpose of annex	638

A.2	Requirement classification for statistical testing	638

A.2.1	Types of requirements in TS 36.133	638

A.2.1.1	Time and delay requirements on UE higher layer actions	638

A.2.1.2	Measurements of power levels, relative powers and time	638

A.2.1.3	Implementation requirements	639

A.2.1.4	Physical layer timing requirements	639

A.3	RRM test configurations	640

A.3.1	Reference Measurement Channels	640

A.3.1.1	PDSCH	640

A.3.1.1.1	FDD	640

A.3.1.1.2	TDD	641

A.3.1.1.3	FDD for UE category 0	643

A.3.1.1.4	HD-FDD for UE category 0	644

A.3.1.1.5	TDD for UE category 0	645

A.3.1.1.6	Frame Structure 3	646

A.3.1.2	PCFICH/PDCCH/PHICH	647

A.3.1.2.1	FDD	647

A.3.1.2.2	TDD	647

A.3.1.2.3	HD-FDD for UE category 0	648

A.3.1.2.4	FS 3	648

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	648

A.3.1.3.1	FDD in CEModeA	649

A.3.1.3.2	HD-FDD in CEModeA	649

A.3.1.3.3	TDD in CEModeA	650

A.3.1.3.4	FDD in CEModeB	650

A.3.1.3.5	HD-FDD in CEModeB	651

A.3.1.3.6	TDD in CEModeB	651

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	652

A.3.1.4.1	FDD in CEModeA	652

A.3.1.4.2	HD-FDD in CEModeA	653

A.3.1.4.3	TDD in CEModeA	654

A.3.1.4.4	FDD in CEModeB	655

A.3.1.4.5	 HD-FDD in CEModeB	656

A.3.1.4.6	TDD in CEModeB	656

A.3.1.5	NPDSCH Reference Channel for UE category NB1	657

A.3.1.5.1	HD-FDD in-band operation	657

A.3.1.5.2	Void	658

A.3.1.5.3	HD-FDD standalone operation	658

A.3.1.5.4	Void	659

A.3.1.5.5	HD-FDD guard band operation	659

A.3.1.5.6	Void	659

A.3.1.6	NPDCCH Reference Channel for UE category NB1	659

A.3.1.6.1	HD-FDD in-band operation	659

A.3.1.6.2	Void	660

A.3.1.6.3	HD-FDD standalone operation	660

A.3.1.6.4	Void	660

A.3.1.6.5	HD-FDD guard band operation	660

A.3.1.6.6	Void	661

A.3.2	OFDMA Channel Noise Generator (OCNG)	661

A.3.2.1	OCNG Patterns for FDD	661

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	662

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	662

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	663

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	664

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	665

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	665

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	666

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	666

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	667

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	667

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	668

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	669

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	670

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	670

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	671

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	671

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	672

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	673

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	673

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	674

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	674

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	675

A.3.2.2	OCNG Patterns for TDD	675

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	676

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	676

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	677

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	677

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	678

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	679

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	680

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	681

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	681

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	682

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	683

A.3.2.3	OCNG Patterns for Narrowband IoT	683

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	685

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	686

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	686

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	687

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	688

A.3.2.4	OCNG Patterns for V2X sidelink	689

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	690

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	690

A.3.3	Reference DRX Configurations	690

A.3.4	ABS Transmission Configurations	691

A.3.4.1	Non-MBSFN ABS Transmission Configurations	691

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	691

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	691

A.3.4.2	MBSFN ABS Transmission Configurations	692

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	692

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	693

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	693

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	693

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	694

A.3.6.1	Introduction	694

A.3.7	Test Cases with Different Channel Bandwidths	694

A.3.7.1	Introduction	694

A.3.7.2	Principle of testing	694

A.3.8	Antenna Configuration	694

A.3.8.1	Antenna connection for 4 Rx capable UEs	694

A.3.8.1.1 Introduction	694

A.3.8.1.2 Principle of testing	694

A.3.8.1.2.1 Single carrier tests	694

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	695

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	696

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	696

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	696

A.3.9.1	Introduction	696

A.3.9.2	Principle of testing	696

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	697

A.3.10.1	Introduction	697

A.3.10.2	Principle of testing	697

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	697

A.3.11.1	Introduction	697

A.3.11.2	Principle of Testing	697

A.3.12	Proximity-based Services	697

A.3.12.1	Introduction	697

A.3.12.2	Reference DRX configurations for ProSe tests	697

A.3.12.3	Test Cases with Different Channel Bandwidths	698

A.3.12.3.1	Introduction	698

A.3.12.3.2	Principle of testing	698

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	698

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	705

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	708

A.3.12.6.1	FDD	708

A.3.12.7	Reference measurement channels for ProSe Direct Communication	708

A.3.12.7.1	FDD	708

A.3.12.8	ProSe Receive Traffic Generator	709

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	709

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	709

A.3.13	Time Offset between Cells	710

A.3.13.1	Introduction	710

A.3.13.2	Definition	710

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	710

A.3.14.1	Introduction	710

A.3.14.2	Principle of testing	710

A.3.15	Dual connectivity test cases with different combination of duplex mode	710

A.3.15.1	Introduction	710

A.3.15.2	Principle of testing	710

A.3.16	Reference PRACH Configurations	711

A.3.17	Listen before talk model	711

A.3.17.1	Introduction	711

A.3.17.2	Definition	711

A.3.18	Reference NPRACH Configurations	712

A.3.19	Dual connectivity test cases with different bandwidth combinations	713

A.3.19.1	Introduction	713

A.3.19.2	Principle of testing	713

A.3.20	Category M1 UE Test Cases	713

A.3.20.1	Introduction	713

A.3.20.2	Principle of Cat-M1 UE Testing	713

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	714

A.3.21.1	Introduction	714

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	715

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	716

A.3.22	Category 1bis UE Test Cases	717

A.3.22.1	Introduction	717

A.3.22.2	Principle of Category 1bis UE Testing	717

A.3.23	Category NB2 UE Test Cases	721

A.3.23.1	Introduction	721

A.3.23.2	Principle of Category NB2 UE Testing	721

A.3.24	V2X sidelink communication	722

A.3.24.1	Introduction	722

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	723

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	727

A.3.25	Category M2 UE Test Cases	728

A.3.25.1	Introduction	728

A.3.25.2	Principle of Cat-M2 UE Testing	728

A.4	E-UTRAN RRC_IDLE state	730

A.4.2	Cell Re-Selection	730

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	730

A.4.2.1.1	Test Purpose and Environment	730

A.4.2.1.2	Test Requirements	731

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	732

A.4.2.2.1	Test Purpose and Environment	732

A.4.2.2.2	Test Requirements	733

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	734

A.4.2.3.1	Test Purpose and Environment	734

A.4.2.3.2	Test Requirements	735

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	736

A.4.2.4.1	Test Purpose and Environment	736

A.4.2.4.2	Test Requirements	737

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	738

A.4.2.5.1	Test Purpose and Environment	738

A.4.2.5.2	Test Requirements	739

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	740

A.4.2.6.1	Test Purpose and Environment	740

A.4.2.6.2	Test Requirements	741

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	742

A.4.2.7.1	Test Purpose and Environment	742

A.4.2.7.2	Test Requirements	743

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	744

A.4.2.8.1	Test Purpose and Environment	744

A.4.2.8.2	Test Requirements	745

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	746

A.4.2.9.1	Test Purpose and Environment	746

A.4.2.9.2	Test Requirements	746

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	746

A.4.2.10.1	Test Purpose and Environment	746

A.4.2.10.2	Test Requirements	750

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	750

A.4.2.11.1	Test Purpose and Environment	750

A.4.2.11.2	Test Requirements	755

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	755

A.4.2.12.1	Test Purpose and Environment	755

A.4.2.12.2	Test Requirements	757

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	758

A.4.2.13.1	Test Purpose and Environment	758

A.4.2.13.2	Test Requirements	759

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	760

A.4.2.14.1	Test Purpose and Environment	760

A.4.2.14.2	Test Requirements	761

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	762

A.4.2.15.1	Test Purpose and Environment	762

A.4.2.15.2	Test Requirements	763

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	764

A.4.2.16.1	Test Purpose and Environment	764

A.4.2.16.2	Test Requirements	765

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	766

A.4.2.17.1	Test Purpose and Environment	766

A.4.2.17.2	Test Requirements	767

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	768

A.4.2.18.1	Test Purpose and Environment	768

A.4.2.18.2	Test Requirements	770

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	771

A.4.2.19.1	Test Purpose and Environment	771

A.4.2.19.2	Test Requirements	773

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	774

A.4.2.20.1	Test Purpose and Environment	774

A.4.2.20.2	Test Requirements	775

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	776

A.4.2.21.1	Test Purpose and Environment	776

A.4.2.21.2	Test Requirements	777

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	778

A.4.2.22.1	Test Purpose and Environment	778

A.4.2.22.2	Test Requirements	779

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	780

A.4.2.23.1	Test Purpose and Environment	780

A.4.2.23.2	Test Requirements	781

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	782

A.4.2.24.1	Test Purpose and Environment	782

A.4.2.24.2	Test Requirements	785

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	786

A.4.2.25.1	Test Purpose and Environment	786

A.4.2.25.2	Test Requirements	787

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	788

A.4.2.26.1	Test Purpose and Environment	788

A.4.2.26.2	Test Requirements	789

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	790

A.4.2.27.1	Test Purpose and Environment	790

A.4.2.27.2	Test Requirements	791

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	792

A.4.2.28.1	Test Purpose and Environment	792

A.4.2.28.2	Test Requirements	793

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	794

A.4.2.29.1	Test Purpose and Environment	794

A.4.2.29.2	Test Requirements	795

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	796

A.4.2.30.1	Test Purpose and Environment	796

A.4.2.30.2	Test Requirements	797

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	798

A.4.2.31.1	Test Purpose and Environment	798

A.4.2.31.2	Test Requirements	799

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	800

A.4.2.32.1	Test Purpose and Environment	800

A.4.2.32.2	Test Requirements	801

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	802

A.4.2.33.1	Test Purpose and Environment	802

A.4.2.33.2	Test Requirements	803

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	804

A.4.2.34.1	Test Purpose and Environment	804

A.4.2.34.2	Test Requirements	805

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	806

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	806

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	806

A.4.3.1.1.1	Test Purpose and Environment	806

A.4.3.1.1.2	Test Requirements	808

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	808

A.4.3.1.2.1	Test Purpose and Environment	808

A.4.3.1.2.2	Test Requirements	811

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	811

A.4.3.1.3.1	Test Purpose and Environment	811

A.4.3.1.3.2	Test Requirements	814

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	814

A.4.3.1.4.1	Test Purpose and Environment	814

A.4.3.1.4.2	Test Requirements	815

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	815

A.4.3.1.5.1	Test Purpose and Environment	815

A.4.3.1.5.2	Test Requirements	818

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	819

A.4.3.2.1	Test Purpose and Environment	819

A.4.3.2.1.1	Void	819

A.4.3.2.1.2	1.28Mcps TDD option	819

A.4.3.2.1.3	Void	821

A.4.3.2.2	Test Requirements	821

A.4.3.2.2.1	1.28Mcps TDD option	821

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	821

A.4.3.2A.1	Test Purpose and Environment	821

A.4.3.2A.2	Test Requirements	825

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	825

A.4.3.3.1	Test Purpose and Environment	825

A.4.3.3.2	Test Requirements	828

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	828

A.4.3.3A.1	Test Purpose and Environment	828

A.4.3.3A.2	Test Requirements	832

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	833

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	833

A.4.3.4.1.1	Test Purpose and Environment	833

A.4.3.4.1.2	Test Requirements	835

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	836

A.4.3.4.2.1	Test Purpose and Environment	836

A.4.3.4.2.2	Test Requirements	838

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	838

A.4.3.4.3.1	Test Purpose and Environment	838

A.4.3.4.3.2	Test Requirements	841

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	841

A.4.3.4.4.1	Test Purpose and Environment	841

A.4.3.4.4.2	Test Requirements	845

A.4.4	E-UTRAN to GSM Cell Re-Selection	845

A.4.4.1	E-UTRAN FDD – GSM:	845

A.4.4.1.1	Test Purpose and Environment	845

A.4.4.1.2	Test Requirements	847

A.4.4.2	E-UTRAN TDD – GSM:	847

A.4.4.2.1	Test Purpose and Environment	847

A.4.4.2.2	Test Requirements	849

A.4.5	E-UTRAN to HRPD Cell Re-Selection	850

A.4.5.1	E-UTRAN FDD – HRPD	850

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	850

A.4.5.1.1.1	Test Purpose and Environment	850

A.4.5.1.1.2	Test Requirements	852

A.4.5.2	E-UTRAN TDD – HRPD	852

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	852

A.4.5.2.1.1	Test Purpose and Environment	852

A.4.5.2.1.2	Test Requirements	855

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	855

A.4.6.1	E-UTRAN FDD – cdma2000 1X	855

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	855

A.4.6.1.1.1	Test Purpose and Environment	855

A.4.6.1.1.2	Test Requirements	858

A.4.6.2	E-UTRAN TDD – cdma2000 1X	858

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	858

A.4.6.2.1.1	Test Purpose and Environment	858

A.4.6.2.1.2	Test Requirements	861

A.4.7	Idle State Positioning Measurement for UE category NB1	861

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	861

A.4.7.1.1	Test Purpose and Environment	861

A.4.7.1.2	Test Requirements	866

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	867

A.4.7.2.1	Test Purpose and Environment	867

A.4.7.4.2	Test Requirements	871

A.5	E-UTRAN RRC CONNECTED Mode Mobility	872

A.5.1	E-UTRAN Handover	872

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	872

A.5.1.1.1	Test Purpose and Environment	872

A.5.1.1.2	Test Requirements	874

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	874

A.5.1.2.1	Test Purpose and Environment	874

A.5.1.2.2	Test Requirements	876

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	876

A.5.1.3.1	Test Purpose and Environment	876

A.5.1.3.2	Test Requirements	878

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	878

A.5.1.4.1	Test Purpose and Environment	878

A.5.1.4.2	Test Requirements	880

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	880

A.5.1.5.1	Test Purpose and Environment	880

A.5.1.5.2	Test Requirements	882

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	882

A.5.1.6.1	Test Purpose and Environment	882

A.5.1.6.2	Test Requirements	884

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	884

A.5.1.7.1	Test Purpose and Environment	884

A.5.1.7.2	Test Requirements	887

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	887

A.5.1.8.1	Test Purpose and Environment	887

A.5.1.8.2	Test Requirements	890

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	890

A.5.1.9.1	Test Purpose and Environment	890

A.5.1.9.2	Test Requirements	891

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	891

A.5.1.10.1	Test Purpose and Environment	891

A.5.1.10.2	Test Requirements	893

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	893

A.5.1.11.1	Test Purpose and Environment	893

A.5.1.11.2	Test Requirements	895

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	895

A.5.1.12.1	Test Purpose and Environment	895

A.5.1.12.2	Test Requirements	897

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	897

A.5.1.13.1	Test Purpose and Environment	897

A.5.1.13.2	Test Requirements	899

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	900

A.5.1.14.1	Test Purpose and Environment	900

A.5.1.14.2	Test Requirements	901

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	902

A.5.1.15.1	Test Purpose and Environment	902

A.5.1.15.2	Test Requirements	903

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	904

A.5.1.16.1	Test Purpose and Environment	904

A.5.1.16.2	Test Requirements	905

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	906

A.5.1.17.1	Test Purpose and Environment	906

A.5.1.17.2	Test Requirements	907

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	908

A.5.1.18.1	Test Purpose and Environment	908

A.5.1.18.2	Test Requirements	909

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	910

A.5.1.19.1	Test Purpose and Environment	910

A.5.1.19.2	Test Requirements	911

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	912

A.5.1.20.1	Test Purpose and Environment	912

A.5.1.20.2	Test Requirements	913

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	914

A.5.1.21.1	Test Purpose and Environment	914

A.5.1.21.2	Test Requirements	915

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	915

A.5.1.22.1	Test Purpose and Environment	915

A.5.1.22.2	Test Requirements	917

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	917

A.5.1.23.1	Test Purpose and Environment	917

A.5.1.23.2	Test Requirements	919

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	919

A.5.1.24.1	Test Purpose and Environment	919

A.5.1.24.2	Test Requirements	921

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	921

A.5.1.25.1	Test Purpose and Environment	921

A.5.1.25.2	Test Requirements	923

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	923

A.5.1.26.1	Test Purpose and Environment	923

A.5.1.26.2	Test Requirements	925

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	925

A.5.1.27.1	Test Purpose and Environment	925

A.5.1.27.2	Test Requirements	927

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	928

A.5.1.28.1	Test Purpose and Environment	928

A.5.1.28.2	Test Requirements	929

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	930

A.5.1.29.1	Test Purpose and Environment	930

A.5.1.29.2	Test Requirements	931

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	932

A.5.1.30.1	Test Purpose and Environment	932

A.5.1.30.2	Test Requirements	933

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	934

A.5.1.31.1	Test Purpose and Environment	934

A.5.1.31.2	Test Requirements	935

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	936

A.5.1.32.1	Test Purpose and Environment	936

A.5.1.32.2	Test Requirements	937

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	938

A.5.1.33.1	Test Purpose and Environment	938

A.5.1.13.2	Test Requirements	939

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	940

A.5.1.34.1	Test Purpose and Environment	940

A.5.1.34.2	Test Requirements	941

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	942

A.5.1.35.1	Test Purpose and Environment	942

A.5.1.35.2	Test Requirements	943

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	944

A.5.1.36.1	Test Purpose and Environment	944

A.5.1.36.2	Test Requirements	945

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	946

A.5.1.37.1	Test Purpose and Environment	946

A.5.1.37.2	Test Requirements	947

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	948

A.5.1.38.1	Test Purpose and Environment	948

A.5.1.38.2	Test Requirements	949

A.5.2	E-UTRAN Handover to other RATs	950

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	950

A.5.2.1.1	Test Purpose and Environment	950

A.5.2.1.2	Test Requirements	951

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	952

A.5.2.2.1	Test Purpose and Environment	952

A.5.2.2.2	Test Requirements	955

A.5.2.3 E-UTRAN FDD- GSM Handover	955

A.5.2.3.1	Test Purpose and Environment	955

A.5.2.3.2	Test Requirements	956

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	957

A.5.2.4.1	Test Purpose and Environment	957

A.5.2.4.1.1	Void	957

A.5.2.4.1.2	1.28 Mcps TDD option	957

A.5.2.4.1.3	Void	959

A.5.2.4.2	Test Requirements	959

A.5.2.4.2.1	Void	959

A.5.2.4.2.2	1.28 Mcps TDD option	959

A.5.2.4.2.3	Void	959

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	959

A.5.2.5.1	Test Purpose and Environment	959

A.5.2.5.1.3	Void	962

A.5.2.5.2	Test Requirements	962

A.5.2.5.2.1	Void	962

A.5.2.5.2.2	1.28 Mcps TDD option	962

A.5.2.5.2.3	Void	962

A.5.2.6	E-UTRAN TDD - GSM Handover	962

A.5.2.6.1	Test Purpose and Environment	962

A.5.2.6.2	Test Requirements	964

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	965

A.5.2.7.1	Test Purpose and Environment	965

A.5.2.7.2	Test Requirements	967

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	967

A.5.2.8.1	Test Purpose and Environment	967

A.5.2.8.2	Test Requirements	968

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	969

A.5.2.9.1	Test Purpose and Environment	969

A.5.2.9.2	Test Requirements	970

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	971

A.5.2.10.1	Test Purpose and Environment	971

A.5.2.10.2	Test Requirements	973

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	973

A.5.2.10A.1	Test Purpose and Environment	973

A.5.2.10A.2	Test Requirements	976

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	976

A.5.2.10B.1	Test Purpose and Environment	976

A.5.2.10B.2	Test Requirements	979

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	979

A.5.2.11.1	Test Purpose and Environment	979

A.5.2.11.2	Test Requirements	979

A.5.3	E-UTRAN Handover to Non-3GPP RATs	980

A.5.3.1	E-UTRAN FDD – HRPD Handover	980

A.5.3.1.1	Test Purpose and Environment	980

A.5.3.1.2	Test Requirements	982

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	982

A.5.3.2.1	Test Purpose and Environment	982

A.5.3.2.2	Test Requirements	985

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	985

A.5.3.3.1	Test Purpose and Environment	985

A.5.3.3.2	Test Requirements	987

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	988

A.5.3.4.1	Test Purpose and Environment	988

A.5.3.4.2	Test Requirements	989

A.5.3.5	E-UTRAN TDD – HRPD Handover	990

A.5.3.5.1	Test Purpose and Environment	990

A.5.3.5.2	Test Requirements	992

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	993

A.5.3.6.1	Test Purpose and Environment	993

A.5.3.6.2	Test Requirements	996

A.6	RRC Connection Control	996

A.6.1	RRC Re-establishment	996

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	996

A.6.1.1.1	Test Purpose and Environment	996

A.6.1.1.2	Test Requirements	997

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	998

A.6.1.2.1	Test Purpose and Environment	998

A.6.1.2.2	Test Requirements	999

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1000

A.6.1.3.1	Test Purpose and Environment	1000

A.6.1.3.2	Test Requirements	1001

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1002

A.6.1.4.1	Test Purpose and Environment	1002

A.6.1.4.2	Test Requirements	1003

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1004

A.6.1.5.1	Test Purpose and Environment	1004

A.6.1.5.2	Test Requirements	1004

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1004

A.6.1.6.1	Test Purpose and Environment	1004

A.6.1.6.2	Test Requirements	1006

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1007

A.6.1.7.1	Test Purpose and Environment	1007

A.6.1.7.2	Test Requirements	1008

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1009

A.6.1.8.1	Test Purpose and Environment	1009

A.6.1.8.2	Test Requirements	1010

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1011

A.6.1.9.1	Test Purpose and Environment	1011

A.6.1.9.2	Test Requirements	1012

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1013

A.6.1.10.1	Test Purpose and Environment	1013

A.6.1.10.2	Test Requirements	1014

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1015

A.6.1.11.1	Test Purpose and Environment	1015

A.6.1.11.2	Test Requirements	1016

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1017

A.6.1.12.1	Test Purpose and Environment	1017

A.6.1.12.2	Test Requirements	1018

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1019

A.6.1.13.1	Test Purpose and Environment	1019

A.6.1.13.2	Test Requirements	1020

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1021

A.6.1.14.1	Test Purpose and Environment	1021

A.6.1.14.2	Test Requirements	1022

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1023

A.6.1.15.1	Test Purpose and Environment	1023

A.6.1.15.2	Test Requirements	1025

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1026

A.6.1.16.1	Test Purpose and Environment	1026

A.6.1.16.2	Test Requirements	1028

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1029

A.6.1.17.1	Test Purpose and Environment	1029

A.6.1.17.2	Test Requirements	1030

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1031

A.6.1.18.1	Test Purpose and Environment	1031

A.6.1.18.2	Test Requirements	1032

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1033

A.6.1.19.1	Test Purpose and Environment	1033

A.6.1.19.2	Test Requirements	1034

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1035

A.6.1.20.1	Test Purpose and Environment	1035

A.6.1.20.2	Test Requirements	1036

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1037

A.6.1.21.1	Test Purpose and Environment	1037

A.6.1.21.2	Test Requirements	1038

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1039

A.6.1.22.1	Test Purpose and Environment	1039

A.6.1.22.2	Test Requirements	1040

A.6.2	Random Access	1041

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1041

A.6.2.1.1	Test Purpose and Environment	1041

A.6.2.1.2	Test Requirements	1043

A.6.2.1.2.1	Random Access Response Reception	1043

A.6.2.1.2.2	No Random Access Response Reception	1043

A.6.2.1.2.3	Receiving a NACK on msg3	1043

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1043

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1044

A.6.2.1.2.6	Contention Resolution Timer expiry	1044

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1044

A.6.2.2.1	Test Purpose and Environment	1044

A.6.2.2.2	Test Requirements	1045

A.6.2.2.2.1	Random Access Response Reception	1046

A.6.2.2.2.2	No Random Access Response Reception	1046

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1046

A.6.2.3.1	Test Purpose and Environment	1046

A.6.2.3.2	Test Requirements	1048

A.6.2.3.2.1	Random Access Response Reception	1048

A.6.2.3.2.2	No Random Access Response reception	1048

A.6.2.3.2.3	Receiving a NACK on msg3	1048

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1049

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1049

A.6.2.3.2.6	Contention Resolution Timer expiry	1049

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1049

A.6.2.4.1	Test Purpose and Environment	1049

A.6.2.4.2	Test Requirements	1051

A.6.2.4.2.1	Random Access Response Reception	1051

A.6.2.4.2.2	No Random Access Response Reception	1051

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1051

A.6.2.5.1	Test Purpose and Environment	1051

A.6.2.5.2	Test Requirements	1052

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1052

A.6.2.6.1	Test Purpose and Environment	1052

A.6.2.6.2	Test Requirements	1052

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1053

A.6.2.7.1	Test Purpose and Environment	1053

A.6.2.7.2	Test Requirements	1055

A.6.2.7.2.1	Random Access Response Reception	1055

A.6.2.7.2.2	No Random Access Response Reception	1055

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1055

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1056

A.6.2.8.1	Test Purpose and Environment	1056

A.6.2.8.2	Test Requirements	1058

A.6.2.8.2.1	Random Access Response Reception	1058

A.6.2.8.2.2	No Random Access Response Reception	1058

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1058

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1059

A.6.2.9.1	Test Purpose and Environment	1059

A.6.2.9.2	Test Requirements	1062

A.6.2.9.2.1	Random Access Response Reception	1062

A.6.2.9.2.2	No Random Access Response Reception	1063

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1063

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1064

A.6.2.10.1	Test Purpose and Environment	1064

A.6.2.10.2	Test Requirements	1066

A.6.2.10.2.1	Random Access Response Reception	1066

A.6.2.10.2.2	No Random Access Response Reception	1066

A.6.2.10.2.3	Receiving a NACK on msg3	1067

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1067

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1067

A.6.2.10.2.6	Contention Resolution Timer expiry	1067

A.6.2.10.2.7	PRACH Resource Selection	1067

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1067

A.6.2.11.1	Test Purpose and Environment	1067

A.6.2.11.2	Test Requirements	1069

A.6.2.11.2.1	Random Access Response Reception	1069

A.6.2.11.2.2	No Random Access Response Reception	1070

A.6.2.11.2.3	Receiving a NACK on msg3	1070

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1070

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1070

A.6.2.11.2.6	Contention Resolution Timer expiry	1070

A.6.2.11.2.7	PRACH Resource Selection	1071

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1071

A.6.2.12.1	Test Purpose and Environment	1071

A.6.2.12.2	Test Requirements	1073

A.6.2.12.2.1	Random Access Response Reception	1073

A.6.2.12.2.2	No Random Access Response Reception	1074

A.6.2.12.2.3	Receiving a NACK on msg3	1074

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1074

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1074

A.6.2.12.2.6	Contention Resolution Timer expiry	1074

A.6.2.12.2.7	PRACH Resource Selection	1075

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1075

A.6.2.13.1	Test Purpose and Environment	1075

A.6.2.13.2	Test Requirements	1077

A.6.2.13.2.1	Random Access Response Reception	1077

A.6.2.13.2.2	No Random Access Response Reception	1077

A.6.2.13.2.3	Receiving a NACK on msg3	1078

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1078

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1078

A.6.2.13.2.6	Contention Resolution Timer expiry	1078

A.6.2.13.2.7	PRACH Resource Selection	1078

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1078

A.6.2.14.1	Test Purpose and Environment	1078

A.6.2.14.2	Test Requirements	1080

A.6.2.14.2.1	Random Access Response Reception	1080

A.6.2.14.2.2	No Random Access Response Reception	1081

A.6.2.14.2.3	Receiving a NACK on msg3	1081

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1081

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1081

A.6.2.14.2.6	Contention Resolution Timer expiry	1081

A.6.2.14.2.7	PRACH Resource Selection	1082

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1082

A.6.2.15.1	Test Purpose and Environment	1082

A.6.2.15.2	Test Requirements	1084

A.6.2.15.2.1	Random Access Response Reception	1084

A.6.2.15.2.2	No Random Access Response Reception	1085

A.6.2.15.2.3	Receiving a NACK on msg3	1085

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1085

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1085

A.6.2.15.2.6	Contention Resolution Timer expiry	1085

A.6.2.15.2.7	PRACH Resource Selection	1086

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1086

A.6.2.16.1	Test Purpose and Environment	1086

A.6.2.16.2	Test Requirements	1089

A.6.2.16.2.1	Random Access Response Reception	1089

A.6.2.16.2.2	No Random Access Response Reception	1090

A.6.2.16.2.3	Receiving a NACK on msg3	1090

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1090

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1090

A.6.2.16.2.6	Contention Resolution Timer expiry	1090

A.6.2.16.2.7	NPRACH Resource Selection	1090

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1091

A.6.2.17.1	Test Purpose and Environment	1091

A.6.2.17.2	Test Requirements	1094

A.6.2.17.2.1	Random Access Response Reception	1094

A.6.2.17.2.2	No Random Access Response Reception	1095

A.6.2.17.2.3	Receiving a NACK on msg3	1095

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1095

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1095

A.6.2.17.2.6	Contention Resolution Timer expiry	1095

A.6.2.17.2.7	NPRACH Resource Selection	1095

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1096

A.6.2.18.1	Test Purpose and Environment	1096

A.6.2.18.2	Test Requirements	1099

A.6.2.18.2.1	Random Access Response Reception	1099

A.6.2.18.2.2	No Random Access Response Reception	1100

A.6.2.18.2.3	Receiving a NACK on msg3	1100

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1100

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1100

A.6.2.18.2.6	Contention Resolution Timer expiry	1100

A.6.2.18.2.7	NPRACH Resource Selection	1100

A.6.3	RRC Connection Release with Redirection	1101

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1101

A.6.3.1.1	Test Purpose and Environment	1101

A.6.3.1.2	Test Requirements	1103

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1103

A.6.3.2.1	Test Purpose and Environment	1103

A.6.3.2.2	Test Requirements	1105

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1105

A.6.3.3.1	Test Purpose and Environment	1105

A.6.3.3.2	Test Requirements	1106

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1107

A.6.3.4.1	Test Purpose and Environment	1107

A.6.3.4.2	Test Requirements	1108

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1109

A.6.3.5.1	Test Purpose and Environment	1109

A.6.3.5.2	Test Requirements	1111

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1111

A.6.3.6.1	Test Purpose and Environment	1111

A. 6.3.6.2	Test Requirements	1114

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1114

A.6.3.7.1	Test Purpose and Environment	1114

A.6.3.7.2	Test Requirements	1117

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1117

A.6.3.8.1	Test Purpose and Environment	1117

A.6.3.8.2	Test Requirements	1120

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1120

A.6.3.9.1	Test Purpose and Environment	1120

A.6.3.9.2	Test Requirements	1122

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1122

A.6.3.10.1	Test Purpose and Environment	1122

A.6.3.10.2	Test Requirements	1124

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1124

A.6.3.11.1	Test Purpose and Environment	1124

A.6.3.11.2	Test Requirements	1126

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1126

A.6.3.12.1	Test Purpose and Environment	1126

A.6.3.12.2	Test Requirements	1129

A.7	Timing and Signalling Characteristics	1120

A.7.1	UE Transmit Timing	1120

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1120

A.7.1.1.1	Test Purpose and Environment	1120

A.7.1.1.2	Test Requirements	1121

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1122

A.7.1.2.1	Test Purpose and Environment	1122

A.7.1.2.2	Test Requirements	1124

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1125

A.7.1.3.1	Test Purpose and Environment	1125

A.7.1.3.2	Test Requirements	1127

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1128

A.7.1.4.1	Test Purpose and Environment	1128

A.7.1.4.2	Test Requirements	1130

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1131

A.7.1.4A.1	Test Purpose and Environment	1131

A.7.1.4A.2	Test Requirements	1131

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1131

A.7.1.5.1	Test Purpose and Environment	1131

A.7.1.5.2	Test Requirements	1132

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1132

A.7.1.6.1	Test Purpose and Environment	1132

A.7.1.6.2	Test Requirements	1134

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1135

A.7.1.7.1	Test Purpose and Environment	1135

A.7.1.7.2	Test Requirements	1136

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1137

A.7.1.7A.1	Test Purpose and Environment	1137

A.7.1.7A.2	Test Requirements	1137

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1137

A.7.1.7B.1	Test Purpose and Environment	1137

A.7.1.7B.2	Test Requirements	1137

A.7.1.8	Void	1138

A.7.1.8.1	Void	1138

A.7.1.8.2	Void	1138

A.7.1.9	Void	1138

A.7.1.9.1	Void	1138

A.7.1.9.2	Void	1138

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1138

A.7.1.10.1	Test Purpose and Environment	1138

A.7.1.10.2	Test Requirements	1140

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1140

A.7.1.11.1	Test Purpose and Environment	1140

A.7.1.11.2	Test Requirements	1142

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1142

A.7.1.12.1	Test Purpose and Environment	1142

A.7.1.12.2	Test Requirements	1144

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1144

A.7.1.13.1	Test Purpose and Environment	1144

A.7.1.13.2	Test Requirements	1146

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1147

A.7.1.14.1	Test Purpose and Environment	1147

A.7.1.14.2	Test Requirements	1148

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1148

A.7.1.15.1	Test Purpose and Environment	1148

A.7.1.15.2	Test Requirements	1149

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1150

A.7.1.16.1	Test Purpose and Environment	1150

A.7.1.16.2	Test Requirements	1151

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1152

A.7.1.17.1	Test Purpose and Environment	1152

A.7.1.17.2	Test Requirements	1154

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1155

A.7.1.18.1	Test Purpose and Environment	1155

A.7.1.18.2	Test Requirements	1157

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1158

A.7.1.19.1	Test Purpose and Environment	1158

A.7.1.19.2	Test Requirements	1159

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1159

A.7.1.20.1	Test Purpose and Environment	1159

A.7.1.20.2	Test Requirements	1161

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1161

A.7.1.21.1	Test Purpose and Environment	1161

A.7.1.21.2	Test Requirements	1163

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1163

A.7.1.22.1	Test Purpose and Environment	1163

A.7.1.22.2	Test Requirements	1165

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1165

A.7.1.23.1	Test Purpose and Environment	1165

A.7.1.23.2	Test Requirements	1167

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1167

A.7.1.24.1	Test Purpose and Environment	1167

A.7.1.24.2	Test Requirements	1168

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1169

A.7.1.25.1	Test Purpose and Environment	1169

A.7.1.25.2	Test Requirements	1170

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1171

A.7.1.26.1	Test Purpose and Environment	1171

A.7.1.26.2	Test Requirements	1172

A.7.2	UE Timing Advance	1173

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1173

A.7.2.1.1	Test Purpose and Environment	1173

A.7.2.1.2	Test Requirements	1175

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1175

A.7.2.2.1	Test Purpose and Environment	1175

A.7.2.2.2	Test Requirements	1177

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1177

A.7.2.3.1	Test Purpose and Environment	1177

A.7.2.3.2	Test Requirements	1178

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1178

A.7.2.4.1	Test Purpose and Environment	1178

A.7.2.4.2	Test Requirements	1180

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1180

A.7.2.5.1	Test Purpose and Environment	1180

A.7.2.5.2	Test Requirements	1182

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1182

A.7.2.5A.1	Test Purpose and Environment	1182

A.7.2.5A.2	Test Requirements	1183

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1183

A.7.2.5B.1	Test Purpose and Environment	1183

A.7.2.5B.2	Test Requirements	1183

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1183

A.7.2.6.1	Test Purpose and Environment	1183

A.7.2.6.2	Test Requirements	1186

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1186

A.7.2.7.1	Test Purpose and Environment	1186

A.7.2.7.2	Test Requirements	1188

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1188

A.7.2.8.1	Test Purpose and Environment	1188

A.7.2.8.2	Test Requirements	1190

A.7.2.9.2	Test Requirements	1191

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1192

A.7.2.10.1	Test Purpose and Environment	1192

A.7.2.10.2	Test Requirements	1193

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1193

A.7.2.11.1	Test Purpose and Environment	1193

A.7.2.11.2	Test Requirements	1195

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1195

A.7.2.12.1	Test Purpose and Environment	1195

A.7.2.12.2	Test Requirements	1197

A.7.3	Radio Link Monitoring	1197

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1198

A.7.3.1.1	Test Purpose and Environment	1198

A.7.3.1.2	Test Requirements	1202

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1202

A.7.3.2.1	Test Purpose and Environment	1202

A.7.3.2.2	Test Requirements	1206

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1206

A.7.3.3.1	Test Purpose and Environment	1206

A.7.3.3.2	Test Requirements	1210

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1210

A.7.3.4.1	Test Purpose and Environment	1210

A.7.3.4.2	Test Requirements	1214

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1214

A.7.3.5.1	Test Purpose and Environment	1214

A.7.3.5.2	Test Requirements	1217

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1218

A.7.3.6.1	Test Purpose and Environment	1218

A.7.3.6.2	Test Requirements	1221

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1221

A.7.3.7.1	Test Purpose and Environment	1221

A.7.3.7.2	Test Requirements	1224

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1224

A.7.3.8.1	Test Purpose and Environment	1224

A.7.3.8.2	Test Requirements	1227

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1227

A.7.3.9.2	Test Requirements	1230

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1230

A.7.3.10.1	Test Purpose and Environment	1230

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1234

A.7.3.11.1	Test Purpose and Environment	1234

A.7.3.11.2	Test Requirements	1239

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1239

A.7.3.12.1	Test Purpose and Environment	1239

A.7.3.12.2	Test Requirements	1244

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1244

A.7.3.13.1	Test Purpose and Environment	1244

A.7.3.13.2	Test Requirements	1247

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1247

A.7.3.14.1	Test Purpose and Environment	1247

A.7.3.14.2	Test Requirements	1250

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1250

A.7.3.15.1	Test Purpose and Environment	1250

A.7.3.15.2	Test Requirements	1254

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1254

A.7.3.16.1	Test Purpose and Environment	1254

A.7.3.16.2	Test Requirements	1259

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1259

A.7.3.17.1	Test Purpose and Environment	1259

A.7.3.17.2	Test Requirements	1263

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1263

A.7.3.18.1	Test Purpose and Environment	1263

A.7.3.18.2	Test Requirements	1267

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1267

A.7.3.19.1	Test Purpose and Environment	1267

A.7.3.19.2	Test Requirements	1272

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1272

A.7.3.20.1	Test Purpose and Environment	1272

A.7.3.20.2	Test Requirements	1277

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1277

A.7.3.21.1	Test Purpose and Environment	1277

A.7.3.21.2	Test Requirements	1282

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1282

A.7.3.22.1	Test Purpose and Environment	1282

A.7.3.22.2	Test Requirements	1287

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1287

A.7.3.23.1	Test Purpose and Environment	1287

A.7.3.23.2	Test Requirements	1288

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1288

A.7.3.24.1	Test Purpose and Environment	1288

A.7.3.24.2	Test Requirements	1289

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1289

A.7.3.25.1	Test Purpose and Environment	1289

A.7.3.25.2	Test Requirements	1290

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1290

A.7.3.26.1	Test Purpose and Environment	1290

A.7.3.26.2	Test Requirements	1293

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1293

A.7.3.27.1	Test Purpose and Environment	1293

A.7.3.27.2	Test Requirements	1296

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1296

A.7.3.28.1	Test Purpose and Environment	1296

A.7.3.28.2	Test Requirements	1299

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1299

A.7.3.29.1	Test Purpose and Environment	1299

A.7.3.29.2	Test Requirements	1302

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1302

A.7.3.30.1	Test Purpose and Environment	1302

A.7.3.30.2	Test Requirements	1305

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1305

A.7.3.31.1	Test Purpose and Environment	1305

A.7.3.31.2	Test Requirements	1308

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1308

A.7.3.32.1	Test Purpose and Environment	1308

A.7.3.32.2	Test Requirements	1311

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1311

A.7.3.33.1	Test Purpose and Environment	1311

A.7.3.33.2	Test Requirements	1314

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1314

A.7.3.34.1	Test Purpose and Environment	1314

A.7.3.34.2	Test Requirements	1317

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1317

A.7.3.35.1	Test Purpose and Environment	1317

A.7.3.35.2	Test Requirements	1320

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1320

A.7.3.36.1	Test Purpose and Environment	1320

A.7.3.36.2	Test Requirements	1323

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1323

A.7.3.37.1	Test Purpose and Environment	1323

A.7.3.37.2	Test Requirements	1326

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1326

A.7.3.38.1	Test Purpose and Environment	1326

A.7.3.38.2	Test Requirements	1329

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1329

A.7.3.39.1	Test Purpose and Environment	1329

A.7.3.39.2	Test Requirements	1332

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1333

A.7.3.40.1	Test Purpose and Environment	1333

A.7.3.40.2	Test Requirements	1335

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1336

A.7.3.41.1	Test Purpose and Environment	1336

A.7.3.41.2	Test Requirements	1339

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1339

A.7.3.42.1	Test Purpose and Environment	1339

A.7.3.42.2	Test Requirements	1342

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1342

A.7.3.43.1	Test Purpose and Environment	1342

A.7.3.43.2	Test Requirements	1345

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1345

A.7.3.44.1	Test Purpose and Environment	1345

A.7.3.44.2	Test Requirements	1348

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1349

A.7.3.45.1	Test Purpose and Environment	1349

A.7.3.45.2	Test Requirements	1351

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1352

A.7.3.46.1	Test Purpose and Environment	1352

A.7.3.46.2	Test Requirements	1355

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1355

A.7.3.47.1	Test Purpose and Environment	1355

A.7.3.47.2	Test Requirements	1358

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1358

A.7.3.48.1	Test Purpose and Environment	1358

A.7.3.48.2	Test Requirements	1361

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1361

A.7.3.49.1	Test Purpose and Environment	1361

A.7.3.49.2	Test Requirements	1364

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1364

A.7.3.50.1	Test Purpose and Environment	1364

A.7.3.50.2	Test Requirements	1367

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1367

A.7.3.51.1	Test Purpose and Environment	1367

A.7.3.51.2	Test Requirements	1370

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1370

A.7.3.52.1	Test Purpose and Environment	1370

A.7.3.52.2	Test Requirements	1373

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1373

A.7.3.53.1	Test Purpose and Environment	1373

A.7.3.53.2	Test Requirements	1376

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1376

A.7.3.54.1	Test Purpose and Environment	1376

A.7.3.54.2	Test Requirements	1379

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1379

A.7.3.55.1	Test Purpose and Environment	1379

A.7.3.55.2	Test Requirements	1382

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1382

A.7.3.56.1	Test Purpose and Environment	1382

A.7.3.56.2	Test Requirements	1385

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1385

A.7.3.57.1	Test Purpose and Environment	1385

A.7.3.57.2	Test Requirements	1388

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1388

A.7.3.58.1	Test Purpose and Environment	1388

A.7.3.58.2	Test Requirements	1391

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1391

A.7.3.59.1	Test Purpose and Environment	1391

A.7.3.59.2	Test Requirements	1394

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1394

A.7.3.60.1	Test Purpose and Environment	1394

A.7.3.60.2	Test Requirements	1398

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1398

A.7.3.61.1	Test Purpose and Environment	1398

A.7.3.61.2	Test Requirements	1402

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1402

A.7.3.62.1	Test Purpose and Environment	1402

A.7.3.62.2	Test Requirements	1406

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1406

A.7.3.63.1	Test Purpose and Environment	1406

A.7.3.63.2	Test Requirements	1410

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1410

A.7.3.64.1	Test Purpose and Environment	1410

A.7.3.64.2	Test Requirements	1413

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1414

A.7.3.65.1	Test Purpose and Environment	1414

A.7.3.65.2	Test Requirements	1417

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1418

A.7.3.66.1	Test Purpose and Environment	1418

A.7.3.66.2	Test Requirements	1420

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1421

A.7.3.67.1	Test Purpose and Environment	1421

A.7.3.67.2	Test Requirements	1424

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1425

A.7.3.68.1	Test Purpose and Environment	1425

A.7.3.68.2	Test Requirements	1427

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1427

A.7.3.69.1	Test Purpose and Environment	1427

A.7.3.69.2	Test Requirements	1429

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1429

A.7.3.70.1	Test Purpose and Environment	1429

A.7.3.70.2	Test Requirements	1431

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1431

A.7.3.71.1	Test Purpose and Environment	1431

A.7.3.71.2	Test Requirements	1434

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1434

A.7.3.72.1	Test Purpose and Environment	1434

A.7.3.72.2	Test Requirements	1436

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1436

A.7.3.73.1	Test Purpose and Environment	1436

A.7.3.73.2	Test Requirements	1438

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1438

A.7.3.74.1	Test Purpose and Environment	1438

A.7.3.74.2	Test Requirements	1441

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1441

A.7.3.75.1	Test Purpose and Environment	1441

A.7.3.75.2	Test Requirements	1443

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1444

A.7.3.76.1	Test Purpose and Environment	1444

A.7.3.76.2	Test Requirements	1447

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1447

A.7.3.77.1	Test Purpose and Environment	1447

A.7.3.77.2	Test Requirements	1450

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1450

A.7.3.78.1	Test Purpose and Environment	1450

A.7.3.78.2	Test Requirements	1453

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1453

A.7.3.79.1	Test Purpose and Environment	1453

A.7.3.79.2	Test Requirements	1456

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1456

A.7.3.80.1	Test Purpose and Environment	1456

A.7.3.80.2	Test Requirements	1459

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1459

A.7.3.81.1	Test Purpose and Environment	1459

A.7.3.81.2	Test Requirements	1462

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1462

A.7.3.82.1	Test Purpose and Environment	1462

A.7.3.82.2	Test Requirements	1465

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1465

A.7.3.83.1	Test Purpose and Environment	1465

A.7.3.83.2	Test Requirements	1466

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1467

A.7.3.84.1	Test Purpose and Environment	1467

A.7.3.84.2	Test Requirements	1469

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1469

A.7.3.85.1	Test Purpose and Environment	1469

A.7.3.85.2	Test Requirements	1470

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1471

A.7.3.86.1	Test Purpose and Environment	1471

A.7.3.86.2	Test Requirements	1473

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1473

A.7.3.87.1	Test Purpose and Environment	1473

A.7.3.87.2	Test Requirements	1475

A.7.4	Interruption for Dual Connectivity	1476

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1476

A.7.4.1.1	Test Purpose and Environment	1476

A.7.4.1.2	Test Requirements	1478

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1478

A.7.4.2.1	Test Purpose and Environment	1478

A.7.4.2.2	Test Requirements	1480

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1480

A.7.4.3.1	Test Purpose and Environment	1480

A.7.4.3.2	Test Requirements	1482

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1482

A.7.4.4.1	Test Purpose and Environment	1482

A.7.4.4.2	Test Requirements	1485

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1485

A.7.4.5.1	Test Purpose and Environment	1485

A.7.4.5.2	Test Requirements	1487

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1487

A.7.4.6.1	Test Purpose and Environment	1487

A.7.4.6.2	Test Requirements	1489

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1489

A.7.4.7.1	Test Purpose and Environment	1489

A.7.4.7.2	Test Requirements	1493

A.7.5	Proximity-based Services	1493

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1493

A.7.5.1.1	Test Purpose and Environment	1493

A.7.5.1.2	Test Requirements	1494

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1495

A.7.5.2.1	Test Purpose and Environment	1495

A.7.5.1.2	Test Requirements	1495

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1496

A.7.5.3.1	Test Purpose and Environment	1496

A.7.5.3.2	Test Requirements	1497

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1498

A.7.5.4.1	Test Purpose and Environment	1498

A.7.5.4.2	Test Requirements	1499

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1500

A.7.5.5.1	Test Purpose and Environment	1500

A.7.5.5.2	Test Requirements	1502

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1503

A.7.5.6.1	Test Purpose and Environment	1503

A.7.5.6.2	Test Requirements	1504

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1505

A.7.5.7.1	Test Purpose and Environment	1505

A.7.5.7.2	Test Requirements	1507

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1507

A.7.5.8.1	Test Purpose and Environment	1507

A.7.5.8.2	Test Requirements	1509

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1510

A.7.5.9.1	Test Purpose and Environment	1510

A.7.5.9.2	Test Requirements	1512

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1513

A.7.5.10.1	Test Purpose and Environment	1513

A.7.5.10.2	Test Requirements	1515

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1516

A.7.5.11.1	Test Purpose and Environment	1516

A.7.5.11.2	Test Requirements	1518

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1518

A.7.5.12.1	Test Purpose and Environment	1518

A.7.5.12.2	Test Requirements	1522

A.7.6	Interruption for carrier aggregation	1523

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1523

A.7.6.1.1	Test Purpose and Environment	1523

A.7.6.1.2	Test Requirements	1526

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1526

A.7.6.2.1	Test Purpose and Environment	1526

A.7.6.2.2	Test Requirements	1529

A.8	UE Measurements Procedures	1529

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1529

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1529

A.8.1.1.1	Test Purpose and Environment	1529

A.8.1.1.2	Test Requirements	1531

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1532

A.8.1.2.1	Test Purpose and Environment	1532

A.8.1.2.2	Test Requirements	1533

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1534

A.8.1.3.1	Test Purpose and Environment	1534

A.8.1.3.2	Test Requirements	1536

A.8.1.4	Void	1536

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1536

A.8.1.5.1	Test Purpose and Environment	1536

A.8.1.5.2	Test Requirements	1538

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1538

A.8.1.6.1	Test Purpose and Environment	1538

A.8.1.6.2	Test Requirements	1540

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1540

A.8.1.7.1	Test Purpose and Environment	1540

A.8.1.7.2	Test Requirements	1542

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1543

A.8.1.8.1	Test Purpose and Environment	1543

A.8.1.8.2	Test Requirements	1545

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1546

A.8.1.9.1	Test Purpose and Environment	1546

A.8.1.9.2	Test Requirements	1546

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1546

A.8.1.10.1	Test Purpose and Environment	1546

A.8.1.10.2	Test Requirements	1547

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1547

A.8.1.11.1	Test Purpose and Environment	1547

A.8.1.11.2	Test Requirements	1549

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1550

A.8.1.12.1	Test Purpose and Environment	1550

A.8.1.12.2	Test Requirements	1551

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1552

A.8.1.13.1	Test Purpose and Environment	1552

A.8.1.13.2	Test Requirements	1554

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1554

A.8.1.14.1	Test Purpose and Environment	1554

A.8.1.14.2	Test Requirements	1556

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1557

A.8.1.15.1	Test Purpose and Environment	1557

A.8.1.15.2	Test Requirements	1558

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1559

A.8.1.16.1	Test Purpose and Environment	1559

A.8.1.16.2	Test Requirements	1561

A.8.1.17	Void	1561

A.8.1.18	Void	1561

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1561

A.8.1.19.1	Test Purpose and Environment	1561

A.8.1.19.2	Test Requirements	1563

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1564

A.8.1.20.1	Test Purpose and Environment	1564

A.8.1.20.2	Test Requirements	1566

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1566

A.8.1.21.1	Test Purpose and Environment	1566

A.8.1.21.2	Test Requirements	1568

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1569

A.8.1.22.1	Test Purpose and Environment	1569

A.8.1.22.2	Test Requirements	1571

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1571

A.8.1.23.1	Test Purpose and Environment	1571

A.8.1.23.2	Test Requirements	1572

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1573

A.8.1.24.1	Test Purpose and Environment	1573

A.8.1.24.2	Test Requirements	1575

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1575

A.8.1.25.1	Test Purpose and Environment	1575

A.8.1.25.2	Test Requirements	1577

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1577

A.8.1.26.1	Test Purpose and Environment	1577

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1580

A.8.1.27.1	Test Purpose and Environment	1580

A.8.1.27.2	Test Requirements	1582

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1582

A.8.1.28.1	Test Purpose and Environment	1582

A.8.1.28.2	Test Requirements	1584

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1584

A.8.1.29.1	Test Purpose and Environment	1584

A.8.1.29.2	Test Requirements	1586

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1587

A.8.1.30.1	Test Purpose and Environment	1587

A.8.1.30.2	Test Requirements	1589

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1589

A.8.1.31.1	Test Purpose and Environment	1589

A.8.1.31.2	Test Requirements	1591

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1592

A.8.1.32.1	Test Purpose and Environment	1592

A.8.1.32.2	Test Requirements	1593

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1594

A.8.1.33.1	Test Purpose and Environment	1594

A.8.1.33.2	Test Requirements	1595

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1596

A.8.1.34.1	Test Purpose and Environment	1596

A.8.1.34.2	Test Requirements	1597

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1598

A.8.1.35.1	Test Purpose and Environment	1598

A.8.1.35.2	Test Requirements	1599

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1600

A.8.1.36.1	Test Purpose and Environment	1600

A.8.1.36.2	Test Requirements	1601

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1602

A.8.1.37.1	Test Purpose and Environment	1602

A.8.1.37.2	Test Requirements	1604

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1604

A.8.1.38.1	Test Purpose and Environment	1604

A.8.1.38.2	Test Requirements	1605

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1606

A.8.1.39.1	Test Purpose and Environment	1606

A.8.1.39.2	Test Requirements	1608

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1608

A.8.1.40.1	Test Purpose and Environment	1608

A.8.1.40.2	Test Requirements	1610

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1610

A.8.1.41.1	Test Purpose and Environment	1610

A.8.1.41.2	Test Requirement	1612

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1612

A.8.1.42.1	Test Purpose and Environment	1612

A.8.1.42.2	Test Requirement	1613

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1614

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1614

A.8.2.1.1	Test Purpose and Environment	1614

A.8.2.1.2	Test Requirements	1615

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1616

A.8.2.2.1	Test Purpose and Environment	1616

A.8.2.2.2	Test Requirements	1618

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1618

A.8.2.3.1	Test Purpose and Environment	1618

A.8.2.3.2	Test Requirements	1620

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1621

A.8.2.4.1	Test Purpose and Environment	1621

A.8.2.4.2	Test Requirements	1623

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1623

A.8.2.5.1	Test Purpose and Environment	1623

A.8.2.5.2	Test Requirements	1625

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1626

A.8.2.6.1	Test Purpose and Environment	1626

A.8.2.6.2	Test Requirements	1629

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1630

A.8.2.7.1	Test Purpose and Environment	1630

A.8.2.7.2	Test Requirements	1631

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1632

A.8.2.8.1	Test Purpose and Environment	1632

A.8.2.8.2	Test Requirements	1634

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1634

A.8.2.9.1	Test Purpose and Environment	1634

A.8.2.9.2	Test Requirements	1636

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1637

A.8.2.10.1	Test Purpose and Environment	1637

A.8.2.10.2	Test Requirements	1639

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1639

A.8.2.11.1	Test Purpose and Environment	1639

A.8.2.11.2	Test Requirements	1641

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1641

A.8.2.12.1	Test Purpose and Environment	1641

A.8.2.12.2	Test Requirements	1643

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1644

A.8.2.13.1	Test Purpose and Environment	1644

A.8.2.13.2	Test Requirements	1646

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1646

A.8.2.14.1	Test Purpose and Environment	1646

A.8.2.14.2	Test Requirement	1648

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1649

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1649

A.8.3.1.1	Test Purpose and Environment	1649

A.8.3.1.2	Test Requirements	1650

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1651

A.8.3.2.1	Test Purpose and Environment	1651

A.8.3.2.2	Test Requirements	1653

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1653

A.8.3.3.1	Test Purpose and Environment	1653

A.8.3.3.2	Test Requirements	1656

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1656

A.8.3.4.1	Test Purpose and Environment	1656

A.8.3.4.2	Test Requirements	1658

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1658

A.8.3.5.2	Test Requirements	1660

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1660

A.8.3.6.1	Test Purpose and Environment	1660

A.8.3.6.2	Test Requirements	1662

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1663

A.8.3.7.1	Test Purpose and Environment	1663

A.8.3.7.2	Test Requirements	1665

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1666

A.8.3.8.1	Test Purpose and Environment	1666

A.8.3.8.2	Test Requirements	1669

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1669

A.8.3.9.1	Test Purpose and Environment	1669

A.8.3.9.2	Test Requirements	1673

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1674

A.8.3.10.1	Test Purpose and Environment	1674

A.8.3.10.2	Test Requirements	1675

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1676

A.8.3.11.1	Test Purpose and Environment	1676

A.8.3.11.2	Test Requirement	1677

A.8.3.12	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1678

A.8.3.12.1	Test Purpose and Environment	1678

A.8.3.12.2	Test Requirement	1679

A.8.3.13	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1680

A.8.3.13.1	Test Purpose and Environment	1680

A.8.3.13.2	Test Requirement	1681

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1682

A.8.3.14.1	Test Purpose and Environment	1682

A.8.3.14.2	Test Requirement	1683

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1684

A.8.3.15.1	Test Purpose and Environment	1684

A.8.3.15.2	Test Requirement	1685

A.8.3.16	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1686

A.8.3.16.1	Test Purpose and Environment	1686

A.8.3.16.2	Test Requirement	1688

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1688

A.8.3.17.1	Test Purpose and Environment	1688

A.8.3.17.2	Test Requirement	1691

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1691

A.8.3.18.1	Test Purpose and Environment	1691

A.8.3.18.2	Test Requirement	1694

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1694

A.8.3.19.1	Test Purpose and Environment	1694

A.8.3.19.2	Test Requirement	1697

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1697

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1697

A.8.4.1.1	Test Purpose and Environment	1697

A.8.4.1.2	Test Requirements	1699

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1700

A.8.4.2.1	Test Purpose and Environment	1700

A.8.4.2.2	Test Requirements	1702

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1702

A.8.4.3.1	Test Purpose and Environment	1702

A.8.4.3.2	Test Requirements	1704

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1705

A.8.4.4.1	Test Purpose and Environment	1705

A.8.4.4.2	Test Requirements	1707

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1708

A.8.4.5.1	Test Purpose and Environment	1708

A.8.4.5.2	Test Requirements	1710

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1710

A.8.4.6.1	Test Purpose and Environment	1710

A.8.4.6.2	Test Requirements	1711

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1711

A.8.4.7.1	Test Purpose and Environment	1711

A.8.4.7.2	Test Requirements	1715

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1715

A.8.4.8.1	Test Purpose and Environment	1715

A.8.4.8.2	Test Requirements	1720

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1720

A.8.4.9.1	Test Purpose and Environment	1720

A.8.4.9.2	Test Requirements	1725

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1725

A.8.4.10.1	Test Purpose and Environment	1725

A.8.4.10.2	Test Requirements	1727

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1727

A.8.4.11.1	Test Purpose and Environment	1727

A.8.4.11.2	Test Requirement	1729

A.8.4.12	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1730

A.8.4.12.1	Test Purpose and Environment	1730

A.8.4.12.2	Test Requirement	1731

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1732

A.8.4.13.1	Test Purpose and Environment	1732

A.8.4.13.2	Test Requirement	1733

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1734

A.8.4.14.1	Test Purpose and Environment	1734

A.8.4.14.2	Test Requirement	1736

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1736

A.8.4.15.1	Test Purpose and Environment	1736

A.8.4.15.2	Test Requirement	1739

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1739

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1739

A.8.5.1.1	Test Purpose and Environment	1739

A.8.5.1.2	Test Requirements	1741

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1741

A.8.5.2.1	Test Purpose and Environment	1741

A.8.5.2.2	Test Requirements	1743

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1743

A.8.5.3.1	Test Purpose and Environment	1743

A.8.5.3.2	Test Requirements	1746

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1746

A.8.5.4.1	Test Purpose and Environment	1746

A.8.5.4.2	Test Requirements	1749

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1749

A.8.5.5.1	Test Purpose and Environment	1749

A.8.5.5.2	Test Requirements	1751

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1751

A.8.5.6.1	Test Purpose and Environment	1751

A.8.5.6.2	Test Requirements	1752

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1753

A.8.5.7.1	Test Purpose and Environment	1753

A.8.5.7.2	Test Requirements	1753

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1753

A.8.5.8.1	Test Purpose and Environment	1753

A.8.5.8.2	Test Requirements	1756

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1756

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1756

A.8.6.1.1	Test Purpose and Environment	1756

A.8.6.1.2	Test Requirements	1758

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1759

A.8.6.2.1	Test Purpose and Environment	1759

A.8.6.2.2	Test Requirements	1761

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1761

A.8.6.3.1	Test Purpose and Environment	1761

A.8.6.3.2	Test Requirements	1764

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1764

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1764

A.8.7.1.1	Test Purpose and Environment	1764

A.8.7.1.1.1	Void	1764

A.8.7.1.1.2	1.28 Mcps TDD option	1764

A.8.7.1.1.3	Void	1766

A.8.7.1.2	Test Requirements	1766

A.8.7.1.2.1	Void	1766

A.8.7.1.2.2	1.28 Mcps TDD option	1766

A.8.7.1.2.3	Void	1766

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1766

A.8.7.2.1	Test Purpose and Environment	1766

A.8.7.2.2	Test Requirements	1769

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1770

A.8.7.3.1	Test Purpose and Environment	1770

A.8.7.3.2	Test Parameters	1770

A.8.7.3.3	Test Requirements	1771

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1772

A.8.7.4.1	Test Purpose and Environment	1772

A.8.7.4.2	Test Requirements	1774

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1774

A.8.7.5.1	Test Purpose and Environment	1774

A.8.7.5.2	Test Requirements	1776

A.8.7A	TBD	1776

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1776

A.8.7A.1.1	Test Purpose and Environment	1776

A.8.7A.1.2	Test Requirements	1779

A.8.8	E-UTRAN FDD – GSM Measurements	1779

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1779

A.8.8.1.1	Test Purpose and Environment	1779

A.8.8.1.2	Test Requirements	1781

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1781

A.8.8.2.1	Test Purpose and Environment	1781

A.8.8.2.2	Test Requirements	1783

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1784

A.8.8.3.1	Test Purpose and Environment	1784

A.8.8.3.2	Test Requirements	1785

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1786

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1786

A.8.9.1.1	Test Purpose and Environment	1786

A.8.9.1.2	Test Requirements	1787

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1788

A.8.9.2.1	Test Purpose and Environment	1788

A.8.9.2.2	Test Requirements	1790

A.8.10	E-UTRAN TDD – GSM Measurements	1790

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1790

A.8.10.1.1	Test Purpose and Environment	1790

A.8.10.1.2	Test Requirements	1791

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1792

A.8.10.2.1	Test Purpose and Environment	1792

A.8.10.2.2	Test Requirements	1794

A.8.11	Monitoring of Multiple Layers	1794

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1794

A.8.11.1.1	Test Purpose and Environment	1794

A.8.11.1.2	Test Requirements	1796

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	1797

A.8.11.2.1	Test Purpose and Environment	1797

A.8.11.2.2	Test Requirements	1798

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	1799

A.8.11.3.1	Test Purpose and Environment	1799

A.8.11.3.2	Test Requirements	1801

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	1801

A.8.11.4.1	Test Purpose and Environment	1801

A.8.11.4.2	Test Requirements	1804

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1804

A.8.11.5.1	Test Purpose and Environment	1804

A.8.11.5.2	Test Requirements	1806

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	1807

A.8.11.6.1	Test Purpose and Environment	1807

A.8.11.6.2	Test Requirements	1809

A.8.12	RSTD Intra-frequency Measurements	1810

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	1810

A.8.12.1.1	Test Purpose and Environment	1810

A.8.12.1.2	Test Requirements	1814

A.8.12.1.2A	Test Requirements for UE Category 1bis	1814

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	1814

A.8.12.2.1	Test Purpose and Environment	1814

A.8.12.2.2	Test Requirements	1819

A.8.12.2.2A	Test Requirements for UE Category 1bis	1819

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	1819

A.8.12.3.1	Test Purpose and Environment	1819

A.8.12.3.2	Test Requirements	1825

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	1825

A.8.12.4.1	Test Purpose and Environment	1825

A.8.12.4.2	Test Requirements	1830

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	1830

A.8.12.5.1	Test Purpose and Environment	1830

A.8.12.5.2	Test Requirements	1835

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	1835

A.8.12.6.1	Test Purpose and Environment	1835

A.8.12.6.2	Test Requirements	1840

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	1840

A.8.12.7.1	Test Purpose and Environment	1840

A.8.12.7.2	Test Requirements	1845

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	1845

A.8.12.8.1	Test Purpose and Environment	1845

A.8.12.8.2	Test Requirements	1850

A.8.13	RSTD Inter-frequency Measurements	1850

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1850

A.8.13.1.1	Test Purpose and Environment	1850

A.8.13.1.2	Test Requirements	1855

A.8.13.1.2A	Test Requirements for UE Category 1bis	1855

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	1855

A.8.13.2.1	Test Purpose and Environment	1855

A.8.13.2.2	Test Requirements	1861

A.8.13.2.2A	Test Requirements for UE Category 1bis	1861

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	1861

A.8.13.3.1	Test Purpose and Environment	1861

A.8.13.3.2	Test Requirements	1868

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	1868

A.8.13.4.1	Test Purpose and Environment	1868

A.8.13.4.2	Test Requirements	1874

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	1874

A.8.13.5.1	Test Purpose and Environment	1874

A.8.13.5.2	Test Requirements	1880

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	1880

A.8.13.6.1	Test Purpose and Environment	1880

A.8.13.6.2	Test Requirements	1886

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	1886

A.8.13.7.1	Test Purpose and Environment	1886

A.8.13.7.2	Test Requirements	1892

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	1892

A.8.13.8.1	Test Purpose and Environment	1892

A.8.13.8.2	Test Requirements	1898

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	1898

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1898

A.8.14.1.1	Test Purpose and Environment	1898

A.8.14.1.2	Test Requirements	1900

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1900

A.8.14.2.1	Test Purpose and Environment	1900

A.8.14.2.2	Test Requirements	1903

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1903

A.8.14.3.1	Test Purpose and Environment	1903

A.8.14.3.2	Test Requirements	1905

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	1906

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1906

A.8.15.1.1	Test Purpose and Environment	1906

A.8.15.1.2	Test Requirements	1907

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1907

A.8.15.2.1	Test Purpose and Environment	1907

A.8.15.2.2	Test Requirements	1910

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1910

A.8.15.3.1	Test Purpose and Environment	1910

A.8.15.3.2	Test Requirements	1912

A.8.16	E-UTRAN Carrier Aggregation Measurements	1913

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	1913

A.8.16.1.1	Test Purpose and Environment	1913

A.8.16.1.2	Test Requirements	1915

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	1915

A.8.16.2.1	Test Purpose and Environment	1915

A.8.16.2.2	Test Requirements	1917

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1917

A.8.16.3.1	Test Purpose and Environment	1917

A.8.16.3.2	Test Requirements	1919

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	1920

A.8.16.3A.1	Test Purpose and Environment	1920

A.8.16.3A.2	Test Requirements	1921

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	1922

A.8.16.4.1	Test Purpose and Environment	1922

A.8.16.4.2	Test Requirements	1923

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	1924

A.8.16.4A.1	Test Purpose and Environment	1924

A.8.16.4A.2	Test Requirements	1926

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1927

A.8.16.5.1	Test Purpose and Environment	1927

A.8.16.5.2	Test Requirements	1927

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	1927

A.8.16.6.1	Test Purpose and Environment	1927

A.8.16.6.2	Test Requirements	1928

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1928

A.8.16.7.1	 Test Purpose and Environment	1928

A.8.16.7.2	 Test Requirements	1929

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	1929

A.8.16.8.1	 Test Purpose and Environment	1929

A.8.16.8.2	 Test Requirements	1930

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1930

A.8.16.9.1	Test Purpose and Environment	1930

A.8.16.9.2	Test Requirements	1931

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	1931

A.8.16.10.1	Test Purpose and Environment	1931

A.8.16.10.2	Test Requirements	1932

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1932

A.8.16.11.1	 Test Purpose and Environment	1932

A.8.16.11.2	 Test Requirements	1933

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	1933

A.8.16.12.1	 Test Purpose and Environment	1933

A.8.16.12.2	 Test Requirements	1933

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	1933

A.8.16.13.1	Test Purpose and Environment	1933

A.8.16.13.2	Test Requirements	1934

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	1934

A.8.16.14.1	Test Purpose and Environment	1934

A.8.16.14.2	Test Requirements	1935

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	1935

A.8.16.15.1	 Test Purpose and Environment	1935

A.8.16.7.2	 Test Requirements	1935

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	1935

A.8.16.16.1	 Test Purpose and Environment	1935

A.8.16.16.2	 Test Requirements	1936

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	1936

A.8.16.17.1	Test Purpose and Environment	1936

A.8.16.17.2	Test Requirements	1938

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	1939

A.8.16.17A.1	Test Purpose and Environment	1939

A.8.16.17A.2	Test Requirements	1939

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1939

A.8.16.17B.1	Test Purpose and Environment	1939

A.8.16.17B.2	Test Requirements	1940

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1940

A.8.16.17C.1	Test Purpose and Environment	1940

A.8.16.17C.2	Test Requirements	1940

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	1940

A.8.16.18.1	Test Purpose and Environment	1940

A.8.16.18.2	Test Requirements	1942

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	1943

A.8.16.18A.1	Test Purpose and Environment	1943

A.8.16.18A.2	Test Requirements	1943

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	1943

A.8.16.18B.1	Test Purpose and Environment	1943

A.8.16.18B.2	Test Requirements	1944

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	1944

A.8.16.18C.1	Test Purpose and Environment	1944

A.8.16.18C.2	Test Requirements	1944

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	1944

A.8.16.18D.1	Test Purpose and Environment	1944

A.8.16.18D.2	Test Requirements	1945

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	1945

A.8.16.19.1	Test Purpose and Environment	1945

A.8.16.19.2	Test Requirements	1947

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1947

A.8.16.19A.1	Test Purpose and Environment	1947

A.8.16.19A.2	Test Requirements	1948

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1948

A.8.16.19B.1	Test Purpose and Environment	1948

A.8.16.19B.2	Test Requirements	1948

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1948

A.8.16.19C.1	Test Purpose and Environment	1948

A.8.16.19C.2	Test Requirements	1949

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	1949

A.8.16.20.1	Test Purpose and Environment	1949

A.8.16.20.2	Test Requirements	1951

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	1952

A.8.16.20A.1	Test Purpose and Environment	1952

A.8.16.20A.2	Test Requirements	1952

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	1952

A.8.16.20B.1	Test Purpose and Environment	1952

A.8.16.20B.2	Test Requirements	1953

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	1953

A.8.16.20C.1	Test Purpose and Environment	1953

A.8.16.20C.2	Test Requirements	1953

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	1953

A.8.16.20D.1	Test Purpose and Environment	1953

A.8.16.20D.2	Test Requirements	1954

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	1954

A.8.16.21.1	Test Purpose and Environment	1954

A.8.16.21.2	Test Requirements	1956

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	1956

A.8.16.22.1	Test Purpose and Environment	1956

A.8.16.22.2	Test Requirements	1958

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	1958

A.8.16.23.1	Test Purpose and Environment	1958

A.8.16.23.2	Test Requirements	1961

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	1961

A.8.16.24.1	Test Purpose and Environment	1961

A.8.16.24.2	Test Requirements	1964

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	1964

A.8.16.25.1	Test Purpose and Environment	1964

A.8.16.25.2	Test Requirements	1967

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	1967

A.8.16.26.1	Test Purpose and Environment	1967

A.8.16.26.2	Test Requirements	1970

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1970

A.8.16.27.1	Test Purpose and Environment	1970

A.8.16.27.2	Test Requirements	1975

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	1975

A.8.16.28.1	Test Purpose and Environment	1975

A.8.16.28.2	Test Requirements	1980

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1980

A.8.16.29.1	Test Purpose and Environment	1980

A.8.16.29.2	Test Requirements	1985

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	1985

A.8.16.30.1	Test Purpose and Environment	1985

A.8.16.30.2	Test Requirements	1990

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	1990

A.8.16.31.1	Test Purpose and Environment	1990

A.8.16.31.2	Test Requirements	1995

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	1995

A.8.16.32.1	Test Purpose and Environment	1995

A.8.16.32.2	Test Requirements	2000

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2000

A.8.16.33.1	Test Purpose and Environment	2000

A.8.16.33.2	Test Requirements	2005

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2005

A.8.16.34.1	Test Purpose and Environment	2005

A.8.16.34.2	Test Requirements	2010

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2010

A.8.16.35.1	Test Purpose and Environment	2010

A.8.16.35.2	Test Requirements	2012

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2013

A.8.16.36.1	Test Purpose and Environment	2013

A.8.16.36.2	Test Requirements	2015

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2016

A.8.16.37.1	Test Purpose and Environment	2016

A.8.16.37.2	Test Requirements	2018

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2019

A.8.16.38.1	Test Purpose and Environment	2019

A.8.16.38.2	Test Requirements	2021

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2022

A.8.16.39.1	Test Purpose and Environment	2022

A.8.16.39.2	Test Requirements	2025

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2025

A.8.16.40.1	Test Purpose and Environment	2025

A.8.16.40.2	Test Requirements	2028

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2028

A.8.16.41.1	Test Purpose and Environment	2028

A.8.16.41.2	Test Requirements	2030

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2031

A.8.16.42.1	Test Purpose and Environment	2031

A.8.16.42.2	Test Requirements	2033

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2034

A.8.16.43.1	Test Purpose and Environment	2034

A.8.16.43.2	Test Requirements	2037

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2037

A.8.16.44.1	Test Purpose and Environment	2037

A.8.16.44.2	Test Requirements	2040

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2040

A.8.16.45.1	Test Purpose and Environment	2040

A.8.16.45.2	Test Requirements	2043

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2043

A.8.16.46.1	Test Purpose and Environment	2043

A.8.16.46.2	Test Requirements	2045

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2046

A.8.16.47.1	Test Purpose and Environment	2046

A.8.16.47.2	Test Requirements	2048

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2049

A.8.16.48.1	Test Purpose and Environment	2049

A.8.16.48.2	Test Requirements	2051

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2052

A.8.16.49.1	Test Purpose and Environment	2052

A.8.16.49.2	Test Requirements	2055

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2055

A.8.16.50.1	Test Purpose and Environment	2055

A.8.16.50.2	Test Requirements	2057

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2058

A.8.16.51.1	Test Purpose and Environment	2058

A.8.16.51.2	Test Requirements	2063

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2063

A.8.16.52.1	Test Purpose and Environment	2063

A.8.16.52.2	Test Requirements	2068

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2068

A.8.16.53.1	Test Purpose and Environment	2068

A.8.16.53.2	Test Requirements	2072

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2072

A.8.16.54.1	Test Purpose and Environment	2072

A.8.16.54.2	Test Requirements	2076

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2076

A.8.16.55.1	Test Purpose and Environment	2076

A.8.16.55.2	Test Requirements	2083

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2083

A.8.16.56.1	Test Purpose and Environment	2083

A.8.16.56.2	Test Requirements	2089

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2089

A.8.16.57.1	Test Purpose and Environment	2089

A.8.16.57.2	Test Requirements	2091

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2092

A.8.16.58.1	Test Purpose and Environment	2092

A.8.16.58.2	Test Requirements	2094

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2095

A.8.16.59.1	Test Purpose and Environment	2095

A.8.16.59.2	Test Requirements	2099

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2100

A.8.16.60.1	Test Purpose and Environment	2100

A.8.16.60.2	Test Requirements	2103

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2104

A.8.16.61.1	Test Purpose and Environment	2104

A.8.16.61.2	Test Requirements	2107

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2107

A.8.16.62.1	Test Purpose and Environment	2107

A.8.16.62.2	Test Requirements	2109

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2110

A.8.16.63.1	Test Purpose and Environment	2110

A.8.16.63.2	Test Requirements	2113

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2114

A.8.16.64.1	Test Purpose and Environment	2114

A.8.16.64.2	Test Requirements	2118

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2118

A.8.16.65.1	Test Purpose and Environment	2118

A.8.16.65.2	Test Requirements	2123

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2123

A.8.16.66.1	Test Purpose and Environment	2123

A.8.16.66.2	Test Requirements	2128

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2128

A.8.16.67.1	Test Purpose and Environment	2128

A.8.16.67.2	Test Requirements	2133

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2133

A.8.16.68.1	Test Purpose and Environment	2133

A.8.16.68.2	Test Requirements	2138

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2138

A.8.16.69.1	Test Purpose and Environment	2138

A.8.16.69.2	Test Requirements	2142

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2142

A.8.16.70.1	Test Purpose and Environment	2142

A.8.16.70.2	Test Requirements	2146

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2146

A.8.16.71.1	Test Purpose and Environment	2146

A.8.16.71.2	Test Requirements	2151

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2151

A.8.16.72.1	Test Purpose and Environment	2151

A.8.16.72.2	Test Requirements	2156

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2156

A.8.16.73.1	Test Purpose and Environment	2156

A.8.16.73.2	Test Requirements	2163

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2163

A.8.16.74.1	Test Purpose and Environment	2163

A.8.16.74.2	Test Requirements	2170

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2170

A.8.16.75.1	Test Purpose and Environment	2170

A.8.16.75.2	Test Requirements	2176

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2176

A.8.16.76.1	Test Purpose and Environment	2176

A.8.16.76.2	Test Requirements	2182

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2182

A.8.16.77.1	Test Purpose and Environment	2182

A.8.16.77.2	Test Requirements	2186

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2186

A.8.16.78.1	Test Purpose and Environment	2186

A.8.16.78.2	Test Requirements	2188

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2189

A.8.16.79.1	Test Purpose and Environment	2189

A.8.16.79.2	Test Requirements	2196

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2196

A.8.16.80.1	Test Purpose and Environment	2196

A.8.16.80.2	Test Requirements	2203

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2203

A.8.16.81.1	Test Purpose and Environment	2203

A.8.16.81.2	Test Requirements	2209

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2209

A.8.16.82.1	Test Purpose and Environment	2209

A.8.16.82.2	Test Requirements	2215

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2215

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2215

A.8.17.1.1	Test Purpose and Environment	2215

A.8.17.1.2	Test Requirements	2221

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2221

A.8.17.2.1	Test Purpose and Environment	2221

A.8.17.2.2	Test Requirements	2228

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2228

A.8.17.3.1	Test Purpose and Environment	2228

A.8.17.3.2	Test Requirements	2229

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2229

A.8.17.4.1	Test Purpose and Environment	2229

A.8.17.4.2	Test Requirements	2230

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2230

A.8.17.5.1	Test Purpose and Environment	2230

A.8.17.5.2	Test Requirements	2231

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2231

A.8.17.6.1	Test Purpose and Environment	2231

A.8.17.6.2	Test Requirements	2232

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2233

A.8.17.7.1	Test Purpose and Environment	2233

A.8.17.7.2	Test Requirements	2233

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2234

A.8.17.8.1	Test Purpose and Environment	2234

A.8.17.8.2	Test Requirements	2234

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2235

A.8.17.9.1	Test Purpose and Environment	2235

A.8.17.9.2	Test Requirements	2236

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2236

A.8.17.10.1	Test Purpose and Environment	2236

A.8.17.10.2	Test Requirements	2243

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2244

A.8.17.11.1	Test Purpose and Environment	2244

A.8.17.11.2	Test Requirements	2250

A.8.18	E-UTRAN TDD – HRPD Measurements	2251

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2251

A.8.18.1.1	Test Purpose and Environment	2251

A.8.18.1.2	Test Requirements	2253

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2253

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2253

A.8.19.1.1	Test Purpose and Environment	2253

A.8.19.1.2	Test Requirements	2254

A.8.20	Inter-frequency/RAT Measurements in CA mode	2255

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2255

A.8.20.1.1	Test Purpose and Environment	2255

A.8.20.1.2	Test Requirements	2256

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2257

A.8.20.2.1	Test Purpose and Environment	2257

A.8.20.2.2	Test Requirements	2258

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2259

A.8.20.2A.1	Test Purpose and Environment	2259

A.8.20.2A.2	Test Requirements	2259

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2259

A.8.20.2B.1	Test Purpose and Environment	2259

A.8.20.2B.2	Test Requirements	2262

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2262

A.8.20.3.1	Test Purpose and Environment	2262

A.8.20.3.2	Test Requirements	2264

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2264

A.8.20.4.1	Test Purpose and Environment	2264

A.8.20.4.1.1	1.28 Mcps TDD option	2264

A.8.20.4.2	Test Requirements	2266

A.8.20.4.2.1	1.28 Mcps TDD option	2266

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2266

A.8.20.4A.1	Test Purpose and Environment	2266

A.8.20.4A.1.1	1.28 Mcps TDD option	2266

A.8.20.4A.2	Test Requirements	2266

A.8.20.4A.2.1	1.28 Mcps TDD option	2266

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2267

A.8.20.4B.1	Test Purpose and Environment	2267

A.8.20.4B.1.1	1.28 Mcps TDD option	2267

A.8.20.4B.2	Test Requirements	2269

A.8.20.4B.2.1	1.28 Mcps TDD option	2269

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2269

A.8.21.1	Test Purpose and Environment	2269

A.8.21.2 Test Requirements	2273

A.8.22	E-UTRAN Discovery Signal Measurements	2273

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2273

A.8.22.1.1	Test Purpose and Environment	2273

A.8.22.1.2	Test Requirements	2276

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2276

A.8.22.2.1	Test Purpose and Environment	2276

A.8.22.2.2	Test Requirements	2279

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2279

A.8.22.3.1	Test Purpose and Environment	2279

A.8.22.3.2	Test Requirements	2282

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2282

A.8.22.4.1	Test Purpose and Environment	2282

A.8.22.4.2	Test Requirements	2285

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2285

A.8.22.5.1	Test Purpose and Environment	2285

A.8.22.5.2	Test Requirements	2289

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2289

A.8.22.6.1	Test Purpose and Environment	2289

A.8.22.6.2	Test Requirements	2293

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2293

A.8.22.7.1	Test Purpose and Environment	2293

A.8.22.7.2	Test Requirements	2297

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2297

A.8.22.8.1	Test Purpose and Environment	2297

A.8.22.8.2	Test Requirements	2301

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2301

A.8.22.9.1	Test Purpose and Environment	2301

A.8.22.9.2	Test Requirements	2303

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2304

A.8.22.10.1	Test Purpose and Environment	2304

A.8.22.10.2	Test Requirements	2306

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2307

A.8.22.11.1	Test Purpose and Environment	2307

A.8.22.11.2	Test Requirements	2310

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2310

A.8.22.12.1	Test Purpose and Environment	2310

A.8.22.12.2	Test Requirements	2314

A.8.23	E-UTRAN Dual Connectivity Measurements	2314

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2314

A.8.23.1.1	Test Purpose and Environment	2314

A.8.23.1.2	Test Requirements	2317

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2317

A.8.23.2.1	Test Purpose and Environment	2317

A.8.23.2.2	Test Requirements	2320

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2320

A.8.23.3.1	Test Purpose and Environment	2320

A.8.23.3.2	Test Requirements	2323

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2323

A.8.23.4.1	Test Purpose and Environment	2323

A.8.23.4.2	Test Requirements	2326

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2326

A.8.23.5.1	Test Purpose and Environment	2326

A.8.23.5.2	Test Requirements	2329

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2329

A.8.23.6.1	Test Purpose and Environment	2329

A.8.23.6.2	Test Requirements	2332

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2332

A.8.23.7.1	Test Purpose and Environment	2332

A.8.23.7.2	Test Requirements	2334

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2335

A.8.23.8.1	Test Purpose and Environment	2335

A.8.23.8.2	Test Requirements	2337

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2338

A.8.23.9.1	Test Purpose and Environment	2338

A.8.23.9.2	Test Requirements	2341

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2341

A.8.23.10.1	Test Purpose and Environment	2341

A.8.23.10.2	Test Requirements	2344

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2344

A.8.23.11.1	Test Purpose and Environment	2344

A.8.23.11.2	Test Requirements	2347

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2347

A.8.23.12.1	Test Purpose and Environment	2347

A.8.23.12.2	Test Requirements	2350

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2350

A.8.23.13.1	Test Purpose and Environment	2350

A.8.23.13.2	Test Requirements	2353

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2353

A.8.23.14.1	Test Purpose and Environment	2353

A.8.23.14.2	Test Requirements	2356

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2356

A.8.23.15.1	Test Purpose and Environment	2356

A.8.23.15.2	Test Requirements	2359

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2359

A.8.23.16.1	Test Purpose and Environment	2359

A.8.23.16.2	Test Requirements	2360

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2361

A.8.23.17.1	Test Purpose and Environment	2361

A.8.23.17.2	Test Requirements	2363

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2363

A.8.23.18.1	Test Purpose and Environment	2363

A.8.23.18.2	Test Requirements	2364

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2365

A.8.23.19.1	Test Purpose and Environment	2365

A.8.23.19.2	Test Requirements	2366

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2367

A.8.23.20.1	Test Purpose and Environment	2367

A.8.23.20.2	Test Requirements	2368

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2369

A.8.23.21.1	Test Purpose and Environment	2369

A.8.23.21.2	Test Requirements	2370

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2371

A.8.23.22.1	Test Purpose and Environment	2371

A.8.23.22.2	Test Requirements	2373

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2373

A.8.23.23.1	Test Purpose and Environment	2373

A.8.23.23.2	Test Requirements	2375

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2376

A.8.23.24.1	Test Purpose and Environment	2376

A.8.23.24.2	Test Requirements	2378

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2379

A.8.23.25.1	Test Purpose and Environment	2379

A.8.23.25.2	Test Requirements	2381

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2382

A.8.23.26.1	Test Purpose and Environment	2382

A.8.23.26.2	Test Requirements	2384

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2385

A.8.23.27.1	Test Purpose and Environment	2385

A.8.23.27.2	Test Requirements	2388

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2388

A.8.23.28.1	Test Purpose and Environment	2388

A.8.23.28.2	Test Requirements	2392

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2392

A.8.23.29.1	Test Purpose and Environment	2392

A.8.23.29.2	Test Requirements	2396

A.8.24	Proximity-based Services	2396

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2396

A.8.24.1.1	Test Purpose and Environment	2396

A.8.24.1.2	Test Requirements	2397

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2398

A.8.24.2.1	Test Purpose and Environment	2398

A.8.24.2.2	Test Requirements	2399

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2399

A.8.24.3.1	Test Purpose and Environment	2400

A.8.24.3.2	Test Requirements	2401

A.8.25	E-UTRAN-WLAN Measurements	2402

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2402

A.8.25.1.1	Test Purpose and Environment	2402

A.8.25.1.2	Test Requirements	2404

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2404

A.8.25.2.1	Test Purpose and Environment	2404

A.8.25.2.2	Test Requirements	2406

A.8.26 Frame Structure 3 (FS3)	2406

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2406

A.8.26.1.1	Test Purpose and Environment	2406

A.8.26.1.2	Test Requirements	2408

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2409

A.8.26.2.1	Test Purpose and Environment	2409

A.8.26.2.2	Test Requirements	2411

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2412

A.8.26.3.1	Test Purpose and Environment	2412

A.8.26.3.2	Test Requirements	2415

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2415

A.8.26.3A.1	Test Purpose and Environment	2415

A.8.26.3A.2	Test Requirements	2419

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2419

A.8.26.4.1	Test Purpose and Environment	2419

A.8.26.4.2	Test Requirements	2423

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2423

A.8.26.4A.1	Test Purpose and Environment	2423

A.8.26.4A.2	Test Requirements	2427

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2427

A.8.26.5.1	Test Purpose and Environment	2427

A.8.26.5.2	Test Requirements	2431

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2431

A.8.26.5A.1	Test Purpose and Environment	2431

A.8.26.5A.2	Test Requirements	2435

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2435

A.8.26.6.1	Test Purpose and Environment	2435

A.8.26.6.2	Test Requirements	2439

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2439

A.8.26.6A.1	Test Purpose and Environment	2439

A.8.26.6A.2	Test Requirements	2443

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2443

A.8.26.7.1	Test Purpose and Environment	2443

A.8.26.7.2	Test Requirements	2446

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2446

A.8.26.8.1	Test Purpose and Environment	2446

A.8.26.8.2	Test Requirements	2450

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2450

A.8.26.9.1	Test Purpose and Environment	2450

A.8.26.9.2	Test Requirements	2453

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2453

A.8.26.10.1	Test Purpose and Environment	2453

A.8.26.10.2	Test Requirements	2456

A.9	Measurement Performance Requirements	2457

A.9.1	RSRP	2457

A.9.1.1	FDD Intra frequency case	2457

A.9.1.1.1	Test Purpose and Environment	2457

A.9.1.1.2	Test parameters	2457

A.9.1.1.3	Test Requirements	2460

A.9.1.2	TDD Intra frequency case	2460

A.9.1.2.1	Test Purpose and Environment	2460

A.9.1.2.2	Test parameters	2460

A.9.1.2.3	Test Requirements	2461

A.9.1.3	FDD—FDD Inter frequency case	2462

A.9.1.3.1	Test Purpose and Environment	2462

A.9.1.3.2	Test parameters	2462

A.9.1.3.3	Test Requirements	2465

A.9.1.4	TDD—TDD Inter frequency case	2465

A.9.1.4.1	Test Purpose and Environment	2465

A.9.1.4.2	Test parameters	2465

A.9.1.4.3	Test Requirements	2468

A.9.1.5	FDD—TDD Inter frequency case	2468

A.9.1.5.1	Test Purpose and Environment	2468

A.9.1.5.2	Test parameters	2468

A.9.1.5.3	Test Requirements	2470

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2471

A.9.1.6.1	Test Purpose and Environment	2471

A.9.1.6.2	Test parameters	2471

A.9.1.6.3	Test Requirements	2474

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2474

A.9.1.7.1	Test Purpose and Environment	2474

A.9.1.7.2	Test parameters	2474

A.9.1.7.3	Test Requirements	2477

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2477

A.9.1.8.1	Test Purpose and Environment	2477

A.9.1.8.2	Test parameters	2477

A.9.1.8.3	Test Requirements	2481

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2481

A.9.1.9.1	Test Purpose and Environment	2481

A.9.1.9.2	Test parameters	2481

A.9.1.9.3	Test Requirements	2484

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2484

A.9.1.10.1	Test Purpose and Environment	2484

A.9.1.10.2	Test parameters	2484

A.9.1.10.3	Test Requirements	2487

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2487

A.9.1.11.1	Test Purpose and Environment	2487

A.9.1.11.2	Test parameters	2487

A.9.1.11.3	Test Requirements	2491

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2491

A.9.1.12.1	Test Purpose and Environment	2491

A.9.1.12.2	Test parameters	2491

A.9.1.12.3	Test Requirements	2492

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2492

A.9.1.13.1	Test Purpose and Environment	2492

A.9.1.13.2	Test parameters	2492

A.9.1.13.3	Test Requirements	2493

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2493

A.9.1.14.1	Test Purpose and Environment	2493

A.9.1.14.2	Test parameters	2493

A.9.1.14.3	Test Requirements	2497

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2497

A.9.1.15.1	Test Purpose and Environment	2497

A.9.1.15.2	Test parameters	2497

A.9.1.15.3	Test Requirements	2499

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2500

A.9.1.16.1	Test Purpose and Environment	2500

A.9.1.16.2	Test parameters	2500

A.9.1.16.3	Test Requirements	2501

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2502

A.9.1.17.1	Test Purpose and Environment	2502

A.9.1.17.2	Test parameters	2502

A.9.1.17.3	Test Requirements	2503

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2504

A.9.1.18.1	Test Purpose and Environment	2504

A.9.1.18.2	Test parameters	2504

A.9.1.18.3	Test Requirements	2505

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2505

A.9.1.19.1	Test Purpose and Environment	2506

A.9.1.19.2	Test parameters	2506

A.9.1.19.3	Test Requirements	2506

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2506

A.9.1.20.1	Test Purpose and Environment	2506

A.9.1.20.2	Test parameters	2506

A.9.1.20.3	Test Requirements	2507

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2508

A.9.1.21.1	Test Purpose and Environment	2508

A.9.1.21.2	Test parameters	2508

A.9.1.21.3	Test Requirements	2508

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2508

A.9.1.22.1	Test Purpose and Environment	2508

A.9.1.22.2	Test parameters	2509

A.9.1.22.3	Test Requirements	2513

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2513

A.9.1.23.1	Test Purpose and Environment	2513

A.9.1.23.2	Test parameters	2513

A.9.1.23.3	Test Requirements	2516

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2517

A.9.1.24.1	Test Purpose and Environment	2517

A.9.1.24.2	Test parameters	2517

A.9.1.24.3	Test Requirements	2517

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2518

A.9.1.25.1	Test Purpose and Environment	2518

A.9.1.25.2	Test parameters	2518

A.9.1.25.3	Test Requirements	2521

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2521

A.9.1.26.1	Test Purpose and Environment	2521

A.9.1.26.2	Test parameters	2521

A.9.1.26.3	Test Requirements	2524

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2524

A.9.1.27.1	Test Purpose and Environment	2524

A.9.1.27.2	Test parameters	2524

A.9.1.27.3	Test Requirements	2527

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2527

A.9.1.28.1	Test Purpose and Environment	2527

A.9.1.28.2	Test parameters	2527

A.9.1.28.3	Test Requirements	2530

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2530

A.9.1.29.1	Test Purpose and Environment	2530

A.9.1.29.2	Test parameters	2530

A.9.1.29.3	Test Requirements	2533

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2533

A.9.1.30.1	Test Purpose and Environment	2533

A.9.1.30.2	Test parameters	2533

A.9.1.30.3	Test Requirements	2536

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2536

A.9.1.31.1	Test Purpose and Environment	2536

A.9.1.31.2	Test parameters	2536

A.9.1.31.3	Test Requirements	2539

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2539

A.9.1.32.1	Test Purpose and Environment	2539

A.9.1.32.2	Test parameters	2539

A.9.1.32.3	Test Requirements	2543

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2543

A.9.1.33.1	Test Purpose and Environment	2543

A.9.1.33.2	Test parameters	2543

A.9.1.33.3	Test Requirements	2546

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2546

A.9.1.34.1	Test Purpose and Environment	2546

A.9.1.34.2	Test parameters	2546

A.9.1.34.3	Test Requirements	2549

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2549

A.9.1.35.1	Test Purpose and Environment	2549

A.9.1.35.2	Test parameters	2549

A.9.1.35.3	Test Requirements	2552

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2552

A.9.1.36.1	Test Purpose and Environment	2552

A.9.1.36.2	Test parameters	2553

A.9.1.36.3	Test Requirements	2556

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2556

A.9.1.37.1	Test Purpose and Environment	2556

A.9.1.37.2	Test parameters	2556

A.9.1.37.3	Test Requirements	2559

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2560

A.9.1.38.1	Test Purpose and Environment	2560

A.9.1.38.2	Test parameters	2560

A.9.1.38.3	Test Requirements	2563

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2564

A.9.1.39.1	Test Purpose and Environment	2564

A.9.1.39.2	Test parameters	2564

A.9.1.39.3	Test Requirements	2570

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2570

A.9.1.40.1	Test Purpose and Environment	2570

A.9.1.40.2	Test parameters	2570

A.9.1.40.3	Test Requirements	2576

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2576

A.9.1.41.1	Test Purpose and Environment	2576

A.9.1.41.2	Test parameters	2576

A.9.1.41.3	Test Requirements	2579

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2579

A.9.1.42.1	Test Purpose and Environment	2579

A.9.1.42.2	Test parameters	2579

A.9.1.42.3	Test Requirements	2582

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2582

A.9.1.43.1	Test Purpose and Environment	2582

A.9.1.43.2	Test parameters	2582

A.9.1.43.3	Test Requirements	2583

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2584

A.9.1.44.1	Test Purpose and Environment	2584

A.9.1.44.2	Test parameters	2584

A.9.1.44.3	Test Requirements	2589

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2589

A.9.1.45.1	Test Purpose and Environment	2589

A.9.1.45.2	Test parameters	2589

A.9.1.45.3	Test Requirements	2594

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2594

A.9.1.46.1	Test Purpose and Environment	2594

A.9.1.46.2	Test parameters	2594

A.9.1.46.3	Test Requirements	2603

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2603

A.9.1.47.1	Test Purpose and Environment	2603

A.9.1.47.2	Test parameters	2604

A.9.1.47.3	Test Requirements	2613

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2613

A.9.1.48.1	Test Purpose and Environment	2613

A.9.1.48.2	Test parameters	2614

A.9.1.48.3	Test Requirements	2624

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2625

A.9.1.49.1	Test Purpose and Environment	2625

A.9.1.49.2	Test parameters	2625

A.9.1.49.3	Test Requirements	2635

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2636

A.9.1.50.1	Test Purpose and Environment	2636

A.9.1.50.2	Test parameters	2636

A.9.1.50.3	Test Requirements	2643

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2643

A.9.1.51.1	Test Purpose and Environment	2643

A.9.1.51.2	Test parameters	2643

A.9.1.51.3	Test Requirements	2649

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2650

A.9.1.52.1	Test Purpose and Environment	2650

A.9.1.52.2	Test parameters	2650

A.9.1.52.3	Test Requirements	2653

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2653

A.9.1.52A.1	Test Purpose and Environment	2653

A.9.1.52A.2	Test parameters	2653

A.9.1.52A.3	Test Requirements	2654

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2654

A.9.1.53.1	Test Purpose and Environment	2654

A.9.1.53.2	Test parameters	2655

A.9.1.53.3	Test Requirements	2658

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2658

A.9.1.53A.1	Test Purpose and Environment	2658

A.9.1.53A.2	Test parameters	2658

A.9.1.53A.3	Test Requirements	2659

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2660

A.9.1.54.1	Test Purpose and Environment	2660

A.9.1.54.2	Test parameters	2660

A.9.1.54.3	Test Requirements	2663

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2663

A.9.1.55.1	Test Purpose and Environment	2663

A.9.1.55.2	Test parameters	2663

A.9.1.55.3	Test Requirements	2667

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2667

A.9.1.56.1	Test Purpose and Environment	2667

A.9.1.56.2	Test parameters	2667

A.9.1.56.3	Test Requirements	2670

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2671

A.9.1.57.1	Test Purpose and Environment	2671

A.9.1.57.2	Test parameters	2671

A.9.1.57.3	Test Requirements	2674

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2674

A.9.1.57A.1	Test Purpose and Environment	2674

A.9.1.57A.2	Test parameters	2674

A.9.1.57A.3	Test Requirements	2675

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2675

A.9.1.58.1	Test Purpose and Environment	2675

A.9.1.58.2	Test parameters	2676

A.9.1.58.3	Test Requirements	2679

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2679

A.9.1.58A.1	Test Purpose and Environment	2679

A.9.1.58A.2	Test parameters	2679

A.9.1.58A.3	Test Requirements	2680

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2680

A.9.1.59.1	Test Purpose and Environment	2680

A.9.1.59.2	Test parameters	2681

A.9.1.59.3	Test Requirements	2683

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2683

A.9.1.60.1	Test Purpose and Environment	2683

A.9.1.60.2	Test parameters	2683

A.9.1.60.3	Test Requirements	2686

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2687

A.9.1.61.1	Test Purpose and Environment	2687

A.9.1.61.2	Test parameters	2687

A.9.1.61.3	Test Requirements	2690

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2690

A.9.1.62.1	Test Purpose and Environment	2690

A.9.1.62.2	Test parameters	2690

A.9.1.62.3	Test Requirements	2693

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2693

A.9.1.63.1	Test Purpose and Environment	2693

A.9.1.63.2	Test parameters	2693

A.9.1.63.3	Test Requirements	2696

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2696

A.9.1.64.1	Test Purpose and Environment	2696

A.9.1.64.2	Test parameters	2696

A.9.1.64.3	Test Requirements	2697

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2698

A.9.1.65.1	Test Purpose and Environment	2698

A.9.1.65.2	Test parameters	2698

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2704

A.9.1.66.1	Test Purpose and Environment	2704

A.9.1.66.2	Test parameters	2704

A.9.1.66.3	Test Requirements	2710

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2710

A.9.1.67.1	Test Purpose and Environment	2710

A.9.1.67.2	Test parameters	2710

A.9.1.67.3	Test Requirements	2711

A.9.2	RSRQ	2712

A.9.2.1	FDD Intra frequency case	2712

A.9.2.1.1	Test Purpose and Environment	2712

A.9.2.1.2	Test parameters	2712

A.9.2.1.3	Test Requirements	2715

A.9.2.2	TDD Intra frequency case	2715

A.9.2.2.1	Test Purpose and Environment	2715

A.9.2.2.2	Test parameters	2715

A.9.2.2.3	Test Requirements	2717

A.9.2.3	FDD—FDD Inter frequency case	2717

A.9.2.3.1	Test Purpose and Environment	2717

A.9.2.3.2	Test parameters	2717

A.9.2.3.3	Test Requirements	2720

A.9.2.4	TDD—TDD Inter frequency case	2720

A.9.2.4.1	Test Purpose and Environment	2720

A.9.2.4.2	Test parameters	2720

A.9.2.4.3	Test Requirements	2723

A.9.2.4A	FDD—TDD Inter frequency case	2723

A.9.2.4A.1	Test Purpose and Environment	2723

A.9.2.4A.2	Test parameters	2723

A.9.2.4A.3	Test Requirements	2725

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2726

A.9.2.5.1	Test Purpose and Environment	2726

A.9.2.5.2	Test parameters	2726

A.9.2.5.3	Test Requirements	2729

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2729

A.9.2.6.1	Test Purpose and Environment	2729

A.9.2.6.2	Test parameters	2729

A.9.2.6.3	Test Requirements	2732

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2732

A.9.2.7.1	Test Purpose and Environment	2732

A.9.2.7.2	Test parameters	2732

A.9.2.7.3	Test Requirements	2736

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2736

A.9.2.8.1	Test Purpose and Environment	2736

A.9.2.8.2	Test parameters	2736

A.9.2.8.3	Test Requirements	2739

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2739

A.9.2.9.1	Test Purpose and Environment	2739

A.9.2.9.2	Test parameters	2739

A.9.2.9.3	Test Requirements	2743

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2743

A.9.2.10.1	Test Purpose and Environment	2743

A.9.2.10.2	Test parameters	2743

A.9.2.10.3	Test Requirements	2747

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2747

A.9.2.11.1	Test Purpose and Environment	2747

A.9.2.11.2	Test parameters	2747

A.9.2.11.3	Test Requirements	2748

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2748

A.9.2.12.1	Test Purpose and Environment	2748

A.9.2.12.2	Test parameters	2748

A.9.2.12.3	Test Requirements	2749

A.9.2.13	Void	2750

A.9.2.13.1	Void	2750

A.9.2.13.2	Void	2750

A.9.2.13.3	Void	2750

A.9.2.14	Void	2750

A.9.2.14.1	Void	2750

A.9.2.14.2	Void	2750

A.9.2.14.3	Void	2750

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2750

A.9.2.15.1	Test Purpose and Environment	2750

A.9.2.15.2	Test parameters	2750

A.9.2.15.3	Test Requirements	2754

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2754

A.9.2.16.1	Test Purpose and Environment	2754

A.9.2.16.2	Test parameters	2754

A.9.2.16.3	Test Requirements	2758

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2758

A.9.2.17.1	Test Purpose and Environment	2758

A.9.2.17.2	Test parameters	2758

A.9.2.17.3	Test Requirements	2760

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2760

A.9.2.18.1	Test Purpose and Environment	2760

A.9.2.18.2	Test parameters	2760

A.9.2.18.3	Test Requirements	2763

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2763

A.9.2.19.1	Test Purpose and Environment	2763

A.9.2.19.2	Test parameters	2763

A.9.2.19.3	Test Requirements	2765

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2765

A.9.2.20.1	Test Purpose and Environment	2765

A.9.2.20.2	Test parameters	2765

A.9.2.20.3	Test Requirements	2768

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2768

A.9.2.21.1	Test Purpose and Environment	2768

A.9.2.21.2	Test parameters	2768

A.9.2.21.3	Test Requirements	2771

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2771

A.9.2.22.1	Test Purpose and Environment	2771

A.9.2.22.2	Test parameters	2771

A.9.2.22.3	Test Requirements	2772

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2772

A.9.2.23.1	Test Purpose and Environment	2772

A.9.2.23.2	Test parameters	2772

A.9.2.23.3	Test Requirements	2775

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2775

A.9.2.24.1	Test Purpose and Environment	2775

A.9.2.24.2	Test parameters	2775

A.9.2.24.3	Test Requirements	2776

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2776

A.9.2.25.1	Test Purpose and Environment	2776

A.9.2.25.2	Test parameters	2776

A.9.2.25.3	Test Requirements	2781

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2781

A.9.2.26.1	Test Purpose and Environment	2781

A.9.2.26.2	Test parameters	2781

A.9.2.26.3	Test Requirements	2785

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2785

A.9.2.27.1	Test Purpose and Environment	2785

A.9.2.27.2	Test parameters	2785

A.9.2.27.3	Test Requirements	2786

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2786

A.9.2.28.1	Test Purpose and Environment	2786

A.9.2.28.2	Test parameters	2786

A.9.2.28.3	Test Requirements	2789

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2789

A.9.2.29.1	Test Purpose and Environment	2789

A.9.2.29.2	Test parameters	2789

A.9.2.29.3	Test Requirements	2792

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2792

A.9.2.30.1	Test Purpose and Environment	2792

A.9.2.30.2	Test parameters	2792

A.9.2.30.3	Test Requirements	2795

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2795

A.9.2.31.1	Test Purpose and Environment	2795

A.9.2.31.2	Test parameters	2795

A.9.2.31.3	Test Requirements	2798

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2798

A.9.2.32.1	Test Purpose and Environment	2798

A.9.2.32.2	Test parameters	2798

A.9.2.32.3	Test Requirements	2801

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2801

A.9.2.33.1	Test Purpose and Environment	2801

A.9.2.33.2	Test parameters	2801

A.9.2.33.3	Test Requirements	2804

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2804

A.9.2.34.1	Test Purpose and Environment	2804

A.9.2.34.2	Test parameters	2804

A.9.2.34.3	Test Requirements	2807

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2807

A.9.2.35.1	Test Purpose and Environment	2807

A.9.2.35.2	Test parameters	2807

A.9.2.35.3	Test Requirements	2810

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2810

A.9.2.36.1	Test Purpose and Environment	2810

A.9.2.36.2	Test parameters	2810

A.9.2.36.3	Test Requirements	2811

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2812

A.9.2.37.1	Test Purpose and Environment	2812

A.9.2.37.2	Test parameters	2812

A.9.2.37.3	Test Requirements	2813

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2814

A.9.2.38.1	Test Purpose and Environment	2814

A.9.2.38.2	Test parameters	2814

A.9.2.38.3	Test Requirements	2818

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2818

A.9.2.39.1	Test Purpose and Environment	2818

A.9.2.39.2	Test parameters	2818

A.9.2.39.3	Test Requirements	2822

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2822

A.9.2.40.1	Test Purpose and Environment	2822

A.9.2.40.2	Test parameters	2822

A.9.2.40.3	Test Requirements	2825

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2825

A.9.2.41.1	Test Purpose and Environment	2825

A.9.2.41.2	Test parameters	2826

A.9.2.41.3	Test Requirements	2829

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2829

A.9.2.42.1	Test Purpose and Environment	2829

A.9.2.42.2	Test parameters	2829

A.9.2.42.3	Test Requirements	2832

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2832

A.9.2.43.1	Test Purpose and Environment	2832

A.9.2.43.2	Test parameters	2832

A.9.2.43.3	Test Requirements	2835

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2835

A.9.2.44.1	Test Purpose and Environment	2835

A.9.2.44.2	Test parameters	2835

A.9.2.44.3	Test Requirements	2837

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2837

A.9.2.45.1	Test Purpose and Environment	2837

A.9.2.45.2	Test parameters	2837

A.9.2.45.3	Test Requirements	2841

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2841

A.9.2.46.1	Test Purpose and Environment	2842

A.9.2.46.2	Test parameters	2842

A.9.2.46.3	Test Requirements	2846

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2846

A.9.2.47.1	Test Purpose and Environment	2847

A.9.2.47.2	Test parameters	2847

A.9.2.47.3	Test Requirements	2851

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2851

A.9.2.48.1	Test Purpose and Environment	2851

A.9.2.48.2	Test parameters	2852

A.9.2.48.3	Test Requirements	2856

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2857

A.9.2.49.1	Test Purpose and Environment	2857

A.9.2.49.2	Test parameters	2857

A.9.2.49.3	Test Requirements	2863

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2864

A.9.2.50.1	Test Purpose and Environment	2864

A.9.2.50.2	Test parameters	2864

A.9.2.50.3	Test Requirements	2870

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2870

A.9.2.51.1	Test Purpose and Environment	2870

A.9.2.51.2	Test parameters	2871

A.9.2.51.3	Test Requirements	2875

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2875

A.9.2.52.1	Test Purpose and Environment	2875

A.9.2.52.2	Test parameters	2875

A.9.2.52.3	Test Requirements	2878

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2879

A.9.2.53.1	Test Purpose and Environment	2879

A.9.2.53.2	Test parameters	2879

A.9.2.53.3	Test Requirements	2885

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2886

A.9.2.54.1	Test Purpose and Environment	2886

A.9.2.54.2	Test parameters	2886

A.9.2.54.3	Test Requirements	2892

A.9.3	UTRAN FDD CPICH RSCP	2892

A.9.3.1	E-UTRAN FDD	2892

A.9.3.1.1	Test Purpose and Environment	2892

A.9.3.1.2	Parameters	2892

A.9.3.1.3	Test Requirements	2895

A.9.3.2 E-UTRAN TDD	2895

A.9.3.2.1	Test Purpose and Environment	2895

A.9.3.2.2	Parameters	2895

A.9.3.2.3	Test Requirements	2898

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2898

A.9.3.3.1	Test Purpose and Environment	2898

A.9.3.3.2	Parameters	2898

A.9.3.3.3	Test Requirements	2899

A.9.4	UTRAN FDD CPICH Ec/No	2900

A.9.4.1	E-UTRAN FDD	2900

A.9.4.1.1	Test Purpose and Environment	2900

A.9.4.1.2	Parameters	2900

A.9.4.1.3	Test Requirements	2902

A.9.4.2	E-UTRAN TDD	2903

A.9.4.2.1	Test Purpose and Environment	2903

A.9.4.2.2	Parameters	2903

A.9.4.2.3	Test Requirements	2905

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2906

A.9.4.3.1	Test Purpose and Environment	2906

A.9.4.3.2	Parameters	2906

A.9.4.3.3	Test Requirements	2907

A.9.5	UTRAN TDD measurement	2907

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2907

A.9.5.1.1	Test Purpose and Environment	2907

A.9.5.1.2	Test parameters	2907

A.9.5.1.3	Test Requirements	2909

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2910

A.9.5.2.1	Test Purpose and Environment	2910

A.9.5.2.2	Test parameters	2910

A.9.5.2.3	Test Requirements	2911

A.9.6	GSM Carrier RSSI	2912

A.9.6.1	E-UTRAN FDD	2912

A.9.6.1.1	Test Purpose and Environment	2912

A.9.6.1.2	Test Requirements	2913

A.9.6.2	E-UTRAN TDD	2913

A.9.6.2.1	Test Purpose and Environment	2913

A.9.6.2.2	Test Requirements	2915

A.9.7	UE Rx – Tx Time Difference	2916

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2916

A.9.7.1.1	Test Purpose and Environment	2916

A.9.7.1.2	Test parameters	2916

A.9.7.1.3	Test Requirements	2917

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2917

A.9.7.2.1	Test Purpose and Environment	2917

A.9.7.2.2	Test parameters	2917

A.9.7.2.3	Test Requirements	2919

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2919

A.9.7.3.1	Test Purpose and Environment	2919

A.9.7.3.2	Test parameters	2919

A.9.7.3.3	Test Requirements	2922

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2922

A.9.7.4.1	Test Purpose and Environment	2922

A.9.7.4.2	Test Parameters	2922

A.9.7.4.3	Test Requirements	2925

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2925

A.9.7.5.1	Test Purpose and Environment	2925

A.9.7.5.2	Test parameters	2925

A.9.7.5.3	Test Requirements	2928

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2928

A.9.7.6.1	Test Purpose and Environment	2928

A.9.7.6.2	Test Parameters	2928

A.9.7.6.3	Test Requirements	2931

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2931

A.9.7.7.1	Test Purpose and Environment	2931

A.9.7.7.2	Test parameters	2931

A.9.7.7.3	Test Requirements	2932

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2933

A.9.7.8.1	Test Purpose and Environment	2933

A.9.7.8.2	Test parameters	2933

A.9.7.8.3	Test Requirements	2934

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2934

A.9.7.9.1	Test Purpose and Environment	2934

A.9.7.9.2	Test parameters	2934

A.9.7.9.3	Test Requirements	2935

A.9.8	RSTD	2936

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2936

A.9.8.1.1	Test Purpose and Environment	2936

A.9.8.1.2	Test Requirements	2938

A.9.8.1.2A	Test Requirements for UE Category 1bis	2938

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2939

A.9.8.2.1	Test Purpose and Environment	2939

A.9.8.2.2	Test Requirements	2942

A.9.8.2.2A	Test Requirements for UE Category 1bis	2942

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2943

A.9.8.3.1	Test Purpose and Environment	2943

A.9.8.3.2	Test Requirements	2945

A.9.8.3.2A	Test Requirements for UE Category 1bis	2945

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2946

A.9.8.4.1	Test Purpose and Environment	2946

A.9.8.4.2	Test Requirements	2948

A.9.8.4.2A	Test Requirements for UE Category 1bis	2948

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2949

A.9.8.5.1	Test Purpose and Environment	2949

A.9.8.5.2	Test Requirements	2951

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2951

A.9.8.6.1	Test Purpose and Environment	2951

A.9.8.6.2	Test Requirements	2954

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2954

A.9.8.7.1	Test Purpose and Environment	2954

A.9.8.7.2	Test Requirements	2955

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2955

A.9.8.8.1	Test Purpose and Environment	2955

A.9.8.8.2	Test Requirements	2956

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2956

A.9.8.9.1	Test Purpose and Environment	2956

A.9.8.9.2	Test Requirements	2957

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2957

A.9.8.10.1	Test Purpose and Environment	2957

A.9.8.10.2	Test Requirements	2958

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2958

A.9.8.11.1	Test Purpose and Environment	2958

A.9.8.11.2	Test Requirements	2959

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2959

A.9.8.12.1	Test Purpose and Environment	2959

A.9.8.12.2	Test Requirements	2960

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2960

A.9.8.13.1	Test Purpose and Environment	2960

A.9.8.13.2	Test Requirements	2961

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2961

A.9.8.14.1	Test Purpose and Environment	2961

A.9.8.14.2	Test Requirements	2968

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2968

A.9.8.15.1	Test Purpose and Environment	2968

A.9.8.15.2	Test Requirements	2974

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2974

A.9.8.16.1	Test Purpose and Environment	2974

A.9.8.16.2	Test Requirements	2977

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2977

A.9.8.17.1	Test Purpose and Environment	2977

A.9.8.17.2	Test Requirements	2980

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2980

A.9.8.18.1	Test Purpose and Environment	2980

A.9.8.18.2	Test Requirements	2983

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2983

A.9.8.19.1	Test Purpose and Environment	2983

A.9.8.19.2	Test Requirements	2986

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2986

A.9.8.20.1	Test Purpose and Environment	2986

A.9.8.20.2	Test Requirements	2989

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2989

A.9.8.21.1	Test Purpose and Environment	2989

A.9.8.21.2	Test Requirements	2993

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2994

A.9.8.22.1	Test Purpose and Environment	2994

A.9.8.22.2	Test Requirements	2997

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2998

A.9.8.23.1	Test Purpose and Environment	2998

A.9.8.23.2	Test Requirements	3001

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3001

A.9.8.24.1	Test Purpose and Environment	3001

A.9.8.24.2	Test Requirements	3005

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3006

A.9.8.25.1	Test Purpose and Environment	3006

A.9.8.25.2	Test Requirements	3009

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3010

A.9.8.26.1	Test Purpose and Environment	3010

A.9.8.26.2	Test Requirements	3012

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3012

A.9.8.27.1	Test Purpose and Environment	3012

A.9.8.27.2	Test Requirements	3015

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3015

A.9.8.28.1	Test Purpose and Environment	3015

A.9.8.28.2	Test Requirements	3018

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3018

A.9.8.29.1	Test Purpose and Environment	3018

A.9.8.29.2	Test Requirements	3022

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3023

A.9.8.30.1	Test Purpose and Environment	3023

A.9.8.30.2	Test Requirements	3026

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3027

A.9.8.31.1	Test Purpose and Environment	3027

A.9.8.31.2	Test Requirements	3031

A.9.9	RSRP and RSRQ on the serving cell	3031

A.9.9.1	FDD Intra frequency serving cell case	3031

A.9.9.1.1	Test Purpose and Environment	3031

A.9.9.1.2	Test parameters	3031

A.9.9.1.3	Test Requirements	3034

A.9.9.2	TDD Intra frequency serving cell case	3034

A.9.9.2.1	Test Purpose and Environment	3034

A.9.9.2.2	Test parameters	3034

A.9.9.2.3	Test Requirements	3036

A.9.10	SSTD	3036

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3036

A.9.10.1.1	Test Purpose and Environment	3036

A.9.10.1.2	Test parameters	3036

A.9.10.1.3	Test Requirements	3038

A.9.10.2	Void	3038

A.9.10.3	Void	3038

A.9.10.4	Void	3038

A.9.11	RSSI	3038

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	3038

A.9.11.1.1	Test Purpose and Environment	3038

A.9.11.1.2	Test parameters	3038

A.9.11.1.3	Test Requirements	3041

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	3041

A.9.11.2.1	Test Purpose and Environment	3041

A.9.11.2.2	Test parameters	3041

A.9.12	Channel occupancy	3044

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	3044

A.9.12.1.1	Test Purpose and Environment	3044

A.9.12.1.2	Test parameters	3044

A.9.12.1.3	Test Requirements	3047

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	3047

A.9.12.2.1	Test Purpose and Environment	3047

A.9.12.2.2	Test parameters	3047

A.9.12.2.3	Test Requirements	3050

A.9.13	RS-SINR	3050

A.9.13.1	FDD Intra-Frequency Case	3050

A.9.13.1.1	Test Purpose and Environment	3050

A.9.13.1.2	Test parameters	3050

A.9.13.1.3	Test Requirements	3053

A.9.13.2	TDD Intra-Frequency Case	3053

A.9.13.2.1	Test Purpose and Environment	3053

A.9.13.2.2	Test parameters	3053

A.9.13.2.3	Test Requirements	3056

A.9.13.3	FDD—FDD Inter frequency case	3056

A.9.13.3.1	Test Purpose and Environment	3056

A.9.13.3.2	Test parameters	3056

A.9.13.3.3	Test Requirements	3059

A.9.13.4	TDD—TDD Inter frequency case	3059

A.9.13.4.1	Test Purpose and Environment	3059

A.9.13.4.2	Test parameters	3059

A.9.13.4.3	Test Requirements	3063

A.9.13.5	FDD—TDD Inter frequency case	3064

A.9.13.5.1	Test Purpose and Environment	3064

A.9.13.5.2	Test parameters	3064

A.9.13.5.3	Test Requirements	3070

A.9.13.6	TDD—FDD Inter frequency case	3070

A.9.13.6.1	Test Purpose and Environment	3070

A.9.13.6.2	Test parameters	3070

A.9.13.6.3	Test Requirements	3074

A.9.14	Channel quality reporting accuracy	3074

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3074

A.9.14.1.1	Test Purpose and Environment	3074

A.9.14.1.2	Test parameters	3074

A.9.14.1.3	Test Requirements	3075

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3075

A.9.14.2.1	Test Purpose and Environment	3075

A.9.14.2.2	Test parameters	3076

A.9.14.2.3	Test Requirements	3076

A.10	Proximity-based Services in Any Cell Selection State	3077

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3077

A.10.1.1	Test Purpose and Environment	3077

A.10.1.2	Test Requirements	3078

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3078

A.10.2.1	Test Purpose and Environment	3078

A.10.2.2	Test Requirements	3079

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3080

A.10.3.1	Test Purpose and Environment	3080

A.10.3.2	Test Requirements	3082

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3083

A.10.4.1	Test Purpose and Environment	3083

A.10.4.2	Test Requirements	3085

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3085

A.11.1	V2V UE Transmission Timing Accuracy Test	3085

A.11.1.1	Test Purpose and Environment	3085

A.11.1.2	Test requirements	3086

A.11.2	Interruptions due to V2V sidelink communication	3086

A.11.2.1	Test Purpose and Environment	3086

A.11.2.2	Test Requirements	3088

A.12	3088

A.12.1	V2X UE Transmission Timing Accuracy Test	3088

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3088

A.12.1.1.1	Test Purpose and Environment	3088

A.12.1.1.2	Test requirements	3089

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3090

A.12.1.2.1	Test Purpose and Environment	3090

A.12.1.2.2	Test Requirements	3090

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3091

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3091

A.12.2.1.1	Test Purpose and Environment	3091

A.12.2.1.2	Test Requirements	3092

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3093

A.12.2.2.1	Test Purpose and Environment	3093

A.12.2.2.2	Test Requirements	3094

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3094

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3094

A.12.3.1.1	Test Purpose and Environment	3094

A.12.3.1.2	Test Requirements	3097

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3098

A.12.3.2.1	Test Purpose and Environment	3098

A.12.3.1.2	Test Requirements	3100

A.12.4	Congestion Control Measurement Test for V2X UE	3100

A.12.4.1	Test Purpose and Environment	3100

A.12.4.2	Test Requirements	3102

A.12.5	Interruptions due to V2X Sidelink Communication	3102

A.12.5.1	Test Purpose and Environment	3102

A.12.5.2	Test Requirements	3104

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3104

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3104

A.12.6.1.1	Test Purpose and Environment	3104

A.12.6.1.2	Test Requirements	3106

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3106

A.12.6.2.1	Test Purpose and Environment	3106

A.12.6.1.2	Test Requirements	3108

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3109

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3109

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3109

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3109

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3109

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3111

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3111

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3112

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3112

B.1.7.1 Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3112

B.1.7.2 Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3113

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3115

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3115

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3115

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3116

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3116

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3116

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3117

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3117

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3117

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3118

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3118

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3119

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3119

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3119

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3120

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3120

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3120

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3121

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3121

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3121

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3121

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3122

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3123

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3123

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3124

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3124

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3126

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3126

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3126

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3126

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3127

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3127

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3129

B.3	Conditions for measurements performance requirements for UE	3129

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3129

B.3.2	Void	3129

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3129

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3129

B.3.5	Conditions for UE Rx – Tx time difference	3129

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3130

B.3.7	Conditions for inter-frequency RSTD measurements	3130

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3130

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3130

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3130

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3131

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3131

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3131

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3131

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3131

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3131

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3132

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3132

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3132

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3133

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3133

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3133

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3133

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3133

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3133

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3133

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3133

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3133

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3134

B.3.21.1	Conditions for RSRP measurements	3134

B.3.21.2	Conditions for RSRQ measurements	3134

B.3.21.3	Conditions for CSI-RSRP measurements	3134

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3134

B.3.22.1	Conditions for RSRP measurements	3134

B.3.22.2	Void	3135

B.3.22.3	Conditions for CSI-RSRP measurements	3135

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3135

B.3.23.1	Conditions for RSRP measurements	3135

B.3.23.2	Conditions for RSRQ measurements	3135

B.3.23.3	Conditions for CSI-RSRP measurements	3135

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3136

B.3.24.1	Conditions for RSRP measurements	3136

B.3.24.2	Conditions for RSRQ measurements	3136

B.3.24.3	Conditions for CSI-RSRP measurements	3136

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3136

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3136

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3136

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3137

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3137

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3137

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3137

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3137

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3138

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3138

B.4	RRM Requirements Exceptions	3138

B.4.1	General	3138

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3138

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3138

B.4.3.1	Inter-band carrier aggregation	3138

B.4.3.2	Intra-band non-contiguous carrier aggregation	3138

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3139

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3139

B.5.1	Conditions for S-RSRP Accuracy Requirements	3139

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3139

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3140

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3140

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3141

B.6	Conditions for V2X	3141

B.6.1	Test parameters for GNSS signals	3141

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3141

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3141

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3142

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3142

Annex C (informative): Change history:	3143