3GPP TS 36.133 V15.17.0 (2022-06)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);
Requirements for support of radio resource management

(Release 15)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	87

1	Scope	88

2	References	88

3	Definitions, symbols and abbreviations	90

3.1	Definitions	90

3.2	Symbols	92

3.3	Abbreviations	93

3.4	Test tolerances	96

3.5	Additional notation	96

3.5.1	Groups of bands	96

3.6	General	98

3.6.1	Applicability of requirements in this specification version	98

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	105

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	106

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	107

3.6.2	Applicability of requirements for EN-DC operation	108

3.6.3	Applicability of requirements for NE-DC operation	109

3.6.4	Applicability of requirements for NGEN-DC operation	110

4	E-UTRAN RRC_IDLE state mobility	110

4.1	Cell Selection	110

4.2	Cell Re-selection	110

4.2.1	Introduction	110

4.2.2	Requirements	110

4.2.2.1	Measurement and evaluation of serving cell	111

4.2.2.2	Void	112

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	112

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	114

4.2.2.5	Measurements of inter-RAT cells	116

4.2.2.5.1	Measurements of UTRAN FDD cells	117

4.2.2.5.2	Measurements of UTRAN TDD cells	118

4.2.2.5.3	Measurements of GSM cells	120

4.2.2.5.4	Measurements of HRPD cells	121

4.2.2.5.5	Measurements of cdma2000 1X	122

4.2.2.5.6	Measurements of NR cells	123

4.2.2.6	Evaluation of cell re-selection criteria	124

4.2.2.7	Maximum interruption in paging reception	124

4.2.2.8	void	125

4.2.2.9	UE measurement capability	125

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	126

4.2.2.10	Reselection to CSG cells	126

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	126

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	127

4.2.2.11	Void	128

4.2.2.12	Void	128

4.2.2.13	Void	128

4.3	Minimization of Drive Tests (MDT)	128

4.3.1	Introduction	129

4.3.2	Measurements	129

4.3.2.1	Requirements	129

4.3.3	Relative Time Stamp Accuracy	129

4.3.3.1	Requirements	129

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	129

4.3.4.1	 Requirements	129

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	130

4.3.5.1	Requirements for timeSinceFailure	130

4.4	MBSFN Measurements	130

4.4.1	Introduction	130

4.4.2	MBSFN RSRP measurements	130

4.4.3	MBSFN RSRQ measurements	130

4.4.4	MCH BLER measurements	131

4.5	Proximity-based Services	131

4.5.1	Introduction	131

4.5.2	Requirements	131

4.5.2.1	Interruptions with ProSe Direct Discovery	131

4.5.2.2	Interruptions with ProSe Direct Communication	131

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	131

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	132

4.6	Cell Selection and Re-selection Requirements for UE category NB1	132

4.6.1	Cell Selection	132

4.6.2	Cell Re-selection	132

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	133

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	133

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	134

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	135

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	136

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	137

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	139

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	140

4.6.2.7	Maximum interruption in paging reception in normal coverage	141

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	141

4.6.2.8	UE measurement capability	141

4.6.2.9	WUS receptions for NB1	142

4.7	Cell Selection and Re-selection Requirements for UE category M1	142

4.7.1	Cell Selection	142

4.7.2	Cell Re-selection	142

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	142

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	142

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	143

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	144

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	146

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	146

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	147

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	147

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	148

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	150

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	151

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	152

4.7.2.3	WUS receptions for UE category M1	152

4.8	Idle State Positioning Measurement Requirements for UE category NB1	153

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	153

4.8.1.1	RSTD Measurement Reporting Delay	154

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	155

4.8.2.1	RSTD Measurement Reporting Delay	156

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	157

4.8.3.1	RSTD Measurement Reporting Delay	158

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	159

4.8.4.1	RSTD Measurement Reporting Delay	160

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	161

4.8.5.1	Measurement Reporting Delay	162

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	162

4.8.6.1	Measurement Reporting Delay	163

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	163

4.8.7.1	Measurement Reporting Delay	164

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	165

4.8.8.1	Measurement Reporting Delay	166

4.9	Idle Mode CA Measurement	167

4.9.1	Introduction	167

4.9.2	Requirement	167

4.9.2.1	Detected cell requirement during state transition and Idle mode	167

4.9.2.2	Measurements of inter-frequency CA candidate cells	167

4.9.2.3	Measurements on serving cell	168

4A	E-UTRAN RRC_INACTIVE state mobility	168

4A.1	Cell Re-selection	168

4A.1.1	Introduction	168

4A.1.2	Requirements	168

4A.1.2.1	UE measurement capability	168

4A.1.2.2	Measurement and evaluation of serving cell	168

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	168

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	168

4A.1.2.5	Evaluation of cell re-selection criteria	168

4A.1.2.6	Maximum interruption in paging reception	168

4A.1.2.7	Measurements of inter-RAT NR cells	168

5	E-UTRAN RRC_CONNECTED state mobility	169

5.1	E-UTRAN Handover	169

5.1.1	Introduction	169

5.1.2	Requirements	169

5.1.2.1	E-UTRAN FDD – FDD	169

5.1.2.1.1	Handover delay	169

5.1.2.1.2	Interruption time	170

5.1.2.2	E-UTRAN FDD – TDD	171

5.1.2.2.1	(Void)	171

5.1.2.2.2	(Void)	171

5.1.2.3	E-UTRAN TDD – FDD	171

5.1.2.3.1	(Void)	171

5.1.2.3.2	(Void)	171

5.1.2.4	E-UTRAN TDD – TDD	171

5.1.2.4.1	Handover delay	171

5.1.2.4.2	Interruption time	172

5.1.2.5	E-UTRAN HD–FDD	173

5.1.2.5.1	Handover delay	173

5.1.2.5.2	Interruption time	174

5.2	Void	175

5.3	Handover to other RATs	175

5.3.1	E-UTRAN - UTRAN FDD Handover	175

5.3.1.1	Introduction	175

5.3.1.1.1	Handover delay	175

5.3.1.1.2	Interruption time	175

5.3.2	E-UTRAN - UTRAN TDD Handover	176

5.3.2.1	Introduction	176

5.3.2.2	Requirements	176

5.3.2.2.1	Handover delay	176

5.3.2.2.2	Interruption time	176

5.3.3	E-UTRAN - GSM Handover	177

5.3.3.1	Introduction	177

5.3.3.2	Requirements	177

5.3.3.2.1	Handover delay	177

5.3.3.2.2	Interruption time	177

5.3.4	E-UTRAN - NR FR1 Handover	177

5.3.4.1	Introduction	177

5.3.4.2	Handover delay	178

5.3.4.3	Interruption time	178

5.3.5	E-UTRAN - NR FR2 Handover	178

5.3.5.1	Introduction	178

5.3.5.2	Handover delay	178

5.3.5.3	Interruption time	179

5.4	Handover to Non-3GPP RATs	179

5.4.1	E-UTRAN – HRPD Handover	179

5.4.1.1	Introduction	179

5.4.1.1.1	Handover delay	180

5.4.1.1.2	Interruption time	180

5.4.2	E-UTRAN – cdma2000 1X Handover	180

5.4.2.1	Introduction	180

5.4.2.1.1	Handover delay	180

5.4.2.1.2	Interruption time	180

5.5	E-UTRAN Handover for Cat-M1 UEs	181

5.5.1	Introduction	181

5.5.2	Requirements in CEModeA	181

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	181

5.5.2.1.1	Handover delay	181

5.5.2.1.2	Interruption time	181

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	182

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	182

5.5.2.3.1	Void	182

5.5.2.3.2	Void	182

5.5.3	Requirements in CEModeB	182

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	182

5.5.3.1.1	Handover delay	182

5.5.3.1.2	Interruption time	183

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	183

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	183

5.6	Void	183

6	RRC Connection Mobility Control	183

6.1	RRC Re-establishment	183

6.1.1	Introduction	184

6.1.2	Requirements	184

6.1.2.1	UE Re-establishment delay requirement	184

6.2	Random Access	184

6.2.1	Introduction	184

6.2.2	Requirements	184

6.2.2.1	Contention based random access	185

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	185

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	185

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	185

6.2.2.1.4	Void	185

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	185

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	185

6.2.2.2	Non-Contention based random access	185

6.2.2.2.1	Correct behaviour when receiving Random Access Response	185

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	185

6.2.3	Requirements for Cat-M1 UEs	186

6.3	RRC Connection Release with Redirection	186

6.3.1	Introduction	186

6.3.2	Requirements	186

6.3.2.1	RRC connection release with redirection to UTRAN FDD	186

6.3.2.2	RRC connection release with redirection to GERAN	186

6.3.2.3	RRC connection release with redirection to UTRAN TDD	187

6.3.2.4	RRC connection release with redirection to NR	187

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	188

6.4.1	Introduction	188

6.4.2	Requirements	188

6.5	RRC Re-establishment for NB-IoT UEs	189

6.5.1	Introduction	189

6.5.2	Requirements	189

6.5.2.1	UE Re-establishment delay requirement in normal coverage	189

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	189

6.6	Random Access for UE category NB1	190

6.6.1	Introduction	190

6.6.2	Requirements	190

6.6.2.1	Correct behaviour when receiving Random Access Response reception	190

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	190

6.6.2.3	Correct behaviour when receiving a NACK on msg3	190

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	190

6.6.2.5	Correct behaviour when contention Resolution timer expires	191

6.6.2.6	MSG3-based channel quality report for UE Category NB1	191

6.6.3	Requirements for NPRACH configuration	191

6.7	RRC Re-establishment for Cat-M1 UEs	192

6.7.1	Introduction	192

6.7.2	Requirements	192

6.7.2.1	UE Re-establishment delay requirement for CEModeA	192

6.7.2.2	UE Re-establishment delay requirement for CEModeB	192

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	193

6.8.1	Introduction	193

6.8.2	Requirements	193

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	193

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	194

6.9.1	Introduction	194

6.9.2	Requirements	194

7	Timing and signalling characteristics	195

7.1	UE transmit timing	195

7.1.1	Introduction	195

7.1.2	Requirements	195

7.2	UE timer accuracy	196

7.2.1	Introduction	196

7.2.2	Requirements	196

7.3	Timing Advance	197

7.3.1	Introduction	197

7.3.2	Requirements	197

7.3.2.1	Timing Advance adjustment delay	197

7.3.2.2	Timing Advance adjustment accuracy	197

7.4	Cell phase synchronization accuracy (TDD)	197

7.4.1	Definition	197

7.4.2	Minimum requirements	197

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	198

7.5.1	Introduction	198

7.5.2	eNodeB Synchronization Requirements	198

7.5.2.1	Synchronized E-UTRAN	198

7.5.2.2	Non-Synchronized E-UTRAN	198

7.6	Radio Link Monitoring	199

7.6.1	Introduction	199

7.6.2	Requirements	200

7.6.2.1	Minimum requirement when no DRX is used	200

7.6.2.2	Minimum requirement when DRX is used	201

7.6.2.3	Minimum requirement at transitions	202

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	202

7.6.2.5	Minimum requirement under IDC Interference	203

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	203

7.7.1	Introduction	203

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	203

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	204

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	205

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	207

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	208

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	208

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	209

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	209

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	209

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	211

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	211

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	212

7.7.14	SCell Activation Delay Requirement for Dormant SCell	213

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	214

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	215

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	216

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	216

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	218

7.8	Interruptions with Carrier Aggregation	220

7.8.1	Introduction	220

7.8.2	Requirements	221

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	221

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	221

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	221

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	221

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	221

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	221

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	222

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	222

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	222

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	223

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	223

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	224

7.8.2.13	Interruptions at SRS carrier based switching	224

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	225

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	225

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	225

7.8.2.17	Interruptions during CQI measurement on dormant SCell	226

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	226

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	226

7.8.2.20	Interruptions at SCell hibernation	226

7.8.2.21	Interruptions at direct SCell activation and hibernation	227

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	227

7.9.1	Introduction	227

7.9.2	Minimum Requirements for Interband Carrier Aggregation	227

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	228

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	228

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	228

7.10.1	Introduction	228

7.10.2	Requirements	228

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	229

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	229

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	229

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	230

7.11	Radio Link Monitoring for UE Category 0	230

7.11.1	Introduction	230

7.11.2	Requirements for FD-FDD and TDD	231

7.11.2.1	Minimum requirement when no DRX is used	231

7.11.2.2	Minimum requirement when DRX is used	232

7.11.2.3	Minimum requirement at transitions	232

7.11.3	Requirements for HD-FDD	233

7.11.3.1	Minimum requirement when no DRX is used	233

7.11.3.2	Minimum requirement when DRX is used	233

7.11.3.3	Minimum requirement at transitions	234

7.12	Interruptions with Dual Connectivity	234

7.12.1	Introduction	234

7.12.2	Requirements	234

7.12.2.1	Interruptions at PSCell addition/release	234

7.12.2.2	Interruptions at transitions between active and non-active during DRX	234

7.12.2.3	Interruptions at transitions from non-DRX to DRX	235

7.12.2.4	Interruptions at SCell addition/release	235

7.12.2.5	Interruptions at SCell activation/deactivation	235

7.12.2.6	Interruptions during measurements on SCC	236

7.12.2.7	Interruptions at SRS carrier based switching	236

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	237

7.13.1	Definition	237

7.13.2	Minimum requirements	237

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	237

7.14.1	Introduction	237

7.14.2	PSCell Addition Delay Requirement	237

7.14.3	PSCell Release Delay Requirement	238

7.15	Maximum Receive Timing Difference in Dual Connectivity	238

7.15.1	Introduction	238

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	238

7.16	Proximity-based Services	238

7.16.1	Introduction	238

7.16.2	Requirements	238

7.16.2.1	ProSe UE transmission timing	238

7.16.2.1.1	Serving cell or PCell as timing reference	239

7.16.2.1.2	SCell or non-serving cell as timing reference	239

7.16.3	Interruptions with ProSe	239

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	239

7.16.3.2	Interruptions at ProSe Direct Communication configuration	240

7.16.3.3	Interruptions during ProSe Direct Discovery	240

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	240

7.16.3.5	Interruptions during ProSe Direct Communication	241

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	241

7.16.4.1	Measurement and evaluation of selected cell	241

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	241

7.16.5	Selection / Reselection of ProSe relay UE	242

7.16.6	ProSe operation under deactivated SCell	242

7.17	Maximum Transmission Timing Difference in Dual Connectivity	242

7.17.1	Introduction	242

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	243

7.18.1	Introduction	243

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	243

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	243

7.19	Radio Link Monitoring for UE Category M1	243

7.19.1	Introduction	243

7.19.2	Requirements for FD-FDD and TDD CE mode A	244

7.19.2.1	Minimum requirement when no DRX is used	245

7.19.2.2	Minimum requirement when DRX is used	245

7.19.2.3	Minimum requirement at transitions	247

7.19.3	Requirements for HD-FDD with CE mode A	247

7.19.3.1	Minimum requirement when no DRX is used	247

7.19.3.2	Minimum requirement when DRX is used	247

7.19.3.3	Minimum requirement at transitions	248

7.19.4	Requirements for FD-FDD and TDD with CE mode B	248

7.19.4.1	Minimum requirement when no DRX is used	249

7.19.4.2	Minimum requirement when DRX is used	250

7.19.4.3	Minimum requirement at transitions	251

7.19.5	Requirements for HD-FDD with CE mode B	252

7.19.5.1	Minimum requirement when no DRX is used	252

7.19.5.2	Minimum requirement when DRX is used	252

7.19.5.3	Minimum requirement at transitions	253

7.20	UE transmit timing for NB-IoT	253

7.20.1	Introduction	253

7.20.2	Requirements	253

7.21	 UE timer accuracy for NB-IoT	254

7.21.1	Introduction	254

7.21.2	Requirements	254

7.22	Timing Advance for NB-IoT	254

7.22.1	Introduction	254

7.22.2	Requirements	254

7.22.2.1	Timing Advance adjustment delay	254

7.22.2.2	Timing Advance adjustment accuracy	255

7.23	Radio Link Monitoring for Category NB1 UE	255

7.23.1	Introduction	255

7.23.2	Requirements for Category NB1 UE	255

7.23.2.1	Minimum requirement when no DRX is used	255

7.23.2.2	Minimum requirement when DRX is used	256

7.23.2.3	Minimum requirement at transitions	256

7.24	UE transmit timing for Category M1	257

7.24.1	Introduction	257

7.24.2	Requirements	257

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	258

7.25.1	Definition	258

7.25.2	Minimum requirements	258

7.26	UE transmit timing for Category M2	258

7.26.1	Introduction	258

7.26.2	Requirements	258

7.27	UE timer accuracy for category M1	259

7.27.1	Introduction	259

7.27.2	Requirements	259

7.28	Timing Advance for Category M1	259

7.28.1	Introduction	259

7.28.2	Requirements	259

7.29	Interruptions requirements with FeMBMS	259

7.29.1	Introduction	259

7.29.2	Requirements	259

7.30	Numerology switching delay requirements with FeMBMS	259

7.30.1	Introduction	259

7.30.2	Requirements	259

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	260

7.31.1	Introduction	260

7.31.2	NR PSCell Addition Delay Requirement	260

7.31.3	NR PSCell Release Delay Requirement	261

7.32	Interruptions with EN-DC	261

7.32.1	Introduction	261

7.32.2	Requirements	261

7.32.2.1	Interruptions at PSCell addition/release	261

7.32.2.2	Interruptions at transitions between active and non-active during DRX	261

7.32.2.3	Interruptions at transitions from non-DRX to DRX	262

7.32.2.4	Interruptions at SCell addition/release	262

7.32.2.5	Interruptions at SCell activation/deactivation	262

7.32.2.6	Interruptions during measurements on SCC	262

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	262

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	263

7.32.2.7	Interruptions at active BWP switching	263

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	264

7.33.1	Introduction	264

7.33.2	Requirements	264

7.34	Void	264

7.35	Interruptions with SFTD measurements	264

7.35.1	Introduction	264

7.35.2	Requirements	264

7.36	Interruptions with NE-DC	265

7.32.1	Introduction	265

7.36.2	Requirements	265

7.36.2.1	Interruptions at transitions between active and non-active during DRX	265

7.36.2.2	Interruptions at transitions from non-DRX to DRX	265

7.36.2.3	Interruptions at SCell addition/release	265

7.36.2.4	Interruptions at SCell activation/deactivation	266

7.36.2.5	Interruptions during measurements on SCC	266

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	266

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	266

7.36.2.6	Interruptions at active BWP switching	266

8	UE Measurements Procedures in RRC_CONNECTED State	267

8.1	General Measurement Requirements	267

8.1.1	Introduction	267

8.1.2	Requirements	267

8.1.2.1	UE measurement capability	267

8.1.2.1.1	Monitoring of multiple layers using gaps	274

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	275

8.1.2.1.1b	Monitoring of multiple layers using gaps (E-UTRA-NR dual connectivity)	276

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	278

8.1.2.1.2	Network controlled small gap	278

8.1.2.2	E-UTRAN intra frequency measurements	280

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	280

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	284

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	288

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	289

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	290

8.1.2.3	E-UTRAN inter frequency measurements	290

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	291

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	296

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	303

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	303

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	303

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	304

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	306

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	307

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	308

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	315

8.1.2.4	Inter RAT measurements	315

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	315

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	320

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	320

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	324

8.1.2.4.5	E-UTRAN FDD – GSM measurements	324

8.1.2.4.6	E-UTRAN TDD – GSM measurements	329

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	329

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	331

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	331

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	331

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	332

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	332

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	332

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	332

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	334

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	334

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	334

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	334

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	335

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	335

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	337

8.1.2.4.21	E-UTRAN FDD – NR measurements	337

8.1.2.4.22	E-UTRAN TDD – NR measurements	340

8.1.2.4.23	Void	340

8.1.2.4.24	Void	340

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	340

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	342

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	342

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	342

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	344

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	346

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	347

8.1.2.6.5	Void	349

8.1.2.6.6	Void	349

8.1.2.6.7	Void	349

8.1.2.6.8	Void	349

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	349

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	350

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	351

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	353

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	355

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	356

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	358

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	360

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	362

8.1.2.7	E-UTRAN E-CID Measurements	363

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	363

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	365

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	366

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	367

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	367

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	367

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	370

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	373

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	376

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	380

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	380

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	380

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	381

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	381

8.1.2.10	Void	382

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	382

8.2.1	Introduction	382

8.2.2	Requirements	382

8.3	Measurements for E-UTRA carrier aggregation	386

8.3.1	Introduction	386

8.3.2	Measurements of the primary component carrier	386

8.3.3	Measurements of a secondary component carrier	386

8.3.3.1	Measurements of a secondary component carrier with active SCell	386

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	387

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	387

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	388

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	389

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	390

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	390

8.4.1	Introduction	390

8.4.2	Measurements on the primary component carrier	390

8.4.3	Measurements on a secondary component carrier	391

8.4.4	Measurements on both primary component carrier and a secondary component carrier	392

8.4.5	Measurements on different secondary component carriers	393

8.5	Measurements for UE category 0	394

8.5.1	Introduction	394

8.5.2 	Requirements	394

8.5.2.1 	E-UTRAN intra frequency measurements	394

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	394

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	397

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	399

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	403

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	404

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	404

8.6	Discovery signal measurements	405

8.6.1	Introduction	405

8.6.2	Requirements for CRS based discovery signal measurements	405

8.6.2.1	E-UTRAN intra frequency measurements	405

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	406

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	408

8.6.2.2	E-UTRAN inter frequency measurements	410

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	411

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	413

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	416

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	416

8.6.3	Requirements for CSI-RS based discovery signal measurements	416

8.6.3.1	E-UTRAN intra frequency measurements	416

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	417

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	419

8.6.3.2	E-UTRAN inter frequency measurements	421

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	422

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	424

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	427

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	427

8.7	Discovery signal measurements for E-UTRA carrier aggregation	427

8.7.1	Introduction	427

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	428

8.7.2.1	Measurements of the primary component carrier	428

8.7.2.2	Measurements of a secondary component carrier	428

8.7.2.3	Measurements of a secondary component carrier with active SCell	428

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	428

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	428

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	429

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	431

8.7.3.1	Measurements of the primary component carrier	431

8.7.3.2	Measurements of a secondary component carrier	431

8.7.3.3	Measurements of a secondary component carrier with active SCell	431

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	431

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	431

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	433

8.8	Measurements for E-UTRA dual connectivity	434

8.8.1	Introduction	434

8.8.2	Intra-frequency measurements requirements on PCell	434

8.8.3	Intra-frequency measurements requirements on PSCell	435

8.8.4	Inter-frequency and inter-RAT measurement requirements	435

8.8.5	Intra-frequency measurements with autonomous gaps	435

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	435

8.8.5.2	ECGI reporting delay	436

8.8.6	Inter-frequency measurements with autonomous gaps	436

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	436

8.8.6.2	ECGI reporting delay	437

8.8.7	SSTD Measurements	437

8.8.7.1	Introduction	437

8.8.7.2	SSTD Measurement requirements	437

8.8.7.3	SSTD Measurement Reporting Delay	438

8.8.8	Intra-frequency measurements requirements on SCell	438

8.9	MBSFN Measurements	438

8.9.1	Introduction	438

8.9.2	MBSFN RSRP Measurements	438

8.9.3	MBSFN RSRQ Measurements	439

8.9.4	MCH BLER Measurements	439

8.10	Proximity-based Services	439

8.10.1	Introduction	439

8.10.2	Requirements	439

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	439

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	440

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	440

8.11.1	Introduction	440

8.11.2	CRS based discovery signal measurements	441

8.11.2.1	E-UTRAN intra-frequency measurements	441

8.11.2.1.1	Requirements	441

8.11.2.1.1.1	Requirements when no DRX is used	441

8.11.2.1.1.1.1	Measurement Reporting Requirements	442

8.11.2.1.1.2	Requirements when DRX is used	443

8.11.2.1.1.2.1	Measurement Reporting Requirements	445

8.11.2.2	E-UTRAN inter-frequency measurements	446

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	446

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	450

8.11.3	CSI-RS based discovery signal measurements	450

8.11.3.1	E-UTRAN intra-frequency measurements	450

8.11.3.1.1	Requirements	450

8.11.3.1.1.1	Requirements when no DRX is used	450

8.11.3.1.1.1.1	Measurement Reporting Requirements	452

8.11.3.1.1.2	Requirements when DRX is used	452

8.11.3.1.1.2.1	Measurement Reporting Requirements	453

8.11.3.2	E-UTRAN inter-frequency measurements	454

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	454

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	458

8.11.4	RSSI measurements	458

8.11.4.1	E-UTRAN intra-frequency measurements	458

8.11.4.2	E-UTRAN inter-frequency measurements	458

8.11.5	Channel occupancy measurements	459

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	459

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	459

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	459

8.12.1	Introduction	459

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	459

8.12.2.1	Introduction	459

8.12.2.2	Measurements of a secondary component carrier	459

8.12.2.3	Measurements of a secondary component carrier with active SCell	459

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	460

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	460

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	462

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	465

8.12.3.1	Introduction	465

8.12.3.2	Measurements of a secondary component carrier	465

8.12.3.3	Measurements of a secondary component carrier with active SCell	465

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	465

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	465

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	467

8.13	Measurements for UE Category M1	469

8.13.1	Introduction	469

8.13.2	Requirements for UE category M1 with CE mode A	469

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	470

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	470

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	474

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	476

8.13.2.2	Void	480

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	480

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	480

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	483

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	485

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	486

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	486

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	488

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	490

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	491

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	491

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	492

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	493

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	493

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	494

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	495

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	495

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	496

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	500

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	502

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	506

8.13.3	Requirements for UE category M1 with CE mode B	506

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	506

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	506

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	511

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	513

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	519

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	520

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	520

8.13.3.2	Void	521

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	521

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	521

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	524

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	526

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	527

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	527

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	528

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	528

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	529

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	529

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	529

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	534

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	536

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	540

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	541

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	541

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	543

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	545

8.14	Measurements for UE category NB1	546

8.14.1	Introduction	546

8.14.2	NB-IoT intra frequency measurements under normal coverage	546

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	546

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	546

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	546

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	546

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	546

8.15	Void	547

8.16	Measurements for UE Category M2	547

8.16.1	Introduction	547

8.16.2	Requirements for UE category M2 with CE mode A	547

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	547

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	548

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	548

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	549

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	549

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	549

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	550

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	550

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	552

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	555

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	555

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	555

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	558

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	560

8.16.3	Requirements for UE category M2 with CE mode B	561

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	561

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	561

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	563

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	566

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	566

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	566

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	569

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	571

8.17	Measurements for E-UTRA – NR Dual Connectivity	572

8.17.1	Introduction	572

8.17.1.1	Measurement Gap Sharing	572

8.17.1A	Intrafrequency Measurements	572

8.17.2	SFTD Measurements	573

8.17.2.1	Introduction	573

8.17.2.2	SFTD Measurement requirements	573

8.17.2.3	SFTD Measurement Reporting Delay	573

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	574

8.17.3.1	Introduction	574

8.17.3.2	E-UTRAN FDD inter frequency measurements	574

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	574

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	575

8.17.3.3	E-UTRAN TDD inter frequency measurements	577

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	577

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	579

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	581

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	581

8.17.4.1.1	NR Inter-RAT cell identification	581

8.17.4.1.2	NR Inter-RAT measurement	583

8.17.4.1.3	NR Inter-RAT measurement reporting	583

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	584

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	584

8.17.5.1	Introduction	584

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	584

8.17.5.2.1	Identification of a new UTRA FDD cell	584

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	585

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	585

8.17.5.2.4	Periodic Reporting	585

8.17.5.2.5	Event Triggered Reporting	585

8.17.5.2.6	Event-triggered Periodic Reporting	586

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	586

8.17.5.3.1	Periodic Reporting	587

8.17.5.3.2	Event Triggered Reporting	587

8.17.5.3.3	Event-triggered Periodic Reporting	588

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	588

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	588

8.17.7.1	Introduction	588

8.17.7.2	Identification of a new UTRA FDD cell for SON	588

8.17.7.2.1	Requirements when no DRX is used	588

8.17.7.2.2	Requirements when DRX is used	589

8.17.7.2.3	Reporting Delay	589

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	590

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	590

8.17.9.1	Introduction	590

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	590

8.17.9.2.1	Identification of a new UTRA FDD cell	590

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	590

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	591

8.17.9.2.4	Periodic Reporting	591

8.17.9.2.5	Event Triggered Reporting	591

8.17.9.2.6	Event-triggered Periodic Reporting	591

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	592

8.17.9.3.1	Periodic Reporting	593

8.17.9.3.2	Event Triggered Reporting	593

8.17.9.3.3	Event-triggered Periodic Reporting	593

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	593

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	593

8.17.11.1	Introduction	593

8.17.11.2	Identification of a new UTRA TDD cell for SON	594

8.17.11.2.1	Requirements when no DRX is used	594

8.17.11.2.2	Requirements when DRX is used	594

8.17.11.2.3	Reporting Delay	595

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	595

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	595

8.17.13.1	Introduction	595

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	595

8.17.13.2.1	GSM carrier RSSI	595

8.17.13.2.2	BSIC verification	596

8.17.13.2.3	Enhanced BSIC verification	597

8.17.13.2.4	Periodic Reporting	598

8.17.13.2.5	Event Triggered Reporting	598

8.17.13.2.6	Event-triggered Periodic Reporting	598

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	598

8.17.13.3.1	GSM carrier RSSI	598

8.17.13.3.2	BSIC verification	599

8.17.13.3.3	Periodic Reporting	600

8.17.13.3.4	Event Triggered Reporting	600

8.17.13.3.5	Event-triggered Periodic Reporting	600

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	601

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	601

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	601

8.17.15.1.1	RSTD measurement Reporting Delay	601

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	601

8.17.15.2.1	RSTD measurement Reporting Delay	602

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	602

8.17.15.3.1	RSTD measurement Reporting Delay	602

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	602

8.17.15.4.1	RSTD measurement Reporting Delay	603

8.18	Measurements for non-BL/CE UE	603

8.18.1	Introduction	603

8.18.2	Requirements for non-BL/CE UE with CE Mode B	603

8.18.2.1	E-UTRAN intra frequency measurements	603

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	603

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	604

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	604

8.19	Measurements for NR – E-UTRA Dual Connectivity	605

8.19.1	Introduction	605

8.19.2	Intra-frequency Measurements	605

8.19.3	Inter-frequency Measurements	606

8.19.4	Void	606

8.19.5	Intra-frequency E-CID Measurements	606

9	Measurements performance requirements for UE	606

9.1	E-UTRAN measurements	606

9.1.1	Introduction	606

9.1.2	Intra-frequency RSRP Accuracy Requirements	607

9.1.2.1	Absolute RSRP Accuracy	607

9.1.2.2	Relative Accuracy of RSRP	607

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	608

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	609

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	610

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	611

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	612

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	613

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	614

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	614

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	614

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	615

9.1.2B.1	Introduction	615

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	615

9.1.3	Inter-frequency RSRP Accuracy Requirements	616

9.1.3.1	Absolute RSRP Accuracy	616

9.1.3.2	Relative Accuracy of RSRP	617

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	618

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	618

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	619

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	619

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	620

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	621

9.1.3B.1	Introduction	621

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	621

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	621

9.1.4	RSRP Measurement Report Mapping	622

9.1.5	Intra-frequency RSRQ Accuracy Requirements	622

9.1.5.1	Absolute RSRQ Accuracy	622

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	623

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	624

9.1.5.4	Absolute WB-RSRQ Accuracy	625

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	626

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	627

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	627

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	627

9.1.5B.1	Introduction	627

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	627

9.1.6	Inter-frequency RSRQ Accuracy Requirements	628

9.1.6.1	Absolute RSRQ Accuracy	628

9.1.6.2	Relative Accuracy of RSRQ	629

9.1.6.3	Absolute WB-RSRQ Accuracy	630

9.1.6.4	Relative WB-RSRQ Accuracy	630

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	631

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	632

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	632

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	632

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	633

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	634

9.1.6B.1	Introduction	634

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	634

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	634

9.1.7	RSRQ Measurement Report Mapping	635

9.1.8	Power Headroom	635

9.1.8.1	Period	636

9.1.8.2	Reporting Delay	636

9.1.8.3	Void	636

9.1.8.4	Report Mapping	636

9.1.9	UE Rx – Tx time difference	636

9.1.9.1	Measurement Requirement	636

9.1.9.2	Measurement Report mapping	637

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	638

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	639

9.1.10	Reference Signal Time Difference (RSTD)	640

9.1.10.1	Intra-Frequency Accuracy Requirement	640

9.1.10.2	Inter-Frequency Accuracy Requirement	641

9.1.10.3	RSTD Measurement Report Mapping	643

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	643

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	644

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	645

9.1.11	Carrier aggregation measurement accuracy	646

9.1.11.1	Primary component carrier accuracy requirement	647

9.1.11.2	Secondary component carrier accuracy requirement	647

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	647

9.1.11.4	Secondary component carrier relative accuracy requirement	647

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	647

9.1.13	Measurement accuracy for UE category 0	648

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	648

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	648

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	649

9.1.14	Accuracy requirements for Discovery Signal Measurements	650

9.1.14.1	Introduction	650

9.1.14.2	RSRP measurements in discovery signal occasions	650

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	650

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	650

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	650

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	651

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	652

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	652

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	652

9.1.14.3.3	CSI-RSRP measurement report mapping	653

9.1.14.4	RSRQ measurements in discovery signal occasions	653

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	653

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	654

9.1.15.1.1	Primary component carrier accuracy requirement	654

9.1.15.1.2	Secondary component carrier accuracy requirement	654

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	654

9.1.15.1.4	Secondary component carrier relative accuracy requirement	654

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	654

9.1.15.2.1	Primary component carrier accuracy requirement	654

9.1.15.2.2	Secondary component carrier accuracy requirement	654

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	654

9.1.15.2.4	Secondary component carrier relative accuracy requirement	654

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	655

9.1.17	RS-SINR Measurements	655

9.1.17.1	Measurement Report Mapping	655

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	656

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	656

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	656

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	656

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	657

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	658

9.1.18.1	Introduction	658

9.1.18.2	RSRP measurements	658

9.1.18.2.1	RSRP measurement report mapping	658

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	658

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	659

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	659

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	660

9.1.18.3	RSRQ measurements	660

9.1.18.3.1	RSRQ measurement report mapping	660

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	660

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	661

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	661

9.1.18.4	CSI-RSRP measurements	662

9.1.18.4.1	CSI-RSRP measurement report mapping	662

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	662

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	662

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	663

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	663

9.1.18.5	RSSI measurements	664

9.1.18.5.1	RSSI measurement report mapping	664

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	664

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	665

9.1.18.6	Channel occupancy measurements	665

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	665

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	665

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	666

9.1.19.1	Introduction	666

9.1.19.2	Accuracy requirements for measurements on SCC	666

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	666

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	666

9.1.20	SFN and Subframe Time Difference (SSTD)	667

9.1.20.1	SSTD Accuracy Requirement	667

9.1.20.2	SSTD Measurement Report Mapping	667

9.1.21	Measurement accuracy for UE category M1	668

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	668

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	669

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	670

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	671

9.1.21.5	RSRP Measurement Report Mapping	672

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	672

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	673

9.1.21.8	RSRQ Measurement Report Mapping	674

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	674

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	675

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	676

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	677

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	679

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	679

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	680

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	681

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	683

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	683

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	684

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	685

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	688

9.1.22	Measurement accuracy for UE Category NB1	691

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	691

9.1.22.2	Void	692

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	692

9.1.22.4	Void	693

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	693

9.1.22.6	Void	694

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	694

9.1.22.8	Void	695

9.1.22.9	NRSRP Measurement Report Mapping	695

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	695

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	696

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	697

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	698

9.1.22.14	NRSRQ Measurement Report Mapping	699

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	700

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	700

9.1.23	Power Headroom for UE Category NB1	701

9.1.23.1	Period	701

9.1.23.2	Reporting Delay	701

9.1.23.3	Report Mapping for UE Category NB1	701

9.1.23.3.1	Void	703

9.1.23.3.2	Void	703

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	703

9.1.24	Void	704

9.1.25	Measurement accuracy for UE category M2	704

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	704

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	705

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	706

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	707

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	709

9.1.26	Measurement Accuracy for non-BL CE UE	710

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	711

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	711

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	712

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	713

9.1.26.5	RSRP Measurement Report Mapping	714

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	714

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	714

9.1.26.8	RSRQ Measurement Report Mapping	715

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	715

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	716

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	717

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	718

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	719

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	719

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	719

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	720

9.1.27	SFN and frame Timing Difference (SFTD)	721

9.1.27.1	SFTD Accuracy Requirement	721

9.2	UTRAN FDD Measurements	723

9.2.1	UTRAN FDD CPICH RSCP	723

9.2.2	Void	724

9.2.3	UTRAN FDD CPICH Ec/No	724

9.3	UTRAN TDD Measurements	724

9.3.1	UTRAN TDD P-CCPCH RSCP	725

9.3.2	Void	725

9.3.3	Void	725

9.4	GSM Measurements	725

9.4.1	GSM carrier RSSI	725

9.5	CDMA2000 1x RTT Measurements	725

9.5.1	CDMA2000 1x RTT Pilot Strength	725

9.6	PCMAX,c	726

9.6.1	Report Mapping	726

9.6.2 	Estimation Period	726

9.6.3	Reporting Delay	726

9.7	IEEE802.11 Measurements	726

9.7.1	WLAN RSSI	726

9.7.2	WLAN RSSI Measurement Report Mapping	726

9.8	MBSFN Measurements	727

9.8.1	Introduction	727

9.8.2	MBSFN RSRP	727

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	727

9.8.2.2	MBSFN RSRP measurement report mapping	728

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	728

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	729

9.8.3	MBSFN RSRQ	729

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	729

9.8.3.2	MBSFN RSRQ measurement report mapping	730

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	730

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	731

9.8.4	MCH BLER	731

9.8.4.1	Measurement report mapping for MCH BLER	731

9.8.4.2	Measurement report mapping for MCH Block Number	732

9.9	ProSe Measurements	733

9.9.1	Introduction	733

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	733

9.9.2.1	Absolute S-RSRP Accuracy	733

9.9.2.2	Relative Accuracy of S-RSRP	734

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	735

9.9.3.1	Absolute SD-RSRP Accuracy	735

9.9.3.2	Relative Accuracy of SD-RSRP	735

9.10	V2X Measurements	736

9.10.1	Introduction	736

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	736

9.10.2.1	Absolute S-RSRP Accuracy	736

9.10.2.2	Relative Accuracy of S-RSRP	737

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	737

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	737

9.10.4	S-RSSI Measurement Accuracy Requirements	738

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	738

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	738

9.11	NR Measurements	739

9.11.1	NR SS-RSRP Measurements	739

9.11.2	NR SS-RSRQ Measurements	739

9.11.3	NR SS-SINR Measurements	740

10	Measurements Performance Requirements for E-UTRAN	740

10.1	Received Interference Power	740

10.1.1	Absolute accuracy requirement	740

10.1.2	Relative accuracy requirement	741

10.1.3	Received Interference Power measurement report mapping	741

10.2	Angle of Arrival (AOA)	741

10.2.1	Range/mapping	741

10.3	Timing Advance (TADV)	741

10.3.1	Report mapping	741

11	ProSe Requirements in Any Cell Selection state	742

11.1	Introduction	742

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	742

11.2.1	Introduction	742

11.2.2	ProSe UE transmission timing	742

11.3	Initiation/Cease of SLSS Transmissions	743

11.3.1	Introduction	743

11.3.2	Requirements	743

11.4	Measurements for ProSe in Any Cell Selection State	743

11.4.1	Introduction	743

11.4.2	Requirements	743

11.4.2.1	E-UTRA FDD	743

11.4.2.2	E-UTRA TDD	744

11.5	Selection / Reselection of ProSe Synchronization Reference	744

11.5.1	Introduction	744

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	744

11.5.2.1	Introduction	744

11.5.2.2	Requirements	744

11.6	Void	745

11.7	Selection / Reselection of ProSe relay UE	745

11.7.1	Introduction	745

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	745

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	745

12.1	Introduction	745

12.2	Transmit Timing	746

12.2.1	GNSS as timing reference	746

12.3	Interruption	746

12.4	Reliability of GNSS signal	746

13	V2X Requirements	746

13.1	Introduction	746

13.2	UE Transmit Timing	747

13.2.1	Introduction	747

13.2.2	GNSS as synchronization reference source	747

13.2.3	Serving cell/PCell as synchronization reference source	747

13.2.4	SyncRef UE as synchronization reference source	747

13.3	Initiation/Cease of SLSS Transmissions	747

13.3.1	Introduction	747

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	747

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	748

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	748

13.4	Selection / Reselection of V2X Synchronization Reference Source	749

13.5	Autonomous Resource Selection/Reselection measurements	750

13.5.1	Introduction	750

13.5.2	PSSCH-RSRP measurements	750

13.5.3	S-RSSI measurements	750

13.6	Congestion Control measurements	750

13.7	Interruption	750

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	750

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	750

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	751

13.8	Reliability of GNSS signal	751

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	751

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	752

Annex A (normative): Test Cases	752

A.1	Purpose of annex	752

A.2	Requirement classification for statistical testing	752

A.2.1	Types of requirements in TS 36.133	752

A.2.1.1	Time and delay requirements on UE higher layer actions	752

A.2.1.2	Measurements of power levels, relative powers and time	752

A.2.1.3	Implementation requirements	753

A.2.1.4	Physical layer timing requirements	753

A.3	RRM test configurations	754

A.3.1	Reference Measurement Channels	754

A.3.1.1	PDSCH	754

A.3.1.1.1	FDD	754

A.3.1.1.2	TDD	759

A.3.1.1.3	FDD for UE category 0	762

A.3.1.1.4	HD-FDD for UE category 0	763

A.3.1.1.5	TDD for UE category 0	764

A.3.1.1.6	Frame Structure 3	765

A.3.1.2	PCFICH/PDCCH/PHICH	766

A.3.1.2.1	FDD	766

A.3.1.2.2	TDD	766

A.3.1.2.3	HD-FDD for UE category 0	767

A.3.1.2.4	FS 3	767

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	767

A.3.1.3.1	FDD in CEModeA	768

A.3.1.3.2	HD-FDD in CEModeA	768

A.3.1.3.3	TDD in CEModeA	769

A.3.1.3.4	FDD in CEModeB	769

A.3.1.3.5	HD-FDD in CEModeB	770

A.3.1.3.6	TDD in CEModeB	770

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	771

A.3.1.4.1	FDD in CEModeA	771

A.3.1.4.2	HD-FDD in CEModeA	772

A.3.1.4.3	TDD in CEModeA	773

A.3.1.4.4	FDD in CEModeB	774

A.3.1.4.5	 HD-FDD in CEModeB	775

A.3.1.4.6	TDD in CEModeB	775

A.3.1.5	NPDSCH Reference Channel for UE category NB1	776

A.3.1.5.1	HD-FDD in-band operation	776

A.3.1.5.2	Void	777

A.3.1.5.3	HD-FDD standalone operation	777

A.3.1.5.4	Void	778

A.3.1.5.5	HD-FDD guard band operation	778

A.3.1.5.6	Void	779

A.3.1.5.7	TDD in-band operation	779

A.3.1.5.8	TDD standalone operation	779

A.3.1.5.9	TDD guard band operation	780

A.3.1.6	NPDCCH Reference Channel for UE category NB1	780

A.3.1.6.1	In-band operation	780

A.3.1.6.2	Void	781

A.3.1.6.3	Standalone operation	781

A.3.1.6.4	Void	781

A.3.1.6.5	Guard band operation	781

A.3.1.6.6	Void	782

A.3.2	OFDMA Channel Noise Generator (OCNG)	782

A.3.2.1	OCNG Patterns for FDD	782

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	783

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	783

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	784

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	785

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	785

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	786

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	786

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	786

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	787

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	788

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	788

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	789

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	789

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	790

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	790

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	791

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	791

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	792

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	792

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	793

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	793

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	794

A.3.2.2	OCNG Patterns for TDD	794

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	795

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	795

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	796

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	796

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	797

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	798

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	799

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	800

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	800

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	801

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	801

A.3.2.3	OCNG Patterns for Narrowband IoT	802

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	803

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	804

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	804

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	805

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	806

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	807

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	808

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	809

A.3.2.4	OCNG Patterns for V2X sidelink	810

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	810

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	811

A.3.3	Reference DRX Configurations	811

A.3.4	ABS Transmission Configurations	812

A.3.4.1	Non-MBSFN ABS Transmission Configurations	812

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	812

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	812

A.3.4.2	MBSFN ABS Transmission Configurations	813

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	813

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	814

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	814

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	814

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	815

A.3.6.1	Introduction	815

A.3.7	Test Cases with Different Channel Bandwidths	815

A.3.7.1	Introduction	815

A.3.7.2	Principle of testing	815

A.3.8	Antenna Configuration	815

A.3.8.1	Antenna connection for 4 Rx capable UEs	815

A.3.8.1.1 Introduction	815

A.3.8.1.2 Principle of testing	815

A.3.8.1.2.1 Single carrier tests	815

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	816

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	817

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	817

A.3.8.2	Antenna connection for 8 Rx capable UEs	817

A.3.8.2.1	Introduction	817

A.3.8.2.2	Principle of testing	817

A.3.8.2.2.1	Single carrier tests	817

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	818

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	818

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	818

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	818

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	818

A.3.9.1	Introduction	818

A.3.9.2	Principle of testing	818

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	819

A.3.10.1	Introduction	819

A.3.10.2	Principle of testing	819

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	819

A.3.11.1	Introduction	819

A.3.11.2	Principle of Testing	819

A.3.12	Proximity-based Services	819

A.3.12.1	Introduction	819

A.3.12.2	Reference DRX configurations for ProSe tests	819

A.3.12.3	Test Cases with Different Channel Bandwidths	820

A.3.12.3.1	Introduction	820

A.3.12.3.2	Principle of testing	820

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	820

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	827

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	830

A.3.12.6.1	FDD	830

A.3.12.7	Reference measurement channels for ProSe Direct Communication	830

A.3.12.7.1	FDD	830

A.3.12.8	ProSe Receive Traffic Generator	831

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	831

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	831

A.3.13	Time Offset between Cells	832

A.3.13.1	Introduction	832

A.3.13.2	Definition	832

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	832

A.3.14.1	Introduction	832

A.3.14.2	Principle of testing	832

A.3.15	Dual connectivity test cases with different combination of duplex mode	832

A.3.15.1	Introduction	832

A.3.15.2	Principle of testing	832

A.3.16	Reference PRACH Configurations	833

A.3.17	Listen before talk model	833

A.3.17.1	Introduction	833

A.3.17.2	Definition	833

A.3.18	Reference NPRACH Configurations	834

A.3.19	Dual connectivity test cases with different bandwidth combinations	835

A.3.19.1	Introduction	835

A.3.19.2	Principle of testing	835

A.3.20	Category M1 UE Test Cases	836

A.3.20.1	Introduction	836

A.3.20.2	Principle of Cat-M1 UE Testing	836

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	837

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	838

A.3.21.1	Introduction	838

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	838

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	839

A.3.22	Category 1bis UE Test Cases	840

A.3.22.1	Introduction	840

A.3.22.2	Principle of Category 1bis UE Testing	840

A.3.23	Category NB2 UE Test Cases	844

A.3.23.1	Introduction	844

A.3.23.2	Principle of Category NB2 UE Testing	844

A.3.24	V2X sidelink communication	847

A.3.24.1	Introduction	847

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	848

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	852

A.3.25	Category M2 UE Test Cases	853

A.3.25.1	Introduction	853

A.3.25.2	Principle of Cat-M2 UE Testing	853

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	854

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	855

A.3.26.1	Introduction	855

A.3.26.2	Principle of testing	855

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	855

A.3.27.1	Introduction	855

A.3.27.2	Principle of INACTIVE cell re-selection Testing	855

A.4	E-UTRAN RRC_IDLE state	855

A.4.2	Cell Re-Selection	855

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	855

A.4.2.1.1	Test Purpose and Environment	855

A.4.2.1.2	Test Requirements	857

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	858

A.4.2.2.1	Test Purpose and Environment	858

A.4.2.2.2	Test Requirements	859

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	860

A.4.2.3.1	Test Purpose and Environment	860

A.4.2.3.2	Test Requirements	861

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	862

A.4.2.4.1	Test Purpose and Environment	862

A.4.2.4.2	Test Requirements	863

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	864

A.4.2.5.1	Test Purpose and Environment	864

A.4.2.5.2	Test Requirements	865

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	866

A.4.2.6.1	Test Purpose and Environment	866

A.4.2.6.2	Test Requirements	867

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	868

A.4.2.7.1	Test Purpose and Environment	868

A.4.2.7.2	Test Requirements	869

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	870

A.4.2.8.1	Test Purpose and Environment	870

A.4.2.8.2	Test Requirements	871

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	872

A.4.2.9.1	Test Purpose and Environment	872

A.4.2.9.2	Test Requirements	872

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	872

A.4.2.10.1	Test Purpose and Environment	872

A.4.2.10.2	Test Requirements	876

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	876

A.4.2.11.1	Test Purpose and Environment	876

A.4.2.11.2	Test Requirements	880

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	880

A.4.2.12.1	Test Purpose and Environment	880

A.4.2.12.2	Test Requirements	882

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	883

A.4.2.13.1	Test Purpose and Environment	883

A.4.2.13.2	Test Requirements	884

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	885

A.4.2.14.1	Test Purpose and Environment	885

A.4.2.14.2	Test Requirements	886

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	887

A.4.2.15.1	Test Purpose and Environment	887

A.4.2.15.2	Test Requirements	888

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	889

A.4.2.16.1	Test Purpose and Environment	889

A.4.2.16.2	Test Requirements	890

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	891

A.4.2.17.1	Test Purpose and Environment	891

A.4.2.17.2	Test Requirements	892

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	893

A.4.2.18.1	Test Purpose and Environment	893

A.4.2.18.2	Test Requirements	895

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	896

A.4.2.19.1	Test Purpose and Environment	896

A.4.2.19.2	Test Requirements	898

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	899

A.4.2.20.1	Test Purpose and Environment	899

A.4.2.20.2	Test Requirements	900

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	901

A.4.2.21.1	Test Purpose and Environment	901

A.4.2.21.2	Test Requirements	902

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	903

A.4.2.22.1	Test Purpose and Environment	903

A.4.2.22.2	Test Requirements	904

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	905

A.4.2.23.1	Test Purpose and Environment	905

A.4.2.23.2	Test Requirements	906

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	907

A.4.2.24.1	Test Purpose and Environment	907

A.4.2.24.2	Test Requirements	909

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	910

A.4.2.25.1	Test Purpose and Environment	910

A.4.2.25.2	Test Requirements	911

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	912

A.4.2.26.1	Test Purpose and Environment	912

A.4.2.26.2	Test Requirements	913

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	914

A.4.2.27.1	Test Purpose and Environment	914

A.4.2.27.2	Test Requirements	915

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	916

A.4.2.28.1	Test Purpose and Environment	916

A.4.2.28.2	Test Requirements	917

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	918

A.4.2.29.1	Test Purpose and Environment	918

A.4.2.29.2	Test Requirements	919

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	920

A.4.2.30.1	Test Purpose and Environment	920

A.4.2.30.2	Test Requirements	921

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	922

A.4.2.31.1	Test Purpose and Environment	922

A.4.2.31.2	Test Requirements	923

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	924

A.4.2.32.1	Test Purpose and Environment	924

A.4.2.32.2	Test Requirements	925

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	926

A.4.2.33.1	Test Purpose and Environment	926

A.4.2.33.2	Test Requirements	927

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	928

A.4.2.34.1	Test Purpose and Environment	928

A.4.2.34.2	Test Requirements	929

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	930

A.4.2.35.1	Test Purpose and Environment	930

A.4.2.35.2	Test Requirements	932

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	933

A.4.2.36.1	Test Purpose and Environment	933

A.4.2.36.2	Test Requirements	935

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	936

A.4.2.37.1	Test Purpose and Environment	936

A.4.2.37.2	Test Requirements	938

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	939

A.4.2.38.1	Test Purpose and Environment	939

A.4.2.38.2	Test Requirements	941

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	942

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	942

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	942

A.4.3.1.1.1	Test Purpose and Environment	942

A.4.3.1.1.2	Test Requirements	944

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	944

A.4.3.1.2.1	Test Purpose and Environment	944

A.4.3.1.2.2	Test Requirements	947

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	947

A.4.3.1.3.1	Test Purpose and Environment	947

A.4.3.1.3.2	Test Requirements	950

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	950

A.4.3.1.4.1	Test Purpose and Environment	950

A.4.3.1.4.2	Test Requirements	951

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	951

A.4.3.1.5.1	Test Purpose and Environment	951

A.4.3.1.5.2	Test Requirements	954

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	955

A.4.3.2.1	Test Purpose and Environment	955

A.4.3.2.1.1	Void	955

A.4.3.2.1.2	1.28Mcps TDD option	955

A.4.3.2.1.3	Void	957

A.4.3.2.2	Test Requirements	957

A.4.3.2.2.1	1.28Mcps TDD option	957

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	957

A.4.3.2A.1	Test Purpose and Environment	957

A.4.3.2A.2	Test Requirements	961

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	961

A.4.3.3.1	Test Purpose and Environment	961

A.4.3.3.2	Test Requirements	964

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	964

A.4.3.3A.1	Test Purpose and Environment	964

A.4.3.3A.2	Test Requirements	969

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	970

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	970

A.4.3.4.1.1	Test Purpose and Environment	970

A.4.3.4.1.2	Test Requirements	972

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	972

A.4.3.4.2.1	Test Purpose and Environment	972

A.4.3.4.2.2	Test Requirements	974

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	974

A.4.3.4.3.1	Test Purpose and Environment	974

A.4.3.4.3.2	Test Requirements	977

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	977

A.4.3.4.4.1	Test Purpose and Environment	977

A.4.3.4.4.2	Test Requirements	981

A.4.4	E-UTRAN to GSM Cell Re-Selection	981

A.4.4.1	E-UTRAN FDD – GSM:	981

A.4.4.1.1	Test Purpose and Environment	981

A.4.4.1.2	Test Requirements	983

A.4.4.2	E-UTRAN TDD – GSM:	983

A.4.4.2.1	Test Purpose and Environment	983

A.4.4.2.2	Test Requirements	985

A.4.5	E-UTRAN to HRPD Cell Re-Selection	986

A.4.5.1	E-UTRAN FDD – HRPD	986

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	986

A.4.5.1.1.1	Test Purpose and Environment	986

A.4.5.1.1.2	Test Requirements	988

A.4.5.2	E-UTRAN TDD – HRPD	988

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	988

A.4.5.2.1.1	Test Purpose and Environment	988

A.4.5.2.1.2	Test Requirements	991

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	991

A.4.6.1	E-UTRAN FDD – cdma2000 1X	991

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	991

A.4.6.1.1.1	Test Purpose and Environment	991

A.4.6.1.1.2	Test Requirements	994

A.4.6.2	E-UTRAN TDD – cdma2000 1X	994

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	994

A.4.6.2.1.1	Test Purpose and Environment	994

A.4.6.2.1.2	Test Requirements	997

A.4.7	Idle State Positioning Measurement for UE category NB1	997

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	997

A.4.7.1.1	Test Purpose and Environment	997

A.4.7.1.2	Test Requirements	1002

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1003

A.4.7.2.1	Test Purpose and Environment	1003

A.4.7.4.2	Test Requirements	1007

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1008

A.4.7.3.1	Test Purpose and Environment	1008

A.4.7.3.2	Test Requirements	1012

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1013

A.4.7.4.1	Test Purpose and Environment	1013

A.4.7.4.2	Test Requirements	1017

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1018

A.5.1	E-UTRAN Handover	1018

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1018

A.5.1.1.1	Test Purpose and Environment	1018

A.5.1.1.2	Test Requirements	1020

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1020

A.5.1.2.1	Test Purpose and Environment	1020

A.5.1.2.2	Test Requirements	1022

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1022

A.5.1.3.1	Test Purpose and Environment	1022

A.5.1.3.2	Test Requirements	1024

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1024

A.5.1.4.1	Test Purpose and Environment	1024

A.5.1.4.2	Test Requirements	1026

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1026

A.5.1.5.1	Test Purpose and Environment	1026

A.5.1.5.2	Test Requirements	1028

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1028

A.5.1.6.1	Test Purpose and Environment	1028

A.5.1.6.2	Test Requirements	1030

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1030

A.5.1.7.1	Test Purpose and Environment	1030

A.5.1.7.2	Test Requirements	1033

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1033

A.5.1.8.1	Test Purpose and Environment	1033

A.5.1.8.2	Test Requirements	1036

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1036

A.5.1.9.1	Test Purpose and Environment	1036

A.5.1.9.2	Test Requirements	1037

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1037

A.5.1.10.1	Test Purpose and Environment	1037

A.5.1.10.2	Test Requirements	1039

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1039

A.5.1.11.1	Test Purpose and Environment	1039

A.5.1.11.2	Test Requirements	1041

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1041

A.5.1.12.1	Test Purpose and Environment	1041

A.5.1.12.2	Test Requirements	1043

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1043

A.5.1.13.1	Test Purpose and Environment	1043

A.5.1.13.2	Test Requirements	1045

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1046

A.5.1.14.1	Test Purpose and Environment	1046

A.5.1.14.2	Test Requirements	1047

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1048

A.5.1.15.1	Test Purpose and Environment	1048

A.5.1.15.2	Test Requirements	1049

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1050

A.5.1.16.1	Test Purpose and Environment	1050

A.5.1.16.2	Test Requirements	1051

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1052

A.5.1.17.1	Test Purpose and Environment	1052

A.5.1.17.2	Test Requirements	1053

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1054

A.5.1.18.1	Test Purpose and Environment	1054

A.5.1.18.2	Test Requirements	1055

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1056

A.5.1.19.1	Test Purpose and Environment	1056

A.5.1.19.2	Test Requirements	1057

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1058

A.5.1.20.1	Test Purpose and Environment	1058

A.5.1.20.2	Test Requirements	1059

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1060

A.5.1.21.1	Test Purpose and Environment	1060

A.5.1.21.2	Test Requirements	1061

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1061

A.5.1.22.1	Test Purpose and Environment	1061

A.5.1.22.2	Test Requirements	1063

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1063

A.5.1.23.1	Test Purpose and Environment	1063

A.5.1.23.2	Test Requirements	1065

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1065

A.5.1.24.1	Test Purpose and Environment	1065

A.5.1.24.2	Test Requirements	1067

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1067

A.5.1.25.1	Test Purpose and Environment	1067

A.5.1.25.2	Test Requirements	1069

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1069

A.5.1.26.1	Test Purpose and Environment	1069

A.5.1.26.2	Test Requirements	1071

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1072

A.5.1.27.1	Test Purpose and Environment	1072

A.5.1.27.2	Test Requirements	1073

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1074

A.5.1.28.1	Test Purpose and Environment	1074

A.5.1.28.2	Test Requirements	1075

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1076

A.5.1.29.1	Test Purpose and Environment	1076

A.5.1.29.2	Test Requirements	1077

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1078

A.5.1.30.1	Test Purpose and Environment	1078

A.5.1.30.2	Test Requirements	1079

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1080

A.5.1.31.1	Test Purpose and Environment	1080

A.5.1.31.2	Test Requirements	1081

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1082

A.5.1.32.1	Test Purpose and Environment	1082

A.5.1.32.2	Test Requirements	1083

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1084

A.5.1.33.1	Test Purpose and Environment	1084

A.5.1.13.2	Test Requirements	1085

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1086

A.5.1.34.1	Test Purpose and Environment	1086

A.5.1.34.2	Test Requirements	1087

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1088

A.5.1.35.1	Test Purpose and Environment	1088

A.5.1.35.2	Test Requirements	1089

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1090

A.5.1.36.1	Test Purpose and Environment	1090

A.5.1.36.2	Test Requirements	1091

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1092

A.5.1.37.1	Test Purpose and Environment	1092

A.5.1.37.2	Test Requirements	1093

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1094

A.5.1.38.1	Test Purpose and Environment	1094

A.5.1.38.2	Test Requirements	1095

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1096

A.5.1.39.1	Test Purpose and Environment	1096

A.5.1.39.2	Test Requirements	1097

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1098

A.5.1.40.1	Test Purpose and Environment	1098

A.5.1.40.2	Test Requirements	1099

A.5.2	E-UTRAN Handover to other RATs	1100

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1100

A.5.2.1.1	Test Purpose and Environment	1100

A.5.2.1.2	Test Requirements	1102

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1103

A.5.2.2.1	Test Purpose and Environment	1103

A.5.2.2.2	Test Requirements	1106

A.5.2.3 E-UTRAN FDD- GSM Handover	1106

A.5.2.3.1	Test Purpose and Environment	1106

A.5.2.3.2	Test Requirements	1107

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1108

A.5.2.4.1	Test Purpose and Environment	1108

A.5.2.4.1.1	Void	1108

A.5.2.4.1.2	1.28 Mcps TDD option	1108

A.5.2.4.1.3	Void	1110

A.5.2.4.2	Test Requirements	1110

A.5.2.4.2.1	Void	1110

A.5.2.4.2.2	1.28 Mcps TDD option	1110

A.5.2.4.2.3	Void	1110

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1110

A.5.2.5.1	Test Purpose and Environment	1110

A.5.2.5.1.3	Void	1113

A.5.2.5.2	Test Requirements	1113

A.5.2.5.2.1	Void	1113

A.5.2.5.2.2	1.28 Mcps TDD option	1113

A.5.2.5.2.3	Void	1113

A.5.2.6	E-UTRAN TDD - GSM Handover	1113

A.5.2.6.1	Test Purpose and Environment	1113

A.5.2.6.2	Test Requirements	1115

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1116

A.5.2.7.1	Test Purpose and Environment	1116

A.5.2.7.2	Test Requirements	1118

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1118

A.5.2.8.1	Test Purpose and Environment	1118

A.5.2.8.2	Test Requirements	1119

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1120

A.5.2.9.1	Test Purpose and Environment	1120

A.5.2.9.2	Test Requirements	1121

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1122

A.5.2.10.1	Test Purpose and Environment	1122

A.5.2.10.2	Test Requirements	1124

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1124

A.5.2.10A.1	Test Purpose and Environment	1124

A.5.2.10A.2	Test Requirements	1127

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1127

A.5.2.10B.1	Test Purpose and Environment	1127

A.5.2.10B.2	Test Requirements	1130

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1130

A.5.2.11.1	Test Purpose and Environment	1130

A.5.2.11.2	Test Requirements	1130

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1131

A.5.3.1	E-UTRAN FDD – HRPD Handover	1131

A.5.3.1.1	Test Purpose and Environment	1131

A.5.3.1.2	Test Requirements	1133

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1133

A.5.3.2.1	Test Purpose and Environment	1133

A.5.3.2.2	Test Requirements	1136

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1136

A.5.3.3.1	Test Purpose and Environment	1136

A.5.3.3.2	Test Requirements	1138

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1139

A.5.3.4.1	Test Purpose and Environment	1139

A.5.3.4.2	Test Requirements	1140

A.5.3.5	E-UTRAN TDD – HRPD Handover	1141

A.5.3.5.1	Test Purpose and Environment	1141

A.5.3.5.2	Test Requirements	1143

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1144

A.5.3.6.1	Test Purpose and Environment	1144

A.5.3.6.2	Test Requirements	1147

A.6	RRC Connection Control	1147

A.6.1	RRC Re-establishment	1147

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1147

A.6.1.1.1	Test Purpose and Environment	1147

A.6.1.1.2	Test Requirements	1148

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1149

A.6.1.2.1	Test Purpose and Environment	1149

A.6.1.2.2	Test Requirements	1150

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1151

A.6.1.3.1	Test Purpose and Environment	1151

A.6.1.3.2	Test Requirements	1152

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1153

A.6.1.4.1	Test Purpose and Environment	1153

A.6.1.4.2	Test Requirements	1154

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1155

A.6.1.5.1	Test Purpose and Environment	1155

A.6.1.5.2	Test Requirements	1155

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1155

A.6.1.6.1	Test Purpose and Environment	1155

A.6.1.6.2	Test Requirements	1157

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1158

A.6.1.7.1	Test Purpose and Environment	1158

A.6.1.7.2	Test Requirements	1159

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1160

A.6.1.8.1	Test Purpose and Environment	1160

A.6.1.8.2	Test Requirements	1161

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1162

A.6.1.9.1	Test Purpose and Environment	1162

A.6.1.9.2	Test Requirements	1163

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1164

A.6.1.10.1	Test Purpose and Environment	1164

A.6.1.10.2	Test Requirements	1165

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1166

A.6.1.11.1	Test Purpose and Environment	1166

A.6.1.11.2	Test Requirements	1167

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1168

A.6.1.12.1	Test Purpose and Environment	1168

A.6.1.12.2	Test Requirements	1169

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1170

A.6.1.13.1	Test Purpose and Environment	1170

A.6.1.13.2	Test Requirements	1171

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1172

A.6.1.14.1	Test Purpose and Environment	1172

A.6.1.14.2	Test Requirements	1173

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1174

A.6.1.15.1	Test Purpose and Environment	1174

A.6.1.15.2	Test Requirements	1176

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1177

A.6.1.16.1	Test Purpose and Environment	1177

A.6.1.16.2	Test Requirements	1179

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1180

A.6.1.17.1	Test Purpose and Environment	1180

A.6.1.17.2	Test Requirements	1181

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1182

A.6.1.18.1	Test Purpose and Environment	1182

A.6.1.18.2	Test Requirements	1183

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1184

A.6.1.19.1	Test Purpose and Environment	1184

A.6.1.19.2	Test Requirements	1185

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1186

A.6.1.20.1	Test Purpose and Environment	1186

A.6.1.20.2	Test Requirements	1187

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1188

A.6.1.21.1	Test Purpose and Environment	1188

A.6.1.21.2	Test Requirements	1189

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1190

A.6.1.22.1	Test Purpose and Environment	1190

A.6.1.22.2	Test Requirements	1191

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1192

A.6.1.23.1	Test Purpose and Environment	1192

A.6.1.23.2	Test Requirements	1194

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1195

A.6.1.24.1	Test Purpose and Environment	1195

A.6.1.24.2	Test Requirements	1197

A.6.2	Random Access	1198

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1198

A.6.2.1.1	Test Purpose and Environment	1198

A.6.2.1.2	Test Requirements	1199

A.6.2.1.2.1	Random Access Response Reception	1199

A.6.2.1.2.2	No Random Access Response Reception	1199

A.6.2.1.2.3	Receiving a NACK on msg3	1199

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1200

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1200

A.6.2.1.2.6	Contention Resolution Timer expiry	1200

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1200

A.6.2.2.1	Test Purpose and Environment	1200

A.6.2.2.2	Test Requirements	1201

A.6.2.2.2.1	Random Access Response Reception	1202

A.6.2.2.2.2	No Random Access Response Reception	1202

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1202

A.6.2.3.1	Test Purpose and Environment	1202

A.6.2.3.2	Test Requirements	1204

A.6.2.3.2.1	Random Access Response Reception	1204

A.6.2.3.2.2	No Random Access Response reception	1204

A.6.2.3.2.3	Receiving a NACK on msg3	1204

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1205

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1205

A.6.2.3.2.6	Contention Resolution Timer expiry	1205

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1205

A.6.2.4.1	Test Purpose and Environment	1205

A.6.2.4.2	Test Requirements	1207

A.6.2.4.2.1	Random Access Response Reception	1207

A.6.2.4.2.2	No Random Access Response Reception	1207

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1207

A.6.2.5.1	Test Purpose and Environment	1207

A.6.2.5.2	Test Requirements	1208

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1208

A.6.2.6.1	Test Purpose and Environment	1208

A.6.2.6.2	Test Requirements	1208

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1209

A.6.2.7.1	Test Purpose and Environment	1209

A.6.2.7.2	Test Requirements	1211

A.6.2.7.2.1	Random Access Response Reception	1211

A.6.2.7.2.2	No Random Access Response Reception	1211

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1211

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1212

A.6.2.8.1	Test Purpose and Environment	1212

A.6.2.8.2	Test Requirements	1214

A.6.2.8.2.1	Random Access Response Reception	1214

A.6.2.8.2.2	No Random Access Response Reception	1214

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1214

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1215

A.6.2.9.1	Test Purpose and Environment	1215

A.6.2.9.2	Test Requirements	1218

A.6.2.9.2.1	Random Access Response Reception	1218

A.6.2.9.2.2	No Random Access Response Reception	1219

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1219

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1220

A.6.2.10.1	Test Purpose and Environment	1220

A.6.2.10.2	Test Requirements	1222

A.6.2.10.2.1	Random Access Response Reception	1222

A.6.2.10.2.2	No Random Access Response Reception	1222

A.6.2.10.2.3	Receiving a NACK on msg3	1223

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1223

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1223

A.6.2.10.2.6	Contention Resolution Timer expiry	1223

A.6.2.10.2.7	PRACH Resource Selection	1223

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1223

A.6.2.11.1	Test Purpose and Environment	1223

A.6.2.11.2	Test Requirements	1225

A.6.2.11.2.1	Random Access Response Reception	1225

A.6.2.11.2.2	No Random Access Response Reception	1226

A.6.2.11.2.3	Receiving a NACK on msg3	1226

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1226

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1226

A.6.2.11.2.6	Contention Resolution Timer expiry	1226

A.6.2.11.2.7	PRACH Resource Selection	1227

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1227

A.6.2.12.1	Test Purpose and Environment	1227

A.6.2.12.2	Test Requirements	1229

A.6.2.12.2.1	Random Access Response Reception	1229

A.6.2.12.2.2	No Random Access Response Reception	1230

A.6.2.12.2.3	Receiving a NACK on msg3	1230

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1230

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1230

A.6.2.12.2.6	Contention Resolution Timer expiry	1230

A.6.2.12.2.7	PRACH Resource Selection	1231

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1231

A.6.2.13.1	Test Purpose and Environment	1231

A.6.2.13.2	Test Requirements	1233

A.6.2.13.2.1	Random Access Response Reception	1233

A.6.2.13.2.2	No Random Access Response Reception	1233

A.6.2.13.2.3	Receiving a NACK on msg3	1234

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1234

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1234

A.6.2.13.2.6	Contention Resolution Timer expiry	1234

A.6.2.13.2.7	PRACH Resource Selection	1234

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1234

A.6.2.14.1	Test Purpose and Environment	1234

A.6.2.14.2	Test Requirements	1236

A.6.2.14.2.1	Random Access Response Reception	1236

A.6.2.14.2.2	No Random Access Response Reception	1237

A.6.2.14.2.3	Receiving a NACK on msg3	1237

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1237

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1237

A.6.2.14.2.6	Contention Resolution Timer expiry	1237

A.6.2.14.2.7	PRACH Resource Selection	1238

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1238

A.6.2.15.1	Test Purpose and Environment	1238

A.6.2.15.2	Test Requirements	1240

A.6.2.15.2.1	Random Access Response Reception	1240

A.6.2.15.2.2	No Random Access Response Reception	1241

A.6.2.15.2.3	Receiving a NACK on msg3	1241

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1241

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1241

A.6.2.15.2.6	Contention Resolution Timer expiry	1241

A.6.2.15.2.7	PRACH Resource Selection	1242

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1242

A.6.2.16.1	Test Purpose and Environment	1242

A.6.2.16.2	Test Requirements	1245

A.6.2.16.2.1	Random Access Response Reception	1245

A.6.2.16.2.2	No Random Access Response Reception	1246

A.6.2.16.2.3	Receiving a NACK on msg3	1246

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1246

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1246

A.6.2.16.2.6	Contention Resolution Timer expiry	1246

A.6.2.16.2.7	NPRACH Resource Selection	1246

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1247

A.6.2.17.1	Test Purpose and Environment	1247

A.6.2.17.2	Test Requirements	1250

A.6.2.17.2.1	Random Access Response Reception	1250

A.6.2.17.2.2	No Random Access Response Reception	1251

A.6.2.17.2.3	Receiving a NACK on msg3	1251

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1251

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1251

A.6.2.17.2.6	Contention Resolution Timer expiry	1251

A.6.2.17.2.7	NPRACH Resource Selection	1251

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1252

A.6.2.18.1	Test Purpose and Environment	1252

A.6.2.18.2	Test Requirements	1255

A.6.2.18.2.1	Random Access Response Reception	1255

A.6.2.18.2.2	No Random Access Response Reception	1256

A.6.2.18.2.3	Receiving a NACK on msg3	1256

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1256

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1256

A.6.2.18.2.6	Contention Resolution Timer expiry	1256

A.6.2.18.2.7	NPRACH Resource Selection	1256

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1257

A.6.2.19.1	Test Purpose and Environment	1257

A.6.2.19.2	Test Requirements	1259

A.6.2.19.2.1	Random Access Response Reception	1259

A.6.2.19.2.2	No Random Access Response Reception	1260

A.6.2.19.2.3	Receiving a NACK on msg3	1260

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1260

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1260

A.6.2.19.2.6	Contention Resolution Timer expiry	1260

A.6.2.19.2.7	NPRACH Resource Selection	1260

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1261

A.6.2.20.1	Test Purpose and Environment	1261

A.6.2.20.2	Test Requirements	1263

A.6.2.20.2.1	Random Access Response Reception	1263

A.6.2.20.2.2	No Random Access Response Reception	1264

A.6.2.20.2.3	Receiving a NACK on msg3	1264

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1264

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1264

A.6.2.20.2.6	Contention Resolution Timer expiry	1264

A.6.2.20.2.7	NPRACH Resource Selection	1264

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1265

A.6.2.21.1	Test Purpose and Environment	1265

A.6.2.21.2	Test Requirements	1267

A.6.2.21.2.1	Random Access Response Reception	1267

A.6.2.21.2.2	No Random Access Response Reception	1268

A.6.2.21.2.3	Receiving a NACK on msg3	1268

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1268

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1268

A.6.2.21.2.6	Contention Resolution Timer expiry	1268

A.6.2.21.2.7	NPRACH Resource Selection	1268

A.6.3	RRC Connection Release with Redirection	1269

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1269

A.6.3.1.1	Test Purpose and Environment	1269

A.6.3.1.2	Test Requirements	1271

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1271

A.6.3.2.1	Test Purpose and Environment	1271

A.6.3.2.2	Test Requirements	1273

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1273

A.6.3.3.1	Test Purpose and Environment	1273

A.6.3.3.2	Test Requirements	1274

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1275

A.6.3.4.1	Test Purpose and Environment	1275

A.6.3.4.2	Test Requirements	1276

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1277

A.6.3.5.1	Test Purpose and Environment	1277

A.6.3.5.2	Test Requirements	1279

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1279

A.6.3.6.1	Test Purpose and Environment	1279

A. 6.3.6.2	Test Requirements	1282

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1282

A.6.3.7.1	Test Purpose and Environment	1282

A.6.3.7.2	Test Requirements	1285

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1285

A.6.3.8.1	Test Purpose and Environment	1285

A.6.3.8.2	Test Requirements	1288

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1288

A.6.3.9.1	Test Purpose and Environment	1288

A.6.3.9.2	Test Requirements	1290

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1290

A.6.3.10.1	Test Purpose and Environment	1290

A.6.3.10.2	Test Requirements	1292

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1292

A.6.3.11.1	Test Purpose and Environment	1292

A.6.3.11.2	Test Requirements	1294

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1294

A.6.3.12.1	Test Purpose and Environment	1294

A.6.3.12.2	Test Requirements	1297

A.7	Timing and Signalling Characteristics	1288

A.7.1	UE Transmit Timing	1288

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1288

A.7.1.1.1	Test Purpose and Environment	1288

A.7.1.1.2	Test Requirements	1289

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1290

A.7.1.2.1	Test Purpose and Environment	1290

A.7.1.2.2	Test Requirements	1292

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1293

A.7.1.3.1	Test Purpose and Environment	1293

A.7.1.3.2	Test Requirements	1295

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1296

A.7.1.4.1	Test Purpose and Environment	1296

A.7.1.4.2	Test Requirements	1297

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1298

A.7.1.4A.1	Test Purpose and Environment	1298

A.7.1.4A.2	Test Requirements	1298

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1298

A.7.1.5.1	Test Purpose and Environment	1298

A.7.1.5.2	Test Requirements	1299

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1299

A.7.1.6.1	Test Purpose and Environment	1299

A.7.1.6.2	Test Requirements	1301

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1302

A.7.1.7.1	Test Purpose and Environment	1302

A.7.1.7.2	Test Requirements	1303

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1304

A.7.1.7A.1	Test Purpose and Environment	1304

A.7.1.7A.2	Test Requirements	1304

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1304

A.7.1.7B.1	Test Purpose and Environment	1304

A.7.1.7B.2	Test Requirements	1304

A.7.1.8	Void	1305

A.7.1.8.1	Void	1305

A.7.1.8.2	Void	1305

A.7.1.9	Void	1305

A.7.1.9.1	Void	1305

A.7.1.9.2	Void	1305

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1305

A.7.1.10.1	Test Purpose and Environment	1305

A.7.1.10.2	Test Requirements	1307

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1307

A.7.1.11.1	Test Purpose and Environment	1307

A.7.1.11.2	Test Requirements	1309

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1309

A.7.1.12.1	Test Purpose and Environment	1309

A.7.1.12.2	Test Requirements	1311

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1311

A.7.1.13.1	Test Purpose and Environment	1311

A.7.1.13.2	Test Requirements	1313

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1314

A.7.1.14.1	Test Purpose and Environment	1314

A.7.1.14.2	Test Requirements	1315

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1315

A.7.1.15.1	Test Purpose and Environment	1315

A.7.1.15.2	Test Requirements	1316

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1317

A.7.1.16.1	Test Purpose and Environment	1317

A.7.1.16.2	Test Requirements	1318

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1319

A.7.1.17.1	Test Purpose and Environment	1319

A.7.1.17.2	Test Requirements	1321

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1322

A.7.1.18.1	Test Purpose and Environment	1322

A.7.1.18.2	Test Requirements	1324

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1325

A.7.1.19.1	Test Purpose and Environment	1325

A.7.1.19.2	Test Requirements	1326

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1326

A.7.1.20.1	Test Purpose and Environment	1326

A.7.1.20.2	Test Requirements	1328

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1328

A.7.1.21.1	Test Purpose and Environment	1328

A.7.1.21.2	Test Requirements	1330

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1330

A.7.1.22.1	Test Purpose and Environment	1330

A.7.1.22.2	Test Requirements	1332

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1332

A.7.1.23.1	Test Purpose and Environment	1332

A.7.1.23.2	Test Requirements	1334

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1334

A.7.1.24.1	Test Purpose and Environment	1334

A.7.1.24.2	Test Requirements	1335

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1336

A.7.1.25.1	Test Purpose and Environment	1336

A.7.1.25.2	Test Requirements	1337

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1338

A.7.1.26.1	Test Purpose and Environment	1338

A.7.1.26.2	Test Requirements	1339

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1340

A.7.1.27.1	Test Purpose and Environment	1340

A.7.1.27.2	Test Requirements	1342

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1342

A.7.1.28.1	Test Purpose and Environment	1342

A.7.1.28.2	Test Requirements	1344

A.7.2	UE Timing Advance	1345

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1345

A.7.2.1.1	Test Purpose and Environment	1345

A.7.2.1.2	Test Requirements	1346

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1347

A.7.2.2.1	Test Purpose and Environment	1347

A.7.2.2.2	Test Requirements	1349

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1349

A.7.2.3.1	Test Purpose and Environment	1349

A.7.2.3.2	Test Requirements	1349

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1349

A.7.2.4.1	Test Purpose and Environment	1349

A.7.2.4.2	Test Requirements	1352

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1352

A.7.2.5.1	Test Purpose and Environment	1352

A.7.2.5.2	Test Requirements	1354

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1354

A.7.2.5A.1	Test Purpose and Environment	1354

A.7.2.5A.2	Test Requirements	1355

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1355

A.7.2.5B.1	Test Purpose and Environment	1355

A.7.2.5B.2	Test Requirements	1355

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1355

A.7.2.6.1	Test Purpose and Environment	1355

A.7.2.6.2	Test Requirements	1358

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1358

A.7.2.7.1	Test Purpose and Environment	1358

A.7.2.7.2	Test Requirements	1360

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1360

A.7.2.8.1	Test Purpose and Environment	1360

A.7.2.8.2	Test Requirements	1362

A.7.2.9.2	Test Requirements	1364

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1364

A.7.2.10.1	Test Purpose and Environment	1364

A.7.2.10.2	Test Requirements	1365

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1365

A.7.2.11.1	Test Purpose and Environment	1365

A.7.2.11.2	Test Requirements	1367

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1367

A.7.2.12.1	Test Purpose and Environment	1367

A.7.2.12.2	Test Requirements	1369

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1370

A.7.2.13.1	Test Purpose and Environment	1370

A.7.2.13.2	Test Requirements	1371

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1372

A.7.2.14.1	Test Purpose and Environment	1372

A.7.2.14.2	Test Requirements	1374

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1374

A.7.2.15.1	Test Purpose and Environment	1374

A.7.2.15.2	Test Requirements	1376

A.7.3	Radio Link Monitoring	1376

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1376

A.7.3.1.1	Test Purpose and Environment	1376

A.7.3.1.2	Test Requirements	1380

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1380

A.7.3.2.1	Test Purpose and Environment	1380

A.7.3.2.2	Test Requirements	1384

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1384

A.7.3.3.1	Test Purpose and Environment	1384

A.7.3.3.2	Test Requirements	1388

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1388

A.7.3.4.1	Test Purpose and Environment	1388

A.7.3.4.2	Test Requirements	1392

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1392

A.7.3.5.1	Test Purpose and Environment	1392

A.7.3.5.2	Test Requirements	1395

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1396

A.7.3.6.1	Test Purpose and Environment	1396

A.7.3.6.2	Test Requirements	1399

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1399

A.7.3.7.1	Test Purpose and Environment	1399

A.7.3.7.2	Test Requirements	1402

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1402

A.7.3.8.1	Test Purpose and Environment	1402

A.7.3.8.2	Test Requirements	1405

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1405

A.7.3.9.2	Test Requirements	1408

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1408

A.7.3.10.1	Test Purpose and Environment	1408

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1412

A.7.3.11.1	Test Purpose and Environment	1412

A.7.3.11.2	Test Requirements	1417

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1417

A.7.3.12.1	Test Purpose and Environment	1417

A.7.3.12.2	Test Requirements	1422

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1422

A.7.3.13.1	Test Purpose and Environment	1422

A.7.3.13.2	Test Requirements	1425

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1425

A.7.3.14.1	Test Purpose and Environment	1425

A.7.3.14.2	Test Requirements	1428

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1428

A.7.3.15.1	Test Purpose and Environment	1428

A.7.3.15.2	Test Requirements	1432

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1432

A.7.3.16.1	Test Purpose and Environment	1432

A.7.3.16.2	Test Requirements	1437

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1437

A.7.3.17.1	Test Purpose and Environment	1437

A.7.3.17.2	Test Requirements	1441

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1441

A.7.3.18.1	Test Purpose and Environment	1441

A.7.3.18.2	Test Requirements	1445

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1445

A.7.3.19.1	Test Purpose and Environment	1445

A.7.3.19.2	Test Requirements	1450

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1450

A.7.3.20.1	Test Purpose and Environment	1450

A.7.3.20.2	Test Requirements	1455

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1455

A.7.3.21.1	Test Purpose and Environment	1455

A.7.3.21.2	Test Requirements	1460

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1460

A.7.3.22.1	Test Purpose and Environment	1460

A.7.3.22.2	Test Requirements	1465

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1465

A.7.3.23.1	Test Purpose and Environment	1465

A.7.3.23.2	Test Requirements	1466

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1466

A.7.3.24.1	Test Purpose and Environment	1466

A.7.3.24.2	Test Requirements	1467

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1467

A.7.3.25.1	Test Purpose and Environment	1467

A.7.3.25.2	Test Requirements	1468

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1468

A.7.3.26.1	Test Purpose and Environment	1468

A.7.3.26.2	Test Requirements	1471

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1471

A.7.3.27.1	Test Purpose and Environment	1471

A.7.3.27.2	Test Requirements	1474

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1474

A.7.3.28.1	Test Purpose and Environment	1474

A.7.3.28.2	Test Requirements	1477

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1477

A.7.3.29.1	Test Purpose and Environment	1477

A.7.3.29.2	Test Requirements	1480

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1480

A.7.3.30.1	Test Purpose and Environment	1480

A.7.3.30.2	Test Requirements	1483

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1483

A.7.3.31.1	Test Purpose and Environment	1483

A.7.3.31.2	Test Requirements	1486

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1486

A.7.3.32.1	Test Purpose and Environment	1486

A.7.3.32.2	Test Requirements	1489

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1489

A.7.3.33.1	Test Purpose and Environment	1489

A.7.3.33.2	Test Requirements	1492

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1492

A.7.3.34.1	Test Purpose and Environment	1492

A.7.3.34.2	Test Requirements	1495

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1495

A.7.3.35.1	Test Purpose and Environment	1495

A.7.3.35.2	Test Requirements	1498

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1498

A.7.3.36.1	Test Purpose and Environment	1498

A.7.3.36.2	Test Requirements	1501

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1501

A.7.3.37.1	Test Purpose and Environment	1501

A.7.3.37.2	Test Requirements	1504

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1504

A.7.3.38.1	Test Purpose and Environment	1504

A.7.3.38.2	Test Requirements	1507

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1507

A.7.3.39.1	Test Purpose and Environment	1507

A.7.3.39.2	Test Requirements	1510

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1511

A.7.3.40.1	Test Purpose and Environment	1511

A.7.3.40.2	Test Requirements	1513

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1514

A.7.3.41.1	Test Purpose and Environment	1514

A.7.3.41.2	Test Requirements	1517

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1517

A.7.3.42.1	Test Purpose and Environment	1517

A.7.3.42.2	Test Requirements	1520

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1520

A.7.3.43.1	Test Purpose and Environment	1520

A.7.3.43.2	Test Requirements	1523

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1523

A.7.3.44.1	Test Purpose and Environment	1523

A.7.3.44.2	Test Requirements	1526

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1527

A.7.3.45.1	Test Purpose and Environment	1527

A.7.3.45.2	Test Requirements	1529

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1530

A.7.3.46.1	Test Purpose and Environment	1530

A.7.3.46.2	Test Requirements	1533

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1533

A.7.3.47.1	Test Purpose and Environment	1533

A.7.3.47.2	Test Requirements	1536

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1536

A.7.3.48.1	Test Purpose and Environment	1536

A.7.3.48.2	Test Requirements	1539

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1539

A.7.3.49.1	Test Purpose and Environment	1539

A.7.3.49.2	Test Requirements	1542

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1542

A.7.3.50.1	Test Purpose and Environment	1542

A.7.3.50.2	Test Requirements	1545

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1545

A.7.3.51.1	Test Purpose and Environment	1545

A.7.3.51.2	Test Requirements	1548

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1548

A.7.3.52.1	Test Purpose and Environment	1548

A.7.3.52.2	Test Requirements	1551

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1551

A.7.3.53.1	Test Purpose and Environment	1551

A.7.3.53.2	Test Requirements	1554

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1554

A.7.3.54.1	Test Purpose and Environment	1554

A.7.3.54.2	Test Requirements	1557

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1557

A.7.3.55.1	Test Purpose and Environment	1557

A.7.3.55.2	Test Requirements	1560

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1560

A.7.3.56.1	Test Purpose and Environment	1560

A.7.3.56.2	Test Requirements	1563

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1563

A.7.3.57.1	Test Purpose and Environment	1563

A.7.3.57.2	Test Requirements	1566

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1566

A.7.3.58.1	Test Purpose and Environment	1566

A.7.3.58.2	Test Requirements	1569

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1569

A.7.3.59.1	Test Purpose and Environment	1569

A.7.3.59.2	Test Requirements	1572

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1572

A.7.3.60.1	Test Purpose and Environment	1572

A.7.3.60.2	Test Requirements	1576

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1576

A.7.3.61.1	Test Purpose and Environment	1576

A.7.3.61.2	Test Requirements	1580

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1580

A.7.3.62.1	Test Purpose and Environment	1580

A.7.3.62.2	Test Requirements	1584

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1584

A.7.3.63.1	Test Purpose and Environment	1584

A.7.3.63.2	Test Requirements	1588

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1588

A.7.3.64.1	Test Purpose and Environment	1588

A.7.3.64.2	Test Requirements	1591

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1592

A.7.3.65.1	Test Purpose and Environment	1592

A.7.3.65.2	Test Requirements	1595

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1596

A.7.3.66.1	Test Purpose and Environment	1596

A.7.3.66.2	Test Requirements	1598

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1599

A.7.3.67.1	Test Purpose and Environment	1599

A.7.3.67.2	Test Requirements	1602

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1603

A.7.3.68.1	Test Purpose and Environment	1603

A.7.3.68.2	Test Requirements	1605

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1605

A.7.3.69.1	Test Purpose and Environment	1605

A.7.3.69.2	Test Requirements	1607

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1607

A.7.3.70.1	Test Purpose and Environment	1607

A.7.3.70.2	Test Requirements	1609

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1609

A.7.3.71.1	Test Purpose and Environment	1609

A.7.3.71.2	Test Requirements	1612

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1612

A.7.3.72.1	Test Purpose and Environment	1612

A.7.3.72.2	Test Requirements	1614

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1614

A.7.3.73.1	Test Purpose and Environment	1614

A.7.3.73.2	Test Requirements	1616

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1616

A.7.3.74.1	Test Purpose and Environment	1616

A.7.3.74.2	Test Requirements	1619

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1619

A.7.3.75.1	Test Purpose and Environment	1619

A.7.3.75.2	Test Requirements	1621

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1622

A.7.3.76.1	Test Purpose and Environment	1622

A.7.3.76.2	Test Requirements	1625

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1625

A.7.3.77.1	Test Purpose and Environment	1625

A.7.3.77.2	Test Requirements	1628

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1628

A.7.3.78.1	Test Purpose and Environment	1628

A.7.3.78.2	Test Requirements	1631

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1631

A.7.3.79.1	Test Purpose and Environment	1631

A.7.3.79.2	Test Requirements	1634

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1634

A.7.3.80.1	Test Purpose and Environment	1634

A.7.3.80.2	Test Requirements	1637

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1637

A.7.3.81.1	Test Purpose and Environment	1637

A.7.3.81.2	Test Requirements	1640

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1640

A.7.3.82.1	Test Purpose and Environment	1640

A.7.3.82.2	Test Requirements	1643

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1643

A.7.3.83.1	Test Purpose and Environment	1643

A.7.3.83.2	Test Requirements	1644

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1645

A.7.3.84.1	Test Purpose and Environment	1645

A.7.3.84.2	Test Requirements	1647

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1647

A.7.3.85.1	Test Purpose and Environment	1647

A.7.3.85.2	Test Requirements	1648

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1649

A.7.3.86.1	Test Purpose and Environment	1649

A.7.3.86.2	Test Requirements	1651

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1651

A.7.3.87.1	Test Purpose and Environment	1651

A.7.3.87.2	Test Requirements	1653

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1654

A.7.3.88.1	Test Purpose and Environment	1654

A.7.3.88.2	Test Requirements	1658

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1658

A.7.3.89.1	Test Purpose and Environment	1658

A.7.3.89.2	Test Requirements	1662

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1662

A.7.3.90.1	Test Purpose and Environment	1662

A.7.3.90.2	Test Requirements	1666

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1666

A.7.3.91.1	Test Purpose and Environment	1666

A.7.3.91.2	Test Requirements	1670

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1670

A.7.3.92.1	Test Purpose and Environment	1670

A.7.3.92.2	Test Requirements	1673

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1674

A.7.3.93.1	Test Purpose and Environment	1674

A.7.3.93.2	Test Requirements	1677

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1678

A.7.3.94.1	Test Purpose and Environment	1678

A.7.3.94.2	Test Requirements	1680

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1681

A.7.3.95.1	Test Purpose and Environment	1681

A.7.3.95.2	Test Requirements	1684

A.7.4	Interruption for Dual Connectivity	1685

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1685

A.7.4.1.1	Test Purpose and Environment	1685

A.7.4.1.2	Test Requirements	1687

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1687

A.7.4.2.1	Test Purpose and Environment	1687

A.7.4.2.2	Test Requirements	1689

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1689

A.7.4.3.1	Test Purpose and Environment	1689

A.7.4.3.2	Test Requirements	1691

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1691

A.7.4.4.1	Test Purpose and Environment	1691

A.7.4.4.2	Test Requirements	1694

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1694

A.7.4.5.1	Test Purpose and Environment	1694

A.7.4.5.2	Test Requirements	1696

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1696

A.7.4.6.1	Test Purpose and Environment	1696

A.7.4.6.2	Test Requirements	1698

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1698

A.7.4.7.1	Test Purpose and Environment	1698

A.7.4.7.2	Test Requirements	1702

A.7.5	Proximity-based Services	1702

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1702

A.7.5.1.1	Test Purpose and Environment	1702

A.7.5.1.2	Test Requirements	1703

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1704

A.7.5.2.1	Test Purpose and Environment	1704

A.7.5.1.2	Test Requirements	1704

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1705

A.7.5.3.1	Test Purpose and Environment	1705

A.7.5.3.2	Test Requirements	1706

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1707

A.7.5.4.1	Test Purpose and Environment	1707

A.7.5.4.2	Test Requirements	1708

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1709

A.7.5.5.1	Test Purpose and Environment	1709

A.7.5.5.2	Test Requirements	1711

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1712

A.7.5.6.1	Test Purpose and Environment	1712

A.7.5.6.2	Test Requirements	1713

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1714

A.7.5.7.1	Test Purpose and Environment	1714

A.7.5.7.2	Test Requirements	1716

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1716

A.7.5.8.1	Test Purpose and Environment	1716

A.7.5.8.2	Test Requirements	1718

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1719

A.7.5.9.1	Test Purpose and Environment	1719

A.7.5.9.2	Test Requirements	1721

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1722

A.7.5.10.1	Test Purpose and Environment	1722

A.7.5.10.2	Test Requirements	1724

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1725

A.7.5.11.1	Test Purpose and Environment	1725

A.7.5.11.2	Test Requirements	1727

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1727

A.7.5.12.1	Test Purpose and Environment	1727

A.7.5.12.2	Test Requirements	1731

A.7.6	Interruption for carrier aggregation	1732

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1732

A.7.6.1.1	Test Purpose and Environment	1732

A.7.6.1.2	Test Requirements	1735

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1735

A.7.6.2.1	Test Purpose and Environment	1735

A.7.6.2.2	Test Requirements	1738

A.8	UE Measurements Procedures	1738

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1738

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1738

A.8.1.1.1	Test Purpose and Environment	1738

A.8.1.1.2	Test Requirements	1740

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1740

A.8.1.2.1	Test Purpose and Environment	1740

A.8.1.2.2	Test Requirements	1742

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1742

A.8.1.3.1	Test Purpose and Environment	1742

A.8.1.3.2	Test Requirements	1745

A.8.1.4	Void	1745

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1745

A.8.1.5.1	Test Purpose and Environment	1745

A.8.1.5.2	Test Requirements	1747

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1747

A.8.1.6.1	Test Purpose and Environment	1747

A.8.1.6.2	Test Requirements	1749

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1749

A.8.1.7.1	Test Purpose and Environment	1749

A.8.1.7.2	Test Requirements	1751

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1752

A.8.1.8.1	Test Purpose and Environment	1752

A.8.1.8.2	Test Requirements	1754

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1755

A.8.1.9.1	Test Purpose and Environment	1755

A.8.1.9.2	Test Requirements	1755

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1755

A.8.1.10.1	Test Purpose and Environment	1755

A.8.1.10.2	Test Requirements	1756

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1756

A.8.1.11.1	Test Purpose and Environment	1756

A.8.1.11.2	Test Requirements	1758

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1759

A.8.1.12.1	Test Purpose and Environment	1759

A.8.1.12.2	Test Requirements	1760

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1761

A.8.1.13.1	Test Purpose and Environment	1761

A.8.1.13.2	Test Requirements	1763

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1763

A.8.1.14.1	Test Purpose and Environment	1763

A.8.1.14.2	Test Requirements	1765

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1766

A.8.1.15.1	Test Purpose and Environment	1766

A.8.1.15.2	Test Requirements	1767

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1768

A.8.1.16.1	Test Purpose and Environment	1768

A.8.1.16.2	Test Requirements	1770

A.8.1.17	Void	1770

A.8.1.18	Void	1770

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1770

A.8.1.19.1	Test Purpose and Environment	1770

A.8.1.19.2	Test Requirements	1772

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1773

A.8.1.20.1	Test Purpose and Environment	1773

A.8.1.20.2	Test Requirements	1775

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1775

A.8.1.21.1	Test Purpose and Environment	1775

A.8.1.21.2	Test Requirements	1777

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1778

A.8.1.22.1	Test Purpose and Environment	1778

A.8.1.22.2	Test Requirements	1780

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1780

A.8.1.23.1	Test Purpose and Environment	1780

A.8.1.23.2	Test Requirements	1781

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1782

A.8.1.24.1	Test Purpose and Environment	1782

A.8.1.24.2	Test Requirements	1784

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1784

A.8.1.25.1	Test Purpose and Environment	1784

A.8.1.25.2	Test Requirements	1786

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1786

A.8.1.26.1	Test Purpose and Environment	1786

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1789

A.8.1.27.1	Test Purpose and Environment	1789

A.8.1.27.2	Test Requirements	1791

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1791

A.8.1.28.1	Test Purpose and Environment	1791

A.8.1.28.2	Test Requirements	1793

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1793

A.8.1.29.1	Test Purpose and Environment	1793

A.8.1.29.2	Test Requirements	1795

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1796

A.8.1.30.1	Test Purpose and Environment	1796

A.8.1.30.2	Test Requirements	1798

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1798

A.8.1.31.1	Test Purpose and Environment	1798

A.8.1.31.2	Test Requirements	1800

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1801

A.8.1.32.1	Test Purpose and Environment	1801

A.8.1.32.2	Test Requirements	1802

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1803

A.8.1.33.1	Test Purpose and Environment	1803

A.8.1.33.2	Test Requirements	1804

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1805

A.8.1.34.1	Test Purpose and Environment	1805

A.8.1.34.2	Test Requirements	1806

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1807

A.8.1.35.1	Test Purpose and Environment	1807

A.8.1.35.2	Test Requirements	1808

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1809

A.8.1.36.1	Test Purpose and Environment	1809

A.8.1.36.2	Test Requirements	1810

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1811

A.8.1.37.1	Test Purpose and Environment	1811

A.8.1.37.2	Test Requirements	1813

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1813

A.8.1.38.1	Test Purpose and Environment	1813

A.8.1.38.2	Test Requirements	1814

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1815

A.8.1.39.1	Test Purpose and Environment	1815

A.8.1.39.2	Test Requirements	1817

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1817

A.8.1.40.1	Test Purpose and Environment	1817

A.8.1.40.2	Test Requirements	1819

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1819

A.8.1.41.1	Test Purpose and Environment	1819

A.8.1.41.2	Test Requirement	1821

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1821

A.8.1.42.1	Test Purpose and Environment	1821

A.8.1.42.2	Test Requirement	1822

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1823

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1823

A.8.2.1.1	Test Purpose and Environment	1823

A.8.2.1.2	Test Requirements	1824

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1825

A.8.2.2.1	Test Purpose and Environment	1825

A.8.2.2.2	Test Requirements	1827

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1827

A.8.2.3.1	Test Purpose and Environment	1827

A.8.2.3.2	Test Requirements	1829

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1830

A.8.2.4.1	Test Purpose and Environment	1830

A.8.2.4.2	Test Requirements	1832

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1832

A.8.2.5.1	Test Purpose and Environment	1832

A.8.2.5.2	Test Requirements	1834

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1835

A.8.2.6.1	Test Purpose and Environment	1835

A.8.2.6.2	Test Requirements	1838

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1839

A.8.2.7.1	Test Purpose and Environment	1839

A.8.2.7.2	Test Requirements	1840

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1841

A.8.2.8.1	Test Purpose and Environment	1841

A.8.2.8.2	Test Requirements	1843

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1843

A.8.2.9.1	Test Purpose and Environment	1843

A.8.2.9.2	Test Requirements	1844

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1845

A.8.2.10.1	Test Purpose and Environment	1845

A.8.2.10.2	Test Requirements	1847

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1847

A.8.2.11.1	Test Purpose and Environment	1847

A.8.2.11.2	Test Requirements	1849

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1849

A.8.2.12.1	Test Purpose and Environment	1849

A.8.2.12.2	Test Requirements	1851

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1852

A.8.2.13.1	Test Purpose and Environment	1852

A.8.2.13.2	Test Requirements	1854

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1854

A.8.2.14.1	Test Purpose and Environment	1854

A.8.2.14.2	Test Requirement	1856

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1856

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1856

A.8.3.1.1	Test Purpose and Environment	1856

A.8.3.1.2	Test Requirements	1857

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1857

A.8.3.2.1	Test Purpose and Environment	1857

A.8.3.2.2	Test Requirements	1860

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1860

A.8.3.3.1	Test Purpose and Environment	1860

A.8.3.3.2	Test Requirements	1863

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1863

A.8.3.4.1	Test Purpose and Environment	1863

A.8.3.4.2	Test Requirements	1865

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1865

A.8.3.5.2	Test Requirements	1867

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1867

A.8.3.6.1	Test Purpose and Environment	1867

A.8.3.6.2	Test Requirements	1869

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1870

A.8.3.7.1	Test Purpose and Environment	1870

A.8.3.7.2	Test Requirements	1872

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1873

A.8.3.8.1	Test Purpose and Environment	1873

A.8.3.8.2	Test Requirements	1876

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1876

A.8.3.9.1	Test Purpose and Environment	1876

A.8.3.9.2	Test Requirements	1880

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1881

A.8.3.10.1	Test Purpose and Environment	1881

A.8.3.10.2	Test Requirements	1882

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1882

A.8.3.11.1	Test Purpose and Environment	1882

A.8.3.11.2	Test Requirement	1884

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1885

A.8.3.12.1	Test Purpose and Environment	1885

A.8.3.12.2	Test Requirement	1886

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1887

A.8.3.13.1	Test Purpose and Environment	1887

A.8.3.13.2	Test Requirement	1888

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1889

A.8.3.14.1	Test Purpose and Environment	1889

A.8.3.14.2	Test Requirement	1890

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1891

A.8.3.15.1	Test Purpose and Environment	1891

A.8.3.15.2	Test Requirement	1892

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1893

A.8.3.16.1	Test Purpose and Environment	1893

A.8.3.16.2	Test Requirement	1895

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1895

A.8.3.17.1	Test Purpose and Environment	1895

A.8.3.17.2	Test Requirement	1898

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1898

A.8.3.18.1	Test Purpose and Environment	1898

A.8.3.18.2	Test Requirement	1901

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1901

A.8.3.19.1	Test Purpose and Environment	1901

A.8.3.19.2	Test Requirement	1904

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1904

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1904

A.8.4.1.1	Test Purpose and Environment	1904

A.8.4.1.2	Test Requirements	1906

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1906

A.8.4.2.1	Test Purpose and Environment	1906

A.8.4.2.2	Test Requirements	1909

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1909

A.8.4.3.1	Test Purpose and Environment	1909

A.8.4.3.2	Test Requirements	1911

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1912

A.8.4.4.1	Test Purpose and Environment	1912

A.8.4.4.2	Test Requirements	1914

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1915

A.8.4.5.1	Test Purpose and Environment	1915

A.8.4.5.2	Test Requirements	1917

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	1917

A.8.4.6.1	Test Purpose and Environment	1917

A.8.4.6.2	Test Requirements	1918

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1918

A.8.4.7.1	Test Purpose and Environment	1918

A.8.4.7.2	Test Requirements	1922

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1922

A.8.4.8.1	Test Purpose and Environment	1922

A.8.4.8.2	Test Requirements	1925

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1925

A.8.4.9.1	Test Purpose and Environment	1925

A.8.4.9.2	Test Requirements	1930

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1930

A.8.4.10.1	Test Purpose and Environment	1930

A.8.4.10.2	Test Requirements	1932

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	1932

A.8.4.11.1	Test Purpose and Environment	1932

A.8.4.11.2	Test Requirement	1934

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1934

A.8.4.12.1	Test Purpose and Environment	1934

A.8.4.12.2	Test Requirement	1936

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1937

A.8.4.13.1	Test Purpose and Environment	1937

A.8.4.13.2	Test Requirement	1938

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1939

A.8.4.14.1	Test Purpose and Environment	1939

A.8.4.14.2	Test Requirement	1941

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1941

A.8.4.15.1	Test Purpose and Environment	1941

A.8.4.15.2	Test Requirement	1944

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	1944

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	1944

A.8.5.1.1	Test Purpose and Environment	1944

A.8.5.1.2	Test Requirements	1946

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	1946

A.8.5.2.1	Test Purpose and Environment	1946

A.8.5.2.2	Test Requirements	1948

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	1948

A.8.5.3.1	Test Purpose and Environment	1948

A.8.5.3.2	Test Requirements	1951

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	1951

A.8.5.4.1	Test Purpose and Environment	1951

A.8.5.4.2	Test Requirements	1953

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1953

A.8.5.5.1	Test Purpose and Environment	1953

A.8.5.5.2	Test Requirements	1956

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	1956

A.8.5.6.1	Test Purpose and Environment	1956

A.8.5.6.2	Test Requirements	1957

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	1958

A.8.5.7.1	Test Purpose and Environment	1958

A.8.5.7.2	Test Requirements	1958

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1958

A.8.5.8.1	Test Purpose and Environment	1958

A.8.5.8.2	Test Requirements	1961

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	1961

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	1961

A.8.6.1.1	Test Purpose and Environment	1961

A.8.6.1.2	Test Requirements	1963

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	1963

A.8.6.2.1	Test Purpose and Environment	1963

A.8.6.2.2	Test Requirements	1966

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	1966

A.8.6.3.1	Test Purpose and Environment	1966

A.8.6.3.2	Test Requirements	1969

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	1969

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	1969

A.8.7.1.1	Test Purpose and Environment	1969

A.8.7.1.1.1	Void	1969

A.8.7.1.1.2	1.28 Mcps TDD option	1969

A.8.7.1.1.3	Void	1971

A.8.7.1.2	Test Requirements	1971

A.8.7.1.2.1	Void	1971

A.8.7.1.2.2	1.28 Mcps TDD option	1971

A.8.7.1.2.3	Void	1971

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	1971

A.8.7.2.1	Test Purpose and Environment	1971

A.8.7.2.2	Test Requirements	1974

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	1975

A.8.7.3.1	Test Purpose and Environment	1975

A.8.7.3.2	Test Parameters	1975

A.8.7.3.3	Test Requirements	1976

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1977

A.8.7.4.1	Test Purpose and Environment	1977

A.8.7.4.2	Test Requirements	1979

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1979

A.8.7.5.1	Test Purpose and Environment	1979

A.8.7.5.2	Test Requirements	1981

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	1981

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	1981

A.8.7A.1.1	Test Purpose and Environment	1981

A.8.7A.1.2	Test Requirements	1984

A.8.8	E-UTRAN FDD – GSM Measurements	1984

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	1984

A.8.8.1.1	Test Purpose and Environment	1984

A.8.8.1.2	Test Requirements	1986

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	1986

A.8.8.2.1	Test Purpose and Environment	1986

A.8.8.2.2	Test Requirements	1988

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	1989

A.8.8.3.1	Test Purpose and Environment	1989

A.8.8.3.2	Test Requirements	1990

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	1991

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	1991

A.8.9.1.1	Test Purpose and Environment	1991

A.8.9.1.2	Test Requirements	1992

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	1993

A.8.9.2.1	Test Purpose and Environment	1993

A.8.9.2.2	Test Requirements	1995

A.8.10	E-UTRAN TDD – GSM Measurements	1995

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	1995

A.8.10.1.1	Test Purpose and Environment	1995

A.8.10.1.2	Test Requirements	1996

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	1997

A.8.10.2.1	Test Purpose and Environment	1997

A.8.10.2.2	Test Requirements	1999

A.8.11	Monitoring of Multiple Layers	1999

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	1999

A.8.11.1.1	Test Purpose and Environment	1999

A.8.11.1.2	Test Requirements	2001

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2002

A.8.11.2.1	Test Purpose and Environment	2002

A.8.11.2.2	Test Requirements	2003

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2003

A.8.11.3.1	Test Purpose and Environment	2003

A.8.11.3.2	Test Requirements	2006

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2006

A.8.11.4.1	Test Purpose and Environment	2006

A.8.11.4.2	Test Requirements	2009

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2009

A.8.11.5.1	Test Purpose and Environment	2009

A.8.11.5.2	Test Requirements	2011

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2012

A.8.11.6.1	Test Purpose and Environment	2012

A.8.11.6.2	Test Requirements	2014

A.8.12	RSTD Intra-frequency Measurements	2015

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2015

A.8.12.1.1	Test Purpose and Environment	2015

A.8.12.1.2	Test Requirements	2019

A.8.12.1.2A	Test Requirements for UE Category 1bis	2019

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2019

A.8.12.2.1	Test Purpose and Environment	2019

A.8.12.2.2	Test Requirements	2024

A.8.12.2.2A	Test Requirements for UE Category 1bis	2024

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2024

A.8.12.3.1	Test Purpose and Environment	2024

A.8.12.3.2	Test Requirements	2030

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2030

A.8.12.4.1	Test Purpose and Environment	2030

A.8.12.4.2	Test Requirements	2035

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2035

A.8.12.5.1	Test Purpose and Environment	2035

A.8.12.5.2	Test Requirements	2040

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2040

A.8.12.6.1	Test Purpose and Environment	2040

A.8.12.6.2	Test Requirements	2045

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2045

A.8.12.7.1	Test Purpose and Environment	2045

A.8.12.7.2	Test Requirements	2050

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2050

A.8.12.8.1	Test Purpose and Environment	2050

A.8.12.8.2	Test Requirements	2055

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2055

A.8.12.9.1	Test Purpose and Environment	2055

A.8.12.9.2	Test Requirements	2060

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2060

A.8.12.10.1	Test Purpose and Environment	2060

A.8.12.10.2	Test Requirements	2065

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2065

A.8.12.11.1	Test Purpose and Environment	2065

A.8.12.11.2	Test Requirements	2070

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2070

A.8.12.12.1	Test Purpose and Environment	2070

A.8.12.12.2	Test Requirements	2075

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2075

A.8.12.13.1	Test Purpose and Environment	2075

A.8.12.13.2	Test Requirements	2080

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2080

A.8.12.14.1	Test Purpose and Environment	2080

A.8.12.14.2	Test Requirements	2085

A.8.13	RSTD Inter-frequency Measurements	2085

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2085

A.8.13.1.1	Test Purpose and Environment	2085

A.8.13.1.2	Test Requirements	2090

A.8.13.1.2A	Test Requirements for UE Category 1bis	2090

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2090

A.8.13.2.1	Test Purpose and Environment	2090

A.8.13.2.2	Test Requirements	2096

A.8.13.2.2A	Test Requirements for UE Category 1bis	2096

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2096

A.8.13.3.1	Test Purpose and Environment	2096

A.8.13.3.2	Test Requirements	2103

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2103

A.8.13.4.1	Test Purpose and Environment	2103

A.8.13.4.2	Test Requirements	2109

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2109

A.8.13.5.1	Test Purpose and Environment	2109

A.8.13.5.2	Test Requirements	2115

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2115

A.8.13.6.1	Test Purpose and Environment	2115

A.8.13.6.2	Test Requirements	2121

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2121

A.8.13.7.1	Test Purpose and Environment	2121

A.8.13.7.2	Test Requirements	2127

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2127

A.8.13.8.1	Test Purpose and Environment	2127

A.8.13.8.2	Test Requirements	2133

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2133

A.8.13.9.1	Test Purpose and Environment	2133

A.8.13.9.2	Test Requirements	2139

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2139

A.8.13.10.1	Test Purpose and Environment	2139

A.8.13.10.2	Test Requirements	2145

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2145

A.8.13.11.1	Test Purpose and Environment	2145

A.8.13.11.2	Test Requirements	2151

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2151

A.8.13.12.1	Test Purpose and Environment	2151

A.8.13.12.2	Test Requirements	2157

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2157

A.8.13.13.1	Test Purpose and Environment	2157

A.8.13.13.2	Test Requirements	2163

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2163

A.8.13.14.1	Test Purpose and Environment	2163

A.8.13.14.2	Test Requirements	2169

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2169

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2169

A.8.14.1.1	Test Purpose and Environment	2169

A.8.14.1.2	Test Requirements	2171

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2171

A.8.14.2.1	Test Purpose and Environment	2171

A.8.14.2.2	Test Requirements	2174

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2174

A.8.14.3.1	Test Purpose and Environment	2174

A.8.14.3.2	Test Requirements	2176

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2177

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2177

A.8.15.1.1	Test Purpose and Environment	2177

A.8.15.1.2	Test Requirements	2178

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2178

A.8.15.2.1	Test Purpose and Environment	2178

A.8.15.2.2	Test Requirements	2181

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2181

A.8.15.3.1	Test Purpose and Environment	2181

A.8.15.3.2	Test Requirements	2183

A.8.16	E-UTRAN Carrier Aggregation Measurements	2184

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2184

A.8.16.1.1	Test Purpose and Environment	2184

A.8.16.1.2	Test Requirements	2186

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2186

A.8.16.2.1	Test Purpose and Environment	2186

A.8.16.2.2	Test Requirements	2188

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2188

A.8.16.3.1	Test Purpose and Environment	2188

A.8.16.3.2	Test Requirements	2190

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2191

A.8.16.3A.1	Test Purpose and Environment	2191

A.8.16.3A.2	Test Requirements	2192

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2193

A.8.16.4.1	Test Purpose and Environment	2193

A.8.16.4.2	Test Requirements	2194

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2195

A.8.16.4A.1	Test Purpose and Environment	2195

A.8.16.4A.2	Test Requirements	2197

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2198

A.8.16.5.1	Test Purpose and Environment	2198

A.8.16.5.2	Test Requirements	2198

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2198

A.8.16.6.1	Test Purpose and Environment	2198

A.8.16.6.2	Test Requirements	2199

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2199

A.8.16.7.1	 Test Purpose and Environment	2199

A.8.16.7.2	 Test Requirements	2200

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2200

A.8.16.8.1	 Test Purpose and Environment	2200

A.8.16.8.2	 Test Requirements	2201

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2201

A.8.16.9.1	Test Purpose and Environment	2201

A.8.16.9.2	Test Requirements	2202

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2202

A.8.16.10.1	Test Purpose and Environment	2202

A.8.16.10.2	Test Requirements	2203

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2203

A.8.16.11.1	 Test Purpose and Environment	2203

A.8.16.11.2	 Test Requirements	2204

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2204

A.8.16.12.1	 Test Purpose and Environment	2204

A.8.16.12.2	 Test Requirements	2204

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2204

A.8.16.13.1	Test Purpose and Environment	2204

A.8.16.13.2	Test Requirements	2205

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2205

A.8.16.14.1	Test Purpose and Environment	2205

A.8.16.14.2	Test Requirements	2206

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2206

A.8.16.15.1	 Test Purpose and Environment	2206

A.8.16.7.2	 Test Requirements	2206

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2206

A.8.16.16.1	 Test Purpose and Environment	2206

A.8.16.16.2	 Test Requirements	2207

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2207

A.8.16.17.1	Test Purpose and Environment	2207

A.8.16.17.2	Test Requirements	2209

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2210

A.8.16.17A.1	Test Purpose and Environment	2210

A.8.16.17A.2	Test Requirements	2210

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2210

A.8.16.17B.1	Test Purpose and Environment	2210

A.8.16.17B.2	Test Requirements	2211

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2211

A.8.16.17C.1	Test Purpose and Environment	2211

A.8.16.17C.2	Test Requirements	2211

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2211

A.8.16.18.1	Test Purpose and Environment	2211

A.8.16.18.2	Test Requirements	2213

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2214

A.8.16.18A.1	Test Purpose and Environment	2214

A.8.16.18A.2	Test Requirements	2214

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2214

A.8.16.18B.1	Test Purpose and Environment	2214

A.8.16.18B.2	Test Requirements	2215

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2215

A.8.16.18C.1	Test Purpose and Environment	2215

A.8.16.18C.2	Test Requirements	2215

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2215

A.8.16.18D.1	Test Purpose and Environment	2215

A.8.16.18D.2	Test Requirements	2216

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2216

A.8.16.19.1	Test Purpose and Environment	2216

A.8.16.19.2	Test Requirements	2218

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2218

A.8.16.19A.1	Test Purpose and Environment	2218

A.8.16.19A.2	Test Requirements	2219

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2219

A.8.16.19B.1	Test Purpose and Environment	2219

A.8.16.19B.2	Test Requirements	2219

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2219

A.8.16.19C.1	Test Purpose and Environment	2219

A.8.16.19C.2	Test Requirements	2220

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2220

A.8.16.20.1	Test Purpose and Environment	2220

A.8.16.20.2	Test Requirements	2222

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2223

A.8.16.20A.1	Test Purpose and Environment	2223

A.8.16.20A.2	Test Requirements	2223

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2223

A.8.16.20B.1	Test Purpose and Environment	2223

A.8.16.20B.2	Test Requirements	2224

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2224

A.8.16.20C.1	Test Purpose and Environment	2224

A.8.16.20C.2	Test Requirements	2224

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2224

A.8.16.20D.1	Test Purpose and Environment	2224

A.8.16.20D.2	Test Requirements	2225

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2225

A.8.16.21.1	Test Purpose and Environment	2225

A.8.16.21.2	Test Requirements	2227

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2227

A.8.16.22.1	Test Purpose and Environment	2227

A.8.16.22.2	Test Requirements	2229

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2229

A.8.16.23.1	Test Purpose and Environment	2229

A.8.16.23.2	Test Requirements	2232

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2232

A.8.16.24.1	Test Purpose and Environment	2232

A.8.16.24.2	Test Requirements	2235

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2235

A.8.16.25.1	Test Purpose and Environment	2235

A.8.16.25.2	Test Requirements	2238

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2238

A.8.16.26.1	Test Purpose and Environment	2238

A.8.16.26.2	Test Requirements	2241

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2241

A.8.16.27.1	Test Purpose and Environment	2241

A.8.16.27.2	Test Requirements	2246

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2246

A.8.16.28.1	Test Purpose and Environment	2246

A.8.16.28.2	Test Requirements	2251

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2251

A.8.16.29.1	Test Purpose and Environment	2251

A.8.16.29.2	Test Requirements	2256

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2256

A.8.16.30.1	Test Purpose and Environment	2256

A.8.16.30.2	Test Requirements	2261

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2261

A.8.16.31.1	Test Purpose and Environment	2261

A.8.16.31.2	Test Requirements	2266

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2266

A.8.16.32.1	Test Purpose and Environment	2266

A.8.16.32.2	Test Requirements	2271

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2271

A.8.16.33.1	Test Purpose and Environment	2271

A.8.16.33.2	Test Requirements	2276

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2276

A.8.16.34.1	Test Purpose and Environment	2276

A.8.16.34.2	Test Requirements	2281

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2281

A.8.16.35.1	Test Purpose and Environment	2281

A.8.16.35.2	Test Requirements	2283

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2284

A.8.16.36.1	Test Purpose and Environment	2284

A.8.16.36.2	Test Requirements	2286

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2287

A.8.16.37.1	Test Purpose and Environment	2287

A.8.16.37.2	Test Requirements	2289

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2290

A.8.16.38.1	Test Purpose and Environment	2290

A.8.16.38.2	Test Requirements	2292

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2293

A.8.16.39.1	Test Purpose and Environment	2293

A.8.16.39.2	Test Requirements	2296

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2296

A.8.16.40.1	Test Purpose and Environment	2296

A.8.16.40.2	Test Requirements	2299

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2299

A.8.16.41.1	Test Purpose and Environment	2299

A.8.16.41.2	Test Requirements	2301

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2302

A.8.16.42.1	Test Purpose and Environment	2302

A.8.16.42.2	Test Requirements	2304

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2305

A.8.16.43.1	Test Purpose and Environment	2305

A.8.16.43.2	Test Requirements	2308

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2308

A.8.16.44.1	Test Purpose and Environment	2308

A.8.16.44.2	Test Requirements	2311

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2311

A.8.16.45.1	Test Purpose and Environment	2311

A.8.16.45.2	Test Requirements	2314

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2314

A.8.16.46.1	Test Purpose and Environment	2314

A.8.16.46.2	Test Requirements	2316

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2317

A.8.16.47.1	Test Purpose and Environment	2317

A.8.16.47.2	Test Requirements	2319

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2320

A.8.16.48.1	Test Purpose and Environment	2320

A.8.16.48.2	Test Requirements	2322

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2323

A.8.16.49.1	Test Purpose and Environment	2323

A.8.16.49.2	Test Requirements	2326

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2326

A.8.16.50.1	Test Purpose and Environment	2326

A.8.16.50.2	Test Requirements	2328

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2329

A.8.16.51.1	Test Purpose and Environment	2329

A.8.16.51.2	Test Requirements	2334

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2334

A.8.16.52.1	Test Purpose and Environment	2334

A.8.16.52.2	Test Requirements	2339

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2339

A.8.16.53.1	Test Purpose and Environment	2339

A.8.16.53.2	Test Requirements	2343

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2343

A.8.16.54.1	Test Purpose and Environment	2343

A.8.16.54.2	Test Requirements	2347

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2347

A.8.16.55.1	Test Purpose and Environment	2347

A.8.16.55.2	Test Requirements	2353

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2353

A.8.16.56.1	Test Purpose and Environment	2353

A.8.16.56.2	Test Requirements	2359

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2359

A.8.16.57.1	Test Purpose and Environment	2359

A.8.16.57.2	Test Requirements	2361

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2362

A.8.16.58.1	Test Purpose and Environment	2362

A.8.16.58.2	Test Requirements	2364

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2365

A.8.16.59.1	Test Purpose and Environment	2365

A.8.16.59.2	Test Requirements	2369

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2370

A.8.16.60.1	Test Purpose and Environment	2370

A.8.16.60.2	Test Requirements	2373

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2374

A.8.16.61.1	Test Purpose and Environment	2374

A.8.16.61.2	Test Requirements	2377

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2377

A.8.16.62.1	Test Purpose and Environment	2377

A.8.16.62.2	Test Requirements	2379

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2380

A.8.16.63.1	Test Purpose and Environment	2380

A.8.16.63.2	Test Requirements	2383

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2384

A.8.16.64.1	Test Purpose and Environment	2384

A.8.16.64.2	Test Requirements	2388

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2388

A.8.16.65.1	Test Purpose and Environment	2388

A.8.16.65.2	Test Requirements	2393

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2393

A.8.16.66.1	Test Purpose and Environment	2393

A.8.16.66.2	Test Requirements	2398

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2398

A.8.16.67.1	Test Purpose and Environment	2398

A.8.16.67.2	Test Requirements	2403

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2403

A.8.16.68.1	Test Purpose and Environment	2403

A.8.16.68.2	Test Requirements	2408

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2408

A.8.16.69.1	Test Purpose and Environment	2408

A.8.16.69.2	Test Requirements	2412

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2412

A.8.16.70.1	Test Purpose and Environment	2412

A.8.16.70.2	Test Requirements	2416

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2416

A.8.16.71.1	Test Purpose and Environment	2416

A.8.16.71.2	Test Requirements	2421

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2421

A.8.16.72.1	Test Purpose and Environment	2421

A.8.16.72.2	Test Requirements	2426

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2426

A.8.16.73.1	Test Purpose and Environment	2426

A.8.16.73.2	Test Requirements	2433

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2433

A.8.16.74.1	Test Purpose and Environment	2433

A.8.16.74.2	Test Requirements	2440

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2440

A.8.16.75.1	Test Purpose and Environment	2440

A.8.16.75.2	Test Requirements	2446

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2446

A.8.16.76.1	Test Purpose and Environment	2446

A.8.16.76.2	Test Requirements	2452

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2452

A.8.16.77.1	Test Purpose and Environment	2452

A.8.16.77.2	Test Requirements	2456

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2456

A.8.16.78.1	Test Purpose and Environment	2456

A.8.16.78.2	Test Requirements	2458

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2459

A.8.16.79.1	Test Purpose and Environment	2459

A.8.16.79.2	Test Requirements	2466

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2466

A.8.16.80.1	Test Purpose and Environment	2466

A.8.16.80.2	Test Requirements	2473

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2473

A.8.16.81.1	Test Purpose and Environment	2473

A.8.16.81.2	Test Requirements	2479

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2479

A.8.16.82.1	Test Purpose and Environment	2479

A.8.16.82.2	Test Requirements	2485

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2485

A.8.16.83.1	Test Purpose and Environment	2485

A.8.16.83.2	Test Requirements	2490

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2490

A.8.16.84.1	Test Purpose and Environment	2490

A.8.16.84.2	Test Requirements	2496

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2496

A.8.16.85.1	Test Purpose and Environment	2496

A.8.16.85.2	Test Requirements	2500

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2500

A.8.16.86.1	Test Purpose and Environment	2500

A.8.16.86.2	Test Requirements	2503

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2503

A.8.16.87.1	Test Purpose and Environment	2503

A.8.16.87.2	Test Requirements	2509

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2509

A.8.16.88.1	Test Purpose and Environment	2509

A.8.16.88.2	Test Requirements	2517

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2517

A.8.16.89.1	Test Purpose and Environment	2517

A.8.16.89.2	Test Requirements	2524

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2524

A.8.16.90.1	Test Purpose and Environment	2524

A.8.16.90.2	Test Requirements	2531

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2531

A.8.16.91.1	Test Purpose and Environment	2531

A.8.16.91.2	Test Requirements	2537

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2537

A.8.16.92.1	Test Purpose and Environment	2537

A.8.16.92.2	Test Requirements	2545

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2545

A.8.16.93.1	Test Purpose and Environment	2545

A.8.16.93.2	Test Requirements	2552

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2552

A.8.16.94.1	Test Purpose and Environment	2552

A.8.16.94.2	Test Requirements	2559

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2559

A.8.16.95.1	Test Purpose and Environment	2559

A.8.16.95.2	Test Requirements	2567

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2567

A.8.16.96.1	Test Purpose and Environment	2567

A.8.16.96.2	Test Requirements	2576

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2576

A.8.16.97.1	Test Purpose and Environment	2576

A.8.16.97.2	Test Requirements	2583

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2583

A.8.16.98.1	Test Purpose and Environment	2583

A.8.16.98.2	Test Requirements	2591

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2591

A.8.16.99.1	Test Purpose and Environment	2591

A.8.16.99.2	Test Requirements	2601

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2601

A.8.16.100.1	Test Purpose and Environment	2601

A.8.16.100.2	Test Requirements	2611

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2611

A.8.16.101.1	Test Purpose and Environment	2611

A.8.16.101.2	Test Requirements	2621

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2621

A.8.16.102.1	Test Purpose and Environment	2621

A.8.16.102.2	Test Requirements	2632

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2632

A.8.16.103.1	Test Purpose and Environment	2632

A.8.16.103.2	Test Requirements	2637

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2638

A.8.16.104.1	Test Purpose and Environment	2638

A.8.16.104.2	Test Requirements	2642

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2643

A.8.16.105.1	Test Purpose and Environment	2643

A.8.16.105.2	Test Requirements	2647

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2648

A.8.16.106.1	Test Purpose and Environment	2648

A.8.16.106.2	Test Requirements	2652

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2652

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2652

A.8.17.1.1	Test Purpose and Environment	2652

A.8.17.1.2	Test Requirements	2659

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2659

A.8.17.2.1	Test Purpose and Environment	2659

A.8.17.2.2	Test Requirements	2666

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2666

A.8.17.3.1	Test Purpose and Environment	2666

A.8.17.3.2	Test Requirements	2667

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2667

A.8.17.4.1	Test Purpose and Environment	2667

A.8.17.4.2	Test Requirements	2668

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2668

A.8.17.5.1	Test Purpose and Environment	2668

A.8.17.5.2	Test Requirements	2669

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2669

A.8.17.6.1	Test Purpose and Environment	2669

A.8.17.6.2	Test Requirements	2670

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2671

A.8.17.7.1	Test Purpose and Environment	2671

A.8.17.7.2	Test Requirements	2671

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2672

A.8.17.8.1	Test Purpose and Environment	2672

A.8.17.8.2	Test Requirements	2672

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2673

A.8.17.9.1	Test Purpose and Environment	2673

A.8.17.9.2	Test Requirements	2674

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2674

A.8.17.10.1	Test Purpose and Environment	2674

A.8.17.10.2	Test Requirements	2681

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2682

A.8.17.11.1	Test Purpose and Environment	2682

A.8.17.11.2	Test Requirements	2688

A.8.18	E-UTRAN TDD – HRPD Measurements	2689

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2689

A.8.18.1.1	Test Purpose and Environment	2689

A.8.18.1.2	Test Requirements	2691

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2691

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2691

A.8.19.1.1	Test Purpose and Environment	2691

A.8.19.1.2	Test Requirements	2692

A.8.20	Inter-frequency/RAT Measurements in CA mode	2693

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2693

A.8.20.1.1	Test Purpose and Environment	2693

A.8.20.1.2	Test Requirements	2694

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2694

A.8.20.2.1	Test Purpose and Environment	2695

A.8.20.2.2	Test Requirements	2696

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2697

A.8.20.2A.1	Test Purpose and Environment	2697

A.8.20.2A.2	Test Requirements	2697

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2697

A.8.20.2B.1	Test Purpose and Environment	2697

A.8.20.2B.2	Test Requirements	2700

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2700

A.8.20.3.1	Test Purpose and Environment	2700

A.8.20.3.2	Test Requirements	2702

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2702

A.8.20.4.1	Test Purpose and Environment	2702

A.8.20.4.1.1	1.28 Mcps TDD option	2702

A.8.20.4.2	Test Requirements	2704

A.8.20.4.2.1	1.28 Mcps TDD option	2704

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2704

A.8.20.4A.1	Test Purpose and Environment	2704

A.8.20.4A.1.1	1.28 Mcps TDD option	2704

A.8.20.4A.2	Test Requirements	2704

A.8.20.4A.2.1	1.28 Mcps TDD option	2704

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2705

A.8.20.4B.1	Test Purpose and Environment	2705

A.8.20.4B.1.1	1.28 Mcps TDD option	2705

A.8.20.4B.2	Test Requirements	2707

A.8.20.4B.2.1	1.28 Mcps TDD option	2707

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2707

A.8.21.1	Test Purpose and Environment	2707

A.8.21.2 Test Requirements	2710

A.8.22	E-UTRAN Discovery Signal Measurements	2710

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2710

A.8.22.1.1	Test Purpose and Environment	2710

A.8.22.1.2	Test Requirements	2713

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2713

A.8.22.2.1	Test Purpose and Environment	2713

A.8.22.2.2	Test Requirements	2716

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2716

A.8.22.3.1	Test Purpose and Environment	2716

A.8.22.3.2	Test Requirements	2719

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2719

A.8.22.4.1	Test Purpose and Environment	2719

A.8.22.4.2	Test Requirements	2722

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2722

A.8.22.5.1	Test Purpose and Environment	2722

A.8.22.5.2	Test Requirements	2726

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2726

A.8.22.6.1	Test Purpose and Environment	2726

A.8.22.6.2	Test Requirements	2730

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2730

A.8.22.7.1	Test Purpose and Environment	2730

A.8.22.7.2	Test Requirements	2734

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2734

A.8.22.8.1	Test Purpose and Environment	2734

A.8.22.8.2	Test Requirements	2738

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2738

A.8.22.9.1	Test Purpose and Environment	2738

A.8.22.9.2	Test Requirements	2740

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2741

A.8.22.10.1	Test Purpose and Environment	2741

A.8.22.10.2	Test Requirements	2743

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2744

A.8.22.11.1	Test Purpose and Environment	2744

A.8.22.11.2	Test Requirements	2747

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2747

A.8.22.12.1	Test Purpose and Environment	2747

A.8.22.12.2	Test Requirements	2751

A.8.23	E-UTRAN Dual Connectivity Measurements	2751

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2751

A.8.23.1.1	Test Purpose and Environment	2751

A.8.23.1.2	Test Requirements	2754

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2754

A.8.23.2.1	Test Purpose and Environment	2754

A.8.23.2.2	Test Requirements	2757

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2757

A.8.23.3.1	Test Purpose and Environment	2757

A.8.23.3.2	Test Requirements	2760

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2760

A.8.23.4.1	Test Purpose and Environment	2760

A.8.23.4.2	Test Requirements	2763

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2763

A.8.23.5.1	Test Purpose and Environment	2763

A.8.23.5.2	Test Requirements	2766

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2766

A.8.23.6.1	Test Purpose and Environment	2766

A.8.23.6.2	Test Requirements	2769

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2769

A.8.23.7.1	Test Purpose and Environment	2769

A.8.23.7.2	Test Requirements	2771

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2772

A.8.23.8.1	Test Purpose and Environment	2772

A.8.23.8.2	Test Requirements	2774

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2775

A.8.23.9.1	Test Purpose and Environment	2775

A.8.23.9.2	Test Requirements	2778

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2778

A.8.23.10.1	Test Purpose and Environment	2778

A.8.23.10.2	Test Requirements	2781

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2781

A.8.23.11.1	Test Purpose and Environment	2781

A.8.23.11.2	Test Requirements	2784

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2784

A.8.23.12.1	Test Purpose and Environment	2784

A.8.23.12.2	Test Requirements	2787

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2787

A.8.23.13.1	Test Purpose and Environment	2787

A.8.23.13.2	Test Requirements	2790

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2790

A.8.23.14.1	Test Purpose and Environment	2790

A.8.23.14.2	Test Requirements	2793

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2793

A.8.23.15.1	Test Purpose and Environment	2793

A.8.23.15.2	Test Requirements	2796

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2796

A.8.23.16.1	Test Purpose and Environment	2796

A.8.23.16.2	Test Requirements	2797

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2798

A.8.23.17.1	Test Purpose and Environment	2798

A.8.23.17.2	Test Requirements	2800

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2800

A.8.23.18.1	Test Purpose and Environment	2800

A.8.23.18.2	Test Requirements	2801

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2802

A.8.23.19.1	Test Purpose and Environment	2802

A.8.23.19.2	Test Requirements	2803

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2804

A.8.23.20.1	Test Purpose and Environment	2804

A.8.23.20.2	Test Requirements	2805

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2806

A.8.23.21.1	Test Purpose and Environment	2806

A.8.23.21.2	Test Requirements	2807

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2808

A.8.23.22.1	Test Purpose and Environment	2808

A.8.23.22.2	Test Requirements	2809

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2810

A.8.23.23.1	Test Purpose and Environment	2810

A.8.23.23.2	Test Requirements	2811

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2812

A.8.23.24.1	Test Purpose and Environment	2812

A.8.23.24.2	Test Requirements	2814

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2815

A.8.23.25.1	Test Purpose and Environment	2815

A.8.23.25.2	Test Requirements	2817

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2818

A.8.23.26.1	Test Purpose and Environment	2818

A.8.23.26.2	Test Requirements	2820

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2821

A.8.23.27.1	Test Purpose and Environment	2821

A.8.23.27.2	Test Requirements	2824

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2824

A.8.23.28.1	Test Purpose and Environment	2824

A.8.23.28.2	Test Requirements	2828

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2828

A.8.23.29.1	Test Purpose and Environment	2828

A.8.23.29.2	Test Requirements	2832

A.8.24	Proximity-based Services	2832

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2832

A.8.24.1.1	Test Purpose and Environment	2832

A.8.24.1.2	Test Requirements	2833

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2834

A.8.24.2.1	Test Purpose and Environment	2834

A.8.24.2.2	Test Requirements	2835

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2835

A.8.24.3.1	Test Purpose and Environment	2836

A.8.24.3.2	Test Requirements	2837

A.8.25	E-UTRAN-WLAN Measurements	2838

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2838

A.8.25.1.1	Test Purpose and Environment	2838

A.8.25.1.2	Test Requirements	2840

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2840

A.8.25.2.1	Test Purpose and Environment	2840

A.8.25.2.2	Test Requirements	2842

A.8.26 Frame Structure 3 (FS3)	2842

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2842

A.8.26.1.1	Test Purpose and Environment	2842

A.8.26.1.2	Test Requirements	2844

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2845

A.8.26.2.1	Test Purpose and Environment	2845

A.8.26.2.2	Test Requirements	2847

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2848

A.8.26.3.1	Test Purpose and Environment	2848

A.8.26.3.2	Test Requirements	2851

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2851

A.8.26.3A.1	Test Purpose and Environment	2851

A.8.26.3A.2	Test Requirements	2854

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2854

A.8.26.4.1	Test Purpose and Environment	2854

A.8.26.4.2	Test Requirements	2858

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2858

A.8.26.4A.1	Test Purpose and Environment	2858

A.8.26.4A.2	Test Requirements	2862

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2862

A.8.26.5.1	Test Purpose and Environment	2862

A.8.26.5.2	Test Requirements	2865

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2865

A.8.26.5A.1	Test Purpose and Environment	2865

A.8.26.5A.2	Test Requirements	2869

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2869

A.8.26.6.1	Test Purpose and Environment	2869

A.8.26.6.2	Test Requirements	2873

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2873

A.8.26.6A.1	Test Purpose and Environment	2873

A.8.26.6A.2	Test Requirements	2877

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2877

A.8.26.7.1	Test Purpose and Environment	2877

A.8.26.7.2	Test Requirements	2880

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2880

A.8.26.8.1	Test Purpose and Environment	2880

A.8.26.8.2	Test Requirements	2884

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2884

A.8.26.9.1	Test Purpose and Environment	2884

A.8.26.9.2	Test Requirements	2887

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2887

A.8.26.10.1	Test Purpose and Environment	2887

A.8.26.10.2	Test Requirements	2890

A.9	Measurement Performance Requirements	2768

A.9.1	RSRP	2768

A.9.1.1	FDD Intra frequency case	2768

A.9.1.1.1	Test Purpose and Environment	2768

A.9.1.1.2	Test parameters	2768

A.9.1.1.3	Test Requirements	2771

A.9.1.2	TDD Intra frequency case	2771

A.9.1.2.1	Test Purpose and Environment	2771

A.9.1.2.2	Test parameters	2771

A.9.1.2.3	Test Requirements	2772

A.9.1.3	FDD—FDD Inter frequency case	2773

A.9.1.3.1	Test Purpose and Environment	2773

A.9.1.3.2	Test parameters	2773

A.9.1.3.3	Test Requirements	2776

A.9.1.4	TDD—TDD Inter frequency case	2776

A.9.1.4.1	Test Purpose and Environment	2776

A.9.1.4.2	Test parameters	2776

A.9.1.4.3	Test Requirements	2778

A.9.1.5	FDD—TDD Inter frequency case	2779

A.9.1.5.1	Test Purpose and Environment	2779

A.9.1.5.2	Test parameters	2779

A.9.1.5.3	Test Requirements	2780

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2781

A.9.1.6.1	Test Purpose and Environment	2781

A.9.1.6.2	Test parameters	2781

A.9.1.6.3	Test Requirements	2784

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2784

A.9.1.7.1	Test Purpose and Environment	2784

A.9.1.7.2	Test parameters	2784

A.9.1.7.3	Test Requirements	2787

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2787

A.9.1.8.1	Test Purpose and Environment	2787

A.9.1.8.2	Test parameters	2787

A.9.1.8.3	Test Requirements	2791

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2791

A.9.1.9.1	Test Purpose and Environment	2791

A.9.1.9.2	Test parameters	2791

A.9.1.9.3	Test Requirements	2794

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2794

A.9.1.10.1	Test Purpose and Environment	2794

A.9.1.10.2	Test parameters	2794

A.9.1.10.3	Test Requirements	2797

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2797

A.9.1.11.1	Test Purpose and Environment	2797

A.9.1.11.2	Test parameters	2797

A.9.1.11.3	Test Requirements	2801

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2801

A.9.1.12.1	Test Purpose and Environment	2801

A.9.1.12.2	Test parameters	2801

A.9.1.12.3	Test Requirements	2802

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2802

A.9.1.13.1	Test Purpose and Environment	2802

A.9.1.13.2	Test parameters	2802

A.9.1.13.3	Test Requirements	2803

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2803

A.9.1.14.1	Test Purpose and Environment	2803

A.9.1.14.2	Test parameters	2803

A.9.1.14.3	Test Requirements	2807

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2807

A.9.1.15.1	Test Purpose and Environment	2807

A.9.1.15.2	Test parameters	2807

A.9.1.15.3	Test Requirements	2809

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2810

A.9.1.16.1	Test Purpose and Environment	2810

A.9.1.16.2	Test parameters	2810

A.9.1.16.3	Test Requirements	2811

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2811

A.9.1.17.1	Test Purpose and Environment	2811

A.9.1.17.2	Test parameters	2812

A.9.1.17.3	Test Requirements	2813

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2813

A.9.1.18.1	Test Purpose and Environment	2813

A.9.1.18.2	Test parameters	2813

A.9.1.18.3	Test Requirements	2814

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2815

A.9.1.19.1	Test Purpose and Environment	2815

A.9.1.19.2	Test parameters	2815

A.9.1.19.3	Test Requirements	2815

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2815

A.9.1.20.1	Test Purpose and Environment	2815

A.9.1.20.2	Test parameters	2815

A.9.1.20.3	Test Requirements	2817

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2817

A.9.1.21.1	Test Purpose and Environment	2817

A.9.1.21.2	Test parameters	2817

A.9.1.21.3	Test Requirements	2817

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2817

A.9.1.22.1	Test Purpose and Environment	2817

A.9.1.22.2	Test parameters	2818

A.9.1.22.3	Test Requirements	2822

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2822

A.9.1.23.1	Test Purpose and Environment	2822

A.9.1.23.2	Test parameters	2822

A.9.1.23.3	Test Requirements	2826

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2826

A.9.1.24.1	Test Purpose and Environment	2826

A.9.1.24.2	Test parameters	2826

A.9.1.24.3	Test Requirements	2827

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2827

A.9.1.25.1	Test Purpose and Environment	2827

A.9.1.25.2	Test parameters	2827

A.9.1.25.3	Test Requirements	2830

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2830

A.9.1.26.1	Test Purpose and Environment	2830

A.9.1.26.2	Test parameters	2830

A.9.1.26.3	Test Requirements	2833

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2833

A.9.1.27.1	Test Purpose and Environment	2833

A.9.1.27.2	Test parameters	2833

A.9.1.27.3	Test Requirements	2836

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2836

A.9.1.28.1	Test Purpose and Environment	2836

A.9.1.28.2	Test parameters	2836

A.9.1.28.3	Test Requirements	2839

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2839

A.9.1.29.1	Test Purpose and Environment	2839

A.9.1.29.2	Test parameters	2839

A.9.1.29.3	Test Requirements	2842

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2842

A.9.1.30.1	Test Purpose and Environment	2842

A.9.1.30.2	Test parameters	2843

A.9.1.30.3	Test Requirements	2846

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2846

A.9.1.31.1	Test Purpose and Environment	2846

A.9.1.31.2	Test parameters	2846

A.9.1.31.3	Test Requirements	2849

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2849

A.9.1.32.1	Test Purpose and Environment	2849

A.9.1.32.2	Test parameters	2850

A.9.1.32.3	Test Requirements	2853

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2853

A.9.1.33.1	Test Purpose and Environment	2853

A.9.1.33.2	Test parameters	2853

A.9.1.33.3	Test Requirements	2856

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2856

A.9.1.34.1	Test Purpose and Environment	2856

A.9.1.34.2	Test parameters	2856

A.9.1.34.3	Test Requirements	2859

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2859

A.9.1.35.1	Test Purpose and Environment	2859

A.9.1.35.2	Test parameters	2859

A.9.1.35.3	Test Requirements	2862

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2863

A.9.1.36.1	Test Purpose and Environment	2863

A.9.1.36.2	Test parameters	2863

A.9.1.36.3	Test Requirements	2866

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2866

A.9.1.37.1	Test Purpose and Environment	2866

A.9.1.37.2	Test parameters	2866

A.9.1.37.3	Test Requirements	2869

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2870

A.9.1.38.1	Test Purpose and Environment	2870

A.9.1.38.2	Test parameters	2870

A.9.1.38.3	Test Requirements	2873

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2874

A.9.1.39.1	Test Purpose and Environment	2874

A.9.1.39.2	Test parameters	2874

A.9.1.39.3	Test Requirements	2880

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2880

A.9.1.40.1	Test Purpose and Environment	2880

A.9.1.40.2	Test parameters	2881

A.9.1.40.3	Test Requirements	2887

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2887

A.9.1.41.1	Test Purpose and Environment	2887

A.9.1.41.2	Test parameters	2887

A.9.1.41.3	Test Requirements	2890

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2890

A.9.1.42.1	Test Purpose and Environment	2890

A.9.1.42.2	Test parameters	2890

A.9.1.42.3	Test Requirements	2893

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2893

A.9.1.43.1	Test Purpose and Environment	2893

A.9.1.43.2	Test parameters	2893

A.9.1.43.3	Test Requirements	2894

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2895

A.9.1.44.1	Test Purpose and Environment	2895

A.9.1.44.2	Test parameters	2895

A.9.1.44.3	Test Requirements	2900

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2900

A.9.1.45.1	Test Purpose and Environment	2900

A.9.1.45.2	Test parameters	2900

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2486

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2486

A.9.1.55.1	Test Purpose and Environment	2486

A.9.1.55.2	Test parameters	2486

A.9.1.55.3	Test Requirements	2490

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2490

A.9.1.56.1	Test Purpose and Environment	2490

A.9.1.56.2	Test parameters	2490

A.9.1.56.3	Test Requirements	2493

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2494

A.9.1.57.1	Test Purpose and Environment	2494

A.9.1.57.2	Test parameters	2494

A.9.1.57.3	Test Requirements	2497

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2497

A.9.1.57A.1	Test Purpose and Environment	2497

A.9.1.57A.2	Test parameters	2497

A.9.1.57A.3	Test Requirements	2498

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2498

A.9.1.58.1	Test Purpose and Environment	2498

A.9.1.58.2	Test parameters	2499

A.9.1.58.3	Test Requirements	2502

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2502

A.9.1.58A.1	Test Purpose and Environment	2502

A.9.1.58A.2	Test parameters	2502

A.9.1.58A.3	Test Requirements	2503

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2503

A.9.1.59.1	Test Purpose and Environment	2503

A.9.1.59.2	Test parameters	2504

A.9.1.59.3	Test Requirements	2506

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2506

A.9.1.60.1	Test Purpose and Environment	2506

A.9.1.60.2	Test parameters	2506

A.9.1.60.3	Test Requirements	2509

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2510

A.9.1.61.1	Test Purpose and Environment	2510

A.9.1.61.2	Test parameters	2510

A.9.1.61.3	Test Requirements	2513

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2513

A.9.1.62.1	Test Purpose and Environment	2513

A.9.1.62.2	Test parameters	2513

A.9.1.62.3	Test Requirements	2516

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2516

A.9.1.53.1	Test Purpose and Environment	2516

A.9.1.53.2	Test parameters	2516

A.9.1.63.3	Test Requirements	2519

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2519

A.9.1.64.1	Test Purpose and Environment	2519

A.9.1.64.2	Test parameters	2519

A.9.1.64.3	Test Requirements	2520

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2521

A.9.1.65.1	Test Purpose and Environment	2521

A.9.1.65.2	Test parameters	2521

A.9.1.65.3	Test Requirements	2527

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2527

A.9.1.66.1	Test Purpose and Environment	2527

A.9.1.66.2	Test parameters	2527

A.9.1.66.3	Test Requirements	2533

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2533

A.9.1.67.1	Test Purpose and Environment	2533

A.9.1.67.2	Test parameters	2533

A.9.1.67.3	Test Requirements	2534

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2535

A.9.1.68.1	Test Purpose and Environment	2535

A.9.1.68.2	Test parameters	2535

A.9.1.68.3	Test Requirements	2539

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2539

A.9.1.69.1	Test Purpose and Environment	2539

A.9.1.69.2	Test parameters	2539

A.9.1.69.3	Test Requirements	2543

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2544

A.9.1.70.1	Test Purpose and Environment	2544

A.9.1.70.2	Test parameters	2544

A.9.1.70.3	Test Requirements	2556

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2557

A.9.1.71.1	Test Purpose and Environment	2557

A.9.1.71.2	Test parameters	2557

A.9.1.71.3	Test Requirements	2569

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2570

A.9.1.72.1	Test Purpose and Environment	2570

A.9.1.72.2	Test parameters	2570

A.9.1.72.3	Test Requirements	2582

A.9.2	RSRQ	2583

A.9.2.1	FDD Intra frequency case	2583

A.9.2.1.1	Test Purpose and Environment	2583

A.9.2.1.2	Test parameters	2583

A.9.2.1.3	Test Requirements	2586

A.9.2.2	TDD Intra frequency case	2586

A.9.2.2.1	Test Purpose and Environment	2586

A.9.2.2.2	Test parameters	2586

A.9.2.2.3	Test Requirements	2588

A.9.2.3	FDD—FDD Inter frequency case	2588

A.9.2.3.1	Test Purpose and Environment	2588

A.9.2.3.2	Test parameters	2588

A.9.2.3.3	Test Requirements	2591

A.9.2.4	TDD—TDD Inter frequency case	2591

A.9.2.4.1	Test Purpose and Environment	2591

A.9.2.4.2	Test parameters	2591

A.9.2.4.3	Test Requirements	2593

A.9.2.4A	FDD—TDD Inter frequency case	2594

A.9.2.4A.1	Test Purpose and Environment	2594

A.9.2.4A.2	Test parameters	2594

A.9.2.4A.3	Test Requirements	2596

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2596

A.9.2.5.1	Test Purpose and Environment	2596

A.9.2.5.2	Test parameters	2597

A.9.2.5.3	Test Requirements	2600

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2600

A.9.2.6.1	Test Purpose and Environment	2600

A.9.2.6.2	Test parameters	2600

A.9.2.6.3	Test Requirements	2603

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2603

A.9.2.7.1	Test Purpose and Environment	2603

A.9.2.7.2	Test parameters	2603

A.9.2.7.3	Test Requirements	2607

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2607

A.9.2.8.1	Test Purpose and Environment	2607

A.9.2.8.2	Test parameters	2607

A.9.2.8.3	Test Requirements	2610

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2610

A.9.2.9.1	Test Purpose and Environment	2610

A.9.2.9.2	Test parameters	2610

A.9.2.9.3	Test Requirements	2614

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2614

A.9.2.10.1	Test Purpose and Environment	2614

A.9.2.10.2	Test parameters	2614

A.9.2.10.3	Test Requirements	2618

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2618

A.9.2.11.1	Test Purpose and Environment	2618

A.9.2.11.2	Test parameters	2618

A.9.2.11.3	Test Requirements	2619

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2619

A.9.2.12.1	Test Purpose and Environment	2619

A.9.2.12.2	Test parameters	2620

A.9.2.12.3	Test Requirements	2620

A.9.2.13	Void	2621

A.9.2.13.1	Void	2621

A.9.2.13.2	Void	2621

A.9.2.13.3	Void	2621

A.9.2.14	Void	2621

A.9.2.14.1	Void	2621

A.9.2.14.2	Void	2621

A.9.2.14.3	Void	2621

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2621

A.9.2.15.1	Test Purpose and Environment	2621

A.9.2.15.2	Test parameters	2621

A.9.2.15.3	Test Requirements	2625

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2625

A.9.2.16.1	Test Purpose and Environment	2625

A.9.2.16.2	Test parameters	2625

A.9.2.16.3	Test Requirements	2630

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2630

A.9.2.17.1	Test Purpose and Environment	2630

A.9.2.17.2	Test parameters	2630

A.9.2.17.3	Test Requirements	2632

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2632

A.9.2.18.1	Test Purpose and Environment	2632

A.9.2.18.2	Test parameters	2632

A.9.2.18.3	Test Requirements	2635

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2635

A.9.2.19.1	Test Purpose and Environment	2635

A.9.2.19.2	Test parameters	2635

A.9.2.19.3	Test Requirements	2637

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2637

A.9.2.20.1	Test Purpose and Environment	2637

A.9.2.20.2	Test parameters	2637

A.9.2.20.3	Test Requirements	2640

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2640

A.9.2.21.1	Test Purpose and Environment	2640

A.9.2.21.2	Test parameters	2640

A.9.2.21.3	Test Requirements	2643

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2643

A.9.2.22.1	Test Purpose and Environment	2644

A.9.2.22.2	Test parameters	2644

A.9.2.22.3	Test Requirements	2644

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2644

A.9.2.23.1	Test Purpose and Environment	2644

A.9.2.23.2	Test parameters	2644

A.9.2.23.3	Test Requirements	2647

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2647

A.9.2.24.1	Test Purpose and Environment	2647

A.9.2.24.2	Test parameters	2647

A.9.2.24.3	Test Requirements	2648

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2648

A.9.2.25.1	Test Purpose and Environment	2648

A.9.2.25.2	Test parameters	2648

A.9.2.25.3	Test Requirements	2653

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2653

A.9.2.26.1	Test Purpose and Environment	2653

A.9.2.26.2	Test parameters	2653

A.9.2.26.3	Test Requirements	2657

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2657

A.9.2.27.1	Test Purpose and Environment	2657

A.9.2.27.2	Test parameters	2657

A.9.2.27.3	Test Requirements	2658

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2658

A.9.2.28.1	Test Purpose and Environment	2658

A.9.2.28.2	Test parameters	2658

A.9.2.28.3	Test Requirements	2661

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2661

A.9.2.29.1	Test Purpose and Environment	2661

A.9.2.29.2	Test parameters	2661

A.9.2.29.3	Test Requirements	2664

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2664

A.9.2.30.1	Test Purpose and Environment	2664

A.9.2.30.2	Test parameters	2664

A.9.2.30.3	Test Requirements	2667

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2667

A.9.2.31.1	Test Purpose and Environment	2667

A.9.2.31.2	Test parameters	2667

A.9.2.31.3	Test Requirements	2670

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2670

A.9.2.32.1	Test Purpose and Environment	2670

A.9.2.32.2	Test parameters	2670

A.9.2.32.3	Test Requirements	2673

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2673

A.9.2.33.1	Test Purpose and Environment	2673

A.9.2.33.2	Test parameters	2673

A.9.2.33.3	Test Requirements	2677

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2677

A.9.2.34.1	Test Purpose and Environment	2677

A.9.2.34.2	Test parameters	2677

A.9.2.34.3	Test Requirements	2680

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2680

A.9.2.35.1	Test Purpose and Environment	2680

A.9.2.35.2	Test parameters	2680

A.9.2.35.3	Test Requirements	2683

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2683

A.9.2.36.1	Test Purpose and Environment	2683

A.9.2.36.2	Test parameters	2683

A.9.2.36.3	Test Requirements	2684

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2685

A.9.2.37.1	Test Purpose and Environment	2685

A.9.2.37.2	Test parameters	2685

A.9.2.37.3	Test Requirements	2686

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2687

A.9.2.38.1	Test Purpose and Environment	2687

A.9.2.38.2	Test parameters	2687

A.9.2.38.3	Test Requirements	2691

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2691

A.9.2.39.1	Test Purpose and Environment	2691

A.9.2.39.2	Test parameters	2691

A.9.2.39.3	Test Requirements	2695

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2695

A.9.2.40.1	Test Purpose and Environment	2695

A.9.2.40.2	Test parameters	2695

A.9.2.40.3	Test Requirements	2698

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2699

A.9.2.41.1	Test Purpose and Environment	2699

A.9.2.41.2	Test parameters	2699

A.9.2.41.3	Test Requirements	2702

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2702

A.9.2.42.1	Test Purpose and Environment	2702

A.9.2.42.2	Test parameters	2702

A.9.2.42.3	Test Requirements	2705

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2705

A.9.2.43.1	Test Purpose and Environment	2705

A.9.2.43.2	Test parameters	2705

A.9.2.43.3	Test Requirements	2708

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2708

A.9.2.44.1	Test Purpose and Environment	2708

A.9.2.44.2	Test parameters	2708

A.9.2.44.3	Test Requirements	2710

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2710

A.9.2.45.1	Test Purpose and Environment	2710

A.9.2.45.2	Test parameters	2710

A.9.2.45.3	Test Requirements	2714

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2715

A.9.2.46.1	Test Purpose and Environment	2715

A.9.2.46.2	Test parameters	2715

A.9.2.46.3	Test Requirements	2719

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.47.1	Test Purpose and Environment	2720

A.9.2.47.2	Test parameters	2720

A.9.2.47.3	Test Requirements	2724

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.48.1	Test Purpose and Environment	2724

A.9.2.48.2	Test parameters	2725

A.9.2.48.3	Test Requirements	2729

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2730

A.9.2.49.1	Test Purpose and Environment	2730

A.9.2.49.2	Test parameters	2730

A.9.2.49.3	Test Requirements	2736

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2737

A.9.2.50.1	Test Purpose and Environment	2737

A.9.2.50.2	Test parameters	2737

A.9.2.50.3	Test Requirements	2743

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2743

A.9.2.51.1	Test Purpose and Environment	2743

A.9.2.51.2	Test parameters	2744

A.9.2.51.3	Test Requirements	2748

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2748

A.9.2.52.1	Test Purpose and Environment	2748

A.9.2.52.2	Test parameters	2748

A.9.2.52.3	Test Requirements	2751

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2752

A.9.2.53.1	Test Purpose and Environment	2752

A.9.2.53.2	Test parameters	2752

A.9.2.53.3	Test Requirements	2758

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2759

A.9.2.54.1	Test Purpose and Environment	2759

A.9.2.54.2	Test parameters	2759

A.9.2.54.3	Test Requirements	2765

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2765

A.9.2.55.1	Test Purpose and Environment	2765

A.9.2.55.2	Test parameters	2766

A.9.2.55.3	Test Requirements	2771

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2771

A.9.2.56.1	Test Purpose and Environment	2771

A.9.2.56.2	Test parameters	2771

A.9.2.56.3	Test Requirements	2776

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2776

A.9.2.57.1	Test Purpose and Environment	2776

A.9.2.57.2	Test parameters	2777

A.9.2.57.3	Test Requirements	2781

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2782

A.9.2.58.1	Test Purpose and Environment	2782

A.9.2.58.2	Test parameters	2782

A.9.2.58.3	Test Requirements	2786

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2787

A.9.2.59.1	Test Purpose and Environment	2787

A.9.2.59.2	Test parameters	2787

A.9.2.59.3	Test Requirements	2792

A.9.3	UTRAN FDD CPICH RSCP	2793

A.9.3.1	E-UTRAN FDD	2793

A.9.3.1.1	Test Purpose and Environment	2793

A.9.3.1.2	Parameters	2793

A.9.3.1.3	Test Requirements	2795

A.9.3.2 E-UTRAN TDD	2795

A.9.3.2.1	Test Purpose and Environment	2795

A.9.3.2.2	Parameters	2795

A.9.3.2.3	Test Requirements	2798

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2798

A.9.3.3.1	Test Purpose and Environment	2798

A.9.3.3.2	Parameters	2798

A.9.3.3.3	Test Requirements	2799

A.9.4	UTRAN FDD CPICH Ec/No	2800

A.9.4.1	E-UTRAN FDD	2800

A.9.4.1.1	Test Purpose and Environment	2800

A.9.4.1.2	Parameters	2800

A.9.4.1.3	Test Requirements	2802

A.9.4.2	E-UTRAN TDD	2803

A.9.4.2.1	Test Purpose and Environment	2803

A.9.4.2.2	Parameters	2803

A.9.4.2.3	Test Requirements	2805

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2806

A.9.4.3.1	Test Purpose and Environment	2806

A.9.4.3.2	Parameters	2806

A.9.4.3.3	Test Requirements	2807

A.9.5	UTRAN TDD measurement	2807

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2807

A.9.5.1.1	Test Purpose and Environment	2807

A.9.5.1.2	Test parameters	2807

A.9.5.1.3	Test Requirements	2809

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2809

A.9.5.2.1	Test Purpose and Environment	2809

A.9.5.2.2	Test parameters	2809

A.9.5.2.3	Test Requirements	2811

A.9.6	GSM Carrier RSSI	2812

A.9.6.1	E-UTRAN FDD	2812

A.9.6.1.1	Test Purpose and Environment	2812

A.9.6.1.2	Test Requirements	2813

A.9.6.2	E-UTRAN TDD	2813

A.9.6.2.1	Test Purpose and Environment	2813

A.9.6.2.2	Test Requirements	2815

A.9.7	UE Rx – Tx Time Difference	2816

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2816

A.9.7.1.1	Test Purpose and Environment	2816

A.9.7.1.2	Test parameters	2816

A.9.7.1.3	Test Requirements	2817

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2817

A.9.7.2.1	Test Purpose and Environment	2817

A.9.7.2.2	Test parameters	2817

A.9.7.2.3	Test Requirements	2819

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2819

A.9.7.3.1	Test Purpose and Environment	2819

A.9.7.3.2	Test parameters	2819

A.9.7.3.3	Test Requirements	2822

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2822

A.9.7.4.1	Test Purpose and Environment	2822

A.9.7.4.2	Test Parameters	2822

A.9.7.4.3	Test Requirements	2825

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2825

A.9.7.5.1	Test Purpose and Environment	2825

A.9.7.5.2	Test parameters	2825

A.9.7.5.3	Test Requirements	2828

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2828

A.9.7.6.1	Test Purpose and Environment	2828

A.9.7.6.2	Test Parameters	2828

A.9.7.6.3	Test Requirements	2831

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2831

A.9.7.7.1	Test Purpose and Environment	2831

A.9.7.7.2	Test parameters	2831

A.9.7.7.3	Test Requirements	2832

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2833

A.9.7.8.1	Test Purpose and Environment	2833

A.9.7.8.2	Test parameters	2833

A.9.7.8.3	Test Requirements	2834

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2834

A.9.7.9.1	Test Purpose and Environment	2834

A.9.7.9.2	Test parameters	2834

A.9.7.9.3	Test Requirements	2835

A.9.8	RSTD	2836

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2836

A.9.8.1.1	Test Purpose and Environment	2836

A.9.8.1.2	Test Requirements	2838

A.9.8.1.2A	Test Requirements for UE Category 1bis	2838

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2839

A.9.8.2.1	Test Purpose and Environment	2839

A.9.8.2.2	Test Requirements	2842

A.9.8.2.2A	Test Requirements for UE Category 1bis	2842

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2843

A.9.8.3.1	Test Purpose and Environment	2843

A.9.8.3.2	Test Requirements	2845

A.9.8.3.2A	Test Requirements for UE Category 1bis	2845

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2846

A.9.8.4.1	Test Purpose and Environment	2846

A.9.8.4.2	Test Requirements	2848

A.9.8.4.2A	Test Requirements for UE Category 1bis	2848

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2849

A.9.8.5.1	Test Purpose and Environment	2849

A.9.8.5.2	Test Requirements	2851

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2851

A.9.8.6.1	Test Purpose and Environment	2851

A.9.8.6.2	Test Requirements	2854

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2854

A.9.8.7.1	Test Purpose and Environment	2854

A.9.8.7.2	Test Requirements	2855

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2855

A.9.8.8.1	Test Purpose and Environment	2855

A.9.8.8.2	Test Requirements	2856

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2856

A.9.8.9.1	Test Purpose and Environment	2856

A.9.8.9.2	Test Requirements	2857

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2857

A.9.8.10.1	Test Purpose and Environment	2857

A.9.8.10.2	Test Requirements	2858

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2858

A.9.8.11.1	Test Purpose and Environment	2858

A.9.8.11.2	Test Requirements	2859

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2859

A.9.8.12.1	Test Purpose and Environment	2859

A.9.8.12.2	Test Requirements	2860

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2860

A.9.8.13.1	Test Purpose and Environment	2860

A.9.8.13.2	Test Requirements	2861

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2861

A.9.8.14.1	Test Purpose and Environment	2861

A.9.8.14.2	Test Requirements	2868

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2868

A.9.8.15.1	Test Purpose and Environment	2868

A.9.8.15.2	Test Requirements	2874

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2874

A.9.8.16.1	Test Purpose and Environment	2874

A.9.8.16.2	Test Requirements	2877

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2877

A.9.8.17.1	Test Purpose and Environment	2877

A.9.8.17.2	Test Requirements	2880

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2880

A.9.8.18.1	Test Purpose and Environment	2880

A.9.8.18.2	Test Requirements	2883

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2883

A.9.8.19.1	Test Purpose and Environment	2883

A.9.8.19.2	Test Requirements	2886

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2886

A.9.8.20.1	Test Purpose and Environment	2886

A.9.8.20.2	Test Requirements	2889

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2889

A.9.8.21.1	Test Purpose and Environment	2889

A.9.8.21.2	Test Requirements	2893

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2894

A.9.8.22.1	Test Purpose and Environment	2894

A.9.8.22.2	Test Requirements	2897

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2898

A.9.8.23.1	Test Purpose and Environment	2898

A.9.8.23.2	Test Requirements	2901

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2901

A.9.8.24.1	Test Purpose and Environment	2901

A.9.8.24.2	Test Requirements	2905

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2906

A.9.8.25.1	Test Purpose and Environment	2906

A.9.8.25.2	Test Requirements	2909

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2910

A.9.8.26.1	Test Purpose and Environment	2910

A.9.8.26.2	Test Requirements	2912

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2912

A.9.8.27.1	Test Purpose and Environment	2912

A.9.8.27.2	Test Requirements	2915

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2915

A.9.8.28.1	Test Purpose and Environment	2915

A.9.8.28.2	Test Requirements	2918

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2918

A.9.8.29.1	Test Purpose and Environment	2918

A.9.8.29.2	Test Requirements	2922

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2923

A.9.8.30.1	Test Purpose and Environment	2923

A.9.8.30.2	Test Requirements	2926

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2927

A.9.8.31.1	Test Purpose and Environment	2927

A.9.8.31.2	Test Requirements	2931

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2931

A.9.8.32.1	Test Purpose and Environment	2931

A.9.8.32.2	Test Requirements	2934

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2934

A.9.8.33.1	Test Purpose and Environment	2934

A.9.8.33.2	Test Requirements	2937

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2937

A.9.8.34.1	Test Purpose and Environment	2937

A.9.8.34.2	Test Requirements	2940

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2940

A.9.8.35.1	Test Purpose and Environment	2940

A.9.8.35.2	Test Requirements	2943

A.9.9	RSRP and RSRQ on the serving cell	2943

A.9.9.1	FDD Intra frequency serving cell case	2943

A.9.9.1.1	Test Purpose and Environment	2943

A.9.9.1.2	Test parameters	2943

A.9.9.1.3	Test Requirements	2946

A.9.9.2	TDD Intra frequency serving cell case	2946

A.9.9.2.1	Test Purpose and Environment	2946

A.9.9.2.2	Test parameters	2946

A.9.9.2.3	Test Requirements	2948

A.9.10	SSTD	2948

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2948

A.9.10.1.1	Test Purpose and Environment	2948

A.9.10.1.2	Test parameters	2948

A.9.10.1.3	Test Requirements	2950

A.9.10.2	Void	2950

A.9.10.3	Void	2950

A.9.10.4	Void	2950

A.9.11 RSSI	2950

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2950

A.9.11.1.1	Test Purpose and Environment	2950

A.9.11.1.2	Test parameters	2950

A.9.11.1.3	Test Requirements	2953

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2953

A.9.11.2.1	Test Purpose and Environment	2953

A.9.11.2.2	Test parameters	2953

A.9.12	Channel occupancy	2956

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2956

A.9.12.1.1	Test Purpose and Environment	2956

A.9.12.1.2	Test parameters	2956

A.9.12.1.3	Test Requirements	2959

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2959

A.9.12.2.1	Test Purpose and Environment	2959

A.9.12.2.2	Test parameters	2959

A.9.12.2.3	Test Requirements	2962

A.9.13	RS-SINR	2962

A.9.13.1	FDD Intra-Frequency Case	2962

A.9.13.1.1	Test Purpose and Environment	2962

A.9.13.1.2	Test parameters	2962

A.9.13.1.3	Test Requirements	2965

A.9.13.2	TDD Intra-Frequency Case	2965

A.9.13.2.1	Test Purpose and Environment	2965

A.9.13.2.2	Test parameters	2965

A.9.13.2.3	Test Requirements	2968

A.9.13.3	FDD—FDD Inter frequency case	2968

A.9.13.3.1	Test Purpose and Environment	2968

A.9.13.3.2	Test parameters	2968

A.9.13.3.3	Test Requirements	2971

A.9.13.4	TDD—TDD Inter frequency case	2971

A.9.13.4.1	Test Purpose and Environment	2971

A.9.13.4.2	Test parameters	2971

A.9.13.4.3	Test Requirements	2975

A.9.13.5	FDD—TDD Inter frequency case	2976

A.9.13.5.1	Test Purpose and Environment	2976

A.9.13.5.2	Test parameters	2976

A.9.13.5.3	Test Requirements	2982

A.9.13.6	TDD—FDD Inter frequency case	2982

A.9.13.6.1	Test Purpose and Environment	2982

A.9.13.6.2	Test parameters	2982

A.9.13.6.3	Test Requirements	2986

A.9.14	Channel quality reporting accuracy	2986

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	2986

A.9.14.1.1	Test Purpose and Environment	2986

A.9.14.1.2	Test parameters	2987

A.9.14.1.3	Test Requirements	2987

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	2988

A.9.14.2.1	Test Purpose and Environment	2988

A.9.14.2.2	Test parameters	2988

A.9.14.2.3	Test Requirements	2989

A.10	Proximity-based Services in Any Cell Selection State	2989

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2989

A.10.1.1	Test Purpose and Environment	2989

A.10.1.2	Test Requirements	2990

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2991

A.10.2.1	Test Purpose and Environment	2991

A.10.2.2	Test Requirements	2992

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	2992

A.10.3.1	Test Purpose and Environment	2992

A.10.3.2	Test Requirements	2994

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	2995

A.10.4.1	Test Purpose and Environment	2995

A.10.4.2	Test Requirements	2997

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	2997

A.11.1	V2V UE Transmission Timing Accuracy Test	2997

A.11.1.1	Test Purpose and Environment	2997

A.11.1.2	Test requirements	2998

A.11.2	Interruptions due to V2V sidelink communication	2998

A.11.2.1	Test Purpose and Environment	2998

A.11.2.2	Test Requirements	3000

A.12	3000

A.12.1	V2X UE Transmission Timing Accuracy Test	3000

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3000

A.12.1.1.1	Test Purpose and Environment	3000

A.12.1.1.2	Test requirements	3001

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3002

A.12.1.2.1	Test Purpose and Environment	3002

A.12.1.2.2	Test Requirements	3002

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3003

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3003

A.12.2.1.1	Test Purpose and Environment	3003

A.12.2.1.2	Test Requirements	3004

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3005

A.12.2.2.1	Test Purpose and Environment	3005

A.12.2.2.2	Test Requirements	3006

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3006

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3006

A.12.3.1.1	Test Purpose and Environment	3006

A.12.3.1.2	Test Requirements	3009

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3010

A.12.3.2.1	Test Purpose and Environment	3010

A.12.3.1.2	Test Requirements	3012

A.12.4	Congestion Control Measurement Test for V2X UE	3012

A.12.4.1	Test Purpose and Environment	3012

A.12.4.2	Test Requirements	3014

A.12.5	Interruptions due to V2X Sidelink Communication	3014

A.12.5.1	Test Purpose and Environment	3014

A.12.5.2	Test Requirements	3016

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3016

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3016

A.12.6.1.1	Test Purpose and Environment	3016

A.12.6.1.2	Test Requirements	3018

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3018

A.12.6.2.1	Test Purpose and Environment	3018

A.12.6.1.2	Test Requirements	3020

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3020

A.12.7.1	Test Purpose and Environment	3020

A.12.7.2	Test Requirements	3023

A.12.8	Interruptions due to V2X Carrier Aggregation	3024

A.12.8.1	Interruptions on a FDD PCell	3024

A.12.8.1.1	Test Purpose and Environment	3024

A.12.8.1.2	Test Requirements	3025

A.12.8.2	Interruptions on a TDD PCell	3025

A.12.8.2.1	Test Purpose and Environment	3025

A.12.8.2.2	Test Requirements	3027

Annex B (normative): Conditions for RRM requirements applicability for operating bands	3028

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3028

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3028

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3028

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3028

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3030

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3031

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3031

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3031

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3031

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3033

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3034

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3034

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3034

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3035

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3035

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3036

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3036

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3036

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3037

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3037

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3038

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3038

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3038

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3038

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3039

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3039

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3039

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3040

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3040

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3040

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3041

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3041

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3042

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3043

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3044

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3045

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3046

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3046

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3046

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3046

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3047

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3047

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3049

B.3	Conditions for measurements performance requirements for UE	3049

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3049

B.3.2	Void	3049

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3049

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3049

B.3.5	Conditions for UE Rx – Tx time difference	3049

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3050

B.3.7	Conditions for inter-frequency RSTD measurements	3050

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3050

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3050

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3050

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3051

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3051

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3051

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3051

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3051

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3051

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3052

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3052

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3052

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3053

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3053

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3053

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3053

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3053

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3053

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3053

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3053

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3053

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.21.1	Conditions for RSRP measurements	3054

B.3.21.2	Conditions for RSRQ measurements	3054

B.3.21.3	Conditions for CSI-RSRP measurements	3054

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3054

B.3.22.1	Conditions for RSRP measurements	3054

B.3.22.2	Void	3055

B.3.22.3	Conditions for CSI-RSRP measurements	3055

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3055

B.3.23.1	Conditions for RSRP measurements	3055

B.3.23.2	Conditions for RSRQ measurements	3055

B.3.23.3	Conditions for CSI-RSRP measurements	3055

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3056

B.3.24.1	Conditions for RSRP measurements	3056

B.3.24.2	Conditions for RSRQ measurements	3056

B.3.24.3	Conditions for CSI-RSRP measurements	3056

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3056

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3056

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3056

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3057

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3057

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3057

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3057

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3057

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3058

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3058

B.4	RRM Requirements Exceptions	3058

B.4.1	General	3058

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3058

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3058

B.4.3.1	Inter-band carrier aggregation	3058

B.4.3.2	Intra-band non-contiguous carrier aggregation	3058

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3059

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3059

B.5.1	Conditions for S-RSRP Accuracy Requirements	3059

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3059

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3060

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3060

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3061

B.6	Conditions for V2X	3061

B.6.1	Test parameters for GNSS signals	3061

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3061

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3061

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3062

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3062

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3063

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3063

Annex C (informative): Change history:	3064