| 3GPP TS 36.133 V17.0.0 (2020-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	89

1	Scope	91

2	References	91

3	Definitions, symbols and abbreviations	93

3.1	Definitions	93

3.2	Symbols	95

3.3	Abbreviations	96

3.4	Test tolerances	99

3.5	Additional notation	99

3.5.1	Groups of bands	99

3.6	General	101

3.6.1	Applicability of requirements in this specification version	101

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	107

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	109

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	110

3.6.2	Applicability of requirements for EN-DC operation	111

3.6.3	Applicability of requirements for NE-DC operation	112

3.6.4	Applicability of requirements for NGEN-DC operation	113

4	E-UTRAN RRC_IDLE state mobility	113

4.1	Cell Selection	113

4.2	Cell Re-selection	113

4.2.1	Introduction	113

4.2.2	Requirements	114

4.2.2.1	Measurement and evaluation of serving cell	115

4.2.2.2	Void	116

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	116

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	117

4.2.2.5	Measurements of inter-RAT cells	120

4.2.2.5.1	Measurements of UTRAN FDD cells	120

4.2.2.5.2	Measurements of UTRAN TDD cells	121

4.2.2.5.3	Measurements of GSM cells	123

4.2.2.5.4	Measurements of HRPD cells	124

4.2.2.5.5	Measurements of cdma2000 1X	125

4.2.2.5.6	Measurements of NR cells	126

4.2.2.5.7	Measurements of NR cells subject to CCA	128

4.2.2.6	Evaluation of cell re-selection criteria	129

4.2.2.7	Maximum interruption in paging reception	130

4.2.2.8	void	130

4.2.2.9	UE measurement capability	130

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	131

4.2.2.10	Reselection to CSG cells	131

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	132

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	132

4.2.2.11	Void	133

4.2.2.12	Void	133

4.2.2.13	Void	133

4.3	Minimization of Drive Tests (MDT)	133

4.3.1	Introduction	134

4.3.2	Measurements	134

4.3.2.1	Requirements	134

4.3.3	Relative Time Stamp Accuracy	134

4.3.3.1	Requirements	134

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	134

4.3.4.1	 Requirements	134

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	135

4.3.5.1	Requirements for timeSinceFailure	135

4.4	MBSFN Measurements	135

4.4.1	Introduction	135

4.4.2	MBSFN RSRP measurements	135

4.4.3	MBSFN RSRQ measurements	135

4.4.4	MCH BLER measurements	136

4.5	Proximity-based Services	136

4.5.1	Introduction	136

4.5.2	Requirements	136

4.5.2.1	Interruptions with ProSe Direct Discovery	136

4.5.2.2	Interruptions with ProSe Direct Communication	136

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	136

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	137

4.6	Cell Selection and Re-selection Requirements for UE category NB1	137

4.6.1	Cell Selection	137

4.6.2	Cell Re-selection	137

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	138

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	139

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	140

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	141

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	142

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	143

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	145

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	146

4.6.2.7	Maximum interruption in paging reception in normal coverage	147

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	148

4.6.2.8	UE measurement capability	148

4.6.2.9	WUS receptions for NB1	148

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	148

4.6.3.1	Introduction	148

4.6.3.2	Requirements on UE synchronization for transmission using PUR	148

4.6.3.3	Requirements on TA validation for transmission using PUR	149

4.7	Cell Selection and Re-selection Requirements for UE category M1	149

4.7.1	Cell Selection	150

4.7.2	Cell Re-selection	150

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	150

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	150

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	151

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	153

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	155

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	155

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	156

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	156

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	157

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	158

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	160

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	162

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in extended coverage	163

4.7.2.3	WUS receptions for UE category M1	163

4.7.3	Channel quality report for UE Category M1 in idle mode	163

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	164

4.7.4.1	Introduction	164

4.7.4.2	Requirements on UE synchronization for transmission using PUR	164

4.7.4.3	Requirements on TA validation for transmission using PUR	164

4.8	Idle State Positioning Measurement Requirements for UE category NB1	165

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	165

4.8.1.1	RSTD Measurement Reporting Delay	167

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	167

4.8.2.1	RSTD Measurement Reporting Delay	168

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	169

4.8.3.1	RSTD Measurement Reporting Delay	170

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	171

4.8.4.1	RSTD Measurement Reporting Delay	172

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	173

4.8.5.1	Measurement Reporting Delay	174

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	174

4.8.6.1	Measurement Reporting Delay	175

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	176

4.8.7.1	Measurement Reporting Delay	177

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	177

4.8.8.1	Measurement Reporting Delay	178

4.9	Idle Mode CA Measurement	179

4.9.1	Introduction	179

4.9.2	Requirement	179

4.9.2.1	Detected cell requirement during state transition and Idle mode	179

4.9.2.2	Measurements of inter-frequency CA candidate cells	180

4.9.2.3	Measurements on serving cell	180

4A	E-UTRAN RRC_INACTIVE state mobility	181

4A.1	Cell Re-selection	181

4A.1.1	Introduction	181

4A.1.2	Requirements	182

4A.1.2.1	UE measurement capability	182

4A.1.2.2	Measurement and evaluation of serving cell	182

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	182

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	182

4A.1.2.5	Evaluation of cell re-selection criteria	182

4A.1.2.6	Maximum interruption in paging reception	182

4A.1.2.7	Measurements of inter-RAT NR cells	182

4A.2	Requirements for UE Category M1	182

4A.2.1	Introduction	182

4A.2.2	Cell Selection	182

4A.2.3	Cell Reselection	182

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	182

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	182

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	182

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	182

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	183

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	183

4A.2.4	Channel quality report for UE Category M1 in idle mode	183

5	E-UTRAN RRC_CONNECTED state mobility	183

5.1	E-UTRAN Handover	184

5.1.1	Introduction	184

5.1.2	Requirements	184

5.1.2.1	E-UTRAN FDD – FDD	184

5.1.2.1.1	Handover delay	184

5.1.2.1.2	Interruption time	184

5.1.2.2	E-UTRAN FDD – TDD	186

5.1.2.2.1	(Void)	186

5.1.2.2.2	(Void)	186

5.1.2.3	E-UTRAN TDD – FDD	186

5.1.2.3.1	(Void)	186

5.1.2.3.2	(Void)	186

5.1.2.4	E-UTRAN TDD – TDD	186

5.1.2.4.1	Handover delay	186

5.1.2.4.2	Interruption time	186

5.1.2.5	E-UTRAN HD–FDD	188

5.1.2.5.1	Handover delay	188

5.1.2.5.2	Interruption time	188

5.1.2.6	E-UTRAN FDD – FDD conditional handover	189

5.1.2.6.1	Handover delay	189

5.1.2.6.2	Measurement time	190

5.1.2.6.3	Preparation time	190

5.1.2.6.4	Interruption time	190

5.1.2.7	E-UTRAN FDD – TDD conditional handover	191

5.1.2.8	E-UTRAN TDD – FDD conditional handover	191

5.1.2.9	E-UTRAN TDD – TDD conditional handover	191

5.2	Void	191

5.3	Handover to other RATs	191

5.3.1	E-UTRAN - UTRAN FDD Handover	191

5.3.1.1	Introduction	191

5.3.1.1.1	Handover delay	191

5.3.1.1.2	Interruption time	191

5.3.2	E-UTRAN - UTRAN TDD Handover	192

5.3.2.1	Introduction	192

5.3.2.2	Requirements	192

5.3.2.2.1	Handover delay	192

5.3.2.2.2	Interruption time	192

5.3.3	E-UTRAN - GSM Handover	193

5.3.3.1	Introduction	193

5.3.3.2	Requirements	193

5.3.3.2.1	Handover delay	193

5.3.3.2.2	Interruption time	193

5.3.4	E-UTRAN - NR FR1 Handover	194

5.3.4.1	Introduction	194

5.3.4.2	Handover delay	194

5.3.4.3	Interruption time	194

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	194

5.3.4A.1	Introduction	194

5.3.4A.2	Handover delay	195

5.3.4A.3	Interruption time	195

5.3.5	E-UTRAN - NR FR2 Handover	196

5.3.5.1	Introduction	196

5.3.5.2	Handover delay	196

5.3.5.3	Interruption time	196

5.4	Handover to Non-3GPP RATs	197

5.4.1	E-UTRAN – HRPD Handover	197

5.4.1.1	Introduction	197

5.4.1.1.1	Handover delay	197

5.4.1.1.2	Interruption time	197

5.4.2	E-UTRAN – cdma2000 1X Handover	198

5.4.2.1	Introduction	198

5.4.2.1.1	Handover delay	198

5.4.2.1.2	Interruption time	198

5.5	E-UTRAN Handover for Cat-M1 UEs	198

5.5.1	Introduction	198

5.5.2	Requirements in CEModeA	198

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	198

5.5.2.1.1	Handover delay	199

5.5.2.1.2	Interruption time	199

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	199

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	199

5.5.2.3.1	Void	200

5.5.2.3.2	Void	200

5.5.3	Requirements in CEModeB	200

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	200

5.5.3.1.1	Handover delay	200

5.5.3.1.2	Interruption time	200

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	201

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	201

5.6	Void	201

5.7	E-UTRAN DAPS Handover	201

5.7.1	Introduction	201

5.7.2	Requirements	201

5.7.2.1	E-UTRAN FDD – FDD	201

5.7.2.1.1	DAPS Handover delay	201

5.7.2.1.2	Interruption time	202

5.7.2.2	E-UTRAN FDD – TDD	202

5.7.2.3	E-UTRAN TDD – FDD	202

5.7.2.4	E-UTRAN TDD – TDD	203

6	RRC Connection Mobility Control	203

6.1	RRC Re-establishment	203

6.1.1	Introduction	203

6.1.2	Requirements	203

6.1.2.1	UE Re-establishment delay requirement	203

6.2	Random Access	204

6.2.1	Introduction	204

6.2.2	Requirements	204

6.2.2.1	Contention based random access	204

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	204

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	204

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	204

6.2.2.1.4	Void	204

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	204

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	204

6.2.2.2	Non-Contention based random access	205

6.2.2.2.1	Correct behaviour when receiving Random Access Response	205

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	205

6.2.3	Requirements for Cat-M1 UEs	205

6.3	RRC Connection Release with Redirection	205

6.3.1	Introduction	205

6.3.2	Requirements	205

6.3.2.1	RRC connection release with redirection to UTRAN FDD	205

6.3.2.2	RRC connection release with redirection to GERAN	206

6.3.2.3	RRC connection release with redirection to UTRAN TDD	206

6.3.2.4	RRC connection release with redirection to NR	207

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	207

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	208

6.4.1	Introduction	208

6.4.2	Requirements	209

6.5	RRC Re-establishment for NB-IoT UEs	209

6.5.1	Introduction	209

6.5.2	Requirements	209

6.5.2.1	UE Re-establishment delay requirement in normal coverage	209

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	210

6.6	Random Access for UE category NB1	210

6.6.1	Introduction	210

6.6.2	Requirements	210

6.6.2.1	Correct behaviour when receiving Random Access Response reception	210

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	211

6.6.2.3	Correct behaviour when receiving a NACK on msg3	211

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	211

6.6.2.5	Correct behaviour when contention Resolution timer expires	211

6.6.2.6	MSG3-based channel quality report for UE Category NB1	211

6.6.3	Requirements for NPRACH configuration	212

6.7	RRC Re-establishment for Cat-M1 UEs	212

6.7.1	Introduction	212

6.7.2	Requirements	212

6.7.2.1	UE Re-establishment delay requirement for CEModeA	212

6.7.2.2	UE Re-establishment delay requirement for CEModeB	213

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	213

6.8.1	Introduction	213

6.8.2	Requirements	213

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	213

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	214

6.9.1	Introduction	214

6.9.2	Requirements	214

7	Timing and signalling characteristics	215

7.1	UE transmit timing	215

7.1.1	Introduction	215

7.1.2	Requirements	215

7.2	UE timer accuracy	217

7.2.1	Introduction	217

7.2.2	Requirements	217

7.3	Timing Advance	217

7.3.1	Introduction	217

7.3.2	Requirements	217

7.3.2.1	Timing Advance adjustment delay	217

7.3.2.2	Timing Advance adjustment accuracy	218

7.4	Cell phase synchronization accuracy (TDD)	218

7.4.1	Definition	218

7.4.2	Minimum requirements	218

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	218

7.5.1	Introduction	218

7.5.2	eNodeB Synchronization Requirements	219

7.5.2.1	Synchronized E-UTRAN	219

7.5.2.2	Non-Synchronized E-UTRAN	219

7.6	Radio Link Monitoring	219

7.6.1	Introduction	219

7.6.2	Requirements	221

7.6.2.1	Minimum requirement when no DRX is used	221

7.6.2.2	Minimum requirement when DRX is used	221

7.6.2.3	Minimum requirement at transitions	222

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	223

7.6.2.5	Minimum requirement under IDC Interference	223

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	223

7.7.1	Introduction	223

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	223

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	225

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	226

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	227

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	228

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	229

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	229

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	229

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	230

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	231

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	232

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	233

7.7.14	SCell Activation Delay Requirement for Dormant SCell	233

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	234

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	235

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	237

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	237

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	239

7.8	Interruptions with Carrier Aggregation	241

7.8.1	Introduction	241

7.8.2	Requirements	241

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	241

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	241

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	241

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	241

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	242

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	242

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	242

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	242

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	243

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	244

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	244

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	244

7.8.2.13	Interruptions at SRS carrier based switching	245

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	245

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	246

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	246

7.8.2.17	Interruptions during CQI measurement on dormant SCell	246

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	246

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	247

7.8.2.20	Interruptions at SCell hibernation	247

7.8.2.21	Interruptions at direct SCell activation and hibernation	247

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	248

7.9.1	Introduction	248

7.9.2	Minimum Requirements for Interband Carrier Aggregation	248

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	248

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	249

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	249

7.10.1	Introduction	249

7.10.2	Requirements	249

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	249

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	249

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	250

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	250

7.11	Radio Link Monitoring for UE Category 0	250

7.11.1	Introduction	250

7.11.2	Requirements for FD-FDD and TDD	252

7.11.2.1	Minimum requirement when no DRX is used	252

7.11.2.2	Minimum requirement when DRX is used	252

7.11.2.3	Minimum requirement at transitions	253

7.11.3	Requirements for HD-FDD	253

7.11.3.1	Minimum requirement when no DRX is used	253

7.11.3.2	Minimum requirement when DRX is used	253

7.11.3.3	Minimum requirement at transitions	254

7.12	Interruptions with Dual Connectivity	254

7.12.1	Introduction	254

7.12.2	Requirements	254

7.12.2.1	Interruptions at PSCell addition/release	254

7.12.2.2	Interruptions at transitions between active and non-active during DRX	255

7.12.2.3	Interruptions at transitions from non-DRX to DRX	255

7.12.2.4	Interruptions at SCell addition/release	255

7.12.2.5	Interruptions at SCell activation/deactivation	255

7.12.2.6	Interruptions during measurements on SCC	256

7.12.2.7	Interruptions at SRS carrier based switching	256

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	257

7.13.1	Definition	257

7.13.2	Minimum requirements	257

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	257

7.14.1	Introduction	257

7.14.2	PSCell Addition Delay Requirement	257

7.14.3	PSCell Release Delay Requirement	258

7.15	Maximum Receive Timing Difference in Dual Connectivity	258

7.15.1	Introduction	258

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	258

7.16	Proximity-based Services	259

7.16.1	Introduction	259

7.16.2	Requirements	259

7.16.2.1	ProSe UE transmission timing	259

7.16.2.1.1	Serving cell or PCell as timing reference	259

7.16.2.1.2	SCell or non-serving cell as timing reference	259

7.16.3	Interruptions with ProSe	259

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	260

7.16.3.2	Interruptions at ProSe Direct Communication configuration	260

7.16.3.3	Interruptions during ProSe Direct Discovery	260

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	260

7.16.3.5	Interruptions during ProSe Direct Communication	261

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	261

7.16.4.1	Measurement and evaluation of selected cell	261

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	261

7.16.5	Selection / Reselection of ProSe relay UE	262

7.16.6	ProSe operation under deactivated SCell	262

7.17	Maximum Transmission Timing Difference in Dual Connectivity	263

7.17.1	Introduction	263

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	263

7.18.1	Introduction	263

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	263

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	263

7.19	Radio Link Monitoring for UE Category M1	263

7.19.1	Introduction	263

7.19.2	Requirements for FD-FDD and TDD CE mode A	264

7.19.2.1	Minimum requirement when no DRX is used	266

7.19.2.2	Minimum requirement when DRX is used	267

7.19.2.3	Minimum requirement at transitions	268

7.19.3	Requirements for HD-FDD with CE mode A	268

7.19.3.1	Minimum requirement when no DRX is used	268

7.19.3.2	Minimum requirement when DRX is used	268

7.19.3.3	Minimum requirement at transitions	270

7.19.4	Requirements for FD-FDD and TDD with CE mode B	270

7.19.4.1	Minimum requirement when no DRX is used	272

7.19.4.2	Minimum requirement when DRX is used	273

7.19.4.3	Minimum requirement at transitions	274

7.19.5	Requirements for HD-FDD with CE mode B	274

7.19.5.1	Minimum requirement when no DRX is used	274

7.19.5.2	Minimum requirement when DRX is used	274

7.19.5.3	Minimum requirement at transitions	276

7.20	UE transmit timing for NB-IoT	276

7.20.1	Introduction	276

7.20.2	Requirements	276

7.21	 UE timer accuracy for NB-IoT	277

7.21.1	Introduction	277

7.21.2	Requirements	277

7.22	Timing Advance for NB-IoT	277

7.22.1	Introduction	277

7.22.2	Requirements	277

7.22.2.1	Timing Advance adjustment delay	277

7.22.2.2	Timing Advance adjustment accuracy	277

7.23	Radio Link Monitoring for Category NB1 UE	277

7.23.1	Introduction	277

7.23.2	Requirements for Category NB1 UE	277

7.23.2.1	Minimum requirement when no DRX is used	278

7.23.2.2	Minimum requirement when DRX is used	278

7.23.2.3	Minimum requirement at transitions	279

7.24	UE transmit timing for Category M1	279

7.24.1	Introduction	279

7.24.2	Requirements	279

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	280

7.25.1	Definition	280

7.25.2	Minimum requirements	280

7.26	UE transmit timing for Category M2	281

7.26.1	Introduction	281

7.26.2	Requirements	281

7.27	UE timer accuracy for category M1	281

7.27.1	Introduction	281

7.27.2	Requirements	281

7.28	Timing Advance for Category M1	282

7.28.1	Introduction	282

7.28.2	Requirements	282

7.29	Interruptions requirements with FeMBMS	282

7.29.1	Introduction	282

7.29.2	Requirements	282

7.30	Numerology switching delay requirements with FeMBMS	282

7.30.1	Introduction	282

7.30.2	Requirements	282

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	282

7.31.1	Introduction	282

7.31.2	NR PSCell Addition Delay Requirement	283

7.31.3	NR PSCell Release Delay Requirement	283

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	284

7.31A.1	Introduction	284

7.31A.2	NR PSCell Addition Delay Requirement	284

7.31A.3	NR PSCell Release Delay Requirement	285

7.32	Interruptions with EN-DC	285

7.32.1	Introduction	285

7.32.2	Requirements	286

7.32.2.1	Interruptions at PSCell addition/release	286

7.32.2.2	Interruptions at transitions between active and non-active during DRX	286

7.32.2.3	Interruptions at transitions from non-DRX to DRX	286

7.32.2.4	Interruptions at SCell addition/release	286

7.32.2.5	Interruptions at SCell activation/deactivation	286

7.32.2.6	Interruptions during measurements on SCC	287

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	287

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	287

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	287

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	287

7.32.2.7	Interruptions at active BWP switching	287

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	288

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	288

7.32.2.10	Interruptions at SCell hibernation	288

7.32.2.11	Interruptions at direct SCell activation and hibernation	289

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	289

7.32.2.13	Interruptions at NR SRS carrier based switching	289

7.32.2.14	Interruptions at NR SCell dormancy	290

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	290

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	290

7.32.2.15	Interruption during NR measurement with autonomous gaps	290

7.32.2.16	Interruptions at SRS carrier based switching	291

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	291

7.33.1	Introduction	291

7.33.2	Requirements	291

7.34	Void	292

7.35	Interruptions with SFTD measurements	292

7.35.1	Introduction	292

7.35.2	Requirements	292

7.36	Interruptions with NE-DC	292

7.36.1	Introduction	292

7.36.2	Requirements	293

7.36.2.1	Interruptions at transitions between active and non-active during DRX	293

7.36.2.2	Interruptions at transitions from non-DRX to DRX	293

7.36.2.3	Interruptions at SCell addition/release	293

7.36.2.4	Interruptions at SCell activation/deactivation	293

7.36.2.5	Interruptions during measurements on SCC	294

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	294

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	294

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	294

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	294

7.36.2.6	Interruptions at active BWP switching	294

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	295

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	295

7.36.2.9	Interruptions at SCell hibernation	295

7.36.2.10	Interruptions at direct SCell activation and hibernation	295

7.36.2.11	Interruptions at NR SRS carrier based switching	296

7.36.2.12	Interruptions at NR SCell dormancy	296

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	296

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	297

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	297

7.36.2.14	Interruption during NR measurement with autonomous gaps	297

7.36.2.15	Interruptions at SRS carrier based switching	298

7.37	Interruptions during NR measurement with autonomous gaps	298

7.37.1	Introduction	298

7.37.2	Requirements	298

8	UE Measurements Procedures in RRC_CONNECTED State	299

8.1	General Measurement Requirements	299

8.1.1	Introduction	299

8.1.2	Requirements	299

8.1.2.1	UE measurement capability	299

8.1.2.1.1	Monitoring of multiple layers using gaps	306

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	307

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	308

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	310

8.1.2.1.2	Network controlled small gap	311

8.1.2.2	E-UTRAN intra frequency measurements	312

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	312

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	317

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	321

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	322

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	323

8.1.2.3	E-UTRAN inter frequency measurements	324

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	324

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	330

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	336

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	336

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	337

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	338

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	339

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	340

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	341

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	348

8.1.2.4	Inter RAT measurements	348

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	348

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	353

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	353

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	357

8.1.2.4.5	E-UTRAN FDD – GSM measurements	357

8.1.2.4.6	E-UTRAN TDD – GSM measurements	362

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	362

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	364

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	364

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	364

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	365

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	365

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	365

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	365

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	367

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	367

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	367

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	367

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	368

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	368

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	370

8.1.2.4.21	E-UTRAN FDD – NR measurements	370

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	374

8.1.2.4.22	E-UTRAN TDD – NR measurements	378

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	378

8.1.2.4.23	Void	378

8.1.2.4.24	Void	378

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	378

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	380

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	380

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	381

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	381

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	382

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	383

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	385

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	387

8.1.2.6.5	Void	388

8.1.2.6.6	Void	388

8.1.2.6.7	Void	388

8.1.2.6.8	Void	388

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	388

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	389

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	390

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	392

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	394

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	396

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	397

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	399

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	401

8.1.2.7	E-UTRAN E-CID Measurements	403

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	403

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	404

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	405

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	406

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	406

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	407

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	409

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	412

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	416

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	419

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	419

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	420

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	420

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	421

8.1.2.10	Void	421

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	421

8.2.1	Introduction	421

8.2.2	Requirements	421

8.3	Measurements for E-UTRA carrier aggregation	426

8.3.1	Introduction	426

8.3.2	Measurements of the primary component carrier	426

8.3.3	Measurements of a secondary component carrier	426

8.3.3.1	Measurements of a secondary component carrier with active SCell	426

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	427

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	427

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	428

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	430

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	430

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	430

8.4.1	Introduction	430

8.4.2	Measurements on the primary component carrier	431

8.4.3	Measurements on a secondary component carrier	432

8.4.4	Measurements on both primary component carrier and a secondary component carrier	432

8.4.5	Measurements on different secondary component carriers	433

8.5	Measurements for UE category 0	434

8.5.1	Introduction	434

8.5.2 	Requirements	435

8.5.2.1 	E-UTRAN intra frequency measurements	435

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	435

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	438

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	440

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	444

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	445

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	445

8.6	Discovery signal measurements	446

8.6.1	Introduction	446

8.6.2	Requirements for CRS based discovery signal measurements	446

8.6.2.1	E-UTRAN intra frequency measurements	446

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	446

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	449

8.6.2.2	E-UTRAN inter frequency measurements	451

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	452

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	454

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	457

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	457

8.6.3	Requirements for CSI-RS based discovery signal measurements	457

8.6.3.1	E-UTRAN intra frequency measurements	457

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	458

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	460

8.6.3.2	E-UTRAN inter frequency measurements	462

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	463

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	465

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	468

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	468

8.7	Discovery signal measurements for E-UTRA carrier aggregation	468

8.7.1	Introduction	468

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	469

8.7.2.1	Measurements of the primary component carrier	469

8.7.2.2	Measurements of a secondary component carrier	469

8.7.2.3	Measurements of a secondary component carrier with active SCell	469

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	469

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	469

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	470

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	472

8.7.3.1	Measurements of the primary component carrier	472

8.7.3.2	Measurements of a secondary component carrier	472

8.7.3.3	Measurements of a secondary component carrier with active SCell	472

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	472

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	472

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	474

8.8	Measurements for E-UTRA dual connectivity	475

8.8.1	Introduction	475

8.8.2	Intra-frequency measurements requirements on PCell	475

8.8.3	Intra-frequency measurements requirements on PSCell	476

8.8.4	Inter-frequency and inter-RAT measurement requirements	476

8.8.5	Intra-frequency measurements with autonomous gaps	476

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	476

8.8.5.2	ECGI reporting delay	477

8.8.6	Inter-frequency measurements with autonomous gaps	477

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	477

8.8.6.2	ECGI reporting delay	478

8.8.7	SSTD Measurements	478

8.8.7.1	Introduction	478

8.8.7.2	SSTD Measurement requirements	478

8.8.7.3	SSTD Measurement Reporting Delay	479

8.8.8	Intra-frequency measurements requirements on SCell	479

8.9	MBSFN Measurements	479

8.9.1	Introduction	479

8.9.2	MBSFN RSRP Measurements	479

8.9.3	MBSFN RSRQ Measurements	480

8.9.4	MCH BLER Measurements	480

8.10	Proximity-based Services	480

8.10.1	Introduction	480

8.10.2	Requirements	480

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	480

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	481

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	481

8.11.1	Introduction	481

8.11.2	CRS based discovery signal measurements	482

8.11.2.1	E-UTRAN intra-frequency measurements	482

8.11.2.1.1	Requirements	482

8.11.2.1.1.1	Requirements when no DRX is used	482

8.11.2.1.1.1.1	Measurement Reporting Requirements	484

8.11.2.1.1.2	Requirements when DRX is used	485

8.11.2.1.1.2.1	Measurement Reporting Requirements	487

8.11.2.2	E-UTRAN inter-frequency measurements	488

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	488

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	492

8.11.3	CSI-RS based discovery signal measurements	492

8.11.3.1	E-UTRAN intra-frequency measurements	492

8.11.3.1.1	Requirements	492

8.11.3.1.1.1	Requirements when no DRX is used	492

8.11.3.1.1.1.1	Measurement Reporting Requirements	494

8.11.3.1.1.2	Requirements when DRX is used	494

8.11.3.1.1.2.1	Measurement Reporting Requirements	495

8.11.3.2	E-UTRAN inter-frequency measurements	496

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	496

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	500

8.11.4	RSSI measurements	500

8.11.4.1	E-UTRAN intra-frequency measurements	500

8.11.4.2	E-UTRAN inter-frequency measurements	500

8.11.5	Channel occupancy measurements	501

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	501

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	501

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	501

8.12.1	Introduction	501

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	501

8.12.2.1	Introduction	501

8.12.2.2	Measurements of a secondary component carrier	501

8.12.2.3	Measurements of a secondary component carrier with active SCell	501

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	502

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	502

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	504

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	507

8.12.3.1	Introduction	507

8.12.3.2	Measurements of a secondary component carrier	507

8.12.3.3	Measurements of a secondary component carrier with active SCell	507

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	507

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	507

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	509

8.13	Measurements for UE Category M1	511

8.13.1	Introduction	511

8.13.2	Requirements for UE category M1 with CE mode A	511

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	512

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	512

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	516

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	519

8.13.2.2	Void	523

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	523

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	523

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	526

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	528

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	529

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	529

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	531

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	534

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	534

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	534

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	534

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	535

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	535

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	536

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	536

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	536

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	537

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	538

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	539

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	539

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	543

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	545

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	549

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	549

8.13.3	Requirements for UE category M1 with CE mode B	550

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	550

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	551

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	555

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	558

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	563

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	564

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	564

8.13.3.2	Void	565

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	565

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	565

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	568

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	570

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	571

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	571

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	571

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	572

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	572

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	572

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	573

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	573

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	573

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	578

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	580

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	584

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	585

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	585

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	587

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	589

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	590

8.14	Measurements for UE category NB1	590

8.14.1	Introduction	590

8.14.2	NB-IoT intra frequency measurements under normal coverage	591

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	591

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	591

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	591

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	591

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	591

8.14.4	Connected mode channel quality report for UE Category NB1	591

8.15	Void	592

8.16	Measurements for UE Category M2	592

8.16.1	Introduction	592

8.16.2	Requirements for UE category M2 with CE mode A	592

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	592

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	593

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	593

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	594

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	594

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	594

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	595

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	595

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	597

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	600

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	600

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	600

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	603

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	605

8.16.3	Requirements for UE category M2 with CE mode B	606

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	606

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	606

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	608

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	611

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	611

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	612

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	614

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	616

8.17	Measurements for E-UTRA – NR Dual Connectivity	617

8.17.1	Introduction	617

8.17.1.1	Measurement Gap Sharing	617

8.17.1A	Intrafrequency Measurements	617

8.17.2	SFTD Measurements	618

8.17.2.1	Introduction	618

8.17.2.2	SFTD Measurement requirements	618

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	618

8.17.2.3	SFTD Measurement Reporting Delay	620

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	620

8.17.3.1	Introduction	620

8.17.3.2	E-UTRAN FDD inter frequency measurements	620

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	620

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	622

8.17.3.3	E-UTRAN TDD inter frequency measurements	624

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	624

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	625

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	627

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	627

8.17.4.1.1	NR Inter-RAT cell identification	628

8.17.4.1.2	NR Inter-RAT measurement	629

8.17.4.1.3	NR Inter-RAT measurement reporting	630

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	631

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	631

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	631

8.17.4A.1.1	NR Inter-RAT cell identification	632

8.17.4A.1.2	NR Inter-RAT measurement	633

8.17.4A.1.3	NR Inter-RAT measurement reporting	634

8.17.4A.1.4	NR inter-RAT RSSI measurements	634

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	635

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	635

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	635

8.17.5.1	Introduction	635

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	635

8.17.5.2.1	Identification of a new UTRA FDD cell	635

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	636

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	636

8.17.5.2.4	Periodic Reporting	636

8.17.5.2.5	Event Triggered Reporting	636

8.17.5.2.6	Event-triggered Periodic Reporting	637

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	637

8.17.5.3.1	Periodic Reporting	638

8.17.5.3.2	Event Triggered Reporting	638

8.17.5.3.3	Event-triggered Periodic Reporting	639

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	639

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	639

8.17.7.1	Introduction	639

8.17.7.2	Identification of a new UTRA FDD cell for SON	639

8.17.7.2.1	Requirements when no DRX is used	639

8.17.7.2.2	Requirements when DRX is used	640

8.17.7.2.3	Reporting Delay	640

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	641

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	641

8.17.9.1	Introduction	641

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	641

8.17.9.2.1	Identification of a new UTRA FDD cell	641

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	641

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	642

8.17.9.2.4	Periodic Reporting	642

8.17.9.2.5	Event Triggered Reporting	642

8.17.9.2.6	Event-triggered Periodic Reporting	643

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	643

8.17.9.3.1	Periodic Reporting	644

8.17.9.3.2	Event Triggered Reporting	644

8.17.9.3.3	Event-triggered Periodic Reporting	644

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	645

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	645

8.17.11.1	Introduction	645

8.17.11.2	Identification of a new UTRA TDD cell for SON	645

8.17.11.2.1	Requirements when no DRX is used	645

8.17.11.2.2	Requirements when DRX is used	645

8.17.11.2.3	Reporting Delay	646

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	646

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	647

8.17.13.1	Introduction	647

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	647

8.17.13.2.1	GSM carrier RSSI	647

8.17.13.2.2	BSIC verification	647

8.17.13.2.3	Enhanced BSIC verification	649

8.17.13.2.4	Periodic Reporting	649

8.17.13.2.5	Event Triggered Reporting	649

8.17.13.2.6	Event-triggered Periodic Reporting	650

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	650

8.17.13.3.1	GSM carrier RSSI	650

8.17.13.3.2	BSIC verification	650

8.17.13.3.3	Periodic Reporting	652

8.17.13.3.4	Event Triggered Reporting	652

8.17.13.3.5	Event-triggered Periodic Reporting	652

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	652

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	652

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	652

8.17.15.1.1	RSTD Measurement Reporting Delay	653

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	653

8.17.15.2.1	RSTD Measurement Reporting Delay	653

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	653

8.17.15.3.1	RSTD Measurement Reporting Delay	654

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	654

8.17.15.4.1	RSTD Measurement Reporting Delay	654

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	655

8.17.16.1	Introduction	655

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	655

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	655

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	655

8.17.17.1	Introduction	655

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	655

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	655

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	655

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	655

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	655

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	655

8.18	Measurements for non-BL/CE UE	655

8.18.1	Introduction	655

8.18.2	Requirements for non-BL/CE UE with CE Mode B	656

8.18.2.1	E-UTRAN intra frequency measurements	656

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	656

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	656

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	657

8.19	Measurements for NR – E-UTRA Dual Connectivity	658

8.19.1	Introduction	658

8.19.2	Intra-frequency Measurements	658

8.19.3	Inter-frequency Measurements	658

8.19.4	Void	658

8.19.5	Intra-frequency E-CID Measurements	658

8.19.6	Intra-frequency measurements with autonomous gaps	659

8.19.6.1	Introduction	659

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	659

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	659

8.19.7	Inter-frequency measurements with autonomous gaps	659

8.19.7.1	Introduction	659

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	659

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	659

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	659

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	659

9	Measurements performance requirements for UE	659

9.1	E-UTRAN measurements	659

9.1.1	Introduction	659

9.1.2	Intra-frequency RSRP Accuracy Requirements	660

9.1.2.1	Absolute RSRP Accuracy	660

9.1.2.2	Relative Accuracy of RSRP	660

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	661

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	662

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	663

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	664

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	665

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	666

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	667

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	667

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	667

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	668

9.1.2B.1	Introduction	668

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	668

9.1.3	Inter-frequency RSRP Accuracy Requirements	669

9.1.3.1	Absolute RSRP Accuracy	669

9.1.3.2	Relative Accuracy of RSRP	670

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	671

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	671

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	672

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	672

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	673

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	674

9.1.3B.1	Introduction	674

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	674

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	674

9.1.4	RSRP Measurement Report Mapping	675

9.1.5	Intra-frequency RSRQ Accuracy Requirements	675

9.1.5.1	Absolute RSRQ Accuracy	675

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	676

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	677

9.1.5.4	Absolute WB-RSRQ Accuracy	678

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	679

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	680

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	680

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	680

9.1.5B.1	Introduction	680

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	680

9.1.6	Inter-frequency RSRQ Accuracy Requirements	681

9.1.6.1	Absolute RSRQ Accuracy	681

9.1.6.2	Relative Accuracy of RSRQ	682

9.1.6.3	Absolute WB-RSRQ Accuracy	683

9.1.6.4	Relative WB-RSRQ Accuracy	683

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	684

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	685

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	685

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	685

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	686

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	687

9.1.6B.1	Introduction	687

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	687

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	687

9.1.7	RSRQ Measurement Report Mapping	688

9.1.8	Power Headroom	688

9.1.8.1	Period	689

9.1.8.2	Reporting Delay	689

9.1.8.3	Void	689

9.1.8.4	Report Mapping	689

9.1.9	UE Rx – Tx time difference	689

9.1.9.1	Measurement Requirement	689

9.1.9.2	Measurement Report mapping	690

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	691

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	692

9.1.10	Reference Signal Time Difference (RSTD)	693

9.1.10.1	Intra-Frequency Accuracy Requirement	693

9.1.10.2	Inter-Frequency Accuracy Requirement	694

9.1.10.3	RSTD Measurement Report Mapping	696

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	696

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	697

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	698

9.1.11	Carrier aggregation measurement accuracy	699

9.1.11.1	Primary component carrier accuracy requirement	700

9.1.11.2	Secondary component carrier accuracy requirement	700

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	700

9.1.11.4	Secondary component carrier relative accuracy requirement	700

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	700

9.1.13	Measurement accuracy for UE category 0	701

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	701

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	701

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	702

9.1.14	Accuracy requirements for Discovery Signal Measurements	703

9.1.14.1	Introduction	703

9.1.14.2	RSRP measurements in discovery signal occasions	703

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	703

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	703

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	703

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	704

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	705

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	705

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	705

9.1.14.3.3	CSI-RSRP measurement report mapping	706

9.1.14.4	RSRQ measurements in discovery signal occasions	706

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	706

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	707

9.1.15.1.1	Primary component carrier accuracy requirement	707

9.1.15.1.2	Secondary component carrier accuracy requirement	707

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	707

9.1.15.1.4	Secondary component carrier relative accuracy requirement	707

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	707

9.1.15.2.1	Primary component carrier accuracy requirement	707

9.1.15.2.2	Secondary component carrier accuracy requirement	707

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	707

9.1.15.2.4	Secondary component carrier relative accuracy requirement	707

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	708

9.1.17	RS-SINR Measurements	708

9.1.17.1	Measurement Report Mapping	708

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	709

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	709

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	709

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	709

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	710

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	711

9.1.18.1	Introduction	711

9.1.18.2	RSRP measurements	711

9.1.18.2.1	RSRP measurement report mapping	711

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	711

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	712

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	712

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	713

9.1.18.3	RSRQ measurements	713

9.1.18.3.1	RSRQ measurement report mapping	713

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	713

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	714

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	714

9.1.18.4	CSI-RSRP measurements	715

9.1.18.4.1	CSI-RSRP measurement report mapping	715

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	715

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	715

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	716

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	716

9.1.18.5	RSSI measurements	717

9.1.18.5.1	RSSI measurement report mapping	717

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	717

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	718

9.1.18.6	Channel occupancy measurements	718

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	718

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	718

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	719

9.1.19.1	Introduction	719

9.1.19.2	Accuracy requirements for measurements on SCC	719

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	719

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	719

9.1.20	SFN and Subframe Time Difference (SSTD)	720

9.1.20.1	SSTD Accuracy Requirement	720

9.1.20.2	SSTD Measurement Report Mapping	720

9.1.21	Measurement accuracy for UE category M1	721

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	721

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	723

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	724

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	726

9.1.21.5	RSRP Measurement Report Mapping	727

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	727

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	728

9.1.21.8	RSRQ Measurement Report Mapping	729

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	729

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	730

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	731

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	732

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	733

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	734

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	735

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	736

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	737

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	738

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	739

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	740

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	743

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	746

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	746

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	749

9.1.22	Measurement accuracy for UE Category NB1	750

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	750

9.1.22.2	Void	751

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	751

9.1.22.4	Void	752

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	752

9.1.22.6	Void	753

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	753

9.1.22.8	Void	754

9.1.22.9	NRSRP Measurement Report Mapping	754

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	754

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	755

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	756

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	757

9.1.22.14	NRSRQ Measurement Report Mapping	758

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	759

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	759

9.1.23	Power Headroom for UE Category NB1	760

9.1.23.1	Period	760

9.1.23.2	Reporting Delay	760

9.1.23.3	Report Mapping for UE Category NB1	760

9.1.23.3.1	Void	762

9.1.23.3.2	Void	762

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	762

9.1.24	Void	763

9.1.25	Measurement accuracy for UE category M2	763

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	763

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	764

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	765

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	766

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	768

9.1.26	Measurement Accuracy for non-BL CE UE	769

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	770

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	772

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	772

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	774

9.1.26.5	RSRP Measurement Report Mapping	775

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	775

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	775

9.1.26.8	RSRQ Measurement Report Mapping	776

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	776

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	777

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	778

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	779

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	780

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	780

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	780

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	781

9.1.27	SFN and frame Timing Difference (SFTD)	782

9.1.27.1	SFTD Accuracy Requirement	782

9.2	UTRAN FDD Measurements	784

9.2.1	UTRAN FDD CPICH RSCP	784

9.2.2	Void	785

9.2.3	UTRAN FDD CPICH Ec/No	785

9.3	UTRAN TDD Measurements	785

9.3.1	UTRAN TDD P-CCPCH RSCP	786

9.3.2	Void	786

9.3.3	Void	786

9.4	GSM Measurements	786

9.4.1	GSM carrier RSSI	786

9.5	CDMA2000 1x RTT Measurements	786

9.5.1	CDMA2000 1x RTT Pilot Strength	786

9.6	PCMAX,c	787

9.6.1	Report Mapping	787

9.6.2 	Estimation Period	787

9.6.3	Reporting Delay	787

9.7	IEEE802.11 Measurements	787

9.7.1	WLAN RSSI	787

9.7.2	WLAN RSSI Measurement Report Mapping	787

9.8	MBSFN Measurements	788

9.8.1	Introduction	788

9.8.2	MBSFN RSRP	788

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	788

9.8.2.2	MBSFN RSRP measurement report mapping	789

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	789

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	790

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	790

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	790

9.8.3	MBSFN RSRQ	791

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	791

9.8.3.2	MBSFN RSRQ measurement report mapping	791

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	792

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	792

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	792

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	793

9.8.4	MCH BLER	793

9.8.4.1	Measurement report mapping for MCH BLER	793

9.8.4.2	Measurement report mapping for MCH Block Number	794

9.9	ProSe Measurements	795

9.9.1	Introduction	795

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	795

9.9.2.1	Absolute S-RSRP Accuracy	795

9.9.2.2	Relative Accuracy of S-RSRP	796

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	797

9.9.3.1	Absolute SD-RSRP Accuracy	797

9.9.3.2	Relative Accuracy of SD-RSRP	797

9.10	V2X Measurements	798

9.10.1	Introduction	798

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	798

9.10.2.1	Absolute S-RSRP Accuracy	798

9.10.2.2	Relative Accuracy of S-RSRP	799

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	799

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	799

9.10.4	S-RSSI Measurement Accuracy Requirements	800

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	800

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	800

9.11	NR Measurements	801

9.11.1	NR SS-RSRP Measurements	801

9.11.2	NR SS-RSRQ Measurements	801

9.11.3	NR SS-SINR Measurements	802

10	Measurements Performance Requirements for E-UTRAN	802

10.1	Received Interference Power	802

10.1.1	Absolute accuracy requirement	802

10.1.2	Relative accuracy requirement	803

10.1.3	Received Interference Power measurement report mapping	803

10.2	Angle of Arrival (AOA)	803

10.2.1	Range/mapping	803

10.3	Timing Advance (TADV)	803

10.3.1	Report mapping	803

11	ProSe Requirements in Any Cell Selection state	804

11.1	Introduction	804

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	804

11.2.1	Introduction	804

11.2.2	ProSe UE transmission timing	804

11.3	Initiation/Cease of SLSS Transmissions	805

11.3.1	Introduction	805

11.3.2	Requirements	805

11.4	Measurements for ProSe in Any Cell Selection State	805

11.4.1	Introduction	805

11.4.2	Requirements	805

11.4.2.1	E-UTRA FDD	805

11.4.2.2	E-UTRA TDD	806

11.5	Selection / Reselection of ProSe Synchronization Reference	806

11.5.1	Introduction	806

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	806

11.5.2.1	Introduction	806

11.5.2.2	Requirements	806

11.6	Void	807

11.7	Selection / Reselection of ProSe relay UE	807

11.7.1	Introduction	807

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	807

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	807

12.1	Introduction	807

12.2	Transmit Timing	808

12.2.1	GNSS as timing reference	808

12.3	Interruption	808

12.4	Reliability of GNSS signal	808

13	V2X Requirements	808

13.1	Introduction	808

13.2	UE Transmit Timing	809

13.2.1	Introduction	809

13.2.2	GNSS as synchronization reference source	809

13.2.3	Serving cell/PCell as synchronization reference source	809

13.2.4	SyncRef UE as synchronization reference source	809

13.3	Initiation/Cease of SLSS Transmissions	809

13.3.1	Introduction	809

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	809

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	810

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	810

13.4	Selection / Reselection of V2X Synchronization Reference Source	811

13.5	Autonomous Resource Selection/Reselection measurements	812

13.5.1	Introduction	812

13.5.2	PSSCH-RSRP measurements	812

13.5.3	S-RSSI measurements	812

13.6	Congestion Control measurements	812

13.7	Interruption	812

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	812

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	812

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	813

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	813

13.8	Reliability of GNSS signal	813

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	814

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	814

Annex A (normative):	Test Cases	811

A.1	Purpose of annex	811

A.2	Requirement classification for statistical testing	811

A.2.1	Types of requirements in TS 36.133	811

A.2.1.1	Time and delay requirements on UE higher layer actions	811

A.2.1.2	Measurements of power levels, relative powers and time	811

A.2.1.3	Implementation requirements	812

A.2.1.4	Physical layer timing requirements	812

A.3	RRM test configurations	813

A.3.1	Reference Measurement Channels	813

A.3.1.1	PDSCH	813

A.3.1.1.1	FDD	813

A.3.1.1.2	TDD	818

A.3.1.1.3	FDD for UE category 0	821

A.3.1.1.4	HD-FDD for UE category 0	822

A.3.1.1.5	TDD for UE category 0	823

A.3.1.1.6	Frame Structure 3	824

A.3.1.2	PCFICH/PDCCH/PHICH	825

A.3.1.2.1	FDD	825

A.3.1.2.2	TDD	825

A.3.1.2.3	HD-FDD for UE category 0	826

A.3.1.2.4	FS 3	826

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	826

A.3.1.3.1	FDD in CEModeA	827

A.3.1.3.2	HD-FDD in CEModeA	827

A.3.1.3.3	TDD in CEModeA	828

A.3.1.3.4	FDD in CEModeB	828

A.3.1.3.5	HD-FDD in CEModeB	829

A.3.1.3.6	TDD in CEModeB	829

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	830

A.3.1.4.1	FDD in CEModeA	830

A.3.1.4.2	HD-FDD in CEModeA	831

A.3.1.4.3	TDD in CEModeA	832

A.3.1.4.4	FDD in CEModeB	833

A.3.1.4.5	 HD-FDD in CEModeB	834

A.3.1.4.6	TDD in CEModeB	834

A.3.1.5	NPDSCH Reference Channel for UE category NB1	835

A.3.1.5.1	HD-FDD in-band operation	835

A.3.1.5.2	Void	836

A.3.1.5.3	HD-FDD standalone operation	836

A.3.1.5.4	Void	837

A.3.1.5.5	HD-FDD guard band operation	837

A.3.1.5.6	Void	838

A.3.1.5.7	TDD in-band operation	838

A.3.1.5.8	TDD standalone operation	838

A.3.1.5.9	TDD guard band operation	839

A.3.1.6	NPDCCH Reference Channel for UE category NB1	839

A.3.1.6.1	In-band operation	839

A.3.1.6.2	Void	840

A.3.1.6.3	Standalone operation	840

A.3.1.6.4	Void	840

A.3.1.6.5	Guard band operation	840

A.3.1.6.6	Void	841

A.3.2	OFDMA Channel Noise Generator (OCNG)	841

A.3.2.1	OCNG Patterns for FDD	841

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	842

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	842

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	843

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	843

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	844

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	845

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	845

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	845

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	846

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	847

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	847

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	848

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	848

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	849

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	849

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	850

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	850

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	851

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	851

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	852

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	852

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	853

A.3.2.2	OCNG Patterns for TDD	853

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	854

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	854

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	855

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	855

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	856

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	857

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	858

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	859

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	859

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	860

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	860

A.3.2.3	OCNG Patterns for Narrowband IoT	861

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	862

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	863

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	863

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	864

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	865

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	866

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	867

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	868

A.3.2.4	OCNG Patterns for V2X sidelink	869

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	869

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	870

A.3.3	Reference DRX Configurations	870

A.3.4	ABS Transmission Configurations	871

A.3.4.1	Non-MBSFN ABS Transmission Configurations	871

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	871

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	871

A.3.4.2	MBSFN ABS Transmission Configurations	872

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	872

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	873

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	873

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	873

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	874

A.3.6.1	Introduction	874

A.3.7	Test Cases with Different Channel Bandwidths	874

A.3.7.1	Introduction	874

A.3.7.2	Principle of testing	874

A.3.8	Antenna Configuration	874

A.3.8.1	Antenna connection for 4 Rx capable UEs	874

A.3.8.1.1 Introduction	874

A.3.8.1.2 Principle of testing	874

A.3.8.1.2.1 Single carrier tests	874

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	875

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	876

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	876

A.3.8.2	Antenna connection for 8 Rx capable UEs	876

A.3.8.2.1	Introduction	876

A.3.8.2.2	Principle of testing	876

A.3.8.2.2.1	Single carrier tests	876

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	877

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	877

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	877

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	877

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	877

A.3.9.1	Introduction	877

A.3.9.2	Principle of testing	877

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	878

A.3.10.1	Introduction	878

A.3.10.2	Principle of testing	878

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	878

A.3.11.1	Introduction	878

A.3.11.2	Principle of Testing	878

A.3.12	Proximity-based Services	878

A.3.12.1	Introduction	878

A.3.12.2	Reference DRX configurations for ProSe tests	878

A.3.12.3	Test Cases with Different Channel Bandwidths	879

A.3.12.3.1	Introduction	879

A.3.12.3.2	Principle of testing	879

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	879

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	886

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	889

A.3.12.6.1	FDD	889

A.3.12.7	Reference measurement channels for ProSe Direct Communication	889

A.3.12.7.1	FDD	889

A.3.12.8	ProSe Receive Traffic Generator	890

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	890

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	890

A.3.13	Time Offset between Cells	891

A.3.13.1	Introduction	891

A.3.13.2	Definition	891

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	891

A.3.14.1	Introduction	891

A.3.14.2	Principle of testing	891

A.3.15	Dual connectivity test cases with different combination of duplex mode	891

A.3.15.1	Introduction	891

A.3.15.2	Principle of testing	891

A.3.16	Reference PRACH Configurations	892

A.3.17	Listen before talk model	892

A.3.17.1	Introduction	892

A.3.17.2	Definition	892

A.3.18	Reference NPRACH Configurations	893

A.3.19	Dual connectivity test cases with different bandwidth combinations	894

A.3.19.1	Introduction	894

A.3.19.2	Principle of testing	894

A.3.20	Category M1 UE Test Cases	895

A.3.20.1	Introduction	895

A.3.20.2	Principle of Cat-M1 UE Testing	895

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	896

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	897

A.3.21.1	Introduction	897

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	897

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	898

A.3.22	Category 1bis UE Test Cases	899

A.3.22.1	Introduction	899

A.3.22.2	Principle of Category 1bis UE Testing	899

A.3.23	Category NB2 UE Test Cases	903

A.3.23.1	Introduction	903

A.3.23.2	Principle of Category NB2 UE Testing	903

A.3.24	V2X sidelink communication	906

A.3.24.1	Introduction	906

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	907

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	911

A.3.25	Category M2 UE Test Cases	912

A.3.25.1	Introduction	912

A.3.25.2	Principle of Cat-M2 UE Testing	912

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	913

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	914

A.3.26.1	Introduction	914

A.3.26.2	Principle of testing	914

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	914

A.3.27.1	Introduction	914

A.3.27.2	Principle of INACTIVE cell re-selection Testing	914

A.4	E-UTRAN RRC_IDLE state	914

A.4.2	Cell Re-Selection	914

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	914

A.4.2.1.1	Test Purpose and Environment	914

A.4.2.1.2	Test Requirements	916

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	917

A.4.2.2.1	Test Purpose and Environment	917

A.4.2.2.2	Test Requirements	918

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	919

A.4.2.3.1	Test Purpose and Environment	919

A.4.2.3.2	Test Requirements	920

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	921

A.4.2.4.1	Test Purpose and Environment	921

A.4.2.4.2	Test Requirements	922

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	923

A.4.2.5.1	Test Purpose and Environment	923

A.4.2.5.2	Test Requirements	924

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	925

A.4.2.6.1	Test Purpose and Environment	925

A.4.2.6.2	Test Requirements	926

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	927

A.4.2.7.1	Test Purpose and Environment	927

A.4.2.7.2	Test Requirements	928

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	929

A.4.2.8.1	Test Purpose and Environment	929

A.4.2.8.2	Test Requirements	930

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	931

A.4.2.9.1	Test Purpose and Environment	931

A.4.2.9.2	Test Requirements	931

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	931

A.4.2.10.1	Test Purpose and Environment	931

A.4.2.10.2	Test Requirements	935

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	935

A.4.2.11.1	Test Purpose and Environment	935

A.4.2.11.2	Test Requirements	939

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	939

A.4.2.12.1	Test Purpose and Environment	939

A.4.2.12.2	Test Requirements	941

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	942

A.4.2.13.1	Test Purpose and Environment	942

A.4.2.13.2	Test Requirements	943

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	944

A.4.2.14.1	Test Purpose and Environment	944

A.4.2.14.2	Test Requirements	945

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	946

A.4.2.15.1	Test Purpose and Environment	946

A.4.2.15.2	Test Requirements	947

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	948

A.4.2.16.1	Test Purpose and Environment	948

A.4.2.16.2	Test Requirements	949

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	950

A.4.2.17.1	Test Purpose and Environment	950

A.4.2.17.2	Test Requirements	951

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	952

A.4.2.18.1	Test Purpose and Environment	952

A.4.2.18.2	Test Requirements	954

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	955

A.4.2.19.1	Test Purpose and Environment	955

A.4.2.19.2	Test Requirements	957

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	958

A.4.2.20.1	Test Purpose and Environment	958

A.4.2.20.2	Test Requirements	959

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	960

A.4.2.21.1	Test Purpose and Environment	960

A.4.2.21.2	Test Requirements	961

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	962

A.4.2.22.1	Test Purpose and Environment	962

A.4.2.22.2	Test Requirements	963

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	964

A.4.2.23.1	Test Purpose and Environment	964

A.4.2.23.2	Test Requirements	965

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	966

A.4.2.24.1	Test Purpose and Environment	966

A.4.2.24.2	Test Requirements	969

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	970

A.4.2.25.1	Test Purpose and Environment	970

A.4.2.25.2	Test Requirements	971

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	972

A.4.2.26.1	Test Purpose and Environment	972

A.4.2.26.2	Test Requirements	973

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	974

A.4.2.27.1	Test Purpose and Environment	974

A.4.2.27.2	Test Requirements	975

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	976

A.4.2.28.1	Test Purpose and Environment	976

A.4.2.28.2	Test Requirements	977

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	978

A.4.2.29.1	Test Purpose and Environment	978

A.4.2.29.2	Test Requirements	979

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	980

A.4.2.30.1	Test Purpose and Environment	980

A.4.2.30.2	Test Requirements	981

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	982

A.4.2.31.1	Test Purpose and Environment	982

A.4.2.31.2	Test Requirements	983

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	984

A.4.2.32.1	Test Purpose and Environment	984

A.4.2.32.2	Test Requirements	985

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	986

A.4.2.33.1	Test Purpose and Environment	986

A.4.2.33.2	Test Requirements	987

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	988

A.4.2.34.1	Test Purpose and Environment	988

A.4.2.34.2	Test Requirements	989

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	990

A.4.2.35.1	Test Purpose and Environment	990

A.4.2.35.2	Test Requirements	992

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	993

A.4.2.36.1	Test Purpose and Environment	993

A.4.2.36.2	Test Requirements	995

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	996

A.4.2.37.1	Test Purpose and Environment	996

A.4.2.37.2	Test Requirements	998

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	999

A.4.2.38.1	Test Purpose and Environment	999

A.4.2.38.2	Test Requirements	1001

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1002

A.4.2.39.1	Test Purpose and Environment	1002

A.4.2.39.2	Test Requirements	1003

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1004

A.4.2.40.1	Test Purpose and Environment	1004

A.4.2.40.2	Test Requirements	1005

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1006

A.4.2.41.1	Test Purpose and Environment	1006

A.4.2.41.2	Test Requirements	1009

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1010

A.4.2.42.1	Test Purpose and Environment	1010

A.4.2.42.2	Test Requirements	1012

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1013

A.4.2.43.1	Test Purpose and Environment	1013

A.4.2.43.2	Test Requirements	1015

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1016

A.4.2.44.1	Test Purpose and Environment	1016

A.4.2.44.2	Test Requirements	1018

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1019

A.4.2.45.1	Test Purpose and Environment	1019

A.4.2.45.2	Test Requirements	1021

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1022

A.4.2.46.1	Test Purpose and Environment	1022

A.4.2.46.2	Test Requirements	1024

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1025

A.4.2.47.1	Test Purpose and Environment	1025

A.4.2.47.2	Test Requirements	1027

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1028

A.4.2.48.1	Test Purpose and Environment	1028

A.4.2.48.2	Test Requirements	1029

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1030

A.4.2.49.1	Test Purpose and Environment	1030

A.4.2.49.2	Test Requirements	1032

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1033

A.4.2.50.1	Test Purpose and Environment	1033

A.4.2.50.2	Test Requirements	1035

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1036

A.4.2.51.1	Test Purpose and Environment	1036

A.4.2.51.2	Test Requirements	1038

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1039

A.4.2.52.1	Test Purpose and Environment	1039

A.4.2.52.2	Test Requirements	1041

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1042

A.4.2.53.1	Test Purpose and Environment	1042

A.4.2.53.2	Test Requirements	1043

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1044

A.4.2.54.1	Test Purpose and Environment	1044

A.4.2.54.2	Test Requirements	1046

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1047

A.4.2.55.1	Test Purpose and Environment	1047

A.4.2.55.2	Test Requirements	1049

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1050

A.4.2.56.1	Test Purpose and Environment	1050

A.4.2.56.2	Test Requirements	1051

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1052

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1052

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1052

A.4.3.1.1.1	Test Purpose and Environment	1052

A.4.3.1.1.2	Test Requirements	1054

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1054

A.4.3.1.2.1	Test Purpose and Environment	1054

A.4.3.1.2.2	Test Requirements	1057

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1057

A.4.3.1.3.1	Test Purpose and Environment	1057

A.4.3.1.3.2	Test Requirements	1060

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1060

A.4.3.1.4.1	Test Purpose and Environment	1060

A.4.3.1.4.2	Test Requirements	1061

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1061

A.4.3.1.5.1	Test Purpose and Environment	1061

A.4.3.1.5.2	Test Requirements	1064

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1065

A.4.3.2.1	Test Purpose and Environment	1065

A.4.3.2.1.1	Void	1065

A.4.3.2.1.2	1.28Mcps TDD option	1065

A.4.3.2.1.3	Void	1067

A.4.3.2.2	Test Requirements	1067

A.4.3.2.2.1	1.28Mcps TDD option	1067

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1067

A.4.3.2A.1	Test Purpose and Environment	1067

A.4.3.2A.2	Test Requirements	1071

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1071

A.4.3.3.1	Test Purpose and Environment	1071

A.4.3.3.2	Test Requirements	1074

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1074

A.4.3.3A.1	Test Purpose and Environment	1074

A.4.3.3A.2	Test Requirements	1079

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1080

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1080

A.4.3.4.1.1	Test Purpose and Environment	1080

A.4.3.4.1.2	Test Requirements	1082

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1082

A.4.3.4.2.1	Test Purpose and Environment	1082

A.4.3.4.2.2	Test Requirements	1084

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1084

A.4.3.4.3.1	Test Purpose and Environment	1084

A.4.3.4.3.2	Test Requirements	1087

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1087

A.4.3.4.4.1	Test Purpose and Environment	1087

A.4.3.4.4.2	Test Requirements	1091

A.4.4	E-UTRAN to GSM Cell Re-Selection	1091

A.4.4.1	E-UTRAN FDD – GSM:	1091

A.4.4.1.1	Test Purpose and Environment	1091

A.4.4.1.2	Test Requirements	1093

A.4.4.2	E-UTRAN TDD – GSM:	1093

A.4.4.2.1	Test Purpose and Environment	1093

A.4.4.2.2	Test Requirements	1095

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1096

A.4.5.1	E-UTRAN FDD – HRPD	1096

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1096

A.4.5.1.1.1	Test Purpose and Environment	1096

A.4.5.1.1.2	Test Requirements	1098

A.4.5.2	E-UTRAN TDD – HRPD	1098

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1098

A.4.5.2.1.1	Test Purpose and Environment	1098

A.4.5.2.1.2	Test Requirements	1101

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1101

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1101

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1101

A.4.6.1.1.1	Test Purpose and Environment	1101

A.4.6.1.1.2	Test Requirements	1104

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1104

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1104

A.4.6.2.1.1	Test Purpose and Environment	1104

A.4.6.2.1.2	Test Requirements	1107

A.4.7	Idle State Positioning Measurement for UE category NB1	1107

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1107

A.4.7.1.1	Test Purpose and Environment	1107

A.4.7.1.2	Test Requirements	1112

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1113

A.4.7.2.1	Test Purpose and Environment	1113

A.4.7.4.2	Test Requirements	1117

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1118

A.4.7.3.1	Test Purpose and Environment	1118

A.4.7.3.2	Test Requirements	1122

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1123

A.4.7.4.1	Test Purpose and Environment	1123

A.4.7.4.2	Test Requirements	1127

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1128

A.5.1	E-UTRAN Handover	1128

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1128

A.5.1.1.1	Test Purpose and Environment	1128

A.5.1.1.2	Test Requirements	1130

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1130

A.5.1.2.1	Test Purpose and Environment	1130

A.5.1.2.2	Test Requirements	1132

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1132

A.5.1.3.1	Test Purpose and Environment	1132

A.5.1.3.2	Test Requirements	1134

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1134

A.5.1.4.1	Test Purpose and Environment	1134

A.5.1.4.2	Test Requirements	1136

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1136

A.5.1.5.1	Test Purpose and Environment	1136

A.5.1.5.2	Test Requirements	1138

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1138

A.5.1.6.1	Test Purpose and Environment	1138

A.5.1.6.2	Test Requirements	1140

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1140

A.5.1.7.1	Test Purpose and Environment	1140

A.5.1.7.2	Test Requirements	1143

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1143

A.5.1.8.1	Test Purpose and Environment	1143

A.5.1.8.2	Test Requirements	1146

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1146

A.5.1.9.1	Test Purpose and Environment	1146

A.5.1.9.2	Test Requirements	1147

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1147

A.5.1.10.1	Test Purpose and Environment	1147

A.5.1.10.2	Test Requirements	1149

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1149

A.5.1.11.1	Test Purpose and Environment	1149

A.5.1.11.2	Test Requirements	1151

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1151

A.5.1.12.1	Test Purpose and Environment	1151

A.5.1.12.2	Test Requirements	1153

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1153

A.5.1.13.1	Test Purpose and Environment	1153

A.5.1.13.2	Test Requirements	1155

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1156

A.5.1.14.1	Test Purpose and Environment	1156

A.5.1.14.2	Test Requirements	1157

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1158

A.5.1.15.1	Test Purpose and Environment	1158

A.5.1.15.2	Test Requirements	1159

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1160

A.5.1.16.1	Test Purpose and Environment	1160

A.5.1.16.2	Test Requirements	1161

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1162

A.5.1.17.1	Test Purpose and Environment	1162

A.5.1.17.2	Test Requirements	1163

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1164

A.5.1.18.1	Test Purpose and Environment	1164

A.5.1.18.2	Test Requirements	1165

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1166

A.5.1.19.1	Test Purpose and Environment	1166

A.5.1.19.2	Test Requirements	1167

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1168

A.5.1.20.1	Test Purpose and Environment	1168

A.5.1.20.2	Test Requirements	1169

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1170

A.5.1.21.1	Test Purpose and Environment	1170

A.5.1.21.2	Test Requirements	1171

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1171

A.5.1.22.1	Test Purpose and Environment	1171

A.5.1.22.2	Test Requirements	1173

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1173

A.5.1.23.1	Test Purpose and Environment	1173

A.5.1.23.2	Test Requirements	1175

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1175

A.5.1.24.1	Test Purpose and Environment	1175

A.5.1.24.2	Test Requirements	1177

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1177

A.5.1.25.1	Test Purpose and Environment	1177

A.5.1.25.2	Test Requirements	1179

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1180

A.5.1.26.1	Test Purpose and Environment	1180

A.5.1.26.2	Test Requirements	1181

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1182

A.5.1.27.1	Test Purpose and Environment	1182

A.5.1.27.2	Test Requirements	1183

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1184

A.5.1.28.1	Test Purpose and Environment	1184

A.5.1.28.2	Test Requirements	1185

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1186

A.5.1.29.1	Test Purpose and Environment	1186

A.5.1.29.2	Test Requirements	1187

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1188

A.5.1.30.1	Test Purpose and Environment	1188

A.5.1.30.2	Test Requirements	1189

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1190

A.5.1.31.1	Test Purpose and Environment	1190

A.5.1.31.2	Test Requirements	1191

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1192

A.5.1.32.1	Test Purpose and Environment	1192

A.5.1.32.2	Test Requirements	1193

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1194

A.5.1.33.1	Test Purpose and Environment	1194

A.5.1.13.2	Test Requirements	1195

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1196

A.5.1.34.1	Test Purpose and Environment	1196

A.5.1.34.2	Test Requirements	1197

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1198

A.5.1.35.1	Test Purpose and Environment	1198

A.5.1.35.2	Test Requirements	1199

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1200

A.5.1.36.1	Test Purpose and Environment	1200

A.5.1.36.2	Test Requirements	1201

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1202

A.5.1.37.1	Test Purpose and Environment	1202

A.5.1.37.2	Test Requirements	1203

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1204

A.5.1.38.1	Test Purpose and Environment	1204

A.5.1.38.2	Test Requirements	1205

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1206

A.5.1.39.1	Test Purpose and Environment	1206

A.5.1.39.2	Test Requirements	1207

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1208

A.5.1.40.1	Test Purpose and Environment	1208

A.5.1.40.2	Test Requirements	1209

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1210

A.5.1.41.1	Test Purpose and Environment	1210

A.5.1.41.2	Test Requirements	1212

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1213

A.5.1.42.1	Test Purpose and Environment	1213

A.5.1.42.2	Test Requirements	1214

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1215

A.5.1.43.1	Test Purpose and Environment	1215

A.5.1.43.2	Test Requirements	1216

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1217

A.5.1.44.1	Test Purpose and Environment	1217

A.5.1.44.2	Test Requirements	1218

A.5.2	E-UTRAN Handover to other RATs	1237

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1237

A.5.2.1.1	Test Purpose and Environment	1237

A.5.2.1.2	Test Requirements	1238

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1239

A.5.2.2.1	Test Purpose and Environment	1239

A.5.2.2.2	Test Requirements	1242

A.5.2.3 E-UTRAN FDD- GSM Handover	1242

A.5.2.3.1	Test Purpose and Environment	1242

A.5.2.3.2	Test Requirements	1243

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1244

A.5.2.4.1	Test Purpose and Environment	1244

A.5.2.4.1.1	Void	1244

A.5.2.4.1.2	1.28 Mcps TDD option	1244

A.5.2.4.1.3	Void	1246

A.5.2.4.2	Test Requirements	1246

A.5.2.4.2.1	Void	1246

A.5.2.4.2.2	1.28 Mcps TDD option	1246

A.5.2.4.2.3	Void	1246

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1246

A.5.2.5.1	Test Purpose and Environment	1246

A.5.2.5.1.3	Void	1249

A.5.2.5.2	Test Requirements	1249

A.5.2.5.2.1	Void	1249

A.5.2.5.2.2	1.28 Mcps TDD option	1249

A.5.2.5.2.3	Void	1249

A.5.2.6	E-UTRAN TDD - GSM Handover	1249

A.5.2.6.1	Test Purpose and Environment	1249

A.5.2.6.2	Test Requirements	1251

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1252

A.5.2.7.1	Test Purpose and Environment	1252

A.5.2.7.2	Test Requirements	1254

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1254

A.5.2.8.1	Test Purpose and Environment	1254

A.5.2.8.2	Test Requirements	1255

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1256

A.5.2.9.1	Test Purpose and Environment	1256

A.5.2.9.2	Test Requirements	1257

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1258

A.5.2.10.1	Test Purpose and Environment	1258

A.5.2.10.2	Test Requirements	1260

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1260

A.5.2.10A.1	Test Purpose and Environment	1260

A.5.2.10A.2	Test Requirements	1263

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1263

A.5.2.10B.1	Test Purpose and Environment	1263

A.5.2.10B.2	Test Requirements	1266

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1266

A.5.2.11.1	Test Purpose and Environment	1266

A.5.2.11.2	Test Requirements	1266

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1267

A.5.3.1	E-UTRAN FDD – HRPD Handover	1267

A.5.3.1.1	Test Purpose and Environment	1267

A.5.3.1.2	Test Requirements	1269

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1269

A.5.3.2.1	Test Purpose and Environment	1269

A.5.3.2.2	Test Requirements	1272

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1272

A.5.3.3.1	Test Purpose and Environment	1272

A.5.3.3.2	Test Requirements	1274

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1275

A.5.3.4.1	Test Purpose and Environment	1275

A.5.3.4.2	Test Requirements	1276

A.5.3.5	E-UTRAN TDD – HRPD Handover	1277

A.5.3.5.1	Test Purpose and Environment	1277

A.5.3.5.2	Test Requirements	1279

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1280

A.5.3.6.1	Test Purpose and Environment	1280

A.5.3.6.2	Test Requirements	1283

A.6	RRC Connection Control	1283

A.6.1	RRC Re-establishment	1283

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1283

A.6.1.1.1	Test Purpose and Environment	1283

A.6.1.1.2	Test Requirements	1284

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1285

A.6.1.2.1	Test Purpose and Environment	1285

A.6.1.2.2	Test Requirements	1286

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1287

A.6.1.3.1	Test Purpose and Environment	1287

A.6.1.3.2	Test Requirements	1288

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1289

A.6.1.4.1	Test Purpose and Environment	1289

A.6.1.4.2	Test Requirements	1290

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1291

A.6.1.5.1	Test Purpose and Environment	1291

A.6.1.5.2	Test Requirements	1291

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1291

A.6.1.6.1	Test Purpose and Environment	1291

A.6.1.6.2	Test Requirements	1293

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1294

A.6.1.7.1	Test Purpose and Environment	1294

A.6.1.7.2	Test Requirements	1295

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1296

A.6.1.8.1	Test Purpose and Environment	1296

A.6.1.8.2	Test Requirements	1297

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1298

A.6.1.9.1	Test Purpose and Environment	1298

A.6.1.9.2	Test Requirements	1299

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1300

A.6.1.10.1	Test Purpose and Environment	1300

A.6.1.10.2	Test Requirements	1301

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1302

A.6.1.11.1	Test Purpose and Environment	1302

A.6.1.11.2	Test Requirements	1303

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1304

A.6.1.12.1	Test Purpose and Environment	1304

A.6.1.12.2	Test Requirements	1305

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1306

A.6.1.13.1	Test Purpose and Environment	1306

A.6.1.13.2	Test Requirements	1307

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1308

A.6.1.14.1	Test Purpose and Environment	1308

A.6.1.14.2	Test Requirements	1309

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1310

A.6.1.15.1	Test Purpose and Environment	1310

A.6.1.15.2	Test Requirements	1312

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1313

A.6.1.16.1	Test Purpose and Environment	1313

A.6.1.16.2	Test Requirements	1315

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1316

A.6.1.17.1	Test Purpose and Environment	1316

A.6.1.17.2	Test Requirements	1317

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1318

A.6.1.18.1	Test Purpose and Environment	1318

A.6.1.18.2	Test Requirements	1319

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1320

A.6.1.19.1	Test Purpose and Environment	1320

A.6.1.19.2	Test Requirements	1321

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1322

A.6.1.20.1	Test Purpose and Environment	1322

A.6.1.20.2	Test Requirements	1323

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1324

A.6.1.21.1	Test Purpose and Environment	1324

A.6.1.21.2	Test Requirements	1325

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1326

A.6.1.22.1	Test Purpose and Environment	1326

A.6.1.22.2	Test Requirements	1327

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1328

A.6.1.23.1	Test Purpose and Environment	1328

A.6.1.23.2	Test Requirements	1330

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1331

A.6.1.24.1	Test Purpose and Environment	1331

A.6.1.24.2	Test Requirements	1333

A.6.2	Random Access	1334

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1334

A.6.2.1.1	Test Purpose and Environment	1334

A.6.2.1.2	Test Requirements	1335

A.6.2.1.2.1	Random Access Response Reception	1335

A.6.2.1.2.2	No Random Access Response Reception	1335

A.6.2.1.2.3	Receiving a NACK on msg3	1335

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1336

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1336

A.6.2.1.2.6	Contention Resolution Timer expiry	1336

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1336

A.6.2.2.1	Test Purpose and Environment	1336

A.6.2.2.2	Test Requirements	1337

A.6.2.2.2.1	Random Access Response Reception	1338

A.6.2.2.2.2	No Random Access Response Reception	1338

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1338

A.6.2.3.1	Test Purpose and Environment	1338

A.6.2.3.2	Test Requirements	1340

A.6.2.3.2.1	Random Access Response Reception	1340

A.6.2.3.2.2	No Random Access Response reception	1340

A.6.2.3.2.3	Receiving a NACK on msg3	1340

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1341

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1341

A.6.2.3.2.6	Contention Resolution Timer expiry	1341

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1341

A.6.2.4.1	Test Purpose and Environment	1341

A.6.2.4.2	Test Requirements	1343

A.6.2.4.2.1	Random Access Response Reception	1343

A.6.2.4.2.2	No Random Access Response Reception	1343

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1343

A.6.2.5.1	Test Purpose and Environment	1343

A.6.2.5.2	Test Requirements	1344

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1344

A.6.2.6.1	Test Purpose and Environment	1344

A.6.2.6.2	Test Requirements	1344

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1345

A.6.2.7.1	Test Purpose and Environment	1345

A.6.2.7.2	Test Requirements	1347

A.6.2.7.2.1	Random Access Response Reception	1347

A.6.2.7.2.2	No Random Access Response Reception	1347

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1347

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1348

A.6.2.8.1	Test Purpose and Environment	1348

A.6.2.8.2	Test Requirements	1350

A.6.2.8.2.1	Random Access Response Reception	1350

A.6.2.8.2.2	No Random Access Response Reception	1350

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1350

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1351

A.6.2.9.1	Test Purpose and Environment	1351

A.6.2.9.2	Test Requirements	1354

A.6.2.9.2.1	Random Access Response Reception	1354

A.6.2.9.2.2	No Random Access Response Reception	1355

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1355

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1356

A.6.2.10.1	Test Purpose and Environment	1356

A.6.2.10.2	Test Requirements	1358

A.6.2.10.2.1	Random Access Response Reception	1358

A.6.2.10.2.2	No Random Access Response Reception	1359

A.6.2.10.2.3	Receiving a NACK on msg3	1359

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1359

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1359

A.6.2.10.2.6	Contention Resolution Timer expiry	1359

A.6.2.10.2.7	PRACH Resource Selection	1360

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1360

A.6.2.11.1	Test Purpose and Environment	1360

A.6.2.11.2	Test Requirements	1362

A.6.2.11.2.1	Random Access Response Reception	1362

A.6.2.11.2.2	No Random Access Response Reception	1363

A.6.2.11.2.3	Receiving a NACK on msg3	1363

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1363

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1363

A.6.2.11.2.6	Contention Resolution Timer expiry	1363

A.6.2.11.2.7	PRACH Resource Selection	1364

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1364

A.6.2.12.1	Test Purpose and Environment	1364

A.6.2.12.2	Test Requirements	1366

A.6.2.12.2.1	Random Access Response Reception	1366

A.6.2.12.2.2	No Random Access Response Reception	1367

A.6.2.12.2.3	Receiving a NACK on msg3	1367

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1367

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1367

A.6.2.12.2.6	Contention Resolution Timer expiry	1367

A.6.2.12.2.7	PRACH Resource Selection	1368

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1368

A.6.2.13.1	Test Purpose and Environment	1368

A.6.2.13.2	Test Requirements	1370

A.6.2.13.2.1	Random Access Response Reception	1370

A.6.2.13.2.2	No Random Access Response Reception	1371

A.6.2.13.2.3	Receiving a NACK on msg3	1371

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1371

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1371

A.6.2.13.2.6	Contention Resolution Timer expiry	1371

A.6.2.13.2.7	PRACH Resource Selection	1372

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1372

A.6.2.14.1	Test Purpose and Environment	1372

A.6.2.14.2	Test Requirements	1374

A.6.2.14.2.1	Random Access Response Reception	1374

A.6.2.14.2.2	No Random Access Response Reception	1375

A.6.2.14.2.3	Receiving a NACK on msg3	1375

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1375

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1375

A.6.2.14.2.6	Contention Resolution Timer expiry	1375

A.6.2.14.2.7	PRACH Resource Selection	1376

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1376

A.6.2.15.1	Test Purpose and Environment	1376

A.6.2.15.2	Test Requirements	1378

A.6.2.15.2.1	Random Access Response Reception	1378

A.6.2.15.2.2	No Random Access Response Reception	1379

A.6.2.15.2.3	Receiving a NACK on msg3	1379

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1379

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1379

A.6.2.15.2.6	Contention Resolution Timer expiry	1379

A.6.2.15.2.7	PRACH Resource Selection	1380

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1380

A.6.2.16.1	Test Purpose and Environment	1380

A.6.2.16.2	Test Requirements	1383

A.6.2.16.2.1	Random Access Response Reception	1383

A.6.2.16.2.2	No Random Access Response Reception	1384

A.6.2.16.2.3	Receiving a NACK on msg3	1384

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1384

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1384

A.6.2.16.2.6	Contention Resolution Timer expiry	1384

A.6.2.16.2.7	NPRACH Resource Selection	1384

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1385

A.6.2.17.1	Test Purpose and Environment	1385

A.6.2.17.2	Test Requirements	1388

A.6.2.17.2.1	Random Access Response Reception	1388

A.6.2.17.2.2	No Random Access Response Reception	1389

A.6.2.17.2.3	Receiving a NACK on msg3	1389

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1389

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1389

A.6.2.17.2.6	Contention Resolution Timer expiry	1389

A.6.2.17.2.7	NPRACH Resource Selection	1389

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1390

A.6.2.18.1	Test Purpose and Environment	1390

A.6.2.18.2	Test Requirements	1393

A.6.2.18.2.1	Random Access Response Reception	1393

A.6.2.18.2.2	No Random Access Response Reception	1394

A.6.2.18.2.3	Receiving a NACK on msg3	1394

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1394

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1394

A.6.2.18.2.6	Contention Resolution Timer expiry	1394

A.6.2.18.2.7	NPRACH Resource Selection	1394

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1395

A.6.2.19.1	Test Purpose and Environment	1395

A.6.2.19.2	Test Requirements	1397

A.6.2.19.2.1	Random Access Response Reception	1397

A.6.2.19.2.2	No Random Access Response Reception	1398

A.6.2.19.2.3	Receiving a NACK on msg3	1398

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1398

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1398

A.6.2.19.2.6	Contention Resolution Timer expiry	1398

A.6.2.19.2.7	NPRACH Resource Selection	1398

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1399

A.6.2.20.1	Test Purpose and Environment	1399

A.6.2.20.2	Test Requirements	1401

A.6.2.20.2.1	Random Access Response Reception	1401

A.6.2.20.2.2	No Random Access Response Reception	1402

A.6.2.20.2.3	Receiving a NACK on msg3	1402

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1402

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1402

A.6.2.20.2.6	Contention Resolution Timer expiry	1402

A.6.2.20.2.7	NPRACH Resource Selection	1402

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1403

A.6.2.21.1	Test Purpose and Environment	1403

A.6.2.21.2	Test Requirements	1405

A.6.2.21.2.1	Random Access Response Reception	1405

A.6.2.21.2.2	No Random Access Response Reception	1406

A.6.2.21.2.3	Receiving a NACK on msg3	1406

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1406

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1406

A.6.2.21.2.6	Contention Resolution Timer expiry	1406

A.6.2.21.2.7	NPRACH Resource Selection	1406

A.6.3	RRC Connection Release with Redirection	1407

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1407

A.6.3.1.1	Test Purpose and Environment	1407

A.6.3.1.2	Test Requirements	1409

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1409

A.6.3.2.1	Test Purpose and Environment	1409

A.6.3.2.2	Test Requirements	1411

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1411

A.6.3.3.1	Test Purpose and Environment	1411

A.6.3.3.2	Test Requirements	1412

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1413

A.6.3.4.1	Test Purpose and Environment	1413

A.6.3.4.2	Test Requirements	1414

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1415

A.6.3.5.1	Test Purpose and Environment	1415

A.6.3.5.2	Test Requirements	1417

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1417

A.6.3.6.1	Test Purpose and Environment	1417

A. 6.3.6.2	Test Requirements	1420

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1420

A.6.3.7.1	Test Purpose and Environment	1420

A.6.3.7.2	Test Requirements	1423

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1423

A.6.3.8.1	Test Purpose and Environment	1423

A.6.3.8.2	Test Requirements	1426

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1426

A.6.3.9.1	Test Purpose and Environment	1426

A.6.3.9.2	Test Requirements	1428

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1428

A.6.3.10.1	Test Purpose and Environment	1428

A.6.3.10.2	Test Requirements	1430

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1430

A.6.3.11.1	Test Purpose and Environment	1430

A.6.3.11.2	Test Requirements	1432

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1432

A.6.3.12.1	Test Purpose and Environment	1432

A.6.3.12.2	Test Requirements	1435

A.7	Timing and Signalling Characteristics	1365

A.7.1	UE Transmit Timing	1365

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1365

A.7.1.1.1	Test Purpose and Environment	1365

A.7.1.1.2	Test Requirements	1366

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1367

A.7.1.2.1	Test Purpose and Environment	1367

A.7.1.2.2	Test Requirements	1369

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1370

A.7.1.3.1	Test Purpose and Environment	1370

A.7.1.3.2	Test Requirements	1372

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1373

A.7.1.4.1	Test Purpose and Environment	1373

A.7.1.4.2	Test Requirements	1374

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1375

A.7.1.4A.1	Test Purpose and Environment	1375

A.7.1.4A.2	Test Requirements	1375

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1375

A.7.1.5.1	Test Purpose and Environment	1375

A.7.1.5.2	Test Requirements	1376

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1376

A.7.1.6.1	Test Purpose and Environment	1376

A.7.1.6.2	Test Requirements	1378

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1379

A.7.1.7.1	Test Purpose and Environment	1379

A.7.1.7.2	Test Requirements	1380

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1381

A.7.1.7A.1	Test Purpose and Environment	1381

A.7.1.7A.2	Test Requirements	1381

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1381

A.7.1.7B.1	Test Purpose and Environment	1381

A.7.1.7B.2	Test Requirements	1381

A.7.1.8	Void	1382

A.7.1.8.1	Void	1382

A.7.1.8.2	Void	1382

A.7.1.9	Void	1382

A.7.1.9.1	Void	1382

A.7.1.9.2	Void	1382

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1382

A.7.1.10.1	Test Purpose and Environment	1382

A.7.1.10.2	Test Requirements	1384

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1384

A.7.1.11.1	Test Purpose and Environment	1384

A.7.1.11.2	Test Requirements	1386

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1386

A.7.1.12.1	Test Purpose and Environment	1386

A.7.1.12.2	Test Requirements	1388

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1388

A.7.1.13.1	Test Purpose and Environment	1388

A.7.1.13.2	Test Requirements	1390

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1391

A.7.1.14.1	Test Purpose and Environment	1391

A.7.1.14.2	Test Requirements	1392

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1392

A.7.1.15.1	Test Purpose and Environment	1392

A.7.1.15.2	Test Requirements	1393

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1394

A.7.1.16.1	Test Purpose and Environment	1394

A.7.1.16.2	Test Requirements	1395

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1396

A.7.1.17.1	Test Purpose and Environment	1396

A.7.1.17.2	Test Requirements	1398

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1399

A.7.1.18.1	Test Purpose and Environment	1399

A.7.1.18.2	Test Requirements	1401

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1402

A.7.1.19.1	Test Purpose and Environment	1402

A.7.1.19.2	Test Requirements	1403

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1403

A.7.1.20.1	Test Purpose and Environment	1403

A.7.1.20.2	Test Requirements	1405

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1405

A.7.1.21.1	Test Purpose and Environment	1405

A.7.1.21.2	Test Requirements	1407

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1407

A.7.1.22.1	Test Purpose and Environment	1407

A.7.1.22.2	Test Requirements	1409

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1409

A.7.1.23.1	Test Purpose and Environment	1409

A.7.1.23.2	Test Requirements	1411

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1411

A.7.1.24.1	Test Purpose and Environment	1411

A.7.1.24.2	Test Requirements	1412

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1413

A.7.1.25.1	Test Purpose and Environment	1413

A.7.1.25.2	Test Requirements	1414

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1415

A.7.1.26.1	Test Purpose and Environment	1415

A.7.1.26.2	Test Requirements	1416

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1417

A.7.1.27.1	Test Purpose and Environment	1417

A.7.1.27.2	Test Requirements	1419

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1419

A.7.1.28.1	Test Purpose and Environment	1419

A.7.1.28.2	Test Requirements	1421

A.7.2	UE Timing Advance	1422

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1422

A.7.2.1.1	Test Purpose and Environment	1422

A.7.2.1.2	Test Requirements	1423

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1424

A.7.2.2.1	Test Purpose and Environment	1424

A.7.2.2.2	Test Requirements	1426

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1426

A.7.2.3.1	Test Purpose and Environment	1426

A.7.2.3.2	Test Requirements	1426

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1426

A.7.2.4.1	Test Purpose and Environment	1426

A.7.2.4.2	Test Requirements	1429

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1429

A.7.2.5.1	Test Purpose and Environment	1429

A.7.2.5.2	Test Requirements	1431

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1431

A.7.2.5A.1	Test Purpose and Environment	1431

A.7.2.5A.2	Test Requirements	1432

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1432

A.7.2.5B.1	Test Purpose and Environment	1432

A.7.2.5B.2	Test Requirements	1432

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1432

A.7.2.6.1	Test Purpose and Environment	1432

A.7.2.6.2	Test Requirements	1435

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1435

A.7.2.7.1	Test Purpose and Environment	1435

A.7.2.7.2	Test Requirements	1437

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1437

A.7.2.8.1	Test Purpose and Environment	1437

A.7.2.8.2	Test Requirements	1439

A.7.2.9.2	Test Requirements	1440

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1441

A.7.2.10.1	Test Purpose and Environment	1441

A.7.2.10.2	Test Requirements	1442

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1442

A.7.2.11.1	Test Purpose and Environment	1442

A.7.2.11.2	Test Requirements	1444

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1444

A.7.2.12.1	Test Purpose and Environment	1444

A.7.2.12.2	Test Requirements	1446

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1446

A.7.2.13.1	Test Purpose and Environment	1446

A.7.2.13.2	Test Requirements	1448

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1448

A.7.2.14.1	Test Purpose and Environment	1448

A.7.2.14.2	Test Requirements	1450

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1450

A.7.2.15.1	Test Purpose and Environment	1450

A.7.2.15.2	Test Requirements	1452

A.7.3	Radio Link Monitoring	1452

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1453

A.7.3.1.1	Test Purpose and Environment	1453

A.7.3.1.2	Test Requirements	1457

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1457

A.7.3.2.1	Test Purpose and Environment	1457

A.7.3.2.2	Test Requirements	1460

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1460

A.7.3.3.1	Test Purpose and Environment	1460

A.7.3.3.2	Test Requirements	1464

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1464

A.7.3.4.1	Test Purpose and Environment	1464

A.7.3.4.2	Test Requirements	1467

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1467

A.7.3.5.1	Test Purpose and Environment	1467

A.7.3.5.2	Test Requirements	1471

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1472

A.7.3.6.1	Test Purpose and Environment	1472

A.7.3.6.2	Test Requirements	1475

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1475

A.7.3.7.1	Test Purpose and Environment	1475

A.7.3.7.2	Test Requirements	1478

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1478

A.7.3.8.1	Test Purpose and Environment	1478

A.7.3.8.2	Test Requirements	1481

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1481

A.7.3.9.2	Test Requirements	1484

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1484

A.7.3.10.1	Test Purpose and Environment	1484

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1487

A.7.3.11.1	Test Purpose and Environment	1487

A.7.3.11.2	Test Requirements	1491

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1491

A.7.3.12.1	Test Purpose and Environment	1491

A.7.3.12.2	Test Requirements	1495

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1495

A.7.3.13.1	Test Purpose and Environment	1495

A.7.3.13.2	Test Requirements	1498

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1498

A.7.3.14.1	Test Purpose and Environment	1498

A.7.3.14.2	Test Requirements	1501

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1501

A.7.3.15.1	Test Purpose and Environment	1501

A.7.3.15.2	Test Requirements	1504

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1504

A.7.3.16.1	Test Purpose and Environment	1504

A.7.3.16.2	Test Requirements	1508

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1508

A.7.3.17.1	Test Purpose and Environment	1508

A.7.3.17.2	Test Requirements	1511

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1511

A.7.3.18.1	Test Purpose and Environment	1511

A.7.3.18.2	Test Requirements	1515

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1515

A.7.3.19.1	Test Purpose and Environment	1515

A.7.3.19.2	Test Requirements	1519

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1519

A.7.3.20.1	Test Purpose and Environment	1519

A.7.3.20.2	Test Requirements	1523

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1523

A.7.3.21.1	Test Purpose and Environment	1523

A.7.3.21.2	Test Requirements	1527

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1527

A.7.3.22.1	Test Purpose and Environment	1527

A.7.3.22.2	Test Requirements	1531

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1531

A.7.3.23.1	Test Purpose and Environment	1531

A.7.3.23.2	Test Requirements	1532

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1532

A.7.3.24.1	Test Purpose and Environment	1532

A.7.3.24.2	Test Requirements	1533

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1533

A.7.3.25.1	Test Purpose and Environment	1533

A.7.3.25.2	Test Requirements	1534

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1534

A.7.3.26.1	Test Purpose and Environment	1534

A.7.3.26.2	Test Requirements	1537

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1537

A.7.3.27.1	Test Purpose and Environment	1537

A.7.3.27.2	Test Requirements	1540

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1540

A.7.3.28.1	Test Purpose and Environment	1540

A.7.3.28.2	Test Requirements	1543

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1543

A.7.3.29.1	Test Purpose and Environment	1543

A.7.3.29.2	Test Requirements	1546

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1546

A.7.3.30.1	Test Purpose and Environment	1546

A.7.3.30.2	Test Requirements	1549

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1549

A.7.3.31.1	Test Purpose and Environment	1549

A.7.3.31.2	Test Requirements	1552

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1552

A.7.3.32.1	Test Purpose and Environment	1552

A.7.3.32.2	Test Requirements	1555

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1555

A.7.3.33.1	Test Purpose and Environment	1555

A.7.3.33.2	Test Requirements	1558

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1558

A.7.3.34.1	Test Purpose and Environment	1558

A.7.3.34.2	Test Requirements	1561

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1561

A.7.3.35.1	Test Purpose and Environment	1561

A.7.3.35.2	Test Requirements	1564

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1564

A.7.3.36.1	Test Purpose and Environment	1564

A.7.3.36.2	Test Requirements	1567

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1567

A.7.3.37.1	Test Purpose and Environment	1567

A.7.3.37.2	Test Requirements	1570

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1570

A.7.3.38.1	Test Purpose and Environment	1570

A.7.3.38.2	Test Requirements	1573

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1573

A.7.3.39.1	Test Purpose and Environment	1573

A.7.3.39.2	Test Requirements	1576

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1577

A.7.3.40.1	Test Purpose and Environment	1577

A.7.3.40.2	Test Requirements	1579

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1580

A.7.3.41.1	Test Purpose and Environment	1580

A.7.3.41.2	Test Requirements	1583

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1583

A.7.3.42.1	Test Purpose and Environment	1583

A.7.3.42.2	Test Requirements	1586

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1586

A.7.3.43.1	Test Purpose and Environment	1586

A.7.3.43.2	Test Requirements	1589

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1589

A.7.3.44.1	Test Purpose and Environment	1589

A.7.3.44.2	Test Requirements	1592

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1593

A.7.3.45.1	Test Purpose and Environment	1593

A.7.3.45.2	Test Requirements	1595

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1596

A.7.3.46.1	Test Purpose and Environment	1596

A.7.3.46.2	Test Requirements	1599

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1599

A.7.3.47.1	Test Purpose and Environment	1599

A.7.3.47.2	Test Requirements	1602

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1602

A.7.3.48.1	Test Purpose and Environment	1602

A.7.3.48.2	Test Requirements	1605

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1605

A.7.3.49.1	Test Purpose and Environment	1605

A.7.3.49.2	Test Requirements	1608

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1608

A.7.3.50.1	Test Purpose and Environment	1608

A.7.3.50.2	Test Requirements	1611

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1611

A.7.3.51.1	Test Purpose and Environment	1611

A.7.3.51.2	Test Requirements	1614

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1614

A.7.3.52.1	Test Purpose and Environment	1614

A.7.3.52.2	Test Requirements	1617

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1617

A.7.3.53.1	Test Purpose and Environment	1617

A.7.3.53.2	Test Requirements	1620

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1620

A.7.3.54.1	Test Purpose and Environment	1620

A.7.3.54.2	Test Requirements	1623

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1623

A.7.3.55.1	Test Purpose and Environment	1623

A.7.3.55.2	Test Requirements	1626

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1626

A.7.3.56.1	Test Purpose and Environment	1626

A.7.3.56.2	Test Requirements	1629

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1629

A.7.3.57.1	Test Purpose and Environment	1629

A.7.3.57.2	Test Requirements	1632

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1632

A.7.3.58.1	Test Purpose and Environment	1632

A.7.3.58.2	Test Requirements	1635

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1635

A.7.3.59.1	Test Purpose and Environment	1635

A.7.3.59.2	Test Requirements	1638

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1638

A.7.3.60.1	Test Purpose and Environment	1638

A.7.3.60.2	Test Requirements	1642

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1642

A.7.3.61.1	Test Purpose and Environment	1642

A.7.3.61.2	Test Requirements	1646

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1646

A.7.3.62.1	Test Purpose and Environment	1646

A.7.3.62.2	Test Requirements	1650

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1650

A.7.3.63.1	Test Purpose and Environment	1650

A.7.3.63.2	Test Requirements	1654

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1654

A.7.3.64.1	Test Purpose and Environment	1654

A.7.3.64.2	Test Requirements	1657

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1658

A.7.3.65.1	Test Purpose and Environment	1658

A.7.3.65.2	Test Requirements	1661

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1662

A.7.3.66.1	Test Purpose and Environment	1662

A.7.3.66.2	Test Requirements	1664

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1665

A.7.3.67.1	Test Purpose and Environment	1665

A.7.3.67.2	Test Requirements	1668

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1669

A.7.3.68.1	Test Purpose and Environment	1669

A.7.3.68.2	Test Requirements	1671

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1671

A.7.3.69.1	Test Purpose and Environment	1671

A.7.3.69.2	Test Requirements	1673

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1673

A.7.3.70.1	Test Purpose and Environment	1673

A.7.3.70.2	Test Requirements	1675

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1675

A.7.3.71.1	Test Purpose and Environment	1675

A.7.3.71.2	Test Requirements	1678

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1678

A.7.3.72.1	Test Purpose and Environment	1678

A.7.3.72.2	Test Requirements	1680

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1680

A.7.3.73.1	Test Purpose and Environment	1680

A.7.3.73.2	Test Requirements	1682

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1682

A.7.3.74.1	Test Purpose and Environment	1682

A.7.3.74.2	Test Requirements	1685

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1685

A.7.3.75.1	Test Purpose and Environment	1685

A.7.3.75.2	Test Requirements	1687

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1688

A.7.3.76.1	Test Purpose and Environment	1688

A.7.3.76.2	Test Requirements	1691

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1691

A.7.3.77.1	Test Purpose and Environment	1691

A.7.3.77.2	Test Requirements	1694

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1694

A.7.3.78.1	Test Purpose and Environment	1694

A.7.3.78.2	Test Requirements	1697

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1697

A.7.3.79.1	Test Purpose and Environment	1697

A.7.3.79.2	Test Requirements	1700

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1700

A.7.3.80.1	Test Purpose and Environment	1700

A.7.3.80.2	Test Requirements	1703

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1703

A.7.3.81.1	Test Purpose and Environment	1703

A.7.3.81.2	Test Requirements	1706

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1706

A.7.3.82.1	Test Purpose and Environment	1706

A.7.3.82.2	Test Requirements	1709

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1709

A.7.3.83.1	Test Purpose and Environment	1709

A.7.3.83.2	Test Requirements	1710

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1711

A.7.3.84.1	Test Purpose and Environment	1711

A.7.3.84.2	Test Requirements	1713

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1713

A.7.3.85.1	Test Purpose and Environment	1713

A.7.3.85.2	Test Requirements	1714

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	1715

A.7.3.86.1	Test Purpose and Environment	1715

A.7.3.86.2	Test Requirements	1717

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	1717

A.7.3.87.1	Test Purpose and Environment	1717

A.7.3.87.2	Test Requirements	1719

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1720

A.7.3.88.1	Test Purpose and Environment	1720

A.7.3.88.2	Test Requirements	1724

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1724

A.7.3.89.1	Test Purpose and Environment	1724

A.7.3.89.2	Test Requirements	1728

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1728

A.7.3.90.1	Test Purpose and Environment	1728

A.7.3.90.2	Test Requirements	1732

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1732

A.7.3.91.1	Test Purpose and Environment	1732

A.7.3.91.2	Test Requirements	1736

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1736

A.7.3.92.1	Test Purpose and Environment	1736

A.7.3.92.2	Test Requirements	1739

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1740

A.7.3.93.1	Test Purpose and Environment	1740

A.7.3.93.2	Test Requirements	1743

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1744

A.7.3.94.1	Test Purpose and Environment	1744

A.7.3.94.2	Test Requirements	1746

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1747

A.7.3.95.1	Test Purpose and Environment	1747

A.7.3.95.2	Test Requirements	1750

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1750

A.7.3.96.1	Test Purpose and Environment	1750

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1752

A.7.3.97.1	Test Purpose and Environment	1752

A.7.3.97.2	Test Requirements	1755

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	1755

A.7.3.98.1	Test Purpose and Environment	1755

A.7.3.98.2	Test Requirements	1758

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1758

A.7.3.99.1	Test Purpose and Environment	1758

A.7.3.99.2	Test Requirements	1761

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1761

A.7.3.100.1	Test Purpose and Environment	1761

A.7.3.100.2	Test Requirements	1763

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	1763

A.7.3.101.1	Test Purpose and Environment	1763

A.7.3.101.2	Test Requirements	1765

A.7.4	Interruption for Dual Connectivity	1765

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1765

A.7.4.1.1	Test Purpose and Environment	1765

A.7.4.1.2	Test Requirements	1768

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1768

A.7.4.2.1	Test Purpose and Environment	1768

A.7.4.2.2	Test Requirements	1770

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	1770

A.7.4.3.1	Test Purpose and Environment	1770

A.7.4.3.2	Test Requirements	1772

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1772

A.7.4.4.1	Test Purpose and Environment	1772

A.7.4.4.2	Test Requirements	1775

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	1775

A.7.4.5.1	Test Purpose and Environment	1775

A.7.4.5.2	Test Requirements	1777

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	1777

A.7.4.6.1	Test Purpose and Environment	1777

A.7.4.6.2	Test Requirements	1779

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	1779

A.7.4.7.1	Test Purpose and Environment	1779

A.7.4.7.2	Test Requirements	1782

A.7.5	Proximity-based Services	1782

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1782

A.7.5.1.1	Test Purpose and Environment	1782

A.7.5.1.2	Test Requirements	1783

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	1784

A.7.5.2.1	Test Purpose and Environment	1784

A.7.5.1.2	Test Requirements	1784

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	1785

A.7.5.3.1	Test Purpose and Environment	1785

A.7.5.3.2	Test Requirements	1786

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	1787

A.7.5.4.1	Test Purpose and Environment	1787

A.7.5.4.2	Test Requirements	1788

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	1789

A.7.5.5.1	Test Purpose and Environment	1789

A.7.5.5.2	Test Requirements	1791

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	1792

A.7.5.6.1	Test Purpose and Environment	1792

A.7.5.6.2	Test Requirements	1793

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	1794

A.7.5.7.1	Test Purpose and Environment	1794

A.7.5.7.2	Test Requirements	1796

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	1796

A.7.5.8.1	Test Purpose and Environment	1796

A.7.5.8.2	Test Requirements	1798

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	1799

A.7.5.9.1	Test Purpose and Environment	1799

A.7.5.9.2	Test Requirements	1801

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	1802

A.7.5.10.1	Test Purpose and Environment	1802

A.7.5.10.2	Test Requirements	1804

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	1805

A.7.5.11.1	Test Purpose and Environment	1805

A.7.5.11.2	Test Requirements	1807

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	1807

A.7.5.12.1	Test Purpose and Environment	1807

A.7.5.12.2	Test Requirements	1811

A.7.6	Interruption for carrier aggregation	1812

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	1812

A.7.6.1.1	Test Purpose and Environment	1812

A.7.6.1.2	Test Requirements	1814

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	1814

A.7.6.2.1	Test Purpose and Environment	1814

A.7.6.2.2	Test Requirements	1817

A.8	UE Measurements Procedures	1817

A.8.1	E-UTRAN FDD Intra-frequency Measurements	1817

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1817

A.8.1.1.1	Test Purpose and Environment	1817

A.8.1.1.2	Test Requirements	1819

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1819

A.8.1.2.1	Test Purpose and Environment	1819

A.8.1.2.2	Test Requirements	1821

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1821

A.8.1.3.1	Test Purpose and Environment	1821

A.8.1.3.2	Test Requirements	1824

A.8.1.4	Void	1824

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1824

A.8.1.5.1	Test Purpose and Environment	1824

A.8.1.5.2	Test Requirements	1826

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1826

A.8.1.6.1	Test Purpose and Environment	1826

A.8.1.6.2	Test Requirements	1828

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1828

A.8.1.7.1	Test Purpose and Environment	1828

A.8.1.7.2	Test Requirements	1830

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1831

A.8.1.8.1	Test Purpose and Environment	1831

A.8.1.8.2	Test Requirements	1833

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	1834

A.8.1.9.1	Test Purpose and Environment	1834

A.8.1.9.2	Test Requirements	1834

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	1834

A.8.1.10.1	Test Purpose and Environment	1834

A.8.1.10.2	Test Requirements	1835

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1835

A.8.1.11.1	Test Purpose and Environment	1835

A.8.1.11.2	Test Requirements	1837

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1838

A.8.1.12.1	Test Purpose and Environment	1838

A.8.1.12.2	Test Requirements	1839

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1840

A.8.1.13.1	Test Purpose and Environment	1840

A.8.1.13.2	Test Requirements	1842

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	1842

A.8.1.14.1	Test Purpose and Environment	1842

A.8.1.14.2	Test Requirements	1844

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1845

A.8.1.15.1	Test Purpose and Environment	1845

A.8.1.15.2	Test Requirements	1846

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1847

A.8.1.16.1	Test Purpose and Environment	1847

A.8.1.16.2	Test Requirements	1849

A.8.1.17	Void	1849

A.8.1.18	Void	1849

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1849

A.8.1.19.1	Test Purpose and Environment	1849

A.8.1.19.2	Test Requirements	1851

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1852

A.8.1.20.1	Test Purpose and Environment	1852

A.8.1.20.2	Test Requirements	1854

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	1854

A.8.1.21.1	Test Purpose and Environment	1854

A.8.1.21.2	Test Requirements	1856

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	1857

A.8.1.22.1	Test Purpose and Environment	1857

A.8.1.22.2	Test Requirements	1859

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1859

A.8.1.23.1	Test Purpose and Environment	1859

A.8.1.23.2	Test Requirements	1860

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1861

A.8.1.24.1	Test Purpose and Environment	1861

A.8.1.24.2	Test Requirements	1863

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1863

A.8.1.25.1	Test Purpose and Environment	1863

A.8.1.25.2	Test Requirements	1865

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	1865

A.8.1.26.1	Test Purpose and Environment	1865

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1868

A.8.1.27.1	Test Purpose and Environment	1868

A.8.1.27.2	Test Requirements	1870

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1870

A.8.1.28.1	Test Purpose and Environment	1870

A.8.1.28.2	Test Requirements	1872

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	1872

A.8.1.29.1	Test Purpose and Environment	1872

A.8.1.29.2	Test Requirements	1874

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	1875

A.8.1.30.1	Test Purpose and Environment	1875

A.8.1.30.2	Test Requirements	1877

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1877

A.8.1.31.1	Test Purpose and Environment	1877

A.8.1.31.2	Test Requirements	1879

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1880

A.8.1.32.1	Test Purpose and Environment	1880

A.8.1.32.2	Test Requirements	1881

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	1882

A.8.1.33.1	Test Purpose and Environment	1882

A.8.1.33.2	Test Requirements	1883

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1884

A.8.1.34.1	Test Purpose and Environment	1884

A.8.1.34.2	Test Requirements	1885

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	1886

A.8.1.35.1	Test Purpose and Environment	1886

A.8.1.35.2	Test Requirements	1887

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1888

A.8.1.36.1	Test Purpose and Environment	1888

A.8.1.36.2	Test Requirements	1889

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1890

A.8.1.37.1	Test Purpose and Environment	1890

A.8.1.37.2	Test Requirements	1892

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1892

A.8.1.38.1	Test Purpose and Environment	1892

A.8.1.38.2	Test Requirements	1893

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1894

A.8.1.39.1	Test Purpose and Environment	1894

A.8.1.39.2	Test Requirements	1896

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1896

A.8.1.40.1	Test Purpose and Environment	1896

A.8.1.40.2	Test Requirements	1898

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1898

A.8.1.41.1	Test Purpose and Environment	1898

A.8.1.41.2	Test Requirement	1900

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1900

A.8.1.42.1	Test Purpose and Environment	1900

A.8.1.42.2	Test Requirement	1901

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1901

A.8.1.43.1	Test Purpose and Environment	1901

A.8.1.43.2	Test Requirements	1904

A.8.2	E-UTRAN TDD Intra-frequency Measurements	1904

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	1904

A.8.2.1.1	Test Purpose and Environment	1904

A.8.2.1.2	Test Requirements	1906

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	1907

A.8.2.2.1	Test Purpose and Environment	1907

A.8.2.2.2	Test Requirements	1909

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1909

A.8.2.3.1	Test Purpose and Environment	1909

A.8.2.3.2	Test Requirements	1911

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1912

A.8.2.4.1	Test Purpose and Environment	1912

A.8.2.4.2	Test Requirements	1914

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1914

A.8.2.5.1	Test Purpose and Environment	1914

A.8.2.5.2	Test Requirements	1916

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1917

A.8.2.6.1	Test Purpose and Environment	1917

A.8.2.6.2	Test Requirements	1919

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1920

A.8.2.7.1	Test Purpose and Environment	1920

A.8.2.7.2	Test Requirements	1921

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1922

A.8.2.8.1	Test Purpose and Environment	1922

A.8.2.8.2	Test Requirements	1924

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	1924

A.8.2.9.1	Test Purpose and Environment	1924

A.8.2.9.2	Test Requirements	1926

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	1927

A.8.2.10.1	Test Purpose and Environment	1927

A.8.2.10.2	Test Requirements	1929

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	1929

A.8.2.11.1	Test Purpose and Environment	1929

A.8.2.11.2	Test Requirements	1931

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	1931

A.8.2.12.1	Test Purpose and Environment	1931

A.8.2.12.2	Test Requirements	1933

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	1934

A.8.2.13.1	Test Purpose and Environment	1934

A.8.2.13.2	Test Requirements	1936

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	1936

A.8.2.14.1	Test Purpose and Environment	1936

A.8.2.14.2	Test Requirement	1938

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	1938

A.8.2.15.1	Test Purpose and Environment	1938

A.8.2.15.2	Test Requirements	1940

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	1940

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	1940

A.8.3.1.1	Test Purpose and Environment	1940

A.8.3.1.2	Test Requirements	1942

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	1942

A.8.3.2.1	Test Purpose and Environment	1942

A.8.3.2.2	Test Requirements	1945

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	1945

A.8.3.3.1	Test Purpose and Environment	1945

A.8.3.3.2	Test Requirements	1948

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1948

A.8.3.4.1	Test Purpose and Environment	1948

A.8.3.4.2	Test Requirements	1950

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1950

A.8.3.5.2	Test Requirements	1952

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	1952

A.8.3.6.1	Test Purpose and Environment	1952

A.8.3.6.2	Test Requirements	1954

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	1955

A.8.3.7.1	Test Purpose and Environment	1955

A.8.3.7.2	Test Requirements	1957

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	1958

A.8.3.8.1	Test Purpose and Environment	1958

A.8.3.8.2	Test Requirements	1961

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	1961

A.8.3.9.1	Test Purpose and Environment	1961

A.8.3.9.2	Test Requirements	1964

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	1965

A.8.3.10.1	Test Purpose and Environment	1965

A.8.3.10.2	Test Requirements	1966

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	1966

A.8.3.11.1	Test Purpose and Environment	1966

A.8.3.11.2	Test Requirement	1968

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1969

A.8.3.12.1	Test Purpose and Environment	1969

A.8.3.12.2	Test Requirement	1970

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	1971

A.8.3.13.1	Test Purpose and Environment	1971

A.8.3.13.2	Test Requirement	1972

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1973

A.8.3.14.1	Test Purpose and Environment	1973

A.8.3.14.2	Test Requirement	1974

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	1975

A.8.3.15.1	Test Purpose and Environment	1975

A.8.3.15.2	Test Requirement	1976

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	1977

A.8.3.16.1	Test Purpose and Environment	1977

A.8.3.16.2	Test Requirement	1979

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	1979

A.8.3.17.1	Test Purpose and Environment	1979

A.8.3.17.2	Test Requirement	1982

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1982

A.8.3.18.1	Test Purpose and Environment	1982

A.8.3.18.2	Test Requirement	1985

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	1985

A.8.3.19.1	Test Purpose and Environment	1985

A.8.3.19.2	Test Requirement	1988

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	1988

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	1988

A.8.4.1.1	Test Purpose and Environment	1988

A.8.4.1.2	Test Requirements	1990

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	1990

A.8.4.2.1	Test Purpose and Environment	1990

A.8.4.2.2	Test Requirements	1993

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	1993

A.8.4.3.1	Test Purpose and Environment	1993

A.8.4.3.2	Test Requirements	1995

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	1996

A.8.4.4.1	Test Purpose and Environment	1996

A.8.4.4.2	Test Requirements	1998

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	1999

A.8.4.5.1	Test Purpose and Environment	1999

A.8.4.5.2	Test Requirements	2001

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2001

A.8.4.6.1	Test Purpose and Environment	2001

A.8.4.6.2	Test Requirements	2002

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2002

A.8.4.7.1	Test Purpose and Environment	2002

A.8.4.7.2	Test Requirements	2006

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2006

A.8.4.8.1	Test Purpose and Environment	2006

A.8.4.8.2	Test Requirements	2009

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2009

A.8.4.9.1	Test Purpose and Environment	2009

A.8.4.9.2	Test Requirements	2013

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2013

A.8.4.10.1	Test Purpose and Environment	2013

A.8.4.10.2	Test Requirements	2015

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2015

A.8.4.11.1	Test Purpose and Environment	2015

A.8.4.11.2	Test Requirement	2017

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2017

A.8.4.12.1	Test Purpose and Environment	2017

A.8.4.12.2	Test Requirement	2019

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2020

A.8.4.13.1	Test Purpose and Environment	2020

A.8.4.13.2	Test Requirement	2021

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2022

A.8.4.14.1	Test Purpose and Environment	2022

A.8.4.14.2	Test Requirement	2024

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2024

A.8.4.15.1	Test Purpose and Environment	2024

A.8.4.15.2	Test Requirement	2027

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2027

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2027

A.8.5.1.1	Test Purpose and Environment	2027

A.8.5.1.2	Test Requirements	2029

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2029

A.8.5.2.1	Test Purpose and Environment	2029

A.8.5.2.2	Test Requirements	2031

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2031

A.8.5.3.1	Test Purpose and Environment	2031

A.8.5.3.2	Test Requirements	2034

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2034

A.8.5.4.1	Test Purpose and Environment	2034

A.8.5.4.2	Test Requirements	2036

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2036

A.8.5.5.1	Test Purpose and Environment	2036

A.8.5.5.2	Test Requirements	2039

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2039

A.8.5.6.1	Test Purpose and Environment	2039

A.8.5.6.2	Test Requirements	2040

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2041

A.8.5.7.1	Test Purpose and Environment	2041

A.8.5.7.2	Test Requirements	2041

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2041

A.8.5.8.1	Test Purpose and Environment	2041

A.8.5.8.2	Test Requirements	2044

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2044

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2044

A.8.6.1.1	Test Purpose and Environment	2044

A.8.6.1.2	Test Requirements	2046

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2046

A.8.6.2.1	Test Purpose and Environment	2046

A.8.6.2.2	Test Requirements	2049

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2049

A.8.6.3.1	Test Purpose and Environment	2049

A.8.6.3.2	Test Requirements	2052

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2052

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2052

A.8.7.1.1	Test Purpose and Environment	2052

A.8.7.1.1.1	Void	2052

A.8.7.1.1.2	1.28 Mcps TDD option	2052

A.8.7.1.1.3	Void	2054

A.8.7.1.2	Test Requirements	2054

A.8.7.1.2.1	Void	2054

A.8.7.1.2.2	1.28 Mcps TDD option	2054

A.8.7.1.2.3	Void	2054

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2054

A.8.7.2.1	Test Purpose and Environment	2054

A.8.7.2.2	Test Requirements	2057

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2058

A.8.7.3.1	Test Purpose and Environment	2058

A.8.7.3.2	Test Parameters	2058

A.8.7.3.3	Test Requirements	2059

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2060

A.8.7.4.1	Test Purpose and Environment	2060

A.8.7.4.2	Test Requirements	2062

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2062

A.8.7.5.1	Test Purpose and Environment	2062

A.8.7.5.2	Test Requirements	2064

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2064

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2064

A.8.7A.1.1	Test Purpose and Environment	2064

A.8.7A.1.2	Test Requirements	2067

A.8.8	E-UTRAN FDD – GSM Measurements	2067

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2067

A.8.8.1.1	Test Purpose and Environment	2067

A.8.8.1.2	Test Requirements	2069

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2069

A.8.8.2.1	Test Purpose and Environment	2069

A.8.8.2.2	Test Requirements	2071

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2072

A.8.8.3.1	Test Purpose and Environment	2072

A.8.8.3.2	Test Requirements	2073

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2074

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2074

A.8.9.1.1	Test Purpose and Environment	2074

A.8.9.1.2	Test Requirements	2075

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2076

A.8.9.2.1	Test Purpose and Environment	2076

A.8.9.2.2	Test Requirements	2078

A.8.10	E-UTRAN TDD – GSM Measurements	2078

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2078

A.8.10.1.1	Test Purpose and Environment	2078

A.8.10.1.2	Test Requirements	2079

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2080

A.8.10.2.1	Test Purpose and Environment	2080

A.8.10.2.2	Test Requirements	2082

A.8.11	Monitoring of Multiple Layers	2082

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2082

A.8.11.1.1	Test Purpose and Environment	2082

A.8.11.1.2	Test Requirements	2084

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2085

A.8.11.2.1	Test Purpose and Environment	2085

A.8.11.2.2	Test Requirements	2086

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2087

A.8.11.3.1	Test Purpose and Environment	2087

A.8.11.3.2	Test Requirements	2089

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2089

A.8.11.4.1	Test Purpose and Environment	2089

A.8.11.4.2	Test Requirements	2092

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2092

A.8.11.5.1	Test Purpose and Environment	2092

A.8.11.5.2	Test Requirements	2094

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2095

A.8.11.6.1	Test Purpose and Environment	2095

A.8.11.6.2	Test Requirements	2097

A.8.12	RSTD Intra-frequency Measurements	2098

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2098

A.8.12.1.1	Test Purpose and Environment	2098

A.8.12.1.2	Test Requirements	2101

A.8.12.1.2A	Test Requirements for UE Category 1bis	2102

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2102

A.8.12.2.1	Test Purpose and Environment	2102

A.8.12.2.2	Test Requirements	2106

A.8.12.2.2A	Test Requirements for UE Category 1bis	2107

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2107

A.8.12.3.1	Test Purpose and Environment	2107

A.8.12.3.2	Test Requirements	2111

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2112

A.8.12.4.1	Test Purpose and Environment	2112

A.8.12.4.2	Test Requirements	2115

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2116

A.8.12.5.1	Test Purpose and Environment	2116

A.8.12.5.2	Test Requirements	2119

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2120

A.8.12.6.1	Test Purpose and Environment	2120

A.8.12.6.2	Test Requirements	2123

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2124

A.8.12.7.1	Test Purpose and Environment	2124

A.8.12.7.2	Test Requirements	2127

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2128

A.8.12.8.1	Test Purpose and Environment	2128

A.8.12.8.2	Test Requirements	2131

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2132

A.8.12.9.1	Test Purpose and Environment	2132

A.8.12.9.2	Test Requirements	2135

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2136

A.8.12.10.1	Test Purpose and Environment	2136

A.8.12.10.2	Test Requirements	2139

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2140

A.8.12.11.1	Test Purpose and Environment	2140

A.8.12.11.2	Test Requirements	2144

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2145

A.8.12.12.1	Test Purpose and Environment	2145

A.8.12.12.2	Test Requirements	2148

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2149

A.8.12.13.1	Test Purpose and Environment	2149

A.8.12.13.2	Test Requirements	2152

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2153

A.8.12.14.1	Test Purpose and Environment	2153

A.8.12.14.2	Test Requirements	2157

A.8.13	RSTD Inter-frequency Measurements	2158

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2158

A.8.13.1.1	Test Purpose and Environment	2158

A.8.13.1.2	Test Requirements	2161

A.8.13.1.2A	Test Requirements for UE Category 1bis	2162

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2162

A.8.13.2.1	Test Purpose and Environment	2162

A.8.13.2.2	Test Requirements	2166

A.8.13.2.2A	Test Requirements for UE Category 1bis	2167

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2167

A.8.13.3.1	Test Purpose and Environment	2167

A.8.13.3.2	Test Requirements	2171

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2172

A.8.13.4.1	Test Purpose and Environment	2172

A.8.13.4.2	Test Requirements	2175

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2176

A.8.13.5.1	Test Purpose and Environment	2176

A.8.13.5.2	Test Requirements	2180

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2181

A.8.13.6.1	Test Purpose and Environment	2181

A.8.13.6.2	Test Requirements	2184

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2185

A.8.13.7.1	Test Purpose and Environment	2185

A.8.13.7.2	Test Requirements	2188

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2189

A.8.13.8.1	Test Purpose and Environment	2189

A.8.13.8.2	Test Requirements	2193

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2194

A.8.13.9.1	Test Purpose and Environment	2194

A.8.13.9.2	Test Requirements	2198

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2199

A.8.13.10.1	Test Purpose and Environment	2199

A.8.13.10.2	Test Requirements	2203

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2204

A.8.13.11.1	Test Purpose and Environment	2204

A.8.13.11.2	Test Requirements	2208

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2209

A.8.13.12.1	Test Purpose and Environment	2209

A.8.13.12.2	Test Requirements	2213

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2214

A.8.13.13.1	Test Purpose and Environment	2214

A.8.13.13.2	Test Requirements	2218

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2219

A.8.13.14.1	Test Purpose and Environment	2219

A.8.13.14.2	Test Requirements	2223

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2224

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2224

A.8.14.1.1	Test Purpose and Environment	2224

A.8.14.1.2	Test Requirements	2226

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2226

A.8.14.2.1	Test Purpose and Environment	2226

A.8.14.2.2	Test Requirements	2229

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2229

A.8.14.3.1	Test Purpose and Environment	2229

A.8.14.3.2	Test Requirements	2231

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2232

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2232

A.8.15.1.1	Test Purpose and Environment	2232

A.8.15.1.2	Test Requirements	2233

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2233

A.8.15.2.1	Test Purpose and Environment	2233

A.8.15.2.2	Test Requirements	2236

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2236

A.8.15.3.1	Test Purpose and Environment	2236

A.8.15.3.2	Test Requirements	2238

A.8.16	E-UTRAN Carrier Aggregation Measurements	2239

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2239

A.8.16.1.1	Test Purpose and Environment	2239

A.8.16.1.2	Test Requirements	2241

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2241

A.8.16.2.1	Test Purpose and Environment	2241

A.8.16.2.2	Test Requirements	2243

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2243

A.8.16.3.1	Test Purpose and Environment	2243

A.8.16.3.2	Test Requirements	2245

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2246

A.8.16.3A.1	Test Purpose and Environment	2246

A.8.16.3A.2	Test Requirements	2247

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2248

A.8.16.4.1	Test Purpose and Environment	2248

A.8.16.4.2	Test Requirements	2249

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2250

A.8.16.4A.1	Test Purpose and Environment	2250

A.8.16.4A.2	Test Requirements	2252

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2253

A.8.16.5.1	Test Purpose and Environment	2253

A.8.16.5.2	Test Requirements	2253

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2253

A.8.16.6.1	Test Purpose and Environment	2253

A.8.16.6.2	Test Requirements	2254

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2254

A.8.16.7.1	 Test Purpose and Environment	2254

A.8.16.7.2	 Test Requirements	2255

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2255

A.8.16.8.1	 Test Purpose and Environment	2255

A.8.16.8.2	 Test Requirements	2256

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2256

A.8.16.9.1	Test Purpose and Environment	2256

A.8.16.9.2	Test Requirements	2257

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2257

A.8.16.10.1	Test Purpose and Environment	2257

A.8.16.10.2	Test Requirements	2258

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2258

A.8.16.11.1	 Test Purpose and Environment	2258

A.8.16.11.2	 Test Requirements	2259

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2259

A.8.16.12.1	 Test Purpose and Environment	2259

A.8.16.12.2	 Test Requirements	2259

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2259

A.8.16.13.1	Test Purpose and Environment	2259

A.8.16.13.2	Test Requirements	2260

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2260

A.8.16.14.1	Test Purpose and Environment	2260

A.8.16.14.2	Test Requirements	2261

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2261

A.8.16.15.1	 Test Purpose and Environment	2261

A.8.16.7.2	 Test Requirements	2261

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2261

A.8.16.16.1	 Test Purpose and Environment	2261

A.8.16.16.2	 Test Requirements	2262

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2262

A.8.16.17.1	Test Purpose and Environment	2262

A.8.16.17.2	Test Requirements	2264

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2265

A.8.16.17A.1	Test Purpose and Environment	2265

A.8.16.17A.2	Test Requirements	2265

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2265

A.8.16.17B.1	Test Purpose and Environment	2265

A.8.16.17B.2	Test Requirements	2266

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2266

A.8.16.17C.1	Test Purpose and Environment	2266

A.8.16.17C.2	Test Requirements	2266

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2266

A.8.16.18.1	Test Purpose and Environment	2266

A.8.16.18.2	Test Requirements	2268

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2269

A.8.16.18A.1	Test Purpose and Environment	2269

A.8.16.18A.2	Test Requirements	2269

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2269

A.8.16.18B.1	Test Purpose and Environment	2269

A.8.16.18B.2	Test Requirements	2270

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2270

A.8.16.18C.1	Test Purpose and Environment	2270

A.8.16.18C.2	Test Requirements	2270

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2270

A.8.16.18D.1	Test Purpose and Environment	2270

A.8.16.18D.2	Test Requirements	2271

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2271

A.8.16.19.1	Test Purpose and Environment	2271

A.8.16.19.2	Test Requirements	2273

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2273

A.8.16.19A.1	Test Purpose and Environment	2273

A.8.16.19A.2	Test Requirements	2274

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2274

A.8.16.19B.1	Test Purpose and Environment	2274

A.8.16.19B.2	Test Requirements	2274

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2274

A.8.16.19C.1	Test Purpose and Environment	2274

A.8.16.19C.2	Test Requirements	2275

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2275

A.8.16.20.1	Test Purpose and Environment	2275

A.8.16.20.2	Test Requirements	2277

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2278

A.8.16.20A.1	Test Purpose and Environment	2278

A.8.16.20A.2	Test Requirements	2278

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2278

A.8.16.20B.1	Test Purpose and Environment	2278

A.8.16.20B.2	Test Requirements	2279

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2279

A.8.16.20C.1	Test Purpose and Environment	2279

A.8.16.20C.2	Test Requirements	2279

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2279

A.8.16.20D.1	Test Purpose and Environment	2279

A.8.16.20D.2	Test Requirements	2280

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2280

A.8.16.21.1	Test Purpose and Environment	2280

A.8.16.21.2	Test Requirements	2282

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2282

A.8.16.22.1	Test Purpose and Environment	2282

A.8.16.22.2	Test Requirements	2284

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2284

A.8.16.23.1	Test Purpose and Environment	2284

A.8.16.23.2	Test Requirements	2287

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2287

A.8.16.24.1	Test Purpose and Environment	2287

A.8.16.24.2	Test Requirements	2290

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2290

A.8.16.25.1	Test Purpose and Environment	2290

A.8.16.25.2	Test Requirements	2292

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2292

A.8.16.26.1	Test Purpose and Environment	2292

A.8.16.26.2	Test Requirements	2295

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2295

A.8.16.27.1	Test Purpose and Environment	2295

A.8.16.27.2	Test Requirements	2299

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2299

A.8.16.28.1	Test Purpose and Environment	2299

A.8.16.28.2	Test Requirements	2303

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2303

A.8.16.29.1	Test Purpose and Environment	2303

A.8.16.29.2	Test Requirements	2307

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2307

A.8.16.30.1	Test Purpose and Environment	2307

A.8.16.30.2	Test Requirements	2311

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2311

A.8.16.31.1	Test Purpose and Environment	2311

A.8.16.31.2	Test Requirements	2315

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2315

A.8.16.32.1	Test Purpose and Environment	2315

A.8.16.32.2	Test Requirements	2319

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2319

A.8.16.33.1	Test Purpose and Environment	2319

A.8.16.33.2	Test Requirements	2323

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2323

A.8.16.34.1	Test Purpose and Environment	2323

A.8.16.34.2	Test Requirements	2327

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2327

A.8.16.35.1	Test Purpose and Environment	2327

A.8.16.35.2	Test Requirements	2329

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2330

A.8.16.36.1	Test Purpose and Environment	2330

A.8.16.36.2	Test Requirements	2332

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2333

A.8.16.37.1	Test Purpose and Environment	2333

A.8.16.37.2	Test Requirements	2335

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2336

A.8.16.38.1	Test Purpose and Environment	2336

A.8.16.38.2	Test Requirements	2338

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2339

A.8.16.39.1	Test Purpose and Environment	2339

A.8.16.39.2	Test Requirements	2342

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2342

A.8.16.40.1	Test Purpose and Environment	2342

A.8.16.40.2	Test Requirements	2345

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2345

A.8.16.41.1	Test Purpose and Environment	2345

A.8.16.41.2	Test Requirements	2347

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2348

A.8.16.42.1	Test Purpose and Environment	2348

A.8.16.42.2	Test Requirements	2350

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2351

A.8.16.43.1	Test Purpose and Environment	2351

A.8.16.43.2	Test Requirements	2354

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2354

A.8.16.44.1	Test Purpose and Environment	2354

A.8.16.44.2	Test Requirements	2357

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2357

A.8.16.45.1	Test Purpose and Environment	2357

A.8.16.45.2	Test Requirements	2360

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2360

A.8.16.46.1	Test Purpose and Environment	2360

A.8.16.46.2	Test Requirements	2362

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2363

A.8.16.47.1	Test Purpose and Environment	2363

A.8.16.47.2	Test Requirements	2365

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2366

A.8.16.48.1	Test Purpose and Environment	2366

A.8.16.48.2	Test Requirements	2368

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2369

A.8.16.49.1	Test Purpose and Environment	2369

A.8.16.49.2	Test Requirements	2372

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2372

A.8.16.50.1	Test Purpose and Environment	2372

A.8.16.50.2	Test Requirements	2374

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2375

A.8.16.51.1	Test Purpose and Environment	2375

A.8.16.51.2	Test Requirements	2379

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2379

A.8.16.52.1	Test Purpose and Environment	2379

A.8.16.52.2	Test Requirements	2383

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2383

A.8.16.53.1	Test Purpose and Environment	2383

A.8.16.53.2	Test Requirements	2387

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2387

A.8.16.54.1	Test Purpose and Environment	2387

A.8.16.54.2	Test Requirements	2391

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2391

A.8.16.55.1	Test Purpose and Environment	2391

A.8.16.55.2	Test Requirements	2396

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2396

A.8.16.56.1	Test Purpose and Environment	2396

A.8.16.56.2	Test Requirements	2401

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2401

A.8.16.57.1	Test Purpose and Environment	2401

A.8.16.57.2	Test Requirements	2403

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2404

A.8.16.58.1	Test Purpose and Environment	2404

A.8.16.58.2	Test Requirements	2406

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2407

A.8.16.59.1	Test Purpose and Environment	2407

A.8.16.59.2	Test Requirements	2410

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2411

A.8.16.60.1	Test Purpose and Environment	2411

A.8.16.60.2	Test Requirements	2414

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2415

A.8.16.61.1	Test Purpose and Environment	2415

A.8.16.61.2	Test Requirements	2418

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2418

A.8.16.62.1	Test Purpose and Environment	2418

A.8.16.62.2	Test Requirements	2420

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2421

A.8.16.63.1	Test Purpose and Environment	2421

A.8.16.63.2	Test Requirements	2424

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2425

A.8.16.64.1	Test Purpose and Environment	2425

A.8.16.64.2	Test Requirements	2429

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2429

A.8.16.65.1	Test Purpose and Environment	2429

A.8.16.65.2	Test Requirements	2434

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2434

A.8.16.66.1	Test Purpose and Environment	2434

A.8.16.66.2	Test Requirements	2439

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2439

A.8.16.67.1	Test Purpose and Environment	2439

A.8.16.67.2	Test Requirements	2444

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2444

A.8.16.68.1	Test Purpose and Environment	2444

A.8.16.68.2	Test Requirements	2449

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2449

A.8.16.69.1	Test Purpose and Environment	2449

A.8.16.69.2	Test Requirements	2452

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2452

A.8.16.70.1	Test Purpose and Environment	2452

A.8.16.70.2	Test Requirements	2455

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2455

A.8.16.71.1	Test Purpose and Environment	2455

A.8.16.71.2	Test Requirements	2459

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2459

A.8.16.72.1	Test Purpose and Environment	2459

A.8.16.72.2	Test Requirements	2463

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2463

A.8.16.73.1	Test Purpose and Environment	2463

A.8.16.73.2	Test Requirements	2468

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2468

A.8.16.74.1	Test Purpose and Environment	2468

A.8.16.74.2	Test Requirements	2473

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2473

A.8.16.75.1	Test Purpose and Environment	2473

A.8.16.75.2	Test Requirements	2478

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2478

A.8.16.76.1	Test Purpose and Environment	2478

A.8.16.76.2	Test Requirements	2483

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2483

A.8.16.77.1	Test Purpose and Environment	2483

A.8.16.77.2	Test Requirements	2486

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2486

A.8.16.78.1	Test Purpose and Environment	2486

A.8.16.78.2	Test Requirements	2489

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2490

A.8.16.79.1	Test Purpose and Environment	2490

A.8.16.79.2	Test Requirements	2495

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2495

A.8.16.80.1	Test Purpose and Environment	2495

A.8.16.80.2	Test Requirements	2500

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2500

A.8.16.81.1	Test Purpose and Environment	2500

A.8.16.81.2	Test Requirements	2505

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2505

A.8.16.82.1	Test Purpose and Environment	2505

A.8.16.82.2	Test Requirements	2510

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2510

A.8.16.83.1	Test Purpose and Environment	2510

A.8.16.83.2	Test Requirements	2515

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2515

A.8.16.84.1	Test Purpose and Environment	2515

A.8.16.84.2	Test Requirements	2521

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2521

A.8.16.85.1	Test Purpose and Environment	2521

A.8.16.85.2	Test Requirements	2524

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2524

A.8.16.86.1	Test Purpose and Environment	2524

A.8.16.86.2	Test Requirements	2527

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2527

A.8.16.87.1	Test Purpose and Environment	2527

A.8.16.87.2	Test Requirements	2532

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2532

A.8.16.88.1	Test Purpose and Environment	2532

A.8.16.88.2	Test Requirements	2540

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2540

A.8.16.89.1	Test Purpose and Environment	2540

A.8.16.89.2	Test Requirements	2545

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2545

A.8.16.90.1	Test Purpose and Environment	2545

A.8.16.90.2	Test Requirements	2550

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2550

A.8.16.91.1	Test Purpose and Environment	2550

A.8.16.91.2	Test Requirements	2556

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2556

A.8.16.92.1	Test Purpose and Environment	2556

A.8.16.92.2	Test Requirements	2564

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2564

A.8.16.93.1	Test Purpose and Environment	2564

A.8.16.93.2	Test Requirements	2569

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2569

A.8.16.94.1	Test Purpose and Environment	2569

A.8.16.94.2	Test Requirements	2574

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2574

A.8.16.95.1	Test Purpose and Environment	2574

A.8.16.95.2	Test Requirements	2582

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2582

A.8.16.96.1	Test Purpose and Environment	2582

A.8.16.96.2	Test Requirements	2589

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2589

A.8.16.97.1	Test Purpose and Environment	2589

A.8.16.97.2	Test Requirements	2594

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2594

A.8.16.98.1	Test Purpose and Environment	2594

A.8.16.98.2	Test Requirements	2600

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2600

A.8.16.99.1	Test Purpose and Environment	2600

A.8.16.99.2	Test Requirements	2607

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2607

A.8.16.100.1	Test Purpose and Environment	2607

A.8.16.100.2	Test Requirements	2615

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2615

A.8.16.101.1	Test Purpose and Environment	2615

A.8.16.101.2	Test Requirements	2622

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2622

A.8.16.102.1	Test Purpose and Environment	2622

A.8.16.102.2	Test Requirements	2630

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	2630

A.8.16.103.1	Test Purpose and Environment	2630

A.8.16.103.2	Test Requirements	2634

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	2635

A.8.16.104.1	Test Purpose and Environment	2635

A.8.16.104.2	Test Requirements	2638

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	2639

A.8.16.105.1	Test Purpose and Environment	2639

A.8.16.105.2	Test Requirements	2643

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	2644

A.8.16.106.1	Test Purpose and Environment	2644

A.8.16.106.2	Test Requirements	2647

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2647

A.8.16.107.1	Test Purpose and Environment	2647

A.8.16.107.2	Test Requirements	2649

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	2650

A.8.16.108.1	Test Purpose and Environment	2650

A.8.16.108.2	Test Requirements	2652

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	2653

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	2653

A.8.17.1.1	Test Purpose and Environment	2653

A.8.17.1.2	Test Requirements	2658

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	2658

A.8.17.2.1	Test Purpose and Environment	2658

A.8.17.2.2	Test Requirements	2664

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	2664

A.8.17.3.1	Test Purpose and Environment	2664

A.8.17.3.2	Test Requirements	2665

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	2665

A.8.17.4.1	Test Purpose and Environment	2665

A.8.17.4.2	Test Requirements	2666

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2666

A.8.17.5.1	Test Purpose and Environment	2666

A.8.17.5.2	Test Requirements	2667

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	2667

A.8.17.6.1	Test Purpose and Environment	2667

A.8.17.6.2	Test Requirements	2668

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	2669

A.8.17.7.1	Test Purpose and Environment	2669

A.8.17.7.2	Test Requirements	2669

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	2670

A.8.17.8.1	Test Purpose and Environment	2670

A.8.17.8.2	Test Requirements	2670

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	2671

A.8.17.9.1	Test Purpose and Environment	2671

A.8.17.9.2	Test Requirements	2672

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	2672

A.8.17.10.1	Test Purpose and Environment	2672

A.8.17.10.2	Test Requirements	2677

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	2678

A.8.17.11.1	Test Purpose and Environment	2678

A.8.17.11.2	Test Requirements	2684

A.8.18	E-UTRAN TDD – HRPD Measurements	2685

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	2685

A.8.18.1.1	Test Purpose and Environment	2685

A.8.18.1.2	Test Requirements	2687

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	2687

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	2687

A.8.19.1.1	Test Purpose and Environment	2687

A.8.19.1.2	Test Requirements	2688

A.8.20	Inter-frequency/RAT Measurements in CA mode	2689

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2689

A.8.20.1.1	Test Purpose and Environment	2689

A.8.20.1.2	Test Requirements	2690

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2690

A.8.20.2.1	Test Purpose and Environment	2691

A.8.20.2.2	Test Requirements	2692

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	2693

A.8.20.2A.1	Test Purpose and Environment	2693

A.8.20.2A.2	Test Requirements	2693

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	2693

A.8.20.2B.1	Test Purpose and Environment	2693

A.8.20.2B.2	Test Requirements	2696

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2696

A.8.20.3.1	Test Purpose and Environment	2696

A.8.20.3.2	Test Requirements	2698

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2698

A.8.20.4.1	Test Purpose and Environment	2698

A.8.20.4.1.1	1.28 Mcps TDD option	2698

A.8.20.4.2	Test Requirements	2700

A.8.20.4.2.1	1.28 Mcps TDD option	2700

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2700

A.8.20.4A.1	Test Purpose and Environment	2700

A.8.20.4A.1.1	1.28 Mcps TDD option	2700

A.8.20.4A.2	Test Requirements	2700

A.8.20.4A.2.1	1.28 Mcps TDD option	2700

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	2701

A.8.20.4B.1	Test Purpose and Environment	2701

A.8.20.4B.1.1	1.28 Mcps TDD option	2701

A.8.20.4B.2	Test Requirements	2703

A.8.20.4B.2.1	1.28 Mcps TDD option	2703

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	2703

A.8.21.1	Test Purpose and Environment	2703

A.8.21.2 Test Requirements	2707

A.8.22	E-UTRAN Discovery Signal Measurements	2707

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2707

A.8.22.1.1	Test Purpose and Environment	2707

A.8.22.1.2	Test Requirements	2710

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	2710

A.8.22.2.1	Test Purpose and Environment	2710

A.8.22.2.2	Test Requirements	2713

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2713

A.8.22.3.1	Test Purpose and Environment	2713

A.8.22.3.2	Test Requirements	2716

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	2716

A.8.22.4.1	Test Purpose and Environment	2716

A.8.22.4.2	Test Requirements	2719

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2719

A.8.22.5.1	Test Purpose and Environment	2719

A.8.22.5.2	Test Requirements	2722

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2722

A.8.22.6.1	Test Purpose and Environment	2722

A.8.22.6.2	Test Requirements	2725

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	2725

A.8.22.7.1	Test Purpose and Environment	2725

A.8.22.7.2	Test Requirements	2728

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	2728

A.8.22.8.1	Test Purpose and Environment	2728

A.8.22.8.2	Test Requirements	2731

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2731

A.8.22.9.1	Test Purpose and Environment	2731

A.8.22.9.2	Test Requirements	2733

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	2734

A.8.22.10.1	Test Purpose and Environment	2734

A.8.22.10.2	Test Requirements	2736

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2737

A.8.22.11.1	Test Purpose and Environment	2737

A.8.22.11.2	Test Requirements	2739

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	2739

A.8.22.12.1	Test Purpose and Environment	2739

A.8.22.12.2	Test Requirements	2742

A.8.23	E-UTRAN Dual Connectivity Measurements	2742

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2742

A.8.23.1.1	Test Purpose and Environment	2742

A.8.23.1.2	Test Requirements	2745

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	2745

A.8.23.2.1	Test Purpose and Environment	2745

A.8.23.2.2	Test Requirements	2748

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	2748

A.8.23.3.1	Test Purpose and Environment	2748

A.8.23.3.2	Test Requirements	2751

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2751

A.8.23.4.1	Test Purpose and Environment	2751

A.8.23.4.2	Test Requirements	2754

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	2754

A.8.23.5.1	Test Purpose and Environment	2754

A.8.23.5.2	Test Requirements	2757

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	2757

A.8.23.6.1	Test Purpose and Environment	2757

A.8.23.6.2	Test Requirements	2760

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	2760

A.8.23.7.1	Test Purpose and Environment	2760

A.8.23.7.2	Test Requirements	2762

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	2763

A.8.23.8.1	Test Purpose and Environment	2763

A.8.23.8.2	Test Requirements	2765

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	2766

A.8.23.9.1	Test Purpose and Environment	2766

A.8.23.9.2	Test Requirements	2769

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2769

A.8.23.10.1	Test Purpose and Environment	2769

A.8.23.10.2	Test Requirements	2772

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2772

A.8.23.11.1	Test Purpose and Environment	2772

A.8.23.11.2	Test Requirements	2775

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	2775

A.8.23.12.1	Test Purpose and Environment	2775

A.8.23.12.2	Test Requirements	2778

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	2778

A.8.23.13.1	Test Purpose and Environment	2778

A.8.23.13.2	Test Requirements	2781

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	2781

A.8.23.14.1	Test Purpose and Environment	2781

A.8.23.14.2	Test Requirements	2784

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	2784

A.8.23.15.1	Test Purpose and Environment	2784

A.8.23.15.2	Test Requirements	2787

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	2787

A.8.23.16.1	Test Purpose and Environment	2787

A.8.23.16.2	Test Requirements	2788

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	2789

A.8.23.17.1	Test Purpose and Environment	2789

A.8.23.17.2	Test Requirements	2791

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2791

A.8.23.18.1	Test Purpose and Environment	2791

A.8.23.18.2	Test Requirements	2792

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2793

A.8.23.19.1	Test Purpose and Environment	2793

A.8.23.19.2	Test Requirements	2794

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2795

A.8.23.20.1	Test Purpose and Environment	2795

A.8.23.20.2	Test Requirements	2796

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2797

A.8.23.21.1	Test Purpose and Environment	2797

A.8.23.21.2	Test Requirements	2798

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	2799

A.8.23.22.1	Test Purpose and Environment	2799

A.8.23.22.2	Test Requirements	2800

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	2801

A.8.23.23.1	Test Purpose and Environment	2801

A.8.23.23.2	Test Requirements	2802

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2803

A.8.23.24.1	Test Purpose and Environment	2803

A.8.23.24.2	Test Requirements	2805

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	2806

A.8.23.25.1	Test Purpose and Environment	2806

A.8.23.25.2	Test Requirements	2808

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	2809

A.8.23.26.1	Test Purpose and Environment	2809

A.8.23.26.2	Test Requirements	2811

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2812

A.8.23.27.1	Test Purpose and Environment	2812

A.8.23.27.2	Test Requirements	2815

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	2815

A.8.23.28.1	Test Purpose and Environment	2815

A.8.23.28.2	Test Requirements	2818

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	2818

A.8.23.29.1	Test Purpose and Environment	2818

A.8.23.29.2	Test Requirements	2821

A.8.24	Proximity-based Services	2821

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2821

A.8.24.1.1	Test Purpose and Environment	2821

A.8.24.1.2	Test Requirements	2822

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	2823

A.8.24.2.1	Test Purpose and Environment	2823

A.8.24.2.2	Test Requirements	2824

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	2824

A.8.24.3.1	Test Purpose and Environment	2825

A.8.24.3.2	Test Requirements	2826

A.8.25	E-UTRAN-WLAN Measurements	2827

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2827

A.8.25.1.1	Test Purpose and Environment	2827

A.8.25.1.2	Test Requirements	2829

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	2829

A.8.25.2.1	Test Purpose and Environment	2829

A.8.25.2.2	Test Requirements	2831

A.8.26 Frame Structure 3 (FS3)	2831

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	2831

A.8.26.1.1	Test Purpose and Environment	2831

A.8.26.1.2	Test Requirements	2833

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	2834

A.8.26.2.1	Test Purpose and Environment	2834

A.8.26.2.2	Test Requirements	2836

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2837

A.8.26.3.1	Test Purpose and Environment	2837

A.8.26.3.2	Test Requirements	2840

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2840

A.8.26.3A.1	Test Purpose and Environment	2840

A.8.26.3A.2	Test Requirements	2843

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	2843

A.8.26.4.1	Test Purpose and Environment	2843

A.8.26.4.2	Test Requirements	2846

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	2846

A.8.26.4A.1	Test Purpose and Environment	2846

A.8.26.4A.2	Test Requirements	2849

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2849

A.8.26.5.1	Test Purpose and Environment	2849

A.8.26.5.2	Test Requirements	2852

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2852

A.8.26.5A.1	Test Purpose and Environment	2852

A.8.26.5A.2	Test Requirements	2855

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	2855

A.8.26.6.1	Test Purpose and Environment	2855

A.8.26.6.2	Test Requirements	2858

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	2858

A.8.26.6A.1	Test Purpose and Environment	2858

A.8.26.6A.2	Test Requirements	2861

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2861

A.8.26.7.1	Test Purpose and Environment	2861

A.8.26.7.2	Test Requirements	2864

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	2864

A.8.26.8.1	Test Purpose and Environment	2864

A.8.26.8.2	Test Requirements	2867

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2867

A.8.26.9.1	Test Purpose and Environment	2867

A.8.26.9.2	Test Requirements	2870

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2870

A.8.26.10.1	Test Purpose and Environment	2870

A.8.26.10.2	Test Requirements	2872

A.9	Measurement Performance Requirements	2858

A.9.1	RSRP	2858

A.9.1.1	FDD Intra frequency case	2858

A.9.1.1.1	Test Purpose and Environment	2858

A.9.1.1.2	Test parameters	2858

A.9.1.1.3	Test Requirements	2861

A.9.1.2	TDD Intra frequency case	2861

A.9.1.2.1	Test Purpose and Environment	2861

A.9.1.2.2	Test parameters	2861

A.9.1.2.3	Test Requirements	2862

A.9.1.3	FDD—FDD Inter frequency case	2863

A.9.1.3.1	Test Purpose and Environment	2863

A.9.1.3.2	Test parameters	2863

A.9.1.3.3	Test Requirements	2866

A.9.1.4	TDD—TDD Inter frequency case	2866

A.9.1.4.1	Test Purpose and Environment	2866

A.9.1.4.2	Test parameters	2866

A.9.1.4.3	Test Requirements	2868

A.9.1.5	FDD—TDD Inter frequency case	2869

A.9.1.5.1	Test Purpose and Environment	2869

A.9.1.5.2	Test parameters	2869

A.9.1.5.3	Test Requirements	2870

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	2871

A.9.1.6.1	Test Purpose and Environment	2871

A.9.1.6.2	Test parameters	2871

A.9.1.6.3	Test Requirements	2874

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	2874

A.9.1.7.1	Test Purpose and Environment	2874

A.9.1.7.2	Test parameters	2874

A.9.1.7.3	Test Requirements	2877

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2877

A.9.1.8.1	Test Purpose and Environment	2877

A.9.1.8.2	Test parameters	2877

A.9.1.8.3	Test Requirements	2881

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2881

A.9.1.9.1	Test Purpose and Environment	2881

A.9.1.9.2	Test parameters	2881

A.9.1.9.3	Test Requirements	2883

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2884

A.9.1.10.1	Test Purpose and Environment	2884

A.9.1.10.2	Test parameters	2884

A.9.1.10.3	Test Requirements	2887

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	2887

A.9.1.11.1	Test Purpose and Environment	2887

A.9.1.11.2	Test parameters	2887

A.9.1.11.3	Test Requirements	2891

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2891

A.9.1.12.1	Test Purpose and Environment	2891

A.9.1.12.2	Test parameters	2891

A.9.1.12.3	Test Requirements	2892

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	2892

A.9.1.13.1	Test Purpose and Environment	2892

A.9.1.13.2	Test parameters	2892

A.9.1.13.3	Test Requirements	2893

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2893

A.9.1.14.1	Test Purpose and Environment	2893

A.9.1.14.2	Test parameters	2893

A.9.1.14.3	Test Requirements	2897

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2897

A.9.1.15.1	Test Purpose and Environment	2897

A.9.1.15.2	Test parameters	2897

A.9.1.15.3	Test Requirements	2899

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	2900

A.9.1.16.1	Test Purpose and Environment	2900

A.9.1.16.2	Test parameters	2900

A.9.1.16.3	Test Requirements	2901

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	2901

A.9.1.17.1	Test Purpose and Environment	2901

A.9.1.17.2	Test parameters	2902

A.9.1.17.3	Test Requirements	2903

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2903

A.9.1.18.1	Test Purpose and Environment	2903

A.9.1.18.2	Test parameters	2903

A.9.1.18.3	Test Requirements	2904

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	2905

A.9.1.19.1	Test Purpose and Environment	2905

A.9.1.19.2	Test parameters	2905

A.9.1.19.3	Test Requirements	2905

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2905

A.9.1.20.1	Test Purpose and Environment	2905

A.9.1.20.2	Test parameters	2905

A.9.1.20.3	Test Requirements	2907

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	2907

A.9.1.21.1	Test Purpose and Environment	2907

A.9.1.21.2	Test parameters	2907

A.9.1.21.3	Test Requirements	2907

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2907

A.9.1.22.1	Test Purpose and Environment	2907

A.9.1.22.2	Test parameters	2908

A.9.1.22.3	Test Requirements	2912

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2912

A.9.1.23.1	Test Purpose and Environment	2912

A.9.1.23.2	Test parameters	2912

A.9.1.23.3	Test Requirements	2916

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	2916

A.9.1.24.1	Test Purpose and Environment	2916

A.9.1.24.2	Test parameters	2916

A.9.1.24.3	Test Requirements	2917

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2917

A.9.1.25.1	Test Purpose and Environment	2917

A.9.1.25.2	Test parameters	2917

A.9.1.25.3	Test Requirements	2920

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2920

A.9.1.26.1	Test Purpose and Environment	2920

A.9.1.26.2	Test parameters	2920

A.9.1.26.3	Test Requirements	2923

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	2923

A.9.1.27.1	Test Purpose and Environment	2923

A.9.1.27.2	Test parameters	2923

A.9.1.27.3	Test Requirements	2926

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	2926

A.9.1.28.1	Test Purpose and Environment	2926

A.9.1.28.2	Test parameters	2926

A.9.1.28.3	Test Requirements	2929

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2929

A.9.1.29.1	Test Purpose and Environment	2929

A.9.1.29.2	Test parameters	2929

A.9.1.29.3	Test Requirements	2932

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2932

A.9.1.30.1	Test Purpose and Environment	2932

A.9.1.30.2	Test parameters	2933

A.9.1.30.3	Test Requirements	2936

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	2936

A.9.1.31.1	Test Purpose and Environment	2936

A.9.1.31.2	Test parameters	2936

A.9.1.31.3	Test Requirements	2939

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	2940

A.9.1.32.1	Test Purpose and Environment	2940

A.9.1.32.2	Test parameters	2940

A.9.1.32.3	Test Requirements	2943

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2943

A.9.1.33.1	Test Purpose and Environment	2943

A.9.1.33.2	Test parameters	2943

A.9.1.33.3	Test Requirements	2946

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	2946

A.9.1.34.1	Test Purpose and Environment	2946

A.9.1.34.2	Test parameters	2946

A.9.1.34.3	Test Requirements	2949

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2949

A.9.1.35.1	Test Purpose and Environment	2949

A.9.1.35.2	Test parameters	2949

A.9.1.35.3	Test Requirements	2952

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	2953

A.9.1.36.1	Test Purpose and Environment	2953

A.9.1.36.2	Test parameters	2953

A.9.1.36.3	Test Requirements	2956

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2956

A.9.1.37.1	Test Purpose and Environment	2956

A.9.1.37.2	Test parameters	2956

A.9.1.37.3	Test Requirements	2959

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2960

A.9.1.38.1	Test Purpose and Environment	2960

A.9.1.38.2	Test parameters	2960

A.9.1.38.3	Test Requirements	2963

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2964

A.9.1.39.1	Test Purpose and Environment	2964

A.9.1.39.2	Test parameters	2964

A.9.1.39.3	Test Requirements	2970

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2970

A.9.1.40.1	Test Purpose and Environment	2970

A.9.1.40.2	Test parameters	2971

A.9.1.40.3	Test Requirements	2977

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	2977

A.9.1.41.1	Test Purpose and Environment	2977

A.9.1.41.2	Test parameters	2977

A.9.1.41.3	Test Requirements	2980

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	2980

A.9.1.42.1	Test Purpose and Environment	2980

A.9.1.42.2	Test parameters	2980

A.9.1.42.3	Test Requirements	2983

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	2983

A.9.1.43.1	Test Purpose and Environment	2983

A.9.1.43.2	Test parameters	2983

A.9.1.43.3	Test Requirements	2984

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2985

A.9.1.44.1	Test Purpose and Environment	2985

A.9.1.44.2	Test parameters	2985

A.9.1.44.3	Test Requirements	2990

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	2990

A.9.1.45.1	Test Purpose and Environment	2990

A.9.1.45.2	Test parameters	2990

A.9.1.45.3	Test Requirements	2416

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2416

A.9.1.46.1	Test Purpose and Environment	2416

A.9.1.46.2	Test parameters	2416

A.9.1.46.3	Test Requirements	2425

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2426

A.9.1.47.1	Test Purpose and Environment	2426

A.9.1.47.2	Test parameters	2426

A.9.1.47.3	Test Requirements	2435

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	2435

A.9.1.48.1	Test Purpose and Environment	2435

A.9.1.48.2	Test parameters	2436

A.9.1.48.3	Test Requirements	2446

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	2447

A.9.1.49.1	Test Purpose and Environment	2447

A.9.1.49.2	Test parameters	2447

A.9.1.49.3	Test Requirements	2457

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	2458

A.9.1.50.1	Test Purpose and Environment	2458

A.9.1.50.2	Test parameters	2458

A.9.1.50.3	Test Requirements	2465

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	2466

A.9.1.51.1	Test Purpose and Environment	2466

A.9.1.51.2	Test parameters	2466

A.9.1.51.3	Test Requirements	2472

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2473

A.9.1.52.1	Test Purpose and Environment	2473

A.9.1.52.2	Test parameters	2473

A.9.1.52.3	Test Requirements	2476

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2476

A.9.1.52A.1	Test Purpose and Environment	2476

A.9.1.52A.2	Test parameters	2476

A.9.1.52A.3	Test Requirements	2477

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2477

A.9.1.53.1	Test Purpose and Environment	2477

A.9.1.53.2	Test parameters	2478

A.9.1.53.3	Test Requirements	2481

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	2481

A.9.1.53A.1	Test Purpose and Environment	2481

A.9.1.53A.2	Test parameters	2481

A.9.1.53A.3	Test Requirements	2482

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	2483

A.9.1.54.1	Test Purpose and Environment	2483

A.9.1.54.2	Test parameters	2483

A.9.1.54.3	Test Requirements	2486

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	2486

A.9.1.55.1	Test Purpose and Environment	2486

A.9.1.55.2	Test parameters	2486

A.9.1.55.3	Test Requirements	2490

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	2490

A.9.1.56.1	Test Purpose and Environment	2490

A.9.1.56.2	Test parameters	2490

A.9.1.56.3	Test Requirements	2493

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2494

A.9.1.57.1	Test Purpose and Environment	2494

A.9.1.57.2	Test parameters	2494

A.9.1.57.3	Test Requirements	2497

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2497

A.9.1.57A.1	Test Purpose and Environment	2497

A.9.1.57A.2	Test parameters	2497

A.9.1.57A.3	Test Requirements	2498

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2498

A.9.1.58.1	Test Purpose and Environment	2498

A.9.1.58.2	Test parameters	2499

A.9.1.58.3	Test Requirements	2502

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	2502

A.9.1.58A.1	Test Purpose and Environment	2502

A.9.1.58A.2	Test parameters	2502

A.9.1.58A.3	Test Requirements	2503

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	2503

A.9.1.59.1	Test Purpose and Environment	2503

A.9.1.59.2	Test parameters	2504

A.9.1.59.3	Test Requirements	2506

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	2506

A.9.1.60.1	Test Purpose and Environment	2506

A.9.1.60.2	Test parameters	2506

A.9.1.60.3	Test Requirements	2509

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	2510

A.9.1.61.1	Test Purpose and Environment	2510

A.9.1.61.2	Test parameters	2510

A.9.1.61.3	Test Requirements	2513

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2513

A.9.1.62.1	Test Purpose and Environment	2513

A.9.1.62.2	Test parameters	2513

A.9.1.62.3	Test Requirements	2516

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2516

A.9.1.53.1	Test Purpose and Environment	2516

A.9.1.53.2	Test parameters	2516

A.9.1.63.3	Test Requirements	2519

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	2519

A.9.1.64.1	Test Purpose and Environment	2519

A.9.1.64.2	Test parameters	2519

A.9.1.64.3	Test Requirements	2520

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2521

A.9.1.65.1	Test Purpose and Environment	2521

A.9.1.65.2	Test parameters	2521

A.9.1.65.3	Test Requirements	2527

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2527

A.9.1.66.1	Test Purpose and Environment	2527

A.9.1.66.2	Test parameters	2527

A.9.1.66.3	Test Requirements	2533

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	2533

A.9.1.67.1	Test Purpose and Environment	2533

A.9.1.67.2	Test parameters	2533

A.9.1.67.3	Test Requirements	2534

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2535

A.9.1.68.1	Test Purpose and Environment	2535

A.9.1.68.2	Test parameters	2535

A.9.1.68.3	Test Requirements	2539

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2539

A.9.1.69.1	Test Purpose and Environment	2539

A.9.1.69.2	Test parameters	2539

A.9.1.69.3	Test Requirements	2543

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2544

A.9.1.70.1	Test Purpose and Environment	2544

A.9.1.70.2	Test parameters	2544

A.9.1.70.3	Test Requirements	2556

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2557

A.9.1.71.1	Test Purpose and Environment	2557

A.9.1.71.2	Test parameters	2557

A.9.1.71.3	Test Requirements	2569

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	2570

A.9.1.72.1	Test Purpose and Environment	2570

A.9.1.72.2	Test parameters	2570

A.9.1.72.3	Test Requirements	2582

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	2583

A.9.1.73.1	Test Purpose and Environment	2583

A.9.1.73.2	Test parameters	2583

A.9.1.73.3	Test Requirements	2586

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	2586

A.9.1.74.1	Test Purpose and Environment	2586

A.9.1.74.2	Test parameters	2586

A.9.1.74.3	Test Requirements	2587

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	2587

A.9.1.75.1	Test Purpose and Environment	2587

A.9.1.75.2	Test parameters	2588

A.9.1.74.3	Test Requirements	2589

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2589

A.9.1.76.1	Test Purpose and Environment	2589

A.9.1.76.2	Test parameters	2589

A.9.1.76.3	Test Requirements	2592

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2592

A.9.1.77.1	Test Purpose and Environment	2592

A.9.1.77.2	Test parameters	2592

A.9.1.77.3	Test Requirements	2595

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	2595

A.9.1.78.1	Test Purpose and Environment	2595

A.9.1.78.2	Test parameters	2595

A.9.1.78.3	Test Requirements	2598

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2598

A.9.1.79.1	Test Purpose and Environment	2598

A.9.1.79.2	Test parameters	2598

A.9.1.79.3	Test Requirements	2601

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2601

A.9.1.80.1	Test Purpose and Environment	2601

A.9.1.80.2	Test parameters	2601

A.9.1.80.3	Test Requirements	2604

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	2604

A.9.1.81.1	Test Purpose and Environment	2604

A.9.1.78.2	Test parameters	2604

A.9.1.81.3	Test Requirements	2607

A.9.2	RSRQ	2607

A.9.2.1	FDD Intra frequency case	2607

A.9.2.1.1	Test Purpose and Environment	2607

A.9.2.1.2	Test parameters	2607

A.9.2.1.3	Test Requirements	2610

A.9.2.2	TDD Intra frequency case	2610

A.9.2.2.1	Test Purpose and Environment	2610

A.9.2.2.2	Test parameters	2610

A.9.2.2.3	Test Requirements	2612

A.9.2.3	FDD—FDD Inter frequency case	2612

A.9.2.3.1	Test Purpose and Environment	2612

A.9.2.3.2	Test parameters	2612

A.9.2.3.3	Test Requirements	2615

A.9.2.4	TDD—TDD Inter frequency case	2615

A.9.2.4.1	Test Purpose and Environment	2615

A.9.2.4.2	Test parameters	2615

A.9.2.4.3	Test Requirements	2617

A.9.2.4A	FDD—TDD Inter frequency case	2618

A.9.2.4A.1	Test Purpose and Environment	2618

A.9.2.4A.2	Test parameters	2618

A.9.2.4A.3	Test Requirements	2620

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	2620

A.9.2.5.1	Test Purpose and Environment	2620

A.9.2.5.2	Test parameters	2621

A.9.2.5.3	Test Requirements	2624

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	2624

A.9.2.6.1	Test Purpose and Environment	2624

A.9.2.6.2	Test parameters	2625

A.9.2.6.3	Test Requirements	2628

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2628

A.9.2.7.1	Test Purpose and Environment	2628

A.9.2.7.2	Test parameters	2628

A.9.2.7.3	Test Requirements	2632

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2632

A.9.2.8.1	Test Purpose and Environment	2632

A.9.2.8.2	Test parameters	2632

A.9.2.8.3	Test Requirements	2635

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	2635

A.9.2.9.1	Test Purpose and Environment	2635

A.9.2.9.2	Test parameters	2635

A.9.2.9.3	Test Requirements	2639

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	2639

A.9.2.10.1	Test Purpose and Environment	2639

A.9.2.10.2	Test parameters	2639

A.9.2.10.3	Test Requirements	2643

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2643

A.9.2.11.1	Test Purpose and Environment	2643

A.9.2.11.2	Test parameters	2643

A.9.2.11.3	Test Requirements	2644

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	2644

A.9.2.12.1	Test Purpose and Environment	2644

A.9.2.12.2	Test parameters	2645

A.9.2.12.3	Test Requirements	2645

A.9.2.13	Void	2646

A.9.2.13.1	Void	2646

A.9.2.13.2	Void	2646

A.9.2.13.3	Void	2646

A.9.2.14	Void	2646

A.9.2.14.1	Void	2646

A.9.2.14.2	Void	2646

A.9.2.14.3	Void	2646

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2646

A.9.2.15.1	Test Purpose and Environment	2646

A.9.2.15.2	Test parameters	2646

A.9.2.15.3	Test Requirements	2650

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2650

A.9.2.16.1	Test Purpose and Environment	2650

A.9.2.16.2	Test parameters	2650

A.9.2.16.3	Test Requirements	2655

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	2655

A.9.2.17.1	Test Purpose and Environment	2655

A.9.2.17.2	Test parameters	2655

A.9.2.17.3	Test Requirements	2657

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	2657

A.9.2.18.1	Test Purpose and Environment	2657

A.9.2.18.2	Test parameters	2657

A.9.2.18.3	Test Requirements	2660

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	2660

A.9.2.19.1	Test Purpose and Environment	2660

A.9.2.19.2	Test parameters	2660

A.9.2.19.3	Test Requirements	2662

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	2662

A.9.2.20.1	Test Purpose and Environment	2662

A.9.2.20.2	Test parameters	2662

A.9.2.20.3	Test Requirements	2665

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2665

A.9.2.21.1	Test Purpose and Environment	2665

A.9.2.21.2	Test parameters	2665

A.9.2.21.3	Test Requirements	2668

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	2668

A.9.2.22.1	Test Purpose and Environment	2669

A.9.2.22.2	Test parameters	2669

A.9.2.22.3	Test Requirements	2669

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2669

A.9.2.23.1	Test Purpose and Environment	2669

A.9.2.23.2	Test parameters	2669

A.9.2.23.3	Test Requirements	2672

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	2672

A.9.2.24.1	Test Purpose and Environment	2672

A.9.2.24.2	Test parameters	2672

A.9.2.24.3	Test Requirements	2673

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	2673

A.9.2.25.1	Test Purpose and Environment	2673

A.9.2.25.2	Test parameters	2673

A.9.2.25.3	Test Requirements	2678

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	2678

A.9.2.26.1	Test Purpose and Environment	2678

A.9.2.26.2	Test parameters	2678

A.9.2.26.3	Test Requirements	2682

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	2682

A.9.2.27.1	Test Purpose and Environment	2682

A.9.2.27.2	Test parameters	2682

A.9.2.27.3	Test Requirements	2683

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2683

A.9.2.28.1	Test Purpose and Environment	2683

A.9.2.28.2	Test parameters	2683

A.9.2.28.3	Test Requirements	2686

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	2686

A.9.2.29.1	Test Purpose and Environment	2686

A.9.2.29.2	Test parameters	2686

A.9.2.29.3	Test Requirements	2689

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2689

A.9.2.30.1	Test Purpose and Environment	2689

A.9.2.30.2	Test parameters	2689

A.9.2.30.3	Test Requirements	2692

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	2692

A.9.2.31.1	Test Purpose and Environment	2692

A.9.2.31.2	Test parameters	2692

A.9.2.31.3	Test Requirements	2695

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2695

A.9.2.32.1	Test Purpose and Environment	2695

A.9.2.32.2	Test parameters	2695

A.9.2.32.3	Test Requirements	2698

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	2698

A.9.2.33.1	Test Purpose and Environment	2698

A.9.2.33.2	Test parameters	2698

A.9.2.33.3	Test Requirements	2702

A.9.2.34	FDD—FDD Inter frequency new RSRQ	2702

A.9.2.34.1	Test Purpose and Environment	2702

A.9.2.34.2	Test parameters	2702

A.9.2.34.3	Test Requirements	2705

A.9.2.35	TDD—TDD Inter frequency new RSRQ	2705

A.9.2.35.1	Test Purpose and Environment	2705

A.9.2.35.2	Test parameters	2705

A.9.2.35.3	Test Requirements	2708

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	2708

A.9.2.36.1	Test Purpose and Environment	2708

A.9.2.36.2	Test parameters	2708

A.9.2.36.3	Test Requirements	2709

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	2710

A.9.2.37.1	Test Purpose and Environment	2710

A.9.2.37.2	Test parameters	2710

A.9.2.37.3	Test Requirements	2711

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2712

A.9.2.38.1	Test Purpose and Environment	2712

A.9.2.38.2	Test parameters	2712

A.9.2.38.3	Test Requirements	2716

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2716

A.9.2.39.1	Test Purpose and Environment	2716

A.9.2.39.2	Test parameters	2716

A.9.2.39.3	Test Requirements	2720

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2720

A.9.2.40.1	Test Purpose and Environment	2720

A.9.2.40.2	Test parameters	2720

A.9.2.40.3	Test Requirements	2723

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2724

A.9.2.41.1	Test Purpose and Environment	2724

A.9.2.41.2	Test parameters	2724

A.9.2.41.3	Test Requirements	2727

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	2727

A.9.2.42.1	Test Purpose and Environment	2727

A.9.2.42.2	Test parameters	2727

A.9.2.42.3	Test Requirements	2730

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	2730

A.9.2.43.1	Test Purpose and Environment	2730

A.9.2.43.2	Test parameters	2730

A.9.2.43.3	Test Requirements	2733

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	2733

A.9.2.44.1	Test Purpose and Environment	2733

A.9.2.44.2	Test parameters	2733

A.9.2.44.3	Test Requirements	2735

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	2735

A.9.2.45.1	Test Purpose and Environment	2735

A.9.2.45.2	Test parameters	2735

A.9.2.45.3	Test Requirements	2739

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	2740

A.9.2.46.1	Test Purpose and Environment	2740

A.9.2.46.2	Test parameters	2740

A.9.2.46.3	Test Requirements	2744

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	2745

A.9.2.47.1	Test Purpose and Environment	2745

A.9.2.47.2	Test parameters	2745

A.9.2.47.3	Test Requirements	2749

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	2749

A.9.2.48.1	Test Purpose and Environment	2749

A.9.2.48.2	Test parameters	2750

A.9.2.48.3	Test Requirements	2754

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2755

A.9.2.49.1	Test Purpose and Environment	2755

A.9.2.49.2	Test parameters	2755

A.9.2.49.3	Test Requirements	2761

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2762

A.9.2.50.1	Test Purpose and Environment	2762

A.9.2.50.2	Test parameters	2762

A.9.2.50.3	Test Requirements	2768

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	2768

A.9.2.51.1	Test Purpose and Environment	2768

A.9.2.51.2	Test parameters	2769

A.9.2.51.3	Test Requirements	2773

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	2773

A.9.2.52.1	Test Purpose and Environment	2773

A.9.2.52.2	Test parameters	2773

A.9.2.52.3	Test Requirements	2776

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	2777

A.9.2.53.1	Test Purpose and Environment	2777

A.9.2.53.2	Test parameters	2777

A.9.2.53.3	Test Requirements	2783

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	2784

A.9.2.54.1	Test Purpose and Environment	2784

A.9.2.54.2	Test parameters	2784

A.9.2.54.3	Test Requirements	2790

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2790

A.9.2.55.1	Test Purpose and Environment	2790

A.9.2.55.2	Test parameters	2791

A.9.2.55.3	Test Requirements	2796

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2796

A.9.2.56.1	Test Purpose and Environment	2796

A.9.2.56.2	Test parameters	2796

A.9.2.56.3	Test Requirements	2801

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2801

A.9.2.57.1	Test Purpose and Environment	2801

A.9.2.57.2	Test parameters	2802

A.9.2.57.3	Test Requirements	2806

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2807

A.9.2.58.1	Test Purpose and Environment	2807

A.9.2.58.2	Test parameters	2807

A.9.2.58.3	Test Requirements	2811

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	2812

A.9.2.59.1	Test Purpose and Environment	2812

A.9.2.59.2	Test parameters	2812

A.9.2.59.3	Test Requirements	2817

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	2818

A.9.2.60.1	Test Purpose and Environment	2818

A.9.2.60.2	Test parameters	2818

A.9.2.60.3	Test Requirements	2821

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	2821

A.9.2.61.1	Test Purpose and Environment	2821

A.9.2.61.2	Test parameters	2821

A.9.2.61.3	Test Requirements	2824

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	2824

A.9.2.62.1	Test Purpose and Environment	2824

A.9.2.62.2	Test parameters	2824

A.9.2.62.3	Test Requirements	2827

A.9.3	UTRAN FDD CPICH RSCP	2827

A.9.3.1	E-UTRAN FDD	2827

A.9.3.1.1	Test Purpose and Environment	2827

A.9.3.1.2	Parameters	2827

A.9.3.1.3	Test Requirements	2830

A.9.3.2 E-UTRAN TDD	2830

A.9.3.2.1	Test Purpose and Environment	2830

A.9.3.2.2	Parameters	2830

A.9.3.2.3	Test Requirements	2833

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	2833

A.9.3.3.1	Test Purpose and Environment	2833

A.9.3.3.2	Parameters	2833

A.9.3.3.3	Test Requirements	2834

A.9.4	UTRAN FDD CPICH Ec/No	2835

A.9.4.1	E-UTRAN FDD	2835

A.9.4.1.1	Test Purpose and Environment	2835

A.9.4.1.2	Parameters	2835

A.9.4.1.3	Test Requirements	2837

A.9.4.2	E-UTRAN TDD	2838

A.9.4.2.1	Test Purpose and Environment	2838

A.9.4.2.2	Parameters	2838

A.9.4.2.3	Test Requirements	2840

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	2841

A.9.4.3.1	Test Purpose and Environment	2841

A.9.4.3.2	Parameters	2841

A.9.4.3.3	Test Requirements	2842

A.9.5	UTRAN TDD measurement	2842

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	2842

A.9.5.1.1	Test Purpose and Environment	2842

A.9.5.1.2	Test parameters	2842

A.9.5.1.3	Test Requirements	2844

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	2844

A.9.5.2.1	Test Purpose and Environment	2844

A.9.5.2.2	Test parameters	2844

A.9.5.2.3	Test Requirements	2846

A.9.6	GSM Carrier RSSI	2847

A.9.6.1	E-UTRAN FDD	2847

A.9.6.1.1	Test Purpose and Environment	2847

A.9.6.1.2	Test Requirements	2848

A.9.6.2	E-UTRAN TDD	2848

A.9.6.2.1	Test Purpose and Environment	2848

A.9.6.2.2	Test Requirements	2850

A.9.7	UE Rx – Tx Time Difference	2851

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	2851

A.9.7.1.1	Test Purpose and Environment	2851

A.9.7.1.2	Test parameters	2851

A.9.7.1.3	Test Requirements	2852

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	2852

A.9.7.2.1	Test Purpose and Environment	2852

A.9.7.2.2	Test parameters	2852

A.9.7.2.3	Test Requirements	2854

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2854

A.9.7.3.1	Test Purpose and Environment	2854

A.9.7.3.2	Test parameters	2854

A.9.7.3.3	Test Requirements	2857

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	2857

A.9.7.4.1	Test Purpose and Environment	2857

A.9.7.4.2	Test Parameters	2857

A.9.7.4.3	Test Requirements	2860

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2860

A.9.7.5.1	Test Purpose and Environment	2860

A.9.7.5.2	Test parameters	2860

A.9.7.5.3	Test Requirements	2863

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2863

A.9.7.6.1	Test Purpose and Environment	2863

A.9.7.6.2	Test Parameters	2863

A.9.7.6.3	Test Requirements	2866

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2866

A.9.7.7.1	Test Purpose and Environment	2866

A.9.7.7.2	Test parameters	2866

A.9.7.7.3	Test Requirements	2867

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2868

A.9.7.8.1	Test Purpose and Environment	2868

A.9.7.8.2	Test parameters	2868

A.9.7.8.3	Test Requirements	2869

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	2869

A.9.7.9.1	Test Purpose and Environment	2869

A.9.7.9.2	Test parameters	2869

A.9.7.9.3	Test Requirements	2870

A.9.8	RSTD	2871

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	2871

A.9.8.1.1	Test Purpose and Environment	2871

A.9.8.1.2	Test Requirements	2873

A.9.8.1.2A	Test Requirements for UE Category 1bis	2873

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	2874

A.9.8.2.1	Test Purpose and Environment	2874

A.9.8.2.2	Test Requirements	2877

A.9.8.2.2A	Test Requirements for UE Category 1bis	2877

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	2878

A.9.8.3.1	Test Purpose and Environment	2878

A.9.8.3.2	Test Requirements	2880

A.9.8.3.2A	Test Requirements for UE Category 1bis	2880

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	2881

A.9.8.4.1	Test Purpose and Environment	2881

A.9.8.4.2	Test Requirements	2883

A.9.8.4.2A	Test Requirements for UE Category 1bis	2883

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	2884

A.9.8.5.1	Test Purpose and Environment	2884

A.9.8.5.2	Test Requirements	2886

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	2886

A.9.8.6.1	Test Purpose and Environment	2886

A.9.8.6.2	Test Requirements	2889

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2889

A.9.8.7.1	Test Purpose and Environment	2889

A.9.8.7.2	Test Requirements	2890

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	2890

A.9.8.8.1	Test Purpose and Environment	2890

A.9.8.8.2	Test Requirements	2891

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2891

A.9.8.9.1	Test Purpose and Environment	2891

A.9.8.9.2	Test Requirements	2892

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	2892

A.9.8.10.1	Test Purpose and Environment	2892

A.9.8.10.2	Test Requirements	2893

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	2893

A.9.8.11.1	Test Purpose and Environment	2893

A.9.8.11.2	Test Requirements	2894

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	2894

A.9.8.12.1	Test Purpose and Environment	2894

A.9.8.12.2	Test Requirements	2895

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	2895

A.9.8.13.1	Test Purpose and Environment	2895

A.9.8.13.2	Test Requirements	2896

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2896

A.9.8.14.1	Test Purpose and Environment	2896

A.9.8.14.2	Test Requirements	2903

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	2903

A.9.8.15.1	Test Purpose and Environment	2903

A.9.8.15.2	Test Requirements	2909

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2909

A.9.8.16.1	Test Purpose and Environment	2909

A.9.8.16.2	Test Requirements	2912

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2912

A.9.8.17.1	Test Purpose and Environment	2912

A.9.8.17.2	Test Requirements	2915

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2915

A.9.8.18.1	Test Purpose and Environment	2915

A.9.8.18.2	Test Requirements	2918

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2918

A.9.8.19.1	Test Purpose and Environment	2918

A.9.8.19.2	Test Requirements	2921

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	2921

A.9.8.20.1	Test Purpose and Environment	2921

A.9.8.20.2	Test Requirements	2924

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	2924

A.9.8.21.1	Test Purpose and Environment	2924

A.9.8.21.2	Test Requirements	2928

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	2929

A.9.8.22.1	Test Purpose and Environment	2929

A.9.8.22.2	Test Requirements	2932

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	2933

A.9.8.23.1	Test Purpose and Environment	2933

A.9.8.23.2	Test Requirements	2936

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	2936

A.9.8.24.1	Test Purpose and Environment	2936

A.9.8.24.2	Test Requirements	2940

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	2940

A.9.8.25.1	Test Purpose and Environment	2940

A.9.8.25.2	Test Requirements	2944

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2945

A.9.8.26.1	Test Purpose and Environment	2945

A.9.8.26.2	Test Requirements	2947

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	2947

A.9.8.27.1	Test Purpose and Environment	2947

A.9.8.27.2	Test Requirements	2950

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	2950

A.9.8.28.1	Test Purpose and Environment	2950

A.9.8.28.2	Test Requirements	2953

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2953

A.9.8.29.1	Test Purpose and Environment	2953

A.9.8.29.2	Test Requirements	2957

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	2958

A.9.8.30.1	Test Purpose and Environment	2958

A.9.8.30.2	Test Requirements	2961

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	2962

A.9.8.31.1	Test Purpose and Environment	2962

A.9.8.31.2	Test Requirements	2966

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	2966

A.9.8.32.1	Test Purpose and Environment	2966

A.9.8.32.2	Test Requirements	2969

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	2969

A.9.8.33.1	Test Purpose and Environment	2969

A.9.8.33.2	Test Requirements	2972

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	2972

A.9.8.34.1	Test Purpose and Environment	2972

A.9.8.34.2	Test Requirements	2975

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	2975

A.9.8.35.1	Test Purpose and Environment	2975

A.9.8.35.2	Test Requirements	2978

A.9.9	RSRP and RSRQ on the serving cell	2978

A.9.9.1	FDD Intra frequency serving cell case	2978

A.9.9.1.1	Test Purpose and Environment	2978

A.9.9.1.2	Test parameters	2978

A.9.9.1.3	Test Requirements	2981

A.9.9.2	TDD Intra frequency serving cell case	2981

A.9.9.2.1	Test Purpose and Environment	2981

A.9.9.2.2	Test parameters	2981

A.9.9.2.3	Test Requirements	2983

A.9.10	SSTD	2983

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	2983

A.9.10.1.1	Test Purpose and Environment	2983

A.9.10.1.2	Test parameters	2983

A.9.10.1.3	Test Requirements	2985

A.9.10.2	Void	2985

A.9.10.3	Void	2985

A.9.10.4	Void	2985

A.9.11 RSSI	2985

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	2985

A.9.11.1.1	Test Purpose and Environment	2985

A.9.11.1.2	Test parameters	2985

A.9.11.1.3	Test Requirements	2988

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	2988

A.9.11.2.1	Test Purpose and Environment	2988

A.9.11.2.2	Test parameters	2988

A.9.12	Channel occupancy	2991

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	2991

A.9.12.1.1	Test Purpose and Environment	2991

A.9.12.1.2	Test parameters	2991

A.9.12.1.3	Test Requirements	2994

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	2994

A.9.12.2.1	Test Purpose and Environment	2994

A.9.12.2.2	Test parameters	2994

A.9.12.2.3	Test Requirements	2997

A.9.13	RS-SINR	2997

A.9.13.1	FDD Intra-Frequency Case	2997

A.9.13.1.1	Test Purpose and Environment	2997

A.9.13.1.2	Test parameters	2997

A.9.13.1.3	Test Requirements	3000

A.9.13.2	TDD Intra-Frequency Case	3000

A.9.13.2.1	Test Purpose and Environment	3000

A.9.13.2.2	Test parameters	3000

A.9.13.2.3	Test Requirements	3003

A.9.13.3	FDD—FDD Inter frequency case	3003

A.9.13.3.1	Test Purpose and Environment	3003

A.9.13.3.2	Test parameters	3003

A.9.13.3.3	Test Requirements	3006

A.9.13.4	TDD—TDD Inter frequency case	3006

A.9.13.4.1	Test Purpose and Environment	3006

A.9.13.4.2	Test parameters	3006

A.9.13.4.3	Test Requirements	3010

A.9.13.5	FDD—TDD Inter frequency case	3011

A.9.13.5.1	Test Purpose and Environment	3011

A.9.13.5.2	Test parameters	3011

A.9.13.5.3	Test Requirements	3017

A.9.13.6	TDD—FDD Inter frequency case	3017

A.9.13.6.1	Test Purpose and Environment	3017

A.9.13.6.2	Test parameters	3017

A.9.13.6.3	Test Requirements	3021

A.9.14	Channel quality reporting accuracy	3021

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	3021

A.9.14.1.1	Test Purpose and Environment	3021

A.9.14.1.2	Test parameters	3022

A.9.14.1.3	Test Requirements	3022

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	3022

A.9.14.2.1	Test Purpose and Environment	3022

A.9.14.2.2	Test parameters	3023

A.9.14.2.3	Test Requirements	3023

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	3024

A.9.14.3.1	Test Purpose and Environment	3024

A.9.14.3.2	Test parameters	3024

A.9.14.3.3	Test Requirements	3025

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	3025

A.9.14.4.1	Test Purpose and Environment	3025

A.9.14.4.2	Test parameters	3025

A.9.14.4.3	Test Requirements	3026

A.10	Proximity-based Services in Any Cell Selection State	3049

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	3049

A.10.1.1	Test Purpose and Environment	3049

A.10.1.2	Test Requirements	3050

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3051

A.10.2.1	Test Purpose and Environment	3051

A.10.2.2	Test Requirements	3052

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	3052

A.10.3.1	Test Purpose and Environment	3052

A.10.3.2	Test Requirements	3054

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	3055

A.10.4.1	Test Purpose and Environment	3055

A.10.4.2	Test Requirements	3057

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	3057

A.11.1	V2V UE Transmission Timing Accuracy Test	3057

A.11.1.1	Test Purpose and Environment	3057

A.11.1.2	Test requirements	3058

A.11.2	Interruptions due to V2V sidelink communication	3058

A.11.2.1	Test Purpose and Environment	3058

A.11.2.2	Test Requirements	3060

A.12	3060

A.12.1	V2X UE Transmission Timing Accuracy Test	3060

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	3060

A.12.1.1.1	Test Purpose and Environment	3060

A.12.1.1.2	Test requirements	3061

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	3062

A.12.1.2.1	Test Purpose and Environment	3062

A.12.1.2.2	Test Requirements	3062

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	3063

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	3063

A.12.2.1.1	Test Purpose and Environment	3063

A.12.2.1.2	Test Requirements	3064

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	3065

A.12.2.2.1	Test Purpose and Environment	3065

A.12.2.2.2	Test Requirements	3066

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	3066

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	3066

A.12.3.1.1	Test Purpose and Environment	3066

A.12.3.1.2	Test Requirements	3069

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	3070

A.12.3.2.1	Test Purpose and Environment	3070

A.12.3.1.2	Test Requirements	3072

A.12.4	Congestion Control Measurement Test for V2X UE	3072

A.12.4.1	Test Purpose and Environment	3072

A.12.4.2	Test Requirements	3074

A.12.5	Interruptions due to V2X Sidelink Communication	3074

A.12.5.1	Test Purpose and Environment	3074

A.12.5.2	Test Requirements	3076

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	3076

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	3076

A.12.6.1.1	Test Purpose and Environment	3076

A.12.6.1.2	Test Requirements	3078

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	3078

A.12.6.2.1	Test Purpose and Environment	3078

A.12.6.1.2	Test Requirements	3080

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	3080

A.12.7.1	Test Purpose and Environment	3080

A.12.7.2	Test Requirements	3083

A.12.8	Interruptions due to V2X Carrier Aggregation	3084

A.12.8.1	Interruptions on a FDD PCell	3084

A.12.8.1.1	Test Purpose and Environment	3084

A.12.8.1.2	Test Requirements	3085

A.12.8.2	Interruptions on a TDD PCell	3085

A.12.8.2.1	Test Purpose and Environment	3085

A.12.8.2.2	Test Requirements	3087

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	3088

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	3088

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	3088

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	3088

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	3088

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	3090

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	3091

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	3091

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	3091

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	3091

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	3093

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	3094

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	3094

B.2.1	Conditions for E-UTRAN intra-frequency measurements	3094

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	3095

B.2.3	Conditions for E-UTRAN inter-frequency measurements	3095

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	3096

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	3096

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	3096

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	3097

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	3097

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3098

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	3098

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	3098

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	3098

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	3099

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3099

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3099

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	3100

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	3100

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	3101

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	3101

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	3102

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	3103

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	3104

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	3105

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	3106

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	3106

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	3106

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	3106

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	3107

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	3107

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	3109

B.3	Conditions for measurements performance requirements for UE	3109

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.2	Void	3109

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	3109

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	3109

B.3.5	Conditions for UE Rx – Tx time difference	3109

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	3110

B.3.7	Conditions for inter-frequency RSTD measurements	3110

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	3110

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	3110

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	3110

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	3111

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3111

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	3111

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	3111

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3112

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	3112

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	3112

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	3113

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	3113

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	3113

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	3113

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	3113

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	3113

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	3113

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.21.1	Conditions for RSRP measurements	3114

B.3.21.2	Conditions for RSRQ measurements	3114

B.3.21.3	Conditions for CSI-RSRP measurements	3114

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3114

B.3.22.1	Conditions for RSRP measurements	3114

B.3.22.2	Void	3115

B.3.22.3	Conditions for CSI-RSRP measurements	3115

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	3115

B.3.23.1	Conditions for RSRP measurements	3115

B.3.23.2	Conditions for RSRQ measurements	3115

B.3.23.3	Conditions for CSI-RSRP measurements	3115

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	3116

B.3.24.1	Conditions for RSRP measurements	3116

B.3.24.2	Conditions for RSRQ measurements	3116

B.3.24.3	Conditions for CSI-RSRP measurements	3116

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3116

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	3116

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	3116

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	3117

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3117

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	3117

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3117

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3117

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	3118

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	3118

B.4	RRM Requirements Exceptions	3118

B.4.1	General	3118

B.4.2	Receiver sensitivity relaxation for UE supporting CA	3118

B.4.3	Receiver sensitivity relaxation for UE configured with CA	3118

B.4.3.1	Inter-band carrier aggregation	3118

B.4.3.2	Intra-band non-contiguous carrier aggregation	3118

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	3119

B.5	Conditions for Measurement Performance Requirements for ProSe UE	3119

B.5.1	Conditions for S-RSRP Accuracy Requirements	3119

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	3119

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3120

B.5.4	Conditions for SD-RSRP Accuracy Requirements	3120

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	3121

B.6	Conditions for V2X	3121

B.6.1	Test parameters for GNSS signals	3121

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	3121

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	3121

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	3122

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	3122

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	3122

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	3122

Annex C (informative):	Change history:	3124