| 3GPP TS 36.133 V18.12.0 (2025-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 18) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	106

1	Scope	108

2	References	108

3	Definitions, symbols and abbreviations	110

3.1	Definitions	110

3.2	Symbols	112

3.3	Abbreviations	113

3.4	Test tolerances	116

3.5	Additional notation	116

3.5.1	Groups of bands	116

3.5.1A	Groups of bands for satellite access	119

3.6	General	120

3.6.1	Applicability of requirements in this specification version	120

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	127

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	128

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	129

3.6.2	Applicability of requirements for EN-DC operation	130

3.6.3	Applicability of requirements for NE-DC operation	131

3.6.4	Applicability of requirements for NGEN-DC operation	132

3.6.5	Applicability of 2-step RA and 4-step RA in RRM requirements	132

3.6.6 Void	132

3.6.7	Applicability of NB-IoT inband operation in NTN NR	132

Rel-18 UEs supporting only standalone operation do not need to be tested for in-band operation with NR over NTN. In-band operation with NR NTN is not supported in Rel-17.	133

4	E-UTRAN RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	133

4.2.1	Introduction	133

4.2.2	Requirements	133

4.2.2.1	Measurement and evaluation of serving cell	134

4.2.2.2	Void	135

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	135

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	137

4.2.2.5	Measurements of inter-RAT cells	139

4.2.2.5.1	Measurements of UTRAN FDD cells	139

4.2.2.5.2	Measurements of UTRAN TDD cells	141

4.2.2.5.3	Measurements of GSM cells	143

4.2.2.5.4	Measurements of HRPD cells	144

4.2.2.5.5	Measurements of cdma2000 1X	145

4.2.2.5.6	Measurements of NR cells	146

4.2.2.5.7	Measurements of NR cells subject to CCA	149

4.2.2.5.8	Measurements of NR cells for RedCap	151

4.2.2.6	Evaluation of cell re-selection criteria	153

4.2.2.7	Maximum interruption in paging reception	153

4.2.2.8	void	154

4.2.2.9	UE measurement capability	154

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	155

4.2.2.10	Reselection to CSG cells	155

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	155

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	156

4.2.2.11	Void	157

4.2.2.12	Void	157

4.2.2.13	Void	157

4.2.2.14	UE measurement capability for RedCap	157

4.3	Minimization of Drive Tests (MDT)	158

4.3.1	Introduction	158

4.3.2	Measurements	158

4.3.2.1	Requirements	158

4.3.3	Relative Time Stamp Accuracy	158

4.3.3.1	Requirements	158

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	159

4.3.4.1	 Requirements	159

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	159

4.3.5.1	Requirements for timeSinceFailure	159

4.4	MBSFN Measurements	159

4.4.1	Introduction	159

4.4.2	MBSFN RSRP measurements	159

4.4.3	MBSFN RSRQ measurements	160

4.4.4	MCH BLER measurements	160

4.5	Proximity-based Services	160

4.5.1	Introduction	160

4.5.2	Requirements	160

4.5.2.1	Interruptions with ProSe Direct Discovery	160

4.5.2.2	Interruptions with ProSe Direct Communication	160

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	161

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	161

4.6	Cell Selection and Re-selection Requirements for UE category NB1	162

4.6.1	Cell Selection	162

4.6.2	Cell Re-selection	162

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	162

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	163

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	164

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	165

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	167

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	168

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	169

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	170

4.6.2.7	Maximum interruption in paging reception in normal coverage	172

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	172

4.6.2.8	UE measurement capability	172

4.6.2.9	WUS receptions for NB1	172

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	173

4.6.3.1	Introduction	173

4.6.3.2	Requirements on UE synchronization for transmission using PUR	173

4.6.3.3	Requirements on TA validation for transmission using PUR	173

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	174

4.6A.1	Cell Selection	174

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	174

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	175

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	176

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	177

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	179

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	181

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	182

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	184

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	186

4.6A.2.7	Maximum interruption in paging reception in normal coverage	188

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	188

4.6A.2.8	UE measurement capability	189

4.6A.2.9	WUS receptions for NB1	189

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	190

4.6A.3.1	Introduction	190

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	190

4.6A.3.3	Requirements on TA validation for transmission using PUR	190

4.7	Cell Selection and Re-selection Requirements for UE category M1	190

4.7.1	Cell Selection	190

4.7.2	Cell Re-selection	190

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	190

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	190

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	191

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	192

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	194

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	196

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	196

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	197

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	197

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	198

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	199

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	202

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	203

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	204

4.7.2.3	WUS receptions for UE category M1	204

4.7.3	Channel quality report for UE Category M1 in idle mode	204

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	205

4.7.4.1	Introduction	205

4.7.4.2	Requirements on UE synchronization for transmission using PUR	205

4.7.4.3	Requirements on TA validation for transmission using PUR	205

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	206

4.7A.1	Cell Selection	206

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	206

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	207

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	207

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	209

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	210

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	213

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	215

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	215

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	216

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	216

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	217

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	219

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	221

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	224

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	224

4.7A.2.3	WUS receptions for UE category M1	225

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	225

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	226

4.7A.4.1	Introduction	226

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	226

4.7A.4.3	Requirements on TA validation for transmission using PUR	226

4.8	Idle State Positioning Measurement Requirements for UE category NB1	226

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	226

4.8.1.1	RSTD Measurement Reporting Delay	228

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	228

4.8.2.1	RSTD Measurement Reporting Delay	229

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	230

4.8.3.1	RSTD Measurement Reporting Delay	231

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	232

4.8.4.1	RSTD Measurement Reporting Delay	233

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	234

4.8.5.1	Measurement Reporting Delay	235

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	235

4.8.6.1	Measurement Reporting Delay	236

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	237

4.8.7.1	Measurement Reporting Delay	238

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	238

4.8.8.1	Measurement Reporting Delay	239

4.9	Idle Mode CA Measurement	240

4.9.1	Introduction	240

4.9.2	Requirement	240

4.9.2.1	Detected cell requirement during state transition and Idle mode	240

4.9.2.2	Measurements of inter-frequency CA candidate cells	241

4.9.2.3	Measurements on serving cell	241

4A	E-UTRAN RRC_INACTIVE state mobility	242

4A.1	Cell Re-selection	242

4A.1.1	Introduction	243

4A.1.2	Requirements	243

4A.1.2.1	UE measurement capability	243

4A.1.2.2	Measurement and evaluation of serving cell	243

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	243

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	243

4A.1.2.5	Evaluation of cell re-selection criteria	243

4A.1.2.6	Maximum interruption in paging reception	243

4A.1.2.7	Measurements of inter-RAT NR cells	243

4A.1.2.8	UE measurement capability for RedCap	243

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	243

4A.2	Requirements for UE Category M1	244

4A.2.1	Introduction	244

4A.2.2	Cell Selection	244

4A.2.3	Cell Reselection	244

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	244

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	244

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	245

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	245

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	245

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	245

4A.2.4	Channel quality report for UE Category M1 in idle mode	247

5	E-UTRAN RRC_CONNECTED state mobility	247

5.1	E-UTRAN Handover	248

5.1.1	Introduction	248

5.1.2	Requirements	248

5.1.2.1	E-UTRAN FDD – FDD	248

5.1.2.1.1	Handover delay	248

5.1.2.1.2	Interruption time	248

5.1.2.2	E-UTRAN FDD – TDD	249

5.1.2.2.1	(Void)	250

5.1.2.2.2	(Void)	250

5.1.2.3	E-UTRAN TDD – FDD	250

5.1.2.3.1	(Void)	250

5.1.2.3.2	(Void)	250

5.1.2.4	E-UTRAN TDD – TDD	250

5.1.2.4.1	Handover delay	250

5.1.2.4.2	Interruption time	250

5.1.2.5	E-UTRAN HD–FDD	252

5.1.2.5.1	Handover delay	252

5.1.2.5.2	Interruption time	252

5.1.2.6	E-UTRAN FDD – FDD conditional handover	253

5.1.2.6.1	Handover delay	253

5.1.2.6.2	Measurement time	254

5.1.2.6.3	Preparation time	254

5.1.2.6.4	Interruption time	254

5.1.2.7	E-UTRAN FDD – TDD conditional handover	254

5.1.2.8	E-UTRAN TDD – FDD conditional handover	255

5.1.2.9	E-UTRAN TDD – TDD conditional handover	255

5.2	Void	255

5.3	Handover to other RATs	255

5.3.1	E-UTRAN - UTRAN FDD Handover	255

5.3.1.1	Introduction	255

5.3.1.1.1	Handover delay	255

5.3.1.1.2	Interruption time	255

5.3.2	E-UTRAN - UTRAN TDD Handover	256

5.3.2.1	Introduction	256

5.3.2.2	Requirements	256

5.3.2.2.1	Handover delay	256

5.3.2.2.2	Interruption time	256

5.3.3	E-UTRAN - GSM Handover	257

5.3.3.1	Introduction	257

5.3.3.2	Requirements	257

5.3.3.2.1	Handover delay	257

5.3.3.2.2	Interruption time	257

5.3.4	E-UTRAN - NR FR1 Handover	257

5.3.4.1	Introduction	257

5.3.4.2	Handover delay	258

5.3.4.3	Interruption time	258

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	258

5.3.4A.1	Introduction	258

5.3.4A.2	Handover delay	259

5.3.4A.3	Interruption time	259

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	260

5.3.4B.1	Introduction	260

5.3.4B.2	Requirements	260

5.3.5	E-UTRAN - NR FR2 Handover	260

5.3.5.1	Introduction	260

5.3.5.2	Handover delay	260

5.3.5.3	Interruption time	260

5.4	Handover to Non-3GPP RATs	261

5.4.1	E-UTRAN – HRPD Handover	261

5.4.1.1	Introduction	261

5.4.1.1.1	Handover delay	261

5.4.1.1.2	Interruption time	261

5.4.2	E-UTRAN – cdma2000 1X Handover	262

5.4.2.1	Introduction	262

5.4.2.1.1	Handover delay	262

5.4.2.1.2	Interruption time	262

5.5	E-UTRAN Handover for Cat-M1 UEs	262

5.5.1	Introduction	262

5.5.2	Requirements in CEModeA	263

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	263

5.5.2.1.1	Handover delay	263

5.5.2.1.2	Interruption time	263

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	263

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	264

5.5.2.3.1	Void	264

5.5.2.3.2	Void	264

5.5.3	Requirements in CEModeB	264

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	264

5.5.3.1.1	Handover delay	264

5.5.3.1.2	Interruption time	264

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	265

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	265

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	265

5.5A.1	Introduction	265

5.5A.2	Requirements in CEModeA	265

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	265

5.5A.2.1.1	Handover delay	265

5.5A.2.1.2	Interruption time	265

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	266

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	266

5.5A.2.3.1	Handover delay	266

5.5A.2.3.2	Measurement time	267

5.5A.2.3.3	Preparation time	268

5.5A.2.3.4	Interruption time	268

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	269

5.5A.3	Requirements in CEModeB	269

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	269

5.5A.3.1.1	Handover delay	269

5.5A.3.1.2	Interruption time	269

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	270

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	270

5.5A.3.3.1	Handover delay	270

5.5A.3.3.2	Measurement time	271

5.5A.3.3.3	Preparation time	272

5.5A.3.3.4	Interruption time	272

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	273

5.6	Void	273

5.7	E-UTRAN DAPS Handover	273

5.7.1	Introduction	273

5.7.2	Requirements	274

5.7.2.1	E-UTRAN FDD – FDD	274

5.7.2.1.1	DAPS Handover delay	274

5.7.2.1.2	Interruption time	274

5.7.2.2	E-UTRAN FDD – TDD	275

5.7.2.3	E-UTRAN TDD – FDD	275

5.7.2.4	E-UTRAN TDD – TDD	275

5.8	EN-DC Handover with PSCell	275

5.8.1	Introduction	275

5.8.1.1	Handover with PSCell Interruption time	275

5.8.1.2	Handover with PSCell - NR PSCell Change Delay requirements	276

5.9	EN-DC Handover with PSCell using CCA	277

5.9.1	Introduction	277

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	277

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	277

6	RRC Connection Mobility Control	278

6.1	RRC Re-establishment	278

6.1.1	Introduction	278

6.1.2	Requirements	278

6.1.2.1	UE Re-establishment delay requirement	279

6.2	Random Access	279

6.2.1	Introduction	279

6.2.2	Requirements	279

6.2.2.1	Contention based random access	279

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	279

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	280

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	280

6.2.2.1.4	Void	280

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	280

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	280

6.2.2.2	Non-Contention based random access	280

6.2.2.2.1	Correct behaviour when receiving Random Access Response	280

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	280

6.2.3	Requirements for Cat-M1 UEs	280

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	281

6.3	RRC Connection Release with Redirection	281

6.3.1	Introduction	281

6.3.2	Requirements	281

6.3.2.1	RRC connection release with redirection to UTRAN FDD	281

6.3.2.2	RRC connection release with redirection to GERAN	281

6.3.2.3	RRC connection release with redirection to UTRAN TDD	282

6.3.2.4	RRC connection release with redirection to NR	282

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	283

6.3.2.6	RRC connection release with redirection to NR Redcap	284

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	285

6.4.1	Introduction	285

6.4.2	Requirements	285

6.5	RRC Re-establishment for NB-IoT UEs	286

6.5.1	Introduction	286

6.5.2	Requirements	286

6.5.2.1	UE Re-establishment delay requirement in normal coverage	286

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	287

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	287

6.5A.1	Introduction	287

6.5A.2	Requirements	287

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	288

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	288

6.6	Random Access for UE category NB1	289

6.6.1	Introduction	289

6.6.2	Requirements	289

6.6.2.1	Correct behaviour when receiving Random Access Response reception	289

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	289

6.6.2.3	Correct behaviour when receiving a NACK on msg3	289

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	289

6.6.2.5	Correct behaviour when contention Resolution timer expires	289

6.6.2.6	MSG3-based channel quality report for UE Category NB1	290

6.6.3	Requirements for NPRACH configuration	290

6.6A	Random Access for UE category NB-IoT for Satellite Access	290

6.6A.1	Introduction	290

6.6A.2	Requirements	291

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	291

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	291

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	291

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	291

6.6A.2.5	Correct behaviour when contention Resolution timer expires	291

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	291

6.6A.3	Requirements for NPRACH configuration	292

6.7	RRC Re-establishment for Cat-M1 UEs	292

6.7.1	Introduction	292

6.7.2	Requirements	292

6.7.2.1	UE Re-establishment delay requirement for CEModeA	293

6.7.2.2	UE Re-establishment delay requirement for CEModeB	293

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	294

6.7A.1	Introduction	294

6.7A.2	Requirements	294

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	294

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	295

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	295

6.8.1	Introduction	295

6.8.2	Requirements	296

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	296

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	296

6.8A.1	Introduction	296

6.8A.2	Requirements	297

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	297

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	297

6.9.1	Introduction	297

6.9.2	Requirements	298

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	298

6.9A.1	Introduction	298

7	Timing and signalling characteristics	299

7.1	UE transmit timing	299

7.1.1	Introduction	299

7.1.2	Requirements	300

7.2	UE timer accuracy	301

7.2.1	Introduction	301

7.2.2	Requirements	301

7.3	Timing Advance	301

7.3.1	Introduction	301

7.3.2	Requirements	301

7.3.2.1	Timing Advance adjustment delay	301

7.3.2.2	Timing Advance adjustment accuracy	302

7.4	Cell phase synchronization accuracy (TDD)	302

7.4.1	Definition	302

7.4.2	Minimum requirements	302

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	303

7.5.1	Introduction	303

7.5.2	eNodeB Synchronization Requirements	303

7.5.2.1	Synchronized E-UTRAN	303

7.5.2.2	Non-Synchronized E-UTRAN	303

7.6	Radio Link Monitoring	303

7.6.1	Introduction	303

7.6.2	Requirements	305

7.6.2.1	Minimum requirement when no DRX is used	305

7.6.2.2	Minimum requirement when DRX is used	305

7.6.2.3	Minimum requirement at transitions	307

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	307

7.6.2.5	Minimum requirement under IDC Interference	307

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	307

7.7.1	Introduction	307

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	308

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	309

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	310

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	311

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	312

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	313

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	314

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	314

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	314

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	315

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	316

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	317

7.7.14	SCell Activation Delay Requirement for Dormant SCell	317

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	319

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	319

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	321

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	321

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	323

7.8	Interruptions with Carrier Aggregation	325

7.8.1	Introduction	325

7.8.2	Requirements	325

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	325

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	325

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	325

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	326

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	326

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	326

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	326

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	326

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	327

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	328

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	328

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	328

7.8.2.13	Interruptions at SRS carrier based switching	329

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	330

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	330

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	330

7.8.2.17	Interruptions during CQI measurement on dormant SCell	330

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	331

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	331

7.8.2.20	Interruptions at SCell hibernation	331

7.8.2.21	Interruptions at direct SCell activation and hibernation	331

7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap	332

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	333

7.9.1	Introduction	333

7.9.2	Minimum Requirements for Interband Carrier Aggregation	333

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	333

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	333

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	334

7.10.1	Introduction	334

7.10.2	Requirements	334

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	334

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	334

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	334

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	335

7.11	Radio Link Monitoring for UE Category 0	335

7.11.1	Introduction	335

7.11.2	Requirements for FD-FDD and TDD	336

7.11.2.1	Minimum requirement when no DRX is used	336

7.11.2.2	Minimum requirement when DRX is used	337

7.11.2.3	Minimum requirement at transitions	337

7.11.3	Requirements for HD-FDD	338

7.11.3.1	Minimum requirement when no DRX is used	338

7.11.3.2	Minimum requirement when DRX is used	338

7.11.3.3	Minimum requirement at transitions	339

7.12	Interruptions with Dual Connectivity	339

7.12.1	Introduction	339

7.12.2	Requirements	339

7.12.2.1	Interruptions at PSCell addition/release	339

7.12.2.2	Interruptions at transitions between active and non-active during DRX	339

7.12.2.3	Interruptions at transitions from non-DRX to DRX	340

7.12.2.4	Interruptions at SCell addition/release	340

7.12.2.5	Interruptions at SCell activation/deactivation	340

7.12.2.6	Interruptions during measurements on SCC	341

7.12.2.7	Interruptions at SRS carrier based switching	341

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	342

7.13.1	Definition	342

7.13.2	Minimum requirements	342

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	342

7.14.1	Introduction	342

7.14.2	PSCell Addition Delay Requirement	342

7.14.3	PSCell Release Delay Requirement	343

7.15	Maximum Receive Timing Difference in Dual Connectivity	343

7.15.1	Introduction	343

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	343

7.16	Proximity-based Services	343

7.16.1	Introduction	343

7.16.2	Requirements	343

7.16.2.1	ProSe UE transmission timing	343

7.16.2.1.1	Serving cell or PCell as timing reference	344

7.16.2.1.2	SCell or non-serving cell as timing reference	344

7.16.3	Interruptions with ProSe	344

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	344

7.16.3.2	Interruptions at ProSe Direct Communication configuration	345

7.16.3.3	Interruptions during ProSe Direct Discovery	345

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	345

7.16.3.5	Interruptions during ProSe Direct Communication	346

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	346

7.16.4.1	Measurement and evaluation of selected cell	346

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	346

7.16.5	Selection / Reselection of ProSe relay UE	347

7.16.6	ProSe operation under deactivated SCell	347

7.17	Maximum Transmission Timing Difference in Dual Connectivity	347

7.17.1	Introduction	347

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	348

7.18.1	Introduction	348

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	348

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	348

7.19	Radio Link Monitoring for UE Category M1	348

7.19.1	Introduction	348

7.19.2	Requirements for FD-FDD and TDD CE mode A	349

7.19.2.1	Minimum requirement when no DRX is used	350

7.19.2.2	Minimum requirement when DRX is used	351

7.19.2.3	Minimum requirement at transitions	352

7.19.3	Requirements for HD-FDD with CE mode A	352

7.19.3.1	Minimum requirement when no DRX is used	353

7.19.3.2	Minimum requirement when DRX is used	353

7.19.3.3	Minimum requirement at transitions	354

7.19.4	Requirements for FD-FDD and TDD with CE mode B	354

7.19.4.1	Minimum requirement when no DRX is used	356

7.19.4.2	Minimum requirement when DRX is used	357

7.19.4.3	Minimum requirement at transitions	358

7.19.5	Requirements for HD-FDD with CE mode B	358

7.19.5.1	Minimum requirement when no DRX is used	358

7.19.5.2	Minimum requirement when DRX is used	359

7.19.5.3	Minimum requirement at transitions	360

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	360

7.19A.1	Introduction	360

7.19A.2	Requirements for FD-FDD and CE mode A	360

7.19A.2.1	Minimum requirement when no DRX is used	362

7.19A.2.2	Minimum requirement when DRX is used	363

7.19A.2.3	Minimum requirement at transitions	364

7.19A.3	Requirements for HD-FDD with CE mode A	364

7.19A.3.1	Minimum requirement when no DRX is used	364

7.19A.3.2	Minimum requirement when DRX is used	365

7.19A.3.3	Minimum requirement at transitions	366

7.19A.4	Requirements for HD-FDD with CE mode B	366

7.19A.4.1	Minimum requirement when no DRX is used	366

7.19A.4.2	Minimum requirement when DRX is used	366

7.19A.4.3	Minimum requirement at transitions	367

7.20	UE transmit timing for NB-IoT	368

7.20.1	Introduction	368

7.20.2	Requirements	368

7.20A	UE transmit timing for NB-IoT for Satellite Access	368

7.20A.1	Introduction	368

7.20A.2	Requirements	369

7.21	 UE timer accuracy for NB-IoT	369

7.21.1	Introduction	369

7.21.2	Requirements	369

7.21A	UE timer accuracy for NB-IoT for Satellite Access	370

7.21A.1	Introduction	370

7.21A.2	Requirements	370

7.22	Timing Advance for NB-IoT	370

7.22.1	Introduction	370

7.22.2	Requirements	370

7.22.2.1	Timing Advance adjustment delay	370

7.22.2.2	Timing Advance adjustment accuracy	370

7.22A	Timing Advance for NB-IoT for Satellite Access	371

7.22A.1	Introduction	371

7.22A.2	Requirements	371

7.22A.2.1	Timing Advance adjustment delay	371

7.22A.2.2	Timing Advance adjustment accuracy	371

7.23	Radio Link Monitoring for Category NB1 UE	371

7.23.1	Introduction	371

7.23.2	Requirements for Category NB1 UE	371

7.23.2.1	Minimum requirement when no DRX is used	372

7.23.2.2	Minimum requirement when DRX is used	372

7.23.2.3	Minimum requirement at transitions	373

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	373

7.23A.1	Introduction	373

7.23A.2	Requirements for Category NB1 UE	373

7.23A.2.1	Minimum requirement when no DRX is used	374

7.23A.2.2	Minimum requirement when DRX is used	374

7.23A.2.3	Minimum requirement at transitions	375

7.24	UE transmit timing for Category M1	375

7.24.1	Introduction	375

7.24.2	Requirements	375

7.24A	UE transmit timing for Category M1 for Satellite Access	376

7.24A.1	Introduction	376

7.24A.2	Requirements	376

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	377

7.25.1	Definition	377

7.25.2	Minimum requirements	377

7.26	UE transmit timing for Category M2	378

7.26.1	Introduction	378

7.26.2	Requirements	378

7.27	UE timer accuracy for category M1	378

7.27.1	Introduction	378

7.27.2	Requirements	378

7.27A	UE timer accuracy for category M1 for Satellite Access	378

7.27A.1	Introduction	378

7.27A.2	Requirements	378

7.28	Timing Advance for Category M1	379

7.28.1	Introduction	379

7.28.2	Requirements	379

7.28A	Timing Advance for Category M1 for Satellite Access	379

7.28A.1	Introduction	379

7.28A.2	Requirements	379

7.28A.2.1	Timing Advance adjustment delay	379

7.28A.2.2	Timing Advance adjustment accuracy	379

7.29	Interruptions requirements with FeMBMS	379

7.29.1	Introduction	379

7.29.2	Requirements	379

7.30	Numerology switching delay requirements with FeMBMS	380

7.30.1	Introduction	380

7.30.2	Requirements	380

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	380

7.31.1	Introduction	380

7.31.2	NR PSCell Addition Delay Requirement	380

7.31.3	NR PSCell Release Delay Requirement	381

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	381

7.31A.1	Introduction	381

7.31A.2	NR PSCell Addition Delay Requirement	381

7.31A.3	NR PSCell Release Delay Requirement	382

7.32	Interruptions with EN-DC	383

7.32.1	Introduction	383

7.32.2	Requirements	383

7.32.2.1	Interruptions at PSCell addition/release	383

7.32.2.2	Interruptions at transitions between active and non-active during DRX	383

7.32.2.3	Interruptions at transitions from non-DRX to DRX	384

7.32.2.4	Interruptions at SCell addition/release	384

7.32.2.5	Interruptions at SCell activation/deactivation	384

7.32.2.6	Interruptions during measurements on SCC	385

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	385

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	385

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	385

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	385

7.32.2.7	Interruptions at active BWP switching	385

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	386

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	386

7.32.2.10	Interruptions at SCell hibernation	386

7.32.2.11	Interruptions at direct SCell activation and hibernation	386

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	387

7.32.2.13	Interruptions at NR SRS carrier based switching	387

7.32.2.14	Interruptions at NR SCell dormancy	388

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	388

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	388

7.32.2.15	Interruption during NR measurement with autonomous gaps	388

7.32.2.16	Interruptions at SRS carrier based switching	389

7.32.2.17	Interruptions at SCG activation/deactivation	389

7.32.2.18	Interruptions due to NR SRS antenna port switching	389

7.32.2.19	Interruptions at fast SCell activation/deactivation	390

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	390

7.32.2.21	Interruptions during RLM/BFD measurements on deactivated PScell	391

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	391

7.33.1	Introduction	391

7.33.2	Requirements	391

7.34	Void	391

7.35	Interruptions with SFTD measurements	391

7.35.1	Introduction	391

7.35.2	Requirements	391

7.36	Interruptions with NE-DC	392

7.36.1	Introduction	392

7.36.2	Requirements	392

7.36.2.1	Interruptions at transitions between active and non-active during DRX	392

7.36.2.2	Interruptions at transitions from non-DRX to DRX	392

7.36.2.3	Interruptions at SCell addition/release	392

7.36.2.4	Interruptions at SCell activation/deactivation	393

7.36.2.5	Interruptions during measurements on SCC	393

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	393

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	393

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	394

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	394

7.36.2.6	Interruptions at active BWP switching	394

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	395

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	395

7.36.2.9	Interruptions at SCell hibernation	395

7.36.2.10	Interruptions at direct SCell activation and hibernation	395

7.36.2.11	Interruptions at NR SRS carrier based switching	395

7.36.2.12	Interruptions at NR SCell dormancy	396

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	396

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	396

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	397

7.36.2.14	Interruption during NR measurement with autonomous gaps	397

7.36.2.15	Interruptions at SRS carrier based switching	397

7.36.2.16	Interruptions due to NR SRS antenna port switching	398

7.37	Interruptions during NR measurement with autonomous gaps	399

7.37.1	Introduction	399

7.37.2	Requirements	399

7.38 SCG Activation and Deactivation Delay	399

7.38.1	Introduction	399

7.38.2	SCG Activation Delay Requirement	399

7.38.3	SCG Deactivation Delay Requirement	400

8	UE Measurements Procedures in RRC_CONNECTED State	400

8.1	General Measurement Requirements	400

8.1.1	Introduction	400

8.1.2	Requirements	401

8.1.2.1	UE measurement capability	401

8.1.2.1.1	Monitoring of multiple layers using gaps	409

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	410

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	411

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	413

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	414

8.1.2.1.2	Network controlled small gap	414

8.1.2.2	E-UTRAN intra frequency measurements	416

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	416

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	421

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	425

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	426

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	427

8.1.2.3	E-UTRAN inter frequency measurements	427

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	428

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	433

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	440

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	440

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	440

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	441

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	443

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	444

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	445

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	452

8.1.2.4	Inter RAT measurements	452

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	452

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	457

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	457

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	461

8.1.2.4.5	E-UTRAN FDD – GSM measurements	461

8.1.2.4.6	E-UTRAN TDD – GSM measurements	466

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	466

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	468

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	468

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	468

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	469

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	469

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	469

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	469

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	471

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	471

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	471

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	471

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	472

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	472

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	474

8.1.2.4.21	E-UTRAN FDD – NR measurements	474

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	478

8.1.2.4.22	E-UTRAN TDD – NR measurements	482

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	482

8.1.2.4.23	Void	482

8.1.2.4.24	Void	482

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	482

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	484

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	484

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	485

8.1.2.4.29	E-UTRAN FDD – NR measurements without measurement gap	485

8.1.2.4.30	E-UTRAN TDD – NR measurements without measurement gap	490

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	490

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	490

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	492

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	493

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	495

8.1.2.6.5	Void	497

8.1.2.6.6	Void	497

8.1.2.6.7	Void	497

8.1.2.6.8	Void	497

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	497

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	497

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	499

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	501

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	503

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	504

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	506

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	508

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	510

8.1.2.7	E-UTRAN E-CID Measurements	511

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	511

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	513

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	514

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	515

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	515

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	515

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	518

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	521

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	525

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	528

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	528

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	528

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	529

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	529

8.1.2.10	Void	530

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	530

8.2.1	Introduction	530

8.2.2	Requirements	530

8.3	Measurements for E-UTRA carrier aggregation	534

8.3.1	Introduction	534

8.3.2	Measurements of the primary component carrier	534

8.3.3	Measurements of a secondary component carrier	534

8.3.3.1	Measurements of a secondary component carrier with active SCell	534

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	535

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	535

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	536

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	538

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	538

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	538

8.4.1	Introduction	538

8.4.2	Measurements on the primary component carrier	539

8.4.3	Measurements on a secondary component carrier	540

8.4.4	Measurements on both primary component carrier and a secondary component carrier	540

8.4.5	Measurements on different secondary component carriers	541

8.5	Measurements for UE category 0	542

8.5.1	Introduction	542

8.5.2 	Requirements	543

8.5.2.1 	E-UTRAN intra frequency measurements	543

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	543

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	546

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	548

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	552

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	552

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	553

8.6	Discovery signal measurements	554

8.6.1	Introduction	554

8.6.2	Requirements for CRS based discovery signal measurements	554

8.6.2.1	E-UTRAN intra frequency measurements	554

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	554

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	557

8.6.2.2	E-UTRAN inter frequency measurements	559

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	560

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	562

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	565

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	565

8.6.3	Requirements for CSI-RS based discovery signal measurements	565

8.6.3.1	E-UTRAN intra frequency measurements	565

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	566

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	568

8.6.3.2	E-UTRAN inter frequency measurements	570

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	571

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	573

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	576

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	576

8.7	Discovery signal measurements for E-UTRA carrier aggregation	576

8.7.1	Introduction	576

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	577

8.7.2.1	Measurements of the primary component carrier	577

8.7.2.2	Measurements of a secondary component carrier	577

8.7.2.3	Measurements of a secondary component carrier with active SCell	577

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	577

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	577

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	578

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	580

8.7.3.1	Measurements of the primary component carrier	580

8.7.3.2	Measurements of a secondary component carrier	580

8.7.3.3	Measurements of a secondary component carrier with active SCell	580

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	580

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	580

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	582

8.8	Measurements for E-UTRA dual connectivity	583

8.8.1	Introduction	583

8.8.2	Intra-frequency measurements requirements on PCell	583

8.8.3	Intra-frequency measurements requirements on PSCell	584

8.8.4	Inter-frequency and inter-RAT measurement requirements	584

8.8.5	Intra-frequency measurements with autonomous gaps	584

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	584

8.8.5.2	ECGI reporting delay	585

8.8.6	Inter-frequency measurements with autonomous gaps	585

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	585

8.8.6.2	ECGI reporting delay	586

8.8.7	SSTD Measurements	586

8.8.7.1	Introduction	586

8.8.7.2	SSTD Measurement requirements	586

8.8.7.3	SSTD Measurement Reporting Delay	587

8.8.8	Intra-frequency measurements requirements on SCell	587

8.9	MBSFN Measurements	587

8.9.1	Introduction	587

8.9.2	MBSFN RSRP Measurements	587

8.9.3	MBSFN RSRQ Measurements	588

8.9.4	MCH BLER Measurements	588

8.10	Proximity-based Services	588

8.10.1	Introduction	588

8.10.2	Requirements	588

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	588

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	589

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	590

8.11.1	Introduction	590

8.11.2	CRS based discovery signal measurements	590

8.11.2.1	E-UTRAN intra-frequency measurements	590

8.11.2.1.1	Requirements	590

8.11.2.1.1.1	Requirements when no DRX is used	590

8.11.2.1.1.1.1	Measurement Reporting Requirements	592

8.11.2.1.1.2	Requirements when DRX is used	593

8.11.2.1.1.2.1	Measurement Reporting Requirements	595

8.11.2.2	E-UTRAN inter-frequency measurements	596

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	596

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	600

8.11.3	CSI-RS based discovery signal measurements	600

8.11.3.1	E-UTRAN intra-frequency measurements	600

8.11.3.1.1	Requirements	600

8.11.3.1.1.1	Requirements when no DRX is used	600

8.11.3.1.1.1.1	Measurement Reporting Requirements	602

8.11.3.1.1.2	Requirements when DRX is used	602

8.11.3.1.1.2.1	Measurement Reporting Requirements	603

8.11.3.2	E-UTRAN inter-frequency measurements	604

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	604

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	608

8.11.4	RSSI measurements	608

8.11.4.1	E-UTRAN intra-frequency measurements	608

8.11.4.2	E-UTRAN inter-frequency measurements	608

8.11.5	Channel occupancy measurements	609

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	609

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	609

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	609

8.12.1	Introduction	609

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	609

8.12.2.1	Introduction	609

8.12.2.2	Measurements of a secondary component carrier	609

8.12.2.3	Measurements of a secondary component carrier with active SCell	609

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	610

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	610

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	612

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	615

8.12.3.1	Introduction	615

8.12.3.2	Measurements of a secondary component carrier	615

8.12.3.3	Measurements of a secondary component carrier with active SCell	615

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	615

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	615

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	617

8.13	Measurements for UE Category M1	619

8.13.1	Introduction	619

8.13.2	Requirements for UE category M1 with CE mode A	619

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	620

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	620

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	625

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	627

8.13.2.2	Void	631

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	631

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	631

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	634

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	636

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	637

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	637

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	639

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	641

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	642

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	642

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	642

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	643

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	643

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	643

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	644

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	644

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	645

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	646

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	646

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	647

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	651

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	653

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	657

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	657

8.13.3	Requirements for UE category M1 with CE mode B	658

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	658

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	659

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	663

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	666

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	671

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	672

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	672

8.13.3.2	Void	673

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	673

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	673

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	676

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	678

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	679

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	679

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	679

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	680

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	680

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	680

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	681

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	681

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	681

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	686

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	688

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	693

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	693

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	693

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	695

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	698

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	698

8.13A	Measurements for UE Category M1 for Satellite Access	699

8.13A.1	Introduction	699

8.13A.2	Requirements for UE category M1 with CE mode A	699

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	700

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	700

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	700

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	702

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	704

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	704

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	704

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	706

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	706

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	706

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	708

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	710

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	710

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	710

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	712

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	713

8.13A.3	Requirements for UE category M1 with CE mode B	713

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	714

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	714

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	714

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	716

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	718

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	718

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	719

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	721

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	721

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	721

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	723

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	725

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	725

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	725

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	727

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	728

8.14	Measurements for UE category NB1	728

8.14.1	Introduction	728

8.14.2	NB-IoT intra frequency measurements under normal coverage	728

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	728

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	728

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	729

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	729

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	729

8.14.4	Connected mode channel quality report for UE Category NB1	729

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	729

8.14.6	NB-IoT neighbour cell measurements	730

8.14.6.1	Introduction	730

8.14.6.2	Requirements	730

8.14.6.3	Intra-frequency neighbour cell measurements	730

8.14.6.4	Inter-frequency neighbour cell measurements	731

8.14.6.5	 Requirements for monitoring multiple carriers	732

8.14A	Measurements for UE category NB-IoT for Satellite Access	732

8.14A.1	Introduction	732

8.14A.2	NB-IoT intra frequency measurements under normal coverage	733

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	733

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	733

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	733

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	733

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	733

8.14A.4	Connected mode channel quality report for UE Category NB1	733

8.15	Void	736

8.16	Measurements for UE Category M2	736

8.16.1	Introduction	736

8.16.2	Requirements for UE category M2 with CE mode A	737

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	737

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	737

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	738

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	738

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	739

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	739

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	739

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	740

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	742

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	744

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	745

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	745

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	747

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	750

8.16.3	Requirements for UE category M2 with CE mode B	750

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	750

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	750

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	753

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	755

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	755

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	756

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	758

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	760

8.17	Measurements for E-UTRA – NR Dual Connectivity	761

8.17.1	Introduction	761

8.17.1.1	Measurement Gap Sharing	761

8.17.1A	Intrafrequency Measurements	761

8.17.2	SFTD Measurements	762

8.17.2.1	Introduction	762

8.17.2.2	SFTD Measurement requirements	762

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	762

8.17.2.3	SFTD Measurement Reporting Delay	763

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	764

8.17.3.1	Introduction	764

8.17.3.2	E-UTRAN FDD inter frequency measurements	764

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	764

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	766

8.17.3.3	E-UTRAN TDD inter frequency measurements	768

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	768

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	769

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	771

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	771

8.17.4.1.1	NR Inter-RAT cell identification	772

8.17.4.1.2	NR Inter-RAT measurement	773

8.17.4.1.3	NR Inter-RAT measurement reporting	774

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	775

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	775

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	775

8.17.4A.1.1	NR Inter-RAT cell identification	776

8.17.4A.1.2	NR Inter-RAT measurement	777

8.17.4A.1.3	NR Inter-RAT measurement reporting	778

8.17.4A.1.4	NR inter-RAT RSSI measurements	778

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	779

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	779

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	780

8.17.5.1	Introduction	780

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	780

8.17.5.2.1	Identification of a new UTRA FDD cell	780

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	780

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	780

8.17.5.2.4	Periodic Reporting	781

8.17.5.2.5	Event Triggered Reporting	781

8.17.5.2.6	Event-triggered Periodic Reporting	781

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	782

8.17.5.3.1	Periodic Reporting	783

8.17.5.3.2	Event Triggered Reporting	783

8.17.5.3.3	Event-triggered Periodic Reporting	783

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	784

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	784

8.17.7.1	Introduction	784

8.17.7.2	Identification of a new UTRA FDD cell for SON	784

8.17.7.2.1	Requirements when no DRX is used	784

8.17.7.2.2	Requirements when DRX is used	784

8.17.7.2.3	Reporting Delay	785

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	786

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	786

8.17.9.1	Introduction	786

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	786

8.17.9.2.1	Identification of a new UTRA FDD cell	786

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	786

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	787

8.17.9.2.4	Periodic Reporting	787

8.17.9.2.5	Event Triggered Reporting	787

8.17.9.2.6	Event-triggered Periodic Reporting	788

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	788

8.17.9.3.1	Periodic Reporting	789

8.17.9.3.2	Event Triggered Reporting	789

8.17.9.3.3	Event-triggered Periodic Reporting	789

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	790

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	790

8.17.11.1	Introduction	790

8.17.11.2	Identification of a new UTRA TDD cell for SON	790

8.17.11.2.1	Requirements when no DRX is used	790

8.17.11.2.2	Requirements when DRX is used	790

8.17.11.2.3	Reporting Delay	791

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	791

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	792

8.17.13.1	Introduction	792

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	792

8.17.13.2.1	GSM carrier RSSI	792

8.17.13.2.2	BSIC verification	792

8.17.13.2.3	Enhanced BSIC verification	794

8.17.13.2.4	Periodic Reporting	794

8.17.13.2.5	Event Triggered Reporting	794

8.17.13.2.6	Event-triggered Periodic Reporting	795

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	795

8.17.13.3.1	GSM carrier RSSI	795

8.17.13.3.2	BSIC verification	795

8.17.13.3.3	Periodic Reporting	797

8.17.13.3.4	Event Triggered Reporting	797

8.17.13.3.5	Event-triggered Periodic Reporting	797

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	797

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	797

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	797

8.17.15.1.1	RSTD Measurement Reporting Delay	798

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	798

8.17.15.2.1	RSTD Measurement Reporting Delay	798

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	798

8.17.15.3.1	RSTD Measurement Reporting Delay	799

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	799

8.17.15.4.1	RSTD Measurement Reporting Delay	799

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	800

8.17.16.1	Introduction	800

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	800

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	800

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	800

8.17.17.1	Introduction	800

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	800

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	800

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	800

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	800

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	800

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	800

8.18	Measurements for non-BL/CE UE	800

8.18.1	Introduction	800

8.18.2	Requirements for non-BL/CE UE with CE Mode B	801

8.18.2.1	E-UTRAN intra frequency measurements	801

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	801

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	801

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	802

8.19	Measurements for NR – E-UTRA Dual Connectivity	803

8.19.1	Introduction	803

8.19.2	Intra-frequency Measurements	803

8.19.3	Inter-frequency Measurements	803

8.19.4	Void	803

8.19.5	Intra-frequency E-CID Measurements	803

8.19.6	Intra-frequency measurements with autonomous gaps	804

8.19.6.1	Introduction	804

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	804

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	804

8.19.7	Inter-frequency measurements with autonomous gaps	804

8.19.7.1	Introduction	804

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	804

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	804

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	804

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	804

8.20	Inter-RAT NR Measurements for RedCap UE	804

8.20.1	Introduction	804

8.20.2	E-UTRAN FDD – NR measurements	804

8.20.2.1	Identification of a new NR cell	804

8.20.2.2	Periodic Reporting	808

8.20.2.3	Event Triggered Reporting	808

8.20.2.4	Event-triggered Periodic Reporting	808

8.20.3	E-UTRAN TDD – NR measurements	808

9	Measurements performance requirements for UE	809

9.1	E-UTRAN measurements	809

9.1.1	Introduction	809

9.1.2	Intra-frequency RSRP Accuracy Requirements	809

9.1.2.1	Absolute RSRP Accuracy	809

9.1.2.2	Relative Accuracy of RSRP	810

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	811

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	811

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	812

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	813

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	814

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	815

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	816

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	816

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	816

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	817

9.1.2B.1	Introduction	817

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	817

9.1.3	Inter-frequency RSRP Accuracy Requirements	818

9.1.3.1	Absolute RSRP Accuracy	818

9.1.3.2	Relative Accuracy of RSRP	819

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	820

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	820

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	821

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	821

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	822

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	823

9.1.3B.1	Introduction	823

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	823

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	823

9.1.4	RSRP Measurement Report Mapping	824

9.1.5	Intra-frequency RSRQ Accuracy Requirements	824

9.1.5.1	Absolute RSRQ Accuracy	824

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	825

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	826

9.1.5.4	Absolute WB-RSRQ Accuracy	827

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	828

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	829

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	829

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	829

9.1.5B.1	Introduction	829

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	829

9.1.6	Inter-frequency RSRQ Accuracy Requirements	830

9.1.6.1	Absolute RSRQ Accuracy	830

9.1.6.2	Relative Accuracy of RSRQ	831

9.1.6.3	Absolute WB-RSRQ Accuracy	832

9.1.6.4	Relative WB-RSRQ Accuracy	832

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	833

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	834

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	834

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	834

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	835

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	836

9.1.6B.1	Introduction	836

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	836

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	836

9.1.7	RSRQ Measurement Report Mapping	837

9.1.8	Power Headroom	837

9.1.8.1	Period	838

9.1.8.2	Reporting Delay	838

9.1.8.3	Void	838

9.1.8.4	Report Mapping	838

9.1.8A	Power Headroom for UE category M1 for satellite access	838

9.1.8A.1	Period	839

9.1.8A.2	Reporting Delay	839

9.1.8A.3	Void	839

9.1.8A.4	Report Mapping	839

9.1.9	UE Rx – Tx time difference	839

9.1.9.1	Measurement Requirement	839

9.1.9.2	Measurement Report mapping	840

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	841

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	842

9.1.10	Reference Signal Time Difference (RSTD)	843

9.1.10.1	Intra-Frequency Accuracy Requirement	843

9.1.10.2	Inter-Frequency Accuracy Requirement	844

9.1.10.3	RSTD Measurement Report Mapping	846

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	846

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	847

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	848

9.1.11	Carrier aggregation measurement accuracy	849

9.1.11.1	Primary component carrier accuracy requirement	850

9.1.11.2	Secondary component carrier accuracy requirement	850

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	850

9.1.11.4	Secondary component carrier relative accuracy requirement	850

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	850

9.1.13	Measurement accuracy for UE category 0	851

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	851

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	851

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	852

9.1.14	Accuracy requirements for Discovery Signal Measurements	853

9.1.14.1	Introduction	853

9.1.14.2	RSRP measurements in discovery signal occasions	853

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	853

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	853

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	853

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	854

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	855

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	855

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	855

9.1.14.3.3	CSI-RSRP measurement report mapping	856

9.1.14.4	RSRQ measurements in discovery signal occasions	856

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	856

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	857

9.1.15.1.1	Primary component carrier accuracy requirement	857

9.1.15.1.2	Secondary component carrier accuracy requirement	857

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	857

9.1.15.1.4	Secondary component carrier relative accuracy requirement	857

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	857

9.1.15.2.1	Primary component carrier accuracy requirement	857

9.1.15.2.2	Secondary component carrier accuracy requirement	857

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	857

9.1.15.2.4	Secondary component carrier relative accuracy requirement	857

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	858

9.1.17	RS-SINR Measurements	858

9.1.17.1	Measurement Report Mapping	858

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	859

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	859

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	859

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	859

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	860

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	861

9.1.18.1	Introduction	861

9.1.18.2	RSRP measurements	861

9.1.18.2.1	RSRP measurement report mapping	861

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	861

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	862

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	862

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	863

9.1.18.3	RSRQ measurements	863

9.1.18.3.1	RSRQ measurement report mapping	863

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	863

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	864

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	864

9.1.18.4	CSI-RSRP measurements	865

9.1.18.4.1	CSI-RSRP measurement report mapping	865

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	865

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	865

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	866

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	866

9.1.18.5	RSSI measurements	867

9.1.18.5.1	RSSI measurement report mapping	867

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	867

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	868

9.1.18.6	Channel occupancy measurements	868

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	868

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	868

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	869

9.1.19.1	Introduction	869

9.1.19.2	Accuracy requirements for measurements on SCC	869

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	869

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	869

9.1.20	SFN and Subframe Time Difference (SSTD)	870

9.1.20.1	SSTD Accuracy Requirement	870

9.1.20.2	SSTD Measurement Report Mapping	870

9.1.21	Measurement accuracy for UE category M1	871

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	871

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	873

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	874

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	876

9.1.21.5	RSRP Measurement Report Mapping	877

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	877

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	878

9.1.21.8	RSRQ Measurement Report Mapping	879

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	879

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	880

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	881

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	882

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	883

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	884

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	885

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	886

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	887

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	888

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	889

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	890

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	893

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	896

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	896

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	899

9.1.21A	Measurement accuracy for UE category M1 for satellite access	900

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	901

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	902

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	903

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	905

9.1.21A.5	RSRP Measurement Report Mapping	906

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	906

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	907

9.1.21A.8	RSRQ Measurement Report Mapping	908

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	908

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	909

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	910

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	911

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	912

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	913

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	914

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	915

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	916

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	916

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	918

9.1.22	Measurement accuracy for UE Category NB1	919

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	919

9.1.22.2	Void	920

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	920

9.1.22.4	Void	921

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	921

9.1.22.6	Void	922

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	922

9.1.22.8	Void	923

9.1.22.9	NRSRP Measurement Report Mapping	923

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	923

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	924

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	925

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	926

9.1.22.14	NRSRQ Measurement Report Mapping	927

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	928

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	928

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	929

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	929

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	929

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	930

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	931

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	932

9.1.22A.5	NRSRP Measurement Report Mapping	933

9.1.22A.6	NRSRQ Measurement Report Mapping	933

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	933

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	933

9.1.23	Power Headroom for UE Category NB1	934

9.1.23.1	Period	934

9.1.23.2	Reporting Delay	934

9.1.23.3	Report Mapping for UE Category NB1	934

9.1.23.3.1	Void	936

9.1.23.3.2	Void	936

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	936

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	937

9.1.23A.1	Period	937

9.1.23A.2	Reporting Delay	937

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	938

9.1.23A.3.1	Void	938

9.1.23A.3.2	Void	938

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	938

9.1.24	Void	938

9.1.25	Measurement accuracy for UE category M2	938

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	938

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	939

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	940

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	941

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	943

9.1.26	Measurement Accuracy for non-BL CE UE	944

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	945

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	947

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	947

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	949

9.1.26.5	RSRP Measurement Report Mapping	950

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	950

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	950

9.1.26.8	RSRQ Measurement Report Mapping	951

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	951

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	952

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	953

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	954

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	955

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	955

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	955

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	956

9.1.27	SFN and frame Timing Difference (SFTD)	957

9.1.27.1	SFTD Accuracy Requirement	957

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	959

9.1.28.1	SFTD Accuracy Requirement under CCA	959

9.2	UTRAN FDD Measurements	960

9.2.1	UTRAN FDD CPICH RSCP	960

9.2.2	Void	961

9.2.3	UTRAN FDD CPICH Ec/No	961

9.3	UTRAN TDD Measurements	961

9.3.1	UTRAN TDD P-CCPCH RSCP	962

9.3.2	Void	962

9.3.3	Void	962

9.4	GSM Measurements	962

9.4.1	GSM carrier RSSI	962

9.5	CDMA2000 1x RTT Measurements	962

9.5.1	CDMA2000 1x RTT Pilot Strength	962

9.6	PCMAX,c	963

9.6.1	Report Mapping	963

9.6.2 	Estimation Period	963

9.6.3	Reporting Delay	963

9.7	IEEE802.11 Measurements	963

9.7.1	WLAN RSSI	963

9.7.2	WLAN RSSI Measurement Report Mapping	963

9.8	MBSFN Measurements	964

9.8.1	Introduction	964

9.8.2	MBSFN RSRP	964

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	964

9.8.2.2	MBSFN RSRP measurement report mapping	965

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	965

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	966

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	966

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	966

9.8.3	MBSFN RSRQ	967

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	967

9.8.3.2	MBSFN RSRQ measurement report mapping	967

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	968

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	968

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	968

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	969

9.8.4	MCH BLER	969

9.8.4.1	Measurement report mapping for MCH BLER	969

9.8.4.2	Measurement report mapping for MCH Block Number	970

9.9	ProSe Measurements	971

9.9.1	Introduction	971

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	971

9.9.2.1	Absolute S-RSRP Accuracy	971

9.9.2.2	Relative Accuracy of S-RSRP	972

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	973

9.9.3.1	Absolute SD-RSRP Accuracy	973

9.9.3.2	Relative Accuracy of SD-RSRP	973

9.10	V2X Measurements	974

9.10.1	Introduction	974

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	974

9.10.2.1	Absolute S-RSRP Accuracy	974

9.10.2.2	Relative Accuracy of S-RSRP	975

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	975

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	975

9.10.4	S-RSSI Measurement Accuracy Requirements	976

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	976

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	976

9.11	NR Measurements	977

9.11.1	NR SS-RSRP Measurements	977

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	977

9.11.2	NR SS-RSRQ Measurements	978

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	978

9.11.3	NR SS-SINR Measurements	978

9.11.4	NR SS-RSRP Measurements under CCA	979

9.11.5	NR SS-RSRQ Measurements under CCA	979

9.11.6	NR SS-SINR Measurements under CCA	979

9.11.7	NR RSSI Measurements under CCA	979

9.11.8	NR Channel Occupancy Measurements under CCA	980

10	Measurements Performance Requirements for E-UTRAN	980

10.1	Received Interference Power	980

10.1.1	Absolute accuracy requirement	980

10.1.2	Relative accuracy requirement	980

10.1.3	Received Interference Power measurement report mapping	981

10.2	Angle of Arrival (AOA)	981

10.2.1	Range/mapping	981

10.3	Timing Advance (TADV)	981

10.3.1	Report mapping	981

11	ProSe Requirements in Any Cell Selection state	982

11.1	Introduction	982

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	982

11.2.1	Introduction	982

11.2.2	ProSe UE transmission timing	982

11.3	Initiation/Cease of SLSS Transmissions	983

11.3.1	Introduction	983

11.3.2	Requirements	983

11.4	Measurements for ProSe in Any Cell Selection State	983

11.4.1	Introduction	983

11.4.2	Requirements	983

11.4.2.1	E-UTRA FDD	983

11.4.2.2	E-UTRA TDD	984

11.5	Selection / Reselection of ProSe Synchronization Reference	984

11.5.1	Introduction	984

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	984

11.5.2.1	Introduction	984

11.5.2.2	Requirements	984

11.6	Void	985

11.7	Selection / Reselection of ProSe relay UE	985

11.7.1	Introduction	985

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	985

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	985

12.1	Introduction	985

12.2	Transmit Timing	986

12.2.1	GNSS as timing reference	986

12.3	Interruption	986

12.4	Reliability of GNSS signal	986

13	V2X Requirements	986

13.1	Introduction	986

13.2	UE Transmit Timing	987

13.2.1	Introduction	987

13.2.2	GNSS as synchronization reference source	987

13.2.3	Serving cell/PCell as synchronization reference source	987

13.2.4	SyncRef UE as synchronization reference source	987

13.3	Initiation/Cease of SLSS Transmissions	987

13.3.1	Introduction	987

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	987

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	988

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	988

13.4	Selection / Reselection of V2X Synchronization Reference Source	989

13.5	Autonomous Resource Selection/Reselection measurements	990

13.5.1	Introduction	990

13.5.2	PSSCH-RSRP measurements	990

13.5.3	S-RSSI measurements	990

13.6	Congestion Control measurements	990

13.7	Interruption	990

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	990

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	990

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	991

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	991

13.8	Reliability of GNSS signal	991

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	992

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	992

Annex A (normative):	Test Cases	993

A.1	Purpose of annex	993

A.2	Requirement classification for statistical testing	993

A.2.1	Types of requirements in TS 36.133	993

A.2.1.1	Time and delay requirements on UE higher layer actions	993

A.2.1.2	Measurements of power levels, relative powers and time	993

A.2.1.3	Implementation requirements	994

A.2.1.4	Physical layer timing requirements	994

A.3	RRM test configurations	995

A.3.1	Reference Measurement Channels	995

A.3.1.1	PDSCH	995

A.3.1.1.1	FDD	995

A.3.1.1.2	TDD	1000

A.3.1.1.3	FDD for UE category 0	1003

A.3.1.1.4	HD-FDD for UE category 0	1004

A.3.1.1.5	TDD for UE category 0	1005

A.3.1.1.6	Frame Structure 3	1006

A.3.1.2	PCFICH/PDCCH/PHICH	1007

A.3.1.2.1	FDD	1007

A.3.1.2.2	TDD	1007

A.3.1.2.3	HD-FDD for UE category 0	1008

A.3.1.2.4	FS 3	1008

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	1008

A.3.1.3.1	FDD in CEModeA	1009

A.3.1.3.2	HD-FDD in CEModeA	1009

A.3.1.3.3	TDD in CEModeA	1010

A.3.1.3.4	FDD in CEModeB	1010

A.3.1.3.5	HD-FDD in CEModeB	1011

A.3.1.3.6	TDD in CEModeB	1011

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	1012

A.3.1.4.1	FDD in CEModeA	1012

A.3.1.4.2	HD-FDD in CEModeA	1013

A.3.1.4.3	TDD in CEModeA	1014

A.3.1.4.4	FDD in CEModeB	1015

A.3.1.4.5	HD-FDD in CEModeB	1016

A.3.1.4.6	TDD in CEModeB	1016

A.3.1.5	NPDSCH Reference Channel for UE category NB1	1017

A.3.1.5.1	HD-FDD in-band operation	1017

A.3.1.5.2	Void	1018

A.3.1.5.3	HD-FDD standalone operation	1018

A.3.1.5.4	Void	1019

A.3.1.5.5	HD-FDD guard band operation	1019

A.3.1.5.6	Void	1020

A.3.1.5.7	TDD in-band operation	1020

A.3.1.5.8	TDD standalone operation	1020

A.3.1.5.9	TDD guard band operation	1021

A.3.1.6	NPDCCH Reference Channel for UE category NB1	1021

A.3.1.6.1	In-band operation	1021

A.3.1.6.2	Void	1022

A.3.1.6.3	Standalone operation	1022

A.3.1.6.4	Void	1022

A.3.1.6.5	Guard band operation	1022

A.3.1.6.6	Void	1023

A.3.2	OFDMA Channel Noise Generator (OCNG)	1023

A.3.2.1	OCNG Patterns for FDD	1023

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	1024

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	1024

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	1025

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	1026

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	1026

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	1027

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	1027

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	1027

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	1028

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	1029

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	1029

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	1030

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	1030

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	1031

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	1031

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	1032

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	1032

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	1033

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	1033

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	1034

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	1034

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	1035

A.3.2.2	OCNG Patterns for TDD	1035

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1036

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	1036

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	1037

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	1037

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	1038

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	1039

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	1040

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	1041

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	1041

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	1042

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	1043

A.3.2.3	OCNG Patterns for Narrowband IoT	1043

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1045

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1046

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	1046

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	1047

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	1048

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1049

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1050

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	1051

A.3.2.3.9	Narrowband IoT OCNG FDD pattern 6: In-band NB-IoT in 5 MHz NTN NR cell	1052

A.3.2.4	OCNG Patterns for V2X sidelink	1053

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1054

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	1054

A.3.3	Reference DRX Configurations	1054

A.3.4	ABS Transmission Configurations	1055

A.3.4.1	Non-MBSFN ABS Transmission Configurations	1055

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	1055

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	1055

A.3.4.2	MBSFN ABS Transmission Configurations	1056

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	1056

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	1057

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	1057

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	1057

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	1058

A.3.6.1	Introduction	1058

A.3.7	Test Cases with Different Channel Bandwidths	1058

A.3.7.1	Introduction	1058

A.3.7.2	Principle of testing	1058

A.3.8	Antenna Configuration	1058

A.3.8.1	Antenna connection for 4 Rx capable UEs	1058

A.3.8.1.1 Introduction	1058

A.3.8.1.2 Principle of testing	1058

A.3.8.1.2.1 Single carrier tests	1058

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	1059

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	1060

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	1060

A.3.8.2	Antenna connection for 8 Rx capable UEs	1060

A.3.8.2.1	Introduction	1060

A.3.8.2.2	Principle of testing	1060

A.3.8.2.2.1	Single carrier tests	1060

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	1061

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	1061

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	1061

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	1061

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	1061

A.3.9.1	Introduction	1061

A.3.9.2	Principle of testing	1061

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	1062

A.3.10.1	Introduction	1062

A.3.10.2	Principle of testing	1062

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	1062

A.3.11.1	Introduction	1062

A.3.11.2	Principle of Testing	1062

A.3.12	Proximity-based Services	1062

A.3.12.1	Introduction	1062

A.3.12.2	Reference DRX configurations for ProSe tests	1062

A.3.12.3	Test Cases with Different Channel Bandwidths	1063

A.3.12.3.1	Introduction	1063

A.3.12.3.2	Principle of testing	1063

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	1063

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	1070

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	1073

A.3.12.6.1	FDD	1073

A.3.12.7	Reference measurement channels for ProSe Direct Communication	1073

A.3.12.7.1	FDD	1073

A.3.12.8	ProSe Receive Traffic Generator	1074

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	1074

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	1074

A.3.13	Time Offset between Cells	1075

A.3.13.1	Introduction	1075

A.3.13.2	Definition	1075

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	1075

A.3.14.1	Introduction	1075

A.3.14.2	Principle of testing	1075

A.3.15	Dual connectivity test cases with different combination of duplex mode	1075

A.3.15.1	Introduction	1075

A.3.15.2	Principle of testing	1075

A.3.16	Reference PRACH Configurations	1076

A.3.17	Listen before talk model	1076

A.3.17.1	Introduction	1076

A.3.17.2	Definition	1076

A.3.18	Reference NPRACH Configurations	1077

A.3.19	Dual connectivity test cases with different bandwidth combinations	1078

A.3.19.1	Introduction	1078

A.3.19.2	Principle of testing	1078

A.3.20	Category M1 UE Test Cases	1079

A.3.20.1	Introduction	1079

A.3.20.2	Principle of Cat-M1 UE Testing	1079

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1080

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	1081

A.3.21.1	Introduction	1081

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	1081

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	1082

A.3.22	Category 1bis UE Test Cases	1083

A.3.22.1	Introduction	1083

A.3.22.2	Principle of Category 1bis UE Testing	1083

A.3.23	Category NB2 UE Test Cases	1087

A.3.23.1	Introduction	1087

A.3.23.2	Principle of Category NB2 UE Testing	1087

A.3.24	V2X sidelink communication	1090

A.3.24.1	Introduction	1090

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	1091

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	1095

A.3.25	Category M2 UE Test Cases	1096

A.3.25.1	Introduction	1096

A.3.25.2	Principle of Cat-M2 UE Testing	1096

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1097

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	1098

A.3.26.1	Introduction	1098

A.3.26.2	Principle of testing	1098

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	1098

A.3.27.1	Introduction	1098

A.3.27.2	Principle of INACTIVE cell re-selection Testing	1098

A.3.28	Testing related to Satellite access	1098

A.3.28.1	Introduction	1098

A.3.28.2	Principle of testing GSO and NGSO scenarios	1098

A.3.28.2	Principle of testing different RRM requirements	1099

A.3.28.3	Principle of testing different ephemeris formats	1103

A.3.28.4	General setup for SIB31/SIB-31-NB	1103

A.3.28.5	Satellite specific parameters configuration	1103

A.3.28.5.1	Satellite specific configuration for serving cell	1103

A.3.28.5.2	Satellite specific configuration for neighbour cell	1103

A.4	E-UTRAN RRC_IDLE state	1104

A.4.2	Cell Re-Selection	1104

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	1104

A.4.2.1.1	Test Purpose and Environment	1104

A.4.2.1.2	Test Requirements	1106

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	1107

A.4.2.2.1	Test Purpose and Environment	1107

A.4.2.2.2	Test Requirements	1108

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	1109

A.4.2.3.1	Test Purpose and Environment	1109

A.4.2.3.2	Test Requirements	1110

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	1111

A.4.2.4.1	Test Purpose and Environment	1111

A.4.2.4.2	Test Requirements	1112

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	1113

A.4.2.5.1	Test Purpose and Environment	1113

A.4.2.5.2	Test Requirements	1114

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	1115

A.4.2.6.1	Test Purpose and Environment	1115

A.4.2.6.2	Test Requirements	1116

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	1117

A.4.2.7.1	Test Purpose and Environment	1117

A.4.2.7.2	Test Requirements	1118

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	1119

A.4.2.8.1	Test Purpose and Environment	1119

A.4.2.8.2	Test Requirements	1120

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	1121

A.4.2.9.1	Test Purpose and Environment	1121

A.4.2.9.2	Test Requirements	1121

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	1121

A.4.2.10.1	Test Purpose and Environment	1121

A.4.2.10.2	Test Requirements	1125

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	1125

A.4.2.11.1	Test Purpose and Environment	1125

A.4.2.11.2	Test Requirements	1129

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1129

A.4.2.12.1	Test Purpose and Environment	1129

A.4.2.12.2	Test Requirements	1131

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1132

A.4.2.13.1	Test Purpose and Environment	1132

A.4.2.13.2	Test Requirements	1133

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1134

A.4.2.14.1	Test Purpose and Environment	1134

A.4.2.14.2	Test Requirements	1135

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1136

A.4.2.15.1	Test Purpose and Environment	1136

A.4.2.15.2	Test Requirements	1137

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1138

A.4.2.16.1	Test Purpose and Environment	1138

A.4.2.16.2	Test Requirements	1139

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	1140

A.4.2.17.1	Test Purpose and Environment	1140

A.4.2.17.2	Test Requirements	1141

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1142

A.4.2.18.1	Test Purpose and Environment	1142

A.4.2.18.2	Test Requirements	1144

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1145

A.4.2.19.1	Test Purpose and Environment	1145

A.4.2.19.2	Test Requirements	1147

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1148

A.4.2.20.1	Test Purpose and Environment	1148

A.4.2.20.2	Test Requirements	1149

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1150

A.4.2.21.1	Test Purpose and Environment	1150

A.4.2.21.2	Test Requirements	1151

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1152

A.4.2.22.1	Test Purpose and Environment	1152

A.4.2.22.2	Test Requirements	1153

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1154

A.4.2.23.1	Test Purpose and Environment	1154

A.4.2.23.2	Test Requirements	1155

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1156

A.4.2.24.1	Test Purpose and Environment	1156

A.4.2.24.2	Test Requirements	1159

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1160

A.4.2.25.1	Test Purpose and Environment	1160

A.4.2.25.2	Test Requirements	1161

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1162

A.4.2.26.1	Test Purpose and Environment	1162

A.4.2.26.2	Test Requirements	1163

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1164

A.4.2.27.1	Test Purpose and Environment	1164

A.4.2.27.2	Test Requirements	1165

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1166

A.4.2.28.1	Test Purpose and Environment	1166

A.4.2.28.2	Test Requirements	1167

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1168

A.4.2.29.1	Test Purpose and Environment	1168

A.4.2.29.2	Test Requirements	1169

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1170

A.4.2.30.1	Test Purpose and Environment	1170

A.4.2.30.2	Test Requirements	1171

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1172

A.4.2.31.1	Test Purpose and Environment	1172

A.4.2.31.2	Test Requirements	1173

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1174

A.4.2.32.1	Test Purpose and Environment	1174

A.4.2.32.2	Test Requirements	1175

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1176

A.4.2.33.1	Test Purpose and Environment	1176

A.4.2.33.2	Test Requirements	1177

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1178

A.4.2.34.1	Test Purpose and Environment	1178

A.4.2.34.2	Test Requirements	1179

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1180

A.4.2.35.1	Test Purpose and Environment	1180

A.4.2.35.2	Test Requirements	1182

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1183

A.4.2.36.1	Test Purpose and Environment	1183

A.4.2.36.2	Test Requirements	1185

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1186

A.4.2.37.1	Test Purpose and Environment	1186

A.4.2.37.2	Test Requirements	1188

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1189

A.4.2.38.1	Test Purpose and Environment	1189

A.4.2.38.2	Test Requirements	1191

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1192

A.4.2.39.1	Test Purpose and Environment	1192

A.4.2.39.2	Test Requirements	1193

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1194

A.4.2.40.1	Test Purpose and Environment	1194

A.4.2.40.2	Test Requirements	1195

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1196

A.4.2.41.1	Test Purpose and Environment	1196

A.4.2.41.2	Test Requirements	1199

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1200

A.4.2.42.1	Test Purpose and Environment	1200

A.4.2.42.2	Test Requirements	1202

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1203

A.4.2.43.1	Test Purpose and Environment	1203

A.4.2.43.2	Test Requirements	1205

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1206

A.4.2.44.1	Test Purpose and Environment	1206

A.4.2.44.2	Test Requirements	1208

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1209

A.4.2.45.1	Test Purpose and Environment	1209

A.4.2.45.2	Test Requirements	1211

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1212

A.4.2.46.1	Test Purpose and Environment	1212

A.4.2.46.2	Test Requirements	1214

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1215

A.4.2.47.1	Test Purpose and Environment	1215

A.4.2.47.2	Test Requirements	1217

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1218

A.4.2.48.1	Test Purpose and Environment	1218

A.4.2.48.2	Test Requirements	1219

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1220

A.4.2.49.1	Test Purpose and Environment	1220

A.4.2.49.2	Test Requirements	1222

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1223

A.4.2.50.1	Test Purpose and Environment	1223

A.4.2.50.2	Test Requirements	1225

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1226

A.4.2.51.1	Test Purpose and Environment	1226

A.4.2.51.2	Test Requirements	1228

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1229

A.4.2.52.1	Test Purpose and Environment	1229

A.4.2.52.2	Test Requirements	1231

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1232

A.4.2.53.1	Test Purpose and Environment	1232

A.4.2.53.2	Test Requirements	1233

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1234

A.4.2.54.1	Test Purpose and Environment	1234

A.4.2.54.2	Test Requirements	1236

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1237

A.4.2.55.1	Test Purpose and Environment	1237

A.4.2.55.2	Test Requirements	1239

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1240

A.4.2.56.1	Test Purpose and Environment	1240

A.4.2.56.2	Test Requirements	1241

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1242

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1242

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1242

A.4.3.1.1.1	Test Purpose and Environment	1242

A.4.3.1.1.2	Test Requirements	1244

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1244

A.4.3.1.2.1	Test Purpose and Environment	1244

A.4.3.1.2.2	Test Requirements	1247

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1247

A.4.3.1.3.1	Test Purpose and Environment	1247

A.4.3.1.3.2	Test Requirements	1250

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1250

A.4.3.1.4.1	Test Purpose and Environment	1250

A.4.3.1.4.2	Test Requirements	1251

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1251

A.4.3.1.5.1	Test Purpose and Environment	1251

A.4.3.1.5.2	Test Requirements	1254

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1255

A.4.3.2.1	Test Purpose and Environment	1255

A.4.3.2.1.1	Void	1255

A.4.3.2.1.2	1.28Mcps TDD option	1255

A.4.3.2.1.3	Void	1257

A.4.3.2.2	Test Requirements	1257

A.4.3.2.2.1	1.28Mcps TDD option	1257

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1257

A.4.3.2A.1	Test Purpose and Environment	1257

A.4.3.2A.2	Test Requirements	1261

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1261

A.4.3.3.1	Test Purpose and Environment	1261

A.4.3.3.2	Test Requirements	1264

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1264

A.4.3.3A.1	Test Purpose and Environment	1264

A.4.3.3A.2	Test Requirements	1269

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1270

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1270

A.4.3.4.1.1	Test Purpose and Environment	1270

A.4.3.4.1.2	Test Requirements	1272

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1272

A.4.3.4.2.1	Test Purpose and Environment	1272

A.4.3.4.2.2	Test Requirements	1274

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1274

A.4.3.4.3.1	Test Purpose and Environment	1274

A.4.3.4.3.2	Test Requirements	1277

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1277

A.4.3.4.4.1	Test Purpose and Environment	1277

A.4.3.4.4.2	Test Requirements	1281

A.4.4	E-UTRAN to GSM Cell Re-Selection	1281

A.4.4.1	E-UTRAN FDD – GSM:	1281

A.4.4.1.1	Test Purpose and Environment	1281

A.4.4.1.2	Test Requirements	1283

A.4.4.2	E-UTRAN TDD – GSM:	1283

A.4.4.2.1	Test Purpose and Environment	1283

A.4.4.2.2	Test Requirements	1285

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1286

A.4.5.1	E-UTRAN FDD – HRPD	1286

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1286

A.4.5.1.1.1	Test Purpose and Environment	1286

A.4.5.1.1.2	Test Requirements	1288

A.4.5.2	E-UTRAN TDD – HRPD	1288

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1288

A.4.5.2.1.1	Test Purpose and Environment	1288

A.4.5.2.1.2	Test Requirements	1291

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1291

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1291

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1291

A.4.6.1.1.1	Test Purpose and Environment	1291

A.4.6.1.1.2	Test Requirements	1294

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1294

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1294

A.4.6.2.1.1	Test Purpose and Environment	1294

A.4.6.2.1.2	Test Requirements	1297

A.4.7	Idle State Positioning Measurement for UE category NB1	1297

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1297

A.4.7.1.1	Test Purpose and Environment	1297

A.4.7.1.2	Test Requirements	1302

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1303

A.4.7.2.1	Test Purpose and Environment	1303

A.4.7.4.2	Test Requirements	1307

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1308

A.4.7.3.1	Test Purpose and Environment	1308

A.4.7.3.2	Test Requirements	1312

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1313

A.4.7.4.1	Test Purpose and Environment	1313

A.4.7.4.2	Test Requirements	1317

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1318

A.5.1	E-UTRAN Handover	1318

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1318

A.5.1.1.1	Test Purpose and Environment	1318

A.5.1.1.2	Test Requirements	1320

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1320

A.5.1.2.1	Test Purpose and Environment	1320

A.5.1.2.2	Test Requirements	1322

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1322

A.5.1.3.1	Test Purpose and Environment	1322

A.5.1.3.2	Test Requirements	1324

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1324

A.5.1.4.1	Test Purpose and Environment	1324

A.5.1.4.2	Test Requirements	1326

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1326

A.5.1.5.1	Test Purpose and Environment	1326

A.5.1.5.2	Test Requirements	1328

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1328

A.5.1.6.1	Test Purpose and Environment	1328

A.5.1.6.2	Test Requirements	1330

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1330

A.5.1.7.1	Test Purpose and Environment	1330

A.5.1.7.2	Test Requirements	1333

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1333

A.5.1.8.1	Test Purpose and Environment	1333

A.5.1.8.2	Test Requirements	1336

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1336

A.5.1.9.1	Test Purpose and Environment	1336

A.5.1.9.2	Test Requirements	1337

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1337

A.5.1.10.1	Test Purpose and Environment	1337

A.5.1.10.2	Test Requirements	1339

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1339

A.5.1.11.1	Test Purpose and Environment	1339

A.5.1.11.2	Test Requirements	1341

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1341

A.5.1.12.1	Test Purpose and Environment	1341

A.5.1.12.2	Test Requirements	1343

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1343

A.5.1.13.1	Test Purpose and Environment	1343

A.5.1.13.2	Test Requirements	1345

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1346

A.5.1.14.1	Test Purpose and Environment	1346

A.5.1.14.2	Test Requirements	1347

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1348

A.5.1.15.1	Test Purpose and Environment	1348

A.5.1.15.2	Test Requirements	1349

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1350

A.5.1.16.1	Test Purpose and Environment	1350

A.5.1.16.2	Test Requirements	1351

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1352

A.5.1.17.1	Test Purpose and Environment	1352

A.5.1.17.2	Test Requirements	1353

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1354

A.5.1.18.1	Test Purpose and Environment	1354

A.5.1.18.2	Test Requirements	1355

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1356

A.5.1.19.1	Test Purpose and Environment	1356

A.5.1.19.2	Test Requirements	1357

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1358

A.5.1.20.1	Test Purpose and Environment	1358

A.5.1.20.2	Test Requirements	1359

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1360

A.5.1.21.1	Test Purpose and Environment	1360

A.5.1.21.2	Test Requirements	1361

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1361

A.5.1.22.1	Test Purpose and Environment	1361

A.5.1.22.2	Test Requirements	1363

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1363

A.5.1.23.1	Test Purpose and Environment	1363

A.5.1.23.2	Test Requirements	1365

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1365

A.5.1.24.1	Test Purpose and Environment	1365

A.5.1.24.2	Test Requirements	1367

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1367

A.5.1.25.1	Test Purpose and Environment	1367

A.5.1.25.2	Test Requirements	1369

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1370

A.5.1.26.1	Test Purpose and Environment	1370

A.5.1.26.2	Test Requirements	1371

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1372

A.5.1.27.1	Test Purpose and Environment	1372

A.5.1.27.2	Test Requirements	1373

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1374

A.5.1.28.1	Test Purpose and Environment	1374

A.5.1.28.2	Test Requirements	1375

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1376

A.5.1.29.1	Test Purpose and Environment	1376

A.5.1.29.2	Test Requirements	1377

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1378

A.5.1.30.1	Test Purpose and Environment	1378

A.5.1.30.2	Test Requirements	1379

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1380

A.5.1.31.1	Test Purpose and Environment	1380

A.5.1.31.2	Test Requirements	1381

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1382

A.5.1.32.1	Test Purpose and Environment	1382

A.5.1.32.2	Test Requirements	1383

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1384

A.5.1.33.1	Test Purpose and Environment	1384

A.5.1.13.2	Test Requirements	1385

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1386

A.5.1.34.1	Test Purpose and Environment	1386

A.5.1.34.2	Test Requirements	1387

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1388

A.5.1.35.1	Test Purpose and Environment	1388

A.5.1.35.2	Test Requirements	1389

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1390

A.5.1.36.1	Test Purpose and Environment	1390

A.5.1.36.2	Test Requirements	1391

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1392

A.5.1.37.1	Test Purpose and Environment	1392

A.5.1.37.2	Test Requirements	1393

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1394

A.5.1.38.1	Test Purpose and Environment	1394

A.5.1.38.2	Test Requirements	1395

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1396

A.5.1.39.1	Test Purpose and Environment	1396

A.5.1.39.2	Test Requirements	1397

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1398

A.5.1.40.1	Test Purpose and Environment	1398

A.5.1.40.2	Test Requirements	1399

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1400

A.5.1.41.1	Test Purpose and Environment	1400

A.5.1.41.2	Test Requirements	1402

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1403

A.5.1.42.1	Test Purpose and Environment	1403

A.5.1.42.2	Test Requirements	1404

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1405

A.5.1.43.1	Test Purpose and Environment	1405

A.5.1.43.2	Test Requirements	1406

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1407

A.5.1.44.1	Test Purpose and Environment	1407

A.5.1.44.2	Test Requirements	1408

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1409

A.5.1.45.1	Test Purpose and Environment	1409

A.5.1.45.1	Test Requirements	1410

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1411

A.5.1.46.1	Test Purpose and Environment	1411

A.5.1.46.2	Test Requirements	1412

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1413

A.5.1.47.1	Test Purpose and Environment	1413

A.5.1.47.2	Test Requirements	1414

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1415

A.5.1.48.1	Test Purpose and Environment	1415

A.5.1.48.2	Test Requirements	1416

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1417

A.5.1.49.1	Test Purpose and Environment	1417

A.5.1.49.2	Test Requirements	1418

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1419

A.5.1.50.1	Test Purpose and Environment	1419

A.5.1.50.2	Test Requirements	1420

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1421

A.5.1.51.1	Test Purpose and Environment	1421

A.5.1.51.2	Test Requirements	1423

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1423

A.5.152.1	Test Purpose and Environment	1423

A.5.1.52.2	Test Requirements	1426

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1427

A.5.1.53.1	Test Purpose and Environment	1427

A.5.1.53.2	Test Requirements	1429

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1430

A.5.1.54.1	Test Purpose and Environment	1430

A.5.1.54.2	Test Requirements	1432

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1433

A.5.1.55.1	Test Purpose and Environment	1433

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1437

A.5.1.56.1	Test Purpose and Environment	1437

A.5.1.56.2	Test Requirements	1440

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1441

A.5.1.57.1	Test Purpose and Environment	1441

A.5.1.57.2	Test Requirements	1443

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1443

A.5.1.58.1	Test Purpose and Environment	1443

A.5.1.58.2	Test Requirements	1446

A.5.2	E-UTRAN Handover to other RATs	1446

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1446

A.5.2.1.1	Test Purpose and Environment	1446

A.5.2.1.2	Test Requirements	1448

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1449

A.5.2.2.1	Test Purpose and Environment	1449

A.5.2.2.2	Test Requirements	1452

A.5.2.3 E-UTRAN FDD- GSM Handover	1452

A.5.2.3.1	Test Purpose and Environment	1452

A.5.2.3.2	Test Requirements	1453

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1454

A.5.2.4.1	Test Purpose and Environment	1454

A.5.2.4.1.1	Void	1454

A.5.2.4.1.2	1.28 Mcps TDD option	1454

A.5.2.4.1.3	Void	1456

A.5.2.4.2	Test Requirements	1456

A.5.2.4.2.1	Void	1456

A.5.2.4.2.2	1.28 Mcps TDD option	1456

A.5.2.4.2.3	Void	1456

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1456

A.5.2.5.1	Test Purpose and Environment	1456

A.5.2.5.1.3	Void	1459

A.5.2.5.2	Test Requirements	1459

A.5.2.5.2.1	Void	1459

A.5.2.5.2.2	1.28 Mcps TDD option	1459

A.5.2.5.2.3	Void	1459

A.5.2.6	E-UTRAN TDD - GSM Handover	1459

A.5.2.6.1	Test Purpose and Environment	1459

A.5.2.6.2	Test Requirements	1461

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1462

A.5.2.7.1	Test Purpose and Environment	1462

A.5.2.7.2	Test Requirements	1464

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1464

A.5.2.8.1	Test Purpose and Environment	1464

A.5.2.8.2	Test Requirements	1465

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1466

A.5.2.9.1	Test Purpose and Environment	1466

A.5.2.9.2	Test Requirements	1467

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1468

A.5.2.10.1	Test Purpose and Environment	1468

A.5.2.10.2	Test Requirements	1470

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1470

A.5.2.10A.1	Test Purpose and Environment	1470

A.5.2.10A.2	Test Requirements	1473

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1473

A.5.2.10B.1	Test Purpose and Environment	1473

A.5.2.10B.2	Test Requirements	1476

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1476

A.5.2.11.1	Test Purpose and Environment	1476

A.5.2.11.2	Test Requirements	1476

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1477

A.5.3.1	E-UTRAN FDD – HRPD Handover	1477

A.5.3.1.1	Test Purpose and Environment	1477

A.5.3.1.2	Test Requirements	1479

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1479

A.5.3.2.1	Test Purpose and Environment	1479

A.5.3.2.2	Test Requirements	1482

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1482

A.5.3.3.1	Test Purpose and Environment	1482

A.5.3.3.2	Test Requirements	1485

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1485

A.5.3.4.1	Test Purpose and Environment	1485

A.5.3.4.2	Test Requirements	1486

A.5.3.5	E-UTRAN TDD – HRPD Handover	1487

A.5.3.5.1	Test Purpose and Environment	1487

A.5.3.5.2	Test Requirements	1490

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1490

A.5.3.6.1	Test Purpose and Environment	1490

A.5.3.6.2	Test Requirements	1493

A.6	RRC Connection Control	1493

A.6.1	RRC Re-establishment	1493

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1493

A.6.1.1.1	Test Purpose and Environment	1493

A.6.1.1.2	Test Requirements	1494

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1495

A.6.1.2.1	Test Purpose and Environment	1495

A.6.1.2.2	Test Requirements	1496

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1497

A.6.1.3.1	Test Purpose and Environment	1497

A.6.1.3.2	Test Requirements	1498

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1499

A.6.1.4.1	Test Purpose and Environment	1499

A.6.1.4.2	Test Requirements	1500

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1501

A.6.1.5.1	Test Purpose and Environment	1501

A.6.1.5.2	Test Requirements	1501

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1501

A.6.1.6.1	Test Purpose and Environment	1501

A.6.1.6.2	Test Requirements	1503

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1504

A.6.1.7.1	Test Purpose and Environment	1504

A.6.1.7.2	Test Requirements	1505

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1506

A.6.1.8.1	Test Purpose and Environment	1506

A.6.1.8.2	Test Requirements	1507

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1508

A.6.1.9.1	Test Purpose and Environment	1508

A.6.1.9.2	Test Requirements	1509

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1510

A.6.1.10.1	Test Purpose and Environment	1510

A.6.1.10.2	Test Requirements	1511

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1512

A.6.1.11.1	Test Purpose and Environment	1512

A.6.1.11.2	Test Requirements	1513

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1514

A.6.1.12.1	Test Purpose and Environment	1514

A.6.1.12.2	Test Requirements	1515

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1516

A.6.1.13.1	Test Purpose and Environment	1516

A.6.1.13.2	Test Requirements	1517

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1518

A.6.1.14.1	Test Purpose and Environment	1518

A.6.1.14.2	Test Requirements	1519

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1520

A.6.1.15.1	Test Purpose and Environment	1520

A.6.1.15.2	Test Requirements	1522

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1523

A.6.1.16.1	Test Purpose and Environment	1523

A.6.1.16.2	Test Requirements	1525

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1526

A.6.1.17.1	Test Purpose and Environment	1526

A.6.1.17.2	Test Requirements	1527

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1528

A.6.1.18.1	Test Purpose and Environment	1528

A.6.1.18.2	Test Requirements	1529

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1530

A.6.1.19.1	Test Purpose and Environment	1530

A.6.1.19.2	Test Requirements	1531

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1532

A.6.1.20.1	Test Purpose and Environment	1532

A.6.1.20.2	Test Requirements	1533

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1534

A.6.1.21.1	Test Purpose and Environment	1534

A.6.1.21.2	Test Requirements	1535

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1536

A.6.1.22.1	Test Purpose and Environment	1536

A.6.1.22.2	Test Requirements	1537

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1538

A.6.1.23.1	Test Purpose and Environment	1538

A.6.1.23.2	Test Requirements	1540

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1541

A.6.1.24.1	Test Purpose and Environment	1541

A.6.1.24.2	Test Requirements	1543

A.6.2	Random Access	1544

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1544

A.6.2.1.1	Test Purpose and Environment	1544

A.6.2.1.2	Test Requirements	1545

A.6.2.1.2.1	Random Access Response Reception	1545

A.6.2.1.2.2	No Random Access Response Reception	1545

A.6.2.1.2.3	Receiving a NACK on msg3	1545

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1546

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1546

A.6.2.1.2.6	Contention Resolution Timer expiry	1546

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1546

A.6.2.2.1	Test Purpose and Environment	1546

A.6.2.2.2	Test Requirements	1547

A.6.2.2.2.1	Random Access Response Reception	1548

A.6.2.2.2.2	No Random Access Response Reception	1548

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1548

A.6.2.3.1	Test Purpose and Environment	1548

A.6.2.3.2	Test Requirements	1550

A.6.2.3.2.1	Random Access Response Reception	1550

A.6.2.3.2.2	No Random Access Response reception	1550

A.6.2.3.2.3	Receiving a NACK on msg3	1550

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1551

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1551

A.6.2.3.2.6	Contention Resolution Timer expiry	1551

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1551

A.6.2.4.1	Test Purpose and Environment	1551

A.6.2.4.2	Test Requirements	1553

A.6.2.4.2.1	Random Access Response Reception	1553

A.6.2.4.2.2	No Random Access Response Reception	1553

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1553

A.6.2.5.1	Test Purpose and Environment	1553

A.6.2.5.2	Test Requirements	1554

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1554

A.6.2.6.1	Test Purpose and Environment	1554

A.6.2.6.2	Test Requirements	1554

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1555

A.6.2.7.1	Test Purpose and Environment	1555

A.6.2.7.2	Test Requirements	1557

A.6.2.7.2.1	Random Access Response Reception	1557

A.6.2.7.2.2	No Random Access Response Reception	1557

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1557

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1558

A.6.2.8.1	Test Purpose and Environment	1558

A.6.2.8.2	Test Requirements	1560

A.6.2.8.2.1	Random Access Response Reception	1560

A.6.2.8.2.2	No Random Access Response Reception	1560

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1560

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1561

A.6.2.9.1	Test Purpose and Environment	1561

A.6.2.9.2	Test Requirements	1564

A.6.2.9.2.1	Random Access Response Reception	1564

A.6.2.9.2.2	No Random Access Response Reception	1565

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1565

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1566

A.6.2.10.1	Test Purpose and Environment	1566

A.6.2.10.2	Test Requirements	1568

A.6.2.10.2.1	Random Access Response Reception	1568

A.6.2.10.2.2	No Random Access Response Reception	1569

A.6.2.10.2.3	Receiving a NACK on msg3	1569

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1569

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1569

A.6.2.10.2.6	Contention Resolution Timer expiry	1569

A.6.2.10.2.7	PRACH Resource Selection	1570

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1570

A.6.2.11.1	Test Purpose and Environment	1570

A.6.2.11.2	Test Requirements	1572

A.6.2.11.2.1	Random Access Response Reception	1572

A.6.2.11.2.2	No Random Access Response Reception	1573

A.6.2.11.2.3	Receiving a NACK on msg3	1573

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1573

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1573

A.6.2.11.2.6	Contention Resolution Timer expiry	1573

A.6.2.11.2.7	PRACH Resource Selection	1574

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1574

A.6.2.12.1	Test Purpose and Environment	1574

A.6.2.12.2	Test Requirements	1576

A.6.2.12.2.1	Random Access Response Reception	1576

A.6.2.12.2.2	No Random Access Response Reception	1577

A.6.2.12.2.3	Receiving a NACK on msg3	1577

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1577

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1577

A.6.2.12.2.6	Contention Resolution Timer expiry	1577

A.6.2.12.2.7	PRACH Resource Selection	1578

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1578

A.6.2.13.1	Test Purpose and Environment	1578

A.6.2.13.2	Test Requirements	1580

A.6.2.13.2.1	Random Access Response Reception	1580

A.6.2.13.2.2	No Random Access Response Reception	1581

A.6.2.13.2.3	Receiving a NACK on msg3	1581

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1581

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1581

A.6.2.13.2.6	Contention Resolution Timer expiry	1581

A.6.2.13.2.7	PRACH Resource Selection	1582

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1582

A.6.2.14.1	Test Purpose and Environment	1582

A.6.2.14.2	Test Requirements	1584

A.6.2.14.2.1	Random Access Response Reception	1584

A.6.2.14.2.2	No Random Access Response Reception	1585

A.6.2.14.2.3	Receiving a NACK on msg3	1585

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1585

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1585

A.6.2.14.2.6	Contention Resolution Timer expiry	1585

A.6.2.14.2.7	PRACH Resource Selection	1586

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1586

A.6.2.15.1	Test Purpose and Environment	1586

A.6.2.15.2	Test Requirements	1588

A.6.2.15.2.1	Random Access Response Reception	1588

A.6.2.15.2.2	No Random Access Response Reception	1589

A.6.2.15.2.3	Receiving a NACK on msg3	1589

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1589

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1589

A.6.2.15.2.6	Contention Resolution Timer expiry	1589

A.6.2.15.2.7	PRACH Resource Selection	1590

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1590

A.6.2.16.1	Test Purpose and Environment	1590

A.6.2.16.2	Test Requirements	1593

A.6.2.16.2.1	Random Access Response Reception	1593

A.6.2.16.2.2	No Random Access Response Reception	1594

A.6.2.16.2.3	Receiving a NACK on msg3	1594

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1594

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1594

A.6.2.16.2.6	Contention Resolution Timer expiry	1594

A.6.2.16.2.7	NPRACH Resource Selection	1594

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1595

A.6.2.17.1	Test Purpose and Environment	1595

A.6.2.17.2	Test Requirements	1598

A.6.2.17.2.1	Random Access Response Reception	1598

A.6.2.17.2.2	No Random Access Response Reception	1599

A.6.2.17.2.3	Receiving a NACK on msg3	1599

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1599

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1599

A.6.2.17.2.6	Contention Resolution Timer expiry	1599

A.6.2.17.2.7	NPRACH Resource Selection	1599

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1600

A.6.2.18.1	Test Purpose and Environment	1600

A.6.2.18.2	Test Requirements	1603

A.6.2.18.2.1	Random Access Response Reception	1603

A.6.2.18.2.2	No Random Access Response Reception	1604

A.6.2.18.2.3	Receiving a NACK on msg3	1604

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1604

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1604

A.6.2.18.2.6	Contention Resolution Timer expiry	1604

A.6.2.18.2.7	NPRACH Resource Selection	1604

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1605

A.6.2.19.1	Test Purpose and Environment	1605

A.6.2.19.2	Test Requirements	1607

A.6.2.19.2.1	Random Access Response Reception	1607

A.6.2.19.2.2	No Random Access Response Reception	1608

A.6.2.19.2.3	Receiving a NACK on msg3	1608

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1608

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1608

A.6.2.19.2.6	Contention Resolution Timer expiry	1608

A.6.2.19.2.7	NPRACH Resource Selection	1608

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1609

A.6.2.20.1	Test Purpose and Environment	1609

A.6.2.20.2	Test Requirements	1611

A.6.2.20.2.1	Random Access Response Reception	1611

A.6.2.20.2.2	No Random Access Response Reception	1612

A.6.2.20.2.3	Receiving a NACK on msg3	1612

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1612

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1612

A.6.2.20.2.6	Contention Resolution Timer expiry	1612

A.6.2.20.2.7	NPRACH Resource Selection	1612

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1613

A.6.2.21.1	Test Purpose and Environment	1613

A.6.2.21.2	Test Requirements	1615

A.6.2.21.2.1	Random Access Response Reception	1615

A.6.2.21.2.2	No Random Access Response Reception	1616

A.6.2.21.2.3	Receiving a NACK on msg3	1616

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1616

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1616

A.6.2.21.2.6	Contention Resolution Timer expiry	1616

A.6.2.21.2.7	NPRACH Resource Selection	1616

A.6.3	RRC Connection Release with Redirection	1617

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1617

A.6.3.1.1	Test Purpose and Environment	1617

A.6.3.1.2	Test Requirements	1619

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1619

A.6.3.2.1	Test Purpose and Environment	1619

A.6.3.2.2	Test Requirements	1621

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1621

A.6.3.3.1	Test Purpose and Environment	1621

A.6.3.3.2	Test Requirements	1622

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1623

A.6.3.4.1	Test Purpose and Environment	1623

A.6.3.4.2	Test Requirements	1624

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1625

A.6.3.5.1	Test Purpose and Environment	1625

A.6.3.5.2	Test Requirements	1627

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1627

A.6.3.6.1	Test Purpose and Environment	1627

A. 6.3.6.2	Test Requirements	1630

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1630

A.6.3.7.1	Test Purpose and Environment	1630

A.6.3.7.2	Test Requirements	1633

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1633

A.6.3.8.1	Test Purpose and Environment	1633

A.6.3.8.2	Test Requirements	1636

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1636

A.6.3.9.1	Test Purpose and Environment	1636

A.6.3.9.2	Test Requirements	1638

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1638

A.6.3.10.1	Test Purpose and Environment	1638

A.6.3.10.2	Test Requirements	1640

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1640

A.6.3.11.1	Test Purpose and Environment	1640

A.6.3.11.2	Test Requirements	1642

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1642

A.6.3.12.1	Test Purpose and Environment	1642

A.6.3.12.2	Test Requirements	1645

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1645

A.6.3.13.1	Test Purpose and Environment	1645

A.6.3.13.2	Test Parameters	1645

A.6.3.13.3	Test Requirements	1649

A.7	Timing and Signalling Characteristics	1650

A.7.1	UE Transmit Timing	1650

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1650

A.7.1.1.1	Test Purpose and Environment	1650

A.7.1.1.2	Test Requirements	1651

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1652

A.7.1.2.1	Test Purpose and Environment	1652

A.7.1.2.2	Test Requirements	1654

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1655

A.7.1.3.1	Test Purpose and Environment	1655

A.7.1.3.2	Test Requirements	1657

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1658

A.7.1.4.1	Test Purpose and Environment	1658

A.7.1.4.2	Test Requirements	1659

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1660

A.7.1.4A.1	Test Purpose and Environment	1660

A.7.1.4A.2	Test Requirements	1660

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1660

A.7.1.5.1	Test Purpose and Environment	1660

A.7.1.5.2	Test Requirements	1661

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1661

A.7.1.6.1	Test Purpose and Environment	1661

A.7.1.6.2	Test Requirements	1663

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1664

A.7.1.7.1	Test Purpose and Environment	1664

A.7.1.7.2	Test Requirements	1665

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1666

A.7.1.7A.1	Test Purpose and Environment	1666

A.7.1.7A.2	Test Requirements	1666

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1666

A.7.1.7B.1	Test Purpose and Environment	1666

A.7.1.7B.2	Test Requirements	1666

A.7.1.8	Void	1667

A.7.1.8.1	Void	1667

A.7.1.8.2	Void	1667

A.7.1.9	Void	1667

A.7.1.9.1	Void	1667

A.7.1.9.2	Void	1667

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1667

A.7.1.10.1	Test Purpose and Environment	1667

A.7.1.10.2	Test Requirements	1669

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1669

A.7.1.11.1	Test Purpose and Environment	1669

A.7.1.11.2	Test Requirements	1671

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1671

A.7.1.12.1	Test Purpose and Environment	1671

A.7.1.12.2	Test Requirements	1673

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1673

A.7.1.13.1	Test Purpose and Environment	1673

A.7.1.13.2	Test Requirements	1675

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1676

A.7.1.14.1	Test Purpose and Environment	1676

A.7.1.14.2	Test Requirements	1677

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1677

A.7.1.15.1	Test Purpose and Environment	1677

A.7.1.15.2	Test Requirements	1678

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1679

A.7.1.16.1	Test Purpose and Environment	1679

A.7.1.16.2	Test Requirements	1680

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1681

A.7.1.17.1	Test Purpose and Environment	1681

A.7.1.17.2	Test Requirements	1683

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1684

A.7.1.18.1	Test Purpose and Environment	1684

A.7.1.18.2	Test Requirements	1686

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1687

A.7.1.19.1	Test Purpose and Environment	1687

A.7.1.19.2	Test Requirements	1688

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1688

A.7.1.20.1	Test Purpose and Environment	1688

A.7.1.20.2	Test Requirements	1690

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1690

A.7.1.21.1	Test Purpose and Environment	1690

A.7.1.21.2	Test Requirements	1692

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1692

A.7.1.22.1	Test Purpose and Environment	1692

A.7.1.22.2	Test Requirements	1694

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1694

A.7.1.23.1	Test Purpose and Environment	1694

A.7.1.23.2	Test Requirements	1696

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1696

A.7.1.24.1	Test Purpose and Environment	1696

A.7.1.24.2	Test Requirements	1697

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1698

A.7.1.25.1	Test Purpose and Environment	1698

A.7.1.25.2	Test Requirements	1699

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1700

A.7.1.26.1	Test Purpose and Environment	1700

A.7.1.26.2	Test Requirements	1701

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1702

A.7.1.27.1	Test Purpose and Environment	1702

A.7.1.27.2	Test Requirements	1704

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1704

A.7.1.28.1	Test Purpose and Environment	1704

A.7.1.28.2	Test Requirements	1706

A.7.2	UE Timing Advance	1707

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1707

A.7.2.1.1	Test Purpose and Environment	1707

A.7.2.1.2	Test Requirements	1708

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1709

A.7.2.2.1	Test Purpose and Environment	1709

A.7.2.2.2	Test Requirements	1711

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1711

A.7.2.3.1	Test Purpose and Environment	1711

A.7.2.3.2	Test Requirements	1711

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1711

A.7.2.4.1	Test Purpose and Environment	1711

A.7.2.4.2	Test Requirements	1714

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1714

A.7.2.5.1	Test Purpose and Environment	1714

A.7.2.5.2	Test Requirements	1716

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1716

A.7.2.5A.1	Test Purpose and Environment	1716

A.7.2.5A.2	Test Requirements	1717

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1717

A.7.2.5B.1	Test Purpose and Environment	1717

A.7.2.5B.2	Test Requirements	1717

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1717

A.7.2.6.1	Test Purpose and Environment	1717

A.7.2.6.2	Test Requirements	1720

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1720

A.7.2.7.1	Test Purpose and Environment	1720

A.7.2.7.2	Test Requirements	1722

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1722

A.7.2.8.1	Test Purpose and Environment	1722

A.7.2.8.2	Test Requirements	1724

A.7.2.9.2	Test Requirements	1725

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1726

A.7.2.10.1	Test Purpose and Environment	1726

A.7.2.10.2	Test Requirements	1727

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1727

A.7.2.11.1	Test Purpose and Environment	1727

A.7.2.11.2	Test Requirements	1729

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1729

A.7.2.12.1	Test Purpose and Environment	1729

A.7.2.12.2	Test Requirements	1731

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1732

A.7.2.13.1	Test Purpose and Environment	1732

A.7.2.13.2	Test Requirements	1733

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1734

A.7.2.14.1	Test Purpose and Environment	1734

A.7.2.14.2	Test Requirements	1736

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1736

A.7.2.15.1	Test Purpose and Environment	1736

A.7.2.15.2	Test Requirements	1738

A.7.3	Radio Link Monitoring	1738

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1738

A.7.3.1.1	Test Purpose and Environment	1738

A.7.3.1.2	Test Requirements	1742

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1742

A.7.3.2.1	Test Purpose and Environment	1742

A.7.3.2.2	Test Requirements	1746

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1746

A.7.3.3.1	Test Purpose and Environment	1746

A.7.3.3.2	Test Requirements	1750

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1750

A.7.3.4.1	Test Purpose and Environment	1750

A.7.3.4.2	Test Requirements	1754

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1754

A.7.3.5.1	Test Purpose and Environment	1754

A.7.3.5.2	Test Requirements	1757

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1758

A.7.3.6.1	Test Purpose and Environment	1758

A.7.3.6.2	Test Requirements	1761

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1761

A.7.3.7.1	Test Purpose and Environment	1761

A.7.3.7.2	Test Requirements	1764

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1764

A.7.3.8.1	Test Purpose and Environment	1764

A.7.3.8.2	Test Requirements	1767

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1767

A.7.3.9.2	Test Requirements	1770

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1770

A.7.3.10.1	Test Purpose and Environment	1770

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1774

A.7.3.11.1	Test Purpose and Environment	1774

A.7.3.11.2	Test Requirements	1779

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1779

A.7.3.12.1	Test Purpose and Environment	1779

A.7.3.12.2	Test Requirements	1784

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1784

A.7.3.13.1	Test Purpose and Environment	1784

A.7.3.13.2	Test Requirements	1787

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1787

A.7.3.14.1	Test Purpose and Environment	1787

A.7.3.14.2	Test Requirements	1790

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1790

A.7.3.15.1	Test Purpose and Environment	1790

A.7.3.15.2	Test Requirements	1794

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1794

A.7.3.16.1	Test Purpose and Environment	1794

A.7.3.16.2	Test Requirements	1799

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1799

A.7.3.17.1	Test Purpose and Environment	1799

A.7.3.17.2	Test Requirements	1803

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1803

A.7.3.18.1	Test Purpose and Environment	1803

A.7.3.18.2	Test Requirements	1807

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1807

A.7.3.19.1	Test Purpose and Environment	1807

A.7.3.19.2	Test Requirements	1812

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1812

A.7.3.20.1	Test Purpose and Environment	1812

A.7.3.20.2	Test Requirements	1817

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1817

A.7.3.21.1	Test Purpose and Environment	1817

A.7.3.21.2	Test Requirements	1822

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1822

A.7.3.22.1	Test Purpose and Environment	1822

A.7.3.22.2	Test Requirements	1827

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1827

A.7.3.23.1	Test Purpose and Environment	1827

A.7.3.23.2	Test Requirements	1828

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1828

A.7.3.24.1	Test Purpose and Environment	1828

A.7.3.24.2	Test Requirements	1829

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1829

A.7.3.25.1	Test Purpose and Environment	1829

A.7.3.25.2	Test Requirements	1830

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1830

A.7.3.26.1	Test Purpose and Environment	1830

A.7.3.26.2	Test Requirements	1833

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1833

A.7.3.27.1	Test Purpose and Environment	1833

A.7.3.27.2	Test Requirements	1836

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1836

A.7.3.28.1	Test Purpose and Environment	1836

A.7.3.28.2	Test Requirements	1839

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1839

A.7.3.29.1	Test Purpose and Environment	1839

A.7.3.29.2	Test Requirements	1842

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1842

A.7.3.30.1	Test Purpose and Environment	1842

A.7.3.30.2	Test Requirements	1845

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1845

A.7.3.31.1	Test Purpose and Environment	1845

A.7.3.31.2	Test Requirements	1848

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1848

A.7.3.32.1	Test Purpose and Environment	1848

A.7.3.32.2	Test Requirements	1851

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1851

A.7.3.33.1	Test Purpose and Environment	1851

A.7.3.33.2	Test Requirements	1854

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1854

A.7.3.34.1	Test Purpose and Environment	1854

A.7.3.34.2	Test Requirements	1857

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1857

A.7.3.35.1	Test Purpose and Environment	1857

A.7.3.35.2	Test Requirements	1860

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1860

A.7.3.36.1	Test Purpose and Environment	1860

A.7.3.36.2	Test Requirements	1863

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1863

A.7.3.37.1	Test Purpose and Environment	1863

A.7.3.37.2	Test Requirements	1866

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1866

A.7.3.38.1	Test Purpose and Environment	1866

A.7.3.38.2	Test Requirements	1869

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1869

A.7.3.39.1	Test Purpose and Environment	1869

A.7.3.39.2	Test Requirements	1872

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1873

A.7.3.40.1	Test Purpose and Environment	1873

A.7.3.40.2	Test Requirements	1875

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1876

A.7.3.41.1	Test Purpose and Environment	1876

A.7.3.41.2	Test Requirements	1879

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1879

A.7.3.42.1	Test Purpose and Environment	1879

A.7.3.42.2	Test Requirements	1882

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1882

A.7.3.43.1	Test Purpose and Environment	1882

A.7.3.43.2	Test Requirements	1885

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1885

A.7.3.44.1	Test Purpose and Environment	1885

A.7.3.44.2	Test Requirements	1888

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1889

A.7.3.45.1	Test Purpose and Environment	1889

A.7.3.45.2	Test Requirements	1891

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1892

A.7.3.46.1	Test Purpose and Environment	1892

A.7.3.46.2	Test Requirements	1895

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1895

A.7.3.47.1	Test Purpose and Environment	1895

A.7.3.47.2	Test Requirements	1898

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1898

A.7.3.48.1	Test Purpose and Environment	1898

A.7.3.48.2	Test Requirements	1901

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1901

A.7.3.49.1	Test Purpose and Environment	1901

A.7.3.49.2	Test Requirements	1904

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1904

A.7.3.50.1	Test Purpose and Environment	1904

A.7.3.50.2	Test Requirements	1907

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1907

A.7.3.51.1	Test Purpose and Environment	1907

A.7.3.51.2	Test Requirements	1910

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1910

A.7.3.52.1	Test Purpose and Environment	1910

A.7.3.52.2	Test Requirements	1913

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1913

A.7.3.53.1	Test Purpose and Environment	1913

A.7.3.53.2	Test Requirements	1916

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1916

A.7.3.54.1	Test Purpose and Environment	1916

A.7.3.54.2	Test Requirements	1919

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1919

A.7.3.55.1	Test Purpose and Environment	1919

A.7.3.55.2	Test Requirements	1922

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1922

A.7.3.56.1	Test Purpose and Environment	1922

A.7.3.56.2	Test Requirements	1925

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1925

A.7.3.57.1	Test Purpose and Environment	1925

A.7.3.57.2	Test Requirements	1928

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1928

A.7.3.58.1	Test Purpose and Environment	1928

A.7.3.58.2	Test Requirements	1931

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1931

A.7.3.59.1	Test Purpose and Environment	1931

A.7.3.59.2	Test Requirements	1934

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1934

A.7.3.60.1	Test Purpose and Environment	1934

A.7.3.60.2	Test Requirements	1938

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1938

A.7.3.61.1	Test Purpose and Environment	1938

A.7.3.61.2	Test Requirements	1942

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1942

A.7.3.62.1	Test Purpose and Environment	1942

A.7.3.62.2	Test Requirements	1946

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1946

A.7.3.63.1	Test Purpose and Environment	1946

A.7.3.63.2	Test Requirements	1950

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1950

A.7.3.64.1	Test Purpose and Environment	1950

A.7.3.64.2	Test Requirements	1953

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1954

A.7.3.65.1	Test Purpose and Environment	1954

A.7.3.65.2	Test Requirements	1957

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1958

A.7.3.66.1	Test Purpose and Environment	1958

A.7.3.66.2	Test Requirements	1960

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1961

A.7.3.67.1	Test Purpose and Environment	1961

A.7.3.67.2	Test Requirements	1964

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1965

A.7.3.68.1	Test Purpose and Environment	1965

A.7.3.68.2	Test Requirements	1967

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1967

A.7.3.69.1	Test Purpose and Environment	1967

A.7.3.69.2	Test Requirements	1969

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1969

A.7.3.70.1	Test Purpose and Environment	1969

A.7.3.70.2	Test Requirements	1971

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1971

A.7.3.71.1	Test Purpose and Environment	1971

A.7.3.71.2	Test Requirements	1974

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1974

A.7.3.72.1	Test Purpose and Environment	1974

A.7.3.72.2	Test Requirements	1976

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1976

A.7.3.73.1	Test Purpose and Environment	1976

A.7.3.73.2	Test Requirements	1978

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1978

A.7.3.74.1	Test Purpose and Environment	1978

A.7.3.74.2	Test Requirements	1981

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1981

A.7.3.75.1	Test Purpose and Environment	1981

A.7.3.75.2	Test Requirements	1983

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1984

A.7.3.76.1	Test Purpose and Environment	1984

A.7.3.76.2	Test Requirements	1987

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1987

A.7.3.77.1	Test Purpose and Environment	1987

A.7.3.77.2	Test Requirements	1990

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1990

A.7.3.78.1	Test Purpose and Environment	1990

A.7.3.78.2	Test Requirements	1993

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1993

A.7.3.79.1	Test Purpose and Environment	1993

A.7.3.79.2	Test Requirements	1996

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	1996

A.7.3.80.1	Test Purpose and Environment	1996

A.7.3.80.2	Test Requirements	1999

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	1999

A.7.3.81.1	Test Purpose and Environment	1999

A.7.3.81.2	Test Requirements	2002

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2002

A.7.3.82.1	Test Purpose and Environment	2002

A.7.3.82.2	Test Requirements	2005

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2005

A.7.3.83.1	Test Purpose and Environment	2005

A.7.3.83.2	Test Requirements	2006

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2007

A.7.3.84.1	Test Purpose and Environment	2007

A.7.3.84.2	Test Requirements	2009

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2009

A.7.3.85.1	Test Purpose and Environment	2009

A.7.3.85.2	Test Requirements	2010

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2011

A.7.3.86.1	Test Purpose and Environment	2011

A.7.3.86.2	Test Requirements	2013

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2013

A.7.3.87.1	Test Purpose and Environment	2013

A.7.3.87.2	Test Requirements	2015

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	2016

A.7.3.88.1	Test Purpose and Environment	2016

A.7.3.88.2	Test Requirements	2020

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	2020

A.7.3.89.1	Test Purpose and Environment	2020

A.7.3.89.2	Test Requirements	2024

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	2024

A.7.3.90.1	Test Purpose and Environment	2024

A.7.3.90.2	Test Requirements	2028

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2028

A.7.3.91.1	Test Purpose and Environment	2028

A.7.3.91.2	Test Requirements	2032

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	2032

A.7.3.92.1	Test Purpose and Environment	2032

A.7.3.92.2	Test Requirements	2035

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2036

A.7.3.93.1	Test Purpose and Environment	2036

A.7.3.93.2	Test Requirements	2039

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	2040

A.7.3.94.1	Test Purpose and Environment	2040

A.7.3.94.2	Test Requirements	2042

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	2043

A.7.3.95.1	Test Purpose and Environment	2043

A.7.3.95.2	Test Requirements	2046

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2047

A.7.3.96.1	Test Purpose and Environment	2047

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2048

A.7.3.97.1	Test Purpose and Environment	2048

A.7.3.97.2	Test Requirements	2051

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2051

A.7.3.98.1	Test Purpose and Environment	2051

A.7.3.98.2	Test Requirements	2054

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2054

A.7.3.99.1	Test Purpose and Environment	2054

A.7.3.99.2	Test Requirements	2057

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2057

A.7.3.100.1	Test Purpose and Environment	2057

A.7.3.100.2	Test Requirements	2059

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2059

A.7.3.101.1	Test Purpose and Environment	2059

A.7.3.101.2	Test Requirements	2061

A.7.4	Interruption for Dual Connectivity	2061

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2061

A.7.4.1.1	Test Purpose and Environment	2061

A.7.4.1.2	Test Requirements	2064

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2064

A.7.4.2.1	Test Purpose and Environment	2064

A.7.4.2.2	Test Requirements	2066

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	2066

A.7.4.3.1	Test Purpose and Environment	2066

A.7.4.3.2	Test Requirements	2068

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2068

A.7.4.4.1	Test Purpose and Environment	2068

A.7.4.4.2	Test Requirements	2071

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2071

A.7.4.5.1	Test Purpose and Environment	2071

A.7.4.5.2	Test Requirements	2073

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	2073

A.7.4.6.1	Test Purpose and Environment	2073

A.7.4.6.2	Test Requirements	2075

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	2075

A.7.4.7.1	Test Purpose and Environment	2075

A.7.4.7.2	Test Requirements	2079

A.7.5	Proximity-based Services	2079

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2079

A.7.5.1.1	Test Purpose and Environment	2079

A.7.5.1.2	Test Requirements	2080

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2081

A.7.5.2.1	Test Purpose and Environment	2081

A.7.5.1.2	Test Requirements	2081

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	2082

A.7.5.3.1	Test Purpose and Environment	2082

A.7.5.3.2	Test Requirements	2083

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2084

A.7.5.4.1	Test Purpose and Environment	2084

A.7.5.4.2	Test Requirements	2085

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	2086

A.7.5.5.1	Test Purpose and Environment	2086

A.7.5.5.2	Test Requirements	2088

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	2089

A.7.5.6.1	Test Purpose and Environment	2089

A.7.5.6.2	Test Requirements	2090

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	2091

A.7.5.7.1	Test Purpose and Environment	2091

A.7.5.7.2	Test Requirements	2093

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	2093

A.7.5.8.1	Test Purpose and Environment	2093

A.7.5.8.2	Test Requirements	2095

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	2096

A.7.5.9.1	Test Purpose and Environment	2096

A.7.5.9.2	Test Requirements	2098

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	2099

A.7.5.10.1	Test Purpose and Environment	2099

A.7.5.10.2	Test Requirements	2101

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	2102

A.7.5.11.1	Test Purpose and Environment	2102

A.7.5.11.2	Test Requirements	2104

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	2104

A.7.5.12.1	Test Purpose and Environment	2104

A.7.5.12.2	Test Requirements	2108

A.7.6	Interruption for carrier aggregation	2109

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	2109

A.7.6.1.1	Test Purpose and Environment	2109

A.7.6.1.2	Test Requirements	2112

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	2112

A.7.6.2.1	Test Purpose and Environment	2112

A.7.6.2.2	Test Requirements	2115

A.8	UE Measurements Procedures	2116

A.8.1	E-UTRAN FDD Intra-frequency Measurements	2116

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2116

A.8.1.1.1	Test Purpose and Environment	2116

A.8.1.1.2	Test Requirements	2117

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2118

A.8.1.2.1	Test Purpose and Environment	2118

A.8.1.2.2	Test Requirements	2119

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2120

A.8.1.3.1	Test Purpose and Environment	2120

A.8.1.3.2	Test Requirements	2122

A.8.1.4	Void	2122

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2122

A.8.1.5.1	Test Purpose and Environment	2122

A.8.1.5.2	Test Requirements	2124

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2124

A.8.1.6.1	Test Purpose and Environment	2124

A.8.1.6.2	Test Requirements	2126

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2126

A.8.1.7.1	Test Purpose and Environment	2126

A.8.1.7.2	Test Requirements	2128

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2129

A.8.1.8.1	Test Purpose and Environment	2129

A.8.1.8.2	Test Requirements	2131

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	2132

A.8.1.9.1	Test Purpose and Environment	2132

A.8.1.9.2	Test Requirements	2132

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	2132

A.8.1.10.1	Test Purpose and Environment	2132

A.8.1.10.2	Test Requirements	2133

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2133

A.8.1.11.1	Test Purpose and Environment	2133

A.8.1.11.2	Test Requirements	2135

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2136

A.8.1.12.1	Test Purpose and Environment	2136

A.8.1.12.2	Test Requirements	2137

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2138

A.8.1.13.1	Test Purpose and Environment	2138

A.8.1.13.2	Test Requirements	2140

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2140

A.8.1.14.1	Test Purpose and Environment	2140

A.8.1.14.2	Test Requirements	2142

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2143

A.8.1.15.1	Test Purpose and Environment	2143

A.8.1.15.2	Test Requirements	2144

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2145

A.8.1.16.1	Test Purpose and Environment	2145

A.8.1.16.2	Test Requirements	2147

A.8.1.17	Void	2147

A.8.1.18	Void	2147

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2147

A.8.1.19.1	Test Purpose and Environment	2147

A.8.1.19.2	Test Requirements	2149

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2150

A.8.1.20.1	Test Purpose and Environment	2150

A.8.1.20.2	Test Requirements	2152

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2152

A.8.1.21.1	Test Purpose and Environment	2152

A.8.1.21.2	Test Requirements	2154

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2155

A.8.1.22.1	Test Purpose and Environment	2155

A.8.1.22.2	Test Requirements	2157

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2157

A.8.1.23.1	Test Purpose and Environment	2157

A.8.1.23.2	Test Requirements	2158

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2159

A.8.1.24.1	Test Purpose and Environment	2159

A.8.1.24.2	Test Requirements	2161

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2161

A.8.1.25.1	Test Purpose and Environment	2161

A.8.1.25.2	Test Requirements	2163

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2163

A.8.1.26.1	Test Purpose and Environment	2163

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2166

A.8.1.27.1	Test Purpose and Environment	2166

A.8.1.27.2	Test Requirements	2168

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2168

A.8.1.28.1	Test Purpose and Environment	2168

A.8.1.28.2	Test Requirements	2170

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2170

A.8.1.29.1	Test Purpose and Environment	2170

A.8.1.29.2	Test Requirements	2172

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2173

A.8.1.30.1	Test Purpose and Environment	2173

A.8.1.30.2	Test Requirements	2175

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2175

A.8.1.31.1	Test Purpose and Environment	2175

A.8.1.31.2	Test Requirements	2177

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2178

A.8.1.32.1	Test Purpose and Environment	2178

A.8.1.32.2	Test Requirements	2179

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2180

A.8.1.33.1	Test Purpose and Environment	2180

A.8.1.33.2	Test Requirements	2181

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2182

A.8.1.34.1	Test Purpose and Environment	2182

A.8.1.34.2	Test Requirements	2183

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2184

A.8.1.35.1	Test Purpose and Environment	2184

A.8.1.35.2	Test Requirements	2185

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2186

A.8.1.36.1	Test Purpose and Environment	2186

A.8.1.36.2	Test Requirements	2187

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2188

A.8.1.37.1	Test Purpose and Environment	2188

A.8.1.37.2	Test Requirements	2190

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2190

A.8.1.38.1	Test Purpose and Environment	2190

A.8.1.38.2	Test Requirements	2191

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2192

A.8.1.39.1	Test Purpose and Environment	2192

A.8.1.39.2	Test Requirements	2194

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2194

A.8.1.40.1	Test Purpose and Environment	2194

A.8.1.40.2	Test Requirements	2196

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2196

A.8.1.41.1	Test Purpose and Environment	2196

A.8.1.41.2	Test Requirement	2198

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2198

A.8.1.42.1	Test Purpose and Environment	2198

A.8.1.42.2	Test Requirement	2199

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2199

A.8.1.43.1	Test Purpose and Environment	2199

A.8.1.43.2	Test Requirements	2202

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2202

A.8.1.44.1	Test Purpose and Environment	2202

A.8.1.44.2	Test Requirements	2205

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2206

A.8.1.45.1	Test Purpose and Environment	2206

A.8.1.45.2	Test Requirements	2208

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2209

A.8.1.46.1	Test Purpose and Environment	2209

A.8.1.46.2	Test Requirements	2210

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2211

A.8.1.47.1	Test Purpose and Environment	2211

A.8.1.47.2	Test Requirements	2213

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2214

A.8.1.48.1	Test Purpose and Environment	2214

A.8.1.48.2	Test Requirements	2216

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2217

A.8.1.49.1	Test Purpose and Environment	2217

A.8.1.49.2	Test Requirements	2218

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2219

A.8.1.50.1	Test Purpose and Environment	2219

A.8.1.50.2	Test Requirements	2221

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2222

A.8.1.51.1	Test Purpose and Environment	2222

A.8.1.51.2	Test Requirements	2224

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2225

A.8.1.52.1	Test Purpose and Environment	2225

A.8.1.52.2	Test Requirements	2226

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2227

A.8.1.53.1	Test Purpose and Environment	2227

A.8.1.53.2	Test Requirements	2229

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2230

A.8.1.54.1	Test Purpose and Environment	2230

A.8.1.54.2	Test Requirements	2232

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2233

A.8.1.55.1	Test Purpose and Environment	2233

A.8.1.55.2	Test Requirements	2234

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2235

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2235

A.8.2.1.1	Test Purpose and Environment	2235

A.8.2.1.2	Test Requirements	2236

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2236

A.8.2.2.1	Test Purpose and Environment	2236

A.8.2.2.2	Test Requirements	2239

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2239

A.8.2.3.1	Test Purpose and Environment	2239

A.8.2.3.2	Test Requirements	2241

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2242

A.8.2.4.1	Test Purpose and Environment	2242

A.8.2.4.2	Test Requirements	2244

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2244

A.8.2.5.1	Test Purpose and Environment	2244

A.8.2.5.2	Test Requirements	2246

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2247

A.8.2.6.1	Test Purpose and Environment	2247

A.8.2.6.2	Test Requirements	2250

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2251

A.8.2.7.1	Test Purpose and Environment	2251

A.8.2.7.2	Test Requirements	2252

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2253

A.8.2.8.1	Test Purpose and Environment	2253

A.8.2.8.2	Test Requirements	2255

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2255

A.8.2.9.1	Test Purpose and Environment	2255

A.8.2.9.2	Test Requirements	2257

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2258

A.8.2.10.1	Test Purpose and Environment	2258

A.8.2.10.2	Test Requirements	2260

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2260

A.8.2.11.1	Test Purpose and Environment	2260

A.8.2.11.2	Test Requirements	2262

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2262

A.8.2.12.1	Test Purpose and Environment	2262

A.8.2.12.2	Test Requirements	2264

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2265

A.8.2.13.1	Test Purpose and Environment	2265

A.8.2.13.2	Test Requirements	2267

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2267

A.8.2.14.1	Test Purpose and Environment	2267

A.8.2.14.2	Test Requirement	2269

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2269

A.8.2.15.1	Test Purpose and Environment	2269

A.8.2.15.2	Test Requirements	2271

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2271

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2271

A.8.3.1.1	Test Purpose and Environment	2271

A.8.3.1.2	Test Requirements	2273

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2273

A.8.3.2.1	Test Purpose and Environment	2273

A.8.3.2.2	Test Requirements	2276

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2276

A.8.3.3.1	Test Purpose and Environment	2276

A.8.3.3.2	Test Requirements	2279

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2279

A.8.3.4.1	Test Purpose and Environment	2279

A.8.3.4.2	Test Requirements	2281

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2281

A.8.3.5.2	Test Requirements	2283

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2283

A.8.3.6.1	Test Purpose and Environment	2283

A.8.3.6.2	Test Requirements	2285

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2286

A.8.3.7.1	Test Purpose and Environment	2286

A.8.3.7.2	Test Requirements	2288

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2289

A.8.3.8.1	Test Purpose and Environment	2289

A.8.3.8.2	Test Requirements	2292

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2292

A.8.3.9.1	Test Purpose and Environment	2292

A.8.3.9.2	Test Requirements	2296

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2297

A.8.3.10.1	Test Purpose and Environment	2297

A.8.3.10.2	Test Requirements	2298

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2298

A.8.3.11.1	Test Purpose and Environment	2298

A.8.3.11.2	Test Requirement	2300

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2301

A.8.3.12.1	Test Purpose and Environment	2301

A.8.3.12.2	Test Requirement	2302

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2303

A.8.3.13.1	Test Purpose and Environment	2303

A.8.3.13.2	Test Requirement	2304

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2305

A.8.3.14.1	Test Purpose and Environment	2305

A.8.3.14.2	Test Requirement	2306

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2307

A.8.3.15.1	Test Purpose and Environment	2307

A.8.3.15.2	Test Requirement	2308

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2309

A.8.3.16.1	Test Purpose and Environment	2309

A.8.3.16.2	Test Requirement	2311

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2311

A.8.3.17.1	Test Purpose and Environment	2311

A.8.3.17.2	Test Requirement	2314

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2314

A.8.3.18.1	Test Purpose and Environment	2314

A.8.3.18.2	Test Requirement	2317

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2317

A.8.3.19.1	Test Purpose and Environment	2317

A.8.3.19.2	Test Requirement	2320

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2320

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2320

A.8.4.1.1	Test Purpose and Environment	2320

A.8.4.1.2	Test Requirements	2322

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2322

A.8.4.2.1	Test Purpose and Environment	2322

A.8.4.2.2	Test Requirements	2325

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2325

A.8.4.3.1	Test Purpose and Environment	2325

A.8.4.3.2	Test Requirements	2327

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2328

A.8.4.4.1	Test Purpose and Environment	2328

A.8.4.4.2	Test Requirements	2330

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2331

A.8.4.5.1	Test Purpose and Environment	2331

A.8.4.5.2	Test Requirements	2333

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2333

A.8.4.6.1	Test Purpose and Environment	2333

A.8.4.6.2	Test Requirements	2334

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2334

A.8.4.7.1	Test Purpose and Environment	2334

A.8.4.7.2	Test Requirements	2338

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2338

A.8.4.8.1	Test Purpose and Environment	2338

A.8.4.8.2	Test Requirements	2341

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2341

A.8.4.9.1	Test Purpose and Environment	2341

A.8.4.9.2	Test Requirements	2346

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2346

A.8.4.10.1	Test Purpose and Environment	2346

A.8.4.10.2	Test Requirements	2348

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2348

A.8.4.11.1	Test Purpose and Environment	2348

A.8.4.11.2	Test Requirement	2350

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2350

A.8.4.12.1	Test Purpose and Environment	2350

A.8.4.12.2	Test Requirement	2352

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2353

A.8.4.13.1	Test Purpose and Environment	2353

A.8.4.13.2	Test Requirement	2354

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2355

A.8.4.14.1	Test Purpose and Environment	2355

A.8.4.14.2	Test Requirement	2357

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2357

A.8.4.15.1	Test Purpose and Environment	2357

A.8.4.15.2	Test Requirement	2360

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2360

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2360

A.8.5.1.1	Test Purpose and Environment	2360

A.8.5.1.2	Test Requirements	2362

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2362

A.8.5.2.1	Test Purpose and Environment	2362

A.8.5.2.2	Test Requirements	2364

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2364

A.8.5.3.1	Test Purpose and Environment	2364

A.8.5.3.2	Test Requirements	2367

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2367

A.8.5.4.1	Test Purpose and Environment	2367

A.8.5.4.2	Test Requirements	2369

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2369

A.8.5.5.1	Test Purpose and Environment	2369

A.8.5.5.2	Test Requirements	2372

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2372

A.8.5.6.1	Test Purpose and Environment	2372

A.8.5.6.2	Test Requirements	2373

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2374

A.8.5.7.1	Test Purpose and Environment	2374

A.8.5.7.2	Test Requirements	2374

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2374

A.8.5.8.1	Test Purpose and Environment	2374

A.8.5.8.2	Test Requirements	2377

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2377

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2377

A.8.6.1.1	Test Purpose and Environment	2377

A.8.6.1.2	Test Requirements	2379

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2379

A.8.6.2.1	Test Purpose and Environment	2379

A.8.6.2.2	Test Requirements	2382

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2382

A.8.6.3.1	Test Purpose and Environment	2382

A.8.6.3.2	Test Requirements	2385

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2385

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2385

A.8.7.1.1	Test Purpose and Environment	2385

A.8.7.1.1.1	Void	2385

A.8.7.1.1.2	1.28 Mcps TDD option	2385

A.8.7.1.1.3	Void	2387

A.8.7.1.2	Test Requirements	2387

A.8.7.1.2.1	Void	2387

A.8.7.1.2.2	1.28 Mcps TDD option	2387

A.8.7.1.2.3	Void	2387

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2387

A.8.7.2.1	Test Purpose and Environment	2387

A.8.7.2.2	Test Requirements	2390

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2391

A.8.7.3.1	Test Purpose and Environment	2391

A.8.7.3.2	Test Parameters	2391

A.8.7.3.3	Test Requirements	2392

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2393

A.8.7.4.1	Test Purpose and Environment	2393

A.8.7.4.2	Test Requirements	2395

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2395

A.8.7.5.1	Test Purpose and Environment	2395

A.8.7.5.2	Test Requirements	2397

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2397

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2397

A.8.7A.1.1	Test Purpose and Environment	2397

A.8.7A.1.2	Test Requirements	2400

A.8.8	E-UTRAN FDD – GSM Measurements	2400

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2400

A.8.8.1.1	Test Purpose and Environment	2400

A.8.8.1.2	Test Requirements	2402

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2402

A.8.8.2.1	Test Purpose and Environment	2402

A.8.8.2.2	Test Requirements	2404

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2405

A.8.8.3.1	Test Purpose and Environment	2405

A.8.8.3.2	Test Requirements	2406

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2407

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2407

A.8.9.1.1	Test Purpose and Environment	2407

A.8.9.1.2	Test Requirements	2408

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2409

A.8.9.2.1	Test Purpose and Environment	2409

A.8.9.2.2	Test Requirements	2411

A.8.10	E-UTRAN TDD – GSM Measurements	2411

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2411

A.8.10.1.1	Test Purpose and Environment	2411

A.8.10.1.2	Test Requirements	2412

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2413

A.8.10.2.1	Test Purpose and Environment	2413

A.8.10.2.2	Test Requirements	2415

A.8.11	Monitoring of Multiple Layers	2415

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2415

A.8.11.1.1	Test Purpose and Environment	2415

A.8.11.1.2	Test Requirements	2417

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2418

A.8.11.2.1	Test Purpose and Environment	2418

A.8.11.2.2	Test Requirements	2419

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2420

A.8.11.3.1	Test Purpose and Environment	2420

A.8.11.3.2	Test Requirements	2422

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2422

A.8.11.4.1	Test Purpose and Environment	2422

A.8.11.4.2	Test Requirements	2425

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2425

A.8.11.5.1	Test Purpose and Environment	2425

A.8.11.5.2	Test Requirements	2427

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2428

A.8.11.6.1	Test Purpose and Environment	2428

A.8.11.6.2	Test Requirements	2430

A.8.12	RSTD Intra-frequency Measurements	2431

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2431

A.8.12.1.1	Test Purpose and Environment	2431

A.8.12.1.2	Test Requirements	2435

A.8.12.1.2A	Test Requirements for UE Category 1bis	2435

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2435

A.8.12.2.1	Test Purpose and Environment	2435

A.8.12.2.2	Test Requirements	2440

A.8.12.2.2A	Test Requirements for UE Category 1bis	2440

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2440

A.8.12.3.1	Test Purpose and Environment	2440

A.8.12.3.2	Test Requirements	2446

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2446

A.8.12.4.1	Test Purpose and Environment	2446

A.8.12.4.2	Test Requirements	2451

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2451

A.8.12.5.1	Test Purpose and Environment	2451

A.8.12.5.2	Test Requirements	2456

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2456

A.8.12.6.1	Test Purpose and Environment	2456

A.8.12.6.2	Test Requirements	2461

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2461

A.8.12.7.1	Test Purpose and Environment	2461

A.8.12.7.2	Test Requirements	2466

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2466

A.8.12.8.1	Test Purpose and Environment	2466

A.8.12.8.2	Test Requirements	2471

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2471

A.8.12.9.1	Test Purpose and Environment	2471

A.8.12.9.2	Test Requirements	2476

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2476

A.8.12.10.1	Test Purpose and Environment	2476

A.8.12.10.2	Test Requirements	2481

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2481

A.8.12.11.1	Test Purpose and Environment	2481

A.8.12.11.2	Test Requirements	2486

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2486

A.8.12.12.1	Test Purpose and Environment	2486

A.8.12.12.2	Test Requirements	2491

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2491

A.8.12.13.1	Test Purpose and Environment	2491

A.8.12.13.2	Test Requirements	2496

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2496

A.8.12.14.1	Test Purpose and Environment	2496

A.8.12.14.2	Test Requirements	2501

A.8.13	RSTD Inter-frequency Measurements	2501

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2501

A.8.13.1.1	Test Purpose and Environment	2501

A.8.13.1.2	Test Requirements	2506

A.8.13.1.2A	Test Requirements for UE Category 1bis	2506

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2506

A.8.13.2.1	Test Purpose and Environment	2506

A.8.13.2.2	Test Requirements	2512

A.8.13.2.2A	Test Requirements for UE Category 1bis	2512

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2512

A.8.13.3.1	Test Purpose and Environment	2512

A.8.13.3.2	Test Requirements	2519

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2519

A.8.13.4.1	Test Purpose and Environment	2519

A.8.13.4.2	Test Requirements	2525

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2525

A.8.13.5.1	Test Purpose and Environment	2525

A.8.13.5.2	Test Requirements	2531

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2531

A.8.13.6.1	Test Purpose and Environment	2531

A.8.13.6.2	Test Requirements	2537

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2537

A.8.13.7.1	Test Purpose and Environment	2537

A.8.13.7.2	Test Requirements	2543

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2543

A.8.13.8.1	Test Purpose and Environment	2543

A.8.13.8.2	Test Requirements	2549

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2549

A.8.13.9.1	Test Purpose and Environment	2549

A.8.13.9.2	Test Requirements	2555

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2555

A.8.13.10.1	Test Purpose and Environment	2555

A.8.13.10.2	Test Requirements	2561

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2561

A.8.13.11.1	Test Purpose and Environment	2561

A.8.13.11.2	Test Requirements	2567

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2567

A.8.13.12.1	Test Purpose and Environment	2567

A.8.13.12.2	Test Requirements	2573

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2573

A.8.13.13.1	Test Purpose and Environment	2573

A.8.13.13.2	Test Requirements	2579

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2579

A.8.13.14.1	Test Purpose and Environment	2579

A.8.13.14.2	Test Requirements	2585

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2585

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2585

A.8.14.1.1	Test Purpose and Environment	2585

A.8.14.1.2	Test Requirements	2587

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2587

A.8.14.2.1	Test Purpose and Environment	2587

A.8.14.2.2	Test Requirements	2590

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2590

A.8.14.3.1	Test Purpose and Environment	2590

A.8.14.3.2	Test Requirements	2592

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2593

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2593

A.8.15.1.1	Test Purpose and Environment	2593

A.8.15.1.2	Test Requirements	2594

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2594

A.8.15.2.1	Test Purpose and Environment	2594

A.8.15.2.2	Test Requirements	2597

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2597

A.8.15.3.1	Test Purpose and Environment	2597

A.8.15.3.2	Test Requirements	2599

A.8.16	E-UTRAN Carrier Aggregation Measurements	2600

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2600

A.8.16.1.1	Test Purpose and Environment	2600

A.8.16.1.2	Test Requirements	2602

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2602

A.8.16.2.1	Test Purpose and Environment	2602

A.8.16.2.2	Test Requirements	2604

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2604

A.8.16.3.1	Test Purpose and Environment	2604

A.8.16.3.2	Test Requirements	2606

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2607

A.8.16.3A.1	Test Purpose and Environment	2607

A.8.16.3A.2	Test Requirements	2608

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2609

A.8.16.4.1	Test Purpose and Environment	2609

A.8.16.4.2	Test Requirements	2610

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2611

A.8.16.4A.1	Test Purpose and Environment	2611

A.8.16.4A.2	Test Requirements	2613

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2614

A.8.16.5.1	Test Purpose and Environment	2614

A.8.16.5.2	Test Requirements	2614

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2614

A.8.16.6.1	Test Purpose and Environment	2614

A.8.16.6.2	Test Requirements	2615

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2615

A.8.16.7.1	 Test Purpose and Environment	2615

A.8.16.7.2	 Test Requirements	2616

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2616

A.8.16.8.1	 Test Purpose and Environment	2616

A.8.16.8.2	 Test Requirements	2617

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2617

A.8.16.9.1	Test Purpose and Environment	2617

A.8.16.9.2	Test Requirements	2618

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2618

A.8.16.10.1	Test Purpose and Environment	2618

A.8.16.10.2	Test Requirements	2619

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2619

A.8.16.11.1	 Test Purpose and Environment	2619

A.8.16.11.2	 Test Requirements	2620

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2620

A.8.16.12.1	 Test Purpose and Environment	2620

A.8.16.12.2	 Test Requirements	2620

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2620

A.8.16.13.1	Test Purpose and Environment	2620

A.8.16.13.2	Test Requirements	2621

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2621

A.8.16.14.1	Test Purpose and Environment	2621

A.8.16.14.2	Test Requirements	2622

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2622

A.8.16.15.1	 Test Purpose and Environment	2622

A.8.16.7.2	 Test Requirements	2622

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2622

A.8.16.16.1	 Test Purpose and Environment	2622

A.8.16.16.2	 Test Requirements	2623

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2623

A.8.16.17.1	Test Purpose and Environment	2623

A.8.16.17.2	Test Requirements	2625

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2626

A.8.16.17A.1	Test Purpose and Environment	2626

A.8.16.17A.2	Test Requirements	2626

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2626

A.8.16.17B.1	Test Purpose and Environment	2626

A.8.16.17B.2	Test Requirements	2627

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2627

A.8.16.17C.1	Test Purpose and Environment	2627

A.8.16.17C.2	Test Requirements	2627

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2627

A.8.16.18.1	Test Purpose and Environment	2627

A.8.16.18.2	Test Requirements	2629

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2630

A.8.16.18A.1	Test Purpose and Environment	2630

A.8.16.18A.2	Test Requirements	2630

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2630

A.8.16.18B.1	Test Purpose and Environment	2630

A.8.16.18B.2	Test Requirements	2631

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2631

A.8.16.18C.1	Test Purpose and Environment	2631

A.8.16.18C.2	Test Requirements	2631

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2631

A.8.16.18D.1	Test Purpose and Environment	2631

A.8.16.18D.2	Test Requirements	2632

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2632

A.8.16.19.1	Test Purpose and Environment	2632

A.8.16.19.2	Test Requirements	2634

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2634

A.8.16.19A.1	Test Purpose and Environment	2634

A.8.16.19A.2	Test Requirements	2635

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2635

A.8.16.19B.1	Test Purpose and Environment	2635

A.8.16.19B.2	Test Requirements	2635

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2635

A.8.16.19C.1	Test Purpose and Environment	2635

A.8.16.19C.2	Test Requirements	2636

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2636

A.8.16.20.1	Test Purpose and Environment	2636

A.8.16.20.2	Test Requirements	2638

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2639

A.8.16.20A.1	Test Purpose and Environment	2639

A.8.16.20A.2	Test Requirements	2639

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2639

A.8.16.20B.1	Test Purpose and Environment	2639

A.8.16.20B.2	Test Requirements	2640

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2640

A.8.16.20C.1	Test Purpose and Environment	2640

A.8.16.20C.2	Test Requirements	2640

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2640

A.8.16.20D.1	Test Purpose and Environment	2640

A.8.16.20D.2	Test Requirements	2641

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2641

A.8.16.21.1	Test Purpose and Environment	2641

A.8.16.21.2	Test Requirements	2643

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2643

A.8.16.22.1	Test Purpose and Environment	2643

A.8.16.22.2	Test Requirements	2645

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2645

A.8.16.23.1	Test Purpose and Environment	2645

A.8.16.23.2	Test Requirements	2648

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2648

A.8.16.24.1	Test Purpose and Environment	2648

A.8.16.24.2	Test Requirements	2651

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2651

A.8.16.25.1	Test Purpose and Environment	2651

A.8.16.25.2	Test Requirements	2654

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2654

A.8.16.26.1	Test Purpose and Environment	2654

A.8.16.26.2	Test Requirements	2657

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2657

A.8.16.27.1	Test Purpose and Environment	2657

A.8.16.27.2	Test Requirements	2662

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2662

A.8.16.28.1	Test Purpose and Environment	2662

A.8.16.28.2	Test Requirements	2667

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2667

A.8.16.29.1	Test Purpose and Environment	2667

A.8.16.29.2	Test Requirements	2672

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2672

A.8.16.30.1	Test Purpose and Environment	2672

A.8.16.30.2	Test Requirements	2677

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2677

A.8.16.31.1	Test Purpose and Environment	2677

A.8.16.31.2	Test Requirements	2682

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2682

A.8.16.32.1	Test Purpose and Environment	2682

A.8.16.32.2	Test Requirements	2687

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2687

A.8.16.33.1	Test Purpose and Environment	2687

A.8.16.33.2	Test Requirements	2692

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2692

A.8.16.34.1	Test Purpose and Environment	2692

A.8.16.34.2	Test Requirements	2697

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2697

A.8.16.35.1	Test Purpose and Environment	2697

A.8.16.35.2	Test Requirements	2699

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2700

A.8.16.36.1	Test Purpose and Environment	2700

A.8.16.36.2	Test Requirements	2702

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2703

A.8.16.37.1	Test Purpose and Environment	2703

A.8.16.37.2	Test Requirements	2705

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2706

A.8.16.38.1	Test Purpose and Environment	2706

A.8.16.38.2	Test Requirements	2708

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2709

A.8.16.39.1	Test Purpose and Environment	2709

A.8.16.39.2	Test Requirements	2712

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2712

A.8.16.40.1	Test Purpose and Environment	2712

A.8.16.40.2	Test Requirements	2715

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2715

A.8.16.41.1	Test Purpose and Environment	2715

A.8.16.41.2	Test Requirements	2717

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2718

A.8.16.42.1	Test Purpose and Environment	2718

A.8.16.42.2	Test Requirements	2720

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2721

A.8.16.43.1	Test Purpose and Environment	2721

A.8.16.43.2	Test Requirements	2724

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2724

A.8.16.44.1	Test Purpose and Environment	2724

A.8.16.44.2	Test Requirements	2727

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2727

A.8.16.45.1	Test Purpose and Environment	2727

A.8.16.45.2	Test Requirements	2730

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2730

A.8.16.46.1	Test Purpose and Environment	2730

A.8.16.46.2	Test Requirements	2732

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2733

A.8.16.47.1	Test Purpose and Environment	2733

A.8.16.47.2	Test Requirements	2735

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2736

A.8.16.48.1	Test Purpose and Environment	2736

A.8.16.48.2	Test Requirements	2738

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2739

A.8.16.49.1	Test Purpose and Environment	2739

A.8.16.49.2	Test Requirements	2742

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2742

A.8.16.50.1	Test Purpose and Environment	2742

A.8.16.50.2	Test Requirements	2744

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2745

A.8.16.51.1	Test Purpose and Environment	2745

A.8.16.51.2	Test Requirements	2750

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2750

A.8.16.52.1	Test Purpose and Environment	2750

A.8.16.52.2	Test Requirements	2755

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2755

A.8.16.53.1	Test Purpose and Environment	2755

A.8.16.53.2	Test Requirements	2759

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2759

A.8.16.54.1	Test Purpose and Environment	2759

A.8.16.54.2	Test Requirements	2763

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2763

A.8.16.55.1	Test Purpose and Environment	2763

A.8.16.55.2	Test Requirements	2769

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2769

A.8.16.56.1	Test Purpose and Environment	2769

A.8.16.56.2	Test Requirements	2775

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2775

A.8.16.57.1	Test Purpose and Environment	2775

A.8.16.57.2	Test Requirements	2777

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2778

A.8.16.58.1	Test Purpose and Environment	2778

A.8.16.58.2	Test Requirements	2780

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2781

A.8.16.59.1	Test Purpose and Environment	2781

A.8.16.59.2	Test Requirements	2785

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2786

A.8.16.60.1	Test Purpose and Environment	2786

A.8.16.60.2	Test Requirements	2789

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2790

A.8.16.61.1	Test Purpose and Environment	2790

A.8.16.61.2	Test Requirements	2793

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2793

A.8.16.62.1	Test Purpose and Environment	2793

A.8.16.62.2	Test Requirements	2795

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2796

A.8.16.63.1	Test Purpose and Environment	2796

A.8.16.63.2	Test Requirements	2799

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2800

A.8.16.64.1	Test Purpose and Environment	2800

A.8.16.64.2	Test Requirements	2804

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2804

A.8.16.65.1	Test Purpose and Environment	2804

A.8.16.65.2	Test Requirements	2809

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2809

A.8.16.66.1	Test Purpose and Environment	2809

A.8.16.66.2	Test Requirements	2814

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2814

A.8.16.67.1	Test Purpose and Environment	2814

A.8.16.67.2	Test Requirements	2819

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2819

A.8.16.68.1	Test Purpose and Environment	2819

A.8.16.68.2	Test Requirements	2824

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2824

A.8.16.69.1	Test Purpose and Environment	2824

A.8.16.69.2	Test Requirements	2828

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2828

A.8.16.70.1	Test Purpose and Environment	2828

A.8.16.70.2	Test Requirements	2832

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2832

A.8.16.71.1	Test Purpose and Environment	2832

A.8.16.71.2	Test Requirements	2837

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2837

A.8.16.72.1	Test Purpose and Environment	2837

A.8.16.72.2	Test Requirements	2842

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2842

A.8.16.73.1	Test Purpose and Environment	2842

A.8.16.73.2	Test Requirements	2849

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2849

A.8.16.74.1	Test Purpose and Environment	2849

A.8.16.74.2	Test Requirements	2855

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2855

A.8.16.75.1	Test Purpose and Environment	2855

A.8.16.75.2	Test Requirements	2861

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2861

A.8.16.76.1	Test Purpose and Environment	2861

A.8.16.76.2	Test Requirements	2867

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2867

A.8.16.77.1	Test Purpose and Environment	2867

A.8.16.77.2	Test Requirements	2871

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2871

A.8.16.78.1	Test Purpose and Environment	2871

A.8.16.78.2	Test Requirements	2873

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2874

A.8.16.79.1	Test Purpose and Environment	2874

A.8.16.79.2	Test Requirements	2881

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2881

A.8.16.80.1	Test Purpose and Environment	2881

A.8.16.80.2	Test Requirements	2888

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2888

A.8.16.81.1	Test Purpose and Environment	2888

A.8.16.81.2	Test Requirements	2894

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2894

A.8.16.82.1	Test Purpose and Environment	2894

A.8.16.82.2	Test Requirements	2900

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2900

A.8.16.83.1	Test Purpose and Environment	2900

A.8.16.83.2	Test Requirements	2905

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2905

A.8.16.84.1	Test Purpose and Environment	2905

A.8.16.84.2	Test Requirements	2911

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2911

A.8.16.85.1	Test Purpose and Environment	2911

A.8.16.85.2	Test Requirements	2915

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2915

A.8.16.86.1	Test Purpose and Environment	2915

A.8.16.86.2	Test Requirements	2918

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2918

A.8.16.87.1	Test Purpose and Environment	2918

A.8.16.87.2	Test Requirements	2924

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2924

A.8.16.88.1	Test Purpose and Environment	2924

A.8.16.88.2	Test Requirements	2932

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2932

A.8.16.89.1	Test Purpose and Environment	2932

A.8.16.89.2	Test Requirements	2939

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2939

A.8.16.90.1	Test Purpose and Environment	2939

A.8.16.90.2	Test Requirements	2946

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2946

A.8.16.91.1	Test Purpose and Environment	2946

A.8.16.91.2	Test Requirements	2952

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2952

A.8.16.92.1	Test Purpose and Environment	2952

A.8.16.92.2	Test Requirements	2960

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2960

A.8.16.93.1	Test Purpose and Environment	2960

A.8.16.93.2	Test Requirements	2967

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2967

A.8.16.94.1	Test Purpose and Environment	2967

A.8.16.94.2	Test Requirements	2974

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2974

A.8.16.95.1	Test Purpose and Environment	2974

A.8.16.95.2	Test Requirements	2982

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2982

A.8.16.96.1	Test Purpose and Environment	2982

A.8.16.96.2	Test Requirements	2991

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2991

A.8.16.97.1	Test Purpose and Environment	2991

A.8.16.97.2	Test Requirements	2998

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2998

A.8.16.98.1	Test Purpose and Environment	2998

A.8.16.98.2	Test Requirements	3006

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	3006

A.8.16.99.1	Test Purpose and Environment	3006

A.8.16.99.2	Test Requirements	3016

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	3016

A.8.16.100.1	Test Purpose and Environment	3016

A.8.16.100.2	Test Requirements	3026

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3026

A.8.16.101.1	Test Purpose and Environment	3026

A.8.16.101.2	Test Requirements	3036

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3036

A.8.16.102.1	Test Purpose and Environment	3036

A.8.16.102.2	Test Requirements	3047

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	3047

A.8.16.103.1	Test Purpose and Environment	3047

A.8.16.103.2	Test Requirements	3052

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	3053

A.8.16.104.1	Test Purpose and Environment	3053

A.8.16.104.2	Test Requirements	3057

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	3058

A.8.16.105.1	Test Purpose and Environment	3058

A.8.16.105.2	Test Requirements	3062

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	3063

A.8.16.106.1	Test Purpose and Environment	3063

A.8.16.106.2	Test Requirements	3067

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3067

A.8.16.107.1	Test Purpose and Environment	3067

A.8.16.107.2	Test Requirements	3069

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3070

A.8.16.108.1	Test Purpose and Environment	3070

A.8.16.108.2	Test Requirements	3072

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	3073

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	3073

A.8.17.1.1	Test Purpose and Environment	3073

A.8.17.1.2	Test Requirements	3079

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	3079

A.8.17.2.1	Test Purpose and Environment	3079

A.8.17.2.2	Test Requirements	3086

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	3086

A.8.17.3.1	Test Purpose and Environment	3086

A.8.17.3.2	Test Requirements	3087

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	3087

A.8.17.4.1	Test Purpose and Environment	3087

A.8.17.4.2	Test Requirements	3088

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3088

A.8.17.5.1	Test Purpose and Environment	3088

A.8.17.5.2	Test Requirements	3089

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3089

A.8.17.6.1	Test Purpose and Environment	3089

A.8.17.6.2	Test Requirements	3090

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	3091

A.8.17.7.1	Test Purpose and Environment	3091

A.8.17.7.2	Test Requirements	3091

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	3092

A.8.17.8.1	Test Purpose and Environment	3092

A.8.17.8.2	Test Requirements	3092

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	3093

A.8.17.9.1	Test Purpose and Environment	3093

A.8.17.9.2	Test Requirements	3094

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	3094

A.8.17.10.1	Test Purpose and Environment	3094

A.8.17.10.2	Test Requirements	3101

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	3102

A.8.17.11.1	Test Purpose and Environment	3102

A.8.17.11.2	Test Requirements	3108

A.8.18	E-UTRAN TDD – HRPD Measurements	3109

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	3109

A.8.18.1.1	Test Purpose and Environment	3109

A.8.18.1.2	Test Requirements	3111

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	3111

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	3111

A.8.19.1.1	Test Purpose and Environment	3111

A.8.19.1.2	Test Requirements	3112

A.8.20	Inter-frequency/RAT Measurements in CA mode	3113

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	3113

A.8.20.1.1	Test Purpose and Environment	3113

A.8.20.1.2	Test Requirements	3114

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3114

A.8.20.2.1	Test Purpose and Environment	3115

A.8.20.2.2	Test Requirements	3116

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	3117

A.8.20.2A.1	Test Purpose and Environment	3117

A.8.20.2A.2	Test Requirements	3117

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	3117

A.8.20.2B.1	Test Purpose and Environment	3117

A.8.20.2B.2	Test Requirements	3120

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	3120

A.8.20.3.1	Test Purpose and Environment	3120

A.8.20.3.2	Test Requirements	3122

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	3122

A.8.20.4.1	Test Purpose and Environment	3122

A.8.20.4.1.1	1.28 Mcps TDD option	3122

A.8.20.4.2	Test Requirements	3124

A.8.20.4.2.1	1.28 Mcps TDD option	3124

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3124

A.8.20.4A.1	Test Purpose and Environment	3124

A.8.20.4A.1.1	1.28 Mcps TDD option	3124

A.8.20.4A.2	Test Requirements	3124

A.8.20.4A.2.1	1.28 Mcps TDD option	3124

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3125

A.8.20.4B.1	Test Purpose and Environment	3125

A.8.20.4B.1.1	1.28 Mcps TDD option	3125

A.8.20.4B.2	Test Requirements	3127

A.8.20.4B.2.1	1.28 Mcps TDD option	3127

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	3127

A.8.21.1	Test Purpose and Environment	3127

A.8.21.2 Test Requirements	3131

A.8.22	E-UTRAN Discovery Signal Measurements	3131

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3131

A.8.22.1.1	Test Purpose and Environment	3131

A.8.22.1.2	Test Requirements	3134

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3134

A.8.22.2.1	Test Purpose and Environment	3134

A.8.22.2.2	Test Requirements	3137

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3137

A.8.22.3.1	Test Purpose and Environment	3137

A.8.22.3.2	Test Requirements	3140

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3140

A.8.22.4.1	Test Purpose and Environment	3140

A.8.22.4.2	Test Requirements	3143

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3143

A.8.22.5.1	Test Purpose and Environment	3143

A.8.22.5.2	Test Requirements	3147

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3147

A.8.22.6.1	Test Purpose and Environment	3147

A.8.22.6.2	Test Requirements	3151

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3151

A.8.22.7.1	Test Purpose and Environment	3151

A.8.22.7.2	Test Requirements	3155

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3155

A.8.22.8.1	Test Purpose and Environment	3155

A.8.22.8.2	Test Requirements	3159

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3159

A.8.22.9.1	Test Purpose and Environment	3159

A.8.22.9.2	Test Requirements	3161

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3162

A.8.22.10.1	Test Purpose and Environment	3162

A.8.22.10.2	Test Requirements	3164

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3165

A.8.22.11.1	Test Purpose and Environment	3165

A.8.22.11.2	Test Requirements	3168

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3168

A.8.22.12.1	Test Purpose and Environment	3168

A.8.22.12.2	Test Requirements	3172

A.8.23	E-UTRAN Dual Connectivity Measurements	3172

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3172

A.8.23.1.1	Test Purpose and Environment	3172

A.8.23.1.2	Test Requirements	3175

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3175

A.8.23.2.1	Test Purpose and Environment	3175

A.8.23.2.2	Test Requirements	3178

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3178

A.8.23.3.1	Test Purpose and Environment	3178

A.8.23.3.2	Test Requirements	3181

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3181

A.8.23.4.1	Test Purpose and Environment	3181

A.8.23.4.2	Test Requirements	3184

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3184

A.8.23.5.1	Test Purpose and Environment	3184

A.8.23.5.2	Test Requirements	3187

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3187

A.8.23.6.1	Test Purpose and Environment	3187

A.8.23.6.2	Test Requirements	3190

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3190

A.8.23.7.1	Test Purpose and Environment	3190

A.8.23.7.2	Test Requirements	3192

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3193

A.8.23.8.1	Test Purpose and Environment	3193

A.8.23.8.2	Test Requirements	3195

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3196

A.8.23.9.1	Test Purpose and Environment	3196

A.8.23.9.2	Test Requirements	3199

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3199

A.8.23.10.1	Test Purpose and Environment	3199

A.8.23.10.2	Test Requirements	3202

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3202

A.8.23.11.1	Test Purpose and Environment	3202

A.8.23.11.2	Test Requirements	3205

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3205

A.8.23.12.1	Test Purpose and Environment	3205

A.8.23.12.2	Test Requirements	3208

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3208

A.8.23.13.1	Test Purpose and Environment	3208

A.8.23.13.2	Test Requirements	3211

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3211

A.8.23.14.1	Test Purpose and Environment	3211

A.8.23.14.2	Test Requirements	3214

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3214

A.8.23.15.1	Test Purpose and Environment	3214

A.8.23.15.2	Test Requirements	3217

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3217

A.8.23.16.1	Test Purpose and Environment	3217

A.8.23.16.2	Test Requirements	3218

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3219

A.8.23.17.1	Test Purpose and Environment	3219

A.8.23.17.2	Test Requirements	3221

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3221

A.8.23.18.1	Test Purpose and Environment	3221

A.8.23.18.2	Test Requirements	3222

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3223

A.8.23.19.1	Test Purpose and Environment	3223

A.8.23.19.2	Test Requirements	3224

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3225

A.8.23.20.1	Test Purpose and Environment	3225

A.8.23.20.2	Test Requirements	3226

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3227

A.8.23.21.1	Test Purpose and Environment	3227

A.8.23.21.2	Test Requirements	3228

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3229

A.8.23.22.1	Test Purpose and Environment	3229

A.8.23.22.2	Test Requirements	3230

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3231

A.8.23.23.1	Test Purpose and Environment	3231

A.8.23.23.2	Test Requirements	3232

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3233

A.8.23.24.1	Test Purpose and Environment	3233

A.8.23.24.2	Test Requirements	3235

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3236

A.8.23.25.1	Test Purpose and Environment	3236

A.8.23.25.2	Test Requirements	3238

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3239

A.8.23.26.1	Test Purpose and Environment	3239

A.8.23.26.2	Test Requirements	3241

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3242

A.8.23.27.1	Test Purpose and Environment	3242

A.8.23.27.2	Test Requirements	3245

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3245

A.8.23.28.1	Test Purpose and Environment	3245

A.8.23.28.2	Test Requirements	3249

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3249

A.8.23.29.1	Test Purpose and Environment	3249

A.8.23.29.2	Test Requirements	3253

A.8.24	Proximity-based Services	3253

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3253

A.8.24.1.1	Test Purpose and Environment	3253

A.8.24.1.2	Test Requirements	3254

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3255

A.8.24.2.1	Test Purpose and Environment	3255

A.8.24.2.2	Test Requirements	3256

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3256

A.8.24.3.1	Test Purpose and Environment	3257

A.8.24.3.2	Test Requirements	3258

A.8.25	E-UTRAN-WLAN Measurements	3259

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3259

A.8.25.1.1	Test Purpose and Environment	3259

A.8.25.1.2	Test Requirements	3261

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3261

A.8.25.2.1	Test Purpose and Environment	3261

A.8.25.2.2	Test Requirements	3263

A.8.26 Frame Structure 3 (FS3)	3263

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3263

A.8.26.1.1	Test Purpose and Environment	3263

A.8.26.1.2	Test Requirements	3265

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3266

A.8.26.2.1	Test Purpose and Environment	3266

A.8.26.2.2	Test Requirements	3268

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3269

A.8.26.3.1	Test Purpose and Environment	3269

A.8.26.3.2	Test Requirements	3272

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3272

A.8.26.3A.1	Test Purpose and Environment	3272

A.8.26.3A.2	Test Requirements	3275

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3275

A.8.26.4.1	Test Purpose and Environment	3275

A.8.26.4.2	Test Requirements	3279

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3279

A.8.26.4A.1	Test Purpose and Environment	3279

A.8.26.4A.2	Test Requirements	3283

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3283

A.8.26.5.1	Test Purpose and Environment	3283

A.8.26.5.2	Test Requirements	3286

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3286

A.8.26.5A.1	Test Purpose and Environment	3286

A.8.26.5A.2	Test Requirements	3290

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3290

A.8.26.6.1	Test Purpose and Environment	3290

A.8.26.6.2	Test Requirements	3294

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3294

A.8.26.6A.1	Test Purpose and Environment	3294

A.8.26.6A.2	Test Requirements	3298

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3298

A.8.26.7.1	Test Purpose and Environment	3298

A.8.26.7.2	Test Requirements	3301

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3301

A.8.26.8.1	Test Purpose and Environment	3301

A.8.26.8.2	Test Requirements	3305

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3305

A.8.26.9.1	Test Purpose and Environment	3305

A.8.26.9.2	Test Requirements	3308

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3308

A.8.26.10.1	Test Purpose and Environment	3308

A.8.26.10.2	Test Requirements	3311

A.9	Measurement Performance Requirements	3313

A.9.1	RSRP	3313

A.9.1.1	FDD Intra frequency case	3313

A.9.1.1.1	Test Purpose and Environment	3313

A.9.1.1.2	Test parameters	3313

A.9.1.1.3	Test Requirements	3316

A.9.1.2	TDD Intra frequency case	3316

A.9.1.2.1	Test Purpose and Environment	3316

A.9.1.2.2	Test parameters	3316

A.9.1.2.3	Test Requirements	3317

A.9.1.3	FDD—FDD Inter frequency case	3318

A.9.1.3.1	Test Purpose and Environment	3318

A.9.1.3.2	Test parameters	3318

A.9.1.3.3	Test Requirements	3321

A.9.1.4	TDD—TDD Inter frequency case	3321

A.9.1.4.1	Test Purpose and Environment	3321

A.9.1.4.2	Test parameters	3321

A.9.1.4.3	Test Requirements	3323

A.9.1.5	FDD—TDD Inter frequency case	3324

A.9.1.5.1	Test Purpose and Environment	3324

A.9.1.5.2	Test parameters	3324

A.9.1.5.3	Test Requirements	3325

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3326

A.9.1.6.1	Test Purpose and Environment	3326

A.9.1.6.2	Test parameters	3326

A.9.1.6.3	Test Requirements	3329

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3329

A.9.1.7.1	Test Purpose and Environment	3329

A.9.1.7.2	Test parameters	3329

A.9.1.7.3	Test Requirements	3332

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3332

A.9.1.8.1	Test Purpose and Environment	3332

A.9.1.8.2	Test parameters	3332

A.9.1.8.3	Test Requirements	3336

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3336

A.9.1.9.1	Test Purpose and Environment	3336

A.9.1.9.2	Test parameters	3336

A.9.1.9.3	Test Requirements	3339

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3339

A.9.1.10.1	Test Purpose and Environment	3339

A.9.1.10.2	Test parameters	3339

A.9.1.10.3	Test Requirements	3342

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3342

A.9.1.11.1	Test Purpose and Environment	3342

A.9.1.11.2	Test parameters	3342

A.9.1.11.3	Test Requirements	3346

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3346

A.9.1.12.1	Test Purpose and Environment	3346

A.9.1.12.2	Test parameters	3346

A.9.1.12.3	Test Requirements	3347

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3347

A.9.1.13.1	Test Purpose and Environment	3347

A.9.1.13.2	Test parameters	3347

A.9.1.13.3	Test Requirements	3348

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3348

A.9.1.14.1	Test Purpose and Environment	3348

A.9.1.14.2	Test parameters	3348

A.9.1.14.3	Test Requirements	3352

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3352

A.9.1.15.1	Test Purpose and Environment	3352

A.9.1.15.2	Test parameters	3352

A.9.1.15.3	Test Requirements	3354

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3355

A.9.1.16.1	Test Purpose and Environment	3355

A.9.1.16.2	Test parameters	3355

A.9.1.16.3	Test Requirements	3356

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3357

A.9.1.17.1	Test Purpose and Environment	3357

A.9.1.17.2	Test parameters	3357

A.9.1.17.3	Test Requirements	3358

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3359

A.9.1.18.1	Test Purpose and Environment	3359

A.9.1.18.2	Test parameters	3359

A.9.1.18.3	Test Requirements	3360

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3361

A.9.1.19.1	Test Purpose and Environment	3361

A.9.1.19.2	Test parameters	3361

A.9.1.19.3	Test Requirements	3361

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3361

A.9.1.20.1	Test Purpose and Environment	3361

A.9.1.20.2	Test parameters	3361

A.9.1.20.3	Test Requirements	3363

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3363

A.9.1.21.1	Test Purpose and Environment	3363

A.9.1.21.2	Test parameters	3363

A.9.1.21.3	Test Requirements	3363

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3363

A.9.1.22.1	Test Purpose and Environment	3363

A.9.1.22.2	Test parameters	3364

A.9.1.22.3	Test Requirements	3368

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3368

A.9.1.23.1	Test Purpose and Environment	3368

A.9.1.23.2	Test parameters	3368

A.9.1.23.3	Test Requirements	3372

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3372

A.9.1.24.1	Test Purpose and Environment	3372

A.9.1.24.2	Test parameters	3372

A.9.1.24.3	Test Requirements	3373

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3373

A.9.1.25.1	Test Purpose and Environment	3373

A.9.1.25.2	Test parameters	3373

A.9.1.25.3	Test Requirements	3376

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3376

A.9.1.26.1	Test Purpose and Environment	3376

A.9.1.26.2	Test parameters	3376

A.9.1.26.3	Test Requirements	3379

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3379

A.9.1.27.1	Test Purpose and Environment	3379

A.9.1.27.2	Test parameters	3379

A.9.1.27.3	Test Requirements	3382

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3382

A.9.1.28.1	Test Purpose and Environment	3382

A.9.1.28.2	Test parameters	3382

A.9.1.28.3	Test Requirements	3385

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3385

A.9.1.29.1	Test Purpose and Environment	3385

A.9.1.29.2	Test parameters	3385

A.9.1.29.3	Test Requirements	3388

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3388

A.9.1.30.1	Test Purpose and Environment	3388

A.9.1.30.2	Test parameters	3389

A.9.1.30.3	Test Requirements	3392

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3392

A.9.1.31.1	Test Purpose and Environment	3392

A.9.1.31.2	Test parameters	3392

A.9.1.31.3	Test Requirements	3395

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3396

A.9.1.32.1	Test Purpose and Environment	3396

A.9.1.32.2	Test parameters	3396

A.9.1.32.3	Test Requirements	3399

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3399

A.9.1.33.1	Test Purpose and Environment	3399

A.9.1.33.2	Test parameters	3399

A.9.1.33.3	Test Requirements	3402

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3402

A.9.1.34.1	Test Purpose and Environment	3402

A.9.1.34.2	Test parameters	3402

A.9.1.34.3	Test Requirements	3405

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3405

A.9.1.35.1	Test Purpose and Environment	3405

A.9.1.35.2	Test parameters	3405

A.9.1.35.3	Test Requirements	3408

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3409

A.9.1.36.1	Test Purpose and Environment	3409

A.9.1.36.2	Test parameters	3409

A.9.1.36.3	Test Requirements	3412

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3412

A.9.1.37.1	Test Purpose and Environment	3412

A.9.1.37.2	Test parameters	3412

A.9.1.37.3	Test Requirements	3415

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3416

A.9.1.38.1	Test Purpose and Environment	3416

A.9.1.38.2	Test parameters	3416

A.9.1.38.3	Test Requirements	3419

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3420

A.9.1.39.1	Test Purpose and Environment	3420

A.9.1.39.2	Test parameters	3420

A.9.1.39.3	Test Requirements	3426

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3426

A.9.1.40.1	Test Purpose and Environment	3426

A.9.1.40.2	Test parameters	3427

A.9.1.40.3	Test Requirements	3433

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3433

A.9.1.41.1	Test Purpose and Environment	3433

A.9.1.41.2	Test parameters	3433

A.9.1.41.3	Test Requirements	3436

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3436

A.9.1.42.1	Test Purpose and Environment	3436

A.9.1.42.2	Test parameters	3436

A.9.1.42.3	Test Requirements	3439

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3439

A.9.1.43.1	Test Purpose and Environment	3439

A.9.1.43.2	Test parameters	3439

A.9.1.43.3	Test Requirements	3440

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3441

A.9.1.44.1	Test Purpose and Environment	3441

A.9.1.44.2	Test parameters	3441

A.9.1.44.3	Test Requirements	3446

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3446

A.9.1.45.1	Test Purpose and Environment	3446

A.9.1.45.2	Test parameters	3446

A.9.1.45.3	Test Requirements	3432

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3432

A.9.1.46.1	Test Purpose and Environment	3432

A.9.1.46.2	Test parameters	3432

A.9.1.46.3	Test Requirements	3441

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3442

A.9.1.47.1	Test Purpose and Environment	3442

A.9.1.47.2	Test parameters	3442

A.9.1.47.3	Test Requirements	3451

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3451

A.9.1.48.1	Test Purpose and Environment	3451

A.9.1.48.2	Test parameters	3452

A.9.1.48.3	Test Requirements	3462

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.49.1	Test Purpose and Environment	3463

A.9.1.49.2	Test parameters	3463

A.9.1.49.3	Test Requirements	3473

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3474

A.9.1.50.1	Test Purpose and Environment	3474

A.9.1.50.2	Test parameters	3474

A.9.1.50.3	Test Requirements	3481

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3482

A.9.1.51.1	Test Purpose and Environment	3482

A.9.1.51.2	Test parameters	3482

A.9.1.51.3	Test Requirements	3488

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3489

A.9.1.52.1	Test Purpose and Environment	3489

A.9.1.52.2	Test parameters	3489

A.9.1.52.3	Test Requirements	3492

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3492

A.9.1.52A.1	Test Purpose and Environment	3492

A.9.1.52A.2	Test parameters	3492

A.9.1.52A.3	Test Requirements	3493

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3493

A.9.1.53.1	Test Purpose and Environment	3493

A.9.1.53.2	Test parameters	3494

A.9.1.53.3	Test Requirements	3497

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3497

A.9.1.53A.1	Test Purpose and Environment	3497

A.9.1.53A.2	Test parameters	3497

A.9.1.53A.3	Test Requirements	3498

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3499

A.9.1.54.1	Test Purpose and Environment	3499

A.9.1.54.2	Test parameters	3499

A.9.1.54.3	Test Requirements	3501

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3501

A.9.1.55.1	Test Purpose and Environment	3501

A.9.1.55.2	Test parameters	3501

A.9.1.55.3	Test Requirements	3505

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3505

A.9.1.56.1	Test Purpose and Environment	3505

A.9.1.56.2	Test parameters	3505

A.9.1.56.3	Test Requirements	3508

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3509

A.9.1.57.1	Test Purpose and Environment	3509

A.9.1.57.2	Test parameters	3509

A.9.1.57.3	Test Requirements	3512

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3512

A.9.1.57A.1	Test Purpose and Environment	3512

A.9.1.57A.2	Test parameters	3512

A.9.1.57A.3	Test Requirements	3513

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3513

A.9.1.58.1	Test Purpose and Environment	3513

A.9.1.58.2	Test parameters	3514

A.9.1.58.3	Test Requirements	3517

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3517

A.9.1.58A.1	Test Purpose and Environment	3517

A.9.1.58A.2	Test parameters	3517

A.9.1.58A.3	Test Requirements	3518

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3518

A.9.1.59.1	Test Purpose and Environment	3518

A.9.1.59.2	Test parameters	3519

A.9.1.59.3	Test Requirements	3521

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3521

A.9.1.60.1	Test Purpose and Environment	3521

A.9.1.60.2	Test parameters	3521

A.9.1.60.3	Test Requirements	3524

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3525

A.9.1.61.1	Test Purpose and Environment	3525

A.9.1.61.2	Test parameters	3525

A.9.1.61.3	Test Requirements	3528

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3528

A.9.1.62.1	Test Purpose and Environment	3528

A.9.1.62.2	Test parameters	3528

A.9.1.62.3	Test Requirements	3531

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3531

A.9.1.53.1	Test Purpose and Environment	3531

A.9.1.53.2	Test parameters	3531

A.9.1.63.3	Test Requirements	3534

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3534

A.9.1.64.1	Test Purpose and Environment	3534

A.9.1.64.2	Test parameters	3534

A.9.1.64.3	Test Requirements	3535

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3536

A.9.1.65.1	Test Purpose and Environment	3536

A.9.1.65.2	Test parameters	3536

A.9.1.65.3	Test Requirements	3542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3542

A.9.1.66.1	Test Purpose and Environment	3542

A.9.1.66.2	Test parameters	3542

A.9.1.66.3	Test Requirements	3548

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3548

A.9.1.67.1	Test Purpose and Environment	3548

A.9.1.67.2	Test parameters	3548

A.9.1.67.3	Test Requirements	3549

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3550

A.9.1.68.1	Test Purpose and Environment	3550

A.9.1.68.2	Test parameters	3550

A.9.1.68.3	Test Requirements	3554

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3554

A.9.1.69.1	Test Purpose and Environment	3554

A.9.1.69.2	Test parameters	3554

A.9.1.69.3	Test Requirements	3558

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3559

A.9.1.70.1	Test Purpose and Environment	3559

A.9.1.70.2	Test parameters	3559

A.9.1.70.3	Test Requirements	3571

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3572

A.9.1.71.1	Test Purpose and Environment	3572

A.9.1.71.2	Test parameters	3572

A.9.1.71.3	Test Requirements	3584

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3585

A.9.1.72.1	Test Purpose and Environment	3585

A.9.1.72.2	Test parameters	3585

A.9.1.72.3	Test Requirements	3597

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3598

A.9.1.73.1	Test Purpose and Environment	3598

A.9.1.73.2	Test parameters	3598

A.9.1.73.3	Test Requirements	3601

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3601

A.9.1.74.1	Test Purpose and Environment	3601

A.9.1.74.2	Test parameters	3601

A.9.1.74.3	Test Requirements	3602

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3602

A.9.1.75.1	Test Purpose and Environment	3602

A.9.1.75.2	Test parameters	3603

A.9.1.75.3	Test Requirements	3604

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3604

A.9.1.76.1	Test Purpose and Environment	3604

A.9.1.76.2	Test parameters	3604

A.9.1.76.3	Test Requirements	3607

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3607

A.9.1.77.1	Test Purpose and Environment	3607

A.9.1.77.2	Test parameters	3607

A.9.1.77.3	Test Requirements	3610

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3610

A.9.1.78.1	Test Purpose and Environment	3610

A.9.1.78.2	Test parameters	3610

A.9.1.78.3	Test Requirements	3613

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3613

A.9.1.79.1	Test Purpose and Environment	3613

A.9.1.79.2	Test parameters	3613

A.9.1.79.3	Test Requirements	3616

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3616

A.9.1.80.1	Test Purpose and Environment	3616

A.9.1.80.2	Test parameters	3616

A.9.1.80.3	Test Requirements	3619

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3619

A.9.1.81.1	Test Purpose and Environment	3619

A.9.1.78.2	Test parameters	3619

A.9.1.81.3	Test Requirements	3622

A.9.2	RSRQ	3622

A.9.2.1	FDD Intra frequency case	3622

A.9.2.1.1	Test Purpose and Environment	3622

A.9.2.1.2	Test parameters	3622

A.9.2.1.3	Test Requirements	3625

A.9.2.2	TDD Intra frequency case	3625

A.9.2.2.1	Test Purpose and Environment	3625

A.9.2.2.2	Test parameters	3625

A.9.2.2.3	Test Requirements	3627

A.9.2.3	FDD—FDD Inter frequency case	3627

A.9.2.3.1	Test Purpose and Environment	3627

A.9.2.3.2	Test parameters	3627

A.9.2.3.3	Test Requirements	3630

A.9.2.4	TDD—TDD Inter frequency case	3630

A.9.2.4.1	Test Purpose and Environment	3630

A.9.2.4.2	Test parameters	3630

A.9.2.4.3	Test Requirements	3632

A.9.2.4A	FDD—TDD Inter frequency case	3633

A.9.2.4A.1	Test Purpose and Environment	3633

A.9.2.4A.2	Test parameters	3633

A.9.2.4A.3	Test Requirements	3635

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3635

A.9.2.5.1	Test Purpose and Environment	3635

A.9.2.5.2	Test parameters	3636

A.9.2.5.3	Test Requirements	3639

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3639

A.9.2.6.1	Test Purpose and Environment	3639

A.9.2.6.2	Test parameters	3640

A.9.2.6.3	Test Requirements	3643

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3643

A.9.2.7.1	Test Purpose and Environment	3643

A.9.2.7.2	Test parameters	3643

A.9.2.7.3	Test Requirements	3647

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3647

A.9.2.8.1	Test Purpose and Environment	3647

A.9.2.8.2	Test parameters	3647

A.9.2.8.3	Test Requirements	3651

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3651

A.9.2.9.1	Test Purpose and Environment	3651

A.9.2.9.2	Test parameters	3651

A.9.2.9.3	Test Requirements	3655

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3655

A.9.2.10.1	Test Purpose and Environment	3655

A.9.2.10.2	Test parameters	3655

A.9.2.10.3	Test Requirements	3659

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3659

A.9.2.11.1	Test Purpose and Environment	3659

A.9.2.11.2	Test parameters	3659

A.9.2.11.3	Test Requirements	3660

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3660

A.9.2.12.1	Test Purpose and Environment	3660

A.9.2.12.2	Test parameters	3661

A.9.2.12.3	Test Requirements	3661

A.9.2.13	Void	3662

A.9.2.13.1	Void	3662

A.9.2.13.2	Void	3662

A.9.2.13.3	Void	3662

A.9.2.14	Void	3662

A.9.2.14.1	Void	3662

A.9.2.14.2	Void	3662

A.9.2.14.3	Void	3662

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3662

A.9.2.15.1	Test Purpose and Environment	3662

A.9.2.15.2	Test parameters	3662

A.9.2.15.3	Test Requirements	3666

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3666

A.9.2.16.1	Test Purpose and Environment	3666

A.9.2.16.2	Test parameters	3666

A.9.2.16.3	Test Requirements	3671

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3671

A.9.2.17.1	Test Purpose and Environment	3671

A.9.2.17.2	Test parameters	3671

A.9.2.17.3	Test Requirements	3673

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3673

A.9.2.18.1	Test Purpose and Environment	3673

A.9.2.18.2	Test parameters	3673

A.9.2.18.3	Test Requirements	3676

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3676

A.9.2.19.1	Test Purpose and Environment	3676

A.9.2.19.2	Test parameters	3676

A.9.2.19.3	Test Requirements	3678

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3678

A.9.2.20.1	Test Purpose and Environment	3678

A.9.2.20.2	Test parameters	3678

A.9.2.20.3	Test Requirements	3681

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3681

A.9.2.21.1	Test Purpose and Environment	3681

A.9.2.21.2	Test parameters	3681

A.9.2.21.3	Test Requirements	3684

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3684

A.9.2.22.1	Test Purpose and Environment	3685

A.9.2.22.2	Test parameters	3685

A.9.2.22.3	Test Requirements	3685

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3685

A.9.2.23.1	Test Purpose and Environment	3685

A.9.2.23.2	Test parameters	3685

A.9.2.23.3	Test Requirements	3688

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3688

A.9.2.24.1	Test Purpose and Environment	3688

A.9.2.24.2	Test parameters	3688

A.9.2.24.3	Test Requirements	3689

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3689

A.9.2.25.1	Test Purpose and Environment	3689

A.9.2.25.2	Test parameters	3689

A.9.2.25.3	Test Requirements	3694

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3694

A.9.2.26.1	Test Purpose and Environment	3694

A.9.2.26.2	Test parameters	3694

A.9.2.26.3	Test Requirements	3698

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3698

A.9.2.27.1	Test Purpose and Environment	3698

A.9.2.27.2	Test parameters	3698

A.9.2.27.3	Test Requirements	3699

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3699

A.9.2.28.1	Test Purpose and Environment	3699

A.9.2.28.2	Test parameters	3699

A.9.2.28.3	Test Requirements	3702

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3702

A.9.2.29.1	Test Purpose and Environment	3702

A.9.2.29.2	Test parameters	3702

A.9.2.29.3	Test Requirements	3705

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3705

A.9.2.30.1	Test Purpose and Environment	3705

A.9.2.30.2	Test parameters	3705

A.9.2.30.3	Test Requirements	3708

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3708

A.9.2.31.1	Test Purpose and Environment	3708

A.9.2.31.2	Test parameters	3708

A.9.2.31.3	Test Requirements	3711

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3711

A.9.2.32.1	Test Purpose and Environment	3711

A.9.2.32.2	Test parameters	3711

A.9.2.32.3	Test Requirements	3714

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3714

A.9.2.33.1	Test Purpose and Environment	3714

A.9.2.33.2	Test parameters	3714

A.9.2.33.3	Test Requirements	3718

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3718

A.9.2.34.1	Test Purpose and Environment	3718

A.9.2.34.2	Test parameters	3718

A.9.2.34.3	Test Requirements	3721

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3721

A.9.2.35.1	Test Purpose and Environment	3721

A.9.2.35.2	Test parameters	3721

A.9.2.35.3	Test Requirements	3724

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3724

A.9.2.36.1	Test Purpose and Environment	3724

A.9.2.36.2	Test parameters	3724

A.9.2.36.3	Test Requirements	3725

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3726

A.9.2.37.1	Test Purpose and Environment	3726

A.9.2.37.2	Test parameters	3726

A.9.2.37.3	Test Requirements	3727

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3728

A.9.2.38.1	Test Purpose and Environment	3728

A.9.2.38.2	Test parameters	3728

A.9.2.38.3	Test Requirements	3732

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3732

A.9.2.39.1	Test Purpose and Environment	3732

A.9.2.39.2	Test parameters	3732

A.9.2.39.3	Test Requirements	3736

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3736

A.9.2.40.1	Test Purpose and Environment	3736

A.9.2.40.2	Test parameters	3736

A.9.2.40.3	Test Requirements	3739

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3740

A.9.2.41.1	Test Purpose and Environment	3740

A.9.2.41.2	Test parameters	3740

A.9.2.41.3	Test Requirements	3743

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3743

A.9.2.42.1	Test Purpose and Environment	3743

A.9.2.42.2	Test parameters	3743

A.9.2.42.3	Test Requirements	3746

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3746

A.9.2.43.1	Test Purpose and Environment	3746

A.9.2.43.2	Test parameters	3746

A.9.2.43.3	Test Requirements	3749

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3749

A.9.2.44.1	Test Purpose and Environment	3749

A.9.2.44.2	Test parameters	3749

A.9.2.44.3	Test Requirements	3751

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3751

A.9.2.45.1	Test Purpose and Environment	3751

A.9.2.45.2	Test parameters	3751

A.9.2.45.3	Test Requirements	3755

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3756

A.9.2.46.1	Test Purpose and Environment	3756

A.9.2.46.2	Test parameters	3756

A.9.2.46.3	Test Requirements	3760

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3761

A.9.2.47.1	Test Purpose and Environment	3761

A.9.2.47.2	Test parameters	3761

A.9.2.47.3	Test Requirements	3765

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3765

A.9.2.48.1	Test Purpose and Environment	3765

A.9.2.48.2	Test parameters	3766

A.9.2.48.3	Test Requirements	3770

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3771

A.9.2.49.1	Test Purpose and Environment	3771

A.9.2.49.2	Test parameters	3771

A.9.2.49.3	Test Requirements	3777

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3778

A.9.2.50.1	Test Purpose and Environment	3778

A.9.2.50.2	Test parameters	3778

A.9.2.50.3	Test Requirements	3784

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3784

A.9.2.51.1	Test Purpose and Environment	3784

A.9.2.51.2	Test parameters	3785

A.9.2.51.3	Test Requirements	3789

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3789

A.9.2.52.1	Test Purpose and Environment	3789

A.9.2.52.2	Test parameters	3789

A.9.2.52.3	Test Requirements	3792

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3793

A.9.2.53.1	Test Purpose and Environment	3793

A.9.2.53.2	Test parameters	3793

A.9.2.53.3	Test Requirements	3799

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3800

A.9.2.54.1	Test Purpose and Environment	3800

A.9.2.54.2	Test parameters	3800

A.9.2.54.3	Test Requirements	3806

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3806

A.9.2.55.1	Test Purpose and Environment	3806

A.9.2.55.2	Test parameters	3807

A.9.2.55.3	Test Requirements	3812

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3812

A.9.2.56.1	Test Purpose and Environment	3812

A.9.2.56.2	Test parameters	3812

A.9.2.56.3	Test Requirements	3817

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3817

A.9.2.57.1	Test Purpose and Environment	3817

A.9.2.57.2	Test parameters	3818

A.9.2.57.3	Test Requirements	3822

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3823

A.9.2.58.1	Test Purpose and Environment	3823

A.9.2.58.2	Test parameters	3823

A.9.2.58.3	Test Requirements	3827

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3828

A.9.2.59.1	Test Purpose and Environment	3828

A.9.2.59.2	Test parameters	3828

A.9.2.59.3	Test Requirements	3833

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3834

A.9.2.60.1	Test Purpose and Environment	3834

A.9.2.60.2	Test parameters	3834

A.9.2.60.3	Test Requirements	3837

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3837

A.9.2.61.1	Test Purpose and Environment	3837

A.9.2.61.2	Test parameters	3837

A.9.2.61.3	Test Requirements	3840

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3840

A.9.2.62.1	Test Purpose and Environment	3840

A.9.2.62.2	Test parameters	3840

A.9.2.62.3	Test Requirements	3843

A.9.3	UTRAN FDD CPICH RSCP	3843

A.9.3.1	E-UTRAN FDD	3843

A.9.3.1.1	Test Purpose and Environment	3843

A.9.3.1.2	Parameters	3843

A.9.3.1.3	Test Requirements	3846

A.9.3.2 E-UTRAN TDD	3846

A.9.3.2.1	Test Purpose and Environment	3846

A.9.3.2.2	Parameters	3846

A.9.3.2.3	Test Requirements	3849

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3849

A.9.3.3.1	Test Purpose and Environment	3849

A.9.3.3.2	Parameters	3849

A.9.3.3.3	Test Requirements	3850

A.9.4	UTRAN FDD CPICH Ec/No	3851

A.9.4.1	E-UTRAN FDD	3851

A.9.4.1.1	Test Purpose and Environment	3851

A.9.4.1.2	Parameters	3851

A.9.4.1.3	Test Requirements	3853

A.9.4.2	E-UTRAN TDD	3854

A.9.4.2.1	Test Purpose and Environment	3854

A.9.4.2.2	Parameters	3854

A.9.4.2.3	Test Requirements	3856

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3857

A.9.4.3.1	Test Purpose and Environment	3857

A.9.4.3.2	Parameters	3857

A.9.4.3.3	Test Requirements	3858

A.9.5	UTRAN TDD measurement	3858

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3858

A.9.5.1.1	Test Purpose and Environment	3858

A.9.5.1.2	Test parameters	3858

A.9.5.1.3	Test Requirements	3860

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3860

A.9.5.2.1	Test Purpose and Environment	3860

A.9.5.2.2	Test parameters	3860

A.9.5.2.3	Test Requirements	3862

A.9.6	GSM Carrier RSSI	3863

A.9.6.1	E-UTRAN FDD	3863

A.9.6.1.1	Test Purpose and Environment	3863

A.9.6.1.2	Test Requirements	3864

A.9.6.2	E-UTRAN TDD	3864

A.9.6.2.1	Test Purpose and Environment	3864

A.9.6.2.2	Test Requirements	3866

A.9.7	UE Rx – Tx Time Difference	3867

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3867

A.9.7.1.1	Test Purpose and Environment	3867

A.9.7.1.2	Test parameters	3867

A.9.7.1.3	Test Requirements	3868

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3868

A.9.7.2.1	Test Purpose and Environment	3868

A.9.7.2.2	Test parameters	3868

A.9.7.2.3	Test Requirements	3870

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3870

A.9.7.3.1	Test Purpose and Environment	3870

A.9.7.3.2	Test parameters	3870

A.9.7.3.3	Test Requirements	3873

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3873

A.9.7.4.1	Test Purpose and Environment	3873

A.9.7.4.2	Test Parameters	3873

A.9.7.4.3	Test Requirements	3876

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3876

A.9.7.5.1	Test Purpose and Environment	3876

A.9.7.5.2	Test parameters	3876

A.9.7.5.3	Test Requirements	3879

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3879

A.9.7.6.1	Test Purpose and Environment	3879

A.9.7.6.2	Test Parameters	3879

A.9.7.6.3	Test Requirements	3882

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3882

A.9.7.7.1	Test Purpose and Environment	3882

A.9.7.7.2	Test parameters	3882

A.9.7.7.3	Test Requirements	3883

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3884

A.9.7.8.1	Test Purpose and Environment	3884

A.9.7.8.2	Test parameters	3884

A.9.7.8.3	Test Requirements	3885

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3885

A.9.7.9.1	Test Purpose and Environment	3885

A.9.7.9.2	Test parameters	3885

A.9.7.9.3	Test Requirements	3886

A.9.8	RSTD	3887

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3887

A.9.8.1.1	Test Purpose and Environment	3887

A.9.8.1.2	Test Requirements	3889

A.9.8.1.2A	Test Requirements for UE Category 1bis	3889

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3890

A.9.8.2.1	Test Purpose and Environment	3890

A.9.8.2.2	Test Requirements	3893

A.9.8.2.2A	Test Requirements for UE Category 1bis	3893

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3894

A.9.8.3.1	Test Purpose and Environment	3894

A.9.8.3.2	Test Requirements	3896

A.9.8.3.2A	Test Requirements for UE Category 1bis	3896

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3897

A.9.8.4.1	Test Purpose and Environment	3897

A.9.8.4.2	Test Requirements	3899

A.9.8.4.2A	Test Requirements for UE Category 1bis	3899

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3900

A.9.8.5.1	Test Purpose and Environment	3900

A.9.8.5.2	Test Requirements	3902

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3902

A.9.8.6.1	Test Purpose and Environment	3902

A.9.8.6.2	Test Requirements	3905

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3905

A.9.8.7.1	Test Purpose and Environment	3905

A.9.8.7.2	Test Requirements	3906

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3906

A.9.8.8.1	Test Purpose and Environment	3906

A.9.8.8.2	Test Requirements	3907

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3907

A.9.8.9.1	Test Purpose and Environment	3907

A.9.8.9.2	Test Requirements	3908

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3908

A.9.8.10.1	Test Purpose and Environment	3908

A.9.8.10.2	Test Requirements	3909

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3909

A.9.8.11.1	Test Purpose and Environment	3909

A.9.8.11.2	Test Requirements	3910

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3910

A.9.8.12.1	Test Purpose and Environment	3910

A.9.8.12.2	Test Requirements	3911

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3911

A.9.8.13.1	Test Purpose and Environment	3911

A.9.8.13.2	Test Requirements	3912

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3912

A.9.8.14.1	Test Purpose and Environment	3912

A.9.8.14.2	Test Requirements	3919

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3919

A.9.8.15.1	Test Purpose and Environment	3919

A.9.8.15.2	Test Requirements	3925

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3925

A.9.8.16.1	Test Purpose and Environment	3925

A.9.8.16.2	Test Requirements	3928

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3928

A.9.8.17.1	Test Purpose and Environment	3928

A.9.8.17.2	Test Requirements	3931

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3931

A.9.8.18.1	Test Purpose and Environment	3931

A.9.8.18.2	Test Requirements	3934

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3934

A.9.8.19.1	Test Purpose and Environment	3934

A.9.8.19.2	Test Requirements	3937

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3937

A.9.8.20.1	Test Purpose and Environment	3937

A.9.8.20.2	Test Requirements	3940

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3941

A.9.8.21.1	Test Purpose and Environment	3941

A.9.8.21.2	Test Requirements	3944

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3945

A.9.8.22.1	Test Purpose and Environment	3945

A.9.8.22.2	Test Requirements	3948

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3949

A.9.8.23.1	Test Purpose and Environment	3949

A.9.8.23.2	Test Requirements	3952

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3952

A.9.8.24.1	Test Purpose and Environment	3952

A.9.8.24.2	Test Requirements	3956

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3957

A.9.8.25.1	Test Purpose and Environment	3957

A.9.8.25.2	Test Requirements	3960

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3961

A.9.8.26.1	Test Purpose and Environment	3961

A.9.8.26.2	Test Requirements	3963

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3963

A.9.8.27.1	Test Purpose and Environment	3963

A.9.8.27.2	Test Requirements	3966

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3966

A.9.8.28.1	Test Purpose and Environment	3966

A.9.8.28.2	Test Requirements	3969

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3969

A.9.8.29.1	Test Purpose and Environment	3969

A.9.8.29.2	Test Requirements	3973

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3974

A.9.8.30.1	Test Purpose and Environment	3974

A.9.8.30.2	Test Requirements	3977

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3978

A.9.8.31.1	Test Purpose and Environment	3978

A.9.8.31.2	Test Requirements	3982

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3982

A.9.8.32.1	Test Purpose and Environment	3982

A.9.8.32.2	Test Requirements	3985

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3985

A.9.8.33.1	Test Purpose and Environment	3985

A.9.8.33.2	Test Requirements	3988

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3988

A.9.8.34.1	Test Purpose and Environment	3988

A.9.8.34.2	Test Requirements	3991

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3991

A.9.8.35.1	Test Purpose and Environment	3991

A.9.8.35.2	Test Requirements	3994

A.9.9	RSRP and RSRQ on the serving cell	3994

A.9.9.1	FDD Intra frequency serving cell case	3994

A.9.9.1.1	Test Purpose and Environment	3994

A.9.9.1.2	Test parameters	3994

A.9.9.1.3	Test Requirements	3997

A.9.9.2	TDD Intra frequency serving cell case	3997

A.9.9.2.1	Test Purpose and Environment	3997

A.9.9.2.2	Test parameters	3997

A.9.9.2.3	Test Requirements	3999

A.9.10	SSTD	3999

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3999

A.9.10.1.1	Test Purpose and Environment	3999

A.9.10.1.2	Test parameters	3999

A.9.10.1.3	Test Requirements	4001

A.9.10.2	Void	4001

A.9.10.3	Void	4001

A.9.10.4	Void	4001

A.9.11 RSSI	4001

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	4001

A.9.11.1.1	Test Purpose and Environment	4001

A.9.11.1.2	Test parameters	4001

A.9.11.1.3	Test Requirements	4004

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	4004

A.9.11.2.1	Test Purpose and Environment	4004

A.9.11.2.2	Test parameters	4004

A.9.12	Channel occupancy	4007

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	4007

A.9.12.1.1	Test Purpose and Environment	4007

A.9.12.1.2	Test parameters	4007

A.9.12.1.3	Test Requirements	4010

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	4010

A.9.12.2.1	Test Purpose and Environment	4010

A.9.12.2.2	Test parameters	4010

A.9.12.2.3	Test Requirements	4013

A.9.13	RS-SINR	4013

A.9.13.1	FDD Intra-Frequency Case	4013

A.9.13.1.1	Test Purpose and Environment	4013

A.9.13.1.2	Test parameters	4013

A.9.13.1.3	Test Requirements	4016

A.9.13.2	TDD Intra-Frequency Case	4016

A.9.13.2.1	Test Purpose and Environment	4016

A.9.13.2.2	Test parameters	4016

A.9.13.2.3	Test Requirements	4019

A.9.13.3	FDD—FDD Inter frequency case	4019

A.9.13.3.1	Test Purpose and Environment	4019

A.9.13.3.2	Test parameters	4019

A.9.13.3.3	Test Requirements	4022

A.9.13.4	TDD—TDD Inter frequency case	4022

A.9.13.4.1	Test Purpose and Environment	4022

A.9.13.4.2	Test parameters	4022

A.9.13.4.3	Test Requirements	4026

A.9.13.5	FDD—TDD Inter frequency case	4027

A.9.13.5.1	Test Purpose and Environment	4027

A.9.13.5.2	Test parameters	4027

A.9.13.5.3	Test Requirements	4033

A.9.13.6	TDD—FDD Inter frequency case	4033

A.9.13.6.1	Test Purpose and Environment	4033

A.9.13.6.2	Test parameters	4033

A.9.13.6.3	Test Requirements	4037

A.9.14	Channel quality reporting accuracy	4037

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4037

A.9.14.1.1	Test Purpose and Environment	4037

A.9.14.1.2	Test parameters	4038

A.9.14.1.3	Test Requirements	4038

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4039

A.9.14.2.1	Test Purpose and Environment	4039

A.9.14.2.2	Test parameters	4039

A.9.14.2.3	Test Requirements	4040

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4040

A.9.14.3.1	Test Purpose and Environment	4040

A.9.14.3.2	Test parameters	4040

A.9.14.3.3	Test Requirements	4041

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4041

A.9.14.4.1	Test Purpose and Environment	4041

A.9.14.4.2	Test parameters	4041

A.9.14.4.3	Test Requirements	4042

A.10	Proximity-based Services in Any Cell Selection State	4065

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	4065

A.10.1.1	Test Purpose and Environment	4065

A.10.1.2	Test Requirements	4066

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	4067

A.10.2.1	Test Purpose and Environment	4067

A.10.2.2	Test Requirements	4068

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	4068

A.10.3.1	Test Purpose and Environment	4068

A.10.3.2	Test Requirements	4070

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	4071

A.10.4.1	Test Purpose and Environment	4071

A.10.4.2	Test Requirements	4073

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	4073

A.11.1	V2V UE Transmission Timing Accuracy Test	4073

A.11.1.1	Test Purpose and Environment	4073

A.11.1.2	Test requirements	4074

A.11.2	Interruptions due to V2V sidelink communication	4074

A.11.2.1	Test Purpose and Environment	4074

A.11.2.2	Test Requirements	4076

A.12	4076

A.12.1	V2X UE Transmission Timing Accuracy Test	4076

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	4076

A.12.1.1.1	Test Purpose and Environment	4076

A.12.1.1.2	Test requirements	4077

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	4078

A.12.1.2.1	Test Purpose and Environment	4078

A.12.1.2.2	Test Requirements	4078

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	4079

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	4079

A.12.2.1.1	Test Purpose and Environment	4079

A.12.2.1.2	Test Requirements	4080

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	4081

A.12.2.2.1	Test Purpose and Environment	4081

A.12.2.2.2	Test Requirements	4082

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	4082

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	4082

A.12.3.1.1	Test Purpose and Environment	4082

A.12.3.1.2	Test Requirements	4085

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	4086

A.12.3.2.1	Test Purpose and Environment	4086

A.12.3.1.2	Test Requirements	4088

A.12.4	Congestion Control Measurement Test for V2X UE	4088

A.12.4.1	Test Purpose and Environment	4088

A.12.4.2	Test Requirements	4090

A.12.5	Interruptions due to V2X Sidelink Communication	4090

A.12.5.1	Test Purpose and Environment	4090

A.12.5.2	Test Requirements	4092

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	4092

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	4092

A.12.6.1.1	Test Purpose and Environment	4092

A.12.6.1.2	Test Requirements	4094

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	4094

A.12.6.2.1	Test Purpose and Environment	4094

A.12.6.1.2	Test Requirements	4096

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	4096

A.12.7.1	Test Purpose and Environment	4096

A.12.7.2	Test Requirements	4099

A.12.8	Interruptions due to V2X Carrier Aggregation	4100

A.12.8.1	Interruptions on a FDD PCell	4100

A.12.8.1.1	Test Purpose and Environment	4100

A.12.8.1.2	Test Requirements	4101

A.12.8.2	Interruptions on a TDD PCell	4101

A.12.8.2.1	Test Purpose and Environment	4101

A.12.8.2.2	Test Requirements	4103

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	4104

A.13.1	RRC_IDLE state for satellite access	4104

A.13.1.1	Cell re-selection for satellite access	4104

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	4104

A.13.1.1.1.1	Test Purpose and Environment	4104

A.13.1.1.1.2	Test Requirements	4106

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	4107

A.13.1.1.2.1	Test Purpose and Environment	4107

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	4110

A.13.1.1.3.1	Test Purpose and Environment	4110

A.13.1.1.3.2	Test Requirements	4112

A.13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage	4113

A.13.1.1.4.1	Test Purpose and Environment	4113

A.13.1.1.4.2	Test Requirements	4114

A.13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO	4115

A.13.1.1.5.1	Test Purpose and Environment	4115

A.13.1.1.5.2	Test Requirements	4116

A.13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO	4117

A.13.1.1.6.1	Test Purpose and Environment	4117

A.13.1.1.6.2	Test Requirements	4118

A.13.1.1.7	HD – FDD Intra frequency case for UE Category NB1 in in-band mode in NTN NR in normal coverage	4119

A.13.1.1.7.1	Test Purpose and Environment	4119

A.13.1.1.7.2	Test Requirements	4121

A.13.1.1.8	HD – FDD Inter-frequency case for UE Category NB1 in-band mode in NTN NR in normal coverage	4122

A.13.1.1.8.1	Test Purpose and Environment	4122

A.13.1.1.8.2	Test Requirements	4124

A.13.2	Void	4125

A.13.3	RRC connection mobility control for satellite access	4125

A.13.3.1	RRC re-establishment for satellite access	4125

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	4125

A.13.3.1.1.1	Test Purpose and Environment	4125

A.13.3.1.1.2	Test Requirements	4126

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4127

A.13.3.1.2.1	Test Purpose and Environment	4127

A.13.3.1.2.2	Test Requirements	4128

A.13.3.1.3	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4129

A.13.3.1.3.1	Test Purpose and Environment	4129

A.13.3.1.3.2	Test Requirements	4130

A.13.3.2	Random Access for Satellite Access	4130

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	4131

A.13.3.2.1.1	Test Purpose and Environment	4131

A.13.3.2.1.2	Test Requirements	4133

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	4135

A.13.3.2.2.1	Test Purpose and Environment	4135

A.13.3.2.2.2	Test Requirements	4136

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	4138

A.13.3.2.3.1	Test Purpose and Environment	4138

A.13.3.2.3.2	Test Requirements	4140

A.13.4	Timing and signalling characteristics for satellite access	4142

A.13.4.1	UE transmit timing for satellite access	4142

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	4142

A.13.4.1.1.1	Test Purpose and Environment	4142

A.13.4.1.1.2	Test Requirements	4143

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	4144

A.13.4.1.2.1	Test Purpose and Environment	4144

A.13.4.1.2.2	Test Requirements	4145

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	4146

A.13.4.1.3.1	Test Purpose and Environment	4146

A.13.4.1.3.2	Test Requirements	4147

A.13.4.2	UE timing advance for satellite access	4148

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	4148

A.13.4.2.1.1	Test Purpose and Environment	4148

A.13.4.2.1.2	Test Requirements	4150

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	4150

A.13.4.2.2.1	Test Purpose and Environment	4150

A.13.4.2.2.2	Test Requirements	4152

A.13.4.3	Radio Link Monitoring for satellite access	4152

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	4152

A.13.4.3.1.1	Test Purpose and Environment	4152

A.13.4.3.1.2	Test Requirements	4155

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	4155

A.13.4.3.2.1	Test Purpose and Environment	4155

A.13.4.3.2.2	Test Requirements	4158

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4158

A.13.4.3.3.1	Test Purpose and Environment	4158

A.13.4.3.3.2	Test Requirements	4161

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	4161

A.13.4.3.4.1	Test Purpose and Environment	4161

A.13.4.3.4.2	Test Requirements	4164

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4164

A.13.4.3.5.1	Test Purpose and Environment	4164

A.13.4.3.5.2	Test Requirements	4167

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4167

A.13.4.3.6.1	Test Purpose and Environment	4167

A.13.4.3.6.2	Test Requirements	4170

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4170

A.13.4.3.7.1	Test Purpose and Environment	4170

A.13.4.3.7.2	Test Requirements	4172

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4173

A.13.4.3.8.1	Test Purpose and Environment	4173

A.13.4.3.8.2	Test Requirements	4175

A.13.4.3.9	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 in-band mode in NTN NR in Enhanced Coverage	4176

A.13.4.3.9.1	Test Purpose and Environment	4176

A.13.4.3.9.2	Test Requirements	4179

A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access	4179

A.13.5.4	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in in-band mode in NTN NR under normal coverage for Satellite Access	4186

A.13.5.4.1	Test Purpose and Environment	4186

A.13.5.4.2	Test Requirements	4188

A.13.5.5	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in in-band mode in NTN NR under normal coverage for Satellite Access	4188

A.13.5.5.1	Test Purpose and Environment	4188

A.13.5.5.2	Test Requirements	4190

A.13.6	Measurement performance requirements for UE for satellite access	4191

A.13.6.1	Void	4191

A.13.6.2	Channel quality reporting accuracy for satellite access	4191

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4191

A.13.6.2.1.1	Test Purpose and Environment	4191

A.13.6.2.1.2	Test parameters	4191

A.13.6.2.1.3	Test Requirements	4192

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4192

A.13.6.2.2.1	Test Purpose and Environment	4192

A.13.6.2.2.2	Test parameters	4192

A.13.6.2.2.3	Test Requirements	4193

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4193

A.13.6.2.3.1	Test Purpose and Environment	4193

A.13.6.2.3.2	Test parameters	4194

A.13.6.2.3.3	Test Requirements	4194

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4195

A.13.6.2.4.1	Test Purpose and Environment	4195

A.13.6.2.4.2	Test parameters	4195

A.13.6.2.4.3	Test Requirements	4196

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	4196

A.13.6.2.5.1	Test Purpose and Environment	4196

A.13.6.2.5.2	Test parameters	4196

A.13.6.2.5.3	Test Requirements	4197

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	4198

A.13.6.2.6.1	Test Purpose and Environment	4198

A.13.6.2.6.2	Test parameters	4198

A.13.6.2.6.3	Test Requirements	4199

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	4199

A.14.1	RRC_IDLE state for satellite access	4199

A.14.1.1	Cell re-selection for satellite access	4199

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4199

A.14.1.1.1.1	Test Purpose and Environment	4199

A.14.1.1.1.2	Test Requirements	4201

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4202

A.14.1.1.2.1	Test Purpose and Environment	4202

A.14.1.1.2.2	Test Requirements	4204

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4205

A.14.1.1.3.1	Test Purpose and Environment	4205

A.14.1.1.3.2	Test Requirements	4207

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4208

A.14.1.1.4.1	Test Purpose and Environment	4208

A.14.1.1.4.2	Test Requirements	4210

A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4211

A.14.1.1.5.1	Test Purpose and Environment	4211

A.14.1.1.5.2	Test Requirements	4213

A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4214

A.14.1.1.6.1	Test Purpose and Environment	4214

A.14.1.1.6.2	Test Requirements	4216

A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering	4217

A.14.1.1.7.1	Test Purpose and Environment	4217

A.14.1.1.7.2	Test Requirements	4218

A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering	4219

A.14.1.1.8.1	Test Purpose and Environment	4219

A.14.1.1.8.2	Test Requirements	4220

A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering	4221

A.14.1.1.9.1	Test Purpose and Environment	4221

A.14.1.1.9.2	Test Requirements	4222

A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering	4223

A.14.1.1.10.1	Test Purpose and Environment	4223

A.14.1.1.10.2	Test Requirements	4224

A.14.2	RRC_CONNECTED state mobility for satellite access	4225

A.14.2.1	E-UTRAN handover for satellite access	4225

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4225

A.14.2.1.1.1	Test Purpose and Environment	4225

A.14.2.1.1.2	Test Requirements	4227

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4228

A.14.2.1.2.1	Test Purpose and Environment	4228

A.14.2.1.2.2	Test Requirements	4229

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4230

A.14.2.1.3.1	Test Purpose and Environment	4230

A.14.2.1.3.2	Test Requirements	4231

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4232

A.14.2.1.4.1	Test Purpose and Environment	4232

A.14.2.1.4.2	Test Requirements	4233

A.14.2.1.5	E-UTRAN FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4234

A.14.2.1.5.1	Test Purpose and Environment	4234

A.14.2.1.5.2	Test Requirements	4235

14.2.1.6	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4236

A.14.2.1.6.1	Test Purpose and Environment	4236

A.14.2.1.6.2	Test Requirements	4237

A.14.2.1.7	E-UTRAN FD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4238

A.14.2.1.7.1	Test Purpose and Environment	4238

A.14.2.1.7.2	Test Requirements	4239

A.14.2.1.8	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4240

A.14.2.1.8.1	Test Purpose and Environment	4240

A.14.2.1.8.2	Test Requirements	4241

A.14.2.1.9	E-UTRAN FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4242

A.14.2.1.9.1	Test Purpose and Environment	4242

A.14.2.1.9.2	Test Requirements	4243

A.14.2.1.10	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4244

A.14.2.1.10.1	Test Purpose and Environment	4244

A.14.2.1.10.2	Test Requirements	4245

A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4246

A.14.2.1.11.1	Test Purpose and Environment	4246

A.14.2.1.12.2	Test Requirements	4247

A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4248

A.14.2.1.12.1	Test Purpose and Environment	4248

A.14.2.1.12.2	Test Requirements	4249

A.14.2.1.15	E-UTRAN FDD-FDD Inter frequency location based conditional handover for Cat-M1 UEs in CEModeA	4254

A.14.2.1.15.1	Test Purpose and Environment	4254

A.14.2.1.16.2	Test Requirements	4255

A.14.2.1.16	E-UTRAN HD-FDD Inter frequency time based conditional handover for Cat-M1 UEs in CEModeA	4256

A.14.2.1.16.1	Test Purpose and Environment	4256

A.14.2.1.16.2	Test Requirements	4257

A.14.3	RRC connection mobility control for satellite access	4258

A.14.3.1	RRC re-establishment for satellite access	4258

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4258

A.14.3.1.1.1	Test Purpose and Environment	4258

A.14.3.1.1.2	Test Requirements	4259

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	4260

A.14.3.1.2.1	Test Purpose and Environment	4260

A.14.3.1.2.2	Test Requirements	4262

A.14.3.1.3	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4263

A.14.3.1.3.1	Test Purpose and Environment	4263

A.14.3.1.3.2	Test Requirements	4265

A.14.3.1.4	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4266

A.14.3.1.4.1	Test Purpose and Environment	4266

A.14.3.1.4.2	 Test Requirements	4268

A.14.3.2	Random access for satellite access	4269

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4269

A.14.3.2.1.1	Test Purpose and Environment	4269

A.14.3.2.1.2	Test Requirements	4271

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4272

A.14.3.2.2.1	Test Purpose and Environment	4272

A.14.3.2.2.2	Test Requirements	4274

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4276

A.14.3.2.3.1	Test Purpose and Environment	4276

A.14.3.2.3.2	Test Requirements	4278

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4279

A.14.3.2.4.1	Test Purpose and Environment	4279

A.14.3.2.4.2	Test Requirements	4281

A.14.4	Timing and signalling characteristics for satellite access	4283

A.14.4.1	UE transmit timing for satellite access	4283

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4283

A.14.4.1.1.1	Test Purpose and Environment	4283

A.14.4.1.1.2	Test Requirements	4285

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4286

A.14.4.1.2.1	Test Purpose and Environment	4286

A.14.4.1.2.2	Test Requirements	4287

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4288

A.14.4.1.3.1	Test Purpose and Environment	4288

A.14.4.1.3.2	Test Requirements	4289

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4290

A.14.4.1.4.1	Test Purpose and Environment	4290

A.14.4.1.4.2	Test Requirements	4291

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	4292

A.14.4.1.5.1	Test Purpose and Environment	4292

A.14.4.1.5.2	Test Requirements	4293

A.14.4.2	UE timing advance for satellite access	4294

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4294

A.14.4.2.1.1	Test Purpose and Environment	4294

A.14.4.2.1.2	Test Requirements	4296

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4296

A.14.4.2.2.1	Test Purpose and Environment	4296

A.14.4.2.2.2	Test Requirements	4299

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4299

A.14.4.2.3.1	Test Purpose and Environment	4299

A.14.4.2.3.2	Test Requirements	4300

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4300

A.14.4.2.4.1	Test Purpose and Environment	4300

A.14.4.2.4.2	Test Requirements	4302

A.14.4.3	Radio Link Monitoring for satellite access	4303

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4303

A.14.4.3.1.1	Test Purpose and Environment	4303

A.14.4.3.1.2	Test Requirements	4306

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4306

A.14.4.3.2.1	Test Purpose and Environment	4306

A.14.4.3.2.2	Test Requirements	4309

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4309

A.14.4.3.3.1	Test Purpose and Environment	4309

A.14.4.3.3.2	Test Requirements	4312

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4312

A.14.4.3.4.1	Test Purpose and Environment	4312

A.14.4.3.4.2	Test Requirements	4315

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4315

A.14.4.3.5.1	Test Purpose and Environment	4315

A.14.4.3.5.2	 Test Requirements	4318

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4318

A.14.4.3.6.1	Test Purpose and Environment	4318

A.14.4.3.6.2	Test Requirements	4321

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4321

A.14.4.3.7.1	Test Purpose and Environment	4321

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4324

A.14.4.3.8.1 	Test Purpose and Environment	4324

A.14.4.3.8.2	Test Requirements	4327

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	4327

A.14.5.1 Intra-frequency measurements for satellite access	4327

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4327

A.14.5.1.1.1	Test Purpose and Environment	4327

A.14.5.1.1.2	Test Requirements	4329

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4330

A.14.5.1.2.1	Test Purpose and Environment	4330

A.14.5.1.2.2	Test Requirements	4332

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4332

A.14.5.1.3.1	Test Purpose and Environment	4332

A.14.5.1.3.2	Test Requirements	4334

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4335

A.14.5.1.4.1	Test Purpose and Environment	4335

A.14.5.1.4.2	Test Requirements	4337

A.14.5.1.5	E-UTRAN FD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4337

A.14.5.1.5.1	Test Purpose and Environment	4337

A.14.5.1.5.2	Test Requirements	4339

A.14.5.1.6 	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4340

A.14.5.1.6.1	Test Purpose and Environment	4340

A.14.5.1.6.2	Test Requirements	4341

A.14.5.1.7	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4342

A.14.5.1.7.1	Test Purpose and Environment	4342

A.14.5.1.7.2	Test Requirements	4343

A.14.5.2 Inter-frequency measurements for satellite access	4344

A.14.5.2.1	E-UTRAN FD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4344

A.14.5.2.1.1	Test Purpose and Environment	4344

A.14.5.2.1.2	Test Requirements	4345

A.14.5.2.2	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4346

A.14.5.2.2.1	Test Purpose and Environment	4346

A.14.5.2.2.2	Test Requirements	4347

A.14.5.2.3	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeB when DRX is used with time-based triggering	4348

A.14.5.2.3.1	Test Purpose and Environment	4348

A.14.5.2.3.2	Test Requirements	4349

A.14.5.2.4	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4350

A.14.5.2.4.1	Test Purpose and Environment	4350

A.14.5.2.4.2	Test Requirement	4351

A.14.5.2.5	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	4352

A.14.5.2.5.1	Test Purpose and Environment	4352

A.14.5.2.5.2	Test Requirement	4354

A.14.5.2.6	E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4354

A.14.5.2.6.1	Test Purpose and Environment	4354

A.14.5.2.6.2	Test Requirement	4356

A.14.5.2.7	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA in DRX	4356

A.14.5.2.7.1	Test Purpose and Environment	4356

A.14.5.2.7.2	Test Requirement	4358

A.14.5.2.8	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4358

A.14.5.2.8.1	Test Purpose and Environment	4358

A.14.5.2.8.2	Test Requirement	4360

A.14.5.2.9	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4361

A.14.5.2.9.1	Test Purpose and Environment	4361

A.14.5.2.9.2	Test Requirement	4362

A.14.6	Measurement performance requirements for UE for satellite access	4363

A.14.6.1	RSRP for satellite access	4363

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4363

A.14.6.1.1.1	Test Purpose and Environment	4363

A.14.6.1.1.2	Test parameters	4363

A.14.6.1.1.3	Test Requirements	4365

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4365

A.14.6.1.2.1	Test Purpose and Environment	4365

A.14.6.1.2.2	Test parameters	4365

A.14.6.1.2.3	Test Requirements	4367

A.14.6.1.3	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4367

A.14.6.1.3.1	Test Purpose and Environment	4367

A.14.6.1.3.2	Test parameters	4367

A.14.6.1.3.3	Test Requirements	4368

A.14.6.1.4	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4369

A.14.6.1.4.1	Test Purpose and Environment	4369

A.14.6.1.4.2	Test parameters	4369

A.14.6.1.4.3	Test Requirements	4370

A.14.6.1.5	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4370

A.14.6.1.5.1	Test Purpose and Environment	4370

A.14.6.1.5.2	Test parameters	4370

A.14.6.1.5.3	Test Requirements	4371

A.14.6.1.6	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4371

A.14.6.1.6.1	Test Purpose and Environment	4371

A.14.6.1.6.2	Test parameters	4372

A.14.6.1.6.3	Test Requirements	4373

A.14.6.2	Channel quality reporting accuracy for satellite access	4373

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4373

A.14.6.2.1.1	Test Purpose and Environment	4373

A.14.6.2.1.2	Test parameters	4373

A.14.6.2.1.3	Test Requirements	4374

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4375

A.14.6.2.2.1	Test Purpose and Environment	4375

A.14.6.2.2.2	Test parameters	4375

A.14.6.2.2.3	Test Requirements	4376

A.14.6.2.3.2	Test parameters	4376

A.14.6.2.3.3	Test Requirements	4377

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	4378

A.14.6.2.4.1	Test Purpose and Environment	4378

A.14.6.2.4.2	Test parameters	4378

A.14.6.2.4.3	Test Requirements	4379

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	4380

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	4380

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	4380

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	4380

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	4380

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	4382

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	4383

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	4383

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	4383

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	4383

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	4385

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	4386

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4386

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	4387

B.1.11	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1 for satellite access	4388

B.1.12	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4388

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	4388

B.2.1	Conditions for E-UTRAN intra-frequency measurements	4388

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	4389

B.2.3	Conditions for E-UTRAN inter-frequency measurements	4389

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	4390

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	4390

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	4390

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	4391

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	4391

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4392

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	4392

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	4392

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	4392

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	4393

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4393

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4393

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	4394

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	4394

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4394

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4395

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	4395

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	4396

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	4397

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	4398

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	4399

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	4400

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	4400

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	4400

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	4400

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	4401

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	4401

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	4403

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4403

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4403

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	4403

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	4404

B.2.28	Conditions for E-UTRAN inter-frequency measurements by UE Category M1 for satellite access	4405

B.2.29	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4405

B.2.30	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4406

B.3	Conditions for measurements performance requirements for UE	4406

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	4406

B.3.2	Void	4406

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	4407

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	4407

B.3.5	Conditions for UE Rx – Tx time difference	4407

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	4407

B.3.7	Conditions for inter-frequency RSTD measurements	4407

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	4407

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	4408

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	4408

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4408

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4408

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4408

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4408

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4408

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4409

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4409

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4409

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4409

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4410

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4410

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4410

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4410

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4410

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4410

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4411

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4411

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4411

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4411

B.3.21.1	Conditions for RSRP measurements	4411

B.3.21.2	Conditions for RSRQ measurements	4411

B.3.21.3	Conditions for CSI-RSRP measurements	4411

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4412

B.3.22.1	Conditions for RSRP measurements	4412

B.3.22.2	Void	4412

B.3.22.3	Conditions for CSI-RSRP measurements	4412

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4412

B.3.23.1	Conditions for RSRP measurements	4412

B.3.23.2	Conditions for RSRQ measurements	4413

B.3.23.3	Conditions for CSI-RSRP measurements	4413

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4413

B.3.24.1	Conditions for RSRP measurements	4413

B.3.24.2	Conditions for RSRQ measurements	4413

B.3.24.3	Conditions for CSI-RSRP measurements	4413

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4413

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4414

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4414

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4414

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4415

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4415

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4415

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4415

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4415

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4415

B.4	RRM Requirements Exceptions	4416

B.4.1	General	4416

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4416

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4416

B.4.3.1	Inter-band carrier aggregation	4416

B.4.3.2	Intra-band non-contiguous carrier aggregation	4416

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4416

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4417

B.5.1	Conditions for S-RSRP Accuracy Requirements	4417

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4417

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4417

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4418

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4418

B.6	Conditions for V2X	4419

B.6.1	Test parameters for GNSS signals	4419

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4419

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4419

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4419

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4420

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4420

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4420

B.8	High level test procedure for SAN RRM tests	4422

Annex C (informative):	Change history:	4422