| 3GPP TS 36.133 V19.3.0 (2025-12) |  |
| --- | --- |
| Technical Specification¦Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management (Release 19) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	107

1	Scope	109

2	References	109

3	Definitions, symbols and abbreviations	111

3.1	Definitions	111

3.2	Symbols	113

3.3	Abbreviations	114

3.4	Test tolerances	117

3.5	Additional notation	117

3.5.1	Groups of bands	117

3.5.1A	Groups of bands for satellite access	120

3.6	General	121

3.6.1	Applicability of requirements in this specification version	121

3.6.1.1	Applicability of requirements for UE capable of network-based CRS interference mitigation	127

3.6.1.2	Applicability of requirements with CRS muting for category M1 UE capable of CRS muting	129

3.6.1.3	Applicability of requirements with CRS muting for category M2 UE capable of CRS muting	130

3.6.2	Applicability of requirements for EN-DC operation	131

3.6.3	Applicability of requirements for NE-DC operation	132

3.6.4	Applicability of requirements for NGEN-DC operation	133

3.6.5	Applicability of 2-step RA and 4-step RA in RRM requirements	133

3.6.6 Applicability of requirements for UE category NB-IoT for frame structure type 1 for NTN-TDD	133

3.6.7	Applicability of NB-IoT inband operation in NTN NR	133

Rel-18 UEs supporting only standalone operation do not need to be tested for in-band operation with NR over NTN. In-band operation with NR NTN is not supported in Rel-17.	133

4	E-UTRAN RRC_IDLE state mobility	133

4.1	Cell Selection	133

4.2	Cell Re-selection	134

4.2.1	Introduction	134

4.2.2	Requirements	134

4.2.2.1	Measurement and evaluation of serving cell	135

4.2.2.2	Void	136

4.2.2.3	Measurements of intra-frequency E-UTRAN cells	136

4.2.2.4	Measurements of inter-frequency E-UTRAN cells	138

4.2.2.5	Measurements of inter-RAT cells	140

4.2.2.5.1	Measurements of UTRAN FDD cells	140

4.2.2.5.2	Measurements of UTRAN TDD cells	142

4.2.2.5.3	Measurements of GSM cells	144

4.2.2.5.4	Measurements of HRPD cells	145

4.2.2.5.5	Measurements of cdma2000 1X	146

4.2.2.5.6	Measurements of NR cells	147

4.2.2.5.7	Measurements of NR cells subject to CCA	150

4.2.2.5.8	Measurements of NR cells for RedCap	152

4.2.2.6	Evaluation of cell re-selection criteria	154

4.2.2.7	Maximum interruption in paging reception	154

4.2.2.8	void	155

4.2.2.9	UE measurement capability	155

4.2.2.9a	UE measurement capability (Increased UE carrier monitoring)	156

4.2.2.10	Reselection to CSG cells	156

4.2.2.10.1	Reselection from a non CSG to an inter-frequency CSG cell	156

4.2.2.10.2	Reselection from a non CSG to an inter-RAT UTRAN FDD CSG cell	157

4.2.2.11	Void	158

4.2.2.12	Void	158

4.2.2.13	Void	158

4.2.2.14	UE measurement capability for RedCap	158

4.3	Minimization of Drive Tests (MDT)	159

4.3.1	Introduction	159

4.3.2	Measurements	159

4.3.2.1	Requirements	159

4.3.3	Relative Time Stamp Accuracy	159

4.3.3.1	Requirements	159

4.3.4	Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	160

4.3.4.1	 Requirements	160

4.3.5	Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	160

4.3.5.1	Requirements for timeSinceFailure	160

4.4	MBSFN Measurements	160

4.4.1	Introduction	160

4.4.2	MBSFN RSRP measurements	160

4.4.3	MBSFN RSRQ measurements	161

4.4.4	MCH BLER measurements	161

4.5	Proximity-based Services	161

4.5.1	Introduction	161

4.5.2	Requirements	161

4.5.2.1	Interruptions with ProSe Direct Discovery	161

4.5.2.2	Interruptions with ProSe Direct Communication	161

4.5.2.3	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	162

4.5.2.4	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	162

4.6	Cell Selection and Re-selection Requirements for UE category NB1	163

4.6.1	Cell Selection	163

4.6.2	Cell Re-selection	163

4.6.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	163

4.6.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	164

4.6.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	165

4.6.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	166

4.6.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	168

4.6.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	169

4.6.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	170

4.6.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	171

4.6.2.7	Maximum interruption in paging reception in normal coverage	173

4.6.2.7A	Maximum interruption in paging reception in enhanced coverage	173

4.6.2.8	UE measurement capability	173

4.6.2.9	WUS receptions for NB1	173

4.6.3	Requirements for transmission using preconfigured uplink resources for UE category NB1	174

4.6.3.1	Introduction	174

4.6.3.2	Requirements on UE synchronization for transmission using PUR	174

4.6.3.3	Requirements on TA validation for transmission using PUR	174

4.6A	Cell Selection and Re-selection Requirements for UE category NB-IoT for Satellite Access	175

4.6A.1	Cell Selection	175

4.6A.2	Cell Re-selection for UE category NB-IoT for Satellite Access	175

4.6A.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	176

4.6A.2.1A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in normal coverage when configured with WUS	177

4.6A.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	178

4.6A.2.3	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in enhanced coverage	180

4.6A.2.3A	Measurement and evaluation of serving NB-IoT cell for HD-FDD UE category NB1 in enhanced coverage when configured with WUS	182

4.6A.2.4	Measurements of intra-frequency NB-IoT cells for UE category NB1 in enhanced coverage	183

4.6A.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	185

4.6A.2.6	Measurements of inter-frequency NB-IoT cells for UE category NB1 in enhanced coverage	187

4.6A.2.7	Maximum interruption in paging reception in normal coverage	189

4.6A.2.7A	Maximum interruption in paging reception in enhanced coverage	189

4.6A.2.8	UE measurement capability	190

4.6A.2.9	WUS receptions for NB1	190

4.6A.3	Requirements for transmission using preconfigured uplink resources for UE category NB-IoT for Satellite Access	191

4.6A.3.1	Introduction	191

4.6A.3.2	Requirements on UE synchronization for transmission using PUR	191

4.6A.3.3	Requirements on TA validation for transmission using PUR	191

4.6B	Cell Selection and Re-selection Requirements for UE category NB-IoT for frame structure type 1 for NTN-TDD	191

4.6B.1	Cell Selection	191

4.6B.2	Cell Re-selection for UE category NB-IoT for Satellite Access	191

4.6B.2.1	Measurement and evaluation of serving NB-IoT cell for UE category NB1 in normal coverage	192

4.6B.2.2	Measurements of intra-frequency NB-IoT cells for UE category NB1 in normal coverage	193

4.6B.2.5	Measurements of inter-frequency NB cells for UE category NB1 in normal coverage	194

4.6B.2.7	Maximum interruption in paging reception in normal coverage	195

4.6B.2.8	UE measurement capability	195

4.7	Cell Selection and Re-selection Requirements for UE category M1	196

4.7.1	Cell Selection	196

4.7.2	Cell Re-selection	196

4.7.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	196

4.7.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	196

4.7.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	197

4.7.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	198

4.7.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	199

4.7.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	201

4.7.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	201

4.7.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	202

4.7.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	202

4.7.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	203

4.7.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	204

4.7.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	207

4.7.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	208

4.7.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	209

4.7.2.3	WUS receptions for UE category M1	209

4.7.3	Channel quality report for UE Category M1 in idle mode	209

4.7.4	Requirements for transmission using preconfigured uplink resources for UE category M1	210

4.7.4.1	Introduction	210

4.7.4.2	Requirements on UE synchronization for transmission using PUR	210

4.7.4.3	Requirements on TA validation for transmission using PUR	210

4.7A	Cell Selection and Re-selection Requirements for UE category M1 for Satellite Access	211

4.7A.1	Cell Selection	211

4.7A.2	Cell Re-selection for UE category M1 for Satellite Access	211

4.7A.2.1	Cell Re-selection requirements for UE category M1 in normal coverage	212

4.7A.2.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	212

4.7A.2.1.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in normal coverage	214

4.7A.2.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	215

4.7A.2.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	218

4.7A.2.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	220

4.7A.2.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	220

4.7A.2.2	Cell Re-selection requirements for UE category M1 in enhanced coverage	221

4.7A.2.2.1	Measurement and evaluation of serving cell for UE category M1 in enhanced coverage	221

4.7A.2.2.1A	Relaxed measurement and evaluation of serving cell for UE category M1 in enhaned coverage	222

4.7A.2.2.2	Measurements of intra-frequency cells for UE category M1 in enhanced coverage	224

4.7A.2.2.3	Measurements of inter-frequency cells for UE category M1 in enhanced coverage	226

4.7A.2.2.4	Maximum allowed layers for multiple monitoring for UE category M1 in enhanced coverage	229

4.7A.2.2.5	Maximum interruption in paging reception for Category M1 UEs in enhanced coverage	229

4.7A.2.3	WUS receptions for UE category M1	230

4.7A.3	Channel quality report for UE Category M1 in idle mode for Satellite Access	230

4.7A.4	Requirements for transmission using preconfigured uplink resources for UE category M1 for Satellite Access	231

4.7A.4.1	Introduction	231

4.7A.4.2	Requirements on UE synchronization for transmission using PUR	231

4.7A.4.3	Requirements on TA validation for transmission using PUR	231

4.8	Idle State Positioning Measurement Requirements for UE category NB1	231

4.8.1	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for normal coverage	231

4.8.1.1	RSTD Measurement Reporting Delay	233

4.8.2	OTDOA Intra-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	233

4.8.2.1	RSTD Measurement Reporting Delay	234

4.8.3	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for normal coverage	235

4.8.3.1	RSTD Measurement Reporting Delay	236

4.8.4	OTDOA Inter-Frequency RSTD Measurements for UE category NB1 for enhanced coverage	237

4.8.4.1	RSTD Measurement Reporting Delay	238

4.8.5	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	239

4.8.5.1	Measurement Reporting Delay	240

4.8.6	Intra-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	240

4.8.6.1	Measurement Reporting Delay	241

4.8.7	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for normal coverage	242

4.8.7.1	Measurement Reporting Delay	243

4.8.8	Inter-Frequency E-CID NRSRP and NRSRQ Measurements for UE category NB2 for enhanced coverage	243

4.8.8.1	Measurement Reporting Delay	244

4.9	Idle Mode CA Measurement	245

4.9.1	Introduction	245

4.9.2	Requirement	245

4.9.2.1	Detected cell requirement during state transition and Idle mode	245

4.9.2.2	Measurements of inter-frequency CA candidate cells	246

4.9.2.3	Measurements on serving cell	246

4A	E-UTRAN RRC_INACTIVE state mobility	247

4A.1	Cell Re-selection	247

4A.1.1	Introduction	248

4A.1.2	Requirements	248

4A.1.2.1	UE measurement capability	248

4A.1.2.2	Measurement and evaluation of serving cell	248

4A.1.2.3	Measurements of intra-frequency E-UTRAN cells	248

4A.1.2.4	Measurements of inter-frequency E-UTRAN cells	248

4A.1.2.5	Evaluation of cell re-selection criteria	248

4A.1.2.6	Maximum interruption in paging reception	248

4A.1.2.7	Measurements of inter-RAT NR cells	248

4A.1.2.8	UE measurement capability for RedCap	248

4A.1.2.9	Measurements of inter-RAT NR cells for RedCap	248

4A.2	Requirements for UE Category M1	249

4A.2.1	Introduction	249

4A.2.2	Cell Selection	249

4A.2.3	Cell Reselection	249

4A.2.3.1	Cell Re-selection requirements for UE category M1 in normal coverage	249

4A.2.3.1.1	Measurement and evaluation of serving cell for UE category M1 in normal coverage	249

4A.2.3.1.2	Measurements of intra-frequency cells for UE category M1 in normal coverage	250

4A.2.3.1.3	Measurements of inter-frequency cells for UE category M1 in normal coverage	250

4A.2.3.1.4	Maximum allowed layers for multiple monitoring for UE category M1 in normal coverage	250

4A.2.3.1.5	Maximum interruption in paging reception for Category M1 UEs in normal coverage	250

4A.2.4	Channel quality report for UE Category M1 in idle mode	252

5	E-UTRAN RRC_CONNECTED state mobility	252

5.1	E-UTRAN Handover	253

5.1.1	Introduction	253

5.1.2	Requirements	253

5.1.2.1	E-UTRAN FDD – FDD	253

5.1.2.1.1	Handover delay	253

5.1.2.1.2	Interruption time	253

5.1.2.2	E-UTRAN FDD – TDD	254

5.1.2.2.1	(Void)	255

5.1.2.2.2	(Void)	255

5.1.2.3	E-UTRAN TDD – FDD	255

5.1.2.3.1	(Void)	255

5.1.2.3.2	(Void)	255

5.1.2.4	E-UTRAN TDD – TDD	255

5.1.2.4.1	Handover delay	255

5.1.2.4.2	Interruption time	255

5.1.2.5	E-UTRAN HD–FDD	257

5.1.2.5.1	Handover delay	257

5.1.2.5.2	Interruption time	257

5.1.2.6	E-UTRAN FDD – FDD conditional handover	258

5.1.2.6.1	Handover delay	258

5.1.2.6.2	Measurement time	259

5.1.2.6.3	Preparation time	259

5.1.2.6.4	Interruption time	259

5.1.2.7	E-UTRAN FDD – TDD conditional handover	259

5.1.2.8	E-UTRAN TDD – FDD conditional handover	260

5.1.2.9	E-UTRAN TDD – TDD conditional handover	260

5.2	Void	260

5.3	Handover to other RATs	260

5.3.1	E-UTRAN - UTRAN FDD Handover	260

5.3.1.1	Introduction	260

5.3.1.1.1	Handover delay	260

5.3.1.1.2	Interruption time	260

5.3.2	E-UTRAN - UTRAN TDD Handover	261

5.3.2.1	Introduction	261

5.3.2.2	Requirements	261

5.3.2.2.1	Handover delay	261

5.3.2.2.2	Interruption time	261

5.3.3	E-UTRAN - GSM Handover	262

5.3.3.1	Introduction	262

5.3.3.2	Requirements	262

5.3.3.2.1	Handover delay	262

5.3.3.2.2	Interruption time	262

5.3.4	E-UTRAN - NR FR1 Handover	262

5.3.4.1	Introduction	262

5.3.4.2	Handover delay	263

5.3.4.3	Interruption time	263

5.3.4A	E-UTRAN - NR FR1 Handover to target cell using CCA	263

5.3.4A.1	Introduction	263

5.3.4A.2	Handover delay	264

5.3.4A.3	Interruption time	264

5.3.4B	E-UTRAN - NR FR1 Handover for RedCap	265

5.3.4B.1	Introduction	265

5.3.4B.2	Requirements	265

5.3.5	E-UTRAN - NR FR2 Handover	265

5.3.5.1	Introduction	265

5.3.5.2	Handover delay	265

5.3.5.3	Interruption time	265

5.4	Handover to Non-3GPP RATs	266

5.4.1	E-UTRAN – HRPD Handover	266

5.4.1.1	Introduction	266

5.4.1.1.1	Handover delay	266

5.4.1.1.2	Interruption time	266

5.4.2	E-UTRAN – cdma2000 1X Handover	267

5.4.2.1	Introduction	267

5.4.2.1.1	Handover delay	267

5.4.2.1.2	Interruption time	267

5.5	E-UTRAN Handover for Cat-M1 UEs	267

5.5.1	Introduction	267

5.5.2	Requirements in CEModeA	268

5.5.2.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	268

5.5.2.1.1	Handover delay	268

5.5.2.1.2	Interruption time	268

5.5.2.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	268

5.5.2.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	269

5.5.2.3.1	Void	269

5.5.2.3.2	Void	269

5.5.3	Requirements in CEModeB	269

5.5.3.1	E-UTRAN FDD – FDD for Cat-M1 FDD UEs	269

5.5.3.1.1	Handover delay	269

5.5.3.1.2	Interruption time	269

5.5.3.2	E-UTRAN FDD – FDD for Cat-M1 HD – FDD UEs	270

5.5.3.3	E-UTRAN TDD – TDD for Cat-M1 TDD UEs	270

5.5A	E-UTRAN Handover for Cat-M1 UEs for Satellite Access	270

5.5A.1	Introduction	270

5.5A.2	Requirements in CEModeA	270

5.5A.2.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	270

5.5A.2.1.1	Handover delay	270

5.5A.2.1.2	Interruption time	270

5.5A.2.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	271

5.5A.2.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	271

5.5A.2.3.1	Handover delay	271

5.5A.2.3.2	Measurement time	272

5.5A.2.3.3	Preparation time	273

5.5A.2.3.4	Interruption time	273

5.5A.2.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	274

5.5A.3	Requirements in CEModeB	274

5.5A.3.1	E-UTRAN FDD – FDD HO for Cat-M1 FDD UEs	274

5.5A.3.1.1	Handover delay	274

5.5A.3.1.2	Interruption time	274

5.5A.3.2	E-UTRAN FDD – FDD HO for Cat-M1 HD – FDD UEs	275

5.5A.3.3	E-UTRAN FDD – FDD conditional HO for Cat-M1 FDD UEs	275

5.5A.3.3.1	Handover delay	275

5.5A.3.3.2	Measurement time	276

5.5A.3.3.3	Preparation time	277

5.5A.3.3.4	Interruption time	277

5.5A.3.4	E-UTRAN FDD – FDD conditional HO for Cat-M1 HD – FDD UEs	278

5.6	Void	278

5.7	E-UTRAN DAPS Handover	278

5.7.1	Introduction	278

5.7.2	Requirements	279

5.7.2.1	E-UTRAN FDD – FDD	279

5.7.2.1.1	DAPS Handover delay	279

5.7.2.1.2	Interruption time	279

5.7.2.2	E-UTRAN FDD – TDD	280

5.7.2.3	E-UTRAN TDD – FDD	280

5.7.2.4	E-UTRAN TDD – TDD	280

5.8	EN-DC Handover with PSCell	280

5.8.1	Introduction	280

5.8.1.1	Handover with PSCell Interruption time	280

5.8.1.2	Handover with PSCell - NR PSCell Change Delay requirements	281

5.9	EN-DC Handover with PSCell using CCA	282

5.9.1	Introduction	282

5.9.1.1	Handover with PSCell – E-UTRA HO Interruption time	282

5.9.1.2	Handover with PSCell - NR PSCell Change Delay requirements	282

6	RRC Connection Mobility Control	283

6.1	RRC Re-establishment	283

6.1.1	Introduction	283

6.1.2	Requirements	283

6.1.2.1	UE Re-establishment delay requirement	284

6.2	Random Access	284

6.2.1	Introduction	284

6.2.2	Requirements	284

6.2.2.1	Contention based random access	284

6.2.2.1.1	Correct behaviour when receiving Random Access Response reception	284

6.2.2.1.2	Correct behaviour when not receiving Random Access Response reception	285

6.2.2.1.3	Correct behaviour when receiving a NACK on msg3	285

6.2.2.1.4	Void	285

6.2.2.1.5	Correct behaviour when receiving a message over Temporary C-RNTI	285

6.2.2.1.6	Correct behaviour when contention Resolution timer expires	285

6.2.2.2	Non-Contention based random access	285

6.2.2.2.1	Correct behaviour when receiving Random Access Response	285

6.2.2.2.2	Correct behaviour when not receiving Random Access Response	285

6.2.3	Requirements for Cat-M1 UEs	285

6.2.3A	Random Access Requirements for Cat-M1 UEs for Satellite Access	286

6.2.4A	Random Access Requirements for Cat-M1 UEs with CB-Msg3 EDT for Satellite Access	286

6.2.4A.1	Correct behaviour when transmitting CB-Msg3	286

6.2.4A.2	Correct behaviour when receiving CB-Msg4	286

6.2.4A.3	Correct behaviour when not receiving CB-Msg4	286

6.3	RRC Connection Release with Redirection	286

6.3.1	Introduction	286

6.3.2	Requirements	287

6.3.2.1	RRC connection release with redirection to UTRAN FDD	287

6.3.2.2	RRC connection release with redirection to GERAN	287

6.3.2.3	RRC connection release with redirection to UTRAN TDD	287

6.3.2.4	RRC connection release with redirection to NR	288

6.3.2.5	RRC connection release with redirection to NR carrier subject to CCA	289

6.3.2.6	RRC connection release with redirection to NR Redcap	290

6.4	CSG Proximity Indication for E-UTRAN and UTRAN	291

6.4.1	Introduction	291

6.4.2	Requirements	291

6.5	RRC Re-establishment for NB-IoT UEs	291

6.5.1	Introduction	291

6.5.2	Requirements	292

6.5.2.1	UE Re-establishment delay requirement in normal coverage	292

6.5.2.2	UE Re-establishment delay requirement in enhanced coverage	292

6.5A	RRC Re-establishment for NB-IoT UEs for Satellite Access	293

6.5A.1	Introduction	293

6.5A.2	Requirements	293

6.5A.2.1	UE Re-establishment delay requirement in normal coverage	293

6.5A.2.2	UE Re-establishment delay requirement in enhanced coverage	294

6.5B	RRC Re-establishment for NB-IoT UEs for frame structure type 1 for NTN-TDD	294

6.5B.1	Introduction	294

6.5B.2	Requirements	295

6.5B.2.1	UE Re-establishment delay requirement in normal coverage	295

6.6	Random Access for UE category NB1	295

6.6.1	Introduction	295

6.6.2	Requirements	296

6.6.2.1	Correct behaviour when receiving Random Access Response reception	296

6.6.2.2	Correct behaviour when not receiving Random Access Response reception	296

6.6.2.3	Correct behaviour when receiving a NACK on msg3	296

6.6.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	296

6.6.2.5	Correct behaviour when contention Resolution timer expires	296

6.6.2.6	MSG3-based channel quality report for UE Category NB1	296

6.6.3	Requirements for NPRACH configuration	297

6.6A	Random Access for UE category NB-IoT for Satellite Access	297

6.6A.1	Introduction	297

6.6A.2	Requirements	297

6.6A.2.1	Correct behaviour when receiving Random Access Response reception	298

6.6A.2.2	Correct behaviour when not receiving Random Access Response reception	298

6.6A.2.3	Correct behaviour when receiving a NACK on msg3	298

6.6A.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	298

6.6A.2.5	Correct behaviour when contention Resolution timer expires	298

6.6A.2.6	MSG3-based channel quality report for UE Category NB1	298

6.6A.3	Requirements for NPRACH configuration	299

6.6A.4	Requirements for CB-Msg3-EDT procedure	299

6.6A.4.1 Correct behaviour when transmitting CB-Msg3	299

6.6A.4.2	Correct behaviour when receiving a CB-Msg4 over CB-RNTI	299

6.6A.4.3	Correct behaviour when detecting CB-Msg3-EDT failure	300

6.6A.4.4	MSG3-based channel quality report for UE Category NB1 with CB-Msg3-EDT procedure	300

6.6B	Random Access for UE category NB-IoT for frame structure type 1 for NTN-TDD	300

6.6B.1	Introduction	300

6.6B.2	Requirements	301

6.6B.2.1	Correct behaviour when receiving Random Access Response reception	301

6.6B.2.2	Correct behaviour when not receiving Random Access Response reception	301

6.6B.2.3	Correct behaviour when receiving a NACK on msg3	301

6.6B.2.4	Correct behaviour when receiving a message over Temporary C-RNTI	301

6.6B.2.5	Correct behaviour when contention Resolution timer expires	301

6.6B.2.6	MSG3-based channel quality report for UE Category NB1	301

6.6B.3	Requirements for NPRACH configuration	302

6.7	RRC Re-establishment for Cat-M1 UEs	302

6.7.1	Introduction	302

6.7.2	Requirements	302

6.7.2.1	UE Re-establishment delay requirement for CEModeA	302

6.7.2.2	UE Re-establishment delay requirement for CEModeB	303

6.7A	RRC Re-establishment for Cat-M1 UEs for Satellite Access	303

6.7A.1	Introduction	303

6.7A.2	Requirements	304

6.7A.2.1	UE Re-establishment delay requirement for CEModeA	304

6.7A.2.2	UE Re-establishment delay requirement for CEModeB	305

6.8	RRC Connection Release with Redirection for Cat-M1 UEs	305

6.8.1	Introduction	305

6.8.2	Requirements	305

6.8.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	305

6.8A	RRC Connection Release with Redirection for UE Category M1 for Satellite Access	306

6.8A.1	Introduction	306

6.8A.2	Requirements	306

6.8A.2.1	RRC connection release with redirection to E-UTRAN with CE Mode A	306

6.9	RRC Connection Redirection to Non-anchor Carrier in NB-IoT	307

6.9.1	Introduction	307

6.9.2	Requirements	307

6.9A	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for Satellite Access	308

6.9A.1	Introduction	308

6.9B	RRC Connection Redirection to Non-anchor Carrier in NB-IoT for frame structure type 1 for NTN-TDD	309

6.9B.1	Introduction	309

6.9B.2	Requirements	309

7	Timing and signalling characteristics	310

7.1	UE transmit timing	310

7.1.1	Introduction	310

7.1.2	Requirements	310

7.2	UE timer accuracy	311

7.2.1	Introduction	311

7.2.2	Requirements	311

7.3	Timing Advance	312

7.3.1	Introduction	312

7.3.2	Requirements	312

7.3.2.1	Timing Advance adjustment delay	312

7.3.2.2	Timing Advance adjustment accuracy	312

7.4	Cell phase synchronization accuracy (TDD)	312

7.4.1	Definition	312

7.4.2	Minimum requirements	313

7.5	Synchronization Requirements for E-UTRAN to 1xRTT and HRPD Handovers	313

7.5.1	Introduction	313

7.5.2	eNodeB Synchronization Requirements	313

7.5.2.1	Synchronized E-UTRAN	313

7.5.2.2	Non-Synchronized E-UTRAN	314

7.6	Radio Link Monitoring	314

7.6.1	Introduction	314

7.6.2	Requirements	315

7.6.2.1	Minimum requirement when no DRX is used	315

7.6.2.2	Minimum requirement when DRX is used	316

7.6.2.3	Minimum requirement at transitions	317

7.6.2.4	Minimum requirement during SI Acquisition with autonomous gaps	317

7.6.2.5	Minimum requirement under IDC Interference	318

7.7	SCell Activation and Deactivation Delay for E-UTRA Carrier Aggregation	318

7.7.1	Introduction	318

7.7.2	SCell Activation Delay Requirement for Deactivated SCell	318

7.7.3	SCell Deactivation Delay Requirement for Activated SCell	320

7.7.4	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	321

7.7.5	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	322

7.7.6	SCell Activation Delay Requirement for Deactivated PUCCH SCell	323

7.7.7	SCell Activation Delay Requirement for Deactivated PUCCH SCell with Multiple SCells	324

7.7.8	SCell Deactivation Delay Requirement for Activated PUCCH SCell	324

7.7.9	SCell Deactivation Delay Requirement for Activated PUCCH SCell with Multiple SCells	324

7.7.10	SCell Activation Delay Requirement for Deactivated SCell under Frame Structure 3	325

7.7.11	SCell Deactivation Delay Requirement for Activated SCell under Frame Structure 3	326

7.7.12	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells under Frame Structure 3	327

7.7.13	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells under Frame Structure 3	328

7.7.14	SCell Activation Delay Requirement for Dormant SCell	328

7.7.15	SCell Hibernation Delay Requirement for Activated SCell	329

7.7.16	SCell Hibernation Delay Requirement for Deactivated SCell	330

7.7.17	SCell Deactivation Delay Requirement for Dormant SCell	332

7.7.18	Direct SCell Activation and Hibernation Delay Requirement	332

7.7.19	Direct SCell Activation and Hibernation Delay Requirement at RRC Reconfiguration during Handover	334

7.8	Interruptions with Carrier Aggregation	336

7.8.1	Introduction	336

7.8.2	Requirements	336

7.8.2.1	Interruptions at SCell addition/release for intra-band CA	336

7.8.2.2	Interruptions at SCell addition/release for inter-band CA	336

7.8.2.3	Interruptions at SCell activation/deactivation for intra-band CA	336

7.8.2.4	Interruptions at SCell activation/deactivation for inter-band CA	336

7.8.2.5	Interruptions during measurements on SCC for intra-band CA	337

7.8.2.6	Interruptions during measurements on SCC for inter-band CA	337

7.8.2.7	Interruptions at SCell addition/release with multiple downlink SCells	337

7.8.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	337

7.8.2.9	Interruptions during measurements on SCC with multiple downlink SCells	338

7.8.2.10	Interruptions at overlapping addition/release/activation/deactivation of SCells	339

7.8.2.11	Interruptions during RSSI measurements on one SCC under Frame Structure 3	339

7.8.2.12	Interruptions during RSSI measurements on multiple SCCs under Frame Structure 3	339

7.8.2.13	Interruptions at SRS carrier based switching	340

7.8.2.14	Interruptions at SCell activation and deactivation of dormant SCell for intra-band CA	340

7.8.2.15	Interruptions at SCell activation and deactivation of dormant SCell for inter-band CA	341

7.8.2.16	Interruptions at SCell activation and deactivation of multiple dormant SCells	341

7.8.2.17	Interruptions during CQI measurement on dormant SCell	341

7.8.2.18	Interruptions during RRM measurement on dormant SCell for intra-band CA	341

7.8.2.19	Interruptions during RRM measurement on dormant SCell for inter-band CA	342

7.8.2.20	Interruptions at SCell hibernation	342

7.8.2.21	Interruptions at direct SCell activation and hibernation	342

7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap	343

7.9	Maximum Transmission Timing Difference in Carrier Aggregation	343

7.9.1	Introduction	343

7.9.2	Minimum Requirements for Interband Carrier Aggregation	343

7.9.3	Minimum Requirements for Intraband non-contiguous Carrier Aggregation	344

7.9.4	Minimum Requirements for Inter-Band Carrier Aggregation under Frame Structure 3	344

7.10	Interruptions with RSTD Measurements with Carrier Aggregation	344

7.10.1	Introduction	344

7.10.2	Requirements	345

7.10.2.1	Interruptions during RSTD measurements on SCC for intra-band CA with one downlink SCell	345

7.10.2.2	Interruptions during RSTD measurements on SCC for inter-band CA with one downlink SCell	345

7.10.2.3	Interruptions during RSTD measurements on SCC with multiple downlink SCells	345

7.10.2.4	Interruptions at overlapping RSTD and inter-frequency measurements	346

7.11	Radio Link Monitoring for UE Category 0	346

7.11.1	Introduction	346

7.11.2	Requirements for FD-FDD and TDD	347

7.11.2.1	Minimum requirement when no DRX is used	347

7.11.2.2	Minimum requirement when DRX is used	348

7.11.2.3	Minimum requirement at transitions	348

7.11.3	Requirements for HD-FDD	349

7.11.3.1	Minimum requirement when no DRX is used	349

7.11.3.2	Minimum requirement when DRX is used	349

7.11.3.3	Minimum requirement at transitions	350

7.12	Interruptions with Dual Connectivity	350

7.12.1	Introduction	350

7.12.2	Requirements	350

7.12.2.1	Interruptions at PSCell addition/release	350

7.12.2.2	Interruptions at transitions between active and non-active during DRX	350

7.12.2.3	Interruptions at transitions from non-DRX to DRX	351

7.12.2.4	Interruptions at SCell addition/release	351

7.12.2.5	Interruptions at SCell activation/deactivation	351

7.12.2.6	Interruptions during measurements on SCC	352

7.12.2.7	Interruptions at SRS carrier based switching	352

7.13	Cell phase synchronization accuracy (Synchronized mode of dual connectivity)	353

7.13.1	Definition	353

7.13.2	Minimum requirements	353

7.14	PSCell Addition and Release Delay for E-UTRA Dual Connectivity	353

7.14.1	Introduction	353

7.14.2	PSCell Addition Delay Requirement	353

7.14.3	PSCell Release Delay Requirement	354

7.15	Maximum Receive Timing Difference in Dual Connectivity	354

7.15.1	Introduction	354

7.15.2	Minimum Requirements for Inter-band Dual Connectivity	354

7.16	Proximity-based Services	354

7.16.1	Introduction	354

7.16.2	Requirements	354

7.16.2.1	ProSe UE transmission timing	354

7.16.2.1.1	Serving cell or PCell as timing reference	355

7.16.2.1.2	SCell or non-serving cell as timing reference	355

7.16.3	Interruptions with ProSe	355

7.16.3.1	Interruptions at ProSe Direct Discovery configuration	355

7.16.3.2	Interruptions at ProSe Direct Communication configuration	356

7.16.3.3	Interruptions during ProSe Direct Discovery	356

7.16.3.4	Interruptions during ProSe Direct Discovery with discovery gaps	356

7.16.3.5	Interruptions during ProSe Direct Communication	357

7.16.4	Cell reselection for ProSe Direct Discovery on non-serving frequency	357

7.16.4.1	Measurement and evaluation of selected cell	357

7.16.4.2	Measurement of intra-frequency E-UTRAN cells	357

7.16.5	Selection / Reselection of ProSe relay UE	358

7.16.6	ProSe operation under deactivated SCell	358

7.17	Maximum Transmission Timing Difference in Dual Connectivity	358

7.17.1	Introduction	358

7.17.2	Minimum Requirements for maximum transmission timing difference Inter-band Dual Connectivity	359

7.18.1	Introduction	359

7.18.2	SCell Activation Delay Requirement for Deactivated SCell	359

7.18.3	SCell Deactivation Delay Requirement for Activated SCell	359

7.19	Radio Link Monitoring for UE Category M1	359

7.19.1	Introduction	359

7.19.2	Requirements for FD-FDD and TDD CE mode A	360

7.19.2.1	Minimum requirement when no DRX is used	361

7.19.2.2	Minimum requirement when DRX is used	362

7.19.2.3	Minimum requirement at transitions	363

7.19.3	Requirements for HD-FDD with CE mode A	363

7.19.3.1	Minimum requirement when no DRX is used	364

7.19.3.2	Minimum requirement when DRX is used	364

7.19.3.3	Minimum requirement at transitions	365

7.19.4	Requirements for FD-FDD and TDD with CE mode B	365

7.19.4.1	Minimum requirement when no DRX is used	367

7.19.4.2	Minimum requirement when DRX is used	368

7.19.4.3	Minimum requirement at transitions	369

7.19.5	Requirements for HD-FDD with CE mode B	369

7.19.5.1	Minimum requirement when no DRX is used	369

7.19.5.2	Minimum requirement when DRX is used	370

7.19.5.3	Minimum requirement at transitions	371

7.19A	Radio Link Monitoring for UE Category M1 for Satellite Access	371

7.19A.1	Introduction	371

7.19A.2	Requirements for FD-FDD and CE mode A	371

7.19A.2.1	Minimum requirement when no DRX is used	373

7.19A.2.2	Minimum requirement when DRX is used	374

7.19A.2.3	Minimum requirement at transitions	375

7.19A.3	Requirements for HD-FDD with CE mode A	375

7.19A.3.1	Minimum requirement when no DRX is used	375

7.19A.3.2	Minimum requirement when DRX is used	376

7.19A.3.3	Minimum requirement at transitions	377

7.19A.4	Requirements for HD-FDD with CE mode B	377

7.19A.4.1	Minimum requirement when no DRX is used	377

7.19A.4.2	Minimum requirement when DRX is used	377

7.19A.4.3	Minimum requirement at transitions	378

7.20	UE transmit timing for NB-IoT	379

7.20.1	Introduction	379

7.20.2	Requirements	379

7.20A	UE transmit timing for NB-IoT for Satellite Access	379

7.20A.1	Introduction	379

7.20A.2	Requirements	380

7.20B	UE transmit timing for NB-IoT for frame structure type 1 for NTN-TDD	380

7.20B.1	Introduction	380

7.20B.2	Requirements	381

7.21	 UE timer accuracy for NB-IoT	381

7.21.1	Introduction	381

7.21.2	Requirements	381

7.21A	UE timer accuracy for NB-IoT for Satellite Access	382

7.21A.1	Introduction	382

7.21A.2	Requirements	382

7.21B	UE timer accuracy for NB-IoT for frame structure type 1 for NTN-TDD	382

7.21B.1	Introduction	382

7.21B.2	Requirements	382

7.22	Timing Advance for NB-IoT	383

7.22.1	Introduction	383

7.22.2	Requirements	383

7.22.2.1	Timing Advance adjustment delay	383

7.22.2.2	Timing Advance adjustment accuracy	383

7.22A	Timing Advance for NB-IoT for Satellite Access	383

7.22A.1	Introduction	383

7.22A.2	Requirements	383

7.22A.2.1	Timing Advance adjustment delay	383

7.22A.2.2	Timing Advance adjustment accuracy	383

7.22B	Timing Advance for NB-IoT for frame structure type 1 for NTN-TDD	383

7.22B.1	Introduction	383

7.22B.2	Requirements	383

7.22B.2.1	Timing Advance adjustment delay	383

7.22B.2.2	Timing Advance adjustment accuracy	384

7.23	Radio Link Monitoring for Category NB1 UE	384

7.23.1	Introduction	384

7.23.2	Requirements for Category NB1 UE	384

7.23.2.1	Minimum requirement when no DRX is used	384

7.23.2.2	Minimum requirement when DRX is used	385

7.23.2.3	Minimum requirement at transitions	385

7.23A	Radio Link Monitoring for Category NB-IoT UE for Satellite Access	386

7.23A.1	Introduction	386

7.23A.2	Requirements for Category NB1 UE	386

7.23A.2.1	Minimum requirement when no DRX is used	386

7.23A.2.2	Minimum requirement when DRX is used	387

7.23A.2.3	Minimum requirement at transitions	387

7.23B	Radio Link Monitoring for Category NB-IoT UE for frame structure type 1 for NTN-TDD	387

7.23B.1	Introduction	387

7.23B.2	Requirements for Category NB1 UE	388

7.23B.2.1	Minimum requirement when no DRX is used	388

7.23B.2.2	Minimum requirement when DRX is used	389

7.23B.2.3	Minimum requirement at transitions	389

7.24	UE transmit timing for Category M1	389

7.24.1	Introduction	389

7.24.2	Requirements	390

7.24A	UE transmit timing for Category M1 for Satellite Access	390

7.24A.1	Introduction	390

7.24A.2	Requirements	391

7.25	Cell phase synchronization accuracy for MBMS services (FDD)	392

7.25.1	Definition	392

7.25.2	Minimum requirements	392

7.26	UE transmit timing for Category M2	392

7.26.1	Introduction	392

7.26.2	Requirements	392

7.27	UE timer accuracy for category M1	393

7.27.1	Introduction	393

7.27.2	Requirements	393

7.27A	UE timer accuracy for category M1 for Satellite Access	393

7.27A.1	Introduction	393

7.27A.2	Requirements	393

7.28	Timing Advance for Category M1	393

7.28.1	Introduction	393

7.28.2	Requirements	393

7.28A	Timing Advance for Category M1 for Satellite Access	393

7.28A.1	Introduction	393

7.28A.2	Requirements	393

7.28A.2.1	Timing Advance adjustment delay	393

7.28A.2.2	Timing Advance adjustment accuracy	394

7.29	Interruptions requirements with FeMBMS	394

7.29.1	Introduction	394

7.29.2	Requirements	394

7.30	Numerology switching delay requirements with FeMBMS	394

7.30.1	Introduction	394

7.30.2	Requirements	394

7.31	NR PSCell Addition and Release Delay for E-UTRA - NR Dual Connectivity	394

7.31.1	Introduction	394

7.31.2	NR PSCell Addition Delay Requirement	395

7.31.3	NR PSCell Release Delay Requirement	395

7.31A	Addition and Release Delay of NR PSCell Operating with CCA for E-UTRA - NR Dual Connectivity	396

7.31A.1	Introduction	396

7.31A.2	NR PSCell Addition Delay Requirement	396

7.31A.3	NR PSCell Release Delay Requirement	397

7.32	Interruptions with EN-DC	397

7.32.1	Introduction	397

7.32.2	Requirements	398

7.32.2.1	Interruptions at PSCell addition/release	398

7.32.2.2	Interruptions at transitions between active and non-active during DRX	398

7.32.2.3	Interruptions at transitions from non-DRX to DRX	398

7.32.2.4	Interruptions at SCell addition/release	398

7.32.2.5	Interruptions at SCell activation/deactivation	398

7.32.2.6	Interruptions during measurements on SCC	399

7.32.2.6.1	Interruptions during measurements on deactivated NR SCC	399

7.32.2.6.2	Interruptions during measurements on deactivated E-UTRA SCC	399

7.32.2.6.3	Interruptions during CQI measurements on dormant E-UTRA SCell	399

7.32.2.6.4	Interruptions during RRM measurements on dormant E-UTRA SCC	400

7.32.2.7	Interruptions at active BWP switching	400

7.32.2.8	Interruptions at SCell activation and deactivation of dormant SCell	401

7.32.2.9	Interruptions at SCell activation and deactivation of multiple dormant SCell	401

7.32.2.10	Interruptions at SCell hibernation	401

7.32.2.11	Interruptions at direct SCell activation and hibernation	401

7.32.2.12	DL Interruptions at UE switching between two uplink carriers	401

7.32.2.13	Interruptions at NR SRS carrier based switching	401

7.32.2.14	Interruptions at NR SCell dormancy	402

7.32.2.14.1	Interruptions due to NR SCell dormancy switch	402

7.32.2.14.2	Interruptions due to CSI and RRM measurements during SCell dormancy	403

7.32.2.15	Interruption during NR measurement with autonomous gaps	403

7.32.2.16	Interruptions at SRS carrier based switching	403

7.32.2.17	Interruptions at SCG activation/deactivation	404

7.32.2.18	Interruptions due to NR SRS antenna port switching	404

7.32.2.19	Interruptions at fast SCell activation/deactivation	404

7.32.2.20	Interruptions due to RRM measurements on deactivated NR SCG	405

7.32.2.21	Interruptions during RLM/BFD measurements on deactivated PScell	405

7.33	Maximum Transmit/Receive Timing Difference in Carrier Aggregation for sTTI and 1ms-TTI with 3 subframe HARQ processing	405

7.33.1	Introduction	405

7.33.2	Requirements	405

7.34	Void	406

7.35	Interruptions with SFTD measurements	406

7.35.1	Introduction	406

7.35.2	Requirements	406

7.36	Interruptions with NE-DC	406

7.36.1	Introduction	406

7.36.2	Requirements	407

7.36.2.1	Interruptions at transitions between active and non-active during DRX	407

7.36.2.2	Interruptions at transitions from non-DRX to DRX	407

7.36.2.3	Interruptions at SCell addition/release	407

7.36.2.4	Interruptions at SCell activation/deactivation	407

7.36.2.5	Interruptions during measurements on SCC	408

7.36.2.5.1	Interruptions during measurements on deactivated NR SCC	408

7.36.2.5.2	Interruptions during measurements on deactivated E-UTRA SCC	408

7.36.2.5.3	Interruptions during CQI measurements on dormant E-UTRA SCell	408

7.36.2.5.4	Interruptions during RRM measurements on dormant E-UTRA SCC	408

7.36.2.6	Interruptions at active BWP switching	408

7.36.2.7	Interruptions at SCell activation and deactivation of dormant SCell	409

7.36.2.8	Interruptions at SCell activation and deactivation of multiple dormant SCell	409

7.36.2.9	Interruptions at SCell hibernation	409

7.36.2.10	Interruptions at direct SCell activation and hibernation	409

7.36.2.11	Interruptions at NR SRS carrier based switching	410

7.36.2.12	Interruptions at NR SCell dormancy	410

7.36.2.12.1	Interruptions due to NR SCell dormancy switch	410

7.36.2.12.2	Interruptions due to CSI and RRM measurements during SCell dormancy	411

7.36.2.13	Interruption during E-UTRA measurement with autonomous gaps	411

7.36.2.14	Interruption during NR measurement with autonomous gaps	411

7.36.2.15	Interruptions at SRS carrier based switching	412

7.36.2.16	Interruptions due to NR SRS antenna port switching	412

7.37	Interruptions during NR measurement with autonomous gaps	413

7.37.1	Introduction	413

7.37.2	Requirements	413

7.38 SCG Activation and Deactivation Delay	413

7.38.1	Introduction	413

7.38.2	SCG Activation Delay Requirement	413

7.38.3	SCG Deactivation Delay Requirement	414

8	UE Measurements Procedures in RRC_CONNECTED State	415

8.1	General Measurement Requirements	415

8.1.1	Introduction	415

8.1.2	Requirements	415

8.1.2.1	UE measurement capability	415

8.1.2.1.1	Monitoring of multiple layers using gaps	423

8.1.2.1.1a	Monitoring of multiple layers using gaps (Increased UE carrier monitoring)	424

8.1.2.1.1b	Monitoring of multiple layers using gaps (EN-DC)	425

8.1.2.1.1c	Monitoring of multiple layers using gaps (NE-DC)	427

8.1.2.1.1d	Monitoring of multiple layers using gaps (RedCap)	428

8.1.2.1.2	Network controlled small gap	428

8.1.2.2	E-UTRAN intra frequency measurements	430

8.1.2.2.1	E-UTRAN FDD intra frequency measurements	430

8.1.2.2.2	E-UTRAN TDD intra frequency measurements	435

8.1.2.2.3	E-UTRAN FDD intra frequency measurements with autonomous gaps	439

8.1.2.2.4	E-UTRAN TDD intra frequency measurements with autonomous gaps	440

8.1.2.2.5	E-UTRAN FDD intra-frequency measurements on carrier with FeMBMS/Unicast mixed cells	441

8.1.2.3	E-UTRAN inter frequency measurements	441

8.1.2.3.1	E-UTRAN FDD – FDD inter frequency measurements	442

8.1.2.3.2	E-UTRAN TDD – TDD inter frequency measurements	447

8.1.2.3.3	E-UTRAN TDD – FDD inter frequency measurements	454

8.1.2.3.4	E-UTRAN FDD – TDD inter frequency measurements	454

8.1.2.3.5	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	454

8.1.2.3.6	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	455

8.1.2.3.7	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	457

8.1.2.3.8	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	458

8.1.2.3.9	E-UTRAN FDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	459

8.1.2.3.10	E-UTRAN TDD – FDD inter frequency measurements with FeMBMS/Unicast mixed cells	466

8.1.2.4	Inter RAT measurements	466

8.1.2.4.1	E-UTRAN FDD – UTRAN FDD measurements	466

8.1.2.4.2	E-UTRAN TDD – UTRAN FDD measurements	471

8.1.2.4.3	E-UTRAN TDD – UTRAN TDD measurements	471

8.1.2.4.4	E-UTRAN FDD – UTRAN TDD measurements	475

8.1.2.4.5	E-UTRAN FDD – GSM measurements	475

8.1.2.4.6	E-UTRAN TDD – GSM measurements	480

8.1.2.4.7	E-UTRAN FDD – UTRAN FDD measurements for SON	480

8.1.2.4.8	E-UTRAN TDD – UTRAN FDD measurements for SON	482

8.1.2.4.9	E-UTRAN FDD – cdma2000 1xRTT measurements	482

8.1.2.4.9.1A	E-UTRAN FDD – cdma2000 1xRTT measurements when no DRX is used	482

8.1.2.4.10	E-UTRAN TDD – cdma2000 1xRTT measurements	483

8.1.2.4.11	E-UTRAN FDD – HRPD measurements	483

8.1.2.4.12	E-UTRAN TDD – HRPD measurements	483

8.1.2.4.13	E-UTRAN TDD – UTRAN TDD measurements for SON	483

8.1.2.4.14	E-UTRAN FDD – UTRAN TDD measurements for SON	485

8.1.2.4.15	E-UTRAN FDD – cdma2000 1xRTT measurements for SON ANR	485

8.1.2.4.16	E-UTRAN TDD – cdma2000 1xRTT measurements for SON ANR	485

8.1.2.4.17	E-UTRAN FDD-UTRAN FDD measurements with autonomous gaps	485

8.1.2.4.18	E-UTRAN TDD-UTRAN FDD measurements with autonomous gaps	486

8.1.2.4.19	E-UTRAN FDD – WLAN measurements	486

8.1.2.4.20	E-UTRAN TDD – WLAN measurements	488

8.1.2.4.21	E-UTRAN FDD – NR measurements	488

8.1.2.4.21A	E-UTRAN FDD – NR measurements when CCA is used	492

8.1.2.4.22	E-UTRAN TDD – NR measurements	496

8.1.2.4.22A	E-UTRAN TDD – NR measurements when CCA is used	496

8.1.2.4.23	Void	496

8.1.2.4.24	Void	496

8.1.2.4.25	E-UTRAN FDD – NR SFTD Measurements	496

8.1.2.4.26	E-UTRAN TDD – NR SFTD Measurements	498

8.1.2.4.27 	E-UTRA FDD - NR measurements with autonomous gaps	498

8.1.2.4.28 	E-UTRA TDD - NR measurements with autonomous gaps	499

8.1.2.4.29	E-UTRAN FDD – NR measurements without measurement gap	499

8.1.2.4.30	E-UTRAN TDD – NR measurements without measurement gap	504

8.1.2.5	E-UTRAN OTDOA Intra-Frequency RSTD Measurements	504

8.1.2.5.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	504

8.1.2.5.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	506

8.1.2.5.3	E-UTRAN FDD Intra-Frequency OTDOA Measurements for UE Category 1bis	507

8.1.2.5.4	E-UTRAN TDD Intra-Frequency OTDOA Measurements for UE Category 1bis	509

8.1.2.6.5	Void	511

8.1.2.6.6	Void	511

8.1.2.6.7	Void	511

8.1.2.6.8	Void	511

8.1.2.6	E-UTRAN Inter-Frequency OTDOA Measurements	511

8.1.2.6.1	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements	511

8.1.2.6.2	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements	513

8.1.2.6.3	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements	515

8.1.2.6.4	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements	516

8.1.2.6.5	E-UTRAN FDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	518

8.1.2.6.6	E-UTRAN TDD-FDD Inter-Frequency OTDOA Measurements for UE Category 1bis	520

8.1.2.6.7	E-UTRAN TDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	521

8.1.2.6.8	E-UTRAN FDD-TDD Inter-Frequency OTDOA Measurements for UE Category 1bis	523

8.1.2.7	E-UTRAN E-CID Measurements	525

8.1.2.7.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	525

8.1.2.7.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	526

8.1.2.7.3	E-UTRAN FDD Intra-frequency E-CID RSRP and RSRQ Measurements	528

8.1.2.7.4	E-UTRAN TDD Intra-frequency E-CID RSRP and RSRQ Measurements	528

8.1.2.8	E-UTRAN intra-frequency measurements under time domain measurement resource restriction	529

8.1.2.8.1	E-UTRAN FDD intra-frequency measurements	529

8.1.2.8.2	E-UTRAN TDD intra-frequency measurements	532

8.1.2.8.3	E-UTRAN FDD intra-frequency measurements with CRS assistance information	535

8.1.2.8.4	E-UTRAN TDD intra-frequency measurements with CRS assistance infromation	538

8.1.2.9	E-UTRAN E-CID Measurements when Time Domain Measurement Resource Restriction Pattern is Configured	542

8.1.2.9.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements	542

8.1.2.9.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements	542

8.1.2.9.3	E-UTRAN FDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	543

8.1.2.9.4	E-UTRAN TDD UE Rx-Tx Time Difference Measurements with CRS Assistance Information	543

8.1.2.10	Void	544

8.2	Capabilities for Support of Event Triggering and Reporting Criteria	544

8.2.1	Introduction	544

8.2.2	Requirements	544

8.3	Measurements for E-UTRA carrier aggregation	548

8.3.1	Introduction	548

8.3.2	Measurements of the primary component carrier	548

8.3.3	Measurements of a secondary component carrier	548

8.3.3.1	Measurements of a secondary component carrier with active SCell	548

8.3.3.2	Measurements of a secondary component carrier with deactivated SCell	549

8.3.3.2.1	E-UTRAN secondary component carrier measurements when no common DRX is used	549

8.3.3.2.2	E-UTRAN secondary component carrier measurements when common DRX is used	550

8.3.3.3	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and activated SCell	552

8.3.3.4	Measurements on a secondary component carrier with FeMBMS/Unicast mixed cells and deactivated SCell	552

8.4	OTDOA RSTD Measurements for E-UTRAN carrier aggregation	552

8.4.1	Introduction	552

8.4.2	Measurements on the primary component carrier	553

8.4.3	Measurements on a secondary component carrier	554

8.4.4	Measurements on both primary component carrier and a secondary component carrier	554

8.4.5	Measurements on different secondary component carriers	555

8.5	Measurements for UE category 0	556

8.5.1	Introduction	556

8.5.2 	Requirements	557

8.5.2.1 	E-UTRAN intra frequency measurements	557

8.5.2.1.1 	E-UTRAN FDD intra frequency measurements	557

8.5.2.1.2 	E-UTRAN intra frequency measurements for HD-FDD	560

8.5.2.1.3	E-UTRAN TDD intra frequency measurements	562

8.5.2.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category 0	566

8.5.2.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category 0	566

8.5.2.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category 0	567

8.6	Discovery signal measurements	568

8.6.1	Introduction	568

8.6.2	Requirements for CRS based discovery signal measurements	568

8.6.2.1	E-UTRAN intra frequency measurements	568

8.6.2.1.1	E-UTRAN FDD intra frequency measurements	568

8.6.2.1.2	E-UTRAN TDD intra frequency measurements	571

8.6.2.2	E-UTRAN inter frequency measurements	573

8.6.2.2.1	E-UTRAN FDD – FDD inter-frequency measurements	574

8.6.2.2.2	E-UTRAN TDD – TDD inter frequency measurements	576

8.6.2.2.3	E-UTRAN TDD – FDD inter frequency measurements	579

8.6.2.2.4	E-UTRAN FDD – TDD inter frequency measurements	579

8.6.3	Requirements for CSI-RS based discovery signal measurements	579

8.6.3.1	E-UTRAN intra frequency measurements	579

8.6.3.1.1	E-UTRAN FDD intra frequency measurements	580

8.6.3.1.2	E-UTRAN TDD intra frequency measurements	582

8.6.3.2	E-UTRAN inter frequency measurements	584

8.6.3.2.1	E-UTRAN FDD – FDD inter frequency measurements	585

8.6.3.2.2	E-UTRAN TDD – TDD inter frequency measurements	587

8.6.3.2.3	E-UTRAN TDD – FDD inter frequency measurements	590

8.6.3.2.4	E-UTRAN FDD – TDD inter frequency measurements	590

8.7	Discovery signal measurements for E-UTRA carrier aggregation	590

8.7.1	Introduction	590

8.7.2	Requirements for CRS based discovery signal measurements for E-UTRA carrier aggregation	591

8.7.2.1	Measurements of the primary component carrier	591

8.7.2.2	Measurements of a secondary component carrier	591

8.7.2.3	Measurements of a secondary component carrier with active SCell	591

8.7.2.4	Measurements of a secondary component carrier with deactivated SCell	591

8.7.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	591

8.7.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	592

8.7.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	594

8.7.3.1	Measurements of the primary component carrier	594

8.7.3.2	Measurements of a secondary component carrier	594

8.7.3.3	Measurements of a secondary component carrier with active SCell	594

8.7.3.4	Measurements of a secondary component carrier with deactivated SCell	594

8.7.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	594

8.7.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	596

8.8	Measurements for E-UTRA dual connectivity	597

8.8.1	Introduction	597

8.8.2	Intra-frequency measurements requirements on PCell	597

8.8.3	Intra-frequency measurements requirements on PSCell	598

8.8.4	Inter-frequency and inter-RAT measurement requirements	598

8.8.5	Intra-frequency measurements with autonomous gaps	598

8.8.5.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	598

8.8.5.2	ECGI reporting delay	599

8.8.6	Inter-frequency measurements with autonomous gaps	599

8.8.6.1	Identification of a new CGI of E-UTRA cell with autonomous gaps	599

8.8.6.2	ECGI reporting delay	600

8.8.7	SSTD Measurements	600

8.8.7.1	Introduction	600

8.8.7.2	SSTD Measurement requirements	600

8.8.7.3	SSTD Measurement Reporting Delay	601

8.8.8	Intra-frequency measurements requirements on SCell	601

8.9	MBSFN Measurements	601

8.9.1	Introduction	601

8.9.2	MBSFN RSRP Measurements	601

8.9.3	MBSFN RSRQ Measurements	602

8.9.4	MCH BLER Measurements	602

8.10	Proximity-based Services	602

8.10.1	Introduction	602

8.10.2	Requirements	602

8.10.2.1	Initiation/Cease of SLSS transmissions with ProSe Direct Discovery	602

8.10.2.2	Initiation/Cease of SLSS transmissions with ProSe Direct Communication	603

8.11	Discovery Signal Measurements under Operation with Frame Structure 3	604

8.11.1	Introduction	604

8.11.2	CRS based discovery signal measurements	604

8.11.2.1	E-UTRAN intra-frequency measurements	604

8.11.2.1.1	Requirements	604

8.11.2.1.1.1	Requirements when no DRX is used	604

8.11.2.1.1.1.1	Measurement Reporting Requirements	606

8.11.2.1.1.2	Requirements when DRX is used	607

8.11.2.1.1.2.1	Measurement Reporting Requirements	609

8.11.2.2	E-UTRAN inter-frequency measurements	610

8.11.2.2.1	E-UTRAN FDD-FS3 inter-frequency measurements	610

8.11.2.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	614

8.11.3	CSI-RS based discovery signal measurements	614

8.11.3.1	E-UTRAN intra-frequency measurements	614

8.11.3.1.1	Requirements	614

8.11.3.1.1.1	Requirements when no DRX is used	614

8.11.3.1.1.1.1	Measurement Reporting Requirements	616

8.11.3.1.1.2	Requirements when DRX is used	616

8.11.3.1.1.2.1	Measurement Reporting Requirements	617

8.11.3.2	E-UTRAN inter-frequency measurements	618

8.11.3.2.1	E-UTRAN FDD – FS3 inter-frequency measurements	618

8.11.3.2.2	E-UTRAN TDD – FS3 inter-frequency measurements	622

8.11.4	RSSI measurements	622

8.11.4.1	E-UTRAN intra-frequency measurements	622

8.11.4.2	E-UTRAN inter-frequency measurements	622

8.11.5	Channel occupancy measurements	623

8.11.5.1	E-UTRAN intra-frequency channel occupancy measurements	623

8.11.5.2	E-UTRAN inter-frequency channel occupancy measurements	623

8.12	Discovery Signal Measurements for E-UTRA Carrier Aggregation under Operation with Frame Structure 3	623

8.12.1	Introduction	623

8.12.2	CRS based discovery signal measurements for E-UTRA carrier aggregation	623

8.12.2.1	Introduction	623

8.12.2.2	Measurements of a secondary component carrier	623

8.12.2.3	Measurements of a secondary component carrier with active SCell	623

8.12.2.4	Measurements of a secondary component carrier with deactivated SCell	624

8.12.2.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	624

8.12.2.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	626

8.12.3	Requirements for CSI-RS based discovery signal measurements for E-UTRA carrier aggregation	629

8.12.3.1	Introduction	629

8.12.3.2	Measurements of a secondary component carrier	629

8.12.3.3	Measurements of a secondary component carrier with active SCell	629

8.12.3.4	Measurements of a secondary component carrier with deactivated SCell	629

8.12.3.4.1	E-UTRAN secondary component carrier measurements when no common DRX is used	629

8.12.3.4.2	E-UTRAN secondary component carrier measurements when common DRX is used	631

8.13	Measurements for UE Category M1	633

8.13.1	Introduction	633

8.13.2	Requirements for UE category M1 with CE mode A	633

8.13.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	634

8.13.2.1.1	E-UTRAN FDD intra frequency measurements	634

8.13.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	639

8.13.2.1.3	E-UTRAN TDD intra frequency measurements	641

8.13.2.2	Void	645

8.13.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	645

8.13.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	645

8.13.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	648

8.13.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	650

8.13.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeA	651

8.13.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	651

8.13.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	653

8.13.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	655

8.13.2.5	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode A	656

8.13.2.5.1	Intra-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	656

8.13.2.5.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	656

8.13.2.5.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	657

8.13.2.5.4	Inter-frequency FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	657

8.13.2.5.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	657

8.13.2.5.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeA	658

8.13.2.5.7	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	658

8.13.2.5.8	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	659

8.13.2.5.9	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M1 in CEModeA	660

8.13.2.6	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	660

8.13.2.6.1	E-UTRAN FDD - FDD inter frequency measurements	660

8.13.2.6.2	E-UTRAN inter-frequency measurements for HD-FDD	665

8.13.2.6.3	E-UTRAN TDD inter frequency measurements	667

8.13.2.7	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	671

8.13.2.8	Channel quality report for UE Category M1 in connected mode with CE mode A	671

8.13.3	Requirements for UE category M1 with CE mode B	672

8.13.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	672

8.13.3.1.1	E-UTRAN FDD intra frequency measurements	673

8.13.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	677

8.13.3.1.3	E-UTRAN TDD intra frequency measurements	680

8.13.3.1.4	E-UTRAN FDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	685

8.13.3.1.5	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD UE category M1 with CE mode B	686

8.13.3.1.6	E-UTRAN TDD intra frequency measurements with autonomous gaps for UE category M1 with CE mode B	686

8.13.3.2	Void	687

8.13.3.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	687

8.13.3.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	687

8.13.3.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	690

8.13.3.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	692

8.13.3.4	E-UTRAN E-CID Measurements Requirements for UE category M1 with CE mode B	693

8.13.3.4.1	Intra-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	693

8.13.3.4.2	Intra-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	693

8.13.3.4.3	Intra-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	693

8.13.3.4.4	Inter-frequency E-CID FDD RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	694

8.13.3.4.5	Inter-frequency HD-FDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	694

8.13.3.4.6	Inter-frequency TDD E-CID RSRP and RSRQ Measurements for Cat-M1 UE in CEModeB	695

8.13.3.5	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	695

8.13.3.5.1	E-UTRAN FDD - FDD inter frequency measurements	695

8.13.3.5.2 	E-UTRAN inter-frequency measurements for HD-FDD	700

8.13.3.5.3	E-UTRAN TDD inter frequency measurements	702

8.13.3.6	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	707

8.13.3.7	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M1 UE in CEModeB	707

8.13.3.7.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	707

8.13.3.7.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	709

8.13.3.7.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	712

8.13.3.8	Channel quality report for UE Category M1 in connected mode with CE mode B	712

8.13A	Measurements for UE Category M1 for Satellite Access	713

8.13A.1	Introduction	713

8.13A.2	Requirements for UE category M1 with CE mode A	713

8.13A.2.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode A	714

8.13A.2.1.1	E-UTRAN FDD intra frequency measurements	714

8.13A.2.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	714

8.13A.2.1.1.2	E-UTRAN intra frequency measurements when DRX is used	716

8.13A.2.1.2	E-UTRAN intra frequency measurements for HD-FDD	718

8.13A.2.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	718

8.13A.2.1.2.2	E-UTRAN intra frequency measurements when DRX is used	718

8.13A.2.2	E-UTRAN inter frequency measurements by UE category M1 with CE mode A	720

8.13A.2.2.1	E-UTRAN FDD - FDD inter frequency measurements	720

8.13A.2.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	720

8.13A.2.2.1.2	E-UTRAN inter frequency measurements when DRX is used	722

8.13A.2.2.2	E-UTRAN inter-frequency measurements for HD-FDD	724

8.13A.2.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	724

8.13A.2.2.2.2	E-UTRAN inter frequency measurements when DRX is used	724

8.13A.2.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode A	726

8.13A.2.4	Channel quality report for UE Category M1 in connected mode with CE mode A	727

8.13A.3	Requirements for UE category M1 with CE mode B	727

8.13A.3.1	E-UTRAN intra frequency measurements by UE category M1 with CE mode B	728

8.13A.3.1.1	E-UTRAN FDD intra frequency measurements	728

8.13A.3.1.1.1	E-UTRAN intra frequency measurements when no DRX is used	728

8.13A.3.1.1.2	E-UTRAN intra frequency measurements when DRX is used	730

8.13A.3.1.2	E-UTRAN intra frequency measurements for HD-FDD	732

8.13A.3.1.2.1	E-UTRAN intra frequency measurements when no DRX is used	732

8.13A.3.1.2.2	E-UTRAN intra frequency measurements when DRX is used	733

8.13A.3.2	E-UTRAN inter frequency measurements by UE category M1 with CE Mode B	735

8.13A.3.2.1	E-UTRAN FDD - FDD inter frequency measurements	735

8.13A.3.2.1.1	E-UTRAN FDD - FDD inter frequency measurements when no DRX is used	735

8.13A.3.2.1.2	E-UTRAN inter frequency measurements when DRX is used	737

8.13A.3.2.2	E-UTRAN inter-frequency measurements for HD-FDD	739

8.13A.3.2.2.1	E-UTRAN inter-frequency measurements when no DRX is used	739

8.13A.3.2.2.2	E-UTRAN inter frequency measurements when DRX is used	739

8.13A.3.3	Maximum allowed layers for multiple monitoring for UE category M1 with CE mode B	741

8.13A.3.4	Channel quality report for UE Category M1 in connected mode with CE mode B	742

8.14	Measurements for UE category NB1	742

8.14.1	Introduction	742

8.14.2	NB-IoT intra frequency measurements under normal coverage	742

8.14.2.1	NB-IoT intra frequency measurements when no DRX is used	742

8.14.2.2	NB-IoT intra frequency measurements when DRX is used	742

8.14.3	NB-IoT intra frequency measurements under enhanced coverage	743

8.14.3.1	NB-IoT intra frequency measurements when no DRX is used	743

8.14.3.2	NB-IoT intra frequency measurements when DRX is used	743

8.14.4	Connected mode channel quality report for UE Category NB1	743

8.14.5	Connected mode channel quality report for UE Category NB2 supporting 16QAM	743

8.14.6	NB-IoT neighbour cell measurements	744

8.14.6.1	Introduction	744

8.14.6.2	Requirements	744

8.14.6.3	Intra-frequency neighbour cell measurements	744

8.14.6.4	Inter-frequency neighbour cell measurements	745

8.14.6.5	 Requirements for monitoring multiple carriers	746

8.14A	Measurements for UE category NB-IoT for Satellite Access	746

8.14A.1	Introduction	746

8.14A.2	NB-IoT intra frequency measurements under normal coverage	747

8.14A.2.1	NB-IoT intra frequency measurements when no DRX is used	747

8.14A.2.2	NB-IoT intra frequency measurements when DRX is used	747

8.14A.3	NB-IoT intra frequency measurements under enhanced coverage	747

8.14A.3.1	NB-IoT intra frequency measurements when no DRX is used	747

8.14A.3.2	NB-IoT intra frequency measurements when DRX is used	747

8.14A.4	Connected mode channel quality report for UE Category NB1	747

8.14B	Measurements for UE category NB-IoT for frame structure type 1 for NTN-TDD	750

8.14B.1	Introduction	750

8.14B.2	NB-IoT intra frequency measurements under normal coverage	751

8.14B.2.1	NB-IoT intra frequency measurements when no DRX is used	751

8.14B.2.2	NB-IoT intra frequency measurements when DRX is used	751

8.14B.4	Connected mode channel quality report for UE Category NB1	751

8.14B.6	NB-IoT neighbour cell measurements	752

8.14B.6.1	Introduction	752

8.14B.6.2	Requirements	752

8.14B.6.3	Intra-frequency neighbour cell measurements	752

8.14B.6.4	Inter-frequency neighbour cell measurements	753

8.14B.6.5	Requirements for monitoring multiple carriers	754

8.15	Void	754

8.16	Measurements for UE Category M2	754

8.16.1	Introduction	754

8.16.2	Requirements for UE category M2 with CE mode A	755

8.16.2.1	E-UTRAN FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	755

8.16.2.1.1	UE Rx-Tx Measurement Reporting Delay	755

8.16.2.2	E-UTRAN TDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	756

8.16.2.2.1	UE Rx-Tx Measurement Reporting Delay	756

8.16.2.2a	E-UTRAN HD-FDD UE Rx-Tx Time Difference Measurements for UE category M2 in CEModeA	757

8.16.2.2a.1	UE Rx-Tx Measurement Reporting Delay	757

8.16.2.3	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	757

8.16.2.3.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	758

8.16.2.3.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	760

8.16.2.3.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	763

8.16.2.4	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeA	763

8.16.2.4.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	763

8.16.2.4.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	765

8.16.2.4.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	768

8.16.3	Requirements for UE category M2 with CE mode B	768

8.16.3.1	E-UTRAN OTDOA Intra-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	768

8.16.3.1.1	E-UTRAN FDD Intra-Frequency OTDOA Measurements	768

8.16.3.1.2	E-UTRAN TDD Intra-Frequency OTDOA Measurements	771

8.16.3.1.3	E-UTRAN HD-FDD Intra-Frequency OTDOA Measurements	774

8.16.3.2	E-UTRAN OTDOA Inter-Frequency RSTD Measurements for Cat-M2 UE in CEModeB	774

8.16.3.2.1	E-UTRAN FDD Inter-Frequency OTDOA Measurements	774

8.16.3.2.2	E-UTRAN TDD Inter-Frequency OTDOA Measurements	776

8.16.3.2.3	E-UTRAN HD-FDD Inter-Frequency OTDOA Measurements	779

8.17	Measurements for E-UTRA – NR Dual Connectivity	779

8.17.1	Introduction	779

8.17.1.1	Measurement Gap Sharing	779

8.17.1A	Intrafrequency Measurements	780

8.17.2	SFTD Measurements	780

8.17.2.1	Introduction	780

8.17.2.2	SFTD Measurement requirements	780

8.17.2.2.a	SFTD Measurement requirements with CCA on target frequency	781

8.17.2.3	SFTD Measurement Reporting Delay	782

8.17.3	E-UTRA Inter-frequency Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	782

8.17.3.1	Introduction	782

8.17.3.2	E-UTRAN FDD inter frequency measurements	783

8.17.3.2.1	E-UTRAN FDD inter frequency measurements when no DRX is used	783

8.17.3.2.2	E-UTRAN FDD inter frequency measurements when DRX is used	784

8.17.3.3	E-UTRAN TDD inter frequency measurements	786

8.17.3.3.1	E-UTRAN TDD inter frequency measurements when no DRX is used	786

8.17.3.3.2	E-UTRAN TDD inter frequency measurements when DRX is used	788

8.17.4	E-UTRA Inter-RAT NR Measurements when Configured with E-UTRA-NR Dual Connectivity Operation	790

8.17.4.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	790

8.17.4.1.1	NR Inter-RAT cell identification	790

8.17.4.1.2	NR Inter-RAT measurement	792

8.17.4.1.3	NR Inter-RAT measurement reporting	792

8.17.4.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	793

8.17.4A	E-UTRA Inter-RAT NR Measurements when CCA is used when Configured with E-UTRA-NR Dual Connectivity Operation	793

8.17.4A.1	E-UTRAN FDD – NR measurements when configured with E-UTRA-NR Dual connectivity	793

8.17.4A.1.1	NR Inter-RAT cell identification	794

8.17.4A.1.2	NR Inter-RAT measurement	795

8.17.4A.1.3	NR Inter-RAT measurement reporting	796

8.17.4A.1.4	NR inter-RAT RSSI measurements	797

8.17.4A.1.5	NR inter-RAT channel occupancy measurements	797

8.17.4A.2	E-UTRAN TDD – NR measurements when configured with E-UTRA-NR Dual connectivity	798

8.17.5	E-UTRAN FDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	798

8.17.5.1	Introduction	798

8.17.5.2	E-UTRAN FDD – UTRAN FDD measurements when no DRX is used	798

8.17.5.2.1	Identification of a new UTRA FDD cell	798

8.17.5.2.2	Enhanced UTRA FDD cell identification requirements	798

8.17.5.2.3	UE UTRA FDD CPICH measurement capability	799

8.17.5.2.4	Periodic Reporting	799

8.17.5.2.5	Event Triggered Reporting	799

8.17.5.2.6	Event-triggered Periodic Reporting	800

8.17.5.3	E-UTRAN FDD – UTRAN FDD measurements when DRX is used	800

8.17.5.3.1	Periodic Reporting	801

8.17.5.3.2	Event Triggered Reporting	801

8.17.5.3.3	Event-triggered Periodic Reporting	802

8.17.6	E-UTRAN TDD – UTRAN FDD measurements when Configured with E-UTRA-NR Dual Connectivity	802

8.17.7	E-UTRAN FDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	802

8.17.7.1	Introduction	802

8.17.7.2	Identification of a new UTRA FDD cell for SON	802

8.17.7.2.1	Requirements when no DRX is used	802

8.17.7.2.2	Requirements when DRX is used	803

8.17.7.2.3	Reporting Delay	803

8.17.8	E-UTRAN TDD – UTRAN FDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	804

8.17.9	E-UTRAN TDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	804

8.17.9.1	Introduction	804

8.17.9.2	E-UTRAN FDD – UTRAN TDD measurements when no DRX is used	804

8.17.9.2.1	Identification of a new UTRA FDD cell	804

8.17.9.2.2	Enhanced UTRA TDD cell identification requirements	804

8.17.9.2.3	UE UTRA TDD P-CCPCH RSCP measurement capability	805

8.17.9.2.4	Periodic Reporting	805

8.17.9.2.5	Event Triggered Reporting	805

8.17.9.2.6	Event-triggered Periodic Reporting	806

8.17.9.3	E-UTRAN TDD – UTRAN TDD measurements when DRX is used	806

8.17.9.3.1	Periodic Reporting	807

8.17.9.3.2	Event Triggered Reporting	807

8.17.9.3.3	Event-triggered Periodic Reporting	807

8.17.10	E-UTRAN FDD – UTRAN TDD measurements when Configured with E-UTRA-NR Dual Connectivity	808

8.17.11	E-UTRAN TDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	808

8.17.11.1	Introduction	808

8.17.11.2	Identification of a new UTRA TDD cell for SON	808

8.17.11.2.1	Requirements when no DRX is used	808

8.17.11.2.2	Requirements when DRX is used	808

8.17.11.2.3	Reporting Delay	809

8.17.12	E-UTRAN FDD – UTRAN TDD measurements for SON when Configured with E-UTRA-NR Dual Connectivity	809

8.17.13	E-UTRAN FDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	810

8.17.13.1	Introduction	810

8.17.13.2	E-UTRAN FDD – GSM measurements when no DRX is used	810

8.17.13.2.1	GSM carrier RSSI	810

8.17.13.2.2	BSIC verification	810

8.17.13.2.3	Enhanced BSIC verification	812

8.17.13.2.4	Periodic Reporting	812

8.17.13.2.5	Event Triggered Reporting	812

8.17.13.2.6	Event-triggered Periodic Reporting	813

8.17.13.3	E-UTRAN FDD – GSM measurements when DRX is used	813

8.17.13.3.1	GSM carrier RSSI	813

8.17.13.3.2	BSIC verification	813

8.17.13.3.3	Periodic Reporting	815

8.17.13.3.4	Event Triggered Reporting	815

8.17.13.3.5	Event-triggered Periodic Reporting	815

8.17.14	E-UTRAN TDD – GSM measurements when Configured with E-UTRA-NR Dual Connectivity	815

8.17.15	E-UTRAN Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	815

8.17.15.1	E-UTRAN FDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	815

8.17.15.1.1	RSTD Measurement Reporting Delay	816

8.17.15.2	E-UTRAN TDD-FDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	816

8.17.15.2.1	RSTD Measurement Reporting Delay	816

8.17.15.3	E-UTRAN TDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	816

8.17.15.3.1	RSTD Measurement Reporting Delay	817

8.17.15.4	E-UTRAN FDD-TDD Inter-Frequency RSTD measurements when configured with E-UTRA-NR Dual Connectivity	817

8.17.15.4.1	RSTD Measurement Reporting Delay	817

8.17.16	E-UTRAN intra-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	818

8.17.16.1	Introduction	818

8.17.16.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	818

8.17.16.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	818

8.17.17	E-UTRAN inter-frequency measurement with autonomous gaps when configured with E-UTRA-NR Dual Connectivity	818

8.17.17.1	Introduction	818

8.17.17.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	818

8.17.17.3	E-UTRAN TDD-FDD inter frequency measurements using autonomous gaps	818

8.17.17.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	818

8.17.17.5	E-UTRAN FDD-TDD inter frequency measurements using autonomous gaps	818

8.17.18	E-UTRA FDD - NR CGI measurements with autonomous gaps	818

8.17.19	E-UTRA TDD - NR CGI measurements with autonomous gaps	818

8.18	Measurements for non-BL/CE UE	818

8.18.1	Introduction	818

8.18.2	Requirements for non-BL/CE UE with CE Mode B	819

8.18.2.1	E-UTRAN intra frequency measurements	819

8.18.2.1.1	E-UTRAN FDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	819

8.18.2.1.2	E-UTRAN intra frequency measurements with autonomous gaps for HD-FDD non-BL/CE with CE Mode B	819

8.18.2.1.3	E-UTRAN TDD intra frequency measurements with autonomous gaps for non-BL/CE with CE Mode B	820

8.19	Measurements for NR – E-UTRA Dual Connectivity	821

8.19.1	Introduction	821

8.19.2	Intra-frequency Measurements	821

8.19.3	Inter-frequency Measurements	821

8.19.4	Void	821

8.19.5	Intra-frequency E-CID Measurements	821

8.19.6	Intra-frequency measurements with autonomous gaps	822

8.19.6.1	Introduction	822

8.19.6.2	E-UTRAN FDD intra frequency measurements with autonomous gaps	822

8.19.6.3	E-UTRAN TDD intra frequency measurements with autonomous gaps	822

8.19.7	Inter-frequency measurements with autonomous gaps	822

8.19.7.1	Introduction	822

8.19.7.2	E-UTRAN FDD-FDD inter frequency measurements with autonomous gaps	822

8.19.7.3	E-UTRAN TDD-FDD inter frequency measurements with autonomous gaps	822

8.19.7.4	E-UTRAN TDD-TDD inter frequency measurements with autonomous gaps	822

8.19.7.5	E-UTRAN FDD-TDD inter frequency measurements with autonomous gaps	822

8.20	Inter-RAT NR Measurements for RedCap UE	822

8.20.1	Introduction	822

8.20.2	E-UTRAN FDD – NR measurements	822

8.20.2.1	Identification of a new NR cell	822

8.20.2.2	Periodic Reporting	826

8.20.2.3	Event Triggered Reporting	826

8.20.2.4	Event-triggered Periodic Reporting	826

8.20.3	E-UTRAN TDD – NR measurements	826

9	Measurements performance requirements for UE	827

9.1	E-UTRAN measurements	827

9.1.1	Introduction	827

9.1.2	Intra-frequency RSRP Accuracy Requirements	827

9.1.2.1	Absolute RSRP Accuracy	827

9.1.2.2	Relative Accuracy of RSRP	828

9.1.2.3	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction	829

9.1.2.4	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction	829

9.1.2.5	Absolute RSRP Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	830

9.1.2.6	Relative Accuracy of RSRP under Time Domain Measurement Resource Restriction with CRS assistance information	831

9.1.2.7	Absolute RSRP Accuracy for UE Category 1bis	832

9.1.2.8	Relative Accuracy of RSRP for UE Category 1bis	833

9.1.2A	Intra-frequency RSRP Accuracy Requirements in High Doppler Conditions	834

9.1.2A.1	Absolute RSRP Accuracy in high Doppler conditions	834

9.1.2A.2	Relative Accuracy of RSRP in high Doppler conditions	834

9.1.2B	Intra-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	835

9.1.2B.1	Introduction	835

9.1.2B.2	Intra-frequency Absolute RSRP Accuracy for CA Idle Mode Measurements	835

9.1.3	Inter-frequency RSRP Accuracy Requirements	836

9.1.3.1	Absolute RSRP Accuracy	836

9.1.3.2	Relative Accuracy of RSRP	837

9.1.3.3	Absolute RSRP Accuracy for UE Category 1bis	838

9.1.3.4	Relative Accuracy of RSRP for UE Category 1bis	838

9.1.3A	Inter-frequency RSRP Accuracy Requirements in High Doppler Conditions	839

9.1.3A.1	Absolute RSRP Accuracy in high Doppler conditions	839

9.1.3A.2	Relative Accuracy of RSRP in high Doppler conditions	840

9.1.3B	Inter-frequency RSRP Accuracy requirements for CA Idle Mode Measurements	841

9.1.3B.1	Introduction	841

9.1.3B.2	Inter-frequency Absolute RSRP Accuracy for Overlapping Carrier	841

9.1.3B.3	Inter-frequency Absolute RSRP Accuracy for Overlapping and Non-overlapping Carrier	841

9.1.4	RSRP Measurement Report Mapping	842

9.1.5	Intra-frequency RSRQ Accuracy Requirements	842

9.1.5.1	Absolute RSRQ Accuracy	842

9.1.5.2	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction	843

9.1.5.3	Absolute RSRQ Accuracy under Time Domain Measurement Resource Restriction with CRS assistance information	844

9.1.5.4	Absolute WB-RSRQ Accuracy	845

9.1.5.5	Absolute RSRQ Accuracy for UE Category 1bis	846

9.1.5A	Intra-frequency RSRQ Accuracy Requirements in High Doppler Conditions	847

9.1.5A.1	Absolute RSRQ Accuracy in high Doppler conditions	847

9.1.5B	Intra-frequency RSRQ Accuracy requirements for CA Idle Mode Measurements	847

9.1.5B.1	Introduction	847

9.1.5B.2	Intra-frequency Absolute RSRQ Accuracy for CA Idle Mode Measurements	847

9.1.6	Inter-frequency RSRQ Accuracy Requirements	848

9.1.6.1	Absolute RSRQ Accuracy	848

9.1.6.2	Relative Accuracy of RSRQ	849

9.1.6.3	Absolute WB-RSRQ Accuracy	850

9.1.6.4	Relative WB-RSRQ Accuracy	850

9.1.6.5	Absolute RSRQ Accuracy for UE Category 1bis	851

9.1.6.6	Relative Accuracy of RSRQ for UE Category 1bis	852

9.1.6A	Inter-frequency RSRQ Accuracy Requirements in High Doppler Conditions	852

9.1.6A.1	Absolute RSRQ Accuracy in high Doppler conditions	852

9.1.6A.2	Relative Accuracy of RSRQ in high Doppler conditions	853

9.1.6B	Inter-frequency absolute RSRQ Accuracy requirements for CA Idle Mode Measurements	854

9.1.6B.1	Introduction	854

9.1.6B.2	Inter-frequency Absolute RSRQ Accuracy for Overlapping Carrier	854

9.1.6B.3	Inter-frequency absolute RSRQ Accuracy for Overlapping and Non-overlapping Carrier	854

9.1.7	RSRQ Measurement Report Mapping	855

9.1.8	Power Headroom	855

9.1.8.1	Period	856

9.1.8.2	Reporting Delay	856

9.1.8.3	Void	856

9.1.8.4	Report Mapping	856

9.1.8A	Power Headroom for UE category M1 for satellite access	856

9.1.8A.1	Period	857

9.1.8A.2	Reporting Delay	857

9.1.8A.3	Void	857

9.1.8A.4	Report Mapping	857

9.1.9	UE Rx – Tx time difference	857

9.1.9.1	Measurement Requirement	857

9.1.9.2	Measurement Report mapping	858

9.1.9.3	Measurement Requirement under Time Domain Measurement Resource Restriction	859

9.1.9.4	Measurement Requirement when Time Domain Measurement Resource Restriction Pattern is Configured with CRS Assistance Information	860

9.1.10	Reference Signal Time Difference (RSTD)	861

9.1.10.1	Intra-Frequency Accuracy Requirement	861

9.1.10.2	Inter-Frequency Accuracy Requirement	862

9.1.10.3	RSTD Measurement Report Mapping	864

9.1.10.4	Higher-Resolution RSTD Measurement Report Mapping	864

9.1.10.5	Intra-Frequency Accuracy Requirement for UE Category 1bis	865

9.1.10.6	Inter-Frequency Accuracy Requirement for UE Category 1bis	866

9.1.11	Carrier aggregation measurement accuracy	867

9.1.11.1	Primary component carrier accuracy requirement	868

9.1.11.2	Secondary component carrier accuracy requirement	868

9.1.11.3	Primary and secondary component carrier relative accuracy requirement	868

9.1.11.4	Secondary component carrier relative accuracy requirement	868

9.1.12	Reference Signal Time Difference (RSTD) Measurement Accuracy Requirements for Carrier Aggregation	868

9.1.13	Measurement accuracy for UE category 0	869

9.1.13.1	Intra-frequency Absolute RSRP Accuracy for UE category 0	869

9.1.13.2	Intra-frequency Relative Accuracy of RSRP for UE category 0	869

9.1.13.3	Intra-frequency Absolute RSRQ Accuracy for UE category 0	870

9.1.14	Accuracy requirements for Discovery Signal Measurements	871

9.1.14.1	Introduction	871

9.1.14.2	RSRP measurements in discovery signal occasions	871

9.1.14.3	CSI-RSRP measurements in discovery signal occasions	871

9.1.14.3.1	Intra-frequency CSI-RSRP measurements	871

9.1.14.3.1.1	Absolute CSI-RSRP measurement requirements	871

9.1.14.3.1.2	Relative CSI-RSRP measurement requirements	872

9.1.14.3.2	Inter-frequency CSI-RSRP measurements	873

9.1.14.3.2.1	Absolute CSI-RSRP measurement requirements	873

9.1.14.3.2.2	Relative CSI-RSRP measurement requirements	873

9.1.14.3.3	CSI-RSRP measurement report mapping	874

9.1.14.4	RSRQ measurements in discovery signal occasions	874

9.1.15	Discovery signal measurements accuracy for E-UTRAN carrier aggregation	874

9.1.15.1	Requirements for CRS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	875

9.1.15.1.1	Primary component carrier accuracy requirement	875

9.1.15.1.2	Secondary component carrier accuracy requirement	875

9.1.15.1.3	Primary and secondary component carrier relative accuracy requirement	875

9.1.15.1.4	Secondary component carrier relative accuracy requirement	875

9.1.15.2	Requirements for CSI-RS based discovery signal measurements accuracy for E-UTRAN carrier aggregation	875

9.1.15.2.1	Primary component carrier accuracy requirement	875

9.1.15.2.2	Secondary component carrier accuracy requirement	875

9.1.15.2.3	Primary and secondary component carrier relative accuracy requirement	875

9.1.15.2.4	Secondary component carrier relative accuracy requirement	875

9.1.16	Accuracy requirements for RSRQ measurement on all OFDM symbols	876

9.1.17	RS-SINR Measurements	876

9.1.17.1	Measurement Report Mapping	876

9.1.17.2	Intra-frequency RS-SINR Measurement Accuracy Requirements	877

9.1.17.2.1	Absolute RS-SINR Measurement Accuracy Requirements	877

9.1.17.3	Inter-frequency RS-SINR Measurement Accuracy Requirements	877

9.1.17.3.1	Absolute RS-SINR Measurement Accuracy Requirements	877

9.1.17.3.2	Relative RS-SINR Measurement Accuracy Requirements	878

9.1.18	Accuracy Requirements for Measurements under Operation with Frame Structure 3	879

9.1.18.1	Introduction	879

9.1.18.2	RSRP measurements	879

9.1.18.2.1	RSRP measurement report mapping	879

9.1.18.2.2	Inter-frequency absolute RSRP measurement accuracy requirements	879

9.1.18.2.3	Inter-frequency relative RSRP measurement accuracy requirements	880

9.1.18.2.4	Intra-frequency absolute RSRP measurement accuracy requirements	880

9.1.18.2.5	Intra-frequency relative RSRP measurement accuracy requirements	881

9.1.18.3	RSRQ measurements	881

9.1.18.3.1	RSRQ measurement report mapping	881

9.1.18.3.2	Inter-frequency absolute RSRQ measurement accuracy requirements	881

9.1.18.3.3	Inter-frequency relative RSRQ measurement accuracy requirements	882

9.1.18.3.4	Intra-frequency absolute RSRQ measurement accuracy requirements	882

9.1.18.4	CSI-RSRP measurements	883

9.1.18.4.1	CSI-RSRP measurement report mapping	883

9.1.18.4.2	Inter-frequency absolute CSI-RSRP measurement accuracy requirements	883

9.1.18.4.3	Inter-frequency relative CSI-RSRP measurement accuracy requirements	883

9.1.18.4.4	Intra-frequency absolute CSI-RSRP measurement accuracy requirements	884

9.1.18.4.5	Intra-frequency relative CSI-RSRP measurement accuracy requirements	884

9.1.18.5	RSSI measurements	885

9.1.18.5.1	RSSI measurement report mapping	885

9.1.18.5.2	Intra-frequency absolute RSSI measurement accuracy requirements	885

9.1.18.5.3	Inter-frequency absolute RSSI measurement accuracy requirements	886

9.1.18.6	Channel occupancy measurements	886

9.1.18.6.1	Intra-frequency channel occupancy measurement accuracy requirements	886

9.1.18.6.2	Inter-frequency channel occupancy measurement accuracy requirements	886

9.1.19	Accuracy Requirements for Carrier Aggregation for Measurements under Operation with Frame Structure 3	887

9.1.19.1	Introduction	887

9.1.19.2	Accuracy requirements for measurements on SCC	887

9.1.19.3	Relative accuracy requirements for measurements on different SCCs	887

9.1.19.4	Relative accuracy requirements for measurements on SCC and PCC	887

9.1.20	SFN and Subframe Time Difference (SSTD)	888

9.1.20.1	SSTD Accuracy Requirement	888

9.1.20.2	SSTD Measurement Report Mapping	888

9.1.21	Measurement accuracy for UE category M1	889

9.1.21.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	889

9.1.21.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	891

9.1.21.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	892

9.1.21.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	894

9.1.21.5	RSRP Measurement Report Mapping	895

9.1.21.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	895

9.1.21.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	896

9.1.21.8	RSRQ Measurement Report Mapping	897

9.1.21.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	897

9.1.21.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	898

9.1.21.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	899

9.1.21.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	900

9.1.21.13 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	901

9.1.21.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	902

9.1.21.15 	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	903

9.1.21.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	904

9.1.21.17	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	905

9.1.21.18	Inter-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	906

9.1.21.19	UE RX-TX time difference Accuracy Requirement for Cat-M1	907

9.1.21.20	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode A	908

9.1.21.21	Intra-Frequency RSTD Accuracy Requirement for UE catergory M1 in CE mode B	911

9.1.21.22	Downlink Channel Report Mapping for UE Category M1	914

9.1.21.23	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	914

9.1.21.24	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	917

9.1.21A	Measurement accuracy for UE category M1 for satellite access	918

9.1.21A.1	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	919

9.1.21A.2	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	920

9.1.21A.3	Intra-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	921

9.1.21A.4	Intra-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	923

9.1.21A.5	RSRP Measurement Report Mapping	924

9.1.21A.6	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode A	924

9.1.21A.7	Intra-frequency Absolute Accuracy of RSRQ for UE category M1 with CE mode B	925

9.1.21A.8	RSRQ Measurement Report Mapping	926

9.1.21A.9	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode A	926

9.1.21A.10	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode A	927

9.1.21A.11	Inter-frequency Absolute RSRP Accuracy for UE category M1 with CE mode B	928

9.1.21A.12	Inter-frequency Relative Accuracy of RSRP for UE category M1 with CE mode B	929

9.1.21A.13	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode A	930

9.1.21A.14	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode A	931

9.1.21A.15	Inter-frequency Absolute Accuracy of RSRQ for UE category M1 in CE mode B	932

9.1.21A.16	Inter-frequency Relative Accuracy of RSRQ for UE category M1 in CE mode B	933

9.1.21A.17	Downlink Channel Report Mapping for UE Category M1	934

9.1.21A.18	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode A	934

9.1.21A.19	Downlink Channel Quality Measurement Accuracy for UE Category M1 with CE Mode B	936

9.1.22	Measurement accuracy for UE Category NB1	937

9.1.22.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	937

9.1.22.2	Void	938

9.1.22.3	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	938

9.1.22.4	Void	939

9.1.22.5	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	939

9.1.22.6	Void	940

9.1.22.7	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	940

9.1.22.8	Void	941

9.1.22.9	NRSRP Measurement Report Mapping	941

9.1.22.10	Intra-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	941

9.1.22.11	Inter-Frequency RSTD Accuracy Requirement for NB1 for normal coverage	942

9.1.22.12	Intra-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	943

9.1.22.13	Inter-Frequency RSTD Accuracy Requirement for NB1 for enhanced coverage	944

9.1.22.14	NRSRQ Measurement Report Mapping	945

9.1.22.15	MSG3-based Measurement Report Mapping for UE Category NB1	946

9.1.22.16	Downlink Channel Quality Measurement Accuracy for UE Category NB1	946

9.1.22.17	Channel quality reporting for UE Category NB2 with 16-QAM	947

9.1.22A	Measurement accuracy for UE Category NB1 for satellite access	947

9.1.22A.1	Intra-frequency Absolute NRSRP Accuracy for UE Category NB1	947

9.1.22A.2	Intra-frequency Absolute NRSRQ Accuracy for UE Category NB1	948

9.1.22A.3	Inter-frequency Absolute NRSRP Accuracy for UE Category NB1	949

9.1.22A.4	Inter-frequency Absolute NRSRQ Accuracy for UE Category NB1	950

9.1.22A.5	NRSRP Measurement Report Mapping	951

9.1.22A.6	NRSRQ Measurement Report Mapping	951

9.1.22A.7	MSG3-based Measurement Report Mapping for UE Category NB1	951

9.1.22A.8	Downlink Channel Quality Measurement Accuracy for UE Category NB1	951

9.1.23	Power Headroom for UE Category NB1	952

9.1.23.1	Period	952

9.1.23.2	Reporting Delay	952

9.1.23.3	Report Mapping for UE Category NB1	952

9.1.23.3.1	Void	954

9.1.23.3.2	Void	954

9.1.23.4	Report Mapping for UE Category NB1 for UE Power Class 6	954

9.1.23A	Power Headroom for UE Category NB1 for Satellite Access	955

9.1.23A.1	Period	955

9.1.23A.2	Reporting Delay	955

9.1.23A.3	Report Mapping for UE Category NB1 for Satellite Access	956

9.1.23A.3.1	Void	956

9.1.23A.3.2	Void	956

9.1.23A.4	Report Mapping for UE Category NB1 for UE Power Class 6 for Satellite Access	956

9.1.24	Void	956

9.1.25	Measurement accuracy for UE category M2	956

9.1.25.1	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	956

9.1.25.2	Inter-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	957

9.1.25.3	UE RX-TX time difference Accuracy Requirement for Cat-M2	958

9.1.25.4	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode A	959

9.1.25.5	Intra-Frequency RSTD Accuracy Requirement for UE catergory M2 in CE mode B	961

9.1.26	Measurement Accuracy for non-BL CE UE	962

9.1.26.1	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	963

9.1.26.2	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	965

9.1.26.3	Intra-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	965

9.1.26.4	Intra-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	967

9.1.26.5	RSRP Measurement Report Mapping	968

9.1.26.6	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	968

9.1.26.7	Intra-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	968

9.1.26.8	RSRQ Measurement Report Mapping	969

9.1.26.9	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode A	969

9.1.26.10	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode A	970

9.1.26.11	Inter-frequency Absolute Accuracy of RSRP for non-BL CE UE in CE mode B	971

9.1.26.12	Inter-frequency Relative Accuracy of RSRP for non-BL CE UE in CE mode B	972

9.1.26.13	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode A	973

9.1.26.14	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode A	973

9.1.26.15	Inter-frequency Absolute Accuracy of RSRQ for non-BL CE UE in CE mode B	973

9.1.26.16	Inter-frequency Relative Accuracy of RSRQ for non-BL CE UE in CE mode B	974

9.1.27	SFN and frame Timing Difference (SFTD)	975

9.1.27.1	SFTD Accuracy Requirement	975

9.1.28	SFN and Frame Timing Difference (SFTD) under CCA	977

9.1.28.1	SFTD Accuracy Requirement under CCA	977

9.2	UTRAN FDD Measurements	978

9.2.1	UTRAN FDD CPICH RSCP	978

9.2.2	Void	979

9.2.3	UTRAN FDD CPICH Ec/No	979

9.3	UTRAN TDD Measurements	979

9.3.1	UTRAN TDD P-CCPCH RSCP	980

9.3.2	Void	980

9.3.3	Void	980

9.4	GSM Measurements	980

9.4.1	GSM carrier RSSI	980

9.5	CDMA2000 1x RTT Measurements	980

9.5.1	CDMA2000 1x RTT Pilot Strength	980

9.6	PCMAX,c	981

9.6.1	Report Mapping	981

9.6.2 	Estimation Period	981

9.6.3	Reporting Delay	981

9.7	IEEE802.11 Measurements	981

9.7.1	WLAN RSSI	981

9.7.2	WLAN RSSI Measurement Report Mapping	981

9.8	MBSFN Measurements	982

9.8.1	Introduction	982

9.8.2	MBSFN RSRP	982

9.8.2.1	Absolute MBSFN RSRP measurement accuracy requirements	982

9.8.2.2	MBSFN RSRP measurement report mapping	983

9.8.2.3	MBSFN RSRP measurement report mapping for 7.5 kHz subcarrier spacing	983

9.8.2.4	MBSFN RSRP measurement report mapping for 1.25 kHz subcarrier spacing	984

9.8.2.5	MBSFN RSRP measurement report mapping for 2.5 kHz subcarrier spacing	984

9.8.2.6	MBSFN RSRP measurement report mapping for 370.37Hz subcarrier spacing	984

9.8.3	MBSFN RSRQ	985

9.8.3.1	Absolute MBSFN RSRQ measurement accuracy requirements	985

9.8.3.2	MBSFN RSRQ measurement report mapping	985

9.8.3.3	MBSFN RSRQ measurement report mapping for 7.5 kHz subcarrier spacing	986

9.8.3.4	MBSFN RSRQ measurement report mapping for 1.25 kHz subcarrier spacing	986

9.8.3.5	MBSFN RSRQ measurement report mapping for 2.5 kHz subcarrier spacing	986

9.8.3.6	MBSFN RSRQ measurement report mapping for 370.37 kHz subcarrier spacing	987

9.8.4	MCH BLER	987

9.8.4.1	Measurement report mapping for MCH BLER	987

9.8.4.2	Measurement report mapping for MCH Block Number	988

9.9	ProSe Measurements	989

9.9.1	Introduction	989

9.9.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	989

9.9.2.1	Absolute S-RSRP Accuracy	989

9.9.2.2	Relative Accuracy of S-RSRP	990

9.9.3	Intra-Frequency SD-RSRP Measurement Accuracy Requirements	991

9.9.3.1	Absolute SD-RSRP Accuracy	991

9.9.3.2	Relative Accuracy of SD-RSRP	991

9.10	V2X Measurements	992

9.10.1	Introduction	992

9.10.2	Intra-Frequency S-RSRP Measurement Accuracy Requirements	992

9.10.2.1	Absolute S-RSRP Accuracy	992

9.10.2.2	Relative Accuracy of S-RSRP	993

9.10.3	PSSCH-RSRP Measurement Accuracy Requirements	993

9.10.3.1	Intra-frequency Absolute PSSCH-RSRP Accuracy	993

9.10.4	S-RSSI Measurement Accuracy Requirements	994

9.10.4.1	Intra-frequency absolute S-RSSI measurement accuracy requirements	994

9.10.4.2 Intra-frequency relative S-RSSI measurement accuracy requirements	994

9.11	NR Measurements	995

9.11.1	NR SS-RSRP Measurements	995

9.11.1A	NR SS-RSRP Measurements for DC Idle Mode Measurements	995

9.11.2	NR SS-RSRQ Measurements	996

9.11.2A	NR SS-RSRQ Measurements for DC Idle Mode Measurements	996

9.11.3	NR SS-SINR Measurements	996

9.11.4	NR SS-RSRP Measurements under CCA	997

9.11.5	NR SS-RSRQ Measurements under CCA	997

9.11.6	NR SS-SINR Measurements under CCA	997

9.11.7	NR RSSI Measurements under CCA	997

9.11.8	NR Channel Occupancy Measurements under CCA	998

10	Measurements Performance Requirements for E-UTRAN	998

10.1	Received Interference Power	998

10.1.1	Absolute accuracy requirement	998

10.1.2	Relative accuracy requirement	998

10.1.3	Received Interference Power measurement report mapping	999

10.2	Angle of Arrival (AOA)	999

10.2.1	Range/mapping	999

10.3	Timing Advance (TADV)	999

10.3.1	Report mapping	999

11	ProSe Requirements in Any Cell Selection state	1000

11.1	Introduction	1000

11.2	UE Transmit Timing for ProSe in Any Cell Selection State	1000

11.2.1	Introduction	1000

11.2.2	ProSe UE transmission timing	1000

11.3	Initiation/Cease of SLSS Transmissions	1001

11.3.1	Introduction	1001

11.3.2	Requirements	1001

11.4	Measurements for ProSe in Any Cell Selection State	1001

11.4.1	Introduction	1001

11.4.2	Requirements	1001

11.4.2.1	E-UTRA FDD	1001

11.4.2.2	E-UTRA TDD	1002

11.5	Selection / Reselection of ProSe Synchronization Reference	1002

11.5.1	Introduction	1002

11.5.2	Selection/Reselection to intra-frequency SyncRef UE	1002

11.5.2.1	Introduction	1002

11.5.2.2	Requirements	1002

11.6	Void	1003

11.7	Selection / Reselection of ProSe relay UE	1003

11.7.1	Introduction	1003

11.7.2	Selection / Reselection of intra-frequency ProSe relay UE	1003

12	V2V Sidelink Communication Requirements for V2V Operation on Dedicated V2V Carrier	1003

12.1	Introduction	1003

12.2	Transmit Timing	1004

12.2.1	GNSS as timing reference	1004

12.3	Interruption	1004

12.4	Reliability of GNSS signal	1004

13	V2X Requirements	1004

13.1	Introduction	1004

13.2	UE Transmit Timing	1005

13.2.1	Introduction	1005

13.2.2	GNSS as synchronization reference source	1005

13.2.3	Serving cell/PCell as synchronization reference source	1005

13.2.4	SyncRef UE as synchronization reference source	1005

13.3	Initiation/Cease of SLSS Transmissions	1005

13.3.1	Introduction	1005

13.3.1.1	Initiation/Cease of SLSS transmissions with Serving cell / PCell as synchronization reference source	1005

13.3.1.2	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	1006

13.3.1.3	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	1006

13.4	Selection / Reselection of V2X Synchronization Reference Source	1007

13.5	Autonomous Resource Selection/Reselection measurements	1008

13.5.1	Introduction	1008

13.5.2	PSSCH-RSRP measurements	1008

13.5.3	S-RSSI measurements	1008

13.6	Congestion Control measurements	1008

13.7	Interruption	1008

13.7.1	Interruptions to WAN due to V2X Sidelink Communication	1008

13.7.2	V2X Sidelink Communication Dropping due to synchronization reference source change	1008

13.7.3	Interruptions to WAN due to V2X Carrier Aggregation	1009

13.7.4	Interruptions to WAN due to NR V2X sidelink communication	1009

13.8	Reliability of GNSS signal	1009

13.9	Component Carrier Addition and Release Delay for V2X Sidelink Carrier Aggregation	1010

13.10	Selection / Reselection of V2X Synchronization Reference Source for V2X Carrier Aggregation	1010

Annex A (normative):	Test Cases	1011

A.1	Purpose of annex	1011

A.2	Requirement classification for statistical testing	1011

A.2.1	Types of requirements in TS 36.133	1011

A.2.1.1	Time and delay requirements on UE higher layer actions	1011

A.2.1.2	Measurements of power levels, relative powers and time	1011

A.2.1.3	Implementation requirements	1012

A.2.1.4	Physical layer timing requirements	1012

A.3	RRM test configurations	1013

A.3.1	Reference Measurement Channels	1013

A.3.1.1	PDSCH	1013

A.3.1.1.1	FDD	1013

A.3.1.1.2	TDD	1018

A.3.1.1.3	FDD for UE category 0	1021

A.3.1.1.4	HD-FDD for UE category 0	1022

A.3.1.1.5	TDD for UE category 0	1023

A.3.1.1.6	Frame Structure 3	1024

A.3.1.2	PCFICH/PDCCH/PHICH	1025

A.3.1.2.1	FDD	1025

A.3.1.2.2	TDD	1025

A.3.1.2.3	HD-FDD for UE category 0	1026

A.3.1.2.4	FS 3	1026

A.3.1.3	MPDCCH Reference Channels for Cat-M1 UEs	1026

A.3.1.3.1	FDD in CEModeA	1027

A.3.1.3.2	HD-FDD in CEModeA	1027

A.3.1.3.3	TDD in CEModeA	1028

A.3.1.3.4	FDD in CEModeB	1028

A.3.1.3.5	HD-FDD in CEModeB	1029

A.3.1.3.6	TDD in CEModeB	1029

A.3.1.4	PDSCH Reference Channel for Cat-M1 UEs	1030

A.3.1.4.1	FDD in CEModeA	1030

A.3.1.4.2	HD-FDD in CEModeA	1031

A.3.1.4.3	TDD in CEModeA	1032

A.3.1.4.4	FDD in CEModeB	1033

A.3.1.4.5	HD-FDD in CEModeB	1034

A.3.1.4.6	TDD in CEModeB	1034

A.3.1.5	NPDSCH Reference Channel for UE category NB1	1035

A.3.1.5.1	HD-FDD in-band operation	1035

A.3.1.5.2	Void	1036

A.3.1.5.3	HD-FDD standalone operation	1036

A.3.1.5.4	Void	1037

A.3.1.5.5	HD-FDD guard band operation	1037

A.3.1.5.6	Void	1038

A.3.1.5.7	TDD in-band operation	1038

A.3.1.5.8	TDD standalone operation	1038

A.3.1.5.9	TDD guard band operation	1039

A.3.1.6	NPDCCH Reference Channel for UE category NB1	1039

A.3.1.6.1	In-band operation	1039

A.3.1.6.2	Void	1040

A.3.1.6.3	Standalone operation	1040

A.3.1.6.4	Void	1040

A.3.1.6.5	Guard band operation	1040

A.3.1.6.6	Void	1041

A.3.2	OFDMA Channel Noise Generator (OCNG)	1041

A.3.2.1	OCNG Patterns for FDD	1041

A.3.2.1.1	OCNG FDD pattern 1: outer resource blocks allocation in 10 MHz	1042

A.3.2.1.2	OCNG FDD pattern 2: full bandwidth allocation in 10 MHz	1042

A.3.2.1.3	OCNG FDD pattern 3: outer resource blocks allocation in 1.4 MHz	1043

A.3.2.1.4	OCNG FDD pattern 4: full bandwidth allocation in 1.4 MHz	1044

A.3.2.1.5	OCNG FDD pattern 5: outer resource blocks allocation in 10 MHz (without MBSFN)	1044

A.3.2.1.6	OCNG FDD pattern 6: full bandwidth allocation in 10 MHz (without MBSFN)	1045

A.3.2.1.7	OCNG FDD pattern 7: full bandwidth allocation in 1.4 MHz (without MBSFN)	1045

A.3.2.1.8	OCNG FDD pattern 8: outer resource blocks allocation in 10 MHz for MBSFN ABS	1045

A.3.2.1.9	OCNG FDD pattern 9: full bandwidth allocation in 10 MHz for MBSFN ABS	1046

A.3.2.1.10	OCNG FDD pattern 10: outer resource blocks allocation in 10 MHz with user data in every subframe (without MBSFN)	1047

A.3.2.1.11	OCNG FDD pattern 11: outer resource blocks allocation in 20 MHz	1047

A.3.2.1.12	OCNG FDD pattern 12: full bandwidth allocation in 20 MHz	1048

A.3.2.1.13	OCNG FDD pattern 13: outer resource blocks allocation in 20 MHz (without MBSFN)	1048

A.3.2.1.14	OCNG FDD pattern 14: full bandwidth allocation in 20 MHz (without MBSFN)	1049

A.3.2.1.15	OCNG FDD pattern 15: outer resource blocks allocation in 5 MHz	1049

A.3.2.1.16	OCNG FDD pattern 16: full bandwidth allocation in 5 MHz	1050

A.3.2.1.17	OCNG FDD pattern 17: outer resource blocks allocation in 20 MHz with user data in every subframe (without MBSFN)	1050

A.3.2.1.18	OCNG FDD pattern 18: outer resource blocks allocation in 5 MHz (without MBSFN)	1051

A.3.2.1.19	OCNG FDD pattern 19: full bandwidth allocation in 5 MHz (without MBSFN)	1051

A.3.2.1.20	OCNG FDD pattern 20: outer resource blocks allocation in 5 MHz with user data in every subframe (without MBSFN)	1052

A.3.2.1.21	OCNG FDD pattern 21: Generic resource blocks allocation (without MBSFN)	1052

A.3.2.1.22	OCNG FDD pattern 22: Generic resource blocks allocation in 5MHz (without MBSFN)	1053

A.3.2.2	OCNG Patterns for TDD	1053

A.3.2.2.1	OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1054

A.3.2.2.2	OCNG TDD pattern 2: full bandwidth allocation in 10 MHz	1054

A.3.2.2.3	OCNG TDD pattern 3: outer resource blocks allocation in 1.4 MHz	1055

A.3.2.2.4	OCNG TDD pattern 4: full bandwidth allocation in 1.4 MHz	1055

A.3.2.2.5	OCNG TDD pattern 5: outer resource blocks allocation in 10 MHz for MBSFN ABS	1056

A.3.2.2.6	OCNG TDD pattern 6: full bandwidth allocation in 10 MHz for MBSFN ABS	1057

A.3.2.2.7	OCNG TDD pattern 7: outer resource blocks allocation in 20 MHz	1058

A.3.2.2.8	OCNG TDD pattern 8: full bandwidth allocation in 20 MHz	1059

A.3.2.2.9	OCNG TDD pattern 9: outer resource blocks allocation in 5 MHz	1059

A.3.2.2.10	OCNG TDD pattern 10: full bandwidth allocation in 5 MHz	1060

A.3.2.2.11	OCNG TDD pattern 11: Generic resource blocks allocation (without MBSFN)	1061

A.3.2.3	OCNG Patterns for Narrowband IoT	1061

A.3.2.3.1	Narrowband IoT OCNG FDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1063

A.3.2.3.2	Narrowband IoT OCNG FDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1064

A.3.2.3.3	Narrowband IoT OCNG FDD pattern 3: standalone NB-IoT	1064

A.3.2.3.4	Narrowband IoT OCNG FDD pattern 4: In-band NB-IoT in 5 MHz EUTRAN cell	1065

A.3.2.3.5	Narrowband IoT OCNG FDD pattern 5: guard band NB-IoT in 5 MHz EUTRAN cell	1066

A.3.2.3.6	Narrowband IoT OCNG TDD pattern 1: In-band NB-IoT in 10 MHz EUTRAN cell	1067

A.3.2.3.7	Narrowband IoT OCNG TDD pattern 2: guard band NB-IoT in 10 MHz EUTRAN cell	1069

A.3.2.3.8	Narrowband IoT OCNG TDD pattern 3: standalone NB-IoT	1069

A.3.2.3.9	Narrowband IoT OCNG FDD pattern 6: In-band NB-IoT in 5 MHz NTN NR cell	1070

A.3.2.4	OCNG Patterns for V2X sidelink	1071

A.3.2.4.1	V2X sidelink OCNG TDD pattern 1: outer resource blocks allocation in 10 MHz	1072

A.3.2.4.2	V2X sidelink OCNG TDD pattern 2: outer resource blocks allocation in 10 MHz	1072

A.3.3	Reference DRX Configurations	1072

A.3.4	ABS Transmission Configurations	1073

A.3.4.1	Non-MBSFN ABS Transmission Configurations	1073

A.3.4.1.1	Non-MBSFN ABS Transmission, 1x2 antenna with PBCH	1073

A.3.4.1.2	Non-MBSFN ABS Transmission, 2x2 antenna without PBCH	1073

A.3.4.2	MBSFN ABS Transmission Configurations	1074

A.3.4.2.1	MBSFN ABS Transmission, 1x2 antenna	1074

A.3.4.2.2	MBSFN ABS Transmission, 2x2 antenna	1075

A.3.5	Impact of Reference Sensitivity Degradation with Carrier Aggregation on Test Cases	1075

A.3.5.1	Impact of Reference Sensitivity Degradation due to Insertion Loss	1075

A.3.6	Carrier Aggregation Test Cases with Different Channel Bandwidth Combinations	1076

A.3.6.1	Introduction	1076

A.3.7	Test Cases with Different Channel Bandwidths	1076

A.3.7.1	Introduction	1076

A.3.7.2	Principle of testing	1076

A.3.8	Antenna Configuration	1076

A.3.8.1	Antenna connection for 4 Rx capable UEs	1076

A.3.8.1.1 Introduction	1076

A.3.8.1.2 Principle of testing	1076

A.3.8.1.2.1 Single carrier tests	1076

A.3.8.1.2.2	Carrier aggregation and Dual connectivity tests	1077

A.3.8.1.2.3	Antenna connection for bands where 2RX is supported	1078

A.3.8.1.2.4	Antenna connection for bands where 4RX is supported	1078

A.3.8.2	Antenna connection for 8 Rx capable UEs	1078

A.3.8.2.1	Introduction	1078

A.3.8.2.2	Principle of testing	1078

A.3.8.2.2.1	Single carrier tests	1078

A.3.8.2.2.2	Carrier aggregation and Dual connectivity tests	1079

A.3.8.2.2.3	Antenna connection for bands where 2RX is supported	1079

A.3.8.2.2.4	Antenna connection for bands where 4RX is supported	1079

A.3.8.2.2.5	Antenna connection for bands where 8RX is supported	1079

A.3.9	Carrier Aggregation Test Cases with Different Duplex Modes	1079

A.3.9.1	Introduction	1079

A.3.9.2	Principle of testing	1079

A.3.10	Carrier Aggregation Test Cases with Different CA Configurations	1080

A.3.10.1	Introduction	1080

A.3.10.2	Principle of testing	1080

A.3.11	Test Cases for Synchronous and Asynchronous Dual Connectivity	1080

A.3.11.1	Introduction	1080

A.3.11.2	Principle of Testing	1080

A.3.12	Proximity-based Services	1080

A.3.12.1	Introduction	1080

A.3.12.2	Reference DRX configurations for ProSe tests	1080

A.3.12.3	Test Cases with Different Channel Bandwidths	1081

A.3.12.3.1	Introduction	1081

A.3.12.3.2	Principle of testing	1081

A.3.12.4	Reference resource pool configurations for ProSe Direct Discovery	1081

A.3.12.5	Reference resource pool configurations for ProSe Direct Communication	1088

A.3.12.6	Reference Measurement Channels for ProSe Direct Discovery	1091

A.3.12.6.1	FDD	1091

A.3.12.7	Reference measurement channels for ProSe Direct Communication	1091

A.3.12.7.1	FDD	1091

A.3.12.8	ProSe Receive Traffic Generator	1092

A.3.12.8.1	ProSe Direct Communication Receive Traffic Generator for FDD	1092

A.3.12.8.2	ProSe Direct Discovery Receive Traffic Generator for FDD	1092

A.3.13	Time Offset between Cells	1093

A.3.13.1	Introduction	1093

A.3.13.2	Definition	1093

A.3.14	Carrier Aggregation under operation with Frame Structure 3 Test Cases with Different Duplex Modes	1093

A.3.14.1	Introduction	1093

A.3.14.2	Principle of testing	1093

A.3.15	Dual connectivity test cases with different combination of duplex mode	1093

A.3.15.1	Introduction	1093

A.3.15.2	Principle of testing	1093

A.3.16	Reference PRACH Configurations	1094

A.3.17	Listen before talk model	1094

A.3.17.1	Introduction	1094

A.3.17.2	Definition	1094

A.3.18	Reference NPRACH Configurations	1095

A.3.19	Dual connectivity test cases with different bandwidth combinations	1096

A.3.19.1	Introduction	1096

A.3.19.2	Principle of testing	1096

A.3.20	Category M1 UE Test Cases	1097

A.3.20.1	Introduction	1097

A.3.20.2	Principle of Cat-M1 UE Testing	1097

A.3.20.3	Principle of Cat-M1 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1098

A.3.21	V2V Sidelink Communication on Dedicated V2V Carrier	1099

A.3.21.1	Introduction	1099

A.3.21.2	Reference resource pool configurations for V2V Sidelink Communication	1099

A.3.21.3	Reference measurement channels for V2V Sidelink Communication	1100

A.3.22	Category 1bis UE Test Cases	1101

A.3.22.1	Introduction	1101

A.3.22.2	Principle of Category 1bis UE Testing	1101

A.3.23	Category NB2 UE Test Cases	1105

A.3.23.1	Introduction	1105

A.3.23.2	Principle of Category NB2 UE Testing	1105

A.3.24	V2X sidelink communication	1108

A.3.24.1	Introduction	1108

A.3.24.2	Reference resource pool configurations for V2X Sidelink Communication	1109

A.3.24.3	Reference measurement channels for V2X Sidelink Communication	1113

A.3.25	Category M2 UE Test Cases	1114

A.3.25.1	Introduction	1114

A.3.25.2	Principle of Cat-M2 UE Testing	1114

A.3.25.3	Principle of Cat-M2 UE testing for inter-frequency RSTD measurement period requirements with measurement gaps	1115

A.3.26	sTTI and processing time reduction test cases with different sTTI/processing time reduction scheme	1116

A.3.26.1	Introduction	1116

A.3.26.2	Principle of testing	1116

A.3.27	LTE INACTIVE Cell Re-selection Test Cases	1116

A.3.27.1	Introduction	1116

A.3.27.2	Principle of INACTIVE cell re-selection Testing	1116

A.3.28	Testing related to Satellite access	1116

A.3.28.1	Introduction	1116

A.3.28.2	Principle of testing GSO and NGSO scenarios	1116

A.3.28.2	Principle of testing different RRM requirements	1117

A.3.28.3	Principle of testing different ephemeris formats	1121

A.3.28.4	General setup for SIB31/SIB-31-NB	1121

A.3.28.5	Satellite specific parameters configuration	1121

A.3.28.5.1	Satellite specific configuration for serving cell	1121

A.3.28.5.2	Satellite specific configuration for neighbour cell	1121

A.4	E-UTRAN RRC_IDLE state	1122

A.4.2	Cell Re-Selection	1122

A.4.2.1	E-UTRAN FDD – FDD Intra frequency case	1122

A.4.2.1.1	Test Purpose and Environment	1122

A.4.2.1.2	Test Requirements	1124

A.4.2.2	E-UTRAN TDD – TDD Intra frequency case	1125

A.4.2.2.1	Test Purpose and Environment	1125

A.4.2.2.2	Test Requirements	1126

A.4.2.3	E-UTRAN FDD – FDD Inter frequency case	1127

A.4.2.3.1	Test Purpose and Environment	1127

A.4.2.3.2	Test Requirements	1128

A.4.2.4	E-UTRAN FDD – TDD Inter frequency case	1129

A.4.2.4.1	Test Purpose and Environment	1129

A.4.2.4.2	Test Requirements	1130

A.4.2.5	E-UTRAN TDD – FDD Inter frequency case	1131

A.4.2.5.1	Test Purpose and Environment	1131

A.4.2.5.2	Test Requirements	1132

A.4.2.6	E-UTRAN TDD – TDD: Inter frequency case	1133

A.4.2.6.1	Test Purpose and Environment	1133

A.4.2.6.2	Test Requirements	1134

A.4.2.7	E-UTRAN FDD – FDD Inter frequency case in the existence of non-allowed CSG cell	1135

A.4.2.7.1	Test Purpose and Environment	1135

A.4.2.7.2	Test Requirements	1136

A.4.2.8	E-UTRAN TDD – TDD Inter frequency case in the existence of non-allowed CSG cell	1137

A.4.2.8.1	Test Purpose and Environment	1137

A.4.2.8.2	Test Requirements	1138

A.4.2.9	E-UTRAN FDD – FDD Intra frequency case for 5MHz bandwidth	1139

A.4.2.9.1	Test Purpose and Environment	1139

A.4.2.9.2	Test Requirements	1139

A.4.2.10	E-UTRAN FDD – FDD reselection using an increased number of carriers	1139

A.4.2.10.1	Test Purpose and Environment	1139

A.4.2.10.2	Test Requirements	1143

A.4.2.11	E-UTRAN TDD – TDD reselection using an increased number of carriers	1143

A.4.2.11.1	Test Purpose and Environment	1143

A.4.2.11.2	Test Requirements	1147

A.4.2.12	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1147

A.4.2.12.1	Test Purpose and Environment	1147

A.4.2.12.2	Test Requirements	1149

A.4.2.13	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	1150

A.4.2.13.1	Test Purpose and Environment	1150

A.4.2.13.2	Test Requirements	1151

A.4.2.14	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1152

A.4.2.14.1	Test Purpose and Environment	1152

A.4.2.14.2	Test Requirements	1153

A.4.2.15 E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1154

A.4.2.15.1	Test Purpose and Environment	1154

A.4.2.15.2	Test Requirements	1155

A.4.2.16 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage	1156

A.4.2.16.1	Test Purpose and Environment	1156

A.4.2.16.2	Test Requirements	1157

A.4.2.17 E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in enhanced coverage	1158

A.4.2.17.1	Test Purpose and Environment	1158

A.4.2.17.2	Test Requirements	1159

A.4.2.18 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1160

A.4.2.18.1	Test Purpose and Environment	1160

A.4.2.18.2	Test Requirements	1162

A.4.2.19	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1163

A.4.2.19.1	Test Purpose and Environment	1163

A.4.2.19.2	Test Requirements	1165

A.4.2.20	E-UTRAN FDD – FDD Intra frequency case for UE Category 1bis	1166

A.4.2.20.1	Test Purpose and Environment	1166

A.4.2.20.2	Test Requirements	1167

A.4.2.21	E-UTRAN TDD – TDD Intra frequency case for UE Category 1bis	1168

A.4.2.21.1	Test Purpose and Environment	1168

A.4.2.21.2	Test Requirements	1169

A.4.2.22	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1170

A.4.2.22.1	Test Purpose and Environment	1170

A.4.2.22.2	Test Requirements	1171

A.4.2.23	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhancedMeasFlag	1172

A.4.2.23.1	Test Purpose and Environment	1172

A.4.2.23.2	Test Requirements	1173

A.4.2.24	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1174

A.4.2.24.1	Test Purpose and Environment	1174

A.4.2.24.2	Test Requirements	1177

A.4.2.25	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1178

A.4.2.25.1	Test Purpose and Environment	1178

A.4.2.25.2	Test Requirements	1179

A.4.2.26	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1180

A.4.2.26.1	Test Purpose and Environment	1180

A.4.2.26.2	Test Requirements	1181

A.4.2.27	E-UTRAN TDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	1182

A.4.2.27.1	Test Purpose and Environment	1182

A.4.2.27.2	Test Requirements	1183

A.4.2.28	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1184

A.4.2.28.1	Test Purpose and Environment	1184

A.4.2.28.2	Test Requirements	1185

A.4.2.29	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage	1186

A.4.2.29.1	Test Purpose and Environment	1186

A.4.2.29.2	Test Requirements	1187

A.4.2.30	E-UTRAN TDD Inter frequency case for Cat-M1 UE in enhanced coverage	1188

A.4.2.30.1	Test Purpose and Environment	1188

A.4.2.30.2	Test Requirements	1189

A.4.2.31	E-UTRAN FDD – FDD Inter frequency case for UE Category 1bis	1190

A.4.2.31.1	Test Purpose and Environment	1190

A.4.2.31.2	Test Requirements	1191

A.4.2.32	E-UTRAN FDD – TDD Inter frequency case for UE Category 1bis	1192

A.4.2.32.1	Test Purpose and Environment	1192

A.4.2.32.2	Test Requirements	1193

A.4.2.33	E-UTRAN TDD – FDD Inter frequency case for UE Category 1bis	1194

A.4.2.33.1	Test Purpose and Environment	1194

A.4.2.33.2	Test Requirements	1195

A.4.2.34	E-UTRAN TDD – TDD: Inter frequency case for UE Category 1bis	1196

A.4.2.34.1	Test Purpose and Environment	1196

A.4.2.34.2	Test Requirements	1197

A.4.2.35	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage	1198

A.4.2.35.1	Test Purpose and Environment	1198

A.4.2.35.2	Test Requirements	1200

A.4.2.36	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage	1201

A.4.2.36.1	Test Purpose and Environment	1201

A.4.2.36.2	Test Requirements	1203

A.4.2.37	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage	1204

A.4.2.37.1	Test Purpose and Environment	1204

A.4.2.37.2	Test Requirements	1206

A.4.2.38	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation	1207

A.4.2.38.1	Test Purpose and Environment	1207

A.4.2.38.2	Test Requirements	1209

A.4.2.39	E-UTRAN FDD – FDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1210

A.4.2.39.1	Test Purpose and Environment	1210

A.4.2.39.2	Test Requirements	1211

A.4.2.40	E-UTRAN TDD – TDD Intra frequency case for UE configured with highSpeedEnhMeasFlag2-r16	1212

A.4.2.40.1	Test Purpose and Environment	1212

A.4.2.40.2	Test Requirements	1213

A.4.2.41 HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1214

A.4.2.41.1	Test Purpose and Environment	1214

A.4.2.41.2	Test Requirements	1217

A.4.2.42	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1218

A.4.2.42.1	Test Purpose and Environment	1218

A.4.2.42.2	Test Requirements	1220

A.4.2.43	HD – FDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1221

A.4.2.43.1	Test Purpose and Environment	1221

A.4.2.43.2	Test Requirements	1223

A.4.2.44	E-UTRAN TDD - TDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with UE specific DRX	1224

A.4.2.44.1	Test Purpose and Environment	1224

A.4.2.44.2	Test Requirements	1226

A.4.2.45	E-UTRAN TDD – TDD Intra frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1227

A.4.2.45.1	Test Purpose and Environment	1227

A.4.2.45.2	Test Requirements	1229

A.4.2.46	E-UTRAN TDD – TDD Inter frequency case for UE Category NB1 In-Band mode in enhanced coverage with UE specific DRX	1230

A.4.2.46.1	Test Purpose and Environment	1230

A.4.2.46.2	Test Requirements	1232

A.4.2.47	HD – FDD Intra frequency case for UE Category NB1 In-Band mode in normal coverage with serving cell RRM measurement relaxation with UE specific DRX	1233

A.4.2.47.1	Test Purpose and Environment	1233

A.4.2.47.2	Test Requirements	1235

A.4.2.48	E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1236

A.4.2.48.1	Test Purpose and Environment	1236

A.4.2.48.2	Test Requirements	1237

A.4.2.49	E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1238

A.4.2.49.1	Test Purpose and Environment	1238

A.4.2.49.2	Test Requirements	1240

A.4.2.50	E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in normal coverage	1241

A.4.2.50.1	Test Purpose and Environment	1241

A.4.2.50.2	Test Requirements	1243

A.4.2.51 E-UTRAN FD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1244

A.4.2.51.1	Test Purpose and Environment	1244

A.4.2.51.2	Test Requirements	1246

A.4.2.52 E-UTRAN HD-FDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1247

A.4.2.52.1	Test Purpose and Environment	1247

A.4.2.52.2	Test Requirements	1249

A.4.2.53 E-UTRAN TDD RSS based Intra frequency case for Cat-M1 UE in enhanced coverage	1250

A.4.2.53.1	Test Purpose and Environment	1250

A.4.2.53.2	Test Requirements	1251

A.4.2.54	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1252

A.4.2.54.1	Test Purpose and Environment	1252

A.4.2.54.2	Test Requirements	1254

A.4.2.55	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	1255

A.4.2.55.1	Test Purpose and Environment	1255

A.4.2.55.2	Test Requirements	1257

A.4.2.56	E-UTRAN TDD – TDD Intra frequency case for Cat-M1 UE in normal coverage	1258

A.4.2.56.1	Test Purpose and Environment	1258

A.4.2.56.2	Test Requirements	1259

A.4.3	E-UTRAN to UTRAN Cell Re-Selection	1260

A.4.3.1	E-UTRAN FDD – UTRAN FDD:	1260

A.4.3.1.1	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of higher priority	1260

A.4.3.1.1.1	Test Purpose and Environment	1260

A.4.3.1.1.2	Test Requirements	1262

A.4.3.1.2	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority	1262

A.4.3.1.2.1	Test Purpose and Environment	1262

A.4.3.1.2.2	Test Requirements	1265

A.4.3.1.3	EUTRA FDD-UTRA FDD cell reselection in fading propagation conditions: UTRA FDD is of lower priority	1265

A.4.3.1.3.1	Test Purpose and Environment	1265

A.4.3.1.3.2	Test Requirements	1268

A.4.3.1.4	EUTRA FDD-UTRA FDD cell reselection: UTRA FDD is of lower priority for 5MHz bandwidth	1268

A.4.3.1.4.1	Test Purpose and Environment	1268

A.4.3.1.4.2	Test Requirements	1269

A.4.3.1.5	Idle mode FDD to UTRA FDD interRAT reselection	1269

A.4.3.1.5.1	Test Purpose and Environment	1269

A.4.3.1.5.2	Test Requirements	1272

A.4.3.2	E-UTRAN FDD – UTRAN TDD:	1273

A.4.3.2.1	Test Purpose and Environment	1273

A.4.3.2.1.1	Void	1273

A.4.3.2.1.2	1.28Mcps TDD option	1273

A.4.3.2.1.3	Void	1275

A.4.3.2.2	Test Requirements	1275

A.4.3.2.2.1	1.28Mcps TDD option	1275

A.4.3.2A	E-UTRA FDD to UTRA TDD cell re-selection for IncMon	1275

A.4.3.2A.1	Test Purpose and Environment	1275

A.4.3.2A.2	Test Requirements	1279

A.4.3.3	E-UTRAN TDD – UTRAN FDD:	1279

A.4.3.3.1	Test Purpose and Environment	1279

A.4.3.3.2	Test Requirements	1282

A.4.3.3A	Idle mode TDD to UTRA FDD interRAT reselection	1282

A.4.3.3A.1	Test Purpose and Environment	1282

A.4.3.3A.2	Test Requirements	1287

A.4.3.4	E-UTRAN TDD – UTRAN TDD:	1288

A.4.3.4.1	E-UTRA to UTRA TDD cell re-selection: UTRA is of higher priority	1288

A.4.3.4.1.1	Test Purpose and Environment	1288

A.4.3.4.1.2	Test Requirements	1290

A.4.3.4.2	E-UTRA to UTRA TDD cell re-selection: UTRA is of lower priority	1290

A.4.3.4.2.1	Test Purpose and Environment	1290

A.4.3.4.2.2	Test Requirements	1292

A.4.3.4.3	EUTRA TDD-UTRA TDD cell reselection in fading propagation conditions: UTRA TDD is of lower priority	1292

A.4.3.4.3.1	Test Purpose and Environment	1292

A.4.3.4.3.2	Test Requirements	1295

A.4.3.4.4	E-UTRA TDD to UTRA TDD cell re-selection for IncMon	1295

A.4.3.4.4.1	Test Purpose and Environment	1295

A.4.3.4.4.2	Test Requirements	1299

A.4.4	E-UTRAN to GSM Cell Re-Selection	1299

A.4.4.1	E-UTRAN FDD – GSM:	1299

A.4.4.1.1	Test Purpose and Environment	1299

A.4.4.1.2	Test Requirements	1301

A.4.4.2	E-UTRAN TDD – GSM:	1301

A.4.4.2.1	Test Purpose and Environment	1301

A.4.4.2.2	Test Requirements	1303

A.4.5	E-UTRAN to HRPD Cell Re-Selection	1304

A.4.5.1	E-UTRAN FDD – HRPD	1304

A.4.5.1.1	E-UTRAN FDD – HRPD Cell Reselection: HRPD is of Lower Priority	1304

A.4.5.1.1.1	Test Purpose and Environment	1304

A.4.5.1.1.2	Test Requirements	1306

A.4.5.2	E-UTRAN TDD – HRPD	1306

A.4.5.2.1	E-UTRAN TDD – HRPD Cell Reselection: HRPD is of Lower Priority	1306

A.4.5.2.1.1	Test Purpose and Environment	1306

A.4.5.2.1.2	Test Requirements	1309

A.4.6	E-UTRAN to cdma2000 1X Cell Re-Selection	1309

A.4.6.1	E-UTRAN FDD – cdma2000 1X	1309

A.4.6.1.1	E-UTRAN FDD – cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1309

A.4.6.1.1.1	Test Purpose and Environment	1309

A.4.6.1.1.2	Test Requirements	1312

A.4.6.2	E-UTRAN TDD – cdma2000 1X	1312

A.4.6.2.1	E-UTRAN TDD –cdma2000 1X Cell Reselection: cdma2000 1X is of Lower Priority	1312

A.4.6.2.1.1	Test Purpose and Environment	1312

A.4.6.2.1.2	Test Requirements	1315

A.4.7	Idle State Positioning Measurement for UE category NB1	1315

A.4.7.1	HD – FDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1315

A.4.7.1.1	Test Purpose and Environment	1315

A.4.7.1.2	Test Requirements	1320

A.4.7.2	HD – FDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1321

A.4.7.2.1	Test Purpose and Environment	1321

A.4.7.4.2	Test Requirements	1325

A.4.7.3	TDD Intra frequency case for UE Category NB1 standalone mode in enhanced coverage	1326

A.4.7.3.1	Test Purpose and Environment	1326

A.4.7.3.2	Test Requirements	1330

A.4.7.4	TDD Inter frequency case for UE Category NB1 standalone mode in enhanced coverage	1331

A.4.7.4.1	Test Purpose and Environment	1331

A.4.7.4.2	Test Requirements	1335

A.5	E-UTRAN RRC CONNECTED Mode Mobility	1336

A.5.1	E-UTRAN Handover	1336

A.5.1.1	E-UTRAN FDD - FDD Intra frequency handover	1336

A.5.1.1.1	Test Purpose and Environment	1336

A.5.1.1.2	Test Requirements	1338

A.5.1.2	E-UTRAN TDD - TDD Intra frequency handover	1338

A.5.1.2.1	Test Purpose and Environment	1338

A.5.1.2.2	Test Requirements	1340

A.5.1.3	E-UTRAN FDD – FDD Inter frequency handover	1340

A.5.1.3.1	Test Purpose and Environment	1340

A.5.1.3.2	Test Requirements	1342

A.5.1.4	E-UTRAN TDD – TDD Inter frequency handover	1342

A.5.1.4.1	Test Purpose and Environment	1342

A.5.1.4.2	Test Requirements	1344

A.5.1.5	E-UTRAN FDD – FDD Inter frequency handover: unknown target cell	1344

A.5.1.5.1	Test Purpose and Environment	1344

A.5.1.5.2	Test Requirements	1346

A.5.1.6 	E-UTRAN TDD – TDD Inter frequency handover; unknown Target Cell	1346

A.5.1.6.1	Test Purpose and Environment	1346

A.5.1.6.2	Test Requirements	1348

A.5.1.7	E-UTRAN FDD – TDD Inter frequency handover	1348

A.5.1.7.1	Test Purpose and Environment	1348

A.5.1.7.2	Test Requirements	1351

A.5.1.8	E-UTRAN TDD – FDD Inter frequency handover	1351

A.5.1.8.1	Test Purpose and Environment	1351

A.5.1.8.2	Test Requirements	1354

A.5.1.9	E-UTRAN FDD - FDD Intra frequency handover for 5MHz bandwidth	1354

A.5.1.9.1	Test Purpose and Environment	1354

A.5.1.9.2	Test Requirements	1355

A.5.1.10	E-UTRAN FDD - FDD Intra frequency handover for UE category 0	1355

A.5.1.10.1	Test Purpose and Environment	1355

A.5.1.10.2	Test Requirements	1357

A.5.1.11	E-UTRAN HD - FDD Intra frequency handover for UE category 0	1357

A.5.1.11.1	Test Purpose and Environment	1357

A.5.1.11.2	Test Requirements	1359

A.5.1.12	E-UTRAN TDD - TDD Intra frequency handover for UE category 0	1359

A.5.1.12.1	Test Purpose and Environment	1359

A.5.1.12.2	Test Requirements	1361

A.5.1.13	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1361

A.5.1.13.1	Test Purpose and Environment	1361

A.5.1.13.2	Test Requirements	1363

A.5.1.14	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	1364

A.5.1.14.1	Test Purpose and Environment	1364

A.5.1.14.2	Test Requirements	1365

A.5.1.15	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA	1366

A.5.1.15.1	Test Purpose and Environment	1366

A.5.1.15.2	Test Requirements	1367

A.5.1.16	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1368

A.5.1.16.1	Test Purpose and Environment	1368

A.5.1.16.2	Test Requirements	1369

A.5.1.17	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB	1370

A.5.1.17.1	Test Purpose and Environment	1370

A.5.1.17.2	Test Requirements	1371

A.5.1.18	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB	1372

A.5.1.18.1	Test Purpose and Environment	1372

A.5.1.18.2	Test Requirements	1373

A.5.1.19	E-UTRAN FDD - FDD Intra frequency handover for UE Category 1bis	1374

A.5.1.19.1	Test Purpose and Environment	1374

A.5.1.19.2	Test Requirements	1375

A.5.1.20	E-UTRAN TDD - TDD Intra frequency handover for UE Category 1bis	1376

A.5.1.20.1	Test Purpose and Environment	1376

A.5.1.20.2	Test Requirements	1377

A.5.1.21	E-UTRAN FDD - FDD Intra frequency RACH-less handover	1378

A.5.1.21.1	Test Purpose and Environment	1378

A.5.1.21.2	Test Requirements	1379

A.5.1.22	E-UTRAN TDD - TDD Intra frequency RACH-less handover	1379

A.5.1.22.1	Test Purpose and Environment	1379

A.5.1.22.2	Test Requirements	1381

A.5.1.23	E-UTRAN FDD – FDD Inter frequency RACH-less handover	1381

A.5.1.23.1	Test Purpose and Environment	1381

A.5.1.23.2	Test Requirements	1383

A.5.1.24	E-UTRAN TDD – TDD Inter frequency RACH-less handover	1383

A.5.1.24.1	Test Purpose and Environment	1383

A.5.1.24.2	Test Requirements	1385

A.5.1.25	E-UTRAN FDD - FDD Intra frequency make-before-break handover	1385

A.5.1.25.1	Test Purpose and Environment	1385

A.5.1.25.2	Test Requirements	1387

A.5.1.26	E-UTRAN TDD - TDD Intra frequency make-before-break handover	1388

A.5.1.26.1	Test Purpose and Environment	1388

A.5.1.26.2	Test Requirements	1389

A.5.1.27	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeA	1390

A.5.1.27.1	Test Purpose and Environment	1390

A.5.1.27.2	Test Requirements	1391

A.5.1.28	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeA	1392

A.5.1.28.1	Test Purpose and Environment	1392

A.5.1.28.2	Test Requirements	1393

A.5.1.29	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeA	1394

A.5.1.29.1	Test Purpose and Environment	1394

A.5.1.29.2	Test Requirements	1395

A.5.1.30	E-UTRAN FDD inter frequency handover for Cat-M1 UEs in CEModeB	1396

A.5.1.30.1	Test Purpose and Environment	1396

A.5.1.30.2	Test Requirements	1397

A.5.1.31	E-UTRAN HD-FDD inter frequency handover for Cat-M1 UEs in CEModeB	1398

A.5.1.31.1	Test Purpose and Environment	1398

A.5.1.31.2	Test Requirements	1399

A.5.1.32	E-UTRAN TDD inter frequency handover for Cat-M1 UEs in CEModeB	1400

A.5.1.32.1	Test Purpose and Environment	1400

A.5.1.32.2	Test Requirements	1401

A.5.1.33	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1402

A.5.1.33.1	Test Purpose and Environment	1402

A.5.1.13.2	Test Requirements	1403

A.5.1.34	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1404

A.5.1.34.1	Test Purpose and Environment	1404

A.5.1.34.2	Test Requirements	1405

A.5.1.35	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	1406

A.5.1.35.1	Test Purpose and Environment	1406

A.5.1.35.2	Test Requirements	1407

A.5.1.36	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1408

A.5.1.36.1	Test Purpose and Environment	1408

A.5.1.36.2	Test Requirements	1409

A.5.1.37	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1410

A.5.1.37.1	Test Purpose and Environment	1410

A.5.1.37.2	Test Requirements	1411

A.5.1.38	E-UTRAN TDD Intra frequency handover for Cat-M1 UEs in CEModeB without SFN acquisition	1412

A.5.1.38.1	Test Purpose and Environment	1412

A.5.1.38.2	Test Requirements	1413

A.5.1.39	E-UTRAN FDD - FDD Intra frequency handover with direct SCell activation	1414

A.5.1.39.1	Test Purpose and Environment	1414

A.5.1.39.2	Test Requirements	1415

A.5.1.40	E-UTRAN TDD - TDD Intra frequency handover with direct SCell activation	1416

A.5.1.40.1	Test Purpose and Environment	1416

A.5.1.40.2	Test Requirements	1417

A.5.1.41	E-UTRAN FDD – FDD Intra-band Inter-frequency sync DAPS handover	1418

A.5.1.41.1	Test Purpose and Environment	1418

A.5.1.41.2	Test Requirements	1420

A.5.1.42	E-UTRAN FDD – FDD Intra-band Inter-frequency async DAPS handover	1421

A.5.1.42.1	Test Purpose and Environment	1421

A.5.1.42.2	Test Requirements	1422

A.5.1.43	E-UTRAN FDD – FDD Inter-band Inter-frequency sync DAPS handover	1423

A.5.1.43.1	Test Purpose and Environment	1423

A.5.1.43.2	Test Requirements	1424

A.5.1.44	E-UTRAN FDD – FDD Inter-band Inter-frequency async DAPS handover	1425

A.5.1.44.1	Test Purpose and Environment	1425

A.5.1.44.2	Test Requirements	1426

A.5.1.45	E-UTRAN FDD - FDD Intra frequency DAPS handover	1427

A.5.1.45.1	Test Purpose and Environment	1427

A.5.1.45.1	Test Requirements	1428

A.5.1.46	E-UTRAN TDD - TDD Intra frequency DAPS handover	1429

A.5.1.46.1	Test Purpose and Environment	1429

A.5.1.46.2	Test Requirements	1430

A.5.1.47	E-UTRAN FDD - FDD Intra frequency conditional handover	1431

A.5.1.47.1	Test Purpose and Environment	1431

A.5.1.47.2	Test Requirements	1432

A.5.1.48	E-UTRAN TDD - TDD Intra frequency conditional handover	1433

A.5.1.48.1	Test Purpose and Environment	1433

A.5.1.48.2	Test Requirements	1434

A.5.1.49	E-UTRAN FDD - FDD Inter frequency conditional handover	1435

A.5.1.49.1	Test Purpose and Environment	1435

A.5.1.49.2	Test Requirements	1436

A.5.1.50	E-UTRAN TDD - TDD Inter frequency conditional handover	1437

A.5.1.50.1	Test Purpose and Environment	1437

A.5.1.50.2	Test Requirements	1438

A.5.1.51	E-UTRAN FDD - TDD Inter frequency conditional handover	1439

A.5.1.51.1	Test Purpose and Environment	1439

A.5.1.51.2	Test Requirements	1441

A.5.1.52	E-UTRAN TDD - FDD Inter frequency conditional handover	1441

A.5.152.1	Test Purpose and Environment	1441

A.5.1.52.2	Test Requirements	1444

A.5.1.53	E-UTRAN TDD – TDD Intra-band Inter-frequency sync DAPS handover	1445

A.5.1.53.1	Test Purpose and Environment	1445

A.5.1.53.2	Test Requirements	1447

A.5.1.54	E-UTRAN TDD – TDD Inter-band Inter-frequency sync DAPS handover	1448

A.5.1.54.1	Test Purpose and Environment	1448

A.5.1.54.2	Test Requirements	1450

A.5.1.55	E-UTRAN FDD - TDD inter-band inter-frequency synchronous DAPS handover	1451

A.5.1.55.1	Test Purpose and Environment	1451

A.5.1.56	E-UTRAN TDD - FDD inter-band inter-frequency synchronous DAPS handover	1455

A.5.1.56.1	Test Purpose and Environment	1455

A.5.1.56.2	Test Requirements	1458

A.5.1.57	E-UTRAN FDD – TDD Inter-band Inter-frequency async DAPS handover	1459

A.5.1.57.1	Test Purpose and Environment	1459

A.5.1.57.2	Test Requirements	1461

A.5.1.58	E-UTRAN TDD – FDD Inter-band Inter-frequency async DAPS handover	1461

A.5.1.58.1	Test Purpose and Environment	1461

A.5.1.58.2	Test Requirements	1464

A.5.2	E-UTRAN Handover to other RATs	1464

A.5.2.1	E-UTRAN FDD – UTRAN FDD Handover	1464

A.5.2.1.1	Test Purpose and Environment	1464

A.5.2.1.2	Test Requirements	1466

A.5.2.2	E-UTRAN TDD - UTRAN FDD Handover	1467

A.5.2.2.1	Test Purpose and Environment	1467

A.5.2.2.2	Test Requirements	1470

A.5.2.3 E-UTRAN FDD- GSM Handover	1470

A.5.2.3.1	Test Purpose and Environment	1470

A.5.2.3.2	Test Requirements	1471

A.5.2.4	E-UTRAN TDD - UTRAN TDD Handover	1472

A.5.2.4.1	Test Purpose and Environment	1472

A.5.2.4.1.1	Void	1472

A.5.2.4.1.2	1.28 Mcps TDD option	1472

A.5.2.4.1.3	Void	1474

A.5.2.4.2	Test Requirements	1474

A.5.2.4.2.1	Void	1474

A.5.2.4.2.2	1.28 Mcps TDD option	1474

A.5.2.4.2.3	Void	1474

A.5.2.5	E-UTRAN FDD – UTRAN TDD Handover	1474

A.5.2.5.1	Test Purpose and Environment	1474

A.5.2.5.1.3	Void	1477

A.5.2.5.2	Test Requirements	1477

A.5.2.5.2.1	Void	1477

A.5.2.5.2.2	1.28 Mcps TDD option	1477

A.5.2.5.2.3	Void	1477

A.5.2.6	E-UTRAN TDD - GSM Handover	1477

A.5.2.6.1	Test Purpose and Environment	1477

A.5.2.6.2	Test Requirements	1479

A.5.2.7	E-UTRAN FDD – UTRAN FDD Handover; Unknown Target Cell	1480

A.5.2.7.1	Test Purpose and Environment	1480

A.5.2.7.2	Test Requirements	1482

A.5.2.8	E-UTRAN FDD - GSM Handover; Unknown Target Cell	1482

A.5.2.8.1	Test Purpose and Environment	1482

A.5.2.8.2	Test Requirements	1483

A.5.2.9	E-UTRAN TDD - GSM Handover; Unknown Target Cell	1484

A.5.2.9.1	Test Purpose and Environment	1484

A.5.2.9.2	Test Requirements	1485

A.5.2.10	E-UTRAN TDD to UTRAN TDD handover: unknown target cell	1486

A.5.2.10.1	Test Purpose and Environment	1486

A.5.2.10.2	Test Requirements	1488

A.5.2.10A	E-UTRAN FDD – UTRAN FDD Multicarrier Handover with two target cells	1488

A.5.2.10A.1	Test Purpose and Environment	1488

A.5.2.10A.2	Test Requirements	1491

A.5.2.10B	E-UTRAN TDD – UTRAN FDD Multicarrier Handover with two target cells	1491

A.5.2.10B.1	Test Purpose and Environment	1491

A.5.2.10B.2	Test Requirements	1494

A.5.2.11	E-UTRAN FDD – UTRAN FDD Handover for 5MHz Bandwidth	1494

A.5.2.11.1	Test Purpose and Environment	1494

A.5.2.11.2	Test Requirements	1494

A.5.3	E-UTRAN Handover to Non-3GPP RATs	1495

A.5.3.1	E-UTRAN FDD – HRPD Handover	1495

A.5.3.1.1	Test Purpose and Environment	1495

A.5.3.1.2	Test Requirements	1497

A.5.3.2	E-UTRAN FDD – cdma2000 1X Handover	1497

A.5.3.2.1	Test Purpose and Environment	1497

A.5.3.2.2	Test Requirements	1500

A.5.3.3	E-UTRAN FDD – HRPD Handover; Unknown Target Cell	1500

A.5.3.3.1	Test Purpose and Environment	1500

A.5.3.3.2	Test Requirements	1503

A.5.3.4	E-UTRAN FDD – cdma2000 1X Handover; Unknown Target cell	1503

A.5.3.4.1	Test Purpose and Environment	1503

A.5.3.4.2	Test Requirements	1504

A.5.3.5	E-UTRAN TDD – HRPD Handover	1505

A.5.3.5.1	Test Purpose and Environment	1505

A.5.3.5.2	Test Requirements	1508

A.5.3.6	E-UTRAN TDD – cdma2000 1X Handover	1508

A.5.3.6.1	Test Purpose and Environment	1508

A.5.3.6.2	Test Requirements	1511

A.6	RRC Connection Control	1511

A.6.1	RRC Re-establishment	1511

A.6.1.1	E-UTRAN FDD Intra-frequency RRC Re-establishment	1511

A.6.1.1.1	Test Purpose and Environment	1511

A.6.1.1.2	Test Requirements	1512

A.6.1.2	E-UTRAN FDD Inter-frequency RRC Re-establishment	1513

A.6.1.2.1	Test Purpose and Environment	1513

A.6.1.2.2	Test Requirements	1514

A.6.1.3	E-UTRAN TDD Intra-frequency RRC Re-establishment	1515

A.6.1.3.1	Test Purpose and Environment	1515

A.6.1.3.2	Test Requirements	1516

A.6.1.4	E-UTRAN TDD Inter-frequency RRC Re-establishment	1517

A.6.1.4.1	Test Purpose and Environment	1517

A.6.1.4.2	Test Requirements	1518

A.6.1.5	E-UTRAN FDD Intra-frequency RRC Re-establishment for 5MHz bandwidth	1519

A.6.1.5.1	Test Purpose and Environment	1519

A.6.1.5.2	Test Requirements	1519

A.6.1.6	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for UE category 0	1519

A.6.1.6.1	Test Purpose and Environment	1519

A.6.1.6.2	Test Requirements	1521

A.6.1.7	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for UE category 0	1522

A.6.1.7.1	Test Purpose and Environment	1522

A.6.1.7.2	Test Requirements	1523

A.6.1.8	E-UTRAN TDD Intra-frequency RRC Re-establishment for UE category 0	1524

A.6.1.8.1	Test Purpose and Environment	1524

A.6.1.8.2	Test Requirements	1525

A.6.1.9	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1526

A.6.1.9.1	Test Purpose and Environment	1526

A.6.1.9.2	Test Requirements	1527

A.6.1.10	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1528

A.6.1.10.1	Test Purpose and Environment	1528

A.6.1.10.2	Test Requirements	1529

A.6.1.11	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1530

A.6.1.11.1	Test Purpose and Environment	1530

A.6.1.11.2	Test Requirements	1531

A.6.1.12	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1532

A.6.1.12.1	Test Purpose and Environment	1532

A.6.1.12.2	Test Requirements	1533

A.6.1.13	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1534

A.6.1.13.1	Test Purpose and Environment	1534

A.6.1.13.2	Test Requirements	1535

A.6.1.14	E-UTRAN TDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1536

A.6.1.14.1	Test Purpose and Environment	1536

A.6.1.14.2	Test Requirements	1537

A.6.1.15	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1538

A.6.1.15.1	Test Purpose and Environment	1538

A.6.1.15.2	Test Requirements	1540

A.6.1.16	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1541

A.6.1.16.1	Test Purpose and Environment	1541

A.6.1.16.2	Test Requirements	1543

A.6.1.17	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1544

A.6.1.17.1	Test Purpose and Environment	1544

A.6.1.17.2	Test Requirements	1545

A.6.1.18	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1546

A.6.1.18.1	Test Purpose and Environment	1546

A.6.1.18.2	Test Requirements	1547

A.6.1.19	E-UTRAN TDD-TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	1548

A.6.1.19.1	Test Purpose and Environment	1548

A.6.1.19.2	Test Requirements	1549

A.6.1.20	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1550

A.6.1.20.1	Test Purpose and Environment	1550

A.6.1.20.2	Test Requirements	1551

A.6.1.21	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1552

A.6.1.21.1	Test Purpose and Environment	1552

A.6.1.21.2	Test Requirements	1553

A.6.1.22	E-UTRAN TDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeB	1554

A.6.1.22.1	Test Purpose and Environment	1554

A.6.1.22.2	Test Requirements	1555

A.6.1.23	E-UTRAN TDD Inter-frequency RRC Re-establishment for UE category NB1 in In-Band mode under normal coverage	1556

A.6.1.23.1	Test Purpose and Environment	1556

A.6.1.23.2	Test Requirements	1558

A.6.1.24	E-UTRAN TDD - TDD Intra-frequency RRC Re-establishment for UE category NB1 in In-Band mode under enhanced coverage	1559

A.6.1.24.1	Test Purpose and Environment	1559

A.6.1.24.2	Test Requirements	1561

A.6.2	Random Access	1562

A.6.2.1	E-UTRAN FDD – Contention Based Random Access Test	1562

A.6.2.1.1	Test Purpose and Environment	1562

A.6.2.1.2	Test Requirements	1563

A.6.2.1.2.1	Random Access Response Reception	1563

A.6.2.1.2.2	No Random Access Response Reception	1563

A.6.2.1.2.3	Receiving a NACK on msg3	1563

A.6.2.1.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1564

A.6.2.1.2.5	Reception of a Correct Message over Temporary C-RNTI	1564

A.6.2.1.2.6	Contention Resolution Timer expiry	1564

A.6.2.2	E-UTRAN FDD – Non-Contention Based Random Access Test	1564

A.6.2.2.1	Test Purpose and Environment	1564

A.6.2.2.2	Test Requirements	1565

A.6.2.2.2.1	Random Access Response Reception	1566

A.6.2.2.2.2	No Random Access Response Reception	1566

A.6.2.3	E-UTRAN TDD – Contention Based Random Access Test	1566

A.6.2.3.1	Test Purpose and Environment	1566

A.6.2.3.2	Test Requirements	1568

A.6.2.3.2.1	Random Access Response Reception	1568

A.6.2.3.2.2	No Random Access Response reception	1568

A.6.2.3.2.3	Receiving a NACK on msg3	1568

A.6.2.3.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1569

A.6.2.3.2.5	Reception of a Correct Message over Temporary C-RNTI	1569

A.6.2.3.2.6	Contention Resolution Timer expiry	1569

A.6.2.4	E-UTRAN TDD – Non-Contention Based Random Access Test	1569

A.6.2.4.1	Test Purpose and Environment	1569

A.6.2.4.2	Test Requirements	1571

A.6.2.4.2.1	Random Access Response Reception	1571

A.6.2.4.2.2	No Random Access Response Reception	1571

A.6.2.5	E-UTRAN FDD – Contention Based Random Access Test for 5MHz bandwidth	1571

A.6.2.5.1	Test Purpose and Environment	1571

A.6.2.5.2	Test Requirements	1572

A.6.2.6	E-UTRAN FDD – Non-contention Based Random Access Test for 5MHz bandwidth	1572

A.6.2.6.1	Test Purpose and Environment	1572

A.6.2.6.2	Test Requirements	1572

A.6.2.7	E-UTRAN FDD – Non-Contention Based Random Access Test For SCell	1573

A.6.2.7.1	Test Purpose and Environment	1573

A.6.2.7.2	Test Requirements	1575

A.6.2.7.2.1	Random Access Response Reception	1575

A.6.2.7.2.2	No Random Access Response Reception	1575

A.6.2.7.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1575

A.6.2.8	E-UTRAN TDD – Non-Contention Based Random Access Test For SCell	1576

A.6.2.8.1	Test Purpose and Environment	1576

A.6.2.8.2	Test Requirements	1578

A.6.2.8.2.1	Random Access Response Reception	1578

A.6.2.8.2.2	No Random Access Response Reception	1578

A.6.2.8.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1578

A.6.2.9	3DL/3UL TDD CA Non-Contention Based Random Access Test for 2 SCells	1579

A.6.2.9.1	Test Purpose and Environment	1579

A.6.2.9.2	Test Requirements	1582

A.6.2.9.2.1	Random Access Response Reception	1582

A.6.2.9.2.2	No Random Access Response Reception	1583

A.6.2.9.2.3	Stop Preamble transmission if maximum number of preamble transmission counter has been reached	1583

A.6.2.10	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1584

A.6.2.10.1	Test Purpose and Environment	1584

A.6.2.10.2	Test Requirements	1586

A.6.2.10.2.1	Random Access Response Reception	1586

A.6.2.10.2.2	No Random Access Response Reception	1587

A.6.2.10.2.3	Receiving a NACK on msg3	1587

A.6.2.10.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1587

A.6.2.10.2.5	Reception of a Correct Message over Temporary C-RNTI	1587

A.6.2.10.2.6	Contention Resolution Timer expiry	1587

A.6.2.10.2.7	PRACH Resource Selection	1588

A.6.2.11	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1588

A.6.2.11.1	Test Purpose and Environment	1588

A.6.2.11.2	Test Requirements	1590

A.6.2.11.2.1	Random Access Response Reception	1590

A.6.2.11.2.2	No Random Access Response Reception	1591

A.6.2.11.2.3	Receiving a NACK on msg3	1591

A.6.2.11.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1591

A.6.2.11.2.5	Reception of a Correct Message over Temporary C-RNTI	1591

A.6.2.11.2.6	Contention Resolution Timer expiry	1591

A.6.2.11.2.7	PRACH Resource Selection	1592

A.6.2.12	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage	1592

A.6.2.12.1	Test Purpose and Environment	1592

A.6.2.12.2	Test Requirements	1594

A.6.2.12.2.1	Random Access Response Reception	1594

A.6.2.12.2.2	No Random Access Response Reception	1595

A.6.2.12.2.3	Receiving a NACK on msg3	1595

A.6.2.12.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1595

A.6.2.12.2.5	Reception of a Correct Message over Temporary C-RNTI	1595

A.6.2.12.2.6	Contention Resolution Timer expiry	1595

A.6.2.12.2.7	PRACH Resource Selection	1596

A.6.2.13	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1596

A.6.2.13.1	Test Purpose and Environment	1596

A.6.2.13.2	Test Requirements	1598

A.6.2.13.2.1	Random Access Response Reception	1598

A.6.2.13.2.2	No Random Access Response Reception	1599

A.6.2.13.2.3	Receiving a NACK on msg3	1599

A.6.2.13.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1599

A.6.2.13.2.5	Reception of a Correct Message over Temporary C-RNTI	1599

A.6.2.13.2.6	Contention Resolution Timer expiry	1599

A.6.2.13.2.7	PRACH Resource Selection	1600

A.6.2.14	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1600

A.6.2.14.1	Test Purpose and Environment	1600

A.6.2.14.2	Test Requirements	1602

A.6.2.14.2.1	Random Access Response Reception	1602

A.6.2.14.2.2	No Random Access Response Reception	1603

A.6.2.14.2.3	Receiving a NACK on msg3	1603

A.6.2.14.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1603

A.6.2.14.2.5	Reception of a Correct Message over Temporary C-RNTI	1603

A.6.2.14.2.6	Contention Resolution Timer expiry	1603

A.6.2.14.2.7	PRACH Resource Selection	1604

A.6.2.15	E-UTRAN TDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage	1604

A.6.2.15.1	Test Purpose and Environment	1604

A.6.2.15.2	Test Requirements	1606

A.6.2.15.2.1	Random Access Response Reception	1606

A.6.2.15.2.2	No Random Access Response Reception	1607

A.6.2.15.2.3	Receiving a NACK on msg3	1607

A.6.2.15.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1607

A.6.2.15.2.5	Reception of a Correct Message over Temporary C-RNTI	1607

A.6.2.15.2.6	Contention Resolution Timer expiry	1607

A.6.2.15.2.7	PRACH Resource Selection	1608

A.6.2.16	Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1608

A.6.2.16.1	Test Purpose and Environment	1608

A.6.2.16.2	Test Requirements	1611

A.6.2.16.2.1	Random Access Response Reception	1611

A.6.2.16.2.2	No Random Access Response Reception	1612

A.6.2.16.2.3	Receiving a NACK on msg3	1612

A.6.2.16.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1612

A.6.2.16.2.5	Reception of a Correct Message over Temporary C-RNTI	1612

A.6.2.16.2.6	Contention Resolution Timer expiry	1612

A.6.2.16.2.7	NPRACH Resource Selection	1612

A.6.2.17	Contention Based Random Access Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1613

A.6.2.17.1	Test Purpose and Environment	1613

A.6.2.17.2	Test Requirements	1616

A.6.2.17.2.1	Random Access Response Reception	1616

A.6.2.17.2.2	No Random Access Response Reception	1617

A.6.2.17.2.3	Receiving a NACK on msg3	1617

A.6.2.17.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1617

A.6.2.17.2.5	Reception of a Correct Message over Temporary C-RNTI	1617

A.6.2.17.2.6	Contention Resolution Timer expiry	1617

A.6.2.17.2.7	NPRACH Resource Selection	1617

A.6.2.18	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1618

A.6.2.18.1	Test Purpose and Environment	1618

A.6.2.18.2	Test Requirements	1621

A.6.2.18.2.1	Random Access Response Reception	1621

A.6.2.18.2.2	No Random Access Response Reception	1622

A.6.2.18.2.3	Receiving a NACK on msg3	1622

A.6.2.18.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1622

A.6.2.18.2.5	Reception of a Correct Message over Temporary C-RNTI	1622

A.6.2.18.2.6	Contention Resolution Timer expiry	1622

A.6.2.18.2.7	NPRACH Resource Selection	1622

A.6.2.19	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in normal coverage	1623

A.6.2.19.1	Test Purpose and Environment	1623

A.6.2.19.2	Test Requirements	1625

A.6.2.19.2.1	Random Access Response Reception	1625

A.6.2.19.2.2	No Random Access Response Reception	1626

A.6.2.19.2.3	Receiving a NACK on msg3	1626

A.6.2.19.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1626

A.6.2.19.2.5	Reception of a Correct Message over Temporary C-RNTI	1626

A.6.2.19.2.6	Contention Resolution Timer expiry	1626

A.6.2.19.2.7	NPRACH Resource Selection	1626

A.6.2.20	TDD Contention Based Random Access Test for UE category NB1 UEs In-band mode in enhanced coverage	1627

A.6.2.20.1	Test Purpose and Environment	1627

A.6.2.20.2	Test Requirements	1629

A.6.2.20.2.1	Random Access Response Reception	1629

A.6.2.20.2.2	No Random Access Response Reception	1630

A.6.2.20.2.3	Receiving a NACK on msg3	1630

A.6.2.20.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1630

A.6.2.20.2.5	Reception of a Correct Message over Temporary C-RNTI	1630

A.6.2.20.2.6	Contention Resolution Timer expiry	1630

A.6.2.20.2.7	NPRACH Resource Selection	1630

A.6.2.21	TDD Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs In-band mode in Enhanced Coverage	1631

A.6.2.21.1	Test Purpose and Environment	1631

A.6.2.21.2	Test Requirements	1633

A.6.2.21.2.1	Random Access Response Reception	1633

A.6.2.21.2.2	No Random Access Response Reception	1634

A.6.2.21.2.3	Receiving a NACK on msg3	1634

A.6.2.21.2.4	Reception of an Incorrect Message over Temporary C-RNTI	1634

A.6.2.21.2.5	Reception of a Correct Message over Temporary C-RNTI	1634

A.6.2.21.2.6	Contention Resolution Timer expiry	1634

A.6.2.21.2.7	NPRACH Resource Selection	1634

A.6.3	RRC Connection Release with Redirection	1635

A.6.3.1	Redirection from E-UTRAN FDD to UTRAN FDD	1635

A.6.3.1.1	Test Purpose and Environment	1635

A.6.3.1.2	Test Requirements	1637

A.6.3.2	Redirection from E-UTRAN TDD to UTRAN FDD	1637

A.6.3.2.1	Test Purpose and Environment	1637

A.6.3.2.2	Test Requirements	1639

A.6.3.3	Redirection from E-UTRAN FDD to GERAN when System Information is provided	1639

A.6.3.3.1	Test Purpose and Environment	1639

A.6.3.3.2	Test Requirements	1640

A.6.3.4	Redirection from E-UTRAN TDD to GERAN when System Information is provided	1641

A.6.3.4.1	Test Purpose and Environment	1641

A.6.3.4.2	Test Requirements	1642

A.6.3.5	E-UTRA TDD RRC connection release redirection to UTRA TDD	1643

A.6.3.5.1	Test Purpose and Environment	1643

A.6.3.5.2	Test Requirements	1645

A.6.3.6	E-UTRA FDD RRC connection release redirection to UTRA TDD	1645

A.6.3.6.1	Test Purpose and Environment	1645

A. 6.3.6.2	Test Requirements	1648

A.6.3.7	E-UTRA TDD RRC connection release redirection to UTRA TDD without SI provided	1648

A.6.3.7.1	Test Purpose and Environment	1648

A.6.3.7.2	Test Requirements	1651

A.6.3.8	E-UTRA FDD RRC connection release redirection to UTRA TDD without SI provided	1651

A.6.3.8.1	Test Purpose and Environment	1651

A.6.3.8.2	Test Requirements	1654

A.6.3.9	Redirection from E-UTRAN FDD to UTRAN FDD without System Information	1654

A.6.3.9.1	Test Purpose and Environment	1654

A.6.3.9.2	Test Requirements	1656

A.6.3.10	Redirection from E-UTRAN FDD to GERAN when System Information is not provided	1656

A.6.3.10.1	Test Purpose and Environment	1656

A.6.3.10.2	Test Requirements	1658

A.6.3.11	Redirection from E-UTRAN TDD to GERAN when System Information is not provided	1658

A.6.3.11.1	Test Purpose and Environment	1658

A.6.3.11.2	Test Requirements	1660

A.6.3.12	E-UTRAN TDD RRC connection release redirection to UTRAN FDD without SI provided	1660

A.6.3.12.1	Test Purpose and Environment	1660

A.6.3.12.2	Test Requirements	1663

A.6.3.13	Redirection from E-UTRA to NR FR1 for redcap UE	1663

A.6.3.13.1	Test Purpose and Environment	1663

A.6.3.13.2	Test Parameters	1663

A.6.3.13.3	Test Requirements	1667

A.7	Timing and Signalling Characteristics	1668

A.7.1	UE Transmit Timing	1668

A.7.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests	1668

A.7.1.1.1	Test Purpose and Environment	1668

A.7.1.1.2	Test Requirements	1669

A.7.1.2	E-UTRAN TDD - UE Transmit Timing Accuracy Tests	1670

A.7.1.2.1	Test Purpose and Environment	1670

A.7.1.2.2	Test Requirements	1672

A.7.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell	1673

A.7.1.3.1	Test Purpose and Environment	1673

A.7.1.3.2	Test Requirements	1675

A.7.1.4	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell	1676

A.7.1.4.1	Test Purpose and Environment	1676

A.7.1.4.2	Test Requirements	1677

A.7.1.4A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell for 20 MHz + 10 MHz	1678

A.7.1.4A.1	Test Purpose and Environment	1678

A.7.1.4A.2	Test Requirements	1678

A.7.1.5	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for 5MHz Bandwidth	1678

A.7.1.5.1	Test Purpose and Environment	1678

A.7.1.5.2	Test Requirements	1679

A.7.1.6	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for SCell in sTAG	1679

A.7.1.6.1	Test Purpose and Environment	1679

A.7.1.6.2	Test Requirements	1681

A.7.1.7	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG	1682

A.7.1.7.1	Test Purpose and Environment	1682

A.7.1.7.2	Test Requirements	1683

A.7.1.7A	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +20MHz	1684

A.7.1.7A.1	Test Purpose and Environment	1684

A.7.1.7A.2	Test Requirements	1684

A.7.1.7B	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for SCell in sTAG for 20MHz +10MHz	1684

A.7.1.7B.1	Test Purpose and Environment	1684

A.7.1.7B.2	Test Requirements	1684

A.7.1.8	Void	1685

A.7.1.8.1	Void	1685

A.7.1.8.2	Void	1685

A.7.1.9	Void	1685

A.7.1.9.1	Void	1685

A.7.1.9.2	Void	1685

A.7.1.10	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1685

A.7.1.10.1	Test Purpose and Environment	1685

A.7.1.10.2	Test Requirements	1687

A.7.1.11	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1687

A.7.1.11.1	Test Purpose and Environment	1687

A.7.1.11.2	Test Requirements	1689

A.7.1.12	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	1689

A.7.1.12.1	Test Purpose and Environment	1689

A.7.1.12.2	Test Requirements	1691

A.7.1.13	3DL/3UL TDD CA UE Transmit Timing Accuracy Tests for 2 SCells	1691

A.7.1.13.1	Test Purpose and Environment	1691

A.7.1.13.2	Test Requirements	1693

A.7.1.14	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1694

A.7.1.14.1	Test Purpose and Environment	1694

A.7.1.14.2	Test Requirements	1695

A.7.1.15	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1695

A.7.1.15.1	Test Purpose and Environment	1695

A.7.1.15.2	Test Requirements	1696

A.7.1.16	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	1697

A.7.1.16.1	Test Purpose and Environment	1697

A.7.1.16.2	Test Requirements	1698

A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1699

A.7.1.17.1	Test Purpose and Environment	1699

A.7.1.17.2	Test Requirements	1701

A.7.1.18	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1702

A.7.1.18.1	Test Purpose and Environment	1702

A.7.1.18.2	Test Requirements	1704

A.7.1.19	E-UTRAN FDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1705

A.7.1.19.1	Test Purpose and Environment	1705

A.7.1.19.2	Test Requirements	1706

A.7.1.20	E-UTRAN TDD - UE Transmit Timing Accuracy Test for RACH-less Handover	1706

A.7.1.20.1	Test Purpose and Environment	1706

A.7.1.20.2	Test Requirements	1708

A.7.1.21	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1708

A.7.1.21.1	Test Purpose and Environment	1708

A.7.1.21.2	Test Requirements	1710

A.7.1.22	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1710

A.7.1.22.1	Test Purpose and Environment	1710

A.7.1.22.2	Test Requirements	1712

A.7.1.23	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeA	1712

A.7.1.23.1	Test Purpose and Environment	1712

A.7.1.23.2	Test Requirements	1714

A.7.1.24	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1714

A.7.1.24.1	Test Purpose and Environment	1714

A.7.1.24.2	Test Requirements	1715

A.7.1.25	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1716

A.7.1.25.1	Test Purpose and Environment	1716

A.7.1.25.2	Test Requirements	1717

A.7.1.26	E-UTRAN TDD - UE Transmit Timing Accuracy Tests for Cat-M2 UE in CEModeB	1718

A.7.1.26.1	Test Purpose and Environment	1718

A.7.1.26.2	Test Requirements	1719

A.7.1.27	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage	1720

A.7.1.27.1	Test Purpose and Environment	1720

A.7.1.27.2	Test Requirements	1722

A.7.1.28	E-UTRAN TDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-band mode under enhanced coverage	1722

A.7.1.28.1	Test Purpose and Environment	1722

A.7.1.28.2	Test Requirements	1724

A.7.2	UE Timing Advance	1725

A.7.2.1	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test	1725

A.7.2.1.1	Test Purpose and Environment	1725

A.7.2.1.2	Test Requirements	1726

A.7.2.2	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test	1727

A.7.2.2.1	Test Purpose and Environment	1727

A.7.2.2.2	Test Requirements	1729

A.7.2.3	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for 5MHz	1729

A.7.2.3.1	Test Purpose and Environment	1729

A.7.2.3.2	Test Requirements	1729

A.7.2.4	E-UTRAN FDD – UE Timing Advance Adjustment Accuracy Test for SCell in sTAG	1729

A.7.2.4.1	Test Purpose and Environment	1729

A.7.2.4.2	Test Requirements	1732

A.7.2.5	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG	1732

A.7.2.5.1	Test Purpose and Environment	1732

A.7.2.5.2	Test Requirements	1734

A.7.2.5A	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +20 MHz	1734

A.7.2.5A.1	Test Purpose and Environment	1734

A.7.2.5A.2	Test Requirements	1735

A.7.2.5B	E-UTRAN TDD – UE Timing Advance Adjustment Accuracy Test for Scell in sTAG for 20 MHz +10 MHz	1735

A.7.2.5B.1	Test Purpose and Environment	1735

A.7.2.5B.2	Test Requirements	1735

A.7.2.6	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1735

A.7.2.6.1	Test Purpose and Environment	1735

A.7.2.6.2	Test Requirements	1738

A.7.2.7	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1738

A.7.2.7.1	Test Purpose and Environment	1738

A.7.2.7.2	Test Requirements	1740

A.7.2.8	E-UTRAN TDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	1740

A.7.2.8.1	Test Purpose and Environment	1740

A.7.2.8.2	Test Requirements	1742

A.7.2.9.2	Test Requirements	1743

A.7.2.10	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1744

A.7.2.10.1	Test Purpose and Environment	1744

A.7.2.10.2	Test Requirements	1745

A.7.2.11	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1745

A.7.2.11.1	Test Purpose and Environment	1745

A.7.2.11.2	Test Requirements	1747

A.7.2.12	E-UTRAN TDD UE Timing Advance Adjustment Accuracy Test in CEModeB	1747

A.7.2.12.1	Test Purpose and Environment	1747

A.7.2.12.2	Test Requirements	1749

A.7.2.13	E-UTRAN FDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1750

A.7.2.13.1	Test Purpose and Environment	1750

A.7.2.13.2	Test Requirements	1751

A.7.2.14	E-UTRAN TDD – UE Timing Advance Adjustment delay Test for sTTI and ShortProcessingTime=TRUE	1752

A.7.2.14.1	Test Purpose and Environment	1752

A.7.2.14.2	Test Requirements	1754

A.7.2.15	E-UTRAN TDD – TDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhanced Coverage	1754

A.7.2.15.1	Test Purpose and Environment	1754

A.7.2.15.2	Test Requirements	1756

A.7.3	Radio Link Monitoring	1756

A.7.3.1	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync	1756

A.7.3.1.1	Test Purpose and Environment	1756

A.7.3.1.2	Test Requirements	1760

A.7.3.2	E-UTRAN FDD Radio Link Monitoring Test for In-sync	1760

A.7.3.2.1	Test Purpose and Environment	1760

A.7.3.2.2	Test Requirements	1764

A.7.3.3	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync	1764

A.7.3.3.1	Test Purpose and Environment	1764

A.7.3.3.2	Test Requirements	1768

A.7.3.4	E-UTRAN TDD Radio Link Monitoring Test for In-sync	1768

A.7.3.4.1	Test Purpose and Environment	1768

A.7.3.4.2	Test Requirements	1772

A.7.3.5	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync in DRX	1772

A.7.3.5.1	Test Purpose and Environment	1772

A.7.3.5.2	Test Requirements	1775

A.7.3.6	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX	1776

A.7.3.6.1	Test Purpose and Environment	1776

A.7.3.6.2	Test Requirements	1779

A.7.3.7	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX	1779

A.7.3.7.1	Test Purpose and Environment	1779

A.7.3.7.2	Test Requirements	1782

A.7.3.8	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX	1782

A.7.3.8.1	Test Purpose and Environment	1782

A.7.3.8.2	Test Requirements	1785

A.7.3.9 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction and Non-MBSFN ABS	1785

A.7.3.9.2	Test Requirements	1788

A.7.3.10	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	1788

A.7.3.10.1	Test Purpose and Environment	1788

A.7.3.11	E-UTRAN FDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1792

A.7.3.11.1	Test Purpose and Environment	1792

A.7.3.11.2	Test Requirements	1797

A.7.3.12	E-UTRAN TDD Radio Link Monitoring Test for In-sync for Non-MBSFN ABS	1797

A.7.3.12.1	Test Purpose and Environment	1797

A.7.3.12.2	Test Requirements	1802

A.7.3.13 E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1802

A.7.3.13.1	Test Purpose and Environment	1802

A.7.3.13.2	Test Requirements	1805

A.7.3.14 E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1805

A.7.3.14.1	Test Purpose and Environment	1805

A.7.3.14.2	Test Requirements	1808

A.7.3.15	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1808

A.7.3.15.1	Test Purpose and Environment	1808

A.7.3.15.2	Test Requirements	1812

A.7.3.16	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resource Restriction with MBSFN ABS	1812

A.7.3.16.1	Test Purpose and Environment	1812

A.7.3.16.2	Test Requirements	1817

A.7.3.17	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1817

A.7.3.17.1	Test Purpose and Environment	1817

A.7.3.17.2	Test Requirements	1821

A.7.3.18	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	1821

A.7.3.18.1	Test Purpose and Environment	1821

A.7.3.18.2	Test Requirements	1825

A.7.3.19	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1825

A.7.3.19.1	Test Purpose and Environment	1825

A.7.3.19.2	Test Requirements	1830

A.7.3.20	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and Non-MBSFN ABS	1830

A.7.3.20.1	Test Purpose and Environment	1830

A.7.3.20.2	Test Requirements	1835

A.7.3.21	E-UTRAN FDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1835

A.7.3.21.1	Test Purpose and Environment	1835

A.7.3.21.2	Test Requirements	1840

A.7.3.22	E-UTRAN TDD Radio Link Monitoring Test for In-sync under Time Domain Measurement Resouce Restriction with CRS assistance information and MBSFN ABS	1840

A.7.3.22.1	Test Purpose and Environment	1840

A.7.3.22.2	Test Requirements	1845

A.7.3.23	E-UTRAN FDD Radio Link Monitoring Test for Out-of-sync for 5MHz Bandwidth	1845

A.7.3.23.1	Test Purpose and Environment	1845

A.7.3.23.2	Test Requirements	1846

A.7.3.24	E-UTRAN FDD Radio Link Monitoring Test for In-sync for 5MHz Bandwidth	1846

A.7.3.24.1	Test Purpose and Environment	1846

A.7.3.24.2	Test Requirements	1847

A.7.3.25	E-UTRAN FDD Radio Link Monitoring Test for In-sync in DRX for 5MHz Bandwidth	1847

A.7.3.25.1	Test Purpose and Environment	1847

A.7.3.25.2	Test Requirements	1848

A.7.3.26	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1848

A.7.3.26.1	Test Purpose and Environment	1848

A.7.3.26.2	Test Requirements	1851

A.7.3.27	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1851

A.7.3.27.1	Test Purpose and Environment	1851

A.7.3.27.2	Test Requirements	1854

A.7.3.28	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1854

A.7.3.28.1	Test Purpose and Environment	1854

A.7.3.28.2	Test Requirements	1857

A.7.3.29	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1857

A.7.3.29.1	Test Purpose and Environment	1857

A.7.3.29.2	Test Requirements	1860

A.7.3.30	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1860

A.7.3.30.1	Test Purpose and Environment	1860

A.7.3.30.2	Test Requirements	1863

A.7.3.31	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync for UE Category 0	1863

A.7.3.31.1	Test Purpose and Environment	1863

A.7.3.31.2	Test Requirements	1866

A.7.3.32	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1866

A.7.3.32.1	Test Purpose and Environment	1866

A.7.3.32.2	Test Requirements	1869

A.7.3.33	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category 0	1869

A.7.3.33.1	Test Purpose and Environment	1869

A.7.3.33.2	Test Requirements	1872

A.7.3.34	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for UE Category 0	1872

A.7.3.34.1	Test Purpose and Environment	1872

A.7.3.34.2	Test Requirements	1875

A.7.3.35	E-UTRAN TDD Radio Link Monitoring Test for In-sync for UE category 0	1875

A.7.3.35.1	Test Purpose and Environment	1875

A.7.3.35.2	Test Requirements	1878

A.7.3.36	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category 0	1878

A.7.3.36.1	Test Purpose and Environment	1878

A.7.3.36.2	Test Requirements	1881

A.7.3.37	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE category 0	1881

A.7.3.37.1	Test Purpose and Environment	1881

A.7.3.37.2	Test Requirements	1884

A.7.3.38	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1884

A.7.3.38.1	Test Purpose and Environment	1884

A.7.3.38.2	Test Requirements	1887

A.7.3.39	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in asynchronous DC	1887

A.7.3.39.1	Test Purpose and Environment	1887

A.7.3.39.2	Test Requirements	1890

A.7.3.40	E-UTRAN TDD-TDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC	1891

A.7.3.40.1	Test Purpose and Environment	1891

A.7.3.40.2	Test Requirements	1893

A.7.3.41	E-UTRAN FDD-FDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1894

A.7.3.41.1	Test Purpose and Environment	1894

A.7.3.41.2	Test Requirements	1897

A.7.3.42	E-UTRAN FDD-FDD DC Radio Link Monitoring Test for In-sync in DRX in asynchronous DC	1897

A.7.3.42.1	Test Purpose and Environment	1897

A.7.3.42.2	Test Requirements	1900

A.7.3.43	E-UTRAN TDD-TDD Radio Link Monitoring Test for In-sync in DRX in synchronous dual connectivity	1900

A.7.3.43.1	Test Purpose and Environment	1900

A.7.3.43.2	Test Requirements	1903

A.7.3.44	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in FDD	1903

A.7.3.44.1	Test Purpose and Environment	1903

A.7.3.44.2	Test Requirements	1906

A.7.3.45	E-UTRAN TDD-FDD DC Radio Link Monitoring Test for Out-of-sync in DRX in synchronous DC with PCell in TDD	1907

A.7.3.45.1	Test Purpose and Environment	1907

A.7.3.45.2	Test Requirements	1909

A.7.3.46	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in FDD	1910

A.7.3.46.1	Test Purpose and Environment	1910

A.7.3.46.2	Test Requirements	1913

A.7.3.47	E-UTRAN TDD-FDD Radio Link Monitoring Test for In-sync in DRX for PSCell in synchronous DC with PCell in TDD	1913

A.7.3.47.1	Test Purpose and Environment	1913

A.7.3.47.2	Test Requirements	1916

A.7.3.48	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1916

A.7.3.48.1	Test Purpose and Environment	1916

A.7.3.48.2	Test Requirements	1919

A.7.3.49	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1919

A.7.3.49.1	Test Purpose and Environment	1919

A.7.3.49.2	Test Requirements	1922

A.7.3.50	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1922

A.7.3.50.1	Test Purpose and Environment	1922

A.7.3.50.2	Test Requirements	1925

A.7.3.51	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1925

A.7.3.51.1	Test Purpose and Environment	1925

A.7.3.51.2	Test Requirements	1928

A.7.3.52	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1928

A.7.3.52.1	Test Purpose and Environment	1928

A.7.3.52.2	Test Requirements	1931

A.7.3.53	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1931

A.7.3.53.1	Test Purpose and Environment	1931

A.7.3.53.2	Test Requirements	1934

A.7.3.54	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1934

A.7.3.54.1	Test Purpose and Environment	1934

A.7.3.54.2	Test Requirements	1937

A.7.3.55	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1937

A.7.3.55.1	Test Purpose and Environment	1937

A.7.3.55.2	Test Requirements	1940

A.7.3.56	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A	1940

A.7.3.56.1	Test Purpose and Environment	1940

A.7.3.56.2	Test Requirements	1943

A.7.3.57	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A	1943

A.7.3.57.1	Test Purpose and Environment	1943

A.7.3.57.2	Test Requirements	1946

A.7.3.58	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	1946

A.7.3.58.1	Test Purpose and Environment	1946

A.7.3.58.2	Test Requirements	1949

A.7.3.59	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	1949

A.7.3.59.1	Test Purpose and Environment	1949

A.7.3.59.2	Test Requirements	1952

A.7.3.60	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	1952

A.7.3.60.1	Test Purpose and Environment	1952

A.7.3.60.2	Test Requirements	1956

A.7.3.61	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	1956

A.7.3.61.1	Test Purpose and Environment	1956

A.7.3.61.2	Test Requirements	1960

A.7.3.62	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1960

A.7.3.62.1	Test Purpose and Environment	1960

A.7.3.62.2	Test Requirements	1964

A.7.3.63	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	1964

A.7.3.63.1	Test Purpose and Environment	1964

A.7.3.63.2	Test Requirements	1968

A.7.3.64	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	1968

A.7.3.64.1	Test Purpose and Environment	1968

A.7.3.64.2	Test Requirements	1971

A.7.3.65	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	1972

A.7.3.65.1	Test Purpose and Environment	1972

A.7.3.65.2	Test Requirements	1975

A.7.3.66	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	1976

A.7.3.66.1	Test Purpose and Environment	1976

A.7.3.66.2	Test Requirements	1978

A.7.3.67	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	1979

A.7.3.67.1	Test Purpose and Environment	1979

A.7.3.67.2	Test Requirements	1982

A.7.3.68	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1983

A.7.3.68.1	Test Purpose and Environment	1983

A.7.3.68.2	Test Requirements	1985

A.7.3.69	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1985

A.7.3.69.1	Test Purpose and Environment	1985

A.7.3.69.2	Test Requirements	1987

A.7.3.70	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEMode A	1987

A.7.3.70.1	Test Purpose and Environment	1987

A.7.3.70.2	Test Requirements	1989

A.7.3.71	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1989

A.7.3.71.1	Test Purpose and Environment	1989

A.7.3.71.2	Test Requirements	1992

A.7.3.72	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1992

A.7.3.72.1	Test Purpose and Environment	1992

A.7.3.72.2	Test Requirements	1994

A.7.3.73	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeA	1994

A.7.3.73.1	Test Purpose and Environment	1994

A.7.3.73.2	Test Requirements	1996

A.7.3.74	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	1996

A.7.3.74.1	Test Purpose and Environment	1996

A.7.3.74.2	Test Requirements	1999

A.7.3.75	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	1999

A.7.3.75.1	Test Purpose and Environment	1999

A.7.3.75.2	Test Requirements	2001

A.7.3.76	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	2002

A.7.3.76.1	Test Purpose and Environment	2002

A.7.3.76.2	Test Requirements	2005

A.7.3.77	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	2005

A.7.3.77.1	Test Purpose and Environment	2005

A.7.3.77.2	Test Requirements	2008

A.7.3.78	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for non-BL CE UE in CEMode A	2008

A.7.3.78.1	Test Purpose and Environment	2008

A.7.3.78.2	Test Requirements	2011

A.7.3.79	E-UTRAN TDD Radio Link Monitoring Test for In-Sync for non-BL CE UE in CEMode A	2011

A.7.3.79.1	Test Purpose and Environment	2011

A.7.3.79.2	Test Requirements	2014

A.7.3.80	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync in DRX for non-BL CE UE configured in CEMode A	2014

A.7.3.80.1	Test Purpose and Environment	2014

A.7.3.80.2	Test Requirements	2017

A.7.3.81	E-UTRAN TDD Radio Link Monitoring Test for In-sync in DRX for non-BL CE UE configured in CEMode A	2017

A.7.3.81.1	Test Purpose and Environment	2017

A.7.3.81.2	Test Requirements	2020

A.7.3.82	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2020

A.7.3.82.1	Test Purpose and Environment	2020

A.7.3.82.2	Test Requirements	2023

A.7.3.83	E-UTRAN FD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2023

A.7.3.83.1	Test Purpose and Environment	2023

A.7.3.83.2	Test Requirements	2024

A.7.3.84	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2025

A.7.3.84.1	Test Purpose and Environment	2025

A.7.3.84.2	Test Requirements	2027

A.7.3.85	E-UTRAN HD-FDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2027

A.7.3.85.1	Test Purpose and Environment	2027

A.7.3.85.2	Test Requirements	2028

A.7.3.86	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB	2029

A.7.3.86.1	Test Purpose and Environment	2029

A.7.3.86.2	Test Requirements	2031

A.7.3.87	E-UTRAN TDD Early In-Sync reporting Test for Cat-M1 UE in CEModeB	2031

A.7.3.87.1	Test Purpose and Environment	2031

A.7.3.87.2	Test Requirements	2033

A.7.3.88	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in normal coverage	2034

A.7.3.88.1	Test Purpose and Environment	2034

A.7.3.88.2	Test Requirements	2038

A.7.3.89	TDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 In-band mode in enhanced coverage	2038

A.7.3.89.1	Test Purpose and Environment	2038

A.7.3.89.2	Test Requirements	2042

A.7.3.90	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Normal Coverage	2042

A.7.3.90.1	Test Purpose and Environment	2042

A.7.3.90.2	Test Requirements	2046

A.7.3.91	TDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2046

A.7.3.91.1	Test Purpose and Environment	2046

A.7.3.91.2	Test Requirements	2050

A.7.3.92	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Normal Coverage	2050

A.7.3.92.1	Test Purpose and Environment	2050

A.7.3.92.2	Test Requirements	2053

A.7.3.93	TDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 In-Band mode in Enhanced Coverage	2054

A.7.3.93.1	Test Purpose and Environment	2054

A.7.3.93.2	Test Requirements	2057

A.7.3.94	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	2058

A.7.3.94.1	Test Purpose and Environment	2058

A.7.3.94.2	Test Requirements	2060

A.7.3.95	TDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 guard band mode in Enhanced Coverage	2061

A.7.3.95.1	Test Purpose and Environment	2061

A.7.3.95.2	Test Requirements	2064

A.7.3.96	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2065

A.7.3.96.1	Test Purpose and Environment	2065

A.7.3.97	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2066

A.7.3.97.1	Test Purpose and Environment	2066

A.7.3.97.2	Test Requirements	2069

A.7.3.98	E-UTRAN TDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for MPDCCH performance improvement	2069

A.7.3.98.1	Test Purpose and Environment	2069

A.7.3.98.2	Test Requirements	2072

A.7.3.99	E-UTRAN FD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2072

A.7.3.99.1	Test Purpose and Environment	2072

A.7.3.99.2	Test Requirements	2075

A.7.3.100	E-UTRAN HD-FDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2075

A.7.3.100.1	Test Purpose and Environment	2075

A.7.3.100.2	Test Requirements	2077

A.7.3.101	E-UTRAN TDD Early Out-of-sync reporting Test for Cat-M1 UE in CEModeB for MPDCCH performance improvement	2077

A.7.3.101.1	Test Purpose and Environment	2077

A.7.3.101.2	Test Requirements	2079

A.7.4	Interruption for Dual Connectivity	2079

A.7.4.1	E-UTRAN FDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2079

A.7.4.1.1	Test Purpose and Environment	2079

A.7.4.1.2	Test Requirements	2082

A.7.4.2	E-UTRAN TDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2082

A.7.4.2.1	Test Purpose and Environment	2082

A.7.4.2.2	Test Requirements	2084

A.7.4.3	E-UTRAN FDD-FDD Interruption at transitions between active and non-active during DRX in asynchronous dual connectivity	2084

A.7.4.3.1	Test Purpose and Environment	2084

A.7.4.3.2	Test Requirements	2086

A.7.4.4	E-UTRAN FDD-TDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2086

A.7.4.4.1	Test Purpose and Environment	2086

A.7.4.4.2	Test Requirements	2089

A.7.4.5	E-UTRAN TDD-FDD DC interruption at transitions between active and non-active during DRX in synchronous DC	2089

A.7.4.5.1	Test Purpose and Environment	2089

A.7.4.5.2	Test Requirements	2091

A.7.4.6	E-UTRAN FDD-TDD DC interruption at SRS carrier based switching	2091

A.7.4.6.1	Test Purpose and Environment	2091

A.7.4.6.2	Test Requirements	2093

A.7.4.7	E-UTRAN TDD-TDD DC interruption at SRS carrier based switching	2093

A.7.4.7.1	Test Purpose and Environment	2093

A.7.4.7.2	Test Requirements	2097

A.7.5	Proximity-based Services	2097

A.7.5.1	E-UTRAN FDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2097

A.7.5.1.1	Test Purpose and Environment	2097

A.7.5.1.2	Test Requirements	2098

A.7.5.2	E-UTRAN TDD – UE ProSe Direct Discovery Transmission Timing Accuracy Test	2099

A.7.5.2.1	Test Purpose and Environment	2099

A.7.5.1.2	Test Requirements	2099

A.7.5.3	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery	2100

A.7.5.3.1	Test Purpose and Environment	2100

A.7.5.3.2	Test Requirements	2101

A.7.5.4	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	2102

A.7.5.4.1	Test Purpose and Environment	2102

A.7.5.4.2	Test Requirements	2103

A.7.5.5	E-UTRAN FDD - Interruptions due to ProSe Direct Communication	2104

A.7.5.5.1	Test Purpose and Environment	2104

A.7.5.5.2	Test Requirements	2106

A.7.5.6	E-UTRAN FDD - Interruptions due to ProSe Direct Discovery with discovery period less than 320ms	2107

A.7.5.6.1	Test Purpose and Environment	2107

A.7.5.6.2	Test Requirements	2108

A.7.5.7	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery	2109

A.7.5.7.1	Test Purpose and Environment	2109

A.7.5.7.2	Test Requirements	2111

A.7.5.8	E-UTRAN FDD-FDD - Cell reselection and timing accuracy for ProSe Direct Discovery transmission on non-serving frequency	2111

A.7.5.8.1	Test Purpose and Environment	2111

A.7.5.8.2	Test Requirements	2113

A.7.5.9	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery reception on non-serving frequency	2114

A.7.5.9.1	Test Purpose and Environment	2114

A.7.5.9.2	Test Requirements	2116

A.7.5.10	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Discovery transmission on non-serving frequency	2117

A.7.5.10.1	Test Purpose and Environment	2117

A.7.5.10.2	Test Requirements	2119

A.7.5.11	E-UTRAN FDD-FDD - Interruptions due to ProSe Direct Communication on non-serving frequency	2120

A.7.5.11.1	Test Purpose and Environment	2120

A.7.5.11.2	Test Requirements	2122

A.7.5.12	E-UTRAN FDD - Selection / Reselection of ProSe relay UE	2122

A.7.5.12.1	Test Purpose and Environment	2122

A.7.5.12.2	Test Requirements	2126

A.7.6	Interruption for carrier aggregation	2127

A.7.6.1	E-UTRAN FDD-TDD CA interruption at SRS carrier based switching	2127

A.7.6.1.1	Test Purpose and Environment	2127

A.7.6.1.2	Test Requirements	2130

A.7.6.2	E-UTRAN TDD-TDD CA interruption at SRS carrier based switching	2130

A.7.6.2.1	Test Purpose and Environment	2130

A.7.6.2.2	Test Requirements	2133

A.8	UE Measurements Procedures	2134

A.8.1	E-UTRAN FDD Intra-frequency Measurements	2134

A.8.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2134

A.8.1.1.1	Test Purpose and Environment	2134

A.8.1.1.2	Test Requirements	2135

A.8.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2136

A.8.1.2.1	Test Purpose and Environment	2136

A.8.1.2.2	Test Requirements	2137

A.8.1.3	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2138

A.8.1.3.1	Test Purpose and Environment	2138

A.8.1.3.2	Test Requirements	2140

A.8.1.4	Void	2140

A.8.1.5	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2140

A.8.1.5.1	Test Purpose and Environment	2140

A.8.1.5.2	Test Requirements	2142

A.8.1.6	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2142

A.8.1.6.1	Test Purpose and Environment	2142

A.8.1.6.2	Test Requirements	2144

A.8.1.7 	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2144

A.8.1.7.1	Test Purpose and Environment	2144

A.8.1.7.2	Test Requirements	2146

A.8.1.8	E-UTRAN FDD-FDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2147

A.8.1.8.1	Test Purpose and Environment	2147

A.8.1.8.2	Test Requirements	2149

A.8.1.9	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for 5MHz bandwidth	2150

A.8.1.9.1	Test Purpose and Environment	2150

A.8.1.9.2	Test Requirements	2150

A.8.1.10	E-UTRAN FDD-FDD Intra-Frequency Event Triggered Reporting under Fading Propagation Conditions in Synchronous Cells with DRX for 5 MHz Bandwidth	2150

A.8.1.10.1	Test Purpose and Environment	2150

A.8.1.10.2	Test Requirements	2151

A.8.1.11	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2151

A.8.1.11.1	Test Purpose and Environment	2151

A.8.1.11.2	Test Requirements	2153

A.8.1.12	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2154

A.8.1.12.1	Test Purpose and Environment	2154

A.8.1.12.2	Test Requirements	2155

A.8.1.13	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2156

A.8.1.13.1	Test Purpose and Environment	2156

A.8.1.13.2	Test Requirements	2158

A.8.1.14	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category 0	2158

A.8.1.14.1	Test Purpose and Environment	2158

A.8.1.14.2	Test Requirements	2160

A.8.1.15	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2161

A.8.1.15.1	Test Purpose and Environment	2161

A.8.1.15.2	Test Requirements	2162

A.8.1.16	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2163

A.8.1.16.1	Test Purpose and Environment	2163

A.8.1.16.2	Test Requirements	2165

A.8.1.17	Void	2165

A.8.1.18	Void	2165

A.8.1.19	E-UTRAN FDD-FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2165

A.8.1.19.1	Test Purpose and Environment	2165

A.8.1.19.2	Test Requirements	2167

A.8.1.20	E-UTRAN FDD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2168

A.8.1.20.1	Test Purpose and Environment	2168

A.8.1.20.2	Test Requirements	2170

A.8.1.21	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for UE category 0	2170

A.8.1.21.1	Test Purpose and Environment	2170

A.8.1.21.2	Test Requirements	2172

A.8.1.22	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for UE category 0	2173

A.8.1.22.1	Test Purpose and Environment	2173

A.8.1.22.2	Test Requirements	2175

A.8.1.23	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2175

A.8.1.23.1	Test Purpose and Environment	2175

A.8.1.23.2	Test Requirements	2176

A.8.1.24	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2177

A.8.1.24.1	Test Purpose and Environment	2177

A.8.1.24.2	Test Requirements	2179

A.8.1.25	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2179

A.8.1.25.1	Test Purpose and Environment	2179

A.8.1.25.2	Test Requirements	2181

A.8.1.26	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeA	2181

A.8.1.26.1	Test Purpose and Environment	2181

A.8.1.27	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2184

A.8.1.27.1	Test Purpose and Environment	2184

A.8.1.27.2	Test Requirements	2186

A.8.1.28	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2186

A.8.1.28.1	Test Purpose and Environment	2186

A.8.1.28.2	Test Requirements	2188

A.8.1.29	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA	2188

A.8.1.29.1	Test Purpose and Environment	2188

A.8.1.29.2	Test Requirements	2190

A.8.1.30	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	2191

A.8.1.30.1	Test Purpose and Environment	2191

A.8.1.30.2	Test Requirements	2193

A.8.1.31 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2193

A.8.1.31.1	Test Purpose and Environment	2193

A.8.1.31.2	Test Requirements	2195

A.8.1.32 E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2196

A.8.1.32.1	Test Purpose and Environment	2196

A.8.1.32.2	Test Requirements	2197

A.8.1.33	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Cat-M1 UE in CEModeB	2198

A.8.1.33.1	Test Purpose and Environment	2198

A.8.1.33.2	Test Requirements	2199

A.8.1.34	E-UTRAN HD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2200

A.8.1.34.1	Test Purpose and Environment	2200

A.8.1.34.2	Test Requirements	2201

A.8.1.35	E-UTRAN TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for Cat-M1 UE in CEModeB	2202

A.8.1.35.1	Test Purpose and Environment	2202

A.8.1.35.2	Test Requirements	2203

A.8.1.36	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2204

A.8.1.36.1	Test Purpose and Environment	2204

A.8.1.36.2	Test Requirements	2205

A.8.1.37	E-UTRAN FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2206

A.8.1.37.1	Test Purpose and Environment	2206

A.8.1.37.2	Test Requirements	2208

A.8.1.38	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2208

A.8.1.38.1	Test Purpose and Environment	2208

A.8.1.38.2	Test Requirements	2209

A.8.1.39	E-UTRAN HD - FDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2210

A.8.1.39.1	Test Purpose and Environment	2210

A.8.1.39.2	Test Requirements	2212

A.8.1.40	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2212

A.8.1.40.1	Test Purpose and Environment	2212

A.8.1.40.2	Test Requirements	2214

A.8.1.41	E-UTRAN FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2214

A.8.1.41.1	Test Purpose and Environment	2214

A.8.1.41.2	Test Requirement	2216

A.8.1.42	E-UTRAN HD-FDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2216

A.8.1.42.1	Test Purpose and Environment	2216

A.8.1.42.2	Test Requirement	2217

A.8.1.43	E-UTRAN FDD-FDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2217

A.8.1.43.1	Test Purpose and Environment	2217

A.8.1.43.2	Test Requirements	2220

A.8.1.44	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2220

A.8.1.44.1	Test Purpose and Environment	2220

A.8.1.44.2	Test Requirements	2223

A.8.1.45	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2224

A.8.1.45.1	Test Purpose and Environment	2224

A.8.1.45.2	Test Requirements	2226

A.8.1.46	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2227

A.8.1.46.1	Test Purpose and Environment	2227

A.8.1.46.2	Test Requirements	2228

A.8.1.47	TDD Intra-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2229

A.8.1.47.1	Test Purpose and Environment	2229

A.8.1.47.2	Test Requirements	2231

A.8.1.48	TDD Intra-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2232

A.8.1.48.1	Test Purpose and Environment	2232

A.8.1.48.2	Test Requirements	2234

A.8.1.49	TDD Intra-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2235

A.8.1.49.1	Test Purpose and Environment	2235

A.8.1.49.2	Test Requirements	2236

A.8.150	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2237

A.8.1.50.1	Test Purpose and Environment	2237

A.8.1.50.2	Test Requirements	2239

A.8.1.51	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2240

A.8.1.51.1	Test Purpose and Environment	2240

A.8.1.51.2	Test Requirements	2242

A.8.1.52	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2243

A.8.1.52.1	Test Purpose and Environment	2243

A.8.1.52.2	Test Requirements	2244

A.8.1.53	TDD Inter-frequency neighbour cell measurement for UE category NB1 in In-Band mode under normal coverage	2245

A.8.1.53.1	Test Purpose and Environment	2245

A.8.1.53.2	Test Requirements	2247

A.8.1.54	TDD Inter-frequency neighbour cell measurement for UE category NB1 in guard-band mode under normal coverage	2248

A.8.1.54.1	Test Purpose and Environment	2248

A.8.1.54.2	Test Requirements	2250

A.8.1.55	TDD Inter-frequency neighbour cell measurement for UE category NB1 in standalone mode under normal coverage	2251

A.8.1.55.1	Test Purpose and Environment	2251

A.8.1.55.2	Test Requirements	2252

A.8.2	E-UTRAN TDD Intra-frequency Measurements	2253

A.8.2.1	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells	2253

A.8.2.1.1	Test Purpose and Environment	2253

A.8.2.1.2	Test Requirements	2254

A.8.2.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX	2254

A.8.2.2.1	Test Purpose and Environment	2254

A.8.2.2.2	Test Requirements	2257

A.8.2.3	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2257

A.8.2.3.1	Test Purpose and Environment	2257

A.8.2.3.2	Test Requirements	2259

A.8.2.4	E-UTRAN TDD - TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2260

A.8.2.4.1	Test Purpose and Environment	2260

A.8.2.4.2	Test Requirements	2262

A.8.2.5	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	2262

A.8.2.5.1	Test Purpose and Environment	2262

A.8.2.5.2	Test Requirements	2264

A.8.2.6	E-UTRAN TDD-TDD Intra-Frequency Event-Triggered Reporting under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	2265

A.8.2.6.1	Test Purpose and Environment	2265

A.8.2.6.2	Test Requirements	2268

A.8.2.7	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2269

A.8.2.7.1	Test Purpose and Environment	2269

A.8.2.7.2	Test Requirements	2270

A.8.2.8	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2271

A.8.2.8.1	Test Purpose and Environment	2271

A.8.2.8.2	Test Requirements	2273

A.8.2.9	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps for Cat-M1 UE in CEModeB	2273

A.8.2.9.1	Test Purpose and Environment	2273

A.8.2.9.2	Test Requirements	2275

A.8.2.10	E-UTRAN TDD Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX for Cat-M1 UE in CEModeB	2276

A.8.2.10.1	Test Purpose and Environment	2276

A.8.2.10.2	Test Requirements	2278

A.8.2.11	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhancedMeasFlag	2278

A.8.2.11.1	Test Purpose and Environment	2278

A.8.2.11.2	Test Requirements	2280

A.8.2.12	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells for UE category 0	2280

A.8.2.12.1	Test Purpose and Environment	2280

A.8.2.12.2	Test Requirements	2282

A.8.2.13	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells with DRX for UE category 0	2283

A.8.2.13.1	Test Purpose and Environment	2283

A.8.2.13.2	Test Requirements	2285

A.8.2.14	E-UTRAN TDD intra-frequency event triggered reporting for serving cell under fading propagation conditions for UE category M1 in CEModeA without gap	2285

A.8.2.14.1	Test Purpose and Environment	2285

A.8.2.14.2	Test Requirement	2287

A.8.2.15	E-UTRAN TDD-TDD intra-frequency event triggered reporting with DRX for UE configured with highSpeedEnhMeasFlag2-r16	2287

A.8.2.15.1	Test Purpose and Environment	2287

A.8.2.15.2	Test Requirements	2289

A.8.3	E-UTRAN FDD - FDD Inter-frequency Measurements	2289

A.8.3.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2289

A.8.3.1.1	Test Purpose and Environment	2289

A.8.3.1.2	Test Requirements	2291

A.8.3.2	E-UTRAN FDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2291

A.8.3.2.1	Test Purpose and Environment	2291

A.8.3.2.2	Test Requirements	2294

A.8.3.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN propagation conditions in asynchronous cells with DRX when L3 filtering is used	2294

A.8.3.3.1	Test Purpose and Environment	2294

A.8.3.3.2	Test Requirements	2297

A.8.3.4	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2297

A.8.3.4.1	Test Purpose and Environment	2297

A.8.3.4.2	Test Requirements	2299

A.8.3.5	E-UTRAN FDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2299

A.8.3.5.2	Test Requirements	2301

A.8.3.6	E-UTRAN FDD-FDD Inter-frequency event triggered reporting without measurement gaps under AWGN propagation conditions in asynchronous cells	2301

A.8.3.6.1	Test Purpose and Environment	2301

A.8.3.6.2	Test Requirements	2303

A.8.3.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2304

A.8.3.7.1	Test Purpose and Environment	2304

A.8.3.7.2	Test Requirements	2306

A.8.3.8	FDD-FDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2307

A.8.3.8.1	Test Purpose and Environment	2307

A.8.3.8.2	Test Requirements	2310

A.8.3.9	FDD-FDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2310

A.8.3.9.1	Test Purpose and Environment	2310

A.8.3.9.2	Test Requirements	2314

A.8.3.10	E-UTRAN FDD-FDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2315

A.8.3.10.1	Test Purpose and Environment	2315

A.8.3.10.2	Test Requirements	2316

A.8.3.11	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells with burst gap	2316

A.8.3.11.1	Test Purpose and Environment	2316

A.8.3.11.2	Test Requirement	2318

A.8.3.12	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2319

A.8.3.12.1	Test Purpose and Environment	2319

A.8.3.12.2	Test Requirement	2320

A.8.3.13	E-UTRAN HD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2321

A.8.3.13.1	Test Purpose and Environment	2321

A.8.3.13.2	Test Requirement	2322

A.8.3.14	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2323

A.8.3.14.1	Test Purpose and Environment	2323

A.8.3.14.2	Test Requirement	2324

A.8.3.15	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2325

A.8.3.15.1	Test Purpose and Environment	2325

A.8.3.15.2	Test Requirement	2326

A.8.3.16	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	2327

A.8.3.16.1	Test Purpose and Environment	2327

A.8.3.16.2	Test Requirement	2329

A.8.3.17	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2329

A.8.3.17.1	Test Purpose and Environment	2329

A.8.3.17.2	Test Requirement	2332

A.8.3.18	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2332

A.8.3.18.1	Test Purpose and Environment	2332

A.8.3.18.2	Test Requirement	2335

A.8.3.19	E-UTRAN HD-FDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2335

A.8.3.19.1	Test Purpose and Environment	2335

A.8.3.19.2	Test Requirement	2338

A.8.4	E-UTRAN TDD - TDD Inter-frequency Measurements	2338

A.8.4.1	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	2338

A.8.4.1.1	Test Purpose and Environment	2338

A.8.4.1.2	Test Requirements	2340

A.8.4.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in synchronous cells	2340

A.8.4.2.1	Test Purpose and Environment	2340

A.8.4.2.2	Test Requirements	2343

A.8.4.3	E-UTRAN TDD-TDD inter-frequency event triggered reporting under AWGN propagation conditions in synchronous cells with DRX when L3 filtering is used	2343

A.8.4.3.1	Test Purpose and Environment	2343

A.8.4.3.2	Test Requirements	2345

A.8.4.4	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2346

A.8.4.4.1	Test Purpose and Environment	2346

A.8.4.4.2	Test Requirements	2348

A.8.4.5	E-UTRAN TDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps with DRX	2349

A.8.4.5.1	Test Purpose and Environment	2349

A.8.4.5.2	Test Requirements	2351

A.8.4.6	E-UTRAN TDD-TDD Inter-frequency event triggered reporting for TDD UL/DL configuration 0	2351

A.8.4.6.1	Test Purpose and Environment	2351

A.8.4.6.2	Test Requirements	2352

A.8.4.7	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for Increased Carrier Monitoring without Reduced Performance Group	2352

A.8.4.7.1	Test Purpose and Environment	2352

A.8.4.7.2	Test Requirements	2356

A.8.4.8	TDD-TDD Interfrequency correct reporting of measurement events with reduced performance group configured, non DRX	2356

A.8.4.8.1	Test Purpose and Environment	2356

A.8.4.8.2	Test Requirements	2359

A.8.4.9	TDD-TDD Inter-frequency correct reporting of measurement events with reduced performance group configured, DRX	2359

A.8.4.9.1	Test Purpose and Environment	2359

A.8.4.9.2	Test Requirements	2364

A.8.4.10	E-UTRAN TDD-TDD Inter-frequency event triggered reporting with MGL=3ms under fading propagation conditions in synchronous cells	2364

A.8.4.10.1	Test Purpose and Environment	2364

A.8.4.10.2	Test Requirements	2366

A.8.4.11	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells with burst gap	2366

A.8.4.11.1	Test Purpose and Environment	2366

A.8.4.11.2	Test Requirement	2368

A.8.4.12	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	2368

A.8.4.12.1	Test Purpose and Environment	2368

A.8.4.12.2	Test Requirement	2370

A.8.4.13	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	2371

A.8.4.13.1	Test Purpose and Environment	2371

A.8.4.13.2	Test Requirement	2372

A.8.4.14	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeA in DRX	2373

A.8.4.14.1	Test Purpose and Environment	2373

A.8.4.14.2	Test Requirement	2375

A.8.4.15	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells for UE category M1 in CEModeB in DRX	2375

A.8.4.15.1	Test Purpose and Environment	2375

A.8.4.15.2	Test Requirement	2378

A.8.5	E-UTRAN FDD - UTRAN FDD Measurements	2378

A.8.5.1	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	2378

A.8.5.1.1	Test Purpose and Environment	2378

A.8.5.1.2	Test Requirements	2380

A.8.5.2	E-UTRAN FDD - UTRAN FDD SON ANR cell search reporting under AWGN propagation conditions	2380

A.8.5.2.1	Test Purpose and Environment	2380

A.8.5.2.2	Test Requirements	2382

A.8.5.3	E-UTRAN FDD-UTRAN FDD event triggered reporting when DRX is used under fading propagation conditions	2382

A.8.5.3.1	Test Purpose and Environment	2382

A.8.5.3.2	Test Requirements	2385

A.8.5.4	E-UTRAN FDD - UTRAN FDD enhanced cell identification under AWGN propagation conditions	2385

A.8.5.4.1	Test Purpose and Environment	2385

A.8.5.4.2	Test Requirements	2387

A.8.5.5	E- UTRAN FDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2387

A.8.5.5.1	Test Purpose and Environment	2387

A.8.5.5.2	Test Requirements	2390

A.8.5.6	E-UTRAN FDD - UTRAN FDD event triggered reporting without measurement gaps under AWGN propagation conditions	2390

A.8.5.6.1	Test Purpose and Environment	2390

A.8.5.6.2	Test Requirements	2391

A.8.5.7	E-UTRAN FDD - UTRAN FDD Event Triggered Reporting under Fading Propagation Conditions for 5 MHz Bandwidth	2392

A.8.5.7.1	Test Purpose and Environment	2392

A.8.5.7.2	Test Requirements	2392

A.8.5.8	E-UTRA FDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2392

A.8.5.8.1	Test Purpose and Environment	2392

A.8.5.8.2	Test Requirements	2395

A.8.6	E-UTRAN TDD - UTRAN FDD Measurements	2395

A.8.6.1	E-UTRAN TDD - UTRAN FDD event triggered reporting under fading propagation conditions	2395

A.8.6.1.1	Test Purpose and Environment	2395

A.8.6.1.2	Test Requirements	2397

A.8.6.2	E- UTRAN TDD - UTRAN FDD identification of a new CGI of UTRAN cell using autonomous gaps	2397

A.8.6.2.1	Test Purpose and Environment	2397

A.8.6.2.2	Test Requirements	2400

A.8.6.3	E-UTRA TDD InterRAT UTRA FDD correct reporting of measurement events with reduced performance group configured, non DRX	2400

A.8.6.3.1	Test Purpose and Environment	2400

A.8.6.3.2	Test Requirements	2403

A.8.7	E-UTRAN TDD – UTRAN TDD Measurements	2403

A.8.7.1	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	2403

A.8.7.1.1	Test Purpose and Environment	2403

A.8.7.1.1.1	Void	2403

A.8.7.1.1.2	1.28 Mcps TDD option	2403

A.8.7.1.1.3	Void	2405

A.8.7.1.2	Test Requirements	2405

A.8.7.1.2.1	Void	2405

A.8.7.1.2.2	1.28 Mcps TDD option	2405

A.8.7.1.2.3	Void	2405

A.8.7.2	E-UTRAN TDD-UTRAN TDD cell search when DRX is used under fading propagation conditions	2405

A.8.7.2.1	Test Purpose and Environment	2405

A.8.7.2.2	Test Requirements	2408

A.8.7.3	E-UTRAN TDD - UTRAN TDD SON ANR cell search reporting in AWGN propagation conditions	2409

A.8.7.3.1	Test Purpose and Environment	2409

A.8.7.3.2	Test Parameters	2409

A.8.7.3.3	Test Requirements	2410

A.8.7.4	E-UTRAN TDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2411

A.8.7.4.1	Test Purpose and Environment	2411

A.8.7.4.2	Test Requirements	2413

A.8.7.5	E-UTRA TDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2413

A.8.7.5.1	Test Purpose and Environment	2413

A.8.7.5.2	Test Requirements	2415

A.8.7A	E-UTRAN FDD – UTRAN TDD Measurements	2415

A.8.7A.1	E-UTRA FDD InterRAT UTRA TDD correct reporting of measurement events with reduced performance group configured, non DRX	2415

A.8.7A.1.1	Test Purpose and Environment	2415

A.8.7A.1.2	Test Requirements	2418

A.8.8	E-UTRAN FDD – GSM Measurements	2418

A.8.8.1	E-UTRAN FDD – GSM event triggered reporting in AWGN	2418

A.8.8.1.1	Test Purpose and Environment	2418

A.8.8.1.2	Test Requirements	2420

A.8.8.2	E-UTRAN FDD-GSM event triggered reporting when DRX is used in AWGN	2420

A.8.8.2.1	Test Purpose and Environment	2420

A.8.8.2.2	Test Requirements	2422

A.8.8.3	E-UTRAN FDD – GSM event triggered reporting in AWGN with enhanced BSIC identification	2423

A.8.8.3.1	Test Purpose and Environment	2423

A.8.8.3.2	Test Requirements	2424

A.8.9	E-UTRAN FDD - UTRAN TDD measurements	2425

A.8.9.1	E-UTRAN FDD - UTRAN TDD event triggered reporting in fading propagation conditions	2425

A.8.9.1.1	Test Purpose and Environment	2425

A.8.9.1.2	Test Requirements	2426

A.8.9.2	E-UTRAN FDD - UTRAN TDD enhanced cell identification under AWGN propagation conditions	2427

A.8.9.2.1	Test Purpose and Environment	2427

A.8.9.2.2	Test Requirements	2429

A.8.10	E-UTRAN TDD – GSM Measurements	2429

A.8.10.1	E-UTRAN TDD – GSM event triggered reporting in AWGN	2429

A.8.10.1.1	Test Purpose and Environment	2429

A.8.10.1.2	Test Requirements	2430

A.8.10.2	E-UTRAN TDD-GSM event triggered reporting when DRX is used in AWGN	2431

A.8.10.2.1	Test Purpose and Environment	2431

A.8.10.2.2	Test Requirements	2433

A.8.11	Monitoring of Multiple Layers	2433

A.8.11.1	Multiple E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions	2433

A.8.11.1.1	Test Purpose and Environment	2433

A.8.11.1.2	Test Requirements	2435

A.8.11.2	E-UTRAN TDD – E-UTRAN TDD and E-UTRAN TDD Inter-frequency event triggered reporting under fading propagation conditions	2436

A.8.11.2.1	Test Purpose and Environment	2436

A.8.11.2.2	Test Requirements	2437

A.8.11.3	E-UTRAN FDD-FDD Inter-frequency and UTRAN FDD event triggered reporting under fading propagation conditions	2438

A.8.11.3.1	Test Purpose and Environment	2438

A.8.11.3.2	Test Requirements	2440

A.8.11.4	InterRAT E-UTRA TDD to E-UTRA TDD and UTRA TDD cell search test case	2440

A.8.11.4.1	Test Purpose and Environment	2440

A.8.11.4.2	Test Requirements	2443

A.8.11.5	Combined E-UTRAN FDD – E-UTRA FDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2443

A.8.11.5.1	Test Purpose and Environment	2443

A.8.11.5.2	Test Requirements	2445

A.8.11.6	Combined E-UTRAN TDD – E-UTRA TDD and GSM cell search. E-UTRA cells in fading; GSM cell in static propagation conditions	2446

A.8.11.6.1	Test Purpose and Environment	2446

A.8.11.6.2	Test Requirements	2448

A.8.12	RSTD Intra-frequency Measurements	2449

A.8.12.1	E-UTRAN FDD intra-frequency RSTD measurement reporting delay test case	2449

A.8.12.1.1	Test Purpose and Environment	2449

A.8.12.1.2	Test Requirements	2453

A.8.12.1.2A	Test Requirements for UE Category 1bis	2453

A.8.12.2	E-UTRAN TDD intra-frequency RSTD measurement reporting delay test case	2453

A.8.12.2.1	Test Purpose and Environment	2453

A.8.12.2.2	Test Requirements	2458

A.8.12.2.2A	Test Requirements for UE Category 1bis	2458

A.8.12.3	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A	2458

A.8.12.3.1	Test Purpose and Environment	2458

A.8.12.3.2	Test Requirements	2464

A.8.12.4	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A	2464

A.8.12.4.1	Test Purpose and Environment	2464

A.8.12.4.2	Test Requirements	2469

A.8.12.5	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A	2469

A.8.12.5.1	Test Purpose and Environment	2469

A.8.12.5.2	Test Requirements	2474

A.8.12.6	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B	2474

A.8.12.6.1	Test Purpose and Environment	2474

A.8.12.6.2	Test Requirements	2479

A.8.12.7	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B	2479

A.8.12.7.1	Test Purpose and Environment	2479

A.8.12.7.2	Test Requirements	2484

A.8.12.8	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B	2484

A.8.12.8.1	Test Purpose and Environment	2484

A.8.12.8.2	Test Requirements	2489

A.8.12.9	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2489

A.8.12.9.1	Test Purpose and Environment	2489

A.8.12.9.2	Test Requirements	2494

A.8.12.10	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2494

A.8.12.10.1	Test Purpose and Environment	2494

A.8.12.10.2	Test Requirements	2499

A.8.12.11	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2499

A.8.12.11.1	Test Purpose and Environment	2499

A.8.12.11.2	Test Requirements	2504

A.8.12.12	E-UTRAN FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2504

A.8.12.12.1	Test Purpose and Environment	2504

A.8.12.12.2	Test Requirements	2509

A.8.12.13	E-UTRAN HD-FDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2509

A.8.12.13.1	Test Purpose and Environment	2509

A.8.12.13.2	Test Requirements	2514

A.8.12.14	E-UTRAN TDD intra-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2514

A.8.12.14.1	Test Purpose and Environment	2514

A.8.12.14.2	Test Requirements	2519

A.8.13	RSTD Inter-frequency Measurements	2519

A.8.13.1	E-UTRAN FDD-FDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2519

A.8.13.1.1	Test Purpose and Environment	2519

A.8.13.1.2	Test Requirements	2524

A.8.13.1.2A	Test Requirements for UE Category 1bis	2524

A.8.13.2	E-UTRAN TDD-TDD inter-frequency RSTD measurement reporting delay test case with the reference cell on the serving carrier frequency	2524

A.8.13.2.1	Test Purpose and Environment	2524

A.8.13.2.2	Test Requirements	2530

A.8.13.2.2A	Test Requirements for UE Category 1bis	2530

A.8.13.3	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A	2530

A.8.13.3.1	Test Purpose and Environment	2530

A.8.13.3.2	Test Requirements	2537

A.8.13.4	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A	2537

A.8.13.4.1	Test Purpose and Environment	2537

A.8.13.4.2	Test Requirements	2543

A.8.13.5	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A	2543

A.8.13.5.1	Test Purpose and Environment	2543

A.8.13.5.2	Test Requirements	2549

A.8.13.6	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B	2549

A.8.13.6.1	Test Purpose and Environment	2549

A.8.13.6.2	Test Requirements	2555

A.8.13.7	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B	2555

A.8.13.7.1	Test Purpose and Environment	2555

A.8.13.7.2	Test Requirements	2561

A.8.13.8	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B	2561

A.8.13.8.1	Test Purpose and Environment	2561

A.8.13.8.2	Test Requirements	2567

A.8.13.9	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2567

A.8.13.9.1	Test Purpose and Environment	2567

A.8.13.9.2	Test Requirements	2573

A.8.13.10	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2573

A.8.13.10.1	Test Purpose and Environment	2573

A.8.13.10.2	Test Requirements	2579

A.8.13.11	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode A with longer PRS occasions	2579

A.8.13.11.1	Test Purpose and Environment	2579

A.8.13.11.2	Test Requirements	2585

A.8.13.12	E-UTRAN FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2585

A.8.13.12.1	Test Purpose and Environment	2585

A.8.13.12.2	Test Requirements	2591

A.8.13.13	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2591

A.8.13.13.1	Test Purpose and Environment	2591

A.8.13.13.2	Test Requirements	2597

A.8.13.14	E-UTRAN TDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions	2597

A.8.13.14.1	Test Purpose and Environment	2597

A.8.13.14.2	Test Requirements	2603

A.8.14	E-UTRAN TDD - FDD Inter-frequency Measurements	2603

A.8.14.1	E-UTRAN TDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2603

A.8.14.1.1	Test Purpose and Environment	2603

A.8.14.1.2	Test Requirements	2605

A.8.14.2	E-UTRAN TDD-FDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2605

A.8.14.2.1	Test Purpose and Environment	2605

A.8.14.2.2	Test Requirements	2608

A.8.14.3	E-UTRAN TDD - FDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2608

A.8.14.3.1	Test Purpose and Environment	2608

A.8.14.3.2	Test Requirements	2610

A.8.15	E-UTRAN FDD - TDD Inter-frequency Measurements	2611

A.8.15.1	E-UTRAN FDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	2611

A.8.15.1.1	Test Purpose and Environment	2611

A.8.15.1.2	Test Requirements	2612

A.8.15.2	E-UTRAN FDD-TDD Inter-frequency event triggered reporting when DRX is used under fading propagation conditions in asynchronous cells	2612

A.8.15.2.1	Test Purpose and Environment	2612

A.8.15.2.2	Test Requirements	2615

A.8.15.3	E-UTRAN FDD - TDD Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps	2615

A.8.15.3.1	Test Purpose and Environment	2615

A.8.15.3.2	Test Requirements	2617

A.8.16	E-UTRAN Carrier Aggregation Measurements	2618

A.8.16.1	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX	2618

A.8.16.1.1	Test Purpose and Environment	2618

A.8.16.1.2	Test Requirements	2620

A.8.16.2	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX	2620

A.8.16.2.1	Test Purpose and Environment	2620

A.8.16.2.2	Test Requirements	2622

A.8.16.3	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2622

A.8.16.3.1	Test Purpose and Environment	2622

A.8.16.3.2	Test Requirements	2624

A.8.16.3A	E-UTRAN FDD-FDD Event triggered reporting on deactivated SCell with network controlled PCell interruption in non-DRX	2625

A.8.16.3A.1	Test Purpose and Environment	2625

A.8.16.3A.2	Test Requirements	2626

A.8.16.4	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell  with PCell interruption in non-DRX	2627

A.8.16.4.1	Test Purpose and Environment	2627

A.8.16.4.2	Test Requirements	2628

A.8.16.4A	E-UTRAN TDD-TDD Event triggered reporting on deactivated SCell with PCell interruption in non-DRX	2629

A.8.16.4A.1	Test Purpose and Environment	2629

A.8.16.4A.2	Test Requirements	2631

A.8.16.5	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2632

A.8.16.5.1	Test Purpose and Environment	2632

A.8.16.5.2	Test Requirements	2632

A.8.16.6	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20 MHz bandwidth	2632

A.8.16.6.1	Test Purpose and Environment	2632

A.8.16.6.2	Test Requirements	2633

A.8.16.7	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2633

A.8.16.7.1	 Test Purpose and Environment	2633

A.8.16.7.2	 Test Requirements	2634

A.8.16.8	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 20 MHz bandwidth	2634

A.8.16.8.1	 Test Purpose and Environment	2634

A.8.16.8.2	 Test Requirements	2635

A.8.16.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2635

A.8.16.9.1	Test Purpose and Environment	2635

A.8.16.9.2	Test Requirements	2636

A.8.16.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 10MHz+5MHz	2636

A.8.16.10.1	Test Purpose and Environment	2636

A.8.16.10.2	Test Requirements	2637

A.8.16.11	E-UTRAN FDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2637

A.8.16.11.1	 Test Purpose and Environment	2637

A.8.16.11.2	 Test Requirements	2638

A.8.16.12	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 10MHz+5MHz	2638

A.8.16.12.1	 Test Purpose and Environment	2638

A.8.16.12.2	 Test Requirements	2638

A.8.16.13	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX for 5MHz +5 MHz bandwidth	2638

A.8.16.13.1	Test Purpose and Environment	2638

A.8.16.13.2	Test Requirements	2639

A.8.16.14	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 5 MHz +5 MHz bandwidth	2639

A.8.16.14.1	Test Purpose and Environment	2639

A.8.16.14.2	Test Requirements	2640

A.8.16.15	E-UTRA FDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5 +5 MHz bandwidth	2640

A.8.16.15.1	 Test Purpose and Environment	2640

A.8.16.7.2	 Test Requirements	2640

A.8.16.16	E-UTRA TDD event triggered reporting on deactivated SCell with PCell interruption in non-DRX for 5+5 MHz bandwidth	2640

A.8.16.16.1	 Test Purpose and Environment	2640

A.8.16.16.2	 Test Requirements	2641

A.8.16.17	E-UTRAN FDD activation and deactivation of known SCell in non-DRX	2641

A.8.16.17.1	Test Purpose and Environment	2641

A.8.16.17.2	Test Requirements	2643

A.8.16.17A	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 20MHz	2644

A.8.16.17A.1	Test Purpose and Environment	2644

A.8.16.17A.2	Test Requirements	2644

A.8.16.17B	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2644

A.8.16.17B.1	Test Purpose and Environment	2644

A.8.16.17B.2	Test Requirements	2645

A.8.16.17C	E-UTRAN FDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2645

A.8.16.17C.1	Test Purpose and Environment	2645

A.8.16.17C.2	Test Requirements	2645

A.8.16.18	E-UTRAN TDD activation and deactivation of known SCell in non-DRX	2645

A.8.16.18.1	Test Purpose and Environment	2645

A.8.16.18.2	Test Requirements	2647

A.8.16.18A	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz	2648

A.8.16.18A.1	Test Purpose and Environment	2648

A.8.16.18A.2	Test Requirements	2648

A.8.16.18B	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 10MHz + 5MHz	2648

A.8.16.18B.1	Test Purpose and Environment	2648

A.8.16.18B.2	Test Requirements	2649

A.8.16.18C	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 5MHz + 5MHz	2649

A.8.16.18C.1	Test Purpose and Environment	2649

A.8.16.18C.2	Test Requirements	2649

A.8.16.18D	E-UTRAN TDD activation and deactivation of known SCell in non-DRX for 20MHz + 10MHz	2649

A.8.16.18D.1	Test Purpose and Environment	2649

A.8.16.18D.2	Test Requirements	2650

A.8.16.19	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX	2650

A.8.16.19.1	Test Purpose and Environment	2650

A.8.16.19.2	Test Requirements	2652

A.8.16.19A	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2652

A.8.16.19A.1	Test Purpose and Environment	2652

A.8.16.19A.2	Test Requirements	2653

A.8.16.19B 	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2653

A.8.16.19B.1	Test Purpose and Environment	2653

A.8.16.19B.2	Test Requirements	2653

A.8.16.19C	E-UTRAN FDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2653

A.8.16.19C.1	Test Purpose and Environment	2653

A.8.16.19C.2	Test Requirements	2654

A.8.16.20	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX	2654

A.8.16.20.1	Test Purpose and Environment	2654

A.8.16.20.2	Test Requirements	2656

A.8.16.20A	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz	2657

A.8.16.20A.1	Test Purpose and Environment	2657

A.8.16.20A.2	Test Requirements	2657

A.8.16.20B	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 10MHz + 5MHz	2657

A.8.16.20B.1	Test Purpose and Environment	2657

A.8.16.20B.2	Test Requirements	2658

A.8.16.20C	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 5MHz + 5MHz	2658

A.8.16.20C.1	Test Purpose and Environment	2658

A.8.16.20C.2	Test Requirements	2658

A.8.16.20D	E-UTRAN TDD activation and deactivation of unknown SCell in non-DRX for 20MHz + 10MHz	2658

A.8.16.20D.1	Test Purpose and Environment	2658

A.8.16.20D.2	Test Requirements	2659

A.8.16.21	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX for 20MHz+10MHz	2659

A.8.16.21.1	Test Purpose and Environment	2659

A.8.16.21.2	Test Requirements	2661

A.8.16.22	E-UTRAN TDD event triggered reporting on deactivating SCell with PCell interruption in non-DRX for 20MHz+10MHz	2661

A.8.16.22.1	Test Purpose and Environment	2661

A.8.16.22.2	Test Requirements	2663

A.8.16.23	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in FDD	2663

A.8.16.23.1	Test Purpose and Environment	2663

A.8.16.23.2	Test Requirements	2666

A.8.16.24	E-UTRAN TDD-FDD CA Event Triggered Reporting Under Deactivated SCell in Non-DRX with PCell in TDD	2666

A.8.16.24.1	Test Purpose and Environment	2666

A.8.16.24.2	Test Requirements	2669

A.8.16.25	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in FDD	2669

A.8.16.25.1	Test Purpose and Environment	2669

A.8.16.25.2	Test Requirements	2672

A.8.16.26	E-UTRAN TDD-FDD CA Event triggered reporting on deactivated SCell with PCell interruption in non-DRX with PCell in TDD	2672

A.8.16.26.1	Test Purpose and Environment	2672

A.8.16.26.2	Test Requirements	2675

A.8.16.27	3 DL PCell in FDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2675

A.8.16.27.1	Test Purpose and Environment	2675

A.8.16.27.2	Test Requirements	2680

A.8.16.28	3 DL PCell in TDD CA Event Triggered Reporting with 2 Deactivated SCells in Non-DRX	2680

A.8.16.28.1	Test Purpose and Environment	2680

A.8.16.28.2	Test Requirements	2685

A.8.16.29	3 DL FDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2685

A.8.16.29.1	Test Purpose and Environment	2685

A.8.16.29.2	Test Requirements	2690

A.8.16.30	3 DL TDD CA Event Triggered Reporting under Deactivated SCells in Non-DRX	2690

A.8.16.30.1	Test Purpose and Environment	2690

A.8.16.30.2	Test Requirements	2695

A.8.16.31	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in FDD	2695

A.8.16.31.1	Test Purpose and Environment	2695

A.8.16.31.2	Test Requirements	2700

A.8.16.32	E-UTRAN TDD-FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX and with PCell in TDD	2700

A.8.16.32.1	Test Purpose and Environment	2700

A.8.16.32.2	Test Requirements	2705

A.8.16.33	E-UTRAN FDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2705

A.8.16.33.1	Test Purpose and Environment	2705

A.8.16.33.2	Test Requirements	2710

A.8.16.34	E-UTRAN TDD 3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2710

A.8.16.34.1	Test Purpose and Environment	2710

A.8.16.34.2	Test Requirements	2715

A.8.16.35	3 DL PCell in FDD CA Activation and Deactivation of Known SCell in Non-DRX	2715

A.8.16.35.1	Test Purpose and Environment	2715

A.8.16.35.2	Test Requirements	2717

A.8.16.36	3 DL PCell in TDD CA Activation and Deactivation of Known SCell in Non-DRX	2718

A.8.16.36.1	Test Purpose and Environment	2718

A.8.16.36.2	Test Requirements	2720

A.8.16.37	3 DL FDD CA activation and deactivation of known SCell in non-DRX	2721

A.8.16.37.1	Test Purpose and Environment	2721

A.8.16.37.2	Test Requirements	2723

A.8.16.38	3 DL TDD CA activation and deactivation of known SCell in non-DRX	2724

A.8.16.38.1	Test Purpose and Environment	2724

A.8.16.38.2	Test Requirements	2726

A.8.16.39	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in FDD	2727

A.8.16.39.1	Test Purpose and Environment	2727

A.8.16.39.2	Test Requirements	2730

A.8.16.40	E-UTRA TDD-FDD 3DL CA Activation and Deactivation of Unknown SCell in Non-DRX with PCell in TDD	2730

A.8.16.40.1	Test Purpose and Environment	2730

A.8.16.40.2	Test Requirements	2733

A.8.16.41	3 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2733

A.8.16.41.1	Test Purpose and Environment	2733

A.8.16.41.2	Test Requirements	2735

A.8.16.42	3 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2736

A.8.16.42.1	Test Purpose and Environment	2736

A.8.16.42.2	Test Requirements	2738

A.8.16.43	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in FDD	2739

A.8.16.43.1	Test Purpose and Environment	2739

A.8.16.43.2	Test Requirements	2742

A.8.16.44	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in FDD	2742

A.8.16.44.1	Test Purpose and Environment	2742

A.8.16.44.2	Test Requirements	2745

A.8.16.45	E-UTRAN TDD-FDD CA activation and deactivation of known SCell in non-DRX with PCell in TDD	2745

A.8.16.45.1	Test Purpose and Environment	2745

A.8.16.45.2	Test Requirements	2748

A.8.16.46	E-UTRAN TDD-FDD CA activation and deactivation of unknown SCell in non-DRX with PCell in TDD	2748

A.8.16.46.1	Test Purpose and Environment	2748

A.8.16.46.2	Test Requirements	2750

A.8.16.47	2DL/2UL FDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2751

A.8.16.47.1	Test Purpose and Environment	2751

A.8.16.47.2	Test Requirements	2753

A.8.16.48	2DL/2UL TDD CA activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2754

A.8.16.48.1	Test Purpose and Environment	2754

A.8.16.48.2	Test Requirements	2756

A.8.16.49	2DL/2UL TDD-FDD CA (FDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2757

A.8.16.49.1	Test Purpose and Environment	2757

A.8.16.49.2	Test Requirements	2760

A.8.16.50	2DL/2UL TDD-FDD CA (TDD PCell) activation and deactivation of known PUCCH SCell without valid TA in non-DRX	2760

A.8.16.50.1	Test Purpose and Environment	2760

A.8.16.50.2	Test Requirements	2762

A.8.16.51	E-UTRAN 4 DL FDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2763

A.8.16.51.1	Test Purpose and Environment	2763

A.8.16.51.2	Test Requirements	2768

A.8.16.52	E-UTRAN 4 DL TDD CA Event Triggered Reporting with 3 deactivated SCells in Non-DRX	2768

A.8.16.52.1	Test Purpose and Environment	2768

A.8.16.52.2	Test Requirements	2773

A.8.16.53	4 DL PCell in FDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2773

A.8.16.53.1	Test Purpose and Environment	2773

A.8.16.53.2	Test Requirements	2777

A.8.16.54	4 DL PCell in TDD CA Event Triggered Reporting with 3 Deactivated SCells in Non-DRX	2777

A.8.16.54.1	Test Purpose and Environment	2777

A.8.16.54.2	Test Requirements	2781

A.8.16.55	E-UTRAN FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2781

A.8.16.55.1	Test Purpose and Environment	2781

A.8.16.55.2	Test Requirements	2787

A.8.16.56	E-UTRAN TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2787

A.8.16.56.1	Test Purpose and Environment	2787

A.8.16.56.2	Test Requirements	2793

A.8.16.57	E-UTRAN FDD 4DL CA activation and deactivation of know SCell in non-DRX	2793

A.8.16.57.1	Test Purpose and Environment	2793

A.8.16.57.2	Test Requirements	2795

A.8.16.58	E-UTRAN TDD 4DL CA activation and deactivation of know SCell in non-DRX	2796

A.8.16.58.1	Test Purpose and Environment	2796

A.8.16.58.2	Test Requirements	2798

A.8.16.59	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2799

A.8.16.59.1	Test Purpose and Environment	2799

A.8.16.59.2	Test Requirements	2803

A.8.16.60	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of known SCell in non-DRX	2804

A.8.16.60.1	Test Purpose and Environment	2804

A.8.16.60.2	Test Requirements	2807

A.8.16.61	E-UTRAN FDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2808

A.8.16.61.1	Test Purpose and Environment	2808

A.8.16.61.2	Test Requirements	2811

A.8.16.62	E-UTRAN TDD 4DL CA activation and deactivation of unknown SCell in non-DRX	2811

A.8.16.62.1	Test Purpose and Environment	2811

A.8.16.62.2	Test Requirements	2813

A.8.16.63	E-UTRAN PCell in FDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2814

A.8.16.63.1	Test Purpose and Environment	2814

A.8.16.63.2	Test Requirements	2817

A.8.16.64	E-UTRAN PCell in TDD FDD-TDD 4 DL CA activation and deactivation of unknown SCell in non-DRX	2818

A.8.16.64.1	Test Purpose and Environment	2818

A.8.16.64.2	Test Requirements	2822

A.8.16.65	5 DL FDD-TDD with PCell in FDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2822

A.8.16.65.1	Test Purpose and Environment	2822

A.8.16.65.2	Test Requirements	2827

A.8.16.66	5 DL FDD-TDD with PCell in TDD CA Event Triggered Reporting with 4 Deactivated SCells in Non-DRX	2827

A.8.16.66.1	Test Purpose and Environment	2827

A.8.16.66.2	Test Requirements	2832

A.8.16.67	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of Unknown SCell in non-DRX	2832

A.8.16.67.1	Test Purpose and Environment	2832

A.8.16.67.2	Test Requirements	2837

A.8.16.68	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of Unknown SCell in non-DRX	2837

A.8.16.68.1	Test Purpose and Environment	2837

A.8.16.68.2	Test Requirements	2842

A.8.16.69	5 DL FDD CA activation and deactivation of unknown SCell in non-DRX	2842

A.8.16.69.1	Test Purpose and Environment	2842

A.8.16.69.2	Test Requirements	2846

A.8.16.70	5 DL TDD CA activation and deactivation of unknown SCell in non-DRX	2846

A.8.16.70.1	Test Purpose and Environment	2846

A.8.16.70.2	Test Requirements	2850

A.8.16.71	5 DL FDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2850

A.8.16.71.1	Test Purpose and Environment	2850

A.8.16.71.2	Test Requirements	2855

A.8.16.72	5 DL TDD CA Event Triggered Reporting with Deactivated SCells in Non-DRX	2855

A.8.16.72.1	Test Purpose and Environment	2855

A.8.16.72.2	Test Requirements	2860

A.8.16.73	5 DL FDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2860

A.8.16.73.1	Test Purpose and Environment	2860

A.8.16.73.2	Test Requirements	2867

A.8.16.74	5 DL TDD CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2867

A.8.16.74.1	Test Purpose and Environment	2867

A.8.16.74.2	Test Requirements	2873

A.8.16.75	5 DL FDD-TDD with PCell in FDD CA activation and deactivation of known SCell in non-DRX	2873

A.8.16.75.1	Test Purpose and Environment	2873

A.8.16.75.2	Test Requirements	2879

A.8.16.76	5 DL FDD-TDD with PCell in TDD CA activation and deactivation of known SCell in non-DRX	2879

A.8.16.76.1	Test Purpose and Environment	2879

A.8.16.76.2	Test Requirements	2885

A.8.16.77	5 DL FDD CA activation and deactivation of know SCell in non-DRX	2885

A.8.16.77.1	Test Purpose and Environment	2885

A.8.16.77.2	Test Requirements	2889

A.8.16.78	5 DL TDD CA activation and deactivation of know SCell in non-DRX	2889

A.8.16.78.1	Test Purpose and Environment	2889

A.8.16.78.2	Test Requirements	2891

A.8.16.79	E-UTRAN PCell in FDD FDD-TDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2892

A.8.16.79.1	Test Purpose and Environment	2892

A.8.16.79.2	Test Requirements	2899

A.8.16.80	E-UTRAN PCell in TDD TDD-FDD 4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2899

A.8.16.80.1	Test Purpose and Environment	2899

A.8.16.80.2	Test Requirements	2906

A.8.16.81	E-UTRAN PCell in FDD FDD-TDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2906

A.8.16.81.1	Test Purpose and Environment	2906

A.8.16.81.2	Test Requirements	2912

A.8.16.82	E-UTRAN PCell in TDD TDD-FDD 5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX	2912

A.8.16.82.1	Test Purpose and Environment	2912

A.8.16.82.2	Test Requirements	2918

A.8.16.83	3 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2918

A.8.16.83.1	Test Purpose and Environment	2918

A.8.16.83.2	Test Requirements	2923

A.8.16.84	3 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2923

A.8.16.84.1	Test Purpose and Environment	2923

A.8.16.84.2	Test Requirements	2929

A.8.16.85	3 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2929

A.8.16.85.1	Test Purpose and Environment	2929

A.8.16.85.2	Test Requirements	2933

A.8.16.86	3 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2933

A.8.16.86.1	Test Purpose and Environment	2933

A.8.16.86.2	Test Requirements	2936

A.8.16.87	4 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2936

A.8.16.87.1	Test Purpose and Environment	2936

A.8.16.87.2	Test Requirements	2942

A.8.16.88	4 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2942

A.8.16.88.1	Test Purpose and Environment	2942

A.8.16.88.2	Test Requirements	2950

A.8.16.89	4 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2950

A.8.16.89.1	Test Purpose and Environment	2950

A.8.16.89.2	Test Requirements	2957

A.8.16.90	4 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2957

A.8.16.90.1	Test Purpose and Environment	2957

A.8.16.90.2	Test Requirements	2964

A.8.16.91	5 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2964

A.8.16.91.1	Test Purpose and Environment	2964

A.8.16.91.2	Test Requirements	2970

A.8.16.92	5 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	2970

A.8.16.92.1	Test Purpose and Environment	2970

A.8.16.92.2	Test Requirements	2978

A.8.16.93	5 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	2978

A.8.16.93.1	Test Purpose and Environment	2978

A.8.16.93.2	Test Requirements	2985

A.8.16.94	5 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	2985

A.8.16.94.1	Test Purpose and Environment	2985

A.8.16.94.2	Test Requirements	2992

A.8.16.95	6 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	2992

A.8.16.95.1	Test Purpose and Environment	2992

A.8.16.95.2	Test Requirements	3000

A.8.16.96	6 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	3000

A.8.16.96.1	Test Purpose and Environment	3000

A.8.16.96.2	Test Requirements	3009

A.8.16.97	6 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3009

A.8.16.97.1	Test Purpose and Environment	3009

A.8.16.97.2	Test Requirements	3016

A.8.16.98	6 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3016

A.8.16.98.1	Test Purpose and Environment	3016

A.8.16.98.2	Test Requirements	3024

A.8.16.99	7 DL CA Event Triggered Reporting under Deactivated SCells in Non-DRX with generic duplex modes	3024

A.8.16.99.1	Test Purpose and Environment	3024

A.8.16.99.2	Test Requirements	3034

A.8.16.100	7 DL CA Event Triggered Reporting on Deactivated SCell with PCell and SCell Interruptions in Non-DRX with generic duplex modes	3034

A.8.16.100.1	Test Purpose and Environment	3034

A.8.16.100.2	Test Requirements	3044

A.8.16.101	7 DL CA Activation and Deactivation of Known SCell in Non-DRX with generic duplex modes	3044

A.8.16.101.1	Test Purpose and Environment	3044

A.8.16.101.2	Test Requirements	3054

A.8.16.102	7 DL CA Activation and Deactivation of Unknown SCell in Non-DRX with generic duplex modes	3054

A.8.16.102.1	Test Purpose and Environment	3054

A.8.16.102.2	Test Requirements	3065

A.8.16.103	Hibernation and Activation of Known SCell in Non-DRX with generic duplex modes	3065

A.8.16.103.1	Test Purpose and Environment	3065

A.8.16.103.2	Test Requirements	3070

A.8.16.104	Hibernation and Activation of Unknown SCell in Non-DRX with generic duplex modes	3071

A.8.16.104.1	Test Purpose and Environment	3071

A.8.16.104.2	Test Requirements	3075

A.8.16.105	Idle Mode measurements of inter-frequency CA candidate cells for early reporting	3076

A.8.16.105.1	Test Purpose and Environment	3076

A.8.16.105.2	Test Requirements	3080

A.8.16.106	Direct Activation of Known SCell in Non-DRX with generic duplex modes	3081

A.8.16.106.1	Test Purpose and Environment	3081

A.8.16.106.2	Test Requirements	3085

A.8.16.107	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3085

A.8.16.107.1	Test Purpose and Environment	3085

A.8.16.107.2	Test Requirements	3087

A.8.16.108	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX with highSpeedEnhMeasFlag2-r16	3088

A.8.16.108.1	Test Purpose and Environment	3088

A.8.16.108.2	Test Requirements	3090

A.8.17	RSTD Measurements for E-UTRAN Carrier Aggregation	3091

A.8.17.1	E-UTRAN FDD RSTD measurement reporting delay test case	3091

A.8.17.1.1	Test Purpose and Environment	3091

A.8.17.1.2	Test Requirements	3097

A.8.17.2	E-UTRAN TDD RSTD measurement reporting delay test case	3097

A.8.17.2.1	Test Purpose and Environment	3097

A.8.17.2.2	Test Requirements	3104

A.8.17.3	E-UTRAN FDD RSTD Measurement Reporting Test Case for 20 MHz	3104

A.8.17.3.1	Test Purpose and Environment	3104

A.8.17.3.2	Test Requirements	3105

A.8.17.4	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20 MHz	3105

A.8.17.4.1	Test Purpose and Environment	3105

A.8.17.4.2	Test Requirements	3106

A.8.17.5	E-UTRAN FDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3106

A.8.17.5.1	Test Purpose and Environment	3106

A.8.17.5.2	Test Requirements	3107

A.8.17.6	E-UTRAN TDD RSTD Measurement Reporting Test Case for 10MHz+5MHz	3107

A.8.17.6.1	Test Purpose and Environment	3107

A.8.17.6.2	Test Requirements	3108

A.8.17.7	E-UTRAN FDD RSTD Measurement Reporting Test Case for 5 + 5 MHz Bandwidth	3109

A.8.17.7.1	Test Purpose and Environment	3109

A.8.17.7.2	Test Requirements	3109

A.8.17.8	E-UTRAN TDD RSTD Measurement Reporting Test Case for 5+5 MHz bandwidth	3110

A.8.17.8.1	Test Purpose and Environment	3110

A.8.17.8.2	Test Requirements	3110

A.8.17.9	E-UTRAN TDD RSTD Measurement Reporting Test Case for 20MHz+10MHz	3111

A.8.17.9.1	Test Purpose and Environment	3111

A.8.17.9.2	Test Requirements	3112

A.8.17.10	E-UTRAN 3 DL FDD CA RSTD Measurement Reporting Delay Test Case	3112

A.8.17.10.1	Test Purpose and Environment	3112

A.8.17.10.2	Test Requirements	3119

A.8.17.11	E-UTRAN 3 DL TDD CA RSTD Measurement Reporting Delay Test Case	3120

A.8.17.11.1	Test Purpose and Environment	3120

A.8.17.11.2	Test Requirements	3126

A.8.18	E-UTRAN TDD – HRPD Measurements	3127

A.8.18.1	E-UTRAN TDD-HRPD event triggered reporting under fading propagation conditions	3127

A.8.18.1.1	Test Purpose and Environment	3127

A.8.18.1.2	Test Requirements	3129

A.8.19	E-UTRAN TDD – CDMA2000 1X Measurements	3129

A.8.19.1	E-UTRAN TDD – CDMA2000 1X event triggered reporting under fading propagation conditions	3129

A.8.19.1.1	Test Purpose and Environment	3129

A.8.19.1.2	Test Requirements	3130

A.8.20	Inter-frequency/RAT Measurements in CA mode	3131

A.8.20.1	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under fading propagation conditions in asynchronous cells	3131

A.8.20.1.1	Test Purpose and Environment	3131

A.8.20.1.2	Test Requirements	3132

A.8.20.2	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3132

A.8.20.2.1	Test Purpose and Environment	3133

A.8.20.2.2	Test Requirements	3134

A.8.20.2A	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +20 MHz bandwidth.	3135

A.8.20.2A.1	Test Purpose and Environment	3135

A.8.20.2A.2	Test Requirements	3135

A.8.20.2B	E-UTRAN TDD-TDD Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells for 20 MHz +10 MHz bandwidth.	3135

A.8.20.2B.1	Test Purpose and Environment	3135

A.8.20.2B.2	Test Requirements	3138

A.8.20.3	E-UTRAN FDD - UTRAN FDD event triggered reporting under fading propagation conditions	3138

A.8.20.3.1	Test Purpose and Environment	3138

A.8.20.3.2	Test Requirements	3140

A.8.20.4	E-UTRAN TDD to UTRAN TDD cell search under fading propagation conditions	3140

A.8.20.4.1	Test Purpose and Environment	3140

A.8.20.4.1.1	1.28 Mcps TDD option	3140

A.8.20.4.2	Test Requirements	3142

A.8.20.4.2.1	1.28 Mcps TDD option	3142

A.8.20.4A	E-UTRAN TDD with 20 MHz +20 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3142

A.8.20.4A.1	Test Purpose and Environment	3142

A.8.20.4A.1.1	1.28 Mcps TDD option	3142

A.8.20.4A.2	Test Requirements	3142

A.8.20.4A.2.1	1.28 Mcps TDD option	3142

A.8.20.4B	E-UTRAN TDD with 20 MHz +10 MHz bandwidth to UTRAN TDD cell search under fading propagation conditions	3143

A.8.20.4B.1	Test Purpose and Environment	3143

A.8.20.4B.1.1	1.28 Mcps TDD option	3143

A.8.20.4B.2	Test Requirements	3145

A.8.20.4B.2.1	1.28 Mcps TDD option	3145

A.8.21	CSG Proximity Indication Testing Case for E-UTRAN FDD – FDD Inter frequency	3145

A.8.21.1	Test Purpose and Environment	3145

A.8.21.2 Test Requirements	3149

A.8.22	E-UTRAN Discovery Signal Measurements	3149

A.8.22.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3149

A.8.22.1.1	Test Purpose and Environment	3149

A.8.22.1.2	Test Requirements	3152

A.8.22.2	E-UTRAN TDD-TDD intra-frequency event triggered reporting under fading propagation conditions in synchronous cells in DRX based on CRS based discovery signal	3152

A.8.22.2.1	Test Purpose and Environment	3152

A.8.22.2.2	Test Requirements	3155

A.8.22.3	E-UTRAN FDD-FDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3155

A.8.22.3.1	Test Purpose and Environment	3155

A.8.22.3.2	Test Requirements	3158

A.8.22.4	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation conditions in DRX based on CRS based discovery signal	3158

A.8.22.4.1	Test Purpose and Environment	3158

A.8.22.4.2	Test Requirements	3161

A.8.22.5	E-UTRAN FDD-FDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3161

A.8.22.5.1	Test Purpose and Environment	3161

A.8.22.5.2	Test Requirements	3165

A.8.22.6	E-UTRAN TDD-TDD intra-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3165

A.8.22.6.1	Test Purpose and Environment	3165

A.8.22.6.2	Test Requirements	3169

A.8.22.7	E-UTRAN FDD-FDD Inter-frequency event triggered reporting in DRX based on CSI-RS based discovery signal	3169

A.8.22.7.1	Test Purpose and Environment	3169

A.8.22.7.2	Test Requirements	3173

A.8.22.8	E-UTRAN TDD-TDD inter-frequency event triggered reporting under fading propagation condition in DRX based on CSI-RS based discovery signal	3173

A.8.22.8.1	Test Purpose and Environment	3173

A.8.22.8.2	Test Requirements	3177

A.8.22.9	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3177

A.8.22.9.1	Test Purpose and Environment	3177

A.8.22.9.2	Test Requirements	3179

A.8.22.10	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CRS based discovery signal	3180

A.8.22.10.1	Test Purpose and Environment	3180

A.8.22.10.2	Test Requirements	3182

A.8.22.11	E-UTRAN FDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3183

A.8.22.11.1	Test Purpose and Environment	3183

A.8.22.11.2	Test Requirements	3186

A.8.22.12	E-UTRAN TDD event triggered reporting under deactivated SCell in non-DRX based on CSI-RS based discovery signal	3186

A.8.22.12.1	Test Purpose and Environment	3186

A.8.22.12.2	Test Requirements	3190

A.8.23	E-UTRAN Dual Connectivity Measurements	3190

A.8.23.1	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3190

A.8.23.1.1	Test Purpose and Environment	3190

A.8.23.1.2	Test Requirements	3193

A.8.23.2	E-UTRAN FDD-FDD DC intra-frequency event triggered reporting with DRX in asynchronous DC	3193

A.8.23.2.1	Test Purpose and Environment	3193

A.8.23.2.2	Test Requirements	3196

A.8.23.3	E-UTRAN TDD-TDD DC intra-frequency event triggered reporting with DRX in synchronous DC	3196

A.8.23.3.1	Test Purpose and Environment	3196

A.8.23.3.2	Test Requirements	3199

A.8.23.4	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3199

A.8.23.4.1	Test Purpose and Environment	3199

A.8.23.4.2	Test Requirements	3202

A.8.23.5	E-UTRAN FDD-FDD DC inter-frequency event triggered reporting with DRX in asynchronous DC	3202

A.8.23.5.1	Test Purpose and Environment	3202

A.8.23.5.2	Test Requirements	3205

A.8.23.6	E-UTRAN TDD-TDD DC inter-frequency event triggered reporting with DRX in synchronous DC	3205

A.8.23.6.1	Test Purpose and Environment	3205

A.8.23.6.2	Test Requirements	3208

A.8.23.7	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Synchronous DC	3208

A.8.23.7.1	Test Purpose and Environment	3208

A.8.23.7.2	Test Requirements	3210

A.8.23.8	E-UTRAN FDD-FDD Addition and Release Delay of known PSCell in Asynchronous DC	3211

A.8.23.8.1	Test Purpose and Environment	3211

A.8.23.8.2	Test Requirements	3213

A.8.23.9	E-UTRAN TDD Addition and Release Delay of known PSCell in Synchronous DC	3214

A.8.23.9.1	Test Purpose and Environment	3214

A.8.23.9.2	Test Requirements	3217

A.8.23.10	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3217

A.8.23.10.1	Test Purpose and Environment	3217

A.8.23.10.2	Test Requirements	3220

A.8.23.11	E-UTRAN TDD-FDD DC intra-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3220

A.8.23.11.1	Test Purpose and Environment	3220

A.8.23.11.2	Test Requirements	3223

A.8.23.12	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in FDD	3223

A.8.23.12.1	Test Purpose and Environment	3223

A.8.23.12.2	Test Requirements	3226

A.8.23.13	E-UTRAN TDD-FDD DC inter-frequency event triggered reporting with DRX in synchronous DC with PCell in TDD	3226

A.8.23.13.1	Test Purpose and Environment	3226

A.8.23.13.2	Test Requirements	3229

A.8.23.14	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in FDD	3229

A.8.23.14.1	Test Purpose and Environment	3229

A.8.23.14.2	Test Requirements	3232

A.8.23.15	E-UTRAN TDD-FDD Addition and Release Delay of known PSCell in Synchronous DC with PCell in TDD	3232

A.8.23.15.1	Test Purpose and Environment	3232

A.8.23.15.2	Test Requirements	3235

A.8.23.16	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with no DRX in asynchronous DC	3235

A.8.23.16.1	Test Purpose and Environment	3235

A.8.23.16.2	Test Requirements	3236

A.8.23.17	E-UTRAN FDD-FDD DC SSTD measurement reporting delay with DRX in asynchronous DC	3237

A.8.23.17.1	Test Purpose and Environment	3237

A.8.23.17.2	Test Requirements	3239

A.8.23.18	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3239

A.8.23.18.1	Test Purpose and Environment	3239

A.8.23.18.2	Test Requirements	3240

A.8.23.19	E-UTRAN FDD - FDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3241

A.8.23.19.1	Test Purpose and Environment	3241

A.8.23.19.2	Test Requirements	3242

A.8.23.20	E-UTRAN TDD - TDD DC Intra-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3243

A.8.23.20.1	Test Purpose and Environment	3243

A.8.23.20.2	Test Requirements	3244

A.8.23.21	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3245

A.8.23.21.1	Test Purpose and Environment	3245

A.8.23.21.2	Test Requirements	3246

A.8.23.22	E-UTRAN FDD - FDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in asynchronous DC	3247

A.8.23.22.1	Test Purpose and Environment	3247

A.8.23.22.2	Test Requirements	3248

A.8.23.23	E-UTRAN TDD - TDD DC Inter-frequency identification of a new CGI of E-UTRA cell using autonomous gaps in synchronous DC	3249

A.8.23.23.1	Test Purpose and Environment	3249

A.8.23.23.2	Test Requirements	3250

A.8.23.24	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3251

A.8.23.24.1	Test Purpose and Environment	3251

A.8.23.24.2	Test Requirements	3253

A.8.23.25	E-UTRAN FDD-FDD DC activation and deactivation of known SCell in Non-DRX in asynchronous DC	3254

A.8.23.25.1	Test Purpose and Environment	3254

A.8.23.25.2	Test Requirements	3256

A.8.23.26	E-UTRAN TDD-TDD DC activation and deactivation of known SCell in Non-DRX in synchronous DC	3257

A.8.23.26.1	Test Purpose and Environment	3257

A.8.23.26.2	Test Requirements	3259

A.8.23.27	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3260

A.8.23.27.1	Test Purpose and Environment	3260

A.8.23.27.2	Test Requirements	3263

A.8.23.28	E-UTRAN FDD-FDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in asynchronous DC	3263

A.8.23.28.1	Test Purpose and Environment	3263

A.8.23.28.2	Test Requirements	3267

A.8.23.29	E-UTRAN TDD-TDD DC event triggered reporting under deactivated SCell with PCell and PSCell interruption in non-DRX in synchronous DC	3267

A.8.23.29.1	Test Purpose and Environment	3267

A.8.23.29.2	Test Requirements	3271

A.8.24	Proximity-based Services	3271

A.8.24.1	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3271

A.8.24.1.1	Test Purpose and Environment	3271

A.8.24.1.2	Test Requirements	3272

A.8.24.2	E-UTRAN TDD - Initiation/Cease of SLSS Transmission with ProSe Direct Discovery	3273

A.8.24.2.1	Test Purpose and Environment	3273

A.8.24.2.2	Test Requirements	3274

A.8.24.3	E-UTRAN FDD - Initiation/Cease of SLSS Transmission with ProSe Direct Communication	3274

A.8.24.3.1	Test Purpose and Environment	3275

A.8.24.3.2	Test Requirements	3276

A.8.25	E-UTRAN-WLAN Measurements	3277

A.8.25.1	E-UTRAN FDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3277

A.8.25.1.1	Test Purpose and Environment	3277

A.8.25.1.2	Test Requirements	3279

A.8.25.2	E-UTRAN TDD-WLAN Event Triggered Reporting in non-DRX under AWGN	3279

A.8.25.2.1	Test Purpose and Environment	3279

A.8.25.2.2	Test Requirements	3281

A.8.26 Frame Structure 3 (FS3)	3281

A.8.26.1	E-UTRAN FDD-FS3 Activation and deactivation of known FS3 SCell with FDD PCell in non-DRX	3281

A.8.26.1.1	Test Purpose and Environment	3281

A.8.26.1.2	Test Requirements	3283

A.8.26.2	E-UTRAN TDD-FS3 Activation and deactivation of known FS3 SCell with TDD PCell in non-DRX	3284

A.8.26.2.1	Test Purpose and Environment	3284

A.8.26.2.2	Test Requirements	3286

A.8.26.3	E-UTRAN FDD-FS3 Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3287

A.8.26.3.1	Test Purpose and Environment	3287

A.8.26.3.2	Test Requirements	3290

A.8.26.3A	E-UTRAN FDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3290

A.8.26.3A.1	Test Purpose and Environment	3290

A.8.26.3A.2	Test Requirements	3293

A.8.26.4	E-UTRAN TDD-FS3 Event triggered reporting on deactivated FS3 SCell and TDD PCell interruption in non-DRX	3293

A.8.26.4.1	Test Purpose and Environment	3293

A.8.26.4.2	Test Requirements	3297

A.8.26.4A	E-UTRAN TDD-TDD 3DL Event triggered reporting on deactivated FS3 SCell and FDD PCell interruption in non-DRX	3297

A.8.26.4A.1	Test Purpose and Environment	3297

A.8.26.4A.2	Test Requirements	3301

A.8.26.5	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3301

A.8.26.5.1	Test Purpose and Environment	3301

A.8.26.5.2	Test Requirements	3304

A.8.26.5A	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3304

A.8.26.5A.1	Test Purpose and Environment	3304

A.8.26.5A.2	Test Requirements	3308

A.8.26.6	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal	3308

A.8.26.6.1	Test Purpose and Environment	3308

A.8.26.6.2	Test Requirements	3312

A.8.26.6A	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in non-DRX for CRS based discovery signal with 2 SCells	3312

A.8.26.6A.1	Test Purpose and Environment	3312

A.8.26.6A.2	Test Requirements	3316

A.8.26.7	E-UTRAN FDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3316

A.8.26.7.1	Test Purpose and Environment	3316

A.8.26.7.2	Test Requirements	3319

A.8.26.8	E-UTRAN TDD-FS3 Intra-frequency event triggered reporting in DRX for CRS based discovery signal	3319

A.8.26.8.1	Test Purpose and Environment	3319

A.8.26.8.2	Test Requirements	3323

A.8.26.9	E-UTRAN FDD-FS3 Inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3323

A.8.26.9.1	Test Purpose and Environment	3323

A.8.26.9.2	Test Requirements	3326

A.8.26.10	E-UTRAN TDD-FS3 inter-frequency event triggered reporting under fading propagation conditions in synchronous cells	3326

A.8.26.10.1	Test Purpose and Environment	3326

A.8.26.10.2	Test Requirements	3329

A.9	Measurement Performance Requirements	3331

A.9.1	RSRP	3331

A.9.1.1	FDD Intra frequency case	3331

A.9.1.1.1	Test Purpose and Environment	3331

A.9.1.1.2	Test parameters	3331

A.9.1.1.3	Test Requirements	3334

A.9.1.2	TDD Intra frequency case	3334

A.9.1.2.1	Test Purpose and Environment	3334

A.9.1.2.2	Test parameters	3334

A.9.1.2.3	Test Requirements	3335

A.9.1.3	FDD—FDD Inter frequency case	3336

A.9.1.3.1	Test Purpose and Environment	3336

A.9.1.3.2	Test parameters	3336

A.9.1.3.3	Test Requirements	3339

A.9.1.4	TDD—TDD Inter frequency case	3339

A.9.1.4.1	Test Purpose and Environment	3339

A.9.1.4.2	Test parameters	3339

A.9.1.4.3	Test Requirements	3341

A.9.1.5	FDD—TDD Inter frequency case	3342

A.9.1.5.1	Test Purpose and Environment	3342

A.9.1.5.2	Test parameters	3342

A.9.1.5.3	Test Requirements	3343

A.9.1.6	FDD RSRP for E-UTRAN Carrier Aggregation	3344

A.9.1.6.1	Test Purpose and Environment	3344

A.9.1.6.2	Test parameters	3344

A.9.1.6.3	Test Requirements	3347

A.9.1.7	TDD RSRP for E-UTRAN Carrier Aggregation	3347

A.9.1.7.1	Test Purpose and Environment	3347

A.9.1.7.2	Test parameters	3347

A.9.1.7.3	Test Requirements	3350

A.9.1.8	FDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3350

A.9.1.8.1	Test Purpose and Environment	3350

A.9.1.8.2	Test parameters	3350

A.9.1.8.3	Test Requirements	3354

A.9.1.9	TDD RSRP under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3354

A.9.1.9.1	Test Purpose and Environment	3354

A.9.1.9.2	Test parameters	3354

A.9.1.9.3	Test Requirements	3356

A.9.1.10	FDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3357

A.9.1.10.1	Test Purpose and Environment	3357

A.9.1.10.2	Test parameters	3357

A.9.1.10.3	Test Requirements	3360

A.9.1.11	TDD RSRP under Time-Domain Measurement Resource Restriction with MBSFN ABS	3360

A.9.1.11.1	Test Purpose and Environment	3360

A.9.1.11.2	Test parameters	3360

A.9.1.11.3	Test Requirements	3364

A.9.1.12	FDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3364

A.9.1.12.1	Test Purpose and Environment	3364

A.9.1.12.2	Test parameters	3364

A.9.1.12.3	Test Requirements	3365

A.9.1.13	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz	3365

A.9.1.13.1	Test Purpose and Environment	3365

A.9.1.13.2	Test parameters	3365

A.9.1.13.3	Test Requirements	3366

A.9.1.14	FDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3366

A.9.1.14.1	Test Purpose and Environment	3366

A.9.1.14.2	Test parameters	3366

A.9.1.14.3	Test Requirements	3370

A.9.1.15	TDD RSRP under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3370

A.9.1.15.1	Test Purpose and Environment	3370

A.9.1.15.2	Test parameters	3370

A.9.1.15.3	Test Requirements	3372

A.9.1.16	FDD Intra frequency case for 5MHz Bandwidth	3373

A.9.1.16.1	Test Purpose and Environment	3373

A.9.1.16.2	Test parameters	3373

A.9.1.16.3	Test Requirements	3374

A.9.1.17	FDD—FDD Inter frequency case for 5MHz Bandwidth	3374

A.9.1.17.1	Test Purpose and Environment	3374

A.9.1.17.2	Test parameters	3375

A.9.1.17.3	Test Requirements	3376

A.9.1.18	FDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3376

A.9.1.18.1	Test Purpose and Environment	3376

A.9.1.18.2	Test parameters	3376

A.9.1.18.3	Test Requirements	3377

A.9.1.19	TDD RSRP for E-UTRAN Carrier Aggregation for 10MHz + 5MHz	3378

A.9.1.19.1	Test Purpose and Environment	3378

A.9.1.19.2	Test parameters	3378

A.9.1.19.3	Test Requirements	3378

A.9.1.20	FDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3378

A.9.1.20.1	Test Purpose and Environment	3378

A.9.1.20.2	Test parameters	3378

A.9.1.20.3	Test Requirements	3380

A.9.1.21	TDD RSRP for E-UTRAN Carrier Aggregation for 5MHz + 5MHz bandwidth	3380

A.9.1.21.1	Test Purpose and Environment	3380

A.9.1.21.2	Test parameters	3380

A.9.1.21.3	Test Requirements	3380

A.9.1.22	RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3380

A.9.1.22.1	Test Purpose and Environment	3380

A.9.1.22.2	Test parameters	3381

A.9.1.22.3	Test Requirements	3385

A.9.1.23 RSRP for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3385

A.9.1.23.1	Test Purpose and Environment	3385

A.9.1.23.2	Test parameters	3385

A.9.1.23.3	Test Requirements	3389

A.9.1.24	TDD RSRP for E-UTRAN Carrier Aggregation for 20MHz + 10MHz	3389

A.9.1.24.1	Test Purpose and Environment	3389

A.9.1.24.2	Test parameters	3389

A.9.1.24.3	Test Requirements	3390

A.9.1.25	FDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3390

A.9.1.25.1	Test Purpose and Environment	3390

A.9.1.25.2	Test parameters	3390

A.9.1.25.3	Test Requirements	3393

A.9.1.26	TDD intra-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3393

A.9.1.26.1	Test Purpose and Environment	3393

A.9.1.26.2	Test parameters	3393

A.9.1.26.3	Test Requirements	3396

A.9.1.27	FDD—FDD inter-frequency absolute and relative RSRP accuracies in CRS based discovery signal	3396

A.9.1.27.1	Test Purpose and Environment	3396

A.9.1.27.2	Test parameters	3396

A.9.1.27.3	Test Requirements	3399

A.9.1.28	TDD—TDD inter-frequency absolute and relative  RSRP accuracies in CRS based discovery signal	3399

A.9.1.28.1	Test Purpose and Environment	3399

A.9.1.28.2	Test parameters	3399

A.9.1.28.3	Test Requirements	3402

A.9.1.29	FDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3402

A.9.1.29.1	Test Purpose and Environment	3402

A.9.1.29.2	Test parameters	3402

A.9.1.29.3	Test Requirements	3405

A.9.1.30	TDD intra frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3405

A.9.1.30.1	Test Purpose and Environment	3405

A.9.1.30.2	Test parameters	3406

A.9.1.30.3	Test Requirements	3409

A.9.1.31	FDD—FDD inter-frequency absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal	3409

A.9.1.31.1	Test Purpose and Environment	3409

A.9.1.31.2	Test parameters	3409

A.9.1.31.3	Test Requirements	3412

A.9.1.32	TDD—TDD inter-frequency absolute and relative  CSI-RSRP accuracies in CSI-RS based discovery signal	3413

A.9.1.32.1	Test Purpose and Environment	3413

A.9.1.32.2	Test parameters	3413

A.9.1.32.3	Test Requirements	3416

A.9.1.33	FDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3416

A.9.1.33.1	Test Purpose and Environment	3416

A.9.1.33.2	Test parameters	3416

A.9.1.33.3	Test Requirements	3419

A.9.1.34	TDD absolute and relative RSRP accuracies for E-UTRAN Carrier Aggregation in CRS based discovery signal	3419

A.9.1.34.1	Test Purpose and Environment	3419

A.9.1.34.2	Test parameters	3419

A.9.1.34.3	Test Requirements	3422

A.9.1.35	FDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3422

A.9.1.35.1	Test Purpose and Environment	3422

A.9.1.35.2	Test parameters	3422

A.9.1.35.3	Test Requirements	3425

A.9.1.36	TDD absolute and relative CSI-RSRP accuracies for E-UTRAN Carrier Aggregation in CSI-RS based discovery signal	3426

A.9.1.36.1	Test Purpose and Environment	3426

A.9.1.36.2	Test parameters	3426

A.9.1.36.3	Test Requirements	3429

A.9.1.37	3 DL PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3429

A.9.1.37.1	Test Purpose and Environment	3429

A.9.1.37.2	Test parameters	3429

A.9.1.37.3	Test Requirements	3432

A.9.1.38	3 DL PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3433

A.9.1.38.1	Test Purpose and Environment	3433

A.9.1.38.2	Test parameters	3433

A.9.1.38.3	Test Requirements	3436

A.9.1.39	3 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3437

A.9.1.39.1	Test Purpose and Environment	3437

A.9.1.39.2	Test parameters	3437

A.9.1.39.3	Test Requirements	3443

A.9.1.40	3 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3443

A.9.1.40.1	Test Purpose and Environment	3443

A.9.1.40.2	Test parameters	3444

A.9.1.40.3	Test Requirements	3450

A.9.1.41	FD-FDD RSRP Intra frequency case for UE category 0	3450

A.9.1.41.1	Test Purpose and Environment	3450

A.9.1.41.2	Test parameters	3450

A.9.1.41.3	Test Requirements	3453

A.9.1.42	HD-FDD RSRP Intra frequency case for UE category 0	3453

A.9.1.42.1	Test Purpose and Environment	3453

A.9.1.42.2	Test parameters	3453

A.9.1.42.3	Test Requirements	3456

A.9.1.43	TDD RSRP Intra frequency case for UE category 0	3456

A.9.1.43.1	Test Purpose and Environment	3456

A.9.1.43.2	Test parameters	3456

A.9.1.43.3	Test Requirements	3457

A.9.1.44	4 DL CA PCell in FDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3458

A.9.1.44.1	Test Purpose and Environment	3458

A.9.1.44.2	Test parameters	3458

A.9.1.44.3	Test Requirements	3463

A.9.1.45	4 DL CA PCell in TDD FDD-TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.45.1	Test Purpose and Environment	3463

A.9.1.45.2	Test parameters	3463

A.9.1.45.3	Test Requirements	3432

A.9.1.46	4 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3432

A.9.1.46.1	Test Purpose and Environment	3432

A.9.1.46.2	Test parameters	3432

A.9.1.46.3	Test Requirements	3441

A.9.1.47	4 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3442

A.9.1.47.1	Test Purpose and Environment	3442

A.9.1.47.2	Test parameters	3442

A.9.1.47.3	Test Requirements	3451

A.9.1.48	5 DL FDD-TDD with PCell in FDD RSRP for E-UTRAN in Carrier Aggregation	3451

A.9.1.48.1	Test Purpose and Environment	3451

A.9.1.48.2	Test parameters	3452

A.9.1.48.3	Test Requirements	3462

A.9.1.49	5 DL FDD-TDD with PCell in TDD RSRP for E-UTRAN in Carrier Aggregation	3463

A.9.1.49.1	Test Purpose and Environment	3463

A.9.1.49.2	Test parameters	3463

A.9.1.49.3	Test Requirements	3473

A.9.1.50	5 DL FDD RSRP for E-UTRAN in Carrier Aggregation	3474

A.9.1.50.1	Test Purpose and Environment	3474

A.9.1.50.2	Test parameters	3474

A.9.1.50.3	Test Requirements	3481

A.9.1.51	5 DL TDD RSRP for E-UTRAN in Carrier Aggregation	3482

A.9.1.51.1	Test Purpose and Environment	3482

A.9.1.51.2	Test parameters	3482

A.9.1.51.3	Test Requirements	3488

A.9.1.52	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3489

A.9.1.52.1	Test Purpose and Environment	3489

A.9.1.52.2	Test parameters	3489

A.9.1.52.3	Test Requirements	3492

A.9.1.52A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3492

A.9.1.52A.1	Test Purpose and Environment	3492

A.9.1.52A.2	Test parameters	3492

A.9.1.52A.3	Test Requirements	3493

A.9.1.53	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3493

A.9.1.53.1	Test Purpose and Environment	3493

A.9.1.53.2	Test parameters	3494

A.9.1.53.3	Test Requirements	3497

A.9.1.53A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeA	3497

A.9.1.53A.1	Test Purpose and Environment	3497

A.9.1.53A.2	Test parameters	3497

A.9.1.53A.3	Test Requirements	3498

A.9.1.54	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	3499

A.9.1.54.1	Test Purpose and Environment	3499

A.9.1.54.2	Test parameters	3499

A.9.1.54.3	Test Requirements	3501

A.9.1.55	FS3 Intra frequency absolute and relative RSRP accuracies with FDD PCell	3501

A.9.1.55.1	Test Purpose and Environment	3501

A.9.1.55.2	Test parameters	3501

A.9.1.55.3	Test Requirements	3505

A.9.1.56	FS3 Intra frequency absolute and relative RSRP accuracies with TDD PCell	3505

A.9.1.56.1	Test Purpose and Environment	3505

A.9.1.56.2	Test parameters	3505

A.9.1.56.3	Test Requirements	3508

A.9.1.57	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3509

A.9.1.57.1	Test Purpose and Environment	3509

A.9.1.57.2	Test parameters	3509

A.9.1.57.3	Test Requirements	3512

A.9.1.57A	FD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3512

A.9.1.57A.1	Test Purpose and Environment	3512

A.9.1.57A.2	Test parameters	3512

A.9.1.57A.3	Test Requirements	3513

A.9.1.58	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3513

A.9.1.58.1	Test Purpose and Environment	3513

A.9.1.58.2	Test parameters	3514

A.9.1.58.3	Test Requirements	3517

A.9.1.58A	HD-FDD RSRP Intra frequency case for Cat-M1 UE for 5MHz Bandwidth in CEModeB	3517

A.9.1.58A.1	Test Purpose and Environment	3517

A.9.1.58A.2	Test parameters	3517

A.9.1.58A.3	Test Requirements	3518

A.9.1.59	TDD RSRP Intra frequency case for Cat-M1 UE in CEModeB	3518

A.9.1.59.1	Test Purpose and Environment	3518

A.9.1.59.2	Test parameters	3519

A.9.1.59.3	Test Requirements	3521

A.9.1.60	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with FDD PCell	3521

A.9.1.60.1	Test Purpose and Environment	3521

A.9.1.60.2	Test parameters	3521

A.9.1.60.3	Test Requirements	3524

A.9.1.61	FS3 Absolute and relative CSI-RSRP accuracies in CSI-RS based discovery signal with TDD PCell	3525

A.9.1.61.1	Test Purpose and Environment	3525

A.9.1.61.2	Test parameters	3525

A.9.1.61.3	Test Requirements	3528

A.9.1.62	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3528

A.9.1.62.1	Test Purpose and Environment	3528

A.9.1.62.2	Test parameters	3528

A.9.1.62.3	Test Requirements	3531

A.9.1.63	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3531

A.9.1.53.1	Test Purpose and Environment	3531

A.9.1.53.2	Test parameters	3531

A.9.1.63.3	Test Requirements	3534

A.9.1.64	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	3534

A.9.1.64.1	Test Purpose and Environment	3534

A.9.1.64.2	Test parameters	3534

A.9.1.64.3	Test Requirements	3535

A.9.1.65	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3536

A.9.1.65.1	Test Purpose and Environment	3536

A.9.1.65.2	Test parameters	3536

A.9.1.65.3	Test Requirements	3542

A.9.1.66	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3542

A.9.1.66.1	Test Purpose and Environment	3542

A.9.1.66.2	Test parameters	3542

A.9.1.66.3	Test Requirements	3548

A.9.1.67	TDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	3548

A.9.1.67.1	Test Purpose and Environment	3548

A.9.1.67.2	Test parameters	3548

A.9.1.67.3	Test Requirements	3549

A.9.1.68	3 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3550

A.9.1.68.1	Test Purpose and Environment	3550

A.9.1.68.2	Test parameters	3550

A.9.1.68.3	Test Requirements	3554

A.9.1.69	4 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3554

A.9.1.69.1	Test Purpose and Environment	3554

A.9.1.69.2	Test parameters	3554

A.9.1.69.3	Test Requirements	3558

A.9.1.70	5 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3559

A.9.1.70.1	Test Purpose and Environment	3559

A.9.1.70.2	Test parameters	3559

A.9.1.70.3	Test Requirements	3571

A.9.1.71	6 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3572

A.9.1.71.1	Test Purpose and Environment	3572

A.9.1.71.2	Test parameters	3572

A.9.1.71.3	Test Requirements	3584

A.9.1.72	7 DL RSRP for E-UTRAN in Carrier Aggregation with generic duplex modes	3585

A.9.1.72.1	Test Purpose and Environment	3585

A.9.1.72.2	Test parameters	3585

A.9.1.72.3	Test Requirements	3597

A.9.1.73	FDD Intra frequency case for CA Idle Mode Measurements	3598

A.9.1.73.1	Test Purpose and Environment	3598

A.9.1.73.2	Test parameters	3598

A.9.1.73.3	Test Requirements	3601

A.9.1.74	FDD—FDD Inter frequency case for CA Idle Mode Measurements for overlapping carrier	3601

A.9.1.74.1	Test Purpose and Environment	3601

A.9.1.74.2	Test parameters	3601

A.9.1.74.3	Test Requirements	3602

A.9.1.75	FDD—FDD Inter frequency case for CA Idle Mode Measurements for non-overlapping carrier	3602

A.9.1.75.1	Test Purpose and Environment	3602

A.9.1.75.2	Test parameters	3603

A.9.1.75.3	Test Requirements	3604

A.9.1.76	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3604

A.9.1.76.1	Test Purpose and Environment	3604

A.9.1.76.2	Test parameters	3604

A.9.1.76.3	Test Requirements	3607

A.9.1.77	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3607

A.9.1.77.1	Test Purpose and Environment	3607

A.9.1.77.2	Test parameters	3607

A.9.1.77.3	Test Requirements	3610

A.9.1.78	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeA	3610

A.9.1.78.1	Test Purpose and Environment	3610

A.9.1.78.2	Test parameters	3610

A.9.1.78.3	Test Requirements	3613

A.9.1.79	FD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3613

A.9.1.79.1	Test Purpose and Environment	3613

A.9.1.79.2	Test parameters	3613

A.9.1.79.3	Test Requirements	3616

A.9.1.80	HD-FDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3616

A.9.1.80.1	Test Purpose and Environment	3616

A.9.1.80.2	Test parameters	3616

A.9.1.80.3	Test Requirements	3619

A.9.1.81	TDD RSS based RSRP Intra frequency case for Cat-M1 UE in CEModeB	3619

A.9.1.81.1	Test Purpose and Environment	3619

A.9.1.78.2	Test parameters	3619

A.9.1.81.3	Test Requirements	3622

A.9.2	RSRQ	3622

A.9.2.1	FDD Intra frequency case	3622

A.9.2.1.1	Test Purpose and Environment	3622

A.9.2.1.2	Test parameters	3622

A.9.2.1.3	Test Requirements	3625

A.9.2.2	TDD Intra frequency case	3625

A.9.2.2.1	Test Purpose and Environment	3625

A.9.2.2.2	Test parameters	3625

A.9.2.2.3	Test Requirements	3627

A.9.2.3	FDD—FDD Inter frequency case	3627

A.9.2.3.1	Test Purpose and Environment	3627

A.9.2.3.2	Test parameters	3627

A.9.2.3.3	Test Requirements	3630

A.9.2.4	TDD—TDD Inter frequency case	3630

A.9.2.4.1	Test Purpose and Environment	3630

A.9.2.4.2	Test parameters	3630

A.9.2.4.3	Test Requirements	3632

A.9.2.4A	FDD—TDD Inter frequency case	3633

A.9.2.4A.1	Test Purpose and Environment	3633

A.9.2.4A.2	Test parameters	3633

A.9.2.4A.3	Test Requirements	3635

A.9.2.5	FDD RSRQ for E-UTRA Carrier Aggregation	3635

A.9.2.5.1	Test Purpose and Environment	3635

A.9.2.5.2	Test parameters	3636

A.9.2.5.3	Test Requirements	3639

A.9.2.6	TDD RSRQ for E-UTRA Carrier Aggregation	3639

A.9.2.6.1	Test Purpose and Environment	3639

A.9.2.6.2	Test parameters	3639

A.9.2.6.3	Test Requirements	3642

A.9.2.7	FDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3642

A.9.2.7.1	Test Purpose and Environment	3642

A.9.2.7.2	Test parameters	3642

A.9.2.7.3	Test Requirements	3646

A.9.2.8	TDD RSRQ under Time Domain Measurement Resource Restriction with Non-MBSFN ABS	3646

A.9.2.8.1	Test Purpose and Environment	3646

A.9.2.8.2	Test parameters	3646

A.9.2.8.3	Test Requirements	3650

A.9.2.9	FDD RSRQ under Time Domain Measurement Resource Restriction with MBSFN ABS	3650

A.9.2.9.1	Test Purpose and Environment	3650

A.9.2.9.2	Test parameters	3650

A.9.2.9.3	Test Requirements	3654

A.9.2.10	TDD Intra frequency case under time domain measurement resource restriction with MBSFN ABS	3654

A.9.2.10.1	Test Purpose and Environment	3654

A.9.2.10.2	Test parameters	3654

A.9.2.10.3	Test Requirements	3658

A.9.2.11	FDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3658

A.9.2.11.1	Test Purpose and Environment	3658

A.9.2.11.2	Test parameters	3658

A.9.2.11.3	Test Requirements	3659

A.9.2.12	TDD RSRQ for E-UTRA Carrier Aggregation (20MHz bandwidth)	3659

A.9.2.12.1	Test Purpose and Environment	3659

A.9.2.12.2	Test parameters	3660

A.9.2.12.3	Test Requirements	3660

A.9.2.13	Void	3661

A.9.2.13.1	Void	3661

A.9.2.13.2	Void	3661

A.9.2.13.3	Void	3661

A.9.2.14	Void	3661

A.9.2.14.1	Void	3661

A.9.2.14.2	Void	3661

A.9.2.14.3	Void	3661

A.9.2.15	FDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3661

A.9.2.15.1	Test Purpose and Environment	3661

A.9.2.15.2	Test parameters	3661

A.9.2.15.3	Test Requirements	3665

A.9.2.16	TDD RSRQ under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3665

A.9.2.16.1	Test Purpose and Environment	3665

A.9.2.16.2	Test parameters	3665

A.9.2.16.3	Test Requirements	3670

A.9.2.17	FDD Intra frequency case for 5 MHz bandwidth	3670

A.9.2.17.1	Test Purpose and Environment	3670

A.9.2.17.2	Test parameters	3670

A.9.2.17.3	Test Requirements	3672

A.9.2.18	FDD—FDD Inter frequency case for 5MHz bandwidth	3672

A.9.2.18.1	Test Purpose and Environment	3672

A.9.2.18.2	Test parameters	3672

A.9.2.18.3	Test Requirements	3675

A.9.2.19	FDD-FDD Inter Frequency WB-RSRQ	3675

A.9.2.19.1	Test Purpose and Environment	3675

A.9.2.19.2	Test parameters	3675

A.9.2.19.3	Test Requirements	3677

A.9.2.20	TDD—TDD Inter Frequency WB-RSRQ	3677

A.9.2.20.1	Test Purpose and Environment	3677

A.9.2.20.2	Test parameters	3677

A.9.2.20.3	Test Requirements	3680

A.9.2.21	FDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3680

A.9.2.21.1	Test Purpose and Environment	3680

A.9.2.21.2	Test parameters	3680

A.9.2.21.3	Test Requirements	3683

A.9.2.22	TDD RSRQ for E-UTRAN Carrier Aggregation for 10MHz+5MHz	3683

A.9.2.22.1	Test Purpose and Environment	3684

A.9.2.22.2	Test parameters	3684

A.9.2.22.3	Test Requirements	3684

A.9.2.23	FDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3684

A.9.2.23.1	Test Purpose and Environment	3684

A.9.2.23.2	Test parameters	3684

A.9.2.23.3	Test Requirements	3687

A.9.2.24	TDD RSRQ for E-UTRA Carrier Aggregation (5MHz + 5MHz bandwidth)	3687

A.9.2.24.1	Test Purpose and Environment	3687

A.9.2.24.2	Test parameters	3687

A.9.2.24.3	Test Requirements	3688

A.9.2.25	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in FDD	3688

A.9.2.25.1	Test Purpose and Environment	3688

A.9.2.25.2	Test parameters	3688

A.9.2.25.3	Test Requirements	3693

A.9.2.26	RSRQ for E-UTRAN TDD-FDD Carrier Aggregation with PCell in TDD	3693

A.9.2.26.1	Test Purpose and Environment	3693

A.9.2.26.2	Test parameters	3693

A.9.2.26.3	Test Requirements	3697

A.9.2.27	TDD RSRQ for E-UTRAN Carrier Aggregation for 20MHz+10MHz	3697

A.9.2.27.1	Test Purpose and Environment	3697

A.9.2.27.2	Test parameters	3697

A.9.2.27.3	Test Requirements	3698

A.9.2.28	FDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3698

A.9.2.28.1	Test Purpose and Environment	3698

A.9.2.28.2	Test parameters	3698

A.9.2.28.3	Test Requirements	3701

A.9.2.29	TDD intra-frequency absolute RSRQ accuracy with CRS based discovery signal	3701

A.9.2.29.1	Test Purpose and Environment	3701

A.9.2.29.2	Test parameters	3701

A.9.2.29.3	Test Requirements	3704

A.9.2.30	FDD-FDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3704

A.9.2.30.1	Test Purpose and Environment	3704

A.9.2.30.2	Test parameters	3704

A.9.2.30.3	Test Requirements	3707

A.9.2.31	TDD-TDD inter-frequency absolute and relative RSRQ accuracies with CRS based discovery signal	3707

A.9.2.31.1	Test Purpose and Environment	3707

A.9.2.31.2	Test parameters	3707

A.9.2.31.3	Test Requirements	3710

A.9.2.32	FDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3710

A.9.2.32.1	Test Purpose and Environment	3710

A.9.2.32.2	Test parameters	3710

A.9.2.32.3	Test Requirements	3713

A.9.2.33	TDD absolute and relative RSRQ accuracy for E-UTRAN Carrier Aggregation in CRS based discovery signal	3713

A.9.2.33.1	Test Purpose and Environment	3713

A.9.2.33.2	Test parameters	3713

A.9.2.33.3	Test Requirements	3717

A.9.2.34	FDD—FDD Inter frequency new RSRQ	3717

A.9.2.34.1	Test Purpose and Environment	3717

A.9.2.34.2	Test parameters	3717

A.9.2.34.3	Test Requirements	3720

A.9.2.35	TDD—TDD Inter frequency new RSRQ	3720

A.9.2.35.1	Test Purpose and Environment	3720

A.9.2.35.2	Test parameters	3720

A.9.2.35.3	Test Requirements	3723

A.9.2.36	FDD—FDD Inter frequency RSRQ measured on all OFDM symbols	3723

A.9.2.36.1	Test Purpose and Environment	3723

A.9.2.36.2	Test parameters	3723

A.9.2.36.3	Test Requirements	3724

A.9.2.37	TDD—TDD Inter frequency RSRQ measurement on all OFDM symbols	3725

A.9.2.37.1	Test Purpose and Environment	3725

A.9.2.37.2	Test parameters	3725

A.9.2.37.3	Test Requirements	3726

A.9.2.38	3 DL PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3727

A.9.2.38.1	Test Purpose and Environment	3727

A.9.2.38.2	Test parameters	3727

A.9.2.38.3	Test Requirements	3731

A.9.2.39	3 DL PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3731

A.9.2.39.1	Test Purpose and Environment	3731

A.9.2.39.2	Test parameters	3731

A.9.2.39.3	Test Requirements	3735

A.9.2.40	3 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3735

A.9.2.40.1	Test Purpose and Environment	3735

A.9.2.40.2	Test parameters	3735

A.9.2.40.3	Test Requirements	3738

A.9.2.41	3 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3739

A.9.2.41.1	Test Purpose and Environment	3739

A.9.2.41.2	Test parameters	3739

A.9.2.41.3	Test Requirements	3742

A.9.2.42	FD-FDD RSRQ Intra frequency case for UE category 0	3742

A.9.2.42.1	Test Purpose and Environment	3742

A.9.2.42.2	Test parameters	3742

A.9.2.42.3	Test Requirements	3745

A.9.2.43	HD-FDD RSRQ Intra frequency case for UE category 0	3745

A.9.2.43.1	Test Purpose and Environment	3745

A.9.2.43.2	Test parameters	3745

A.9.2.43.3	Test Requirements	3748

A.9.2.44	TDD RSRQ Intra frequency case for UE category 0	3748

A.9.2.44.1	Test Purpose and Environment	3748

A.9.2.44.2	Test parameters	3748

A.9.2.44.3	Test Requirements	3750

A.9.2.45	4 DL CA PCell in FDD FDD-TDD RSRQ for E-UTRAN in Carrier Aggregation	3750

A.9.2.45.1	Test Purpose and Environment	3750

A.9.2.45.2	Test parameters	3750

A.9.2.45.3	Test Requirements	3754

A.9.2.46	4 DL CA PCell in TDD TDD-FDD RSRQ for E-UTRAN in Carrier Aggregation	3755

A.9.2.46.1	Test Purpose and Environment	3755

A.9.2.46.2	Test parameters	3755

A.9.2.46.3	Test Requirements	3759

A.9.2.47	5 DL FDD-TDD with PCell in FDD RSRQ for E-UTRAN in Carrier Aggregation	3760

A.9.2.47.1	Test Purpose and Environment	3760

A.9.2.47.2	Test parameters	3760

A.9.2.47.3	Test Requirements	3764

A.9.2.48	5 DL FDD-TDD with PCell in TDD RSRQ for E-UTRAN in Carrier Aggregation	3764

A.9.2.48.1	Test Purpose and Environment	3764

A.9.2.48.2	Test parameters	3765

A.9.2.48.3	Test Requirements	3769

A.9.2.49	5 DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3770

A.9.2.49.1	Test Purpose and Environment	3770

A.9.2.49.2	Test parameters	3770

A.9.2.49.3	Test Requirements	3776

A.9.2.50	5 DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3777

A.9.2.50.1	Test Purpose and Environment	3777

A.9.2.50.2	Test parameters	3777

A.9.2.50.3	Test Requirements	3783

A.9.2.51	FS3 Intra frequency absolute and relative RSRQ accuracies with FDD PCell	3783

A.9.2.51.1	Test Purpose and Environment	3783

A.9.2.51.2	Test parameters	3784

A.9.2.51.3	Test Requirements	3788

A.9.2.52	FS3 Intra frequency absolute and relative RSRQ accuracies with TDD PCell	3788

A.9.2.52.1	Test Purpose and Environment	3788

A.9.2.52.2	Test parameters	3788

A.9.2.52.3	Test Requirements	3791

A.9.2.53	4DL FDD RSRQ for E-UTRAN in Carrier Aggregation	3792

A.9.2.53.1	Test Purpose and Environment	3792

A.9.2.53.2	Test parameters	3792

A.9.2.53.3	Test Requirements	3798

A.9.2.54	4DL TDD RSRQ for E-UTRAN in Carrier Aggregation	3799

A.9.2.54.1	Test Purpose and Environment	3799

A.9.2.54.2	Test parameters	3799

A.9.2.54.3	Test Requirements	3805

A.9.2.55	3 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3805

A.9.2.55.1	Test Purpose and Environment	3805

A.9.2.55.2	Test parameters	3806

A.9.2.55.3	Test Requirements	3811

A.9.2.56	4 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3811

A.9.2.56.1	Test Purpose and Environment	3811

A.9.2.56.2	Test parameters	3811

A.9.2.56.3	Test Requirements	3816

A.9.2.57	5 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3816

A.9.2.57.1	Test Purpose and Environment	3816

A.9.2.57.2	Test parameters	3817

A.9.2.57.3	Test Requirements	3821

A.9.2.58	6 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3822

A.9.2.58.1	Test Purpose and Environment	3822

A.9.2.58.2	Test parameters	3822

A.9.2.58.3	Test Requirements	3826

A.9.2.59	7 DL RSRQ for E-UTRAN in Carrier Aggregation with generic duplex modes	3827

A.9.2.59.1	Test Purpose and Environment	3827

A.9.2.59.2	Test parameters	3827

A.9.2.59.3	Test Requirements	3832

A.9.2.60	FDD Intra frequency case for CA Idle Mode Measurements	3833

A.9.2.60.1	Test Purpose and Environment	3833

A.9.2.60.2	Test parameters	3833

A.9.2.60.3	Test Requirements	3836

A.9.2.61	FDD—FDD Inter frequency case for CA Idle Mode Measurements on overlapping carrier	3836

A.9.2.61.1	Test Purpose and Environment	3836

A.9.2.61.2	Test parameters	3836

A.9.2.61.3	Test Requirements	3839

A.9.2.62	FDD—FDD Inter frequency case for CA Idle Mode Measurements on non-overlapping carrier	3839

A.9.2.62.1	Test Purpose and Environment	3839

A.9.2.62.2	Test parameters	3839

A.9.2.62.3	Test Requirements	3842

A.9.3	UTRAN FDD CPICH RSCP	3842

A.9.3.1	E-UTRAN FDD	3842

A.9.3.1.1	Test Purpose and Environment	3842

A.9.3.1.2	Parameters	3842

A.9.3.1.3	Test Requirements	3845

A.9.3.2 E-UTRAN TDD	3845

A.9.3.2.1	Test Purpose and Environment	3845

A.9.3.2.2	Parameters	3845

A.9.3.2.3	Test Requirements	3848

A.9.3.3	E-UTRAN FDD for 5MHz Bandwidth	3848

A.9.3.3.1	Test Purpose and Environment	3848

A.9.3.3.2	Parameters	3848

A.9.3.3.3	Test Requirements	3849

A.9.4	UTRAN FDD CPICH Ec/No	3850

A.9.4.1	E-UTRAN FDD	3850

A.9.4.1.1	Test Purpose and Environment	3850

A.9.4.1.2	Parameters	3850

A.9.4.1.3	Test Requirements	3852

A.9.4.2	E-UTRAN TDD	3853

A.9.4.2.1	Test Purpose and Environment	3853

A.9.4.2.2	Parameters	3853

A.9.4.2.3	Test Requirements	3855

A.9.4.3	E-UTRAN FDD for 5MHz Bandwidth	3856

A.9.4.3.1	Test Purpose and Environment	3856

A.9.4.3.2	Parameters	3856

A.9.4.3.3	Test Requirements	3857

A.9.5	UTRAN TDD measurement	3857

A.9.5.1	P-CCPCH RSCP absolute accuracy for E-UTRAN FDD	3857

A.9.5.1.1	Test Purpose and Environment	3857

A.9.5.1.2	Test parameters	3857

A.9.5.1.3	Test Requirements	3859

A.9.5.2	P-CCPCH RSCP absolute accuracy for E-UTRAN TDD	3859

A.9.5.2.1	Test Purpose and Environment	3859

A.9.5.2.2	Test parameters	3859

A.9.5.2.3	Test Requirements	3861

A.9.6	GSM Carrier RSSI	3862

A.9.6.1	E-UTRAN FDD	3862

A.9.6.1.1	Test Purpose and Environment	3862

A.9.6.1.2	Test Requirements	3863

A.9.6.2	E-UTRAN TDD	3863

A.9.6.2.1	Test Purpose and Environment	3863

A.9.6.2.2	Test Requirements	3865

A.9.7	UE Rx – Tx Time Difference	3866

A.9.7.1	E-UTRAN FDD UE Rx – Tx time difference case	3866

A.9.7.1.1	Test Purpose and Environment	3866

A.9.7.1.2	Test parameters	3866

A.9.7.1.3	Test Requirements	3867

A.9.7.2	E-UTRA TDD UE Rx – Tx time difference case	3867

A.9.7.2.1	Test Purpose and Environment	3867

A.9.7.2.2	Test parameters	3867

A.9.7.2.3	Test Requirements	3869

A.9.7.3	E-UTRAN FDD UE Rx–Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3869

A.9.7.3.1	Test Purpose and Environment	3869

A.9.7.3.2	Test parameters	3869

A.9.7.3.3	Test Requirements	3872

A.9.7.4	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with Non-MBSFN ABS	3872

A.9.7.4.1	Test Purpose and Environment	3872

A.9.7.4.2	Test Parameters	3872

A.9.7.4.3	Test Requirements	3875

A.9.7.5	E-UTRAN FDD UE Rx–Tx time difference under Time Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3875

A.9.7.5.1	Test Purpose and Environment	3875

A.9.7.5.2	Test parameters	3875

A.9.7.5.3	Test Requirements	3878

A.9.7.6	E-UTRAN TDD UE Rx-Tx Time Difference under Time-Domain Measurement Resource Restriction with CRS Assistance Information and Non-MBSFN ABS	3878

A.9.7.6.1	Test Purpose and Environment	3878

A.9.7.6.2	Test Parameters	3878

A.9.7.6.3	Test Requirements	3881

A.9.7.7	E-UTRAN FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3881

A.9.7.7.1	Test Purpose and Environment	3881

A.9.7.7.2	Test parameters	3881

A.9.7.7.3	Test Requirements	3882

A.9.7.8	E-UTRAN HD-FDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3883

A.9.7.8.1	Test Purpose and Environment	3883

A.9.7.8.2	Test parameters	3883

A.9.7.8.3	Test Requirements	3884

A.9.7.9	E-UTRAN TDD UE Rx-Tx time difference case for Cat-M1/M2 UE in CEModeA	3884

A.9.7.9.1	Test Purpose and Environment	3884

A.9.7.9.2	Test parameters	3884

A.9.7.9.3	Test Requirements	3885

A.9.8	RSTD	3886

A.9.8.1	E-UTRAN FDD RSTD intra frequency case	3886

A.9.8.1.1	Test Purpose and Environment	3886

A.9.8.1.2	Test Requirements	3888

A.9.8.1.2A	Test Requirements for UE Category 1bis	3888

A.9.8.2	E-UTRAN TDD RSTD intra frequency case	3889

A.9.8.2.1	Test Purpose and Environment	3889

A.9.8.2.2	Test Requirements	3892

A.9.8.2.2A	Test Requirements for UE Category 1bis	3892

A.9.8.3	E-UTRAN FDD-FDD RSTD inter frequency case	3893

A.9.8.3.1	Test Purpose and Environment	3893

A.9.8.3.2	Test Requirements	3895

A.9.8.3.2A	Test Requirements for UE Category 1bis	3895

A.9.8.4	E-UTRAN TDD-TDD RSTD inter frequency case	3896

A.9.8.4.1	Test Purpose and Environment	3896

A.9.8.4.2	Test Requirements	3898

A.9.8.4.2A	Test Requirements for UE Category 1bis	3898

A.9.8.5	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation	3899

A.9.8.5.1	Test Purpose and Environment	3899

A.9.8.5.2	Test Requirements	3901

A.9.8.6	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation	3901

A.9.8.6.1	Test Purpose and Environment	3901

A.9.8.6.2	Test Requirements	3904

A.9.8.7	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3904

A.9.8.7.1	Test Purpose and Environment	3904

A.9.8.7.2	Test Requirements	3905

A.9.8.8	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz bandwidth	3905

A.9.8.8.1	Test Purpose and Environment	3905

A.9.8.8.2	Test Requirements	3906

A.9.8.9	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3906

A.9.8.9.1	Test Purpose and Environment	3906

A.9.8.9.2	Test Requirements	3907

A.9.8.10	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 10MHz+5MHz	3907

A.9.8.10.1	Test Purpose and Environment	3907

A.9.8.10.2	Test Requirements	3908

A.9.8.11	E-UTRAN FDD RSTD Measurement Accuracy in Carrier Aggregation for 5 + 5MHz bandwidth	3908

A.9.8.11.1	Test Purpose and Environment	3908

A.9.8.11.2	Test Requirements	3909

A.9.8.12	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 5+5MHz bandwidth	3909

A.9.8.12.1	Test Purpose and Environment	3909

A.9.8.12.2	Test Requirements	3910

A.9.8.13	E-UTRAN TDD RSTD Measurement Accuracy in Carrier Aggregation for 20MHz+10MHz	3910

A.9.8.13.1	Test Purpose and Environment	3910

A.9.8.13.2	Test Requirements	3911

A.9.8.14	E-UTRAN FDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3911

A.9.8.14.1	Test Purpose and Environment	3911

A.9.8.14.2	Test Requirements	3918

A.9.8.15	E-UTRAN TDD RSTD Measurement Accuracy in 3DL Carrier Aggregation	3918

A.9.8.15.1	Test Purpose and Environment	3918

A.9.8.15.2	Test Requirements	3924

A.9.8.16	HD – FDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3924

A.9.8.16.1	Test Purpose and Environment	3924

A.9.8.16.2	Test Requirements	3927

A.9.8.17	HD – FDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3927

A.9.8.17.1	Test Purpose and Environment	3927

A.9.8.17.2	Test Requirements	3930

A.9.8.18	HD – FDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3930

A.9.8.18.1	Test Purpose and Environment	3930

A.9.8.18.2	Test Requirements	3933

A.9.8.19	HD – FDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3933

A.9.8.19.1	Test Purpose and Environment	3933

A.9.8.19.2	Test Requirements	3936

A.9.8.20	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode A	3936

A.9.8.20.1	Test Purpose and Environment	3936

A.9.8.20.2	Test Requirements	3939

A.9.8.21	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CEModeA	3940

A.9.8.21.1	Test Purpose and Environment	3940

A.9.8.21.2	Test Requirements	3943

A.9.8.22	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode A	3944

A.9.8.22.1	Test Purpose and Environment	3944

A.9.8.22.2	Test Requirements	3947

A.9.8.23	E-UTRAN FDD RSTD intra-frequency measurement accuracy in CE Mode B	3948

A.9.8.23.1	Test Purpose and Environment	3948

A.9.8.23.2	Test Requirements	3951

A.9.8.24	E-UTRAN HD-FDD RSTD intra-frequency measurement accuracy in CE Mode B	3951

A.9.8.24.1	Test Purpose and Environment	3951

A.9.8.24.2	Test Requirements	3955

A.9.8.25	E-UTRAN TDD RSTD intra-frequency measurement accuracy in CE Mode B	3956

A.9.8.25.1	Test Purpose and Environment	3956

A.9.8.25.2	Test Requirements	3959

A.9.8.26	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3960

A.9.8.26.1	Test Purpose and Environment	3960

A.9.8.26.2	Test Requirements	3962

A.9.8.27	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode A	3962

A.9.8.27.1	Test Purpose and Environment	3962

A.9.8.27.2	Test Requirements	3965

A.9.8.28	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode A	3965

A.9.8.28.1	Test Purpose and Environment	3965

A.9.8.28.2	Test Requirements	3968

A.9.8.29	E-UTRAN FDD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3968

A.9.8.29.1	Test Purpose and Environment	3968

A.9.8.29.2	Test Requirements	3972

A.9.8.30	E-UTRAN HD-FDD RSTD inter-frequency measurement accuracy in CE Mode B	3973

A.9.8.30.1	Test Purpose and Environment	3973

A.9.8.30.2	Test Requirements	3976

A.9.8.31	E-UTRAN TDD RSTD inter-frequency measurement accuracy in CE Mode B	3977

A.9.8.31.1	Test Purpose and Environment	3977

A.9.8.31.2	Test Requirements	3981

A.9.8.32	TDD Intra frequency case for UE Category NB1 inband mode in normal coverage	3981

A.9.8.32.1	Test Purpose and Environment	3981

A.9.8.32.2	Test Requirements	3984

A.9.8.33	TDD Inter frequency case for UE Category NB1 inband mode in normal coverage	3984

A.9.8.33.1	Test Purpose and Environment	3984

A.9.8.33.2	Test Requirements	3987

A.9.8.34	TDD Intra frequency case for UE Category NB1 inband mode in enhanced coverage	3987

A.9.8.34.1	Test Purpose and Environment	3987

A.9.8.34.2	Test Requirements	3990

A.9.8.35	TDD Inter frequency case for UE Category NB1 inband mode in enhanced coverage	3990

A.9.8.35.1	Test Purpose and Environment	3990

A.9.8.35.2	Test Requirements	3993

A.9.9	RSRP and RSRQ on the serving cell	3993

A.9.9.1	FDD Intra frequency serving cell case	3993

A.9.9.1.1	Test Purpose and Environment	3993

A.9.9.1.2	Test parameters	3993

A.9.9.1.3	Test Requirements	3996

A.9.9.2	TDD Intra frequency serving cell case	3996

A.9.9.2.1	Test Purpose and Environment	3996

A.9.9.2.2	Test parameters	3996

A.9.9.2.3	Test Requirements	3998

A.9.10	SSTD	3998

A.9.10.1	EUTRAN FDD-FDD SSTD accuracy in asynchronous DC	3998

A.9.10.1.1	Test Purpose and Environment	3998

A.9.10.1.2	Test parameters	3998

A.9.10.1.3	Test Requirements	4000

A.9.10.2	Void	4000

A.9.10.3	Void	4000

A.9.10.4	Void	4000

A.9.11 RSSI	4000

A.9.11.1	FS3 average RSSI accuracy case (PCell using FDD)	4000

A.9.11.1.1	Test Purpose and Environment	4000

A.9.11.1.2	Test parameters	4000

A.9.11.1.3	Test Requirements	4003

A.9.11.2	FS3 average RSSI accuracy case (PCell using TDD)	4003

A.9.11.2.1	Test Purpose and Environment	4003

A.9.11.2.2	Test parameters	4003

A.9.12	Channel occupancy	4006

A.9.12.1	FS3 channel occupancy test (PCell using FDD)	4006

A.9.12.1.1	Test Purpose and Environment	4006

A.9.12.1.2	Test parameters	4006

A.9.12.1.3	Test Requirements	4009

A.9.12.2	FS3 channel occupancy test (PCell using TDD)	4009

A.9.12.2.1	Test Purpose and Environment	4009

A.9.12.2.2	Test parameters	4009

A.9.12.2.3	Test Requirements	4012

A.9.13	RS-SINR	4012

A.9.13.1	FDD Intra-Frequency Case	4012

A.9.13.1.1	Test Purpose and Environment	4012

A.9.13.1.2	Test parameters	4012

A.9.13.1.3	Test Requirements	4015

A.9.13.2	TDD Intra-Frequency Case	4015

A.9.13.2.1	Test Purpose and Environment	4015

A.9.13.2.2	Test parameters	4015

A.9.13.2.3	Test Requirements	4018

A.9.13.3	FDD—FDD Inter frequency case	4018

A.9.13.3.1	Test Purpose and Environment	4018

A.9.13.3.2	Test parameters	4018

A.9.13.3.3	Test Requirements	4021

A.9.13.4	TDD—TDD Inter frequency case	4021

A.9.13.4.1	Test Purpose and Environment	4021

A.9.13.4.2	Test parameters	4021

A.9.13.4.3	Test Requirements	4025

A.9.13.5	FDD—TDD Inter frequency case	4026

A.9.13.5.1	Test Purpose and Environment	4026

A.9.13.5.2	Test parameters	4026

A.9.13.5.3	Test Requirements	4032

A.9.13.6	TDD—FDD Inter frequency case	4032

A.9.13.6.1	Test Purpose and Environment	4032

A.9.13.6.2	Test parameters	4032

A.9.13.6.3	Test Requirements	4036

A.9.14	Channel quality reporting accuracy	4036

A.9.14.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4036

A.9.14.1.1	Test Purpose and Environment	4036

A.9.14.1.2	Test parameters	4037

A.9.14.1.3	Test Requirements	4037

A.9.14.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4038

A.9.14.2.1	Test Purpose and Environment	4038

A.9.14.2.2	Test parameters	4038

A.9.14.2.3	Test Requirements	4039

A.9.14.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4039

A.9.14.3.1	Test Purpose and Environment	4039

A.9.14.3.2	Test parameters	4039

A.9.14.3.3	Test Requirements	4040

A.9.14.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4040

A.9.14.4.1	Test Purpose and Environment	4040

A.9.14.4.2	Test parameters	4040

A.9.14.4.3	Test Requirements	4041

A.10	Proximity-based Services in Any Cell Selection State	4064

A.10.1	E-UTRAN FDD – UE ProSe Direct Communication Transmission Timing Accuracy Test	4064

A.10.1.1	Test Purpose and Environment	4064

A.10.1.2	Test Requirements	4065

A.10.2	E-UTRAN FDD – Initiation/Cease of SLSS Transmission with ProSe Direct Communication	4066

A.10.2.1	Test Purpose and Environment	4066

A.10.2.2	Test Requirements	4067

A.10.3	E-UTRAN FDD – SyncRef UE Selection / Reselection Test	4067

A.10.3.1	Test Purpose and Environment	4067

A.10.3.2	Test Requirements	4069

A.10.4	E-UTRAN FDD – Cell Identification on downlink frequency associated with ProSe frequency (when UE is transmitting for ProSe)	4070

A.10.4.1	Test Purpose and Environment	4070

A.10.4.2	Test Requirements	4072

A.11	V2V Sidelink Communication for V2V Operation on Dedicated V2V Carrier	4072

A.11.1	V2V UE Transmission Timing Accuracy Test	4072

A.11.1.1	Test Purpose and Environment	4072

A.11.1.2	Test requirements	4073

A.11.2	Interruptions due to V2V sidelink communication	4073

A.11.2.1	Test Purpose and Environment	4073

A.11.2.2	Test Requirements	4075

A.12	4075

A.12.1	V2X UE Transmission Timing Accuracy Test	4075

A.12.1.1	V2X UE Transmission Timing Accuracy Test for eNB as Timing Reference	4075

A.12.1.1.1	Test Purpose and Environment	4075

A.12.1.1.2	Test requirements	4076

A.12.1.2	V2X UE Transmission Timing Accuracy Test for SyncRef UE as Timing Reference	4077

A.12.1.2.1	Test Purpose and Environment	4077

A.12.1.2.2	Test Requirements	4077

A.12.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	4078

A.12.2.1	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for eNB as Timing Reference	4078

A.12.2.1.1	Test Purpose and Environment	4078

A.12.2.1.2	Test Requirements	4079

A.12.2.2	Initiation/Cease of SLSS Transmission with V2X Sidelink Communication for SyncRef UE as Timing Reference	4080

A.12.2.2.1	Test Purpose and Environment	4080

A.12.2.2.2	Test Requirements	4081

A.12.3	V2X Synchronization Reference Selection/Reselection Tests	4081

A.12.3.1	V2X Synchronization Reference Selection/Reselection Tests for GNSS configured as the highest priority	4081

A.12.3.1.1	Test Purpose and Environment	4081

A.12.3.1.2	Test Requirements	4084

A.12.3.2	V2X Synchronization Reference Selection/Reselection Tests for eNB configured as the highest priority	4085

A.12.3.2.1	Test Purpose and Environment	4085

A.12.3.1.2	Test Requirements	4087

A.12.4	Congestion Control Measurement Test for V2X UE	4087

A.12.4.1	Test Purpose and Environment	4087

A.12.4.2	Test Requirements	4089

A.12.5	Interruptions due to V2X Sidelink Communication	4089

A.12.5.1	Test Purpose and Environment	4089

A.12.5.2	Test Requirements	4091

A.12.6	V2X UE Autonomous Resource Selection/Reselection Measurement Test	4091

A.12.6.1	V2X UE Autonomous Resource Selection/Reselection Tests for PSSCH-RSRP measurements	4091

A.12.6.1.1	Test Purpose and Environment	4091

A.12.6.1.2	Test Requirements	4093

A.12.6.2	V2X UE Autonomous Resource Selection/Reselection Tests for S-RSSI measurements	4093

A.12.6.2.1	Test Purpose and Environment	4093

A.12.6.1.2	Test Requirements	4095

A.12.7	V2X Synchronization Reference Selection/Reselection Tests for V2X Carrier Aggregation	4095

A.12.7.1	Test Purpose and Environment	4095

A.12.7.2	Test Requirements	4098

A.12.8	Interruptions due to V2X Carrier Aggregation	4099

A.12.8.1	Interruptions on a FDD PCell	4099

A.12.8.1.1	Test Purpose and Environment	4099

A.12.8.1.2	Test Requirements	4100

A.12.8.2	Interruptions on a TDD PCell	4100

A.12.8.2.1	Test Purpose and Environment	4100

A.12.8.2.2	Test Requirements	4102

A.13	E-UTRAN Standalone Tests for UE Category NB for Satellite Access	4103

A.13.1	RRC_IDLE state for satellite access	4103

A.13.1.1	Cell re-selection for satellite access	4103

A.13.1.1.1	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage	4103

A.13.1.1.1.1	Test Purpose and Environment	4103

A.13.1.1.1.2	Test Requirements	4105

A.13.1.1.2	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with serving cell RRM measurement relaxation	4106

A.13.1.1.2.1	Test Purpose and Environment	4106

A.13.1.1.3	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in normal coverage with UE specific DRX	4109

A.13.1.1.3.1	Test Purpose and Environment	4109

A.13.1.1.3.2	Test Requirements	4111

A.13.1.1.4	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in normal coverage	4112

A.13.1.1.4.1	Test Purpose and Environment	4112

A.13.1.1.4.2	Test Requirements	4113

A.13.1.1.5	HD – FDD Intra frequency case for UE Category NB1 Standalone mode in enhanced coverage, location-based cell reselection for NGSO	4114

A.13.1.1.5.1	Test Purpose and Environment	4114

A.13.1.1.5.2	Test Requirements	4115

A.13.1.1.6	HD – FDD Inter frequency case for UE Category NB1 Standalone mode in enhanced coverage, time-based cell reselection for NGSO	4116

A.13.1.1.6.1	Test Purpose and Environment	4116

A.13.1.1.6.2	Test Requirements	4117

A.13.1.1.7	HD – FDD Intra frequency case for UE Category NB1 in in-band mode in NTN NR in normal coverage	4118

A.13.1.1.7.1	Test Purpose and Environment	4118

A.13.1.1.7.2	Test Requirements	4120

A.13.1.1.8	HD – FDD Inter-frequency case for UE Category NB1 in-band mode in NTN NR in normal coverage	4121

A.13.1.1.8.1	Test Purpose and Environment	4121

A.13.1.1.8.2	Test Requirements	4123

A.13.2	Void	4124

A.13.3	RRC connection mobility control for satellite access	4124

A.13.3.1	RRC re-establishment for satellite access	4124

A.13.3.1.1	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under normal coverage	4124

A.13.3.1.1.1	Test Purpose and Environment	4124

A.13.3.1.1.2	Test Requirements	4125

A.13.3.1.2	HD-FDD Intra-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4126

A.13.3.1.2.1	Test Purpose and Environment	4126

A.13.3.1.2.2	Test Requirements	4127

A.13.3.1.3	HD-FDD Inter-frequency RRC Re-establishment for UE category NB1 in Standalone mode under enhanced coverage	4128

A.13.3.1.3.1	Test Purpose and Environment	4128

A.13.3.1.3.2	Test Requirements	4129

A.13.3.2	Random Access for Satellite Access	4129

A.13.3.2.1	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in normal coverage	4130

A.13.3.2.1.1	Test Purpose and Environment	4130

A.13.3.2.1.2	Test Requirements	4132

A.13.3.2.2	Contention Based Random Access Test for UE category NB1 UEs in Satellite Access - Standalone mode in Enhanced Coverage	4134

A.13.3.2.2.1	Test Purpose and Environment	4134

A.13.3.2.2.2	Test Requirements	4135

A.13.3.2.3	Contention Based Random Access on Non-anchor Carrier Test for UE category NB1 UEs Standalone mode in Enhanced Coverage	4137

A.13.3.2.3.1	Test Purpose and Environment	4137

A.13.3.2.3.2	Test Requirements	4139

A.13.4	Timing and signalling characteristics for satellite access	4141

A.13.4.1	UE transmit timing for satellite access	4141

A.13.4.1.1	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access	4141

A.13.4.1.1.1	Test Purpose and Environment	4141

A.13.4.1.1.2	Test Requirements	4142

A.13.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access	4143

A.13.4.1.2.1	Test Purpose and Environment	4143

A.13.4.1.2.2	Test Requirements	4144

A.13.4.1.3	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access	4145

A.13.4.1.3.1	Test Purpose and Environment	4145

A.13.4.1.3.2	Test Requirements	4146

A.13.4.2	UE timing advance for satellite access	4147

A.13.4.2.1	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access	4147

A.13.4.2.1.1	Test Purpose and Environment	4147

A.13.4.2.1.2	Test Requirements	4149

A.13.4.2.2	HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access	4149

A.13.4.2.2.1	Test Purpose and Environment	4149

A.13.4.2.2.2	Test Requirements	4151

A.13.4.3	Radio Link Monitoring for satellite access	4151

A.13.4.3.1	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in normal coverage	4151

A.13.4.3.1.1	Test Purpose and Environment	4151

A.13.4.3.1.2	Test Requirements	4154

A.13.4.3.2	HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category NB1 Standalone mode in enhanced coverage	4154

A.13.4.3.2.1	Test Purpose and Environment	4154

A.13.4.3.2.2	Test Requirements	4157

A.13.4.3.3	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4157

A.13.4.3.3.1	Test Purpose and Environment	4157

A.13.4.3.3.2	Test Requirements	4160

A.13.4.3.4	HD-FDD Radio Link Monitoring Test for In-sync with DRX for UE Category NB1 Standalone mode in Normal Coverage	4160

A.13.4.3.4.1	Test Purpose and Environment	4160

A.13.4.3.4.2	Test Requirements	4163

A.13.4.3.5	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4163

A.13.4.3.5.1	Test Purpose and Environment	4163

A.13.4.3.5.2	Test Requirements	4166

A.13.4.3.6	HD-FDD Radio Link Monitoring Test for In-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4166

A.13.4.3.6.1	Test Purpose and Environment	4166

A.13.4.3.6.2	Test Requirements	4169

A.13.4.3.7	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Normal Coverage	4169

A.13.4.3.7.1	Test Purpose and Environment	4169

A.13.4.3.7.2	Test Requirements	4171

A.13.4.3.8	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 Standalone mode in Enhanced Coverage	4172

A.13.4.3.8.1	Test Purpose and Environment	4172

A.13.4.3.8.2	Test Requirements	4174

A.13.4.3.9	HD-FDD Radio Link Monitoring Test for Out-of-sync without DRX for UE Category NB1 in-band mode in NTN NR in Enhanced Coverage	4175

A.13.4.3.9.1	Test Purpose and Environment	4175

A.13.4.3.9.2	Test Requirements	4178

A.13.5	UE measurement procedures in RRC_CONNECTED state for UE category NB1 for satellite access	4178

A.13.5.4	HD-FDD Intra-frequency neighbour cell measurement for UE category NB1 in in-band mode in NTN NR under normal coverage for Satellite Access	4185

A.13.5.4.1	Test Purpose and Environment	4185

A.13.5.4.2	Test Requirements	4187

A.13.5.5	HD-FDD Inter-frequency neighbour cell measurement for UE category NB1 in in-band mode in NTN NR under normal coverage for Satellite Access	4187

A.13.5.5.1	Test Purpose and Environment	4187

A.13.5.5.2	Test Requirements	4189

A.13.6	Measurement performance requirements for UE for satellite access	4190

A.13.6.1	Void	4190

A.13.6.2	Channel quality reporting accuracy for satellite access	4190

A.13.6.2.1	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under normal coverage	4190

A.13.6.2.1.1	Test Purpose and Environment	4190

A.13.6.2.1.2	Test parameters	4190

A.13.6.2.1.3	Test Requirements	4191

A.13.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category NB1 Standalone mode under enhanced coverage	4191

A.13.6.2.2.1	Test Purpose and Environment	4191

A.13.6.2.2.2	Test parameters	4191

A.13.6.2.2.3	Test Requirements	4192

A.13.6.2.3	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under normal coverage	4192

A.13.6.2.3.1	Test Purpose and Environment	4192

A.13.6.2.3.2	Test parameters	4193

A.13.6.2.3.3	Test Requirements	4193

A.13.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy on non-anchor carrier for UE Category NB1 Standalone mode under enhanced coverage	4194

A.13.6.2.4.1	Test Purpose and Environment	4194

A.13.6.2.4.2	Test parameters	4194

A.13.6.2.4.3	Test Requirements	4195

A.13.6.2.5	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under normal coverage	4195

A.13.6.2.5.1	Test Purpose and Environment	4195

A.13.6.2.5.2	Test parameters	4195

A.13.6.2.5.3	Test Requirements	4196

A.13.6.2.6	E-UTRAN HD-FDD Downlink channel quality reporting accuracy in RRC_CONNECTED for UE Category NB1 Standalone mode under enhanced coverage	4197

A.13.6.2.6.1	Test Purpose and Environment	4197

A.13.6.2.6.2	Test parameters	4197

A.13.6.2.6.3	Test Requirements	4198

A.14	E-UTRAN Standalone Tests for UE Category M1 for Satellite Access	4198

A.14.1	RRC_IDLE state for satellite access	4198

A.14.1.1	Cell re-selection for satellite access	4198

A.14.1.1.1	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4198

A.14.1.1.1.1	Test Purpose and Environment	4198

A.14.1.1.1.2	Test Requirements	4200

A.14.1.1.2	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage	4201

A.14.1.1.2.1	Test Purpose and Environment	4201

A.14.1.1.2.2	Test Requirements	4203

A.14.1.1.3	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4204

A.14.1.1.3.1	Test Purpose and Environment	4204

A.14.1.1.3.2	Test Requirements	4206

A.14.1.1.4	E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in normal coverage with serving cell RRM measurement relaxation	4207

A.14.1.1.4.1	Test Purpose and Environment	4207

A.14.1.1.4.2	Test Requirements	4209

A.14.1.1.5	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4210

A.14.1.1.5.1	Test Purpose and Environment	4210

A.14.1.1.5.2	Test Requirements	4212

A.14.1.1.6	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage	4213

A.14.1.1.6.1	Test Purpose and Environment	4213

A.14.1.1.6.2	Test Requirements	4215

A.14.1.1.7	E-UTRAN FDD – FDD Intra frequency case for Cat-M1 UE in normal coverage, time-based triggering	4216

A.14.1.1.7.1	Test Purpose and Environment	4216

A.14.1.1.7.2	Test Requirements	4217

A.14.1.1.8 E-UTRAN HD – FDD Intra frequency case for Cat-M1 UE in enhanced coverage, time-based triggering	4218

A.14.1.1.8.1	Test Purpose and Environment	4218

A.14.1.1.8.2	Test Requirements	4219

A.14.1.1.9	E-UTRAN FDD – FDD Inter frequency case for Cat-M1 UE in enhanced coverage, location-based triggering	4220

A.14.1.1.9.1	Test Purpose and Environment	4220

A.14.1.1.9.2	Test Requirements	4221

A.14.1.1.10	E-UTRAN HD – FDD Inter frequency case for Cat-M1 UE in normal coverage, location-based triggering	4222

A.14.1.1.10.1	Test Purpose and Environment	4222

A.14.1.1.10.2	Test Requirements	4223

A.14.2	RRC_CONNECTED state mobility for satellite access	4224

A.14.2.1	E-UTRAN handover for satellite access	4224

A.14.2.1.1	E-UTRAN FDD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4224

A.14.2.1.1.1	Test Purpose and Environment	4224

A.14.2.1.1.2	Test Requirements	4226

A.14.2.1.2	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA without SFN acquisition	4227

A.14.2.1.2.1	Test Purpose and Environment	4227

A.14.2.1.2.2	Test Requirements	4228

A.14.2.1.3	E-UTRAN FDD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4229

A.14.2.1.3.1	Test Purpose and Environment	4229

A.14.2.1.3.2	Test Requirements	4230

A.14.2.1.4	E-UTRAN HD-FDD Intra frequency conditional handover for Cat-M1 UEs in CEModeA	4231

A.14.2.1.4.1	Test Purpose and Environment	4231

A.14.2.1.4.2	Test Requirements	4232

A.14.2.1.5	E-UTRAN FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4233

A.14.2.1.5.1	Test Purpose and Environment	4233

A.14.2.1.5.2	Test Requirements	4234

14.2.1.6	E-UTRAN HD-FDD Intra frequency handover for Cat-M1 UEs in CEModeA	4235

A.14.2.1.6.1	Test Purpose and Environment	4235

A.14.2.1.6.2	Test Requirements	4236

A.14.2.1.7	E-UTRAN FD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4237

A.14.2.1.7.1	Test Purpose and Environment	4237

A.14.2.1.7.2	Test Requirements	4238

A.14.2.1.8	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeA	4239

A.14.2.1.8.1	Test Purpose and Environment	4239

A.14.2.1.8.2	Test Requirements	4240

A.14.2.1.9	E-UTRAN FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4241

A.14.2.1.9.1	Test Purpose and Environment	4241

A.14.2.1.9.2	Test Requirements	4242

A.14.2.1.10	E-UTRAN HD-FDD Inter frequency handover for Cat-M1 UEs in CEModeB	4243

A.14.2.1.10.1	Test Purpose and Environment	4243

A.14.2.1.10.2	Test Requirements	4244

A.14.2.1.11	E-UTRAN FDD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4245

A.14.2.1.11.1	Test Purpose and Environment	4245

A.14.2.1.12.2	Test Requirements	4246

A.14.2.1.12	E-UTRAN HD-FDD Inter frequency conditional handover for Cat-M1 UEs in CEModeA	4247

A.14.2.1.12.1	Test Purpose and Environment	4247

A.14.2.1.12.2	Test Requirements	4248

A.14.2.1.15	E-UTRAN FDD-FDD Inter frequency location based conditional handover for Cat-M1 UEs in CEModeA	4253

A.14.2.1.15.1	Test Purpose and Environment	4253

A.14.2.1.16.2	Test Requirements	4254

A.14.2.1.16	E-UTRAN HD-FDD Inter frequency time based conditional handover for Cat-M1 UEs in CEModeA	4255

A.14.2.1.16.1	Test Purpose and Environment	4255

A.14.2.1.16.2	Test Requirements	4256

A.14.3	RRC connection mobility control for satellite access	4257

A.14.3.1	RRC re-establishment for satellite access	4257

A.14.3.1.1	E-UTRAN FD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4257

A.14.3.1.1.1	Test Purpose and Environment	4257

A.14.3.1.1.2	Test Requirements	4258

A.14.3.1.2	E-UTRAN HD-FDD Intra-frequency RRC Re-establishment for Cat-M1 UE in CEModeA	4259

A.14.3.1.2.1	Test Purpose and Environment	4259

A.14.3.1.2.2	Test Requirements	4261

A.14.3.1.3	E-UTRAN FD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4262

A.14.3.1.3.1	Test Purpose and Environment	4262

A.14.3.1.3.2	Test Requirements	4264

A.14.3.1.4	E-UTRAN HD-FDD Inter-frequency RRC Re-establishment for Cat-M1 UE in CEModeA for Satellite access	4265

A.14.3.1.4.1	Test Purpose and Environment	4265

A.14.3.1.4.2	 Test Requirements	4267

A.14.3.2	Random access for satellite access	4268

A.14.3.2.1	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4268

A.14.3.2.1.1	Test Purpose and Environment	4268

A.14.3.2.1.2	Test Requirements	4270

A.14.3.2.2	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Normal Coverage for satellite access	4271

A.14.3.2.2.1	Test Purpose and Environment	4271

A.14.3.2.2.2	Test Requirements	4273

A.14.3.2.3	E-UTRAN FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4275

A.14.3.2.3.1	Test Purpose and Environment	4275

A.14.3.2.3.2	Test Requirements	4277

A.14.3.2.4	E-UTRAN HD-FDD Contention Based Random Access Test for Cat-M1 UEs in Enhanced Coverage for satellite access	4278

A.14.3.2.4.1	Test Purpose and Environment	4278

A.14.3.2.4.2	Test Requirements	4280

A.14.4	Timing and signalling characteristics for satellite access	4282

A.14.4.1	UE transmit timing for satellite access	4282

A.14.4.1.1	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4282

A.14.4.1.1.1	Test Purpose and Environment	4282

A.14.4.1.1.2	Test Requirements	4284

A.14.4.1.2	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeA	4285

A.14.4.1.2.1	Test Purpose and Environment	4285

A.14.4.1.2.2	Test Requirements	4286

A.14.4.1.3	E-UTRAN FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4287

A.14.4.1.3.1	Test Purpose and Environment	4287

A.14.4.1.3.2	Test Requirements	4288

A.14.4.1.4	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB	4289

A.14.4.1.4.1	Test Purpose and Environment	4289

A.14.4.1.4.2	Test Requirements	4290

A.14.4.1.5	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Cat-M1 UE in CEModeB with segment transmission in NGSO for Satellite Access	4291

A.14.4.1.5.1	Test Purpose and Environment	4291

A.14.4.1.5.2	Test Requirements	4292

A.14.4.2	UE timing advance for satellite access	4293

A.14.4.2.1	E-UTRAN FDD Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4293

A.14.4.2.1.1	Test Purpose and Environment	4293

A.14.4.2.1.2	Test Requirements	4295

A.14.4.2.2	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test for Cat-M1 UE in CEModeA	4295

A.14.4.2.2.1	Test Purpose and Environment	4295

A.14.4.2.2.2	Test Requirements	4298

A.14.4.2.3	E-UTRAN FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4298

A.14.4.2.3.1	Test Purpose and Environment	4298

A.14.4.2.3.2	Test Requirements	4299

A.14.4.2.4	E-UTRAN HD-FDD UE Timing Advance Adjustment Accuracy Test in CEModeB	4299

A.14.4.2.4.1	Test Purpose and Environment	4299

A.14.4.2.4.2	Test Requirements	4301

A.14.4.3	Radio Link Monitoring for satellite access	4302

A.14.4.3.1	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4302

A.14.4.3.1.1	Test Purpose and Environment	4302

A.14.4.3.1.2	Test Requirements	4305

A.14.4.3.2	E-UTRAN FD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4305

A.14.4.3.2.1	Test Purpose and Environment	4305

A.14.4.3.2.2	Test Requirements	4308

A.14.4.3.3	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync for Cat-M1 UE in CEMode A for Satellite access	4308

A.14.4.3.3.1	Test Purpose and Environment	4308

A.14.4.3.3.2	Test Requirements	4311

A.14.4.3.4	E-UTRAN HD-FDD Radio Link Monitoring Test for In-Sync for Cat-M1 UE in CEMode A for Satellite access	4311

A.14.4.3.4.1	Test Purpose and Environment	4311

A.14.4.3.4.2	Test Requirements	4314

A.14.4.3.5	E-UTRAN FD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4314

A.14.4.3.5.1	Test Purpose and Environment	4314

A.14.4.3.5.2	 Test Requirements	4317

A.14.4.3.6	E-UTRAN FD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4317

A.14.4.3.6.1	Test Purpose and Environment	4317

A.14.4.3.6.2	Test Requirements	4320

A.14.4.3.7	E-UTRAN HD-FDD Radio Link Monitoring Test for Out-of-sync in DRX for UE category M1 configured in CEMode A	4320

A.14.4.3.7.1	Test Purpose and Environment	4320

A.14.4.3.8	E-UTRAN HD-FDD Radio Link Monitoring Test for In-sync in DRX for UE Category M1 configured in CEMode A	4323

A.14.4.3.8.1 	Test Purpose and Environment	4323

A.14.4.3.8.2	Test Requirements	4326

A.14.5	UE measurement procedures in RRC_CONNECTED state for satellite access	4326

A.14.5.1 Intra-frequency measurements for satellite access	4326

A.14.5.1.1	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4326

A.14.5.1.1.1	Test Purpose and Environment	4326

A.14.5.1.1.2	Test Requirements	4328

A.14.5.1.2	E-UTRAN FDD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4329

A.14.5.1.2.1	Test Purpose and Environment	4329

A.14.5.1.2.2	Test Requirements	4331

A.14.5.1.3	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in asynchronous cells for Cat-M1 UE in CEModeA	4331

A.14.5.1.3.1	Test Purpose and Environment	4331

A.14.5.1.3.2	Test Requirements	4333

A.14.5.1.4	E-UTRAN HD-FDD intra-frequency event triggered reporting under AWGN conditions in synchronous cells for Cat-M1 UE in CEModeA in DRX	4334

A.14.5.1.4.1	Test Purpose and Environment	4334

A.14.5.1.4.2	Test Requirements	4336

A.14.5.1.5	E-UTRAN FD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4336

A.14.5.1.5.1	Test Purpose and Environment	4336

A.14.5.1.5.2	Test Requirements	4338

A.14.5.1.6 	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA with location-based triggering	4339

A.14.5.1.6.1	Test Purpose and Environment	4339

A.14.5.1.6.2	Test Requirements	4340

A.14.5.1.7	E-UTRAN HD-FDD Intra-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4341

A.14.5.1.7.1	Test Purpose and Environment	4341

A.14.5.1.7.2	Test Requirements	4342

A.14.5.2 Inter-frequency measurements for satellite access	4343

A.14.5.2.1	E-UTRAN FD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4343

A.14.5.2.1.1	Test Purpose and Environment	4343

A.14.5.2.1.2	Test Requirements	4344

A.14.5.2.2	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeA when DRX is used with time-based triggering	4345

A.14.5.2.2.1	Test Purpose and Environment	4345

A.14.5.2.2.2	Test Requirements	4346

A.14.5.2.3	E-UTRAN HD-FDD Inter-frequency event triggered reporting in asynchronous cells for UE category M1 in CEModeB when DRX is used with time-based triggering	4347

A.14.5.2.3.1	Test Purpose and Environment	4347

A.14.5.2.3.2	Test Requirements	4348

A.14.5.2.4	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4349

A.14.5.2.4.1	Test Purpose and Environment	4349

A.14.5.2.4.2	Test Requirement	4350

A.14.5.2.5	E-UTRAN FDD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA when DRX is used	4351

A.14.5.2.5.1	Test Purpose and Environment	4351

A.14.5.2.5.2	Test Requirement	4353

A.14.5.2.6	E-UTRAN HD-FDD Inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeA	4353

A.14.5.2.6.1	Test Purpose and Environment	4353

A.14.5.2.6.2	Test Requirement	4355

A.14.5.2.7	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 in CEModeA in DRX	4355

A.14.5.2.7.1	Test Purpose and Environment	4355

A.14.5.2.7.2	Test Requirement	4357

A.14.5.2.8	E-UTRAN FDD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4357

A.14.5.2.8.1	Test Purpose and Environment	4357

A.14.5.2.8.2	Test Requirement	4359

A.14.5.2.9	E-UTRAN HD-FDD inter-frequency event triggered reporting under AWGN conditions in asynchronous cells for UE category M1 with discontinuous MPDCCH monitoring in CEModeB	4360

A.14.5.2.9.1	Test Purpose and Environment	4360

A.14.5.2.9.2	Test Requirement	4361

A.14.6	Measurement performance requirements for UE for satellite access	4362

A.14.6.1	RSRP for satellite access	4362

A.14.6.1.1	FD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4362

A.14.6.1.1.1	Test Purpose and Environment	4362

A.14.6.1.1.2	Test parameters	4362

A.14.6.1.1.3	Test Requirements	4364

A.14.6.1.2	HD-FDD RSRP Intra frequency case for Cat-M1 UE in CEModeA	4364

A.14.6.1.2.1	Test Purpose and Environment	4364

A.14.6.1.2.2	Test parameters	4364

A.14.6.1.2.3	Test Requirements	4366

A.14.6.1.3	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4366

A.14.6.1.3.1	Test Purpose and Environment	4366

A.14.6.1.3.2	Test parameters	4366

A.14.6.1.3.3	Test Requirements	4367

A.14.6.1.4	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeA	4368

A.14.6.1.4.1	Test Purpose and Environment	4368

A.14.6.1.4.2	Test parameters	4368

A.14.6.1.4.3	Test Requirements	4369

A.14.6.1.5	FD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4369

A.14.6.1.5.1	Test Purpose and Environment	4369

A.14.6.1.5.2	Test parameters	4369

A.14.6.1.5.3	Test Requirements	4370

A.14.6.1.6	HD-FDD RSRP Inter frequency case for Cat-M1 UE in CEModeB	4370

A.14.6.1.6.1	Test Purpose and Environment	4370

A.14.6.1.6.2	Test parameters	4371

A.14.6.1.6.3	Test Requirements	4372

A.14.6.2	Channel quality reporting accuracy for satellite access	4372

A.14.6.2.1	E-UTRAN FD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4372

A.14.6.2.1.1	Test Purpose and Environment	4372

A.14.6.2.1.2	Test parameters	4372

A.14.6.2.1.3	Test Requirements	4373

A.14.6.2.2	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode A for Satellite access	4374

A.14.6.2.2.1	Test Purpose and Environment	4374

A.14.6.2.2.2	Test parameters	4374

A.14.6.2.2.3	Test Requirements	4375

A.14.6.2.3.2	Test parameters	4375

A.14.6.2.3.3	Test Requirements	4376

A.14.6.2.4	E-UTRAN HD-FDD Downlink channel quality reporting accuracy for UE Category M1 in CE Mode B for Satellite access	4377

A.14.6.2.4.1	Test Purpose and Environment	4377

A.14.6.2.4.2	Test parameters	4377

A.14.6.2.4.3	Test Requirements	4378

Annex B (normative):	Conditions for RRM requirements applicability for operating bands	4379

B.1	Conditions for E-UTRAN RRC_IDLE state mobility	4379

B.1.1	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection	4379

B.1.2	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection	4379

B.1.3	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1	4379

B.1.4	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1	4381

B.1.5	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1	4382

B.1.6	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category 1bis	4382

B.1.7	Conditions for measurements of E-UTRAN cells for cell re-selection for UE Category M2	4382

B.1.7.1	Conditions for measurements of intra-frequence E-UTRAN cells for cell selection	4382

B.1.7.2	Condition for measurements of inter-frequence E-UTRAN cells for cell selection	4384

B.1.8	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1	4385

B.1.9	Conditions for measurements of intra-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4385

B.1.10	Conditions for measurements of intra-frequency NB-IoT cells for cell re-selection for UE Category NB1 and NB2 for satellite access	4386

B.1.11	Conditions for measurements of inter-frequency NB-IoT cells for cell re-selection for UE Category NB1 for satellite access	4387

B.1.12	Conditions for measurements of inter-frequency E-UTRAN cells for cell re-selection for UE Category M1 for satellite access	4387

B.2	Conditions for UE Measurements Procedures in RRC_CONNECTED State	4387

B.2.1	Conditions for E-UTRAN intra-frequency measurements	4387

B.2.2	Conditions for E-UTRAN intra-frequency measurements with autonomous gaps	4388

B.2.3	Conditions for E-UTRAN inter-frequency measurements	4388

B.2.4	Conditions for E-UTRAN inter-frequency measurements with autonomous gaps	4389

B.2.5	Conditions for E-UTRAN OTDOA intra-frequency RSTD Measurements	4389

B.2.6	Conditions for E-UTRAN OTDOA inter-frequency RSTD Measurements	4389

B.2.7	Conditions for Measurements of the secondary component carrier with deactivated SCell	4390

B.2.8	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction	4390

B.2.9	Conditions for E-UTRAN Intra-Frequency Measurements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4391

B.2.10	Conditions for E-UTRAN intra-frequency discovery signal measurements	4391

B.2.10.1	Conditions for E-UTRAN intra-frequency CRS-based measurements	4391

B.2.10.2	Conditions for E-UTRAN intra-frequency CSI-RS based measurements	4391

B.2.11	Conditions for E-UTRAN inter-frequency discovery signal measurements	4392

B.2.11.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4392

B.2.11.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4392

B.2.12	Conditions for E-UTRAN intra-frequency discovery signal measurements under operation with frame structure 3	4393

B.2.13	Conditions for E-UTRAN inter-frequency discovery signal measurements under operation with frame structure 3	4393

B.2.13.1	Conditions for E-UTRAN inter-frequency CRS-based measurements	4393

B.2.13.2	Conditions for E-UTRAN inter-frequency CSI-RS based measurements	4394

B.2.14	Conditions for E-UTRAN intra-frequency measurements by UE Category M1	4394

B.2.15	Conditions for NB-IoT intra-frequency measurements by UE Category NB1	4395

B.2.16	Conditions for NB-IoT intra-frequency RSTD measurements by UE Category NB1	4396

B.2.17	Conditions for NB-IoT inter-frequency RSTD measurements by UE Category NB1	4397

B.2.18	Conditions for E-UTRAN inter-frequency measurements by UE Category M1	4398

B.2.19	Conditions for E-UTRAN measurements by UE Category M2	4399

B.2.19.1	Conditions for E-UTRAN intra-frequency measurements	4399

B.2.19.2	Conditions for E-UTRAN inter-frequency measurements	4399

B.2.20	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M1	4399

B.2.21	Conditions for E-UTRAN inter-frequency RSTD measurements by UE Category M2	4400

B.2.22	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M1	4400

B.2.23	Conditions for E-UTRAN intra-frequency RSTD measurements by UE Category M2	4402

B.2.24	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4402

B.2.25	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1	4402

B.2.26	Conditions for E-UTRAN intra-frequency measurements by UE Category M1 for satellite access	4402

B.2.27	Conditions for NB-IoT intra-frequency measurements by UE Category NB1 and NB2 for satellite access	4403

B.2.28	Conditions for E-UTRAN inter-frequency measurements by UE Category M1 for satellite access	4404

B.2.29	Conditions for intra-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4404

B.2.30	Conditions for inter-frequency neighbour cell measurements of NB-IoT cells for UE Category NB1 for satellite access	4405

B.3	Conditions for measurements performance requirements for UE	4405

B.3.1	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements	4405

B.3.2	Void	4405

B.3.3	Conditions for inter-frequency RSRP and RSRQ Accuracy Requirements	4406

B.3.4	Conditions for inter-frequency relative RSRP and RSRQ Accuracy Requirements	4406

B.3.5	Conditions for UE Rx – Tx time difference	4406

B.3.6	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements	4406

B.3.7	Conditions for inter-frequency RSTD measurements	4406

B.3.8	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements	4406

B.3.9	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction	4407

B.3.10	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction	4407

B.3.11	Conditions for Intra-Frequency Absolute RSRP and RSRQ Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4407

B.3.12	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements under Time Domain Measurement Resource Restriction with CRS Assistance Information	4407

B.3.13	Conditions for UE Rx–Tx Time Difference Measurement under Time Domain Measurement Resource Restriction with CRS Assistance Information	4407

B.3.14	Conditions for Intra-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4407

B.3.14.1	Conditions for Intra-frequency CRS-based measurements	4407

B.3.14.2	Conditions for Intra-frequency CSI-RS-based measurements	4408

B.3.15	Conditions for Intra-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4408

B.3.15.1	Conditions for Intra-frequency CRS-based measurements	4408

B.3.15.2	Conditions for Intra-frequency CSI-RS-based measurements	4408

B.3.16	Conditions for Inter-Frequency Absolute Discovery Signal Measurement Accuracy Requirements	4409

B.3.16.1	Conditions for Inter-frequency CRS-based measurements	4409

B.3.16.2	Conditions for Inter-frequency CSI-RS-based measurements	4409

B.3.17	Conditions for Inter-Frequency Relative Discovery Signal Measurement Accuracy Requirements	4409

B.3.17.1	Conditions for Inter-frequency CRS-based measurements	4409

B.3.17.2	Conditions for Inter-frequency CSI-RS-based measurements	4409

B.3.18	Conditions for Intra-frequency Absolute RS-SINR Accuracy Requirements	4410

B.3.19	Conditions for Inter-frequency Absolute RS-SINR Accuracy Requirements	4410

B.3.20	Conditions for Inter-frequency Relative RS-SINR Accuracy Requirements	4410

B.3.21	Conditions for Intra-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4410

B.3.21.1	Conditions for RSRP measurements	4410

B.3.21.2	Conditions for RSRQ measurements	4410

B.3.21.3	Conditions for CSI-RSRP measurements	4410

B.3.22	Conditions for Intra-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4411

B.3.22.1	Conditions for RSRP measurements	4411

B.3.22.2	Void	4411

B.3.22.3	Conditions for CSI-RSRP measurements	4411

B.3.23	Conditions for Inter-Frequency Absolute Accuracy Requirements for Measurements under Operation with Frame Structure 3	4411

B.3.23.1	Conditions for RSRP measurements	4411

B.3.23.2	Conditions for RSRQ measurements	4412

B.3.23.3	Conditions for CSI-RSRP measurements	4412

B.3.24	Conditions for Inter-Frequency Relative Accuracy Requirements for Measurements under Operation with Frame Structure 3	4412

B.3.24.1	Conditions for RSRP measurements	4412

B.3.24.2	Conditions for RSRQ measurements	4412

B.3.24.3	Conditions for CSI-RSRP measurements	4412

B.3.25	Conditions for NB-IoT intra-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4412

B.3.26	Conditions for NB-IoT inter-frequency Absolute NRSRP and NRSRQ Accuracy Requirements for UE Category NB1	4413

B.3.27	Conditions for intra-frequency RSRP and RSRQ Accuracy Requirements for Category 0	4413

B.3.28	Conditions for Intra-Frequency Relative RSRP Accuracy Requirements for Category 0	4413

B.3.29	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4414

B.3.30	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for NB1	4414

B.3.31	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4414

B.3.32	Conditions for inter-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4414

B.3.33	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M1	4414

B.3.34	Conditions for intra-frequency Reference Signal Time Difference (RSTD) measurements for Cat M2	4414

B.4	RRM Requirements Exceptions	4415

B.4.1	General	4415

B.4.2	Receiver sensitivity relaxation for UE supporting CA	4415

B.4.3	Receiver sensitivity relaxation for UE configured with CA	4415

B.4.3.1	Inter-band carrier aggregation	4415

B.4.3.2	Intra-band non-contiguous carrier aggregation	4415

B.4.3.3	Inter-band carrier aggregation with operating bands without uplink band	4415

B.5	Conditions for Measurement Performance Requirements for ProSe UE	4416

B.5.1	Conditions for S-RSRP Accuracy Requirements	4416

B.5.2	Conditions for Relative S-RSRP Accuracy Requirements	4416

B.5.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4416

B.5.4	Conditions for SD-RSRP Accuracy Requirements	4417

B.5.5	Conditions for Relative SD-RSRP Accuracy Requirements	4417

B.6	Conditions for V2X	4418

B.6.1	Test parameters for GNSS signals	4418

B.6.2	Conditions for Absolute S-RSRP Accuracy Requirements	4418

B.6.3	Conditions for Relative S-RSRP Accuracy Requirements	4418

B.6.4	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	4418

B.6.5	Conditions for Absolute PSSCH-RSRP Accuracy Requirements	4419

B.7	Conditions for sTTI and 1ms-TTI with 3 Subframe HARQ Processing	4419

B.7.1	Conditions for Maximum Timing Difference Between Uplink and Downlink Carriers in Carrier Aggregation	4419

B.8	High level test procedure for SAN RRM tests	4421

Annex C (informative):	Change history:	4421