3GPP TS 36.213 V16.7.1 (2021-10)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Physical layer procedures

(Release 16)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organisational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organisational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organisational Partners' Publications Offices.

Keywords

E-UTRA, radio, layer 1

3GPP

Postal address

3GPP support office address

650 Route des Lucioles – Sophia Antipolis

Valbonne – France

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	8

1	Scope	9

2	References	9

3	Symbols and abbreviations	10

3.1	Symbols	10

3.2	Abbreviations	10

4	Synchronization procedures	12

4.1	Cell search	12

4.2	Timing synchronization	12

4.2.1	Radio link monitoring	12

4.2.2	Inter-cell synchronization	12

4.2.3	Transmission timing adjustments	12

4.3	Timing for Secondary Cell Activation / Deactivation	14

5	Power control	15

5.1	Uplink power control	15

5.1.1	Physical uplink shared channel	16

5.1.1.1	UE behaviour	16

5.1.1.2	Power headroom	31

5.1.2	Physical uplink control channel	34

5.1.2.1	UE behaviour	34

5.1.3	Sounding Reference Symbol (SRS)	40

5.1.3.1	UE behaviour	40

5.1.3.2	Power headroom for Type3 report	43

5.1.4	Power allocation for EUTRA dual connectivity	43

5.1.4.1	Dual connectivity power control Mode 1	44

5.1.4.2	Dual connectivity power control Mode 2	51

5.1.4a	Power allocation for dual active protocol stack	55

5.1.5	Power allocation for PUCCH-SCell	56

5.2	Downlink power allocation	57

5.2.1	eNodeB Relative Narrowband TX Power (RNTP) restrictions	59

6	Random access procedure	61

6.1	Physical non-synchronized random access procedure	61

6.1.1	Timing	62

6.2	Random Access Response Grant	63

7	Physical downlink shared channel related procedures	68

7.1	UE procedure for receiving the physical downlink shared channel	69

7.1.1	Single-antenna port scheme	88

7.1.2	Transmit diversity scheme	88

7.1.3	Large delay CDD scheme	89

7.1.4	Closed-loop spatial multiplexing scheme	89

7.1.5	Multi-user MIMO scheme	89

7.1.5A	Dual layer scheme	89

7.1.5B	Up to 8 layer transmission scheme	89

7.1.6	Resource allocation	89

7.1.6.1	Resource allocation type 0	91

7.1.6.2	Resource allocation type 1	92

7.1.6.3	Resource allocation type 2	93

7.1.6.4	PDSCH starting position	96

7.1.6.4A	PDSCH starting position for BL/CE UEs	98

7.1.6.5	Physical Resource Block (PRB) bundling	98

7.1.7	Modulation order and transport block size determination	100

7.1.7.1	Modulation order and redundancy version determination	103

7.1.7.2	Transport block size determination	109

7.1.7.2.1	Transport blocks not mapped to two or more layer spatial multiplexing	114

7.1.7.2.2	Transport blocks mapped to two-layer spatial multiplexing	123

7.1.7.2.3	Transport blocks mapped for DCI Format 1C and DCI Format 6-2	123

7.1.7.2.4	Transport blocks mapped to three-layer spatial multiplexing	124

7.1.7.2.5	Transport blocks mapped to four-layer spatial multiplexing	124

7.1.7.2.6	Transport blocks mapped for BL/CE UEs configured with CEModeB and PDSCH bandwidth up to 1.4MHz	125

7.1.7.2.7	Transport blocks mapped for BL/CE UEs SystemInformationBlockType1-BR	126

7.1.7.2.8	Transport blocks mapped for UEs configured with ce-pdsch-maxBandwidth-config value of 5 MHz or with pdsch-MaxBandwidth-SC-MTCH value of 24 PRBs	126

7.1.7.3	Redundancy Version determination for Format 1C	126

7.1.8	Storing soft channel bits	127

7.1.9	PDSCH resource mapping parameters	127

7.1.10	Antenna ports quasi co-location for PDSCH	129

7.1.11	PDSCH subframe assignment for BL/CE UE	130

7.2	UE procedure for reporting Channel State Information (CSI)	132

7.2.1	Aperiodic CSI Reporting using PUSCH	142

7.2.2	Periodic CSI Reporting using PUCCH	168

7.2.3	Channel Quality Indicator (CQI) definition	209

7.2.4	Precoding Matrix Indicator (PMI) definition	223

7.2.5	Channel-State Information – Reference Signal (CSI-RS) definition	251

7.2.6	Channel-State Information – Interference Measurement (CSI-IM) Resource definition	253

7.2.7	Zero Power CSI-RS Resource definition	253

7.2.8	CSI-RS Activation / Deactivation	253

7.3	UE procedure for reporting HARQ-ACK	254

7.3.1	FDD HARQ-ACK reporting procedure	258

7.3.2	TDD HARQ-ACK reporting procedure	263

7.3.2.1	TDD HARQ-ACK reporting procedure for same UL/DL configuration	263

7.3.2.2	TDD HARQ-ACK reporting procedure for different UL/DL configurations	279

7.3.3	FDD-TDD HARQ-ACK reporting procedure for primary cell frame structure type 1	286

7.3.4	FDD-TDD HARQ-ACK reporting procedure for primary cell frame structure type 2	288

8	Physical uplink shared channel related procedures	290

8.0	UE procedure for transmitting the physical uplink shared channel	291

8.0.1	Single-antenna port scheme	318

8.0.2	Closed-loop spatial multiplexing scheme	318

8.1	Resource allocation for PDCCH/EPDCCH/SPDCCH with uplink DCI format	319

8.1.1	Uplink resource allocation type 0	319

8.1.2	Uplink resource allocation type 1	320

8.1.3	Uplink resource allocation type 2	320

8.1.4	Uplink resource allocation type 3	321

8.1.5	Uplink resource allocation type 4	322

8.1.5.1	UL Resource Block Groups	322

8.1.6	Uplink resource allocation type 5	323

8.2	UE sounding procedure	325

8.3	UE HARQ-ACK procedure	337

8.3A	Autonomous uplink feedback procedure	339

8.4	UE PUSCH hopping procedure	339

8.4.1	Type 1 PUSCH hopping	340

8.4.2	Type 2 PUSCH hopping	340

8.5	UE Reference Symbol (RS) procedure	341

8.6	Modulation order, redundancy version and transport block size determination	342

8.6.1	Modulation order and redundancy version determination	342

8.6.2	Transport block size determination	350

8.6.3	Control information MCS offset determination	357

8.7	UE transmit antenna selection	361

8.8	Transmission timing adjustments	361

9	Physical downlink control channel procedures	361

9.1	UE procedure for determining physical downlink control channel assignment	362

9.1.1	PDCCH assignment procedure	362

9.1.2	PHICH assignment procedure	366

9.1.3	Control Format Indicator (CFI) assignment procedure	369

9.1.4	EPDCCH assignment procedure	370

9.1.4.1	EPDCCH starting position	377

9.1.4.2	Antenna ports quasi co-location for EPDCCH	377

9.1.4.3	Resource mapping parameters for EPDCCH	378

9.1.4.4	PRB-pair indication for EPDCCH	378

9.1.5	MPDCCH assignment procedure	379

9.1.5.1	MPDCCH starting position	387

9.1.5.2	Antenna ports quasi co-location for MPDCCH	387

9.1.5.3	Preconfigured Uplink Resource ACK/fallback procedure	387

9.1.6	SPDCCH assignment procedure	387

9.1.6.1	Resource mapping parameters for SPDCCH	389

9.1.6.2	PRB-pair indication for SPDCCH	389

9.1.6.3	Physical Resource Block (PRB) bundling for DMRS-based SPDCCH	390

9.1.6.4	Antenna ports quasi co-location for DMRS-based SPDCCH	390

9.2	PDCCH/EPDCCH/MPDCCH/SPDCCH validation for semi-persistent scheduling	391

9.2A	PDCCH/EPDCCH validation for autonomous uplink transmissions	393

9.3	PDCCH/EPDCCH/MPDCCH/SPDCCH control information procedure	394

10	Physical uplink control channel procedures	395

10.1	UE procedure for determining physical uplink control channel assignment	396

10.1.1	PUCCH format information	401

10.1.2	FDD HARQ-ACK feedback procedures	408

10.1.2.1	FDD HARQ-ACK procedure for one configured serving cell	408

10.1.2.2	FDD HARQ-ACK procedures for more than one configured serving cell	411

10.1.2.2.1	PUCCH format 1b with channel selection HARQ-ACK procedure	412

10.1.2.2.2	PUCCH format 3 HARQ-ACK procedure	415

10.1.2.2.3	PUCCH format 4 HARQ-ACK procedure	417

10.1.2.2.4	PUCCH format 5 HARQ-ACK procedure	420

10.1.3	TDD HARQ-ACK feedback procedures	421

10.1.3.1	TDD HARQ-ACK procedure for one configured serving cell	423

10.1.3.2	TDD HARQ-ACK procedure for more than one configured serving cell	437

10.1.3.2.1	PUCCH format 1b with channel selection HARQ-ACK procedure	437

10.1.3.2.2	PUCCH format 3 HARQ-ACK procedure	453

10.1.3.2.3	PUCCH format 4 HARQ-ACK procedure	461

10.1.3.2.4	PUCCH format 5 HARQ-ACK procedure	477

10.1.3A	FDD-TDD HARQ-ACK feedback procedures for primary cell frame structure type 2	477

10.1.4	HARQ-ACK Repetition procedure	479

10.1.5	Scheduling Request (SR) procedure	480

10.2	Uplink HARQ-ACK timing	482

11	Physical Multicast Channel (PMCH) related procedures	488

11.1	UE procedure for receiving the PMCH	488

11.2	UE procedure for receiving MCCH and system information change notification	491

12	Assumptions independent of physical channel	491

13	Uplink/Downlink configuration determination procedure for Frame Structure Type 2	492

13.1	UE procedure for determining eIMTA-uplink/downlink configuration	493

13A	Subframe configuration for Frame Structure Type 3	494

14 UE procedures related to Sidelink	497

14.1	Physical Sidelink Shared Channel related procedures	498

14.1.1	UE procedure for transmitting the PSSCH	498

14.1.1.1	UE procedure for determining subframes for transmitting PSSCH for sidelink transmission mode 1	500

14.1.1.1.1 Determination of subframe indicator bitmap	500

14.1.1.2	UE procedure for determining resource blocks for transmitting PSSCH for sidelink transmission mode 1	503

14.1.1.2.1	PSSCH resource allocation for sidelink transmission mode 1	503

14.1.1.2.2	PSSCH frequency hopping for sidelink transmission mode 1	504

14.1.1.3	UE procedure for determining subframes for transmitting PSSCH for sidelink transmission mode 2	504

14.1.1.4	UE procedure for determining resource blocks for transmitting PSSCH for sidelink transmission mode 2	505

14.1.1.4A	UE procedure for determining subframes and resource blocks for transmitting PSSCH for sidelink transmission mode 3	505

14.1.1.4B	UE procedure for determining subframes and resource blocks for transmitting PSSCH and reserving resources for sidelink transmission mode 4	506

14.1.1.4C	UE procedure for determining subframes and resource blocks for PSSCH transmission associated with an SCI format 1	506

14.1.1.5	UE procedure for PSSCH power control	507

14.1.1.6	UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink transmission mode 4 and in sensing measurement in sidelink transmission mode 3	509

14.1.1.7	Conditions for selecting resources when the number of HARQ transmissions is two in sidelink transmission mode 4	512

14.1.2	UE procedure for receiving the PSSCH	512

14.1.3	UE procedure for determining resource block pool and subframe pool for sidelink transmission mode 2	513

14.1.5	UE procedure for determining resource block pool and subframe pool for sidelink transmission mode 3 and 4	514

14.2	Physical Sidelink Control Channel related procedures	515

14.2.1	UE procedure for transmitting the PSCCH	515

14.2.1.1	UE procedure for determining subframes and resource blocks for transmitting PSCCH for sidelink transmission mode 1	518

14.2.1.2	UE procedure for determining subframes and resource blocks for transmitting PSCCH for sidelink transmission mode 2	518

14.2.1.3	UE procedure for PSCCH power control	518

14.2.2	UE procedure for receiving the PSCCH	520

14.2.3	UE procedure for determining resource block pool and subframe pool for PSCCH	520

14.2.4	UE procedure for determining resource block pool for PSCCH in sidelink transmission mode 3 and 4	521

15	Void	524

16	UE Procedures related to narrowband IoT	525

16.1	Synchronization procedures	525

16.1.1	Cell search	525

16.1.2	Timing synchronization	525

16.2	Power control	525

16.2.1	Uplink power control	525

16.2.1.1	Narrowband physical uplink shared channel	525

16.2.1.1.1	UE behaviour	525

16.2.1.1.2	Power headroom	526

16.2.1.2	SR	527

16.2.1.2.1	UE behaviour	527

16.2.2	Downlink power allocation	527

16.3	Random access procedure	528

16.3.1	Physical non-synchronized random access procedure	528

16.3.2	Timing	528

16.3.3	Narrowband random access response grant	529

16.4	Narrowband physical downlink shared channel related procedures	531

16.4.1	UE procedure for receiving the narrowband physical downlink shared channel	531

16.4.1.1	Single-antenna port scheme	535

16.4.1.2	Transmit diversity scheme	535

16.4.1.3	Resource allocation	535

16.4.1.4	NPDSCH starting position	538

16.4.1.5	Modulation order and transport block size determination	539

16.4.1.5.1	Transport blocks not mapped for SystemInformationBlockType1-NB	539

16.4.1.5.2	Transport blocks mapped for SystemInformationBlockType1-NB	540

16.4.2	UE procedure for reporting ACK/NACK	540

16.5	Narrowband physical uplink shared channel related procedures	542

16.5.1	UE procedure for transmitting format 1 narrowband physical uplink shared channel	542

16.5.1.1	Resource allocation	545

16.5.1.2	Modulation order, redundancy version and transport block size determination	546

16.5.2	UE procedure for NPUSCH retransmission	548

16.5.3	UE procedure for transmitting SR	548

16.6	Narrowband physical downlink control channel related procedures	548

16.6.1	NPDCCH starting position	554

16.6.2	NPDCCH control information procedure	554

16.6.3	NPDCCH validation for semi-persistent scheduling	555

16.6.4	Preconfigured uplink resource ACK/fallback procedure	555

16.7	Assumptions independent of physical channel related to narrowband IoT	555

16.8	UE procedure for acquiring cell-specific reference signal sequence and raster offset	555

16.9	UE procedure for receiving narrowband wake up signal	556

17	Wake-up signal related procedures for BL/CE UE	557

Annex A (informative): Change history	558