3GPP TS 37.355 V17.0.0 (2022-03)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

LTE Positioning Protocol (LPP)

(Release 17)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Bluetooth® is a Trade Mark of the Bluetooth SIG registered for the benefit of its members

# Contents

Foreword	13

1	Scope	14

2	References	14

3	Definitions and Abbreviations	16

3.1	Definitions	16

3.2	Abbreviations	17

4	Functionality of Protocol	19

4.1	General	19

4.1.1	LPP Configuration	19

4.1.2	LPP Sessions and Transactions	20

4.1.3	LPP Position Methods	20

4.1.4	LPP Messages	20

4.2	Common LPP Session Procedure	21

4.3	LPP Transport	21

4.3.1	Transport Layer Requirements	21

4.3.2	LPP Duplicate Detection	21

4.3.3	LPP Acknowledgement	22

4.3.3.1	General	22

4.3.3.2	Procedure related to Acknowledgement	22

4.3.4	LPP Retransmission	23

4.3.4.1	General	23

4.3.4.2	Procedure related to Retransmission	23

4.3.5	LPP Message Segmentation	24

5	LPP Procedures	25

5.1	Procedures related to capability transfer	25

5.1.1	Capability Transfer procedure	25

5.1.2	Capability Indication procedure	25

5.1.3	Reception of LPP Request Capabilities	26

5.1.4	Transmission of LPP Provide Capabilities	26

5.2	Procedures related to Assistance Data Transfer	26

5.2.1	Assistance Data Transfer procedure	26

5.2.1a	Periodic Assistance Data Transfer procedure	27

5.2.1b	Periodic Assistance Data Transfer with Update procedure	29

5.2.2	Assistance Data Delivery procedure	29

5.2.2a	Periodic Assistance Data Delivery procedure	30

5.2.3	Transmission of LPP Request Assistance Data	32

5.2.4	Reception of LPP Provide Assistance Data	32

5.3	Procedures related to Location Information Transfer	32

5.3.1	Location Information Transfer procedure	32

5.3.2	Location Information Delivery procedure	33

5.3.3	Reception of Request Location Information	33

5.3.4	Transmission of Provide Location Information	34

5.4	Error Handling Procedures	34

5.4.1	General	34

5.4.2	Procedures related to Error Indication	34

5.4.3	LPP Error Detection	34

5.4.4	Reception of an LPP Error Message	35

5.5	Abort Procedure	36

5.5.1	General	36

5.5.2	Procedures related to Abort	36

5.5.3	Reception of an LPP Abort Message	36

6	Information Element Abstract Syntax Definition	36

6.1	General	36

6.2	LPP PDU Structure	37

–	LPP-PDU-Definitions	37

–	LPP-Message	37

–	LPP-MessageBody	38

–	LPP-TransactionID	38

6.3	Message Body IEs	39

–	RequestCapabilities	39

–	ProvideCapabilities	39

–	RequestAssistanceData	40

–	ProvideAssistanceData	40

–	RequestLocationInformation	41

–	ProvideLocationInformation	42

–	Abort	42

–	Error	43

6.4	Common IEs	43

6.4.1	Common Lower-Level IEs	43

–	AccessTypes	43

–	ARFCN-ValueEUTRA	43

–	ARFCN-ValueNR	44

–	ARFCN-ValueUTRA	44

–	CarrierFreq-NB	44

–	CellGlobalIdEUTRA-AndUTRA	45

–	CellGlobalIdGERAN	45

–	ECGI	46

–	Ellipsoid-Point	46

–	Ellipsoid-PointWithUncertaintyCircle	46

–	EllipsoidPointWithUncertaintyEllipse	46

–	EllipsoidPointWithAltitude	47

–	EllipsoidPointWithAltitudeAndUncertaintyEllipsoid	47

–	EllipsoidArc	47

–	EPDU-Sequence	47

–	FreqBandIndicatorNR	48

–	HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid	48

–	HA-EllipsoidPointWithScalableUncertaintyEllipse	49

–	HighAccuracyEllipsoidPointWithUncertaintyEllipse	49

–	HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid	49

–	HorizontalVelocity	49

–	HorizontalWithVerticalVelocity	50

–	HorizontalVelocityWithUncertainty	50

–	HorizontalWithVerticalVelocityAndUncertainty	50

–	LocationCoordinateTypes	50

–	NCGI	51

–	NR-PhysCellId	51

–	PeriodicAssistanceDataControlParameters	51

–	Polygon	52

–	PositioningModes	52

–	SegmentationInfo	52

–	VelocityTypes	53

6.4.2	Common Positioning	53

–	CommonIEsRequestCapabilities	53

–	CommonIEsProvideCapabilities	53

–	CommonIEsRequestAssistanceData	54

–	CommonIEsProvideAssistanceData	54

–	CommonIEsRequestLocationInformation	55

–	CommonIEsProvideLocationInformation	60

–	CommonIEsAbort	63

–	CommonIEsError	63

6.4.3	Common NR Positioning Information Elements	63

–	Area-ID-CellList	63

–	DL-PRS-ID-Info	64

–	LCS-GCS-TranslationParameter	64

–	LOS-NLOS-Indicator	65

–	NR-AdditionalPathList	65

–	NR-DL-PRS-AssistanceData	66

–	NR-DL-PRS-BeamInfo	69

–	NR-DL-PRS-Expected-LOS-NLOS-Assistance	70

–	NR-DL-PRS-Info	71

–	NR-DL-PRS-ProcessingCapability	75

–	NR-DL-PRS-QCL-ProcessingCapability	77

–	NR-DL-PRS-ResourceID	78

–	NR-DL-PRS-ResourcesCapability	78

–	NR-DL-PRS-ResourceSetID	79

–	NR-DL-PRS-TRP-TEG-Info	79

–	NR-On-Demand-DL-PRS-Configurations	80

–	NR-On-Demand-DL-PRS-Information	80

–	NR-On-Demand-DL-PRS-Request	82

–	NR-On-Demand-DL-PRS-Support	82

–	NR-PositionCalculationAssistance	83

–	NR-RTD-Info	83

–	NR-SelectedDL-PRS-IndexList	85

–	NR-SSB-Config	86

–	NR-TimeStamp	86

–	NR-TimingQuality	87

–	NR-TRP-BeamAntennaInfo	87

–	NR-TRP-LocationInfo	89

–	NR-UE-TEG-Capability	91

–	NR-UL-SRS-Capability	92

–	ReferencePoint	95

–	RelativeLocation	95

6.5	Positioning Method IEs	97

6.5.1	OTDOA Positioning	97

6.5.1.1	OTDOA Assistance Data	97

–	OTDOA-ProvideAssistanceData	97

6.5.1.2	OTDOA Assistance Data Elements	97

–	OTDOA-ReferenceCellInfo	97

–	PRS-Info	99

–	TDD-Config	101

–	OTDOA-NeighbourCellInfoList	101

–	OTDOA-ReferenceCellInfoNB	104

–	PRS-Info-NB	106

–	OTDOA-NeighbourCellInfoListNB	109

6.5.1.3	OTDOA Assistance Data Request	112

–	OTDOA-RequestAssistanceData	112

6.5.1.4	OTDOA Location Information	112

–	OTDOA-ProvideLocationInformation	112

6.5.1.5	OTDOA Location Information Elements	113

–	OTDOA-SignalMeasurementInformation	113

–	OTDOA-SignalMeasurementInformation-NB	116

–	OTDOA-MeasQuality	118

–	AdditionalPath	119

6.5.1.6	OTDOA Location Information Request	119

–	OTDOA-RequestLocationInformation	119

6.5.1.7	OTDOA Capability Information	120

–	OTDOA-ProvideCapabilities	120

6.5.1.8	OTDOA Capability Information Request	123

–	OTDOA-RequestCapabilities	123

6.5.1.9	OTDOA Error Elements	123

–	OTDOA-Error	123

–	OTDOA-LocationServerErrorCauses	123

–	OTDOA-TargetDeviceErrorCauses	124

6.5.2	A-GNSS Positioning	124

6.5.2.1	GNSS Assistance Data	124

–	A-GNSS-ProvideAssistanceData	124

–	GNSS-CommonAssistData	124

–	GNSS-GenericAssistData	125

–	GNSS-PeriodicAssistData	126

6.5.2.2	GNSS Assistance Data Elements	126

–	GNSS-ReferenceTime	126

–	GNSS-SystemTime	128

–	GPS-TOW-Assist	128

–	NetworkTime	129

–	GNSS-ReferenceLocation	132

–	GNSS-IonosphericModel	132

–	KlobucharModelParameter	132

–	KlobucharModel2Parameter	133

–	NeQuickModelParameter	134

–	GNSS-EarthOrientationParameters	134

–	GNSS-RTK-ReferenceStationInfo	135

–	GNSS-RTK-CommonObservationInfo	137

–	GNSS-RTK-AuxiliaryStationData	138

–	GNSS-SSR-CorrectionPoints	140

–	GNSS-Integrity-ServiceParameters	141

–	GNSS-Integrity-ServiceAlert	141

–	GNSS-TimeModelList	142

–	GNSS-DifferentialCorrections	143

–	GNSS-NavigationModel	146

–	StandardClockModelList	148

–	NAV-ClockModel	149

–	CNAV-ClockModel	149

–	GLONASS-ClockModel	150

–	SBAS-ClockModel	151

–	BDS-ClockModel	151

–	BDS-ClockModel2	152

–	NavIC-ClockModel	153

–	NavModelKeplerianSet	153

–	NavModelNAV-KeplerianSet	154

–	NavModelCNAV-KeplerianSet	156

–	NavModel-GLONASS-ECEF	157

–	NavModel-SBAS-ECEF	158

–	NavModel-BDS-KeplerianSet	159

–	NavModel-BDS-KeplerianSet2	160

–	NavModel-NavIC-KeplerianSet	163

–	GNSS-RealTimeIntegrity	164

–	GNSS-DataBitAssistance	165

–	GNSS-AcquisitionAssistance	166

–	GNSS-Almanac	169

–	AlmanacKeplerianSet	170

–	AlmanacNAV-KeplerianSet	171

–	AlmanacReducedKeplerianSet	172

–	AlmanacMidiAlmanacSet	173

–	AlmanacGLONASS-AlmanacSet	174

–	AlmanacECEF-SBAS-AlmanacSet	175

–	AlmanacBDS-AlmanacSet	176

–	AlmanacNavIC-AlmanacSet	177

–	GNSS-UTC-Model	178

–	UTC-ModelSet1	178

–	UTC-ModelSet2	179

–	UTC-ModelSet3	180

–	UTC-ModelSet4	180

–	UTC-ModelSet5	181

–	GNSS-AuxiliaryInformation	182

–	BDS-DifferentialCorrections	183

–	BDS-GridModelParameter	184

–	GNSS-RTK-Observations	184

–	GLO-RTK-BiasInformation	187

–	GNSS-RTK-MAC-CorrectionDifferences	188

–	GNSS-RTK-Residuals	190

–	GNSS-RTK-FKP-Gradients	191

–	GNSS-SSR-OrbitCorrections	193

–	GNSS-SSR-ClockCorrections	196

–	GNSS-SSR-CodeBias	198

–	GNSS-SSR-URA	199

–	GNSS-SSR-PhaseBias	200

–	GNSS-SSR-STEC-Correction	202

–	GNSS-SSR-GriddedCorrection	206

–	NavIC-DifferentialCorrections	211

–	NavIC-GridModelParameter	212

6.5.2.3	GNSS Assistance Data Request	213

–	A-GNSS-RequestAssistanceData	213

–	GNSS-CommonAssistDataReq	214

–	GNSS-GenericAssistDataReq	215

–	GNSS-PeriodicAssistDataReq	217

6.5.2.4	GNSS Assistance Data Request Elements	218

–	GNSS-ReferenceTimeReq	218

–	GNSS-ReferenceLocationReq	218

–	GNSS-IonosphericModelReq	218

–	GNSS-EarthOrientationParametersReq	219

–	GNSS-RTK-ReferenceStationInfoReq	219

–	GNSS-RTK-AuxiliaryStationDataReq	219

–	GNSS-SSR-CorrectionPointsReq	220

–	GNSS-Integrity-ServiceParametersReq	220

–	GNSS-Integrity-ServiceAlertReq	220

–	GNSS-TimeModelListReq	220

–	GNSS-DifferentialCorrectionsReq	221

–	GNSS-NavigationModelReq	221

–	GNSS-RealTimeIntegrityReq	223

–	GNSS-DataBitAssistanceReq	223

–	GNSS-AcquisitionAssistanceReq	224

–	GNSS-AlmanacReq	224

–	GNSS-UTC-ModelReq	225

–	GNSS-AuxiliaryInformationReq	225

–	BDS-DifferentialCorrectionsReq	225

–	BDS-GridModelReq	226

–	GNSS-RTK-ObservationsReq	226

–	GLO-RTK-BiasInformationReq	226

–	GNSS-RTK-MAC-CorrectionDifferencesReq	227

–	GNSS-RTK-ResidualsReq	227

–	GNSS-RTK-FKP-GradientsReq	228

–	GNSS-SSR-OrbitCorrectionsReq	228

–	GNSS-SSR-ClockCorrectionsReq	229

–	GNSS-SSR-CodeBiasReq	229

–	GNSS-SSR-URA-Req	229

–	GNSS-SSR-PhaseBiasReq	230

–	GNSS-SSR-STEC-CorrectionReq	230

–	GNSS-SSR-GriddedCorrectionReq	230

–	NavIC-DifferentialCorrectionsReq	231

–	NavIC-GridModelReq	231

6.5.2.5	GNSS Location Information	231

–	A-GNSS-ProvideLocationInformation	231

6.5.2.6	GNSS Location Information Elements	232

–	GNSS-SignalMeasurementInformation	232

–	MeasurementReferenceTime	232

–	GNSS-MeasurementList	234

–	GNSS-LocationInformation	238

6.5.2.7	GNSS Location Information Request	239

–	A-GNSS-RequestLocationInformation	239

6.5.2.8	GNSS Location Information Request Elements	239

–	GNSS-PositioningInstructions	239

6.5.2.9	GNSS Capability Information	240

–	A-GNSS-ProvideCapabilities	240

6.5.2.10	GNSS Capability Information Elements	242

–	GNSS-CommonAssistanceDataSupport	242

–	GNSS-ReferenceTimeSupport	243

–	GNSS-ReferenceLocationSupport	243

–	GNSS-IonosphericModelSupport	244

–	GNSS-EarthOrientationParametersSupport	244

–	GNSS-RTK-ReferenceStationInfoSupport	244

–	GNSS-RTK-AuxiliaryStationDataSupport	244

–	GNSS-Integrity-ServiceParametersSupport	244

–	GNSS-Integrity-ServiceAlertSupport	244

–	GNSS-GenericAssistanceDataSupport	245

–	GNSS-TimeModelListSupport	247

–	GNSS-DifferentialCorrectionSupport	247

–	GNSS-NavigationModelSupport	247

–	GNSS-RealTimeIntegritySupport	248

–	GNSS-DataBitAssistanceSupport	248

–	GNSS-AcquisitionAssistanceSupport	248

–	GNSS-AlmanacSupport	249

–	GNSS-UTC-ModelSupport	249

–	GNSS-AuxiliaryInformationSupport	250

–	BDS-DifferentialCorrectionsSupport	250

–	BDS-GridModelSupport	250

–	GNSS-RTK-ObservationsSupport	250

–	GLO-RTK-BiasInformationSupport	250

–	GNSS-RTK-MAC-CorrectionDifferencesSupport	251

–	GNSS-RTK-ResidualsSupport	251

–	GNSS-RTK-FKP-GradientsSupport	251

–	GNSS-SSR-OrbitCorrectionsSupport	251

–	GNSS-SSR-ClockCorrectionsSupport	252

–	GNSS-SSR-URA-Support	252

–	GNSS-SSR-STEC-CorrectionSupport	253

–	GNSS-SSR-GriddedCorrectionSupport	253

–	NavIC-DifferentialCorrectionsSupport	253

–	NavIC-GridModelSupport	254

6.5.2.11	GNSS Capability Information Request	254

–	A-GNSS-RequestCapabilities	254

6.5.2.12	GNSS Error Elements	254

–	A-GNSS-Error	254

–	GNSS-LocationServerErrorCauses	255

–	GNSS-TargetDeviceErrorCauses	255

6.5.2.13	Common GNSS Information Elements	256

–	GNSS-FrequencyID	256

–	GNSS-ID	256

–	GNSS-ID-Bitmap	257

–	GNSS-Link-CombinationsList	257

–	GNSS-NavListInfo	257

–	GNSS-NetworkID	257

–	GNSS-PeriodicControlParam	258

–	GNSS-ReferenceStationID	258

–	GNSS-SignalID	258

–	GNSS-SignalIDs	260

–	GNSS-SubNetworkID	261

–	SBAS-ID	262

–	SBAS-IDs	262

–	SV-ID	262

6.5.3	Enhanced Cell ID Positioning	263

6.5.3.1	E-CID Location Information	263

–	ECID-ProvideLocationInformation	263

6.5.3.2	E-CID Location Information Elements	263

–	ECID-SignalMeasurementInformation	263

6.5.3.3	E-CID Location Information Request	265

–	ECID-RequestLocationInformation	265

6.5.3.4	E-CID Capability Information	265

–	ECID-ProvideCapabilities	265

6.5.3.5	E-CID Capability Information Request	266

–	ECID-RequestCapabilities	266

6.5.3.6	E-CID Error Elements	266

–	ECID-Error	266

–	ECID-LocationServerErrorCauses	267

–	ECID-TargetDeviceErrorCauses	267

6.5.4	Terrestrial Beacon System Positioning	267

6.5.4.1	TBS Location Information	267

–	TBS-ProvideLocationInformation	267

6.5.4.2	TBS Location Information Elements	268

–	TBS-MeasurementInformation	268

–	MBS-BeaconMeasList	268

6.5.4.3	TBS Location Information Request	269

–	TBS-RequestLocationInformation	269

6.5.4.4	TBS Capability Information	269

–	TBS-ProvideCapabilities	269

-	MBS-AssistanceDataSupportList	270

6.5.4.5	TBS Capability Information Request	271

–	TBS-RequestCapabilities	271

6.5.4.6	TBS Error Elements	271

–	TBS-Error	271

–	TBS-LocationServerErrorCauses	271

–	TBS-TargetDeviceErrorCauses	271

6.5.4.7	TBS Assistance Data	272

–	TBS-ProvideAssistanceData	272

6.5.4.8	TBS Assistance Data Elements	272

–	TBS-AssistanceDataList	272

–	MBS-AlmanacAssistance	272

–	MBS-AcquisitionAssistance	273

6.5.4.9	TBS Assistance Data Request	273

–	TBS-RequestAssistanceData	273

6.5.5	Sensor based Positioning	274

6.5.5.0	Introduction	274

6.5.5.1	Sensor Location Information	274

–	Sensor-ProvideLocationInformation	274

6.5.5.2	Sensor Location Information Elements	274

–	Sensor-MeasurementInformation	274

–	Sensor-MotionInformation	275

6.5.5.3	Sensor Location Information Request	276

–	Sensor-RequestLocationInformation	276

6.5.5.4	Sensor Capability Information	277

–	Sensor-ProvideCapabilities	277

6.5.5.5	Sensor Capability Information Request	278

–	Sensor-RequestCapabilities	278

6.5.5.6	Sensor Error Elements	278

–	Sensor-Error	278

–	Sensor-LocationServerErrorCauses	279

–	Sensor-TargetDeviceErrorCauses	279

6.5.5.7	Sensor Assistance Data	279

–	Sensor-ProvideAssistanceData	279

6.5.5.8	Sensor Assistance Data Elements	279

–	Sensor-AssistanceDataList	279

6.5.5.9	Sensor Assistance Data Request	281

–	Sensor-RequestAssistanceData	281

6.5.6	WLAN-based Positioning	281

6.5.6.1	WLAN Location Information	282

–	WLAN-ProvideLocationInformation	282

6.5.6.2	WLAN Location Information Elements	282

–	WLAN-MeasurementInformation	282

6.5.6.3	WLAN Location Information Request	283

–	WLAN-RequestLocationInformation	283

6.5.6.4	WLAN Capability Information	284

–	WLAN-ProvideCapabilities	284

6.5.6.5	WLAN Capability Information Request	285

–	WLAN-RequestCapabilities	285

6.5.6.6	WLAN Error Elements	285

–	WLAN-Error	285

–	WLAN-LocationServerErrorCauses	286

–	WLAN-TargetDeviceErrorCauses	286

6.5.7	Bluetooth-based Positioning	290

6.5.7.1	Bluetooth Location Information	290

–	BT-ProvideLocationInformation	290

6.5.7.2	Bluetooth Location Information Elements	290

–	BT-MeasurementInformation	290

6.5.7.3	Bluetooth Location Information Request	290

–	BT-RequestLocationInformation	290

6.5.7.4	Bluetooth Capability Information	291

–	BT-ProvideCapabilities	291

6.5.7.5	Bluetooth Capability Information Request	292

–	BT-RequestCapabilities	292

6.5.7.6	BT Error Elements	292

–	BT-Error	292

–	BT-LocationServerErrorCauses	292

–	BT-TargetDeviceErrorCauses	293

6.5.8	NR UL Positioning	293

6.5.8.1	NR UL Capability Information	293

–	NR-UL-ProvideCapabilities	293

6.5.8.2	NR UL Capability Information Request	294

–	NR-UL-RequestCapabilities	294

6.5.9	NR E-CID Positioning	294

6.5.9.1	NR E-CID Location Information	294

–	NR-ECID-ProvideLocationInformation	294

6.5.9.2	NR E-CID Location Information Elements	294

–	NR-ECID-SignalMeasurementInformation	294

6.5.9.3	NR E-CID Location Information Request	295

–	NR-ECID-RequestLocationInformation	295

6.5.9.4	NR E-CID Capability Information	296

–	NR-ECID-ProvideCapabilities	296

6.5.9.5	NR E-CID Capability Information Request	296

–	NR-ECID-RequestCapabilities	296

6.5.9.6	NR E-CID Error Elements	297

–	NR-ECID-Error	297

–	NR-ECID-LocationServerErrorCauses	297

–	NR-ECID-TargetDeviceErrorCauses	297

6.5.10	NR DL-TDOA Positioning	298

6.5.10.1	NR DL-TDOA Assistance Data	298

–	NR-DL-TDOA-ProvideAssistanceData	298

6.5.10.2	NR DL-TDOA Assistance Data Request	299

–	NR-DL-TDOA-RequestAssistanceData	299

6.5.10.3	NR DL-TDOA Location Information	300

–	NR-DL-TDOA-ProvideLocationInformation	300

6.5.10.4	NR DL-TDOA Location Information Elements	300

–	NR-DL-TDOA-SignalMeasurementInformation	300

–	NR-DL-TDOA-LocationInformation	303

6.5.10.5	NR DL-TDOA Location Information Request	303

–	NR-DL-TDOA-RequestLocationInformation	303

6.5.10.6	NR DL-TDOA Capability Information	304

–	NR-DL-TDOA-ProvideCapabilities	304

6.5.10.6a	NR DL-TDOA Capability Information Elements	306

–	NR-DL-TDOA-MeasurementCapability	306

6.5.10.7	NR DL-TDOA Capability Information Request	307

–	NR-DL-TDOA-RequestCapabilities	307

6.5.10.8	NR DL-TDOA Error Elements	307

–	NR-DL-TDOA-Error	307

–	NR-DL-TDOA-LocationServerErrorCauses	307

–	NR-DL-TDOA-TargetDeviceErrorCauses	308

6.5.11	NR DL-AoD Positioning	308

6.5.11.1	NR DL-AoD Assistance Data	308

–	NR-DL-AoD-ProvideAssistanceData	308

6.5.11.2	NR DL-AoD Assistance Data Request	309

–	NR-DL-AoD-RequestAssistanceData	309

6.5.11.3	NR DL-AoD Location Information	310

–	NR-DL-AoD-ProvideLocationInformation	310

6.5.11.4	NR DL-AoD Location Information Elements	310

–	NR-DL-AoD-SignalMeasurementInformation	310

–	NR-DL-AoD-LocationInformation	312

6.5.11.5	NR DL-AoD Location Information Request	313

–	NR-DL-AoD-RequestLocationInformation	313

6.5.11.6	NR DL-AoD Capability Information	313

–	NR-DL-AoD-ProvideCapabilities	313

6.5.11.6a	NR DL-AoD Capability Information Elements	316

–	NR-DL-AoD-MeasurementCapability	316

6.5.11.7	NR DL-AoD Capability Information Request	316

–	NR-DL-AoD-RequestCapabilities	316

6.5.11.8	NR DL-AoD Error Elements	317

–	NR-DL-AoD-Error	317

–	NR-DL-AoD-LocationServerErrorCauses	317

–	NR-DL-AoD-TargetDeviceErrorCauses	317

6.5.12	NR Multi-RTT Positioning	318

6.5.12.1	NR Multi-RTT Assistance Data	318

–	NR-Multi-RTT-ProvideAssistanceData	318

6.5.12.2	NR Multi-RTT Assistance Data Request	318

–	NR-Multi-RTT-RequestAssistanceData	318

6.5.12.3	NR Multi-RTT Location Information	319

–	NR-Multi-RTT-ProvideLocationInformation	319

6.5.12.4	NR Multi-RTT Location Information Elements	319

–	NR-Multi-RTT-SignalMeasurementInformation	319

6.5.12.5	NR Multi-RTT Location Information Request	323

–	NR-Multi-RTT-RequestLocationInformation	323

6.5.12.6	NR Multi-RTT Capability Information	325

–	NR-Multi-RTT-ProvideCapabilities	325

6.5.12.6a	NR Multi-RTT Capability Information Elements	326

–	NR-Multi-RTT-MeasurementCapability	326

6.5.12.7	NR Multi-RTT Capability Information Request	327

–	NR-Multi-RTT-RequestCapabilities	327

6.5.12.8	NR Multi-RTT Error Elements	327

–	NR-Multi-RTT-Error	327

–	NR-Multi-RTT-LocationServerErrorCauses	328

–	NR-Multi-RTT-TargetDeviceErrorCauses	328

6.6	Multiplicity and type constraint values	328

–	Multiplicity and type constraint definitions	328

–	End of LPP-PDU-Definitions	329

7	Broadcast of assistance data	330

7.1	General	330

7.2	Mapping of posSibType to assistance data element	330

7.3	Procedures related to broadcast information elements	331

7.4	Broadcast information elements	333

7.4.1	Basic production	333

–	LPP-Broadcast-Definitions	333

7.4.2	Element definitions	333

–	AssistanceDataSIBelement	333

–	OTDOA-UE-Assisted	334

–	NR-UEB-TRP-LocationData	335

–	NR-UEB-TRP-RTD-Info	335

–	End of LPP-Broadcast-Definitions	335

7.5	Broadcast ciphering (informative)	335

Annex A (informative):	Change History	338

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document contains the definition of the LTE Positioning Protocol (LPP) for the radio access technologies E-UTRA/LTE and NR.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 36.305: "Stage 2 functional specification of User Equipment (UE) positioning in E-UTRAN".

[3]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)".

[4]	IS-GPS-200, Revision D, Navstar GPS Space Segment/Navigation User Interfaces, March 7th, 2006.

[5]	IS-GPS-705, Navstar GPS Space Segment/User Segment L5 Interfaces, September 22, 2005.

[6]	IS-GPS-800, Navstar GPS Space Segment/User Segment L1C Interfaces, September 4, 2008.

[7]	IS-QZSS, Quasi Zenith Satellite System Navigation Service Interface Specifications for QZSS, Ver.1.1, July 31, 2009.

[8]	Galileo OS Signal in Space ICD (OS SIS ICD), Issue 1.2, February 2014, European Union.

[9]	Global Navigation Satellite System GLONASS Interface Control Document, Version 5.1, 2008.

[10]	Specification for the Wide Area Augmentation System (WAAS), US Department of Transportation, Federal Aviation Administration, DTFA01-96-C-00025, 2001.

[11]	RTCM-SC104, RTCM Recommended Standards for Differential GNSS Service (v.2.3), August 20, 2001.

[12]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); "Radio Resource Control (RRC); Protocol specification".

[13]	3GPP TS 25.331: "Radio Resource Control (RRC); Protocol Specification".

[14]	3GPP TS 44.031: "Location Services (LCS); Mobile Station (MS) - Serving Mobile Location Centre (SMLC) Radio Resource LCS Protocol (RRLP)".

[15]	3GPP TS 23.032: "Universal Geographical Area Description (GAD)".

[16]	3GPP TS 36.211: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical Channels and Modulation".

[17]	3GPP TS 36.214: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer – Measurements".

[18]	3GPP TS 36.133: "Evolved Universal Terrestrial Radio Access (E-UTRA); Requirements for support of radio resource management".

[19]	3GPP TS 23.003: "Numbering, addressing and identification".

[20]	OMA-TS-LPPe-V1_0, LPP Extensions Specification, Open Mobile Alliance.

[21]	3GPP TS 36.101: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception".

[22]	ITU-T Recommendation X.691 (07/2002) "Information technology - ASN.1 encoding rules: Specification of Packed Encoding Rules (PER)" (Same as the ISO/IEC International Standard 8825-2).

[23]	BDS-SIS-ICD-B1I-3.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1I (Version 3.0)", February, 2019.

[24]	ATIS-0500027: "Recommendations for Establishing Wide Scale Indoor Location Performance", May 2015.

[25]	Bluetooth Special Interest Group: "Bluetooth Core Specification v4.2", December 2014.

[26]	IEEE 802.11, Part 11: "Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications".

[27]	IETF RFC 6225, "Dynamic Host Configuration Protocol Options for Coordinate-Based Location Configuration Information", July 2011.

[28]	3GPP TS 36.213: "Evolved Universal Terrestrial Radio Access (E-UTRA); Physical layer procedures".

[29]	"Earth Gravitational Model 96 (EGM96)", National Geospatial-Intelligence Agency, NASA.

[30]	RTCM Standard 10403.3: "Differential GNSS (Global Navigation Satellite Systems) Services" – Version 3, October 7, 2016.

[31]	IGS ANTEX: "The Antenna Exchanged Format" – version 1.4, September 15, 2010.

[32]	Federal Information Processing Standards Publication 197, "Specification for the ADVANCED ENCRYPTION STANDARD (AES)", November 26, 2001.

[33]	NIST Special Publication 800-38A, "Recommendation for Block Cipher Modes of Operation Methods and Techniques", 2001.

[34]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[35]	3GPP TS 38.331: "NR; Radio Resource Control (RRC); Protocol specification".

[36]	3GPP TS 38.215: "NR; Physical layer measurements".

[37]	3GPP TS 38.101-1: "NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone".

[38]	IRNSS Signal-In-Space (SPS) Interface Control Document (ICD) for standard positioning service version 1.1, Aug 2017.

[39]	BDS-SIS-ICD-B1C-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B1C (Version 1.0)", December, 2017.

[40]	3GPP TS 38.305: "NG Radio Access Network (NG-RAN); Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

[41]	3GPP TS 38.211: "3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical channels and modulation".

[42]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[43]	IS-QZSS-L6-001, Quasi-Zenith Satellite System Interface Specification – Centimetre Level Augmentation Service, Cabinet Office, November 5, 2018.

[44]	3GPP TR 38.901: "Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz".

[45]	3GPP TS 38.214: "NR; Physical layer procedures for data".

[46]	3GPP TS 38.133: "NR; Requirements for support of radio resource management".

[47]	3GPP TS 38.300: "NR; NR and NG-RAN Overall Description; Stage 2".

[48]	3GPP TS 38.213: "NR; Physical layer procedures for control".

[49]	BDS-SIS-ICD-B2a-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B2a (Version 1.0)", December, 2017.

[50]	BDS-SIS-ICD-B3I-1.0: "BeiDou Navigation Satellite System Signal In Space Interface Control Document Open Service Signal B3I (Version 1.0)", December, 2017.

# 3	Definitions and Abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1], TS 36.305 [2], TS 23.271 [3], 38.305 [40] and TS 23.273 [42] apply. Other definitions are provided below.

Anchor carrier: In NB-IoT, a carrier where the UE assumes that NPSS/NSSS/NPBCH/SIB-NB for FDD or NPSS/NSSS/NPBCH for TDD are transmitted.

Location Server: a physical or logical entity (e.g., E-SMLC, SUPL SLP, or LMF) that manages positioning for a target device by obtaining measurements and other location information from one or more positioning units and providing assistance data to positioning units to help determine this. A Location Server may also compute or verify the final location estimate.

NB-IoT: NB-IoT allows access to network services via E-UTRA with a channel bandwidth limited to 200 kHz.

Observed Time Difference Of Arrival (OTDOA): The time interval that is observed by a target device between the reception of downlink signals from two different TPs. If a signal from TP 1 is received at the moment t1, and a signal from TP 2 is received at the moment t2, the OTDOA is t2 – t1.

PRS-only TP: A TP which only transmits PRS signals or DL-PRS for PRS-based TBS positioning and is not associated with a cell.

Reference Source: a physical entity or part of a physical entity that provides signals (e.g., RF, acoustic, infra-red) that can be measured (e.g., by a Target Device) in order to obtain the location of a Target Device.

Relative Time Difference (RTD): The relative time difference between a TRP i and a TRP j, is defined as tj – ti, where ti and tj are defined as the time when TRP i and j transmit the start of one subframe respectively.

Target Device: the device that is being positioned (e.g., UE or SUPL SET).

Transmission Point (TP): A set of geographically co-located transmit antennas (e.g. antenna array (with one or more antenna elements)) for one cell, part of one cell or one PRS-only TP. Transmission Points can include base station (eNodeB) antennas, remote radio heads, a remote antenna of a base station, an antenna of a PRS-only TP, etc. One cell can be formed by one or multiple transmission points. For a homogeneous deployment, each transmission point may correspond to one cell.

Transmission-Reception Point (TRP): A set of geographically co-located antennas (e.g. antenna array (with one or more antenna elements)) supporting TP and/or RP functionality.

TRP Tx Timing Error Group (TRP Tx TEG): A TRP Tx TEG is associated with the transmissions of one or more DL-PRS Resources, which have the Tx timing error difference within a certain margin.

UE Rx Timing Error Group (UE Rx TEG): A UE Rx TEG is associated with one or more DL timing measurements, which have the Rx timing error difference within a certain margin.

UE RxTx Timing Error Group (UE RxTx TEG): A UE RxTx TEG is associated with one or more UE Rx-Tx time difference measurements, which have the 'Rx timing errors+Tx timing errors' difference within a certain margin.

UE Tx Timing Error Group (UE Tx TEG): A UE Tx TEG is associated with the transmissions of one or more UL SRS Resources for Positioning, which have the Tx timing error difference within a certain margin.

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply.

ADR	Accumulated Delta-Range

A-GNSS	Assisted-GNSS

AoA	Angle-of-Arrival

AoD	Angle-of-Departure

AP	Access Point

ARFCN	Absolute Radio Frequency Channel Number

ARP	Antenna Reference Point

BDS	BeiDou Navigation Satellite System

BIPM	Bureau International des Poids et Mesures (International Bureau of Weights and Measures)

BSSID	Basic Service Set Identifier

BTS	Base Transceiver Station (GERAN)

CID	Cell-ID (positioning method)

CNAV	Civil Navigation

CRS	Cell-specific Reference Signals

DL-AoD	Downlink Angle-of-Departure

DL PRS-RSRPP	DL PRS Reference Signal Received Path Power

DL-TDOA	Downlink Time Difference Of Arrival

ECEF	Earth-Centered, Earth-Fixed

ECGI	Evolved Cell Global Identifier

ECI	Earth-Centered-Inertial

E-CID	Enhanced Cell-ID (positioning method)

EGNOS	European Geostationary Navigation Overlay Service

E-SMLC	Enhanced Serving Mobile Location Centre

E-UTRA	Evolved Universal Terrestrial Radio Access

E-UTRAN	Evolved Universal Terrestrial Radio Access Network

EOP	Earth Orientation Parameters

EPDU	External Protocol Data Unit

FDMA	Frequency Division Multiple Access

FEC	Forward Error Correction

FKP	(German) Flächen-Korrektur-Parameter (area correction parameter)

FTA	Fine Time Assistance

GAGAN	GPS Aided Geo Augmented Navigation

GLONASS	GLObal'naya NAvigatsionnaya Sputnikovaya Sistema (Engl.: Global Navigation Satellite System)

GNSS	Global Navigation Satellite System

GPS	Global Positioning System

HA GNSS	High-Accuracy GNSS (RTK, PPP)

ICD	Interface Control Document

IGS	International GNSS Service

IOD	Issue of Data

IRNSS	Indian Regional Navigation Satellite System

IS	Interface Specification

LLA	Latitude Longitude Altitude

LMF	Location Management Function

LOS	Line-of-Sight

LPP	LTE Positioning Protocol

LPPa	LTE Positioning Protocol Annex

LSB	Least Significant Bit

MAC	Master Auxiliary Concept

MBS	Metropolitan Beacon System

MO-LR	Mobile Originated Location Request

MSAS	Multi-functional Satellite Augmentation System

MSB	Most Significant Bit

msd	mean solar day

MT-LR	Mobile Terminated Location Request

Multi-RTT	Multiple-Round Trip Time

NAV	Navigation

NavIC	NAVigation with Indian Constellation

NB-IoT	NarrowBand Internet of Things

NCGI	NR Cell Global Identifier

NICT	National Institute of Information and Communications Technology

NI-LR	Network Induced Location Request

NLOS	Non-Line-of-Sight

NPRS	Narrowband Positioning Reference Signals

NR	NR Radio Access

NRSRP	Narrowband Reference Signal Received Power

NRSRQ	Narrowband Reference Signal Received Quality

NTSC	National Time Service Center of Chinese Academy of Sciences

OSR	Observation Space Representation

OTDOA	Observed Time Difference Of Arrival

PBCH	Physical Broadcast Channel

PDU	Protocol Data Unit

PPP	Precise Point Positioning

PRB	Physical Resource Block

PRC	Pseudo-Range Correction

PRS	Positioning Reference Signals

posSIB	Positioning System Information Block

PZ-90	Parametry Zemli 1990 Goda – Parameters of the Earth Year 1990

QZS	Quasi Zenith Satellite

QZSS	Quasi-Zenith Satellite System

QZST	Quasi-Zenith System Time

RF	Radio Frequency

RP	Reception Point

RRC	Range-Rate Correction

Radio Resource Control

RSRP	Reference Signal Received Power

RSRQ	Reference Signal Received Quality

RSTD	Reference Signal Time Difference

RTK	Real-Time Kinematic

RTT	Round Trip Time

RU	Russia

SBAS	Space Based Augmentation System

SET	SUPL Enabled Terminal

SFN	System Frame Number

SLP	SUPL Location Platform

SRS	Sounding Reference Signal

SS	Synchronization Signal

SSB	Synchronization Signal Block, SS/PBCH Block

SSID	Service Set Identifier

SSR	State Space Representation

STEC	Slant TEC

SUPL	Secure User Plane Location

SV	Space Vehicle

TB	Terrestrial Beacon

TBS	Terrestrial Beacon System

TEC	Total Electron Content

TECU	TEC Units

TEG	Timing Error Group

TLM	Telemetry

TOA	Time Of Arrival

TOD	Time Of Day

TOW	Time Of Week

TP	Transmission Point

TRP	Transmission-Reception Point

UDRE	User Differential Range Error

ULP	User Plane Location Protocol

URA	User Range Accuracy

USNO	US Naval Observatory

UT1	Universal Time No.1

UTC	Coordinated Universal Time

WAAS	Wide Area Augmentation System

WGS-84	World Geodetic System 1984

WLAN	Wireless Local Area Network

# 4	Functionality of Protocol

## 4.1	General

### 4.1.1	LPP Configuration

LPP is used point-to-point between a location server (E-SMLC, LMF or SLP) and a target device (UE or SET) in order to position the target device using position-related measurements obtained by one or more reference sources. Figure 4.1.1-1 shows the configuration as applied to the control- and user-plane location solutions for E-UTRAN and NG-RAN (as defined in TS 36.305 [2], TS 38.305 [40], TS 23.273 [42] and TS 23.271 [3]).

NB-IoT is a non-backward compatible variant of E-UTRAN supporting a reduced set of functionalities. In this specification, procedures and messages specified for the UE equally apply to the UE in NB-IoT.

Figure 4.1.1-1: LPP Configuration for Control- and User-Plane Positioning in E-UTRAN or NG-RAN

### 4.1.2	LPP Sessions and Transactions

An LPP session is used between a Location Server and the target device in order to obtain location related measurements or a location estimate or to transfer assistance data. A single LPP session is used to support a single location request (e.g., for a single MT-LR, MO-LR or NI-LR). Multiple LPP sessions can be used between the same endpoints to support multiple different location requests (as required by TS 23.271 [3]). Each LPP session comprises one or more LPP transactions, with each LPP transaction performing a single operation (capability exchange, assistance data transfer, or location information transfer). In E-UTRAN and NG-RAN, the LPP transactions are realized as LPP procedures. The instigator of an LPP session will always instigate the first LPP transaction, but subsequent transactions may be instigated by either end. LPP transactions within a session may occur serially or in parallel. LPP transactions are indicated at the LPP protocol level with a transaction ID in order to associate messages with one another (e.g., request and response).

Messages within a transaction are linked by a common transaction identifier.

### 4.1.3	LPP Position Methods

Internal LPP positioning methods and associated signalling content are defined in this specification.

This version of the specification defines OTDOA (based on LTE signals), A-GNSS, E-CID (based on LTE signals), Sensor, TBS, WLAN, Bluetooth, NR E-CID, NR DL-TDOA, NR DL-AoD and NR Multi-RTT positioning methods.

### 4.1.4	LPP Messages

Each LPP transaction involves the exchange of one or more LPP messages between the location server and the target device. The general format of an LPP message consists of a set of common fields followed by a body. The body (which may be empty) contains information specific to a particular message type. Each message type contains information specific to one or more positioning methods and/or information common to all positioning methods.

The common fields are as follows:

| Field | Role |
| --- | --- |
| Transaction ID | Identify messages belonging to the same transaction |
| Transaction End Flag | Indicate when a transaction (e.g. one with periodic responses) has ended |
| Sequence Number | Enable detection of a duplicate LPP message at a receiver |
| Acknowledgement | Enable an acknowledgement to be requested and/or returned for any LPP message |

NOTE:	Use of the Transaction ID and Transaction End fields conform to the procedures in clause 5 and are independent of the means used to transport LPP messages (e.g., whether using a NAS MO-LR Request, NAS Generic Transport or user-plane solution).

The following message types are defined:

-	Request Capabilities;

-	Provide Capabilities;

-	Request Assistance Data;

-	Provide Assistance Data;

-	Request Location Information;

-	Provide Location Information;

-	Abort;

-	Error.

## 4.2	Common LPP Session Procedure

The purpose of this procedure is to support an LPP session comprising a sequence of LPP transactions. The procedure is described in Figure 4.2-1.

Figure 4.2-1 LPP Session Procedure

1.	Endpoint A, which may be either the target or the server, initiates an LPP session by sending an LPP message for an initial LPP transaction j to the other endpoint B (which has an opposite role to A).

2.	Endpoints A and B may exchange further messages to continue the transaction started in step 1.

3.	Either endpoint may instigate further transactions by sending additional LPP messages.

4.	A session is terminated by a final transaction N in which LPP messages will be exchanged between the two endpoints.

Within each transaction, all constituent messages shall contain the same transaction identifier. The last message sent in each transaction shall have the IE endTransaction set to TRUE. Transactions that occur in parallel shall use different transaction IDs; transaction IDs for completed transactions may be reused at any time after the final message of the previous transaction with the same ID is known to have been received.

## 4.3	LPP Transport

### 4.3.1	Transport Layer Requirements

LPP requires reliable, in-sequence delivery of LPP messages from the underlying transport layers. This clause describes the transport capabilities that are available within LPP. A UE implementing LPP for the control-plane solution shall support LPP reliable transport (including all three of duplicate detection, acknowledgement, and retransmission).

LPP reliable transport functionality is not used in the user-plane solution.

The following requirements in clauses 4.3.2, 4.3.3, and 4.3.4 for LPP reliable transport apply only when the capability is supported.

### 4.3.2	LPP Duplicate Detection

A sender shall include a sequence number in all LPP messages sent for a particular location session. The sequence number shall be distinct for different LPP messages sent in the same direction in the same location session (e.g., may start at zero in the first LPP message and increase monotonically in each succeeding LPP message). Sequence numbers used in the uplink and downlink are independent (e.g., can be the same).

A receiver shall record the most recent received sequence number for each location session. If a message is received carrying the same sequence number as that last received for the associated location session, it shall be discarded. Otherwise (i.e., if the sequence number is different or if no sequence number was previously received or if no sequence number is included), the message shall be processed.

Sending and receiving sequence numbers shall be deleted in a server when the associated location session is terminated and shall be deleted in a target device when there has been no activity for a particular location session for 10 minutes.

NOTE:	For LPP control-plane use, a target device can be aware of a location session from information provided at the NAS level for downlink transport of an LPP message.

### 4.3.3	LPP Acknowledgement

#### 4.3.3.1	General

Each LPP message may carry an acknowledgement request and/or an acknowledgement indicator. A LPP message including an acknowledgement request (i.e., that include the IE ackRequested set to TRUE) shall also include a sequence number. Upon reception of an LPP message which includes the IE ackRequested set to TRUE, a receiver returns an LPP message with an acknowledgement response (i.e., that includes the ackIndicator IE set to the same sequence number of the message being acknowledged). An acknowledgement response may contain no LPP message body (in which case only the sequence number being acknowledged is significant); alternatively, the acknowledgement may be sent in an LPP message along with an LPP message body. An acknowledgement is returned for each received LPP message that requested an acknowledgement including any duplicate(s). Once a sender receives an acknowledgement for an LPP message, and provided any included sequence number is matching, it is permitted to send the next LPP message. No message reordering is needed at the receiver since this stop-and-wait method of sending ensures that messages normally arrive in the correct order.

When an LPP message is transported via a NAS MO-LR request, the message does not request an acknowledgement.

#### 4.3.3.2	Procedure related to Acknowledgement

Figure 4.3.3.2-1 shows the procedure related to acknowledgement.

Figure 4.3.3.2-1: LPP Acknowledgement procedure

1.	Endpoint A sends an LPP message N to Endpoint B which includes the IE ackRequested set to TRUE and a sequence number.

2.	If LPP message N is received and Endpoint B is able to decode the ackRequested value and sequence number, Endpoint B shall return an acknowledgement for message N. The acknowledgement shall contain the IE ackIndicator set to the same sequence number as that in message N.

3.	When the acknowledgement for LPP message N is received and provided the included ackIndicator IE matches the sequence number sent in message N, Endpoint A sends the next LPP message N+1 to Endpoint B when this message is available.

### 4.3.4	LPP Retransmission

#### 4.3.4.1	General

This capability builds on the acknowledgement and duplicate detection capabilities. When an LPP message which requires acknowledgement is sent and not acknowledged, it is resent by the sender following a timeout period up to three times. If still unacknowledged after that, the sender aborts all LPP activity for the associated session. The timeout period is determined by the sender implementation but shall not be less than a minimum value of 250 ms.

In addition, for NB-IoT the timeout period may be determined by the sender implementation based on e.g., the coverage level of the UE.

#### 4.3.4.2	Procedure related to Retransmission

Figure 4.3.4.2-1 shows the procedure related to retransmission when combined with acknowledgement and duplicate detection.

Figure 4.3.4.2-1: LPP Retransmission procedure

1.	Endpoint A sends an LPP message N to Endpoint B for a particular location session and includes a request for acknowledgement along with a sequence number.

2.	If LPP message N is received and Endpoint B is able to decode the ackRequested value and sequence number (regardless of whether the message body can be correctly decoded), Endpoint B shall return an acknowledgement for message N. If the acknowledgement is received by Endpoint A (such that the acknowledged message can be identified and sequence numbers are matching), Endpoint A skips steps 3 and 4.

3.	If the acknowledgement in step 2 is not received after a timeout period, Endpoint A shall retransmit LPP message N and shall include the same sequence number as in step 1.

4.	If LPP message N in step 3 is received and Endpoint B is able to decode the ackRequested value and sequence number (regardless of whether the message body can be correctly decoded and whether or not the message is considered a duplicate), Endpoint B shall return an acknowledgement. Steps 3 may be repeated one or more times if the acknowledgement in step 4 is not received after a timeout period by Endpoint A. If the acknowledgement in step 4 is still not received after sending three retransmissions, Endpoint A shall abort all procedures and activity associated with LPP support for the particular location session.

5.	Once an acknowledgement in step 2 or step 4 is received, Endpoint A sends the next LPP message N+1 for the location session to Endpoint B when this message is available.

### 4.3.5	LPP Message Segmentation

An LPP message body may be sent in several shorter LPP messages instead of one long LPP message to deliver a large amount of information (e.g., in case the LPP message size exceeds the maximum message size supported by lower layers). When a sender employs LPP message segmentation, the sender shall include the IE SegmentationInfo in each LPP message segment. The sender shall indicate in all but the final message segment that more messages are on the way.

When a receiver receives an LPP message indicating that more messages are on the way, the receiver may store the LPP message. If the receiver receives a subsequent LPP message for the same session and transaction ID, the receiver shall assume that the new LPP message continues the segmentation of the earlier message and may store the new message if the new message indicates that more messages are on the way. If the new message indicates that no more messages are on the way, the receiver shall assume that message segmentation is complete and shall process the new message and any stored message segments for the same session and transaction ID.

The reliable transport rules specified in clause 4.3.2, 4.3.3, and 4.3.4 apply to each individual LPP message segment, independently of the value of the IE SegmentationInfo.

The rules for setting the common fields of the LPP message specified in clause 4.1.4 (Transaction ID, Transaction End Flag, Sequence Number, Acknowledgment) apply to each individual LPP message segment, independently of the value of the IE SegmentationInfo.

Figure 4.3.5-1: LPP Message Segmentation procedure

1.	Endpoint A sends an LPP message to Endpoint B for a particular location session and includes the IE SegmentationInfo set to moreMessagesOnTheWay to indicate that this is one of many LPP message segments used to deliver the entire LPP message body.

2	Endpoint A may send one or more additional LPP messages to Endpoint B with the IE SegmentationInfo set to moreMessagesOnTheWay to continue delivering the segmented LPP message.

3.	Endpoint A sends the final LPP message segment to Endpoint B and includes the IE SegmentationInfo set to noMoreMessages to indicate that this is the final LPP message segment. Endpoint B assumes that the complete LPP message body has been received.

# 5	LPP Procedures

## 5.1	Procedures related to capability transfer

The purpose of the procedures that are grouped together in this clause is to enable the transfer of capabilities from the target device to the server. Capabilities in this context refer to positioning and protocol capabilities related to LPP and the positioning methods supported by LPP.

These procedures instantiate the Capability Transfer transaction from TS 36.305 [2] and TS 38.305 [40].

### 5.1.1	Capability Transfer procedure

The Capability Transfer procedure is shown in Figure 5.1.1-1.

Figure 5.1.1-1: LPP Capability Transfer procedure

1.	The server sends a RequestCapabilities message to the target. The server may indicate the types of capability needed.

2.	The target responds with a ProvideCapabilities message to the server. The capabilities shall correspond to any capability types specified in step 1. This message shall include the endTransaction IE set to TRUE.

### 5.1.2	Capability Indication procedure

The Capability Indication procedure allows the target to provide unsolicited capabilities to the server and is shown in Figure 5.1.2-1.

Figure 5.1.2-1: LPP Capability Indication procedure

1.	The target sends a ProvideCapabilities message to the server. This message shall include the endTransaction IE set to TRUE.

### 5.1.3	Reception of LPP Request Capabilities

Upon receiving a RequestCapabilities message, the target device shall generate a ProvideCapabilities message as a response.

The target device shall:

1>	for each positioning method for which a request for capabilities is included in the message:

2>	if the target device supports this positioning method:

3>	include the capabilities of the device for that supported positioning method in the response message;

1>	set the IE LPP-TransactionID in the response message to the same value as the IE LPP-TransactionID in the received message;

1>	deliver the response message to lower layers for transmission.

### 5.1.4	Transmission of LPP Provide Capabilities

When triggered to transmit a ProvideCapabilities message, the target device shall:

1>	for each positioning method whose capabilities are to be indicated:

2>	set the corresponding IE to include the device's capabilities;

2>	if OTDOA capabilities are to be indicated:

3>	include the IE supportedBandListEUTRA;

1>	deliver the response to lower layers for transmission.

## 5.2	Procedures related to Assistance Data Transfer

The purpose of the procedures in this clause is to enable the target to request assistance data from the server to assist in positioning, and to enable the server to transfer assistance data to the target in the absence of a request.

These procedures instantiate the Assistance Data Transfer transaction from TS 36.305 [2] and TS 38.305 [40].

### 5.2.1	Assistance Data Transfer procedure

The Assistance Data Transfer procedure is shown in Figure 5.2.1-1.

Figure 5.2.1-1: LPP Assistance data transfer procedure

1.	The target sends a RequestAssistanceData message to the server.

2.	The server responds with a ProvideAssistanceData message to the target containing assistance data. The transferred assistance data should match or be a subset of the assistance data requested in step 1. The server may also provide any not requested information that it considers useful to the target. If step 3 does not occur, this message shall set the endTransaction IE to TRUE.

3.	The server may transmit one or more additional ProvideAssistanceData messages to the target containing further assistance data. The transferred assistance data should match or be a subset of the assistance data requested in step 1. The server may also provide any not requested information that it considers useful to the target. The last message shall include the endTransaction IE set to TRUE.

### 5.2.1a	Periodic Assistance Data Transfer procedure

The Periodic Assistance Data Transfer procedure is shown in Figure 5.2.1a-1. This procedure enables a target to request a server to send assistance data periodically.

NOTE 1:	In this version of the specification, periodic assistance data transfer is supported for HA GNSS (e.g., RTK) positioning only.

Figure 5.2.1a-1: LPP Periodic Assistance data transfer procedure

1.	The target sends a RequestAssistanceData message to the server using some available transactionID T1. The message contains a periodicSessionID S (different to any other periodicSessionID currently in use between the target and server) in the IE CommonIEsRequestAssistanceData. The message also includes a positioning method specific assistance data request element (e.g., IE A-GNSS-RequestAssistanceData) identifying the type of assistance data being requested together with desired periodicity conditions for sending it and a duration for ending the assistance data transfer (e.g., in IE GNSS-PeriodicAssistDataReq).

2.	The server responds with a ProvideAssistanceData message to the target. The message uses the transactionID T1 in step 1 and indicates the end of this transaction. The message contains the periodicSessionID S in IE CommonIEsProvideAssistanceData. If the request can be supported, the message contains the control parameters in the positioning method specific assistance data (e.g., IE A-GNSS-ProvideAssistanceData) which may confirm or redefine the type of assistance data or periodicity parameters requested at step 1 (e.g., in IE GNSS-PeriodicAssistData). If the target requested non-periodic assistance data in addition to the periodic assistance data in step 1, the ProvideAssistanceData message may also include the non-periodic assistance data in this step 2 (but not any periodic assistance data).
If the request cannot be supported (fully or partly), an error reason is provided in the positioning method specific IE (e.g., IE A-GNSS-Error). If the request cannot even partly be supported remaining steps are then not performed.

NOTE 2:	The target device infers from an absence of the periodicSessionID that the location server does not support periodic assistance data delivery. In that case, the target device does not expect the Data Transaction (Steps 3-7).

3.	When the first periodic message is available, the server sends an unsolicited ProvideAssistanceData message to the target containing the periodicSessionID S and the periodic assistance data confirmed in step 2. The message uses some available transactionID T2 that may be different to T1.

NOTE 3:	The positioning method specific control parameters (e.g., IE GNSS-PeriodicAssistData) are not included in the data transaction.

4.	The server may continue to send further ProvideAssistanceData messages to the target containing the periodic assistance data confirmed or redefined in step 2 when each additional periodicity condition occurs.

NOTE 4:	The target device expects a ProvideAssistanceData messages at the in Step 2 confirmed interval(s). If some or all of the assistance data is not available at each periodic interval, an error indication is provided in the positioning method specific IE (e.g., IE A-GNSS-Error).

5.	If the target requires the session to end, the target sends an Abort message to the server for transaction T2 that may optionally include an abortCause. Remaining steps are then omitted.

6.	If the server requires the session to end, the server sends an Abort message to the target for transaction T2 that may optionally include an abortCause. Remaining steps are then omitted.

7.	When the duration or other conditions for ending the periodic assistance data transfer occur, the last ProvideAssistanceData message transferred indicates the end of transaction T2.

### 5.2.1b	Periodic Assistance Data Transfer with Update procedure

Figure 5.2.1b-1: LPP Periodic Assistance data transfer with update procedure

1.	Steps 1-2 and optionally steps 3-4 are performed for the Periodic Assistance Data Transfer procedure in clause 5.2.1a with the following exceptions:

-	The RequestAssistanceData message in step 1 indicates the update capabilities of the target device.

-	The ProvideAssistanceData message in step 2 indicates the update capabilities of the target device which are supported by the server.

2.	If the target device changes its primary cell and if the update capabilities of the target device supported by the server in step 1 include update of a primary cell ID, the target device sends a RequestAssistanceData message to the server using some available transactionID T3, which is different from T2 (previously used in step 2). The message contains the periodicSessionID S (previously used in step 1) and the new primary cell ID in the IE CommonIEsRequestAssistanceData.

3.	The server responds with a ProvideAssistanceData message to the target. The message uses the transactionID T3 in step 2 and indicates the end of this transaction. The message contains the periodicSessionID S in IE CommonIEsProvideAssistanceData. Steps 2-3 are repeated each time the target device changes its primary cell.

4.	Steps 4-7 are performed for the Periodic Assistance Data Transfer procedure in clause 5.2.1a.

### 5.2.2	Assistance Data Delivery procedure

The Assistance Data Delivery procedure allows the server to provide unsolicited assistance data to the target and is shown in Figure 5.2.2-1.

Figure 5.2.2-1: LPP Assistance data transfer procedure

1.	The server sends a ProvideAssistanceData message to the target containing assistance data. If step 2 does not occur, this message shall set the endTransaction IE to TRUE.

2.	The server may transmit one or more additional ProvideAssistanceData messages to the target containing additional assistance data. The last message shall include the endTransaction IE set to TRUE.

### 5.2.2a	Periodic Assistance Data Delivery procedure

The Periodic Assistance Data Delivery procedure allows the server to provide unsolicited periodic assistance data to the target and is shown in Figure 5.2.2a-1.

NOTE 1:	In this version of the specification, periodic assistance data delivery is supported for HA GNSS (e.g., RTK) positioning only.

Figure 5.2.2a-1: LPP Periodic Assistance data delivery procedure

1.	The server sends a ProvideAssistanceData message to the target using some available transactionID T1 and indicates the end of this transaction. The message contains a periodicSessionID S (different to any other periodicSessionID currently in use between the server and target) in the IE CommonIEsProvideAssistanceData. The message includes positioning method specific assistance data control parameters (e.g., in IE A-GNSS-ProvideAssistanceData) identifying the type of periodic assistance data being delivered together with periodicity conditions for sending it and a duration for ending the assistance data delivery (e.g., in IE GNSS-PeriodicAssistData). The ProvideAssistanceData message may also include non-periodic assistance data (but not any periodic assistance data).

2.	When the first periodic message is available, the server sends an unsolicited ProvideAssistanceData message to the target containing the periodicSessionID S and the periodic assistance data announced in step 1. The message uses some available transactionID T2 that may be different to T1.

NOTE 2:	The positioning method specific control parameters (e.g., IE GNSS-PeriodicAssistData) are not included in the data transaction.

3.	The server may continue to send further ProvideAssistanceData messages to the target containing the periodic assistance data announced in step 2 when each additional periodicity condition occurs.

NOTE3:	The target device expects a ProvideAssistanceData messages at the in Step 2 announced interval(s). If some or all of the assistance data is not available at each periodic interval, an error indication is provided in the positioning method specific IE (e.g., IE A-GNSS-Error).

4.	If the target requires the session to end, the target sends an Abort message to the server for transaction T2 that may optionally include an abortCause. Remaining steps are then omitted.

5.	If the server requires the session to end, the server sends an Abort message to the target for transaction T2 that may optionally include an abortCause. Remaining steps are then omitted.

6.	When the duration or other conditions for ending the periodic assistance data transfer occur, the last ProvideAssistanceData message transferred indicates the end of transaction T2.

### 5.2.3	Transmission of LPP Request Assistance Data

When triggered to transmit a RequestAssistanceData message, the target device shall:

1>	set the IEs for the positioning-method-specific request for assistance data to request the data indicated by upper layers.

### 5.2.4	Reception of LPP Provide Assistance Data

Upon receiving a ProvideAssistanceData message, the target device shall:

1>	for each positioning method contained in the message:

2>	deliver the related assistance data to upper layers.

## 5.3	Procedures related to Location Information Transfer

The purpose of the procedures in this clause is to enable the server to request location measurement data and/or a location estimate from the target, and to enable the target to transfer location measurement data and/or a location estimate to a server in the absence of a request.

These procedures instantiate the Location Information Transfer transaction in TS 36.305 [2] and TS 38.305 [40].

NOTE:	The service layer (e.g. NAS or OMA SUPL ULP) would be used to transfer information associated with a location request from a target to a server (MO-LR).

### 5.3.1	Location Information Transfer procedure

The Location Information Transfer procedure is shown in Figure 5.3.1-1.

Figure 5.3.1-1: LPP Location Information transfer procedure

1.	The server sends a RequestLocationInformation message to the target to request location information, indicating the type of location information needed and potentially the associated QoS.

2.	The target sends a ProvideLocationInformation message to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. If step 3 does not occur, this message shall set the endTransaction IE to TRUE.

3.	If requested in step 1, the target sends additional ProvideLocationInformation messages to the server to transfer location information. The location information transferred should match or be a subset of the location information requested in step 1 unless the server explicitly allows additional location information. The last message shall include the endTransaction IE set to TRUE.

### 5.3.2	Location Information Delivery procedure

The Location Information Delivery allows the target to provide unsolicited location information to the server. The procedure is shown in Figure 5.3.2-1.

Figure 5.3.2-1: LPP Location Information Delivery procedure

1.	The target sends a ProvideLocationInformation message to the server to transfer location information. If step 2 does not occur, this message shall set the endTransaction IE to TRUE.

2.	The target may send one or more additional ProvideLocationInformation messages to the server containing additional location information data. The last message shall include the endTransaction IE set to TRUE.

### 5.3.3	Reception of Request Location Information

Upon receiving a RequestLocationInformation message, the target device shall:

1>	if the requested information is compatible with the target device capabilities and configuration:

2>	include the requested information in a ProvideLocationInformation message;

2>	set the IE LPP-TransactionID in the response to the same value as the IE LPP-TransactionID in the received message;

2>	deliver the ProvideLocationInformation message to lower layers for transmission.

1>	otherwise:

2>	if one or more positioning methods are included that the target device does not support:

3>	continue to process the message as if it contained only information for the supported positioning methods;

3>	handle the signaling content of the unsupported positioning methods by LPP error detection as in 5.4.3.

### 5.3.4	Transmission of Provide Location Information

When triggered to transmit ProvideLocationInformation message, the target device shall:

1>	for each positioning method contained in the message:

2>	set the corresponding IE to include the available location information;

1>	deliver the response to lower layers for transmission.

## 5.4	Error Handling Procedures

### 5.4.1	General

This clause describes how a receiving entity (target device or location server) behaves in cases when it receives erroneous or unexpected data or detects that certain data are missing.

### 5.4.2	Procedures related to Error Indication

Figure 5.4.2-1 shows the Error indication procedure.

Figure 5.4.2-1: LPP Error Indication procedure

1.	Endpoint A sends an LPP message to Endpoint B.

2.	Endpoint B determines that the LPP message in step 1 contains an error. Endpoint B returns an Error message to Endpoint A indicating the error or errors and discards the message in step 1. If Endpoint B is able to determine that the erroneous LPP message in step 1 is an LPP Error or Abort Message, Endpoint B discards the message in step 1 without returning an Error message to Endpoint A.

### 5.4.3	LPP Error Detection

Upon receiving any LPP message, the receiving entity shall attempt to decode the message and verify the presence of any errors and:

1>	if decoding errors are encountered:

2>	if the receiver can not determine that the received message is an LPP Error or Abort message:

3>	return an LPP Error message to the sender and include the received LPP-TransactionID, if this was decoded, and type of error;

3>	if the receiver can determine the session and the LPP-TransactionID and the received message includes the IE SegmentationInfo and the receiver has previously stored message segments for this session and LPP-TransactionID:

4>	discard all stored LPP message segments for this session and LPP-TransactionID;

3>	discard the received message and stop the error detection procedure;

1>	if the message is a duplicate of a previously received message:

2>	discard the message and stop the error detection procedure;

1>	if the LPP-TransactionID matches the LPP-TransactionID for a procedure that is still ongoing for the same session and the message type is invalid for the current state of the procedure:

2>	abort the ongoing procedure;

2>	return an LPP Error message to the sender and include the received transaction ID and type of error;

2>	if the message includes the IE SegmentationInfo and the receiver has previously stored message segments for this session and LPP-TransactionID:

3>	discard all stored LPP message segments for this session and LPP-TransactionID;

2>	discard the message and stop the error detection procedure;

1>	if the message includes the IE SegmentationInfo:

2>	if the receiver has previously stored LPP message segments for this session and LPP-TransactionID:

3>	if the received message type is different to the stored message type:

4>	return an LPP Error message to the sender and include the received transaction ID and type of error;

4>	discard the message and all stored LPP message segments for this session and LPP-TransactionID and stop the error detection procedure;

2>	if the IE SegmentationInfo has the value moreMessagesOnTheWay:

3>	store the received message;

NOTE:	As an implementation option, the receiver of an LPP Provide Assistance Data or LPP Provide Location Information message may process the received message segment instead of storing the message.

2>	if the IE SegmentationInfo has the value noMoreMessages:

3>	continue error detection for the received message and any stored LPP message segments for this session and LPP-TransactionID;

1> if the message type is an LPP RequestCapabilities and some of the requested information is not supported:

2>	return any information that can be provided in a normal response.

1>	if the message type is an LPP RequestAssistanceData or RequestLocationInformation and some or all of the requested information is not supported:

2>	return any information that can be provided in a normal response, which includes indications on other information that is not supported.

### 5.4.4	Reception of an LPP Error Message

Upon receiving an Error message, a device shall:

1>	abort any ongoing procedure associated with the LPP-TransactionID if included in the received message.

The device may:

1>	restart the aborted procedure taking into consideration the returned error information.

## 5.5	Abort Procedure

### 5.5.1	General

The purpose of the abort procedure is to allow the target device or location server to abort an ongoing procedure due to some unexpected event (e.g., cancellation of a location request by an LCS client). It can also be used to stop an ongoing procedure (e.g., periodic location reporting from the target device).

### 5.5.2	Procedures related to Abort

Figure 5.5.2-1 shows the Abort procedure.

Figure 5.5.2-1: LPP Abort procedure

1.	A procedure P is ongoing between endpoints A and B.

2.	Endpoint A determines that the procedure must be aborted and sends an Abort message to Endpoint B carrying the transaction ID for procedure P. Endpoint B aborts procedure P.

### 5.5.3	Reception of an LPP Abort Message

Upon receiving an Abort message, a device shall:

1>	abort any ongoing procedure associated with the transaction ID indicated in the message.

# 6	Information Element Abstract Syntax Definition

## 6.1	General

The contents of each LPP message is specified in clause 6.2 using ASN.1 to specify the message syntax and using tables when needed to provide further detailed information about the fields specified in the message syntax.

The ASN.1 in this clause uses the same format and coding conventions as described in Annex A of TS 36.331 [12].

Transfer syntax for LPP messages is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [22]. The encoded LPP message always contains a multiple of 8 bits.

Transfer syntax for LPP IEs is derived from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [22]. The encoded LPP IE always contains a multiple of 8 bits. This applies when a single LPP IE is encoded as the basic production, i.e. for other purposes than encoding the LPP IE within an LPP message.

The need for fields to be present in a message or an abstract type, i.e., the ASN.1 fields that are specified as OPTIONAL in the abstract notation (ASN.1), is specified by means of comment text tags attached to the OPTIONAL statement in the abstract syntax. The meaning of each tag is specified in table 6.1-1. The use of these tags in the uplink (target to server) direction does not impose any requirements on the server.

Table 6.1-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present A field for which the need is specified by means of conditions. For each conditionTag, the need is specified in a tabular form following the ASN.1 segment. In case, according to the conditions, a field is not present, the target takes no action and where applicable shall continue to use the existing value (and/or the associated functionality) unless explicitly stated otherwise in the description of the field itself. |
| Need OP | Optionally present A field that is optional to signal. For downlink messages, the target is not required to take any special action on absence of the field beyond what is specified in the procedural text or the field description table following the ASN.1 segment. The target behaviour on absence should be captured either in the procedural text or in the field description. |
| Need ON | Optionally present, No action A field that is optional to signal. If the message is received by the target, and in case the field is absent, the target takes no action and where applicable shall continue to use the existing value (and/or the associated functionality). |
| Need OR | Optionally present, Release A field that is optional to signal. If the message is received by the target, and in case the field is absent, the target shall discontinue/ stop using/ delete any existing value (and/ or the associated functionality). |

When specifying information elements which are to be represented by BIT STRINGs, if not otherwise specifically stated in the field description of the concerned IE or elsewhere, the following principle applies with regards to the ordering of bits:

-	The first bit (leftmost bit) contains the most significant bit (MSB);

-	the last bit (rightmost bit) contains the least significant bit (LSB).

## 6.2	LPP PDU Structure

#### –	LPP-PDU-Definitions

This ASN.1 segment is the start of the LPP PDU definitions.

-- ASN1START

LPP-PDU-Definitions

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

-- ASN1STOP

#### –	LPP-Message

The LPP-Message provides the complete set of information for an invocation or response pertaining to an LPP transaction.

-- ASN1START

LPP-Message ::= SEQUENCE {

transactionID			LPP-TransactionID	OPTIONAL,	-- Need ON

endTransaction			BOOLEAN,

sequenceNumber			SequenceNumber		OPTIONAL,	-- Need ON

acknowledgement			Acknowledgement		OPTIONAL,	-- Need ON

lpp-MessageBody			LPP-MessageBody		OPTIONAL	-- Need ON

}

SequenceNumber ::= INTEGER (0..255)

Acknowledgement ::= SEQUENCE {

ackRequested	BOOLEAN,

ackIndicator	SequenceNumber		OPTIONAL

}

-- ASN1STOP

| LPP-Message field descriptions |
| --- |
| transactionID This field is omitted if an lpp-MessageBody is not present (i.e. in an LPP message sent only to acknowledge a previously received message) or if it is not available to the transmitting entity (e.g., in an LPP-Error message triggered by a message that could not be parsed). If present, this field shall be ignored at a receiver in an LPP message for which the lpp-MessageBody is not present. |  |
| endTransaction This field indicates whether an LPP message is the last message carrying an lpp-MessageBody in a transaction (TRUE) or not last (FALSE). When LPP message segmentation is used, only the final LPP message segment may indicate the end of the transaction. |  |
| sequenceNumber This field may be included when LPP operates over the control plane and an lpp-MessageBody is included but shall be omitted otherwise. |  |
| acknowledgement This field is included in an LPP acknowledgement and in any LPP message requesting an acknowledgement when LPP operates over the control plane and is omitted otherwise. |  |
| ackRequested This field indicates whether an LPP acknowledgement is requested (TRUE) or not (FALSE). A value of TRUE may only be included when an lpp-MessageBody is included. |  |
| ackIndicator This field indicates the sequence number of the message being acknowledged. |  |
| lpp-MessageBody This field may be omitted in the case the message is sent only to acknowledge a previously received message. |

#### –	LPP-MessageBody

The LPP-MessageBody identifies the type of an LPP message and contains all LPP information specifically associated with that type.

-- ASN1START

LPP-MessageBody ::= CHOICE {

c1						CHOICE {

requestCapabilities			RequestCapabilities,

provideCapabilities			ProvideCapabilities,

requestAssistanceData		RequestAssistanceData,

provideAssistanceData		ProvideAssistanceData,

requestLocationInformation	RequestLocationInformation,

provideLocationInformation	ProvideLocationInformation,

abort						Abort,

error						Error,

spare7 NULL, spare6 NULL, spare5 NULL, spare4 NULL,

spare3 NULL, spare2 NULL, spare1 NULL, spare0 NULL

},

messageClassExtension	SEQUENCE {}

}

-- ASN1STOP

#### –	LPP-TransactionID

The LPP-TransactionID identifies a particular LPP transaction and the initiator of the transaction.

-- ASN1START

LPP-TransactionID ::= SEQUENCE {

initiator				Initiator,

transactionNumber		TransactionNumber,

...

}

Initiator ::= ENUMERATED {

locationServer,

targetDevice,

...

}

TransactionNumber ::= INTEGER (0..255)

-- ASN1STOP

## 6.3	Message Body IEs

#### –	RequestCapabilities

The RequestCapabilities message body in a LPP message is used by the location server to request the target device capability information for LPP and the supported individual positioning methods.

-- ASN1START

RequestCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestCapabilities-r9		RequestCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestCapabilities-r9-IEs ::= SEQUENCE {

commonIEsRequestCapabilities		CommonIEsRequestCapabilities		OPTIONAL,	-- Need ON

a-gnss-RequestCapabilities			A-GNSS-RequestCapabilities			OPTIONAL,	-- Need ON

otdoa-RequestCapabilities			OTDOA-RequestCapabilities			OPTIONAL,	-- Need ON

ecid-RequestCapabilities			ECID-RequestCapabilities			OPTIONAL,	-- Need ON

epdu-RequestCapabilities			EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[	sensor-RequestCapabilities-r13	Sensor-RequestCapabilities-r13		OPTIONAL,	-- Need ON

tbs-RequestCapabilities-r13		TBS-RequestCapabilities-r13			OPTIONAL,	-- Need ON

wlan-RequestCapabilities-r13	WLAN-RequestCapabilities-r13		OPTIONAL,	-- Need ON

bt-RequestCapabilities-r13		BT-RequestCapabilities-r13			OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestCapabilities-r16	NR-ECID-RequestCapabilities-r16		OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestCapabilities-r16

NR-Multi-RTT-RequestCapabilities-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestCapabilities-r16

NR-DL-AoD-RequestCapabilities-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestCapabilities-r16

NR-DL-TDOA-RequestCapabilities-r16	OPTIONAL,	-- Need ON

nr-UL-RequestCapabilities-r16	NR-UL-RequestCapabilities-r16		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	ProvideCapabilities

The ProvideCapabilities message body in a LPP message indicates the LPP capabilities of the target device to the location server.

-- ASN1START

ProvideCapabilities ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideCapabilities-r9		ProvideCapabilities-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideCapabilities-r9-IEs ::= SEQUENCE {

commonIEsProvideCapabilities		CommonIEsProvideCapabilities			OPTIONAL,

a-gnss-ProvideCapabilities			A-GNSS-ProvideCapabilities				OPTIONAL,

otdoa-ProvideCapabilities			OTDOA-ProvideCapabilities				OPTIONAL,

ecid-ProvideCapabilities			ECID-ProvideCapabilities				OPTIONAL,

epdu-ProvideCapabilities			EPDU-Sequence							OPTIONAL,

...,

[[	sensor-ProvideCapabilities-r13	Sensor-ProvideCapabilities-r13			OPTIONAL,

tbs-ProvideCapabilities-r13		TBS-ProvideCapabilities-r13				OPTIONAL,

wlan-ProvideCapabilities-r13	WLAN-ProvideCapabilities-r13			OPTIONAL,

bt-ProvideCapabilities-r13		BT-ProvideCapabilities-r13				OPTIONAL

]],

[[	nr-ECID-ProvideCapabilities-r16	NR-ECID-ProvideCapabilities-r16			OPTIONAL,

nr-Multi-RTT-ProvideCapabilities-r16

NR-Multi-RTT-ProvideCapabilities-r16	OPTIONAL,

nr-DL-AoD-ProvideCapabilities-r16

NR-DL-AoD-ProvideCapabilities-r16		OPTIONAL,

nr-DL-TDOA-ProvideCapabilities-r16

NR-DL-TDOA-ProvideCapabilities-r16		OPTIONAL,

nr-UL-ProvideCapabilities-r16	NR-UL-ProvideCapabilities-r16			OPTIONAL

]]

}

-- ASN1STOP

#### –	RequestAssistanceData

The RequestAssistanceData message body in a LPP message is used by the target device to request assistance data from the location server.

-- ASN1START

RequestAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestAssistanceData-r9	RequestAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsRequestAssistanceData		CommonIEsRequestAssistanceData		OPTIONAL,

a-gnss-RequestAssistanceData		A-GNSS-RequestAssistanceData		OPTIONAL,

otdoa-RequestAssistanceData			OTDOA-RequestAssistanceData			OPTIONAL,

epdu-RequestAssistanceData			EPDU-Sequence						OPTIONAL,

...,

[[	sensor-RequestAssistanceData-r14

Sensor-RequestAssistanceData-r14	OPTIONAL,

tbs-RequestAssistanceData-r14	TBS-RequestAssistanceData-r14		OPTIONAL,

wlan-RequestAssistanceData-r14	WLAN-RequestAssistanceData-r14		OPTIONAL

]],

[[	nr-Multi-RTT-RequestAssistanceData-r16	NR-Multi-RTT-RequestAssistanceData-r16	OPTIONAL,

nr-DL-AoD-RequestAssistanceData-r16		NR-DL-AoD-RequestAssistanceData-r16		OPTIONAL,

nr-DL-TDOA-RequestAssistanceData-r16	NR-DL-TDOA-RequestAssistanceData-r16	OPTIONAL

]]

}

-- ASN1STOP

#### –	ProvideAssistanceData

The ProvideAssistanceData message body in a LPP message is used by the location server to provide assistance data to the target device either in response to a request from the target device or in an unsolicited manner.

-- ASN1START

ProvideAssistanceData ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideAssistanceData-r9	ProvideAssistanceData-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideAssistanceData-r9-IEs ::= SEQUENCE {

commonIEsProvideAssistanceData		CommonIEsProvideAssistanceData		OPTIONAL,	-- Need ON

a-gnss-ProvideAssistanceData		A-GNSS-ProvideAssistanceData		OPTIONAL,	-- Need ON

otdoa-ProvideAssistanceData			OTDOA-ProvideAssistanceData			OPTIONAL,	-- Need ON

epdu-Provide-Assistance-Data		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-ProvideAssistanceData-r14	Sensor-ProvideAssistanceData-r14	OPTIONAL,	-- Need ON

tbs-ProvideAssistanceData-r14		TBS-ProvideAssistanceData-r14		OPTIONAL,	-- Need ON

wlan-ProvideAssistanceData-r14		WLAN-ProvideAssistanceData-r14		OPTIONAL	-- Need ON

]],

[[	nr-Multi-RTT-ProvideAssistanceData-r16

NR-Multi-RTT-ProvideAssistanceData-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-ProvideAssistanceData-r16

NR-DL-AoD-ProvideAssistanceData-r16	OPTIONAL,	-- Need ON

nr-DL-TDOA-ProvideAssistanceData-r16

NR-DL-TDOA-ProvideAssistanceData-r16

OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	RequestLocationInformation

The RequestLocationInformation message body in a LPP message is used by the location server to request positioning measurements or a position estimate from the target device.

-- ASN1START

RequestLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

requestLocationInformation-r9	RequestLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

RequestLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsRequestLocationInformation

CommonIEsRequestLocationInformation	OPTIONAL,	-- Need ON

a-gnss-RequestLocationInformation	A-GNSS-RequestLocationInformation	OPTIONAL,	-- Need ON

otdoa-RequestLocationInformation	OTDOA-RequestLocationInformation	OPTIONAL,	-- Need ON

ecid-RequestLocationInformation		ECID-RequestLocationInformation		OPTIONAL,	-- Need ON

epdu-RequestLocationInformation		EPDU-Sequence						OPTIONAL,	-- Need ON

...,

[[

sensor-RequestLocationInformation-r13

Sensor-RequestLocationInformation-r13

OPTIONAL,	-- Need ON

tbs-RequestLocationInformation-r13	TBS-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

wlan-RequestLocationInformation-r13	WLAN-RequestLocationInformation-r13	OPTIONAL,	-- Need ON

bt-RequestLocationInformation-r13	BT-RequestLocationInformation-r13	OPTIONAL	-- Need ON

]],

[[	nr-ECID-RequestLocationInformation-r16

NR-ECID-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-Multi-RTT-RequestLocationInformation-r16

NR-Multi-RTT-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-AoD-RequestLocationInformation-r16

NR-DL-AoD-RequestLocationInformation-r16

OPTIONAL,	-- Need ON

nr-DL-TDOA-RequestLocationInformation-r16

NR-DL-TDOA-RequestLocationInformation-r16

OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

RequestLocationInformation field descriptions

commonIEsRequestLocationInformation This field specifies the location information type requested by the location server and optionally other configuration information associated with the requested location information. This field should always be included in this version of the protocol.

#### –	ProvideLocationInformation

The ProvideLocationInformation message body in a LPP message is used by the target device to provide positioning measurements or position estimates to the location server.

-- ASN1START

ProvideLocationInformation ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

provideLocationInformation-r9	ProvideLocationInformation-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

ProvideLocationInformation-r9-IEs ::= SEQUENCE {

commonIEsProvideLocationInformation

CommonIEsProvideLocationInformation	OPTIONAL,

a-gnss-ProvideLocationInformation	A-GNSS-ProvideLocationInformation	OPTIONAL,

otdoa-ProvideLocationInformation	OTDOA-ProvideLocationInformation	OPTIONAL,

ecid-ProvideLocationInformation		ECID-ProvideLocationInformation		OPTIONAL,

epdu-ProvideLocationInformation		EPDU-Sequence						OPTIONAL,

...,

[[

sensor-ProvideLocationInformation-r13

Sensor-ProvideLocationInformation-r13

OPTIONAL,

tbs-ProvideLocationInformation-r13	TBS-ProvideLocationInformation-r13	OPTIONAL,

wlan-ProvideLocationInformation-r13	WLAN-ProvideLocationInformation-r13	OPTIONAL,

bt-ProvideLocationInformation-r13	BT-ProvideLocationInformation-r13	OPTIONAL

]],

[[	nr-ECID-ProvideLocationInformation-r16

NR-ECID-ProvideLocationInformation-r16		OPTIONAL,

nr-Multi-RTT-ProvideLocationInformation-r16

NR-Multi-RTT-ProvideLocationInformation-r16 OPTIONAL,

nr-DL-AoD-ProvideLocationInformation-r16

NR-DL-AoD-ProvideLocationInformation-r16	OPTIONAL,

nr-DL-TDOA-ProvideLocationInformation-r16

NR-DL-TDOA-ProvideLocationInformation-r16	OPTIONAL

]]

}

-- ASN1STOP

#### –	Abort

The Abort message body in a LPP message carries a request to abort an ongoing LPP procedure.

-- ASN1START

Abort ::= SEQUENCE {

criticalExtensions		CHOICE {

c1						CHOICE {

abort-r9		Abort-r9-IEs,

spare3 NULL, spare2 NULL, spare1 NULL

},

criticalExtensionsFuture	SEQUENCE {}

}

}

Abort-r9-IEs ::= SEQUENCE {

commonIEsAbort		CommonIEsAbort			OPTIONAL,	-- Need ON

...,

epdu-Abort			EPDU-Sequence			OPTIONAL	-- Need ON

}

-- ASN1STOP

#### –	Error

The Error message body in a LPP message carries information concerning a LPP message that was received with errors.

-- ASN1START

Error ::= CHOICE {

error-r9					Error-r9-IEs,

criticalExtensionsFuture	SEQUENCE {}

}

Error-r9-IEs ::= SEQUENCE {

commonIEsError		CommonIEsError			OPTIONAL,	-- Need ON

...,

epdu-Error			EPDU-Sequence			OPTIONAL	-- Need ON

}

-- ASN1STOP

## 6.4	Common IEs

Common IEs comprise IEs that are applicable to more than one LPP positioning method.

### 6.4.1	Common Lower-Level IEs

#### –	AccessTypes

The IE AccessTypes is used to indicate several cellular access types using a bit map.

-- ASN1START

AccessTypes ::= SEQUENCE {

accessTypes		BIT STRING {	eutra		(0),

utra		(1),

gsm			(2),

nb-iot		(3),

nr-v1510	(4) } (SIZE (1..8)),

...

}

-- ASN1STOP

AccessTypes field descriptions

accessTypes This field specifies the cellular access type(s). This is represented by a bit string, with a one-value at the bit position means the particular access type is addressed; a zero-value means not addressed.

#### –	ARFCN-ValueEUTRA

The IEs ARFCN-ValueEUTRA and ARFCN-ValueEUTRA-v9a0 are used to indicate the ARFCN of the E-UTRA carrier frequency, as defined in TS 36.331 [12].

-- ASN1START

ARFCN-ValueEUTRA ::= INTEGER (0..maxEARFCN)

ARFCN-ValueEUTRA-v9a0 ::=	INTEGER (maxEARFCN-Plus1..maxEARFCN2)

ARFCN-ValueEUTRA-r14 ::=	INTEGER (0..maxEARFCN2)

-- ASN1STOP

NOTE:	For fields using the original value range, as defined by IE ARFCN-ValueEUTRA i.e. without suffix, value maxEARFCN indicates that the E-UTRA carrier frequency is indicated by means of an extension.

#### –	ARFCN-ValueNR

The IE ARFCN-ValueNR is used to indicate the ARFCN applicable for a downlink, uplink or bi-directional (TDD) NR global frequency raster, as defined in TS 38.101-2 [34] and TS 38.101-1 [37].

-- ASN1START

ARFCN-ValueNR-r15 ::= INTEGER (0..3279165)

-- ASN1STOP

#### –	ARFCN-ValueUTRA

The IE ARFCN-ValueUTRA is used to indicate the ARFCN of the UTRA carrier frequency, as defined in TS 25.331 [13].

-- ASN1START

ARFCN-ValueUTRA ::=	INTEGER (0..16383)

-- ASN1STOP

#### –	CarrierFreq-NB

The IE CarrierFreq-NB is used to provide the NB-IoT carrier frequency, as defined in TS 36.101 [21].

-- ASN1START

CarrierFreq-NB-r14 ::=		SEQUENCE {

carrierFreq-r14				ARFCN-ValueEUTRA-r14,

carrierFreqOffset-r14		CarrierFreqOffsetNB-r14				OPTIONAL,

...

}

-- ASN1STOP

CarrierFreq-NB field descriptions

carrierFreq This field specifies the ARFCN applicable for the NB-IoT carrier frequency as defined in TS 36.101 [21, Table 5.7.3-1].

carrierFreqOffset This field specifies the offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [21].

#### –	CarrierFreqOffsetNB

The IE CarrierFreqOffsetNB is used to provide the offset of the NB-IoT channel number to EARFCN of a NB-IoT carrier.

-- ASN1START

CarrierFreqOffsetNB-r14 ::=		ENUMERATED {

v-10, v-9, v-8,	v-7, v-6, v-5, v-4, v-3, v-2, v-1, v-0dot5,

v0, v1, v2, v3, v4, v5, v6, v7, v8, v9

}

-- ASN1STOP

CarrierFreqOffsetNB field descriptions

CarrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to EARFCN as defined in TS 36.101 [21]. Value v-10 means -10, v-9 means -9, and so on.

#### –	CellGlobalIdEUTRA-AndUTRA

The IE CellGlobalIdEUTRA-AndUTRA specifies the global Cell Identifier for E-UTRA or UTRA, the globally unique identity of a cell in E-UTRA or UTRA.

-- ASN1START

CellGlobalIdEUTRA-AndUTRA ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

cellIdentity		CHOICE {

eutra	BIT STRING (SIZE (28)),

utra	BIT STRING (SIZE (32))

},

...

}

-- ASN1STOP

CellGlobalIdEUTRA-AndUTRA field descriptions

plmn-Identity This field identifies the PLMN of the cell as defined in TS 36.331 [12].

cellIdentity This field defines the identity of the cell within the context of the PLMN as defined in TS 36.331 [12] and TS 25.331 [13]. The size of the bit string allows for the 32-bit extended UTRAN cell ID; in the case the cell ID is shorter, the first bits of the string are set to 0.

#### –	CellGlobalIdGERAN

The IE CellGlobalIdGERAN specifies the global Cell Identifier for GERAN, the globally unique identity of a cell in GERAN.

-- ASN1START

CellGlobalIdGERAN ::= SEQUENCE {

plmn-Identity		SEQUENCE {

mcc		SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc		SEQUENCE (SIZE (2..3))	OF INTEGER (0..9)

},

locationAreaCode		BIT STRING (SIZE (16)),

cellIdentity			BIT STRING (SIZE (16)),

...

}

-- ASN1STOP

CellGlobalIdGERAN field descriptions

plmn-Identity This field identifies the PLMN of the cell.

locationAreaCode This field is a fixed length code identifying the location area within a PLMN.

cellIdentity This field specifies the cell Identifier which is unique within the context of the GERAN location area.

#### –	ECGI

The IE ECGI specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA (TS 36.331 [12]).

NOTE:	The IE ECGI is also used for NB-IoT access.

-- ASN1START

ECGI ::= SEQUENCE {

mcc				SEQUENCE (SIZE (3))	OF INTEGER (0..9),

mnc				SEQUENCE (SIZE (2..3))	OF INTEGER (0..9),

cellidentity	BIT STRING (SIZE (28))

}

-- ASN1STOP

#### –	Ellipsoid-Point

The IE Ellipsoid-Point is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-Point ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

#### –	Ellipsoid-PointWithUncertaintyCircle

The IE Ellipsoid-PointWithUncertaintyCircle is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Ellipsoid-PointWithUncertaintyCircle ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

uncertainty					INTEGER (0..127)

}

-- ASN1STOP

#### –	EllipsoidPointWithUncertaintyEllipse

The IE EllipsoidPointWithUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

EllipsoidPointWithUncertaintyEllipse ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

uncertaintySemiMajor		INTEGER (0..127),

uncertaintySemiMinor		INTEGER (0..127),

orientationMajorAxis		INTEGER (0..179),

confidence					INTEGER (0..100)

}

-- ASN1STOP

#### –	EllipsoidPointWithAltitude

The IE EllipsoidPointWithAltitude is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

EllipsoidPointWithAltitude ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..32767)				-- 15 bit field

}

-- ASN1STOP

#### –	EllipsoidPointWithAltitudeAndUncertaintyEllipsoid

The IE EllipsoidPointWithAltitudeAndUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

altitudeDirection			ENUMERATED {height, depth},

altitude					INTEGER (0..32767),				-- 15 bit field

uncertaintySemiMajor		INTEGER (0..127),

uncertaintySemiMinor		INTEGER (0..127),

orientationMajorAxis		INTEGER (0..179),

uncertaintyAltitude			INTEGER (0..127),

confidence					INTEGER (0..100)

}

-- ASN1STOP

#### –	EllipsoidArc

The IE EllipsoidArc is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

EllipsoidArc ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607),	-- 24 bit field

innerRadius					INTEGER (0..65535),				-- 16 bit field,

uncertaintyRadius			INTEGER (0..127),

offsetAngle					INTEGER (0..179),

includedAngle				INTEGER (0..179),

confidence					INTEGER (0..100)

}

-- ASN1STOP

#### –	EPDU-Sequence

The EPDU-Sequence contains IEs that are defined externally to LPP by other organizations.

-- ASN1START

EPDU-Sequence ::= SEQUENCE (SIZE (1..maxEPDU)) OF EPDU

maxEPDU INTEGER ::= 16

EPDU ::= SEQUENCE {

ePDU-Identifier			EPDU-Identifier,

ePDU-Body				EPDU-Body

}

EPDU-Identifier ::= SEQUENCE {

ePDU-ID					EPDU-ID,

ePDU-Name				EPDU-Name		OPTIONAL,

...

}

EPDU-ID ::= INTEGER (1..256)

EPDU-Name ::= VisibleString (SIZE (1..32))

EPDU-Body ::= OCTET STRING

-- ASN1STOP

EPDU-Sequence field descriptions

EPDU-ID This field provides a unique integer ID for the externally defined positioning method. Its value is assigned to the external entity that defines the EPDU. See table External PDU Identifier Definition for a list of external PDU identifiers defined in this version of the specification.

EPDU-Name This field provides an optional character encoding which can be used to provide a quasi-unique name for an external PDU – e.g., by containing the name of the defining organization and/or the name of the associated public or proprietary standard for the EPDU.

EPDU-Body The content and encoding of this field are defined externally to LPP.

External PDU Identifier Definition

| EPDU-ID | EPDU Defining entity | Method name | Reference |
| --- | --- | --- | --- |
| 1 | OMA LOC | OMA LPP extensions (LPPe) | OMA-TS-LPPe-V1_0 [20] |

#### –	FreqBandIndicatorNR

The IE FreqBandIndicatorNR specifies the NR band indicator (TS 38.331 [35]).

-- ASN1START

FreqBandIndicatorNR-r16 ::= INTEGER (1..1024)

-- ASN1STOP

#### –	HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid

The IE HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16 ::= SEQUENCE {

degreesLatitude-r16							INTEGER(-2147483648..2147483647),

degreesLongitude-r16						INTEGER(-2147483648..2147483647),

altitude-r16								INTEGER(-64000..1280000),

uncertaintySemiMajor-r16					INTEGER (0..255),

uncertaintySemiMinor-r16					INTEGER (0..255),

orientationMajorAxis-r16					INTEGER (0..179),

horizontalConfidence-r16					INTEGER (0..100),

uncertaintyAltitude-r16						INTEGER (0..255),

verticalConfidence-r16						INTEGER (0..100),

ha-HorizontalExtendedRangeUsed-r16			BOOLEAN,

ha-VerticalExtendedRangeUsed-r16			BOOLEAN

}

-- ASN1STOP

#### –	HA-EllipsoidPointWithScalableUncertaintyEllipse

The IE HA-EllipsoidPointWithScalableUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HA-EllipsoidPointWithScalableUncertaintyEllipse-r16 ::= SEQUENCE {

degreesLatitude-r16							INTEGER(-2147483648..2147483647),

degreesLongitude-r16						INTEGER(-2147483648..2147483647),

uncertaintySemiMajor-r16					INTEGER (0..255),

uncertaintySemiMinor-r16					INTEGER (0..255),

orientationMajorAxis-r16					INTEGER (0..179),

confidence-r16								INTEGER (0..100),

ha-ExtendedUncertaintyRangeUsed-r16			BOOLEAN

}

-- ASN1STOP

#### –	HighAccuracyEllipsoidPointWithUncertaintyEllipse

The IE HighAccuracyEllipsoidPointWithUncertaintyEllipse is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15 ::= SEQUENCE {

degreesLatitude-r15				INTEGER(-2147483648..2147483647),

degreesLongitude-r15			INTEGER(-2147483648..2147483647),

uncertaintySemiMajor-r15		INTEGER (0..255),

uncertaintySemiMinor-r15		INTEGER (0..255),

orientationMajorAxis-r15		INTEGER (0..179),

confidence-r15					INTEGER (0..100)

}

-- ASN1STOP

#### –	HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid

The IE HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15 ::= SEQUENCE {

degreesLatitude-r15				INTEGER(-2147483648..2147483647),

degreesLongitude-r15			INTEGER(-2147483648..2147483647),

altitude-r15					INTEGER(-64000..1280000),

uncertaintySemiMajor-r15		INTEGER (0..255),

uncertaintySemiMinor-r15		INTEGER (0..255),

orientationMajorAxis-r15		INTEGER (0..179),

horizontalConfidence-r15		INTEGER (0..100),

uncertaintyAltitude-r15			INTEGER (0..255),

verticalConfidence-r15			INTEGER (0..100)

}

-- ASN1STOP

#### –	HorizontalVelocity

The IE HorizontalVelocity is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalVelocity ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047)

}

-- ASN1STOP

#### –	HorizontalWithVerticalVelocity

The IE HorizontalWithVerticalVelocity is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalWithVerticalVelocity ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255)

}

-- ASN1STOP

#### –	HorizontalVelocityWithUncertainty

The IE HorizontalVelocityWithUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalVelocityWithUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

uncertaintySpeed			INTEGER(0..255)

}

-- ASN1STOP

#### –	HorizontalWithVerticalVelocityAndUncertainty

The IE HorizontalWithVerticalVelocityAndUncertainty is used to describe a velocity shape as defined in TS 23.032 [15].

-- ASN1START

HorizontalWithVerticalVelocityAndUncertainty ::= SEQUENCE {

bearing						INTEGER(0..359),

horizontalSpeed				INTEGER(0..2047),

verticalDirection			ENUMERATED{upward, downward},

verticalSpeed				INTEGER(0..255),

horizontalUncertaintySpeed	INTEGER(0..255),

verticalUncertaintySpeed	INTEGER(0..255)

}

-- ASN1STOP

#### –	LocationCoordinateTypes

The IE LocationCoordinateTypes defines a list of possible geographic shapes as defined in TS 23.032 [15].

-- ASN1START

LocationCoordinateTypes ::= SEQUENCE {

ellipsoidPoint											BOOLEAN,

ellipsoidPointWithUncertaintyCircle						BOOLEAN,

ellipsoidPointWithUncertaintyEllipse					BOOLEAN,

polygon													BOOLEAN,

ellipsoidPointWithAltitude								BOOLEAN,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid		BOOLEAN,

ellipsoidArc											BOOLEAN,

...,

[[

highAccuracyEllipsoidPointWithUncertaintyEllipse-r15

BOOLEAN		OPTIONAL, -- Need ON

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15

BOOLEAN		OPTIONAL  -- Need ON

]],

[[

ha-EllipsoidPointWithScalableUncertaintyEllipse-r16

BOOLEAN		OPTIONAL, -- Need ON

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16

BOOLEAN		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

#### –	NCGI

The IE NCGI specifies the NR Cell Global Identifier (NCGI) which is used to identify NR cells globally (TS 38.331 [35]).

-- ASN1START

NCGI-r15 ::= SEQUENCE {

mcc-r15					SEQUENCE (SIZE (3)) 	OF INTEGER (0..9),

mnc-r15					SEQUENCE (SIZE (2..3)) 	OF INTEGER (0..9),

nr-cellidentity-r15		BIT STRING (SIZE (36))

}

-- ASN1STOP

#### –	NR-PhysCellId

The IE NR-PhysCellId specifies the NR physical cell identifier (TS 38.331 [35]).

-- ASN1START

NR-PhysCellID-r16 ::= INTEGER (0..1007)

-- ASN1STOP

#### –	PeriodicAssistanceDataControlParameters

The IE PeriodicAssistanceDataControlParameters is used in a periodic assistance data delivery procedure as described in clauses 5.2.1a and 5.2.2a.

-- ASN1START

PeriodicAssistanceDataControlParameters-r15 ::= SEQUENCE {

periodicSessionID-r15			PeriodicSessionID-r15,

...,

[[

updateCapabilities-r15		UpdateCapabilities-r15		OPTIONAL	 -- Need ON

]]

}

PeriodicSessionID-r15 ::= SEQUENCE {

periodicSessionInitiator-r15	ENUMERATED { locationServer, targetDevice, ... },

periodicSessionNumber-r15		INTEGER (0..255),

...

}

UpdateCapabilities-r15 ::= BIT STRING {primaryCellID-r15	(0)} (SIZE(1..8))

-- ASN1STOP

PeriodicAssistanceDataControlParameters field descriptions

periodicSessionID This field identifies a particular periodic assistance data delivery session and the initiator of the session.

updateCapabilities This field identifies the capabilities of the sending entity to support an update of periodic assistance data. A bit value set to one indicates a capability is supported and a bit value set to zero indicates a capability is not supported.

#### –	Polygon

The IE Polygon is used to describe a geographic shape as defined in TS 23.032 [15].

-- ASN1START

Polygon ::= SEQUENCE (SIZE (3..15)) OF PolygonPoints

PolygonPoints ::= SEQUENCE {

latitudeSign				ENUMERATED {north, south},

degreesLatitude				INTEGER (0..8388607),			-- 23 bit field

degreesLongitude			INTEGER (-8388608..8388607)		-- 24 bit field

}

-- ASN1STOP

#### –	PositioningModes

The IE PositioningModes is used to indicate several positioning modes using a bit map.

-- ASN1START

PositioningModes ::= SEQUENCE {

posModes		BIT STRING {	standalone	(0),

ue-based	(1),

ue-assisted	(2)

} (SIZE (1..8)),

...

}

-- ASN1STOP

PositioningModes field descriptions

posModes This field specifies the positioning mode(s). This is represented by a bit string, with a one-value at the bit position means the particular positioning mode is addressed; a zero-value means not addressed.

#### –	SegmentationInfo

The IE SegmentationInfo is used by a sender to indicate that LPP message segmentation is used, as specified in clause 4.3.5.

-- ASN1START

SegmentationInfo-r14 ::= ENUMERATED { noMoreMessages, moreMessagesOnTheWay }

-- ASN1STOP

SegmentationInfo field descriptions

SegmentationInfo noMoreMessages indicates that this is the only or last LPP message segment used to deliver the entire message body. moreMessagesOnTheWay indicates that this is one of multiple LPP message segments used to deliver the entire message body.

#### –	VelocityTypes

The IE VelocityTypes defines a list of possible velocity shapes as defined in TS 23.032 [15].

-- ASN1START

VelocityTypes ::= SEQUENCE {

horizontalVelocity										BOOLEAN,

horizontalWithVerticalVelocity							BOOLEAN,

horizontalVelocityWithUncertainty						BOOLEAN,

horizontalWithVerticalVelocityAndUncertainty			BOOLEAN,

...

}

-- ASN1STOP

### 6.4.2	Common Positioning

#### –	CommonIEsRequestCapabilities

The CommonIEsRequestCapabilities carries common IEs for a Request Capabilities LPP message Type.

-- ASN1START

CommonIEsRequestCapabilities ::= SEQUENCE {

...,

[[

lpp-message-segmentation-req-r14	BIT STRING {	serverToTarget	(0),

targetToServer	(1) }	OPTIONAL -- Need ON

]]

}

-- ASN1STOP

CommonIEsRequestCapabilities field descriptions

lpp-message-segmentation-req This field, if present, indicates that the target device is requested to provide its LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the server is able to send segmented LPP messages to the target device; if bit 0 is set to value 0 it indicates that the server is not able to send segmented LPP messages to the target device. If bit 1 is set to value 1, it indicates that the server is able to receive segmented LPP messages from the target device; if bit 1 is set to value 0 it indicates that the server is not able to receive segmented LPP messages from the target device.

#### –	CommonIEsProvideCapabilities

The CommonIEsProvideCapabilities carries common IEs for a Provide Capabilities LPP message Type.

-- ASN1START

CommonIEsProvideCapabilities ::= SEQUENCE {

...,

[[

segmentationInfo-r14			SegmentationInfo-r14			OPTIONAL,	-- Cond Segmentation

lpp-message-segmentation-r14	BIT STRING { serverToTarget	(0),

targetToServer	(1) }	OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |

CommonIEsProvideCapabilities field descriptions

segmentationInfo This field indicates whether this ProvideCapabilities message is one of many segments, as specified in clause 4.3.5.

lpp-message-segmentation This field, if present, indicates the target device's LPP message segmentation capabilities. If bit 0 is set to value 1, it indicates that the target device supports receiving segmented LPP messages; if bit 0 is set to value 0 it indicates that the target device does not support receiving segmented LPP messages. If bit 1 is set to value 1, it indicates that the target device supports sending segmented LPP messages; if bit 1 is set to value 0 it indicates that the target device does not support sending segmented LPP messages.

#### –	CommonIEsRequestAssistanceData

The CommonIEsRequestAssistanceData carries common IEs for a Request Assistance Data LPP message Type.

-- ASN1START

CommonIEsRequestAssistanceData ::= SEQUENCE {

primaryCellID		ECGI		OPTIONAL,	-- Cond EUTRA

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Cond Segmentation

]],

[[

periodicAssistanceDataReq-r15

PeriodicAssistanceDataControlParameters-r15

OPTIONAL,	-- Cond PerADreq

primaryCellID-r15			NCGI-r15					OPTIONAL	-- Cond NR

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EUTRA | The field is mandatory present for E-UTRA or NB-IoT access. The field shall be omitted for non-EUTRA and non-NB-IoT user plane support. |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |
| PerADreq | The field is mandatory present if the target device requests periodic assistance data delivery. Otherwise it is not present. |
| NR | The field is mandatory present for NR access. The field shall be omitted for non-NR user plane support. |

CommonIEsRequestAssistanceData field descriptions

primaryCellID This parameter identifies the current primary cell for the target device.

segmentationInfo This field indicates whether this RequestAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceDataReq This field indicates a request for periodic assistance data delivery, as specified in clause 5.2.1a.

#### –	CommonIEsProvideAssistanceData

The CommonIEsProvideAssistanceData carries common IEs for a Provide Assistance Data LPP message Type.

-- ASN1START

CommonIEsProvideAssistanceData ::= SEQUENCE {

...,

[[

segmentationInfo-r14		SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

periodicAssistanceData-r15	PeriodicAssistanceDataControlParameters-r15

OPTIONAL	-- Cond PerAD

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| PerAD | The field is mandatory present in a periodic assistance data delivery session. Otherwise it is not present. |

CommonIEsRequestAssistanceData field descriptions

segmentationInfo This field indicates whether this ProvideAssistanceData message is one of many segments, as specified in clause 4.3.5.

periodicAssistanceData This field indicates a periodic assistance data delivery, as specified in clauses 5.2.1a and 5.2.2a.

#### –	CommonIEsRequestLocationInformation

The CommonIEsRequestLocationInformation carries common IEs for a Request Location Information LPP message Type.

-- ASN1START

CommonIEsRequestLocationInformation ::= SEQUENCE {

locationInformationType		LocationInformationType,

triggeredReporting			TriggeredReportingCriteria	OPTIONAL,	-- Cond ECID

periodicalReporting			PeriodicalReportingCriteria OPTIONAL,	-- Need ON

additionalInformation		AdditionalInformation		OPTIONAL,	-- Need ON

qos							QoS							OPTIONAL,	-- Need ON

environment					Environment					OPTIONAL,	-- Need ON

locationCoordinateTypes		LocationCoordinateTypes		OPTIONAL,	-- Need ON

velocityTypes				VelocityTypes				OPTIONAL,	-- Need ON

...,

[[

messageSizeLimitNB-r14	MessageSizeLimitNB-r14		OPTIONAL	-- Need ON

]],

[[

segmentationInfo-r14	SegmentationInfo-r14		OPTIONAL	-- Need ON

]],

[[

scheduledLocationRequest-r17

ScheduledLocationRequest-r17

OPTIONAL,	-- Need ON

integrityInformationRequest-r17

IntegrityInformationRequest-r17

OPTIONAL	-- Need ON

]]

}

LocationInformationType ::= ENUMERATED {

locationEstimateRequired,

locationMeasurementsRequired,

locationEstimatePreferred,

locationMeasurementsPreferred,

...

}

PeriodicalReportingCriteria ::=		SEQUENCE {

reportingAmount						ENUMERATED {

ra1, ra2, ra4, ra8, ra16, ra32,

ra64, ra-Infinity

} DEFAULT ra-Infinity,

reportingInterval					ENUMERATED {

noPeriodicalReporting, ri0-25,

ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64

}

}

TriggeredReportingCriteria ::=		SEQUENCE {

cellChange							BOOLEAN,

reportingDuration					ReportingDuration,

...

}

ReportingDuration ::=				INTEGER (0..255)

AdditionalInformation ::= ENUMERATED {

onlyReturnInformationRequested,

mayReturnAditionalInformation,

...

}

QoS ::= SEQUENCE {

horizontalAccuracy			HorizontalAccuracy		OPTIONAL,	-- Need ON

verticalCoordinateRequest	BOOLEAN,

verticalAccuracy			VerticalAccuracy		OPTIONAL,	-- Need ON

responseTime				ResponseTime			OPTIONAL,	-- Need ON

velocityRequest				BOOLEAN,

...,

[[	responseTimeNB-r14		ResponseTimeNB-r14		OPTIONAL	-- Need ON

]],

[[	horizontalAccuracyExt-r15	HorizontalAccuracyExt-r15		OPTIONAL,	-- Need ON

verticalAccuracyExt-r15		VerticalAccuracyExt-r15			OPTIONAL	-- Need ON

]]

}

HorizontalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

VerticalAccuracy ::= SEQUENCE {

accuracy		INTEGER(0..127),

confidence		INTEGER(0..100),

...

}

HorizontalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

VerticalAccuracyExt-r15 ::= SEQUENCE {

accuracyExt-r15		INTEGER(0..255),

confidence-r15		INTEGER(0..100),

...

}

ResponseTime ::= SEQUENCE {

time								INTEGER (1..128),

...,

[[	responseTimeEarlyFix-r12		INTEGER (1..128)		OPTIONAL		-- Need ON

]],

[[	unit-r15				ENUMERATED { ten-seconds, ... , ten-milli-seconds-r17 }																OPTIONAL		-- Need ON

]]

}

ResponseTimeNB-r14 ::= SEQUENCE {

timeNB-r14							INTEGER (1..512),

responseTimeEarlyFixNB-r14			INTEGER (1..512)		OPTIONAL,		-- Need ON

...,

[[	unitNB-r15				ENUMERATED { ten-seconds, ... }	OPTIONAL		-- Need ON

]]

}

Environment ::= ENUMERATED {

badArea,

notBadArea,

mixedArea,

...

}

MessageSizeLimitNB-r14 ::= SEQUENCE {

measurementLimit-r14				INTEGER (1..512)		OPTIONAL,		-- Need ON

...

}

ScheduledLocationRequest-r17 ::= SEQUENCE {

scheduledLocationTime-r17		ScheduledLocationTime-r17,

...

}

ScheduledLocationTime-r17 ::= SEQUENCE {

utcTime-r17			UTCTime,

gnssTime-r17		SEQUENCE {

gnss-TOD-msec-r17		INTEGER (0..3599999),

gnss-TimeID-r17			GNSS-ID

},

networkTime-r17	CHOICE {

e-utraTime-r17			SEQUENCE {

lte-physCellId-r17			INTEGER (0..503),

lte-arfcnEUTRA-r17			ARFCN-ValueEUTRA,

lte-cellGlobalId-r17		CellGlobalIdEUTRA-AndUTRA

OPTIONAL,	-- Need ON

lte-systemFrameNumber-r17	INTEGER (0..1023)

},

nrTime-r17				SEQUENCE {

nr-PhysCellID-r17			NR-PhysCellID-r16,

nr-ARFCN-r17				ARFCN-ValueNR-r15,

nr-CellGlobalID-r17		NCGI-r15		OPTIONAL,	-- Need ON

nr-SFN-r17					INTEGER (0..1023),

nr-Slot-r17 				CHOICE {

scs15-r17			INTEGER (0..9),

scs30-r17			INTEGER (0..19),

scs60-r17			INTEGER (0..39),

scs120-r17			INTEGER (0..79)

}								OPTIONAL	-- Need ON

},

...

},

relativeTime-r17	INTEGER (1..1024),

...

}

IntegrityInformationRequest-r17 ::= SEQUENCE {

targetIntegrityRisk-r17		INTEGER (10..90),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ECID | The field is optionally present, need ON, if E-CID or NR E-CID is requested. Otherwise it is not present. |

CommonIEsRequestLocationInformation field descriptions

locationInformationType This IE indicates whether the server requires a location estimate or measurements. For 'locationEstimateRequired', the target device shall return a location estimate if possible, or indicate a location error if not possible. For 'locationMeasurementsRequired', the target device shall return measurements if possible, or indicate a location error if not possible. For 'locationEstimatePreferred', the target device shall return a location estimate if possible, but may also or instead return measurements for any requested position methods for which a location estimate is not possible. For 'locationMeasurementsPreferred', the target device shall return location measurements if possible, but may also or instead return a location estimate for any requested position methods for which return of location measurements is not possible.

triggeredReporting This IE indicates that triggered reporting is requested and comprises the following subfields: - cellChange: If this field is set to TRUE, the target device provides requested location information each time the primary cell has changed. - reportingDuration: Maximum duration of triggered reporting in seconds. A value of zero is interpreted to mean an unlimited (i.e. "infinite") duration. The target device should continue triggered reporting for the reportingDuration or until an LPP Abort or LPP Error message is received. The triggeredReporting field should not be included by the location server and shall be ignored by the target device if the periodicalReporting IE or responseTime IE or responseTimeNB IE is included in CommonIEsRequestLocationInformation.

periodicalReporting This IE indicates that periodic reporting is requested and comprises the following subfields: - reportingAmount indicates the number of periodic location information reports requested. Enumerated values correspond to 1, 2, 4, 8, 16, 32, 64, or infinite/indefinite number of reports. If the reportingAmount is 'infinite/indefinite', the target device shou-ld continue periodic reporting until an LPP Abort message is received. The value 'ra1' shall not be used by a sender. - reportingInterval indicates the interval between location information reports and the response time requirement for the first location information report. Enumerated values ri0-25, ri0-5, ri1, ri2, ri4, ri8, ri16, ri32, ri64 correspond to reporting intervals of 1, 2, 4, 8, 10, 16, 20, 32, and 64 seconds, respectively. Measurement reports containing no measurements or no location estimate are required when a reportingInterval expires before a target device is able to obtain new measurements or obtain a new location estimate. The value 'noPeriodicalReporting' shall not be used by a sender.

additionalInformation This IE indicates whether a target device is allowed to return additional information to that requested. If this IE indicates 'onlyReturnInformationRequested' then the target device shall not return any additional information to that requested by the server. If this IE indicates 'mayReturnAdditionalInformation' then the target device may return additional information to that requested by the server. If a location estimate is returned, any additional information is restricted to that associated with a location estimate (e.g. might include velocity if velocity was not requested but cannot include measurements). If measurements are returned, any additional information is restricted to additional measurements (e.g. might include E-CID measurements if A-GNSS measurements were requested but not E-CID measurements).

qos This IE indicates the quality of service and comprises a number of sub-fields. In the case of measurements, some of the sub-fields apply to the location estimate that could be obtained by the server from the measurements provided by the target device assuming that the measurements are the only sources of error. Fields are as follows: - horizontalAccuracy indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracy' corresponds to the encoded uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - verticalCoordinateRequest indicates whether a vertical coordinate is required (TRUE) or not (FALSE) - verticalAccuracy indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracy' corresponds to the encoded uncertainty altitude as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. - responseTime - time indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. If the periodicalReporting IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - responseTimeEarlyFix indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unit field is absent, this is given as an integer number of seconds between 1 and 128. If the unit field is present with enumerated value 'ten-seconds', the maximum response time is given in units of 10-seconds, between 10 and 1280 seconds. If the unit field is present with enumerated value 'ten-milli-seconds', the maximum response time is given in units of 10-milli-seconds, between 0.01 and 1.28 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFix IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the time IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFix IE. A server should set the responseTimeEarlyFix IE to a value less than that for the time IE. A target shall ignore the responseTimeEarlyFix IE if its value is not less than that for the time IE. - unit indicates the unit of the time and responseTimeEarlyFix fields. Enumerated value 'ten-seconds' corresponds to a resolution of 10 seconds. Enumerated value 'ten-milli-seconds' corresponds to a resolution of 0.01 seconds. If this field is absent, the unit/resolution is 1 second. Enumerated value 'ten-milli-seconds' is only applicable for NR E-CID Positioning, NR DL-TDOA Positioning, NR DL-AoD Positioning, and NR Multi-RTT Positioning. - velocityRequest indicates whether velocity (or measurements related to velocity) is requested (TRUE) or not (FALSE). - responseTimeNB If the periodicalReporting IE or responseTime IE is included in CommonIEsRequestLocationInformation, this field should not be included by the location server and shall be ignored by the target device (if included). - timeNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. - responseTimeEarlyFixNB indicates the maximum response time as measured between receipt of the RequestLocationInformation and transmission of a ProvideLocationInformation containing early location measurements or an early location estimate. If the unitNB field is absent, this is given as an integer number of seconds between 1 and 512. If the unitNB field is present, the maximum response time is given in units of 10-seconds, between 10 and 5120 seconds. When this IE is included, a target should send a ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing early location information according to the responseTimeEarlyFixNB IE and a subsequent ProvideLocationInformation (or more than one ProvideLocationInformation if location information will not fit into a single message) containing final location information according to the timeNB IE. A target shall omit sending a ProvideLocationInformation if the early location information is not available at the expiration of the time value in the responseTimeEarlyFixNB IE. A server should set the responseTimeEarlyFixNB IE to a value less than that for the timeNB IE. A target shall ignore the responseTimeEarlyFixNB IE if its value is not less than that for the timeNB IE. - unitNB indicates the unit of the timeNB and responseTimeEarlyFixNB fields. Enumerated value 'ten-second' corresponds to a resolution of 10 seconds. If this field is absent, the unit/resolution is 1 second. - horizontalAccuracyExt indicates the maximum horizontal error in the location estimate at an indicated confidence level. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the horizontalAccuracy field is included in QoS. - verticalAccuracyExt indicates the maximum vertical error in the location estimate at an indicated confidence level and is only applicable when a vertical coordinate is requested. The 'accuracyExt' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'confidence' corresponds to confidence as defined in TS 23.032 [15]. This field should not be included by the location server and shall be ignored by the target device if the verticalAccuracy field is included in QoS. All QoS requirements shall be obtained by the target device to the degree possible but it is permitted to return a response that does not fulfill all QoS requirements if some were not attainable. The single exception is time and timeNB which shall always be fulfilled – even if that means not fulfilling other QoS requirements. A target device supporting NB-IoT access shall support the responseTimeNB IE. A target device supporting HA GNSS shall support the HorizontalAccuracyExt, VerticalAccuracyEx, and unit fields with enumerated value 'ten-seconds'. A target device supporting NB-IoT access and HA GNSS shall support the unitNB field.

environment This field provides the target device with information about expected multipath and non line of sight (NLOS) in the current area. The following values are defined: - badArea: possibly heavy multipath and NLOS conditions (e.g. bad urban or urban). - notBadArea: no or light multipath and usually LOS conditions (e.g. suburban or rural). - mixedArea: environment that is mixed or not defined. If this field is absent, a default value of 'mixedArea' applies.

locationCoordinateTypes This field provides a list of the types of location estimate that the target device may return when a location estimate is obtained by the target.

velocityTypes This fields provides a list of the types of velocity estimate that the target device may return when a velocity estimate is obtained by the target.

messageSizeLimitNB This field provides an octet limit on the amount of location information a target device can return. - measurementLimit indicates the maximum amount of location information the target device should return in response to the RequestLocationInformation message received from the location server. The limit applies to the overall size of the LPP message at LPP level (LPP Provide Location Information), and is specified in steps of 100 octets. The message size limit is then given by the value provided in measurementLimit times 100 octets.

segmentationInfo This field indicates whether this RequestLocationInformation message is one of many segments, as specified in clause 4.3.5

scheduledLocationRequest This field indicates that the target device is requested to obtain location measurements or location estimate valid at the scheduledLocationTime T and comprises the following subfields: - scheduledLocationTime indicates the time T when the location measurements or location estimate is to be obtained. - utcTime provides T in UTC in the form of YYMMDDhhmmssZ. - gnssTime provides T in GNSS system time of the GNSS indicated by gnss-TimeID. - gnss-TOD-msec specifies the GNSS TOD in 1-milli-second resolution rounded down to the nearest millisecond unit. - networkTime provides T in E-UTRA or NR network time. - lte-physCellId, lte-arfcnEUTRA, lte-cellGlobalId identifies the reference cell (E-UTRA) that is used for the network time. - lte-systemFrameNumber specifies the system frame number in E-UTRA. - nr-PhysCellID, nr-ARFCN , nr-CellGlobalID identifies the reference cell (NR) that is used for the network time. - nr-SFN specifies the system frame number in NR. - nr-Slot specifies the slot number in NR for the indicated subcarrier spacing (SCS). The total NR network time is given by nr-SFN + nr-Slot. - relativeTime provides T in seconds from current time, where current time is defined as the time the CommonIEsRequestLocationInformation was received. NOTE: A location estimate returned to an LCS Client, AF or UE for a scheduled location time can be treated by the LCS Client, AF or UE as an estimate of the location of the UE at the scheduled location time (see TS 23.273 [42]).

integrityInformationRequest This field, if present, indicates that the target device is requested to report integrity information for the location estimate and comprises the following integrity requirements: - targetIntegrityRisk indicates the Target Integrity Risk (TIR) for which the Protection Level (PL) is requested. The TIR is given by P=10-0.1n [hour-1] where n is the value of targetIntegrityRisk and the range is 10-1 to 10-9 per hour.

#### –	CommonIEsProvideLocationInformation

The CommonIEsProvideLocationInformation carries common IEs for a Provide Location Information LPP message Type.

-- ASN1START

CommonIEsProvideLocationInformation ::= SEQUENCE {

locationEstimate			LocationCoordinates		OPTIONAL,

velocityEstimate			Velocity				OPTIONAL,

locationError				LocationError			OPTIONAL,

...,

[[	earlyFixReport-r12		EarlyFixReport-r12		OPTIONAL

]],

[[	locationSource-r13		LocationSource-r13		OPTIONAL,

locationTimestamp-r13	UTCTime					OPTIONAL

]],

[[

segmentationInfo-r14	SegmentationInfo-r14	OPTIONAL		-- Cond Segmentation

]],

[[

integrityInfo-r17		IntegrityInfo-r17		OPTIONAL

]]

}

LocationCoordinates ::= CHOICE {

ellipsoidPoint								Ellipsoid-Point,

ellipsoidPointWithUncertaintyCircle			Ellipsoid-PointWithUncertaintyCircle,

ellipsoidPointWithUncertaintyEllipse		EllipsoidPointWithUncertaintyEllipse,

polygon										Polygon,

ellipsoidPointWithAltitude					EllipsoidPointWithAltitude,

ellipsoidPointWithAltitudeAndUncertaintyEllipsoid

EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ellipsoidArc								EllipsoidArc,

...,

highAccuracyEllipsoidPointWithUncertaintyEllipse-v1510

HighAccuracyEllipsoidPointWithUncertaintyEllipse-r15,

highAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-v1510

HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

ha-EllipsoidPointWithScalableUncertaintyEllipse-v1680																				HA-EllipsoidPointWithScalableUncertaintyEllipse-r16,

ha-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-v1680

HA-EllipsoidPointWithAltitudeAndScalableUncertaintyEllipsoid-r16

}

Velocity ::= CHOICE {

horizontalVelocity							HorizontalVelocity,

horizontalWithVerticalVelocity				HorizontalWithVerticalVelocity,

horizontalVelocityWithUncertainty			HorizontalVelocityWithUncertainty,

horizontalWithVerticalVelocityAndUncertainty

HorizontalWithVerticalVelocityAndUncertainty,

...

}

LocationError ::= SEQUENCE {

locationfailurecause			LocationFailureCause,

...

}

LocationFailureCause ::= ENUMERATED {

undefined,

requestedMethodNotSupported,

positionMethodFailure,

periodicLocationMeasurementsNotAvailable,

...

}

EarlyFixReport-r12 ::= ENUMERATED {

noMoreMessages,

moreMessagesOnTheWay

}

LocationSource-r13 ::= BIT STRING {	a-gnss				(0),

wlan				(1),

bt					(2),

tbs					(3),

sensor				(4),

ha-gnss-v1510		(5),

motion-sensor-v1550 (6),

dl-tdoa-r16 		(7),

dl-aod-r16			(8) } (SIZE(1..16))

IntegrityInfo-r17 ::= SEQUENCE {

horizontalProtectionLevel-r17		INTEGER (0..50000),

verticalProtectionLevel-r17			INTEGER (0..50000)				OPTIONAL,

achievableTargetIntegrityRisk-r17	INTEGER (10..90)				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Segmentation | This field is optionally present, need OP, if lpp-message-segmentation-req has been received from the location server with bit 1 (targetToServer) set to value 1. The field shall be omitted if lpp-message-segmentation-req has not been received in this location session, or has been received with bit 1 (targetToServer) set to value 0. |

CommonIEsProvideLocationInformation field descriptions

locationEstimate This field provides a location estimate using one of the geographic shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each geographic shape follow the rules in TS 23.032 [15]. The conditions for including this field are defined for the locationInformationType field in a Request Location Information message.

velocityEstimate This field provides a velocity estimate using one of the velocity shapes defined in TS 23.032 [15]. Coding of the values of the various fields internal to each velocity shape follow the rules in TS 23.032 [15].

locationError This field shall be included if and only if a location estimate and measurements are not included in the LPP PDU. The field includes information concerning the reason for the lack of location information. The LocationFailureCause 'periodicLocationMeasurementsNotAvailable' shall be used by the target device if periodic location reporting was requested, but no measurements or location estimate are available when the reportingInterval expired.

earlyFixReport This field shall be included if and only if the ProvideLocationInformation message contains early location measurements or an early location estimate. The target device shall set the values of this field as follows: - noMoreMessages: This is the only or last ProvideLocationInformation message used to deliver the entire set of early location information. - moreMessagesOnTheWay: This is one of multiple ProvideLocationInformation messages used to deliver the entire set of early location information (if early location information will not fit into a single message). If this field is included, the IE SegmentationInfo shall not be included.

locationSource This field provides the source positioning technology for the location estimate. NOTE 1: In this version of the specification, the entry 'tbs' is used only for TBS positioning based on MBS signals. NOTE 2: The entry 'sensor' is used only for positioning technology that uses barometric pressure sensor. The entry 'motion-sensor' is used for positioning technology that uses sensor(s) to detect displacement and movement, e.g. accelerometers, gyros, magnetometers.

locationTimestamp This field provides the UTC time when the location estimate is valid and should take the form of YYMMDDhhmmssZ.

segmentationInfo This field indicates whether this ProvideLocationInformation message is one of many segments, as specified in clause 4.3.5

integrityInfo This field provides the integrity result for the locationEstimate. - horizontalProtectionLevel provides the Horizontal Protection Level (HPL) for the locationEstimate along the semi-major axis of the error ellipse. Scale factor 0.01 metre; range 0 – 500 metres. - verticalProtectionLevel provides the Vertical Protection Level (VPL) for the locationEstimate. Scale factor 0.01 metre; range 0 – 500 metres. - achievableTargetIntegrityRisk indicates the achievable Target Integrity Risk (TIR) for which the HPL and VPL are provided. The achievable TIR is given by P=10-0.1n [hour-1] where n is the value of achievableTargetIntegrityRisk and the range is 10-1 to 10-9 per hour. If this field is absent, the achievable TIR is the same as the targetIntegrityRisk in IntegrityInformationRequest.

NOTE: 	The Protection Level (PL) is a statistical upper-bound of the Positioning Error (PE) that ensures that, the probability per unit of time of the true error being greater than the AL and the PL being less than or equal to the AL, for longer than the TTA, is less than the required TIR, i.e., the PL satisfies the following inequality: 
Prob per unit of time [((PE>AL) & (PL<=AL)) for longer than TTA] < required TIR
When the PL bounds the positioning error in the horizontal plane or on the vertical axis then it is called Horizontal Protection Level (HPL) or Vertical Protection Level (VPL) respectively.
A specific equation for the PL is not specified as this is implementation-defined. For the PL to be considered valid, it must simply satisfy the inequality above.

#### –	CommonIEsAbort

The CommonIEsAbort carries common IEs for an Abort LPP message Type.

-- ASN1START

CommonIEsAbort ::= SEQUENCE {

abortCause			ENUMERATED {

undefined,

stopPeriodicReporting,

targetDeviceAbort,

networkAbort,

...,

stopPeriodicAssistanceDataDelivery-v1510

}

}

-- ASN1STOP

CommonIEsAbort field descriptions

abortCause This IE defines the request to abort an ongoing procedure. The abort cause 'stopPeriodicReporting' should be used by the location server to stop any ongoing location reporting configured as periodicalReporting or triggeredReporting in the CommonIEsRequestLocationInformation. The abort cause 'stopPeriodicAssistanceDataDelivery' should be used by the location server or target device to stop any ongoing periodic assistance data delivery, as specified in clauses 5.2.1a and 5.2.2a.

#### –	CommonIEsError

The CommonIEsError carries common IEs for an Error LPP message Type.

-- ASN1START

CommonIEsError ::= SEQUENCE {

errorCause		ENUMERATED {

undefined,

lppMessageHeaderError,

lppMessageBodyError,

epduError,

incorrectDataValue,

...,

lppSegmentationError-v1450

}

}

-- ASN1STOP

CommonIEsError field descriptions

errorCause This IE defines the cause for an error. 'lppMessageHeaderError', 'lppMessageBodyError' and 'epduError' is used if a receiver is able to detect a coding error in the LPP header (i.e., in the common fields), LPP message body or in an EPDU, respectively. 'lppSegmentationError' is used if a receiver detects an error in LPP message segmentation.

### 6.4.3	Common NR Positioning Information Elements

#### –	Area-ID-CellList

The IE Area-ID-CellList provides the NR Cell-IDs of the TRPs belonging to a particular network area.

-- ASN1START

Area-ID-CellList-r17 ::= SEQUENCE {

area-id-r17				INTEGER (1..maxAreaIDs-r17),

nr-cell-ID-List-r17		NR-Cell-ID-List-r17,

...

}

NR-Cell-ID-List-r17 ::= SEQUENCE (SIZE(1..maxCellIDsIDsPerArea-r17)) OF NR-Cell-IDs-r17

NR-Cell-IDs-r17 ::= SEQUENCE {

nr-CellGlobalID-r17				NCGI-r15					OPTIONAL,	-- Need ON

nr-PhysCellID-r17				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-ARFCN-r17					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

Area-ID-CellList field descriptions

area-id This field specifies the Area ID of the network area to which the TRPs in the nr-cell-ID-List belongs to.

nr-cell-ID-List This field provides the Cell-IDs of the TRPs belonging to the network area identified by area-id.

#### –	DL-PRS-ID-Info

The IE DL-PRS-ID-Info provides the IDs of the reference TRPs' DL-PRS Resources.

-- ASN1START

DL-PRS-ID-Info-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-DL-PRS-ResourceID-List-r16	SEQUENCE (SIZE (1..nrMaxResourceIDs-r16)) OF

NR-DL-PRS-ResourceID-r16

OPTIONAL, -- Need ON

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16

OPTIONAL  -- Need ON

}

-- ASN1STOP

DL-PRS-ID-Info field descriptions

nr-DL-PRS-ResourceID-List This field provides a list of DL-PRS Resource IDs under the same DL-PRS Resource Set.

#### –	LCS-GCS-TranslationParameter

The IE LCS-GCS-TranslationParameter provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44].

-- ASN1START

LCS-GCS-TranslationParameter-r16 ::= SEQUENCE {

alpha-r16						INTEGER (0..359),

alpha-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

beta-r16						INTEGER (0..359),

beta-fine-r16					INTEGER (0..9)					OPTIONAL,	-- Cond AzElFine

gamma-r16						INTEGER (0..359),

gamma-fine-r16					INTEGER (0..9) 					OPTIONAL,	-- Cond AzElFine

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| AzElFine | The field is mandatory present if the angles where this IE is used are provided with 0.1 degrees resolution; otherwise it is not present. |

LCS-GCS-TranslationParameter field descriptions

alpha This field specifies the bearing angle α for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 1 degree; range 0 to 359 degrees.

alpha-fine This field provides finer granularity for the alpha. The total bearing angle α is given by alpha + alpha-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

beta This field specifies the downtilt angle β for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 1 degree; range 0 to 359 degrees.

beta-fine This field provides finer granularity for the beta. The total downtilt angle β is given by beta + beta-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

gamma This field specifies the slant angle γ for the translation of the LCS to a GCS as defined in TR 38.901 [44]. Scale factor 1 degree; range 0 to 359 degrees.

gamma-fine This field provides finer granularity for the gamma. The total slant angle γ is given by gamma + gamma-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

#### –	LOS-NLOS-Indicator

The IE LOS-NLOS-Indicator provides information on the likelihood of a Line-of-Sight (LOS) propagation path from the source to the receiver.

-- ASN1START

LOS-NLOS-Indicator-r17 ::= SEQUENCE {

indicator-r17			CHOICE {

soft-r17				INTEGER (0..10),

hard-r17				BOOLEAN,

...

},

...

}

-- ASN1STOP

LOS-NLOS-Indicator field descriptions

indicator This field provides information on the likelihood of a Line-of-Sight propagation path from the source to the receiver and has the following choices: - soft: This field specifies the likelihood of a LOS propagation path in the range between 0 and 1 with 0.1 steps resolution. Value '0' indicates NLOS and value '1' indicates LOS. The value provides an estimate of the propability for a LOS propagation path between source and receiver. Scale factor 0.1; range 0 to 1. - hard: This field specifies whether the propagation path between source and receiver is estimated to be LOS (true) or NLOS (false).

#### –	NR-AdditionalPathList

The IE NR-AdditionalPathList is used by the target device to provide information about additional paths in association to the TOA measurements associated to NR positioning in the form of a relative time difference and a quality value. The additional path nr-relativeTimeDifference is the detected path timing relative to the detected path timing used for the TOA value, and each additional path can be associated with a quality value nr-path-Quality.

-- ASN1START

NR-AdditionalPathList-r16 ::= SEQUENCE (SIZE(1..2)) OF NR-AdditionalPath-r16

NR-AdditionalPathListExt-r17 ::= SEQUENCE (SIZE(1..8)) OF NR-AdditionalPath-r16

NR-AdditionalPath-r16 ::= SEQUENCE {

nr-RelativeTimeDifference-r16	CHOICE {

k0-r16					INTEGER(0..16351),

k1-r16					INTEGER(0..8176),

k2-r16					INTEGER(0..4088),

k3-r16					INTEGER(0..2044),

k4-r16					INTEGER(0..1022),

k5-r16					INTEGER(0..511),

...

},

nr-PathQuality-r16				NR-TimingQuality-r16					OPTIONAL,

...,

[[

nr-DL-PRS-RSRPP-r17				INTEGER (0..126)						OPTIONAL

]]

}

-- ASN1STOP

NR-AdditionalPathListfield descriptions

nr-RelativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing of the reference resource. The mapping of reported values and measured quantity value is defined in TS 38.133 [46] clause 10.1.23.3.3 and 10.1.25.3.3. A positive value indicates that the particular path is later in time than the detected path of the reference; a negative value indicates that the particular path is earlier in time than the detected path of the reference.

nr-PathQuality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.

nr-DL-PRS-RSRPP This field specifies the DL PRS reference signal received path power (DL PRS-RSRPP) of the NR-AdditionalPath reported, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46]. [FFS]

#### –	NR-DL-PRS-AssistanceData

The IE NR-DL-PRS-AssistanceData is used by the location server to provide DL-PRS assistance data.

NOTE 1:	The location server should include at least one TRP for which the SFN can be obtained by the target device, e.g. the serving TRP.

NOTE 2:	The nr-DL-PRS-ReferenceInfo defines the "assistance data reference" TRP whose DL-PRS configuration is included in nr-DL-PRS-AssistanceDataList. The nr-DL-PRS-SFN0-Offset's and nr-DL-PRS-expectedRSTD's in nr-DL-PRS-AssistanceDataList are provided relative to the "assistance data reference" TRP.

NOTE 3:	The network signals a value of zero for the nr-DL-PRS-SFN0-Offset, nr-DL-PRS-expectedRSTD, and nr-DL-PRS-expectedRSTD-uncertainty of the "assistance data reference" TRP in nr-DL-PRS-AssistanceDataList.

NOTE 4:	For NR DL-TDOA positioning (see clause 6.5.10) the nr-DL-PRS-ReferenceInfo defines also the requested "RSTD reference".

For DL-PRS processing, the LPP layer may inform lower layers to start performing DL-PRS measurements and provide to lower layers the information about the location of DL-PRS, e.g. DL-PRS-PointA, DL-PRS Positioning occasion information.

-- ASN1START

NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-ReferenceInfo-r16 		DL-PRS-ID-Info-r16,

nr-DL-PRS-AssistanceDataList-r16	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-AssistanceDataPerFreq-r16,

nr-SSB-Config-r16					SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF

NR-SSB-Config-r16	OPTIONAL,	-- Need ON

...

}

NR-DL-PRS-AssistanceDataPerFreq-r16 ::= SEQUENCE {

nr-DL-PRS-PositioningFrequencyLayer-r16

NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-AssistanceDataPerTRP-r16,

...

}

NR-DL-PRS-AssistanceDataPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-DL-PRS-SFN0-Offset-r16		NR-DL-PRS-SFN0-Offset-r16,

nr-DL-PRS-ExpectedRSTD-r16		INTEGER (-3841..3841),

nr-DL-PRS-ExpectedRSTD-Uncertainty-r16

INTEGER (0..246),

nr-DL-PRS-Info-r16				NR-DL-PRS-Info-r16,

...,

[[

prs-OnlyTP-r16				ENUMERATED { true }		OPTIONAL	-- Need ON

]],

[[

nr-DL-PRS-ExpectedAoD-or-AoA-r17

NR-DL-PRS-ExpectedAoD-or-AoA-r17	OPTIONAL	-- Need ON

]]

}

NR-DL-PRS-PositioningFrequencyLayer-r16 ::= SEQUENCE {

dl-PRS-SubcarrierSpacing-r16	ENUMERATED {kHz15, kHz30, kHz60, kHz120, ...},

dl-PRS-ResourceBandwidth-r16	INTEGER (1..63),

dl-PRS-StartPRB-r16				INTEGER (0..2176),

dl-PRS-PointA-r16				ARFCN-ValueNR-r15,

dl-PRS-CombSizeN-r16			ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-CyclicPrefix-r16			ENUMERATED {normal, extended, ...},

...

}

NR-DL-PRS-SFN0-Offset-r16 ::= SEQUENCE {

sfn-Offset-r16					INTEGER (0..1023),

integerSubframeOffset-r16		INTEGER (0..9),

...

}

NR-DL-PRS-ExpectedAoD-or-AoA-r17 ::= CHOICE {

expectedAoD-r17			SEQUENCE {

expected-DL-Azimuth-AoD-r17		INTEGER (0..3599),

expected-DL-Azimuth-AoD-Unc-r17	INTEGER (0..FFS),

expected-DL-Zenith-AoD-r17		INTEGER (0..1800),

expected-DL-Zenith-AoD-Unc-r17	INTEGER	(0..FFS)

},

expectedAoA-r17			SEQUENCE {

expected-DL-Azimuth-AoA-r17		INTEGER (0..3599),

expected-DL-Azimuth-AoA-Unc-r17	INTEGER (0..FFS),

expected-DL-Zenith-AoA-r17		INTEGER (0..1800),

expected-DL-Zenith-AoA-Unc-r17	INTEGER	(0..FFS)

}

}

-- ASN1STOP

NR-DL-PRS-AssistanceData field descriptions

nr-DL-PRS-ReferenceInfo This field specifies the IDs of the assistance data reference TRP.

nr-DL-PRS-AssistanceDataList This field specifies the DL-PRS resources for each frequency layer.

nr-SSB-Config This field specifies the SSB configuration of the TRPs.

nr-DL-PRS-PositioningFrequencyLayer This field specifies the Positioning Frequency Layer for the nr-DL-PRS-AssistanceDataPerFreq field.

nr-DL-PRS-AssistanceDataPerFreq This field specifies the DL-PRS Resources for the TRPs within the Positioning Frequency Layer.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resource ID to uniquely identify a DL-PRS Resource, and is associated with a single TRP.

nr-PhysCellID This field specifies the physical cell identity of the TRP. When the field prs-OnlyTP is included, this field is not included.

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, as defined in TS 38.331 [35]. When the field prs-OnlyTP is included, this field is not included.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. When the field prs-OnlyTP is included, this field is not included.

nr-DL-PRS-SFN0-Offset This field specifies the time offset of the SFN#0 slot#0 for the given TRP with respect to SFN#0 slot#0 of the assistance data reference TRP and comprises the following subfields: - sfn-Offset specifies the SFN offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the assistance data reference TRP to the beginning of the closest subsequent radio frame #0 of this neighbour TRP. - integerSubframeOffset specifies the frame boundary offset at the TRP antenna location between the assistance data reference TRP and this neighbour TRP counted in full subframes. The offset is counted from the beginning of a subframe #0 of the assistance data reference TRP to the beginning of the closest subsequent subframe #0 of this neighbour TRP, rounded down to multiples of subframes.

nr-DL-PRS-ExpectedRSTD This field indicates the RSTD value that the target device is expected to measure between this TRP and the assistance data reference TRP. The nr-DL-PRS-ExpectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two TRPs. The resolution is 4Ts, with Ts=1/(15000*2048) seconds.

nr-DL-PRS-ExpectedRSTD-Uncertainty This field indicates the uncertainty in nr-DL-PRS-ExpectedRSTD value. The uncertainty is related to the location server′s a-priori estimate of the target device location. The nr-DL-PRS-ExpectedRSTD and nr-DL-PRS-ExpectedRSTD-Uncertainty together define the search window for the target device. The resolution R is - Ts if all PRS resources are in frequency range 2, - 4Ts otherwise, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the subframe for the PRS of this TRP is received within the search window of size - [-nr-DL-PRS-ExpectedRSTD-UncertaintyR ; nr-DL-PRS-ExpectedRSTD-UncertaintyR] centred at TREF+1 millisecondN+nr-DL-PRS-ExpectedRSTD4Ts, where TREF is the reception time of the beginning of the subframe for the PRS of the assistance data reference TRP at the target device antenna connector, and N can be calculated based on - nr-DL-PRS-SFN0-Offset - dl-PRS-Periodicity-and-ResourceSetSlotOffset - dl-PRS-ResourceSlotOffset.

nr-DL-PRS-Info This field specifies the PRS configuration of the TRP.

dl-PRS-SubcarrierSpacing This field specifies the subcarrier spacing of the DL-PRS Resource. 15, 30, 60 kHz for FR1; 60, 120 kHz for FR2. All DL-PRS Resources and DL-PRS Resource Sets in the same Positioning Frequency layer have the same value of dl-PRS-SubcarrierSpacing.

dl-PRS-ResourceBandwidth This field specifies the number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. All DL-PRS Resources of the DL-PRS Resource Set have the same bandwidth. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of DL-PRS Bandwidth and Start PRB. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-PRS-StartPRB This field specifies the start PRB index defined as offset with respect to reference DL-PRS Point A for the Positioning Frequency Layer. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-StartPRB.

dl-PRS-PointA This field specifies the absolute frequency of the reference resource block for the DL-PRS. Its lowest subcarrier is also known as DL-PRS Point A. A single DL-PRS Point A for DL-PRS Resource allocation is provided per Positioning Frequency Layer. All DL-PRS Resources belonging to the same DL-PRS Resource Set have the same DL-PRS Point A.

dl-PRS-CombSizeN This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size N.

dl-PRS-CyclicPrefix This field specifies the Cyclic Prefix length of the DL-PRS Resource. All DL-PRS Resources Sets belonging to the same Positioning Frequency Layer have the same value of dl-PRS-CyclicPrefix.

prs-OnlyTP This field, if present, indicates that the NR-DL-PRS-AssistanceData is provided for a PRS-only TP. Whether the field is present or absent should be the same for all the NR-DL-PRS-AssistanceData of all the PRS transmitted under the same TP. The target device shall not assume that any other signals or physical channels are present for the TRP other than DL-PRS.

nr-DL-PRS-ExpectedAoD-or-AoA This field specifies the expected AoD or AoA at the target device location together with uncertainty. - expected-DL-Azimuth-AoD: This field specifies the expected azimuth angle of departure. Scale factor 0.1 degree; range 0 to 359.9 degrees. - expected-DL-Azimuth-AoD-Unc: This field specifies the (single-sided) uncertainty of the expected azimuth angle of departure. Scale factor 0.1 degree; range 0 to FFS degrees. - expected-DL-Zenith-AoD: This field specifies the expected elevation angle of departure. Scale factor 0.1 degree; range 0 to 180 degrees. - expected-DL-Zenith-AoD-Unc: This field specifies the (single-sided) uncertainty of the expected elevation angle of departure. Scale factor 0.1 degree; range 0 to FFS degrees. - expected-DL-Azimuth-AoA: This field specifies the expected azimuth angle of arrival. Scale factor 0.1 degree; range 0 to 359.9 degrees. - expected-DL-Azimuth-AoA-Unc: This field specifies the (single-sided) uncertainty of the expected azimuth angle of arrival. Scale factor 0.1 degree; range 0 to FFS degrees. - expected-DL-Zenith-AoA: This field specifies the expected elevation angle of arrival. Scale factor 0.1 degree; range 0 to 180 degrees. - expected-DL-Zenith-AoA-Unc: This field specifies the (single-sided) uncertainty of the expected elevation angle of arrival. Scale factor 1 degree; range 0 to FFS degrees. FFS whether this applies for DL-AOD positioning method only; waiting for RAN1 feedback.

#### –	NR-DL-PRS-BeamInfo

The IE NR-DL-PRS-BeamInfo is used by the location server to provide spatial direction information of the DL-PRS Resources.

-- ASN1START

NR-DL-PRS-BeamInfo-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-BeamInfoPerFreqLayer-r16

NR-DL-PRS-BeamInfoPerFreqLayer-r16 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-BeamInfoPerTRP-r16

NR-DL-PRS-BeamInfoPerTRP-r16 ::= SEQUENCE {

dl-PRS-ID-r16						INTEGER (0..255),

nr-PhysCellID-r16					NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r16					NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r16						ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

associated-DL-PRS-ID-r16			INTEGER (0..255)		OPTIONAL,	-- Need OP

lcs-GCS-TranslationParameter-r16	LCS-GCS-TranslationParameter-r16

OPTIONAL,	-- Need OP

dl-PRS-BeamInfoSet-r16				DL-PRS-BeamInfoSet-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-BeamInfoSet-r16 ::= SEQUENCE (SIZE(1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-BeamInfoResourceSet-r16

DL-PRS-BeamInfoResourceSet-r16 ::= SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-BeamInfoElement-r16

DL-PRS-BeamInfoElement-r16 ::= SEQUENCE {

dl-PRS-Azimuth-r16				INTEGER (0..359),

dl-PRS-Azimuth-fine-r16			INTEGER (0..9)					OPTIONAL,	-- Need ON

dl-PRS-Elevation-r16			INTEGER (0..180)				OPTIONAL,	-- Need ON

dl-PRS-Elevation-fine-r16		INTEGER (0..9)					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-DL-PRS-Beam-Info field descriptions

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35]. The server should include this field if it considers that it is needed to resolve ambiguity in the TRP indicated by nr-PhysCellID.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

associated-DL-PRS-ID This field specifies the dl-PRS-ID of the associated TRP from which the beam information and parameters for LCS to GCS translation are adopted. If the field is omitted, the beam information is provided via the dl-prs-BeamInfoSet field and the LCS to GCS translation parameter is provided via the lcs-GCS-TranslationParameter. If the field is present, the fields lcs-GCS-TranslationParameter and dl-PRS-BeamInfoSet shall be absent.

lcs-GCS-TranslationParameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44]. If this field and the field associated-DL-PRS-ID are absent, the dl-PRS-Azimuth and dl-PRS-Elevation are provided in a GCS.

dl-PRS-BeamInfoSet This field provides the DL-PRS beam information for each DL-PRS Resource of the DL-PRS Resource Set associated with this TRP.

dl-PRS-Azimuth This field specifies the azimuth angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 1 degree; range 0 to 359 degrees.

dl-PRS-Azimuth-fine This field provides finer granularity for the dl-PRS-Azimuth. The total azimuth angle of the boresight direction is given by dl-PRS-Azimuth + dl-PRS-Azimuth-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

dl-PRS-Elevation This field specifies the elevation angle of the boresight direction in which the DL-PRS Resources associated with this DL-PRS Resource ID in the DL-PRS Resource Set are transmitted. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 1 degree; range 0 to 180 degrees.

dl-PRS-Elevation-fine This field provides finer granularity for the dl-PRS-Elevation. The total elevation angle of the boresight direction is given by dl-PRS-Elevation + dl-PRS-Elevation-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

#### –	NR-DL-PRS-Expected-LOS-NLOS-Assistance

The IE NR-DL-PRS-Expected-LOS-NLOS-Assistance is used by the location server to provide the expected likelihood of a LOS propagation path from a TRP to the target device.

-- ASN1START

NR-DL-PRS-Expected-LOS-NLOS-Assistance-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-Expected-LOS-NLOS-AssistancePerFreqLayer-r17

NR-DL-PRS-Expected-LOS-NLOS-AssistancePerFreqLayer-r17 ::=

SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-Expected-LOS-NLOS-AssistancePerTRP-r17

NR-DL-PRS-Expected-LOS-NLOS-AssistancePerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17				INTEGER (0..255),

nr-PhysCellID-r17			NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r17			NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r17				ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-los-nlos-indicator-r17	CHOICE {

per-trp-r17			LOS-NLOS-Indicator-r17,

per-resource-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

NR-DL-PRS-Expected-LOS-NLOS-AssistancePerResource-r17,

...

},

...

}

NR-DL-PRS-Expected-LOS-NLOS-AssistancePerResource-r17 ::=

SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

LOS-NLOS-Indicator-r17

-- ASN1STOP

NR-DL-PRS-Expected-LOS-NLOS-Assistance field descriptions

dl-PRS-ID This field specifies the DL-PRS ID of the TRP for which the LOS/NLOS Information is provided.

nr-PhysCellID This field specifies the physical Cell-ID of the TRP for which the LOS/NLOS Information is provided, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the TRP for which the LOS/NLOS Information is provided, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-los-nlos-indicator This field provides the expected likelihood of a LOS propagation path from a TRP to the target device (choice per-trp) or for all DL-PRS Resources of the TRP (choice per-resource).

#### –	NR-DL-PRS-Info

The IE NR-DL-PRS-Info defines downlink PRS configuration.

-- ASN1START

NR-DL-PRS-Info-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetList-r16		SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

NR-DL-PRS-ResourceSet-r16,

...

}

NR-DL-PRS-ResourceSet-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16,

dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16,

dl-PRS-ResourceRepetitionFactor-r16	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,	-- Need OP

dl-PRS-ResourceTimeGap-r16			ENUMERATED {s1, s2, s4, s8, s16, s32, ...}

OPTIONAL, 	-- Cond Rep

dl-PRS-NumSymbols-r16				ENUMERATED {n2, n4, n6, n12, ...},

dl-PRS-MutingOption1-r16			DL-PRS-MutingOption1-r16			OPTIONAL,	-- Need OP

dl-PRS-MutingOption2-r16			DL-PRS-MutingOption2-r16			OPTIONAL,	-- Need OP

dl-PRS-ResourcePower-r16			INTEGER (-60..50),

dl-PRS-ResourceList-r16				SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

NR-DL-PRS-Resource-r16,

...

}

DL-PRS-MutingOption1-r16 ::= SEQUENCE {

dl-prs-MutingBitRepetitionFactor-r16

ENUMERATED { n1, n2, n4, n8, ... }	OPTIONAL,	-- Need OP

nr-option1-muting-r16				NR-MutingPattern-r16,

...

}

DL-PRS-MutingOption2-r16 ::= SEQUENCE {

nr-option2-muting-r16				NR-MutingPattern-r16,

...

}

NR-MutingPattern-r16 ::= CHOICE {

po2-r16								BIT STRING (SIZE(2)),

po4-r16								BIT STRING (SIZE(4)),

po6-r16								BIT STRING (SIZE(6)),

po8-r16								BIT STRING (SIZE(8)),

po16-r16							BIT STRING (SIZE(16)),

po32-r16							BIT STRING (SIZE(32)),

...

}

NR-DL-PRS-Resource-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16,

dl-PRS-SequenceID-r16				INTEGER (0.. 4095),

dl-PRS-CombSizeN-AndReOffset-r16	CHOICE {

n2-r16							INTEGER (0..1),

n4-r16							INTEGER (0..3),

n6-r16							INTEGER (0..5),

n12-r16							INTEGER (0..11),

...

},

dl-PRS-ResourceSlotOffset-r16		INTEGER (0..nrMaxResourceOffsetValue-1-r16),

dl-PRS-ResourceSymbolOffset-r16		INTEGER (0..12),

dl-PRS-QCL-Info-r16					DL-PRS-QCL-Info-r16		OPTIONAL,	--Need ON

...,

[[

dl-PRS-ResourcePrioritySubset-r17	DL-PRS-ResourcePrioritySubset-r17	OPTIONAL	-- Need ON

]]

}

DL-PRS-QCL-Info-r16 ::= CHOICE {

ssb-r16						SEQUENCE {

pci-r16							NR-PhysCellID-r16,

ssb-Index-r16					INTEGER (0..63),

rs-Type-r16						ENUMERATED {typeC, typeD, typeC-plus-typeD}

},

dl-PRS-r16					SEQUENCE {

qcl-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16,

qcl-DL-PRS-ResourceSetID-r16	NR-DL-PRS-ResourceSetID-r16

}

}

NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset-r16 ::= CHOICE {

scs15-r16		CHOICE {

n4-r16					INTEGER (0..3),

n5-r16					INTEGER (0..4),

n8-r16					INTEGER (0..7),

n10-r16					INTEGER (0..9),

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n160-r16				INTEGER (0..159),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

...

},

scs30-r16		CHOICE {

n8-r16					INTEGER (0..7),

n10-r16					INTEGER (0..9),

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

...

},

scs60-r16		CHOICE {

n16-r16					INTEGER (0..15),

n20-r16					INTEGER (0..19),

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n256-r16				INTEGER (0..255),

n320-r16				INTEGER (0..319),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

n40960-r16				INTEGER (0..40959),

...

},

scs120-r16		CHOICE {

n32-r16					INTEGER (0..31),

n40-r16					INTEGER (0..39),

n64-r16					INTEGER (0..63),

n80-r16					INTEGER (0..79),

n128-r16				INTEGER (0..127),

n160-r16				INTEGER (0..159),

n256-r16				INTEGER (0..255),

n320-r16				INTEGER (0..319),

n512-r16				INTEGER (0..511),

n640-r16				INTEGER (0..639),

n1280-r16				INTEGER (0..1279),

n2560-r16				INTEGER (0..2559),

n5120-r16				INTEGER (0..5119),

n10240-r16				INTEGER (0..10239),

n20480-r16				INTEGER (0..20479),

n40960-r16				INTEGER (0..40959),

n81920-r16				INTEGER (0..81919),

...

},

...

}

DL-PRS-ResourcePrioritySubset-r17 ::= SEQUENCE (SIZE (1..maxNumPrioResources-r17)) OF

NR-DL-PRSResourcePriorityItem-r17

NR-DL-PRSResourcePriorityItem-r17 ::= SEQUENCE {

nr-DL-PRS-PrioResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16		OPTIONAL,

-- Cond NotSameAsPrev

nr-DL-PRS-PrioResourceID-r17			NR-DL-PRS-ResourceID-r16,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Rep | The field is mandatory present, if dl-PRS-ResourceRepetitionFactor is present. Otherwise it is not present. |
| NotSameAsPrev | The field is mandatory present for the first element in DL-PRS-ResourcePrioritySubset and optionally present, need OP, for each additional element in DL-PRS-ResourcePrioritySubset. If this field is absent, the nr-DL-PRS-PrioResourceSetID is the same as the previous nr-DL-PRS-PrioResourceSetID in DL-PRS-ResourcePrioritySubset. |

NR-DL-PRS-Info field descriptions

nr-DL-PRS-ResourceSetID This field specifies the DL-PRS Resource Set ID, which is used to identify the DL-PRS Resource Set of the TRP across all the frequency layers.

dl-PRS-Periodicity-and-ResourceSetSlotOffset This field specifies the periodicity of DL-PRS allocation in slots configured per DL-PRS Resource Set and the slot offset with respect to SFN #0 slot #0 for a TRP where the DL-PRS Resource Set is configured (i.e. slot where the first DL-PRS Resource of DL-PRS Resource Set occurs).

dl-PRS-ResourceRepetitionFactor This field specifies how many times each DL-PRS Resource is repeated for a single instance of the DL-PRS Resource Set. It is applied to all resources of the DL-PRS Resource Set. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively. If this field is absent, the value for dl-PRS-ResourceRepetitionFactor is 1 (i.e., no resource repetition).

dl-PRS-ResourceTimeGap This field specifies the offset in units of slots between two repeated instances of a DL-PRS Resource corresponding to the same DL-PRS Resource ID within a single instance of the DL-PRS Resource Set. The time duration spanned by one DL-PRS Resource Set containing repeated DL-PRS Resources should not exceed DL-PRS-Periodicity.

dl-PRS-NumSymbols This field specifies the number of symbols per DL-PRS Resource within a slot.

dl-PRS-MutingOption1 This field specifies the DL-PRS muting configuration of the TRP for the Option-1 muting, as specified in TS 38.214 [45], and comprises the following sub-fields: - dl-prs-MutingBitRepetitionFactor indicates the number of consecutive instances of the DL-PRS Resource Set corresponding to a single bit of the nr-option1-muting bit map. Enumerated values n1, n2, n4, n8 correspond to 1, 2, 4, 8 consecutive instances, respectively. If this sub-field is absent, the value for dl-prs-MutingBitRepetitionFactor is n1. - nr-option1-muting defines a bitmap of the time locations where the DL-PRS Resource is transmitted (value '1') or not (value '0') for a DL-PRS Resource Set, as specified in TS 38.214 [45]. If this field is absent, Option-1 muting is not in use for the TRP.

dl-PRS-MutingOption2 This field specifies the DL-PRS muting configuration of the TRP for the Option-2 muting, as specified in TS 38.214 [45], and comprises the following sub-fields: - nr-option2-muting defines a bitmap of the time locations where the DL-PRS Resource is transmitted (value '1') or not (value '0'). Each bit of the bitmap corresponds to a single repetition of the DL-PRS Resource within an instance of a DL-PRS Resource Set, as specified in TS 38.214 [45]. The size of this bitmap should be the same as the value for dl-PRS-ResourceRepetitionFactor. If this field is absent, Option-2 muting is not in use for the TRP.

dl-PRS-ResourcePower This field specifies the average EPRE of the resources elements that carry the PRS in dBm that is used for PRS transmission. The UE assumes constant EPRE is used for all REs of a given DL-PRS resource.

dl-PRS-SequenceID This field specifies the sequence Id used to initialize cinit value used in pseudo random generator TS 38.211 [41], clause 5.2.1 for generation of DL-PRS sequence for transmission on a given DL-PRS Resource.

dl-PRS-CombSizeN-AndReOffset This field specifies the Resource Element spacing in each symbol of the DL-PRS Resource and the Resource Element (RE) offset in the frequency domain for the first symbol in a DL-PRS Resource. All DL-PRS Resource Sets belonging to the same Positioning Frequency Layer have the same value of comb size. The relative RE offsets of following symbols are defined relative to the RE Offset in the frequency domain of the first symbol in the DL-PRS Resource according to TS 38.211 [41]. The comb size configuration should be aligned with the comb size configuration for the frequency layer.

dl-PRS-ResourceSlotOffset This field specifies the starting slot of the DL-PRS Resource with respect to the corresponding DL-PRS-Resource Set Slot Offset.

dl-PRS-ResourceSymbolOffset This field specifies the starting symbol of the DL-PRS Resource within a slot determined by dl-PRS-ResourceSlotOffset.

dl-PRS-QCL-Info This field specifies the QCL indication with other DL reference signals for serving and neighbouring cells and comprises the following subfields: - ssb indicates the SSB information for QCL source and comprises the following sub-fields: - pci specifies the physical cell ID of the cell with the SSB that is configured as the source reference signal for the DL-PRS. The UE obtains the SSB configuration for the SSB configured as source reference signal for the DL-PRS by indexing to the field nr-SSB-Config with this physical cell identity. - ssb-Index indicates the index for the SSB configured as the source reference signal for the DL-PRS. - rs-Type indicates the QCL type. - dl-PRS indicates the PRS information for QCL source reference signal and comprises the followings sub-fields: - qcl-DL-PRS-ResourceID specifies DL-PRS Resource ID of the DL-PRS resource used as the source reference signal. - qcl-DL-PRS-ResourceSetID indicates the DL-PRS Resource Set ID of the DL-PRS Resource Set used as the source reference signal.

dl-PRS-ResourcePrioritySubset This field provides a subset of DL-PRS Resources, which is associated with nr-DL-PRS-ResourceID for the purpose of prioritization of DL-AoD reporting. For each nr-DL-PRS-ResourceID the dl-PRS-ResourcePrioritySubset indicates the associated DL-PRS Resources the target device should prioritize for DL-PRS RSRP and DL-PRS First Path RSRP measurement reporting in IE NR-DL-AoD-SignalMeasurementInformation.

#### –	NR-DL-PRS-ProcessingCapability

The IE NR-DL-PRS-ProcessingCapability defines the common DL-PRS Processing capability. In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

The PRS-ProcessingCapabilityPerBand is defined for a single positioning frequency layer on a certain band (i.e., a target device supporting multiple positioning frequency layers is expected to process one frequency layer at a time).

-- ASN1START

NR-DL-PRS-ProcessingCapability-r16 ::= SEQUENCE {

prs-ProcessingCapabilityBandList-r16	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r16,

maxSupportedFreqLayers-r16				INTEGER (1..4),

simulLTE-NR-PRS-r16						ENUMERATED { supported}	OPTIONAL,

...,

[[

prs-ProcessingCapabilityBandList-r17	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

PRS-ProcessingCapabilityPerBand-r17

]]

}

PRS-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

supportedBandwidthPRS-r16			CHOICE {

fr1										ENUMERATED {mhz5, mhz10, mhz20, mhz40,

mhz50, mhz80, mhz100},

fr2										ENUMERATED {mhz50, mhz100, mhz200, mhz400},

...

},

dl-PRS-BufferType-r16		 		ENUMERATED {type1, type2, ...},

durationOfPRS-Processing-r16		SEQUENCE {

durationOfPRS-ProcessingSymbols-r16	ENUMERATED {nDot125, nDot25, nDot5, n1,

n2, n4, n6, n8, n12, n16, n20, n25,

n30, n32, n35, n40, n45, n50},

durationOfPRS-ProcessingSymbolsInEveryTms-r16

ENUMERATED {n8, n16, n20, n30, n40, n80,

n160,n320, n640, n1280},

...

},

maxNumOfDL-PRS-ResProcessedPerSlot-r16	SEQUENCE {

scs15-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs30-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs60-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

scs120-r16								ENUMERATED {n1, n2, n4, n8, n16, n24, n32,

n48, n64}					OPTIONAL,

...

},

...

}

PRS-ProcessingCapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17					FreqBandIndicatorNR-r16,

supportedDL-PRS-ProcessingSamples-r17	ENUMERATED { m1 }						OPTIONAL,

prs-ProcessingWindowType1A-r17			ENUMERATED { supported }				OPTIONAL,

prs-ProcessingWindowType1B-r17			ENUMERATED { supported }				OPTIONAL,

prs-ProcessingWindowType2-r17			ENUMERATED { supported }				OPTIONAL,

supportedPrioHandlingOptOfPPW-r17		ENUMERATED { option1,option2,option3 }	OPTIONAL,

prs-BufferingCapability-r17				ENUMERATED { type1, type2 }				OPTIONAL,

maxDL-PRS-ResourcesProcessInSlot-r17	ENUMERATED { n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }	OPTIONAL,

prs-InactiveBufferingCapability-r17		ENUMERATED { type1, type2 }				OPTIONAL,

maxDL-PRS-ResourcesProcessInSlotRRC-Inactive-r17

ENUMERATED { n1, n2, n4, n6, n8, n12,

n16, n24, n32, n48, n64 }	OPTIONAL,

lowerRxBeamSweepingThan8-FR2-r17		ENUMERATED { supported }				OPTIONAL,

numberOfRxBeamSweepingFactor-r17		ENUMERATED { FFS }						OPTIONAL,

...

}

-- ASN1STOP

NR-DL-PRS-ProcessingCapability field descriptions

maxSupportedFreqLayers Indicates the maximum number of positioning frequency layers supported by UE.

supportedBandwidthPRS Indicates the maximum number of DL-PRS bandwidth in MHz, which is supported and reported by UE.

dl-PRS-BufferType Indicates DL-PRS buffering capability. Value type1 indicates sub-slot/symbol level buffering and value type2 indicates slot level buffering.

durationOfPRS-Processing Indicates the duration N of DL-PRS symbols in units of ms a UE can process every T ms assuming maximum DL-PRS bandwidth provided in supportedBandwidthPRS and comprises the following subfields: - durationOfPRS-ProcessingSymbols: This field specifies the values for N. Enumerated values indicate 0.125, 0.25, 0.5, 1, 2, 4, 8, 12, 16, 20, 25, 30, 35, 40, 45, 50 ms. - durationOfPRS-ProcessingSymbolsInEveryTms: This field specifies the values for T. Enumerated values indicate 8, 16, 20, 30, 40, 80, 160, 320, 640, 1280 ms. See NOTE.

maxNumOfDL-PRS-ResProcessedPerSlot Indicates the maximum number of DL-PRS resources that UE can process in a slot. SCS: 15 kHz, 30 kHz, 60 kHz are applicable for FR1 bands. SCS: 60 kHz, 120 kHz are applicable for FR2 bands.

simulLTE-NR-PRS Indicates whether the UE supports parallel processing of LTE PRS and NR PRS.

supportedDL-PRS-ProcessingSamples Indicates the capability to support reporting a measurement based on measuring M=1 samples (instances) of a DL-PRS Resource Set.

prs-ProcessingWindowType1A Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1A refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS Processing Window. The DL signals/channels from all DL CCs (per UE) are affected across LTE and NR.

prs-ProcessingWindowType1B Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 1B refers to the determination of prioritization between DL-PRS and other DL signals/channels in all OFDM symbols within the PRS processing window. The DL signals/channels from a certain band are affected.

prs-ProcessingWindowType2 Indicates the supported DL-PRS processing types subject to the UE determining that DL-PRS to be higher priority for DL-PRS measurement outside MG and in a DL-PRS Processing Window. Type 2 refers to the determination of prioritization between DL-PRS and other DL signals/channels only in DL-PRS symbols within the PRS processing window.

supportedPrioHandlingOptOfPPW Indicates support of priority handing options of DL-PRS when the DL-PRS measurement is outside measurement gap. - option1: UE indicates support of two priority states. - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: DL-PRS is lower priority than all PDCCH/PDSCH/CSI-RS - option2: UE indicates support of three priority states - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS - State 2: DL-PRS is lower priority than PDCCH and URLLC PDSCH and higher priority than other PDSCH/CSI-RS Note: The URLLC channel corresponds a dynamically scheduled PDSCH whose PUCCH resource for carrying ACK/NAK is marked as high-priority. - State 3: DL-PRS is lower priority than all PDCCH/PDSCH/CSI-RS - option3: UE indicates support of single priority state - State 1: DL-PRS is higher priority than all PDCCH/PDSCH/CSI-RS

prs-BufferingCapability Indicates the DL-PRS Processing Capability outside MG - buffering capability. - type1: sub-slot/symbol level buffering - type2: slot level buffering

maxDL-PRS-ResourcesProcessInSlot Indicates the DL-PRS Processing Capability outside MG - buffering capability. Max number of DL-PRS resources that UE can process in a slot under it.

prs-InactiveBufferingCapability Indicates the DL-PRS processing capabilities in RRC_INACTIVE state. DL PRS buffering capability: - type1: sub-slot/symbol level buffering - type2: slot level buffering

maxDL-PRS-ResourcesProcessInSlotRRC-Inactive Indicates the DL-PRS processing capabilities in RRC_INACTIVE state. Max number of DL-PRS Resources that UE can process in a slot under it.

lowerRxBeamSweepingThan8-FR2 Indicates support of the lower Rx beam sweeping factor than 8 for FR2.

numberOfRxBeamSweepingFactor Indicates the number of Rx beam sweeping factors for FR2.

NOTE:	When the target device provides the durationOfPRS-Processing capability (N, T) for any P(≥T) time window defined in TS 38. 214 [45] clause 5.1.6.5, the target device should be capable of processing all DL-PRS resources within P, if

-	N≥K where K is defined in the TS 38.214 [45] clause 5.1.6.5, and

-	the number of DL-PRS Resources in each slot does not exceed the maxNumOfDL-PRS-ResProcessedPerSlot, and

-	the configured measurement gap and a maximum ratio of measurement gap length (MGL) / measurement gap repetition period (MGRP) is as specified in TS 38.133 [46].

#### –	NR-DL-PRS-QCL-ProcessingCapability

The IE NR-DL-PRS-QCL-ProcessingCapability defines the common UE DL-PRS QCL Processing capability. The UE can include this IE only if the UE supports NR-DL-PRS-ProcessingCapability. Otherwise, the UE does not include this IE.

In the case of capabilities for multiple NR positioning methods are provided, the IE NR-DL-PRS-QCL-ProcessingCapability applies across the NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities, NR-DL-AoD-ProvideCapabilities, and NR-Multi-RTT-ProvideCapabilities.

-- ASN1START

NR-DL-PRS-QCL-ProcessingCapability-r16 ::= SEQUENCE {

dl-PRS-QCL-ProcessingCapabilityBandList-r16			SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-PRS-QCL-ProcessingCapabilityPerBand-r16,

...

}

DL-PRS-QCL-ProcessingCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16						FreqBandIndicatorNR-r16,

ssb-FromNeighCellAsQCL-r16					ENUMERATED { supported}	OPTIONAL,

prs-FromServNeighCellAsQCL-r16				ENUMERATED { supported} OPTIONAL,

...

}

-- ASN1STOP

NR-DL-PRS-QCL-ProcessingCapability field descriptions

ssb-FromNeighCellAsQCL Indicates the support of SSB from neighbour cell as QCL source of a DL-PRS. UE supporting this feature also support reusing SSB measurement from RRM for receiving PRS. Note: It refers to Type-C for FR1 and Type-C & Type-D support for FR2.

prs-FromServNeighCellAsQCL Indicates the support of DL-PRS from serving/neighbour cell as QCL source of a DL-PRS. Note 1: It refers to Type-D support for FR2. Note 2: A PRS from a PRS-only TP is treated as PRS from a non-serving cell.

#### –	NR-DL-PRS-ResourceID

The IE NR-DL-PRS-ResourceID defines the identity of a DL-PRS Resource of a DL-PRS Resource Set of a TRP.

-- ASN1START

NR-DL-PRS-ResourceID-r16 ::= INTEGER (0..nrMaxNumDL-PRS-ResourcesPerSet-1-r16)

-- ASN1STOP

#### –	NR-DL-PRS-ResourcesCapability

The IE NR-DL-PRS-ResourcesCapability defines the DL-PRS resources capability for each positioning method. The UE can include this IE only if the UE supports NR-DL-PRS-ProcessingCapability. Otherwise, the UE does not include this IE.

-- ASN1START

NR-DL-PRS-ResourcesCapability-r16 ::= SEQUENCE {

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer-r16

INTEGER (1..2),

maxNrOfTRP-AcrossFreqs-r16					ENUMERATED { n4, n6, n12, n16, n32,

n64, n128, n256, ...},

maxNrOfPosLayer-r16							INTEGER (1..4),

dl-PRS-ResourcesCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-PRS-ResourcesCapabilityPerBand-r16,

dl-PRS-ResourcesBandCombinationList-r16		DL-PRS-ResourcesBandCombinationList-r16,

...

}

DL-PRS-ResourcesCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16						FreqBandIndicatorNR-r16,

maxNrOfDL-PRS-ResourcesPerResourceSet-r16	ENUMERATED { n1, n2, n4, n8, n16, n32, n64, ...},

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer-r16

ENUMERATED { n6, n24, n32, n64, n96, n128,

n256, n512, n1024, ...},

...

}

DL-PRS-ResourcesBandCombinationList-r16 ::=	SEQUENCE (SIZE (1..maxBandComb-r16)) OF

DL-PRS-ResourcesBandCombination-r16

DL-PRS-ResourcesBandCombination-r16 ::=	SEQUENCE {

bandList-r16							SEQUENCE (SIZE (1..maxSimultaneousBands-r16)) OF

FreqBandIndicatorNR-r16,

maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet-r16

CHOICE {

fr1-Only-r16							ENUMERATED {n6, n24, n64, n128, n192,

n256, n512, n1024, n2048},

fr2-Only-r16							ENUMERATED {n24, n64, n96, n128, n192,

n256, n512, n1024, n2048},

fr1-FR2Mix-r16							SEQUENCE {

fr1-r16									ENUMERATED {n6, n24, n64, n96, n128,

n192, n256, n512, n1024, n2048},

fr2-r16									ENUMERATED {n24, n64, n96, n128, n192,

n256, n512, n1024, n2048},

...

},

...

},

...

}

-- ASN1STOP

NR-DL-PRS-ResourcesCapability field descriptions

maxNrOfDL-PRS-ResourceSetPerTrpPerFrequencyLayer Indicates the maximum number of DL-PRS Resource Sets per TRP per frequency layer supported by UE.

maxNrOfTRP-AcrossFreqs Indicates the maximum number of TRPs across all positioning frequency layers.

maxNrOfPosLayer Indicates the maximum number of supported positioning layer.

dl-PRS-ResourcesBandCombinationList Provides the capabilities of DL-PRS Resources for the indicated band combination in bandList. This field is provided for all band combinations for which the target device supports DL-PRS.

maxNrOfDL-PRS-ResourcesPerResourceSet Indicates the maximum number of DL-PRS Resources per DL-PRS Resource Set. Value 16, 32, 64 are only applicable to FR2 bands. Value 1 is not applicable for DL-AoD.

maxNrOfDL-PRS-ResourcesPerPositioningFrequencylayer Indicates the maximum number of DL-PRS resources per positioning frequency layer. Value 6 is only applicable to FR1 bands.

maxNrOfDL-PRS-ResourcesAcrossAllFL-TRP-ResourceSet Indicates the maximum number of DL-PRS Resources supported by UE across all frequency layers, TRPs and DL-PRS Resource Sets. fr1-Only: This is applicable for FR1 only band combinations; fr2-Only: This is applicable for FR2 only band combinations; fr1-FR2Mix: This is applicable for band combinations containing FR1 and FR2 bands. fr1 means for FR1 in FR1/FR2 mixed operation, and fr2 means for FR2 in FR1/FR2 mixed operation.

#### –	NR-DL-PRS-ResourceSetID

The IE NR-DL-PRS-ResourceSetID defines the identity of a DL-PRS Resource Set of a TRP.

-- ASN1START

NR-DL-PRS-ResourceSetID-r16 ::= INTEGER (0..nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16)

-- ASN1STOP

#### –	NR-DL-PRS-TRP-TEG-Info

The IE NR-DL-PRS-TRP-TEG-Info is used by the location server to provide the association information of DL-PRS Resources with TRP Tx TEGs.

-- ASN1START

NR-DL-PRS-TRP-TEG-Info-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-DL-PRS-TRP-TEG-InfoPerFreqLayer-r17

NR-DL-PRS-TRP-TEG-InfoPerFreqLayer-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-DL-PRS-TRP-TEG-InfoPerTRP-r17

NR-DL-PRS-TRP-TEG-InfoPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r16						INTEGER (0..255),

nr-PhysCellID-r16					NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r16					NCGI-r15			OPTIONAL,	-- Need ON

nr-ARFCN-r16						ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

dl-PRS-TEG-InfoSet-r17				SEQUENCE (SIZE(1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-TEG-InfoPerResource-r17,

...

}

DL-PRS-TEG-InfoPerResource-r17 ::= SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-TEG-InfoElement-r17

DL-PRS-TEG-InfoElement-r17 ::= SEQUENCE {

dl-prs-trp-Tx-TEG-ID-r17		INTEGER (0..maxNumOfTRP-TxTEGs-1-r17),

...

}

-- ASN1STOP

NR-DL-PRS-TRP-TEG-Info field descriptions

dl-PRS-ID This field specifies the DL-PRS ID of the TRP for which the TRP Tx TEG information is provided.

nr-PhysCellID This field specifies the physical Cell-ID of the TRP for which the TRP Tx TEG information is provided, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the TRP for which the TRP Tx TEG information is provided, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

dl-PRS-TEG-InfoSet This field specifies the TRP Tx TEG ID associated with the transmissions of each DL-PRS Resource of the TRP. The dl-prs-trp-Tx-TEG-ID in dl-PRS-TEG-InfoSet is associated with the nr-DL-PRS-ResourceID of NR-DL-PRS-Info using the same structure and order.

#### –	NR-On-Demand-DL-PRS-Configurations

The IE NR-On-Demand-DL-PRS-Configurations provides a set of possible DL-PRS configurations which can be requested by the target device on-demand.

-- ASN1START

NR-On-Demand-DL-PRS-Configurations-r17 ::= SEQUENCE {

on-demand-dl-prs-configuration-list-r17		SEQUENCE (SIZE (1..maxDL-PRS-Configs-r17)) OF

On-Demand-DL-PRS-Configuration-r17,

...

}

On-Demand-DL-PRS-Configuration-r17 ::= SEQUENCE {

dl-prs-configuration-id-r17					DL-PRS-Configuration-ID-r17,

nr-DL-PRS-PositioningFrequencyLayer-r17		NR-DL-PRS-PositioningFrequencyLayer-r16,

nr-DL-PRS-Info-r17							NR-DL-PRS-Info-r16,

...

}

DL-PRS-Configuration-ID-r17 ::= SEQUENCE {

nr-dl-prs-configuration-id-r17				INTEGER (1..maxDL-PRS-Configs-r17),

...

}

-- ASN1STOP

NR-DL-PRS-On-Demand-Configurations field descriptions

dl-prs-configuration-id This field provides an identity for the On-Demand-DL-PRS-Configuration.

#### –	NR-On-Demand-DL-PRS-Information

The IE NR-On-Demand-DL-PRS-Information defines the requested on-demand DL-PRS.

-- ASN1START

NR-On-Demand-DL-PRS-Information-r17 ::= SEQUENCE {

dl-prs-FrequencyRangeReq-r17			ENUMERATED { fr1, fr2, ...}				OPTIONAL,

nr-on-demand-dl-prs-info-per-FrequLayer-r17

NR-On-Demand-DL-PRS-Info-per-FrequLayer-r17

OPTIONAL,

...

}

NR-On-Demand-DL-PRS-Info-per-FrequLayer-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-On-Demand-DL-PRS-FrequLayerElement-r17

NR-On-Demand-DL-PRS-FrequLayerElement-r17 ::= SEQUENCE {

dl-prs-ResourceSetPeriodicityReq-r17	ENUMERATED { p4, p5, p8, p10, p16, p20, p32, p40,

p64, p80, p160, p320, p640, p1280, p2560,

p5120, p10240, p20480, p40960, p81920, ...}

OPTIONAL,

dl-prs-ResourceBandwidthReq-r17			INTEGER (1..63)							OPTIONAL,

dl-prs-ResourceRepetitionFactorReq-r17	ENUMERATED {n2, n4, n6, n8, n16, n32, ...}

OPTIONAL,

dl-prs-NumSymbolsReq-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-CombSizeN-Req-r17				ENUMERATED {n2, n4, n6, n12, ...}		OPTIONAL,

dl-prs-QCL-InformationReq-r17			DL-PRS-QCL-InformationReq-r17			OPTIONAL,

...

}

DL-PRS-QCL-InformationReq-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

DL-PRS-QCL-InformationReqPerTRP-r17

DL-PRS-QCL-InformationReqPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16							OPTIONAL,

nr-CellGlobalID-r17					NCGI-r15									OPTIONAL,

nr-ARFCN-r17						ARFCN-ValueNR-r15							OPTIONAL,

dl-prs-QCL-InformationReqSet-r17	SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-QCL-InfoReq-r17,

...

}

DL-PRS-QCL-InfoReq-r17 ::= SEQUENCE {

nr-DL-PRS-ResourceSetID-r17			NR-DL-PRS-ResourceSetID-r16,

dl-prs-QCL-InformationReq-r17		CHOICE {

nr-DL-PRS-QCL-Source-r17			DL-PRS-QCL-Info-r16,

dl-prs-QCL-Info-requested-r17		ENUMERATED { true }

},

...

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Information field descriptions

dl-prs-FrequencyRangeReq This field specifies the frequency range for which the on-demand DL-PRS is requested.

nr-on-demand-dl-prs-info-per-FrequLayer This field specifies the desired number of Positioning Frequency Layers for which the on-demand DL-PRS is requested and the on-demand DL-PRS parameter per Positioning Frequency Layer.

dl-prs-ResourceSetPeriodicityReq This field specifies the requested periodicity of the DL-PRS Resource Set in slots. The periodicity depends on the subcarrier spacing (SCS) and takes values 2μ4, 5, 8, 10, 16, 20, 32, 40, 64, 80, 160, 320, 640, 1280, 2560, 5120, 10240 slots, where μ=0, 1, 2, 3 for SCS of 15, 30, 60 and 120 kHz respectively. μ refers to the target devices current primary cell.

dl-prs-ResourceBandwidthReq This field specifies the requested number of PRBs allocated for the DL-PRS Resource (allocated DL-PRS bandwidth) in multiples of 4 PRBs. Integer value 1 corresponds to 24 PRBs, value 2 corresponds to 28 PRBs, value 3 corresponds to 32 PRBs and so on.

dl-prs-ResourceRepetitionFactorReq This field specifies the requested DL-PRS Resource repetition. Enumerated values n2, n4, n6, n8, n16, n32 correspond to 2, 4, 6, 8, 16, 32 resource repetitions, respectively.

dl-prs-NumSymbolsReq This field specifies the requested number of symbols per DL-PRS Resource within a slot.

dl-prs-CombSizeN-Req This field specifies the requested Resource Element spacing in each symbol of the DL-PRS Resource.

dl-prs-QCL-InformationReq This field specifies the recommended or requested QCL indication with other DL reference signals. - dl-PRS-ID indicates the DL-PRS ID of the TRP for which the QCL information is recommended. - nr-PhysCellID indicates the physical Cell-ID of the TRP for which the QCL information is recommended, as defined in TS 38.331 [35]. - nr-CellGlobalID indicates the NCGI, the globally unique identity of a cell in NR, of the TRP for which the QCL information is recommended, as defined in TS 38.331 [35]. - nr-ARFCN indicates the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - dl-prs-QCL-InformationReqSet indicates the recommended QCL information per DL-PRS Resource Set. - nr-DL-PRS-ResourceSetID indicates the DL-PRS Resource Set ID for which the nr-DL-PRS-QCL-Source is recommended - nr-DL-PRS-QCL-Source indicates the QCL source. - dl-prs-QCL-Info-requested indicates that the UE requests to provide the QCL information in the assistance data.

#### –	NR-On-Demand-DL-PRS-Request

The IE NR-On-Demand-DL-PRS-Request is used by the target device to request on-demand DL-PRS from a location server.

-- ASN1START

NR-On-Demand-DL-PRS-Request-r17 ::= SEQUENCE {

dl-prs-StartTime-and-Duration-r17			DL-PRS-StartTime-and-Duration-r17		OPTIONAL,

nr-on-demand-DL-PRS-Information-r17 		NR-On-Demand-DL-PRS-Information-r17		OPTIONAL,

dl-prs-configuration-id-PrefList-r17		SEQUENCE (SIZE (1..maxDL-PRS-Configs-r17)) OF 
													DL-PRS-Configuration-ID-r17			OPTIONAL,

...

}

DL-PRS-StartTime-and-Duration-r17 ::= SEQUENCE {

dl-prs-start-time-r17		INTEGER (1..1024)						OPTIONAL,

dl-prs-duration-r17			SEQUENCE {

seconds-r17		INTEGER (0..59)		OPTIONAL,

minutes-r17		INTEGER (0..59)		OPTIONAL,

hours-r17		INTEGER (0..23)		OPTIONAL,

...

}										OPTIONAL,

...

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Request field descriptions

dl-prs-StartTime-and-Duration This field specifies the requested start time and duration for the on-demand DL-PRS and comprises the following subfields: - dl-prs-start-time specifies the desired start time for the requested DL-PRS. It indicates the time in seconds from the time the IE NR-On-Demand-DL-PRS-Request was received. - dl-prs-duration specifies the desired duration of the requested DL-PRS. The desired duration is the sum of the seconds, minutes, hours fields. If this field is included, at least one of the seconds, minutes, hours fields shall be present.

nr-on-demand-DL-PRS-Information This field specifies the on-demand DL-PRS configuration information requested by the target device.

dl-prs-configuration-id-PrefList This field specifies the on-demand DL-PRS configuration associated with DL-PRS-Configuration-ID in IE NR-On-Demand-DL-PRS-Configurations the target device wishes to obtain in the order of preference. The first DL-PRS-Configuration-ID in the list is the most preferred configuration, the second DL-PRS-Configuration-ID the second most preferred, etc.

#### –	NR-On-Demand-DL-PRS-Support

The IE NR-On-Demand-DL-PRS-Support defines the target device's on-demand DL-PRS capabilities.

-- ASN1START

NR-On-Demand-DL-PRS-Support-r17 ::= SEQUENCE {

nr-on-demand-DL-PRS-InformationSup-r17			ENUMERATED { supported }			OPTIONAL,

nr-on-demand-DL-PRS-ConfigurationsSup-r17		ENUMERATED { supported }			OPTIONAL,

...

}

-- ASN1STOP

NR-On-Demand-DL-PRS-Support field descriptions

nr-on-demand-DL-PRS-InformationSup This field, if present, indicates that the target device supports the IE NR-On-Demand-DL-PRS-Information in IE NR-On-Demand-DL-PRS-Request.

nr-on-demand-DL-PRS-ConfigurationsSup This field, if present, specifies that the target device supports the dl-prs-configuration-id-PrefList in IE NR-On-Demand-DL-PRS-Request.

#### –	NR-PositionCalculationAssistance

The IE NR-PositionCalculationAssistance is used by the location server to provide assistance data to enable UE-based downlink positioning.

-- ASN1START

NR-PositionCalculationAssistance-r16 ::= SEQUENCE {

nr-TRP-LocationInfo-r16 		NR-TRP-LocationInfo-r16				OPTIONAL,	-- Need ON

nr-DL-PRS-BeamInfo-r16			NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Need ON

nr-RTD-Info-r16					NR-RTD-Info-r16						OPTIONAL,	-- Need ON

...,

[[

nr-TRP-BeamAntennaInfo-r17		NR-TRP-BeamAntennaInfo-r17			OPTIONAL,	-- Need ON

nr-DL-PRS-Expected-LOS-NLOS-Assistance-r17

NR-DL-PRS-Expected-LOS-NLOS-Assistance-r17

OPTIONAL,	-- Need ON

nr-DL-PRS-TRP-TEG-Info-r17 		NR-DL-PRS-TRP-TEG-Info-r17			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

NR-PositionCalculationAssistance field descriptions

nr-TRP-LocationInfo This field provides the location coordinates of the antenna reference points of the TRPs.

nr-DL-PRS-BeamInfo This field provides the spatial directions of DL-PRS Resources for TRPs.

nr-RTD-Info This field provides the time synchronization information between the reference TRP and neighbour TRPs.

nr-TRP-BeamAntennaInfo This field provides the relative DL-PRS Resource power between PRS resources per angle per TRP.

nr-DL-PRS-Expected-LOS-NLOS-Assistance This field provides the expected likelihood of a LOS propagation path from a TRP to the target device. The information is provided per TRP or per DL-PRS Resource.

nr-DL-PRS-TRP-TEG-Info This field provides the TRP Tx TEG ID associated with the transmission of each DL-PRS Resource of the TRP.

#### –	NR-RTD-Info

The IE NR-RTD-Info is used by the location server to provide time synchronization information between a reference TRP and a list of neighbour TRPs.

-- ASN1START

NR-RTD-Info-r16 ::= SEQUENCE {

referenceTRP-RTD-Info-r16		ReferenceTRP-RTD-Info-r16,

rtd-InfoList-r16				RTD-InfoList-r16,

...

}

ReferenceTRP-RTD-Info-r16 ::= SEQUENCE {

dl-PRS-ID-Ref-r16				INTEGER (0..255),

nr-PhysCellID-Ref-r16			NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-Ref-r16			NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-Ref-r16				ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

refTime-r16						CHOICE {

systemFrameNumber-r16		BIT STRING (SIZE (10)),

utc-r16						UTCTime,

...

},

rtd-RefQuality-r16				NR-TimingQuality-r16	OPTIONAL,	-- Need ON

...

}

RTD-InfoList-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF RTD-InfoListPerFreqLayer-r16

RTD-InfoListPerFreqLayer-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPsPerFreq-r16)) OF RTD-InfoElement-r16

RTD-InfoElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16		OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15				OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15		OPTIONAL,	-- Need ON

subframeOffset-r16				INTEGER (0..1966079),

rtd-Quality-r16					NR-TimingQuality-r16,

...

}

-- ASN1STOP

NR-RTD-Info field descriptions

referenceTRP-RTD-Info This field defines the reference TRP for the RTD and comprises the following sub-fields: - dl-PRS-ID-Ref: This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource, and is associated to the reference TRP. - nr-PhysCellId-Ref: This field specifies the physical cell identity of the reference TRP. - nr-CellGlobalId-Ref: This field specifies the NCGI, the globally unique identity of a cell in NR, of the reference TRP. - nr-ARFCN-Ref: This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - refTime: This field specifies the reference time at which the rtd-InfoList is valid. The systemFrameNumber choice refers to the SFN of the reference TRP. - rtd-RefQuality: This field specifies the quality of the timing of reference TRP, used to determine the RTD values provided in rtd-InfoList.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP for which the RTD-InfoElement is applicable.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP for which the RTD-InfoElement is applicable, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP for which the RTD-InfoElement is applicable, as defined in TS 38.331 [35]. The server should include this field if it considers that it is needed to resolve ambiguity in the TRP indicated by nr-PhysCellID.

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID for which the RTD-InfoElement is applicable.

subframeOffset This field specifies the subframe boundary offset at the TRP antenna location between the reference TRP and this neighbour TRP in time units where Δfmax=480∙103 Hz and (TS 38.211 [41]). The offset is counted from the beginning of a subframe #0 of the reference TRP to the beginning of the closest subsequent subframe of this neighbour TRP. Scale factor 1 Tc.

rtd-Quality This field specifies the quality of the RTD.

#### –	NR-SelectedDL-PRS-IndexList

The IE NR-SelectedDL-PRS-IndexList is used by the location server to provide the selected DL-PRS Resource of nr-DL-PRS-AssistanceDataList to the target device.

In the case of assistance data for multiple NR positioning methods are provided, the IE NR-DL-PRS-AssistanceData shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData.

-- ASN1START

NR-SelectedDL-PRS-IndexList-r16 ::=	SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-SelectedDL-PRS-PerFreq-r16

NR-SelectedDL-PRS-PerFreq-r16 ::= SEQUENCE {

nr-SelectedDL-PRS-FrequencyLayerIndex-r16	INTEGER (0..nrMaxFreqLayers-1-r16),

nr-SelectedDL-PRS-IndexListPerFreq-r16 		SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-SelectedDL-PRS-IndexPerTRP-r16

OPTIONAL,	--Need OP

...

}

NR-SelectedDL-PRS-IndexPerTRP-r16 ::= SEQUENCE {

nr-SelectedTRP-Index-r16					INTEGER (0..nrMaxTRPsPerFreq-1-r16),

dl-SelectedPRS-ResourceSetIndexList-r16		SEQUENCE (SIZE (1..nrMaxSetsPerTrpPerFreqLayer-r16))

OF	DL-SelectedPRS-ResourceSetIndex-r16

OPTIONAL,	--Need OP

...

}

DL-SelectedPRS-ResourceSetIndex-r16 ::= SEQUENCE {

nr-DL-SelectedPRS-ResourceSetIndex-r16		INTEGER (0..nrMaxSetsPerTrpPerFreqLayer-1-r16),

dl-SelectedPRS-ResourceIndexList-r16		SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF

DL-SelectedPRS-ResourceIndex-r16

OPTIONAL	--Need OP

}

DL-SelectedPRS-ResourceIndex-r16 ::= SEQUENCE {

nr-DL-SelectedPRS-ResourceIdIndex-r16		INTEGER (0..nrMaxNumDL-PRS-ResourcesPerSet-1-r16),

...

}

-- ASN1STOP

NR-SelectedDL-PRS-IndexList field descriptions

nr-SelectedDL-PRS-FrequencyLayerIndex This field indicates the frequency layer provided in IE NR-DL-PRS-AssistanceData. Value 0 corresponds to the first frequency layer provided in nr-DL-PRS-AssistanceDataList in IE NR-DL-PRS-AssistanceData, value 1 to the second frequency layer in nr-DL-PRS-AssistanceDataList, and so on.

nr-SelectedDL-PRS-IndexListPerFreq This field provides the list of addressed TRPs of the selected frequency layer. If this field is absent, all DL-PRS Resources of all TRPs of the indicated frequency layer are addressed.

nr-SelectedTRP-Index This field indicates the addressed TRP of the selected frequency layer. Value 0 corresponds to the first entry in nr-DL-PRS-AssistanceDataPerFreq provided in IE NR-DL-PRS-AssistanceData, value 1 corresponds to the second entry in nr-DL-PRS-AssistanceDataPerFreq, and so on.

dl-SelectedPRS-ResourceSetIndexList This field provides the list of addressed DL-PRS Resource Sets of the selected TRPs of the selected frequency layer. If this field is absent, all DL-PRS Resource Sets and Resources of the indicated TRP are addressed.

nr-DL-SelectedPRS-ResourceSetIndex This field indicates the addressed DL-PRS Resource Set of the selected TRP of the selected frequency layer. Value 0 corresponds to the first entry in nr-DL-PRS-ResourceSetList in IE NR-DL-PRS-Info provided in IE NR-DL-PRS-AssistanceData. Value 1 corresponds to the second entry in the nr-DL-PRS-ResourceSetList in IE NR-DL-PRS-Info.

dl-SelectedPRS-ResourceIndexList This field provides the list of addressed DL-PRS Resources of the selected DL-PRS Resource Set of the selected TRP of the selected frequency layer. If this field is absent, all DL-PRS Resources of the indicated DL-PRS Resource Set are addressed.

nr-dl-SelectedPRS-ResourceIdIndex This field indicates the addressed DL-PRS Resource of the selected DL-PRS Resource Set of the TRP of the selected frequency layer. Value 0 corresponds to the first entry in dl-PRS-ResourceList in IE NR-DL-PRS-Info provided in IE NR-DL-PRS-AssistanceData. Value 1 corresponds to the second entry in the dl-PRS-ResourceList in IE NR-DL-PRS-Info, and so on.

#### –	NR-SSB-Config

The IE NR-SSB-Config defines SSB configuration.

-- ASN1START

NR-SSB-Config-r16 ::= SEQUENCE {

nr-PhysCellID-r16					NR-PhysCellID-r16,

nr-ARFCN-r16						ARFCN-ValueNR-r15,

ss-PBCH-BlockPower-r16				INTEGER (-60..50),

halfFrameIndex-r16					INTEGER (0..1),

ssb-periodicity-r16					ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, ...},

ssb-PositionsInBurst-r16			CHOICE {

shortBitmap-r16						BIT STRING (SIZE (4)),

mediumBitmap-r16					BIT STRING (SIZE (8)),

longBitmap-r16						BIT STRING (SIZE (64))

}																			OPTIONAL, --Need OR

ssb-SubcarrierSpacing-r16			ENUMERATED {kHz15, kHz30, kHz60, kHz120, kHz240, ...},

sfn-SSB-Offset-r16					INTEGER (0..15),

...

}

-- ASN1STOP

NR-SSB-Config field descriptions

nr-ARFCN This field specifies the ARFCN of the first RE of SSB's RB#10.

ss-PBCH-BlockPower Average EPRE of the resources elements that carry secondary synchronization signals in dBm that the NW used for SSB transmission, see TS 38.213 [48], clause 7.

halfFrameIndex Indicates the 5 msec offset of the SSB within a 10 msec system frame.

ssb-periodicity The SSB periodicity in ms for the rate matching purpose.

ssb-PositionsInBurst Indicates the time domain positions of the transmitted SS-blocks in a half frame with SS/PBCH blocks as defined in TS 38.213 [48], clause 4.1. The first/leftmost bit corresponds to SS/PBCH block index 0, the second bit corresponds to SS/PBCH block index 1, and so on. Value 0 in the bitmap indicates that the corresponding SS/PBCH block is not transmitted while value 1 indicates that the corresponding SS/PBCH block is transmitted.

ssb-SubcarrierSpacing Subcarrier spacing of SSB. Only the values 15 kHz or 30 kHz (FR1), and 120 kHz or 240 kHz (FR2) are applicable.

sfn-SSB-Offset Indicates the 10 msec system frame offset of the SSB within the SSB periodicity. Value 0 indicates that the SSB is transmitted in the first system frame; 1 indicates that the SSB is transmitted in the second system frame and so on. This field shall be configured according to the field ssb-Periodicity and the indicated system frame shall not exceed the configured SSB periodicity.

#### –	NR-TimeStamp

The IE NR-TimeStamp defines the UE measurement associated time stamp.

-- ASN1START

NR-TimeStamp-r16 ::= SEQUENCE {

dl-PRS-ID-r16				INTEGER (0..255),

nr-PhysCellID-r16			NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16			NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16				ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

nr-SFN-r16					INTEGER (0..1023),

nr-Slot-r16 				CHOICE {

scs15-r16				INTEGER (0..9),

scs30-r16				INTEGER (0..19),

scs60-r16				INTEGER (0..39),

scs120-r16				INTEGER (0..79)

},

...

}

-- ASN1STOP

NR-TimeStamp field descriptions

dl-PRS-ID This field specifies the DL-PRS ID of the TRP for which the nr-SFN is applicable.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID associated with the dl-PRS-ID.

nr-SFN This field specifies the NR system frame number for the time stamp.

nr-Slot This field specifies the NR slot number within the NR system frame number indicated by nr-SFN for the time stamp.

#### –	NR-TimingQuality

The IE NR-TimingQuality defines the quality of a timing value (e.g., of a TOA measurement).

-- ASN1START

NR-TimingQuality-r16 ::= SEQUENCE {

timingQualityValue-r16			INTEGER (0..31),

timingQualityResolution-r16		ENUMERATED {mdot1, m1, m10, m30, ...},

...

}

-- ASN1STOP

NR-TimingQuality field descriptions

timingQualityValue This field provides an estimate of uncertainty of the timing value for which the IE NR-TimingQuality is provided in units of metres.

timingQualityResolution This field provides the resolution used in the timingQualityValue field. Enumerated values mdot1, m1, m10, m30 correspond to 0.1, 1, 10, 30 metres, respectively.

#### –	NR-TRP-BeamAntennaInfo

The IE NR-TRP-BeamAntennaInfo is used by the location server to provide beam antenna information of the TRP.

-- ASN1START

NR-TRP-BeamAntennaInfo-r17 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-TRP-BeamAntennaInfoPerFreqLayer-r17

NR-TRP-BeamAntennaInfoPerFreqLayer-r17 ::= SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

NR-TRP-BeamAntennaInfoPerTRP-r17

NR-TRP-BeamAntennaInfoPerTRP-r17 ::= SEQUENCE {

dl-PRS-ID-r17						INTEGER (0..255),

nr-PhysCellID-r17					NR-PhysCellID-r16					OPTIONAL,	-- Need ON

nr-CellGlobalID-r17					NCGI-r15							OPTIONAL,	-- Need ON

nr-ARFCN-r17						ARFCN-ValueNR-r15					OPTIONAL,	-- Need ON

lcs-GCS-TranslationParameter-r17	LCS-GCS-TranslationParameter-r16	OPTIONAL,	-- Need OP

nr-TRP-BeamAntennaAngles-r17		NR-TRP-BeamAntennaAngles-r17,

...

}

NR-TRP-BeamAntennaAngles-r17 ::= SEQUENCE (SIZE(1..3600)) OF

NR-TRP-BeamAntennaInfoAzimuthElevation-r17

NR-TRP-BeamAntennaInfoAzimuthElevation-r17 ::= SEQUENCE {

azimuth-r17					INTEGER (0..359)							OPTIONAL,	-- Cond Az

azimuth-fine-r16			INTEGER (0..9)								OPTIONAL,	-- Need ON

elevationList-r17			SEQUENCE (SIZE(1..1801)) OF ElevationElement-R17,

...

}

ElevationElement-R17 ::= SEQUENCE {

elevation-r17				INTEGER (0..180)							OPTIONAL,	-- Cond El

elevation-fine-r17			INTEGER (0..9)								OPTIONAL,	-- Need ON

beamPowerList-r17			SEQUENCE (SIZE (2..maxNumResourcesPerAngle-r17)) OF

BeamPowerElement-r17,

...

}

BeamPowerElement-r17 ::= SEQUENCE {

nr-dl-prs-ResourceSetID-r17		NR-DL-PRS-ResourceSetID-r16				OPTIONAL,	-- Need OP

nr-dl-prs-ResourceID-r17		NR-DL-PRS-ResourceID-r16,

nr-dl-prs-RelativePower-r17		INTEGER (0..500), -- FFS

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Az | The field is mandatory present if the field elevation is absent; otherwise it is optionally present, need ON. |
| El | The field is mandatory present if the field azimuth is absent; otherwise it is optionally present, need ON. |

NR-TRP-BeamAntennaInfo field descriptions

dl-PRS-ID This field specifies the DL-PRS ID of the TRP for which the Beam Antenna Information is provided.

nr-PhysCellID This field specifies the physical Cell-ID of the TRP for which the Beam Antenna Information is provided, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the TRP for which the Beam Antenna Information is provided, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

lcs-GCS-TranslationParameter This field provides the angles α (bearing angle), β (downtilt angle) and γ (slant angle) for the translation of a Local Coordinate System (LCS) to a Global Coordinate System (GCS) as defined in TR 38.901 [44]. If this field is absent, the azimuth and elevation are provided in a GCS.

nr-TRP-BeamAntennaAngles This field provides the relative power between DL-PRS Resources per angle per TRP.

azimuth This field specifies the azimuth angle for which the relative power between DL-PRS Resources is provided. For a Global Coordinate System (GCS), the azimuth angle is measured counter-clockwise from geographical North. For a Local Coordinate System (LCS), the azimuth angle is measured measured counter-clockwise from the x-axis of the LCS. Scale factor 1 degree; range 0 to 359 degrees.

azimuth-fine This field provides finer granularity for the azimuth. The total azimuth angle is given by azimuth + azimuth-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

elevation This field specifies the elevation angle for which the relative power between DL-PRS Resources is provided for the given azimuth. For a Global Coordinate System (GCS), the elevation angle is measured relative to zenith and positive to the horizontal direction (elevation 0 deg. points to zenith, 90 deg to the horizon). For a Local Coordinate System (LCS), the elevation angle is measured relative to the z-axis of the LCS (elevation 0 deg. points to the z-axis, 90 deg to the x-y plane). Scale factor 1 degree; range 0 to 180 degrees.

elevation-fine This field provides finer granularity for the elevation. The total elevation angle is given by elevation + elevation-fine. Scale factor 0.1 degrees; range 0 to 0.9 degrees.

beamPowerList This field provides the relative power between DL-PRS Resources for the angle given by azimuth and elevation. The first BeamPowerElement in this list provides the peak power for this angle and is defined as 0dB power. All the remaining BeamPowerElement's in this list provide the relative DL-PRS Resource power relative to this first element in the list.

nr-dl-prs-ResourceSetID This field specifies the DL-PRS Resource Set ID of the DL-PRS Resource for which the nr-dl-prs-RelativePower is provided. If this field is absent, the DL-PRS Resource Set ID for this instance of the beamPowerList is the same as the DL-PRS Resource Set ID of the previous instance in the beamPowerList. This field shall be included at least in the first instance of the beamPowerList.

nr-dl-prs-ResourceID This field specifies the DL-PRS Resource for which the nr-dl-prs-RelativePower is provided.

nr-dl-prs-RelativePower This field provides the relative power of the DL-PRS Resource, relative to the first element in the beamPowerList. Scale factor -0.1 dB; range 0..-50 dB (FFS)

Editor's Note: Changes to NR-TRP-BeamAntennaInfo-r17 may be needed based on further RAN1 input.

#### –	NR-TRP-LocationInfo

The IE NR-TRP-LocationInfo is used by the location server to provide the coordinates of the antenna reference points for a set of TRPs. For each TRP, the ARP location can be provided for each associated PRS Resource ID per PRS Resource Set.

-- ASN1START

NR-TRP-LocationInfo-r16 ::= SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF

NR-TRP-LocationInfoPerFreqLayer-r16

NR-TRP-LocationInfoPerFreqLayer-r16 ::= SEQUENCE {

referencePoint-r16			ReferencePoint-r16				OPTIONAL,	-- Cond NotSameAsPrev

trp-LocationInfoList-r16	SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF

TRP-LocationInfoElement-r16,

...

}

TRP-LocationInfoElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16			OPTIONAL,	-- Need ON

nr-CellGlobalID-r16				NCGI-r15					OPTIONAL,	-- Need ON

nr-ARFCN-r16					ARFCN-ValueNR-r15			OPTIONAL,	-- Need ON

associated-DL-PRS-ID-r16		INTEGER (0..255)			OPTIONAL,	-- Need OP

trp-Location-r16				RelativeLocation-r16					OPTIONAL,	-- Need OP

trp-DL-PRS-ResourceSets-r16		SEQUENCE (SIZE(1..nrMaxSetsPerTrpPerFreqLayer-r16)) OF

DL-PRS-ResourceSets-TRP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-ResourceSets-TRP-Element-r16 ::= SEQUENCE {

dl-PRS-ResourceSetARP-r16			RelativeLocation-r16				OPTIONAL,	-- Need OP

dl-PRS-Resource-ARP-List-r16		SEQUENCE (SIZE(1..nrMaxResourcesPerSet-r16)) OF

DL-PRS-Resource-ARP-Element-r16	OPTIONAL,	-- Need OP

...

}

DL-PRS-Resource-ARP-Element-r16 ::= SEQUENCE {

dl-PRS-Resource-ARP-location-r16	RelativeLocation-r16				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsPrev | The field is mandatory present in the first entry of the NR-TRP-LocationInfoPerFreqLayer list; otherwise it is optionally present, need OP. |

NR-TRP-LocationInfo field descriptions

referencePoint This field specifies the reference point used to define the TRP location in the trp-LocationInfoList. If this field is absent, the reference point is the same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list.

trp-LocationInfoList This field provides the antenna reference point locations of the DL-PRS Resources for the TRPs and comprises the following sub-fields: - dl-PRS-ID: This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource, and is associated to a single TRP. - nr-PhysCellID: This field specifies the physical cell identity of the associated TRP. - nr-CellGlobalID: This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP. - nr-ARFCN: This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID. - associated-DL-PRS-ID: This field, if present, specifies the dl-PRS-ID of the associated TRP from which the trp-location information is adopted. If the field is present, the field trp-Location shall be absent. - trp-Location: This field provides the location of the TRP relative to the referencePoint location. If this field is absent the TRP location coincides with the referencePoint location, unless the field associated-dl-PRS-ID is present, in which case the trp-Location is adopted from the associated TRP indicated by associated-dl-PRS-ID. - trp-DL-PRS-ResourceSets: This field provides the antenna reference point location(s) of the DL-PRS Resource Set(s) associated with this TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resource Set(s) coincides with the trp-Location location. This field comprises the following sub-fields: - dl-PRS-ResourceSetARP: This field provides the antenna reference point location of the DL-PRS Resource Set relative to the trp-Location location. If this field is absent, the antenna reference point location of this DL-PRS Resource Set coincides with the trp-Location location. - dl-PRS-Resource-ARP-List: This field provides the antenna reference point location(s) of the DL-PRS Resource(s) associated with this Resource Set of the TRP. If this field is absent, the antenna reference point location(s) of the DL-PRS Resources coincides with the dl-PRS-ResourceSetARP location. This field comprises the following sub-fields: - dl-PRS-Resource-ARP-location: This field provides the antenna reference point location of the DL-PRS Resource associated with the DL-PRS Resource Set of the TRP relative to the dl-PRS-ResourceSetARP location. If this field is absent, the antenna reference point location of this DL-PRS Resource coincides with the dl-PRS-ResourceSetARP location.

#### –	NR-UE-TEG-Capability

The IE NR-UE-TEG-Capability defines the TEG capability of the target device. In the case of capabilities for DL-TDOA and Multi-RTT positioning methods are provided, the IE NR-UE-TEG-Capability applies across the two NR positioning methods and the target device shall indicate the same values for the capabilities in IEs NR-DL-TDOA-ProvideCapabilities and NR-Multi-RTT-ProvideCapabilities (as applicable).

-- ASN1START

NR-UE-TEG-Capability-r17 ::= SEQUENCE {

nr-UE-TEG-ID-CapabilityBandList-r17		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-UE-TEG-ID-CapabilityPerBand-r17,

...

}

NR-UE-TEG-ID-CapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

nr-UE-RxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,

nr-UE-TxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,

nr-UE-RxTxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8, n12, n16,

n24, n32, n36, n48, n64}		OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED {n2, n3, n4, n6, n8}				OPTIONAL,

measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul-r17

ENUMERATED {n1, n2, n3, n4, n6, n8}			OPTIONAL,

...

}

-- ASN1STOP

NR-UE-TEG-Capability field descriptions

nr-UE-RxTEG-ID-MaxSupport Indicates the maximum number of UE-RxTEGs, which is supported and reported by the UE. This field is applicable for UE assisted DL-TDOA and Multi-RTT positioning.

nr-UE-TxTEG-ID-MaxSupport Indicates the maximum number of UE-TxTEGs, which is supported and reported by the UE. This field is applicable for Multi-RTT positioning.

nr-UE-RxTxTEG-ID-MaxSupport Indicates the maximum number of UE-RxTxTEGs, which is supported and reported by the UE. This field is applicable for Multi-RTT positioning.

measureSameDL-PRS-ResourceWithDifferentRxTEGs Indicates the maximum number of different UE-RxTEGs that a UE can support to measure the same DL-PRS Resource of a TRP. This field is applicable for UE assisted DL-TDOA and Multi-RTT positioning.

measureSameDL-PRS-ResourceWithDifferentRxTEGsSimul Indicates the maximum number of UE Rx TEGs for measuring the same DL-PRS Resource simultaneously. This field is applicable for UE assisted DL-TDOA and Multi-RTT positioning.

#### –	NR-UL-SRS-Capability

The IE NR-UL-SRS-Capability defines the UE uplink SRS capability.

-- ASN1START

NR-UL-SRS-Capability-r16 ::= SEQUENCE {

srs-CapabilityBandList-r16					SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

SRS-CapabilityPerBand-r16,

srs-PosResourceConfigCA-BandList-r16		SEQUENCE (SIZE (1..nrMaxConfiguredBands-r16)) OF

SRS-PosResourcesPerBand-r16			OPTIONAL,

maxNumberSRS-PosPathLossEstimateAllServingCells-r16

ENUMERATED {n1, n4, n8, n16}			OPTIONAL,

maxNumberSRS-PosSpatialRelationsAllServingCells-r16

ENUMERATED {n0, n1, n2, n4, n8, n16}	OPTIONAL,

...

}

SRS-CapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16			FreqBandIndicatorNR-r16,

olpc-SRS-Pos-r16				OLPC-SRS-Pos-r16									OPTIONAL,

spatialRelationsSRS-Pos-r16		SpatialRelationsSRS-Pos-r16							OPTIONAL,

...,

[[

olpc-SRS-PosRRC-Inactive-r17				OLPC-SRS-Pos-r16						OPTIONAL,

spatialRelationsSRS-PosRRC-Inactive-r16		SpatialRelationsSRS-Pos-r16				OPTIONAL

]]

}

OLPC-SRS-Pos-r16 ::= SEQUENCE {

olpc-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnSSB-Neigh-r16 		ENUMERATED {supported}						OPTIONAL,

olpc-SRS-PosBasedOnPRS-Neigh-r16		ENUMERATED {supported}						OPTIONAL,

maxNumberPathLossEstimatePerServing-r16	ENUMERATED {n1, n4, n8, n16}				OPTIONAL,

...

}

SpatialRelationsSRS-Pos-r16 ::=	SEQUENCE {

spatialRelation-SRS-PosBasedOnSSB-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnCSI-RS-Serving-r16	ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Serving-r16		ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSRS-r16				ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnSSB-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

spatialRelation-SRS-PosBasedOnPRS-Neigh-r16			ENUMERATED {supported}			OPTIONAL,

...

}

SRS-PosResourcesPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16							FreqBandIndicatorNR-r16,

maxNumberSRS-PosResourceSetsPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n12, n16},

maxNumberSRS-PosResourcesPerBWP-r16				ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberPeriodicSRS-PosResourcesPerBWP-r16		ENUMERATED {n1, n2, n4, n8, n16, n32, n64},

maxNumberAP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

maxNumberSP-SRS-PosResourcesPerBWP-r16			ENUMERATED {n1, n2, n4, n8, n16, n32, n64}

OPTIONAL,

...

}

-- ASN1STOP

NR-UL-SRS-Capability field descriptions

maxNumberSRS-PosPathLossEstimateAllServingCells Indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning across all cells in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

maxNumberSRS-PosSpatialRelationsAllServingCells indicates the maximum number of maintained spatial relations for all the SRS resource sets for positioning across all serving cells in addition to the spatial relations maintained spatial relations per serving cell for the PUSCH/PUCCH/SRS transmissions. It is only applied for FR2. The UE can include this field only if the UE supports any of spatialRelation-SRS-PosBasedOnSSB-Serving, spatialRelation-SRS-PosBasedOnCSI-RS-Serving, spatialRelation-SRS-PosBasedOnPRS-Serving, spatialRelation-SRS-PosBasedOnSSB-Neigh or spatialRelation-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

olpc-SRS-Pos Indicates whether the UE supports open-loop power control for SRS for positioning. The capability signalling comprises the following parameters. - olpc-SRS-PosBasedOnPRS-Serving indicates whether the UE supports OLPC for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports NR-DL-PRS-ProcessingCapability and srs-PosResources TS38.331 [35] Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports OLPC for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - olpc-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports OLPC for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports olpc-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell. - maxNumberPathLossEstimatePerServing indicates the maximum number of pathloss estimates that the UE can simultaneously maintain for all the SRS resource sets for positioning per serving cell in addition to the up to four pathloss estimates that the UE maintains per serving cell for the PUSCH/PUCCH/SRS transmissions. The UE shall include this field if the UE supports any of olpc-SRS-PosBasedOnPRS-Serving, olpc-SRS-PosBasedOnSSB-Neigh and olpc-SRS-PosBasedOnPRS-Neigh. Otherwise, the UE does not include this field.

spatialRelationsSRS-Pos Indicates whether the UE supports spatial relations for SRS for positioning. It is only applicable for FR2. The capability signalling comprises the following parameters. - spatialRelation-SRS-PosBasedOnSSB-Serving indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the serving cell in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnCSI-RS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on CSI-RS from the serving cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Serving indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the serving cell in the same band. The UE can include this field only if the UE supports any of DL-PRS Resources for DL-AoD, DL-PRS Resources for DL-TDOA or DL-PRS Resources for Multi-RTT, or srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSRS indicates whether the UE supports spatial relation for SRS for positioning based on SRS in the same band. The UE can include this field only if the UE supports srs-PosResources TS 38.331 [35]. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnSSB-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on SSB from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnSSB-Serving. Otherwise, the UE does not include this field. - spatialRelation-SRS-PosBasedOnPRS-Neigh indicates whether the UE supports spatial relation for SRS for positioning based on PRS from the neighbouring cell in the same band. The UE can include this field only if the UE supports spatialRelation-SRS-PosBasedOnPRS-Serving. Otherwise, the UE does not include this field. Note: A PRS from a PRS-only TP is treated as PRS from a non-serving cell.

olpc-SRS-PosRRC-Inactive Indicates whether the UE supports open-loop power control for SRS for positioning in RRC_INACTIVE state.

spatialRelationsSRS-PosRRC-Inactive Indicates whether the UE supports spatial relations for SRS for positioning in RRC_INACTIVE state.

srs-PosResourceConfigCA-BandList This field indicates the number of SRS for positioning resources supported by the target device. The target device includes this field for each band for the current configured CA band combination. The capability signalling comprises the following parameters. - freqBandIndicatorNR indicates the current configured NR band of the target device. - maxNumberSRS-PosResourceSetsPerBWP indicates the maximum number of SRS Resource Sets for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n12, n16 correspond to 1, 2, 4, 8, 12, 16 SRS Resource Sets for positioning, respectively. - maxNumberSRS-PosResourcesPerBWP indicates the maximum number of periodic, semi-persistent, and aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 SRS Resources for positioning, respectively. - maxNumberPeriodicSRS-PosResourcesPerBWP indicates the maximum number of periodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 periodic SRS Resources for positioning, respectively. - maxNumberAP-SRS-PosResourcesPerBWP indicates the maximum number of aperiodic SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 aperiodic SRS Resources for positioning, respectively. - maxNumberSP-SRS-PosResourcesPerBWP indicates the maximum number of semi-persistent SRS Resources for positioning supported by the target device per BWP. Enumerated values n1, n2, n4, n8, n16, n32, n64 correspond to 1, 2, 4, 8, 16, 32, 64 semi-persistent SRS Resources for positioning, respectively.

#### –	ReferencePoint

The IE ReferencePoint provides a well-defined location relative to which other locations may be defined.

-- ASN1START

ReferencePoint-r16 ::= SEQUENCE {

referencePointGeographicLocation-r16 		CHOICE {

location3D-r16 			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

ha-location3D-r16 		HighAccuracyEllipsoidPointWithAltitudeAndUncertaintyEllipsoid-r15,

...

},

...

}

-- ASN1STOP

ReferencePoint field descriptions

referencePointGeographicLocation This field provides the geodetic location of the reference point.

#### –	RelativeLocation

The IE RelativeLocation provides a location relative to some known reference location.

-- ASN1START

RelativeLocation-r16 ::= SEQUENCE {

milli-arc-second-units-r16 	ENUMERATED { mas0-03, mas0-3, mas3, mas30, ...},

height-units-r16			ENUMERATED {mm, cm, m, ...},

delta-latitude-r16			Delta-Latitude-r16,

delta-longitude-r16			Delta-Longitude-r16,

delta-height-r16			Delta-Height-r16,

locationUNC-r16				LocationUncertainty-r16				OPTIONAL,		-- Need OP

...

}

Delta-Latitude-r16 ::= SEQUENCE {

delta-Latitude-r16						INTEGER (-1024..1023),

coarse-delta-Latitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Longitude-r16 ::= SEQUENCE {

delta-Longitude-r16						INTEGER (-1024..1023),

coarse-delta-Longitude-r16				INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

Delta-Height-r16 ::= SEQUENCE {

delta-Height-r16						INTEGER (-1024..1023),

coarse-delta-Height-r16					INTEGER (0..4095)		OPTIONAL,		-- Need OP

...

}

LocationUncertainty-r16 ::= SEQUENCE {

horizontalUncertainty-r16				INTEGER (0..255),

horizontalConfidence-r16				INTEGER (0..100),

verticalUncertainty-r16					INTEGER (0..255),

verticalConfidence-r16					INTEGER (0..100)

}

-- ASN1STOP

RelativeLocation field descriptions

milli-arc-second-units This field provides the units and scale factor for the delta-latitude and delta-longitude fields. Enumerated values mas0-03, mas0-3, mas3, and mas30, correspond to 0.03, 0.3, 3, and 30 milliarcseconds, respectively.

height-units This field provides the units and scale factor for the delta-height field. Enumerated values mm, cm, and m correspond to 10-3 metre, 10-2 metre, and 1 metres, respectively.

delta-latitude This field specifies the delta value in latitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Latitude specifies the delta value in latitude in the unit provided in milli-arc-second-units field. - coarse-delta-Latitude specifies the delta value in latitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Latitude field. If this field is absent, the value for coarse-delta-Latitude is zero. I.e., the full delta-latitude is given by: (delta-Latitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-longitude This field specifies the delta value in longitude of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Longitude specifies the delta value in longitude in the unit provided in milli-arc-second-units field. - coarse-delta-Longitude specifies the delta value in longitude in 1024 times the size of the unit provided in milli-arc-second-units field and with the same sign as in the delta-Longitude field. If this field is absent, the value for coarse-delta-Longitude is zero. I.e., the full delta-longitude is given by: (delta-Longitude × milli-arc-second-units) ± (coarse-delta-Latitude × 1024 × milli-arc-second-units) [milli-arc-seconds]

delta-height This field specifies the delta value in ellipsoidal height of the desired location, defined as "desired location" minus "reference point location" and comprises the following sub-fields: - delta-Height specifies the delta value in ellipsoidal height in the unit provided in height-units field. - coarse-delta-Height specifies the delta value in ellipsoidal height in 1024 times the size of the unit provided in height-units field and with the same sign as in the delta-Height field. If this field is absent, the value for coarse-delta-Height is zero. I.e., the full delta-height is given by: (delta-Height × height-units) ± (coarse-delta-Height × 1024 × height-units) [metres]

locationUNC This field specifies the uncertainty of the location coordinates and comprises the following sub-fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15]. If this field is absent, the uncertainty is the same as for the associated reference point location.

## 6.5	Positioning Method IEs

### 6.5.1	OTDOA Positioning

This clause defines the information elements for downlink OTDOA positioning, which includes TBS positioning based on PRS signals (TS 36.305 [2]).

#### 6.5.1.1	OTDOA Assistance Data

#### –	OTDOA-ProvideAssistanceData

The IE OTDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted downlink OTDOA. It may also be used to provide OTDOA positioning specific error reason.

Throughout clause 6.5.1, "assistance data reference cell" refers to the cell defined by the IE OTDOA-ReferenceCellInfo and "NB-IoT assistance data reference cell" refers to the cell defined by the IE OTDOA-ReferenceCellInfoNB (see clause 6.5.1.2). "RSTD reference cell" applies only in clause 6.5.1.5.

If both IEs, OTDOA-ReferenceCellInfo and OTDOA-ReferenceCellInfoNB are included in OTDOA-ProvideAssistanceData, the assistance data reference cell and NB-IoT assistance data reference cell correspond to the same cell, and the target device may assume that PRS and NPRS antenna ports are quasi co-located, as defined in TS 36.211 [16].

Throughout clause 6.5.1, the term "cell" refers to "transmission point (TP)", unless distinguished in the field description.

NOTE 1:	The location server should include at least one cell for which the SFN can be obtained by the target device, e.g. the serving cell, in the assistance data, either as the assistance data reference cell or in the neighbour cell list. Otherwise the target device will be unable to perform the OTDOA measurement and the positioning operation will fail.

NOTE 2:	Due to support of cells containing multiple TPs and PRS-only TPs not associated with cells, the term "cell" as used in clause 6.5.1 may not always correspond to a cell for the E-UTRAN.

NOTE 3:	For NB-IoT access, due to support of NPRS on multiple carriers, the term "cell" as used in clause 6.5.1 refers to the anchor carrier, unless otherwise stated.

-- ASN1START

OTDOA-ProvideAssistanceData ::= SEQUENCE {

otdoa-ReferenceCellInfo			OTDOA-ReferenceCellInfo				OPTIONAL,	-- Need ON

otdoa-NeighbourCellInfo			OTDOA-NeighbourCellInfoList			OPTIONAL,	-- Need ON

otdoa-Error						OTDOA-Error							OPTIONAL,	-- Need ON

...,

[[

otdoa-ReferenceCellInfoNB-r14	OTDOA-ReferenceCellInfoNB-r14		OPTIONAL,	-- Need ON

otdoa-NeighbourCellInfoNB-r14	OTDOA-NeighbourCellInfoListNB-r14	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### 6.5.1.2	OTDOA Assistance Data Elements

#### –	OTDOA-ReferenceCellInfo

The IE OTDOA-ReferenceCellInfo is used by the location server to provide assistance data reference cell information for OTDOA assistance data. The slot number offsets and expected RSTDs in OTDOA-NeighbourCellInfoList are provided relative to the cell defined by this IE. If earfcnRef of this assistance data reference cell is different from that of the serving cell, the LPP layer shall inform lower layers to start performing inter-frequency RSTD measurements with this cell and provide to lower layers the information about this assistance data reference cell, e.g. EARFCN and PRS positioning occasion information.

NOTE:	The location server should always include the PRS configuration of the assistance data reference and neighbour cells. Otherwise the UE may not meet the accuracy requirements as defined in TS 36.133 [18].

-- ASN1START

OTDOA-ReferenceCellInfo ::= SEQUENCE {

physCellId					INTEGER (0..503),

cellGlobalId				ECGI						OPTIONAL,		-- Need ON

earfcnRef					ARFCN-ValueEUTRA			OPTIONAL,		-- Cond NotSameAsServ0

antennaPortConfig			ENUMERATED {ports1-or-2, ports4, ... }

OPTIONAL,		-- Cond NotSameAsServ1

cpLength					ENUMERATED { normal, extended, ... },

prsInfo						PRS-Info					OPTIONAL,		-- Cond PRS

...,

[[ earfcnRef-v9a0			ARFCN-ValueEUTRA-v9a0		OPTIONAL		-- Cond NotSameAsServ2

]],

[[	tpId-r14				INTEGER (0..4095)			OPTIONAL,		-- Need ON

cpLengthCRS-r14			ENUMERATED { normal, extended, ... }

OPTIONAL,		-- Cond CRS

sameMBSFNconfigRef-r14	BOOLEAN						OPTIONAL,		-- Need ON

dlBandwidth-r14			ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,		-- Cond NotSameAsServ3

addPRSconfigRef-r14		SEQUENCE (SIZE (1..maxAddPRSconfig-r14)) OF PRS-Info

OPTIONAL		-- Need ON

]],

[[

nr-LTE-SFN-Offset-r15	INTEGER (0..1023)			OPTIONAL		-- Cond NR

]],

[[

tdd-config-v1520					TDD-Config-v1520	OPTIONAL,		-- Need ON

nr-LTE-fineTiming-Offset-r15		INTEGER (0..19)		OPTIONAL		-- Cond FineOffset

]]

}

maxAddPRSconfig-r14			INTEGER ::= 2

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsServ0 | This field is absent if earfcnRef-v9a0 is present. Otherwise, the field is mandatory present if the EARFCN of the OTDOA assistance data reference cell is not the same as the EARFCN of the target devices' current primary cell. |
| NotSameAsServ1 | The field is mandatory present if the antenna port configuration of the OTDOA assistance data reference cell is not the same as the antenna port configuration of the target devices' current primary cell. |
| NotSameAsServ2 | The field is absent if earfcnRef is present. Otherwise, the field is mandatory present if the EARFCN of the OTDOA assistance data reference cell is not the same as the EARFCN of the target devices' current primary cell. |
| PRS | The field is mandatory present if positioning reference signals are available in the assistance data reference cell (TS 36.211 [16]); otherwise it is not present. |
| CRS | The field is optionally present, need ON, if prsInfo is present. Otherwise it is not present. |
| NotSameAsServ3 | The field is mandatory present if the downlink bandwidth configuration of the assistance data reference cell is not the same as the downlink bandwidth configuration of the target devices' current primary cell and if PRS frequency hopping is used in the assistance data reference cell (TS 36.211 [16]); otherwise it is not present. |
| NR | The field is optionally present, need ON, if the target device is served by an NR cell; otherwise it is not present. |
| FineOffset | The field is optionally present, need ON, if nr-LTE-SFN-Offset is present. Otherwise it is not present. |

OTDOA-ReferenceCellInfo field descriptions

physCellId This field specifies the physical cell identity of the assistance data reference cell, as defined in TS 36.331 [12].

cellGlobalId This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the assistance data reference cell, as defined in TS 36.331 [12]. The server should include this field if it considers that it is needed to resolve ambiguity in the cell indicated by physCellId.

earfcnRef This field specifies the EARFCN of the assistance data reference cell.

antennaPortConfig This field specifies whether 1 (or 2) antenna port(s) or 4 antenna ports for cell specific reference signals (CRS) are used in the assistance data reference cell.

cpLength This field specifies the cyclic prefix length of the assistance data reference cell PRS if the prsInfo field is present, otherwise this field specifies the cyclic prefix length of the assistance data reference cell CRS.

prsInfo This field specifies the first PRS configuration of the assistance data reference cell.

tpId This field specifies an identity of the transmission point. This field together with the physCellId and/or prsID may be used to identify the transmission point in the case the same physical cell ID is shared by multiple transmission points.

cpLengthCRS This field specifies the cyclic prefix length of the assistance data reference cell CRS. If this field is present, the target device may assume the CRS and PRS antenna ports of the assistance data reference cell are quasi co-located (as defined in TS 36.211 [16]).

sameMBSFNconfigRef This field indicates whether the MBSFN subframe configuration of the assistance data reference cell is the same as the current primary cell of the target device. TRUE means the same, and FALSE means not the same.

dlBandwidth This field specifies the downlink bandwidth configuration of the assistance data reference cell, NRB in downlink, see TS 36.101 [21, table 5.6-1]. Enumerated value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

addPRSconfigRef This field specifies the additional (second and possibly third) PRS configuration(s) of the assistance data reference cell.

nr-LTE-SFN-Offset This field specifies the SFN offset between the serving NR cell and the LTE assistance data reference cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the NR serving cell to the beginning of the closest subsequent radio frame #0 of the assistance data reference cell.

tdd-config This field specifies the TDD specific physical channel configuration of the assistance data reference cell. The field should be present if the assistance data reference cell is a TDD cell and if the TDD UL/DL configuration of the assistance data reference cell is not the same as the target devices' current primary cell or if the target devices' current primary cell is a FDD cell.

nr-LTE-fineTiming-Offset This field specifies the frame boundary offset between the NR serving cell and the LTE assistance data reference cell in units of 0.5 ms. The offset is counted from the beginning of a subframe #0 of the NR serving cell to the beginning of the closest subsequent subframe #0 of the LTE assistance data reference cell, rounded down to multiples of 0.5 ms. Value 0 corresponds to 0 ms, value 1 corresponds to 0.5 ms, 2 to 1 ms and so on.

#### –	PRS-Info

The IE PRS-Info provides the information related to the configuration of PRS in a cell.

-- ASN1START

PRS-Info ::= SEQUENCE {

prs-Bandwidth			ENUMERATED { n6, n15, n25, n50, n75, n100, ... },

prs-ConfigurationIndex	INTEGER (0..4095),

numDL-Frames			ENUMERATED {sf-1, sf-2, sf-4, sf-6, ..., sf-add-v1420},

...,

prs-MutingInfo-r9		CHOICE {

po2-r9					BIT STRING (SIZE(2)),

po4-r9					BIT STRING (SIZE(4)),

po8-r9					BIT STRING (SIZE(8)),

po16-r9					BIT STRING (SIZE(16)),

...,

po32-v1420				BIT STRING (SIZE(32)),

po64-v1420				BIT STRING (SIZE(64)),

po128-v1420				BIT STRING (SIZE(128)),

po256-v1420				BIT STRING (SIZE(256)),

po512-v1420				BIT STRING (SIZE(512)),

po1024-v1420			BIT STRING (SIZE(1024))

}														OPTIONAL,				-- Need OP

[[	prsID-r14				INTEGER (0..4095)			OPTIONAL,				-- Need ON

add-numDL-Frames-r14	INTEGER (1..160)			OPTIONAL,				-- Cond sf-add

prsOccGroupLen-r14		ENUMERATED {g2, g4, g8, g16, g32, g64, g128,... }

OPTIONAL,				-- Cond Occ-Grp

prsHoppingInfo-r14	CHOICE {

nb2-r14				INTEGER (0.. maxAvailNarrowBands-Minus1-r14),

nb4-r14				SEQUENCE (SIZE (3))

OF INTEGER (0.. maxAvailNarrowBands-Minus1-r14)

}														OPTIONAL				-- Cond PRS-FH

]]

}

maxAvailNarrowBands-Minus1-r14		INTEGER ::= 15	-- Maximum number of narrowbands minus 1

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| sf-add | The field is mandatory present if the numDL-Frames field has the value 'sf-add'; otherwise it is not present. |
| Occ-Grp | The field is mandatory present if a PRS occasion group is configured; otherwise it is not present. |
| PRS-FH | The field is mandatory present if frequency hopping is used for PRS; otherwise it is not present. |

PRS-Info field descriptions

prs-Bandwidth This field specifies the bandwidth that is used to configure the positioning reference signals on. Enumerated values are specified in number of resource blocks (n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on) and define 1.4, 3, 5, 10, 15 and 20 MHz bandwidth.

prs-ConfigurationIndex This field specfies the positioning reference signals configuration index IPRS as defined in TS 36.211 [16].

numDL-Frames This field specifies the number of consecutive downlink subframes NPRS with positioning reference signals, as defined in TS 36.211 [16]. Enumerated values define 1, 2, 4, or 6 consecutive downlink subframes. The value sf-add indicates that NPRS is provided in the field add-numDL-Frames.

prs-MutingInfo This field specifies the PRS muting configuration of the cell. The PRS muting configuration is defined by a periodic PRS muting sequence with periodicity TREP where TREP, counted in the number of PRS occasion groups (TS 36.133 [18]), can be 2, 4, 8, 16, 32, 64, 128, 256, 512, or 1024 which is also the length of the selected bit string that represents this PRS muting sequence. If a bit in the PRS muting sequence is set to "0", then the PRS is muted in all the PRS occasions in the corresponding PRS occasion group. A PRS occasion group comprises one or more PRS occasions as indicated by prsOccGroupLen. Each PRS occasion comprises NPRS downlink positioning subframes as defined in TS 36.211 [16]. The first bit of the PRS muting sequence corresponds to the first PRS occasion group that starts after the beginning of the assistance data reference cell SFN=0. The sequence is valid for all subframes after the target device has received the prs-MutingInfo. If this field is not present the target device may assume that the PRS muting is not in use for the cell. When the SFN of the assistance data reference cell is not known to the UE and prs-MutingInfo is provided for a cell in the OTDOA-NeighbourCellInfoList IE, the UE may assume no PRS is transmitted by that cell. When the UE receives a TREP-bit muting pattern together with a PRS periodicity TPRS for the same cell which exceeds 10240 subframes (i.e., TREP × TPRS > 10240 subframes), the UE shall assume an n-bit muting pattern based on the first n-bits, where n = 10240/TPRS.

prsID This field specifies the PRS-ID as defined in TS 36.211 [16].

add-numDL-Frames This field specifies the number of consecutive downlink subframes NPRS with positioning reference signals, as defined in TS 36.211 [16]. Integer values define 1, 2, 3, …, 160 consecutive downlink subframes.

prsOccGroupLen This field specifies the PRS occasion group length, defined as the number of consecutive PRS occasions comprising a PRS occasion group. Each PRS occasion of the PRS occasion group consists of numDL-Frames or add-numDL-Frames consecutive downlink subframes with positioning reference signals. Enumerated values define 2, 4, 8, 16, 32, 64 or 128 consecutive PRS occasions. If omitted, the PRS occasion group length is 1. The product of the PRS periodicity T_PRS from the prs-ConfigurationIndex and the PRS occasion group length cannot exceed 1280.

prsHoppingInfo This field specifies the PRS frequency hopping configuration (TS 36.211 [16]). The choice nb2 indicates hopping between 2 narrowbands; the choice nb4 indicates hopping between 4 narrowbands. The first PRS positioning occasion of the first PRS occasion group that starts after the beginning of SFN=0 of the assistance data reference cell is located at the centre of the system bandwidth. The frequency band of each subsequent PRS occasion is indicated by nb2 or nb4, respectively, which defines the narrowband index as specified in TS 36.211 [16]. If this field is absent, no PRS frequency hopping is used.

#### –	TDD-Config

The IE TDD-Config is used to specify the TDD specific physical channel configuration.

-- ASN1START

TDD-Config-v1520 ::= SEQUENCE {

subframeAssignment-v1520			ENUMERATED { sa0, sa1, sa2, sa3, sa4, sa5, sa6 },

...

}

-- ASN1STOP

TDD-Config field descriptions

subframeAssignment This field specifies the TDD UL/DL subframe configuration where sa0 points to Configuration 0, sa1 to Configuration 1 etc. as specified in TS 36.211 [16], table 4.2-2. The target device assumes the same value for all assistance data cells residing on same frequency band.

#### –	OTDOA-NeighbourCellInfoList

The IE OTDOA-NeighbourCellInfoList is used by the location server to provide neighbour cell information for OTDOA assistance data. If the target device is not capable of supporting additional neighbour cells (as indicated by the absence of the IE additionalNeighbourCellInfoList in OTDOA-ProvideCapabilities), the set of cells in the OTDOA-NeighbourCellInfoList is grouped per frequency layer and in the decreasing order of priority for measurement to be performed by the target device, with the first cell in the list being the highest priority for measurement and with the same earfcn not appearing in more than one instance of OTDOA-NeighbourFreqInfo.

If the target device is capable of supporting additional neighbour cells (as indicated by the presence of the IE additionalNeighbourCellInfoList in OTDOA-ProvideCapabilities), the list may contain all cells (up to 3x24 cells) belonging to the same frequency layer or cells from different frequency layers with the first cell in the list still being the highest priority for measurement.

The prioritization of the cells in the list is left to server implementation. The target device should provide the available measurements in the same order as provided by the server.

If inter-frequency neighbour cells are included in OTDOA-NeighbourCellInfoList, where an inter-frequency is a E-UTRA frequency which is different from the E-UTRA serving cell frequency, the LPP layer shall inform lower layers to start performing inter-frequency RSTD measurements for these neighbour cells and also provide to lower layers the information about these neighbour cells, e.g. EARFCN and PRS positioning occasion information.

-- ASN1START

OTDOA-NeighbourCellInfoList ::= SEQUENCE (SIZE (1..maxFreqLayers)) OF OTDOA-NeighbourFreqInfo

OTDOA-NeighbourFreqInfo ::= SEQUENCE (SIZE (1..24)) OF OTDOA-NeighbourCellInfoElement

OTDOA-NeighbourCellInfoElement ::= SEQUENCE {

physCellId							INTEGER (0..503),

cellGlobalId						ECGI				OPTIONAL,		-- Need ON

earfcn								ARFCN-ValueEUTRA	OPTIONAL,		-- Cond NotSameAsRef0

cpLength							ENUMERATED {normal, extended, ...}

OPTIONAL,		-- Cond NotSameAsRef1

prsInfo								PRS-Info			OPTIONAL,		-- Cond NotSameAsRef2

antennaPortConfig					ENUMERATED {ports-1-or-2, ports-4, ...}

OPTIONAL, 		-- Cond NotsameAsRef3

slotNumberOffset					INTEGER (0..19)		OPTIONAL,		-- Cond NotSameAsRef4

prs-SubframeOffset					INTEGER (0..1279)	OPTIONAL,		-- Cond InterFreq

expectedRSTD						INTEGER (0..16383),

expectedRSTD-Uncertainty			INTEGER (0..1023),

...,

[[ earfcn-v9a0					ARFCN-ValueEUTRA-v9a0	OPTIONAL		-- Cond NotSameAsRef5

]],

[[	tpId-r14					INTEGER (0..4095)		OPTIONAL,		-- Need ON

prs-only-tp-r14				ENUMERATED { true }		OPTIONAL,		-- Cond TBS

cpLengthCRS-r14				ENUMERATED { normal, extended, ... }

OPTIONAL,		-- Cond CRS

sameMBSFNconfigNeighbour-r14	BOOLEAN				OPTIONAL,		-- Need ON

dlBandwidth-r14				ENUMERATED {n6, n15, n25, n50, n75, n100}

OPTIONAL,		-- Cond NotSameAsRef6

addPRSconfigNeighbour-r14	SEQUENCE (SIZE (1..maxAddPRSconfig-r14)) OF

Add-PRSconfigNeighbourElement-r14

OPTIONAL		-- Need ON

]],

[[

tdd-config-v1520			TDD-Config-v1520		OPTIONAL		-- Need ON

]]

}

Add-PRSconfigNeighbourElement-r14 ::= SEQUENCE {

add-prsInfo-r14					PRS-Info				OPTIONAL,		-- Cond NotSameAsRef7

...

}

maxFreqLayers	INTEGER ::= 3

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotsameAsRef0 | The field is absent if earfcn-v9a0 is present. If earfcn-v9a0 is not present, the field is mandatory present if the EARFCN is not the same as for the assistance data reference cell; otherwise it is not present. |
| NotsameAsRef1 | The field is mandatory present if the cyclic prefix length is not the same as for the assistance data reference cell; otherwise it is not present. |
| NotsameAsRef2 | The field is mandatory present if the first PRS configuration is not the same as for the assistance data reference cell; otherwise it is not present. |
| NotsameAsRef3 | The field is mandatory present if the antenna port configuration is not the same as for the assistance data reference cell; otherwise it is not present. |
| NotsameAsRef4 | The field is mandatory present if the slot timing is not the same as for the assistance data reference cell; otherwise it is not present. |
| NotSameAsRef5 | The field is absent if earfcn is present. If earfcn is not present, the field is mandatory present if the EARFCN is not the same as for the assistance data reference cell; otherwise it is not present. |
| InterFreq | The field is optionally present, need OP, if the EARFCN is not the same as for the assistance data reference cell; otherwise it is not present. |
| TBS | The field is mandatory present if the OTDOA-NeighbourCellInfoElement is provided for a PRS-only TP; otherwise it is not present. |
| CRS | The field is optionally present, need ON, if prsInfo is present. Otherwise it is not present. |
| NotSameAsRef6 | The field is mandatory present if PRS frequency hopping is used on this neighbour cell (TS 36.211 [16]) and if the downlink bandwidth configuration is not the same as for the assistance data reference cell; otherwise it is not present. |
| NotSameAsRef7 | The field is mandatory present if any instance of the additional PRS configurations of addPRSconfigNeighbour is not the same as the corresponding instance of the additional PRS configuration of the addPRSconfigRef for the assistance data reference cell; otherwise it is not present. |

OTDOA-NeighbourCellInfoList field descriptions

physCellId This field specifies the physical cell identity of the neighbour cell, as defined in TS 36.331 [12].

cellGlobalId This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the neighbour cell, as defined in TS 36.331 [12]. The server should provide this field if it considers that it is needed to resolve any ambiguity in the cell identified by physCellId.

earfcn This field specifies the EARFCN of the neighbour cell.

cpLength This field specifies the cyclic prefix length of the neigbour cell PRS if PRS are present in this neighbour cell, otherwise this field specifies the cyclic prefix length of CRS in this neighbour cell.

prsInfo This field specifies the first PRS configuration of the neighbour cell. When the EARFCN of the neighbour cell is the same as for the assistance data reference cell, the target device may assume that each PRS positioning occasion in the neighbour cell at least partially overlaps with a PRS positioning occasion in the assistance data reference cell where the maximum offset between the transmitted PRS positioning occasions may be assumed to not exceed half a subframe. When the EARFCN of the neighbour cell is the same as for the assistance data reference cell, the target may assume that this cell has the same PRS periodicity (TPRS) as the assistance data reference cell.

antennaPortConfig This field specifies whether 1 (or 2) antenna port(s) or 4 antenna ports for cell specific reference signals are used.

slotNumberOffset This field specifies the slot number offset at the transmitter between this cell and the assistance data reference cell. The slotNumberOffset together with the current slot number of the assistance data reference cell may be used to calculate the current slot number of this cell which may further be used to generate the CRS sequence by the target device. The offset corresponds to the number of full slots counted from the beginning of a radio frame of the assistance data reference cell to the beginning of the closest subsequent radio frame of this cell. If this field is absent, the slot timing is the same as for the assistance data reference cell.

prs-SubframeOffset This field specifies the offset between the first PRS subframe of the first PRS occasion group of the first PRS configuration in the assistance data reference cell on the reference carrier frequency layer and the first PRS subframe in the closest subsequent PRS occasion group of the PRS configuration with the longest PRS occasion group periodicity (NOTE 1) of this cell on the other carrier frequency layer. The value is given in number of full sub-frames. If the EARFCN is not the same as for the assistance data reference cell and the field is not present but PRS are available on this cell, the receiver shall consider the PRS subframe offset for this cell to be 0.

expectedRSTD If PRS is transmitted: This field indicates the RSTD value that the target device is expected to measure between this cell and the assistance data reference cell. The expectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of PRS positioning occasions between the two cells. The RSTD value can be negative and is calculated as (expectedRSTD-8192). The resolution is 3Ts, with Ts=1/(15000*2048) seconds. If PRS is not transmitted: This field indicates the RSTD value that the target device is expected to measure between this cell and the assistance data reference cell. The expectedRSTD field takes into account the expected propagation time difference as well as transmit time difference between the two cells. The RSTD value can be negative and is calculated as (expectedRSTD-8192). The resolution is 3´Ts, with Ts=1/(15000*2048) seconds.

expectedRSTD-Uncertainty If PRS is transmitted: This field indicates the uncertainty in expectedRSTD value. The uncertainty is related to the location server′s a-priori estimation of the target device location. The expectedRSTD and expectedRSTD-Uncertainty together define the search window for the target device. The scale factor of the expectedRSTD-Uncertainty field is 3Ts, with Ts=1/(15000*2048) seconds. The target device may assume that the beginning of the PRS occasion group of the PRS configuration with the longest PRS occasion group periodicity (NOTE) of the neighbour cell is received within the search window of size [ expectedRSTD-Uncertainty3Ts, expectedRSTD-Uncertainty3Ts] centred at TREF + 1 millisecondN + (expectedRSTD8192) 3Ts, where TREF is the reception time of the beginning of the first PRS occasion group of the first PRS configuration of the assistance data reference cell at the target device antenna connector, N = 0 when the EARFCN of the neighbour cell is equal to that of the assistance data reference cell, and N = prs-SubframeOffset otherwise. If PRS is not transmitted: This field indicates the uncertainty in expectedRSTD value. The uncertainty is related to the location server′s a-priori estimation of the target device location. The expectedRSTD and expectedRSTD-Uncertainty together define the search window for the target device. The scale factor of the expectedRSTD-Uncertainty field is 3Ts, with Ts=1/(15000*2048) seconds. If Tx is the reception time of the beginning of the subframe X of the assistance data reference cell at the target device antenna connector, the target device may assume that the beginning of the closest subframe of this neighbour cell to subframe X is received within the search window of size [ expectedRSTD-Uncertainty3Ts, expectedRSTD-Uncertainty3Ts] centred at Tx + (expectedRSTD8192) 3Ts,

tpId This field specifies an identity of the transmission point. This field together with the physCellId and/or prsID may be used to identify the transmission point in the case the same physical cell ID is shared by multiple transmission points.

prs-only-tp This field, if present, indicates that the OTDOA-NeighbourCellInfoElement is provided for a PRS-only TP. For the purpose of RSTD measurements from a PRS-only TP, the target device shall not assume any other signals or physical channels are present other than PRS (TS 36.213 [28]). For the purpose of RSTD measurements from a PRS-only TP, the target device shall use the physCellId only for PRS generation, and only if no PRS-ID is provided for this TP.

cpLengthCRS This field specifies the cyclic prefix length of this assistance data neighbour cell CRS. If this field is present, the target device may assume the CRS and PRS antenna ports of this assistance data neighbour cell are quasi co-located (as defined in TS 36.211 [16]).

sameMBSFNconfigNeighbour This field indicates whether the MBSFN subframe configuration of the neighbour cell is the same as the current primary cell of the target device. TRUE means the same, and FALSE means not the same.

dlBandwidth This field specifies the downlink bandwidth configuration of the neighbour cell, NRB in downlink, see TS 36.101 [21, table 5.6-1]. Enumerated value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on.

addPRSconfigNeighbour This field specifies the additional (second and possibly third) PRS configuration(s) of the neighbour cell. When the EARFCN of the neighbour cell is the same as for the assistance data reference cell, the target device may assume that each PRS positioning occasion in each instance of addPRSconfigNeighbour in the neighbour cell at least partially overlaps with a PRS positioning occasion of the same instance of addPRSconfigRef in the assistance data reference cell where the maximum offset between the transmitted PRS positioning occasions may be assumed to not exceed half a subframe. When the EARFCN of the neighbour cell is the same as for the assistance data reference cell, the target may assume that each instance of addPRSconfigNeighbour of this cell has the same PRS periodicity (TPRS) as the corresponding instance of addPRSconfigRef of the assistance data reference cell.

tdd-config This field specifies the TDD specific physical channel configuration of the neighbour cell earfcn. The field should be present if this neighbour cell is a TDD cell and if the TDD UL/DL configuration for assistance data cells on this earfcn has not been provided in any other instance of OTDOA-NeighbourCellInfoElement or in IE OTDOA-ReferenceCellInfo, and is not the same as the target device′s current primary cell when this is a TDD cell. NOTE 2.

NOTE 1:	If this cell has more than one PRS configuration with equal longest PRS occasion group periodicity (i.e., PRS occasion group length times TPRS), the first such configuration is referenced. In order to avoid ambiguity for frequency hopping, a PRS occasion group should contain at least 2 PRS occasions with hopping between 2 narrowbands and at least 4 PRS occasions with hopping between 4 narrowbands.

NOTE 2:	The target device assumes the same TDD UL/DL configuration for all TDD cells residing on same frequency band specified by earfcn. Therefore, the location server should include the field tdd-config only once for assistance data cells with the same earfcn in IE OTDOA-ProvideAssistanceData. The location server does not need to include the field tdd-config for any assistance data cell in IE OTDOA-ProvideAssistanceData with the same earfcn or the same TDD UL/DL configuration as the target devices′ current primary cell if this is a TDD cell.

#### –	OTDOA-ReferenceCellInfoNB

The IE OTDOA-ReferenceCellInfoNB is used by the location server to provide NB-IoT assistance data reference cell information for OTDOA assistance data.

-- ASN1START

OTDOA-ReferenceCellInfoNB-r14 ::= SEQUENCE {

physCellIdNB-r14				INTEGER (0..503)			OPTIONAL,	-- Cond NoPRS-AD1

cellGlobalIdNB-r14				ECGI						OPTIONAL,	-- Cond NoPRS-AD2

carrierFreqRef-r14				CarrierFreq-NB-r14			OPTIONAL,	-- Cond NotSameAsServ1

earfcn-r14						ARFCN-ValueEUTRA-r14		OPTIONAL,	-- Cond Inband

eutra-NumCRS-Ports-r14			ENUMERATED {ports1-or-2, ports4}

OPTIONAL,	-- Cond NoPRS-AD3

otdoa-SIB1-NB-repetitions-r14	ENUMERATED { r4, r8, r16 }	OPTIONAL,	-- Cond NotSameAsServ2

nprsInfo-r14					PRS-Info-NB-r14				OPTIONAL,	-- Cond NPRS-Type1

...,

[[

nprsInfo-Type2-v1470			PRS-Info-NB-r14				OPTIONAL	-- Cond NPRS-Type2

]],

[[	tdd-config-r15				TDD-Config-v1520			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NoPRS-AD1 | This field is mandatory present if the OTDOA-ReferenceCellInfo IE is not included in OTDOA-ProvideAssistanceData, or if the OTDOA-ReferenceCellInfo IE is included in OTDOA-ProvideAssistanceData and the narrowband physical layer cell identity is not the same as the physical cell identity provided in OTDOA-ReferenceCellInfo IE. Otherwise it is not present. |
| NoPRS-AD2 | This field is optionally present, need ON, if the OTDOA-ReferenceCellInfo IE is not included in OTDOA-ProvideAssistanceData, or if the OTDOA-ReferenceCellInfo IE is included in OTDOA-ProvideAssistanceData and the global cell identity is not the same as provided in OTDOA-ReferenceCellInfo IE. |
| NotSameAsServ1 | This field is mandatory present if the carrier frequency of the NB-IoT assistance data reference cell is not the same as the carrier frequency of the target devices' current serving NB-IoT cell. Otherwise it is not present. |
| Inband | This field is mandatory present, if the NPRS is configured within the LTE spectrum allocation (inband deployment). Otherwise it is not present. |
| NoPRS-AD3 | This field is mandatory present if the OTDOA-ReferenceCellInfo IE is not included in OTDOA-ProvideAssistanceData and if the NB-IoT assistance data reference cell is deployed within the LTE spectrum allocation (inband deployment). Otherwise it is not present. |
| NotSameAsServ2 | This field is mandatory present, if NPRS configuration Part B only is configured on the NB-IoT assistance data reference cell, and if the repetition number of SIB1-NB of the NB-IoT assistance data reference cell is not the same as the repetition number of SIB1-NB of the target devices′ current serving NB-IoT cell. Otherwise it is not present. |
| NPRS-Type1 | The field is mandatory present if Type 1 narrowband positioning reference signals are available in the assistance data reference cell (TS 36.211 [16]); otherwise it is not present. |
| NPRS-Type2 | The field is mandatory present if Type 2 narrowband positioning reference signals are available in the assistance data reference cell (TS 36.211 [16]); otherwise it is not present. |

OTDOA-ReferenceCellInfoNB field descriptions

physCellIdNB This field specifies the narrowband physical layer cell identity of the NB-IoT assistance data reference cell, as defined in TS 36.331 [12]. If this field is absent and if the OTDOA-ReferenceCellInfo IE is included in OTDOA-ProvideAssistanceData the narrowband physical layer cell identity is the same as the physCellId provided in OTDOA-ReferenceCellInfo IE.

cellGlobalIdNB This field specifies the global cell identity of the NB-IoT assistance data reference cell, as defined in TS 36.331 [12]. If this field is absent and if the OTDOA-ReferenceCellInfo IE with cellGlobalId is included in OTDOA-ProvideAssistanceData, the global cell identity is the same as provided in OTDOA-ReferenceCellInfo IE.

carrierFreqRef This field specifies the carrier frequency of the NB-IoT assistance data reference cell.

earfcn This field specifies the EARFCN of the E-UTRAN frequency, in which the NB-IoT cell is deployed.

eutra-NumCRS-Ports This field specifies whether 1 (or 2) antenna port(s) or 4 antenna ports for cell specific reference signals (CRS) are used in the NB-IoT assistance data reference cell. If this field is absent and if the OTDOA-ReferenceCellInfo IE is included in OTDOA-ProvideAssistanceData, the number of CRS antenna ports is the same as provided in OTDOA-ReferenceCellInfo IE.

otdoa-SIB1-NB-repetitions This field specifies the repetition number of SIB1-NB of the NB-IoT assistance data reference cell. Enumerated values r4 correspond to 4 repetions, r8 to 8 repetitions, and r16 to 16 repetions. Note, when NPRS configuration Part B only is configured on the NB-IoT assistance data reference cell (i.e., anchor carrier), nprs-NumSF does also count/include subframes containing NPSS, NSSS, NPBCH, or SIB1-NB, but the UE can assume that no NPRS are transmitted in these subframes (TS 36.211 [16]).

nprsInfo This field specifies the Type 1 NPRS (TS 36.211 [16]) configuration of the NB-IoT assistance data reference cell. When the target device receives this field with operationModeInfoNPRS set to value ′standalone′, the target device shall assume no NPRS are transmitted on that NB-IoT carrier.

nprsInfo-Type2 This field specifies the Type 2 NPRS (TS 36.211 [16]) configuration of the NB-IoT assistance data reference cell.

tdd-config Indicates the TDD specific physical channel configuration of the NB-IoT assistance data reference cell operating in TDD mode. This field should be present if the DL/UL subframe configuration of the NB-IoT assistance data reference cell is not the same as the DL/UL subframe configuration of the current serving NB-IoT cell of the target device, or if the current serving NB-IoT cell of the target device operates in FDD mode.

#### –	PRS-Info-NB

The IE PRS-Info-NB provides the information related to the configuration of NPRS in a cell. If PRS-Info-NB includes configurations for multiple NPRS carrier frequencies, the target device may assume the antenna ports for the NPRS carrier are quasi co-located, as defined in TS 36.211 [16].

-- ASN1START

PRS-Info-NB-r14 ::= SEQUENCE (SIZE (1..maxCarrier-r14)) OF NPRS-Info-r14

NPRS-Info-r14 ::= SEQUENCE {

operationModeInfoNPRS-r14	ENUMERATED { inband, standalone },

nprs-carrier-r14			CarrierFreq-NB-r14	OPTIONAL,	-- Cond Standalone/Guardband

nprsSequenceInfo-r14		INTEGER (0..174)	OPTIONAL,	-- Cond Inband

nprsID-r14					INTEGER (0..4095)	OPTIONAL,	-- Cond NPRS-ID

partA-r14					SEQUENCE {

nprsBitmap-r14			CHOICE {

subframePattern10-r14	BIT STRING (SIZE (10)),

subframePattern40-r14 	BIT STRING (SIZE (40))

},

nprs-MutingInfoA-r14	CHOICE {

po2-r14					BIT STRING (SIZE(2)),

po4-r14					BIT STRING (SIZE(4)),

po8-r14					BIT STRING (SIZE(8)),

po16-r14				BIT STRING (SIZE(16)),

...

}															OPTIONAL,		-- Cond MutingA

...

}																OPTIONAL,		-- Cond PartA

partB-r14					SEQUENCE {

nprs-Period-r14			ENUMERATED { ms160, ms320, ms640, ms1280, ... , ms2560-v1510},

nprs-startSF-r14		ENUMERATED { zero, one-eighth, two-eighths, three-eighths,

four-eighths, five-eighths, six-eighths,

seven-eighths, ...},

nprs-NumSF-r14			ENUMERATED { sf10, sf20, sf40, sf80, sf160, sf320,

sf640, sf1280, ... , sf2560-v1510},

nprs-MutingInfoB-r14	CHOICE {

po2-r14					BIT STRING (SIZE(2)),

po4-r14					BIT STRING (SIZE(4)),

po8-r14					BIT STRING (SIZE(8)),

po16-r14				BIT STRING (SIZE(16)),

...

}															OPTIONAL,		-- Cond MutingB

...,

[[	sib1-SF-TDD-r15			ENUMERATED {sf0, sf4, sf0and5}		OPTIONAL		-- Cond SIB1-TDD

]]

}																OPTIONAL,		-- Cond PartB

...,

[[

partA-TDD-r15					SEQUENCE {

nprsBitmap-r15			CHOICE {

subframePattern10-TDD-r15	BIT STRING (SIZE (8)),

subframePattern40-TDD-r15 	BIT STRING (SIZE (32)),

...

},

nprs-MutingInfoA-r15	CHOICE {

po2-r15					BIT STRING (SIZE(2)),

po4-r15					BIT STRING (SIZE(4)),

po8-r15					BIT STRING (SIZE(8)),

po16-r15				BIT STRING (SIZE(16)),

...

}															OPTIONAL,	-- Cond MutingA

...

}																OPTIONAL	-- Cond PartA-TDD

]]

}

maxCarrier-r14	INTEGER ::= 5

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Standalone/Guardband | This field is mandatory present, if the NPRS is configured in standalone or guardband operation mode. Otherwise it is not present. |
| Inband | This field is mandatory present, if the NPRS is configured within the LTE spectrum allocation (inband deployment) and the LTE carrier frequency is not provided in the assistance data. Otherwise it is not present. |
| NPRS-ID | The field is mandatory present, if the NPRS is generated based on the NPRS-ID (TS 36.211 [16]), different from the PCI. Otherwise the field is not present. |
| MutingA | The field is mandatory present, if muting is used for the NPRS Part A or Part A TDD configuration. Otherwise the field is not present. |
| PartA | The field is mandatory present, if NPRS is configured based on a bitmap of subframes which are not NB-IoT DL subframes (i.e., invalid DL subframes) (Part A configuration). Otherwise the field is not present. This field is not applicable for NB-IoT operating in TDD mode. |
| MutingB | The field is mandatory present, if muting is used for the NPRS Part B configuration. Otherwise the field is not present. |
| PartB | The field is mandatory present, if NPRS is configured based on a NPRS period, a NPRS subframe offset, and a number of consecutive NPRS downlink subframes per positioning occasion (Part B configuration). Otherwise the field is not present. If NPRS configuration Part A and Part B are both configured, then a subframe contains NPRS if both configurations indicate that it contains NPRS. |
| PartA-TDD | The field is mandatory present, if NPRS is configured for NB-IoT operating in TDD mode and if NPRS is configured based on a bitmap of subframes which are not NB-IoT DL subframes (i.e., invalid DL subframes) (Part A TDD configuration). Otherwise the field is not present. |
| SIB1-TDD | The field is mandatory present, if NPRS is configured for NB-IoT operating in TDD mode and if SIB1-NB is transmitted on this carrier frequency. Otherwise the field is not present. |

PRS-Info-NB field descriptions

operationModeInfoNPRS This field specifies the operation mode of the NPRS carrier. The value ′standalone′ indicates standalone or guardband operation mode.

nprs-carrier This field specifies the NB-IoT carrier frequency for the NPRS.

nprsSequenceInfo This field specifies the index of the PRB containing the NPRS as defined in the table nprsSequenceInfo to E-UTRA PRB index relation below.

nprsID This field specifies the NPRS-ID as defined in TS 36.211 [16].

sib1-SF-TDD This field indicates the subframe(s) used to transmit SIB1-NB. Values sf0 and sf4 correspond with subframe #0 and #4 respectively. Value sf0and5 corresponds with subframes #0 and #5.

subframePattern10, subframePattern40 This field specifies the NPRS subframe Part A configuration over 10 ms or 40 ms. Subframes not containing NPRS are indicated with value ′0′ in the bitmap; subframes containing NPRS are indicated with value ′1′ in the bitmap. The first/leftmost bit corresponds to the subframe #0 of the radio frame satisfying SFN mod x = 0, where x is the size of the bit string divided by 10.

nprs-MutingInfoA This field specifies the NPRS muting configuration of the NB-IoT carrier Part A configuration. The NPRS muting configuration is defined by a periodic NPRS muting sequence with periodicity TREP where TREP, counted in the number of NPRS positioning occasions, can be 2, 4, 8, or 16 which is also the length of the selected bit string that represents this NPRS muting sequence. If a bit in the NPRS muting sequence is set to ′0′, then the NPRS is muted in the corresponding NPRS positioning occasion. A NPRS positioning occasion for Part A comprises one radio frame (i.e., 10 subframes). The first/leftmost bit of the NPRS muting sequence corresponds to the first NPRS positioning occasion that starts after the beginning of the NB-IoT assistance data reference cell SFN=0. The sequence is valid for all subframes after the target device has received the nprs-MutingInfoA. When the SFN of the NB-IoT assistance data reference cell is not known to the target device and nprs-MutingInfoA is provided for a cell in the OTDOA-NeighbourCellInfoListNB IE, the target device may assume no NPRS is transmitted by that cell.

nprs-Period This field specifies the NPRS occasion period TNPRS (TS 36.211 [16]). Enumerated values correspond to 160 ms, 320 ms, 640 ms, 1280 ms, and 2560 ms. The value ms2560 is only applicable to TDD mode.

nprs-startSF This field specifies the subframe offset (TS 36.211 [16]). Enumerated values correspond to  of 0, 1/8, 2/8, 3/8, 4/8, 5/8, 6/8, or 7/8.

nprs-NumSF This field specifies the number of consecutive downlink subframes NNPRS in one NPRS positioning occasion (TS 36.211 [16]). Enumerated values correspond to 10, 20, 40, 80, 160, 320, 640, 1280, and 2560 subframes. The values sf10 and sf20 are only applicable to FDD mode. The value sf2560 is only applicable to TDD mode. When the target device receives a nprs-NumSF which exceeds the nprs-Period (i.e., NNPRS > TNPRS), the target device may assume no NPRS is transmitted by that cell.

nprs-MutingInfoB This field specifies the NPRS muting configuration of the NB-IoT carrier Part B configuration. The NPRS muting configuration is defined by a periodic NPRS muting sequence with periodicity TREP where TREP, counted in the number of NPRS positioning occasions, can be 2, 4, 8, or 16 which is also the length of the selected bit string that represents this NPRS muting sequence. If a bit in the NPRS muting sequence is set to ′0′, then the NPRS is muted in the corresponding NPRS positioning occasion. A NPRS positioning occasion for Part B comprises NNPRS consecutive downlink positioning subframes, where NNPRS is given by the nprs-NumSF field. The first/leftmost bit of the NPRS muting sequence corresponds to the first NPRS positioning occasion that starts after the beginning of the NB-IoT assistance data reference cell SFN=0. The sequence is valid for all subframes after the target device has received the nprs-MutingInfoB. When the SFN of the NB-IoT assistance data reference cell is not known to the UE and nprs-MutingInfoB is provided for a cell in the OTDOA-NeighbourCellInfoListNB IE, the target device may assume no NPRS is transmitted by that cell. When the UE receives a TREP-bit muting pattern together with a NPRS periodicity TNPRS for the same carrier which exceeds 10240 subframes (i.e., TREP × TNPRS > 10240 subframes), the target device shall assume an n-bit muting pattern based on the first n bits, where n = 10240/TNPRS.

subframePattern10-TDD, subframePattern40-TDD This field specifies the NPRS subframe Part A configuration for TDD over 10 ms or 40 ms. The UE shall assume that subframe number 1 and 2 are not used for NPRS. The MSB of the NPRS bitmap corresponds to subframe 0, the second MSB corresponds to subframe 3, the third MSB corresponds to subframe 4 and so on, as also shown in Figure NPRS bitmap to subframe number mapping below.

nprsSequenceInfo to E-UTRA PRB index relation

| nprsSequenceInfo | E-UTRA PRB index for odd number of [16] | nprsSequenceInfo | E-UTRA PRB index for even number of [16] |
| --- | --- | --- | --- |
| 0 - 74 | -37, -36, …, 37 | 75 – 174 | -50, -49, …, 49 |

NOTE:	Based on the above relation, in inband deployment, the carrier frequency of the NPRS carrier (fNB-IoT) can be calculated as follows:

fNB-IoT=fEUTRA + 7.5 + 180∙nPRB'                   if nprsSequenceInfo≤74 and nPRB'>0fEUTRA - 7.5 + 180∙nPRB'                    if nprsSequenceInfo≤74 and nPRB'<0fEUTRA + 180∙nPRB'                            if nprsSequenceInfo≤74 and nPRB'=0fEUTRA + 97.5 + 180∙nPRB'                  if nprsSequenceInfo≥75 and nPRB'≥0fEUTRA - 97.5 + 180∙nPRB' + 1           if nprsSequenceInfo≥75 and nPRB'<0

where fEUTRA is derived from earfcn according to TS 36.101 [21, 5.7.3].

Figure 6.5.1.2-1: NPRS bitmap to subframe number mapping

#### –	OTDOA-NeighbourCellInfoListNB

The IE OTDOA-NeighbourCellInfoListNB is used by the location server to provide NB-IoT neighbour cell information for OTDOA assistance data.

-- ASN1START

OTDOA-NeighbourCellInfoListNB-r14 ::= SEQUENCE (SIZE (1..maxCells-r14)) OF

OTDOA-NeighbourCellInfoNB-r14

OTDOA-NeighbourCellInfoNB-r14 ::= SEQUENCE {

physCellIdNB-r14				INTEGER (0..503)		OPTIONAL,		-- Cond NoPRS-AD1

cellGlobalIdNB-r14				ECGI					OPTIONAL,		-- Cond NoPRS-AD2

carrierFreq-r14					CarrierFreq-NB-r14		OPTIONAL,		-- Cond NotSameAsRef1

earfcn-r14						ARFCN-ValueEUTRA-r14	OPTIONAL,		-- Cond Inband

eutra-NumCRS-Ports-r14			ENUMERATED {ports-1-or-2, ports-4, ...}

OPTIONAL, 		-- Cond NotsameAsRef2

otdoa-SIB1-NB-repetitions-r14	ENUMERATED { r4, r8, r16 }

OPTIONAL,		-- Cond	NotSameAsRef3

nprsInfo-r14					PRS-Info-NB-r14			OPTIONAL,		-- Cond NotsameAsRef4

nprs-slotNumberOffset-r14		INTEGER (0..19)			OPTIONAL,		-- Cond NotsameAsRef5

nprs-SFN-Offset-r14				INTEGER (0..63)			OPTIONAL,		-- Cond NotsameAsRef6

nprs-SubframeOffset-r14			INTEGER (0..1279)		OPTIONAL,		-- Need OP

expectedRSTD-r14				INTEGER (0..16383)		OPTIONAL,		-- Cond NoPRS-AD3

expectedRSTD-Uncertainty-r14	INTEGER (0..1023)		OPTIONAL,		-- Cond NoPRS-AD3

prsNeighbourCellIndex-r14		INTEGER (1..72)			OPTIONAL,		-- Cond PRS-AD

...,

[[

nprsInfo-Type2-v1470			PRS-Info-NB-r14			OPTIONAL		-- Cond NotSameAsRef4

]],

[[	tdd-config-r15				TDD-Config-v1520		OPTIONAL		-- Need ON

]]

}

maxCells-r14	INTEGER ::= 72

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NoPRS-AD1 | This field is mandatory present if the OTDOA-NeighbourCellInfoList IE is not included in OTDOA-ProvideAssistanceData, or if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData and the narrowband physical layer cell identity of this cell is not the same as the physical cell identity of the corresponding cell (as indicated by prsNeighbourCellIndex) in OTDOA-NeighbourCellInfoList IE. |
| NoPRS-AD2 | This field is optionally present, need ON, if the OTDOA-NeighbourCellInfoList IE is not included in OTDOA-ProvideAssistanceData, or if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData and the global cell identity of this cell is not the same as for the corresponding cell (as indicated by prsNeighbourCellIndex) in OTDOA-NeighbourCellInfoList IE. |
| Inband | This field is mandatory present, if the NPRS is configured within the LTE spectrum allocation (inband deployment). Otherwise it is not present. |
| NotSameAsRef1 | The field is mandatory present if the carrier frequency is not the same as for the NB-IoT assistance data reference cell; otherwise it is not present. |
| NotSameAsRef2 | The field is mandatory present if this cell is deployed within the LTE spectrum allocation (inband deployment) and if the number of E-UTRA CRS antenna ports is not the same as for the NB-IoT assistance data reference cell; otherwise it is not present. |
| NotSameAsRef3 | This field is mandatory present if NPRS configuration Part B only is configured on this neighbour cell, and if the repetition number of SIB1-NB of this neighbor cell is not the same as the repetition number of SIB1-NB of the NB-IoT assistance data reference cell. Otherwise it is not present. |
| NotSameAsRef4 | The field is mandatory present, if the NPRS configuration is not the same as for the NB-IoT assistance data reference cell; otherwise it is not present. |
| NotSameAsRef5 | The field is mandatory present if the slot timing is not the same as for the NB-IoT assistance data reference cell; otherwise it is not present. |
| NotSameAsRef6 | The field is mandatory present if the frame timing is not the same as for the NB-IoT assistance data reference cell; otherwise it is not present. |
| NoPRS-AD3 | This field is mandatory present if the OTDOA-NeighbourCellInfoList IE is not included in OTDOA-ProvideAssistanceData, or if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData and prsNeighbourCellIndex is absent for this cell. |
| PRS-AD | This field is optionally present, need OP, if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData; otherwise it is not present. |

OTDOA-NeighbourCellInfoListNB field descriptions

physCellIdNB This field specifies the narrowband physical cell identity of the NB-IoT neighbour cell, as defined in TS 36.331 [12]. If this field is absent and if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData the narrowband physical layer cell identity is the same as the physCellId provided for the corresponding cell (as indicated by prsNeighbourCellIndex) in OTDOA-NeighbourCellInfoList IE.

cellGlobalIdNB This field specifies the global cell ID of the NB-IoT neighbour cell, as defined in TS 36.331 [12]. If this field is absent and if the OTDOA-NeighbourCellInfoList IE with cellGlobalId is included in OTDOA-ProvideAssistanceData, the global cell identity of the NB-IoT neighbour cell is the same as provided for the corresponding cell (as indicated by prsNeighbourCellIndex) in OTDOA-NeighbourCellInfoList IE.

carrierFreq This field specifies the carrier frequency of the NB-IoT neighbour cell.

earfcn This field specifies the EARFCN of the E-UTRAN frequency, in which the NB-IoT cell is deployed.

eutra-NumCRS-Ports This field specifies whether 1 (or 2) antenna port(s) or 4 antenna ports for cell specific reference signals are used.

otdoa-SIB1-NB-repetitions This field specifies the repetition number of SIB1-NB of the neighbour cell. Enumerated values r4 correspond to 4 repetions, r8 to 8 repetitions, and r16 to 16 repetions. Note, when NPRS configuration Part B only is configured on this NB-IoT neighbour cell (i.e., anchor carrier), nprs-NumSF does also count/include subframes containing NPSS, NSSS, NPBCH, or SIB1-NB, but the UE can assume that no NPRS are transmitted in these subframes (TS 36.211 [16]).

nprsInfo This field specifies the Type 1 NPRS (TS 36.211 [16]) configuration of the NB-IoT neighbour cell. When the carrier frequency of the NB-IoT neighbour cell is the same as for the NB-IoT assistance data reference cell, the target device may assume that each NPRS positioning occasion for each NPRS carrier frequency in the neighbour cell at least partially overlaps with a NPRS positioning occasion for each NPRS carrier frequency in the NB-IoT assistance data reference cell where the maximum offset between the transmitted NPRS positioning occasions may be assumed to not exceed half a subframe. When the carrier frequency of the neighbour cell is the same as for the NB-IoT assistance data reference cell, and NPRS configuration Part B is configured, the target may assume that this cell has the same NPRS periodicity (TNPRS) as the assistance data reference cell for each NPRS carrier frequency. When the target device receives this field with operationModeInfoNPRS set to value ′standalone′, the target device shall assume no NPRS are transmitted on that NB-IoT carrier.

nprs-slotNumberOffset This field specifies the slot number offset at the transmitter between this cell and the NB-IoT assistance data reference cell. The offset corresponds to the number of full slots counted from the beginning of a radio frame of the NB-IoT assistance data reference cell to the beginning of the closest subsequent radio frame of this cell. If this field is absent, the slot timing is the same as for the NB-IoT assistance data reference cell.

nprs-SFN-Offset This field specifies the SFN offset (modulo 64) at the transmitter between this cell and the NB-IoT assistance data reference cell. The offset corresponds to the number of full radio frames counted from the beginning of a radio frame #0 of the NB-IoT assistance data reference cell to the beginning of the closest subsequent radio frame #0 of this cell. The UE may use this field together with the nprs-slotNumberOffset and otdoa-SIB1-NB-repetitions to determine the SIB1-NB subframes of this neighbour cell.

nprs-SubframeOffset This field specifies the offset between the first NPRS subframe in the NB-IoT assistance data reference cell (NOTE 1) and the first NPRS subframe in the closest subsequent NPRS positioning occasion of the NPRS carrier with the longest NPRS periodicity of this cell (NOTE 2). The value is given in number of full sub-frames. If this field is not present, the receiver shall consider the NPRS subframe offset to be 0.

expectedRSTD This field indicates the RSTD value that the target device is expected to measure between this cell and the NB-IoT assistance data reference cell. The expectedRSTD field takes into account the expected propagation time difference as well as transmit time difference of NPRS positioning occasions between the two cells. The RSTD value can be negative and is calculated as (expectedRSTD-8192). The resolution is 3Ts, with Ts=1/(15000*2048) seconds. If this field is absent and if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData, the expected RSTD is the same as provided in OTDOA-NeighbourCellInfoList IE for the corresponding cell (as indicated by prsNeighbourCellIndex).

expectedRSTD-Uncertainty This field indicates the uncertainty in expectedRSTD value. The uncertainty is related to the location server′s a-priori estimation of the target device location. The expectedRSTD and expectedRSTD-Uncertainty together define the search window for the target device. The scale factor of the expectedRSTD-Uncertainty field is 3Ts, with Ts=1/(15000*2048) seconds. If this field is absent and if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData, the expected RSTD uncertainty is the same as provided in OTDOA-NeighbourCellInfoList IE for the corresponding cell (as indicated by prsNeighbourCellIndex). The target device may assume that the beginning of the NPRS positioning occasion of the NPRS carrier with the longest NPRS periodicity of the neighbour cell (NOTE 2) is received within the search window of size [expectedRSTD-Uncertainty3Ts, expectedRSTD-Uncertainty3Ts] centred at TREF + 1 millisecondN + (expectedRSTD8192) 3Ts, where TREF is the reception time of the beginning of the NPRS positioning occasion of the NB-IoT assistance data reference cell (NOTE 1) at the target device antenna connector, and N = nprs-SubframeOffset.

prsNeighbourCellIndex This field contains an index of the entry in IE OTDOA-NeighbourCellInfoList. Value 1 corresponds to the first cell in OTDOA-NeighbourCellInfoList, value 2 to the second, and so on. If this field is absent, and if the OTDOA-NeighbourCellInfoList IE is included in OTDOA-ProvideAssistanceData, it means there is no corresponding cell in OTDOA-NeighbourCellInfoList IE for this cell. The target device may assume the antenna ports of the PRS of the cell indicated by prsNeighbourCellIndex and the NPRS of this cell are quasi co-located, as defined in TS 36.211 [16].

nprsInfo-Type2 This field specifies the Type 2 NPRS (TS 36.211 [16]) configuration of the NB-IoT neighbour cell.

tdd-config Indicates the TDD specific physical channel configuration of the NB-IoT assistance data neighbour cell operating in TDD mode. This field should be present if the DL/UL subframe configuration of the NB-IoT assistance data neighbour cell is not the same as for the NB-IoT assistance data reference cell, or if the current serving NB-IoT cell of the target device operates in FDD mode.

NOTE 1:	If the NB-IoT assistance data reference cell (i.e., anchor carrier) has no NPRS configured, the first NPRS carrier in PRS-Info-NB is referenced.

NOTE 2:	"Cell" in this context may not necessarily be the anchor carrier. If this "cell" has more than one NPRS carrier with equal longest periodicity, the first such NPRS carrier in PRS-Info-NB is referenced. The length of a NPRS positioning occasion for Part A in this context is the length of the nprsBitmap bit string.

#### 6.5.1.3	OTDOA Assistance Data Request

#### –	OTDOA-RequestAssistanceData

The IE OTDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

OTDOA-RequestAssistanceData ::= SEQUENCE {

physCellId			INTEGER (0..503),

...,

[[

adType-r14			BIT STRING { prs (0), nprs (1) } (SIZE (1..8))		OPTIONAL

]],

[[

nrPhysCellId-r15	INTEGER (0..1007)									OPTIONAL

]]

}

-- ASN1STOP

OTDOA-RequestAssistanceData field descriptions

physCellId This field specifies the E-UTRA physical cell identity of the current primary cell of the target device.

adType This field specifies the assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. Bit 0 indicates that PRS assistance data are requested, bit 1 indicates that NPRS assistance data are requested.

nrPhysCellId This field specifies the NR physical cell identity of the current primary cell of the target device. If this field is present, the target device sets the physCellId to an arbitrary value which shall be ignored by the location server.

#### 6.5.1.4	OTDOA Location Information

#### –	OTDOA-ProvideLocationInformation

The IE OTDOA-ProvideLocationInformation is used by the target device to provide OTDOA location measurements to the location server. It may also be used to provide OTDOA positioning specific error reason.

-- ASN1START

OTDOA-ProvideLocationInformation ::= SEQUENCE {

otdoaSignalMeasurementInformation	OTDOA-SignalMeasurementInformation	OPTIONAL,

otdoa-Error							OTDOA-Error							OPTIONAL,

...,

[[

otdoaSignalMeasurementInformation-NB-r14	OTDOA-SignalMeasurementInformation-NB-r14

OPTIONAL

]]

}

-- ASN1STOP

#### 6.5.1.5	OTDOA Location Information Elements

#### –	OTDOA-SignalMeasurementInformation

The IE OTDOA-SignalMeasurementInformation is used by the target device to provide RSTD measurements to the location server. The RSTD measurements are provided for a neighbour cell and the RSTD reference cell, both of which are provided in the IE OTDOA-ProvideAssistanceData. The RSTD reference cell may or may not be the same as the assistance data reference cell provided in OTDOA-ReferenceCellInfo or OTDOA-ReferenceCellInfoNB. If the target device stops reporting inter-frequency RSTD measurements, where the inter-frequency RSTD measurement is an OTDOA RSTD measurement with at least one cell on a frequency different from the serving cell frequency, the LPP layer shall inform lower layers that inter-frequency RSTD measurements are stopped.

NOTE 1:	If there are more than 24 NeighbourMeasurementElement to be sent, the target device may send them in multiple ProvideLocationInformation messages, as described under clause 5.3.

NOTE 2:	If NPRS/PRS antenna ports are quasi co-located, the target device provides a single RSTD measurement for the quasi co-located antenna ports of NPRS/PRS.

-- ASN1START

OTDOA-SignalMeasurementInformation ::= SEQUENCE {

systemFrameNumber		BIT STRING (SIZE (10)),

physCellIdRef			INTEGER (0..503),

cellGlobalIdRef			ECGI					OPTIONAL,

earfcnRef				ARFCN-ValueEUTRA		OPTIONAL,		-- Cond NotSameAsRef0

referenceQuality		OTDOA-MeasQuality		OPTIONAL,

neighbourMeasurementList	NeighbourMeasurementList,

...,

[[ earfcnRef-v9a0		ARFCN-ValueEUTRA-v9a0	OPTIONAL		-- Cond NotSameAsRef1

]],

[[ tpIdRef-r14			INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer0

prsIdRef-r14		INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer1

additionalPathsRef-r14

AdditionalPathList-r14	OPTIONAL,

nprsIdRef-r14		INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer2

carrierFreqOffsetNB-Ref-r14

CarrierFreqOffsetNB-r14	OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14		BIT STRING (SIZE (10))	OPTIONAL		-- Cond H-SFN

]],

[[

motionTimeSource-r15		MotionTimeSource-r15		OPTIONAL

]]

}

NeighbourMeasurementList ::= SEQUENCE (SIZE(1..24)) OF NeighbourMeasurementElement

NeighbourMeasurementElement ::= SEQUENCE {

physCellIdNeighbour		INTEGER (0..503),

cellGlobalIdNeighbour	ECGI					OPTIONAL,

earfcnNeighbour			ARFCN-ValueEUTRA		OPTIONAL,		-- Cond NotSameAsRef2

rstd					INTEGER (0..12711),

rstd-Quality			OTDOA-MeasQuality,

...,

[[ earfcnNeighbour-v9a0	ARFCN-ValueEUTRA-v9a0	OPTIONAL		-- Cond NotSameAsRef3

]],

[[ tpIdNeighbour-r14	INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer0

prsIdNeighbour-r14	INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer1

delta-rstd-r14		INTEGER (0..5)			OPTIONAL,

additionalPathsNeighbour-r14

AdditionalPathList-r14	OPTIONAL,

nprsIdNeighbour-r14	INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer2

carrierFreqOffsetNB-Neighbour-r14

CarrierFreqOffsetNB-r14	OPTIONAL		-- Cond NB-IoT

]],

[[

delta-SFN-r15			INTEGER (-8192..8191)	OPTIONAL

]]

}

AdditionalPathList-r14 ::= SEQUENCE (SIZE(1..maxPaths-r14)) OF AdditionalPath-r14

maxPaths-r14	INTEGER ::= 2

MotionTimeSource-r15 ::= SEQUENCE {

timeSource-r15				ENUMERATED {servingCell, referenceCell, gnss, mixed,

other, none, ...}

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsRef0 | The field is absent if the corresponding earfcnRef-v9a0 is present. Otherwise, the target device shall include this field if the EARFCN of the RSTD reference cell is not the same as the EARFCN of the assistance data reference cell provided in the OTDOA assistance data. |
| NotSameAsRef1 | The field is absent if the corresponding earfcnRef is present. Otherwise, the target device shall include this field if the EARFCN of the RSTD reference cell is not the same as the EARFCN of the assistance data reference cell provided in the OTDOA assistance data. |
| NotSameAsRef2 | The field is absent if the corresponding earfcnNeighbour-v9a0 is present. Otherwise, the target device shall include this field if the EARFCN of this neighbour cell is not the same as the earfcnRef for the RSTD reference cell. |
| NotSameAsRef3 | The field is absent if the corresponding earfcnNeighbour is present. Otherwise, the target device shall include this field if the EARFCN of this neighbour cell is not the same as the earfcnRef for the RSTD reference cell. |
| ProvidedByServer0 | The target device shall include this field if a tpId for this transmission point is included in the OTDOA-ProvideAssistanceData. Otherwise the field is absent. |
| ProvidedByServer1 | The target device shall include this field if a prsID for this transmission point is included in the OTDOA-ProvideAssistanceData. Otherwise the field is absent. |
| ProvidedByServer2 | The target device shall include this field if an nprsID for this cell is included in the OTDOA-ProvideAssistanceData and if this cell is a NB-IoT only cell (without associated LTE PRS cell). Otherwise the field is absent. |
| NB-IoT | The target device shall include this field if the cell is a NB-IoT only cell (without associated LTE PRS cell). Otherwise the field is absent. |
| H-SFN | The target device shall include this field if it was able to determine a hyper SFN of the RSTD reference cell. |

OTDOA-SignalMeasurementInformation field descriptions

systemFrameNumber If the deltaSFN and motionTimeSource fields are not present, this field specifies the SFN of the RSTD reference cell containing the starting subframe of the PRS or NPRS positioning occasion if PRS or NPRS are available on the RSTD reference cell, or subframe of the CRS for RSTD measurements if PRS and NPRS are not available on the RSTD reference cell during which the most recent neighbour cell RSTD measurement was performed. In the case of more than a single PRS configuration on the RSTD reference cell, the first PRS configuration is referenced. If the deltaSFN and motionTimeSource fields are present, this field specifies the SFN of the RSTD reference cell when the TOA measurement for the RSTD reference cell has been made.

physCellIdRef This field specifies the physical cell identity of the RSTD reference cell.

cellGlobalIdRef This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the RSTD reference cell. The target shall provide this IE if it knows the ECGI of the RSTD reference cell.

earfcnRef This field specifies the EARFCN of the RSTD reference cell.

referenceQuality This field specifies the target device′s best estimate of the quality of the TOA measurement from the RSTD reference cell, TSubframeRxRef, where TSubframeRxRef is the time of arrival of the signal from the RSTD reference cell. When deltaSFN and motionTimeSource are both included, the target device shall not include measurement errors caused by motion of the target device in referenceQuality (e.g. the target device may assume the target device was stationary during OTDOA measurements).

neighbourMeasurementList This list contains the measured RSTD values for neighbour cells together with the RSTD reference cell, along with quality for each measurement.

tpIdRef This field specifies the transmission point ID of the RSTD reference cell.

prsIdRef This field specifies the PRS-ID of the first PRS configuration of the RSTD reference cell.

additionalPathsRef This field specifies one or more additional detected path timing values for the RSTD reference cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nprsIdRef This field specifies the NPRS-ID of the RSTD reference cell.

carrierFreqOffsetNB-Ref This field specifies the offset of the NB-IoT channel number to EARFCN given by earfcnRef as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper SFN as defined in TS 36.331 [12] of the RSTD reference cell for the systemFrameNumber.

motionTimeSource This field provides reference information concerning the movement of the target device and comprises the following subfields: - timeSource specifies the external time source to which UE time was locked during the OTDOA measurements. Enumerated value "mixed" indicates that UE time was locked to more than one external time source during OTDOA measurements (e.g. is applicable to a change in serving cell when the serving cell was used as a time source). The value "other" indicates some other external time source. The value "none" indicates that UE time was not locked to an external time source. If this field is present, the target device shall also provide the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

physCellIdNeighbour This field specifies the physical cell identity of the neighbour cell for which the RSTDs are provided.

cellGlobalIdNeighbour This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the neighbour cell for which the RSTDs are provided. The target device shall provide this IE if it was able to determine the ECGI of the neighbour cell at the time of measurement.

earfcnNeighbour This field specifies the EARFCN of the neighbour cell used for the RSTD measurements.

rstd This field specifies the relative timing difference between this neighbour cell and the RSTD reference cell, as defined in TS 36.214 [17]. Mapping of the measured quantity is defined as in TS 36.133 [18] clause 9.1.10.3.

rstd-Quality This field specifies the target device′s best estimate of the quality of the measured rstd. When deltaSFN and motionTimeSource both included, the target device shall not include measurement errors caused by motion of the target device in rstd-Quality (e.g. the target device may assume the target device was stationary during OTDOA measurements).

tpIdNeighbour This field specifies the transmission point ID for the neighbour cell for which the RSTDs are provided.

prsIdNeighbour This field specifies the PRS-ID of the first PRS configuration of the neighbour cell for which the RSTDs are provided.

delta-rstd This field specifies the higher-resolution RSTD RSTD as defined in TS 36.133 [18] clause 9.1.10.4. Mapping of the measured quantity is defined as in TS 36.133 [18] clause 9.1.10.4.

additionalPathsNeighbour This field specifies one or more additional detected path timing values for the neighbour cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nprsIdNeighbour This field specifies the NPRS-ID of the neighbour cell for which the RSTDs are provided.

carrierFreqOffsetNB-Neighbour This field specifies the offset of the NB-IoT channel number to EARFCN given by earfcnNeighbour as defined in TS 36.101 [21].

delta-SFN This field provides information concerning the movement of the target device: Together with systemFrameNumber specifies the measurementSFN of the RSTD reference cell when the TOA measurement for this neighbour cell has been made for determining the rstd. The measurementSFN is given by systemFrameNumber + delta-SFN. (The actual SFN is the measurementSFN modulo 1024.). The measurementSFN is used in IE Sensor-MotionInformation to provide movement information corresponding to the TOA measurement time. If this field is present, the target device shall also provide the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

#### –	OTDOA-SignalMeasurementInformation-NB

The IE OTDOA-SignalMeasurementInformation-NB is used by the target device to provide RSTD measurements to the location server. The RSTD measurements are provided for a neighbour cell and the RSTD reference cell, both of which are provided in the IE OTDOA-ProvideAssistanceData. The RSTD reference cell may or may not be the same as the assistance data reference cell provided in OTDOA-ReferenceCellInfo or OTDOA-ReferenceCellInfoNB. If the target device stops reporting inter-frequency RSTD measurements, where the inter-frequency RSTD measurement is an OTDOA RSTD measurement with at least one cell on a frequency different from the serving cell frequency, the LPP layer shall inform lower layers that inter-frequency RSTD measurements are stopped.

NOTE 1:	If there are more than 24 NeighbourMeasurementElement-NB to be sent, the target device may send them in multiple ProvideLocationInformation messages, as described under clause 5.3.

NOTE 2:	If NPRS/PRS antenna ports are quasi co-located, the target device provides a single RSTD measurement for the quasi co-located antenna ports of NPRS/PRS.

-- ASN1START

OTDOA-SignalMeasurementInformation-NB-r14 ::= SEQUENCE {

systemFrameNumber-r14			BIT STRING (SIZE (10)),

physCellIdRef-r14				INTEGER (0..503),

cellGlobalIdRef-r14				ECGI					OPTIONAL,

earfcnRef-r14					ARFCN-ValueEUTRA-r14	OPTIONAL,	-- Cond NotSameAsRef0

referenceQuality-r14			OTDOA-MeasQuality		OPTIONAL,

neighbourMeasurementList-r14	NeighbourMeasurementList-NB-r14,

tpIdRef-r14						INTEGER (0..4095)		OPTIONAL,	-- Cond ProvidedByServer0

prsIdRef-r14					INTEGER (0..4095)		OPTIONAL,	-- Cond ProvidedByServer1

additionalPathsRef-r14			AdditionalPathList-r14	OPTIONAL,

nprsIdRef-r14					INTEGER (0..4095)		OPTIONAL,	-- Cond ProvidedByServer2

carrierFreqOffsetNB-Ref-r14		CarrierFreqOffsetNB-r14	OPTIONAL,	-- Cond NB-IoT

hyperSFN-r14					BIT STRING (SIZE (10))	OPTIONAL,	-- Cond H-SFN

...

}

NeighbourMeasurementList-NB-r14 ::= SEQUENCE (SIZE(1..24)) OF NeighbourMeasurementElement-NB-r14

NeighbourMeasurementElement-NB-r14 ::= SEQUENCE {

physCellIdNeighbour-r14		INTEGER (0..503),

cellGlobalIdNeighbour-r14	ECGI					OPTIONAL,

earfcnNeighbour-r14			ARFCN-ValueEUTRA-r14	OPTIONAL,		-- Cond NotSameAsRef2

rstd-r14					INTEGER (0..12711),

rstd-Quality-r14			OTDOA-MeasQuality,

tpIdNeighbour-r14			INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer0

prsIdNeighbour-r14			INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer1

delta-rstd-r14				INTEGER (0..5)			OPTIONAL,

additionalPathsNeighbour-r14

AdditionalPathList-r14	OPTIONAL,

nprsIdNeighbour-r14			INTEGER (0..4095)		OPTIONAL,		-- Cond ProvidedByServer2

carrierFreqOffsetNB-Neighbour-r14

CarrierFreqOffsetNB-r14	OPTIONAL,		-- Cond NB-IoT

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameAsRef0 | The target device shall include this field if the EARFCN of the RSTD reference cell is not the same as the EARFCN of the assistance data reference cell provided in the OTDOA assistance data. |
| NotSameAsRef2 | The target device shall include this field if the EARFCN of this neighbour cell is not the same as the earfcnRef for the RSTD reference cell. |
| ProvidedByServer0 | The target device shall include this field if a tpId for this transmission point is included in the OTDOA-ProvideAssistanceData. Otherwise the field is absent. |
| ProvidedByServer1 | The target device shall include this field if a prsID for this transmission point is included in the OTDOA-ProvideAssistanceData. Otherwise the field is absent. |
| ProvidedByServer2 | The target device shall include this field if an nprsID for this cell is included in the OTDOA-ProvideAssistanceData and if this cell is a NB-IoT only cell (without associated LTE PRS cell). Otherwise the field is absent. |
| NB-IoT | The target device shall include this field if the cell is a NB-IoT only cell (without associated LTE PRS cell). Otherwise the field is absent. |
| H-SFN | The target device shall include this field if it was able to determine a hyper SFN of the RSTD reference cell. |

OTDOA-SignalMeasurementInformation-NB field descriptions

systemFrameNumber This field specifies the SFN of the RSTD reference cell containing the starting subframe of the PRS or NPRS positioning occasion if PRS or NPRS are available on the RSTD reference cell, or subframe of the CRS for RSTD measurements if PRS and NPRS are not available on the RSTD reference cell during which the most recent neighbour cell RSTD measurement was performed. In the case of more than a single PRS configuration on the RSTD reference cell, the first PRS configuration is referenced.

physCellIdRef This field specifies the physical cell identity of the RSTD reference cell.

cellGlobalIdRef This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the RSTD reference cell. The target shall provide this IE if it knows the ECGI of the RSTD reference cell.

earfcnRef This field specifies the EARFCN of the RSTD reference cell.

referenceQuality This field specifies the target device′s best estimate of the quality of the TOA measurement from the RSTD reference cell, TSubframeRxRef, where TSubframeRxRef is the time of arrival of the signal from the RSTD reference cell.

neighbourMeasurementList This list contains the measured RSTD values for neighbour cells together with the RSTD reference cell, along with quality for each measurement.

tpIdRef This field specifies the transmission point ID of the RSTD reference cell.

prsIdRef This field specifies the PRS-ID of the first PRS configuration of the RSTD reference cell.

additionalPathsRef This field specifies one or more additional detected path timing values for the RSTD reference cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nprsIdRef This field specifies the NPRS-ID of the RSTD reference cell.

carrierFreqOffsetNB-Ref This field specifies the offset of the NB-IoT channel number to EARFCN given by earfcnRef as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper SFN as defined in TS 36.331 [12] of the RSTD reference cell for the systemFrameNumber.

physCellIdNeighbour This field specifies the physical cell identity of the neighbour cell for which the RSTDs are provided.

cellGlobalIdNeighbour This field specifies the ECGI, the globally unique identity of a cell in E-UTRA, of the neighbour cell for which the RSTDs are provided. The target device shall provide this IE if it was able to determine the ECGI of the neighbour cell at the time of measurement.

earfcnNeighbour This field specifies the EARFCN of the neighbour cell used for the RSTD measurements.

rstd This field specifies the relative timing difference between this neighbour cell and the RSTD reference cell, as defined in TS 36.214 [17]. Mapping of the measured quantity is defined as in TS 36.133 [18] clause 9.1.10.3.

rstd-Quality This field specifies the target device′s best estimate of the quality of the measured rstd.

tpIdNeighbour This field specifies the transmission point ID for the neighbour cell for which the RSTDs are provided.

prsIdNeighbour This field specifies the PRS-ID of the first PRS configuration of the neighbour cell for which the RSTDs are provided.

delta-rstd This field specifies the higher-resolution RSTD RSTD as defined in TS 36.133 [18] clause 9.1.10.4. Mapping of the measured quantity is defined as in TS 36.133 [18] clause 9.1.10.4.

additionalPathsNeighbour This field specifies one or more additional detected path timing values for the neighbour cell, relative to the path timing used for determining the rstd value. If this field was requested but is not included, it means the UE did not detect any additional path timing values.

nprsIdNeighbour This field specifies the NPRS-ID of the neighbour cell for which the RSTDs are provided.

carrierFreqOffsetNB-Neighbour This field specifies the offset of the NB-IoT channel number to EARFCN given by earfcnNeighbour as defined in TS 36.101 [21].

#### –	OTDOA-MeasQuality

-- ASN1START

OTDOA-MeasQuality ::= SEQUENCE {

error-Resolution		BIT STRING (SIZE (2)),

error-Value				BIT STRING (SIZE (5)),

error-NumSamples		BIT STRING (SIZE (3))				OPTIONAL,

...

}

-- ASN1STOP

OTDOA-MeasQuality field descriptions

error-Resolution This field specifies the resolution R used in error-Value field. The encoding on two bits is as follows: ′00′ 5 metres ′01′ 10 metres ′10′ 20 metres ′11′ 30 metres

error-Value This field specifies the target device′s best estimate of the uncertainty of the OTDOA (or TOA) measurement. The encoding on five bits is as follows: ′00000′ 0 to (R*1-1) metres ′00001′ R*1 to (R*2-1) metres ′00010′ R*2 to (R*3-1) metres … ′11111′ R*31 metres or more; where R is the resolution defined by error-Resolution field. E.g., R=20 m corresponds to 0-19 m, 20-39 m,…,620+ m.

error-NumSamples If the error-Value field provides the sample uncertainty of the OTDOA (or TOA) measurement, this field specifies how many measurements have been used by the target device to determine this (i.e., sample size). Following 3 bit encoding is used: ′000′ Not the baseline metric ′001′ 5-9 ′010′ 10-14 ′011′ 15-24 ′100′ 25-34 ′101′ 35-44 ′110′ 45-54 ′111′ 55 or more. In the case of the value ′000′, the error-Value field contains the target device′s best estimate of the uncertainty of the OTDOA (or TOA) measurement not based on the baseline metric. E.g., other measurements such as signal-to-noise-ratio or signal strength can be utilized to estimate the error-Value. If this field is absent, the value of this field is ′000′.

#### –	AdditionalPath

The IE AdditionalPath is used by the target device to provide information about additional paths associated with the RSTD measurements in the form of a relative time difference and a quality value. The additional path relativeTimeDifference is the detected path timing relative to the detected path timing used for the rstd value (TS 36.214 [17]), and each additional path can be associated with a quality value path-Quality.

-- ASN1START

AdditionalPath-r14 ::= SEQUENCE {

relativeTimeDifference-r14	INTEGER (-256..255),

path-Quality-r14			OTDOA-MeasQuality				OPTIONAL,

...

}

-- ASN1STOP

AdditionalPath field descriptions

relativeTimeDifference This field specifies the additional detected path timing relative to the detected path timing used for the rstd value in units of 0.5 Ts, with Ts=1/(15000*2048) seconds. A positive value indicates that the particular path is later in time than the detected path used for RSTD; a negative value indicates that the particular path is earlier in time than the detected path used for RSTD.

path-Quality This field specifies the target device′s best estimate of the quality of the detected timing of the additional path.

#### 6.5.1.6	OTDOA Location Information Request

#### –	OTDOA-RequestLocationInformation

The IE OTDOA-RequestLocationInformation is used by the location server to request OTDOA location measurements from a target device. Details of the required measurements (e.g. details of assistance data reference cell and neighbour cells) are conveyed in the OTDOA-ProvideAssistanceData IE in a separate Provide Assistance Data message.

-- ASN1START

OTDOA-RequestLocationInformation ::= SEQUENCE {

assistanceAvailability		BOOLEAN,

...,

[[

multipathRSTD-r14		ENUMERATED { requested }	OPTIONAL,		-- Need ON

maxNoOfRSTDmeas-r14		INTEGER (1..32)				OPTIONAL		-- Need ON

]],

[[

motionMeasurements-r15	ENUMERATED { requested }	OPTIONAL		-- Need ON

]]

}

-- ASN1STOP

OTDOA-RequestLocationInformation field descriptions

assistanceAvailability This field indicates whether the target device may request additional OTDOA assistance data from the server. TRUE means allowed and FALSE means not allowed.

multipathRSTD This field, if present, indicates that the target device is requested to report additional detected path timing information per RSTD reference and neighbour cell.

maxNoOfRSTDmeas This field, if present, indicates the maximum number of NeighbourMeasurementElement fields (i.e., RSTD measurements) the target device can provide in OTDOA-SignalMeasurementInformation.

motionMeasurements This field, if present, indicates that the target device is requested to report the motion measurements (deltaSFN and motionTimeSource) in OTDOA-SignalMeasurementInformation as well as the IE Sensor-MotionInformation in IE Sensor-ProvideLocationInformation.

#### 6.5.1.7	OTDOA Capability Information

#### –	OTDOA-ProvideCapabilities

The IE OTDOA-ProvideCapabilities is used by the target device to indicate its capability to support OTDOA and to provide its OTDOA positioning capabilities to the location server.

-- ASN1START

OTDOA-ProvideCapabilities ::= SEQUENCE {

otdoa-Mode		BIT STRING {	ue-assisted				(0),

ue-assisted-NB-r14		(1),

ue-assisted-NB-TDD-r15	(2) } (SIZE (1..8)),

...,

supportedBandListEUTRA		SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA		OPTIONAL,

supportedBandListEUTRA-v9a0	SEQUENCE (SIZE (1..maxBands)) OF SupportedBandEUTRA-v9a0

OPTIONAL,

interFreqRSTDmeasurement-r10		ENUMERATED { supported }						OPTIONAL,

additionalNeighbourCellInfoList-r10	ENUMERATED { supported }						OPTIONAL,

prs-id-r14							ENUMERATED { supported }						OPTIONAL,

tp-separation-via-muting-r14		ENUMERATED { supported }						OPTIONAL,

additional-prs-config-r14			ENUMERATED { supported }						OPTIONAL,

prs-based-tbs-r14					ENUMERATED { supported }						OPTIONAL,

additionalPathsReport-r14			ENUMERATED { supported }						OPTIONAL,

densePrsConfig-r14					ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsBandwidth-r14		ENUMERATED { n6, n15, n25, n50, n75, n100, ...}	OPTIONAL,

prsOccGroup-r14						ENUMERATED { supported }						OPTIONAL,

prsFrequencyHopping-r14				ENUMERATED { supported }						OPTIONAL,

maxSupportedPrsConfigs-r14			ENUMERATED { c2, c3 }							OPTIONAL,

periodicalReporting-r14				ENUMERATED { supported }						OPTIONAL,

multiPrbNprs-r14					ENUMERATED { supported }						OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }							OPTIONAL,

numberOfRXantennas-r14				ENUMERATED { rx1, ... }							OPTIONAL,

motionMeasurements-r15				ENUMERATED { supported }						OPTIONAL,

interRAT-RSTDmeasurement-r15		ENUMERATED { supported }						OPTIONAL,

scheduledLocationRequest-r17		SEQUENCE {

utcTime-r17			ENUMERATED { supported }	OPTIONAL,

gnssTime-r17		GNSS-ID-Bitmap				OPTIONAL,

e-utraTime-r17		ENUMERATED { supported }	OPTIONAL,

nrTime-r17			ENUMERATED { supported }	OPTIONAL,

relativeTime-r17	ENUMERATED { supported }	OPTIONAL,

...

}												OPTIONAL

}

maxBands INTEGER ::= 64

SupportedBandEUTRA ::= SEQUENCE {

bandEUTRA							INTEGER (1..maxFBI)

}

SupportedBandEUTRA-v9a0 ::=		SEQUENCE {

bandEUTRA-v9a0						INTEGER (maxFBI-Plus1..maxFBI2)		OPTIONAL

}

maxFBI								INTEGER	::=	64	-- Maximum value of frequency band indicator

maxFBI-Plus1						INTEGER ::= 65	-- lowest value extended FBI range

maxFBI2								INTEGER ::= 256	-- highest value extended FBI range

-- ASN1STOP

OTDOA-ProvideCapabilities field descriptions

otdoa-Mode This field specifies the OTDOA mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the particular OTDOA mode is supported; a zero value means not supported. A zero-value in all bit positions in the bit string means OTDOA positioning method is not supported by the target device. ue-assisted: Bit 0 indicates that the target device supports UE-assisted OTDOA and LTE PRS. ue-assisted-NB: Bit 1 indicates that the target device supports UE-assisted OTDOA and NB-IoT NPRS. ue-assisted-NB-TDD: Bit 2 indicates that the target device supports UE-assisted OTDOA and NB-IoT NPRS for TDD.

SupportedBandEUTRA This field specifies the frequency bands for which the target device supports RSTD measurements. One entry corresponding to each supported E-UTRA band as defined in TS 36.101 [21]. In the case the target device includes bandEUTRA-v9a0, the target device shall set the corresponding entry of bandEUTRA (i.e. without suffix) to maxFBI.

interFreqRSTDmeasurement This field, if present, indicates that the target device supports inter-frequency RSTD measurements within and between the frequency bands indicated in SupportedBandEUTRA.

additionalNeighbourCellInfoList This field, if present, indicates that the target device supports up to 3×24 OTDOA-NeighbourCellInfoElement in OTDOA-NeighbourCellInfoList in OTDOA-ProvideAssistanceData without any restriction for the earfcn in each OTDOA-NeighbourCellInfoElement as specified in clause 6.5.1.2.

prs-id This field, if present, indicates that the target device supports PRS generation based on the PRS-ID as specified in TS 36.211 [16] and support for TP-ID in OTDOA-ReferenceCellInfo and OTDOA-NeighbourCellInfoList.

tp-separation-via-muting This field, if present, indicates that the target device supports RSTD measurements for cells which have associated transmission points (e.g., Remote Radio Heads) within the cell coverage and where these associated transmission points have the same physical cell identity as the associated cell, and where these transmission points are identified via a different muting pattern. The field also indicates support for TP-ID in OTDOA-ReferenceCellInfo and OTDOA-NeighbourCellInfoList.

additional-prs-config This field, if present, indicates that the target device supports additional PRS configurations. The additional PRS configuration in PRS-Info IE comprise: - support for prs-ConfigurationIndex > 2399; - support for NPRS values in addition to 1, 2, 4 and 6 (add-numDL-Frames in PRS-Info); - support for muting bit string lengths > 16 bits.

prs-based-tbs This field, if present, indicates that the target device supports RSTD measurements for PRS-only TPs.

additionalPathsReport This field, if present, indicates that the target device supports reporting of timing information for additional detected paths for RSTD reference and each neighbour cell.

densePrsConfig This field, if present, indicates that the target device supports a subset of the additional PRS configurations associated with capability additional-prs-config which comprises: - support for prs-ConfigurationIndex > 2404; - support for NPRS values of 10, 20, 40, 80 and 160 (in addition to 1, 2, 4 and 6). In the case additional-prs-config is present, this field is not present.

maxSupportedPrsBandwidth This field, if present, indicates the maximum PRS bandwidth supported by the target device. Enumerated value n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on. If this field is not present, the target device is assumed to support the PRS bandwidth associated with the target device type, which for LTE devices including Cat-M1/M2 is 100 resource blocks and for NB-IoT devices is 1 resource block.

prsOccGroup This field, if present, indicates that the target device supports PRS occasion groups, which implies that each bit of a configured muting pattern applies per PRS occasion group.

prsFrequencyHopping This field, if present, indicates that the target device supports PRS occasion frequency hopping, as specified in TS 36.211 [16].

maxSupportedPrsConfigs This field, if present, indicates that the target device supports multiple PRS configurations per cell. Enumerated value c2 indicates support for up to 2 configurations; c3 indicates support for up to 3 configurations.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of RSTD measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

multiPrbNprs This field, if present, indicates that the target device supports NPRS configuration in more than one resource block (i.e., maxCarrier in PRS-Info-NB greater 1).

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform RSTD measurements.

numberOfRXantennas This field is not applicable to NB-IoT devices. This field, if present, indicates the number of UE downlink receive antennas for RSTD measurements (see TS 36.133 [18]). Enumerated value rx1 indicates a single antenna receiver. If this field is absent, the target device is assumed to support two RX antennas for RSTD measurements.

motionMeasurements This field, if present, indicates that the target device supports reporting of motion measurements (deltaSFN and motionTimeSource) in OTDOA-SignalMeasurementInformation. The presence of this field implies presence of sensor-MotionInformationSup in IE Sensor-ProvideCapabilities.

interRAT-RSTDmeasurement This field, if present, indicates that the target device supports inter-RAT RSTD measurements (TS 38.215 [36]); i.e., E-UTRA RSTD measurements when the target device is served by an NR cell.

scheduledLocationRequest This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

#### 6.5.1.8	OTDOA Capability Information Request

#### –	OTDOA-RequestCapabilities

The IE OTDOA-RequestCapabilities is used by the location server to request the capability of the target device to support OTDOA and to request OTDOA positioning capabilities from a target device.

-- ASN1START

OTDOA-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.1.9	OTDOA Error Elements

#### –	OTDOA-Error

The IE OTDOA-Error is used by the location server or target device to provide OTDOA error reasons to the target device or location server, respectively.

-- ASN1START

OTDOA-Error ::= CHOICE {

locationServerErrorCauses		OTDOA-LocationServerErrorCauses,

targetDeviceErrorCauses			OTDOA-TargetDeviceErrorCauses,

...

}

-- ASN1STOP

#### –	OTDOA-LocationServerErrorCauses

The IE OTDOA-LocationServerErrorCauses is used by the location server to provide OTDOA error reasons to the target device.

-- ASN1START

OTDOA-LocationServerErrorCauses ::= SEQUENCE {

cause		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

...

},

...

}

-- ASN1STOP

#### –	OTDOA-TargetDeviceErrorCauses

The IE OTDOA-TargetDeviceErrorCauses is used by the target device to provide OTDOA error reasons to the location server.

-- ASN1START

OTDOA-TargetDeviceErrorCauses ::= SEQUENCE {

cause		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureReferenceCell,

unableToMeasureAnyNeighbourCell,

attemptedButUnableToMeasureSomeNeighbourCells,

...

},

...

}

-- ASN1STOP

### 6.5.2	A-GNSS Positioning

#### 6.5.2.1	GNSS Assistance Data

#### –	A-GNSS-ProvideAssistanceData

The IE A-GNSS-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based and UE-assisted A-GNSS. It may also be used to provide GNSS positioning specific error reasons.

-- ASN1START

A-GNSS-ProvideAssistanceData ::= SEQUENCE {

gnss-CommonAssistData			GNSS-CommonAssistData				OPTIONAL,	-- Need ON

gnss-GenericAssistData			GNSS-GenericAssistData				OPTIONAL,	-- Need ON

gnss-Error						A-GNSS-Error						OPTIONAL,	-- Need ON

...,

[[

gnss-PeriodicAssistData-r15	GNSS-PeriodicAssistData-r15			OPTIONAL	-- Cond CtrTrans

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| CtrTrans | The field is mandatory present in the control transaction of a periodic assistance data delivery session as described in clauses 5.2.1a and 5.2.2a. Otherwise it is not present. |

#### –	GNSS-CommonAssistData

The IE GNSS-CommonAssistData is used by the location server to provide assistance data which can be used for any GNSS.

-- ASN1START

GNSS-CommonAssistData ::= SEQUENCE {

gnss-ReferenceTime				GNSS-ReferenceTime					OPTIONAL,	-- Need ON

gnss-ReferenceLocation			GNSS-ReferenceLocation				OPTIONAL,	-- Need ON

gnss-IonosphericModel			GNSS-IonosphericModel				OPTIONAL,	-- Need ON

gnss-EarthOrientationParameters	GNSS-EarthOrientationParameters		OPTIONAL,	-- Need ON

...,

[[

gnss-RTK-ReferenceStationInfo-r15

GNSS-RTK-ReferenceStationInfo-r15	OPTIONAL,	-- Need ON

gnss-RTK-CommonObservationInfo-r15

GNSS-RTK-CommonObservationInfo-r15	OPTIONAL,	-- Cond RTK

gnss-RTK-AuxiliaryStationData-r15

GNSS-RTK-AuxiliaryStationData-r15	OPTIONAL	-- Need ON

]],

[[

gnss-SSR-CorrectionPoints-r16

GNSS-SSR-CorrectionPoints-r16		OPTIONAL	-- Need ON

]],

[[

gnss-Integrity-ServiceParameters-r17

GNSS-Integrity-ServiceParameters-r17		OPTIONAL,	-- Need ON

gnss-Integrity-ServiceAlert-r17

GNSS-Integrity-ServiceAlert-r17				OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| RTK | The field is mandatory present if the IE GNSS-RTK-Observations is included in IE GNSS-GenericAssistData; otherwise it is not present. |

#### –	GNSS-GenericAssistData

The IE GNSS-GenericAssistData is used by the location server to provide assistance data for a specific GNSS. The specific GNSS for which the provided assistance data are applicable is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be provided.

-- ASN1START

GNSS-GenericAssistData ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataElement

GNSS-GenericAssistDataElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-ID							SBAS-ID							OPTIONAL, 	-- Cond GNSS-ID-SBAS

gnss-TimeModels					GNSS-TimeModelList				OPTIONAL,	-- Need ON

gnss-DifferentialCorrections	GNSS-DifferentialCorrections	OPTIONAL,	-- Need ON

gnss-NavigationModel			GNSS-NavigationModel			OPTIONAL,	-- Need ON

gnss-RealTimeIntegrity			GNSS-RealTimeIntegrity			OPTIONAL,	-- Need ON

gnss-DataBitAssistance			GNSS-DataBitAssistance			OPTIONAL,	-- Need ON

gnss-AcquisitionAssistance		GNSS-AcquisitionAssistance		OPTIONAL,	-- Need ON

gnss-Almanac					GNSS-Almanac					OPTIONAL,	-- Need ON

gnss-UTC-Model					GNSS-UTC-Model					OPTIONAL,	-- Need ON

gnss-AuxiliaryInformation		GNSS-AuxiliaryInformation		OPTIONAL,	-- Need ON

...,

[[

bds-DifferentialCorrections-r12

BDS-DifferentialCorrections-r12	OPTIONAL,	-- Cond	GNSS-ID-BDS

bds-GridModel-r12			BDS-GridModelParameter-r12		OPTIONAL	-- Cond	GNSS-ID-BDS

]],

[[

gnss-RTK-Observations-r15	GNSS-RTK-Observations-r15		OPTIONAL,	-- Need ON

glo-RTK-BiasInformation-r15	GLO-RTK-BiasInformation-r15		OPTIONAL,	-- Cond GNSS-ID-GLO

gnss-RTK-MAC-CorrectionDifferences-r15

GNSS-RTK-MAC-CorrectionDifferences-r15

OPTIONAL,	-- Need ON

gnss-RTK-Residuals-r15		GNSS-RTK-Residuals-r15			OPTIONAL,	-- Need ON

gnss-RTK-FKP-Gradients-r15	GNSS-RTK-FKP-Gradients-r15		OPTIONAL,	-- Need ON

gnss-SSR-OrbitCorrections-r15

GNSS-SSR-OrbitCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-ClockCorrections-r15

GNSS-SSR-ClockCorrections-r15	OPTIONAL,	-- Need ON

gnss-SSR-CodeBias-r15		GNSS-SSR-CodeBias-r15			OPTIONAL	-- Need ON

]],

[[

gnss-SSR-URA-r16					GNSS-SSR-URA-r16		OPTIONAL,	-- Need ON

gnss-SSR-PhaseBias-r16				GNSS-SSR-PhaseBias-r16	OPTIONAL,	-- Need ON

gnss-SSR-STEC-Correction-r16		GNSS-SSR-STEC-Correction-r16

OPTIONAL,	-- Need ON

gnss-SSR-GriddedCorrection-r16		GNSS-SSR-GriddedCorrection-r16

OPTIONAL,	-- Need ON

navic-DifferentialCorrections-r16	NavIC-DifferentialCorrections-r16

OPTIONAL,	-- Cond	GNSS-ID-NavIC

navic-GridModel-r16					NavIC-GridModelParameter-r16

OPTIONAL	-- Cond	GNSS-ID-NavIC

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| GNSS-ID-BDS | The field may be present if the GNSS-ID = bds; otherwise it is not present. |
| GNSS-ID-GLO | The field is optionally present, need ON, if the GNSS ID = glonass; otherwise it is not present. |
| GNSS-ID-NAVIC | The field is optionally present, need ON, if the GNSS-ID = navic; otherwise it is not present |

#### –	GNSS-PeriodicAssistData

The IE GNSS-PeriodicAssistData is used by the location server to provide control parameters for a periodic assistance data delivery session (e.g., interval and duration) to the target device.

NOTE:	Omission of a particular assistance data type field in IE GNSS-PeriodicAssistData means that the location server does not provide this assistance data type in a data transaction of a periodic assistance data delivery session, as described in clauses 5.2.1a and 5.2.2a. Inclusion of no assistance data type fields in IE GNSS-PeriodicAssistData means that a periodic assistance data delivery session is terminated.

-- ASN1START

GNSS-PeriodicAssistData-r15 ::= SEQUENCE {

gnss-RTK-PeriodicObservations-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

glo-RTK-PeriodicBiasInformation-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-MAC-PeriodicCorrectionDifferences-r15

GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-PeriodicResiduals-r15			GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-RTK-FKP-PeriodicGradients-r15		GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-SSR-PeriodicOrbitCorrections-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicClockCorrections-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicCodeBias-r15			GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

...,

[[

gnss-SSR-PeriodicURA-r16				GNSS-PeriodicControlParam-r15	OPTIONAL,	-- Need ON

gnss-SSR-PeriodicPhaseBias-r16			GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicSTEC-Correction-r16	GNSS-PeriodicControlParam-r15	OPTIONAL, 	-- Need ON

gnss-SSR-PeriodicGriddedCorrection-r16	GNSS-PeriodicControlParam-r15	OPTIONAL 	-- Need ON

]],

[[

gnss-Integrity-PeriodicServiceAlert-r17	GNSS-PeriodicControlParam-r15	OPTIONAL 	-- Need ON

]]

}

-- ASN1STOP

Editor's Note: FFS whether the GNSS-Integrity-ServiceParameters need to be provided periodically.

#### 6.5.2.2	GNSS Assistance Data Elements

#### –	GNSS-ReferenceTime

The IE GNSS-ReferenceTime is used by the location server to provide the GNSS specific system time with uncertainty and the relationship between GNSS system time and network air-interface timing of the eNodeB/NodeB/BTS transmission in the reference cell.

If the IE networkTime is present, the IEs gnss-SystemTime and networkTime provide a valid relationship between GNSS system time and air-interface network time, as seen at the approximate location of the target device, i.e. the propagation delay from the gNB/ng-eNB/eNodeB/NodeB/BTS to the target device shall be compensated for by the location server. Depending on implementation, the relation between GNSS system time and air-interface network time may have varying accuracy. The uncertainty of this timing relation is provided in the IE referenceTimeUnc. If the propagation delay from the eNodeB/NodeB/BTS to the target device is not accurately known, the location server shall use the best available approximation of the propagation delay and take the corresponding delay uncertainty into account in the calculation of the IE referenceTimeUnc.

If the IE networkTime is not present, the IE gnssSystemTime is an estimate of current GNSS system time at time of reception of the IE GNSS-ReferenceTime by the target device. The location server should achieve an accuracy of +/- 3 seconds for this estimate including allowing for the transmission delay between the location server and the target device. Note that the target device should further compensate gnss-SystemTime for the time between the reception of GNSS-ReferenceTime and the time when the gnss-SystemTime is used.

The location server shall provide a value for the gnss-TimeID only for GNSSs supported by the target device.

The IE GNSS-ReferenceTimeForOneCell can be provided multiple times (up to 16) to provide fine time assistance for several (neighbour) cells.

-- ASN1START

GNSS-ReferenceTime ::= SEQUENCE {

gnss-SystemTime				GNSS-SystemTime,

referenceTimeUnc			INTEGER (0..127)					OPTIONAL,	-- Cond noFTA

gnss-ReferenceTimeForCells	SEQUENCE (SIZE (1..16)) OF

GNSS-ReferenceTimeForOneCell	OPTIONAL,	-- Need ON

...

}

GNSS-ReferenceTimeForOneCell ::= SEQUENCE {

networkTime				NetworkTime,

referenceTimeUnc			INTEGER (0..127),

bsAlign						ENUMERATED {true}	OPTIONAL,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| noFTA | The field may be present if gnss-ReferenceTimeForCells is absent; otherwise it is not present. |

GNSS-ReferenceTime field descriptions

gnss-SystemTime This field provides the specific GNSS system time.

networkTime This field specifies the cellular network time at the epoch corresponding to gnss-SystemTime.

referenceTimeUnc This field provides the accuracy of the relation between gnssSystemTime and networkTime time if IE networkTime is provided. When IE networkTime is not provided, this field can be included to provide the accuracy of the provided gnssSystemTime. If GNSS TOD is the given GNSS time, then the true GNSS time, corresponding to the provided network time as observed at the target device location, lies in the interval [GNSS TOD - referenceTimeUnc, GNSS TOD + referenceTimeUnc]. The uncertainty r, expressed in microseconds, is mapped to a number K, with the following formula: r = C*(((1+x)K)-1) with C = 0.5 and x = 0.14. To encode any higher value of uncertainty than that corresponding in the above formula to K=127, the same value, K=127, shall also be used. The uncertainty is then coded on 7 bits, as the binary encoding of K. Example values for the referenceTimeUnc Format: see table K to uncertainty relation below.

bsAlign This flag, if present, indicates that the transmission timings of all cells sharing, depending on the RAT, the same carrier frequency and Tracking Area/Location Area/Routing Area as the cell indicated, are frame aligned. This information allows the target device to derive the GNSS - cellular time relation for any of these cells based on the timing relation information provided in GNSS-ReferenceTime. The flag should be set consistently in all these cells. This flag does not guarantee SFN alignment.

K to uncertainty relation

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 nanoseconds |
| 1 | 70 nanoseconds |
| 2 | 149.8 nanoseconds |
| - | - |
| 50 | 349.62 microseconds |
| - | - |
| 127 | ≥ 8.43 seconds |

#### –	GNSS-SystemTime

-- ASN1START

GNSS-SystemTime ::= SEQUENCE {

gnss-TimeID						GNSS-ID,

gnss-DayNumber					INTEGER (0..32767),

gnss-TimeOfDay					INTEGER (0..86399),

gnss-TimeOfDayFrac-msec			INTEGER (0..999)		OPTIONAL,	-- Need ON

notificationOfLeapSecond		BIT STRING (SIZE(2))	OPTIONAL,	-- Cond gnss-TimeID-glonass

gps-TOW-Assist					GPS-TOW-Assist			OPTIONAL,	-- Cond gnss-TimeID-gps

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| gnss-TimeID-glonass | The field may be present if gnss-TimeID=`glonass′; otherwise it is not present. |
| gnss-TimeID-gps | The field may be present if gnss-TimeID=`gps′; otherwise it is not present. |

GNSS-SystemTime field descriptions

gnss-TimeID This field specifies the GNSS for which the GNSS-SystemTime is provided.

gnss-DayNumber This field specifies the sequential number of days (with day count starting at 0) from the origin of the GNSS System Time as follows: GPS, QZSS, SBAS – Days from January 6th 1980 00:00:00 UTC (USNO); Galileo – Days from Galileo System Time (GST) start epoch, defined as 13 seconds before midnight between 21st August and 22nd August 1999; i.e., GST was equal to 13 seconds at August 22nd 1999 00:00:00 UTC; GLONASS – Days from December 31st 1995 21:00:00 UTC (SU), which is local UTC Moscow January 1st 1996 00:00:00, defined as UTC(SU) + 3 hours in [9]; BDS – Days from January 1st 2006 00:00:00 UTC (NTSC). NavIC – Days from NavIC System Time start epoch, defined as 13 seconds before midnight between 21st August and 22nd August 1999; i.e., NavIC System Time was equal to 00:00:00 at August 21st, 1999 23:55:47 UTC (BIPM).

gnss-TimeOfDay This field specifies the integer number of seconds from the GNSS day change.

gnss-TimeOfDayFrac-msec This field specifies the fractional part of the gnssTimeOfDay field in 1-milli-seconds resolution. The total GNSS TOD is gnss-TimeOfDay + gnssTimeOfDayFrac-msec.

notificationOfLeapSecond This field specifies the notification of forthcoming leap second correction, as defined by parameter KP in [9, Table 4.7].

gps-TOW-Assist This field contains several fields in the Telemetry (TLM) Word and Handover Word (HOW) that are currently being broadcast by the respective GPS satellites. Combining this information with GPS TOW enables the target device to know the entire 1.2-second (60-bit) pattern of TLM and HOW that is transmitted at the start of each six-second NAV subframe by the particular GPS satellite.

#### –	GPS-TOW-Assist

-- ASN1START

GPS-TOW-Assist ::= SEQUENCE (SIZE(1..64)) OF GPS-TOW-AssistElement

GPS-TOW-AssistElement ::= SEQUENCE {

satelliteID		INTEGER (1..64),

tlmWord			INTEGER (0..16383),

antiSpoof		INTEGER (0..1),

alert			INTEGER (0..1),

tlmRsvdBits		INTEGER (0..3),

...

}

-- ASN1STOP

GPS-TOW-Assist field descriptions

satelliteID This field identifies the satellite for which the GPS-TOW-Assist is applicable. This field is identical to the GPS PRN Signal No. defined in [4].

tlmWord This field contains a 14-bit value representing the Telemetry Message (TLM) being broadcast by the GPS satellite identified by the particular satelliteID, with the MSB occurring first in the satellite transmission, as defined in [4].

antiSpoof This field contains the Anti-Spoof flag that is being broadcast by the GPS satellite identified by satelliteID, as defined in [4].

alert This field contains the Alert flag that is being broadcast by the GPS satellite identified by satelliteID, as defined in [4].

tlmRsvdBits This field contains the two reserved bits in the TLM Word being broadcast by the GPS satellite identified by satelliteID, with the MSB occurring first in the satellite transmission, as defined in [4].

#### –	NetworkTime

-- ASN1START

NetworkTime ::= SEQUENCE {

secondsFromFrameStructureStart				INTEGER(0..12533),

fractionalSecondsFromFrameStructureStart	INTEGER(0..3999999),

frameDrift									INTEGER (-64..63)	OPTIONAL,	-- Cond GNSSsynch

cellID		CHOICE {

eUTRA		SEQUENCE {

physCellId			INTEGER (0..503),

cellGlobalIdEUTRA	CellGlobalIdEUTRA-AndUTRA	OPTIONAL,	-- Need ON

earfcn				ARFCN-ValueEUTRA,

...,

[[ earfcn-v9a0		ARFCN-ValueEUTRA-v9a0 OPTIONAL	-- Cond EARFCN-max

]]

},

uTRA		SEQUENCE {

mode	CHOICE {

fdd		SEQUENCE {

primary-CPICH-Info	INTEGER (0..511),

...

},

tdd		SEQUENCE {

cellParameters		INTEGER (0..127),

...

}

},

cellGlobalIdUTRA	CellGlobalIdEUTRA-AndUTRA	OPTIONAL,	-- Need ON

uarfcn				ARFCN-ValueUTRA,

...

},

gSM			SEQUENCE {

bcchCarrier			INTEGER (0..1023),

bsic				INTEGER (0..63),

cellGlobalIdGERAN	CellGlobalIdGERAN			OPTIONAL,	-- Need ON

...

},

...,

nBIoT-r14	SEQUENCE {

nbPhysCellId-r14	INTEGER (0..503),

nbCellGlobalId-r14	ECGI						OPTIONAL,	-- Need ON

nbCarrierFreq-r14	CarrierFreq-NB-r14,

...

},

nr-r15		SEQUENCE {

nrPhysCellId-r15	INTEGER (0..1007),

nrCellGlobalID-r15	NCGI-r15					OPTIONAL,	-- Need ON

nrARFCN-r15			ARFCN-ValueNR-r15,

...

}

},

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding earfcn (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| GNSSsynch | The field is present and set to 0 if NetworkTime is synchronized to gnss-SystemTime; otherwise the field is optionally present, need OR. |

NetworkTime field descriptions

secondsFromFrameStructureStart This field specifies the number of seconds from the beginning of the longest frame structure in the corresponding air interface. In the case of E-UTRA, the SFN cycle length is 10.24 seconds. In the case of UTRA, the SFN cycle length is 40.96 seconds. In the case of GSM, the hyperfame length is 12533.76 seconds. In the case of NB-IoT, the Hyper-SFN cycle lengths is 10485.76 seconds. In the case of NR, the SFN cycle length is 10.24 seconds.

fractionalSecondsFromFrameStructureStart This field specifies the fractional part of the secondsFromFrameStructureStart in 250 ns resolution. The total time since the particular frame structure start is secondsFromFrameStructureStart + fractionalSecondsFromFrameStructureStart

frameDrift This field specifies the drift rate of the GNSS-network time relation with scale factor 2-30 seconds/second, in the range from -5.9605e-8 to +5.8673e-8 sec/sec.

cellID This field specifies the cell for which the GNSS–network time relation is provided.

physCellId This field specifies the physical cell identity of the reference cell (E-UTRA), as defined in TS 36.331 [12], for which the GNSS network time relation is provided.

cellGlobalIdEUTRA This field specifies the Evolved Cell Global Identifier (ECGI), the globally unique identity of a cell in E-UTRA, of the reference cell for the GNSS-network time relation, as defined in TS 36.331 [12].

earfcn This field specifies E-ARFCN of the reference cell for the GNSS-network time relation (E-UTRA). In the case the server includes earfcn-v9a0, the server shall set the corresponding earfcn (i.e. without suffix) to maxEARFCN.

primary-CPICH-Info This field specifies the physical cell identity of the reference cell (UTRA) for the GNSS-network time relation, as defined in TS 25.331 [13].

cellParameters This field specifies the physical cell identity of the reference cell (UTRA) for the GNSS-network time relation, as defined in TS 25.331 [13].

cellGlobalIdUTRA The filed specifies the global UTRAN Cell Identifier, the globally unique identity of a cell in UTRA, of the reference cell for the GNSS-network time relation, as defined in TS 25.331 [13].

uarfcn This field specifies ARFCN of the reference cell for the GNSS-network time relation (UTRA).

bcchCarrier This field specifies the absolute GSM RF channel number of the BCCH of the reference base station (GERAN) for the GNSS-network time relation, as defined in TS 44.031 [14].

bsic This field specifies the Base Station Identity Code of the reference base station (GERAN) for the GNSS-network time relation, as defined in TS 44.031 [14].

cellGlobalIdGERAN This field specifies the Cell Global Identification (CGI), the globally unique identity of a cell in GERAN, of the reference base station for the GNSS-network time relation.

nbPhysCellId This field specifies the narrowband physical layer cell identity of the NB-IoT reference cell, as defined in TS 36.331 [12], for which the GNSS network time relation is provided.

nbCellGlobalId This field specifies the global cell identifier of the NB-IoT reference cell for which the GNSS-network time relation is provided, as defined in TS 36.331 [12].

nbCarrierFreq This field specifies the carrier frequency of the NB-IoT reference cell for which the GNSS-network time relation is provided.

nrPhysCellId This field specifies the physical cell identity of the reference cell (NR), as defined in TS 38.331 [35], for which the GNSS network time relation is provided.

nrCellGlobalID This field specifies the NR Cell Global Identifier (NCGI) of the reference cell (NR) for the GNSS-network time relation, as defined in TS 38.331 [35].

nrARFCN This field specifies NR-ARFCN of the reference cell (NR) for the GNSS-network time relation.

#### –	GNSS-ReferenceLocation

The IE GNSS-ReferenceLocation is used by the location server to provide the target device with a-priori knowledge of its location in order to improve GNSS receiver performance. The IE GNSS-ReferenceLocation is provided in WGS-84 reference system.

-- ASN1START

GNSS-ReferenceLocation ::= SEQUENCE {

threeDlocation			EllipsoidPointWithAltitudeAndUncertaintyEllipsoid,

...

}

-- ASN1STOP

#### –	GNSS-IonosphericModel

The IE GNSS-IonosphericModel is used by the location server to provide parameters to model the propagation delay of the GNSS signals through the ionosphere. Proper use of these fields allows a single-frequency GNSS receiver to remove parts of the ionospheric delay from the pseudorange measurements. Three Ionospheric Models are supported: The Klobuchar model as defined in [4], the NeQuick model as defined in [8] , and the klobucharModel2 as defined in [39].

-- ASN1START

GNSS-IonosphericModel ::= SEQUENCE {

klobucharModel			KlobucharModelParameter		OPTIONAL,	-- Need ON

neQuickModel			NeQuickModelParameter		OPTIONAL,	-- Need ON

...,

[[	klobucharModel2-r16		KlobucharModel2Parameter-r16	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

#### –	KlobucharModelParameter

-- ASN1START

KlobucharModelParameter ::= SEQUENCE {

dataID			BIT STRING (SIZE (2)),

alfa0			INTEGER (-128..127),

alfa1			INTEGER (-128..127),

alfa2			INTEGER (-128..127),

alfa3			INTEGER (-128..127),

beta0			INTEGER (-128..127),

beta1			INTEGER (-128..127),

beta2			INTEGER (-128..127),

beta3			INTEGER (-128..127),

...

}

-- ASN1STOP

KlobucharModelParamater field descriptions

dataID When dataID has the value ′11′ it indicates that the parameters have been generated by QZSS, and the parameters have been specialized and are applicable within the area defined in [7]. When dataID has the value ′01′ it indicates that the parameters have been generated by BDS, and UE shall use these parameters according to the description given in 5.2.4.7 in [23], [50]. When the dataID has the value ′10′, it indicates that the parameters have been generated by NavIC, and UE shall use these parameters according to the description given in [38]. When dataID has the value ′00′ it indicates the parameters are applicable worldwide [4], [7].

alpha0 This field specifies the 0 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 2-30 seconds.

alpha1 This field specifies the 1 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 2-27 seconds/semi-circle.

alpha2 This field specifies the 2 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 2-24 seconds/semi-circle2.

alpha3 This field specifies the 3 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 2-24 seconds/semi-circle3.

beta0 This field specifies the 0 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 211 seconds.

beta1 This field specifies the 1 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 214 seconds/semi-circle.

beta2 This field specifies the 2 parameter of the Klobuchar model, as specified in [4], [23], [38, [50]]. Scale factor 216 seconds/semi-circle2.

beta3 This field specifies the 3 parameter of the Klobuchar model, as specified in [4], [23], [38], [50]. Scale factor 216 seconds/semi-circle3.

#### –	KlobucharModel2Parameter

-- ASN1START

KlobucharModel2Parameter-r16 ::= SEQUENCE {

alfa1-r16			INTEGER (0..1023),

alfa2-r16			INTEGER (-128..127),

alfa3-r16			INTEGER (0..255),

alfa4-r16			INTEGER (0..255),

alfa5-r16			INTEGER (0..255),

alfa6-r16			INTEGER (-128..127),

alfa7-r16			INTEGER (-128..127),

alfa8-r16			INTEGER (-128..127),

alfa9-r16			INTEGER (-128..127),

...

}

-- ASN1STOP

KlobucharModel2Parameter field descriptions

alfa1 This field specifies the 1 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa2 This field specifies the 2 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa3 This field specifies the 3 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa4 This field specifies the 4 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa5 This field specifies the  parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor -2-3 TECU.

alfa6 This field specifies the 6 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa7 This field specifies the 7 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa8 This field specifies the 8 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

alfa9 This field specifies the 9 parameter of the Klobuchar model, as specified in 7.8.1 in [39], [49]. Scale factor 2-3 TECU.

#### –	NeQuickModelParameter

-- ASN1START

NeQuickModelParameter ::= SEQUENCE {

ai0			INTEGER (0..2047),

ai1			INTEGER (-1024..1023),

ai2			INTEGER (-8192..8191),

ionoStormFlag1	INTEGER (0..1)		OPTIONAL,	-- Need OP

ionoStormFlag2	INTEGER (0..1)		OPTIONAL,	-- Need OP

ionoStormFlag3	INTEGER (0..1)		OPTIONAL,	-- Need OP

ionoStormFlag4	INTEGER (0..1)		OPTIONAL,	-- Need OP

ionoStormFlag5	INTEGER (0..1)		OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

NeQuickModelParameter field descriptions

ai0 Effective Ionisation Level 1st order parameter. Scale factor 2-2 Solar Flux Units (SFUs), [8] clause 5.1.6.

ai1 Effective Ionisation Level 2nd order parameter. Scale factor 2-8 Solar Flux Units/degree, [8] clause 5.1.6.

ai2 Effective Ionisation Level 3rd order parameter. Scale factor 2-15 Solar Flux Units/degree2, [8] clause 5.1.6.

ionoStormFlag1, ionoStormFlag2, ionoStormFlag3, ionoStormFlag4, ionoStormFlag5 These fields specify the ionosphere disturbance flags (1,…,5) for five different regions as described in [8], clause 5.1.6. If the ionosphere disturbance flag for a region is not present the target device shall treat the ionosphere disturbance condition as unknown.

#### –	GNSS-EarthOrientationParameters

The IE GNSS-EarthOrientationParameters is used by the location server to provide parameters to construct the ECEF and ECI coordinate transformation as defined in [4]. The IE GNSS-EarthOrientationParameters indicates the relationship between the Earth′s rotational axis and WGS-84 reference system.

-- ASN1START

GNSS-EarthOrientationParameters ::= SEQUENCE {

teop				INTEGER (0..65535),

pmX					INTEGER (-1048576..1048575),

pmXdot				INTEGER (-16384..16383),

pmY					INTEGER (-1048576..1048575),

pmYdot				INTEGER (-16384..16383),

deltaUT1			INTEGER (-1073741824..1073741823),

deltaUT1dot			INTEGER (-262144..262143),

...

}

-- ASN1STOP

GNSS-EarthOrientationParameters field descriptions

teop This field specifies the EOP data reference time in seconds, as specified in [4], [39]. Scale factor 24 seconds.

pmX This field specifies the X-axis polar motion value at reference time in arc-seconds, as specified in [4], [39], [49]. Scale factor 2-20 arc-seconds.

pmXdot This field specifies the X-axis polar motion drift at reference time in arc-seconds/day, as specified in [4], [39], [49]. Scale factor 2-21 arc-seconds/day.

pmY This field specifies the Y-axis polar motion value at reference time in arc-seconds, as specified in [4], [39], [49]. Scale factor 2-20 arc-seconds.

pmYdot This field specifies the Y-axis polar motion drift at reference time in arc-seconds/day, as specified in [4], [39], [49]. Scale factor 2-21 arc-seconds/day.

deltaUT1 This field specifies the UT1-UTC difference at reference time in seconds, as specified in [4], [39], [49]. Scale factor 2-24 seconds.

deltaUT1dot This field specifies the Rate of UT1-UTC difference at reference time in seconds/day, as specified in [4], [39], [49] Scale factor 2-25 seconds/day.

#### –	GNSS-RTK-ReferenceStationInfo

The IE GNSS-RTK-ReferenceStationInfo is used by the location server to provide the Earth-centered, Earth-fixed (ECEF) coordinates of the antenna reference point (ARP) of the stationary reference station for which the GNSS-RTK-Observations assistance data are provided together with reference station antenna description.

The parameters provided in IE GNSS-RTK-ReferenceStationInfo are used as specified for message type 1006, 1033 and 1032 in [30].

-- ASN1START

GNSS-RTK-ReferenceStationInfo-r15 ::= SEQUENCE {

referenceStationID-r15					GNSS-ReferenceStationID-r15,

referenceStationIndicator-r15			ENUMERATED {physical, non-physical},

antenna-reference-point-ECEF-X-r15		INTEGER (-137438953472..137438953471),

antenna-reference-point-ECEF-Y-r15		INTEGER (-137438953472..137438953471),

antenna-reference-point-ECEF-Z-r15		INTEGER (-137438953472..137438953471),

antennaHeight-r15						INTEGER (0..65535)					OPTIONAL, -- Need ON

antennaDescription-r15					AntennaDescription-r15				OPTIONAL, -- Need ON

antenna-reference-point-unc-r15			AntennaReferencePointUnc-r15		OPTIONAL, -- Need ON

physical-reference-station-info-r15		PhysicalReferenceStationInfo-r15	OPTIONAL, -- Cond NP

...,

[[

equalIntegerAmbiguityLevel-r16			EqualIntegerAmbiguityLevel-r16		OPTIONAL -- Need ON

]]

}

AntennaDescription-r15 ::= SEQUENCE {

antennaDescriptor-r15					VisibleString (SIZE (1..256)),

antennaSetUpID-r15						ENUMERATED { non-zero }				OPTIONAL, -- Need OP

...

}

AntennaReferencePointUnc-r15 ::= SEQUENCE {

uncertainty-X-r15						INTEGER (0..255),

confidence-X-r15						INTEGER (0..100),

uncertainty-Y-r15						INTEGER (0..255),

confidence-Y-r15						INTEGER (0..100),

uncertainty-Z-r15						INTEGER (0..255),

confidence-Z-r15						INTEGER (0..100),

...

}

PhysicalReferenceStationInfo-r15 ::= SEQUENCE {

physicalReferenceStationID-r15			GNSS-ReferenceStationID-r15,

physical-ARP-ECEF-X-r15					INTEGER (-137438953472..137438953471),

physical-ARP-ECEF-Y-r15					INTEGER (-137438953472..137438953471),

physical-ARP-ECEF-Z-r15					INTEGER (-137438953472..137438953471),

physical-ARP-unc-r15					AntennaReferencePointUnc-r15		OPTIONAL, -- Need ON

...

}

EqualIntegerAmbiguityLevel-r16 ::= CHOICE {

allReferenceStations-r16				NULL,

referenceStationList-r16				ReferenceStationList-r16

}

ReferenceStationList-r16 ::= SEQUENCE (SIZE(1..16)) OF GNSS-ReferenceStationID-r15

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NP | The field is optionally present, need ON, if the referenceStationIndicator has the value ′non-physical′; otherwise it is not present. |

| GNSS-RTK-ReferenceStationInfo field descriptions |
| --- |
| referenceStationID The Reference Station ID is determined by the RTK service provider. |
| referenceStationIndicator This fields specifies type of reference station. Enumerated value physical indicates a real, physical reference station; value non-physical indicates a non-physical or computed reference station. |
| antenna-reference-point-ECEF-X This field specifies the antenna reference point X-coordinate in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| antenna-reference-point-ECEF-Y This field specifies the antenna reference point Y-coordinate in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| antenna-reference-point-ECEF-Z This field specifies the antenna reference point Z-coordinate in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| antennaHeight This field specifies the height of the Antenna Reference Point above the marker used in the survey campaign. Scale factor 0.0001 m; range 0–6.5535 m. |
| antennaDescriptor This field provides an ASCII descriptor of the reference station antenna using IGS naming convention [31]. The descriptor can be used to look up model specific phase centre corrections of that antenna. |
| antennaSetUpID This field, if present, indicates that the standard IGS Model is not valid (≠ 0 [30]). If this field is absent the standard IGS Model is valid (′0 = Use standard IGS Model′ [30]). |
| antenna-reference-point-unc This field specifies the uncertainty of the ARP coordinates. uncertainty-X, uncertainty-Y, and uncertainty-Z correspond to the encoded high accuracy uncertainty of the X, Y, and Z-coordinate, respectively, as defined in TS 23.032 [15]. confidence-X, confidence-Y, and confidence-Z corresponds to confidence as defined in TS 23.032 [15]. |
| physical-reference-station-info This field provides the earth-centred, earth-fixed (ECEF) coordinates of the antenna reference point (ARP) for the real (or "physical") reference station used. This field may be used in the case of the non-physical reference station approach to allow the target device to refer baseline vectors to a physical reference rather than to a non-physical reference without any connection to a physical point. |
| physicalReferenceStationID This field specifies the station ID of a real reference station, when the referenceStationIndicator has the value ′non-physical′. |
| physical-ARP-ECEF-X This field specifies the antenna reference point X-coordinate in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| physical-ARP-ECEF-Y This field specifies the antenna reference point Y-coordinate in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| physical-ARP-ECEF-Z This field specifies the antenna reference point Z-coordinate in the World Geodetic System 1984 (WGS 84) datum. Scale factor 0.0001 m; range ±13,743,895.3471 m. |
| physical-ARP-unc This field specifies the uncertainty of the ARP coordinates. |
| equalIntegerAmbiguityLevel This field specifies the integer ambiguity level of this reference station in relation to other reference stations. Either, the presence or absence of allReferenceStations indicates whether the integer ambiguity level may be assumed to be aligned between all reference stations or not (interpreted as no alignment is facilitated from the location server), or referenceStationList provides a list of reference stations for which the integer ambiguity level may be assumed to be the same. |  |

#### –	GNSS-RTK-CommonObservationInfo

The IE GNSS-RTK-CommonObservationInfo is used by the location server to provide common information applicable to the IE GNSS-RTK-Observations.

The parameters provided in IE GNSS-RTK-CommonObservationInfo are used as specified for message type 1071-1127 in [30].

-- ASN1START

GNSS-RTK-CommonObservationInfo-r15 ::= SEQUENCE {

referenceStationID-r15				GNSS-ReferenceStationID-r15,

clockSteeringIndicator-r15			INTEGER (0..3),

externalClockIndicator-r15			INTEGER (0..3),

smoothingIndicator-r15				BIT STRING (SIZE(1)),

smoothingInterval-r15				BIT STRING (SIZE(3)),

...

}

-- ASN1STOP

GNSS-RTK-CommonObservationInfo field descriptions

referenceStationID This field specifies the Station ID for which the GNSS-RTK-Observations are provided.

clockSteeringIndicator This field provides the clock steering indicator. The interpretation of the value is as follows: 0 clock steering is not applied In this case, the receiver clock must be kept in the range of ±1 ms (approximately ±300 km) 1 clock steering has been applied In this case, the receiver clock must be kept in the range of ±1 microsecond (approximately ±300 metres). 2 unknown clock steering status 3 reserved

externalClockIndicator This field provides the external clock indicator. The interpretation of the value is as follows: 0 internal clock is used 1 external clock is used, clock status is "locked" 2 external clock is used, clock status is "not locked", which may indicate external clock failure and that the transmitted data may not be reliable. 3 unknown clock is used

smoothingIndicator This field provides the GNSS Divergence-free Smoothing Indicator. The interpretation of the value is as follows: 1 Divergence-free smoothing is used 0 Other type of smoothing is used

smoothingInterval The GNSS Smoothing Interval is the integration period over which the pseudorange code phase measurements are averaged using carrier phase information. Divergence-free smoothing may be continuous over the entire period for which the satellite is visible. A value of zero indicates no smoothing is used. See table "smoothingInterval value to Smoothing Interval relation" below.

smoothingInterval value to Smoothing Interval relation

| smoothingInterval value | Smoothing Interval |
| --- | --- |
| 000 (0) | No smoothing |
| 001 (1) | < 30 s |
| 010 (2) | 30-60 s |
| 011 (3) | 1-2 min |
| 100 (4) | 2-4 min |
| 101 (5) | 4-8 min |
| 110 (6) | >8 min |
| 111 (7) | Unlimited smoothing interval |

#### –	GNSS-RTK-AuxiliaryStationData

The IE GNSS-RTK-AuxiliaryStationData is used by the location server to provide the coordinates of the antenna reference point (ARP) of Auxiliary Reference Stations, relative to the coordinates provided in IE GNSS-RTK-ReferenceStationInfo. The reference station provided in IE GNSS-RTK-ReferenceStationInfo is the Master Reference Station. Therefore, one Master Reference Station with its associated Auxiliary Stations is used in a single Provide Assistance Data message.

The parameters provided in IE GNSS-RTK-AuxiliaryStationData are used as specified for message type 1014 in [30].

-- ASN1START

GNSS-RTK-AuxiliaryStationData-r15 ::= SEQUENCE {

networkID-r15					GNSS-NetworkID-r15,

subNetworkID-r15				GNSS-SubNetworkID-r15					OPTIONAL,	-- Need ON

master-referenceStationID-r15	GNSS-ReferenceStationID-r15,

auxiliaryStationList-r15		AuxiliaryStationList-r15,

...

}

AuxiliaryStationList-r15 ::= SEQUENCE (SIZE (1..32)) OF AuxiliaryStationElement-r15

AuxiliaryStationElement-r15 ::= SEQUENCE {

aux-referenceStationID-r15				GNSS-ReferenceStationID-r15,

aux-master-delta-latitude-r15			INTEGER (-524288..524287),

aux-master-delta-longitude-r15			INTEGER (-1048576..1048575),

aux-master-delta-height-r15				INTEGER (-4194304..4194303),

aux-ARP-unc-r15							Aux-ARP-Unc-r15					OPTIONAL,	-- Need ON

...

}

Aux-ARP-Unc-r15 ::= SEQUENCE {

horizontalUncertainty-r15				INTEGER (0..255),

horizontalConfidence-r15				INTEGER (0..100),

verticalUncertainty-r15					INTEGER (0..255)				OPTIONAL,	-- Need ON

verticalConfidence-r15					INTEGER (0..100)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GNSS-RTK-AuxiliaryStationData field descriptions

networkID This field defines the network and the source of the particular set of reference stations and their observation information. The RTK service provider should ensure that the networkID is unique in the region serviced. The networkID indicates an area and its reference stations where the service providers will provide a homogenous solution with levelled integer ambiguities between its reference stations. In general, the area indicated by networkID will comprise one subnetwork with a unique subNetworkID.

subNetworkID This field identifies the subnetwork of a network identified by networkID. In general the area indicated by networkID will consist of one subnetwork. The subNetworkID indicates the actual solution number of integer ambiguity level. If one network has only one subnetwork, this indicates that an ambiguity level throughout the whole network is established.

master-referenceStationID This field identifies the Master Reference Station.

aux-referenceStationID This field identifies the Auxiliary Reference Station.

aux-master-delta-latitude This field provides the delta value in latitude of Antenna Reference Point of "Auxiliary Reference Station minus Master Reference Station" in geographical coordinates based on GRS80 ellipsoid parameters for the same ECEF system as used in IE GNSS-RTK-ReferenceStationInfo. Scale factor 25×10-6 degrees; range ±13.1071 degrees.

aux-master-delta-longitude This field provides the delta value in longitude of Antenna Reference Point of "Auxiliary Reference Station minus Master Reference Station" in geographical coordinates based on GRS80 ellipsoid parameters for the same ECEF system as used in IE GNSS-RTK-ReferenceStationInfo. Scale factor 25×10-6 degrees; range ±26.2142 degrees.

aux-master-delta-height This field provides the delta value in ellipsoidal height of Antenna Reference Point of "Auxiliary Reference Station minus Master Reference Station" in geographical coordinates based on GRS80 ellipsoid parameters for the same ECEF system as used in IE GNSS-RTK-ReferenceStationInfo. Scale factor 1 millimetre; range ±4194.303 m.

aux-ARP-unc This field specifies the uncertainty of the auxiliary station ARP coordinates and comprise the following fields: - horizontalUncertainty indicates the horizontal uncertainty of the ARP latitude/longitude. The ′horizontalUncertainty′ corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and ′horizontalConfidence′ corresponds to confidence as defined in TS 23.032 [15]. - verticalUncertainty indicates the vertical uncertainty of the ARP altitude. The 'verticalUncertainty' corresponds to the encoded high accuracy uncertainty as defined in TS 23.032 [15] and 'verticalConfidence' corresponds to confidence as defined in TS 23.032 [15].

#### –	GNSS-SSR-CorrectionPoints

The IE GNSS-SSR-CorrectionPoints is used by the location server to provide a list of correction point coordinates or an array of correction points ("grid") for which the GNSS-SSR-GriddedCorrection are valid.

-- ASN1START

GNSS-SSR-CorrectionPoints-r16 ::= SEQUENCE {

correctionPointSetID-r16			INTEGER (0..16383),

correctionPoints-r16				CHOICE {

listOfCorrectionPoints-r16			GNSS-SSR-ListOfCorrectionPoints-r16,

arrayOfCorrectionPoints-r16			GNSS-SSR-ArrayOfCorrectionPoints-r16

},

...

}

GNSS-SSR-ListOfCorrectionPoints-r16 ::= SEQUENCE {

referencePointLatitude-r16		INTEGER (-16384..16383),

referencePointLongitude-r16		INTEGER (-32768..32767),

relativeLocationsList-r16		SEQUENCE (SIZE (0..63)) OF RelativeLocationElement-r16,

...

}

RelativeLocationElement-r16 ::= SEQUENCE {

deltaLatitude-r16				INTEGER (-512..511),

deltaLongitude-r16				INTEGER (-1024..1023),

...

}

GNSS-SSR-ArrayOfCorrectionPoints-r16 ::=SEQUENCE {

referencePointLatitude-r16			INTEGER (-16384..16383),

referencePointLongitude-r16			INTEGER (-32768..32767),

numberOfStepsLatitude-r16			INTEGER (0..63),

numberOfStepsLongitude-r16			INTEGER (0..63),

stepOfLatitude-r16					INTEGER (1..511),

stepOfLongitude-r16					INTEGER (1..1023),

bitmaskOfGrids-r16					BIT STRING (SIZE(64))				OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

GNSS-SSR-CorrectionPoints field descriptions

correctionPointSetID This field provides the ID of the Atmospheric Correction Point set. It is a regionally unique arbitrary number that is used by the UE to ensure that the atmospheric corrections are being applied to the correct set of points.

referencePointLatitude This field specifies the latitude for the reference point, expressed in the range of -90° , +90°, coded as a number between -214 and 214-1, coded in 2's complement binary on 15 bits. The relation between the latitude X in the range [-90°, 90°] and the coded number N is: where denotes the greatest integer less than or equal to x (floor operator). For the listOfCorrectionPoints, the reference point defines the 1st correction point location. For the arrayOfCorrectionPoints, the reference point defines the northwest corner of the correction point array.

referencePointLongitude This field specifies the longitude for the reference point, expressed in the range -180°, +180°, coded as a number between -215 and 215-1, coded in 2's complement binary on 16 bits. The relation between the longitude X in the range [-180°, 180°) and the coded number N is: For the listOfCorrectionPoints, the reference point defines the 1st correction point location. For the arrayOfCorrectionPoints, the reference point defines the northwest corner of the correction point array.

relativeLocationsList This field specifies the 2nd, 3rd, …, 64th correction point location.

deltaLatitude This field specifies the delta value in latitude of this correction point location relative to the previous point on the list or the reference point in the case of the first additional point, defined as "correction point location" minus "previous correction point location" in units of 0.01 degrees.

deltaLongitude This field specifies the delta value in longitude of this correction point location relative to the previous point on the list or the reference point in the case of the first additional point, defined as "correction point location" minus "previous correction point location" in units of 0.01 degrees.

numberOfStepsLatitude, numberOfStepsLongitude These fields specify the number of steps for latitude and longitude direction respectively.

stepOfLatitude, stepOfLongitude These fields specify the spacing of the correction points for latitude and longitude respectively. The unit and scale factor is 0.01 degrees.

bitmaskOfGrids This field specifies the availability of correction data at the correction points in the array. If a specific bit is enabled (set to '1'), the correction is available. Only the first numberOfStepsLatitude×numberOfStepsLongitude bits are used, the remainder are set to '0'. Starting with the northwest corner of the array (top left on a north oriented map) the correction points are enumerated with row precedence – first row west to east, second row west to east, until last row west to east – ending with the southeast corner of the array. If the field is omitted all correction points are used and none omitted.

#### –	GNSS-Integrity-ServiceParameters

The IE GNSS-Integrity-ServiceParameters is used by the location server to provide the range of Integrity Risk (IR) for which the GNSS integrity assistance data are valid.

-- ASN1START

GNSS-Integrity-ServiceParameters-r17 ::= SEQUENCE {

irMinimum-r17						INTEGER (0..255),

irMaximum-r17						INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-Integrity-ServiceParameters field descriptions

irMinimum This field specifies the Minimum Integrity Risk (IR) which is the minimum IR for which the error bounds provided in the IEs TBD are valid. The IR is calculated by P=10-0.04n where n is the value of irMinimum and the range is 10-10.2 to 1.

irMaximum This field specifies the Maximum Integrity Risk (IR) which is the maximum IR for which the error bounds provided in the IEs TBD are valid. The IR is calculated by P=10-0.04n where n is the value of irMaximum and the range is 10-10.2 to 1.

#### –	GNSS-Integrity-ServiceAlert

The IE GNSS-Integrity-ServiceAlert is used by the location server to indicate whether the corresponding assistance data can be used for integrity related applications.

-- ASN1START

GNSS-Integrity-ServiceAlert-r17 ::= SEQUENCE {

ionosphereDoNotUse-r17				BOOLEAN,

troposphereDoNotUse-r17				BOOLEAN,

...

}

-- ASN1STOP

GNSS-Integrity-ServiceAlert field descriptions

ionosphereDoNotUse This field indicates whether the ionospheric corrections in IE GNSS-SSR-STEC-Correction can be used for integrity related applications (FALSE) or not (TRUE).

troposphereDoNotUse This field indicates whether the tropospheric corrections in IE GNSS-SSR-GriddedCorrection can be used for integrity related applications (FALSE) or not (TRUE).

Editor's Note: FFS on whether to also include a "Service DNU".

#### –	GNSS-TimeModelList

The IE GNSS-TimeModelList is used by the location server to provide the GNSS-GNSS system time offset between the GNSS system time indicated by IE GNSS-ID in IE GNSS-GenericAssistDataElement to the GNSS system time indicated by IE gnss-TO-ID. Several GNSS-TimeModelElement IEs can be included with different gnss-TO-ID fields. The location server should provide a GNSS-TimeModelList for the same GNSS-ID as the gnss-TimeID in IE GNSS-SystemTime in GNSS-ReferenceTime assistance. If the location server does not provide a GNSS-TimeModelList for the same GNSS-ID as the gnss-TimeID in IE GNSS-SystemTime in GNSS-ReferenceTime assistance the target device assumes tA1 and tA2 are equal to zero.

-- ASN1START

GNSS-TimeModelList ::= SEQUENCE (SIZE (1..15)) OF GNSS-TimeModelElement

GNSS-TimeModelElement ::= SEQUENCE {

gnss-TimeModelRefTime		INTEGER (0..65535),

tA0							INTEGER (-67108864..67108863),

tA1							INTEGER (-4096..4095)					OPTIONAL,	-- Need ON

tA2							INTEGER (-64..63)						OPTIONAL,	-- Need ON

gnss-TO-ID					INTEGER (1..15),

weekNumber					INTEGER (0..8191)						OPTIONAL,	-- Need ON

deltaT						INTEGER (-128..127)						OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GNSS-TimeModelElement field descriptions

gnss-TimeModelRefTime This field specifies the reference time of week for GNSS-TimeModelElement and it is given in GNSS specific system time. Scale factor 24 seconds.

tA0 This field specifies the bias coefficient of the GNSS-TimeModelElement. Scale factor 2-35 seconds.

tA1 This field specifies the drift coefficient of the GNSS-TimeModelElement. Scale factor of 2-51 seconds/second.

tA2 This field specifies the drift rate correction coefficient of the GNSS-TimeModelElement. Scale factor of 2-68 seconds/second2.

gnss-TO-ID This field specifies the GNSS system time of the GNSS for which the GNSS-TimeModelElement is applicable. GNSS-TimeModelElement contains parameters to convert GNSS system time from the system indicated by GNSS-ID to GNSS system time indicated by gnss-TO-ID. The conversion is defined in [4,5,6]. See table of gnss-TO-ID to Indication relation below. NOTE.

weekNumber This field specifies the reference week of the GNSS-TimeModelElement given in GNSS specific system time. The location server should include this field, if tA1 or tA2 is included. Scale factor 1 week.

deltaT This field specifies the integer number of seconds of the GNSS-GNSS time offset provided in the GNSS-TimeModelElement. Scale factor 1 second.

gnss-TO-ID to Indication relation

| Value of gnss-TO-ID | Indication |
| --- | --- |
| 1 | GPS |
| 2 | Galileo |
| 3 | QZSS |
| 4 | GLONASS |
| 5 | BDS |
| 6 | NavIC |
| 7-15 | reserved |

NOTE:	The time relationship between the system time indicated by GNSS-ID and system time indicated by gnss-TO-ID is given by the following equation:

tGNSS = tE - ( A0GGTO + A1GGTO (tE - tGGTO + 604800 (WN - WNGGTO)) + A2GGTO (tE - tGGTO + 
			604800 (WN - WNGGTO))2 )

where

tGNSS		is the system time of week for the GNSS indicated by gnss-TO-ID.
tE			is the system time of week for the GNSS indicated by GNSS-ID.
WN		is the week number of the GNSS system time indicated by GNSS-ID corresponding to the tE. 
tGGTO		is the system time of week for the time model data in the GNSS time indicated by GNSS-ID
			and given by the gnss-TimeModelRefTime field.
WNGGTO	is the week number for the time model data in the GNSS time indicated by GNSS-ID
			corresponding to the tGGTO and given by the weekNumber field.
A0GGTO	is given by the tA0 field.
A1GGTO	is given by the tA1 field.
A2GGTO	is given by the tA2 field.

If the tA1 and tA2 are not included in the GNSS-TimeModelElement, the target device assumes A1GGTO and A2GGTO are equal to zero.

The GNSS system times in the IE GNSS-TimeModelList and used in the equation above are all given in Time of Week (TOW) and Week Number (WN) in the indicted GNSS specific system time. For conversion between TOW/WN and Day Number/Time of Day (gnss-DayNumber/gnss-TimeOfDay) a GNSS week consists of 7 days since the origin of the particular GNSS System time (with the week number count starting at 0), and a day consists of 86400 seconds.

#### –	GNSS-DifferentialCorrections

The IE GNSS-DifferentialCorrections is used by the location server to provide differential GNSS corrections to the target device for a specific GNSS. Differential corrections can be provided for up to 3 signals per GNSS.

-- ASN1START

GNSS-DifferentialCorrections ::= SEQUENCE {

dgnss-RefTime		INTEGER (0..3599),

dgnss-SgnTypeList	DGNSS-SgnTypeList,

...

}

DGNSS-SgnTypeList ::= SEQUENCE (SIZE (1..3)) OF DGNSS-SgnTypeElement

DGNSS-SgnTypeElement ::= SEQUENCE {

gnss-SignalID		GNSS-SignalID,

gnss-StatusHealth	INTEGER (0..7),

dgnss-SatList		DGNSS-SatList,

...

}

DGNSS-SatList ::= SEQUENCE (SIZE (1..64)) OF DGNSS-CorrectionsElement

DGNSS-CorrectionsElement ::= SEQUENCE {

svID				SV-ID,

iod				BIT STRING (SIZE(11)),

udre				INTEGER (0..3),

pseudoRangeCor		INTEGER (-2047..2047),

rangeRateCor		INTEGER (-127..127),

udreGrowthRate		INTEGER (0..7)			OPTIONAL,	-- Need ON

udreValidityTime	INTEGER (0..7)			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GNSS-DifferentialCorrections field descriptions

dgnss-RefTime This field specifies the time for which the DGNSS corrections are valid, modulo 1 hour. dgnss-RefTime is given in GNSS specific system time. Scale factor 1-second.

dgnss-SgnTypeList This list includes differential correction data for different GNSS signal types, identified by GNSS-SignalID.

gnss-StatusHealth This field specifies the status of the differential corrections. The values of this field and their respective meanings are defined as in table gnss-StatusHealth Value to Indication relation below. The first six values in this field indicate valid differential corrections. When using the values described below, the "UDRE Scale Factor" value is applied to the UDRE values contained in the element. The purpose is to indicate an estimate in the amount of error in the corrections. The value "110" indicates that the source of the differential corrections (e.g., reference station or external DGNSS network) is currently not being monitored. The value "111" indicates that the corrections provided by the source are invalid, as judged by the source.

dgnss-SatList This list includes differential correction data for different GNSS satellites, identified by SV-ID.

iod This field specifies the Issue of Data field which contains the identity for the GNSS-NavigationModel.

udre This field provides an estimate of the uncertainty (1-) in the corrections for the particular satellite. The value in this field shall be multiplied by the UDRE Scale Factor in the gnss-StatusHealth field to determine the final UDRE estimate for the particular satellite. The meanings of the values for this field are shown in the table udre Value to Indication relation below.

pseudoRangeCor This field specifies the correction to the pseudorange for the particular satellite at dgnss-RefTime, t0. The value of this field is given in metres and the scale factor is 0.32 metres in the range of ±655.04 metres. The method of calculating this field is described in [11]. If the location server has received a request for GNSS assistance data from a target device which included a request for the GNSS Navigation Model and DGNSS, the location server shall determine, for each satellite, if the navigation model stored by the target device is still suitable for use with DGNSS corrections and if so and if DGNSS corrections are supported the location server should send DGNSS corrections without including the GNSS Navigation Model. The iod value sent for a satellite shall always be the IOD value that corresponds to the navigation model for which the pseudo-range corrections are applicable. The target device shall only use the pseudoRangeCor value when the IOD value received matches its available navigation model. Pseudo-range corrections are provided with respect to GNSS specific geodetic datum (e.g., PZ-90.02 if GNSS-ID indicates GLONASS). Scale factor 0.32 metres.

rangeRateCor This field specifies the rate-of-change of the pseudorange correction for the particular satellite, using the satellite ephemeris and clock corrections identified by the iod field. The value of this field is given in metres per second and the resolution is 0.032 metres/second in the range of ±4.064 metres/second. For some time t1 > t0, the corrections for iod are estimated by PRC(t1, IOD) = PRC(t0, IOD) + RRC(t0,IOD)(t1 - t0), and the target device uses this to correct the pseudorange it measures at t1, PRm(t1,IOD), by PR(t1, IOD) = PRm(t1, IOD) + PRC(t1, IOD) . The location server shall always send the RRC value that corresponds to the PRC value that it sends. The target device shall only use the RRC value when the iod value received matches its available navigation model. Scale factor 0.032 metres/second.

udreGrowthRate This field provides an estimate of the growth rate of uncertainty (1-) in the corrections for the particular satellite identified by SV-ID. The estimated UDRE at time value specified in the udreValidityTime t1 is calculated as follows: UDRE(t0+t1) = UDRE(t0) udreGrowthRate , where t0 is the DGNSS Reference Time dgnss-RefTime for which the corrections are valid, t1 is the udreValidityTime field, UDRE(t0) is the value of the udre field, and udreGrowthRate field is the factor as shown in the table Value of udreGrowthRate to Indication relation below.

udreValidityTime This field specifies the time when the udreGrowthRate field applies and is included if udreGrowthRate is included. The meaning of the values for this field is as shown in the table Value of udreValidityTime to Indication relation below.

gnss-StatusHealth Value to Indication relation

| gnss-StatusHealth Value | Indication |
| --- | --- |
| 000 | UDRE Scale Factor = 1.0 |
| 001 | UDRE Scale Factor = 0.75 |
| 010 | UDRE Scale Factor = 0.5 |
| 011 | UDRE Scale Factor = 0.3 |
| 100 | UDRE Scale Factor = 0.2 |
| 101 | UDRE Scale Factor = 0.1 |
| 110 | Reference Station Transmission Not Monitored |
| 111 | Data is invalid - disregard |

udre Value to Indication relation

| udre Value | Indication |
| --- | --- |
| 00 | UDRE 1.0 m |
| 01 | 1.0 m < UDRE 4.0 m |
| 10 | 4.0 m < UDRE 8.0 m |
| 11 | 8.0 m < UDRE |

Value of udreGrowthRate to Indication relation

| Value of udreGrowthRate | Indication |
| --- | --- |
| 000 | 1.5 |
| 001 | 2 |
| 010 | 4 |
| 011 | 6 |
| 100 | 8 |
| 101 | 10 |
| 110 | 12 |
| 111 | 16 |

Value of udreValidityTime to Indication relation

| Value of udreValidityTime | Indication [seconds] |
| --- | --- |
| 000 | 20 |
| 001 | 40 |
| 010 | 80 |
| 011 | 160 |
| 100 | 320 |
| 101 | 640 |
| 110 | 1280 |
| 111 | 2560 |

#### –	GNSS-NavigationModel

The IE GNSS-NavigationModel is used by the location server to provide precise navigation data to the GNSS capable target device. In response to a request from a target device for GNSS Assistance Data, the location server shall determine whether to send the navigation model for a particular satellite to a target device based upon factors like the T-Toe limit specified by the target device and any request from the target device for DGNSS (see also GNSS-DifferentialCorrections). GNSS Orbit Model can be given in Keplerian parameters or as state vector in Earth-Centered Earth-Fixed coordinates, dependent on the GNSS-ID and the target device capabilities. The meaning of these parameters is defined in relevant ICDs of the particular GNSS and GNSS specific interpretations apply. For example, GPS and QZSS use the same model parameters but some parameters have a different interpretation [7].

-- ASN1START

GNSS-NavigationModel ::= SEQUENCE {

nonBroadcastIndFlag		INTEGER (0..1),

gnss-SatelliteList		GNSS-NavModelSatelliteList,

...

}

GNSS-NavModelSatelliteList ::= SEQUENCE (SIZE(1..64)) OF GNSS-NavModelSatelliteElement

GNSS-NavModelSatelliteElement ::= SEQUENCE {

svID				SV-ID,

svHealth			BIT STRING (SIZE(8)),

iod					BIT STRING (SIZE(11)),

gnss-ClockModel		GNSS-ClockModel,

gnss-OrbitModel		GNSS-OrbitModel,

...,

[[	svHealthExt-v1240 BIT STRING (SIZE(4))			OPTIONAL		-- Need ON

]]

}

GNSS-ClockModel ::= CHOICE {

standardClockModelList	StandardClockModelList,			-- Model-1

nav-ClockModel			NAV-ClockModel,					-- Model-2

cnav-ClockModel			CNAV-ClockModel,				-- Model-3

glonass-ClockModel		GLONASS-ClockModel,				-- Model-4

sbas-ClockModel			SBAS-ClockModel,				-- Model-5

...,

bds-ClockModel-r12		BDS-ClockModel-r12,				-- Model-6

bds-ClockModel2-r16		BDS-ClockModel2-r16,			-- Model-7

navic-ClockModel-r16	NavIC-ClockModel-r16			-- Model-8

}

GNSS-OrbitModel ::= CHOICE {

keplerianSet			NavModelKeplerianSet,			-- Model-1

nav-KeplerianSet		NavModelNAV-KeplerianSet,		-- Model-2

cnav-KeplerianSet		NavModelCNAV-KeplerianSet,		-- Model-3

glonass-ECEF			NavModel-GLONASS-ECEF,			-- Model-4

sbas-ECEF				NavModel-SBAS-ECEF,				-- Model-5

...,

bds-KeplerianSet-r12	NavModel-BDS-KeplerianSet-r12,	-- Model-6

bds-KeplerianSet2-r16	NavModel-BDS-KeplerianSet2-r16,	-- Model-7

navic-KeplerianSet-r16	NavModel-NavIC-KeplerianSet-r16	-- Model-8

}

-- ASN1STOP

GNSS-NavigationModel field descriptions

nonBroadcastIndFlag This field indicates if the GNSS-NavigationModel elements are not derived from satellite broadcast data or are given in a format not native to the GNSS. A value of 0 means the GNSS-NavigationModel data elements correspond to GNSS satellite broadcasted data; a value of 1 means the GNSS-NavigationModel data elements are not derived from satellite broadcast.

gnss-SatelliteList This list provides ephemeris and clock corrections for GNSS satellites indicated by SV-ID.

svHealth This field specifies the satellite's current health. The health values are GNSS system specific. The interpretation of svHealth depends on the GNSS-ID and is as shown in table GNSS to svHealth Bit String(8) relation below.

iod This field specifies the Issue of Data and contains the identity for GNSS Navigation Model. In the case of broadcasted GPS NAV ephemeris, the iod contains the IODC as described in [4]. In the case of broadcasted Modernized GPS ephemeris, the iod contains the 11-bit parameter toe as defined in [4, Table 30-I] [6, Table 3.5-1]. In the case of broadcasted SBAS ephemeris, the iod contains the 8 bits Issue of Data as defined in [10] Message Type 9. In the case of broadcasted QZSS QZS-L1 ephemeris, the iod contains the IODC as described in [7]. In the case of broadcasted QZSS QZS-L1C/L2C/L5 ephemeris, the iod contains the 11-bit parameter toe as defined in [7]. In the case of broadcasted GLONASS ephemeris, the iod contains the parameter tb as defined in [9]. In the case of broadcasted Galileo ephemeris, the iod contains the IOD index as described in [8]. In the case of broadcasted BDS B1I/B3I ephemeris, the iod contains 11 MSB bits of the toe as defined in [23], [50]. In the case of broadcasted BDS B1C/B2a ephemeris, the iod contains the IODC as described in [39], [49]. In the case of broadcasted NavIC ephemeris, the iod contains 11 MSB bits of the toe as defined in [38]. The interpretation of iod depends on the GNSS-ID and is as shown in table GNSS to iod Bit String(11) relation below.

svHealthExt This field specifies the satellite's additional current health. The health values are GNSS system specific. The interpretation of svHealthExt depends on the GNSS-ID and is as shown in table GNSS to svHealthExt Bit String(4) relation below.

GNSS to svHealth Bit String(8) relation

| GNSS | svHealth Bit String(8) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| GPS L1/CA(1) | SV Health [4] |  |  |  |  |  | '0' (reserved) | '0' (reserved) |
| Modernized GPS(2) | L1C Health [6] | L1 Health [4,5] | L2 Health [4,5] | L5 Health [4,5] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| SBAS(3) | Ranging On (0),Off(1) [10] | Corrections On(0),Off(1) [10] | Integrity On(0),Off(1)[10] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| QZSS(4) QZS-L1 | SV Health [7] |  |  |  |  |  | '0' (reserved) | '0' (reserved) |
| QZSS(5) QZS- L1C/L2C/L5 | L1C Health [7] | L1 Health [7] | L2 Health [7] | L5 Health [7] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| GLONASS | Bn (MSB) [9, page 30] | FT [9, Table 4.4] |  |  |  | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| Galileo [8, clause 5.1.9.3] | E5a Data Validity Status | E5b Data Validity Status | E1-B Data Validity Status | E5a Signal Health Status |  | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| BDS(6) B1I [23] | B1I Health (SatH1) [23], [50] | B3I Health (SatH1) [23], [50] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| BDS(7) B1C [39]/B2a [49] | Sat Clock Health [39], [49] | B1C Health [39], [49] | B2a Health [39],[49] | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| NavIC | L5 health | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) | '0' (reserved) |
| Note 1: If GNSS-ID indicates 'gps', and GNSS Orbit Model-2 is included, this interpretation of svHealth applies. Note 2: If GNSS-ID indicates 'gps', and GNSS Orbit Model-3 is included, this interpretation of svHealth applies. If a certain signal is not supported on the satellite indicated by SV-ID, the corresponding health bit shall be set to '1' (i.e., signal can not be used). Note 3: svHealth, in the case that GNSS-ID indicates 'sbas', includes the 5 LSBs of the Health included in GEO Almanac Message Parameters (Type 17) [10]. Note 4: If GNSS-ID indicates 'qzss', and GNSS Orbit Model-2 is included, this interpretation of svHealth applies. Note 5: If GNSS-ID indicates 'qzss', and GNSS Orbit Model-3 is included, this interpretation of svHealth applies. Note 6: If GNSS-ID indicates 'bds', and GNSS Orbit Model-6 is included, this interpretation of svHealth applies. Note 7: If GNSS-ID indicates 'bds', and GNSS Orbit Model-7 is included, this interpretation of svHealth applies. |  |  |  |  |  |  |  |  |

GNSS to iod Bit String(11) relation

| GNSS | iod Bit String(11) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 | Bit 9 | Bit 10 | Bit 11 (LSB) |
| GPS L1/CA | '0' | Issue of Data, Clock [4] |  |  |  |  |  |  |  |  |  |
| Modernized GPS | toe (seconds, scale factor 300, range 0 – 604500) [4,5,6] |  |  |  |  |  |  |  |  |  |  |
| SBAS | '0' | '0' | '0' | Issue of Data ([10], Message Type 9) |  |  |  |  |  |  |  |
| QZSS QZS-L1 | '0' | Issue of Data, Clock [7] |  |  |  |  |  |  |  |  |  |
| QZSS QZS-L1C/L2C/L5 | toe (seconds, scale factor 300, range 0 – 604500) [7] |  |  |  |  |  |  |  |  |  |  |
| GLONASS | '0' | '0' | '0' | '0' | tb (minutes, scale factor 15) [9] |  |  |  |  |  |  |
| Galileo | '0' | IODnav [8] |  |  |  |  |  |  |  |  |  |
| BDS B1I/B3I | 11 MSB bits of toe (seconds, scale factor 512, range 0 – 604672) [23], [50] |  |  |  |  |  |  |  |  |  |  |
| BDS B1C/B2a | '0' | Issue of Data, Clock [39], [49] |  |  |  |  |  |  |  |  |  |
| NavIC | 11 MSB bits of toe (seconds, scale factor 512) [38] |  |  |  |  |  |  |  |  |  |  |

GNSS to svHealthExt Bit String(4) relation

| GNSS | svHealthExt Bit String(4) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 (LSB) |
| Galileo [8, clause 5.1.9.3] | E5b Signal Health Status |  | E1-B Signal Health Status |  |

#### –	StandardClockModelList

-- ASN1START

StandardClockModelList ::= SEQUENCE (SIZE(1..2)) OF StandardClockModelElement

StandardClockModelElement ::= SEQUENCE {

stanClockToc			INTEGER (0..16383),

stanClockAF2			INTEGER (-32..31),

stanClockAF1			INTEGER (-1048576..1048575),

stanClockAF0			INTEGER (-1073741824..1073741823),

stanClockTgd			INTEGER (-512..511)				OPTIONAL,	-- Need ON

sisa					INTEGER (0..255),

stanModelID				INTEGER (0..1)					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

StandardClockModelList field descriptions

standardClockModelList gnss-ClockModel Model-1 contains one or two clock model elements. If included, clock Model-1 shall be included once or twice depending on the target device capability. If the target device is supporting multiple Galileo signals, the location server shall include both F/Nav and I/Nav clock models in gnss-ClockModel if the location server assumes the target device to perform location information calculation using multiple signals.

stanClockToc Parameter toc defined in [8]. Scale factor 60 seconds.

stanClockAF2 Parameter af2 defined in [8]. Scale factor 2-59 seconds/second2.

stanClockAF1 Parameter af1 defined in [8]. Scale factor 2-46 seconds/second.

stanClockAF0 Parameter af0 defined in [8]. Scale factor 2-34 seconds.

stanClockTgd Parameter TGD, Broadcast Group Delay (BGD), defined in [8]. Scale factor 2-32 seconds. This field is required if the target device supports only single frequency Galileo signal.

sisa Signal-In-Space Accuracy (SISA), defined in [8] clause 5.1.11.

stanModelID This field specifies the identity of the clock model according to the table Value of stanModelID to Identity relation below. This field is required if the location server includes both F/Nav and I/Nav Galileo clock models in gnss-ClockModel.

Value of stanModelID to Identity relation

| Value of stanModelID | Identity |
| --- | --- |
| 0 | I/Nav (E1,E5b) |
| 1 | F/Nav (E1,E5a) |

#### –	NAV-ClockModel

-- ASN1START

NAV-ClockModel ::= SEQUENCE {

navToc			INTEGER (0..37799),

navaf2			INTEGER (-128..127),

navaf1			INTEGER (-32768..32767),

navaf0			INTEGER (-2097152..2097151),

navTgd			INTEGER (-128..127),

...

}

-- ASN1STOP

NAV-ClockModel field descriptions

navToc Parameter toc, time of clock (seconds) [4,7] Scale factor 24 seconds.

navaf2 Parameter af2, clock correction polynomial coefficient (sec/sec2) [4,7]. Scale factor 2-55 seconds/second2.

navaf1 Parameter af1, clock correction polynomial coefficient (sec/sec) [4,7]. Scale factor 2-43 seconds/second.

navaf0 Parameter af0, clock correction polynomial coefficient (seconds) [4,7]. Scale factor 2-31 seconds.

navTgd Parameter TGD, group delay (seconds) [4,7]. Scale factor 2-31 seconds.

#### –	CNAV-ClockModel

-- ASN1START

CNAV-ClockModel ::= SEQUENCE {

cnavToc			INTEGER (0..2015),

cnavTop			INTEGER (0..2015),

cnavURA0		INTEGER (-16..15),

cnavURA1		INTEGER (0..7),

cnavURA2		INTEGER (0..7),

cnavAf2			INTEGER (-512..511),

cnavAf1			INTEGER (-524288..524287),

cnavAf0			INTEGER (-33554432..33554431),

cnavTgd			INTEGER (-4096..4095),

cnavISCl1cp		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl1cd		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl1ca		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl2c		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl5i5		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

cnavISCl5q5		INTEGER (-4096..4095)			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

CNAV-ClockModel field descriptions

cnavToc Parameter toc, clock data reference time of week (seconds) [4,5,6,7]. Scale factor 300 seconds.

cnavTop Parameter top, clock data predict time of week (seconds) [4,5,6,7]. Scale factor 300 seconds

cnavURA0 Parameter URAoc Index, SV clock accuracy index (dimensionless) [4,5,6,7].

cnavURA1 Parameter URAoc1 Index, SV clock accuracy change index (dimensionless) [4,5,6,7].

cnavURA2 Parameter URAoc2 Index, SV clock accuracy change rate index (dimensionless) [4,5,6,7].

cnavAf2 Parameter af2-n, SV clock drift rate correction coefficient (sec/sec2) [4,5,6,7]. Scale factor 2-60 seconds/second2.

cnavAf1 Parameter af1-n, SV clock drift correction coefficient (sec/sec) [4,5,6,7]. Scale factor 2-48 seconds/second.

cnavAf0 Parameter af0-n, SV clock bias correction coefficient (seconds) [4,5,6,7]. Scale factor 2-35 seconds.

cnavTgd Parameter TGD, Group delay correction (seconds) [4,5,6,7]. Scale factor 2-35 seconds.

cnavISCl1cp Parameter ISCL1CP, inter signal group delay correction (seconds) [6,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L1C signal.

cnavISCl1cd Parameter ISCL1CD, inter signal group delay correction (seconds) [6,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L1C signal.

cnavISCl1ca Parameter ISCL1C/A, inter signal group delay correction (seconds) [4,5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L1CA signal.

cnavISCl2c Parameter ISCL2C, inter signal group delay correction (seconds) [4,5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L2C signal.

cnavISCl5i5 Parameter ISCL5I5, inter signal group delay correction (seconds) [5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L5 signal.

cnavISCl5q5 Parameter ISCL5Q5, inter signal group delay correction (seconds) [5,7]. Scale factor 2-35 seconds. The location server should include this field if the target device is GPS capable and supports the L5 signal.

#### –	GLONASS-ClockModel

-- ASN1START

GLONASS-ClockModel ::= SEQUENCE {

gloTau			INTEGER (-2097152..2097151),

gloGamma		INTEGER (-1024..1023),

gloDeltaTau		INTEGER (-16..15)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GLONASS-ClockModel field descriptions

gloTau Parameter n(tb), satellite clock offset (seconds) [9]. Scale factor 2-30 seconds.

gloGamma Parameter n(tb), relative frequency offset from nominal value (dimensionless) [9]. Scale factor 2-40.

gloDeltaTau Parameter n, time difference between transmission in G2 and G1 (seconds) [9]. Scale factor 2-30 seconds. The location server should include this parameter if the target device is dual frequency GLONASS receiver capable.

#### –	SBAS-ClockModel

-- ASN1START

SBAS-ClockModel ::= SEQUENCE {

sbasTo			INTEGER (0..5399),

sbasAgfo		INTEGER (-2048..2047),

sbasAgf1		INTEGER (-128..127),

...

}

-- ASN1STOP

SBAS-ClockModel field descriptions

sbasTo Parameter t0 [10]. Scale factor 16 seconds.

sbasAgfo Parameter aGfo [10]. Scale factor 2-31 seconds.

sbasAgf1 Parameter aGf1 [10]. Scale factor 2-40 seconds/second.

#### –	BDS-ClockModel

The IE BDS-ClockModel is used for BDS B1I defined in [23] and BDS B3I defined in [50].

-- ASN1START

BDS-ClockModel-r12 ::= SEQUENCE {

bdsAODC-r12			INTEGER (0..31),

bdsToc-r12			INTEGER (0..131071),

bdsA0-r12			INTEGER (-8388608..8388607),

bdsA1-r12			INTEGER (-2097152..2097151),

bdsA2-r12			INTEGER (-1024..1023),

bdsTgd1-r12			INTEGER (-512..511),

...,

[[ bdsTgd2-r16		INTEGER (-512..511)			OPTIONAL		-- Need ON

]]

}

-- ASN1STOP

BDS-ClockModel field descriptions

bdsAODC Parameter Age of Data, Clock (AODC), see [23], [50], Table 5-6.

bdsToc Parameter Toc, Time of clock (seconds) [23], [50]. Scale factor 23 seconds.

bdsA0 Parameter a0, Clock correction polynomial coefficient (seconds) [23], [50]. Scale factor 2-33 seconds.

bdsA1 Parameter a1, Clock correction polynomial coefficient (sec/sec) [23], [50]. Scale factor 2-50 sec/sec.

bdsA2 Parameter a2, Clock correction polynomial coefficient (sec/sec2) [23], [50]. Scale factor 2-66 sec/sec2.

bdsTgd1 Parameter Equipment group delay differential TGD1 [23], [50]. Scale factor is 0.1 nanosecond.

bdsTgd2 Parameter Equipment group delay differential TGD2 [23], [50]. Scale factor is 0.1 nanosecond.

#### –	BDS-ClockModel2

The IE BDS-ClockModel2 is used for BDS B1C defined in [39] and BDS B2a defined in [49].

-- ASN1START

BDS-ClockModel2-r16 ::= SEQUENCE {

bdsToc-r16		INTEGER (0..2047),

bdsA0-r16		INTEGER (-16777216..16777215),

bdsA1-r16		INTEGER (-2097152..2097151),

bdsA2-r16		INTEGER (-1024..1023),

bdsTgdB1Cp-r16	INTEGER (-2048..2047),

bdsIscB1Cd-r16	INTEGER (-2048..2047),

...,

[[	bdsTgdB2ap-r17	INTEGER (-2048..2047)				OPTIONAL,  -- Need ON

bdsIscB2ad-r17	INTEGER (-2048..2047)				OPTIONAL   -- Need ON

]]

}

-- ASN1STOP

BDS-ClockModel2 field descriptions

bdsToc Parameter Toc, Clock correction parameters reference time (seconds), see [39], 7.5.1 and [49], 7.5.1. Scale factor 300 seconds.

bdsA0 Parameter a0, Satellite clock time bias correction coefficient (seconds), see [39], 7.5.1 and [49], 7.5.1. Scale factor 2-34 seconds.

bdsA1 Parameter a1, Satellite clock time drift correction coefficient (sec/sec), see [39], 7.5.1 and [49], 7.5.1. Scale factor 2-50 sec/sec.

bdsA2 Parameter a2, Satellite clock time drift rate correction coefficient (sec/sec2), see [39], 7.5.1 and [49], 7.5.1. Scale factor 2-66 sec/sec2.

bdsTgdB1Cp Parameter TGDB1Cp Group delay differential of the B1C pilot component (seconds), see [39], 7.6.1 and [49], 7.6.1. Scale factor is 2-34 seconds.

bdsIscB1Cd Parameter ISCB1Cd Group delay differential between the B1C data and pilot components (seconds), see [39], 7.6.1. Scale factor is 2-34 seconds.

bdsTgdB2ap Parameter TGDB2ap Group delay differential of the B2a pilot component (seconds), see [39], 7.6.1 and [49], 7.6.1. Scale factor is 2-34 seconds.

bdsIscB2ad Parameter ISCB2ad Group delay differential between the B2a data and pilot components (seconds), see [49], 7.6.1. Scale factor is 2-34 seconds.

#### –	NavIC-ClockModel

-- ASN1START

NavIC-ClockModel-r16 ::= SEQUENCE {

navic-Toc-r16			INTEGER (0..65535),

navic-af2-r16			INTEGER (-128..127),

navic-af1-r16			INTEGER (-32768..32767),

navic-af0-r16			INTEGER (-2097152..2097151),

navic-Tgd-r16			INTEGER (-128..127),

...

}

-- ASN1STOP

NavIC-ClockModel field descriptions

navic-Toc Parameter toc, time of clock (seconds) [38], Table-11. Scale factor 24 seconds.

navic-af2 Parameter af2, clock correction polynomial coefficient (sec/sec2) [38]. Scale factor 2-55 seconds/second2.

navic-af1 Parameter af1, clock correction polynomial coefficient (sec/sec) [38]. Scale factor 2-43 seconds/second.

navic-af0 Parameter af0, clock correction polynomial coefficient (seconds) [38]. Scale factor 2-31 seconds.

navic-Tgd Parameter TGD, group delay (seconds) [38]. Scale factor 2-31 seconds.

#### –	NavModelKeplerianSet

-- ASN1START

NavModelKeplerianSet ::= SEQUENCE {

keplerToe		INTEGER (0 .. 16383),

keplerW			INTEGER (-2147483648..2147483647),

keplerDeltaN	INTEGER (-32768..32767),

keplerM0		INTEGER (-2147483648..2147483647),

keplerOmegaDot	INTEGER (-8388608.. 8388607),

keplerE		INTEGER (0..4294967295),

keplerIDot		INTEGER (-8192..8191),

keplerAPowerHalf INTEGER (0.. 4294967295),

keplerI0		INTEGER (-2147483648..2147483647),

keplerOmega0	INTEGER (-2147483648..2147483647),

keplerCrs		INTEGER (-32768..32767),

keplerCis		INTEGER (-32768..32767),

keplerCus		INTEGER (-32768..32767),

keplerCrc		INTEGER (-32768..32767),

keplerCic		INTEGER (-32768..32767),

keplerCuc		INTEGER (-32768..32767),

...

}

-- ASN1STOP

NavModelKeplerianSet field descriptions

keplerToe Parameter toe, time-of-ephemeris in seconds [8]. Scale factor 60 seconds.

keplerW Parameter , argument of perigee (semi-circles) [8]. Scale factor 2-31 semi-circles.

keplerDeltaN Parameter n, mean motion difference from computed value (semi-circles/sec) [8]. Scale factor 2-43 semi-circles/second.

keplerM0 Parameter M0, mean anomaly at reference time (semi-circles) [8]. Scale factor 2-31 semi-circles.

keplerOmegaDot Parameter OMEGAdot, rate of change of right ascension (semi-circles/sec) [8]. Scale factor 2-43 semi-circles/second.

keplerE Parameter e, eccentricity [8]. Scale factor 2-33.

KeplerIDot Parameter Idot, rate of change of inclination angle (semi-circles/sec) [8]. Scale factor 2-43 semi-circles/second.

keplerAPowerHalf Parameter sqrtA, square root of semi-major Axis in (metres) ½ [8]. Scale factor 2-19 metres½.

keplerI0 Parameter i0, inclination angle at reference time (semi-circles) [8]. Scale factor 2-31 semi-circles.

keplerOmega0 Parameter OMEGA0, longitude of ascending node of orbit plane at weekly epoch (semi-circles) [8]. Scale factor 2-31 semi-circles.

keplerCrs Parameter Crs, amplitude of the sine harmonic correction term to the orbit radius (metres) [8]. Scale factor 2-5 metres.

keplerCis Parameter Cis, amplitude of the sine harmonic correction term to the angle of inclination (radians) [8]. Scale factor 2-29 radians.

keplerCus Parameter Cus, amplitude of the sine harmonic correction term to the argument of latitude (radians) [8]. Scale factor 2-29 radians.

keplerCrc Parameter Crc, amplitude of the cosine harmonic correction term to the orbit radius (metres) [8]. Scale factor 2-5 metres.

keplerCic Parameter Cic, amplitude of the cosine harmonic correction term to the angle of inclination (radians) [8]. Scale factor 2-29 radians.

keplerCuc Parameter Cuc, amplitude of the cosine harmonic correction term to the argument of latitude (radians) [8]. Scale factor 2-29 radians.

#### –	NavModelNAV-KeplerianSet

-- ASN1START

NavModelNAV-KeplerianSet ::= SEQUENCE {

navURA			INTEGER (0..15),

navFitFlag		INTEGER (0..1),

navToe			INTEGER (0..37799),

navOmega		INTEGER (-2147483648..2147483647),

navDeltaN		INTEGER (-32768..32767),

navM0			INTEGER (-2147483648..2147483647),

navOmegaADot	INTEGER (-8388608..8388607),

navE			INTEGER (0..4294967295),

navIDot			INTEGER (-8192..8191),

navAPowerHalf	INTEGER (0..4294967295),

navI0			INTEGER (-2147483648..2147483647),

navOmegaA0		INTEGER (-2147483648..2147483647),

navCrs			INTEGER (-32768..32767),

navCis			INTEGER (-32768..32767),

navCus			INTEGER (-32768..32767),

navCrc			INTEGER (-32768..32767),

navCic			INTEGER (-32768..32767),

navCuc			INTEGER (-32768..32767),

addNAVparam		SEQUENCE {

ephemCodeOnL2	INTEGER (0..3),

ephemL2Pflag	INTEGER (0..1),

ephemSF1Rsvd	SEQUENCE {

reserved1		INTEGER (0..8388607),	-- 23-bit field

reserved2		INTEGER (0..16777215),	-- 24-bit field

reserved3		INTEGER (0..16777215),	-- 24-bit field

reserved4		INTEGER (0..65535)		-- 16-bit field

},

ephemAODA		INTEGER (0..31)

}	OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NavModelNAV-KeplerianSet field descriptions

navURA Parameter URA Index, SV accuracy (dimensionless) [4,7].

navFitFlag Parameter Fit Interval Flag, fit interval indication (dimensionless) [4,7]

navToe Parameter toe, time of ephemeris (seconds) [4,7]. Scale factor 24 seconds.

navOmega Parameter , argument of perigee (semi-circles) [4,7]. Scale factor 2-31 semi-circles.

navDeltaN Parameter n, mean motion difference from computed value (semi-circles/sec) [4,7]. Scale factor 2-43 semi-circles/second.

navM0 Parameter M0, mean anomaly at reference time (semi-circles) [4,7]. Scale factor 2-31 semi-circles.

navOmegaADot Parameter , rate of right ascension (semi-circles/sec) [4,7]. Scale factor 2-43 semi-circles/second.

navE Parameter e, eccentricity (dimensionless) [4,7]. Scale factor 2-33.

navIDot Parameter IDOT, rate of inclination angle (semi-circles/sec) [4,7]. Scale factor 2-43 semi-circles/second.

navAPowerHalf Parameter , square root of semi-major axis (metres/2) [4,7]. Scale factor 2-19 metres½.

navI0 Parameter i0, inclination angle at reference time (semi-circles) [4,7]. Scale factor 2-31 semi-circles.

navOmegaA0 Parameter 0, longitude of ascending node of orbit plane at weekly epoch (semi-circles) [4,7]. Scale factor 2-31 semi-circles.

navCrs Parameter Crs, amplitude of sine harmonic correction term to the orbit radius (metres) [4,7]. Scale factor 2-5 metres.

navCis Parameter Cis, amplitude of sine harmonic correction term to the angle of inclination (radians) [4,7]. Scale factor 2-29 radians.

navCus Parameter Cus, amplitude of sine harmonic correction term to the argument of latitude (radians) [4,7]. Scale factor 2-29 radians.

navCrc Parameter Crc, amplitude of cosine harmonic correction term to the orbit radius (metres) [4,7]. Scale factor 2-5 metres.

navCic Parameter Cic, amplitude of cosine harmonic correction term to the angle of inclination (radians) [4,7]. Scale factor 2-29 radians.

navCuc Parameter Cuc, amplitude of cosine harmonic correction term to the argument of latitude (radians) [4,7]. Scale factor 2-29 radians.

addNAVparam These fields include data and reserved bits in the GPS NAV message [4,14]. These additional navigation parameters, if provided by the location server, allow the target device to perform data wipe-off similar to what is done by the target device with the GNSS-DataBitAssistance.

#### –	NavModelCNAV-KeplerianSet

-- ASN1START

NavModelCNAV-KeplerianSet ::= SEQUENCE {

cnavTop				INTEGER (0..2015),

cnavURAindex		INTEGER (-16..15),

cnavDeltaA			INTEGER (-33554432..33554431),

cnavAdot			INTEGER (-16777216..16777215),

cnavDeltaNo			INTEGER (-65536..65535),

cnavDeltaNoDot		INTEGER (-4194304..4194303),

cnavMo				INTEGER (-4294967296..4294967295),

cnavE				INTEGER (0..8589934591),

cnavOmega			INTEGER (-4294967296..4294967295),

cnavOMEGA0			INTEGER (-4294967296..4294967295),

cnavDeltaOmegaDot	INTEGER (-65536..65535),

cnavIo				INTEGER (-4294967296..4294967295),

cnavIoDot			INTEGER (-16384..16383),

cnavCis				INTEGER (-32768..32767),

cnavCic				INTEGER (-32768..32767),

cnavCrs				INTEGER (-8388608..8388607),

cnavCrc				INTEGER (-8388608..8388607),

cnavCus				INTEGER (-1048576..1048575),

cnavCuc				INTEGER (-1048576..1048575),

...

}

-- ASN1STOP

NavModelCNAV-KeplerianSet field descriptions

cnavTop Parameter top, data predict time of week (seconds) [4,5,6,7]. Scale factor 300 seconds.

cnavURAindex Parameter URAoe Index, SV accuracy (dimensionless) [4,5,6,7].

cnavDeltaA Parameter A, semi-major axis difference at reference time (metres) [4,5,6,7]. Scale factor 2-9 metres.

cnavAdot Parameter , change rate in semi-major axis (metres/second) [4,5,6,7]. Scale factor 2-21 metres/second.

cnavDeltaNo Parameter n0, mean motion difference from computed value at reference time (semi-circles/sec) [4,5,6,7]. Scale factor 2-44 semi-circles/second.

cnavDeltaNoDot Parameter , rate of mean motion difference from computed value (semi-circles/sec2) [4,5,6,7]. Scale factor 2-57 semi-circles/second2.

cnavMo Parameter M0-n, mean anomaly at reference time (semi-circles) [4,5,6,7]. Scale factor 2-32 semi-circles.

cnavE Parameter en, eccentricity (dimensionless) [4,5,6,7]. Scale factor 2-34.

cnavOmega Parameter n, argument of perigee (semi-circles) [4,5,6,7]. Scale factor 2-32 semi-circles.

cnavOMEGA0 Parameter 0-n, reference right ascension angle (semi-circles) [4,5,6,7]. Scale factor 2-32 semi-circles.

cnavDeltaOmegaDot Parameter , rate of right ascension difference (semi-circles/sec) [4,5,6,7]. Scale factor 2-44 semi-circles/second.

cnavIo Parameter io-n, inclination angle at reference time (semi-circles) [4,5,6,7]. Scale factor 2-32 semi-circles.

cnavIoDot Parameter I0-n-DOT, rate of inclination angle (semi-circles/sec) [4,5,6,7]. Scale factor 2-44 semi-circles/second.

cnavCis Parameter Cis-n, amplitude of sine harmonic correction term to the angle of inclination (radians) [4,5,6,7]. Scale factor 2-30 radians.

cnavCic Parameter Cic-n, amplitude of cosine harmonic correction term to the angle of inclination (radians) [4,5,6,7]. Scale factor 2-30 radians.

cnavCrs Parameter Crs-n, amplitude of sine harmonic correction term to the orbit radius (metres) [4,5,6,7]. Scale factor 2-8 metres.

cnavCrc Parameter Crc-n, amplitude of cosine harmonic correction term to the orbit radius (metres) [4,5,6,7]. Scale factor 2-8 metres.

cnavCus Parameter Cus-n, amplitude of the sine harmonic correction term to the argument of latitude (radians) [4,5,6,7]. Scale factor 2-30 radians.

cnavCuc Parameter Cuc-n, amplitude of cosine harmonic correction term to the argument of latitude (radians) [4,5,6,7]. Scale factor 2-30 radians.

#### –	NavModel-GLONASS-ECEF

-- ASN1START

NavModel-GLONASS-ECEF ::= SEQUENCE {

gloEn				INTEGER (0..31),

gloP1				BIT STRING (SIZE(2)),

gloP2				BOOLEAN,

gloM				INTEGER (0..3),

gloX				INTEGER (-67108864..67108863),

gloXdot				INTEGER (-8388608..8388607),

gloXdotdot			INTEGER (-16..15),

gloY				INTEGER (-67108864..67108863),

gloYdot				INTEGER (-8388608..8388607),

gloYdotdot			INTEGER (-16..15),

gloZ				INTEGER (-67108864..67108863),

gloZdot				INTEGER (-8388608..8388607),

gloZdotdot			INTEGER (-16..15),

...

}

-- ASN1STOP

NavModel-GLONASS-ECEF field descriptions

gloEn Parameter En, age of data (days) [9]. Scale factor 1 days.

gloP1 Parameter P1, time interval between two adjacent values of tb (minutes) [9].

gloP2 Parameter P2, change of tb flag (dimensionless) [9].

gloM Parameter M, type of satellite (dimensionless) [9].

gloX Parameter , x-coordinate of satellite at time tb (kilometres) [9]. Scale factor 2-11 kilometres.

gloXdot Parameter , x-coordinate of satellite velocity at time tb (kilometres/second) [9]. Scale factor 2-20 kilometres/second.

gloXdotdot Parameter , x-coordinate of satellite acceleration at time tb (kilometres/second2) [9]. Scale factor 2-30 kilometres/second2.

gloY Parameter , y-coordinate of satellite at time tb (kilometres) [9]. Scale factor 2-11 kilometres.

gloYdot Parameter , y-coordinate of satellite velocity at time tb (kilometres/second) [9]. Scale factor 2-20 kilometres/second.

gloYdotdot Parameter , y-coordinate of satellite acceleration at time tb (kilometres/second2) [9]. Scale factor 2-30 kilometres/second2.

gloZ Parameter , z-coordinate of satellite at time tb (kilometres) [9]. Scale factor 2-11 kilometres.

gloZdot Parameter , z-coordinate of satellite velocity at time tb (kilometres/second) [9]. Scale factor 2-20 kilometres/second.

gloZdotdot Parameter , z-coordinate of satellite acceleration at time tb (kilometres/second2) [9]. Scale factor 2-30 kilometres/second2.

#### –	NavModel-SBAS-ECEF

-- ASN1START

NavModel-SBAS-ECEF ::= SEQUENCE {

sbasTo				INTEGER (0..5399)					OPTIONAL,	-- Cond ClockModel

sbasAccuracy		BIT STRING (SIZE(4)),

sbasXg				INTEGER (-536870912..536870911),

sbasYg				INTEGER (-536870912..536870911),

sbasZg				INTEGER (-16777216..16777215),

sbasXgDot			INTEGER (-65536..65535),

sbasYgDot			INTEGER (-65536..65535),

sbasZgDot			INTEGER (-131072..131071),

sbasXgDotDot		INTEGER (-512..511),

sbagYgDotDot		INTEGER (-512..511),

sbasZgDotDot		INTEGER (-512..511),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| ClockModel | This field is mandatory present if gnss-ClockModel Model-5 is not included; otherwise it is not present. |

NavModel-SBAS-ECEF field descriptions

sbasTo Parameter t0, time of applicability (seconds) [10]. Scale factor 16 seconds.

sbasAccuracy Parameter Accuracy, (dimensionless) [10].

sbasXg Parameter XG, (metres) [10]. Scale factor 0.08 metres.

sbasYg Parameter YG, (metres) [10]. Scale factor 0.08 metres.

sbasZg Parameter ZG, (metres) [10]. Scale factor 0.4 metres.

sbasXgDot Parameter XG, Rate-of-Change, (metres/second) [10]. Scale factor 0.000625 metres/second.

sbasYgDot Parameter YG, Rate-of-Change, (metres/second) [10] Scale factor 0.000625 metres/second.

sbasZgDot Parameter ZG, Rate-of-Change, (metres/second) [10]. Scale factor 0.004 metres/second.

sbasXgDotDot Parameter XG, Acceleration, (metres/second2) [10]. Scale factor 0.0000125 metres/second2.

sbagYgDotDot Parameter YG, Acceleration, (metres/second2) [10]. Scale factor 0.0000125 metres/second2.

sbasZgDotDot Parameter ZG Acceleration, (metres/second2) [10]. Scale factor 0.0000625 metres/second2.

#### –	NavModel-BDS-KeplerianSet

The IE NavModel-BDS-KeplerianSet is used for BDS B1I defined in [23], [50].

-- ASN1START

NavModel-BDS-KeplerianSet-r12 ::= SEQUENCE {

bdsAODE-r12				INTEGER (0..31),

bdsURAI-r12				INTEGER (0..15),

bdsToe-r12				INTEGER (0..131071),

bdsAPowerHalf-r12		INTEGER (0..4294967295),

bdsE-r12				INTEGER (0..4294967295),

bdsW-r12				INTEGER (-2147483648..2147483647),

bdsDeltaN-r12			INTEGER (-32768..32767),

bdsM0-r12				INTEGER (-2147483648..2147483647),

bdsOmega0-r12			INTEGER (-2147483648..2147483647),

bdsOmegaDot-r12			INTEGER (-8388608..8388607),

bdsI0-r12				INTEGER (-2147483648..2147483647),

bdsIDot-r12				INTEGER (-8192..8191),

bdsCuc-r12				INTEGER (-131072..131071),

bdsCus-r12				INTEGER (-131072..131071),

bdsCrc-r12				INTEGER (-131072..131071),

bdsCrs-r12				INTEGER (-131072..131071),

bdsCic-r12				INTEGER (-131072..131071),

bdsCis-r12				INTEGER (-131072..131071),

...

}

-- ASN1STOP

NavModel-BDS-KeplerianSet field descriptions

bdsAODE Parameter Age of Data, Ephemeris (AODE), see [23], [50], Table 5-8.

bdsURAI Parameter URA Index, URA is used to describe the signal-in-space accuracy in metres as defined in [23], [50].

bdsToe Parameter toe, Ephemeris reference time (seconds) [23], [50]. Scale factor 23 seconds.

bdsAPowerHalf Parameter A1/2, Square root of semi-major axis (metres/2) [23], [50]. Scale factor 2-19 metres/2.

bdsE Parameter e, Eccentricity, dimensionless [23], [50]. Scale factor 2-33.

bdsW Parameter , Argument of perigee (semi-circles) [23], [50]. Scale factor 2-31 semi-circles.

bdsDeltaN Parameter n, Mean motion difference from computed value (semi-circles/sec) [23], [50]. Scale factor 2-43 semi-circles/second.

bdsM0 Parameter M0, Mean anomaly at reference time (semi-circles) [23], [50]. Scale factor 2-31 semi-circles.

bdsOmega0 Parameter 0, Longitude of ascending node of orbital of plane computed according to reference time (semi-circles) [23], [50]. Scale factor 2-31 semi-circles.

bdsOmegaDot Parameter  Rate of right ascension (semi-circles/sec) [23], [50]. Scale factor 2-43 semi-circles/second.

bdsI0 Parameter i0, Inclination angle at reference time (semi-circles) [23], [50]. Scale factor 2-31 semi-circles.

bdsIDot Parameter Idot, Rate of inclination angle (semi-circles/sec) [23], [50]. Scale factor 2-43 semi-circles/second.

bdsCuc Parameter Cuc, Amplitude of cosine harmonic correction term to the argument of latitude (radians) [23], [50]. Scale factor 2-31 radians.

bdsCus Parameter Cus, Amplitude of sine harmonic correction term to the argument of latitude (radians) [23], [50]. Scale factor 2-31 radians.

bdsCrc Parameter Crc, Amplitude of cosine harmonic correction term to the orbit radius (metres) [23], [50]. Scale factor 2-6 metres.

bdsCrs Parameter Crs, Amplitude of sine harmonic correction term to the orbit radius (metres) [23], [50]. Scale factor 2-6 metres.

bdsCic Parameter Cic, Amplitude of cosine harmonic correction term to the angle of inclination (radians) [23], [50]. Scale factor 2-31 radians.

bdsCis Parameter Cis, Amplitude of sine harmonic correction term to the angle of inclination (radians) [23], [50]. Scale factor 2-31 radians.

#### –	NavModel-BDS-KeplerianSet2

The IE NavModel-BDS-KeplerianSet2 is used for BDS B1C and BDS B2a defined in [39], [49].

-- ASN1START

NavModel-BDS-KeplerianSet2-r16 ::= SEQUENCE {

bdsIODE-r16			INTEGER (0..255),

bdsToe-r16					INTEGER (0..2047),

bdsDeltaA-r16			INTEGER (-33554432..33554431),

bdsAdot-r16					INTEGER (-16777216..16777216),

bdsDeltaN0-r16			INTEGER (-65536..65535),

bdsDeltaN0dot-r16		INTEGER (-4194304..4194303),

bdsM0-r16					INTEGER (-4294967296..4294967295),

bdsE-r16					INTEGER (0..8589934591),

bdsOmega-r16			INTEGER (-4294967296..4294967295),

bdsOmega0-r16			INTEGER (-4294967296..4294967295),

bdsI0-r16					INTEGER (-4294967296..4294967295),

bdsOmegaDot-r16 		INTEGER (-262144..262143),

bdsI0Dot-r16			INTEGER (-16384..16383),

bdsCuc-r16				INTEGER (-1048576..1048575),

bdsCus-r16				INTEGER (-1048576..1048575),

bdsCrc-r16				INTEGER (-8388608..8388607),

bdsCrs-r16				INTEGER (-8388608..8388607),

bdsCic-r16				INTEGER (-32768..32767),

bdsCis-r16				INTEGER (-32768..32767),

...

}

-- ASN1STOP

NavModel-BDS-KeplerianSet2 field descriptions

bdsIODE Parameter, Issue Of Data, Ephemeris (IODE), see [39], 7.4.1 and [49], 7.4.1.

bdsToe Parameter toe, Ephemeris reference time (seconds), defined in [39], 7.7.1 and [49], 7.7.1. Scale factor 300 seconds.

bdsDeltaA Parameter A, Semi-major axis difference at reference time (metre), defined in [39], 7.7.1 and [49], 7.7.1. Scale factor 2-9 metres.

bdsAdot Parameter , Change rate in semi-major axis (metre/second), defined in [39], 7.7.1 and [49], 7.7.1. Scale factor 2-21 metre/second. The value 16777216 is not signalled.

bdsDeltaN0 Parameter n0, Mean motion difference from computed value at reference time (semi-circles /sec), defined in [39], 7.7.1 and [49], 7.7.1. Scale factor 2-44 semi-circles /second.

bdsDeltaN0dot Parameter n0dot, Rate of mean motion difference from computed value at reference time (semi-circles /sec2), defined in [39], 7.7.1 and [49], 7.7.1. Scale factor 2-57 semi-circles /second2.

bdsM0 Parameter M0, Mean anomaly at reference time (semi-circles) [39], [49]. Scale factor 2-32 semi-circles.

bdsE Parameter e, Eccentricity [39], [49]. Scale factor 2-34.

bdsOmega Parameter  Argument of perigee (semi-circles) [39], [49]. Scale factor 2-32 semi-circles.

bdsOmega0 Parameter0, Longitude of ascending node of orbital plane at weekly epoch (semi-circles) [39], [49]. Scale factor 2-32 semi-circles.

bdsI0 Parameter i0, Inclination angle at reference time (semi-circles) [39], [49]. Scale factor 2-32 semi-circles.

bdsOmegaDot Parameter , Rate of right ascension (semi-circles/sec) [39], [49]. Scale factor 2-44 semi-circles/second.

bdsI0Dot Parameter i0dot, Rate of inclination angle (semi-circles/sec) [39], [49]. Scale factor 2-44 semi-circles/second.

bdsCuc Parameter Cuc, Amplitude of cosine harmonic correction to the argument of latitude (radians) [39], [49]. Scale factor 2-30 radians.

bdsCus Parameter Cus, Amplitude of sine harmonic correction to the argument of latitude (radians) [39], [49]. Scale factor 2-30 radians.

bdsCrc Parameter Crc, Amplitude of cosine harmonic correction term to the orbit radius (metres) [39], [49]. Scale factor 2-8 metres.

bdsCrs Parameter Crs, Amplitude of sine harmonic correction term to the orbit radius (metres) [39], [49]. Scale factor 2-8 metres.

bdsCic Parameter Cic, Amplitude of cosine harmonic correction term to the angle of inclination (radians) [39], [49]. Scale factor 2-30 radians.

bdsCis Parameter Cis, Amplitude of sine harmonic correction term to the angle of inclination (radians) [39], [49]. Scale factor 2-30 radians.

#### –	NavModel-NavIC-KeplerianSet

-- ASN1START

NavModel-NavIC-KeplerianSet-r16	 ::= SEQUENCE {

navic-Toe-r16			INTEGER (0..65536),

navic-URAI-r16			INTEGER (0..15),

navic-W-r16				INTEGER (-2147483648..2147483647),

navic-DeltaN-r16		INTEGER (-2097152..2097151),

navic-M0-r16			INTEGER (-2147483648..2147483647),

navic-OmegaDot-r16		INTEGER (-2147483648..2147483647),

navic-E-r16				INTEGER (0..4294967295),

navic-IDot-r16			INTEGER (-8192..8191),

navic-APowerHalf-r16 	INTEGER (0.. 4294967295),

navic-I0-r16			INTEGER (-2147483648..2147483647),

navic-Omega0-r16		INTEGER (-2147483648..2147483647),

navic-Crs-r16			INTEGER (-32768..32767),

navic-Cis-r16			INTEGER (-32768..32767),

navic-Cus-r16			INTEGER (-32768..32767),

navic-Crc-r16			INTEGER (-32768..32767),

navic-Cic-r16			INTEGER (-32768..32767),

navic-Cuc-r16			INTEGER (-32768..32767),

...

}

-- ASN1STOP

NavModel-NavIC-KeplerianSet field descriptions

navic-Toe Parameter toe, time-of-ephemeris in seconds [38]. Scale factor 24 seconds.

navic-URAI Parameter User Range Accuracy Index (in metres). This is a one-sigma estimate of the user range errors in the navigation data for the transmitting satellite as described under clause 6.2.1.4 in [38]

navic-W Parameter , argument of perigee (semi-circles) [38]. Scale factor 2-31 semi-circles.

navic-DeltaN Parameter n, mean motion difference from computed value (semi-circles/sec) [38] Scale factor 2-41 semi-circles/second

navic-M0 Parameter M0, mean anomaly at reference time (semi-circles) [38] Scale factor 2-31 semi-circles.

navic-OmegaDot Parameter OMEGAdot, rate of change of right ascension (semi-circles/sec) [38] Scale factor 2-41 semi-circles/second

navic-E Parameter e, eccentricity [38] Scale factor 2-33.

navic-IDot Parameter Idot, rate of change of inclination angle (semi-circles/sec) [38] Scale factor 2-43 semi-circles/second.

navic-APowerHalf Parameter sqrtA, square root of semi-major Axis in (metres)½ [38] Scale factor 2-19 metres ½.

navic-I0 Parameter i0, inclination angle at reference time (semi-circles) [38] Scale factor 2-31 semi-circles.

navic-Omega0 Parameter OMEGA0, longitude of ascending node of orbit plane at weekly epoch (semi-circles) [38] Scale factor 2-31 semi-circles.

navic-Crs Parameter Crs, amplitude of the sine harmonic correction term to the orbit radius (metres) [38] Scale factor 2-4 metres

navic-Cis Parameter Cis, amplitude of the sine harmonic correction term to the angle of inclination (radians) [38] Scale factor 2-28 radians

navic-Cus Parameter Cus, amplitude of the sine harmonic correction term to the argument of latitude (radians) [38] Scale factor 2-28 radians

navic-Crc Parameter Crc, amplitude of the cosine harmonic correction term to the orbit radius (metres) [38] Scale factor 2-4 metres

navic-Cic Parameter Cic, amplitude of the cosine harmonic correction term to the angle of inclination (radians) [38] Scale factor 2-28 radians

navic-Cuc Parameter Cuc, amplitude of the cosine harmonic correction term to the argument of latitude (radians) [38] Scale factor 2-28 radians

#### –	GNSS-RealTimeIntegrity

The IE GNSS-RealTimeIntegrity is used by the location server to provide parameters that describe the real-time status of the GNSS constellations. GNSS-RealTimeIntegrity data communicates the health of the GNSS signals to the mobile in real-time.

The location server shall always transmit the GNSS-RealTimeIntegrity with the current list of unhealthy signals (i.e., not only for signals/SVs currently visible at the reference location), for any GNSS positioning attempt and whenever GNSS assistance data are sent. If the number of bad signals is zero, then the GNSS-RealTimeIntegrity IE shall be omitted.

NOTE 1:	If GNSS integrity assistance data are provided (i.e., any of GNSS-Integrity-ServiceParameters, GNSS-Integrity-ServiceAlert, ORBIT-IntegrityParameters, SSR-IntegrityOrbitBounds, CLOCK-IntegrityParameters, SSR-IntegrityClockBounds, SSR-IntegrityCodeBiasBounds, SSR-IntegrityPhaseBiasBounds, STEC-IntegrityParameters, STEC-IntegrityErrorBounds, SSR-GriddedCorrectionIntegrityParameters, TropoDelayIntegrityErrorBounds) the following interpretation of the IE GNSS-RealTimeIntegrity applies:

-	Absence of the IE GNSS-RealTimeIntegrity indicates DNU=FALSE according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40] for all GNSS satellites for which integrity assistance data are provided.

-	Presence of the IE GNSS-RealTimeIntegrity for a GNSS satellite and signal combination indicates DNU=TRUE for this GNSS satellite and signal combination according to the Integrity Principle of Operation specified in clause 8.1.1a of TS 38.305 [40].

NOTE 2:	The UE assumes that only those satellites for which the GNSS integrity assistance data are provided are monitored by the network and can be used for integrity related applications.

-- ASN1START

GNSS-RealTimeIntegrity ::= SEQUENCE {

gnss-BadSignalList	GNSS-BadSignalList,

...

}

GNSS-BadSignalList ::= SEQUENCE (SIZE(1..64)) OF BadSignalElement

BadSignalElement ::= SEQUENCE {

badSVID			SV-ID,

badSignalID		GNSS-SignalIDs	OPTIONAL,	-- Need OP

...

}

-- ASN1STOP

GNSS-RealTimeIntegrity field descriptions

gnss-BadSignalList This field specifies a list of satellites with bad signal or signals.

badSVID This field specifies the GNSS SV-ID of the satellite with bad signal or signals.

badSignalID This field identifies the bad signal or signals of a satellite. This is represented by a bit string in GNSS-SignalIDs, with a one-value at a bit position means the particular GNSS signal type of the SV is unhealthy; a zero-value means healthy. Absence of this field means that all signals on the specific SV are bad.

#### –	GNSS-DataBitAssistance

The IE GNSS-DataBitAssistance is used by the location server to provide data bit assistance data for specific satellite signals for data wipe-off. The data bits included in the assistance data depends on the GNSS and its signal.

-- ASN1START

GNSS-DataBitAssistance ::= SEQUENCE {

gnss-TOD				INTEGER (0..3599),

gnss-TODfrac			INTEGER (0..999)		OPTIONAL,	-- Need ON

gnss-DataBitsSatList	GNSS-DataBitsSatList,

...

}

GNSS-DataBitsSatList ::= SEQUENCE (SIZE(1..64))OF GNSS-DataBitsSatElement

GNSS-DataBitsSatElement ::= SEQUENCE {

svID					SV-ID,

gnss-DataBitsSgnList	GNSS-DataBitsSgnList,

...

}

GNSS-DataBitsSgnList ::= SEQUENCE (SIZE(1..8)) OF GNSS-DataBitsSgnElement

GNSS-DataBitsSgnElement ::= SEQUENCE {

gnss-SignalType			GNSS-SignalID,

gnss-DataBits			BIT STRING (SIZE (1..1024)),

...

}

-- ASN1STOP

GNSS-DataBitAssistance field descriptions

gnss-TOD This field specifies the reference time of the first bit of the data in GNSS-DataBitAssistance in integer seconds in GNSS specific system time, modulo 1 hour. Scale factor 1 second.

gnss-TODfrac This field specifies the fractional part of the gnss-TOD in 1-milli-second resolution. Scale factor 1 millisecond. The total GNSS TOD is gnss-TOD + gnss-TODfrac.

gnss-DataBitsSatList This list specifies the data bits for a particular GNSS satellite SV-ID and signal GNSS-SignalID.

svID This field specifies the GNSS SV-ID of the satellite for which the GNSS-DataBitAssistance is given.

gnss-SignalType This field identifies the GNSS signal type of the GNSS-DataBitAssistance.

gnss-DataBits Data bits are contained in GNSS system and data type specific format. In the case of GPS L1 C/A, it contains the NAV data modulation bits as defined in [4] . In the case of Modernized GPS L1C, it contains the encoded and interleaved modulation symbols as defined in [6] clause 3.2.3.1. In the case of Modernized GPS L2C, it contains either the NAV data modulation bits, the FEC encoded NAV data modulation symbols, or the FEC encoded CNAV data modulation symbols, dependent on the current signal configuration of this satellite as defined in [4, Table 3-III]. In the case of Modernized GPS L5, it contains the FEC encoded CNAV data modulation symbols as defined in [5]. In the case of SBAS, it contains the FEC encoded data modulation symbols as defined in [10]. In the case of QZSS QZS-L1, it contains the NAV data modulation bits as defined in [7] clause 5.2. In the case of QZSS QZS-L1C, it contains the encoded and interleaved modulation symbols as defined in [7] clause 5.3. In the case of QZSS QZS-L2C, it contains the encoded modulation symbols as defined in [7] clause 5.5. In the case of QZSS QZS-L5, it contains the encoded modulation symbols as defined in [7] clause 5.6. In the case of GLONASS, it contains the 100 sps differentially Manchester encoded modulation symbols as defined in [9] clause 3.3.2.2. In the case of Galileo, it contains the FEC encoded and interleaved modulation symbols. The logical levels 1 and 0 correspond to signal levels -1 and +1, respectively. In the case of BDS B1I, it contains the encoded and interleaved modulation symbols as defined in [23], clause 5.1.3. In the case of BDS B1C, it contains the encoded and interleaved modulation symbols as defined in [39], clause 6.2.2. In the case of BDS B2a, it contains the encoded and interleaved modulation symbols as defined in [49], clause 6.2.2. In the case of BDS B3I, it contains the encoded and interleaved modulation symbols as defined in [50], clause 5.1.3. In the case of NavIC, it contains the FEC encoded and interleaved Navigation symbols as defined in [38].

#### –	GNSS-AcquisitionAssistance

The IE GNSS-AcquisitionAssistance is used by the location server to provide parameters that enable fast acquisition of the GNSS signals. Essentially, these parameters describe the range and derivatives from respective satellites to the reference location at the reference time GNSS-SystemTime provided in IE GNSS-ReferenceTime.

Whenever GNSS-AcquisitionAssistance is provided by the location server, the IE GNSS-ReferenceTime shall be provided as well. E.g., even if the target device request for assistance data includes only a request for GNSS-AcquisitionAssistance, the location server shall also provide the corresponding IE GNSS-ReferenceTime.

Figure 6.5.2.2-1 illustrates the relation between some of the fields, using GPS TOW as exemplary reference.

-- ASN1START

GNSS-AcquisitionAssistance ::= SEQUENCE {

gnss-SignalID				GNSS-SignalID,

gnss-AcquisitionAssistList	GNSS-AcquisitionAssistList,

...,

confidence-r10				INTEGER (0..100)	OPTIONAL	-- Need ON

}

GNSS-AcquisitionAssistList ::= SEQUENCE (SIZE(1..64)) OF GNSS-AcquisitionAssistElement

GNSS-AcquisitionAssistElement ::= SEQUENCE {

svID						SV-ID,

doppler0					INTEGER (-2048..2047),

doppler1					INTEGER (0..63),

dopplerUncertainty			INTEGER (0..4),

codePhase					INTEGER (0..1022),

intCodePhase				INTEGER (0..127),

codePhaseSearchWindow		INTEGER (0..31),

azimuth						INTEGER (0..511),

elevation					INTEGER (0..127),

...,

codePhase1023				BOOLEAN				OPTIONAL,	-- Need OP

dopplerUncertaintyExt-r10	ENUMERATED {	d60,

d80,

d100,

d120,

noInformation, ... }	OPTIONAL	-- Need ON

}

-- ASN1STOP

GNSS-AcquisitionAssistance field descriptions

gnss-SignalID This field specifies the GNSS signal for which the acquisition assistance are provided.

gnss-AcquisitionAssistList These fields provide a list of acquisition assistance data for each GNSS satellite.

confidence This field specifies the confidence level of the reference location area or volume used to calculate the acquisition assistance parameters (search windows). A high percentage value (e.g., 98% or more) indicates to the target device that the provided search windows are reliable. The location server should include this field to indicate the confidence level of the provided information.

svID This field specifies the GNSS SV-ID of the satellite for which the GNSS-AcquisitionAssistance is given.

doppler0 This field specifies the Doppler (0th order term) value. A positive value in Doppler defines the increase in satellite signal frequency due to velocity towards the target device. A negative value in Doppler defines the decrease in satellite signal frequency due to velocity away from the target device. Doppler is given in unit of m/s by multiplying the Doppler value in Hz by the nominal wavelength of the assisted signal. Scale factor 0.5 m/s in the range from -1024 m/s to +1023.5 m/s.

doppler1 This field specifies the Doppler (1st order term) value. A positive value defines the rate of increase in satellite signal frequency due to acceleration towards the target device. A negative value defines the rate of decrease in satellite signal frequency due to acceleration away from the target device. Scale factor 1/210 m/s2 in the range from -0.2 m/s2 to +0.1 m/s2. Actual value of Doppler (1st order term) is calculated as (-42 + doppler1) * 1/210 m/s2, with doppler1 in the range of 0…63.

dopplerUncertainty This field specifies the Doppler uncertainty value. It is defined such that the Doppler experienced by a stationary target device is in the range [DopplerDoppler Uncertainty] to [DopplerDoppler Uncertainty]. Doppler Uncertainty is given in unit of m/s by multiplying the Doppler Uncertainty value in Hz by the nominal wavelength of the assisted signal. Defined values: 2.5 m/s, 5 m/s, 10 m/s, 20 m/s, 40 m/s as encoded by an integer n in the range 0-4 according to: 2-n(40) m/s; n = 0 – 4. If the dopplerUncertaintyExt field is present, the target device that supports the dopplerUncertaintyExt shall ignore this field.

codePhase This field together with the codePhase1023 field specifies the code phase, in units of milli-seconds, in the range from 0 to 1 millisecond scaled by the nominal chipping rate of the GNSS signal, where increasing values of the field signify increasing predicted signal code phases, as seen by a receiver at the reference location at the reference time. The reference location would typically be an a priori estimate of the target device location. Scale factor 2-10 ms in the range from 0 to (1-2-10) ms. Note: The value (1-2-10) ms is encoded using the codePhase1023 IE.

intCodePhase This field contains integer code phase (expressed modulo 128 ms). The satellite integer milli-seconds code phase currently being transmitted at the reference time, as seen by a receiver at the reference location is calculated as reference time (expressed in milli-seconds) minus (intCodePhase + (n×128 ms)), as shown in Figure 6.5.2.2-1, with n = …-2,-1,0,1,2…. Scale factor 1 ms in the range from 0 to 127 ms.

codePhaseSearchWindow This field contains the code phase search window. The code phase search window accounts for the uncertainty in the estimated target device location but not any uncertainty in reference time. It is defined such that the expected code phase is in the range [Code PhaseCode Phase Search Window] to [Code PhaseCode Phase Search Window] given in units of milli-seconds. Range 0-31, mapping according to the table codePhaseSearchWindow Value to Code Phase Search Window [ms] relation shown below.

azimuth This field specifies the azimuth angle. An angle of x degrees means the satellite azimuth a is in the range (x a < x+0.703125) degrees. Scale factor 0.703125 degrees.

elevation This field specifies the elevation angle. An angle of y degrees means the satellite elevation e is in the range (y e < y+0.703125) degrees. Scale factor 0.703125 degrees.

codePhase1023 This field if set to TRUE indicates that the code phase has the value 1023 × 2-10 = (1-2-10) ms. This field may only be set to TRUE if the value provided in the codePhase IE is 1022. If this field is set to FALSE, the code phase is the value provided in the codePhase IE in the range from 0 to (1 - 2×2-10) ms. If this field is not present and the codePhase IE has the value 1022, the target device may assume that the code phase is between (1 - 2×2-10) and (1 - 2-10) ms.

dopplerUncertaintyExt If this field is present, the target device that supports this field shall ignore the dopplerUncertainty field. The location server should include this field only if supported by the target device. This field specifies the Doppler uncertainty value. It is defined such that the Doppler experienced by a stationary target device is in the range [DopplerDoppler Uncertainty] to [DopplerDoppler Uncertainty]. Doppler Uncertainty is given in unit of m/s by multiplying the Doppler Uncertainty value in Hz by the nominal wavelength of the assisted signal. Enumerated values define 60 m/s, 80 m/s, 100 m/s, 120 m/s, and "No Information".

codePhaseSearchWindow Value to Code Phase Search Window [ms] relation

| codePhaseSearchWindow Value | Code Phase Search Window [ms] |
| --- | --- |
| '00000' | No information |
| '00001' | 0,002 |
| '00010' | 0,004 |
| '00011' | 0,008 |
| '00100' | 0,012 |
| '00101' | 0,016 |
| '00110' | 0,024 |
| '00111' | 0,032 |
| '01000' | 0,048 |
| '01001' | 0,064 |
| '01010' | 0,096 |
| '01011' | 0,128 |
| '01100' | 0,164 |
| '01101' | 0,200 |
| '01110' | 0,250 |
| '01111' | 0,300 |
| '10000' | 0,360 |
| '10001' | 0,420 |
| '10010' | 0,480 |
| '10011' | 0,540 |
| '10100' | 0,600 |
| '10101' | 0,660 |
| '10110' | 0,720 |
| '10111' | 0,780 |
| '11000' | 0,850 |
| '11001' | 1,000 |
| '11010' | 1,150 |
| '11011' | 1,300 |
| '11100' | 1,450 |
| '11101' | 1,600 |
| '11110' | 1,800 |
| '11111' | 2,000 |

Figure 6.5.2.2-1: Exemplary calculation of some GNSS Acquisition Assistance fields.

#### –	GNSS-Almanac

The IE GNSS-Almanac is used by the location server to provide the coarse, long-term model of the satellite positions and clocks. The meaning of these parameters is defined in relevant ICDs of the particular GNSS and GNSS specific interpretations apply. For example, GPS and QZSS use the same model parameters but some parameters have a different interpretation [7]. GNSS-Almanac is useful for receiver tasks that require coarse accuracy, such as determining satellite visibility. The model is valid for up to a few weeks, typically. Since it is a long-term model, the field should be provided for all satellites available in the GNSS constellation (i.e., not only for SVs visible at the reference location and including SVs flagged as unhealthy in almanac). The completeAlmanacProvided field indicates whether or not the location server provided almanacs for the complete GNSS constellation.

-- ASN1START

GNSS-Almanac ::= SEQUENCE {

weekNumber					INTEGER (0..255)	OPTIONAL,	-- Need ON

toa							INTEGER (0..255)	OPTIONAL,	-- Need ON

ioda						INTEGER (0..3)		OPTIONAL,	-- Need ON

completeAlmanacProvided		BOOLEAN,

gnss-AlmanacList			GNSS-AlmanacList,

...,

[[	toa-ext-v1240			INTEGER (256..1023)	OPTIONAL,	-- Need ON

ioda-ext-v1240			INTEGER (4..15)		OPTIONAL	-- Need ON

]],

[[

weekNumber-ext-r16		INTEGER (256..8191)			OPTIONAL,	-- Need ON

toa-ext2-r16			INTEGER (256..65535)		OPTIONAL	-- Need ON

]]

}

GNSS-AlmanacList ::= SEQUENCE (SIZE(1..64)) OF GNSS-AlmanacElement

GNSS-AlmanacElement ::= CHOICE {

keplerianAlmanacSet				AlmanacKeplerianSet,		-- Model-1

keplerianNAV-Almanac			AlmanacNAV-KeplerianSet,	-- Model-2

keplerianReducedAlmanac			AlmanacReducedKeplerianSet,	-- Model-3

keplerianMidiAlmanac			AlmanacMidiAlmanacSet,		-- Model-4

keplerianGLONASS				AlmanacGLONASS-AlmanacSet,	-- Model-5

ecef-SBAS-Almanac				AlmanacECEF-SBAS-AlmanacSet,-- Model-6

...,

keplerianBDS-Almanac-r12		AlmanacBDS-AlmanacSet-r12,	-- Model-7

keplerianNavIC-Almanac-r16		AlmanacNavIC-AlmanacSet-r16	-- Model-8

}

-- ASN1STOP

GNSS-Almanac field descriptions

weekNumber, weekNumber-ext This field specifies the almanac reference week number in GNSS specific system time to which the almanac reference time toa is referenced, modulo 256 weeks. Either weekNumber or weekNumber-ext is required for non-GLONASS GNSSs. In the case of Galileo, the almanac reference week number WNa natively contains only the 2 LSB's [8], clause 5.1.10]. In the case of BDS B1C and BDS B2a, the almanac reference week number is defined in [39], 7.9.1 and [49], 7.9.1. In the case of NavIC, the almanac reference week number is defined in [38].

toa, toa-ext, toa-ext2 In the cases that GNSS-ID does not indicate Galileo or NavIC, this field specifies the almanac reference time given in GNSS specific system time, in units of seconds with a scale factor of 212. toa is required for non-GLONASS GNSSs when the toa-ext2 is not present. In the case that GNSS-ID indicates Galileo, this field specifies the almanac reference time given in GNSS specific system time, in units of seconds with a scale factor of 600 seconds. Either toa or toa-ext is required for Galileo GNSS. In the case that GNSS-ID indicates NavIC, this field specifies the almanac reference time given in GNSS specific system time, in units of seconds with a scale factor of 16 seconds [38]. Either toa or toa-ext2 is required for NavIC GNSS.

ioda, ioda-ext This field specifies the issue of data. Either ioda or ioda-ext is required for Galileo GNSS.

completeAlmanacProvided If set to TRUE, the gnss-AlmanacList contains almanacs for the complete GNSS constellation indicated by GNSS-ID.

gnss-AlmanacList This list contains the almanac model for each GNSS satellite in the GNSS constellation.

#### –	AlmanacKeplerianSet

-- ASN1START

AlmanacKeplerianSet ::= SEQUENCE {

svID					SV-ID,

kepAlmanacE				INTEGER (0..2047),

kepAlmanacDeltaI		INTEGER (-1024..1023),

kepAlmanacOmegaDot		INTEGER (-1024..1023),

kepSV-StatusINAV		BIT STRING (SIZE (4)),

kepSV-StatusFNAV		BIT STRING (SIZE (2))			OPTIONAL,	-- Need ON

kepAlmanacAPowerHalf	INTEGER (-4096..4095),

kepAlmanacOmega0		INTEGER (-32768..32767),

kepAlmanacW				INTEGER (-32768..32767),

kepAlmanacM0			INTEGER (-32768..32767),

kepAlmanacAF0			INTEGER (-32768..32767),

kepAlmanacAF1			INTEGER (-4096..4095),

...

}

-- ASN1STOP

AlmanacKeplerianSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

kepAlmanacE Parameter e, eccentricity, dimensionless [8]. Scale factor 2-16.

kepAlmanacDeltaI Parameter i, inclination at reference time relative to i0=56°; semi-circles [8]. Scale factor 2-14 semi-circles.

kepAlmanacOmegaDot Parameter , rate of change of right ascension (semi-circles/sec) [8]. Scale factor 2-33 semi-circles/seconds.

kepSV-StatusINAV This field contains the I/NAV signal health status [8], clause 5.1.10 , E5bHS and E1-BHS, where E5bHS occupies the 2 MSBs in kepSV-StatusINAV, and E1-BHS the two LSBs.

kepSV-StatusFNAV This field contains the F/NAV signal health status [8], clause 5.1.10 ,E5aHS. If the target device is supporting multiple Galileo signals, the location server shall include this field.

kepAlmanacAPowerHalf Parameter (a1/2), difference with respect to the square root of the nominal semi-major axis, (metres)1/2 [8]. Scale factor 2-9 metres½ .

kepAlmanacOmega0 Parameter OMEGA0, longitude of ascending node of orbital plane at weekly epoch (semi-circles) [8]. Scale factor 2-15 semi-circles.

kepAlmanacW Parameter , argument of perigee (semi-circles) [8]. Scale factor 2-15 semi-circles.

kepAlmanacM0 Parameter M0, mean anomaly at reference time (semi-circles) [8]. Scale factor 2-15 semi-circles.

kepAlmanacAF0 Parameter af0, satellite clock correction bias, seconds [8]. Scale factor 2-19 seconds.

kepAlmanacAF1 Parameter af1, satellite clock correction linear, sec/sec [8]. Scale factor 2-38 seconds/second.

#### –	AlmanacNAV-KeplerianSet

-- ASN1START

AlmanacNAV-KeplerianSet ::= SEQUENCE {

svID					SV-ID,

navAlmE					INTEGER (0..65535),

navAlmDeltaI			INTEGER (-32768..32767),

navAlmOMEGADOT			INTEGER (-32768..32767),

navAlmSVHealth			INTEGER (0..255),

navAlmSqrtA				INTEGER (0..16777215),

navAlmOMEGAo			INTEGER (-8388608..8388607),

navAlmOmega				INTEGER (-8388608..8388607),

navAlmMo				INTEGER (-8388608..8388607),

navAlmaf0				INTEGER (-1024..1023),

navAlmaf1				INTEGER (-1024..1023),

...

}

-- ASN1STOP

AlmanacNAV-KeplerianSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

navAlmE Parameter e, eccentricity, dimensionless [4,7]. Scale factor 2-21.

navAlmDeltaI Parameter i, correction to inclination, semi-circles [4,7]. Scale factor 2-19 semi-circles.

navAlmOMEGADOT Parameter , rate of right ascension, semi-circles/sec [4,7]. Scale factor 2-38 semi-circles/second.

navAlmSVHealth Parameter SV Health, satellite health [4,7].

navAlmSqrtA Parameter , square root of the semi-major axis, metres/2 [4,7] Scale factor 2-11 metres/2.

navAlmOMEGAo Parameter 0, longitude of ascending node of orbit plane at weekly epoch, semi-circles [4,7]. Scale factor 2-23 semi-circles.

navAlmOmega Parameter , argument of perigee semi-circles [4,7]. Scale factor 2-23 semi-circles.

navAlmMo Parameter M0, mean anomaly at reference time semi-circles [4,7]. Scale factor 2-23 semi-circles.

navAlmaf0 Parameter af0, apparent satellite clock correction seconds [4,7]. Scale factor 2-20 seconds.

navAlmaf1 Parameter af1, apparent satellite clock correction sec/sec [4,7]. Scale factor 2-38 semi-circles seconds/second.

#### –	AlmanacReducedKeplerianSet

-- ASN1START

AlmanacReducedKeplerianSet ::= SEQUENCE {

svID					SV-ID,

redAlmDeltaA			INTEGER (-128..127),

redAlmOmega0			INTEGER (-64..63),

redAlmPhi0				INTEGER (-64..63),

redAlmL1Health			BOOLEAN,

redAlmL2Health			BOOLEAN,

redAlmL5Health			BOOLEAN,

...

}

-- ASN1STOP

AlmanacReducedKeplerianSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

redAlmDeltaA Parameter A, metres [4], [5], [6], [7], [39], [49]. Scale factor 2+9 metres.

redAlmOmega0 Parameter 0, semi-circles [4], [5], [6], [7], [39], [49]. Scale factor 2-6 semi-circles.

redAlmPhi0 Parameter 0, semi-circles [4], [5], [6], [7], [39], [49]. Scale factor 2-6 semi-circles.

redAlmL1Health Parameter L1 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the Satellite clock health state (the 8th bit) defined in table 7-14 [39] for BDS B1C and in table 7-14 [49] for BDS B2a.

redAlmL2Health Parameter L2 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B1C signal health state (the 7th bit) defined in table 7-14 [39] for BDS B1C and in table 7-14 [49] for BDS B2a.

redAlmL5Health Parameter L5 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B2a signal health state (the 6th bit) defined in table 7-14 [39] for BDS B1C and in table 7-14 [49] for BDS B2a.

#### –	AlmanacMidiAlmanacSet

-- ASN1START

AlmanacMidiAlmanacSet ::= SEQUENCE {

svID					SV-ID,

midiAlmE				INTEGER (0..2047),

midiAlmDeltaI			INTEGER (-1024..1023),

midiAlmOmegaDot			INTEGER (-1024..1023),

midiAlmSqrtA			INTEGER (0..131071),

midiAlmOmega0			INTEGER (-32768..32767),

midiAlmOmega			INTEGER (-32768..32767),

midiAlmMo				INTEGER (-32768..32767),

midiAlmaf0				INTEGER (-1024..1023),

midiAlmaf1				INTEGER (-512..511),

midiAlmL1Health			BOOLEAN,

midiAlmL2Health			BOOLEAN,

midiAlmL5Health			BOOLEAN,

...

}

-- ASN1STOP

AlmanacMidiAlmanacSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

midiAlmE Parameter e, dimensionless [4], [5], [6], [7], [39], [49]. Scale factor 2-16.

midiAlmDeltaI Parameter i, semi-circles [4], [5], [6], [7], [39], [49]. Scale factor 2-14 semi-circles.

midiAlmOmegaDot Parameter , semi-circles/sec [4], [5], [6], [7], [39], [49]. Scale factor 2-33 semi-circles/second.

midiAlmSqrtA Parameter , metres1/2 [4], [5], [6], [7], [39], [49]. Scale factor 2-4 metres1/2.

midiAlmOmega0 Parameter 0, semi-circles [4], [5], [6], [7], [39], [49]. Scale factor 2-15 semi-circles.

midiAlmOmega Parameter , semi-circles [4], [5], [6], [7], [39], [49]. Scale factor 2-15 semi-circles.

midiAlmMo Parameter M0, semi-circles [4], [5], [6], [7], [39, [49]]. Scale factor 2-15 semi-circles.

midiAlmaf0 Parameter afo, seconds [4], [5], [6], [7], [39], [49]. Scale factor 2-20 seconds.

midiAlmaf1 Parameter af1, sec/sec [4], [5], [6], [7], [39], [49]. Scale factor 2-37 seconds/second.

midiAlmL1Health Parameter L1 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the satellite clock health state (the 8th bit) defined in table 7-14 [39] for BDS B1C and in table 7-14 [49] for BDS B2a.

midiAlmL2Health Parameter L2 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B1C signal health state (the 7th bit) defined in table 7-14 [39] for BDS B1C and in table 7-14 [49] for BDS B2a.

midiAlmL5Health Parameter L5 Health, dimensionless [4], [5], [6], [7]. If GNSS-ID = BDS, this field indicates the B2a signal health state (the 6th bit) defined in table 7-14 [39] for BDS B1C and in table 7-14 [49] for BDS B2a.

#### –	AlmanacGLONASS-AlmanacSet

-- ASN1START

AlmanacGLONASS-AlmanacSet ::= SEQUENCE {

gloAlm-NA				INTEGER (1..1461),

gloAlmnA				INTEGER (1..24),

gloAlmHA				INTEGER (0..31),

gloAlmLambdaA			INTEGER (-1048576..1048575),

gloAlmtlambdaA			INTEGER (0..2097151),

gloAlmDeltaIa			INTEGER (-131072..131071),

gloAlmDeltaTA			INTEGER (-2097152..2097151),

gloAlmDeltaTdotA		INTEGER (-64..63),

gloAlmEpsilonA			INTEGER (0..32767),

gloAlmOmegaA			INTEGER (-32768..32767),

gloAlmTauA				INTEGER (-512..511),

gloAlmCA				INTEGER (0..1),

gloAlmMA				BIT STRING (SIZE(2))			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

AlmanacGLONASS-AlmanacSet field descriptions

gloAlm-NA Parameter NA, days [9]. Scale factor 1 days.

gloAlmnA Parameter nA, dimensionless [9].

gloAlmHA Parameter HnA, dimensionless [9].

gloAlmLambdaA Parameter nA, semi-circles [9]. Scale factor 2-20 semi-circles.

gloAlmtlambdaA Parameter tnA, seconds [9]. Scale factor 2-5 seconds.

gloAlmDeltaIa Parameter inA, semi-circles [9]. Scale factor 2-20 semi-circles.

gloAlmDeltaTA Parameter TnA, sec/orbit period [9]. Scale factor 2-9 seconds/orbit period.

gloAlmDeltaTdotA Parameter T_DOTnA, sec/orbit period2 [9]. Scale factor 2-14 seconds/orbit period2.

gloAlmEpsilonA Parameter nA, dimensionless [9]. Scale factor 2-20.

gloAlmOmegaA Parameter nA, semi-circles [9]. Scale factor 2-15 semi-circles.

gloAlmTauA Parameter nA, seconds [9]. Scale factor 2-18 seconds.

gloAlmCA Parameter CnA, dimensionless [9].

gloAlmMA Parameter MnA, dimensionless [9]. This parameter is present if its value is nonzero; otherwise it is not present.

#### –	AlmanacECEF-SBAS-AlmanacSet

-- ASN1START

AlmanacECEF-SBAS-AlmanacSet ::= SEQUENCE {

sbasAlmDataID			INTEGER (0..3),

svID					SV-ID,

sbasAlmHealth			BIT STRING (SIZE(8)),

sbasAlmXg				INTEGER (-16384..16383),

sbasAlmYg				INTEGER (-16384..16383),

sbasAlmZg				INTEGER (-256..255),

sbasAlmXgdot			INTEGER (-4..3),

sbasAlmYgDot			INTEGER (-4..3),

sbasAlmZgDot			INTEGER (-8..7),

sbasAlmTo				INTEGER (0..2047),

...

}

-- ASN1STOP

AlmanacECEF-SBAS-AlmanacSet field descriptions

sbasAlmDataID Parameter Data ID, dimensionless [10].

svID This field identifies the satellite for which the GNSS Almanac Model is given.

sbasAlmHealth Parameter Health, dimensionless [10].

sbasAlmXg Parameter XG, metres [10]. Scale factor 2600 metres.

sbasAlmYg Parameter YG, metres [10]. Scale factor 2600 metres.

sbasAlmZg Parameter ZG, metres [10]. Scale factor 26000 metres.

sbasAlmXgdot Parameter XG Rat-of-Change, metres/second [10]. Scale factor 10 metres/second.

sbasAlmYgDot Parameter YG Rate-of-Change, metres/second [10]. Scale factor 10 metres/second.

sbasAlmZgDot Parameter ZG Rate-of-Change, metres/second [10]. Scale factor 40.96 metres/second.

sbasAlmTo Parameter t0, seconds [10]. Scale factor 64 metres/second.

#### –	AlmanacBDS-AlmanacSet

-- ASN1START

AlmanacBDS-AlmanacSet-r12 ::= SEQUENCE {

svID					SV-ID,

bdsAlmToa-r12			INTEGER (0..255)					OPTIONAL,	-- Cond NotSameForAllSV

bdsAlmSqrtA-r12			INTEGER (0..16777215),

bdsAlmE-r12				INTEGER (0..131071),

bdsAlmW-r12				INTEGER (-8388608..8388607),

bdsAlmM0-r12			INTEGER (-8388608..8388607),

bdsAlmOmega0-r12		INTEGER (-8388608..8388607),

bdsAlmOmegaDot-r12		INTEGER (-65536..65535),

bdsAlmDeltaI-r12		INTEGER (-32768..32767),

bdsAlmA0-r12			INTEGER (-1024..1023),

bdsAlmA1-r12			INTEGER (-1024..1023),

bdsSvHealth-r12			BIT STRING (SIZE(9))				OPTIONAL,	-- Cond SV-ID

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameForAllSV | This field may be present if the toa is not the same for all SVs; otherwise it is not present and the toa is provided in GNSS-Almanac. |
| SV-ID | This field is mandatory present if SV-ID is between 0 and 63; otherwise it is not present. |

AlmanacBDS-AlmanacSet field descriptions

svID This field identifies the satellite for which the GNSS Almanac Model is given.

bdsAlmToa Parameter toa, Almanac reference time (seconds) [23], [50]. Scale factor 212 seconds.

bdsAlmSqrtA Parameter A1/2, Square root of semi-major axis (metres1/2) [23], [50]. Scale factor 2-11 metres1/2.

bdsAlmE Parameter e, Eccentricity, dimensionless [23], [50]. Scale factor 2-21.

bdsAlmW Parameter , Argument of Perigee (semi-circles) [23], [50]. Scale factor 2-23 semi-circles.

bdsAlmM0 Parameter M0, Mean anomaly at reference time (semi-circles) [23], [50]. Scale factor 2-23 semi-circles.

bdsAlmOmega0 Parameter 0, Longitude of ascending node of orbital plane computed according to reference time (semi-circles) [23], [50]. Scale factor 2-23 semi-circles.

bdsAlmOmegaDot Parameter  Rate of right ascension (semi-circles/second) [23], [50]. Scale factor 2-38 semi-circles/second.

bdsAlmDeltaI Parameter i, Correction of orbit reference inclination at reference time (semi-circles) [23], [50]. Scale factor 2-19 semi-circles.

bdsAlmA0 Parameter a0, Satellite clock bias (seconds) [23], [50]. Scale factor 2-20 seconds.

bdsAlmA1 Parameter a1, Satellite clock rate (sec/sec) [23], [50]. Scale factor 2-38 seconds/seconds.

bdsSvHealth This field indicates satellites health information as defined in [23], [50] Table 5-16. The left most bit is the MSB.

#### –	AlmanacNavIC-AlmanacSet

-- ASN1START

AlmanacNavIC-AlmanacSet-r16	 ::= SEQUENCE {

svID-r16					SV-ID,

navic-AlmToa-r16			INTEGER (0..65535)			OPTIONAL,	-- Cond NotSameForAllSV

navic-AlmE-r16				INTEGER (0..65535),

navic-AlmOMEGADOT-r16		INTEGER (-32768..32767),

navic-AlmSqrtA-r16			INTEGER (0..16777215),

navic-AlmOMEGAo-r16			INTEGER (-8388608..8388607),

navic-AlmOmega-r16			INTEGER (-8388608..8388607),

navic-AlmMo-r16				INTEGER (-8388608..8388607),

navic-Almaf0-r16			INTEGER (-1024..1023),

navic-Almaf1-r16			INTEGER (-1024..1023),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| NotSameForAllSV | This field is optionally present, need ON, if the toa is not the same for all SVs; otherwise it is not present and the toa is provided in GNSS-Almanac. |

AlmanacNavIC-AlmanacSet field descriptions

svID This field identifies the satellite for which the Almanac model is given

navic-AlmToa This field provides the time of almanac set Scale factor 16 seconds.

navic-AlmE Parameter e, eccentricity, dimensionless Scale factor 2-21.

navic-AlmOMEGADOT Parameter , rate of right ascension, semi-circles/sec Scale factor 2-38 semi-circles/second

navic-AlmSqrtA Parameter , square root of the semi-major axis, metres1/2 Scale factor 2-11 metres1/2.

navic-AlmOMEGAo Parameter 0, longitude of ascending node of orbit plane at weekly epoch, semi-circles Scale factor 2-23 semi-circles.

navic-AlmOmega Parameter , argument of perigee semi-circles Scale factor 2-23 semi-circles.

navic-AlmMo Parameter M0, mean anomaly at reference time semi-circles Scale factor 2-23 semi-circles.

navic-Almaf0 Parameter af0, apparent satellite clock correction seconds Scale factor 2-20 seconds.

navic-Almaf1 Parameter af1, apparent satellite clock correction sec/sec Scale factor 2-38 semi-circles seconds/second.

#### –	GNSS-UTC-Model

The IE GNSS-UTC-Model is used by the location server to provide several sets of parameters needed to relate GNSS system time to Universal Time Coordinate (UTC), as defined in [4], [5], [6], [7], [8], [9], [10], [23], [38], [39], [49].

The UTC time standard, UTC(k), is GNSS specific. E.g., if GNSS-ID indicates GPS, GNSS-UTC-Model contains a set of parameters needed to relate GPS system time to UTC(USNO); if GNSS-ID indicates QZSS, GNSS-UTC-Model contains a set of parameters needed to relate QZST to UTC(NICT); if GNSS-ID indicates GLONASS, GNSS-UTC-Model contains a set of parameters needed to relate GLONASS system time to UTC(RU); if GNSS-ID indicates SBAS, GNSS-UTC-Model contains a set of parameters needed to relate SBAS network time for the SBAS indicated by SBAS-ID to the UTC standard defined by the UTC Standard ID; if GNSS-ID indicates BDS, GNSS-UTC-Model contains a set of parameters needed to relate BDS system time to UTC (NTSC), where UTC-ModelSet2 is used for BDS B1C and BDS B2a, and UTC-ModelSet5 is used for BDS B1I; if the GNSS-ID indicates NavIC, the GNSS-UTC-Model contains a set of parameters needed to relate NavIC system time to the UTC (BIPM).

-- ASN1START

GNSS-UTC-Model ::= CHOICE {

utcModel1			UTC-ModelSet1,			-- Model-1

utcModel2			UTC-ModelSet2,			-- Model-2

utcModel3			UTC-ModelSet3,			-- Model-3

utcModel4			UTC-ModelSet4,			-- Model-4

...,

utcModel5-r12		UTC-ModelSet5-r12		-- Model-5

}

-- ASN1STOP

#### –	UTC-ModelSet1

-- ASN1START

UTC-ModelSet1 ::= SEQUENCE {

gnss-Utc-A1			INTEGER (-8388608..8388607),

gnss-Utc-A0			INTEGER (-2147483648..2147483647),

gnss-Utc-Tot		INTEGER (0..255),

gnss-Utc-WNt		INTEGER (0..255),

gnss-Utc-DeltaTls	INTEGER (-128..127),

gnss-Utc-WNlsf		INTEGER (0..255),

gnss-Utc-DN			INTEGER (-128..127),

gnss-Utc-DeltaTlsf	INTEGER (-128..127),

...

}

-- ASN1STOP

UTC-ModelSet1 field descriptions

gnss-Utc-A1 Parameter A1, scale factor 2-50 seconds/second [4,7,8].

gnss-Utc-A0 Parameter A0, scale factor 2-30 seconds [4,7,8].

gnss-Utc-Tot Parameter tot, scale factor 212 seconds [4,7,8].

gnss-Utc-WNt Parameter WNt, scale factor 1 week [4,7,8].

gnss-Utc-DeltaTls Parameter tLS, scale factor 1 second [4,7,8].

gnss-Utc-WNlsf Parameter WNLSF, scale factor 1 week [4,7,8].

gnss-Utc-DN Parameter DN, scale factor 1 day [4,7,8].

gnss-Utc-DeltaTlsf Parameter tLSF, scale factor 1 second [4,7,8].

#### –	UTC-ModelSet2

-- ASN1START

UTC-ModelSet2 ::= SEQUENCE {

utcA0				INTEGER (-32768..32767),

utcA1				INTEGER (-4096..4095),

utcA2				INTEGER (-64..63),

utcDeltaTls			INTEGER (-128..127),

utcTot				INTEGER (0..65535),

utcWNot				INTEGER (0..8191),

utcWNlsf			INTEGER (0..255),

utcDN				BIT STRING (SIZE(4)),

utcDeltaTlsf		INTEGER (-128..127),

...,

[[

utcWNlsf-ext-r16	INTEGER (256..8191)	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

UTC-ModelSet2 field descriptions

utcA0 Parameter A0-n, bias coefficient of GNSS time scale relative to UTC time scale (seconds) [4], [5], [6], [7], [38], [39], [49]. Scale factor 2-35 seconds.

utcA1 Parameter A1-n, drift coefficient of GNSS time scale relative to UTC time scale (sec/sec) [4], [5], [6], [7], [38], [39], [49]. Scale factor 2-51 seconds/second.

utcA2 Parameter A2-n, drift rate correction coefficient of GNSS time scale relative to UTC time scale (sec/sec2) [4], [5], [6], [7], [38], [39], [49]. Scale factor 2-68 seconds/second2.

utcDeltaTls Parameter tLS, current or past leap second count (seconds) [4], [5], [6], [7], [38], [39], [49]. Scale factor 1 second.

utcTot Parameter tot, time data reference time of week (seconds) [4], [5], [6], [7], [38], [39], [49]. Scale factor 24 seconds.

utcWNot Parameter WNot, time data reference week number (weeks) [4], [5], [6], [7], [38], [39], [49]. Scale factor 1 week.

utcWNlsf, utcWNlsf-ext Parameter WNLSF, leap second reference week number (weeks) [4], [5], [6], [7], [38], [39], [49]. If the field utcWNlsf-ext is present, the field utcWNlsf shall be ignored by the receiver. Scale factor 1 week.

utcDN Parameter DN, leap second reference day number (days) [4], [5], [6], [7], [38], [39], [49]. Scale factor 1 day.

utcDeltaTlsf Parameter tLSF, current or future leap second count (seconds) [4], [5], [6], [7], [38], [39], [49]. Scale factor 1 second.

#### –	UTC-ModelSet3

-- ASN1START

UTC-ModelSet3 ::= SEQUENCE {

nA					INTEGER (1..1461),

tauC				INTEGER (-2147483648..2147483647),

b1					INTEGER (-1024..1023)					OPTIONAL,	-- Cond GLONASS-M

b2					INTEGER (-512..511)						OPTIONAL,	-- Cond GLONASS-M

kp					BIT STRING (SIZE(2))					OPTIONAL,	-- Cond GLONASS-M

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GLONASS-M | The field is mandatory present if GLONASS-M satellites are present in the current GLONASS constellation; otherwise it is not present. |

UTC-ModelSet3 field descriptions

nA Parameter NA, calendar day number within four-year period beginning since the leap year (days) [9]. Scale factor 1 day.

tauC Parameter c, GLONASS time scale correction to UTC(SU) (seconds) [9]. Scale factor 2-31 seconds.

b1 Parameter B1, coefficient to determine UT1 (seconds) [9]. Scale factor 2-10 seconds.

b2 Parameter B2, coefficient to determine UT1 (seconds/msd) [9]. Scale factor 2-16 seconds/msd.

kp Parameter KP, notification of expected leap second correction (dimensionless) [9].

#### –	UTC-ModelSet4

-- ASN1START

UTC-ModelSet4 ::= SEQUENCE {

utcA1wnt			INTEGER (-8388608..8388607),

utcA0wnt			INTEGER (-2147483648..2147483647),

utcTot				INTEGER (0..255),

utcWNt				INTEGER (0..255),

utcDeltaTls			INTEGER (-128..127),

utcWNlsf			INTEGER (0..255),

utcDN				INTEGER (-128..127),

utcDeltaTlsf		INTEGER (-128..127),

utcStandardID		INTEGER (0..7),

...

}

-- ASN1STOP

UTC-ModelSet4 field descriptions

utcA1wnt Parameter A1WNT, sec/sec ([10], Message Type 12). Scale factor 2-50 seconds/second.

utcA0wnt Parameter A0WNT, seconds ([10], Message Type 12). Scale factor 2-30 seconds.

utcTot Parameter tot, seconds ([10], Message Type 12). Scale factor 212 seconds.

utcWNt Parameter WNt, weeks ([10], Message Type 12). Scale factor 1 week.

utcDeltaTls Parameter tLS, seconds ([10], Message Type 12). Scale factor 1 second.

utcWNlsf Parameter WNLSF, weeks ([10], Message Type 12). Scale factor 1 week.

utcDN Parameter DN, days ([10], Message Type 12). Scale factor 1 day.

utcDeltaTlsf Parameter tLSF, seconds ([10], Message Type 12). Scale factor 1 second.

utcStandardID If GNSS-ID indicates 'sbas', this field indicates the UTC standard used for the SBAS network time indicated by SBAS-ID to UTC relation as defined in the table Value of UTC Standard ID to UTC Standard relation shown below ([10], Message Type 12).

Value of UTC Standard ID to UTC Standard relation

| Value of UTC Standard ID | UTC Standard |
| --- | --- |
| 0 | UTC as operated by the Communications Research Laboratory (CRL), Tokyo, Japan |
| 1 | UTC as operated by the National Institute of Standards and Technology (NIST) |
| 2 | UTC as operated by the U. S. Naval Observatory (USNO) |
| 3 | UTC as operated by the International Bureau of Weights and Measures (BIPM) |
| 4-7 | Reserved for future definition |

#### –	UTC-ModelSet5

-- ASN1START

UTC-ModelSet5-r12 ::= SEQUENCE {

utcA0-r12			INTEGER (-2147483648..2147483647),

utcA1-r12			INTEGER (-8388608..8388607),

utcDeltaTls-r12		INTEGER (-128..127),

utcWNlsf-r12		INTEGER (0..255),

utcDN-r12			INTEGER (0..255),

utcDeltaTlsf-r12	INTEGER (-128..127),

...

}

-- ASN1STOP

UTC-ModelSet5 field descriptions

utcA0 Parameter A0UTC, BDS clock bias relative to UTC, seconds [23]. Scale factor 2-30 seconds.

utcA1 Parameter A1UTC, BDS clock rate relative to UTC, sec/sec [23]. Scale factor 2-50 sec/sec.

utcDeltaTls Parameter tLS, delta time due to leap seconds before the new leap second effective, seconds [23]. Scale factor 1 second.

utcWNlsf Parameter WNLSF, week number of the new leap second, weeks [23]. Scale factor 1 week.

utcDN Parameter DN, day number of week of the new leap second, days [23]. Scale factor 1 day.

utcDeltaTlsf Parameter tLSF, delta time due to leap seconds after the new leap second effective, seconds [23]. Scale factor 1 second.

#### –	GNSS-AuxiliaryInformation

The IE GNSS-AuxiliaryInformation is used by the location server to provide additional information dependent on the GNSS-ID. If GNSS-AuxiliaryInformation is provided together with other satellite dependent GNSS assistance data (i.e., any of GNSS-DifferentialCorrections, GNSS-NavigationModel, GNSS-DataBitAssistance, or GNSS-AcquisitionAssistance IEs), the GNSS-AuxiliaryInformation should be provided for the same satellites and in the same LPP message as the other satellite dependent GNSS assistance data.

-- ASN1START

GNSS-AuxiliaryInformation ::= CHOICE {

gnss-ID-GPS		GNSS-ID-GPS,

gnss-ID-GLONASS	GNSS-ID-GLONASS,

...,

[[	gnss-ID-BDS-r16		GNSS-ID-BDS-r16

]]

}

GNSS-ID-GPS ::= SEQUENCE	(SIZE(1..64)) OF GNSS-ID-GPS-SatElement

GNSS-ID-GPS-SatElement ::= SEQUENCE {

svID				SV-ID,

signalsAvailable	GNSS-SignalIDs,

...

}

GNSS-ID-GLONASS ::= SEQUENCE (SIZE(1..64)) OF GNSS-ID-GLONASS-SatElement

GNSS-ID-GLONASS-SatElement ::= SEQUENCE {

svID				SV-ID,

signalsAvailable	GNSS-SignalIDs,

channelNumber		INTEGER (-7..13)		OPTIONAL,		-- Cond FDMA

...

}

GNSS-ID-BDS-r16 ::= SEQUENCE	(SIZE(1..64)) OF GNSS-ID-BDS-SatElement-r16

GNSS-ID-BDS-SatElement-r16 ::= SEQUENCE {

svID-r16			SV-ID,

satType-r16				INTEGER (0..3),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| FDMA | The field is mandatory present if the GLONASS SV indicated by svID broadcasts FDMA signals; otherwise it is not present. |

GNSS-AuxiliaryInformation field descriptions

gnss-ID-GPS This choice may only be present if GNSS-ID indicates GPS.

gnss-ID-GLONASS This choice may only be present if GNSS-ID indicates GLONASS.

gnss-ID-BDS This choice may only be present if GNSS-ID indicates BDS.

svID This field specifies the GNSS SV for which the GNSS-AuxiliaryInformation is given.

signalsAvailable This field indicates the ranging signals supported by the satellite indicated by svID. This field is given as a bit string as defined in GNSS-SignalIDs for a particular GNSS. If a bit is set to '1' it indicates that the satellite identified by svID transmits ranging signals according to the signal correspondence in GNSS-SignalIDs. If a bit is set to '0' it indicates that the corresponding signal is not supported on the satellite identified by svID.

channelNumber This field indicates the GLONASS carrier frequency number of the satellite identified by svID, as defined in [9].

satType This field identifies the BDS B1C and BDS B2a Satellite orbit type, defined in [39], [49]. 1 indicates the GEO satellite, 2 indicates the IGSO satellite, 3 indicates the MEO satellite, and 0 is reserved.

#### –	BDS-DifferentialCorrections

The IE BDS-DifferentialCorrections is used by the location server to provide differential corrections to the target device for BDS B1I and BDS B3I.

-- ASN1START

BDS-DifferentialCorrections-r12 ::= SEQUENCE {

dbds-RefTime-r12			INTEGER (0..3599),

bds-SgnTypeList-r12			BDS-SgnTypeList-r12,

...

}

BDS-SgnTypeList-r12 ::= SEQUENCE (SIZE (1..3)) OF BDS-SgnTypeElement-r12

BDS-SgnTypeElement-r12 ::= SEQUENCE {

gnss-SignalID				GNSS-SignalID				OPTIONAL,	-- Need ON

dbds-CorrectionList-r12		DBDS-CorrectionList-r12,

...

}

DBDS-CorrectionList-r12 ::= SEQUENCE (SIZE (1..64)) OF DBDS-CorrectionElement-r12

DBDS-CorrectionElement-r12 ::= SEQUENCE {

svID						SV-ID,

bds-UDREI-r12				INTEGER (0..15),

bds-RURAI-r12				INTEGER (0..15),

bds-ECC-DeltaT-r12			INTEGER (-4096..4095),

...

}

-- ASN1STOP

BDS-DifferentialCorrections field descriptions

dbds-RefTime This field specifies the time for which the differential corrections are valid, modulo 1 hour. dbds-RefTime is given in BDS system time. Scale factor 1-second.

bds-UDREI This field indicates user differential range error information by user differential range error index (UDREI) as defined in [23], [50], clause 5.3.3.8.2.

bds-RURAI This field indicates Regional User Range Accuracy (RURA) information by Regional User Range Accuracy Index (UDREI) as defined in [23], [50], clause 5.3.3.7.

bds-ECC-DeltaT This field indicates the BDS differential correction information which is expressed in equivalent clock correction (t). Add the value of t to the observed pseudo-range to correct the effect caused by the satellite clock offset and ephemeris error. Value -4096 means the t is not available. The scale factor is 0.1 metre.

#### –	BDS-GridModelParameter

The IE BDS-GridModelParameter is used by the location server to provide Ionospheric Grid Information to the target device for BDS B1I and BDS B3I.

-- ASN1START

BDS-GridModelParameter-r12 ::= SEQUENCE {

bds-RefTime-r12			INTEGER (0..3599),

gridIonList-r12			GridIonList-r12,

...

}

GridIonList-r12 ::= SEQUENCE (SIZE (1..320)) OF GridIonElement-r12

GridIonElement-r12 ::= SEQUENCE {

igp-ID-r12				INTEGER (1..320),

dt-r12					INTEGER (0..511),

givei-r12				INTEGER (0..15) ,

...

}

-- ASN1STOP

BDS-GridModelParamater field descriptions

bds-RefTime This field specifies the time for which the grid model parameters are valid, modulo 1 hour. bds-RefTime is given in BDS system time. Scale factor 1-second.

gridIonList This list provides ionospheric grid point information for each grid point. Up to 16 instances are used in this version of the specification. The values 17 to 320 are reserved for future use.

igp-ID This field indicates the ionospheric grid point (IGP) number as defined in [23], [50], clause 5.3.3.9.

dt This field indicates dT as defined in [23], [50], clause 5.3.3.9.1, i.e. the vertical delay at the corresponding IGP indicated by igp-ID. The scale factor is 0.125 metre.

givei This field indicates the Grid Ionospheric Vertical Error Index (GIVEI) which is used to describe the delay correction accuracy at ionospheric grid point indicated by igp-ID, the mapping between GIVEI and GIVE is defined in [23], [50], clause 5.3.3.9.2.

#### –	GNSS-RTK-Observations

The IE GNSS-RTK-Observations is used by the location server to provide GNSS reference station observables (pseudorange, phaserange, phaserange-rate (Doppler), and carrier-to-noise ratio) of the GNSS signals. Essentially, these parameters describe the range and derivatives from respective satellites to the reference station location provided in IE GNSS-RTK-ReferenceStationInfo.

The parameters provided in IE GNSS-RTK-Observations are used as specified for message type 1071-1127 in [30].

-- ASN1START

GNSS-RTK-Observations-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

gnss-ObservationList-r15				GNSS-ObservationList-r15,

...

}

GNSS-ObservationList-r15 ::= SEQUENCE (SIZE(1..64)) OF GNSS-RTK-SatelliteDataElement-r15

GNSS-RTK-SatelliteDataElement-r15 ::= SEQUENCE{

svID-r15								SV-ID,

integer-ms-r15							INTEGER (0..254)				OPTIONAL,	-- Need ON

rough-range-r15							INTEGER (0..1023),

rough-phase-range-rate-r15				INTEGER (-8192..8191)			OPTIONAL,	-- Need ON

gnss-rtk-SatelliteSignalDataList-r15	GNSS-RTK-SatelliteSignalDataList-r15,

...

}

GNSS-RTK-SatelliteSignalDataList-r15 ::= SEQUENCE (SIZE(1..24)) OF

GNSS-RTK-SatelliteSignalDataElement-r15

GNSS-RTK-SatelliteSignalDataElement-r15 ::= SEQUENCE {

gnss-SignalID-r15					GNSS-SignalID,

fine-PseudoRange-r15				INTEGER (-524288..524287),

fine-PhaseRange-r15					INTEGER (-8388608..8388607),

lockTimeIndicator-r15				INTEGER (0..1023),

halfCycleAmbiguityIndicator-r15		BIT STRING (SIZE (1)),

carrier-to-noise-ratio-r15			INTEGER (0..1023)					OPTIONAL,	-- Need ON

fine-PhaseRangeRate-r15				INTEGER (-16384..16383)				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

GNSS-RTK-Observations field descriptions

epochTime This field specifies the epoch time of the observations. The gnss-TimeID in GNSS SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

svID This field specifies the GNSS SV-ID of the satellite for which the GNSS Observations are provided.

integer-ms This field contains the integer number of milliseconds in the satellite rough range. Rough range can be used to restore complete observables for a given satellite. Scale factor 1 milli-second in the range from 0 to 254 milli-seconds.

rough-range This field contains the sub-milliseconds in the satellite rough range (modulo 1 millisecond). Scale factor 2-10 milli-seconds in the range from 0 to (1-2-10) milli-seconds.

rough-phase-range-rate This field contains the GNSS satellite rough phaserange rate. Scale factor 1 m/s. Range ±8191 m/s.

gnss-SignalID This field specifies the GNSS signal for which the GNSS observations are provided.

fine-PseudoRange This field contains the GNSS signal fine pseudorange. Full pseudorange corresponding to the given signal is the sum of this field and the fields integer-ms and rough-range. NOTE 1. Scale factor 2–29 milli-seconds. Range ±(2–10–2–29) milli-seconds.

fine-PhaseRange This field contains the GNSS signal fine phaserange. Being added to fields integer-ms and rough-range allows getting the full phaserange observable corresponding to given signal. NOTE 2. Scale factor 2–31 milli-seconds. Range ±(2–8–2–31) milli-seconds.

lockTimeIndicator This field provides a measure of the amount of time during which the receiver has maintained continuous lock on that satellite signal. If a cycle slip occurs during the previous measurement cycle, the lock time indicator shall be reset to zero. The mapping of lock-time parameters as defined in [30] is according to the table lockTimeIndicator value to lock-time parameters relation shown below.

halfCycleAmbiguityIndicator Value 0 indicates no half-cycle ambiguity. Value 1 indicates half-cycle ambiguity. When providing phaserange with unresolved polarity encoding this bit shall be set to 1. A target device that is not capable of handling half-cycle ambiguities shall skip such phaserange observables. If polarity resolution forced phaserange to be corrected by half-a-cycle, then the lockTimeIndicator must be reset to zero, indicating that despite continuous tracking the final phaserange experienced non-continuity.

carrier-to-noise-ratio This field provides the GNSS signal carrier-to-noise-ratio in dB-Hz. Scale factor 2–4 dB-Hz in the range from 0.0625 to 63.9375 dB-Hz.

fine-PhaseRangeRate This field contains the GNSS signal fine Phase Range Rate. Full phaserange rate is the sum of this field and the rough-phase-range-rate field. NOTE 3. Scale factor 0.0001 m/s. Range ±1.6383 m/s.

NOTE 1:	Complete Pseudorange for each signal (i) of given satellite can be restored as follows: 
Pseudorange(i) = c/1000 × (integer-ms + rough_range/1024 + 2–29 × fine_Pseudorange(i)), metre.

NOTE 2: 	Complete Phaserange for each signal (i) of given satellite can be restored as follows:
Phaserange(i) = c/1000 × (integer-ms + rough_range/1024 + 2–31 × fine_Phaserange(i)), metre.

NOTE 3: 	Complete PhaseRangeRate for each signal (i) of given satellite can be restored as follows:
PhaseRangeRate(i) = rough-phase-range-rate + 0.0001*fine-PhaseRangeRate (i), metre/second.

NOTE 4:	The speed of light c is 299,792,458 metres per second.

lockTimeIndicator value to lock-time parameters relation

| lockTimeIndicator value (i) | Supplementary coefficient (k) [30] | Minimum Lock Time (ms) [30] | Range of Indicated Lock Times (t) (ms) [30] |
| --- | --- | --- | --- |
| 0 – 63 | 1 | i | 0 ≤ t < 64 |
| 64 – 95 | 2 | 2 × i – 64 | 64 ≤ t < 128 |
| 96 – 127 | 4 | 4 × i – 256 | 128 ≤ t < 256 |
| 128 – 159 | 8 | 8 × i – 768 | 256 ≤ t < 512 |
| 160 – 191 | 16 | 16 × i – 2048 | 512 ≤ t < 1024 |
| 192 – 223 | 32 | 32 × i – 5120 | 1024 ≤ t < 2048 |
| 224 – 255 | 64 | 64 × i – 12288 | 2048 ≤ t < 4096 |
| 256 – 287 | 128 | 128 × i – 28672 | 4096 ≤ t < 8192 |
| 288 – 319 | 256 | 256 × i – 65536 | 8192 ≤ t < 16384 |
| 320 – 351 | 512 | 512 × i – 147456 | 16384 ≤ t < 32768 |
| 352 – 383 | 1024 | 1024 × i – 327680 | 32768 ≤ t < 65536 |
| 384 – 415 | 2048 | 2048 × i – 720896 | 65536 ≤ t < 131072 |
| 416 – 447 | 4096 | 4096 × i – 1572864 | 131072 ≤ t < 262144 |
| 448 – 479 | 8192 | 8192 × i – 3407872 | 262144 ≤ t < 524288 |
| 480 – 511 | 16384 | 16384 × i – 7340032 | 524288 ≤ t < 1048576 |
| 512 – 543 | 32768 | 32768 × i – 15728640 | 1048576 ≤ t < 2097152 |
| 544 – 575 | 65536 | 65536 × i – 33554432 | 2097152 ≤ t < 4194304 |
| 576 – 607 | 131072 | 131072 × i – 71303168 | 4194304 ≤ t < 8388608 |
| 608 – 639 | 262144 | 262144 × i – 150994944 | 8388608 ≤ t < 16777216 |
| 640 – 671 | 524288 | 524288 × i – 318767104 | 16777216 ≤ t < 33554432 |
| 672 – 703 | 1048576 | 1048576 × i – 671088640 | 33554432 ≤ t < 67108864 |
| 704 | 2097152 | 2097152 × i – 1409286144 | 67108864 ≤ t |
| 705 – 1023 | Reserved |  |  |

#### –	GLO-RTK-BiasInformation

The IE GLO-RTK-BiasInformation is used by the location server to provide the so-called "GLONASS Code-Phase bias values" (CPB) for up to all FDMA GLONASS observations.

If IE GNSS-RTK-Observations for gnss-ID = glonass are provided, but IE GLO-RTK-BiasInformation is not provided, the target device assumes that the CPB information has been applied to the GLONASS observation data a priori.

The parameters provided in IE GLO-RTK-BiasInformation are used as specified for message type 1230 in [30].

-- ASN1START

GLO-RTK-BiasInformation-r15 ::= SEQUENCE{

referenceStationID-r15		GNSS-ReferenceStationID-r15,

cpbIndicator-r15			BIT STRING (SIZE(1)),

l1-ca-cpBias-r15			INTEGER (-32768..32767)			OPTIONAL,		-- Need ON

l1-p-cpBias-r15				INTEGER (-32768..32767)			OPTIONAL, 		-- Need ON

l2-ca-cpBias-r15			INTEGER (-32768..32767)			OPTIONAL, 		-- Need ON

l2-p-cpBias-r15				INTEGER (-32768..32767)			OPTIONAL, 		-- Need ON

...

}

-- ASN1STOP

GLO-RTK-BiasInformation field descriptions

referenceStationID This field specifies the Station ID for which the GLO-RTK-BiasInformation is provided.

cpbIndicator This field specifies the GLONASS Code-Phase Bias Indicator. The interpretation of the value is as follows: 0 – The GLONASS Pseudorange and Phaserange observations in IE GNSS-RTK-Observations are not aligned to the same measurement epoch. 1 – The GLONASS Pseudorange and Phaserange observations in IE GNSS-RTK-Observations are aligned to the same measurement epoch.

l1-ca-cpBias This field specifies the GLONASS L1 C/A Code-Phase Bias, which represents the offset between the L1 C/A Pseudorange and L1 Phaserange measurement epochs in metres. If cpbIndicator is set to 0, the measurement epoch of the GLONASS L1 Phaserange measurements may be aligned using: Aligned GLONASS L1 Phaserange = Full GLONASS L1 Phaserange + GLONASS L1 C/A Code-Phase Bias. If cpbIndicator is set to 1, the measurement epoch of the GLONASS L1 Phaserange measurements may be unaligned using: Unaligned GLONASS L1 Phaserange = Full GLONASS L1 Phaserange – GLONASS L1 C/A Code-Phase Bias. Scale factor 0.02 m. Range ±655.34 m.

l1-p-cpBias This field specifies the GLONASS L1 P Code-Phase Bias, which represents the offset between the L1 P Pseudorange and L1 Phaserange measurement epochs in metres. If cpbIndicator is set to 0, the measurement epoch of the GLONASS L1 Phaserange measurements may be aligned using: Aligned GLONASS L1 Phaserange = Full GLONASS L1 Phaserange + GLONASS L1 P Code-Phase Bias. If cpbIndicator is set to 1, the measurement epoch of the GLONASS L1 Phaserange measurements may be unaligned using: Unaligned GLONASS L1 Phaserange = Full GLONASS L1 Phaserange – GLONASS L1 P Code-Phase Bias. Scale factor 0.02 m. Range ±655.34 m.

l2-ca-cpBias This field specifies the GLONASS L2 C/A Code-Phase Bias, which represents the offset between the L2 C/A Pseudorange and L2 Phaserange measurement epochs in metres. If cpbIndicator is set to 0, the measurement epoch of the GLONASS L2 Phaserange measurements may be aligned using: Aligned GLONASS L2 Phaserange = Full GLONASS L2 Phaserange + GLONASS L2 C/A Code-Phase Bias. If cpbIndicator is set to 1, the measurement epoch of the GLONASS L2 Phaserange measurements may be unaligned using: Unaligned GLONASS L2 Phaserange = Full GLONASS L2 Phaserange – GLONASS L2 C/A Code-Phase Bias. Scale factor 0.02 m. Range ±655.34 m.

l2-p-cpBias This field specifies the GLONASS L2 P Code-Phase Bias, which represents the offset between the L2 P Pseudorange and L2 Phaserange measurement epochs in metres. If cpbIndicator is set to 0, the measurement epoch of the GLONASS L2 Phaserange measurements may be aligned using: Aligned GLONASS L2 Phaserange = Full GLONASS L2 Phaserange + GLONASS L2 P Code-Phase Bias. If cpbIndicator is set to 1, the measurement epoch of the GLONASS L2 Phaserange measurements may be unaligned using: Unaligned GLONASS L2 Phaserange = Full GLONASS L2 Phaserange – GLONASS L2 P Code-Phase Bias. Scale factor 0.02 m. Range ±655.34 m.

#### –	GNSS-RTK-MAC-CorrectionDifferences

The IE GNSS-RTK-MAC-CorrectionDifferences is used by the location server to provide dispersive (ionospheric) and non-dispersive (geometric) correction difference components for up to 32 pairs of Auxiliary and Master Reference Stations. The Master Reference Station coordinates are provided in IE GNSS-RTK-ReferenceStationInfo and the Auxiliary Station coordinates are provided in IE GNSS-RTK-AuxiliaryStationData.

The parameters provided in IE GNSS-RTK-MAC-CorrectionDifferences are used as specified for message type 1017 and 1039 in [30] and apply to all GNSSs.

-- ASN1START

GNSS-RTK-MAC-CorrectionDifferences-r15 ::= SEQUENCE {

networkID-r15						GNSS-NetworkID-r15,

subNetworkID-r15					GNSS-SubNetworkID-r15				OPTIONAL,	-- Need ON

master-ReferenceStationID-r15		GNSS-ReferenceStationID-r15,

l1-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

l2-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

rtkCorrectionDifferencesList-r15	RTK-CorrectionDifferencesList-r15,

...

}

RTK-CorrectionDifferencesList-r15 ::= SEQUENCE (SIZE (1..32)) OF

RTK-CorrectionDifferencesElement-r15

RTK-CorrectionDifferencesElement-r15 ::= SEQUENCE {

epochTime-r15							GNSS-SystemTime,

auxiliary-referenceStationID-r15		GNSS-ReferenceStationID-r15,

geometric-ionospheric-corrections-differences-r15

Geometric-Ionospheric-Corrections-Differences-r15,

...

}

Geometric-Ionospheric-Corrections-Differences-r15 ::= SEQUENCE (SIZE(1..64)) OF

Geometric-Ionospheric-Corrections-Differences-Element-r15

Geometric-Ionospheric-Corrections-Differences-Element-r15 ::= SEQUENCE {

svID-r15										SV-ID,

ambiguityStatusFlag-r15							INTEGER (0..3),

non-synch-count-r15								INTEGER (0..7),

geometricCarrierPhaseCorrectionDifference-r15	INTEGER (-65536..65535),

iod-r15											BIT STRING (SIZE(11)),

ionosphericCarrierPhaseCorrectionDifference-r15	INTEGER (-65536..65535),

...

}

-- ASN1STOP

GNSS-RTK-MAC-CorrectionDifferences field descriptions

networkID This field provides the network ID.

subNetworkID This field identifies the subnetwork of a network identified by networkID.

master-ReferenceStationID This field specifies the station ID of the Master Reference Station.

l1, l2 These fields specify the dual-frequency combination of L1 and L2 link/frequencies for which the rtkCorrectionDifferencesList is provided. If the fields are absent, the default interpretation in table 'L1/L2 default interpretation' applies.

rtkCorrectionDifferencesList This field provides the correction differences for Auxiliary-Master Reference Station pairs.

epochTime This field specifies the epoch time of observations used to derive the correction differences. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

auxiliary-referenceStationID This field specifies the station ID of the Auxiliary Reference Station.

svID This field specifies the satellite for which the data is provided.

ambiguityStatusFlag This field provides the ambiguity status. 'L1' below corresponds to the link indicated by the l1 field; 'L2' below corresponds to the link indicated by the l2 field. 0 - Reserved for future use (artificial observations) 1 - Correct Integer Ambiguity Level for L1 and L2 2 - Correct Integer Ambiguity Level for L1-L2 widelane 3 - Uncertain Integer Ambiguity Level. Only a likely guess is used.

non-synch-count This field provides the count of unrecoverable cycle slips. Whenever an unrecoverable cycle slip occurs this count shall be increased. The counter shall not be increased more than once per minute. Data for satellites with cycle slips more frequent than once per minute should not be provided.

geometricCarrierPhaseCorrectionDifference This field provides the Geometric Carrier Phase Correction Difference (GCPCD), which is the Correction Difference for the geometric part (troposphere and orbits) calculated based on integer leveled L1 and L2 correction differences (L1CD and L2CD). L1CD, L2CD, and ICPCD are presented in metres. 'L1' below corresponds to the link indicated by the l1 field; 'L2' below corresponds to the link indicated by the l2 field. Scale factor 0.5 millimetre; range ±32.767 metres.

iod This field specifies the IOD value of the broadcast ephemeris used for calculation of Correction Differences (see IE GNSS-NavigationModel).

ionosphericCarrierPhaseCorrectionDifference This field provides the Ionospheric Carrier Phase Correction Difference (ICPCD), which is the Correction Difference for the ionospheric part calculated based on integer leveled L1 and L2 correction differences (L1CD and L2CD). L1CD, L2CD, and ICPCD are presented in metres. 'L1' below corresponds to the link indicated by the l1 field; 'L2' below corresponds to the link indicated by the l2 field. Scale factor 0.5 millimetre; range ±32.767 metres.

L1/L2 default interpretation

| GNSS | l1 | l2 |
| --- | --- | --- |
| GPS | L1 | L2 |
| SBAS | L1 | L5 |
| QZSS | L1 | L2 |
| Galileo | E1 | E5a |
| GLONASS | G1 | G2 |
| BDS | B1 | B2 |

#### –	GNSS-RTK-Residuals

The IE GNSS-RTK-Residuals is used by the location server to provide Network RTK correction residual error information.

If the interpolation of the corrections for the target device location is performed at the location server, resulting in a non-physical reference station, the GNSS-RTK-Residuals are referenced to the non-physical reference station.

If the interpolation of the corrections is performed by the target device (e.g., using GNSS-RTK-MAC-CorrectionDifferences), the GNSS-RTK-Residuals are referenced to the closest master or auxiliary station to the target device.

The parameters provided in IE GNSS-RTK-Residuals are used as specified for message type 1030 and 1031 in [30] and apply to all GNSSs.

-- ASN1START

GNSS-RTK-Residuals-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

referenceStationID-r15				GNSS-ReferenceStationID-r15,

n-Refs-r15							INTEGER (0..127),

l1-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

l2-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

rtk-residuals-list-r15				RTK-Residuals-List-r15,

...

}

RTK-Residuals-List-r15 ::= SEQUENCE (SIZE(1..64)) OF RTK-Residuals-Element-r15

RTK-Residuals-Element-r15 ::= SEQUENCE {

svID-r15			SV-ID,

s-oc-r15			INTEGER (0..255),

s-od-r15			INTEGER (0..511),

s-oh-r15			INTEGER (0..63),

s-lc-r15			INTEGER (0..1023),

s-ld-r15			INTEGER (0..1023),

...

}

-- ASN1STOP

GNSS-RTK-Residuals field descriptions

epochTime This field specifies the epoch time of the Network RTK Residual Error data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

referenceStationID This field specifies the Reference Station ID. The Reference Station may be a physical or non-physical station.

n-Refs This field specifies the number of reference stations used to derive the residual statistics (1 to 127; 127 indicates 127 or more stations). The number of reference stations should never be zero. If zero is encountered the target device should ignore the message.

l1, l2 These fields specify the dual-frequency combination of L1 and L2 link/frequencies for which the rtk residuals-list is provided. If the fields are absent, the default interpretation in table 'L1/L2 default interpretation' in IE GNSS-RTK-MAC-CorrectionDifferences applies.

svID This field specifies the satellite for which the data is provided.

s-oc This field specifies the constant term of standard deviation (1 sigma) for non-dispersive interpolation residuals, s0c. Scale factor 0.5 millimetre; range 0–127 millimetre. NOTE 1.

s-od This field specifies the distance dependent term of standard deviation (1 sigma) for nondispersive interpolation residuals, s0d. Scale factor 0.01 ppm; range 0–5.11 ppm. NOTE 1.

s-oh This field specifies the height dependent term of standard deviation (1 sigma) for nondispersive interpolation residuals, s0h. Scale factor 0.1 ppm; range 0–5.1 ppm. NOTE 1.

s-lc This field specifies the constant term of standard deviation (1 sigma) for dispersive interpolation residuals (as affecting L1 frequency), slc. 'L1' corresponds to the link indicated by the l1 field. Scale factor 0.5 millimetre; range 0–511 millimetre

s-ld This field specifies the distance dependent term of standard deviation (1 sigma) for dispersive interpolation residuals (as affecting L1 frequency), sld. 'L1' corresponds to the link indicated by the l1 field. NOTE 2.

NOTE 1: 	The complete standard deviation for the expected non-dispersive interpolation residual is computed from s-oc, s-od and s-oh using the formula:

where dRef is the distance of the target device from the nearest physical reference station in [km] and |dhRef| is the absolute value of the height difference between the nearest physical reference station and the target device in [km].

NOTE 2:	The complete standard deviation for the expected dispersive interpolation residual is computed from s-lc and s-ld using the formula:

where dRef is the distance of the target device from the nearest physical reference station in [km]. 
The standard deviation for the L2 frequency is calculated using the formula:
. 'L2' corresponds to the link indicated by the l2 field; c/f1c/f2 are the nominal wavelengths of the links indicated by the l1, l2 fields, respectively.

#### –	GNSS-RTK-FKP-Gradients

The IE GNSS-RTK-FKP-Gradients is used by the location server to provide the FKP Network RTK gradients of distance-dependent errors like ionosphere, troposphere and orbits. The target device may use the gradients to compute the influence of the distance dependent errors for its own position.

The parameters provided in IE GNSS-RTK-FKP-Gradients are used as specified for message type 1034 and 1035 in [30] and apply to all GNSSs.

-- ASN1START

GNSS-RTK-FKP-Gradients-r15 ::= SEQUENCE {

referenceStationID-r15				GNSS-ReferenceStationID-r15,

epochTime-r15						GNSS-SystemTime,

l1-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

l2-r15								GNSS-FrequencyID-r15				OPTIONAL,	-- Need OP

fkp-gradients-list-r15				FKP-Gradients-List-r15,

...

}

FKP-Gradients-List-r15 ::= SEQUENCE (SIZE(1..64)) OF FKP-Gradients-Element-r15

FKP-Gradients-Element-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

north-geometric-gradient-r15		INTEGER (-2048..2047),

east-geometric-gradient-r15			INTEGER (-2048..2047),

north-ionospheric-gradient-r15		INTEGER (-8192..8191),

east-ionospheric-gradient-r15		INTEGER (-8192..8191),

...

}

-- ASN1STOP

GNSS-RTK-FKP-Gradients field descriptions

referenceStationID This field specifies the Reference Station ID. The Reference Station may be a physical or non-physical station.

epochTime This field specifies the epoch time of the FKP data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

l1, l2 These fields specify the dual-frequency combination of L1 and L2 link/frequencies for which the fkp-gradients-list is provided. If the fields are absent, the default interpretation in table 'L1/L2 default interpretation' in IE GNSS-RTK-MAC-CorrectionDifferences applies. NOTE.

svID This field specifies the satellite for which the data is provided.

iod This field specifies the IOD value of the broadcast ephemeris used for calculation of FKP data (see IE GNSS-NavigationModel).

north-geometric-gradient This field specifies the gradient (FKP) of the geometric (non-dispersive) error components in South-North direction in parts per million of the south-north distance to the reference station. Scale factor 0.01 ppm; range ±20.47 ppm.

east-geometric-gradient This field specifies the gradient (FKP) of the geometric (non-dispersive) error components in West-East direction in parts per million of the west-east distance to the reference station. Scale factor 0.01 ppm; range ±20.47 ppm.

north-ionospheric-gradient This field specifies the gradient (FKP) of the ionospheric (dispersive) error component in South-North direction. Scale factor 0.01 ppm; range ±81.91 ppm.

east-ionospheric-gradient This field specifies the gradient (FKP) of the ionospheric (dispersive) error component in West-East direction. Scale factor 0.01 ppm; range ±81.91 ppm.

NOTE:	As described in [30], the distance dependent error for the geometric part 0 and ionospheric part I is computed from the gradients provided in FKP-Gradients-Element. The distance dependent error for a carrier phase measurements Ф on a signal with frequency f can be computed by:

where f1, f is the link/frequency indicated by the l1, l2 fields, respectively.

#### –	GNSS-SSR-OrbitCorrections

The IE GNSS-SSR-OrbitCorrections is used by the location server to provide radial, along-track and cross-track orbit corrections together with integrity information. The target device may use the SSR-OrbitCorrectionList to compute a satellite position correction to be combined with the satellite position calculated from broadcast ephemeris.

The parameters provided in IE GNSS-SSR-OrbitCorrections – except for ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds –  are used as specified for SSR Clock Messages (e.g., message type 1057 and 1063) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-OrbitCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

satelliteReferenceDatum-r15			ENUMERATED { itrf, regional, ... },

iod-ssr-r15							INTEGER (0..15),

ssr-OrbitCorrectionList-r15			SSR-OrbitCorrectionList-r15,

...,

[[

orbit-IntegrityParameters-r17	ORBIT-IntegrityParameters-r17	OPTIONAL -- Need ON

]]

}

SSR-OrbitCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-OrbitCorrectionSatelliteElement-r15

SSR-OrbitCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

iod-r15								BIT STRING (SIZE(11)),

delta-radial-r15					INTEGER (-2097152..2097151),

delta-AlongTrack-r15				INTEGER (-524288..524287),

delta-CrossTrack-r15				INTEGER (-524288..524287),

dot-delta-radial-r15				INTEGER (-1048576..1048575)		OPTIONAL, -- Need ON

dot-delta-AlongTrack-r15			INTEGER (-262144..262143) 		OPTIONAL, -- Need ON

dot-delta-CrossTrack-r15			INTEGER (-262144..262143) 		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityOrbitBounds-r17	SSR-IntegrityOrbitBounds-r17	OPTIONAL  -- Cond Integrity1

]]

}

ORBIT-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetConstFault-r17					INTEGER (0..255),

meanConstFaultDuration-r17				INTEGER (1..3600),

probOnsetSatFault-r17					INTEGER (0..255),

meanSatFaultDuration-r17				INTEGER (1..3600),

orbitRangeErrorCorrelationTime-r17		INTEGER (0..255)			OPTIONAL, -- Need ON

orbitRangeRateErrorCorrelationTime-r17	INTEGER (0..255)			OPTIONAL, -- Cond Integrity2

...

}

SSR-IntegrityOrbitBounds-r17 ::= SEQUENCE {

meanOrbitError-r17				RAC-OrbitalErrorComponents-r17,

varOrbitError-r17				RAC-OrbitalErrorComponents-r17,

meanOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

varOrbitRateError-r17			RAC-OrbitalErrorComponents-r17,

...

}

RAC-OrbitalErrorComponents-r17	::= SEQUENCE {

radial-r17			INTEGER (0..255),

alongTrack-r17		INTEGER (0..255),

crossTrack-r17		INTEGER (0..255)

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if ORBIT-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if orbitRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-OrbitCorrections field descriptions

epochTime This field specifies the epoch time of the orbit corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation below. NOTE 1.

satelliteReferenceDatum This field specifies the satellite refence datum for the orbit corrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the orbit corrections are provided.

iod This field specifies the IOD value of the broadcast ephemeris for which the orbit corrections are valid (see IE GNSS-NavigationModel). NOTE 2.

delta-radial This field specifies the radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.1 mm; range ±209.7151 m.

delta-AlongTrack This field specifies the along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

delta-CrossTrack This field specifies the cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.4 mm; range ±209.7148 m.

dot-delta-radial This field specifies the velocity of radial orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.001 mm/s; range ±1.048575 m/s.

dot-delta-AlongTrack This field specifies the velocity of along-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

dot-delta-CrossTrack This field specifies the velocity of cross-track orbit correction for broadcast ephemeris. NOTE 3. Scale factor 0.004 mm/s; range ±1.048572 m/s.

probOnsetConstFault This field specifies the Probability of Onset of Constellation Fault per Time Unit where a constellation fault is at least two satellites being faulty simultaneously due to the same event. This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetConstFault and the range is 10-10.2 to 1 per hour.

meanConstFaultDuration This field specifies the Mean Constellation Fault Duration which is the mean duration between when a constellation fault occurs, and the user is alerted by the service through FFS (or the integrity violation is over). Scale factor 1 s; range 1-3600 s.

probOnsetSatFault This field specifies the Probability of Onset of Satellite Fault per Time Unit which is the probability of occurrence of satellite error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetSatFault and the range is 10-10.2 to 1 per hour.

meanSatFaultDuration This field specifies the Mean Satellite Fault Duration which is the mean duration between when a satellite fault occurs, and the user is alerted by the service through FFS (or the integrity violation is over). Scale factor 1 s; range 1-3,600 s.

orbitRangeErrorCorrelationTime This field specifies the Orbit Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

orbitRangeRateErrorCorrelationTime This field specifies the Orbit Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to orbit. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanOrbitError This field specifies the Mean Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each mean is calculated using: μ=0.025i, &i≤2005+0.5(i-200), 200<&i≤240 25+2i-240, &i>240 [m] Range is 0-55 m.

varOrbitError This field specifies the Variance Orbit Error bound in satellite radial, along-track and cross-track coordinates, which are the variance values for a set of three overbounding models that bound the residual orbit error in satellite radial, along-track and cross-track directions. Each variance is calculated using: σ2=0.025i, &i≤2005+0.5(i-200), 200<&i≤240 25+2i-240, &i>240 [m] Range is 0-55 m.

meanOrbitRateError This field specifies the Mean Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the mean values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

varOrbitRateError This field specifies the Variance Orbit Rate Error in satellite radial, along-track and cross-track coordinates, which are the variance values for a set of three overbounding models that bound the residual satellite orbit rate error in satellite radial, along-track and cross-track directions. Scale factor 0.001 m/s; range 0-0.255 m/s.

NOTE 1: 	The update intervals are aligned to the GPS time scale for all GNSSs in order to allow synchronous operation for multiple GNSS services. This means that the update intervals may not be aligned to the beginning of the day for another GNSS. Due to the leap seconds, this is generally the case for GLONASS.

NOTE 2:	In the cases that gnss-ID indicates 'gps' or 'qzss', the iod refers to the NAV broadcast ephemeris (GPS L1 C/A or QZSS QZS-L1, respectively, in table GNSS to iod Bit String(11) relation in IE GNSS-NavigationModel).

NOTE 3: 	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

Value of ssrUpdateInterval to SSR Update Interval relation

| Value of ssrUpdateInterval | SSR Update Interval |
| --- | --- |
| 0 | 1 second |
| 1 | 2 seconds |
| 2 | 5 seconds |
| 3 | 10 seconds |
| 4 | 15 seconds |
| 5 | 30 seconds |
| 6 | 60 seconds |
| 7 | 120 seconds |
| 8 | 240 seconds |
| 9 | 300 seconds |
| 10 | 600 seconds |
| 11 | 900 seconds |
| 12 | 1800 seconds |
| 13 | 3600 seconds |
| 14 | 7200 seconds |
| 15 | 10800 seconds |

#### –	GNSS-SSR-ClockCorrections

The IE GNSS-SSR-ClockCorrections is used by the location server to provide clock correction parameters together with integrity information. The target device may use the SSR-ClockCorrectionList to compute a clock correction to be applied to the broadcast satellite clock parameters, identified by iod of corresponding GNSS-SSR-OrbitCorrections.

The parameters provided in IE GNSS-SSR-ClockCorrections – except for CLOCK-IntegrityParameters and SSR-IntegrityClockBounds – are used as specified for SSR Clock Messages (e.g., message type 1058 and 1064) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-ClockCorrections-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

iod-ssr-r15							INTEGER (0..15),

ssr-ClockCorrectionList-r15			SSR-ClockCorrectionList-r15,

...,

[[

clock-IntegrityParameters-r17	CLOCK-IntegrityParameters-r17		OPTIONAL   -- Need ON

]]

}

SSR-ClockCorrectionList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-ClockCorrectionSatelliteElement-r15

SSR-ClockCorrectionSatelliteElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

delta-Clock-C0-r15					INTEGER (-2097152..2097151),

delta-Clock-C1-r15					INTEGER (-1048576..1048575)			OPTIONAL, -- Need ON

delta-Clock-C2-r15					INTEGER (-67108864..67108863)		OPTIONAL, -- Need ON

...,

[[

ssr-IntegrityClockBounds-r17	SSR-IntegrityClockBounds-r17		OPTIONAL  -- Need ON

]]

}

CLOCK-IntegrityParameters-r17 ::= SEQUENCE {

clockRangeErrorCorrelationTime-r17		INTEGER (0..255),

clockRangeRateErrorCorrelationTime-r17	INTEGER (0..255),

...

}

SSR-IntegrityClockBounds-r17 ::= SEQUENCE {

meanClock-r17							INTEGER (0..255),

stdDevClock-r17							INTEGER (0..255),

meanClockRate-r17						INTEGER (0..255),

stdDevClockRate-r17						INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-ClockCorrections field descriptions

epochTime This field specifies the epoch time of the clock corrections. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the satellite for which the clock corrections are provided.

delta-Clock-C0 This field specifies the C0 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.1 mm; range ±209.7151 m.

delta-Clock-C1 This field specifies the C1 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.001 mm/s; range ±1.048575 m/s.

delta-Clock-C2 This field specifies the C2 polynomial coefficient for correction of broadcast satellite clock. NOTE 1. Scale factor 0.00002 mm/s2; range ±1.34217726 m/s2.

clockRangeErrorCorrelationTime This field specifies the Clock Range Error Correlation Time which is the upper bound of the correlation time of the satellite residual range error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

clockRangeRateErrorCorrelationTime This field specifies the Clock Range Rate Error Correlation Time which is the upper bound of the correlation time of the satellite residual range rate error due to clock. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanClock This field specifies the Mean Clock Error bound which is the mean value for an overbounding model that bounds the residual clock error. The bound is meanClock + K * stdDevClock and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. The mean is calculated using: μ=0.025i, &i≤2005+0.5(i-200), 200<&i≤240 25+2i-240, &i>240 [m] Range is 0-55 m.

stdDevClock This field specifies the Standard Deviation Clock Error bound which is the standard deviation for an overbounding model that bounds the residual clock error. The standard deviation is calculated using: σ=0.025i, &i≤2005+0.5(i-200), 200<&i≤240 25+2i-240, &i>240 [m] Range is 0-55 m.

meanClockRate This field specifies the Mean Clock Rate Error bound which is the mean value for an overbounding model that bounds the residual clock rate error. The bound is meanClockRate + K * stdDevClockRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

stdDevClockRate This field specifies the Standard Deviation Clock Rate Error bound which is the standard deviation for an overbounding model that bounds the residual clock rate error. Scale factor 0.001 m/s; range 0.000-0.255 m/s.

NOTE 1: 	The reference time t0 is epochTime + ½ × ssrUpdateInterval. The reference time t0 for ssrUpdateInterval '0' is epochTime.

#### –	GNSS-SSR-CodeBias

The IE GNSS-SSR-CodeBias is used by the location server to provide GNSS signal code bias together with integrity information. The target device may add the code bias to the pseudo-range measurement of the corresponding code signal to get corrected pseudo-ranges.

NOTE:	Any code biases transmitted in the broadcast messages (e.g., the GPS group delay differential TGD [4] (NAV-ClockModel)) are not applied at all by the target device.

The parameters provided in IE GNSS-SSR-CodeBias – except for SSR-IntegrityCodeBiasBounds – are used as specified for SSR Code Bias Messages (e.g., message type 1059 and 1065) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-CodeBias-r15 ::= SEQUENCE {

epochTime-r15						GNSS-SystemTime,

ssrUpdateInterval-r15				INTEGER (0..15),

iod-ssr-r15							INTEGER (0..15),

ssr-CodeBiasSatList-r15				SSR-CodeBiasSatList-r15,

...

}

SSR-CodeBiasSatList-r15 ::= SEQUENCE (SIZE(1..64)) OF SSR-CodeBiasSatElement-r15

SSR-CodeBiasSatElement-r15 ::= SEQUENCE {

svID-r15							SV-ID,

ssr-CodeBiasSignalList-r15			SSR-CodeBiasSignalList-r15,

...

}

SSR-CodeBiasSignalList-r15 ::= SEQUENCE (SIZE(1..16)) OF SSR-CodeBiasSignalElement-r15

SSR-CodeBiasSignalElement-r15 ::= SEQUENCE {

signal-and-tracking-mode-ID-r15		GNSS-SignalID,

codeBias-r15						INTEGER (-8192..8191),

...,

[[

ssr-IntegrityCodeBiasBounds-r17		SSR-IntegrityCodeBiasBounds-r17		OPTIONAL	-- Need ON

]]

}

SSR-IntegrityCodeBiasBounds-r17 ::= SEQUENCE {

meanCodeBias-r17					INTEGER (0..255),

stdDevCodeBias-r17					INTEGER (0..255),

meanCodeBiasRate-r17				INTEGER (0..255),

stdDevCodeBiasRate-r17				INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-CodeBias field descriptions

epochTime This field specifies the epoch time of the code bias data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the code biases are provided.

signal-and-tracking-mode-ID This field specifies the GNSS signal for which the code biases are provided.

codeBias This field provides the code bias for the GNSS signal indicated by signal-and-tracking-mode-ID. Scale factor 0.01 m; range ±81.91 m.

meanCodeBias This field specifies the Mean Code Bias Error bound which is the mean value for an overbounding model that bounds the residual code bias error. The bound is meanCodeBias + K * stdDevCodeBias and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevCodeBias This field specifies the Standard Deviation Code Bias Error bound which is the standard deviation for an overbounding model that bounds the residual code bias error. Scale factor 0.005 m; range 0-1.275 m.

meanCodeBiasRate This field specifies the Mean Code Bias Rate Error bound which is the mean value for an overbounding model that bounds the residual code bias rate error. The bound is meanCodeBiasRate + K * stdDevCodeBiasRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevCodeBiasRate This field specifies the Standard Deviation Code Bias Rate Error bound which is the standard deviation for an overbounding model that bounds the residual code bias rate error. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

#### –	GNSS-SSR-URA

The IE GNSS-SSR-URA is used by the location server to provide quality information for the provided SSR assistance data.

The parameters provided in IE GNSS-SSR-URA are used as specified for the SSR URA Messages (e.g., message type 1061 and 1067) in [30] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-URA-r16 ::= SEQUENCE {

epochTime-r16						GNSS-SystemTime,

ssrUpdateInterval-r16				INTEGER (0..15),

iod-ssr-r16							INTEGER (0..15),

ssr-URA-SatList-r16					SSR-URA-SatList-r16,

...

}

SSR-URA-SatList-r16 ::= SEQUENCE (SIZE(1..64)) OF SSR-URA-SatElement-r16

SSR-URA-SatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

ssr-URA-r16							BIT STRING (SIZE (6)),

...

}

-- ASN1STOP

GNSS-SSR-URA field descriptions

epochTime This field specifies the epoch time of the SSR User Range Accuracy (URA). The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the SSR URA is provided.

ssr-URA This field specifies the User Range Accuracy (URA) (1-sigma) for the range correction provided in the SSR assistance data. The URA is represented by a combination of CLASS and VALUE. The 3 MSB define the CLASS with a range of 0-7 and the 3 LSB define the VALUE with a range of 0-7. The URA is computed by: SSR URA mm≤3CLASS1+VALUE4-1 [mm] See Table 'Relationship between SSR troposphere quality and URA indicator and physical quantity' in IE GNSS-SSR-GriddedCorrection.

#### –	GNSS-SSR-PhaseBias

The IE GNSS-SSR-PhaseBias is used by the location server to provide GNSS signal phase bias together with integrity information. The target device may add the phase bias to the phase-range measurement of the corresponding phase signal to get corrected phase-ranges.

The parameters provided in IE GNSS-SSR-PhaseBias – except for SSR-IntegrityPhaseBiasBounds – are used as specified for Compact SSR GNSS Satellite Phase Bias Messages (e.g., message type 4073,5) in [43] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-PhaseBias-r16 ::= SEQUENCE {

epochTime-r16						GNSS-SystemTime,

ssrUpdateInterval-r16				INTEGER (0..15),

iod-ssr-r16							INTEGER (0..15),

ssr-PhaseBiasSatList-r16			SSR-PhaseBiasSatList-r16,

...

}

SSR-PhaseBiasSatList-r16 ::= SEQUENCE (SIZE(1..64)) OF SSR-PhaseBiasSatElement-r16

SSR-PhaseBiasSatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

ssr-PhaseBiasSignalList-r16			SSR-PhaseBiasSignalList-r16,

...

}

SSR-PhaseBiasSignalList-r16 ::= SEQUENCE (SIZE(1..16)) OF SSR-PhaseBiasSignalElement-r16

SSR-PhaseBiasSignalElement-r16 ::= SEQUENCE {

signal-and-tracking-mode-ID-r16		GNSS-SignalID,

phaseBias-r16						INTEGER (-16384..16383),

phaseDiscontinuityIndicator-r16		INTEGER (0..3),

phaseBiasIntegerIndicator-r16		INTEGER (0..3)					OPTIONAL,	-- Need OP

...,

[[

ssr-IntegrityPhaseBiasBounds-r17	SSR-IntegrityPhaseBiasBounds-r17	OPTIONAL	-- Need ON

]]

}

SSR-IntegrityPhaseBiasBounds-r17 ::= SEQUENCE {

meanPhaseBias-r17					INTEGER (0..255),

stdDevPhaseBias-r17					INTEGER (0..255),

meanPhaseBiasRate-r17				INTEGER (0..255),

stdDevPhaseBiasRate-r17				INTEGER (0..255),

...

}

-- ASN1STOP

GNSS-SSR-PhaseBias field descriptions

epochTime This field specifies the epoch time of the phase bias data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the phase biases are provided.

signal-and-tracking-mode-ID This field specifies the GNSS signal for which the phase biases are provided.

phaseBias This field provides the phase bias for the GNSS signal indicated by signal-and-tracking-mode-ID. Scale factor 0.001 m; range ±16.383 m.

phaseDiscontinuityIndicator This field provides the phase discontinuity counter for the GNSS signal indicated by signal-and-tracking-mode-ID. This counter is increased for every discontinuity in phase (roll-over from 3 to 0).

phaseBiasIntegerIndicator This field informs whether the phase bias is Undifferenced Integer (Value 0), Widelane Integer (Value 1) or Non-Integer (Value 2): Value 0: The Undifferenced Integer Phase Bias supports PPP-RTK fixed, widelane or float mode. Value 1: The Widelane Integer Phase Bias indicates that after application of the Phase Bias value, this signal can be differenced with any other signal from the same satellite that also has Widelane Integer Phase Bias indicated to form a new combined carrier phase measurement of integer quality, supporting PPP-RTK widelane fixed mode. Value 2: The Non-Integer Phase Bias supports PPP-RTK float mode. Value 3: Reserved. If the phaseBiasIntegerIndicator field is not present then it is interpreted as having Value 0 (Undifferenced Integer).

meanPhaseBias This field specifies the Mean Phase Bias Error bound which is the mean value for an overbounding model that bounds the residual phase bias error. The bound is meanPhaseBias + K * stdDevPhaseBias and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevPhaseBias This field specifies the Standard Deviation Phase Bias Error bound which is the standard deviation for an overbounding model that bounds the residual phase bias error. Scale factor 0.005 m; range 0-1.275 m.

meanPhaseBiasRate This field specifies the Mean Phase Bias Rate Error bound which is the mean value for an overbounding model that bounds the residual phase bias rate error. The bound is meanPhaseBiasRate + K * stdDevPhaseBiasRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevPhaseBiasRate This field specifies the Standard Deviation Phase Bias Rate Error bound which is the standard deviation for an overbounding model that bounds the residual phase bias rate error. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

#### –	GNSS-SSR-STEC-Correction

The IE GNSS-SSR-STEC-Correction is used by the location server to provide ionosphere slant delay correction together with integrity information. The ionosphere slant delay (STEC) consists of the polynomial part provided in GNSS-SSR-STEC-Correction and the residual part provided in GNSS-SSR-GriddedCorrection.

The parameters provided in IE GNSS-SSR-STEC-Correction – except for STEC-IntegrityParameters and STEC-IntegrityErrorBounds – are used as specified for Compact SSR STEC Correction Messages (e.g., message type 4073,8) in [43] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-STEC-Correction-r16 ::= SEQUENCE {

epochTime-r16						GNSS-SystemTime,

ssrUpdateInterval-r16				INTEGER (0..15),

iod-ssr-r16							INTEGER (0..15),

correctionPointSetID-r16			INTEGER (0..16383),

stec-SatList-r16					STEC-SatList-r16,

...,

[[

stec-IntegrityParameters-r17	STEC-IntegrityParameters-r17		OPTIONAL	-- Need ON

]]

}

STEC-SatList-r16 ::= SEQUENCE (SIZE(1..64)) OF STEC-SatElement-r16

STEC-SatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

stecQualityIndicator-r16			BIT STRING (SIZE(6)),

stec-C00-r16						INTEGER (-8192..8191),

stec-C01-r16						INTEGER (-2048..2047)				OPTIONAL, -- Need ON

stec-C10-r16						INTEGER (-2048..2047)				OPTIONAL, -- Need ON

stec-C11-r16						INTEGER (-512..511)					OPTIONAL, -- Need ON

...,

[[

stec-IntegrityErrorBounds-r17	STEC-IntegrityErrorBounds-r17	OPTIONAL  -- Cond Integrity1

]]

}

STEC-IntegrityParameters-r17 ::= SEQUENCE {

probOnsetIonoFault-r17						INTEGER (0..255),

meanIonoFaultDuration-r17					INTEGER (1..256),

ionoRangeErrorCorrelationTime-r17			INTEGER (1..255)		OPTIONAL, -- Need ON

ionoRangeRateErrorCorrelationTime-r17		INTEGER (1..255)		OPTIONAL, -- Cond Integrity2

...

}

STEC-IntegrityErrorBounds-r17 ::= SEQUENCE {

meanIonosphere-r17							INTEGER (0..255),

stdDevIonosphere-r17						INTEGER (0..255),

meanIonosphereRate-r17						INTEGER (0..255),

stdDevIonosphereRate-r17					INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Integrity1 | The field is mandatory present if STEC-IntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if ionoRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-STEC-Correction field descriptions

epochTime This field specifies the epoch time of the STEC correction data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

correctionPointSetID This field provides the ID of the GNSS-SSR-CorrectionPoints set. The reference point used for the STEC calculations (see NOTE below) is the reference point provided in IE GNSS-SSR-CorrectionPoints with the same correctionPointSetID.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

svID This field specifies the GNSS satellite for which the STEC corrections are provided.

stecQualityIndicator This field specifies SSR STEC quality indicator. The STEC quality indicator is represented by a combination of CLASS and VALUE. The 3 MSB define the CLASS with a range of 0-7 and the 3 LSB define the VALUE with a range of 0-7. See Table 'Relationship between SSR STEC quality indicator and physical quantity' below.

stec-C00 This field provides the polynomial coefficient C00 used to define the STEC. as defined in [43]. NOTE Scale factor 0.05 TECU; range ±409.55 TECU.

stec-C01 This field provides the polynomial coefficient C01 used to define the STEC as defined in [43]. NOTE Scale factor 0.02 TECU/deg; range ±40.94 TECU/deg.

stec-C10 This field provides the polynomial coefficient C10 used to define the STEC as defined in [43]. NOTE Scale factor 0.02 TECU/deg; range ±40.94 TECU/deg.

stec-C11 This field provides the polynomial coefficient C11 used to define the STEC as defined in [43]. NOTE Scale factor 0.02 TECU/deg2; range ±10.22 TECU/deg2.

probOnsetIonoFault This field specifies the Probability of Onset of Ionosphere Fault per Time Unit which is the probability of occurrence of ionosphere error to exceed the residual error bound for more than the Time to Alert (TTA). This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2), with irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetIonoFault and the range is 10-10.2 to 1 per hour.

meanIonoFaultDuration This field specifies the Mean Ionosphere Fault Duration which is the mean duration between when an ionosphere integrity violation occurs, and the user is alerted through GNSS-Integrity-ServiceAlert (or the integrity violation is over). Scale factor 1 s; range 1-256 s.

ionoRangeErrorCorrelationTime This field specifies the Ionosphere Range Error Correlation Time which is the upper bound of the correlation time of the ionosphere residual range error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

ionoRangeRateErrorCorrelationTime This field specifies the Ionosphere Range Rate Error Correlation Time which is the upper bound of the correlation time of the ionosphere residual range rate error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanIonosphere This field specifies the Mean Ionospherre Error bound which is the mean value for an overbounding model that bounds the residual ionosphere error. The bound is meanIonosphere + K * stdDevIonosphere and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. The mean is calculated using: μ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

stdDevIonosphere This field specifies the Standard Deviation Ionosphere Error bound which is the standard deviation for an overbounding model that bounds the residual ionosphere error. The standard deviation is calculated using: σ=0.01i, &i≤2002+0.1(i-200), 200<&i≤230 5+0.5i-230, &i>230 [m] Range is 0-17.5 m.

meanIonosphereRate This field specifies the Mean Ionosphere Rate Error which is the mean value for an overbounding model that bounds the residual ionosphere rate error. The bound is meanIonosphereRate + K * stdDevIonosphereRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevIonosphereRate This field specifies the Standard Deviation Ionosphere Rate Error which is the standard deviation for an overbounding model that bounds the residual ionosphere rate error. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

NOTE:	The polynomial coefficients C00, C01, C10, C11 are used to define the STEC as follows:

(1)	If only C00 is included in STEC-SatElement:

Iai = C00.

(2)	If only C00, C01 and C10 are included in STEC-SatElement:

Iai = C00 + C01( – 0) +  C10( – 0).

(3)	If all of C00, C01, C10 and C11 are included in STEC-SatElement:

Iai = C00 + C01( – 0) + C10( – 0) +C11( – 0) ( – 0).

Other combinations of C00, C01, C10, C11 than (1)-(3) above are undefined in this version of the specification.

The equations above depend on the latitude  and longitude  of an evaluated point and latitude 0 and longitude 0 of the reference point which is defined in IE GNSS-SSR-CorrectionPoints (referencePointLatitude and referencePointLongitude).

Relationship between SSR STEC quality indicator and physical quantity

| CLASS | VALUE | Index | SSR STEC Quality Indicator Q [TECU] |
| --- | --- | --- | --- |
| 7 | 7 | 63 | 33.6664 < Q |
| 7 | 6 | 62 | 30.2992 < Q ≤ 33.6664 |
| 7 | 5 | 61 | 26.9319 < Q ≤ 30.2992 |
| 7 | 4 | 60 | 23.5647 < Q ≤ 26.9319 |
| 7 | 3 | 59 | 20.1974 < Q ≤ 23.5647 |
| 7 | 2 | 58 | 16.8301 < Q ≤ 20.1974 |
| 7 | 1 | 57 | 13.4629 < Q ≤ 16.8301 |
| 7 | 0 | 56 | 12.3405 < Q ≤ 13.4629 |
| 6 | 7 | 55 | 11.2180 < Q ≤ 12.3405 |
| 6 | 6 | 54 | 10.0956 < Q ≤ 11.2180 |
| 6 | 5 | 53 | 8.9732 < Q ≤ 10.0956 |
| 6 | 4 | 52 | 7.8508 < Q ≤ 8.9732 |
| 6 | 3 | 51 | 6.7284 < Q ≤ 7.8508 |
| 6 | 2 | 50 | 5.6059 < Q ≤ 6.7284 |
| 6 | 1 | 49 | 4.4835 < Q ≤ 5.6059 |
| 6 | 0 | 48 | 4.1094 < Q ≤ 4.4835 |
| 5 | 7 | 47 | 3.7352 < Q ≤ 4.1094 |
| 5 | 6 | 46 | 3.3611 < Q ≤ 3.7352 |
| 5 | 5 | 45 | 2.9870 < Q ≤ 3.3611 |
| 5 | 4 | 44 | 2.6128 < Q ≤ 2.9870 |
| 5 | 3 | 43 | 2.2387 < Q ≤ 2.6128 |
| 5 | 2 | 42 | 1.8645 < Q ≤ 2.2387 |
| 5 | 1 | 41 | 1.4904 < Q ≤ 1.8645 |
| 5 | 0 | 40 | 1.3657 < Q ≤ 1.4904 |
| 4 | 7 | 39 | 1.2410 < Q ≤ 1.3657 |
| 4 | 6 | 38 | 1.1163 < Q ≤ 1.2410 |
| 4 | 5 | 37 | 0.9915 < Q ≤ 1.1163 |
| 4 | 4 | 36 | 0.8668 < Q ≤ 0.9915 |
| 4 | 3 | 35 | 0.7421 < Q ≤ 0.8668 |
| 4 | 2 | 34 | 0.6174 < Q ≤ 0.7421 |
| 4 | 1 | 33 | 0.4927 < Q ≤ 0.6174 |
| 4 | 0 | 32 | 0.4511 < Q ≤ 0.4927 |
| 3 | 7 | 31 | 0.4096 < Q ≤ 0.4511 |
| 3 | 6 | 30 | 0.3680 < Q ≤ 0.4096 |
| 3 | 5 | 29 | 0.3264 < Q ≤ 0.3680 |
| 3 | 4 | 28 | 0.2848 < Q ≤ 0.3264 |
| 3 | 3 | 27 | 0.2433 < Q ≤ 0.2848 |
| 3 | 2 | 26 | 0.2017 < Q ≤ 0.2433 |
| 3 | 1 | 25 | 0.1601 < Q ≤ 0.2017 |
| 3 | 0 | 24 | 0.1463 < Q ≤ 0.1601 |
| 2 | 7 | 23 | 0.1324 < Q ≤ 0.1463 |
| 2 | 6 | 22 | 0.1186 < Q ≤ 0.1324 |
| 2 | 5 | 21 | 0.1047 < Q ≤ 0.1186 |
| 2 | 4 | 20 | 0.0908 < Q ≤ 0.1047 |
| 2 | 3 | 19 | 0.0770 < Q ≤ 0.0908 |
| 2 | 2 | 18 | 0.0631 < Q ≤ 0.0770 |
| 2 | 1 | 17 | 0.0493 < Q ≤ 0.0631 |
| 2 | 0 | 16 | 0.0447 < Q ≤ 0.0493 |
| 1 | 7 | 15 | 0.0400 < Q ≤ 0.0447 |
| 1 | 6 | 14 | 0.0354 < Q ≤ 0.0400 |
| 1 | 5 | 13 | 0.0308 < Q ≤ 0.0354 |
| 1 | 4 | 12 | 0.0262 < Q ≤ 0.0308 |
| 1 | 3 | 11 | 0.0216 < Q ≤ 0.0262 |
| 1 | 2 | 10 | 0.0169 < Q ≤ 0.0216 |
| 1 | 1 | 9 | 0.0123 < Q ≤ 0.0169 |
| 1 | 0 | 8 | 0.0108 < Q ≤ 0.0123 |
| 0 | 7 | 7 | 0.0092 < Q ≤ 0.0108 |
| 0 | 6 | 6 | 0.0077 < Q ≤ 0.0092 |
| 0 | 5 | 5 | 0.0062 < Q ≤ 0.0077 |
| 0 | 4 | 4 | 0.0046 < Q ≤ 0.0062 |
| 0 | 3 | 3 | 0.0031 < Q ≤ 0.0046 |
| 0 | 2 | 2 | 0.0015 < Q ≤ 0.0031 |
| 0 | 1 | 1 | Q ≤ 0.0015 |
| 0 | 0 | 0 | undefined/unknown |

#### –	GNSS-SSR-GriddedCorrection

The IE GNSS-SSR-GriddedCorrection is used by the location server to provide troposphere delay correction, together with the residual part of the STEC corrections and integrity information.

The parameters provided in IE GNSS-SSR-GriddedCorrection – except for SSR-GriddedCorrectionIntegrityParameters and TropoDelayIntegrityErrorBounds-r17 – are used as specified for Compact SSR Gridded Correction Message (e.g., message type 4073,9) in [43] and apply to all GNSSs.

-- ASN1START

GNSS-SSR-GriddedCorrection-r16 ::= SEQUENCE {

epochTime-r16								GNSS-SystemTime,

ssrUpdateInterval-r16						INTEGER (0..15),

iod-ssr-r16									INTEGER (0..15),

troposphericDelayQualityIndicator-r16		BIT STRING (SIZE(6))		OPTIONAL, -- Cond Tropo

correctionPointSetID-r16					INTEGER (0..16383),

gridList-r16								GridList-r16,

...,

[[

ssr-GriddedCorrectionIntegrityParameters-r17

SSR-GriddedCorrectionIntegrityParameters-r17

OPTIONAL  -- Need ON

]]

}

GridList-r16 ::= SEQUENCE (SIZE(1..64)) OF GridElement-r16

GridElement-r16 ::= SEQUENCE {

tropospericDelayCorrection-r16	TropospericDelayCorrection-r16	OPTIONAL, -- Need ON

stec-ResidualSatList-r16		STEC-ResidualSatList-r16		OPTIONAL, -- Need ON

...

}

TropospericDelayCorrection-r16 ::= SEQUENCE {

tropoHydroStaticVerticalDelay-r16		INTEGER (-256..255),

tropoWetVerticalDelay-r16				INTEGER (-128..127),

...,

[[

tropoDelayIntegrityErrorBounds-r17		TropoDelayIntegrityErrorBounds-r17

OPTIONAL -- Cond Integrity1

]]

}

STEC-ResidualSatList-r16 ::= SEQUENCE (SIZE(1..64)) OF STEC-ResidualSatElement-r16

STEC-ResidualSatElement-r16 ::= SEQUENCE {

svID-r16							SV-ID,

stecResidualCorrection-r16			CHOICE {

b7-r16					INTEGER (-64..63),

b16-r16					INTEGER (-32768..32767)

},

...

}

SSR-GriddedCorrectionIntegrityParameters-r17 ::= SEQUENCE {

probOnsetTroposphereFault-r17					INTEGER (0..255),

meanTroposphereFaultDuration-r17				INTEGER (1..256),

troposphereRangeErrorCorrelationTime-r17		INTEGER (1..255)	OPTIONAL, -- Need ON

troposphereRangeRateErrorCorrelationTime-r17	INTEGER (1..255)	OPTIONAL, -- Cond Integrity2

...

}

TropoDelayIntegrityErrorBounds-r17 ::= SEQUENCE {

meanTroposphereVerticalHydroStaticDelay-r17			INTEGER (0..255),

stdDevTroposphereVerticalHydroStaticDelay-r17		INTEGER (0..255),

meanTroposphereVerticalWetDelay-r17					INTEGER (0..255),

stdDevTroposphereVerticalWetDelay-r17				INTEGER (0..255),

meanTroposphereVerticalHydroStaticDelayRate-r17		INTEGER (0..255),

stdDevTroposphereVerticalHydroStaticDelayRate-r17	INTEGER (0..255),

meanTroposphereVerticalWetDelayRate-r17				INTEGER (0..255),

stdDevTroposphereVerticalWetDelayRate-r17			INTEGER (0..255),

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Tropo | The field is mandatory present if tropospericDelayCorrection is included in gridList. Otherwise it is not present. |
| Integrity1 | The field is mandatory present if SSR-GriddedCorrectionIntegrityParameters is present; otherwise it is not present. |
| Integrity2 | The field is mandatory present if troposphereRangeErrorCorrelationTime is present; otherwise it is not present. |

GNSS-SSR-GriddedCorrection field descriptions

epochTime This field specifies the epoch time of the gridded correction data. The gnss-TimeID in GNSS-SystemTime shall be the same as the GNSS-ID in IE GNSS-GenericAssistDataElement.

ssrUpdateInterval This field specifies the SSR Update Interval. The SSR Update Intervals for all SSR parameters start at time 00:00:00 of the GPS time scale. A change of the SSR Update Interval during the transmission of SSR data should ensure consistent data for a target device. See table Value of ssrUpdateInterval to SSR Update Interval relation in IE GNSS-SSR-OrbitCorrections.

iod-ssr This field specifies the Issue of Data number for the SSR data. A change of iod-ssr is used to indicate a change in the SSR generating configuration.

troposphericDelayQualityIndicator This field specifies the quality indicator of the tropospheric delay. The troposphere quality indicator is represented by a combination of CLASS and VALUE. The 3 MSB define the CLASS with a range of 0-7 and the 3 LSB define the VALUE with a range of 0-7. The troposphere quality indicator is computed by: SSR Troposphere Quality [mm] ≤ 3CLASS1+VALUE4-1 [mm] See Table 'Relationship between SSR troposphere quality and URA indicator and physical quantity' below.

correctionPointSetID This field provides the ID of the GNSS-SSR-CorrectionPoints set. The GNSS-SSR-GriddedCorrection are valid for the correction points provided in IE GNSS-SSR-CorrectionPoints with the same correctionPointSetID.

gridList This field provides the troposphere delay correction together with the residual part of the STEC corrections for up to 64 correction points defined in IE GNSS-SSR-CorrectionPoints. If the IE GNSS-SSR-CorrectionPoints, which belongs to the correctionPointSetID, includes the listOfCorrectionPoints, the gridList includes the same number of entries, and listed in the same order, as in the listOfCorrectionPoints. If the IE GNSS-SSR-CorrectionPoints, which belongs to this correctionPointSetID, includes the arrayOfCorrectionPoints the gridList includes the same number of entries, and listed in the same order, as defined by the enabled bits in the bitmaskOfGrids.

tropoHydroStaticVerticalDelay This field specifies the variation in the hydro static troposphere vertical delay relative to nominal value. The target device should add the constant nominal value of 2.3 m to calculate the tropospheric hydro-static vertical delay. Scale factor 0.004 m; range ±1.02 m.

tropoWetVerticalDelay This field specifies the variation in the wet troposphere vertical delay relative to nominal value. The target device should add the constant value of 0.252 m to calculate the tropospheric wet (non hydro-static) vertical delay. Scale factor 0.004 m; range ±0.508 m.

svID This field specifies the GNSS satellite for which the STEC residual corrections are provided.

stecResidualCorrection This field specifies the STEC residual correction. Scale factor 0.04 TECU; range ±2.52 TECU (b7) or ±1310.68 TECU (b16).

probOnsetTroposphereFault This field specifies the Probability of Onset of Troposphere Fault per Time Unit which is the probability of occurrence of troposphere error to exceed the residual error bound for more than the Time to Alert (TTA) This field specifies the onset probability that the residual range or range rate error exceeds a bound created using the minimum allowed inflation factor Kmin, and bounding parameters as mean + Kmin * stdDev where Kmin = normInv(irMaximum / 2) and irMaximum as provided in IE GNSS-Integrity-ServiceParameters. The probability is calculated by P=10-0.04n [hour-1] where n is the value of probOnsetTroposphereFault and the range is 10-10.2 to 1 per hour.

meanTroposphereFaultDuration This field specifies the Mean Troposphere Fault Duration which is the mean duration between when a troposphere integrity violation occurs, and the user is alerted through GNSS-Integrity-ServiceAlert (or the integrity violation is over). Scale factor 1 s; range 1-256 s.

troposphereRangeErrorCorrelationTime This field specifies the Troposphere Range Error Correlation Time which is the upper bound of the correlation time of the troposphere residual range error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

troposphereRangeRateErrorCorrelationTime This field specifies the Troposphere Range Rate Error Correlation Time which is the upper bound of the correlation time of the troposphere residual range rate error. The time is calculated using: t=10i, &i≤1801800+100(i-180), 180<&i≤234 7200+1000i-234, &i>234 [s] Range is 1-28,200 s.

meanTroposphereVerticalHydroStaticDelay This field specifies the Mean Troposphere Vertical Hydro Static Delay Error bound which is the mean value for an overbounding model that bounds the residual troposphere error in the vertical hydro static delay component. The bound is meanTroposphereVerticalHydroStaticDelay + K * stdDevTroposphereVerticalHydroStaticDelay and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevTroposphereVerticalHydroStaticDelay This field specifies the Standard Deviation Troposphere Vertical Hydro Static Delay Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere error in the vertical hydro static delay component. Scale factor 0.005 m; range 0-1.275 m.

meanTroposphereVerticalWetDelay This field specifies the Mean Troposphere Vertical Wet Static Delay Error bound which is the mean value for an overbounding model that bounds the residual troposphere error in the vertical wet delay component. The bound is meanTroposphereVerticalWetDelay + K * stdDevTroposphereVerticalWetDelay and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.005 m; range 0-1.275 m.

stdDevTroposphereVerticalWetDelay This field specifies the Standard Deviation Troposphere Vertical Wet Static Delay Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere error in the vertical wet delay component. Scale factor 0.005 m; range 0-1.275 m.

meanTroposphereVerticalHydroStaticDelayRate This field specifies the Mean Troposphere Vertical Hydro Static Delay Rate Error bound which is the mean value for an overbounding model that bounds the residual troposphere rate error in the vertical hydro static delay component. The bound is meanTroposphereVerticalHydroStaticDelayRate + K * stdDevTroposphereVerticalHydroStaticDelayRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevTroposphereVerticalHydroStaticDelayRate This field specifies the Standard Deviation Troposphere Vertical Hydro Static Delay Rate Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere rate error in the vertical hydro static delay component. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

meanTroposphereVerticalWetDelayRate This field specifies the Mean Troposphere Vertical Wet Static Delay Rate Error bound which is the mean value for an overbounding model that bounds the residual troposphere rate error in the vertical wet delay component. The bound is meanTroposphereVerticalWetDelayRate + K * stdDevTroposphereVerticalWetDelayRate and shall be so that the probability of it to be exceeded shall be lower than IRallocation for irMinimum < IRallocation < irMaximum, where K = normInv(IRallocation / 2) and irMinimum, irMaximum as provided in IE GNSS-Integrity-ServiceParameters. This IRallocation is a fraction of the Target Integrity Risk that represents the integrity risk budget available. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

stdDevTroposphereVerticalWetDelayRate This field specifies the Standard Deviation Troposphere Vertical Wet Static Delay Rate Error bound which is the standard deviation for an overbounding model that bounds the residual troposphere rate error in the vertical wet delay component. Scale factor 0.00005 m/s; range 0-0.01275 m/s.

Relationship between SSR troposphere quality and URA indicator and physical quantity

| CLASS | VALUE | Index | SSR troposphere quality indicator and SSR URA Q [mm] |
| --- | --- | --- | --- |
| 7 | 7 | 63 | 5466.50 < Q |
| 7 | 6 | 62 | 4919.75 < Q ≤ 5466.50 |
| 7 | 5 | 61 | 4373.75 < Q ≤ 4919.75 |
| 7 | 4 | 60 | 3826.25 < Q ≤ 4373.00 |
| 7 | 3 | 59 | 3279.50 < Q ≤ 3826.25 |
| 7 | 2 | 58 | 2732.75 < Q ≤ 3279.50 |
| 7 | 1 | 57 | 2186.00 < Q ≤ 2732.75 |
| 7 | 0 | 56 | 2003.75 < Q ≤ 2186.00 |
| 6 | 7 | 55 | 1821.50 < Q ≤ 2003.75 |
| 6 | 6 | 54 | 1639.25 < Q ≤ 1821.50 |
| 6 | 5 | 53 | 1457.00 < Q ≤ 1639.25 |
| 6 | 4 | 52 | 1274.75 < Q ≤ 1457.00 |
| 6 | 3 | 51 | 1092.50 < Q ≤ 1274.75 |
| 6 | 2 | 50 | 910.25 < Q ≤ 1092.50 |
| 6 | 1 | 49 | 728.00 < Q ≤ 910.25 |
| 6 | 0 | 48 | 667.25 < Q ≤ 728.00 |
| 5 | 7 | 47 | 606.50 < Q ≤ 667.25 |
| 5 | 6 | 46 | 545.75 < Q ≤ 606.50 |
| 5 | 5 | 45 | 485.00 < Q ≤ 545.75 |
| 5 | 4 | 44 | 424.25 < Q ≤ 485.00 |
| 5 | 3 | 43 | 363.50 < Q ≤ 425.25 |
| 5 | 2 | 42 | 302.75 < Q ≤ 363.50 |
| 5 | 1 | 41 | 242.00 < Q ≤ 302.75 |
| 5 | 0 | 40 | 221.75 < Q ≤ 242.00 |
| 4 | 7 | 39 | 201.50 < Q ≤ 221.75 |
| 4 | 6 | 38 | 181.25 < Q ≤ 201.50 |
| 4 | 5 | 37 | 161.00 < Q ≤ 181.25 |
| 4 | 4 | 36 | 140.75 < Q ≤ 161.00 |
| 4 | 3 | 35 | 120.50 < Q ≤ 140.75 |
| 4 | 2 | 34 | 100.25 < Q ≤ 120.50 |
| 4 | 1 | 33 | 80.00 < Q ≤ 100.25 |
| 4 | 0 | 32 | 73.25 < Q ≤ 80.00 |
| 3 | 7 | 31 | 66.50 < Q ≤ 73.25 |
| 3 | 6 | 30 | 59.75 < Q ≤ 66.50 |
| 3 | 5 | 29 | 53.00 < Q ≤ 59.75 |
| 3 | 4 | 28 | 46.25 < Q ≤ 53.00 |
| 3 | 3 | 27 | 39.50 < Q ≤ 46.25 |
| 3 | 2 | 26 | 32.75 < Q ≤ 39.50 |
| 3 | 1 | 25 | 26.00 < Q ≤ 32.75 |
| 3 | 0 | 24 | 23.75 < Q ≤ 26.00 |
| 2 | 7 | 23 | 21.50 < Q ≤ 23.75 |
| 2 | 6 | 22 | 19.25 < Q ≤ 21.50 |
| 2 | 5 | 21 | 17.00 < Q ≤ 19.25 |
| 2 | 4 | 20 | 14.75 < Q ≤ 17.00 |
| 2 | 3 | 19 | 12.50 < Q ≤ 14.75 |
| 2 | 2 | 18 | 10.25 < Q ≤ 12.50 |
| 2 | 1 | 17 | 8.00 < Q ≤ 10.25 |
| 2 | 0 | 16 | 7.25 < Q ≤ 8.00 |
| 1 | 7 | 15 | 6.50 < Q ≤ 7.25 |
| 1 | 6 | 14 | 5.75 < Q ≤ 6.50 |
| 1 | 5 | 13 | 5.00 < Q ≤ 5.75 |
| 1 | 4 | 12 | 4.25 < Q ≤ 5.00 |
| 1 | 3 | 11 | 3.50 < Q ≤ 4.25 |
| 1 | 2 | 10 | 2.75 < Q ≤ 3.50 |
| 1 | 1 | 9 | 2.00 < Q ≤ 2.75 |
| 1 | 0 | 8 | 1.75 < Q ≤ 2.00 |
| 0 | 7 | 7 | 1.50 < Q ≤ 1.75 |
| 0 | 6 | 6 | 1.25 < Q ≤ 1.50 |
| 0 | 5 | 5 | 1.00 < Q ≤ 1.25 |
| 0 | 4 | 4 | 0.75 < Q ≤ 1.00 |
| 0 | 3 | 3 | 0.50 < Q ≤ 0.75 |
| 0 | 2 | 2 | 0.25 < Q ≤ 0.50 |
| 0 | 1 | 1 | Q ≤ 0.25 |
| 0 | 0 | 0 | undefined/unknown |

#### –	NavIC-DifferentialCorrections

The IE NavIC-DifferentialCorrections parameters provide users with sets of correction terms that apply to the clock and ephemeris data transmitted by other satellites in the AutoNav mode as defined in [38] under clause 6.2.6.

-- ASN1START

NavIC-DifferentialCorrections-r16 ::= SEQUENCE {

navic-RefTOWC-r16				INTEGER (0..50400),

navic-CorrectionListAutoNav-r16	NavIC-CorrectionListAutoNav-r16,

...

}

NavIC-CorrectionListAutoNav-r16 ::= SEQUENCE (SIZE (1..64)) OF NavIC-CorrectionElementAutoNav-r16

NavIC-CorrectionElementAutoNav-r16 ::= SEQUENCE {

svID						SV-ID,

navic-Tod-r16				INTEGER (0..65535),

navic-iodec-r16				INTEGER (0..255),

navic-UDRAI-r16				INTEGER (-16..15),

navic-UDRArateI-r16			INTEGER (-16..15),

navic-EDC-r16				NavIC-EDC-r16,

navic-CDC-r16				NavIC-CDC-r16,

...

}

NavIC-EDC-r16 ::= SEQUENCE {

navic-AlphaEDC-r16				INTEGER (-8192..8191),

navic-BetaEDC-r16				INTEGER (-8192..8191),

navic-GammaEDC-r16				INTEGER (-16384..16383),

navic-AoIcorrection-r16			INTEGER (-2048..2047),

navic-AoRAcorrection-r16		INTEGER (-2048..2047),

navic-SemiMajorcorrection-r16	INTEGER (-2048..2047),

...

}

NavIC-CDC-r16 ::= SEQUENCE {

navic-ClockBiasCorrection-r16	INTEGER (-4096..4095),

navic-ClockDriftCorrection-r16	INTEGER (-128..127),

...

}

-- ASN1STOP

NavIC-DifferentialCorrections field descriptions

navic-RefTOWC The transmission timing of the navigation message provided through the Time of Week Count (TOWC) corresponding to the given set of grid ionospheric parameters. It indicates the number of 12 second counts represented in 17 bits. The TOW count value ranges from 1 to 50400 to cover one entire week. The Time of Week (TOW) in seconds is obtained by multiplying TOWC with 12 as defined in [38], clause 5.7.

navic-Tod This field indicates the NavIC Time of Differential Correction in seconds. Scale factor 16 seconds

navic-iodec This field indicates Issue of Data Ephemeris and Clock which provides the user with a convenient means of detecting any change in the ephemeris and clock parameters as described under clause 6.2.1.3 in [38]

navic-UDRAI This field indicates the index for the User Differential Range Accuracy (in metres) value which enables users to estimate the accuracy obtained after differential corrections are applied as described under clause 6.2.6 in [38]

navic-UDRArateI This field indicates the index for the change rate of User Differential Range Accuracy (metres/second)value which enables users to estimate the accuracy obtained after differential corrections are applied as described under clause 6.2.6 in [38]

navic-AlphaEDC This field indicates the Alpha correction to Ephemeris parameter (Δα), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [38]. Scale factor 2–34

navic-BetaEDC This field indicates Beta correction to Ephemeris parameter (Δβ), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [38]. Scale factor 2–34

navic-GammaEDC This field indicates the Gamma correction to Ephemeris parameter (Δγ), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [38]. Scale factor 2–32 semi-circles.

navic-AoIcorrection This field indicates the Angle of inclination correction (Δi), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [38]. Scale factor 2–32 semi-circles.

navic-AoRAcorrection This field indicates the Angle of right ascension correction (ΔΩ), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [38]. Scale factor 2–32 semi-circles.

navic-SemiMajorcorrection This field indicates the Semi-major correction (ΔA), which is one of the six keplerian elements defining the ephemeris differential corrections (EDC) for NavIC as defined under clause 6.1.3.5 in [38]. Scale factor 2–9 metres.

navic-ClockBiasCorrection This field indicates correction to the satellite clock bias coefficient (δaf0), which is one of the two Satellite clock differential corrections (CDC) containing corrections to the NavIC satellite clock polynomial coefficients as defined under clause 6.1.3.5 in [38]. Scale factor 2–35 seconds.

navic-ClockDriftCorrection This field indicates correction to the satellite clock drift coefficient (δaf1), which is one of the two Satellite clock differential corrections (CDC) containing corrections to the NavIC satellite clock polynomial coefficients as defined under clause 6.1.3.5 in [38]. Scale factor 2–51 sec / sec.

#### –	NavIC-GridModelParameter

-- ASN1START

NavIC-GridModelParameter-r16 ::= SEQUENCE {

navic-RefTOWC-r16	INTEGER (0..50400),

regionMasked-r16	INTEGER (0..1023),

regionIgpList-r16	RegionIgpList-r16,

...

}

RegionIgpList-r16 ::= SEQUENCE (SIZE (1..16)) OF RegionIgpElement-r16

RegionIgpElement-r16 ::= SEQUENCE {

regionID-r16	INTEGER (0..15),

givei1-r16		INTEGER (0..15),

givd1-r16		INTEGER (0..511),

givei2-r16		INTEGER (0..15),

givd2-r16		INTEGER (0..511),

givei3-r16		INTEGER (0..15),

givd3-r16		INTEGER (0..511),

givei4-r16		INTEGER (0..15),

givd4-r16		INTEGER (0..511),

givei5-r16		INTEGER (0..15),

givd5-r16		INTEGER (0..511),

givei6-r16		INTEGER (0..15),

givd6-r16		INTEGER (0..511),

givei7-r16		INTEGER (0..15),

givd7-r16		INTEGER (0..511),

givei8-r16		INTEGER (0..15),

givd8-r16		INTEGER (0..511),

givei9-r16		INTEGER (0..15),

givd9-r16		INTEGER (0..511),

givei10-r16		INTEGER (0..15),

givd10-r16		INTEGER (0..511),

givei11-r16		INTEGER (0..15),

givd11-r16		INTEGER (0..511),

givei12-r16		INTEGER (0..15),

givd12-r16		INTEGER (0..511),

givei13-r16		INTEGER (0..15),

givd13-r16		INTEGER (0..511),

givei14-r16		INTEGER (0..15),

givd14-r16		INTEGER (0..511),

givei15-r16		INTEGER (0..15),

givd15-r16		INTEGER (0..511),

...

}

-- ASN1STOP

NavIC-GridModelParameter field descriptions

navic-RefTOWC The transmission timing of the navigation message provided through the Time of Week Count (TOWC) corresponding to the given set of grid ionospheric parameters. It indicates the number of 12 second counts represented in 17 bits. The TOW count value ranges from 1 to 50400 to cover one entire week. The Time of Week (TOW) in seconds is obtained by multiplying TOWC with 12 as defined in [38], clause 5.7.

regionMasked Total 90 Ionospheric Grid Points(IGP) are defined in [38] clause 6.2.3 table 25. 15 IGP points are grouped into a single region. The region masked indicates the total number of regions for which the corrections are provided. For the current service area of the IRNSS, regions masked are 6.

regionIgpList This list provides the set of IGPs corresponding to each region. Up to 6 instances (0 to 5) are used in this version of the specification. The values 6 to 15 are reserved for future use.

regionID regionID along with index of the IGPS point corresponding gives the location of IGPS point as defined in [38], table 25, clause 6.2.3.

givei1, give2, .. , give15 This field indicates the Grid Ionospheric Vertical Error Index (GIVEI) which is used to describe the delay correction accuracy at ionospheric grid point indicated by the igp-ID, the mapping between GIVEI and GIVE is defined in [38], clause 6.2.2 and table 27.

givd1, givd2, … , givd15 This field indicates the Grid Ionospheric Vertical Delay (GIVD) as defined in [38], clause 5.3.3.8.1, i.e. the vertical delay at the corresponding Ionospheric Grid points (IGPs) indicated by igp-ID. The scale factor is 0.125 metre.

#### 6.5.2.3	GNSS Assistance Data Request

#### –	A-GNSS-RequestAssistanceData

The IE A-GNSS-RequestAssistanceData is used by the target device to request GNSS assistance data from a location server.

-- ASN1START

A-GNSS-RequestAssistanceData ::= SEQUENCE {

gnss-CommonAssistDataReq		GNSS-CommonAssistDataReq		OPTIONAL, -- Cond CommonADReq

gnss-GenericAssistDataReq		GNSS-GenericAssistDataReq		OPTIONAL, -- Cond GenADReq

...,

[[

gnss-PeriodicAssistDataReq-r15

GNSS-PeriodicAssistDataReq-r15	OPTIONAL -- Cond PerADReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| CommonADReq | The field is mandatory present if the target device requests GNSS-CommonAssistData; otherwise it is not present. |
| GenADReq | This field is mandatory present if the target device requests GNSS-GenericAssistData for one or more specific GNSS; otherwise it is not present. |
| PerADReq | This field is mandatory present if the target device requests periodic GNSS assistance data delivery. This field may only be included if any of the fields are included in IE GNSS-GenericAssistDataReq: - GNSS-RTK-ObservationsReq, - GLO-RTK-BiasInformationReq, - GNSS-RTK-MAC-CorrectionDifferencesReq, - GNSS-RTK-ResidualsReq, - GNSS-RTK-FKP-GradientsReq, - GNSS-SSR-OrbitCorrectionsReq, - GNSS-SSR-ClockCorrectionsReq, - GNSS-SSR-CodeBiasReq. - GNSS-SSR-URA-Req, - GNSS-SSR-PhaseBiasReq, - GNSS-SSR-STEC-CorrectionReq, - GNSS-SSR-GriddedCorrectionReq, or - GNSS-Integrity-ServiceAlerReq. |

#### –	GNSS-CommonAssistDataReq

The IE GNSS-CommonAssistDataReq is used by the target device to request assistance data that are applicable to any GNSS from a location server.

-- ASN1START

GNSS-CommonAssistDataReq ::= SEQUENCE {

gnss-ReferenceTimeReq				GNSS-ReferenceTimeReq

OPTIONAL, -- Cond RefTimeReq

gnss-ReferenceLocationReq			GNSS-ReferenceLocationReq

OPTIONAL, -- Cond RefLocReq

gnss-IonosphericModelReq			GNSS-IonosphericModelReq

OPTIONAL, -- Cond IonoModReq

gnss-EarthOrientationParametersReq	GNSS-EarthOrientationParametersReq

OPTIONAL, -- Cond EOPReq

...,

[[

gnss-RTK-ReferenceStationInfoReq-r15

GNSS-RTK-ReferenceStationInfoReq-r15

OPTIONAL, -- Cond ARPReq

gnss-RTK-AuxiliaryStationDataReq-r15

GNSS-RTK-AuxiliaryStationDataReq-r15

OPTIONAL -- Cond AuxARPReq

]],

[[

gnss-SSR-CorrectionPointsReq-r16

GNSS-SSR-CorrectionPointsReq-r16

OPTIONAL -- Cond PointsReq

]],

[[

gnss-Integrity-ServiceParametersReq-r17

GNSS-Integrity-ServiceParametersReq-r17

OPTIONAL, 	-- Cond IntServiceReq

gnss-Integrity-ServiceAlertReq-r17

GNSS-Integrity-ServiceAlertReq-r17

OPTIONAL 	-- Cond IntAlertReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| RefTimeReq | The field is mandatory present if the target device requests GNSS-ReferenceTime; otherwise it is not present. |
| RefLocReq | This field is mandatory present if the target device requests GNSS-ReferenceLocation; otherwise it is not present. |
| IonoModReq | This field is mandatory present if the target device requests GNSS-IonosphericModel; otherwise it is not present. |
| EOPReq | This field is mandatory present if the target device requests GNSS-EarthOrientationParameters; otherwise it is not present. |
| ARPReq | This field is mandatory present if the target device requests GNSS-RTK-ReferenceStationInfo; otherwise it is not present. |
| AuxARPReq | This field is mandatory present if the target device requests GNSS-RTK-AuxiliaryStationData; otherwise it is not present. |
| PointsReq | This field is mandatory present if the target device requests GNSS-SSR-CorrectionPoints; otherwise it is not present. |
| IntServiceReq | This field is mandatory present if the target device requests GNSS-Integrity-ServiceParameters; otherwise it is not present. |
| IntAlertReq | This field is mandatory present if the target device requests GNSS-Integrity-ServiceAlert; otherwise it is not present. |

#### –	GNSS-GenericAssistDataReq

The IE GNSS-GenericAssistDataReq is used by the target device to request assistance data from a location server for one or more specific GNSSs. The specific GNSS for which the assistance data are requested is indicated by the IE GNSS-ID and (if applicable) by the IE SBAS-ID. Assistance for up to 16 GNSSs can be requested.

-- ASN1START

GNSS-GenericAssistDataReq ::= SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataReqElement

GNSS-GenericAssistDataReqElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-ID							SBAS-ID							OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsReq				GNSS-TimeModelListReq			OPTIONAL, -- Cond TimeModReq

gnss-DifferentialCorrectionsReq	GNSS-DifferentialCorrectionsReq	OPTIONAL, -- Cond DGNSS-Req

gnss-NavigationModelReq			GNSS-NavigationModelReq			OPTIONAL, -- Cond NavModReq

gnss-RealTimeIntegrityReq		GNSS-RealTimeIntegrityReq		OPTIONAL, -- Cond RTIReq

gnss-DataBitAssistanceReq		GNSS-DataBitAssistanceReq		OPTIONAL, -- Cond DataBitsReq

gnss-AcquisitionAssistanceReq	GNSS-AcquisitionAssistanceReq	OPTIONAL, -- Cond AcquAssistReq

gnss-AlmanacReq					GNSS-AlmanacReq					OPTIONAL, -- Cond AlmanacReq

gnss-UTCModelReq				GNSS-UTC-ModelReq				OPTIONAL, -- Cond UTCModReq

gnss-AuxiliaryInformationReq	GNSS-AuxiliaryInformationReq	OPTIONAL, -- Cond AuxInfoReq

...,

[[

bds-DifferentialCorrectionsReq-r12

BDS-DifferentialCorrectionsReq-r12

OPTIONAL,	-- Cond DBDS-Req

bds-GridModelReq-r12		BDS-GridModelReq-r12			OPTIONAL	-- Cond BDS-GridModReq

]],

[[

gnss-RTK-ObservationsReq-r15

GNSS-RTK-ObservationsReq-r15	OPTIONAL,	-- Cond RTK-OSR-Req

glo-RTK-BiasInformationReq-r15

GLO-RTK-BiasInformationReq-r15	OPTIONAL,	-- Cond GLO-CPB-Req

gnss-RTK-MAC-CorrectionDifferencesReq-r15

GNSS-RTK-MAC-CorrectionDifferencesReq-r15

OPTIONAL,	-- Cond MAC-Req

gnss-RTK-ResidualsReq-r15	GNSS-RTK-ResidualsReq-r15		OPTIONAL,	-- Cond Res-Req

gnss-RTK-FKP-GradientsReq-r15

GNSS-RTK-FKP-GradientsReq-r15	OPTIONAL,	-- Cond FKP-Req

gnss-SSR-OrbitCorrectionsReq-r15

GNSS-SSR-OrbitCorrectionsReq-r15

OPTIONAL, 	-- Cond OC-Req

gnss-SSR-ClockCorrectionsReq-r15

GNSS-SSR-ClockCorrectionsReq-r15

OPTIONAL, 	-- Cond CC-Req

gnss-SSR-CodeBiasReq-r15	GNSS-SSR-CodeBiasReq-r15		OPTIONAL 	-- Cond CB-Req

]],

[[

gnss-SSR-URA-Req-r16		GNSS-SSR-URA-Req-r16			OPTIONAL,	-- Cond URA-Req

gnss-SSR-PhaseBiasReq-r16	GNSS-SSR-PhaseBiasReq-r16		OPTIONAL,	-- Cond PB-Req

gnss-SSR-STEC-CorrectionReq-r16

GNSS-SSR-STEC-CorrectionReq-r16	OPTIONAL,	-- Cond STEC-Req

gnss-SSR-GriddedCorrectionReq-r16	GNSS-SSR-GriddedCorrectionReq-r16

OPTIONAL,	-- Cond Grid-Req

navic-DifferentialCorrectionsReq-r16

NavIC-DifferentialCorrectionsReq-r16

OPTIONAL,	-- Cond DNavIC-Req

navic-GridModelReq-r16		NavIC-GridModelReq-r16		OPTIONAL	-- Cond NavIC-GridModReq

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModReq | The field is mandatory present if the target device requests GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Req | The field is mandatory present if the target device requests GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModReq | The field is mandatory present if the target device requests GNSS-NavigationModel; otherwise it is not present. |
| RTIReq | The field is mandatory present if the target device requests GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsReq | The field is mandatory present if the target device requests GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistReq | The field is mandatory present if the target device requests GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacReq | The field is mandatory present if the target device requests GNSS-Almanac; otherwise it is not present. |
| UTCModReq | The field is mandatory present if the target device requests GNSS-UTCModel; otherwise it is not present. |
| AuxInfoReq | The field is mandatory present if the target device requests GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Req | The field is mandatory present if the target device requests BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| BDS-GridModReq | The field is mandatory present if the target device requests BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| RTK-OSR-Req | The field is mandatory present if the target device requests GNSS-RTK-Observations; otherwise it is not present. |
| GLO-CPB-Req | The field is mandatory present if the target device requests GLO-RTK-BiasInformation; otherwise it is not present. |
| MAC-Req | The field is mandatory present if the target device requests GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Req | The field is mandatory present if the target device requests GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Req | The field is mandatory present if the target device requests GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Req | The field is mandatory present if the target device requests GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Req | The field is mandatory present if the target device requests GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Req | The field is mandatory present if the target device requests GNSS-SSR-CodeBias; otherwise it is not present. |
| URA-Req | The field is mandatory present if the target device requests GNSS-SSR-URA; otherwise it is not present. |
| PB-Req | The field is mandatory present if the target device requests GNSS-SSR-PhaseBias; otherwise it is not present. |
| STEC-Req | The field is mandatory present if the target device requests GNSS-SSR-STEC-Correction; otherwise it is not present. |
| Grid-Req | The field is mandatory present if the target device requests GNSS-SSR-GriddedCorrection; otherwise it is not present. |
| DNavIC-Req | The field is mandatory present if the target device requests NavIC-DifferentialCorrections; otherwise it is not present. This field may only be present if the gnss-ID indicates 'navic'. |
| NavIC-GridModReq | The field is mandatory present if the target device requests NavIC-GridModel; otherwise it is not present. This field may only be present if the gnss-ID indicates 'navic'. |

#### –	GNSS-PeriodicAssistDataReq

The IE GNSS-PeriodicAssistDataReq is used by the target device to request periodic assistance data delivery from a location server.

-- ASN1START

GNSS-PeriodicAssistDataReq-r15 ::= SEQUENCE {

gnss-RTK-PeriodicObservationsReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pOSR

glo-RTK-PeriodicBiasInformationReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCPB

gnss-RTK-MAC-PeriodicCorrectionDifferencesReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pMAC

gnss-RTK-PeriodicResidualsReq-r15		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pRes

gnss-RTK-FKP-PeriodicGradientsReq-r15	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pFKP

gnss-SSR-PeriodicOrbitCorrectionsReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pOC

gnss-SSR-PeriodicClockCorrectionsReq-r15

GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCC

gnss-SSR-PeriodicCodeBiasReq-r15		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pCB

...,

[[

gnss-SSR-PeriodicURA-Req-r16			GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pURA

gnss-SSR-PeriodicPhaseBiasReq-r16		GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pPB

gnss-SSR-PeriodicSTEC-CorrectionReq-r16	GNSS-PeriodicControlParam-r15	OPTIONAL, -- Cond pSTEC

gnss-SSR-PeriodicGriddedCorrectionReq-r16

GNSS-PeriodicControlParam-r15	OPTIONAL  -- Cond pGrid

]],

[[

gnss-Integrity-PeriodicServiceAlertReq-r17

GNSS-PeriodicControlParam-r15	OPTIONAL  -- Cond pDNU

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| pOSR | The field is mandatory present if the target device requests periodic GNSS-RTK-Observations; otherwise it is not present. |
| pCPB | The field is mandatory present if the target device requests periodic GLO-RTK-BiasInformation; otherwise it is not present. |
| pMAC | The field is mandatory present if the target device requests periodic GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| pRes | The field is mandatory present if the target device requests periodic GNSS-RTK-Residuals; otherwise it is not present. |
| pFKP | The field is mandatory present if the target device requests periodic GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| pOC | The field is mandatory present if the target device requests periodic GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| pCC | The field is mandatory present if the target device requests periodic GNSS-SSR-ClockCorrections; otherwise it is not present. |
| pCB | The field is mandatory present if the target device requests periodic GNSS-SSR-CodeBias; otherwise it is not present. |
| pURA | The field is mandatory present if the target device requests periodic GNSS-SSR-URA; otherwise it is not present. |
| pPB | The field is mandatory present if the target device requests periodic GNSS-SSR-PhaseBias; otherwise it is not present. |
| pSTEC | The field is mandatory present if the target device requests periodic GNSS-SSR-STEC-Correction; otherwise it is not present. |
| pGrid | The field is mandatory present if the target device requests periodic GNSS-SSR-GriddedCorrection; otherwise it is not present. |
| pDNU | The field is mandatory present if the target device requests periodic GNSS-Integrity-ServiceAlert; otherwise it is not present. |

#### 6.5.2.4	GNSS Assistance Data Request Elements

#### –	GNSS-ReferenceTimeReq

The IE GNSS-ReferenceTimeReq is used by the target device to request the GNSS-ReferenceTime assistance from the location server.

-- ASN1START

GNSS-ReferenceTimeReq ::= SEQUENCE {

gnss-TimeReqPrefList	SEQUENCE (SIZE (1..8)) OF GNSS-ID,

gps-TOW-assistReq		BOOLEAN								OPTIONAL, -- Cond gps

notOfLeapSecReq			BOOLEAN								OPTIONAL, -- Cond glonass

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| gps | The field is mandatory present if gnss-TimeReqPrefList includes a GNSS-ID= 'gps'; otherwise it is not present. |
| glonass | The field is mandatory present if gnss-TimeReqPrefList includes a GNSS-ID= 'glonass'; otherwise it is not present. |

GNSS-ReferenceTimeReq field descriptions

gnss-TimeReqPrefList This field is used by the target device to request the system time for a specific GNSS, specified by GNSS-ID in the order of preference. The first GNSS-ID in the list is the most preferred GNSS for reference time, the second GNSS-ID is the second most preferred, etc.

gps-TOW-assistReq This field is used by the target device to request the gps-TOW-Assist field in GNSS-SystemTime. TRUE means requested.

notOfLeapSecReq This field is used by the target device to request the notificationOfLeapSecond field in GNSS-SystemTime. TRUE means requested.

#### –	GNSS-ReferenceLocationReq

The IE GNSS-ReferenceLocationReq is used by the target device to request the GNSS-ReferenceLocation assistance from the location server.

-- ASN1START

GNSS-ReferenceLocationReq ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-IonosphericModelReq

The IE GNSS-IonosphericModelReq is used by the target device to request the GNSS-IonosphericModel assistance from the location server.

-- ASN1START

GNSS-IonosphericModelReq ::=	SEQUENCE {

klobucharModelReq		BIT STRING (SIZE(2))	OPTIONAL,	-- Cond klobuchar

neQuickModelReq			NULL					OPTIONAL,	-- Cond	nequick

...,

[[	klobucharModel2Req-r16 	NULL				OPTIONAL	-- Cond klobuchar2

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| klobuchar | The field is mandatory present if the target device requests klobucharModel; otherwise it is not present. The BIT STRING defines the dataID requested, defined in IE KlobucharModelParameter. |
| nequick | The field is mandatory present if the target device requests neQuickModel; otherwise it is not present. |
| klobuchar2 | The field is mandatory present if the target device requests klobucharModel2; otherwise it is not present. |

#### –	GNSS-EarthOrientationParametersReq

The IE GNSS-EarthOrientationParametersReq is used by the target device to request the GNSS-EarthOrientationParameters assistance from the location server.

-- ASN1START

GNSS-EarthOrientationParametersReq ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-RTK-ReferenceStationInfoReq

The IE GNSS-RTK-ReferenceStationInfoReq is used by the target device to request the GNSS-RTK-ReferenceStationInfo assistance from the location server.

-- ASN1START

GNSS-RTK-ReferenceStationInfoReq-r15 ::= SEQUENCE {

antennaDescriptionReq-r15			BOOLEAN,

antennaHeightReq-r15				BOOLEAN,

physicalReferenceStationReq-r15		BOOLEAN,

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-ReferenceStationInfoReq field descriptions

antennaDescriptionReq This field specifies whether or not the location server is requested to include the field AntennaDescription in the GNSS-RTK-ReferenceStationInfo IE. TRUE means requested.

antennaHeightReq This field specifies whether or not the location server is requested to include the field antennaHeight in the GNSS-RTK-ReferenceStationInfo IE. TRUE means requested.

physicalReferenceStationReq This field specifies whether or not the location server is requested to include the field physical-reference-station-info in the GNSS-RTK-ReferenceStationInfo IE. TRUE means requested.

stationID This field specifies the Station ID for which the GNSS-RTK-ReferenceStationInfo is requested.

#### –	GNSS-RTK-AuxiliaryStationDataReq

The IE GNSS-RTK-AuxiliaryStationDataReq is used by the target device to request the GNSS-RTK-AuxiliaryStationData assistance from the location server.

-- ASN1START

GNSS-RTK-AuxiliaryStationDataReq-r15 ::= SEQUENCE {

master-referenceStationID-r15		GNSS-ReferenceStationID-r15		OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-AuxiliaryStationDataReq field descriptions

master-referenceStationID This field specifies the Master Reference Station ID for which the Auxiliary Stations are requested.

#### –	GNSS-SSR-CorrectionPointsReq

The IE GNSS-SSR-CorrectionPointsReq is used by the target device to request the GNSS-SSR-CorrectionPoints assistance from the location server.

-- ASN1START

GNSS-SSR-CorrectionPointsReq-r16 ::=	SEQUENCE {

correctionPointSetID-Req-r16			INTEGER (0..16383)			OPTIONAL,

...

}

-- ASN1STOP

GNSS-SSR-CorrectionPointsReq field descriptions

correctionPointSetID-Req This field specifies the ID of the Atmospheric Correction Point set for which the GNSS-SSR-CorrectionPoints are requested.

#### –	GNSS-Integrity-ServiceParametersReq

The IE GNSS-Integrity-ServiceParametersReq is used by the target device to request the GNSS-Integrity-ServiceParameters assistance from the location server.

-- ASN1START

GNSS-Integrity-ServiceParametersReq-r17 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-Integrity-ServiceAlertReq

The IE GNSS-Integrity-ServiceAlertReq is used by the target device to request the GNSS-Integrity-ServiceAlert assistance from the location server.

-- ASN1START

GNSS-Integrity-ServiceAlertReq-r17 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-TimeModelListReq

The IE GNSS-TimeModelListReq is used by the target device to request the GNSS-TimeModelElement assistance from the location server.

-- ASN1START

GNSS-TimeModelListReq ::= SEQUENCE (SIZE(1..15)) OF GNSS-TimeModelElementReq

GNSS-TimeModelElementReq ::= SEQUENCE {

gnss-TO-IDsReq	INTEGER (1..15),

deltaTreq		BOOLEAN,

...

}

-- ASN1STOP

GNSS-TimeModelElementReq field descriptions

gnss-TO-IDsReq This field specifies the requested gnss-TO-ID. The meaning and encoding is the same as the gnss-TO-ID field in the GNSS-TimeModelElement IE.

deltaTreq This field specifies whether or not the location server is requested to include the deltaT field in the GNSS-TimeModelElement IE. TRUE means requested.

#### –	GNSS-DifferentialCorrectionsReq

The IE GNSS-DifferentialCorrectionsReq is used by the target device to request the GNSS-DifferentialCorrections assistance from the location server.

-- ASN1START

GNSS-DifferentialCorrectionsReq ::=	SEQUENCE {

dgnss-SignalsReq			GNSS-SignalIDs,

dgnss-ValidityTimeReq		BOOLEAN,

...

}

-- ASN1STOP

GNSS-DifferentialCorrectionsReq field descriptions

dgnss-SignalsReq This field specifies the GNSS Signal(s) for which the GNSS-DifferentialCorrections are requested. A one-value at a bit position means DGNSS corrections for the specific signal are requested; a zero-value means not requested. The target device shall set a maximum of three bits to value 'one'.

dgnss-ValidityTimeReq This field specifies whether the udreGrowthRate and udreValidityTime in GNSS-DifferentialCorrections are requested or not. TRUE means requested.

#### –	GNSS-NavigationModelReq

The IE GNSS-NavigationModelReq is used by the target device to request the GNSS-NavigationModel assistance from the location server.

-- ASN1START

GNSS-NavigationModelReq ::=	CHOICE {

storedNavList		StoredNavListInfo,

reqNavList			ReqNavListInfo,

...

}

StoredNavListInfo ::= SEQUENCE {

gnss-WeekOrDay			INTEGER (0..4095),

gnss-Toe				INTEGER (0..255),

t-toeLimit				INTEGER (0..15),

satListRelatedDataList	SatListRelatedDataList	OPTIONAL,

...

}

SatListRelatedDataList ::= SEQUENCE (SIZE (1..64)) OF SatListRelatedDataElement

SatListRelatedDataElement ::= SEQUENCE {

svID				SV-ID,

iod				BIT STRING (SIZE(11)),

clockModelID		INTEGER (1..8)			OPTIONAL,

orbitModelID		INTEGER (1..8)			OPTIONAL,

...

}

ReqNavListInfo ::=	SEQUENCE {

svReqList				BIT STRING (SIZE (64)),

clockModelID-PrefList	SEQUENCE (SIZE (1..8)) OF	INTEGER (1..8)		OPTIONAL,

orbitModelID-PrefList	SEQUENCE (SIZE (1..8)) OF	INTEGER (1..8)		OPTIONAL,

addNavparamReq			BOOLEAN				OPTIONAL,	-- Cond orbitModelID-2

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| orbitModelID-2 | The field is mandatory present if orbitModelID-PrefList is absent or includes a Model-ID = '2'; otherwise it is not present. |

GNSS-NavigationModelReq field descriptions

storedNavList This list provides information to the location server about which GNSS-NavigationModel data the target device has currently stored for the particular GNSS indicated by GNSS-ID.

reqNavList This list provides information to the location server which GNSS-NavigationModel data are requested by the target device.

gnss-WeekOrDay If GNSS-ID does not indicate 'glonass', this field defines the GNSS Week number of the assistance currently held by the target device. If GNSS-ID is set to 'glonass', this field defines the calendar number of day within the four-year interval starting from 1st of January in a leap year, as defined by the parameter NT in [9] of the assistance currently held by the target device.

gnss-Toe If GNSS-ID does not indicate 'glonass', this field defines the GNSS time of ephemeris in hours of the latest ephemeris set contained by the target device. If GNSS-ID is set to 'glonass', this field defines the time of ephemeris in units of 15 minutes of the latest ephemeris set contained by the target device (range 0 to 95 representing time values between 0 and 1425 minutes). In this case, values 96 to 255 shall not be used by the sender.

t-toeLimit If GNSS-ID does not indicate 'glonass', this IE defines the ephemeris age tolerance of the target device in units of hours. If GNSS-ID is set to 'glonass', this IE defines the ephemeris age tolerance of the target device in units of 30 minutes.

satListRelatedDataList This list defines the clock and orbit models currently held by the target device for each SV. This field is not included if the target device does not have any stored clock and orbit models for any SV.

svID This field identifies the particular GNSS satellite.

iod This field identifies the issue of data currently held by the target device.

clockModelID, orbitModelID These fields define the clock and orbit model number currently held by the target device. If these fields are absent, the default interpretation of the table GNSS-ID to clockModelID & orbitModelID relation below applies.

svReqList This field defines the SV for which the navigation model assistance is requested. Each bit position in this BIT STRING represents a SV-ID. Bit 0 represents SV-ID=0 and bit 63 represents SV-ID=63. A one-value at a bit position means the navigation model data for the corresponding SV-ID is requested, a zero-value means not requested.

clockModelIDPrefList, orbitModelID-PrefList These fields define the Model-IDs of the clock and orbit models that the target device wishes to obtain in the order of preference. The first Model-ID in the list is the most preferred model, the second Model-ID the second most preferred, etc. If these fields are absent, the default interpretation of the table GNSS-ID to clockModelID-PrefList & orbitModelIDPrefList relation below applies.

addNavparamReq This field specifies whether the location server is requested to include the addNAVparam fields in GNSS-NavigationModel IE (NavModel-NAVKeplerianSet field) or not. TRUE means requested.

GNSS-ID to clockModelID & orbitModelID relation

| GNSS-ID | clockModelID | orbitModelID |
| --- | --- | --- |
| gps | 2 | 2 |
| sbas | 5 | 5 |
| qzss | 2 | 2 |
| galileo | 1 | 1 |
| glonass | 4 | 4 |
| bds | 6 | 6 |
| navic | 8 | 8 |

GNSS-ID to clockModelID-PrefList & orbitModelID-PrefList relation

| GNSS-ID | clockModelID-PrefList | orbitModelID-PrefList |
| --- | --- | --- |
| gps | Model-2 | Model-2 |
| sbas | Model-5 | Model-5 |
| qzss | Model-2 | Model-2 |
| galileo | Model-1 | Model-1 |
| glonass | Model-4 | Model-4 |
| bds | Model-6 | Model-6 |
| navic | Model-8 | Model-8 |

#### –	GNSS-RealTimeIntegrityReq

The IE GNSS-RealTimeIntegrityReq is used by the target device to request the GNSS-RealTimeIntegrity assistance from the location server.

-- ASN1START

GNSS-RealTimeIntegrityReq ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-DataBitAssistanceReq

The IE GNSS-DataBitAssistanceReq is used by the target device to request the GNSS-DataBitAssistance assistance from the location server.

-- ASN1START

GNSS-DataBitAssistanceReq ::=	SEQUENCE {

gnss-TOD-Req		INTEGER (0..3599),

gnss-TOD-FracReq	INTEGER (0..999)		OPTIONAL,

dataBitInterval		INTEGER (0..15),

gnss-SignalType		GNSS-SignalIDs,

gnss-DataBitsReq	GNSS-DataBitsReqSatList	OPTIONAL,

...

}

GNSS-DataBitsReqSatList ::= SEQUENCE (SIZE(1..64)) OF GNSS-DataBitsReqSatElement

GNSS-DataBitsReqSatElement ::= SEQUENCE {

svID				SV-ID,

...

}

-- ASN1STOP

GNSS-DataBitAssistanceReq field descriptions

gnss-TOD-Req This field specifies the reference time for the first data bit requested in GNSS specific system time, modulo 1 hour. Scale factor 1 second.

gnss-TOD-FracReq This field specifies the fractional part of gnss-TOD-Req in 1-milli-second resolution. Scale factor 1 millisecond.

dataBitInterval This field specifies the time length for which the Data Bit Assistance is requested. The GNSS-DataBitAssistance shall be relative to the time interval (gnss-TOD-Req, gnss-TOD-Req + dataBitInterval). The dataBitInterval r, expressed in seconds, is mapped to a binary number K with the following formula: r =0.1 2 K Value K=15 means that the time interval is not specified.

gnss-SignalType This field specifies the GNSS Signal(s) for which the GNSS-DataBitAssistance are requested. A one-value at a bit position means GNSS-DataBitAssistance for the specific signal is requested; a zero-value means not requested.

gnss-DataBitsReq This list contains the SV-IDs for which the GNSS-DataBitAssistance is requested.

#### –	GNSS-AcquisitionAssistanceReq

The IE GNSS-AcquisitionAssistanceReq is used by the target device to request the GNSS-AcquisitionAssistance assistance from the location server.

-- ASN1START

GNSS-AcquisitionAssistanceReq ::=	SEQUENCE {

gnss-SignalID-Req		GNSS-SignalID,

...

}

-- ASN1STOP

GNSS-AcquisitionAssistanceReq field descriptions

gnss-SignalID-Req This field specifies the GNSS signal type for which GNSSAcquisitionAssistance is requested.

#### –	GNSS-AlmanacReq

The IE GNSS-AlmanacReq is used by the target device to request the GNSS-Almanac assistance from the location server.

-- ASN1START

GNSS-AlmanacReq ::= SEQUENCE {

modelID				INTEGER(1..8)	OPTIONAL,

...

}

-- ASN1STOP

GNSS-AlmanacReq field descriptions

modelID This field specifies the Almanac Model ID requested. If this field is absent, the default interpretation as in the table GNSS-ID to modelID relation below applies.

GNSS-ID to modelID relation

| GNSS-ID | modelID |
| --- | --- |
| gps | 2 |
| sbas | 6 |
| qzss | 2 |
| galileo | 1 |
| glonass | 5 |
| bds | 7 |
| navic | 8 |

#### –	GNSS-UTC-ModelReq

The IE GNSS-UTC-ModelReq is used by the target device to request the GNSS-UTC-Model assistance from the location server.

-- ASN1START

GNSS-UTC-ModelReq ::=	SEQUENCE {

modelID				INTEGER(1..8)	OPTIONAL,

...

}

-- ASN1STOP

GNSS-UTC-ModelReq field descriptions

modelID This field specifies the GNSS-UTCModel set requested. If this field is absent, the default interpretation as in the table GNSS-ID to modelID relation below applies.

GNSS-ID to modelID relation

| GNSS-ID | modelID |
| --- | --- |
| gps | 1 |
| sbas | 4 |
| qzss | 1 |
| galileo | 1 |
| glonass | 3 |
| bds | 5 |
| navic | 2 |

#### –	GNSS-AuxiliaryInformationReq

The IE GNSS-AuxiliaryInformationReq is used by the target device to request the GNSS-AuxiliaryInformation assistance from the location server.

-- ASN1START

GNSS-AuxiliaryInformationReq ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	BDS-DifferentialCorrectionsReq

The IE BDS-DifferentialCorrectionsReq is used by the target device to request the BDS-DifferentialCorrections assistance from the location server.

-- ASN1START

BDS-DifferentialCorrectionsReq-r12 ::=	SEQUENCE {

dgnss-SignalsReq			GNSS-SignalIDs,

...

}

-- ASN1STOP

BDS-DifferentialCorrectionsReq field descriptions

dgnss-SignalsReq This field specifies the BDS Signal(s) for which the BDS-DifferentialCorrections are requested. A one-value at a bit position means BDS differential corrections for the specific signal are requested; a zero-value means not requested. The target device shall set a maximum of three bits to value 'one'. This only applies for the B1I/B3I signal.

#### –	BDS-GridModelReq

The IE BDS-GridModelReq is used by the target device to request the BDS-GridModel assistance from the location server.

-- ASN1START

BDS-GridModelReq-r12 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-RTK-ObservationsReq

The IE GNSS-RTK-ObservationsReq is used by the target device to request the GNSS-RTK-Observations assistance from the location server.

-- ASN1START

GNSS-RTK-ObservationsReq-r15::= 	SEQUENCE {

gnss-RTK-SignalsReq-r15				GNSS-SignalIDs,

gnss-RTK-Integer-ms-Req-r15			BOOLEAN,

gnss-RTK-PhaseRangeRateReq-r15		BOOLEAN,

gnss-RTK-CNR-Req-r15				BOOLEAN,

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-ObservationsReq field descriptions

gnss-RTK-SignalsReq This field specifies the GNSS Signal(s) for which the GNSS-RTK-Observations are requested. A one-value at a bit position means RTK observations for the specific signal are requested; a zero-value means not requested.

gnss-RTK-Integer-ms-Req This field specifies whether the integer-ms is requested or not. TRUE means requested.

gnss-RTK-PhaseRangeRateReq This field specifies whether the rough-phase-range-rate and fine-PhaseRangeRate are requested or not. TRUE means requested.

gnss-RTK-CNR-Req This field specifies whether the carrier-to-noise-ratio is requested or not. TRUE means requested.

stationID This field specifies the Station ID for which the GNSS-RTK-Observations are requested.

#### –	GLO-RTK-BiasInformationReq

The IE GLO-RTK-BiasInformationReq is used by the target device to request the GLO-RTK-BiasInformation assistance from the location server.

-- ASN1START

GLO-RTK-BiasInformationReq-r15 ::= 	SEQUENCE {

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

...

}

-- ASN1STOP

GLO-RTK-BiasInformationReq field descriptions

stationID This field specifies the Station ID for which the GLO-RTK-BiasInformation is requested.

#### –	GNSS-RTK-MAC-CorrectionDifferencesReq

The IE GNSS-RTK-MAC-CorrectionDifferencesReq is used by the target device to request the GNSS-RTK-MAC-CorrectionDifferences assistance from the location server.

-- ASN1START

GNSS-RTK-MAC-CorrectionDifferencesReq-r15 ::= 	SEQUENCE {

master-ReferenceStationID-r15		GNSS-ReferenceStationID-r15				OPTIONAL,

aux-ReferenceStationList-r15		AUX-ReferenceStationList-r15			OPTIONAL,

linkCombinations-PrefList-r15		GNSS-Link-CombinationsList-r15			OPTIONAL,

...

}

AUX-ReferenceStationList-r15 ::= SEQUENCE (SIZE (1..32)) OF AUX-ReferenceStationID-Element-r15

AUX-ReferenceStationID-Element-r15 ::= SEQUENCE {

aux-stationID-r15		GNSS-ReferenceStationID-r15,

...

}

-- ASN1STOP

GNSS-RTK-MAC-CorrectionDifferencesReq field descriptions

master-ReferenceStationID, aux-ReferenceStationList These fields specify the Master and Auxiliary Reference Station IDs for which the GNSS-RTK-MAC-CorrectionDifferences are requested.

linkCombinations-PrefList This field specifies the dual-frequency combination of L1 and L2 link/frequencies for which the target device wishes to obtain the GNSS-RTK-MAC-CorrectionDifferences in the order of preference. The first GNSS-Link-Combinations in GNSS-Link-CombinationsList is the most preferred combination, the second GNSS-Link-Combinations in GNSS-Link-CombinationsList is the second most preferred, etc.

#### –	GNSS-RTK-ResidualsReq

The IE GNSS-RTK-ResidualsReq is used by the target device to request the GNSS-RTK-Residuals assistance from the location server.

-- ASN1START

GNSS-RTK-ResidualsReq-r15 ::= 	SEQUENCE {

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

linkCombinations-PrefList-r15		GNSS-Link-CombinationsList-r15	OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-ResidualsReq field descriptions

stationID This field specifies the Station ID for which the GNSS-RTK-Residuals are requested.

linkCombinations-PrefList This field specifies the dual-frequency combination of L1 and L2 link/frequencies for which the target device wishes to obtain the GNSS-RTK-Residuals in the order of preference. The first GNSS-Link-Combinations in GNSS-Link-CombinationsList is the most preferred combination, the second GNSS-Link-Combinations in GNSS-Link-CombinationsList is the second most preferred, etc.

#### –	GNSS-RTK-FKP-GradientsReq

The IE GNSS-RTK-FKP-GradientsReq is used by the target device to request the GNSS-RTK-FKP-Gradients assistance from the location server.

-- ASN1START

GNSS-RTK-FKP-GradientsReq-r15 ::= 	SEQUENCE {

stationID-r15						GNSS-ReferenceStationID-r15		OPTIONAL,

linkCombinations-PrefList-r15		GNSS-Link-CombinationsList-r15	OPTIONAL,

...

}

-- ASN1STOP

GNSS-RTK-FKP-GradientsReq field descriptions

stationID This field specifies the Station ID for which the GNSS-RTK-FKP-Gradients are requested.

linkCombinations-PrefList This field specifies the dual-frequency combination of L1 and L2 link/frequencies for which the target device wishes to obtain the GNSS-RTK-FKP-Gradients in the order of preference. The first GNSS-Link-Combinations in GNSS-Link-CombinationsList is the most preferred combination, the second GNSS-Link-Combinations in GNSS-Link-CombinationsList is the second most preferred, etc.

#### –	GNSS-SSR-OrbitCorrectionsReq

The IE GNSS-SSR-OrbitCorrectionsReq is used by the target device to request the GNSS-SSR-OrbitCorrections assistance from the location server.

-- ASN1START

GNSS-SSR-OrbitCorrectionsReq-r15 ::= SEQUENCE {

storedNavList-r15				GNSS-NavListInfo-r15				OPTIONAL,

...,

[[

orbit-IntegrityReq-r17	BIT STRING {	orbitRangeErrorCorrelationTimeReq		(0),

orbitRangeRateErrorCorrelationTimeReq	(1)

} (SIZE(1..8))									OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-OrbitCorrectionsReq field descriptions

storedNavList This list provides information to the location server about which NAV data the target device has currently stored for the particular GNSS indicated by GNSS-ID.

orbit-IntegrityReq This field, if present, indicates that the target device requests the IEs ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds. A one-value at the bit position '0' means that the target device requests the field orbitRangeErrorCorrelationTime in IE ORBIT-IntegrityParameters; a one-value at bit position '1' means that the target device requests the field orbitRangeRateErrorCorrelationTime in IE ORBIT-IntegrityParameters.

#### –	GNSS-SSR-ClockCorrectionsReq

The IE GNSS-SSR-ClockCorrectionsReq is used by the target device to request the GNSS-SSR-ClockCorrections assistance from the location server.

-- ASN1START

GNSS-SSR-ClockCorrectionsReq-r15 ::= SEQUENCE {

storedNavList-r15				GNSS-NavListInfo-r15				OPTIONAL,

...,

[[

clock-IntegrityParametersReq-r17	ENUMERATED { true }			OPTIONAL,

ssr-IntegrityClockBounds-r17		ENUMERATED { true }			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-ClockCorrectionsReq field descriptions

storedNavList This list provides information to the location server about which NAV data the target device has currently stored for the particular GNSS indicated by GNSS-ID.

clock-IntegrityParametersReq This field. If present, indicates that te target device requests the CLOCK-IntegrityParameters in IE GNSS-SSR-ClockCorrections.

ssr-IntegrityClockBounds This field. If present, indicates that the target device requests the SSR-IntegrityClockBounds in IE GNSS-SSR-ClockCorrections.

#### –	GNSS-SSR-CodeBiasReq

The IE GNSS-SSR-CodeBiasReq is used by the target device to request the GNSS-SSR-CodeBias assistance from the location server.

-- ASN1START

GNSS-SSR-CodeBiasReq-r15 ::= SEQUENCE {

signal-and-tracking-mode-ID-Map-r15		GNSS-SignalIDs,

storedNavList-r15						GNSS-NavListInfo-r15				OPTIONAL,

...,

[[

ssr-IntegrityCodeBiasBoundsReq-r17		ENUMERATED { requested }			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-CodeBiasReq field descriptions

signal-and-tracking-mode-ID-Map This field specifies the GNSS signal(s) for which the GNSS-SSR-CodeBias is requested.

storedNavList This list provides information to the location server about which NAV data the target device has currently stored for the particular GNSS indicated by GNSS-ID.

ssr-IntegrityCodeBiasBoundsReq This field, if present, indicates that the SSR-IntegrityCodeBiasBounds are requested.

#### –	GNSS-SSR-URA-Req

The IE GNSS-SSR-URA-Req is used by the target device to request the GNSS-SSR-URA assistance from the location server.

-- ASN1START

GNSS-SSR-URA-Req-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-SSR-PhaseBiasReq

The IE GNSS-SSR-PhaseBiasReq is used by the target device to request the GNSS-SSR-PhaseBias assistance from the location server.

-- ASN1START

GNSS-SSR-PhaseBiasReq-r16 ::= SEQUENCE {

signal-and-tracking-mode-ID-Map-r16		GNSS-SignalIDs,

storedNavList-r16						GNSS-NavListInfo-r15				OPTIONAL,

...,

[[

ssr-IntegrityPhaseBiasBoundsReq-r17		ENUMERATED { requested }			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-PhaseBiasReq field descriptions

signal-and-tracking-mode-ID-Map This field specifies the GNSS signal(s) for which the GNSS-SSR-PhaseBias is requested.

storedNavList This list provides information to the location server about which NAV data the target device has currently stored for the particular GNSS indicated by GNSS-ID.

ssr-IntegrityPhaseBiasBoundsReq This field, if present, indicates that the SSR-IntegrityPhaseBiasBounds are requested.

#### –	GNSS-SSR-STEC-CorrectionReq

The IE GNSS-SSR-STEC-CorrectionReq is used by the target device to request the GNSS-SSR-STEC-Correction assistance from the location server.

-- ASN1START

GNSS-SSR-STEC-CorrectionReq-r16 ::= SEQUENCE {

...,

[[

stec-IntegrityReq-r17	BIT STRING {	ionoRangeErrorCorrelationTimeReq		(0),

ionoRangeRateErrorCorrelationTimeReq	(1)

} (SIZE(1..8))									OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-STEC-CorrectionReq field descriptions

stec-IntegrityReq This field, if present, indicates that the target device requests the IEs STEC-IntegrityParameters and STEC-IntegrityErrorBounds. A one-value at the bit position '0' means that the target device requests the field ionoRangeErrorCorrelationTime in IE STEC-IntegrityParameters; a one-value at bit position '1' means that the target device requests the field ionoRangeRateErrorCorrelationTime in IE STEC-IntegrityParameters.

#### –	GNSS-SSR-GriddedCorrectionReq

The IE GNSS-SSR-GriddedCorrectionReq is used by the target device to request the GNSS-SSR-GriddedCorrection assistance from the location server.

-- ASN1START

GNSS-SSR-GriddedCorrectionReq-r16 ::= SEQUENCE {

...,

[[

griddedCorrectionIntegrityReq-r17		ENUMERATED { requested }					OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-GriddedCorrectionReq field descriptions

griddedCorrectionIntegrityReq This field, if present, indicates that the target device requests the IEs SSR-GriddedCorrectionIntegrityParameters and TropoDelayIntegrityErrorBounds.

#### –	NavIC-DifferentialCorrectionsReq

The IE NavIC-DifferentialCorrectionsReq is used by the target device to request the NavIC-DifferentialCorrections assistance from the location server.

-- ASN1START

NavIC-DifferentialCorrectionsReq-r16 ::=	SEQUENCE {

dgnss-SignalsReq-r16			GNSS-SignalIDs,

...

}

-- ASN1STOP

NavIC-DifferentialCorrectionsReq field descriptions

dgnss-SignalsReq This field specifies the NavIC Signal(s) for which the NavIC-DifferentialCorrections are requested. A one-value at a bit position means the NavIC differential corrections for the specific signal are requested; a zero-value means not requested. The target device shall set a maximum of three bits to value 'one'.

#### –	NavIC-GridModelReq

The IE NavIC-GridModelReq is used by the target device to request the NavIC-GridModel assistance from the location server.

-- ASN1START

NavIC-GridModelReq-r16 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.2.5	GNSS Location Information

#### –	A-GNSS-ProvideLocationInformation

The IE A-GNSS-ProvideLocationInformation is used by the target device to provide location measurements (e.g., pseudo-ranges, location estimate, velocity) to the location server, together with time information. It may also be used to provide GNSS positioning specific error reason.

-- ASN1START

A-GNSS-ProvideLocationInformation ::= SEQUENCE {

gnss-SignalMeasurementInformation	GNSS-SignalMeasurementInformation		OPTIONAL,

gnss-LocationInformation			GNSS-LocationInformation				OPTIONAL,

gnss-Error							A-GNSS-Error							OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.2.6	GNSS Location Information Elements

#### –	GNSS-SignalMeasurementInformation

The IE GNSS-SignalMeasurementInformation is used by the target device to provide GNSS signal measurement information to the location server and GNSS-network time association if requested by the location server. This information includes the measurements of code phase, Doppler, C/No and optionally accumulated carrier phase, also called accumulated deltarange (ADR), which enable the UE-assisted GNSS method where position is computed in the location server. Figure 6.5.2.6-1 illustrates the relation between some of the fields.

-- ASN1START

GNSS-SignalMeasurementInformation ::= SEQUENCE {

measurementReferenceTime		MeasurementReferenceTime,

gnss-MeasurementList			GNSS-MeasurementList,

...

}

-- ASN1STOP

GNSS-SignalMeasurementInformation field descriptions

measurementReferenceTime This field specifies the GNSS system time for which the information provided in gnss-MeasurementList is valid. It may also include network time, if requested by the location server and supported by the target device.

gnss-MeasurementList This field provides GNSS signal measurement information for up to 16 GNSSs.

#### –	MeasurementReferenceTime

The IE MeasurementReferenceTime is used to specify the time when the measurements provided in A-GNSS-ProvideLocationInformation are valid. It may also include GNSS-network time association, in which case reported measurements shall be valid for the cellular frame boundary defined in the network time association.

-- ASN1START

MeasurementReferenceTime ::= SEQUENCE {

gnss-TOD-msec		INTEGER (0..3599999),

gnss-TOD-frac		INTEGER (0..3999)			OPTIONAL,

gnss-TOD-unc		INTEGER (0..127)			OPTIONAL,

gnss-TimeID			GNSS-ID,

networkTime			CHOICE {

eUTRA	SEQUENCE {

physCellId			INTEGER (0..503),

cellGlobalId		CellGlobalIdEUTRA-AndUTRA		OPTIONAL,

systemFrameNumber	BIT STRING (SIZE (10)),

...

},

uTRA	SEQUENCE {

mode					CHOICE {

fdd			SEQUENCE {

primary-CPICH-Info	INTEGER (0..511),

...

},

tdd			SEQUENCE {

cellParameters		INTEGER (0..127),

...

}

},

cellGlobalId			CellGlobalIdEUTRA-AndUTRA		OPTIONAL,

referenceSystemFrameNumber

INTEGER (0..4095),

...

},

gSM		SEQUENCE {

bcchCarrier			INTEGER (0..1023),

bsic				INTEGER (0..63),

cellGlobalId		CellGlobalIdGERAN					OPTIONAL,

referenceFrame		SEQUENCE {

referenceFN		INTEGER (0..65535),

referenceFNMSB		INTEGER (0..63)		OPTIONAL,

...

},

deltaGNSS-TOD		INTEGER (0 .. 127)		OPTIONAL,

...

},

...,

nbIoT-r14

SEQUENCE {

nbPhysCellId-r14	INTEGER (0..503),

nbCellGlobalId-r14	ECGI						OPTIONAL,

sfn-r14				BIT STRING (SIZE (10)),

hyperSFN-r14		BIT STRING (SIZE (10))		OPTIONAL,

...

},

nr-r15	SEQUENCE {

nrPhysCellId-r15	INTEGER (0..1007),

nrCellGlobalID-r15	NCGI-r15					OPTIONAL,

nr-sfn-r15			BIT STRING (SIZE (10)),

...

}

}		OPTIONAL,

...

}

-- ASN1STOP

MeasurementReferenceTime field descriptions

gnss-TOD-msec This field specifies the GNSS TOD for which the measurements and/or location estimate are valid. The 22 bits of GNSS TOD are the least significant bits. The most significant bits shall be derived by the location server to unambiguously derive the GNSS TOD. The value for GNSS TOD is derived from the GNSS specific system time indicated in gnss-TimeID rounded down to the nearest millisecond unit. Scale factor 1 millisecond.

gnss-TOD-frac This field specifies the fractional part of the GNSS TOD in 250 ns resolution. The total GNSS TOD is given by gnss-TOD-msec + gnss-TOD-frac. Scale factor 250 nanoseconds.

gnss-TOD-unc This field provides the accuracy of the relation GNSS-network time when GNSS-network time association is provided. When GNSS-network time association is not provided, this element can be included to provide the accuracy of the reported gnss-TOD-msec. If GNSS TOD is the given GNSS time, then the true GNSS time, corresponding to the provided network time if applicable, as observed at the target device location, lies in the interval [GNSS TOD – gnss-TOD-unc, GNSS TOD + gnss-TOD-unc]. The uncertainty r, expressed in microseconds, is mapped to a number K, with the following formula: r = C*(((1+x)K)-1) with C = 0.5 and x = 0.14. To encode any higher value of uncertainty than that corresponding in the above formula to K=127, the same value, K=127, shall also be used. The uncertainty is then coded on 7 bits, as the binary encoding of K. Examples of gnss-TOD-unc value are as in the table Value of K to Value of uncertainty relation below. This field shall be included if the target device provides GNSS-network time relationship.

gnss-TimeID This field specifies the GNSS system time for which the gnss-TOD-msec (and gnss-TOD-frac if applicable) is provided.

networkTime These fields specify the network time event which the GNSS TOD time stamps. This field shall be included if the target device provides GNSS-network time relationship.

physCellId This field identifies the reference cell (E-UTRA), as defined in TS 36.331 [12], that is used for the GNSS-network time relation.

cellGlobalId This field specifies the globally unique cell identifier (Evolved Cell Global Identifier (ECGI) in E-UTRA, global UTRAN Cell Identifier in UTRA, or Cell Global Identification (CGI) in GERAN) of the reference cell, as defined in TS 36.331 [12] for E-UTRA and in TS 25.331 [13] for UTRA, for which the GNSS network time relation is provided.

systemFrameNumber This field specifies the system frame number in E-UTRA which the GNSS time time stamps, as defined in TS 36.331 [12].

mode This field identifies the reference cell for the GNSS-network time relation, as defined in TS 25.331 [13].

referenceSystemFrameNumber This field specifies the system frame number in UTRA, as defined in TS 25.331 [13], which is used for time stamping.

bcchCarrier, bsic This field identifies the reference cell for the GNSS-network time relation in GERAN, as defined in TS 44.031 [14].

referenceFN, referenceFNMSB These fields specify the frame number in GERAN which the GNSS time time stamps, as defined in TS 44.031 [14]. The time of the reference frame boundary is as observed by the target device, i.e. without Timing Advance compensation. The referenceFNMSB field indicates the most significant bits of the frame number of the reference BTS corresponding to the GNSS-MeasurementList. Starting from the complete GSM frame number denoted FN, the target device calculates Reference FN MSB as Reference FN MSB = floor(FN/42432) The complete GSM frame number FN can then be reconstructed in the location server by combining the fields referenceFN with referenceFNMSB in the following way FN = referenceFNMSB *42432 + referenceFN

deltaGNSS-TOD This field specifies the difference in milliseconds between gnss-TOD-msec reported and the milli-second part of the SV time tsv_1 of the first SV in the list reported from the target device, as defined in TS 44.031 [14]. The deltaGNSS-TOD is defined as deltaGNSS-TOD = gnss-TOD-msec - fix(tsv_1) where fix() denotes rounding to the nearest integer towards zero.

nbPhysCellId This field identifies the reference cell, as defined in TS 36.331 [12] that is used for the GNSS-network time relation.

nbCellGlobalId This field specifies the global cell identifier of the NB-IoT reference cell, as defined in TS 36.331 [12], for which the GNSS network time relation is provided.

sfn This field specifies the system frame number in NB-IoT which the GNSS time time stamps, as defined in TS 36.331 [12].

hyperSFN This field specifies the hyper-SFN in NB-IoT which the GNSS time time stamps, as defined in TS 36.331 [12].

nrPhysCellId This field identifies the reference cell (NR), as defined in TS 38.331 [35], that is used for the GNSS-network time relation.

nrCellGlobalID This field specifies the NR Cell Global Identifier (NCGI) of the reference cell, as defined in TS 38.331 [35], for which the GNSS network time relation is provided.

nr-sfn This field specifies the system frame number in NR which the GNSS time time stamps, as defined in TS 38.331 [35],

Value of K to Value of uncertainty relation

| Value of K | Value of uncertainty |
| --- | --- |
| 0 | 0 microseconds |
| 1 | 0.07 microseconds |
| 2 | 0.1498 microseconds |
| - | - |
| 50 | 349.62 microseconds |
| - | - |
| 127 | ≥ 8430000 microseconds |

#### –	GNSS-MeasurementList

The IE GNSS-MeasurementList is used by the target device to provide measurements of code phase, Doppler, C/No and optionally accumulated carrier phase, also called accumulated deltarange (ADR).

-- ASN1START

GNSS-MeasurementList ::= SEQUENCE (SIZE(1..16)) OF GNSS-MeasurementForOneGNSS

GNSS-MeasurementForOneGNSS ::= SEQUENCE {

gnss-ID					GNSS-ID,

gnss-SgnMeasList		GNSS-SgnMeasList,

...

}

GNSS-SgnMeasList ::= SEQUENCE (SIZE(1..8)) OF GNSS-SgnMeasElement

GNSS-SgnMeasElement ::= SEQUENCE {

gnss-SignalID			GNSS-SignalID,

gnss-CodePhaseAmbiguity	INTEGER (0..127)		OPTIONAL,

gnss-SatMeasList		GNSS-SatMeasList,

...

}

GNSS-SatMeasList ::= SEQUENCE (SIZE(1..64)) OF GNSS-SatMeasElement

GNSS-SatMeasElement ::= SEQUENCE {

svID				SV-ID,

cNo					INTEGER (0..63),

mpathDet			ENUMERATED {notMeasured (0), low (1), medium (2), high (3), ...},

carrierQualityInd	INTEGER (0..3)				OPTIONAL,

codePhase			INTEGER (0..2097151),

integerCodePhase	INTEGER (0..127)			OPTIONAL,

codePhaseRMSError	INTEGER (0..63),

doppler				INTEGER (-32768..32767)	OPTIONAL,

adr					INTEGER (0..33554431)		OPTIONAL,

...,

[[

adrMSB-r15			INTEGER (0..15)						OPTIONAL,

adrSign-r15			ENUMERATED {positive, negative}		OPTIONAL,

adrRMSerror-r15		INTEGER (0..127)					OPTIONAL,

delta-codePhase-r15	INTEGER (0..7)						OPTIONAL

]]

}

-- ASN1STOP

GNSS-MeasurementList field descriptions

gnss-ID This field identifies the GNSS constellation on which the GNSS signal measurements were measured. Measurement information for up to 16 GNSSs can be included.

gnss-SgnMeasList This list provides GNSS signal measurement information for up to 8 GNSS signal types per GNSS.

gnss-SignalID This field identifies the signal on which GNSS signal measurement parameters were measured.

gnss-CodePhaseAmbiguity This field provides the ambiguity of the code phase measurement. It is given in units of milli-seconds in the range between between 0 and 127 milli-seconds. The total code phase for a satellite k (Satk) is given modulo this gnss-CodePhaseAmbiguity and is reconstructed with: Code_Phase_Tot(Satk) = codePhase(Satk) + integerCodePhase(Satk) + n * gnss-CodePhaseAmbiguity, n= 0,1,2,... If there is no code phase ambiguity, the gnss-CodePhaseAmbiguity shall be set to 0. The field is optional. If gnss-CodePhaseAmbiguity is absent, the default value is 1 milli-second.

gnss-SatMeasList This list provides GNSS signal measurement information for up to 64 GNSS satellites.

svID This field identifies the satellite on which the GNSS signal measurements were measured.

cNo This field provides an estimate of the carrier-to-noise ratio of the received signal from the particular satellite. The target device shall set this field to the value of the satellite C/N0, as referenced to the antenna connector, in units of 1 dB-Hz, in the range from 0 to 63 dB-Hz. Scale factor 1 dB-Hz.

mpathDet This field contains the multipath indicator value, defined in the table Value of mpathDet to Multipath Indication relation below.

carrierQualityInd If the fields adrMSB, adrSign, adrRMSerror, and delta-codePhase are not present: This field indicates the quality of a carrier phase measurement. The LSB indicates the data polarity, that is, if the data from a specific satellite is received inverted, this is indicated by setting the LSB value to '1'. In the case the data is not inverted, the LSB is set to '0'. The MSB indicates if accumulation of the carrier phase has been continuous, that is, without cycle slips since the previous measurement report. If the carrier phase accumulation has been continuous, the MSB value is set to '1X'. Otherwise, the MSB is set to '0X'. This field is optional but shall be included if the adr field is included. See table Bit to Polarity Indication relation below. If any of the fields adrMSB, adrSign, adrRMSerror, or delta-codePhase are present: This field indicates the quality of a carrier phase measurement. The LSB indicates the half-cycle ambiguity, that is, if there are no half-cycle ambiguities present in the ADR measurement report the LSB is set to '0'. In the case there are half-cycle ambiguities present in the ADR measurement report the LSB is set to '1'. When reporting ADR with unresolved polarity encoding the target device shall set this bit to 1. The MSB indicates if accumulation of the carrier phase has been continuous, that is, without cycle slips since the previous measurement report. If the carrier phase accumulation has been continuous (no cycle slips), the MSB value is set to '1X'. Otherwise, the MSB is set to '0X'. If polarity resolution forced the ADR measurement to be corrected by half-a-cycle, then the MSB must be set to '0', indicating that despite continuous tracking the reported ADR experienced non-continuity. See table Bit to Ambiguity Indication relation below. The target device shall include this field if the adr field is included.

codePhase This field contains the whole and fractional value of the code-phase measurement made by the target device for the particular satellite signal at the time of measurement in the units of ms. GNSS specific code phase measurements (e.g. chips) are converted into unit of ms by dividing the measurements by the nominal values of the measured signal chipping rate. Scale factor 2-21 milli-seconds, in the range from 0 to (1-2-21) milli-seconds.

integerCodePhase This field indicates the integer milli-second part of the code phase that is expressed modulo the gnss-CodePhaseAmbiguity. The value of the ambiguity is given in the gnss-CodePhaseAmbiguity field. The integerCodePhase is optional. If integerCodePhase is absent, the default value is 0 milli-second. Scale factor 1 milli-second, in the range from 0 to 127 milli-seconds.

codePhaseRMSError This field contains the pseudorange RMS error value. This parameter is specified according to a floating-point representation shown in the table below.

doppler This field contains the Doppler measured by the target device for the particular satellite signal. This information can be used to compute the 3-D velocity of the target device. Doppler measurements are converted into unit of m/s by multiplying the Doppler measurement in Hz by the nominal wavelength of the measured signal. Scale factor 0.04 metre/second. This field is optional, but shall be included, if the velocityRequest in CommonIEsRequestLocationInformation is set to TRUE.

adr This field contains the absolute value of the ADR measurement measured by the target device for the particular satellite signal. This information can be used to compute the 3-D velocity or high-accuracy position of the target device. ADR measurements are converted into units of metre by multiplying the ADR measurement by the nominal wavelength of the measured signal. Scale factor 2-10 metres, in the range from 0 to 32767.5 metres. This field is optional, but shall be included, if the adrMeasReq in GNSS-PositioningInstructions is set to TRUE and if ADR measurements are supported by the target device (i.e., adr-Support is set to TRUE in A-GNSS-ProvideCapabilities).

adrMSB This field contains the 4-MSBs of the ADR measurement in the case the ADR measurement is outside the range of the field adr alone. Scale factor 32768 metres. If present, the full ADR measurement is constructed as adrMSB × 32768 + adr × 2-10 metres, representing measurements in the range from 0 to 524287.9990234375 metres. This field is optional, but shall be included, if the capability adrEnhancementsSupport is set to TRUE and the ADR measurement is outside the range of the adr field.

adrSign This field indicates the sign of the ADR measurement.

adrRMSerror This field contains the ADR root mean squared error value. Scale factor 2-10 metres.

delta-codePhase This field specifies the higher resolution of the codePhase measurement. Scale factor 2-24 milli-seconds. The full code phase measurement is constructed as codePhase × 2-21 + delta-codePhase × 2-24 milli-seconds, in the range from 0 to (1-2-24) milli-seconds.

Value of mpathDet to Multipath Indication relation

| Value of mpathDet | Multipath Indication |
| --- | --- |
| 00 | Not measured |
| 01 | Low, MP error < 5m |
| 10 | Medium, 5m < MP error < 43m |
| 11 | High, MP error > 43m |

Bit to Polarity Indication relation

| Value | Polarity Indication |
| --- | --- |
| 0 | Data Direct, carrier phase not continuous |
| 1 | Data Inverted, carrier phase not continuous |
| 2 | Data Direct, carrier phase continuous |
| 3 | Data Inverted, carrier phase continuous |

Bit to Ambiguity Indication relation

| Value | Value MSB, LSB | Polarity Indication |
| --- | --- | --- |
| 0 | 00 | carrier phase not continuous, no half-cycle ambiguity |
| 1 | 01 | carrier phase not continuous, half-cycle ambiguity |
| 2 | 10 | carrier phase continuous, no half-cycle ambiguity |
| 3 | 11 | carrier phase continuous, half-cycle ambiguity |

floating-point representation

| Index | Mantissa | Exponent | Floating-Point value, xi | Pseudorange value, P |
| --- | --- | --- | --- | --- |
| 0 | 000 | 000 | 0.5 | P < 0.5 |
| 1 | 001 | 000 | 0.5625 | 0.5 <= P < 0.5625 |
| I | x | y | 0.5 * (1 + x/8) * 2y | xi-1 <= P < xi |
| 62 | 110 | 111 | 112 | 104 <= P < 112 |
| 63 | 111 | 111 | -- | 112 <= P |

Figure 6.5.2.6-1: Exemplary calculation of some GNSS Signal Measurement Information fields.

#### –	GNSS-LocationInformation

The IE GNSS-LocationInformation is included by the target device when location and optionally velocity information derived using GNSS or hybrid GNSS and other measurements is provided to the location server.

-- ASN1START

GNSS-LocationInformation ::= SEQUENCE {

measurementReferenceTime		MeasurementReferenceTime,

agnss-List						GNSS-ID-Bitmap,

...

}

-- ASN1STOP

GNSS-LocationInformation field descriptions

measurementReferenceTime This field specifies the GNSS system time for which the location estimate and optionally velocity are valid. It may also include GNSS-network time relationship, if requested by the location server and supported by the target device.

agnss-List This field provides a list of satellite systems used by the target device to calculate the location estimate and velocity estimate, if included. This is represented by a bit string in GNSS-ID-Bitmap, with a one-value at the bit position means the particular method has been used; a zero-value means not used.

#### 6.5.2.7	GNSS Location Information Request

#### –	A-GNSS-RequestLocationInformation

The IE A-GNSS-RequestLocationInformation is used by the location server to request location information from the target device using GNSS.

-- ASN1START

A-GNSS-RequestLocationInformation ::= SEQUENCE {

gnss-PositioningInstructions		GNSS-PositioningInstructions,

...

}

-- ASN1STOP

#### 6.5.2.8	GNSS Location Information Request Elements

#### –	GNSS-PositioningInstructions

The IE GNSS-PositioningInstructions is used to provide GNSS measurement instructions.

-- ASN1START

GNSS-PositioningInstructions ::= SEQUENCE {

gnss-Methods				GNSS-ID-Bitmap,

fineTimeAssistanceMeasReq	BOOLEAN,

adrMeasReq					BOOLEAN,

multiFreqMeasReq			BOOLEAN,

assistanceAvailability		BOOLEAN,

...,

[[

ha-GNSS-Req-r15			ENUMERATED { true }		OPTIONAL	-- Cond UEB

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need OP, if the locationInformationType is set to locationEstimateRequired, locationEstimatePreferred, or locationMeasurementsPreferred; oltherwise it is not present. |

GNSS-PositioningInstructions field descriptions

gnssMethods This field indicates the satellite systems allowed by the location server. This is represented by a bit string in GNSS-ID-Bitmap, with a one-value at the bit position means the particular GNSS is allowed; a zero-value means not allowed. The target device shall not request assistance data or report or obtain measurements for systems that are not indicated in this bit map. At least one of the bits in this bit map shall be set to value one.

fineTimeAssistanceMeasReq This field indicates whether the target device is requested to report GNSS-network time association. TRUE means requested.

adrMeasReq This field indicates whether the target device is requested to include ADR measurements in GNSS-MeasurementList IE or not. TRUE means requested.

multiFreqMeasReq This field indicates whether the target device is requested to report measurements on multiple supported GNSS signal types in GNSS-MeasurementList IE or not. TRUE means requested.

assistanceAvailability This field indicates whether the target device may request additional GNSS assistance data from the server. TRUE means allowed and FALSE means not allowed.

ha-GNSS-Req This field, if present, indicates that any location estimate provided by the target device should be obtained using high accuracy RTK/PPP methods.

#### 6.5.2.9	GNSS Capability Information

#### –	A-GNSS-ProvideCapabilities

The IE A-GNSS-Provide-Capabilities is used by the target device to indicate its capability to support A-GNSS and to provide its A-GNSS location capabilities (e.g., GNSSs and assistance data supported) to the location server.

-- ASN1START

A-GNSS-ProvideCapabilities ::= SEQUENCE {

gnss-SupportList			GNSS-SupportList				OPTIONAL,

assistanceDataSupportList	AssistanceDataSupportList		OPTIONAL,

locationCoordinateTypes		LocationCoordinateTypes			OPTIONAL,

velocityTypes				VelocityTypes					OPTIONAL,

...,

[[ periodicalReportingNotSupported-r14

PositioningModes				OPTIONAL,

idleStateForMeasurements-r14

ENUMERATED { required }			OPTIONAL

]],

[[ periodicAssistanceData-r15

BIT STRING { solicited 	 (0),

unsolicited (1)	} (SIZE (1..8))		OPTIONAL

]],

[[ scheduledLocationRequest-r17	SEQUENCE {

utcTime-r17			PositioningModes		OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}										OPTIONAL,

e-utraTime-r17		PositioningModes		OPTIONAL,

nrTime-r17			PositioningModes		OPTIONAL,

relativeTime-r17	PositioningModes		OPTIONAL,

...

}								OPTIONAL

]]

}

GNSS-SupportList ::= SEQUENCE (SIZE(1..16)) OF GNSS-SupportElement

GNSS-SupportElement ::= SEQUENCE {

gnss-ID							GNSS-ID,

sbas-IDs						SBAS-IDs					OPTIONAL,	-- Cond GNSS-ID-SBAS

agnss-Modes						PositioningModes,

gnss-Signals					GNSS-SignalIDs,

fta-MeasSupport					SEQUENCE {

cellTime	AccessTypes,

mode		PositioningModes,

...

}							OPTIONAL,	-- Cond fta

adr-Support						BOOLEAN,

velocityMeasurementSupport		BOOLEAN,

...,

[[

adrEnhancementsSupport-r15	ENUMERATED { true }			OPTIONAL,

ha-gnss-Modes-r15			PositioningModes			OPTIONAL

]]

}

AssistanceDataSupportList ::= SEQUENCE {

gnss-CommonAssistanceDataSupport	GNSS-CommonAssistanceDataSupport,

gnss-GenericAssistanceDataSupport	GNSS-GenericAssistanceDataSupport,

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| fta | The field is mandatory present if the target device supports the reporting of fine time assistance measurements; otherwise it is not present. |

A-GNSS-ProvideCapabilities field descriptions

gnss-SupportList This field specifies the list of GNSS supported by the target device and the target device capabilities associated with each of the supported GNSS. This field shall be present if the gnss-SupportListReq in the A-GNSS -RequestCapabilities IE is set to TRUE and if the target device supports the A-GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports the assisted GNSS positioning method.

gnss-ID This field specifies the GNSS supported by the target device for which the capabilities in GNSS-SupportElement are provided.

sbas-IDs This field specifies the SBAS(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular SBAS is supported; a zero-value means not supported.

agnss-Modes This field specifies the GNSS mode(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS mode is supported; a zero-value means not supported.

gnss-Signals This field specifies the GNSS signal(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS signal type is supported; a zero-value means not supported.

fta-MeasSupport This field specifies that the target device is capable of performing fine time assistance measurements (i.e., GNSS-cellular time association reporting). The cellTime field specifies for which cellular network(s) this capability is supported. This is represented by a bit string, with a one-value at the bit position means FTA measurements for the specific cellular network time is supported; a zero-value means not supported. The mode field specifies for which GNSS mode(s) FTA measurements are supported by the target device. This is represented by a bit string, with a one-value at the bit position means FTA measurements for the GNSS mode is supported; a zero-value means not supported.

adr-Support This field specifies whether the target device supports ADR measurement reporting. TRUE means supported.

velocityMeasurementSupport This field specifies whether the target device supports measurement reporting related to velocity. TRUE means supported.

assistanceDataSupportList This list defines the assistance data and assistance data choices supported by the target device. This field shall be present if the assistanceDataSupportListReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports GNSS assistance data. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports any GNSS assistance data.

locationCoordinateTypes This parameter identifies the geographical location coordinate types that a target device supports for GNSS. TRUE indicates that a location coordinate type is supported and FALSE that it is not. This field shall be present if the locationVelocityTypesReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports UE-based or standalone GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports UE-based or standalone GNSS positioning method.

velocityTypes This parameter identifies the velocity types that a target device supports for GNSS. TRUE indicates that a velocity type is supported and FALSE that it is not. FALSE for all velocity types indicates that velocity reporting is not supported. This field shall be present if the locationVelocityTypesReq in the A-GNSS-RequestCapabilities IE is set to TRUE and if the target device supports UE-based or standalone GNSS positioning method. If the IE A-GNSS-Provide-Capabilities is provided unsolicited, this field shall be included if the target device supports UE-based or standalone GNSS positioning method.

periodicalReportingNotSupported This field, if present, specifies the positioning modes for which the target device does not support periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is not supported; a zero-value means supported. If this field is absent, the location server may assume that the target device supports periodicalReporting in CommonIEsRequestLocationInformation for each supported positioning mode.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform GNSS measurements.

periodicAssistanceData This field identifies the periodic assistance data delivery procedures supported by the target device. This is represented by a bit string, with a one value at the bit position means the periodic assistance data delivery procedure is supported; a zero value means not supported. Bit 0 (solicited) represents the procedure according to clause 5.2.1a; bit (1) (unsolicited) represents the procedure according to clause 5.2.2a.

adrEnhancementsSupport This field, if present, indicates that the target device supports the fields adrMSB, adrSign, adrRMSerror, and delta-codePhase in IE GNSS-MeasurementList. This field may only be present if adr-Support is set to TRUE, and shall be absent if adr-Support is set to FALSE.

ha-gnss-Modes This field specifies the High-Accuracy GNSS mode(s) supported by the target device for the GNSS indicated by gnss-ID. This is represented by a bit string, with a one-value at the bit position means the particular GNSS mode is supported; a zero-value means not supported.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

#### 6.5.2.10	GNSS Capability Information Elements

#### –	GNSS-CommonAssistanceDataSupport

The IE GNSS-CommonAssistanceDataSupport is used by the target device to provide information on supported GNSS common assistance data types to the location server.

-- ASN1START

GNSS-CommonAssistanceDataSupport ::= SEQUENCE {

gnss-ReferenceTimeSupport				GNSS-ReferenceTimeSupport

OPTIONAL, -- Cond RefTimeSup

gnss-ReferenceLocationSupport			GNSS-ReferenceLocationSupport

OPTIONAL, -- Cond RefLocSup

gnss-IonosphericModelSupport			GNSS-IonosphericModelSupport

OPTIONAL, -- Cond IonoModSup

gnss-EarthOrientationParametersSupport	GNSS-EarthOrientationParametersSupport

OPTIONAL, -- Cond EOPSup

...,

[[

gnss-RTK-ReferenceStationInfoSupport-r15

GNSS-RTK-ReferenceStationInfoSupport-r15

OPTIONAL, -- Cond ARPSup

gnss-RTK-AuxiliaryStationDataSupport-r15

GNSS-RTK-AuxiliaryStationDataSupport-r15

OPTIONAL -- Cond AuxARPSup

]],

[[

gnss-Integrity-ServiceParametersSupport-r17

GNSS-Integrity-ServiceParametersSupport-r17

OPTIONAL, 	-- Cond IntServiceSup

gnss-Integrity-ServiceAlertSupport-r17

GNSS-Integrity-ServiceAlertSupport-r17

OPTIONAL 	-- Cond IntAlertSup

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| RefTimeSup | The field is mandatory present if the target device supports GNSS-ReferenceTime; otherwise it is not present. |
| RefLocSup | This field is mandatory present if the target device supports GNSS-ReferenceLocation; otherwise it is not present. |
| IonoModSup | This field is mandatory present if the target device supports GNSS-IonosphericModel; otherwise it is not present. |
| EOPSup | This field is mandatory present if the target device supports GNSS-EarthOrientationParameters; otherwise it is not present. |
| ARPSup | This field is mandatory present if the target device supports GNSS-RTK-ReferenceStationInfo; otherwise it is not present. |
| AuxARPSup | This field is mandatory present if the target device supports GNSS-RTK-AuxiliaryStationData; otherwise it is not present. |
| IntServiceSup | This field is mandatory present if the target device supports GNSS-Integrity-ServiceParameters; otherwise it is not present. |
| IntAlertSup | This field is mandatory present if the target device supports GNSS-Integrity-ServiceAlert; otherwise it is not present. |

#### –	GNSS-ReferenceTimeSupport

-- ASN1START

GNSS-ReferenceTimeSupport ::=	SEQUENCE {

gnss-SystemTime		GNSS-ID-Bitmap,

fta-Support			AccessTypes										OPTIONAL, -- Cond fta

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| fta | The field is mandatory present if the target device supports fine time assistance in GNSSReferenceTime IE; otherwise it is not present. |

GNSS-ReferenceTimeSupport field descriptions

gnss-SystemTime This field specifies the GNSS system time(s) supported by the target device. This is represented by a bit string in GNSS-ID-Bitmap, with a one-value at the bit position means the particular GNSS system time is supported; a zero-value means not supported.

fta-Support This field specifies that the target device supports fine time assistance (i.e., GNSS-cellular time association) in GNSS-ReferenceTime IE. This is represented by a bit string in AccessTypes, with a one-value at the bit position means FTA for the specific cellular network time is supported; a zero-value means not supported.

#### –	GNSS-ReferenceLocationSupport

-- ASN1START

GNSS-ReferenceLocationSupport ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-IonosphericModelSupport

-- ASN1START

GNSS-IonosphericModelSupport ::=	SEQUENCE {

ionoModel		BIT STRING {	klobuchar	(0),

neQuick		(1),

klobuchar2-r16	(2) } (SIZE (1..8)),

...

}

-- ASN1STOP

GNSS-IonosphericModelSupport field descriptions

ionoModel This field specifies the ionospheric model(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular ionospheric model is supported; a zero-value means not supported.

#### –	GNSS-EarthOrientationParametersSupport

-- ASN1START

GNSS-EarthOrientationParametersSupport ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-RTK-ReferenceStationInfoSupport

-- ASN1START

GNSS-RTK-ReferenceStationInfoSupport-r15 ::= 	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-RTK-AuxiliaryStationDataSupport

-- ASN1START

GNSS-RTK-AuxiliaryStationDataSupport-r15 ::= 	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-Integrity-ServiceParametersSupport

-- ASN1START

GNSS-Integrity-ServiceParametersSupport-r17 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-Integrity-ServiceAlertSupport

-- ASN1START

GNSS-Integrity-ServiceAlertSupport-r17 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-GenericAssistanceDataSupport

The IE GNSS-GenericAssistanceDataSupport is used by the target device to provide information on supported GNSS generic assistance data types to the location server for each supported GNSS.

-- ASN1START

GNSS-GenericAssistanceDataSupport ::=

SEQUENCE (SIZE (1..16)) OF GNSS-GenericAssistDataSupportElement

GNSS-GenericAssistDataSupportElement ::= SEQUENCE {

gnss-ID								GNSS-ID,

sbas-ID								SBAS-ID						OPTIONAL, -- Cond GNSS-ID-SBAS

gnss-TimeModelsSupport				GNSS-TimeModelListSupport

OPTIONAL, -- Cond TimeModSup

gnss-DifferentialCorrectionsSupport	GNSS-DifferentialCorrectionsSupport

OPTIONAL, -- Cond DGNSS-Sup

gnss-NavigationModelSupport			GNSS-NavigationModelSupport

OPTIONAL, -- Cond NavModSup

gnss-RealTimeIntegritySupport		GNSS-RealTimeIntegritySupport

OPTIONAL, -- Cond RTISup

gnss-DataBitAssistanceSupport		GNSS-DataBitAssistanceSupport

OPTIONAL, -- Cond DataBitsSup

gnss-AcquisitionAssistanceSupport	GNSS-AcquisitionAssistanceSupport

OPTIONAL, -- Cond AcquAssistSup

gnss-AlmanacSupport					GNSS-AlmanacSupport

OPTIONAL, -- Cond AlmanacSup

gnss-UTC-ModelSupport				GNSS-UTC-ModelSupport

OPTIONAL, -- Cond UTCModSup

gnss-AuxiliaryInformationSupport	GNSS-AuxiliaryInformationSupport

OPTIONAL, -- Cond AuxInfoSup

...,

[[

bds-DifferentialCorrectionsSupport-r12

BDS-DifferentialCorrectionsSupport-r12

OPTIONAL, -- Cond DBDS-Sup

bds-GridModelSupport-r12		BDS-GridModelSupport-r12

OPTIONAL	-- Cond BDS-GridModSup

]],

[[

gnss-RTK-ObservationsSupport-r15

GNSS-RTK-ObservationsSupport-r15

OPTIONAL,	-- Cond RTK-OSR-Sup

glo-RTK-BiasInformationSupport-r15

GLO-RTK-BiasInformationSupport-r15

OPTIONAL,	-- Cond GLO-CPB-Sup

gnss-RTK-MAC-CorrectionDifferencesSupport-r15

GNSS-RTK-MAC-CorrectionDifferencesSupport-r15

OPTIONAL,	-- Cond MAC-Sup

gnss-RTK-ResidualsSupport-r15	GNSS-RTK-ResidualsSupport-r15

OPTIONAL,	-- Cond Res-Sup

gnss-RTK-FKP-GradientsSupport-r15

GNSS-RTK-FKP-GradientsSupport-r15

OPTIONAL,	-- Cond FKP-Sup

gnss-SSR-OrbitCorrectionsSupport-r15

GNSS-SSR-OrbitCorrectionsSupport-r15

OPTIONAL, 	-- Cond OC-Sup

gnss-SSR-ClockCorrectionsSupport-r15

GNSS-SSR-ClockCorrectionsSupport-r15

OPTIONAL, 	-- Cond CC-Sup

gnss-SSR-CodeBiasSupport-r15	GNSS-SSR-CodeBiasSupport-r15

OPTIONAL 	-- Cond CB-Sup

]],

[[

gnss-SSR-URA-Support-r16		GNSS-SSR-URA-Support-r16	OPTIONAL,	-- Cond URA-Sup

gnss-SSR-PhaseBiasSupport-r16	GNSS-SSR-PhaseBiasSupport-r16

OPTIONAL,	-- Cond PB-Sup

gnss-SSR-STEC-CorrectionSupport-r16

GNSS-SSR-STEC-CorrectionSupport-r16

OPTIONAL,	-- Cond STEC-Sup

gnss-SSR-GriddedCorrectionSupport-r16

GNSS-SSR-GriddedCorrectionSupport-r16

OPTIONAL,	-- Cond Grid-Sup

navic-DifferentialCorrectionsSupport-r16

NavIC-DifferentialCorrectionsSupport-r16

OPTIONAL,	-- Cond DNavIC-Sup

navic-GridModelSupport-r16		NavIC-GridModelSupport-r16

OPTIONAL	-- Cond NavIC-GridModSup

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| GNSS-ID-SBAS | The field is mandatory present if the GNSS-ID = sbas; otherwise it is not present. |
| TimeModSup | The field is mandatory present if the target device supports GNSS-TimeModelList; otherwise it is not present. |
| DGNSS-Sup | The field is mandatory present if the target device supports GNSS-DifferentialCorrections; otherwise it is not present. |
| NavModSup | The field is mandatory present if the target device supports GNSS-NavigationModel; otherwise it is not present. |
| RTISup | The field is mandatory present if the target device supports GNSS-RealTimeIntegrity; otherwise it is not present. |
| DataBitsSup | The field is mandatory present if the target device supports GNSS-DataBitAssistance; otherwise it is not present. |
| AcquAssistSup | The field is mandatory present if the target device supports GNSS-AcquisitionAssistance; otherwise it is not present. |
| AlmanacSup | The field is mandatory present if the target device supports GNSS-Almanac; otherwise it is not present. |
| UTCModSup | The field is mandatory present if the target device supports GNSS-UTC-Model; otherwise it is not present. |
| AuxInfoSup | The field is mandatory present if the target device supports GNSS-AuxiliaryInformation; otherwise it is not present. |
| DBDS-Sup | The field is mandatory present if the target device supports BDS-DifferentialCorrections; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| BDS-GridModSup | The field is mandatory present if the target device supports BDS-GridModel; otherwise it is not present. This field may only be present if gnss-ID indicates 'bds'. |
| RTK-OSR-Sup | The field is mandatory present if the target device supports GNSS-RTK-Observations; otherwise it is not present. Note, support for GNSS-RTK-Observations implies support for GNSS-RTK-CommonObservationInfo as well. |
| GLO-CPB-Sup | The field is mandatory present if the target device supports GLO-RTK-BiasInformation; otherwise it is not present. This field may only be present if gnss-ID indicates 'glonass'. |
| MAC-Sup | The field is mandatory present if the target device supports GNSS-RTK-MAC-CorrectionDifferences; otherwise it is not present. |
| Res-Sup | The field is mandatory present if the target device supports GNSS-RTK-Residuals; otherwise it is not present. |
| FKP-Sup | The field is mandatory present if the target device supports GNSS-RTK-FKP-Gradients; otherwise it is not present. |
| OC-Sup | The field is mandatory present if the target device supports GNSS-SSR-OrbitCorrections; otherwise it is not present. |
| CC-Sup | The field is mandatory present if the target device supports GNSS-SSR-ClockCorrections; otherwise it is not present. |
| CB-Sup | The field is mandatory present if the target device supports GNSS-SSR-CodeBias; otherwise it is not present. |
| URA-Sup | The field is mandatory present if the target device supports GNSS-SSR-URA; otherwise it is not present. |
| PB-Sup | The field is mandatory present if the target device supports GNSS-SSR-PhaseBias; otherwise it is not present. |
| STEC-Sup | The field is mandatory present if the target device supports GNSS-SSR-STEC-Correction; otherwise it is not present. |
| Grid-Sup | The field is mandatory present if the target device supports GNSS-SSR-GriddedCorrection; otherwise it is not present. Note, support for GNSS-SSR-GriddedCorrection implies support for GNSS-SSR-CorrectionPoints as well. |
| DNavIC-Sup | The field is mandatory present if the target device supports NavIC-DifferentialCorrections; otherwise it is not present. This field may only be present if the gnss-ID indicates 'navic'. |
| NavIC-GridModSup | The field is mandatory present if the target device supports NavIC-GridModel; otherwise it is not present. This field may only be present if the gnss-ID indicates 'navic'. |

#### –	GNSS-TimeModelListSupport

-- ASN1START

GNSS-TimeModelListSupport ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-DifferentialCorrectionSupport

-- ASN1START

GNSS-DifferentialCorrectionsSupport ::=	SEQUENCE {

gnssSignalIDs			GNSS-SignalIDs,

dgnss-ValidityTimeSup	BOOLEAN,

...

}

-- ASN1STOP

GNSS-DifferentialCorrectionsSupport field descriptions

gnssSignalIDs This field specifies the GNSS signal types for which differential corrections are supported by the target device. This is represented by a bit string in GNSS-SignalIDs, with a one-value at the bit position means differential corrections for the particular GNSS signal type is supported; a zero-value means not supported.

dgnss-ValidityTimeSup This field specifies if the target device supports estimation of UDRE based on growth rate and validity time for differential corrections. TRUE means supported.

#### –	GNSS-NavigationModelSupport

-- ASN1START

GNSS-NavigationModelSupport ::= SEQUENCE {

clockModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7-r16	(6),

model-8-r16	(7) } (SIZE (1..8))		OPTIONAL,

orbitModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7-r16	(6),

model-8-r16	(7) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-NavigationModelSupport field descriptions

clockModel This field specifies the gnss-ClockModel choice(s) in GNSS-NavigationModel IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular clock model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-NavigationModel assistance, it shall support clockModel Model-2. If the target device supports SBAS and GNSS-NavigationModel assistance, it shall support clockModel Model-5. If the target device supports QZSS and GNSS-NavigationModel assistance, it shall support clockModel Model-2. If the target device supports Galileo and GNSS-NavigationModel assistance, it shall support clockModel Model-1. If the target device supports GLONASS and GNSS-NavigationModel assistance, it shall support clockModel Model-4. If the target device supports BDS and GNSS-NavigationModel assistance, it shall support clockModel Model-6. If the target device supports NavIC and GNSS-NavigationModel assistance, it shall support clockModel Model-8. If this field is absent, the target device supports the mandatory (native) clockModel choice only as listed above for the GNSS indicated by GNSS-ID.

orbitModel This field specifies the gnss-OrbitModel choice(s) in GNSS-NavigationModel IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular orbit model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-NavigationModel assistance, it shall support orbitModel Model-2. If the target device supports SBAS and GNSS-NavigationModel assistance, it shall support orbitModel Model-5. If the target device supports QZSS and GNSS-NavigationModel assistance, it shall support orbitModel Model-2. If the target device supports Galileo and GNSS-NavigationModel assistance, it shall supportorbitModel Model-1. If the target device supports GLONASS and GNSS-NavigationModel assistance, it shall support orbitModel Model-4. If the target device supports BDS and GNSS-NavigationModel assistance, it shall support orbitModel Model-6. If the target device supports NavIC and GNSS-NavigationModel assistance, it shall support orbitModel Model-8. If this field is absent, the target device supports the mandatory (native) orbitModel choice only as listed above for the GNSS indicated by GNSS-ID.

#### –	GNSS-RealTimeIntegritySupport

-- ASN1START

GNSS-RealTimeIntegritySupport ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-DataBitAssistanceSupport

-- ASN1START

GNSS-DataBitAssistanceSupport ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-AcquisitionAssistanceSupport

-- ASN1START

GNSS-AcquisitionAssistanceSupport ::=	SEQUENCE {

...,

confidenceSupport-r10					ENUMERATED { true }		OPTIONAL,

dopplerUncertaintyExtSupport-r10		ENUMERATED { true }		OPTIONAL

}

-- ASN1STOP

GNSS-AcquisitionAssistanceSupport field descriptions

confidenceSupport If this field is present, the target device supports the confidence field in GNSS-AcquisitionAssistance.

dopplerUncertaintyExtSupport If this field is present, the target device supports the dopplerUncertaintyExt field in GNSS-AcquisitionAssistance.

#### –	GNSS-AlmanacSupport

-- ASN1START

GNSS-AlmanacSupport ::=	SEQUENCE {

almanacModel		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4),

model-6		(5),

model-7		(6) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-AlmanacSupport field descriptions

almanacModel This field specifies the almanacModel choice(s) in GNSS-Almanac IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular almanac model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-Almanac assistance, it shall support Model-2. If the target device supports SBAS and GNSS-Almanac assistance, it shall support Model-6. If the target device supports QZSS and GNSS-Almanac assistance, it shall support Model-2. If the target device supports Galileo and GNSS-Almanac assistance, it shall support Model-1. If the target device supports GLONASS and GNSS-Almanac assistance, it shall support Model-5. If the target device supports BDS and GNSS-Almanac assistance, it shall support Model-7. If this field is absent, the target device supports the mandatory (native) almanacModel choice only as listed above for the GNSS indicated by GNSS-ID.

#### –	GNSS-UTC-ModelSupport

-- ASN1START

GNSS-UTC-ModelSupport ::=	SEQUENCE {

utc-Model		BIT STRING {	model-1		(0),

model-2		(1),

model-3		(2),

model-4		(3),

model-5		(4) } (SIZE (1..8))		OPTIONAL,

...

}

-- ASN1STOP

GNSS-UTC-ModelSupport field descriptions

utc-Model This field specifies the GNSS-UTC-Model choice(s) in GNSS-UTC-Model IE supported by the target device for the GNSS indicated by GNSS-ID. This is represented by a bit string, with a one-value at the bit position means the particular UTC model is supported; a zero-value means not supported. If the target device supports GPS and GNSS-UTC-Model assistance, it shall support Model-1. If the target device supports SBAS and GNSS-UTC-Model assistance, it shall support Model-4. If the target device supports QZSS and GNSS-UTC-Model assistance, it shall support Model-1. If the target device supports Galileo and GNSS-UTC-Model assistance, it shall support Model-1. If the target device supports GLONASS and GNSS-UTC-Model assistance, it shall support Model-3. If the target device supports BDS and GNSS-UTC-Model assistance, it shall support Model-5. If this field is absent, the target device supports the mandatory (native) utc-Model choice only as listed above for the GNSS indicated by GNSS-ID.

#### –	GNSS-AuxiliaryInformationSupport

-- ASN1START

GNSS-AuxiliaryInformationSupport ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	BDS-DifferentialCorrectionsSupport

-- ASN1START

BDS-DifferentialCorrectionsSupport-r12 ::=	SEQUENCE {

gnssSignalIDs			GNSS-SignalIDs,

...

}

-- ASN1STOP

BDS-DifferentialCorrectionsSupport field descriptions

gnssSignalIDs This field specifies the BDS signal types for which differential corrections are supported by the target device. This is represented by a bit string in GNSS-SignalIDs, with a one-value at the bit position means differential corrections for the particular BDS signal type is supported; a zero-value means not supported.

#### –	BDS-GridModelSupport

-- ASN1START

BDS-GridModelSupport-r12 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-RTK-ObservationsSupport

-- ASN1START

GNSS-RTK-ObservationsSupport-r15 ::= 	SEQUENCE {

gnssSignalIDs-r15			GNSS-SignalIDs,

...

}

-- ASN1STOP

GNSS-RTK-ObservationsSupport field descriptions

gnssSignalIDs This field specifies the GNSS signal types for which GNSS-RTK-Observations are supported by the target device. This is represented by a bit string in GNSS-SignalIDs, with a one-value at the bit position means GNSS-RTK-Observations for the particular GNSS signal type is supported; a zero-value means not supported.

#### –	GLO-RTK-BiasInformationSupport

-- ASN1START

GLO-RTK-BiasInformationSupport-r15 ::= 	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-RTK-MAC-CorrectionDifferencesSupport

-- ASN1START

GNSS-RTK-MAC-CorrectionDifferencesSupport-r15 ::= 	SEQUENCE {

link-combinations-support-r15		GNSS-Link-CombinationsList-r15,

...

}

-- ASN1STOP

GNSS-RTK-MAC-CorrectionDifferencesSupport field descriptions

link-combinations-support This field specifies the GNSS link/frequency combinations for which GNSS-RTK-MAC-CorrectionDifferences are supported by the target device for the GNSS indicated by GNSS-ID.

#### –	GNSS-RTK-ResidualsSupport

-- ASN1START

GNSS-RTK-ResidualsSupport-r15 ::= 	SEQUENCE {

link-combinations-support-r15		GNSS-Link-CombinationsList-r15,

...

}

-- ASN1STOP

GNSS-RTK-ResidualsSupport field descriptions

link-combinations-support This field specifies the GNSS link/frequency combinations for which GNSS-RTK-Residuals are supported by the target device for the GNSS indicated by GNSS-ID.

#### –	GNSS-RTK-FKP-GradientsSupport

-- ASN1START

GNSS-RTK-FKP-GradientsSupport-r15 ::= 	SEQUENCE {

link-combinations-support-r15		GNSS-Link-CombinationsList-r15,

...

}

-- ASN1STOP

GNSS-RTK-FKP-GradientsSupport field descriptions

link-combinations-support This field specifies the GNSS link/frequency combinations for which GNSS-RTK-FKP-Gradients are supported by the target device for the GNSS indicated by GNSS-ID.

#### –	GNSS-SSR-OrbitCorrectionsSupport

-- ASN1START

GNSS-SSR-OrbitCorrectionsSupport-r15 ::= 	SEQUENCE {

...,

[[

orbit-IntegritySup-r17	BIT STRING {	orbitRangeErrorCorrelationTimeSup		(0),

orbitRangeRateErrorCorrelationTimeSup	(1)

} (SIZE(1..8))									OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-OrbitCorrectionsSupport field descriptions

orbit-IntegritySup This field, if present, indicates that the target device supports the IEs ORBIT-IntegrityParameters and SSR-IntegrityOrbitBounds. A one-value at the bit position '0' means that the target device supports the field orbitRangeErrorCorrelationTime in IE ORBIT-IntegrityParameters; a one-value at bit position '1' means that the target device supports the field orbitRangeRateErrorCorrelationTime in IE ORBIT-IntegrityParameters.

#### –	GNSS-SSR-ClockCorrectionsSupport

-- ASN1START

GNSS-SSR-ClockCorrectionsSupport-r15 ::= 	SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-SSR-CodeBiasSupport

-- ASN1START

GNSS-SSR-CodeBiasSupport-r15 ::= SEQUENCE {

signal-and-tracking-mode-ID-Sup-r15		GNSS-SignalIDs,

...,

[[

ssr-IntegrityCodeBiasBoundsSup-r17		ENUMERATED { supported }			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-CodeBiasSupport field descriptions

signal-and-tracking-mode-ID-Sup This field specifies the GNSS signal(s) for which the GNSS-SSR-CodeBias is supported by the target device.

ssr-IntegrityCodeBiasBoundsSup This field, if present, indicates that the target device supports the IE SSR-IntegrityCodeBiasBounds.

#### –	GNSS-SSR-URA-Support

-- ASN1START

GNSS-SSR-URA-Support-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

#### –	GNSS-SSR-PhaseBiasSupport

-- ASN1START

GNSS-SSR-PhaseBiasSupport-r16 ::= SEQUENCE {

signal-and-tracking-mode-ID-Sup-r16		GNSS-SignalIDs,

...,

[[

ssr-IntegrityPhaseBiasBoundsSup-r17		ENUMERATED { supported }			OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-PhaseBiasSupport field descriptions

signal-and-tracking-mode-ID-Sup This field specifies the GNSS signal(s) for which the GNSS-SSR-PhaseBias is supported by the target device.

ssr-IntegrityPhaseBiasBoundsSup This field, if present, indicates that the target device supported the IE SSR-IntegrityPhaseBiasBounds.

#### –	GNSS-SSR-STEC-CorrectionSupport

-- ASN1START

GNSS-SSR-STEC-CorrectionSupport-r16 ::= 	SEQUENCE {

...,

[[

stec-IntegritySup-r17	BIT STRING {	ionoRangeErrorCorrelationTimeSup		(0),

ionoRangeRateErrorCorrelationTimeSup	(1)

} (SIZE(1..8))									OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-STEC-CorrectionSupport field descriptions

stec-IntegritySup This field, if present, indicates that the target device supports the IEs STEC-IntegrityParameters and STEC-IntegrityErrorBounds. A one-value at the bit position '0' means that the target device supports the field ionoRangeErrorCorrelationTime in IE STEC-IntegrityParameters; a one-value at bit position '1' means that the target device supports the field ionoRangeRateErrorCorrelationTime in IE STEC-IntegrityParameters.

#### –	GNSS-SSR-GriddedCorrectionSupport

-- ASN1START

GNSS-SSR-GriddedCorrectionSupport-r16 ::= 	SEQUENCE {

...,

[[

griddedCorrectionIntegritySup-r17		ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

GNSS-SSR-GriddedCorrectionSupport field descriptions

griddedCorrectionIntegritySup This field, if present, indicates that the target device supports the IEs SSR-GriddedCorrectionIntegrityParameters and TropoDelayIntegrityErrorBounds.

#### –	NavIC-DifferentialCorrectionsSupport

-- ASN1START

NavIC-DifferentialCorrectionsSupport-r16 ::=	SEQUENCE {

gnssSignalIDs-r16			GNSS-SignalIDs,

...

}

-- ASN1STOP

NavIC-DifferentialCorrectionsSupport field descriptions

gnssSignalIDs This field specifies the NavIC signal types for which differential corrections are supported by the target device. This is represented by a bit string in GNSS-SignalIDs, with a one-value at the bit position means differential corrections for the particular NavIC signal type is supported; a zero-value means not supported.

#### –	NavIC-GridModelSupport

-- ASN1START

NavIC-GridModelSupport-r16 ::=	SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.2.11	GNSS Capability Information Request

#### –	A-GNSS-RequestCapabilities

The IE A-GNSS-Request-Capabilities is used by the location server to request A-GNSS location capabilities (e.g., GNSSs and assistance data supported) from the target device.

-- ASN1START

A-GNSS-RequestCapabilities ::= SEQUENCE {

gnss-SupportListReq				BOOLEAN,

assistanceDataSupportListReq	BOOLEAN,

locationVelocityTypesReq		BOOLEAN,

...

}

-- ASN1STOP

A-GNSS-RequestCapabilities field descriptions

gnss-SupportListReq This field specifies whether the target device is requested to include the gnss-SupportList field in the A-GNSS-ProvideCapabilities IE or not. TRUE means requested.

assistanceDataSupportListReq This field specifies whether the target device is requested to include the assistanceDataSupportList field in the A-GNSS-ProvideCapabilities IE or not. TRUE means requested.

locationVelocityTypesReq This field specifies whether the target device is requested to include the locationCoordinateTypes field and velocityTypes field in the A-GNSS-ProvideCapabilities IE or not. TRUE means requested.

#### 6.5.2.12	GNSS Error Elements

#### –	A-GNSS-Error

The IE A-GNSS-Error is used by the location server or target device to provide GNSS error reasons.

-- ASN1START

A-GNSS-Error ::= CHOICE {

locationServerErrorCauses		GNSS-LocationServerErrorCauses,

targetDeviceErrorCauses			GNSS-TargetDeviceErrorCauses,

...

}

-- ASN1STOP

#### –	GNSS-LocationServerErrorCauses

The IE GNSS-LocationServerErrorCauses is used by the location server to provide GNSS error reasons to the target device.

-- ASN1START

GNSS-LocationServerErrorCauses ::= SEQUENCE {

cause		ENUMERATED	{

undefined,

undeliveredAssistanceDataIsNotSupportedByServer,

undeliveredAssistanceDataIsSupportedButCurrentlyNotAvailableByServer,								undeliveredAssistanceDataIsPartlyNotSupportedAndPartlyNotAvailableByServer,

...,

unconfirmedPeriodicAssistanceDataIsNotSupported-v1510,

unconfirmedPeriodicAssistanceDataIsSupportedButCurrentlyNotAvailable-v1510,

unconfirmedPeriodicAssistanceDataIsPartlyNotSupportedAndPartlyNotAvailable-v1510,

undeliveredPeriodicAssistanceDataIsCurrentlyNotAvailable-v1510

},

...

}

-- ASN1STOP

GNSS-LocationServerErrorCauses field descriptions

cause This field provides a GNSS specific error cause. The cause values 'unconfirmedPeriodicAssistanceDataIsNotSupported', 'unconfirmedPeriodicAssistanceDataIsSupportedButCurrentlyNotAvailable' and 'unconfirmedPeriodicAssistanceDataIsPartlyNotSupportedAndPartlyNotAvailable' may only be included in the control transaction of a periodic assistance data transfer procedure, as described in clause 5.2.1a. The cause value 'undeliveredPeriodicAssistanceDataIsCurrentlyNotAvailable' may only be included in the data transaction of a periodic assistance data transfer procedure when periodic assistance data are not available when the periodicity condition occurs, as described in clauses 5.2.1a and 5.2.2a.

#### –	GNSS-TargetDeviceErrorCauses

The IE GNSS-TargetDeviceErrorCauses is used by the target device to provide GNSS error reasons to the location server.

-- ASN1START

GNSS-TargetDeviceErrorCauses ::= SEQUENCE {

cause		ENUMERATED {	undefined,

thereWereNotEnoughSatellitesReceived,

assistanceDataMissing,

notAllRequestedMeasurementsPossible,

...

},

fineTimeAssistanceMeasurementsNotPossible		NULL		OPTIONAL,

adrMeasurementsNotPossible						NULL		OPTIONAL,

multiFrequencyMeasurementsNotPossible			NULL		OPTIONAL,

...

}

-- ASN1STOP

GNSS-TargetDeviceErrorCauses field descriptions

cause This field provides a GNSS specific error cause. If the cause value is 'notAllRequestedMeasurementsPossible', the target device was not able to provide all requested GNSS measurements (but may be able to report a location estimate or location measurements). In this case, the target device should include any of the fineTimeAssistanceMeasurementsNotPossible, adrMeasurementsNotPossible, or multiFrequenceMeasurementsNotPossible fields, as applicable.

#### 6.5.2.13	Common GNSS Information Elements

#### –	GNSS-FrequencyID

The IE GNSS-FrequencyID is used to indicate a specific GNSS link/frequency. The interpretation of GNSS-FrequencyID depends on the GNSS-ID.

-- ASN1START

GNSS-FrequencyID-r15	::= SEQUENCE {

gnss-FrequencyID-r15		INTEGER (0 .. 7),

...

}

-- ASN1STOP

GNSS-FrequencyID field descriptions

gnss-FrequencyID This field specifies a particular GNSS link/frequency. The interpretation of gnss-FrequencyID depends on the GNSS-ID and is as shown in the table Value & Explanation relation below.

Value & Explanation relation

| System | Value | Explanation |  |
| --- | --- | --- | --- |
|  |  | Link | Centre Frequency [MHz] |
| GPS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| SBAS | 0 | L1 | 1575.42 |
|  | 1 | L5 | 1176.45 |
|  | 2-7 | reserved |  |
| QZSS | 0 | L1 | 1575.42 |
|  | 1 | L2 | 1227.60 |
|  | 2 | L5 | 1176.45 |
|  | 3-7 | reserved |  |
| GLONASS k = -7..13 | 0 | G1 | 1602+k×0.5625 |
|  | 1 | G2 | 1246+k×0.4375 |
|  | 2 | G3 | 1202.025 |
|  | 3-7 | reserved |  |
| Galileo | 0 | E1 | 1575.420 |
|  | 1 | E6 | 1278.750 |
|  | 2 | E5a | 1176.450 |
|  | 3 | E5b | 1207.140 |
|  | 4 | E5 | 1191.795 |
|  | 5-7 | reserved |  |
| BDS | 0 | B1I | 1561.098 |
|  | 1 | B1C | 1575.420 |
|  | 2 | B2 | 1207.140 |
|  | 3 | B3 | 1268.520 |
|  | 4 | B2a | 1176.450 |
|  | 5-7 | reserved |  |
| NavIC | 0 | L5 | 1176.450 |
|  | 1-7 | reserved |  |

#### –	GNSS-ID

The IE GNSS-ID is used to indicate a specific GNSS.

-- ASN1START

GNSS-ID ::= SEQUENCE {

gnss-id				ENUMERATED{ gps, sbas, qzss, galileo, glonass, ..., bds, navic-v1610 },

...

}

-- ASN1STOP

#### –	GNSS-ID-Bitmap

The IE GNSS-ID-Bitmap is used to indicate several GNSSs using a bit map.

-- ASN1START

GNSS-ID-Bitmap ::= SEQUENCE {

gnss-ids			BIT STRING {	gps			(0),

sbas		(1),

qzss		(2),

galileo		(3),

glonass		(4),

bds			(5),

navic-v1610	(6) } (SIZE (1..16)),

...

}

-- ASN1STOP

GNSS-ID-Bitmap field descriptions

gnss-ids This field specifies the GNSS(s). This is represented by a bit string, with a one-value at the bit position means the particular GNSS is addressed; a zero-value means not addressed.

#### –	GNSS-Link-CombinationsList

-- ASN1START

GNSS-Link-CombinationsList-r15 ::= SEQUENCE (SIZE(1..8)) OF GNSS-Link-Combinations-r15

GNSS-Link-Combinations-r15 ::= SEQUENCE {

l1-r15		GNSS-FrequencyID-r15,

l2-r15		GNSS-FrequencyID-r15,

...

}

-- ASN1STOP

#### –	GNSS-NavListInfo

-- ASN1START

GNSS-NavListInfo-r15 ::= SEQUENCE (SIZE (1..64)) OF SatListElement-r15

SatListElement-r15 ::= SEQUENCE {

svID-r15		SV-ID,

iod-r15			BIT STRING (SIZE(11)),

...

}

-- ASN1STOP

#### –	GNSS-NetworkID

The IE GNSS-NetworkID defines the reference network and the source of the particular set of reference stations and their observation information. This IE is used for MAC Network RTK as described in [30].

-- ASN1START

GNSS-NetworkID-r15 ::= SEQUENCE {

networkID-r15					INTEGER (0..255),

...

}

-- ASN1STOP

#### –	GNSS-PeriodicControlParam

The IE GNSS-PeriodicControlParam is used to specify control parameters for a periodic assistance data delivery.

-- ASN1START

GNSS-PeriodicControlParam-r15 ::= SEQUENCE {

deliveryAmount-r15			INTEGER (1..32),

deliveryInterval-r15		INTEGER (1..64),

...

}

-- ASN1STOP

GNSS-PeriodicControlParam field descriptions

deliveryAmount This field specifies the number of periodic assistance data deliveries. Integer values N=1…31 correspond to an amount of 2N. Integer value N=32 indicates an 'infinite/indefinite' amount, which means that the assistance data delivery should continue until a LPP Abort message is received.

deliveryInterval This field specifies the interval between assistance data deliveries in seconds.

#### –	GNSS-ReferenceStationID

The IE GNSS-ReferenceStationID is used to identify a specific GNSS Reference Station.

-- ASN1START

GNSS-ReferenceStationID-r15 ::= SEQUENCE {

referenceStationID-r15			INTEGER (0..65535),

providerName-r15				VisibleString (SIZE (1..32))		OPTIONAL, -- Need ON

...

}

-- ASN1STOP

GNSS-ReferenceStationID field descriptions

referenceStationID This field provides the reference station identity.

providerName This field is associated with a GNSS correction data provider to ensure that the referenceStationID's are unique from a target device perspective.

#### –	GNSS-SignalID

The IE GNSS-SignalID is used to indicate a specific GNSS signal type. The interpretation of GNSS-SignalID depends on the GNSS-ID.

-- ASN1START

GNSS-SignalID	::= SEQUENCE {

gnss-SignalID		INTEGER (0 .. 7),

...,

[[

gnss-SignalID-Ext-r15	INTEGER (8..23)			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

GNSS-SignalID field descriptions

gnss-SignalID, gnss-SignalID-Ext This field specifies a particular GNSS signal. The interpretation of gnss-SignalID and gnss-SignalID-Ext depends on the GNSS-ID and is as shown in the table System to Value & Explanation relation below. If the field gnss-SignalID-Ext is present, the gnss-SignalID should be set to value 7 and shall be ignored by the receiver.

System to Value & Explanation relation

| System | Value | Explanation |
| --- | --- | --- |
| GPS | 0 | GPS L1 C/A |
|  | 1 | GPS L1C |
|  | 2 | GPS L2C |
|  | 3 | GPS L5 |
|  | 4 | GPS L1 P |
|  | 5 | GPS L1 Z-tracking |
|  | 6 | GPS L2 C/A |
|  | 7 | GPS L2 P |
|  | 8 | GPS L2 Z-tracking |
|  | 9 | GPS L2 L2C(M) |
|  | 10 | GPS L2 L2C(L) |
|  | 11 | GPS L2 L2C(M+L) |
|  | 12 | GPS L5 I |
|  | 13 | GPS L5 Q |
|  | 14 | GPS L5 I+Q |
|  | 15 | GPS L1 L1C(D) |
|  | 16 | GPS L1 L1C(P) |
|  | 17 | GPS L1 L1C(D+P) |
|  | 18-23 | Reserved |
| SBAS | 0 | L1 C/A |
|  | 1 | L5 I |
|  | 2 | L5 Q |
|  | 3 | L5 I+Q |
|  | 4-7 | Reserved |
| QZSS | 0 | QZS-L1 C/A |
|  | 1 | QZS-L1C |
|  | 2 | QZS-L2C |
|  | 3 | QZS-L5 |
|  | 4 | QZS-LEX S |
|  | 5 | QZS-LEX L |
|  | 6 | QZS-LEX S+L |
|  | 7 | QZS-L2 L2C(M) |
|  | 8 | QZS-L2 L2C(L) |
|  | 9 | QZS-L2 L2C(M+L) |
|  | 10 | QZS-L5 I |
|  | 11 | QZS-L5 Q |
|  | 12 | QZS-L5 I+Q |
|  | 13 | QZS L1 L1C(D) |
|  | 14 | QZS L1 L1C(P) |
|  | 15 | QZS L1 L1C(D+P) |
|  | 16-23 | Reserved |
| GLONASS | 0 | GLONASS G1 C/A |
|  | 1 | GLONASS G2 C/A |
|  | 2 | GLONASS G3 |
|  | 3 | GLONASS G1 P |
|  | 4 | GLONASS G2 P |
|  | 5 | GLONASS G1a(D) |
|  | 6 | GLONASS G1a(P) |
|  | 7 | GLONASS G1a (D+P) |
|  | 8 | GLONASS G2a(I) |
|  | 9 | GLONASS G2a(P) |
|  | 10 | GLONASS G2a(I+P) |
|  | 11 | GLONASS G3 I |
|  | 12 | GLONASS G3 Q |
|  | 13 | GLONASS G3 I+Q |
|  | 14-23 | Reserved |
| Galileo | 0 | Galileo E1 |
|  | 1 | Galileo E5A |
|  | 2 | Galileo E5B |
|  | 3 | Galileo E6 |
|  | 4 | Galileo E5A + E5B |
|  | 5 | Galileo E1 C No data |
|  | 6 | Galileo E1 A |
|  | 7 | Galileo E1 B I/NAV OS/CS/SoL |
|  | 8 | Galileo E1 B+C |
|  | 9 | Galileo E1 A+B+C |
|  | 10 | Galileo E6 C |
|  | 11 | Galileo E6 A |
|  | 12 | Galileo E6 B |
|  | 13 | Galileo E6 B+C |
|  | 14 | Galileo E6 A+B+C |
|  | 15 | Galileo E5B I |
|  | 16 | Galileo E5B Q |
|  | 17 | Galileo E5B I+Q |
|  | 18 | Galileo E5(A+B) I |
|  | 19 | Galileo E5(A+B) Q |
|  | 20 | Galileo E5(A+B) I+Q |
|  | 21 | Galileo E5A I |
|  | 22 | Galileo E5A Q |
|  | 23 | Galileo E5A I+Q |
| BDS | 0 | B1 I |
|  | 1 | B1 Q |
|  | 2 | B1 I+Q |
|  | 3 | B3 I |
|  | 4 | B3 Q |
|  | 5 | B3 I+Q |
|  | 6 | B2 I |
|  | 7 | B2 Q |
|  | 8 | B2 I+Q |
|  | 9 | B1C(D) |
|  | 10 | B1C(P) |
|  | 11 | B1C(D+P) |
|  | 12 | B2a(D) |
|  | 13 | B2a(P) |
|  | 14 | B2a(D+P) |
|  | 15-23 | Reserved |
| NavIC | 0 | NavIC L5 SPS |
|  | 1-23 | Reserved |

#### –	GNSS-SignalIDs

The IE GNSSSignal-IDs is used to indicate several GNSS signals using a bit map. The interpretation of GNSSSignal-IDs depends on the GNSS-ID.

-- ASN1START

GNSS-SignalIDs	::= SEQUENCE {

gnss-SignalIDs		BIT STRING (SIZE(8)),

...,

[[

gnss-SignalIDs-Ext-r15	BIT STRING (SIZE(16))			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

GNSS-SignalIDs field descriptions

gnss-SignalIDs, gnss-SignalIDs-Ext This field specifies one or several GNSS signals using a bit map. A one-value at the bit position means the particular signal is addressed; a zero-value at the particular bit position means the signal is not addressed. The interpretation of the bit map in gnssSignalIDs and gnss-SignalIDs-Ext depends on the GNSS-ID and is shown in the table below. Unfilled table entries indicate no assignment and shall be set to zero.

Interpretation of the bit map in gnssSignalIDs

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1 C/A | L1C | L2C | L5 | L1P | L1 Z | L2 C/A | L2 P |
| SBAS | L1 C/A | L5 I | L5 Q | L5 I+Q |  |  |  |  |
| QZSS | QZS-L1 C/A | QZS-L1C | QZS-L2C | QZS-L5 | LEX S | LEX L | LEX S+L | L2C(M) |
| GLONASS | G1 C/A | G2 C/A | G3 | G1 P | G2 P | G1a(D) | G1a(P) | G1a(D+P) |
| Galileo | E1 | E5a | E5b | E6 | E5a+E5b | E1 C No Data | E1 A | E1 B I/NAV OS/CS/SoL |
| BDS | B1 I | B1 Q | B1 I+Q | B3 I | B3 Q | B3 I+Q | B2 I | B2 Q |
| NavIC | L5 SPS |  |  |  |  |  |  |  |

Interpretation of the bit map in gnssSignalIDs-Ext

| GNSS | Bit 1 (MSB) | Bit 2 | Bit 3 | Bit 4 | Bit 5 | Bit 6 | Bit 7 | Bit 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L2 Z | L2C(M) | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS | L2C(L) | L2C(M+L) | L5 I | L5 Q | L5 I+Q | L1C(D) | L1C(P) | L1C(D+P) |
| GLONASS | G2a(I) | G2a(P) | G2a(I+P) | G3 I | G3 Q | G3(I+Q) |  |  |
| Galileo | E1 B+C | E1 A+B+C | E6C | E6A | E6B | E6 B+C | E6 A+B+C | E5B I |
| BDS | B2 I+Q | B1C(D) | B1C(P) | B1C(D+P) | B2a(D) | B2a(P) | B2a(D+P) |  |
| NavIC |  |  |  |  |  |  |  |  |

| GNSS | Bit 9 | Bit 10 | Bit 11 | Bit 12 | Bit 13 | Bit 14 | Bit 15 | Bit 16 (LSB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GPS | L1C(P) | L1C(D+P) |  |  |  |  |  |  |
| SBAS |  |  |  |  |  |  |  |  |
| QZSS |  |  |  |  |  |  |  |  |
| GLONASS |  |  |  |  |  |  |  |  |
| Galileo | E5B Q | E5B I+Q | E5(A+B) I | E5(A+B) Q | E5(A+B) I+Q | E5A I | E5A Q | E5A I+Q |
| BDS |  |  |  |  |  |  |  |  |
| NavIC |  |  |  |  |  |  |  |  |

#### –	GNSS-SubNetworkID

The IE GNSS-SubNetworkID defines the subnetwork of a network identified by GNSS-NetworkID. This IE is used for MAC Network RTK as described in [30].

-- ASN1START

GNSS-SubNetworkID-r15 ::= SEQUENCE {

subNetworkID-r15				INTEGER (0..15),

...

}

-- ASN1STOP

#### –	SBAS-ID

The IE SBAS-ID is used to indicate a specific SBAS.

-- ASN1START

SBAS-ID ::= SEQUENCE {

sbas-id				ENUMERATED { waas, egnos, msas, gagan, ...},

...

}

-- ASN1STOP

#### –	SBAS-IDs

The IE SBAS-IDs is used to indicate several SBASs using a bit map.

-- ASN1START

SBAS-IDs ::= SEQUENCE {

sbas-IDs		BIT STRING {	waas		(0),

egnos		(1),

msas		(2),

gagan		(3)	} (SIZE (1..8)),

...

}

-- ASN1STOP

SBAS-IDs field descriptions

sbas-IDs This field specifies one or several SBAS(s) using a bit map. A one-value at the bit position means the particular SBAS is addressed; a zero-value at the particular bit position means the SBAS is not addressed.

#### –	SV-ID

The IE SV-ID is used to indicate a specific GNSS satellite. The interpretation of SV-ID depends on the GNSS-ID.

-- ASN1START

SV-ID ::= SEQUENCE {

satellite-id		INTEGER(0..63),

...

}

-- ASN1STOP

SV-ID field descriptions

satellite-id This field specifies a particular satellite within a specific GNSS. The interpretation of satellite-id depends on the GNSS-ID see the table below.

Interpretation of satellite-id

| System | Value of satellite-id | Interpretation of satellite-id |
| --- | --- | --- |
| GPS | '0' – '62' '63' | Satellite PRN Signal No. 1 to 63 Reserved |
| SBAS | '0' – '38' '39' – '63' | Satellite PRN Signal No. 120 to 158 Reserved |
| QZSS | '0' – '9' '10' – '63' | Satellite PRN Signal No. 193 to 202 Reserved |
| GLONASS | '0' – '23' '24 – '63' | Slot Number 1 to 24 Reserved |
| Galileo | '0' – '35' '36' – '63' | Code No. 1 to 36 Reserved |
| BDS | '0' – '62' '63' | Satellite ranging code number No.1 to 63 Reserved |
| NavIC | '0' – '13' '14'–'63' | Satellite PRN Signal No. 1 to 14 Reserved |

### 6.5.3	Enhanced Cell ID Positioning

#### 6.5.3.1	E-CID Location Information

#### –	ECID-ProvideLocationInformation

The IE ECID-ProvideLocationInformation is used by the target device to provide E-CID location measurements to the location server. It may also be used to provide E-CID positioning specific error reason.

-- ASN1START

ECID-ProvideLocationInformation ::= SEQUENCE {

ecid-SignalMeasurementInformation	ECID-SignalMeasurementInformation		OPTIONAL,

ecid-Error							ECID-Error								OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.3.2	E-CID Location Information Elements

#### –	ECID-SignalMeasurementInformation

The IE ECID-SignalMeasurementInformation is used by the target device to provide various UE-measurements to the location server.

-- ASN1START

ECID-SignalMeasurementInformation ::= SEQUENCE {

primaryCellMeasuredResults	MeasuredResultsElement	OPTIONAL,

measuredResultsList			MeasuredResultsList,

...

}

MeasuredResultsList ::= SEQUENCE (SIZE(1..32)) OF MeasuredResultsElement

MeasuredResultsElement ::= SEQUENCE {

physCellId						INTEGER (0..503),

cellGlobalId					CellGlobalIdEUTRA-AndUTRA	OPTIONAL,

arfcnEUTRA						ARFCN-ValueEUTRA,

systemFrameNumber				BIT STRING (SIZE (10))		OPTIONAL,

rsrp-Result						INTEGER (0..97)				OPTIONAL,

rsrq-Result						INTEGER (0..34)				OPTIONAL,

ue-RxTxTimeDiff					INTEGER (0..4095)			OPTIONAL,

...,

[[	arfcnEUTRA-v9a0				ARFCN-ValueEUTRA-v9a0		OPTIONAL		-- Cond EARFCN-max

]],

[[	nrsrp-Result-r14			INTEGER (0..113)			OPTIONAL,

nrsrq-Result-r14			INTEGER (0..74)				OPTIONAL,

carrierFreqOffsetNB-r14		CarrierFreqOffsetNB-r14		OPTIONAL,		-- Cond NB-IoT

hyperSFN-r14				BIT STRING (SIZE (10))		OPTIONAL

]],

[[

rsrp-Result-v1470			INTEGER (-17..-1)			OPTIONAL,

rsrq-Result-v1470			INTEGER (-30..46)			OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| EARFCN-max | The field is mandatory present if the corresponding arfcnEUTRA (i.e. without suffix) is set to maxEARFCN. Otherwise the field is not present. |
| NB-IoT | The field is mandatory present if the measured cell is a NB-IoT cell. Otherwise it is not present. |

ECID-SignalMeasurementInformation field descriptions

primaryCellMeasuredResults This field contains measurements for the primary cell (if the primary cell is a E-UTRA or NB-IoT cell), when the target device reports measurements for both primary cell (E-UTRA or NB-IoT) and neighbour cells. This field shall be omitted when the target device reports measurements for the primary cell (E-UTRA or NB-IoT) only, in which case the measurements for the primary cell (E-UTRA or NB-IoT) is reported in the measuredResultsList. This field shall be omitted when the primary cell is not a E-UTRA or NB-IoT cell.

measuredResultsList This list contains the E-CID measurements for up to 32 E-UTRA or NB-IoT cells.

physCellId This field specifies the physical cell identity of the measured cell.

cellGlobalId This field specifies cell global ID of the measured cell. The target device shall provide this field if it was able to determine the ECGI of the measured cell at the time of measurement.

arfcnEUTRA This field specifies the ARFCN of the measured E-UTRA carrier frequency, as defined in TS 36.331 [12]. In the case the target device includes arfcnEUTRA-v9a0, the target device shall set the corresponding arfcnEUTRA (i.e. without suffix) to maxEARFCN.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

rsrp-Result This field specifies the reference signal received power (RSRP) measurement, as defined in TS 36.331 [12], TS 36.214 [17]. In the case the target device includes rsrp-Result-v1470, the target device shall set the corresponding rsrp-Result (i.e. without suffix) to value 0.

rsrq-Result This field specifies the reference signal received quality (RSRQ) measurement, as defined in TS 36.331 [12], TS 36.214 [17]. In the case the target device includes rsrq-Result-v1470, the target device shall set the corresponding rsrq-Result (i.e. without suffix) to value 0 or 34.

ue-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 36.214 [17]. It is provided only for measurements on the UE's primary cell. Measurement report mapping is according to TS 36.133 [18].

nrsrp-Result This field specifies the narrowband reference signal received power (NRSRP) measurement, as defined in TS 36.214 [17]. Measurement report mapping is according to TS 36.133 [18].

nrsrq-Result This field specifies the narrowband reference signal received quality (NRSRQ) measurement, as defined in TS 36.214 [17]. Measurement report mapping to the value defined in TS 36.133 [18]. Values 0..29 map to values NRSRQ_-30..NRSRQ_-1. Values 30..62 map to NRSRQ_01..NRSRQ_33. Values 63..74 map to NRSRQ_35..NRSRQ_46. The UE does not report NRSRQ_00 nor NRSRQ_34.

carrierFreqOffsetNB This field specifies the offset of the NB-IoT channel number to ARFCN given by arfcnEUTRA as defined in TS 36.101 [21].

hyperSFN This field specifies the hyper-SFN of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the hyper-SFN of the cell at the time of measurement.

#### 6.5.3.3	E-CID Location Information Request

#### –	ECID-RequestLocationInformation

The IE ECID-RequestLocationInformation is used by the location server to request E-CID location measurements from a target device.

-- ASN1START

ECID-RequestLocationInformation ::= SEQUENCE {

requestedMeasurements		BIT STRING {	rsrpReq		(0),

rsrqReq		(1),

ueRxTxReq	(2),

nrsrpReq-r14	(3),

nrsrqReq-r14	(4)} (SIZE(1..8)),

...

}

-- ASN1STOP

ECID-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the E-CID measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

#### 6.5.3.4	E-CID Capability Information

#### –	ECID-ProvideCapabilities

The IE ECID-ProvideCapabilities is used by the target device to indicate its capability to support E-CID and to provide its E-CID location capabilities to the location server.

-- ASN1START

ECID-ProvideCapabilities ::= SEQUENCE {

ecid-MeasSupported	BIT STRING {	rsrpSup		(0),

rsrqSup		(1),

ueRxTxSup	(2),

nrsrpSup-r14	(3),

nrsrqSup-r14	(4)} (SIZE(1..8)),

...,

[[	ueRxTxSupTDD-r13					ENUMERATED { true }				OPTIONAL

]],

[[	periodicalReporting-r14				ENUMERATED { supported }		OPTIONAL,

triggeredReporting-r14				ENUMERATED { supported }		OPTIONAL,

idleStateForMeasurements-r14		ENUMERATED { required }			OPTIONAL

]],

[[	scheduledLocationRequest-r17		SEQUENCE {

utcTime-r17			ENUMERATED { supported }	OPTIONAL,

gnssTime-r17		GNSS-ID-Bitmap				OPTIONAL,

e-utraTime-r17		ENUMERATED { supported }	OPTIONAL,

nrTime-r17			ENUMERATED { supported }	OPTIONAL,

relativeTime-r17	ENUMERATED { supported }	OPTIONAL,

...

}									OPTIONAL

]]

}

-- ASN1STOP

ECID-Provide-Capabilities field descriptions

ecid-MeasSupported This field specifies the E-CID measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Cell ID positioning method is supported by the target device. If the UE Rx-Tx time difference measurement is supported by the target device (i.e., ueRxTxSup field is set to one), it means that the UE supports the UE Rx-Tx time difference measurement reporting via both LPP signaling and RRC signalling. If a target device doesn't support LPP, the E-SMLC may assume the target device can not report the UE Rx-Tx time difference measurement results via RRC signalling.

ueRxTxSupTDD This field, if present, indicates that any UE Rx-Tx time difference measurement reporting for TDD from the target device includes the NTAoffset according to TS 36.211 [16], TS 36.214 [17] and uses the UE Rx-Tx time difference measurement report mapping for TDD as specified in TS 36.133 [18]. This field may only be included if the ueRxTxSup field in ecid-MeasSupported is set to value one.

periodicalReporting This field, if present, indicates that the target device supports periodicalReporting of E-CID measurements. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

triggeredReporting This field, if present, indicates that the target device supports triggeredReporting for the cellChange event. If this field is absent, the location server may assume that the target device does not support triggeredReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform E-CID measurements.

scheduledLocationRequest This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

#### 6.5.3.5	E-CID Capability Information Request

#### –	ECID-RequestCapabilities

The IE ECID-RequestCapabilities is used by the location server to request E-CID positioning capabilities from a target device.

-- ASN1START

ECID-RequestCapabilities ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.3.6	E-CID Error Elements

#### –	ECID-Error

The IE ECID-Error is used by the location server or target device to provide E-CID error reasons to the target device or location server, respectively.

-- ASN1START

ECID-Error ::= CHOICE {

locationServerErrorCauses		ECID-LocationServerErrorCauses,

targetDeviceErrorCauses			ECID-TargetDeviceErrorCauses,

...

}

-- ASN1STOP

#### –	ECID-LocationServerErrorCauses

The IE ECID-LocationServerErrorCauses is used by the location server to provide E-CID error reasons to the target device.

-- ASN1START

ECID-LocationServerErrorCauses ::= SEQUENCE {

cause		ENUMERATED	{	undefined,

...

},

...

}

-- ASN1STOP

#### –	ECID-TargetDeviceErrorCauses

The IE ECID-TargetDeviceErrorCauses is used by the target device to provide E-CID error reasons to the location server.

-- ASN1START

ECID-TargetDeviceErrorCauses ::= SEQUENCE {

cause		ENUMERATED {	undefined,

requestedMeasurementNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

rsrpMeasurementNotPossible				NULL		OPTIONAL,

rsrqMeasurementNotPossible				NULL		OPTIONAL,

ueRxTxMeasurementNotPossible			NULL		OPTIONAL,

...,

[[

nrsrpMeasurementNotPossible-r14		NULL		OPTIONAL,

nrsrqMeasurementNotPossible-r14		NULL		OPTIONAL

]]

}

-- ASN1STOP

ECID-TargetDeviceErrorCauses field descriptions

cause This field provides a E-CID specific error cause. If the cause value is 'notAllRequestedMeasurementsPossible', the target device was not able to provide all requested E-CID measurements (but may be able to provide some measurements). In this case, the target device should include any of the rsrpMeasurementNotPossible, rsrqMeasurementNotPossible, ueRxTxMeasurementNotPossible, nrsrpMeasurementNotPossible, or nrsrqMeasurementNotPossible fields, as applicable.

### 6.5.4	Terrestrial Beacon System Positioning

#### 6.5.4.1	TBS Location Information

#### –	TBS-ProvideLocationInformation

The IE TBS-ProvideLocationInformation is used by the target device to provide TBS location measurements to the location server. It may also be used to provide TBS positioning specific error reason.

-- ASN1START

TBS-ProvideLocationInformation-r13 ::= SEQUENCE {

tbs-MeasurementInformation-r13			TBS-MeasurementInformation-r13		OPTIONAL,

tbs-Error-r13							TBS-Error-r13						OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.4.2	TBS Location Information Elements

#### –	TBS-MeasurementInformation

The IE TBS-MeasurementInformation is used by the target device to provide TBS location measurements to the location server.

-- ASN1START

TBS-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13	UTCTime						OPTIONAL,

mbs-SgnMeasList-r13				MBS-BeaconMeasList-r13		OPTIONAL,	-- Cond MBS

...

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| MBS | The field is mandatory present if the TBS-MeasurementInformation is provided for an MBS system; otherwise it is not present. |

TBS-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the TBS measurements are performed and should take the form of YYMMDDhhmmssZ.

mbs-SgnMeasList This field provides the MBS measurements for up to 64 MBS beacons.

#### –	MBS-BeaconMeasList

The IE MBS-BeaconMeasList is used by the target device to provide MBS location measurements to the location server, as defined in the MBS ICD [24].

-- ASN1START

MBS-BeaconMeasList-r13 ::= SEQUENCE (SIZE(1..64)) OF MBS-BeaconMeasElement-r13

MBS-BeaconMeasElement-r13 ::= SEQUENCE {

transmitterID-r13				INTEGER (0..32767),

codePhase-r13					INTEGER (0..2097151),

codePhaseRMSError-r13			INTEGER (0..63),

...,

[[ rssi-r14						INTEGER (-130..-30)		OPTIONAL

]]

}

-- ASN1STOP

MBS-BeaconMeasList field descriptions

transmitterID This field contains the MBS transmitter identifier.

codePhase This field contains the value of the code-phase measurement made by the target device for the particular beacon signal at the time of measurement in the units of ms. MBS specific code phase measurements (e.g. chips) are converted into unit of ms by dividing the measurements by the nominal values of the measured signal chipping rate. Scale factor 2-21 milli-seconds, in the range from 0 to (1-2-21) milli-seconds.

codePhaseRMSError This field contains the pseudorange RMS error value. This parameter is specified according to a floating-point representation shown in the table below.

rssi This field provides an estimate of the received signal strength from the MBS beacon as referenced to the UE antenna connector. If the estimated received signal strength for the MBS beacon is less than -130 dBm, the UE shall report an RSSI value of -130. If the estimated received signal strength for the MBS beacon is greater than -30 dBm, the UE shall report an RSSI value of -30. Scale factor 1 dBm.

floating-point representation

| Index | Mantissa | Exponent | Floating-Point value, xi | Pseudorange value, P [m] |
| --- | --- | --- | --- | --- |
| 0 | 000 | 000 | 0.5 | P < 0.5 |
| 1 | 001 | 000 | 0.5625 | 0.5 <= P < 0.5625 |
| i | x | y | 0.5 * (1 + x/8) * 2y | xi-1 <= P < xi |
| 62 | 110 | 111 | 112 | 104 <= P < 112 |
| 63 | 111 | 111 | -- | 112 <= P |

#### 6.5.4.3	TBS Location Information Request

#### –	TBS-RequestLocationInformation

The IE TBS-RequestLocationInformation is used by the location server to request location information for TBS-based methods from the target device.

-- ASN1START

TBS-RequestLocationInformation-r13 ::= SEQUENCE {

mbsSgnMeasListReq-r13				BOOLEAN,

...,

[[	mbsAssistanceAvailability-r14	BOOLEAN							OPTIONAL,	-- Need ON

mbsRequestedMeasurements-r14	BIT STRING {

rssi		(0)} (SIZE(1..8))	OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

TBS-RequestLocationInformation field descriptions

mbsSgnMeasListReq This field indicates whether the target device is requested to report MBS measurements in TBS-MeasurementInformation IE or not. TRUE means requested.

mbsAssistanceAvailability This field indicates whether the target device may request additional MBS assistance data from the server. TRUE means allowed and FALSE means not allowed.

mbsRequestedMeasurements This field indicates the additional MBS measurements requested and may only be included if mbsSgnMeasListReq is set to TRUE. This field is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. rssi: Beacon signal strength at the target

#### 6.5.4.4	TBS Capability Information

#### –	TBS-ProvideCapabilities

The IE TBS-ProvideCapabilities is used by the target device to indicate its capability to support TBS and to provide its TBS location capabilities to the location server.

-- ASN1START

TBS-ProvideCapabilities-r13 ::= SEQUENCE {

tbs-Modes-r13			BIT STRING {	standalone		(0),

ue-assisted		(1),

ue-based		(2)} (SIZE (1..8)),

...,

[[	mbs-AssistanceDataSupportList-r14	MBS-AssistanceDataSupportList-r14		OPTIONAL,

periodicalReportingSupported-r14	PositioningModes						OPTIONAL,

mbs-ConfigSupport-r14	BIT STRING {	tb1		(0),

tb2		(1),

tb3		(2),

tb4		(3)} (SIZE (1..8))			OPTIONAL,

mbs-IdleStateForMeasurements-r14		ENUMERATED { required }				OPTIONAL

]],

[[	scheduledLocationRequest-r17		SEQUENCE {

utcTime-r17			PositioningModes		OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}										OPTIONAL,

e-utraTime-r17		PositioningModes		OPTIONAL,

nrTime-r17			PositioningModes		OPTIONAL,

relativeTime-r17	PositioningModes		OPTIONAL,

...

}											OPTIONAL

]]

}

-- ASN1STOP

TBS-ProvideCapabilities field descriptions

tbs-Modes This field specifies the TBS mode(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular TBS mode is supported; a zero-value means not supported.

mbs-AssistanceDataSupportList This list defines the MBS assistance data supported by the target device. This field shall be present if the target device supports MBS assistance data.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

mbs-ConfigSupport This field specifies the MBS configurations supported by the target device. This field shall be present if the target device supports MBS [24].

mbs-IdleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform MBS measurements.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

#### -	MBS-AssistanceDataSupportList

The IE MBS-AssistanceDataSupportList is used by the target device to indicate its capability to support MBS Assistance Data and to provide its capabilities to the location server.

-- ASN1START

MBS-AssistanceDataSupportList-r14 ::= SEQUENCE {

mbs-AcquisitionAssistanceDataSupport-r14	BOOLEAN,

mbs-AlmanacAssistanceDataSupport-r14		BOOLEAN,

...

}

-- ASN1STOP

MBS-AssistanceDataSupportList field descriptions

mbs-AcquisitionAssistanceDataSupport This field specifies whether the target device supports MBS Acquisition Assistance Data. TRUE means supported.

mbs-AlmanacAssistanceDataSupport This field specifies whether the target device supports MBS Almanac Assistance Data. TRUE means supported.

#### 6.5.4.5	TBS Capability Information Request

#### –	TBS-RequestCapabilities

The IE TBS-RequestCapabilities is used by the location server to request TBS positioning capabilities from a target device.

-- ASN1START

TBS-RequestCapabilities-r13 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.4.6	TBS Error Elements

#### –	TBS-Error

The IE TBS-Error is used by the location server or target device to provide TBS error reasons to the target device or location server, respectively.

-- ASN1START

TBS-Error-r13 ::= CHOICE {

locationServerErrorCauses-r13		TBS-LocationServerErrorCauses-r13,

targetDeviceErrorCauses-r13			TBS-TargetDeviceErrorCauses-r13,

...

}

-- ASN1STOP

#### –	TBS-LocationServerErrorCauses

The IE TBS-LocationServerErrorCauses is used by the location server to provide error reasons for TBS positioning to the target device.

-- ASN1START

TBS-LocationServerErrorCauses-r13 ::= SEQUENCE {

cause-r13		ENUMERATED	{	undefined,

...,

assistanceDataNotSupportedByServer-v1420,

assistanceDataSupportedButCurrentlyNotAvailableByServer-v1420

},

...

}

-- ASN1STOP

#### –	TBS-TargetDeviceErrorCauses

The IE TBS-TargetDeviceErrorCauses is used by the target device to provide error reasons for TBS positioning to the location server.

-- ASN1START

TBS-TargetDeviceErrorCauses-r13 ::= SEQUENCE {

cause-r13		ENUMERATED {	undefined,

thereWereNotEnoughMBSBeaconsReceived,

...,

assistanceDataMissing-v1420

},

...

}

-- ASN1STOP

TBS-TargetDeviceErrorCauses field descriptions

cause This field provides a TBS specific error cause.

#### 6.5.4.7	TBS Assistance Data

#### –	TBS-ProvideAssistanceData

The IE TBS-ProvideAssistanceData is used by the location server to provide assistance data to assist in position estimation at the UE (e.g. for UE-based mode) and/or to expedite the acquisition of TBS signals. It may also be used to provide TBS positioning specific error reasons.

-- ASN1START

TBS-ProvideAssistanceData-r14 ::= SEQUENCE {

tbs-AssistanceDataList-r14	TBS-AssistanceDataList-r14	OPTIONAL,	-- Need ON

tbs-Error-r14				TBS-Error-r13				OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

#### 6.5.4.8	TBS Assistance Data Elements

#### –	TBS-AssistanceDataList

The IE TBS-AssistanceDataList is used by the location server to provide the TBS specific assistance data to the UE.

-- ASN1START

TBS-AssistanceDataList-r14 ::= SEQUENCE {

mbs-AssistanceDataList-r14		MBS-AssistanceDataList-r14		OPTIONAL,	-- Need ON

...

}

MBS-AssistanceDataList-r14 ::= SEQUENCE (SIZE (1..maxMBS-r14)) OF MBS-AssistanceDataElement-r14

MBS-AssistanceDataElement-r14 ::= SEQUENCE {

mbs-AlmanacAssistance-r14		MBS-AlmanacAssistance-r14			OPTIONAL,	-- Need ON

mbs-AcquisitionAssistance-r14	MBS-AcquisitionAssistance-r14		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

#### –	MBS-AlmanacAssistance

The IE MBS-AlmanacAssistance is used by the location server to provide LLA of MBS transmitters to enable position estimation at the UE.

-- ASN1START

MBS-AlmanacAssistance-r14 ::= SEQUENCE {

transmitterID-r14			INTEGER (0..32767),

transmitterLatitude-r14		BIT STRING (SIZE (26)),

transmitterLongitude-r14	BIT STRING (SIZE (27)),

transmitterAltitude-r14		BIT STRING (SIZE (15)),

timeCorrection-r14			INTEGER (0..25)		OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

MBS-AlmanacAssistance field descriptions

transmitterID This field specifies the MBS transmitter ID [24].

transmitterLatitude This field specifies latitude of the MBS transmitter, degrees. Scale factor 4/220 decimal degrees, added to -90°. Valid range -90° to 90° [24].

transmitterLongitude This field specifies longitude of the MBS transmitter, degrees. Scale factor 4/220 decimal degrees, added to -180°. Valid range -180° to 180° [24].

transmitterAltitude This field specifies altitude of the MBS transmitter, metres. Scale factor 0.29 metres, added to -500 metres. Valid range -500 to 9002.43 metres [24].

timeCorrection This field contains the residual timing error for a particular beacon, in units of nano-seconds, in the range from 0 to 25. This field is used for UE-based mode only, by subtracting from the codePhase measurement made by the target device [24].

#### –	MBS-AcquisitionAssistance

The IE MBS-AcquisitionAssistance is used by the location server to provide parameters that support acquisition of the MBS signals [24].

-- ASN1START

MBS-AcquisitionAssistance-r14 ::= SEQUENCE {

transmitterID-r14				INTEGER (0..32767)						OPTIONAL,	-- Need ON

mbsConfiguration-r14			ENUMERATED {tb1, tb2, tb3, tb4, ...}	OPTIONAL,	-- Need ON

pnCodeIndex-r14					INTEGER (1..128)						OPTIONAL,	-- Need ON

freq-r14						INTEGER (919750000..927250000)			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

MBS-AcquisitionAssistance field descriptions

transmitterID This field contains the MBS transmitter identifier [24].

mbsConfiguration This field specifies MBS configuration as defined in the MBS ICD [24].

pnCodeIndex This field specifies the index of the MBS PN code [24].

freq This field specifies the MBS signal centre frequency in units of Hz [24].

#### 6.5.4.9	TBS Assistance Data Request

#### –	TBS-RequestAssistanceData

The IE TBS-RequestAssistanceData is used by the target device to request TBS assistance data from a location server.

-- ASN1START

TBS-RequestAssistanceData-r14 ::= SEQUENCE {

mbs-AlmanacAssistanceDataReq-r14		BOOLEAN,

mbs-AcquisitionAssistanceDataReq-r14	BOOLEAN,

...

}

-- ASN1STOP

### 6.5.5	Sensor based Positioning

#### 6.5.5.0	Introduction

This clause defines support for sensor-based positioning. The supported sensor methods are "Barometric pressure sensor" and "Motion sensor" as described in TS 36.305, clauses 8.6 and 8.10 respectively [2].

#### 6.5.5.1	Sensor Location Information

#### –	Sensor-ProvideLocationInformation

The IE Sensor-ProvideLocationInformation is used by the target device to provide location information for sensor-based methods to the location server. It may also be used to provide sensor specific error reason.

-- ASN1START

Sensor-ProvideLocationInformation-r13 ::= SEQUENCE {

sensor-MeasurementInformation-r13		Sensor-MeasurementInformation-r13		OPTIONAL,

sensor-Error-r13						Sensor-Error-r13						OPTIONAL,

...,

[[

sensor-MotionInformation-r15			Sensor-MotionInformation-r15			OPTIONAL

]]

}

-- ASN1STOP

#### 6.5.5.2	Sensor Location Information Elements

#### –	Sensor-MeasurementInformation

The IE Sensor-MeasurementInformation is used by the target device to provide UE sensor measurements to the location server.

-- ASN1START

Sensor-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13		UTCTime						OPTIONAL,

uncompensatedBarometricPressure-r13	INTEGER (30000..115000)		OPTIONAL, -- Cond Barometer

...,

[[

uncertainty-r14					SEQUENCE {

range-r14		INTEGER (0..1000),

confidence-r14	INTEGER (1..100)

}									OPTIONAL

]],

[[	adjustment-r16				INTEGER (-5000..5000) 				OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Barometer | The field is mandatory present if the Sensor-MeasurementInformation is provided for barometric pressure; otherwise it is not present. |

Sensor-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the sensor measurements are performed and should take the form of YYMMDDhhmmssZ.

uncompensatedBarometricPressure This field provides the uncompensated barometric pressure as measured by the UE sensor, in units of Pa.

uncertainty This field provides the expected range for the pressure measurement in units of Pa and the confidence as a percentage that the true pressure lies in a range of (measurement – range) to (measurement + range).

adjustment This field provides any adjustment available in the UE, in units of Pa, to allow the production of a compensated atmospheric pressure measurement where compensated atmospheric pressure = uncompensatedBarometricPressure + adjustment

#### –	Sensor-MotionInformation

The IE Sensor-MotionInformation is used by the target device to provide UE movement information to the location server. The movement information comprises an ordered series of points. This information may be obtained by the target device using one or more motion sensors.

-- ASN1START

Sensor-MotionInformation-r15 ::= SEQUENCE {

refTime-r15					DisplacementTimeStamp-r15,

displacementInfoList-r15	DisplacementInfoList-r15,

...

}

DisplacementInfoList-r15 ::= SEQUENCE (SIZE (1..128)) OF DisplacementInfoListElement-r15

DisplacementInfoListElement-r15 ::= SEQUENCE {

deltaTimeStamp-r15			DeltaTime-r15,

displacement-r15			Displacement-r15			OPTIONAL,

...

}

DisplacementTimeStamp-r15 ::= CHOICE {

utcTime-r15					UTC-Time-r15,

gnssTime-r15				MeasurementReferenceTime,

systemFrameNumber-r15		SFN-r15,

measurementSFN-r15			INTEGER(-8192..9214),

...

}

DeltaTime-r15 ::= CHOICE {

deltaTimeSec-r15			INTEGER (1..16384),

deltaTimeSFN-r15			INTEGER (1..4096),

...

}

SFN-r15 ::= SEQUENCE {

sfn-r15						BIT STRING (SIZE (10)),

hyperSFN-r15				BIT STRING (SIZE (10))		OPTIONAL,

...

}

Displacement-r15 ::= SEQUENCE {

bearing-r15					INTEGER (0..3599),

bearingUncConfidence-r15	INTEGER (0..100)				OPTIONAL,

bearingRef-r15				ENUMERATED { geographicNorth, magneticNorth, local },

horizontalDistance-r15		INTEGER (0..8191),

horizontalDistanceUnc-r15	INTEGER (0..255)				OPTIONAL,

horizontalUncConfidence-r15	INTEGER (0..100)				OPTIONAL,

verticalDirection-r15		ENUMERATED{upward, downward}	OPTIONAL,

verticalDistance-r15		INTEGER(0..8191)				OPTIONAL,

verticalDistanceUnc-r15		INTEGER (0..255)				OPTIONAL,

verticalUncConfidence-r15	INTEGER (0..100)				OPTIONAL,

...

}

UTC-Time-r15 ::= SEQUENCE {

utcTime-r15					UTCTime,

utcTime-ms-r15				INTEGER (0..999),

...

}

-- ASN1STOP

Sensor-MotionInformation field descriptions

refTime This field provides the reference time t0 associated with the starting position of the first displacement in the displacement list.

displacementInfoList This field provides an ordered series of direction and distance travelled by the target device and comprises the following subfields: - deltaTimeStamp specifies the time between tn-1 and tn, were n corresonds to the order of entry in the DispacementInfoList (n=0 correspond to the time provided in refTime). - displacement provides the direction and distance travelled between time tn-1 and tn.

utcTime This field provides the time stamp of the refTime in UTC time and comprises the following subfields: - utcTime in the form of YYMMDDhhmmssZ. - utcTime-ms specifies the fractional part of the UTC time in ms resolution.

gnssTime This field provides the time stamp of the refTime in GNSS time.

systemFrameNumber This field provides the time stamp of the refTime in serving cell SFN time.

measurementSFN This field provides the time stamp of the refTime in form of the measurement SFN as defined in deltaSFN in IE OTDOA-SignalMeasurementInformation. This field may be included when OTDOA measurements are included.

deltaTimeSec This field provides the time between tn-1 and tn in units of milliseconds.

deltaTimeSFN This field provides the time between tn-1 and tn in units of system frame numbers.

bearing This field specifies the direction (heading) of the horizontal displacement measured clockwise from bearingRef. Scale factor 0.1 degree.

bearingRef This field specifies the reference direction for the bearing. Enumerated value 'geographicNorth' indicates that the bearing is measured clockwise from the Geographic North; 'magneticNorth' indicates that the bearing is measured clockwise from the Magnetic North; 'local' indicates that the bearing is measured clockwise from an arbitrary (undefined) reference direction.

horizontalDistance This field specifies the horizonal distance travelled between time tn-1 and tn. Scale factor 1 cm.

horizontalDistanceUnc, horizontalUncConfidence This field specifies the horizontal uncertainty of the displacement (corresponding to tn). horizontalDistanceUnc correspond to the encoded high accuracy uncertainty as defined in TS 23.032 [15]. horizontalUncConfidence corresponds to confidence as defined in TS 23.032 [15].

verticalDistance This field specifies the vertical distance travelled between time tn-1 and tn. Scale factor 1 cm.

verticalDistanceUnc, verticalUncConfidence This field specifies the vertical uncertainty of the displacement (corresponding to tn). verticalDistanceUnc correspond to the encoded high accuracy uncertainty as defined in TS 23.032 [15]. verticalUncConfidence corresponds to confidence as defined in TS 23.032 [15].

#### 6.5.5.3	Sensor Location Information Request

#### –	Sensor-RequestLocationInformation

The IE Sensor-RequestLocationInformation is used by the location server to request location information for sensor-based methods from a target device.

-- ASN1START

Sensor-RequestLocationInformation-r13 ::= SEQUENCE {

uncompensatedBarometricPressureReq-r13		BOOLEAN,

...,

[[	assistanceAvailability-r14				BOOLEAN		OPTIONAL	-- Need ON

]],

[[	sensor-MotionInformationReq-r15			BOOLEAN		OPTIONAL	-- Need ON

]],

[[	adjustmentReq-r16						BOOLEAN		OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

Sensor-RequestLocationInformation field descriptions

uncompensatedBarometricPressureReq This field indicates whether the target device is requested to report Barometric pressure measurements in Sensor-MeasurementInformation IE or not. TRUE means requested.

assistanceAvailability This field indicates whether the target device may request additional Sensor assistance data from the server. TRUE means allowed and FALSE means not allowed.

sensor-MotionInformationReq This field indicates whether the target device is requested to report movement information in IE Sensor-MotionInformation or not. TRUE means requested.

adjustmentReq This field indicates whether the target device is requested to report adjustment in IE Sensor-MeasurementInformation or not. TRUE means requested.

#### 6.5.5.4	Sensor Capability Information

#### –	Sensor-ProvideCapabilities

The IE Sensor-ProvideCapabilities is used by the target device to provide capabilities for sensor-based methods from to the location server.

-- ASN1START

Sensor-ProvideCapabilities-r13 ::= SEQUENCE {

sensor-Modes-r13			BIT STRING {	standalone	(0),

ue-assisted	(1),

ue-based	(2)} (SIZE (1..8)),

...,

[[	sensor-AssistanceDataSupportList-r14	Sensor-AssistanceDataSupportList-r14	OPTIONAL,

periodicalReportingSupported-r14		PositioningModes						OPTIONAL,

idleStateForMeasurements-r14			ENUMERATED { required }					OPTIONAL

]],

[[	sensor-MotionInformationSup-r15			ENUMERATED { true }						OPTIONAL

]],

[[	adjustmentSupported-r16					ENUMERATED	{ true }					OPTIONAL

]],

[[	scheduledLocationRequest-r17			SEQUENCE {

utcTime-r17			PositioningModes		OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}										OPTIONAL,

e-utraTime-r17		PositioningModes		OPTIONAL,

nrTime-r17			PositioningModes		OPTIONAL,

relativeTime-r17	PositioningModes		OPTIONAL,

...

}											OPTIONAL

]]

}

Sensor-AssistanceDataSupportList-r14 ::= SEQUENCE {

...,

[[	validityPeriodSupported-v1520			ENUMERATED { true }		OPTIONAL,

validityAreaSupported-v1520				ENUMERATED { true }		OPTIONAL

]]

}

-- ASN1STOP

Sensor-ProvideCapabilities field descriptions

sensor-Modes This field specifies the sensor mode(s) supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular sensor mode is supported; a zero-value means not supported.

sensor-AssistanceDataSupportList This field specifies a list of sensor assistance data supported by the target device. This field shall be present if the target device supports assistance data for Barometric pressure sensor.

validityPeriodSupported This field, if present, indicates that the target device supports period i.e. pressure validity period and pressure rate as part of the Sensor-AssistanceDataList.

valitidyAreaSupported This field, if present, indicates that the target device supports area i.e. pressure validity area and North/East pressure gradient as part of the Sensor-AssistanceDataList.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform sensor measurements.

sensor-MotionInformationSup This field, if present, indicates that the target device supports displacement reporting in IE Sensor-MotionInformation.

adjustmentSupported This field, if present, indicates that the target device supports the adjustment IE in Sensor-MeasurementInformation.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

#### 6.5.5.5	Sensor Capability Information Request

#### –	Sensor-RequestCapabilities

The IE Sensor-RequestCapabilities is used by the location server to request capabilities for sensor-based methods from the target device.

-- ASN1START

Sensor-RequestCapabilities-r13 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.5.6	Sensor Error Elements

#### –	Sensor-Error

The IE Sensor-Error is used by the location server or target device to provide Sensor Error Reasons to the target device or location server, respectively.

-- ASN1START

Sensor-Error-r13 ::= CHOICE {

locationServerErrorCauses-r13		Sensor-LocationServerErrorCauses-r13,

targetDeviceErrorCauses-r13			Sensor-TargetDeviceErrorCauses-r13,

...

}

-- ASN1STOP

#### –	Sensor-LocationServerErrorCauses

The IE Sensor-LocationServerErrorCauses is used by the location server to provide error reasons for Sensor positioning to the target device.

-- ASN1START

Sensor-LocationServerErrorCauses-r13 ::= SEQUENCE {

cause-r13		ENUMERATED	{	undefined,

...,

assistanceDataNotSupportedByServer-v1420,

assistanceDataSupportedButCurrentlyNotAvailableByServer-v1420

},

...

}

-- ASN1STOP

#### –	Sensor-TargetDeviceErrorCauses

The IE Sensor-TargetDeviceErrorCauses is used by the target device to provide error reasons for Sensor positioning to the location server.

-- ASN1START

Sensor-TargetDeviceErrorCauses-r13 ::= SEQUENCE {

cause-r13			ENUMERATED		{	undefined,

...,

assistanceDataMissing-v1420

},

...

}

-- ASN1STOP

#### 6.5.5.7	Sensor Assistance Data

#### –	Sensor-ProvideAssistanceData

The IE Sensor-ProvideAssistanceData is used by the location server to provide assistance data to assist in altitude computation at the UE (e.g. for UE-based mode). It may also be used to provide Sensor positioning specific error reasons.

-- ASN1START

Sensor-ProvideAssistanceData-r14 ::= SEQUENCE {

sensor-AssistanceDataList-r14		Sensor-AssistanceDataList-r14		OPTIONAL,	-- Need ON

sensor-Error-r14					Sensor-Error-r13					OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

#### 6.5.5.8	Sensor Assistance Data Elements

#### –	Sensor-AssistanceDataList

The IE Sensor-AssistanceDataList is used by the location server to provide the Sensor specific assistance data to the UE.

-- ASN1START

Sensor-AssistanceDataList-r14::= SEQUENCE {

refPressure-r14		INTEGER (-20000..10000),

refPosition-r14		EllipsoidPointWithAltitudeAndUncertaintyEllipsoid	OPTIONAL,	-- Need ON

refTemperature-r14	INTEGER (-64..63)									OPTIONAL,	-- Need ON

...,

[[

period-v1520		SEQUENCE {

pressureValidityPeriod-v1520	PressureValidityPeriod-v1520,

referencePressureRate-v1520		INTEGER	(-128..127)					OPTIONAL,	-- Need ON

...

}																		OPTIONAL,	-- Need ON

area-v1520			SEQUENCE {

pressureValidityArea-v1520		PressureValidityArea-v1520,

gN-pressure-v1520				INTEGER (-1024..1023)				OPTIONAL,	-- Need ON

gE-pressure-v1520				INTEGER (-1024..1023)				OPTIONAL,	-- Need ON

...

}																		OPTIONAL	-- Need ON

]]

}

PressureValidityArea-v1520 ::= SEQUENCE {

centerPoint-v1520				Ellipsoid-Point,

validityAreaWidth-v1520			INTEGER (1..128),

validityAreaHeight-v1520		INTEGER (1..128),

...

}

PressureValidityPeriod-v1520 ::= SEQUENCE {

beginTime-v1520					GNSS-SystemTime,

beginTimeAlt-v1520				INTEGER (0..2881)						OPTIONAL,	-- Need ON

duration-v1520					INTEGER (1..2881),

...

}

-- ASN1STOP

Sensor-AssistanceDataList field descriptions

refPressure This field specifies the atmospheric pressure (Pa) nominal at sea level, EGM96 [29] to the target. The scale factor is 1 Pa. The value is added to the nominal pressure of 101325 Pa.

refPosition This field specifies the reference position at which the pressure measurement is made, as an ellipsoid point with altitude and uncertainty ellipsoid.

refTemperature Local temperature measurement at the reference where the pressure measurement is made. The scale factor 1K. The value is added to 273K.

period This field specifies the pressure validity period and reference pressure rate.

pressureValidityPeriod - beginTime: this field specifies the start time of the pressure validity period in GNSS System Time. - beginTimeAlt: this field specifies an alternative start time. It may be used by the target device if GNSS-System Time is not available. The alternative start time is relative to the time the message was received. The scale factor is 15 min. The range is from 0 minutes to 43215 minutes = 30 days. - duration: this field specifies the duration of the validity period after the begin time. The scale factor is 15 minutes. The range is from 15 minutes to 43215 minutes = 30 days.

referencePressureRate This field specifies the rate of change of pressure. When this field is included, the reference pressure applies only at the start of the pressure validity period. The scale factor is 10Pa/hour.

area This field specifies the area within which the provided atmospheric reference pressure is valid and any spatial drift.

pressureValidityArea - centerPoint: this field specifies the coordinates of the centre of the rectangular validity area. - validityAreaWidth: this field specifies the width of the rectangular validity area. Width is measured from the centre along the latitude and is measured as the total width of the rectangle. The scale factor is 1km. The range is from 1km to 128km. - validityAreaHeight: this fields specifies the height of the rectangular validity area. Height is measured from the centre along the longitude and is measured as the total height of the rectangle. The scale factor is 1km. The range is from 1km to 128km. If this field is present, refPosition should not be provided by the location server and if provided, shall be ignored by the target device.

gN-pressure This field specifies the northward gradient of the reference pressure calculated from the centre of the pressureValidityArea. The scale factor is 1 Pa/km. If this field is not provided, the gradient is assumed to be zero.

gE-pressure This field specifies the eastward gradient of the reference pressure calculated from the centre of the pressureValidityArea. The scale factor is 1 Pa/km. If this field is not provided, the gradient is assumed to be zero.

#### 6.5.5.9	Sensor Assistance Data Request

#### –	Sensor-RequestAssistanceData

The IE Sensor-RequestAssistanceData is used by the target device to request Sensor assistance data from a location server.

-- ASN1START

Sensor-RequestAssistanceData-r14 ::= SEQUENCE {

...

}

-- ASN1STOP

### 6.5.6	WLAN-based Positioning

This clause defines support for positioning using measurements related to WLAN access points.

#### 6.5.6.1	WLAN Location Information

#### –	WLAN-ProvideLocationInformation

The IE WLAN-ProvideLocationInformation is used by the target device to provide measurements for one or more WLANs to the location server. It may also be used to provide WLAN positioning specific error reason.

-- ASN1START

WLAN-ProvideLocationInformation-r13 ::= SEQUENCE {

wlan-MeasurementInformation-r13		WLAN-MeasurementInformation-r13		OPTIONAL,

wlan-Error-r13						WLAN-Error-r13						OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.6.2	WLAN Location Information Elements

#### –	WLAN-MeasurementInformation

-- ASN1START

WLAN-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13		UTCTime						OPTIONAL,

wlan-MeasurementList-r13			WLAN-MeasurementList-r13	OPTIONAL,

...

}

WLAN-MeasurementList-r13 ::= SEQUENCE (SIZE(1..maxWLAN-AP-r13)) OF WLAN-MeasurementElement-r13

WLAN-MeasurementElement-r13 ::= SEQUENCE {

wlan-AP-Identifier-r13		WLAN-AP-Identifier-r13,

rssi-r13					INTEGER (-127..128)					OPTIONAL,

rtt-r13						WLAN-RTT-r13						OPTIONAL,

apChannelFrequency-r13		INTEGER (0..256)					OPTIONAL,

servingFlag-r13				BOOLEAN								OPTIONAL,

...

}

WLAN-AP-Identifier-r13 ::= SEQUENCE {

bssid-r13					OCTET STRING (SIZE (6)),

ssid-r13					OCTET STRING (SIZE (1..32))			OPTIONAL,

...

}

WLAN-RTT-r13 ::= SEQUENCE {

rttValue-r13	INTEGER (0..16777215),

rttUnits-r13	ENUMERATED {	microseconds,

hundredsofnanoseconds,

tensofnanoseconds,

nanoseconds,

tenthsofnanoseconds,

... },

rttAccuracy-r13	INTEGER (0..255)								OPTIONAL,

...

}

-- ASN1STOP

WLAN-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the WLAN measurements are performed and should take the form of YYMMDDhhmmssZ.

wlan-MeasurementList This field provides the WLAN measurements for up to 64 WLAN APs.

wlan-AP-Identifier This field provides the BSSID and optionally the SSID of the wireless network served by the WLAN AP [26].

rssi This field provides the AP signal strength (RSSI) of a beacon frame, probe response frame or measurement pilot frame measured at the target in dBm as defined in Table 6-7 of [26].

rtt This field provides the measured round trip time between the target device and WLAN AP and optionally the accuracy expressed as the standard deviation of the delay. Units for each of these are 1000ns, 100ns, 10ns, 1ns, and 0.1ns.

apChannelFrequency This field provides the AP channel number identification of the reported WLAN AP.

servingFlag This parameter indicates whether a set of WLAN AP measurements were obtained for a serving WLAN AP (TRUE) or a non-serving WLAN AP (FALSE). A target device with multiple radio support may indicate more than one type of serving access for the same time instant.

rttValue This field specifies the Round Trip Time (RTT) measurement between the target device and WLAN AP in units given by the field rttUnits.

rttUnits This field specifies the Units for the fields rttValue and rttAccuracy. The available Units are 1000ns, 100ns, 10ns, 1ns, and 0.1ns.

rttAccuracy This field provides the estimated accuracy of the provided rttValue expressed as the standard deviation in units given by the field rttUnits.

#### 6.5.6.3	WLAN Location Information Request

#### –	WLAN-RequestLocationInformation

The IE WLAN-RequestLocationInformation is used by the location server to request WLAN measurements from a target device.

-- ASN1START

WLAN-RequestLocationInformation-r13 ::= SEQUENCE {

requestedMeasurements-r13	BIT STRING {

rssi		(0),

rtt			(1)} (SIZE(1..8)),

...,

[[	assistanceAvailability-r14	BOOLEAN			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

WLAN-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the WLAN measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. rssi: AP signal strength at the target rtt: Round Trip Time between target and AP

assistanceAvailability This field indicates whether the target device may request additional WLAN assistance data from the server. TRUE means allowed and FALSE means not allowed.

#### 6.5.6.4	WLAN Capability Information

#### –	WLAN-ProvideCapabilities

The IE WLAN-ProvideCapabilites is used by the target device to provide its capabilities for WLAN positioning to the location server.

-- ASN1START

WLAN-ProvideCapabilities-r13 ::= SEQUENCE {

wlan-Modes-r13			BIT STRING	{	standalone		(0),

ue-assisted		(1),

ue-based		(2)}	(SIZE (1..8)),

wlan-MeasSupported-r13	BIT STRING	{

rssi-r13		(0),

rtt-r13			(1)}	(SIZE(1..8)),

...	,

[[	wlan-AP-AD-Supported-r14

BIT STRING {	ap-identifier	(0),

ap-location		(1)}	(SIZE (1..8))

OPTIONAL,

periodicalReportingSupported-r14	PositioningModes						OPTIONAL,

idleStateForMeasurements-r14

ENUMERATED {	required	}							OPTIONAL

]],

[[	scheduledLocationRequest-r17		SEQUENCE {

utcTime-r17			PositioningModes		OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}										OPTIONAL,

e-utraTime-r17		PositioningModes		OPTIONAL,

nrTime-r17			PositioningModes		OPTIONAL,

relativeTime-r17	PositioningModes		OPTIONAL,

...

}											OPTIONAL

]]

}

-- ASN1STOP

WLAN-ProvideCapabilities field descriptions

wlan-Modes This field specifies the WLAN mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the WLAN mode is supported; a zero value means not supported.

wlan-MeasSupported This field specifies the measurements supported by the target device when accessing a WLAN. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic WLAN positioning method is supported by the target device which is reporting of the WLAN identity. The following bits are assigned for the indicated measurements. rssi: AP signal strength at the target rtt: Round Trip Time between target and AP

wlan-AP-AD-Supported This field specifies the WLAN AP assistance data supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. A zero-value in all bit positions or absence of this field means no assistance data is supported. The following bits are assigned for the indicated assistance data. ap-identifier: WLAN AP identity information ap-location: WLAN AP location information

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform WLAN measurements.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

#### 6.5.6.5	WLAN Capability Information Request

#### –	WLAN-RequestCapabilities

The IE WLAN-RequestCapabilities is used by the location server to request WLAN positioning capabilities information from a target device.

-- ASN1START

WLAN-RequestCapabilities-r13 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.6.6	WLAN Error Elements

#### –	WLAN-Error

The IE WLAN-Error is used by the location server or target device to provide error reasons for WLAN positioning to the target device or location server, respectively.

-- ASN1START

WLAN-Error-r13 ::= CHOICE {

locationServerErrorCauses-r13		WLAN-LocationServerErrorCauses-r13,

targetDeviceErrorCauses-r13			WLAN-TargetDeviceErrorCauses-r13,

...

}

-- ASN1STOP

#### –	WLAN-LocationServerErrorCauses

The IE WLAN-LocationServerErrorCauses is used by the location server to provide error reasons for WLAN positioning to the target device.

-- ASN1START

WLAN-LocationServerErrorCauses-r13 ::= SEQUENCE {

cause-r13								ENUMERATED	{undefined,

...,

requestedADNotAvailable-v1420,

notAllrequestedADAvailable-v1420

},

...,

[[	apLocationDataUnavailable-r14		NULL	OPTIONAL		-- Need ON

]]

}

-- ASN1STOP

WLAN-LocationServerErrorCauses field descriptions

cause This field provides a WLAN AP specific error cause for the server applicable to provision of assistance data. If the cause value is 'requestedADNotAvailable', none of the requested assistance data could be provided and no further information needs to be included. If the cause value is 'notAllRequestedADAvailable', the server was able to provide some but not all requested WLAN AP assistance data. In this case, the server should include any of the specific error indications as applicable. Note that inclusion of these fields is applicable when some of the associated information can be provided for some WLAN APs but not for all WLAN APs.

#### –	WLAN-TargetDeviceErrorCauses

The IE WLAN-TargetDeviceErrorCauses is used by the target device to provide error reasons for WLAN positioning to the location server.

-- ASN1START

WLAN-TargetDeviceErrorCauses-r13 ::= SEQUENCE {

cause-r13								ENUMERATED {undefined,

requestedMeasurementsNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

wlan-AP-RSSI-MeasurementNotPossible-r13				NULL		OPTIONAL,

wlan-AP-RTT-MeasurementNotPossible-r13				NULL		OPTIONAL,

...

}

-- ASN1STOP

WLAN-TargetDeviceErrorCauses field descriptions

cause This field provides a WLAN specific error cause. If the cause value is 'notAllRequestedMeasurementsPossible', the target device was not able to provide all requested WLAN measurements (but may be able to provide some measurements). In this case, the target device should include any of the wlan-AP-RSSI-MeasurementNotPossible, or wlan-AP-RTT-MeasurementNotPossible fields, as applicable.

#### 6.5.6.7	WLAN Assistance Data

#### –	WLAN-ProvideAssistanceData

The IE WLAN-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-based and UE-assisted WLAN positioning. It may also be used to provide WLAN positioning specific error reason.

-- ASN1START

WLAN-ProvideAssistanceData-r14 ::= SEQUENCE {

wlan-DataSet-r14	SEQUENCE (SIZE (1..maxWLAN-DataSets-r14)) OF WLAN-DataSet-r14

OPTIONAL,	-- Need ON

wlan-Error-r14		WLAN-Error-r13										OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

WLAN-ProvideAssistanceData field descriptions

wlan-DataSet This field provides data for sets of WLAN APs.

wlan-Error This field provides error information and may be included when a Provide Assistance Data is sent in response to a Request Assistance Data. It is allowed to include both a wlan-DataSet field and a wlan-Error field (e.g. when only some requested WLAN assistance data is provided).

#### 6.5.6.8	WLAN Assistance Data Elements

#### –	WLAN-DataSet

The IE WLAN-DataSet is used by the location server to provide WLAN AP information for one set of WLAN APs.

-- ASN1START

WLAN-DataSet-r14 ::= SEQUENCE {

wlan-AP-List-r14				SEQUENCE (SIZE (1..maxWLAN-AP-r14)) OF WLAN-AP-Data-r14,

supportedChannels-11a-r14		SupportedChannels-11a-r14		OPTIONAL,	-- Need ON

supportedChannels-11bg-r14		SupportedChannels-11bg-r14		OPTIONAL,	-- Need ON

...

}

SupportedChannels-11a-r14 ::= SEQUENCE {

ch34-r14		BOOLEAN,

ch36-r14		BOOLEAN,

ch38-r14		BOOLEAN,

ch40-r14		BOOLEAN,

ch42-r14		BOOLEAN,

ch44-r14		BOOLEAN,

ch46-r14		BOOLEAN,

ch48-r14		BOOLEAN,

ch52-r14		BOOLEAN,

ch56-r14		BOOLEAN,

ch60-r14		BOOLEAN,

ch64-r14		BOOLEAN,

ch149-r14		BOOLEAN,

ch153-r14		BOOLEAN,

ch157-r14		BOOLEAN,

ch161-r14		BOOLEAN

}

SupportedChannels-11bg-r14 ::= SEQUENCE {

ch1-r14			BOOLEAN,

ch2-r14			BOOLEAN,

ch3-r14			BOOLEAN,

ch4-r14			BOOLEAN,

ch5-r14			BOOLEAN,

ch6-r14			BOOLEAN,

ch7-r14			BOOLEAN,

ch8-r14			BOOLEAN,

ch9-r14			BOOLEAN,

ch10-r14		BOOLEAN,

ch11-r14		BOOLEAN,

ch12-r14		BOOLEAN,

ch13-r14		BOOLEAN,

ch14-r14		BOOLEAN

}

-- ASN1STOP

WLAN-DataSet field descriptions

wlan-AP-List This field provides information for WLAN APs in the data set.

supportedChannels-11a This field defines the superset of all channels supported by all WLAN APs in the data set of type 801.11a (5GHz band).

supportedChannels-11bg This field defines the superset of all channels supported by all WLAN APs in the data set of type 801.11b or 802.11g (2.4 GHz band).

#### –	WLAN-AP-Data

The IE WLAN-AP-Data is used by the location server to provide information for one WLAN AP as part of WLAN AP assistance data.

-- ASN1START

WLAN-AP-Data-r14 ::= SEQUENCE {

wlan-AP-Identifier-r14				WLAN-AP-Identifier-r13,

wlan-AP-Location-r14				WLAN-AP-Location-r14		OPTIONAL,	-- Need ON

...

}

WLAN-AP-Location-r14 ::= SEQUENCE {

locationDataLCI-r14					LocationDataLCI-r14,

...

}

LocationDataLCI-r14 ::= SEQUENCE {

latitudeUncertainty-r14				BIT STRING (SIZE (6)),

latitude-r14						BIT STRING (SIZE (34)),

longitudeUncertainty-r14			BIT STRING (SIZE (6)),

longitude-r14						BIT STRING (SIZE (34)),

altitudeUncertainty-r14				BIT STRING (SIZE (6))		OPTIONAL,	-- Need ON

altitude-r14						BIT STRING (SIZE (30))		OPTIONAL,	-- Need ON

datum-r14							BIT STRING (SIZE (8)),

...

}

-- ASN1STOP

WLAN-AP-Data field descriptions

wlan-AP-Location - locationDataLCI This field provides the location of the WLAN AP in the form of Location Configuration Information (LCI) defined in [27] and includes the following subfields: latitudeUncertainty: 6-bits quantifying the amount of uncertainty in latitude. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 34 are reserved. Its relation with the corresponding value in degrees is expressed with the following formula: latitudeUncertainty = 8 - ceil(log2(uncertainty in degrees)) latitude: A 34-bits fixed point value consisting of 9-bits of integer and 25-bits of fraction indicating the Latitude (+/- 90 degrees) of the AP. longitudeUncertainty: 6-bits quantifying the amount of uncertainty in longitude. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 34 are reserved. Its relation with the corresponding value in degrees is expressed with the following formula: longitudeUncertainty = 8 - ceil(log2(uncertainty in degrees)) longitude: A 34-bits fixed point value consisting of 9-bits of integer and 25-bits of fraction indicating the Longitude (+/- 180 degrees) of the AP. altitudeUncertainty: 6-bits value quantifying the amount of uncertainty in the altitude value. A value of 0 is reserved to indicate that the uncertainty is unknown; values greater than 30 are reserved. Its relation with the corresponding value in metres is expressed with the following formula: altitudeUncertainty = 21 - ceil(log2( uncertainty in metres)) altitude: A 30-bit fixed point value consisting of 22-bits of integer and 8-bits of fraction indicating the altitude of the AP in metres. datum: 8-bits indicating the map datum used for the coordinates. Defined codes are: Bit 1: World Geodetic System 1984 (WGS-84) Bit 2: North American Datum 1983 (NAD-83) with North American Vertical Datum 1988 (NAVD-88) Bit 3: North American Datum 1983 (NAD-83) with Mean Lower Low Water (MLLW) vertical datum. Bits 4 – 8 are reserved.

#### 6.5.6.9	WLAN Assistance Data Request

#### –	WLAN-RequestAssistanceData

The IE WLAN-RequestAssistanceData is used by the target device to request WLAN assistance data from a location server.

-- ASN1START

WLAN-RequestAssistanceData-r14 ::= SEQUENCE {

requestedAD-r14			BIT STRING {	ap-identifier		(0),

ap-location			(1)}	(SIZE (1..8)),

visibleAPs-r14			SEQUENCE (SIZE (1..maxVisibleAPs-r14)) OF WLAN-AP-Identifier-r13	OPTIONAL,

wlan-AP-StoredData-r14	SEQUENCE (SIZE (1..maxKnownAPs-r14)) OF WLAN-AP-Identifier-r13	OPTIONAL,

...

}

-- ASN1STOP

WLAN-RequestAssistanceData field descriptions

requestedAD This field specifies the WLAN AP assistance data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. The following assistance data types are included: ap-identifier: WLAN AP identity information ap-location: WLAN AP location information

visibleAPs This field enables a target to indicate to a server the identities of currently visible WLAN APs. This may assist a server to provide assistance data for WLAN APs nearby to the target. A target shall provide visible APs in order of received signal strength with the AP with the highest signal strength provided first.

wlan-AP-StoredData This field enables a target to indicate to a server the identities of WLAN APs for which the target has stored assistance data received previously from the server. This may enable the server to avoid resending data for the same APs.

### 6.5.7	Bluetooth-based Positioning

#### 6.5.7.1	Bluetooth Location Information

#### –	BT-ProvideLocationInformation

The IE BT-ProvideLocationInformation is used by the target device to provide measurements for one or more Bluetooth beacons to the location server. It may also be used to provide Bluetooth positioning specific error reason.

-- ASN1START

BT-ProvideLocationInformation-r13 ::= SEQUENCE {

bt-MeasurementInformation-r13		BT-MeasurementInformation-r13	OPTIONAL,

bt-Error-r13						BT-Error-r13					OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.7.2	Bluetooth Location Information Elements

#### –	BT-MeasurementInformation

-- ASN1START

BT-MeasurementInformation-r13 ::= SEQUENCE {

measurementReferenceTime-r13		UTCTime						OPTIONAL,

bt-MeasurementList-r13				BT-MeasurementList-r13		OPTIONAL,

...

}

BT-MeasurementList-r13 ::= SEQUENCE (SIZE(1..maxBT-Beacon-r13)) OF BT-MeasurementElement-r13

BT-MeasurementElement-r13 ::= SEQUENCE {

btAddr-r13						BIT STRING (SIZE (48)),

rssi-r13						INTEGER (-128..127)				OPTIONAL,

...

}

-- ASN1STOP

BT-MeasurementInformation field descriptions

measurementReferenceTime This field provides the UTC time when the Bluetooth measurements are performed and should take the form of YYMMDDhhmmssZ.

bt-MeasurementList This field provides the Bluetooth measurements for up to 32 Bluetooth beacons.

btAddr This field specifies the Bluetooth public address of the Bluetooth beacon [25].

rssi This field provides the beacon received signal strength indicator (RSSI) in dBm.

#### 6.5.7.3	Bluetooth Location Information Request

#### –	BT-RequestLocationInformation

The IE BT-RequestLocationInformation is used by the location server to request Bluetooth measurements from a target device.

-- ASN1START

BT-RequestLocationInformation-r13 ::= SEQUENCE {

requestedMeasurements-r13	BIT STRING {

rssi		(0)} (SIZE(1..8)),

...

}

-- ASN1STOP

BT-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the Bluetooth measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested. The following measurement requests can be included. rssi: Bluetooth beacon signal strength at the target

#### 6.5.7.4	Bluetooth Capability Information

#### –	BT-ProvideCapabilities

The IE BT-ProvideCapabilites is used by the target device to provide its capabilities for Bluetooth positioning to the location server.

-- ASN1START

BT-ProvideCapabilities-r13 ::= SEQUENCE {

bt-Modes-r13			BIT STRING {	standalone		(0),

ue-assisted		(1)}	(SIZE (1..8)),

bt-MeasSupported-r13	BIT STRING {	rssi-r13		(0)}	(SIZE (1..8)),

...,

[[

idleStateForMeasurements-r14

ENUMERATED {	required	}							OPTIONAL,

periodicalReportingSupported-r14

PositioningModes										OPTIONAL

]],

[[	scheduledLocationRequest-r17	SEQUENCE {

utcTime-r17			PositioningModes					OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}												OPTIONAL,

e-utraTime-r17		PositioningModes					OPTIONAL,

nrTime-r17			PositioningModes					OPTIONAL,

relativeTime-r17	PositioningModes					OPTIONAL,

...

}														OPTIONAL

]]

}

-- ASN1STOP

BT-ProvideCapabilities field descriptions

bt-Modes This field specifies the Bluetooth mode(s) supported by the target device. This is represented by a bit string, with a one value at the bit position means the Bluetooth mode is supported; a zero value means not supported.

bt-MeasSupported This field specifies the Bluetooth measurements supported by the target device. This is represented by a bit string, with a one-value at the bit position means the particular measurement is supported; a zero-value means not supported. A zero-value in all bit positions in the bit string means only the basic Bluetooth positioning method is supported by the target device which is reporting of the Bluetooth beacon identity. The following bits are assigned for the indicated measurements. rssi: Bluetooth beacon signal strength at the target device

idleStateForMeasurements This field, if present, indicates that the target device requires idle state to perform BT measurements.

periodicalReportingSupported This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one value at the bit position means periodicalReporting for the positioning mode is supported; a zero value means not supported. If this field is absent, the location server may assume that the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

#### 6.5.7.5	Bluetooth Capability Information Request

#### –	BT-RequestCapabilities

The IE BT-RequestCapabilities is used by the location server to request Bluetooth positioning capabilities from a target device.

-- ASN1START

BT-RequestCapabilities-r13 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.7.6	BT Error Elements

#### –	BT-Error

The IE BT-Error is used by the location server or target device to provide error reasons for Bluetooth positioning to the target device or location server, respectively.

-- ASN1START

BT-Error-r13 ::= CHOICE {

locationServerErrorCauses-r13		BT-LocationServerErrorCauses-r13,

targetDeviceErrorCauses-r13			BT-TargetDeviceErrorCauses-r13,

...

}

-- ASN1STOP

#### –	BT-LocationServerErrorCauses

The IE BT-LocationServerErrorCauses is used by the location server to provide error reasons for Bluetooth positioning to the target device.

-- ASN1START

BT-LocationServerErrorCauses-r13 ::= SEQUENCE {

cause-r13								ENUMERATED	{undefined,	...},

...

}

-- ASN1STOP

#### –	BT-TargetDeviceErrorCauses

The IE BT-TargetDeviceErrorCauses is used by the target device to provide error reasons for Bluetooth positioning to the location server.

-- ASN1START

BT-TargetDeviceErrorCauses-r13 ::= SEQUENCE {

cause-r13								ENUMERATED {undefined,

requestedMeasurementsNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

bt-Beacon-rssiMeasurementNotPossible-r13	NULL		OPTIONAL,

...

}

-- ASN1STOP

BT-TargetDeviceErrorCauses field descriptions

cause This field provides a Bluetooth specific error cause. If the cause value is 'notAllRequestedMeasurementsPossible', the target device was not able to provide all requested Bluetooth measurements (but may be able to provide some measurements). In this case, the target device should include bt-Beacon-rssiMeasurementNotPossible field.

### 6.5.8	NR UL Positioning

#### 6.5.8.1	NR UL Capability Information

#### –	NR-UL-ProvideCapabilities

The IE NR-UL-ProvideCapabilities is used by the target device to indicate its capability to support UL-PRS and to provide its UL-PRS capabilities to the location server.

-- ASN1START

NR-UL-ProvideCapabilities-r16 ::= SEQUENCE {

nr-UL-SRS-Capability-r16		NR-UL-SRS-Capability-r16,

...,

[[

nr-UE-TxTEG-ID-CapabilityBandList-r17	SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

NR-UE-TxTEG-ID-CapabilityPerBand-r17	OPTIONAL

]]

}

NR-UE-TxTEG-ID-CapabilityPerBand-r17 ::= SEQUENCE {

freqBandIndicatorNR-r17				FreqBandIndicatorNR-r16,

nr-UE-TxTEG-ID-MaxSupport-r17		ENUMERATED {n1, n2, n3, n4, n6, n8}				OPTIONAL,

...

}

-- ASN1STOP

NR-UL-ProvideCapabilities field descriptions

nr-UE-TxTEG-ID-MaxSupport Indicates the maximum number of UE-TxTEG for SRS Resource for Positioning, which is supported and reported by the UE for UL-TDOA.

#### 6.5.8.2	NR UL Capability Information Request

#### –	NR-UL-RequestCapabilities

The IE NR-UL-RequestCapabilities is used by the location server to request the capability of the target device to support UL-PRS and to request UL-PRS capabilities from a target device.

-- ASN1START

NR-UL-RequestCapabilities-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

### 6.5.9	NR E-CID Positioning

This clause defines the information elements for NR E-CID positioning (TS 38.305 [40]).

#### 6.5.9.1	NR E-CID Location Information

#### –	NR-ECID-ProvideLocationInformation

The IE NR-ECID-ProvideLocationInformation is used by the target device to provide NR E-CID location measurements to the location server. It may also be used to provide NR E-CID positioning specific error reason.

-- ASN1START

NR-ECID-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-ECID-SignalMeasurementInformation-r16	NR-ECID-SignalMeasurementInformation-r16 OPTIONAL,

nr-ECID-Error-r16							NR-ECID-Error-r16						 OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.9.2	NR E-CID Location Information Elements

#### –	NR-ECID-SignalMeasurementInformation

The IE NR-ECID-SignalMeasurementInformation is used by the target device to provide NR E-CID measurements to the location server.

-- ASN1START

NR-ECID-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-PrimaryCellMeasuredResults-r16	NR-MeasuredResultsElement-r16,

nr-MeasuredResultsList-r16			NR-MeasuredResultsList-r16				OPTIONAL,

...

}

NR-MeasuredResultsList-r16 ::= SEQUENCE (SIZE(1..32)) OF NR-MeasuredResultsElement-r16

NR-MeasuredResultsElement-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16,

nr-ARFCN-r16					CHOICE {

ssb-ARFCN-r16					ARFCN-ValueNR-r15,

csi-RS-pointA-r16				ARFCN-ValueNR-r15

},

nr-CellGlobalID-r16				NCGI-r15									OPTIONAL,

systemFrameNumber-r16			BIT STRING (SIZE (10))						OPTIONAL,	resultsSSB-Cell-r16				MeasQuantityResults-r16						OPTIONAL,

resultsCSI-RS-Cell-r16			MeasQuantityResults-r16						OPTIONAL,

resultsSSB-Indexes-r16			ResultsPerSSB-IndexList-r16					OPTIONAL,

resultsCSI-RS-Indexes-r16		ResultsPerCSI-RS-IndexList-r16				OPTIONAL,

...

}

MeasQuantityResults-r16 ::= SEQUENCE {

nr-RSRP-r16						INTEGER (0..127)							OPTIONAL,

nr-RSRQ-r16						INTEGER (0..127)							OPTIONAL

}

ResultsPerSSB-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerSSB-Index-r16

ResultsPerSSB-Index-r16 ::= SEQUENCE {

ssb-Index-r16					INTEGER (0..63),

ssb-Results-r16					MeasQuantityResults-r16

}

ResultsPerCSI-RS-IndexList-r16::= SEQUENCE (SIZE (1..64)) OF ResultsPerCSI-RS-Index-r16

ResultsPerCSI-RS-Index-r16 ::= SEQUENCE {

csi-RS-Index-r16				INTEGER (0..95),

csi-RS-Results-r16				MeasQuantityResults-r16

}

-- ASN1STOP

NR-ECID-SignalMeasurementInformation field descriptions

nr-PrimaryCellMeasuredResults This field contains the NR E-CID measurements for the primary cell.

nr-MeasuredResultsList This field contains the NR E-CID measurements for up to 32 neighbour cells.

nr-PhysCellID This field specifies the NR physical cell identity of the measured cell.

nr-ARFCN This field specifies the ARFCN of the first RE of SSB's RB#10 or the point A of CSI-RS.

nr-CellGlobalID This field specifies the NR cell global ID of the measured cell. The target device shall provide this field if it was able to determine the NCGI of the measured cell at the time of measurement.

systemFrameNumber This field specifies the system frame number of the measured cell during which the measurements have been performed. The target device shall include this field if it was able to determine the SFN of the cell at the time of measurement.

resultsSSB-Cell This field specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

resultsCSI-RS-Cell This field specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) measurement aggregated at cell level, as defined in TS 38.331 [35].

resultsSSB-Indexes This field specifies the SS reference signal received power (SS-RSRP) and quality (SS-RSRQ) measurement per SSB resource, as defined in TS 38.331 [35].

resultsCSI-RS-Indexes This field specifies the CSI-RS reference signal received power (CSI-RSRP) and quality (CSI-RSRQ) per CSI-RS resource, as defined in TS 38.331 [35].

nr-RSRP This field specifies the integer value for RSRP measurements according to Table 10.1.6.1-1 in TS 38.133 [46].

nr-RSRQ This field specifies the integer value for RSRQ measurements accordng to Table 10.1.11.1-1 in TS 38.133 [46].

#### 6.5.9.3	NR E-CID Location Information Request

#### –	NR-ECID-RequestLocationInformation

The IE NR-ECID-RequestLocationInformation is used by the location server to request NR E-CID location measurements from a target device.

-- ASN1START

NR-ECID-RequestLocationInformation-r16 ::= SEQUENCE {

requestedMeasurements-r16		BIT STRING {	ssrsrpReq		(0),

ssrsrqReq		(1),

csirsrpReq		(2),

csirsrqReq		(3)} (SIZE(1..8)),

...

}

-- ASN1STOP

NR-ECID-RequestLocationInformation field descriptions

requestedMeasurements This field specifies the NR E-CID measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

#### 6.5.9.4	NR E-CID Capability Information

#### –	NR-ECID-ProvideCapabilities

The IE NR-ECID-ProvideCapabilities is used by the target device to indicate its capability to support NR E-CID and to provide its NR E-CID positioning capabilities to the location server.

-- ASN1START

NR-ECID-ProvideCapabilities-r16 ::= SEQUENCE {

nr-ECID-MeasSupported-r16		BIT STRING {	ssrsrpSup		(0),

ssrsrqSup		(1),

csirsrpSup		(2),

csirsrqSup		(3)} (SIZE(1..8)),

periodicalReporting-r16		ENUMERATED { supported }							OPTIONAL,

triggeredReporting-r16		ENUMERATED { supported }							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17		ENUMERATED { supported }				OPTIONAL,

scheduledLocationRequest-r17		SEQUENCE {

utcTime-r17			ENUMERATED { supported }	OPTIONAL,

gnssTime-r17		GNSS-ID-Bitmap				OPTIONAL,

e-utraTime-r17		ENUMERATED { supported }	OPTIONAL,

nrTime-r17			ENUMERATED { supported }	OPTIONAL,

relativeTime-r17	ENUMERATED { supported }	OPTIONAL,

...

}												OPTIONAL

]]

}

-- ASN1STOP

NR-ECID-ProvideCapabilities field descriptions

nr-ECID-MeasSupported: Indicates the supported NR ECID measurements: - ssrsrpSup indicates the UE supports SSB based cell/beam specific RSRP measurement; - ssrsrqSup indicates the UE supports SSB based cell/beam specific RSRQ measurement; - csirsrpSup indicates the UE supports CSI-RS based cell/beam specific RSRP measurement; - csirsrqSup indicates the UE supports CSI-RS based cell/beam specific RSRQ measurement.

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

scheduledLocationRequest This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

#### 6.5.9.5	NR E-CID Capability Information Request

#### –	NR-ECID-RequestCapabilities

The IE NR-ECID-RequestCapabilities is used by the location server to request the capability of the target device to support NR E-CID and to request NR E-CID positioning capabilities from a target device.

-- ASN1START

NR-ECID-RequestCapabilities-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.9.6	NR E-CID Error Elements

#### –	NR-ECID-Error

The IE NR-ECID-Error is used by the location server or target device to provide NR E-CID error reasons to the target device or location server, respectively.

-- ASN1START

NR-ECID-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-ECID-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-ECID-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-ECID-LocationServerErrorCauses

The IE NR-ECID-LocationServerErrorCauses is used by the location server to provide NR E-CID error reasons to the target device.

-- ASN1START

NR-ECID-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

...

},

...

}

-- ASN1STOP

#### –	NR-ECID-TargetDeviceErrorCauses

The IE NR-ECID-TargetDeviceErrorCauses is used by the target device to provide NR E-CID error reasons to the location server.

-- ASN1START

NR-ECID-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

requestedMeasurementNotAvailable,

notAllrequestedMeasurementsPossible,

...

},

ss-RSRPMeasurementNotPossible-r16				NULL		OPTIONAL,

ss-RSRQMeasurementNotPossible-r16				NULL		OPTIONAL,

csi-RSRPMeasurementNotPossible-r16				NULL		OPTIONAL,

csi-RSRQMeasurementNotPossible-r16				NULL		OPTIONAL,

...

}

-- ASN1STOP

NR-ECID-TargetDeviceErrorCauses field descriptions

cause This field provides a NR E-CID specific error cause. If the cause value is 'notAllRequestedMeasurementsPossible', the target device was not able to provide all requested NR E-CID measurements (but may be able to provide some measurements). In this case, the target device should include any of the ss-RSRPMeasurementNotPossible, ss-RSRQMeasurementNotPossible, csi-RSRPMeasurementNotPossible, or csi-RSRQMeasurementNotPossible fields, as applicable.

### 6.5.10	NR DL-TDOA Positioning

This clause defines the information elements for NR downlink TDOA positioning (TS 38.305 [40]).

#### 6.5.10.1	NR DL-TDOA Assistance Data

#### –	NR-DL-TDOA-ProvideAssistanceData

The IE NR-DL-TDOA-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-TDOA. It may also be used to provide NR DL-TDOA positioning specific error reason.

-- ASN1START

NR-DL-TDOA-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16 	OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL, 	-- Cond UEB

nr-DL-TDOA-Error-r16				NR-DL-TDOA-Error-r16				OPTIONAL,	-- Need ON

...,

[[ nr-On-Demand-DL-PRS-Configurations-r17

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

SEQUENCE (SIZE (1..maxDL-PRS-Configs-r17)) OF

DL-PRS-Configuration-ID-r17		OPTIONAL,	-- Need ON

area-ID-CellList-r17				Area-ID-CellList-r17				OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need ON, for UE based NR DL-TDOA; otherwise it is not present. |

NR-DL-TDOA-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-TDOA-ProvideAssistanceData message.

nr-PositionCalculationAssistance This field provides position calculation assistance data for UE-based mode.

nr-DL-TDOA-Error This field provides DL-TDOA error reasons.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE 1: In the case of available on-demand DL-PRS configurations for multiple NR positioning methods are provided, the nr-On-Demand-DL-PRS-Configurations shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData. NOTE 2: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-DL-TDOA-ProvideAssistanceData message.

area-ID-CellList This field specifies the network area for which this NR-DL-TDOA-ProvideAssistanceData message is valid.

#### 6.5.10.2	NR DL-TDOA Assistance Data Request

#### –	NR-DL-TDOA-RequestAssistanceData

The IE NR-DL-TDOA-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-TDOA-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16							OPTIONAL,

nr-AdType-r16					BIT STRING {	dl-prs 	(0),

posCalc (1) } (SIZE (1..8)),

...,

[[

nr-PosCalcAssistanceRequest-r17	BIT STRING {	trpLoc 		(0),

beamInfo	(1),

rtdInfo		(2),

beamAntInfo	(3),

losNlosInfo	(4),

trpTEG-Info	(5)

}	(SIZE (1..8))				OPTIONAL,

nr-on-demand-DL-PRS-Request-r17	NR-On-Demand-DL-PRS-Request-r17				OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

nr-PosCalcAssistanceRequest This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not; - bit 5 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is requested or not. This field may only be present if the 'posCalc' bit in nr-AdType is set to value '1'.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for DL-TDOA. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

#### 6.5.10.3	NR DL-TDOA Location Information

#### –	NR-DL-TDOA-ProvideLocationInformation

The IE NR-DL-TDOA-ProvideLocationInformation is used by the target device to provide NR DL-TDOA location measurements to the location server. It may also be used to provide NR DL-TDOA positioning specific error reason.

-- ASN1START

NR-DL-TDOA-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-DL-TDOA-SignalMeasurementInformation-r16

NR-DL-TDOA-SignalMeasurementInformation-r16

OPTIONAL,

nr-dl-tdoa-LocationInformation-r16		NR-DL-TDOA-LocationInformation-r16

OPTIONAL,

nr-DL-TDOA-Error-r16					NR-DL-TDOA-Error-r16			OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.10.4	NR DL-TDOA Location Information Elements

#### –	NR-DL-TDOA-SignalMeasurementInformation

The IE NR-DL-TDOA-SignalMeasurementInformation is used by the target device to provide NR DL-TDOA measurements to the location server.

NOTE 1:	The dl-PRS-ReferenceInfo defines the "RSTD reference" TRP. The nr-RSTD's and nr-RSTD-ResultDiff's in nr-DL-TDOA-MeasList are provided relative to the "RSTD reference" TRP.

NOTE 2:	The "RSTD reference" TRP may or may not be the same as the "assistance data reference" TRP provided by nr-DL-PRS-ReferenceInfo in IE NR-DL-PRS-AssistanceData.

NOTE 3:	The target device includes a value of zero for the nr-RSTD and nr-RSTD-ResultDiff of the "RSTD reference" TRP in nr-DL-TDOA-MeasList.

Editor's Note: FFS on "multiple measurement instances":
Agreement: Support enabling

-	A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and

-	A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and

-	Each measurement instance is reported with its own timestamp

-	FFS: The measurement instances are within a [configured] measurement time window

-	FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set

-	FFS: N (including N=1)

-	FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions

-	FFS: M (including M=1)

-	FFS: details of signalling, procedures, and UE capability if any

-	FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric

Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s).

Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

-- ASN1START

NR-DL-TDOA-SignalMeasurementInformation-r16 ::= SEQUENCE {

dl-PRS-ReferenceInfo-r16		DL-PRS-ID-Info-r16,

nr-DL-TDOA-MeasList-r16			NR-DL-TDOA-MeasList-r16,

...

}

NR-DL-TDOA-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-TDOA-MeasElement-r16

NR-DL-TDOA-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-r16						CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-DL-TDOA-AdditionalMeasurements-r16

NR-DL-TDOA-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17					INTEGER (0..maxNumOfRxTEGs-1-r17)			OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			LOS-NLOS-Indicator-r17						OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-DL-TDOA-AdditionalMeasurementsExt-r17

NR-DL-TDOA-AdditionalMeasurementsExt-r17	OPTIONAL

]]

}

NR-DL-TDOA-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasTDOA-r17)) OF

NR-DL-TDOA-AdditionalMeasurementElement-r16

NR-DL-TDOA-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-RSTD-ResultDiff-r16			CHOICE {

k0-r16						INTEGER (0..8191),

k1-r16						INTEGER (0..4095),

k2-r16						INTEGER (0..2047),

k3-r16						INTEGER (0..1023),

k4-r16						INTEGER (0..511),

k5-r16						INTEGER (0..255),

...

},

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..61)									OPTIONAL,

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

...,

[[

nr-UE-Rx-TEG-ID-r17				INTEGER (0..maxNumOfRxTEGs-1-r17)				OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)									OPTIONAL,

nr-los-nlos-Indicator-r17		LOS-NLOS-Indicator-r17							OPTIONAL,

nr-AdditionalPathListExt-r17	NR-AdditionalPathListExt-r17					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-SignalMeasurementInformation field descriptions

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the TOA and DL PRS-RSRP (if included) measurement is performed. The nr-SFN and nr-Slot in IE NR-TimeStamp correspond to the TRP provided in dl-PRS-ReferenceInfo as specified in TS 38.214 [45]. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-RSTD This field specifies the relative timing difference between this neighbour TRP and the PRS reference TRP, as defined in TS 38.215 [36]. Mapping of the measured quantity is defined as in TS 38.133 [46].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-UE-Rx-TEG-ID This field provides the ID of the UE Rx TEG associated with the TOA measurement. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff. When different UE Rx TEGs for RSTD measurements are requested, the maximum number of reported RSTD measurements associated with different DL-PRS Resources per UE Rx TEG per target TRP is 4.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46]. FFS

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the TOA measurement for the TRP or resource. Note, the TOA measurement refers to the TOA of this neighbour TRP or the reference TRP, as applicable, used to determine the nr-RSTD or nr-RSTD-ResultDiff.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-RSTD value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-RSTD-ResultDiff This field provides the additional DL RSTD measurement result relative to nr-RSTD. The RSTD value of this measurement is obtained by adding the value of this field to the value of the nr-RSTD field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS First Path RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46]. FFS.

#### –	NR-DL-TDOA-LocationInformation

The IE NR-DL-TDOA-LocationInformation is included by the target device when location information derived using NR DL-TDOA is provided to the location server.

-- ASN1START

NR-DL-TDOA-LocationInformation-r16 ::= SEQUENCE {

measurementReferenceTime-r16	CHOICE {

systemFrameNumber-r16			NR-TimeStamp-r16,

utc-time-r16					UTCTime,

...

}															OPTIONAL,

...

}

-- ASN1STOP

NR-DL-TDOA-LocationInformation field descriptions

measurementReferenceTime This field specifies the time for which the location estimate is valid.

#### 6.5.10.5	NR DL-TDOA Location Information Request

#### –	NR-DL-TDOA-RequestLocationInformation

The IE NR-DL-TDOA-RequestLocationInformation is used by the location server to request NR DL-TDOA location measurements from a target device.

-- ASN1START

NR-DL-TDOA-RequestLocationInformation-r16 ::= SEQUENCE {

nr-DL-PRS-RstdMeasurementInfoRequest-r16	ENUMERATED { true }				OPTIONAL,-- Need ON

nr-RequestedMeasurements-r16				BIT STRING { prsrsrpReq (0),

firstPathRsrpReq-r17 (1)

} (SIZE(1..8)),

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-TDOA-ReportConfig-r16					NR-DL-TDOA-ReportConfig-r16		OPTIONAL, -- Need ON

additionalPaths-r16							ENUMERATED { requested }		OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTEG-Request-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17			SEQUENCE {

type-r17		ENUMERATED {hardvalue,softvalue},

granularity-r17	ENUMERATED {trpspecific,resourcespecific},

...

}										OPTIONAL, -- Need ON

additionalPathsExt-r17						ENUMERATED { requested }		OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17		ENUMERATED { requested }		OPTIONAL  -- Need ON

]]

}

NR-DL-TDOA-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSTD-MeasurementsPerTRPPair-r16	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 		INTEGER (0..5)					OPTIONAL, -- Need ON

...,

[[

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

requestedDL-PRS-ProcessingSamples-r17		ENUMERATED { m1, ... }			OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-TDOA-RequestLocationInformation field descriptions

nr-DL-PRS-RstdMeasurementInfoRequest This field indicates whether the target device is requested to report DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) used for determining the timing of each TRP in RSTD measurements.

nr-RequestedMeasurements This field specifies the NR DL-TDOA measurements requested. This is represented by a bit string, with a one-value at the bit position means the particular measurement is requested; a zero-value means not requested.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTEG-Request This field, if present, indicates that the target device is requested to provide the nr-UE-Rx-TEG-ID in IE NR-DL-TDOA-SignalMeasurementInformation.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-TDOA-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-DL-TDOA-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRPP for the additional paths in IE NR-AdditionalPathList.

maxDL-PRS-RSTD-MeasurementsPerTRPPair This field specifies the maximum number of. DL-PRS RSTD measurements per pair of TRPs. The maximum number is defined across all Positioning Frequency Layers.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the DL RSTD measurements. Value (0..5) corresponds to (k0..k5) used for nr-RSTD and nr-RSTD-ResultDiff in NR-DL-TDOA-MeasElement. The UE may select a different granularity value for nr-RSTD and nr-RSTD-ResultDiff.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-TxTEG-Request should also be present.

requestedDL-PRS-ProcessingSamples This field, if present, indicates the requested number of DL-PRS processing samples. Enumerated value 'm1' indicates 1-sample DL-PRS processing is requested as defined in TS 38.133 [46].

#### 6.5.10.6	NR DL-TDOA Capability Information

#### –	NR-DL-TDOA-ProvideCapabilities

The IE NR-DL-TDOA-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-TDOA and to provide its NR DL-TDOA positioning capabilities to the location server.

-- ASN1START

NR-DL-TDOA-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-TDOA-Mode-r16						PositioningModes,

nr-DL-TDOA-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-TDOA-MeasurementCapability-r16	NR-DL-TDOA-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					PositioningModes							OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup 		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3),

losNlosInfoSup	(4),

trpTEG-InfoSup	(5)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		ENUMERATED { hardvalue, softvalue, both },

granularity-r17	ENUMERATED { trpspecific, resourcespecific, both},

...

}								OPTIONAL,	-- Cond losNlosInfoSup

nr-DL-TDOA-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		ENUMERATED { hardvalue, softvalue, both },

granularity-r17	ENUMERATED { trpspecific, resourcespecific, both},

...

}														OPTIONAL,

additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

additionalPathsPowerSupport-r17			ENUMERATED { supported }					OPTIONAL,

scheduledLocationRequest-r17			SEQUENCE {

utcTime-r17			PositioningModes		OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}										OPTIONAL,

e-utraTime-r17		PositioningModes		OPTIONAL,

nrTime-r17			PositioningModes		OPTIONAL,

relativeTime-r17	PositioningModes		OPTIONAL,

...

}											OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxAreaIDs-r17)			OPTIONAL,								...

}														OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ProcessingRRC-Inactive-r17	ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| losNlosInfoSup | The field is mandatory present if the losNlosInfoSup bit-4 in nr-PosCalcAssistanceSupport is set to value '1'; otherwise it is not present. |

NR-DL-TDOA-ProvideCapabilities field descriptions

nr-DL-TDOA-Mode This field specifies the NR DL-TDOA mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-TDOA. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, provides further information on the NR-DL-PRS-Expected-LOS-NLOS-Assistance support: - type indicates whether the target device supports 'hard' value, 'soft' value, or both in LOS-NLOS-Indicator in IE NR-DL-PRS-Expected-LOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-Expected-LOS-NLOS-Assistance 'per-trp', 'per-resource', or both.

nr-DL-TDOA-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-TDOA-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value, 'soft' value, or both in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-DL-TDOA-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device,

additionalPathsPowerSupport This field, if present, indicates that the target device supports the nr-DL-PRS-RSRPP for the additional paths in IE NR-AdditionalPathList.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of area IDs the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for PRS measurements.

nr-DL-PRS-ProcessingRRC-Inactive This field, if present, indicates that the target device supports DL-PRS processing in RRC_INACTIVE state.

#### 6.5.10.6a	NR DL-TDOA Capability Information Elements

#### –	NR-DL-TDOA-MeasurementCapability

The IE NR-DL-TDOA-MeasurementCapability defines the DL-TDOA measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-TDOA. Otherwise, the UE does not include this IE.

-- ASN1START

NR-DL-TDOA-MeasurementCapability-r16 ::= SEQUENCE {

dl-RSTD-MeasurementPerPairOfTRP-FR1-r16			INTEGER (1..4),

dl-RSTD-MeasurementPerPairOfTRP-FR2-r16			INTEGER (1..4),

supportOfDL-PRS-RSRP-MeasFR1-r16				ENUMERATED { supported}	OPTIONAL,

supportOfDL-PRS-RSRP-MeasFR2-r16				ENUMERATED { supported}	OPTIONAL,

...,

[[

supportOfDL-PRS-FirstPathRSRP-MeasFR1-r17		ENUMERATED { supported}				OPTIONAL,

supportOfDL-PRS-FirstPathRSRP-MeasFR2-r17		ENUMERATED { supported}				OPTIONAL,

nr-UE-TEG-Capability-r17						NR-UE-TEG-Capability-r17			OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17						ENUMERATED { supported }			OPTIONAL

]]

}

-- ASN1STOP

NR-DL-TDOA-MeasurementCapability field descriptions

dl-RSTD-MeasurementPerPairOfTRP-FR1 Indicates number of DL RSTD measurements per pair of TRPs on FR1.

dl-RSTD-MeasurementPerPairOfTRP-FR2 Indicates number of DL RSTD measurements per pair of TRPs on FR2.

supportOfDL-PRS-RSRP-MeasFR1 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR1.

supportOfDL-PRS-RSRP-MeasFR2 Indicates whether the UE supports DL-PRS RSRP measurement for DL-TDOA on FR2.

supportOfDL-PRS-FirstPathRSRP-MeasFR1 Indicates whether the target device supports DL-PRS First Path RSRP measurement for DL-TDOA on FR1.

supportOfDL-PRS-FirstPathRSRP-MeasFR2 Indicates whether the target device supports DL-PRS First Path RSRP measurement for DL-TDOA on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state.

Editor's Note: Measurement Capabilities are FFS and require further RAN1 input.

Editor's Note: FFS on FR1/FR2 differentiation for RSRPP.

#### 6.5.10.7	NR DL-TDOA Capability Information Request

#### –	NR-DL-TDOA-RequestCapabilities

The IE NR-DL-TDOA-RequestCapabilities is used by the location server to request the capability of the target device to support NR DL-TDOA and to request NR DL-TDOA positioning capabilities from a target device.

-- ASN1START

NR-DL-TDOA-RequestCapabilities-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.10.8	NR DL-TDOA Error Elements

#### –	NR-DL-TDOA-Error

The IE NR-DL-TDOA-Error is used by the location server or target device to provide NR DL-TDOA error reasons to the target device or location server, respectively.

-- ASN1START

NR-DL-TDOA-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-DL-TDOA-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-DL-TDOA-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-DL-TDOA-LocationServerErrorCauses

The IE NR-DL-TDOA-LocationServerErrorCauses is used by the location server to provide NR DL-TDOA error reasons to the target device.

-- ASN1START

NR-DL-TDOA-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer,

...,

on-demand-dl-prs-NotSupportedByServer-r17,

on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-r17

},

...

}

-- ASN1STOP

#### –	NR-DL-TDOA-TargetDeviceErrorCauses

The IE NR-DL-TDOA-TargetDeviceErrorCauses is used by the target device to provide NR DL-TDOA error reasons to the location server.

-- ASN1START

NR-DL-TDOA-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-TDOA,

locationCalculationAssistanceDataMissing,

...

},

...

}

-- ASN1STOP

### 6.5.11	NR DL-AoD Positioning

This clause defines the information elements for NR downlink AoD positioning (TS 38.305 [40]).

#### 6.5.11.1	NR DL-AoD Assistance Data

#### –	NR-DL-AoD-ProvideAssistanceData

The IE NR-DL-AoD-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted and UE-based NR DL-AoD. It may also be used to provide NR DL-AoD positioning specific error reason.

-- ASN1START

NR-DL-AoD-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16		NR-DL-PRS-AssistanceData-r16		OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16		NR-SelectedDL-PRS-IndexList-r16		OPTIONAL,	-- Need ON

nr-PositionCalculationAssistance-r16

NR-PositionCalculationAssistance-r16

OPTIONAL,	-- Cond UEB

nr-DL-AoD-Error-r16					NR-DL-AoD-Error-r16					OPTIONAL,	-- Need ON

...,

[[

nr-DL-PRS-BeamInfo-r17				NR-DL-PRS-BeamInfo-r16				OPTIONAL,	-- Cond UEA

nr-On-Demand-DL-PRS-Configurations-r17

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

SEQUENCE (SIZE (1..maxDL-PRS-Configs-r17)) OF

DL-PRS-Configuration-ID-r17		OPTIONAL,	-- Need ON

area-ID-CellList-r17				Area-ID-CellList-r17				OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| UEB | The field is optionally present, need ON, for UE based NR DL-AoD; otherwise it is not present. |
| UEA | The field is optionally present, need ON, for UE-assisted NR DL-AoD; otherwise it is not present. |

NR-DL-AoD-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-TDOA-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-DL-AoD-ProvideAssistanceData message.

nr-PositionCalculationAssistance This field provides position calculation assistance data for UE-based mode.

nr-DL-AoD-Error This field provides DL-AoD error reasons.

nr-DL-PRS-BeamInfo This field provides spatial direction information of the DL-PRS Resources included in nr-DL-PRS-AssistanceData or indicated by nr-SelectedDL-PRS-IndexList.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE 1: In the case of available on-demand DL-PRS configurations for multiple NR positioning methods are provided, the nr-On-Demand-DL-PRS-Configurations shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData. NOTE 2: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-Multi-RTT-ProvideAssistanceData or NR-DL-TDOA-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-DL-AoD-ProvideAssistanceData message.

area-ID-CellList This field specifies the network area for which this NR-DL-AoD-ProvideAssistanceData message is valid.

#### 6.5.11.2	NR DL-AoD Assistance Data Request

#### –	NR-DL-AoD-RequestAssistanceData

The IE NR-DL-AoD-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-DL-AoD-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-AdType-r16					BIT STRING { 	dl-prs 	(0),

posCalc (1) } (SIZE (1..8)),

...,

[[

nr-PosCalcAssistanceRequest-r17				BIT STRING {	trpLoc 		(0),

beamInfo	(1),

rtdInfo		(2),

beamAntInfo	(3),

losNlosInfo	(4),

trpTEG-Info	(5)

}	(SIZE (1..8))		OPTIONAL,

nr-DL-PRS-ExpectedAoD-or-AoA-Req-r17		ENUMERATED { eAoD, eAoA }			OPTIONAL,

nr-DL-PRS-BeamInfoReq-r17					ENUMERATED { requested }			OPTIONAL,

nr-on-demand-DL-PRS-Request-r17				NR-On-Demand-DL-PRS-Request-r17		OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-AdType This field indicates the requested assistance data. dl-prs means requested assistance data is nr-DL-PRS-AssistanceData, posCalc means requested assistance data is nr-PositionCalculationAssistance for UE based positioning.

nr-PosCalcAssistanceRequest This field indicates the Position Calculation Assistance Data requested. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is requested; a zero-value means not requested. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is requested or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is requested or not; - bit 4 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is requested or not; - bit 5 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is requested or not. This field may only be present if the 'posCalc' bit in nr-AdType is set to value '1'.

nr-DL-PRS-ExpectedAoD-or-AoA-Req This field, if present, indicates that the IE NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData is requested. Enumerated value 'eAoD' indicates that expected AoD information is requested; value 'eAoA' indicates that expected AoA information is requested. This field may only be present if the 'dl-prs' bit in nr-AdType is set to value '1'.

nr-DL-PRS-BeamInfoReq This field, if present, indicates that the IE NR-DL-PRS-BeamInfo is requested.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for DL-AoD. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

#### 6.5.11.3	NR DL-AoD Location Information

#### –	NR-DL-AoD-ProvideLocationInformation

The IE NR-DL-AoD-ProvideLocationInformation is used by the target device to provide NR DL-AoD location measurements to the location server. It may also be used to provide NR DL-AoD positioning specific error reason.

-- ASN1START

NR-DL-AoD-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-DL-AoD-SignalMeasurementInformation-r16

NR-DL-AoD-SignalMeasurementInformation-r16

OPTIONAL,

nr-dl-AoD-LocationInformation-r16		NR-DL-AoD-LocationInformation-r16

OPTIONAL,

nr-DL-AoD-Error-r16						NR-DL-AoD-Error-r16				OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.11.4	NR DL-AoD Location Information Elements

#### –	NR-DL-AoD-SignalMeasurementInformation

The IE NR-DL-AoD-SignalMeasurementInformation is used by the target device to provide NR DL-AoD measurements to the location server.

Editor's Note: FFS on "multiple measurement instances":
Agreement: Support enabling

-	A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and

-	A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and

-	Each measurement instance is reported with its own timestamp

-	FFS: The measurement instances are within a [configured] measurement time window

-	FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set

-	FFS: N (including N=1)

-	FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions

-	FFS: M (including M=1)

-	FFS: details of signalling, procedures, and UE capability if any

-	FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric

Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s).

Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

-- ASN1START

NR-DL-AoD-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-DL-AoD-MeasList-r16			NR-DL-AoD-MeasList-r16,

...

}

NR-DL-AoD-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-DL-AoD-MeasElement-r16

NR-DL-AoD-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15								OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15						OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

nr-DL-AoD-AdditionalMeasurements-r16

NR-DL-AoD-AdditionalMeasurements-r16	OPTIONAL,

...,

[[

nr-DL-PRS-FirstPathRSRP-Result-r17

INTEGER (0..126)						OPTIONAL,

nr-los-nlos-Indicator-r17		LOS-NLOS-Indicator-r17					OPTIONAL,

nr-DL-AoD-AdditionalMeasurementsExt-r17

NR-DL-AoD-AdditionalMeasurementsExt-r17	OPTIONAL

]]

}

NR-DL-AoD-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..7)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasAoD-r17)) OF

NR-DL-AoD-AdditionalMeasurementElement-r16

NR-DL-AoD-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16	 			OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 			OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-DL-PRS-RSRP-ResultDiff-r16	INTEGER (0..30),

nr-DL-PRS-RxBeamIndex-r16		INTEGER (1..8)							OPTIONAL,

...,

[[

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..30)							OPTIONAL,

nr-los-nlos-Indicator-r17		LOS-NLOS-Indicator-r17					OPTIONAL

]]

}

-- ASN1STOP

NR-DL-AoD-SignalMeasurementInformation field descriptions

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-TimeStamp This field specifies the time instance at which the measurement is performed.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46].

nr-DL-PRS-RxBeamIndex This field provides an index of the target device receive beam used for DL-PRS measurements. If the value of the receive beam index for two or more DL PRS measurements is the same, it indicates that the target device receive beam for the two or more DL PRS measurements were made with the same RX beam. The field is mandatory present if at least two DL-PRS RSRP measurements from the same DL-PRS Resource Set have been made with the same RX beam by the target device; otherwise it is not present.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46]. FFS

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the RSRP or First Path-RSRP measurement for the TRP or resource.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS First Path RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46]. FFS

#### –	NR-DL-AoD-LocationInformation

The IE NR-DL-AoD-LocationInformation is included by the target device when location information derived using NR DL-AoD is provided to the location server.

-- ASN1START

NR-DL-AoD-LocationInformation-r16 ::= SEQUENCE {

measurementReferenceTime-r16	CHOICE {

sfn-time-r16					NR-TimeStamp-r16,

utc-time-r16					UTCTime,

...

}															OPTIONAL,

...

}

-- ASN1STOP

NR-DL-AoD-LocationInformation field descriptions

measurementReferenceTime This field specifies the time for which the location estimate is valid.

#### 6.5.11.5	NR DL-AoD Location Information Request

#### –	NR-DL-AoD-RequestLocationInformation

The IE NR-DL-AoD-RequestLocationInformation is used by the location server to request NR DL-AoD location measurements from a target device.

-- ASN1START

NR-DL-AoD-RequestLocationInformation-r16 ::= SEQUENCE {

nr-AssistanceAvailability-r16				BOOLEAN,

nr-DL-AoD-ReportConfig-r16					NR-DL-AoD-ReportConfig-r16,

...

}

NR-DL-AoD-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementsPerTRP-r16		INTEGER (1..8)			OPTIONAL, -- Need ON

...,

[[

maxDL-PRS-RSRP-MeasurementsPerTRP-r17		INTEGER (9..24)				OPTIONAL, -- Need ON

firstPathRSRP-MeasurementReq-r17			ENUMERATED { requested }	OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17			SEQUENCE {

type-r17		ENUMERATED {hardvalue,softvalue},

granularity-r17	ENUMERATED {trpspecific,resourcespecific},

...

}									OPTIONAL, -- Need ON

requestedDL-PRS-ProcessingSamples-r17		ENUMERATED { m1, ... }		OPTIONAL  -- Need ON

]]

}

-- ASN1STOP

NR-DL-AoD-RequestLocationInformation field descriptions

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RSRP-MeasurementsPerTRP This field specifies the maximum number of DL-PRS RSRP measurements on different DL-PRS Resources from the same TRP.

firstPathRSRP-MeasurementReq This field, if present, indicates that the target device is requested to measure the PRS RSRP of the first path.

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-DL-AoD-SignalMeasurementInformation.

requestedDL-PRS-ProcessingSamples This field, if present, indicates the requested number of DL-PRS processing samples. Enumerated value 'm1' indicates 1-sample DL-PRS processing is requested as defined in TS 38.133 [46].

#### 6.5.11.6	NR DL-AoD Capability Information

#### –	NR-DL-AoD-ProvideCapabilities

The IE NR-DL-AoD-ProvideCapabilities is used by the target device to indicate its capability to support NR DL-AoD and to provide its NR DL-AoD positioning capabilities to the location server.

-- ASN1START

NR-DL-AoD-ProvideCapabilities-r16 ::= SEQUENCE {

nr-DL-AoD-Mode-r16						PositioningModes,

nr-DL-AoD-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-DL-AoD-MeasurementCapability-r16		NR-DL-AoD-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

periodicalReporting-r16					PositioningModes						OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			PositioningModes							OPTIONAL,

nr-PosCalcAssistanceSupport-r17			BIT STRING {	trpLocSup 		(0),

beamInfoSup		(1),

rtdInfoSup		(2),

beamAntInfoSup	(3),

losNlosInfoSup	(4),

trpTEG-InfoSup	(5)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-AssistanceDataSupport-r17	SEQUENCE {

type-r17		ENUMERATED { hardvalue, softvalue, both },

granularity-r17	ENUMERATED { trpspecific, resourcespecific, both},

...

}								OPTIONAL,	-- Cond losNlosInfoSup

nr-DL-PRS-ExpectedAoD-or-AoA-Sup-r17	BIT STRING {	eAoD 		(0),

eAoA		(1)

}	(SIZE (1..8))				OPTIONAL,

nr-DL-PRS-BeamInfoSup-r17				ENUMERATED { supported }					OPTIONAL,

dl-PRS-ResourcePrioritySubset-Sup-r17	ENUMERATED { supported }					OPTIONAL,

nr-DL-AoD-On-Demand-DL-PRS-Support-r17	NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		ENUMERATED { hardvalue, softvalue, both },

granularity-r17	ENUMERATED { trpspecific, resourcespecific, both},

...

}														OPTIONAL,

scheduledLocationRequest-r17			SEQUENCE {

utcTime-r17			PositioningModes		OPTIONAL,

gnssTime-r17		SEQUENCE {

posModes-r17		PositioningModes,

gnss-TimeIDs-r17	GNSS-ID-Bitmap

}										OPTIONAL,

e-utraTime-r17		PositioningModes		OPTIONAL,

nrTime-r17			PositioningModes		OPTIONAL,

relativeTime-r17	PositioningModes		OPTIONAL,

...

}											OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxAreaIDs-r17)			OPTIONAL,

...

}											OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ProcessingRRC-Inactive-r17	ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| losNlosInfoSup | The field is mandatory present if the losNlosInfoSup bit-4 in nr-PosCalcAssistanceSupport is set to value '1'; otherwise it is not present. |

NR-DL-AoD-ProvideCapabilities field descriptions

nr-DL-AoD-Mode This field specifies the NR DL-AoD mode(s) supported by the target device.

periodicalReporting This field, if present, specifies the positioning modes for which the target device supports periodicalReporting. This is represented by a bit string, with a one-value at the bit position means periodicalReporting for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support periodicalReporting in CommonIEsRequestLocationInformation.

ten-ms-unit-ResponseTime This field, if present, specifies the positioning modes for which the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation. This is represented by a bit string, with a one-value at the bit position means 'ten-milli-seconds' response time unit for the positioning mode is supported; a zero-value means not supported. If this field is absent, the target device does not support 'ten-milli-seconds' response time unit in CommonIEsRequestLocationInformation.

nr-PosCalcAssistanceSupport This field indicates the Position Calculation Assistance Data supported by the target device for UE-based DL-AoD. This is represented by a bit string, with a one-value at the bit position means the particular assistance data is supported; a zero-value means not supported. - bit 0 indicates whether the field nr-TRP-LocationInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 1 indicates whether the field nr-DL-PRS-BeamInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 2 indicates whether the field nr-RTD-Info in IE NR-PositionCalculationAssistance is supported or not; - bit 3 indicates whether the field nr-TRP-BeamAntennaInfo in IE NR-PositionCalculationAssistance is supported or not; - bit 4 indicates whether the field nr-DL-PRS-Expected-LOS-NLOS-Assistance in IE NR-PositionCalculationAssistance is supported or not; - bit 5 indicates whether the field nr-DL-PRS-TRP-TEG-Info in IE NR-PositionCalculationAssistance is supported or not.

nr-los-nlos-AssistanceDataSupport This field, if present, provides further information on the NR-DL-PRS-Expected-LOS-NLOS-Assistance support: - type indicates whether the target device supports 'hard' value, 'soft' value, or both in LOS-NLOS-Indicator in IE NR-DL-PRS-Expected-LOS-NLOS-Assistance. - granularity indicates whether the target device supports nr-los-nlos-indicator in IE NR-DL-PRS-Expected-LOS-NLOS-Assistance 'per-trp', 'per-resource', or both.

nr-DL-PRS-ExpectedAoD-or-AoA-Sup This field, if present, indicates that the target device supports the NR-DL-PRS-ExpectedAoD-or-AoA in NR-DL-PRS-AssistanceData.

nr-DL-PRS-BeamInfoSup This field, if present, indicates that the target device supports the NR-DL-PRS-BeamInfo in IE NR-DL-AoD-ProvideAssistanceData.

dl-PRS-ResourcePrioritySubset-Sup This field, if present, indicates that the target device supports the DL-PRS-ResourcePrioritySubset in IE NR-DL-PRS-Info.

nr-DL-AoD-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-DL-AoD-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value, 'soft' value, or both in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both.

scheduledLocationRequest This field, if present, specifies the positioning modes for which the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time for each positioning mode. If this field is absent, the target device does not support scheduled location requests.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of area IDs the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for PRS measurements.

nr-DL-PRS-ProcessingRRC-Inactive This field, if present, indicates that the target device supports DL-PRS processing in RRC_INACTIVE state.

#### 6.5.11.6a	NR DL-AoD Capability Information Elements

#### –	NR-DL-AoD-MeasurementCapability

The IE NR-DL-AoD-MeasurementCapability defines the DL-AoD measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for DL-AoD. Otherwise, the UE does not include this IE;

-- ASN1START

NR-DL-AoD-MeasurementCapability-r16 ::= SEQUENCE {

maxDL-PRS-RSRP-MeasurementFR1-r16		INTEGER (1..8),

maxDL-PRS-RSRP-MeasurementFR2-r16		INTEGER (1..8),

dl-AoD-MeasCapabilityBandList-r16		SEQUENCE (SIZE (1..nrMaxBands-r16)) OF

DL-AoD-MeasCapabilityPerBand-r16,

...,

[[

maxDL-PRS-RSRP-MeasurementFR1-r17		ENUMERATED { n16, n24 }					OPTIONAL,

maxDL-PRS-RSRP-MeasurementFR2-r17		ENUMERATED { n16, n24 }					OPTIONAL,

maxDL-PRS-FirstPathRSRP-MeasPerTRP-r17	ENUMERATED { n2, n4, n8, n16, n24 }		OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17				ENUMERATED { supported }				OPTIONAL

]]

}

DL-AoD-MeasCapabilityPerBand-r16 ::= SEQUENCE {

freqBandIndicatorNR-r16				FreqBandIndicatorNR-r16,

simul-NR-DL-AoD-DL-TDOA-r16			ENUMERATED { supported}	OPTIONAL,

simul-NR-DL-AoD-Multi-RTT-r16		ENUMERATED { supported}	OPTIONAL,

...

}

-- ASN1STOP

NR-DL-AoD-MeasurementCapability field descriptions

maxDL-PRS-RSRP-MeasurementFR1 Indicates the maximum number of DL-PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR1. If this field with -r17 suffix is present, the target device should set the field with -r16 suffix to value '8'.

maxDL-PRS-RSRP-MeasurementFR2 Indicates the maximum number of DL-PRS RSRP measurements on different PRS resources from the same TRP supported by the UE on FR2. If this field with -r17 suffix is present, the target device should set the field with -r16 suffix to value '8'.

maxDL-PRS-FirstPathRSRP-MeasPerTRP This field, if present, indicates that the target device supports measuring and reporting the PRS RSRPP of the first path. The enumerated value indicates the maximum number of first path PRS RSRPP per TRP supported.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state.

simul-NR-DL-AoD-DL-TDOA Indicates whether the UE supports simultaneous processing for DL-AoD and DL-TDOA measurements. The UE can include this field only if the UE supports DL-TDOA and DL-AoD. Otherwise, the UE does not include this field.

simul-NR-DL-AoD-Multi-RTT Indicates whether the UE supports simultaneous processing for DL-AoD and UE Multi-RTT measurements. The UE can include this field only if the UE supports Multi-RTT and DL-AoD. Otherwise, the UE does not include this field.

Editor's Note: Measurement Capabilities are FFS and require further RAN1 input.

#### 6.5.11.7	NR DL-AoD Capability Information Request

#### –	NR-DL-AoD-RequestCapabilities

The IE NR-DL-AoD-RequestCapabilities is used by the location server to request the capability of the target device to support NR DL-AoD and to request NR DL-AoD positioning capabilities from a target device.

-- ASN1START

NR-DL-AoD-RequestCapabilities-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.11.8	NR DL-AoD Error Elements

#### –	NR-DL-AoD-Error

The IE NR-DL-AoD-Error is used by the location server or target device to provide NR DL-AoD error reasons to the target device or location server, respectively.

-- ASN1START

NR-DL-AoD-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-DL-AoD-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-DL-AoD-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-DL-AoD-LocationServerErrorCauses

The IE NR-DL-AoD-LocationServerErrorCauses is used by the location server to provide NR DL-AoD error reasons to the target device.

-- ASN1START

NR-DL-AoD-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

notProvidedAssistanceDataNotSupportedByServer,

...,

on-demand-dl-prs-NotSupportedByServer-r17,

on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-r17

},

...

}

-- ASN1STOP

#### –	NR-DL-AoD-TargetDeviceErrorCauses

The IE NR-DL-AoD-TargetDeviceErrorCauses is used by the target device to provide NR DL-AoD error reasons to the location server.

-- ASN1START

NR-DL-AoD-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

thereWereNotEnoughSignalsReceivedForUeBasedDL-AoD,

locationCalculationAssistanceDataMissing,

...

},

...

}

-- ASN1STOP

### 6.5.12	NR Multi-RTT Positioning

This clause defines the information elements for NR Multi-RTT positioning (TS 38.305 [40]).

#### 6.5.12.1	NR Multi-RTT Assistance Data

#### –	NR-Multi-RTT-ProvideAssistanceData

The IE NR-Multi-RTT-ProvideAssistanceData is used by the location server to provide assistance data to enable UE-assisted NR Multi-RTT. It may also be used to provide NR Multi-RTT positioning specific error reason.

-- ASN1START

NR-Multi-RTT-ProvideAssistanceData-r16 ::= SEQUENCE {

nr-DL-PRS-AssistanceData-r16			NR-DL-PRS-AssistanceData-r16	OPTIONAL,	-- Need ON

nr-SelectedDL-PRS-IndexList-r16			NR-SelectedDL-PRS-IndexList-r16 OPTIONAL,	-- Need ON

nr-Multi-RTT-Error-r16					NR-Multi-RTT-Error-r16			OPTIONAL,	-- Need ON

...,

[[ nr-On-Demand-DL-PRS-Configurations-r17

NR-On-Demand-DL-PRS-Configurations-r17

OPTIONAL,	-- Need ON

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList-r17

SEQUENCE (SIZE (1..maxDL-PRS-Configs-r17)) OF

DL-PRS-Configuration-ID-r17		OPTIONAL,	-- Need ON

area-ID-CellList-r17					Area-ID-CellList-r17			OPTIONAL	-- Need ON

]]

}

-- ASN1STOP

NR-Multi-RTT-ProvideAssistanceData field descriptions

nr-DL-PRS-AssistanceData This field specifies the assistance data reference and neighbour TRPs and provides the DL-PRS configuration for the TRPs. Note, if this field is absent but the nr-SelectedDL-PRS-IndexList field is present, the nr-DL-PRS-AssistanceData may be provided in IE NR-DL-TDOA-ProvideAssistanceData or NR-DL-AoD-ProvideAssistanceData.

nr-SelectedDL-PRS-IndexList This field specifies the DL-PRS Resources which are applicable for this NR-Multi-RTT-ProvideAssistanceData message.

nr-Multi-RTT-Error This field provides Multi-RTT error reasons.

nr-On-Demand-DL-PRS-Configurations This field provides a set of available DL-PRS configurations which can be requested by the target device on-demand. NOTE 1: In the case of available on-demand DL-PRS configurations for multiple NR positioning methods are provided, the nr-On-Demand-DL-PRS-Configurations shall be present in only one of NR-Multi-RTT-ProvideAssistanceData, NR-DL-AoD-ProvideAssistanceData, or NR-DL-TDOA-ProvideAssistanceData. NOTE 2: If this field is absent but the nr-On-Demand-DL-PRS-Configurations-Selected-IndexList is present, the nr-On-Demand-DL-PRS-Configurations may be provided in IE NR-DL-AoD-ProvideAssistanceData or NR-DL-TDOA-ProvideAssistanceData.

nr-On-Demand-DL-PRS-Configurations-Selected-IndexList This field specifies the selected available on-demand DL-PRS configurations which are applicable for this NR-Multi-RTT-ProvideAssistanceData message.

area-ID-CellList This field specifies the network area for which this NR-Multi-RTT-ProvideAssistanceData message is valid.

#### 6.5.12.2	NR Multi-RTT Assistance Data Request

#### –	NR-Multi-RTT-RequestAssistanceData

The IE NR-Multi-RTT-RequestAssistanceData is used by the target device to request assistance data from a location server.

-- ASN1START

NR-Multi-RTT-RequestAssistanceData-r16 ::= SEQUENCE {

nr-PhysCellID-r16				NR-PhysCellID-r16						OPTIONAL,

nr-AdType-r16					BIT STRING { 	dl-prs (0),

ul-srs (1) } (SIZE (1..8)),

...,

[[

nr-on-demand-DL-PRS-Request-r17				NR-On-Demand-DL-PRS-Request-r17			OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-RequestAssistanceData field descriptions

nr-PhysCellID This field specifies the NR physical cell identity of the current primary cell of the target device.

nr-on-demand-DL-PRS-Request This field indicates the on-demand DL-PRS requested for Multi-RTT. This field may be included when the dl-prs bit in nr-AdType is set to value '1'.

#### 6.5.12.3	NR Multi-RTT Location Information

#### –	NR-Multi-RTT-ProvideLocationInformation

The IE NR-Multi-RTT-ProvideLocationInformation is used by the target device to provide NR Multi-RTT location measurements to the location server. It may also be used to provide NR Multi-RTT positioning specific error reason.

-- ASN1START

NR-Multi-RTT-ProvideLocationInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-SignalMeasurementInformation-r16

NR-Multi-RTT-SignalMeasurementInformation-r16

OPTIONAL,

nr-Multi-RTT-Error-r16					NR-Multi-RTT-Error-r16					OPTIONAL,

...

}

-- ASN1STOP

#### 6.5.12.4	NR Multi-RTT Location Information Elements

#### –	NR-Multi-RTT-SignalMeasurementInformation

The IE NR-Multi-RTT-SignalMeasurementInformation is used by the target device to provide NR Multi-RTT measurements to the location server.

Editor's Note: FFS on "multiple measurement instances":
Agreement: Support enabling

-	A UE to report one or more measurement instances (of RSTD, DL RSRP, and/or UE Rx-Tx time difference measurements) in a single measurement report to LMF for UE-assisted positioning, and

-	A TRP to report one or more measurement instances (of RTOA, UL RSRP, and/or gNB Rx-Tx time difference measurements) in a single measurement report to LMF, and

-	Each measurement instance is reported with its own timestamp

-	FFS: The measurement instances are within a [configured] measurement time window

-	FFS: Each UE measurement instance can be configured with N instances of the DL-PRS Resource Set

-	FFS: N (including N=1)

-	FFS: Each TRP measurement instance can be configured with M SRS measurement time occasions

-	FFS: M (including M=1)

-	FFS: details of signalling, procedures, and UE capability if any

-	FFS: whether and how to consider the additional enhancement related to measurement reporting of multi-paths and quality metric

Note 1: A measurement instance refers to one or more measurements, which can either be the same or different types, which are obtained from the same DL PRS resource(s), or the same UL SRS resource(s).

Note 2: This enhancement has no intention to change the mapping of measurement types to Rel-16 positioning techniques and no intention to introduce new positioning techniques either.

-- ASN1START

NR-Multi-RTT-SignalMeasurementInformation-r16 ::= SEQUENCE {

nr-Multi-RTT-MeasList-r16		NR-Multi-RTT-MeasList-r16,

nr-NTA-Offset-r16				ENUMERATED { nTA1, nTA2, nTA3, nTA4, ... }		OPTIONAL,

...,

[[

nr-SRS-TxTEG-Set-r17			SEQUENCE (SIZE(1..maxTxTEG-Sets-r17)) OF

NR-SRS-TxTEG-Element-r17					OPTIONAL

-- Cond Case2-3

]]

}

NR-Multi-RTT-MeasList-r16 ::= SEQUENCE (SIZE(1..nrMaxTRPs-r16)) OF NR-Multi-RTT-MeasElement-r16

NR-Multi-RTT-MeasElement-r16 ::= SEQUENCE {

dl-PRS-ID-r16					INTEGER (0..255),

nr-PhysCellID-r16				NR-PhysCellID-r16								OPTIONAL,

nr-CellGlobalID-r16				NCGI-r15										OPTIONAL,

nr-ARFCN-r16					ARFCN-ValueNR-r15								OPTIONAL,

nr-DL-PRS-ResourceID-r16		NR-DL-PRS-ResourceID-r16						OPTIONAL,

nr-DL-PRS-ResourceSetID-r16		NR-DL-PRS-ResourceSetID-r16 					OPTIONAL,

nr-UE-RxTxTimeDiff-r16			CHOICE {

k0-r16						INTEGER (0..1970049),

k1-r16						INTEGER (0..985025),

k2-r16						INTEGER (0..492513),

k3-r16						INTEGER (0..246257),

k4-r16						INTEGER (0..123129),

k5-r16						INTEGER (0..61565),

...

},

nr-AdditionalPathList-r16		NR-AdditionalPathList-r16						OPTIONAL,

nr-TimeStamp-r16				NR-TimeStamp-r16,

nr-TimingQuality-r16			NR-TimingQuality-r16,

nr-DL-PRS-RSRP-Result-r16		INTEGER (0..126)								OPTIONAL,

nr-Multi-RTT-AdditionalMeasurements-r16

NR-Multi-RTT-AdditionalMeasurements-r16			OPTIONAL,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-Result-r17	INTEGER (0..126)							OPTIONAL,

nr-los-nlos-Indicator-r17			LOS-NLOS-Indicator-r17						OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL,

nr-Multi-RTT-AdditionalMeasurementsExt-r17

NR-Multi-RTT-AdditionalMeasurementsExt-r17	OPTIONAL

]]

}

NR-Multi-RTT-AdditionalMeasurements-r16 ::= SEQUENCE (SIZE (1..3)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementsExt-r17 ::= SEQUENCE (SIZE (1..maxAddMeasRTT-r17)) OF

NR-Multi-RTT-AdditionalMeasurementElement-r16

NR-Multi-RTT-AdditionalMeasurementElement-r16 ::= SEQUENCE {

nr-DL-PRS-ResourceID-r16			NR-DL-PRS-ResourceID-r16					OPTIONAL,

nr-DL-PRS-ResourceSetID-r16			NR-DL-PRS-ResourceSetID-r16 				OPTIONAL,

nr-DL-PRS-RSRP-ResultDiff-r16		INTEGER (0..61)								OPTIONAL,

nr-UE-RxTxTimeDiffAdditional-r16	CHOICE {

k0-r16							INTEGER (0..8191),

k1-r16							INTEGER (0..4095),

k2-r16							INTEGER (0..2047),

k3-r16							INTEGER (0..1023),

k4-r16							INTEGER (0..511),

k5-r16							INTEGER (0..255),

...

},

nr-TimingQuality-r16				NR-TimingQuality-r16,

nr-AdditionalPathList-r16			NR-AdditionalPathList-r16					OPTIONAL,

nr-TimeStamp-r16					NR-TimeStamp-r16,

...,

[[

nr-UE-RxTx-TEG-Info-r17				NR-UE-RxTx-TEG-Info-r17						OPTIONAL,

nr-DL-PRS-FirstPathRSRP-ResultDiff-r17

INTEGER (0..61)								OPTIONAL,

nr-los-nlos-Indicator-r17			LOS-NLOS-Indicator-r17						OPTIONAL,

nr-AdditionalPathListExt-r17		NR-AdditionalPathListExt-r17				OPTIONAL

]]

}

NR-SRS-TxTEG-Element-r17 ::= SEQUENCE {

nr-TimeStamp-r17				NR-TimeStamp-r16						OPTIONAL,	-- Need OP

nr-UE-Tx-TEG-ID-r17				INTEGER (0..maxNumOfTxTEGs-1-r17),

srs-PosResourceList-r17			SEQUENCE (SIZE (1..maxNumOfSRS-PosResourceSets-r17)) OF

SRS-PosResources-r17,

...

} -- FFS if nr-TimeStamp-r17 is needed

SRS-PosResources-r17 ::= SEQUENCE {

srs-PosResourceSetId-r17		INTEGER (0..maxNumOfSRS-PosResourceSets-1-r17),

srs-PosResourceIdList-r17		SEQUENCE (SIZE (1..maxNumOfSRS-PosResources-r17)) OF

INTEGER (0..maxNumOfSRS-PosResources-1-r17),

...

}

NR-UE-RxTx-TEG-Info-r17 ::= CHOICE { -- FFS if the CHOICE structure is needed

case1-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17)

},

case2-r17				SEQUENCE {

nr-UE-RxTx-TEG-ID-r17	INTEGER (0..maxNumOfRxTxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

case3-r17				SEQUENCE {

nr-UE-Rx-TEG-ID-r17		INTEGER (0..maxNumOfRxTEGs-1-r17),

nr-UE-Tx-TEG-Index-r17	INTEGER (1..maxTxTEG-Sets-r17)

},

...

} -- FFS the nr-UE-Tx-TEG-ID-r17 in case2 and case3 (pending RAN1)

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| Case2-3 | The field is mandatory present if the IE NR-UE-RxTx-TEG-Info is provided for choice's case2 and case3. |

NR-Multi-RTT-SignalMeasurementInformation field descriptions

nr-NTA-Offset This field provides the NTAoffset used by the target device as specified in TS 38.133 [46], Table 7.1.2-2. Enumerated values nTA1, nTA2, nTA3, and nTA4 correspond to NTAoffset of 25600 Tc, 0 Tc, 39936 Tc, and 13792 Tc, respectively.

nr-SRS-TxTEG-Set This field provides the SRS for Positioning Resources associated with a particular UE Tx TEG and comprises the following subfields: - nr-TimeStamp specifies the start time for which the NR-SRS-TxTEG-Element is valid. If this field is absent, the nr-TimeStamp of this instance of the NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set is the same as the nr-TimeStamp of the previous instance of the NR-SRS-TxTEG-Element. If this field is also absent in the first NR-SRS-TxTEG-Element of the nr-SRS-TxTEG-Set, all NR-SRS-TxTEG-Element's provided are valid for the measurement period of the NR-Multi-RTT-SignalMeasurementInformation. (FFS) - nr-ue-Tx-TEG-ID specifies the ID of this UE Tx TEG. - srs-PosResourceList specifies the SRS for Positioning Resources belonging to this UE Tx TEG.

dl-PRS-ID This field is used along with a DL-PRS Resource Set ID and a DL-PRS Resources ID to uniquely identify a DL-PRS Resource. This ID can be associated with multiple DL-PRS Resource Sets associated with a single TRP. Each TRP should only be associated with one such ID.

nr-PhysCellID This field specifies the physical cell identity of the associated TRP, as defined in TS 38.331 [35].

nr-CellGlobalID This field specifies the NCGI, the globally unique identity of a cell in NR, of the associated TRP, as defined in TS 38.331 [35].

nr-ARFCN This field specifies the NR-ARFCN of the TRP's CD-SSB (as defined in TS 38.300 [47]) corresponding to nr-PhysCellID.

nr-UE-RxTxTimeDiff This field specifies the UE Rx–Tx time difference measurement, as defined in TS 38.215 [36].

nr-AdditionalPathList This field specifies one or more additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathListExt shall be absent.

nr-TimeStamp This field specifies the time instance for which the measurement is performed.

nr-TimingQuality This field specifies the target device′s best estimate of the quality of the measurement.

nr-DL-PRS-RSRP-Result This field specifies the NR DL-PRS reference signal received power (DL PRS-RSRP) measurement, as defined in TS 38.215 [36]. The mapping of the quantity is defined as in TS 38.133 [46].

nr-UE-RxTx-TEG-Info This field provides the ID(s) of the UE TEG associated with the nr-UE-RxTxTimeDiff or nr-UE-RxTxTimeDiffAdditional measurement. One of the following combinations of TEG IDs can be provided: - case1 provides the UE RxTx TEG ID; - case2 provides the UE RxTx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on; - case3 provides the UE Rx TEG ID together with the UE Tx TEG ID. The nr-UE-Tx-TEG-Index provides the index to the nr-SRS-TxTEG-Set field for the applicable UE Tx TEG ID, where value '1' indicates the first NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, value '2' indicates the second NR-SRS-TxTEG-Element in nr-SRS-TxTEG-Set, and so on.

nr-DL-PRS-FirstPathRSRP-Result This field specifies the NR DL PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time, as defined in TS 38.215 [36]. The mapping of the measured quantity is defined as in TS 38.133 [46]. FFS

nr-los-nlos-Indicator This field specifies the target device's best estimate of the LOS or NLOS of the UE Rx-Tx Time Difference, RSRP or First Path-RSRP measurement for the TRP or resource.

nr-AdditionalPathListExt This field provides up to 8 additional detected path timing values for the TRP or resource, relative to the path timing used for determining the nr-UE-RxTxTimeDiff value. If this field was requested but is not included, it means the UE did not detect any additional path timing values. If this field is present, the field nr-AdditionalPathList shall be absent.

nr-DL-PRS-RSRP-ResultDiff This field provides the additional DL-PRS RSRP measurement result relative to nr-DL-PRS-RSRP-Result. The DL-PRS RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-RSRP-Result. The mapping of this field is defined as in TS 38.133 [46].

nr-UE-RxTxTimeDiffAdditional This field provides the additional UE Rx-Tx Difference measurement result relative to nr-UE-RxTxTimeDiff. The UE Rx-Tx Difference value of this measurement is obtained by adding the value of this field to the value of the nr-UE-RxTxTimeDiff field. The mapping of the field is defined in TS 38.133 [46].

nr-DL-PRS-FirstPathRSRP-ResultDiff This field specifies the additional NR DL-PRS reference signal received path power (DL PRS-RSRPP) of the first detected path in time relative to nr-DL-PRS-FirstPathRSRP-Result. The DL-PRS First Path RSRP value of this measurement is obtained by adding the value of this field to the value of the nr-DL-PRS-FirstPathRSRP-Result field. The mapping of the field is defined in TS 38.133 [46]. FFS

Editor's Note: RAN1: "FFS: whether there is a need to include the positioning SRS resource set ID in ueTxTEG". "FFS: A triplet of UE {RxTx TEG ID, Rx TEG ID, Tx TEG ID}".

#### 6.5.12.5	NR Multi-RTT Location Information Request

#### –	NR-Multi-RTT-RequestLocationInformation

The IE NR-Multi-RTT-RequestLocationInformation is used by the location server to request NR Multi-RTT location measurements from a target device.

-- ASN1START

NR-Multi-RTT-RequestLocationInformation-r16 ::= SEQUENCE {

nr-UE-RxTxTimeDiffMeasurementInfoRequest-r16

ENUMERATED { true }					OPTIONAL, -- Need ON

nr-RequestedMeasurements-r16		BIT STRING { prsrsrpReq(0),

firstPathRsrpReq-r17(1) } (SIZE(1..8)),

nr-AssistanceAvailability-r16		BOOLEAN,

nr-Multi-RTT-ReportConfig-r16		NR-Multi-RTT-ReportConfig-r16,

additionalPaths-r16					ENUMERATED { requested }			OPTIONAL, -- Need ON

...,

[[

nr-UE-RxTxTEG-Request-r17			ENUMERATED { case1, case2, case3, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

measureSameDL-PRS-ResourceWithDifferentRxTEGs-r17

ENUMERATED { n0, n2, n3, n4, n6, n8, ... }

OPTIONAL, -- Need ON

requestedDL-PRS-ProcessingSamples-r17

ENUMERATED { m1, ... }				OPTIONAL, -- Need ON

nr-los-nlos-IndicatorRequest-r17	SEQUENCE {

type-r17		ENUMERATED {hardvalue,softvalue},

granularity-r17	ENUMERATED {trpspecific,resourcespecific},

...

}									OPTIONAL, -- Need ON

additionalPathsExt-r17				ENUMERATED { requested }			OPTIONAL, -- Need ON

additionalPathsDL-PRS-RSRP-Request-r17

ENUMERATED { requested }			OPTIONAL  -- Need ON

]]

}

NR-Multi-RTT-ReportConfig-r16 ::= SEQUENCE {

maxDL-PRS-RxTxTimeDiffMeasPerTRP-r16 	INTEGER (1..4)					OPTIONAL, -- Need ON

timingReportingGranularityFactor-r16 	INTEGER (0..5)					OPTIONAL	-- Need ON

}

-- ASN1STOP

NR-Multi-RTT-RequestLocationInformation field descriptions

nr-UE-RxTxTimeDiffMeasurementInfoRequest This field, if present, indicates that the target device is requested to report the DL-PRS Resource ID(s) or DL-PRS Resource Set ID(s) associated with the DL-PRS Resources(s) or the DL-PRS Resource Set(s) which are used in determining the UE Rx-Tx time difference measurements.

nr-AssistanceAvailability This field indicates whether the target device may request additional PRS assistance data from the server. TRUE means allowed and FALSE means not allowed.

maxDL-PRS-RxTxTimeDiffMeasPerTRP This field specifies the maximum number of UE-Rx-Tx time difference measurements for different DL-PRS Resources or DL-PRS Resource Sets per TRP.

timingReportingGranularityFactor This field specifies the recommended reporting granularity for the UE Rx-Tx time difference measurements. Value (0..5) corresponds to (k0..k5) used for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional in NR-Multi-RTT-MeasElement. The UE may select a different granularity value for nr-UE-RxTxTimeDiff and nr-UE-RxTxTimeDiffAdditional.

additionalPaths This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathList in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPathsExt shall be absent.

nr-UE-RxTxTEG-Request This field, if present, indicates that the target device is requested to provide the NR-UE-RxTx-TEG-Info in IE NR-Multi-RTT-SignalMeasurementInformation. Enumerated value 'case1' indicates that the target device is requested to provide the case1 choice in NR-UE-RxTx-TEG-Info, enumerated value 'case2' indicates that the target device is requested to provide the case2 choice in NR-UE-RxTx-TEG-Info, and so on.

measureSameDL-PRS-ResourceWithDifferentRxTxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE RxTx TEGs and with the same UE Tx TEG. Enumerated value 'n0' indicates that the number N of different UE RxTx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE RxTx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE RxTx TEGs, and so on. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTEGs should not be present.

measureSameDL-PRS-ResourceWithDifferentRxTEGs This field, if present, indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with N different UE Rx TEGs. Enumerated value 'n0' indicates that the number N of different UE Rx TEGs to measure the same DL PRS Resource can be determined by the target device, value 'n2' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 2 different UE Rx TEGs, value 'n3' indicates that the target device is requested to measure the same DL-PRS Resource of a TRP with 3 different UE Rx TEGs, and so on. If this field is present, the field nr-UE-RxTxTEG-Request should also be present. If this field is present, the field measureSameDL-PRS-ResourceWithDifferentRxTxTEGs should not be present.

requestedDL-PRS-ProcessingSamples This field, if present, indicates the requested number of DL-PRS processing samples. Enumerated value 'm1' indicates 1-sample DL-PRS processing is requested as defined in TS 38.133 [46].

nr-los-nlos-IndicatorRequest This field, if present, indicates that the target device is requested to provide the indicated type and granularity of the estimated LOS-NLOS-Indicator in the NR-Multi-RTT-SignalMeasurementInformation.

additionalPathsExt This field, if present, indicates that the target device is requested to provide the nr-AdditionalPathListExt in IE NR-Multi-RTT-SignalMeasurementInformation. If this field is present, the field additionalPaths shall be absent.

additionalPathsDL-PRS-RSRP-Request This field, if present, indicates that the target device is requested to provide the nr-DL-PRS-RSRP for the additional paths in IE NR-AdditionalPathList.

#### 6.5.12.6	NR Multi-RTT Capability Information

#### –	NR-Multi-RTT-ProvideCapabilities

The IE NR-Multi-RTT-ProvideCapabilities is used by the target device to indicate its capability to support NR Multi-RTT and to provide its NR Multi-RTT positioning capabilities to the location server.

-- ASN1START

NR-Multi-RTT-ProvideCapabilities-r16 ::= SEQUENCE {

nr-Multi-RTT-PRS-Capability-r16			NR-DL-PRS-ResourcesCapability-r16,

nr-Multi-RTT-MeasurementCapability-r16	NR-Multi-RTT-MeasurementCapability-r16,

nr-DL-PRS-QCL-ProcessingCapability-r16	NR-DL-PRS-QCL-ProcessingCapability-r16,

nr-DL-PRS-ProcessingCapability-r16		NR-DL-PRS-ProcessingCapability-r16,

nr-UL-SRS-Capability-r16				NR-UL-SRS-Capability-r16,

additionalPathsReport-r16				ENUMERATED { supported }					OPTIONAL,

periodicalReporting-r16					ENUMERATED { supported }					OPTIONAL,

...,

[[

ten-ms-unit-ResponseTime-r17			ENUMERATED { supported }					OPTIONAL,

nr-Multi-RTT-On-Demand-DL-PRS-Support-r17

NR-On-Demand-DL-PRS-Support-r17				OPTIONAL,

nr-UE-RxTx-TEG-ID-ReportingSupport-r17	BIT STRING {	case1 	(0),

case2	(1),

case3	(2)

}	(SIZE (1..8))				OPTIONAL,

nr-los-nlos-IndicatorSupport-r17		SEQUENCE {

type-r17		ENUMERATED { hardvalue, softvalue, both },

granularity-r17	ENUMERATED { trpspecific, resourcespecific, both},

...

}														OPTIONAL,	additionalPathsExtSupport-r17			ENUMERATED { n4, n6, n8 }					OPTIONAL,

additionalPathsPowerSupport-r17			ENUMERATED { supported }					OPTIONAL,

scheduledLocationRequest-r17			SEQUENCE {

utcTime-r17		 ENUMERATED { supported }	OPTIONAL,

gnssTime-r17	 GNSS-ID-Bitmap				OPTIONAL,

e-utraTime-r17	 ENUMERATED { supported }	OPTIONAL,

nrTime-r17		 ENUMERATED { supported }	OPTIONAL,

relativeTime-r17 ENUMERATED { supported }	OPTIONAL,

...

}											OPTIONAL,

nr-dl-prs-AssistanceDataValidity-r17	SEQUENCE {

area-validity-r17	INTEGER (1..maxAreaIDs-r17)			OPTIONAL,

...

}											OPTIONAL,

multiMeasInSameMeasReport-r17			ENUMERATED { supported }					OPTIONAL,

mg-ActivationRequest-r17				ENUMERATED { supported }					OPTIONAL,

nr-DL-PRS-ProcessingRRC-Inactive-r17	ENUMERATED { supported }					OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-ProvideCapabilities field descriptions

ten-ms-unit-ResponseTime This field, if present, indicates that the target device supports the enumerated value 'ten-milli-seconds' in the IE ResponseTime in IE CommonIEsRequestLocationInformation.

nr-Multi-RTT-On-Demand-DL-PRS-Support This field, if present, indicates that the target device supports on-demand DL-PRS requests.

nr-UE-RxTx-TEG-ID-ReportingSupport This field, if present, indicates that the target device supports nr-UE-RxTx-TEG-Info reporting in IE NR-Multi-RTT-SignalMeasurementInformation. This is represented by a bit string, with a one-value at the bit position means the particular case is supported; a zero-value means not supported: - bit 0 indicates that the target device supports the 'case1' choice in NR-UE-RxTx-TEG-Info. - bit 1 indicates that the target device supports the 'case2' choice in NR-UE-RxTx-TEG-Info. - bit 2 indicates that the target device supports the 'case3' choice in NR-UE-RxTx-TEG-Info.

nr-los-nlos-IndicatorSupport This field, if present, indicates that the target device supports nr-los-nlos-Indicator reporting in IE NR-Multi-RTT-SignalMeasurementInformation. - type indicates whether the target device supports 'hard' value, 'soft' value, or both in IE LOS-NLOS-Indicator. - granularity indicates whether the target device supports LOS-NLOS-Indicator reporting per TRP, per DL-PRS Resource, or both.

additionalPathsExtSupport This field, if present, indicates that the target device supports the nr-AdditionalPathListExt reporting in IE NR-Multi-RTT-SignalMeasurementInformation. The enumerated value indicates the number of additional paths supported by the target device,

additionalPathsPowerSupport This field, if present, indicates that the target device supports the nr-DL-PRS-RSRP for the additional paths in IE NR-AdditionalPathList.

scheduledLocationRequest This field, if present, indicates that the target device supports scheduled location requests – i.e., supports the IE ScheduledLocationRequest in IE CommonIEsRequestLocationInformation – and the time base(s) supported for the scheduled location time.

nr-dl-prs-AssistanceDataValidity This field, if present, indicates that the target device supports validity conditions for pre-configured assistance data and comprises the following subfields: - area-validity indicates that the target device supports pre-configured assistance data with area validity. The integer number indicates the maximum number of area IDs the target device supports.

multiMeasInSameMeasReport This field, if present, indicates that the target device supports multiple measurement instances in a single measurement report.

mg-ActivationRequest This field, if present, indicates that the target device supports low latency measurement gap activation request for PRS measurements.

nr-DL-PRS-ProcessingRRC-Inactive This field, if present, indicates that the target device supports DL-PRS processing in RRC_INACTIVE state.

#### 6.5.12.6a	NR Multi-RTT Capability Information Elements

#### –	NR-Multi-RTT-MeasurementCapability

The IE NR-Multi-RTT-MeasurementCapability defines the Multi-RTT measurement capability. The UE can include this IE only if the UE supports NR-DL-PRS-ResourcesCapability for Multi-RTT. Otherwise, the UE does not include this IE;

-- ASN1START

NR-Multi-RTT-MeasurementCapability-r16 ::= SEQUENCE {

maxNrOfRx-TX-MeasFR1-r16					INTEGER (1..4)	OPTIONAL,

maxNrOfRx-TX-MeasFR2-r16					INTEGER (1..4)	OPTIONAL,

supportOfRSRP-MeasFR1-r16					ENUMERATED { supported}	OPTIONAL,

supportOfRSRP-MeasFR2-r16					ENUMERATED { supported}	OPTIONAL,

srs-AssocPRS-MultiLayersFR1-r16				ENUMERATED { supported}	OPTIONAL,

srs-AssocPRS-MultiLayersFR2-r16				ENUMERATED { supported}	OPTIONAL,

...,

[[

supportOfDL-PRS-FirstPathRSRP-MeasFR1-r17	ENUMERATED { supported}		OPTIONAL,

supportOfDL-PRS-FirstPathRSRP-MeasFR2-r17	ENUMERATED { supported}		OPTIONAL,

nr-UE-TEG-Capability-r17					NR-UE-TEG-Capability-r17	OPTIONAL,

dl-PRS-MeasRRC-Inactive-r17					ENUMERATED { supported }	OPTIONAL

]]

}

-- ASN1STOP

NR-Multi-RTT-MeasurementCapability field descriptions

maxNrOfRx-TX-MeasFR1 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS resource/resource set on FR1.

maxNrOfRx-TX-MeasFR2 Indicates the maximum number of UE Rx–Tx time difference measurements corresponding to a single SRS resource/resource set for positioning with each measurement corresponding to a single DL-PRS resource/resource set on FR2.

srs-AssocPRS-MultiLayersFR1 Indicates whether the UE supports measurements derived on one or more DL-PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands. This is for FR1 only.

srs-AssocPRS-MultiLayersFR2 Indicates whether the UE supports measurements derived on one or more DL-PRS resource/resource sets which may be in different positioning frequency layers for SRS transmitted in a single CC. PRS and SRS may be on different bands. This is for FR2 only.

supportOfRSRP-MeasFR1 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR1.

supportOfRSRP-MeasFR2 Indicates whether the UE supports RSRP measurement for Multi-RTT on FR2.

supportOfDL-PRS-FirstPathRSRP-MeasFR1 Indicates whether the target device supports DL-PRS First Path RSRP measurement for Multi-RTT on FR1.

supportOfDL-PRS-FirstPathRSRP-MeasFR2 Indicates whether the target device supports DL-PRS First Path RSRP measurement for Multi-RTT on FR2.

nr-UE-TEG-Capability Indicates the UE TEG capability.

dl-PRS-MeasRRC-Inactive This field, if present, indicates that the target device supports DL-PRS measurement in RRC_INACTIVE state.

Editor's Note: Measurement Capabilities are FFS and require further RAN1 input.

Editor's Note: FFS on FR1/FR2 differentiation for RSRPP.

#### 6.5.12.7	NR Multi-RTT Capability Information Request

#### –	NR-Multi-RTT-RequestCapabilities

The IE NR-Multi-RTT-RequestCapabilities is used by the location server to request the capability of the target device to support NR Multi-RTT and to request NR Multi-RTT positioning capabilities from a target device.

-- ASN1START

NR-Multi-RTT-RequestCapabilities-r16 ::= SEQUENCE {

...

}

-- ASN1STOP

#### 6.5.12.8	NR Multi-RTT Error Elements

#### –	NR-Multi-RTT-Error

The IE NR-Multi-RTT-Error is used by the location server or target device to provide NR Multi-RTT error reasons to the target device or location server, respectively.

-- ASN1START

NR-Multi-RTT-Error-r16 ::= CHOICE {

locationServerErrorCauses-r16		NR-Multi-RTT-LocationServerErrorCauses-r16,

targetDeviceErrorCauses-r16			NR-Multi-RTT-TargetDeviceErrorCauses-r16,

...

}

-- ASN1STOP

#### –	NR-Multi-RTT-LocationServerErrorCauses

The IE NR-Multi-RTT-LocationServerErrorCauses is used by the location server to provide NR Multi-RTT error reasons to the target device.

-- ASN1START

NR-Multi-RTT-LocationServerErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED	{	undefined,

assistanceDataNotSupportedByServer,

assistanceDataSupportedButCurrentlyNotAvailableByServer,

...,

on-demand-dl-prs-NotSupportedByServer-r17,

on-demand-dl-prs-SupportedButCurrentlyNotAvailableByServer-r17

},

...

}

-- ASN1STOP

#### –	NR-Multi-RTT-TargetDeviceErrorCauses

The IE NR-Multi-RTT-TargetDeviceErrorCauses is used by the target device to provide NR Multi-RTT error reasons to the location server.

-- ASN1START

NR-Multi-RTT-TargetDeviceErrorCauses-r16 ::= SEQUENCE {

cause-r16		ENUMERATED {	undefined,

dl-assistance-data-missing,

unableToMeasureAnyTRP,

attemptedButUnableToMeasureSomeNeighbourTRPs,

ul-srs-configuration-missing,

unableToTransmit-ul-srs,

...

},

...

}

-- ASN1STOP

## 6.6	Multiplicity and type constraint values

#### –	Multiplicity and type constraint definitions

-- ASN1START

maxEARFCN					INTEGER ::= 65535	-- Maximum value of EUTRA carrier frequency

maxEARFCN-Plus1				INTEGER ::= 65536	-- Lowest value extended EARFCN range

maxEARFCN2					INTEGER ::= 262143	-- Highest value extended EARFCN range

maxMBS-r14					INTEGER ::= 64

maxWLAN-AP-r13				INTEGER ::= 64

maxKnownAPs-r14				INTEGER ::= 2048

maxVisibleAPs-r14			INTEGER ::= 32

maxWLAN-AP-r14				INTEGER ::= 128

maxWLAN-DataSets-r14		INTEGER ::= 8

maxBT-Beacon-r13			INTEGER ::= 32

nrMaxBands-r16							INTEGER ::= 1024	-- Maximum number of supported bands in

-- UE capability.

nrMaxFreqLayers-r16						INTEGER ::= 4		-- Max freq layers

nrMaxFreqLayers-1-r16					INTEGER ::= 3

nrMaxNumDL-PRS-ResourcesPerSet-1-r16 	INTEGER ::= 63

nrMaxNumDL-PRS-ResourceSetsPerTRP-1-r16	INTEGER ::= 7

nrMaxResourceIDs-r16					INTEGER ::= 64		-- Max Resource IDs

nrMaxResourceOffsetValue-1-r16 			INTEGER ::= 511

nrMaxResourcesPerSet-r16				INTEGER ::= 64		-- Maximum resources for one set

nrMaxSetsPerTrpPerFreqLayer-r16			INTEGER ::= 2		-- Maximum resource sets for one TRP

nrMaxSetsPerTrpPerFreqLayer-1-r16		INTEGER ::= 1

nrMaxTRPs-r16							INTEGER ::= 256		-- Max TRPs per UE

nrMaxTRPsPerFreq-r16					INTEGER ::= 64		-- Max TRPs per freq layers

nrMaxTRPsPerFreq-1-r16					INTEGER ::= 63

maxSimultaneousBands-r16				INTEGER ::= 4		-- Maximum number of simultaneously

-- measured bands

maxBandComb-r16							INTEGER ::= 1024

nrMaxConfiguredBands-r16				INTEGER ::= 16

maxNumOfRxTEGs-1-r17					INTEGER ::= 31	-- FFS

maxNumOfTxTEGs-1-r17					INTEGER ::= 7	-- FFS

maxTxTEG-Sets-r17						INTEGER ::= 64	-- FFS 8 TxTEGs and max 8 time stamps

maxNumOfRxTxTEGs-1-r17					INTEGER ::= 255	-- FFS

maxNumOfTRP-TxTEGs-1-r17				INTEGER ::= FFS

maxNumOfSRS-PosResourceSets-r17			INTEGER ::= 16

maxNumOfSRS-PosResourceSets-1-r17		INTEGER ::= 15

maxNumOfSRS-PosResources-r17            INTEGER ::= 64

maxNumOfSRS-PosResources-1-r17          INTEGER ::= 63

maxNumResourcesPerAngle-r17				INTEGER ::= 24 -- FFS

maxNumPrioResources-r17					INTEGER ::= 24 -- FFS

maxAddMeasTDOA-r17						INTEGER ::= 31 --  (4x8)-1 FFS

maxAddMeasAoD-r17						INTEGER ::= 24 -- FFS

maxAddMeasRTT-r17						INTEGER ::= 31 --  (4x8)-1 FFS

maxDL-PRS-Configs-r17					INTEGER ::=	8

maxCellIDsIDsPerArea-r17				INTEGER ::= FFS

maxAreaIDs-r17							INTEGER ::= FFS

-- ASN1STOP

#### –	End of LPP-PDU-Definitions

-- ASN1START

END

-- ASN1STOP

# 7	Broadcast of assistance data

## 7.1	General

Broadcast of positioning assistance data is supported via Positioning System Information Blocks (posSIBs) as specified in TS 36.331 [12] or TS 38.331 [35]. The posSIBs are carried in RRC System Information (SI) messages (TS 36.331 [12] or TS 38.331 [35]).

For LTE RRC System Information (SI), a single SystemInformationBlockPos IE is defined in TS 36.331 [12] which is carried in IE PosSystemInformation-r15-IEs specified in TS 36.331 [12]. The mapping of positioning SIB type (posSibType) to assistance data carried in SystemInformationBlockPos is specified in clause 7.2.

For NR RRC System Information (SI), a single SIBpos IE is defined in TS 38.331 [35] which is carried in IE PosSystemInformation-r16-IEs specified in TS 38.331 [35]. The mapping of positioning SIB type (posSibType) to assistance data carried in SIBpos is specified in clause 7.2.

## 7.2	Mapping of posSibType to assistance data element

The supported posSibType's are specified in Table 7.2-1. The GNSS Common and Generic Assistance Data IEs are defined in clause 6.5.2.2. The OTDOA Assistance Data IEs and NR DL-TDOA/DL-AoD Assistance Data IEs are defined in clause 7.4.2. The Barometric Assistance Data IEs are defined in clause 6.5.5.8. The TBS (based on MBS signals) Assistance Data IEs are defined in clause 6.5.4.8.

Table 7.2-1: Mapping of posSibType to assistanceDataElement

|  | posSibType | assistanceDataElement |
| --- | --- | --- |
| GNSS Common Assistance Data (clause 6.5.2.2) | posSibType1-1 | GNSS-ReferenceTime |
|  | posSibType1-2 | GNSS-ReferenceLocation |
|  | posSibType1-3 | GNSS-IonosphericModel |
|  | posSibType1-4 | GNSS-EarthOrientationParameters |
|  | posSibType1-5 | GNSS-RTK-ReferenceStationInfo |
|  | posSibType1-6 | GNSS-RTK-CommonObservationInfo |
|  | posSibType1-7 | GNSS-RTK-AuxiliaryStationData |
|  | posSibType1-8 | GNSS-SSR-CorrectionPoints |
|  | posSibType1-9 | GNSS-Integrity-ServiceParameters |
|  | posSibType1-10 | GNSS-Integrity-ServiceAlert |
| GNSS Generic Assistance Data (clause 6.5.2.2) | posSibType2-1 | GNSS-TimeModelList |
|  | posSibType2-2 | GNSS-DifferentialCorrections |
|  | posSibType2-3 | GNSS-NavigationModel |
|  | posSibType2-4 | GNSS-RealTimeIntegrity |
|  | posSibType2-5 | GNSS-DataBitAssistance |
|  | posSibType2-6 | GNSS-AcquisitionAssistance |
|  | posSibType2-7 | GNSS-Almanac |
|  | posSibType2-8 | GNSS-UTC-Model |
|  | posSibType2-9 | GNSS-AuxiliaryInformation |
|  | posSibType2-10 | BDS-DifferentialCorrections |
|  | posSibType2-11 | BDS-GridModelParameter |
|  | posSibType2-12 | GNSS-RTK-Observations |
|  | posSibType2-13 | GLO-RTK-BiasInformation |
|  | posSibType2-14 | GNSS-RTK-MAC-CorrectionDifferences |
|  | posSibType2-15 | GNSS-RTK-Residuals |
|  | posSibType2-16 | GNSS-RTK-FKP-Gradients |
|  | posSibType2-17 | GNSS-SSR-OrbitCorrections |
|  | posSibType2-18 | GNSS-SSR-ClockCorrections |
|  | posSibType2-19 | GNSS-SSR-CodeBias |
|  | posSibType2-20 | GNSS-SSR-URA |
|  | posSibType2-21 | GNSS-SSR-PhaseBias |
|  | posSibType2-22 | GNSS-SSR-STEC-Correction |
|  | posSibType2-23 | GNSS-SSR-GriddedCorrection |
|  | posSibType2-24 | NavIC-DifferentialCorrections |
|  | posSibType2-25 | NavIC-GridModelParameter |
| OTDOA Assistance Data (clause 7.4.2) | posSibType3-1 | OTDOA-UE-Assisted |
| Barometric Assistance Data (clause 6.5.5.8) | posSibType4-1 | Sensor-AssistanceDataList |
| TBS Assistance Data (clause 6.5.4.8) | posSibType5-1 | TBS-AssistanceDataList |
| NR DL-TDOA/DL-AoD Assistance Data (clauses 6.4.3, 7.4.2) | posSibType6-1 | NR-DL-PRS-AssistanceData |
|  | posSibType6-2 | NR-UEB-TRP-LocationData |
|  | posSibType6-3 | NR-UEB-TRP-RTD-Info |
|  | posSibType6-4 | NR-TRP-BeamAntennaInfo |
|  | posSibType6-5 | NR-DL-PRS-TRP-TEG-Info |
| On-demand DL-PRS Configurations (clause 6.4.3) | posSibType6-6 | NR-On-Demand-DL-PRS-Configurations |

Editor's Note: FFS on additional posSibType's.

## 7.3	Procedures related to broadcast information elements

Upon receiving AssistanceDataSIBelement, the target device shall:

1>	if the segmentationInfo is not included:

2>	if the cipheringKeyData is included:

3>	if the UE has obtained a valid cipher key value and the first portion of the initial Counter denoted C0 corresponding to the cipherSetID using NAS signalling:

4>	if C0 contains less than 128-bits:

5>	pad out the bit string with zeroes in most significant bit positions to achieve 128 bits.

4>	if the d0 field contains less than 128-bits:

5>	pad out the bit string with zeroes in least significant bit positions to achieve 128 bits, denoted D0.

4>	determine the initial Counter C1 = (C0 + D0) mod 2128 (where all values are treated as non-negative integers);

4>	determine any subsequent counter Ci from the previous counter Ci-1 as Ci = (Ci-1 + 1) mod 2128;

4>	use the sequence of counters <C1, C2, C3, …> and the cipher key value to decipher the assistanceDataElement;

4>	decode the deciphered assistanceDataElement and deliver the related assistance data to upper layers.

3>	else:

4>	discard the AssistanceDataSIBelement.

2>	else:

3>	decode the assistanceDataElement and deliver the related assistance data to upper layers.

1>	else:

2>	if segmentationOption indicates 'pseudo-seg':

3>	if the cipheringKeyData is included:

4>	if the UE has obtained a valid cipher key value and the first portion of the initial Counter denoted C0 corresponding to the cipherSetID using NAS signalling:

5>	if C0 contains less than 128-bits:

6>	pad out the bit string with zeroes in most significant bit positions to achieve 128 bits.

5>	if the d0 field contains less than 128-bits:

6>	pad out the bit string with zeroes in least significant bit positions to achieve 128 bits, denoted D0.

5>	determine the initial Counter C1 = (C0 + D0) mod 2128 (where all values are treated as non-negative integers);

5>	determine any subsequent counter Ci from the previous counter Ci-1 as Ci = (Ci-1 + 1) mod 2128;

5>	use the sequence of counters <C1, C2, C3, …> and the cipher key value to decipher the assistanceDataElement segment;

5>	decode the deciphered assistanceDataElement segment and deliver the related assistance data portion together with the assistanceDataSegmentType and assistanceDataSegmentNumber to upper layers.

4>	else:

5>	discard the AssistanceDataSIBelement segment.

3>	else:

4>	decode the assistanceDataElement segment and deliver the related assistance data portion together with the assistanceDataSegmentType and assistanceDataSegmentNumber to upper layers.

2>	if segmentationOption indicates 'octet-string-seg':

3>	if all segments of assistanceDataElement have been received:

4>	assemble the assistance data element from the received assistanceDataElement segments;

5>	if the cipheringKeyData is included in the first segment:

6>	if the UE has obtained a valid cipher key value and the first portion of the initial Counter denoted C0 corresponding to the cipherSetID using NAS signalling:

7>	if C0 contains less than 128-bits:

8>	pad out the bit string with zeroes in most significant bit positions to achieve 128 bits.

7>	if the d0 field contains less than 128-bits:

8>	pad out the bit string with zeroes in least significant bit positions to achieve 128 bits, denoted D0.

7>	determine the initial Counter C1 = (C0 + D0) mod 2128 (where all values are treated as non-negative integers);

7>	determine any subsequent counter Ci from the previous counter Ci-1 as Ci = (Ci-1 + 1) mod 2128;

7>	use the sequence of counters <C1, C2, C3, …> and the cipher key value to decipher the assembled assistance data element;

7>	decode the assembled and deciphered assistance data element and deliver the related assistance data to upper layers.

6>	else:

7>	discard the assembled assistance data element.

5>	else:

6>	decode the assembled assistance data element and deliver the related assistance data to upper layers.

NOTE:	As an optional optimisation when segmentationOption indicates 'octet-string-seg', a target device may verify if the cipheringKeyData is included in the first segment as soon as the first segment is received and, if included, may verify that the UE has obtained a valid cipher key value and the first portion of the initial Counter denoted C0 corresponding to the cipherSetID using NAS signalling. When the UE has not obtained a valid cipher key value and initial Counter C0 using NAS signalling, the UE may discard the first segment and ignore all subsequent segments.

The value for D0 shall be different for different AssistanceDataSIBelement's to ensure that the counters derived from C1 for any assistanceDataElement are different to the counters for any other assistanceDataElement for a given ciphering key.

D0 shall contain at least 16 least significant bits (LSBs) set to zero to ensure that the values of D0 differ from another by a large value.

## 7.4	Broadcast information elements

### 7.4.1	Basic production

This clause defines the LPP broadcast information elements which are encoded as 'basic production' for system information broadcast purposes (see TS 36.331 [12], TS 38.331 [35]).

The 'basic production' is obtained from their ASN.1 definitions by use of Basic Packed Encoding Rules (BASIC-PER), Unaligned Variant, as specified in ITU-T Rec. X.691 [22]. It always contains a multiple of 8 bits.

#### –	LPP-Broadcast-Definitions

This ASN.1 segment is the start of the LPP Broadcast definitions.

-- ASN1START

LPP-Broadcast-Definitions

DEFINITIONS AUTOMATIC TAGS ::=

BEGIN

IMPORTS

OTDOA-ReferenceCellInfo,

OTDOA-NeighbourCellInfoList,

NR-TRP-LocationInfo-r16,

NR-DL-PRS-BeamInfo-r16,

NR-RTD-Info-r16

FROM LPP-PDU-Definitions;

-- ASN1STOP

### 7.4.2	Element definitions

#### –	AssistanceDataSIBelement

The IE AssistanceDataSIBelement is used in the IE SystemInformationBlockPos as specified in TS 36.331 [12] and IE SIBpos as specified in TS 38.331 [35].

-- ASN1START

AssistanceDataSIBelement-r15 ::= SEQUENCE {

valueTag-r15						INTEGER (0..63)					OPTIONAL, -- Need OP

expirationTime-r15					UTCTime							OPTIONAL, -- Need OP

cipheringKeyData-r15				CipheringKeyData-r15 			OPTIONAL, -- Need OP

segmentationInfo-r15				SegmentationInfo-r15			OPTIONAL, -- Need OP

assistanceDataElement-r15			OCTET STRING,

...

}

CipheringKeyData-r15 ::= SEQUENCE {

cipherSetID-r15 					INTEGER (0..65535),

d0-r15 								BIT STRING (SIZE (1..128)),

...

}

SegmentationInfo-r15 ::= SEQUENCE {

segmentationOption-r15				ENUMERATED {pseudo-seg, octet-string-seg},

assistanceDataSegmentType-r15		ENUMERATED {notLastSegment, lastSegment},

assistanceDataSegmentNumber-r15		INTEGER (0..63),

...

}

-- ASN1STOP

AssistanceDataSIBelement field descriptions

valueTag This field is used to indicate to the target device any changes in the broadcast assistance data content. The valueTag is incremented by one, by the location server, every time a modified assistance data content is provided. This field is not included if the broadcast assistance data changes too frequently. If valueTag and expirationTime are absent, the UE assumes that the broadcast assistance data content changes at every broadcast interval.

expirationTime This field indicates how long the broadcast assistance data content is valid. It is specified as UTC time and indicates when the broadcast assistance data content will expire.

cipheringKeyData If present, indicates that the assistanceDataElement octet string is ciphered.

segmentationInfo If present, indicates that the assistanceDataElement is one of many segments.

assistanceDataElement The assistanceDataElement OCTET STRING depends on the posSibType and is specified in Table 7.2-1. NOTE.

cipherSetID This field identifies a cipher set comprising a cipher key value and the first component C0 of the initial counter C1.

d0 This field provides the second component for the initial ciphering counter C1. This field is defined as a bit string with a length of 1 to 128 bits. A target device first pads out the bit string if less than 128 bits with zeroes in least significant bit positions to achieve 128 bits. C1 is then obtained from D0 and C0 (defined by the cipherSetID) as: C1 = (D0 + C0) mod 2128 (with all values treated as non-negative integers).

segmentationOption Indicates the used segmentation option.

assistanceDataSegmentType Indicates whether the included assistanceDataElement segment is the last segment or not.

assistanceDataSegmentNumber Segment number of the assistanceDataElement segment. A segment number of zero corresponds to the first segment, one corresponds to the second segment, and so on. Segments numbers wraparound should there be more than 64 segments

NOTE:	For example, if the posSibType in IE PosSIB-Type defined in TS 36.331 [12] and TS 38.331 [35] indicates 'posSibType1-7', the assistanceDataElement OCTET STRING includes the LPP IE GNSS-RTK-AuxiliaryStationData.

#### –	OTDOA-UE-Assisted

The IE OTDOA-UE-Assisted is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 36.331 [12] indicates 'posSibType3-1'.

-- ASN1START

OTDOA-UE-Assisted-r15 ::= SEQUENCE {

otdoa-ReferenceCellInfo-r15			OTDOA-ReferenceCellInfo,

otdoa-NeighbourCellInfo-r15			OTDOA-NeighbourCellInfoList,

...

}

-- ASN1STOP

OTDOA-UE-Assisted field descriptions

otdoa-ReferenceCellInfo LPP IE OTDOA-ReferenceCellInfo as defined in clause 6.5.1.2.

otdoa-NeighbourCellInfo LPP IE OTDOA-NeighbourCellInfoList as defined in clause 6.5.1.2.

#### –	NR-UEB-TRP-LocationData

The IE NR-UEB-TRP-LocationData is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 38.331 [35] indicates 'posSibType6-2'.

-- ASN1START

NR-UEB-TRP-LocationData-r16 ::= SEQUENCE {

nr-trp-LocationInfo-r16 			NR-TRP-LocationInfo-r16,

nr-dl-prs-BeamInfo-r16				NR-DL-PRS-BeamInfo-r16			OPTIONAL,	-- Need ON

...

}

-- ASN1STOP

NR-UEB-TRP-LocationData field descriptions

nr-trp-LocationInfo LPP IE NR-TRP-LocationInfo as defined in clause 6.4.2.1.

nr-dl-prs-BeamInfo LPP IE NR-DL-PRS-Beam-Info as defined in clause 6.4.2.1.

#### –	NR-UEB-TRP-RTD-Info

The IE NR-UEB-TRP-RTD-Info is used in the assistanceDataElement if the posSibType in IE PosSIB-Type defined in TS 38.331 [35] indicates 'posSibType6-3'.

-- ASN1START

NR-UEB-TRP-RTD-Info-r16 ::= SEQUENCE {

nr-rtd-Info-r16						NR-RTD-Info-r16,

...

}

-- ASN1STOP

NR-UEB-TRP-RTD-Info field descriptions

nr-rtd-Info LPP IE NR-RTD-Info as defined in clause 6.4.2.1.

#### –	End of LPP-Broadcast-Definitions

-- ASN1START

END

-- ASN1STOP

## 7.5	Broadcast ciphering (informative)

The assistanceDataElement OCTET STRING included in IE AssistanceDataSIBelement may be ciphered using the 128-bit Advanced Encryption Standard (AES) algorithm (with counter mode).

AES as specified in [32] and [33] is a block mode cipher algorithm that ciphers blocks of 128 bits at a time. However, Counter mode enables usage for a bit string that is not an exact multiple of 128 bits. Further, Counter mode enables a target (or a server) to perform most of the deciphering (or ciphering) processing independently of receipt of the data to be deciphered (or ciphered) which may enable more efficient processing. Provided counters are chosen in a non-repeating manner by the server (which is a requirement for Counter mode), every block of data will be ciphered in a unique manner.

The algorithm makes use of a sequence of counters <C1, C2, C3, …> each containing 128 bits, where C1 is specified by the server and each subsequent counter (C2, C3 etc.) is obtained from the previous counter by adding one modulo 2128. Each counter Ci is ciphered using the AES algorithm with a common 128-bit key to produce an output block Oi of 128 bits. To perform ciphering, the assistanceDataElement is divided into blocks B1, B2, … Bn of 128 bits each, except for the last block Bn which may contain fewer than 128 bits. The ciphered assistanceDataElement is obtained as a sequence of n blocks containing 128 bits each (except possibly for the last block) given by (O1 XOR B1), (O2 XOR B2), … (On XOR Bn), where XOR denotes bitwise exclusive OR. In the case of the last block, if Bn contains m bits (m<128), then the m most significant bits of On would be used for the exclusive OR. Deciphering is performed in the same way except that the blocks B1, B2, … Bn are now obtained from the ciphered message and the result of the exclusive OR operations yields the original unciphered message. Figure 7.5-1 provides an illustration of Counter mode for the generic case of an arbitrary block cipher algorithm CIPHk.

Figure 7.5-1: Illustration of Block Ciphering with Counter Mode [33].

The algorithms require specific conventions for bit ordering. The bit ordering applicable to ciphering for a ASN.1 PER encoded assistanceDataElement is the bit ordering produced by the ASN.1 PER encoding where the first bit is the leading bit number zero, the second bit is bit one etc..

The initial counter C1 used to cipher an entire assistanceDataElement is provided to a target by a server in two portions. The first portion, denoted C0, is provided using point to point mode along with the 128-bit ciphering key and an identifier for both of these values as specified in TS 23.271 [3]. The second portion, denoted D0 (d0), is provided in unciphered form in IE CipheringKeyData. A target then obtains C1 as:

C1 = (C0 + D0) mod 2128	(where all values are treated as non-negative integers)

To obtain any subsequent counter Ci from the previous counter Ci1 for any message, the following operation is used:

Ci = (Ci1 + 1) mod 2128

NOTE:	As specified in clause 7.3 the value for D0 is different for different AssistanceDataSIBelement's to ensure that the counters derived from C1 for any assistanceDataElement can be different to the counters for any other assistanceDataElement. However, a long assistanceDataElement or a segmented assistanceDataElement would require the use of consecutive counter values C1 to Cn, where n is the assistanceDataElement size in bits divided by 128 (and rounded up). There would then be a danger of small changes in the value of D0 for ciphering of different assistanceDataElement's (e.g. D0 being chosen as 1 larger than a previous D0 value) reusing previous counter values. To avoid this, the values of D0 contain 16 least significant bits (LSBs) set to zero, as specified in clause 7.3.

Annex A (informative):
Change History

| Change history of TS 36.355 up to v15.5.0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Cat | Subject/Comment | New version |
| 2009-10 | RAN2 #67bis | R2-096252 |  |  |  | RAN2 agreed TS 36.355 v0.1.0 | 0.1.0 |
| 2009-11 | RAN2 #68 | R2-097492 |  |  |  | RAN2 agreed TS 36.355 v2.0.0 | 2.0.0 |
| 2009-12 | RP-46 | RP-091208 |  |  |  | RAN #46 approval of TS 36.355 | 9.0.0 |
| 2010-03 | RP-47 | RP-100304 | 0001 | - |  | Clarification on Position location | 9.1.0 |
|  | RP-47 | RP-100304 | 0002 | - |  | Clarification on UE Rx-Tx time difference supporting capability | 9.1.0 |
|  | RP-47 | RP-100304 | 0003 | 2 |  | Completion of LPP common material | 9.1.0 |
|  | RP-47 | RP-100304 | 0004 | 5 |  | Completion of OTDOA in LPP | 9.1.0 |
|  | RP-47 | RP-100304 | 0006 | - |  | Provision of Frame Drift Information in Network Time | 9.1.0 |
|  | RP-47 | RP-100304 | 0007 | - |  | Clarification of measurement reference point | 9.1.0 |
|  | RP-47 | RP-100304 | 0010 | - |  | GNSS-DifferentialCorrectionsSupport | 9.1.0 |
|  | RP-47 | RP-100304 | 0011 | - |  | BSAlign Indication in GNSS Reference Time | 9.1.0 |
|  | RP-47 | RP-100304 | 0012 | 1 |  | Changes to reflect LPP ASN.1 review | 9.1.0 |
|  | RP-47 | RP-100304 | 0013 | 1 |  | Introduction of LPP reliability sublayer | 9.1.0 |
|  | RP-47 | RP-100304 | 0015 | - |  | LPP error procedures and conditions | 9.1.0 |
|  | RP-47 | RP-100304 | 0016 | - |  | Triggered Location Information Transfer due to Cell Change | 9.1.0 |
| 2010-06 | RP-48 | RP-100558 | 0018 | 2 |  | Addition of need codes to optional LPP information elements | 9.2.0 |
|  | RP-48 | RP-100558 | 0019 | 1 |  | Miscellaneous corrections to LPP stage 3 | 9.2.0 |
|  | RP-48 | RP-100558 | 0020 | 1 |  | Small corrections to LPP specification | 9.2.0 |
|  | RP-48 | RP-100558 | 0021 | - |  | Clarifications of OTDOA parameters | 9.2.0 |
|  | RP-48 | RP-100558 | 0022 | 1 |  | Signalling support for PRS muting in OTDOA | 9.2.0 |
|  | - | - | - | - |  | Two times capital R replaced by lower case r in "MeasuredResultsElement" (undoing not intended change) | 9.2.1 |
| 2010-09 | RP-49 | RP-100852 | 0024 | - |  | Addition of an EPDU to an LPP Error and LPP Abort | 9.3.0 |
|  | RP-49 | RP-100852 | 0026 | - |  | Division of LPP into Separate ASN.1 Modules with a Global Identifier | 9.3.0 |
|  | RP-49 | RP-100852 | 0028 | - |  | Proposed Corrections to LPP Reliable Transport | 9.3.0 |
|  | RP-49 | RP-100852 | 0029 | - |  | Proposed Corrections to the PeriodicalReportingCriteria in LPP | 9.3.0 |
|  | RP-49 | RP-100852 | 0030 | 1 |  | Various corrections and clarifications to LPP | 9.3.0 |
|  | RP-49 | RP-100852 | 0031 | - |  | Support of functional components for LPP reliable transport | 9.3.0 |
|  | RP-49 | RP-100852 | 0032 | 1 |  | Introduction of EPDU ID requested by OMA LOC | 9.3.0 |
|  | RP-49 | RP-100852 | 0035 | 1 |  | Several corrections in LPP | 9.3.0 |
|  | RP-49 | RP-100852 | 0036 | - |  | Clarification to Assistance Data Transfer Procedure | 9.3.0 |
| 2010-12 | RP-50 | RP-101207 | 0037 | - |  | Correction of reliable transport terminology in description of LPP-Message | 9.4.0 |
|  | RP-50 | RP-101207 | 0038 | - |  | One cell with known SFN in OTDOA assistance data | 9.4.0 |
|  | RP-50 | RP-101207 | 0039 | 1 |  | UE frequency capability for LPP | 9.4.0 |
|  | RP-50 | RP-101207 | 0041 | - |  | Correction to LPP reliable transport | 9.4.0 |
|  | RP-50 | RP-101207 | 0042 | - |  | Correction to LPP Error procedure | 9.4.0 |
|  | RP-50 | RP-101207 | 0043 | - |  | Addition of missing reference to LPPe | 9.4.0 |
|  | RP-50 | RP-101207 | 0044 | 2 |  | Correction to the ODTOA assistance data | 9.4.0 |
|  | RP-50 | RP-101226 | 0040 | - |  | Update of 'serving cell' terminology in 36.355 | 10.0.0 |
| 2011-03 | RP-51 | RP-110269 | 0046 | - |  | Editorial corrections to 36.355 | 10.1.0 |
|  | RP-51 | RP-110269 | 0048 | - |  | Removal of FFS for retransmission timer in LPP | 10.1.0 |
|  | RP-51 | RP-110269 | 0050 | - |  | Correction to code phase encoding in GNSS acquisition assistance | 10.1.0 |
|  | RP-51 | RP-110269 | 0052 | 1 |  | Clarification on SFN provided with OTDOA measurement | 10.1.0 |
|  | RP-51 | RP-110269 | 0053 | 1 |  | Introduction of OTDOA inter-freq RSTD measurement indication procedure | 10.1.0 |
|  | RP-51 | RP-110269 | 0057 | - |  | Small corrections in 36.355 | 10.1.0 |
|  | RP-51 | RP-110269 | 0058 | 3 |  | Further corrections to the OTDOA assistance data | 10.1.0 |
| 2011-06 | RP-52 | RP-110830 | 0060 | - |  | Clarifications to description of OTDOA positioning fields | 10.2.0 |
| 2011-09 | RP-53 | RP-111279 | 0062 | 1 |  | Various corrections to LPP | 10.3.0 |
|  | RP-53 | RP-111279 | 0064 | - |  | Mandatory support of PRS for OTDOA measurements | 10.3.0 |
| 2011-12 | RP-54 | RP-111709 | 0066 | - |  | Clarification of packed encoding rules of LPP | 10.4.0 |
|  | RP-54 | RP-111709 | 0068 | - |  | Clarification of first bit in BIT STRING definitions | 10.4.0 |
| 2012-06 | RP-56 | RP-120808 | 0071 | - |  | Usage of additionalInformation IE | 10.5.0 |
| 2012-09 | RP-57 | RP-121424 | 0074 | 2 |  | Corrections to GNSS Acquisition Assistance Data | 10.6.0 |
|  | RP-57 | - | - | - |  | Upgrade to the Release 11 - no technical change | 11.0.0 |

| 2012-12 | RP-58 | RP-121931 | 0077 | - |  | Correcting the referencing of QoS parameters | 11.1.0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RP-58 | RP-121931 | 0080 | - |  | Correction to missing field description in GNSS-AcquisitionAssistance IE | 11.1.0 |
| 2013-03 | RP-59 | RP-130237 | 0083 | 1 |  | Extending E-UTRA Frequency Band and EARFCN value range | 11.2.0 |
|  | RP-59 | RP-130230 | 0086 | - |  | Correction to PRS Muting Configuration | 11.2.0 |
| 2013-06 | RP-60 | RP-130803 | 0088 | - |  | Correction for ASN.1 errors from CR0083r1 | 11.3.0 |
|  | RP-60 | RP-130803 | 0091 | - |  | Correction to integer code phase field description in GNSS Acquisition Assistance | 11.3.0 |
|  | RP-60 | RP-130803 | 0093 | - |  | Correction to serving cell terminology | 11.3.0 |
|  | RP-60 | RP-130803 | 0094 | - |  | Encoding of LPP IEs | 11.3.0 |
| 2013-09 | RP-61 | RP-131314 | 0098 | - |  | Correction on svReqList | 11.4.0 |
| 2013-12 | RP-62 | RP-131984 | 0103 | - |  | Correction to missing capability indication for inter-frequency RSTD measurements | 11.5.0 |
|  | RP-62 | RP-131984 | 0107 | 1 |  | Correction to Galileo assistance data elements | 11.5.0 |
|  | RP-62 | RP-132000 | 0104 | 1 |  | Stage 3 CR of TS 36.355 for introducing BDS in LTE | 12.0.0 |
|  | RP-62 | RP-131984 | 0108 | - |  | Correction to Galileo assistance data elements | 12.0.0 |
| 2014-03 | RP-63 | RP-140342 | 0112 | 1 |  | Clarification to gnss-DayNumber | 12.1.0 |
| 2014-06 | RP-64 | RP-140871 | 0119 | - |  | Signaling of OTDOA Neighbour Cell Information and Measurements | 12.2.0 |
| 2014-12 | RP-66 | RP-142114 | 0122 | - |  | Correction to Galileo Assistance Data | 12.3.0 |
|  | RP-66 | RP-142114 | 0123 | - |  | Addition of an Early Position Fix to LPP | 12.3.0 |
|  | RP-66 | RP-142120 | 0124 | - |  | BDS update to version 2.0 | 12.3.0 |
| 2015-03 | RP-67 | RP-150369 | 0126 | 2 |  | Correction of GLONASS system time | 12.4.0 |
|  | RP-67 | RP-150376 | 0125 | 1 |  | LPP clean-up | 12.4.0 |
| 2015-12 | RP-70 | RP-152055 | 0134 | 1 |  | Correction to the definition of Need codes | 12.5.0 |
| 2015-12 | RP-70 | RP-152068 | 0137 | 3 |  | RAT-Independent positioning enhancements | 13.0.0 |
| 2016-03 | RP-71 | RP-160463 | 0138 | 1 |  | Correction to GLONASS IOD value range | 13.1.0 |
|  | RP-71 | RP-160470 | 0140 | 1 |  | r13 Information Element correction | 13.1.0 |
|  | RP-71 | RP-160470 | 0141 | - |  | WLAN AP Identifier correction | 13.1.0 |
|  | RP-71 | RP-160470 | 0142 | 1 |  | LPP clean-up | 13.1.0 |
| 2016-09 | RP-73 | RP-161750 | 0143 | 4 |  | Correction of ECID positioning for TDD | 13.2.0 |
| 2016-12 | RP-74 | RP-162317 | 0160 | 1 |  | Clarification of WLAN RSSI value range | 13.3.0 |
| 2016-12 | RP-74 | RP-162326 | 0155 | 1 |  | CR for 36.355 Further Indoor positioning enhancements | 14.0.0 |
|  | RP-74 | RP-162327 | 0157 | - |  | Barometric Pressure Uncertainty IEs | 14.0.0 |
|  | RP-74 | RP-162326 | 0161 | 1 |  | Introduction of Further Indoor Positioning Enhancements | 14.0.0 |
| 2017-03 | RP-75 | RP-170636 | 0162 | 3 | B | Introduction of positioning for further enhanced MTC | 14.1.0 |
|  | RP-75 | RP-170642 | 0163 | - | C | Addition of periodical and triggered reporting capabilitiy signalling | 14.1.0 |
|  | RP-75 | RP-170642 | 0165 | 2 | F | Further Indoor positioning enhancements corrections | 14.1.0 |
|  | RP-75 | RP-170637 | 0166 | - | B | Introduction of positioning support for NB-IoT | 14.1.0 |
| 2017-06 | RP-76 | RP-171224 | 0169 | 3 | F | Compact Signal Measurement Information for OTDOA | 14.2.0 |
|  | RP-76 | RP-171223 | 0171 | 1 | F | Correction to PRS Subframe Offset | 14.2.0 |
|  | RP-76 | RP-171223 | 0173 | 1 | F | Correction to SFN time stamp in OTDOA Signal Measurement Information | 14.2.0 |
|  | RP-76 | RP-171223 | 0174 | 1 | F | Correction to OTDOA capabilities | 14.2.0 |
|  | RP-76 | RP-171224 | 0175 | 1 | F | Correction to NPRS | 14.2.0 |
|  | RP-76 | RP-171225 | 0176 | 2 | F | LPP clean-up | 14.2.0 |
|  | RP-76 | RP-171224 | 0177 | - | F | Corrections to number of NPRS carriers and ECID measurements for NB-IoT | 14.2.0 |
|  | RP-76 | RP-171224 | 0178 | 1 | F | Removal of FFS for retransmission timer in LPP | 14.2.0 |
|  | RP-76 | RP-171224 | 0181 | 1 | F | Signalling optimisation for NB-IoT Enhancements | 14.2.0 |
| 2017-09 | RP-77 | RP-171913 | 0182 | 2 | F | Clarification on definition of PRS Occasion Group | 14.3.0 |
|  | RP-77 | RP-171914 | 0183 | 1 | F | Additional OTDOA Capabilities | 14.3.0 |
|  | RP-77 | RP-171911 | 0184 | - | F | Clarification to GNSS-TimeModelList | 14.3.0 |
|  | RP-77 | RP-171913 | 0185 | 1 | F | Minor corrections on TS 36.355 for Rel-14 MTC | 14.3.0 |
| 2017-12 | RP-78 | RP-172616 | 0187 | 2 | F | Correction on PRS hopping configuration | 14.4.0 |
| 2018-03 | RP-79 | RP-180446 | 0189 | 1 | F | Segmentation of LPP Messages | 14.5.0 |
| 2018-04 | RP-79 |  |  |  |  | New version to fix ASN.1 formatting | 14.5.1 |
| 2018-06 | RP-80 | RP-181235 | 0202 | 2 | F | Clarification for NRSRQ reporting with E-CID | 14.6.0 |
| 2018-06 | RP-80 | RP-181219 | 0204 | 2 | B | Introduction of IMU support for OTDOA | 15.0.0 |
|  | RP-80 | RP-181219 | 0205 | 1 | B | Addition of RTK and PPP support | 15.0.0 |
|  | RP-80 | RP-181219 | 0207 | 1 | B | Addition of broadcast of positioning assistance data | 15.0.0 |
|  | RP-80 | RP-181215 | 0209 | 1 | B | Addition of NR Support | 15.0.0 |
|  | RP-80 | RP-181252 | 0210 | 1 | B | Addition of NB-IoT TDD support | 15.0.0 |
| 2018-09 | RP-81 | RP-181963 | 0215 | 1 | A | Support for NPRS enhancements | 15.1.0 |
|  | RP-81 | RP-181945 | 0218 | 1 | F | Corrections to TDD in 36.355 | 15.1.0 |
|  | RP-81 | RP-181961 | 0221 | 3 | A | Correction to RSRQ range in 36.355 | 15.1.0 |
|  | RP-81 | RP-181942 | 0222 | 1 | F | OTDOA Assistance Data Request for NR | 15.1.0 |
|  | RP-81 | RP-181960 | 0223 | - | F | LPP clean-up | 15.1.0 |
|  | RP-81 | RP-181952 | 0224 | 1 | F | GAD shapes for high accuracy positioning | 15.1.0 |
|  | RP-81 | RP-181952 | 0226 | 1 | B | Positioning SIB value tag and expiration time | 15.1.0 |
| 2018-12 | RP-82 | RP-182672 | 0213 | 3 | F | Addition of TDD UL/DL configuration to OTDOA assistance data | 15.2.0 |
|  | RP-82 | RP-182681 | 0228 | 2 | F | Introduction of TDD UL/DL configuration for NB-IoT in 36.355 | 15.2.0 |
|  | RP-82 | RP-182659 | 0229 | 3 | F | SFN offset for OTDOA | 15.2.0 |
|  | RP-82 | RP-182674 | 0230 | 1 | F | Alignment of IE/field names between LPP and RRC specifications | 15.2.0 |
|  | RP-82 | RP-182672 | 0232 | 1 | F | Sensor Assistance Data Elements Correction | 15.2.0 |
| 2019-03 | RP-83 | RP-190550 | 0234 | 3 | F | Stage 2 and stage 3 sensor methods description alignment | 15.3.0 |
| 2019-06 | RP-84 | RP-191376 | 0239 | 1 | F | Minor corrections on NR Support | 15.4.0 |
|  | RP-84 | RP-191384 | 0240 | 4 | F | Periodic assistance data transfer with cell ID change procedure | 15.4.0 |
| 2019-09 | RP-85 | RP-192196 | 0243 | 1 | F | Distinguishing Location Source when sensor method is used | 15.5.0 |

NOTE:	The table above will not be further updated in the future. It shows all TS 36.355 CRs taken over into
TS 37.355 v1.0.0.

| Change history of TS 37.355 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Cat | Subject/Comment | New version |
| 2019-12 | RP-86 | RP-192450 | - | - | - | TS 36.355 v15.5.0 contents was transferred into this new TS 37.355 as by definition 36 series specifications cover E-UTRA/LTE only aspects and multi-RAT aspects need to be covered in 37 series specifications. The only changes compared to TS 36.355 v15.5.0: - new 37 series TS number is added: TS 37.355 - "Evolved Universal Terrestrial Radio Access (E-UTRA)" is removed from the TS title as beginning with Rel-15 TS 36.355 needs to cover also NR - clarification in the Scope clause that this TS covers radio access technologies E-UTRA/LTE and NR - the CR history table of TS 36.355 was kept for easier reference of all changes included in TS 37.355 but a new CR history table was added for TS 37.355. | 1.0.0 |
| 2019-12 | RP-86 |  |  |  |  | The approved version upgraded to v15.0.0 by MCC. | 15.0.0 |
| 2020-03 | RP-87 | RP-200367 | 0001 | 2 | C | Addition of broadcast of barometric pressure assistance data | 16.0.0 |
|  | RP-87 | RP-200357 | 0002 | 1 | F | Sensor Provide Location Information Elements Correction | 16.0.0 |
|  | RP-87 | RP-200365 | 0247 | 8 | B | CR of TS 37.355 for introducing NavIC in LTE – core part | 16.0.0 |
|  | RP-87 | RP-200357 | 0248 | 1 | B | Introduction of B1C signal in BDS system in A-GNSS | 16.0.0 |
|  | RP-87 | RP-200367 | 0249 | 1 | C | Addition of broadcast TBS assistance data | 16.0.0 |
|  | RP-87 | RP-200345 | 0250 | 2 | B | Introduction of NR positioning | 16.0.0 |
|  | RP-87 | RP-200357 | 0252 | 1 | B | Introducing support for GNSS Integer Ambiguity Level Indications | 16.0.0 |
| 2020-07 | RP-88 | RP-201196 | 0257 | 2 | F | Introduction of NavIC Keplerian set IE | 16.1.0 |
|  | RP-88 | RP-201190 | 0259 | 1 | F | Update B1I signal ICD file to v3.0 in BDS system in A-GNSS | 16.1.0 |
|  | RP-88 | RP-201175 | 0260 | - | F | LPP Clean-Up | 16.1.0 |
|  | RP-88 | RP-201175 | 0261 | 1 | B | Introduction of Release-16 UE positioning capabilities | 16.1.0 |
| 2020-09 | RP-89 | RP-201989 | 0272 | - | F | LPP miscellaneous corrections | 16.2.0 |
| 2020-12 | RP-90 | RP-202775 | 0274 | - | F | Clarification of quality and time stamp for RSTD measurements | 16.3.0 |
|  | RP-90 | RP-202789 | 0280 | 2 | F | Correction of hanging ASN.1 code after END | 16.3.0 |
|  | RP-90 | RP-202775 | 0282 | - | F | Correction on LPP spec | 16.3.0 |
| 2021-03 | RP-91 | RP-210695 | 0284 | 1 | F | Corrections on the field description of commonIEsProvideAssistanceData in TS37.355 | 16.4.0 |
| 2021-06 | RP-92 | RP-211474 | 0288 | 4 | F | LPP Layer interaction with lower layers for Positioning Frequency layer and Measurement Gap | 16.5.0 |
|  | RP-92 | RP-211482 | 0292 | 3 | F | Correction to the need code for downlink LPP message-R16 | 16.5.0 |
|  | RP-92 | RP-211474 | 0294 | 2 | F | Miscellaneous corrections on the field description | 16.5.0 |
|  | RP-92 | RP-211474 | 0300 | 2 | F | Correction to PRS configuration | 16.5.0 |
|  | RP-92 | RP-211474 | 0301 | 2 | F | Correction to the uplink LPP message | 16.5.0 |
|  | RP-92 | RP-211474 | 0302 | 3 | F | Correction to DL-PRS capability | 16.5.0 |
|  | RP-92 | RP-211474 | 0306 | 2 | F | Correction to NR-ARFCN of the TRP | 16.5.0 |
|  | RP-92 | RP-211474 | 0311 | - | F | Description on timestamp reference in NR positioning measurement report | 16.5.0 |
| 2021-09 | RP-93 | RP-212443 | 0305 | 5 | F | Correction to PRS-only TP | 16.6.0 |
|  | RP-93 | RP-212443 | 0312 | 1 | F | Correction for LPP assistance information | 16.6.0 |
|  | RP-93 | RP-212443 | 0313 | 1 | F | Corrections on the conditional presence tag clarification for Uplink LPP message | 16.6.0 |
|  | RP-93 | RP-212443 | 0318 | - | F | Correction to the need code in NR-SelectedDL-PRS-IndexList | 16.6.0 |
| 2021-12 | RP-94 | RP-213344 | 0321 | 2 | F | Updates based on RAN1 NR positioning features list | 16.7.0 |
|  | RP-94 | RP-213345 | 0323 | 3 | F | Correction on BDS B2I clock model [Rel16BDS] | 16.7.0 |
| 2022-03 | RP-95 | RP-220835 | 0326 | - | F | Addition of missing need code for the BDS TGD2 parameter | 16.8.0 |
|  | RP-95 | RP-220472 | 0329 | 1 | A | Correction on the object identifier of LPP ASN.1 for R16 | 16.8.0 |
|  | RP-95 | RP-220835 | 0330 | - | F | Correction of reference TRP for DL-AoD and Multi-RTT measurement report | 16.8.0 |
|  | RP-95 | RP-220835 | 0331 | - | F | Correction to NR-DL-PRS-ResourcesCapability field description | 16.8.0 |
|  | RP-95 | RP-220835 | 0333 | 1 | F | Introducing new high accuracy GAD shape with scalable uncertainty | 16.8.0 |
| 2022-03 | RP-95 | RP-220479 | 0332 | - | B | Introduction of R17 Positioning Enhancements in LPP | 17.0.0 |