| 3GPP TR 37.717-31-11 V1.0.0 (2022-06) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Networks; Dual Connectivity (DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL) (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2020, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	16

1	Scope	18

2	References	18

3	Definitions of terms, symbols and abbreviations	18

3.1	Terms	18

3.2	Symbols	18

3.3	Abbreviations	18

4	Background	19

4.1	TR maintenance	19

5	DC of 3 LTE band (3DL/1UL) + 1 NR band: Specific Band Combination Part	19

5.1	Inter-band EN-DC	19

5.1.1	DC_1-3_(n)41	19

5.1.1.2	 ∆TIB and ∆RIB values	19

5.1.1.3	REFSENS requirements	20

5.1.2	 DC_1-3-41_n28	20

5.1.2.1	Configuration for EN-DC	20

5.1.2.2	∆TIB and ∆RIB values	20

5.1.2.3	REFSENS requirements	20

5.1.3	DC_3-7-8_n40	21

5.1.3.1	Configurations for EN-DC	21

5.1.3.2	∆TIB and ∆RIB values	21

5.1.3.3	 Reference sensitivity exceptions	21

5.1.4	DC_3-7-28_n1	21

5.1.4.1	Configurations for EN-DC	21

5.1.4.2	∆TIB and ∆RIB values	22

5.1.4.3	 Reference sensitivity exceptions	22

5.1.5	DC_5-7-66_n66	22

5.1.5.1	Configurations for EN-DC	22

5.1.5.2	∆TIB and ∆RIB values	22

5.1.5.3	 Reference sensitivity exceptions	23

5.1.6	DC_3-19-42_n1	23

5.1.6.1	Configuration for EN-DC	23

5.1.6.2	∆TIB and ∆RIB values	23

5.1.6.3	Reference sensitivity exceptions	23

5.1.7	DC_3-21-42_n1	23

5.1.7.1	Configuration for EN-DC	23

5.1.7.2	∆TIB and ∆RIB values	24

5.1.7.3	Reference sensitivity exceptions	24

5.1.8	DC_19-21-42_n1	24

5.1.8.1	Configuration for EN-DC	24

5.1.8.2	∆TIB and ∆RIB values	24

5.1.8.3	Reference sensitivity exceptions	25

5.1.9	 DC_2-28-66_n66	25

5.1.9.1	 Operating bands for EN-DC	25

5.1.9.2 	Configuration for EN-DC	25

5.1.9.3	 ∆TIB and ∆RIB values	25

5.1.10	 DC_7-28-66_n66	26

5.1.10.1	 Operating bands for EN-DC	26

5.1.10.2 	Configuration for EN-DC	26

5.1.10.3	 ∆TIB and ∆RIB values	26

5.1.11	 DC_2-7-28_n66	26

5.1.11.1	 Operating bands for EN-DC	26

5.1.11.2 	Configuration for EN-DC	27

5.1.11.3	 ∆TIB and ∆RIB values	27

5.1.12	DC_1-8-11_n3	27

5.1.12.1	Configurations for EN-DC	27

5.1.12.2	∆TIB and ∆RIB values	27

5.1.12.3	Reference sensitivity exceptions	28

5.1.13	DC_1-8-42_n28	28

5.1.13.1	Configurations for EN-DC	28

5.1.13.2	∆TIB and ∆RIB values	28

5.1.13.3	Reference sensitivity exceptions	28

5.1.14	DC_1-7-32_n28	29

5.1.14.1	Configuration for EN-DC	29

5.1.14.2	∆TIB and ∆RIB values	29

5.1.14.3	Reference sensitivity exceptions	29

5.1.15	DC_1-7-32_n78	29

5.1.15.1	Configuration for EN-DC	29

5.1.15.2	∆TIB and ∆RIB values	29

5.1.15.3	Reference sensitivity exceptions	30

5.1.16	DC_1-20-32_n28	30

5.1.16.1	Configuration for EN-DC	30

5.1.16.2	∆TIB and ∆RIB values	30

5.1.16.3	Reference sensitivity exceptions	30

5.1.17	DC_1-20-32_n78	30

5.1.17.1	Configuration for EN-DC	30

5.1.17.2	∆TIB and ∆RIB values	31

5.1.17.3	Reference sensitivity exceptions	31

5.1.18	DC_3-7-32_n78	31

5.1.18.1	Configuration for EN-DC	31

5.1.18.2	∆TIB and ∆RIB values	31

5.1.18.3	Reference sensitivity exceptions	31

5.1.19	DC_3-20-32_n78	32

5.1.19.1	Configuration for EN-DC	32

5.1.19.2	∆TIB and ∆RIB values	32

5.1.19.3	Reference sensitivity exceptions	32

5.1.20	DC_7-20-32_n1	32

5.1.20.1	Configuration for EN-DC	32

5.1.20.2	∆TIB and ∆RIB values	32

5.1.20.3	Reference sensitivity exceptions	33

5.1.21	DC_7-20-32_n28	33

5.1.21.1	Configuration for EN-DC	33

5.1.21.2	∆TIB and ∆RIB values	33

5.1.21.3	Reference sensitivity exceptions	33

5.1.22	DC_1-20-32_n3	33

5.1.23	DC_2-4-7_n28	34

5.1.24	DC_2-5-7_n66	35

5.1.25	DC_2-5-66_n7	36

5.1.26	DC_2-5-66_n66	37

5.1.27	DC_2-7-66_n28	38

5.1.28	DC_3-20-32_n1	38

5.1.29	DC_1-3-18_n3	39

5.1.29.1	Configuration for DC	39

5.1.29.2	∆TIB and ∆RIB values	39

5.1.29.3	REFSENS requirements	40

5.1.30	DC_1-3-41_n3	40

5.1.30.1	Configuration for DC	40

5.1.30.2	∆TIB and ∆RIB values	40

5.1.30.3	REFSENS requirements	41

5.1.31	DC_1-3-41_n41	41

5.1.31.1	Configuration for DC	41

5.1.31.2	∆TIB and ∆RIB values	41

5.1.31.3	REFSENS requirements	42

5.1.32	DC_2-5-7_n66 and DC_2-5-7-7_n66	42

5.1.32.1	Configuration for DC	42

5.1.32.2	∆TIB and ∆RIB values	42

5.1.32.3	REFSENS requirements	43

5.1.38	 DC_1-3-18_n28	47

5.1.38.1	Configuration for EN-DC	47

5.1.38.2	∆TIB and ∆RIB values	47

No additional MSD requirement need to be defined for this dual connectivity configuration.	47

5.1.39	 DC_1-3-18_n41	47

5.1.39.1	Configuration for EN-DC	47

5.1.39.2	∆TIB and ∆RIB values	48

5.1.40	DC_2-7-28_n7	48

5.1.41	DC_2A-66A-71A_n71A	49

5.1.42	DC_2-5-66_n77A	50

5.1.43	DC_2-13-66_n77A	50

5.1.44	DC_2-48-66_n77A	51

5.1.45	 DC_1-3-40_n78	52

5.1.45.1	Configuration for EN-DC	52

5.1.45.2	∆TIB and ∆RIB values	52

5.1.45.3	REFSENS requirements	52

5.1.46	 DC_1-7-40_n78	53

5.1.46.1	Configuration for EN-DC	53

5.1.46.2	∆TIB and ∆RIB values	53

5.1.46.3	REFSENS requirements	53

5.1.47	 DC_1-8-40_n78	53

5.1.47.1	Configuration for EN-DC	53

5.1.47.2	∆TIB and ∆RIB values	53

5.1.47.3	REFSENS requirements	54

5.1.48	 DC_3-7-40_n78	54

5.1.48.1	Configuration for EN-DC	54

5.1.48.2	∆TIB and ∆RIB values	54

5.1.48.3	REFSENS requirements	55

5.1.49	 DC_3-8-40_n78	55

5.1.49.1	Configuration for EN-DC	55

5.1.49.2	∆TIB and ∆RIB values	55

5.1.49.3	REFSENS requirements	55

5.1.50	 DC_7-8-40_n78	56

5.1.50.1	Configuration for EN-DC	56

5.1.50.2	∆TIB and ∆RIB values	56

5.1.50.3	REFSENS requirements	56

5.1.51	DC_1-7-8_n28	56

5.1.51.1	 Configurations for EN-DC	56

5.1.51.2	 ∆TIB and ∆RIB values	57

5.1.51.3	 Reference sensitivity exceptions	57

5.1.52	DC_3-7-8_n28	57

5.1.52.1	 Configurations for EN-DC	57

5.1.52.2	 ∆TIB and ∆RIB values	57

5.1.52.3	 Reference sensitivity exceptions	58

5.1.53	DC_1-7-28_n3	58

5.1.53.1	 Configurations for EN-DC	58

5.1.53.2	 ∆TIB and ∆RIB values	58

5.1.53.3	 Reference sensitivity exceptions	58

5.1.54	DC_3-8-40_n1	59

5.1.54.1	 Configurations for EN-DC	59

5.1.54.2	 ∆TIB and ∆RIB values	59

5.1.54.3	 Reference sensitivity exceptions	59

5.1.55	DC_7-8-40_n1	59

5.1.55.1	 Configurations for EN-DC	59

5.1.55.2	 ∆TIB and ∆RIB values	60

5.1.55.3	 Reference sensitivity exceptions	60

5.1.56	DC_2-28-66_n7	60

5.1.56.1	 Configurations for EN-DC	60

5.1.56.2	 ∆TIB and ∆RIB values	60

5.1.56.3	 Reference sensitivity exceptions	61

5.1.57	DC_2-5-7_n7	61

5.1.57.1	 Configurations for EN-DC	61

5.1.57.2	 ∆TIB and ∆RIB values	61

5.1.57.3	 Reference sensitivity exceptions	61

5.1.58	DC_2-7-66_n7/DC_2-7-66-66_n7	62

5.1.58.1	 Configurations for EN-DC	62

5.1.58.2	 ∆TIB and ∆RIB values	62

5.1.58.3	 Reference sensitivity exceptions	62

5.1.59	DC_5-7-66_n7/DC_5-7-66-66_n7	62

5.1.59.1	 Configurations for EN-DC	62

5.1.59.2	 ∆TIB and ∆RIB values	63

5.1.59.3	 Reference sensitivity exceptions	63

5.1.60	DC_7-28-66_n7	63

5.1.60.1	 Configurations for EN-DC	63

5.1.60.2	 ∆TIB and ∆RIB values	63

5.1.60.3	 Reference sensitivity exceptions	64

5.1.61	DC_2-7-66_n77	64

5.1.61.1	Configurations for EN-DC	64

5.1.61.2	∆TIB and ∆RIB values	64

5.1.61.3	 Reference sensitivity exceptions	64

5.1.62	DC_1-20-40_n78	65

5.1.62.1	Configuration for EN-DC	65

5.1.62.2	∆TIB and ∆RIB values	65

5.1.62.3	Reference sensitivity exceptions	65

5.1.63	DC_1-8-42_n3	65

5.1.63.1	Configurations for EN-DC	65

5.1.63.2	∆TIB and ∆RIB values	66

5.1.63.3	Reference sensitivity exceptions	66

5.1.64	DC_1-3-42_n28	66

5.1.64.1	Configurations for EN-DC	66

5.1.64.2	∆TIB and ∆RIB values	66

5.1.64.3	Reference sensitivity exceptions	67

5.1.65	DC_2-29-66_n78	67

5.1.65.1	Configuration for EN-DC	67

5.1.65.2	∆TIB and ∆RIB values	67

5.1.65.3	REFSENS requirements	67

5.1.66	DC_7-8-32_n1	68

5.1.66.1	Configuration for EN-DC	68

5.1.66.2	∆TIB and ∆RIB values	68

5.1.66.3	Reference sensitivity exceptions	68

5.1.67	DC_7-20-32_n78	68

5.1.67.1	Configuration for EN-DC	68

5.1.67.2	∆TIB and ∆RIB values	68

5.1.67.3	Reference sensitivity exceptions	69

5.1.68	DC_2A-12A-66A_n41A	69

5.1.68.2	Configuration for DC	69

5.1.69	DC_2A-66A-71A_n41A	70

5.1.69.2	Configuration for DC	70

5.1.70	DC_2A-7A-12A_n66A	71

5.1.70.2	Configuration for DC	71

5.1.71	DC_2A_2A-5A-7A_n66A	72

5.1.71.2	Configuration for DC	72

5.1.72	DC_2A-7A-71A_n66A	73

5.1.72.2	Configuration for DC	73

5.1.73	DC_2A-7A-12A_n78A	74

5.1.73.2	Configuration for DC	74

5.1.74	DC_2A-12A-66A_n78A	75

5.1.74.2	Configuration for DC	75

5.1.75	DC_7A-12A-66A_n78A	76

5.1.75.2	Configuration for DC	76

5.1.76	DC_7A-66A-71A_n78A	77

5.1.76.2	Configuration for DC	77

5.1.77	DC_2A-7A -71A_n78A	78

5.1.77.2	Configuration for DC	78

5.1.78	DC_2A-7A -66A_n2A	79

5.1.78.2	Configuration for DC	79

5.1.79	DC_2A-5A -7A_n2A	80

5.1.79.2	Configuration for DC	80

5.1.80	DC_5A-7A -66A_n2A	81

5.1.80.2	Configuration for DC	81

5.1.81	DC_2A-7A -71A_n2A	82

5.1.81.2	Configuration for DC	82

5.1.82	DC_2A-66A -71A_n2A	83

5.1.82.2	Configuration for DC	83

5.1.83	DC_2A-7A -12A_n2A	84

5.1.83.2	Configuration for DC	84

5.1.84	DC_7A-66A-71A_n2A	85

5.1.84.2	Configuration for DC	85

5.1.85	DC_7A-12A-66A_n2A	86

5.1.85.2	Configuration for DC	86

5.1.86	 DC_1-28-40_n78	87

5.1.86.1	Configuration for EN-DC	87

5.1.86.2	∆TIB and ∆RIB values	87

5.1.86.3	REFSENS requirements	87

5.1.87	 DC_3-28-40_n78	87

5.1.87.1	Configuration for EN-DC	87

5.1.87.2	∆TIB and ∆RIB values	87

5.1.87.3	REFSENS requirements	88

5.1.88	 DC_1-11-18_n3	88

5.1.88.1	Configuration for EN-DC	88

5.1.88.2	∆TIB and ∆RIB values	88

5.1.88.3	REFSENS requirements	89

5.1.89	 DC_1-11-18_n28	89

5.1.89.1	Configuration for EN-DC	89

5.1.89.2	∆TIB and ∆RIB values	89

5.1.89.3	REFSENS requirements	89

5.1.90	 DC_1-11-18_n41	90

5.1.90.1	Configuration for EN-DC	90

5.1.90.2	∆TIB and ∆RIB values	90

5.1.90.3	REFSENS requirements	90

5.1.91	 DC_2-5-66_n48	90

5.1.91.1	Configuration for EN-DC	90

5.1.91.2	∆TIB and ∆RIB values	90

5.1.91.3	REFSENS requirements	91

5.1.92	 DC_2-13-48_n77	91

5.1.92.1	Configuration for EN-DC	91

5.1.92.2	∆TIB and ∆RIB values	91

5.1.92.3	REFSENS requirements	91

5.1.93	 DC_2-46-48_n2	92

5.1.93.1	Configuration for EN-DC	92

5.1.93.2	∆TIB and ∆RIB values	92

5.1.93.3	REFSENS requirements	92

5.1.94	 DC_2-48-66_n2	92

5.1.94.1	Configuration for EN-DC	92

5.1.94.2	∆TIB and ∆RIB values	92

5.1.94.3	REFSENS requirements	93

5.1.95	 DC_2-48-66_n66	93

5.1.95.1	Configuration for EN-DC	93

5.1.95.2	∆TIB and ∆RIB values	93

5.1.95.3	REFSENS requirements	94

5.1.96	 DC_13-48-66_n77	94

5.1.96.1	Configuration for EN-DC	94

5.1.96.2	∆TIB and ∆RIB values	94

5.1.96.3	REFSENS requirements	94

5.1.97.1	Configurations for EN-DC	94

5.1.97.2	∆TIB and ∆RIB values	94

5.1.97.3	Reference sensitivity exceptions	95

5.1.98	DC_1-3-38_n28	95

5.1.98.1	 Configurations for EN-DC	95

5.1.98.2	 ∆TIB and ∆RIB values	95

5.1.98.3	 Reference sensitivity exceptions	96

5.1.99	DC_1-7-38_n28	96

5.1.99.1	 Configurations for EN-DC	96

5.1.99.2	 ∆TIB and ∆RIB values	96

5.1.99.3	 Reference sensitivity exceptions	96

5.1.100	DC_3-7-38_n28	97

5.1.100.1	 Configurations for EN-DC	97

5.1.100.2	 ∆TIB and ∆RIB values	97

5.1.100.3	 Reference sensitivity exceptions	97

5.1.101	DC_2-5-30_n2	97

5.1.101.1	Operating bands for EN-DC	97

5.1.101.2	Configuration for DC	98

5.1.101.3	 ∆TIB and ∆RIB values	98

5.1.101.4	REFSENS requirements	98

5.1.102	DC_2-5-30_n66	98

5.1.102.1	Operating bands for EN-DC	98

5.1.102.2	Configuration for DC	99

5.1.102.3	 ∆TIB and ∆RIB values	99

5.1.102.4	REFSENS requirements	99

5.1.103	DC_2-14-30_n2	99

5.1.103.1	Operating bands for EN-DC	99

5.1.103.2	Configuration for DC	100

5.1.103.3	 ∆TIB and ∆RIB values	100

5.1.103.4	REFSENS requirements	100

5.1.104	DC_2-29-30_n66	100

5.1.104.1	Operating bands for EN-DC	100

5.1.104.2	Configuration for DC	101

5.1.104.3	 ∆TIB and ∆RIB values	101

5.1.104.4	REFSENS requirements	101

5.1.105	DC_2-46-66_n5	101

5.1.105.1	Operating bands for EN-DC	101

5.1.105.2	Configuration for DC	102

5.1.105.3	 ∆TIB and ∆RIB values	102

5.1.105.4	REFSENS requirements	102

5.1.106	DC_5-30-66_n2	102

5.1.106.1	Operating bands for EN-DC	102

5.1.106.2	Configuration for DC	103

5.1.106.3	 ∆TIB and ∆RIB values	103

5.1.106.4	REFSENS requirements	103

5.1.107	DC_5-30-66_n66	103

5.1.107.1	Operating bands for EN-DC	103

5.1.107.2	Configuration for DC	104

5.1.107.3	 ∆TIB and ∆RIB values	104

5.1.107.4	REFSENS requirements	104

5.1.108	DC_14-30-66_n66	104

5.1.108.1	Operating bands for EN-DC	104

5.1.108.2	Configuration for DC	105

5.1.108.3	 ∆TIB and ∆RIB values	105

5.1.108.4	REFSENS requirements	105

5.1.109	DC_14-30-66_n2	105

5.1.109.1	Operating bands for EN-DC	105

5.1.109.2	Configuration for DC	106

5.1.109.3	 ∆TIB and ∆RIB values	106

5.1.109.4	REFSENS requirements	106

5.1.110	DC_2-2-14-30_n66	106

5.1.110.1	Operating bands for EN-DC	106

5.1.110.2	Configuration for DC	107

5.1.110.3	 ∆TIB and ∆RIB values	107

5.1.110.4	REFSENS requirements	107

5.1.111	DC_1-3-7_n3	108

5.1.111.1	Operating bands for EN-DC	108

5.1.111.2	Configuration for DC	108

5.1.111.3	 ∆TIB and ∆RIB values	108

5.1.111.4	REFSENS requirements	108

5.1.112	1-3-28_n3	109

5.1.112.1	Operating bands for EN-DC	109

5.1.112.2	Configuration for DC	109

5.1.112.3	 ∆TIB and ∆RIB values	109

5.1.112.4	REFSENS requirements	109

5.1.113	3-7-28_n3	110

5.1.113.1	Operating bands for EN-DC	110

5.1.113.2	Configuration for DC	110

5.1.113.3	 ∆TIB and ∆RIB values	110

5.1.113.4	REFSENS requirements	110

5.1.114	DC_2-29-66_n260	111

5.1.114.1	Operating bands for EN-DC	111

5.1.114.2	Configuration for DC	111

5.1.114.3	 ∆TIB and ∆RIB values	111

5.1.114.4	REFSENS requirements	111

5.1.115	DC_2-46-66_n260	112

5.1.115.1	Operating bands for EN-DC	112

5.1.115.2	Configuration for DC	112

5.1.115.3	 ∆TIB and ∆RIB values	114

5.1.115.4	REFSENS requirements	114

5.1.116	DC_29-30-66_n260	114

5.1.116.1	Operating bands for EN-DC	114

5.1.116.2	Configuration for DC	115

5.1.116.4	REFSENS requirements	115

5.1.117	DC_3-20-28_n1	116

5.1.117.1	 Configurations for EN-DC	116

5.1.117.2	 ∆TIB and ∆RIB values	116

5.1.117.3	 Reference sensitivity exceptions	116

5.1.118	DC_7-20-28_n1	116

5.1.118.1	 Configurations for EN-DC	116

5.1.118.2	 ∆TIB and ∆RIB values	117

5.1.118.3	 Reference sensitivity exceptions	117

5.1.119.1	Configuration for EN-DC	117

5.1.119.2	∆TIB and ∆RIB values	117

5.1.119.3	REFSENS requirements	117

5.1.120.1	Configuration for EN-DC	118

5.1.120.2	∆TIB and ∆RIB values	118

5.1.120.3	REFSENS requirements	118

5.1.121.1	Configuration for EN-DC	118

5.1.121.2	∆TIB and ∆RIB values	118

5.1.121.3	REFSENS requirements	119

5.1.122.1	Configuration for EN-DC	119

5.1.122.2	∆TIB and ∆RIB values	119

5.1.122.3	REFSENS requirements	119

5.1.126	DC_7-29-66_n78	121

5.1.126.1	Configurations for EN-DC	121

5.1.126.2	∆TIB and ∆RIB values	122

5.1.126.3	 Reference sensitivity exceptions	122

5.1.127	DC_1-7-32_n3	122

5.1.127.1	Configuration for EN-DC	122

5.1.127.2	∆TIB and ∆RIB values	122

5.1.127.3	Reference sensitivity exceptions	122

5.1.128	DC_1-7-32_n8	123

5.1.128.1	Configuration for EN-DC	123

5.1.128.2	∆TIB and ∆RIB values	123

5.1.128.3	Reference sensitivity exceptions	123

5.1.129	DC_1-7-38_n8	123

5.1.129.1	Configuration for EN-DC	123

5.1.129.2	∆TIB and ∆RIB values	123

5.1.129.3	Reference sensitivity exceptions	124

5.1.130	DC_1-20-28_n3	124

5.1.130.1	Configuration for EN-DC	124

5.1.130.2	∆TIB and ∆RIB values	124

5.1.130.3	Reference sensitivity exceptions	124

5.1.131	DC_1-20-32_n8	124

5.1.131.1	Configuration for EN-DC	124

5.1.131.2	∆TIB and ∆RIB values	125

5.1.131.3	Reference sensitivity exceptions	125

5.1.132	DC_1-28-32_n3	125

5.1.132.1	Configuration for EN-DC	125

5.1.132.2	∆TIB and ∆RIB values	125

5.1.132.3	Reference sensitivity exceptions	125

5.1.133	DC_3-7-32_n1	126

5.1.133.1	Configuration for EN-DC	126

5.1.133.2	∆TIB and ∆RIB values	126

5.1.133.3	Reference sensitivity exceptions	126

5.1.134	DC_3-8-20_n1	126

5.1.134.1	Configuration for EN-DC	126

5.1.134.2	∆TIB and ∆RIB values	126

5.1.134.3	Reference sensitivity exceptions	127

5.1.135	DC_7-8-20_n1	127

5.1.135.1	Configuration for EN-DC	127

5.1.135.2	∆TIB and ∆RIB values	127

5.1.135.3	Reference sensitivity exceptions	127

5.1.136	DC_7-8-20_n3	127

5.1.136.1	Configuration for EN-DC	127

5.1.136.2	∆TIB and ∆RIB values	128

5.1.136.3	Reference sensitivity exceptions	128

5.1.137	DC_7-20-28_n3	128

5.1.137.1	Configuration for EN-DC	128

5.1.137.2	∆TIB and ∆RIB values	128

5.1.137.3	Reference sensitivity exceptions	128

5.1.138	DC_7-20-32_n1	129

5.1.138.1	Configuration for EN-DC	129

5.1.138.2	∆TIB and ∆RIB values	129

5.1.138.3	Reference sensitivity exceptions	129

5.1.139	DC_7-20-32_n3	129

5.1.139.1	Configuration for EN-DC	129

5.1.139.2	∆TIB and ∆RIB values	129

5.1.139.3	Reference sensitivity exceptions	130

5.1.140	DC_7-20-32_n8	130

5.1.140.1	Configuration for EN-DC	130

5.1.140.2	∆TIB and ∆RIB values	130

5.1.140.3	Reference sensitivity exceptions	130

5.1.141	DC_7-20-38_n1	130

5.1.141.1	Configuration for EN-DC	130

5.1.141.2	∆TIB and ∆RIB values	131

5.1.141.3	Reference sensitivity exceptions	131

5.1.142	DC_7-28-32_n1	131

5.1.142.1	Configuration for EN-DC	131

5.1.142.2	∆TIB and ∆RIB values	131

5.1.142.3	Reference sensitivity exceptions	132

5.1.143	DC_7-28-32_n3	132

5.1.143.1	Configuration for EN-DC	132

5.1.143.2	∆TIB and ∆RIB values	132

5.1.143.3	Reference sensitivity exceptions	132

5.1.144	DC_8-20-32_n1	132

5.1.144.1	Configuration for EN-DC	132

5.1.144.2	∆TIB and ∆RIB values	132

5.1.144.3	Reference sensitivity exceptions	133

5.1.145	DC_20-28-32_n1	133

5.1.145.1	Configuration for EN-DC	133

5.1.145.2	∆TIB and ∆RIB values	133

5.1.145.3	Reference sensitivity exceptions	133

5.1.146	DC_20-28-32_n3	133

5.1.146.1	Configuration for EN-DC	133

5.1.146.2	∆TIB and ∆RIB values	134

5.1.146.3	Reference sensitivity exceptions	134

5.1.147	DC_20-32-38_n1	134

5.1.147.1	Configuration for EN-DC	134

5.1.147.2	∆TIB and ∆RIB values	134

5.1.147.3	Reference sensitivity exceptions	134

5.1.148	DC_3-7-7-28_n1	135

5.1.148.1	Configurations for EN-DC	135

5.1.148.2	 ∆TIB and ∆RIB values	135

5.1.148.3	 Reference sensitivity exceptions	135

5.1.149	DC_1-8-20_n28	135

5.1.149.1	Configuration for EN-DC	135

5.1.149.2	∆TIB and ∆RIB values	136

5.1.149.3	Reference sensitivity exceptions	136

5.1.150	DC_2-5-30_n77	136

5.1.150.1	Configuration for EN-DC	136

5.1.150.2	∆TIB and ∆RIB values	136

5.1.150.3	Reference sensitivity exceptions	137

5.1.151	DC_2-12-30_n77	137

5.1.151.1	Configuration for EN-DC	137

5.1.151.2	∆TIB and ∆RIB values	137

5.1.151.3	Reference sensitivity exceptions	137

5.1.152	DC_2-12-66_n77	138

5.1.152.1	Configuration for EN-DC	138

5.1.152.2	∆TIB and ∆RIB values	138

5.1.152.3	Reference sensitivity exceptions	138

5.1.153	DC_2-14-30_n77	139

5.1.153.1	Configuration for EN-DC	139

5.1.153.2	∆TIB and ∆RIB values	139

5.1.153.3	Reference sensitivity exceptions	139

5.1.154	DC_2-14-66_n77	139

5.1.154.1	Configuration for EN-DC	139

5.1.154.2	∆TIB and ∆RIB values	140

5.1.154.3	Reference sensitivity exceptions	140

5.1.155	DC_2-29-30_n77	140

5.1.155.1	Configuration for EN-DC	140

5.1.155.2	∆TIB and ∆RIB values	140

5.1.155.3	Reference sensitivity exceptions	141

5.1.156	DC_2-29-66_n77	141

5.1.156.1	Configuration for EN-DC	141

5.1.156.2	∆TIB and ∆RIB values	141

5.1.156.3	Reference sensitivity exceptions	141

5.1.157	DC_2-30-66_n77	142

5.1.157.1	Configuration for EN-DC	142

5.1.157.2	∆TIB and ∆RIB values	142

5.1.157.3	Reference sensitivity exceptions	142

5.1.158	DC_5-30-66_n77	143

5.1.158.1	Configuration for EN-DC	143

5.1.158.2	∆TIB and ∆RIB values	143

5.1.158.3	Reference sensitivity exceptions	143

5.1.159	DC_12-30-66_n77	143

5.1.159.1	Configuration for EN-DC	143

5.1.159.2	∆TIB and ∆RIB values	144

5.1.159.3	Reference sensitivity exceptions	144

5.1.160	DC_14-30-66_n77	144

5.1.160.1	Configuration for EN-DC	144

5.1.160.2	∆TIB and ∆RIB values	144

5.1.160.3	Reference sensitivity exceptions	145

5.1.161	DC_29-30-66_n77	145

5.1.161.1	Configuration for EN-DC	145

5.1.161.2	∆TIB and ∆RIB values	145

5.1.161.3	Reference sensitivity exceptions	145

5.1.162	DC_1-3-7_n38	146

5.1.162.1	Configurations for EN-DC	146

5.1.162.2	∆TIB and ∆RIB values	146

5.1.162.3	Reference sensitivity exceptions	146

5.1.163	DC_1-7-20_n38	146

5.1.163.1	Configurations for EN-DC	146

5.1.163.2	∆TIB and ∆RIB values	146

5.1.163.3	Reference sensitivity exceptions	147

5.1.164	DC_3-7-20_n38	147

5.1.164.1	Configurations for EN-DC	147

5.1.164.2	∆TIB and ∆RIB values	147

5.1.164.3	Reference sensitivity exceptions	147

5.1.165	DC_1-3-32_n28	148

5.1.165.1	 Configurations for EN-DC	148

5.1.165.2	 ∆TIB and ∆RIB values	148

5.1.165.3	 Reference sensitivity exceptions	148

5.1.166	DC_3-7-32_n28	148

5.1.166.1	 Configurations for EN-DC	148

5.1.166.2	 ∆TIB and ∆RIB values	149

5.1.166.3	 Reference sensitivity exceptions	149

5.1.167	DC_3-20-32_n28	149

5.1.167.1	 Configurations for EN-DC	149

5.1.167.2	 ∆TIB and ∆RIB values	149

5.1.167.3	 Reference sensitivity exceptions	150

5.1.168	DC_3-28-32_n1	150

5.1.168.1	 Configurations for EN-DC	150

5.1.168.2	 ∆TIB and ∆RIB values	150

5.1.168.3	 Reference sensitivity exceptions	150

5.1.170	 DC_2-5-48_n77	151

5.1.170.1	Configuration for EN-DC	151

5.1.170.2	∆TIB and ∆RIB values	151

5.1.170.3	REFSENS requirements	151

5.1.171	 DC_5-48-66_n77	152

5.1.171.1	Configuration for EN-DC	152

5.1.171.2	∆TIB and ∆RIB values	152

5.1.171.3	REFSENS requirements	152

5.1.172	 DC_2-13-48_n77	152

5.1.172.1	Configuration for EN-DC	152

5.1.172.2	∆TIB and ∆RIB values	152

5.1.172.3	REFSENS requirements	153

5.1.173.1	Configuration for EN-DC	153

5.1.173.2	∆TIB and ∆RIB values	153

5.1.173.3	REFSENS requirements	153

5.1.174	DC_2-7-28_n78	154

5.1.174.1	 Configurations for EN-DC	154

5.1.174.2	 ∆TIB and ∆RIB values	154

5.1.174.3	 Reference sensitivity exceptions	154

5.1.175	DC_5-7-66_n78	154

5.1.175.1	 Configurations for EN-DC	154

5.1.175.2	 ∆TIB and ∆RIB values	155

5.1.175.3	 Reference sensitivity exceptions	155

5.1.178.2	Configuration for DC	157

5.1.179.2	Configuration for DC	158

5.1.180.2	Configuration for DC	159

5.1.181.2	Configuration for DC	160

5.1.182	DC_1-8-20_n3	160

5.1.182.1	Configuration for EN-DC	160

5.1.182.2	∆TIB and ∆RIB values	161

5.1.182.3	Reference sensitivity exceptions	161

5.1.183	DC_1-8-28_n3	161

5.1.183.1	Configuration for EN-DC	161

5.1.183.2	∆TIB and ∆RIB values	161

5.1.183.3	Reference sensitivity exceptions	161

5.1.184	DC_1-8-28_n78	162

5.1.184.1	Configuration for EN-DC	162

5.1.184.2	∆TIB and ∆RIB values	162

5.1.184.3	Reference sensitivity exceptions	162

5.1.185	DC_1-8-32_n3	162

5.1.185.1	Configuration for EN-DC	162

5.1.185.2	∆TIB and ∆RIB values	162

5.1.185.3	Reference sensitivity exceptions	163

5.1.186	DC_1-8-32_n78	163

5.1.186.1	Configuration for EN-DC	163

5.1.186.2	∆TIB and ∆RIB values	163

5.1.186.3	Reference sensitivity exceptions	163

5.1.187	DC_1-20-28_n78	163

5.1.187.1	Configuration for EN-DC	163

5.1.187.2	∆TIB and ∆RIB values	164

5.1.187.3	Reference sensitivity exceptions	164

5.1.188	DC_1-20-38_n8	164

5.1.188.1	Configuration for EN-DC	164

5.1.188.2	∆TIB and ∆RIB values	164

5.1.188.3	Reference sensitivity exceptions	164

5.1.189	DC_3-8-28_n78	165

5.1.189.1	Configuration for EN-DC	165

5.1.189.2	∆TIB and ∆RIB values	165

5.1.189.3	Reference sensitivity exceptions	165

5.1.191	DC_3-8-32_n1	166

5.1.191.1	Configuration for EN-DC	166

5.1.191.2	∆TIB and ∆RIB values	166

5.1.191.3	Reference sensitivity exceptions	166

5.1.192	DC_3-8-32_n78	167

5.1.192.1	Configuration for EN-DC	167

5.1.192.2	∆TIB and ∆RIB values	167

5.1.192.3	Reference sensitivity exceptions	167

5.1.193	DC_3-20-28_n78	167

5.1.193.1	Configuration for EN-DC	167

5.1.193.2	∆TIB and ∆RIB values	167

5.1.193.3	Reference sensitivity exceptions	168

5.1.194	DC_7-8-32_n78	168

5.1.194.1	Configuration for EN-DC	168

5.1.194.2	∆TIB and ∆RIB values	168

5.1.194.3	Reference sensitivity exceptions	168

5.1.195	DC_7-8-38_n1	168

5.1.195.1	Configuration for EN-DC	168

5.1.195.2	∆TIB and ∆RIB values	169

5.1.195.3	Reference sensitivity exceptions	169

5.1.196	DC_7-20-38_n8	169

5.1.196.1	Configuration for EN-DC	169

5.1.196.2	∆TIB and ∆RIB values	169

5.1.196.3	Reference sensitivity exceptions	169

5.1.197	DC_7-28-38_n1	170

5.1.197.1	Configuration for EN-DC	170

5.1.197.2	∆TIB and ∆RIB values	170

5.1.197.3	Reference sensitivity exceptions	170

5.1.198	DC_8-20-28_n78	170

5.1.198.1	Configuration for EN-DC	170

5.1.198.2	∆TIB and ∆RIB values	170

5.1.198.3	Reference sensitivity exceptions	171

5.1.199	DC_8-20-38_n1	171

5.1.199.1	Configuration for EN-DC	171

5.1.199.2	∆TIB and ∆RIB values	171

5.1.199.3	Reference sensitivity exceptions	171

5.1.200	DC_8-32-38_n1	171

5.1.200.1	Configuration for EN-DC	171

5.1.200.2	∆TIB and ∆RIB values	172

5.1.200.3	Reference sensitivity exceptions	172

5.1.201	DC_20-28-38_n1	172

5.1.201.1	Configuration for EN-DC	172

5.1.201.2	∆TIB and ∆RIB values	172

5.1.201.3	Reference sensitivity exceptions	172

5.1.202	DC_28-32-38_n1	173

5.1.202.1	Configuration for EN-DC	173

5.1.202.2	∆TIB and ∆RIB values	173

5.1.202.3	Reference sensitivity exceptions	173

5.1.204	DC_n77_1-3_8	173

5.1.204.1	 Configurations for NE-DC	173

5.1.204.2	 ∆TIB and ∆RIB values	173

5.1.205	DC_2-30-(n)5	174

5.1.205.1	Configuration for EN-DC	174

5.1.205.2	∆TIB and ∆RIB values	174

5.1.205.3	Reference sensitivity exceptions	174

5.1.206	DC_30-66-(n)5	174

5.1.206.1	Configuration for EN-DC	174

5.1.206.2	∆TIB and ∆RIB values	175

5.1.206.3	Reference sensitivity exceptions	175

5.1.207	DC_2-13-66_n2	175

5.1.207.1	Configuration for EN-DC	175

5.1.207.2	∆TIB and ∆RIB values	176

5.1.207.3	Reference sensitivity exceptions	176

5.1.208	DC_3-32-38_n28	176

5.1.208.1	 Configurations for EN-DC	176

5.1.208.2	Co-existence studies	176

5.1.208.3 ∆TIB and ∆RIB values	176

5.1.208.4	 Reference sensitivity exceptions	177

5.1.209	DC_3-8-32_n28	177

5.1.209.1	Configurations for DC	177

5.1.209.2	Co-existence studies	177

5.1.209.3	∆TIB and ∆RIB values	180

5.1.209.4	Reference sensitivity exceptions	180

REFSENS exceptions are not needed.Annex A - Change history	180

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document is a technical report for Dual Connectivity (DC) of 3 LTE bands (3DL/1UL) and 1 NR band (1DL/1UL) under Rel-17 time frame. The purpose is to gather the relevant background information and studies in order to address Dual Connectivity (DC) of 3 LTE band (3DL/1UL) and 1 NR band (1DL/1UL) for the Rel-17 band combinations. The co-existence analysis and RF front end requirements such as ΔRIB,c and ΔTIB,c are described based on the band combination basis since such information have no difference between the DC configurations consisting with the same E-UTRA band and the same NR band. The actual requirements are added to the corresponding technical specification.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-200664, “New WID: Dual Connectivity (EN-DC) of 3 bands LTE inter-band CA (3DL/1UL) and 1 NR band (1DL/1UL)”, RAN#88-e

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

# 4	Background

The present document is a technical report for Dual Connectivity (DC) of 3 bands LTE inter-band CA and 1 NR band under Rel-17 timeframe. The document covers each band combination specific issues (i.e. one sub-clause defined per band combination)

## 4.1	TR maintenance

A single company is responsible for introducing all approved TPs in the current TR, i.e. TR editor. However, it is the responsibility of the contact person of each band combination to ensure that the TPs related to the band combination have been implemented.

# 5	DC of 3 LTE band (3DL/1UL) + 1 NR band: Specific Band Combination Part

<Editor’s note: The requirements for specific band combinations shall be described according to the same manner as specified in TS38.101-3.>

## 5.1	Inter-band EN-DC

## 5.1.1	DC_1-3_(n)41

### 5.1.1.1	Configurations for DC

Table 5.1.1.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A_(n)41AA | DC_1A_n41A DC_3A_n41A |

### 5.1.1.2		∆TIB and ∆RIB values

Table 5.1.1.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_(n)41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.1.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3_(n)41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.1.3	REFSENS requirements

### No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.2		DC_1-3-41_n28

### 5.1.2.1	Configuration for EN-DC

Table 5.1.2.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-41A_n28A | DC_1A_n28A DC_3A_n28A DC_41A_n28A |
| DC_1A-3A-41C_n28A | DC_1A_n28A DC_3A_n28A DC_41A_n28A DC_41C_n28A |

### 5.1.2.2	∆TIB and ∆RIB values

Table 5.1.2.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n28 | 0.6 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

Table 5.1.2.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n28 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n28 | 0.2 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545 – 2690 MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496 – 2545 MHz. |  |  |

### 5.1.2.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.3	DC_3-7-8_n40

### 5.1.3.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-8A_n40A | DC_3A_n40A DC_7A_n40A DC_8A_n40A |

### 5.1.3.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8_n40 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n40 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-8_n40 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n40 | 0.5 |

### 5.1.3.3		Reference sensitivity exceptions

No further REFSENS exceptions needed.

## 5.1.4	DC_3-7-28_n1

### 5.1.4.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |

### 5.1.4.2	∆TIB and ∆RIB values

Table 5.1.4.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.5 |
|  | n1 | 0.6 |

Table 5.1.4.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.4.3		Reference sensitivity exceptions

REFSENS exceptions needed due to band 28 uplink harmonic into band n1 is already specified for DC_28A_n1A.

## 5.1.5	DC_5-7-66_n66

### 5.1.5.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A-66A_n66A DC_5A-7C-66A_n66A | DC_5A_n66A DC_7A_n66A DC_66A_n66A2 |
| NOTE 2: Only single switched UL is supported |  |

### 5.1.5.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_5-7-66_n66 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n66 |  |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_5-7-66_n66 | 5 | 0.3 |
|  | 7 | 0 |
|  | 66 | 0.3 |
|  | n66 |  |

### 5.1.5.3		Reference sensitivity exceptions

MSD have been defined for lower order combinations. No further MSD is needed.

## 5.1.6	DC_3-19-42_n1

### 5.1.6.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-19A-42A_n1A DC_3A-19A-42C_n1A | DC_3A_n1A DC_19A_n1A DC_42A_n1A |

### 5.1.6.2	∆TIB and ∆RIB values

For DC_3-19-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-19-42, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-19-42_n1 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-19-42_n1 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |

### 5.1.6.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.7	DC_3-21-42_n1

### 5.1.7.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-21A-42A_n1A DC_3A-21A-42C_n1A | DC_3A_n1A DC_21A_n1A DC_42A_n1A |

### 5.1.7.2	∆TIB and ∆RIB values

For DC_3-21-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-21-42, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-21-42_n1 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
|  | n1 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-21-42_n1 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
|  | n1 | 0.2 |

### 5.1.7.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.8	DC_19-21-42_n1

### 5.1.8.1	Configuration for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_19A-21A-42A_n1A DC_19A-21A-42C_n1A | DC_19A_n1A DC_21A_n1A DC_42A_n1A |

### 5.1.8.2	∆TIB and ∆RIB values

For DC_19-21-42_n1, the TIB,c and RIB,c values are reused from the LTE combination CA_1-19-21-42, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_19-21-42_n1 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
|  | n1 | 0.3 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_19-21-42_n1 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
|  | n1 | 0 |

### 5.1.8.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.9		DC_2-28-66_n66

### 5.1.9.1	 Operating bands for EN-DC

Table 5.1.9.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-28-66_n66 | CA_2-28-66 | n66 |

### 5.1.9.2 	Configuration for EN-DC

Table 5.1.9.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-28A-66A_n66A | DC_2A_n66A DC_28A_n66A DC_66A_n66A4 | CA_2A-28A-66A | n66A |
| NOTE 4: Only single switched UL is supported. |  |  |  |

### 5.1.9.3	 ∆TIB and ∆RIB values

Table 5.1.9.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-28-66_n66 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.9.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-28-66_n66 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

## 5.1.10		DC_7-28-66_n66

### 5.1.10.1	 Operating bands for EN-DC

Table 5.1.10.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_7-28-66_n66 | CA_7-28-66 | n66 |

### 5.1.10.2 	Configuration for EN-DC

Table 5.1.10.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-28A-66A_n66A DC_7C-28A-66A_n66A | DC_7A_n66A DC_28A_n66A DC_66A_n66A4 | CA_7A-28A-66A CA_7C-28A-66A | n66A |
| NOTE 4: Only single switched UL is supported. |  |  |  |

### 5.1.10.3	 ∆TIB and ∆RIB values

Table 5.1.10.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28-66_n66 | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.10.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28-66_n66 | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

## 5.1.11		DC_2-7-28_n66

### 5.1.11.1	 Operating bands for EN-DC

Table 5.1.11.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band |
| --- | --- | --- |
| DC_2-7-28_n66 | CA_2-7-28 | n66 |

### 5.1.11.2 	Configuration for EN-DC

Table 5.1.11.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-28A_n66A DC_2A-7C-28A_n66A | DC_2A_n66A DC_7A_n66A DC_28A_n66A | CA_2A-7A-28A CA_2A-7C-28A | n66A |

### 5.1.11.3	 ∆TIB and ∆RIB values

Table 5.1.11.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-28_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
|  | n66 | 0.5 |

Table 5.1.11.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-28_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | n66 | 0.5 |

## 5.1.12	DC_1-8-11_n3

### 5.1.12.1	Configurations for EN-DC

Table 5.1.12.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A-11A_n3A | DC_1A_n3A DC_8A_n3A DC_11A_n3A | CA_1A-8A-11A | n3A |

### 5.1.12.2	∆TIB and ∆RIB values

For DC_1-8-11_n3, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.12.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.8 |
|  | n3 | 0.9 |

Table 5.1.12.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n3 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0.3 |
|  | n3 | 0.5 |

### 5.1.12.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-11_n3 was covered by the studies for the fallback modes of DC_1-8_n3, DC_1-11_n3 and DC_8-11_n3. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.13	DC_1-8-42_n28

### 5.1.13.1	Configurations for EN-DC

Table 5.1.13.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_1A-8A-42A_n28A | DC_1A_n28A DC_8A_n28A DC_42A_n28A | CA_1A-8A-42A | n28A |
| DC_1A-8A-42C_n28A | DC_1A_n28A DC_8A_n28A DC_42A_n28A DC_42C_n28A | CA_1A-8A-42C | n28A |

### 5.1.13.2	∆TIB and ∆RIB values

For DC_1-8-42_n28, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.13.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 5.1.13.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

### 5.1.13.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-42_n28 was covered by the studies for the fallback modes of DC_1-8_n28, DC_1-42_n28 and DC_8-42_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.14	DC_1-7-32_n28

### 5.1.14.1	Configuration for EN-DC

Table 5.1.14.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n28A | DC_1A_n28A DC_7A_n28A |

### 5.1.14.2	∆TIB and ∆RIB values

Table 5.1.14.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-32_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | n28 | 0.7 |

Table 5.1.14.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-32_n28 | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.14.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.15	DC_1-7-32_n78

### 5.1.15.1	Configuration for EN-DC

Table 5.1.15.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n78A | DC_1A_n78A DC_7A_n78A |

### 5.1.15.2	∆TIB and ∆RIB values

Table 5.1.15.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-32_n78 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

Table 5.1.15.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-32_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 32 | 0 |
|  | n78 | 0.8 |

### 5.1.15.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.16	DC_1-20-32_n28

### 5.1.16.1	Configuration for EN-DC

Table 5.1.16.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-32A_n28A | DC_1A_n28A DC_20A_n28A |

### 5.1.16.2	∆TIB and ∆RIB values

Table 5.1.16.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-32_n28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | n28 | 0.7 |

Table 5.1.16.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-32_n28 | 1 | 0 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.16.3	Reference sensitivity exceptions

Exceptions for the B1 IMD5 hit from the 20A_n28A UL are TBD.

## 5.1.17	DC_1-20-32_n78

### 5.1.17.1	Configuration for EN-DC

Table 5.1.17.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-32A_n78A | DC_1A_n78A DC_20A_n78A |

### 5.1.17.2	∆TIB and ∆RIB values

Table 5.1.17.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-32_n78 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.17.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-32_n78 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.1.17.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.18	DC_3-7-32_n78

### 5.1.18.1	Configuration for EN-DC

Table 5.1.18.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-32A_n78A DC_3C-7A-32A_n78A | DC_3A_n78A DC_3C_n78A DC_7A_n78A |

### 5.1.18.2	∆TIB and ∆RIB values

Table 5.1.18.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-32_n78 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.18.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-32_n78 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.1.18.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 have been solved by fallback band combinations DC_3A-32A_n78A and DC_7A-32A_n78A.

## 5.1.19	DC_3-20-32_n78

### 5.1.19.1	Configuration for EN-DC

Table 5.1.19.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-20A-32A_n78A | DC_3A_n78A DC_20A_n78A |

### 5.1.19.2	∆TIB and ∆RIB values

Table 5.1.19.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-32_n78 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.19.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-20-32_n78 | 3 | 0.2 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

### 5.1.19.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.20	DC_7-20-32_n1

### 5.1.20.1	Configuration for EN-DC

Table 5.1.20.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |

### 5.1.20.2	∆TIB and ∆RIB values

Table 5.1.20.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-32_n1 | 7 | 0.6 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |

Table 5.1.20.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-32_n1 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.1.20.3	Reference sensitivity exceptions

Exceptions for IMD hits on B32 are TBD.

## 5.1.21	DC_7-20-32_n28

### 5.1.21.1	Configuration for EN-DC

Table 5.1.21.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n28A | DC_7A_n28A DC_20A_n28A |

### 5.1.21.2	∆TIB and ∆RIB values

Table 5.1.21.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-32_n28 | 7 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.7 |

Table 5.1.21.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-32_n28 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.21.3	Reference sensitivity exceptions

Compared to its fallback modes, there are no additional MSD requirements for this band combination.

## 5.1.22	DC_1-20-32_n3

### 5.1.22.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_1A-20A-32A_n3A | DC_1A_n3A DC_20A_n3A |

### 5.1.22.2	∆TIB and ∆RIB values

For DC_1-20-32_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-32_n3 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | n3 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-20-32_n3 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

### 5.1.22.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.23	DC_2-4-7_n28

### 5.1.23.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-4A-7A_n28A | DC_2A_n28A DC_4A_n28A DC_7A_n28A |

### 5.1.23.2	∆TIB and ∆RIB values

For DC_2-4-7_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-4-7_n28 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-4-7_n28 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.2 |

### 5.1.23.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.24	DC_2-5-7_n66

### 5.1.24.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |

### 5.1.24.2	∆TIB and ∆RIB values

For DC_2-5-7_n66, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.24.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.25	DC_2-5-66_n7

### 5.1.25.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-66A_n7A DC_2A-5A-66A-66A_n7A | DC_2A_n7A DC_5A_n7A DC_66A_n7A |

### 5.1.25.2	∆TIB and ∆RIB values

For DC_2-5-66_n7, the TIB,c and RIB,c values are reused from the DC_2-7-13_n66, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n7 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.25.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.26	DC_2-5-66_n66

### 5.1.26.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-5A-66A_n66A | DC_2A_n66A DC_5A_n66A DC_66A_n66A1 |
| NOTE1: Only single switched UL is supported |  |

### 5.1.26.2	∆TIB and ∆RIB values

For DC_2-5-66_n66, the TIB,c and RIB,c values are reused from the DC_2-5_n66, and are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.1.26.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.27	DC_2-7-66_n28

### 5.1.27.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_2A-7A-66A_n28A | DC_2A_n28A DC_7A_n28A DC_66A_n28A |

### 5.1.27.2	∆TIB and ∆RIB values

For DC_2-7-66_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n28 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n28 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n28 | 0.2 |

### 5.1.27.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.28	DC_3-20-32_n1

### 5.1.28.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band configurations EN-DC (four bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-20A-32A_n1A | DC_3A_n1A DC_20A_n1A |

### 5.1.28.2	∆TIB and ∆RIB values

For DC_3-20-32_n1, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-32_n1 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n1 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-32_n1 | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.1.28.3	REFSENS requirements

No additional MSD requirement is needed.

## 5.1.29	DC_1-3-18_n3

### 5.1.29.1	Configuration for DC

Table 5.1.29.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-18A_n3A | DC_1A_n3A DC_3A_n3A2 DC_18A_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.1.29.2	∆TIB and ∆RIB values

For DC_1-3-18_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-18A.

Table 5.1.29.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.29.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n3 | 0 |

### 5.1.29.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.30	DC_1-3-41_n3

### 5.1.30.1	Configuration for DC

Table 5.1.30.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-41A_n3A DC_1A-3A-41C_n3A | DC_1A_n3A DC_3A_n3A2 DC_41A_n3A DC_41C_n3A |
| NOTE 2: Only single switched UL is supported |  |

### 5.1.30.2	∆TIB and ∆RIB values

For DC_1-3-41_n3, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-41A.

Table 5.1.30.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n3 | 0.5 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.30.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n3 | 0 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.30.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.31	DC_1-3-41_n41

### 5.1.31.1	Configuration for DC

Table 5.1.31.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_1A-3A-41A_n41A | DC_1A_n41A DC_3A_n41A |

### 5.1.31.2	∆TIB and ∆RIB values

For DC_1-3-41_n41, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_1A-3A-41A.

Table 5.1.31.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.31/0.82 |
|  | n41 | 0.31/0.82 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

Table 5.1.31.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-41_n41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 01/0.52 |
|  | n41 | 01/0.52 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. NOTE 2: Applicable for the frequency range of 2496-2515 MHz. |  |  |

### 5.1.31.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.32	DC_2-5-7_n66 and DC_2-5-7-7_n66

### 5.1.32.1	Configuration for DC

Table 5.1.32.1-1: Inter-band EN-DC configurations (four bands)

| DC configuration | Uplink configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A-7A_n66A DC_2A-5A-7C_n66A DC_2A-5A-7A-7A_n66A | DC_2A_n66A DC_5A_n66A DC_7A_n66A |

### 5.1.32.2	∆TIB and ∆RIB values

For DC_2-5-7_n66 and DC_2-5-7-7_n66, the TIB,c and RIB,c values are given in the tables below. Numbers come from LTE CA_2A-5A-7A-66A.

Table 5.1.32.2-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 DC_2-5-7-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.32.2-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n66 DC_2-5-7-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.32.3	REFSENS requirements

There are no additional MSD requirements for this band combination.

## 5.1.33	DC_1-3-11_n28

### 5.1.33.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-11A_n28A | DC_1A_n28A DC_3A_n28A DC_11A_n28A |

### 5.1.33.2	∆TIB and ∆RIB values

For DC_1-3-11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-11_n28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-11_n28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

### 5.1.33.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-11_n28 was covered by the studies for the fallback modes of DC_1-3_n28, DC_1-11_n28 and DC_3-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.34	DC_1-3-11_n77

### 5.1.34.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-11A_n77A | DC_1A_n77A DC_3A_n77A DC_11A_n77A |
| DC_1A-3A-11A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_11A_n77A |

### 5.1.34.2	∆TIB and ∆RIB values

For DC_1-3-11_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-11_n77 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-11_n77 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

### 5.1.34.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-11_n77 was covered by the studies for the fallback modes of DC_1-3_n77, DC_1-11_n77 and DC_3-11_n77.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.35	DC_3-8-11_n28

### 5.1.35.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-11A_n28A | DC_3A_n28A DC_8A_n28A DC_11A_n28A |

### 5.1.35.2	∆TIB and ∆RIB values

For DC_3-8-11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-11_n28 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8-11_n28 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n28 | 0.2 |

### 5.1.35.3	Reference sensitivity exceptions

Co-existence study for DC_3-8-11_n28 was covered by the studies for the fallback modes of DC_3-8_n28, DC_3-11_n28 and DC_8-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.36	DC_3-8-11_n77

### 5.1.36.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-11A_n77A | DC_3A_n77A DC_8A_n77A DC_11A_n77A |
| DC_3A-8A-11A_n77(2A) | DC_3A_n77A DC_8A_n77A DC_11A_n77A |

### 5.1.36.2	∆TIB and ∆RIB values

For DC_3-8-11_n77, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-11_n77 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-8-11_n77 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | n77 | 0.5 |

### 5.1.36.3	Reference sensitivity exceptions

Co-existence study for DC_3-8-11_n77 was covered by the studies for the fallback modes of DC_3-8_n77, DC_3-11_n77 and DC_8-11_n77.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.37	DC_1-8-11_n28

### 5.1.37.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-11A_n28A | DC_1A_n28A DC_8A_n28A DC_11A_n28A |

### 5.1.37.2	∆TIB and ∆RIB values

For DC_1-8-11_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | n28 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-11_n28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | n28 | 0.2 |

### 5.1.37.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-11_n28 was covered by the studies for the fallback modes of DC_1-8_n28, DC_1-11_n28 and DC_8-11_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.38		DC_1-3-18_n28

### 5.1.38.1	Configuration for EN-DC

Table 5.1.38.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-18A_n28A | DC_1A_n28A DC_3A_n28A DC_18A_n28A |

### 5.1.38.2	∆TIB and ∆RIB values

Table 5.1.38.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n28 | 0.6 |

Table 5.1.38.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n28 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n28 | 0.2 |

### 5.1.38.3	REFSENS requirements

## No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.39		DC_1-3-18_n41

### 5.1.39.1	Configuration for EN-DC

Table 5.1.39.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-18A_n41A | DC_1A_n41A DC_3A_n41A DC_18A_n41A |

### 5.1.39.2	∆TIB and ∆RIB values

Table 5.1.39.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n41 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
|  | n41 | 0.31 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

Table 5.1.39.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-18-n41 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
|  | n41 | 01 |
| NOTE 1: Applicable for the frequency range of 2515-2690 MHz. |  |  |

### 5.1.39.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.40	DC_2-7-28_n7

### 5.1.40.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-28A_n7A | DC_2A_n7A DC_7A_n7A4 DC_28A_n7A |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.40.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-7-28_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-7-28_n7 | 2 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
|  | n7 | 0 |

5.1.40.3		Reference sensitivity exceptions

No further MSD is needed defined.

## 5.1.41	DC_2A-66A-71A_n71A

### 5.1.41.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-66A-71A_n71A | DC_2A_n71A DC_66A_n71A |

Note that DC_71_n71 is not used as uplink configuration.

### 5.1.41.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-66-71_n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n71 |  |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-66-71_n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0 |
|  | n71 |  |

5.1.41.3		Reference sensitivity exceptions

REFSENS exceptions needed due to band 71 uplink harmonic into band 2 is already specified in Table 7.3B.2.3.1-1 of TS 38.101-3.

## 5.1.42	DC_2-5-66_n77A

### 5.1.42.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n77A DC_2A-2A-5A-66A_n77A DC_2A-5A-66A-66A_n77A | DC_2A_n77A DC_5A_n77A DC_66A_n77A |

### 5.1.42.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-5-66_n77 DC_2-2-5-66_n77 DC_2-5-66-66_n77 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.42.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

## 5.1.43	DC_2-13-66_n77A

### 5.1.43.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-66A_n77A DC_2A-2A-13A-66A_n77A DC_2A-13A-66A-66A_n77A | DC_2A_n77A DC_13A_n77A DC_66A_n77A |

### 5.1.43.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-13-66-66_n77 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n77 DC_2-2-13-66_n77 DC_2-13-66-66_n77 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.43.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

## 5.1.44	DC_2-48-66_n77A

### 5.1.44.1	Configurations for EN-DC

Table 5.5B.4.3-1: Inter-band EN-DC configurations within FR1 (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n77A | DC_2A_n77A DC_48A_n77A DC_66A_n77A |

### 5.1.44.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-48-66_n77 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-48-66_n77 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n77 | 0.5 |

5.1.44.3		Reference sensitivity exceptions

REFSENS exception have when needed been defined for lower order combinations.

## 5.1.45		DC_1-3-40_n78

### 5.1.45.1	Configuration for EN-DC

Table 5.1.45.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-40A_n78A DC_1A-3A-40C_n78A | DC_1A_n78A DC_3A_n78A DC_40A_n78A |

### 5.1.45.2	∆TIB and ∆RIB values

Table 5.1.45.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-40_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.45.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-40_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.45.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.46		DC_1-7-40_n78

### 5.1.46.1	Configuration for EN-DC

Table 5.1.46.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-40A_n78A DC_1A-7A-40C_n78A | DC_1A_n78A DC_7A_n78A DC_40A_n78A |

### 5.1.46.2	∆TIB and ∆RIB values

Table 5.1.46.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-40_n78 | 1 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.46.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-7-40_n78 | 1 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.46.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.47		DC_1-8-40_n78

### 5.1.47.1	Configuration for EN-DC

Table 5.1.47.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-8A-40A_n78A DC_1A-8A-40C_n78A | DC_1A_n78A DC_8A_n78A DC_40A_n78A |

### 5.1.47.2	∆TIB and ∆RIB values

Table 5.1.47.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-40_n78 | 1 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.47.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-8-40_n78 | 1 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.47.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.48		DC_3-7-40_n78

### 5.1.48.1	Configuration for EN-DC

Table 5.1.48.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-40A_n78A DC_3A-7A-40C_n78A | DC_3A_n78A DC_7A_n78A DC_40A_n78A |

### 5.1.48.2	∆TIB and ∆RIB values

Table 5.1.48.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n78 | 3 | 0.6 |
|  | 7 | 0.5 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.48.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-7-40_n78 | 3 | 0.2 |
|  | 7 | 0 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.48.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.49		DC_3-8-40_n78

### 5.1.49.1	Configuration for EN-DC

Table 5.1.49.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-8A-40A_n78A DC_3A-8A-40C_n78A | DC_3A_n78A DC_8A_n78A DC_40A_n78A |

### 5.1.49.2	∆TIB and ∆RIB values

Table 5.1.49.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n78 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.49.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n78 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.49.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.50		DC_7-8-40_n78

### 5.1.50.1	Configuration for EN-DC

Table 5.1.50.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-8A-40A_n78A DC_7A-8A-40C_n78A | DC_7A_n78A DC_8A_n78A DC_40A_n78A |

### 5.1.50.2	∆TIB and ∆RIB values

Table 5.1.50.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n78 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.35 |
|  | n78 | 0.85 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

Table 5.1.50.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n78 | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one NR band and without simultaneous Rx/Tx. |  |  |

### 5.1.50.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.51	DC_1-7-8_n28

### 5.1.51.1	 Configurations for EN-DC

Table 5.1.51.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-8A_n28A | DC_1A_n28A DC_7A_n28A DC_8A_n28A |

### 5.1.51.2	 ∆TIB and ∆RIB values

Table 5.1.51.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-8_n28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.51.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-8_n28 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.2 |

### 5.1.51.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.52	DC_3-7-8_n28

### 5.1.52.1	 Configurations for EN-DC

Table 5.1.52.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-8A_n28A | DC_3A_n28A DC_7A_n28A DC_8A_n28A |

### 5.1.52.2	 ∆TIB and ∆RIB values

Table 5.1.52.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-8_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
|  | n28 | 0.5 |

Table 5.1.52.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-8_n28 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | n28 | 0.1 |

### 5.1.52.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.53	DC_1-7-28_n3

### 5.1.53.1	 Configurations for EN-DC

Table 5.1.53.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-28A_n3A | DC_1A_n3A DC_7A_n3A DC_28A_n3A |

### 5.1.53.2	 ∆TIB and ∆RIB values

Table 5.1.53.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-28_n3 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.53.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-28_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |

### 5.1.53.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.54	DC_3-8-40_n1

### 5.1.54.1	 Configurations for EN-DC

Table 5.1.54.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-8A-40A_n1A DC_3A-8A-40C_n1A | DC_3A_n1A DC_8A_n1A DC_40A_n1A |

### 5.1.54.2	 ∆TIB and ∆RIB values

Table 5.1.54.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-40_n1 | 3 | 0.5 |
|  | 8 | 0.5 |
|  | 40 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.54.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8-40_n1 | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0.2 |
|  | n1 | 0.1 |

### 5.1.54.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.55	DC_7-8-40_n1

### 5.1.55.1	 Configurations for EN-DC

Table 5.1.55.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-8A-40A_n1A DC_7A-8A-40C_n1A | DC_7A_n1A DC_8A_n1A DC_40A_n1A |

### 5.1.55.2	 ∆TIB and ∆RIB values

Table 5.1.55.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-8-40_n1 | 7 | 0.8 |
|  | 8 | 0.6 |
|  | 40 | 0.9 |
|  | n1 | 0.6 |

Table 5.1.55.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-8-40_n1 | 7 | 0.3 |
|  | 8 | 0.2 |
|  | 40 | 0.8 |
|  | n1 | 0 |

### 5.1.55.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.56	DC_2-28-66_n7

### 5.1.56.1	 Configurations for EN-DC

Table 5.1.56.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-28A-66A_n7A | DC_2A_n7A DC_28A_n7A DC_66A_n7A |

### 5.1.56.2	 ∆TIB and ∆RIB values

Table 5.1.56.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-28-66_n7 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.56.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-28-66_n7 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.56.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.57	DC_2-5-7_n7

### 5.1.57.1	 Configurations for EN-DC

Table 5.1.57.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-7A_n7A | DC_2A_n7A DC_5A_n7A DC_7A_n7A1 |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.57.2	 ∆TIB and ∆RIB values

Table 5.1.57.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.57.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n7 | 2 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
|  | n7 | 0 |

### 5.1.57.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.58	DC_2-7-66_n7/DC_2-7-66-66_n7

### 5.1.58.1	 Configurations for EN-DC

Table 5.1.58.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-66A_n7A DC_2A-7A-66A-66A_n7A | DC_2A_n7A DC_7A_n7A1 DC_66A_n7A |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.58.2	 ∆TIB and ∆RIB values

Table 5.1.58.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.58.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n7 DC_2-7-66-66_n7 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.58.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.59	DC_5-7-66_n7/DC_5-7-66-66_n7

### 5.1.59.1	 Configurations for EN-DC

Table 5.1.59.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A-66A_n7A DC_5A-7A-66A-66A_n7A | DC_5A_n7A DC_7A_n7A1 DC_66A_n7A |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.59.2	 ∆TIB and ∆RIB values

Table 5.1.59.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.59.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-66_n7 DC_5-7-66-66_n7 | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.59.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.60	DC_7-28-66_n7

### 5.1.60.1	 Configurations for EN-DC

Table 5.1.60.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-28A-66A_n7A | DC_7A_n7A1 DC_78A_n7A DC_66A_n7A |
| NOTE 1: Only single switched UL is supported. |  |

### 5.1.60.2	 ∆TIB and ∆RIB values

Table 5.1.60.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-28-66_n7 | 7 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

Table 5.1.60.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-28-66_n7 | 7 | 0.5 |
|  | 28 | 0.2 |
|  | 66 | 0.5 |
|  | n7 | 0.5 |

### 5.1.60.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.61	DC_2-7-66_n77

### 5.1.61.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-66A_n77A DC_2A-7A-7A-66A_n77A DC_2A-7A-66A_n77(2A) DC_2A-7A-7A-66A_n77(2A) DC_2A-7C-66A_n77A DC_2A-7C-66A_n77(2A) | DC_2A_n77A DC_7A_n77A DC_66A_n77A |

### 5.1.61.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-7-66_n77 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-7-66_n77 | 2 | 0.2 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.61.3		Reference sensitivity exceptions

No further REFSENS exceptions needed.

## 5.1.62	DC_1-20-40_n78

### 5.1.62.1	Configuration for EN-DC

Table 5.1.62.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-40A_n78A | DC_1A_n78A DC_20A_n78A DC_40A_n78A |

### 5.1.62.2	∆TIB and ∆RIB values

It is proposed to re-use relaxation values from DC_1-20_n41-n78 which is very similar.

Table 5.1.62.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-40_n78 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | 40 | 0.59 |
|  | n78 | 0.89 |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.62.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-40_n78 | 1 | 0 |
|  | 20 | 0 |
|  | 40 | 09 |
|  | n78 | 0.89 |
| NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.62.3	Reference sensitivity exceptions

MSD have been defined for lower order combinations. No further MSD is needed.

### 5.1.63	DC_1-8-42_n3

#### 5.1.63.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-42A_n3A DC_1A-8A-42C_n3A | DC_1A_n3A DC_8A_n3A DC_42A_n3A DC_42C_n3A |

#### 5.1.63.2	∆TIB and ∆RIB values

For DC_1-8-42_n3, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-42_n3 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
|  | n3 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-42_n3 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
|  | n3 | 0.2 |

#### 5.1.63.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-42_n3 was covered by the studies for the fallback modes of DC_1-8_n3, DC_1-42_n3 and DC_8-42_n3.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.64	DC_1-3-42_n28

#### 5.1.64.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-42A_n28A | DC_1A_n28A DC_3A_n28A DC_42A_n28A |
| DC_1A-3A-42C_n28A | DC_1A_n28A DC_3A_n28A DC_42A_n28A DC_42C_n28A |

#### 5.1.64.2	∆TIB and ∆RIB values

For DC_1-3-42_n28, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-42_n28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | n28 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-42_n28 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | n28 | 0.5 |

#### 5.1.64.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-42_n28 was covered by the studies for the fallback modes of DC_1-3_n28, DC_1-42_n28 and DC_3-42_n28.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.65	DC_2-29-66_n78

#### 5.1.65.1	Configuration for EN-DC

Table 5.1.65.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-29A-66A_n78A | DC_2A_n78A DC_66A_n78A |

#### 5.1.65.2	∆TIB and ∆RIB values

### For DC_2A-29A-66A_n78A, the TIB,c and RIB,c values are reused from the LTE combination CA_2-13-48-66, and are given in the tables below

Table 5.1.65.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-66_n78 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.65.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-29-66_n78 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |

#### 5.1.65.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.66	DC_7-8-32_n1

#### 5.1.66.1	Configuration for EN-DC

Table 5.1.66.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-32A_n1A | DC_7A_n1A DC_8A_n1A |

#### 5.1.66.2	∆TIB and ∆RIB values

Table 5.1.66.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-32A_n1A | 7 | 0.7 |
|  | 8 | 0.6 |
|  | 32 | N/A |
|  | n1 | 0.7 |

Table 5.1.66.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-32A_n1A | 7 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.66.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.67	DC_7-20-32_n78

#### 5.1.67.1	Configuration for EN-DC

Table 5.1.67.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n78A | DC_7A_n78A DC_20A_n78A |

#### 5.1.67.2	∆TIB and ∆RIB values

Table 5.1.67.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n78A | 7 | 0.7 |
|  | 20 | 0.5 |
|  | 32 | N/A |
|  | n78 | 0.8 |

Table 5.1.67.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n78A | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n78 | 0.5 |

#### 5.1.67.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.68	DC_2A-12A-66A_n41A

## 5.1.68.1	Operating bands for EN-DC

Table 5.1.68.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12-66_n41 | CA_2-12-66 | n41 |  |

### 5.1.68.2	Configuration for DC

Table 5.1.68.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n41A | DC_2A_n41A, DC_12A_n41A, DC_66A_n41A | CA_2A-12A-66A | n41A |
| DC_2A-2A-12A-66A_n41A | DC_2A_n41A, DC_12A_n41A, DC_66A_n41A | CA_2A-2A-12A-66A | n41A |

### 5.1.68.3		∆TIB and ∆RIB values

For DC_2-12-66_n41, the TIB,c and RIB,c values are given in the tables below based on values CA_2-7-12-66 in 36.101.

Table 5.1.68.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-66_n41 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n41 | 0.5 |

Table 5.1.68.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-66_n41 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n41 | 0.5 |

### 5.1.68.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.69	DC_2A-66A-71A_n41A

## 5.1.69.1	Operating bands for EN-DC

Table 5.1.69.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66-71_n41 | CA_2-66-71 | n41 |  |

### 5.1.69.2	Configuration for DC

Table 5.1.69.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n41A | DC_2A_n41A, DC_66A_n41A DC_71A_n41A | CA_2A-66A-71A | n41A |
| DC_2A-2A-66A-71A_n41A | DC_2A_n41A, DC_66A_n41A DC_71A_n41A | CA_2A-2A-66A-71A | n41A |

### 5.1.69.3		∆TIB and ∆RIB values

For DC_2-66-71_n41, the TIB,c and RIB,c values are given in the tables below, based on values for DC_2-66-n41_n71 in 38.101-3.

Table 5.1.69.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-71_n41 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.8 |
|  | n41 | 0.81 |
|  |  | 1.32 |

Table 5.1.69.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-71_n41 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0.5 |
|  | n41 | 0.51 |
|  |  | 12 |

### 5.1.69.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.70	DC_2A-7A-12A_n66A

## 5.1.70.1	Operating bands for EN-DC

Table 5.1.70.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-12_n66 | CA_2-7-12 | n66 |  |

### 5.1.70.2	Configuration for DC

Table 5.1.70.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-12A_n66A | DC_2A_n66A, DC_7A_n66A DC_12A_n66A | CA_2A-7A-12A | n66A |
| DC_2A-2A-7A-12A_n66A | DC_2A_n66A, DC_7A_n66A DC_12A_n66A | CA_2A-2A-7A-12A | n66A |

### 5.1.70.3		∆TIB and ∆RIB values

For DC_2-7-12_n66, the TIB,c and RIB,c values are given in the tables below. Based on values for CA_2-7-12-66 in 36.101.

Table 5.1.70.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-12_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
|  | n66 | 0.5 |

Table 5.1.70.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-12_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
|  | n66 | 0.3 |

### 5.1.70.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.71	DC_2A_2A-5A-7A_n66A

## 5.1.71.1	Operating bands for EN-DC

Table 5.1.71.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-2-5-7_n66 | CA_2-5-7 | n66 |  |

### 5.1.71.2	Configuration for DC

Table 5.1.71.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-2A-5A-7A_n66A | DC_2A_n66A, DC_5A_n66A DC_7A_n66A | CA_2A-2A-5A-7A | n66A |

### 5.1.71.3		∆TIB and ∆RIB values

For DC_2-5-7_n66, the TIB,c and RIB,c values are given in the tables below. Based on values for DC_2-7-13_n66 in 38.101-3.

Table 5.1.71.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-2-5-7_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.71.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-2-5-7_n66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 7 | 0.5 |
|  | n66 | 0.5 |

### 5.1.71.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.72	DC_2A-7A-71A_n66A

## 5.1.72.1	Operating bands for EN-DC

Table 5.1.72.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-71_n66 | CA_2-7-71 | n66 |  |

### 5.1.72.2	Configuration for DC

Table 5.1.72.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-71A_n66A | DC_2A_n66A, DC_7A_n66A DC_71A_n66A | CA_2A-7A-71A | n66A |
| DC_2A-2A-7A-71A_n66A | DC_2A_n66A, DC_7A_n66A DC_71A_n66A | CA_2A-2A-7A-71A | n66A |

### 5.1.72.3		∆TIB and ∆RIB values

For DC_2-7-71_n66, the TIB,c and RIB,c values are given in the tables below, based on values for DC_2-7-66_n71 in 38.101-3.

Table 5.1.72.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-71_n66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 71 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.72.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-71_n66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 71 | 0 |
|  | n66 | 0.3 |

### 5.1.72.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.73	DC_2A-7A-12A_n78A

## 5.1.73.1	Operating bands for EN-DC

Table 5.1.73.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-12_n78 | CA_2-7-12 | n78 |  |

### 5.1.73.2	Configuration for DC

Table 5.1.73.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-12A_n78A | DC_2A_n78A, DC_7A_n78A DC_12A_n78A | CA_2A-7A-12A | n78A |
| DC_2A-2A-7A-12A_n78A | DC_2A_n78A, DC_7A_n78A DC_12A_n78A | CA_2A-2A-7A-12A | n78A |

### 5.1.73.3		∆TIB and ∆RIB values

For DC_2A-7A-12A_n78A, the TIB,c and RIB,c values are given in the tables below based on values for DC_1-7-28_n78 in 38.101-3.

Table 5.1.73.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-12_n78 | 2 | 0.6 |
|  | 7 | 0.6 |
|  | 12 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.73.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-12_n78 | 2 | 0.2 |
|  | 7 | 0.2 |
|  | 12 | 0.2 |
|  | n78 | 0.5 |

### 5.1.73.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.74	DC_2A-12A-66A_n78A

## 5.1.74.1	Operating bands for EN-DC

Table 5.1.74.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-12-66_n78 | CA_2-12-66 | n78 |  |

### 5.1.74.2	Configuration for DC

Table 5.1.74.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-12A-66A_n78A | DC_2A_n78A, DC_12A_n78A, DC_66A_n78A | CA_2A-12A-66A | n78A |
| DC_2A-2A-12A-66A_n78A | DC_2A_n78A, DC_12A_n78A, DC_66A_n78A | CA_2A-2A-12A-66A | n78A |

### 5.1.74.3		∆TIB and ∆RIB values

For DC_2-12-66_n78, the TIB,c and RIB,c values are given in the tables below based on values DC_2-48-66_n12 in 38.101-3.

Table 5.1.74.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-12-66_n78 | 2 | 0.6 |
|  | 12 | 0.3 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.74.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-12-66_n78 | 2 | 0.3 |
|  | 12 | 0 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |

### 5.1.74.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.75	DC_7A-12A-66A_n78A

## 5.1.75.1	Operating bands for EN-DC

Table 5.1.75.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-12-66_n78 | CA_7-12-66 | n78 |  |

### 5.1.75.2	Configuration for DC

Table 5.1.75.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A-66A_n78A | DC_7A_n78A, DC_12A_n78A, DC_66A_n78A | CA_7A-12A-66A | n78A |

### 5.1.75.3		∆TIB and ∆RIB values

For DC_7-12-66_n78, the TIB,c and RIB,c values are given in the tables below based on values for CA_3-28-41-42 in 36.101.

Table 5.1.75.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12-66_n78 | 7 | 0.8 |
|  | 12 | 0.5 |
|  | 66 | 1 |
|  | n78 | 0.8 |

Table 5.1.75.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12-66_n78 | 7 | 0.5 |
|  | 12 | 0.2 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |

### 5.1.75.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.76	DC_7A-66A-71A_n78A

## 5.1.76.1	Operating bands for EN-DC

Table 5.1.76.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66-71_n78 | CA_7-66-71 | n78 |  |

### 5.1.76.2	Configuration for DC

Table 5.1.76.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A-71A_n78A | DC_7A_n78A, DC_66A_n78A, DC_71A_n78A | CA_7A-66A-71A | n78A |

### 5.1.76.3		∆TIB and ∆RIB values

For DC_7-66-71_n78, the TIB,c and RIB,c values are given in the tables below based on values for DC_3-7-20_n78 in 38.101-3.

Table 5.1.76.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66-71_n78 | 7 | 0.6 |
|  | 66 | 0.6 |
|  | 71 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.76.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66-71_n78 | 7 | 0.2 |
|  | 66 | 0.2 |
|  | 71 | 0 |
|  | n78 | 0.5 |

### 5.1.76.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.77	DC_2A-7A -71A_n78A

## 5.1.77.1	Operating bands for EN-DC

Table 5.1.77.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-71_n78 | CA_2-7-71 | n78 |  |

### 5.1.77.2	Configuration for DC

Table 5.1.77.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -71A_n78A | DC_2A_n78A, DC_7A_n78A, DC_71A_n78A | CA_2A-7A-71A | n78A |
| DC_2A-2A-7A -71A_n78A | DC_2A_n78A, DC_7A_n78A, DC_71A_n78A | CA_2A-2A-7A-71A | n78A |

### 5.1.77.3		∆TIB and ∆RIB values

For DC_2-7-71_n78, the TIB,c and RIB,c values are given in the tables below based on values for DC_1-7-28_n78.

Table 5.1.77.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-71_n78 | 2 | 0.6 |
|  | 7 | 0.6 |
|  | 71 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.77.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-71_n78 | 2 | 0.2 |
|  | 7 | 0.2 |
|  | 71 | 0.2 |
|  | n78 | 0.5 |

### 5.1.77.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.78	DC_2A-7A -66A_n2A

## 5.1.78.1	Operating bands for EN-DC

Table 5.1.78.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-66_n2 | CA_2-7-66 | n2 |  |

### 5.1.78.2	Configuration for DC

Table 5.1.78.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -66A_n2A | DC_7A_n2A, DC_66A_n2A | CA_2A-7A-66A | n2A |

### 5.1.78.3		∆TIB and ∆RIB values

For DC_2-7-66_n2, the TIB,c and RIB,c values are given in the tables below, based on values for CA_2-7-66 in 36.101.

Table 5.1.78.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n2 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.78.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n2 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.3 |

### 5.1.78.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.79	DC_2A-5A -7A_n2A

## 5.1.79.1	Operating bands for EN-DC

Table 5.1.79.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5-7_n2 | CA_2-5-7 | n2 |  |

### 5.1.79.2	Configuration for DC

Table 5.1.79.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A -7A_n2A | DC_5A_n2A DC_7A_n2A | CA_2A-5A-7A | n2A |

### 5.1.79.3		∆TIB and ∆RIB values

For DC_2-5-7_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-7 in 36.101.

Table 5.1.79.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n2 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
|  | n2 | 0.3 |

Table 5.1.79.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n2 | 2 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
|  | n2 | 0 |

### 5.1.79.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.80	DC_5A-7A -66A_n2A

## 5.1.80.1	Operating bands for EN-DC

Table 5.1.80.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-7-66_n2 | CA_5-7-66 | n2 |  |

### 5.1.80.2	Configuration for DC

Table 5.1.80.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-7A -66A_n2A | DC_5A_n2A, DC_7A_n2A, DC_66A_n2A | CA_5A-7A-66A | n2A |

### 5.1.80.3		∆TIB and ∆RIB values

For DC_5-7-66_n2, the TIB,c and RIB,c values are given in the tables below based on values for DC_2-7-13_n66 in 38.101-3.

Table 5.1.80.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-66_n2 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.80.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-66_n2 | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n2 | 0.3 |

### 5.1.80.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.81	DC_2A-7A -71A_n2A

## 5.1.81.1	Operating bands for EN-DC

Table 5.1.81.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-71_n2 | CA_2-7-71 | n2 |  |

### 5.1.81.2	Configuration for DC

Table 5.1.81.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -71A_n2A | DC_7A_n2A, DC_71A_n2A | CA_2A-7A-71A | n2A |

### 5.1.81.3		∆TIB and ∆RIB values

For DC_2-7-71_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-7_n71 in 38.101-3.

Table 5.1.81.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-71_n2 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 71 | 0.6 |
|  | n2 | 0.5 |

Table 5.1.81.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-71_n2 | 2 | 0 |
|  | 7 | 0 |
|  | 71 | 0.2 |
|  | n2 | 0 |

### 5.1.81.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.82	DC_2A-66A -71A_n2A

## 5.1.82.1	Operating bands for EN-DC

Table 5.1.82.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-66-71_n2 | CA_2-66-71 | n2 |  |

### 5.1.82.2	Configuration for DC

Table 5.1.82.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-66A -71A_n2A | DC_66A_n2A, DC_71A_n2A | CA_2A-66A-71A | n2A |

### 5.1.82.3		∆TIB and ∆RIB values

For DC_2-66-71_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-66-71 in 36.101.

Table 5.1.82.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66-71_n2 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.82.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66-71_n2 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0 |
|  | n2 | 0.3 |

### 5.1.82.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.83	DC_2A-7A -12A_n2A

## 5.1.83.1	Operating bands for EN-DC

Table 5.1.83.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-12_n2 | CA_2-7-12 | n2 |  |

### 5.1.83.2	Configuration for DC

Table 5.1.83.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A -12A_n2A | DC_7A_n2A, DC_12A_n2A | CA_2A-7A-12A | n2A |

### 5.1.83.3		∆TIB and ∆RIB values

For DC_2-7-12_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-7_12 in 36.101.

Table 5.1.83.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-12_n2 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.83.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-12_n2 | 2 | 0 |
|  | 7 | 0 |
|  | 12 | 0 |
|  | n2 | 0 |

### 5.1.83.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.84	DC_7A-66A-71A_n2A

## 5.1.84.1	Operating bands for EN-DC

Table 5.1.84.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-66-71_n2 | CA_7-66-71 | n2 |  |

### 5.1.84.2	Configuration for DC

Table 5.1.84.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-66A-71A_n2A | DC_7A_n2A, DC_66A_n2A, DC_71A_n2A | CA_7A-66A-71A | n2A |

### 5.1.84.3		∆TIB and ∆RIB values

For DC_7-66-71_n2, the TIB,c and RIB,c values are given in the tables below based on values for DC_2-7-66_n71.

Table 5.1.84.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-66-71_n2 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.84.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-66-71_n2 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0 |
|  | n2 | 0.3 |

### 5.1.84.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.85	DC_7A-12A-66A_n2A

## 5.1.85.1	Operating bands for EN-DC

Table 5.1.85.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 7-12-66_n2 | CA_7-12-66 | n2 |  |

### 5.1.85.2	Configuration for DC

Table 5.1.85.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_7A-12A-66A_n2A | DC_7A_n2A, DC_12A_n2A, DC_66A_n2A | CA_7A-12A-66A | n2A |

### 5.1.85.3		∆TIB and ∆RIB values

For DC_7-12-66_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-4-7-12 in 36.101.

Table 5.1.85.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-12-66_n2 | 7 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.85.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-12-66_n2 | 7 | 0.5 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.85.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.86		DC_1-28-40_n78

### 5.1.86.1	Configuration for EN-DC

Table 5.1.86.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-28A-40A_n78A | DC_1A_n78A DC_28A_n78A DC_40A_n78A |

### 5.1.86.2	∆TIB and ∆RIB values

Proposed relaxations are same as for DC_1-28_n40-n78.

Table 5.1.86.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-28-40_n78 | 1 | 0.5 |
|  | 28 | 0.5 |
|  | 40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.86.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-28-40_n78 | 1 | 0 |
|  | 28 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.86.3	REFSENS requirements

No additional MSD requirement need to be defined for this EN-DC configuration.

## 5.1.87		DC_3-28-40_n78

### 5.1.87.1	Configuration for EN-DC

Table 5.1.87.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-28A-40A_n78A | DC_3A_n78A DC_28A_n78A DC_40A_n78A |

### 5.1.87.2	∆TIB and ∆RIB values

Proposed relaxations are same as for DC_3-28_n40-n78.

Table 5.1.87.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28-40_n78 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 40 | 0.36 |
|  | n78 | 0.86 |
| NOTE 6: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

Table 5.1.87.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-28-40_n78 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 40 | 0.45 |
|  | n78 | 0.55 |
| NOTE 5: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. |  |  |

### 5.1.87.3	REFSENS requirements

No additional MSD requirement need to be defined for this EN-DC configuration.

## 5.1.88		DC_1-11-18_n3

### 5.1.88.1	Configuration for EN-DC

Table 5.1.88.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A-18A_n3A | DC_1A_n3A DC_11A_n3A DC_18A_n3A |

### 5.1.88.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-11, CA_1-3-18, CA_3-11-18, CA_1-11-18 and are given in the tables below.

Table 5.1.88.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n3 | 1 | 0.3 |
|  | 11 | 0.9 |
|  | 18 | 0.3 |
|  | n3 | 0.8 |

Table 5.1.88.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n3 | 1 | 0 |
|  | 11 | 0.5 |
|  | 18 | 0 |
|  | n3 | 0.3 |

### 5.1.88.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.89		DC_1-11-18_n28

### 5.1.89.1	Configuration for EN-DC

Table 5.1.89.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A-18A_n28A | DC_1A_n28A DC_11A_n28A DC_18A_n28A |

### 5.1.89.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-11-18, CA_1-11-28, CA_1-18-28 and are given in the tables below.

Table 5.1.89.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n28 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.4 |
|  | n28 | 0.6 |

Table 5.1.89.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n28 | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
|  | n28 | 0.1 |

### 5.1.89.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.90		DC_1-11-18_n41

### 5.1.90.1	Configuration for EN-DC

Table 5.1.90.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-11A-18A_n41A | DC_1A_n41A DC_11A_n41A DC_18A_n41A |

### 5.1.90.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-11-18, CA_1-18-41 and are given in the tables below.

Table 5.1.90.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n41 | 1 | 0.5 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
|  | n41 | 0.5 |

Table 5.1.90.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-11-18_n41 | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
|  | n41 | 0 |

### 5.1.90.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.91		DC_2-5-66_n48

### 5.1.91.1	Configuration for EN-DC

Table 5.1.91.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-66A_n48A DC_2A-5A-66A_n48B DC_2A-5A-66A-66A_n48A DC_2A-5A-66A-66A_n48B | DC_2A_n48A DC_5A_n48A DC_66A_n48A |

### 5.1.91.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 38.101-3 DC_2-5-66_n77, and are given in the tables below.

Table 5.1.91.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
|  | n48 | 0.8 |

Table 5.1.91.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n48 DC_2-5-66-66_n48 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
|  | n48 | 0.5 |

### 5.1.91.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.92		DC_2-13-48_n77

### 5.1.92.1	Configuration for EN-DC

Table 5.1.92.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-48A_n77A | DC_2A_n77A, DC_13A_n77A, DC_48A_n77A |

### 5.1.92.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-13-48, and are given in the tables below.

Table 5.1.92.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-48_n77 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.92.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13-48_n77 | 2 | 0.2 |
|  | 13 | 0 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.1.92.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.93		DC_2-46-48_n2

### 5.1.93.1	Configuration for EN-DC

Table 5.1.93.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-46A-48A_n2A DC_2A-46C-48A_n2A DC_2A-46D-48A_n2A DC_2A-46E-48A_n2A | DC_2A_n2A4 DC_48A_n2A |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.93.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-46-48, and are given in the tables below

Table 5.1.93.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-48_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | n2 | 0.6 |

Table 5.1.93.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-46-48_n2 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | n2 | 0.3 |

### 5.1.93.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.94		DC_2-48-66_n2

### 5.1.94.1	Configuration for EN-DC

Table 5.1.94.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n2A DC_2A-48C-66A_n2A DC_2A-48D-66A_n2A DC_2A-48E-66A_n2A | DC_66A_n2A DC_48A_n2A DC_2A_n2A4 |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.94.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-48-66, and are given in the tables below

Table 5.1.94.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n2 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n2 | 0.6 |

Table 5.1.94.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n2 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

### 5.1.94.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.95		DC_2-48-66_n66

### 5.1.95.1	Configuration for EN-DC

Table 5.1.95.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-48A-66A_n66A DC_2A-48C-66A_n66A DC_2A-48D-66A_n66A DC_2A-48E-66A_n66A | DC_66A_n66A4 DC_48A_n66A DC_2A_n66A |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.95.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_2-48-66 and TS 38.101-3 DC_2-48_n66, and are given in the tables below.

Table 5.1.95.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n66 | 0.6 |

Table 5.1.95.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-48-66_n66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
|  | n66 | 0.3 |

### 5.1.95.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.96		DC_13-48-66_n77

### 5.1.96.1	Configuration for EN-DC

Table 5.1.96.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_13A-48A-66A_n77A | DC_13A_n77A, DC_66A_n77A, |

### 5.1.96.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are refer to TS 36.101 CA_13-48-66, and are given in the tables below.

Table 5.1.96.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48-66_n77 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.96.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-48-66_n77 | 13 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

### 5.1.96.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.97	DC_1-3-20_n7

### 5.1.97.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-3A-20A_n7A | DC_1A_n7A DC_3A_n7A DC_20A_n7A |

### 5.1.97.2	∆TIB and ∆RIB values

For DC_1-3-20_n7, the same TIB,c and RIB,c values of DC_1-7-20_n3 are adopted and given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-20_n7 | 1 | 0.3 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | n7 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-20_n7 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | n7 | 0 |

### 5.1.97.3	Reference sensitivity exceptions

Co-existence study for DC_1-3-20_n7 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.98	DC_1-3-38_n28

### 5.1.98.1	 Configurations for EN-DC

Table 5.1.98.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-38A_n28A DC_1A-3C-38A_n28A | DC_1A_n28A DC_3A_n28A DC_38A_n28A |

### 5.1.98.2	 ∆TIB and ∆RIB values

Table 5.1.98.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-38_n28 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 38 | 0.5 |
|  | n28 | 0.6 |

Table 5.1.98.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-38_n28 | 1 | 0 |
|  | 3 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

### 5.1.98.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.99	DC_1-7-38_n28

### 5.1.99.1	 Configurations for EN-DC

Table 5.1.99.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-7A-38A_n28AX | DC_1A_n28A |
| NOTE X: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

### 5.1.99.2	 ∆TIB and ∆RIB values

Table 5.1.99.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n28 | 1 | 0.3 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n28 | 0.6 |

Table 5.1.99.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-7-38_n28 | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n28 | 0.2 |

### 5.1.99.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.100	DC_3-7-38_n28

### 5.1.100.1	 Configurations for EN-DC

Table 5.1.100.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-38A_n28AX DC_3C-7A-38A_n28AX | DC_3A_n28A |
| NOTE X: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |

### 5.1.100.2	 ∆TIB and ∆RIB values

Table 5.1.100.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-38_n28 | 3 | 0.3 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n28 | 0.3 |

Table 5.1.100.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-38_n28 | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n28 | 0.2 |

### 5.1.100.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.101	DC_2-5-30_n2

### 5.1.101.1	Operating bands for EN-DC

Table 5.1.101.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5-30_n2 | CA_2-5-30 | n2 |  |

### 5.1.101.2	Configuration for DC

Table 5.1.101.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-30A_n2A | DC_2A_n2A1 DC_5A_n2A DC_30A_n2A | CA_2A-5A-30A | n2A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.101.3		∆TIB and ∆RIB values

For DC_2-5-30_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-30 in 36.101.

Table 5.1.101.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30_n2 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n2 | 0.5 |

Table 5.1.101.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-30_n2 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | n2 | 0.4 |

### 5.1.101.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.102	DC_2-5-30_n66

### 5.1.102.1	Operating bands for EN-DC

Table 5.1.102.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-5-30_n66 | CA_2-5-30 | n66 |  |

### 5.1.102.2	Configuration for DC

Table 5.1.102.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A | CA_2A-5A-30A | n66 |
| DC_2A-2A-5A-30A_n66A | DC_2A_n66A DC_5A_n66A DC_30A_n66A | CA_2A-2A-5A-30A | n66 |

### 5.1.102.3		∆TIB and ∆RIB values

For DC_2-5-30_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-30-66 in 36.101.

Table 5.1.102.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-30_n66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.102.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-30_n66 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.102.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.103	DC_2-14-30_n2

### 5.1.103.1	Operating bands for EN-DC

Table 5.1.103.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-14-30_n2 | CA_2-14-30 | n2 |  |

### 5.1.103.2	Configuration for DC

Table 5.1.103.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A-30A_n2A | DC_2A_n2A1 DC_14A_n2A DC_30A_n2A | CA_2A-14A-30A | n2A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.103.3		∆TIB and ∆RIB values

For DC_2-14-30_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-14-30 in 36.101.

Table 5.1.103.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-30_n2 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.103.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14-30_n2 | 2 | 0.3 |
|  | 14 | 0 |
|  | 30 | 0.3 |
|  | n2 | 0.3 |

### 5.1.103.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.104	DC_2-29-30_n66

### 5.1.104.1	Operating bands for EN-DC

Table 5.1.104.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-29-30_n66 | CA_2-29-30 | n66 |  |

### 5.1.104.2	Configuration for DC

Table 5.1.104.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A | CA_2A-29A-30A | n66 |
| DC_2A-2A-29A-30A_n66A | DC_2A_n66A DC_30A_n66A | CA_2A-2A-29A-30A | n66 |

### 5.1.104.3		∆TIB and ∆RIB values

For DC_2-29-30_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-29-30-66 in 36.101.

Table 5.1.104.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-30_n66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.104.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29-30_n66 | 2 | 0.4 |
|  | 29 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.104.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.105	DC_2-46-66_n5

### 5.1.105.1	Operating bands for EN-DC

Table 5.1.105.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46-66_n5 | CA_2-46-66 | n5 |  |

### 5.1.105.2	Configuration for DC

Table 5.1.105.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-46A-66A | n5 |
| DC_2A-46C-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-46C-66A | n5 |
| DC_2A-46D-66A_n5A | DC_2A_n5A DC_66A_n5A | CA_2A-46D-66A | n5 |

### 5.1.105.3		∆TIB and ∆RIB values

For DC_2-46-66_n5, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-46-66 in 36.101.

Table 5.1.105.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-66_n5 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.105.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46-66_n5 | 2 | 0.3 |
|  | 46 | 0 |
|  | 66 | 0.3 |
|  | n5 | 0 |

### 5.1.105.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.106	DC_5-30-66_n2

### 5.1.106.1	Operating bands for EN-DC

Table 5.1.106.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-30-66_n2 | CA_5-30-66 | n2 |  |

### 5.1.106.2	Configuration for DC

Table 5.1.106.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-30A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A | CA_5A-30A-66A | n2A |
| DC_5A-30A-66A-66A_n2A | DC_5A_n2A DC_30A_n2A DC_66A_n2A | CA_5A-30A-66A-66A | n2A |

### 5.1.106.3		∆TIB and ∆RIB values

For DC_5-3A-66_n2, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-5-30-66 in 36.101.

Table 5.1.106.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30-66_n2 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.106.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30-66_n2 | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

### 5.1.106.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.107	DC_5-30-66_n66

### 5.1.107.1	Operating bands for EN-DC

Table 5.1.107.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 5-30-66_n66 | CA_5-30-66 | n66 |  |

### 5.1.107.2	Configuration for DC

Table 5.1.107.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_5A-30A-66A_n66A | DC_5A_n66A DC_30A_n66A DC_66A_n66A1 | CA_5A-30A-66A | n66A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.107.3		∆TIB and ∆RIB values

For DC_5-30-66_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_5-30-66 in 36.101.

Table 5.1.107.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-30-66_n66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.107.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-30-66_n66 | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

### 5.1.107.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.108	DC_14-30-66_n66

### 5.1.108.1	Operating bands for EN-DC

Table 5.1.108.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-30-66_n66 | CA_14-30-66 | n66 |  |

### 5.1.108.2	Configuration for DC

Table 5.1.108.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A-66A_n66A | DC_14A_n66A DC_30A_n66A DC_66A_n66A1 | CA_14A-30A-66A | n66A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.108.3		∆TIB and ∆RIB values

For DC_14-30-66_n66, the TIB,c and RIB,c values are given in the tables below based on values for CA_14-30-66 in 36.101.

Table 5.1.108.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30-66_n66 | 14 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n66 | 0.5 |

Table 5.1.108.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30-66_n66 | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n66 | 0.4 |

### 5.1.108.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.109	DC_14-30-66_n2

### 5.1.109.1	Operating bands for EN-DC

Table 5.1.109.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 14-30-66-66_n2 | CA_14-30-66 | n2 |  |

### 5.1.109.2	Configuration for DC

Table 5.1.109.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_14A-30A-66A_n2A | DC_14A_n2A, DC_30A_n2A DC_66A_n2A | CA_14A-30A-66A | n2A |
| DC_14A-30A-66A-66A_n2A | DC_14A_n2A, DC_30A_n2A DC_66A_n2A | CA_14A-30A-66A-66A | n2A |

### 5.1.109.3		∆TIB and ∆RIB values

For DC_14-30-66_n2, the TIB,c and RIB,c values are given in the tables below based on values CA_2-14-30-66 in 36.101.

Table 5.1.109.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_14-30-66-n2 | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.109.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_14-30-66-n2 | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n2 | 0.4 |

### 5.1.109.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.110	DC_2-2-14-30_n66

### 5.1.110.1	Operating bands for EN-DC

Table 5.1.110.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-14-30_n66 | CA_2-14-30 | n66 |  |

### 5.1.110.2	Configuration for DC

Table 5.1.110.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A1 | CA_2A-14A-30A | n66A |
| DC_2A-2A-14A-30A_n66A | DC_2A_n66A DC_14A_n66A DC_30A_n66A DC_66A_n66A1 | CA_2A-2A-14A-30A | n66A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.110.3		∆TIB and ∆RIB values

For DC_2-2-14-30_n66, the TIB,c and RIB,c values are given in the tables below based on values CA_2-14-30-66 in 36.101.

Table 5.1.110.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-14-30_n66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | n66 | 0.5 |

Table 5.1.110.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-14-30_n66 | 2 | 0.4 |
|  | 14 | 0 |
|  | 30 | 0.5 |
|  | n66 | 0.4 |

### 5.1.110.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.111	DC_1-3-7_n3

### 5.1.111.1	Operating bands for EN-DC

Table 5.1.111.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3-7_n3 | CA_1-3-7 | n3 |  |

### 5.1.111.2	Configuration for DC

Table 5.1.111.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-7A_n3A | DC_1A_n3A DC_3A_n3A1 DC_7A_n3A | CA_1A-3A-7A | n3A |
| DC_1A-3A-7C_n3A | DC_1A_n3A DC_3A_n3A1 DC_7A_n3A DC_7C_n3A | CA_1A-3A-7C | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.111.3		∆TIB and ∆RIB values

For DC_1-3-7_n3, the TIB,c and RIB,c values are given in the tables below based on values for CA_1-3-7 in 36.101.

Table 5.1.111.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n3 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.111.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-7_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | n3 | 0 |

### 5.1.111.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.112	1-3-28_n3

### 5.1.112.1	Operating bands for EN-DC

Table 5.1.112.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 1-3-28_n3 | CA_1-3-28 | n3 |  |

### 5.1.112.2	Configuration for DC

Table 5.1.112.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A-28A_n3A | DC_1A_n3A DC_3A_n3A1 DC_28A_n3A | CA_1A-3A-28A | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.112.3		∆TIB and ∆RIB values

For DC_1-3-28_n3, the TIB,c and RIB,c values are given in the tables below based on values for CA_1-3-28 in 36.101.

Table 5.1.112.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-28_n3 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |

Table 5.1.112.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-28_n3 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | n3 | 0 |

### 5.1.112.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.113	3-7-28_n3

### 5.1.113.1	Operating bands for EN-DC

Table 5.1.113.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 3-7-28_n3 | CA_3-7-28 | n3 |  |

### 5.1.113.2	Configuration for DC

Table 5.1.113.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_3A-7A-28A_n3A | DC_3A_n3A1 DC_7A_n3A DC_28A_n3A | CA_3A-7A-28A | n3A |
| DC_3A-7C-28A_n3A | DC_3A_n3A1 DC_7A_n3A DC_7C_n3A DC_28A_n3A | CA_3A-7C-28A | n3A |
| NOTE 1: Only single switched UL is supported |  |  |  |

### 5.1.113.3		∆TIB and ∆RIB values

For DC_3-7-28_n3, the TIB,c and RIB,c values are given in the tables below based on values for CA_3-7-28 in 36.101.

Table 5.1.113.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-28_n3 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n3 | 0.5 |

Table 5.1.113.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-28_n3 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
|  | n3 | 0 |

### 5.1.113.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.114	DC_2-29-66_n260

### 5.1.114.1	Operating bands for EN-DC

Table 5.1.114.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-29-66_n260 | CA_2-29-66 | n260 |  |

### 5.1.114.2	Configuration for DC

Table 5.1.114.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-29A-66A_n260A DC_2A-29A-66A_n260G DC_2A-29A-66A_n260H DC_2A-29A-66A_n260I DC_2A-29A-66A_n260J DC_2A-29A-66A_n260K DC_2A-29A-66A_n260L DC_2A-29A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-29A-66A | n260A n260G n260H n260I n260J n260K n260L n260M |

### 5.1.114.3		∆TIB and ∆RIB values

For DC_2-29-66_n260, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-29-66 in 36.101.

Table 5.1.114.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-29-66_n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.1.114.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-29-66_n260 | 2 | 0.3 |
|  | 29 | 0 |
|  | 66 | 0.3 |
|  | n260 | 0 |

### 5.1.114.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.115	DC_2-46-66_n260

### 5.1.115.1	Operating bands for EN-DC

Table 5.1.115.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-46-66_n260 | CA_2-46-66 | n260 |  |

### 5.1.115.2	Configuration for DC

Table 5.1.115.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-46A-66A_n260A DC_2A-46A-66A_n260G DC_2A-46A-66A_n260H DC_2A-46A-66A_n260I DC_2A-46A-66A_n260J DC_2A-46A-66A_n260K DC_2A-46A-66A_n260L DC_2A-46A-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46A-66A | n260A n260G n260H n260I n260J n260K n260L n260M |
| DC_2A-46C-66A_n260A DC_2A-46C-66A_n260G DC_2A-46C-66A_n260H DC_2A-46C-66A_n260I DC_2A-46C-66A_n260J DC_2A-46C-66A_n260K DC_2A-46C-66A_n260L DC_2A-46C-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46C-66A | n260A n260G n260H n260I n260J n260K n260L n260M |
| DC_2A-46D-66A_n260A DC_2A-46D-66A_n260G DC_2A-46D-66A_n260H DC_2A-46D-66A_n260I DC_2A-46D-66A_n260J DC_2A-46D-66A_n260K DC_2A-46D-66A_n260L DC_2A-46D-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46D-66A | n260A n260G n260H n260I n260J n260K n260L n260M |
| DC_2A-46E-66A_n260A DC_2A-46E-66A_n260G DC_2A-46E-66A_n260H DC_2A-46E-66A_n260I DC_2A-46E-66A_n260J DC_2A-46E-66A_n260K DC_2A-46E-66A_n260L DC_2A-46E-66A_n260M | DC_2A_n260A DC_66A_n260A DC_2A_n260G DC_66A_n260G DC_2A_n260H DC_66A_n260H DC_2A_n260I DC_66A_n260I DC_2A_n260J DC_66A_n260J DC_2A_n260K DC_66A_n260K DC_2A_n260L DC_66A_n260L DC_2A_n260M DC_66A_n260M | CA_2A-46E-66A | n260A n260G n260H n260I n260J n260K n260L n260M |

### 5.1.115.3		∆TIB and ∆RIB values

For DC_2-46-66_n260, the TIB,c and RIB,c values are given in the tables below based on values for CA_2-46-66 in 36.101.

Table 5.1.115.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-46-66_n260 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.1.115.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-46-66_n260 | 2 | 0 |
|  | 46 | 0 |
|  | 66 | 0 |
|  | n260 | 0 |

### 5.1.115.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.116	DC_29-30-66_n260

### 5.1.116.1	Operating bands for EN-DC

Table 5.1.116.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 29-30-66_n260 | CA_29-30-66 | n260 |  |

### 5.1.116.2	Configuration for DC

Table 5.1.116.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_29A-30-66A_n260A DC_29A-30-66A_n260G DC_29A-30-66A_n260H DC_29A-30-66A_n260I DC_29A-30-66A_n260J DC_29A-30-66A_n260K DC_29A-30-66A_n260L DC_29A-30-66A_n260M | DC_30A_n260A DC_66A_n260A DC_30A_n260G DC_66A_n260G DC_30A_n260H DC_66A_n260H DC_30A_n260I DC_66A_n260I DC_30A_n260J DC_66A_n260J DC_30A_n260K DC_66A_n260K DC_30A_n260L DC_66A_n260L DC_30A_n260M DC_66A_n260 M | CA_29A-30A-66A | n260A n260G n260H n260I n260J n260K n260L n260M |

### 5.1.116.3		∆TIB and ∆RIB values

For DC_29-30-66_n260, the TIB,c and RIB,c values are given in the tables below based on values for CA_29-30-66 in 36.101.

Table 5.1.116.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_29-30-66_n260 | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n260 | 0 |

Table 5.1.116.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_29-30-66_n260 | 29 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n260 | 0 |

### 5.1.116.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.117	DC_3-20-28_n1

### 5.1.117.1	 Configurations for EN-DC

Table 5.1.117.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-20A-28A_n1A | DC_3A_n1A DC_20A_n1A DC_28A_n1A |

### 5.1.117.2	 ∆TIB and ∆RIB values

Table 5.1.117.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-28_n1 | 3 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.3 |

Table 5.1.117.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-28_n1 | 3 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.117.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.118	DC_7-20-28_n1

### 5.1.118.1	 Configurations for EN-DC

Table 5.1.118.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-20A-28A_n1A | DC_7A_n1A DC_20A_n1A DC_28A_n1A |

### 5.1.118.2	 ∆TIB and ∆RIB values

Table 5.1.118.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-28_n1 | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.118.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_7-20-28_n1 | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.118.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

### 5.1.119.1	Configuration for EN-DC

Table 5.1.119.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-5A_n77A DC_1A-3A-5A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_5A_n77A |

### 5.1.119.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-5 and TS 38.101-3 DC_1-3_n77,DC_1-5_n78 and DC_3-5_n78.

Table 5.1.119.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.119.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-5_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 5 | 0.2 |
|  | n77 | 0.5 |

### 5.1.119.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.120.1	Configuration for EN-DC

Table 5.1.120.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-7A_n77A DC_1A-3A-7A_n77(2A) DC_1A-3A-7A-7A_n77A DC_1A-3A-7A-7A_n77(2A) | DC_1A_n77A DC_3A_n77A DC_7A_n77A |

### 5.1.120.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-3-7 and TS 38.101-3 DC_1-3_n77,DC_1-7_n78 and DC_3-7_n77.

Table 5.1.120.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n77 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.120.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n77 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

### 5.1.120.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.121.1	Configuration for EN-DC

Table 5.1.121.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-5A-7A_n77A DC_1A-5A-7A_n77(2A) DC_1A-5A-7A-7A_n77A DC_1A-5A-7A-7A_n77(2A) | DC_1A_n77A DC_5A_n77A DC_7A_n77A |

### 5.1.121.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_1-5-7 and TS 38.101-3 DC_1-5_n78,DC_1-7_n78 and DC_5-7_n78.

Table 5.1.121.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-5-7_n77 | 1 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.121.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-5-7_n77 | 1 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

### 5.1.121.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

### 5.1.122.1	Configuration for EN-DC

Table 5.1.122.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-5A-7A_n77A DC_3A-5A-7A_n77(2A) DC_3A-5A-7A-7A_n77A DC_3A-5A-7A-7A_n77(2A) | DC_3A_n77A DC_5A_n77A DC_7A_n77A |

### 5.1.122.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from TS 36.101 CA_3-5-7 and TS 38.101-3 DC_3-5_n78,DC_3-7_n77 and DC_5-7_n78.

Table 5.1.122.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-5-7_n77 | 3 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.122.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_3-5-7_n77 | 3 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n77 | 0.5 |

### 5.1.122.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.123	DC_1-7-38_n3

### 5.1.123.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-38A_n3A | DC_1A_n3A |

### 5.1.123.2	∆TIB and ∆RIB values

For DC_1-7-38_n3, same TIB,c and RIB,c values  as LTE CA_1-3-7-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n3 | 1 | 0.6 |
|  | n3 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-38_n3 | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

### 5.1.123.3	Reference sensitivity exceptions

Co-existence study for DC_1-7-38_n3 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.124	DC_1-20-38_n3

### 5.1.124.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-38A_n3A | DC_1A_n3A DC_20A_n3A |

### 5.1.124.2	∆TIB and ∆RIB values

For DC_1-20-38_n3, same TIB,c and RIB,c values  as LTE CA_1-3-20-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-20-38_n3 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | 38 | 0.3 |
|  | n3 | 0.3 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-20-38_n3 | 1 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n3 | 0 |

### 5.1.124.3	Reference sensitivity exceptions

Co-existence study for DC_1-20-38_n3 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.125	DC_7-20-38_n3

### 5.1.125.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-38A_n3A | DC_20A_n3A |

### 5.1.125.2	∆TIB and ∆RIB values

For DC_7-20-38_n3, same TIB,c and RIB,c values  as LTE CA_3-7-28-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-38_n3 | 7 | 0.5 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |
|  | n3 | 0.5 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-38_n3 | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |
|  | n3 | 0 |

### 5.1.125.3	Reference sensitivity exceptions

Co-existence study for DC_7-20-38_n3 was covered by the studies for the fallback modes. No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.126	DC_7-29-66_n78

### 5.1.126.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_7A-29A-66A_n78A | DC_7A_n78A DC_66A_n78A |

### 5.1.126.2	∆TIB and ∆RIB values

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_7-29-66_n78 | 7 | 0.5 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-29-66_n78 | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |

### 5.1.126.3		Reference sensitivity exceptions

No further REFSENS exceptions needed.

### 5.1.127	DC_1-7-32_n3

#### 5.1.127.1	Configuration for EN-DC

Table 5.1.127.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n3A | DC_1A_n3A DC_7A_n3A |

#### 5.1.127.2	∆TIB and ∆RIB values

Table 5.1.127.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-32A_n3A | 1 | 0.6 |
|  | 7 | 0.6 |
|  | n3 | 0.6 |

Table 5.1.127.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-32A_n3A | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.127.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.128	DC_1-7-32_n8

#### 5.1.128.1	Configuration for EN-DC

Table 5.1.128.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-32A_n8A | DC_1A_n8A DC_7A_n8A |

#### 5.1.128.2	∆TIB and ∆RIB values

Table 5.1.128.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-32A_n8A | 1 | 0.7 |
|  | 7 | 0.7 |
|  | n8 | 0.6 |

Table 5.1.128.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-32A_n8A | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n8 | 0.2 |

#### 5.1.128.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.129	DC_1-7-38_n8

#### 5.1.129.1	Configuration for EN-DC

Table 5.1.129.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-7A-38A_n8A | DC_1A_n8A |

#### 5.1.129.2	∆TIB and ∆RIB values

Table 5.1.129.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-7A-38A_n8A | 1 | 0.5 |
|  | n8 | 0.5 |

Table 5.1.129.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-7A-38A_n8A | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
|  | n8 | 0 |

#### 5.1.129.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.130	DC_1-20-28_n3

#### 5.1.130.1	Configuration for EN-DC

Table 5.1.130.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-28A_n3A | DC_1A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.130.2	∆TIB and ∆RIB values

Table 5.1.130.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-28A_n3A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |

Table 5.1.130.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-20A-28A_n3A | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0 |

#### 5.1.130.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.131	DC_1-20-32_n8

#### 5.1.131.1	Configuration for EN-DC

Table 5.1.131.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-32A_n8A | DC_1A_n8A DC_20A_n8A |

#### 5.1.131.2	∆TIB and ∆RIB values

Table 5.1.131.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-32A_n8A | 1 | 0.5 |
|  | 20 | 0.4 |
|  | n8 | 0.4 |

Table 5.1.131.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-20A-32A_n8A | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n8 | 0 |

#### 5.1.131.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.132	DC_1-28-32_n3

#### 5.1.132.1	Configuration for EN-DC

Table 5.1.132.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-28A-32A_n3A | DC_1A_n3A DC_28A_n3A |

#### 5.1.132.2	∆TIB and ∆RIB values

Table 5.1.132.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-28A-32A_n3A | 1 | 0.5 |
|  | 28 | 0.6 |
|  | n3 | 0.5 |

Table 5.1.132.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-28A-32A_n3A | 1 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.132.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.133	DC_3-7-32_n1

#### 5.1.133.1	Configuration for EN-DC

Table 5.1.133.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-7A-32A_n1A | DC_3A_n1A DC_7A_n1A |

#### 5.1.133.2	∆TIB and ∆RIB values

Table 5.1.133.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-7A-32A_n1A | 3 | 0.6 |
|  | 7 | 0.6 |
|  | n1 | 0.6 |

Table 5.1.133.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-7A-32A_n1A | 3 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.133.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.134	DC_3-8-20_n1

#### 5.1.134.1	Configuration for EN-DC

Table 5.1.134.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-20A_n1A | DC_3A_n1A DC_8A_n1A DC_20A_n1A |

#### 5.1.134.2	∆TIB and ∆RIB values

Table 5.1.134.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-8A-20A_n1A | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
|  | n1 | 0.3 |

Table 5.1.134.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-8A-20A_n1A | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | n1 | 0 |

#### 5.1.134.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.135	DC_7-8-20_n1

#### 5.1.135.1	Configuration for EN-DC

Table 5.1.135.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-20A_n1A | DC_7A_n1A DC_8A_n1A DC_20A_n1A |

#### 5.1.135.2	∆TIB and ∆RIB values

Table 5.1.135.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-20A_n1A | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.135.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-20A_n1A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n1 | 0 |

#### 5.1.135.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.136	DC_7-8-20_n3

#### 5.1.136.1	Configuration for EN-DC

Table 5.1.136.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-20A_n3A | DC_7A_n3A DC_8A_n3A DC_20A_n3A |

#### 5.1.136.2	∆TIB and ∆RIB values

Table 5.1.136.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-20A_n3A | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 20 | 0.4 |
|  | n3 | 0.5 |

Table 5.1.136.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-20A_n3A | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0 |
|  | n3 | 0 |

#### 5.1.136.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.137	DC_7-20-28_n3

#### 5.1.137.1	Configuration for EN-DC

Table 5.1.137.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-28A_n3A | DC_7A_n3A DC_20A_n3A DC_28A_n3A |

#### 5.1.137.2	∆TIB and ∆RIB values

Table 5.1.137.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-28A_n3A | 7 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
|  | n3 | 0.5 |

Table 5.1.137.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-28A_n3A | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.1 |
|  | n3 | 0 |

#### 5.1.137.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.138	DC_7-20-32_n1

#### 5.1.138.1	Configuration for EN-DC

Table 5.1.138.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n1A | DC_7A_n1A DC_20A_n1A |

#### 5.1.138.2	∆TIB and ∆RIB values

Table 5.1.138.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n1A | 7 | 0.7 |
|  | 20 | 0.3 |
|  | n1 | 0.7 |

Table 5.1.138.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n1A | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.138.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.139	DC_7-20-32_n3

#### 5.1.139.1	Configuration for EN-DC

Table 5.1.139.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n3A | DC_7A_n3A DC_20A_n3A |

#### 5.1.139.2	∆TIB and ∆RIB values

Table 5.1.139.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n3A | 7 | 0.7 |
|  | 20 | 0.3 |
|  | n3 | 0.3 |

Table 5.1.139.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n3A | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.139.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.140	DC_7-20-32_n8

#### 5.1.140.1	Configuration for EN-DC

Table 5.1.140.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-32A_n8A | DC_7A_n8A DC_20A_n8A |

#### 5.1.140.2	∆TIB and ∆RIB values

Table 5.1.140.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-32A_n8A | 7 | 0.7 |
|  | 20 | 0.6 |
|  | n8 | 0.6 |

Table 5.1.140.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-32A_n8A | 7 | 0 |
|  | 20 | 0.2 |
|  | 32 | 0 |
|  | n8 | 0.2 |

#### 5.1.140.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.141	DC_7-20-38_n1

#### 5.1.141.1	Configuration for EN-DC

Table 5.1.141.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-38A_n1A | DC_20A_n8A |

#### 5.1.141.2	∆TIB and ∆RIB values

Table 5.1.141.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-38A_n1A | 20 | 0.3 |
|  | n1 | 0.5 |

Table 5.1.141.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-38A_n1A | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.141.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.142	DC_7-28-32_n1

#### 5.1.142.1	Configuration for EN-DC

Table 5.1.142.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-28A-32A_n1A | DC_7A_n1A DC_28A_n1A |

#### 5.1.142.2	∆TIB and ∆RIB values

Table 5.1.142.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-28A-32A_n1A | 7 | 0.7 |
|  | 28 | 0.6 |
|  | n1 | 0.7 |

Table 5.1.142.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-28A-32A_n1A | 7 | 0 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.142.3	Reference sensitivity exceptions

### 5.1.143	DC_7-28-32_n3

#### 5.1.143.1	Configuration for EN-DC

Table 5.1.143.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-28A-32A_n3A | DC_7A_n3A DC_28A_n3A |

#### 5.1.143.2	∆TIB and ∆RIB values

Table 5.1.143.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-28A-32A_n3A | 7 | 0.7 |
|  | 28 | 0.3 |
|  | n3 | 0.7 |

Table 5.1.143.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-28A-32A_n3A | 7 | 0 |
|  | 28 | 0 |
|  | 32 | 0 |
|  | n3 | 0 |

#### 5.1.143.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.144	DC_8-20-32_n1

#### 5.1.144.1	Configuration for EN-DC

Table 5.1.144.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_8A-20A-32A_n1A | DC_8A_n1A DC_20A_n1A |

#### 5.1.144.2	∆TIB and ∆RIB values

Table 5.1.144.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-20A-32A_n1A | 8 | 0.4 |
|  | 20 | 0.4 |
|  | 32 | N/A |
|  | n1 | 0.5 |

Table 5.1.144.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8A-20A-32A_n1A | 8 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.144.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.145	DC_20-28-32_n1

#### 5.1.145.1	Configuration for EN-DC

Table 5.1.145.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-28A-32A_n1A | DC_20A_n1A DC_28A_n1A |

#### 5.1.145.2	∆TIB and ∆RIB values

Table 5.1.145.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-28A-32A_n1A | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.145.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-28A-32A_n1A | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n1 | 0 |

#### 5.1.145.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.146	DC_20-28-32_n3

#### 5.1.146.1	Configuration for EN-DC

Table 5.1.146.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-28A-32A_n3A | DC_20A_n3A DC_28A_n3A |

#### 5.1.146.2	∆TIB and ∆RIB values

Table 5.1.146.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-28A-32A_n3A | 20 | 0.5 |
|  | 28 | 0.6 |
|  | n3 | 0.5 |

Table 5.1.146.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-28A-32A_n3A | 20 | 0.3 |
|  | 28 | 0.2 |
|  | 32 | 0 |
|  | n3 | 0.3 |

#### 5.1.146.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.147	DC_20-32-38_n1

#### 5.1.147.1	Configuration for EN-DC

Table 5.1.147.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-32A-38A_n1A | DC_20A_n1A DC_38A_n1A |

#### 5.1.147.2	∆TIB and ∆RIB values

Table 5.1.147.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-32A-38A_n1A | 20 | 0.3 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.147.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-32A-38A_n1A | 20 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.147.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.148	DC_3-7-7-28_n1

### 5.1.148.1	Configurations for EN-DC

Table 5.1.148.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-7A-28A_n1A | DC_3A_n1A DC_7A_n1A DC_28A_n1A |

### 5.1.148.2	 ∆TIB and ∆RIB values

Table 5.1.148.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-7-28_n1 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.5 |
|  | n1 | 0.6 |

Table 5.1.148.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-7-28_n1 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
|  | n1 | 0 |

### 5.1.148.3		Reference sensitivity exceptions

REFSENS exceptions have already been specified for DC_3-7-28_n1.

### 5.1.149	DC_1-8-20_n28

#### 5.1.149.1	Configuration for EN-DC

Table 5.1.149.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-20A_n28A6,19 | DC_1A_n28A DC_8A_n28A DC_20A_n28A |
| NOTE 6: The frequency range in band n28 is restricted for this band combination to 703-733 MHz for the UL and 758 – 788 MHz for the DL. NOTE 19: The implementation with 3 low-band antennas is targeted for FWA form factor for this band combination in Release 17. |  |

#### 5.1.149.2	∆TIB and ∆RIB values

Table 5.1.149.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-20A_n28A | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | n28 | 0.6 |

Table 5.1.149.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-8A-20A_n28A | 1 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | n28 | 0.2 |

#### 5.1.149.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.150	DC_2-5-30_n77

### 5.1.150.1	Configuration for EN-DC

Table 5.1.150.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-5A-30A_n77A DC_2A-2A-5A-30A_n77A | DC_2A_n77A DC_5A_n77A DC_30A_n77A |

### 5.1.150.2	∆TIB and ∆RIB values

For DC_2-5-30_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-5_n77, DC_2-30_n77, and DC_5-30_n77, and are given in the tables below.

Table 5.1.150.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-5-30_n77 DC_2-2-5-30_n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.150.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-5-30_n77 DC_2-2-5-30_n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

### 5.1.150.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.151	DC_2-12-30_n77

### 5.1.151.1	Configuration for EN-DC

Table 5.1.151.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-12A-30A_n77A DC_2A-2A-12A-30A_n77A | DC_2A_n77A DC_12A_n77A DC_30A_n77A |

### 5.1.151.2	∆TIB and ∆RIB values

For DC_2-12-30_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-12_n77, DC_2-30_n77, and DC_12-30_n77, and are given in the tables below.

Table 5.1.151.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-12-30_n77 DC_2-2-12-30_n77 | 2 | 0.6 |
|  | 12 | 0.5 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.151.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-12-30_n77 DC_2-2-12-30_n77 | 2 | 0.2 |
|  | 12 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

### 5.1.151.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.152	DC_2-12-66_n77

### 5.1.152.1	Configuration for EN-DC

Table 5.1.152.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-12A-66A_n77A DC_2A-2A-12A-66A_n77A DC_2A-12A-66A-66A_n77A | DC_2A_n77A DC_12A_n77A DC_66A_n77A |

### 5.1.152.2	∆TIB and ∆RIB values

For DC_2-12-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-12_n77, DC_2-66_n77, and DC_12-66_n77, and are given in the tables below.

Table 5.1.152.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-12-66_n77 DC_2-2-12-66_n77 DC_2-12-66-66_n77 | 2 | 0.6 |
|  | 12 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.152.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-12-66_n77 DC_2-2-12-66_n77 DC_2-12-66-66_n77 | 2 | 0.2 |
|  | 12 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.152.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.153	DC_2-14-30_n77

### 5.1.153.1	Configuration for EN-DC

Table 5.1.153.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-14A-30A_n77A DC_2A-2A-14A-30A_n77A | DC_2A_n77A DC_14A_n77A DC_30A_n77A |

### 5.1.153.2	∆TIB and ∆RIB values

For DC_2-14-30_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-14_n77, DC_2-30_n77, and DC_14-30_n77, and are given in the tables below.

Table 5.1.153.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-14-30_n77 DC_2-2-14-30_n77 | 2 | 0.6 |
|  | 14 | 0.5 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.153.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-14-30_n77 DC_2-2-14-30_n77 | 2 | 0.2 |
|  | 14 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

### 5.1.153.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.154	DC_2-14-66_n77

### 5.1.154.1	Configuration for EN-DC

Table 5.1.154.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-14A-66A_n77A DC_2A-2A-14A-66A_n77A DC_2A-14A-66A-66A_n77A | DC_2A_n77A DC_14A_n77A DC_66A_n77A |

### 5.1.154.2	∆TIB and ∆RIB values

For DC_2-14-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-14_n77, DC_2-66_n77, and DC_14-66_n77, and are given in the tables below.

Table 5.1.154.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-14-66_n77 DC_2-2-14-66_n77 DC_2-14-66-66_n77 | 2 | 0.6 |
|  | 14 | 0.6 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.154.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-14-66_n77 DC_2-2-14-66_n77 DC_2-14-66-66_n77 | 2 | 0.2 |
|  | 14 | 0.2 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.154.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.155	DC_2-29-30_n77

### 5.1.155.1	Configuration for EN-DC

Table 5.1.155.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-29A-30A_n77A DC_2A-2A-29A-30A_n77A | DC_2A_n77A DC_30A_n77A |

### 5.1.155.2	∆TIB and ∆RIB values

For DC_2-29-30_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-29_n77, DC_2-30_n77, and DC_29-30_n77, and are given in the tables below.

Table 5.1.155.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-29-30_n77 DC_2-2-29-30_n77 | 2 | 0.6 |
|  | 30 | 0.3 |
|  | n77 | 0.8 |

Table 5.1.155.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-29-30_n77 DC_2-2-29-30_n77 | 2 | 0.2 |
|  | 29 | 0.2 |
|  | 30 | 0 |
|  | n77 | 0.5 |

### 5.1.155.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.156	DC_2-29-66_n77

### 5.1.156.1	Configuration for EN-DC

Table 5.1.156.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-29A-66A_n77A | DC_2A_n77A DC_66A_n77A |

### 5.1.156.2	∆TIB and ∆RIB values

For DC_2-29-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-29_n77, DC_2-66_n77, and DC_29-66_n77, and are given in the tables below.

Table 5.1.156.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-29-66_n77 | 2 | 0.6 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.156.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-29-66_n77 | 2 | 0.2 |
|  | 29 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.156.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.157	DC_2-30-66_n77

### 5.1.157.1	Configuration for EN-DC

Table 5.1.157.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-30A-66A_n77A DC_2A-2A-30A-66A_n77A DC_2A-30A-66A-66A_n77A | DC_2A_n77A DC_30A_n77A DC_66A_n77A |

### 5.1.157.2	∆TIB and ∆RIB values

For DC_2-30-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_2-30_n77, DC_2-66_n77, and DC_30-66_n77, and are given in the tables below.

Table 5.1.157.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-30-66_n77 DC_2-2-30-66_n77 DC_2-30-66-66_n77 | 2 | 0.6 |
|  | 30 | 0.3 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.157.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-30-66_n77 DC_2-2-30-66_n77 DC_2-30-66-66_n77 | 2 | 0.2 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n77 | 0.5 |

### 5.1.157.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.158	DC_5-30-66_n77

### 5.1.158.1	Configuration for EN-DC

Table 5.1.158.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_5A-30A-66A_n77A DC_5A-30A-66A-66A_n77A | DC_5A_n77A DC_30A_n77A DC_66A_n77A |

### 5.1.158.2	∆TIB and ∆RIB values

For DC_5-30-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_5-30_n77, DC_5-66_n77, and DC_30-66_n77, and are given in the tables below.

Table 5.1.158.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_5-30-66_n77 DC_5-30-66-66_n77 | 5 | 0.6 |
|  | 30 | 0.3 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.158.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_5-30-66_n77 DC_5-30-66-66_n77 | 5 | 0.2 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
|  | n77 | 0.5 |

### 5.1.158.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.159	DC_12-30-66_n77

### 5.1.159.1	Configuration for EN-DC

Table 5.1.159.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_12A-30A-66A_n77A DC_12A-30A-66A-66A_n77A | DC_12A_n77A DC_30A_n77A DC_66A_n77A |

### 5.1.159.2	∆TIB and ∆RIB values

For DC_12-30-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_12-30_n77, DC_12-66_n77, and DC_30-66_n77, and are given in the tables below.

Table 5.1.159.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_12-30-66_n77 DC_12-30-66-66_n77 | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.159.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_12-30-66_n77 DC_12-30-66-66_n77 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.159.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.160	DC_14-30-66_n77

### 5.1.160.1	Configuration for EN-DC

Table 5.1.160.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_14A-30A-66A_n77A DC_14A-30A-66A-66A_n77A | DC_14A_n77A DC_30A_n77A DC_66A_n77A |

### 5.1.160.2	∆TIB and ∆RIB values

For DC_14-30-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_14-30_n77, DC_14-66_n77, and DC_30-66_n77, and are given in the tables below.

Table 5.1.160.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_14-30-66_n77 DC_14-30-66-66_n77 | 14 | 0.6 |
|  | 30 | 0.3 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.160.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_14-30-66_n77 DC_14-30-66-66_n77 | 14 | 0.2 |
|  | 30 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.160.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.161	DC_29-30-66_n77

### 5.1.161.1	Configuration for EN-DC

Table 5.1.161.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_29A-30A-66A_n77A | DC_30A_n77A DC_66A_n77A |

### 5.1.161.2	∆TIB and ∆RIB values

For DC_29-30-66_n77, the TIB,c and RIB,c values are derived from the worst-case values from DC_29-30_n77, DC_29-66_n77, and DC_30-66_n77, and are given in the tables below.

Table 5.1.161.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_29-30-66_n77 | 30 | 0.3 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.161.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_29-30-66_n77 | 29 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.5 |
|  | n77 | 0.5 |

### 5.1.161.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.162	DC_1-3-7_n38

### 5.1.162.1	Configurations for EN-DC

Table 5.2B.4.4-1: Inter-band EN-DC configurations (four bands)

| EN-DC band configuration | UL EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-3A-7A_n38Ax,y | N/A | CA_1A-3A | N/A |
| NOTE x: The combination is not used alone as fall back mode of other band combinations. NOTE y: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.1.162.2	∆TIB and ∆RIB values

For DC_1-3-7_n38, same TIB,c and RIB,c values as LTE CA_1-3-7-38 are reused and given in the tables below

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-7_n38 | 1 | 0.6 |
|  | 3 | 0.6 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-7_n38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | n38 | 0 |

### 5.1.162.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.163	DC_1-7-20_n38

### 5.1.163.1	Configurations for EN-DC

Table 5.2B.4.4-1: Inter-band EN-DC configurations (four bands)

| EN-DC band configuration | UL EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-7A-20A_n38Ax,y | N/A | CA_1A-20A | N/A |
| NOTE x: The combination is not used alone as fall back mode of other band combinations. NOTE y: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.1.163.2	∆TIB and ∆RIB values

For DC_1-7-20_n38, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-20_n38 | 1 | 0.5 |
|  | 20 | 0.3 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-20_n38 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | n38 | 0.2 |

### 5.1.163.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.164	DC_3-7-20_n38

### 5.1.164.1	Configurations for EN-DC

Table 5.2B.4.4-1: Inter-band EN-DC configurations (four bands)

| EN-DC band configuration | UL EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_3A-7A-20A_n38Ax,y | N/A | CA_3A-20A | N/A |
| NOTE x: The combination is not used alone as fall back mode of other band combinations. NOTE y: Power imbalance between downlink carriers on Band 7 and band n38 is assumed to be within 6dB. The power spectral density imbalance condition also applies for these carriers when applicable EN-DC configuration is a subset of a higher order EN-DC configuration. |  |  |  |

### 5.1.164.2	∆TIB and ∆RIB values

For DC_3-7-20_n38, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-20_n38 | 3 | 0.5 |
|  | 20 | 0.3 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3-7-20_n38 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | n38 | 0.2 |

### 5.1.164.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.165	DC_1-3-32_n28

### 5.1.165.1	 Configurations for EN-DC

Table 5.1.165.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-32A_n28A DC_1A-3C-32A_n28A | DC_1A_n28A DC_3A_n28A DC_3C_n28A |

### 5.1.165.2	 ∆TIB and ∆RIB values

Table 5.1.165.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3-32_n28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n28 | 0.6 |

Table 5.1.165.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3-32_n28 | 1 | 0 |
|  | 3 | 0.5 |
|  | 32 | 0 |
|  | n28 | 0.5 |

### 5.1.165.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.166	DC_3-7-32_n28

### 5.1.166.1	 Configurations for EN-DC

Table 5.1.166.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-7A-32A_n28A DC_3C-7A-32A_n28A | DC_3A_n28A DC_3C_n28A DC_7A_n28A |

### 5.1.166.2	 ∆TIB and ∆RIB values

Table 5.1.166.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-7-32_n28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | n28 | 0.3 |

Table 5.1.166.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-7-32_n28 | 3 | 0.5 |
|  | 7 | 0 |
|  | 32 | 0 |
|  | n28 | 0.5 |

### 5.1.166.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.167	DC_3-20-32_n28

### 5.1.167.1	 Configurations for EN-DC

Table 5.1.167.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-20A-32A_n28A DC_3C-20A-32A_n28A | DC_3A_n28A DC_3C_n28A DC_20A_n28A |

### 5.1.167.2	 ∆TIB and ∆RIB values

Table 5.1.167.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-20-32_n28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | n28 | 0.5 |

Table 5.1.167.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-20-32_n28 | 3 | 0.5 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | n28 | 0.5 |

### 5.1.167.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.168	DC_3-28-32_n1

### 5.1.168.1	 Configurations for EN-DC

Table 5.1.168.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-28A-32A_n1A | DC_3A_n1A DC_28A_n1A |

### 5.1.168.2	 ∆TIB and ∆RIB values

Table 5.1.168.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-28-32_n1 | 3 | 0.3 |
|  | 28 | 0.6 |
|  | n1 | 0.3 |

Table 5.1.168.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-28-32_n1 | 3 | 0.5 |
|  | 28 | 0.5 |
|  | 32 | 0 |
|  | n1 | 0 |

### 5.1.168.3		Reference sensitivity exceptions

## 5.1.169	DC_1-3-38_n78

### 5.1.169.1	Configurations for EN-DC

Table 5.1.169.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_1A-3A-38A_n78A | DC_1A_n78A DC_3A_n78A |

### 5.1.169.2	∆TIB and ∆RIB values

For DC_1-3-38_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.169.2-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_1-3-38_n78 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 38 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.169.2-2: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-3-38_n78 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 38 | 0.4 |
|  | n78 | 0.5 |

### 5.1.169.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.170		DC_2-5-48_n77

### 5.1.170.1	Configuration for EN-DC

Table 5.1.170.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-5A-48A_n77A DC_2A-5A-48C_n77A DC_2A-5A-48C_n77C | DC_2A_n77A DC_5A_n77A |

### 5.1.170.2	∆TIB and ∆RIB values

Relaxation values are copied from CA_n2-n5-n77 and CA_n2-n5-n48.

Table 5.1.170.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-48_n77 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 48 | 0.8 |
|  | n77 | 0.8 |

Table 5.1.170.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-5-48_n77 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | 48 | 0.5 |
|  | n77 | 0.5 |

### 5.1.170.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.171		DC_5-48-66_n77

### 5.1.171.1	Configuration for EN-DC

Table 5.1.171.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-48A-66A_n77A DC_5A-48C-66A_n77A DC_5A-48C-66A_n77C | DC_5A_n77A DC_66A_n77A |

### 5.1.171.2	∆TIB and ∆RIB values

Relaxation values are copied from CA_n5-n66-n77 and CA_n5-n48-n77

Table 5.1.171.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-48-66_n77 | 5 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.171.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_5-48-66_n77 | 5 | 0.2 |
|  | 48 | 0.5 |
|  | 66 | 0.2 |
|  | n77 | 0.5 |

### 5.1.171.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

## 5.1.172		DC_2-13-48_n77

### 5.1.172.1	Configuration for EN-DC

Table 5.1.172.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-13A-48A_n77A | DC_13A_n77A |

### 5.1.172.2	∆TIB and ∆RIB values

Relaxation values are copied from DC_2-13_n77 and DC_2-48_n77

Table 5.1.172.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-48_n77 | 2 | 0.6 |
|  | 13 | 0.5 |
|  | 48 | 0.6 |
|  | n77 | 0.8 |

Table 5.1.172.2-1: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-13-48_n77 | 2 | 0.2 |
|  | 13 | 0.2 |
|  | 48 | 0.2 |
|  | n77 | 0.5 |

### 5.1.172.3	REFSENS requirements

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.173.1	Configuration for EN-DC

Table 5.1.173.1-1: Inter-band EN-DC configurations (four bands)

| EN-DC configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-29A_n78A DC_2A-7C-29A_n78A | DC_2A_n78A DC_7A_n78A |
| DC_2A-7A-7A-29A_n78A | DC_2A_n78A DC_7A_n78A |

### 5.1.173.2	∆TIB and ∆RIB values

The TIB,c and RIB,c values are defined as below tables.

Table 5.1.173.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-29_n78 DC_2-7-7-29_n78 | 2 | 0.6 |
|  | 7 | 0.5 |
|  | 29 | 0 |
|  | n78 | 0.8 |

Table 5.1.173.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_2-7-29_n78 DC_2-7-7-29_n78 | 2 | 0.2 |
|  | 7 | 0.5 |
|  | 29 | 0.2 |
|  | n78 | 0.5 |

### 5.1.173.3	REFSENS requirements

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.174	DC_2-7-28_n78

### 5.1.174.1	 Configurations for EN-DC

Table 5.1.174.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_2A-7A-28A_n78A DC_2A-7C-28A_n78A | DC_2A_n78A DC_7A_n78A DC_7C_n78A DC_28A_n78A |

### 5.1.174.2	 ∆TIB and ∆RIB values

Relaxation values are copied from DC_2-7-28_n7

Table 5.1.174.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-28_n78 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.174.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-28_n78 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

### 5.1.174.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.175	DC_5-7-66_n78

### 5.1.175.1	 Configurations for EN-DC

Table 5.1.175.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_5A-7A-66A_n78A DC_5A-7C-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |
| DC_5A-7A-66A-66A_n78A DC_5A-7C-66A-66A_n78A | DC_5A_n78A DC_7A_n78A DC_7C_n78A DC_66A_n78A |

### 5.1.175.2	 ∆TIB and ∆RIB values

Relaxation values are copied from DC_5-7-66_n7

Table 5.1.175.2-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_5-7-66_n78 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.175.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_5-7-66_n78 | 5 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n78 | 0.5 |

### 5.1.175.3		Reference sensitivity exceptions

REFSENS exceptions are not needed.

## 5.1.176	DC_1-7-38_n78

### 5.1.176.1	Configurations for EN-DC

Table 5.2B.4.4-1: Inter-band EN-DC configurations (four bands)

| EN-DC band configuration | UL EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_1A-7A-38A_n78A10 | DC_1A_n78A | 1A | n78A |
| NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |  |  |

### 5.1.176.2	∆TIB and ∆RIB values

For DC_1-7-38_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.176.2-1: ΔTIB,c

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-7-38_n78 | 1 | 0.3 |
|  | n78 | 0.8 |

Table 5.1.176.2-2: ΔRIB,c

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-7-38_n78 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 38 | 0 |
|  | n78 | 0.8 |

### 5.1.176.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.177	DC_7-20-38_n78

### 5.1.177.1	Configurations for EN-DC

Table 5.2B.4.4-1: Inter-band EN-DC configurations (four bands)

| EN-DC band configuration | UL EN-DC configuration | Uplink E-UTRA configuration | Uplink NR configuration |
| --- | --- | --- | --- |
| DC_7A-20A-38A_n78A10 | DC_20A_n78A | 20A | n78A |
| NOTE 10: Band 7 and Band 38 are restricted as DL Scell. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within 6dB. |  |  |  |

### 5.1.177.2	∆TIB and ∆RIB values

For DC_7-20-38_n78, the TIB,c and RIB,c values are given in the tables below.

Table 5.1.177.2-1: ΔTIB,c

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7-20-38_n78 | 20 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.177.2-2: ΔRIB,c

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7-20-38_n78 | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.4 |
|  | n78 | 0.6 |

### 5.1.177.3	Reference sensitivity exceptions

There is no additional MSD requirement for this configuration.

## 5.1.178	DC_2-5-7_n78

### 5.1.178.1	Operating bands for EN-DC

Table 5.1.178.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5-7_n78 | CA_2-5-7 | n78 |  |

### 5.1.178.2	Configuration for DC

Table 5.1.178.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-7A_n78A | DC_2A_n78A DC_5A_n78A DC_7A_n78A | CA_2A-5A-7A | n78A |

### 5.1.178.3		∆TIB and ∆RIB values

For DC_2-5-7_n78, the TIB,c and RIB,c values are given in the tables below based on values for CA_n3-n5-n7-n78.

Table 5.1.178.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-7_n78 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.178.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-7_n78 | 2 | 0.2 |
|  | 5 | 0.2 |
|  | 7 | 0.2 |
|  | n78 | 0.5 |

### 5.1.178.4		REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.179	DC_2-5-66_n78

### 5.1.179.1	Operating bands for EN-DC

Table 5.1.179.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-5-66_n78 | CA_2-5-66 | n78 |  |

### 5.1.179.2	Configuration for DC

Table 5.1.179.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-5A-66A_n78A | DC_2A_n78A DC_5A_n78A DC_66A_n78A | CA_2A-5A-66A | n78A |

### 5.1.179.3		∆TIB and ∆RIB values

For DC_2-5-66_n78, the TIB,c and RIB,c values are given in the tables below based on values for CA_n5-n25-n66-n78.

Table 5.1.179.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-5-66_n78 | 2 | 0.6 |
|  | 5 | 0.6 |
|  | 66 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.179.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-5-66_n78 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 66 | 0.3 |
|  | n78 | 0.5 |

### 5.1.179.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

## 5.1.180	DC_2-7-13_n25

### 5.1.180.1	Operating bands for EN-DC

Table 5.1.180.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-13_n25 | CA_2-7-13 | n25 |  |

### 5.1.180.2	Configuration for DC

Table 5.1.180.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A | CA_2A-7A-13A | n25A |
| DC_2A-7A-7A-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A | CA_2A-7A-7A-13A | n25A |
| DC_2A-7C-13A_n25A7,8 | DC_7A_n25A DC_13A_n25A | CA_2A-7C-13A | n25A |
| NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |

### 5.1.180.3		∆TIB and ∆RIB values

For DC_2-7-13_n25, the TIB,c and RIB,c values are given in the tables below based on values for DC_7-28_n2.

Table 5.1.180.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-13_n25 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | n25 | 0.5 |

Table 5.1.180.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-13_n25 | 2 | 0 |
|  | 7 | 0 |
|  | 13 | 0 |
|  | n25 | 0 |

### 5.1.180.4	REFSENS requirements

## MSD requirements are covered in lower order combinations.

## 5.1.181	DC_2-7-66_n25

### 5.1.181.1	Operating bands for EN-DC

Table 5.1.181.1-1: Band combinations EN-DC (four bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| 2-7-66_n25 | CA_2-7-66 | n25 |  |

### 5.1.181.2	Configuration for DC

Table 5.1.181.2-1: Inter-band EN-DC configurations (four bands)

| EN-DC Configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_2A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A | CA_2A-7A-66A | n25A |
| DC_2A-7A-7A-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A | CA_2A-7A-7A-66A | n25A |
| DC_2A-7C-66A_n25A7,8 | DC_7A_n25A DC_66A_n25A | CA_2A-7C-66A | n25A |
| NOTE 7: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for intra-band contiguous or non-contiguous EN-DC apply for the Band 42 and Band n77/n78 combination and for the Band 2 and Band n25 combinations. NOTE 8: For UEs not indicating interBandMRDC-WithOverlapDL-Bands-r16, the minimum requirements for inter-band EN-DC apply for the Band 42 and Band n77/n78 combination when the maximum power spectral density imbalance between downlink carriers contained in overlapping or partially overlapping DL bands is within 6 dB. |  |  |  |

### 5.1.181.3		∆TIB and ∆RIB values

For DC_2-7-66_n25, the TIB,c and RIB,c values are given in the tables below based on values for DC_2-7_n66.

Table 5.1.181.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-7-66_n25 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

Table 5.1.181.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-7-66_n25 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
|  | n25 | 0.5 |

### 5.1.181.4	REFSENS requirements

MSD requirements are covered in lower order combinations.

### 5.1.182	DC_1-8-20_n3

#### 5.1.182.1	Configuration for EN-DC

Table 5.1.182.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-20A_n3A | DC_1A_n3A DC_8A_n3A DC_20A_n3A |

#### 5.1.182.2	∆TIB and ∆RIB values

Table 5.1.182.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-20A_n3A | 1 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
|  | n3 | 0.3 |

Table 5.1.182.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-8A-20A_n3A | 1 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
|  | n3 | 0 |

#### 5.1.182.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.183	DC_1-8-28_n3

#### 5.1.183.1	Configuration for EN-DC

Table 5.1.183.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-28A_n3A | DC_1A_n3A DC_8A_n3A DC_28A_n3A |

#### 5.1.183.2	∆TIB and ∆RIB values

Table 5.1.183.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-28A_n3A | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
|  | n3 | 0.3 |

Table 5.1.183.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-8A-28A_n3A | 1 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
|  | n3 | 0 |

#### 5.1.183.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.184	DC_1-8-28_n78

#### 5.1.184.1	Configuration for EN-DC

Table 5.1.184.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-28A_n78A | DC_1A_n78A DC_8A_n78A DC_28A_n78A |

#### 5.1.184.2	∆TIB and ∆RIB values

Table 5.1.184.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-28A_n78A | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.184.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-8A-28A_n78A | 1 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.184.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.185	DC_1-8-32_n3

#### 5.1.185.1	Configuration for EN-DC

Table 5.1.185.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-32A_n3A | DC_1A_n3A DC_8A_n3A |

#### 5.1.185.2	∆TIB and ∆RIB values

Table 5.1.185.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-32A_n3A | 1 | 0.5 |
|  | 8 | 0.3 |
|  | n3 | 0.8 |

Table 5.1.185.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-8A-32A_n3A | 1 | 0 |
|  | 8 | 0 |
|  | 32 | 0.5 |
|  | n3 | 0.3 |

#### 5.1.185.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.186	DC_1-8-32_n78

#### 5.1.186.1	Configuration for EN-DC

Table 5.1.186.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-32A_n78A | DC_1A_n78A DC_8A_n78A |

#### 5.1.186.2	∆TIB and ∆RIB values

Table 5.1.186.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-8A-32A_n78A | 1 | 0.5 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.186.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-8A-32A_n78A | 1 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

#### 5.1.186.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.187	DC_1-20-28_n78

#### 5.1.187.1	Configuration for EN-DC

Table 5.1.187.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-28A_n78A | DC_1A_n78A DC_20A_n78A DC_28A_n78A |

#### 5.1.187.2	∆TIB and ∆RIB values

Table 5.1.187.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-28A_n78A | 1 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.187.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-20A-28A_n78A | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.187.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.188	DC_1-20-38_n8

#### 5.1.188.1	Configuration for EN-DC

Table 5.1.188.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-20A-38A_n8A | DC_1A_n8A DC_20A_n8A DC_38A_n8A |

#### 5.1.188.2	∆TIB and ∆RIB values

Table 5.1.188.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1A-20A-38A_n8A | 1 | 0.5 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |
|  | n8 | 0.6 |

Table 5.1.188.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1A-20A-38A_n8A | 1 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n8 | 0 |

#### 5.1.188.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.189	DC_3-8-28_n78

#### 5.1.189.1	Configuration for EN-DC

Table 5.1.189.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-28A_n78A | DC_3A_n78A DC_8A_n78A DC_28A_n78A |

#### 5.1.189.2	∆TIB and ∆RIB values

Table 5.1.189.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-8A-28A_n78A | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.189.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-8A-28A_n78A | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.189.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.190	DC_1-8-11_n79

### 5.1.190.1	Configurations for EN-DC

Table 5.2B.4.4-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_1A-8A-11A_n79A2 | DC_1A_n79A DC_8A_n79A DC_11A_n79A |
| NOTE 2: Applicable for UE supporting inter-band EN-DC with mandatory simultaneous Rx/Tx capability |  |

### 5.1.190.2	∆TIB and ∆RIB values

For DC_1-8-11_n79, the TIB,c and RIB,c values are given in the tables below.

Table 6.2B.4.2.3.4-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8-11_n79 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.4 |
|  | n79 | 0 |

Table 7.3B.3.3.4-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_1-8-11_n79 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0 |
|  | n79 | 0 |

### 5.1.190.3	Reference sensitivity exceptions

Co-existence study for DC_1-8-11_n79 was covered by the studies for the fallback modes of DC_1-8_n79, DC_1-11_n79 and DC_8-11_n79.

No additional MSD requirement need to be defined for this dual connectivity configuration.

### 5.1.191	DC_3-8-32_n1

#### 5.1.191.1	Configuration for EN-DC

Table 5.1.191.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-32A_n1A | DC_3A_n1A DC_8A_n1A |

#### 5.1.191.2	∆TIB and ∆RIB values

Table 5.1.191.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-8A-32A_n1A | 3 | 0.5 |
|  | 8 | 0.3 |
|  | n1 | 0.8 |

Table 5.1.191.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-8A-32A_n1A | 3 | 0 |
|  | 8 | 0 |
|  | 32 | 0.5 |
|  | n1 | 0.3 |

#### 5.1.191.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.192	DC_3-8-32_n78

#### 5.1.192.1	Configuration for EN-DC

Table 5.1.192.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-8A-32A_n78A | DC_3A_n78A DC_8A_n78A |

#### 5.1.192.2	∆TIB and ∆RIB values

Table 5.1.192.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-8A-32A_n78A | 3 | 0.8 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.192.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-8A-32A_n78A | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 32 | 0.5 |
|  | n78 | 0.5 |

#### 5.1.192.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.193	DC_3-20-28_n78

#### 5.1.193.1	Configuration for EN-DC

Table 5.1.193.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_3A-20A-28A_n78A | DC_3A_n78A DC_20A_n78A DC_28A_n78A |

#### 5.1.193.2	∆TIB and ∆RIB values

Table 5.1.193.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3A-20A-28A_n78A | 3 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.193.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_3A-20A-28A_n78A | 3 | 0.2 |
|  | 20 | 0.1 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.193.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.194	DC_7-8-32_n78

#### 5.1.194.1	Configuration for EN-DC

Table 5.1.194.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-32A_n78A | DC_7A_n78A DC_8A_n78A |

#### 5.1.194.2	∆TIB and ∆RIB values

Table 5.1.194.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-32A_n78A | 7 | 0.7 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |

Table 5.1.194.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-32A_n78A | 7 | 0 |
|  | 8 | 0.2 |
|  | 32 | 0 |
|  | n78 | 0.5 |

#### 5.1.194.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.195	DC_7-8-38_n1

#### 5.1.195.1	Configuration for EN-DC

Table 5.1.195.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-8A-38A_n1A | DC_8A_n1A |

#### 5.1.195.2	∆TIB and ∆RIB values

Table 5.1.195.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-8A-38A_n1A | 8 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.195.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-8A-38A_n1A | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0.2 |
|  | n1 | 0 |

#### 5.1.195.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.196	DC_7-20-38_n8

#### 5.1.196.1	Configuration for EN-DC

Table 5.1.196.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-20A-38A_n8A | DC_20A_n8A |

#### 5.1.196.2	∆TIB and ∆RIB values

Table 5.1.196.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-20A-38A_n8A | 20 | 0.6 |
|  | n8 | 0.6 |

Table 5.1.196.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-20A-38A_n8A | 7 | 0 |
|  | 20 | 0.2 |
|  | 38 | 0.2 |
|  | n8 | 0.2 |

#### 5.1.196.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.197	DC_7-28-38_n1

#### 5.1.197.1	Configuration for EN-DC

Table 5.1.197.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_7A-28A-38A_n1A | DC_28A_n1A |

#### 5.1.197.2	∆TIB and ∆RIB values

Table 5.1.197.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_7A-28A-38A_n1A | 28 | 0.6 |
|  | n1 | 0.5 |

Table 5.1.197.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_7A-28A-38A_n1A | 7 | 0 |
|  | 28 | 0.2 |
|  | 38 | 0.2 |
|  | n1 | 0 |

#### 5.1.197.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.198	DC_8-20-28_n78

#### 5.1.198.1	Configuration for EN-DC

Table 5.1.198.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_8A-20A-28A_n78A | DC_8A_n78A DC_20A_n78A DC_28A_n78A |

#### 5.1.198.2	∆TIB and ∆RIB values

Table 5.1.198.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-20A-28A_n78A | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
|  | n78 | 0.8 |

Table 5.1.198.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8A-20A-28A_n78A | 8 | 0.2 |
|  | 20 | 0.1 |
|  | 28 | 0.2 |
|  | n78 | 0.5 |

#### 5.1.198.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.199	DC_8-20-38_n1

#### 5.1.199.1	Configuration for EN-DC

Table 5.1.199.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_8A-20A-38A_n1A | DC_8A_n1A DC_20A_n1A DC_38A_n1A |

#### 5.1.199.2	∆TIB and ∆RIB values

Table 5.1.199.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-20A-38A_n1A | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.199.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8A-20A-38A_n1A | 8 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.199.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.200	DC_8-32-38_n1

#### 5.1.200.1	Configuration for EN-DC

Table 5.1.200.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_8A-32A-38A_n1A | DC_8A_n1A DC_38A_n1A |

#### 5.1.200.2	∆TIB and ∆RIB values

Table 5.1.200.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_8A-32A-38A_n1A | 8 | 0.3 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.200.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_8A-32A-38A_n1A | 8 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.200.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.201	DC_20-28-38_n1

#### 5.1.201.1	Configuration for EN-DC

Table 5.1.201.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_20A-28A-38A_n1A | DC_20A_n1A DC_28A_n1A DC_38A_n1A |

#### 5.1.201.2	∆TIB and ∆RIB values

Table 5.1.201.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_20A-28A-38A_n1A | 20 | 0.6 |
|  | 28 | 0.6 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.201.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_20A-28A-38A_n1A | 20 | 0.2 |
|  | 28 | 0.2 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.201.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

### 5.1.202	DC_28-32-38_n1

#### 5.1.202.1	Configuration for EN-DC

Table 5.1.202.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_28A-32A-38A_n1A | DC_28A_n1A DC_38A_n1A |

#### 5.1.202.2	∆TIB and ∆RIB values

Table 5.1.202.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_28A-32A-38A_n1A | 28 | 0.7 |
|  | 38 | 0.5 |
|  | n1 | 0.5 |

Table 5.1.202.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_28A-32A-38A_n1A | 28 | 0.2 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n1 | 0 |

#### 5.1.202.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.204	DC_n77_1-3_8

### 5.1.204.1	 Configurations for NE-DC

Table 5.1.167.1-1: Band combinations NE-DC (four bands)

| NE-DC Configuration | Uplink NE-DC configuration |
| --- | --- |
| DC_n77A_1A-3A-8A DC_n77(2A)_1A-3A-8A | DC_n77A_1A DC_n77A_3A DC_n77A_8A |

### 5.1.204.2	 ∆TIB and ∆RIB values

Similar to DC_1-3-8_n77 in section 5.1.27.3 of 37.716-31-11

## 5.1.205	DC_2-30-(n)5

### 5.1.205.1	Configuration for EN-DC

Table 5.1.205.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_2A-30A-(n)5AA DC_2A-2A-30A-(n)5AA | DC_2A_n5A DC_30A_n5A DC_(n)5AA4 |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.205.2	∆TIB and ∆RIB values

For DC_2-30-(n)5, the TIB,c and RIB,c values are derived from DC_2-30_n5 and are given in the tables below.

Table 5.1.205.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_2-30-(n)5 DC_2-2-30-(n)5 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.205.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-30-(n)5 DC_2-2-30-(n)5 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | n5 | 0 |

### 5.1.205.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

## 5.1.206	DC_30-66-(n)5

### 5.1.206.1	Configuration for EN-DC

Table 5.1.206.1-1: Band combinations EN-DC (four bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) |
| --- | --- |
| DC_30A-66A-(n)5AA | DC_30A_n5A DC_66A_n5A DC_(n)5AA4 |
| NOTE 4: Only single switched UL is supported. |  |

### 5.1.206.2	∆TIB and ∆RIB values

For DC_30-66-(n)5, the TIB,c and RIB,c values are derived from DC_30-66_n5 and are given in the tables below.

Table 5.1.206.2-1: ΔTIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔTIB,c (dB) |
| --- | --- | --- |
| DC_30-66-(n)5 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
|  | n5 | 0.3 |

Table 5.1.206.2-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band EN-DC configuration | E-UTRA or NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_30-66-(n)5 | 5 | 0.5 |
|  | 30 | 0 |
|  | 66 | 0.4 |
|  | n5 | 0.5 |

### 5.1.206.3	Reference sensitivity exceptions

There are no additional MSD requirements for this configuration. The MSD requirements are covered by the fallback configurations.

### 5.1.207	DC_2-13-66_n2

#### 5.1.207.1	Configuration for EN-DC

Table 5.1.207.1-1: Band combinations EN-DC (four bands)

| EN-DC band configuration | UL configuration(s) |
| --- | --- |
| DC_2A-13A-66A_n2A | DC_66A_n2A |
| DC_2A-13A-66A-66A_n2A | DC_66A_n2A |

#### 5.1.207.2	∆TIB and ∆RIB values

Table 5.1.207.2.-1: ΔTIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-13-66_n2 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
|  | n2 | 0.5 |

Table 5.1.207.2.-1: ΔRIB,c due to EN-DC (four bands)

| EN-DC band | E-UTRA and NR Band | ΔRIB,c (dB) |
| --- | --- | --- |
| DC_2-13-66_n2 | 2 | 0.3 |
|  | 13 | 0.0 |
|  | 66 | 0.3 |
|  | n2 | 0.3 |

#### 5.1.207.3	Reference sensitivity exceptions

No additional IMD exceptions required compared to fallbacks.

## 5.1.208	DC_3-32-38_n28

### 5.1.208.1	 Configurations for EN-DC

Table 5.1.208.1-1: Band combinations EN-DC (four bands)

| EN-DC Configuration | Uplink EN-DC configuration |
| --- | --- |
| DC_3A-32A-38A_n28A DC_3C-32A-38A_n28A | DC_3A_n28A DC_38A_n28A |

### 5.1.208.2	Co-existence studies

Based on co-existence studies of Band 38 + Band n28 and Band 3 + Band n28, own Rx impact of the 4th band is the followings.

- The 2nd order harmonics generated by Band n28 may fall into own Rx of Band 32.

- IMD5 may fall into Rx of band 32 when band 3 and n28 transmit.

### 5.1.208.3 ∆TIB and ∆RIB values

For DC_3-32-38_n28, the TIB,c and RIB,c values are reused from the band combination DC_ 7-8-32_n1, and are given in the tables below.

Table 5.1.208.3-1: ΔTIB,c due to EN-DC(four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-32-38_n28 | 3 | 0.7 |
|  | 38 | 0.7 |
|  | n28 | 0.6 |

Table 5.1.208.3-2: ΔRIB,c due to EN-DC (four bands)

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-32-38_n28 | 3 | 0 |
|  | 32 | 0 |
|  | 38 | 0 |
|  | n28 | 0.2 |

### 5.1.208.4		Reference sensitivity exceptions

No additional MSD requirement needs to be defined for this dual connectivity configuration.

## 5.1.209	DC_3-8-32_n28

### 5.1.209.1	Configurations for DC

Table 5.1.209.1-1: Inter-band DC configurations (three bands)

| DC configuration | Uplink configuration |
| --- | --- |
| DC_3A-8A-32A_n28A | DC_3A_n28A DC_8A_n28A |
| DC_3C-8A-32A_n28A | DC_3A_n28A DC_8A_n28A |

### 5.1.209.2	Co-existence studies

For UE coexistence study of Band 3 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.209.2-1.

Table 5.1.209.2-1: Harmonic and IMD analysis between Band 3 and Band n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 1710 | 1785 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 3420 | 3570 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 5130 | 5355 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 6840 | 7140 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 8550 | 8925 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 962 | 1082 | 2413 | 2533 |
| Two-tone 3rd order IMD products | ¦2*fx_high – fy_low¦ | ¦2*fx_low – fy_high¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 214 | 379 | 2672 | 2867 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 3116 | 3281 | 4123 | 4318 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 324 | 534 | 4382 | 4652 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 3819 | 4029 | 5833 | 6103 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low –2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 1924 | 2074 | 4826 | 5066 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 6092 | 6437 | 1027 | 1282 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_high -3*fx_low¦ | ¦2*fy_low - 3*fx_high¦ |
| IMD frequency limits (MHz) | 3634 | 3949 | 1176 | 1461 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 7543 | 7888 | 4522 | 4777 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6536 | 6851 | 5529 | 5814 |

As we can see from the above table:

- the 2nd harmonic may fall into DL reception frequency of Band 32.

- the 5th IMD may fall into DL reception frequency of Band 32.

For UE coexistence study of Band 8 + Band n28, the 2nd, 3rd, 4th and 5th order harmonics and 2nd, 3rd, 4th and 5th order intermodulation products were calculated and presented in Table 5.1.209.2-2.

Table 5.1.209.2-2: Harmonic and IMD analysis between Band 8 and Band n28

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 703 | 748 | 880 | 915 |
| 2nd harmonics frequency limits | 2*fx_low | 2*fx_high | 2* fy_low | 2* fy_high |
| 2nd harmonics frequency limits (MHz) | 1406 | 1496 | 1760 | 1830 |
| 3rd harmonics frequency limits | 3*fx_low | 3*fx_high | 3* fy_low | 3* fy_high |
| 3rd harmonics frequency limits (MHz) | 2109 | 2244 | 2640 | 2745 |
| 4th harmonics frequency limits | 4*fx_low | 4*fx_high | 4* fy_low | 4* fy_high |
| 4th harmonics frequency limits (MHz) | 2812 | 2992 | 3520 | 3660 |
| 5th harmonics frequency limits | 5*fx_low | 5*fx_high | 5* fy_low | 5* fy_high |
| 5th harmonics frequency limits (MHz) | 3515 | 3740 | 4400 | 4575 |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 132 | 212 | 1583 | 1663 |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 491 | 616 | 1012 | 1127 |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 2286 | 2411 | 2463 | 2578 |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 1194 | 1364 | 1892 | 2042 |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 2989 | 3159 | 3343 | 3493 |
| Two-tone 4th order IMD products | ¦2*fx_high –2* fy_low¦ | ¦2*fx_low –2* fy_high¦ | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |
| IMD frequency limits (MHz) | 264 | 334 | 3166 | 3326 |
| Two-tone 5th order IMD products | ¦fx_high – 4*fy_low¦ | ¦fx_low – 4*fy_high¦ | ¦fy_high – 4*fx_low¦ | ¦fy_low – 4*fx_high¦ |
| IMD frequency limits (MHz) | 2772 | 2957 | 1897 | 2112 |
| Two-tone 5th order IMD products | ¦2*fx_high - 3*fy_low¦ | ¦2*fx_low - 3*fy_high¦ | ¦2*fy_high -3*fx_low¦ | ¦2*fy_low - 3*fx_high¦ |
| IMD frequency limits (MHz) | 1144 | 1339 | 279 | 484 |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 4223 | 4408 | 3692 | 3907 |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 4046 | 4241 | 3869 | 4074 |

As we can see from the above table:

the 2nd harmonic caused by UL transmission on Band 8 may fall into DL reception frequency of Band 3.

the 2nd harmonic caused by UL transmission on Band n28 may fall into DL reception frequency of Band 32.

### 5.1.209.3	∆TIB and ∆RIB values

For DC_3-8-32_n28, the TIB,c and RIB,c values are given in the following table.

Table 5.1.209.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_3-8-32_n28 | 3 | 0.3 |
|  | 8 | 0.3 |
|  | n28 | 0.6 |

Table 5.1.209.3-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_3-8-32_n28 | 3 | 0 |
|  | 8 | 0 |
|  | 32 | 0 |
|  | n28 | 0.2 |

### 5.1.209.4	Reference sensitivity exceptions

For UL band 3_n28, since there MSD requirements specified for band 32 due to UL harmonic caused by band n28, and IMD exception has already been covered by DC_3-8_n28, no additional MSD requirement need to be defined for this dual connectivity configuration.

For UL band 8_n28, since there are MSD requirements specified for band 32 due to UL harmonic caused by band n28, and those specified for band 3 due to UL harmonic caused by band 8, they can be reused here. No additional MSD requirement need to be defined for this dual connectivity configuration.

# REFSENS exceptions are not needed.
Annex A - Change history

| Date | Meeting | TDoc | Subject/Comment | New version |
| --- | --- | --- | --- | --- |
| 2020-08 | 3GPP RAN4#96-e | R4-2010681 | TR skeleton | 0.0.1 |
| 2020-08 | 3GPP RAN4#96-e |  | Implemented TP’s from RAN4 #96-e: R4-2010246, “TP for TR 37.717-31-11 DC_1-3_(n)41”, Samsung, KDDI R4-2010247, “TP for TR 37.717-31-11 DC_1-3-41_n28”, Samsung, KDDI R4-2010434, “TP for 37.717-31-11 to introduce DC_3A-7A-8A_n40A”, Nokia R4-2010435, “TP for 37.717-31-11 to introduce DC_3A-7A-28A_n1A”, Nokia R4-2010437. “TP for 37.717-31-11 to introduce DC_5A-7-66A_n66A”, Nokia R4-2010514, “TP for DC_3-19-42_n1 for TR 37.717-31-11”, NTT DOCOMO INC. R4-2010515, “TP for DC_3-21-42_n1 for TR 37.717-31-11”, NTT DOCOMO INC. R4-2010516, “TP for DC_19-21-42_n1 for TR 37.717-31-11”, NTT DOCOMO INC. R4-2010896, “TP for TR 37.717-31-11: DC_2A-28A-66A_n66A”, Huawei, HiSilicon R4-2010897, “TP for TR 37.717-31-11: DC_7A-28A-66A_n66A / DC_7C-28A-66A_n66A”, Huawei, HiSilicon R4-2010898, “TP for TR 37.717-31-11: DC_2A-7A-28A_n66A / DC_2A-7C-28A_n66A”, Huawei, HiSilicon R4-2010899, “TP for TR 37.717-31-11: DC_3A-7A-28A_n1A”, Huawei, HiSilicon R4-2009996, ”TP for TR 37.717-31-11: EN-DC_1-8-11_n3”, SoftBank Corp. R4-2009997, ”TP for TR 37.717-31-11: EN-DC_1-8-42_n28”, SoftBank Corp. R4-2009770, “TP for TR 37.717-31-11: DC_1-7-32_n28”, VODAFONE Group Plc R4-2009771, “TP for TR 37.717-31-11: DC_1-7-32_n78”, VODAFONE Group Plc R4-2009772, “TP for TR 37.717-31-11: DC_1-20-32_n28”, VODAFONE Group Plc R4-2009774, “TP for TR 37.717-31-11: DC_1-20-32_n78”, VODAFONE Group Plc R4-2009775, “TP for TR 37.717-31-11: DC_3-7-32_n78”, VODAFONE Group Plc R4-2009776, “TP for TR 37.717-31-11: DC_3-20-32_n78”, VODAFONE Group Plc R4-2009777, “TP for TR 37.717-31-11: DC_7-20-32_n1”, VODAFONE Group Plc R4-2009778, “TP for TR 37.717-31-11: DC_7-20-32_n28”, VODAFONE Group Plc | 0.1.0 |
| 2020-11 | 3GPP RAN4#97-e | R4-2015925 | Implemented TP’s from RAN4 #97-e: R4-2014037, “TP for 37.717-31-11 for DC_1-20-32_n3”, Huawei,HiSilicon R4-2014038, “TP for 37.717-31-11 for DC_2-4-7_n28”, Huawei,HiSilicon R4-2014039, “TP for 37.717-31-11 for DC_2-5-7_n66”, Huawei,HiSilicon R4-2014040, “TP for 37.717-31-11 for DC_2-5-66_n7”, Huawei,HiSilicon R4-2014041, “TP for 37.717-31-11 for DC_2-5-66_n66”, Huawei,HiSilicon R4-2014042, “TP for 37.717-31-11 for DC_2-7-66_n28”, Huawei,HiSilicon R4-2014043, “TP for 37.717-31-11 for DC_3-20-32_n1”, Huawei,HiSilicon R4-2014107, “TP for TR 37.717-31-11 DC_1-3-18_n3”, Samsung, KDDI R4-2014108, “TP for TR 37.717-31-11 DC_1-3-41_n3”, Samsung, KDDI R4-2014109, “TP for TR 37.717-31-11 DC_1-3-41_n41”, Samsung, KDDI R4-2014130, “TP for TR 37.717-31-11 DC_2-5-7_n66”, Samsung, TELUS, Bell mobility R4-2014615, ”TP for TR 37.717-31-11: EN-DC_1-3-11_n28”, SoftBank Corp. R4-2014616, ”TP for TR 37.717-31-11: EN-DC_1- 3-11_n77”, SoftBank Corp. R4-2014617, ”TP for TR 37.717-31-11: EN-DC_3-8-11_n28”, SoftBank Corp. R4-2014618, ”TP for TR 37.717-31-11: EN-DC_3-8-11_n77”, SoftBank Corp. R4-2014619, ”TP for TR 37.717-31-11: EN-DC_1-8-11_n28”, SoftBank Corp. R4-2014807, “TP for TR 37.717-31-11: DC_1A-3A-18A_n28A”, KDDI Corporation R4-2014845, “TP for TR 37.717-31-11: DC_1A-3A-18A_n41A”, KDDI Corporation R4-2015231, “TP for 37.717-31-11 to introduce DC_2A-7A-28A_n7A”, Nokia R4-2015247, “TP for 37.717-31-11 to introduce DC_2A-66A-71A_n71A”, Nokia, T-Mobile R4-2015248, “TP for 37.717-31-11 to introduce DC_2-5-66_n77A”, Nokia, Verizon R4-2015249, “TP for 37.717-31-11 to introduce DC_2-13-66_n77A”, Nokia, Verizon R4-2015250, “TP for 37.717-31-11 to introduce DC_2-48-66_n77A”, Nokia, Verizon R4-2015272, “TP to TR 37.717-31-11 DC_1A-3A-40C_n78A”, Huawei, HiSilicon, Nokia, Ericsson R4-2015273, “TP to TR 37.717-31-11 DC_1A-7A-40C_n78A”, Huawei, HiSilicon, Ericsson R4-2015274, “TP to TR 37.717-31-11 DC_1A-8A-40C_n78A”, Huawei, HiSilicon, Nokia R4-2015275, “TP to TR 37.717-31-11 DC_3A-7A-40C_n78A”, Huawei, HiSilicon, Ericsson R4-2015276, “TP to TR 37.717-31-11 DC_3A-8A-40C_n78A”, Huawei, HiSilicon, Nokia R4-2015277, “TP to TR 37.717-31-11 DC_7A-8A-40C_n78A”, Huawei, HiSilicon R4-2015405, “TP for TR 37.717-31-11: DC_1A-7A-8A_n28A”, Huawei, HiSilicon R4-2015406, “TP for TR 37.717-31-11: DC_3A-7A-8A_n28A”, Huawei, HiSilicon R4-2015407, “TP for TR 37.717-31-11: DC_1A-7A-28A_n3A”, Huawei, HiSilicon R4-2015408, “TP for TR 37.717-31-11: DC_3A-8A-40A_n1A/DC_3A-8A-40C_n1A”, Huawei, HiSilicon R4-2015409, “TP for TR 37.717-31-11: DC_7A-8A-40A_n1A/DC_7A-8A-40C_n1A”, Huawei, HiSilicon R4-2015411, “TP for TR 37.717-31-11: DC_2A-28A-66A_n7A”, Huawei, HiSilicon R4-2015412, “TP for TR 37.717-31-11: DC_2A-5A-7A_n7A”, Huawei, HiSilicon R4-2015413, “TP for TR 37.717-31-11: DC_2A-7A-66A_n7A/DC_2A-7A-66A-66A_n7A”, Huawei, HiSilicon R4-2015414, “TP for TR 37.717-31-11: DC_5A-7A-66A_n7A/DC_5A-7A-66A-66A_n7A”, Huawei, HiSilicon R4-2015415, “TP for TR 37.717-31-11: DC_7A-28A-66A_n7A”, Huawei, HiSilicon R4-2015712, “TP for TR 37.717-31-11: DC_2-7-66_n77”, Huawei, HiSilicon, Bell Mobility, Telus | 0.2.0 |
| 2021-02 | 3GPP RAN4#98-e | R4-2101892 | Implemented TP’s from RAN4 #98-e: R4-2103007, “TP to TR 37.717-31-11: DC_1-20-40_n78”, Nokia, Telefonica R4-2100652, ”TP for TR 37.717-31-11: EN-DC_1-8-42_n3”, SoftBank Corp. R4-2100671, ”TP for TR 37.717-31-11: EN-DC_1-3-42_n28”, SoftBank Corp. R4-2100985, “TP for TR 37.717-31-11: DC_2-29-66_n78”, Samsung, TELUS, Bell mobility R4-2101552, “TP for TR 37.717-31-11: DC_7-8-32_n1”, VODAFONE Group Plc R4-2101553, “TP for TR 37.717-31-11: DC_7-20-32_n78”, VODAFONE Group Plc R4-2101895, “TP to TR TR 37.717-31-11 to include 3-20-40_n78”, Ericsson R4-2102020, “TP to TR 37.717-31-11 to include 2A-12A-66A_n41A, 2A-2A-12A-66A_n41A”, Ericsson R4-2102021, “TP to TR 37.717-31-11 to include 2A-66A-71A_n41A, 2A-2A-66A-71A_n41A”, Ericsson R4-2102022, “TP to TR 37.717-31-11 to include 2A-7A-12A_n66A, 2A-2A-7A-12A_n66A”, Ericsson R4-2102023, “TP to TR 37.717-31-11 to include 2A-2A-5A-7A_n66A”, Ericsson R4-2102024, “TP to TR 37.717-31-11 to include 2A-7A-71A_n66A, 2A-2A-7A-71A_n66A”, Ericsson R4-2102025, “TP to TR 37.717-31-11 to include 2A-7A-12A_n78A, 2A-2A-7A-12A_n78A”, Ericsson R4-2102026, “TP to TR 37.717-31-11 to include 2A-12A-66A_n78A, 2A-2A-12A-66A_n78A”, Ericsson R4-2102027, “TP to TR 37.717-31-11 to include 7A-12A-66A_n78A”, Ericsson R4-2102028, “TP to TR 37.717-31-11 to include 7A-66A-71A_n78A”, Ericsson R4-2102029, “TP to TR 37.717-31-11 to include 2A-7A-71A_n78A, 2A-2A-7A-71A_n78A”, Ericsson R4-2102030, “TP to TR 37.717-31-11 to include 2A-7A-66A_n2A”, Ericsson R4-2102031, “TP to TR 37.717-31-11 to include 2A-5A-7A_n2A”, Ericsson R4-2102032, “TP to TR 37.717-31-11 to include 5A-7A-66A_n2A”, Ericsson R4-2102033, “TP to TR 37.717-31-11 to include 2A-7A-71A_n2A”, Ericsson R4-2102034, “TP to TR 37.717-31-11 to include 2A-66A-71A_n2A”, Ericsson R4-2102035, “TP to TR 37.717-31-11 to include 2A-7A-12A_n2A”, Ericsson R4-2102036, “TP to TR 37.717-31-11 to include 7A-66A-71A_n2A”, Ericsson R4-2102037, “TP to TR 37.717-31-11 to include 7A-12A-66A_n2A”, Ericsson | 0.3.0 |
| 2021-04 | 3GPP RAN4#98-bis-e | R4-2106705 | Implemented TP’s from RAN4 #98-bis-e: R4-2105282, “TP to TR 37.717-31-11: DC_1A-28A-40A_n78A”, Nokia, Telefonica R4-2105283, “TP to TR 37.717-31-11: DC_3A-28A-40A_n78A”, Nokia, Telefonica R4-2105052, “TP for TR 37.717-31-11: DC_1-11-18_n3”, Samsung, KDDI R4-2105053, “TP for TR 37.717-31-11: DC_1-11-18_n28”, Samsung, KDDI R4-2105054, “TP for TR 37.717-31-11: DC_1-11-18_n41”, Samsung, KDDI R4-2105075, “TP for TR 37.717-31-11: DC_2-5-66_n48”, Samsung, Verizon R4-2105076, “TP for TR 37.717-31-11: DC_2-13-48_n77”, Samsung, Verizon R4-2105299, “TP for TR 37.717-31-11: DC_2-46-48_n2”, Samsung, Verizon R4-2105300, “TP for TR 37.717-31-11: DC_2-48-66_n2”, Samsung, Verizon R4-2105301, “TP for TR 37.717-31-11: DC_2-48-66_n66”, Samsung, Verizon R4-2105302, “TP for TR 37.717-31-11: DC_13-48-66_n77”, Samsung, Verizon R4-2105304, “TP for TR 37.717-31-11: DC_1A-3A-20A_n7A”, ZTE Corporation R4-2106643, “TP for TR 37.717-31-11: DC_1A-3A-38A_n28A/ DC_1A-3C-38A_n28A”, Huawei, HiSilicon R4-2106644, “TP for TR 37.717-31-11: DC_1A-7A-38A_n28A”, Huawei, HiSilicon R4-2106645, “TP for TR 37.717-31-11: DC_3A-7A-38A_n28A/DC_3C-7A-38A_n28A”, Huawei, HiSilicon R4-2107051, “TP for TR 37 717-31-11 to include DC_2A-5A-30A_n2A”, Ericsson, AT&T R4-2107052, “TP for TR 37 717-31-11 to include DC_2A-5A-30A_n66A”, Ericsson, AT&T R4-2107053, “TP for TR 37 717-31-11 to include DC_2A-14A-30A_n2A”, Ericsson, AT&T R4-2107054, “TP for TR 37 717-31-11 to include DC_2A-29A-30A_n66A”, Ericsson, AT&T R4-2107056, “TP for TR 37 717-31-11 to include DC_2A-46D-66A_n5A”, Ericsson, AT&T R4-2107058, “TP for TR 37 717-31-11 to include DC_5A-30A-66A_n2A”, Ericsson, AT&T R4-2107059, “TP for TR 37 717-31-11 to include DC_5A-30A-66A_n66A”, Ericsson, AT&T R4-2107060, “TP for TR 37 717-31-11 to include DC_14A-30A-66A_n66A”, Ericsson, AT&T R4-2107061, “TP for TR 37 717-31-11 to include DC_14A-30A-66A-66A_n2A”, Ericsson, AT&T R4-2107063, “TP for TR 37 717-31-11 to include DC_2A-14A-30A_n66A”, Ericsson, AT&T R4-2107064, “TP for TR 37 717-31-11 to include DC_1A-3A-7A_n3A”, Ericsson, Telstra R4-2107065, “TP for TR 37 717-31-11 to include DC_1A-3A-28A_n3A”, Ericsson, Telstra R4-2107066, “TP for TR 37 717-31-11 to include DC_3A-7A-28A_n3A”, Ericsson, Telstra R4-2107055, “TP for TR 37 717-31-11 to include DC_2A-29A-66A_n260M”, Ericsson, AT&T R4-2107057, “TP for TR 37 717-31-11 to include DC_2A-46D-66A_n260M”, Ericsson, AT&T R4-2107062, “TP for TR 37 717-31-11 to include DC_29A-30A-66A_n260M”, Ericsson, AT&T | 0.4.0 |
| 2021-05 | 3GPP RAN4#99-e | R4-2111080 | Implemented TP’s from RAN4 #99-e: R4-2110248, TP for TR 37.717-31-11: DC_3A-20A-28A_n1A, Huawei, HiSilicon R4-2110249, TP for TR 37.717-31-11: DC_7A-20A-28A_n1A, Huawei, HiSilicon | 0.5.0 |
| 2021-08 | 3GPP RAN4#100-e | R4-2113563 | Implemented TP’s from RAN4 #100-e: R4-2112456, TP for TR 37.717-31-11 DC_1-3-5_n77Samsung, SKT R4-2112457, TP for TR 37.717-31-11 DC_1-3-7_n77, Samsung, SKT R4-2112458, TP for TR 37.717-31-11 DC_1-5-7_n77, Samsung, SKT R4-2112459, TP for TR 37.717-31-11 DC_3-5-7_n77, Samsung, SKT R4-2112933, TP for TR 37.717-31-11: DC_1A-7A-38A_n3A, ZTE Corporation R4-2112934, TP for TR 37.717-31-11: DC_1A-20A-38A_n3A, ZTE Corporation R4-2112935, TP for TR 37.717-31-11: DC_7A-20A-38A_n3A, ZTE Corporation R4-2113064, TP for TR 37.717-31-11: DC_7A-29A-66A_n78A, Huawei, HiSilicon, Bell Mobility, Telus R4-2113484, TP for TR 37.717-31-11: DC_1-7-32_n3, VODAFONE Group Plc R4-2113485, TP for TR 37.717-31-11: DC_1-7-32_n8, VODAFONE Group Plc R4-2113486, TP for TR 37.717-31-11: DC_1-7-38_n8, VODAFONE Group Plc R4-2113505, TP for TR 37.717-31-11: DC_1-20-28_n3, VODAFONE Group Plc R4-2113506, TP for TR 37.717-31-11: DC_1-20-32_n8, VODAFONE Group Plc R4-2113526, TP for TR 37.717-31-11: DC_1-28-32_n3, VODAFONE Group Plc R4-2113527, TP for TR 37.717-31-11: DC_3-7-32_n1, VODAFONE Group Plc R4-2113529, TP for TR 37.717-31-11: DC_3-8-20_n1, VODAFONE Group Plc R4-2113531, TP for TR 37.717-31-11: DC_7-8-20_n1, VODAFONE Group Plc R4-2113533, TP for TR 37.717-31-11: DC_7-8-20_n3, VODAFONE Group Plc R4-2113534, TP for TR 37.717-31-11: DC_7-20-28_n3, VODAFONE Group Plc R4-2113535, TP for TR 37.717-31-11: DC_7-20-32_n1, VODAFONE Group Plc R4-2113536, TP for TR 37.717-31-11: DC_7-20-32_n3, VODAFONE Group Plc R4-2113544, TP for TR 37.717-31-11: DC_7-20-32_n8, VODAFONE Group Plc R4-2113545, TP for TR 37.717-31-11: DC_7-20-38_n1, VODAFONE Group Plc R4-2113548, TP for TR 37.717-31-11: DC_7-28-32_n1, VODAFONE Group Plc R4-2113549, TP for TR 37.717-31-11: DC_7-28-32_n3, VODAFONE Group Plc R4-2113551, TP for TR 37.717-31-11: DC_8-20-32_n1, VODAFONE Group Plc R4-2113610, TP for TR 37.717-31-11: DC_20-28-32_n1, VODAFONE Group Plc R4-2113611, TP for TR 37.717-31-11: DC_20-28-32_n3, VODAFONE Group Plc R4-2113612, TP for TR 37.717-31-11: DC_20-32-38_n1, VODAFONE Group Plc R4-2114160, TP to TR 37.717-31-11 for DC_3-7-7-28-n1, Huawei, HiSilicon | 0.6.0 |
| 2021-11 | 3GPP RAN4#101-e | R4-2118495 | Implemented TP’s from RAN4 #101-e: R4-2119793, TP for TR 37.717-31-11: DC_1-8-20_n28, VODAFONE Group Plc R4-2117276, TP for TR 37.717-31-11 Addition of DC_2-5-30_n77, AT&T R4-2117277, TP for TR 37.717-31-11 Addition of DC_2-12-30_n77, AT&T R4-2117278, TP for TR 37.717-31-11 Addition of DC_2-12-66_n77, AT&T R4-2117279, TP for TR 37.717-31-11 Addition of DC_2-14-30_n77, AT&T R4-2117280, TP for TR 37.717-31-11 Addition of DC_2-14-66_n77, AT&T R4-2117281, TP for TR 37.717-31-11 Addition of DC_2-29-30_n77, AT&T R4-2117282, TP for TR 37.717-31-11 Addition of DC_2-29-66_n77, AT&T R4-2117283, TP for TR 37.717-31-11 Addition of DC_2-30-66_n77, AT&T R4-2117284, TP for TR 37.717-31-11 Addition of DC_5-30-66_n77, AT&T R4-2117285, TP for TR 37.717-31-11 Addition of DC_12-30-66_n77, AT&T R4-2117286, TP for TR 37.717-31-11 Addition of DC_14-30-66_n77, AT&T R4-2117287, TP for TR 37.717-31-11 Addition of DC_29-30-66_n77, AT&T R4-2118198, TP for TR 37.717-31-11: DC_1A-3A-7A_n38A, ZTE Corporation R4-2118199, TP for TR 37.717-31-11: DC_1A-7A-20A_n38A, ZTE Corporation R4-2118200, TP for TR 37.717-31-11: DC_3A-7A-20A_n38A, ZTE Corporation R4-2118568, TP for TR 37.717-31-11: DC_1A-3A-32A_n28A and DC_1A-3C-32A_n28A, Huawei, HiSilicon, CKH IoD UK R4-2118569, TP for TR 37.717-31-11: DC_3A-7A-32A_n28A and DC_3C-7A-32A_n28A, Huawei, HiSilicon R4-2118570, TP for TR 37.717-31-11: DC_3A-20A-32A_n28A and DC_3C-20A-32A_n28A, Huawei, HiSilicon R4-2118571, TP for TR 37.717-31-11: DC_3A-28A-32A_n1A, Huawei, HiSilicon R4-2118572, Updated TP for TR 37.717-31-11 to correct the UL configurations for DC_7A-28A-32A_n1A and DC_20A-32A-38A_n1A, Huawei, HiSilicon, DT | 0.7.0 |
| 2022-02 | 3GPP RAN4#102-e | R4-2205685 | Implemented TP’s from RAN4 #101-bis-e: R4-2200373, TP for TR 37.717-31-11 DC_n77A_1A-3A-8A and DC_n77(2A)_1A-3A-8A, Huawei Technologies France R4-2200624, TP for TR 37.717-31-11_DC_1A-3A-38A_n78A, ZTE Corporation R4-2202172, TP to TR 37.717-31-11: DC_2-5-48_n77, Nokia, Verizon R4-2200714, TP to TR 37.717-31-11: DC_5-48-66_n77, Nokia, Verizon R4-2200715, TP to TR 37.717-31-11: DC_2-13-48_n77, Nokia, Verizon R4-2201060, TP for TR 37.717-31-11 DC_2-7-29_n78, Samsung, Telus, Bell Mobility R4-2201098, TP to TR 37.717-31-11 Addition of DC_2-7-28_n78, Nokia R4-2201099, TP to TR 37.717-31-11 Addition of DC_5-7-66_n78, Nokia R4-2201358, TP for TR 37.717-31-11_DC_1A-7A-38A_n78A, ZTE Corporation R4-2201359, TP for TR 37.717-31-11_DC_7A-20A-38A_n78A, ZTE Corporation R4-2201730, TP for TR 37.717-31-11 to include DC_2-5-7_n78, Ericsson, Rogers R4-2201731, TP for TR 37.717-31-11 to include DC_2-5-66_n78, Ericsson, Rogers Implemented TP’s from RAN4 #102-e: R4-2205703, TP for TR 37.717-31-11 to include DC_2-7-66_n25, Ericsson, Bell Mobility R4-2205704, TP for TR 37.717-31-11 to include DC_2-7-13_n25, Ericsson, Bell Mobility R4-2203632, TP for TR 37.717-31-11: DC_1-8-20_n3, VODAFONE Group Plc R4-2203633, TP for TR 37.717-31-11: DC_1-8-28_n3, VODAFONE Group Plc R4-2203634, TP for TR 37.717-31-11: DC_1-8-28_n78, VODAFONE Group Plc R4-2203635, TP for TR 37.717-31-11: DC_1-8-32_n3, VODAFONE Group Plc R4-2204098, TP for TR 37.717-31-11: DC_1-8-32_n78, VODAFONE Group Plc R4-2204101, TP for TR 37.717-31-11: DC_1-20-28_n78, VODAFONE Group Plc R4-2204107, TP for TR 37.717-31-11: DC_1-20-38_n8, VODAFONE Group Plc R4-2204110, TP for TR 37.717-31-11: DC_3-8-28_n78, VODAFONE Group Plc R4-2204112, TP for TR 37.717-31-11: EN-DC_1-8-11_n79, SoftBank Corp. R4-2205251, Updated TP for TR 37.717-31-11 DC_3C-7A-32A_n78A, Huawei, HiSilicon R4-2204113, TP for TR 37.717-31-11: DC_3-8-32_n1, VODAFONE Group Plc R4-2204116, TP for TR 37.717-31-11: DC_3-8-32_n78, VODAFONE Group Plc R4-2204119, TP for TR 37.717-31-11: DC_3-20-28_n78, VODAFONE Group Plc R4-2204122, TP for TR 37.717-31-11: DC_7-8-32_n78, VODAFONE Group Plc R4-2204125, TP for TR 37.717-31-11: DC_7-8-38_n1, VODAFONE Group Plc R4-2204126, TP for TR 37.717-31-11: DC_7-20-38_n8, VODAFONE Group Plc R4-2204127, TP for TR 37.717-31-11: DC_7-28-38_n1, VODAFONE Group Plc R4-2204128, TP for TR 37.717-31-11: DC_8-20-28_n78, VODAFONE Group Plc R4-2204129, TP for TR 37.717-31-11: DC_8-20-38_n1, VODAFONE Group Plc R4-2204139, TP for TR 37.717-31-11: DC_8-32-38_n1, VODAFONE Group Plc R4-2204141, TP for TR 37.717-31-11: DC_20-28-38_n1, VODAFONE Group Plc R4-2204142, TP for TR 37.717-31-11: DC_28-32-38_n1, VODAFONE Group Plc | 0.8.0 |
| 2022-02 | 3GPP RAN4#103-e | R4-2209552 | Implemented TP’s from RAN4 #103-e: R4-2207719, TP for TR 37.717-31-11 Addition of DC_2-30-(n)5, AT&T R4-2207720, TP for TR 37.717-31-11 Addition of DC_30-66-(n)5, AT&T R4-2207940, TP for TR 37.717-31-11: DC_2-13-66_n2, Verizon, Samsung R4-2210374, TP for TR 37.717-31-11: DC_3A-32A-38A_n28A and DC_3C-32A-38A_n28A, Huawei, HiSilicon R4-2210375, TP for TR 37.717-31-11 DC_3A-8A-32A_n28A and DC_3C-8A-32A_n28A, Huawei, HiSilicon | 0.9.0 |
| 2022-06 | 3GPP RAN #96 | RP-221324 | Presented for approval at RAN plenary. | 1.0.0 |