3GPP TR 37.871 V1.1.0 (2016-10)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

Evolved Universal Terrestrial Radio Access (E-UTRA);

Multi-Band Base Station testing with three or more bands

(Release 14)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2016, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	5

3.1	Definitions	5

3.2	Symbols	5

3.3	Abbreviations	5

4	Background	6

4.1	Task description	6

4.1.1	Objective of Core part WI	6

4.1.2	Objective of Performance part WI	6

4.2	Band combinations	6

5	Necessary changes to the core requirements	6

5.1	Symbols	6

5.2	Operating band unwanted emissions	7

5.3	Clarification of multi-band operation	7

5.4	Requirements for BS capable of multi-band operation	8

6	Necessary changes to the test requirements	8

6.1	Testing approach	8

6.2	RF bandwidth position	8

7	Project plan	9

Annex A: Change history	10

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document is a technical report of the Multi-Band Base Station testing with three or more bands work item which was approved at TSG RAN #70 [2]. The objective of this work item is to provide specification support for Multi-Band Base Station testing with three or more bands.

The report provides motivation, requirements and a list of recommended changes to the specifications.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	RP-152205, " New Work Item proposal: Multi-Band Base Station testing with three or more bands".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. 
An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ACRONYM>	<Explanation>

# 4	Background

## 4.1	Task description

### 4.1.1	Objective of Core part WI

Identify the necessary changes, if any, to the existing multi-band BS requirements for BS capable of operation in three or more bands. In addition, identify additional band combinations beside Band (8+20+28) for feasible implementation of multi-band BS capable of operation in three or more bands.

### 4.1.2	Objective of Performance part WI

To specify multi-band BS testing with three or more bands in the RAN4 specifications, using the existing multi-band BS testing with two bands as a base. The work will focus on the following steps:

1.	Conclude whether a generic testing approach can be applied to all the identified band combinations or separate testing approaches are necessary.

2.	In case a generic testing approach cannot be applied to all the identified band combinations with an initial focus on the completion of testing for Band (8+20+28).

2.	Specify multi-band BS testing for all the identified band combinations. In particular, to decide:

1.	The RF bandwidth location and carrier placement within the operating bands and frequency ranges supported by the BS.

2.	Conclude on the multi-band combinations to be tested out of the supported ones for BS that supports multiple multi-band combinations.

3.	Other aspects of multi-band BS testing that are necessary to complete the specification.

## 4.2	Band combinations

The following band combinations are considered feasible for implementation of multi-band BS capable of operation in three or more bands:

1.	Bands (8+20+28) in Europe.

2.	Bands (1+3+7).

3.	Bands (25+4+7).

Only these band combinations will be considered in this work item.

# 5	Necessary changes to the core requirements

## 5.1	Symbols

Figure 3.2-3 in TS37.104 illustrates a dual-band BS only. As the figure is only used for illustration of the Radio Bandwidth and the figure would become cumbersome as the number of supported bands continue to increase in the future with advance in technology, it is agreed to simply clarify the title of the figure as follows:

Figure 3.2-3: Illustration of Radio Bandwidth related symbols and definitions for Multi-band Multi-standard Radio (Dual-band Base Station)

## 5.2	Operating band unwanted emissions

It was discussed in [2, 6] that the existing specification text in the Operating band unwanted emissions clause (extracted below) may implicitly hint that it is a singular band. We agree that there may be ambiguity, and propose to clarify the phrase as follows:

For BS capable of multi-band operation where multiple bands are mapped on the same antenna connector, the operating band unwanted emission limits apply also in a supported operating band without any carriers transmitted, in the case where there are carriers transmitted in other supported operating band(s). In this case where there is no carrier transmitted in an operating band, the operating band unwanted emission limit, as defined in the tables of the present subclause for the largest frequency offset (fmax), of a band where there are no carriers transmitted shall apply from 10 MHz below the lowest frequency, up to 10 MHz above the highest frequency of the supported downlink operating band without any carrier transmitted. And no cumulative limits are applied in the inter-band gap between a supported downlink band with carrier(s) transmitted and a supported downlink band without any carrier transmitted.

## 5.3	Clarification of multi-band operation

Multi-band transmitter/receiver is presently defined in all BS core and test specifications as a Transmitter/Receiver “characterized by the ability to process two or more carriers in common active RF components simultaneously, where at least one carrier is configured at a different operating band (which is not a sub-band or superseding-band of another supported operating band) than the other carrier(s)”. In case of a BS with three or more bands, there could be cases where some bands use a common RF, while at least one other band is in a separate RF. In the specifications, it is presently not clear what requirements that would apply for such a band with separate RF.

For such a case where transmitted or received carriers are not processed in active RF components together with carriers in any other band, it is therefore clarified that single-band requirements shall apply. The following text would provide such an explanation in the subclauses that describe “Requirements for BS capable of multi-band operation”:

For a band supported by a Base Station where the transmitted carriers are not processed in active RF components together with carriers in any other band, single-band transmitter requirements shall apply. For a band supported by a Base Station where the received carriers are not processed in active RF components together with carriers in any other band, single-band receiver requirements shall apply.

Without the clarification, it would remain unclear what requirements that apply for bands that use active RF components separate from other bands.

## 5.4	Requirements for BS capable of multi-band operation

It was discussed at RAN4#79 that the statement (extracted from clause 4.8 of TS 37.104 [3]) below means that the listed single-band requirements apply to each antenna connector, even when multiple bands (e.g. two out of three supported bands) are mapped on one antenna connector while the other supported band is mapped on another connector. Indeed in this case exclusions or provisions for multi-band capable BS are applicable to the antenna connector that is mapped to multiple bands.

In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector.

Note that this would only be an issue for BS that supports more than two bands, otherwise only one of the two supported bands is mapped on each antenna connector.

To solve the identified issue, it is proposed to update the statement above into the following to clarify that the listed single-band requirements apply only to each antenna connector that is mapped to single-band:

In the case where multiple bands are mapped on separate antenna connectors, the following applies:

-	Single-band transmitter spurious emissions, operating band unwanted emissions, ACLR, transmitter intermodulation and receiver spurious emissions requirements apply to each antenna connector that is mapped to single-band.

# 6	Necessary changes to the test requirements

## 6.1	Testing approach

Considering the identified band combinations, the 3rd order IMD products caused in the BS by transmitting of Band 1, Band 3 and Band 7 downlink (DL) carriers have been studied and it has been shown that the 3rd order IMD products will not fall into the BS receive band of Band 1, 3 or 7.

On the other band, it has been shown that the 3rd order IMD products caused in the BS by transmitting of Band 2, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Since Band 25 is a superseding-band of Band 2, it can be deduced that the 3rd order IMD products caused in the BS by transmitting of Band 25, Band 4 and Band 7 DL carriers may fall into the BS receive band of Band 7. Hence one may argue that a special testing approach is required for MB BS supporting Bands (25+4+7), e.g. the test configuration would need to ensure that the 3rd order IMD products of the BS transmitter(s) under test will fall into its receiver of Band 7. However, this issue is not specific to this band combination and would also happen for single band (SB) BS operating in certain frequency range, e.g. in Band 2 and Band 3, and currently there is no specific test approach defined in the RAN4 specifications for such SB BS. Therefore, one may also argue that no special testing approach is required for MB BS supporting Bands (25+4+7).

In view of the above discussion, it is agreed that a generic testing approach can be applied to the three identified band combinations, while separate testing approaches may still be considered if found necessary for testing of some requirements.

## 6.2	RF bandwidth position

As seen in the IMD analyses for CA band combinations, the frequency range with problematic IMD issue depends on the band combinations. Therefore, to allow BS implementation to avoid frequency range with problematic IMD issue, it is agreed that the supported frequency range(s) together with the corresponding maximum Base Station RF Bandwidth(s) of the middle band(s) shall be declared by the BS manufacturer, and the declared maximum Base Station RF Bandwidth(s) shall be used as the RF bandwidth in the corresponding frequency range(s) of the middle band(s) in the BS testing as follows:

For BS capable of multi-band operation, unless otherwise stated, the test shall be performed at BRFBW_T’RFBW and B’RFBW_TRFBW defined as following:

BRFBW_ T’RFBW: the Base Station RF Bandwidths located at the bottom of the supported frequency range in the lower operating band and at the highest possible simultaneous frequency position, within the Maximum Radio Bandwidth, BWmax, in the upper operating band. The Base Station RF Bandwidth(s) are located at the bottom of the supported frequency range(s) in the middle band(s).

B’RFBW_TRFBW: the Base Station RF Bandwidths located at the top of the supported frequency range in the upper operating band and at the lowest possible simultaneous frequency position, within the Maximum Radio Bandwidth, BWmax, in the lower operating band. The Base Station RF Bandwidth(s) are located at the top of the supported frequency range(s) in the middle band(s).

NOTE:	BRFBW_T’RFBW = B’RFBW_TRFBW = BRFBW_TRFBW when the declared Maximum Radio Bandwidth BWmax, spans all operating bands. BRFBW_TRFBW means the Base Station RF Bandwidths are located at the bottom of the supported frequency range in the lower operating band and at the top of the supported frequency range in the upper operating band, and the Base Station RF Bandwidth(s) are located at the bottom of the supported frequency range(s) in the middle band(s) in the first test and then at the top of the supported frequency range(s) in the middle band(s) in the second test.

# 7	Project plan

To facilitate the discussion and simulation work, the work plan is proposed as follows:

- RAN4#78 (February 2016):

  - Approve skeleton TR

  - Approve project work plan

  - Identify band combinations

- RAN4#78bis (April 2016):

  - Identify the necessary changes, if any, to the existing multi-band BS requirements

  - Conclude whether a generic testing approach can be applied to all the identified band combinations

- RAN4#79 (May 2016):

  - Discuss and decide multi-band BS testing for all the identified band combinations

- RAN4#80 (August 2016):

  - Resolve any remaining open issues

  - Approve Core and Performance CRs

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2016-02 | RAN4#78 | R4-160473 |  |  | Skeleton TR | N/A | 0.0.1 |
| 2016-04 | RAN4#78bis | R4-161598 |  |  | Updated TR including the approved text proposals in RAN4#78: R4-160258, R4-161377 | 0.0.1 | 0.1.0 |
| 2016-05 | RAN4#79 | R4-164192 |  |  | Updated TR including the approved text proposals in RAN4#78bis: R4-163133 | 0.1.0 | 0.2.0 |
| 2016-05 | RAN4#79 | R4-164933 |  |  | Updated TR including the approved text proposals in RAN4#79: R4-164702, R4-164194 | 0.2.0 | 0.3.0 |
| 2016-06 | RAN#72 | RP-160778 |  |  | TR presented to RAN for information | 0.3.0 | 1.0.0 |
| 2016-10 | RAN4#80bis | R4-168081 |  |  | Updated TR including the approved text proposals in RAN4#80: R4-165179, R4-166206, R4-166948 | 1.0.0 | 1.1.0 |