| 3GPP TS 38.133 V16.10.0 (2021-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Requirements for support of radio resource management (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2021, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	42

1	Scope	44

2	References	44

3	Definitions, symbols and abbreviations	45

3.1	Definitions	45

3.2	Symbols	46

3.3	Abbreviations	47

3.4	Test tolerances	50

3.5	Frequency bands grouping	50

3.5.1	Introduction	50

3.5.2	NR operating bands in FR1	50

3.5.3	NR operating bands in FR2	51

3.6	Applicability of requirements in this specification version	51

3.6.1	RRC connected state requirements in DRX	52

3.6.2	Number of serving carriers	52

3.6.2.1	Number of serving carriers for SA	52

3.6.2.2	Number of serving carriers for EN-DC	52

3.6.2.3	Number of serving carriers for NE-DC	52

3.6.2.4	Number of serving carriers for NR-DC	53

3.6.3	Applicability for intra-band FR2	53

3.6.4	Applicability for FR2 UE power classes	53

3.6.5	Applicability for SDL bands	53

3.6.6	Applicability of requirements for NGEN-DC operation	53

3.6.7	Applicability of QCL	53

3.6.9	Applicability of requirements for scheduling availability	54

3.6.10	Applicability of requirements for measurement restrictions	54

4	SA: RRC_IDLE state mobility	54

4.1	Cell Selection	54

4.2	Cell Re-selection	54

4.2.1	Introduction	54

4.2.2	Requirements	55

4.2.2.1	UE measurement capability	55

4.2.2.2	Measurement and evaluation of serving cell	55

4.2.2.3	Measurements of intra-frequency NR cells	56

4.2.2.4	Measurements of inter-frequency NR cells	57

4.2.2.5	Measurements of inter-RAT E-UTRAN cells	59

4.2.2.6	Maximum interruption in paging reception	61

4.2.2.7	General requirements	61

4.2.2.8	Minimum requirement at transitions	61

4.2.2.9	Measurements of intra-frequency NR cells for UE configured with relaxed measurement criterion	62

4.2.2.9.1	Introduction	62

4.2.2.9.2	Measurements for UE fulfilling low mobility criterion	62

4.2.2.9.3	Measurements for UE fulfilling not-at-cell edge criterion	63

4.2.2.9.4	Measurements for UE fulfilling low mobility and not-at-cell edge criteria	64

4.2.2.10	Measurements of inter-frequency NR cells for UE configured with relaxed measurement criterion	64

4.2.2.10.1	Introduction	64

4.2.2.10.2	Measurements for UE fulfilling low mobility criterion	64

4.2.2.10.3	Measurements for UE fulfilling not-at-cell edge criterion	65

4.2.2.10.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	66

4.2.2.11	Measurements of inter-RAT E-UTRAN cells for UE configured with relaxed measurement criterion	66

4.2.2.11.1	Introduction	66

4.2.2.11.2	Measurements for UE fulfilling low mobility criterion	67

4.2.2.11.3	Measurements for UE fulfilling with not-at-cell edge criterion	68

4.2.2.11.4	Measurements for UE fulfilling low mobility and not-at-cell edge criterion	69

4.2A	Cell Re-selection when subject to CCA	69

4.2A.1	Introduction	69

4.2A.2	Requirements	70

4.2A.2.1	UE measurement capability	70

4.2A.2.2	Measurement and evaluation when subject to CCA on the serving cell	70

4.2A.2.3	Measurements of intra-frequency NR cells when subject to CCA on the serving cell and target cell	71

4.2A.2.4	Measurements of inter-frequency NR cells when subject to CCA on the target cell	72

4.2A.2.5	Measurements of inter-RAT E-UTRAN cells when subject to CCA on the serving cell	74

4.2A.2.6	Maximum interruption in paging reception when subject to CCA on the target cell	74

4.2A.2.7	General requirements	74

4.3	Minimization of Drive Tests (MDT)	75

4.3.1	Introduction	75

4.3.2	Measurement Requirements	75

4.3.3	Requirements for Relative Time Stamp Accuracy	75

4.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	76

4.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	76

4.4	Idle Mode CA/DC Measurements	76

4.4.1	Introduction	76

4.4.2	Measurement Requirements	76

4.4.2.1	Detected cell requirement during state transition and Idle mode	76

4.4.2.2	Measurements of inter-frequency CA/DC candidate cells	77

4.4.2.3	Measurements on serving cell	78

4.4.2.4	Measurements of E-UTRAN inter-RAT DC candidate cells	78

5	SA: RRC_INACTIVE state mobility	80

5.1	Cell Re-selection	80

5.1.1	Introduction	80

5.1.2	Requirements	80

5.1.2.1	UE measurement capability	80

5.1.2.2	Measurement and evaluation of serving cell	80

5.1.2.3	Measurements of intra-frequency NR cells	80

5.1.2.4	Measurements of inter-frequency NR cells	80

5.1.2.5	Measurements of inter-RAT E-UTRAN cells	80

5.1.2.6	Maximum interruption in paging reception	80

5.1.2.7	General requirements	80

5.1A	Cell Re-selection with CCA	80

5.1A.1	Introduction	80

5.1A.2	Requirements	81

5.1A.2.1	UE measurement capability	81

5.1A.2.2	Measurement and evaluation when CCA is used on the serving cell	81

5.1A.2.3	Measurements of intra-frequency NR cells when CCA is used on the serving cell and target cell	81

5.1A.2.4	Measurements of inter-frequency NR cells when CCA is used on the target cell	81

5.1A.2.5	Measurements of inter-RAT E-UTRAN cells when CCA is used on the serving cell	81

5.1A.2.6	Maximum interruption in paging reception when CCA is used on the target cell	81

5.1A.2.7	General requirements	81

5.2	Void	81

5.3	Minimization of Drive Tests (MDT)	81

5.3.1	Introduction	81

5.3.2	Measurement Requirements	82

5.3.3	Requirements for Relative Time Stamp Accuracy	82

5.3.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting	82

5.3.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting	82

5.3.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting	82

5.4	Idle Mode CA/DC Measurements	82

5.4.1	Introduction	82

5.4.2	Measurement Requirements	82

5.4.2.1	Detected cell requirement during state transition and Idle mode	83

5.4.2.2	Measurements of inter-frequency CA/DC candidate cells	83

5.4.2.3	Measurements on serving cell	83

5.4.2.4	Measurements on E-UTRAN inter-RAT DC candidate cells	83

6	RRC_CONNECTED state mobility	83

6.1	Handover	83

6.1.1	NR Handover	83

6.1.1.1	Introduction	83

6.1.1.2	NR FR1 - NR FR1 Handover	83

6.1.1.2.1	Handover delay	83

6.1.1.2.2	Interruption time	83

6.1.1.3	NR FR2- NR FR1 Handover	84

6.1.1.3.1	Handover delay	84

6.1.1.3.2	Interruption time	84

6.1.1.4	NR FR2- NR FR2 Handover	85

6.1.1.4.1	Handover delay	85

6.1.1.4.2	Interruption time	85

6.1.1.5	NR FR1- NR FR2 Handover	86

6.1.1.5.1	Handover delay	86

6.1.1.5.2	Interruption time	86

6.1.2	NR Handover to other RATs	87

6.1.2.1	NR – E-UTRAN Handover	87

6.1.2.1.1	Introduction	87

6.1.2.1.2	Handover delay	87

6.1.2.1.3	Interruption time	87

6.1.2.2	NR – UTRAN Handover	88

6.1.2.2.1	Introduction	88

6.1.2.2.2	Handover delay	88

6.1.2.2.3	Interruption time	88

6.1.3	NR DAPS Handover	89

6.1.3.1	Introduction	89

6.1.3.2	NR FR1 - NR FR1 DAPS Handover	89

6.1.3.2.1	DAPS handover delay	89

6.1.3.2.2	Interruption time	90

6.1.3.3	NR FR2- NR FR1 DAPS Handover	92

6.1.3.3.1	DAPS handover delay	92

6.1.3.3.2	Interruption time	92

6.1.3.4	NR FR1- NR FR2 DAPS Handover	93

6.1.3.4.1	DAPS handover delay	93

6.1.3.4.2	Interruption time	93

6.1.4	NR Conditional Handover	94

6.1.4.1	Introduction	94

6.1.4.2	NR FR1 – NR FR1 conditional handover	94

6.1.4.3	NR FR2 – NR FR1 conditional handover	95

6.1.4.4	NR FR2 – NR FR2 conditional handover	96

6.1.4.4.1	Handover delay	96

6.1.4.4.2	Measurement time	96

6.1.4.4.3	Preparation time	96

6.1.4.4.4	Interruption time	96

6.1.4.5	NR FR1 – NR FR2 conditional handover	97

6.1A	Void	97

6.1A.1	Void	97

6.1A.1.1	Void	97

6.1A.1.2	Void	97

6.1A.1.2.1	Void	97

6.1A.1.2.2	Void	97

6.1B	Handover to target cell using CCA	97

6.1B.1	NR Handover	97

6.1B.1.1	Introduction	97

6.1B.1.2	NR FR1 - NR FR1 Handover	98

6.1B.1.2.1	Handover delay	98

6.1B.1.2.2	Interruption time	98

6.2	RRC Connection Mobility Control	99

6.2.1	SA: RRC Re-establishment	99

6.2.1.1	Introduction	99

6.2.1.2	Requirements	99

6.2.1.2.1	UE Re-establishment delay requirement	99

6.2.1A	RRC Re-establishment with CCA	100

6.2.1A.1	Introduction	100

6.2.1A.2	Requirements	101

6.2.1A.2.1	UE Re-establishment with CCA delay requirement	101

6.2.2	Random access	102

6.2.2.1	Introduction	102

6.2.2.2	Requirements for 4-step RA type	103

6.2.2.2.1	Contention based random access	103

6.2.2.2.2	Non-Contention based random access	104

6.2.2.2.3	UE behaviour when configured with supplementary UL	105

6.2.2.3	Requirements for 2-step RA type	105

6.2.2.3.1	Contention based random access	105

6.2.2.3.2	Non-Contention based random access	106

6.2.2.3.3	UE behaviour when configured with supplementary UL	107

6.2.2A	Random access when CCA is used on target frequency	107

6.2.2A.1	Introduction	107

6.2.2A.2	Requirements for 4-step RA type	107

6.2.2A.2.1	Contention based random access	107

6.2.2A.2.2	Non-Contention based random access	108

6.2.2A.3	Requirements for 2-step RA type	109

6.2.2A.3.1	Contention based random access	110

6.2.2A.3.2	Non-Contention based random access	111

6.2.3	SA: RRC Connection Release with Redirection	111

6.2.3.1	Introduction	111

6.2.3.2	Requirements	112

6.2.3.2.1	RRC connection release with redirection to NR	112

6.2.3.2.2	RRC connection release with redirection to E-UTRAN	113

6.2.3.2.3	RRC connection release with redirection to NR carrier subject to CCA	113

7	Timing	114

7.1	UE transmit timing	114

7.1.1	Introduction	114

7.1.2	Requirements	114

7.1.2.1	Gradual timing adjustment	116

7.1.2.2	Void	116

7.2	UE timer accuracy	116

7.2.1	Introduction	116

7.2.2	Requirements	116

7.3	Timing advance	117

7.3.1	Introduction	117

7.3.2	Requirements	117

7.3.2.1	Timing Advance adjustment delay	117

7.3.2.2	Timing Advance adjustment accuracy	117

7.4	Cell phase synchronization accuracy	117

7.4.1	Definition	117

7.4.2	Minimum requirements	117

7.5	Maximum Transmission Timing Difference	117

7.5.1	Introduction	117

7.5.2	Minimum Requirements for inter-band EN-DC	118

7.5.2.1	Minimum Requirements for inter-band synchronous EN-DC	118

7.5.3	Minimum Requirements for intra-band EN-DC	118

7.5.4	Minimum Requirements for NR Carrier Aggregation	119

7.5.5	Minimum Requirements for inter-band NE-DC	119

7.5.5.1	Minimum Requirements for inter-band synchronous NE-DC	120

7.5.6	Minimum Requirements for inter-band NR DC	120

7.6	Maximum Receive Timing Difference	120

7.6.1	Introduction	120

7.6.2	Minimum Requirements for inter-band EN-DC	121

7.6.2.1	Minimum Requirements for inter-band synchronous EN-DC	121

7.6.3	Minimum Requirements for intra-band EN-DC	121

7.6.4	Minimum Requirements for NR Carrier Aggregation	122

7.6.5	Minimum Requirements for inter-band NE-DC	123

7.6.5.1	Minimum Requirements for inter-band synchronous NE-DC	123

7.6.6	Minimum Requirements for inter-band NR DC	123

7.7	deriveSSB-IndexFromCell tolerance	124

7.7.1	Minimum requirements	124

7.8	Void	124

8	Signalling characteristics	124

8.1	Radio Link Monitoring	124

8.1.1	Introduction	124

8.1.2	Requirements for SSB based radio link monitoring	125

8.1.2.1	Introduction	125

8.1.2.2	Minimum requirement	126

8.1.2.3	Measurement restrictions for SSB based RLM	128

8.1.3	Requirements for CSI-RS based radio link monitoring	128

8.1.3.1	Introduction	128

8.1.3.2	Minimum requirement	129

8.1.3.3	Measurement restrictions for CSI-RS based RLM	131

8.1.4	Minimum requirement at transitions	132

8.1.5	Minimum requirement for UE turning off the transmitter	132

8.1.6	Minimum requirement for L1 indication	132

8.1.7	Scheduling availability of UE during radio link monitoring	133

8.1.7.1	Scheduling availability of UE performing radio link monitoring with a same subcarrier spacing as PDSCH/PDCCH on FR1	133

8.1.7.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1	133

8.1.7.3	Scheduling availability of UE performing radio link monitoring on FR2	133

8.1.7.4	Scheduling availability of UE performing radio link monitoring on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	134

8.1A	Radio Link Monitoring with CCA on Target Frequency	134

8.1A.1	Introduction	134

8.1A.2	Requirements for SSB Based Radio Link Monitoring	135

8.1A.2.1	Introduction	135

8.1A.2.2	Minimum Requirement	136

8.1A.3	Minimum requirement at transitions	137

8.1A.4	Minimum requirement for UE turning off the transmitter	137

8.1A.5	Minimum requirement for L1 indication	138

8.1A.6	Scheduling availability of UE during radio link monitoring	138

8.1A.6.1	Scheduling availability of UE performing radio link monitoring with the same subcarrier spacing as PDSCH/PDCCH	138

8.1A.6.2	Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH	138

8.2	Interruption	138

8.2.1	EN-DC Interruption	138

8.2.1.1	Introduction	138

8.2.1.2	Requirements	139

8.2.1.2.1	Interruptions at transitions between active and non-active during DRX	139

8.2.1.2.2	Interruptions at transitions from non-DRX to DRX	140

8.2.1.2.3	Interruptions at SCell addition/release	140

8.2.1.2.4	Interruptions at SCell activation/deactivation	141

8.2.1.2.5	Interruptions during measurements on SCC	142

8.2.1.2.6	Interruptions at UL carrier RRC reconfiguration	143

8.2.1.2.7	Interruptions due to Active BWP switching Requirement	144

8.2.1.2.8	Interruptions at direct SCell activation and hibernation	145

8.2.1.2.9	Interruptions at SCell hibernation	145

8.2.1.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	145

8.2.1.2.11	Interruptions due to UE-specific CBW change	146

8.2.1.2.12	 Interruptions at NR SRS carrier based switching	146

8.2.1.2.13	 Interruptions at E-UTRA SRS carrier based switching	147

8.2.1.2.14	DL Interruptions at switching between two uplink carriers	148

8.2.1.2.15	Interruptions due to SCell dormancy	149

8.2.1.2.16	 Interruptions when identifying CGI of an NR cell with autonomous gaps	149

8.2.1.2.17	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	150

8.2.2	SA: Interruptions with Standalone NR Carrier Aggregation	150

8.2.2.1	Introduction	150

8.2.2.2	Requirements	151

8.2.2.2.1	Interruptions at SCell addition/release	151

8.2.2.2.2	Interruptions at SCell activation/deactivation	152

8.2.2.2.3	Interruptions during measurements on deactivated SCC	153

8.2.2.2.4	Interruptions at UL carrier RRC reconfiguration	154

8.2.2.2.5	Interruptions due to Active BWP switching Requirement	154

8.2.2.2.6	Interruptions at inter-frequency SFTD measurement	155

8.2.2.2.7	Interruptions at SCell activation/deactivation with multiple downlink SCells	156

8.2.2.2.8	Interruptions due to UE-specific CBW change	156

8.2.2.2.9	 Interruptions at NR SRS carrier based switching	156

8.2.2.2.10	DL Interruptions at UE switching between two uplink carriers	158

8.2.2.2.11	Interruptions at direct SCell activation	158

8.2.2.2.12	Interruptions due to SCell dormancy	159

8.2.2.2.13	Interruptions at transitions between active and non-active during DRX	159

8.2.2.2.14	 Interruptions when identifying CGI of an NR cell with autonomous gaps	159

8.2.2.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	160

8.2.3	NE-DC Interruptions	160

8.2.3.1	Introduction	160

8.2.3.2	Requirements	161

8.2.3.2.1	Interruptions at transitions between active and non-active during DRX	161

8.2.3.2.2	Interruptions at transitions from non-DRX to DRX	161

8.2.3.2.3	Interruptions at PSCell/SCell addition/release	161

8.2.3.2.4	Interruptions at SCell activation/deactivation	162

8.2.3.2.5	Interruptions during measurements on SCC	164

8.2.3.2.6	Interruptions at UL carrier RRC reconfiguration	165

8.2.3.2.7	Interruptions due to Active BWP switching Requirement	165

8.2.3.2.8	Interruptions at direct SCell activation and hibernation	165

8.2.3.2.9	Interruptions at SCell hibernation	166

8.2.3.2.10	Interruptions at SCell activation/deactivation with multiple downlink SCells	166

8.2.3.2.11	 Interruptions at NR SRS carrier based switching	166

8.2.3.2.12	 Interruptions at E-UTRA SRS carrier based switching	168

8.2.3.2.13	Interruptions due to SCell dormancy	168

8.2.3.2.14	Interruptions when identifying CGI of an NR cell with autonomous gaps	169

8.2.3.2.15	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	169

8.2.4	NR-DC: Interruptions	170

8.2.4.1	Introduction	170

8.2.4.2	Requirements	171

8.2.4.2.1	Interruptions at PSCell/SCell addition/release	171

8.2.4.2.2	Interruptions at SCell activation/deactivation	172

8.2.4.2.3	Interruptions during measurements on SCC	173

8.2.4.2.4	Interruptions at UL carrier RRC reconfiguration	173

8.2.4.2.5	Interruptions due to Active BWP switching Requirement	173

8.2.4.2.6	Interruptions at transitions between active and non-active during DRX	174

8.2.4.2.7	Interruptions at transitions from non-DRX to DRX	174

8.2.4.2.8	Interruptions at SCell activation/deactivation with multiple downlink SCells	174

8.2.4.2.9	Interruptions at NR SRS carrier based switching	174

8.2.4.2.10	Interruptions at direct SCell activation	176

8.2.4.2.11	 Interruptions when identifying CGI of an NR cell with autonomous gaps	176

8.2.4.2.12	 Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps	177

8.2.4.2.13	 Interruptions due to SCell dormancy	177

8.2.4.2A	Void	178

8.2.4.2A.1	Void	178

8.2.4.2A.2	Void	178

8.2.4.2A.3	Void	178

8.3	SCell Activation and Deactivation Delay	178

8.3.1	Introduction	178

8.3.2	SCell Activation Delay Requirement for Deactivated SCell	178

8.3.3	SCell Deactivation Delay Requirement for Activated SCell	182

8.3.4	Direct SCell Activation at SCell addition	183

8.3.5	Direct SCell Activation at Handover	184

8.3.7	SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells	186

8.3.8	SCell Deactivation Delay Requirement for Activated SCell with Multiple Downlink SCells	190

8.3.9	Direct SCell Activation of Multiple Downlink SCells at SCell addition	190

8.3.10	Direct SCell Activation of Multiple Downlink SCells at Handover	192

8.3A	SCell Activation and Deactivation Delay in Carriers with CCA	193

8.3A.1	Introduction	193

8.3A.2	SCell Activation Delay Requirement for Deactivated SCell	193

8.3A.3	SCell Deactivation Delay Requirement for Activated SCell	196

8.4	UE UL carrier RRC reconfiguration delay	196

8.4.1	Introduction	196

8.4.2	UE UL carrier configuration delay requirement	196

8.4.3	UE UL carrier deconfiguration delay requirement	197

8.5	Link Recovery Procedures	197

8.5.1	Introduction	197

8.5.2	Requirements for SSB based beam failure detection	198

8.5.2.1	Introduction	198

8.5.2.2	Minimum requirement	198

8.5.2.3	Measurement restriction for SSB based beam failure detection	200

8.5.3	Requirements for CSI-RS based beam failure detection	200

8.5.3.1	Introduction	200

8.5.3.2	Minimum requirement	201

8.5.3.3	Measurement restrictions for CSI-RS beam failure detection	203

8.5.4	Minimum requirement for L1 indication	204

8.5.5	Requirements for SSB based candidate beam detection	204

8.5.5.1	Introduction	204

8.5.5.2	Minimum requirement	204

8.5.5.3	Measurement restriction for SSB based candidate beam detection	206

8.5.6	Requirements for CSI-RS based candidate beam detection	207

8.5.6.1	Introduction	207

8.5.6.2	Minimum requirement	207

8.5.6.3	Measurement restriction for CSI-RS based candidate beam detection	209

8.5.7	Scheduling availability of UE during beam failure detection	210

8.5.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH on FR1	210

8.5.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1	210

8.5.7.3	Scheduling availability of UE performing beam failure detection on FR2	210

8.5.7.4	Scheduling availability of UE performing beam failure detection on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR DC	211

8.5.8	Scheduling availability of UE during candidate beam detection	211

8.5.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	211

8.5.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	211

8.5.8.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	211

8.5.8.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA and NR-DC	212

8.5.9	Requirements for Beam Failure Recovery in SCell	212

8.5.9.1	Introduction	212

8.5.9.2	Requirement	212

8.5.10	Minimum requirement at transitions for beam failure detection	213

8.5A	Link Recovery Procedures when CCA is used on target frequency	213

8.5A.1	Introduction	213

8.5A.2	Requirements for SSB based beam failure detection	214

8.5A.2.1	Introduction	214

8.5A.2.2	Minimum requirement	214

8.5A.2.3	Measurement restriction for SSB based beam failure detection	215

8.5A.4	Minimum requirement for L1 indication	215

8.5A.5	Requirements for SSB based candidate beam detection	215

8.5A.5.1	Introduction	215

8.5A.5.2	Minimum requirement	215

8.5A.5.3	Measurement restriction for SSB based candidate beam detection	216

8.5A.7	Scheduling availability of UE during beam failure detection	216

8.5A.7.1	Scheduling availability of UE performing beam failure detection with a same subcarrier spacing as PDSCH/PDCCH	216

8.5A.7.2	Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH	216

8.5A.8	Scheduling availability of UE during candidate beam detection	216

8.5A.8.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	217

8.5A.8.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	217

8.6	Active BWP switch delay	217

8.6.1	Introduction	217

8.6.2	DCI and timer based BWP switch delay on a single CC	217

8.6.2A	DCI based BWP switch delay on multiple CCs	218

8.6.2A.1	Simultaneous DCI based BWP switch delay on multiple CCs	218

8.6.2A.2	Non-simultaneous DCI based BWP switch delay on multiple CCs	220

8.6.2B	Timer based BWP switch delay on multiple CCs	220

8.6.2B.1	Simultaneous timer based BWP switch delay on multiple CCs	220

8.6.2B.2	Non-simultaneous timer based BWP switch delay on multiple CCs	221

8.6.3	RRC based BWP switch delay on a single CC	221

8.6.3A	RRC based BWP switch delay on multiple CCs	222

8.6.3A.1	Simultaneous RRC based BWP switch delay on multiple CCs	222

8.6.3A.2	Non-simultaneous RRC based BWP switch delay on multiple CCs	222

8.6.4	BWP switch delay on Consistent UL CCA recovery	223

8.7	Void	223

8.8	NE-DC: E-UTRAN PSCell Addition and Release Delay	223

8.8.1	Introduction	223

8.8.2	E-UTRAN PSCell Addition Delay Requirement	223

8.8.3	E-UTRAN PSCell Release Delay Requirement	224

8.9	NR-DC: PSCell Addition and Release Delay	224

8.9.1	Introduction	224

8.9.2	PSCell Addition Delay Requirement	224

8.9.3	PSCell Release Delay Requirement	225

8.10	Active TCI state switching delay	225

8.10.4	DCI based TCI state switch delay	227

8.10.5	RRC based TCI state switch delay	227

8.10.6	Active TCI state list update delay	227

8.10A	Active TCI state switching delay with CCA	228

8.10A.1	Introduction	228

8.10A.2	Known conditions for TCI state	228

8.10A.3	MAC-CE based TCI state switch delay	228

8.10A.4	DCI based TCI state switch delay	229

8.10A.5	RRC based TCI state switch delay	229

8.10A.6	Active TCI state list update delay	230

8.11	PSCell Change	230

8.11A	void	231

8.11B	Conditional PSCell Change	231

8.11B.1	Introduction	231

8.11B.2	Conditoinal PSCell Change delay	231

8.11B.2.1	Measurement time	231

8.12	Uplink spatial relation switch delay	232

8.12.1	Introduction	232

8.12.2	Known conditions for spatial relation when associated with DL-RS	232

8.12.3	MAC-CE based spatial relation switch delay	232

8.12.4	DCI based spatial relation switch delay	233

8.12.5	RRC based spatial relation switch delay	233

8.13	UE-specific CBW change	234

8.13.1	Introduction	234

8.13.2	UE-specific CBW change delay	234

8.14	Pathloss reference signal switching delay	234

8.14.1	Introduction	234

8.14.2	Known conditions for pathloss reference signal	234

8.14.3	MAC-CE based pathloss reference signal switch delay	235

9	Measurement Procedure	235

9.1	General measurement requirement	235

9.1.1	Introduction	235

9.1.2	Measurement gap	236

9.1.2.1	EN-DC: Measurement Gap Sharing	247

9.1.2.1a	SA: Measurement Gap Sharing	247

9.1.2.1b	NE-DC: Measurement Gap Sharing	248

9.1.2.1c	NR-DC: Measurement Gap Sharing	249

9.1.3	UE Measurement capability	250

9.1.3.1	EN-DC: Monitoring of multiple layers using gaps	250

9.1.3.1a	SA: Monitoring of multiple layers using gaps	250

9.1.3.1b	NE-DC: Monitoring of multiple layers using gaps	251

9.1.3.1c	NR-DC: Monitoring of multiple layers using gaps	251

9.1.3.2	EN-DC: Maximum allowed layers for multiple monitoring	252

9.1.3.2a	SA: Maximum allowed layers for multiple monitoring	253

9.1.3.2b	NE-DC: Maximum allowed layers for multiple monitoring	253

9.1.3.2c	NR-DC: Maximum allowed layers for multiple monitoring	254

9.1A.3.2	Void	255

9.1.3A	UE Measurement capability under operation mode with CCA	255

9.1.3A.1	EN-DC: Monitoring of multiple layers using gaps under CCA	255

9.1.3A.1A	SA: Monitoring of multiple layers using gaps under CCA	255

9.1.3A.2	EN-DC: Maximum allowed layers for multiple monitoring under CCA	255

9.1A.3.2a	Void	256

9.1.3A.2A	SA: Maximum allowed layers for multiple monitoring under CCA	256

9.1.4	Capabilities for Support of Event Triggering and Reporting Criteria	256

9.1.4.1	Introduction	256

9.1.4.2	Requirements	256

9.1.5	Carrier-specific scaling factor	260

9.1.5.1	Monitoring of multiple layers outside gaps	260

9.1.5.1.1	EN-DC mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	262

9.1.5.1.2	SA mode: carrier-specific scaling factor for SSB-based, CSI-RS based L3 measurements and RSSI and channel occupancy measurements performed outside gaps	263

9.1.5.1.3	NR-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed outside gaps	264

9.1.5.1.4	NE-DC mode: carrier-specific scaling factor for SSB-based and CSI-RS based measurements performed outside gaps	265

9.1.5.2	Monitoring of multiple layers within gaps	266

9.1.5.2.1	EN-DC mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	267

9.1.5.2.2	SA mode: carrier-specific scaling factor for SSB, CSI-RS-based L3 measurements and RSSI and channel occupancy measurements performed within gaps	269

9.1.5.2.3	NE-DC: carrier-specific scaling factor for SSB-based and CSI-RS based L3 measurements performed within gaps	270

9.1.5.2.4	NR-DC: carrier-specific scaling factor for SSB-based and CSI-RS-based L3 measurements performed within gaps	272

9.1.5.2.5	SA mode: carrier-specific scaling factor for PRS-based measurements performed within gaps	274

9.1.5.2.6	NE-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	274

9.1.5.2.7	NR-DC: carrier-specific scaling factor for PRS-based measurements performed within gaps	274

9.1.6	Minimum requirement at transitions	274

9.2	NR intra-frequency measurements	275

9.2.1	Introduction	275

9.2.2	Requirements applicability	275

9.2.3	Number of cells and number of SSB	276

9.2.3.1	Requirements for FR1	276

9.2.3.2	Requirements for FR2	276

9.2.4	Measurement Reporting Requirements	276

9.2.4.1	Periodic Reporting	276

9.2.4.2	Event-triggered Periodic Reporting	276

9.2.4.3	Event Triggered Reporting	277

9.2.5	Intrafrequency measurements without measurement gaps	277

9.2.5.1	Intrafrequency cell identification	277

9.2.5.2	Measurement period	280

9.2.5.3	Scheduling availability of UE during intra-frequency measurements	281

9.2.5.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	282

9.2.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	282

9.2.5.3.3	Scheduling availability of UE performing measurements on FR2	282

9.2.5.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	283

9.2.5.4	SFTD Measurements between PCell and PSCell	283

9.2.5.4.1	Introduction	283

9.2.5.4.2	SFTD Measurement delay	284

9.2.5.4.3	SFTD Measurement Reporting Delay	284

9.2.6	Intra-frequency measurements with measurement gaps	284

9.2.6.1	Void	284

9.2.6.2	Intra-frequency cell identification	284

9.2.6.3	Intrafrequency Measurement Period	286

9.2A	NR intra-frequency measurements with CCA	287

9.2A.1	Introduction	287

9.2A.2	Requirements applicability	288

9.2A.3	Number of cells and number of SSB	288

9.2A.4	Measurement Reporting Requirements	288

9.2A.5	Intra-frequency measurements without measurement gaps	289

9.2A.5.2	Measurement period	291

9.2A.5.3	Scheduling availability of UE during intra-frequency measurements	292

9.2A.5.3.1	Scheduling availability of UE performing measurements in TDD bands	292

9.2A.5.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH	293

9.2A.6	Intra-frequency measurements with measurement gaps	293

9.2A.6.1	Intra-frequency cell identification	293

9.2A.6.2	Intra-frequency Measurement Period	294

9.2A.7	Intra-frequency RSSI and Channel occupancy measurements	295

9.2A.7.1	Intra-frequency RSSI measurements	295

9.2A.7.2	Intra-frequency Channel occupancy measurements	296

9.2A.7.3	Scheduling restriction during RSSI and Channel Occupancy measurements	297

9.3	NR inter-frequency measurements	297

9.3.1	Introduction	297

9.3.2	Requirements applicability	298

9.3.2.1	Void	298

9.3.2.2	Void	298

9.3.3	Number of cells and number of SSB	298

9.3.3.1	Requirements for FR1	298

9.3.3.2	Requirements for FR2	298

9.3.4	Inter-frequency measurement with measurement gaps	298

9.3.4.1	Void	300

9.3.4.2	Void	300

9.3.5	Inter-frequency measurements	300

9.3.5.1	Void	300

9.3.5.2	Void	300

9.3.5.3	Void	300

9.3.6	Inter-frequency measurements reporting requirements	300

9.3.6.1	Periodic Reporting	300

9.3.6.2	Event-triggered Periodic Reporting	301

9.3.6.3	Event-triggered Reporting	301

9.3.7	Void	301

9.3.8	Inter-frequency SFTD measurement requirements	301

9.3.8.1	Introduction	301

9.3.8.2	SFTD Measurement delay	301

9.3.8.3	SFTD Measurement reporting delay	302

9.3.9	Inter frequency measurements without measurement gaps	303

9.3.9.1	Inter frequency Cell identification	303

9.3.9.2	Measurement period	304

9.3.9.3	Scheduling availability of UE during inter-frequency measurements	305

9.3.9.3.1	Scheduling availability of UE performing measurements in TDD bands on FR1	305

9.3.9.3.2	Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1	306

9.3.9.3.3	Scheduling availability of UE performing measurements on FR2	306

9.3.9.3.4	Scheduling availability of UE performing measurements on FR1 or FR2 in case of FR1-FR2 inter-band CA	306

9.3A	NR inter-frequency measurements in carrier frequencies with CCA	307

9.3A.1	Introduction	307

9.3A.2	Requirements applicability	307

9.3A.3	Number of cells and number of SSB	307

9.3A.3.1	Requirements	307

9.3A.4	Inter-frequency cell identification	308

9.3A.5	Inter-frequency measurements	309

9.3A.6	NR Inter-frequency measurements reporting requirements	310

9.3A.6.1	Periodic Reporting	310

9.3A.6.2	Event-triggered Periodic Reporting	310

9.3A.6.3	Event-triggered Reporting	310

9.3A.8	Inter-frequency RSSI measurements	310

9.3A.9	Inter-frequency channel occupancy measurements	311

9.4	Inter-RAT measurements	311

9.4.1	Introduction	311

9.4.2	NR − E-UTRAN FDD measurements	313

9.4.2.1	Introduction	313

9.4.2.2	Requirements when no DRX is used	313

9.4.2.3	Requirements when DRX is used	314

9.4.2.4	Measurement reporting requirements	315

9.4.2.4.1	Periodic Reporting	315

9.4.2.4.2	Event-Triggered Periodic Reporting	315

9.4.2.4.3	Event-Triggered Reporting	315

9.4.3	NR − E-UTRAN TDD measurements	315

9.4.3.1	Introduction	315

9.4.3.2	Requirements when no DRX is used	315

9.4.3.3	Requirements when DRX is used	316

9.4.3.4	Measurement reporting requirements	318

9.4.3.4.1	Periodic Reporting	318

9.4.3.4.2	Event-Triggered Periodic Reporting	318

9.4.3.4.3	Event-Triggered Reporting	318

9.4.4	Inter-RAT RSTD measurements	318

9.4.4.1	NR − E-UTRAN FDD RSTD measurements	318

9.4.4.1.1	Introduction	318

9.4.4.1.2	Requirements	319

9.4.4.2	NR − E-UTRAN TDD RSTD measurements	322

9.4.4.2.1	Introduction	322

9.4.4.2.2	Requirements	323

9.4.5	Inter-RAT E-CID measurements	326

9.4.5.1	NR−E-UTRAN FDD E-CID RSRP and RSRQ measurements	326

9.4.5.1.1	Introduction	326

9.4.5.1.2	Requirements	326

9.4.5.1.3	Measurement Reporting Delay	326

9.4.5.2	NR−E-UTRAN TDD E-CID RSRP and RSRQ measurements	326

9.4.5.2.1	Introduction	326

9.4.5.2.2	Requirements	326

9.4.5.2.3	Measurement Reporting Delay	327

9.4.6	NR − UTRAN FDD measurements	327

9.4.6.1	Introduction	327

9.4.6.2	Requirements when no DRX is used	327

9.4.6.3	Requirements when DRX is used	328

9.4.7	NR – E-UTRAN measurements with autonomous gaps	330

9.4.7.1	CGI identification of an E-UTRA cell with autonomous gaps	330

9.4.7.2	CGI reporting delay	330

9.5	L1-RSRP measurements for Reporting	331

9.5.1	Introduction	331

9.5.2	Requirements applicability	331

9.5.3	Measurement Reporting Requirements	331

9.5.3.1	Periodic Reporting	332

9.5.3.2	Semi-Persistent Reporting	332

9.5.3.3	Aperiodic Reporting	332

9.5.4	L1-RSRP measurement requirements	332

9.5.4.1	SSB based L1-RSRP Reporting	332

9.5.4.2	CSI-RS based L1-RSRP Reporting	334

9.5.4A	Void	337

9.5.4A.1	Void	337

9.5.5	Measurement restriction for CSI-RS and SSB for L1-RSRP measurement	337

9.5.5.1	Measurement restriction for SSB based L1-RSRP	337

9.5.5.2	Measurement restriction for CSI-RS based L1-RSRP	337

9.5.6	Scheduling availability of UE during L1-RSRP measurement	338

9.5.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	338

9.5.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	338

9.5.6.3	Scheduling availability of UE performing L1-RSRP measurement on FR2	339

9.5.6.4	Scheduling availability of UE performing L1-RSRP measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	339

9.5A	L1-RSRP measurements for Reporting under CCA	340

9.5A.1	Introduction	340

9.5A.2	Requirements applicability	340

9.5A.3	Measurement Reporting Requirements	340

9.5A.3.1	Periodic Reporting	340

9.5A.3.2	Semi-Persistent Reporting	341

9.5A.3.3	Aperiodic Reporting	341

9.5A.4	L1-RSRP measurement requirements	341

9.5A.4.1	SSB based L1-RSRP Reporting	341

9.5A.5	Measurement restriction for L1-RSRP measurement	342

9.5A.5.1	Measurement restriction for SSB based L1-RSRP	342

9.5A.6	Scheduling availability of UE during L1-RSRP measurement	342

9.5A.6.1	Scheduling availability of UE performing L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH	342

9.5A.6.2	Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH	343

9.5A.6.3	Scheduling availability of UE performing L1-RSRP measurement in case of FR1-FR2 inter-band CA	343

9.6	NE-DC: Measurements	343

9.6.1	Introduction	343

9.6.2	SFTD Measurements	343

9.6.2.1	Introduction	343

9.6.2.2	SFTD Measurement requirements	343

9.7	Cross Link Interference measurements	344

9.7.1	Introduction	344

9.7.2	SRS-RSRP measurements	344

9.7.2.1	Introduction	344

9.7.2.2	Requirements applicability	344

9.7.2.3	Measurement Reporting Requirements	345

9.7.2.3.1	Periodic Reporting	345

9.7.2.3.2	Event-triggered Periodic Reporting	345

9.7.2.3.3	Event Triggered Reporting	345

9.7.2.4	Measurement capability	345

9.7.2.5	SRS-RSRP measurement period	345

9.7.3	CLI-RSSI measurements	346

9.7.3.1	Introduction	346

9.7.3.2	Requirements applicability	346

9.7.3.3	Measurement Reporting Requirements	346

9.7.3.3.1	Periodic Reporting	346

9.7.3.3.2	Event-triggered Periodic Reporting	346

9.7.3.3.3	Event Triggered Reporting	346

9.7.3.4	Measurement capability	347

9.7.3.5	CLI-RSSI measurement period	347

9.7.4	Scheduling availability of UE during CLI measurements	347

9.7.4.1	Scheduling availability of UE performing measurement on FR1	347

9.7.4.2	Scheduling availability of UE performing measurement on FR2	347

9.8	L1-SINR measurements for Reporting	348

9.8.1	Introduction	348

9.8.2	Requirements applicability	348

9.8.3	Measurement Reporting Requirements	349

9.8.3.1	Periodic Reporting	349

9.8.3.2	Semi-Persistent Reporting	349

9.8.4	L1-SINR measurement requirements	350

9.8.4.1	L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	350

9.8.4.3	L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	353

9.8.5	Measurement restriction for L1-SINR measurement	355

9.8.5.1	Measurement restriction if SSB configured for L1-SINR Measurement	355

9.8.5.2	Measurement restriction if CSI-RS configured for L1-SINR measurement	356

9.8.5.3	Measurement restriction if CSI-IM configured for L1-SINR measurement	356

9.8.6	Scheduling availability of UE during L1-SINR measurement	357

9.8.6.1	Scheduling availability of UE performing L1-SINR measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1	357

9.8.6.2	Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1	357

9.8.6.4	Scheduling availability of UE performing L1-SINR measurement on FR1 or FR2 in case of FR1-FR2 inter-band CA	358

9.9	 NR measurements for positioning	358

9.9.1	Introduction	358

9.9.2	RSTD measurements	359

9.9.2.1	Introduction	359

9.9.2.2	Requirements Applicability	359

9.9.2.3	Measurement Capability	359

9.9.2.4	Measurement Reporting Requirements	359

9.9.2.4.1	Void	359

9.9.2.4.2	Void	359

9.9.2.4.3	Void	359

9.9.2.5	Measurements Period Requirements	359

9.9.2.6	Void	361

9.9.3	PRS-RSRP measurements	361

9.9.3.1	Introduction	361

9.9.3.2	Requirements applicability	361

9.9.3.3	Measurement Capability	361

9.9.3.4	Measurement Reporting Requirements	361

9.9.3.5	Measurement Period Requirements	362

9.9.4	UE Rx-Tx time difference measurements	364

9.9.4.1 Introduction	364

9.9.4.2 Requirements Applicability	364

9.9.4.3 Measurement Capability	364

9.9.4.4 Measurement Reporting Requirements	364

9.9.4.5 Measurement Period Requirements	364

9.9.5	NR E-CID measurements	366

9.9.5.1	Introduction	366

9.9.5.2	Measurement Requirements	366

9.9.5.2.1	Intra-frequency Measurement Requirements	366

9.9.5.2.2	Inter-frequency Measurement Requirements	367

9.9.5.2.3	Measurement Reporting Delay	367

9.10	CSI-RS based L3 measurements	367

9.10.1	Introduction	367

9.10.2	CSI-RS based intra-frequency measurements	368

9.10.2.1	Introduction	368

9.10.2.2	Requirements applicability	368

9.10.2.3	Number of cells and number of CSI-RS	369

9.10.2.3.1	Requirements for FR1	369

9.10.2.3.2	Requirements for FR2	369

9.10.2.4	Measurement Reporting Requirements	370

9.10.2.4.1	Periodic Reporting	370

9.10.2.4.2	Event-triggered Periodic Reporting	370

9.10.2.4.3	Event Triggered Reporting	370

9.10.2.5	Intra-frequency measurements without measurement gaps	370

9.10.2.6	Scheduling availability of UE during CSI-RS based intra-frequency measurements	371

9.10.2.6.1	Scheduling availability of UE performing CSI-RS based measurements in TDD bands	372

9.10.2.6.2	Scheduling availability of UE performing CSI-RS based measurements in FR2	372

9.10.3	CSI-RS based Inter-frequency measurements	372

9.10.3.1	Introduction	372

9.10.3.2	Requirements applicability	372

9.10.3.3	Number of cells and number of CSI-RS resources	373

9.10.3.3.1	Requirements for FR1	373

9.10.3.3.2	Requirements for FR2	373

9.10.3.4	Measurements reporting requirements	373

9.10.3.4.1	Periodic Reporting	373

9.10.3.4.2	Event-triggered Periodic Reporting	374

9.10.3.4.3	Event-triggered Reporting	374

9.10.3.5	Inter frequency measurements with measurement gaps	374

9.11	NR measurements with autonomous gaps	375

9.11.1	Introduction	375

9.11.2	CGI identification of an NR cell with autonomous gaps	375

9.11.3	CGI reporting delay	376

10	Measurement Performance requirements	377

10.1	NR measurements	377

10.1.1	Introduction	377

10.1.2	Intra-frequency RSRP accuracy requirements for FR1	377

10.1.2.1	Intra-frequency SS-RSRP accuracy requirements	377

10.1.2.1.1	Absolute SS-RSRP Accuracy	377

10.1.2.1.2	Relative SS-RSRP Accuracy	378

10.1.2.2	Void	379

10.1.2.3	Intra-frequency CSI-RSRP accuracy requirements	379

10.1.2.3.1	Absolute CSI-RSRP Accuracy	379

10.1.2.3.2	Relative CSI-RSRP Accuracy	380

10.1.2B	Intra-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	381

10.1.2B.1	Intra-frequency SS-RSRP accuracy requirements	381

10.1.2B.1.1	Absolute SS-RSRP Accuracy	381

10.1.3	Intra-frequency RSRP accuracy requirements for FR2	382

10.1.3.1	Intra-frequency SS-RSRP accuracy requirements	382

10.1.3.1.1	Absolute SS-RSRP Accuracy	382

10.1.3.1.2	Relative SS-RSRP Accuracy	383

10.1.3.2	Void	384

10.1.3.3	Intra-frequency CSI-RSRP accuracy requirements	384

10.1.3.3.1	Absolute CSI-RSRP Accuracy	384

10.1.3.3.2	Relative CSI-RSRP Accuracy	384

10.1.3B	Intra-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	385

10.1.3B.1	Intra-frequency SS-RSRP accuracy requirements	385

10.1.3B.1.1	Absolute SS-RSRP Accuracy	386

10.1.4	Inter-frequency RSRP accuracy requirements for FR1	386

10.1.4.1	Inter-frequency SS-RSRP accuracy requirements	386

10.1.4.1.1	Absolute Accuracy of SS-RSRP in FR1	386

10.1.4.1.2	Relative Accuracy of SS-RSRP in FR1	387

10.1.4.2	Void	388

10.1.4.3	Inter-frequency CSI-RSRP accuracy requirements	388

10.1.4.3.1	Absolute Accuracy of CSI-RSRP in FR1	388

10.1.4.3.2	Relative Accuracy of CS-RSRP in FR1	389

10.1.4B	Inter-frequency RSRP accuracy requirements for FR1 for CA/DC Idle Mode Measurements	390

10.1.4B.1	Inter-frequency SS-RSRP accuracy requirements	390

10.1.4B.1.1	Absolute Accuracy of SS-RSRP in FR1	390

10.1.5	Inter-frequency RSRP accuracy requirements for FR2	391

10.1.5.1	Inter-frequency SS-RSRP accuracy requirements	391

10.1.5.1.1	Absolute SS-RSRP Accuracy	391

10.1.5.1.2	Relative SS-RSRP Accuracy	392

10.1.5.2	Void	393

10.1.5.3	Inter-frequency CSI-RSRP accuracy requirements	393

10.1.5.3.1	Absolute CSI-RSRP Accuracy	393

10.1.5.3.2	Relative CSI-RSRP Accuracy	393

10.1.5B	Inter-frequency RSRP accuracy requirements for FR2 for CA/DC Idle Mode Measurements	394

10.1.6 RSRP Measurement Report Mapping	395

10.1.7	Intra-frequency RSRQ accuracy requirements for FR1	397

10.1.7.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	397

10.1.7.1.1	Absolute SS-RSRQ Accuracy in FR1	397

10.1.7.2	Intra-frequency CSI-RSRQ accuracy requirements	398

10.1.7.2.1	Absolute CSI-RSRQ Accuracy	398

10.1.7B	Intra-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	399

10.1.7B.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	399

10.1.7B.1.1	Absolute SS-RSRQ Accuracy in FR1	399

10.1.8	Intra-frequency RSRQ accuracy requirements for FR2	400

10.1.8.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	400

10.1.8.1.1	Absolute SS-RSRQ Accuracy in FR2	400

10.1.8.2	Intra-frequency CSI-RSRQ accuracy requirements	401

10.1.8.2.1	Absolute CSI-RSRQ Accuracy	401

10.1.8B	Intra-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	402

10.1.8B.1	Intra-frequency SS-RSRQ accuracy requirements in FR2	402

10.1.8B.1.1	Absolute SS-RSRQ Accuracy in FR2	402

10.1.9	Inter-frequency RSRQ accuracy requirements for FR1	403

10.1.9.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	403

10.1.9.1.1	Absolute Accuracy of SS-RSRQ in FR1	403

10.1.9.1.2	Relative Accuracy of SS-RSRQ in FR1	404

10.1.9.2	Inter-frequency CSI-RSRQ accuracy requirements	405

10.1.9.2.1	Absolute CSI-RSRQ Accuracy	405

10.1.9.2.2	Relative CSI-RSRQ Accuracy	406

10.1.9B	Inter-frequency RSRQ accuracy requirements for FR1 for CA/DC Idle Mode Measurements	407

10.1.9B.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	407

10.1.9B.1.1	Absolute Accuracy of SS-RSRQ in FR1	407

10.1.10	Inter-frequency RSRQ accuracy requirements for FR2	408

10.1.10.2	Inter-frequency CSI-RSRQ accuracy requirements	410

10.1.10.2.1	Absolute CSI-RSRQ Accuracy	410

10.1.10.2.2	Relative CSI-RSRQ Accuracy	410

10.1.10B	Inter-frequency RSRQ accuracy requirements for FR2 for CA/DC Idle Mode Measurements	411

10.1.10B.1	Inter-frequency SS-RSRQ accuracy requirements in FR2	411

10.1.10B.1.1	Absolute Accuracy of SS-RSRQ in FR2	411

10.1.11	RSRQ report mapping	412

10.1.12	Intra-frequency SINR accuracy requirements for FR1	413

10.1.12.2	Intra-frequency CSI-SINR accuracy requirements in FR1	413

10.1.12.2.1	Absolute CSI-SINR Accuracy in FR1	413

10.1.13	Intra-frequency SINR accuracy requirements for FR2	414

10.1.13.2	Intra-frequency CSI-SINR accuracy requirements in FR2	415

10.1.13.2.1	Absolute CSI-SINR Accuracy in FR2	415

10.1.14	Inter-frequency SINR accuracy requirements for FR1	416

10.1.14.2	Inter-frequency CSI-SINR accuracy requirements in FR1	418

10.1.14.2.1	Aboslute Accuracy of CSI-SINR in FR1	418

10.1.15	Inter-frequency SINR accuracy requirements for FR2	420

10.1.15.2	Inter-frequency CSI-SINR accuracy requirements in FR2	422

10.1.15.2.1	Aboslute Accuracy of CSI-SINR in FR2	422

10.1.15.2.2	Relative Accuracy of CSI-SINR in FR2	422

10.1.16	SINR report mapping	423

10.1.17	Power Headroom	424

10.1.18	PCMAX,c,f	425

10.1.19	L1-RSRP accuracy requirements for FR1	425

10.1.20	L1-RSRP accuracy requirements for FR2	429

10.1.21	SFTD accuracy requirements	432

10.1.22	CLI measurement accuracy requirements	436

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	440

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	440

10.1.23	RSTD Measurements	442

10.1.23.1	Introduction	442

10.1.23.2	Measurement Accuracy Requirements	443

10.1.23.3	Report mapping	446

10.1.23.3.1	 Absolute DL RSTD Measurement Reporting	446

10.1.23.3.2	 Differential Reporting for DL RSTD Measurement	448

10.1.23.3.3	Additional Path Report Mapping for DL RSTD	450

10.1.24	PRS-RSRP Measurements	452

10.1.24.1	Introduction	452

10.1.24.2	Measurement Accuracy Requirements	452

10.1.24.2.1 Absolute PRS RSRP accuracy	452

10.1.24.3	Report mapping	456

10.1.24.3.1	Absolute PRS-RSRP Measurement Report Mapping	456

10.1.24.3.2	Differential Report Mapping for PRS-RSRP Measurement	457

10.1.25	UE Rx-Tx Time Difference Measurements	459

10.1.25.1	Introduction	459

10.1.25.2	Measurement Accuracy Requirements	460

10.1.25.3	Report mapping	464

10.1.25.3.1	 Absolute UE Rx-Tx Measurement Report Mapping	464

10.1.25.3.2	 Differential UE Rx-Tx Measurement Report Mapping	466

10.1.25.3.3	 Additional Path Report Mapping for UE Rx-Tx Time Difference	468

10.1.26	FR2 P-MPR report	470

10.1.26.1	Report mapping	470

10.1.27	L1-SINR accuracy requirements for FR1	470

10.1.27.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	470

10.1.27.1.1	Absolute Accuracy	470

10.1.27.1.2	Relative Accuracy	471

10.1.27.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	472

10.1.27.2.1	Absolute Accuracy	472

10.1.27.2.2	Relative Accuracy	473

10.1.27.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	475

10.1.27.3.1	Absolute Accuracy	475

10.1.27.3.2	Relative Accuracy	477

10.1.28	L1-SINR accuracy requirements for FR2	478

10.1.28.1	L1-SINR accuracy requirements with CSI-RS based CMR and no dedicated IMR configured	478

10.1.28.1.1	Absolute Accuracy	478

10.1.28.1.2	Relative Accuracy	479

10.1.28.2	L1-SINR accuracy requirements with SSB based CMR and dedicated IMR configured	480

10.1.28.2.1	Absolute Accuracy	480

10.1.28.2.2	Relative Accuracy	481

10.1.28.3	L1-SINR accuracy requirements with CSI-RS based CMR and dedicated IMR configured	482

10.1.28.3.1	Absolute Accuracy	482

10.1.28.3.2	Relative Accuracy	483

10.1.29	Intra-frequency RSRQ accuracy requirements under CCA	485

10.1.29.1	Intra-frequency SS-RSRQ accuracy requirements in FR1	485

10.1.29.1.1	Absolute SS-RSRQ Accuracy	485

10.1.30	Inter-frequency RSRQ accuracy requirements under CCA	485

10.1.30.1	Inter-frequency SS-RSRQ accuracy requirements in FR1	485

10.1.30.1.1	Aboslute Accuracy of SS-RSRQ	485

10.1.30.1.2	Relative Accuracy of SS-RSRQ	486

10.1.31	Intra-frequency SINR accuracy requirements under CCA	487

10.1.32	Inter-frequency SINR accuracy requirements under CCA	487

10.1.33	L1-RSRP accuracy requirements under CCA	489

10.1.34	RSSI measurements under CCA	490

10.1.34.1	Intra-frequency absolute RSSI measurement accuracy requirements in FR1	490

10.1.34.2	Inter-frequency absolute RSSI measurement accuracy requirements in FR1	490

10.1.34.3	RSSI measurement report mapping	491

10.1.35	Channel occupancy measurements under CCA	491

10.1.35.1	Intra-frequency channel occupancy measurement accuracy requirements in FR1	491

10.1.35.2	Inter-frequency channel occupancy measurement accuracy requirements in FR1	491

10.1.36	Intra-frequency RSRP accuracy requirements under CCA	492

10.1.36.1	Intra-frequency SS-RSRP accuracy requirements in FR1	492

10.1.36.1.1	Absolute SS-RSRP Accuracy	492

10.1.36.1.2	Relative SS-RSRP Accuracy	492

10.1.37	Inter-frequency RSRP accuracy requirements under CCA	493

10.1.37.1	Inter-frequency SS-RSRP accuracy requirements in FR1	493

10.1.37.1.1	Absolute Accuracy of SS-RSRP	493

10.1.37.1.2	Relative Accuracy of SS-RSRP	494

10.2	E-UTRAN measurements	494

10.2.1	Introduction	494

10.2.2	E-UTRAN RSRP measurements	495

10.2.3	E-UTRAN RSRQ measurements	495

10.2.4	E-UTRAN RSTD measurements	495

10.2.5	E-UTRAN RS-SINR measurements	495

10.2.6	E-UTRAN RSRP measurements for CA/DC Idle Mode Measurements	495

10.2.7	E-UTRAN RSRQ measurements for CA/DC Idle Mode Measurements	496

10.3	UTRAN FDD Measurements	496

10.3.1	UTRAN FDD CPICH RSCP	496

10.3.2	UTRAN FDD CPICH Ec/No	497

10.4	V2X measurements	497

10.4.1	Introduction	497

10.4.2	Intra-frequency PSBCH-RSRP accuracy requirements for FR1	498

10.4.2.1	PSBCH-RSRP Absolute Accuracy	498

10.4.2.2	PSBCH-RSRP Relative Accuracy	498

10.4.3	Intra-Frequency SL-RSSI Measurement Accuracy Requirements for FR1	499

10.4.3.1	Absolute SL-RSSI Accuracy	499

10.4.4	Intra-Frequency L1 SL-RSRP Measurement Accuracy Requirements for FR1	499

10.4.4.1	Absolute L1 SL-RSRP Accuracy	499

11	Void	500

12	V2X Requirements	500

12.1	Introduction	500

12.2	UE Transmit Timing	501

12.2.1	Introduction	501

12.2.2	GNSS as synchronization reference source	501

12.2.3	NR Cell as synchronization reference source	501

12.2.4	E-URTAN Cell as synchronization reference source	502

12.2.5	SyncRef UE as synchronization reference source	502

12.3	Initiation/Cease of SLSS Transmissions	502

12.3.1	Introduction	502

12.3.1.1	Initiation/Cease of SLSS transmissions with NR cell as synchronization reference source	502

12.3.1.2	Initiation/Cease of SLSS transmissions with EUTRAN cell as synchronization reference source	503

12.3.1.3	Initiation/Cease of SLSS transmissions with GNSS as synchronization reference source	504

12.3.1.4	Initiation/Cease of SLSS transmissions with SyncRef UE as synchronization reference source	504

12.4	Selection / Reselection of V2X Synchronization Reference Source	505

12.5	L1 SL-RSRP measurements	506

12.5.1	Introduction	506

12.5.2	SL-RSRP measurements	506

12.6	Congestion Control measurements	506

12.7	Interruption	506

12.7.1	Interruptions to WAN due to V2X Sidelink Communication	506

12.7.2	V2X Sidelink Communication Dropping due to synchronization source change	507

12.7.3	Interruptions to WAN due to switching between E-UTRA V2X Sidelink and NR V2X Sidelink	509

12.8	Reliability of GNSS signal	509

12.9	Scheduling availability	509

12.9.1	Scheduling availability of UE switching between E-UTRA sidelink and NR sidelink	509

13	Measurement Performance Requirements for NR gNB	510

13.1	UL-RTOA	510

13.1.1	Report mapping	510

13.2	gNB Rx-Tx time difference	512

13.2.1	Report mapping	512

13.2.2	Measurement Accuracy Requirements	514

13.2.2.1	Introduction	514

13.2.2.2	Requirements	514

13.3	UL SRS RSRP measurement	515

13.3.1	Report mapping	515

13.3.2	Measurement accuracy requirements	516

13.3.2.1	Introduction	516

13.3.2.2	Requirements	516

13.4	AoA/ZoA	517

13.4.1	Report mapping	517

Annex A (normative): Test Cases	423

A.1	Purpose of annex	423

A.2	Requirement classification for statistical testing	423

A.2.1	Types of requirements in TS 38.133	423

A.2.1.1	Time and delay requirements on UE higher layer actions	423

A.2.1.2	Measurements of power levels, relative powers and time	423

A.2.1.3	Implementation requirements	424

A.2.1.4	Physical layer timing requirements	424

A.2.1.5	Requirements under CCA	424

A.3	RRM test configurations	425

A.3.1	Reference measurement channels	425

A.3.1.1	PDSCH	425

A.3.1.1.1	FDD	425

A.3.1.1.2	TDD	426

A.3.1A	Reference measurement channels under CCA	429

A.3.1A.1	PDSCH	429

A.3.1A.1.1	TDD	429

A.3.1A.2	CORESET for RMSI scheduling	430

A.3.1A.2.1	TDD	430

A.3.1A.3	CORESET for RMC scheduling	431

A.3.1A.3.1	TDD	431

A.3.1A.4	TDD UL/DL configuration	432

A.3.1A.5	RMC burst transmission model	432

A.3.1.2	CORESET for RMSI scheduling	433

A.3.1.2.1	FDD	433

A.3.1.2.2	TDD	434

A.3.1.3	CORESET for RMC scheduling	437

A.3.1.3.1	FDD	437

A.3.1.3.2	TDD	438

A.3.1.4	TDD UL/DL configuration	441

A.3.2	OFDMA channel noise generator (OCNG)	442

A.3.2.1	Generic OFDMA Channel Noise Generator (OCNG)	442

A.3.2.1.1	OCNG pattern 1: Generic OCNG pattern for all unused REs	442

A.3.2.1.2	OCNG pattern 2: Generic OCNG pattern for all unused REs for 2AoA setup	443

A.3.2.1.3	OCNG pattern 3: Generic OCNG pattern for unused REs in the same bandwidth as CORESET	443

A.3.2.1.4	OCNG pattern 4: Generic OCNG pattern for all unused REs outside SSB slot(s)	444

A.3.2.2	Void	445

A.3.3	Reference DRX configurations	445

A.3.3.1	DRX Configuration 1: DRX cycle = 40 ms and TAT = 500 ms	445

A.3.3.2	DRX Configuration 2: DRX cycle = 640 ms and TAT = 500 ms	445

A.3.3.3	DRX Configuration 3: DRX cycle = 40 ms and TAT = Infinity	445

A.3.3.4	DRX Configuration 4: DRX cycle = 160 ms and TAT = Infinity	446

A.3.3.5	DRX Configuration 5: DRX cycle = 320 ms and TAT = Infinity	446

A.3.3.6	DRX Configuration 6: DRX cycle = 320 ms and TAT = 500 ms	446

A.3.3.7	DRX Configuration 7: DRX cycle = 640 ms and TAT = Infinity	447

A.3.3.8	DRX Configuration 8: DRX cycle = 320 ms and TAT = Infinity	447

A.3.3.9	DRX Configuration 9: DRX cycle = 40 ms and TAT = 500 ms	447

A.3.3.10	DRX Configuration 10: DRX cycle = 640 ms and TAT = 500 ms	448

A.3.3.11	DRX Configuration 11: DRX cycle = 20 ms and TAT = Infinity	448

A.3.3.12	DRX Configuration 12: DRX cycle = 640 ms and TAT = Infinity	448

A.3.4	Test Cases with Different Channel Bandwidths	448

A.3.4.1	Test Cases with Different E-UTRA Channel Bandwidths	448

A.3.4.1.1	Introduction	448

A.3.4.1.2	Principle of testing	449

A.3.5	Test Cases for Synchronous and Asynchronous DC Operations	449

A.3.5.1	EN-DC Test Cases for Synchronous and Asynchronous EN-DC Operations	449

A.3.5.1.1	Introduction	449

A.3.5.1.2	Principle of Testing	449

A.3.6	Antenna configurations	449

A.3.6.1	Antenna configurations for FR1	449

A.3.6.1.1	Antenna connection for 4 Rx capable UEs	449

A.3.6.1.1.1	Introduction	449

A.3.6.1.1.2	Principle of testing	449

A.3.6.2	Antenna configurations for FR2	452

A.3.6A	Antenna configurations with unlicensed bands	452

A.3.6A.1	Antenna configurations for FR1	452

A.3.6A.1.1	Antenna connection for 4 Rx capable UEs	452

A.3.6A.1.1.1	Introduction	452

A.3.6A.1.1.2	Principle of testing	452

A.3.7	EN-DC test setup	454

A.3.7.1	Introduction	454

A.3.7.2	E-UTRAN Serving Cell Parameters	454

A.3.7.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	454

A.3.7.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	455

A.3.7A	NR FR1-FR2 test setup	456

A.3.7B	EN-DC test setup with unlicensed bands	457

A.3.7B.1	Introduction	457

A.3.7B.2	E-UTRAN Serving Cell Parameters	457

A.3.7B.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) under CCA in FR1	457

A.3.7C	LTE-FR1/FR2 test setup	458

A.3.7D	NE-DC test setup	458

A.3.7D.1	Introduction	458

A.3.7D.2	E-UTRAN Serving Cell Parameters	458

A.3.7D.2.1	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR1	458

A.3.7D.2.2	E-UTRAN Serving Cell Parameters for Tests with NR Cell(s) in FR2	458

A.3.8	PRACH configurations	458

A.3.8.1	Introduction	458

A.3.8.2	PRACH configurations in FR1	459

A.3.8.2.1	FR1 PRACH configuration 1	459

A.3.8.2.2	FR1 PRACH configuration 2	459

A.3.8.2.3	FR1 PRACH configuration 3	460

A.3.8.2.4	FR1 PRACH configuration 4	461

A.3.8.3	PRACH configurations in FR2	462

A.3.8.3.1	FR2 PRACH configuration 1	462

A.3.8.3.2	FR2 PRACH configuration 2	463

A.3.8.3.3	FR2 PRACH configuration 3	464

A.3.8.3.4	FR2 PRACH configuration 4	465

A.3.8A	PRACH configurations under CCA	466

A.3.8A.1	Introduction	466

A.3.8A.2	PRACH configurations in FR1	466

A.3.8A.2.1	FR1 PRACH configuration 1 under CCA	466

A.3.8A.2.2	FR1 PRACH configuration 2 under CCA	467

A.3.9	BWP configurations	468

A.3.9.1	Introduction	468

A.3.9.2	Downlink BWP configurations	469

A.3.9.2.1	Initial BWP	469

A.3.9.2.2	Dedicated BWP	469

A.3.9.3	Uplink BWP configurations	469

A.3.9.3.1	Initial BWP	469

A.3.9.3.2	Dedicated BWP	470

A.3.10	SSB Configurations	470

A.3.10.1	SSB Configurations for FR1	470

A.3.10.1.1	SSB pattern 1 in FR1: SSB allocation for SSB SCS=15 kHz in 10 MHz	470

A.3.10.1.5	SSB pattern 5 in FR1: SSB allocation for SSB SCS=15 kHz starting from odd SFN in 10 MHz	472

A.3.10A	SSB Configurations under CCA	477

A.3.10A.1	SSB Configurations under CCA for FR1	477

A.3.10A.1.1	SSB pattern 1 under CCA for semi-static channel access: SSB allocation for SSB SCS=30kHz in 40MHz	477

A.3.10A.1.2	SSB pattern 2 under CCA for dynamic channel access: SSB allocation for SSB SCS=30kHz in 40MHz	478

A.3.10A.1.3	SSB pattern 3 under CCA for semi-static channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	478

A.3.10A.1.4	SSB pattern 4 under CCA for dynamic channel access: SSB allocation for SSB SCS=30 kHz in 40 MHz	479

A.3.11	SMTC Configurations	479

A.3.11.1	SMTC pattern 1: SMTC period = 20 ms with SMTC duration = 1 ms	479

A.3.11.2	SMTC pattern 2: SMTC period = 20 ms with SMTC duration = 5 ms	479

A.3.11.3	SMTC pattern 3: SMTC period = 160 ms with SMTC duration = 1 ms	480

A.3.11.4	SMTC pattern 4: SMTC period = 20 ms with SMTC duration = 1 ms	480

A.3.11.5	SMTC pattern 5: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.11.6	SMTC pattern 6: SMTC period = 20 ms with SMTC duration = 5 ms	480

A.3.12	Test Cases with Different CC Configurations	480

A.3.12.1 EN-DC Test Cases with Different EN-DC Configurations	480

A.3.12.1.1	Introduction	480

A.3.12.1.2	Principle of testing	481

A.3.12.2	Carrier Aggregation Test Cases with Different CA Configurations	481

A.3.12.2.1	Introduction	481

A.3.12.2.2	Principle of testing	481

A.3.13	Test Cases in SA and EN-DC Operations	481

A.3.13.1	Introduction	481

A.3.13.2	Principle of Testing	481

A.3.13A Test Cases involving E-UTRA/FR1 and FR2 carriers	482

A.3.13A.1	Introduction	482

A.3.13A.2	Principle of Testing in EN-DC	482

A.3.13A.3	Principle of Testing in SA	482

A.3.13B	Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.1	Active BWP switch Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.1.1	Introduction	483

A.3.13B.1.2	Principle of Testing	483

A.3.13B.2	SFTD accuracy Test Cases for EN-DC and NE-DC Operations	483

A.3.13B.2.1	Introduction	483

A.3.13B.2.2	Principle of Testing	483

A.3.14	CSI-RS configurations	484

A.3.14.1	FDD	484

A.3.14.2	TDD	486

A.3.15	Angle of Arrival (AoA) for FR2 RRM test cases	493

A.3.15.1	Setup 1: Single AoA in Rx beam peak direction	493

A.3.15.2	Setup 2: Single AoA in non Rx beam peak direction	493

A.3.15.2.1	Setup 2a: Single AoA in non Rx beam peak direction without change in direction	493

A.3.15.2.2	Setup 2b: Single AoA in non Rx beam peak direction with change in direction	494

A.3.15.3	Setup 3: 2 AoAs	494

A.3.15.4	Setup 4: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak	494

A.3.15.4.1	Setup 4a: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak without change in direction	494

A.3.15.4.2	Setup 4b: 2 AoAs, 1 AoA in Rx beam peak direction, 1 in non Rx beam peak with change in direction	494

A.3.16	TCI State Configuration	495

A.3.16.1	Introduction	495

A.3.16.2	TCI states	495

A.3.17	Configurations of CSI-RS for tracking	495

A.3.17.1	Configuration of CSI-RS for tracking for FR1	495

A.3.17.1.1	FDD	495

A.3.17.1.2	TDD	496

A.3.17.2	Configuration of CSI-RS for tracking for FR2	497

A.3.17.2.1	TDD	497

A.3.18	Additional definitions related to OTA testing for FR2 RRM test cases	498

A.3.18.1	Introduction	498

A.3.18.2	PRACH Power Measurement	498

A.3.19	Test applicability for DAPS handover	498

A.3.19.1	Introduction	498

A.3.19.2	Principle of testing	498

A.3.20	MsgA configurations	499

A.3.20.1	Introduction	499

A.3.20.2	MsgA configurations in FR1	499

A.3.20.2.1	FR1 MsgA configuration 1	499

A.3.20.2.2	FR1 MsgA configuration 2	500

A.3.20.3	MsgA configurations in FR2	502

A.3.20.3.1	FR2 MsgA configuration 1	502

A.3.20.3.2	FR2 MsgA configuration 2	503

A.3.20A	MsgA configurations under CCA	505

A.3.20A.1	Introduction	505

A.3.20A.2	MsgA configurations in FR1	505

A.3.20A.2.1	FR1 MsgA configuration 1 under CCA	505

A.3.20A.2.2	FR1 MsgA configuration 2 under CCA	506

A.3.21	V2X sidelink communication	509

A.3.21.1	Introduction	509

A.3.21.2	Reference resource pool configurations for V2X Sidelink Communication	509

A.3.21.3	Reference measurement channels for V2X Sidelink Communication	516

A.3.22	CSI-IM configurations	516

A.3.22.1	FDD	516

A.3.22.2	TDD	517

A.3.23	Spatial Relation Configuration	519

A.3.23.1	Introduction	519

A.3.23.2	Spatial Relation	519

A.3.24	SRS configuration	520

A.3.25	Channel bandwidth (CBW) configurations	522

A.3.25.1	DL UE specific CBW	522

A.3.25.2	UL UE specific CBW	522

A.3.26	CCA model	522

A.3.26.1	Introduction	522

A.3.26.2	CCA model for operation on a carrier frequency with CCA in FR1	522

A.3.26.2.1	DL CCA model	522

A.3.26.2.2	UL CCA model	524

A.3.27	Test Cases with at Least One Cell on a Carrier Frequency with CCA	524

A.3.27.1	Introduction	525

A.3.27.2	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	525

A.3.27.3	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	525

A.3.27.4	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	525

A.3.27.5	E-UTRA Standalone Tests with at Least One NR Cell under CCA	525

A.3.28	Discovery Burst Transmission Window configuration under CCA	525

A.3.28.1	DBT Window pattern 1: DBT Window period = 20 ms with DBT Window duration = 1 ms	525

A.3.29	Testing principles for UE capable of only NR bands with shared spectrum access	525

A.3.29.1	Introduction	525

A.3.29.2	Principle of testing for UE capable of EN-DC with only NR bands with shared spectrum access	526

A.3.29.3	Principle of testing for UE capable of SA operation with only NR bands with shared spectrum access	526

A.3.30	CSI-RS configurations for RRM	527

A.3.30.1	FDD	527

A.3.30.2	TDD	528

A.3.31	PRS Configurations	530

A.3.31.1.	PRS Configurations for FR1	530

A.3.31.1.1.	PRS pattern 1 in FR1: SCS=15 KHz	530

A.3.31.1.2.	PRS pattern 2 in FR1: SCS=30 KHz	531

A.3.31.2.	PRS Configurations for FR2	531

A.3.31.2.1.	PRS pattern 1 in FR2: SCS=120 KHz	531

A.4	EN-DC tests with all NR cells in FR1	532

A.4.1	Void	532

A.4.2	Void	532

A.4.3	RRC_CONNECTED state mobility	532

A.4.3.1	Void	532

A.4.3.2	RRC Connection Mobility Control	532

A.4.3.2.1	Void	532

A.4.3.2.2	Random Access	532

A.4.3.2.2.1	4-step RA type contention based random access test in FR1 for PSCell in EN-DC	532

A.4.3.2.2.2	4-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	536

A.4.3.2.2.3	2-step RA type contention based random access test in FR1 for PSCell in EN-DC	540

A.4.3.2.2.4	2-step RA type non-contention based random access test in FR1 for PSCell in EN-DC	544

A.4.3.2.3	Void	548

A.4.4	Timing	548

A.4.4.1	UE transmit timing	548

A.4.4.1.1	NR UE Transmit Timing Test for FR1	548

A.4.4.1.1.1	Test Purpose and environment	548

A.4.4.1.1.2	Test requirements	552

A.4.4.2	UE timer accuracy	552

A.4.4.3	Timing advance	552

A.4.4.3.1	EN-DC FR1 timing advance adjustment accuracy	552

A.4.4.3.1.1	Test Purpose and Environment	552

A.4.4.3.1.2	Test Parameters	552

A.4.4.3.1.3	Test Requirements	556

A.4.5	Signaling characteristics	556

A.4.5.1	Radio link Monitoring	556

A.4.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	557

A.4.5.1.1.1	Test Purpose and Environment	557

A.4.5.1.1.2	Test Requirements	561

A.4.5.1.2	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in non-DRX mode	561

A.4.5.1.2.1	Test Purpose and Environment	561

A.4.5.1.2.2	Test Requirements	567

A.4.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	567

A.4.5.1.3.1	Test Purpose and Environment	567

A.4.5.1.3.2	Test Requirements	573

A.4.5.1.4	Radio Link Monitoring In-sync Test for FR1 PSCell configured with SSB-based RLM RS in DRX mode	573

A.4.5.1.4.1	Test Purpose and Environment	573

A.4.5.1.4.2	Test Requirements	579

A.4.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	579

A.4.5.1.5.1	Test Purpose and Environment	579

A.4.5.1.5.2	Test Requirements	585

A.4.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in non-DRX mode	585

A.4.5.1.6.1	Test Purpose and Environment	585

A.4.5.1.6.2	Test Requirements	591

A.4.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	591

A.4.5.1.7.1	Test Purpose and Environment	591

A.4.5.1.7.2	Test Requirements	596

A.4.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR1 PSCell configured with CSI-RS-based RLM in DRX mode	597

A.4.5.1.8.1	Test Purpose and Environment	597

A.4.5.1.8.2	Test Requirements	602

A.4.5.2	Interruption	602

A.4.5.2.1	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	602

A.4.5.2.1.1	Test Purpose and Environment	602

A.4.5.2.1.2	Test Requirements	606

A.4.5.2.2	E-UTRAN – NR FR1 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	606

A.4.5.2.2.1	Test Purpose and Environment	606

A.4.5.2.2.2	Test Requirements	610

A.4.5.2.3	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	610

A.4.5.2.3.1	Test Purpose and Environment	610

A.4.5.2.3.2	Test Requirements	614

A.4.5.2.4	E-UTRAN – NR FR1 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	615

A.4.5.2.4.1	Test Purpose and Environment	615

A.4.5.2.4.2	Test Requirements	619

A.4.5.2.5	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	620

A.4.5.2.5.1	Test Purpose and Environment	620

A.4.5.2.5.2	Test Requirements	623

A.4.5.2.6	E-UTRAN – NR FR1 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	623

A.4.5.2.6.1	Test Purpose and Environment	623

A.4.5.2.6.2	Test Requirements	627

A.4.5.2.7	Void	627

A.4.5.2.8	E-UTRAN - NR FR1 interruptions at NR SRS carrier based switching in asynchronous EN-DC	627

A.4.5.2.8.1	Test Purpose and Environment	627

A.4.5.2.8.2	Test Requirements	630

A.4.5.2.9	E-UTRAN – NR interruptions at E-UTRA SRS carrier based switching	631

A.4.5.2.9.1	Test Purpose and Environment	631

A.4.5.2.9.2	Test Requirements	635

A.4.5.3	SCell Activation and Deactivation Delay	636

A.4.5.3.1	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	636

A.4.5.3.1.1	Test Purpose and Environment	636

A.4.5.3.1.2	Test Requirements	641

A.4.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 320 ms SCell measurement cycle	642

A.4.5.3.2.1	Test Purpose and Environment	642

A.4.5.3.2.2	Test Requirements	642

A.4.5.3.3	SCell Activation and deactivation of unknown SCell in FR1	642

A.4.5.3.3.1	Test Purpose and Environment	642

A.4.5.3.3.2	Test Requirements	643

A.4.5.3.4	SCell Activation and deactivation of multiple unknown SCells in FR1 with single activation/deactivation command	643

A.4.5.3.4.1	Test Purpose and Environment	643

A.4.5.3.4.2	Test Requirements	648

A.4.5.3.5	Direct SCell activation at SCell addition of known SCell in FR1	648

A.4.5.3.5.1	Test Purpose and Environment	648

A.4.5.3.5.2	Test Requirements	652

A.4.5.4	UE UL carrier RRC reconfiguration Delay	652

A.4.5.4.1	UE UL carrier RRC reconfiguration Delay	652

A.4.5.4.1.1 Test Purpose and Environment	652

A.4.5.4.1.2	Test Requirements	660

A.4.5.5	Beam Failure Detection and Link recovery procedures	661

A.4.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in non-DRX mode	661

A.4.5.5.1.1	Test Purpose and Environment	661

A.4.5.5.1.2	Test Requirements	666

A.4.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with SSB-based BFD and LR in DRX mode	666

A.4.5.5.2.1	Test Purpose and Environment	666

A.4.5.5.2.2	Test Requirements	672

A.4.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	672

A.4.5.5.3.1	Test Purpose and Environment	672

A.4.5.5.3.2	Test Requirements	678

A.4.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR1 PSCell configured with CSI-RS-based BFD and LR in DRX mode	678

A.4.5.5.4.1	Test Purpose and Environment	678

A.4.5.5.4.2	Test Requirements	684

A.4.5.5.5	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	684

A.4.5.5.5.1	Test Purpose and Environment	684

A.4.5.5.5.2	Test Requirements	690

A.4.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	690

A.4.5.5.6.1	Test Purpose and Environment	690

A.4.5.5.6.2	Test Requirements	697

A.4.5.6	Active BWP switch	698

A.4.5.6.1	DCI-based and Timer-based Active BWP Switch	698

A.4.5.6.1.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	698

A.4.5.6.1.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	703

A.4.5.6.2	RRC-based Active BWP Switch	709

A.4.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	713

A.4.5.6.3.1	Simultaneous E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in EN-DC on multiple CCs	713

A.4.5.6.3.1.1	Test Purpose and Environment	713

A.4.5.6.3.1.2	Test Requirements	718

A.4.5.6.4	SCell dormancy switch	719

A.4.5.6.4.1	E-UTRAN – NR FR1 PSCell SCell dormancy switch of single FR1 SCell outside active time	719

A.4.5.6.4.1.1	Test Purpose and Environment	719

A.4.5.6.4.1.2	Test Requirements	724

A.4.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR1 SCells inside active time	724

A.4.5.6.4.2.1	Test Purpose and Environment	724

A.4.5.6.4.2.2	Test Requirements	729

A.4.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	729

A.4.5.6.5.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC on multiple CCs	729

A.4.5.6.5.1.1	Test Purpose and Environment	729

A.4.5.6.5.1.2	Test Requirements	734

A.4.5.7	PSCell addition and release delay	734

A.4.5.7.1	Addition and Release Delay of known NR PSCell	734

A.4.5.7.1.1	Test purpose and environment	734

A.4.5.7.1.2	Test Requirements	738

A.4.5.8	DL Interruptions at switching between two uplink carriers	739

A.4.5.8.1	Test Purpose and Environment	739

A.4.5.8.2	Test Requirements	742

A.4.5.9	UE specific CBW change	742

A.4.5.9.1	UE specific CBW change on FR1 NR PSCell with non-DRX in synchronous EN- DC	742

A.4.5.9.1.1	Test Purpose and Environment	742

A.4.5.9.1.2	Test Requirements	747

A.4.6	Measurement procedure	747

A.4.6.1	Intra-frequency Measurements	747

A.4.6.1.1	EN-DC event triggered reporting tests without gap under non-DRX	747

A.4.6.1.1.1	Test purpose and Environment	747

A.4.6.1.1.2	Test parameters	747

A.4.6.1.1.3	Test Requirements	751

A.4.6.1.2	EN-DC event triggered reporting tests without gap under DRX	751

A.4.6.1.2.1	Test purpose and Environment	751

A.4.6.1.2.2	Test parameters	751

A.4.6.1.2.2	Test Requirements	755

A.4.6.1.3	EN-DC event triggered reporting tests with per-UE gaps under non-DRX	755

A.4.6.1.3.1	Test purpose and Environment	755

A.4.6.1.3.2	Test parameters	755

A.4.6.1.3.3	Test Requirements	759

A.4.6.1.4	EN-DC event triggered reporting tests with per-UE gaps under DRX	759

A.4.6.1.4.1	Test purpose and Environment	759

A.4.6.1.4.2	Test parameters	759

A.4.6.1.4.3	Test Requirements	763

A.4.6.1.5	EN-DC event triggered reporting tests without gap under non-DRX with SSB index reading	763

A.4.6.1.5.1	Test purpose and Environment	763

A.4.6.1.5.2	Test parameters	763

A.4.6.1.5.3	Test Requirements	765

A.4.6.1.6	EN-DC event triggered reporting tests with SSB index reading with per-UE gaps	766

A.4.6.1.6.1	Test purpose and Environment	766

A.4.6.1.6.2	Test parameters	766

A.4.6.1.6.3	Test Requirements	768

A.4.6.1.7	EN-DC event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	769

A.4.6.1.7.1	Test purpose and Environment	769

A.4.6.1.7.2	Test parameters	769

A.4.6.1.7.3	Test Requirements	773

A.4.6.2	Inter-frequency Measurements	773

A.4.6.2.1	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	773

A.4.6.2.1.1	Test Purpose and Environment	773

A.4.6.2.1.2	Test Requirements	777

A.4.6.2.2	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	777

A.4.6.2.2.1	Test Purpose and Environment	777

A.4.6.2.2.2	Test Requirements	781

A.4.6.2.3	Void	782

A.4.6.2.4	Void	782

A.4.6.2.5	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	782

A.4.6.2.5.1	Test Purpose and Environment	782

A.4.6.2.5.2	Test Requirements	786

A.4.6.2.6	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	787

A.4.6.2.6.1	Test Purpose and Environment	787

A.4.6.2.6.2	Test Requirements	791

A.4.6.2.7	Void	792

A.4.6.2.8	Void	792

A.4.6.3	Void	792

A.4.6.4	L1-RSRP measurement for beam reporting	792

A.4.6.4.1	SSB based L1-RSRP measurement when DRX is not used	792

A.4.6.4.1.1	Test Purpose and Environment	792

A.4.6.4.1.2	Test parameters	792

A.4.6.4.1.3	Test Requirements	795

A.4.6.4.2	SSB based L1-RSRP measurement when DRX is used	795

A.4.6.4.2.1	Test Purpose and Environment	795

A.4.6.4.2.2	Test parameters	796

A.4.6.4.2.3	Test Requirements	799

A.4.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	799

A.4.6.4.3.1	Test Purpose and Environment	799

A.4.6.4.3.2	Test parameters	800

A.4.6.4.3.3	Test Requirements	803

A.4.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	803

A.4.6.4.4.1	Test Purpose and Environment	803

A.4.6.4.4.2	Test parameters	804

A.4.6.4.4.3	Test Requirements	806

A.4.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	806

A.4.6.4.5.1	Test Purpose and Environment	806

A.4.6.4.5.2	Test parameters	807

A.4.6.4.2.3	Test Requirements	810

A.4.6.5	CLI measurements	810

A.4.6.5.1	SRS-RSRP measurement with non-DRX	810

A.4.6.5.1.1	Test Purpose and Environment	810

A.4.6.5.1.2	Test Parameters	811

A.4.6.5.1.3	Test Requirements	814

A.4.6.5.2	CLI-RSSI measurement with non-DRX	814

A.4.6.5.2.1	Test Purpose and Environment	814

A.4.6.5.2.2	Test Parameters	815

A.4.6.5.2.3	Test Requirements	816

A.4.6.6	Measurements with autonomous gaps	817

A.4.6.6.1	EN-DC intra-frequency CGI identification of NR FR1 cell with autonomous gaps in synchronous EN-DC	817

A.4.6.6.1.1	Test Purpose and Environment	817

A.4.6.6.1.2	Test Requirements	821

A.4.6.7	L1-SINR measurement for beam reporting	821

A.4.6.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR when DRX is not used	821

A.4.6.7.1.1	Test Purpose and Environment	821

A.4.6.7.1.2	Test parameters	822

A.4.6.7.1.3	Test Requirements	825

A.4.6.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	825

A.4.6.7.2.1	Test Purpose and Environment	825

A.4.6.7.2.2	Test parameters	826

A.4.6.7.2.3	Test Requirements	829

A.4.6.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	829

A.4.6.7.3.1	Test Purpose and Environment	829

A.4.6.7.3.2	Test parameters	830

A.4.6.7.3.3	Test Requirements	833

A.4.6.8	CSI-RS based intra-frequency Measurement	833

A.4.6.8.1	EN-DC event triggered reporting tests without gap under DRX	833

A.4.6.8.1.1	Test purpose and Environment	833

A.4.6.8.1.2	Test Requirements	836

A.4.6.9	CSI-RS based inter-frequency Measurement	837

A.4.6.9.1	EN-DC event triggered reporting tests for FR1 cell when non-DRX is used	837

A.4.6.9.1.1	Test Purpose and Environment	837

A.4.6.9.1.2	Test Requirements	840

A.4.7	Measurement Performance requirements	841

A.4.7.1	SS-RSRP	841

A.4.7.1.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	841

A.4.7.1.1.1	Test Purpose and Environment	841

A.4.7.1.1.2	Test parameters	841

A.4.7.1.1.3	Test Requirements	845

A.4.7.1.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	845

A.4.7.1.2.1	Test Purpose and Environment	845

A.4.7.1.2.2	Test parameters	846

A.4.7.1.2.3	Test Requirements	850

A.4.7.1.3	Void	850

A.4.7.2	SS-RSRQ	850

A.4.7.2.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	850

A.4.7.2.1.1	Test Purpose and Environment	850

A.4.7.2.1.2	Test Parameters	851

A.4.7.2.1.3	Test Requirements	856

A.4.7.2.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	856

A.4.7.2.2.1	Test Purpose and Environment	856

A.4.7.2.2.2	Test Parameters	856

A.4.7.2.2.3	Test Requirements	861

A.4.7.3	SS-SINR	861

A.4.7.3.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	861

A.4.7.3.1.1	Test Purpose and Environment	861

A.4.7.3.1.2	Test Parameters	861

A.4.7.3.1.3	Test Requirements	865

A.4.7.3.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	865

A.4.7.3.2.1	Test Purpose and Environment	865

A.4.7.3.2.2	Test Parameters	865

A.4.7.3.2.3	Test Requirements	870

A.4.7.4	L1-RSRP measurement for beam reporting	870

A.4.7.4.1	SSB based L1-RSRP measurement	870

A.4.7.4.1.1	Test Purpose and Environment	870

A.4.7.4.1.2	Test parameters	871

A.4.7.4.1.3	Test Requirements	874

A.4.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	874

A.4.7.4.2.1	Test Purpose and Environment	874

A.4.7.4.2.2	Test parameters	875

A.4.7.4.2.3	Test Requirements	878

A.4.7.5	SFTD accuracy	878

A.4.7.5.1	SFTD accuracy	878

A.4.7.5.1.1	Test Purpose and Environment	878

A.4.7.5.1.2	Test Parameters	878

A.4.7.5.1.3	Test Requirements	883

A.4.7.5.2	Void	883

A.4.7.5.3	Void	883

A.4.7.6	CLI measurements	883

A.4.7.6.1	EN-DC SRS-RSRP measurement accuracy with FR1 serving cell	883

A.4.7.6.1.1	Test Purpose and Environment	883

A.4.7.6.1.2	Test parameters	884

A.4.7.6.1.3	Test Requirements	889

A.4.7.6.2	EN-DC CLI-RSSI measurement accuracy with FR1 serving cell	889

A.4.7.6.2.1	Test Purpose and Environment	889

A.4.7.6.2.2	Test parameters	890

A.4.7.6.2.3	Test Requirements	893

A.4.7.7	L1-SINR measurement for beam reporting	893

A.4.7.7.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	893

A.4.7.7.1.1	Test Purpose and Environment	893

A.4.7.7.1.2	Test parameters	894

A.4.7.7.1.3	Test Requirements	898

A.4.7.7.2	L1-SINR measurement with SSB based CMR and dedicated IMR	898

A.4.7.7.2.1	Test Purpose and Environment	898

A.4.7.7.2.2	Test parameters	899

A.4.7.7.2.3	Test Requirements	903

A.4.7.7.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	903

A.4.7.7.3.1	Test Purpose and Environment	903

A.4.7.7.3.2	Test parameters	904

A.4.7.7.3.3	Test Requirements	907

A.4.7.8	CSI-RSRP	907

A.4.7.8.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	907

A.4.7.8.1.1	Test Purpose and Environment	907

A.4.7.8.1.2	Test parameters	908

A.4.7.8.1.3	Test Requirements	911

A.4.7.8.2	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	911

A.4.7.8.2.1	Test Purpose and Environment	911

A.4.7.8.2.2	Test parameters	912

A.4.7.8.2.3	Test Requirements	915

A.4.7.9	CSI-RSRQ	916

A.4.7.9.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	916

A.4.7.9.1.1	Test Purpose and Environment	916

A.4.7.9.1.2	Test Parameters	916

A.4.7.9.1.3	Test Requirements	921

A.4.7.9.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	921

A.4.7.9.2.1	Test Purpose and Environment	921

A.4.7.9.2.2	Test Parameters	921

A.4.7.2.2.3	Test Requirements	926

A.4.7.10	CSI-SINR	926

A.4.7.10.1	EN-DC Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	926

A.4.7.10.1.1	Test Purpose and Environment	926

A.4.7.10.1.2	Test Parameters	927

A.4.7.10.1.3	Test Requirements	930

A.4.7.10.2	EN-DC Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	930

A.4.7.10.2.1	Test Purpose and Environment	930

A.4.7.10.2.2	Test Parameters	930

A.4.7.10.2.3	Test Requirements	933

A.4.8	Void	934

A.4A	NE-DC test with all NR cells in FR1	934

A.4A.1	Signaling characteristics	934

A.4A.1.1	E-UTRAN PSCell addition	934

A.4A.1.1.1	Test purpose and environment	934

A.4A.1.1.2	Test Requirements	938

A.4A.1.2	Active BWP switch	939

A.4A.1.2.1	E-UTRAN PSCell – NR PCell FR1 DCI-based and Timer-based DL active BWP switch in non-DRX in synchronous NE-DC	939

A.4A.1.2.1.1	Test Purpose and Environment	939

A.4A.1.2.1.2	Test Requirements	943

A.4A.2	Measurement performance	944

A.4A.2.1	SFTD accuracy	944

A.4A.2.1.1	SFTD accuracy	944

A.4A.2.1.1.1	Test Purpose	944

A.4A.2.1.1.2	Test Environment	944

A.4A.2.1.1.3	Test Requirements	947

A.5	EN-DC tests with one or more NR cells in FR2	948

A.5.1	Void	948

A.5.2	Void	948

A.5.3	RRC_CONNECTED state mobility	948

A.5.3.1	Void	948

A.5.3.2	RRC Connection Mobility Control	948

A.5.3.2.1	Void	948

A.5.3.2.2	Random Access	948

A.5.3.2.2.1	4-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	948

A.5.3.2.2.2	4-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	951

A.5.3.2.2.3	2-step RA type contention based random access test in FR2 for PSCell/SCell in EN-DC	957

A.5.3.2.2.4	2-step RA type non-contention based random access test in FR2 for PSCell/SCell in EN-DC	960

A.5.3.2.3	Void	964

A.5.4	Timing	964

A.5.4.1	UE transmit timing	964

A.5.4.1.1	NR UE Transmit Timing Test for FR2	964

A.5.4.1.1.1	Test Purpose and environment	964

A.5.4.1.1.2	Test requirements	967

A.5.4.2	UE timer accuracy	968

A.5.4.3	Timing advance	968

A.5.4.3.1 EN-DC FR2 timing advance adjustment accuracy	968

A.5.4.3.1.1 Test Purpose and Environment	968

A.5.4.3.1.2 Test Parameters	968

A.5.4.3.1.3	Test Requirements	972

A.5.5	Signaling characteristics	972

A.5.5.1	Radio link Monitoring	972

A.5.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	972

A.5.5.1.1.1	Test Purpose and Environment	972

A.5.5.1.1.2	Test Requirements	975

A.5.5.1.2	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in non-DRX mode	976

A.5.5.1.2.1	Test Purpose and Environment	976

A.5.5.1.2.2	Test Requirements	979

A.5.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	980

A.5.5.1.3.1	Test Purpose and Environment	980

A.5.5.1.3.2	Test Requirements	983

A.5.5.1.4	Radio Link Monitoring In-sync Test for FR2 PSCell configured with SSB-based RLM RS in DRX mode	983

A.5.5.1.4.1	Test Purpose and Environment	983

A.5.5.1.4.2	Test Requirements	987

A.5.5.1.5	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	987

A.5.5.1.6	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in non-DRX mode	993

A.5.5.1.7	EN-DC Radio Link Monitoring Out-of-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	998

A.5.5.1.8	EN-DC Radio Link Monitoring In-sync Test for FR2 PSCell configured with CSI-RS-based RLM in DRX mode	1004

A.5.5.1.8.2	Test Requirements	1010

A.5.5.1.9	EN-DC Radio Link Monitoring UE Scheduling Restrictions on FR2	1010

A.5.5.1.9.1	Test Purpose and Environment	1010

A.5.5.1.9.2	Test Requirements	1013

A.5.5.2	Interruption	1013

A.5.5.2.1	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in synchronous EN-DC	1013

A.5.5.2.1.1	Test Purpose and Environment	1013

A.5.5.2.1.2	Test Requirements	1016

A.5.5.2.2	E-UTRAN – NR FR2 interruptions at transitions between active and non-active during DRX in asynchronous EN-DC	1016

A.5.5.2.2.1	Test Purpose and Environment	1016

A.5.5.2.2.2	Test Requirements	1019

A.5.5.2.3	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in synchronous EN-DC	1019

A.5.5.2.3.1	Test Purpose and Environment	1019

A.5.5.2.3.2	Test Requirements	1023

A.5.5.2.4	E-UTRAN – NR FR2 interruptions during measurements on deactivated NR SCC in asynchronous EN-DC	1023

A.5.5.2.4.1	Test Purpose and Environment	1023

A.5.5.2.4.2	Test Requirements	1026

A.5.5.2.5	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in synchronous EN-DC	1027

A.5.5.2.5.1	Test Purpose and Environment	1027

A.5.5.2.5.2	Test Requirements	1030

A.5.5.2.6	E-UTRAN – NR FR2 interruptions during measurements on deactivated E-UTRAN SCC in asynchronous EN-DC	1031

A.5.5.2.6.1	Test Purpose and Environment	1031

A.5.5.2.6.2	Test Requirements	1033

A.5.5.2.7	E-UTRAN – NR FR2 interruptions at E-UTRA SRS carrier based switching	1034

A.5.5.2.7.1	Test Purpose and Environment	1034

A.5.5.2.7.2	Test Requirements	1037

A.5.5.2.8 E-UTRAN – NR FR2 interruptions at NR SRS carrier based switching	1037

A.5.5.2.8.1 Test Purpose and Environment	1037

A.5.5.2.8.3	Test Requirements	1039

A.5.5.3	SCell Activation and Deactivation Delay	1039

A.5.5.3.1	SCell Activation and deactivation of SCell in FR2 intra-band	1039

A.5.5.3.1.1	Test Purpose and Environment	1039

A.5.5.3.1.2	Test Requirements	1042

A.5.5.3.2	SCell Activation and deactivation of known SCell in FR1 for 160ms SCell measurement cycle	1042

A.5.5.3.2.1	Test Purpose and Environment	1042

A.5.5.3.2.2	Test Requirements	1046

A.5.5.3.3	Void	1046

A.5.5.3.4	Void	1046

A.5.5.3.5	SCell Activation and deactivation of SCell in FR2	1046

A.5.5.3.5.1	Test Purpose and Environment	1046

A.5.5.3.5.2	Test Requirements	1051

A.5.5.3.6	Multiple SCell Activation and deactivation of one unknown SCell and one known SCell in FR2	1052

A.5.5.3.6.1	Test Purpose and Environment	1052

A.5.5.3.6.2	Test Requirements	1055

A.5.5.3.7	Direct SCell activation at SCell addition of known SCell in FR2	1056

A.5.5.3.7.1	Test Purpose and Environment	1056

A.5.5.3.7.2	Test Requirements	1059

A.5.5.4	Void	1059

A.5.5.5	Beam Failure Detection and Link recovery procedures	1059

A.5.5.5.1	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1059

A.5.5.5.1.1	Test Purpose and Environment	1059

A.5.5.5.1.2	Test Requirements	1065

A.5.5.5.2	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with SSB-based BFD and LR in DRX mode	1065

A.5.5.5.2.1	Test Purpose and Environment	1065

A.5.5.5.2.2	Test Requirements	1071

A.5.5.5.3	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in non-DRX mode	1071

A.5.5.5.3.1	Test Purpose and Environment	1071

A.5.5.5.3.2	Test Requirements	1077

A.5.5.5.4	EN-DC Beam Failure Detection and Link Recovery Test for FR2 PSCell configured with CSI-RS-based BFD and LR in DRX mode	1077

A.5.5.5.4.1	Test Purpose and Environment	1077

A.5.5.5.4.2	Test Requirements	1083

A.5.5.5.5	EN-DC scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PSCell configured with SSB-based BFD and LR in non-DRX mode	1083

A.5.5.5.5.1	Test Purpose and Environment	1083

A.5.5.5.5.2	Test Requirements	1089

A.5.5.5.6	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1089

A.5.5.5.6.1	Test Purpose and Environment	1089

A.5.5.5.6.2	Test Requirements	1094

A.5.5.5.7	EN-DC Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1094

A.5.5.5.7.1	Test Purpose and Environment	1094

A.5.5.5.7.2	Test Requirements	1100

A.5.5.6	Active BWP switch	1100

A.5.5.6.1	DCI-based and Timer-based Active BWP Switch	1100

A.5.5.6.1.1	E-UTRAN – NR PSCell FR2 DL active BWP switch with non-DRX in synchronous EN-DC	1100

A.5.5.6.1.1.1	Test Purpose and Environment	1100

A.5.5.6.1.1.2	Test Requirements	1104

A.5.5.6.1.2	E-UTRAN – NR PSCell FR2 with FR2 SCell DL active BWP switch in non-DRX in synchronous EN-DC	1104

A.5.5.6.2	RRC-based Active BWP Switch	1109

A.5.5.6.3 Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1113

A.5.5.6.3.1	E-UTRAN – NR PSCell FR2 and NR SCell FR2 DL active BWP switch on multiple CCs in synchronous EN-DC	1113

A.5.5.6.3.1.1	Test Purpose and Environment	1113

A.5.5.6.3.1.2	Test Requirements	1117

A.5.5.6.4	SCell dormancy switch	1117

A.5.5.6.4.1	E-UTRAN – NR FR2 PSCell SCell dormancy switch of single FR2 SCell inside active time	1117

A.5.5.6.4.1.1	Test Purpose and Environment	1117

A.5.5.6.4.1.2	Test Requirements	1121

A.5.5.6.4.2	E-UTRAN – NR FR1 PSCell SCell dormancy switch of two FR2 SCells outside active time	1121

A.5.5.6.4.2.1	Test Purpose and Environment	1121

A.5.5.6.4.2.2	Test Requirements	1128

A.5.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1128

A.5.5.7	PSCell addition and release delay	1131

A.5.5.7.1	Addition and Release Delay of NR PSCell	1131

A.5.5.7.1.1	Test purpose and environment	1131

A.5.5.7.1.2	Test Requirements	1135

A.5.5.8	Active TCI state switch delay	1136

A.5.5.8.1	MAC-CE based active TCI state switch	1136

A.5.5.8.1.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1136

A.5.5.8.1.1.1	Test Purpose and Environment	1136

A.5.5.8.1.1.2	Test Requirements	1138

A.5.5.8.2	RRC based active TCI state switch	1138

A.5.5.8.2.1	E-UTRAN – NR PSCell FR2 active TCI state switch for a known TCI state	1138

A.5.5.8.2.1.1	Test Purpose and Environment	1138

A.5.5.8.2.1.2	Test Requirements	1141

A.5.5.9	Uplink spatial relation switch delay	1141

A.5.5.9.1	MAC-CE based uplink spatial relation switch	1141

A.5.5.9.1.1	E-UTRAN – NR PSCell FR2 uplink spatial relation switch for a known spatial relation	1141

A.5.5.9.1.1.1	Test Purpose and Environment	1141

A.5.5.9.1.1.2	Test Requirements	1144

A.5.5.9.2	RRC based spatial relation switch	1144

A.5.5.9.2.1	E-UTRAN – NR PSCell FR2 spatial relation switch associated with a known DL-RS	1144

A.5.5.9.2.1.1	Test Purpose and Environment	1144

A.5.5.9.2.1.2	Test Requirements	1147

A.5.5.10	UE specific CBW change	1147

A.5.5.10.1	UE specific CBW change on FR2 NR PSCell	1147

A.5.5.10.1.1	Test Purpose and Environment	1147

A.5.5.10.1.2	Test Requirements	1150

A.5.6	Measurement procedure	1151

A.5.6.1	Intra-frequency Measurements	1151

A.5.6.1.1	EN-DC event triggered reporting test without gap under non-DRX	1151

A.5.6.1.1.1	Test purpose and Environment	1151

A.5.6.1.1.2	Test Requirements	1155

A.5.6.1.2	EN-DC event triggered reporting test without gap under DRX	1155

A.5.6.1.2.1	Test purpose and Environment	1155

A.5.6.1.2.2	Test Requirements	1157

A.5.6.1.3	EN-DC event triggered reporting test with per-UE gaps under non-DRX	1158

A.5.6.1.3.1	Test purpose and Environment	1158

A.5.6.1.3.2	Test Requirements	1162

A.5.6.1.4	EN-DC event triggered reporting test with per-UE gaps under DRX	1162

A.5.6.1.4.1	Test purpose and Environment	1162

A.5.6.1.4.2	Test Requirements	1165

A.5.6.2	Inter-frequency Measurements	1166

A.5.6.2.1 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1166

A.5.6.2.1.1	Test Purpose and Environment	1166

A.5.6.2.1.2	Test Requirements	1169

A.5.6.2.2 	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1169

A.5.6.2.2.1	Test Purpose and Environment	1169

A.5.6.2.2.2	Test Requirements	1173

A.5.6.2.3 	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1173

A.5.6.2.3.1	Test Purpose and Environment	1173

A.5.6.2.3.2	Test Requirements	1177

A.5.6.2.4	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1177

A.5.6.2.4.1	Test Purpose and Environment	1177

A.5.6.2.4.2	Test Requirements	1181

A.5.6.2.5	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is not used	1181

A.5.6.2.5.1	Test Purpose and Environment	1181

A.5.6.2.5.2	Test Requirements	1186

A.5.6.2.6	EN-DC event triggered reporting tests for FR2 cell without SSB time index detection when DRX is used	1186

A.5.6.2.6.1	Test Purpose and Environment	1186

A.5.6.2.6.2	Test Requirements	1190

A.5.6.2.7	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is not used	1190

A.5.6.2.7.1	Test Purpose and Environment	1190

A.5.6.2.7.2	Test Requirements	1195

A.5.6.2.8	EN-DC event triggered reporting tests for FR2 cell with SSB time index detection when DRX is used	1195

A.5.6.2.8.1	Test Purpose and Environment	1195

A.5.6.2.8.2	Test Requirements	1200

A.5.6.3	L1-RSRP measurement for beam reporting	1200

A.5.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1200

A.5.6.3.1.1	Test Purpose and Environment	1200

A.5.6.3.1.2	Test parameters	1200

A.5.6.3.1.3	Test Requirements	1202

A.5.6.3.2	SSB based L1-RSRP measurement when DRX is used	1202

A.5.6.3.2.1	Test Purpose and Environment	1202

A.5.6.3.2.2	Test parameters	1203

A.5.6.3.2.3	Test Requirements	1205

A.5.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1205

A.5.6.3.3.1	Test Purpose and Environment	1205

A.5.6.3.3.2	Test parameters	1206

A.5.6.3.3.3	Test Requirements	1208

A.5.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1209

A.5.6.3.4.1	Test Purpose and Environment	1209

A.5.6.3.4.2	Test parameters	1209

A.5.6.3.4.3	Test Requirements	1211

A.5.6.4	CLI measurements	1212

A.5.6.4.1	SRS-RSRP measurement with DRX	1212

A.5.6.4.1.1	Test Purpose and Environment	1212

A.5.6.4.1.2	Test Parameters	1212

A.5.6.4.1.3	Test Requirements	1214

A.5.6.4.2	CLI-RSSI measurement with DRX	1214

A.5.6.4.2.1	Test Purpose and Environment	1214

A.5.6.4.2.2	Test Parameters	1215

A.5.6.4.2.3	Test Requirements	1217

A.5.6.5	Measurements with autonomous gaps	1217

A.5.6.5.1 	EN-DC inter-frequency CGI identification of NR neighbor cell in FR2	1217

A.5.6.5.1.1	Test Purpose and Environment	1217

A.5.6.5.1.2	Test Requirements	1220

A.5.6.6	L1-SINR measurement for beam reporting	1221

A.5.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1221

A.5.6.6.1.1	Test Purpose and Environment	1221

A.5.6.6.1.2	Test parameters	1221

A.5.6.6.1.3	Test Requirements	1223

A.5.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1223

A.5.6.6.2.1	Test Purpose and Environment	1223

A.5.6.6.2.2	Test parameters	1224

A.5.6.6.2.3	Test Requirements	1227

A.5.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1227

A.5.6.6.3.1	Test Purpose and Environment	1227

A.5.6.6.3.2	Test parameters	1227

A.5.6.6.3.3	Test Requirements	1229

A.5.6.7	CSI-RS based Intra-frequency Measurements	1229

A.5.6.7.1	EN-DC event triggered reporting test without gap under non-DRX	1229

A.5.6.7.1.1	Test purpose and Environment	1229

A.5.6.7.1.2	Test Requirements	1232

A.5.6.8	CSI-RS based Inter-frequency Measurements	1232

A.5.6.8.1 	EN-DC event triggered reporting tests for NR FR2 cell when DRX is used	1232

A.5.6.8.1.1	Test Purpose and Environment	1232

A.5.6.8.1.2	Test Requirements	1236

A.5.7	Measurement Performance requirements	1236

A.5.7.1	SS-RSRP	1237

A.5.7.1.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1237

A.5.7.1.1.1	Test Purpose and Environment	1237

A.5.7.1.1.2	Test parameters	1237

A.5.7.1.1.3	Test Requirements	1239

A.5.7.1.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1240

A.5.7.1.2.1	Test Purpose and Environment	1240

A.5.7.1.2.2	Test parameters	1240

A.5.7.1.2.3	Test Requirements	1244

A.5.7.1.3	EN-DC inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	1245

A.5.7.1.3.1	Test Purpose and Environment	1245

A.5.7.1.3.2	Test parameters	1245

A.5.7.1.3.3	Test Requirements	1248

A.5.7.2	SS-RSRQ	1249

A.5.7.2.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1249

A.5.7.2.1.1	Test Purpose and Environment	1249

A.5.7.2.1.2	Test Parameters	1249

A.5.7.2.1.3	Test Requirements	1251

A.5.7.2.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1252

A.5.7.2.2.1	Test Purpose and Environment	1252

A.5.7.2.2.2	Test Parameters	1252

A.5.7.2.2.3	Test Requirements	1254

A.5.7.3	SS-SINR	1254

A.5.7.3.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1254

A.5.7.3.1.1	Test Purpose and Environment	1254

A.5.7.3.1.2	Test Parameters	1254

A.5.7.3.1.3	Test Requirements	1256

A.5.7.3.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1256

A.5.7.3.2.1	Test Purpose and Environment	1256

A.5.7.3.2.2	Test Parameters	1256

A.5.7.3.2.3	Test Requirements	1258

A.5.7.4	L1-RSRP measurement for beam reporting	1258

A.5.7.4.1	SSB based L1-RSRP measurement	1258

A.5.7.4.1.1	Test Purpose and Environment	1258

A.5.7.4.1.2	Test parameters	1259

A.5.7.4.1.3	Test Requirements	1261

A.5.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1262

A.5.7.4.2.1	Test Purpose and Environment	1262

A.5.7.4.2.2	Test parameters	1262

A.5.7.4.2.3	Test Requirements	1264

A.5.7.5	CLI measurements	1265

A.5.7.5.1	EN-DC SRS-RSRP measurement accuracy with FR2 serving cell	1265

A.5.7.5.1.1	Test Purpose and Environment	1265

A.5.7.5.1.2	Test parameters	1265

A.5.7.5.1.3	Test Requirements	1268

A.5.7.5.2	EN-DC CLI-RSSI measurement accuracy with FR2 serving cell	1269

A.5.7.5.2.1	Test Purpose and Environment	1269

A.5.7.5.2.2	Test parameters	1269

A.5.7.5.2.3	Test Requirements	1271

A.5.7.6	L1-SINR measurement for beam reporting	1272

A.5.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1272

A.5.7.6.1.1	Test Purpose and Environment	1272

A.5.7.6.1.2	Test parameters	1272

A.5.7.6.1.3	Test Requirements	1274

A.5.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1274

A.5.7.6.2.1	Test Purpose and Environment	1274

A.5.7.6.2.2	Test parameters	1275

A.5.7.6.2.3	Test Requirements	1277

A.5.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1278

A.5.7.6.3.1	Test Purpose and Environment	1278

A.5.7.6.3.2	Test parameters	1278

A.5.7.6.3.3	Test Requirements	1280

A.5.7.7	CSI-RSRP	1280

A.5.7.7.1	EN-DC intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1280

A.5.7.7.1.2	Test parameters	1281

A.5.7.7.1.3	Test Requirements	1283

A.5.7.7.2	EN-DC inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	1284

A.5.7.7.2.1	Test Purpose and Environment	1284

A.5.7.7.2.2	Test parameters	1284

A.5.7.7.2.3	Test Requirements	1288

A.5.7.8	CSI-RSRQ	1289

A.5.7.8.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	1289

A.5.7.8.1.1	Test Purpose and Environment	1289

A.5.7.8.1.2	Test Parameters	1289

A.5.7.8.1.3	Test Requirements	1291

A.5.7.8.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1291

A.5.7.8.2.1	Test Purpose and Environment	1291

A.5.7.8.2.2	Test Parameters	1291

A.5.7.9	CSI-SINR	1293

A.5.7.9.1	EN-DC Intra-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1293

A.5.7.9.1.1	Test Purpose and Environment	1293

A.5.7.9.1.2	Test Parameters	1293

Table A.5.7.9.1.2-1: CSI-SINR Intra frequency CSI-SINR supported test configurations	1294

A.5.7.9.1.3	Test Requirements	1296

A.5.7.9.2	EN-DC Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	1296

A.5.7.9.2.1	Test Purpose and Environment	1296

A.5.7.9.2.2	Test Parameters	1296

A.5.7.9.2.3	Test Requirements	1299

A.5.8	Void	1299

A.6	NR standalone tests with all NR cells in FR1	1011

A.6.1	SA: RRC_IDLE state mobility	1011

A.6.1.1	Cell re-selection to NR	1011

A.6.1.1.1	Cell reselection to FR1 intra-frequency NR case	1011

A.6.1.1.1.1	Test Purpose and Environment	1011

A.6.1.1.1.2	Test Parameters	1011

A.6.1.1.1.3	Test Requirements	1015

A.6.1.1.2	Cell reselection to FR1 inter-frequency NR case	1015

A.6.1.1.2.1	Test Purpose and Environment	1015

A.6.1.1.2.2	Test Parameters	1015

A.6.1.1.2.3	Test Requirements	1019

A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1019

A.6.1.1.3.1	Test Purpose and Environment	1019

A.6.1.1.3.2	Test Parameters	1019

A.6.1.1.3.3	Test Requirements	1024

A.6.1.1.4	Cell reselection to FR1 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1024

A.6.1.1.4.1	Test Purpose and Environment	1024

A.6.1.1.4.2	Test Parameters	1024

A.6.1.1.4.3	Test Requirements	1028

A.6.1.1.5	Cell reselection to FR1 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1028

A.6.1.1.5.1	Test Purpose and Environment	1028

A.6.1.1.5.2	Test Parameters	1028

A.6.1.1.5.3	Test Requirements	1032

A.6.1.1.6	Cell reselection to FR1 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1033

A.6.1.1.6.1	Test Purpose and Environment	1033

A.6.1.1.6.2	Test Parameters	1033

A.6.1.1.6.3	Test Requirements	1037

A.6.1.1.7	Cell reselection to FR1 intra-frequency NR case for UE configured with highSpeedMeasFlag-r16	1038

A.6.1.1.7.1	Test Purpose and Environment	1038

A.6.1.1.7.2	Test Parameters	1038

A.6.1.1.7.3	Test Requirements	1042

A.6.1.2	Inter-RAT E-UTRAN cell re-selection	1042

A.6.1.2.1	Cell reselection to higher priority E-UTRAN	1042

A.6.1.2.1.1	Test Purpose and Environment	1042

A.6.1.2.1.2	Test Parameters	1042

A.6.1.2.1.3	Test Requirements	1045

A.6.1.2.2	Cell reselection to lower priority E-UTRAN	1046

A.6.1.2.2.1	Test Purpose and Environment	1046

A.6.1.2.2.2	Test Parameters	1046

A.6.1.2.2.3	Test Requirements	1049

A.6.1.2.3	Cell reselection to lower priority E-UTRAN for UE fulfilling low mobility relaxed measurement criterion	1050

A.6.1.2.3.1	Test Purpose and Environment	1050

A.6.1.2.3.2	Test Parameters	1050

A.6.1.2.3.3	Test Requirements	1053

A.6.1.2.4	Cell reselection to lower priority E-UTRAN for UE fulfilling not-at-cell edge relaxed measurement criterion	1054

A.6.1.2.4.1	Test Purpose and Environment	1054

A.6.1.2.4.2	Test Parameters	1054

A.6.1.2.4.3	Test Requirements	1057

A.6.1.2.5.3	Test Requirements	1061

A.6.2	SA: RRC_INACTIVE state mobility	1062

A.6.3	RRC_CONNECTED state mobility	1062

A.6.3.1	Handover	1062

A.6.3.1.1	Intra-frequency handover from FR1 to FR1; known target cell	1062

A.6.3.1.1.1	Test Purpose and Environment	1062

A.6.3.1.1.2	Test Parameters	1062

A.6.3.1.1.3 Test Requirements	1066

A.6.3.1.2	Intra-frequency handover from FR1 to FR1; unknown target cell	1066

A.6.3.1.2.1	Test Purpose and Environment	1066

A.6.3.1.2.2	Test Parameters	1066

A.6.3.1.2.3	Test Requirements	1070

A.6.3.1.3	Inter-frequency handover from FR1 to FR1; unknown target cell	1070

A.6.3.1.3.1	Test Purpose and Environment	1070

A.6.3.1.3.2	Test Parameters	1070

A.6.3.1.3.3 Test Requirements	1074

A.6.3.1.4	 SA NR - E-UTRAN handover	1074

A.6.3.1.4.1	Test Purpose and Environment	1074

A.6.3.1.4.2	Test Requirements	1080

A.6.3.1.5	SA NR - E-UTRAN handover with unknown target cell	1080

A.6.3.1.5.1	Test Purpose and Environment	1080

A.6.3.1.5.2	Test Requirements	1086

A.6.3.1.6	 SA NR - UTRAN FDD handover	1086

A.6.3.1.6.1	Test Purpose and Environment	1086

A.6.3.1.6.2	Test Requirements	1090

A.6.3.1.7	Intra-frequency synchronous DAPS handover in FR1	1090

A.6.3.1.7.1	Test Purpose and Environment	1090

A.6.3.1.7.2	Test Parameters	1090

A.6.3.1.7.3 Test Requirements	1094

A.6.3.1.8	Intra-frequency asynchronous DAPS handover in FR1	1095

A.6.3.1.8.1	Test Purpose and Environment	1095

A.6.3.1.8.2	Test Parameters	1095

A.6.3.1.8.3 Test Requirements	1099

A.6.3.1.9	Intra-band inter-frequency synchronous DAPS handover test in SA for FR1	1100

A.6.3.1.9.1	Test Purpose and Environment	1100

A.6.3.1.9.2	Test Parameters	1100

A.6.3.1.9.3 Test Requirements	1104

A.6.3.1.10	Intra-band inter-frequency asynchronous DAPS handover test in SA for FR1	1104

A.6.3.1.10.1	Test Purpose and Environment	1104

A.6.3.1.10.2	Test Parameters	1104

A.6.3.1.10.3 Test Requirements	1108

A.6.3.1.11	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR1	1108

A.6.3.1.11.1	Test Purpose and Environment	1108

A.6.3.1.11.2	Test Parameters	1108

A.6.3.1.11.3 Test Requirements	1115

A.6.3.1.12	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR1	1115

A.6.3.1.12.1	Test Purpose and Environment	1115

A.6.3.1.12.2	Test Parameters	1115

A.6.3.1.12.3 Test Requirements	1122

A.6.3.2	RRC Connection Mobility Control	1122

A.6.3.2.1	SA: RRC Re-establishment	1122

A.6.3.2.1.1	Intra-frequency RRC Re-establishment in FR1	1122

A.6.3.2.1.2	Inter-frequency RRC Re-establishment in FR1	1126

A.6.3.2.1.3	Intra-frequency RRC Re-establishment in FR1 without serving cell timing	1130

A.6.3.2.2	Random Access	1133

A.6.3.2.2.1	4-step RA type contention based random access test in FR1 for NR standalone	1133

A.6.3.2.2.2	4-step RA type non-contention based random access test in FR1 for NR standalone	1138

A.6.3.2.2.3	2-step RA type contention based random access test in FR1 for NR standalone	1142

A.6.3.2.2.4	2-step RA type non-contention based test in FR1 for NR standalone	1147

A.6.3.2.3	SA: RRC Connection Release with Redirection	1151

A.6.3.2.3.1	Redirection from NR in FR1 to NR in FR1	1151

A.6.3.2.3.2	Redirection from NR in FR1 to E-UTRAN	1155

A.6.3.3	Conditional handover	1162

A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1	1162

A.6.3.3.1.1	Test Purpose and Environment	1162

A.6.3.3.1.2	Test Parameters	1162

A.6.3.3.1.3 Test Requirements	1166

A.6.3.3.2	Inter-frequency conditional handover from FR1 to FR1	1166

A.6.3.3.2.1	Test Purpose and Environment	1166

A.6.3.3.2.2	Test Parameters	1166

A.6.3.3.2.3 Test Requirements	1170

A.6.4	Timing	1170

A.6.4.1	UE transmit timing	1170

A.6.4.1.1	NR UE Transmit Timing Test for FR1	1170

A.6.4.1.1.1	Test Purpose and environment	1170

A.6.4.1.1.2	Test requirements	1174

A.6.4.2	UE timer accuracy	1174

A.6.4.3	Timing advance	1174

A.6.4.3.1	SA FR1 timing advance adjustment accuracy	1174

A.6.4.3.1.1	Test Purpose and Environment	1174

A.6.4.3.1.2	Test Parameters	1174

A.6.4.3.1.3	Test Requirements	1178

A.6.5	Signalling characteristics	1178

A.6.5.1	Radio link Monitoring	1178

A.6.5.1.1	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1179

A.6.5.1.1.1	Test Purpose and Environment	1179

A.6.5.1.1.2	Test Requirements	1184

A.6.5.1.2	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in non-DRX mode	1184

A.6.5.1.2.1	Test Purpose and Environment	1184

A.6.5.1.2.2	Test Requirements	1190

A.6.5.1.3	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1190

A.6.5.1.3.1	Test Purpose and Environment	1190

A.6.5.1.3.2	Test Requirements	1196

A.6.5.1.4	Radio Link Monitoring In-sync Test for FR1 PCell configured with SSB-based RLM RS in DRX mode	1196

A.6.5.1.4.1	Test Purpose and Environment	1196

A.6.5.1.4.2	Test Requirements	1202

A.6.5.1.5	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1202

A.6.5.1.5.1	Test Purpose and Environment	1202

A.6.5.1.5.2	Test Requirements	1208

A.6.5.1.6	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in non-DRX mode	1208

A.6.5.1.6.1	Test Purpose and Environment	1208

A.6.5.1.6.2	Test Requirements	1214

A.6.5.1.7	Radio Link Monitoring Out-of-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1214

A.6.5.1.7.1	Test Purpose and Environment	1214

A.6.5.1.7.2	Test Requirements	1218

A.6.5.1.8	Radio Link Monitoring In-sync Test for FR1 PCell configured with CSI-RS-based RLM in DRX mode	1218

A.6.5.1.8.1	Test Purpose and Environment	1218

A.6.5.1.8.2	Test Requirements	1224

A.6.5.2	Interruption	1224

A.6.5.2.1	Interruptions during measurements on deactivated NR SCC in FR1	1224

A.6.5.2.1.2	Test Requirements	1228

A.6.5.2.2	SA interruptions at NR SRS carrier based switching	1229

A.6.5.2.2.1	Test Purpose and Environment	1229

A.6.5.2.2.2	Test Parameters	1229

A.6.5.2.2.3	Test Requirements	1231

A.6.5.3	SCell Activation and Deactivation Delay	1232

A.6.5.3.1	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160ms SCell measurement cycle	1232

A.6.5.3.1.1	Test Purpose and Environment	1232

A.6.5.3.1.2	Test Requirements	1236

A.6.5.3.2	SCell Activation and deactivation of known SCell in FR1 in non-DRX for 320ms SCell measurement cycle	1237

A.6.5.3.2.1	Test Purpose and Environment	1237

A.6.5.3.2.2	Test Requirements	1237

A.6.5.3.3	SCell Activation and deactivation of unknown SCell in FR1 in non-DRX	1237

A.6.5.3.3.1	Test Purpose and Environment	1237

A.6.5.3.3.2	Test Requirements	1238

A.6.5.3.4	Direct SCell activation at SCell addition of known SCell in FR1	1238

A.6.5.3.4.1	 Test Purpose and Environment	1238

A.6.5.3.4.2	 Test Requirements	1242

A.6.5.3.5	Direct SCell activation at handover with known SCell in FR1	1242

A.6.5.3.5.1	Test Purpose and Environment	1242

A.6.5.3.5.2	Test Requirements	1246

A.6.5.4	UE UL carrier RRC reconfiguration Delay	1247

A.6.5.4.1	UE UL carrier RRC reconfiguration Delay	1247

A.6.5.4.1.1	Test Purpose and Environment	1247

A.6.5.4.1.2	Test Requirements	1257

A.6.5.4.2	Void	1258

A.6.5.5	Beam Failure Detection and Link recovery procedures	1258

A.6.5.5.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	1258

A.6.5.5.1.1	Test Purpose and Environment	1258

A.6.5.5.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	1263

A.6.5.5.2.1	Test Purpose and Environment	1263

A.6.5.5.2.2	Test Requirements	1270

A.6.5.5.3	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1270

A.6.5.5.3.1	Test Purpose and Environment	1270

A.6.5.5.3.2	Test Requirements	1276

A.6.5.5.4	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in DRX mode	1276

A.6.5.5.4.1	Test Purpose and Environment	1276

A.6.5.5.4.2	Test Requirements	1282

A.6.5.5.5	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode	1282

A.6.5.5.5.1	Test Purpose and Environment	1282

A.6.5.5.5.2	Test Requirements	1286

A.6.5.5.6	Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in DRX mode	1287

A.6.5.5.6.1	Test Purpose and Environment	1287

A.6.5.5.6.2	Test Requirements	1291

A.6.5.6	Active BWP switch	1291

A.6.5.6.1	DCI-based and Timer-based Active BWP Switch	1291

A.6.5.6.1.1	NR FR1- NR FR1 DL active BWP switch of SCell with non-DRX in SA	1291

A.6.5.6.1.2	NR FR1 DL active BWP switch with non-DRX in SA	1297

A.6.5.6.2	RRC-based Active BWP Switch	1301

A.6.5.6.2.1	NR FR1 DL active BWP switch of Cell with non-DRX in SA	1301

A.6.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1305

A.6.5.6.3.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1305

A.6.5.6.3.1.1	Test Purpose and Environment	1305

A.6.5.6.3.1.2	Test Requirements	1310

A.6.5.6.4	SCell dormancy switch	1311

A.6.5.6.4.1	NR FR1 PCell SCell dormancy switch of single FR1 SCell outside active time	1311

A.6.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR1 SCells inside active time	1317

A.6.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1321

A.6.5.6.5.1	NR FR1- NR FR1 DL active BWP switch on multiple CCs with non-DRX in SA	1321

A.6.5.7	DL interruptions at switching between two uplink carriers	1325

A.6.5.7.1	DL interruptions at switching between two uplink carriers in FDD-TDD CA	1325

A.6.5.7.1.1	Test Purpose and Environment	1325

A.6.5.7.1.2	Test Requirements	1329

A.6.5.7.2	DL interruptions at switching between two uplink carriers in TDD-TDD CA	1329

A.6.5.7.2.1	Test Purpose and Environment	1329

A.6.5.7.2.2	Test Requirements	1333

A.6.5.8	UE specific CBW change	1333

A.6.5.8.1	UE specific CBW change on PCell in FR1 in non-DRX	1333

A.6.5.8.1.1	Test Purpose and Environment	1333

A.6.5.8.1.2	Test Requirements	1337

A.6.6	Measurement procedure	1337

A.6.6.1	Intra-frequency Measurements	1337

A.6.6.1.1	SA event triggered reporting tests without gap under non-DRX	1337

A.6.6.1.1.1	Test purpose and Environment	1337

A.6.6.1.1.2	Test parameters	1337

A.6.6.1.1.3	Test Requirements	1341

A.6.6.1.2	SA event triggered reporting tests without gap under DRX	1341

A.6.6.1.2.1	Test purpose and Environment	1341

A.6.6.1.2.2	Test parameters	1341

A.6.6.1.2.3	Test Requirements	1345

A.6.6.1.3	SA event triggered reporting tests with per-UE gaps under non-DRX	1345

A.6.6.1.3.1	Test purpose and Environment	1345

A.6.6.1.3.2	Test parameters	1345

A.6.6.1.3.3	Test Requirements	1349

A.6.6.1.4	SA event triggered reporting tests with per-UE gaps under DRX	1349

A.6.6.1.4.1	Test purpose and Environment	1349

A.6.6.1.4.2	Test parameters	1349

A.6.6.1.4.3	Test Requirements	1353

A.6.6.1.5	SA event triggered reporting tests without gap under non-DRX with SSB index reading	1353

A.6.6.1.5.1	Test purpose and Environment	1353

A.6.6.1.5.2	Test parameters	1353

A.6.6.1.5.3	Test Requirements	1355

A.6.6.1.6	SA event triggered reporting tests with per-UE gaps under non-DRX with SSB index reading	1356

A.6.6.1.6.1	Test purpose and Environment	1356

A.6.6.1.6.2	Test parameters	1356

A.6.6.1.6.3	Test Requirements	1357

A.6.6.1.7	SA event triggered reporting tests under DRX for UE configured with highSpeedMeasFlag-r16	1358

A.6.6.1.7.1	Test purpose and Environment	1358

A.6.6.1.7.2	Test parameters	1358

A.6.6.1.7.3	Test Requirements	1362

A.6.6.2	Inter-frequency Measurements	1362

A.6.6.2.1	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	1362

A.6.6.2.1.1	Test Purpose and Environment	1362

A.6.6.2.1.2	Test Requirements	1366

A.6.6.2.2	SA event triggered reporting tests for FR1 without SSB time index detection when DRX is used	1366

A.6.6.2.2.1	Test Purpose and Environment	1366

A.6.6.2.2.2	Test Requirements	1371

A.6.6.2.3	Void	1372

A.6.6.2.4	Void	1372

A.6.6.2.5	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	1372

A.6.6.2.5.1	Test Purpose and Environment	1372

A.6.6.2.5.2	Test Requirements	1376

A.6.6.2.6	SA event triggered reporting tests for FR1 with SSB time index detection when DRX is used	1376

A.6.6.2.6.1	Test Purpose and Environment	1376

A.6.6.2.6.2	Test Requirements	1381

A.6.6.2.7	Void	1382

A.6.6.2.8	Void	1382

A.6.6.2.9	SA event triggered reporting tests with additional mandatory gap pattern	1382

A.6.6.2.9.1	Test Purpose and Environment	1382

A.6.6.2.9.2	Test Requirements	1385

A.6.6.2.10	SA event triggered reporting tests for FR1 when DRX is used	1385

A.6.6.2.10.1	Test Purpose and Environment	1385

A.6.6.2.10.2	Test Requirements	1389

A.6.6.2.11	SA event triggered reporting tests for FR1 without gap when DRX is not used	1390

A.6.6.2.11.1	Test Purpose and Environment	1390

A.6.6.2.11.2	Test Requirements	1393

A.6.6.3	Inter-RAT Measurements	1393

A.6.6.3.1	SA NR - E-UTRAN event-triggered reporting in non-DRX in FR1	1393

A.6.6.3.1.1	Test Purpose and Environment	1393

A.6.6.3.1.2	Test Requirements	1399

A.6.6.3.2	SA NR - E-UTRAN event-triggered reporting in DRX in FR1	1399

A.6.6.3.2.1	Test Purpose and Environment	1399

A.6.6.3.2.2	Test Requirements	1406

A.6.6.3.3	SA NR - E-UTRAN event-triggered reporting in DRX in FR1 for UE configured with highSpeedMeasFlag-r16	1406

A.6.6.3.3.1	Test Purpose and Environment	1406

A.6.6.3.3.2	Test Requirements	1413

A.6.6.4	L1-RSRP measurement for beam reporting	1413

A.6.6.4.1	SSB based L1-RSRP measurement when DRX is not used	1413

A.6.6.4.1.1	Test Purpose and Environment	1413

A.6.6.4.1.2	Test parameters	1413

A.6.6.4.1.3	Test Requirements	1416

A.6.6.4.2	SSB based L1-RSRP measurement when DRX is used	1416

A.6.6.4.2.1	Test Purpose and Environment	1416

A.6.6.4.2.2	Test parameters	1417

A.6.6.4.2.3	Test Requirements	1420

A.6.6.4.3	CSI-RS based L1-RSRP measurement when DRX is not used	1420

A.6.6.4.3.1	Test Purpose and Environment	1420

A.6.6.4.3.2	Test parameters	1421

A.6.6.4.3.3	Test Requirements	1424

A.6.6.4.4	CSI-RS based L1-RSRP measurement when DRX is used	1424

A.6.6.4.4.1	Test Purpose and Environment	1424

A.6.6.4.4.2	Test parameters	1425

A.6.6.4.4.3	Test Requirements	1428

A.6.6.4.5	SSB based L1-RSRP measurement when DRX is used for UE configured with highSpeedMeasFlag-r16	1428

A.6.6.4.5.1	Test Purpose and Environment	1428

A.6.6.4.5.2	Test parameters	1429

A.6.6.4.5.3	Test Requirements	1432

A.6.6.5	1432

A.6.6.5.1	SA NR - UTRAN FDD event-triggered reporting in non-DRX in FR1	1432

A.6.6.5.1.1	Test Purpose and Environment	1432

A.6.6.5.1.2	Test Requirements	1436

A.6.6.6	CLI measurements	1436

A.6.6.6.1	SRS-RSRP measurement with DRX	1436

A.6.6.6.1.1	Test Purpose and Environment	1436

A.6.6.6.1.2	Test Parameters	1437

A.6.6.6.1.3	Test Requirements	1440

A.6.6.6.2	CLI-RSSI measurement with DRX	1440

A.6.6.6.2.1	Test Purpose and Environment	1440

A.6.6.6.2.2	Test Parameters	1441

A.6.6.6.2.3	Test Requirements	1443

A.6.6.7	NR measurements with autonomous gaps	1443

A.6.6.7.1	SA intra-frequency CGI identification of NR neighbor cell in FR1	1443

A.6.6.7.1.1	Test Purpose and Environment	1443

A.6.6.7.1.2	Test Parameters	1443

A.6.6.7.1.3	Test Requirements	1446

A.6.6.7.2	Identification of a new CGI of inter-RAT E-UTRA cell using autonomous gaps in NR SA	1446

A.6.6.7.2.1	Test Purpose and Environment	1446

A.6.6.7.2.2	Test Requirements	1449

A.6.6.8	L1-SINR measurement for beam reporting	1450

A.6.6.8.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is used	1450

A.6.6.8.1.1	Test Purpose and Environment	1450

A.6.6.8.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is not used	1453

A.6.6.8.2.1	Test Purpose and Environment	1453

A.6.6.8.2.2	Test parameters	1454

A.6.6.8.2.3	Test Requirements	1458

A.6.6.8.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is not used	1458

A.6.6.8.3.1	Test Purpose and Environment	1458

A.6.6.8.3.2	Test parameters	1459

A.6.6.8.3.3	Test Requirements	1462

A.6.6.9	Idle Mode CA/DC Measurements	1462

A.6.6.9.1	SA Idle mode CA/DC measurement for FR1	1462

A.6.6.9.1.1	Test Purpose and Environment	1462

A.6.6.9.1.2	Test Requirements	1469

A.6.6.10	CSI-RS based intra-frequency Measurements	1469

A.6.6.10.1	SA event triggered reporting tests without gap under non-DRX	1469

A.6.6.10.1.1	Test purpose and Environment	1469

A.6.6.10.1.2	Test Requirements	1472

A.6.6.11	CSI-RS based inter-frequency Measurements	1472

A.6.6.11.1	 SA event triggered reporting tests with gap under DRX	1472

A.6.6.11.1.1	Test Purpose and Environment	1472

A.6.6.11.1.2	Test Requirements	1476

A.6.6.12	RSTD measurements	1476

A. 6.6.12.1	NR RSTD measurement reporting delay test case for single positioning frequency layer in FR1 SA	1476

A. 6.6.12.1.1	Test Purpose and Environment	1476

A.6.6.12.1.2	Test Requirements	1482

A. 6.6.12.2	NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR1 SA	1482

A. 6.6.12.2.1	Test Purpose and Environment	1482

A.6.6.12.2.2	Test Requirements	1488

A.6.6.13 PRS-RSRP measurements	1488

A.6.6.13.1	PRS-RSRP reporting delay test case for single positioning frequency layer	1488

A.6.6.13.1.1	Test purpose and Environment	1488

A.6.6.13.1.2	Test Requirements	1492

A.6.6.13.2	PRS-RSRP reporting delay test case for dual positioning frequency layer	1492

A.6.6.13.2.1	Test purpose and Environment	1492

A.6.6.13.2.2	Test Requirements	1496

A.6.6.14	UE Rx-Tx time difference measurements	1496

A.6.6.14.1 UE Rx-Tx time difference measurement for single positioning frequency layer in FR1 SA	1496

A.6.6.14.1.1	Test purpose and environment	1496

A.6.6.14.1.2	Test requirements	1500

A.6.6.14.2 UE Rx-Tx time difference measurement for dual positioning frequency layers in FR1 SA	1500

A.6.6.14.2.1	Test purpose and environment	1500

A.6.6.14.2.2	Test requirements	1504

A.6.6.15	Idle Mode measurements of inter-RAT CA candidate cells for early reporting	1504

A.6.6.15.1	Test Purpose and Environment	1504

A.6.6.15.2	Test Requirements	1511

A.6.7	Measurement Performance requirements	1512

A.6.7.1	SS-RSRP	1512

A.6.7.1.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1512

A.6.7.1.1.1	Test Purpose and Environment	1512

A.6.7.1.1.2	Test parameters	1512

A.6.7.1.1.3	Test Requirements	1517

A.6.7.1.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1517

A.6.7.1.2.1	Test Purpose and Environment	1517

A.6.7.1.2.2	Test parameters	1517

A.6.7.1.2.3	Test Requirements	1521

A.6.7.1.3	Void	1521

A.6.7.2	SS-RSRQ	1521

A.6.7.2.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1521

A.6.7.2.1.1	Test Purpose and Environment	1521

A.6.7.2.1.2	Test Parameters	1522

A.6.7.2.1.3	Test Requirements	1527

A.6.7.2.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1527

A.6.7.2.2.1	Test Purpose and Environment	1527

A.6.7.2.2.2	Test Parameters	1527

A.6.7.2.2.3	Test Requirements	1532

A.6.7.3	SS-SINR	1532

A.6.7.3.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1532

A.6.7.3.1.1	Test Purpose and Environment	1532

A.6.7.3.1.2	Test Parameters	1533

A.6.7.3.1.3	Test Requirements	1537

A.6.7.3.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1537

A.6.7.3.2.1	Test Purpose and Environment	1537

A.6.7.3.2.2	Test Parameters	1537

A.6.7.3.2.3	Test Requirements	1543

A.6.7.4	L1-RSRP measurement for beam reporting	1544

A.6.7.4.1	SSB based L1-RSRP measurement	1544

A.6.7.4.1.1	Test Purpose and Environment	1544

A.6.7.4.1.2	Test parameters	1544

A.6.7.4.1.3	Test Requirements	1548

A.6.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	1548

A.6.7.4.2.1	Test Purpose and Environment	1548

A.6.7.4.2.2	Test parameters	1549

A.6.7.4.2.3	Test Requirements	1553

A.6.7.5	E-UTRAN RSRP	1553

A.6.7.5.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1553

A.6.7.5.1.1	Test Purpose and Environment	1553

A.6.7.5.1.2	Test parameters	1554

A.6.7.5.1.3	Test Requirements	1559

A.6.7.6	E-UTRAN RSRQ	1560

A.6.7.6.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1560

A.6.7.6.1.1	Test Purpose and Environment	1560

A.6.7.6.1.2	Test parameters	1560

A.6.7.6.1.3	Test Requirements	1565

A.6.7.7	E-UTRAN RS-SINR	1566

A.6.7.7.1	SA: inter-RAT measurement accuracy with FR1 serving cell	1566

A.6.7.7.1.1	Test Purpose and Environment	1566

A.6.7.7.1.2	Test parameters	1566

A.6.7.7.1.3	Test Requirements	1571

A.6.7.8	CLI measurements	1572

A.6.7.8.1	SA SRS-RSRP measurement accuracy with FR1 serving cell	1572

A.6.7.8.1.1	Test Purpose and Environment	1572

A.6.7.8.1.2	Test parameters	1572

A.6.7.8.1.3	Test Requirements	1578

A.6.7.8.2	SA CLI-RSSI measurement accuracy with FR1 serving cell	1578

A.6.7.8.2.1	Test Purpose and Environment	1578

A.6.7.8.2.2	Test parameters	1579

A.6.7.8.2.3	Test Requirements	1582

A.6.7.9	L1-SINR measurement for beam reporting	1583

A.6.7.9.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	1583

A.6.7.9.1.1	Test Purpose and Environment	1583

A.6.7.9.1.2	Test parameters	1583

A.6.7.9.1.3	Test Requirements	1587

A.6.7.9.2	L1-SINR measurement with SSB based CMR and dedicated IMR	1587

A.6.7.9.2.1	Test Purpose and Environment	1587

A.6.7.9.2.2	Test parameters	1588

A.6.7.9.2.3	Test Requirements	1593

A.6.7.9.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	1593

A.6.7.9.3.1	Test Purpose and Environment	1593

A.6.7.9.3.2	Test parameters	1593

A.6.7.9.3.3	Test Requirements	1598

A.6.7.10	CSI-RSRP	1598

A.6.7.10.1	SA: intra-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1598

A.6.7.10.1.1	Test Purpose and Environment	1598

A.6.7.10.1.2	Test parameters	1599

A.6.7.10.1.3	Test Requirements	1604

A.6.7.10.2	SA inter-frequency case measurement accuracy with FR1 serving cell and FR1 target cell	1604

A.6.7.10.2.1	Test Purpose and Environment	1604

A.6.7.10.2.2	Test parameters	1604

A.6.7.10.2.3	Test Requirements	1609

A.6.7.11	CSI-RSRQ	1609

A.6.7.11.1	SA: Intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1609

A.6.7.11.1.1	Test Purpose and Environment	1609

A.6.7.11.1.2	Test Parameters	1609

A.6.7.11.1.3	Test Requirements	1614

A.6.7.11.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1614

A.6.7.11.2.1	Test Purpose and Environment	1614

A.6.7.11.2.2	Test Parameters	1614

A.6.7.11.2.3	Test Requirements	1619

A.6.7.12	CSI-SINR	1620

A.6.7.12.1	SA intra-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1620

A.6.7.12.1.1	Test Purpose and Environment	1620

A.6.7.12.1.2	Test Parameters	1620

A.6.7.12.1.3	Test Requirements	1624

A.6.7.12.2	SA Inter-frequency measurement accuracy with FR1 serving cell and FR1 target cell	1625

A.6.7.12.2.1	Test Purpose and Environment	1625

A.6.7.12.2.2	Test Parameters	1625

A.6.7.12.2.3	Test Requirements	1631

A.6.7.13	RSTD measurements	1631

A.6.7.13.1	RSTD measurement accuracy test case for single positioning frequency layer	1631

A.6.7.13.1.1	Test purpose and Environment	1631

A.6.7.13.1.2	Test Requirements	1634

A.6.7.13.2	RSTD measurement accuracy test case for dual positioning frequency layer	1634

A.6.7.13.2.1	Test purpose and Environment	1634

A.6.7.13.2.2	Test Requirements	1638

A.6.7.14	PRS-RSRP measurements	1638

A.6.7.14.1	SA: measurement accuracy with PRS in FR1	1638

A.6.7.14.1.1	Test Purpose and Environment	1638

A.6.7.14.1.2	Test parameters	1638

A.6.7.14.1.3	Test Requirements	1644

A.6.7.15	UE Rx-Tx time difference measurements	1644

A.6.7.15.1 UE Rx-Tx time difference measurement accuracy for single positioning frequency layer in FR1 SA	1644

A.6.7.15.1.1	Test purpose and environment	1644

A.6.7.15.1.2	Test parameters	1645

A.6.7.15.1.3	Test requirements	1648

A.7.1	SA: RRC_IDLE state mobility	1648

A.7.1.1	Cell re-selection to NR	1648

A.7.1.1.1	Cell reselection to FR2 intra-frequency NR case	1648

A.7.1.1.1.1	Test Purpose and Environment	1648

A.7.1.1.1.2	Test Parameters	1648

A.7.1.1.1.3	Test Requirements	1652

A.7.1.1.2	Cell reselection to FR2 inter-frequency NR case	1652

A.7.1.1.2.1	Test Purpose and Environment	1652

A.7.1.1.2.2	Test Parameters	1652

A.7.1.1.2.3	Test Requirements	1656

A.7.1.1.3	Cell reselection to FR2 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1656

A.7.1.1.3.1	Test Purpose and Environment	1656

A.7.1.1.3.2	Test Parameters	1656

A.7.1.1.3.3	Test Requirements	1659

A.7.1.1.4	Cell reselection to FR2 intra-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1659

A.7.1.1.4.1	Test Purpose and Environment	1659

A.7.1.1.4.2	Test Parameters	1659

A.7.1.1.4.3	Test Requirements	1662

A.7.1.1.5	Cell reselection to FR2 inter-frequency NR case for UE fulfilling low mobility relaxed measurement criterion	1662

A.7.1.1.5.1	Test Purpose and Environment	1662

A.7.1.1.5.2	Test Parameters	1662

A.7.1.1.5.3	Test Requirements	1666

A.7.1.1.6	Cell reselection to FR2 inter-frequency NR case for UE fulfilling not-at-cell edge relaxed measurement criterion	1666

A.7.1.1.6.1	Test Purpose and Environment	1666

A.7.1.1.6.2	Test Parameters	1666

A.7.1.1.6.3	Test Requirements	1668

A.7.2	SA: RRC_INACTIVE state mobility	1669

A.7.3	RRC_CONNECTED state mobility	1669

A.7.3.1	Handover	1669

A.7.3.1.1	Inter-frequency handover from FR1 to FR2; unknown target cell	1669

A.7.3.1.1.1	Test Purpose and Environment	1669

A.7.3.1.1.2	Test Parameters	1669

A.7.3.1.1.3	Test Requirements	1673

A.7.3.1.2	Intra-frequency handover from FR2 to FR2; unknown target cell	1673

A.7.3.1.2.1	Test Purpose and Environment	1673

A.7.3.1.2.2	Test Parameters	1673

A.7.3.1.2.3	Test Requirements	1676

A.7.3.1.3	Inter-frequency handover from FR2 to FR2; unknown target cell	1676

A.7.3.1.3.1	Test Purpose and Environment	1676

A.7.3.1.3.2	Test Parameters	1676

A.7.3.1.3.3 Test Requirements	1677

A.7.3.1.4	Inter-band inter-frequency synchronous DAPS handover from FR1 to FR2	1678

A.7.3.1.4.1	Test Purpose and Environment	1678

A.7.3.1.4.2	Test Parameters	1678

A.7.3.1.4.3 Test Requirements	1685

A.7.3.1.5	Inter-band inter-frequency asynchronous DAPS handover from FR1 to FR2	1685

A.7.3.1.5.1	Test Purpose and Environment	1685

A.7.3.1.5.2	Test Parameters	1685

A.7.3.1.5.3 Test Requirements	1692

A.7.3.2	RRC Connection Mobility Control	1692

A.7.3.2.1	SA: RRC Re-establishment	1692

A.7.3.2.1.1	Intra-frequency RRC Re-establishment in FR2	1692

A.7.3.2.1.2	Inter-frequency RRC Re-establishment in FR2	1695

A.7.3.2.1.3	Intra-frequency RRC Re-establishment in FR2 without serving cell timing	1698

A.7.3.2.1.3.1	Test Purpose and Environment	1698

A.7.3.2.1.3.2	Test Requirements	1700

A.7.3.2.2	Random Access	1701

A.7.3.2.2.1	4-step RA type contention based random access test in FR2 for NR Standalone	1701

A.7.3.2.2.2	4-step RA type non-contention based random access test in FR2 for NR Standalone	1705

A.7.3.2.2.3	2-step RA type contention based random access test in FR2 for NR Standalone	1710

A.7.3.2.2.4	2-step RA type non-contention based random access test in FR2 for NR Standalone	1713

A.7.3.2.3	SA: RRC Connection Release with Redirection	1716

A.7.3.2.3.1	Redirection from NR in FR2 to NR in FR2	1716

A.7.3.3	Conditional Handover	1720

A.7.3.3.1	Intra-frequency conditional handover from FR2 to FR2	1720

A.7.3.3.1.1	Test Purpose and Environment	1720

A.7.3.3.1.2	Test Parameters	1720

A.7.3.3.1.2.3	Test Requirements	1723

A.7.3.3.2	Inter-frequency conditional handover from FR2 to FR2; unknown target cell	1723

A.7.3.3.2.1	Test Purpose and Environment	1723

A.7.3.3.2.2	Test Parameters	1723

A.7.3.3.2.3 Test Requirements	1725

A.7.4	Timing	1725

A.7.4.1	UE transmit timing	1725

A.7.4.1.1	NR UE Transmit Timing Test for FR2	1725

A.7.4.1.1.1	Test Purpose and environment	1725

A.7.4.1.1.2	Test requirements	1728

A.7.4.2	UE timer accuracy	1728

A.7.4.3	Timing advance	1728

A.7.4.3.1	SA FR2 timing advance adjustment accuracy	1728

A.7.4.3.1.1	Test Purpose and Environment	1728

A.7.4.3.1.2	Test Parameters	1728

A.7.4.3.1.3 Test Requirements	1732

A.7.5	Signaling characteristics	1732

A.7.5.1	Radio link Monitoring	1732

A.7.5.1.1	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1732

A.7.5.1.1.1	Test Purpose and Environment	1732

A.7.5.1.1.2	Test Requirements	1735

A.7.5.1.2	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in non-DRX mode	1736

A.7.5.1.2.1	Test Purpose and Environment	1736

A.7.5.1.2.2	Test Requirements	1740

A.7.5.1.3	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1741

A.7.5.1.3.1	Test Purpose and Environment	1741

A.7.5.1.3.2	Test Requirements	1744

A.7.5.1.4	Radio Link Monitoring In-sync Test for FR2 PCell configured with SSB-based RLM RS in DRX mode	1744

A.7.5.1.4.1	Test Purpose and Environment	1744

A.7.5.1.4.2	Test Requirements	1748

A.7.5.1.5	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1748

A.7.5.1.5.1	Test Purpose and Environment	1748

A.7.5.1.5.2	Test Requirements	1753

A.7.5.1.6	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in non-DRX mode	1753

A.7.5.1.6.1	Test Purpose and Environment	1753

A.7.5.1.6.2	Test Requirements	1757

A.7.5.1.7	Radio Link Monitoring Out-of-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1757

A.7.5.1.7.1	Test Purpose and Environment	1757

A.7.5.1.7.2	Test Requirements	1761

A.7.5.1.8	Radio Link Monitoring In-sync Test for FR2 PCell configured with CSI-RS-based RLM in DRX mode	1761

A.7.5.1.8.1	Test Purpose and Environment	1761

A.7.5.1.8.2	Test Requirements	1766

A.7.5.1.9	UE Radio Link Monitoring Scheduling Restrictions on FR2	1766

A.7.5.1.9.1	Test Purpose and Environment	1766

A.7.5.1.9.2	Test Requirements	1769

A.7.5.2	Interruption	1769

A.7.5.2.1	Interruptions during measurements on deactivated NR SCC in FR2	1769

A.7.5.2.2	SA interruptions at NR SRS carrier-based switching	1773

A.7.5.2.2.1	Test Purpose and Environment	1773

A.7.5.2.2.2	Test Parameters	1773

A.7.5.2.2.3	Test Requirements	1775

A.7.5.3	SCell Activation and Deactivation Delay	1775

A.7.5.3.1	SCell Activation and deactivation for SCell in FR2 intra-band in non-DRX	1775

A.7.5.3.1.1	Test Purpose and Environment	1775

A.7.5.3.1.2	Test Requirements	1777

A.7.5.3.2	SCell Activation and deactivation for FR1+FR2 inter-band with target SCell in FR2	1777

A.7.5.3.2.1	Test Purpose and Environment	1777

A.7.5.3.2.2	Test Requirements	1781

A.7.5.3.3	SCell Activation and deactivation for SCell in FR2 inter-band in non-DRX	1782

A.7.5.3.3.1	Test Purpose and Environment	1782

A.7.5.3.3.2	Test Requirements	1785

A.7.5.3.4	Direct SCell activation at SCell addition of known SCell in FR2	1786

A.7.5.3.4.1	Test Purpose and Environment	1786

A.7.5.3.4.2	Test Requirements	1789

A.7.5.3.5	Direct SCell activation at handover with known SCell in FR2	1790

A.7.5.3.5.1	Test Purpose and Environment	1790

A.7.5.3.5.2	Test Requirements	1792

A.7.5.4	Void	1793

A.7.5.5	Beam Failure Detection and Link recovery procedures	1793

A.7.5.5.1	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1793

A.7.5.5.1.1	Test Purpose and Environment	1793

A.7.5.5.1.2	Test Requirements	1798

A.7.5.5.2	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with SSB-based BFD and LR in DRX mode	1798

A.7.5.5.2.1	Test Purpose and Environment	1798

A.7.5.5.2.2	Test Requirements	1803

A.7.5.5.3	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode	1803

A.7.5.5.3.1	Test Purpose and Environment	1803

A.7.5.5.3.2	Test Requirements	1809

A.7.5.5.4	Beam Failure Detection and Link Recovery Test for FR2 PCell configured with CSI-RS-based BFD and LR in DRX mode	1809

A.7.5.5.4.1	Test Purpose and Environment	1809

A.7.5.5.4.2	Test Requirements	1815

A.7.5.5.5	Scheduling availability restriction during Beam Failure Detection and Link Recovery for FR2 PCell configured with SSB-based BFD and LR in non-DRX mode	1815

A.7.5.5.5.1	Test Purpose and Environment	1815

A.7.5.5.5.2	Test Requirements	1820

A.7.5.5.6	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in non-DRX mode	1821

A.7.5.5.6.1	Test Purpose and Environment	1821

A.7.5.5.6.2	Test Requirements	1825

A.7.5.5.7	Beam Failure Detection and Link Recovery Test for FR2 SCell configured with CSI-RS-based BFD and LR in DRX mode	1825

A.7.5.5.7.1	Test Purpose and Environment	1825

A.7.5.5.7.2	Test Requirements	1830

A.7.5.6	Active BWP switch	1830

A.7.5.6.1	DCI-based and Timer-based Active BWP Switch	1830

A.7.5.6.1.1	NR FR2- NR FR2 DL active BWP switch of SCell with non-DRX in SA	1830

A.7.5.6.1.2	NR FR1- NR FR2 DL active BWP switch of SCell with non-DRX in SA A.7.5.6.1.2.1 Test Purpose and Environment	1835

A.7.5.6.1.3	NR FR2 DL active BWP switch with non-DRX in SA	1840

A.7.5.6.1.3.1	Test Purpose and Environment	1840

A.7.5.6.1.3.2	Test Requirements	1843

A.7.5.6.2	RRC-based Active BWP Switch	1843

A.7.5.6.3	Simultaneous DCI-based and Timer-based Active BWP Switch on multiple CCs	1847

A.7.5.6.3.1	Active BWP switch on multiple SCells with non-DRX in SA	1847

A.7.5.6.4	SCell dormancy switch	1850

A.7.5.6.4.1	NR FR2 PCell SCell dormancy switch of single FR2 SCell inside active time	1850

A.7.5.6.4.2	NR FR1 PCell SCell dormancy switch of two FR2 SCells outside active time	1855

A.7.5.6.4.2.2	 Test Requirements	1860

A.7.5.6.5	Simultaneous RRC-based Active BWP Switch on multiple CCs	1860

A.7.5.6.5.1	Active BWP switch on multiple SCells with non-DRX in SA	1860

A.7.5.7	PSCell addition and release delay	1865

A.7.5.7.1	Addition and Release Delay of known NR PSCell	1865

A.7.5.7.1.1	Test Purpose and Environment	1865

A.7.5.7.2	Addition and Release Delay of unknown NR PSCell	1868

A.7.5.7.2.1	Test Purpose and Environment	1868

A.7.5.7.2.2	Test Requirements	1871

A.7.5.8	Active TCI state switch delay	1871

A.7.5.8.1	MAC-CE based active TCI state switch	1871

A.7.5.8.2	RRC based active TCI state switch	1874

A.7.5.9	Uplink spatial relation switch delay	1877

A.7.5.9.1	MAC-CE based Spatial Relation switch	1877

A.7.5.9.1.1	 NR PCell FR2 spatial relation associated with known DL-RS	1877

A.7.5.9.1.1.2	Test Requirements	1880

A.7.5.9.2	RRC based spatial relation switch	1880

A.7.5.9.2.1	NR PCell FR2 spatial relation switch associated with a known DL-RS	1880

A.7.5.9.2.1.1	Test Purpose and Environment	1880

A.7.5.9.2.1.2	Test Requirements	1883

A.7.5.10	UE specific CBW change	1883

A.7.5.10.1	NR FR2 UE specific CBW change of PCell with non-DRX in SA	1883

A.7.5.10.1.1	Test Purpose and Environment	1883

A.7.5.10.1.2	Test Requirements	1886

A.7.6	Measurement procedure	1887

A.7.6.1	Intra-frequency Measurements	1887

A.7.6.1.1	SA event triggered reporting test without gap under non-DRX	1887

A.7.6.1.1.1	Test purpose and Environment	1887

A.7.6.1.1.2	Test Requirements	1890

A.7.6.1.2	SA event triggered reporting test without gap under DRX	1890

A.7.6.1.2.1	Test purpose and Environment	1890

A.7.6.1.2.2	Test Requirements	1893

A.7.6.1.3	SA event triggered reporting test with per-UE gaps under non-DRX	1894

A.7.6.1.3.1	Test purpose and Environment	1894

A.7.6.1.3.2	Test Requirements	1897

A.7.6.1.4	SA event triggered reporting test with per-UE gaps under DRX	1897

A.7.6.1.4.1	Test purpose and Environment	1897

A.7.6.1.4.2	Test Requirements	1900

A.7.6.2	Inter-frequency Measurements	1901

A.7.6.2.1	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2)	1901

A.7.6.2.1.1	Test Purpose and Environment	1901

A.7.6.2.1.2	Test Requirements	1904

A.7.6.2.2	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is used (PCell in FR2)	1904

A.7.6.2.2.1	Test Purpose and Environment	1904

A.7.6.2.2.2	Test Requirements	1908

A.7.6.2.3	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is not used (PCell in FR2)	1908

A.7.6.2.3.1	Test Purpose and Environment	1908

A.7.6.2.3.2	Test Requirements	1912

A.7.6.2.4	SA event triggered reporting tests For FR2 with SSB time index detection when DRX is used (PCell in FR2)	1912

A.7.6.2.4.1	Test Purpose and Environment	1912

A.7.6.2.4.2	Test Requirements	1916

A.7.6.2.5	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (PCell in FR1)	1916

A.7.6.2.5.1	Test Purpose and Environment	1916

A.7.6.2.5.2	Test Requirements	1920

A.7.6.2.6	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is used (PCell in FR1)	1921

A.7.6.2.6.1	Test Purpose and Environment	1921

A.7.6.2.6.2	Test Requirements	1925

A.7.6.2.7	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is not used (PCell in FR1)	1926

A.7.6.2.7.1	Test Purpose and Environment	1926

A.7.6.2.7.2	Test Requirements	1930

A.7.6.2.8	SA event triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR1)	1931

A.7.6.2.8.1	Test Purpose and Environment	1931

A.7.6.2.8.2	Test Requirements	1935

A.7.6.2.9	SA event triggered reporting tests For FR2 without SSB time index detection when DRX is not used (PCell in FR2) (rel16 additional mandatory gap pattern 17)	1936

A.7.6.2.9.1	Test Purpose and Environment	1936

A.7.6.2.9.2	Test Requirements	1939

A.7.6.2.10	SA event triggered reporting test without gap under non-DRX	1939

A.7.6.2.10.1	Test Purpose and Environment	1939

A.7.6.2.10.2	Test Requirements	1941

A.7.6.2.11	SA event triggered reporting test without gap under DRX	1941

A.7.6.2.11.1	Test Purpose and Environment	1941

A.7.6.2.11.2	Test Requirements	1944

A.7.6.3	L1-RSRP measurement for beam reporting	1945

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1945

A.7.6.3.1	SSB based L1-RSRP measurement when DRX is not used	1945

A.7.6.3.1.1	Test Purpose and Environment	1945

A.7.6.3.1.2	Test parameters	1945

A.7.6.3.1.3	Test Requirements	1947

A.7.6.3.2	SSB based L1-RSRP measurement when DRX is used	1947

A.7.6.3.2.1	Test Purpose and Environment	1947

A.7.6.3.2.2	Test parameters	1948

A.7.6.3.2.3	Test Requirements	1950

A.7.6.3.3	CSI-RS based L1-RSRP measurement when DRX is not used	1950

A.7.6.3.3.1	Test Purpose and Environment	1950

A.7.6.3.3.2	Test parameters	1951

A.7.6.3.3.3	Test Requirements	1953

A.7.6.3.4	CSI-RS based L1-RSRP measurement when DRX is used	1954

A.7.6.3.4.1	Test Purpose and Environment	1954

A.7.6.3.4.2	Test parameters	1954

A.7.6.3.3.3	Test Requirements	1956

A.7.6.4	CLI measurements	1957

A.7.6.4.1	SRS-RSRP measurement with non-DRX	1957

A.7.6.4.1.1	Test Purpose and Environment	1957

A.7.6.4.1.2	Test Parameters	1957

A.7.6.4.1.3	Test Requirements	1959

A.7.6.4.2	CLI-RSSI measurement with non-DRX	1959

A.7.6.4.2.1	Test Purpose and Environment	1959

A.7.6.4.2.2	Test Parameters	1960

A.7.6.4.2.3	Test Requirements	1961

A.7.6.5	NR Measurements with autonomous gaps	1962

A.7.6.5.1	SA interfrequency CGI reporting in autonomous gaps test (PCell in FR2)	1962

A.7.6.5.1.1	Test Purpose and Environment	1962

A.7.6.5.1.2	Test Requirements	1965

A.7.6.6	L1-SINR measurement for beam reporting	1965

A.7.6.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured when DRX is not used	1965

A.7.6.6.1.1	Test Purpose and Environment	1965

A.7.6.6.1.2	Test parameters	1965

A.7.6.6.1.3	Test Requirements	1967

A.7.6.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR when DRX is used	1967

A.7.6.6.2.1	Test Purpose and Environment	1967

A.7.6.6.2.2	Test parameters	1968

A.7.6.6.2.3	Test Requirements	1970

A.7.6.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR configured when DRX is used	1971

A.7.6.6.3.1	Test Purpose and Environment	1971

A.7.6.6.3.2	Test parameters	1971

A.7.6.6.3.3	Test Requirements	1973

A.7.6.7	CSI-RS based intra-frequency Measurements	1973

A.7.6.7.1	SA event triggered reporting test without gap under DRX for CSI-RS based intra-frequency measurement	1973

A.7.6.7.1.1	Test purpose and Environment	1973

A.7.6.7.1.2	Test Requirements	1976

A.7.6.8	CSI-RS based inter-frequency Measurements	1976

A.7.6.8.1	SA event triggered reporting tests for FR2 CSI-RS based measurement when non-DRX is used (PCell in FR2)	1976

A.7.6.8.1.1	Test Purpose and Environment	1976

A.7.6.2.2.2	Test Requirements	1980

A.7.6.9	RSTD measurements	1980

A.7.6.9.1	 NR RSTD measurement reporting delay test case for single positioning frequency layer in FR2 SA	1980

A.7.6.9.1.1	Test Purpose and Environment	1980

A.7.6.9.1.2	Test Requirements	1984

A.7.6.9.2	 NR RSTD measurement reporting delay test case for dual positioning frequency layers in FR2 SA	1985

A.7.6.9.2.1	Test Purpose and Environment	1985

A.7.6.9.2.2	Test Requirements	1990

A.7.6.10 PRS-RSRP measurements	1991

A.7.6.10.1 PRS-RSRP reporting delay test case for single positioning frequency layer	1991

A.7.6.10.1.1	Test Purpose and Environment	1991

A.7.6.10.1.2	Test Requirements	1995

A.7.6.10.2 PRS-RSRP reporting delay test case for dual positioning frequency layer	1995

A.7.6.10.2.1	Test Purpose and Environment	1995

A.7.6.10.2.2	Test Requirements	1999

A.7.6.11	UE Rx-Tx time difference measurements	1999

A.7.6.11.1 UE Rx-Tx time difference measurements for single positioning frequency layer in FR2 SA	1999

A.7.6.11.1.1	Test purpose and environment	1999

A.7.6.11.1.2	Test requirements	2001

A.7.6.11.2 UE Rx-Tx time difference measurement period for dual positioning frequency layers in FR2 SA	2002

A.7.6.11.2.1	Test purpose and environment	2002

A.7.6.11.2.2	Test requirements	2004

A.7.7	Measurement Performance requirements	2005

A.7.7.1	SS-RSRP	2005

A.7.7.1.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2005

A.7.7.1.1.1	Test Purpose and Environment	2005

A.7.7.1.1.2	Test parameters	2005

A.7.7.1.1.3	Test Requirements	2009

A.7.7.1.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2010

A.7.7.1.2.1	Test Purpose and Environment	2010

A.7.7.1.2.2	Test parameters	2010

A.7.7.1.2.3	Test Requirements	2014

A.7.7.1.3	SA inter-frequency measurement accuracy with FR1 serving cell and FR2 target cell	2015

A.7.7.1.3.1	Test Purpose and Environment	2015

A.7.7.1.3.2	Test parameters	2015

A.7.7.1.3.3	Test Requirements	2019

A.7.7.2	SS-RSRQ	2019

A.7.7.2.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2019

A.7.7.2.1.1	Test Purpose and Environment	2019

A.7.7.2.1.2	Test Parameters	2019

A.7.7.2.1.3	Test Requirements	2021

A.7.7.2.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2021

A.7.7.2.2.1	Test Purpose and Environment	2021

A.7.7.2.2.2	Test Parameters	2021

A.7.7.2.2.3	Test Requirements	2023

A.7.7.3	SS-SINR	2023

A.7.7.3.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2023

A.7.7.3.1.1	Test Purpose and Environment	2023

A.7.7.3.1.2	Test Parameters	2023

A.7.7.3.1.3	Test Requirements	2026

A.7.7.3.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2026

A.7.7.3.2.1	Test Purpose and Environment	2026

A.7.7.3.2.2	Test Parameters	2026

A.7.7.3.2.3	Test Requirements	2028

A.7.7.4	L1-RSRP measurement for beam reporting	2028

A.7.7.4.1	SSB based L1-RSRP measurement	2028

A.7.7.4.1.1	Test Purpose and Environment	2028

A.7.7.4.1.2	Test parameters	2029

A.7.7.4.1.3	Test Requirements	2031

A.7.7.4.2	CSI-RS based L1-RSRP measurement on resource set with repetition off	2032

A.7.7.4.2.1	Test Purpose and Environment	2032

A.7.7.4.2.2	Test parameters	2032

A.7.7.4.2.3	Test Requirements	2034

A.7.7.5	CLI measurements	2035

A.7.7.5.1	SA SRS-RSRP measurement accuracy with FR2 serving cell	2035

A.7.7.5.1.1	Test Purpose and Environment	2035

A.7.7.5.1.2	Test parameters	2035

A.7.7.5.1.3	Test Requirements	2038

A.7.7.5.2	SA CLI-RSSI measurement accuracy with FR2 serving cell	2039

A.7.7.5.2.1	Test Purpose and Environment	2039

A.7.7.5.2.2	Test parameters	2039

A.7.7.5.2.3	Test Requirements	2041

A.7.7.6	L1-SINR measurement for beam reporting	2042

A.7.7.6.1	L1-SINR measurement with CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off	2042

A.7.7.6.1.1	Test Purpose and Environment	2042

A.7.7.6.1.2	Test parameters	2042

A.7.7.6.1.3	Test Requirements	2044

A.7.7.6.2	L1-SINR measurement with SSB based CMR and dedicated IMR	2044

A.7.7.6.2.1	Test Purpose and Environment	2044

A.7.7.6.2.2	Test parameters	2045

A.7.7.6.2.3	Test Requirements	2047

A.7.7.6.3	L1-SINR measurement with CSI-RS based CMR and dedicated IMR	2047

A.7.7.6.3.1	Test Purpose and Environment	2047

A.7.7.6.3.2	Test parameters	2048

A.7.7.6.3.3	Test Requirements	2050

A.7.7.7	CSI-RSRP	2050

A.7.7.7.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2050

A.7.7.7.1.1	Test Purpose and Environment	2050

A.7.7.7.1.2	Test parameters	2051

A.7.7.7.1.3	Test Requirements	2054

A.7.7.7.2	SA inter-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2055

A.7.7.7.2.1	Test Purpose and Environment	2055

A.7.7.7.2.2	Test parameters	2055

A.7.7.7.2.3	Test Requirements	2058

A.7.7.8	CSI-RSRQ	2058

A.7.7.8.1	SA intra-frequency measurement accuracy with FR2 serving cell and FR2 target cell	2058

A.7.7.8.1.1	Test Purpose and Environment	2058

A.7.7.8.1.2	Test Parameters	2059

A.7.7.8.1.3	Test Requirements	2061

A.7.7.8.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2061

A.7.7.8.2.1	Test Purpose and Environment	2061

A.7.7.8.2.2	Test Parameters	2061

A.7.7.8.2.3	Test Requirements	2063

A.7.7.9	CSI-SINR	2063

A.7.7.9.1	SA intra-frequency case measurement accuracy with FR2 serving cell and FR2 target cell	2063

A.7.7.9.1.1	Test Purpose and Environment	2063

A.7.7.9.1.2	Test Parameters	2063

A.7.7.9.1.3	Test Requirements	2066

A.7.7.9.2	SA Inter-frequency measurement accuracy with FR2 serving cell and FR2 TDD target cell	2066

A.7.7.9.2.2	Test Parameters	2066

A.7.7.9.2.3	Test Requirements	2068

A.7.7.10	RSTD measurements	2068

A.7.7.10.1	RSTD measurement accuracy test case for single positioning frequency layer	2068

A.7.7.10.1.1	Test purpose and Environment	2068

A.7.7.10.1.2	Test Requirements	2071

A.7.7.10.2	RSTD measurement accuracy test case for dual positioning frequency layer	2071

A.7.7.10.2.1	Test purpose and Environment	2071

A.7.7.10.2.2	Test Requirements	2073

A.7.7.11	PRS-RSRP measurements	2073

A.7.7.11.1	SA measurement accuracy with PRS in FR2	2073

A.7.7.11.1.1	Test Purpose and Environment	2073

A.7.7.11.1.2	Test parameters	2073

A.7.7.11.1.3	Test Requirements	2076

A.7.7.12	UE Rx-Tx time difference measurements	2077

A.7.7.12.1 UE Rx-Tx time difference measurement period for single positioning frequency layer in FR2 SA	2077

A.7.7.12.1.1	Test purpose and environment	2077

A.7.7.12.1.2	Test parameters	2077

A.7.7.12.1.3	Test requirements	2079

A.8	E-UTRA standalone tests for NR RRM	2079

A.8.1	Void	2079

A.8.2	RRC_IDLE state mobility	2079

A.8.2.1	Inter-RAT NR Cell re-selection	2079

A.8.2.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1	2079

A.8.2.1.1.1	Test Purpose and Environment	2079

A.8.2.1.1.2	Test Requirements	2084

A.8.2.1.2	E-UTRA Cell reselection to lower priority NR target Cell in FR1 for UE configured with highSpeedInterRAT-NR-r16	2085

A.8.2.1.2.1	Test Purpose and Environment	2085

A.8.2.1.2.2	Test Requirements	2090

A.8.2.2	E-UTRA – NR Inter-RAT Early Measruement Reporting	2091

A.8.2.2.1	E-UTRA – NR Early Measruement Reporting for NR in FR1	2091

A.8.2.2.1.1	Test Purpose and Environment	2091

A.8.2.2.1.2	Test Requirements	2096

A.8.3	RRC_CONNECTED state mobility	2096

A.8.3.1	Handover	2096

A.8.3.1.1	E-UTRAN - NR handover in FR1	2096

A.8.3.1.1.1	Test Purpose and Environment	2096

A.8.3.1.1.2	Test Requirements	2101

A.8.4	Measurement procedure	2101

A.8.4.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay	2101

A.8.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay in non-DRX	2101

A.8.4.1.1.1	Test Purpose and Environment	2101

A.8.4.1.1.2	Test Requirements	2103

A.8.4.1.2	E-UTRA – NR Inter-RAT SFTD Measurement Delay in DRX	2104

A.8.4.1.2.1	Test Purpose and Environment	2104

A.8.4.1.2.2	Test Requirements	2105

A.8.4.2	E-UTRA – NR Inter-RAT Measurements	2105

A.8.4.2.1	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2105

A.8.4.2.1.1	Test Purpose and Environment	2105

A.8.4.2.1.2	Test Requirements	2110

A.8.4.2.2	NR Inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2110

A.8.4.2.2.1	Test Purpose and Environment	2110

A.8.4.2.2.2	Test Requirements	2114

A.8.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2114

A.8.4.2.3.1	Test Purpose and Environment	2114

A.8.4.2.3.2	Test Requirements	2119

A.8.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2119

A.8.4.2.4.1	Test Purpose and Environment	2119

A.8.4.2.4.2	Test Requirements	2123

A.8.4.2.5	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is not used	2123

A.8.4.2.5.1	Test Purpose and Environment	2123

A.8.4.2.5.2	Test Requirements	2125

A.8.4.2.6	NR Inter-RAT event triggered reporting tests for FR2 without SSB time index detection when DRX is used	2126

A.8.4.2.6.1	Test Purpose and Environment	2126

A.8.4.2.6.2	Test Requirements	2128

A.8.4.2.7	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is not used	2129

A.8.4.2.7.1	Test Purpose and Environment	2129

A.8.4.2.7.2	Test Requirements	2131

A.8.4.2.8	NR Inter-RAT event triggered reporting tests for FR2 with SSB time index detection when DRX is used	2132

A.8.4.2.8.1	Test Purpose and Environment	2132

A.8.4.2.8.2	Test Requirements	2134

A.8.4.2.9	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection in DRX for UE configured with highSpeedInterRAT-NR-r16	2135

A.8.4.2.9.1	Test Purpose and Environment	2135

A.8.4.2.9.2	Test Requirements	2141

A.8.5	Measurement performance	2141

A.8.5.1	SFTD accuracy	2141

A.8.5.1.1	SFTD accuracy	2141

A.8.5.1.1.1	Test Purpose	2141

A.8.5.1.1.2	Test Environment	2141

A.8.5.1.1.3	Test Requirements	2147

A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements	2147

A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2147

A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2152

A.8.5.2.1.2.1	Test Purpose and Environment	2152

A.8.5.2.1.2.2	Test Parameters	2152

A.8.5.2.1.2.3	Test Requirements	2154

A.8.5.2.2	SS-RSRQ	2154

A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2154

A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2159

A.8.5.2.2.2.1	Test Purpose and Environment	2159

A.8.5.2.2.2.2	Test Parameters	2159

A.8.5.2.2.2.3	Test Requirements	2161

A.8.5.2.3	SS-SINR	2161

A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell	2161

A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell	2165

A.8.5.2.3.2.1	Test Purpose and Environment	2165

A.8.5.2.3.2.2	Test Parameters	2165

A.8.5.2.3.2.3	Test Requirements	2167

A.9	V2X Tests	2168

A.9.1	V2X Tests in FR1	2168

A.9.1.1	Test for V2X UE Transmit Timing	2168

A.9.1.1.1	Test for GNSS as Synchronization Reference Source	2168

A.9.1.1.1.1	Test Purpose and Environment	2168

A.9.1.1.1.2	Test requirements	2168

A.9.1.1.2	Test for SyncRef UE as Synchronization Reference Source	2168

A.9.1.1.2.1	Test Purpose and Environment	2168

A.9.1.1.2.2	Test requirements	2169

A.9.1.1.3	Test for FR1 NR Cell as Synchronization Reference Source	2169

A.9.1.1.3.1	Test Purpose and Environment	2169

A.9.1.1.3.2	Test requirements	2172

A.9.1.2	Test for Initiation/Cease of SLSS Transmission with V2X Sidelink Communication	2172

A.9.1.2.1	Test for FR1 NR Cell as synchronization reference source without gap under non-DRX	2172

A.9.1.2.1.1	Test Purpose and Environment	2172

A.9.1.2.1.2	Test Requirements	2176

A.9.1.2.2	Test for SyncRef UE as synchronization reference source	2176

A.9.1.2.2.1	Test Purpose and Environment	2176

A.9.1.2.2.2	Test Requirements	2177

A.9.1.3	Test for V2X Synchronization Reference Selection/Reselection	2178

A.9.1.3.1	Test for GNSS configured as the highest priority	2178

A.9.1.3.1.1	Test Purpose and Environment	2178

A.9.1.3.1.2	Test Requirements	2180

A.9.1.3.2	 Test for FR1 NR Cell configured as the highest priority	2181

A.9.1.3.2.1	Test Purpose and Environment	2181

A.9.1.3.2.2	Test Requirements	2184

A.9.1.4	Test for L1 SL-RSRP Measurement	2185

A.9.1.4.1	Test for V2X UE Autonomous Resource Selection/Reselection	2185

A.9.1.4.1.1	Test Purpose and Environment	2185

A.9.1.4.1.2	Test Requirements	2187

A.9.1.4.2	Test for V2X UE Resource Pre-emption	2187

A.9.1.4.2.1	Test Purpose and Environment	2187

A.9.1.4.2.2	Test Requirements	2191

A.9.1.4.3	Test for V2X UE Resource Re-evaluation	2191

A.9.1.4.3.1	Test Purpose and Environment	2191

A.9.1.4.3.2	Test Requirements	2196

A.9.1.5	Test for Congestion Control Measurement	2196

A.9.1.5.1	Test Purpose and Environment	2196

A.9.1.5.2	Test Requirements	2198

A.9.1.6	Test for Interruption	2198

A.9.1.6.1	Test for Interruption to WAN due to V2X Sidelink Communication	2198

A.9.1.6.1.1	Test Purpose and Environment	2198

A.9.1.6.1.2	Test Requirements	2202

A.10	EN-DC Tests with NR PSCell under CCA and Other NR Cells in FR1	2202

A.10.1	RRC_CONNECTED state mobility	2202

A.10.1.1	RRC connection mobility control	2202

A.10.1.1.1	Random Access	2202

A.10.1.1.1.1	4-step RA type contention-based random access for NR PSCell with CCA	2202

A.10.1.1.1.1.1	Test Purpose and Environment	2202

A.10.1.1.1.1.2	Test Requirements	2205

A.10.1.1.1.1.2.1	Random Access Preamble Transmission	2205

A.10.1.1.1.1.2.2	Random Access Response Reception	2205

A.10.1.1.1.1.2.3	No Random Access Response Reception	2206

A.10.1.1.1.1.2.4	Receiving an UL grant for msg3 retransmission	2206

A.10.1.1.1.1.2.5	 Contention Resolution Timer expiry	2206

A.10.1.1.1.2	4-step RA type non-contention based random access for NR PSCell with CCA	2207

A.10.1.1.1.2.1	Test Purpose and Environment	2207

A.10.1.1.1.2.2	Test Requirements	2210

A.10.1.1.1.2.2.1	SSB-based Random Access Preamble Transmission	2210

A.10.1.1.1.2.2.2	Random Access Response Reception	2211

A.10.1.1.1.2.2.3	No Random Access Response Reception	2211

A.10.1.1.1.3	2-step RA type contention-based random access for NR PSCell with CCA	2211

A.10.1.1.1.3.1	Test Purpose and Environment	2211

A.10.1.1.1.3.2	Test Requirements	2215

A.10.1.1.1.3.2.1	MsgA Transmission	2215

A.10.1.1.1.3.2.2	MsgB Reception	2215

A.10.1.1.1.3.2.3	No MsgB Reception	2216

A.10.1.1.1.4	2-step RA type non-contention based random access for NR PSCell with CCA	2216

A.10.1.1.1.4.1	Test Purpose and Environment	2216

A.10.1.1.1.4.2	Test Requirements	2220

A.10.1.1.1.4.2.1	MsgA Transmission	2220

A.10.1.1.1.4.2.2	MsgB Reception	2221

A.10.1.1.1.4.2.3	No MsgB Reception	2221

A.10.2	Timing	2222

A.10.2.1	UE transmit timing	2222

A.10.2.2	UE timing advance	2226

A.10.3	Signalling characteristics	2229

A.10.3.1	Radio link monitoring	2229

A.10.3.1.1	Introduction	2229

A.10.3.1.2	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2230

A.10.3.1.2.1	Test purpose and environment	2230

A.10.3.1.2.2	Test requirements	2234

A.10.3.1.3	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in non-DRX mode	2234

A.10.3.1.3.1	Test purpose and environment	2234

A.10.3.1.3.2	Test requirements	2240

A.10.3.1.4	Radio link monitoring out-of-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2240

A.10.3.1.4.1	Test purpose and environment	2240

A.10.3.1.4.2	Test requirements	2240

A.10.3.1.5	Radio link monitoring in-sync test for PSCell configured with SSB-based RLM RS in DRX mode	2240

A.10.3.1.5.1	Test purpose and environment	2240

A.10.3.1.5.2	Test requirements	2240

A.10.3.2	Interruption	2240

A.10.3.2.1	E-UTRAN – NR interruptions during SCell operations with CCA	2240

A.10.3.2.1.1	Test Purpose and Environment	2240

A.10.3.2.1.2	Test Requirements	2245

A.10.3.3	SCell activation and deactivation delay	2245

A.10.3.4	Beam failure detection and link recovery procedures	2252

A.10.3.5	Active BWP switching	2265

A.10.3.5.1	UL active BWP switch delay with consistent UL LBT failure on PSCell subject to UL CCA in EN-DC	2265

A.10.3.5.1.2	Test Requirements	2270

A.10.3.5.2	DCI-based and Timer-based Active BWP Switch	2271

A.10.3.5.2.1	E-UTRAN – NR PSCell FR1 DL active BWP switch in non-DRX in synchronous EN-DC	2271

A.10.3.5.2.2	E-UTRAN – NR PSCell FR1 DL active BWP switch with FR1 SCell in non-DRX in synchronous EN-DC	2274

A.10.3.5.3	RRC-based Active BWP Switch	2278

A.10.3.6	PSCell addition and release delay	2282

A.10.3.6.1	Addition and Release Delay of known NR PSCell on the carrier under CCA	2282

A.10.3.6.1.1	Test purpose and environment	2282

A.10.3.6.1.2	Test Requirements	2286

A.10.3.7	Active TCI state switching delay	2287

A.10.4	Measurement procedure	2287

A.10.4.1	Intra-frequency measurements	2287

A.10.4.1.1	Event-triggered reporting tests on PSCC without gaps under non-DRX	2287

A.10.4.1.1.1	Test purpose and environment	2287

A.10.4.1.1.2	Test parameters	2287

A.10.4.1.1.3	Test Requirements	2287

A.10.4.1.2	Event-triggered reporting tests on PSCC without gaps under DRX	2288

A.10.4.1.2.1	Test purpose and environment	2288

A.10.4.1.2.2	Test parameters	2288

A.10.4.1.2.3	Test Requirements	2288

A.10.4.1.3	Event-triggered reporting tests on PSCC with per-UE gaps under non-DRX	2288

A.10.4.1.3.1	Test purpose and environment	2288

A.10.4.1.3.2	Test parameters	2288

A.10.4.1.3.3	Test Requirements	2288

A.10.4.1.4	Event-triggered reporting tests on PSCC with per-UE gaps under DRX	2289

A.10.4.1.4.1	Test purpose and environment	2289

A.10.4.1.4.2	Test parameters	2289

A.10.4.1.4.3	Test Requirements	2289

A.10.4.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2289

A.10.4.1.5.1	Test purpose and environment	2289

A.10.4.1.5.2	Test parameters	2289

A.10.4.1.5.3	Test Requirements	2289

A.10.4.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2290

A.10.4.1.6.1	Test purpose and environment	2290

A.10.4.1.6.2	Test parameters	2290

A.10.4.1.6.3	Test Requirements	2290

A.10.4.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2290

A.10.4.1.7.1	Test purpose and environment	2290

A.10.4.1.7.2	Test parameters	2290

A.10.4.1.7.3	Test Requirements	2290

A.10.4.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2291

A.10.4.1.8.1	Test purpose and environment	2291

A.10.4.1.8.2	Test parameters	2291

A.10.4.1.8.3	Test Requirements	2291

A.10.4.1.9	RSSI measurement reporting on PSCC	2291

A.10.4.1.9.1	Test purpose and environment	2291

A.10.4.1.9.2	Test parameters	2291

A.10.4.1.10	Channel occupancy measurement reporting on PSCC	2292

A.10.4.1.10.1	Test purpose and environment	2292

A.10.4.1.10.2	Test parameters	2292

A.10.4.1.11	RSSI measurement reporting on SCC	2292

A.10.4.1.11.1	Test purpose and environment	2292

A.10.4.1.11.2	Test parameters	2292

A.10.4.1.12	Channel occupancy measurement reporting on SCC	2293

A.10.4.1.12.1	Test purpose and environment	2293

A.10.4.1.12.2	Test parameters	2293

A.10.4.2	Inter-frequency measurements	2293

A.10.4.2.1	RSSI measurement reporting	2293

A.10.4.2.1.1	Test purpose and environment	2293

A.10.4.2.1.2	Test parameters	2293

A.10.4.2.2	Channel occupancy measurement reporting	2294

A.10.4.2.2.1	Test purpose and environment	2294

A.10.4.2.2.2	Test parameters	2294

A.10.4.2.3	EN-DC event triggered reporting tests for FR1 with CCA cell without SSB time index detection when DRX is not used	2294

A.10.4.2.3.1	Test Purpose and Environment	2294

A.10.4.2.3.2	Test Requirements	2299

A.10.4.2.4	EN-DC event triggered reporting tests for FR1 cell with CCA without SSB time index detection when DRX is used	2300

A.10.4.2.4.1	Test Purpose and Environment	2300

A.10.4.2.4.2	Test Requirements	2304

A.10.4.2.5	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is not used	2305

A.10.4.2.5.1	Test Purpose and Environment	2305

A.10.4.2.5.2	Test Requirements	2309

A.10.4.2.6	EN-DC event triggered reporting tests for FR1 cell with CCA with SSB time index detection when DRX is used	2310

A.10.4.2.6.1	Test Purpose and Environment	2310

A.10.4.2.6.2	Test Requirements	2314

A.10.4.2.7	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is not used	2315

A.10.4.2.7.1	Test Purpose and Environment	2315

A.10.4.2.7.2	Test Requirements	2321

A.10.4.2.8	EN-DC event triggered reporting tests for FR1 cell without SSB time index detection when DRX is used	2321

A.10.4.2.8.1	Test Purpose and Environment	2321

A.10.4.2.8.2	Test Requirements	2327

A.10.4.2.9	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is not used	2328

A.10.4.2.9.1	Test Purpose and Environment	2328

A.10.4.2.9.2	Test Requirements	2333

A.10.4.2.10	EN-DC event triggered reporting tests for FR1 cell with SSB time index detection when DRX is used	2333

A.10.4.2.10.1	Test Purpose and Environment	2333

A.10.4.2.10.2	Test Requirements	2339

A.10.4.3	L1-RSRP measurements for beam reporting	2340

A.10.4.3.1	SSB based L1-RSRP measurement on PSCC when DRX is not used	2340

A.10.4.3.1.1	Test Purpose and Environment	2340

A.10.4.3.1.2	Test parameters	2340

A.10.4.3.1.3	Test Requirements	2342

A.10.4.3.2	SSB based L1-RSRP measurement on PSCC when DRX is used	2343

A.10.4.3.2.1	Test Purpose and Environment	2343

A.10.4.3.2.2	Test parameters	2343

A.10.4.3.2.3	Test Requirements	2345

A.10.4.3.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2346

A.10.4.3.3.1	Test Purpose and Environment	2346

A.10.4.3.3.2	Test parameters	2346

A.10.4.3.3.3	Test Requirements	2349

A.10.4.3.4	SSB based L1-RSRP measurement on SCC when DRX is used	2350

A.10.4.3.4.1	Test Purpose and Environment	2350

A.10.4.3.4.2	Test parameters	2350

A.10.4.3.4.3	Test Requirements	2353

A.10.4.4	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2354

A.10.4.4.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2354

A.10.4.4.1.1	Test Purpose and Environment	2354

A.10.4.4.1.2	Test Requirements	2358

A.10.4.4.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2359

A.10.4.4.2.1	Test Purpose and Environment	2359

A.10.4.4.2.2	Test Requirements	2364

A.10.4.4.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2365

A.10.4.4.3.1	Test Purpose and Environment	2365

A.10.4.4.3.2	Test Requirements	2369

A.10.4.4.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2370

A.10.4.4.4.1	Test Purpose and Environment	2370

A.10.4.4.4.2	Test Requirements	2375

A.10.5	Measurement performance	2376

A.10.5.1	SS-RSRP	2376

A.10.5.1.1	Intra-frequency measurement accuracy on a CCA serving cell	2376

A.10.5.1.1.1	Test Purpose and Environment	2376

A.10.5.1.1.2	Test parameters	2376

A.10.5.1.1.3	Test Requirements	2379

A.10.5.1.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2379

A.10.5.1.2.1	Test Purpose and Environment	2379

A.10.5.1.2.2	Test parameters	2379

A.10.5.1.2.3	Test Requirements	2382

A.10.5.2	SS-RSRQ	2382

A.10.5.2.1	Intra-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2382

A.10.5.2.1.1	Test Purpose and Environment	2382

A.10.5.2.1.2	Test Parameters	2382

A.10.5.2.1.3	Test Requirements	2385

A.10.5.2.2	Inter-frequency measurement accuracy with FR1 CCA serving cell and FR1 CCA target cell	2385

A.10.5.2.2.1	Test Purpose and Environment	2385

A.10.5.2.2.2	Test Parameters	2385

A.10.5.2.2.3	Test Requirements	2388

A.10.5.3	SS-SINR	2388

A.10.5.4	L1-RSRP measurement for beam reporting with CCA serving cell	2388

A.10.5.4.1	SSB based L1-RSRP measurement	2388

A.10.5.4.1.1	Test Purpose and Environment	2388

A.10.5.4.1.2	Test parameters	2388

A.10.5.4.1.3	Test Requirements	2391

A.10.5.5	RSSI	2391

A.10.5.5.1 	RSSI measurement accuracy on PSCC with CCA	2391

A.10.5.5.1.1	Test Purpose and Environment	2391

A.10.5.5.1.2	Test parameters	2391

A.10.5.5.1.3	Test Requirements	2394

A.10.5.5.2 	RSSI measurement accuracy on SCC with CCA	2394

A.10.5.5.2.1	Test Purpose and Environment	2394

A.10.5.5.2.2	Test parameters	2394

A.10.5.5.2.3	Test Requirements	2397

A.10.5.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2397

A.10.5.5.3.1	Test Purpose and Environment	2397

A.10.5.5.3.2	Test parameters	2397

A.10.5.5.3.3	Test Requirements	2401

A.10.5.6	Channel occupancy	2401

A.10.5.6.1 	Channel occupancy measurement accuracy on PSCC with CCA	2401

A.10.5.6.1.1	Test Purpose and Environment	2401

A.10.5.6.1.2	Test parameters	2401

A.10.5.6.1.3	Test Requirements	2405

A.10.5.6.2 	Channel occupancy measurement accuracy on SCC with CCA	2405

A.10.5.6.2.1	Test Purpose and Environment	2405

A.10.5.6.2.2	Test parameters	2405

A.10.5.6.2.3	Test Requirements	2408

A.10.5.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2408

A.10.5.6.3.1	Test Purpose and Environment	2408

A.10.5.6.3.2	Test parameters	2408

A.10.5.6.3.3	Test Requirements	2412

A.11	NR Standalone Tests with NR PCell under CCA and Other NR Cells in FR1	2412

A.11.1	RRC_IDLE state mobility	2412

A.11.1.1	Cell re-selection with both source and target NR carrier frequencies under CCA	2412

A.11.1.1.1	Cell reselection to FR1 intra-frequency NR cells when subject to CCA on the serving and target cell	2412

A.11.1.1.1.1	Test Purpose and Environment	2412

A.11.1.1.1.2	Test Parameters	2413

A.11.1.1.1.3	Test Requirements	2416

A.11.1.1.2	Cell reselection to FR1 inter-frequency NR case when subject to CCA on the serving and target cell	2416

A.11.1.1.2.1	Test Purpose and Environment	2416

A.11.1.1.2.2	Test Parameters	2416

A.11.1.1.2.3	Test Requirements	2420

A.11.1.2	Cell re-selection to NR with source NR carrier frequency under CCA	2420

A.11.1.2.1	Cell reselection to FR1 inter-frequency NR case when serving cell is subject to CCA	2420

A.11.1.2.1.1	Test Purpose and Environment	2420

A.11.1.2.1.2	Test Parameters	2421

A.11.1.2.1.3	Test Requirements	2427

A.11.1.3	Cell re-selection from NR carrier with target NR carrier frequency under CCA	2428

A.11.1.3.1	Cell reselection to FR1 inter-frequency NR case when target cell is subject to CCA	2428

A.11.1.3.1.1	Test Purpose and Environment	2428

A.11.1.3.1.2	Test Parameters	2428

A.11.1.3.1.3	Test Requirements	2433

A.11.1.4	Inter-RAT cell re-selection to E-UTRAN with source NR carrier frequency under CCA	2434

A.11.1.4.1	Cell reselection to higher priority E-UTRAN when serving cell is subject to CCA	2434

A.11.1.4.1.1	Test Purpose and Environment	2434

A.11.1.4.1.2	Test Parameters	2434

A.11.1.4.1.3	Test Requirements	2437

A.11.1.4.2	Cell reselection to lower priority E-UTRAN when serving cell is subject to CCA	2438

A.11.1.4.2.1	Test Purpose and Environment	2438

A.11.1.4.2.2	Test Requirements	2440

A.11.2	RRC_CONNECTED state mobility	2441

A.11.2.1	Handover	2441

A.11.2.1.1	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; known target cell	2441

A.11.2.1.1.1	Test Purpose and Environment	2441

A.11.2.1.1.2	Test Parameters	2441

A.11.2.1.1.3 Test Requirements	2444

A.11.2.1.2	Intra-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2444

A.11.2.1.2.1	Test Purpose and Environment	2444

A.11.2.1.2.2	Test Parameters	2444

A.11.2.1.2.3	Test Requirements	2447

A.11.2.1.3	Inter-frequency handover from FR1 carrier under CCA to FR1 carrier under CCA; unknown target cell	2447

A.11.2.1.3.1	Test Purpose and Environment	2447

A.11.2.1.3.2	Test Parameters	2447

A.11.2.1.3.3 Test Requirements	2450

A.11.2.1.4	Inter-frequency handover from FR1 carrier under CCA to FR1; known target cell	2450

A.11.2.1.4.1	Test Purpose and Environment	2450

A.11.2.1.4.2	Test Parameters	2450

A.11.2.1.4.3 Test Requirements	2454

A.11.2.1.5	Inter-frequency handover from FR1 carrier under CCA to FR1; unknown target cell	2455

A.11.2.1.5.1	Test Purpose and Environment	2455

A.11.2.1.5.2	Test Parameters	2455

A.11.2.1.5.3 Test Requirements	2458

A.11.2.1.6	Inter-frequency handover from FR1 to FR1 carrier under CCA; unknown target cell	2459

A.11.2.1.6.1	Test Purpose and Environment	2459

A.11.2.1.6.2	Test Parameters	2459

A.11.2.1.6.3 Test Requirements	2462

A.11.2.1.7	 SA NR FR1 carrier under CCA - E-UTRAN handover with known target cell	2463

A.11.2.1.7.1	Test Purpose and Environment	2463

A.11.2.1.7.2	Test Requirements	2469

A.11.2.1.8	SA NR FR1 carrier under CCA - E-UTRAN handover with unknown target cell	2469

A.11.2.1.8.1	Test Purpose and Environment	2469

A.11.2.1.8.2	Test Requirements	2474

A.11.2.2	RRC connection mobility control	2474

A.11.2.2.1	RRC re-establishment	2474

A.11.2.2.1.1	Intra-frequency RRC Re-establishment with CCA in FR1	2474

A.11.2.2.1.2	Inter-frequency RRC Re-establishment with CCA in FR1	2477

A.11.2.2.1.4	Inter-frequency RRC Re-establishment from NR FR1 carrier without CCA to NR FR1 carrier under CCA	2483

A.11.2.2.2	Random Access	2488

A.11.2.2.2.1	4-step RA type contention-based random access for NR PCell with CCA	2488

A.11.2.2.2.1.1	Test Purpose and Environment	2488

A.11.2.2.2.1.2	Test Requirements	2491

A.11.2.2.2.1.2.1	Random Access Preamble Transmission	2491

A.11.2.2.2.1.2.2	Random Access Response Reception	2491

A.11.2.2.2.1.2.3	No Random Access Response Reception	2492

A.11.2.2.2.1.2.4	Receiving an UL grant for msg3 retransmission	2492

A.11.2.2.2.1.2.5	Reception of an Incorrect Message over Temporary C-RNTI	2492

A.11.2.2.2.1.2.6	Reception of a Correct Message over Temporary C-RNTI	2493

A.11.2.2.2.1.2.7	Contention Resolution Timer expiry	2493

A.11.2.2.2.2	4-step RA type non-contention based random access for NR PSCell with CCA	2493

A.11.2.2.2.2.1	Test Purpose and Environment	2493

A.11.2.2.2.2.2	Test Requirements	2496

A.11.2.2.2.2.2.1	SSB-based Random Access Preamble Transmission	2496

A.11.2.2.2.2.2.2	Random Access Response Reception	2497

A.11.2.2.2.2.2.3	No Random Access Response Reception	2497

A.11.2.2.2.3	2-step RA type contention-based random access for NR PCell with CCA	2497

A.11.2.2.2.3.1	Test Purpose and Environment	2497

A.11.2.2.2.3.2	Test Requirements	2501

A.11.2.2.2.3.2.1	MsgA Transmission	2501

A.11.2.2.2.3.2.2	MsgB Reception	2501

A.11.2.2.2.3.2.3	No MsgB Reception	2502

A.11.2.2.2.4	2-step RA type non-contention-based random access for NR PCell with CCA	2502

A.11.2.2.2.4.1	Test Purpose and Environment	2502

A.11.2.2.2.4.2	Test Requirements	2506

A.11.2.2.2.4.2.1	MsgA Transmission	2506

A.11.2.2.2.4.2.2	MsgB Reception	2507

A.11.2.2.2.4.2.3	No MsgB Reception	2507

A.11.2.2.3	RRC connection release with redirection	2508

A.11.2.2.3.1	Redirection from NR FR1 carrier under CCA to NR FR1 carrier under CCA	2508

A.11.3	Timing	2516

A.11.3.1	UE transmit timing	2516

A.11.3.2	UE timing advance	2520

A.11.3.2.1	UE Timing Advance Adjustment Accuracy with PCell under DL CCA	2520

A.11.3.2.1.1	Test Purpose and Environment	2520

A.11.3.2.1.2	Test Parameters	2520

A.11.3.2.1.3	Test Requirements	2524

A.11.4	Signalling characteristics	2524

A.11.4.1	Radio link monitoring	2524

A.11.4.1.1	Introduction	2524

A.11.4.1.2	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2525

A.11.4.1.2.1	Test purpose and environment	2525

A.11.4.1.2.2	Test requirements	2529

A.11.4.1.3	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in non-DRX mode	2529

A.11.4.1.3.1	Test purpose and environment	2529

A.11.4.1.3.2	Test requirements	2535

A.11.4.1.4	Radio link monitoring out-of-sync test for PCell configured with SSB-based RLM RS in DRX mode	2535

A.11.4.1.4.1	Test purpose and environment	2535

A.11.4.1.4.2	Test requirements	2535

A.11.4.1.5	Radio link monitoring in-sync test for PCell configured with SSB-based RLM RS in DRX mode	2535

A.11.4.1.5.1	Test purpose and environment	2535

A.11.4.1.5.2	Test requirements	2535

A.11.4.2	Interruption	2536

A.11.4.2.1	NR interruptions during Scell operations with CCA on PCell and SCell	2536

A.11.4.2.1.1 Test Purpose and Environment	2536

A.11.4.2.1.2	Test Requirements	2540

A.11.4.3	SCell activation and deactivation delay	2540

A.11.4.4	Beam failure detection and link recovery procedures	2547

A.11.4.4.1	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in non-DRX mode	2547

A.11.4.4.1.1	Test Purpose and Environment	2547

A.11.4.4.1.2	Test Requirements	2553

A.11.4.4.2	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with SSB-based BFD and LR in DRX mode	2553

A.11.4.4.2.1	Test Purpose and Environment	2553

A.11.4.4.2.2	Test Requirements	2560

A.11.4.5	Active BWP switching	2560

A.11.4.5.1	UL active BWP switch delay with consistent UL LBT failure on PCell subject to UL CCA	2560

A.11.4.5.1.2	Test Requirements	2565

A.11.4.6	Active TCI state switching delay	2575

A.11.5	Measurement procedure	2575

A.11.5.1	Intra-frequency measurements	2575

A.11.5.1.1	Event-triggered reporting tests on PCC without gaps under non-DRX	2575

A.11.5.1.1.1	Test purpose and environment	2575

A.11.5.1.1.2	Test parameters	2575

A.11.5.1.1.3	Test Requirements	2576

A.11.5.1.2	Event-triggered reporting tests on PCC without gaps under DRX	2576

A.11.5.1.2.1	Test purpose and environment	2576

A.11.5.1.2.2	Test parameters	2576

A.11.5.1.2.3	Test Requirements	2576

A.11.5.1.3	Event-triggered reporting tests on PCC with per-UE gaps under non-DRX	2577

A.11.5.1.3.1	Test purpose and environment	2577

A.11.5.1.3.2	Test parameters	2577

A.11.5.1.3.3	Test Requirements	2577

A.11.5.1.4	Event-triggered reporting tests on PCC with per-UE gaps under DRX	2577

A.11.5.1.4.1	Test purpose and environment	2577

A.11.5.1.4.2	Test parameters	2577

A.11.5.1.4.3	Test Requirements	2577

A.11.5.1.5	Event-triggered reporting tests on SCC without gaps under non-DRX	2577

A.11.5.1.5.1	Test purpose and environment	2577

A.11.5.1.5.2	Test parameters	2578

A.11.5.1.6	Event-triggered reporting tests on SCC without gaps under DRX	2578

A.11.5.1.6.1	Test purpose and environment	2578

A.11.5.1.6.2	Test parameters	2578

A.11.5.1.6.3	Test Requirements	2578

A.11.5.1.7	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2578

A.11.5.1.7.1	Test purpose and environment	2578

A.11.5.1.7.2	Test parameters	2578

A.11.5.1.7.3	Test Requirements	2578

A.11.5.1.8	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2579

A.11.5.1.8.1	Test purpose and environment	2579

A.11.5.1.8.2	Test parameters	2579

A.11.5.1.8.3	Test Requirements	2579

A.11.5.1.9	RSSI measurement reporting on PCC	2579

A.11.5.1.9.1	Test purpose and environment	2579

A.11.5.1.9.2	Test parameters	2579

A.11.5.1.10	Channel occupancy measurement reporting on PCC	2580

A.11.5.1.10.1	Test purpose and environment	2580

A.11.5.1.10.2	Test parameters	2580

A.11.5.1.11	RSSI measurement reporting on SCC	2580

A.11.5.1.11.1	Test purpose and environment	2580

A.11.5.1.11.2	Test parameters	2580

A.11.5.1.12	Channel occupancy measurement reporting on SCC	2581

A.11.5.1.12.1	Test purpose and environment	2581

A.11.5.1.12.2	Test parameters	2581

A.11.5.2	Inter-frequency measurements	2581

A.11.5.2.1	RSSI measurement reporting	2581

A.11.5.2.1.1	Test purpose and environment	2581

A.11.5.2.1.2	Test parameters	2581

A.11.5.2.2	Channel occupancy measurement reporting	2582

A.11.5.2.2.1	Test purpose and environment	2582

A.11.5.2.2.2	Test parameters	2582

A.11.5.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2582

A.11.5.2.3.1	Test Purpose and Environment	2582

A.11.5.2.3.2	Test Requirements	2586

A.11.5.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2587

A.11.5.2.4.1	Test Purpose and Environment	2587

A.11.5.2.4.2	Test Requirements	2592

A.11.5.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2592

A.11.5.2.5.1	Test Purpose and Environment	2592

A.11.5.2.5.2	Test Requirements	2596

A.11.5.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2597

A.11.5.2.6.1	Test Purpose and Environment	2597

A.11.5.2.6.2	Test Requirements	2602

A.11.5.2.7	Event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2602

A.11.5.2.7.1	Test Purpose and Environment	2602

A.11.5.2.7.2	Test Requirements	2606

A.11.5.2.8	Event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2607

A.11.5.2.8.1	Test Purpose and Environment	2607

A.11.5.2.8.2	Test Requirements	2611

A.11.5.2.9	Event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2611

A.11.5.2.9.1	Test Purpose and Environment	2611

A.11.5.2.9.2	Test Requirements	2616

A.11.5.2.10	Event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2617

A.11.5.2.10.1	Test Purpose and Environment	2617

A.11.5.2.10.2	Test Requirements	2622

A.11.5.3	Inter-RAT E-UTRAN measurements	2623

A.11.5.4	L1-RSRP measurements for beam reporting	2623

A.11.5.4.1	SSB based L1-RSRP measurement when DRX is not used	2623

A.11.5.4.1.1	Test Purpose and Environment	2623

A.11.5.4.1.2	Test parameters	2623

A.11.5.4.1.3	Test Requirements	2625

A.11.5.4.2	SSB based L1-RSRP measurement when DRX is used	2625

A.11.5.4.2.1	Test Purpose and Environment	2625

A.11.5.4.2.2	Test parameters	2626

A.11.5.4.2.3	Test Requirements	2628

A.11.5.4.3	SSB based L1-RSRP measurement on SCC when DRX is not used	2628

A.11.5.4.3.1	Test Purpose and Environment	2628

A.11.5.4.3.2	Test parameters	2629

A.11.5.4.3.3	Test Requirements	2632

A.11.5.4.4	SSB based L1-RSRP measurement on SCC when DRX is used	2633

A.11.5.4.4.1	Test Purpose and Environment	2633

A.11.5.4.4.2	Test parameters	2633

A.11.5.4.4.3	Test Requirements	2636

A.11.6	Measurement performance	2637

A.11.6.1	SS-RSRP	2637

A.11.6.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2637

A.11.6.1.1.1	Test Purpose and Environment	2637

A.11.6.1.1.2	Test parameters	2637

A.11.6.1.1.3	Test Requirements	2639

A.11.6.1.2	Intra-frequency measurement accuracy on SCC on a carrier frequency with CCA	2639

A.11.6.1.2.1	Test Purpose and Environment	2639

A.11.6.1.2.2	Test parameters	2639

A.11.6.1.2.3	Test Requirements	2641

A.11.6.2	SS-RSRQ	2641

A.11.6.3	SS-SINR	2641

A.11.6.4	L1-RSRP measurement for beam reporting with CCA serving cell	2641

A.11.6.4.1	SSB based L1-RSRP measurement	2641

A.11.6.4.1.1	Test Purpose and Environment	2641

A.11.6.4.1.2	Test parameters	2641

A.11.6.4.1.3	Test Requirements	2644

A.11.6.5	RSSI	2644

A.11.6.5.1 	Intra-frequency RSSI measurement accuracy on PCC with CCA	2644

A.11.6.5.1.1	Test Purpose and Environment	2644

A.11.6.5.1.2	Test parameters	2644

A.11.6.5.1.3	Test Requirements	2647

A.11.6.5.2 	Intra-frequency RSSI measurement accuracy on SCC with CCA	2647

A.11.6.5.2.1	Test Purpose and Environment	2647

A.11.6.5.2.2	Test parameters	2647

A.11.6.5.2.3	Test Requirements	2650

A.11.6.5.3 	Inter-frequency RSSI measurement accuracy on a carrier with CCA	2650

A.11.6.5.3.1	Test Purpose and Environment	2650

A.11.6.5.3.2	Test parameters	2650

A.11.6.5.3.3	Test Requirements	2653

A.11.6.6	Channel occupancy	2653

A.11.6.6.1 	Intra-frequency channel occupancy measurement accuracy on PCC with CCA	2653

A.11.6.6.1.1	Test Purpose and Environment	2653

A.11.6.6.1.2	Test parameters	2653

A.11.6.6.1.3	Test Requirements	2657

A.11.6.6.2 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2657

A.11.6.6.2.1	Test Purpose and Environment	2657

A.11.6.6.2.2	Test parameters	2657

A.11.6.6.2.3	Test Requirements	2660

A.11.6.6.3 	Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2660

A.11.6.6.3.1	Test Purpose and Environment	2660

A.11.6.6.3.2	Test parameters	2660

A.11.6.6.3.3	Test Requirements	2663

A.11.6.7	E-UTRAN RSRP	2663

A.11.6.8	E-UTRAN RSRQ	2663

A.11.6.9	E-UTRAN SINR	2663

A.12	E-UTRA Standalone Tests with at Least One NR Cell under CCA	2663

A.12.1	RRC_IDLE state mobility	2663

A.12.1.1	Inter-RAT cell re-selection to NR on a carrier frequency with CCA	2663

A.12.1.1.1	E-UTRA Cell reselection to higher priority NR target Cell in FR1 when target cell is subject to CCA	2663

A.12.1.1.1.1	Test Purpose and Environment	2663

A.12.1.1.1.2	Test Requirements	2669

A.12.2	RRC_CONNECTED state mobility	2670

A.12.2.1	Handover	2670

A.12.2.1.1	E-UTRAN - NR with CCA handover	2670

A.12.2.1.1.1	Test Purpose and Environment	2670

A.12.2.1.1.2	Test Requirements	2675

A.12.3	Signalling characteristics	2676

A.12.3.1	Interruptions	2676

A.12.4	Measurement procedure	2676

A.12.4.1	E-UTRANNR inter-RAT SFTD measurements	2676

A.12.4.1.1	E-UTRA – NR Inter-RAT SFTD Measurement Delay with NR under CCA in non-DRX	2676

A.12.4.1.1.1	Test Purpose and Environment	2676

A.12.4.1.1.2	Test Requirements	2680

A.12.4.2	E-UTRANNR inter-RAT measurements on NR carrier frequency under CCA	2680

A.12.4.2.1	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is not used	2680

A.12.4.2.1.1	Test Purpose and Environment	2680

A.12.4.2.1.2	Test Requirements	2684

A.12.4.2.2	E-UTRA-NR inter-RAT event triggered reporting tests for FR1 without SSB time index detection when DRX is used	2685

A.12.4.2.2.1	Test Purpose and Environment	2685

A.12.4.2.2.2	Test Requirements	2689

A.12.4.2.3	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is not used	2690

A.12.4.2.3.1	Test Purpose and Environment	2690

A.12.4.2.3.2	Test Requirements	2694

A.12.4.2.4	NR Inter-RAT event triggered reporting tests for FR1 with SSB time index detection when DRX is used	2694

A.12.4.2.4.1	Test Purpose and Environment	2694

A.12.4.2.4.2	Test Requirements	2698

A.12.4.2.5	RSSI measurement reporting	2698

A.12.4.2.5.1	Test purpose and environment	2698

A.12.4.2.5.2	Test parameters	2698

A.12.4.2.6	Channel occupancy measurement reporting	2699

A.12.4.2.6.1	Test purpose and environment	2699

A.12.4.2.6.2	Test parameters	2699

A.12.5	Measurement performance	2699

A.12.5.1	E-UTRANNR SFTD	2699

A.12.5.2	E-UTRANNR SS-RSRP	2705

A.12.5.3	E-UTRANNR SS-RSRQ	2705

A.12.5.4	E-UTRANNR SS-SINR	2705

A.12.5.5	E-UTRANNR RSSI	2705

A.12.5.6	E-UTRANNR channel occupancy	2705

A.13	 NR Standalone Tests with NR SCell under CCA and All Other NR Cells in FR1	2705

A.13.1	Timing	2705

A.13.1.1	UE transmit timing	2705

A.13.1.2	Timing advance	2705

A.13.2	Signalling characteristics	2705

A.13.2.1	Interruption	2705

A.13.2.1.1	NR interruptions during SCell operations with CCA on SCell	2705

A.13.2.1.1.1	Test Purpose and Environment	2705

A.13.2.1.1.2	Test Requirements	2709

A.13.2.2	SCell activation and deactivation delay	2709

A.13.2.3	Active TCI state switching delay	2716

A.13.3	Measurement procedure	2716

A.13.3.1	Intra-frequency measurements	2716

A.13.3.1.1	Event-triggered reporting tests on SCC without gaps under non-DRX	2716

A.13.3.1.1.1	Test purpose and environment	2716

A.13.3.1.1.2	Test parameters	2716

A.13.3.1.1.3	Test Requirements	2721

A.13.3.1.2	Event-triggered reporting tests on SCC without gaps under DRX	2721

A.13.3.1.2.1	Test purpose and environment	2721

A.13.3.1.2.2	Test parameters	2721

A.13.3.1.2.3	Test Requirements	2725

A.13.3.1.3	Event-triggered reporting tests on SCC with per-UE gaps under non-DRX	2725

A.13.3.1.3.1	Test purpose and environment	2725

A.13.3.1.3.2	Test parameters	2725

A.13.3.1.3.3	Test Requirements	2730

A.13.3.1.4	Event-triggered reporting tests on SCC with per-UE gaps under DRX	2730

A.13.3.1.4.1	Test purpose and environment	2730

A.13.3.1.4.2	Test parameters	2730

A.13.3.1.4.3	Test Requirements	2735

A.13.3.1.5	RSSI measurement reporting on SCC	2735

A.13.3.1.5.1	Test purpose and environment	2735

A.13.3.1.5.2	Test parameters	2735

A.13.3.1.6	Channel occupancy measurement reporting on SCC	2737

A.13.3.1.6.1	Test purpose and environment	2737

A.13.3.1.6.2	Test parameters	2738

A.13.3.2	Inter-frequency measurements	2738

A.13.3.2.1	RSSI measurement reporting	2738

A.13.3.2.1.1	Test purpose and environment	2738

A.13.3.2.1.2	Test parameters	2738

A.13.3.2.2	Channel occupancy measurement reporting	2739

A.13.3.2.2.1	Test purpose and environment	2739

A.13.3.2.2.2	Test parameters	2739

A.13.3.2.3	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is not used	2739

A.13.3.2.3.1	Test Purpose and Environment	2739

A.13.3.2.3.2	Test Requirements	2745

A.13.3.2.4	Event triggered reporting tests for FR1 with CCA without SSB time index detection when DRX is used	2745

A.13.3.2.4.1	Test Purpose and Environment	2745

A.13.3.2.4.2	Test Requirements	2750

A.13.3.2.5	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is not used	2751

A.13.3.2.5.1	Test Purpose and Environment	2751

A.13.3.2.5.2	Test Requirements	2756

A.13.3.2.6	Event triggered reporting tests for FR1 with CCA with SSB time index detection when DRX is used	2756

A.13.3.2.6.1	Test Purpose and Environment	2756

A.13.3.2.6.2	Test Requirements	2762

A.13.3.3	L1-RSRP measurements for beam reporting	2763

A.13.3.3.1	SSB based L1-RSRP measurement when DRX is not used	2763

A.13.3.3.1.1	Test Purpose and Environment	2763

A.13.3.3.1.2	Test parameters	2763

A.13.3.3.1.3	Test Requirements	2767

A.13.3.3.2	SSB based L1-RSRP measurement when DRX is used	2767

A.13.3.3.2.1	Test Purpose and Environment	2767

A.13.3.3.2.2	Test parameters	2767

A.13.3.3.2.3	Test Requirements	2771

A.13.4	Measurement performance	2771

A.13.4.1	SS-RSRP	2771

A.13.4.1.1	Intra-frequency measurement accuracy on a carrier frequency with CCA	2771

A.13.4.1.1.1	Test Purpose and Environment	2771

A.13.4.1.1.2	Test parameters	2771

A.13.4.1.1.3	Test Requirements	2773

A.13.4.2	SS-RSRQ	2773

A.13.4.3	SS-SINR	2773

A.13.4.4	L1-RSRP measurement for beam reporting with CCA serving cell	2773

A.13.4.4.1	SSB based L1-RSRP measurement	2773

A.13.4.4.1.1	Test Purpose and Environment	2773

A.13.4.4.1.2	Test parameters	2773

A.13.4.4.1.3	Test Requirements	2778

A.13.4.5	RSSI	2778

A.13.4.5.1 	Intra-frequency RSSI measurement accuracy on a carrier with CCA	2778

A.13.4.5.1.1	Test Purpose and Environment	2778

A.13.4.5.1.2	Test parameters	2778

A.13.4.5.1.3	Test Requirements	2782

A.13.4.5.2	  Inter-frequency RSSI measurement accuracy on a carrier with CCA	2782

A.13.4.5.2.1	Test Purpose and Environment	2782

A.13.4.5.2.2	Test parameters	2782

A.13.4.5.2.3	Test Requirements	2786

A.13.4.6	Channel occupancy	2786

A.13.4.6.1 	Intra-frequency channel occupancy measurement accuracy on SCC with CCA	2786

A.13.4.6.1.1	Test Purpose and Environment	2786

A.13.4.6.1.2	Test parameters	2786

A.13.4.6.1.3	Test Requirements	2790

A.13.4.6.2	  Inter-frequency channel occupancy measurement accuracy on a carrier with CCA	2790

A.13.4.6.2.1	Test Purpose and Environment	2790

A.13.4.6.2.2	Test parameters	2790

A.13.4.6.2.3	Test Requirements	2794

Annex B (normative): Conditions for RRM requirements applicability for operating bands	1818

B.1	Conditions for NR RRC_IDLE state mobility	1818

B.1.1	Introduction	1818

B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection	1818

B.1.3	Conditions for measurements on NR inter-frequency cells for cell re-selection	1819

B.2	Conditions for UE measurements procedures and performance requirements in RRC_CONNECTED state	1820

B.2.1	Introduction	1820

B.2.1.1	General	1820

B.2.1.2	Derivation of Minimum SSB_RP values for FR1	1820

B.2.1.3	Derivation of Minimum SSB_RP values for FR2	1820

B.2.1.3.1	Minimum SSB_RP values for Rx Beam Peak angle of arrival	1820

B.2.1.3.2	Minimum SSB_RP values for angle of arrival within Spherical coverage	1821

B.2.1.4	Gain to SS-RSRP and CSI-RSRP measurement point for FR1	1821

B.2.1.5	Gain to SS-RSRP and CSI-RSRP measurement point for FR2	1822

B.2.1.5.1	Gain to SS-RSRP and CSI-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.1.6	Gain to PRS-RSRP measurement point for FR2	1822

B.2.1.6.1	Gain to PRS-RSRP measurement point for Rx Beam Peak angle of arrival	1822

B.2.2	Conditions for NR intra-frequency measurements	1823

B.2.3	Conditions for NR inter-frequency measurements	1824

B.2.4	Conditions for NR L1-RSRP reporting	1826

B.2.4.1	Conditions for SSB based L1-RSRP reporting	1826

B.2.4.2	Conditions for CSI-RS based L1-RSRP reporting	1827

B.2.5	Conditions for RRC connection release with redirection to NR	1828

B.2.6	Void	1829

B.2.6.1	Void	1829

B.2.6.2	Void	1830

B.2.7	Conditions for SRS-RSRP measurements	1830

B.2.8	Conditions for NR L1-SINR reporting	1830

B.2.8.1	Conditions for L1-SINR reporting with CSI-RS based CMR and no dedicated IMR configured	1830

B.2.8.2	Conditions for L1-SINR reporting with SSB based CMR and dedicated IMR configured	1831

B.2.8.2.1	L1-SINR reporting with SSB based CMR and dedicated ZP-IMR configured	1831

B.2.8.2.2	L1-SINR reporting with SSB based CMR and dedicated NZP-IMR configured	1832

B.2.8.3	Conditions for L1-SINR reporting with CSI-RS based CMR and dedicated IMR configured	1834

B.2.8.3.1	L1-SINR reporting with CSI-RS based CMR and dedicated ZP-IMR configured	1834

B.2.8.3.2	L1-SINR reporting with CSI-RS based CMR and dedicated NZP-IMR configured	1835

B.2.9	Conditions for NR intra-frequency measurements under CCA	1836

B.2.10	Conditions for NR inter-frequency measurements under CCA	1836

B.2.11	Conditions for NR L1-RSRP reporting under CCA	1836

B.2.11.1	Conditions for SSB based L1-RSRP reporting	1836

B.2.12	Conditions for NR CSI-RS based intra-frequency measurements	1837

B.2.13	Conditions for NR CSI-RS based inter-frequency measurements	1838

B.2.14	Conditions for NR PRS-based measurements	1840

B.3	RRM Requirements Exceptions	1841

B.3.1	Introduction	1841

B.3.2	Receiver sensitivity relaxation for CA	1841

B.3.2.1	Receiver sensitivity relaxation for UE supporting CA in FR1	1841

B.3.2.2	Receiver sensitivity relaxation for UE configured with CA in FR1	1842

B.3.2.2.1	Inter-band carrier aggregation	1842

B.3.2.2.2	Reference sensitivity exceptions due to UL harmonic interference for CA	1842

B.3.2.2.3	Reference sensitivity exceptions due to intermodulation interference due to 2UL CA	1842

B.3.2.3	Receiver sensitivity relaxation for UE supporting CA in FR2	1842

B.3.2.4	Receiver sensitivity relaxation for UE configured with CA in FR2	1842

B.3.2.4.1	Intra-band contiguous carrier aggregation	1842

B.3.2.4.2	Intra-band non-contiguous carrier aggregation	1842

B.3.3	Receiver sensitivity relaxation for DC	1843

B.3.3.1	Receiver sensitivity relaxation for EN-DC	1843

B.3.3.2	Receiver sensitivity relaxation for NE-DC	1843

B.3.4	Receiver sensitivity relaxation for SUL	1843

B.3.4.1	Receiver sensitivity relaxation for UE supporting SUL in FR1	1843

B.3.4.2	Receiver sensitivity relaxation for UE configured with SUL in FR1	1843

B.3.4.2.1	Reference sensitivity exceptions due to UL harmonic interference for SUL	1843

B.4	Conditions for V2X	1843

B.4.1	Test parameters for GNSS signals	1843

B.4.2	Conditions for PSBCH-RSRP Accuracy Requirements	1844

B.4.3	Conditions for Selection/Reselection to Intra-frequency SyncRef UE	1844

B.4.4	Conditions for L1 SL-RSRP Accuracy Requirements	1844

Annex C (informative): Change history	1846